TSG SA Meeting #SP-110	SP-251448

09 - 12 December, 2025, Baltimore, USA	(S4-251629)

Source:	SA WG4

Title:	FORCE Technology Listening Lab Report - IVAS characterization Phase

Document for:	Approval

Agenda item:	4.4

# Summary

The document presents a report on subjective testing performed as a part of 3GPP-SA4 IVAS Codec Characterization Phase. FORCE Technology performed the experiment according to the procedures and test plan specified in permanent documents IVAS-7b (Processing Plan for Characterization Phase) and IVAS-8b (Test Plan for Characterization Phase). No deviations from or exceptions to the listening test procedures and specifications described in the test plan have been observed or experienced.

Key Acronyms

CuT	Codec under Test

DCR	Degradation Category Rating

DK	Danish (language)

ETSI	European Telecommunications Standards Institute

GAL	Global Analysis Laboratory

HL	Host Laboratory

LL	Listening Laboratory

MC	Material collection entity

FB	Full Band

# 1. Introduction

FORCE Technology performed 6 different experiments within the 3GPP SA4 IVAS Codec Characterization exercise. The experiment name, methodology (as defined in ITU-T P.800 [1] and ITU-R BS.1534 [2]) and language for each experiment is listed in Table 1.

Table 1: Allocation of methodologies and languages

| Exp. | Methodology | Language |
| --- | --- | --- |
| P800-7 | P.SUPPL800 | DK |
| P800-11 | P.SUPPL800 | DK |
| P800-13 | P.SUPPL800 | DK |
| P800-22 | P.SUPPL800 | DK |
| P800-23 | P.SUPPL800 | DK |
| BS1534-1 | ITU-R BS.1534 (Binaural) | - |

2. Test material

#### a. P800-DCR

FORCE Technology delivered to the MC original speech samples in 48-kHz sampled with 16-bit resolution. The format was uncompressed *.wav. The unprocessed speech material for experiments P800-7, 11, 13, 22, 23 to be conducted by the LL was delivered in the defined timeframe and conformed to restrictions indicated in “Practical procedures for subjective testing”, ITU-T Handbook, 2011 [3].

The file naming structure of the input content samples was defined in [4] and represented by:

cmapxxgyszz.wav

where:

- c is for the listening lab designator (c for FORCE Technology)

- m is mono

- a is 0 degrees ahead

- p is P.800 test type designator

- xx is the experiment designator, e.g. 07 (see Table 9 of [5])

- g is the gender of the talker

- y is the talker number (m1, 2, 3 or f1, 2, 3)

- s stands for sample

- zz is the sentence number (01-14)

#### b. BS.1534 (MUSHRA) with headphones and multi-channel audio

The test material for BS.1534 (MUSHRA) tests was selected and delivered by the MC.

3. Listening Environment

#### a. P800-DCR

The tests were performed in acoustically treated critical listening rooms in FORCE Technology Hørsholm, which conform to the requirements of P.800, including:

-having a background noise level of less than NR-25, meeting the minimum requirements in P.800 in full.

-listening equipment conformed to specified requirements ([1],[3]) in full. Beyerdynamic DT770 Pro headphones (stereo presentation at 73dB SPL corresponding to -26dBoV RMS 1kHz sinewave) have been used for P800 DCR experiments. All used headphones were calibrated and verified before and after experiments. Professional listening test software (SenseLabOnline) was used to collect the votes.

b. BS.1534 (MUSHRA) with headphones

The tests were performed in acoustically treated critical listening rooms in FORCE Technology laboratories that conforms to the requirements of ITU-R BS.1116 [6] including:

-room size, shape, and proportions meet the requirements of 8.2.2 of [6],

-reverberation time and noise floor meet the requirements of 8.2.3 and 8.3.4.2 of [6], respectively..

Sennheiser HD650 headphones have been used for BS.1534 MUSHRA experiments. All used headphones have been verified before and after experiments were performed. Professional listening test software (SenseLabOnline) was used to collect the subjective results.

4. Test instructions/order

## a. P.800-DCR

The instructions for subjects in each of the languages tested by the FORCE Technolgy for the characterization testing are available in Annex A of this document. The instructions to subjects were available in written format during the entire test session. During the training session of each test, the instructions were verbally explained and if needed briefly discussed by a dedicated expert person that was able to answer questions from the subjects in their native language.

Sample presentation order followed the pseudo-randomized sequences obtained from IVAS_char_rands_force.xlsx.

- BS.1534 (MUSHRA)

- The instructions for subjects in each language tested by FORCE Technology for the Selection Testing are available in Annex B. The sample presentation order within one test item was randomized by the listening test software (SenseLabOnline).

5. Test Subjects

## a. P.800-DCR

For each P.SUPPL800 DCR listening test, only native speakers of the tested language, fulfilling the requirements of [1] were used. Each experiment required 30 listeners (6 panels of 5 listeners each). The age and gender information for the set of subjects used in each listening test, and over all listening tests in each tested language tested by FORCE Technology are provided in Annex C of this report.

- BS.1534 (MUSHRA)

For BS.1534 MUSHRA experiments, screened (Audiometry, listening acuity) and regularly trained expert listeners, from FORCE Technology SenseLab’s expert listener panel have been used. Assessors have been excluded from the aggregated responses if he or she rated the hidden reference condition for > 15% of the test items lower than a score of 90 as required by [2].

6. Raw data delivery

All raw voting data have been delivered to the GAL using the data delivery file provided by the GAL for all tests. This delivery had happened in the required time frame.

Individual FORCE Technology test results are included in the GAL report.

Annex A

Sample Instructions to Subjects in Danish as used by FORCE Technology during the subjective listening tests performed during 3GPP SA4 IVAS characterization phase:

A.1. Test instructions in Danish for DCR speech experiments (P800-7,11,13,22,23):

I dette forsøg, skal du vurdere systemer, som kan blive en del af fremtidens telekommunikationstjenester der har en grad af indlevelse, vha. rumlig lyd. Rumlig lyd betyder at man kan høre hvor forskellige lydkilder kommer fra. For eksempel kan det lyde som om én person taler på venstre side, en anden fra højre, talere lyder som om de bevæger sig rundt osv.

På hvert skærmbillede vil du først høre et referencelydeksempel, og herefter et testlydeksempel. Testlydeksemplet har det samme indhold som referencelydeksemplet, men det kan være blevet forringet efter det har været sendt igennem et telekommunikationssystem.

Din opgave er at vurdere den overordnede forringelse, som det andet lydeksempel har ift. det første, herunder både forringelse af lydkvaliteten (f.eks. som følger af tilføjet støj, ruhed, klik eller andre forvrængninger), og/eller forskelle i den rumlige repræsentation (f.eks. lydkildernes position, afstand, rumlige bredde, bevægelse osv.)

Du skal først lytte grundigt til begge lydeksemplerne på skærmbilledet. Når de er stoppet med at spille, bedes du vælge den kategori på skalaen, som beskriver din overordnede opfattelse, af mængden af en hvilken som helst forringelse du kan høre i det andet lydeksempel, relativt til det første lydeksempel.

5 - Ingen forringelse

4 - Lille forringelse

3 - Moderat forringelse

2 - Stor forringelse

1 - Meget stor forringelse

Bemærk at mængden af forringelse i de forskellige testlydeksempler, forventes at spænde over hele skalaen i dette forsøg.

Undlad at tale med de andre deltagere, om det du hører i forsøget, herunder også dine holdninger til lydeksemplerne.

Hvis du har spørgsmål til forsøget, har brug for en pause, forfriskninger el.l. så kontakt venligst forsøgslederen.

God fornøjelse med lytteforsøget.

Annex B

Instructions to expert subjects used by FORCE Technology during the BS-1534 MUSHRA tests performed during the 3GPP SA4 IVAS characterization phase

(training and test instructions were identical, with the exception of identifying the training as “training”).

In this MUSHRA (ITU-R BS.1534-3) test you will listen to and rate different audio samples.

Your task consists of two parts:

1) Identify which of the test-audio examples is identical to the reference, and rate it at 100 on the scale.

2) Rate the other audio examples on their Basic Audio Quality, relative to the reference.

Please note!

It is very important that you do not rate the hidden reference under 90 on the scale! If you have any doubt, place your rating of the audio example over 90 – it is not wrong, but correct to rate something, which sounds almost like the reference, in the top of the scale.

This test consists of short training followed by 12 trials of actual test.

Please get in touch with the test instructor if you have any questions.

Enjoy the listening test!

Note: Instructions were translated from Danish.

Annex B

Age and gender information for the set of subjects used in each listening test, and over all listening tests in each tested language tested by FORCE Technology

Table B.1 Age and gender information for the set of subjects used in each P.800 listening test

| Exp. | Language | #female listeners | #male listeners | #diverse listeners | Mean Age (years) | Age StdDev (years) |
| --- | --- | --- | --- | --- | --- | --- |
| P800-7 | DK | 11 | 19 | 0 | 28,5 | 12,2 |
| P800-11 | DK | 11 | 19 | 0 | 25,5 | 12,5 |
| P800-13 | DK | 10 | 20 | 0 | 28,4 | 14,4 |
| P800-22 | DK | 5 | 25 | 0 | 21,3 | 6,7 |
| P800-23 | DK | 9 | 21 | 0 | 27,3 | 12,6 |

Table B.2 Age and gender information across all P.800 subjects within each language tested by FORCE Technology.

| Language | #females/#males #diverse | Mean Age (years) | Age StdDev (years) |
| --- | --- | --- | --- |
| Danish | 46 / 104 / 0 | 26,2 | 12,2 |

Reference Documents

[1]	ITU-T P.800, “Methods for subjective determination of transmission quality”, 1996.

[2]	ITU-R BS.1534, “Method for the subjective assessment of intermediate quality level

of audio systems, 2015

[3] 	“Practical procedures for subjective testing”, ITU-T Handbook, 2011.

[4]	IVAS-7b (Processing Plan for Characterization Phase)

[5]	IVAS-8b (Test Plan for Characterization Phase)

[6] 	ITU-R BS.1116, “Methods for the subjective assessment of small impairments”, 2015.