3GPP TSG-SA WG6 Meeting #69	S6-254430

Wuhan, China 13th – 17th October 2025	(revision of S6-254075)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0393 | rev | 1 | Current version: | 19.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Editor’s Note on server-to-server messages (MCData) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clauses 7.17.2.3 Ad hoc group data session request (MCData server – MCData server) and 7.17.2.6 Ad hoc group data session response (MCData server – MCData server) contain the following editor’s note: Editor's Note: It is FFS if the server to server message is needed in a call request or response message. These two messages, applicable to the multiple MC system scenario are crrently not used in ad hoc group call procedures, it is therefore suggested to convert the editor’s notes into formal notes in frozen releases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Converting editor’s notes into formal notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s notes remain in frozen specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.3, 7.17.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 7.17.2.3	Ad hoc group data session request (MCData server – MCData server)

Table 7.17.2.3-1 describes the information flow ad hoc group data session request between the MCData servers.

Table 7.17.2.3-1 Ad hoc group data session request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group data communication |
| MCData ID list (see NOTE 1, NOTE 3) | O | MCData IDs of the participants being invited for the ad hoc group data communication |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for a broadcast ad hoc group data communication |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group imminent peril data communication |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group emergency data communication |
| Preconfigured MCData group ID | O | Group identity whose configuration is to be applied for this ad hoc group data communication. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group data communication |
| NOTE 1: This element is included only when the data communication initiating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

NOTE:	Whether the server-to-server message is needed in a call request is out of scope of the present document.

* * * Next Change * * *

#### 7.17.2.6	Ad hoc group data session response (MCData server – MCData server)

Table 7.17.2.6-1 describes the information flow ad hoc group data session response between the MCData servers.

Table 7.17.2.6-1: Ad hoc group data session response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |
| Functional alias | O | The functional alias of the MCData user receiving data |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group data communication |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group data communication request (success or failure) |

NOTE:	Whether the server-to-server message is needed in a call response is out of scope of the present document.

* * * End of Changes * * *