3GPP TSG-SA WG6 Meeting #69	S6-254535

Wuhan, China, 13th – 17th October 2025                                                    was S6-254312

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0172 | rev | 1 | Current version: | 19.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 9.12.2.4.2 EN resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor's note: Whether SA2 provided services can support UE-to-UE communication analytics is FFS. Whether ADAES supports off-network communication analytics is FFS. Above EN is removed by adding the relevant reference to the SA2 defined analytics in TS 23.288. ADAES supports off-network functional architecture as defined in clause 5.2.3 in TS 23.436 and uses NWDAF defined UE communication analytics as indicated in the procedure defined in the clause 8.15 in TS 23.436. But, no further update on ADAES analytics support is required in the procedure defined in the clause 9.12.2.4.2 in TS 23.433, hence its removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added relevant reference to SA2 TS for the indicated analytics. Typo correction in the call flow diagram (diret -> direct). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Un resolved EN leads to incomplete requirement leading to incomplete feature implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 9.12.2.4.2	SEALDD enabled UE-to-UE communication

The SEALDD servers has Data Delivery (DD) policy being provisioned for UE-to-UE communication. The DD policy is enforced by the SEALDD server to switch the SEALDD connection for UE-to-UE communication (either direct or indirect).

Pre-conditions:

1.	The SEALDD server has DD policies available.

2.	The SEALDD clients in UE1 and UE2 has discovered the same SEALDD server.

3.	The SEALDD clients in UE1 and UE2 have the ProSe application capabilities.

4.	SEALDD UE-to-UE policy has been provided to SEALDD client.

Figure 9.12.2.4.2-1: Policy enforced by SEALDD server for connectivity between two UEs

1a.	The UE-to-UE communication is established between UE 1 and UE 2 via SEALDD client as specified in clause 6.4.3.1 of 3GPP TS 23.304 [17].

1b.	The SEALDD client in UE1/2 informs the SEALDD server about the establishment of direct SEALDD communication with Sdd_XRTransmissionConnnection_Inform operation.

2.	Based on SEALDD UE-to-UE policy, the SEALDD server requests 5GC (NWDAF/NEF) for UE relative proximity analytics for UE1 and UE2 as defined in the clause 6.19 of 3GPP TS 23.288 [19], requests 5GC (GMLC/NEF) for SL/Ranging service exposure about the relative locations or distances and directions related to the UE1 and UE2, and requests ADAES for UE-to-UE application performance analytics as described in 3GPP TS 23.436 [13] clause 8.4, requests 5GC (NWDAF/NEF) for QoS Sustainability Analytics as defined in the clause 6.9 of 3GPP TS 23.288 [19] and Service Experience Analytics for UEs as defined in the clause 6.4 of 3GPP TS 23.288 [19] and/or requests 5GC(PCF/NEF) for QoS monitoring.

3a.	According to the SEALDD UE-to-UE policy, if relative proximity analytics result shows that both UEs are not in vicinity to each other (distance between UEs > proximity threshold in UE-to-UE policy), and/or the QoS/QoE for UE-to-UE direct communication analytics result is no so good (e.g. PLR in analytics report > PLR threshold in UE-to-UE policy), the SEALDD continues with step 4 for indirect communication preparation; otherwise, the SEALDD keeps monitoring (as described in step 2) for the two UEs.

3b.	According to the SEALDD UE-to-UE policy, if the ProSe discovery result shows that both UEs are not in vicinity to each other(e.g., no discovery response received),  and/or the UE-to-UE application performance result(collected as defined in 3GPP TS 23.436 [13] clause 8.4 step 5) is not so good(latency in application performance result >  latency in UE-to-UE policy), the SEALDD client(s) on either or both of the two UEs decides to perform path switching and continues with step 4 for indirect communication preparation.4.	After establishment of SEALDD connections for UE1 and UE2, the SEALDD traffic carrying XR flows are processed in the SEALDD server. The SEALDD server forwards traffic between UE1 and UE2, and may buffer data burst from originating UE and send it to destination UE smoothly.

5.	If later on, both UEs are in proximity which is capable of direct communication and the QoS/QoE analytics result from ADAES for UE-to-UE direct communication shows good quality, the SEALDD server helps to establish direct SEALDD communication by informing the SEALDD client of any of the two UEs with Sdd_XRTransmissionConnnection_Trigger operation including the information of its peer SEALDD client (of UE1 or UE2). Then the SEALDD client 1 or 2 establishes SEALDD connection towards the peer SEALDD client via SEALDD-PC5 and the SEALDD traffic carrying XR flows are exchanged directly between UE1 and UE2 via SEALDD-PC5.

Based on the monitoring and analytics result (as requested in step 2), and SEALDD UE-to-UE policy:

-	if the current UE1-UE2 communication mode is direct and SEALDD server decides to switch to indirect communication mode, the SEALDD server establishes SEALDD connections with SEALDD client 1 and SEALDD client 2 as described in step 4, then triggers direct SEALDD communication release using Sdd_XRTransmissionConnnection_Trigger operation towards the SEALDD client which received establishment request in step 5 and the SEALDD client further releases the direct SEALDD connection towards its peer SEALDD client via SEALDD-PC5.

-	if the current UE1-UE2 communication mode is indirect and SEALDD server decides to switch to direct communication mode, the SEALDD server triggers direct SEALDD communication setup as described in step 5 and releases SEALDD connections towards SEALDD client 1 and SEALDD client 2.

* * * End of Change * * * *