3GPP TSG-SA6 Meeting #69	S6-254784

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025                               revision of S6-254636

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0164 | rev | 2 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding a whole work flow using SEALDD in D.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 20, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, 3GPP TS 23.433 defines individual functionalities for policy configuration, data storage, and data storage fetch. However, an overall view describing how these functionalities are preformed to achieve End-to-End (E2E) media data delivery in SEALDD is still missing. Such an integrated workflow would help readers have a overall view of how the E2E media delivery functionality—which concurrently involves policy configuration, data storage, and data storage fetch—is implemented in SEALDD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description of the E2E Media Data Delivery procedure in D.2 Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the SEALDD layer achieves end-to-end media data transfer remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

Annex D (Informative):
Media data delivery in SEALDD

# D.1	General

The SEALDD layer provides a general functionality to support data storage and data delivery by introducing 5G interaction (N5/N33 in control plane, N6 in user plane), including application data storage and management, application signalling and data transmission, E2E URLCC transmission, IoT message transmission (i.e. MSGin5G), etc.

# D.2	SEALDD deployment for media delivery

For media delivery functionality in SEALDD, as shown in Figure D.2-1, the functionality/protocol defined in SA4 can be reused and further enhanced by SEALDD for supporting media type traffic, 5GMS/RTC AF and 5GMS RTC AS in 3GPP TS 26.501 [14] and 3GPP TS 26.506 [15] is utilized in SEALDD server to support the general media type delivery. The 5G transmission adaptation component is added in SEALDD, to facilitate the specific optimization for XR application provided by 5G network.

NOTE: The functionality about 5G transmission adaptation component for media type will be further detailed based on the other progresses (e.g. application enabler for XR Services).

Figure D.2-1: SEALDD deployment for media delivery

When utilizing the SEALDD to deliver the media stream, the SEALDD server integrates the 5GMS/RTC AF and 5GMS/RTC AS, the SEALDD client integrates as the media session handler and the media player/framework, and follows the support the uplink media transmission and the downlink media transmission defined in 3GPP TS 26.501 [14] and 3GPP TS 26.506 [15]. The SEALDD-UU integrates the M4 and M5 interfaces of 5GMS according to 3GPP TS 26.501 [14], and the RTC-4 and RTC-5 interfaces of 5G RTC system according to 3GPP TS 26.506 [15]. The SEALDD-S integrates the M1 and M2 interfaces of 5GMS according to 3GPP TS 26.501 [14], and the RTC-1 interface of 5G RTC system according to 3GPP TS 26.506 [15].

D.2.1	The example of whole service flow for data storage and data storage fetch using SEALDD

The figure D.2.1-1 provides an example of whole service flow of using SEALDD.

Figure D.2.1-1: Whole work flow using SEALDD

1.	The VAL server1 configures the SEALDD policy to the SEALDD server, as specified in clause 9.10.2.1.

2.	As specified in clause 9.5.2.1, the VAL server2 sends a Sdd_DataStorage_Creation request which includes the data to be stored and information associated with the data (such as the identifier of application data) to the SEALDD server. Upon verifying the authorization for the storage service creation, the SEALDD server sends a response to the requesting consumer indicating that the request has been accepted.

3.	The SEALDD server could receive the Sdd_DataStorage_Fetch request from multiple SEALDD client with the same identifier of the stored data. Upon performing the authorization check to verify if the requested data (according to the data access control policy) is accessed by the SEALDD client(s), the SEALDD server responds with the requested data, as specified in clause 9.5.2.3 (3a). If the requested data fetched by SEALDD client(s) is not be cached at the SEALDD server, the SEALDD server uses the downlink pull mode to retrieve the data/content from the VAL server over SEALDD-S interface (3b), as described in clause 9.1.

# * * * End of Change * * * *