3GPP SA WG2#172	S2-2509955

Dallas, US, 17-21 November, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	Proposed Interim Agrements and Conclusions for KI#3

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements update and conclusion for KI#3 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Based on TR 23.700-14 V1.1.0, current interim conclusion includes NEF selecting Sensing Fuction based on its supported sensensing area and Sensing Function selecting the Sensing Entities based on the information about the Sensing Entities.

However, the following parts are missing:

1. The sensing Function should be selected based on the required sensing criteria of the sensing request and the Sensing Function supported sensing criteria

2. What sensing criteria in addition to spported sensing area can be used for Sensing Function selection.

3. How does the Sensing Function get the information about the Sensing Entities.

4. What information does the information about the Sensing Entities include.

This pCR is proposed to resolve the issues mentioned above.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * * Start of Changes * * * *

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on the required sensing criteria of the sensing request and the Sensing Function supported sensing criteria.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	Sensing Function can get the information about the Sensing Entities via Sensing Entity registration to the Sensing Function or querying the AMF.

NOTE 1:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 2:	The sensing criteria may include sensing area and sensing service type.

NOTE 3:	The information about the Sensing Entity may include sensing area, sensing service type, supported sensing QoS and on-going sensing services.

* * * * Next Change * * * *

8.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on the required sensing criteria of the sensing request and the Sensing Function supported sensing criteria.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	Sensing Function can get the information about the Sensing Entities via Sensing Entity registration to the Sensing Function or querying the AMF.

NOTE 1:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 2:	The sensing criteria may include sensing area and sensing service type.

NOTE 3:	The information about the Sensing Entity may include sensing area, sensing service type, supported sensing QoS and on-going sensing services.

* * * * End of Changes * * * *