SA WG2 Meeting #172	S2-2510009

17 - 21 November, 2025, Dallas, USA

Title:	KI#1: interim conclusions updates

Source:	Nokia

Document for:	Approval

Agenda Item:	20.3.1

Discussion

1- On the following topic for further consideration:

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

Our view is that the DCF can be a standalone function or can be co-located with an existing function in the 5GC. Whether to implement DCF functionality in a standalone NF or co-located with existing NF should be left to operators’ deployment choice.

Proposal 1:

We propose to capture the following in the interim agreements part:

-	The DCF can be a standalone function or can be co-located with an existing function in the 5GC.

2 - On RAN2 answers to SA2 questions in R2-2507928 / S2-2509869:

RAN2 Answer to Question 1.1

RAN2 concluded that there is no need to support paging of UEs in IDLE or INACTIVE mode to provide them with data reporting configuration or to trigger data collection procedure.

It is then our understanding that there is a need to provide data collection configuration only to connected mode UEs, and so the entity selecting UEs for data collection will need to know the information of which UEs are in connected mode, together with the information of change of mode from connected to idle.

Proposal 2:

Restrict the UE data collection to connected UEs.

RAN2 Answer to Question 1.2

In Rel-19 RAN2 did not study whether data collection on some specific geographical area is needed and thus RAN2 did not conclude that continuation of measurements for data collection is required upon UE connected mode mobility as long as the UE stays within the initial geographical area.

However, RAN2 agreed that the data transfer of the collected data over the user plane should be continued during connected mode mobility.

In order to avoid triggering data collection in the whole network and enable the selection of some of the gNBs for data collection, it is proposed that SA2 makes the assumption that data collection will be triggered in a specific geographical area that will be translated into TAIs or cells by 5GC.

Proposal 3:

Restrict the UE data collection to a specific area that will be translated into TAIs or cells by 5GC.

It is also our understanding that there is a need to control connected mode UEs entering in such area to deliver the data collection configuration to these UEs. This then advocates for some control either at AMF or at gNB level on which UEs to send data collection configuration to.

Proposal 4:

AMF or gNB to control which UE to send data collection configuration to.

RAN2 Answer to Question 2.1

In Rel-19 RAN2 agreed that for UE-side data collection for UE-side model training, the RAN can provide a list of candidate data configuration options, and the UE can indicate the preferred data configuration options from this list. The UE can also indicate its preference to start or stop the data collection, after being enabled by the gNB to do send start/stop indications. However, the RAN can also provide UE with data collection configuration or release the data collection configuration at any point in time, with or without UE request.

RAN2 did not discuss how the RAN creates the list of candidate data configuration options, and how RAN decides when to configure a UE to start or stop the data collection. RAN2 assumes that the UE-side server via the CN or the OAM may provide some information that is used by the RAN to create the list of candidate data configuration options and when to start or stop the data collection for a UE.

RAN2's view is that if the RAN receives some information from the UE-side server via the CN or OAM there may be some dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN. RAN2 have not agreed how this information is provided to the RAN and the details on the dependencies.

From RAN2 answer, we can conclude that 5G Core would receive from the UE model training server some configuration for data collection that would then need to be transferred to the gNBs so that gNB can define the list of candidate data configuration options to be sent to the relevant UEs. It is our understanding that the configuration for data collection from the UE model training server can be transparent to the 5G Core, i.e. the 5G Core would only forward the information to gNBs.

Proposal 5:

5G Core to forward configuration for data collection from the UE model training server transparently to gNBs via AMFs.

RAN2 Answer to Question 3.1

The measurement configuration for UE-side data collection is provided per UE, but RAN2 have not discussed how the UEs for data collection are selected.

RAN2 answer is about measurement configuration between gNB and the UE.

RAN2 did not provide answer to how the UEs can be selected. In order to avoid triggering data collection on all UEs or on individual UEs, it is proposed to consider that UE side model training server (OTT server) will trigger UE data collection on specific chipsets, using Type Allocation Codes (TACs).

Proposal 6:

Rely only on TAC as a criteria to select UEs for data collection from UE model training server perspective.

RAN2 Answer to Question 3.2

RAN2 concluded that in some use-cases additional information such as (e.g., UE data collection and feature related radio capabilities, and user consent may be relevant for selection of UEs, but RAN2 have not discussed where (i.e., gNB, CN, UE server) this information would be considered.

This part advocates for some control at gNB side for final selection of the UEs, based e.g. on UE data collection radio capabilities, e.g. whether the UE supports beam management use case and/or CSI prediction use case (see TS 38.306).

Proposal 7:

gNBs to control UEs selection for data collection based on input provided by DCF (via AMF) and other information available at gNB, such as UE data collection radio capabilities.

Proposal

It is proposed to discuss the above proposals and following changes proposed for TR 23.700-04.

---------- Start of change ----------

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A new entity called DCF is defined as the UP path termination entity in the 5GC to support the transfer of standardized UE data over UP, using PDU connectivity service provided by a PDU session. The DCF further transfers the data received from the UE to the AF for UE side model training.

-	The DCF can be a standalone function or can be co-located with an existing function in the 5GC.

-	The UE model training server can request for data collection:

-	in a specific geographical area, or in specific TAI(s), or in specific cell(s). When the request is for a specific geographical area, mapping will be done to TAI(s) or cell(s) by either NEF or DCF.

-	for specific Type Allocation Codes (TACs).

-	The UE model training server provides, in its request for data collection, the “configuration for data collection”. The configuration for data collection is then transferred to the gNBs so that gNBs can create the list of “candidate data configuration options” to be sent to the relevant UEs.

NOTE:	The contents the “configuration for data collection” from the UE model training server would need to be defined together with RAN1 and RAN2.

-	If the UE model training server is trusted, it can discover the DCF based on NF discovery procedure from NRF or local configuration. If the UE model training server is untrusted, the NEF discovers the DCF based on NF discovery procedure from NRF or local configuration.

-	When the DCF receives request for data collection, it provides the “configuration for data collection” to the AMF, together with the area for data collection and information on the target UEs for data collection (i.e. Type Allocation Codes (TACs)). The DCF also provides its UP information (i.e. IP address or FQDN).

-	The AMF then contacts gNBs according to the area for data collection and provides gNBs with the “configuration for data collection”, together with the area for data collection and the information on target UEs for data collection (i.e. TACs) and the DCF UP information. During initial UE context setup, the AMF provides TAC for the UE so that gNB can later do the mapping between UEs and TAC.

-	gNBs are responsible for selecting connected target UEs. Once UEs are selected, gNB provides “candidate data configuration options” to the selected UEs, together with DCF provided UP information.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. In order to transfer the standardized UE data, the UE establishes a new PDU session or reuses an existing PDU Session already established for UE data transfer, based on URSP rules.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

## 7.2	Topics for further consideration

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

-	Whether to store UE collected data in 5GC.

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

---------- End of changes ----------