SA WG2 Meeting #S2-172	S2-2510025

17 - 21 November, 2025, Dallas, USA

Source:	China Mobile

Title:	[WT#4] WT and KI update for Integrated Sensing and Communication

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: WT#4 and KI update on ISAC based on S2-2509793 at SA2#171.

1.	Discussion

This paper proposes WT#4 and KI update on ISAC based on S2-2509793 at SA2#171.

Firstly, to meet the requirements of wide-area sensing, the sensing continuity should be supported in consideration of mobility of target objects.

Secondly, coordination with SA5 for charging aspect will be needed.

2.	Proposal

It is proposed to agree the following changes in TR 23.801-01.

**** First Change (based on S2-2509793) ****

Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering all sensing modes and multiple sources of sensing data.

The following aspects will be studied:

a)	Architecture and functional support for ISAC, including:

a.1)	Service authorization and revocation.

a.2)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

a.3)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

a.4)	Collection and transport of sensing related data.

a.5)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Core NFs make use the sensing results is not part of 6G sensing.

b)	Whether and how to support sensing in consideration of the mobility of the UE as sensing entity, and sensing continuity in consideration of mobility of target objects

c)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 includes the study of all sensing modes and their selection, where sensing entities involve RAN node and UE.

NOTE 4:	WT#4 can involve coordination with WT#5.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects, and SA5 for charging and OAM aspects will be needed.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

**** Second Change (based on S2-2509793) ****

5.X.	Key Issue #X: Support of Integrated Sensing and Communication

5.X.1	Description

Study the integration of Sensing and Communication over 3GPP access, considering all sensing modes and multiple sources of sensing data.

The following aspects will be studied:

a)	Architecture and functional support for ISAC, including:

a.1)	Service authorization and revocation.

a.2)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

a.3)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

a.4)	Collection and transport of sensing related data.

a.5)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Core NFs make use the sensing results is not part of 6G sensing.

b)	Whether and how to support sensing in consideration of the mobility of the UE as sensing entity, and sensing continuity in consideration of mobility of target objects.

c)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 includes the study of all sensing modes and their selection, where sensing entities involve RAN node and UE.

NOTE 4:	WT#4 can involve coordination with WT#5.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects, and SA5 for charging and OAM aspects will be needed.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

**** End of Changes ****