3GPP TSG-WG SA2 Meeting #172	S2-2510214

Dallas, USA, Nov 17-21, 2025

Source:	Ericsson

Title:	KI#1 interim conclusion for capability realization in serving PLMN support for SMS over NAS delivery to PSAP

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution proposes an interim conclusion for KI#1 in TR 23.700-65, v0.2.0.

# Discussion

In TR 23.700-65, v0.2.0, following solutions address the capability realization and negotiation between UE and network for SMS over NAS delivery to PSAP

- Sol#2: During Registration/Attach procedure, the UE provides existing “SMS supported” indication, AMF/MME provides the indication if SMS over NAS delivery to PSAP is supported with additionally the existing “emergency number list” parameter.

- Sol#3: During Registration/Attach procedure, the UE indicates the support of SMS over NAS delivery to PSAP. The AMF/MME provides the “existing emergency number list”.

- Sol#4: During Registration/Attach procedure, the UE provides existing “SMS supported” indication, AMF/MME provides the existing “emergency number list” parameter as an indication of SMS over NAS delivery to PSAP support.

- Sol#5: During Registration/Attach procedure, the UE indicates the support of SMS over NAS delivery to PSAP. The AMF/MME provides indication if SMS over NAS delivery to PSAP is supported with additionally the existing “emergency number list” parameter

- Sol#6: During Mobility procedure, the network signals an indicator indicating capability to support SMS over NAS delivery to PSAP.

- Sol#7: During Registration procedure, UE indicates new indication “SMS for LBO support”, AMF indicates new indication “SMS for LBO Allowed” with additional “emergency number list” and/or serving SMS-C address.

Analysis:

- Considering that existing “emergency number list” can be provided by network also when network supports emergency call, it is not good enough for the UE to realize the support of SMS over NAS delivery to PSAP based on existing “emergency number list”.

- The realization of the network capability of supporting SMS over NAS delivery to PSAP is critical and implied by all solutions listed above. However, the indication of UE capability for supporting SMS over NAS delivery to PSAP is not beneficial from network side, e.g. network can provide the optional indication to UE regardless of the UE support. For the UE supports the function, it will start to use the SMS over NAS delivery to PSAP service. For the UE does not support such function, it will use the service and ignore the indication.

Proposal 1:

It is proposed to document the following interim conclusions for capability realization for SMS over NAS delivery to PSAP:

-	Network indicates to UE during Registration/Attach/TAU Accept message if the network support SMS over NAS delivery to PSAP, if the UE has indicated the support of “SMS support” using existing indication.

-	Network may include the existing “emergency number list” in the Accept message due to the support of SMS over NAS delivery to PSAP, in addition to the support of IMS emergency call.

* * * Start of change * * * *

## 8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 are made:

-	For the SMS over NAS delivery to PSAP capability realization in serving PLMN:

-	Network (i.e. AMF/MME) indicates to UE during Registration/Attach/TAU Accept message if network support SMS over NAS delivery to PSAP, if the UE has indicated the support of “SMS support” using existing indication.

-	Network (i.e. AMF/MME) may include the existing “emergency number list” in the Accept message due to the support of SMS over NAS delivery to PSAP, in addition to the support of IMS emergency call.

* * * End of change * * * *