SA WG2 Meeting #172	S2-2510278

17 – 21 Nov, 2025, Dallas, US

Source:	Apple

Title:	[WT#1.4] Voice and Essential/Regulatory Services in 6G

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Work Task and Key Issue descriptions for support of voice and essential services in 6G based on the progress made in SA2#171.

# 1   Discussion

This contribution builds upon the progress made in SA2#170 and SA2#171. Following earlier discussions, this paper proposes a revision of the Work Task scope based on the postponed document S2-2509781 from SA2#171, as well as corresponding Key Issues.

On the topic of voice fallback, we would like to point out that voice services have incorporated many special features over the years, many of which are provided by RAN or even through media layers. Looking back in history, both 4G and 5G systems offer support for voice through IMS. However, there are several differences between VoLTE and VoNR. The presence of such differences had made it difficult to offer VoNR right from the start in 5G. This was not only because some of the associated sub-features are optional but because interoperability and testing must be ensured. At the same time, VoLTE had to be matched by VoNR to guarantee the same level of voice quality.

To offer 6G voice natively from Day-1 may require 6G RAN and media layers to provide compatibility with VoNR. Alternatively, a similar level of voice quality may be provided through other means. Considering that 6G will be a new RAT with a new PHY and some changes to AS user plane, both options are challenging. To support Voice over 6G from Day-1 demands voice specific adjustments, for example, with regards to power consumption (DRX), cell edge performance (coverage enhancements), RTT (UL audio scheduling alignment), rate adaptation, and more.

Since voice is a very essential feature, the sourcing company prefers voice (e.g. Vo6G) support from Day-1, including emergency sessions and location information for emergency voice. But from a user experience standpoint, the quality of Vo6G must be comparable to VoNR.

In conclusion, an optional fallback procedure (e.g. to VoNR, to be used only when needed) may be studied as a mitigation in case 6G cannot provide sufficient coverage or stable user experience, for example, during early deployment phases. Ideally the fallback option should be lightweight and applicable under special conditions only.

### Proposals

It is proposed to agree the following changes in TR 23.801-01.

# 2   Proposed Changes to 23.801-01

**** First Change (all new text) ****

## Annex A.X. WT#1.4 6G Essential Services

Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

**** Second Change ****

## 5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in  5GS to support voice services as the starting point for discussion) – the following aspects will be studied:

- How a UE supporting 6G RAN selects and connects to a network for voice service.

- Whether and how to update the IMS specification to support an IPCAN in the 6GS.

- How the 6GS supports IMS based voice over the 6G access network.

- Whether and how to support fallback like functionality for voice services from 6GS to 5GS and EPS.

- How to support domain selection for IMS voice services in 6GS.

- How to support the voice continuity between 6G RAT and selected 3GPP  RATs or between 6G RAT and selected non-3GPP Accesses.

- How to support the compatibility between Policy Functions for 6G and the existing deployed AF with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh).

NOTE 1:	NTN specific aspects for voice will be studied in WT#7.

NOTE 2:	Support of voice services via 6G NTN need to be coordinated with WT#7.

NOTE 3:	The study will be aligned with RAN and SA4 and coordinated where needed.

NOTE 4:	Interworking, migration and service continuity aspects need to be coordinated with WT#2.

NOTE 0c:	Support of voice services via 6G NTN need to be coordinated with WT#7.

NOTE 5: The existing IMS architecture is used as a starting point for the study.

## 5.Q. Key Issue #Q: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key issue P.

Additionally, this key issue will study:

- The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

- The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

- How to identify emergency services in 6GS in case of non-UE detectable emergency call, such that the required priority for an emergency call is applied.

- How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

- Whether and how to support fallback like functionality for emergency voice services.

# 5.S. Key Issue #S: Messaging Services for 6G

For messaging service in 6G this Key Issue will study:

- How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [x1], TS 22.261 [x2], TS 22.105 [x3].

NOTE:	 For SMS to Emergency centre support it is assumed that the outcome of 5G-A Rel-20 study will be adopted also for 6G.

# 5.T. Key Issue #T: Other Essential/Regulatory Services for 6G

This Key Issue will study the following services:

- How to support Multimedia Priority Services (MPS) in 6GS.

- How to  provide priority treatment to access control, signalling and media packets delivery for Mission Critical Services (MCX Services) in 6GS.

- How to support Public Warning System (PWS) in 6GS.

NOTE 1:	For PWS stage 2 capabilities will be specified by CT.

NOTE 2:	The scope will keep alignment with RAN.

**** End of Changes ****