SA WG2 Meeting #171	S2-2510319

# Dallas, USA, 17 – 21 November 2025                           (revision of S2-250xxxx)

Source: 	CATT

Title: 	Interim Agreements for KI#2

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes Interim Agreements for KI#2.

1	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change (all texts new) * * * *

7.1.2	Agreed Principles for KI#2

For Topology 1, the following principles are agreed for KI#2:

1. DO-A capable AIoT Devices Registration

a.	DO-A capable AIoT Devices support Initial Registration. The DO-A capable AIoT Device performs Initial Registration autonomously when it wants to inform the network of its presence or has data to be sent.

b.	DO-A capable AIoT Devices support Mobility Registration. The DO-A capable AIoT Device performs Mobility Registration when it moves outside the AIoT Registration Area.

NOTE 1:	AIoT Registration Area is different from Rel-19 defined AIoT Area and is to be defined by RAN3.

Editor’s note:	Whether Periodic Registration is supported or not is FFS.

NOTE 2:	Radio resources allocation for the AS D2R message carrying the Registration Request is to be defined by RAN WGs.

c.	DO-A capable AIoT Device provides in the AS D2R message the AIOTF ID (derived from Temporary ID allocated by the AIOTF) or the PLMN ID/NID/Third Party ID (derived from AIoT Device Permanent ID) to the NG-RAN for AIOTF selection.

d.	The Registration Request message is an AIoT NAS message and includes either AIoT Device Permanent ID or Temporary ID.

NOTE 3:	Security aspects of the Registration procedure are to be defined by SA3.

2. DO-A capable AIoT Device data transfer

a.	DO-A capable AIoT Devices send DO-A data to the AIOTF autonomously or by responding to a request from AIOTF.

b.	DO-A capable AIoT Devices send DO-A data to the AIOTF via an AIoT NAS message which contains DO-A data and optionally Temporary ID.

NOTE 4:	Radio resources allocation for the AS D2R message carrying the AIoT NAS message is to be defined by RAN WGs.

c.	The AIOTF acknowledges the DO-A capable AIoT Device via an AIoT NAS message which contains the result of AIoT data transfer.

NOTE 5:	Security aspects of the AIoT data transfer procedure are to be defined by SA3.

d.	The AIOTF obtains the routing information from the AIoT Device Profile Data and sends the DO-A data and the AIoT Device Permanent ID to NEF or the AF(s).

# * * * * End of Changes * * * *