3GPP TSG-SA WG2 Meeting #172	S2-2510676

# November 17 - 21, 2025 Dallas, USA

Source:	Samsung

Title:	Interim agreement update and final conclusion for KI#1

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement update and final conclusions for KI#1 System Architecture to Support Sensing

# 1. Discussion

TS 23.700-14 v1.1.0 has some agreed principle for KI#1 System Architecture to Support Sensing but there are some other aspects that need to be addressed.

1)	Need for a dedicated NF for Authorization of Sensing Service Request:

In TR 23.700-14 Clause 7.1.2, it was agreed that sensing service request will be authorized using sensing authorization information. This information will be pre-configured or configured by OAM and stored in either SF and a new NF other than SF.

There can be multiple SF in one deployment basically per service area in one PLMN. In this case if SF is reused to store this sensing authorization information, then there will be a need to store these data in all the SFs present in the PLMN. Additionally, whenever there is a need to update some of the sensing authorization information, again all the SFs need to be updated. Hence it is better if the sensing authorization information for all kind of sensing service consumer kept in a separate dedicated NF and this new dedicate function can authorize and send the status to SF.

Proposal: Based on the above explanation it is proposed that a dedicated NF apart from SF need to be introduced for storing sensing authorization information. This can be called Sensing Authorization Function (SAF)

2)	Need for AMF to be in the path between gNB and SF:

For the configuration of sensing parameters to the gNB by Sensing Function and sensing data collection from gNB to the Sensing Function, there is a need for discussion whether there will be direct connection between Sensing Function and gNB or the communication will be via AMF.

For 5GA sensing, the scope is very limited and only gNB as sensing entity is supported. So the architecture should be very simple keeping less impact to the 3GPP system, particularly bringing any major impact to gNB. So it is preferred if AMF is kept in the path and the communication is largely unaffected between gNB and AMF and on the other side with some simple enhancement, communication between AMF and SF can be achieved.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). In addition to SF, another new Network Function (i.e. Sensing Authorization Function) is defined to store the sensing authorization information.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: No dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: The SF upon receiving the sensing service request from NEF (i.e. in case of AF is outside of the trusted domain) or trusted AF, retrieves the sensing authorization information from SAF and authorizes the sensing service request.

NOTE 6:	The authorization and revocation of sensing service request details will be handled in KI#2.

NOTE 7:	The discovery of SF, SAF and SE details will be handled in KI#3.

NOTE 8:	The sensing result exposure details will be handled in KI#5.

Principle 5: The SF upon successful authorization of the sensing service request and discovery of gNB, configures the sensing parameter to gNB and SF sends these parameters via AMF to gNB the sensing service request.

NOTE 9:	The configuration of sensing parameters to SE details will be handled in KI#6.

Principle 6: The gNB provides the sensing data to Sensing Function via AMF.

NOTE 10:	The sensing data collection by Sensing Function from gNB details will be handled in KI#4.

* * * Second Change * * * *

# 8	Conclusions

Principle 1: One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). In addition to SF, another new Network Function (i.e. Sensing Authorization Function) is defined to store the sensing authorization information.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: No dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: The SF upon receiving the sensing service request from NEF (i.e. in case of AF is outside of the trusted domain) or trusted AF, retrieves the sensing authorization information from SAF and authorizes the sensing service request.

NOTE 6:	The authorization and revocation of sensing service request details will be handled in KI#2.

NOTE 7:	The discovery of SF, SAF and SE details will be handled in KI#3.

NOTE 7:	The sensing result exposure details will be handled in KI#5.

Principle 5: The SF upon successful authorization of the sensing service request and discovery of gNB, configures the sensing parameter to gNB and SF sends these parameters via AMF to gNB the sensing service request.

NOTE 8:	The configuration of sensing parameters to SE details will be handled in KI#6.

Principle 6: The gNB provides the sensing data to Sensing Function via AMF.

NOTE 10:	The sensing data collection by Sensing Function from gNB details will be handled in KI#4.

* * * End of Changes * * * *