3GPP SA WG2 Meeting #172	 S2-2510743

17 - 21 November 2025, Dallas. USA 	(was S2-2509133,7368,5392)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1554 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XRM_Ph2_Clarification on the DL Protocol Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Protocol Description is used to support identifying PDU Set Information for packets, size of a Data Burst, the time to next Data Burst and/or last packet of a Data Burst, as described in clause 6.3.1 of TS 23.503. It is not only used for the PDU Set Information marking, as described in clauses 5.37.5, 5.37.8, 5.37.9 and 5.37.10 of TS 23.501. Clarification it inclause 6.1.3.27.4 as above, to avoid the ambiguity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on the DL Protocol Description in clause 6.1.3.27.4 as above, to avoid the ambiguity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature descripion is inaccuracy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.27.4	PDU Set based handling

PDU Set based handling support is defined in clause 5.37.5 of TS 23.501 [2]. The AF may provide the Protocol Description (UL and/or DL) and/or On-path N6 Signalling Information as described in TS 23.501 [2].

The PCF shall, based on local configuration or on information received from the AF, include the UL and/or DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) and/or On-path N6 Signalling Information (see clause 5.37.9 of TS 23.501 [2]) in the PCC Rule.

The PCF may also, based on local configuration or on information received from the AF, generate PDU Set Control Information and include it in the PCC rule. The PDU Set Control Information includes the PDU Set QoS Parameters (see clause 5.7.7 of TS 23.501 [2]). The PCF may in addition, based on local configuration or on information received from the AF, generate Alternative QoS Parameter sets containing PDU Set QoS parameters. Based on the PDU Set QoS parameters received in the PCC rule or a pre-configured PCC rule, the SMF provides 5G-AN with the PDU Set QoS Parameters and optionally Alternative PDU Set QoS parameters (as described in clause 5.37.5 of TS 23.501 [2]). If the PCC rule includes the Protocol Description and/or On-path N6 Signalling Information, it is used for PDU Set identification (as described in clauses 5.37.5 and 5.37.9 of TS 23.501 [2]) or Data Burst information identification (as described in clauses 5.37.8 and 5.37.10 of TS 23.501 [2]).

When the AF requests that a data session to a UE (including the case of 5G-RG) is set up with a specific QoS (as described in clause 6.1.3.22), the AF may also provide Requested PDU Set Delay Budget (UL and/or DL), Requested PDU Set Error Rate (UL and/or DL) and/or Requested PDU Set Integrated Handling Information (UL and/or DL) to describe the requirements for the PDU Set based QoS handling (both the Requested PDU Set Delay Budget and Requested PDU Set Error Rate in one direction shall be provided together when they are provided by the AF) and the PCF generates the corresponding PDU Set QoS parameters (described in clause of 5.7.7 of TS 23.501 [2]) in the PCC rule. If Alternative Service Requirements are provided by the AF (described in clause 6.1.3.22), the AF is expected to also provide Requested PDU Set Delay Budget (UL and/or DL) and Requested PDU Set Error Rate (UL and/or DL) in the Alternative Service Requirements and the PCF generates the corresponding PDU Set QoS parameters in the Alternative QoS Parameter Sets in the PCC rule.

# * * * End of changes * * * *