SA WG2 Meeting #171	S2-2509795

13 – 17 Oct, 2025, Wuhan, China	(Revision of S2-2509728)

Source:	OPPO, China Unicom, China Telecom, China Mobile, MediaTek, NTT DOCOMO, Verizon, AT&T, Intel, Lenovo, CATT, vivo, Ofinno, InterDigital, Toyota, ETRI, LG Electronics, NEC, ZTE, Huawei, HiSilicon, Futurewei, Rakuten Mobile, SoftBank, SK Telecom, Xiaomi, KPN N.V., Turk Telekom, Tencent, TNO, Boost Mobile

Title:	[WT#6] 6G Computing Support

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper is a resubmission of WT#6 (6G computing) based on the postponed paper S2-2508133 at SA2#170.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

# 2. Addressing Missing Aspects from Merged Papers at SA2#170

Input from following SA2#170 papers were considered in the merged revision:

| 20.6.6 | S2-2506361 | P-CR | Approval | 23.801-01: [WT#6]Working tasks and key issues for 6G computing | China Mobile |
| --- | --- | --- | --- | --- | --- |
| 20.6.6 | S2-2506387 | P-CR | Approval | 23.801-01: [WT#6] 6G Computing Support | OPPO |
| 20.6.6 | S2-2506680 | P-CR | Approval | 23.801-01: [WT#6, Computing in 6G] Key Issue on Computing in 6G | NEC Corporation. |
| 20.6.6 | S2-2506891 | P-CR | Approval | 23.801-01: [WT#6] Compute as a service in 6G network | Intel |
| 20.6.6 | S2-2506924 | P-CR | Approval | 23.801-01: [WT#6, Computing] Key Issue for Computing in 6G | Lenovo |
| 20.6.6 | S2-2506985 | P-CR | Approval | 23.801-01: [WT#6, 6G computing] 6G computing for UE | Vivo |
| 20.6.6 | S2-2506995 | P-CR | Approval | 23.801-01: [WT#6] Computing Enabler service, coordination of communication and computing service | Huawei, HiSilicon |
| 20.6.6 | S2-2507020 | P-CR | Approval | 23.801-01: [WT#6] Working tasks and key issues for 6G computing (Resource as a Service (RaaS)) | Rakuten Mobile, SK Telecom, SoftBank, NVIDIA, NEC, NIST |
| 20.6.6 | S2-2507024 | P-CR | Approval | 23.801-01: [WT#6, Computing] Scope and New Key Issue for Computing | LG Electronics |
| 20.6.6 | S2-2507128 | P-CR | Approval | 23.801-01: [WT#6, Computing] Discussion and proposal for computing | ZTE |
| 20.6.6 | S2-2507165 | P-CR | Approval | 23.801-01: [WT#6, Computing in 6G] Work Task Scope and Key Issues for computing in 6G | Tencent, Tencent Cloud |
| 20.6.6 | S2-2507184 | P-CR | Approval | 23.801-01: [WT#6, Computing] Support computing in 6G | CATT |
| 20.6.6 | S2-2507430 | P-CR | Approval | 23.801-01: SA WG2#170_WT_6_supporting for computing service | Xiaomi |
| 20.6.6 | S2-2506810 | P-CR | Approval | 23.801-01: [WT#06] Scope and key issue on computing | NTT DOCOMO, T-Mobile USA, InterDigital Inc, Ofinno, KPN N.V. |
| 20.6.6 | S2-2507280 | P-CR | Approval | 23.801-01: [WT#6] Scope for work task on compute | Qualcomm |
| 20.6.6 | S2-2507395 | P-CR | Approval | 23.801-01: [WT#6] Justification: of work task scope for 6G Compute | Nokia, Ericsson |
| 20.6.6 | S2-2506441 | P-CR | Approval | 23.801-01: [WT#6] Work Task clarification | Samsung |
| 20.6.6 | S2-2506889 | P-CR | Approval | 23.801-01: [WT#6, Computing] Work task description for 6G Computing | Apple |
| 20.6.6 | S2-2506965 | P-CR | Approval | 23.801-01: [WT#6] WT Scope and KI proposal on computing | China Telecom |
| 20.6.6 | S2-2506982 | P-CR | Approval | 23.801-01: [WT#6, all topics] Computing service for 6G | MediaTek Germany GmbH |
| 20.6.6 | S2-2506729 | P-CR | Approval | 23.801-01: [WT#6] Work Task and Key Issues of WT#6 - Computing | ETRI |

## 2.1 Exposure to 3rd party AF

This aspect was mentioned by S2-2506361, S2-2506680, S2-2506891, S2-2506995, S2-2507020, S2-2507024, S2-2507128, S2-2507165, S2-2507430, S2-2506889, S2-2506965, S2-2506729.

It’s now addressed in the revision.

## 2.2 Service Continuity upon Computing Resource Re-location

This aspect was mentioned by S2-2506361, S2-2507165, S2-2507184, S2-2506729.

It’s now addressed in the revision.

## 2.3 Define new QoS characteristics or metrics

This aspect was mentioned by S2-2506361, S2-2507395.

It’s now addressed in the revision.

## 2.4 Charging

This aspect was mentioned by S2-2507430 and should be left to SA5. As charging aspect has been addressed in the 6G SID as general issue, there is no need to add a WT/KI bullet for computing.

## 2.5 Computing Server Discovery

This aspect was mentioned by S2-2507395.

It’s now addressed in the revision.

## 2.6 Leveraging 5G EC design

This aspect was mentioned by S2-2507395 and S2-2507280.

As we are going to discuss which of 5G features will be inherited by 6GS, this can be left to that general discussion, once determined, the features defined by 5G EC design can be considered during the solution development and normative phase.

In 5G EC design, only the communication delay aspect was considered, so just reusing 5G EC design with slight enhancement can’t satisfy the services requiring computing delay.

A NOTE is added to address this aspect in the revision.

## 2.7 Exposure computing to UE

This aspect was mentioned by S2-2506729 and is now addressed in the revision.

# 3. Addressing Missing Aspects from Merged Papers at SA2#171

Input from following SA2#171 papers were considered in the merged revision:

| 20.6.6 | S2-2508302 | P-CR | Approval | 23.801-01: [WT#6] 6G Computing Support WT and KI. | OPPO, MediaTek, NTT DOCOMO, Tencent, Toyota, China Mobile, vivo, Ofinno, KPN, NEC |
| --- | --- | --- | --- | --- | --- |
| 20.6.6 | S2-2508372 | P-CR | Approval | 23.801-01: [WT#6] Scope and key issue on computing. | NTT DOCOMO |
| 20.6.6 | S2-2508477 | P-CR | Approval | 23.801-01: [WT#6] 6G Computing Support. | [OPPO, China Unicom,] China Telecom, [China Mobile, MediaTek, NTT DOCOMO, Verizon, AT&T, Intel, Lenovo, CATT, vivo, Ofinno, InterDigital, Toyota, ETRI, LG Electronics, NEC, ZTE, Huawei, HiSilicon, Futurewei, Rakuten Mobile, SoftBank, SK Telecom, Xiaomi, K |
| 20.6.6 | S2-2508517 | P-CR | Approval | 23.801-01: [WT#6] Discussion and proposal for computing. | ZTE |
| 20.6.6 | S2-2508574 | P-CR | Approval | 23.801-01: [WT#6, Computing] WT and KI for 6G computing. | Huawei, HiSilicon |
| 20.6.6 | S2-2508589 | P-CR | Approval | 23.801-01: [WT#6] 6G Computing Support. | [OPPO, China Unicom, China Telecom, China Mobile, MediaTek, NTT DOCOMO, Verizon, AT&T, Intel, Lenovo, CATT, vivo, Ofinno, InterDigital, Toyota, ETRI,] LG Electronics[, NEC, ZTE, Huawei, HiSilicon, Futurewei, Rakuten Mobile, SoftBank, SK Telecom, Xiaomi, K |
| 20.6.6 | S2-2508627 | P-CR | Approval | 23.801-01: [WT#6] WT and key issues for supporting 6G computing. | China Mobile |
| 20.6.6 | S2-2508649 | DISCUSSION | Endorsement | Requirements for Computing Service in 6G | AT&T Labs, Inc |
| 20.6.6 | S2-2508698 | P-CR | Approval | 23.801-01: [WT#6, Computing] Working Scope and Key Issues for WT#6 Computing. | Lenovo |
| 20.6.6 | S2-2508898 | P-CR | Agreement | 23.801-01: [WT#6, Computing] Support computing in 6G. | CATT |
| 20.6.6 | S2-2509004 | P-CR | Approval | 23.801-01: [WT#6] 6G Computing Support. | Rakuten Mobile |
| 20.6.6 | S2-2509093 | P-CR | Approval | 23.801-01: [WT#6, Computing] WT update & KI proposal for 6G Computing. | Apple |
| 20.6.6 | S2-2509216 | P-CR | Approval | 23.801-01: Discussion and way forward for WT#6 (compute support). | Qualcomm Incorporated, Ericsson |
| 20.6.6 | S2-2509225 | P-CR | Approval | 23.801-01: [WT#6, computing] supporting for computing service. | Xiaomi |
| 20.6.6 | S2-2509238 | P-CR | Approval | 23.801-01: [WT#6] 6G Compute work task scope and key issue. | Nokia, T-Mobile US |
| 20.6.6 | S2-2508602 | P-CR | Approval | 23.801-01: [WT#6 Computing] WT update. | Samsung |

## 3.1. Removal of NOTE 1 and NOTE 2

NOTE 1 and 2 have been well covered by a newly consolidated NOTE 3, thus they are removed.

## 3.2 Location of the computing resource

It’s proposed by S2-2509238 and S2-2509093 to make the example for the location of computing resource in the brackets more general, as such the description is generalized.

## 3.3 Removal of the EAS discovery defined in 23.548

It’s proposed by S2-2508627 and S2-2508649 not to restrict the computing side/Application Server discovery with 5G design and keep the description general, as such the description is generalized.

## 3.4 Potential coordination with other WTs

S2-2509238 proposed to address the potential coordination with WT#1.1 on service enablement and WT#1.2 on service exposure, the exposure aspect has been covered by a newly added NOTE 3.

S2-2508574 proposed to address the potential coordination with WT#1.2 on QoS Framework and User Plane, these aspects have been covered by a newly consolidated NOTE 3.

S2-2508602 proposed to address the potential coordination with WT#1.1 and UE-CN interaction and WT#5 on computing service related data. W.r.t. the WT#5 on the computing service related data, the computing service related data was never discussed in this WT, it’s unclear what’s the relationship with WT#5. Thus on the potential coordination with WT#1.1 is addressed in new NOTE 5.

## 3.5 Computing site/resource

S2-2508698 proposed a new NOTE to leave the explanation of computing site and computing resource to study phase, while S2-2509238. S2-2508898 and S2-2508372 proposed a new NOTE to illustrate the computing site and it’s relationship with computing resource and the location, a new NOTE 6 is added to reflect this request.

## 3.6 Detailed description on identification of computing service provided by 6G CN

S2-2508602 proposed to add a new sub-WT on this aspect. This aspect is currently covered by a sub-bullet under WT#6.2, there was comment at SA2#170 to make the description more general and avoid hinting solution, but the proposed new sub-WT looks orientation, as such the proposal is not adopted.

## 3.7 Monitor or provision the computing resource

S2-2508698 proposed to use ‘monitor’, S2-2509093 proposed to use ‘provision’, considering the computing resource may be dynamically changed, ‘monitor’ seems more suitable than ‘provision’, the change is reflected.

## 3.8 Exposure of computing resource related information

S2-2508898 proposed to clarify the exposure is to UE or AF, which is adopted.

## 3.9 Proposal in S2-2509216

The first bullet has been addressed by WT#6.1 and WT#6.3 in a clearer way.

On the second bullet, the newly numbered NOTE 1 and 2 have addressed the potential coordination with SA5 and SA6, and also the gay analysis on SA2 defined EC feature for 5GS.

Whether gap analysis for WT#6 should be done before kicking off the solution development should be further discussed at F2F meeting unless there is clear guidance that the gap analysis should be done for all WTs. Thus this aspect is not adopted.

## 3.10 Policy control for computing service

It’s proposed by S2-2508698, this aspect has been covered by WT#6.1 in the postposed version.

## 3.11 Proposal in S2-2509225

It’s not based on the postponed version, but all aspects have been addressed in the postponed version. W.r.t. the architecture aspect, this should be the outcome of solution development for WT#6.2.

## 3.12 Configuring the required computing service from UE/AF to the computing site/Application server

It’s proposed by S2-2508698, this aspect has been covered by WT#6.1 in the postponed version.

## 3.13 Proposal to change the leading bullet

It’s proposed by S2-2508602. The original text was a compromise during SID discussion, unless there is big issue, let’s avoid changing it.

## 3.14 Service Continuity on changing access path

It’s mentioned in S2-2509093 and not explicitly stated what access path it refers to besides the 3GPP access. Per offline check with the author, it’s about the path switch between 3GPP access and non-3GPP access. Currently there is no description on use case and requirement in SA1 6G TR on support of computing via non-3GPP access, this needs to be first discussed in SA1.

## 3.15 Definition of the computing service and computing resource

SA1 has definition in TR 22.870:

6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party.

NOTE 1: The computing resources can refer to hardware and/or software that provides the required processing, storage capability etc. to perform computational tasks (e.g. XR rendering).

SA2 can use this definition as starting point to further polish it.

**** First Change (all new text) ****

# Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

WT#6.1:  Definition of the computing service and its corresponding computing resource:

  - Whether and how to define the computing service.

  - Whether and how to define the computing resource (e.g., computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:  The term of Computing Service in TR22.870 can be used as starting point.

WT#6.2: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.3: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource (e.g., within or outside the core network).

  - Whether and how to identify, collect/monitor, provision the computing resource related information.

  - Whether and how to expose the computing resource related information and/or network metrics.

.

  - Whether and how to improve the service experience/QoS (e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.4: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

WT#6.5: Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G

NOTE 2:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 3: Potential coordination with other WTs (e.g., WT#1.1 on service enablement and UE-CN interaction, WT#1.2 on QoS, exposure framework and UP aspects, etc.) may be needed.

NOTE 4:	Computing site represents the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a computing service consumer. From networking perspective, the computing site is located within 6G CN (via user plane function in 6G) or Data Network.

**** Second Change (all new text, intending to be same with WT description) ****

## 5.X	Key Issue #X: 6G Computing Support

Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource (e.g., within or outside the core network).

  - Whether and how to identify, collect or monitor the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource related information and/or network metrics to UE or AF.

  - Whether and how to improve the service experience/QoS (e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

  - Whether and how to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 1:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 2: Potential gaps on SA2 defined 5G Edge Computing design may be identified in study phase.

NOTE 3: Potential coordination with other WTs (e.g., WT#1.1 on service enablement and UE-CN interaction, WT#1.2 on QoS, exposure framework and UP aspects, etc.) may be needed.

NOTE 4:	Compute site represents the compute resources in a specific location of network topology. The compute site can offer compute services by allocating its compute resources to run application workloads upon demand of a compute service consumer. From networking perspective, the compute site may be in the Service Hosting Environment inside the 3GPP network or in a location of a data network identified by a DNAI.

**** End of Changes ****