3GPP TSG-SA6 Meeting #71	S6-260411

# Goa, India, 9th - 13th Feb 2026		   		                                         (revision of S6-260026)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0698 | rev | 3 | Current version: | 19.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to location management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2026-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CT1 has introduced a new TS 24.283 Mission Critical Location Management (MCLoc); Protocol specification. The LMC-LMS interface (CSC-14 reference point) in this new protocol specification utilizes HTTP only. Same change is applicable also for CSC-22 (LMS-LMS) and CSC-23 (LMS-gateway server). For CSC-15 (LMS-MCS server), a clarification is needed that SIP is used only for group dynamic data subscribe/notify. TS 23.280 needs to be updated with these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove SIP reference points from CSC-14, CSC-22 and CSC-23. For CSC-15, clarify that SIP is used only for subscribe/notify of group dynamic data. Additionally, small errors in the clauses affected are corrected: - In figure 7.3.1.2-3, HTTP-2 is added to CSC-5. Also, for better readability, some text in the figure moved to correct locations. - Minor corrections to 10.9.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2, 7.5.2.14, 7.5.2.15, 7.5.2.23, 7.5.2.24, 10.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.3.1.2	Functional model for an MC system

Figure 7.3.1.2-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1.2-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1.2-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:	The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

NOTE 2:	The recording server, recording admin client and replay client and mass storage(s) are optional entities. They are not essential to enable MC services. The recording admin client and replay client can also be implemented in devices that do not contain MCPTT / MCData / MCVideo service clients and that are connected to the CSC and recording server using other communication networks than EPS. See subclause 7.3.1.3.

NOTE 3:	The Le reference point is not an essential reference point to enable MC services. It is only required if location information of MC service UE(s) is obtained from the PLMN operator via the LCS.

In the model shown in figure 7.3.1.2-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1.2-2 shows the functional model for the signalling control plane.

Figure 7.3.1.2-2: Functional model for signalling control plane

Figure 7.3.1.2-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1.2-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 4:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 5:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1.2-3.

NOTE 6:	CSC-5, CSC-15, CSC-24, REC-3, REC-4 and REC-5 make use of SIP-1 and/or SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1.2-3.

# * * * * 2nd change * * * *

#### 7.5.2.14	Reference point CSC-14 (between the location management client and the location management server)

The CSC-14 reference point, which exists between the location management client and the location management server, is used by the location management server to receive location information report from location management client.

The CSC-14 reference point uses the HTTP-1 and HTTP-2 reference points.

#### 7.5.2.15	Reference point CSC-15 (between the location management server and the MC service server)

The CSC-15 reference point, which exists between the location management server and the MC service server, is used by the MC service server to request and receive location information from location management server. This reference point is also utilized by the location management server to retrieve the group dynamic data (affiliation status) from the MC Service server for a specific MC Service Group ID as well as functional alias status for a specific functional alias.

The CSC-15 reference point uses SIP-2 reference point for transport and routing of group dynamic data subscription/notification related signalling. For all other signalling, the CSC-15 reference point uses the HTTP-1 and HTTP-2 reference points.

# * * * * 3rd change * * * *

#### 7.5.2.23	Reference point CSC-22 (between location management servers in different MC systems)

The CSC-22 reference point, which exists between location management servers in different MC systems, is used to share location information related signalling for location management for interconnected MC systems.

The CSC-22 reference point uses the HTTP-1, HTTP-2 and HTTP-3 reference points.

#### 7.5.2.24	Reference point CSC-23 (between location management server and MC gateway server)

The CSC-23 reference point, which exists between location management server and MC gateway server, is used to handle location management related signalling.

The CSC-23 reference point uses the HTTP-1, HTTP-2 and HTTP-3 reference points.

# * * * * 4th change * * * *

### 10.9.1	General

Location information of MC service user shall be provided by the location management client to the location management server. The location information may also be provided by the 3GPP core network (i.e., via the PLMN operator). The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

NOTE 1:	It is up to implementation whether location information reporting from the core network is considered as an optional or primary source of location information to the MC system.

The location information provided by the 3GPP core network can be obtained from the location services feature in EPS (i.e., LCS) as described in 3GPP TS 23.271 [29], or from the PCRF as part of the PCC mechanism once the MC service server subscribes to related notifications from the PCRF as described in 3GPP TS 23.203 [8]. An authorized MC service user may subscribe to event-triggered location information of one or more MC service users. By default, a subscribed MC service user will receive location information based on the location reporting configuration in the location user profile (see A.8) of the reporting MC service user.

An authorized and subscribed MC service user may initiate a trigger request procedure to modify the location reporting triggers for one or more reporting MC service users. The location management server aggregates trigger requests from the MC service servers and from location management clients towards the same target location management client and may adjust the location reporting triggers of the reporting MC service users as required.

When a subscribed MC service user cancels the subscription, the location management server stops delivering location reports to this MC service user and the location management server may adjust the aggregated location reporting triggers of the reporting MC service users as required.

The location management client can be configured to store location information while not reporting location information to the location management server. The location management client may report stored location information to the location management server.

NOTE 2:	Configuration and location information transmission may occur while there is a communication link between the location management client and location management server.

In the information flows of clause 10.9.2, and procedures of clause 10.9.3:

-	the term "requesting MC service user" is used to refer to the MC service user which sends a request to subscribe to location information reports, to trigger a location information report or to modify existing location information triggers from another MC service user;

-	the term "requested MC service user" is used to refer to the MC service user whose location information is requested or existing location information triggers are to be modified; and

-	the term "reporting MC service user" is used to refer to the MC service user which provides location related information based on on-demand request or trigger criteria.

# * * * * End of changes * * * *