3GPP TSG-SA WG6 Meeting #70	S6-255423

Dallas, United States 17th – 21st November 2025	(revision of S6-255277)

Source:	Samsung

Title:	Pseudo-CR on New solution to Key Issue #2

Spec:	3GPP TR 23.700-43 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	Narendranath Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR proposes new solution to Key Issue #2, to handle the control of usage of API provider domain service API information considering multi tenant CAPIF scenarios.

2. Reason for Change

New solution is proposed to Key Issue #2, enabling the API provider to control the usage of API provider domain service API information, by providing the permission information using the existing service API publish procedure. This solution address the open issue on API provider’s permission to usage of service API information.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 v0.2.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |
| Sol #X |  | X |  |  |  |  |  |
| Sol #... |  |  |  |  |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Permission to usage of service API information

### 6.x.1	General

This solution relates to KI #2.

KI#2 identifies the use case of controlling the usage of an API provider domain’s service API information such as Service API invocation logs, for analytics services to application entities outside of API provider domain. The open issue is captured on how the API provider permits the usage of service API information.

This solution proposes the enhancement of CAPIF Publish Service API procedure with permission information. The APF includes the permission information in the Service API Publish Request message, as specified in Publish service APIs procedure in clause 8.3 of 3GPP TS 23.222 [2]. The permission information is used by CCF in considering the API provider domain’s service API information for offering analytics (e.g., generating API analytics) or any other service offerings.

### 6.x.2	Architecture Impacts

This solution enhances the existing Publish Service API procedure. There are no new functional elements or reference points introduced in this solution. There are no architecture impacts.

### 6.x.3	Solution description

This solution enhances the existing Publish Service API procedure with permission information. The enhancements proposed to Publish service APIs procedure in clause 8.3 of 3GPP TS 23.222 [2] are shown in below (indicated in bold).

Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function. Pre-conditions: 1. Authorization details of the APF are available with the CAPIF core function. 2. API invokers may have subscribed with the CAPIF core function to obtain new service API information. Figure 8.3.3-1: Publish service APIs 1. The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API. If the service API is to be shared to other CAPIF core functions, the shareable information and the CAPIF provider domain information are included. The request may include Permission Information, indicating to the CCF to apply the Permission Information for the usage or sharing of the requesting APF’s API provider domain’s service API invocation logs for offering of services (e.g., analytics) to entities outside of API provider domain. - 2. Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information and permission information provided by the API publishing function is stored at the CAPIF core function (API registry). CCF applies the Permission Information for the usage of the requesting APF’s API provider domain service API invocation logs for offering of services (e.g., analytics) 3. The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

#### 6.x.3.1	Information flows

This solution proposes the update to Service API publish request in clause 8.3.2.1 of 3GPP TS 23.222 [2], with permission information. The updates are shown below (in bold).

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 2); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods.. Shareable information; O (see NOTE 1); Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained.. Permission Information; O; List of permissions related to the usage of API provider domain’s service API invocation logs.. >permission value; M; Permission issued: “Allowed”, “Not Allowed”.. NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs of 3GPP AEFs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19].. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use..

Editor’s Note: Whether and how to indicate the target identifiers information for which the permission value is applicable is FFS.

### 6.x.4	Solution evaluation

Editor’s Note: The solution evaluation is FFS.

* * * End of Changes * * * *