# 3GPP TSG-SA WG6 Meeting #70	S6-255574

# Dallas, USA, 17 - 21 November 2025                                            (revision of S6-255151)

Source:	ZTE Corporation

Title:	Update Solution#2 for KI#1

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI#1 on support for sensing results exposure procedure.

2. Reason for Change

This paper proposes a solution to KI# 1 on support for sensing results exposure procedure. This solution addresses the key issue on how to acquire the sensing results from the appropriate sensing service supplier and expose the (enhanced) sensing results to the sensing consumer.

According to the latest interim agreements of SA2 on Authorization in TR23.700-14 as below:

7.1.2 Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service The following principles are agreed for KI#2: a) The Sensing Service authorization consists of two steps: 1) Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9]; 2) Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality; NOTE 1: In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway, and the wording may need alignment according to the conclusion of key issue 1. b) Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any, and further details on Sensing authorization information can be determined during the normative phase. Table 7.1.2-1: Sensing Authorization information for Sensing Service AF Authorization Data; Description. AF ID; Identifier used to identify the AF.. Allowed/Not allowed area for sensing; Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations. (Note 1). Allowed/Not allowed time period for sensing; Indicate the allowed time period within which AF can trigger a particular sensing service operation. (Allowed) sensing service type; Indicate the allowed sensing service type (e.g., object tracking, detection) for the indicated AF to trigger the sensing services operations.. Note 1: the Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed.. c) The Sensing Authorization information is pre-configured or configured by OAM, and stored in Sensing authorization functionality for authorisation of the sensing service request. d) Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. e) The AF can cancel the on-going Sensing Service Request when needed. f) There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

In this paper, more details on sensing service subscription procedure are provided based on above agreements to make alignment with SA2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

6.3	Solution #2: Support the sensing results exposure based on the sensing subscription request

6.3.1	Solution description

This solution aims to address the issues identified in Key Issue 1 and provides a possible procedure to perform the sensing results exposure.

Editor's note: This solution is built on functional architecture described in solution X for key issue 1. The sensing capabilities in the enabler layer may be provided by a separate SEAL sensing server or directly by the specific SEAL/VAL enabler server. The decision on whether a separate SEAL sensing server is needed will be based on the evaluation of the solutions and FFS.

In this solution, after acquiring the sensing requirements from the sensing service consumer, a Sensing enabler server will send sensing request to a 5GC NF(e.g.,NEF). After it receives the sensing results from the 5GC NF, it determines whether the sensing requirements (e.g. the required accuracy of sensing result) are met, if not, it may further select other sensing service supplier(s) to acquire more sensing results, then it may generate enhanced sensing results based on the received sensing results from the 5GC NF and other sensing service supplier(s) and expose them to the sensing service consumer.

In this solution, the sensing service supplier which can provide sensing results can be another 5GC NF or vertical application server.

Editor's note:	Whether the sensing service supplier can be 3rd party AF is FFS.

6.3.2	Procedures

The high-level procedure of sensing service exposure is shown in Figure 6.3.2-1.

Pre-conditions:

- The Sensing enabler server has acquired sensing capability information of one or more Sensing Service Supplier(s).

- The Sensing enabler server has acquired Sensing Authorization information for Sensing Service from CN.

Figure 6.3.2-1: Sensing service exposure procedure

- The VAL Server/Client acting as sensing consumer performs sensing service subscription procedure by sending sensing service subscription request and receiving sensing service subscription response. During the sensing service subscription procedure, the Sensing enabler server can acquire the sensing requirements of the VAL Server/Client such as VAL ID, Sensing Service Area of interest, sensing service type, Sensing target object type, Accuracy requirements, QoS requirements and so on. The Sensing enabler server performs authorization check and determines whether the Sensing Service Area of interest, sensing service type and Sensing target object type is authorized according to the acquired Sensing Authorization information for Sensing Service from CN. Then it sends sensing service subscription response including the authorization information (e.g.,authorized Sensing Service Area, authorized sensing service type and authorized Sensing target object type) to the sensing consumer. If the sensing consumer is authorized to acquire one-time sensing results reporting, the sensing service subscription response includes the allowed time period of triggering sensing request.

- Sensing enabler server sends sensing request to 5GC NF(e.g.,NEF) and receives sensing response. The Sensing enabler server determines the content of the sensing request including the accuracy and QoS requirements in the received sensing service subscription request and the authorized requesting information after authorization check in step1.

Editor's note:	The parameters in the sensing request and response are FFS and should be determined based on SA2’s progress.

- Sensing enabler server receives sensing results from 5GC NF(e.g.,NEF) and determine whether the sensing requirements are met.

- If the sensing requirements are met,  skip to step 8, if not, the Sensing enabler server selects other Sensing Service Supplier(s) according to the sensing capability information of Sensing Service Supplier(s)  and sensing requirement as received in step1 (e.g. by matching sensing capability information of Sensing Service Supplier to the sensing subscription parameters received from the sensing consumer).

- The Sensing enabler server sends sensing request to each selected Sensing Service Supplier and receives sensing response. Different Sensing Service Supplier may be given different sensing request parameters.

Editor's note:	The parameters in the sensing request and response are FFS.

- Each Sensing Service Supplier sends sensing results to the Sensing enabler server based on the sensing request  parameters respectively.

- The Sensing enabler server further generate enhanced sensing results(e.g.,with higher accuracy) based on the received sensing results from the 5GC NF and other Sensing Service Supplier(s).

- The Sensing enabler server sends the (enhanced) sensing results to the VAL Server/Client.

6.3.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *