3GPP TSG-SA WG6 Meeting #70	S6-255607

Dallas, United States 17th – 21st November 2025	(revision of S6-255109)

Source:	ETRI

Title:	Solution for the sensing enabler service procedure for spatial map object detection and tracking services

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Byung Jun AHN <bjahn@etri.re.kr>

1. Introduction

This pCR provides a solution to KI#3,  which addresses how to associate sensing results with spatial map objects and utilize them to detect object and its changes (including movement tracking).

Proposed solution provides procedures for sensing enabler services that detect objects, track target spatial map objects, and provide additional value-added service results based on the collected sensing results.

2. Reason for Change

How to associate sensing results with spatial map objects and utilize them to detect object changes including movement tracking remains an open issue. Solutions are required to address this issue, and the proposed solution is also intended to address this issue.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol #x |  |  | x |  |

* * * Next Change * * * *

6.x	Solution #x: Sensing enabler service procedure for spatial map object detection and tracking

6.x.1	Solution description

6.x.1.0	General

The Spatial Map server can request sensing enabler services for sensing results, object detection, movement tracking to detect object changes, mobility prediction, estimated duration of staying in-range, etc.

The following clause specifies procedures for sensing enabler services that detect objects, track target spatial map objects, and provide additional value-added service results based on the collected sensing results.

6.x.1.1	Procedure

Pre-conditions:

- SEAL Sensing Enabler server has acquired information of Application Specific Layer server(s).

- SEAL Sensing Enabler server can identify the object from sensing results and associate it with a target object, utilizing sensing results from 5GC NF and spatial map information in Sensing enabler service subscribe request.

- SEAL Sensing Enabler server can track the identified object by monitoring the collected sensing results form 5GC NF.

Editor's note:	How to identify the object from sensing results and track it are dependent on what are included in the sensing result(s), and details are FFS.

Figure 6.x.1.2-1 Sensing enabler service procedure for spatial map object detection and tracking

1.	SM server sends a request message to SEAL Sensing Enabler server to subscribe a sensing enabler service. The request includes, as specified in Table 6.x.1.2.1-1, requestor ID, area of interest and object information in the target spatial map, requested sensing service, etc. The requestor (SM server) may subscribe to one of the following services:

a.	The “Object detection service” specifies that the Sensing Enabler server shoud notify the requestor of all objects detected in the area of interest. The notification triggering event type for the service may include event based (e.g., promptly) or periodically (with duration and interval time).

b.	The “Object tracking service” specifies that the Sensing Enabler server shoud notify the requestor when changes to the specified target objects are detected. The Sensing Enabler server monitors changes of target object utilizing sensing result(s) which are collected continuously from the 5GC NF. The notification triggering criteria for the service may include when a target object is added, removed, moved in or out of the given area of interest, or when its position, direction, velocity etc. is changed. The request may also include tracking end condition and notification triggering event type. If the requester only requires detection results for changed objects, the notification filter can specify "Changed objects only". Otherwise, the Sensing Enabler server notifies the SM server of the detection results for all target objects so that it can determine the changes.

c.	The “Enhanced sensing enabler service” specifies that the Sensing Enabler server shoud provide value-added sensing enabler services such as tracking object mobility prediction (e.g., ADAE analytics) and estimated duration for target object to remain in-range. The Sensing Enabler server can use accumulated tracking results and trajectory records, etc., for the prediction and estimation service.

2.	SEAL Sensing Enabler server authorized the SM server, and validates the request. If authorized, the SEAL Sensing Enabler server adds the requestor to the subscriber list of the requested service.

3.	SEAL Sensing Enabler server sends response message with the result. For the successful subscribe result, a subscription ID is included in the message. Otherwise, the response message may include the failure cause.

4.	SEAL Sensing Enabler server interacts with 5GC NF to collect the required sensing results.

5. 	The Sensing Enabler server determines wether sensing results received from 5GC NF meet the requirements to derive desired results for the subscribed service. If the received sensing results meet the requirements, skip to step 7. Otherwise, the Sensing Enabler server may want to use data from the Application Specific layer/VAL layer server.

6.	SEAL Sensing Enabler server interacts with the Application Specific layer/VAL layer server.

7.	The SEAL Sensing Enabler server generates result(s) for the services to which SM server subscribed. For the “Object detection services”, based on the received sensing results, SEAL Sensing Enabler server identifies the object from sensing results and associate it with an existing spatial map object. For “Object tracking services”, the SEAL Sensing Enabler server determines whether the generated object detection results meet the triggering criteria for change of objects condition. If so, go to step 8. Otherwise, step 4 – step 6 are repeated. If SM server requested “Enhanced sensing enabler service” to obtain additional value-added service results, the Sensing Enabler server generates requested service results using accumulated tracking results and trajectory history, etc., to predict the mobility of the tracked object and calculate the expected duration of the target object's stay within the range. If 5G NF does not support periodic reporting and continuous monitoring of tracking objects, for requests that require continuous tracking of target objects (e.g. for mobility prediction, estimated duration of staying in-range, etc.), step 4 – step 6 are repeated.

NOTE:	In this release, SM specific sensing enabler service procedures are provided. If and when these sensing enabler services are available in 3GPP CN sensing function, usage of the 3GPP CN sensing services will be considered.

8.	The SEAL Sensing Enabler server detects the notification triggering event. The types of notification triggering events for each sensing enabler service are as specified in Table 6.x.1.2.1-2, Table 6.x.1.2.1-3 and Table 6.x.1.2.1-4.

9.	The SEAL Sensing Enabler server notifies the SM server which requested the corresponding sensing enabler service. The notification includes subscription ID and notification information. The notification information may include object information detected as requested, and additional value-added service results such as object mobility prediction and estimated duration of stay. The objects in the notification information may include either all objects detected in the given area of interest, or only include changed objects, depending on the type of service requested (e.g. object tracking service with changed objects only indication). For periodic notification, step 4 – step 9 are repeated.

6.x.1.2	Information Flows

6.x.1.2.1	Sensing enabler service subscribe request

Table 6.x.1.2.1-1 shows the Sensing enabler service subscribe request sent by SEAL SM server to SEAL Sensing Enabler server to subscribe to sensing enabler service.

Table 6.x.1.2.1-1: Sensing enabler service subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (i.e., SM server) |
| Expiry time | O | Proposed subscription expiry time |
| Target spatial map information | M | Information on the spatial map targeted by the sensing enabler service |
| > Spatial map ID | O | ID of the target spatial map |
| > Spatial map coverage | O (NOTE 1) | The three-dimensional space coordinates information of the spatial map. |
| > Area of interest | O (NOTE 1) | Three-dimensional area information to detect and tracking objects. This area can be the same or a subset of the spatial map coverage space. |
| > List of objects | O | List of spatial map object information in the area specified (i.e., spatial map coverage or area of interest). |
| >> Object ID | M | The identifier of the corresponding object |
| >> Object type | O | Type of the object. Stationary or trackable, and other classified type based on the object specific attribute information (e.g., specific type of living things, materials, machineries, etc.). |
| >> Object specific information | O | Object specific attribute information (e.g., size, shape, texture, etc.) |
| >> Object mobility information | M | Specifies position, direction, velocity of the object. |
| >>> Object position | O | 3D position of the object. For stationary object, this IE shall be present. For trackable object, 3D position of it shall be provided while it is being tracked, otherwise no value. |
| >>> Direction (NOTE 2) | O | Direction of the trackable object while it’s being tracked, otherwise no value. |
| >>> Velocity (NOTE 2) | O | Velocity of the trackable object while it’s being tracked, otherwise no value. |
| Requested sensing service type | M | Specifies sensing service type and required parameters. |
| > Object detection service | O (NOTE 3) | Detect objects in the given area of interest. Each element contains the information as specified in Table 6.x.1.2.1-2. |
| > Object tracking service | O (NOTE 3) | Detect changes and tracking of specified target objects in a given area of interest. Each element contains the information as specified in Table 6.x.1.2.1-3. |
| > Enhanced sensing enabler service | O (NOTE 3) | Provides value-added sensing enabler services based on enhanced sensing results, etc. Each element contains the information as specified in Table 6.x.1.2.1-4. |
| Sensing capability requirements | O | Capabilities required to obtain desired sensing results for the requested service (e.g., resolution, capabilities required to determine object specific attributes such as size, shape, texture, etc.) |
| NOTE 1: At least one of the sub-elements shall be provided. NOTE 2: These IEs may be present when object is moving or changes its location frequently. NOTE 3: At least one of the sub-elements shall be provided. |  |  |

Table 6.x.1.2.1-2 describes object detection service details for the sensing enabler service subscribe request specified in Table 6.x.1.2.1-1.

Table 6.x.1.2.1-2: object detection service information

| Information element | Status | Description |
| --- | --- | --- |
| Object type | O | Specifies the type of objects to be detected. |
| Object specific attribute information | O | Specifies object specific attribute information to be detected. |
| Notification triggering event type | O | Specifies when to notify the detected objects, possible values are: - Event based (e.g., promptly); or - Periodically (with duration and interval time) |

Table 6.x.1.2.1-3 describes object tracking service details for the sensing enabler service subscribe request specified in Table 6.x.1.2.1-1.

Table 6.x.1.2.1-3: object tracking service information

| Information element | Status | Description |
| --- | --- | --- |
| List of target object IDs | M | Target object IDs to detect changes, to be tracked |
| Triggering criteria | O | Specifies the condition to determine whether the detected event satisfies requirements of requested sensing service. |
| >Change of objects | O | When objects are added, removed, moved in or out of the given area of interest, changed position, direction, velocity etc. |
| Tracking end condition | O | Specifies the condition until when changed target objects are notified (e.g., untll when no more target object to track is left, or specifies the tracking end period (i.e., duration), etc.) |
| Notification triggering event type | O | Specifies when to notify, possible events are: - Whenever change of object is detected, notify it promptly until tracking end condition is met. - Periodically (with duration and interval time) |
| Notification filter | O | Specifies what to notify. |
| >Changed objects only | O | If specified, notify sensing results of changed objects only, otherwise notify sensing results of all target objects so that SM server can determine the changes. |

Table 6.x.1.2.1-4 describes enhanced sensing service details for the sensing enabler service subscribe request specified in Table 6.x.1.2.1-1.

Table 6.x.1.2.1-4: enhanced sensing service information

| Information element | Status | Description |
| --- | --- | --- |
| Type of service | M | Specifies value-added sensing enabler service. Possible services are: - tracking object mobility prediction. - estimated duration for which target object stays in-range. |
| Target object ID | M | Target object ID being serviced |
| Tracking range of interest | O | Range for target object tracking |
| Notification triggering event type | O | Specifies when to notify, possible events are: - When requested sensing enabler service result is ready, notify it promptly. - Periodically (with duration and interval time) |

6.x.1.2.2	Sensing enabler service subscribe response

Table 6.x.1.2.2-1 shows the Sensing enabler service subscribe response sent by SEAL Sensing Enabler server to SEAL SM server as the response to sensing enabler service subscribe request.

Table 6.x.1.2.2-1: Sensing enabler service subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result (i.e., success or failure) of the sensing enabler service subscribe request |
| Subscription ID (NOTE) | O | Sensing enabler service subscription identifier |
| Expire time | O | Subscription expiry time |
| Failure cause | O | Indicates reason for the failure. |
| NOTE: This IE is mandatory when Result IE indicates success. |  |  |

6.x.1.2.3	Sensing enabler service notification

Table 6.x.1.2.3-1: Sensing enabler service notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Sensing enabler service subscription identifier |
| Notification information | M | Specifies requested sensing service result. The objects in the notification information may include either all objects detected in the given area of interest, or only include changed objects, depending on the type of service requested (e.g., object tracking service with changed objects only indication). |
| > List of detected objects | O | Object information detected in the given area of interest. Each element includes the information described below. |
| >> Object ID | O | Detected object identifier |
| >> Object type | O | Type of the object. Stationary or trackable, and other classified type based on the object specific attribute information (e.g., specific type of living things, materials, machineries, etc.). |
| >> Object specific attribute information | O | Object specific attribute information (e.g., size, shape, texture, etc.) |
| >> Object mobility information | O | Specifies position, direction, velocity of the detected object. |
| >> Changes to the object | O | Detected object change information requested by object tracking service |
| >> Sensing result | O | Sensing result received from 5G NF or enhanced sensing result |
| > Estimated duration of stay | O | Estimated duration for which the target object stays in the tracking range of interest. |
| > Object mobility prediction | O | Mobility prediction information of the target object. |

6.x.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Change * * * *