3GPP TSG-SA WG6 Meeting #70	S6-255611

Dallas, United States 17th – 21st November 2025	(revision of S6-255414, S6-255079)

Source:	InterDigital

Title:	KI #6 evaluation and conclusions

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Quang Ly, <quang.ly@interdigital.com>

1. Introduction

This contribution proposes an overall evaluation and conclusions for key issue #6.

2. Reason for Change

KI #6 evaluation is required for overall KI #6 conclusions. Conclusions for KI #6 are also provided.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 10.7	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios

Key Issue #6 studied potential enhancements of AI/ML services where AIML enablement layer is deployed in distributed or hierarchical fashion as indicated in Annex A.3 and A.4 of 3GPP TS 23.482 [2]. The open issues studied in KI #6 are as follows:

1.	Study and identify the use cases and definition of hierarchical and distributed AIMLE architecture.

2.	Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

3.	Whether and how to enhance the architecture and related functions to support for AIMLE services, AIML operation assuming diverse (e.g., hierarchical and distributed) deployments scenarios.

4.	Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

5.	Whether and how the AIMLE services can support the discovery of AIMLE servers.

6.	Whether and how the conditions under which a change of AIMLE server is identified.

There are six solutions that address KI #6 for hierarchical AIMLE deployments. Each of the six solutions address use cases involving AIMLE services that can be provided in hierarchical AIMLE deployments. Solution #9, #14, and #26 complement each other in addressing open issues 2, 3, and 5 for AIMLE server registration and discovery as well as AIMLE client discovery. Solutions #13, #21, and #24 all address open issue 3 but for different aspects of AIMLE services, such as ML model training, ML model collaborative inferencing, and split node related registration procedures. There is no solution that addresses open issues 4 and 6.

Solution #9 addresses open issue 2 by supporting AIMLE servers operating in edge data networks to register to a central AIMLE server in hierarchical AIMLE deployments. Standardized procedures for AIMLE server registration, update, and de-registration are defined. Through these procedures, the central AIMLE server can dynamically maintain awareness of edge AIMLE servers’ capabilities and the set of AIMLE clients connected to them, thereby enabling efficient hierarchical coordination and service discovery.

Solution #13 addresses open issue 3 where a central AIMLE server can support ML model training in hierarchical AIMLE deployments. The procedure allows an ML model to be trained by AIMLE clients distributed in different EDN service areas which are managed by different edge AIMLE servers.

Solution #14 addresses open issues 2 and 3 by enhancing existing AIMLE client discovery to support AIMLE service provisioning in hierarchical AIMLE deployments. New IEs are added to AIMLE client discovery procedure to assist with the discovery of AIMLE clients served by edge deployed AIMLE servers.

Solution #21 addresses open issues 3 by updating existing split node registration, registration update, and de-registration procedures described in 3GPP TS 23.482 [2]. Through these procedures, AIMLE clients can inform an AIMLE server of their capability to participate in split AI/ML operations in hierarchical AIMLE deployments.

Solution #24 addresses open issue 3 where ML model collaborative inference is considered in edge data networks for hierarchical AIMLE deployments. A central AIMLE server distributes inference tasks to multiple edge AIMLE servers for parallel execution to enable efficient use of distributed computing resources by offloading heavy or data-local computation to the edge AIMLE servers.

Solution #26 addresses open issue 5 where AIMLE servers can be discovered by AIMLE service consumers. AIMLE clients or VAL servers are pre-configured with a Default AIMLE server URI to allow the AIMLE service consumers to discover other AIMLE servers when the default AIMLE server is not available to support AI/ML operations.

* * * Next Change * * * *

## 11.x	Conclusions of key issue #6

The following solutions will be considered for normative work for KI #6:

- Solution #9 provides support for AIMLE servers in edge data networks to register to central AIMLE servers in hierarchical AIMLE deployments.

- Solution #13 provides support for ML model training in hierarchical AIMLE deployments.

- Solution #14 provides support for AIMLE client discovery in hierarchical AIMLE deployments.

- Solution #24 provides support for ML model collaborative inference in hierarchical AIMLE deployments.

- Solution #26 provides support for AIMLE server discovery in hierarchical AIMLE deployments.

NOTE:	Whether to unify solution #26 with Clause 8.25 of 3GPP TS 23.482 [2] will be determined during normative phase.

Editor's Note: The conclusion for solution #21 is pending completion of the solution evaluation.

* * * End of Change * * * *