3GPP TSG-SA WG6 Meeting #70	S6-255651

Dallas, USA, 17th – 21st November 2025	(revision of S6-255615)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0336 | rev | 2 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Obtain API Invoker Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of the Obtain API Invoker information service operation in clause 10.6.4 refers to information flows described in clauses 8.16.2.1 and 8.16.2,2. However, the information flows in 8.16.2.1 and 8.16.2.2 are flows related to the interaction between the API invoker and the AEF, instead of the interaction between the AEF and the CCF. Therefore, the information flows for the Obtain_API_Invoker_Info service operation are missing and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of 8.16.2.3 and 8.16.2.4 adding missing information flows. No impact foreseen to Obtain_API_Invoker_Info defined in TS 29.222. Correction of clause 10.6.4 to the appropriate information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications lead to mistinterpretations and implementation mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.2.3 (NEW), 8.16.2.4 (NEW), 8.16.3, 10.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.16.2.3	Obtain API invoker information request

Table 8.16.2.3-1 describes the information flows to request security information of the API invoker using the Obtain API Invoker information request from the AEF to the CAPIF Core Function.

Table 8.16.2.3-1: Obtain API invoker information request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Authentication indication | O (see NOTE) | Indicates whether the consumer requests API Invoker authentication information. |
| Authorization indication | O (see NOTE) | Indicates whether the consumer requests API Invoker authorization information. |
| NOTE: Either the authentication indication or the authorization indication shall be included. |  |  |

* * * Second Change * * * *

#### 8.16.2.4	Obtain API invoker information response

Table 8.16.2.3-1 describes only the information flows returned from the CAPIF Core Function to the AEF with the security method(s) information for the API Invoker.

Table 8.16.2.4-1: Obtain API Invoker information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Security Information | O | This information element shall be present when the result is success. It includes the security information as specified in 3GPP TS 33.122 [12]. |
| Failure Cause | O | This information element may be present when the result is failure. Indicates that the required authorization information with resource owner is required. |

* * * Second Change * * * *

### 8.16.3	Procedure

Figure 8.16.3-1 illustrates the procedure for API invoker authorization to access service APIs.

Pre-conditions:

1.	The API invoker has been authenticated.

2.	The API invoker associated authorization information is available at AEF.

Figure 8.16.3-1: Procedure for API invoker authorization to access service APIs

1.	The API invoker triggers service API invocation request to the AEF, including the service API to be invoked.

NOTE 1:	Authentication can also be performed if not authenticated previously.

NOTE 2:	The API invoker can trigger several service API invocations asynchronously.

2.	Upon receiving the service API invocation request, the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information.

2a.	If the AEF does not have information required to authorize service API invocation, the AEF obtains the authorization information from the CAPIF core function as described in clauses 8.16.2.3 and 8.16.2.4.

3.	The AEF executes the service logic for the invoked service API.

4.	The API invoker receives the service API invocation response as a result of the service API invocation. If in step 2, the AEF determines that the required resource owner authorization information is not available for the requested service API, then the AEF responds to API Invoker with failure status message indicating that the authorization information with resource owner is not available, as specified in Table 8.16.2.2-1.

* * * Second Change * * * *

### 10.6.4	Obtain_API_Invoker_Info operation

API operation name: Obtain_API_Invoker_Info

Description: Obtains the API invoker information.

Known Consumers: API exposing function.

Inputs: Refer subclause 8.16.2.3.

Outputs: Refer subclause 8.16.2.4.

See subclause 8.16.3 for the details of usage of this API operation.

* * * End of Change * * * *