3GPP TSG-SA WG6 Meeting #70	S6-255657

Dallas, United States 17th – 21st November 2025	(revison of S6-255533, S6-255028)

Source:	InterDigital

Title:	New Solution on AIoT information provisioning

Spec:	3GPP TR 23.700-26 v0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Michel Roy, <michel.roy@interdigital.com>

1. Introduction

This contribution proposes a new solution on AIoT service provisioning.

2. Reason for Change

This solution address key issue#3 to introduce a solution for AIoT information provisioning and management of such information.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x | x | x | x |
| Sol #4 |  |  | x |  |
| Sol #5 |  | x | x | x |
| Sol #6 |  | x |  | x |
| Sol #7 |  | x | x | x |
| Sol #8 |  | x |  |  |
| Sol #9 |  | x |  |  |
| Sol #10 |  | x |  |  |
| Sol #11 |  |  | x |  |
| Sol #12 |  | x | x |  |
| Sol #13 |  | x |  | x |
| Sol #y |  |  | x |  |
| Sol #x |  |  |  |  |

* * * Next Change * * * *

## 7.y		Solution #y: Solution for AIoT information provisioning

### 7.y.1	Solution description

#### 7.y.1.1 	Introduction

This solution address key issue#3 to introduce a solution for AIoT information provisioning for the application enablement layer AIoT service.

An AIoT service consumer needs to provision information (application AIoT device profile) in the AIoT Enablement Server (AIOTES), before consuming AIoT services from the AIOTES.

NOTE 1:	 The application AIoT device profile is information known at the application layer consumer (e.g., VAL server or VAL client) about AIoT devices and required by the AIoT Enablement Server (AIOTES) to provide AIoT services to the AIoT service consumer.

NOTE 2:	According to the architecture in Solution #1, the VAL client consumes AIoT services from the AIOTES via the AIoT Enabler Client (AIOTEC).

NOTE 3:	The functionality of the AIOTEC and AIOTES is aligned and avoid conflict with SA2 functionality.

The provisioned application AIoT device profile is stored by the AIoT enablement server (AIOTES) and can be used for requesting operations such as inventory or commands from the network.

This solution defines procedures for:

- Provisioning application AIoT device profile information in clause 7.y.1.2;

- Updating provisioned application AIoT device profile information in clause 7.y.1.3

- Deleting provisioned application AIoT device profile information in clause 7.y.1.4

#### 7.y.1.2 	AIoT information provisioning

The figure 7.y.1.2-1 describes the procedure for provisioning application AIoT device information in the AIOTES.

Figure 7.y.1.2-1: AIoT information provisioning

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning request to the AIOTES. The request includes the requestor identifier and a list of application AIoT device profiles as described in clause 7.y.3.1.

NOTE:	In this release, the AIOTEC is not a UE reader, only acts as an AIoT service consumer and application AIoT device profile information is provided by the VAL layer.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES stores the provided application AIoT device profiles. The application AIoT device profile includes information used by the AIOTES when requesting AIoT operations (e.g., Inventory, Command) from the network as described in 3GPP TS 23.369 [3]. If the application AIoT device profile includes maximum rate, the AIOTES performs AIoT supported operations at a rate that does not exceed the maximum rate when requested by a consumer, and the AIOTES determines to wait if requested to perform operations at a faster rate. If the application AIoT device profile includes maximum transmission size, the AIOTES uses the maximum transmission size to determine the number of interactions required with the AIoT device, and the AIOTES can perform more than one operation if the requested data size exceeds the maximum transmission rate. If the application AIoT device profile includes area information, the AIOTES requests AIoT operations (e.g., Inventory, Command) with the network according to the area information.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.2. If the request was authorized, the response includes an indication of success for the request and the identifiers of application AIoT device profiles that were successfully provisioned. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.y.1.3 	AIoT information provisioning update

The figure 7.y.1.3-1 describes the procedure for updating application AIoT device profiles in the AIOTES.

Figure 7.y.1.3-1: AIoT information provisioning update

- The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning update request to the AIOTES. The request includes the requestor identifier and a list of application AIoT device profiles as described in clause 7.y.3.3.

NOTE:	In this release, the AIoT management client is not an UE reader, only acts as an AIoT service consumer and application AIoT device profile information is provided by the VAL layer.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES updates the application AIoT device profiles identified via their identifier.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.4. If the request was authorized, the response includes an indication of success for the request and the identifier of application AIoT device profiles that were successfully updated. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.y.1.4 	AIoT information provisioning delete

The figure 7.y.1.4-1 describes the procedure for AIoT information provisioning delete.

Figure 7.y.1.3-1: AIoT information provisioning delete

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning delete request to the AIOTES. The request includes the requestor identifier and a list of identifiers of application AIoT device profiles as described in clause 7.y.3.5.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES deletes the application AIoT device profiles information identified via their identifier.

### 3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.6. If the request was authorized, the response includes an indication of success for the request and the identifier of application AIoT device profiles that were successfully deleted. Otherwise, the response includes an indication of failure and may include a failure cause.

### 7.y.2	Architecture Impacts

This solution proposes a functional entity, as an AIoT Enablement Server (AIOTES), is supported in the application enablement layer as described in Solution #1 and Solution #2.

### 7.y.3	Corresponding APIs

#### 7.y.3.1	AIoT Information Provisioning request

Table 7.y.3.1-1 describes the AIoT information provisioning request.

Table 7.y.3.1-1: AIoT Information Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of application AIoT device profiles | M | The list of application AIoT device profile to be provisioned as described in Table 7.y.3.1-2. |

Table 7.y.3.1-2: Application AIoT device profile

| Information element | Status | Description |
| --- | --- | --- |
| Device identifier | M | The identifier of an AIoT device which is used for interactions with the network as described in 3GPP TS 23.369 [3]. |
| Device type | M | The type of the AIoT device (e.g., smart-tag, sensor, etc.). |
| Device capabilities | M | The capabilities of the AIoT device. |
| > Supported operations | M | The operation types supported by the AIoT device (e.g., Inventory, Command) as described in 3GPP TS 23.369 [3]. For Command operation, information about the supported command parameters and the command response size is included. |
| > Maximum rate | O | The maximum rate at which operations can be sent to the AIoT device. This information indicates the maximum rate for requesting operations to the AIoT device and may indicate the energy replenishment time between operations. |
| > Maximum transmission size | O | The maximum amount of data that the AIoT device is capable of transmitting in one operation. This information indicates whether multiple operations may be required to obtain information from the AIoT device. |
| Area information | O | The area information associated with the AIoT device. |
| Device status | M | The status (e.g., enabled, disabled) of the AIoT device. |

#### 7.y.3.2	AIoT Information Provisioning response

Table 7.y.3.2-1 describes the AIoT information provisioning response.

Table 7.y.3.2-1: AIoT Information Provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were provisioned. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.y.3.3	AIoT Information Provisioning Update request

Table 7.y.3.3-1 describes the AIoT information provisioning update request.

Table 7.y.3.3-1: AIoT Information Provisioning Update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of application AIoT device profiles | M | The list of application AIoT device profile to be updated as described in Table 7.y.3.1-2. |

#### 7.y.3.4	AIoT Information Provisioning Update response

Table 7.y.3.4-1 describes the AIoT information provisioning update response.

Table 7.y.3.4-1: AIoT Information Provisioning Update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were updated. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.y.3.5	AIoT Information Provisioning Delete request

Table 7.y.3.5-1 describes the AIoT information provisioning delete request.

Table 7.y.3.5-1: AIoT Information Provisioning Delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of device identifiers | M | The list of device identifiers (application AIoT device profiles) to be deleted. |

#### 7.y.3.6	AIoT Information Provisioning Delete response

Table 7.y.3.6-1 describes the AIoT information provisioning delete response.

Table 7.y.3.6-1: AIoT Information Provisioning Delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were deleted. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.y.4	Solution evaluation

This solution addresses Key Issue #3 on provisioning and managing application AIoT device profiles provided by the VAL layer. The solution allows the VAL layer to provision and manage application AIoT device profiles with the AIoT enablement service (e.g., AIOTES) to enable AIoT operations with the network.

The provisioned information includes application AIoT device profile(s) and enables the AIoT enablement service (e.g., AIOTES) to perform operations (e.g., Inventory, Command) by considering capabilities associated with AIoT devices. The management operations described in the solution proposes procedures for provisioning, updating, and removing application AIoT device profiles.

The solution uses the capabilities of inventory and command operations as described in 3GPP TS 23.369 [3].

The solution is compatible with architectures described in Solution #1 and Solution #2.

* * * End of Changes * * * *