3GPP TSG RAN 6G Workshop Meeting 	6GWS-250040

Incheon, Republic of Korea, March 10-11th, 2025

Title:	Vision and priorities for 6G RAT to support NTN

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	4 	Vision & priorities for next generation radio technology

# Introduction

This document proposes new capabilities for the candidate radio access technology of the 6G system  in order to best support targeted NTN deployment scenarios (characterized by Orbit types, Payload types, service link frequency bands, terminal types, etc..).

# Recommendations on 6G radio interface/access technology capabilities for the support of NTN (Satellite access)

## Radio link characteristics

### GNSS independent operation with GSO and NGSO space segment

- Technical issue:

  - Operation with GSO and NGSO (at an altitude > 300 km) space segment is characterized by extended and variable Doppler and latency

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall support UE without GNSS receiver

    - This should not exclude the support for UEs with GNSS receiver, which are expected to provide better performance.

  - In case of UE without GNSS receiver, the 6G RAT can be configured to provide robust frequency, timing synchronization and framing scheme able to accommodate extended value and variation of latency and Doppler and adapted channel estimation. => NTN capable UE shall be able to operate with 6G RAT NTN without depending on GNSS for UL timing/frequency synchronization (i.e. in initial access, idle and connected modes)

    - This may include the support of connectivity via several satellites simultaneously

### Low multi path propagation

- Technical issue:

  - Propagation channel is mainly ‘quasi’ line of sight with low multi path conditions

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall enable signal configurations for efficient operation under small multi path propagation conditions.

### Extended coverage

- Technical issue:

  - Provide service in light indoor/in vehicle environment

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall be able to provide extended Maximum Coupling Loss compared to 5G NR possibly through extended coverage mode.

### Satellite service allocated spectrum

- Technical issue:

  - Enable operation in all satellite service allocated frequency bands which may be paired and unpaired bands.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall support both FDD and TDD like operation via satellite access (hence compatible with the large and variable RTD).

  - The 6G RAT shall support efficient spectrum coexistence, and possibly sharing between TN and NTN component of a given frequency band.

### Satellite Propagation impairments

- Technical issue:

  - Enable unicast and broadcast operation in specific mobility conditions e.g. helicopter blades, poles along roads or train tracks, tree shadowing,  ..) characterized by deep fades of up to hundreds of ms.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall provide a Robust transmission schemes to mitigate short duration periodic/frequent blockages/fading (e.g. interleaving techniques)

    - Note: Support robust multicast/broadcast transmission without channel state information (CSI).

### Enhanced error floor without HARQ

- Technical issue:

  - Due to the long propagation delay, Satellite communication are typically operated at very low error rate to prevent HARQ which would further degrade the QoS.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall provide a more efficient FEC to operate at a target BLER as low as 10-6 or lower with very low SNR preventing use of re-transmission at physical layer.

## Access related satellite specifics

### Efficient grant free access protocol

- Technical issue:

  - The challenge is to increase the density of terminals willing to transmit quasi simultaneously low bandwidth services via satellite.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall provide an efficient grant-free access scheme for a massive number of simultaneous transmissions/connections especially for low bandwidth services (i.e. messaging) on the uplink of a satellite access. The solution may take as reference the existing two-step random access (2SRA) protocol

### TN/NTN mobility

- Technical issue:

  - Seamless TN/NTN mobility (i.e. hand-over) especially in connected mode with a discrepancy of latency

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall support minimal interruption for real time services. (e.g. simultaneous multi connectivity in parallel with hand-over procedure, enhanced Layer ½ Triger Mobility).

## Space segment constraints

### Extension of satellite foot print

- Technical issue:

  - Limited RF power on board which prevent to illuminate all beams generated by the satellite at a given time.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall provide feature(s) which allow extended periodicity of control channels e.g. SSB like (e.g. Beam hopping techniques).

### Flexible RAN functional split

- Technical issue:

  - On board power is limited and this also applies to regenerative payloads. In addition, the feeder links are constrained typically in terms of variable latency/bandwidth/Switch occurrences. Note that the feeder may also include single or multi-hops inter satellite links.

- Recommendations on 6G radio access capabilities:

  - The 6G RAN architecture should support various base station functional splits able to accommodate the feeder link characteristics and allows to select the minimum set of RAN functions to be embarked on board (in view of the energy limitation) for the mission purpose.

### Enhanced Security

- Technical issue: temporary autonomous onboard 6G functions will require the use of asymmetric cryptographic schemes in order to exchange cryptographic keys. The key exchange and authentication mechanisms need to be resilient against attacks performed on Quantum computers in order to protect the long-term security of the corresponding data links.

- Requirements on 6G radio Interface/access definition:

  - 6G RAT shall support secure networking protocols that use quantum-resistant cryptographic primitives for key-exchange and authentication, potentially supported by classical primitives as hybrid solution.

### 4G NB-IoT_NTN, 5G NR_NTN and 6G radio access coexistence in same beams

- Technical issue:

  - It should be possible under specific conditions/configurations to deploy 6G RAT on satellite networks supporting 5G RAT for migration purposes.

- Recommendations on radio access capabilities:

  - It should be possible to serve 6G UE as well as legacy NTN capable UEs (NB-IoT_NTN and/or NR_NTN) with a space segment in a given beam footprint.

## Service

It is assumed that all services targeted by the 6G system may be supported by the 6G NTNs provided that they require round trip delay and bandwidth compatible with the NTN deployment scenario characteristics.

### Multi connectivity/carrier aggregation

- Technical issue:

  - How to provide broadband connectivity at high reliability with a satellite access operating in high frequency bands characterized by low radio link availability

- Recommendations on 6G radio access capabilities:

  - The 6G RAN shall be able to support simultaneous dual connectivity/Carrier aggregation between

    - low frequency band satellite access technology (with high radio link availability) and high frequency band satellite access technology (with lower radio link availability)

    - satellite and mobile access technology

### GNSS free Positioning Navigation and Timing

- Technical issue:

  - Integrating GNSS into IoT devices operating over satellite networks significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning updates. For such devices, the battery life may be reduced in cases of frequent GNSS position fixes.

- Recommendations on 6G radio access capabilities:

  - The NTN capable UE operating on a satellite access, shall be able to determine its position using 3GPP positioning technology (GNSS independent scheme)

### Enhanced network verified UE location service

- Technical issue:

  - In NTN, the radio cells may span across country borders while some services may be subject to national regulations (Public warning service, emergency communications, lawful intercept, charging). The current (Rel-18 defined) network verified UE location scheme may not be able to determine reliably the UE location with an accuracy of few kms with large beam size

- Recommendations on 6G radio access capabilities:

  - The satellite network shall be able to determine reliably the location of a UE with an accuracy of at least a typical TN cell size

### Broadcasting service

- Technical issue:

  - In NTN, the radio cells may span across country borders while broadcasting services may be subject to national regulations

- Recommendations on 6G radio access capabilities:

  - The 6G system shall provide mechanisms to geo fence a Broadcast service provided by satellite access (i.e. bounded to a targeted geographical area).

## Diversity of Terminals

It is assumed that the 6G system is able to provide connectivity via the NTN access to a wide range of user equipment, each with various power constraints, bandwidth capabilities and usage conditions:

- IoT devices

- pedestrian wearable devices including smartphones

- land/air vehicle-mounted devices

- flying/surface drone-mounted devices

- aeronautical, maritime and/or railway moving platforms

- building mounted devices

- as well as NGSO satellite-mounted devices

While most will implement quasi omni-directional some will implement directive antenna e.g. with flat panel antenna able to electronically steer beams towards the satellite(s).

### UE with directive antenna

- Technical issue:

  - Non handheld terminals such a ‘VSAT like’ are characterized by a directive antenna. The terminal shall operate only when the antenna is accurately pointed towards the serving satellite.

- Recommendations on 6G radio access capabilities:

  - The 6G RAT shall provide assistance information to the terminal for the initial antenna pointing.

# Conclusion

Proposal 1: TSG-RAN to take into account the above recommendations for the definition of the 6G radio Interface/access technology.

Proposal 2: TSG-RAN to consider in priority the following recommendations (see chapter “Radio link characteristics”) for the definition of the 6G radio Interface/access technology as part of Rel-20 study /21 work items:

| GNSS independent operation with GSO and NGSO space segment | Satellite service allocated spectrum |
| --- | --- |
| Low multi path propagation | Satellite Propagation impairments |
| Extended coverage | Enhanced error floor without HARQ |

Proposal 3: TSG-RAN to further discuss in which release(s) of 6G the other recommendations should be addressed

| Efficient grant free access protocol | 4G NB-IoT_NTN, 5G NR_NTN and 6G radio access coexistence in same beams |
| --- | --- |
| TN/NTN mobility | Multi connectivity/carrier aggregation |
| Extension of satellite foot print | GNSS free Positioning Navigation and Timing |
| On board power constraints | Enhanced network verified UE location service |
| Flexible RAN functional split | Broadcasting service |
| Enhanced Security | UE with directive antenna |

END