Source:	Vestel Electronics

Title:	A Sensing-Mode Handoff Mechanism for High-Resolution and Accurate Sensing in 6G Networks

Document for:	Discussion

Agenda Item:

Work Item / Release:

Abstract: This contribution presents methods to enable handover between sensing devices. Specifically, it addresses the transition between mono-static and bi-static sensing configurations, or vice versa. The proposed approach operates within a pulse repetition interval (PRI), during which two pulse trains are transmitted sequentially. The first pulse train is transmitted initially, followed by the second. Reflections of these transmitted pulse trains are received and analysed within the same interval to detect their presence and/or location. The decision to perform the handover (HO) is based on the detection results of the second received signal (or pulse), particularly its presence or location. By supporting centralized coordination for global optimization and decentralized operation for local adaptability, this methodology enhances the accuracy and reduces the delay of sensing systems. It enables dynamic transitions between sensing configurations, optimizing performance in varying operational scenarios and network architectures.

# Introduction

The advent of 5G and beyond wireless networks has catalyzed a growing demand for seamless connectivity, driven by the rising requirements of both communication applications and sensing services. This dual demand has spurred significant research into distributed systems and their network-level optimizations. Integrated sensing and communication systems stand out as a promising solution, offering improved efficiency in spectral and hardware resources while enabling a wide range of applications such as autonomous vehicles, smart transportation systems, connected vehicles, virtual reality (VR), and extended reality (XR) in emerging 5G and beyond [1].

ISAC operates through six basic sensing modes: transmission reception point (TRP) monostatic, TRP-TRP bistatic, TRP- user equipment (UE) bistatic, UE-TRP bistatic, UE-UE bistatic, and UE monostatic [2]. These modes can include multiple transmitters or receivers for complex scenarios, making them adaptable to various tasks in next-generation networks. However, most studies focus on basic modes and overlook how environmental changes can influence the preference for certain modes. In dynamic environments, factors like mobility or interference may prefer a different mode, requiring adaptive switching for optimal performance.

According to the 3rd Generation Partnership Project (3GPP) key performance indicators (KPIs) outlined for sensing requirements in 5G and beyond, future systems must achieve ultra-low latency (below 100 ms), high confidence levels (99% for safety-critical applications and 95% for others), and a range resolution of 0.5 meters for object detection and tracking. In addition, accuracy of positioning and velocity estimation by ISAC represent the closeness of measured sensing results of the target object to its true value are typically around 1~10 m and 1~10 m/s respectively [3]. To meet these stringent requirements, the sensing functionality (SF) could be positioned closer to the radio access network (RAN) node, either through direct integration or through embedding [5].

From an ISAC network perspective, current research explores whether base stations (BSs) should independently manage sensing and communication operations or adopt a centralized approach. However, most existing studies assume uninterrupted-communication and sensing capabilities, often overlooking the associated network burden and overhead. Centralized data handling, which involves transferring information from the network edge to a central cloud, necessitates significant scheduling awareness among cooperative BSs and UEs. Addressing these challenges is critical to fully realize the potential of ISAC systems in real-world deployments.

ISAC systems promise transformative improvements in high-accuracy sensing and ultra-reliable low-latency communication (URLLC) applications within 5G and beyond networks. To evaluate their performance, this work considers both stand-alone and integrated system configurations, highlighting their respective advantages and challenges.

# Handover Decision Algorithm

The approach of the present contribution makes the decision to handover (HO) a sensing mode from monostatic to bi-static or from bi-static to monostatic, based on use cases, requirements and sensing KPIs (ambiguity, distance, and resolutions), by transmitting two pulse trains. The participating sensing devices include a base station (BS), a user equipment (UE), or a central network (CN).

The pulse trains can differ using techniques like unique coding, modulation separation, or TOA-based separation. Identification can be achieved by analyzing factors such as SINR, pulse timing, and TOA differences to decide whether a HO should occur.

FIG. 1: The present approach assists the sensing modes HO in ISAC-Network.

FIG. 2:  Different scenarios for sensing modes HO based on sensing range ambiguity.

# Sensing measurements, reporting and cooperative scheduling

### Decentralized approach

A HO from monostatic to bi-static sensing may be needed dependent on the detected presence and/or location of the first and second reception signal within the pulse repetition interval.

Each scenario needs to be continuously supported by sensing services with low sensing latency and resolution.

Service Flow: HO Execution for Sensing

Fig. 3 shows a monostatic and bistatic scenario of the HO approach of the present approach in decentralized networks.

- Sensing Request Initiation: The system receives a sensing service request.

- Base Station (BS) Selection: The closest BS is selected to perform sensing.

- Pulse Train Transmission: The selected transmitting BS (Tx-BS) sends two pulse trains.

- Pulse Echo Reception: The receiving BS (Rx-BS) captures the echoes of the two pulses.

- Parameter Estimation and HO Decision

  - The Rx-BS estimates the sensing parameters (distance, velocity, or RCS) and evaluates range ambiguity.

  - Based on these evaluations, a HO decision is made.

- HO Preparation:

  - An HO request is sent, and preparations for the HO begin.

  - Sensing continues until acknowledgment is received from the Rx-BS to proceed with bistatic sensing.

- HO Execution: The HO is executed, transitioning to bistatic sensing operations.

FIG.3:  Monostatic to bistatic HO.

### Centralized approach

Figure 4 illustrates the setup for leveraging sensing mode HO for centralized network, highlighting specific aspects of the proposed approach. The setup outlines the sensing process, utilizing centralized nodes to achieve efficient sensing KPIs while maintaining user communication requirements.

FIG.4 The network setup

### Service Flow: HO Execution for Sensing

In centralized HO decision, a higher network entity, such as a central node of a network decides on whether a HO is needed. Each scenario needs to continuously be supported by sensing services with the lowest sensing latency and high resolution.

- Sensing Request Service: The process begins with a sensing service request, initiating the need for object detection or environment monitoring.

- Network Monitoring: The system monitors available base stations (BSs) and selects one capable of either performing sensing signal or receiving sensing data based on availability and network conditions.

- Pulse Transmission: The transmitting BS (Tx-BS) sends two pulse trains as part of the sensing operation.

- Pulse Reception: The receiving BS (Rx-BS) receives echoes of the transmitted pulses.

- Data Processing: The Rx-BS forwards the received data to a cloud-RAN (CRAN) for analysis. The CRAN estimates the sensing parameters (AoA, AoD, ToA, RCS, range, velocity), resolves ambiguity, and determines the need for HO.

- Sensing HO Decision: A decision is made based on CRAN's analysis to either continue with the current configuration or initiate a sensing HO.

- HO Management: If a HO is required, the network coordinates the sensing mode transition by considering the estimation results and the availability of other BSs.

- HO Execution: The process concludes with the execution of the sensing HO, ensuring a seamless transition to maintain service continuity, transitioning to bistatic sensing operations.

- Conclusion

The proposed method presents a mechanism that seamlessly transitions between monostatic and bistatic sensing modes, addressing critical requirements for ISAC networks, such as ultra-reliability, low latency, high availability, high data rates, precise synchronization, effective interference management, and efficient resource allocation.

Recognizing that 5G and beyond ISAC networks may be implemented using centralized or decentralized architectures depending on application needs, the proposed mechanism supports a smart sensing mode HO applicable to both scenarios.

- References

[1] 3GPP, "TR 22.837 V19.2.0, NR; Study on Integrated Sensing and Communication (Release 19)," 2024.
[2] ETSI, "Integrated Sensing and Communications (ISAC); Use Cases and Deployment Scenarios," ETSI Group Report ISAC 001 V0.0.10, December 2024.
[3] 3GPP, "TS 22.137 V19.1.0, Integrated Sensing and Communication (Release 19)," March 2024.
[4] ETSI, "TP on Sensing Function Considerations and a Potential Approach," ISC(24)000228r2, 2024.
[5] ETSI, "Views on Sensing Measurement Report of ISAC," ISC(24)000266, 2024.
[6] ETSI, "TP on Sensing Function Considerations and a Potential Approach," ISC(24)000228r2, 2024.