Source:	Vestel Electronics

Title:	Range Ambiguity Identification and Elimination by Exploiting Temporal Broadening Effect for 6G THz Sensing

Document for:	Discussion

Agenda Item:

Work Item / Release:

Abstract: This contribution presents a method for identifying range ambiguities in THz-band monostatic sensing by leveraging the distance-dependent temporal broadening effect (TBE) unique to the THz band. The received echo pulse from the intended target is matched to its corresponding transmitted pulse based on the extent of TBE it undergoes.

# Introduction

The next generation of wireless communication networks span a diverse array of applications such as real-time industrial control, remote surgery, remote education, simultaneous imaging, mapping and localization, mission critical extended reality (XR), interactive immersive XR, and cooperative mobile robots that require sensing functionality in addition to conventional communication [1]. To address this, novel sensing prototypes are being developed that can not only operate independently but also integrate seamlessly with existing wireless communication networks. The THz-band emerges as a promising choice for providing high-resolution and precise sensing information. Its unique propagation mechanisms, such as enhanced molecular absorption effect, can be intelligently exploited to improve sensing performance.

The short-duration (~femtoseconds) THz pulses can be used to determine the target’s distance, direction (azimuth and elevation angles), and radial velocity. By transmitting a train of pulses and analysing the echoes, THz sensing transceivers can estimate the potential target’s sensing parameters (range and Doppler). The maximum detection range of such systems depends on the transmit power and the duration of the transmitted pulses [2]. The objects located beyond this range can lead to ambiguities, as the delay of echoes from distant targets may exceed the pulse repetition interval (PRI), complicating the accurate determination of their positions.

# Range Ambiguity in Monostatic Sensing

Range ambiguity in monostatic sensing refers to the difficulty in accurately determining the true range of a target when multiple pulses are transmitted and received in rapid succession. This occurs when the time delay between the transmission of a pulse and the reception of its echo is such that the sensing system cannot distinguish whether the echo corresponds to the most recent pulse or a previous one. Fig. 1 demonstrates a monostatic sensing scenario where a target is moving outside the unambiguous region, while Fig. 2 explains range ambiguity problem.

Fig. 1: Illustration of monostatic sensing and the range ambiguity.

Fig. 2: Range ambiguity in pulse-based monostatic sensing.

# Relation of Range Ambiguity and TBE

Molecular absorption is one of the critical challenges faced by THz frequencies which is dependent on distance and frequency [3]. In the time domain, molecular absorption manifests as a temporal broadening effect (TBE), leading to time dispersion of the transmitted pulses. For a given frequency, the distance-dependent TBE in the THz band can be leveraged to link a reflected received pulse to its corresponding transmitted pulse. By estimating the maximum pulse broadening within a PRI, based on the maximum unambiguous range, any received echo that exceeds this broadening threshold is not associated with the transmitted pulse in the same PRI. Instead, it is mapped to the appropriate transmitted pulse, considering the extent of broadening it has undergone, thereby avoiding range ambiguity. Range ambiguity identification based on TBE is illustrated in Fig. 3.

Fig. 3: Range ambiguity identification based on TBE.

- Conclusion

The bandwidth abundant THz band facilitates the provision of high-resolution sensing. However, THz-band specific molecular absorption and TBE, which is considered detrimental for communication, can be exploited for accurate sensing and detect and mitigate the effect of range ambiguity in monostatic pulse-based sensing systems which arises when an object/target moves beyond the intended coverage area of the sensing transceiver.

References

[1] ETSI GR ISAC 001, “Integrated Sensing and Communications (ISAC); Use Cases and Deployment Scenarios,” V0.0.6, November 2024.

[2] Levanon, Nadav. "Mitigating range ambiguity in high PRF radar using inter-pulse binary coding." IEEE Transactions on aerospace and electronic systems 45.2 (2009): 687-697.

[3] THz (24)000067, “Discussion on Molecular Absorption Modelling in THz band”, Apple, France.