3GPP TS 38.115-2 V19.0.0 (2025-09)

Technical Specification

3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Repeater conformance testing - Part 2: Radiated conformance testing; (Release 19)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	12

1	Scope	14

2	References	14

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Symbols	18

3.3	Abbreviations	19

4	General radiated test conditions and declarations	20

4.1	Measurement uncertainties and test requirements	20

4.1.1	General	20

4.1.2	Acceptable uncertainty of Test System	20

4.1.2.1	General	20

4.1.2.2	Radiated characteristics measurements	20

4.1.3	Interpretation of measurement results	23

4.2	Radiated requirement reference points	23

4.2.0	NCR type 1-H	23

4.2.1	RF repeater type 2-O	24

4.2.2	NCR type 2-O	25

4.3	RF repeater classes	26

4.3.1	RF repeater class for downlink	26

4.3.2	RF repeater class for uplink	27

4.3A.1	NCR class for downlink	27

4.3A.2	NCR class for uplink and MT	27

4.4	Regional requirements	28

4.5	Repeater configurations	28

4.5.1	Downlink configurations	28

4.5.2	Uplink configurations	28

4.5.3	Power supply options	29

4.6	Manufacturer's declarations	29

4.7	Test configurations	39

4.7.1	General	39

4.7.2	Test signal configurations	39

4.7.2.1	Test signal used to build Test Configurations	39

4.7.2.2	RTC1: Contiguous spectrum operation for RF repeater type 2-O	40

4.7.2.2.1	RTC1 generation	40

4.7.2.2.2	RTC1 power allocation	40

4.7.2.4	RTC2: Non-contiguous spectrum operation for RF repeater type 2-O	40

4.7.2.4.1	RTC2 generation	40

4.7.2.4.2	RTC2 power allocation	41

4.8	Applicability of requirements	42

4.9	RF channels and test models	44

4.9.1	RF channels	44

4.9.2	Test models	45

4.9.2.1	General	45

4.9.2.2	FR2 test models for repeater type 2-O for DL	45

4.9.2.3	FR2 test models for repeater type 2-O for UL	45

4.9.2.3.1	FR2 test model 1.1 (RUL-FR2-TM1.1)	46

4.9.2.3.2	FR2 test model 2 (RUL-FR2-TM2)	47

4.9.2.3.2a	FR2 test model 2a (RUL-FR2-TM2a)	47

4.9.2.3.3	FR2 test model 3.1 (RUL-FR2-TM3.1)	47

4.9.2.3.4	FR2 test model 3.1a (RUL-FR2-TM3.1a)	48

4.9.2.4	Data content of physical channels and signals for RUL-FR2-TM	48

4.9.2.4.1	PUSCH	48

4.9.3	Test beam direction set	49

4.9.3.1    General	49

4.9.3.2    Test beam direction set for protection of FSS UL	49

4.9A	RF channels and test models of NCR	50

4.9A.1	RF channels	50

4.9A.2	Test models	51

4.9A.2.1	General	51

4.9A.2.2	FR2 test models for NCR type 2-O for DL	51

4.9A.2.3	FR2 test models for NCR type 2-O for UL	51

4.9A.2.3.1	FR2 test model 1.1 (NCRUL-FR2-TM1.1)	52

4.9A.2.3.2	FR2 test model 2 (NCRUL-FR2-TM2)	53

4.9A.2.3.2a	FR2 test model 2a (NCRUL-FR2-TM2a)	53

4.9A.2.3.3	FR2 test model 3.1 (NCRUL-FR2-TM3.1)	53

4.9A.2.3.4	FR2 test model 3.1a (NCRUL-FR2-TM3.1a)	54

4.9A.2.4	Data content of physical channels and signals for NCRUL-FR2-TM	54

4.9A.2.4.2	PUSCH	54

4.10	Requirements for contiguous and non-contiguous spectrum	55

4.11	Format and interpretation of tests	55

4.12	Reference coordinate system	56

5	Operating bands and channel arrangement	57

6	Radiated characteristics	57

6.1	General	57

6.2	OTA output power	58

6.2.1	General	58

6.2.2	OTA output power (EIRP)	58

6.2.2.1	Definition and applicability	58

6.2.2.2	Minimum requirement	59

6.2.2.3	Test purpose	59

6.2.2.4	Method of test	60

6.2.2.4.1	Initial conditions	60

6.2.2.4.2	Procedure	60

6.2.2.5	Test requirement for RF repeater type 2-O and NCR-Fwd type 2-O	61

6.2.2.6	Test requirement for NCR	61

6.2.2.6.1	Test requirement for NCR-MT	61

6.2.2.6.1.1	Test requirement for NCR-MT type 1-H	61

6.2.3	OTA repeater output power (TRP)	62

6.2.3.1	Definition and applicability	62

6.2.3.2	Minimum requirement	62

6.2.3.3	Test purpose	62

6.2.3.4	Method of test	62

6.2.3.4.1	Initial conditions	62

6.2.3.4.2	Procedure	63

6.2.3.5	Test requirement	64

6.2.3.6	Test requirement for NCR	64

6.2.3.6.1	Test requirement for NCR-Fwd type 2-O	64

6.2.3.6.2	Test requirement for NCR-MT	64

6.2.3.6.3	Test requirement for NCR-Fwd	65

6.3	OTA frequency stability	65

6.3.1	Definition and applicability	65

6.3.2	Minimum Requirement	65

6.3.3	Test purpose	65

6.3.4	Method of test	65

6.3.5	Test Requirements for RF repeater and NCR	65

6.4	OTA out of band gain	66

6.4.1	Definition and applicability	66

6.4.2	Minimum Requirement	66

6.4.3	Test purpose	66

6.4.4	Method of test	66

6.4.4.1	Initial conditions	66

6.4.4.2	Procedure	66

6.4.5	Test Requirements for RF repeater and NCR	67

6.5	OTA unwanted emissions	67

6.5.1	General	67

6.5.2	OTA Adjacent Channel Leakage Power Ratio (ACLR)	68

6.5.2.1	Definition and applicability	68

6.5.2.2	Minimum requirement	68

6.5.2.3	Test purpose	68

6.5.2.4	Method of test	68

6.5.2.4.1	Initial conditions	68

6.5.2.4.2	Procedure	68

6.5.2.5	Test requirements	69

6.5.3	OTA operating band unwanted emissions	74

6.5.3.1	Definition and applicability	74

6.5.3.2	Minimum requirement	75

6.5.3.3	Test purpose	75

6.5.3.4	Method of test	75

6.5.3.4.1	Initial conditions	75

6.5.3.4.2	Procedure	75

6.5.3.5	Test requirements	77

6.5.3.5.1	OTA operating band unwanted emission limits (Category A)	77

6.5.3.5.2	OTA operating band unwanted emission limits (Category B)	78

6.5.3.5.3	Additional OTA operating band unwanted emission requirements	79

6.5.4	OTA transmitter spurious emissions	79

6.5.4.1	Definition and applicability	79

6.5.4.2	Minimum requirement	79

6.5.4.3	Test purpose	80

6.5.4.4	Method of test	80

6.5.4.4.1	Initial conditions	80

6.5.4.4.2	Procedure	80

6.5.4.5	Test requirements	81

6.5.4.5.1	General	81

6.5.4.5.2	OTA transmitter spurious emissions (Category A)	81

6.5.4.5.3	OTA transmitter spurious emissions (Category B)	82

6.5.4.5.4	Additional OTA transmitter spurious emissions requirements	82

6.5.4.5.4.1	Limits for protection of Earth Exploration Satellite Service	82

6.6	OTA Repeater Error Vector Magnitude	83

6.6.1	Downlink repeater error vector magnitude	83

6.6.1.1	General	83

6.6.1.2	Minimum requirements	84

6.6.1.3	Test purpose	84

6.6.1.4	Method of test	84

6.6.1.4.1	Initial conditions	84

6.6.1.4.2	Procedure	84

6.6.1.5	Test requirement	85

6.6.2	Uplink Repeater error vector magnitude	86

6.6.2.1	General	86

6.6.2.2	Minimum requirement	87

6.6.2.3	Test purpose	87

6.6.2.4	Method of test	87

6.6.2.4.1	Initial conditions	87

6.6.2.4.2	Procedure	87

6.6.2.5	Test requirement	88

6.7	OTA input intermodulation	88

6.7.1	Definition and applicability	88

6.7.1.1	General	88

6.7.1.2	Minimum requirements	89

6.7.1.3	Test purpose	89

6.7.1.4	Method of test	89

6.7.1.4.1	Initial conditions	89

6.7.1.4.2	Procedure	89

6.7.1.5	Test requirements	89

6.8	OTA Adjacent Channel Rejection Ratio (ACRR)	90

6.8.1	Definitions and applicability	90

6.8.2	Co-existence with NR	90

6.8.2.1	Minimum requirements	90

6.8.2.2	Test purpose	90

6.8.2.3	Method of test	91

6.8.2.3.1	Initial conditions	91

6.8.2.3.2	Procedure	91

6.8.2.3.3	Test Requirements	92

6.9	OTA transmit ON/OFF power	93

6.9.1	OTA transmitter OFF power	93

6.9.1.1	Definition and applicability	93

6.9.1.2	Minimum requirement for RF repeater	93

6.9.1.3	Test purpose	94

6.9.1.4	Method of test	94

6.9.1.5	Test requirements	94

6.9.2	OTA transient period	94

6.9.2.1	Definition and applicability	94

6.9.2.2	Minimum requirement for RF repeater	95

6.9.2.3	Test purpose	95

6.9.2.4	Method of test	95

6.9.2.4.1	Initial conditions	95

6.9.2.4.2	Procedure	95

6.9.2.5	Test requirements	96

6.11.2.1	Frequency error requirements for NCR-MT	99

6.11.2.1.1	Minimum requirement for NCR-MT type 2-O	99

6.11.2.2	Transmit modulation quality	99

6.11.2.2.1	Minimum requirement for NCR-MT type 2-O	99

6.11.4.1	Initial conditions	99

6.11.4.2	Initial conditions	99

6.12.4.1	Initial conditions	102

6.12.4.2	Procedure	102

6.13	OTA maximum input level	103

6.13.1	Definition and applicability	103

6.13.2	Minimum requirement	103

6.13.3	Test purpose	103

6.13.4	Method of test	103

6.13.4.1	Initial conditions	103

6.13.4.2	Procedure	103

6.13.5	Test requirements	104

6.14	OTA adjacent channel selectivity	104

6.14.1	Definition and applicability	104

6.14.2	Minimum requirement	104

6.14.3	Test purpose	104

6.14.4	Method of test	104

6.14.4.1	Initial conditions	104

6.14.4.2	Procedure	105

6.14.5	Test requirements	105

6.15	OTA blocking characteristics	105

6.15.1	Definition and applicability	105

6.15.2	Minimum requirement	106

6.15.3	Test purpose	106

6.15.4	Method of test	106

6.15.4.1	Initial conditions	106

6.15.4.2	Procedure for general blocking	106

6.15.4.3	Procedure for out of band blocking	107

6.15.5	Test requirements	108

6.16	OTA receiver spurious emissions	108

6.16.1	Definition and applicability	108

6.16.2	Minimum requirement	108

6.16.3	Test purpose	108

6.16.4	Method of test	109

6.16.4.1	Initial conditions	109

6.16.4.2	Procedure	109

6.16.5	Test requirements	110

6.17	OTA spatial emission	110

6.17.1	Definition and applicability	110

6.17.2	Minimum requirement	110

6.17.3	Test purpose	110

6.17.4	Method of test	110

6.17.4.1	Initial conditions	110

6.17.4.2	Procedure	110

6.17.5	Test requirements	111

6.17.5.1	Requirement for NCR type 1-H	111

7	Radiated performance requirements	112

7.1	NCR-MT performance requirements	112

7.1.1	General	112

7.1.1.1	Scope and definitions	112

7.1.1.2	OTA demodulation branches	113

7.1.2	Demodulation performance requirements	113

7.1.2.1	General	113

7.1.2.1.1	Applicability rule for NCR-MT	113

7.1.2.2	Performance requirements for PDSCH	114

7.1.2.2.1	Definition and applicability	114

7.1.2.2.2	Minimum requirements	114

7.1.2.2.3	Test purpose	114

7.1.2.2.4	Method of test	114

7.1.2.2.5	Test requirements	116

7.1.2.3	Demodulation performance requirements for PDCCH	116

7.1.2.3.1	Definition and applicability	116

7.1.2.3.2	Minimum requirement	116

7.1.2.3.3	Test purpose	116

7.1.2.3.4	Method of test	116

7.1.2.3.5	Test requirements	118

7.1.3	CSI reporting requirements	118

7.1.3.1	General	118

7.1.3.1.1	Applicability of requirements	118

7.1.3.1.2	Common test parameters	118

7.1.3.2	Reporting of Channel Quality Indicator (CQI)	119

7.1.3.2.1	Definition and applicability	119

7.1.3.2.2	Minimum requirement	119

7.1.3.2.3	Test purpose	120

7.1.3.2.4	Method of test	120

7.1.3.2.4.1	Initial conditions	120

7.1.3.2.4.2	Procedure	120

7.1.3.2.5	Test requirement	122

7.1.3.2.5.1	Test requirement for NCR type 2-O	122

A.1	Repeater stimulus signal 1	123

A.2	Repeater stimulus signal 2	123

A.3	Repeater stimulus signal spectral purity requirements	123

Annex B (normative):	Environmental requirements for the Repeater equipment	124

B.1	General	125

B.2	Normal test environment	125

B.3	Extreme test environment	125

B.3.1	Extreme temperature	125

B.4	Vibration	126

B.5	Power supply	126

B.6	Measurement of test environments	126

Annex C (informative):	Test tolerances and derivation of test requirements	127

C.1	Measurement of radiated characteristics	128

Annex D (normative):	Calibration	131

Annex E (informative):	OTA measurement system set-up	131

E.1	OTA output power EIRP, OTA Frequency stability, OTA Error Vector Magnitude and OTA Transmit ON/OFF power	132

E.2	Out of band gain	133

E.3	Unwanted emission: Operating band unwanted emission and ACLR	134

E.4	Input intermodulation	135

E.5	Adjacent Channel Rejection Ratio	136

E.6 	NCR-MT type 2-O PDSCH and PDCCH testing	136

E.7	NCR-MT type 2-O CQI testing	137

Annex F (normative):	In-channel TX tests	138

F.1	General	139

F.2	Basic principles	139

F.2.1	Output signal of the TX under test	139

F.2.2	Ideal signal	139

F.2.3	Measurement results	140

F.2.4	Measurement points	140

F.3	Pre-FFT minimization process	141

F.4	Timing of the FFT window	142

F.5	Resource element TX power	143

F.6	Post-FFT equalisation	143

F.7	EVM	146

F.7.0	General	146

F.7.1	Averaged EVM	146

Annex G (normative):	TRP measurement procedures	148

Annex H (normative):	 NCR-MT Fixed Reference Channels	148

H.1.1	Fixed Reference Channels for PDSCH performance requirements	148

H.1.2	Fixed Reference Channels for PDCCH performance requirements	148

H.1.3	Fixed Reference Channels for CQI reporting requirements	149

Annex I (normative):	 Propagation conditions	150

I.1	Static propagation condition	150

I.1.1	NCR-MT Receiver with 2 Rx	150

I.1.2	NCR-MT Receiver with 4Rx	151

I.2	Multi-path fading propagation conditions	151

I.2.1	Delay profiles	151

I.2.1.1	Delay profiles for FR2-1	153

I.2.2	Combinations of channel model parameters	153

I.2.3	MIMO channel correlation matrices	154

I.2.3.1	MIMO correlation matrices using Uniform Linear Array (ULA)	154

I.2.3.1.1	Definition of MIMO correlation matrices	154

I.2.3.1.2	MIMO correlation matrices at high, medium and low level	155

I.2.3.2	Multi-antenna channel models using cross polarized antennas	156

I.2.3.2.1	Definition of MIMO correlation matrices using cross polarized antennas	157

I.2.3.2.2	Spatial correlation matrices at NCR-MT and gNB sides	158

I.2.3.2.2.1	Spatial correlation matrices at NCR-MT side	158

I.2.3.2.2.2	Spatial correlation matrices at gNB side	158

I.2.3.2.3	MIMO correlation matrices using cross polarized antennas	158

I.2.3.2.4	Beam steering approach	158

I.3	Physical signals, channels mapping and precoding	160

I.3.1	General	160

Annex J (normative):	161

Expected EIRP calculation and spatial sampling grid	161

J.1	General	161

J.2	Averaging processes	162

J.2.1 Averaging over beam directions	162

J.2.2 Averaging over horizontal and vertical angles	162

J.3 Spatial sampling grid	163

Annex K (informative):	Change history	164

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for RF Repeater type 2-O and network controlled repeater (NCR) type 1-H and 2-O. These have been derived from, and are consistent with the radiated requirements for Repeater type 2-O and network controlled repeater (NCR) type 1-H and 2-O in Repeater specification defined in TS 38.106 [2].

RF Repeater type 1-C and NCR type 1-C  only have conducted requirements so they does not require compliance to this specification.

NCR type 1- H has both conducted and radiated requirements so it requires compliance to this specification and TS 38.115-1 [23].

RF Repeater type 2-O and NCR type 2-O have only radiated requirements so they require compliance to this specification only.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.106: "NR repeater radio transmission and reception"

[3]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[9]	IEC 60 721: "Classification of environmental conditions"

[10]	IEC 60 068-2-1: "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]	IEC 60 068-2-2: "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]	IEC 60 068-2-6: "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

[14]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[15]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[16]	3GPP TS 38.101-4: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[17]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[18]	3GPP TS 38.211: "NR; Physical channels and modulation[19]	3GPP TR 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception"

[20]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone"

[21]	3GPP TS 38.176-2: "NR; Integrated Access and Backhaul (IAB) conformance testing; Part 2: Radiated conformance testing"

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"

[23]	3GPP TS 38.115-1: "NR; Repeater conformance testing, Part 1: Conducted conformance testing"

[24]	Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

[25]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

Beam peak direction: direction where the maximum EIRP is found

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

directional requirement: requirement which is applied in a specific direction within the OTA coverage range.

Equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

Fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

gap between passbands: frequency gap between two consecutive passbands that belong to the same operating band, where the RF requirements in the gap are based on co-existence for un-coordinated operation

Inter-passband gap: The frequency gap between two supported consecutive passbands that belong to different operating bands.

Maximum passband TRP output power: mean power level measured per passband during the transmitter ON state in a specified reference condition and corresponding to the declared rated passband TRP output power (Prated,p,,TRP)

Measurement bandwidth: RF bandwidth in which an emission level is specified

Mechanical tilt: The angle between the vertical reference direction and its projection on BS antenna array plane.

NCR type 1-H: NCR-MT or NCR-Fwd operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

NCR type 2-O: NCR-MT or NCR-Fwd operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

Nominal channel bandwidth: Bandwidth calculated as min(100MHz, BWpassband) in FR1 or min(400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

Non-contiguous spectrum: spectrum consisting of two or more passbands separated by inter-passband gap(s).

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA coverage range: a common range of directions within which OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range.

Passband: The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.

passband edge: Frequency at the edge of the passband.

Repeater RF Bandwidth: RF bandwidth in which a repeater transmits and/or receives single or multiple passband(s) within a supported operating band

NOTE:	In single passband operation, the Repeater RF Bandwidth is equal to the passband bandwidth.

Radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

Rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction during the transmitter ON state

Rated passband TRP output power: mean power level declared by the manufacturer per passband, that the manufacturer has declared to be available at the RIB during the transmitter ON state

Rated total TRP output power: mean power level associated with a particular operating band, that the manufacturer has declared to be available at the RIB during the transmitter ON state in a specified reference condition

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

Repeater type 2-O: Repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Requirement set: one of the NR requirements set as defined for NR repeater

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the repeater.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction.

Transmitter ON state: Time period during which the repeater is transmitting downlink or uplink signals in the corresponding direction.

Transmitter transient period: Time period during which the repeater is changing from the OFF state to the ON state or vice versa.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWNominal	Nominal channel bandwidth

BWPassband	Passband bandwidth

f	Separation between the passband edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the operating band edgeFDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FFBWhigh	Highest supported frequency within supportedoperating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper passband edge

Foffset,low	Frequency offset from FC,low to the lower passband edge

f_offset	Separation between the passband edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the operating band

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

Prated,in	Rated pass band input power to the repeater for the test

Prated,p,EIRP 	Rated passband EIRP output power

Prated,p,TRP	Rated passband TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

Pin,p,EIRP	Input power intended to produce the maximum rated output power (Prated,p,TRP) at the RIB

Prated,out,FBWhigh	The rated output EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,out,FBWlow	The rated output EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Pmax,p,EIRP	Maximum passband EIRP output power when repeater is configured at the rated passband TRP output power (Prated,p,TRP)

Pmax,p,TRP	Maximum passband TRP output power measured per RIB

Wgap	Inter passband Bandwidth gap size

	The angle in the reference coordinate system, defined in sub-clause 4.14, between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the y/z plane. The angle is aligned with the down-tilt angle

H	The angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

MT	The down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).	The angle in the reference coordinate system, defined in sub-clause 4.12, between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°

φH	The horizontal angle in the global coordinate system, defined with respect to the horizon.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

AoA	Angle of Arrival

BW	Bandwidth

CACLR	Cumulative ACLR

CP-OFDM	Cyclic Prefix-OFDM

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

EIRP	Effective Isotropic Radiated Power

EEIRP	Expected EIRP

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

FSS	Fixed Satellite Service

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NCR-Fwd	NCR Forward

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RIB	Radiated Interface Boundary

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

TRP	Total Radiated Power

UL	Uplink

WA	Wide Area

# 4	General radiated test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in part 2 of this specification, i.e., to all radiated tests defined for FR2. The FR2 frequency range is defined in clause 5.1 of TS 38.106 [2].

The minimum requirements are given in TS 38.106 [2]. Test Tolerances for the radiated test requirements explicitly stated in the present document are given in annex C of the present document.

Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

Reuse of Test Tolerance values from TS 38.521-2 [20] for the purpose of MT-specific test requirements is subject to the following conditions:

-	NCR test methodology follows test procedures in TS 38.521-2 [20],

-	Maximum device size limit as captured in TS 38.521-2 [20].

Otherwise, reuse of Test Tolerance values from TS 38.521-2 [20] for the purpose of MT-specific test requirements may not be justified for some NCR products.

### 4.1.2	Acceptable uncertainty of Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the Test System is specified below for each test defined explicitly in the present specification, where appropriate. The maximum acceptable uncertainty of the Test System for test requirements included by reference is defined in the respective referred test specification.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

#### 4.1.2.2	Radiated characteristics measurements

The maximum OTA Test System uncertainty for radiated characteristics measurements are given in tables 4.1.2.2-1. Details for derivation of OTA Test System uncertainty are given in corresponding clauses in TR 37.941 [13].

Table 4.1.2.2-1: Maximum Test System uncertainty for radiated characteristics tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.3 OTA repeater output power (TRP) | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.4 OTA frequency stability | ±12 Hz Measurement results of 5000 Hz |  |
| 6.5 OTA out of band gain | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz |  |
| 6.6.2 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.3 OTA operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.4 OTA spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.7 OTA EVM | 1.25% signal analyser 2% stimulus signal |  |
| 6.8 OTA input intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz ±5.4 dB, 43.5 GHz < f ≤ 48.2 GHz |  |
| 6.9 OTA ACRR | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.2 OTA transient period | N/A |  |
| 6.11 OTA Output power dynamics | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |
| 6.12.1 OTA Frequency Error Requirements for NCR-MT | ± 0.01 ppm |  |
| 6.12.2 OTA Transmit Modulation Quality | 1% |  |
| 6.13 OTA Reference Sensitivity | ±2.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±2.4 dB, 37 GHz < f ≤ 43.5 GHz |  |
| 6.15 OTA Adjacent channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz |  |
| 6.16 OTA Blocking Characteristics | In-band blocking: ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz Out of band blocking: ±3.6 dB, 24.25 GHz < f ≤ 43 GHz |  |
| 6.17 OTA spurious emissions | ± 2.5 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.18 OTA spatial emission | ±1.7, 6.425 GHz < f ≤ 7.075 GHz |  |

### 4.1.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle.

The Shared Risk principle is defined in Recommendation ITU-R M.1545 [3].

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause 4.1.2 of the present document.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows.

Any additional uncertainty in the Test System over and above that specified in clause 4.1.2 shall be used to tighten the test requirement, making the test harder to pass. For some tests e.g., receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause 4.1.2 does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause 4.1.2 had been used.

## 4.2	Radiated requirement reference points

### 4.2.0	NCR type 1-H

For NCR type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

For NCR type 1-H, the NCR-Fwd conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector or UE-side TAB connector) for downlink or uplink for the configuration in normal operating conditions.

For NCR type 1-H, the NCR-MT conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector) for the configuration in normal operating conditions.

Figure 4.2.0-1: NCR type 1-H reference points

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate TAB connectors.

### 4.2.1	RF repeater type 2-O

For repeater type 2-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Figure 4.2-1: Radiated reference points for repeater type 2-O

### 4.2.2	NCR type 2-O

For NCR type 2-O, the radiated characteristics for NCR-Fwd and NCR-MT are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

For NCR type 2-O, the NCR-MT radiated RF requirements are applied at the BS side RIB for the configuration in normal operating conditions.

Figure 4.2.2-1: Radiated reference points for NCR type 2-O

NOTE:	the NCR-MT and NCR-Fwd may have the same or separate RIB.

## 4.3	RF repeater classes

### 4.3.1	RF repeater class for downlink

The requirements in this specification apply to downlink Wide Area repeaters, downlink Medium Range repeaters and downlink Local Area repeaters unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For repeater type 2-O, repeater downlink classes are defined as indicated below:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell scenarios with a repeater to UE minimum distance along the ground equal to 35 m.

-	Medium Range repeaters are characterised by requirements derived from Micro Cell scenarios with a repeater to UE minimum distance along the ground equal to 5 m.

-	Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m.

### 4.3.2	RF repeater class for uplink

The requirements in this specification apply to uplink Wide Area repeaters and uplink Local Area repeaters unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For repeater type 2-O, repeater uplink classes are defined as indicated below:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area repeaters are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

## 4.3A	NCR classes

### 4.3A.1	NCR class for downlink

The requirements in this specification apply to downlink Wide Area NCR, downlink Medium Range NCR and downlink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR with and without connectors.

For NCR type 2-O, NCR downlink classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell scenarios with a NCR to UE minimum distance along the ground equal to 35 m.

-	Medium Range NCR are characterised by requirements derived from Micro Cell scenarios with a NCR to UE minimum distance along the ground equal to 5 m.

-	Local Area NCR are characterised by requirements derived from Pico Cell scenarios with a NCR to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios.

### 4.3A.2	NCR class for uplink and MT

The requirements in this specification apply to uplink Wide Area NCR and uplink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR with and without connectors.

For NCR type 2-O, NCR uplink classes and MT classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area NCR are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 7.3.4 | OTA repeater output power: Additional requirements | These requirements may be applied regionally as additional repeater output power requirements. |
| 7.5.3.2 | OTA operating band unwanted emissions | Category A or Category B operating band unwanted emissions limits may be applied regionally. |
| 7.5.4.2 | OTA Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [4], may apply regionally. |
| 7.5.4.2.3 | OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the repeater operating band. |

## 4.5	Repeater configurations

### 4.5.1	Downlink configurations

Unless otherwise stated, the radiated downlink characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.5.1-1: Test interface for downlink

### 4.5.2	Uplink configurations

Unless otherwise stated, the radiated downlink characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.5.2-1: Test interface for uplink

### 4.5.3	Power supply options

If the repeater is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

## 4.6	Manufacturer's declarations

The following repeater manufacturer's declarations listed in table 4.6-1, when applicable to the repeater under test, are required to be provided by the manufacturer for radiated requirements testing for repeater type 2-O, NCR type 1-H and NCR type 2-O. Declarations can be made independently for UL and DL.

For the NCR type 1-H declarations required for the conducted requirements testing, refer to TS 38.115-1 [3], clause 4.6.

Table 4.6-1: Manufacturers declarations for repeater type 2-O radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the repeater enclosure. |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the repeater enclosure. |
| D.3 | Beam identifier | A unique title to identify a beam, e.g., a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the repeater is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 1) |
| D.4 | Operating bands and passband frequency ranges | List of NR operating band(s) supported by the repeater and passband frequency range(s) within the operating band(s) that the repeater can operate in. Supported bands declared for every beam (D.3). (Note 2) |
| D.5 | Repeater class | Declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. |
| D.6 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). |
| D.7 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). |
| D.8 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). |
| D.9 | Rated beam EIRP | The rated EIRP level per passband (Prated,p,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.8), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 5, Note 6, Note 7) |
| D.10 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). |
| D.11 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.10) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. |
| D.12 | Parallel beams | List of beams which have been declared equivalent (D.11) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. |
| D.13 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 3) |
| D.14 | OTA coverage range reference direction | The direction describing the reference direction of the OTA coverage range (D.13). (Note 4) |
| D.15 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. |
| D.16 | The rated passband OTA repeater power, Prated,p,TRP | Prated,p,TRP is declared as TRP OTA power per passband, declared per supported operating band. (Note 5, Note 7) |
| D.17 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 5, Note 7) |
| D.18 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [4]. |
| D.19 | Additional operating band unwanted emissions | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. |
| D.20 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. |
| D.21 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. |
| D.22 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,out,FBWlow) | The rated EIRP level per passband at lower frequency range of the fractional bandwidth (Prated,out,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges (D.21). (Note 5, Note 6, Note 7) |
| D.23 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh) | The rated EIRP level per passband at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges in (D.21). (Note 5, Note 6, Note 7) |
| D.24 | Long delay repeater | Declared only if the repeater internal delay between the input and output for this repeater does not fit within the TDD transient time. The repeater is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of repeaters internal delay is declared using this declaration. |
| D.25 | Input signal EIRP for maximum output power | Declaration of input signal EIRP required to reach maximum output power. Declared per passband. |
| D.26 | Repeater radiating direction | Declaration on whether the repeater is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the repeater radiates. |
| D.27 | Maximum repeater RF Bandwidth | Maximum repeater RF Bandwidth in the operating band for single-band operation. Declared per supported operating band. (Note 8) |
| NOTE 1: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 2: These operating bands are related to their respective single-band RIBs. NOTE 3: OTA coverage range is used for conformance testing of such TX OTA requirements as frequency error or EVM. NOTE 4: The OTA coverage range reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 5: If a Repeater type 2-O is capable of 64QAM operation but not capable of 256QAM operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured for 64QAM operation. NOTE 6: If D.22 and D.23 are declared for certain frequency range (D.21), there shall be no "Rated beam EIRP" declaration (D.9) for the operating band containing that particular frequency range. NOTE 7: If a repeater type 2-O is capable of 256QAM operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, a different declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured neither for 256QAM nor 64QAM operation. NOTE 8: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. |  |  |

The following NCR manufacturer's declarations listed in table 4.6-2, when applicable to the NCR under test, are required to be provided by the manufacturer for radiated requirements testing for NCR type 1-H and NCR type 2-O. Declarations can be made independently for UL and DL.

Table 4.6-2: Manufacturers declarations for NCR type 1-H and NCR type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | NCR type 1-H | NCR-Fwd type 2-O | NCR-MT type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the NCR enclosure. | X | X | X |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the NCR enclosure. | X | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the NCR is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 1) | x | x | x |
| D.4 | Operating bands and passband frequency ranges | List of NR operating band(s) supported by the NCR and passband frequency range(s) within the operating band(s) that the NCR can operate in. Supported bands declared for every beam (D.3). (Note 2) | c | x | x |
| D.5 | NCR class | Declared as Wide Area NCR, Medium Range NCR, or Local Area NCR. | c | x | x |
| D.6 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | c | x | x |
| D.7 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | c | x | x |
| D.8 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). For the EEIRP requirement in subclause 6.17, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.33). (Note 11) | x | x | x |
| D.9 | Rated beam EIRP | The rated EIRP level per passband (Prated,p,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.8), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 5, 6, 7) | x | x | x |
| D.10 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.11 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.10) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.12 | Parallel beams | List of beams which have been declared equivalent (D.11) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.13 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 3) | x | x | x |
| D.14 | OTA coverage range reference direction | The direction describing the reference direction of the OTA coverage range (D.13). (Note 4) | x | x | x |
| D.15 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.16 | The rated passband OTA NCR power, Prated,p,TRP | Prated,p,TRP is declared as TRP OTA power per passband, declared per supported operating band. (Note 5, 7) | N/A | x | x |
| D.17 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 5, 7) | N/A | x | x |
| D.18 | Spurious emission category | Declare the NCR spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [4]. | c | x | x |
| D.19 | Additional operating band unwanted emissions | The manufacturer shall declare whether the NCR under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. | c | x | x |
| D.20 | Co-existence with other systems | The manufacturer shall declare whether the NCR under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. | c | x | x |
| D.21 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | c | x | x |
| D.22 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,out,FBWlow) | The rated EIRP level per passband at lower frequency range of the fractional bandwidth (Prated,out,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges (D.21). (Note 5, 6, 7) | c | x | x |
| D.23 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh) | The rated EIRP level per passband at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges in (D.21). (Note 5, 6, 7)] | c | x | x |
| D.24 | Long delay NCR | Declared only if the NCR internal delay between the input and output for this NCR does not fit within the TDD transient time. The NCR is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of NCRs internal delay is declared using this declaration. | c | x |  |
| D.25 | Input signal EIRP for maximum output power | Declaration of input signal EIRP required to reach maximum output power. Declared per passband. | c | x |  |
| D.26 | NCR radiating direction | Declaration on whether the NCR is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the NCR radiates. | c | x |  |
| D.27 | Maximum NCR RF Bandwidth | Maximum NCR RF Bandwidth in the operating band for single-band operation. Declared per supported operating band. (Note 8) | c | x |  |
| D.28 | Support of simultaneous Tx of NCR-Fwd and NCR-MT | Declaration on whether the NCR support the simultaneous Tx of NCR-Fwd and NCR-MT | c | x | x |
| D.29 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | N/A | N/A | x |
| D.30 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.29). | N/A | N/A | x |
| D.31 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | N/A | N/A | x |
| D.32 | Single-band RIB or multi-band RIB | Declaration whether it is single band RIB or multi-band RIB. (Note 9) | c | x | x |
| D.33 | The mechanical down-tilt | For EEIRP the declaration of mechanical tilt together with D.8.(Note 10) | x | n/a | n/a |
| NOTE 1: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 2: These operating bands are related to their respective single-band RIBs. NOTE 3: OTA coverage range is used for conformance testing of such TX OTA requirements as frequency error or EVM. NOTE 4: The OTA coverage range reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 5: If a NCR type 1-H or NCR type 2-O is capable of 64QAM operation but not capable of 256QAM operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured for 64QAM operation. NOTE 6: If D.22 and D.23 are declared for certain frequency range (D.21), there shall be no "Rated beam EIRP" declaration (D.9) for the operating band containing that particular frequency range. NOTE 7: If a NCR type 1-H or NCR type 2-O is capable of 256QAM operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, a different declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured neither for 256QAM nor 64QAM operation. NOTE 8: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 9: In case of NCR type 1-H, this declaration applies per TAB connector. NOTE 10: The minimum mechanical down tilt and the maximum electrical steering range shall be declared. If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances: One instance is minimum mechanical down tilt and the reduced electrical steering range, and the other instance is maximum electrical steering range and increased mechanical down tilt. NOTE 11: EEIRP requirement applies only to NCR type 1-H. |  |  |  |  |  |

## 4.7	Test configurations

### 4.7.1	General

Test configurations in this specification refer to the configuration of test signals from test equipment that are provided to the repeater input.

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in clause 4.6.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.9.2.

NOTE:	If required, carriers are shifted to align with the channel raster.

### 4.7.2	Test signal configurations

#### 4.7.2.1	Test signal used to build Test Configurations

The signal's channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to tables 4.7.2.1-1.

Table 4.7.2.1-1: Signal to be used to build NR TCs

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: Repeater vendor can decide to test with 50 MHz channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: The narrowest specified BS channel bandwidth for the operating band (D.7) shall be used if 100MHz channel bandwidth is not specified for the band. |  |  |

#### 4.7.2.2	RTC1: Contiguous spectrum operation for RF repeater type 2-O

The purpose of test configuration RTC1 is to test all repeater requirements that need an input signal in the passband when there is only one passband per operating band.

##### 4.7.2.2.1	RTC1 generation

RTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum repeater RF Bandwidth supported shall be used (D.27);

-	Select the carrier to be tested according to 4.7.2 and place it adjacent to the lower passband edge. If the width of the passband is at least twice the bandwidth of the signal to be tested then place a second signal adjacent to the upper passband edge. Otherwise reposition the carrier to be tested according to the single carrier test frequencies described in section 4.9.1.

The test configuration should be constructed sequentially on a per band basis. All configured component carriers are transmitted simultaneously in the tests where the repeater should be ON.

##### 4.7.2.2.2	RTC1 power allocation

Set the TRP of each carrier to the same level so that the sum of the carrier powers equals the expected input power to the repeater for the test (i.e., either Prated,in or Prated,in + 10dB) according to the manufacturer's declaration in clause 4.6.

#### 4.7.2.4	RTC2: Non-contiguous spectrum operation for RF repeater type 2-O

The purpose of RTC2 is to test all repeater requirements that need an input signal in the passband when there is more than one passband per operating band.

##### 4.7.2.4.1	RTC2 generation

RTC2 is constructed on a per band basis using the following method:

-	The repeater passband bandwidths shall be the declared maximum repeater RF Bandwidth supported for multiple passbands (D.27). The repeater RF Bandwidth consists of one sub-block gap and the two highest and lowest declared passbands.

-	For each passband, select the carrier to be tested according to 4.7.2. If the width of the passband is at least twice that of the carrier to be tested then place a carrier adjacent to the upper passband edge and another carrier (as described in 4.7.2) adjacent to the lower passband edge. Otherwise, tests shall be applied with one carrier adjacent to the lower sub-block edge and one carrier adjacent to the upper sub-block edge for each sub-block gap.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset_high and Foffset_low for the carriers adjacent to the sub-block gap.

##### 4.7.2.4.2	RTC2 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the expected input power to the repeater for the test (i.e., either Pin,p,EIRP or Pin,p,EIRP + 10dB) according to the manufacturer's declaration in clause 4.6.

#### 4.7.2.5	NCRTC1: Contiguous spectrum operation for NCR type 2-O

The purpose of test configuration NCRTC1 is to test all NCR requirements that need an input signal in the passband when there is only one passband per operating band.

##### 4.7.2.5.1	NCRTC1 generation

NCRTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum NCR RF Bandwidth supported shall be used (D.27);

-	Place an NCR-MT carrier at the lower end of the passband. Generate an NR carrier using test equipment at the upper edge of the passband, and a second NR carrier adjacent to the NCR-MT carrier within the passband. If there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

-	Place an NCR-MT carrier at the upper end of the passband. Generate an NR carrier using test equipment at the lower edge of the passband, and a second NR carrier adjacent to the NCR-MT carrier within the passband. If there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the lower end of the passband.

The test configuration should be constructed sequentially on a per band basis. All configured component carriers are transmitted simultaneously in the tests where the NCR should be ON.

##### 4.7.2.5.2	NCRTC1 power allocation

Set the TRP of each carrier to the same level so that the sum of the carrier powers equals the expected input power to the NCR for the test (i.e., either Prated,in or Prated,in + 10dB) according to the manufacturer's declaration in clause 4.6. For the NCR-MT, set the output power according to the manufacturer's declaration in clause 4.6.

#### 4.7.2.6	NCRTC2: Non-contiguous spectrum operation for NCR type 2-O

The purpose of NCRTC2 is to test all NCR requirements that need an input signal in the passband when there is more than one passband per operating band.

##### 4.7.2.6.1	NCRTC2 generation

NCRTC2 is constructed on a per band basis using the following method:

-	The NCR passband bandwidths shall be the declared maximum NCR RF Bandwidth supported for multiple passbands (D.27). The repeater RF Bandwidth consists of one sub-block gap and the two highest and lowest declared passbands.

-	Place an NCR-MT carrier at the lower end of each passband. Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

-	Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the lower end of the passband.

-	Place an NCR-MT carrier at the lower end of the lower passband and place an NCR-MT carrier at the upper end of the upper passband. Generate an NR carrier using test equipment at the opposite edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the opposite end of the passband.

-	Place an NCR-MT carrier at the upper end of the lower passband and place an NCR-MT carrier at the lower end of the upper passband. Generate an NR carrier using test equipment at the opposite edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the opposite end of the passband.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset_high and Foffset_low for the carriers adjacent to the sub-block gap.

##### 4.7.2.6.2	NCRTC2 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the expected input power to the NCR for the test (i.e., either Pin,p,EIRP or Pin,p,EIRP + 10dB) according to the manufacturer's declaration in clause 4.6. For the NCR-MT, set the output power according to the manufacturer's declaration in clause 4.6.

## 4.8	Applicability of requirements

### 4.8.1	Applicability of test configurations

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for a RF repeater or NCR capable of single passband, and/or multi-bandoperation in both contiguous and non-contiguous spectrum in single band.

For a repeater or NCR declared to support a single passband within a single band, the test configurations in the second column of table 4.8.1-1 shall be used for testing.

For a repeater or NCR declared to support more than one passband within a single band (D.4) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for all passbands, the test configurations in the third column of table 4.8.1-1 shall be used for testing.

For a repeater or NCR declared to support more than one passband within a single band (D.4) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for all passbands, the test configurations in the fourth column of table 4.8.1-1 shall be used for testing.

Table 4.8.1-1: Test configurations for a repeater capable of single or multiple passbands in a single band

| Test case | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| --- | --- | --- | --- |
| Repeater output power | RTC1 | RTC1 | RTC1, RTC2 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD repeater) | RTC1 | RTC1 | RTC1, RTC2 |
| Error Vector Magnitude | RTC1 | RTC1 | RTC1, RTC2 |
| Adjacent Channel Leakage power Ratio (ACLR) | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | RTC2 | RTC2 |
| Operating band unwanted emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Transmitter spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Output intermodulation | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Input intermodulation | N/A | N/A | N/A |
| Adjacent Channel Rejection Ratio (ACRR) | RTC1 | RTC2 | RTC1, RTC2 |
| Receiver spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |

Table 4.8.1-2: Test configurations for a NCR capable of single or multiple passbands in a single band

| Test case | Single passband NCR | Multiple passband capable NCR with identical parameters per passband | Multiple passband capable NCR with different parameters per passband |
| --- | --- | --- | --- |
| NCR output power | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A | N/A |
| Transmit ON/OFF power | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Error Vector Magnitude | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Adjacent Channel Leakage power Ratio (ACLR) | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, RTC2 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | NCRTC2 | NCRTC2 |
| Operating band unwanted emissions | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Transmitter spurious emissions | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Input intermodulation | N/A | N/A | N/A |
| Adjacent Channel Rejection Ratio (ACRR) | NCRTC1 | NCRTC2 | NCRTC1, NCRTC2 |
| Output power dynamics | SC | SC | SC |
| OTA spatial emission | SC | SC | SC |
| Transmitter signal quality | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Reference sensitivity | SC | SC | SC |
| Adjacent channel selectivity | NCRTC1 | NCRTC2 | NCRTC1, INCRTC2 |
| Receiver Blocking characteristics | NCRTC1 | NCRTC2 | NCRTC1, NCRTC2 |
| Receiver spurious emissions | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |

## 4.9	RF channels and test models

### 4.9.1	RF channels

For the single passband testing many tests in the present document are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the repeater. These are denoted as RF channels B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single passband at each of the RF channels B, M and T.

Many tests in the present document are performed with the maximum repeater RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum repeater RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum repeater RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW: maximum repeater RF Bandwidth located at the top of the supported frequency range in the operating band.

When a test is performed by a test laboratory, the position of B, M and T for single passband, BRFBW, MRFBW and TRFBW for single band operation shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.9.2	Test models

#### 4.9.2.1	General

The following clauses will describe the test models needed for repeater type 2-O.

#### 4.9.2.2	FR2 test models for repeater type 2-O for DL

FR2 test model in clause 4.9.2.2 in TS 38.141-2 [6] applies to repeater type 2-O as below:

-	NR-FR2-TM1.1 applies to RDL-FR2-TM1.1

-	NR-FR2-TM2 applies to RDL-FR2-TM2

-	NR-FR2-TM2a applies to RDL-FR2-TM2a

-	NR-FR2-TM3.1 applies to RDL-FR2-TM3.1

-	NR-FR2-TM3.1a applies to RDL-FR2-TM3.1a

#### 4.9.2.3	FR2 test models for repeater type 2-O for UL

The set-up of physical channels for transmitter tests shall be according to one of the test models (R-FR2-TMs) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.106 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR2 TDD, repeater test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.3-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [22].

Table 4.9.2.3-1: Configurations of TDD for repeater type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all FR2 test models are specified in table 4.9.2.3-2 and table 4.9.2.3-3 for PUSCH. Specific physical channel parameters for FR2 test models are described in clauses 4.9.2.3.1 to 4.9.2.3.3.

Table 4.9.2.3-2: Common physical channel parameters for repeater type 2-O PUSCH

| Parameter | Value |
| --- | --- |
| mapping type | PUSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | 'pos2' |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 'pos0' |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PUSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS = 4 KPT-RS = 2 krefRE=00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

Table 4.9.2.3-3: Common physical channel parameters for PUSCH by RNTI for repeater type 2-O test models

| Parameter | Value |
| --- | --- |
| PUSCH |  |

##### 4.9.2.3.1	FR2 test model 1.1 (RUL-FR2-TM1.1)

This model shall be used for tests on:

-	OTA output power (EIRP)

-	OTA Repeater output power (TRP)

-	OTA out of band gain

-	OTA Unwanted emissions

-	OTA ACLR

-	OTA operating band unwanted emissions

-	OTA transmitter spurious emissions

-	OTA input intermodulation

-	OTA ACRR

-	OTA transmit ON/OFF power

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM1.1 are defined in table 4.9.2.3.1-1.

Table 4.9.2.3.1-1: Specific physical channel parameters of RUL-FR2-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PUSCH | NRB |
| Modulation PUSCH | QPSK |

##### 4.9.2.3.2	FR2 test model 2 (RUL-FR2-TM2)

This model shall be used for tests on:

-	Uplink OTA repeater EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM2 are defined in table 4.9.2.3.2-1 for 64QAM. For 16QAM and QPSK, specific physical channel parameters for RUL-FR2-TM2 are defined in table 4.9.2.3.2-1 with 64QAM PUSCH PRB replaced with selected modulation order PUSCH PRB according to the corresponding test procedure.

Table 4.9.2.3.2-1: Specific physical channel parameters of RUL-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PUSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PUSCH PRBs which are not allocated | NRB-1 |

##### 4.9.2.3.2a	FR2 test model 2a (RUL-FR2-TM2a)

This model shall be used for tests on:

-	Uplink OTA repeater EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM2a are defined in table 4.9.2.3.2-1 with all 64QAM PUSCH PRBs replaced by 256QAM PUSCH PRBs.

##### 4.9.2.3.3	FR2 test model 3.1 (RUL-FR2-TM3.1)

This model shall be used for tests on:

-	Transmitted signal quality

-	OTA frequency stability (at max power)

-	Uplink OTA repeater EVM for modulation (at max power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM3.1 are defined in table 4.9.2.3.1-1 with all QPSK PUSCH PRBs replaced with selected modulation order PUSCH PRBs according to the corresponding test procedure.

##### 4.9.2.3.4	FR2 test model 3.1a (RUL-FR2-TM3.1a)

This model shall be used for tests on:

-	Transmitted signal quality

-	OTA frequency stability (at max power)

-	Uplink OTA repeater EVM for 256QAM modulation (at max power)

Common physical channel parameters are defined in clause 4.9.2.3. Specific physical channel parameters for RUL-FR2-TM3.1a shall be defined in table 4.9.2.3.1-1 with all QPSK PUSCH PRBs replaced by 256QAM.

#### 4.9.2.4	Data content of physical channels and signals for RUL-FR2-TM

Randomisation of the data content is obtained by utilizing a PN sequence generator and the length-31 Gold sequence scrambling of TS 38.211 [18], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [18] use the following additional parameters:

-

-	q = 0 (single code word)

-	Rank 1, single layer

##### 4.9.2.4.1	PUSCH

-	Generate the required amount of bits from the output of the PN23 sequence generator [28]. The PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries.

-	Perform user specific scrambling according to TS 38.211 [18], clause 6.3.1.1.

-

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [18], clause 6.3.1.3.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [18], clause 6.3.1.3.

-	Perform PDSCH mapping type A according to TS 38.211 [18].

-	DM-RS sequence generation according to TS 38.211 [18], clause 6.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.3-3.

-

-

-	DM-RS mapping according to TS 38.211 [18], clause 6.4.1.1.3 with parameters listed in table 4.9.2.3-3.

-	For RUL-FR2-TM PT-RS sequence generation according to TS 38.211 [18], clause 6.4.1.2.1, with parameters listed in table 4.9.2.3-3.

-	For RUL-FR2-TM PT-RS mapping according to TS 38.211 [18], clause 6.4.1.2.2, with parameters listed in table 4.9.2.3-3.

### 4.9.3	Test beam direction set

#### 4.9.3.1    General

Conformance testing of OTA spatial emission requirement is based on a test beam direction set.

#### 4.9.3.2    Test beam direction set for protection of FSS UL

The following beam direction set is utilized for conformance testing of requirements in clause 6.17.5.1 for NCR type 1-H operating in band n104. The test beam directions are defined in table 4.9.3.2-1.

Table 4.9.3.2-1: Test beam direction set for protection of FSS UL

| Test Beam index | Elevation angle | Azimuth angle | Weighting factor |
| --- | --- | --- | --- |
| 1 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 2 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 3 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 4 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 5 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 6 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 7 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 8 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 9 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 10 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 11 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 12 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 13 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 14 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 15 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 16 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 17 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 18 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 19 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 20 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 21 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.8. |  |  |  |

The test beam directions relative to Θmin, Θmax, Φmax, Φmin are visualised in Figure 4.9.3.2-1.

Figure 4.9.3.2-1 Test beam set for conformance testing

## 4.9A	RF channels and test models of NCR

### 4.9A.1	RF channels

For the single passband or carrier testing many tests in this specification are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the NCR. These are denoted as RF channels B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single passband or carrier at each of the RF channels B, M and T.

Many tests in the present document are performed with the maximum NCR RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum NCR RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum NCR RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW: maximum NCR RF Bandwidth located at the top of the supported frequency range in the operating band.

When a test is performed by a test laboratory, the position of B, M and T for single passband or carrier, BRFBW, MRFBW and TRFBW for single band operation, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.9A.2	Test models

#### 4.9A.2.1	General

The following clauses will describe the test models needed for NCR type 2-O.

#### 4.9A.2.2	FR2 test models for NCR type 2-O for DL

FR2 test model in clause 4.9.2.2 in TS 38.141-2 [6] applies to NCR type 2-O as below:

-	NR-FR2-TM1.1 applies to NCRDL-FR2-TM1.1

-	NR-FR2-TM2 applies to NCRDL-FR2-TM2

-	NR-FR2-TM2a applies to NCRDL-FR2-TM2a

-	NR-FR2-TM3.1 applies to NCRDL-FR2-TM3.1

-	NR-FR2-TM3.1a applies to NCRDL-FR2-TM3.1a

#### 4.9A.2.3	FR2 test models for NCR type 2-O for UL

The set-up of physical channels for transmitter tests shall be according to one of the test models (NCRUL-FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.106 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR2 TDD, NCR test models are derived based on the uplink/downlink configuration as shown in the table 4.9A.2.3-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [22].

Table 4.9A.2.3-1: Configurations of TDD for NCR type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all FR2 test models are specified in table 4.9A.2.3-2 and table 4.9A.2.3-3 for PUSCH. Specific physical channel parameters for FR2 test models are described in clauses 4.9A.2.3.1 to 4.9A.2.3.3.

Table 4.9A.2.3-2: Common physical channel parameters for NCR type 2-O PUSCH

| Parameter | Value |
| --- | --- |
| mapping type | PUSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | 'pos2' |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 'pos0' |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PUSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS = 4 KPT-RS = 2 krefRE=00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

Table 4.9A.2.3-3: Common physical channel parameters for PUSCH by RNTI for NCR type 2-O test models

| Parameter | Value |
| --- | --- |
| PUSCH nRNTI=0 |  |

##### 4.9A.2.3.1	FR2 test model 1.1 (NCRUL-FR2-TM1.1)

This model shall be used for tests on:

OTA output power (EIRP)-	OTA Repeater output power (TRP)

-	OTA out of band gain

-	OTA Transmit ON/OFF power

-	OTA Unwanted emissions

-	OTA ACLR

-	OTA operating band unwanted emissions

-	OTA transmitter spurious emissions

-	OTA Receiver spurious emissions

-	OTA input intermodulation

-	OTA ACRR

-	OTA transmit ON/OFF power

Common physical channel parameters are defined in clause 4.9A.2.3. Specific physical channel parameters for NCRUL-FR2-TM1.1 are defined in table 4.9A.2.3.1-1.

Table 4.9A.2.3.1-1: Specific physical channel parameters of NCRUL-FR2-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PUSCH nRNTI=0 | NRB |
| Modulation PUSCH nRNTI=0 | QPSK |

##### 4.9A.2.3.2	FR2 test model 2 (NCRUL-FR2-TM2)

This model shall be used for tests on:

-	Uplink OTA NCR EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9A.2.3. Specific physical channel parameters for NCRUL-FR2-TM2 are defined in table 4.9A.2.3.2-1 for 64QAM. For 16QAM and QPSK, specific physical channel parameters for NCRUL-FR2-TM2 are defined in table 4.9A.2.3.2-1 with 64QAM PUSCH PRB replaced with selected modulation order PUSCH PRB according to the corresponding test procedure.

Table 4.9A.2.3.2-1: Specific physical channel parameters of NCRUL-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PUSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PUSCH PRBs which are not allocated | NRB-1 |

##### 4.9A.2.3.2a	FR2 test model 2a (NCRUL-FR2-TM2a)

This model shall be used for tests on:

-	Uplink OTA NCR EVM of single PRB allocation (at lower PSD TX power limit at min power)

-	OTA frequency stability (at lower PSD TX power limit at min power)

Common physical channel parameters are defined in clause 4.9A.2.3. Specific physical channel parameters for NCRUL-FR2-TM2a are defined in table 4.9A.2.3.2-1 with all 64QAM PUSCH PRBs replaced by 256QAM PUSCH PRBs.

##### 4.9A.2.3.3	FR2 test model 3.1 (NCRUL-FR2-TM3.1)

This model shall be used for tests on:

-	OTA Output power dynamics

-	OTA Total power dynamic range (upper TX PSD power limit at max power with all 64QAM PRBs allocated)

-	OTA Transmitted signal quality

-	OTA Frequency error (at max power)

-	OTA EVM for modulation (at max power)

Common physical channel parameters are defined in clause 4.9A.2.3. Specific physical channel parameters for NCRUL-FR2-TM3.1 are defined in table 4.9A.2.3.1-1 with all QPSK PUSCH PRBs replaced with selected modulation order PUSCH PRBs according to the corresponding test procedure.

##### 4.9A.2.3.4	FR2 test model 3.1a (NCRUL-FR2-TM3.1a)

This model shall be used for tests on:

-	OTA Output power dynamics

-	OTA Total power dynamic range (upper TX PSD power limit at max power with all 256QAM PRBs allocated)

-	OTA Transmitted signal quality

-	OTA Frequency stability (at max power)

-	OTA Frequency error (at max power)

-	EVM for 256QAM modulation (at max power)

Common physical channel parameters are defined in clause 4.9A.2.3. Specific physical channel parameters for NCRUL-FR2-TM3.1a shall be defined in table 4.9A.2.3.1-1 with all QPSK PUSCH PRBs replaced by 256QAM.

#### 4.9A.2.4	Data content of physical channels and signals for NCRUL-FR2-TM

##### 4.9A.2.4.1	General

Randomisation of the data content is obtained by utilizing a PN sequence generator and the length-31 Gold sequence scrambling of TS 38.211 [18], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [18] use the following additional parameters:

-	NIDcell=1

-	q = 0 (single code word)

-	Rank 1, single layer

##### 4.9A.2.4.2	PUSCH

-	Generate the required amount of bits from the output of the PN23 sequence generator. The PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries.

-	Perform user specific scrambling according to TS 38.211 [18], clause 6.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [18], clause 6.3.1.3.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [18], clause 6.3.1.3.

-	Perform PDSCH mapping type A according to TS 38.211 [18].

-	DM-RS sequence generation according to TS 38.211 [18], clause 6.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9A.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [18], clause 6.4.1.1.3 with parameters listed in table 4.9.2.2-3.

-	For NCRUL-FR2-TM PT-RS sequence generation according to TS 38.211 [18], clause 6.4.1.2.1, with parameters listed in table 4.9A.2.2-3.

-	For NCRUL-FR2-TM PT-RS mapping according to TS 38.211 [18], clause 6.4.1.2.2, with parameters listed in table 4.9A.2.2-3.

## 4.10	Requirements for contiguous and non-contiguous spectrum

A spectrum allocation where a repeater or NCR operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for repeater or NCR configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For repeater or NCR operation in non-contiguous spectrum, some requirements apply both at the repeater or NCR passband edges and inside the sub-block gaps. For each such requirement, it is stated how the limits apply relative to the repeater or NCR passband edges and the sub-block edges respectively.

## 4.11	Format and interpretation of tests

Each test has a standard format:

X	Title

All tests are applicable to all equipment within the scope of the present document, unless otherwise stated.

X.1	Definition and applicability

This clause gives the general definition of the parameter under consideration and specifies whether the test is applicable to all equipment or only to a certain subset. Required manufacturer declarations may be included here.

X.2	Minimum requirement

This clause contains the reference to the clause to the 3GPP reference (or core) specification which defines the minimum requirement.

X.3	Test purpose

This clause defines the purpose of the test.

X.4	Method of test

X.4.1	General

In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here.

X.4.2y	First test method

X.4.2y.1	Initial conditions

This clause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up. The OTA Test System is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned.

X.4.2y.2	Procedure

This clause describes the steps necessary to perform the test and provides further details of the test definition like domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging). The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions).

X.4.3y	Alternative test method (if any)

If there are alternative test methods, each is described with its initial conditions and procedures.

X.5	Test requirement

This clause defines the pass/fail criteria for the equipment under test, see clause 4.1.3 (Interpretation of measurement results). Test requirements for every minimum requirement referred in clause X.2 are listed here. Cases where minimum requirements do not apply need not be mentioned.

## 4.12	Reference coordinate system

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP) at certain angles with respect to the repeater or NCR. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system should be associated to an identifiable physical feature on the repeater or NCR enclosure. The location of the origin and the orientation of the reference coordinate system are for the repeater or NCR manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.12-1.

Figure 4.12-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

# 5	Operating bands and channel arrangement

For the NR repeater operation in NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.106 [2], clause 5 and its relevant clauses.

For the radiated testing purposes in this specification, only FR2-1 operating bands are considered.

# 6	Radiated characteristics

## 6.1	General

Unless otherwise stated, the radiated characteristics are specified at RIB for repeater type 2-O configuration in normal operating conditions.

Requirements apply in both DL and UL unless otherwise stated, or declared.

-	For the DL the BS-side RIB is the input and the UE-side RIB is the output.

-	For the UL the UE-side RIB is the input and the BS-side RIB is the output.

General test conditions for radiated tests of the repeater type 2-O are given in clause 4, including interpretation of measurement results and configurations for testing. Repeater configurations for the tests are defined in clause 4.5.

If a number of single-band RIB have been declared equivalent (D.32), only a representative one is necessary to be tested to demonstrate conformance.

## 6.2	OTA output power

### 6.2.1	General

OTA output power includes both OTA output EIRP power and OTA output TRP.

### 6.2.2	OTA output power (EIRP)

#### 6.2.2.1	Definition and applicability

Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each declared beam, the requirement is based on declarations captured in clause 4.6 for a beam identifier (D.3), reference beam direction pair (D.6), rated beam EIRP (D.9) at the beam's reference direction pair, OTA peak directions set (D.7), the beam direction pairs at the maximum steering directions (D.8) and their associated rated beam EIRP and beamwidth(s) for reference beam direction pair and maximum steering directions (D.10).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in clause 6.2.4.

NOTE 1:	OTA peak directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For passbands where the supported fractional bandwidth (FBW) is larger than 6%, two rated beam EIRP may be declared by manufacturer:

-	Prated,out,FBWlow for lower supported frequency range, and

-	Prated,out,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated beam EIRP is:

-	Prated,out,FBWlow, for the output whose frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,out,FBWhigh, for the output whose frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

The repeater radiated transmit power requirements are specified at single-band RIB.

If beams have been declared equivalent and parallel (D.11, D.12), only a representative beam is necessary to be tested to demonstrate conformance.

The repeater rated beam EIRP output power for repeater type 2-O UL transmission shall be within limits as specified in table 6.2.2.1-1.

Table 6.2.2.1-1: Repeater rated beam EIRP output power limits for repeater type 2-O UL transmission

| Repeater class | Prated,p,EIRP |
| --- | --- |
| Wide Area | (note 1) |
| Local Area | ≤ + 55 + X dBm, Note 2 |
| NOTE 1: There is no upper limit for the Prated,p,EIRP of the repeater type 2-O UL transmission. NOTE 2: X = 10*log (ceil (passband bandwidth/100MHz)) |  |

There is no upper limit for the rated beam EIRP output power of NCR-Fwd type 2-O DL transmission.

The rated beam EIRP output power for NCR-Fwd type 2-O UL transmission shall be within limits as specified in table 6.2.2.1-2.

Table 6.2.2.1-2: Rated TRP output power limits and rated beam EIRP output power limits for NCR- type 2-O UL transmission

| NCR-Fwd class | Prated,p,EIRP |
| --- | --- |
| Wide Area | (note 1) |
| Local Area | ≤ + 55 + X dBm (Note 2) |
| NOTE 1: There is no upper limit for Prated,p,EIRP of the NCR type 2-O UL transmission. NOTE 2: X = 10*log (ceil (passband bandwidth/100MHz)) |  |

#### 6.2.2.2	Minimum requirement

The minimum requirement applies per single-band RIB supporting transmission in the operating band.

The minimum requirement for repeater type 2-O is defined for normal and extreme conditions in TS 38.106 [2], clause 7.2.2.

The minimum requirement for NCR type 1-H and NCR type 2-O is defined for normal and extreme conditions in TS 38.106 [2], clause 7.2.3.

#### 6.2.2.3	Test purpose

The test purpose is to verify the ability to accurately generate and direct radiated power per beam, across the frequency range, for all declared beams.

#### 6.2.2.4	Method of test

##### 6.2.2.4.1	Initial conditions

Test environment:

-	Normal, see annex A.2,

-	Extreme, see annexes A.3 and A.5.

A measurement system set-up is shown in annex E.

RF channels to be tested for single carrier:	B, M and T; see clause 4.9.1.

RF channels positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW for single-band RIB, see clause 4.9.1.

Under extreme test environment, it is sufficient to test on one NR-ARFCN or one RF bandwidth position, and with one applicable test configuration defined in clauses 4.7 and 4.8. Testing shall be performed under extreme power supply conditions, as defined in annex B.5.

NOTE:	Tests under extreme power supply conditions also test extreme temperatures.

Directions to be tested:

-	OTA peak directions set reference beam direction pair (D.6), and

-	OTA peak directions set maximum steering directions (D.8).

Beams to be tested: Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.9).

Power levels to be tested:

-	The lowest input power (Pp,in,EIRP) that produces the rated passband TRP output power (Prated,p,TRP).

-	The lowest input power (Pp,in,EIRP) that produces the rated passband TRP output power (Prated,p,TRP), plus 10 dB.

##### 6.2.2.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the DUT at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the DUT with the test system.

3)	For repeater type 2-O, orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved.

For NCR-Fwd, orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved; for NCR-MT, configure the NCR-MT such that the beam peak direction(s) applied during the power measurement step 7.

4)	Configure the beam peak direction of the DUT according to the declared beam direction pair if necessary.

5)	Set the test signal generator power at the RIB as shown in annex E with a power equivalent to the power level to be tested, according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2, in the correct direction in respect to the repeater.

6)	Measure EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Test steps 3 to 6 are repeated for all declared beams (D.7) and their reference beam direction pairs and maximum steering directions (D.6 and D.8), and for all applicable power levels.

For extreme conditions tests the methods in TS 38.141-2 [6], annex B.7 may be used.

#### 6.2.2.5	Test requirement for RF repeater type 2-O and NCR-Fwd type 2-O

For each single-band RIB under test, the power measured in clause 6.2.2.4.2 in step 6 shall remain within the values provided in table 6.2.2.5-1 for normal and extreme test environments, relative to the manufacturer's declared Prated,p,EIRP (D.9) for repeater type 2-O:

Table 6.2.2.5-1: Test requirements for radiated transmit power accuracy

| Normal test environment | Extreme test environment |
| --- | --- |
| 24.25 GHz < f ≤ 29.5 GHz: ± 5.1 dB | 24.25 GHz < f ≤ 29.5 GHz: ± 7.6 dB |
| 37 GHz < f ≤ 43.5 GHz: ± 5.4 dB | 37 GHz < f ≤ 43.5 GHz: ± 7.8 dB |
| 43.5 GHz < f ≤ 48.2 GHz: ± 5.6 dB | 43.5 GHz < f ≤ 48.2 GHz: ± 8.0 dB |

#### 6.2.2.6	Test requirement for NCR

##### 6.2.2.6.1	Test requirement for NCR-MT

##### 6.2.2.6.1.1	Test requirement for NCR-MT type 1-H

For NCR-MT type 1-H, the IAB requirement specified in clause 9.2.2 in TS 38.174 [19] apply.

For each declared conformance beam direction pair, the EIRP measurement results in clause 6.2.4.2 shall remain within the values provided in table 6.2.2.6.1.1-1, relative to the manufacturer's declared rated beam EIRP (D.9) value:

Table 6.2.2.6.1.1-1: Test requirement for radiated transmit power for NCR-MT

|  | Normal test environment |  |
| --- | --- | --- |
| NCR-MT type 1-H | f ≤ 3 GHz: ± 3.3 dB |  |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB |  |

6.2.2.6.1.2	Test requirement for NCR-MT type 2-O

For each declared conformance beam direction pair, the EIRP measurement results in clause 6.2.2.4.3 shall remain within the values provided in table 6.2.2.6.1.2-1, relative to the manufacturer's declared rated beam EIRP (D.9) value:

Table 6.2.2.6.1.2-1: Test requirement for radiated transmit power for NCR-MT

| NCR type | Normal test environment | Extreme test environment |
| --- | --- | --- |
| NCR-MT type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 6 dB 37 GHz < f ≤ 43.5 GHz: ± 6 dB 43.5 GHz < f ≤ 48.2 GHz: ± 6 dB | 24.15 GHz < f ≤ 29.5 GHz: ± [7.1] dB 37 GHz < f ≤ 43.5 GHz: ± [7.1] dB 43.5 GHz < f ≤ 48.2 GHz: ± [7.1] dB |

### 6.2.3	OTA repeater output power (TRP)

#### 6.2.3.1	Definition and applicability

OTA repeater output power is declared as rated carrier TRP, with the output power accuracy requirement defined at the RIB.

The repeater rated TRP output power for repeater type 2-O UL transmission shall be within limits as specified in table 6.2.3.1-1.

Table 6.2.3.1-1: Repeater rated TRP output power limits for repeater type 2-O UL transmission

| Repeater class | Prated,p,TRP |
| --- | --- |
| Wide Area | (note 1) |
| Local Area | ≤ + 35 + X dBm (note 2) |
| NOTE 1: There is no upper limit for the Prated,p,TRP of the repeater type 2-O UL transmission. NOTE 2: X = 10*log (ceil (passband bandwidth/100MHz)) |  |

The output power limit for the respective repeater classes in table 6.2.3.1-1 shall be compared to the rated output power and the declared repeater class. It is not subject to testing.

#### 6.2.3.2	Minimum requirement

The minimum requirement applies per single-band RIB supporting transmission in the operating band.

The minimum requirement for repeater type 2-O is defined for normal conditions in TS 38.106 [2], clause 7.2.2.

The minimum requirement for NCR type 2-O is defined for normal conditions in TS 38.106 [2], clause 7.2.3.

#### 6.2.3.3	Test purpose

The test purpose is to verify the accuracy of the maximum passband TRP output power (Pmax,p,TRP) across the frequency range for all RIBs.

#### 6.2.3.4	Method of test

##### 6.2.3.4.1	Initial conditions

Test environment: Normal, see annex A.2.

A measurement system set-up is shown in annex E.

RF channels to be tested for single carrier: B, M, T; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single band operation; see clause 4.9.1.

Beams to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex G) as long as the required TRP level is achieved.

Power levels to be tested:

-	The lowest input power (Pp,in,EIRP) that produces the rated passband TRP output power (Prated,p,TRP).

-	The lowest input power (Pp,in,EIRP) that produces the rated passband TRP output power (Prated,p,TRP), plus 10 dB.

##### 6.2.3.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex G. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 5, and 7.

1)	Place the DUT at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the DUT with the test system.

3)	For repeater type 2-O, orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved.

For NCR-Fwd, orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved; for NCR-MT, configure the NCR-MT such that the beam peak direction(s) applied during the power measurement step 7 are consistent with the grid and measurement approach for the TRP test.

4)	Configure the DUT such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

5)	Set the test signal generator power at the RIB as shown in annex G with a power equivalent to the tested input power level, transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2, in the correct direction in respect to the repeater.

6)	For repeater type 2-O, orient the positioner (and DUT) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G) whilst maintaining the correct direction of arrival for the test signal.

For NCR-Fwd, orient the positioner (and DUT) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G) whilst maintaining the correct direction of arrival for the test signal; for NCR-MT, orient the positioner (NCR) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

7)	Measure the radiated power for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex G).

9)	Calculate TRP using the EIRP measurements.

#### 6.2.3.5	Test requirement

The final TRP measurement result in clause 6.2.3.4.2 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 37 GHz < f ≤ 43.5 GHz.

-	within +5.6 dB and –5.6 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 43.5 GHz < f ≤ 48.2 GHz.

### 6.2.3.6	Test requirement for NCR

##### 6.2.3.6.1	Test requirement for NCR-Fwd type 2-O

The final TRP measurement result in clause 6.2.3.4.3 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 37 GHz < f ≤ 43.5 GHz.

-	within +5.6 dB and –5.6 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 43.5 GHz < f ≤ 48.2 GHz.

##### 6.2.3.6.2	Test requirement for NCR-MT

6.2.3.6.2.1	Test requirement for NCR-MT type 2-O

The final TRP measurement result in clause 6.3.4.2 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 37 GHz < f ≤ 43.5 GHz.

-	within +5.6 dB and –5.6 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 43.5 GHz < f ≤ 48.2 GHz.

##### 6.2.3.6.3	Test requirement for NCR-Fwd

6.2.3.6.3.1	Test requirement for NCR-Fwd type 2-O

The final TRP measurement result in clause 6.2.3.4.3 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 37 GHz < f ≤ 43.5 GHz.

-	within +5.6 dB and –5.6 dB of the manufacturer's declared rated TRP output power Prated,p,TRP for carrier frequency 43.5 GHz < f ≤ 48.2 GHz.

## 6.3	OTA frequency stability

### 6.3.1	Definition and applicability

Frequency stability is the ability to maintain the same frequency on the output signal with respect to the input signal.

### 6.3.2	Minimum Requirement

The minimum requirement for repeater type 2-O is in TS 38.106 [2], clause 7.3.2.

The minimum requirement for NCR type 2-O is in TS 38.106 [2], clause 7.3.3.

### 6.3.3	Test purpose

The test purpose is to verify that frequency stability is within the limit specified by the minimum requirement.

### 6.3.4	Method of test

Requirement is tested together with modulation quality test, as described in clause 6.6.

### 6.3.5	Test Requirements for RF repeater and NCR

The frequency deviation of the output signal with respect to the input signal shall be accurate to within ±(0.01 ppm + 12 Hz) observed over 1 ms.

## 6.4	OTA out of band gain

### 6.4.1	Definition and applicability

Out of band gain refers to the gain of the repeater outside the passband.

### 6.4.2	Minimum Requirement

The minimum requirement for RF repeater type 2-O is in TS 38.106 [2], clause 7.4.2.

The minimum requirement for NCR type 2-O is in TS 38.106 [2], clause 7.4.3.

### 6.4.3	Test purpose

The test purpose is to verify that out of band gain is within the limit specified by the minimum requirement.

### 6.4.4	Method of test

#### 6.4.4.1	Initial conditions

Test environment:

-	Normal, see annex B.2,

#### 6.4.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the RF repeater or NCR at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the RF repeater or NCR with the test system.

3)	Orient the positioner (and RF repeater or NCR and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved..

4)	Set the CW generator power at the RIB as shown in annex E with a power equivalent to Pin,p,EIRP, in the correct direction in respect to the RF repeater or NCR.

5)	Orient the positioner (and RF repeater or NCR and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the radiated power for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex G), whilst maintaining the input signal in the correct direction with respect to the RF repeater or NCR.

8)	Calculate TRP using the EIRP measurements. The out of band gain at the frequency under test is given by the difference in dB between the measured output TRP and the EIRP of the input signal.

9)	Repeat steps 2-8, shifting the offset frequency of the CW from the edge of the passband from 200kHz to 10MHz in steps of 200kHz for each passband.

### 6.4.5	Test Requirements for RF repeater and NCR

The gain outside the passband shall not exceed the maximum level specified in table 6.4.5-1, where:

-	f_offset_CW is the offset between the outer channel edge frequency of the outer channel in the passband and a CW signal.

Table 6.4.5-1: Out of band gain limits

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 70.1 dB (NOTE 1) 70.4 dB (NOTE 2) 70.6 dB (NOTE 3) |
| 150 MHz  f_offset_CW < 400 MHz | 57.1 dB (NOTE 1) 57.4 dB (NOTE 2) 57.6 dB (NOTE 3) |
| 400 MHz  f_offset_CW < f_offset_max | 37.1 dB (NOTE 1) 37.4 dB (NOTE 2) 37.6 dB (NOTE 3) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25–29.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 37-43.5GHz NOTE 3: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz |  |

## 6.5	OTA unwanted emissions

### 6.5.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [4]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the passband resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the repeater type 2-O and NCR-Fwd type 2-O transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). OTA Unwanted emissions outside of this frequency range are limited by an OTA spurious emissions requirement.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 6.5.1-1 for repeater type 2-O for NR operating bands.

Table 6.5.1-1: Maximum offset ΔfOBUE outside the downlink/uplink operating band for repeater type 2-O

| Repeater type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| Repeater type 2-O NCR-Fwd type 2-O | FDL,high – FDL,low ≤ 4000 MHz FUL,high – FUL,low ≤ 4000 MHz | 1500 |

The unwanted emission requirements are applied per cell for all the configurations. Requirements for OTA unwanted emissions are captured using TRP, directional requirements as described per requirement.

If the NCR supports simultaneous NCR-MT and NCR-Fwd transmission, then the unwanted emissions requirements should be defined on the total emissions from NCR-MT and NCR-Fwd.

### 6.5.2	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 6.5.2.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON state.

#### 6.5.2.2	Minimum requirement

The minimum requirement for repeater type 2-O is defined in TS 38.106 [2], clause 7.5.2.2.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.5.2.3.1.1.

For WA NCR-MT type 2-O, the BS ACLR requirements specified in clause 9.7.3.3 in TS 38.104 [5] apply.

For LA NCR-MT type 2-O, the UE ACLR requirements specified in clause 6.5.2.3 in TS 38.101-2 [14] apply.

For simultaneous transmission the limits apply for sum of NCR-MT transmission and NCR-Fwd transmission

#### 6.5.2.3	Test purpose

To verify that the adjacent channel leakage power ratio requirement shall be met as specified by the minimum requirement.

#### 6.5.2.4	Method of test

##### 6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

##### 6.5.2.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex G.

1)	Place the repeater at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	The measurement devices characteristics shall be:

-	measurement filter bandwidth: defined in clause 6.5.2.5.

-	detection mode: true RMS voltage or true power averaging.

4a)	For RF repeater, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models RDL-FR2-TM1.1 and RUL-FR2-TM1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

For NCR-Fwd, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRDL-FR2-TM1.1 and NCRUL-FR2-TM1.1 in clause 4.9A.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

For NCR-MT, set the NCR-MT with the declared maximum output power according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRUL-FR2-TM1.1 in clause 4.9.2.

4b)	Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off. Verify measured result is enough below requirement limit.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the absolute power of the assigned channel frequency and the (adjacent channel frequency).

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex G).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel using the measurements made in Step 7.

9)	Calculate relative ACLR estimate.

NOTE 1:	ACLR is calculated by the ratio of the absolute TRP of the assigned channel frequency and the absolute TRP of the adjacent frequency channel.

10)	Measure OTA ACLR for the frequency offsets both side of the passband edge and carrier edge as specified in table 6.5.2.5-1. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

11)	For the OTA ACLR requirement applied inside gap between passbands:

a)	Measure OTA ACLR inside gap between passbands, if applicable.

b)	Measure OTA CACLR inside gap between passbands, if applicable.

#### 6.5.2.5	Test requirements

The OTA ACLR limit for repeater type 2-O and NCR-Fwd type 2-O is specified in table 6.5.2.5-1 for DL and UL for Wide Area class and DL for Local Area class.

The OTA ACLR limit for repeater type 2-O and NCR-Fwd type 2-O is specified in table 6.5.2.5-1a for UL for Local Area class.

The OTA ACLR absolute limit for repeater type 2-O and NCR-Fwd type 2-O is specified in table 6.5.2.5-2.

The OTA ACLR (CACLR) absolute limit for repeater type 2-O and NCR-Fwd type 2-O in table 6.5.2.5-2 or 6.5.2.5-5 or the ACLR (CACLR) limit in table 6.5.2.5-1, 6.5.2.5-3 or 6.5.2.5-4, whichever is less stringent, shall apply.

For a RIB operating in non-contiguous spectrum, the OTA ACLR requirement for repeater type 2-O and NCR-Fwd type 2-O in table 6.5.2.5-3 shall apply in gaps between passbands for the frequency ranges defined in the table, while the OTA CACLR requirement for repeater type 2-O and NCR-Fwd type 2-O in table 6.5.2.5-4 shall apply in gaps between passbands for the frequency ranges defined in the table.

The CACLR in a gap between passbands is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the gap between passbands, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective passband edges.

The assumed filter for the adjacent channel frequency is defined in table 6.5.2.5-4 and the filters on the assigned channels are defined in table 6.5.2.5-6.

For operation in non-contiguous spectrum, the CACLR for NR carriers located on either side of the gap between passbands shall be higher than the value specified in table 6.5.2.5-4.

Table 6.5.2.5-1: Repeater type 2-O and NCR-Fwd type 2-O ACLR limit for DL and UL for WA class and DL for LA class

| Repeater nominal channel bandwidth BWNominal (MHz) | Repeater adjacent channel centre frequency offset below or above passband edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWNominal/2 | NR of same BW (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) |
| NOTE 1: BWNominal is the nominal channel bandwidth. BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides the largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz NOTE 6: For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the the bandwidth between the lower edge of the passband and the lower edge of the NCR-MT carrier for lower side, or between the upper edge of the passband and the upper edge of the NCR-MT carrier for upper side. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT based on NCR-MT channel bandwidth shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier. |  |  |  |  |

Table 6.5.2.5-1a: Repeater type 2-O and NCR-Fwd type 2-O ACLR limit for UL LA class

| Repeater nominal channel bandwidth BWNominal (MHz) | Repeater adjacent channel centre frequency offset below or above passband edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWNominal/2 | NR of same BW (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| NOTE 1: BWNominal is the nominal channel bandwidth. BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides the largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz NOTE 6: For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the the bandwidth between the lower edge of the passband and the lower edge of the NCR-MT carrier for lower side, or between the upper edge of the passband and the upper edge of the NCR-MT carrier for upper side. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT based on NCR-MT channel bandwidth shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier. |  |  |  |  |

Table 6.5.2.5-2: Repeater type 2-O and NCR-Fwd type 2-O ACLR absolute limit

| Repeater class | ACLR absolute limit (Note 1) | ACLR absolute limit (Note 2) |
| --- | --- | --- |
| Wide-area DL and UL | -10.3 dBm/MHz | -10.1 dBm/MHz |
| Medium-range DL | -17.3 dBm/MHz | -17.1 dBm/MHz |
| Local-area DL | -17.3 dBm/MHz | -17.1 dBm/MHz |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 43.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz |  |  |

Table 6.5.2.5-3: Repeater type 2-O and NCR-Fwd type 2-O ACLR limit in non-contiguous spectrum for DL and UL for WA class and DL for LA class

| Repeater nominal channel bandwidth BWNominal (MHz) | Gap between passbands size (Wgap) where the limit applies (MHz) | Repeater adjacent channel centre frequency offset below or above passband edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | Wgap≥ 100 (Note 6) Wgap≥ 250 (Note 7) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) |
| 50, 100, 200, 400 | Wgap≥ 400 (Note 7) Wgap≥ 250 (Note 6) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) |
| NOTE 1: BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides the largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz. NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 52.6 GHz. NOTE 6: Applicable in case the nominal channel bandwidth at the other edge of the gap is ≤ 100 MHz. NOTE 7: Applicable in case the nominal channel bandwidth at the other edge of the gap is > 100 MHz. NOTE 8: For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the the bandwidth between the lower edge of the passband and the lower edge of the NCR-MT carrier for lower side, or between the upper edge of the passband and the upper edge of the NCR-MT carrier for upper side. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT based on NCR-MT channel bandwidth shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier. |  |  |  |  |  |

Table 6.5.2.2-3a: Repeater type 2-O and NCR-Fwd type 2-O ACLR limit in non-contiguous spectrum for UL for LA class

| Repeater nominal channel bandwidth BWNominal (MHz) | Gap between passbands size (Wgap) where the limit applies (MHz) | Repeater adjacent channel centre frequency offset below or above the passband edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | Wgap≥ 100 (Note 6) Wgap≥ 250 (Note 7) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| 50, 100, 200, 400 | Wgap≥ 400 (Note 7) Wgap≥ 250 (Note 6) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| NOTE 1: BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides the largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz. NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 52.6 GHz. NOTE 6: Applicable in case the nominal channel bandwidth at the other edge of the gap is ≤ 100 MHz. NOTE 7: Applicable in case the nominal channel bandwidth at the other edge of the gap is > 100 MHz. NOTE 8: For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the the bandwidth between the lower edge of the passband and the lower edge of the NCR-MT carrier for lower side, or between the upper edge of the passband and the upper edge of the NCR-MT carrier for upper side. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT based on NCR-MT channel bandwidth shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier. |  |  |  |  |  |

Table 6.5.2.5-4: Repeater type 2-O and NCR-Fwd type 2-O CACLR limit in non-contiguous spectrum for DL and UL for WA class and DL for LA class

| Repeater nominal channel bandwidth BWNominal (MHz) | Gap between passbands size (Wgap) where the limit applies (MHz) | Repeater adjacent channel centre frequency offset below or above passband edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | 50 ≤Wgap< 100 (Note 6) 50 ≤Wgap< 250 (Note 7) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) |
| 50, 100, 200, 400 | 200 ≤Wgap< 400 (Note 7) 200 ≤Wgap< 250 (Note 6) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) |
| NOTE 1: BWConfig is the nominal bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides nominal bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz. NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 52.6 GHz. NOTE 6: Applicable in case the nominal channel bandwidth at the other edge of the gap is ≤ 100 MHz. NOTE 7: Applicable in case the nominal channel bandwidth at the other edge of the gap is > 100 MHz. |  |  |  |  |  |

Table 6.5.2.5-4a: Repeater type 2-O and NCR-Fwd type 2-O CACLR limit in non-contiguous spectrum for UL for LA class

| Repeater channel bandwidth BWNominal (MHz) | Gap between passbands size (Wgap) where the limit applies (MHz) | Repeater adjacent channel centre frequency offset below or above the passband edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | 50 ≤Wgap< 100 (Note 6) 50 ≤Wgap< 250 (Note 7) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| 50, 100, 200, 400 | 200 ≤Wgap< 400 (Note 7) 200 ≤Wgap< 250 (Note 6) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| NOTE 1: BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides the largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz. NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 52.6 GHz. NOTE 6: Applicable in case the nominal channel bandwidth at the other edge of the gap is ≤ 100 MHz. NOTE 7: Applicable in case the nominal channel bandwidth at the other edge of the gap is > 100 MHz. |  |  |  |  |  |

Table 6.5.2.5-5: Repeater type 2-O and NCR-Fwd type 2-O CACLR absolute limit

| Repeater class | ACLR absolute limit (Note 1) | ACLR absolute limit (Note 2) |
| --- | --- | --- |
| Wide-area DL and UL | -10.3 dBm/MHz | -10.1 dBm/MHz |
| Medium-range DL | -17.3 dBm/MHz | -17.1 dBm/MHz |
| Local-area DL | -17.3 dBm/MHz | -17.1 dBm/MHz |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 43.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz |  |  |

Table 6.5.2.5-6: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the gap between passbands | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

For OTA ACLR requirement of Wide area NCR-MT, the test requirement defined in clause 6.7.3.5.2 of TS 38.141-2 [6] is applicable.

For OTA ACLR requirement for Local area NCR-MT, the test requirement is defined as UE ACLR requirements specified in clause 6.5.2.3 in TS 38.101-2 [14] plus measurement uncertainty as specified in the following table 6.5.2.5-7.

Table 6.5.2.5-7: measurement uncertainty for NRACLR for FR2-1

|  | The applicable frequency range |  |  |
| --- | --- | --- | --- |
|  | 24.25 – 29.5 GHz | 37 – 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz |
| Relative ACLR | ±2.3 dB | ±2.6 dB | ±2.8 dB |
| Absolute ACLR: | ±2.7 dB | ±2.7 dB | ±2.9 dB |

### 6.5.3	OTA operating band unwanted emissions

#### 6.5.3.1	Definition and applicability

The requirements of either clause 6.5.3.4.1 (Category A limits) or clause 6.5.3.4.2 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for General OTA transmitter spurious emissions requirements (repeater type 2-O and NCR-Fwd type 2-O) in clause 6.5.4. In addition, the limits in clause 6.5.3.4.3 may also apply.

Out-of-band emissions in FR2 are limited by OTA operating band unwanted emission limits.

For repeater type 2-O and NCR-Fwd type 2-O, unless otherwise stated, the OTA operating band unwanted emission limits in FR2 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band.

The values of ΔfOBUE are defined in table 6.5.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in contiguous spectrum, the requirements apply to the frequencies (ΔfOBUE) starting from the edge of the passband. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any gap between passbands.

Emissions shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the passband edge frequency and the nominal -3dB point of the measuring filter closest to the passband edge.

-	f_offset is the separation between the passband edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 6.5.1-1.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

In addition, inside any gap between passbands for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the limits specified for the adjacent sub-blocks on each side of the gap between passbands. The limit for each sub-block is specified in clauses 6.5.3.4.1 and 6.5.3.4.2 below, where in this case:

-	f is the separation between the sub-block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub-block edge.

-	f_offset is the separation between the sub-block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the gap between passbands bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

#### 6.5.3.2	Minimum requirement

The minimum requirement for repeater type 2-O is defined in TS 38.106 [2], clause 7.5.3.2.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.5.3.3.1.1.

For Wide Area NCR-MT type 2-O, the BS OBUE requirements specified in clause 9.7.4.3 in TS 38.104 [5] apply.

For Local Area NCR-MT type 2-O, the UE SEM requirements specified in clause 6.5.2.1 in TS 38.101-2 [14] apply.

#### 6.5.3.3	Test purpose

This test measures the emissions close to the assigned channel bandwidth of the wanted signal, while the transmitter is in operation.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

##### 6.5.3.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I.

1)	Place the RF repeater or NCR at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	The measurement devices characteristics shall be:

-	detection mode: true RMS.

-	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

4a)	For RF repeater, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models RDL-FR2-TM1.1and RUL-FR2-TM1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

-	For NCR-Fwd, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRDL-FR2-TM1.1 and NCRUL-FR2-TM1.1 in clause 4.9A.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

-	For NCR-MT, set the NCR-MT with the declared maximum output power according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRUL-FR2-TM1.1 in clause 4.9.2.

4b)	Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off. Verify measured result is enough below requirement limit.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G) whilst maintaining the correct direction of arrival for the test signal.

6)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For connector under test declared to operate in non-contiguous spectrum, the emission within the sub-block gap shall be measured using the specified measurement bandwidth from the closest sub block edge.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex G).

8)	Calculate TRPEstimate using the measurements made in Step 7.

#### 6.5.3.5	Test requirements

##### 6.5.3.5.1	OTA operating band unwanted emission limits (Category A)

Repeater type 2-O and NCR-Fwd type 2-O unwanted emissions shall not exceed the maximum levels specified in table 6.5.3.5.1-1 or 6.5.3.5.1-2 or 6.5.3.5.1-3.

Table 6.5.3.5.1-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 32.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. |  |  |  |

Table 6.5.3.5.1-2: OBUE limits applicable in the frequency range 37 – 43.5 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. |  |  |  |

Table 6.5.3.5.1-3: OBUE limits applicable in the frequency range 43.5 – 48.2 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.1 dBm, Max(Prated,t,TRP – 30.1 dB, -9.1 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. |  |  |  |

For OTA OBUE requirement of  Wide area NCR-MT, the test requirement defined in clause 6.7.4.5.2.2 of TS 38.141-2 [6] is applicable.

For OTA OBUE requirement for Local area NCR-MT, the test requirement is defined as UE OBUE requirements specified in clause 6.5.2.1 in TS 38.101-2 [14] plus measurement uncertainty as specified in the following table 6.5.3.4.1-4.

Table 6.5.3.5.1-4: measurement uncertainty for OBUE for FR2-1

|  | The applicable frequency range |  |  |
| --- | --- | --- | --- |
|  | 24.25 – 29.5 GHz | 37 – 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz |
| OBUE: | ±2.7 dB | ±2.7 dB | ±2.9 dB |

##### 6.5.3.5.2	OTA operating band unwanted emission limits (Category B)

Repeater type 2-O and NCR-Fwd type 2-O unwanted emissions shall not exceed the maximum levels specified in table 6.5.3.5.2-1 or 6.5.3.5.2-2 or 6.5.3.5.2-3.

Table 6.5.3.5.2-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 32.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Table 6.5.3.5.2-2: OBUE limits applicable in the frequency range 37 – 43.5 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

For OTA OBUE requirement of  Wide area NCR-MT, the test requirement defined in clause 6.7.4.5.2.3 of TS 38.141-2 [6] is applicable.

Table 6.5.3.5.2-2: OBUE limits applicable in the frequency range 43.5 - 48.2 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

##### 6.5.3.5.3	Additional OTA operating band unwanted emission requirements

6.5.3.5.3.1	Protection of Earth Exploration Satellite Service

For repeater operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 6.5.3.5.3.1-1 for DL and in table 6.5.3.5.3.1-2 for UL.

Table 6.5.3.5.3.1-1: OBUE limits for protection of Earth Exploration Satellite Service for DL

| Frequency range | Limit | Measurement Bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm (Note 1) | 200 MHz |
| 23.6 – 24 GHz | -9 dBm (Note 2) | 200 MHz |
| NOTE 1: This limit applies to repeater brought into use on or before 1 September 2027 in countries not adopting EU Decision 2020/590 [24]. NOTE 2: This limit applies to repeater brought into use after 1 September 2027 or to repeater in countries adopting EU Decision 2020/590 [24]. |  |  |

Table 6.5.3.5.3.1-2: OBUE limits for protection of Earth Exploration Satellite Service for UL

| Frequency range | Limit | Measurement Bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | 1 dBm | 200 MHz |

### 6.5.4	OTA transmitter spurious emissions

#### 6.5.4.1	Definition and applicability

For repeater type 2-O and NCR-Fwd type 2-O, the OTA transmitter spurious emission limits apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of the downlink operating band, up to ΔfOBUE above the highest frequency of the downlink operating band, where the ΔfOBUE is defined in table 6.5.1-1.

#### 6.5.4.2	Minimum requirement

The minimum requirement  for repeater type 2-O is defined in TS 38.106 [2], clause 7.5.4.2..

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.5.3.1.1.

For Wide Area NCR-MT type 2-O, the OTA TX spurious emission requirements are as defined in clause 7.5.4.2 in TS 38.106 [2].

For Local Area NCR-MT type 2-O, the OTA TX spurious emission requirements defined for NR UE in clause 6.5.3 in TS 38.101-2 [14] apply.

#### 6.5.4.3	Test purpose

This test measures conducted spurious emissions while the transmitter is in operation.

#### 6.5.4.4	Method of test

##### 6.5.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B when testing the spurious emissions below FDL_low - ΔfOBUE,

-	T when testing the spurious emissions above FDL_high + ΔfOBUE; see clause 4.9.1.

##### 6.5.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex.

1)	Place the RF repeater or NCR at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	The measurement devices characteristics shall be:

-	detection mode: true RMS.

4a)	for RF repeater, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models RDL-FR2-TM1.1 and RUL-FR2-TM1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

-	For NCR-Fwd, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRDL-FR2-TM1.1 and NCRUL-FR2-TM1.1 in clause 4.9A.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

-	For NCR-MT, set the NCR-MT with the declared maximum output power according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRUL-FR2-TM1.1 in clause 4.9.2.

4b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off. Verify measured result is enough below requirement limit.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed with the specified measurement bandwidth (see annex I) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the emission at the specified frequencies with specified measurement bandwidth.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex G).

8)	Calculate TRPEstimate using the measurements made in Step 7.

#### 6.5.4.5	Test requirements

##### 6.5.4.5.1	General

The requirements of either clause 6.5.4.5.2 (Category A limits) or clause 6.5.4.5.3 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for Operating band unwanted emissions in clause 6.5.3.

##### 6.5.4.5.2	OTA transmitter spurious emissions (Category A)

For RF repeater, NCR-Fwd and Wide-area NCR-MT, the power of any spurious emission shall not exceed the limits in table 6.5.4.5.2-1

Table 6.5.4.5.2-1: Repeater radiated Tx spurious emission limits in FR2

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [4], s4.1 NOTE 2: Upper frequency as in ITU-R SM.329 [4], s2.5 table 1. |  |  |  |

For Local area NCR-MT, the power of any spurious emission shall not exceed the limits in table6.5.4.5.2-2 and 6.5.4.5.2-3.

Table 6.5.4.5.2-2: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 |

Table 6.5.4.5.2-3: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |

##### 6.5.4.5.3	OTA transmitter spurious emissions (Category B)

The power of any spurious emission shall not exceed the limits in table 6.5.4.5.3-1.

Table 6.5.4.5.3-1: Repeater radiated Tx spurious emission limits in FR2 (Category B)

| Frequency range (Note 4) | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 2nd harmonic of the upper frequency edge of the operating band | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [9], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in Table 6.5.4.5.3-2. |  |  |  |

Table 6.5.4.5.3-2: Step frequencies for defining the Repeater radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [9], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 6.5.1-1 |  |  |  |  |  |  |

##### 6.5.4.5.4	Additional OTA transmitter spurious emissions requirements

These requirements may be applied for the protection of systems operating in frequency ranges other than the repeater-Node. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the repeater-Node, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation.

##### 6.5.4.5.4.1	Limits for protection of Earth Exploration Satellite Service

For repeater operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 6.5.4.5.4.1-1 and Table 6.5.4.5.4.1-2.

Table 6.5.4.5.4.1-1: Limits for protection of Earth Exploration Satellite Service for DL

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm | 200 MHz | Note 1 |
| 23.6 – 24 GHz | -9 dBm | 200 MHz | Note 2 |
| NOTE 1: This limit applies to repeater brought into use on or before 1 September 2027 in countries not adopting EU Decision 2020/590 [24]. NOTE 2: This limit applies to repeater brought into use after 1 September 2027 or to repeater in countries adopting EU Decision 2020/590 [24]. |  |  |  |

Table 6.5.4.5.4.1-2: OBUE limits for protection of Earth Exploration Satellite Service for UL

| Frequency range | Limit | Measurement Bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | 1 dBm | 200 MHz |

## 6.6	OTA Repeater Error Vector Magnitude

### 6.6.1	Downlink repeater error vector magnitude

#### 6.6.1.1	General

The Repeater Error Vector Magnitude (EVM) is a measure of the difference between the symbols provided at the input of the repeater and the measured signal symbols at the output of the repeater after the equalization by the measurement equipment. This difference is called the error vector. Details about how the RF repeater or NCR EVM is determined are the same as specified in TS 38.104 [5] Annex [C] for FR2. The RF repeater or NCR EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent. The accuracy of the input symbols is counted in the measurement uncertainty.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range on the transmit side and the AoA of the incident wave of the received signal is in the reference direction at the receive side.

The RF repeater or NCR EVM requirement is applicable when the RF repeater or NCR is operating with an input power level within the range from what is required to reach the rated beam EIRP output power (Prated,p,EIRP) to the minimum power levels in table 6.6.1.1-1.

Table 6.6.1.1-1: Minimum input power for RF repeater or NCR EVM

| RF repeater or NCR class | Minimum input power (dBm/MHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 24.25 – 33.4 GHz |  |  | 37 – 52.6 GHz |  |  |
|  | Up to 16 QAM | 64QAM 1 | 256QAM 2 | Up to 16 QAM | 64QAM 1 | 256QAM 2 |
| WA, MR, LA | -77- GRX_ANT | -73- GRX_ANT | -66- GRX_ANT | -75- GRX_ANT | -71- GRX_ANT | -64- GRX_ANT |
| Note 1: support of 64QAM is based on the declaration Note 2: support of 256QAM is based on the declaration |  |  |  |  |  |  |

Where GRX_ANT is the gain of the receive side antennas and is based on EIRP and TRP declaration.

#### 6.6.1.2	Minimum requirements

The minimum requirement for repeater type 2-O is defined is in TS 38.106 [2] clause 7.6.1.2.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.6.1.2A.1.1.

#### 6.6.1.3	Test purpose

To verify that the downlink RF repeater or NCR EVM deterioration is within the limit specified by the minimum requirements after the signal passed through the RF Repeater or NCR.

#### 6.6.1.4	Method of test

##### 6.6.1.4.1	Initial conditions

Test environment: normal; see Annex B.2

RF channels to be tested for single carrier:

-	B and T; see clause 4.9.1.

Passband positions to be tested for multi-carrier:

-	BRFBW and TRFBW in single-band operation, see clause 4.9.1;

Directions to be tested:

-	The OTA coverage range reference direction (D.14).

-	The OTA coverage range maximum directions (D.15).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

##### 6.6.1.4.2	Procedure

1a)	Place the RF repeater or NCR at the positioner.

1b) Verify measurement impact from feeding test signal by generating a signal for RF repeater or NCR input with RF repeater or NCR to be turned off. Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the RF repeater or NCR with the test system.

3)	Orient the positioner (and RF repeater or NCR) in order that the direction to be tested aligns with the test antenna.

For repeater type 2-O or NCR-Fwd type 2-O declared to be capable of single carrier operation only, set the RF repeater or NCR to transmit a signal according to the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	RDL-FR2-TM3.1a or NCRDL-FR2-TM3.1a with 256QAM signal if 256QAM is supported by RF repeater or NCR without power back-off, or

-	RDL-FR2-TM3.1a or NCRDL-FR2-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by RF repeater or NCR with power back-off, and RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with highest modulation order supported without power back-off, or

-	RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by RF repeater or NCR without power back-off, or

-	RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with highest modulation order without power back-off if 64QAM is not supported by RF repeater or NCR, or

-	if 64 QAM is supported by RF repeater or NCR with power back-off, RDL-FR2-TM 3.1 or NCRDL-FR2-TM3.1 with 64QAM at manufacturer's declared rated output power (Prated,c,EIRP) and RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with highest modulation order supported at maximum power.

For repeater type 2-O or NCR-Fwd type 2-O declared to be capable of multi-carrier, set the RF repeater or NCR to transmit according to:

-	RDL-FR2-TM3.1a or NCRDL-FR2-TM3.1a with 256QAM signal if 256QAM is supported by RF repeater or NCR without power back-off, or

-	RDL-FR2-TM3.1a or NCRDL-FR2-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by RF repeater or NCR with power back-off, and RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1a at maximum power, or

-	RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by RF repeater or NCR without power back-off, or

-	RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with highest modulation order supported without power back-off if 64QAM is not supported by RF repeater or NCR, or

-	if 64QAM is supported by RF repeater or NCR with power back-off, RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with 64QAM signal at manufacturer's declared rated output power (Prated,c,EIRP) and RDL-FR2-TM3.1 or NCRDL-FR2-TM3.1 with highest supported modulation order at maximum power

For RDL-FR1-TM 3.1a or NCRDL-FR2-TM3.1a and RDL-FR2-TM 3.1 or NCRDL-FR2-TM3.1, power back-off shall be applied if it is declared.

4)	For each carrier, measure the RF repeater or NCR EVM and frequency error as defined in annex L in TS 38.141-2 [6].

5)	Adjust the input power to the RF Repeater or NCR to create the maximum nominal RF Repeater or NCR output power at maximum gain.

6)	Measure the RF repeater or NCR EVM and frequency error as defined in TS 38.141-2 [6] Annex L.

7)	Repeat the procedure with all the narrower bandwidths.

#### 6.6.1.5	Test requirement

The downlink of the RF Repeater or NCR EVM levels for different modulation schemes shall not exceed values in table 6.6.1.5-1.

Table 6.6.1.5-1: RF Repeater or NCR EVM test requirements

| Parameter | Required EVM |
| --- | --- |
| QPSK, 16QAM | 13.9 % |
| 64QAM | 9.5 % 1 |
| 256QAM | 5.3 %2 |
| Note 1: support of 64QAM is based on the declaration Note 2: support of 256QAM is based on the declaration. |  |

### 6.6.2	Uplink Repeater error vector magnitude

#### 6.6.2.1	General

The RF Repeater or NCR Error Vector Magnitude is a measure of the difference between the reference waveform provided at the input of the RF repeater or NCR and the measured waveform at the output of the RF repeater or NCR. This difference is called the error vector. Details about how the RF repeater or NCR EVM is determined are the same as specified in TS 38.101-2 [14] Annex F. Before calculating the RF repeater or NCR EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the RF repeater or NCR EVM.

The measured waveform is further equalised using the channel estimates subjected to the RF repeater or NCR EVM equaliser spectrum flatness requirement specified in TS 38.101-2 [14] sub-clauses 6.4.2.4 and 6.4.2.5. For DFT-s-OFDM waveforms, the RF repeater or NCR EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the RF repeater or NCR EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. The accuracy of the input waveform is counted in the measurement uncertainty.

The basic RF repeater or NCR EVM measurement interval is one slot in the time domain. The RF repeater or NCR EVM measurement interval is reduced by any symbols that contains an allowable power transient in the measurement interval as defined in TS 38.101-2 [14] clause 6.3.3 for EVM for UE.

All the parameters defined in clause 6.6.2 are defined using the measurement methodology specified in TS 38.101-2 [14] Annex F.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range on the transmit side and the AoA of the incident wave of the received signal is in the reference direction at the receive side.

The RF repeater or NCR EVM requirement is applicable when the RF repeater or NCR is operating with an input power level within the range from what is required to reach the rated beam EIRP output power (Prated,p,EIRP) to the minimum input power levels in table 6.6.2.1-1.

Table 6.6.2.1-1: Minimum input power for RF repeater or NCR EVM

| RF repeater or NCR class | Minimum input power (dBm/MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 24.25 – 33.4 GHz |  | 37 – 52.6 GHz |  |
|  | Up to 16 QAM | 64QAM 1 | Up to 16 QAM | 64QAM1 |
| WA, MR, LA | -77- GRX_ANT | -73- GRX_ANT | -75- GRX_ANT | -71- GRX_ANT |
| Note 1:- support of 64QAM is based on the declaration |  |  |  |  |

Where GRX_ANT is the gain of the receive side antennas and is calculated from EIRP and TRP declaration.

#### 6.6.2.2	Minimum requirement

The minimum requirement for repeater type 2-O is defined is in TS 38.106 [2] clause 7.6.2.2.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.6.2.3.1.1.

#### 6.6.2.3	Test purpose

To verify that the uplink RF repeater or NCR EVM deterioration is within the limit specified by the minimum requirements after the signal passed through the RF Repeater or NCR.

#### 6.6.2.4	Method of test

##### 6.6.2.4.1	Initial conditions

RF channels to be tested for single carrier:

-	B and T; see clause 4.9.1.

Passband positions to be tested for multi-carrier:

-	BRFBW and TRFBW in single-band operation, see clause 4.9.1;

Directions to be tested:

-	The OTA coverage range reference direction (D.14).

-	The OTA coverage range maximum directions (D.15).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

##### 6.6.2.4.2	Procedure

1a)	Place the RF repeater or NCR at the positioner.

1b) Verify measurement impact from feeding test signal by generating a signal for RF repeater or NCR input with RF repeater or NCR to be turned off. Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the RF repeater or NCR with the test system.

3)	Orient the positioner (and RF repeater or NCR) in order that the direction to be tested aligns with the test antenna.

For repeater type 2-O or NCR-Fwd type 2-O declared to be capable of multi-carrier operation, set the RF repeater or NCR to transmit according to:

-	RUL-FR2-TM3.1 or NCRUL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by RF repeater or NCR without power back-off, or

-	RUL-FR2-TM3.1 or NCRUL-FR2-TM3.1 with highest modulation order supported without power back-off if 64QAM is not supported by RF repeater or NCR, or

-	if 64QAM is supported by RF repeater or NCR with power back-off, RUL-FR2-TM3.1 or NCRUL-FR2-TM3.1 with 64QAM signal at manufacturer's declared rated output power (Prated,c,EIRP) and RUL-FR2-TM3.1 or NCRUL-FR2-TM3.1 with highest supported modulation order at maximum power

For RUL-FR1-TM 3.1a or NCRUL-FR2-TM3.1a and RUL-FR2-TM 3.1 or NCRUL-FR2-TM3.1, power back-off shall be applied if it is declared.

6)	For each carrier, measure the RF repeater or NCR EVM and frequency error as defined in annex L.

7)	Repeat steps 5 and 6 for RUL-FR2-TM2 if 256QAM is not supported by RF repeater or NCR or for NCRUL-FR1-TM2a if 256QAM is supported by RF repeater or NCR. For RUL-FR1-TM2 and RUL-FR1-TM2a or NCRUL-FR1-TM2 and NCRUL-FR1-TM2a the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.4.3.4.2 and test requirements in clause 6.4.3.5.1.

#### 6.6.2.5	Test requirement

The uplink of the RF Repeater or NCR EVM levels for different modulation schemes shall not exceed values in table 6.6.2.5-1.

Table 6.6.2.5-1: RF Repeater or NCR EVM test requirements

| Parameter | Unit | Average repeater EVM level |
| --- | --- | --- |
| QPSK, 16QAM | % | 13.9 |
| 64 QAM | % | 9.51 |
| Note 1: support of 64QAM is based on the declaration |  |  |

## 6.7	OTA input intermodulation

### 6.7.1	Definition and applicability

#### 6.7.1.1	General

The input intermodulation is a measure of the capability of the repeater to inhibit the generation of interference in the passband, in the presence of interfering signals on frequencies other than the passband.

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the RF repeater or NCR to maintain the wanted frequency free of internally created interference.

The measurements shall apply to both uplink and downlink paths of the RF repeater or NCR, during the transmitter ON state at maximum gain.

#### 6.7.1.2	Minimum requirements

The minimum requirement for repeater type 2-O is defined is in TS 38.106 [2] clause 7.7.2.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.7.3.1.1.

#### 6.7.1.3	Test purpose

The purpose of this test is to verify that the RF repeater or NCR meets the intermodulation characteristics requirements as specified by the minimum requirements.

#### 6.7.1.4	Method of test

##### 6.7.1.4.1	Initial conditions

Test environment: normal; see Annex B.2.

A measurement system set-up is shown in Annex E.

##### 6.7.1.4.2	Procedure

1a)	Verify measurement impact from feeding test signal by generating a signal for RF repeater or NCR input with RF repeater or NCR to be turned off.  Verify measured result is enough below requirement limit.

1b)	Set the RF repeater or NCR to maximum gain.

2)	Place the RF repeater or NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex E.

3)	Align the test antennas and RF repeater or NCR both in input and output directions of the RF repeater or NCR.

4)	Align the RF repeater or NCR and test antennas so that RF repeater or NCR antennas are polarization matched with the test antenna(s)

5)	Configure the beam peak direction of the RF repeater or NCR according to declared reference beam direction pair for the appropriate beam identifier.

6)	Adjust the frequency of the input signals, either below or above the passband, so that one carrier, f1, is 1 MHz outside the channel edge frequency of the first or last channel in the passband, and the lowest order intermodulation product from the two carriers is positioned in the centre of the passband.

7)	Measure the increase in output power in the passband when the interferer is applied

8)	Repeat the measurement for all supported polarizations and all specified measurement directions.

9)	Repeat the measurement for the opposite path of the RF repeater or NCR.

#### 6.7.1.5	Test requirements

For the parameters specified in table 6.7.1.5-1, the power in the passband shall not increase with more than:

13.9 dB, 24.25 GHz < f ≤ 29.5 GHz

13.9 dB, 37 GHz < f ≤ 43.5 GHz

15.4 dB, 43.5 GHz < f ≤ 48.2 GHz

at the output of the RF repeater or NCR as measured with 1 MHz measurement bandwidth, compared to the level obtained without interfering signals applied.

Table 6.7.1.5-1 specifies the parameters for two interfering signals, where:

-	f1 offset is the offset from the channel edge frequency of the first or last channel in the passband of the closer carrier.

-	GRX_ANT is the gain of the receive side antennas and is calculated from EIRP and TRP declaration.

Table 6.7.1.5-1: Input intermodulation requirement parameters

| f1 offset | Interfering signal levels | Type of signals | Measurement bandwidth |
| --- | --- | --- | --- |
| 1 MHz | -53dBm – G_RX_ANT | 2 CW carriers | 1 MHz |

## 6.8	OTA Adjacent Channel Rejection Ratio (ACRR)

### 6.8.1	Definitions and applicability

OTA Adjacent Channel Rejection Ratio (ACRR) is the ratio of the average gain over a carrier of the RF repeater or NCR in the passband to the average gain of the RF repeater or NCR over an adjacent channel outside the RF repeater or NCR passband. The requirement shall apply to the uplink and downlink of the RF repeater or NCR. The bandwidth of the channel inside the passband and the adjacent channel shall be of the same type (reference carrier) with bandwidths as defined by nominal channel bandwidth.

The requirement is differentiated between downlink and uplink.

The requirement shall apply during the transmitter ON state.

The ACRR is a ratio of gain in the adjacent channel to gain in the wanted channel. The gain in each case is defined as the ratio of TRP output power to directional input power.

### 6.8.2	Co-existence with NR

This requirement shall be applied for the protection of NR signals in geographic areas in which NR RF Repeater or NCR and NR BS are deployed so that they serve adjacent channels. The reference carrier is a NR carrier.

#### 6.8.2.1	Minimum requirements

The minimum requirement  for repeater type 2-O is defined in TS 38.106 [2] sub-clause 7.8.1.1.

The minimum requirement for NCR-Fwd type 2-O is defined in TS 38.106 [2], clause 7.7.3.1.1.

#### 6.8.2.2	Test purpose

To verify that the RF Repeater or NCR OTA ACRR requirement is met as specified in sub-clause 6.8.2.1.

#### 6.8.2.3	Method of test

##### 6.8.2.3.1	Initial conditions

Test environment: 	normal; see Annex B.2.

RF channels to be tested for single carrier: B, M, T; see clause 4.9.1.

Beams to be tested:

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex G) as long as the required TRP level is achieved.

##### 6.8.2.3.2	Procedure

1a)	Place the RF repeater or NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.7.

1b) Verify measurement impact from feeding test signal by generating a signal for RF repeater or NCR input with RF repeater or NCR to be turned off. Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation of the RF repeater or NCR with the test system.

3)	Align the RF repeater or NCR with the test antenna in the declared direction to be tested.

4)	Align the RF repeater or NCR to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Set the signal generator to transmit a signal modulated with test model RDL-FR2-TM1.1 for RF repeater downlink or NCRDL-FR2-TM1.1 for NCR downlink and test model RUL-FR2-TM1.1 for RF repeater uplink or NCRUL-FR2-TM1.1 for NCR uplink as defined in section 4.9 at the first or last channel with channel offset from frequency range of passband defined in section 6.8.2.3.3 within the pass band.

6)	Adjust the input power to the RF Repeater or NCR to create the maximum nominal RF Repeater or NCR output power at maximum gain

7)	Orient the positioner (and RF repeater or NCR) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex G).

8)	Measure the absolute power of the assigned channel frequency.

9)	Repeat step 7-8 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex G).

10)		Calculate TRPEstimate for the absolute total radiated power of a carrier of the RF repeater or NCR in the passband and using the measurements made in Step 9.

11)		Set the signal generator to transmit the same signal and the same input power at one of the channel offsets outside the RF repeater or NCR pass band according to Tables in section 6.8.2.3.3.

12)		Repeat step 7) to 9) for the measurement of the input signal outside the RF repeater or NCR pass-band;

13)		Calculate TRPEstimate for the absolute total radiated power of a carrier over an adjacent channel outside the RF repeater or NCR passband and using the measurements made in Step 9.

14)		Calculate relative ACRR estimate.

15)	Repeat step 5) to 14) until all channel offsets in Tables in section 6.8.2.3.3 are measured.

##### 6.8.2.3.3	Test Requirements

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal in the passband and a received signal on an adjacent channel outside RF repeater or NCR passband is from the same direction and are the same as the TX reference direction for the opposite DL/UL setting.

For a RF repeater or NCR operating at passband operating in FR2, the ACRR requirements in table 6.8.2.3.3-1 shall apply in downlink. In normal conditions the ACRR for downlink shall be higher than the value specified in the Table 6.8.2.3.3-1.

Table 6.8.2.3.3-1: RF Repeater or NCR Downlink ACRR

| Co-existence with other systems | Repeater or NCR Class | Channel offset from frequency edge of passband (MHz) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACRR limit (dB) |
| --- | --- | --- | --- | --- |
| NR | Wide Area RF repeater or NCR | BWNominal/2 | Square (BWConfig) | 25.3 (Note 2) 23.3 (Note 3) 23.1 (Note 4) |
|  | Medium Range RF repeater or NCR | BWNominal/2 | Square (BWConfig) | 25.3 (Note 2) 23.3 (Note 3) 23.1 (Note 4) |
|  | Local Area RF repeater or NCR | BWNominal/2 | Square (BWConfig) | 25.3 (Note 1,2) 23.3 (Note 1,3) 23.1 (Note 1,4) |
| NOTE 1: This requirement is not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25-29.5GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37-43.5 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 43.5-48.2GHz. NOTE 5: BWNominal is the nominal channel bandwidth. BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 6: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

For a RF repeater or NCR operating at passband operating in FR2, the ACRR requirements in table 6.8.2.3.3-2 shall apply in uplink. In normal conditions the ACRR for uplink shall be higher than the value specified in the Table 6.8.2.3.3-2.

Table 6.8.2.3.3-2: RF Repeater or NCR Uplink ACRR

| Co-existence with other systems | Repeater or NCR Class | Channel offset from frequency edge of passband (MHz) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACRR limit (dB) |
| --- | --- | --- | --- | --- |
| NR | Wide Area RF repeater or NCR | BWNominal/2 | Square (BWConfig) | 25.3 (Note 2) 23.3 (Note 3) 23.1 (Note 4) |
|  | Local Area RF repeater or NCR | BWNominal/2 | Square (BWConfig) | 14.3 (Note 1, 2) 13.3 (Note 1, 3) 13.1 (Note 1, 4) |
| NOTE 1: This requirement is not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25-29.5GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37-43.5 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 43.5-48.2GHz. NOTE 5: BWNominal is the nominal channel bandwidth. BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 6: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

## 6.9	OTA transmit ON/OFF power

### 6.9.1	OTA transmitter OFF power

#### 6.9.1.1	Definition and applicability

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the passband bandwidth of the RF repeater or NCR (BWpassband) centred on the assigned channel frequency during the transmitter OFF state. N = SCS/15, where SCS is Sub Carrier Spacing in kHz of the input signal. The OTA transmitter OFF power is defined as TRP.

#### 6.9.1.2	Minimum requirement for RF repeater

The minimum requirement is in TS 38.106 [2], clause 7.9.2.2.

#### 6.9.1.2A	Minimum requirement for NCR

The OTA transmitter OFF TRP spectral density for repeater type 2-O shall be less than -36 dBm/MHz.

#### 6.9.1.2A.1	Minimum requirement for NCR

##### 6.9.1.2A.1.1	Minimum requirement for NCR-Fwd

6.9.1.2A.1.1.1	Minimum requirement for NCR-Fwd type 2-O

The requirements in clause 7.9.2.2  in TS 38.106 [2] apply for NCR-Fwd type 2-O.

##### 6.9.1.2A.1.2	Minimum requirement for NCR-MT

6.9.1.2A.1.2.1	Minimum requirement for NCR-MT type 2-O

For Wide Area NCR-MT type 2-O, the OTA transmitter OFF TRP spectral density for shall be less than -36 dBm/MHz. For Local Area NCR-MT type 2-O, the transmit OFF power shall not exceed the values specified in Tables 6.9.1.2A.1.2.1-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.9.1.2A.1.2.1-1: Transmit OFF power for FR2-1

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n259, n260, n261, n262 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

#### 6.9.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

#### 6.9.1.4	Method of test

Requirement is tested together with transmitter transient period, as described in clause 6.9.2.4.

#### 6.9.1.5	Test requirements

The conformance testing of transmit OFF power is included in the conformance testing of transmitter transient period; therefore, see clause 6.9.2.5 for test requirements.

### 6.9.2	OTA transient period

#### 6.9.2.1	Definition and applicability

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF state to the transmitter ON state or vice versa. The transmitter transient period is illustrated in figure 6.9.2.1-1.

Figure 6.9.2.1-1: Example of relations between transmitter ON state, transmitter OFF state and transmitter transient period

This requirement shall be applied at each RIB supporting transmission in the operating band.

For a RF repeater or NCR that is not declared to be a long delay repeater (D.24), the beginning and end point of downlink and uplink bursts are referenced to the slot timing at the input.

For a RF repeater or NCR that is declared to be a long delay repeater (D.24), the beginning and end point of downlink and uplink bursts are referenced to the slot timing at the input plus the declared repeater delay.

#### 6.9.2.2	Minimum requirement for RF repeater

The minimum requirement is in TS 38.106 [2], clause 7.9.3.2.

#### 6.9.2.2A	Minimum requirement for NCR

##### 6.9.2.2A.1	Minimum requirement for NCR-Fwd

6.9.2.2A.1.1	Minimum requirement for NCR-Fwd type 2-O

The requirements in clause 7.9.3.2 in TS 38.106 [2] apply for NCR-Fwd type 2-O.

##### 6.9.2.2A.1.1	Minimum requirement for NCR-MT

6.9.2.2A.1.1.1	Minimum requirement for NCR-MT type 2-O

For Wide Area NCR-MT type 2-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 6.9.2.2A.1.1.1-1.

Table 6.9.2.2A.1.1.1-1: Minimum requirement for the OTA transmitter transient period for Wide Area NCR-MT type 2-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 3 |
| ON to OFF | 3 |

For Local Area NCR-MT type 2-O, the requirement from TS 38.101-2 [14] section 6.3.3.2 applies.

#### 6.9.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

#### 6.9.2.4	Method of test

##### 6.9.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see clause 4.9.1.

Directions to be tested:

-	The requirement is verified by an EIRP measurement at a direction corresponding to the OTA peak directions set reference beam direction pair (D.6) for the beam identifier (D.3) which provides the highest intended EIRP.

##### 6.9.2.4.2	Procedure

1a)	Place the repeater at the positioner.

1b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off. Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

4a)	For RF repeater transient period measurement, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models RDL-FR2-TM 1.1 and RUL-FR2-TM 1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Prated,in, EIRP + 10dB.

For NCR-Fwd transient period measurement, set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRDL-FR2 TM 1.1 and NCRUL-FR2 TM 1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Prated,in, EIRP + 10dB.

For NCR-MT transient period measurement, set  according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NCRUL-FR2 TM 1.1 in clause 4.9.2 at the declared maximum output power.

4b)For OFF power measurement for RF repeater and NCR-Fwd, set the signal generator RF output turned off for not to generate input signal.

For OFF power measurement for NCR-Fwd, set the NCR-MT to turn off its transmitter.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the mean EIRP spectral density as the power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the passband bandwidth of the repeater centred on the central frequency of the passband. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON state + 3 μs to 35/N μs before start of next transmitter ON state- 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

NOTE:	Make sure that the measurement receiver is not overloaded.

#### 6.9.2.5	Test requirements

The measured mean EIRP spectral density shall be less than -33.1 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 24.15 GHz < f ≤ 29.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.6) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density shall be less than -32.7 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ 43.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.6) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density shall be less than -32.4 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 43.5 GHz < f ≤ 48.2 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.6) for the beam identifier (D.3) which provides the highest intended EIRP

## 6.10	OTA output power dynamics

### 6.10.1	Definition and applicability

OTA output power dynamic for NCR-MT consist of three requirements, transmit OFF power, transmit ON-OFF transition period requirement and power control requirements. The transmit OFF power and transition period requirement is specified in clause 6.9.  For power control requirement for NCR-MT, it is the difference between the maximum and the minimum controlled transmit power in the channel bandwidth for a specified reference condition. The maximum and minimum output powers are defined as the mean power in at least one sub-frame 1ms

This requirement shall apply at each RIB supporting transmission in the operating band.

### 6.10.2	Minimum requirement

For WA NCR-MT type 2-O, the IAB-MT requirements specified in clause 9.4.2.1.3 output dynamic range requirement in TS 38.174 [19] applies.

For LA NCR-MT type 2-O, the UE requirements specified in clause 6.3.4.3 of relative power tolerance and clause 6.3.4.4 of aggregate  power tolerance in TS 38.102-2 [14] applies

### 6.10.3	Test purpose

The test purpose is to verify that the NCR-MT OTA total power dynamic range is within the limits specified by the minimum requirement.

### 6.10.4	Method of test

#### 6.10.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.9A.1.

Beams to be tested:

-	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.9).

Directions to be tested:

-	The OTA peak directions set reference beam direction pair (D.8).

#### 6.10.4.2	Procedure

1)	Place the NCR at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the NCR-MT with the test system.

3)	Orient the positioner (and NCR-MT) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the NCR-MT according to the declared beam direction pair.

5)	For NCR type 2-O, set the NCR-MT to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model:

-	NCRUL-FR2-TM3.1;

6)	Measure the power by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) over 1ms and calculate total EIRP for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For NCR type 2-O, set the NCR-MT to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NCRUL-FR2-TM3.1;

8)	Measure the power by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) over 1ms and calculate total EIRP for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 6.10.5	Test requirements

For Wide area NCR-MT the ΔP between the power measured in step 6 and step 8 of clause 6.10.5-1 shall be:

Table 6.10.5-1: NCR-MT type 2-O Output power dynamics test requirements.

| NCR-MT Type | NCR-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) -1.2 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) -1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.8 < ΔP ≤ 10 log(Maximum RB) + 15.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |

For Local area NCR-MT, the test requirement for power control in clause 6.3.4 in TS 38.521-2 [20] is applicable.

## 6.11	OTA transmit signal quality

### 6.11.1	Definition and applicability

Transmit signal quality is specified in terms of: frequency error and transmit modulation quality requirements.

### 6.11.2	Minimum requirement

#### 6.11.2.1	Frequency error requirements for NCR-MT

##### 6.11.2.1.1	Minimum requirement for NCR-MT type 2-O

For NCR-MT type 2-O, the requirements specified in clause 9.6.1.2.3 in TS 38.174 [19] applies.

#### 6.11.2.2	Transmit modulation quality

##### 6.11.2.2.1	Minimum requirement for NCR-MT type 2-O

For NCR-MT type 2-O, the requirements specified in clause 9.6.2.2.3 in TS 38.174 [19] applies.

### 6.11.3	Test purpose

The test purpose is to verify that OTA transmit signal quality is within the limit specified by the minimum requirement.

### 6.11.4	Method of test

#### 6.11.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B and T; see clause 4.9A.1.

Passband bandwidth positions to be tested for multi-carrier:

-	BRFBW and TRFBW in single-band operation, see clause 4.9A.1;

Directions to be tested:

-	The OTA coverage range reference direction (D.14).

-	The OTA coverage range maximum directions (D.15).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

#### 6.11.4.2	Initial conditions

1)	Place the NCR at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the NCR-MT with the test system.

3)	Orient the positioner (and NCR-MT) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the NCR-MT according to the direction to be tested.

5)	Set the NCR-MT to output according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For NCR-MT type 2-O declared to be capable of single carrier operation only, set the NCR-MT to transmit a signal according to the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	NCRUL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by NCR-MT without power back-off, or

-	NCRUL-FR2-TM 3.1 with highest modulation order without power back-off if 64QAM is not supported by NCR-MT, or

-	if 64 QAM is supported by NCR-MT with power back-off, NCRUL-FR2-TM 3.1 with 64QAM at manufacturer's declared rated output power (Prated,p,EIRP) and NCRUL-FR2-TM3.1 with highest modulation order supported at maximum power.

For NCR-MT type 2-O declared to be capable of multi-carrier operation, set the NCR-MT to transmit according to:

-	NCRUL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by NCR-MT without power back-off, or

-	NCRUL-FR2-TM3.1 with highest modulation order supported without power back-off if 64QAM is not supported by NCR-MT, or

-	if 64QAM is supported by NCR-MT with power back-off, NCRUL-FR2-TM3.1 with 64QAM signal at manufacturer's declared rated output power (Prated,p,EIRP) and NCRUL-FR2-TM3.1 with highest supported modulation order at maximum power

For NCRUL-FR1-TM 3.1a and NCRUL-FR2-TM 3.1, power back-off shall be applied if it is declared.

6)	For each carrier, measure the EVM and frequency error as defined in annex E.

7)	Repeat steps 5 and 6 for NCRUL-FR2-TM2 if 256QAM is not supported by NCR-MT type 2-O or for NCRUL-MT-FR2-TM2a if 256QAM is supported by NCR-MT type 2-O. For NCRUL-FR2-TM2 and NCRUL-FR1-TM2a the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.10.4.2.

### 6.11.5	Test requirements

For NCR-MT type 2-O, the EVM of each NR carrier for different modulation schemes on  PUSCH shall be less than the limits in table 6.11.5-1.

Table 6.11.5-1: EVM requirements for NCR-MT type 2-O

| Modulation scheme for PUSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |
| 64QAM | 9 |
| 256QAM | 4.5 |
| NOTE: 256QAM is not supported by FR2-1 NCR-MT PUSCH |  |

EVM requirements shall apply for each NR carrier over all allocated resource blocks and uplink slots for NCR-MT. PT-RS should be configured for localized setting for every fourth symbol for every second RB for NCR-MT. Different modulation schemes listed in table 6.11.5-1 shall be considered for rank 1.

For all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and uplink slots for NCR-MT within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Tables 6.11.5-2 and 6.11.5-3 below specify the EVM window length (W) for normal CP for NCR-MT type 2-O.

Table 6.11.5-2: EVM window length for normal CP, FR2-1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix lengthen FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 50 | 1024 | 72 | 36 | 50 |
| 100 | 2048 | 144 | 72 | 50 |
| 200 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.11.5-3: EVM window length for normal CP, FR2-1, 120 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 50 | 512 | 36 | 18 | 50 |
| 100 | 1024 | 72 | 36 | 50 |
| 200 | 2048 | 144 | 72 | 50 |
| 400 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 4. Symbol 0 of slot 0 and slot 4 may have a longer CP and therefore a lower percentage. |  |  |  |  |

## 6.12	OTA reference sensitivity

### 6.12.1	Definition and applicability

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 6.12.2	Minimum requirement for NCR-MT type 2-O

For NCR type 2-O of WA class, the reference sensitivity requirement is specified the same as Wide Area IAB-MT reference sensitivity power level in TS 38.174 [19], clause 10.3.3.3.

For NCR type 2-O of LA class, the reference sensitivity  requirement is specified the same as reference sensitivity power level for power class 3 in TS 38.101-2 [14], clause 7.3.2.3.

### 6.12.3	Test purpose

The test purpose is to verify that the NCR-MT can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 6.12.4	Method of test

#### 6.12.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.9A.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.30),

-	OTA REFSENS conformance test directions (D.31).

#### 6.12.4.2	Procedure

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the NCR under test.

5)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 6.12.5.

6)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in clause 6.12.5.

7)	Measure the throughput.

8)	Repeat steps 3 to 7 for all OTA REFSENS conformance test directions of the NCR (D.31), and supported polarizations.

### 6.12.5	Test requirements

For NCR type 2-O of WA class, the test requirement for OTA reference sensitivity requirement is defined in TS 38.176-2 [21], clause 7.3.5.

For NCR type 2-O of LA class, the test requirement for OTA reference sensitivity requirement is defined in TS 38.521-2 [20], clause 7.3.2.5.

This test requirement applies at MT RIB only.

## 6.13	OTA maximum input level

### 6.13.1	Definition and applicability

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

This requirement applies at NCR-MT RIB only.

### 6.13.2	Minimum requirement

For NCR type 2-O of LA class, the maximum input power is specified in TS 38.101-2 [14], clause 7.4.

### 6.13.3	Test purpose

Maximum input level tests ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

### 6.13.4	Method of test

#### 6.13.4.1	Initial conditions

Test environment: Normal: see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9A.1.

Directions to be tested: OTA REFSENS receiver target reference direction (D.30).

#### 6.13.4.2	Procedure

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the NCR under test.

5)	Set the test signal mean power so that the calibrated radiated power at the NCR Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in clause 6.13.5.

6)	Measure the throughput.

### 6.13.5	Test requirements

For NCR type 2-O of LA class, the test requirement for OTA maximum input level is defined in TS 38.521-2 [20], clause 7.4.5.

This test requirement applies at NCR-MT RIB only.

NOTE:	According to TS 38.521-2 [20], the minimum conformance requirements in OTA maximum input level test cases are not testable due to maximum input level unachievable in IFF OTA test setup.

## 6.14	OTA adjacent channel selectivity

### 6.14.1	Definition and applicability

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

This requirement applies at NCR-MT RIB only.

### 6.14.2	Minimum requirement

For NCR type 2-O of WA class, the ACS requirement is specified in TS 38.174 [19], clause 10.5.1.4.

For NCR type 2-O of LA class, the ACS requirement is specified in TS 38.101-2 [14], clause 7.5.

### 6.14.3	Test purpose

Adjacent channel selectivity tests the ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

### 6.14.4	Method of test

#### 6.14.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.9A.1.

NCR RF Bandwidth edge position to be tested for multi-carrier:

-	MRFBW in single-band operation, see clause 4.9A.1;

Directions to be tested:

-	For NCR type 2-O, OTA REFSENS receiver target reference direction (D.30).

#### 6.14.4.2	Procedure

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Align the NCR so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the test signal mean power so that the calibrated radiated power at the NCR Antenna Array coordinate system reference point is as follows:

a)

For NCR-MT type 2-O, set the signal generator for the wanted signal to transmit as specified in clause 6.14.5.

b)

For NCR-MT type 2-O, set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in clause 6.14.5.

7)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

### 6.14.5	Test requirements

For NCR type 2-O of WA class, the test requirement for OTA ACS is defined in TS 38.176-2 [21], clause 7.5.1.5.3.

For NCR type 2-O of LA class, the test requirement for OTA ACS is defined in TS 38.521-2 [20], clause 7.5.5.

This test requirement applies at NCR-MT RIB only.

NOTE:	According to TS 38.521-2 [20], multiple cases of the OTA ACS requirement for NCR type 2-O of LA class cannot be tested due to testability issue. For more details, refer to TS 38.521-2 [20], clause 7.5.5.

## 6.15	OTA blocking characteristics

### 6.15.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

This requirement applies at NCR-MT RIB only.

NOTE:	According to TS 38.521-2 [20], multiple cases of the OTA blocking requirement for NCR type 2-O of LA class cannot be tested due to testability issue. For more details, refer to TS 38.521-2 [20], clause 7.6.2.5.

### 6.15.2	Minimum requirement

For WA class NCR type 2-O, the OTA blocking requirement is specified in TS 38.174 [19], clause 10.5.2.4.

For LA class NCR type 2-O, the OTA blocking requirement is specified in TS 38.101-2 [14], clause 7.6.

### 6.15.3	Test purpose

The test purpose is to verify the ability of the NCR-MT receiver to withstand high-levels of in-band interference from unwanted signals at specified frequency offsets without undue degradation of its sensitivity.

### 6.15.4	Method of test

#### 6.15.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.9A.1.

NCR RF Bandwidth edge position to be tested for multi-carrier:

-	MRFBW in single-band operation, see clause 4.9A.1;

Directions to be tested:

-	For NCR type 2-O, OTA REFSENS receiver target reference direction (D.30).

#### 6.15.4.2	Procedure for general blocking

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Align the NCR so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the test signal mean power so that the calibrated radiated power at the NCR Antenna Array coordinate system reference point is as follows:

a)

For NCR-MT type 2-O, set the signal generator for the wanted signal to transmit as specified in clause 6.15.5.

b)

For NCR-MT type 2-O, set the signal generator for the interfering signal at the in-band blocking frequency of the wanted signal to transmit as specified in clause 6.15.5.

7)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

#### 6.15.4.3	Procedure for out of band blocking

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Align the NCR so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the test signal mean power so that the calibrated radiated power at the NCR Antenna Array coordinate system reference point is as follows:

a)

For NCR-MT type 2-O, set the signal generator for the wanted signal to transmit as specified in clause 6.15.5.

b)

For NCR-MT type 2-O, set the signal generator for the interfering signal at the out of band blocking frequency of the wanted signal to transmit as specified in clause 6.15.5. The interfering signal shall be swept within the frequency range specified in clause 6.15.5 with the step size specified in table 6.15.4.3-1.

Table 6.15.4.3-1: Interferer signal step size

| Frequency range (MHz) | Minimum supported NCR-MT channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

7)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

### 6.15.5	Test requirements

For NCR type 2-O of WA class, the test requirement for OTA blocking is defined in TS 38.176-2 [21], clause 7.5.2.5.5.

For NCR type 2-O of LA class, the test requirement for OTA blocking is defined in TS 38.521-2 [20], clause 7.6.2.5.

This test requirement applies at NCR-MT RIB only.

## 6.16	OTA receiver spurious emissions

### 6.16.1	Definition and applicability

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver. The receiver spurious emissions power level is measured as TRP.

This requirement applies at NCR-MT RIB only.

### 6.16.2	Minimum requirement

For NCR type 2-O of WA class, the OTA receiver spurious emission requirement is specified in TS 38.174 [19], clause 10.7.3.2.

For NCR type 2-O of LA class, the OTA receiver spurious emission requirement is specified in TS 38.101-2 [14], clause 7.9.

### 6.16.3	Test purpose

Test verifies whether receiver spurious emissions meet the requirements described in clause 6.16.5. Excess receiver spurious emissions increase the interference to other systems.

### 6.16.4	Method of test

#### 6.16.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see clause 4.9A.1:

-	For FR2:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band operation, see clause 4.9A.1:

-	For FR2:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

#### 6.16.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.16.5.

5)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

6)	Set the TDD NCR to receive only.

7)	Orient the positioner (and NCR) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

8)	Measure the emission at the specified frequencies with specified measurement bandwidth

9)	Repeat step 7-10 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

NOTE 1:	The TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep or the TRP measurement grid sweep may be done in any order

10)	Calculate TRP at each specified frequency using the directional measurements.

### 6.16.5	Test requirements

For NCR type 2-O of WA class, the test requirement for OTA receiver spurious emission is defined in TS 38.176-2 [21], clause 7.7.5.

For NCR type 2-O of LA class, the test requirement for OTA receiver spurious emission is defined in TS 38.521-2 [20], clause 7.9.5.

This test requirement applies at NCR-MT RIB only.

## 6.17	OTA spatial emission

### 6.17.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.17.2	Minimum requirement

The minimum requirement for NCR type 1-H operation is defined in TS 38.106 [2], clause 7.18.2.

### 6.17.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

### 6.17.4	Method of test

#### 6.17.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level with the corresponding BW.

#### 6.17.4.2	Procedure

When calibrated and operated within the guidance of 3GPP TR 37.941 [25] the measurement methods are applicable and selected depending on availability at the test facility.

The test range shall be calibrated according to calibration method described in TR 37.941, clause 8.3.

1. Place the NCR at the positioner, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the coordinate system used by the test system.

3. The measurement device characteristics shall be: Detection mode set to True RMS.

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2. The configured EIRP for each of the test beams shall be in direct relation to the intended use of the NCR.

5. Orient the positioner (and the repeater) to the angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex J, clause J.3.

6. Configure test beam(s) and test equipment for measuring test beam k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRP as a sum of two orthogonal polarizations:

EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denote two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beams.

8b. Repeat step 5 and 7 for all M and N positioner angles

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beams.

10. Calculate EEIRPi from measured EIRPkθn,φm for bins i=1..7, as described in Annex J.

### 6.17.5	Test requirements

#### 6.17.5.1	Requirement for NCR type 1-H

For NCR type 1-H operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, the power measured EEIRP shall not exceed the limits specified in table 6.17.5.1-1 for mechanical down-tilt angle equal to zero.NOTE: OTA spatial emissions in table 6.17.5.1-1 are defined with respect to the horizon with angles above the horizon positive and below the horizon negative The reference NCR coordinate system in sub-clause 4.12 system is defined with respect to the BS enclosure with angles above the boresight negative and below the boresight positive. If the NCR is aligned with the horizon (mechanical tilt is zero) then the coordinate systems θ  sign is inverted.

Table 6.17.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| NOTE 1: The requirement shall apply to all supported mechanical tilts. |  |  |

Figure 6.17.5.1-1: Definitions of θHLi and θHHi angles.

# 7	Radiated performance requirements

## 7.1	NCR-MT performance requirements

### 7.1.1	General

#### 7.1.1.1	Scope and definitions

Radiated performance requirements specify the ability of the NCR-MT type 2-O to correctly demodulate signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the NCR-MT are specified for the fixed reference channels defined in annex H and the propagation conditions in annex I. The requirements only apply to those FRCs that are supported by the NCR-MT.

The radiated performance requirements for NCR-MT type 2-O are limited to two OTA demodulations branches as described in clause 7.1.1.2. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the NCR-MT, with the required SNR applied separately per polarization.

NOTE: 	NCR-MT can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector (for NCR-MT type 1-C).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector (for NCR-MT type 1-C).

#### 7.1.1.2	OTA demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the NCR-MT type 2-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the NCR-MT type 2-O does not use polarization diversity then radiated performance requirements can only be tested for a single demodulation branch (i.e. 1RX test setup).

### 7.1.2	Demodulation performance requirements

#### 7.1.2.1	General

##### 7.1.2.1.1	Applicability rule for NCR-MT

7.1.2.1.1.1	General

Unless otherwise stated, for an NCR-MT declared to support more than 2 demodulation branches (for NCR-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and demodulation branches is up to NCR-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

7.1.2.1.1.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1).

7.1.2.1.1.3	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

#### 7.1.2.2	Performance requirements for PDSCH

##### 7.1.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex H.

Which specific test(s) are applicable to NCR-MT is based on the test applicability rules defined in clause 7.1.2.1.1.

##### 7.1.2.2.2	Minimum requirements

For NCR-MT type 2-O, the minimum requirement is in TS 38.106 [2], clause 9.3.1.1.

##### 7.1.2.2.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

##### 7.1.2.2.4	Method of test

7.1.2.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

7.1.2.2.4.2	Test procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.6.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.6. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex I, and according to additional test parameters listed in table 7.1.2.2.4.2-1.

Table: 7.1.2.2.4.2-1 Test parameters for testing PDSCH

| Parameter |  | NCR-MT type 2-O |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 for Table 7.1.2.2.5.2-1 1 for Table 7.1.2.2.5.2-2 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex I.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.2.2.5.1 for NCR-MT type 2-O respectively, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.2.2.4.2-2.

Table 7.1.2.2.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 6.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the throughput and BLER.

##### 7.1.2.2.5	Test requirements

7.1.2.2.5.1	Test requirement for NCR-MT type 2-O

The throughput or BLER shall be equal to or larger than the fraction of maximum throughput of BLER for the FRCs stated in Tables 7.1.2.2.5.2-1 and 7.1.2.2.5.2-2 at the given SNR with the test parameters stated in Table 7.1.2.2.4.2-1.

Table 7.1.2.2.5.2-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex H) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex I) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR2-NCR.1.1-1 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | -0.1 |

Table 7.1.2.2.5.2-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex H) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex I) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR2-NCR.1.1-1 | 100/120 | TDLA30-75 | 2x2, ULA Low | 1 | 3.6 |

#### 7.1.2.3	Demodulation performance requirements for PDCCH

##### 7.1.2.3.1	Definition and applicability

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

Which specific test(s) are applicable to NCR-MT is based on the test applicability rules defined in clause 7.1.2.1.1.

##### 7.1.2.3.2	Minimum requirement

For NCR-MT type 2-O, the minimum requirement is in TS 38.106 [2], clause 9.3.1.2.

##### 7.1.2.3.3	Test purpose

The test shall verify the receiver's ability to detect the Downlink Scheduling Grant (Pm-dsg) under multipath fading propagation conditions for a given SNR.

##### 7.1.2.3.4	Method of test

7.1.2.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

7.1.2.3.4.2	Test procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.6.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.6. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex H, and according to additional test parameters listed in table 7.1.2.3.4.2-1.

Table: 7.1.2.3.4.2-1 Test parameters for testing PDCCH

| Parameter | NCR-MT type 2-O |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note) | 3D1S1U, S=10D:2G:2U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 3 for test with aggregation level 2, 8 2 for test with aggregation level 4 |
| REG bundle size | 2 for test with aggregation level 2, 8 6 for test with aggregation level 4 |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Note: The same requirements are applicable to TDD with different UL-DL patterns. |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex I.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.2.3.5.1 for NCR-MT type 2-O respectively, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.2.3.4.2-2.

Table 7.1.2.3.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 2: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the miss-detection of the Downlink Scheduling Grant (Pm-dsg).

##### 7.1.2.3.5	Test requirements

7.1.2.3.5.1	Test requirement for NCR-MT type 2-O

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 7.1.2.3.5.1-1 at the given SNR with the test parameters stated in Table 7.1.2.3.4.2-1.

Table 7.1.2.3.5.2-1: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex H) | Propagation conditions (Annex I) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | 60 | 1 | 2 | M-FR2-NCR. 1.2-1 | TDLA30-75 | 1x2, ULA Low | 1 | 8.1 |
| 1-2 | 100/120 | 60 | 1 | 4 | M-FR2-NCR. 1.2-2 | TDLA30-75 | 1x2, ULA Low | 1 | 4.6 |
| 1-3 | 100/120 | 60 | 1 | 8 | M-FR2-NCR. 1.2-3 | TDLA30-75 | 2x2, ULA Low | 1 | 1.9 |

7.1.3	CSI reporting requirements

#### 7.1.3.1	General

##### 7.1.3.1.1	Applicability of requirements

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported.

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

##### 7.1.3.1.2	Common test parameters

Parameters specified in Table 7.1.3.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 7.1.3.1.2-1: Test parameters for CSI test cases

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Duplex mode |  |  |  | TDD |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 120 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | 66 |
| Active DL BWP index |  |  |  | 1 |
| PDSCH configuration |  | Mapping type |  | Type A |
|  |  | k0 |  | 0 |
|  |  | Starting symbol (S) |  | 2 |
|  |  | Length (L) |  | 12 |
|  |  | PDSCH aggregation factor |  | 1 |
|  |  | PRB bundling type |  | Static |
|  |  | PRB bundling size |  | 2 |
|  |  | Resource allocation type |  | type 0 |
|  |  | RBG size |  | Config 2 |
|  |  | VRB-to-PRB mapping type |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  | Type 1 |
|  |  | Number of additional DMRS |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  |  | DMRS ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  | 2 |
|  |  | Time density (LPT-RS) |  | 1 |
|  |  | Resource Element Offset |  | 2 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |

#### 7.1.3.2	Reporting of Channel Quality Indicator (CQI)

##### 7.1.3.2.1	Definition and applicability

The performance requirement of CSI reporting is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [15]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

##### 7.1.3.2.2	Minimum requirement

The minimum requirement for NCR-MT type 2-O is in TS 38.106 [2] clause 9.4.1.

##### 7.1.3.2.3	Test purpose

The test shall verify the receiver's ability to report correct median CQI and expected BLER performance under AWGN conditions.

##### 7.1.3.2.4	Method of test

###### 7.1.3.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

###### 7.1.3.2.4.2	Procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.7.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex H, and according to additional test parameters listed in table 7.1.3.2.4.2-1.

Table 7.1.3.2.4.2-1: Test parameters for testing CQI reporting requirements for FR2

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex I.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex I.3.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/4 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.75 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | M-FR2-NCR.1.3-1 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS and/or other unspecified test parameters with respect to TS 38.101-4 [16] are left up to test implementation, if transmitted or needed. Note 3: If the NCR-MT reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |  |  |  |

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.3.2.5.1, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.3.2.4.2-2.

Table 7.2.3.2.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 2: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the median CQI and the BLER at (median CQI +1) and (median CQI -1).

##### 7.1.3.2.5	Test requirement

###### 7.1.3.2.5.1	Test requirement for NCR type 2-O

For the parameters specified in Table 7.1.3.2.4.2-1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.
Annex A (normative):
Repeater stimulus signals

# A.1	Repeater stimulus signal 1

This repeater stimulus signal shall be used for tests on:

-	Uplink maximum output power

-	Uplink operating band unwanted emissions

-	Uplink spurious emissions

Two uplink fixed reference channels for performance requirements (16QAM ¾) for FDD according to the TS38.141-2 [6], table A.4-5, channel reference G-FR2-A4-3 of 50 MHz bandwidth generated on separate centre frequencies with equal power and combined with a time difference of 266,7 us (4 OFDM symbols).

The PUSCH data payload shall contain only zeroes (0000 0000)

Each reference channel shall be subjected to time windowing and filtering so that it fulfils the spectral purity requirements defined in A.3

# A.2	Repeater stimulus signal 2

This repeater stimulus signal shall be used for tests on:

-	Downlink operating band unwanted emissions

-	Downlink spurious emissions

Two NR-FR2-TM1.1 channels according to the TS38.141-2 [6] of 50 MHz bandwidth generated on separate centre frequencies with equal power and combined with a time difference of 1400 us (21 OFDM symbols).

Each NR-FR2-TM1.1 channel shall be subjected to time windowing and filtering so that it fulfils the spectral purity requirements defined in annex A.3.

# A.3	Repeater stimulus signal spectral purity requirements

The reference channels or test models constituting the repeater stimulus signal shall fulfil the spectral purity requirements defined by tables A.3-1, A.3-2 and A.3-3, where;

Table A.3-1: Repeater stimulus signal spectral purity requirements for frequency range 24.25 GHz ~ 33.4 GHz

| Center frequency of measurement filter | Minimum requirements | Measurement filter type and filter bandwidth | Integration bandwidth |
| --- | --- | --- | --- |
| 0.5 MHz outside from channel edge | 28.7 dBc (Note 1) | Square (1 MHz) | 1 MHz |
| Center of adjacent channel frequency with same nominal channel BW | 28.7 dBc (Note 2) | Square (nominal channel BW) | Nominal channel BW |
| NOTE 1: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of 1 MHz. NOTE 2: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of nominal carrier BW |  |  |  |

Table A.3-2: Repeater stimulus signal spectral purity requirements for frequency range 37.0 GHz ~ 43.5 GHz

| Center frequency of measurement filter | Minimum requirements | Measurement filter type and filter bandwidth | Integration bandwidth |
| --- | --- | --- | --- |
| 0.5 MHz outside from channel edge | 26.4 dBc (Note 1) | Square (1 MHz) | 1 MHz |
| Center of adjacent channel frequency with same nominal channel BW | 26.4 dBc (Note 2) | Square (nominal channel BW) | Nominal channel BW |
| NOTE 1: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of 1 MHz. NOTE 2: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of nominal carrier BW |  |  |  |

Table A.3-3: Repeater stimulus signal spectral purity requirements for frequency range 43.5 GHz ~ 48.2 GHz

| Center frequency of measurement filter | Minimum requirements | Measurement filter type and filter bandwidth | Integration bandwidth |
| --- | --- | --- | --- |
| 0.5 MHz outside from channel edge | 26.2 dBc (Note 1) | Square (1 MHz) | 1 MHz |
| Center of adjacent channel frequency with same nominal channel BW | 26.2 dBc (Note 2) | Square (nominal channel BW) | Nominal channel BW |
| NOTE 1: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of 1 MHz. NOTE 2: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of nominal carrier BW |  |  |  |

Annex B (normative):
Environmental requirements for the Repeater equipment

# B.1	General

For each test in the present document, the environmental conditions under which the repeater is to be tested are defined.

# B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table B.2.1.

Table B.2-1: Limits of conditions for normal test environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15 C | 30 C |
| Relative humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

NOTE:	This may, for instance, be the case for measurements of radiated emissions performed on an open field test site.

# B.3	Extreme test environment

The manufacturer shall declare one of the following:

1)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-3 [7];

2)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-4 [8];

3)	The equipment that does not comply with the mentioned classes, the relevant classes from IEC 60 721 [9] documentation for temperature, humidity and vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions is not tested in the present document. These may be stated and tested separately.

## B.3.1	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-1 [10].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-2 [11].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer's declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60 068-2-6 [12]. Other environmental conditions shall be within the ranges specified in annex A.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

# B.6	Measurement of test environments

The measurement accuracy of the repeater test environments defined in annex A shall be:

Pressure:	5 kPa

Temperature:	2 degrees

Relative humidity:	5 %

DC voltage:	1.0 %

AC voltage:	1.5 %

Vibration:	10 %

Vibration frequency:	0.1 Hz

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

Annex C (informative):
Test tolerances and derivation of test requirements

# C.1	Measurement of radiated characteristics

Table C.1-1: Derivation of test requirements (radiated tests)

| Test | Minimum requirement in TS 38.106 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | See TS 38.106 [2], clause 7.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA repeater output power (TRP) |  | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA frequency stability | See TS 38.106 [2], clause 7.3 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5 OTA out of band gain | See TS 38.106 [2], clause 7.4 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA ACLR | See TS 38.106 [2], clause 7.5.2 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 43.5GHz < f ≦ 48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.6.3 OTA operating band unwanted emission | See TS 38.106 [2], clause 7.5.3 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.6.4 OTA spurious emissions | See TS 38.106 [2], clause 7.5.4 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7 OTA EVM | See TS 38.106 [2], clause 7.6 | 1.4%, QPSK, 16QAM 1.5%, 64QAM NOTE 2 1.8%, 256QAM Note 3 | Test requirement limit shifted by RSS of minimum requirement and stimulus signal EVM. Analyser error added to requirement limit. Formula: EVM limit + TT |
| 6.7 OTA input intermodulation | See TS 38.106 [2], clause 7.7 | 3.9 dB, 24.25 GHz < f ≤ 29.5 GHz 3.9 dB, 37 GHz < f ≤ 43.5 GHz 5.4 dB, 43.5 GHz < f ≤ 48.2 GHz | Formula: Minimum Requirement + TT |
| 6.9 OTA ACRR | See TS 38.106 [2], clause 7.8 | 2.7 dB (24.25 – 29.5 GHz) 2.7 dB (37 – 43.5 GHz) 2.9 dB (43.5 GHz < f ≤ 48.2 GHz) | Formula: Minimum Requirement – TT |
| 6.10.1 OTA transmitter OFF power | See TS 38.106 [2], clause 7.9.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.10 OTA output power dynamics | See TS 38.106 [2], clause 7.10.2 | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz | Formula: Minimum Requirement + TT |
| 6.11 OTA transmit signal quality | See TS 38.106 [2], clause 7.11.2 and 7.11.3 | ± 0.01 ppm | Formula: Minimum Requirement + TT |
| 6.12 OTA reference sensitivity | See TS 38.106 [2], clause 7.11.2 and 7.13.2 | ±3.3 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.3 dB, 37 GHz < f ≤ 43.5 GHz | Formula: Minimum Requirement + TT |
| 6.13 OTA maximum input level | See TS 38.106 [2], clause 7.14.2 | 0dB | Formula: Minimum Requirement + TT |
| 6.14 OTA adjacent channel selectivity | See TS 38.106 [2], clause 7.15.2 | ±4.2 dB, 24.25 GHz < f ≤ 29.5 GHz ±4.2 dB, 37 GHz < f ≤ 43.5 GHz | Formula: Minimum Requirement + TT |
| 6.15 OTA blocking characteristics | See TS 38.106 [2], clause 7.16.2 | In-band blocking: ±4.2 dB, 24.25 GHz < f ≤ 29.5 GHz ±4.2 dB, 37 GHz < f ≤ 43.5 GHz Out of band blocking: ±4.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±4.4 dB, 37 GHz < f ≤ 43.5 GHz | Formula: Minimum Requirement + TT |
| 6.16 OTA spurious emissions | See TS 38.106 [2], clause 7.17.2 | ± 2.5 dB, 30 MHz ≤ f ≤ 6 GHz ±2.9 dB, 6 GHz < f ≤ 40 GHz ±5.2 dB, 40 GHz < f ≤ 60 GHz | Formula: Minimum Requirement + TT |
| 6.10.2 OTA transient period | See TS 38.106 [2], clause 7.9.3 | N/A |  |
| NOTE 1: TT values are applicable for normal condition unless otherwise stated. NOTE 2: Support of 64QAM is based on the declaration NOTE 3: Support of 256QAM is based on the declaration. |  |  |  |

Annex D (normative):
Calibration

For Repeater type 2-O and NCR type 2-O, OTA test requirements specific and OTA measurement chamber specific calibration (and measurement) procedures were captured in TR 37.941 [13] for the following requirements sets:

-	TX directional requirements

-	within passband and out-of-passband TRP requirements.

Annex E (informative):
OTA measurement system set-up

For the measurement setup for NCR-MT type 2-O, the existing measurement setup in Annex E.1 of TS 38.176-2 [21] is applicable.

# E.1	OTA output power EIRP, OTA Frequency stability, OTA Error Vector Magnitude and OTA Transmit ON/OFF power

Figure E.1-1: Measuring system set-up for maximum EIRP testing

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	The OTA chamber shown in figure E.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, etc.)

Note 3:	UL/DL timing can be provided to the repeater.

Note 4:      This is applicable for both repeater and NCR

Note 5:      For NCR-MT conformance testing, testing signal for NCR-Fwd input should be terminated.

# E.2	Out of band gain

Figure E.2-1: Measuring system set-up for out of band gain

Note 1:	That repeater is a bi-directional device. The signal generator may need protection.

Note 2:	The OTA chamber shown in figure E.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, etc.)

Note 3:	It is possible to keep the repeater static but move the measurement probes or use multiple probes.

Note 4:      This is applicable for both repeater and NCR.

# E.3	Unwanted emission: Operating band unwanted emission and ACLR

Figure E.3-1: Measuring system set-up for unwanted emission: ACLR, Operating band unwanted emission and spurious emission requirement

Note 1:	That repeater is a bi-directional device. The signal generator may need protection.

Note 2:	The OTA chamber shown in figure E.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, etc.)

Note 3:	It is possible to keep the repeater static but move the measurement probes or use multiple probes.

Note 4:	UL/DL timing can be provided to the repeater.

Note 5:      This is applicable for both repeater and NCR.

Note 6:      For NCR-MT conformance testing, testing signal for NCR-Fwd input should be terminated.

# E.4	Input intermodulation

Figure E.4-1: Measuring system set-up for input intermodulation.

Note 1:	That repeater is a bi-directional device. The signal generator may need protection.

Note 2:	The OTA chamber shown in figure E.4-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, etc.)

Note 3:	It is possible to keep the repeater static but move the measurement probes or use multiple probes.

Note 4:	This is applicable for both repeater and NCR.

# E.5	Adjacent Channel Rejection Ratio

Figure E.5-1: Measuring system set-up for Adjacent Channel Rejection Ratio

Note 1:	That repeater is a bi-directional device. The signal generator may need protection.

Note 2:	The OTA chamber shown in figure E.5-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, etc.)

Note 3:	It is possible to keep the repeater static but move the measurement probes or use multiple probes.

Note 4:	UL/DL timing can be provided to the repeater.

Note 5:	This is applicable for both repeater and NCR.

## E.6 	NCR-MT type 2-O PDSCH and PDCCH testing

Figure E.6-1: Functional set-up for PDCCH, PDSCH performance requirements with Rx diversity (2 Rx case shown)

The OTA chamber shown in Figure E.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (e.g. far field anechoic chamber, CATR, etc.).

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

## E.7	NCR-MT type 2-O CQI testing

Figure E.7-1: Functional set-up for PUSCH, PRACH, CSI reporting performance requirements in static AWGN channel with Rx diversity (2 Rx case shown)

The OTA chamber shown in Figure E.7-1 is intended to be generic and can be replaced with any suitable OTA chamber (e.g. far field anechoic chamber, CATR, etc.).

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

Annex F (normative):
In-channel TX tests

# F.1	General

The in-channel TX test enables the measurement of all relevant parameters that describe the in-channel quality of the output signal of the repeater under test in a single measurement process.

The parameters describing the in-channel quality of a transmitter, however, are not necessarily independent. The algorithm chosen for description inside this annex places particular emphasis on the exclusion of all interdependencies among the parameters.

# F.2	Basic principles

The process is based on the comparison of the actual output signal of the TX under test, received by an ideal receiver, with an ideal signal, that is generated by the measuring equipment and represents an ideal error free received signal. All signals are represented as equivalent (generally complex) baseband signals.

The description below uses numbers and illustrations as examples only. These numbers are taken from a TDD frame structure with normal CP length, 120 kHz SCS and a transmission bandwidth configuration of 400 MHz (NRB = 264). The application of the text below, however, is not restricted to this parameter set.

## F.2.1	Output signal of the TX under test

The output signal of the TX under test is acquired by the measuring equipment and stored for further processing. It is sampled at a sampling rate which is the product of the SCS and the FFT size, and it is named .

For FR2, FFT size is determined by the transmission bandwidth in table 6.6.3.5-2 for 60 kHz SCS, and table 6.6.3.5-3 in [5] for 120 kHz SCS. In the time domain it comprises at least 10 ms. It is modelled as a signal with the following parameters:

-	demodulated data content,

-	carrier frequency,

-	amplitude and phase for each subcarrier.

For the example in the annex, the FFT size is 4096 based on table 6.6.3.5-3 in [5]. The sampling rate of 491.52 Msps is the product of the FFT size and SCS.

## F.2.2	Ideal signal

Two types of ideal signals are defined:

The first ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

-	demodulated data content,

-	nominal carrier frequency,

-	nominal amplitude and phase for each subcarrier.

It is represented as a sequence of samples at the sampling rate determined from annex F.2.1 in the time domain. The structure of the signal is described in the test models.

The second ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters for FR1 and FR2:

-	nominal demodulation reference signal and nominal PT-RS if present (all other modulation symbols are set to 0 V),

-	nominal carrier frequency,

-	nominal amplitude and phase for each applicable subcarrier,

-	nominal timing.

It is represented as a sequence of samples at the sampling rate determined from annex F.2.1 in the time domain.

## F.2.3	Measurement results

The measurement results, achieved by the in-channel TX test are the following:

-	Carrier frequency stability.

-	EVM.

Other side results are: residual amplitude- and phase response of the TX chain after equalisation.

## F.2.4	Measurement points

The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, as depicted in figure G.2.4-1. The FFT window of FFT size samples out of (FFT size + cyclic prefix length) samples in the time domain is selected in the "Remove CP" box.

For FR2, FFT size and the cyclic prefix length is determined from table 6.6.3.5-2 for 60 kHz SCS, and table 6.6.3.5-3 in [5]for 120 kHz SCS.

In one subframe, there are two symbols with the length of the cyclic prefix larger than the values listed in tables 6.6.3.5-2 and table 6.6.3.5-3 in [5]. Table F.2.4-1 lists the slot number and the symbol number and the formula how to compute the length of cyclic prefix for those two symbols according to the sampling rate.

Table F.2.4-1: Slot number and symbol number identifying the longer CP length for normal CP

| SCS (kHz) | Frequency Range | # slots in subframe | Symbol # and slot # with longer CP | Longer CP length |
| --- | --- | --- | --- | --- |
| 15 | FR1 | 1 | (symbol 0, slot 0) (symbol 7, slot 0) | CP length + FFT size / 128 |
| 30 |  | 2 | (symbol 0, slot 0) (symbol 0, slot 1) | CP length + FFT size / 64 |
| 60 |  | 4 | (symbol 0, slot 0) (symbol 0, slot 2) | CP length + FFT size / 32 |
| 60 | FR2 | 4 | (symbol 0, slot 0) (symbol 0, slot 2) | CP length + FFT size / 32 |
| 120 |  | 8 | (symbol 0, slot 0) (symbol 0, slot 4) | CP length + FFT size / 16 |

For the example used in the annex, the "Remove CP" box selects 4096 samples out of 4384 samples. Symbol 0 of slot 0 and slot 4 has 256 more samples in the cyclic prefix than the other symbols (the longer CP length = 544).

Figure F.2.4-1: Reference point for FR2 EVM measurements

# F.3	Pre-FFT minimization process

Sample timing, carrier frequency in  are varied in order to minimise the difference between  and , after the amplitude ratio of  and  has been scaled. Best fit (minimum difference) is achieved when the RMS difference value between  and  is an absolute minimum.

The carrier frequency variation is the measurement result: carrier frequency error.

From the acquired samples, one value of carrier frequency error can be derived.

NOTE 1:	The minimisation process, to derive the RF error can be supported by post-FFT operations. However the minimisation process defined in the pre-FFT domain comprises all acquired samples (i.e. it does not exclude the samples in between the FFT widths and it does not exclude the bandwidth outside the transmission bandwidth configuration).

NOTE 2:	The algorithm would allow to derive carrier frequency error and sample frequency error of the TX under test separately. However there are no requirements for sample frequency error. Hence the algorithm models the RF and the sample frequency commonly (not independently). It returns one error and does not distinguish between both.

After this process the samples  are called .

# F.4	Timing of the FFT window

The FFT window length is FFT size samples per OFDM symbol. For TDD, the number of FFTs performed is the number of downlink symbols in the measurement interval.

The position in time for the FFT shall be determined.

For the example used in the annex, the FFT window length is 4096 samples per OFDM symbol. 832 FFTs (i.e., 3,407,872 samples) cover less than the acquired number of samples (i.e. 3,651,584 samples in 10 ms). There are 816 symbols with 4384 samples and 16 symbols with 4640 samples.

In an ideal signal, the FFT may start at any instant within the cyclic prefix without causing an error. The TX filter, however, reduces the window. The EVM requirements shall be met within a window W < CP. There are three different instants for FFT:

-	Centre of the reduced window, called ,

-	, and

-	.

The value of EVM window length W is obtained from the transmission bandwidth and table 6.6.3.5-2 for 60 kHz SCS, and table 6.6.3.5-3 in TS 38.104 [5] for 120 kHz SCS.

The repeater shall transmit a signal according to the test models intended for EVM. The demodulation reference signal of the second ideal signal shall be used to find the centre of the FFT window.

The timing of the measured signal is determined in the pre FFT domain as follows, using  and :

1.	The measured signal is delay spread by the TX filter. Hence the distinct borders between the OFDM symbols and between data and CP are also spread and the timing is not obvious.

2.	In the ideal signal , the timing is known.

Correlation between bullet (1) and (2) will result in a correlation peak. The meaning of the correlation peak is approximately the "impulse response" of the TX filter.

3.	The meaning of "impulse response" assumes that the autocorrelation of the ideal signal  is a Dirac peak and that the correlation between the ideal signal  and the data in the measured signal is 0. The correlation peak, (the highest, or in case of more than one highest, the earliest) indicates the timing in the measured signal.

The number of samples, used for FFT is reduced compared to . This subset of samples is called .

From the acquired samples one timing can be derived.

The timing of the centre  is determined according to the cyclic prefix length of the OFDM symbols. For normal CP, there are two values for  in a 1 ms period:

-	 = length of cyclic prefix / 2,

-	 = Longer CP length - length of cyclic prefix / 2,

Where the length of cyclic prefix is obtained from table 6.6.3.5-2 for 60 kHz SCS, and table 6.6.3.5-3 in TS 38.104 [5] for 120 kHz SCS, and the longer CP length is obtained from table F.2.4-1.

As per the example values.

-	 within the CP of length 288 for most OFDM symbols in 1 ms,

-	 (= 544 – 144) within the CP of length 544 for OFDM symbol 0 of slot 0 and slot 4.

# F.5	Resource element TX power

Perform FFT on  with the FFT window timing .

The result is called . The RE TX (RETP) power is then defined as:

RETP=Z't,f2SCS

where SCS is the subcarrier spacing in Hz.

From RETP, the OFDM symbol TX power (OSTP) is derived as follows:

OSTP=1NsymRETP

Where the summation accumulates  values of all Nsym OFDM symbols that carry PDSCH and not containing PDCCH, RS or SSB within a slot.

.

From the acquired samples  values for each OSTP can be obtained and averaged where  is the number of slots with downlink symbols in a 10 ms measurement interval and is computed according to the values in table 4.9.2.2-1 in [5].

For the example used in the annex,  and .

# F.6	Post-FFT equalisation

Perform  FFTs on , one for each OFDM symbol within 10 ms measurement interval with the FFT window timing to produce an array of samples,  in the time axis t by FFT size in the frequency axis f.

For the example in the annex, 1120 FFTs are performed on . The result is an array of samples, 1120 in the time axis by 4096 in the frequency axis.

The equalizer coefficients  and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT ideal signal , for each demodulation reference signal, over 10 ms measurement interval. This process creates a set of complex ratios:

2.	Perform time averaging at each demodulation reference signal subcarrier of the complex ratios, the time-averaging length is 10 ms measurement interval. Prior to the averaging of the phases   an unwrap operation must be performed according to the following definition:

-	The unwrap operation corrects the radian phase angles of  by adding multiples of 2 * π when absolute phase jumps between consecutive time instances  are greater than or equal to the jump tolerance of π radians.

-	This process creates an average amplitude and phase for each demodulation reference signal subcarrier (i.e. every second subcarrier).

-	Where N is the number of demodulation reference signal time-domain locations  from  for each demodulation reference signal subcarrier f.

3.	The equalizer coefficients for amplitude and phase  and  at the demodulation reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged demodulation reference signal subcarriers. The moving average window size is 19 and averaging is over the DM-RS subcarriers in the allocated RBs. For DM-RS subcarriers at or near the edge of the channel, or when the number of available DM-RS subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size, the window size is reduced accordingly as per figure F.6-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier.

Figure F.6-1: Reference subcarrier smoothing in the frequency domain

a)	To account for the common phase error (CPE) experienced in millimetre wave frequencies, , in the estimated coefficients contain phase rotation due to the CPE, , in addition to the phase of the equalizer coefficient , that is:

For OFDM symbols where PT-RS does not exist,  can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE,, contained in, , estimation and compensation of the CPE needs to follow. is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time .

Estimate of the CPE, , at OFDM symbol time, , can then be obtained from using the PT-RS employing the expression:

In the above equation,  is the set of subcarriers where PT-RS are mapped,  where  is the set of OFDM symbols where PT-RS are mapped while  and  are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from  to remove influence of the CPE, and obtain estimate of the complex coefficient's phase:

(t)

# F.7	EVM

## F.7.0	General

For EVM create two sets of , according to the timing  and , using the equalizer coefficients from F.6.

The equivalent ideal samples are calculated from  (annex G.2.2) and are called .

The EVM is the difference between the ideal signal and the equalized measured signal.

Where:

-	T is the set of symbols with the considered modulation scheme being active within the slot,

-	 is the set of subcarriers within the resource blocks with the considered modulation scheme being active in symbol t,

-	 is the ideal signal reconstructed by the measurement equipment in accordance with relevant test models,

-	  is the equalized signal under test.

NOTE:	Although the basic unit of measurement is one slot, the equalizer is calculated over the entire 10 ms measurement interval to reduce the impact of noise in the reference signals.

## F.7.1	Averaged EVM

Let  be the number of slots with downlink symbols within a 10 ms measurement interval. For TDD, the averaging in the time domain can be calculated from  slots of different 10 ms measurement intervals and should have a minimum of  slots averaging length where  is the number of slots in a 10 ms measurement interval.

is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

Where  is the number of resource blocks with the considered modulation scheme in slot i.

The  is calculated, using the maximum of at the window W extremities. Thus  is calculated using  and  is calculated using (l and h, low and high; where low is the timing  and and high is the timing ).

In order to unite at least  slots, consider the minimum integer number of 10 ms measurement intervals, where  is determined by

And  for 60 kHz SCS and  for 120 kHz SCS.

Unite by RMS.

The resulting is compared against the limit.

Annex G (normative):
TRP measurement procedures

For TRP measurement procedures, refer to annex I in TS 38.141-2 [6].

Annex H (normative): 
NCR-MT Fixed Reference Channels

## H.1.1	Fixed Reference Channels for PDSCH performance requirements

The parameters for the reference measurement channels are specified in table H.1.1-1 for FR2 PDSCH performance requirements.

Table H.1.1-1: FRC parameters for FR2-1 PDSCH performance requirements, 1 transmission layer, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference Channel |  | M-FR2-NCR.1.1-1 |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

## H.1.2	Fixed Reference Channels for PDCCH performance requirements

The parameters for the reference measurement channels are specified in table H.1.2-2 for FR2-1 PDCCH performance requirements.

Table H.1.2-2: Fixed Reference Channels for FR2-1 PDCCH

| Reference channel | M-FR2-NCR.1.2-1 | M-FR2-NCR.1.2-2 | M-FR2-NCR.1.2-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 120 | 120 | 120 |
| CORESET frequency domain allocation | 60 | 60 | 60 |
| CORESET time domain allocation | 1 | 1 | 1 |
| Aggregation level | 2 | 4 | 8 |
| DCI Format | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) (bits) | 40 | 56 | 56 |

## H.1.3	Fixed Reference Channels for CQI reporting requirements

The parameters for the reference measurement channels are specified in Table H.1.3-1 for FR2-1 CQI reporting requirements with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [15] and with the MCS definition specified in clause 5.1.3 of TS 38.214 [15].

Table H.1.3-1: FRC for FR2-1 CQI reporting with CQI table 1 and MCS table 1

| Reference channel |  |  |  | M-FR2-NCR.1.3-1 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s for PDSCH |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |

Annex I (normative): 
Propagation conditions

## I.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

## I.1.1	NCR-MT Receiver with 2 Rx

For 1 port transmission the channel matrix is defined in the frequency domain by:

.

For 2 port transmission the channel matrix is defined in the frequency domain by:

.

## I.1.2	NCR-MT Receiver with 4Rx

For 1 port transmission the channel matrix is defined in the frequency domain by

.

For 2 port transmission the channel matrix is defined in the frequency domain by

.

## I.2	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

-	Different models are used for FR1 (410 MHz - 7.125GHz) and FR2-1 (24.25 GHz – 52.6 GHz).

I.2.1	Delay profiles

The delay profiles are simplified from the TR 38.901 [17] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in annex I.2.1.1 and I.2.1.2 can be used as such.

Step 1: Use the original TDL model from TR 38.901 [17].

Step 2: Re-order the taps in ascending delays.

Step 3: Perform delay scaling according to the procedure described in clause 7.7.3 in TR 38.901 [17].

Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows:

-	Find the weakest tap from all taps (both merged and unmerged taps are considered)

-	If there are two or more taps having the same value and are the weakest, select the tap with the smallest delay as the weakest tap.

-	When the weakest tap is the first delay tap, merge taps as follows:

-	Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

-	Remove the second delay tap.

-	When the weakest tap is the last delay tap, merge taps as follows:

-	Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

-	Remove the second-to-last tap.

-	Otherwise:

-	For each side of the weakest tap, identify the neighbour tap that has the smaller delay difference to the weakest tap.

-	When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

-	Select the neighbour tap that is weaker in power for merging.

-	Otherwise, select the neighbour tap that has smaller delay difference for merging.

-	To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap.

-	When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

-	When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

-	Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 20 ns  15 ns, 10 ns & 25 ns  20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed.

-	Repeat step 6 until the final number of taps is 12.

Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

Step 9: Re-normalize the highest tap to 0 dB.

NOTE 1:	Some values of the delay profile created by the simplification steps may differ from the values in tables I.2.1.1-2, I.2.1.1-3, I.2.1.1-4, and I.2.1.2-2 for the corresponding model.

NOTE 2:	For Step 5 and Step 6, the power values are expressed in the linear domain using 6 digits of precision. The operations are in the linear domain.

#### I.2.1.1	Delay profiles for FR2-1

The delay profiles for FR2-1 are specified in I.2.1.1-1 and the tapped delay line models are specified in table I.2.1.1-2.

Table I.2.1.1-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |

Table I.2.1.1-2: TDLA30 (DS = 30 ns)

| Tap # | Delay (ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 |  |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

I.2.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., TDLA<DS>-<Doppler>, TDLB<DS>-<Doppler> or TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table I.2.2-1 and I.2.2-2 show the propagation conditions that are used for the performance measurements in multi-path fading environment for low, medium and high Doppler frequencies for FR1 and FR2, respectively.

Table I.2.2-1: Channel model parameters for FR1

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-10 | TDLA30 | 10 Hz |

Table I.2.2-2: Channel model parameters for FR2

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-75 | TDLA30 | 75 Hz |

I.2.3	MIMO channel correlation matrices

The MIMO channel correlation matrices defined in I.2.3 apply for the antenna configuration using uniform linear arrays at both gNB and NCR-MT and for the antenna configuration using cross polarized antennas.

#### I.2.3.1	MIMO correlation matrices using Uniform Linear Array (ULA)

The MIMO channel correlation matrices defined in I.2.3.1 apply for the antenna configuration using uniform linear array (ULA) at both gNB and NCR-MT.

##### I.2.3.1.1	Definition of MIMO correlation matrices

Table I.2.3.1.1-1 defines the correlation matrix for the gNB.

Table I.2.3.1.1-1: gNB correlation matrix

|  | gNB correlation |
| --- | --- |
| One antenna |  |
| Two antennas |  |

Table I.2.3.1.1-2 defines the correlation matrix for the NCR-MT:

Table I.2.3.1.1-2: NCR-MT correlation matrix

|  | NCR-MT correlation |
| --- | --- |
| Two antennas |  |
| Four antennas |  |
| Note: The correlation matrix RUE applies to NCR-MT for NCR-MT requirements |  |

Table I.2.3.1.1-3 defines the channel spatial correlation matrix. The parameters, α and β in table I.2.3.1.1-3 defines the spatial correlation between the antennas at the gNB and NCR-MT respectively.

Table I.2.3.1.1-3: correlation matrices

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |

For cases with more antennas at either gNB or NCR-MT or both, the channel spatial correlation matrix can still be expressed as the Kronecker product of RgNB and RUE according to Rspat=RgNB⊗RUE.

##### I.2.3.1.2	MIMO correlation matrices at high, medium and low level

The  and  for different correlation types are given in table I.2.3.1.2-1.

Table I.2.3.1.2-1: Correlation for high, medium and low level

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.3 | 0.9 | 0.9 | 0.9 |

The correlation matrices for high, medium and low correlation are defined in tables I.2.3.1.2-2, I.2.3.1.2-3 and I.2.3.1.2-4 as below.

The values in table I.2.3.1.2-2 have been adjusted for the 2x4 high correlation cases to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

Where the value "a" is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 2x4 high correlation case, a=0.00010.

Table I.2.3.1.2-2: MIMO correlation matrices for high correlation

| 1x2 case |  |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |

Table I.2.3.1.2-3: MIMO correlation matrices for medium correlation

| 1x2 case | N/A |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |

Table I.2.3.1.2-4: MIMO correlation matrices for low correlation

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |

In table I.2.3.1.2-4,  is a  identity matrix.

NOTE:	For completeness, the correlation matrices were defined for high, medium and low correlation but performance requirements exist only for low correlation.

#### I.2.3.2	Multi-antenna channel models using cross polarized antennas

The MIMO channel correlation matrices defined in I.2.3.2 apply to two cases as presented below:

-	One TX antenna and multiple RX antennas case, with cross polarized antennas used at gNB

-	Multiple TX antennas and multiple RX antennas case, with cross polarized antennas used at both UE and gNB

The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB. For one TX antenna case, antenna element with +90 degree polarization slant angle is deployed at UE. For multiple TX antennas case, cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at UE.

For the cross-polarized antennas, the N antennas are labelled such that antennas for one polarization are listed from 1 to N/2 and antennas for the other polarization are listed from N/2+1 to N, where N is the number of TX or RX antennas.

##### I.2.3.2.1	Definition of MIMO correlation matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

Where

-	 is the spatial correlation matrix at the UE with same polarization,

-	 is the spatial correlation matrix at the gNB with same polarization,

-	 is a polarization correlation matrix,

-	 is a permutation matrix, and

-	 denotes transpose.

Table I.2.3.2.1-1 defines the polarization correlation matrix.

Table I.2.3.2.1-1 : Polarization correlation matrix

|  | One TX antenna | Multiple TX antennas |
| --- | --- | --- |
| Polarization correlation matrix |  |  |

The matrixis defined as

where  and  is the number of TX and RX antennas respectively, and  is the ceiling operator.

The matrix  is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in I.2.3.2.

##### I.2.3.2.2	Spatial correlation matrices at NCR-MT and gNB sides

###### I.2.3.2.2.1	Spatial correlation matrices at NCR-MT side

For 2-antenna receiver using one pair of cross-polarized antenna elements, .

For 4-antenna receiver using two pairs of cross-polarized antenna elements, .

###### I.2.3.2.2.2	Spatial correlation matrices at gNB side

For 1-antenna transmitter, .

For 2-antenna transmitter using one pair of cross-polarized antenna elements, .

##### I.2.3.2.3	MIMO correlation matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in table I.2.3.2.3-1.

Table I.2.3.2.3-1: Values for parameters α,  and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
| Α |  | γ |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |

##### I.2.3.2.4	Beam steering approach

For the 2D cross-polarized antenna array at gNB, given the channel spatial correlation matrix in I.2.3.2.1, I.2.3.2.2 and I.2.3.2.3, the corresponding random channel matrix H can be calculated. The signal model for the k-th slot is denoted as

And the steering matrix is further expressed as following:

where

-	H is the Nr×Nt channel matrix per subcarrier.

-	 is the steering matrix,

-	 is the steering matrix in first dimension with same polarization,

-	 is the steering matrix in second dimension with same polarization,

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1.

For 1 antenna element with the same polarization in one direction,

.

For 2 antenna elements with the same polarization in one direction,

.

For 3 antenna elements with the same polarization in one direction,

.

For 4 antenna elements with the same polarization in one direction,

.

where the index  stands for first dimension and second dimension respectively.

-	 controls the phase variation in first dimension and second dimension respectively, and the phase for k-th subframe is denoted by, where is the random start value with the uniform distribution, i.e., ,  is the step of phase variation, which is defined in Table I.2.3.2.4-1, and k is the linear increment of 2-μ for every slot throughout the simulation, the index  stands for first dimension and second dimension respectively.

-	 is the precoding matrix for Nt transmission antennas,

-	y is the received signal, x is the transmitted signal, and n is AWGN.

-	 corresponds to subcarrier spacing configuration,

For the 1D cross-polarized antenna array at gNB, the corresponding random channel matrix H can be calculated by letting N2=1, i.e.,

Table I.2.3.2.4-1: The step of phase variation

| Variation Step | Value (rad/ms) |
| --- | --- |
|  | 1.2566×10-3 |

## I.3	Physical signals, channels mapping and precoding

I.3.1	General

Unless otherwise stated, the transmission on antenna port(s)  is defined by using a precoder matrix  of size , where is the number of physical transmit antenna elements configured per test ,  is the number of ports for a reference signal or physical channel configured per test, and   is the first port for that reference signal or physical channel as defined in clauses 7.3 and 7.4 in TS 38.211 [18]. This precoder takes as an input a block of signals for antenna port(s) , , , with  being the number of modulation symbols per antenna port including the reference signal symbols, and generates a block of signals  the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

For Clause 6 and 8, the transmission of PDCCH and PDCCH DMRS on antenna port  is defined by using a precoder matrix  of size 2x1. This precoder takes as an input a block of signals for antenna port(s) ,  and generates a block of signals  the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

The precoder matrix is specific to the test case configuration  is defined in Clause 5.2.2.2 of TS 38.214 [15].

The transmission on PT-RS antenna port is associated (using same precoder) with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH.

The physical antenna elements are identified by indices, where  is the number of physical antenna elements configured per test.

Modulation symbols  with  (i.e. PSS, SSS, PBCH and DM-RS for PBCH) are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for tracking with one port are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for beam refinement with one port are directly mapped to first physical antenna element.

Modulation symbols  for NZP CSI-RS which configured for CSI acquisition with    are mapped to the physical antenna index  where is the number of NZP CSI-RS ports configured per test.

Annex J (normative):

Expected EIRP calculation and spatial sampling grid

## J.1	General

The Expected EIRP calculation is based on two averaging processes:

-	Averaging over the test beam directions.

-	Averaging over horizontal and vertical angles within specified vertical angles bins.

Clause J.2 describes the details of the calculation of the EEIRP from the measured EIRP samples.

NOTE: Clause J.2 assumes that averaging over beam directions is performed first. It is also correct to perform first the averaging over horizontal and vertical angles (as described in J.2.2) and then averaging over the test beams (as described in J.2.1).

The required sampling resolution for the EIRP measurement per test beam direction is described in Annex J.3

## J.2	Averaging processes

### J.2.1 Averaging over beam directions

The first averaging process involves averaging over different test beam directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=k=1KwkEIRPk(θ,φ)

The power measured is the sum of the EIRP in both polarisations. The EIRPθ,φ is the average EIRPk(θ,φ) over the K number of test beam directions for the angle (θ,φ).

For the case where non-zero mechanical tilt is considered the reference coordinate system in clause 4.12 in this document needs to be converted to the coordinate system used by the test requirement (Figure 6.17.5.1-1). The angles in the reference coordinate system can be translated to the coordinate system with respect to horizon as:

θH=90-arccoscosφsinθ+90sin-θMT+cosθ+90cos-θMT

φH=argcosφsinθ+90cos-θMT-cosθ+90sin-θMT+jsinφsinθ+90 ,

where:

θ 		The angle in the reference coordinate system defined in sub-clause 4.12 between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

φ 	is the angle in the reference coordinate system defined in sub-clause 4.14 between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

θH 	is the angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

θMT 	is the down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).

φH         The horizontal angle in the global coordinate system, defined with respect to the horizon.

### J.2.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore:

EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn  ,

Where

(qn, jm) represents discrete spatial measurement points in the measured average EIRP samples. The sample resolution is defined in Annex J.3.

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰ and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φm	is the corresponding angle of the mth midpoint between the azimuth intervals

nL	is the lowest elevation index of sampling points within the θHLi , θHHi range

nH	is the highest elevation index of sampling points within the θHLi , θHHi range

M	is the number of azimuth points within the (-180°, 180°) range

N	is the number of elevation points within the θHLi , θHHi range

Note: All angles are in degrees.

### J.3 Spatial sampling grid

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD ,

where  is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=w2+h2 ,

where w is the array antenna width, and h is the array antenna height.

Annex K (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN4#103e | R4-2210846 |  |  |  | TS skeleton | 0.0.1 |
| 2022-08 | RAN4#104e | R4-2214368 |  |  |  | Added approved TPs in RAN4#104-e including R4-2211703,TP for TS 38.115-2: Clause 4.2-4.5 R4-2211704,TP for TS 38.115-2: Clause 5 operating bands R4-2213722,TP for TS 38.115-2: section 4.10~4.12 R4-2213977,TP to TS 38.115-2 Annex A Repeater stimulus signals (OTA) R4-2214742,TP for TS 38.115-2: Clause 3 definitions R4-2214788,TP to TS 38.115-2: Frequency Stability, Out of band gain, unwanted emissions R4-2214792,TP to TS 38.115-2: TDD Switching R4-2214802,TP to TS 38.115-2 Manufacturer declarations for NR FR2 repeaters R4-2214804,TP to TS 38.115-2 clause 6.7 OTA Input intermodulation R4-2214842,TP for TS 38.115-2: section 6.8 R4-2214843,TP for TS 38.115-2: Annex D and E R4-2214868,TP to TS 38.115-2 clause 6.6 OTA EVM | 0.1.0 |
| 2022-10 | RAN4#104bis-e | R4-2216568 |  |  |  | Added approved TPs in RAN4#104-e including R4-2214786,TP to TS 38.115-2: Test Configurations and Requirement applicability R4-2214790,TP to TS 38.115-2: In-band measurements Annex R4-2214866,TP to TS 38.115-2 clause 4.9 RF channels and test models Added approved TPs in RAN4#104bis-e including R4-2217302,TP to TS 38.115-2 – Annex I TRP measurement procedures R4-2217300,TP for TS 38.115-2: Scope, reference and editorial changes R4-2217308,TP to TS 38.115-2: Measurement uncertainties and test requirements (4.1) R4-2217310,TP to TS 38.115-2: OTA output power (6.1, 6.2) R4-2217309,TP to TS 38.115-2: Annex B: Test tolerances and derivation of test requirements R4-2216844,TP to TS 38.115-2: Annex A: Environmental requirements for the repeater | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2219348 |  |  |  | Added approved TPs in RAN4#105 including R4-2220215 TP to 38.115-2: OTA ACLR requirement R4-2220218 TP to 38.115-2: OTA EVM requirement R4-2220271 TP to 38.115-2: OTA ACRR requirement R4-2220227 Draft CR to 38.115-2: Spectrum purity R4-2220228 TP for TS 38.115-2 OTA Out of band gain requirements R4-2220289 Maintenance TP for TS 38.115-2 | 0.3.0 |
| 2022-12 | RAN#98e | RP-223091 |  |  |  | Draft version for approval to RAN Plenary meeting: RP-223091, TS 38.115-2 v1.0.0 NR; Repeater conformance testing - Part 2: Radiated conformance testing | 1.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-12 | RAN#98-e |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |
| 2023-03 | RAN#99 | RP-230518 | 0001 | 1 | F | CR for TS 38.115-2: Correction of some errors | 17.1.0 |
| 2023-03 | RAN#99 | RP-230518 | 0002 |  | F | CR to 38.115-2: Correction on OTA repeater EVM test requirements | 17.1.0 |
| 2023-03 | RAN#99 | RP-230518 | 0003 |  | F | CR to 38.115-2: Correction on derivation of test requirements | 17.1.0 |
| 2023-03 | RAN#99 | RP-230518 | 0004 |  | F | CR to 38.115-2: Correction of repeater ACLR test procedure | 17.1.0 |
| 2023-06 | RAN#100 | RP-231339 | 0005 | 1 | F | CR for TS 38.115-2, Add manufacturer declarations for test configurations and RF channels | 17.2.0 |
| 2023-06 | RAN#100 | RP-231339 | 0006 |  | F | CR to 38.115-2: Corrections on repeater output power requirements | 17.2.0 |
| 2023-06 | RAN#100 | RP-231339 | 0007 | 1 | F | CR to TS 38.115-2: Clarifcations for repeater test models | 17.2.0 |
| 2023-09 | RAN#101 | RP-232496 | 0008 | 1 | F | [NR_repeaters] CR to 38.115-2: Input intermodulation | 17.3.0 |
| 2023-12 | RAN#102 | RP-233350 | 0009 |  | F | CR for TS 38.115-2, Remove multi-band related content for repeater type 2-O | 17.4.0 |
| 2023-12 | RAN#102 | RP-233350 | 0010 | 1 | F | CR to 38.115-2: Correction of terminologies for NR repeaters | 17.4.0 |
| 2024-03 | RAN#103 | RP-240574 | 0011 |  | F | (NR_repeaters-Perf) CR for TS 38.115-2, Correction on BS related description issues | 17.5.0 |
| 2024-03 | RAN#103 | RP-240574 | 0012 | 1 | F | CR to 38.115-2: NR repeater transmitter spurious emissions requirements (rel-18) | 17.5.0 |
| 2024-03 | RAN#103 | RP-240588 | 0013 |  | B | Big CR on Performance Requirements for NCR-MT in 38.115-2 | 18.0.0 |
| 2024-06 | RAN#104 | RP-241422 | 0015 |  | F | CR on FRCs of NCR MT Demodulation Performance in 38.115-2 | 18.1.0 |
| 2024-06 | RAN#104 | RP-241422 | 0016 |  | F | CR on Propagation Condition of NCR-MT for 38.115-2 | 18.1.0 |
| 2024-06 | RAN#104 | RP-241422 | 0017 |  | B | Big CR to TS 38.115-2: the introduction of NCR | 18.1.0 |
| 2024-09 | RAN#105 | RP-242172 | 0019 | 1 | F | (NR_netcon_repeater-Perf)CR for TS 38.115-2, Correction on network controlled repeater classes for NCR | 18.2.0 |
| 2024-09 | RAN#105 | RP-242172 | 0023 | 1 | F | Maintenance CR of NCR to TS 38.115-2 | 18.2.0 |
| 2024-09 | RAN#105 | RP-242172 | 0024 | 1 | F | CR on 38.115-2 for NCR conformance testing | 18.2.0 |
| 2024-09 | RAN#105 | RP-242172 | 0025 | 1 | F | CR to TS 38.115-2 with updates and corrections | 18.2.0 |
| 2024-09 | RAN#105 | RP-242172 | 0026 | 1 | F | NCR type 2-O testing: MT requirements testing restrictions | 18.2.0 |
| 2024-12 | RAN#106 | RP-243053 | 0028 | 1 | F | CR on Correlation Matrix of NCR Demodulation in 38.115-2 | 18.3.0 |
| 2024-12 | RAN#106 | RP-243053 | 0029 |  | F | CR to 38.115-2: Rated TRP output power limit | 18.3.0 |
| 2024-12 | RAN#106 | RP-243053 | 0030 | 1 | F | CR to 38.115-2: ACLR requirements for NCR | 18.3.0 |
| 2024-12 | RAN#106 | RP-243053 | 0031 | 1 | F | CR to TS 38.115-2 with NCR type 1-H missing in some requirements | 18.3.0 |
| 2025-03 | RAN#107 | RP-250592 | 0033 | 1 | F | (NR_repeaters-Perf)CR for TS38.115-2: On mmWave EESS protection | 18.4.0 |
| 2025-03 | RAN#107 | RP-250592 | 0034 | 1 | F | Maintenance CR to TS 38.115-2: NCR conformance part | 18.4.0 |
| 2025-03 | RAN#107 | RP-250592 | 0035 |  | F | CR on NCR-MT Correlation Matrix in 38.115-2 | 18.4.0 |
| 2025-06 | RAN#108 | RP-250921 | 0038 |  | A | (NR_repeaters-Perf) CR to TS 38.115-2: Correction of the OTA input intermodulation test requirement Note: Huawei asked that in Table C.1-1, Test 6.7 OTA input intermodulation, for the TT +/- should not be implemented in the CR as it was a typo in CR. | 18.5.0 |
| 2025-06 | RAN#108 | RP-250921 | 0040 |  | A | (NR_repeaters-Perf) CR to TS 38.115-2: Removal of TRP annex | 18.5.0 |
| 2025-06 | RAN#108 | RP-250921 | 0042 |  | A | (NR_repeaters-Perf) CR to TS 38.115-2: spec quality and consistency improvements | 18.5.0 |
| 2025-06 | RAN#108 | RP-250929 | 0043 |  | F | CR on NCR Mimo Correlation Matrices in 38.115-2 | 18.5.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-09 | RAN#109 | RP-252416 | 0044 | 2 | B | CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR | 19.0.0 |