| 3GPP TR 23.700-21 V1.2.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application enablement architecture for mobile metaverse services; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Key issues	11

4.1	Key issue #1: Enabler support for managing spatial anchors	11

4.1.1	Description	11

4.1.2	Open issues	11

4.2	Key Issue #2: Exposure of user sensitive information	11

4.2.1	Description	11

4.2.2	Open issues	12

4.3 	Key issue #3: Avatar and digital asset support	12

4.3.1	Description	12

4.3.2	Open issues	13

4.4 	Key issue #4: Spatial mapping	13

4.4.1	Description	13

4.4.2	Open issues	13

4.5	Key issue #5: Support for avatar discovery and QoS control	14

4.5.1	Description	14

4.5.2	Open issues	15

4.6	Key issue #6: Support device discovery to offload task for metaverse services	15

4.6.1	Description	15

4.6.2	Open issues	15

4.7	Key issue #7: Support for metaverse services requiring multiple devices	15

4.7.1	Description	15

4.7.2	Open issues	16

4.8	Key issue #8: Enabler support for spatial anchors based services	16

4.8.1	Description	16

4.8.2	Open issues	16

4.9	Key issue #9: Support for permission control of digital assets	16

4.9.1	Description	16

4.9.2	Open issue	17

5	Application enablement architecture requirements	17

5.1	General requirements	17

5.2	Spatial anchors management service requirements	17

5.3	Spatial map management service requirements	18

6	Application enablement architecture for metaverse services	18

6.1	Option #1: On-network mobile metaverse application layer architecture	18

6.1.1	Application enablement architecture	18

6.1.2	Functional Elements	19

6.1.2.1	Mobile Metaverse Enablement Server	19

6.1.2.2	Mobile Metaverse Enablement Client	19

6.1.3	Reference Points	19

6.1.3.1	MM-UU	19

6.1.3.2	MM-S	20

6.1.3.3	MM-E	20

6.1.3.4	MM-C	20

6.2	Option #2: A-DACM architecture to support Metaverse services	20

6.2.1	Application enablement architecture	20

6.2.2	Functional Elements	22

6.2.2.1	A-DACM function	22

6.2.3	Reference Points	22

6.2.3.1	SEAL-S	22

6.2.4	Service-based interface	22

6.2.4.1	Sdacm	22

6.3	Option #3: Reuse existing SEAL architecture for metaverse services	22

6.3.1	Application enablement architecture	22

6.4	Option #4: Architecture to support Metaverse services with permission control	22

6.4.1	Application enablement architecture	22

6.4.2	Functional Elements	23

6.4.2.1	DAPM function	23

6.4.2.2	UE DAPM function	23

6.4.3	Reference Points	23

6.4.3.1	PM-V	23

6.4.3.2	CM-PM	24

6.4.3.3	PM-C	24

7	Solutions	24

7.0	Mapping of solutions to key issues	24

7.1	Solution #1: Spatial anchor discovery	25

7.1.1	Solution description	25

7.1.1.1	General	25

7.1.1.2	Procedure	25

7.1.1.3	Creating spatial anchor group	26

7.1.1.3.1	General	26

7.1.1.3.2	Procedure	26

7.1.2	Architecture Impacts	27

7.1.3	Corresponding APIs	27

7.1.3.1	Overview	27

7.1.3.2	Information flows	27

7.1.4	Solution evaluation	28

7.2	Solution #2: Support for spatial anchor management	28

7.2.1	Solution description	28

7.2.2	Architecture Impacts	29

7.2.3	Procedures	29

7.2.3.1	Creating spatial anchor	29

7.2.3.2	Updating spatial anchor	30

7.2.3.3	Get spatial anchor	30

7.2.3.4	Spatial anchor information subscription	31

7.2.4	Solution evaluation	31

7.3	Solution #3: Spatial anchor subscription	32

7.3.1	Solution description	32

7.3.1.1	General	32

7.3.1.2	VAL server subscription procedure for information about UE with spatial anchor interest	32

7.3.1.3	VAL client subscription procedure for spatial anchors in range	33

7.3.2	Architecture Impacts	34

7.3.3	Corresponding APIs	34

7.3.3.1	Overview	34

7.3.3.2	Information flows	35

7.3.4	Solution evaluation	36

7.4	Solution #4: Support for spatial anchor analytics information	37

7.4.1	Solution description	37

7.4.2	Procedures	37

7.4.2.1	Spatial anchor analytics information (subscription-notify)	37

7.4.3	Architecture Impacts	38

7.4.4	Corresponding APIs	38

7.4.4.1	Overview	38

7.4.5	Solution evaluation	38

7.5	Solution #5: Support for digital avatars	38

7.5.1	Solution description	38

7.5.2	Architecture Impacts	38

7.5.3	Procedures	39

7.5.3.1	Create application specific avatar profile	39

7.5.3.2	Update application specific avatar profile	41

7.5.3.3	Get application specific avatar profile	42

7.5.3.4	Delete application specific avatar profile	42

7.5.3.5	Application specific avatar profile(s) subscription	43

7.5.4	Solution evaluation	43

7.7	Solution #7: Digital avatar support by A-DACM	46

7.7.1	Solution description	46

7.7.1.1	General	46

7.7.1.2	Avatar profile creation	46

7.7.1.3	Avatar profile upload	47

7.7.1.4	Avatar linking to user/subscriber	47

7.7.1.5	Avatar download	48

7.7.2	Architecture Impacts	48

7.7.3	Corresponding APIs	48

7.7.4	Solution evaluation	49

7.8	Solution #8: Support for spatial map management	49

7.8.1	Solution description	49

7.8.2	Architecture Impacts	49

7.8.3	Procedures	49

7.8.3.1	Producing spatial map	49

7.8.3.2	Update spatial map	50

7.8.3.3	Get spatial map	51

7.8.3.4	Subscribe/unsubscribe spatial map	51

7.8.3.5	Spatial localization services	52

7.8.3.6	Delete spatial map	53

7.8.4	Corresponding APIs	54

7.8.4.1	Produce spatial map	54

7.8.4.2	Update spatial map	54

7.8.4.3	Get spatial map	55

7.8.4.4	Subscribe/unsubscribe spatial map	55

7.8.4.5	Delete spatial map	57

7.8.5	Solution evaluation	58

7.9	Solution #9: Avatar discovery procedure	58

7.9.1	Solution description	58

7.9.2	Architecture Impacts	59

7.9.3	Corresponding APIs	59

7.9.4	Solution evaluation	60

7.10	Solution #10: Metaverse digital asset synchronization QoS control	60

7.10.1	Solution description	60

7.10.2	Architecture Impacts	62

7.10.3	Corresponding APIs	62

7.10.4	Solution evaluation	62

7.11	Solution #11: Application QoS coordination for Mobile Metaverse Sessions	62

7.11.1	Solution description	62

7.11.2	Architecture Impacts	65

7.11.3	Corresponding APIs	65

7.11.4	Solution evaluation	65

7.12	Solution #12: Support device discovery to offload task for metaverse services	65

7.12.1	Solution description	65

7.12.1.1	PIN client requests to discover another PIN element	66

7.12.2	Architecture Impacts	66

7.12.3	Corresponding APIs	66

7.12.4	Solution evaluation	67

7.13	Solution #13: Support for metaverse services requiring multiple devices	67

7.13.1	Solution description	67

7.13.1.1	Enhancement to EAS profile	67

7.13.1.2	Enhancement to EAS discovery request (between AC to EEC)	68

7.13.1.3	Enhancement to EAS discovery procedure (between EEC to EES)	68

7.13.2	Architecture Impacts	68

7.13.3	Corresponding APIs	68

7.13.4	Solution evaluation	69

7.15	Solution #15: Support for VAL UEs in spatial map	70

7.15.1	Solution description	70

7.15.2	Architecture Impacts	71

7.15.3	Procedures	71

7.15.3.1	Create spatial map augmented with VAL UE	71

7.15.3.2	Update spatial map augmented with VAL UEs	72

7.15.4	Corresponding APIs	73

7.15.4.1	Overview	73

7.15.4.2	Information Flows	73

7.15.5	Solution evaluation	74

7.16	Solution #16: Spatial mapping service exposure and discovery	74

7.16.1	Solution description	74

7.16.1.1	General	74

7.16.1.2	Exposing a spatial mapping service	75

7.16.1.3	Discovering a spatial mapping service	75

7.16.1.4	Service exposure via CAPIF	76

7.16.2	Architecture Impacts	77

7.16.3	Corresponding APIs	77

7.16.3.1	Overview	77

7.16.3.2	Information flows	77

7.16.4	Solution evaluation	78

7.17	Solution #17: Discovery of UEs consenting and capable of providing spatial mapping information	78

7.17.1	Solution description	78

7.17.1.1	General	78

7.17.1.2	Registering a spatial mapping information provider	79

7.17.1.3	Discovering and triggering a spatial mapping information provider	79

7.17.2	Architecture Impacts	80

7.17.3	Corresponding APIs	81

7.17.3.1	Overview	81

7.17.3.2	Information flows	81

7.17.4	Solution evaluation	82

7.18	Solution #18: Discovery and requirements translation for 5G-enabled Traffic Flow Simulation and Situational Awareness	83

7.18.1	Solution description	83

7.18.2	Architecture Impacts	85

7.18.3	Corresponding APIs	85

7.18.4	Solution evaluation	85

8	Deployment scenarios	85

8.1	General	85

8.2	Deployment model #1: Deployment of enabler for Metaverse Services as SEAL server(s)	86

9	Business Relationships	86

10	Overall evaluation	86

10.1	Architecture evaluations	86

10.1.1	On-network mobile metaverse application layer architecture	86

10.1.2	SEAL architecture to support metaverse applications	86

10.2	Key issue evaluations	87

10.2.1	Key issue #1: Enabler support for managing spatial anchors	87

10.2.2	Key issue #2: Exposure of user sensitive information	87

10.2.3	Key issue #3: Avatar and digital asset support	87

10.2.4	Key issue #4: Spatial mapping	87

10.2.1	Key issue #5: Support for avatar discovery and QoS control	88

10.2.6	Key issue #6: Support device discovery to offload task for metaverse services	88

10.2.7	Key issue #7: Support for metaverse services requiring multiple devices	89

10.2.8	Key issue #8 solutions evaluation	89

11	Conclusions	89

11.1	Architecture conclusions	89

11.2	Conclusions of key issues	90

Annex A (informative): Change history	91

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report capturing the study on application enablement architecture for mobile metaverse services over 3GPP networks. The aspects of the study include defining key issues and their related solutions to identify architecture requirements, application layer functional model (either by defining new functional model or by enhancing existing functional model) to support identified architectural requirements and corresponding solutions to provide enablement layer support to metaverse applications.

The study is based on the requirements as defined in 3GPP TS 22.156 [2] and related use cases in 3GPP TR 22.856 [3]. The study takes into consideration the work done in other 3GPP working groups (like SA2, SA4).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2".

[5]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[6]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TR 26.119: "Media Capabilities for Augmented Reality".

[12]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[13]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[14]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Avatar profile: It is avatar specific configuration and parameters (e.g. allowed locations) applicable to one or more application(s). The avatar profile may be associated with one or more VAL user(s).

Application-Digital Asset Container Management: An entity that securely manages the application related digital assets (e.g. avatars).

Digital Asset Identifier: In the context of this TR, digital asset identifier is used to uniquely identify a digital asset across different mobile metaverse services.

Digital Asset Permission Management: An entity that provides authorization service for consumers or vertical applications to operate digital assets, manages the permissions, and controls the operation of the digital assets.

Processed sensor data: data for the Spatial mapping service to estimate the number of objects, type of the object, position, direction and speed of the object, etc.

Spatial map information: information to be included and managed for a Spatial map.

VAL UE augmented spatial map: spatial map augmented with VAL UE information.

Spatial anchor map: Combine the spatial anchor information with the spatial map information.

For the purposes of the present document, the following terms given in 3GPP TS 22.156 [2] apply:

Avatar

Digital asset

Digital wallet

Spatial anchor

Spatial map

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A-DACM	Application - Digital Asset Container Management

DAPM	Digital Asset Permission Management

MDRB	Metaverse Digital Representation Block

MMEC	Mobile Metaverse Enablement Client

MMEL	Mobile Metaverse Enablement  Layer

MMES	Mobile Metaverse Enablement  Server

# 4	Key issues

## 4.1	Key issue #1: Enabler support for managing spatial anchors

### 4.1.1	Description

SA1 in 3GPP TS 22.156 [2] defines spatial anchors as "an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content".

Application providers can use spatial anchors to associate application content and services with location information which can be managed by the 3GPP system. In turn, spatial anchors can be used by application clients to discover and access application content and services in a location aware fashion.

Clause 5.4 "Use Case on Spatial Anchor Enabler" of 3GPP TR 22.856 [3] provides detailed use case for spatial anchor enabler. The use case defines mainly two entities:

1)	Spatial anchor producer: It creates the spatial anchor along with its place/location in the 3D space. It determines what to share and its location, and any constraints (e.g. who to share the spatial anchor with) and additional information.

2)	Spatial anchor consumer: It recognizes anchors associated with locations in 3D space, and use the spatial anchor to obtain the associated information.

In order to create spatial anchor, the producer captures the location of the product and assocates a new spatial anchor with this location and product information. The producer can adjust/update the spatial anchors for its location or service information or removes it completely. The consumer can retrieve information about spatial anchors.

SA6 already supports SEAL layer. SA6 can enhance the existing SEAL capability (like location management) to enable support for spatial anchor.

### 4.1.2	Open issues

SA6 defined enablers as of now do not support operations required for managing spatial anchors. The open issues are:

1)	How to enhance existing appliction enabler (either adding new capability in existing server or defining new capability server) to support CRUD operations for spatial anchors (association between location and service information on customer premises)?

2)	How to discover spatial anchors by the consumer (e.g. UE, VAL server)?

3)	How can UE(s) within a certain range of spatial anchor be detected?

## 4.2	Key Issue #2: Exposure of user sensitive information

### 4.2.1	Description

Ensuring appropriate user consent has been obtained is a critical aspect when handling sensitive information relating to or collected from a user, their devices or the applications installed at their devices. This is highlighted at stage 1 where many of the requirements captured in 3GPP TS 22.156 [2] are subject to operator policy, regulatory requirements and user consent, the latter being of particular importance to this key issue.

For instance, with the expected capability to access, manage and expose user specific avatar related information through the enabler layer it is of utmost importance to capture the consent of the user. Aspects of such information might also be obtained through core network capability exposure, e.g., relating to user identity, body movement or location, and re-exposed by the enabler layer but not without the consent of the user. Another example are, digital assets, where ownership rights are considered to be classified as sensitive information and the owner's consent must be obtained before other users or third party(service provider) can use them. An example of a digital asset is a user designed and produced digital human image. When another user likes that digital human image and wants to use it, they need to request and obtain the owner's consent before doing so.

Each user needs to be in full control of which network and device entities are allowed to access, manage and expose information pertaining to themselves and their devices. Such end user approvals ensure that only legitimate, user trusted, entitles can obtain and utilise information and services involving that user's sensitive information.

### 4.2.2	Open issues

SA6 defined enablers as of now do not consider usage of user consent in the context of supporting localized mobile metaverse services. The open issues are:

-	How to ensure appropriate usage of user consent at the enabler layer to support localized mobile metaverse services in 3GPP specified networks? Appropriate usage of user consent includes ensuring only the legitimate use by non-owners of user sensitive information such as digital assets.

NOTE 1:	Aspects pertaining to the definition of end-user consent/authorization over APIs are in the scope of SA3.

NOTE 2:	Obtaining end-user consent is expected to leverage the RNAA framework, where it is of note that resource owner communication with the authorization function in the CAPIF core function was not in scope of Rel-18.

NOTE 3:	Some aspects may depend on interactions with SA WG2 FS_UIA_ARC. For example, utilisation of user-specific identities in relation to specific components of a user’s information.

NOTE 4：The solution of the case of authorized use of user sensitive information by non-owners may need coordination with SA3.

Editor's Note: The open issues of this key issue can be revisited to consider mobile metaverse constructs such as digital avatars.

Editor's Note: The solution to the key issue needs coordination with SA3.

## 4.3 	Key issue #3: Avatar and digital asset support

### 4.3.1	Description

In addition to new 5GS performance requirements, support for Metaverse services have introduced new use cases and requirements related to optimal support for multi-user, highly performant applications. These use cases led to new CN functionality supporting richer digital representations of physical entities (e.g. user profiles in SA2, XR scene in SA4)

As defined in 3GPP TS 22.156 [2], avatars are digital representations of users interacting with the metaverse and with other users. The application enabler layer can enable creation, discovery, and management of avatar profiles for users to offload applications and enable Core Network functionality across services and verticals. The application layer can enable such functionality in a similar manner for avatars as well as for other digital assets.

Considering different application-level profiles of the user, some digital assets can be access/used together. The application enabler layer can support managing such multiple digital assets together.

NOTE 1: The work in SA6 focuses on providing enablement service to application specific Avatar profile and if available, can leverage the work done in SA2 (e.g. user profiles) and SA4.

Editor's Note: How application specific Avatar profile can be linked to user subscription is FFS and depends on SA2 work.

Therefore, solutions addressing this key issue may also leverage work from SA2 and SA4 on user profiles and digital assets.

The following use cases can be considered for study of value-added features:

-	A user provides configuration (or preferences) indicating which metaverse application services can access the avatar information when the user performs transition between metaverse services, to make sure that consistency and continuity of their digital representation is maintained.

-	SA6 can maintain a list of applications allowed to use the avatar (similar to allowed MNO list for federation in EDGEAPP)

-	SA6 can maintain the associations between Avatar profile and user information (e.g. non-3GPP or VAL-layer user information, 5GC exposed information, etc.)

NOTE 2: 5GC exposed user information is to use/reuse SA2 specifications.

-	A User creates multiple avatars and indicates his/her preferences such as: use of specific avatars based on location,  use of audio-only service only when experiencing low bandwidth.

-	SA6 can maintain user’s choices and: provide avatar information to different application servers based on the location of the user, reject requests for avatar-based communication in poor network conditions, etc.

-	A user creates over time multiple digital assets. In addition to configuring individual parameters and preferences for each asset or asset type separately, he/she also provides configuration/ preferences for managing the digital assets to be accessed together for specific applications, which requires identifying common information and/or metadata for multiple digital assets.

### 4.3.2	Open issues

This key issue will study:

-	What SA6-defined information is required to expose the SA2-defined, SA4-defined and SA3-defined avatar information to support allowed application configuration, location-based avatar contextual information?

-	Whether and how application enablement layer manages and exposes information about digital avatars  to the consumers (i.e. application clients and servers) or across verticals?

NOTE 1:	The study considers non-IMS based metaverse services.

NOTE 2:	The study considers that a user can own multiple digital assets (1:n), while each digital asset has one owner (1:1). Further, a user can use more than one digital assets (1:n), while a digital asset can be used by multiple allowed users (1:m).

## 4.4 	Key issue #4: Spatial mapping

### 4.4.1	Description

In clause 5.2.1 "Localized mobile metaverse service" of 3GPP TS 22.156 [2],

Localized mobile metaverse services are immersive and integrated into a user's ordinary experiences. Such service experiences are location-related and can include presentation of AR, MR media.

Users' localization is important in order to discover spatial anchors. The 5G system offers a spatial localization service to determine this information.

[R-5.2.1-003] Subject to operator policy, regulatory requirements and user consent, the 5G system shall provide a means for a UE to provide sensor data, (e.g., from UE sensors, cameras, etc.) to the network in order to derive localization information, e.g., to produce or modify a spatial map or discover or find spatial anchors. The 5G system shall enable an authorized third party to obtain all the spatial anchors in a given three dimensional area.

NOTE 1:	How an authorized third party identifies which three-dimensional area to request spatial anchors in is not in scope of the 3GPP standard. Spatial localization and mapping information could be used to identify areas of interest.

[R-5.2.1-004] Subject to operator policy and regulatory requirements, the 5G system shall support mechanisms to expose a spatial map or derived localization information to authorized third parties.

SA6 already supports SEAL layer. SA6 can enhance the existing SEAL capability (like location management) to enable support for spatial mapping.

### 4.4.2	Open issues

SA6 defined enablers as of now do not support operations required for spatial mapping. The open issues are:

1)	How to produce or modify a spatial map by the consumer (e.g. UE, VAL server)?

2)	How to expose a spatial map to authorized third parties?

## 4.5	Key issue #5: Support for avatar discovery and QoS control

### 4.5.1	Description

Mobile Metaverse brings a new dimension to XR/VR services (since meta is about a persistent large-scale virtual interactive experience, where the digital assets are owned or deployed by the end users); and requires enhancements to 3GPP systems to ensure connectivity /performance and provide support.

Two relevant use case as described in 3GPP TS 22.156 [2] are about the 1) Mobile Metaverse Based Selective Multi-modal Feedback Service and 2) Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness. For instance, for the latter use case, to support traffic flow simulation and situational awareness service, the 5G network need to provide low latency, high data rate and high reliability transmission, and in addition, the 5G network may also need to be further enhanced to meet the service requirements for 5G-enabled traffic flow simulation and situation awareness. Meanwhile, in addition to the real objects which may host the UE, their corresponding virtual objects are also capable of interacting with each other and interact with physical objects via 5GS.

For such scenarios, there are some issues related to how the digital devices are instantiated and onboarded to edge/cloud platforms (supporting mobile metaverse services) and how a digital device discovers other digital devices within the mobile metaverse service.

Furthermore, in mobile metaverse scenarios there can be multiple sessions involved. The figure below shows an example of possible sessions for a mobile metaverse service.

Figure 4.5.1-1: Example use case for multiple sessions in mobile metaverse service

There can be multiple multimodal sessions considering:

1)	the interaction between the UE in the physical world and the digital UE at the metaverse (for providing sensor data/measurements and getting multimodal feedback) (green and blue links).

2)	interaction between digital UEs (or avatar UE) interacting at the metaverse world. Such avatars can be hosted in the same or different edge/cloud platforms or at the ASP domain.

3)	interaction between UE1 and UE2 via the network (which are in vicinity in metaverse but can be far away and served by different RATs/networks) for transactions between the UEs within the metaverse session.

4)	interaction of metaverse server with the UE1 and UE2 avatars to configure the interactions for the metaverse service and provide the digital environment as well as to provide ASP policies for the interactions.

Such multitude of sessions (with different traffic requirements) within the same service poses some issues related to how QoS is being monitored and coordinated considering the end-to-end mobile metaverse service requirements.

### 4.5.2	Open issues

This Key Issue aims to study:

-	How to support the discovery and grouping of digital/avatar and physical UEs so as to interact via the mobile network?

-	How to monitor and configure the application QoS parameters for the multimodal sessions within meta service?

NOTE 1:	Coordination with FS_XRApp may be required for the QoS aspects impacting the multimodal sessions

## 4.6	Key issue #6: Support device discovery to offload task for metaverse services

### 4.6.1	Description

SA1 studied use cases to support metaverse services in 3GPP TR 22.856 [3]. However, many of the devices may not be equiped with sufficient capacity to process the data. And so, they need support from other devices to process data as much near as possible.

In order to provide immersive interactive location agnostic service experience to mobile metaverse service customers, large amount of computing resouces is needed to perform real-time processing for audio, video, and interactive data, etc. It is possible that some of the devices (e.g. glasses) will not have enough computing resources to perform the real-time rendering, and may need to offload the computing to the nearby devices.

For example, SA6 has defined architecture for Application layer support for Personal IoT Network in 3GPP TS 23.542 [12]. To support above use case, a PINE element (i.e. glasses with limited computing capability) need to discover another nearby PINE element (i.e. a UE with sufficient computing capability) to offload the computational task. Current PINAPP architecture supports subscribe-notify mechanism where a PINE element gets notification when a new PINE element joins the PIN. However, not all devices may subscribe for such event. It is required to study how a PINE element (i.e. glasses with limited computing capability) discovers another nearby PINE elements.

### 4.6.2	Open issues

In this key issue, SA6 will study following open issues:

1)	Whether and how the existing SA6 defined enabler (e.g. PINAPP) is sufficient to support metaverse applications or any enhancements are required (e.g. to discover device from anothter device to offload task)?

## 4.7	Key issue #7: Support for metaverse services requiring multiple devices

### 4.7.1	Description

SA1 studied use cases to support metaverse services in 3GPP TR 22.856 [3]. In many of the use case, metaverse applications are defined at edge to provide law latency, and further, a user needs multiple devices inorder to work with metaverse services.

For example, consider a scenario where a metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game). Current discovery mechanism (e.g. EAS discovery mechanism) do not support such application server requirements to be considered in the discovery procedure. The possible gap is to consider required capabilities of supported devices (like having joystic or haptic device) to run specific metaverse service.

The existing SA6 defined enabler layer procedures are generic for all applications (e,g, EDGEAPP architecture).

It is required to study

-	How the interaction among multiple devices and AS can be supported in order to provide or to consume metaverse service;

### 4.7.2	Open issues

In this key issue, open issues are:

1)	To study whether and how to enhance architecture and procedures (e.g. EDGEAPP) to support metaverse service requiring multiple devices?

## 4.8	Key issue #8: Enabler support for spatial anchors based services

### 4.8.1	Description

SA1 in 3GPP TS 22.156 [2] defines spatial anchors as "an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content".

Application providers can use spatial anchors to associate application content and services with location information which can be managed by the 3GPP system. In turn, spatial anchors can be used by application clients to discover and access application content and services in a location aware fashion.

Clause 5.1 "Use Case on Localized mobile Metaverse serivce" of 3GPP TR 22.856 [3] provides a detailed use case for spatial anchor enabled services. It describes the use case where multiple spatial anchors and their services are linked to the space that the user is in. In the usecase, it is mentioned that one or more mobile metaverse services associated with a single location, to be combined to form a single location related service experience.

The clause 5.1 use case describes the example of recommending a spatial anchor of a restaurant to the user in the evening time in order to provide an enhanced service experience to the user based on the previous search history spatial anchor. This creates the need for the enabler to provide some intelligent spatial anchor service recommendations which could help the user and improve the service experience.

The consumers created spatial anchors may want to understand the analytics of the spatial anchors like users density per spatial anchor, spatial anchor density in a given location, number of times the spatial anchor is accessed to improve  their services, spatial anchor improvements like spatial anchor placement planning, user engagement, etc.

### 4.8.2	Open issues

The open issues are:

1)	How to combine one or more mobile metaverse services associated with spatial anchors?

2)	How to provide the exposure of the spatial anchor analytics to the consumers?

## 4.9	Key issue #9: Support for permission control of digital assets

### 4.9.1	Description

Digital asset is defined in 3GPP TS 22.156 [2] as "digitally stored information that is uniquely identifiable and can be used to realize value according to their licensing conditions and applicable regulations. Examples of digital assets include digital representation (avatar), software licenses, gift certificates, tokens and files (e.g. music files) that have been purchased. This is not an exhaustive list of examples. "

With the use scenarios of digital assets is getting richer in the mobile metaverse, the demands for the use of digital assets is growing. When digital assets are used by multiple users or used on multiple metaverse application platforms, the service provider needs to provide a permission control mechanism, i.e. whether the operation to the digital asset by the specific user or a group of users is allowed.

The use case can be considered for study as follows:

-	In the same metaverse service, an owner upload his/her digital asset to trusty public digital asset container (e.g. DACM). When owner wants to share his/her own digital assets (e.g. picture) and the owner only wants to share the digital asset to some specific user(s) in the mobile metaverse. Meanwhile a consumer wants to access the digital asset, the service provider supports for checking whether the consumer has the permission and provides access service to the consumer.

-	In multiple metaverse services, when a owner wants to operate his digital asset among multiple metaverse applications. The service provider supports for access/download/modification/delete digital assets by the user or a group of users in multiple metaverses to ensure the consistency of the digital assets.

In KI#2, appropriate usage of user consent includes ensuring only the legitimate use by non-owners of user sensitive information such as digital assets are covered. However the focus of this KI is how to manage the permission of specific operation of a digital asset (e.g. assign the access right, revoke the deletion right, etc), including across multiple metaverses.

CAPIF specified in 3GPP TS 23.222 [14] provides Resource owner-aware Northbound API Access (RNAA) to control access to the owner’s resource. The Enhancement of RNAA is also studying by SA6 in Rel-19 FS_CAPIF_Ph3. Currently CAPIF focuses on the management of service API and cannot fulfill the requirements permission control of digital asset in the two use cases above.

### 4.9.2	Open issue

This key issue is going to study:

Whether and how to manage the permission control for digital asset, including in across multiple metaverses use case.

NOTE:	Operations include access, download, modification and deletion of digital assets.

# 5	Application enablement architecture requirements

## 5.1	General requirements

Following general requirements are identified:

[AR-5.1-a]	The application enabler service shall support management of spatial anchors.

[AR-5.1-b]	The application enabler service shall support management of digital avatars including create, read, update, delete avatar profiles and uploading and downloading the avatar media.

[AR-5.1-c]	The application enabler service shall support UE (supporting metaverse service) to offload computational process to another UE.

[AR-5.1-d]	The application enabler service shall support metaverse applications which requires multiple devices to serve the user.

## 5.2	Spatial anchors management service requirements

[AR-5.2-a]	The spatial anchor management service shall provide mechanism to create, update, get/discover spatial anchors.

[AR-5.2-b]	The spatial anchor management service shall provide mechanism to subscribe for spatial anchor related information including usage information, UEs within certain range.

## 5.3	Spatial map management service requirements

[AR-5.3-a]	The spatial map management service shall provide mechanisms to create, update/delete, get and subscribe spatial maps.

[AR-5.3-b]	The spatial map management service shall provide mechanisms to expose spatial map information to authorized third parties.

## 5.4	Digital asset and avatar management service requirements

[AR-5.4-1]	Subject to operator policy, regulatory requirements and user consent, the metaverse enablement service shall provide digital asset management mechanisms as follows:

-	to create, update, retrieve, delete and discover digital assets securely.

-	to manage associations between digital assets and user identifiers.

-	to allow an authorized third party to manage digital asset(s) associated with a user.

-	to differentiate between  multiple types of digital assets, e.g. avatars, tokens, licenses.

-	to manage sets of related digital assets jointly (e.g. as blocks of digital assets).

[AR-5.4-2]	The metaverse enablement service shall provide mechanisms to create, update, get/discover avatars as digital assets.

# 6	Application enablement architecture for metaverse services

## 6.1	Option #1: On-network mobile metaverse application layer architecture

This clause proposes a mobile metaverse enablement layer as a possible architecture.

### 6.1.1	Application enablement architecture

Figure 6.1.1-1 illustrates the detailed on-network mobile metaverse application layer functional model.

Figure 6.1.1-1: On-network mobile metaverse application layer functional model

The mobile metaverse application layer functional entities are grouped into the vertical application layer and the mobile metaverse application enablement layer. The mobile metaverse application enablement layer offers the mobile metaverse application enablement capabilities to the vertical application layer. The mobile metaverse application layer functional model utilizes SEAL services as specified in 3GPP TS 23.434 [8].

The mobile metaverse application enablement layer functional entities on the UE and in the network are respectively the mobile metaverse enabler client (MMEC) and mobile metaverse enabler server (MMES). The MMEC communicates with the MMES over the MM-UU reference point. The MMEC provides functionality to the VAL client(s) over the MM-C reference point. The VAL server(s) communicate with the MMES over the MM-S reference point. The MMES communicates with other MMES instances over the MM-E reference point. The MMES communicates with the underlying 3GPP network system using 3GPP interfaces specified by the 3GPP network system.

### 6.1.2	Functional Elements

#### 6.1.2.1	Mobile Metaverse Enablement Server

A Mobile Metaverse Enablement Server (MMES) provides metaverse application enablement services used by Metaverse VAL servers and Mobile Metaverse Enablement Clients (MMEC).

#### 6.1.2.2	Mobile Metaverse Enablement Client

A Mobile Metaverse Enablement Client provides metaverse application enablement services to VAL clients.

### 6.1.3	Reference Points

#### 6.1.3.1	MM-UU

MM-UU reference point enables interactions between a MMES and MMEC.

#### 6.1.3.2	MM-S

MM-S reference point enables interactions between a MMES and Metaverse VAL servers.

#### 6.1.3.3	MM-E

MM-E reference point enables interactions between MMES instances.

#### 6.1.3.4	MM-C

MM-C reference point enables interactions between a MMEC and Metaverse VAL clients.

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [8].

Figure 6.2.1-1: A-DACM architecture to support Metaverse services using SEAL architecture

The SEAL architecture includes A-DACM client and server which supports the VAL layer. The A-DACM server interacts with the VAL server(s) over the A-DACM-S reference point. The A-DACM client interacts with the VAL client(s) over the A-DACM-C reference point. The interactions between A-DACM client and A-DACM server is ove the A-DACM-UU reference point. The A-DACM server may utilize the 5GC services over N33 reference point. The A-DACM client may be located within the VAL client(s) in which case the A-DACM-C reference point will be internal to VAL client(s). The service APIs offered by A-DACM server on A-DACM-UU and A-DACM-S to the VAL layer functions (via A-DACM client) follow the service-based architecture as shown in the figure 6.2.1-3.

Figure 6.2.1-2 illustrates the A-DACM architecture to support Metaverse services using CAPIF architecture specified in 3GPP TS 23.222 [14].

Figure 6.2.1-2: A-DACM architecture to support Metaverse services using CAPIF architecture

The SEAL architecture includes A-DACM server which supports the VAL layer. The VAL server(s) (acting as CAPIF’s API invoker) interact with the A-DACM server (acting as CAPIF’s AEF) over the A-DACM-S reference point (compliant with CAPIF-2/2e). The VAL client(s) (acting as CAPIF’s API invoker) interact with the A-DACM server (acting as CAPIF’s AEF) over the A-DACM-VC reference point (compliant with CAPIF-2/2e). The service APIs offered by A-DACM server on A-DACM-VC and A-DACM-S to the VAL layer functions follow the service-based architecture as shown in the figure 6.2.1-3. The procedures of CAPIF as specified in 3GPP TS 23.222 [14] apply for the interactions between VAL layer and A-DACM server. The A-DACM server may utilize the 5GC services over N33 reference point. The A-DACM client may be located within the VAL client(s) in which case the A-DACM-UU reference point and A-DACM-VC reference points are same.

Figure 6.2.1-3 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-2: Service-based SEAL architecture to support Metaverse services

The A-DACM server exposes the APIs over the Sdacm interface to the VAL layer. The VAL functions (VAL clients and servers) and the A-DACM client (serving a VAL client) consume the services by interacting on the Sadacm interface of the A-DACM server. The CAPIF can be used by VAL functions to discover the services of the A-DACM server.

NOTE:	The A-DACM function can be realized as a new SEAL service or part of existing SEAL service.

### 6.2.2	Functional Elements

#### 6.2.2.1	A-DACM function

The A-DACM function expose the services/APIs to the VAL layer services over Sdacm interface for the following avatar management capabilities:

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

### 6.2.3	Reference Points

#### 6.2.3.1	SEAL-S

This reference point supports the interactions between VAL server(s) and A-DACM function. The VAL server consumes the avatar management services supported by the A-DACM function as specified in clause 6.2.2.1.

### 6.2.4	Service-based interface

#### 6.2.4.1	Sdacm

This interface supports the interactions between VAL function (VAL servers) and A-DACM function. The VAL function consumes the avatar management services supported by the A-DACM function as specified in clause 6.2.2.1.

## 6.3	Option #3: Reuse existing SEAL architecture for metaverse services

### 6.3.1	Application enablement architecture

The SEAL architecture as defined in 3GPP TS 23.434 [8] is reused to enhance to support metaverse services, e.g. SEAL LM enhancement to support spatial anchor management and spatial map management, SEAL CM enhancement to support avatar profile(s) management. The SEAL clients are enhanced to provide client side functionalities for metaverse application. The SEAL servers are enhanced to provide server side functionalities for metaverse applications.

## 6.4	Option #4: Architecture to support Metaverse services with permission control

### 6.4.1	Application enablement architecture

Figure 6.4.1-1 illustrates a potential architecture to support Metaverse services.

Figure 6.4.1-1: Architecture to support Metaverse services

DAPM function provides functionality to the VAL server(s) over the PM-V reference point. DAMP function communicates with A-DACM function over CM-PM reference point. The DAPM function communicate with the UE DAPM function over the PM-C reference point. DAPM supports to provide the service by using CAPIF architecture.

Editor’s Note: Whether DAPM is a new functionality or enhanced other SEAL/CAPIF server is FFS.

Editor’s Note: Whether CM-PM is internal interface or external interface is FFS.

Editor’s Note3: Whether and how to modify/enhance option #4 to adjust SBA is FFS.

### 6.4.2	Functional Elements

#### 6.4.2.1	DAPM function

The DAPM function provides the services to VAL layer for the permission management of multiple operation rights of digital assets. The main capabilities as following:

-	create, update, record, check, delete the operation permission information of digital asset, the information includes digital assets identity, authorized operation, time limit;

-	authorize multiple operation rights of digital asset to user, the operation includes access, update/modification, delete etc.;

-	Digital asset owner information storage and inquire the owner for authorization; and

-	permissions validation;

#### 6.4.2.2	UE DAPM function

UE DAPM function provides client service of digital asset permission management.

### 6.4.3	Reference Points

#### 6.4.3.1	PM-V

PM-V reference point enables interactions between VAL server(s) and DAPM function. The VAL server consumes the services supported by the DAPM function as specified in clause 6.2.2.1.

#### 6.4.3.2	CM-PM

CM-PM reference point enables interactions between DAPM function and A-DACM function.

#### 6.4.3.3	PM-C

PM-C reference point enables interactions between DAPM function and UE DAPM function.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 | KI #9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |  | X |  |
| Sol #5 |  |  | X |  |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |  |
| Sol #8 |  |  |  | X |  |  |  |  |  |
| Sol #9 |  |  | X |  | X |  |  |  |  |
| Sol #10 |  |  | X |  | X |  |  |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |  |
| Sol #13 |  |  |  |  |  |  | X |  |  |
| Sol #14 |  | X |  |  |  |  |  |  |  |
| Sol #15 |  |  |  | X |  |  |  |  |  |
| Sol #16 |  |  |  | X |  |  |  |  |  |
| Sol #17 |  |  |  | X |  |  |  |  |  |
| Sol #18 |  |  |  |  | X |  |  |  |  |

## 7.1	Solution #1: Spatial anchor discovery

### 7.1.1	Solution description

#### 7.1.1.1	General

This solution is for KI#1 and addresses the open issue of spatial anchor discovery by the consumer.

This solution is based on the Mobile Metaverse enablement layer architecture.

In this solution, the mobile metaverse application enablement layer provides capabilities to discover and determine the validity of spatial anchors. Spatial anchors may be discovered and validated based on consumer location and orientation, consumer capabilities and preferences, and spatial anchor configuration and policy.

#### 7.1.1.2	Procedure

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client (e.g., an AR application) needs to display enriched localized content to a user

3) The spatial anchor specific information (e.g. the description or label of spatial anchor) has been provisioned in the mobile metaverse enabler server.

Figure 7.1.1.2-1: Spatial anchor discovery procedure

1.	A metaverse VAL client discover s spatial anchors. The VAL client performs spatial anchor discovery using the mobile metaverse enabler client (MMEC) of the UE. The VAL client provides spatial anchor discovery requirements to the MMEC to discover spatial anchors of interest. The discovery requirements include a VAL client identifier, a VAL client type, information on location(s) of interest, and information on service(s) of interest (e.g., service type, service provider identifier, time of day when service is requested, etc.) for the service associated with spatial anchors, and indication to enable persistent search. The request may include discovery filter including the spatial anchor specific information (e.g. the user preference information). Additionally, the discovery filter also includes universal discoverable visibility requirement. The universal discoverable visibility defines the spatial anchor is discoverable for any user.

NOTE 1: The requirements for spatial anchor discovery may be further defined in the normative phase.

2.	The MMEC sends a spatial anchor discovery request to the mobile metaverse enabler server (MMES). The request includes information elements defined in Table 7.1.3-1. The request may include discovery filter including the spatial anchor specific information (e.g. the user preference information).

3.	Upon receiving the request, the MMES validates if the requestor is authorized to discover spatial anchors. If the requestor is authorized, the MMES uses the information included the spatial anchor discovery request to determine spatial anchors to include in the discovery response.

If spatial anchor discovery filters are provided in the request, the MMES determines any spatial anchors matching the discovery filters. For each determined spatial anchor, the MMES checks whether the requestor is authorized to discover the spatial anchor based on the identity and location of the requestor and the access controls and service area characteristics of the spatial anchor. To determine a location of interest to the requestor, the MMES may use location information in the request, if available, or obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8]. Additionally, the MMES may obtain and consider data analytics, such as UE mobility prediction, during spatial anchor determination. The MMES may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [10] and 3GPP TS 23.502 [9].

The MMES determines the matched spatial anchors based on the spatial anchor specific information (e.g. user preference information in step 1) and the details of spatial anchor stored in MMES server (e.g. the description or label of spatial anchor obtained in pre-condition).

4.	The MMES sends a spatial anchor discovery response to the MMEC. If the MMES has successfully determined spatial anchors, the response may include a success indication, a list of the determined spatial anchors as defined in Table 7.1.3-2. If the MMES has not determined any spatial anchor, the response may include a failure indication and a cause of failure. If the request includes the Group SA ID then the MMES provides all the SA information associated for the requested group SA ID in the response. The response also includes the recommended SA in the discovery response based on the discovery filter information and stored spatial anchor information. If universal discoverable visibility indicated in request discovery filter, then the Discovered spatial anchor List in the response corresponds to Discovered universal spatial anchor List.

NOTE 2:	The logic to generate the recommended list of spatial anchor is out of scope and implementation-specific.

5.	Upon receiving the discovery response, if the response indicates success, the MMEC may store the received spatial anchor information in a spatial anchor cache. The MMEC evaluates the validity conditions of discovered spatial anchors.

6.	The MMEC provides the valid spatial anchor information to the VAL client(s) based on VAL client requirement(s). Upon receiving valid spatial anchor information, the VAL client can access the service(s) associated with each spatial anchor.

NOTE 3:	This procedure may be adapted to allow a VAL server to discover spatial anchor(s).

#### 7.1.1.3	Creating spatial anchor group

##### 7.1.1.3.1	General

Consider a scenario where the user wearing the headset is in the supermarket or mall. The mall could have many shops(like clothes, vintage items), restaurants, etc. The VAL server can create a group of spatial anchors within the shop and represent the shop with a grouped spatial anchor. Similarly it can create a group of spatial anchors like an Italian food spatial anchor, vegan food spatial anchor, etc. From outside or at a certain distance, the user can discover the high-level overview of spatial anchor indications (like a single spatial anchor representing a group of clothes, etc. rendered in the user's headset).

##### 7.1.1.3.2	Procedure

Figure 7.1.1.3.2-1 depicts the procedure for creating the group of spatial anchor services linked to each other. The service is provided by MMES and consumed by VAL server or MMEC.

Figure 7.1.1.3.2-1: Create spatial anchor group

1)	The VAL server (or MMEC) sends a request message to the MMES server to create a group of spatial anchors. The request includes VAL server identity, spatial anchor identity list and spatial anchor service area, security credentials.

NOTE: The group spatial anchor provides an abstraction for the underlying linked spatial anchor(s), which enables to discovery of the linked spatial anchors as per the request of the user or based on the location, etc.

2)	The MMES server authorizes VAL server (or MMEC client). If the requestor is authorized, then the MMES server creates a group spatial anchor (along with its identity) for the linked spatial anchors for the given spatial anchor service area. The MMES sends the response back to the VAL server (or MMEC client) including the spatial anchor (along with its identity) of the spatial anchor for a successful case.

### 7.1.2	Architecture Impacts

Option #1 in clause 6.1 is the basis for this solution.

### 7.1.3	Corresponding APIs

#### 7.1.3.1	Overview

This clause provides a summary of the corresponding APIs for solution #1.

-	Spatial anchor discovery request API (request / response model; API provider: MMES; known consumers: MMEC; corresponding to step 1 to 3 of clause 7.1.1.2).

-	Spatial anchor discovery response API (request / response model; API provider: MMES; known consumers: MMEC; corresponding to step 1 to 3 of clause 7.1.1.2).

-	Create spatial anchor group request API (request / response model; API provider: MMES; known consumers: VAL server; corresponding to step 1 to 2 of clause 7.1.1.3).

-	Create spatial anchor group response API (request / response model; API provider: MMES; known consumers: VAL server; corresponding to step 1 to 2 of clause 7.1.1.3).

#### 7.1.3.2	Information flows

Table 7.1.3.2-1 shows the request sent by a MMEC to a MMES for the discovery of spatial anchor(s).

Table 7.1.3.2-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Location of interest | O | Location of spatial anchors the requestor is interest in. |
| Spatial anchor discovery filters | O | Set of characteristics to determine matching spatial anchors (e.g., spatial anchor location, VAL client type, universal filter). |
| Current UE location | O | Current location of the UE |
| Current UE pose | O | Current pose of the user (as defined in clause 6.2.2 of 3GPP TS 26.119 [11]) |
| SA Group ID | O | Indicates the identifier of the requested group of SA |

Table 7.1.3.2-2 shows the response sent by the MetaApp server to the requester for a spatial anchor discovery request.

Table 7.1.3.2-2: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered spatial anchor List | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Spatial anchor location | M | Location of the spatial anchor (e.g., GPS coordinates, civic address, etc.). |
| > Spatial anchor validity criteria | M | The validity criteria of a spatial anchor (e.g., spatial criteria, time criteria, etc.). |
| > VAL server endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) of the service(s) associated with the spatial anchor. |
| > VAL server information | O | Information about the service offered by the VAL server (e.g., service description, service name, etc.). |
| Group spatial anchor information | O See NOTE | Indicates the grouped spatial anchor ID and details of each spatial anchor within the group |
| Recommended spatial anchor List | O | List of recommened spatial anchor(s) and associated details like location and SA ID’ |
| NOTE: This IE is present only if SA group ID is present |  |  |

### 7.1.4	Solution evaluation

This solution is for KI#1 and addresses the open issue of spatial anchor discovery by the consumer. This solution is based on the Mobile Metaverse enablement layer architecture as specified in clause 6.1. The solution proposes MMEC to send discovery request with appropriate discovery filters. The MMES discovers the spatial anchors based on the provided discovery filters and send the response. The solution assumes that spatial anchors are created at MMES by the application service provider. The solution receives location information from NEF or SEAL LMS. The solution is a feasible solution. The solution will be adapted to a SEALarchitecture for managing spatial anchors where the responsibilities of MMEC and MMES can map to SEAL client and SEAL server. The solution allows the consumers to discover the grouped spatial anchors, and discover recommended spatial anchors and universal spatial anchors.

## 7.2	Solution #2: Support for spatial anchor management

### 7.2.1	Solution description

This solution maps to KI# 1. This solution enhances Service Enabler Architecture Layer (SEAL) Location Management (LM) service to provide spatial anchor management service to the VAL server or the UE. The architecture impacts are captured in clause 7.2.2 while procedures to manage spatial anchors are captured in clause 7.2.3.

### 7.2.2	Architecture Impacts

This clause provided architecture enhancements to the SEAL LM architecture as defined in 3GPP TS 23.434 [8]. The SEAL LM client is enhanced to provide client side functionalities for managing (i.e. create, read, update and delete) spatial anchors for metaverse application. The SEAL LM server is enhanced to provide server side functionalities for managing spatial anchors for the metaverse applications.

VAL client uses the SEAL LM client to manage spatial anchors for the metaverse applications over LM-C reference point. Similarly, VAL server uses SEAL LM server to manage spatial anchors for the metaverse applications over LM-S reference point. The LM-UU interface is enhanced to provide management of spatial anchors between SEAL LM client and SEAL LM server.

### 7.2.3	Procedures

#### 7.2.3.1	Creating spatial anchor

Consider a supermarket example, where a user with a headset discovers  multiple spatial anchor(s). Some spatial anchor providers can mark their spatial anchors visibility level as discoverable for all users during creation process. During the discovery process, the user also discovers spatial anchors provided by different spatial anchor metaverse providers (e.g. spatial anchors like emergency exit SA, medical SA etc.) which are marked as universal.

Figure 7.2.3.1-1 depicts the procedure for creating association of spatial anchor with location for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.1-1: Create spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to create association of the spatial anchor with the location. The request includes VAL server identity, position of the anchor and application service identifier, security credentials, spatial anchor service area. The request may include service information of the product to associate it with the spatial anchor, access control rules defining which entities are permitted to discover and access the spatial anchor, and customer premise information (e.g. a residence, office or shop). The request also includes spatial anchor discoverable visibility levels like universal to facilitate shared spatial anchor discovery. The universal discoverable visibility level defines the spatial anchor is discoverable for any user.

NOTE: Terminologies and further usage of visibility level will be reconsidered during the normative work.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server creates the association between spatial anchor and the location, and stores the information as received in the request message. The server creates identity for the spatial anchor. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including identity for the spatial anchor for a successful case. The SEAL LM server may create the spatial anchor map based on the spatial anchor information in step 1 and the stored or retrieved spatial mapping information from the spatial map server.

NOTE 1: This solution of adding spatial anchors into spatial map can be adapted using concluded architecture option.

NOTE 2: This solution of adding spatial anchors into spatial map can be adapted to other spatial anchor management procedure.

NOTE 3: Whether spatial anchors can be added as a layer to spatial map or not will be considered in the normative work.

#### 7.2.3.2	Updating spatial anchor

Figure 7.2.3.2-1 depicts the procedure for updating spatial anchor association for metaverse applications. The update operation includes modifying existing spatial anchor or deleting the spatial anchor. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.2-1: Update spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update the existing spatial anchor association. The request is used to modify or delete the spatial anchor. The request message includes the identity of the spatial anchor and the application service identifier. To modify the spatial anchor, the request may include service information of the product to associate it with the spatial anchor, spatial anchor service area and access control rules, and customer premise information (e.g. a residence, office or shop). The request also includes spatial anchor discoverable visibility levels like universal or non-universal to facilitate shared spatial anchor discovery.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server checks whether the spatial anchor as identified by the identity of the spatial anchor exists or not. If the spatial anchor does not exists then the SEAL LM server sends failure response. Otherwise, the SEAL LM server updates the spatial anchor details (for update request) or deletes the spatial anchor details (for delete request). The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including reason for failure for failure case.

#### 7.2.3.3	Get spatial anchor

Figure 7.2.3.3-1 depicts the procedure for getting spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.3-1: Get spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial anchor details. The request includes discovery filters like the position of the user and application service identifier. If the request to get specific spatial anchor, the request may include application specific information (e.g. type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server determines all spatial anchors from the repository based on the discovery filters provided in the request message. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including list of spatial anchor details for successful case.

#### 7.2.3.4	Spatial anchor information subscription

Figure 7.2.3.3-1 depicts the procedure for subscribing to receive spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.4-1: Spatial anchor information subscription

1)	The VAL server (or SEAL LM client) sends a subscription request to the SEAL LM server to get notifications about the spatial anchor details. The request includes discovery filters like the position and application service identifier. If the request to subscribe to specific spatial anchor, the request may include application specific information (e.g. type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server sends successful response along with subscription identity.

NOTE 1:	The SEAL LM server starts monitoring the location of the user if it is not already monitoring.

3-4)	Upon occurrence of the event (e.g. spatial anchor added or removed or updated), the SEAL LM server determines all spatial anchors from the repository which are matched with the discovery filters provided in the request. The SEAL LM server sends the notification to the VAL server (or SEAL LM client) including list of spatial anchor details. The response may include the recommended list of spatial anchor details based on the requested spatial anchor discovery filter, application specific information (e.g. type of service information) and the stored spatial anchor(s).

NOTE: The logic to provide a recommended list of spatial anchor details is implementation-specific and out of scope

### 7.2.4	Solution evaluation

This solution maps to KI# 1. This solution enhances Service Enabler Architecture Layer (SEAL) Location Management (LM) service by enhancing SEAL LM client and SEAL LM server functionalities as specified in clause 7.2.2. The solution proposes to create, update and get spatial anchors. The solution also proposes to subscribe for spatial anchors. The solution is a feasible solution. The solution will be adapted to a SEAL architecture for managing spatial anchors where the enhancements proposed to SEAL LMC and SEAL LMS can map to spatial anchors SEAL client and SEAL server.

## 7.3	Solution #3: Spatial anchor subscription

### 7.3.1	Solution description

#### 7.3.1.1	General

This solution is for KI#1 and addresses the open issue of how to detect UE(s) within a certain range of a spatial anchor and detect spatial anchors within a certain range of a UE.

This solution is based on the Mobile Metaverse enablement layer architecture.

A first procedure is proposed to allow a VAL server to subscribe for information about UE with spatial anchor interest. The mobile metaverse application enablement layer provides capabilities to subscribe for notifications providing information on UE with spatial anchor interest and that may enter, exit or be in range of spatial anchor of interest. The notification may include information about UEs that have discovered spatial anchor(s) but may not yet have connected to the service associated with the spatial anchor(s).

A second procedure is proposed to allow a UE (e.g., MMEC, VAL client) to subscribe for information about spatial anchors of interest. The mobile metaverse application enablement layer provides capabilities to subscribe for notifications providing information on spatial anchor interest and that may be in range, become in range or become out of range of the UE.

#### 7.3.1.2	VAL server subscription procedure for information about UE with spatial anchor interest

This procedure goes beyond capabilities defined in SEAL LMS for providing UEs within range of a target location described in 3GPP TS 23.434 [8]. SEAL LMS provides information about UE(s) within range of a location, which may be the location of a spatial anchor, regardless of the interest of such UE(s) for spatial anchors. This solution adds spatial anchor awareness such that UE(s) that consume spatial anchors and have discovered spatial anchors of interest are considered. Spatial anchor awareness allows a VAL server (e.g., that may have provisioned spatial anchors or may offer a service associated with a spatial anchor) to discover UE(s) with interest in the associated spatial anchors. SEAL LMS and this solution can be used in conjunction by a VAL server.

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL server requires information about UEs with spatial anchor interest.

Figure 7.3.1.2-1: UE with spatial anchor interest subscription/notification procedure

1.	A metaverse VAL server sends a UE with spatial anchor interest subscription request to the mobile metaverse enabler server (MMES). The request includes the VAL server identifier, security credentials, a VAL server notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor of interest detection filters. The detection filters may include spatial anchor identifiers to detect UEs that have discovered specific spatial anchors, spatial anchor associated service identifiers to detect UEs that have discovered spatial anchors associated with a service identifier, location area to detect UEs that have discovered spatial anchors within a location area, spatial anchor range to detect UEs that have discovered and are within range or become out of range of spatial anchors. The subscription may also indicate whether the notification is for a UE entering the range, leaving the range or both.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and performs the necessary actions to detect UE with spatial anchor interest. To detect UEs entering or leaving the range of spatial anchor of interest, the MMES considers if the spatial anchors of interest have been discovered by a UE and obtains the corresponding UE location information. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction. The MMES may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [10] and 3GPP TS 23.502 [9]. The MMES may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

3.	The MMES sends a UE with spatial anchor interest subscription response to the VAL server. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

4.	Based on the detection filters provided in the subscription request, the MMES detects UE(s) with interest in spatial anchors (e.g., UE that discovers the monitored spatial anchor), and identifies whether the UE enters or exits the monitored spatial anchor range.

5.	The MMES sends a UE with spatial anchor interest notification to the subscribed VAL server; the notification includes the subscription identifier and information about the detection of UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

#### 7.3.1.3	VAL client subscription procedure for spatial anchors in range

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client requires information about spatial anchors of interest

Figure 7.3.1.3-1: Spatial anchor of interest subscription/notification procedure

1.	A metaverse VAL client subscribes for spatial anchor of interest information with the mobile metaverse enabler client (MMEC), and the subscription includes a VAL client identifier and spatial anchor requirements. The spatial anchor requirements includes spatial anchor of interest discovery filters and a range. The detection filters may include spatial anchor identifiers to detect specific spatial anchors, spatial anchor associated service identifiers to detect spatial anchors associated with a service identifier, location area to detect spatial anchors within a location area, range to detect spatial anchors within a range, an indication to enable persistent search to notify spatial anchors within range of UE’s location, the current pose of the user along with direction of the movement, optionally spatial anchor specific information. The subscription may also indicate whether notifications are for spatial anchors becoming in range of the UE, becoming out of range of the UE or both.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a product name, a URL, a media data, a product type, a manufacturing company, a date, an expiry date, a ratings, etc. Such information may be stored in key-value pair and is transparent to the mobile metaverse enabler.

2.	The MMEC sends a spatial anchor of interest subscription request to the mobile metaverse enabler server (MMES). The request includes the MMEC identifier, VAL client identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and discovery filters provided by the VAL client.

3.	Upon receiving the request, the MMES authorizes the MMEC subscription request and performs the necessary actions to detect spatial anchor of interest. To detect spatial anchor of interest within range or becoming in-range or out-of-range of the UE, the MMES starts monitoring the corresponding UE location and pose information of the requestor. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction. The MMES may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [10] and 3GPP TS 23.502 [9]. The MMES may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs becomes in-range or out-of-range of the spatial anchors of interest.

4.	The MMES sends a spatial anchor of interest subscription response to the MMEC. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

5.	Based on the discovery filters provided in the subscription request, the MMES monitors the distance between the current UE location and the spatial anchor of interest and identifies the list of spatial anchors which are within range of the UE..

6.	The MMES sends a spatial anchor of interest notification to the subscribed MMEC; the notification includes the subscription identifier and information about the detection of spatial anchors of interest (e.g., spatial anchor identifier(s), range from the UE, etc.).

7-	The MMEC provides to the VAL client the information received in the notification about the spatial anchors of interest.

### 7.3.2	Architecture Impacts

Option #1 in clause 6.1 is the basis for this solution.

### 7.3.3	Corresponding APIs

#### 7.3.3.1	Overview

This clause provides a summary of the corresponding APIs for solution #3.

-	Discover UE with Spatial Anchor interest API (subscribe / notify model; API provider: MMES; known consumers: VAL server; corresponding to step 1 to 5 of clause 7.3.1.2).

-	Discover Spatial Anchor API (subscribe / notify model; API provider: MMEC; known consumer: VAL client; corresponding to step 1 and 7 of clause 7.3.1.3).

-	Discover Spatial Anchor API (subscribe / notify model; API provider: MMES; known consumer: MMEC; corresponding to step 2 to 6 of clause 7.3.1.3).

#### 7.3.3.2	Information flows

Table 7.3.3,2-1 shows the request sent by a MMEC to a MMES for the UE with SA interest subscription request.

Table 7.3.3.2-1: UE with Spatial Anchor interest subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Notification endpoint | M | Endpoint where notification is received |
| Spatial anchors discovery filter | M | Discovery filters to identify UEs |
| > Spatial anchor identifier(s) | M | Identifier of the spatial anchors that have discovered by the UE |
| > Area of interest | O | Location area of interest within which UEs have discovered spatial anchors |
| > Application specific range | O | Application specific range within which UEs have discovered spatial anchors |
| > Event | O | UE Entering the spatial anchor range, UE leaving the spatial anchors range |

Table 7.3.3.2-2 shows the response sent by the MMES server to the requester for a spatial anchor discovery response.

Table 7.3.3.2-2: UE with Spatial Anchor interest subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Subscription identity | O | Identify of the subscription (for successful case) |
| Failure cause | O | Indicates reason for the failure |

Table 7.3.3.2-3 shows the notification sent by a MMES server to the requester for the UE with SA interest notification.

Table 7.3.3.2-1: UE with Spatial Anchor interest notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identify of the subscription (for successful case) |
| List of spatial anchors | M | UE details for each spatial anchors |
| > List of UE(s) | O | List of UEs entering or leaving the spatial anchors range |
| >> event type per UE | O | Indicating UEs entering or leaving the spatial anchors range |

Table 7.3.3.2-4 shows the request sent by a MMEC to a MMES for the SA of interest subscription request.

Table 7.3.3.2-4: Spatial Anchor of interest subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Notification endpoint | M | Endpoint where notification is received |
| Spatial anchors discovery filter | M | Discovery filters to identify spatial anchors |
| > Spatial anchor identifier(s) | M | Identifier of the spatial anchors to detect specific spatial anchors |
| > Area of interest | O | Location area of interest within which spatial anchors are interested |
| > Application specific range | O | Application specific range from UE within which spatial anchors have discovered spatial anchors |
| > current UE location | O | Current location of the UE |
| > current pose | O | Current post of the user (as defined in clause 6.2.2 of 3GPP TS 26.119 [11]) |
| > spatial anchor specific information | O | Spatial anchor specific information which is transparent to the enabler layer. |

Table 7.3.3.2-5 shows the response sent by the MMES server to the requester for a spatial anchor discovery response.

Table 7.3.3.2-5: Spatial Anchor of interest subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Subscription identity | O | Identify of the subscription (for successful case) |
| Failure cause | O | Indicates reason for the failure |

Table 7.3.3.2-6 shows the notification sent by a MMES server to the requestor for the UE with SA interest notification.

Table 7.3.3.2-6: Spatial Anchor of interest notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identify of the subscription (for successful case) |
| List of spatial anchors | M | List of spatial anchors within range of the UE |

### 7.3.4	Solution evaluation

This solution addresses Key Issue #1 by providing spatial anchor subscribe/notify procedures; a first procedure allows the VAL server to subscribe for information about UEs with spatial anchor interest, and a second procdure allows for a consumer UE to subscribe for information about spatial anchors of interest.

The solution is based on a mobile metaverse application enablement layer architecture with an enabler client (MMEC) and an enabler server (MMES). The solution is not dependent on the "layer" architecture; thus, capabilities of the MMEC and MMES may alternatively be provided respectively by a SEAL client and SEAL server of a SEAL service (e.g., new or enhanced).

The solution enables an authorized VAL server to discover UEs with interest in specific spatial anchor by providing detection filters in a subscription request to the MMES. The MMES uses the detection filters to identify UEs that match the detection criteria (e.g., have discovered specific spatial anchors), and to notify the VAL server about the identified UEs.

The solution enables an authorized VAL client (e.g., via the MMEC) to discover spatial anchors of interest based on persistent search by providing interest discovery filters in a subscription request to the MMES. The service enablers use the detection filters to identify spatial anchors that match the detection criteria, and to notify the MMEC about the identified spatial anchors.

The solution depends on other solutions for support of spatial anchor registration.

The solution is feasible. The solution will be adapted to a SEAL architecture for managing spatial anchors where the responsibilities of MMEC and MMES can map to SEAL client and SEAL server.

## 7.4	Solution #4: Support for spatial anchor analytics information

### 7.4.1	Solution description

This solution maps to KI# 8.

### 7.4.2	Procedures

#### 7.4.2.1	Spatial anchor analytics information (subscription-notify)

Figure 7.4.2.1-1 depicts the procedure for subscribing to receive spatial anchor analytics information for metaverse applications. The service is provided by MMES server and consumed by VAL server.

Figure 7.4.2.1-1: Spatial anchor analytics information subscription

1.	A metaverse VAL server sends a spatial anchor analytics subscription request to the mobile metaverse enabler server (MMES). The request can include the VAL server identifier, security credentials, a VAL server notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor analytics filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and analytics sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and subscribes the 5GS NEF services, NWDAF, ADAE services for collecting analytics as per the analytics filter. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction, application session analytics. The MMES may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [10] and 3GPP TS 23.502 [9].

The MMES sends a spatial anchor analytics subscription response to the VAL server. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

3.	The MMES generates the spatial anchor-related analytics based on the obtained data and the stored spatial anchor data as per the analytics sampling interval. It provides response to the VAL server as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

NOTE1:	Whether Metaverse enabler layer can perform spatial anchor analytics or ADAES needs to be enhanced for that, can be decided during normative work.

NOTE2:	The procedure can be modified during the normative work.

### 7.4.3	Architecture Impacts

Option #1 in clause 6.1 is the basis for this solution.

### 7.4.4	Corresponding APIs

#### 7.4.4.1	Overview

This clause provides a summary of the corresponding APIs for solution #4.

-	Spatial anchor analytics information subscription request API (subscribe / notify model; API provider: MMES; known consumers: VAL server; corresponding to step 1 to 3 of clause 7.4.2.1).

-	Spatial anchor analytics information notification API (subscribe / notify model; API provider: MMES; known consumers: VAL server; corresponding to step 1 to 3 of clause 7.4.2.1).

### 7.4.5	Solution evaluation

This solution addresses Key Issue #8(open issue# 2) by providing spatial anchor analytics information procedures based on MMES. This solution enables the VAL server to request the analytics related to the spatial anchor. The solution has no additional requirement to the core network.

## 7.5	Solution #5: Support for digital avatars

### 7.5.1	Solution description

Metaverse is considered as a digital world which is a replica of a real world. Most of the Metaverse application needs avatar for the user to interact with the application. And for each application, user may required to create, modify, get and delete avatars. For each avatar (regardless of application) will have some common properties. Depending on the metaverse application from where the user is interested to take the service, he/she can choose his/her avatar and the related information when needed. Also, a user can move between metaverse applications using the same avatar seamlessly and taking into account the constraints of the visited application.

This solution maps to KI# 3. This solution provides support to manage digital avatars for the users.

### 7.5.2	Architecture Impacts

This clause provided architecture enhancements to the SEAL CM architecture as defined in 3GPP TS 23.434 [8]. The SEAL CM client is enhanced to provide client side functionalities for managing (i.e. create, read, update and delete) application specific avatar profile(s) for metaverse application. The application specific avatar profile(s) contains avatar related properties and configurations.

The SEAL CM server is enhanced to provide server side functionalities for managing application specific avatar profile(s) for the metaverse applications. It allows the consumer of the service to create, update, fetch, and delete the application specific avatar profile(s). The application specific avatar profile(s) can be stored into VAL user database.

VAL client uses the SEAL CM client to manage application specific avatar profile(s) for the metaverse applications over CM-C reference point. Similarly, VAL server uses SEAL CM server to manage application specific avatar profile(s) for the metaverse applications over CM-S reference point. The CM-UU interface is enhanced to provide management of application specific avatar profile(s) between SEAL CM client and SEAL CM server.

A user can have one or more application specific avatar profile(s). An application specific avatar profile can be shared by one or more users (for example, an avatar profile is created for a sales representative, and used by all sales personals).

### 7.5.3	Procedures

#### 7.5.3.1	Create application specific avatar profile

The procedure for creating avatar profile is illustrated in figure 7.5.3.1-1. The configuration management server exposes the API for the consumer to create the avatar profile(s). The application specific avatar profile(s) may be created based on user’s request via configuration management client or based on request from the VAL server.

The application specific avatar profile(s) may be associated with a single user or with multiple users. For application specific avatar profile(s) which can be used by multiple users, the users can use the avatar at a same time simultaneously or one at a time depending on the application requirement. The configuration management server stores the avatar and its related properties and configuration.

Editor’s Note: The term "User’s identity" in this document is to be clarified as either "User profile" or "User Identifier". The term "Avatar ID" in this document is to be disambiguated from "Avatar Profile ID".

The following assumptions and conventions also apply, subject to normative updates:

a)	If 5GC exposes a User Profile to the service enablement layer, it is identified by a User Profile ID and can include one or more 5GC User Identifiers and a list of linked subscriptions.

b)	Digital identifiers are unique in the context of the service enablement layer.

NOTE 1:It is to be determined in the normative phase whether and how digital identifiers can be globally unique, i.e. including VAL application layer

c)	Avatar profile identifier uniquely identifies an avatar profile in the service enablement layer. An avatar profile is an Asset type-specific profile (see Table 7.6.3-1).

NOTE 2:The relationship between the avatar profile identifier and digital asset identifier is to be determined in the normative phase.

In this study, User identity, Digital identity (of a user) and digital identifier (of a digital asset) are different terms per metaverse service provider.

a)	User’s identity identifies user.

b)	Digital identifier (of the digital asset) uniquely identifies the digital asset of the user. Each digital asset will have a digital identifier to uniquely identify the asset. For example, an Avatar is one of the digital assets associated with the user. So, digital asset identifier is the Avatar ID for an Avatar.

-	In some case, an asset can be only associated with a user (e.g. passport of a user) as a digital identity and such asset will have digital asset identifier.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.1-1: Creating application specific avatar profile

1.	The configuration management client sends a create avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and other parameters to create avatar profile. The security credentials include security token. The request may include allowed user list, current status (in use or not), current position, allowed application list, allowed locations, associated digital wallets, associated digital accessories, one or more predictive model, creation time, expiry time till when the avatar profile is valid, history information like at which locations avatar is used, which applications used the avatar profile, time of the use, etc.

Table 7.5.3.1-1: Create avatar profile request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Id | M | Identify of the requestor (VAL UE or VAL server). |
| Security parameters | M | Security parameters for authentication and authorization. |
| Application specific avatar profile (NOTE 2) | M | Application specific avatar profile |
| > avatar profile ID | O | Identifier of the avatar profile |
| > user ID (NOTE 3) (NOTE 4) | M | Identity of the user for which the avatar profile is created |
| > access control list | O | List of tuples indicating the requestors allowed to perform operations with this avatar and the allowed operation types. |
| > current status | O | Current status (in use or not) of the profile |
| > spatial conditions | O | List of spatial conditions (e.g. locations) where this avatar profile is allowed to be accessed |
| >> current location | O | Current location where the profile is being used |
| > allowed application list | O | List of application IDs which are allowed to use this avatar profile |
| > associated accessories | O | List of accessories (e.g. Hat, watch, shoes) which are purchased for this avatar profile |
| > list of predictive models (NOTE 1) | O | List of one or more predictive model for the user, in order to predict user’s behaviour. |
| > expiry time | O | Time until this avatar profile is valid |
| > History information | O | Usage history for this avatar profile |
| >> list of locations | O | List of locations where the avatar profile is used. |
| >> time of use | O | Time information when the avatar profile is used |
| >> list of applications | O | List of the application IDs which used this avatar profile. |
| NOTE 1: This IE is only applicable if the avatar profile is not shared. NOTE 2: At least one component IE is to be provided. NOTE 3: It is to be determined in the normative phase whether this IE is to be implemented as an enablement layer-specific identifier, as 5GC exposed user identifier, as a 5GC exposed user profile identifier, as VAL application-specific identifier, as a combination of the previous options, etc. Any 5GC relevant information is to be harmonized with SA2 user profile agreements, if any. NOTE 4: The status of this IE is to be determined in the normative phase based on a variety of considerations, e.g. requestor, whether and how access control specifies avatar ownership, etc. For CM Clients as requestors, the IE is to be mandatory. |  |  |

NOTE 3:	It is assumed that "user ID" IE in Table 7.5.3.1-1 provides linkage to 3GPP subscription identifiers (if exposed by 5GC to the service enablement layer), therefore no other subscription identifiers are currently specified in this table. It is to be determined in the normative phase whether and how application-specific user identifiers are linked to this IE.

NOTE 4:	Alignment of the application specific avatar profile and SA2 defined base avatar information will be considered during normative work.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server creates the resource to store the avatar profile. The configuration management server stores the profile as received in the request.

3.	The configuration management server sends create avatar profile response to the configuration management client. In case of success, the response also includes the identity of the avatar profile.

NOTE 5:	The identity of the avatar profile is the digital asset identifier for an Avatar.

#### 7.5.3.2	Update application specific avatar profile

The procedure for updating avatar profile is illustrated in figure 7.5.3.2-1. The configuration management server exposes the API for the consumer to update the avatar profile. The application specific avatar profile(s) may be updated based on user’s request via configuration management client or based on request from the VAL server. This procedure is also used to link more users to the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.1-1: Creating application specific avatar profile

1.	The configuration management client sends a update avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials, identity of the avatar profile and parameters to update avatar profile. The security credentials includes security token. The other parameters includes allowed user list, current status (in use or not), current position, allowed application list, allowed locations, associated digital wallets, associated digital accessories, one or more predictive model, creation time, expiry time till when the avatar profile is valid, historical information like at which locations avatar is used, which applications used the avatar profile, time of the use, etc.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity. If avatar profile is present, then the configuration management server updates the profile with the new parameters as provided in the request message.

3.	The configuration management server sends update avatar profile response to the configuration management client indicating success or failure of the procedure. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.3	Get application specific avatar profile

The procedure to get avatar profile is illustrated in figure 7.5.3.3-1. The configuration management server exposes the API for the consumer to get the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.3-1: Get application specific avatar profile

1.	The configuration management client sends a get avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and identity of the avatar profile.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity.

3.	The configuration management server sends get avatar profile response to the configuration management client indicating success or failure of the procedure. In case of success, the response includes avatar profile with all required parameters. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.4	Delete application specific avatar profile

The procedure to delete avatar profile is illustrated in figure 7.5.3.4-1. The configuration management server exposes the API for the consumer to delete the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.4-1: Delete application specific avatar profile

1.	The configuration management client sends a delete avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and identity of the avatar profile.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity. If the avatar profile is present then the configuration management server deletes the avatar profile.

3.	The configuration management server sends delete avatar profile response to the configuration management client indicating success or failure of the procedure. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.5	Application specific avatar profile(s) subscription

Figure 7.5.3.5-1 depicts the procedure for application specific avatar profile(s) subscription. The service is provided by SEAL CM server and consumed by VAL server or SEAL CM client.

Figure 7.5.3.1-1: Application specific avatar profile(s) subscription

1)	The VAL server (or SEAL CM client) sends a subscription request to the SEAL CM server to get notifications about the application specific avatar profile(s). The request includes identity of the avatar profile, user ID, filter criteria like the application specific information (e.g., VAL user location, VAL client type, type of service), event (e.g. application specific avatar profile created or removed or updated).

2)	The SEAL CM server authorizes VAL server (or SEAL CM client). If the requestor is authorized, then the SEAL CM server sends successful response along with subscription identity.

3-4)	Upon occurrence of the event (e.g. application specific avatar profile created or removed or updated), the SEAL CM server determines all application specific avatar profile(s) from the repository which are matched with the filter criteria provided in the request. The SEAL CM server sends the notification to the VAL server (or SEAL CM client) including list of application specific avatar profile.

### 7.5.4	Solution evaluation

This solution maps to KI# 3. This solution enhances Service Enabler Architecture Layer (SEAL) Configuration Management (CM) service by enhancing SEAL CM client and SEAL CM server functionalities as specified in clause 7.5.2. The solution proposes to create, update, get and delete avatar profile. The solution also proposes to subscribe for avatar profile. The solution is a feasible solution. The solution will be adapted to the chosen architecture (either new SEAL service or enhancing exising SEAL CM service).

## 7.6	Solution #6: Avatar support as Digital Asset

### 7.6.1	Solution description

The following clauses specify procedures and information flows for Key Issue #3 for support of avatars, as a type of digital asset managed using Metaverse Digital Representation Block (MDRB).

MDRB is an information block containing one or more related digital assets, information about the associated users and about the associated metaverse services. MDRB also includes MMEL policies and parameters for the handling of the set of digital representations stored and managed together as part of MDRB.

NOTE 1:	How to choose or assign the digital assets to be managed together as part of an MDRB is not in scope of this solution.

NOTE 2:	In the normative phase, MDRB may be implemented as the information block used by A-DACM (solution #7) for management of digital assets, e.g. MDRB can be renamed as DAC, and MMES can implement A-DACM or have access to an external A-DACM.

Preconditions:

1.	Digital representations are available at the VAL application layer.

Figure 7.6.1.1: MDRB support request procedure

Procedural steps:

1.	The VAL server sends a request to the corresponding MMES to enable MDRB support for the metaverse application functionality related to the digital representation created. The message(s) include information about the digital representation(s) to be supported, e.g. avatar.

NOTE 1:	In the normative phase, the step 1 request can map to one or more requests, e.g. create /update /delete.

2.	The MMES determines applicable MMEL parameters for the handling of the digital representation. The MMEL support information and parameters are associated with MDRB.

NOTE 2:	The MDRB stores information necessary for accessing and management of the associated digital assets and their exposure to/from the VAL application layer. This information is provided by the VAL layer or is derived based on the VAL layer request and policies.

Based on the information received from the VAL layer and on local policies, the MMES optionally determines to send one or more requests to the underlying 3GPP Network to determine information available in the 3GPP network for the MDRB.

NOTE 3:	The information available in the 3GPP network to be included as part of MDRB is to be determined in the normative phase and aligned with specifications from SA2, SA3 and SA4.

NOTE 4:	Depending on the architectural option chosen, this step relies on one of the following options: MMES can implement A-DACM or access an external A-DACM (e.g. as new or existing SEAL entity)..

3.	The MMES generates response and sends the message to the VAL Server.

### 7.6.2	Architecture Impacts

This solution is described using the metaverse enablement architecture option #1 in clause 6.1. Notes describe however how it can be implemented using other architectural frameworks, i.e. A-DACM functionality (as proposed in solution #7), or existing SEAL servers (as proposed in solution #5).

### 7.6.3	Corresponding APIs

Table 7.6.3-1 shows the MDRB support request from VAL Server to MMES.

Table 7.6.3-1: Request MDRB support

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| MDRB identifier | O | An identifier for the MDRB for which MMEL support is requested |
| List of Metaverse service provider identifiers | M | List of service providers for the Metaverse services |
| List of associated user information (NOTE 4) | M | List of user information as detailed below. |
| > 3GPP subscriber association information (NOTE 1, NOTE 3) | O | This information is used in MMEL for interactions with 5GS which require subscriber/and/or user information. |
| > Metaverse service user information (NOTE 3) | O | Application-level information related to one or more application users. The information may be anonymized, etc. to meet the metaverse application security requirements. This information may represent or be provided by a third party. |
| List of digital assets (NOTE 4) | M | List of digital asset parameters for MDRB management. |
| > Asset type | M | Choice of asset type, e.g. avatar, wallet, etc. |
| > Asset type-specific profile | M | Profile information specific to the asset, e.g. an avatar profile similar to Table 7.5.3.1-1. |
| > Asset profile information (NOTE 2) | O | Information used to obtain 5GS services, and which depends on asset type, metaverse use policies. For example, for an avatar asset, the information includes identifiers (e.g. digital asset identifier), etc necessary to utilize SA4 services |
| > Communication parameters | O | Communication parameters and requirements for sessions associated with the digital asset. The parameters are also used to create or derive the communication policies applicable to the asset which are persistent within metaverse |
| > Metaverse experience requirements | O | Information provided to create or derive associated metaverse experience parameters for each asset, e.g. information used to derive applicable rendering parameters |
| MRDB exposure application rules | O | Rules for exposure of MRDB information. The policy may include location, temporal, etc. conditions. |
| NOTE 1: IE to be determined in the normative phase based on SA2 work for user identity exposure NOTE 2: IE to be determined in the normative phase based on SA4 work for avatar services NOTE 3: IE to be determined in the normative phase including SA3 security considerations NOTE 4: Information exposed is based on applicable security requirements and policies in both application and network domains. |  |  |

NOTE 1:	The normative phase determines how the MDRB identifier is assigned and used, beyond the role of identifier of the request.

Table 7.6.3-2 shows the MDRB support response from MMES to VAL Server.

Table 7.6.3-2: MDRB support response

| Information element | Status | Description |
| --- | --- | --- |
| MDRB identifier | M | Identifier of the MDRB for which support is provided |
| List of capabilities provided | M | List of MMEL capabilities available for support of the MDRB |
| Shareable MDRB information | O | MDRB information, which is shareable with the application domain. |
| MRDB common exposure policy` | O | Policy for exposure of MRDB information, determined based on the rules in MMEL service request, local or underlying network policies, etc. Individual exposure rules and policies can be additionally applied by 5GS. The policy may include location, temporal, etc. conditions. |
| NOTE: The shareable MDRB information provides applications with access to Core Network functionality currently under development, as well as to Service Layer metadata enabling the identification and coordination of these services. |  |  |

### 7.6.4	Solution evaluation

This solution addresses KI# 3 by providing a generic framework for Avatar support as a type of digital asset. This solution is described using the metaverse enablement architecture option #1 in clause 6.1. Descriptions for the use of other architectural frameworks, i.e. A-DACM functionality (as proposed in solution #7), or existing SEAL servers (as proposed in solution #5) are also provided.

The solution proposes MDRB to manage the avatars. The solution also proposes to include avatar-based session related parameters (like Communication parameters and Metaverse experience requirements) in MDRB.

## 7.7	Solution #7: Digital avatar support by A-DACM

### 7.7.1	Solution description

#### 7.7.1.1	General

This solution maps to KI#3. This solution is based on A-DACM function to support the following avatar management services offered to VAL layer (VAL server):

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

Editor's Note: How VAL client(s) can consume these services is FFS.

#### 7.7.1.2	Avatar profile creation

Figure 7.7.1.2-1 illustrates the high level flow for avatar profile creation procedure.

Figure 7.7.1.2-1: Procedure for Avatar profile creation

1.	VAL server sends the Create Avatar Profile Request to A-DACM function with the details such as Avatar ID (optional), Avatar properties (e.g. name), Avatar usage policies (e.g. how to use the avatars).

NOTE:	The avatar ID is the digital asset identifier for an Avatar.

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, then A-DACM function stores the avatar profile.

3.	The A-DACM function sends a Create Avatar Profile Response to the VAL server indicating success or failure of the operation.

#### 7.7.1.3	Avatar profile upload

Figure 7.7.1.3-1 illustrates the high level flow for avatar upload procedure.

Pre-conditions:

1.	Avatar ID is available as per the procedure in clause 7.7.1.2

Figure 7.7.1.3-1: Procedure for Avatar upload

1.	The VAL server sends the Upload Avatar Request to the A-DACM function with the Avatar ID and Avatar object/media information.

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function performs the storage of avatar object/media information else the storage of the avatar object/media information.

3.	The A-DACM function sends a Upload Avatar Response to VAL server indicating success or failure of the operation.

#### 7.7.1.4	Avatar linking to user/subscriber

Figure 7.7.1.4-1 illustrates the high level flow for avatar linking to user/subscriber procedure.

Figure 7.7.1.4-1: Procedure for Avatar linking to user/subscriber

1.	The VAL server sends a Link Avatar Request to the A-DACM function with Avatar ID and GPSI/External identifier of the UE.

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function checks for the authorization (e.g. user consent) of the subscriber (as per step 1) for linking the avatar with the user/subscriber. If authorized, the A-DACM function links the Avatar ID with the subscriber/user information and stores the related information.

3.	The A-DACM function sends a Link Avatar Response to the VAL server indicating success or failure of the operation.

#### 7.7.1.5	Avatar download

Figure 7.x.1.5-1 illustrates the high level flow for avatar download procedure.

Figure 7.7.1.5-1: Procedure for Avatar download

1.	The VAL server sends a Download Avatar Request to the A-DACM function with the Avatar ID, GPSI/External ID of the UE, Filter/Adaptation information (e.g. VAL information, media adaptations).

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function performs the media adaptation as per the additional filters/adaptation information in the request on the avatar object/media. The adapted avatar media/object is used in the further response to the VAL server.

3.	The A-DACM function sends a response to the VAL server indicating success or failure of the operation. If success, the avatar object/media is included in the response.

### 7.7.2	Architecture Impacts

The solution utilizes the architecture specified in clause 6.2, Option #2.

### 7.7.3	Corresponding APIs

Editor's Note:	This clause provides the corresponding APIs for supporting the solution.

### 7.7.4	Solution evaluation

This clause provides an evaluation of the solution addressing KI#3. The A-DACM function can support the avatar management services towards the VAL layer.

NOTE:	The security mechanisms for this solution is the responsibility of SA3.

## 7.8	Solution #8: Support for spatial map management

### 7.8.1	Solution description

This solution maps to KI#4. This solution enhances SEAL LM service to provide spatial map management service.

### 7.8.2	Architecture Impacts

This clause provides the architecture impacts of the solution. The solution provides enhancements to the SEAL LM functionalities for managing (i.e. produce, update, get, subscribe) spatial maps. To manage spatial maps between VAL server/SEAL LM client and SEAL LM server, reference points LM-S and LM-UU are used in this solution.

### 7.8.3	Procedures

#### 7.8.3.1	Producing spatial map

Figure 7.8.3.1-1 depicts the procedure for producing a spatial map. For the request from the spatial map requestor (VAL server or SEAL LM client), the service is provided by SEAL LM server. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. space, objects).

Figure 7.8.3.1-1: Producing spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to produce the spatial map. The request includes requestor ID, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map and layering information parameters (e.g. layer 1-basic space, layer 2-objects).

NOTE 1:	The detailed information (e.g. access control rules, layer information including relationship between layers) to be included in the spatial map, in the request message, will be further defined in the normative work phase.

The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request.

2)	The SEAL LM server produces a requested spatial map using the layering information and the processed sensor data required to estimate the number of objects, type of the object, position, direction and speed of the object, etc. obtained from various data sources (e.g. VAL server, VAL database or VAL UE) mapped to one or more layers.

NOTE 2:	In this study, for current release, some of the processed sensor data is stored at VAL layer (e.g. in database at VAL layer) and the SEAL LM server gets access to such database to produce spatial map via application specific way which is out of SA6.

3)	The SEAL LM server sends response message to the requestor with produced spatial map ID and if spatial map is already produced, the response message includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them and/or list of layers and its corresponding layer IDs. If response indicates "in-progress", the VAL server (or SEAL LM client) subscribes to receive notification when spatial map is ready for the utilization (e.g. for localization service).

NOTE 3:	The detailed spatial map information in the response message will be further defined in the normative work phase. As an example, a list of spatial anchors which belong to the spatial map could be included.

NOTE 4:	The necessary of supporting third party triggered spatial map producing, in addition to spatial map service provider self-determined of map producing requires further discussion in normative phase.

NOTE 5:	When the LM server sends "in-progress" response, the Produce Spatial Map request can be considered as implicit subscription. The "in-progress" response may required more parameters – which can be considered in normative work. A separate subscription procedure is required or not will be considered in normative work.

#### 7.8.3.2	Update spatial map

Figure 7.8.3.2-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL LM client), SEAL LM server modifies or deletesthe spatial map as requested.

Figure 7.8.3.2-1: Update spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update a spatial map. The request includes requestor ID, spatial map ID, indicator to modify or delete, information on what to modify such as layering information parameters, updated access control rules and updated spatial map coverage area, etc.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request.The SEAL LM server delete or modifies the spatial map as requested.

3)	Then it sends response message to the requestor with result of the request and modified spatial map information with the spatial map ID, list of layers and its corresponding layer IDs.

NOTE:	Whether update and delete operations for spatial map management can be merged or need separate procedures will be considered in the normative work.

#### 7.8.3.3	Get spatial map

Figure 7.8.3.3-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to get the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.3-1: Get spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial map. The request includes requestor ID, spatial map ID to get the information on it, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e. a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest or the specific list of layers with corresponding layer IDs.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this study.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then the server collects the requested spatial map information. The SEAL LM server sends the get spatial map response message to the authorized third party consumer with spatial map ID and spatial map information when matching spatial map is found, otherwise it sends failure indication with reason.

#### 7.8.3.4	Subscribe/unsubscribe spatial map

Figure 7.8.3.4-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.4-1: Subscribe/unsubscribe spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to subscribe/unsubscribe spatial map(s). The request includes requestor ID, spatial map ID(s) or three-dimensional area of interest (or localization information) to identify the applicable spatial map(s), a list of events to subscribe/unsubscribe, which trigger the notification, with triggering criteria, minimum time between consecutive notifications, list of layers with its corresponding layer IDs, etc. For the unsubscribe spatial map request, the subscription ID is included in the message.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then server add/delete the requestor into/from the subscriber list and event list of the spatial map, and sends response message with the result. For the subscribe spatial map request, the subscription ID is included in the message as well.

NOTE 1:	For the VAL server (or SEAL LM client) which unsubscribed all the events of the spatial map, the SEAL LM server removes the VAL server (or SEAL LM client) completely from the subscriber list of the spatial map. In this case, step 3) will not be initiated.

3)	When an event is detected, the SEAL LM server notifies the VAL servers (or SEAL LM clients) which subscribed the corresponding event. Spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map, added or updated or removed layer information, etc.

NOTE 2:	Further triggering events and triggering criteria etc. will be defined in the normative work phase.

#### 7.8.3.5	Spatial localization services

Figure 7.8.3.5-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to invoke spatial localization services. The service is provided by SEAL LM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 7.8.3.5-1: Spatial localization services

1)	The VAL server (or SEAL LMC) sends request to perform localization service in the spatial map to the SEAL LMS. The request includes identity of the map on which the action is to be performed.

Table 7.8.3.5-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target user identity | O | Identity of the user to localize |

2)	The SEAL LMS authorizes the requestor (e.g. VAL server or SEAL LMC). If the requestor is not authorized then the SEAL LMS sends failure response. If the request is authorized:  the SEAL LMS identifies the target users along with its three dimensional localization in the spatial map. The SEAL LMS identifies the target user and also assigns unique identity within maps. If area of interest IE is present then the SEAL LMS identifies the target users within the area of interest only, otherwise, the SEAL LMS identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL LMS sends response to the VAL server (or SEAL LMC). The response includes (as shown in Table-2) the target user localization and identity.

Table 7.8.3.5-2: Spatial localization service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the action |
| User details | O | A list of users |
| > User id | O | Identity of the User |
| > User localization | O | Current localization information of the user in the spatial map |

NOTE:	Whether and how localization of objects can be realized needs further discussions.

#### 7.8.3.6	Delete spatial map

Figure 7.8.3.6-1 depicts the procedure to delete the spatial map(s). For the request from the spatial map consumer (VAL server or SEAL LM client), SEAL LM server deletes the spatial map(s) as requested.

Figure 7.8.3.6-1: Delete spatial map procedure

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to delete the spatial map(s). The request includes requestor ID, a list of spatial map ID(s) to delete and optionally the reason to delete.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request. The SEAL LM server deletes the spatial map(s) as requested.

3)	Then it sends response message to the requestor with result of the request.

### 7.8.4	Corresponding APIs

#### 7.8.4.1	Produce spatial map

Table 7.8.4.1-1 shows the produce spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to produce a spatial map.

Table 7.8.4.1-1: Produce spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. layers information, access control rules) |

NOTE 1:	The requested information will be further defined during the normative work phase.

Table 7.8.4.1-2 shows the produce spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the produce spatial map request.

Table 7.8.4.1-2: Produce spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result (NOTE 1) | M | Result of the producing spatial map (e.g., success, in-progress or fail), including cause of failure if needed |
| Spatial map ID (NOTE 2) | O | ID of the produced spatial map |
| Spatial map information (NOTE 3) | O | If Spatial map ID is valid, a collection of information is included. The information corresponds to the produced spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them or layers information with corresponding layer IDs |
| NOTE 1: The result IE contains in-progress when the enabler server does not have sufficient sensor data to produce spatial map. NOTE 2: This IE is added when result IE indicates success or in-progress. NOTE 3: This IE is added when result IE indicates success. |  |  |

NOTE 2:	Details about Spatial map information is FFS and will be considered during the normative work phase.

#### 7.8.4.2	Update spatial map

Table 7.8.4.2-1 describes the information flow from VAL server (or SEAL LM client) to SEAL LM server for the update spatial map request.

Table 7.8.4.2-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to update |
| Instruction | M | Instruction to update or delete |
| Requested update information | O | Information on what to update/modify or delete, e.g. layers information, access control rules or spatial map coverage area, etc. |

NOTE 1:	The CRUD operations for spatial map management APIs will be enhanced in normative work.

NOTE 2:	The Requested update information will be further defined during the normative work phase.

Table 7.8.4.2-2 describes the information flow from the SEAL LM server to VAL server (or SEAL LM client) for the update spatial map request.

Table 7.8.4.2-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the update spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | M | If the Result is success, ID of the updated spatial map is included. |
| Spatial map information | O | If Spatial map ID is valid, updated spatial map information is included (e.g. layers information, access control rules or spatial map coverage area). |

#### 7.8.4.3	Get spatial map

Table 7.8.4.3-1 shows the get spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to get  spatial maps.

Table 7.8.4.3-1: Get spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Spatial map ID(s) | O | ID(s) of the spatial map to get information of it |
| > Area of interest | O | Three-dimensional area information to discover spatial maps |
| > Localization information | O | A known location to discover spatial maps in the three dimensional space, including an orientation |
| > Layers information | O | The layers information which is to be retrieved from the spatial map |
| > Discovery filters | O | A set of characteristics to search the matching spatial maps |

NOTE:	The Discovery filters will be further defined during the normative work phase.

Table 7.8.4.3-2 shows the get spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the get spatial map request.

Table 7.8.4.3-2: Get spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| List of discovered spatial maps | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial map ID | M | ID of a spatial map discovered |
| > Spatial map information | M | A collection of information that corresponds to the discovered spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them or layers information |

#### 7.8.4.4	Subscribe/unsubscribe spatial map

Table 7.8.4.4-1 shows the subscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to subscribe spatial maps.

Table 7.8.4.4-1: Subscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| > Layers information | O | The layers information within the spatial map(s) to subscribe |
| List of events for discovered target spatial map(s) | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | Type of event to subscribe |
| > Event ID | O | ID of the event to subscribe |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g., added or removed objects, position or direction changes of the objects in the spatial map etc. |
| > Notification intervals | O | Specifies when to notify the detected event, e.g., promptly or periodically with minimum time between consecutive notifications, etc. |

NOTE 1:	Details of how to define events, including by whom and what, will be specified during the normative work phase.

Table 7.8.4.4-2 shows the subscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe spatial map request.

Table 7.8.4.4-2: Subscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the subscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list subscribed | M | List of the events subscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event subscribed |
| >>> Event ID | M | ID of the event subscribed |
| >>> Result | M | Result of the subscribe request for the event (e.g., success or fail), including cause of failure if needed. |
| >>> Subscription ID | O | Allocated ID for the successful subscription to the event |

Table 7.8.4.4-3 shows the unsubscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to unsubscribe spatial maps.

Table 7.8.4.4-3: Unsubscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to unsubscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to unsubscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to unsubscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| > Layers information | O | The layers information within the spatial map(s) to unsubscribe |
| List of events for discovered target spatial map(s) | M | Events to unsubscribe. Each element includes the information described below. |
| > Event type | O | Type of event to unsubscribe |
| > Event ID | O | ID of the event to unsubscribe |
| > Subscription ID | O | Subscription ID to unsubscribe, which was allocated by the SEAL server when the requestor subscribed the event |

Table 7.8.4.4-4 shows the unsubscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe/unsubscribe spatial map request.

Table 7.8.4.4-4: Unsubscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the unsubscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list unsubscribed | M | List of the events unsubscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event unsubscribed |
| >>> Event ID | M | ID of the event unsubscribed |
| >>> Result | M | Result of the unsubscribe request for the event (e.g., success or fail), including cause of failure if needed. |

Table 7.8.4.4-5 shows the spatial map notification sent by SEAL LM server to VAL server (or SEAL LM client) when a subscribed triggering event is detected.

Table 7.8.4.4-5: Notification

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the spatial map that detected the triggering event |
| Event ID | M | ID of the detected triggering event |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |

NOTE 2:	The Notification information will be further defined during the normative work phase.

#### 7.8.4.5	Delete spatial map

Table 7.8.4.5-1 descibes the information flow from the VAL server (or SEAL LM client) to the SEAL LM server for the delete spatial map request.

Table 7.8.4.5-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| List of Spatial map ID(s) | M | Spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map to delete |
| > Reason to delete | O | Reason to delete |

Table 7.8.4.5-2 descibes the information flow from the SEAL LM server to the VAL server (or SEAL LM client) for the delete spatial map response.

Table 7.8.4.5-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result list | M | A list of result(s) for requested spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map requested to delete |
| > Result | M | The result of the delete operation per spatial map (e.g., success or fail), including cause of failure if needed. |

### 7.8.5	Solution evaluation

This solution addresses KI#4. It provides spatial map management service procedures to publish, update/delete, get and subscribe to the spatial map. The solution is technically feasible for providing spatial map service. A new SEAL service and enhanced SEAL LM service are suitable architecture options to enable this solution. The architecture for this solution is yet to be determined. The solution will be adapted to the conclusion of the architecture evaluations.

## 7.9	Solution #9: Avatar discovery procedure

### 7.9.1	Solution description

This solution maps to KI#3 and KI#5. The solution provides a mechanism for discovering avatars.

Pre-conditions:

1)	The VAL client has associated user or avatar profile information for an avatar which is authorized to discover.

Figure 7.9.1-1: Avatar discovery procedure

1.	A VAL client triggers MMEC to search for a digital avatar based on available user and/or avatar profile information.

2.	The MMEC sends an avatar discovery request to a MMES and includes the discovery criteria, e.g. avatar-associated user and/or profile parameters. For example, the request includes (corresponding to Table 7.6.3-1): "Asset type" = avatar, "Metaverse service user information" = information for the user associated with the avatar. Alternatively, the request includes (corresponding to Table 7.6.3-1): "Asset type" = avatar, "Asset type-specific profile" = criteria corresponding to Table 7.5.3.1-1.

The MMEC also includes in the request VAL user and/or MMEC identifiers.

3.	The MMES checks that avatar discovery is authorized, based on VAL user and/or MMEC identifiers. If authorized, the MMES searches an internal repository for an MDRB/avatar profile that is associated with the discovery criteria. If found, the MMES uses VAL user and/or MMEC identifiers to check that the requester is found in the "allowed user list" of the discovered avatar. The MMES can also check the current status and location of the avatar.

4.	For an authorized discovery, the MMES send a response with a status of the request, the digital avatar identifier, the current status and location of the digital avatar, and the metaverse service provider identifier.

5.	The MMEC provides the information returned in step 4 to the VAL client.

### 7.9.2	Architecture Impacts

The architectural impact for this solution is described in clause 6.1, Option #1. The solution may be adapted to using a SEAL server.

### 7.9.3	Corresponding APIs

Table 7.9.3-1 shows the avatar discovery request from the MMEC to the MMES.

Table 7.9.3-1: Avatar discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Requestor security credentials | M | The security credentials of the requestor. |
| Requesting avatar | M | The identifier of the avatar of the requestor. |
| Avatar discovery filters | M | Set of characteristics to determine matching avatar (e.g., avatar identifier, avatar name, avatar description, location, VAL client type, Asset type, Owner user, Allowed user, Digital avatar profile).. |
| Discovery token | M | A token provided for the discovery request. |

NOTE:	The usage of discovery token will be considered in normative phase.

Table 7.9.3-2 shows the avatar discovery response from the MMES to the MMEC.

Table 7.9.3-2: Avatar discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success or fail. |
| Discovery results | M | List of discovery results |
| >Avatar identifiers | O (NOTE) | Identifiers of the avatar |
| >Avatar names | O (NOTE) | Name of the avatar |
| >Discovered avatar profile List | O (NOTE) | Profile of the digital avatar, refer to Table 7.5.3.1-1. |
| >URL | O | A URL of the service provider for where the avatar is present |
| >Avatar status | M | Status of the avatar |
| >Avatar descriptions | O | Description for the avatar |
| NOTE: At least one of the information elements must be present. |  |  |

### 7.9.4	Solution evaluation

This solution maps to KI# 3. This solution is based on the Mobile Metaverse enablement layer architecture as specified in clause 6.1. The solution proposes to discover avatar based on query parameters.

This solution addresses Key Issues #3 and #5 by providing a mechanism for a VAL client to discover avatars, and their associated avatar status, and avatar profiles. The solution is based on the mobile metaverse application enablement layer architecture involving an enabler client (MMEC) and an enabler server (MMES) as described in clause 6.1.

## 7.10	Solution #10: Metaverse digital asset synchronization QoS control

### 7.10.1	Solution description

The following clauses specify procedures and information flows for Key Issue #3 and Key Issue #5 for QoS control for avatars as digital assets.

Preconditions:

1.	A digital asset (e.g. avatar representation) has been created (e.g. as element in MDRB) and MMEL policies have been associated.

NOTE 1:	This solution is described using terminology from clause 6.1, as well as examples or alternatives related to other solutions.

Figure 7.10.1-1: Overall Procedure

0.	A VAL server  and/or VAL client  sends a digital asset synchronization session creation request to a MMES or MMEC to establish end-to-end connections for the coordinated exchange of digital asset information with required session information (e.g. QoS) between VAL client(s) and VAL server(s) for the purpose of synchronization. The request contains information pertaining to the digital asset synchronization session requirements, e.g. VAL client and VAL server endpoint information and required QoS for each end-to-end connection of the digital asset synchronization session.

NOTE 2: Alternatively, the same VAL layer request targets the SEALDD layer, and the SEALDD layer resolves the digital asset synchronization session parameters by querying MMEL. In this alternative, digital asset synchronization session policies and information are assumed to be available to both SEALDD layer and MMEL and the flow continues with step 4.

1.	The MMEC and/or MMES determine applicable digital asset synchronization session policies. This is done, in addition to the information already available in the policies established at precondition, on the session requirements provided in step 1.

2.	The MMES interacts with the SEALDD server to determine and configure additional communication requirements, e.g.  MMES sends SEALDD connection establishment requests to SEALDD server.

3.	Based on interactions from the previous steps, the SEALDD layer creates or updates SEALDD flows to support the digital asset synchronization session based on the requirements provided.

4.	Once SEALDD flows have been established for the digital asset synchronization, MMECs and MMES begin exchanging application traffic (via the SEALDD flows) as required by the application for the purpose of digital asset synchronization.

5.	For each SEALDD flow configured with MMEL support, state information may be maintained by MMES and/or MMEC. The collected state and corresponding measurements performed are based on the configured policies or management requests.

6.	MMEL state and/or monitoring information may be used to enable notifications to be sent to the VAL layer and enable adaptive functionality in the VAL layer.

7.	When the digital asset synchronization session is no longer needed, the VAL client or server sends a request to MMEC or MMES to tear down the digital asset synchronization session and associated SEALDD flow.

### 7.10.2	Architecture Impacts

This solution is described using a new Metaverse Enabler layer (MMES/MMEC entities as described in clause 6.1 and leveraging SEALDD functionality in the SEAL layer) for support of metaverse services.

### 7.10.3	Corresponding APIs

Table 7.10.3-1 details digital asset synchronization session communication parameters received in steps 1- 2 at the SEALDD layer from the VAL Server (directly or via MMES).

Table 7.10.3-1: Digital asset synchronization session communication parameters

| Information element | Status | Description |
| --- | --- | --- |
| List of digital assets | M | A list of individual digital assets associated with this synchronization session. Each digital asset requiring a synchronization session has client-side and server-side representations. |
| > Server-side endpoint ID | M | The unique identifier (e.g., URI) of the VAL server-side endpoint of the synchronization session for this digital asset. |
| > Client-side endpoint ID | M | The unique identifier (e.g., URI) of the VAL client-side endpoint of the synchronization session for this digital asset. |
| > Hosting UE ID | M | Identifier of the UE hosting the VAL client-side of this digital asset |
| > QoS info | M | QoS requirements for the digital asset synchronization session supporting this digital asset, e.g. required bandwidth, latency, jitter, reliability, schedule. |
| NOTE: How digital asset identifier is available and share will be considered in normative phase. |  |  |

NOTE 1:	Table 7.10.3-1 provides digital asset synchronization session communication parameters only. In the normative phase, additional parameters are necessary to describe the one or more messages to be detailed in this clause for steps 1-2.

### 7.10.4	Solution evaluation

This solution enables MMEL to provide digital asset synchronization services to mobile metaverse VAL applications. The procedure allows for QoS coordination and is relevant to metaverse applications requiring synchronization of digital asset information.

The solution complements solution #11 by describing how SEALDD can be leveraged. for establishment of session #1 and session #2, when they are established for synchronization purposes.

## 7.11	Solution #11: Application QoS coordination for Mobile Metaverse Sessions

### 7.11.1	Solution description

This solution maps to KI# 5. This solution provides a mechanism for application session QoS coordination for mobile metaverse services (for use cases such as Mobile Metaverse Based Selective Multi-modal Feedback Service or Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness as discussed in SA1).

In this solution, the mobile metaverse service consists of the following sessions which can be present for multi-user interactions (as can be seen also in Figure 7.11.1-1). The sessions include UE1 and UE2 and avatar counterparts of UE1 and UE2 (called as Avatar UEs for simplicity; however referring to the digital users corresponding to the UEs).

-	App-specific session #1: Metaverse app in VAL client 1 sends to Avatar of UE 1 (in DN side) sensor data / measurements on the physical environment related to VAL UE1 over VAL-UU interface. Avatar of UE1 sends back haptic feedback to UE1 (for UE1 and/or UE2 and the environment).

-	App-specific session #2: Metaverse app in VAL client 2 sends to Avatar of UE 2 (in DN side) sensor data / measurements on the physical environment related to VAL UE2 over VAL-UU interface. Avatar of UE2 sends back haptic feedback to UE2 (for UE1 and/or UE2 and the environment).

-	App-specific session #3: Exchange of service related / feedback data between avatar users (for example micro-transactions such as avatar updates/modifications or environment changes).

-	App-specific session #4: Sensor data / measurements are exchanged between VAL UEs (communication can be over side link) for traffic related to the metaverse service.

Figure 7.11.2-1: Overview of App Sessions in mobile metaverse service

Pre-conditions:

1)	Mobile metaverse app sessions are established.

2)	MMES has discovered the avatar UE information corresponding to the VAL UEs of interest (has associated the identities of avatar users and corresponding physical UEs).

Figure 7.11.1-2: Procedure for metaverse app session QoS coordination

1.	Mobile Metaverse Service Provider (MM-SP) or a VAL UE 1 sends a subscription/request message for MM-specific QoS management. This is followed by a result as response or ACK. Such message includes the list of VAL UEs, service area, the MM service ID, the digital asset ID/info, end to end QoS requirements, the area at the metaverse world where the service is expected to run (e.g., virtual land, virtual office), the metadata for metaverse environment or a link to download this data.

2.	The MMES configures the application QoS parameters by decomposing the end-to-end QoS requirements to application QoS parameters for each individual session (e.g., network session for UE 1, network session for UE 2, session between avatars residing in different EDN/DN) which are part of the end-to-end application service. The MMES obtains or configures QoS policies per each session based on the decomposed QoS requirement.

3.	A trigger event is detected at the MMES via the following ways:

a.	The MMES receives a trigger event from the 5GC (SMF/NEF), denoting a QoS downgrade notification for the UE 1 session (as described in clause 5.7.2.4.1b of 3GPP TS 23.501 [9]).

b.	A QoS downgrade trigger event is sent from the MMEC of VAL UE 1 to the MMES, denoting an application QoS degradation (experienced or expected) e.g., based on the experienced packet delay or packet loss for the Uu link (e.g., packet loss great than threshold value). The conditions for triggering the QoS downgrade indication from the VAL UE1 is based on the threshold that may be provided in advance by the MMES (at the end-to-end QoS management response by the MMES).

c.	A QoS downgrade trigger event is sent from the avatar UE 1 to the MMES, denoting an application QoS degradation (experienced or expected) e.g., based on the experienced packet delay or packet loss for the Uu link (e.g., packet loss great than threshold value).

4.	The MMES evaluates the fulfilment/non-fulfilment of the end-to-end QoS based on the trigger event. MMES may retrieve additional information based on subscription to support its evaluation from the UEs or the avatar UEs / MM-SP. This could be from the 5GC (NEF Monitoring Events as in 3GPP TS 23.502 [9], QoS sustainability analytics as in 3GPP TS 23.288 [10]) or SEAL LMS (on demand location reporting for one or both UEs 1 and 2).

In this step, the MMES may also trigger the initiation or retrieval (if simulations are running on the background assuming digital twin platform at the DN side) of simulations for different what-if hypotheses, and in particular to capture the possible output (performance /availability/failure rates) if different QoS related action is taken. For example, if QoS of app session #1 is upgraded as a compensation to session #2 downgrade, then QoS/resource management impact to other UEs of the same or different services need to be checked. The simulation runs all possible outcomes of a particular potential decision and does it for different combinations of decisions.

5.	Then, the MMES based on the evaluation or simulation outputs of step 4, determines an action, which is the QoS parameter adaptation of one or more of the links (QoS profile downgrade for the link receive QoS notification control, and QoS upgrade for the link which can be upgraded).

6.	The MMES, acting as AF, sends to the 5GC (to SMF via NEF or to PCF via N5) a request for a change of the QoS profile mapped to the one or more network sessions (for UE 1 and UE 2 and their avatars) or the update of the PCC rules to apply the new traffic policy (as specified in 3GPP TS 23.502 [9] in clause 4.15.6.6a: AF session with required QoS update procedure).

7.	The MMES sends the new application or network QoS policies/parameters to the involved entities (physical and metaverse UEs and MM-SP).

### 7.11.2	Architecture Impacts

This solution requires a new Metaverse Enabler layer (MMES/MMEC entities) to support a new capability on QoS coordination for metaverse services.

### 7.11.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #x.

-	MM-specific QoS management API (request / response model or subscribe/notify; API provider: MMES; known consumer: VAL UE, VAL server; corresponding to step 1).

-	MM-specific QoS update notify API (command/ notify; API provider: MMES; known consumer: VAL UE, VAL server; corresponding to step 7).

### 7.11.4	Solution evaluation

This solution provides a mechanism at MMES/MMEC for application session QoS coordination for mobile metaverse services, via compensating the possible downgrade in certain links (e.g., due to high latency in one or more links). This solution is only applicable to metaverse applications since the sessions are for both physical UEs and avatars corresponding to these UEs.

Other more generic solution has been proposed in the context of UASAPP/SEAL for QoS coordination between application sessions of two VAL UEs (e.g., UAV and UAV-C) in clause 14.3.5 of 3GPP TS 23.434 [8] (QoS/resource management for network-assisted UE-to-UE communications). However, this solution addresses a more complex and specific to metaverse use case which deals with both UE-to-Network sessions (UE to Avatar), but also sessions between physical UEs and sessions between avatars (which can be in different edge / cloud platforms).

NOTE:	Possible re-use of existing solutions for QoS coordination by SEAL NRM (or XR enabler) will be further examined in normative phase.

The solution is feasible and consumes the services already provided by 5GC (for QoS monitoring and QoS influence).

## 7.12	Solution #12: Support device discovery to offload task for metaverse services

### 7.12.1	Solution description

This solution maps to KI#6. This solution proposes to enhance Application layer support for Personal IoT Network as defined in 3GPP TS 23.542 [12].

#### 7.12.1.1	PIN client requests to discover another PIN element

Figure 7.12.1.1-1 illustrates procedure of PIN client requests to discover another PIN element, based on request/response model.

Pre-conditions:

1.	The UE (PIN client) has already discovered PIN as specified in clause 8.5.7 of 3GPP TS 23.542 [12];

2.	The UE (PIN client) has already joined the PIN as specified in clause 8.5.8.2 of 3GPP TS 23.542 [12]

Figure 7.12.1.1-1: discovery PIN element

1.	The PIN client of a PIN element sends request the PIN Management client to discover another PIN elements. The request includes the security credentials of the PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location, PIN ID as an identifier of the PIN and services/capabilities which is required by the requesting PIN element and time duration till when the PIN element is required.

2.	The PIN Management client authorizes the PIN client of a PIN element and discovers list of PIN elements based on provided parameters in the request and on information of PIN Element registered in the PIN. The PIN elements registered in the PIN may be known at the PIN Management Client (e.g., in the PIN dynamic profile) or otherwise may be obtained by the PIN Management Client from the PIN Server. The PIN Management client sends response back to the PIN client. The response contains list of PIN elements along with PIN client profiles for each discovered PIN elements, in case of success. The response includes failure cause in case of failure.

NOTE 1:	The information about PIN Element registered in the PIN (e.g., in PIN Dynamic profile, and in PINE registration) may be updated in the normative phase to include information about task offloading (e.g., task offloading capable, application(s), time duration, etc.).

NOTE 2:	If multiple PIN elements are discovered, then how to select the best PIN element to offload the task is up to application level implementation.

NOTE 3:	The PIN Management PINE join into PIN response as specified in clause 8.5.8.3.3 does not include list of already joined PIN elements, so the newly joined PIN element is not aware of already joined PIN elements.

### 7.12.2	Architecture Impacts

This solution does not impact the existing architecture as defined in 3GPP TS 23.542 [12].

### 7.12.3	Corresponding APIs

Table 7.12.3-1 shows the request sent by a PIN client of a PIN element to a PEMC for the PIN element discovery request.

Table 7.12.3-1: PIN element discovery request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to join in. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| Discovery filters | M | Provides discovery filters to discover PIN elements |
| > UE location | O | The location information of the UE. The UE location is described in clause 7.2.7 of 3GPP TS 23.542 [12]. |
| > required PIN services | O | Specifies PIN services as required by the PIN element (as specified in Table 8.2.2.1-1 of 3GPP TS 23.542 [12]). |
| > time duration | O | Specifies time duration till when the service from the PIN element is required |

Table 7.12.3-2 shows the response sent by the PEMC to the requester for a PIN element discovery response.

Table 7.12.3-2: PIN element discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered PIN elements List | O | List of discovered PIN element(s). Each element includes the information described below |
| > PIN client ID | M | Identifier of the PIN client. |
| > PIN client profile | O | PIN client profile |

### 7.12.4	Solution evaluation

The solution resolves open issue 1) of KI #6. The solution provides a procedrue for a PIN element to discover another PIN elements to offload the computational task. The solution does not required any architecture update. The solution impacts PIN client and PIN management client. The solution is a feasible solution.

## 7.13	Solution #13: Support for metaverse services requiring multiple devices

### 7.13.1	Solution description

This solution maps to KI# 7. A metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game).

Consider a use case where a metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game). This solution proposes to enhance EAS profile as defined in 3GPP TS 23.558 [13] to enable EAS to provide required associated devices information. This solution further proposes to enhance EAS discovery procedure as defined in 3GPP TS 23.558 [13] to enable EEC to provide associated devices information.

#### 7.13.1.1	Enhancement to EAS profile

Following information element is added as an enhancements to EAS profile as defined in clause 8.2.4 of 3GPP TS 23.558 [13].

Table 7.13.1.1-1: Enhancement to EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| List of associated devices | O | List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type. |

#### 7.13.1.2	Enhancement to EAS discovery request (between AC to EEC)

Following information element is added as an enhancements to EAS discovery request as defined in clause 8.14.3.8 of 3GPP TS 23.558 [13].

Table 7.13.1.2-1: Enhancement to EAS discovery request in clause 8.14.3.8 of 3GPP TS 23.558 [13]

| Information element | Status | Description |
| --- | --- | --- |
| List of associated devices | O | List of associated devices available (e.g. haptic device, joy stick) with the UE. It includes device type. |

#### 7.13.1.3	Enhancement to EAS discovery procedure (between EEC to EES)

Following information element is added as an enhancements to EAS discovery request as defined in clause 8.5.3.2 of 3GPP TS 23.558 [13].

Table 7.13.1.3-1: Enhancement to EAS discovery request in clause 8.5.3.2 of 3GPP TS 23.558 [13]

| Information element | Status | Description |
| --- | --- | --- |
| List of associated devices | O | List of associated devices (e.g. haptic device, joy stick) available with the UE. It includes device type. |

The EES determines the EAS based on the discovery filter provided by EEC and based on the information received in EAS profile in EAS registration. If an EAS requires specific list of associated devices and the required devices are available with the UE as specified in the discovery request, then the EAS is added into the list of discovered servers.

### 7.13.2	Architecture Impacts

This clause describes the architecture impacts to the EDGEAPP architecture as defined in 3GPP TS 23.558 [13]. Figure 7.13.2-1 illustrates the associated devices (e.g. tactile gloves or VR glasses) are connected with the application client of the UE.

Figure 7.13.2-1: Associated devices connected with AC of the UE

The associated devices (UE-2 and UE-3) are connected to the application client of the UE using 3GPP or non-3GPP connection. The associated devices do not have EEC.

NOTE 1:	For simplicity, other elements of the EDGEAPP architecture are not shown in above figure as there are no impacts.

NOTE 2:	The application client in UE-1 is aware of the associated devices using application specific ways which is out of scope of SA6.

### 7.13.3	Corresponding APIs

The enhancement to APIs as defined in 3GPP TS 23.558 [13] are specified in clause 7.13.1.

### 7.13.4	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution addressing KI#x.

This solution resolved open issue 1 of the KI 7. The architecture proposes to have application level connection between application client of the UE and other associated devices as shown in clause 7.13.2. The solution enhances existing EAS profile and EAS discovery procedures of 3GPP TS 23.558 [13].

## 7.14	Solution #14: Signalling of user consent

### 7.14.1	Solution description

This solution maps to KI#2. This solution addresses key issue 2 on exposure of user sensitive information, where it is highlighted that ensuring appropriate user consent has been obtained is a critical aspect when handling sensitive information relating to or collected from a user, their devices or the applications installed at their devices.

As part of solution #5 on support for digital avatars, Table 7.5.3.1-1 provides an example of the attributes that a client (configuration management client / MMEC / VAL server) may expose to a server (configuration management server / MMES). Many, if not all, of those attributes can be considered as being privacy sensitive information, particularly those relating to the application specific avatar profile. For instance, those attributes relating to user identification (e.g., user ID, allowed user list), location (e.g., current location, the list of locations as a sub-attributes of the history information) and predicted user behaviour (e.g., list of predictive models). The table also includes the security parameters attribute, which is described being for authentication and authorization. However, user consent is not explicitly highlighted.

User consent is obtained through interaction between the authorization server and user where the client triggering the interaction can specify the scope of the access request. In turn, the authorization server uses the scope response parameter to inform the client of the scope of the access granted. This capability has been introduced in CAPIF supporting RNAA, reference clause 6.2.3 of 3GPP TS 23.222 [14].

It is proposed in this solution that such a mechanism could be leveraged to not only request the scope of (server resource) access, but also the scope of what the client itself can expose whilst requesting such access.

NOTE:	Enhancing scope field is within SA3 scope.

The process is demonstrated in figure 7.14.1-1 in relation to the create application specific avatar profile request procedure of solution #5, clause 7.5.3.1.

In step 1 (out of scope of this specification) the user consent is obtained along with its scope, i.e., which attributes the client has permissions to include in subsequent server requests.

In step 2 the create avatar profile request is issued by the client, including the user consent for privacy sensitive information exposure by the client as provided through step 1.

In step 3 the server authorizes the client’s request, including verifying that the necessary user consent has been provided for the attributes included in the request.

In step 4 the server provides a success response if the authorization and verification checks are successful.

Figure 7.14.1-1: Enhancements to figure 7.5.3.1-1 to include user consent verification

### 7.14.2	Architecture Impacts

The solution is applicable to the architectures specified in the subclauses of clause 6.

### 7.14.3	Corresponding APIs

The solution is applicable to all procedures in which the client of a mobile metaverse service (e.g., the MMEC of clause 6.1.1 or SEAL CM client of clause 7.5.2) can expose user sensitive information towards a server offering mobile metaverse services (e.g., the MMES of clause 6.1.1 or SEAL CM server of clause 7.5.2 / A-DACM of clause 6.2.2.1).

Specifically in those instances where a client request includes user sensitive information (e.g., the Create avatar profile request of table 7.5.3.1-1 or the Request MDRB support of table 7.6.3-1).

### 7.14.4	Solution evaluation

This solution addresses KI#2 through enhancement of mobile metaverse service server functionality with the capability to verify that mobile metaverse service clients have user consent for exposing user sensitive information in the requests they make towards the servers. This is achieved by enhancing the existing server authorization capability to also check for appropriate user consent of the information exposed by the client in requests before accepting the request. This mechanism can protect that server from processing user sensitive information and provides indication to the client on what it is permitted to expose, although cannot guarantee the client won’t expose it. The security mechanism for providing indication of obtained user consent for the exposure of user sensitive information by a client of the mobile metaverse service are not in scope of the present document.

NOTE 2:	The procedure in Figure 7.14.1-1 may need enhancement based on coordination with SA3.

Editor's Note: Prior to inclusion of this solution in the normative phase, the feasibility of providing a security mechanism to enable a client to obtain user consent for sensitive information needs coordination with SA3.

## 7.15	Solution #15: Support for VAL UEs in spatial map

### 7.15.1	Solution description

This solution maps to KI#4. This solution enables to support the management of spatial map with VAL UEs information related to the VAL server and its VAL services. During create or update of the spatial map, based on request from the VAL server, this solution  enhances spatial map with information related to the VAL UEs that are available in the area related to the spatial map and are related to the VAL services of the VAL server. The produced spatial map will include VAL UEs information (position in spatial map) association with VAL services.

### 7.15.2	Architecture Impacts

The solution enhances the SEAL LM functionalities to manage the spatial map with VAL UEs information in the area related to the spatial map and that are related to the VAL server’s VAL services. For the spatial map management between the VAL server/SEAL LM client and SEAL LM server, reference points LM-S and LM-UU can be used in this solution.

### 7.15.3	Procedures

#### 7.15.3.1	Create spatial map augmented with VAL UE

Figure 7.x.3.1-1 illustrates the procedure to create a VAL UE augmented spatial map. The request is from VAL server or an SEAL LM client. The service is provided by SEAL LM server.

Editor’s note:	This procedure is to provide the spatial map service with VAL UE, not spatial map. The terminologies will be reconsidered.

Figure 7.15.3.1-1: Create spatial map with VAL UE

1.	The VAL server (or SEAL LM client) sends request to produce the VAL UE augmented spatial map to the SEAL LM server. The request includes area of interest, an indication to identify VAL UEs in the spatial map, VAL service(s) list. The indication informs the SEAL LM server to include (or tag) the information (example, position, location) related to the VAL UEs in the spatial map created, that are available in the area of interest related to the spatial map to be created. VAL Service(s) information indicates to SEAL LM server to tag the VAL UEs information related to these VAL service(s) in the created spatial map.

2.	The SEAL LM server will authenticate and authorize the requesting entity VAL server/SEAL LM client. If the VAL server or SEAL LM client is not authorized then the SEAL LM server sends failure response. If Authentication and authorization is success, then the SEAL LM server fetches the list of VAL UEs in the area of interest related to the spatial map to be created as in the request message, from SEAL LM server using clause 9.3.10 of 3GPP TS 23.434 [8] and/or from NEF as specified in 3GPP TS 23.502 [9]. The SEAL LM server identifies the VAL UEs information related to the VAL service(s) in the request message from the VAL server or SEAL LM client.

3.	The SEAL LM server produces the requested spatial map, using the processed sensor data required to estimate the number of objects, type of the object, position, direction and speed of the object, etc .

4.	The SEAL LM server updates the produced spatial map with the VAL UEs information obtained in step 2, that is, information (example, position, location) of the identified VAL UEs in the spatial map and tag them with the VAL Services(s) they are related to as in the Produce spatial map request message from the VAL server or SEAL LM client.

5.	The SEAL LM server sends the VAL UE augmented spatial map response message including the information related to the created spatial map. The information may include, map identity, VAL UEs position in spatial map, VAL UEs tagged with the related VAL service(s), and stationery / moving objects with attributes related to them.

#### 7.15.3.2	Update spatial map augmented with VAL UEs

Figure 7.15.3.1-1 illustrates the procedure to updating a VAL UE augmented spatial map. The request is from VAL server or an SEAL LM client. The service is provided by SEAL LM server.

Pre-condition:

1.	Spatial map is created and VAL server (or SEAL LM client) is aware of the spatial map identity.

2.	The VAL server (or SEAL LM client) is authorized to perform the update of the spatial map with VAL UEs information.

Figure 7.15.3.2-1: Update spatial map with VAL Ues

1.	The VAL server (or SEAL LM client) sends request to update the VAL UE augmented spatial map to the SEAL LM server. The request includes map identity (identity of the spatial map) that needs to be updated, an indication to identify VAL UEs in the spatial map, VAL service(s) list and may include VAL UEs of interest. The indication informs the SEAL LM server to update the spatial map with the information (example, position, location) related to the VAL UEs that are available in the area/location related to the spatial map to be updated. VAL Service(s) information indicates to the SEAL LM server to update the VAL UEs information related to these VAL service(s) in the updated spatial map.

2.	The SEAL LM server will authenticate and authorize the VAL server or SEAL LM client. If the VAL server or SEAL LM client is not authorized then the SEAL LM server sends failure response. If Authentication and authorization is success, the SEAL LM server determines the area/location information from the target spatial map to be updated using map identity information in the request message and fetches the VAL UEs available in the area/location related to the spatial map, from SEAL LM server using clause 9.3.10 of 3GPP TS 23.434 [8] and/or from NEF as specified in 3GPP TS 23.502 [9]. The SEAL LM server further fetches the VAL service(s) information related to the identified VAL UEs in the area of interest, from the SEAL CM server using clause 11.3.5 of 3GPP TS 23.434 [8]. The SEAL LM server identifies the VAL UEs information related to the VAL service(s) in the request message from VAL server or SEAL LM client.

3.	The SEAL LM server updates the spatial map with the VAL UEs information obtained in step 2, that is, information (example, position, location) of the identified VAL UEs and tag identify them with the VAL Services(s) they are related to as in the update spatial map request message from the VAL server or SEAL LM client.

4.	The SEAL LM server sends the VAL UE augmented spatial map response message including the information related to the updated spatial map. The information may include, map identity, VAL UEs position in spatial map, VAL UEs tags with related VAL service(s), stationery or moving objects with attributes related to them.

### 7.15.4	Corresponding APIs

#### 7.15.4.1	Overview

This clause provides summary of the corresponding APIs for the solution #15.

1.	Produce spatial map with VAL UEs API (Request / Response, API Provider: SEAL LM server, Known consumers: VAL server/ SEAL LM client, corresponding to step 1 to 5 of clause 7.15.3.1)

2.	Update spatial map with VAL UEs API (Request / Response, API Provider: SEAL LM server, Known consumers: VAL server/ SEAL LM client, corresponding to step 1 to 4 of clause 7.15.3.2)

#### 7.15.4.2	Information Flows

Table 7.15.4.2-1 shows the information in the request sent by VAL Server/ SEAL LM client to SEAL LM server to produce VAL UE augmented spatial map with VAL UEs

Table 7.15.4.2-1: Produce VAL UE augmented spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL LM client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of Interest | M | Location area of interest related to which the spatial map is to be created. |
| Identify VAL UEs | M | Indication to identify the VAL UEs in the spatial map. |
| > VAL UE of interest | O | The VAL UEs of interest to be identified |
| VAL service(s) | M | The VAL service information |

Table 7.15.4.2-2 shows the VAL UE augmented spatial map response message from SEAL LM server to VAL Server/ SEAL LM client with information about created spatial map

Table 7.15.4.2-2: VAL UE augmented spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |
| Spatial map information | M | Information related to the created spatial map (for successful case) |
| > map identity | M | Identity information of the created spatial map |
| > VAL UEs Information | O | Information (example, position, location) of the identified VAL UEs in the spatial map and tagged with the VAL Services(s) they are related to as in the request message from the VAL server |
| >> VAL UE details | O | Identity and/or other details related to the VAL UE |
| >> Location | M | Location information of the VAL UE in the spatial map |

Table 7.15.4.2-3 shows the request sent by VAL Server/ SEAL LM client to SEAL LM server to update VAL UE augmented spatial map with VAL UEs

Table 7.15.4.2-3: Update VAL UE augmented spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL LM client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Map identity | M | Identity information of the spatial map to be updated. |
| Identify VAL UEs | O | Indication to identify the VAL UEs in the spatial map. |
| > VAL UE of interest | O | The VAL UEs of interest to be identified |
| VAL service(s) | O | The VAL service information |

Table 7.15.4.2-4 shows the update VAL UE augmented spatial map response message from SEAL LM server to VAL Server/ SEAL LM client with information about updated spatial map

Table 7.15.4.2-4: Update VAL UE augmented spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |
| Spatial map information | M | Information related to the updated spatial map (for successful case) |
| > map identity | M | Identity information of the updated spatial map |
| > VAL UEs Information | O | Information (example, position, location) of the identified VAL UEs in the spatial map and tagged with the VAL Services(s) they are related to as in the request message from the VAL server / SEAL LM client. |
| > VAL UE details | O | Identity related to the VAL UE |
| > Location | M | Location information of the VAL UE in the spatial map |
| > VAL Service(s) | O | VAL Services(s) related to VAL UE. |

### 7.15.5	Solution evaluation

This solution addresses KI#4 by providing the VAL UE augmented spatial map create/update services. The procedures are based on the SEAL-LM architecture and functional model. The solution enhances the existing SEAL LM server by adding new functionalities to manage spatial maps augmented with VAL UE information. This solution enhances the spatial map management with enriched information related to VAL UEs available in the spatial map with the related the VAL service(s) information. This solution is complementary to Solution #8 specified in clause 7.8 and can be combined in normative phase. A consistent way of obtaining processed sensing information for objects by SEAL LMS including objects behaving as VAL UEs is required and will be determined in normative phase.

## 7.16	Solution #16: Spatial mapping service exposure and discovery

### 7.16.1	Solution description

#### 7.16.1.1	General

This solution is for KI#4 and addresses the open issue of how to produce, modify and expose a spatial map.

This solution is presented based on the Mobile Metaverse enablement layer architecture described in clause 6.1 (e.g., option #1); the solution is also applicable if introducing a new SEAL service reusing an existing SEAL service as described in clause 6.3 (e.g., option #3).

In this solution, a VAL server may provide a spatial mapping service; the VAL server may expose the spatial mapping service via MNO APIs for the purposes of complementing the MNO spatial mapping coverage when the MNO does not or cannot produce or maintain a spatial map for a location. For example, a company may provide and maintain spatial maps for their factory and may expose a spatial mapping service to users within the factory via the MNO APIs. A user within the factory may discover and use the spatial mapping service for the factory via the MNO APIs.

In this solution, the mobile metaverse service provides capabilities for exposing a spatial mapping service and for discovering spatial mapping services. Spatial mapping services may be discovered based on consumer application requirements and capabilitites, and mapping service requirements and capabilities.

NOTE: The MMES can integrate CAPIF core function and reuse 3GPP TS 23.222 [14] as implementation choice without standard impact.

#### 7.16.1.2	Exposing a spatial mapping service

This procedure allows an authorized VAL server to expose a spatial mapping service to authorized consumers.

Figure 7.16.1.2-1: Exposing a spatial mapping service

1.	A VAL server providing a spatial mapping service sends a spatial mapping service exposure request to the MMES. The request includes a VAL server identifier, spatial mapping service type, spatial mapping service address (e.g., FQDN, URI, IP, endpoint), spatial mapping capabilities (e.g., spatial mapping and/or spatial localization capabilities), and spatial map information (e.g., identifier, coverage area, civic address, latest update time, etc.).

2.	Upon receiving the request, the MMES authorizes the exposure request and stores the spatial mapping service information.

3.	The MMES sends a spatial mapping service exposure response to the spatial mapping service indicating success or failure to expose the spatial mapping service. In the failure case, a reason may be provided.

#### 7.16.1.3	Discovering a spatial mapping service

This procedure allows a consumer application on a UE to discover a spatial mapping service according to provided requirements.

Pre-conditions:

1)	A spatial mapping service is exposed via the MMES.

Figure 7.16.1.3-1: Discovering a spatial mapping service

1.	A VAL client requires spatial mapping services for a specific location.

2.	The VAL client requests a spatial mapping service discovery from the MMEC to discover available spatial mapping services. The request includes discovery filters based on the VAL client requirements (e.g., requested area of interest, etc.).

3.	The MMEC sends a spatial mapping service discovery request to the MMES. The request may include information received from the VAL client and additional UE information (e.g., UE identifier, UE location, UE pose, etc.).

4.	Upon receiving the request, the MMES authorizes the discovery request and if the request is authorized determines a spatial mapping service based on the information provided in the request (e.g., discovery filters, identifiers, etc.).

NOTE 1:	The MMES considers the spatial mapping service offered by the MNO and the spatial mapping service(s) exposed via the MNO to determine a spatial mapping service that meets the UE requirements.

NOTE 2:	The spatial mapping service discovery request may be sent following a spatial map discovery request that has failed with the MNO spatial mapping service (e.g., MNO indicated no coverage for the area of interest).

The MMES may also obtain location information from the 5GC about the UE by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8].

5. 	The MMES sends a spatial mapping service discovery response to the MMEC. The response may include one or more determined spatial mapping service(s) with the associated information (i.e., address, identifier, capabilities, and other information provided at spatial mapping services exposure in clause 7.16.1.2).

6.	The MMEC informs the VAL client and includes information received from the MMES.

NOTE 3:	If the information provided to the VAL client includes spatial mapping service(s), the VAL client can obtain spatial map(s) from that service for the area of interest.

#### 7.16.1.4	Service exposure via CAPIF

The mobile metaverse services can support exposing the spatial mapping service offered by the VAL server via CAPIF as specified in 3GPP TS 23.222 [14] by deploying CAPIF core function within the MMES to support publish and discovery of the VAL server service APIs.

NOTE 1:	The MMES supporting the exposure of the spatial mapping service offered by the VAL server using CAPIF is not explicitly depicted in clause 7.16.1.2.

NOTE 2:	Determining if the MNO or if a 3rd party spatial mapping server has a requested spatial coverage is performed by the MMES.

### 7.16.2	Architecture Impacts

This solution can be realized according to architectures described in clause 6.1 or clause 6.3 or using new SEAL Server.

### 7.16.3	Corresponding APIs

#### 7.16.3.1	Overview

This clause provides a summary of the corresponding APIs for solution #16.

-	Spatial mapping service exposure API (request / response model; API provider: MMES; known consumers: VAL server; corresponding to step 1 and 3 of clause 7.16.1.2).

-	Spatial mapping discovery API (request / response model; API provider: MMEC; known consumer: VAL client; corresponding to step 2 and 6 of clause 7.16.1.3).

-	Spatial mapping discovery API (request / response model; API provider: MMES; known consumer: MMEC; corresponding to step 3 and 5 of clause 7.16.1.3).

NOTE:	The spatial mapping discovery API provided by the MMEC and MMES may be enhanced with thesubscribe / notify model in the normative phase.

#### 7.16.3.2	Information flows

Table 7.16.3.2-1 shows the request sent by a VAL server to a MMES for exposing a spatial mapping service.

Table 7.16.3.2-1: Spatial mapping service exposure request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Mapping Service list | M | The list of Spatial Mapping Services to be exposed. Each element includes the information described below. |
| > Identifier | M | The identifier of a spatial mapping service |
| > Type | O | The type of spatial mapping service |
| > address | M | The address to access the spatial mapping service (e.g., FQDN, URI, IP, endpoint) |
| > capabilities | O | The capabilities offered by a spatial mapping service (e.g., spatial mapping, spatial localization capabilities, or both) |
| > Spatial Map information | M | The information about Spatial Map(s) (SM) available at the spatial mapping service (e.g., one or more spatial map may be provided by the service). Each element includes the information described below. |
| >> SM identifier | M | The identifier of a spatial map. |
| >> SM area coverage | M | The spatial area coverage of a spatial map. |
| >> SM location | O | The location of a spatial map (e.g., civic address, etc.). |
| >> SM update time | O | The time when the spatial map was last updated. |
| >> SM update frequency | O | The frequency at which a spatial map is updated. |

Table 7.16.3.2-2 shows the response sent by a MMES to a VAL server when exposing a spatial mapping service.

Table 7.16.3.2-2: Spatial mapping service exposure response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |

Table 7.16.3.2-3 shows the request sent by a MMEC to a MMES for discovering a spatial mapping service.

Table 7.16.3.2-3: Spatial mapping service discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial mapping service discovery filters | M | Set of characteristics to determine spatial mapping service(s). The discovery filters are aligned with the information described in “list of Spatial Mapping Services” described in table 7.16.3.2-1. |

Table 7.16.3.2-4 shows the response sent by a MMES to a MMEC when discovering a spatial mapping service.

Table 7.16.3.2-4: Spatial mapping service discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Spatial mapping services list | O | The list of spatial mapping services determined based on the discovery filters. Each element includes information as described in “list of Spatial Mapping Services” described in table 7.16.3.2-1. |
| Failure cause | O | Indicates reason for the failure |

### 7.16.4	Solution evaluation

This solution is for KI#4 and addresses the open issue of how to produce, modify and expose a spatial map.

This solution can be realized according to architectures described in clause 6.1 or clause 6.3.This solution proposes that the mobile metaverse service provides capabilities for exposing a spatial mapping service provided by a VAL server for complementing the MNO spatial coverage and proposes that the mobile metaverse service provides capabilities for discovering spatial mapping services offered by the MNO and/or VAL server(s) according to a consumer application requirements.

## 7.17	Solution #17: Discovery of UEs consenting and capable of providing spatial mapping information

### 7.17.1	Solution description

#### 7.17.1.1	General

This solution is for KI#4 and addresses the open issue of how to enable UE(s) to provide spatial mapping information for maintaining a spatial map.

This solution is presented based on the Mobile Metaverse enablement layer architecture described in clause 6.1 (e.g., option #1); the solution is also applicable if introducing a new SEAL service or reusing an existing SEAL service as described in clause 6.3 (e.g., option #3).

In this solution, mobile metaverse enablement provides capabilities to register, discover, and request measurements from UE that consent and are capable of providing spatial mapping information. The UEs may be discovered based on location of interest and spatial mapping capabilities.

NOTE 1:	For the current release, it is assumed that spatial mapping information for maintaining a spatial map is stored by the VAL layer.

NOTE 2:	How the VAL layer uses the obtained spatial mapping information is out of scope.

#### 7.17.1.2	Registering a spatial mapping information provider

This procedure allows a consenting VAL client (e.g., UE) that can provide spatial mapping information to register as a provider of spatial mapping information to be discoverable by authorized consumers.

NOTE:	In this solution, the spatial mapping information provider is consenting since the provider initiates the registration. A non-consenting spatial mapping information provider should not register to provide spatial mapping information.

Figure 7.17.1.2-1: Spatial mapping information provider registration

1.	The MMEC sends a spatial mapping information provider registration request to the MMES. The request includes a UE identifier, spatial information details (e.g., identifier, type (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.), position (e.g., geographic position, localization), state (e.g., availability), accuracy) and may include a VAL client identifier.

2.	Upon receiving the request, the MMES authorizes the registration request and stores the spatial mapping information provider details included in the request.

The MMES may also obtain location information from the 5GC about the UE by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8].

3.	The MMES sends a spatial mapping information provider registration response to the MMEC indicating success or failure to register the spatial mapping information provider, including a failure reason if needed.

#### 7.17.1.3	Discovering and triggering a spatial mapping information provider

This procedure allows a spatial mapping service to discover and trigger a registered spatial mapping information provider for spatial mapping information.

Pre-conditions:

1)	Spatial mapping information providers have registered with the MMES.

Figure 7.17.1.3-1: Spatial mapping information provider discovery and information procedure

1.	A spatial mapping service requires spatial mapping information in a specific location to create or update a spatial map.

2.	The spatial mapping service sends a spatial mapping information provider discovery request to the MMES to discover available providers. The request includes discovery filters such as the location area of interest and requirements on spatial mapping information. The spatial mapping information requirements may include type (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.), accuracy, and reporting (e.g., one-time measurement, periodic measurements, continuous stream, etc.).

3.	Upon receiving the request, the MMES authorizes the discovery request and determines a list of available spatial mapping information providers based the provided discovery filters and the registered spatial mapping information providers. Additionally, the MMES may obtain UE location information for the determined providers. The MMES may obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8].

4. 	The MMES sends a spatial mapping information notification to the selected spatial mapping information provider(s) (i.e., via the MMEC). The notification includes details on spatial mapping information reporting (e.g., which spatial mapping information is needed, desired reporting frequency, etc.) and endpoint information for sending the spatial mapping information. The MMEC may inform a VAL client about the notification.

NOTE 1:	Registering as a spatial mapping information provider with the MMES as described in clause 7.17.1.2 subscribes the MMEC to receive spatial mapping information notification.

5. 	The MMES sends a spatial mapping information provider discovery response to the spatial mapping service. The response includes a list of determined spatial mapping information providers.

6.	The MMEC or VAL client initiates a session with the spatial mapping service indicated in the notification, using the received endpoint information and sends spatial mapping information as indicated in the notification.

NOTE 2:	The notification may indicate to start or stop sending spatial mapping information.

### 7.17.2	Architecture Impacts

Option #1 in clause 6.1, option #3 in clause 6.3 or a new SEAL client and server can be used to realize this solution.

### 7.17.3	Corresponding APIs

#### 7.17.3.1	Overview

This clause provides a summary of the corresponding APIs for solution #17.

-	Spatial mapping information provider registration API (request / response model; API provider: MMES; known consumers: MMEC; corresponding to step 1 and 3 of clause 7.17.1.2).

-	Spatial mapping information provider discovery API (request / response model; API provider: MMES; known consumer: VAL server; corresponding to step 2 and 5 of clause 7.17.1.3).

-	Spatial mapping information API (subscribe / notify model; API provider: MMES; known consumer: MMEC; corresponding to step 4 of clause 7.17.1.3).

#### 7.17.3.2	Information flows

Table 7.17.3.2-1 shows the request sent by a MMEC to a MMES for registering a spatial mapping information provider.

Table 7.17.3.2-1: Spatial mapping information provider registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE identifier) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL client identifier | M | The identity of the VAL client. |
| Notification endpoint | M | The endpoint to notify that spatial mapping information is needed. |
| Spatial mapping information details | M | The Spatial Mapping Information (SMI) details. Each element includes the information described below. |
| > SMI identifier | M | The identifier of spatial mapping information. |
| > SMI type | M | The type of spatial mapping information (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.). |
| > SMI state | O | The state of spatial mapping information (e.g., availability). |
| > SMI accuracy | O | The accuracy of spatial mapping information. |
| > SMI format | O | The format of a spatial mapping information (e.g., raw, processed, etc.) |

Table 7.17.3.2-2 shows the response sent by a MMES to a MMEC for registering a spatial mapping information provider.

Table 7.17.3.2-2: Spatial mapping information provider registration response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Registration identifier | O | Indicates the registration identifier is successful |
| Failure cause | O | Indicates reason for the failure |

Table 7.17.3.2-3 shows the request sent by a VAL Server to a MMES for discovering a spatial mapping information provider(s).

Table 7.17.3.2-3: Spatial mapping information provider discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial mapping information provider discovery filters | M | Set of characteristics to determine spatial mapping information provider(s). The discovery filters are aligned with the information described in "Spatial mapping information details" described in table 7.17.3.2-1. |

Table 7.17.3.2-4 shows the response sent by a MMES to a VAL server when discovering a spatial mapping information provider.

Table 7.17.3.2-4: Spatial mapping information provider discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Spatial mapping information provider list | O | The list of spatial mapping information providers determined based on the discovery filters. Each element includes information as described in "Spatial mapping information details" described in table 7.17.3.2-1. |
| Failure cause | O | Indicates reason for the failure |

Table 7.17.3.2-5 shows the notification sent by a MMES to a MMEC for triggering a spatial mapping information session from a provider.

Table 7.17.3.2-5: Spatial mapping information notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMES, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial mapping information provider identifier | M | The identifier of the spatial mapping information provider (e.g., UE identifier, VAL client identifier). |
| Spatial mapping information reporting details | M | Set of characteristics used for spatial mapping information reporting. |
| > SMI identifier | M | The identifier of spatial mapping information. |
| > SMI reporting duration | O | The duration for which the spatial mapping information is required. |
| > SMI reporting frequency | O | The frequency at which a spatial mapping information is required. |
| > SMI reporting endpoint | M | The endpoint where to report the spatial mapping information. |

### 7.17.4	Solution evaluation

This solution is for KI#4 and addresses the open issue of how to enable UE(s) to provide spatial mapping information for maintaining a spatial map.

This solution can be realized according to architectures described in clause 6.1 or clause 6.3 or a new SEAL server and client. This solution proposes that the mobile metaverse service provides capabilities for registering spatial mapping information providers by a MMEC or VAL client with the MMES and capabilities for requesting spatial mapping information from these providers to maintain spatial maps.

## 7.18	Solution #18: Discovery and requirements translation for 5G-enabled Traffic Flow Simulation and Situational Awareness

### 7.18.1	Solution description

In Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness use case (as discussed in 3GPP TR 22.856 [3]), real-time information and data about the real objects can be delivered to the virtual objects of the road infrastructure and traffic participants including vulnerable road users can form a smart transport metaverse as shown in Figure 7.18.1.1. Then real-time processing and computing can be conducted to support traffic simulation and also situational awareness and real time path guidance and real-time safety, or security alerts can be generated for ICVs as well as the driver and passengers.

Figure 7.18.1.1: Use case of 5G-enabled Traffic Flow Simulation and Situational Awareness [3GPP TR 22.856]

To support such use case, it is essential to discover the identities of the virtual UEs (as counterparts of the physical UEs) and identify the sessions required as well as the per session requirement in both physical and virtual world.

This solution maps to KI #5. This solution provides a mechanism applicable to mobile metaverse services (especially for 5G-enabled Traffic Flow Simulation and Situational Awareness as discussed in Stage 1) to support:

-	the discovery of avatar and physical users for a mobile metaverse service and a given service area.- the translation of metaverse-specific service requirements to provisioning policies and parameters for the involved application sessions within the mobile metaverse service.

NOTE:	In this solution, the sessions include UE1 and UE2 and avatar counterparts of UE1 and UE2 (called as Avatar UEs for simplicity; however, referring to the digital users or virtual objects or digital twins corresponding to the UEs). Such avatar UEs may be deployed in one or more DNs.

Figure 7.18.1-2: Procedure for discovery and requirements translation

1.	The Mobile Metaverse Specific Server (MMSS) at the DN side sends an application service requirement message to the NRM server (including mobile metaverse enablement capabilities) for configuring the communication service/network requirements for a mobile metaverse service. This requirement comprises a set of application performance (QoE, QoS) and availability targets for the mobile metaverse service, as well as the target area and time of validity.

NOTE 1: Details on the mobile metaverse enablement capabilities will be provided during normative work.

2.	The NRM Server translates the application requirement to a network communication requirement for the mobile metaverse service. Such requirement includes the communication means / methods, the traffic patterns (which can be the application traffic schedules for the application sessions), the network topological area for which the service applies.

3.	The NRM Server, based on the translated network requirements, obtains information (identities/addresses, metaverse-related capabilities, and location info) for all the registered/connected UEs within the network coverage area where the metaverse service plans to be deployed. For location information, such retrieval can be based on SEAL LMS service for receiving the list of UEs and their locations in each area/zone (see clause 9.3.12 in 3GPP TS 23.434 [8]), or by querying this information from the UEs (via NRM clients) for the area of interest (see clause 9.3.4 and clause 9.3.5 in 3GPP TS 23.434 [8]).

4.	The NRM server upon obtaining the information on UEs and their status in a given area, detects the UEs which are metaverse-capable and are connected to the 3GPP network. Metaverse-capability is detected based on whether there is a mobile metaverse specific application at the target VAL UEs and can be defined as a capability of a VAL UE having an avatar counterpart (as virtual object or digital twin) in the metaverse world.

NOTE 2: For non-metaverse capable UEs, the NRM server may trigger the creation of a digital twin of the VAL UE to allow the interaction between this VAL UE and the virtual object (e.g. for sensor data sharing, traffic assistance). The triggering procedure and the creation of a digital twin / avatar of the VAL UE is out of scope of this solution.

5.	Then, the NRM server identifies the application sessions required for the end-to-end service and translatthe network requirements to different per session performance requirements for expected communications among the discovered avatar - physical UEs (also considering their relative location in both digital and physical world). Such identification is based on the communication needs among Meta App residing in the VAL UEs (in the physical world), as well as communication between a Meta App in the VAL UE and the VAL server where the avatar of the VAL UE is located (e.g., EDN) for supporting the Traffic Flow Simulation and Situational Awareness use case. As example, sessions will be established for all VAL UEs (among Meta Apps in VAL UEs) and between each per of VAL UE and its avatar, within the target area of interest.

NOTE 3: For the application QoS requirements setting and coordination, this step may re-use NRM capability for coordinated QoS provisioning operation in network assisted UE-to-UE communications as in clause 14.3.5.3 of 3GPP TS 23.434 [8]. This will be further explored in normative phase.

6.	The NRM server sends the service provisioning parameters and policies for the metaverse-specific sessions and the given area to the involved UEs and MMSS. The provisioning parameters may include per session QoS requirements for each metaverse session received/sent at/by the UE, where each metaverse session may be composed of multi-modal type of communication conveying audio, video or haptic/sensor information from/between the UE.

### 7.18.2	Architecture Impacts

This solution requires an enhanced capability at the SEAL NRM for supporting the discovery and requirements translation for metaverse services.

### 7.18.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #x.

-	MM-specific service requirements API (request / response model or subscribe/notify; API provider: SEAL NRM server; known consumer: MMSS, VAL server; corresponding to step 1).

-	MM-specific provisioning policy API (command/ notify; API provider: SEAL NRM server; known consumer: VAL UE, VAL server; corresponding to step 6).

### 7.18.4	Solution evaluation

This solution provides a mechanism for discovery and requirement translation for mobile metaverse services. This solution is only applicable to metaverse applications like “Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness” as discussed in Stage 1 since the sessions are for both physical and avatar UEs.

This solution provides enhancements to SEAL NRM services, and in particular it introduces a new capability for mobile metavese requirements translation and also utilizes existing SEAL NRM capability on coordinated QoS provisioning operation in network assisted UE-to-UE communications as provided in 3GPP TS 23.434 clause 14.3.5.3 [8].

Comparing to existing SEAL NRM procedure for network resource adaptation as provided in clause 14.3.3.3.1 of 3GPP TS 23.434 [X], the mobile metaverse requirements translation capability is non-overlapping since it is not about translating a VAL requirement to a network resource requirement, but to network communication requirement including communication means / methods, the traffic patterns and the network topological area. However, possible enhancement of this capability to support metaverse requirements translation could be further explored in normative phase.

The solution is feasible and doesn’t have any dependency to other WGs.

NOTE:	Possible enhancement of existing SEAL NRM capability as provided in 3GPP TS 23.434 clause 14.3.5.3 and clause 14.3.3.3.1 will be re-visited in normative phase.

# 8	Deployment scenarios

## 8.1	General

This clause describes different deployment models to provide metaverse services to the end user.

## 8.2	Deployment model #1: Deployment of enabler for Metaverse Services as SEAL server(s)

Figure 8.2-1 illustrates deployment of the metaverse services as SEAL server(s) in a single PLMN operator domain and the VAL server(s) in the VAL service provider domain.

Figure 8.2.1-1: metaverse services as SEAL server(s)

The SEAL server(s) can be one or more SEAL services, providing enablement services to the metaverse application servers. The SEAL-S interface provides multiple service APIs towards Metaverse application server.

# 9	Business Relationships

The business relationship as specified in clause 5 of 3GPP TS 23.434 [8] is applicable for metaverse service eco system too, where VAL user is metaverse service user, VAL service provider is metaverse service provider and further, VAL service provider and SEAL service provider can be same organization.

# 10	Overall evaluation

## 10.1	Architecture evaluations

### 10.1.1	On-network mobile metaverse application layer architecture

The architecture is proposed as option #1 in clause 6.1. The architecture supports spatial anchors management (as specified in solution#1, solution #3, solution#4), avatar management (as specified in solution #6). The architecture proposes to create metaverse application enabler layer. The architecture option is a feasible option. The architecture can be considered to define new spatial anchor management service.

### 10.1.2	SEAL architecture to support metaverse applications

The SEAL based architectures are proposed as option#2 in clause 6.2 and option#3 in clause 6.3.

For avatar and/or digital asset management,

-	architecture option#2 is the basis for the solution#7 and proposes a new SEAL service; and

-	architecture option#3 is the basis for the solution#5 and proposes to re-use SEAL CMS service.

Avatar and/or Digital Asset objects are user related objects which require support for real-time interactions between the vertical application layer entities deployed in UEs or on the network.

As the avatar and/or digital assets can be consumed by several vertical applications including metaverse applications, it is feasible to support the avatar and/or digital asset management APIs via a new SEAL architecture option.

NOTE:	The avatar/digital asset based session management is not within the scope of digital avatar management.

## 10.2	Key issue evaluations

### 10.2.1	Key issue #1: Enabler support for managing spatial anchors

Solution #1, Solution #2 and Solution #3 addresses the key issue 1.

Solution #1 resolves open issue 2) and provides spatial anchors discovery procedure based on architecture option#1.

Solution #2 resolves open issue 1) and provides spatial anchors create, update, get and subscription procedures based on enhancing SEAL LMS.

Solution #3 resolves open issue 3) and provides spatial anchor subscription procedure.

All 3 solutions can be considered for normative work. Solution 1 can be combined with get procedure of solution #2. The procedure in clause 7.3.1.3 (VAL client subscription procedure for spatial anchors in range) can be merged with the procedure in clause 7.2.3.4 (Spatial anchor information subscription).

### 10.2.2	Key issue #2: Exposure of user sensitive information

Solution #X addresses key issue #2 by providing a mechanism for servers to avoid servicing requests for which user consent has not been granted for the information (e.g., the request body) provided by the client in such requests.

### 10.2.3	Key issue #3: Avatar and digital asset support

Solution #5, Solution #6, Solution #7 and Solution #9 addresses the key issue 3.

Solution #5 proposes to manage avatar profile by enhancing SEAL CM service. The solution proposes the create, update, get, delete avatar profile. The solution also proposes to subscribe for avatar profile.

Solution #6 proposes to manage avatar using MDRB information block managed by MMES. The solution proposes generic framework for avatar support.

Solution #7 proposes new SEAL service to manage digital avatar. It proposes to support functionalities for avatar profile creation along with other avatar related functions like avatar upload, download, update, delete and linking to user/subscriber.

Solution #9 proposes avatar discovery procedure.

The Avatar profile proposed in Solution #5, Solution #6, Solution #7 and Solution #9 will be considered for merger in normative work, while avatar based session related parameters (like Communication parameters and Metaverse experience requirements) can be considered for another SEAL service (e.g. SEAL DD).

Solution #9 can be consider for merging with clause 7.5.3.3 in normative work.

NOTE 1:	Whether and which IE is valid in MDRB and avatar profile, and how to merge them will determine in normative phase.

NOTE 2:	Whether VAL server can create/update/delete avatar profile or not needs further discussion.

### 10.2.4	Key issue #4: Spatial mapping

Solution #8 addresses the key issue 4 by enhancing SEAL LM server.

Solution #8  resolves open issue 1) and 2) by enhancing SEAL-LM client and server functionalities, and provides procedures to produce, update, get and subscribe spatial maps.

Solution #8 can be adopted in the normative work by defining new SEAL server for spatial map management.

Solution #15 supports spatial map augmented with VAL UE. This solution can be considered to merged with solution #8 in normative phase.

Solution #16 resolves open issue 1) and 2) by providing spatial map exposure and discovery service when a VAL server provides a spatial mapping service. Solution #16 is based on architecture option #1 and can be adapted to other proposed architectures options. The following aspects of Solution #16 can be considered for normative work:

1.	The VAL server can inform the MMES of the VAL server’s spatial map coverage (Solution #16, clause 7.16.1.2).

2.	The MMES can inform the consumer about the VAL server when the MMES does not have coverage for a requested area (e.g., solution #8).

3.	Reuse of CAPIF for the publish and discovery of the VAL server’s spatial map services will be considered in normative work.

Solution #17 resolves open issue 1) by providing registration and discovery of spatial mapping information (e.g. sensor data) provider entities. The solution enables a VAL server to discover sensor devicews which can provide spatial mapping information to maintain spatial map. Solution #17 is based on architecture option #1 and can be adapted to other proposed architecture options. The solution can be considered for the normative work.

Solution #8, Solution #16 and Solution #17 are complementary and can be combined in the normative phase.

NOTE: The aspects of Solution #16 considered for normative phase are described in clause 10.2.4.

In this release, VAL server can consume the spatial mapping enabler service, can discover UE(s) per Solution #17, and also act for providing sensor data to spatial map enabler service.

### 10.2.1	Key issue #5: Support for avatar discovery and QoS control

Solution #9, Solution #10 and Solution #11 address the key issue 5.

Solution #9 resolves open issue 1) and supports discovering avatars using mobile metaverse enablement service.

Solution #10 provides digital asset synchronization services to mobile metaverse VAL applications. The procedure allows for QoS coordination and is relevant to metaverse applications requiring synchronization of digital asset information.

Solution #11 resolves open issue 2) and supports the application QoS setting and coordination for the multi-modal sessions involved in a mobile metaverse service.

Solution #18 resolves open issue 1) and enhances SEAL NRM to support the discovery and requirements translation for 5G-enabled Traffic Flow Simulation and Situational Awareness use case.

All 4 solutions can be considered for normative work. Solution 9 can be re-used for Solution 11 for supporting the discovery of avatars (as pre-condition). Solution 10 and 11 are not overlapping, since Solution 11 tackles the QoS attributes coordination among metaverse sessions to ensure meeting the application service requirements, whereas the Solution 10 addresses the synchronization of the digital asset related sessions. Solution X is complementary to other solutions, since Solution 9 can be re-used for the avatar discovery after performing the translation of requirements, and Solution 11 follows Solution 9 if QoS coordination is needed in the run-time phase.

NOTE:	For solution #11 and Solution #12, possible enhancement of existing SEAL NRM capability as provided in clause 14.3.5.3 of 3GPP TS 23.434 [X] will be re-visited in normative phase.

### 10.2.6	Key issue #6: Support device discovery to offload task for metaverse services

Solution #12 addresses the key issue 6.

The solution uses PINAPP architecture and provides procedure to discover other PIN elements to offload the task.

### 10.2.7	Key issue #7: Support for metaverse services requiring multiple devices

Solution #13 addresses the key issue 7 and can be considered for normative work.

Solution #13 resolves open issue 1 of KI #7 by enhancing existing EDGEAPP architecture and procedures.

NOTE:	Whether application specific mechanisms are sufficient or enable enhancements are required needs further discussion in the normative work.

### 10.2.8	Key issue #8 solutions evaluation

The open issue of KI#8 includes:

1)	How to combine one or more mobile metaverse services associated with spatial anchors?

2)	How to provide the exposure of the spatial anchor analytics to the consumers?

For open issue#1, there are two ways in solution#1 to find a group of spatial anchors:

-	Way#1: preconfiguring the high-level abstraction for a group of spatial anchors linked with each other in spatial anchor create procedure. This enables to combine multiple spatial anchors at a location. The discovery procedure enables the consumers to discover individual spatial anchors and group of spatial anchors(if any available in the requested location) based on the discovery filter request. This solution solves the issue to differentiate between the the group based SA and individual or single spatial anchors using group ID. The group ID based combination of spatial achors also allows to include different SA.

NOTE 1: The procedure to combine multiple spatial anchors can be discussed further.

-	Way#2: giving the description or label of spatial anchor in spatial anchor create procedure without preconfiguring group spatial anchor information. The group spatial anchor may be returned in spatial anchor discovery procedure based on the spatial anchor specific information (e.g. user preference information) and the description or label of spatial anchor.

NOTE 2: The procedure of giving the description or label in way#2 will further discusss in normative work.

For open issue#2, solution #4 provides the spatial anchor analytics information. The procedure is based on MMES to generate the spatial anchor analytics information analytics. It is feasible and do not introduce any new requirement to the existing 5GS.

# 11	Conclusions

## 11.1	Architecture conclusions

The study concludes with following architectural considerations for the normative work:

1.	For Key issue #1 Enabler support for managing spatial anchors, it is concluded to define new SEAL server to support spatial anchor management. The new SEAL server will consider to re-use the architecture as specified in option#1.

2.	For key issue#3 and key issue#5 related to avatar and/or digital asset management, it is concluded to define new SEAL server to support the corresponding service APIs. The architecture option#2 specified in clause 6.2 is considered as the basis for normative work.

## 11.2	Conclusions of key issues

The study concludes with following solution considerations for the normative work:

1.	Following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

ii.	for Key issue #2 (Exposure of user sensitive information): Solution #X has been proposed to address this key issue.

Editor's Note:	Coordination with SA3 is required prior to considering Solution #X in the normative phase.

iii.	for Key issue #3 (Avatar and digital asset support): Solution #5, Solution #6, Solution #7 and Solution #9 can be consider for normative work with possible mergers as specified in clause 10.2.x. The solutions will be adapted to the architecture option concluded in bullet 2 of clause 11.1 corresponding to avatar/digital asset management.

iv.	for Key issue #4 (Spatial mapping): Solution #8 , Solution #15, Solution #16 and Solution #17 can be considered for the normative work by defining a new SEAL server for spatial map management with necessary changes to adapt to the new SEAL server.

v.	for Key issue #5 (Support for avatar discovery and QoS control): Solution #9, Solution #10, Solution #11, and Solution #X can be considered for normative work. All solutions will be adapted to the conclusion of the architecture option in the normative work. Possible re-use or enhancement of existing solutions for QoS coordination (e.g., in SEAL NRM or SEALDD) will be considered in normative work.

vi.	for Key issue #6 (Support device discovery to offload task for metaverse services) Solution #12 can be considered for normative work.

vii.	for Key issue #7 (Support for metaverse services requiring multiple devices): The feasibility and scenario of Solution#13 is not confirmed in TR phase, we can further discuss it in normative phase..

viii.	For Key issue #8(Enabler support for spatial anchors based services): Solution #1, Solution#4 can be consider for normative work. The second way of solution#1 as described in clause 10.2.x can be used to support the group anchor discovery. All solutions will be adapted to the conclusion of the architecture option in the normative work.

ix.	for Key issue #9 (Support for permission control of digital assets): The architecture and solution for this KI can be discussed in normative phase.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA6#58 | S6-233940 |  |  |  | FS_MetaApp - Skeleton for new TR | 0.0.0 |
| 2023-12 | SA6#58 | S6-234040 |  |  |  | FS_MetaApp - Scope | 0.1.0 |
| 2023-12 | SA6#58 | S6-234059 |  |  |  | Pseudo-CR on KI on Spatial anchor enabler | 0.1.0 |
| 2023-12 | SA6#58 | S6-234086 |  |  |  | KI on sensitive information exposure | 0.1.0 |
| 2024-02 | SA6#59 | S6-240631 |  |  |  | Digital avatars KI | 0.2.0 |
| 2024-02 | SA6#59 | S6-240638 |  |  |  | SA2 linkage to KI on User Sensitive Information | 0.2.0 |
| 2024-02 | SA6#59 | S6-240728 |  |  |  | Pseudo-CR on application enablement architecture for mobile metaverse services | 0.2.0 |
| 2024-02 | SA6#59 | S6-240732 |  |  |  | KI Spatial mapping | 0.2.0 |
| 2024-02 | SA6#59 | S6-240735 |  |  |  | Pseudo-CR on update of KI on exposure of user sensitive information | 0.2.0 |
| 2024-02 | SA6#59 | S6-240750 |  |  |  | Pseudo-CR on New solution on spatial anchor discovery | 0.2.0 |
| 2024-02 | SA6#59 | S6-240766 |  |  |  | KI on discovery and QoS control for metaverse services | 0.2.0 |
| 2024-02 | SA6#59 | S6-240775 |  |  |  | KI#1 Update | 0.2.0 |
| 2024-02 | SA6#59 | S6-240786 |  |  |  | Pseudo-CR on Solution for spatial anchors management | 0.2.0 |
| 2024-04 | SA6#60 | S6-241474 |  |  |  | _FS_Metaverse_App - definitions | 0.3.0 |
| 2024-04 | SA6#60 | S6-241589 |  |  |  | FS_Metaverse_App-New KI on PINAPP enhancement | 0.3.0 |
| 2024-04 | SA6#60 | S6-241590 |  |  |  | Pseudo-CR on Enhancements to EDGEAPP to support metaverse services | 0.3.0 |
| 2024-04 | SA6#60 | S6-241478 |  |  |  | update solution#2 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241617 |  |  |  | Pseudo-CR on New solution for Spatial Anchor subscription | 0.3.0 |
| 2024-04 | SA6#60 | S6-241480 |  |  |  | Pseudo-CR on Solution #1 EN resolution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241561 |  |  |  | Addition of New Study Aspects to the Key Issue#1 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241625 |  |  |  | Enhancement to the update solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241562 |  |  |  | Update to spatial anchor discovery solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241624 |  |  |  | Update to the spatial anchor get solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241665 |  |  |  | New solution on the spatial anchor analytics related information | 0.3.0 |
| 2024-04 | SA6#60 | S6-241086 |  |  |  | KI on User Sensitive Information update | 0.3.0 |
| 2024-04 | SA6#60 | S6-241488 |  |  |  | FS_Metaverse_App-Update to KI 3 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241645 |  |  |  | FS_Metaverse_App-Solution for digital avatar management | 0.3.0 |
| 2024-04 | SA6#60 | S6-241507 |  |  |  | Avatar support | 0.3.0 |
| 2024-04 | SA6#60 | S6-241661 |  |  |  | SEAL Architecture Option to support Digital Avatars | 0.3.0 |
| 2024-04 | SA6#60 | S6-241662 |  |  |  | Solution for KI#3 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241638 |  |  |  | Pseudo-CR on Solution for Spatial mapping | 0.3.0 |
| 2024-05 | SA6#61 | S6-242560 |  |  |  | architecture requiremens and deployment models | 0.4.0 |
| 2024-05 | SA6#61 | S6-242715 |  |  |  | Digital Asset Identifier | 0.4.0 |
| 2024-05 | SA6#61 | S6-242562 |  |  |  | Solution 3 – Update | 0.4.0 |
| 2024-05 | SA6#61 | S6-242564 |  |  |  | Solution evaluations for sol 1 and 2, overall evaluation and conclusion | 0.4.0 |
| 2024-05 | SA6#61 | S6-242663 |  |  |  | MetaverseApp KI #3 update | 0.4.0 |
| 2024-05 | SA6#61 | S6-242684 |  |  |  | MetaverseApp solution 6 update for framework alignment | 0.4.0 |
| 2024-05 | SA6#61 | S6-242068 |  |  |  | MetaverseApp avatar discovery solution | 0.4.0 |
| 2024-05 | SA6#61 | S6-242685 |  |  |  | Pseudo-CR to resolve Solution #8 ENs | 0.4.0 |
| 2024-05 | SA6#61 | S6-242569 |  |  |  | MetaverseApp avatar synch support solution | 0.4.0 |
| 2024-05 | SA6#61 | S6-242570 |  |  |  | New Solution to KI # 5 | 0.4.0 |
| 2024-05 | SA6#61 | S6-242632 |  |  |  | Solution for KI 6 | 0.4.0 |
| 2024-05 | SA6#61 | S6-242728 |  |  |  | Solution for KI 7 | 0.4.0 |
| 2024-05 | SA6#61 | S6-242634 |  |  |  | Introducing a SA services group | 0.4.0 |
| 2024-05 | SA6#61 | S6-242635 |  |  |  | Introducing SA services QoS and discovery levels | 0.4.0 |
| 2024-05 | SA6#61 | S6-242723 |  |  |  | new KI on permission control over digital assets | 0.4.0 |
| 2024-05 | SA6#61 | S6-242637 |  |  |  | Reuse existing SEAL architecture for metaverse services | 0.4.0 |
| 2024-05 | SA6#61 | S6-242043 |  |  |  | Architecture - Option 1 update | 0.4.0 |
| 2024-05 | SA6#61 | S6-242749 |  |  |  | conclusion on architecture options | 0.4.0 |
| 2024-05 | SA6#61 | S6-242749 |  |  |  | Reapplication of pCR S6-242749 and editorial corrections | 0.4.1 |
| 2024-06 | SA#104 | SP-240736 |  |  |  | Submitted to SA#104 for information | 1.0.0 |
| 2024-07 | SA6#62-AE | S6a240275 |  |  |  | Pseudo-CR on Solution for KI 2 on exposure of user sensitive information | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240371 |  |  |  | Solution for spatial map management | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240372 |  |  |  | New solution for spatial computing service discovery | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240303 |  |  |  | New solution for enabling spatial mapping information providers | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240217 |  |  |  | Spatial map services | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240284 |  |  |  | Address EN With The New Solution#4 | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240373 |  |  |  | Clause 7.5 EN | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240343 |  |  |  | Spatial map management API | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240344 |  |  |  | Architecture evaluation for SEAL | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240207 |  |  |  | update solution#5 for digital asset identifier | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240208 |  |  |  | update solution#6 for digital asset identifier | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240209 |  |  |  | update solution#7 for digital asset identifier | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240210 |  |  |  | update solution#10 for digital asset identifier | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240254 |  |  |  | Spatial map management service requirements | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240241 |  |  |  | Solution #8 – updated solution evaluation | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240214 |  |  |  | Specification cleanup | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240345 |  |  |  | General requirements | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240140 |  |  |  | Pseudo-CR on business relationship | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240346 |  |  |  | Resolve EN related to spatial anchors architecture | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240347 |  |  |  | solution evaluations and conclusion for KI 3 | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240143 |  |  |  | Resolving ENs for solution #5 | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240348 |  |  |  | Update to solution 8 | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240349 |  |  |  | Update solution#2 to support multiple anchor discovery | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240324 |  |  |  | Solution 9 update | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240350 |  |  |  | Clause 4.3 EN | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240326 |  |  |  | Avatar and asset management requirements | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240328 |  |  |  | Clause 7.6 EN | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240351 |  |  |  | Solution 10 update | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240301 |  |  |  | Solution 12 update | 1.1.0 |
| 2024-07 | SA6#62-AE | S6a240198 |  |  |  | Update to architecture conclusion | 1.1.0 |
| 2024-07 | SA6#62-AE | S6-240342 |  |  |  | Enhance Architecture Option#2 for support permission control of digital assets | 1.1.0 |
| 2024-08 | SA6#62 | S6-243765 |  |  |  | Conclusion for KI#9 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243762 |  |  |  | Pseudo-CR on updates to solution 13 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243687 |  |  |  | update to solution#8 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243712 |  |  |  | Updates to Spatial Map Creation | 1.2.0 |
| 2024-08 | SA6#62 | S6-243649 |  |  |  | Clarification about terminology on spatial anchor | 1.2.0 |
| 2024-08 | SA6#62 | S6-243688 |  |  |  | Pseudo-CR on KI#4 conclusio | 1.2.0 |
| 2024-08 | SA6#62 | S6-243650 |  |  |  | New Solution to KI # 5 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243460 |  |  |  | Evaluation and Conclusion of KI#5 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243746 |  |  |  | A-DACM architecture update | 1.2.0 |
| 2024-08 | SA6#62 | S6-243463 |  |  |  | Delete spatial map procedure segregated from Update procedure | 1.2.0 |
| 2024-08 | SA6#62 | S6-243464 |  |  |  | Updates To Solution#1 And Solution#2 | 1.2.0 |
| 2024-08 | SA6#62 | S6-243465 |  |  |  | Overall Evaluation For The KI#8 | 1.2.0 |