| 3GPP TR 23.700-29 V0.5.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	12

2	References	12

3	Definitions of terms and abbreviations	13

3.1	Terms	13

3.2	Abbreviations	14

4	Architectural Assumptions and Principles	14

5	Key Issues	15

5.1	Key Issue #1: Support of Regenerative-based satellite access	15

5.1.1	Description	15

5.2	Key Issue #2: Support of Store and Forward Satellite operation	15

5.2.1	Description	15

5.3	Key Issue #3: Support of UE-satellite-UE communication	15

5.3.1	Description	15

6	Solutions	17

6.0	Mapping of Solutions to Key Issues	17

6.1	Solution #1: N2/S1 disconnection for satellites leaving an area served by a CN	18

6.1.1	Description	18

6.1.2	Procedures	19

6.1.3	Impacts to Services, Entities and Interfaces	20

6.2	Solution #2 for KI#1: Mobility optimization for Regenerative-based satellite access	20

6.2.1	Description	20

6.2.1.1	TAI/CGI report over N2/S1-AP interface	21

6.2.1.2	TNL Connection Suspending over N2/S1-AP interface	21

6.2.1.3	Paging	22

6.2.2	Procedures	22

6.2.2.1	Connecting a RAN node on-board to the CN on the ground	22

6.2.2.2	Information Exchanging between RAN node and CN	22

6.2.2.2.1	RAN node Configuration for Supported TA list report	22

6.2.2.2.2	Reporting Supported TA list to the CN on the ground	23

6.2.2.3	TNL management over N2/S1-MME interface	23

6.2.2.4	Paging	24

6.2.3	Impacts on services, entities and interfaces	25

6.3	Solution #3: Support for suspend and resume of NG interface	25

6.3.1	Key Issue mapping	25

6.3.2	Functional Description	25

6.3.4	Impacts on existing services, entities and interfaces	26

6.4	Solution #4: Handling awareness of supported TAIs when RAN is onboard	26

6.4.1	Description	26

6.4.2	Procedures	28

6.4.2.1	Procedures of gNB connect to the same AMF via the same gateway	28

6.4.2.2	Procedures of gNB connect to a different AMF via a different gateway	29

6.4.2.3	Procedures of gNB connect to the same AMF via different gateway	29

6.4.3	Impacts to Services, Entities and Interfaces	29

6.5	Solution #5: Support of Feeder Link Switchover with TAU for Non-Pooled MMEs	30

6.5.1	Description	30

6.5.2	Procedures	31

6.5.3	Impacts to Services, Entities and Interfaces	32

6.6	Solution #6: Load re-balancing for feeder link switchover	32

6.6.1	Description	32

6.6.2	Procedures	32

6.6.2.1	MME procedures	32

6.6.2.2	AMF procedures	33

6.6.3	Impacts to Services, Entities and Interfaces	35

6.7	Solution #7 for Key Issue #1: Feeder link switchover with DL data continuity	35

6.7.1	Description	35

6.7.2	Procedures	35

6.7.2.1	Feeder link switchover without AMF change	35

6.7.3	Impacts to Services, Entities and Interfaces	37

6.8	Solution #8 for Key Issue #1: QoS/ Policy handling for Regenerative-based satellite access	37

6.8.1	Description	37

6.8.2	Procedures	38

6.8.2.1	General	38

6.8.3	Impacts to Services, Entities and Interfaces	39

6.9	Solution #9: Support of Regenerative-based satellite access with deploying RAN-agent and Proxy RAN node or Link Layer Proxy NF	39

6.9.1	Description	39

6.9.1.1	Support of Regenerative-based satellite access with RAN-agent and Proxy RAN node	40

6.9.1.2	Support of Regenerative-based satellite access with Link Layer Proxy	41

6.9.2	Procedures	42

6.9.2.1	NG Setup, RAN configuration Update and UE registration	42

6.9.2.2	Handover with LLP and MME relocation	43

6.9.3	Impacts to Services, Entities and Interfaces	45

6.10	Solution #10: Regenerative-based satellite access with IWF on S1 or N2/N3 interface	45

6.10.1	Key Issue mapping	45

6.10.2	Description	45

6.10.3	Procedures	47

6.10.4	Impacts on services, entities and interfaces	47

6.11	Solution #11: MME Split Architecture to support Control plane procedure for S&F	47

6.11.1	Key Issue mapping	47

6.11.2	Description	47

6.11.3	Procedures	49

6.11.3.1	Architecture enhancement to support S&F	49

6.11.3.2	Attach Procedure in EPC in S&F scenario	51

6.11.3.3	MO Data Transport in Control Plane CIoT EPS Optimisation in S&F	54

6.11.3.4	Connection Suspend and Resume Procedure	57

6.11.4	Impacts on services, entities and interfaces	60

6.12	Solution #12: S&F for multiple satellite deployment with anchor MME-ground	60

6.12.1	Description	60

6.12.2	Procedures	61

6.12.2.1.1	Periodic timer and mobile reachable timer	62

6.12.2.1.2	How UE identifies that it is allowed to attach with network for S/F	63

6.12.2.1.3	GUTI management and MME selection	63

6.12.2.1.4	MME-ground selection by MME-onboard	63

6.12.2.1.5	Authentication procedure/subscription details fetch during any other procedure	63

6.12.2.1.6	Attach with PDN connectivity, multiple satellite	63

6.12.3	Impacts on services, entities and interfaces	64

6.13	Solution #13: S&F for single satellite deployments with eNB and MME-onboard	64

6.13.1	Description	64

6.13.2	Procedures	65

6.13.2.1	Other changes:	66

6.13.2.1.1	Periodic timer and mobile reachable timer	66

6.13.2.1.2	How UE identifies that it is allowed to attach with network for S/F	66

6.13.2.1.3	UE selecting single satellite in a given area	67

6.13.2.1.4	Authentication procedure/subscription details fetch during any other procedure	67

6.13.3	Impacts on services, entities and interfaces	67

6.14	Solution #14: Initial Registration with prior rejection in satellite network for S&F operation	67

6.14.1	Key Issue mapping	67

6.14.2	Description	67

6.14.3	Procedures	69

6.14.4	Impacts on services, entities and interfaces	70

6.15	Solution #15: S&F solution for multi-satellite IoT NTN	71

6.15.1	Description	71

6.15.2	Procedures	73

6.15.2.1	Advertising capabilities and activation of S&F Satellite operation	74

6.15.2.2	NAS procedures spanning several satellite passes	74

6.15.2.3	Data transfer in S&F Satellite operation mode	78

6.15.3	Impacts to Services, Entities and Interfaces	80

6.16	Solution #16: Enabling S&F operation with C-SGN onboard	81

6.16.1	Description	81

6.16.2	Procedures	82

6.16.2.1	Procedures of NAS Pause/Recovery of S&F Operation	82

6.16.2.2 	Procedures of Store/Forward of SMS transmission	84

6.16.2.2.1	Procedure of MO SMS transmission	84

6.16.2.2.2	Procedure of MT SMS transmission	86

6.16.2.3	Procedure of Store/Forward of Data transmission	86

6.16.2.3.1	Procedure of MO Data transmission	87

6.16.2.3.2	Procedure of MT data transmission	88

6.16.2.4	Example of initial attach procedure	88

6.16.2.4.1	1st round orbiting	89

6.16.2.4.2	2nd round orbiting	90

6.16.2.4.3	3rd round orbiting	91

6.16.3	Impacts to Services, Entities and Interfaces	91

6.17	Solution #17 for KI#2: Support of Store and Forward Satellite operation for small data transmission	92

6.17.1	Description	92

6.17.1.1	General	92

6.17.1.2	Reference Architecture	92

6.17.1.3	UE Authentication for S&F and Parameter Negotiation	93

6.17.1.4	Storing and Forwarding MO Data	94

6.17.1.5	Storing and Forwarding MT Data	94

6.17.2	Procedures	95

6.17.2.1	Attach with MO data transmission	95

6.17.2.2	Tracking Area Update with MO/MT data transmission	96

6.17.2.3	Service Request for MO Data transmission	96

6.17.3	Impacts on services, entities and interfaces	98

6.18	Solution #18: Attach Procedure with MME and HSS on board the satellite	99

6.18.1	Description	99

6.18.2	Procedures	100

6.18.2.1	Procedures for E-UTRAN Attach in case the onboard HSS holds subscription and credentials for the UE	100

6.18.2.2	Procedures for E-UTRAN Attach in case no subscription and credentials for the UE is stored in the onboard HSS	102

6.18.2.3	Procedure for MO Data Transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation	104

6.18.2.4	Procedure for MT Data Transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation	105

6.18.3	Impacts to Services, Entities and Interfaces	106

6.19	Solution #19: Store and Forward Operation using a UE Proxy and Endpoint Proxy	107

6.19.1	Description	107

6.19.2	Procedures	109

6.19.2.1	Support of MO and MT Transactions	109

6.19.2.2	Support of Security	112

6.19.2.3	Transition between SSF and Real Time Modes	113

6.19.2.4	Support of Discontinuous Satellite Coverage in SSF Mode	113

6.19.3	Impacts to Services, Entities and Interfaces	114

6.20	Solution #20: Solution on enabling S&F operation and data delivery with only eNB onboard satellite	116

6.20.1	Description	116

6.20.2	Procedures	117

6.20.2.1	Attach Procedure	117

6.20.2.2	MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if feeder link is available	119

6.20.2.3	MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if the feeder link is unavailable	121

6.20.2.4	MO Data Delivery in CIoT EPS Optimisation for Store and Forward Operation	122

6.20.3	Impacts to Services, Entities and Interfaces	123

6.21	Solution #21: Support of store and forward satellite operation in EPS	124

6.21.1	Description	124

6.21.2	Procedures	125

6.21.3	Impacts to Services, Entities and Interfaces	129

6.22	Solution #22 for Key Issue #2: Support of Store and Forward Satellite operation for SMS and Data Transmission	130

6.22.1	Introduction	130

6.22.2	Description	130

6.22.3	Procedures	132

6.22.3.1	Provision of available communication information	132

6.22.3.2	MT SMS Delivery procedure related to HSS	132

6.22.3.3	MT SMS Delivery procedure related to MME	134

6.22.3.3A MO SMS Delivery procedure	136

6.22.3.4 MT/MO data transmission	137

6.22.3.5 Support for multiple satellites serving a UE	139

6.22.3.5.1 UE context synchronization among multiple satellites	139

6.22.3.5.2 MT/MO SMS delivery procedure	140

6.22.3.5.3 MT/MO data transmission procedure	140

6.22.4	Impacts on services, entities and interfaces	140

6.23	Solution #23: Support for SMS in satellite network for S&F operation	141

6.23.1	Key Issue mapping	141

6.23.2	Description	141

6.23.3	Procedures	143

6.23.3.1	Mobile Terminated SMS	143

6.23.3.2	Mobile Originated SMS	144

6.23.4	Impacts on services, entities and interfaces	144

6.24	Solution #24: Enhancement of registration and PDU session establishment to support on-board S&F	144

6.24.1	Description	144

6.24.2	Procedures	146

6.24.2.1	Option 1: Procedures of User plane enabled S&F	146

6.24.2.1.1	Procedures of registration in S&F mode	147

6.24.2.2	Option 2: Procedures of 5GC enabled S&F	148

6.24.2.2.1	Enhancement of the registration procedures	148

6.24.2.2.1	Enhancement of the session management procedures	148

6.24.3	Impacts to Services, Entities and Interfaces	148

6.25	Solution #25: UE reachability estimation	149

6.25.1	Key Issue mapping	149

6.25.2	Description	149

6.25.3.1	UE Reachability Estimation (Satellite Location/Coverage Information)	149

6.25.3.1.1	UE reachability estimation	149

6.25.3.1.2	URE and MME-T interaction and use-cases	150

6.25.3.1.3	Reachability Modes	150

6.25.4	Impacts on services, entities and interfaces	151

6.26	Solution #26: Provision of Store and Forward Satellite Specific Parameters	151

6.26.1	Description	151

6.26.2	Procedures	152

6.26.3	Impacts on services, entities and interfaces	152

6.27	Solution #27: Performance optimization for the transport of MT/MO S&F traffic	153

6.27.1	Description	153

6.27.1.1	S&F Deployment Architecture	153

6.27.1.2	Performance impact from the accumulation of stored MT/MO traffic	154

6.27.1.3	Performance optimization to forward accumulated MT/MO traffic	154

6.27.2	Procedures	156

6.27.2.1	General procedure for performance optimization	157

6.27.2.2	Enhanced procedure for S&F UE Registration	157

6.27.2.3	Enhanced procedure for S&F Service Request	158

6.27.3	Impacts on services, entities and interfaces	158

6.28	Solution #28: UE-Satellite-UE Communication via UPFs onboard the satellites	159

6.28.1	Description	159

6.28.1.1	Reference Architecture	159

6.28.1.2	Identifying UEs applicable for UE-Satellite-UE Communication	160

6.28.1.3	UL CL/BP/local PSA UPF selection for UE-Satellite-UE Communication	160

6.28.2	Procedures	161

6.28.2.1	UE-satellite-UE communication procedure in IMS system	161

6.28.2.2	Ground common SMF for UE-SAT-UE communication	163

6.28.2.3	Ground common I/V-SMF for UE-SAT-UE communication	164

6.28.2.4	Handover procedure when serving satellite changes	165

6.28.3	Impacts to Services, Entities and Interfaces	166

6.29	Solution #29: SMF activated UE-satellite-UE communication for IMS service	167

6.29.1	Description	167

6.29.2	Procedures	167

6.29.3	Impacts to Services, Entities and Interfaces	169

6.30	Solution #30: Solution to determine UE-SATs-UE eligibility and enable communication setup.	169

6.30.1	Description	169

6.30.2	Procedures	170

6.30.3	Impacts on existing nodes and functionalities	171

6.31	Solution #31: UE-satellite-UE communication for IMS voice and video (UPF onboard)	172

6.31.1	Description	172

6.31.2	Procedures	173

6.31.3	Impacts on services, entities and interfaces	173

6.32	Solution #32 for Key Issue #3: Support of UE-satellite-UE communication for IMS services with UPF/AGW inserted on satellites	174

6.32.1	Description	174

6.32.1.1	Reference Architecture	174

6.32.2	Procedures	176

6.32.2.1	Provision of access network information in 5GC	176

6.32.2.2	Provision of access network information to IMS	176

6.32.2.3	IMS procedures to determine to perform the UE-satellite-UE communication	177

6.32.2.4	IMS influence on traffic routing	179

6.32.3	Impacts to Services, Entities and Interfaces	179

6.33	Solution #33: Solution for UE-satellite-UE communication with IMS media entity onboard	180

6.33.1	Description	180

6.33.2	Procedures	182

6.33.2.1	UE-Satellite-UE Communication Signalling	182

6.33.2.2	Minimize service interruption if the Serving Satellite changes	183

6.33.3	Impacts on services, entities and interfaces	184

6.34	Solution #34: Enhancement on AMF discovery for Regenerative-based satellite	185

6.34.1	Description	185

6.34.2	Procedures	185

6.34.3	Impacts on services, entities and interfaces	186

6.35	Solution #35: N2/S1 procedures to handle feeder link switch	187

6.35.1	Description	187

6.35.2	Procedures	188

6.35.3	Impacts to Services, Entities and Interfaces	188

6.36	Solution #36: User plane path failure management due to  satellites leaving an area served by a CN	189

6.36.1	Description	189

6.36.2	Procedures	189

6.36.3	Impacts to Services, Entities and Interfaces	189

6.37	Solution #37: AMF Split Architecture to support Control plane procedure for S&F	190

6.37.1	Key Issue mapping	190

6.37.2	Description	190

6.37.3	Procedures	191

6.37.3.1	5GS Architecture enhancement to support S&F	191

6.37.3.2	Registration Procedure in 5GS in S&F scenario	192

6.37.3.3	UE Reachability Estimation (Satellite Location/Coverage Information)	196

6.37.3.3.1	UE reachability estimation	196

6.37.3.3.2	URE-TNFs interaction and use-cases	196

6.37.3.3.3	Reachability Modes	197

6.37.4	Impacts on services, entities and interfaces	197

6.38	Solution #38: Exposure of Registration in S&F Mode	198

6.38.1	Description	198

6.38.2	Procedures	199

6.38.3	Impacts on services, entities and interfaces	199

6.39	Solution #39: Session creation and parameters provisioning for S&F	199

6.39.1	Key Issue mapping	199

6.39.2	Description	199

6.39.3	MME, SGW and PGW onboard	200

6.39.4	Impacts on services, entities and interfaces	201

6.40	Solution #40: User plane offload from P-CSCF/IMS-AGW located on the ground	201

6.40.1	Description	201

6.40.1.1	General	201

6.40.1.2	Architecture assumption	202

6.40.2	Procedures	202

6.40.2.1	Session establishment procedure	202

6.40.2.1.1	Procedure of option 1	202

6.40.2.1.2	Procedure of option 2	205

6.40.2.2	Satellite handover procedure	207

6.40.3	Impacts on existing services, entities and interfaces	208

6.41	Solution #41: Support of UE-satellite-UE communication for IMS services with UPF/AGW on board	208

6.41.1	Description	208

6.41.1.1	Reference Architecture	209

6.41.2	Procedures	210

6.41.2.1	Procedure when serving satellite changes	210

6.41.3	Impacts to Services, Entities and Interfaces	213

6.42	Solution #42: UE-satellite-UE communication in the context of LEO satellite constellations with ISL	214

6.42.1	Description	214

6.42.2	Procedures	215

6.42.2.1	Association of Relay entity representative of satellite constellations	215

6.42.2.2	Registration Procedure	217

6.42.2.3	IMS PDU Session Establishment Procedure	218

6.42.2.4	IMS registration and voice call setup procedure	219

6.42.2.4.1	IMS registration	219

6.42.2.4.2	Voice call setup procedure - MO	220

6.42.2.4.3	Voice call setup procedure - MT	221

6.42.3	Impacts on services, entities, and interfaces	222

6.43	Solution #43: UE-Sat-UE communication using resource allocation on-board satellite	222

6.43.1	Key Issue mapping	222

6.43.2	Description	223

6.43.3	Procedures	225

6.43.3.1	Overview	225

6.43.3.2	PCC and SMF Procedures for IMS Session Establishment at terminating P-CSCF and PCF	225

6.43.3.3	PCC and SMF Procedures for IMS Session Establishment at Originating P-CSCF and PCF	228

6.43.3.4 Change of Satellite in UE-Sat-UE scenario (mobility)	231

6.43.3.4 Roaming case in UE-Sat-UE scenario	234

6.43.3.5 Support for jurisdictions where LI can apply	234

6.44	Solution #44: eNB buffered signalling and CP data transfer with eNB monitoring list	241

6.44.1	Description	241

6.44.2	Architecture	242

6.44.2	Procedures	243

6.44.2.1	Attach Procedure	243

6.44.2.2	MT control plane data transfer in S&F operation mode	245

6.44.2.3	MO control plane data transfer in S&F operation mode	246

6.44.2.4	Negotiation of S&F Monitoring List	247

6.45	Solution#45: Support SMS in S&F satellite operation	248

6.45.1	Introduction	248

6.45.2	Description	248

6.45.3	Procedures	249

6.45.3.1	MT SMS Delivery via SMS-GMSC on satellite	249

6.45.3	Impacts on Existing Nodes and Functionality	251

6.X	Solution #X: <Solution Title>	252

6.X.1	Description	252

6.X.2	Procedures	252

6.X.3	Impacts to Services, Entities and Interfaces	252

7	Overall Evaluation	252

7.1	Solution grouping for Key Issue #1: Support of Regenerative-based satellite access	252

7.2	Key Issue #2 Principles for evaluation	253

8	Conclusions	253

Annex A: Example network scenarios for regenerative payload generic architecture (e/gNB on LEO satellite)	254

Annex B: Store and Forward Satellite operation	258

Annex C: Example of network scenarios for UE-Sat-UE generic architecture (regenerative payload capabilities on LEO satellite)	260

Annex D: Change history	262

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of this Technical Report is to study the following aspects of 5GS/EPS integrating of satellite component in Release 19, including:

-	Regenerative payload generic architecture:

-	Impact to 5GS and EPS to support gNB/eNB onboard the satellite.

-	Store and Forward Satellite operation for delay tolerant services:

-	The control plane and user plane enhancements if applicable, including the minimum necessary set of Core Network elements to be embedded in the satellite, to support S&F Satellite operation, both for IoT NTN (EPS) and NR NTN (5GS);

-	The parameters needed to characterize and support S&F Satellite operation from a delay tolerant services perspective if applicable, both for IoT NTN (EPS) and NR NTN (5GS).

NOTE:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

-	UE-Satellite-UE communication enhancements for 5GS, supporting NR NTN NGSO constellation with and without ISL, with feeder link always available:

-	Following SA1 requirements, study how IMS enablers including mission critical can be supported;

-	Minimum necessary set of 5GS network functions onboard the satellite(s) to support such UE-Satellite-UE communication.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[8]	3GPP TR 22.865: "Study on satellite access Phase 3".

[9]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[10]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[11]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[12]	3GPP TS 38.413: " NG-RAN; NG application protocol (NGAP)".

[13]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[14]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[15]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[16]	IETF RFC 7976: "Updates to Private Header (P-Header) Extension Usage in Session Initiation Protocol (SIP) Requests and Responses" (https://www.rfc-editor.org/rfc/rfc7976.txt).

[17]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[18]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[19]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[20]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)".

[21]	3GPP TS 23.334: "IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) - IMS Access Gateway (IMS-AGW) interface: Procedures descriptions".

[22]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[23]	3GPP TS 33.401: “3GPP System Architecture Evolution (SAE); Security architecture”.

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 23.401 [5] and TS 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)). Depending on the orbit, the serving satellite is covering a given geographic area for a limited period of time.

S&F Satellite operation: operation mode providing communication service (in storing and forwarding information) to a UE in periods of time and/or geographical areas in which the serving satellite is not simultaneously connected to the ground network via feeder link or ISL. For the case of UL, "store" refers to on-board storage of UL information from UE and "forward" refers to forwarding of stored UL information to the ground network. For the case of DL, "store" refers to on-board storage of DL information from the ground network and "forward" refers to forwarding of stored DL information to the UE.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [6].

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ISL	Inter-Satellite Link

NTN	Non-Terrestrial Network

S&F	Store and Forward

SSF	Satellite S&F

SSFC	SSF Centre

# 4	Architectural Assumptions and Principles

The following architecture assumptions are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline.

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [5] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [7] are used as a baseline.

-	At least eNB/gNB is assumed to be on board the satellite.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	Store and Forward Satellite Operation assumes that UE-satellite-ground network connectivity can be intermittent as defined in clause 3.1.

-	Store and Forward Satellite Operation shall work without ISL.

-	NR UE-Satellite-UE communication assumes a connection to the ground is always available.

-	NR UE-Satellite-UE communication assumes only IMS services including MMTEL and mission critical communications are considered.

-	The enhancement for NR UE-Satellite-UE communication assumes a minimum set of core network entities, including at least UPF, is present onboard of the satellites.

-	NR UE-Satellite-UE communication assumes that the serving satellite will be of MEO or of LEO type.

-	NR UE-Satellite-UE communication for MMTEL service assumes communication between two UEs under the coverage of the same satellite or different satellites.

NOTE:	The study of the scenario when two UEs are under the coverage of the same satellite will be prioritized.

# 5	Key Issues

## 5.1	Key Issue #1: Support of Regenerative-based satellite access

### 5.1.1	Description

The deployment of an eNB or an gNB on a satellite for regenerative based satellite access for LEO/MEO deployment is to be studied. The aspects to be studied include:

-	Identify and study whether there is any impact on 5GS and EPS to support an gNB/eNB embedded on a satellite:

-	any impact of RAN nodes changing for any given 5GC/EPC and for a given area in the case of RAN nodes moving.

## 5.2	Key Issue #2: Support of Store and Forward Satellite operation

### 5.2.1	Description

S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS). To support S&F Satellite operation for such services, it is proposed to study the following items:

-	If applicable, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite(s) for the intended service(s);

-	Whether and how to trigger S&F Satellite operation, and how to execute S&F Satellite operation;

-	What enhancements on the related UE and network procedures are needed to support S&F Satellite operation, including:

-	Whether to inform the UE when the S&F Satellite operation is applied or not.

NOTE  1:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

NOTE 2:	Coordination with SA3-LI on LI aspects is needed.

NOTE 3:	The candidate solutions need to indicate which service(s) they are addressing (CIoT CP Optimizations, CIoT UP Optimizations, SMS).

## 5.3	Key Issue #3: Support of UE-satellite-UE communication

### 5.3.1	Description

UE-satellite-UE communication refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network.

It is expected that a link to the ground, via ISL to another satellite with a link to a ground gateway, or via multiple other satellites using ISLs to another satellite with a link to ground gateway or direct from the serving satellite to a ground gateway, is always available.

To support UE-satellite-UE communication for IMS voice and video as well as MCPTT services, the following issues are proposed to be studied:

-	Architectural enhancement to support UE-satellite-UE communication, including minimum necessary set of 5GC network functions and IMS components (if any) onboard the satellite(s), including:

-	How to ensure routing of user plane traffic (for MMTEL and MC applications) remains in the satellite.

-	Potential impacts on IMS procedures and functions to manage UE-satellite-UE communication.

-	Conditions (such as eligibility, restrictions, authorization …) for realizing UE-satellite-UE communication for UEs.

-	Minimize service interruption if the Serving Satellite changes.

The solutions are assumed to:

-	Minimize any potential UE, RAN, CN or IMS impacts.

-	Support regulatory service requirements, and PLMN country or regional service restrictions.

NOTE 1:	Emergency call service is out of scope of UE-sat-UE communication.

NOTE 2:	Coordination with SA WG3-LI is required before any potential solutions are concluded on to ensure they meet all regulatory requirements.

NOTE 3:	SA WG6 is assumed to determine what Mission Critical services can be supported. Possible impacts on the MCPTT services or other applicable MC services need to be studied by SA WG6.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 | X |  |  |  |
| 6 | X |  |  |  |
| 7 | X |  |  |  |
| 8 | X |  |  |  |
| 9 | X |  |  |  |
| 10 | X |  |  |  |
| 11 |  | X |  |  |
| 12 |  | X |  |  |
| 13 |  | X |  |  |
| 14 |  | X |  |  |
| 15 |  | X |  |  |
| 16 |  | X |  |  |
| 17 |  | X |  |  |
| 18 |  | X |  |  |
| 19 |  | X |  |  |
| 20 |  | X |  |  |
| 21 |  | X |  |  |
| 22 |  | X |  |  |
| 23 |  | X |  |  |
| 24 |  | X |  |  |
| 25 |  | X |  |  |
| 26 |  | X |  |  |
| 27 |  | X |  |  |
| 28 |  |  | X |  |
| 29 |  |  | X |  |
| 30 |  |  | X |  |
| 31 |  |  | X |  |
| 32 |  |  | X |  |
| 33 |  |  | X |  |
| 34 | X |  |  |  |
| 35 | X |  |  |  |
| 36 | X |  |  |  |
| 37 |  | X |  |  |
| 38 |  | X |  |  |
| 39 |  | X |  |  |
| 40 |  |  | X |  |
| 41 |  |  | X |  |
| 42 | X |  | X |  |
| 43 |  |  | X |  |
| 44 |  | X |  |  |
| 45 |  | X |  |  |

## 6.1	Solution #1: N2/S1 disconnection for satellites leaving an area served by a CN

### 6.1.1	Description

This is a solution addressing Key Issue #1.

The solution assumes that an eNB/gNB is on-board the satellite, and the EPC/5GC is on ground.

When a LEO satellite with an on-board eNB/gNB leaves an area served by a certain core network, there is no use of the N2/S1 interface anymore. It may even be that the satellite has no longer any ground connectivity towards that specific 5GC. To avoid errors in AMF/MME due to stale N2/S1 connections, e.g. AMF/MME sending paging requests or AMF configuration updates to a unavailable gNB/eNB, a graceful removal of the N2/S1 interface connection is proposed in this solution.

This solution is based on the following basic principles and assumptions:

-	Enhancements to the N2 and S1 procedures so that an eNB/gNB on-board a satellite can disconnect the N2/S1 interface when the eNB/gNB leaves an area served the core network (CN).

-	If/when the satellite returns to the area served by the CN, a new N2/S1 interface setup is initiated based on the existing procedure.

NOTE:	The protocol aspects and necessary changes in related procedures of the NGAP/S1AP disconnect is to be determined by RAN3. The procedures, if agreed for normative work, would be documented in TS 36.413 [14] and TS 38.413 [12]. This solution only shows an example of how it could be done.

Editor's note:	It is FFS whether Earth-fixed cells and Earth-fixed TAs are used.

Editor's note:	It is FFS the eNB/gNB "setting" on the horizon passes control on per-cell basis to the eNB/gNB "rising" on the horizon, or whether the "setting" and "rising" eNB/gNB use different Cell IDs and TAIs.

Editor's note:	It is FFS whether the AMF perceives a different Global gNB ID for each satellite serving the same geographical area.

### 6.1.2	Procedures

Figure 6.1.2-1: Release of N2/S1 interface

1.	The gNB/eNB knows, based on its ephemeris and configuration, that it is about to stop serving a certain 5GC/EPC area and it is about to lose the connectivity to the 5GC/EPC via a ground connection.

Before triggering a disconnect of the N2/S1 interface, it is assumed that the gNB/eNB has e.g. triggered a handover (or a release in case of NB-IoT) to move the connected mode UEs to another gNB/eNB that is serving the area. How this is done is based on pre-rel-19 procedures and not impacted by this solution.

2.	The gNB/eNB sends a N2/S1 Disconnect Request for all active N2/S1 connections to the 5GC/EPC.

3.	The AMF/MME replies with a N2/S1 Disconnect Response.

4.	The gNB/eNB releases any UE context associated to the N2/S1 connections and removes the N2/S1 connection.

5.	The AMF/MME releases any associated UE to the RRC_IDLE state and removes the N2/S1 connection.

Figure 6.1.2-2: Setup of N2/S1 interface

1.	The gNB/eNB knows, based on its ephemeris and configuration, that it starts serving a certain AMF/MME.

2.	The gNB/eNB sends a N2/S1 SETUP REQUEST to the AMF/MME, based on existing N2/S1 procedures.

3.	The AMF/MME provides a reply.

### 6.1.3	Impacts to Services, Entities and Interfaces

NG-RAN and E-UTRAN:

-	Support for detecting when satellite is about to stop serving (and/or lose ground connectivity to) a given AMF/MME and trigger N2/S1 disconnect to release the N2/S1 connection.

MME/AMF:

-	Support for N2/S1 procedure to release N2/S1 connection and delete corresponding context.

## 6.2	Solution #2 for KI#1: Mobility optimization for Regenerative-based satellite access

### 6.2.1	Description

This is a candidate solution for Key Issue #1: Support of Regenerative-based satellite access considering the following aspects:

-	How does the RAN node connect to an AMF on the ground, including what information will be exchanged between RAN and CN?

-	How to efficiently manage N2 connections due to the RAN node's movement?

-	How does RAN node broadcast TAI? And how does an AMF page a UE?

-	How to switch a UE without handover capability from one RAN node to another?

The basic reference architecture supporting regenerative-based satellite access is shown below.

Figure 6.2.1-1: Regenerative-based satellite access

The assumptions of this solution include:

-	A gNB moves along with the satellite on which it mounts.

-	A TA (tracking area) is fixed to the geographic area.

#### 6.2.1.1	TAI/CGI report over N2/S1-AP interface

In existing procedures, the RAN node initiates the NG/S1 Setup procedure to report the Supported TA List to the AMF/MME. However, RAN node mobility will lead to continuous change of the TAs supported by the RAN node, and then cause frequent message exchange between RAN nodes on board and the core network on the ground if the existing NG/S1 protocol is still applied.

This solution proposes the following enhancements to reduce the messages exchanged for TAI/CGI reports over the N2/S1-AP interface:

-	The on-board RAN nodes may not notify the AMF/MME of the supported TA List or may periodically notify the AMF/MME of the supported TA List. The period for updating the Supported TA List may be pre-configured (e.g. up to the operator's policy) or up to the implementation

-	The AMF/MME should be configured with ephemeris information of all on-board (ng-)eNB by O&M. The AMF/MME can calculate the TA(s) that the RAN node can serve based on ephemeris information and can correct errors of the TAC based on the reported TA list by the RAN node.

-	The mapped cell ID reported from the RAN node on board to the AMF/MME is decoupled with RAN node ID, i.e. only corresponding to geographic area, so that the last know cell of a UE is irrelevant to RAN node.

Editor's note:	The format of the mapped cell ID reported from the RAN node on board to the AMF/MME is FFS.

#### 6.2.1.2	TNL Connection Suspending over N2/S1-AP interface

RAN node mobility, in case RAN nodes are boarded on satellite and the CN on the ground, will cause much frequent disconnection and re-connection of TNL over the N2/S1-AP interface of an AMF/MME. When the SCTP endpoint detects the path failure based on heartbeat time out, it will close the SCTP association and delete the transmission control block, which means that, when the satellite flies back to the PLMN A, it has to re-establish the SCTP association via four-way handshake. If all satellites need to re-establish the TNL toward the AMF, there will be lots for signaling towards the AMF on the ground.

The solution proposes to suspend the TNL between the RAN node and the CN if the RAN node attempts to disconnect the backhaul connection towards the AMF/MME in a PLMN. When the RAN node decides that it cannot serve the current PLMN any longer, it should indicate the AMF/MME to suspend the N2/S1-AP connection and store the context for resuming the N2/S1-AP connection in the future. Both the RAN node and the AMF/MME store the RAN node context information for resuming the N2/S1-AP connection. The RAN node and the 5GC should further adjust the heartbeat timer of the SCTP over N2/S1-AP interface to avoid heartbeat time out.

#### 6.2.1.3	Paging

RAN mobility will also cause the change of RAN nodes serving a TA(s), thus for paging an UE in a TA(s), the AMF needs to know which RAN node should page the UE over the air interface. As this solution doesn't require the RAN node onboard to continuously report the Supported TA list, the AMF has to rely on satellite ephemeris to calculate which RAN node can currently serve the UE. If the AMF/MME needs to page the UE, the AMF should select the RAN node to provide services to the UE based on the last visited cell provided by the RAN node and the ephemeris information of all on-board (ng-)eNB.

### 6.2.2	Procedures

#### 6.2.2.1	Connecting a RAN node on-board to the CN on the ground

Figure 6.2.2.1-1: Connecting a RAN node on-board to the CN on the ground

1.	If the on-board RAN node initiates the NG/S1 Setup procedure, the RAN node sends the NG SETUP REQUEST message as specified in clause 9.2.6.1 of TS 38.413 [12] to the AMF or the S1 SETUP REQUEST message as specified in clause 9.1.8.4 of TS 36.413 [14] to the MME.

2.	Upon receiving the NG/S1 SETUP REQUEST message, the AMF/MME support regenerative-based satellite access, the AMF should store the TAC(s) in the Supported TA List IE received in the NG/S1 SETUP REQUEST message and send the NG/S1 SETUP RESPONSE message to the AMF/MME.

#### 6.2.2.2	Information Exchanging between RAN node and CN

##### 6.2.2.2.1	RAN node Configuration for Supported TA list report

Figure 6.2.2.2.1-1: RAN node Configuration for Supported TA list report

Compared to the existing ANF/MME configuration update procedure, the differences are as follows:

1.	AMF/MME CONFIGURATION UPDATE message may include:

a)	Request Supported TA List indicator.

2.	AMF/MME CONFIGURATION ACKNOWLEDGE message may include:

a)	Supported TA List IE.

##### 6.2.2.2.2	Reporting Supported TA list to the CN on the ground

Figure 6.2.2.2.2-1: Reporting Supported TA list to the CN on the ground

1.	If the on-board RAN node will not be able to provide services to any TA in the TA list last provided to the AMF/MME, the on-board RAN node may initiate the RAN/ENB Configuration Update procedure to update the supported TA list by sending a RAN CONFIGURATION UPDATE message as specified in clause 9.2.6.4 of TS 38.413 [12] to the AMF or the ENB CONFIGURATION UPDATE message as specified in as specified in clause 9.2.6.4 of TS 38.413 [12] to the MME.

2	The AMF/MME shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE based on received Supported TA List IE in the RAN/ENB CONFIGURATION UPDATE message. The AMF/MME may take Tracking Area contained in the Supported TA List IE into account to optimise signalling towards this RAN node.

#### 6.2.2.3	TNL management over N2/S1-MME interface

Figure 6.2.2.2-2: Disconnecting and Reconnecting a RAN node with the CN on the ground

1.	If the on-board RAN node is about to disconnect the feeder link towards the AMF/MME in the PLMN, the on-board RAN node should initiate the RAN/ENB Configuration Update procedure to inform the AMF/MME to store the context of the NG/S1 interface and update the heartbeat timer of the SCTP over N2/S1-AP interface. For the on-board RAN node that is about to leave, the RAN/ENB CONFIGURATION UPDATE message includes the following differences:

a)	should include the suspend indicator;

b)	may include suspend cause; and

a)	may include the suspend time.

2	Upon receiving the RAN/ENB CONFIGURATION UPDATE message the AMF/MME:

a)	should store the context information related to the NG/S1 interface association and non-UE association, necessary to resume the connection;

b)	should determine the heartbeat timer of the SCTP over N2/S1-AP interface based on ephemeris information of all on-board (ng-)eNB configured by the O&M and transfer the heartbeat timer to the lower layer;

c)	should send the RAN CONFIGURATION UPDATE ACKNOWLEDGE message with the heartbeat timer to acknowledge that it successfully updated the configuration data.

3.	If the on-board RAN node is reconnecting the feeder link towards the AMF/MME in the PLMN, the on-board RAN node should initiate the RAN/ENB Configuration Update procedure to inform the AMF/MME to resume the connection by sending the RAN/ENB CONFIGURATION UPDATE message includes the following differences:

a)	should include the resume indicator;

b)	should include the Global RAN Node ID; and

c)	may include the resume time.

4.	If the AMF/MME can resume the connection, the AMF shall update the heartbeat timer and send the RAN CONFIGURATION UPDATE ACKNOWLEDGE message with the updated heartbeat timer to acknowledge that it successfully resumed the connection.

#### 6.2.2.4	Paging

If the network needs to signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane connection activation for PDU Session(s) to deliver mobile terminating user data) with a UE, the Network Triggered Service Request procedure is triggered by SMSF, PCF, LMF, GMLC, NEF, AMF or UDM, the SMF (and UPF, if applicable) as specified in clause 4.2.3.3 of TS 23.502 [3]. Figure 6.2.2.4-1 shows the Network Triggered Service Request procedure.

Figure 6.2.2.4-1: Network Triggered Service Request

Compared to the existing Network Triggered Service Request procedure, the differences are as follows:

Ephemeris information of all on-board (ng-)eNB is configured in the AMF by O&M.

Step4b: The AMF sends the paging message to the RAN node which can provide services to the UE based on the last visited cell provided by the RAN node and the ephemeris information of all on-board (ng-)eNB.

Upon expiry of timer T3513 as specified in clause 10.2 of TS 24.501 [15], the network should reinitiate paging in case of regenerative-based satellite access (see example in figure 6.2.2.3-2).

If the AMF/MME should send the paging message to all the RAN nodes currently serving the TA in the Registration Area, then the RAN nodes pages the UE.

### 6.2.3	Impacts on services, entities and interfaces

The solution impacts the following network functions:

-	MME/AMF supports to be configured with ephemeris information of all on-board (ng-)eNB by O&M;

-	MME/AMF and RAN support to suspend the TNL between the RAN node and the CN;

-	MME/AMF and RAN support to resume the TNL between the RAN node and the CN;

-	MME/AMF supports to request the supported TAC list in the MME/AMF Configuration Update procedure;

-	RAN supports reporting the supported TAC list in the MME/AMF CONFIGURATION UPDATE ACKNOWLEDGE message; and

-	RAN may support to be pre-configured (e.g. up to the operator's policy) the period for updating the Supported TA List.

## 6.3	Solution #3: Support for suspend and resume of NG interface

### 6.3.1	Key Issue mapping

This solution addresses the scenarios mentioned in the KI#1.

### 6.3.2	Functional Description

This solution follows following basic principles and assumptions:

Figure 6.3.2-1

1.	The gNB is in the payload of the satellite.

2.	When the feeder link becomes available, the gNB sends NG SETUP REQUEST message including feeder link available time i.e. gNB available time to the AMF.

3.	The AMF stores the feeder link available time for this gNB. The gNB and the AMF communicate the NGAP messages over the NG interface during the feeder link available time.

4.	When the gNB moves away at the location and the feeder link becomes not available, the gNB and the AMF suspends the NG connection i.e the gNB and the AMF do not communicate any NGAP message at this interface.

5.	When the feederlink availability time indicates that the feeder link is available both the gNB and the AMF resumes the NG connection implicitly.

The same principle is applied to the EPS at the S1 interface between eNB and MME.

Editor's note:	It is FFS how to manage the heart beat management procedure of SCTP protocol in steps 4 and 5.

Editor's note:	It is FFS how to perform the UE context management.

Editor's note:	The detailed step of resume procedure is FFS.

NOTE:	Adverse weather condition might impact the feeder link establishment procedure.

### 6.3.4	Impacts on existing services, entities and interfaces

gNB, NTN GW,and AMF needs to implement the suspend and resume \NG connection.

eNB, NTN GW and MME needs to implement the suspend and resume S1 connection.

## 6.4	Solution #4: Handling awareness of supported TAIs when RAN is onboard

### 6.4.1	Description

This is a candidate solution for Key Issue 1 - "Support of Regenerative-based satellite access ".

In contrast to Rel-17/Rel-18, where transparent payload was defined for satellite access and backhaul, the introduction of regenerative payload in Rel-19 involves deploying gNB on board, leading to impacts on the interface management between the RAN and the 5GC, especially considering paging. It is necessary to inform AMF that the RAN is orbiting away and the AMF then avoid sending any paging requests to this RAN.

This solution aims to repurpose the existing mechanism with minor enhancements, facilitating more efficient RAN context management for the AMF to avoid sending paging requests to a RAN that is far away. The solution addresses the following scenarios:

-	A RAN node moves away and stops serving a specific CN (AMF/MME) for a period before returning. The solution proposes an NGAP suspend/resume mechanism.

-	The RAN node is mobile while serving a CN and adjusts the TAIs it broadcasts, utilizing the same fixed TA management mechanism as in R17. The proposed solution suggests preconfiguring a "TAI timetable" for each RAN node in the AMF.

Figure 6. 4.1-1: Connection status between one satellite (RAN onboard) and AMF(s) via earth station(s) during the satellite is orbiting

Regarding the suspend/resume mechanism, when RAN orbits the Earth, as shown in Figure 6.4.1-1, it will have regular connection/disconnection with gateways (as shown in Figure 6.4.1-1). As gateways support IP connectivity with AMF(s), RAN may have regular connection/disconnection with AMF(s). There are 3 cases considering the connection relationship among gNB, gateway, and AMF.

-	Case I: gNB connects to the same AMF via the same gateway.

-	Case II: gNB connects to the same AMF via a different gateway (e.g. Annex A, Figure A-3).

-	Case III: gNB connects to a different AMF via a different gateway (e.g. Annex A, Figure A-4).

Such regularity can be used to reduce the same content exchange between RAN and AMF (RAN node level). Therefore:

-	An NG suspend/resume mechanism is needed to inform the AMF that the RAN is orbiting away, stop paging via such RAN, and resume the RAN context faster when the RAN is orbiting back.

Figure 6.4.1-2: Example of mapping relationship between RAN ID, supporting TAI list and valid period in case of one satellite

Editor's note:	How to page the UE via the pre-configured mapping relationship when ISL is available is FFS.

Regarding the preconfigured "TAI timetable" per RAN node, the current mechanism supposes that RAN updates the broadcast TAC in real-time according to the ephemeris and confirms the broadcast TAI list is associated with the geographical area covered by the satellite beam. When RAN updates the TAI list, RAN will inform AMF of the updates. Even though the interactions between RAN and AMF are very frequent, in the context of R17/E18, the cost of interactions is limited because the interactions are via ground link (e.g. fibre, cable, etc.). However, in R19, such interactions will be over feeder link, whose bandwidth is scarce. Besides, the ephemeris data can be updated also very frequently in R17/R18. Therefore, a mechanism is needed to reduce the frequent interactions by:

-	Quantifying the broadcasting TAI list refreshing cycle to reduce the frequency of interactions. As shown in Figure 6.4.1-1, e.g. at T1, RAN broadcasts TAI 1-12, at T1+δ, RAN broadcasts TAI 3-13, at T2, RAN broadcasts TAI 6-18 and etc. RAN only updates the TAI list at the quantified time point.

-	Integrating valid period in the mapping relationship between RAN ID and supporting TAI list. Such mapping relationship can also be stored at AMF by pre-configuration/RAN provision, as RAN orbiting has a predictable trajectory, such mapping relationship information can be reused/valid regularly.

By preconfiguring such per RAN node TAI timetable, AMF can also stop interactions with a RAN node that is not valid.

In conclusion, this solution proposes efficient ways to enhance paging when RAN is onboard and offers a more efficient context management approach at AMF by capturing the regularity of connections and regularity of context validations.

### 6.4.2	Procedures

#### 6.4.2.1	Procedures of gNB connect to the same AMF via the same gateway

Figure 6.4.2-1: Procedures of efficient RAN context management on AMF

0.	Pre-condition: AMF and RAN both are pre-configured with the mapping relationship among Global RAN Node ID, supporting TAI list, and valid period.

1.	When the feeder link with gateway-1 is available, the NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data (e.g. updates of the mapping relationship if it is different from the pre-configuration) to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data, as defined in clause 8.7 of TS 38.413 [12]. The AMF will update the mapping relationship and the starting time of the valid period if needed.

2.	NG-RAN detects the connection to AMF is about unavailable due to moving, e.g. due to feeder link unavailability.

3.	The NG-RAN sends an NG SUSPEND REQUEST message to AMF to inform AMF the connection will be lost before the connection is lost. A suspend period can be included to inform AMF how long the RAN can orbit back. The suspend period can be derived based on ephemeris data.

The NG-RAN may keep the NG-related context as described in clause 9.2.6 of TS 38.413 [12] unreleased.

4.	AMF starts to suspend the NG-RAN context (i.e. keeps the NG-related context as described in clause 9.2.6 of TS 38.413 [12] unreleased when detects the connection is not available) based on e.g. the informed suspending period as received in step 3, pre-configured information etc, to reduce the interaction of the same context in the next cycle.

5.	AMF will stop sending paging requests to this NG-RAN until RAN resumes connection with AMF. The AMF may update this NG-RAN's mapping relationship, especially the valid period based on the suspend period.AMF sends a response to the NG-RAN.

6.	NG-RAN may keep the NG-related context as described in clause 9.2.6 of TS 38.413 [12] unreleased if it has enough storage.

7.	NG-RAN detects the connection to AMF is to be available.

8.	NG-RAN sends NG RESUME REQUEST to the AMF.

9.	AMF active the RAN context.

10.	AMF sends a corresponding response to RAN. If RAN does not store the AMF context, AMF will send the corresponding context to RAN based on an indication if received in step 8.

NOTE:	whether the suspend/resume interaction between RAN and AMF is via the current defined message or a new message depends on the RANs work group.

Editor's note:	How does the SCTP association is handled when NG connection is suspended is FFS.

#### 6.4.2.2	Procedures of gNB connect to a different AMF via a different gateway

The procedure as defined in clause 6.4.2.1 can be reused.

#### 6.4.2.3	Procedures of gNB connect to the same AMF via different gateway

Whenever gNB connects to a different gateway, if the gNB changes its Global RAN Node ID. procedures as defined in clause 6.4.2.2 can be reused, as shown in Figure 6.4.1-2.

Editor's note:	if gNB does not change Global RAN Node ID, the procedure is FFS.

### 6.4.3	Impacts to Services, Entities and Interfaces

AMF:

-	Preconfigure mapping relationship among Global RAN Node ID, supporting TAI list and valid period.

-	Update mapping relationship, especially the valid period starting time-based on NG Setup Request.

-	Suspend RAN context when the connection between RAN and AMF is interrupted.

-	Active RAN context when the connection between RAN and AMF is recovered.

RAN:

-	Preconfigure mapping relationship among Global RAN Node ID, supporting TAI list and valid period.

-	Send updates of mapping relationship to AMF to minimize the volume of signalling.

Change Global RAN Node ID when the connecting gateway changes.

## 6.5	Solution #5: Support of Feeder Link Switchover with TAU for Non-Pooled MMEs

### 6.5.1	Description

This is a candidate solution to address Key Issue #1. The proposed solution follows the following assumptions and principles.

-	One eNB on a satellite for regenerative based satellite access for LEO/MEO deployment.

-	Single satellite deployment.

-	Feeder link switchover causes eNB to be served by a different MME than source MME.

-	The involved MMEs are not pooled, or not within the same pool.

-	The NTN Gateway might not have connectivity to all MMEs.

Editor's note:	The limited connectivity between NTN gateways and MMEs may not represent all network architectures. It addresses scenarios where certain MMEs to be reachable through specific NTN Gateways due to reasons such as geographical distribution, network load balancing and traffic management, or disaster recovery and redundancy.

Regenerative payload, when there is an eNB embedded on the satellite, results in the termination of the S1-AP interface at the satellite. This configuration encounters challenge due to eNB moving, which might lead to the feeder link switchover from a source NTN GW to a target NTN GW when a LEO/MEO satellite exits the coverage area of the source NTN GW. While the feeder link switchover aspects are TNL related hence are out of scope of SA2, in such instances, the eNB could be served, before and after the feeder link switchover by different MMEs, as shown in Fig 6.5.1-1.

Figure 6.5.1-1: Feeder Link Switchover causes MME change

As the UE remains in the same cell, handover is not triggered in this case. But the UE is now served by a different MME due to the feeder link switchover. In the case that these involved MMEs are not pooled or not within the same pool, to update the new MME with UE's context, when the new feeder link becomes operational, the UE should initiate a TAU procedure.

To trigger the UEs to initiate the TAU procedure, there could be various methods to notify UE when the new feeder link becomes operational. In principle, the eNB may continue broadcasting system information while the switch is ongoing. If the embedded eNB is aware of the feeder link switchover, it might broadcast e.g. "barred cell due to feeder link switchover" message across the cell, to prevent UEs from accessing the cell before new feeder link becomes operational.

Editor's note:	Whether and how eNB broadcasts "feeder link switchover" information is FFS and may need coordination with RAN WG2.

Alternatively, before the feeder link switchover, if any ground-based entity, such as the source NTN Gateway or MME, predicts an imminent feeder link switchover based on the satellite's control data, e.g. satellite ephemeris information, MME might notify UE via a proper NAS message the feeder link switchover information including the type of switchover and timing of the feeder link switchover.

Editor's note:	What is the appropriate NAS message to inform the UE is FFS.

During feeder link switchover, the TNL node on the satellite will be triggered to configure the transport association for the target NTN GW feeder link. The details of that are out of scope of SA2.

To avoid large number of UEs causing excessive signalling load on the network, Overload control mechanism, described in clause 4.13.8.6 of TS 23.401 [5] might be used.

### 6.5.2	Procedures

Figure 6.5.2-1 illustrates the call flow and system changes for feeder link switchover-triggered Tracking Area Update (TAU) procedure.

Figure 6.5.2-1: High-level procedure for TAU triggered by Feeder Link Switchover

1.	As described in clause 6.5.1, the feeder link switchover triggers UE to start TAU procedure, after being notified by eNB that new feeder link TNL association has become operational.

2.	The UE initiates the TAU procedure by sending, to the eNB, a TAU Request with cause, e.g. feeder link switchover, in addition to regular parameters described in clause 5.3.3.1 of TS 23.401 [5].

3.	The eNB derives the MME address from the RRC parameters as per existing procedures in TS 23.401 [5]. If that MME is not associated with that eNB, the eNB selects an MME as described in clause 4.3.8.3 of TS 23.401 [5].

4.	In steps 4-19, regular TAU procedures will be followed, as specified in clause 5.3.3.1 for TAU with SGW change and in clause 5.3.3.2 for TAU without SGW change in TS 23.401 [5].

Following the steps 4-19, the MME sends a TAU Accept in step 20. And in step 21, UE might acknowledge the received TAU Accept message by returning a TAU Complete message to the MME.

After the TAU procedures, for scenarios involving a hard feeder link switchover, where the onboard eNB can connect to only one NTN Gateway at a time, and when the MMEs associated with the source and target NTN Gateways are different, the S1 release procedure may be performed locally by the eNB and by the MME, with cause, e.g. feeder link switchover, without using any of the signalling directly between eNB and MME.

In the case of soft feeder link switchover where the onboard eNB can connect to multiple NTN Gateways simultaneously during a given period, and when the MME associated with the source and target NTN Gateways are different, the process for releasing S1 connections may be initiated by either the eNB or the MME associated with the source NTN Gateway, with cause, e.g. feeder link switchover.

NOTE:	The details of feeder link switchover as TNL related and therefore out of scope of SA WG2.

### 6.5.3	Impacts to Services, Entities and Interfaces

This solution impacts the following entities.

RAN:

-	Release S1-AP based on feeder link switchover information.

-	Broadcast feeder link switchover information

MME:

-	Release S1-AP based on feeder link switchover information.

-	New triggering condition, feeder link switchover, for TAU

-	New IE in NAS message to include feeder link switchover information.

UE:

-	Initiate the Tracking Area Update procedure when being informed of the occurrence of feeder link switchover.

## 6.6	Solution #6: Load re-balancing for feeder link switchover

### 6.6.1	Description

Key Issue #1 focus on the deployment of regenerative payload on LEO/MEO satellite access, and whether there is any impact on 5GS and EPS. In the regenerative payload configuration, the feeder link switchover, due to eNB/gNB moving, might require a reconfiguration of connections, leading to increased NAS signalling overhead as network transitions between different NTN GWs and potentially between MMEs/AMFs.

As a candidate solution to address Key Issue #1, this solution leverages load-rebalancing procedures to handle feeder link switchover when the involved MMEs are in the same pool, or when the involved AMFs are within/without the same AMF set.

Editor's note:	Whether and how Load Rebalancing TAU procedure can be used for separate MME pools is FFS.

### 6.6.2	Procedures

#### 6.6.2.1	MME procedures

The deployment of MMEs is usually based on factors such as geography, latency, network load, and specific service requirements. During feeder link switchover, as a LEO/MEO satellite moves beyond the coverage of a source NTN GW, the eNB could be served, before and after the feeder link switchover, by different MMEs. Even if MMEs are accessible from an NTN GW, a switch to an MME that offers lower latency, aligned with service requirements, might be preferable.

The MME Load Re-balancing functionality, as specified in clause 4.3.7.3 of TS 23.401 [5], allows UEs that are registered on an MME (within an MME Pool Area) to be moved to another MME. Fig. 6.6.2.1-1 provides a high-level procedure to illustrate how MME load re-balancing supports the feeder link switchover scenario.

Figure 6.6.2.1-1: High-level procedure for load re-balancing driven feeder link switchover in EPS

0.	Based on satellite control data, e.g. ephemeris information, the eNB and Source MME may have their Load Balancing parameters adjusted beforehand based on the Feeder Link Switchover information, e.g. the Weight Factor could be set to zero for the source MME.

1.	To off-load connected mode UEs, the source MME initiates the S1 Release procedure, as specified in clause 4.3.7.3 of TS 23.401 [5], but with a new release cause, e.g. "feeder link switchover".

2.	If the RRC connection is not already released, the eNB sends a RRC Connection Release message to the UE in Ack Mode. Once the message is acknowledged by the UE, the eNB deletes the UE's context.

3.	The eNB confirms the S1 Release by returning an S1 UE Context Release Complete (ECGI, TAI, Secondary RAT usage data) message to the MME. With this, the signalling connection between the MME and the eNB for that UE is released.

4.	In case there is no MME pool or the source MME and target MME are not in the same MME pool, UE initiates a Load Rebalancing-triggered TAU but provides neither the S-TMSI nor the GUMMEI to eNB in the RRC establishment.

Editor's note:	Whether is necessary to provide a new TAU cause rather than Load Rebalancing in this case is FFS.

5.	When the UE provides neither the S-TMSI nor the GUMMEI in the RRC establishment, the eNB should select an MME based on the Weight Factors of the MMEs.

#### 6.6.2.2	AMF procedures

Figure 6.6.2.2-1 provides procedures to support feeder link switchover in 5GS.

Figure 6.6.2.2-1: High-level procedures for load re-balancing driven feeder link switchover in 5GS

0.	Feeder link switchover preparation: In 5GS, 5G-AN node shall have the capability to support multiple TNL associations per AMF. Based on satellite control data, e.g. ephemeris information, the source AMF shall provide the gNB node with the weight factors for each TNL association of the AMF. The gNB node, when selecting a TNL association, will consider Weight factors of candidate TNL associations.

Triggered by the feeder link switchover, or before the disruption of the feeder link, the gNB node and AMFs have exchanged configuration data. During these interactions, the AMF has supplied the gNB with information about the set of TNL association to be established, weight factor associated with each of the TNL association within the AMF, and weight factor for each AMF Name within the AMF set.

When the same AMF is used before and after the switchover, because multiple TNL endpoints can be configured per gNB node/AMF pair, the AMF can dynamically add or remove TNL associations, by requesting to remove the TNL association on the source NTN GW and add new TNL association at the target NTN GW, or just keeping the same association while changing local and remote IP address in this association. The gNB will also assign re-balancing weight factor for the TNLA on the source and target NTN GW.

1.	If the source AMF and the target AMF are different, but within the same AMF set. The AMF load re-balancing functionality could be employed here. The AMF load re-balancing functionality permits cross-clause of its subscribers that are registered on an AMF (within an AMF Set) to be moved to another AMF within the same AMF set with minimal impacts on the network and end users.

AMF may request some or all the 5G-AN node(s) to redirect a cross-clause of UE(s) returning from CM-IDLE state to be redirected to another AMF within the same AMF set, if the 5G-AN is configured to support this.

For UE(s) in CONNECTED mode, similar mechanisms for AMF Management can be used to move the UE to another AMF in the same AMF set as described in clause 5.21.2, except that the old AMF deregisters itself from NRF.

2.	If the source AMF or the target AMF does not belong to any AMF set, or they are not in the same AMF set, the AN Release procedure, as described in clause 4.2.6 of TS 23.502, could be applied.

AMF sends an N2 UE Context Release Command with cause, e.g. "feeder link switchover" to the gNB.

3.	Follow Steps 3-7, as described in clause 4.2.6 of TS 23.502 [3]. A proper cause, e.g. feeder link switchover, should be indicated when this procedure is used for the purpose of feeder link switchover.

4.	Registration Request with Load Rebalancing purpose: UE initiates a Feeder Link Switchover-triggered Registration Request but provides neither the 5G-S-TMSI nor the GUAMI to gNB in the RRC establishment.

Editor's note:	Whether is necessary to provide a new Registration Request cause rather than Load Rebalancing in this case is FFS.

5.	When the UE provides neither the 5G-S-TMSI nor the GUAMI in the RRC establishment, the gNB should select an AMF. The gNB node, when selecting a TNL association, will consider weight factor associated with each of the TNL association within the AMF, and weight factor for each AMF Name within the AMF set.

NOTE:	NAS level congestion control is achieved by providing the UE a back-off time. But it should be noted that when indicating a specific target AMF, the AMF should ensure that the load re-balancing will not cause overload in the target AMF.

### 6.6.3	Impacts to Services, Entities and Interfaces

This solution impacts the following entities.

RAN:

-	Release NG-AP or S1-AP taking into consideration of feeder link switchover information.

MME:

-	Release S1-AP taking into consideration of feeder link switchover information.

AMF:

-	Release NG-AP taking into consideration of feeder link switchover information.

## 6.7	Solution #7 for Key Issue #1: Feeder link switchover with DL data continuity

### 6.7.1	Description

It is observed that the feeder link switchover for Regenerative-based satellite access is different from Transparent-based satellite access. Also there is a potential issue of DL data continuity in case of hard feeder link switchover. In this solution a procedure for Feeder link switchover for Regenerative-based satellite access is proposed. During hard feeder link switchover, the Core Network should temporarily buffer the DL data.

The mechanism of this solution can also apply to Regenerative-based satellite access for IoT/EPS accordingly, then the detailed procedure is not described in this clause.

### 6.7.2	Procedures

#### 6.7.2.1	Feeder link switchover without AMF change

Figure 6.7.2-1 describes the procedures for Feeder link switchover without AMF change.

Figure 6.7.2.1-1: Feeder link switchover without AMF change

1.	The NG-RAN onboard the satellite connects to 5GC via NTN Gateway 1.

For the upcoming Feeder link switchover, the NG-RAN may perform switchover preparation depending on a solution out of scope of SA2, e.g. if it is assumed that two "logical" gNBs are onboard satellite to assist the Feeder link switchover.

In the case of hard feeder link switchover, the NG-RAN informs 5GC about the upcoming hard feeder link switchover. The NG-RAN can inform AMF the upcoming hard feeder link switchover per UE session.

Editor's note:	Whether it is scalable to inform the AMF about the upcoming hard feeder link switchover using UE associated signalling or is better to use "per node" signalling is FFS.

2.	In the case of hard feeder link switchover, the AMF informs the corresponding SMF(s) about the upcoming hard feeder link switchover receive from NG-RAN. The AMF can inform SMF the upcoming hard feeder link switchover in Nsmf_PDUSession_UpdateSMContext Request.

Editor's note:	It is FFS whether it is feasible to contact all SMFs during the glitch between the two feeder links connections and whether the solution is scalable.

3.	Based on hard feeder link switchover information, the SMF controls the UPF to buffer the DL data of the corresponding PDU session.

4.	The SMF responds to AMF.

5.	The NG-RAN onboard the satellite performs Feeder link switchover to NTN Gateway 2. After the Feeder link is switched, the NG-RAN is assumed to get a new Transport Network Layer address. Details of this procedure are out of scope of 3GPP.

6.	The NG-RAN connects to AMF via NTN Gateway 2 by using the new Transport Network Layer address.

NOTE:	Depending on the RAN solution, the NG-RAN onboard the satellite may just update the TNLA with AMF with the new Transport Network Layer address; or a new NG-RAN onboard the satellite may connects to AMF with the new Transport Network Layer address if it assumes two gNBs onboard satellite to assist the Feeder link switchover, along with the disconnection or suspension between the old NG-RAN and AMF. The details of the procedure are out of scope of SA WG2.

7.	The NG-RAN notifies the AMF about the N3 DL TNL address corresponding to new Transport Network Layer address. The NG-RAN can use N2 Path Switch Request based on mechanism of Xn handover, or PDU Session Resource Modify Indication.

8.	The AMF informs the SMF(s) about the new N3 TNL address for DL data for the corresponding PDU session in Nsmf_PDUSession_UpdateSMContext Request.

9.	The SMF controls the UPF(s) to send DL data to the new N3 TNL address of NG-RAN.

If there is buffered DL data in UPF the SMF controls the UPF(s) to send buffered data and received data in sequence.

The SMF responds to AMF with Nsmf_PDUSession_UpdateSMContext Response. The SMF may includes new UL TNL information for UL data.

10.	The AMF responds to NG-RAN by N2 Path Switch ACK or PDU Session Resource ModifyConfirm.

Editor's note:	It is FFS on how to support Feeder link switchover with AMF change.

### 6.7.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

AMF:

-	Reception of new TNLA from old NG-RAN or new NG-RAN due to Feeder link switchover. Possible normative impact in SA2 depends on RAN WGs.

SMF/UPF:

-	Buffering of the DL data.

NG-RAN

-	Connecting to AMF with new TNLA.

-	Provision to AMF the N3 DL TNLA corresponding to new Transport Network Layer address.

## 6.8	Solution #8 for Key Issue #1: QoS/ Policy handling for Regenerative-based satellite access

### 6.8.1	Description

It is observed that the QoS, RAT Type for Regenerative-based satellite access and Transparent-based satellite access is different. The differences can be illustrated by the figure below:

Figure 6.8.1-1: QoS for Regenerative-based and Transparent-based satellite access

Additionally, considering the potential inclusion of ISL as part of the backhaul, the concept of dynamic satellite backhaul is also extended to cover regenerative payload configurations for CN PDB.

In this solution a new QoS/PCC handing mechanism for Regenerative-based satellite access for NR NTN is proposed, including 5QI for Regenerative-based satellite access, PCF decision of PCC policy, CN PDB determination.

The mechanism of this solution can also apply to Regenerative-based satellite access for IoT/EPS accordingly, then the detailed procedure is not described in this clause.

### 6.8.2	Procedures

#### 6.8.2.1	General

The general procedure is based on PDU session Establishment or Modification procedures described in clauses 4.3.2 or 4.3.3 TS 23.502 [3], with additional treatment for Regenerative-based satellite access.

Figure 6.8.2-1 describes the general procedures of the solution.

Figure 6.8.2.1-1: General procedures for QoS/ Policy handling for Regenerative payload

1.	UE requests PDU Session Establishment/Modification via Regenerative-based satellite access for NR NTN.

2.	Based on the information provided by NG-RAN onboarding the satellite, the AMF determines the RAT Type information for Regenerative-based satellite access of the UE, which is distinguished from currently defined satellite access RAT type, e.g. "NR(LEO)", "NR(MEO)", "NR(GEO)" and "NR(OTHERSAT)". Then the AMF includes this RAT Type information in the message to SMF, e.g. PDU Session Create/Update SM context Request. Examples of the RAT information for Regenerative-based satellite access, to represent the specific RAT type are proposed below:

-	REGENERATIVE_MEO;

-	REGENERATIVE_LEO;

The AMF determines the satellite backhaul category for regenerative payload and provides it to SMF in the Nsmf_PDUSession_CreateSMContext message to determine e.g. the CN PDB or dynamic CN PDB.

Editor's note:	Whether legacy defined 8 satellite backhaul categories can be reused in the regenerative payload is FFS.

3.	The SMF may respond to AMF as described in clauses 4.3.2 or 4.3.3 of 4.3.3 of TS 23.502 [3].

4.	The SMF decides to perform SM Policy Association Establishment/Modify procedures as defined in clauses 4.3.2 or 4.3.3 of TS 23.502 [3], including the RAT Type information for Regenerative-based satellite access in the request to PCF.

5.	The PCF takes RAT Type information for Regenerative-based satellite access into account for the policy decision, e.g. together with any delay requirements provided by the AF. The PCF can decide the corresponding 5QI for Regenerative-based satellite access. Then PCF provides corresponding policy information to SMF.

For the UE Policy Association Establishment, the UE Policy Association Establishment procedure in clause 4.16.11 of TS 23.502 [3] can be reused as baseline for the regenerative payload with the following adaptations:

-	In step 2, in Npcf_UEPolicyControl Create Request, the RAT information for Regenerative-based satellite access is sent to PCF for e.g. the URSP determination.

6.	The SMF sends N1N2Message to AMF, including the QoS information for Regenerative-based satellite access.

The SMF can decide the CN PDB for Regenerative-based satellite access based on the specific types of satellite access, e.g. GEO, LEO, and include it in QoS information for Regenerative-based satellite access. This may help the NG-RAN to obtain a more accurate AN PDB and CN PDB.

For Regenerative-based satellite access, the CN PDB is the delay on the ground, plus the delay on air which is one way propagation delay of the specific types of satellite access.

7.	The AMF initiates N2 PDU Session Request/Modify accordingly, including the QoS information for Regenerative-based satellite access in N2 SM information sent to NG-RAN.

8.	The NG-RAN onboarding the satellite performs AN resource setup according to the QoS parameter provided by SMF, e.g. 5QI for Regenerative-based satellite access.

The NG-RAN onboarding the satellite can determine the AN PDB and CN PDB for specific types of satellite access, e.g. GEO, LEO, based on configuration or QoS profile received from SMF at step 6.

For Regenerative-based satellite access, the AN PDB is one way propagation delay, which is different from two way propagation delay applicable for the Transparent-based satellite access.

### 6.8.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

AMF:

-	Determination of the RAT Type information for Regenerative-based satellite access of the UE.

-	Provision to other NFs the RAT Type information for Regenerative-based satellite access.

SMF:

-	Provision to other NFs the RAT Type information for Regenerative-based satellite access.

-	Determination of the CN PDB for Regenerative-based satellite access based on the specific types of satellite access.

PCF:

-	Policy and QoS decision for Regenerative-based satellite access.

NG-RAN

-	Provision to AMF the RAT type information for Regenerative-based satellite access.

-	QoS handling & AN resource setup according to the QoS parameter provided by SMF, e.g. 5QI for Regenerative-based satellite access.

-	Determination of the AN PDB and CN PDB for specific types of satellite access.

## 6.9	Solution #9:	Support of Regenerative-based satellite access with deploying RAN-agent and Proxy RAN node or Link Layer Proxy NF

### 6.9.1	Description

This solution resolves KI#1 about any impact of RAN nodes changing for any given 5GC/EPC and for a given area in the case of RAN nodes on-board moving.

Embarking gNB on-board LEO/MEO implies the on-board gNB serving for the UE access will be change as the satellite moving, which may cause the following network impacts assuming the UE is not moving:

-	N2/N3 connection frequent change (disconnection of N2/N3 between old serving gNB and AMF/UPF, reconnection of N2/N3 between new serving gNB and AMF/UPF) will cause lots of signalling on N2 interface.

-	Frequent change of the gNB serving for the TA the UE located which may cause AMF unaware of which gNB should paging the UE.

#### 6.9.1.1	Support of Regenerative-based satellite access with RAN-agent and Proxy RAN node

The solution proposes to enhance 5G network architecture to reduce the impact of 5GC. Solution#8 in TR 23.737-h20 is proposed to reuse to address the above mentioned issues.

In this solution:

1.	Deploying RAN agent close to gateways on earth as shown in Figure 6.9.1.1-1. The RAN agent is used to process control plane signalling, i.e. handle N2 signalling and forward the N2 messages between gNB and AMF.

2.	Deploying Proxy RAN nodes which is collocated with RAN agent. The Proxy RAN node is used to process user plane data, i.e. terminate the N3 connection with UPF and establish N3' connection with serving gNB.

NOTE 1:	This is to study the architecture enhancement. The feasibility of the solution with the new defined NFs and relevant interfaces are in the scope of RAN and needs to coordinate with RAN WGs.

Figure 6.9.1.1-1: Support of LEO/MEO satellite based gNB using RAN agent and Proxy RAN node

NOTE 2:	A similar mechanism can also be applied for EPS where the gNB uses eNB instead.

The solution assumes that Earth-fixed Tracking Areas are used. The cells can be Earth-moving or Earth-fixed cells.

The solution assumes that in a given geographical area the "setting" satellite passes the control on per-cell basis to the "rising" satellite.

The functions of RAN agent include:

-	Establishing TNL Associations with the gNB, and establishing TNL Associations with the AMF;

-	Terminating the NG AP message(s) from on-board gNB for NG Setup or RAN Configuration Update;

-	Sending a NG AP message to the AMF to establish or update NG connection by including a proxy RAN node ID and/or proxy RAN node name which are statically configured with same format as RAN node ID and RAN node name;

-	Storing the mapping between TA(s) that each connected on-board gNB serves and the gNB ID;

-	Storing the mapping between the serving gNB ID and the proxy RAN node ID. Updating the mapping information if the serving gNB is changed.

-	Provide N3 and N3' tunnel information through Nx interface to indicate proxy RAN node to establish N3 and N3' connection

-	Receiving Paging message from AMF, and forwarding the Paging message to the gNB(s) which is serving the Registration Area allocated to the UE;

-	Allocating a RAN UE NGAP proxy ID which has same format as RAN UE NGAP ID allocated by the gNB. The RAN UE NGAP proxy ID doesn't change as long as the UE is within the serving area of the RAN agent;

-	Storing 1:1 mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID;

-	Updating the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID if RAN UE NGAP ID allocated by the gNB is changed;

-	Replacing the RAN UE NGAP ID included in any UE associated NG AP message from the gNB with the corresponding RAN UE NGAP proxy ID;

-	Replacing the RAN UE NGAP proxy ID included in any UE associated NG AP message from the AMF with the corresponding RAN UE NGAP ID.

NOTE 3:	Security related functions need to be addressed by SA WG3 during the normalisation phase.

Existing Registration management and connection management can be reused in the 5G network with support of gNBs on NGSO satellites with following changes:

-	The RAN agent needs to identify which on-board gNB(s) is serving the Registration Area allocated to a UE before paging the UE.

The functions of Proxy RAN node include:

-	Receiving N3 and N3' tunnel information to establish N3 and N3' connection.

-	Receiving data traffic from serving gNB using N3' tunnel and sending the data traffic to the UPF using N3 tunnel.

-	Receiving data traffic from UPF using N3 tunnel and sending the data traffic to the serving gNB using N3' tunnel.

#### 6.9.1.2	Support of Regenerative-based satellite access with Link Layer Proxy

RAN agent and Proxy RAN node in clause 6.9.1.1 can also be integrated as a core network function called Link Layer Proxy, the LLP is decoupled from NTN gateways and has a 1:1 mapping relationship to MME/AMF set.

The control plane and user plane in the LLP can be separated based on internal private interface. Figure 6.9.1.2-1 and Figure 6.9.1.2-2 presents the EPS and 5GS architecture for regenerative-based satellite access with Link Layer Proxy (LLP).

Figure 6.9.1.2-1: EPS architecture for regenerative-based satellite access with Link Layer Proxy (LLP)

Figure 6.9.1.2-2: 5GS architecture for regenerative-based satellite access with Link Layer Proxy (LLP)

### 6.9.2	Procedures

#### 6.9.2.1	NG Setup, RAN configuration Update and UE registration

The following call flow should be applied to RAN agent or LLP, the call flow uses RAN agent in 5GS as an example.

Figure 6.9.2: Call Flow for RAN agent

1.	The address of an RAN agent is configured to the earth station, so that the NG setup messages sent by satellite RAN nodes can be sent to the RAN agent.

2.	The RAN agent stores the TA list supported by the satellite RAN node, and replaces the RAN node ID with a Proxy RAN node ID which is allocated by the RAN agent. The RAN agent stores the mapping between the RAN node ID and the Proxy RAN node ID. TheRAN agent may provide its serving area (TA list) to the AMF.

3-4.The AMF responds the request, the RAN agent replaces the Proxy RAN node ID with the RAN node ID, and forwards the response message to the NG RAN node.

5.	If satellite RAN node changes the supported TA list due to mobility, then the satellite RAN node sends RAN Configuration Update message including the updated TA list the RAN node supports. The message is not required to be sent to the AMF.

6.	The RAN agent sends an acknowledge message.

7.	When a UE registers via satellite RAN. The NAS transport message carrying Registration Request will be sent to the RAN agent first.

8.	The RAN agent needs to allocate a RAN UE NGAP proxy ID which is similar as RAN UE NGAP ID, and replaces the RAN UE NGAP ID in the NAS transport message with the allocated RAN UE NGAP proxy ID (so that the downlink message can be routed to the AMF agent). The RAN agent stores the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID.

9-10.	The AMF responds the registration request, the RAN agent replaces the RAN UE NGAP proxy ID with the RAN UE NGAP ID, and forwards the response message.

#### 6.9.2.2	Handover with LLP and MME relocation

This procedure presents how LLP relocation happens when MME relocation is needed, e.g. as presented in Figure A-4 of Annex A, MME relocation may happen with UE handover between satellites using different earth station location.

The following call flow can be applied to RAN agent and Proxy RAN node or LLP, the call flow uses LLP in EPS as an example.

NOTE 1:	The handover procedure is not applicable to NB IoT UE in NTN.

Figure 6.9.2.2-1: S1 Handover with LLP and MME relocation

As well as LLP mapping the eNB ID with LLP ID and mapping eNB UE S1AP ID with LLP S1AP ID in UE level S1 message, the differences with current S1 Handover with MME relocation procedure are listed as follows:

-	Steps 1-5: After the LLP receives the Handover Required message from Target eNB in step 2, the LPP determines that the Target eNB is not under this LLP's management, it does not replace the Target eNB ID with LLP ID, and sends the message to Source MME in step 3.

Based on the target TAI and target eNB ID (which is not the LLP ID), the source MME can deduce it is S1 Handover with regenerative satellite access using the LLP and both MME and LLP relocation is needed, step 4a and step 5 is same with current MME relocation procedure.

-	Steps 6a-6c: Based on the target TAI, the Target MME send the Handover Request to the Target LLP in step 6a. After the Target LLP receives the Handover Request, based on EPS bearers to setup, the target LLP allocates UL LLP tunnels resources and replace the S-GW tunnel information with the allocated the LLP tunnel information. The target LLP then sends the Handover Request to the Target eNB.

-	Steps 7a-7c: The target eNB sends the Handover Request Ack to the Target LLP in step 7a, based on the EPS Bearers to setup in the message, the LLP allocates DL LLP tunnel resources and replaces RAN tunnel information with the DL LLP tunnel information. The Target LLP then sends the Handover Request Ack message to the Target MME.

-	Steps 8-14: Same with current S1 Handover with MME relocation procedure with the difference that LLP transfers UE level S1 messages between RAN and MME and performs eNB ID and eNB UE SA1AP ID mapping.

### 6.9.3	Impacts to Services, Entities and Interfaces

There is no impact to the existing NFs or Entities (e.g. gNB, AMF, SMF). However, the proposed solution needs to deploy two new NFs if it is RAN agent and Proxy RAN node to deploy, as the following:

RAN agent:

-	New NF, which is located near the satellite ground station. The functionalities can be referred to clause 6.9.1.

Proxy RAN node:

-	New NF, which is co-located with RAN agent. The functionalities can be referred to clause 6.9.1.

Nx interface:

-	New interface, which is used to provide N3 and N3' tunnel information to the proxy RAN node by RAN agent.

N2'/N3' interface:

-	Can reuse the functionality of N2/N3 interface.

If it is the LLP to deploy, the impacts are listed as following:

Link Layer Proxy:

-	New NF, located on the ground with 1:1 mapping relationship with MME/AMF set, decoupled from the NTN gateway locations. Functionality of the LLP is described in the solution.

N2'/N3' or S1-MME'/S1-U' interface:

-	Can reuse the functionality of N2/N3 and S1-MME'/S1-U' interface.

## 6.10	Solution #10: Regenerative-based satellite access with IWF on S1 or N2/N3 interface

### 6.10.1	Key Issue mapping

This solution addresses Key Issue #1 (Support of Regenerative-based satellite access).

### 6.10.2	Description

Depicted in Figure 6.10.2-1 is the 5GS architecture for regenerative-based satellite access with N2/N3 interworking function (IWF).

Figure 6.10.2-1: 5GS architecture for regenerative-based satellite access with N2/N3 IWF

The following are the salient features of the architecture in Figure 6.10.2-1:

-	Earth-fixed Tracking Areas and Earth-fixed cells are used.

-	gNB is on board of the satellite.

-	Each LEO satellite serves a pre-defined geographic area on the Earth for a predetermined period of time (referred to as the service time).

-	At the beginning of the service time the rising satellite establishes a connection with the Interworking Function (IWF) on the ground via the Nx reference point.

-	The IWF is perceived by 5GC as a static gNB.

-	The Nx reference point has a control-plane and a user-plane functionality.

Editor's note:	The feasibility of the solution needs to be confirmed by relevant RAN WGs.

Editor's note:	It is FFS whether Nx will be standardised by 3GPP. If it is, the standardisation falls within the remit of RAN WGs.

-	During the service time, the gNB on the satellite maps the radio component of the cell to the corresponding geographical area of the Earth-fixed cell. This is achieved using mechanical and/or electronic steering of the antenna beams on the satellite.

-	As the satellite approaches the end of service time, the setting satellite transfers the control over the Earth-fixed TAs and Earth-fixed cells to the rising satellite. The transfer of the control from the setting satellite to the rising satellite is on per-cell basis and is achieved using the functionality of the Nx reference point and/or the Inter-Satellite Link (ISL).

-	The transfer of the control from one satellite to another is transparent to the Core Network.

NOTE:	Further details on the transfer of control from one satellite to another fall under the remit of RAN WGs.

-	There can be multiple IWFs in a satellite network. UE context transfer between IWFs follows existing handover Xn/N2 procedures as far as the CN is concerned.

Editor's note:	How to support scenarios 3 and 4 mentioned in Annex A of 23.700-29-020 is FFS.

Depicted in Figure 6.10.2-2 is the corresponding EPS architecture for regenerative-based satellite access with S1 interworking function (IWF).

Figure 6.10.2-2: EPS architecture for regenerative-based satellite access with S1 IWF

The description of salient features for the 5GS architecture in Figure 6.10.2-1 also applies mutatis mutandis to the EPS architecture in Figure 6.10.2-2.

### 6.10.3	Procedures

There is no impact on existing S1 or N2/N3 procedures from EPC and 5GC perspective, respectively.

### 6.10.4	Impacts on services, entities and interfaces

The solution has no impact on UE, 5GC or EPC.

The Interworking Function (IWF) is perceived by 5GC as a static gNB/eNB and is not supposed to require any standardisation in SA2 specifications, other than description of the IWF functionality in an informative annex. The OAM aspects of the IWF are expected to be defined by SA5 WG.

## 6.11	Solution #11: MME Split Architecture to support Control plane procedure for S&F

### 6.11.1	Key Issue mapping

This solution aims to resolve Key Issue #2, "Support of Store and Forward Satellite operation".

### 6.11.2	Description

In this proposal, the MME, which is the mobility anchor for the UE, is split into 2 network elements, MME-NT (MME on board satellite) and MME-T (MME terrestrial). The MME-NT shall act as a mobility anchor point to UE and does only subset of MME functionality such as assist in Store and Forward operations i.e. combination of two steps not concurrent in time. The MME-T on the ground is responsible for maintaining UE context, including security context. MME-T is also responsible for NAS ciphering/deciphering and integrity protection. MME-NT is responsible for maintain S1 connection towards RAN node and maintain associated connection ID per UE towards MME-T. MME-NT is responsible for encoding NAS message received from MME-T into S1AP payload towards RAN and decode NAS payload from messages received from RAN and send it to MME-T.

Figure 6.11.2-2: Example of split MME architecture based on UE coverage or ground station connectivity at different time

The Store and Forward Satellite (S&F) operation in a EPC system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

Figure 6.11.2-1: Example of MME split architecture for Store and Forward operation

In regenerative architecture, the RAN will be placed in a satellite. In the Store & Forward (S&F) scheme, the MME is at satellite will be responsible for providing Store and forward feature. MME in satellites are represented as MME-NT (non-terrestrial) in the above diagram. Let's assume the time T1 is between 10:00 and 10:20 and T2 is between 10:40 and 11:00. The satellites will have ground station coverage when hovering over Rennes and Orleans but not when crossing Le Mans.

At time T1, the black satellite covers Le Mans, and the white satellite provides coverage to Rennes. At T2, the black satellite will be at Orleans and the white satellite will be covering Le Mans.

At time T3, the white satellite will sync up with the MME-T at Orleans.

### 6.11.3	Procedures

#### 6.11.3.1	Architecture enhancement to support S&F

In regenerative architecture, the RAN will be placed in Satellite. In the Store & Forward (S&F) scheme, the MME is at Satellite will be responsible for providing Store and Forward feature. MME in the satellite is represented as MME-NT-1 (non-terrestrial) in the above diagram. E.g. in above figure 6.11.2-2 for the time T1 is between 10:00-10:20 and T2 is 10:30-10:50 and the satellite at T1 is providing coverage towards UE but not connected to the ground station, it shall store UL messages and forward DL messages towards UE; When the satellite at T2 is not providing coverage for UE but connected to ground station shall store any DL messages for the UE and forward any UL messages it had received at T1. At T3, the MME-T can select the appropriate MME-NT-2 in satellite for storing DL messages meant for the UE.

Figure 6.11.2-1: Architecture enhancement for control plane in Store and Forward operation

-	MME-T manages all the existing functionality of existing MME. Instead of maintaining S1AP ID per UE context, it will maintain a context identifier for the S10` messages towards MME-NT

-	MME-T is responsible for NAS ciphering/integrity protection function.

-	MME-T needs to forward the RAN S1AP ID towards MME-NT to help MME-NT to maintain UE context per S1AP instance.

NOTE 1: The MME-NT needs to store the interim GUTI information and the identifier of the serving MME-T. The MME-NT can additionally store RAN S1-AP ID and RAN control plane IP address and similarly MME S1-AP ID vs MME-T address.

#### 6.11.3.2	Attach Procedure in EPC in S&F scenario

Figure 6.11.3-2: Attach procedure in S&F

1.	A UE, camping on an E-UTRAN cell reads the related System Information Broadcast including whether supports S&F operation. If the UE is able to perform the S&F operation, it initiates the Attach procedure by the transmission, to the eNodeB, of an Attach Request (IMSI or old GUTI, Old GUTI type, last visited TAI (if available) and the S&F indication). The S&F indication in the Attach Request may be used to indicate MME-NT that the UE is trying to attach for S&F communication.

2.	The eNodeB(RAN-1) forwards the Attach Request message in a S1-MME control message (Initial UE message) towards MME-NT-1. In the case of satellite access for Cellular IoT, the MME-NT-1 may verify the UE location and determine whether the PLMN is allowed to operate at the UE location.

3.	If the MME-NT-1 is not in contact with the ground station when receiving a message in step2, MME-NT-1 shall store the attach request message and, generate a interim GUTI if the UE has included S&F indication in attach request and send an new NAS clear text message towards UE asking to save the interim GUTI for future NAS transactions. MME shall also provide the validity time for this interim GUTI. (UE may send an ack for the same, not shown in the diagram).

4.	When MME-NT-1 regains ground connectivity, it shall forward the attach request, IMSI along with the interim GUTI created for this request in step3 towards MME-T. A UE having valid interim GUTI shall not reset the MM context.

5	If no UE context for the UE exists at the MME-T, and if the Attach Request (sent in step 1) was not integrity protected, or if the check of the integrity failed, then authentication and NAS security setup to activate integrity protection and NAS ciphering are carried out. The MME-T borrows the AV from HSS.

6.	MME-T, after getting the authentication key from HSS, shall try to ascertain the next available satellite which can reach the UE next. When found, it shall create the Authentication NAS payload and forward it to MME-NT-2 (the next available satellite to reach UE). MME-T shall also provide the last known location of UE.

NOTE 1:	The MME-T will need the information on next suitable satellite which can serve the UE in shortest possible time. This information can be provided within the PLMN, by a UE reachability Estimator (URE) (as presented in Solution #25), or from an external source.

6.	When MME-NT-2 reaches the UE area, it will page the UE using either the IMSI or interim GUTI or both or UE can also reach out to RAN when seeing the new cell and reattempt the attach procedure with the previously valid interim GUTI. In both cases, when the UE gets RRC connected, the MME-NT shall forward the stored (in step 6) Authentication Request message to UE.

7.	UE responds back with an authentication response towards MME-NT-2, which MME-NT-2 shall store it till it regains ground connection.

8.	When MME-NT-2 regains connectivity with the ground station, it shall forward the stored Authentication response from UE to MME-T. MME-T validates the response.

9.	If the UE is authenticated by MME-T successfully, MME-T initiates Security mode by selecting the next available satellite (as per NOTE 1) that can serve the UE next. When found, it shall relay the Security mode command to MME-NT-3 (shown as MME-NT-1 in the diagram). MME-NT-3 shall store the security mode command until it reaches the UE serving area. MME-T shall also provide the last known location of UE.

10.	When the MME-NT-3 reaches the UE serving area, it shall page the UE using the interim GUTI or IMSI or both. or UE can also reach out to RAN when seeing the new cell and reattempt the attach procedure with the previously valid interim GUTI. When the UE becomes connected, the MME-NT-3 will forward the stored security mode command message to UE.

11.	Once the UE applies the security mode, it shall acknowledge the security mode command towards MME-NT-3. MME-NT-3 shall store it till it regains the ground connection again.

12.	When the MME-NT-3 regains ground connectivity, it relays the stored security mode ack message to MME-T.

13.	After receiving the security mode ack from MME-NT-3, the MME-T selects a Serving GW and allocates an EPS Bearer Identity for the Default Bearer associated with the UE. Then it sends a Create Session Request (IMSI, MSISDN, MME-T TEID for control plane, PDN GW address, PDN Address, APN) message to the selected Serving GW.

14.	The Serving GW creates a new entry in its EPS Bearer table and sends a Create Session Request (IMSI, MSISDN, APN, Serving GW Address for the user plane, Serving GW TEID of the user plane, Serving GW TEID of the control plane) message to the PDN GW indicated by the PDN GW address received in the previous step. (not shown in this diagram and SGW+PGW are represented as SAE GW).

15.	When the create session is received from SAE-GW, the MME-T shall store till it finds the next available satellite to serve the UE next. Once found, the MME-T shall send Attach accept along with Create session response information such as user plane address and TEID to MME-NT-4. MME-T shall also provide the last known location of UE.

16.	The MME-NT-4 shall store the message until it reaches the UE serving area again. When it reaches the UE serving area, it shall page the UE using the interim GUTI, IMSI or both. UE can also connect on its own when it sees a new cell in a new satellite. In either case, when the UE comes to connected state, the MME-NT-4 shall forward the message to RAN and UE. The RAN will create its PDN resources based the user plane IP and TEID information. The UE shall receive the Attach Accept, new GUTI, along with the PDN information.

17.	UE shall acknowledge by sending the Attach complete message to MME-NT-4. MME-NT-4 shall store it till it regains the ground connectivity.

18.	When the MME-NT-4 regains the ground connectivity, it shall forward the stored Attach complete message and uplink data if any to the MME-T.

19.	After receiving Attach Complete, MME-T shall configure the PDN connection by sending a Modify bearer Request to SAE-GW by including RAN's tunnel ID and IP information. (not shown in the diagram).

NOTE 2:	The UE shall support storing interim GUTI and use it to respond to Paging or while sending any initial NAS request until UE is attached to the network.

NOTE 3:	MME-NT should be able to create distinct interim GUTI (one assumption is to have a distinct MME ID for each MME in satellite).

NOTE 4:	MME-NTs are not keeping any security material (both AS and NAS). Integrity and ciphering are to be done at MME-T for every NAS message. This is to keep MME-NT as lightweight as possible to save precious Satellite energy resources.

NOTE 5:	How to create trust between MME-T and MME-NT is up to SA WG3.

NOTE 6:	Due to the dependency on paging procedures (steps 6, 10 and 16) to bring the UE into connected mode for DL signalling messages, the failure in paging will prolong the attach procedure.

NOTE 7: The DoS attack on MME-NT due to handling of integrity protected messages without owning the NAS security keys needs to be studied in SA3.

NOTE 8: According to the TS 23.401(clause 4.13) and TS 24.301 (clause 10), for the introduction of satellite support for Cellular IoT, whenever the UE is accessing the network using a satellite RAT, the MME and the UE shall set the NAS timers long enough according to this satellite RAT. However, for the Store and Forward scenarios it is expected that the NAS timers need to have much longer duration, depending on the deployment of the satellites. A more detailed analysis about the estimation of NAS timers can be considered during the normative phase, in collaboration with CT1 and RAN WGs.

NOTE 9: A UE can be denied accessing any other available PLMN till the time the validity timer of the interim GUTI is not expired in the UE. i.e. UE can not select any other network, until the UE is rejected or deregistered from the current PLMN providing S&F mode of service. Also, any fake MME onboard satellite can potentially assign a larger validity timer to UE to withhold the UE from accessing or selecting a valid network. How to make sure the trustworthiness of the satellite and its onboard RAN/MME is up to SA3 study (e.g. allocating trust keys in both UE and onboard network element to validate UE request and network response). The UE needs additional PLMN selection/reselection criteria upon expiry of the interim GUTI validity timer to select/reselect PLMN appropriate for required service.

#### 6.11.3.3	MO Data Transport in Control Plane CIoT EPS Optimisation in S&F

High level procedure of Control plane CIoT EPS optimization in Store and Forward scenario be shown as following.

Figure 6.11.3.1-1: High-level Procedure of S&F Operation for MO Data Transport in Control Plane CIoT EPS Optimisation

0.	The UE is ECM-IDLE.

1.	The UE establishes a RRC connection and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data. The NAS PDU is relayed to the MME-NT-1 by the eNodeB using a S1-AP Initial UE message.

2.	The MME-NT-1 shall store the NAS payload without integrity check or deciphering until the satellite regains ground connection.

NOTE 1: The MME-NT doesn’t need to create any UE context, rather it will transparently pass the information to MME-T. The MME-NT can optionally store RAN-S1AP ID for future transactions throughout the procedure completion.

3.	The MME-NT-1 sends the NAS PDU received in step 1 to MME-T when the Satellite 1 regains ground connection.

4.	The MME-T checks the integrity of the incoming NAS PDU and decrypts the data it contains.

5.	If the S11-U connection is not established, the MME sends a Modify Bearer Request message for each PDN connection to the Serving GW.

6.	If the Info IEs and/or UE Time Zone and Serving Network id are present in step 4, the Serving GW shall send the Modify Bearer Request message (RAT Type, MO Exception data counter) to the PDN GW.

7.	The PDN GW sends the Modify Bearer Response to the Serving GW.

8.	If a Modify Bearer Request message was sent at step 5 the Serving GW shall return a Modify Bearer Response (Serving GW address and TEID for uplink traffic) to the MME as a response to a Modify Bearer Request message. The Serving GW address for S11-U User Plane and Serving GW TEID are used by the MME to forward UL data to the SGW.

9.	The MME sends Uplink data to the P-GW via the S-GW.

10.	If the MME-T is not aware of pending MT traffic and S1-U bearers are not established, step 11 is skipped and step 13 applies.

11.	If Downlink data are received in step 10, the MME-T ciphers and integrity protects the Downlink data.

12.	MME-T stores the DL message until it find suitable MME-NT which can serve the UE area next.

NOTE 2:	The MME-T will need the information on next suitable satellite which can serve the UE e.g., in shortest possible time. This information can be provided within the PLMN, by a UE reachability Estimator (URE) (as presented in Solution #25) or from an external source.

13a-b.	Once a suitable satellite is found, the MME-T will forward the NAS DL message to MME-NT-2. If step 11 is executed then Downlink data are encapsulated in a NAS PDU and sent to the eNodeB in a S1-AP Downlink NAS Message.

14.	The MME-NT-2 shall attempt to page the UE via its own RAN on-board the same satellite. The UE responds and get RRC connected.

15.	The eNodeB sends a RRC Downlink data message including the Downlink data encapsulated in NAS PDU.

16.	The eNodeB sends a NAS Delivery indication to the MME-NT-2 if requested, MME-NT-2 shall store the "S1-AP NAS Non Delivery Indication" till it regains ground connection.

17.	When ground connectivity is available for MME-NT-2, it will deliver the stored delivery indication NAS message to MME-T.

NOTE 3: According to the TS 23.401(clause 4.13) and TS 24.301 (clause 10), for the introduction of satellite support for Cellular IoT, whenever the UE is accessing the network using a satellite RAT, the MME and the UE shall set the NAS timers long enough according to this satellite RAT. However, for the Store and Forward scenarios it is expected that the NAS timers will have much longer duration, depending on the deployment of the satellites. A more detailed analysis about the estimation of NAS timers could take place during the normative phase, in collaboration with CT1 and RAN WGs.

#### 6.11.3.4	Connection Suspend and Resume Procedure

Figure 6.11.3.4-1: Connection Suspend and Resume procedure

0.	The UE is in RRC connected and ECM-Connected.

1.	The eNodeB(RAN-1) in first satellite initiates the Connection Suspend procedure.

2.  The eNodeB(RAN-1) indicates to the MME-NT-1 that the UE's RRC connection is to be suspended. Data related to the S1AP association, UE Context and bearer context, necessary to resume the connection is kept in the eNodeB(RAN-1), UE and the MME-NT-1.

3.	eNodeB (RAN-1) sends RRC message to suspend the RRC Connection towards the UE, as described in TS 36.300.

4.	The MME-NT-1 transmits the S10 “UE Context Suspend notification” message including UE context information (bearer context, AS security context and resume ID) to the ground MME (MME-T).

5.  The MME-T stores the UE context information that can be retrieved once the MME-T finds a suitable satellite containing MME-NT which can reach the UE. MME-T marks the UE as ECM-idle, but the data related to the S1AP association, UE Context and bearer context are kept intact, which is necessary to resume the connection between the UE and a satellite (that will resume the connection with the UE)

6. The MME-T sends a Release Access Bearers Request message to the Serving GW that requests the release of all S1-U bearers for the UE.

7. The Serving GW releases all eNodeB related information (address and downlink TEIDs) for the UE and responds with a Release Access Bearers Response message to the MME. Other elements of the UE's Serving GW context are not affected. If downlink packets arrive for the UE, the Serving GW starts buffering downlink packets received for the UE and initiating the "Network Triggered Service Request" procedure, described in clause 5.3.4.3 in TS 23.401. The Serving GW informs the MME in the Release Access Bearer Response message about release of S1-U bearers.

8.	The MME-T determines which non-terrestrial network element(s) (i.e., satellite(s)) could provide coverage to the UE. The MME-T, according to identified satellites (e.g. MME-NT-2), triggers the resume notification procedure for one or more satellites.

NOTE 1:	The MME-T will need the information on next suitable satellite which can serve the UE in shortest possible time. This information can be provided within the PLMN, by a UE reachability Estimator (URE) (as presented in Solution #25), or from an external source.

9. The MME-T transmits an S10 “UE Context Resume notification” message to the MME-NT-2 including UE context information, resume ID to assist the resume of the connection of the UE, DL tunnel information of S-GW towards the selected satellites (e.g., Satellite 2) and E-RAB request to create tunnel resources in RAN.

10. The MME-NT-2 transmits to the eNodeB(RAN-2) UE Context Resume notification. The eNodeB(RAN-2) creates the access network bearers to receive the DL traffic from the S-GW and to store the UE Context information and resume ID for the UE resumption procedure once Satellite 2 can provide coverage/connection service to the UE. RAN-2 creates necessary UL U-plane resources.

11-12. The RAN-2 creates the UE context for resuming UE. RAN-2 shall send an acknowledgement to notify on the resumption of context towards MME-NT2

13. The MME-NT2 transmits to the MME-T confirmation that the access bearer has been established, providing access network tunnel information (S1 TEID(s) (DL)) for the provision of the DL traffic from the S-GW to the eNodeB (RAN-2).

14. The MME-T sends a Modify Bearer Request message to the Serving GW including the access network tunnel information of eNodeB (RAN-2) that requests to to establish the user plane with the eNodeB (RAN-2).

15. The Serving GW sends the Modify Bearer Response to the MME-T.

14. The S-GW transmits the buffered DL packets to the eNodeB(RAN-2). The RAN-2 shall store it against the UE context until the RRC resume operation is not complete. The MME can trigger paging via RAN-2 to indicate DL stored data.

15. The UE, once is under the coverage of the eNodeB(RAN-2) or on receipt of paging, triggers the RRC Connection Resume procedure including information needed by the eNodeB (RAN-2) to access UE's stored AS context, according to the Connection Resume procedure in TS 36.300

NOTE 2: In case of no paging from MME, how the UE determines the next available RAN for RRC context resume is up to the UE implementation and/or RAN.

#### 6.11.3.5	MT Data Transport in Control Plane CIoT EPS Optimisation in S&F

Figure 6.11.3.5-1 depicts a high-level Procedure of S&F Operation for MT Data Transport in Control Plane CIoT EPS Optimisation.

Figure 6.11.3.5-1: High-level Procedure of S&F Operation for MT Data Transport in Control Plane CIoT EPS Optimisation

0.	The UE is ECM-IDLE.

1.	PGW sends the downlink data to the SGW. SGW buffers the downlink data packet.

2.	If the Serving GW is buffering data in step 1, SGW sends a Downlink Data Notification message to MME-T. The MME responds to the SGW with a Downlink Data Notification Ack message.

3.	When there is available MME-NT (MME-NT-1), the MME-T establish the S11-U with SGW. The MME sends the Modify Bearer Request message (MME address, MME TEID DL) to SGW.

4.	SGW responds to the MME-T the Modify Bearer response (Serving GW address and TEID for uplink traffic).

5.	Buffered Downlink data is sent by the S-GW to the MME-T.

6.	MME-T encrypts and integrity protects Downlink data

7.	MME-T sends DL data to MME-NT-1 with UE context using the S10 message to the MME-NT-1.

8.	The MME-NT-1 creates the temporary UE context for the ciphered DL data associated with the GUTI.

9.	The MME-NT-1 attempts to page the UE via its own eNB on-board on the same satellite.

10.	The UE responds and get connected.

11.	The MME-NT-1 sends the DL data using the DL S1-AP msg to the eNB.

12.	eNB forwards the DL data to the UE.

13.	eNB responds to the MME-NT-1 of the NAS delivery notification.

14. When ground connectivity is available for MME-NT-1, it will deliver the stored NAS delivery notification message to MME-T.

### 6.11.4	Impacts on services, entities and interfaces

UE:

-	NAS impact to handle larger delay due to S&F mode of operation.

-	NAS impact due to new interim GUTI allocation.

MME-T:

-	Handling of new S10 messages to carry both ciphered and plain text NAS messages to/from MME-NTs.

- 	Transmits Resume notification messages to MME-NT, in case of connection Suspend and Resume procedure.

MME-NT:

-	Handling of new S10 messages to carry both ciphered and plain text NAS messages to/from MME-T.

-	Handling of plain text NAS and NGAP messages on behalf of MME-T.

-  Transmits Suspend notification messages to MME-T, in case of connection Suspend and Resume procedure

## 6.12	Solution #12: S&F for multiple satellite deployment with anchor MME-ground

### 6.12.1	Description

For control plane management of registration and connection management procedures, we propose to have eNB and MME onboard the satellite.

Below are the principles of the solution:

1.	MME-ground is an anchor node situated in ground network which has the UE context. MME-ground synchronizes the UE context with all the MME-onboard(s).

2.	The eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

3.	Whenever a procedure needs an interaction with a core network node in the ground then MME-onboard(stores it when feeder link is not available) and forwards the respective message to the MME-ground when feeder link is available, the MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard or sends back the response message to the MME-onboard.

4.	The MME-ground first receives information on any downlink signalling message, it forwards to MME-onboard(when feeder link is available), the MME-onboard can page the UE(when service link is available) and deliver respective message/execute the procedure with the UE.

5.	To reduce number of iterations, if the MME-ground can be trusted then Authentication information and Subscription data fetching(i.e. update location and ack) is performed together before the UE can be authenticated. This is the decision of the HSS if such procedure should be allowed. If HSS does not allow such combining of the procedure to complete the attach procedure at least two iterations of interaction with ground network is required a) to fetch authentication vectors and b) to fetch subscription data after authentication procedure is executed in first iteration.

6.	GUTI management, Periodic registration management, how UE identifies that it is allowed to register for S&F are described in procedures section.

7.	Satellite-1, Satellite-2 are used as illustration in the flow. But the system can deploy "n" number of satellites for the same flow(or procedure).

### 6.12.2	Procedures

Figure 6.12.2-1: Attach without PDN connectivity

The eNB, MME-onboard are assumed to be onboard the satellite, there is a ground MME which is acting like an anchor for the MMEs onboard the satellite.

The attach procedure as described in clause .5.3.2.1 of TS 23.401are executed with below modifications.

At time T0 (i.e. when service link is available, and feeder link connectivity is not available).

1)	In step 1, if the UE identifies that current serving cell support S&F mode and the UE is allowed to use S&F(see clause 6.12.3.1.2). then UE sends Attach Request message to network.

2)	Steps up to step 4 in clause .5.3.2.1 of TS 23.401 [5] are executed between MME-onboard and the UE.

3)	MME-onboard sends NAS message Partial attach accept(which can be unprotected) to the UE, indicating to the UE that the sent ATTACH REQUEST message is stored by the MME-onboard and network will reach UE once the UE authentication and subscription details are fetched by the MME-onboard from ground network. The Partial attach accept includes a temporary GUTI (though UE is not registered with network) and a wait timer till which UE should wait for network to send the paging message. The UE shall not select any other network which will provide store and forward service to the UE till the wait timer is running. The UE may select another network if available which can provide normal services to the UE while wait timer is running. The MME memorizes the UE serving area.

4)	The UE receives Partial attach accept message. The UE shall not trigger Attach request again until paging message is received or wait timer is expired.

Editor's note:	Security aspects of not performing authentication for steps 1-4 (e.g. denial of service attack, false BS attack) need to be resolved.

At time T1 (i.e. when service link is not available, and feeder link connectivity is available).

5)	MME-onboard selects MME-ground(see clause 6.12.3.1.3). MME-onboard forward stored Attach request message, UE serving area to MME-ground.

6)	Step 5 is executed between MME-ground and HSS, i.e. MME-ground fetches the authentication vector and other details from HSS.

7)	Steps 8 & 11 are executed. i.e. Update location with HSS and Update location ACK is received by MME-ground. i.e. all the subscription details are retrieved by MME-ground. The MME indicates S&F to HSS to retrieve Subscription details specific to S&F(if any) and indicates that it is pre-fetching the subscription data without authenticating the UE.

8)	The MME-ground syncs the UE information it retrieved with the MME-onboard. For optimization purpose, the MME-onboard can be any MME-onboard which will serve the UE next.

At time T2 (i.e. when service link is available, and feeder link connectivity is not available).

9)	The MME-onboard when enters the UE serving area will page the UE with the assigned GUTI in Partial attach accept message.

10)	The UE in response to receiving the paging message, the UE re-sends the Attach request message.

Editor's note:	Recovery from failure to page the UE is FFS.

11)	MME-onboard executes the step 5 of authentication and security procedure with the UE, once authentication procedure is successful then system executes remaining steps to complete Attach procedure with the UE. The MME-onboard also provide S&F policies to the UE which includes a UE context activation timer. The UE starts UE context activation timer after it receives the NAS message, the UE should consider itself registered with the network at the expiry of this timer. The MME based on subscription data available may send the reject message to the UE before or after the authentication/security procedure are executed.

At time T3(i.e. when service link is not available, and feeder link connectivity is available).

12)	MME-onboard indicates to MME-ground, if UE is successfully registered or registration procedure was not successful. MME-onboard syncs UE context with MME-ground if successful registration procedure is achieved.

13)	MME-ground sends Update location indicating to HSS that UE is authenticated successfully or not authenticated successfully, which implies to HSS that subscription data is not required in response.

14)	If UE context is created or changed in any of the MME-onboard, this will be synced with all the MME-onboard(s) when they connect with ground network. Deployment should handle that this sync of UE context with all satellites should not take more time than UE context activation timer.

Editor's note:	After change to the UE context in an MME-onboard, the UE must wait until the new UE context is transferred to other satellites before again accessing a satellite. How this can be managed is FFS.

15)	After expiry of UE context activation timer the UE enters EMM-REGISTERED state.

##### 6.12.2.1.1	Periodic timer and mobile reachable timer

The mobile reachable timer(MRT) is run only at the MME-ground, each time UE gets in connected mode with any of the MME-onboard, the MME-onboard informs the MME-ground, then MME-ground restart the MRT timer.

The mobile reachable timer value has to be increased at network so that it takes into account delay in receiving information from the MME-onboard after UE has come to connected mode( including case of PTAU procedure).

For e.g. Mobile reachable timer = PTAU timer given to UE + Maximum potential delay for MME-onboard to connect with MME-ground after UE has come to connected mode( including case of PTAU procedure).

Each MME-onboard runs a MME-UE-context onboard timer, at expiry of this timer, the MME-onboard synchronizes the UE context with MME-ground.

##### 6.12.2.1.2	How UE identifies that it is allowed to attach with network for S/F

If UE determines that it has no other network which can provide normal services then UE may determine to attach in store and forward mode. Similar to CAG mechanism available in 5GS, we propose that UE is configured with S&F information and if configured S&F ID is broadcasted along with the indication of support for S/F mode(i.e. feeder link is not available) then UE will attach with the network in S&F mode. If the UE is not configured with S&F ID and the serving network broadcasts support of S&F(i.e. feeder link is not available) then UE may attempt attach to register and receive allowed S&F IDs from network.

##### 6.12.2.1.3	GUTI management and MME selection

The GUTI is assigned to the UE only by the MME-ground which is stored commonly with all the MME(s)-onboard in the UE context. The eNB-onboard selects the MME-onboard:

a)	Based on the MME-group ID which can be kept common for all the MME(s)-onboard which can serve the UE based on UE serving area and the MME-ground; or

b)	Following range of MME IDs which are expected to be same for selection. i.e if eNB receives one MME-ID(during RRC connection establishment) it can select any of the MMEs it has connection from the range of MME IDs.

c)	The eNB will follow current mechanism to select MME-onboard, it may take into account S&F indication in the RRC signalling procedure

##### 6.12.2.1.4	MME-ground selection by MME-onboard

MME-onboard selects the MME-ground when UE first attaches the network.

MME-onboard should follow same selection mechanism as eNB selects MME in Rel-18 to select a MME-ground.

The MME-onboard is assume to support all the features MME-ground supports.

MME-ground once assigns the GUTI, MME-onboard can uniquely identify the MME-ground, MME-onboard is configured with all ground station IDs which MME-onboard can connect to reach MME-ground. This information is provided to all MME(s)-onboard during the UE attach procedure and successful UE context is synchronized.

MME-ground is configured with potential MME-onboard(s) which can serve the UE based on the UE serving area. The MME-ground is locally configured with this information or can be configured by O&M or AF(through SCEF path).

##### 6.12.2.1.5	Authentication procedure/subscription details fetch during any other procedure

If network determines to execute authentication procedure/fetch subscription details etc during any time for e.g. during service request procedure execution then the concept of partial attach accept discussed above is applicable. i.e. UE is indicated in NAS message that network has stored uplink message and network will get back to the UE once UE authentication/subscription details are fetched from ground network.

##### 6.12.2.1.6	Attach with PDN connectivity, multiple satellite

The gNB, MME, S-GW and P-GW are assumed to be onboard the satellite.

Assumption-1: If we assume there is no need for PCRF because PCC policies can be configured in the P-GW onboard satellite because. communication is for delay tolerant devices there is no specific or very small policies may need to be applied.

Assumption-2: In case of dynamic PCC deployment is required then PCRF also is assumed to be on board the satellite.

Then is no changes are expected on top of above flow. Because P-GW/S-GW/PCRF are available for interactions with MME onboard the satellite to establish the PDN connections

Similar to MME-ground, there is a P-GW-ground, S-GW-ground to sync with the UE context.

Editor's note:	Any other enhancements required to handle data transfer or onboard core network entities is FFS.

### 6.12.3	Impacts on services, entities and interfaces

a)	MME-ground is an anchor node situated in ground network which has the UE context. MME-ground synchronizes the UE context with all the MME-onboard(s).

b)	The eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures with the UE. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

c)	Whenever a procedure needs an interaction with a core network node in the ground then MME-onboard(stores it when feeder link is not available) and forwards the respective message to the MME-ground when feeder link is available, the MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard or sends back the response message to the MME-onboard.

d)	GUTI management, Periodic registration management, how UE identifies that it is allowed to register for S&F are described in the procedures clause.

## 6.13	Solution #13: S&F for single satellite deployments with eNB and MME-onboard

### 6.13.1	Description

a)	Single satellite deployment in this solution implies out of possible N satellites deployed by network, the UE is served by only one satellite. The N can be one or more than one i.e. network can choose to deploy only one satellite for all UEs or more than one satellites catering to different subset of UEs.

b)	The eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

c)	Whenever a procedure needs an interaction with a core network node in the ground then MME-onboard stores the respective message when feeder link is not available and progresses the procedure when feeder link is available.

d)	To reduce number of iterations, if the MME-onboard can be trusted then Authentication information and Subscription data fetching(i.e. update location and ack) is performed together before the UE can be authenticated. This is the decision of the HSS if such procedure should be allowed. If HSS does not allow such combining of the procedure, to complete the attach procedure at least two iterations of interaction with ground network is required 1) to fetch authentication vectors and 2) to fetch subscription data after authentication procedure is executed in first iteration.

e)	The NAS signalling messages are stored at MME-onboard and on data path at P-GW when service link is available and feeder link is not available. This is delivered to ground network when feeder link is available

f)	Other details are described in procedures clause.

### 6.13.2	Procedures

Figure 6.13.2-1: Attach without PDN connectivity

Case-1: Attach without PDN connectivity, single satellite.

The eNB, MME are assumed to be onboard the satellite. rest of the system is on the ground.

The attach procedure as described in clause .5.3.2.1 of TS 23.401 [5] are executed with below modifications:

At time T0 (i.e. when 5G system on board satellite is serving the UE, and feeder link connectivity is not available)

1)	In step 1, if the UE identifies that current serving cell support S&F mode and the UE is allowed to use S&F (see clause 6.13.2.1.2). then UE sends Attach Request message to network. The UE may additionally include IMSI identifier even though it has GUTI to avoid potential identification procedure.

2)	Steps up to step-4 as described in clause 5.3.2.1 of TS 23.401 [5] are executed between MME and the UE,

3)	MME-onboard sends NAS message Partial attach accept(which can be unprotected) to the UE, indicating to the UE that the sent ATTACH REQUEST message is stored by the MME-onboard and network will reach UE once the UE authentication and subscription details are fetched by the MME-onboard from ground network. The Partial attach accept includes a temporary GUTI (though UE is not registered with network) and includes a wait timer till which UE should wait for network to send the paging message. The UE shall not select any other network which will again provide store and forward service to the UE till the wait timer is running. The UE may select another network if available which can provide normal services to the UE while wait timer is running. The MME memorizes the UE serving area.

Editor's note:	Any other enhancements to handle temporary context is FFS.

4)	The UE receives Partial attach accept message. The UE shall not trigger Attach request again until paging message is received or wait timer is expired.

At time T1 (i.e. when 5G system on board satellite is connected to ground station but cannot connect to the UE, and feeder link connectivity is available)

5)	Step 5 in clause .5.3.2.1 of TS 23.401 [5] is executed between MME and HSS, i.e. MME fetches the authentication vector and other details from HSS etc..

6)	Steps 8 & 11 are executed. i.e. Update location with HSS and Update location ACK is received by MME. i.e. all the subscription details are retrieved by MME. The MME indicates S&F to HSS to retrieve Subscription details specific to S&F(if any) and indicates that it is pre-fetching the subscription data without authenticating the UE.

At time T2 (i.e. when 5G system on board satellite start serving the UE again, and feeder link connectivity is not available).

7)	The MME when enters the UE serving area will page the UE with the assigned GUTI in Partial attach accept message.

8)	The UE in response to receiving the paging message, the UE re-sends the Attach request message.

Editor's note:	Any additional handling required if UE misses paging message if FFS.

9)	MME executes the step 5 in clause .5.3.2.1 of TS 23.401 [5] authentication and security procedure with the UE, once authentication procedure is successful then system executes remaining steps to complete Attach procedure with the UE. The MME may send ATTACH ACCEPT before receiving security mode complete from UE to combine this procedures, in this case the UE will process security mode command, if successful, followed by ATTACH accept and send Attach complete (including security mode complete) message response to network. The MME also provide S&F policies to the UE. The MME based on subscription data available may send the reject message to the UE before or after the authentication/security procedure are executed.

At time T3:

10)	MME sends Update location indicating to HSS that UE is authenticated successfully or not authenticated successfully.

Case-2: Attach with PDN connectivity, single satellite.

The gNB, MME, S-GW and P-GW are assumed to be onboard the satellite.

Assumption-1: If we assume there is no need for PCRF, due to at least one of below assumption:

1.	PCC policies can be configured in the P-GW onboard satellite; or

2.	As communication is for delay tolerant devices there is no specific or very small policies may need to be applied.

Assumption-2: In the case of dynamic PCC deployment is required then PCRF also is assumed to be on board the satellite.

Then no changes are expected on top of flow of case-1. Because P-GW/S-GW/PCRF are available for interactions with MME onboard the satellite to establish the PDN connections. Any data sent over data path is stored at P-GW. There can be proxy AF deployed over the satellite if real time acks have to be delivered to the UE. IN this case proxy AF will store any data which is delivered to ground AF when feeder link is available.

Editor's note:	Any other enhancements required to handle data transfer or onboard core network entities is FFS.

NOTE:	Any security impacts due to this solution will be determined by SA WG3.

#### 6.13.2.1	Other changes:

##### 6.13.2.1.1	Periodic timer and mobile reachable timer

The mobile reachable timer value has to be increased at network so that it takes into account delay in serving the UE after PTAU timer has expired at the UE.

For e.g. Mobile reachable timer = PTAU timer given to UE + Maximum potential delay for MME-onboard to again serve the UE.

##### 6.13.2.1.2	How UE identifies that it is allowed to attach with network for S/F

If UE determines that it has no other network which can provide normal services, then UE may determine to attach in store and forward mode. Similar to CAG mechanism available in 5GS, we propose that UE is configured with S&F information and if configured S&F ID is broadcasted along with the indication of support for S/F mode then UE will attach with the network in S&F mode.

##### 6.13.2.1.3	UE selecting single satellite in a given area

If an area can be served by multiple satellites, it is assumed that each eNB onboard the satellite will broadcast different TAI(s). This TAI(s) other than of serving satellite are provided as forbidden TAIs to the UE, so that UE can avoid selecting other satellites(except the serving satellite).

Editor's note:	If any additional handling required is FFS.

##### 6.13.2.1.4	Authentication procedure/subscription details fetch during any other procedure

If network determines to execute authentication procedure/fetch subscription details etc during any time for e.g. during service request procedure execution then the concept of partial attach accept discussed above is applicable. i.e. UE is indicated in NAS message that network has stored uplink message and network will get back to the UE once UE authentication/subscription details are fetched from ground network.

### 6.13.3	Impacts on services, entities and interfaces

This solution impacts the following system entities:

a)	The eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

b)	Whenever a procedure needs an interaction with a core network node in the ground then MME-onboard stores the respective message when feeder link is not available and progresses the procedure when feeder link is available.

c)	The NAS signalling messages are stored at MME-onboard and on data path at P-GW when service link is available and feeder link is not available. This is delivered to ground network when feeder link is available

d)	Other aspects are described in procedures clause.

## 6.14	Solution #14: Initial Registration with prior rejection in satellite network for S&F operation

### 6.14.1	Key Issue mapping

This solution addresses the Initial Attach / Initial Registration of Key Issue #2 (Support of Store and Forward Satellite operation).

The Initial Attach / Initial Registration procedure is assumed to be common to all S&F services.

### 6.14.2	Description

Consider the extreme case depicted in Figure 6.14.2-1 with a satellite orbit consisting of only one LEO satellite:

Figure 6.14.2-1: Incomplete satellite orbit with only one satellite

Assuming that the satellite orbit is located at 500 km altitude, the orbital period is roughly equal to 90 minutes. The size of the satellite's coverage area depends on several factors, but for simplicity it is assumed that a given spot on the Earth is served by the satellite for e.g. 5 minutes during each satellite pass. Due to Earth's rotation, a given satellite will pass over the same geographical area only after several orbits. Given that there is only one satellite in Figure 6.14.2-1, the UE will be served intermittently i.e. only during e.g. 5 minutes within a period of several hours.

In the scenario depicted in Figure 6.14.2-1 the satellite establishes a contact with the UE and with the ground station at T1 and T3, respectively. At no time does the satellite have simultaneous connection to both the UE and the ground station due to the absence of Inter-Satellite Links (ISLs) with additional satellites.

As a consequence, none of the signalling procedures between the UE and the Core Network can be executed in their entirety as an atomic procedure.

The solution is based on the EPS architecture. It is assumed that the MME function is split into two parts: MME-SAT functionality residing on the satellite (together with RAN functionality) and MME-GND functionality residing on the ground. This is illustrated in Figure 6.14.2-2.

Figure 6.14.2-2: Satellite architecture for S&F operation (registration related)

The exact split of functionality between MME-SAT and MME-GND, as well as the definition of Ssf reference point, is considered outside of 3GPP scope.

Functionality that is not relevant for the Registration procedure is omitted from Figure 6.14.2-2.

### 6.14.3	Procedures

The procedure for Initial Attach in a Satellite network for S&F operation is depicted in Figure 6.14.3-1. The steps enclosed in dashed line boxes are executed as atomic procedures.

Figure 6.14.3-1: Initial Attach in satellite network for S&F operation

1-3.	When UE determines that it is in satellite coverage, the UE attempts to register with the network by sending an Initial Attach Request (without PDN connectivity) message to MME-SAT. If the UE is enhanced for S&F operation, the Attach Request message includes a S&F indication. MME-SAT stores UE's IMSI. Given that the satellite has no connection with the ground station, it rejects UE's request by sending the Attach Reject message. The MME-SAT stores the time (“Request Time”) at which message 3 was sent.

If the UE has indicated that it is enhanced for S&F operation, the Attach Reject message includes a Cause value indicating that the Attach procedure is suspended, as well as a Timer value, indicating how long the UE should refrain from attempting another Attach to this network. The Cause and the Timer in the Attach Reject message need to be protected (e.g. using a digital signature).

NOTE 1:	MME-SAT determines the Timer value based on the knowledge of the satellite constellation (e.g. which satellite will be the first one to fly over UE’s location after this satellite has established contact with the ground station).

NOTE 2:	The security mechanism for protection of the content in the Attach Reject message will be defined by SA3.

NOTE 2a:	After message 3 is sent, the UE will look for coverage (and might move) and might find another access network and then it will perform a Tracking Area Update / Registration Update with that (e.g., terrestrial) network. It is vital (e.g., for emergency communication from the UE) that message 4 and/or message 14, below, does not lead to the UE’s registration with that new network to be cancelled.

If the UE has not indicated that it is enhanced for S&F operation (i.e. a legacy UE), the Attach Reject message includes a Cause value indicating “Congestion”.

4-5.	When MME-SAT establishes contact with the ground station, it requests authentication data for the UE by sending the Authentication Data Request (IMSI, "Request Time") message to the HSS via MME-GND. The HSS checks that no other (e.g., terrestrial) MME has sent an Update Location Request after the "Request Time" and, if not, the HSS generates authentication vectors for the UE and sends them to the MME-GND) using the Authentication Data Response (EPS Auth. Vectors) message.

6-7.	MME-GND may immediately update the UE location with the HSS by sending the Update Location Request message in order to obtain any subscription-related service restrictions for the UE. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives the final Update Location Request (in step 13). The HSS sends UE's subscription data to MME-GND using the Update Location Ack message.

9.	When MME-GND establishes contact with the first satellite fly over the UE’s location, MME-GND forwards the authentication vectors and the subscription information to MME-SAT.

9-13.	When UE subsequently determines that it is in satellite coverage, and if the Timer provided in step 3 has expired, the UE enhanced for S&F re-attempts to register with the network by sending an Initial Attach Request (without PDN connectivity) message to MME-SAT. MME-SAT authenticates the UE using the stored authentication vectors and accepts the Attach Request. From UE's perspective the Initial Attach procedure is completed at this point.

NOTE 3:	The UE that is not enhanced for S&F operation (i.e. the legacy UE) is not able to handle the new Timer value and will try indiscriminately to Attach whenever it determines that it is in satellite coverage. It can therefore be rejected several times until it attempts to Attach to the satellite carrying the authentication vectors.

14-15.	When MME-SAT subsequently establishes contact with the ground station, it updates the current UE location with the HSS by sending the Update Location Request message (via MME-GND). The HSS sends UE's subscription data to MME-SAT using the Update Location Ack message.

NOTE 4:	If the MME-SAT has not initiated a provisional Location Update in step 6-7 and if the subscription data in the Update Location Ack message indicate that the UE is not allowed to be served in the location where the Attach was requested, the MME-SAT initiates UE detach during the subsequent contact with the UE.

### 6.14.4	Impacts on services, entities and interfaces

MME impact:

-	Ability for split operation with MME-SAT and MME-GND component on the satellite and on the ground, respectively.

-	Ability for differentiated handling of UEs: sending Accept Reject including protected Cause and Timer value to UE enhanced for S&F; sending Accept Reject with Cause = “Congestion” for legacy UE.

- 	addition of time stamp (“Request Time”) to Authentication Request sent to HSS.

-	(optional) Support for provisional location update indication in the Update Location Request.

HSS impact:

- 	HSS functionality to check for requests received after the “Request Time” from other access networks.

-	(optional) Support for provisional location update indication in the Update Location Request.

UE impact:

-	Inclusion of a “S&F” indication in Attach Request.

-	Handling of the new Cause and Timer values in Attach Reject.

## 6.15	Solution #15: S&F solution for multi-satellite IoT NTN

### 6.15.1	Description

The description of the solution principles and architecture assumptions for the proposed solution are conducted taking as a reference architecture the Optimized EPS architecture option for CIoT as specified in TS 23.401 [5]. The consideration of such architecture is mainly motivated for the sake of clarity in the presentation and does not intend to introduce any restriction or constraint to the general architectures defined in TS 23.401 [5] and TS 23.682 [7].

The Optimised EPS architecture option for CIoT is reproduced in Figure 6.15.1-1 and Figure 6.15.1-2 for, respectively, non-roaming and roaming scenarios.

Figure 6.15.1-1: Optimised EPS architecture option for CIoT - Non-roaming architecture (Source: Figure L.2-1 in TS 23.401 [5])

Figure 6.15.1-2: Optimised EPS architecture option for CIoT - Roaming architecture (Source: Figure L.3-1 in TS 23.401 [5])

In the Optimised EPS architecture option for CIoT, the C-SGN (CIoT Serving Gateway Node) is a combined node EPC implementation option that includes the three main EPC entities, i.e. MME, S-GW and P-GW, with no limitation in terms of functionality. Accordingly, the external interfaces of the C-SGN node are those of the respective combined EPC entities towards other entities.

Based on the representation of the Optimized EPS architecture option for CIoT, Figure 6.15.1-3 shows a potential distribution/split of the core network (CN) functions between the satellite and the ground. Such distribution/split is based on the following principles:

-	The set of CN functions on board the satellite can be a subset of the C-SGN functions, with no need for 3GPP to specify the specific split and interfaces between the subset of the C-SGN functions on board the satellite (referred to as C-SGN-SAT in Figure 6.15.1-3) and the remaining subset of the C-SGN functions deployed on ground (referred to as C-SGN-GND in Figure 6.15.1-3). Accordingly, the interface between C-SGN-SAT and C-SGN-GND, referred to as S&F interface in Figure 6.15.1-3, could remain implementation dependent.

-	The minimum subset of CN functions that should necessarily form part of the C-SGN-SAT would be the MME functions needed to (1) handle the S1 interface with the onboard eNB and (2) the MME functions needed to terminate NAS protocol signalling from/to UEs via the onboard eNB.

Under such architectural approach, standardization efforts could be focused on the impact that the support of S&F Satellite operation could have, if any, on existing interfaces, i.e.: Uu, NAS protocols, S1, S8/SGi, T6a/T7/T8, S6a and SGd.

Figure 6.15.1-3: Proposed distribution of CN functions between the satellite and the ground for S&F Satellite operation, considering the Optimized EPS architecture option for CIoT as a baseline

Moreover, as discussed in SA2#159, it is important that the S&F Satellite operation mode can properly support multi-satellite deployment scenarios by allowing a UE to be potentially served by multiple satellites, i.e. not restricting the S&F operation for a UE to a single satellite within the constellation. Multi-satellite support is indeed considered critical to be able to reduce the latencies in data transfer delivery, given that the revisit time of a single satellite may be around 12h or longer for typical orbits (see R2-2107453).

In this respect, the same principles previously established for the distribution/split between the C-SGN-SAT and C-SGN-GND can be directly extended to the multi-satellite case, as represented in Fig. 6.15.1-4. Consequently, specifics of the multi-satellite scenario, such as how to handle UE/MME context synchronisation between multiple C-SGN-SAT entities and other challenges, can be kept implementation dependent, given that multiple and diverging solutions may exist depending on particular constellation and network configurations.

Figure 6.15.1-4: Proposed distribution of CN functions between the satellites and the ground for S&F Satellite operation, considering the Optimized EPS architecture option for CIoT as a baseline, in a multi-satellite deployment scenario

In light of the above considerations, the proposed solution for S&F Satellite operation support relies on the following principles and architecture assumptions:

1)	The split of CN functions between the satellite and ground is achieved by means of splitting the C-SGN functions between a C-SGN-SAT and a C-SGN-GND, so that termination end-points for S1 and NAS protocols remain on the satellite as part of the CN-SGN-SAT and termination end-points for SGd, S6a, S8/SGi and T6a/T6ai remain on the ground as part of the CN-SGN-GND.

2)	The split between C-SGN-SAT and C-SGN-GND is also applicable to a multi-satellite scenario, where a single instance of C-SGN-GND on ground could interact with multiple instances of C-SGN-SAT on board the satellites.

3)	The interface between the C-SGN-SAT and C-SGN-GND entities could be left to vendor implementation, given that multiple and diverging solutions may exist depending on particular constellation and network configurations.

4)	Within this architecture approach, standardization efforts could be focused on the impact that the support of S&F Satellite operation could have, if any, on existing interfaces, i.e.: Uu, NAS protocols, S1, S8/SGi, T6a/T7/T8, S6a and SGd.

### 6.15.2	Procedures

Based on the architecture depicted in Figure 6.15.1-4 and giving special relevance to the following architecture assumption "Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused", this section describes high-level procedures and information flows for:

-	The EPS to inform UEs if  the network is operating in S&F Satellite operation mode or not.

-	Carrying out  system procedures between the UE and EPS that may not be completed during a single satellite pass due to S&F Satellite operation but can be realized over several satellite passes (e.g. such as the initial attach procedure in a roaming scenario).

-	User data transfer in S&F Satellite operation mode.

#### 6.15.2.1	Advertising capabilities and activation of S&F Satellite operation

A E-UTRAN cell with satellite access for Cellular IoT (CIoT) should be able to inform a UE of:

-	Whether the satellite cell is being operated in S&F Satellite operation mode or in normal/default mode.

-	Whether there is any service restriction/limitation, or alternatively which services/options are available, while the cell is in S&F Satellite operation mode (e.g. service availability could be restricted to support of only CP CIoT EPS Optimization services during S&F Satellite operation mode)

The advertisement of such information shall prevent UEs from requesting service access or triggering some procedures that might not be supported during the S&F Satellite operation mode. Moreover, the awareness of whether a satellite cell is operated in S&F or normal/default mode could also be considered by the UE for:

-	Selection of the proper application layer profile/behaviour (e.g. preventing the usage of IoT applications using TCP as transport layer during S&F operation mode and instead relying on applications that use UDP transport, NIDD or SMS).

-	Network selection (e.g. a UE may choose not to select a given PLMN operated in S&F mode if other PLMNs operated in normal mode are available).

It is also important to consider that a satellite cell may be switching between S&F Satellite operation mode and normal/default mode (i.e. a given satellite may not be operating all the time under a single mode). For instance, at geographical areas where the satellite cannot be connected to ground network via a feeder link, the satellite can use S&F Satellite operation mode to give minimum service. However, in areas that the same satellite can be connected to the ground network simultaneously, the satellite cell operation may remain in S&F Satellite operation mode or switch to normal mode, subject to network operator's policies.

As a potential solution approach, the following options can be considered for informing UEs of whether S&F Satellite operation is available/activated or not in a satellite cell:

-	Option#1. S&F Satellite operation is advertised via system information broadcasting (SIB).

-	Option#2. S&F Satellite operation is advertised via dedicated NAS messaging.

Whether only one or both options are necessary to provide complementary information should be further assessed (e.g. SIB can be used to signal only whether the cell is operated in S&F mode or not, while and NAS messaging can be used to provide further information on service limitation and/or supported S&F capabilities).

#### 6.15.2.2	NAS procedures spanning several satellite passes

In a satellite cell operating in S&F Satellite operation mode, if a UE initiates a signalling procedure during a satellite pass (e.g. E-UTRAN Initial Attach procedure) and the procedure cannot be completed within the duration of the pass because some information is missing in the serving satellite (e.g. E-UTRAN Authentication Vectors, AVs, are not available within the satellite and should be retrieved from an HSS on the ground), the initiated procedure should be terminated/suspended with a proper rejection cause and an indication about when/how the UE may re-attempt/resume the procedure (e.g. the initial attach procedure may be rejected with a cause like "procedure suspended due to S&F operation" and an indication such as "re-try in 'X' minutes" could be given for the UE to retry during the next pass of the same or a different satellite where E-UTRAN AVs could have been already uploaded into the satellite to be able to complete the procedure).

Moreover, in a multi-satellite constellation, it should be possible to initiate/suspend a procedure in a given satellite and resume/terminate it in another satellite of the same PLMN in order to benefit from reduced latencies (i.e. the UE does not have to wait for a pass of the same satellite to be able to resume/terminate the procedure).

In this context, extensions/components that might be considered in a potential solution are:

-	Means for the UE to indicate to the network that it supports S&F (e.g. a UE network capability flag to signal that S&F is supported on the UE side).

-	New reject causes and (backoff) timers for terminating/suspending the NAS procedures and assisting the UE on subsequent re-attempts in the same or other satellite cells of the same PLMN.

-	Means for the network to communicate to the UE in which satellites user data transfer is allowed after a valid network registration (e.g. list of satellite IDs for which the registration is valid).

An illustration of how this sort of extensions/components can be used is given in Figure 6.15.2.2-1 for the case of a E-UTRAN initial attach procedure. Without loss of generality, the description of the procedure considers the involvement of three different satellites (SAT#i, SAT#j, SAT#k) to highlight the facts that (1) the execution of the attach procedure can be initiated in one satellite and completed in another and that, after the attach completion, (2) user data transmission should be possible via any of the three satellites. Moreover, in this example, it is assumed that the HSS with the subscription information (including the E-UTRAN AVs needed for completing the attach procedure) is on the ground, as it could be the case of a roaming scenario.

The procedure described in Figure 6.15.2.2-1 assumes the following sequence of contacts (i.e. satellite passes) between the UE, satellites and ground:

Table 6.15.2.2-1

| Contacts | UE | SAT#i | SAT#j | SAT#k | GROUND |
| --- | --- | --- | --- | --- | --- |
| Contact#1 | X | X |  |  |  |
| Contact#2 | X |  | X |  |  |
| Contact#3 |  | X |  |  | X |
| Contact#4 |  |  |  | X | X |
| Contact#5 | X |  |  | X |  |
| Contact#6 |  |  |  | X | X |
| Contact#7 |  |  | X |  | X |
| Contact#8 | X | X |  |  |  |
| Contact#9 | X |  | X |  |  |

Figure 6.15.2.2-1: Illustration of E-UTRAN Initial Attach procedure under S&F Satellite operation in a multi-satellite deployment and roaming scenario (i.e. HSS on ground)

0.	As a starting point it is assumed that the UE is switched on and needs to register with the network. It is assumed that the (terrestrial) home PLMN of the UE is not found and the UE starts the search for a cell of a satellite PLMN.

1.	SAT#i comes within the communication range of the UE. The satellite cell is detected by the UE. The UE acquires, among others, the PLMN and the TLEs of multiple satellites of the constellation (SIB32). The UE also learns from SIB31 or SIB32 that the satellite cell is operating in S&F Satellite mode.

2.	The UE triggers the initial attach procedure. Within the Attach request, the UE network capabilities incorporate an 'S&F feature supported' flag, signalling the device's ability to handle S&F operations.

3.	SAT#i rejects the initial attach procedure due to the absence of subscription data on board the satellite, such as E-UTRAN Authentication Vector (AV), and the lack of a currently active feeder link to the ground network for subscription data retrieval during this satellite pass. If the UE has signalled support for S&F (e.g. Rel-19 UE), the SAT#i issues an ATTACH REJECT message with:

-	A new reject cause (S&F rejection cause in Figure 6.15.2.2-1) to indicate that the attach procedure needs to be suspended.

-	A timer (backoff) value (S&F Timer#1 in Figure 6.15.2.2-1) to indicate to the UE when the UE can re-attempt the initial attach procedure. During the duration of this timer value, the UE should not to initiate a new attach attempt with any satellite cell operating in S&F mode within the same PLMN.

NOTE 1:	This timer may not prevent the UE to re-attempt the initial attach in a satellite cell of the same PLMN not operating in S&F mode.

NOTE 2:	A list of potential satellite cells or satellite identifiers may also be provided along with the attach response message to restrict the resumption of the procedure to a subset of the satellites of the constellation.

If the UE has not signalled that it supports S&F (e.g. case of a pre-Rel-19 UE), another reject cause can be used (e.g. congestion) to force the UE to retry in a next satellite pass. This option is up to network implementation.

Internally in the SAT#i, the initial attach attempt of the UE is recorded/saved so that SAT#i will try to retrieve the missing information to complete the attach as soon as the connectivity with the ground network is available.

4.	SAT#j comes within the communication range of the UE. Like SAT#i in step 03, SAT#j is assumed to be in S&F operation mode and have no subscription information on board either (e.g. E-UTRAN AVs) to be able to resume/complete the attach registration with the UE. The satellite cell is detected. The UE identifies that this satellite cell belongs to the same PLMN as the cell where it previously attempted the initial attach (step 03). The UE learns that SAT#j satellite cell is operating in S&F mode. In this situation, the UE does not trigger an attach attempt because SAT#j operates in S&F mode and the timer value (S&F Timer#1) has not yet expired.

4a.	"S&F Timer#1" expires on the UE. From this point on, the UE knows that the initial attach procedure can be re-attempted in any upcoming satellite cell of the PLMN.

5.	SAT#i gets connected to the ground network via the feeder link.

6.	C-SGN-SAT of SAT#i and C-SGN-GND exchange information on the initial attach attempt detected by SAT#i in step 03.

7.	C-SGN-GND interacts over S6a interface with the HSS of the home PLMN of the UE requesting the initial attach. The IMSI of the UE is used to execute the Authentication Information Retrieval (AIR) procedure. At this point, the C-SGN-GND could also trigger an Update Location Request (ULR) procedure to pre-fetch the necessary subscription data to be able to complete the attach procedure in a next satellite pass.

C-SGN-GND obtains the subscriber information from the HSS, including the E-UTRAN AVs.

8.	E-UTRAN AVs are uploaded to C-SGN-SAT in SAT#i.

9.	SAT#k gets connected to the ground network via the feeder link. Assumption is that SAT#k is expected to fly over the area where the UE that attempted the attach is located. E-UTRAN AVs for that subscriber/UE are also uploaded to the C-SGN-SAT of SAT#k.

10.	SAT#k comes within the communication range of the UE.

11.	The satellite cell is detected by the UE. The UE re-tries the initial attach procedure in SAT#k given that the "S&F Timer#1" value is already expired.

12.	Authentication and security setup steps can be now completed successfully, given that SAT#k has the necessary information about the UE/IMSI.

13.	Network attach is accepted. A new UE/MME context is created. As part of the NAS attach accept, a timer value ("S&F Timer#2" in Figure 6.15.2.2-1) is provided to the UE to indicate a proper time when the UE can attempt user data transmission/reception in any satellite. This timer value is needed to prevent the UE from trying to start data transfer in a next satellite before the network has been able to complete the update of the HSS (location update procedure) on the ground and the new established UE/MME context has been propagated/synchronised in other satellites of the constellation (FFS if the service gap control could be reused for this same purpose).

14.	The UE sends the NAS attach complete and sets the timer.

15.	SAT#k gets connected to the ground network via the feeder link. The C-SGN-GND learns about the new UE/MME context created in SAT#k.

16. C-SGN-GND triggers a location procedure over S6a with the HSS of the home PLMN of the UE that completed the attach so that the HSS gets updated about the registration. With the registration finalized, the system has successfully retrieved all subscriber-related information from the HSS of the home PLMN.

17.	SAT#j gets connected to the ground network via the feeder link. The new UE/MME context is uploaded into SAT#j.

NOTE 3:	If pre-fetching of subscription data was performed in previous Step 7c/d and the subscriber-related information retrieved from the HSS in Step 16 indicates that the UE is not allowed to be served in the location where the Attach was requested, the C-SGN-SAT on SAT#j initiates UE detach during the subsequent contact with the UE.

18.	SAT#i comes within the communication range of the UE. The UE may detect the satellite cell. However, the UE does not trigger any user data transmission because the timer value (S&F Timer#2) provided in step 13 has not yet expired.

18a.	S&F Timer#2 expires on UE. From this point on, the UE assumes that the registration is valid in the network (e.g. the network has already managed to distribute the newly created UE/MME context to the several satellites) and that user data transmission/reception can be attempted in any upcoming satellite cell of the PLMN.

19.	SAT#j comes within the communication range of the UE.

20.	User data transmission can be performed in SAT#j

#### 6.15.2.3	Data transfer in S&F Satellite operation mode

Following successful registration with the satellite PLMN and establishment of a valid UE/MME context across all or a subset of the satellites of the constellation, data transfer can take place between the UE and any of those satellites when in reach of the UE.

Without preventing other approaches, this scenario considers the use of Control Plane CIoT EPS Optimisation as the baseline protocol for data transfer between the UE and satellite cells operating in S&F Satellite operation mode.

Data transfer under Control Plane CIoT EPS Optimisation relies on an exchange of ESM DATA TRANSPORT messages between the UE and the MME, encapsulating user data within the User Data Container Information Element (IE), as defined in TS 24.301 [17]. Furthermore, the protocol allows for the exchange of ESM STATUS messages between the UE and MME to report specific error conditions upon the reception of ESM protocol data.

On this basis, a solution approach to enable data transfer in S&F mode involves augmenting the existing Control Plane CIoT EPS Optimisation with:

new IEs in the ESM DATA TRANSPORT messages and/or new causes in the ESM STATUS messages to handle S&F data transfer aspects such as S&F data quotas, delivery priority levels and providing information on expected delivery times (some further details on these aspects are given below as part of the description of the steps in Fig. 6.15.2.3-1.

From a service perspective, such a data transfer mechanism could be conceived as an extension of the "Control Plane CIoT EPS Optimisation" for when the satellite cell is in S&F mode, which could be referred to as "Control Plane CIoT EPS Optimisation with S&F extensions". Alternatively, a new data transfer service associated with S&F Satellite operation could be defined, potentially termed "S&F CP data transfer".

An illustration of the data transfer mechanism is depicted in Figure 6.15.2.3-1 for the Mobile Originated (MO) case. For the Mobile Terminated (MT) case, the same approach applies, with the only difference being the trigger of the paging procedure by the CN-SGN-SAT when the UE is expected to be in reach from the flying satellite.

Figure 6.15.2.3-1: MO user data transfer

0.	As a starting point it is assumed that:

-	The UE is already registered in the satellite network.

-	The UE has previously acquired long-term ephemeris (TLEs) during past contacts with satellites (from SIB32).

-	New data originating at the application layer is ready to be transferred.

-	The UE is capable of estimating the timing of its next contact with a forthcoming satellite (SAT#i) that has a valid UE/MME context to interact with the UE.

1.	SAT#i comes within the communication range of the UE. The UE detects the satellite cell and verifies that S&F Satellite mode is supported. The UE triggers the "Control Plane CIoT EPS Optimisation with S&F extensions" (or "S&F CP data transfer") procedure:

-	The UE either establishes an RRC connection or sends an RRCEarlyDataRequest message, as defined in TS 36.300 [13], which includes an integrity-protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data.

-	The UE may also indicate in a NAS Release Assistance Information in the NAS PDU whether no further Uplink or Downlink Data transmissions are expected, or only a single Downlink data transmission (e.g. Acknowledgement or response to Uplink data) subsequent to this Uplink Data transmission is expected.

-	The UE may also include in the NAS PDU any needed S&F IEs (e.g. a new IE to ask for data storage allowance)

2a.	The eNodeB, based on configuration, may retrieve the EPS negotiated QoS profile from the MME, if not previously retrieved. The eNodeB may retrieve additional parameters (e.g. UE Radio Capabilities).

3.	The NAS PDU sent in step 02 is relayed to the MME by the eNodeB using a S1-AP Initial UE message. If the RRCEarlyDataRequest message was received in step 02, the eNodeB includes the "EDT Session" indication in the S1-AP Initial UE message.

4.	The C-GCN-SAT checks the integrity of the incoming NAS PDU, decrypts the encapsulated data and stores it.

5.	Additional user data may be exchanged using the ESM data transport mechanism as defined in TS 24.301 [17]. Data is stored in the satellite. The ESM data transport messages now may include specific S&F Information Elements (IEs) to manage S&F data quotas (e.g. notification on data storage allowance or status messages forcing the release of the data transfer procedure when the quota is reached) or inform about expected S&F delivery time. For instance, similarly to how the Release Assistance Information (RAI) IE in ESM data transfer is used to inform the network on further uplink and downlink data transmission expectations, a new IE or an extension of the RAI IE could be defined to handle S&F data transfer and used to e.g. provide information to the UE on whether further data transmission is allowed or quota limitations have been reached. A new IE could also be defined to indicate the priority of S&F data, out of a few priority levels (e.g. low, medium, high) and another IE could be defined for the network to inform, possible upon demand, of the expected delivery time for the data being buffered (e.g. time left for the data to be delivered to the ground network).

6.	To complete the data transfer, RRC connection and S1 release is triggered. Alternatively, RRC Early Data Complete is sent if the data transfer procedure was initiated using an RRCEarlyDataRequest message.

7.	SAT#i gets connected to the ground network via the feeder link. The satellite forwards the stored user data to the C-SGN-GND.

8.	Downloaded user data is forwarded to the destination via proper interfaces.

The way to enforce retention/validity periods (e.g. discard the data after retention/validity period expiration) for the data buffered within the C-SGN-SAT can be left to implementation or further considered during the normative phase.

### 6.15.3	Impacts to Services, Entities and Interfaces

No impacts foreseen in terms of defining new entities or interfaces for the EPS architecture.

However, the following extensions would be needed to support S&F Satellite operation in a multi-satellite constellation scenario:

1)	For the network to advertise the support of S&F capabilities and/or activation of S&F satellite operation mode:

a)	New IEs in SIB (e.g. SIB31 or SIB32); and/or

b)	New IEs in NAS messaging.

2)	For the UE to advertise the support of S&F capabilities:

a)	A UE network capability flag indicating S&F support.

3)	For NAS procedures spanning several satellite passes and potentially different satellites:

a)	New reject causes and backoff timers for terminating/suspending NAS procedures and assisting the UE on how to proceed with subsequent re-attempts in future passes of the same or different satellite cells.

b)	New IEs for the network to inform the UE of the satellites in which user data transfer is allowed after completing network registration (e.g. list of satellite identifiers for which the registration is valid).

4)	For user data transfer, the Control Plane CIoT EPS Optimisation data transfer procedure could be extended with:

new IEs in the ESM DATA TRANSPORT messages and/or new causes in the ESM STATUS messages to handle S&F data transfer aspects such as S&F data quotas, delivery priority levels and providing information on expected delivery times.

5)	In addition, the following extensions could be also needed for a end-to-end solution:

a)	Subscription parameters related to S&F satellite operation in the HSS and option to pre-fetch the necessary data for S&F operation before UE authentication.

b)	Awareness of the S&F satellite operation to the SCS/AS entities via T8 procedures.

## 6.16	Solution #16: Enabling S&F operation with C-SGN onboard

### 6.16.1	Description

This is a candidate solution for Key Issue 2 - "Support of Store and Forward Satellite operation".

To facilitate Store-and-Forward (S&F) operations for delay-tolerant services, the assumption is made to activate the entire C-SGN on board. The architecture is derived from Annex L, specifically the "Optimised EPS Architecture option for CIoT" outlined in TS 23.401 [5]. In this architecture, the C-SGN incorporates the functionalities of MME, S-GW, and P-GW. Furthermore, it supports all the requisite interfaces to accommodate various types of delay-tolerant services.

Figure 6.16.1-1: The architecture and interfaces of C-SGN onboard

As shown in Figure 6.16.1-1, the C-SGN onboard architecture is assumed that:

-	A UE is served by the same satellite. all the time (i.e., the UE context is kept in one SAT(MME) without changing).

-	the onboarding C-SGN functionalities are only used for S&F operation, i.e. any UE that connects to the satellite without S&F operation cannot be served by the onboarding C-SGN.

-	S-GW is responsible for storing/forwarding the data (or non-IP data) for CP CIoT EPS optimizations, UP CIoT EPS optimizations.

-	MME is responsible for storing/forwarding the data for SMS services.

-	ISL is not used to transfer the contexts among satellites.

Therefore, to finalize the registration of a UE that supports S&F operations within the network, multiple orbital cycles are required. The number of cycles is determined by the involvement of various procedures, encompassing the UE, onboarding functionalities (i.e. RAN, MME, S-GW, P-GW), and other CN NFs on the ground such as HSS, PCRF, AF, etc. This is due to the service link serves:

-	AS layer management between UE and RAN.

-	NAS layer management between UE and MME.

And feeder link serves:

-	CN internal functionalities management between C-SGN onboard and CN NFs on the ground, i.e. MME/P-GW and HSS, P-GW and HSS/PCRF, P-GW and DN.

This architecture needs enhancements on:

-	Availability detection of service link/feeder link.

-	S&F operation triggered condition detection.

-	Signalling procedure development on pause/recover mechanisms regarding service link's availability, NAS suspend/resume.

-	New procedure development on store/forward mechanism regarding UL/DL data transmission.

In this solution, it is proposed to use a mechanism about NAS suspend and resume to adapt the S&F operations for NAS message handling. When the feeder link is not available, the MME onboard will pause UE NAS message and notify the UE that its related NAS procedure is suspended (e.g., the attach request), the UE will correspondingly keep the NAS related context and waits for the network NAS resume notification to recover the NAS procedure. When the MME finished the interactions with other functions (e.g, the HSS) and the service is available, the MME will notify the UE to recover the NAS procedure and receive the DL NAS message.

### 6.16.2	Procedures

#### 6.16.2.1	Procedures of NAS Pause/Recovery of S&F Operation

This procedure can be applied to any NAS interactions between UE and MME as listed in TS 23.401 [5]. An example of how to utilize NAS Pause/Recovery to complete the initial attach procedure is given in clause 6.16.2.4, and how to utilize NAS pause/recovery to complete the MO/MT data transmission is given in clause 6.16.2.3.

Figure 6.16.2.1-1

0.	Pre-condition: UE has established RRC connection with RAN, and RAN has established S1 interface with MME

1.	MME decides to pause NAS.

The trigger condition is when MME receives S1 messages from RAN/NAS message from UE and the procedure is to detect the necessaries to interact with CN NFs on the ground. For example, when UE sends an Attach Request, MME will interact with HSS to initiate the authentication and authorization procedures, as HSS is on the ground, the procedure can proceed only when the feeder link is available.

2.	MME sends NAS Pause request to the UE including the following parameters:

-	UE identifier, e.g. GUTI if it is assigned by MME, or IMSI that is reported by the UE;

NOTE: 	The scenario when IMSI is as UE identifier can be used for the initial UE, i.e. when UE sends attach request and MME has not yet received authentication/authorization information from HSS.

-	a NAS pause timer/availability window that is calculated by the MME to let UE be aware of when the same satellite will come back to the coverage for the UE.

The NAS pause timer/availability window is used when the UE detects a necessity to send a message to the network, e.g. when UE wants to sends a service request to be back the ECM-CONNECTED state, or when UE wants to send a MO data (e.g. CP CIoT, UP CIoT, SMS, non-IP etc).

NOTE:	Which service type (e.g. CP CIoT, UP CIoT, SMS, non-IP etc) to be adopted for UE for S&F operation is determined during interaction with network and sent to the UE(e.g., during the attach procedure or PDN connectivity procedure).

3.	RAN may Pause AS if an indication is provided by MME in step 2 to save UE energy consumption. The AS pause can be established with the current RAN mechanism, e.g. RAN sends RRCRelease message to UE and moves UE to RRC_IDLE mode.

NOTE:	whether the AS pause/recovery procedure can be proceeded via new RAN mechanism or not depends on RAN.

4.	UE starts to pause NAS during the procedure even though the service link is still available based on the request as received in step 2. UE will start a NAS pause timer accordingly.

Pause NAS means any concurrent NAS messages should be stopped until receive a Recover Request from MME.

5.	UE sends a Pause NAS complete message to MME.

6.	When the RAN orbits back to serve the UE, the RAN and UE recover the AS connection, the resumption can be triggered by UE (more power saving), or by RAN. When the AS connection is back, the RAN notifies AMF that the UE is back. The AS recovery can be established with the current RAN mechanism, e.g. RAN sends RRCSetup message.

7.	MME decides to recover NAS, i.e. continue the paused NAS procedure as specified in step 2 based on e.g. after receiving the notification from RAN, or the NAS pause timer is expired, or a request sent by the UE, or MME receives enough message via the interactions with the NFs on-ground.

For example, e.g. in initial attach procedure, when MME receives the authentication request from HSS and the service link is available again, the MME will send a notification to the UE to indicate UE the concurrent NAS signaling can be continued.

Or when MME receives a service request from UE that the UE has a necessary to be ECM-CONNECTED to sends a message.

8.	MME sends NAS Recovery request to UE at least the following parameter to inform MME:

-	UE identifier, e.g. with GUTI if it is assigned by the MME, or IMSI.

NOTE: 	The scenario when IMSI is as UE identifier can be used for the initial UE, i.e. when UE sends attach request and MME has not yet received authentication/authorization information from HSS.

9.	UE sends back a Recovery NAS complete message to MME. To make full use of the service link available period, the MME/UE may include NAS messages within the recovery messages.

Afterward, normal NAS messages can be exchanged between UE and MME.

#### 6.16.2.2 	Procedures of Store/Forward of SMS transmission

The procedures are used for SMS service.

NOTE: 	this procedure can also be used for CP CIoT optimizations.

##### 6.16.2.2.1	Procedure of MO SMS transmission

Pre-condition: UE is registered to the S&F network. A NAS pause timer was assigned by the MME when the service link was about to be unavailable as defined in clause 6.16.2.1. UE has a NAS pause timer running.

1.	UE has a SMS message to be sent. When the service link is available, and the UE’s NAS pause timer is expired, UE will initiate the NAS recovery procedure by establishing step 6-9 as defined in clause 6.16.2.1. Briefly, UE firstly restore the RRC connection with the RAN, RAN restores the S1 connection with the MME (both onboard), UE subsequently sends Service request message to the MME. When MME receives the SR message, MME will send the NAS recovery request to the UE. Afterwords, UE is back to be in ECM-CONNECTED state, and can send the MO SMS message to the MME.

2. 	MME stores the MO SMS message.

3. MME stops receiving the MO SMS message. The trigger condition could be when service link is about to be unavailable and/or the storage quota for the UE is exceeded.

4. MME sends NAS resume procedure to the UE, as defined instep 1-5 in clause 6.16.2.1.

No service period.

5. When the feeder link is available, the MME sends the stored MO SMS to the SMS-GMSC on-ground.

6. The SMS-GMSC replies with a delivery report to the MME.

7. When the service link is available, the MME performs paging procedure to get UE connection and then sends the delivery report to UE.

NOTE: This solution is not applied to legacy UE (i.e., not support S&F operation).

##### 6.16.2.2.2	Procedure of MT SMS transmission

Pre-condition: SMS-GMSC is assumed to be informed the S&F operation and is aware of the availability information of onboard P-GW/S-GW/MME.

1.	SMS-GMSC receives a SMS message for a UE, SMS-GMSC checks the availability of MME onboard based on the fetched the routing information from HSS. When the availability information indicates the feeder link to the P-GW/S-GW/MME is available, SMS-GMSC sends the MT SMS to the MME.

2.	MME stores the MT SMS

3. MME stops receiving the MT SMS. The trigger condition would be when the feeder link is about to be unavailable and/or the storage quota for the UE is exceeded.

No service period.

4.	When feeder link is available, MME will initiate the NAS recovery procedure by establishing step 6-9 as defined in clause 6.16.2.1. Briefly, UE firstly restore the RRC connection with the RAN, RAN restores the S1 connection with the MME (both onboard), MME is notified and subsequently sends NAS recovery request to the UE. Afterwords, UE is back to be in ECM-CONNECTED state, and MME can send the MT SMS message to the MME. MT SMS message can also be integrated in the NAS recovery request message.

5	When the feeder link is available, the MME sends the delivery report to the SC / SMS-GMSC.

#### 6.16.2.3	Procedure of Store/Forward of Data transmission

This procedure applies to CP CIoT EPS Optimizations, UP CIoT EPS Optimizations, non-IP data transmissions.

##### 6.16.2.3.1	Procedure of MO Data transmission

Figure 6.16.2.3.1-1

Pre-condition: same precondition as defined in clause 6.15.2.2.1.

1.	UE sends UL data via RAN and S-GW with NAS recovery firstly, similar procedure as defined in clause 6.16.2.2.1 step 1.

When the delay-tolerant service is CP CIoT optimizations, the path is: UERANMMES-GW;

When the delay-tolerant service is UP CIoT optimizaitions or non-IP data, the path is: UERANS-GW

2.	S-GW stores the UL data

3.	S-GW stops receiving the UL data when the service link is unavailable or when the storage quota for the UE is exceeded.

4.	S-GW detects feeder link is available. The availability detection of feeder link can be based on pre-configured information, MME notifications, or gateway notifications (out of 3GPP scope). At the same time the connection between CN C-SGN onboard and CN NFs on the ground is recovered.

5.	S-GW forwards the stored data to P-GW, and P-GW forwards to the ground.

##### 6.16.2.3.2	Procedure of MT data transmission

Figure 6.16.2.3.2-1

Pre-condition: CIoT service server is assumed to be informed the S&F operation and is aware of the availability information of onboard P-GW/S-GW/MME.

1.	P-GW and S-GW receives DL data. CIoT service center receives MT data for a UE, CIoT service center checks the availability of P-GW/S-GW/MME onboard. When the availability information indicates the feeder link to the P-GW/S-GW/MME is available, CIoT service server sends the MT SMS to the P-GW.

When the delay-tolerant service is CP CIoT optimizations, the path is: CIoT service server P-GWS-GW;

When the delay-tolerant service is UP CIoT optimizaitions or non-IP data, the path is: CIoT service server SCEFP-GWS-GW.

2.	S-GW stores the DL data

3.	S-GW detects stops receiving the DL data when the feeder link is not available or when the storage quota for the UE is exceeded.

4.	S-GW detects service link is available. The availability detection of service link can be based on pre-configured information, MME notifications. The NAS is recovered between UE and MME, similar to what has been defined in step 4b in clause 6.16.2.2.2.

5.	S-GW forwards the stored data to RAN, and RAN forwards to the UE.

#### 6.16.2.4	Example of initial attach procedure

This provides an example of how the UE can complete the attach procedure with C-SGN onboard. Generally, if the UE hasn't established any connection with the network before, it needs at least 3 rounds of satellite orbiting periods, each round of orbiting UE and network can complete some parts of the procedure.

##### 6.16.2.4.1	1st round orbiting

Figure 6.16.2.4.1-1

1.	When service link is available, UE initiates the RRC connection with RAN and sends Attach Request to MME.

2.	MME pause the NAS because MME needs to wait for the response from HSS . As MME has not completed the interactions with the HSS (on ground), MME cannot assign a GUTI to the UE yet. Therefore, when MME decides to pause the NAS, the MME will utilize IMSI as the UE identifier to inform UE the NAS pause decision, as specified in clause 6.12.2.1.

3.	When feeder link is available, MME initiates the interactions with HSS for the authentication and authorization.

4.	MME waits for the Auth response from UE and assigns a GUTI to the UE.

##### 6.16.2.4.2	2nd round orbiting

Figure 6.16.2.4.2-1

1.	When service link is available, MME recovers the interactions with UE to complete the authentication and authorization procedure. The NAS recovery procedure will be executed as described in clause 6.16.2.1.

2.	MME pause the NAS because MME needs to wait for the response from HSS, S-GW-P-GW and etc. . As MME has assigned a GUTI to the UE, the MME will utilize IMSI as the UE identifier to inform UE the NAS pause decision, as specified in clause 6.12.2.1.

3.	When feeder link is available, MME recovers the interactions with CN NFs on the ground to complete e.g. UE context setup, PDN session establishment, optionally dynamic PCC rules retrieval, IP address registration in HSS.

4.	MME waits for the attach response from UE.

##### 6.16.2.4.3	3rd round orbiting

Figure 6.16.2.4.3-1

1.	When the service link is available, MME recovers the interactions with UE to complete the attach procedure. The NAS recovery procedure will be executed as described in clause 6.16.2.1. Afterwards, UE can send UL data.

### 6.16.3	Impacts to Services, Entities and Interfaces

MME:

-	Detection of service link/feeder link availability.

-	Initiate NAS pause/recover procedure when CN interactions with CN NFs on the ground is needed.

-	Pause/recover NAS based on service link's availability.

-	Store/Forward SMS data

RAN:

-	Optionally, initiate AS pause procedure if NAS is paused, and initiate AS recovery procedure.

UE:

-	Pause/recover NAS.

-	Optionally, Pause/recover AS.

S-GW:

-	Detection of service link/feeder link availability.

-	Store/Forward UL/DL data for CP CIoT optimizations, UP CIoT optimization, and Non-IP services.

## 6.17	Solution #17 for KI#2: Support of Store and Forward Satellite operation for small data transmission

### 6.17.1	Description

#### 6.17.1.1	General

This solution resolves KI#2 to support Store and Forward Satellite operation for small data transmission.

Since the UE-satellite-ground network connectivity is intermittent for the Store and Forward Satellite Operation, the UE's initial attach cannot be completed at one time when the UE is in the coverage of the satellite, if the satellite does not support authentication and security procedures towards the UE, i.e. several rounds of interactions between the UE and satellite and between the satellite and the ground network are required in order to perform the authentication and security procedures as specified in clause 5.3.10 of TS 23.401 [5]. However, for reason of the information security and the complexity of the network operation, it's infeasible to store and maintain all the subscription information on the satellite. To speed up the Attach procedures, it is proposed to store the part of subscription information in UE individually and the on-board core network elements can perform some of the authentication and security procedures to complete the initial attach procedure once when the UE is in the coverage of the satellite.

During CIoT MO data transmission procedure, the data is contained in the NAS message (i.e. Service Request message) which is forwarded transparently by the eNB on board the satellite towards the satellite MME. As specified in clause 5.3.4B.1 of TS 23.401 [5], the data transport in Control Plane CIoT EPS Optimisation is accomplished by using the NAS transport capabilities of RRC and S1-AP protocols and the data transport of GTP-u tunnels between MME and S-GW and between S-GW and P-GW. Therefore, a S-GW deployed on the satellite is proposed to support store and forward the data on the satellite.

For Non-IP connection, the data transport in Control Plane CIoT EPS Optimisation is provided by via the MME with the SCEF. As specified in clause 4.5.14 of TS 23.682 [7], the SCEF may perform buffering of MO and/or MT Non-IP data, and neither the MME/SGSN nor the IWK-SCEF are expected to buffer data pertinent to PDN connection to the SCEF. So, a SCEF deployed on the satellite is proposed to buffer the data to support the Non-IP data delivery (NIDD).

NOTE:	The detailed procedure for the authentication and security procedures performed between the UE and the on-board network elements is be defined by SA WG3.

Editor's note:	This solution only supports to access one satellite.

#### 6.17.1.2	Reference Architecture

In order to support Store and Forward Satellite operation for small data transmission, the following figure shows the high level reference architecture.

Figure 6.17.1.2: Reference Architecture for support of Store and Forward Satellite operation for small data transmission

The core network elements/functions to be deployed on board the satellite include:

-	SFCF(S&F control function): A new entity/function responsible for caching S&F UE subscription information, and executing security related operations.

-	S-MME(Satellite Mobility Management Entity): A MME on-board satellite that not only supports the feature of Control Plane CIoT EPS Optimisation as specified in TS 23.401 [5], but also supports the determination of S&F data retention period.

-	S-S-GW(Satellite Serving Gateway): A SGW that is not only able to store S&F user data as defined in the feature of Control Plane CIoT EPS Optimisation, but also supports S&F data storage quota enforcement, retention period execution, and the management of timers for S&F data storage.

-	S-SCEF(Service Capability Exposure Function): A SCEF that is not only able to store S&F user data for Non-IP Data Delivery as specified in the TS 23.682 [7], but also supports S&F data storage quota enforcement, retention period execution, and the management of timers for S&F data storage.

NOTE:	The on-board network elements can be one or more components. The functions of the SFCF can also be part of the S-MME.

The enhancement of core network functions deployed on the ground include:

-	HSS: S&F UE subscription information preparation.

-	P-GW: identifying the satellite for MT data delivery.

The following reference points are proposed:

SFa: Reference point used between SFCF and the S-MME in the satellite.

#### 6.17.1.3	UE Authentication for S&F and Parameter Negotiation

In order to authenticate UE without contacting HSS on the ground, this solution proposes to pre-store ciphered UE subscription information for S&F operation in the UE side (USIM), and upload such ciphered UE subscription information to the SFCF during the initial attach procedure. When the SFCF receives the ciphered UE subscription information, it can decrypt it and obtain the UE subscription information for S&F operation. Thereafter, the SFCF can authenticate the UE based on the UE subscription information for S&F operation.

HSS on the ground is responsible for generating the ciphered UE subscription information for S&F operation which could be pre-configured in the USIM. The HSS also provides the SFCF on the satellite the key materials to decrypt and verify UE subscription information for S&F operation.

NOTE:	It is out of 3GPP scope how the HSS on the ground provides the key materials to SFCFs onboard.

When the UE is attaching to the S-MME for S&F operation, it includes the ciphered UE subscription information for S&F operation. The S-MME will send an authentication request to the SFCF together with the ciphered UE subscription information, and get the authentication result from the SFCF. If the UE authentication successes, the S-MME will determine the S&F data retention period based on the satellite coverage available information and indicate it to the UE.

#### 6.17.1.4	Storing and Forwarding MO Data

A UE can send the MO data to the serving satellite after successful attach. The MO data can be stored in the S-GW or SCEF. The S-GW or SCEF may start the S&F data storage timer which is determined based on satellite ephemeris information and satellite storage quota, so that the S-GW or SCEF can recycle the satellite storage after the S&F data storage timer expires.

After the UE complete the MO data transmission, it either initiates a detach or TAU/RAU procedure if UE wants to use the PSM and request the Periodic TAU/RAU Timer, or sends a data transmission completion notification to the S-MME, which may trigger a MME-initiated detach procedure. If the UE doesn't complete the MO data transmission before the serving satellite flies away, it can access another satellite supporting S&F operation, and send the remaining MO data.

The S-S-GW or S-SCEF storing MO data will forward the stored data to the corresponding server on the ground once a S5 or T8 interface over feeder link is available. The S-S-GW or S-SCEF detects the S5 or T8 interface available based on satellite internal implementation. The SCEF may receive a data transmission completion notification from the S&F server and stop the S&F data storage timer; otherwise, it should consider the data forwarding fails, and notify the MME of the failure.

#### 6.17.1.5	Storing and Forwarding MT Data

When the P-GW/NTN gateway receives a notification for UE's downlink data packet from the server, it has to obtain the UE location to determine which satellite is serving that UE.

NOTE:	It is FFS how the P-GW/NTN gateway obtains the UE location.

When the feeder link of that satellite is available, the P-GW/NTN gateway forwards the downlink data notification to the S-S-GW/S-SCEF. The S-S-GW sends Downlink Data Notification ACK including S&F data storage quota to the P-GW. Then, the S-S-GW receives and stores the data.

The S-MME will wait for UE resuming the connection by TAU procedure (e.g. after the Periodic TAU/RAU Timer of PSM expires, or return to the satellite's coverage) or re-attach procedure for MT data transfer.

### 6.17.2	Procedures

#### 6.17.2.1	Attach with MO data transmission

Figure 6.17.2-1: Initial attach and MO data transmission for CIoT under S&F Satellite Operation

0a.	The HSS provides S&F UE subscription information key materials to the SFCF. The UE stores ciphered S&F UE subscription information.

NOTE:	The HSS provisioning of S&F UE subscription information key materials to SFCF and S&F UE subscription information to UE is dependent on SA WG3 and CT WG6.

0b.	When the UE is in the satellite coverage, the UE receives the broadcast message from a satellite eNB, which indicates storage and forwarding mode and/or satellite ID. The UE can, based on its policies, decide to access this network or other networks not in S&F mode at that time.

1.	The UE initiates the Attach procedure to the eNodeB with an Attach Request including the support of storage and forwarding, ciphered UE subscription information and expected S&F data storage quota. The UE also indicates its support of the Control plane CIoT EPS Optimisations.

2.	According to the attach request, the S-MME sends an authentication request to the SFCF together with the ciphered UE subscription information, and get the authentication result from the SFCF. Then, the S-MME performs authentication and setups NAS security.

3.	The S-MME obtains the UE location to verify and determines whether the PLMN is allowed to operate at the UE location as specified in the TS 23.401 [5].

4.	If Control Plane CIoT EPS Optimisation applies, the S-MME sends Create Session Request (Support of Storage and forwarding, expected S&F data storage quota) to S-S-GW. For PDN type "non-IP" when Control Plane CIoT EPS Optimisations are enabled for the UE, the S-MME establishes a connection to the S-SCEF as specified in the TS 23.682 [7].

5.	The S-S-GW returns a Create Session Response (determined S&F data storage quota) message to the S-MME. For PDN type "non-IP", the S-SCEF address indicated in subscription data as specified in the TS 23.682 [7].

6.	The S-MME determines the S&F data retention period based on the satellite coverage availability information.

7.	The S-MME sends Attach Accept to UE which may include satellite ID, S&F data retention period, determined S&F data storage quota.

8.	RRC connection configuration and attach complete as specified in the TS 23.401 [5].

9.	The UE may send data that meets the S&F data storage quota to S-S-GW/S-SCEF.

10.	The S-S-GW/S-SCEF stores data. The S-GW/SCEF may also start the S&F data storage timer.

11.	When the feeder link is available, based on the internal implementation of satellites, the S-S-GW/S-SCEF can have a connection with the ground CN functions.

12.	The S-S-GW/S-SCEF forward the data to the ground server.

Editor's note:	It is FFS how to support the roaming scenarios.

#### 6.17.2.2	Tracking Area Update with MO/MT data transmission

According to the reference architecture, UE may trigger tracking area update procedures the same as the satellite access and discontinuous coverage conditions for Cellular IoT as specified in the clause 5.3.3.0 of TS 23.401 [5].

Moreover, after the UE complete the data transmission and if it wants to use the PSM, it may also trigger a tracking area update procedure to request a Periodic TAU/RAU Timer value during TAU procedures as specified in the clause 4.5.4 of TS 23.682 [7].

#### 6.17.2.3	Service Request for MO Data transmission

Figure 6.17.2-3 shows the procedure for Mobile Originated Data Transport in Control Plane CIoT EPS Optimisation for Store and Forward Operation.

Figure 6.17.2-3: MO Data transport in Control Plane CIoT EPS Optimisation for Store and Forward Operation

1.	The UE has registered with the network.

2.	The UE sends data to the S-S-GW/S-SCEF.

3.	The S-GW/SCEF stores data. The S-S-GW/S-SCEF may also start the S&F data storage timer.

4a.	If the UE has completed the MO data transmission, it may initiate a detach procedures and the network release the session as specified in the TS 23.401 [5]. Or the UE may initiate TAU procedures and negotiate the Periodic TAU/RAU Timer value with the network to use PSM. Or the UE may send data transmission completion notification to the S-MME, which may initiate the detach procedures.

4b.	If the UE doesn't complete the MO data transmission, it may store the data and access another satellite supporting S&F operation, and send the remaining MO data.

5.	When the feeder link is available, based on the internal implementation of satellites, S-S-GW or S-SCEF knows that the S5 or T8 interface is available, and forwards data to the corresponding server on the ground.

6a.	If the data is fully transmitted from the S-S-GW/S-SCEF to the ground P-GW or server, the S-S-GW or S-SCEF may receive the data transmission completion notification from the P-GW or S&F server and stops the S&F data storage timer.

6b.	If data transmission fails from the S-S-GW/S-SCEF to the ground P-GW or server, the S-S-GW or S-SCEF may receive a data transmission failure notification from the P-GW or S&F server, skip step 7 - 8.

7.	If the data is fully transmitted from the S-S-GW/S-SCEF to the ground P-GW or server, the S-S-GW or S-SCEF may send session release request to the P-GW or S&F server.

8. The P-GW or S&F server may send session release response to the S-S-GW or S-SCEF.

9a.	The S-S-GW or S-SCEF sends session release response to S-MME if a session release response is received.

9b.	If S-S-GW receives a data transmission failure notification in step 6b, it sends session release failure to S-MME due to data transmission failure/abnormality.

10a.	S-MME deletes UE context after receiving session release response.

10b.	S-MME keeps the UE context and notifies the UE of the abnormal cause received from S-S-GW or S-SCEF when the UE connects to the network next time.

### 6.17.3	Impacts on services, entities and interfaces

This solution impacts the following system entities.

HSS:

-	S&F UE subscription information preparation.

S&F control function (SFCF):

-	Catch S&F UE subscription information.

-	Execute security related operations.

Satellite eNodeB(S-eNB):

-	Broadcast Satellite ID and/or Storage and forwarding mode.

S&F UE:

-	Stores S&F UE subscription information.

-	Reports Storage and forwarding support.

-	Reports Expected S&F data storage quota.

Satellite MME(S-MME):

-	Determines S&F data retention period.

Satellite S-GW(S-SGW):

-	Determines S&F data storage quota.

-	Control S&F data storage timer.

Satellite SCEF(S-SCEF):

-	Determines S&F data storage quota.

-	Control S&F data storage timer.

Ground P-GW:

-	Identify the satellite for target UE to transfer MT data.

## 6.18	Solution #18: Attach Procedure with MME and HSS on board the satellite

### 6.18.1	Description

This solution address key issue#2.

In order to support the attach under the S&F Satellite operation, the proposed solution follows the assumptions and principles as below:

-	The eNB, MME and HSS are placed on board the same satellite.

-	The eNB on board satellite broadcasts that it is in S&F satellite operation mode.

-	The UE has a subscription and credentials in the onboard HSS, the onboard HSS is synchronized with the HSS on the ground when feeder link is available.

NOTE: For the UE in roaming case, i.e. there is no subscription and credentials for the H-PLMN stored in the onboard HSS, the MME onboard will reject the attach request and fetch the subscription and credential for the rejected UE from the HSS in HPLMN when the feeder link become available, as described in clause 6.18.2.2 or other methods captured in Sol#14,15.

Given the above assumption, the following procedures involved eNB, MME and HSS can be completed when the service link is available, no matter what the status of feeder link is:

-	The negotiation between UE and MME regarding the capability to support S&F satellite operation.

-	The authentication and NAS security can be accomplished when service link is available.

Therefore, if a UE has set "Attach without PDN Connectivity is supported" in the Preferred Network behaviour during attach procedure and the network has acknowledged its support to the UE, the initial attach procedure without PDN Connectivity can be completed when the service link is available.

If the UE also wants to establish a PDN connectivity during the attach procedure, this solution proposed to attach the UE to the MME and complete the NAS procedure without setting up GTP-C tunnels between the MME and SGW/PGW when service link is available. After the feeder link become available, the GTP-C tunnels can be established. In this case, only Control Plane CIoT EPS Optimisation is considered, and PDN connection only use the Control Plane CIoT EPS Optimisation, i.e. the MME includes a Control Plane Only PDN Connection Indicator in Create Session Request.

A new network function, the Local Subscriber Server (LSS), is introduced to integrate an HSS on the ground with following enhancements:

The UE state (e.g. EMM state) and UE context are stored in the LSS.

The LSS interacts with the onboard MME to synchronize the UE states and UE context when feeder link is available.

It is assumed that a subset of UEs have their subscriptions and credentials in the onboard HSS. Different satellite stores distinct subscriptions and credentials for different subsets of UEs. If the UE accesses a satellite which has its subscription and credential, the UE can proceed to attach, otherwise the attach request is rejected due to the lack of UE subscription and credential. Besides, the satellite can record the IMSI and current TAI for the UE whose attach request is rejected, and retrieve the subscription and credentials for the rejected UE c, and then the UE can access the satellite and tries to attach again when the service link become available. Besides, based on the ephemeris and current TAI of the rejected UE, LSS may also synchronize the rejected UE’s subscription and credentials with other satellites that may cover the rejected UE.

The onboard MME can synchronize the UE states and UE context with LSS when feeder link is available. For instance, After the attach procedure without PDN Connectivity is complete, the UE shall enter EMM-REGISTERED state in the MME onboard satellite-1, and then when the feeder link become available, the UE state (e.g. EMM-REGISTERED) and UE context for UE-1 is stored in the LSS. After that, based on the UE-1’s location and the Ephemeris information of satellite-2, the LSS can synchronize with MME onboard satellite-2 the UE-1’s state (e.g. EMM-REGISTERED) and UE context when the feeder link is available.

### 6.18.2	Procedures

#### 6.18.2.1	Procedures for E-UTRAN Attach in case the onboard HSS holds subscription and credentials for the UE

The procedure for E-UTRAN Attach under the S&F Satellite operation in case the onboard HSS holds subscription and credentials for the UE is shown in the Figure 6.18.2-1

Figure 6.18.2-1: Attach Procedure under the S&F Satellite Operation in case the onboard HSS holds subscription and credentials for the UE

0.	When service link is available, the eNB broadcasts that it is in the S&F satellite operation mode.

1.	If the UE has the capability to support the S&F satellite operation, it initiates the Attach procedure by the transmission, to the eNB, of an Attach Request Message. In addition to the required parameters as specified in clause 5.3.2.1 of TS 23.401 [5], the UE also indicates its capability to support the S&F satellite operation in the Attach Request message.

2.	The eNB forwards the Attach Request message to MME as specified in clause 5.3.2.1 of TS 23.401 [5]. MME checks if the UE has the capability to support the S&F satellite operation, if not, rejects the Attach Request with an appropriate cause value.

3.	If the UE has its subscriptions and credentials in the onboard HSS, Authentication and NAS security is performed as specified in clause 5.3.2.1 of TS 23.401 [5].

4.	MME sends an Attach Accept message to the eNB in the S1-AP Initial Context Setup Request message or DL NAS Transport message. In addition to the required parameters as specified in clause 5.3.2.1 of TS 23.401 [5], MME indicates its capability to support the S&F satellite operation in the Attach Accept message. And then, the eNB forwards the Attach Accept message to the UE. If the UE has not included the ESM message container in the Attach Request in step 1, the following steps are skipped. If the UE has included the ESM message container in the Attach Request in step 1, the MME allocates an EPS Bearer Identity for the Default Bearer associated with the UE. The EPS QoS info can be set to the default value received from the HSS.

5.	The UE sends the Attach Complete message to MME. The MME enters the EMM-REGISTERED for the UE.

6.	When the feeder link is available, the MME synchronizes the UE states and UE context with LSS. The MME sends the UE states (e.g. EMM-REGISTERED) and UE context to the LSS.

6a.	If there’s another satellite that doesn’t holds the UE context for the registered UE. Based on the ephemeris and UE location (e.g. the current Tracking Area List) , LSS may determine to synchronize the UE states (e.g. EMM-REGISTERED), UE context, and credentials with such satellite for the possible data transmission.

7.	If the UE has included the ESM message container in the Attach Request in step 1, when feeder link is available, MME sends a Create Session Request message and a Modify Bearer Request message to the SGW as specified in clause 5.3.2.1 of TS 23.401 [5].

8.	The SGW sends a Create Session Request message and a Modify Bearer Request message to the PGW as specified in clause 5.3.2.1 of TS 23.401 [5].

9.	The PGW returns a Create Session Response message and a Modify Bearer Response message to the SGW as specified in clause 5.3.2.1 of TS 23.401 [5].

10.	The SGW returns a Create Session Response message and a Modify Bearer Response message to the MME as specified in clause 5.3.2.1 of TS 23.401 [5].

11.	The DL data can be sent from S/PGW to MME. The MME receives and stores the DL data for the UE.

12.	When the service link is available, i.e. the satellite covers the tracking area(s) in which the UE is registered, the MME sends a paging message to the eNB. If eNB receive the paging message from the MME, the UE is paged by the eNB.

13.	UE establishes a radio bearer for DL data transmission.

14.	A S1-MME path is established for DL data transmission.

15.	The MME sends the DL data to the UE.

16.	The UL data can also be sent to the MME.

#### 6.18.2.2	Procedures for E-UTRAN Attach in case no subscription and credentials for the UE is stored in the onboard HSS

The procedure for E-UTRAN Attach under the S&F Satellite operation in case no subscription and credentials for the UE is stored in the onboard HSS is shown in the Figure 6.18.2-2.

Figure 6.18.2-2: Attach Procedure under the S&F Satellite Operation in case no subscription and credentials for the UE is stored in the onboard HSS

- When service link is available, the eNB broadcasts that it is in the S&F satellite operation mode.

- If the UE has the capability to support the S&F satellite operation, it initiates the Attach procedure by the transmission, to the eNB, of an Attach Request Message. In addition to the required parameters as specified in clause 5.3.2.1 of TS 23.401 [5], the UE also indicates its capability to support the S&F satellite operation in the Attach Request message.

- The eNB forwards the Attach Request message to MME as specified in clause 5.3.2.1 of TS 23.401 [5]. MME checks if the UE has the capability to support the S&F satellite operation, if not, rejects the Attach Request with an appropriate cause value.

- If there is no subscription and credentials stored in the HSS, the Authentication and NAS security cannot be performed, and the attach request is rejected.

- MME stores the IMSI and current TAI for the rejected UE.

- MME sends the Attach Reject message to UE with an appropriate cause.

- When the feeder link is available, the MME sends the rejected UE’s IMSI and current TAI to LSS. LSS provides the rejected UE’s subscription and security credentials (e.g. subscriber Key) to the onboard HSS. The onboard HSS can use the subscriber Key to generate the Authentication Vectors (RAND, AUTN, XRES, KASME) and provides the AVs to MME for Authentication as described in clause 6.1 of TS 33.401[xx]. After that, based on the ephemeris and current TAI of rejected UE, LSS may decide to synchronize the rejected UE’s subscription and credentials with other satellites that will cover the rejected UE.

Editor's note:	SA3 would need to evaluate the feasibility of the splitting of HSS.

7.	When UE determines that it is in satellite coverage again, the UE re-initiates the Attach procedure as shown in Figure 6.18.2-1 if the onboard HSS holds its subscription and credentials. Otherwise, repeat steps 1-6.

#### 6.18.2.3	Procedure for MO Data Transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation

The procedure for MO data transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation is shown in the Figure 6.18.2-3

Figure 6.18.2-3: MO data transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation

- The UE is ECM-IDLE.

- When the service link is available, the UE establishes an RRC connection and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data.

- The NAS PDU sent in step 1 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

- The MME checks the integrity of the incoming NAS PDU and decrypts the data it contains. The MME stores the UL data until the feeder link is available.

- If the S11-U connection is not established, the MME sends a Modify Bearer Request message for each PDN connection to the Serving GW.

- If the Info IEs and/or UE Time Zone and Serving Network id are present in step 4, the Serving GW shall send the Modify Bearer Request message (RAT Type, MO Exception data counter) to the PDN GW.

- The PDN GW sends the Modify Bearer Response to the Serving GW.

- If a Modify Bearer Request message was sent at step 4 the Serving GW shall return a Modify Bearer Response (Serving GW address and TEID for uplink traffic) to the MME as a response to a Modify Bearer Request message. The Serving GW address for S11-U User Plane and Serving GW TEID are used by the MME to forward UL data to the SGW.

- The MME sends Uplink data to the P-GW via the S-GW.

- If the MME is not aware of pending MT traffic and S1-U bearers are not established, the following steps can be skipped. Otherwise, Downlink data may arrive at the P-GW and the P-GW sends them to the MME via the S-GW.

- If Downlink data are received in step 9, the MME encrypts and integrity protects the Downlink data, and store the DL data until the service link is available.

- If the service link is available, the MME sends Paging message to the eNB, and the UE is paged by the eNB.

- RRC connection is established.

- The UE sends Control Plane Service Request NAS message over RRC Connection request and S1-AP initial message.

- Downlink data are encapsulated in a NAS PDU and sent to the eNodeB in a S1-AP Downlink NAS Message.

15.	The eNB sends an RRC Downlink data message including the Downlink data encapsulated in NAS PDU to UE.

#### 6.18.2.4	Procedure for MT Data Transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation

The procedure for MT data transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation is shown in the Figure 6.18.2-4:

Figure 6.18.2-4: MT data transport in Control Plane CIoT EPS Optimisation under S&F Satellite Operation

- The UE is EPS attached and in ECM-Idle mode.

- The S-GW receives downlink data for a UE.

- SGW stores the DL data.

- If the Serving GW is buffering data in step 2, the Serving GW sends a Downlink Data Notification message to the MME to establish the S11-U connection. SGW can send a Downlink data notification message to each satellite when the feeder link is available. Based on the Ephemeris and UE context (e.g. the current Tracking Area List), MME determines whether the UE will be reachable. If the satellite will cover UE’s current Tracking Area List, MME responses to the SGW with a Downlink Data Notification Ack message. Otherwise, a NACK message will be sent to SGW.

- If the MME determines the UE will be reachable, Step 4-15 of Figure 6.18.2-3 as described in clause 6.18.2.3 is triggered to deliver DL data to the UE.

### 6.18.3	Impacts to Services, Entities and Interfaces

This solution impacts the following entities.

UE:

-	Indicate its capability to support the S&F satellite operation in the Attach Request message.

eNB:

-	Indicate it is in the S&F satellite operation mode in the SIB.

MME:

-	Indicate its capability to support the S&F satellite operation in the Attach Accept message.

-	Receive and Store the UL data when service link is available, and forward the UL data when feeder link is available.

-	Receive and Store the DL data when feeder link is available, and for ward the DL data when service link is available.

-	Complete the NAS procedure of the attach procedure without setting up GTP-C tunnels between the MME and SGW/PGW when service link is available.

-	Synchronize the UE states and UE context with the LSS.

-	Store the IMSI and current TAI for the rejected UE.

-	Send the rejected UE’s IMSI and current TAI to LSS, and retrieves the subscription and authentication vectors from the LSS.

-	Store the UL data.

LSS (New network function):

- Synchronize the UE states and UE context with the onboard MME.

- Provide the rejected UE’s subscription and authentication vectors to the onboard MME.

- Synchronize the UE subscription and security credentials with the onboard HSS.

HSS (onboard the satellite):

- Synchronize the UE subscription and security credentials with LSS.

SGW:

- Store the DL data.

## 6.19	Solution #19: Store and Forward Operation using a UE Proxy and Endpoint Proxy

### 6.19.1	Description

This solution supports KI#2.

A satellite operating in satellite store and forward (SSF) mode contains the functionally of a RAN node (eNB or gNB) plus a CN and has an on board proxy for remote endpoints, referred to as an endpoint proxy. A UE with Release 17 or 18 satellite access capability sees a satellite in SSF mode as providing access to a PLMN (that is on board the satellite) and applications on the UE and the user of the UE interact with the endpoint proxy that enables data and voice transfer to occur as if performed end to end. The solution can support applications that are able to operate in a simplex or half duplex mode - e.g. SMS, CP or UP data transfer, one way voice message transfer, HTTP (e.g. Internet) queries and responses.

The solution is illustrated in Figure 6.19.1-1 and operates as follows.

Figure 6.19.1-1: SSF Operation using a UE Proxy and Endpoint Proxy and an SSFC

A satellite without a feeder link may operate in SSF mode. In this mode, an SSF indication is broadcast in SIB1 along with a PLMN ID for the satellite operator. The PLMN ID could be per country (where a different PLMN ID is broadcast in each country) or might be an international ID with a 9xx MCC. UEs that do not support SSF mode do not access the satellite or get rejected if they do. UEs that support SSF mode and that are allowed to access the broadcast PLMN may access the satellite using existing Release 17 or 18 satellite access capability - e.g. may establish an RRC signalling connection and perform a NAS Attach (4G) or Registration (5G) with a new indication of SSF capability. The onboard satellite CN includes a UDM or HSS that can support secure access for UEs as described later in clause 6.19.2. UEs may also establish PDN connections or PDU sessions (to the onboard satellite CN) and perform IMS Registration (to an onboard satellite IMS)..

The UE can then perform mobile originated (MO) transactions such as sending SMS, sending data (e.g. using IP or non-IP protocols), establishing SIP sessions and sending SIP media (e.g. voice, video, text) or sending HTTP queries. Each MO transaction is sent to the onboard endpoint proxy which stores transaction data (e.g. SMS, data, voice, HTTP queries) and associated protocol and remote endpoint data and returns responses at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeouts and enable a user (if participating) to be aware of the one way communication status. For example, in case of a user, a preconfigured SMS, voice or HTTP reply might be returned indicating when an SMS message, voice message or HTTP query sent by the user may reach the remote endpoint. For real time (e.g. IMS) services, the solution is restricted to one way transfer of messages (e.g. a voice message or a video clip). The recipient can then return a similar one way message in a reply.

Shortly before satellite coverage is lost, the satellite detaches or de-Registers the UE. If there is a radio link failure before this, the satellite and UE both perform a local detach or local de-Registration. If the UE can access a new satellite before coverage is lost, a satellite may transfer UE status including data for ongoing transactions to the new satellite using ISL and the UE then performs a handover.

After the UE loses coverage and when the satellite obtains a feeder link to a ground based PLMN allowed to support the UE at its previous location, the satellite (or the onboard endpoint proxy) transfers all of the stored data for the UE MO transactions to a ground based server, referred to as an SSF centre (SSFC). The transfer procedure is not defined by 3GPP and the SSFC may belong to the satellite operator or to the MNO for the ground based PLMN. The SSFC may contain a proxy for the UE, referred to as a UE proxy, that forwards the voice or data for each MO transaction to the associated remote endpoint and is able to receive and store voice and data for mobile terminated (MT) transactions that may be returned by the remote endpoints to the UE.

If a UE proxy is contained in the SSFC, it may first perform a NAS Attach or Registration on behalf of the UE with the PLMN. This can occur at an Uu level where the SSFC connects to an eNB or gNB for the PLMN and emulates the behaviour of an NTN gateway that has real time satellite access to the UE.

Alternatively,

-	the SSFC may connect to the CN of the PLMN at an S1 or N2 level and, from the perspective of the CN, emulate the behaviour of an eNB or gNB that has real time satellite access to the UE. Alternatively,

-	the SSFC may be integrated with the ground based PLMN in a manner not defined by 3GPP which can avoid use of 3GPP signalling and procedures between the SSFC and the PLMN. If the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface the existing mechanism between UPF/PGW or NEF/SCEF and the DN/PDN.

Each of these alternatives impacts the SSFC but need have no normative impact on 3GPP specifications.

Subsequent to NAS Attach or Registration and establishment of any PDN connections or PDU sessions and IMS Registration, the UE proxy initiates MO transactions corresponding to those initiated earlier by the UE which forward the MO voice and data to the remote endpoints. The SSFC also interacts with remote endpoints to receive and store voice and data for MT transactions and, similar to the endpoint proxy in the satellite, can return responses to the remote endpoints at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeous and enable a user at a remote endpoint to be aware of the one way communication status. For example, in the case of a user, a preconfigured SMS or voice reply might be returned indicating when an SMS message or voice message may reach the UE.

For power saving, a specific S&F monitoring list can used for a UE and SSFC. The UE only listen to the RAN/SATs in the S&F monitoring list, and SSFC only uses the RAN/SATs in this specific S&F monitoring list to send MT data.

The SSFC and UE proxy can simulate continuous reachability of the UE and possibly remain permanently in a CM Connected state. The MT transaction data received and stored by the UE proxy is transferred to one or more satellites that are expected to later provide coverage to the UE. When the UE accesses such a satellite, the endpoint proxy in the satellite can retrieve address information for the corresponding UE (e.g. PDN address/UE IP address, TLTRI) if the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface and transfer the MT transaction data to the UE along with supporting any new MO transactions from the UE based on the received address information.

Satellites carrying MT transaction data for the UE do not page the UE but instead rely on the UE instigating access. However, after initiating MO transactions, a UE or user can be advised when a satellite with MT transaction data in reply will become available and/or can be provided with coverage data for satellites to facilitate access.

On MT transactions, the UE could simply attach to every visible satellite and then wait for possible MO transactions from the user or applications on the UE or for MT transactions from the satellite this could be highly inefficient. A smarter UE could first wait for an indication from the user or from an application on the UE of a pending MO transaction or could wait based on knowledge of when MT transactions may arrive e.g. according to S&F monitoring list, before performing an attach. This could be implementation dependent.

### 6.19.2	Procedures

#### 6.19.2.1	Support of MO and MT Transactions

An MO or MT transaction can correspond to:

-	Transfer of one SMS message (e.g. using NAS or IMS).

-	Transfer of a set of data to or from one remote endpoint using CP CIoT or UP.

-	SIP session establishment to or from one remote endpoint, one way transfer of media (e.g. voice, video and/or text) followed by SIP session release.

-	An HTTP request and/or response where the response may be preconfigured.

Figure 6.19.2.1-1 shows how MO and MT transactions can be supported between a UE and remote endpoints in SSF mode.

UE

Satellite

RAN

CN

Ep Proxy

SSFC

UE Proxy

PLMN

HPLMN

Remote Endpoin((s)

T1: INITIAL UE INTERACTION WITH SATELLITE – SERVICE LINK, NO FEEDER LINK

T2: SSFC INTERACTION WITH PLMN AND ENDPOINTS – NO SERVICE LINK, FEEDER LINK

T3: SUBSEQUENT UE INTERACTION WITH SATELLITE – SERVICE LINK, NO FEEDER LINK

1. SIB1 (SSF indication)

2. NAS Attach or Registration, establish PDN connections/PDU sessions, IMS Registration

3. MO Transaction(s)

4. NAS Detach or De-registration

5. Transfer UE status and MO Transaction(s)

6. NAS Attach or Registration, establish PDN connections/PDU sessions, IMS Registration

7. MO Transaction(s)

8. MT Transaction(s)

9. Transfer Mt Transaction(s)

11. SIB1 (SSF indication)

12. NAS Attach or Registration, establish PDN connections/PDU sessions, IMS Registration

15. MO Transaction(s)

16. NAS Detach or De-registration

14. MT Transaction(s)

10.Endpoint proxy subscribes from the CN

Figure 6.19.2.1-1: Support of MO and MT Transactions

1.	A satellite operating in SSF mode broadcasts an SSF indication and a PLMN ID for the satellite operator in SIB1.

2.	At a time T1, a UE that supports SSF mode, obtains an RRC Connection and sends a NAS Attach Request or Registration Request as for Release 17/18. The UE also includes an indication of SSF capability. The MME/AMF will reject the Attach or Registration if the SSF indication is not included. The rest of the Attach or Registration procedure occurs as for Release 17/18. If the UE needs to transfer data over UP or transfer SIP media data, the UE establishes PDN connection(s) or PDU session(s) and, for SIP media, performs an IMS Registration. All UE interactions are with the RAN, CN and Endpoint proxy in the satellite and do no involve any other ground based entities. Support of security for a Registration, Attach or IMS Registration is described in clause 6.19.2.2.

3.	The UE instigates one or more MO transactions to the satellite on board CN. Existing 3GPP procedures in Release 17/18 are used without change. The on board CN forwards MO transaction data and protocol and signalling information applicable to the remote endpoint(s) to the endpoint proxy which stores this and returns any responses to the UE necessary to ensure correct operation of transport and application protocols. The endpoint proxy may also return preconfigured application level messages to a user of the UE to clarify SSF operation. For example, a preconfigured voice message may indicate an expected delay in forwarding MO transactions to the remote endpoint(s) and may further indicate a later time when MT transaction responses may be received back at the UE from the remote endpoint(s). The endpoint proxy may also request and obtain the location of the UE (e.g. location at time T1). The endpoint proxy stores the MO transaction data from the UE.

4.	Shortly before the UE will lose satellite coverage; the satellite performs a NAS Detach or De-registration of the UE. If satellite coverage is lost before this can occur, the UE and onboard satellite CN each perform a local Detach or local De-registration. If ISL is supported and another satellite becomes available to the UE before coverage is lost, the satellite can transfer UE state and ongoing transaction information to the new satellite using ISL and instigate UE handover. Preventing a UE attempting handover or cell change to a new satellite when ISL is not supported can be supported without new impact by broadcasting a different TA from each satellite and prohibiting UE access to other TAs.

5.	At a later time T2 when the satellite has feeder link access to an SSFC with access to a PLMN allowed to serve the UE at its location at time T1, the endpoint proxy transfers UE status (e.g. GPSI of the UE if the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface) and MO transaction data to a UE proxy in the SSFC. The transfer procedure is not defined by 3GPP. If there were no MO transactions at step 3, an indication of UE presence only is transferred.

6.	The SSFC and UE proxy simulate satellite real time access to the UE at its location at time T1. If the PLMN has defined a TA and fixed cell that covers this location, the SSFC and UE proxy can indicate UE presence in this TA and fixed cell by indicating the TA and cell with SSFC connection to the PLMN at an S1 or N2 level. For SSFC connection at a Uu or N1 level, the UE proxy can send the UE location a time T1 to the PLMN when requested. The UE proxy sends a NAS Attach Request or Registration Request to the MME or AMF at a Uu/N1 level or S1/N2 level according to how the SSFC connects to the PLMN. The rest of the Attach or Registration procedure occurs normally. If the UE proxy needs to transfer data over UP or transfer SIP media data, the UE establishes PDN connection(s) or PDU session(s) and, for SIP media, performs an IMS Registration with security. Support of security for a Registration, Attach or IMS Registration is described in clause 6.19.2.2.

NOTE:	The UE Proxy may, but is not required to, transfer SMS messages and data using the same procedures and protocols as in step 3. For example, the UE could use CIoT to transfer data at step 3 whereas the UE Proxy could use UP.

7.	The UE Proxy instigates one or more MO transactions to transfer any data and SIP media for the MO transactions at step 3 to the remote endpoint(s). Existing 3GPP procedures are used without change. The UE proxy may send preconfigured application level messages to a user of a remote endpoint to clarify SSF operation. For example, in the case of a SIP voice media message, the UE proxy may indicate to the user that it is about to play a voice message from the UE, may indicate the time and possibly location at which the message was originally sent and may further prompt the user to replay a voice message or return a voice message reply.

8.	Remote endpoint(s) (e.g. a user at a remote endpoint) may instigate one or more MT transactions to return responses to the UE Proxy for the MO transaction(s) received at step 7. The UE Proxy stores the received data from the UE and associated protocol and signalling information and returns any responses to a remote endpoint necessary to ensure correct operation of transport and application protocols. The UE Proxy may also send preconfigured application level messages to a user at the remote endpoint to clarify SSF operation. For example, application level messages may indicate an expected delay in forwarding MT transactions to the UE and may further indicate a later time when MO transaction responses from the UE may be received back at the remote endpoint. The UE proxy may also assemble an internal MT transaction that might be an SMS message or voice message that confirms delivery of the MO transactions at step 7 (e.g. indicates the times when MO transactions were delivered).

9.	The SSFC periodically transfers MT transaction data from the UE obtained at step 8 to one or more satellites that will later provide coverage to the UE location for time T1. Each satellite passes the MT transaction data, for a UE identified by the GPSI if the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface, to an Endpoint Proxy for the UE where it is stored. The MT transaction data may be retained at the SSFC until a satellite later indicates delivery of the MT transactions to the UE. The SSFC can maintain the UE Proxy in CM Connected state or allow the PLMN to place the UE Proxy in IDLE state. In the latter case, the SSFC simulates continuous reachability of the UE.

10.	If the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface, after received MT transaction data from the UE from the SSFC in step 9, the endpoint proxy sends the GPSI of the UE to the CN onboard to subscribe to address information (e.g. PDN address/UE IP address for IP packet, TLTRI for unstructured data) or the reachability of the UE (i.e. UE has attached/registered to the CN onboard for SMS).

11. At a later time T3, a satellite that received MT transaction data for the UE from the SSFC and UE Proxy at step 9 is able to provide coverage at the UE location. The satellite broadcasts an SSF indication and the PLMN ID for the satellite operator in SIB1 as at step 1.

12.	The UE obtains an RRC Connection and sends a NAS Attach Request or Registration Request as at step 2. Other actions in step 2 are also performed.

13. The CN notifies the address information or the reachability of the UE to the endpoint proxy, if the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface.

14.	The Endpoint Proxy instigates one or more MT transactions to the UE to forward the MT transactions received at step 9 based on the received address information or the reachability of the UE in step 13 (e.g. sends MT data to the TLTRI of the SCEF/NEF, replaces the target IP address of the MT transaction data to the PDN address/UE IP address and sends to the PGW/UPF if the SSFC connects to remote endpoints via N6/SGi or N33/T8 interface). Existing 3GPP procedures in Release 17/18 are used without change. The endpoint proxy may also provide preconfigured application level messages to a user of the UE to clarify SSF operation. For example, transfer of voice messages to the user may be preceded by a preconfigured voice message indicating the sender of each message and the time when each message was sent by the remote endpoint.

15.	The UE may instigate one or more MO transactions to the endpoint proxy as in step 3 to reply to MT transactions received at step 12 or to initiate new transactions.

16.	Shortly before the UE will lose satellite coverage, and unless handover to a new satellite is possible using ISL, the satellite performs a NAS Detach or De-registration of the UE as in step 4, or the UE and onboard satellite CN each perform a local Detach or local De-registration.

#### 6.19.2.2	Support of Security

For UE access to a satellite in SSF mode with 4G access, the IOPS solution defined in Informative Annex F in TS 33.401 [23] is used, where satellites replace the HSSs used for IOPS. This requires an IOPS capable USIM in a UE which can either be the normal USIM enhanced for IOPS or a dual USIM that is used by the UE only for SSF access. With an IOPS solution, and for each supported UE, each satellite would be provisioned with the IMSI for the UE and a security key K* derived from the long term (master) key K in the USIM. The derivation of K* from K can be based on a satellite ID as described in TS 33.401 Annex F [23] for HSS IDs. The satellite does not perform the derivation of K* and is just provisioned with K* by the HPLMN and satellite operators. The USIM performs the derivation after receiving the satellite ID as part of authentication. The derived key K* is used for authentication  and ciphering. Each satellite can have a different ID and different K* value for a UE or groups of satellites can share an ID and K* value. There are no new impacts to an ME (except for selecting a dual USIM for SSF access). The IOPS solution avoids exposure of the long term (master) key K to satellites (and the SSFC) and is thus more secure.

The IOPS solution could also be used for 5G access since the authentication parameters can be the same and the same IOPS capable USIM can be used for both 4G and 5G access.

Editor's note:	SA3 would need to verify the above and define more details for the security procedures.

For SSFC access to a ground based serving PLMN, the initial satellite for a UE first transfers the authenticated IMSI or SUPI/SUCI to the SSFC. The SSFC then maps the IMSI or SUPI/SUCI to a second IMSI or SUPI/SUCI  for the UE that is provisioned in the SSFC by the HPLMN and satellite operators. The second IMSI or SUPI has a second long term security key K2 (different to the key K)  that is also provisioned  in the SSFC. The SSFC uses the second IMSI or SUPI and the second key K2 to authenticate the second IMSI or SUPI to the HPLMN, where the second IMSI or SUPI and second key K2 are also configured in the HPLMN HSS (4G) or UDM or ARPF (5G). The second IMSI or SUPI is also associated with existing public identifiers (e.g. MSISDN or GPSI) for the UE to allow MT service data from remote endpoints to reach the UE without any new impact to the remote endpoints. The capability to share the same public user identities among different private identities for a UE is already supported by some PLMNs in the form of a second IMSI or SUPI/SUCI for an additional user device such as a smart watch that is associated with a primary user device (e.g. a smart phone). The capability can include simultaneous coexistence of an attach or registration by both devices. HPLMN specific logic can then be used to direct MT services to one or both of the devices.

The K2 values for different UEs (who have the same HPLMN) may be different or may be the same because authentication of the UE proxy is actually an authentication of the SSFC for either case and not the individual UE which is only authenticated by the satellite. The usage of a second IMSI or SUPI allows a different security key K2 in the SSFC to the security key K in the USIM.

When a dual USIM in a UE (not the normal USIM) is used, and is used only for SSF access with a different IMSI or SUPI/SUCI than that in the primary USIM, the mapping by the SSFC of the IMSI or SUPI/SUCI to a second IMSI or SUPI/SUCI is not needed and the IMSI or SUPI/SUCI from the dual USIM can be used to authenticate the UE proxy with the HPLMN. Even though the same IMSI or SUPI would then be authenticated by both a satellite and HPLMN, the two keys (K and K2) that are used for the two authentications can then still be different.

NOTE 1: The end to end operation of the security solution will be defined by SA3.

#### 6.19.2.3	Transition between SSF and Real Time Modes

Orbiting LEO and MEO satellites can support SSF mode along orbital segments where no feeder link is available and real time mode along segments where a feeder link is available. UEs which only see satellites on the SSF segments or only on the real tine segments would not need to deal with mode transitions. However other UEs will as described here.

When a satellite supporting real time mode loses access to a feeder link, the satellite would cease support for real time mode and may switch to SSF mode. UEs which accessed the satellite previously may be able to handover to another satellite with a feeder link if one is available, but will otherwise experience a coverage gap. Some of these UEs may then commence SSF access as described in clause 6.19.2.1.

When a satellite supporting SSF mode obtains a feeder link, the satellite may cease support for SSF mode and switch back to real time mode. UEs in SSF mode can be detached or de-Registered prior to cessation of SSF mode as described in clause 6.19.2.1. Satellite coverage data provided to a UE (if supported) can indicate the durations of SSF satellite coverage and, separately, the durations of real time satellite coverage. UEs can then know ahead of time when SSF coverage will cease. When an SSF coverage gap is about to occur, a UE can detach or de-Register from the satellite.

A UE that switches between SSF and real time modes may experience problems in interacting with remote endpoints as data and voice sent in real time mode could arrive (at the UE or at a remote endpoint) before data and voice sent earlier for SSF mode. MT data sent to a UE that was previously attached or registered for real time access will also be buffered in a serving PLMN, while the UE is in a real time coverage gap, which could lead to an opposite effect where MT data sent later to the UE when in SSF mode arrives first. In addition, an Attach or Registration for a UE in real time mode would normally pre-empt and release an existing Attach or Registration by the UE for SSF mode or the reverse. This is not a problem when a UE transitions from real time mode to SSF mode as the real time attach or registration is no longer needed. But for a transition from SSF mode back to real time mode, the real time Attach or Registration would either be released if the SSFC instigates the Attach or Registration for SSF mode in the serving PLMN after the real time Attach or Registration has occurred, or would cause a release of the Attach or Registration for SSF mode (and cause ongoing MO and MT transactions for SSF mode to be aborted) if the SSFC Attach or Registration had occurred first.

The security solution described in clause 6.19.2.2 avoids conflicts between SSF and real time Attach or Registration due to use of a different IMSI or SUPI/SUCI for each, which allows an Attach or Registration for SSF mode to coexist with an Attach or Registration for real time access (via TN or NTN) without either of these affecting the other. If the UE (e.g. a user or application) also consistently uses only SSF mode or only real time mode for communication with each remote endpoint, mis-ordering of MO data and voice arrival can also be avoided. Mis-ordering of MT data arrival at the UE might be tolerated by the user given awareness of the dual usage of SSF mode and real time mode.

#### 6.19.2.4	Support of Discontinuous Satellite Coverage in SSF Mode

With discontinuous satellite coverage, a UE may not be able to send and receive much MO and MT transaction data in SSF mode before coverage is lost and may be limited to only certain times of sending and receiving. For example with just one minute of satellite coverage, MO transactions initiated by a user and MT transactions sent to a user would be very limited (e.g. to voice messages of less than 1 minute in duration).

To overcome such limitations as not being able to send and receive much MO and MT data, a UE (e.g. an Application in the UE) can package and assemble a generic data message containing data supplied by a user or other applications for multiple MO transactions. The generic data message can also contain information for the intended remote endpoints. Thus is illustrated in Figure 6.19.2.4-1.

Figure 6.19.2.4-1: Packaging of MO or MT Transactions into a Single Packaged Transaction

The generic data message is sent to a satellite in a single MO transactions and later transferred to the UE proxy in the SSFC where it is unpackaged into the original data for the different MO transactions. The original data is then forwarded as normal to the remote endpoints. Similar packaging and unpackaging can occur for MT transactions where data for multiple MT transactions and information for the associated remote endpoints is packaged by the SSFC into a single MT transactions which is transferred to a satellite, later transferred to the UE as a single MT transaction and finally unpackaged by the UE (e.g. an Application in the UE) into the original MT transactions which are made available (e.g. at a later time) to the user or other applications of the UE.

The transfer of a single packaged MO transaction from the UE to a satellite or a single packaged MT transaction from the satellite to the UE can occur using high bandwidth and be less limited by limited coverage time at the UE and availability of the user.

To overcome such limitations as to be limited to only certain times of sending and receiving to assist with UE power saving, the UE msy also include a preferred satellite monitoring mode (e.g., single satellite, a subset of satellites, all satellites) in the NAS Attach Request or Registration Request. If the MME/AMF accepts the Attach or Registration, the MME/AMF exposures the preferred satellite monitoring mode and the UE location to the endpoint proxy, then the endpoint proxy may determine a S&F monitoring list based on the information provided by MME/AMF, the communication pattern of the UE, the mobility trajectory of the UE, location of the SSFC and satellite ephemeris, and provides the S&F monitoring list to the MME/AMF, which then provides the S&F monitoring list to the UE.

After Detach or De-registration, the UE keeps the S&F monitoring list. The S&F monitoring list is transferred to the SSFC as part of the UE status and the SSFC may select target satellite(s) based on the S&F monitoring list.

### 6.19.3	Impacts to Services, Entities and Interfaces

SMS Service:

-	Applications and users of SMS need to expect longer than normal delay.

Data Services (e.g. IP, UDP/IP. TCP/IP. Non-IP):

-	Applications and users are restricted to sending data one way where an Endpoint Proxy or UE Proxy sends preconfigured Reponses at a transport and application level to allow correct protocol operation without timeouts or abnormal termination.

Voice Service:

-	Applications and users are restricted to one way voice messaging.

HTTP Services (e.g. Internet access):

-	Applications and users are restricted to sending one way HTTP queries or responses where an Endpoint Proxy or UE Proxy sends preconfigured replies to allow HTTP operation without timeouts or abnormal termination.

UE:

-	New SIB1 SSF flag, new SSF capability in a NAS Attach Request or Registration Request, local Detach or De-registration if needed when satellite access is lost.

-	Use of an IOPS capable USIM or dual USIM.

-	Impacts to advise a user and applications of SSF mode.

-	Send preferred satellite monitoring mode (e.g., single satellite, a subset of satellites, all satellites) to CN and store the S&F monitoring list received from the CN is supporting S&F monitoring list.

PLMN RAN:

-	No new impacts.

VPLMN CN:

-	No new impacts.

HPLMN CN:

Impacts to manage a second IMSI or SUPI/SUCI for each UE.

-	Send preferred satellite monitoring mode to the endpoint proxy, is S&F monitoring list is used.

-	Receive S&F monitoring list from the endpoint proxy and send it to the UE, if use.

Satellites:

-	New SIB1 SSF flag, verification of UE SSF capability in a NAS Attach Request or Registration Request, Detach or De-registration of a UE before satellite access is lost.

-	Support of on board eNB or gNB and CN functionality according to Release 17 or 18.

-	Support of an endpoint proxy for certain MO and MT services.

-	Support of interaction with an SSFC.

-	Support for provisioning of UE security credentials.

-	Support of transferring MO/MT data with an SSFC on ground for the corresponding UE.

SSFC:

-	Support of interaction with satellites (not in scope for 3GPP).

-	Support of a UE proxy for certain MO and MT services.

-	Support for provisioning and use of UE security credentials.

-	Support of transferring MO/MT data for the corresponding UE with an endpoint proxy on board.

-	Support of satellite selection for MT data transmission based on the S&F monitoring list, if used.

## 6.20	Solution #20: Solution on enabling S&F operation and data delivery with only eNB onboard satellite

### 6.20.1	Description

This solution resolves KI#2 Store and Forward operation with only eNB onboard satellite.

This solution assumes the following:

-	Only eNB is embedded and the eNB can buffers data for Store and Forward operation and no other Core Network function is embedded in the satellite.

-	UE and MME negotiate about the support of Store and Forward operation capability during attach procedure.

-	MME and eNB store in the UE context that the UE is operate in Store and Forward operation mode.

-	It is assumed that there is only one satellite serving the UE for Store and Forward Operation.

-	The satellite broadcasts that it supports Store and Forward operation in the SIB and satellite ID. The UE operates for S&F operation mode accesses the satellite based on this information.

-	During the feeder link is unavailable, it is assumed that the S1 interface is maintained between the satellite eNB and MME. (e.g. S1 suspend/resume)

-	The satellite eNB shall Store and Forward the received message between the UE and MME.

-	It is assumed that the UE communicate with the same serving satellite based on the satellite ID broadcasted from the serving satellite during the attach procedure.

-	The eNB and MME know that they support S&F operation each other based on the pre-configuration or in the S1 SETUP REQUET and S1 SETUP RESPONSE message, the eNB and the MME indicate to each other that they support S&F operation.

Editor's note:	It is FFS that how to manage S1 interface connection regarding S&F during the connection between eNB and MME is interrupted and will be determined by RAN3.

The solution reuses Control Plane CIoT EPS Optimisation and the User Plane CIoT EPS Optimisation functionality including Connection Suspend procedure and Connection Resume procedure. Also, functions for High latency communication are reused to invoke extended buffering of MT data at the S-GW when the UE is not reachable.

When the MME receives Downlink Data Notification from S-GW, MME cannot know whether and when the downlink data can be forwarded to the serving satellite that is associated with the MME because the MME does not aware whether feeder link is available. Also, S-GW cannot know how long it should buffer the data before the feeder link available.

In Rel-18, the MME can use satellite coverage availability information to support satellite access by UEs with discontinuous coverage operation. Satellite coverage availability information may be provisioned to the MME by O&M and it describes when and where satellite coverage is expected to be available in an area.

This solution introduces feeder link availability information to support the store and forward operation for eNB embedded in the satellite. This information may also be provisioned to the MME (How the information is provisioned to the MME is out of scope of this solution). Feeder link availability information contains e.g.:

-	For each satellite, satellite ID and orbit information.

-	Gound gateways connected to each MME and/or location of ground gateway (e.g. Cell, TA, Geographical location).

-	Time windows when gateways are connectable via feeder link for each satellite.

Based on the above information, the MME can know when each satellite is available via feeder link through the ground gateway connected to the MME. It is also preconfigured to the MME and eNB with the information on when and which areas (e.g. supported TAI list and valid period) will be covered by the satellites. This can be used to determine whether to forward the MT data for the UE to the satellite for store and forward operation.

The MME can derive Downlink Buffering Duration time as the expected time until the feeder link could be available to the serving satellite based on the feeder link availability information. MME provide it to the S-GW and S-GW stores Downlink Data Buffer Expiration Time based on the Downlink Buffering Duration time received from the MME.

### 6.20.2	Procedures

#### 6.20.2.1	Attach Procedure

The procedure for Initial Attach for S&F operation is depicted in Figure 6.20.2.1-1. The steps enclosed in dashed line boxes are executed when either only service link or feeder link is available.

Figure 6.20.2.1-1: Attach Procedure with eNB onboard satellite

1.	UE provides Attach Request to the satellite broadcasts that it supports Store and Forward operation in the SIB with S&F support indication in the UE Core Network Capability when the UE operate for S&F operation mode and the UE also indicates Control Plane/User Plane CIoT EPS Optimisation supported in Preferred Network Behaviour. The RRC message contains the Attach Request contains S&F indication. The UE calculate the orbital period of the satellite based on the ephemeris information of the SIB from eNB. The UE will suspend the NAS procedure based on this information even if the RRC Connection is released, until it receives the NAS message from the MME. The eNB and UE stores UE context including the UE ID.

NOTE:	Details of the UE context stored for next RRC connection will be determined by RAN.

For the case of UEs attaching with Control Plane CIoT EPS Optimisation with no user plane establishment, steps 12, 15 to 17, 20, 21 are replaced by S1 AP NAS Transport and RRC Direct Transfer messages that just transport the NAS Attach Accept and NAS Attach Complete messages.

2.	Based on the S&F indication in the RRC message, the eNB supporting Store and Forward operation stores the UE request until the feeder link is available. When the eNB in satellite detect that a feeder link is available, based on the pre-configuration or support of S&F operation from MME, forward attach request to the MME via ground gateway.

3-4.	The MME triggers the authentication procedure and the MME forwards the authentication message to the eNB. The eNB stores the message until the service link is available again. The MME determines NAS timer to suspend the NAS procedure until the satellite connect to the MME again based on the preconfigured coverage information.

5.	When the service link is available, based on the orbital period calculated by the UE, the UE establish RRC Connection to the same eNB to resume the attach procedure and the eNB forwards the message from the MME to the UE based on the UE ID in step 1.

6-11.	The attach procedure continues.

12. The MME stores in the UE context that the UE operates in Store and Forward operation mode. The MME sends Initial Context Setup Request to eNB. The eNB stores in the UE context that the UE operates in Store and Forward operation mode. Attach Accept message contains S&F support indication in the UE Core Network Capability and indicates the CIoT EPS Optimisations accepted by the MME in the Supported Network Behaviour information.

If the MME does not allow the UE to operate in Store and Forward operation mode, the MME rejects the UE request and indicate the eNB to release the UE contexts using UE Context Release Command. For the UE that operates in Store and Forward operation mode, the eNB releases the UE context after forwarding the Attach Reject message to UE.

For the case of UEs attaching with Control Plane CIoT EPS Optimisation with no user plane establishment, S1 AP NAS Transport message is used to send NAS Attach Accept message.

13.	For the UE that operates in Store and Forward operation mode with User Plane CIoT EPS Optimisations, the MME triggers steps 10-13 in Figure 6.20.2.2-1 to trigger suspend mode and release the S1-U bearer.

14.	RRC Security Mode is performed.

15.	RRC Connection Reconfiguration includes Attach Accept message. The MME provides TAU timer considering the time period until the satellite cover the location of the UE again based on the coverage information on when and which areas (e.g. supported TAI list and valid period) will be covered by this serving satellite.

If the eNB received an S1-AP Downlink NAS Transport message (e.g. containing the Attach Accept message), the eNB sends a RRC Direct Transfer message to the UE.

16-17.	UE sends RRC Connection Reconfiguration Complete and Direct Transfer (Attach Complete) consecutively.

18.	UE may send Uplink data at this step.

19.	Based on the procedure performed at step 13, the eNB sends RRC message to suspend the RRC Connection towards the UE including UE Resume ID.

20-21.	When the feeder link is available, eNB sends Initial Context Setup Response and Attach Complete message consecutively to the MME.

22.	Steps 5-8 in Figure 6.20.2.4-1 may be performed to send MO data.

#### 6.20.2.2	MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if feeder link is available

Figure 6.20.2.2-1: MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if feeder link is available

0.	The UE is EPS attached and in ECM-Idle mode. For the UE that operates in User Plane CIoT EPS Optimisations is in ECM-Idle mode with suspend.

1.	S-GW receives a downlink data packet for a UE, it buffers the downlink data packet.

2.	The S-GW sends a Downlink Data Notification message to the serving MME for the UE.

3.	The MME checks that the UE is operating as a Store and Forward operation mode. If it is, based on the feeder link availability information and last known UE location, the MME further checks if there is a feeder link to the serving satellite eNB which will cover the UE.

4.	The MME detects that there is a feeder link with satellite and can be reached to the satellite at the time of receiving Downlink data notification, the MME response with Downlink Data Notification Ack message. If there is a reachable feeder link.

When the feeder link is available but the MME determines that the serving satellite will not cover the area where the UE is located (e.g. coverage change due to the Earth rotates) based on the preconfigured coverage information, the DL data need to be buffered in the Core Network. Based on the TAU timer provided by the MME, the UE can trigger TAU with another satellite that supports Store and Forward operation in the SIB if the UE cannot find the current serving satellite after the TAU timer expires, or if the UE find a satellite broadcasting new TA that is not in the list of TAIs that the UE registered with the network, and the serving satellite can be changed. In this case, the MME determine DL buffering duration time considering the TAU timer provided to the UE in addition to the feeder link availability information, and provide it to S-GW as in step 4 of Figure 6.20.2.3-1. Then, steps 4-6 in Figure 6.20.2.3-1 are performed. After the serving satellite change, the UE context in the old satellite can be released by the (old) MME when the (old) MME connects to the old satellite via feeder link.

5.	[UP CIoT] The MME sends S1-AP message to the eNB for User Plane activation between the satellite and the S-GW if the MME determines that the feeder link to the MME is available. The S1-AP message include the information that buffering data in the satellite is needed for Store and Forward operation, and UE location information (e.g. TA) for assistance information when paging.

Editor's note:	What S1-AP message will be used is FFS and will be determined by RAN3.

If the eNB receives S1-AP message from MME at step5, the eNB checks whether it serves the area corresponding to the UE location information in the received S1-AP message. If so, the eNB paging the UE and the UE initiated Connection resume procedure as clause 5.3.5A of TS 24.301 [17] is performed and the rest of this procedure is skipped. Otherwise, continues step 6.

6-8.	[UP CIoT] The eNB sends UE Context Resume Request message to the MME to re-activate the S1-U bearers. The MME requests the S-GW to re-activate the S1-U bearers for the UE. The MME sends UE Context Resume Response to the eNB after Modify bearer with S-GW.

9.	[UP CIoT] Downlink data is transferred from the S-GW to the eNB and the eNB buffers the downlink data.

9a-9c.	[CP CIoT] The MME sends a Modify Bearer Request message to activate S11-U. Then, Buffered Downlink data is sent by the S-GW to the MME and the MME encrypts and integrity protects Downlink data.

10.	[UP CIoT] The MME may send S1-AP message to the eNB to indicates that the feeder link will be unavailable.

Editor's note:	What S1-AP message will be used is FFS and will be determined by RAN3.

[CP CIoT] The MME includes Downlink data using a NAS PDU in this Downlink S1-AP message.

11-13.	[UP CIoT] The eNB sends UE Context Suspend Request and S1-U bearer is released.

After this step, feeder link between satellite and MME become unavailable. During the feeder link is unavailable, the S1 interface is maintained between the eNB and MME.

14.	When the satellite covers the TA where the UE is located which is received at step 5, eNB sends paging request to send stored data to the UE.

15.	[UP CIoT] The UE sends the RRC Connection Resume message and the UE becomes RRC Connected state.

[CP CIoT] The UE sends Control Plane Service Request NAS message over RRC Connection request. and S1-AP initial message.

16.	Stored Downlink data in the satellite is forwarded to the UE by UP or CP using a NAS PDU in this Downlink S1-AP message.

17.	While the Service link is still up, further Uplink data can be transferred from the UE to the satellite and the satellite stores the uplink data until the feeder link is available again and forward the data.

18.	[UP CIoT] The eNB sends RRC message to suspend the RRC Connection towards the UE including UE Resume ID based on the preconfigured coverage information.

#### 6.20.2.3	MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if the feeder link is unavailable

Figure 6.20.2.3-1: MT Data delivery in CIoT EPS Optimisation for Store and Forward Operation if the feeder link is unavailable

0.	The UE is EPS attached and in ECM-Idle mode. For the UE that operates in User Plane CIoT EPS Optimisations is in ECM-Idle mode with suspend.

1.	The S-GW receives a downlink data packet for a UE, it buffers the downlink data packet.

2.	The S-GW sends a Downlink Data Notification message to the serving MME for the UE.

3.	The MME checks that the UE is operating as a Store and Forward operation mode. If it is, based on the feeder link availability information, the MME further checks if there is a feeder link to the satellite eNB which will cover the UE.

4.	The MME detects that there is no feeder link with satellite and cannot be reached to the satellite at the time of receiving Downlink data notification, shall invoke extended buffering depending on operator configuration. The MME derives Downlink Buffering Duration time that expected time before the feeder link can be available to the serving satellite based on the feeder link availability information. the MME indicates Downlink Buffering Requested to the S-GW in the Downlink Data Notification Ack message and includes a Downlink Buffering Duration time.

If the MME determines that the serving satellite will not cover the area where the UE is located (e.g. coverage change due to the Earth rotates) based on the preconfigured coverage information, the MME can determine DL buffering duration time considering the TAU timer provided to the UE in addition to the feeder link availability information.

5.	A S-GW that receives a Downlink Buffering Requested indication in a Downlink Data Notification Ack message stores a new value for the Downlink Data Buffer Expiration Time based on the Downlink Buffering Duration time and continues buffering the Data until the Downlink Data Buffer Expiration Time expires.

6.	If the MME determines that the feeder link become available based on the feeder link availability information and the MME has provided Downlink Buffering Requested indication in a Downlink Data Notification Ack at step 4, the MME triggers step 5 of Figure 6.20.2.2-1 to activate user plane data for User Plane CIoT EPS Optimisation case or the MME triggers step 9a of Figure 6.20.2.2-1 to activate S11-U for Control Plane CIoT EPS Optimisation case. Then the procedure continues to the step 18 of Figure 6.20.2.2-1.

NOTE 1:	The feeder link may be available when the same serving satellite returns to the MME or if the UE performed TAU with the new satellite (due to TAU timer expiration or mobility), when the new satellite is connected to the MME with the feeder link.

NOTE 2:	The UL and DL delays depend on the orbital period of the satellite, and multiple orbital periods may be required. For example, if the satellite can find the UE or gateway within its orbital period, the UL and DL delays will be less than the orbital period. However, if the satellite cannot find the UE or gateway within its orbital period, the UE will trigger TAU which requires 2 more orbital periods, resulting in 3 orbital periods. If re-authentication is required during the TAU, additional 2 orbital periods are required, resulting in 5 orbital periods.

#### 6.20.2.4	MO Data Delivery in CIoT EPS Optimisation for Store and Forward Operation

Figure 6.20.2.4-1: MO Data delivery in CIoT EPS Optimisation for Store and Forward Operation

0.	The UE is EPS attached and in ECM-Idle mode. For the UE that operates in User Plane CIoT EPS Optimisations is in ECM-Idle mode with suspend and Steps 10 - 13 in Figure 6.20.2.2-1 is performed before the feeder link between eNB and MME become unavailable.

1.	[CP CIoT] The UE sends the RRC Connection Resume message and the UE becomes RRC Connected state.

[UP CIoT] The UE establishes RRC connection including an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data.

2.	[UP CIoT] The UE sends Uplink data to the satellite.

3.	The satellite may forward stored downlink data to the UE by UP or CP using a NAS PDU in this Downlink S1-AP message.

4.	[UP CIoT] The eNB sends RRC message to suspend the RRC Connection towards the UE including UE Resume ID based on the preconfigured coverage information.

5.	[UP CIoT] After Feeder link become available, steps 6-8 in Figure 6.20.2.2-1 are performed to activate S1-U bearer.

6.	[UP CIoT] eNB forwards stored uplink data to the S-GW.

7.	[UP CIoT] Buffered downlink data can be transferred from the S-GW to the satellite and the satellite stores the downlink data until the service link is available again.

8.	[CP CIoT] The NAS PDU sent in step 1 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

9.	[CP CIoT] The MME checks the integrity of the incoming NAS PDU and decrypts the data it contains.

10.	[CP CIoT] The MME sends a Modify Bearer Request message to activate S11-U if the S11-U connection is not established.

If the UE's Location and/or Info IEs and/or UE Time Zone and Serving Network id are received from MME, the Serving GW shall send the Modify Bearer Request message (RAT Type, MO Exception data counter) to the PDN GW.

11.	[CP CIoT] The MME forwards the stored uplink data to the S-GW and S-GW forward it to the P-GW.

12.	[CP CIoT] Buffered downlink data can be transferred from the S-GW to the MME.

13.	[CP CIoT] If Downlink data are received in step 12, the MME encrypts and integrity protects the Downlink data.

14.	[UP CIoT] Steps 10-13 in Figure 6.20.2.2-1 are performed to trigger suspend mode and release the S1-U bearer.

### 6.20.3	Impacts to Services, Entities and Interfaces

UE:

-	Suspend the NAS procedure based on the orbital period calculated based on the ephemeris information in the SIB from eNB.

-	RRC connection indicates S&F mode operation.

-	Trigger TAU with another satellite that supports Store and Forward operation in the SIB if the UE cannot find the current satellite after the TAU timer expires.

eNB:

-	New S1-AP message from MME for User Plane activation and indicates that buffering data in the satellite is needed for Store and Forward operation, and includes the UE location information (e.g. TA) for assistance information when paging.

-	New S1-AP message from MME indicates that the feeder link will be unavailable.

-	Support user plane activation with S-GW when the eNB is not connected with UE.

-	Support RRC Connection resume with UE when the eNB is connected with CN.

-	Paging for stored data/signalling when the eNB covers the area corresponding to the UE location information (e.g. TA) provided by MME.

-	Preconfigure when and which areas (e.g. supported TAI list and valid period) are covered by the satellites.

-	Support Store and Forward the NAS message between the UE and the MME for the UE operates in S&F mode.

-	During the feeder link is unavailable, support to maintain the S1 interface between the eNB and MME. (e.g. suspend/resume)

-	UE Context in eNB includes whether the UE is operating in Store and Forward operation mode

-	New IEs in SIB to broadcast that it supports Store and Forward operation.

-	Indicates to the MME that it supports S&F operation in the S1 SETUP REQUEST message.

MME:

-	Support feeder link availability information.

-	Derive Downlink Buffering Duration time based on the feeder link availability information.

-	UE Context in MME includes whether the UE is operating in Store and Forward operation mode

-	New S1-AP message to the eNB for User Plane activation and indicates that buffering data in the satellite is needed for Store and Forward operation. It also includes the UE location information (e.g. TA) for assistance information when paging.

-	New S1-AP message to the eNB indicates that the feeder link will be unavailable.

-	Preconfigure when and which areas (e.g. supported TAI list and valid period) are covered by the satellites.

-	During the feeder link is unavailable, support to maintain the S1 interface between the eNB and MME. (e.g. suspend/resume)

-	Determine the periodic TAU timer provided to the UE based on the based on the coverage information on when and which areas (e.g. supported TAI list and valid period) will be covered by the satellite

-	Store DL/UL NAS data when service link or feeder link is not available.

-	Indicates to the eNB that it supports S&F operation in the S1 SETUP RESPONSE message.

## 6.21	Solution #21:	Support of store and forward satellite operation in EPS

### 6.21.1	Description

This solution resolves KI#2 about supporting store and forward satellite operation for delay-tolerant/non-real-time services. The solution is based on architecture with eNB on board and all EPC functionality are deployed on the ground, as shown in the following figure:

Figure 6.21.1-1: Regenerative based satellite architecture with eNB on board

The following basic principles and assumptions are applied to the solution:

-	The full eNB is on board satellite, only one satellite serves the UE access, all EPC functionalities are deployed on the ground.

-	The UE subscription information contains store and forward information (e.g. data storage quota, data forward priority, etc), if the UE subscribed S&F capability.

-	Assuming UE finished attachment procedure before performing store and forward capability.

Editor's note:	It is FFS how the UE performs attachment in case of no simultaneously connection for service link and feeder link.

-	When eNB detects the feeder link is not available, it performs uplink data storage and forwards the stored uplink data if feeder link becomes available.

-	When MME detects the feeder link is not available, it performs downlink data storage or it indicates S-GW to perform downlink data storage. The MME forwards the stored downlink data or it indicates S-GW to forward the stored downlink data to eNB if the feeder links becomes available.

-	When eNB detects the service link is not available, it performs downlink data storage and forwards the stored downlink data if service link becomes available.

-	Architectural assumptions and principles defined in clause 4 in 23700-29 is applied to this solution.

### 6.21.2	Procedures

1	Attach procedure.

Figure 6.21.2-1: UE attach procedure based on regenerative satellite architecture

The attach procedure for supporting store and forward feature is described as below:

1.	The UE accesses EPC via satellite access with eNB on-board. The UE sends attach request message to the network.

2.	The attach request is transferred to the MME by the eNB using S1-AP message. If the on-board eNB supports S&F, the eNB S&F capability is included in the S1-AP message as well and sent to MME.

3.	When the MME has no UE subscription information, it retrieves the information from HSS. If the UE subscribed S&F service, S&F information (e.g. data storage quota, data forward priority) is contained in the UE subscription.

4.	Based on the eNB supporting S&F capability, the UE S&F subscription information, the MME determines S&F capability is applied to the UE.

5.	The S&F subscription information is stored at the MME.

6.	Other steps of attach procedure as defined in clause 5.3.2.1 of TS 23.501 [2] are finished between UE and network.

7.	The MME uses S1-AP to send attach accept to eNB. If S&F capability is determined to applied to the UE, the S&F subscription information is also sent to the eNB.

8.	The eNB returns attach accept message to the UE.

9.	The UE sends attach complete message to the network.

2	MO data transmission using control plane signalling based on S&F operation

Figure 6.21.2-2: MO data transmission using control plane signalling based on S&F operation

The MO data transmission based on S&F operation is described as below:

0.	The UE is ECM-IDLE.

1.	The UE establishes a RRC connection or send the RRC early data request message as defined in TS 36.300 [13] and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS bearer ID and encrypted Uplink Data. The UE may also indicate a NAS Release Assistance Information in the NAS PDU whether no further uplink or downlink data transmissions are expected, or only a single downlink data transmission (e.g. Acknowledge or response to uplink data) subsequent to this uplink data transmission is expected

2.	If the on-board eNB detects feeder link is unavailable, it stores the NAS DATA PDU based on the data storage quota. How to handle the data if the quota is over can be based on operator policy. E.g. discard the data and inform the event to UE.

3.	After a period of time, if the on-board eNB detects the feeder link becomes available, then it forwards the stored NAS DATA PDU based on the data forward priority. For example, if there is network congestion during the feeder link recovery, the data with high priority can be transmitted first.

4.	The on-board eNB forwards the NAS DATA PDU in the S1-AP intial UE message to the MME.

5.	The MME checks the integrity of the incoming NAS PDU and decrypts the data. If the S11-U connection is not established, the MME sends a Modify Bearer Request to SGW to establish S11-U connection with S-GW. The MME sends uplink data via the S11-U connection to the S-GW and further to P-GW.

6.	If the NAS Release Assistance Information from the UE indicates no downlink data is expected, or if the MME is not aware of pending MT traffic and S1-U is not established, step6-8 skips. Otherwise, downlink data may arrive at the P-GW and the P-GW sends the downlink data to the MME via the S-GW.

7.	If the MME detects feeder link is unavailable, it stores the downlink data based on the data storage quota.

8.	When the feeder link is detected becomes available, the MME encrypts and integrity protects to the stored downlink data.

9.	The MME forwards the stored downlink data to the eNodeB in a S1-AP Downlink NAS Message based on data forward priority. Data encapsulation may be applied in the transmission. The MME may indicates acknowledgment is requested as per UE subscription. If the NAS Release Assistance Information in step 1 indicated that downlink data was expected and if MME does not aware of additional MT traffic or S1-U bearer, the MME sends a S1 UE Context Release Command immediately after the S1-AP to indicate eNB to release the RRC connection promptly after successfully sending data to the UE.

10.	The eNB receives the downlink data. If it detects the service link is unavailable, it stores the downlink data. The storeage is based on the storeage quota.

11.	If the service link is detected available, the eNB sends a RRC downlink data message including the downlink data to the UE. If the eNB receiving the S1-AP message in step 9 was followed by an S1 UE Context Release Command, an S1 release procedure is enforced after the downlink data is transmitted to the UE and the acknowledgement is sent to the MME by the eNB.

12.	Other steps of the MO data transport in Control Plane CIoT EPS Optimization defined in clause 5.3.4B.2 of TS 23.401 [5] are performed. During these steps, if feeder link or service link is detected unavailable, the Store and Forward feature as described in steps 2-3, 7-8, or 10-11 will be performed will be performed at eNB or MME accordingly.

3	MT data transmission using control plane signalling based on S&F operation:

Figure 6.21.2-3: MT data transmission using control plane signalling based on S&F operation

The MT data transmission based on S&F operation is described as below:

1.	When the S-GW receives a downlink data for a UE, it forwards the downlink data to the MME as described in steps 1-2, 7-11 in clause 5.3.4B.3.

2.	If the MME detects feeder link is unavailable, it stores the downlink data based on the data storage quota.

3.	After a period of time, the MME detects the feeder link becomes available.

4.	The MME sends Paging message as specified in step 3a of clause 5.3.4.3.

5.	The eNB receives the Paging message. When eNB detects service link is unavailable, it records the paging information, e.g. the UE location information.

Editor's note:	It is FFS whether and how to response to MME if the service link is unavailable when eNB receiving Paging. How to set the timer used to supervise the paging procedure by MME is FFS.

6.	The eNB detects the service link becomes available. It uses the paging information received in step 4 to send paging to the UE.

7.	The eNB sends Paging message to the UE as specified in step 4a of clause 5.3.4.3.

8.	Upon reception of paging indication, the UE sends Control Plane Service Request NAS message over RRC Connection request. The Control Plane Service Request NAS message does not trigger data radio bearer establishment by the MME.

9.	The eNB stores Control Plane Service Request NAS message if the feeder link is unavailable.

10.	When the feeder link recovers, the eNB sends Control Plane Service Request NAS message over S1-AP initial message.

11.	When the MME receives Control Plane Service Request NAS message, it can send the stored downlink data to the eNB based on the data forward priority using a NAS PDU carried by a Downlink S1-AP message. The downlink data may be encrypted and integrity protected by the MME.

12.	If the on-board eNB detects service link is unavailable, it stores the downlink data based on the data storage quota.

13.	After a period of time, the on-board eNB detects the service link becomes available.The stored downlink data is delivered to the UE via a Downlink RRC message based on the data forward priority. This is taken as implicit acknowledgment of the Service Request message sent in step 8.

14.	Other steps of the MT data transport in Control Plane CIoT EPS Optimization defined in clause 5.3.4B.3 of TS 23.401 [5] are performed. During these steps, if feeder link or service link is detected unavailable, Store and Forward feature as described in steps 2-3, 9, 12 will be performed at eNB or MME accordingly.

### 6.21.3	Impacts to Services, Entities and Interfaces

The on-board eNB has the following impacts:

-	Reports S&F capability to MME.

-	Receives and stores S&F subscription information for the UE.

-	Detects the available/unavailable of service link/feeder link.

-	Performs S&F operation based on UE S&F subscription information.

The MME has the following impacts:

-	Determine to apply S&F capability for a UE based on eNB S&F capability and UE subscription information.

-	Sends UE S&F subscription information to the eNB.

-	Detects the available/unavailable of feeder link.

-	Performs S&F operation based on UE S&F subscription information.

The S-GW may have the following impacts:

-	Performs S&F operation based on MME indication.

## 6.22	Solution #22 for Key Issue #2: Support of Store and Forward Satellite operation for SMS and Data Transmission

### 6.22.1	Introduction

This solution corresponds to KT#2 to enable SMS and data transmission services with store and forward satellite operation. It supports one satellite serving a UE (Scenario1) and Multiple satellites serving a UE (Scenario 2).

### 6.22.2	Description

This solution is applicable for the satellite architecture for SMS in MME and data transmission in Control Plane CIoT EPS Optimisation, as shown in figure below, based on the architectures in Annex C of TS 23.272 [18] and TS 23.401.

The core network elements deployed on satellite include:

- MME: An on-board MME can be used to store MO/MT SMS and MO/MT data in Control Plane CIoT EPS Optimisation. The MME forwards MO SMS/MO data to S-GW/SMS-IWMSC when the feeder link is detected as available based on e.g. the ephemeris information and connectivity information of feeder link. The MME forwards MT SMS/MT data to UE when the service link to UE is available based on e.g. the ephemeris information and UE location.

NOTE:	The mechanism proposed in this contribution can also apply for SMS over SGs, for example, MSC is deployed with MME on the satellite.

Figure 6.22.2-1: Satellite Architecture for SMS and data transmission

There could be two scenarios to offer UE services over satellite access.

- Scenario 1: Only one satellite serves a UE and different satellites serve different UEs. Only the satellite serving the UE needs to retain UE context, rather than requiring multiple satellites to retain UE context. Each satellite does not need a large capacity since different UEs contexts are distributed within different satellites. However, the UE must wait coverage from the serving satellite for MO/MT signaling or data.

- Scenario 2: Multiple satellites serve a UE. The multiple satellites could provide services to UE therefore reducing service delivery delay. However, synchronization of UE context among multiple satellites is necessary before serving a UE, and typically, one satellite maintaining contexts for all UEs needs large capacity of satellite, e.g. storage for all UEs context and processing for synchronization.

For the procedures outlined in this solution, clause 6.22.3.1 applies to both Scenario 1 and Scenario 2. Clause 6.22.3.2, 6.22.3.3, 6.22.3.3A and 6.22.3.4 apply to Scenario 1. Clause 6.22.3.5 applies to the Scenario 2, by specifying enhancement on the mechanisms of 6.22.3.2, 6.22.3.3, 6.22.3.3A and 6.22.3.4 to support multiple satellites.

It is assumed that the UE attaches to the EPS network in S&F operation mode by using other solutions, e.g. Solution#11, 12, 13 and 15 described in other chapters.

The following general mechanisms are used by this solution to support store and forward satellite operation for both Scenario 1 and Scenario 2.

- Since the NFs on-board satellite do not always have continuous feeder link connected to ground, if ground NFs do not know the availability communication information, including when NFs on-board satellite can communicate with and the corresponding connection information, DL signalling or data destinated to on-board NFs may fail and/or be retransmitted multiple times. In order to eliminate the impact on DL data and DL signalling including SMS, the on-board NF should provide the availability communication information to the ground NF. Specifically, the on-board NF (e.g. MME) can determine the availability communication information based on the ephemeris information and connectivity information of feeder link. When the satellite is connected to the ground via feeder link, the on-board NF can send the availability communication information to the ground NF (e.g. HSS and S-GW). Detailed procedure is described in clause 6.22.3.1.

For Scenario 1 with different satellites serving different UEs, the following mechanisms apply:

- Before the UE decides to establishes connection to the current E-UTRAN onboard satellite for UL services, e.g. UL signalling/SMS or Data, the UE determines whether the current E-UTRAN onboard satellite can provide services for UE, based on comparation of the information provided before by MME and the broadcast information of the E-UTRAN. If not then the UE waits another applicable E-UTRAN onboard satellite.

- The E-UTRAN onboard satellite broadcasts the information according to the information provided by the connected MME. MME can provide this information to E-UTRAN during S-1 setup procedure. This information can be e.g. GUMMEI.

- During the completion of the Attachment, UE obtains this information from the MME. It can be e.g. GUMMEI derived from GUTI. When the UE needs to start Initial Attach the UE does not care the the broadcast information of the E-UTRAN. It means the UE can access any E-UTRAN onboard satellite for Initial Attachment.

The following general mechanisms are used by this solution to support Scenario 2.

- UE context synchronization among multiple satellites (MMEs) can be achieved through a synchronization system e.g. OAM, which is out of scope of 3GPP. The synchronization system only control the entities onboarding the Satellite, not the network entities on the ground.

- The HSS stores information of multiple MMEs toward UE, i.e. the HSS is aware of the list of MMEs which can serve the UE, along with MME availability communication information.

- The S-GW stores information of multiple MMEs toward UE, i.e. the S-GW is aware of the list of MMEs which can serve the UE, along with MME availability communication information.

- In order to ensure that multiple on-board network elements retain the context of the UE, upon receiving the Update Location Request message from the new MME, the HSS does not trigger the Cancel Location procedure to the old MME if it is multiple satellites serving a UE.

The following mechanisms are used by this solution to support SMS with store and forward satellite operation. If not specified, it applies to both Scenario 1 and Scenario 2.

When the ground NF (e.g. HSS) knows that there is a DL signalling to be sent, it determines whether the DL signalling/SMS can be sent to on-board NF (e.g. MME) based on the stored available communication information of the on-board NF. For MT SMS, the HSS can return a failure report or the address of MME to the SMS-GMSC and the detailed description is described in clause 6.22.3.2.

After the MME receives MT SMS from SMS-GMSC, MME decides two options, as further detailed in clause 6.22.3.3:

- Option A: The MME stores the MT SMS received from SMS-GMSC and forwards it to UE later when the UE can be reached.

- Option B: The MME rejects the MT SMS received from SMS-GMSC. The MME can receive the MT SMS again from SMS-GMSC at a future given time when the MME approaches the UE, then the MME performs Option A. This policy aims to economically utilize the costly storage resource on satellite. For Scenario 2, a more appropriate MME may be identified to handle the MT SMS. Then the MT SMS is sent to the new MME and subsequently Option A is performed.

The following mechanisms are used by this solution to support data transmission with store and forward satellite operation. If not specified, it applies to both Scenario 1 and Scenario 2.

- When S-GW receives the MT data from P-GW, it determines whether the MT data can be sent to on-board MME or be buffered locally based on the stored availability communication information of the MME.

- When receiving Downlink Data Notification message from S-GW, the MME responds to S-GW either indicating its readiness to receive MT data, or requesting the S-GW to buffer MT data if the MME determines to handle it at a future given time, or a more appropriate MME is identified to handle the MT data for Scenario 2.

### 6.22.3	Procedures

#### 6.22.3.1	Provision of available communication information

This procedure is used to inform the ground NFs about the availability communication information.

The availability communication information can be transmitted by on-board NFs to ground NFs at the node level once the feeder link is available. The per-node messages are transmitted through interfaces e.g. S6a between the MME and HSS, and S11-C between the MME and S-GW. The detailed procedure is assumed to be defined by CT.

This available communication information indicates the period of time when the on-board NFs can be communicated with the ground NFs and the corresponding connection information. It may have the following forms:

-	the available communication time at present, which can be represented by the end time (UTC) or duration of the current communication period, and the corresponding connection information, e.g. MME Number for MT SMS and MME IP address for S11-C.

-	all the available communication time intervals and all the connection information, e.g. MME Number for MT SMS and MME IP address for S11-C, within a certain long period of time (24 hours).

#### 6.22.3.2	MT SMS Delivery procedure related to HSS

This procedure is used when the HSS knows there is an MT SMS to be sent from SMS-GMSC to UE.

Figure 6.22.3.2-1: MT SMS Delivery procedure related to HSS

1.	The UE attaches or performs Tracking Area Update to EPS network via Regenerative-based satellite access, which supports store and forward operation. The on-board MME may provide available communication information to ground HSS through the procedure defined in clause 6.22.3.1.

2.	The SC initiates transfer of MT SMS.

3.	The HSS is requested to retrieve the routing information for routing the MT SMS to the MME.

4.	When receiving Send Routing Info for SM Request message, the HSS is aware that an MT SMS needs to be delivered to the UE. The HSS determines whether the MT SMS can be sent to on-board MME, based on the stored available communication information.

Case 1: Forward the MT SMS.

5a.	The HSS responds to the SMS-GMSC with the addresses of the serving nodes that are registered for MT SMS.

6a.	The SMS-GMSC forwards the MT SMS message to the MME.

7a.	Perform MT SMS Delivery procedure related to MME as specified in clause 6.22.3.3.

Case 2: Forward the MT SMS at a correct time.

5b.	The HSS returns the failure information within the Send Routing Info for SM Answer message.

6b.	The SMS-GMSC sends a failure report to SC.

7b.	The HSS determines the correct time when the MT SMS can be sent based on the available communication information of the MME.

8.	The HSS sends an Alert-SC message to the SMS-GMSC. Then a SC-Alert message is sent by SMS-GMSC to the SC.

9-10.	The messages are same as steps 2-3.

11.	Perform step 4 and steps 5a-7a as defined in Case 1.

#### 6.22.3.3	MT SMS Delivery procedure related to MME

This procedure is used after the MME receives a MT SMS from SMS-GMSC when the satellite is connected to the ground via feeder link.

Figure 6.22.3.3-1: MT SMS Delivery procedure related to MME

1.	The MME receives the MT SMS and determines which option to perform. The MME can store the MT SMS received from SMS-GMSC and forwards it to UE later as described in Option A. The MME can reject the MT SMS received from SMS-GMSC. The MME receives the MT SMS again from SMS-GMSC at a future given time when the MME approaches the UE, as described in Option B. This policy aims to economically utilize the costly storage resource on satellite.

Option A: The MME performs S&F for SMS.

2a.	The MME stores the MT SMS.

3a.	The MME determines to send the MT SMS to the UE at the time when the satellite can cover the UE, e.g. based on the locations of the UE.

4a.	The MME pages the UE. If the UE is in the ECM-IDLE state, upon reception of paging message from MME, the UE triggers Service Request procedure.

5a.	The MME encapsulates the MT SMS in a NAS message and sends the message to the UE.

6a.	The UE acknowledges receipt of the MT SMS to the MME.

7a.	The UE sends the delivery report to the MME via the UL NAS Transport message.

If the UE has an MO SMS, it can encapsulate the MO SMS in a UL NAS message.

8a.	The MME stores the MO SMS or the delivery report.

9a-10a.	The delivery report or MO SMS message is transmitted to the destination SC, e.g. via SMS-GMSC or SMS-IWMSC when the satellite is connected to the ground via feeder link.

Option B: The MME performs S&F for SMS at a future given time.

3b.	The MME sends a failure report.

4b.	Upon reception of the failure report, the SMS-GMSC requests the HSS to add an SC address to the MWD by sending SM-DeliveryReportStatus message.

5b.	The SMS-GMSC, in parallel with step 4b, sends a failure report to the SC.

6b.	The MME determines when the MT SMS can be received again, e.g. when MME is approaching to UE, in order to economically utilize the costly storage resource on satellite.

7b.	The MME sends a Notify Request message to HSS notifying that the MME is reachable. Then an Alert-SC message is sent by HSS to the SMS-IWMSC indicating that the MME (not the UE) is now available for MT SMS delivery. The SMS-GMSC forwards an SC-Alert message to SC.

8b.	The MT SMS is sent from SC/SMS-GMSC to MME following existing procedures.

9b.	After receiving MT SMS, then the MME performs Option A.

#### 6.22.3.3A MO SMS Delivery procedure

Figure 6.22.3.3A MO SMS Delivery procedure

0.	The UE is ECM-IDLE.

1.	When the UE needs to send MO SMS and there is an available service link to serve the UE, the UE establishes connection to E-UTRAN and MME and the MO SMS is received by MME.

For Scenario 1 with different satellites serving different UEs, the UE determines whether the current E-UTRAN onboard satellite can provide services for UE, based on comparation of the information provided before by MME and the broadcast information of the E-UTRAN, as described in 6.22.2.

2.	The MME stores MO SMS until the feeder link is available.

3-4. When the feeder link is available, the MO SMS is forwarded to SC and a Delivery Report message is received by MME. The MME stores the Delivery Report until the service link is available.

- When the service link is available, MME sends Delivery Report to UE following existing procedures.

#### 6.22.3.4 MT/MO data transmission

Figure 6.22.3.4-1: MT data transmission in S&F operation mode

0.	The UE is EPS attached and in ECM-Idle mode.

1-2.	The P-GW sends the DL data to S-GW received from SGi, e.g. from Application Server. The S-GW determines whether the DL data can be sent to on-board MME, based on the stored availability communication information of MME. The S-GW buffers DL data, if the on-board MME is unavailable.

3.	If the S-GW is buffering data in step 2, when the feeder link between satellite and the ground is available, the S-GW sends the Downlink Data Notification message to the MME serving UE when it is Control Plane CIoT EPS Optimisation, based on the stored connection information, e.g. MME IP address for S11-C. The TEID of an MME for a given UE remains unchanged across different feeder links.

4.	If the MME determines not to handle the DL data this time, e.g. due to restricted and costful storage resource on satellite, the MME responds to the S-GW with a Downlink Data Notification ACK message, including the indication for Downlink Buffering Requested and the Downlink Buffering Duration time, in order to delay the DL data transmission.

5.	The bearer modification procedure is performed.

6-7.	Buffered Downlink data is sent by the S-GW to the MME and MME stores the Downlink data.

8.	When the service link is available, the UE is paged by E-UTRAN/MME if the UE is in the ECM-IDLE state. The MME can determine the paging time, e.g. based on the locations of the UE.

9.	Steps 12-21 in clause 5.3.4B.3 of TS 23.401, if it is Control Plane CIoT EPS Optimisation, are performed. Then the DL data stored in MME is sent to UE.

Figure 6.22.3.4-2: MO data transmission in S&F operation mode

0.	The UE is ECM-IDLE.

1.	When the UE needs to send UL data and there is an available service link to serve the UE, the UE establishes connection to E-UTRAN and MME for Control Plane CIoT EPS Optimisation and the UL data is received by MME, as specified in clause 5.3.4B.2 of TS 23.401, if it is Control Plane CIoT EPS Optimisation.

For Scenario 1 with different satellites serving different UEs, the UE determines whether the current E-UTRAN onboard satellite can provide services for UE, based on comparation of the information provided before by MME and the broadcast information of the E-UTRAN, as described in 6.22.2.

2.	The MME stores the UL data until the feeder link is available.

3.	When the feeder link is available, other steps defined in clause 5.3.4B.2 of TS 23.401 are performed. Then the stored UL data is sent to S-GW.

#### 6.22.3.5 Support for multiple satellites serving a UE

##### 6.22.3.5.1 UE context synchronization among multiple satellites

Figure 6.22.3.5.1-1: UE context synchronization among multiple satellites serving a UE

1.	UE attaches to EPS network via MME on-board Satellite #1 by using other solutions, e.g. Solution#11, 12 and 15, described in other chapters.

2.	During the attachment, when the feeder link is available, the MME on-board Satellite #1 performs location updating and session creation procedures with ground HSS and ground S-GW, respectively. The on-board MME can include a mutiple-MMEs support indication in e.g. Update Location Request message and Create Session Request message to the HSS and S-GW, respectively. The HSS and S-GW are aware that the MME on-board Satellite #1 serves the UE and store the list of MMEs serving the UE to support Multiple satellites serving a UE (Scenario 2).

If the UE moves from legacy network(e.g. TN network) to NTN network supporting S&F operation, then the HSS shall perform Cancel Location procedure towards the old MME in legacy network, since the old legacy MME did not include mutiple-MMEs support indication during the previous Update Location Request procedures.

3.	After attach completion, multiple satellites synchronize information, including UE context, through a Synchronization System, e.g. OAM. The synchronization may also take place during the attach procedure.

4.	If another satellite, e.g. Satellite #2, serving the UE connects to ground then MME on-board Satellite #2 may perform location updating and bearer modification procedures. The on-board MME can include a mutiple-MMEs support indication within e.g. Update Location Request message and Modify Bearer Request message to the HSS and S-GW, respectively. The HSS and S-GW are aware that the MME on-board Satellite #2 serves the UE and update the list of MMEs serving the UE to support Multiple satellites serving a UE (Scenario 2). The HSS does not trigger Cancel Location procedure to the old MME on-board Satellite #1, as described in clause 6.22.2.

If the UE moves to a new legacy network, e.g. TN network that does not supporting S&F operation, the MME in the legacy network shall send Update Location Request and Create Session Request following existing procedures. Then the HSS shall store the new MME information serving UE and perform Cancel Location procedure towards the old MMEs in S&F NTN network. If the SGW is not changed, then the SGW shall remove all the context of the old MMEs within the S&F supporting NTN network.

NOTE: 	There may be a NTN network supporting S&F operation where not all the satellites can serve a UE because of the specific satellite constellation. When UE moves to a new area within the NTN network, the UE performs Tracking Area Update due to TA change, then the new MME on the satellite serving the new area performs step 2 and MMEs list in the HSS is updated accordingly. The old MME serving the old area shall remove itself from the MMEs list in the HSS by sending purge message including multiple-MMEs support indication to HSS, based on control of the synchronization system.

##### 6.22.3.5.2 MT/MO SMS delivery procedure

MT SMS delivery procedure for Scenario 2 is performed by reusing the MT SMS delivery procedure for Scenario 1 in 6.22.3.2 and 6.22.3.3 with the following enhancements:

-	In step 4 of clause 6.22.3.2, the HSS determines one MME within the list stored in HSS, to be responsible for sending MT SMS to the UE, based on the list of MMEs serving UE and the availability communication information. Then the HSS sends MME Number for MT SMS to SMS-GMSC.

-	In step 1 of clause 6.22.3.3, the MME can determine whether to process the MT SMS. If not, a new MME, e.g. the one capable of providing service to the UE most quickly, is identified via e.g. synchronization system, to handle the MT SMS. Then the old MME performs steps 3b-5b and the new MME performs steps 6b-9b in clause 6.22.3.3.

MO SMS delivery procedure in S&F operation mode for Scenario 2 can reuse the procedure for Scenario 1 defined in clause 6.22.3.3A.

##### 6.22.3.5.3 MT/MO data transmission procedure

MT data transmission procedure for the Scenario 2 is performed by reusing the MT data transmission procedure for Scenario 1 defined in clause 6.22.3.4, with the following enhancements:

- In step 2 of Figure 6.22.3.4-1, the S-GW determines one MME within the list stored in S-GW, to be responsible for sending MT data to the UE, based on the list of MMEs serving UE and the availability communication information. Then the S-GW sends Downlink data Notification message to the MME.

- In step 4 of Figure 6.22.3.4-1, the MME can determine whether to process the MT data. If not, a new MME, e.g. the one capable of providing service to the UE most quickly, is identified via e.g. synchronization system, to handle the MT data. Then the new MME performs steps 5-9 in Figure 6.22.3.4-1.

MO data transmission procedure in S&F operation mode for Scenario 2 can reuse the MO data transmission procedure for Scenario 1 defined in clause 6.22.3.4.

### 6.22.4	Impacts on services, entities and interfaces

The following impacts are foreseen by this solution:

MME:

-	The MME determines the available communication information and provides it to the ground NFs (e.g. HSS) and/or S-GW) at the node level.

-	Upon reception of an MT SMS from SMS-GMSC, the MME decides to store and forward it this time, or reject it and then receives the MT SMS again at a future given time, based on policy. For Scenario 2, the MME responsible for delivering MT SMS to UE may be changed through e.g. synchronization system.

-	The MME stores MT SMS/MO SMS, and forwards it to UE/SMS-IWMSC when the UE/SMS-IWMSC can be reached later.

-	Upon reception of an DL data Notification message from S-GW, the MME decides to receive this time or at a future given time, based on policy. For Scenario 2, the MME responsible for delivering MT data to UE may be changed via e.g. synchronization system.

-	The MME stores MT data/MO data, and forwards it to UE/S-GW when the UE/S-GW can be reached later.

-	The MME determines to page the UE at appropriate time and/or location if there are MT SMS or MT data.

-	Synchronization of UEs EPS context information among multiple MMEs by synchronization system, e.g. OAM.

HSS:

-	The HSS receives the available communication information from on-board NF (e.g. MME) at the node level.

-	The HSS determines whether the DL signalling can be sent to on-board NF (e.g. MME) based on the available communication information of the on-board NF.

-	For MT SMS, the HSS determines to forward MT SMS, or rejects it and forwards it at a correct time based on the available communication information.

-	The HSS stores the list of MMEs serving the UE along with the corresponding availability communication information for multiple MMEs. Upon receiving the Update Location Request message from the new MME, the HSS does not send a Cancel Location message to the old MME for Scenario 2.

S-GW:

- The S-GW receives the availability communication information from on-board NF (e.g. MME) at the node level.

- The S-GW determines whether the MT data can be sent to on-board NF (e.g. MME) or be buffered locally based on the availability communication information of the on-board NF.

- The S-GW stores the list of MMEs serving the UE along with the corresponding availability communication information for multiple MMEs.

UE:

- Determines whether the current E-UTRAN onboard satellite can provide UL services for UE, based on the broadcast information of the E-UTRAN, in case of Scenario 1 with different satellites serving different UEs.

E-UTRAN onboard satellite:

-	Broadcasts the corresponding information provided by the connected MME, in case of Scenario 1 with different satellites serving different UEs.

## 6.23	Solution #23: Support for SMS in satellite network for S&F operation

### 6.23.1	Key Issue mapping

This solution addresses the SMS support of Key Issue #2 (Support of Store and Forward Satellite operation).

### 6.23.2	Description

Consider the extreme case depicted in Figure 6.23.2-1 with a satellite orbit consisting of only one LEO satellite:

Figure 6.23.2-1: Incomplete satellite orbit with only one satellite

Assuming that the satellite orbit is located at 500 km altitude, the orbital period is roughly equal to 90 minutes. The size of the satellite's coverage area depends on several factors, but for simplicity it is assumed that a given spot on the Earth is served by the satellite for 5 minutes during each satellite pass. Due to Earth's rotation, a given satellite will pass over the same geographical area only after several orbits. Given that there is only one satellite in Figure 6.23.2-1, the UE will be served intermittently i.e. only during 5 minutes within a period of several hours.

In the scenario depicted in Figure 6.23.2-1 the satellite establishes a contact with the UE and with the ground station at T1 and T3, respectively. At no time does the satellite have simultaneous connection to both the UE and the ground station due to the absence of Inter-Satellite Links (ISLs) with additional satellites.

As a consequence, none of the signalling procedures between the UE and the Core Network can be executed in their entirety as an atomic procedure.

The solution is based on the EPS architecture. It is assumed that the MME function is split into two parts: MME-SAT functionality residing on the satellite (together with RAN functionality) and MME-GND functionality residing on the ground. This is illustrated in Figure 6.23.2-2.

Figure 6.23.2-2: Satellite architecture for S&F operation (SMS related)

The exact split of functionality between MME-SAT and MME-GND, as well as the definition of Ssf reference point, is considered outside of 3GPP scope.

Functionality that is not relevant for the SMS-related procedures is omitted from Figure 6.23.2-2.

### 6.23.3	Procedures

#### 6.23.3.1	Mobile Terminated SMS

The procedure for Mobile Terminated SMS (MT SMS) in a satellite network for S&F operation is depicted in Figure 6.23.3-1. The steps enclosed in dashed line boxes are executed as atomic procedures.

Figure 6.23.3.1-1: MT SMS in satellite network for S&F operation

1-3.	When the SC / SMS-GMSC has an MT SMS for the UE, the SMS-GMSC interrogates the HSS to obtain routing information. The HSS response includes the address of the MME-GND. The SMS-GMS forwards the MT SMS to the MME-GND using the ForwardShortMsg message. The MME-GND stores the MT SMS. The SM Delivery Timer in the ForwardShortMsg message should be set to a value that is large enough to anticipate the delayed reception of the delivery report in step 7.

NOTE:	The SM Delivery Timer should be set to values on the order of several hours. According to TS 29.338 [20] the SM Delivery Timer is of type Unsigned32 and contains the value in seconds of the timer for SM Delivery, meaning that existing specifications allow for using large enough timer values.

4.	If MME-GND has knowledge of the approximate UE location, it determines to forward the MT SMS to the MME-SAT residing on the satellite that is the first one to fly over of the area where the UE is located. Otherwise, MME-GND selects the MME-SAT that is the first one to establish contact with MME-GND. When the ground station establishes contact with the selected MME-SAT, the MME-GND forwards the MT-SMS to MME-SAT.

5-6.	When MME-SAT determines that the UE is in satellite coverage using existing mechanisms for satellite access with discontinuous coverage, the MME-SAT establishes connection with the UE and delivers the MT SMS. The MME-SAT stores the delivery report.

7-8.	When MME-SAT subsequently establishes contact with the ground station, the MME-SAT sends the delivery report to the SC / SMS-GMSC (via MME-GND). The SMS-GMSC may inform the HSS of the delivery report status as defined in clause 10.1 of TS 23.040 [19].

#### 6.23.3.2	Mobile Originated SMS

The procedure for Mobile Originated SMS (MO SMS) in a satellite network for S&F operation is depicted in Figure 6.23.3-2. The steps enclosed in dashed line boxes are executed as atomic procedures.

Figure 6.23.3.1-1: MO SMS in satellite network for S&F operation

1-2. When the UE has an MO-SMS for the network, the UE establishes a connection with the satellite after determining that it is in satellite's coverage and initiates message transfer to the MME-SAT. The MME-SAT stores the MO-SMS.

3. The MME-SAT stores the MO-SMS and immediately sends the delivery report to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

NOTE:	The solution assumes that the likelihood for the MO SMS not being delivered to the SC once the feeder link becomes available is negligible. If such an unlikely event does occur, the UE would have erroneously been indicated successful delivery of the MO SMS to the SC.

4-5. When MME-SAT establishes contact with the ground station, the MME-SAT forwards the MO-SMS to SMS-GMSC (via MME-GND). The SMS-GMSC replies with a delivery report to the MME-SAT (via MME-GND). The MME-SAT discards the delivery report as it has already sent one in step 3.

### 6.23.4	Impacts on services, entities and interfaces

MME impact:

-	Ability for split operation with MME-SAT and MME-GND component on the satellite and on the ground, respectively.

## 6.24	Solution #24: Enhancement of registration and PDU session establishment to support on-board S&F

### 6.24.1	Description

The Store and Forward (S&F) Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

To support the S&F operation in single satellite case, there could be two deployment options.

Option 1: User plane enabled S&F. Only user plane function is on-board, i.e. the UPF and optionally the EAS could be onboard. User plane on board enabled S&F can support the rapid implementation of delay-insensitive services (CIoT, etc.), and has low requirements for onboard resources. In addition, with the on-board EAS, some service could be provided on the satellite, which could save the feeder link resource, and reduce the transmission delay.

In this case, A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms for a UE to register with the network when the network is in S&F Satellite operation. A S&F registration solution is proposed and will be elaborated in the following clause.

Figure 6.24.1-1: Architecture assumption for User plane enabled S&F

Option 2: 5GC enabled S&F. Both user plane function and part of the control plane are on-board, i.e. the UPF, AMF, SMF, UDM/AUSF and optionally the EAS could be onboard. 5GC enabled S&F has requirements for onboard resources, and control signalling exchange on the satellite can save feeder link resources.

In this case, most 5GC service could be provided on-board with minor enhancement. The SMF is supposed to have the S&F policy about the S&F operation. Detailed enhancement will be elaborated in the following clause.

Figure 6.24.1-2: Architecture assumption for 5GC enabled S&F

### 6.24.2	Procedures

#### 6.24.2.1	Option 1: Procedures of User plane enabled S&F

To support mechanisms for a UE to register with the network when the network is in S&F Satellite operation, this solution proposes the UE request the registration and PDU session establishment into single procedure. Following enhancement are supposed to be supported by (R)AN.

1.	When the S&F operations are supported and indicated in the UE request, (R)AN shall store the received UE registration and PDU session establishment request. RAN is aware of the ephemeris information, by configuration.

2.	Trigger the feeder link/service link establishment when they are available. Upon the feeder link/service link is available, send out the stored UE registration and PDU session establishment request.

##### 6.24.2.1.1	Procedures of registration in S&F mode

Figure 6.24.2.1.1-1: Procedures of registration in S&F mode

1.	UE sends the registration and PDU session establishment request to (R)AN. The request message includes the AN message (AN parameters, Registration Request), the detailed can be fined in clause 4.2.2.2.2 of TS 23.502 [3]. And the AN parameters include the indicator of the S&F registration.

The indicator of the S&F registration indicates the UE support the S&F registration and wants to perform a S&F registration, and the PDU session establishment would be initiated in the same time.

Editor's note:	How to support the combination of the registration and PDU session establishment is FFS.

Editor's note:	How to support security mechanism is FFS, and need to coordinate with SA WG3.

2.	When the UE provides the S&F registration request, and the (R)AN could cover this area when the service link is available again based on the ephemeris information or based on the pre-configuration, the (R)AN stores the UE request received.

Editor's note:	How to handle the change of satellite is FFS.

3.	The (R)AN sends a S&F registration notification to the UE. The notification would inform the UE the registration is in the S&F mode, indicating the registration response will be delayed.

Instead of AMF trigger the paging, UE would be in RRC_inactive state, (R)AN store the UE context, and send the RAN Notification area and RAN Notification Area Update timer to UE. The RAN Notification Area Update timer is alignment with the ephemeris information, i.e. UE will send the periodic RAN Notification Area Update to (R)AN at the time that the service link is available again.

Editor's note:	How to handle the case that (R)AN should store the UE context while the security procedure has not complete between the UE and 5GC.

4.	Based on the ephemeris information, or the pre-configuration, (R)AN sends out the stored request upon the feeder link is available.

5.	Complete the registration procedures and PDU session establishment procedures on the ground as defined in clause 4.2.2.2 step 2-19 and clause 4.3.2.2 step 2-12. of TS 23.502 [3].

With the indication of the S&F registration, the AMF is aware that the UE is in S&F mode, then the AMF may, based on configuration, send the satellite ID to the SMF. The SMF selects on-board UPFs based on satellite ID, ephemeris information. The SMF activates the on-board UPF buffering in the N4 session signaling.

Editor' s note: How the SMF selects on-board UPFs based on satellite ID, ephemeris information is FFS.

6.	The AMF sends the registration accept and PDU Session Establishment Accept to the (R)AN. The (R)AN stores the information received from AMF that will send to the UE. The (R)AN stores the UE context and acknowledges to the AMF.

7.	When the timer is triggered, which means the service link is available, to make UE trigger the periodic RAN Notification Area Update procedure to connect with (R)AN. the (R)AN send out the stored Registration Accept information and PDU Session Establishment Accept information to UE. And (R)AN forwards the NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept)) provided in step 12 to the UE.

8.	UE sends the N2 PDU session response to the (R)AN, (R)AN stores this signal, and will send it to the 5GC when the feeder link is available.

9.	Optionally the Uplink Data will be stored in the on-board UPF, wait for the next service link available time.

Editor's note:	How to handle the stored data after the PDU session establishment procedure is FFS.

Editor's note:	How to handle the NAS timer to support the above procedure depends on CT WG1 decision.

#### 6.24.2.2	Option 2: Procedures of 5GC enabled S&F

##### 6.24.2.2.1	Enhancement of the registration procedures

When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in clause 4.2.2.2 of TS 23.502 [3]. The differences are:

-	The step 16 could be skipped, since the AM policy could be predefined.

-	After the on-board registration completion, the on-board UDM may send the stored UE subscribe data to the ground UDM for backup.

-	The capability of NF(s) supporting the S&F mode need to be indicated.

Editor's note:	How the subscription data synchronization among the UDMs in the satellite and ground is achieved is FFS.

##### 6.24.2.2.1	Enhancement of the session management procedures

When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in clause 4.2.2.2 of TS 23.502 [3]. The differences are:

-	The step 7 and/or step 9 could be skipped, since the SM policy could be predefined.

-	The capability of NF supporting the S&F need to be indicated.

### 6.24.3	Impacts to Services, Entities and Interfaces

User plane on-board:

-	SMF: selection of UPF based on the satellite ID and ephemeris information.

-	(R)AN: Store the request from the UE in S&F mode.

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

Control plane and user plane on-board:

-	UDM: subscription data synchronization

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

## 6.25	Solution #25: UE reachability estimation

### 6.25.1	Key Issue mapping

This solution aims to the resolve the Key Issue #2, "Support of Store and Forward Satellite operation".

### 6.25.2	Description

The URE is a new network element located on ground responsible for estimating which non-terrestrial network element can reach the UE in shortest amount of time possible. The following figure shows a possible architecture, including the URE.

Figure 6.25.2-1: Architecture of EPC including UE Reachability Estimator

#### 6.25.3.1	UE Reachability Estimation (Satellite Location/Coverage Information)

##### 6.25.3.1.1	UE reachability estimation

The UE reachability is a function of both:

1.	UE location (and optionally trajectory and velocity, if available); and

2.	Satellite (MME-NT) location, velocity, and ephemeris. This includes when there is a feeder link between the ground and a SAT and when there is Uu radio connectivity between the SAT and the UE.

For a static UE, URE estimates reachability based on the satellite's ephemeris. A precise "Reachability Schedule" (RS) can be calculated. This RS precisely contains the time and duration during which a UE is reachable by the satellite through which the UE is reachable in each time duration and the latest time this satellite has reached the terrestrial core network before covering the UE.

For UEs in motion, if the trajectory and velocity of the UE are known, the URE takes these into account when constructing the RS. In this case, the RS may include reachability probabilities with each entry. The RS can also be more precisely constructed if the UE trip plan is known to the URE.

##### 6.25.3.1.2	URE and MME-T interaction and use-cases

An MME-T that would like to know when a given UE is reachable interrogates the URE for this UE. The URE responds with the earliest possible "UE Reachability Time", "UE Reachability Duration" and candidate Satellites (MME-NT). There are several use cases where an MME-T may interrogate the URE, for example:

1.	To set a timer for a NAS procedure of a satellite-served UE,

2.	To determine when to send messages (signalling, data, SMS) to the UE, or

3.	During NAS procedures, the MME-T may take "UE Reachability Time" into account when assigning the NAS timer values (such as Periodic Update Timer and/or T3510 attach timer).

As a response to a MME-T's interrogation, the URE replies with the earliest possible "UE Reachability Time" and "UE Reachability Duration". As an option, more than one alternative might be provided, e.g. "UE Reachability Time 1" and "UE Reachability Duration 1" via satellite 1, "UE Reachability Time 2" and "UE Reachability Duration 2" via satellite 2, etc.

Based on network configuration/policies and the interrogator MME-T, the URE may include additional information in its response as the SAT-ID and/or the IDs of some or all MME-NTs serving the UE during the provided reachability time and duration.

##### 6.25.3.1.3	Reachability Modes

The information provided in the URE response to the terrestrial network element varies depending on a selected "Reachability Mode" (RM). The RM determines how the UE is reachable; for example, "Same SAT Mode" (SSM), "Any SAT Mode" (ASM) or "Fastest Reachable Mode" (FRM) (via inter-SAT link).

"Same SAT Mode" (SSM) assumes that the same satellite that has received a request from a UE would be the same satellite to reply to the UE. It can also be assumed that the same satellite through which the UE registered to the network would be the satellite to use for MT signalling or MT data.

The "Any SAT Mode" (ASM), assumes that the UE will be reached by the first satellite that can cover this UE and that will have a feeder link to the terrestrial core network between the time of interrogation and before the satellite covers the UE.

The "Fastest Reachable Mode" assumes the UE needs to be reached and delivered the message as soon as possible.

Query parameters from MME-T towards URE:

-	UE identity.

-	Optional: UE location (e.g. TAI of static UE or UE location coordinates using e.g. GNSS).

-	Reachability Mode(s).

-	Optional: expected UE travel route if available.

-	Optional: satellite constellation ID or satellite operator ID (if not provided, a default constellation (configured) to be assumed).

-	Storage quota per constellation/per satellite/geography/PLMN.

If the user location is not available in the request to the URE, the URE service needs to determine the user location, e.g. by querying the HSS, the HSS may provide an optional satellite constellation ID or satellite operator Id if the serving MME of the UE has registered this information onto HSS.

Assuming the last known TAI: registration area (set of TAI(s)) and gets also registered in HSS, for UEs registered via a S&F NTN RAT -Radio Access Type- (this may correspond to UE(s) with specific subscriptions that may benefit from satellite with S&F feature), the URE gets the location information from HSS.

Otherwise, the URE may need to fetch location information from MME. This registration area will help in getting constellation/satellite information that is currently covering that region. The MME-T can also provide expected UE behaviour information. URE can take that into consideration to assume UE's current location.

The URE based on the satellite constellation ID or satellite operator ID, on the UE location (last known TA/ registration area) and on its knowledge of the satellite ephemeris determines:

1.	The time and duration (time window Tw1) at which MME-NTs are capable of serving a given UE is reachable (contactable via feeder link).

2.	A subsequent time window (Tw2) through which the UE is reachable by these MME-NTs (when the UE has satellite coverage).

NOTE:	The URE can be used for ground network element to find suitable satellite. There is no restriction on number of core network elements provisioned on-board the satellite. The MME-NT is shown as an example in above case.

Editor's note:	How does the URE obtains the UE trajectory or velocity is FFS.

Editor's note:	How the ephemeris data on satellite movement is provisioned in URE is FFS.

### 6.25.4	Impacts on services, entities and interfaces

MME-T:

-	New interface towards URE to fetch UE reachability estimates.

HSS:

-	Optional interface from URE to HSS for retrieving last known location.

## 6.26	Solution #26: Provision of Store and Forward Satellite Specific Parameters

### 6.26.1	Description

This solution considers the following key issue:

-	Key Issue #2: Support of Store and Forward Satellite operation.

Especially, it focuses on the provisioning procedure needed before performing S&F Satellite operation

In the existing 5GS, the provisioning capability allows an external party to provision the information, such as expected UE behaviour and service specific parameters. The service specific information consists of information to support the specific service in 5G system. Provisioned data can be used by the other NFs.

This solution reuses this provisioning capability for Store and Forward Satellite Specific Service, and defines the specific parameters for 5G system.

The Store and Forward Satellite Specific Parameters can be the parameters provided from an AF (of the operator or a trusted 3rd party) by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2 of TS 23.502 [3].

Editor's note:	Whether and what parameters to expose as Store and Forward Satellite Specific Parameters is FFS.

To minimize the existing system impacts, EPS can include or update these kinds of parameters as the subscription information, based on SLA with a trusted 3rd party.

Provisioned Store and Forward Satellite Specific Parameters can be used by the other NFs such as SMF, PCF, etc. for example, being forwarded as subscription information, and considered for the policy generation and enforcement.

### 6.26.2	Procedures

Figure 6.26.2-1 shows the overall procedure for Provision of Store and Forward Satellite Specific Parameters.

Figure 6.26.2-1: Overall Procedure for Provision of Store and Forward Satellite Specific Parameters

1.	The AF provides one or more parameter(s) to be created or updated, or deleted in a Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update or Nnef_ParameterProvision_Delete Request to the NEF. The payload of the Nnef_ParameterProvision_Update Request may include Store and Forward Satellite Specific Parameters.

2.	If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store, or delete the provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create, Nudm_ParameterProvision_Update or Nudm_ParameterProvision_Delete Request message, the message includes the provisioned data and NEF reference ID.

NOTE:	The remaining steps of the procedures are based on Figure 4.15.6.2-1 of TS 23.502 [3].

### 6.26.3	Impacts on services, entities and interfaces

NEF and UDM service operations:

-	Adding new parameter "Store and Forward Satellite Specific Parameters" as Input parameter of Nnef_ParameterProvision and Nudm_ParameterProvision.

## 6.27	Solution #27: Performance optimization for the transport of MT/MO S&F traffic

### 6.27.1	Description

This solution targets partially for the KI#2, i.e. S&F satellite operation, revolving around how to optimize the transport of both MT (Downlink) and MO (Uplink) traffic, including data and signalling, under the scenario of intermittent feeder link, for NR NTN case. Its main objective lies at addressing the potential overloading due to the exodus of stored messages (once service-link and/or feeder-link re-gain(s) connectivity). This solution proposal can be complementary to other S&F solution enhancements.

Specifically, it addresses:

-	The minimum necessary set of core network functions that would be on-board satellite(s);

-	The enhancement on the UE and network procedures to support S&F.

According to the definition of 'S&F satellite operation' in the present document, the S&F service will handle both UL (MO) and DL (MT) traffic. While the feeder link is unavailable, the MO suggests the on-board (satellite) storage of UL messages, and the MT indicates the on-ground storage of DL messages. However, for MT service, even if MT-traffic can be sent from on-ground CN to on-board satellite RAN, the potentially unavailable service link may prevent such MT traffic from being delivered further to UE. Therefore, there should be existing some network function(s) on-board satellite that would be able to store the MT traffic also.

Further, even if the MO and MT S&F services have different requirements, there should be an integrated S&F-based deployment architecture that could handle holistically both MO and MT services.

#### 6.27.1.1	S&F Deployment Architecture

The deployment architecture for the proposed solution is shown in the Figure 6.27.1.1. The proposed deployment scheme also answers the item: "The minimum necessary set of core network functions that would be on-board satellite(s)".

In the diagram, we add one I-UPF on-board satellite. Note that it is the only core network function that we propose will be on-board satellite. The 5GC (with PSA-UPF) are on-ground. This I-UPF and the corresponding gNB can be deployed on the same or different satellites. If I-UPF & gNB are on different satellites, according to the architectural assumptions & principles in the present document, the impact of the ISL between their respective satellites is insignificant.

Figure 6.27.1.1: S&F Deployment Architecture

As we have elucidated in the clause 6.27.1, an integrated solution architecture should be used to handle both MT and MO services together. If the I-UPF is not deployed, upon the unavailability of the feeder link, while the MT service could have MT traffic stored on the ground-UPF, there does not have an effective network element to store the MO traffic.

NOTE:	An alternative way is to have MO traffic stored in on-board gNB. In this scenario, we have to either coordinate with RAN WGs or add external network elements/functionalities to handle it. This will unavoidably drag the timeline of this SID.

With the introduction of an on-board I-UPF, the MO traffic could be stored here, and then forwarded once the feeder link (between the satellite with the I-UPF on-board and the ground-station) is available. Other advantages might be:

1)	For the MT service, if the feeder link is available but the service link is unavailable (rare but just in case for any reason), then MT traffic could be on-board stored here temporarily waiting for later forwarding;

2)	Since we have both (on-board) I-UPF and (on-ground) PSA-UPF, the performance optimization algorithm we are going to explain later would be consistent.

#### 6.27.1.2	Performance impact from the accumulation of stored MT/MO traffic

When the feeder link is out of service, the S&F Satellite operation stores MO traffic from UEs on-board satellite(s), and simultaneously MT traffic from DN on the ground.

Figure 6.27.1.2: Deployment Sites of Performance Optimization

As our solution in the clause 6.27.1.1 has proposed to deploy an I-UPF on-board satellite and the PSA-UPF on the ground, the Figure 6.27.1.2 shows different S&F cases for both MT (Downlink) and MO (Uplink) traffic:

-	MT: When MT traffic reaches the (on-ground) PSA UPF, if the feeder link is unavailable, it will be stored there (i.e. marked as 'D1' in the figure). And, if the feeder link is available, but the service link (to UE) is unavailable, the MT-traffic can be stored on the (on-board) I-UPF (i.e. marked as 'D2' in the figure);

-	MO: When the feeder link is unavailable, MO-traffic will be stored on the (on-board) I-UPF (i.e. marked as 'U1' in the figure).

As we can see, once the feeder link (or the service link for MT) is available, the accumulated MO/MT traffic, temporarily stored either on-board or on-ground, will be delivered immediately to their respective destinations. If the volume of the accumulated (MO and/or MT) traffic is large, this kind of exodus will unavoidably cause overload performance impact to the network, e.g. at locations 'D1, D2 and U1'.

#### 6.27.1.3	Performance optimization to forward accumulated MT/MO traffic

At the first glance, since the Priority Level associated with 5G QoS indicates a priority in scheduling resources among QoS flows, it could be a good candidate to mitigate the potential overload impact. Further, a corresponding S&F forwarding priority could also be defined/derived based on the 'Priority-Level' parameter. However, the challenges are:

-	The number of priority levels is limited, which means that lots of stored messages will share the same priority. This undermines the granularity to prioritize differentiated levels of stored messages;

-	Even on the same priority level, when the unavailable duration of a feeder link is sufficiently long, there might be quite a lot accumulation of S&F messages (thanks to the few of priority levels). Once the feeder link is available again, all those accumulated messages (supposed to be on the same priority level) will compete immediately and also equally for resources to transport stored MT/MO traffic, which introduces inevitably the overload performance impact.

Therefore, we proposed in this solution draft an optimization algorithm to tackle the performance enhancements of the S&F Satellite Operation. This draft also answers the item, i.e. the enhancement on the UE and network procedures to support S&F, from the KI#2.

In principle, our proposal intends to utilize both the existing 5G QoS characteristics as well as the satellite related work that has been standardized in 5G documents (TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4]). Our objectives are to:

-	achieve prioritized transmission among stored (MO/MT) traffic;

-	mitigate the potential network overloading;

-	avoid the starvation of lower priority-level QoS flow; and

-	distribute the algorithmic control logics, etc.

Figure 6.27.1.3: Performance Optimization for accumulated MT/MO delivery

The optimization scheme is shown in the Figure 6.27.1.3. The main idea of the algorithm stems from the availability duration of a feeder link once it is available, which can be, in our assumption, derived from the satellite ephemeris (e.g. period, height, elevation angle, etc.), similar to the satellite work in TS 23.501 [2]. In the figure, the time Tb is the moment when the feeder link of a satellite is available again (from unavailability period), and the time Te is the moment the same feeder link is projected to go unavailable. The difference (Tb-Te) is the whole feeder link availability duration.

For the QoS flow on the ith highest priority level,

Ti = Time [satellite-ephemeris, QoS flow priority-level, offsets]

where 'offsets' are optional factors, like message arrival time, message maximum latency, etc.

Then, (Ti-Tb) is the max time-range over which all currently stored traffic (MO or MT) belonging to a QoS flow should be forwarded. Here, the function Time to compute Ti will make sure the higher the priority-level of a QoS flow, the lower the Ti, and thus the shorter the time range (Ti-Tb). For example, as shown in the Figure 6.27.1.3, the 1st-hightest priority-level renders the shortest time T1, the 2nd -highest has the 2nd shortest T2, and so on. The lowest priority-level spans almost the whole feeder-link availability duration.

Let the priority-based max time range (PBR):		PBRi = Ti - Tb.

Then, for the kth stored message from a QoS flow on the ith highest priority level, its forwarding time

FTk = OMF [PBRi, mesg-order-index(k), total number of stored messages]

Where: OMF stands for 'Overload Mitigation Function'; the 'total number of stored messages' indicates the complete set of currently stored messages of the QoS flow.

NOTE:	The function OMF is implementation specific, which might be dependent on an operator's local policy. For example, OMF could be comprised of the application of randomization, space-out, and more.

This algorithm can help alleviate the overload impact and achieve all the objectives we set forth above:

-	Achieve prioritized transmission among stored messages:

-	[Explanation] The computation function of Ti warrants the shorter duration for a higher priority level. So, when we apply the randomization and space-out to stored messages, the stored messages from a higher-priority QoS flow tend to be, statistically, forwarded earlier than those from a lower-priority flow.

-	Mitigate the potential network overloading:

-	[Explanation] The possible application of randomization and space-out over a PBR helps avoid the exodus delivery of all stored messages from all priority levels instantly once the feeder link is available.

-	Avoid the starvation of a lower priority-level QoS flow:

-	[Explanation] As shown in the Figure 6.27.1.3, while, at each priority level, a stored message will be forwarded in accordance with the timer FTk (as shown above), it does not schedule all stored messages from a higher priority level ahead of all stored messages from a lower priority level. Thus, it avoids starvation.

The reason is because all PBRi's start from Tb, which still gives, statistically, a lower priority-level message some chance to be forwarded earlier. This also conforms to the case in reality. For example, if all network links are in services and resources are available when a lower-priority message reaches a UPF, it will be scheduled to forward even if a higher-priority message arrives later.

-	Distribute the algorithmic control logics, etc.

-	[Explanation] The computation of the timer FTk occurs at each QoS flow, which does not need a centralized controller.

### 6.27.2	Procedures

Precondition: UE has successfully registered with the 5G network.

#### 6.27.2.1	General procedure for performance optimization

Figure 6.27.2.1: High-level Procedure for Overloading Mitigation

0a.	An AF function sends the satellite ephemeris data to NEF (untrusted) or to PCF (trusted), from which PCF generates PCC rule and sends to SMF.

NOTE 1:	The provision of satellite ephemeris data could reference how the SCAI, i.e. satellite coverage availability information, is provisioned in TS 23.501 [2].

NOTE 2:	PCF can accordingly determine the feeder link availability duration, i.e. the time Tb and Te, based on the satellite ephemeris data.

0b.	SMF sends PCC rule to UPF/I-UPF during session establishment/update.

NOTE 3:	Our solution proposal designates an (I-)UPF would be the NF to store & forward MO/MT traffic.

1.	A feeder link is available (again) and the S&F service starts the forwarding operation logic.

2a.	(I-)UPF derives the max time-range for each priority level.

2b.	(I-)UPF determines the forwarding timer (FTk) for each stored message per QoS flow per priority level.

3.	The forwarding timer (FTk) of a stored message starts.

4.	The forwarding timer (FTk) (of a stored message) expires and (I-)UPF starts forwarding the corresponding MO/MT traffic accordingly.

5.	Repeat steps 2b, 3 and 4, until (I-)UPF finishes forwarding all stored MO/MT traffic.

Editor's note:	How the I-UPF knows when the RAN is ready for DL forwarding is FFS.

#### 6.27.2.2	Enhanced procedure for S&F UE Registration

This procedural enhancement is for a UE supporting the satellite S&F service to register with the 5G system. The objective is, upon UE registration, for UEs and 5GS to negotiate the S&F operation support. The enhancement is supplementary to the procedure as defined in clause 4.2.2.2.2 of TS 23.502 [3].

The enhanced steps will not have a material impact to the overloading performance optimization. We only list here for the control-plane completeness. The following figure is pretty much self-explanatory.

Figure 6.27.2.2-1

#### 6.27.2.3	Enhanced procedure for S&F Service Request

This procedural enhancement is for the service request between UEs and 5GS that support the S&F satellite service. The objective is for 5GC to select (I-)UPFs that support S&F service. The enhancement is supplementary to the procedures as defined in clauses 4.2.3.2 and 4.2.3.3 of TS 23.502 [3].

Similar to clause 6.27.2.2, the enhanced steps will not have a material impact to the overloading performance optimization. We only list here for the control-plane completeness. The following figure is pretty much self-explanatory.

Figure 6.27.2.3-1

### 6.27.3	Impacts on services, entities and interfaces

AF:

-	Send satellite ephemeris data, including the starting coverage time and the ending time of a satellite for a serving area.

PCF:

-	Receive the satellite ephemeris data.

-	Determine feeder link availability duration (e.g. Tb and Te).

-	Generate PCC rule.

SMF:

-	Receive & send the PCC rule (containing the feeder link availability duration info).

(I-)UPF:

-	Receive the PCC rule (containing the feeder link availability duration info).

-	Detect the in-service (i.e. available) time of a feeder link.

-	Generate the max time-range for each priority level (i.e. PBRi = Ti - Tb).

-	Calculate the forward timer FTk for each stored S&F message, based on PBRi, message-order-index, etc.

-	Run the FTk timer, and forward out the corresponding message when FTk expires.

## 6.28	Solution #28: UE-Satellite-UE Communication via UPFs onboard the satellites

### 6.28.1	Description

This solution corresponds to KI#3 Support of UE-satellite-UE communication.

Assumptions of the solution:

-	gNB and UPF are the necessary nodes to be deployed on the satellites.

-	During the call setup procedure, the IMS system will determine to use UE-Satellite-UE communication. The media plane node of the IMS system, i.e. IMS AGW is deployed on the ground, which means the media between MO UE and MT UE will bypass the IMS AGW and IMS LI is not supported. This further means that UE-Satellite-UE communication is possible when transcoding is not needed.

-	Both MO UE and MT UE are served by the same HPLMN.

-	Both MO UE and MT UE are service by the same SMF or I/V-SMF instance. A dedicated SMF (or I/V-SMF) that supports UE-SAT-UE communication is used based on network configuration.

#### 6.28.1.1	Reference Architecture

The examples of network architecture are listed in the Annex C. For this solution, gNBs and UPFs are deployed on the satellites, and the diagram is shown in the Figure 6.28.1-1.

Figure 6.28.1-1: Network architecture

UE1 and UE2 can be served by a same satellite or different satellites. In this figure, the two UEs are served by Satellite 1 and Satellite 2 respectively. The two serving satellites are connected with ISL.

The signalling of the UEs is transferred to/from the ground 5GC and IMS networks via the nodes, i.e. gNBs and UPFs, on the satellite. Once UE-Satellite-UE (UE-SAT-UE) communication is used, the user plane traffic between the UE1 and UE2 will be routed by the satellites only without going through the ground networks.

#### 6.28.1.2	Identifying UEs applicable for UE-Satellite-UE Communication

During the call setup procedure, the IMS system is able to trigger the UE-SAT-UE communication based on NR satellite access type, the UEs' codec capabilities, operator policy and UEs' subscription data. The subscription data may be Cx/Dx data which are validated by S-CSCF, or Sh data validated by AS.

Once the IMS system determines to use UE-SAT-UE communication, P-CSCF should notify the 5GC system during the dedicated QoS flow creation or update procedures by indicating IP addresses of MO UE and MT UE.

#### 6.28.1.3	UL CL/BP/local PSA UPF selection for UE-Satellite-UE Communication

To enable UE-satellite-UE communication using NGSO satellite constellation, UL CL/BP/local PSA UPF located on the satellite needs to be selected to establish PDU Session for each involved UE.

The MO UE and MT UE may select two different UPFs as UL CL/BP/local PSA UPF respectively which deployed on different satellites. In this case, ISL is used for traffic between the two satellites.

The gNB and UPF can be deployed on different satellite, as depict in Figure 6.28.1-2:

Figure 6.28.1-2: UE-satellite-UE communication with gNB and UPF on board

The satellites in the same constellation can be deployed into several groups. Each group has a satellite with UPF on board which can provide connections to several other satellites with gNB on-board through ISL. Each group can be depicted by a satellite association information which can be preconfigured at SMF. The satellite association information indicates the relationship of gNB on-board satellite ID, UPF on-board satellite ID and DNAI that can be accessed through the UPF. For example, the satellite constellation can be deployed into two groups. One is {satellite 1-1/gNB, satellite 1-2/gNB, satellite 1-3/UPF, DNAI-1, DNAI-2}, which means gNB on satellite 1-1, 1-2 can connect to the UPF on satellite 1-3 through ISL and DNAI-1, DNAI-2 can be accessed via the UPF. Another is {satellite 2-1/gNB, satellite 2-2/gNB, satellite 2-3/UPF, DNAI-1}, which means gNB on satellite 2-1, 2-2 can connect to the UPF on satellite 2-3 through ISL and DNAI-1 can be accessed via the UPF.

Figure 6.28.1-3: Satellite Group Association

The UL CL/BP/local PSA UPF selection can be based on satellite association information. The SMF performs UL CL/BP/local PSA UPF selection and insertion during PDU session establishment procedure or PDU session modification procedure based on gNB on-board Satellite ID provided by the AMF, which includes:

-	Based on configuration, the AMF may determine gNB on-board satellite ID serving the UE and send it to the SMF.

-	The SMF determines the DNAI, UPF on-board satellite ID based on the gNB on-board satellite ID, satellite association information, DNN provided by UE.

-	The SMF selects UL CL/BP/local PSA based on the DNAI corresponding to UPF on-board satellite ID.

### 6.28.2	Procedures

#### 6.28.2.1	UE-satellite-UE communication procedure in IMS system

Figure 6.28.2-1 depicts the signaling flows of the solution.

Figure 6.28.2.1-1: UE-SAT-UE communication procedure in IMS system

In this figure, P-CSCF1, S-CSCF and AS1 are the serving IMS nodes of UE1, and UE2's serving nodes are P-CSCF2, S-CSCF2 and AS2. Other nodes like I-CSCF and HSS are not shown for brevity purpose. The two UEs may be served by same nodes.

1.	UE1 starts a call by sending SIP Invite message.

2.	The SIP Invite message is transferred to S-CSCF1 and AS1 by P-CSCF1. P-CSCF1 shall include the UE1's SDP information (e.g. IP, port, codecs) in the SIP message.

3.	(Optional) S-CSCF1 verifies UE1's subscription data and codec information to check if the UE-SAT-UE communication is allowed and possible. If the UE-SAT-UE communication is subscribed, it means that UE's media traffic is allowed to be delivered to the remote UE without going through the IMS AGW. Then S-CSCF adds an UE-SAT-UE communication indication in the SIP message.

The subscription data may also be verified by AS. In this case, the subscription information (i.e. UE-SAT-UE communication) is contained in the transparent subscription data and handled by AS1.

4.	The SIP Invite message is routed to terminating nodes S-CSCF2 and AS2. The UE-SAT-UE communication indication and UE1's SDP information received in step 2 are included.

5.	(Optional) The S-CSCF2 and AS2 may also make the verification of the subscription data for UE-SAT-UE communication.

6.	If the S-CSCF2 and AS2 also support the UE-SAT-UE communication service, the S-CSCF2 and AS2 delivers the UE-SAT-UE indication, UE1's SDP information to P-CSCF2 in the SIP invite message.

7.	The SIP Invite message is sent to UE2.8.	UE2 returns SIP 183 message to the network with its own SDP information.

9.	P-CSCF2 checks the codecs of the two UEs. If transcoding is not necessary, the P-CSCF triggers dedicated resource allocation procedure by sending AAR or Policy Authorization request message via Rx/N5 interface. The UE-SAT-UE communication indication and UE1's IP address are included.

10.	P-CSCF2 sends the SIP 183 message to S-CSCF2 and AS2. The UE-SAT-UE communication indication and UE2's SDP information are include.

11-12.	The SIP 183 message is delivered to the originating P-CSCF1 with the UE-SAT-UE communication indication and UE2's SDP information.

13.	If the UE-SAT-UE communication indication and UE2's SDP information is received, the P-CSCF1 triggers dedicated resource allocation procedure by sending AAR or Policy Authorization request message via Rx/N5 interface. If the resource has been allocated in the previous procedure, P-CSCF will update the policy to PCF. The UE-SAT-UE indication and UE2's IP address are included.

14.	The SIP 183 message is sent to the UE1.

15.	SIP PRACK and 200 OK (PRACK) messages.

16.	UE1 sends the SIP Update message.

17.	The SIP messages exchange between UE1 and UE2 continues, and the policies may be further updated. Finally, the call is established with UE-SAT-UE communication enabled.

Editor note:	Whether and how the IMS system needs to verify the satellite constellation to obtain the inter-link information between the satellites are FFS.

#### 6.28.2.2	Ground common SMF for UE-SAT-UE communication

Figure 6.28.2.2-1 depicts the signalling flows of the solution .

Figure 6.28.2.2-1: Ground common SMF for UE-SAT-UE communication

1.	UE1 is served by gNB1 deployed on the satellite SAT1 and establishes the PDU session for IMS call, i.e. the DNN is IMS. The dedicated SMF can be discovered by AMF via NRF, and the discovery condition may include the access satellite RAT type and the ID of the satellite.

The UPF on the ground is selected as anchor UPF for the session.

2.	UE2 is served by gNB2 deployed on the satellite SAT2 and establishes the IMS PDU session, and the same serving SMF is chosen as in step 1.

3.	UE1 and UE2 register to the IMS system, and a call is initiated between the two UEs. During the call setup procedure, IMS system determines that media path could be optimized by using UE-SAT-UE communication.

For this case, IMS AGW is deployed on the ground, which means the media path will bypass the IMS AGW.

4.	P-CSCF requires the 5GS to allocate or update the resources, in which the UE-SAT-UE communication indication and the peer UE IP address are included. The ID of peer UE, i.e. SUPI or GPSI may also be included.

PCF sends the request to SMF, and includes the UE-SAT-UE indication and peer UE IP address. The peer UE ID, i.e. SUPI or GPSI may also be included.

5.	When SMF receives the session update request with the UE-SAT-UE communication indication and the peer UE IP address, SMF interprets that the UE-SAT-UE communication is required and inserts the UPF onboard the satellite as ULCL/BP and local PSA UPF. SMF choose the proper UPF according to the DNAI(s) corresponding to satellite ID. SMF may verify the UE-SAT-UE communication is allowed or not based on UE's subscription data.

In this procedure, assuming SMF firstly chooses the UPF1, which is deployed with gNB1 on satellite SAT1 as the ULCL/BP and local PSA UPF, the SMF will first request the UPF1 to allocate tunnel info for N19 forwarding.

6.	The SMF inserts UPF2 onboard the satellite as ULCL/BP and local PSA UPF, and the packet detection and forward policies are similar as that defined for 5GSATB if N19 reference point is re-used.

7.	The SMF then updates the packet forwarding policies to UPF1 for UE1 as above step 6.

8.	The media path between UE1 and UE2 is UE1 <-> SAT1(gNB1+UPF1) <-> SAT2(gNB2+UPF2) <-> UE2.

#### 6.28.2.3	Ground common I/V-SMF for UE-SAT-UE communication

Figure 6.28.2.3-1 depicts the signaling flows of the case where UE1 is in the home PLMN, UE2 is roaming (home-routed) in the UE1's PLMN.

Figure 6.28.2.3-1: Ground common I/V-SMF for UE-SAT-UE communication

1.	UE1 establishes the IMS PDU session via the gNB1 on the satellite, and the ground UPFx is selected as the PSA UPF of the session.

2.	UE2 is roaming in the UE1's PLMN and establishes the IMS PDU session via the gNB2 on the satellite, V-SMF is selected by serving AMF2 and the ground UPFy is selected in the PLMN. UPF in home PLMN is not indicated in the figure.

The two PDU sessions are established separately, therefore the two UEs may be served by same nodes of the 5GC or different nodes.

3.	UE1 and UE2 registers to the IMS network, and a call is initiated between the two UEs.

4.	The session update message is received by the UE1's serving SMF1 from P-CSCF, and UE-SAT-UE communication indication and the peer UE IP address are included.

5.	If the SMF1 detects that it doesn't serve the peer UE, it triggers the I-SMF selection procedure according to the received parameters (i.e. UE-SAT-UE communication indication and the peer UE IP address) . For this case, a dedicated SMF (i.e. I/V-SMF in the figure) that supports UE-SAT-UE communication is selected as I-SMF based on network configuration.

6.	The I/V-SMF inserts the UPF1 deployed on the serving satellite SAT1 for the IMS media session. The UPF selection can be based on the gNB ID and the satellite ID which are received from AMF in the PDU session establishment procedure.

7.	P-CSCF updates the IMS session of the UE2.

8.	UE2's serving SMF2 triggers the V-SMF re-selection and also selects the dedicated SMF as the I/V-SMF similar as the step 5.

9.	The I/V-SMF inserts the UPF2 for the UE's session, and the forward policies for UPF2 are similar with those defined in step 6 of clause 6.28.2.2.

10.	The I/V-SMF updates the detection and forward policies for UPF1 as in step 7 of clause 6.28.2.2.

11.	The media path between UE1 and UE2 communication is UE1 <-> SAT1(gNB1+UPF1) <-> SAT2(gNB2+UPF2) <-> UE2.

#### 6.28.2.4	Handover procedure when serving satellite changes

Figure 6.28.2.4-1 depicts the handover procedure when serving satellite changes.

Figure 6.28.2.3-1: Handover procedure when serving satellite changes

0a.	UE1 and UE2 is in UE-SAT-UE communication.UE1 is served by satellite S-SAT1 and UE2 is served by satellite SAT2. The media path between the UEs is UE1 <-> SAT1(S-gNB1+S-UPF1) <-> SAT2(UPF2+gNB22) <->UE2.

0b-0c.	Due to movement of UEs or the satellites, the serving satellite of UEs may changes. This procedure assumes that UE1 is be served by satellite T-SAT3, and handover to T-SAT3 from S-SAT1 happens.

NOTE:	The following procedure takes Xn based inter NG-RAN handover for example. For N2 based handover serving AMF may change, however, the serving common SMF or I/V-SMF is not changed for both cases.

1.	After the execution of handover, the target T-gNB3 sends N2 path switch request.

2.	AMF sends the Nsmf_PDUSession_UpdateSMContext Request to SMF.

3.	SMF chooses T-UPF3 in the target satellite as new I-UPF for UE-SAT-UE communication purpose according to the target gNB and the target satellite information. The AN Tunnel Info of T-gNB3, CN Tunnel Info of MT UE's UPF2 and QoS flow information of IMS media are included in the N4 session establishment request message.

4.	SMF modify the session information of the ground UPFx by updating the AN Tunnel Info of T-gNB3.

5-6.	The path switch response is sent to target gNB from SMF via AMF.

7.	Now the path for the uplink media of UE1 is updated to UE1 -> SAT3(T-gNB3+T-UPF3) -> SAT2(UPF2+gNB2) -> UE2.

8.	SMF modifies the UE2's session information to UPF2 by updating the CN Tunnel Info of UE1's target T-UPF3.

9.	SMF may start a guard timer to delay the release of the resources in source UPF to avoid possible packet loss.

10.	End Marker is sent to target T-gNB3 from UPF2 via source S-UPF1 and S-gNB1.

11.	T-gNB3 triggers the resource release to S-gNB1.

12.	The downlink media path for UE1 from UE2 is updated to: UE2 -> SAT2(gNB2+UPF2) -> SAT3(T-UPF3+T-gNB3) -> UE1.

13.	After the guard timer expires, the SMF releases the session in the source UPF.

### 6.28.3	Impacts to Services, Entities and Interfaces

This solution may have the following impacts:

PCF:

-	Support the UE-SAT-UE communication indication and target UE address.

SMF:

-	Support the UE-SAT-UE communication indication and target UE address.

-	Trigger I-SMF insertion and deletion procedure for UE-SAT-UE communication.

-	Trigger V-SMF re-selection procedure for UE-SAT-UE communication.

-	Delay release session of source UPF in the handover procedure.

-	Preconfigure satellite association information

-	Performs UL CL/BP/local PSA UPF selection based on the gNB on-board satellite ID and satellite association information.

AMF:

-	Discover a dedicated I/V-SMF for UE-SAT-UE communication.

IMS:

-	P-CSCF, S-CSCF and AS determines if UE-SAT-UE communication is allowed and support the delivery of UE-SAT-UE communication indication in the SIP messages.

-	P-CSCF sends the UE-SAT-UE communication indication and peer UE's IP address to 5GC.

-	S-CSCF or AS verifies UE's subscription data for UE-SAT-UE communication.

-	P-CSCF checks the media codecs of MO UE and MT UE to confirm whether transcoding is needed.

## 6.29	Solution #29: SMF activated UE-satellite-UE communication for IMS service

### 6.29.1	Description

IMS voice calls through the UE-satellite-UE communication link can significantly reduce the load on the feeder link and minimize end-to-end delays between UEs. In the context of onboarding RAN and UPF on a satellite, UEs within the same HPLMN will establish connections via the same satellite, i.e. RAN and UPF. In this solution the same SMF will control such connections, paving the way for a local IMS voice call switching mechanism through UE-satellite-UE communication.

This solution seeks to leverage the local switching mechanism defined in R18 (as outlined in clause 5.43.3 of TS 23.501 [2]), where the SMF is responsible for managing the local switch. It assumes that UE-1 and UE-2 are connected and controlled by the same SMF, and the SMF determines their connection through the same GEO satellite with UPF onboard based on the satellite ID reported by the AMF.

Under the same assumptions, this SMF-controlled UE-satellite-UE communication solution for IMS voice calls assumes the following:

-	The PDU sessions of both UE-1 and UE-2 for the IMS voice service are managed by the same SMF.

Editor's note:	How to select the same SMF instance for IMS both UEs is FFS.

-	The IP address assigned to the UEs for the IMS Packet Data Unit (PDU) session is IPv6, implying no Network Address Translation (NAT) involvement (no Access Gateway (AGW) in the IMS media path).

Editor's note:	How does LI is implemented in case of no AGW is deployed onboard is FFS.

During the establishment of the IMS PDU session of each UE, the SMF selects the UPF (PSA1) deployed on the ground. When UE-1 initiates an IMS voice call with UE-2, the common SMF inserts the UPF deployed on the satellite as the Uplink Control Layer (UL CL) and Packet Switching Area (PSA2) for UE-1 and UE-2 individually. The voice Real-time Transport Protocol (RTP) packets are then locally switched in the UPF deployed on the satellite, as illustrated in Figure 6.29.1-1.

Figure 6.29.1-1: IMS signalling and RTP path with only UPF onboard

### 6.29.2	Procedures

Editor's note:	How this solution can apply to roaming UEs is FFS.

Editor's note:	How does the UE-satellite-UE communication link work if the serving satellite changes or at least one UE moves out of the serving satellite's coverage is FFS.

Figure 6.29.2-1:Procedures of activation of UE-satellite-UE communication during IMS call

Pre-condition:

-	Both UE-1 and UE-2 have an IMS PDU session, and the two IMS PDU sessions share the same RAN and SMF. (based on R18 satellite backhaul mechanism).

-	The SMF knows whether a UE is in a satellite with UPF onboarding based on the satellite ID reported by AMF. (based on R18 satellite backhaul mechanism).

-	UE-1 and UE-2 have finished IMS registration.

1)	UE-1 initiates an IMS call to UE-2 via SIP Invite message with SDP offer, as described in 23.228 [9].

2-4)	The SIP Invite message is sent to UE-2 as described in 23.228 [9].

The path is: UE-1's P-CSCF-> UE-1's S-CSCF-> UE-2's S-CSCF-> UE-2's P-CSCF->UE-2.

5)	UE-2 replies 183 with SDP answer to UE-2's P-CSCF.

6)	UE-2's P-CSCF initiates the voice QoS flow setup procedure based on the SDP answer. UE-2's P-CSCF informs PCF the derived source and destination IP address via "fDescs" attribute, as described in Annex B (IMS Related P-CSCF Procedures over N5) in TS 29.514 [10].

7)	UE-2's PCF performs a SM policy association modification procedure to trigger to establish a voice QoS flow.

The PCF provides PCC rule to SMF. The PCC Rule contains the source and destination IP address, which are derived from "fDescs" attribute.

8)	If the SMF has determined that UE-2 is in a satellite with UPF deployed based on the satellite ID reported by AMF, the SMF will derive the SUPIs of UE-1 and UE-2 based to the source/destination IP address contained in the PCC rule. Then SMF further derives the satellite ID of UE-1 and UE-2 based on the derived SUPIs.

The SMF detects that UE-1 and UE-2 are under the same satellite based on the derived satellite ID and makes sure that local switching is possible e.g. same codec is used and transcoding is not needed.

Editor's note:	How SMF can detect that other conditions e.g. same codec is used and therefore transcoding is not needed is FFS.

9)	SMF inserts the UPF deployed on the satellite as UL CL and PSA2 for the IMS PDU session of UE-2 as described in clause 4.3.5.4 of TS 23.502 [3], and configures the UPF deployed on the satellite with local forwarding rules. (based on the R18 satellite backhaul mechanism).

10)	SMF updates PSA1 for downlink traffic as described in clause 4.3.5.4 of TS 23.502 [3].

11)	SMF sends N4 Session Modification to the UPF deployed on satellite to establish the voice QoS flow for UE-2.

13)-14)	The subsequence PDU session modification procedure is performed as described in clause 4.3.3.2 of TS 23.502 [3].

15)	The SMF notifies the PCF that the PCC decision has been enforced by performing an SMF-initiated SM Policy Association Modification procedure as described in clause 4.3.3.2 of TS 23.502 [3].

16)	The PCF notifies the P-CSCF that the voice QoS flow has been established.

After step 6, the voice QoS flow for UE-2 is established.

17)-18)	The 183 with the SDP answer is sent to UE-1's P-CSCF

The path is: UE-2's P-CSCF-> UE-2's S-CSCF-> UE-1's S-CSCF-> UE-1's P-CSCF.

19)	The step 6-16 is performed again for UE-1

After Step 19, The voice QoS flow for UE-1 is established.

20)	UE-1's P-CSCF sends the 183 with SDP answer to UE-1.

21)	UE-2 sends 180 Ringing response to UE-1, and start ringing to alert the user.

22)	UE-2 sends 200 OK to UE-1 when the user of UE-2 answer the IMS voice call.

The voice RTP packets are locally switched by the UPF deployed on satellite based on the local forwarding rules received in steps 9 and 19.

### 6.29.3	Impacts to Services, Entities and Interfaces

SMF:

-	Derives UE-1 and UE-2's IDs based on source/destination IP address and further derives the satellite ID of UE-1 and UE-2.

-	Detects/verifies UE-1 and UE-2 are under same satellite coverage based on UE-1 and UE-2's satellite ID.

-	Inserts the UPF deployed on satellite (collocated with RAN onboard) as UL CL and PSA2

## 6.30	Solution #30: Solution to determine UE-SATs-UE eligibility and enable communication setup.

### 6.30.1	Description

The proposed solution complies with all architectural assumptions and principles of the present document as well as network scenarios for UE-Sat-UE generic architecture (regenerative payload capabilities on LEO satellite), as described in Annex C.

It evaluates eligibility necessary condition for realizing UE-satellite-UE communication for UEs, with impacts on IMS procedures and functions, and provides information to 5GC to route user plane in satellite(s), as enounced in part of KI#3.

Solution is based on information provided in the "P-Access-Network-Info" SIP header field. This field will be used by the UEs with access-type value equals "3GPP-NR-SAT" in this case where a UE is connected to RAN via satellite access

As defined in clause 7.2A.4 of TS 24.229 [11]: "P-Access-Network-Info header field":

24)	if the access-type field is equal to "3GPP-NR-SAT", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

As defined in TS 38.413 [12]: NR Cell Identity comprises a 36-bit value consisting of two parts: the gNodeB ID (22-32 bit) and the Sector ID (4-14 bits).

With such information, the P-Access-Network-Info header field enables to make the link between radio cell/gNodeB and the hosting satellite in case of regenerative satellite with gNodeB on board.

As defined in RFC 7976 [16]: "P-Access-Network-Info header field can appear in all SIP methods and non-100 responses, except in CANCEL methods, CANCEL responses, and ACK methods triggered by non-2xx responses."

By the way, the caller and called UEs can introduce P-Access-Network-Info header in IMS SIP communication setup exchanges with fresh radio cell and gNodeB information and any SIP server (S-CSCF in the example) participating to the communication setup will be at a given moment of the SIP exchanges in possession of serving radio cells and gNodeBs for the caller and called UEs.

The SIP server will then interrogate a satellite connectivity service, which monitors in real time satellite constellation motion and satellites interconnectivity, providing serving radio cells and gNodeBs for the caller and called UEs.

Editor note:	The need to standardize interface between xCSCF and satellite connectivity service is FFS.

The satellite connectivity service will map this information onto satellites of the constellation in real time and determine if satellites hosting radio cells and gNodeBs for the caller and called UEs are connected via ISL.

If it is the case, then local routing of user plane in satellite or between satellites is possible, and SIP server will indicate to 5GC, i.e.: to SMF via PCF (Rx/N5 interface), the IP addresses for the 2UEs.

Editor note:	Whether and how the PCF/SMF nodes configure local routing for user plane on the satellites is FFS.

NOTE 1:	In the restriction of non-roaming case, it is assumed that a single IMS network is in charge to establish the UE-SATs-UE communication and for sake of clarity a single S-CSCFx is represented but the solution can be used in case of different CSCFs.

NOTE 2:	Such satellite connectivity service specification is out of 3GPP scope.

NOTE 3:	Solution applies to scenario where local switching of user plane on board of satellite is done at UPF level, not at IMS-AGW level.

### 6.30.2	Procedures

The main steps for the procedure are the following:

Figure 6.30.2.1: determine UE-SATs-UE eligibility and enable communication setup.

1.	When establishing a communication, the Caller UE inserts the field "P_Access_Network_Info" in a SIP message, e.g.: INVITE, indicating gNodeB identifier corresponding to its serving satellite together with serving Cell ID.

2.	IMS core network SIP server (e.g.: S-CSCF) stores CellId_A and gNodeBId_A.

3.	The signalling message is routed by IMS core network to the called UE.

4.	The Called UE inserts the field "P_Access_Network_Info" in a SIP message, e.g.: RINGING, indicating gNodeB identifier corresponding to its serving satellite together with serving Cell ID.

5.	IMS core network SIP server (e.g.: S-CSCF) stores CellId_B and gNodeBId_B.

6.	IMS core network SIP server (e.g.: S-CSCF) interrogates a satellite connectivity service, which monitors in real time satellite constellation motion and satellites interconnectivity, providing CellId_A, gNodeBId_A, CellId_B, gNodeBId_B.

7.	The satellite connectivity service monitors in real time cells and gNodeB over the satellites constellation and checks if the 2 UEs are connected to cells of the same satellite or with interconnected satellites (through ISL).

8.	If yes, IMS core network SIP server (e.g.: S-CSCF) provides to SMF via PCF (Rx/N5 interface) any available information that may help to establish a local switching on user plane on the satellite(s) (e.g. IP addresses), for both UEs A and B).

9.	SMF configures satellites local routing of user plane connections.

10.	Session establishment continues with local routing of user plane in satellites.

11.	If no, session establishment continues with user plane going to ground.

Editor note:	Whether and how the PCF/SMF nodes configure local routing for user plane on the satellites is FFS.

### 6.30.3	Impacts on existing nodes and functionalities

UE:

-	Provide P-Access-Network-Info header with fresh NR Cell Identity value.

xCSCF:

-	Collect NR Cell Identity and IP address for the caller and the called UEs.

-	Interrogate a satellite connectivity service, which monitors in real time satellite constellation motion and satellites interconnectivity, providing serving radio cell and gNodeB for the caller and called UEs.

-	provide UEs IP addresses to PCF via Rx/N5 interface.

## 6.31	Solution #31: UE-satellite-UE communication for IMS voice and video (UPF onboard)

### 6.31.1	Description

This solution resolves Key Issue #3: Support of UE-satellite-UE communication and focus on IMS voice and video service.

This solution based on the following assumptions:

-	the PSA UPF anchored by IMS PDU session is on the ground and the UPF acting as UL CL/BP/local PSA to be deployed onboard satellite.

-	All the IMS components are on the ground.

-	The call parties (UE-A and UE-B) are served by the same PCF and the same SMF.

-	The call parties are under the same satellite (or the different satellites belonging to the same constellation if ISL is deployed and has suitable capabilities).

-	The call parties can either be roaming (with LBO) or non-roaming UEs.

Figure 6.31.1-1: Basic architecture for IMS voice and video service UE-satellite-UE communication with UPF local switching (example of the call parties are under the same satellite)

### 6.31.2	Procedures

Figure 6.31.2-1: Procedure of UE-satellite-UE communication for IMS voice and video (UPF onboard local switching)

1.	During a call session setup process, the IMS component P-CSCF determines that local switching in the UPF onboard is possible. To determine this, the P-CSCF takes into account whether the UE is accessing network by satellite, operator policies and other conditions such as whether media transcoding is required, whether home routing is requested for the session in roaming cases.

2.	The P-CSCF includes the identifications of call parties in the policy request towards the 5GC.

3.	If the 5GC determines the peer UE is also served by the same PCF and the same SMF, the 5GC configures UPF onboard local switching. Otherwise, this step is omitted and the 5GC informs P-CSCF the UPF onboard local switching is impossible in step 4.

Editor's note:	It is FFS how the network determines that the same PCF is selected for the two UE's PDU Sessions.

Editor's note:	It is FFS how the network determines that the same SMF is selected for the two UE's PDU Sessions.

4.	The 5GC sends the result of UPF onboard local switching configuration to the IMS.

5.	Other steps to complete the call session setup process as defined in clause 5.6 of TS 23.228 [9].

The media is local switched on the satellite via the local PSA UPF onboard.

Editor's note:	How the serving PLMN performs lawful interception when the traffic is local switched on satellite is FFS.

Editor's note:	Whether the use of local switching needs to be included in the charging information is FFS.

Editor's note:	It is FFS how a change of satellite is handled.

### 6.31.3	Impacts on services, entities and interfaces

IMS:

-	The P-CSCF determines whether UPF local switching is possible when the UEs involved in the communication service with each other are accessing network via the satellite.

-	The P-CSCF informs 5GC which two UEs are call parties for UE-satellite-UE communication with UPF local switching.

-	The P-CSCF receive the result of UPF onboard local switching configuration from 5GC.

5GC:

-	The 5GC receives identifications of call parties and indication that UPF onboard local switching is possible from IMS.

-	The 5GC sends the result of UPF onboard local switching configuration to the P-CSCF.

Editor's note:	The impacts to 5GC need to be broken down to different NFs.

## 6.32	Solution #32 for Key Issue #3: Support of UE-satellite-UE communication for IMS services with UPF/AGW inserted on satellites

### 6.32.1	Description

This solution resolves the Key Issue #3 to support UE-satellite-UE communication for IMS services, and some content can resolve Key Issue #1.

This solution proposes enhancement to both 5GC and IMS, including:

-	Enhance 5GC procedures and functions to transfer and handle the access network information with specific treatment for Regenerative-based satellite access. It is also applicable for Key Issue #1

-	Enhance interactions between 5GC and IMS, enabling 5GC to provide access network information with specific treatment for Regenerative-based satellite access,

-	Enhance IMS procedures and functions to distribute the access network information with specific treatment for Regenerative-based satellite access, enabling IMS to determine the conditions for realizing UE-satellite-UE communication for IMS services,

-	Establish an IMS-influencing traffic routing mechanism, enabling user plane traffic to remain in the satellites.

It is necessary for the 5GC network function UPF, and the IMS component AGW to be deployed to the satellite(s). For some operator, the AGW is necessarily deployed in UE's media path to support some important features, e.g. transcoding, security isolation from IMS core, Lawful Interception.

Considering AGW on board satellites, it is assumed that P-CSCF is pre-configured with satellite constellation information.

In roaming scenario, depends on inter-operator agreement, if the access network of UE can be exchanged between different IMS domains, then this solution can support the roaming scenario.

Editor's note:	How to handle LEO constellations where the satellite changes is FFS.

#### 6.32.1.1	Reference Architecture

The examples of network architecture are listed in the Annex C. For this solution, gNBs, UPFs and IMS AGWs are deployed on the satellites. The PSA UPFs for IMS session are on the ground.

IMS AGW can be deployed on each satellite with gNB and UPF, as shown in the Figure 6.32.1-1.

Figure 6.32.1.1-1: Network architecture with IMS AGW deployed on each satellite

In this figure, the two UEs are served by Satellite 1 and Satellite 2 respectively, and the two serving satellites are connected with ISL. On each satellite, gNB, UPF and IMS AGW are installed. The signalling of UE1 and UE2 goes to the ground IMS network, while the media path between UE1 and UE2 does not go through the ground for UE-Satellite-UE(UE-SAT-UE) communication. The two UEs can be served by a same satellite or different satellites.

An alternative architecture is to install the common IMS AGW in some satellites, and the common IMS AGW can serve users that access network from other satellites as shown in Figure 6.32.1-2.

Figure 6.32.1.1-2: Network architecture with common IMS AGW

For this architecture, the media path between UEs just goes through the common IMS AGW to achieve UE-SAT-UE communication.

### 6.32.2	Procedures

#### 6.32.2.1	Provision of access network information in 5GC

This procedure is based on PDU session Establishment or Modification procedures described in clauses 4.3.2 or 4.3.3 of TS 23.502 [3], with specific treatment for Regenerative-based satellite access.

NOTE.	The mechanism of this procedure can also apply for Key Issue #1 accordingly.

Figure 6.32.2.1-1 describes the general procedure.

Figure 6.32.2.1-1: Provision of access network information in 5GC

1.	UE requests IMS PDU Session Establishment/Modification via Regenerative-based satellite access for NR NTN.

2.	Based on information provided by NG-RAN onboarding the satellite, the AMF determines the RAT Type information and/or serving satellite ID for Regenerative-based satellite access of the UE. The RAT Type information is distinguished from currently defined satellite access RAT type for Transparent-based satellite access , e.g. "NR(LEO)", "NR(MEO)", "NR(GEO)" and "NR(OTHERSAT)". The serving satellite ID is the ID of satellite with onboard RAN component connected by UE, which could be distinguished from currently defined satellite ID for satellite backhaul. Then the AMF includes RAT Type information and/or serving satellite ID in the message to SMF, e.g. PDU Session Create/Update SM context Request.

3.	The SMF may respond to AMF as described in clauses 4.3.2 or 4.3.3 of 4.3.3 of TS 23.502 [3].

4.	The SMF decides to perform SM Policy Association Establishment/Modify procedures as defined in clauses 4.3.2 or 4.3.3 of TS 23.502 [3], including the RAT Type information and/or serving satellite ID for Regenerative-based satellite access in the request to PCF. PCF transfers and handles this access network information with specific treatment for Regenerative-based satellite access (e.g. RAT type information and/or Serving satellite ID) as specified in clause 6.32.2.2.

#### 6.32.2.2	Provision of access network information to IMS

In this procedure, the 5G core network can provide the access network information of the UE via regenerative-based satellite access (e.g. RAT type information and/or Serving satellite ID) to IMS. This information is required in IMS to comply with regulatory requirements (e.g. data retention, lawful interception) and determine whether to perform UE-satellite-UE communication IMS services for this UE.

Figure 6.32.2.2-1 describes the general procedure.

Figure 6.32.2.2-1: Provision of access network information to IMS

1.	P-CSCF receives SIP Messages (e.g. Register request, INVITE request, Offer response) from the UE.

2.	P-CSCF invokes Npcf_PolicyAuthorization_Create/Update Request to request the access network information for its served UE, as described in clause 4.16.5.2 of TS 23.502 [3]. The access network information can contain NPLI (Network Provide Location Information) including user location and/or UE Time Zone information, RAT type and serving satellite ID.

3.	PCF retrieves the access network information.

4.	PCF invokes Npcf_PolicyAuthorization_Create/Update Response to report the access network information for the UE.

#### 6.32.2.3	IMS procedures to determine to perform the UE-satellite-UE communication

This procedure describes how the access network information, with specific treatment for Regenerative-based satellite access provided by 5GC (e.g. RAT type information, and/or Serving satellite ID, and/or NPLI), can be distributed within IMS. This information is necessary in IMS to determine whether the conditions are met to enable UE-satellite-UE communication for IMS services, e.g. voice, video. This includes verifying if UEs have subscribed to UE-Satellite-UE communication for IMS services, and checking the currently used RAT Types for regenerative-based satellite access, and if they are close to each other.

This procedure is based on the procedures described in Annex R of TS 23.228 [9]. Figure 6.32.2.3-1 describes the general procedure.

Figure 6.32.2.3-1: IMS procedures to determine to perform the UE-satellite-UE communication

P-CSCF#1 and P-CSCF#2 represent either the originating side or terminating side of the IMS session.

1.	The UE#1, S-CSCF#1 or other IMS functions send a SIP message (e.g. INVITE, Response) to P-CSCF#1.

2.	P-CSCF#1 initiates to retrieve UE#1's access network information by using the procedure described in clause 6.32.2.2.

3.	P-CSCF#1 sends the SIP Message to S-CSCF#1, including UE#1's access network information.

4.	S-CSCF#1 validates the UE#1's service profile and invokes any appropriate service logic for this SIP message to determine if UE-satellite-UE communication for IMS services can be provided for UE#1.

5.	S-CSCF#1 forwards the SIP message to S-CSCF#2 of the peer UE (assuming S-CSCF#2 is in the same trust domain), by initiating S-S procedures as described in clause 5.5.2 of TS 23.228 [9].

If S-CSCF#1 determines the UE-satellite-UE communication for IMS services can be provided for UE#1, S-CSCF#1 forwards the SIP message, including UE#1's access network information to S-CSCF#2 (assuming S-CSCF#2 is in the same trust domain).

6.	S-CSCF#2 validates the UE#2's service profile and invokes any appropriate service logic for this SIP message to determine if UE-satellite-UE communication for IMS services can be provided for UE#2.

7.	S-CSCF#2 forwards the SIP message to P-CSCF#2.

If S-CSCF#2 determines the UE-satellite-UE communication for IMS services can be provided for UE#2, S-CSCF#2 forwards the SIP message, including UE#1's access network information to P-CSCF#2.

8.	P-CSCF#2 retrieves UE#2's access network information from 5GC by using the procedures described in clause 6.32.2.2.

If P-CSCF#2 receives the UE#1's access network information from S-CSCF#2, P-CSCF#2 is aware that UE#1 and UE#2 are both served by Satellite with gNB, UPF and IMS AGW onboard, and UE-satellite-UE communication for IMS services may be provided.

9.	P-CSCF#2 determines whether to perform UE-satellite-UE communication for IMS services for UE#1 and UE#2. For example, based on access network information of UE1 and UE2, P-CSCF#2 can, based on configuration, determine whether the two UEs are connected to the same satellite or with interconnected satellites (through ISL).

When determining that UE-satellite-UE communication can be performed and based on UE#2's serving satellite ID, for the media path of UE#2, P-CSCF#2 performs (re)selection of AGW onboard the satellite to ensure IMS media traffic routing remain in the satellites, correlated with the procedure described in clause 6.32.2.4.

When determining that UE-satellite-UE communication cannot be performed, the UE-UE IMS service should be routed on ground, e.g. because of long distance between two UEs, or the peer UE does not access via Regenerative-based satellite access which is the high probability scenario at presence, the P-CSCF#2 select the AGW on ground following normal IMS procedures.

NOTE:	Like P-CSCF#2, P-CSCF#1 shall also receive the access network information for both UE#1 and UE#2 during the IMS procedures. Then the P-CSCF#1 shall perform above same AGW (re)selection for UE#1.

#### 6.32.2.4	IMS influence on traffic routing

This procedure is proposed to ensure traffic routing remains in the satellites. It is based on Application Function influence on traffic routing procedures described in clause 4.3.6 of TS 23.502 [3].

-	P-CSCF sends request to influence SMF routing decisions for traffic, by initiating Application Function influence on traffic routing procedures described in clause 4.3.6 of TS 23.502 [3]. This will be performed in the step 9 of the procedure described in clause 6.32.2.3, after P-CSCF determines to provide UE-satellite-UE communication for IMS services for the UE. In this procedure, P-CSCF acts as the AF, and AGW on board satellite acts as the EAS.

-	For the IMS PDU session, the SMF selects and inserts UPF(s) on board satellite(s) in the data path to act as UL CL UPF and Local PSA UPF. The PSA UPF of the IMS PDU session was selected on the ground during the PDU Session Establishment procedure.

After this procedure, the IMS signalling of the UE remains going to the P-CSCF on the ground through PSA UPF on the ground. The media path of the UE goes through UL CL UPF, local PSA UPF and AGW on board satellite(s), and is connected to the AGW of the peer UE. Figure 6.32.2.4-1 shows the IMS Data path for UE-satellite-UE communication.

Figure 6.32.2.4-1: IMS Data path for UE-satellite-UE communication

### 6.32.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

AMF:

-	Determination of the RAT Type information and serving satellite ID for Regenerative-based satellite access of the UE.

-	Provision to other NFs the RAT Type information and serving satellite ID for Regenerative-based satellite access.

SMF:

-	Provision to other NFs the RAT Type information and serving satellite ID for Regenerative-based satellite access

PCF:

-	Provision to IMS functions the RAT Type information and serving satellite ID for Regenerative-based satellite access

P-CSCF:

-	Provision of RAT Type information and serving satellite ID for its served UE to other IMS functions.

-	Determination of UE-satellite-UE communication for IMS services.

-	(re)Selection of AGW onboard satellite correlated with Application Function influence on traffic routing procedures.

S-CSCF:

-	Provision of RAT Type information and serving satellite ID for its served UE to other IMS functions.

-	Provision of RAT Type information and serving satellite ID for the remote UE to other IMS functions.

-	Validation of UE-satellite-UE communication for IMS services.

## 6.33	Solution #33: Solution for UE-satellite-UE communication with IMS media entity onboard

### 6.33.1	Description

This solution resolves Key Issue #3 about the Support of UE-satellite-UE communication. Focus on the Architectural enhancement to support UE-satellite-UE communication, including minimum necessary set of 5GC network functions and IMS components (if any) onboard the satellite(s), and minimize service interruption if the Serving Satellite changes.

To support UE-satellite-UE communication for IMS voice and video services, and simultaneously achieve local offloading of traffic to reduce service transmission latency, it is necessary from an architectural perspective to place gNB,UPF and IMS media plane(such as IMS-AGW) on board the satellite.

NOTE:	These solution only consider the scenario when gNB, UPF and IMS-AGW are deployed on the same satellite and UEs are within the same cell, as shown in Figure 6.33.1-1, UE-satellite-UE communication can be achieved without the user plane traffic going through the ground network. The solutions for UEs located in different cells and communicating via ISL for UE-Satellite-UE are FFS.

Figure 6.33.1-1: Basic concept of UEs- SAT- UEs communications on satellite in same cell

This solution is based on the Rel-18 approach agreed in TS 23.228 [9], TS 24.229 [11] and TS 23.502 [3]. Related to PDU Session Establishment, IMS registration signaling, and the IEs included in the messages.

In this solution, for the Architectural enhancement to support UE-satellite-UE communication, as shown in Figure 1, it is necessary from an architectural perspective to place gNB,UPF and IMS media plane(such as IMS-AGW) on board the satellite, and the system should select the on board UPF and IMS media plane function to ensure routing of user plane traffic remains in satellite. The potential impacts on IMS procedures and functions to manage UE-satellite-UE communication as follows:

-	For the PDU Session Establishment procedure of UE-A and UE-B, SMF should select the UPF(SAT) based on the Satellite ID and RAT Type. The Satellite ID may be provisioned to the AMF by O&M, AMF should determines the RAT Type and Satellite ID based on the Global RAN Node IDs, then send RAT Type and Satellite ID to SMF. SMF utilize the NRF to discover UPF(SAT) based on the RAT Type and Satellite ID.

-	For the IMS Registration procedure of UE-A and UE-B, the Access Type in PANI of SIP REGISTER should be '3GPP-NR-SAT-LEO','3GPP-NR-SAT-MEO' for the flexible service control.

-	For the Calling procedure(MO) of UE-A to UE-B, all the IMS-AGW(SAT) IP address may be provisioned to P-CSCF by O&M. For MO procedure, P-CSCF(O) should select the IMS-AGW(SAT) for UE-A based on the (NR Cell Global Identity)NCGI in PANI of SIP INVITE from UE-A.

-	For the Calling procedure(MT) of UE-A to UE-B, P-CSCF(T) should select the IMS-AGW(SAT) for UE-B based on the (NR Cell Global Identity)NCGI in PANI of SIP 183 from UE-B. As UE-A and UE-B connect to the same satellite within the same earth stationary cells, their NCGI will be the same, the IMS-AGW(SAT) selected by P-CSCF(O) and P-CSCF(T) will be the same.

-	For scenarios that feeder link is unavailable, all the other steps are same with above, to establish a connection with the terrestrial network, satellites only perform data transmission via ISL.

### 6.33.2	Procedures

#### 6.33.2.1	UE-Satellite-UE Communication Signalling

Figure 6.33.2.1-1: UE-Satellite-UE Communication Solution Procedure

1.	For UPF(SAT) service Registration procedure, NRF store the UPF(SAT) profile(configured by OAM system, should include the IP address and Satellite ID of UPF(SAT)).

2.	AMF can obtain Global RAN ID from RAN through NG Setup process.

3.	After the satellite gNB establishes an NGAP connection with the AMF, the UE-A initiates a PDU session establishment request through satellite access.

4.	AMF determines the RAT Type and Satellite ID that UE is camping on, based on the Global RAN Node IDs indicated by NG-RAN. The AMF send the Nsmf_PDUSession_CreateSMContext Request(Satellite ID, RAT Type) to SMF. The Satellite ID of all satellite may be provisioned to the AMF by O&M as the same way Coverage availability information provisioning to the AMF.(see clause 5.4.13.3 of TS 23.501 [2])

5.	SMF utilize the NRF to discover UPF(SAT) based on the RAT Type and Satellite ID, and SMF select the appropriate UPF(SAT) of the PDU session.

6.	Both UE-A and UE-B proceed PDU Session Establishment procedure as described in step3-step5. In this scenario, as UE-A and UE-B are in the same earth stationary cell, they will connect to same satellite, same UPF(SAT) are selected for the PDU Session of UE-A and UE-B.

7.	After PDU session Establishment, UE-A/UE-B initiate IMS Registration procedure, the SIP REGISTER message send from UE to P-CSCF, P-CSCF to I-CSCF, and I-CSCF to S-CSCF should include the Access Type, it should indicate the specific type of satellite accessed by the UE, the specific Access Type are '3GPP-NR-SAT-LEO','3GPP-NR-SAT-MEO'). S-CSCF should determine the success and failure of user registration based on the Access Type, User Profile and Operator Policy.

8.	UE-A initiate IMS Call to UE-B, in this procedure, both P-CSCF(O) and P-CSCF(T) should select the IMS-AGW(SAT) for UE-A and UE-B based on the Access Type and NCGI. As UE-A and UE-B are in the same earth stationary cell and will connect to same satellite, same IMS-AGW(SAT) are selected for UE-A and UE-B.

#### 6.33.2.2	Minimize service interruption if the Serving Satellite changes

Due to the high mobility of LEO/MEO satellites, there are two scenarios after the UE connects with a satellite: multiple satellites provide continuous coverage to the same Earth Stationary Cell (as shown in Figure 6.33.2.2-1), and multiple satellites do not provide continuous coverage to the same Earth Stationary Cell (as shown in Figure 6.33.2.2-2). In these two scenarios, the serving satellite connected to the UE will change, which will impact service continuity.

Figure 6.33.2.2-1: Discontinuous Satellite Coverage

As Satellite Coverage availability information may be provisioned to the AMF by O&M, in cases where Satellite 1 moves away and no longer covers the cell that UEs located, and Satellite 2 is also unable to cover this cell, AMF shall initiate AN Release procedure(As specified in clause 4.2.6 of TS 23.502 [3]) based on the Satellite Coverage availability information.

Editor's note:	The specific HO procedure under discontinuous satellite coverage scenario is FFS.

Figure 6.33.2.2-2: Continuous Satellite Coverage

In scenarios where Satellite 1 is moving away while Satellite 2 is already covering the cell that UE located, there is continuous satellite coverage to the cell. During this process, the cell in which the UE is located has always had satellite coverage, AMF initiate Handover procedure based on the Satellite Coverage availability information. The specifics of the Handover procedure depend on the connectivity between the satellites, only the UE's connected Satellite gNB needs to switch from Satellite 1 to Satellite 2, without changing satellite onboard User Plane Function ( i.e. UPF(SAT), IMS-AGW(SAT),etc. ). However, if there is no ISL between the two satellites, then the onboard gNB, UPF, and IMS-AGW of the satellite must be switched from Satellite 1 to Satellite 2.

Editor's note:	The specific HO procedure under continuous satellite coverage scenario is FFS.

### 6.33.3	Impacts on services, entities and interfaces

This solution impacts the following entities.

O&M:

-	Capability to configured the UPF(SAT) profile(Include the Satellite ID of UPF(SAT))

-	Capability to provision all the Satellite ID of all satellite to AMF.

SMF:

-	Capability to utilize the NRF to discover UPF(SAT) based on the RAT Type and Satellite ID.

AMF:

-	Capability to determines the RAT Type and Satellite ID the UE is camping on based on the Global RAN Node IDs.

-	Capability to initiate AN release procedure based on the Satellite Coverage availability information.

-	Capability to initiate Handover procedure based on the Satellite Coverage availability information.

P-CSCF:

-	Capability to select the IMS-AGW(SAT) based on the Access Type and NCGI.

UE:

-	Capability to send Access Type('3GPP-NR-SAT-LEO','3GPP-NR-SAT-MEO') to the P-CSCF.

## 6.34	Solution #34: Enhancement on AMF discovery for Regenerative-based satellite

### 6.34.1	Description

This is a solution addressing Key Issue#1.

For LEO/MEO deployment, the gNB deployed on a satellite moves around the earth, it is not preferable to locally configure the IP addresses of all AMFs on the earth that can be connected by the satellite gNB. This solution provides mechanism for Satellite/gNB to dynamically discover the AMFs that is allowed to be connected by the Satellite/gNB.

Editor's note:	How this solution supports RAN sharing is to be clarified.

In this solution, an AMF agent is deployed by the satellite operator and closed to the NTN-GW. The AMF agent supports N2 interface towards Satellite/gNB via the NTN-GW. The AMF Agent provides address information of all AMFs within its service area that can be connected by the Satellite/gNB, so the Satellite/gNB can establish direct SCTP connection and NGAP connection towards these AMFs.

The Satellite/gNB is configured with the address of AMF Agent based on the ephemeris information of the Satellite/gNB, and this information may be updated by the satellite operator.

### 6.34.2	Procedures

The following procedures shows how to enhance the AMF discovery Regenerative-based satellite.

Figure 6.34.2.1-1: AMF selection enhancement for Regenerative-based satellite

0.	The Satellite/gNB is configured with the address information of AMF Agent based on the ephemeris information of the Satellite/gNB.

1.	When the Satellite/gNB moves into the service area of the AMF Agent, it initiates SCTP connection establishment towards the AMF Agent via an NTN-GW.

2.	After the SCTP connection is established, the Satellite/gNB initiates NG Setup procedure by sending NG Setup Request message to the AMF Agent. The message includes relevant configuration information e.g. Global RAN Node ID, list of supported TA(s), list of supported PLMN(s), supported S-NSSAI(s) per TA and so on

3.	The AMF Agent may use the Global RAN Node ID to identify the Satellite/gNB and determine the AMFs within the AMF Agent's service area that are allowed to be connected by the Satellite/gNB. The AMF Agent returns NG Setup Response message towards the Satellite/gNB, including new AMF information. The new AMF information may include at least the SCTP address information of the AMF.

4-9.	Based on local configuration, the Satellite/gNB starts to establish SCTP connection and NGAP connection towards the AMFs using the AMF information as received in the step 3.

### 6.34.3	Impacts on services, entities and interfaces

NGAP interface:

-	The NG Setup response message is enhanced to include the address information of all AMFs within AMF agent's service area that is allowed to be connected by the Satellite/gNB

AMF Agent:

-	AMF with enhancement on the NGAP interface as described in clause 6.34.2.

Satellite/gNB:

-	gNB with enhancement on the NGAP interface as described in clause 6.34.2.

## 6.35	Solution #35: N2/S1 procedures to handle feeder link switch

### 6.35.1	Description

This solution addresses KI#1.

There are different potential deployment scenarios for how feeder link switch takes place. In Figure 6.35.1-1 below, examples of where the gNB can reach the same AMF or not reach the same AMF after the feeder link switch are shown.

Figure 6.35.1-1. Feederlink switch scenarios

Both of these scenarios could be performed using "hard" ("break-before-make") switch and "soft" (make-before-break") switch. These options are illustrated in Figure 6.35.1-2 below.

Figure 6.35.1-2. Soft vs hard switch

There are several options for how to handle feeder link switches based on existing N2 and S1 procedures and capabilities. This solution describes how this can be done, in order to easier identify whether there are any gaps in the existing standard.

Scenario 1 (no AMF/MME change):

In case of deployment scenario 1, the TNL between gNB/eNB and AMF/MME would change while the gNB/eNB is serving a specific set of cells and Tracking Areas. To update the TNL association between gNB/eNB and AMF/MME different options are possible based on the existing standard:

-	Support for multiple TNL associations is available in 5GS (i.e. N2), as described in TS 23.501 [2] and TS 38.413 [12]. In this case the gNB can initiate a new TNL association via the new feederlink and remove the old TNL association that was active over the old feederlink.

-	Support for replacing an old N2/S1 interface with a new N2/S1 interface while preserving the active UE contexts is supported for both N2 and S1, as described in TS 38.413 [12] and TS 36.413 [14]. In this case the gNB/eNB establishes a new SCTP association via the new feederlink and sends a N2/S1 SETUP Request including the UE Retention Information IE. The AMF/MME accepts to retain the existing UE related contexts and signalling connections. The AMF/MME sends a N2/S1 SETUP Response and includes the UE Retention Information IE.

-	To update the N3/S1-U GTP-U tunnels with the new gNB/eNB TNL address, the gNB/eNB can provide the new tunnel endpoint information (F-TEIDs) to the core network by sending PDU SESSION RESOURCE MODIFY INDICATION / E-RAB MODIFICATION INDICATION.

Scenario 2 (with AMF/MME change):

Scenario 2 would mean that a gNB/eNB may need to change the AMF/MME during the time it serves UEs in specific cells and Tracking Areas, i.e. without any UE mobility taking place. Such scenario is not present in the case of terrestrial networks. However, if an operator nevertheless would like to deploy such network for satellite access, it is possible to support using the following options based on the existing standard:

-	Similar to what is described in Annex A, Figure A-3, a second (logical) gNB/eNB can be activated on the same satellite at time T1.5 (Figure 6.35.1-2) which connects to the AMF/MME via the new feeder link. The two (logical) gNB/eNB on-board then trigger handover of the UEs from the old gNB/eNB (using old feederlink) to the new gNB/eNB (using new feederlink) on the same satellite. It is assumed that the two (logical) gNBs/eNBs have different RAN node IDs. This solution can address both scenarios 1 and 2 but assumes that "soft" feederlink switch is supported.

Editor's note:	Whether it is possible for the two (logical) gNBs/eNBs to have the same RAN node ID is FFS.

-	In case of hard feederlink switch with AMF/MME change it is not possible to use the N2/S1 handover procedure since connectivity with both source and target gNB/eNBs is needed. However, since AMF/MME change is assumed to be required it is also reasonable that the two AMF/MME serve different sets of TAIs. Therefore, the gNB/eNB could change the TAI it broadcast at the same time as it switches the feederlink and connects to the new AMF/MME. This will trigger the UE to perform a Mobility Registration Update or TAU, which will ensure that the UEs are moved from old AMF/MME to new AMF/MME. This solution is possible also in case of soft feederlink switch.

NOTE:	Different AMFs/MMEs serving the same geographic area can be configured to support different TAI ranges with earth-fixed TAs if the TAIs have overlapping geographic coverage.

### 6.35.2	Procedures

There are no impacts to the 3GPP procedures.

Editor's note:	Call flows illustrating the scenario 2 options are FFS.

### 6.35.3	Impacts to Services, Entities and Interfaces

gNB/eNB and AMF/MME implementations would need to support above mentioned existing features (e.g. multiple TNL associations, UE retention)

There are no impacts to 3GPP protocols or the SA WG2 standards. Potential impacts to describe the above methods in the context of NTN is assumed to be out of SA WG2 scope.

## 6.36	Solution #36: User plane path failure management due to  satellites leaving an area served by a CN

### 6.36.1	Description

This solution resolves Key Issue #1: Support of Regenerative-based satellite access.

This solution assumes that the gNB is deployed on LEO satellite and 5GC is on the ground.

To prevent user plane path failure errors, this solution is illustrated.

### 6.36.2	Procedures

Figure 6.36.2-2 Method 1 of user plane path failure handling procedure

1.	The gNB informs the AMF that movement information of itself.

2.	The AMF informs the SMF when gNB on-board satellite has left the serving aera.

3&4.	When the UPF does not receives regular Echo Response in the last path detection, the UPF informs the SMF that user plane path failure via existing mechanism (e.g. N4 notification).

5.	The SMF receives node report request from the UPF and handle that depends on different situations.

Editor's note:	It is FFS when user plane is rebuilt later, the SMF may instruct the UPF to keep the UE context information.

### 6.36.3	Impacts to Services, Entities and Interfaces

Editor's note:	Impacts to Services, Entities and Interfaces is FFS.

## 6.37	Solution #37: AMF Split Architecture to support Control plane procedure for S&F

### 6.37.1	Key Issue mapping

This solution aims to resolve the Key Issue #2, "Support of Store and Forward Satellite operation".

### 6.37.2	Description

As agreed in KI#2, The Store and Forward Satellite (S&F) operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

Figure 6.37.2-1: AMF split architecture for Store and Forward operation

In regenerative architecture, the RAN will be placed in Satellite. In the Store & Forward (S&F) scheme, the AMF is at Satellite will be responsible for providing Store and forward feature. AMF in satellite is represented as AMF-NT (non-terrestrial) in the above figure. Let's assume the time T1 is between 10:00-10:20 and T2 is 10:40-11:00. The satellites will have ground station coverage when hovering over Rennes and Orleans but not when crossing Le Mans.

At time T1, the black satellite covers Le Mans, and the white satellite provides coverage to Rennes. At T2, the black satellite will be in Orleans, and the white satellite will cover Le Mans.

At time T3, the white satellite will sync up with the AMF-NT at Orleans.

### 6.37.3	Procedures

#### 6.37.3.1	5GS Architecture enhancement to support S&F

As agreed in KI#2, the delay tolerant/non-real time services can be served via S&F Satellite operation.

Figure 6.37.3.1-1: AMF split architecture for Store and Forward operation for 5GS

The key difference between the 3GPP Rel-18 and the proposed method is the buffering of MO and MT data on the NF in the orbit and buffering of MT data on the NF on the ground. The AMF role is split between peer AMF entities on the ground and onboard satellite.

For 5GS, the peer AMF entities hide the store-and-forward functionality from the other NFs. In this document, the AMF on the ground as AMF-T (for Terrestrial). The corresponding peer entities on board satellite are called AMF-NT (for Non-terrestrial).

UE registration and the related security procedures are assumed to have taken place earlier. It is foreseen that the target UEs would be under direct coverage at least occasionally and thus, the registration and mobility updates can be either via terrestrial network or when satellite cell has got ground station connectivity. The store and forward procedures shown in this document can be used for small data transmission when the UE is served by satellite cell that at least occasionally loses its ground station connectivity. The need to use store and forward operation can be detected at registration or mobility update or periodic update time.

In this solution, the following aspects are considered:

1.	AMF instance (AMF-T) on the ground interfaces AMF instance on board (AMF-NT)

2.	AMF-T has access to satellite orbital data.

3.	AMF-T is aware of satellite coverage in terms of feeder link availability.

4.	AMF-T supports the AMF's role towards the other NFs on the ground.

5.	AMF-T supports an implementation-specific interface with delay-aware satellite-specific AMF functionality.

6.	AMF and UDM store the "Store-and-forward indication" in UE context in AMF at UE registration time.

7.	AMF-T discovers the next AMF-NT that can reach the UE.

#### 6.37.3.2	Registration Procedure in 5GS in S&F scenario

Figure 6.37.3.2-1: Registration procedure in S&F

1.	UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, SUCI or 5G-GUTI or PEI)

The Registration type indicates if the UE wants to perform an Initial Registration, a Mobility Registration Update, a Periodic Registration Update, an Emergency Registration.

The NAS message container shall be included if the UE is sending a Registration Request message as an Initial NAS message and the UE has a valid 5G NAS security context and the UE needs to send non-cleartext IEs. If the UE does not need to send non-cleartext IEs, the UE shall send a Registration Request message without including the NAS message container.

If the UE does not have a valid 5G NAS security context, the UE shall send the Registration Request message without including the NAS message container. The UE shall include the entire Registration Request message (i.e. containing cleartext IEs and non-cleartext IEs) in the NAS message container that is sent as part of the Security Mode Complete message.

2.	(R)AN to AMF-NT-1: N2 message (N2 parameters, Registration Request (as described in step 1).

If the AMF-NT-1 do not have connectivity to the ground when receiving Registration Request, it will store the Registration Request message.

3.	AMF-NT-1 to UE via (R)AN: Sends DL NAS transport with temporary UE identifier (Interim GUTI). Here assuming all the AMF-NT in all satellite in the constellation will have unique AMF ID, thus creating non-colliding GUTI allocations. (GUTI = PLMN ID+AMF Region ID+ AMF Set ID+AMF ID+TMSI).

UE will remember this temporary identifier (Interim GUTI) for future transactions. AMF may also provide validity time for this temporary identifier to UE.

NOTE 1:	UE needs to maintain a NAS sub-state (e.g. Registration-on-going) to enable itself to process paging using temporarily assigned GUTI. It is up to stage 3 to define this new UE NAS sub-state. The RAN shall make the UE RRC-Idle before moving out of coverage or UE can autonomously go to RRC-idle when it detects loss of coverage.

4.	AMF-NT-1 to AMF-T: When Satellite containing RAN-1 and AMF-NT-1 moves away from the UE and gets connected to ground station, the AMF-NT-1 shall forward the stored Registration Request message along with the Interim GUTI to AMF-T.

5.	The AMF-T may decide to initiate UE authentication by invoking an AUSF. In that case, the AMF-T selects an AUSF based on SUPI or SUCI.

If the AMF-T is configured to support Emergency Registration for unauthenticated SUPIs and the UE indicated Registration type Emergency Registration, the AMF skips the authentication or the AMF accepts that the authentication may fail and continues the Registration procedure.

6.	If authentication is required, the AMF requests it from the AUSF; Upon request from the AMF, the AUSF shall execute authentication of the UE. The authentication is performed as described in TS 33.501 [22]. The AUSF selects a UDM and gets the authentication data from UDM.

7.	When the AUSF returns the authentication data to AMF-T, the AMF-T shall store it against the interim GUTI received from AMF-NT onboard the satellite.

AMF-T will now determine the next probable satellite which can reach the UE location next.

NOTE 2:	One such determination mechanism is described in clause 6.37.3.3

8.	AMF-T to next probable satellite's AMF-NT-2: Namf_N1N2MessageTransfer (containing Authentication Request NAS message and interim GUTI). AMF-T shall also provide the last known location of UE.

9.	Either the UE comes back after seeing the new cell (RAN-2) or AMF-NT-2 pages the UE using the interim GUTI. When the UE gets connected to RAN-2; RAN-2 shall send initial UE message (containing service request)

10.	The AMF-NT-2 sends the authentication request message to UE received in step 9.

11.	UE sends the authentication response message (including HXRES*) to AMF-NT-2. As the AMF-NT-2 do not have connectivity to the ground when receiving authentication response, it will store the authentication response message.

12.	When AMF-NT-2 comes in contact with ground station (moves away from the UE as well), forwards the authentication response to AMF-T.

13.	AMF-T validates (HXRES*=XRES*) sends the Authentication Request to AUSF.

14.	AUSF sends authentication response with KSEAF to AMF-T.

15.	AMF-T to AMF-NT-3: Since the NAS security context do not exist, the NAS security initiation is performed by sending Namf_N1N2MessageTransfer (containing both security mode command for the UE) & (registration request message received at step 4) & interim GUTI. AMF-NT-3 stores the information until reaches the coverage area of the UE. (AMF-T finds the suitable AMF-NT-3 based on the criteria discusses in step 8). AMF-T shall also provide the last known location of UE.

16.	AMF-NT-3 tries to reach UE via paging using the interim GUTI or UE tries to establish RRC connection using service request with interim GUTI. Then AMF-NT-3 sends the NAS security mode command to UE.

17-18.	UE enables the security and acknowledges it to AMF-NT-3.

19.	AMF-NT-3 upon receiving security mode command ack from UE stores it till it regains the ground station connectivity again and then sends the security mode command ack to AMF-T.

20.	AMF-T does the steps 13, 14, 15 described in clause 4.2.2.2.2 in TS 23.502 [3]. AMF-T at this stage may determine, whether UE is allowed to be served in S&F scenario or not.

21.	AMF-T then selects suitable candidate satellite which is going to serve the UE next. The selection criteria can be as per description in step 8. AMF-T sends the Registration accept message to AMF-NT-4 via N1N2MessageTransfer. AMF-NT-2 stores the Registration accept message. AMF-T shall also provide the last known location of UE.

22-23.		When AMF-NT-4 reaches the UE serving area, it follows the steps mentioned in step 10. AMF-NT-4 sends the registration accept and shares the TMSI, security keys to RAN-4.

24-25.	UE may respond with registration complete towards AMF-NT-4. Which AMF-NT-4 will store and forward until it reaches the ground connectivity again with AMF-T.

Editor's note:	How to avoid/minimise DoS attack on AMF-NT while handling integrity protected messages from UE (as it lacks the NAS keys for decoding) is FFS.

Editor's note:	How to avoid/minimise fake base station attack when delivering plain text RRC message (message 3) is FFS.

Editor's note:	How to handle NAS timers due to extended delay in serving NAS transactions in S&F.

Editor's note:	How to distribute the NAS key set and maintain SN across multiple AMF needs to be studied further.

Editor's note:	It is FFS on the worst-case scenario for number of satellite change required to complete any C-plane procedure.

NOTE 3:	The UE shall support storing interim GUTI and use it for responding to Paging or while sending any initial NAS request.

NOTE 4:	AMF-NT should be able to create distinct interim GUTI (one assumption is to have distinct AMF pointer ID for each AMF in satellite).

NOTE 5:	AMF-NT are not keeping any security material (both AS and NAS). Integrity and ciphering are to be done at AMF-T for every NAS message. This is to keep AMF-NT as light weight s/w as possible.

NOTE 6:	The UE shall store the temporary NAS context till the validity time of the interim GUTI.

NOTE 7:	How to create trust between AMF-T and AMF-NT is up to SA WG3.

NOTE 8:	Due to the dependency on paging procedures (step 10, 17 and 23) to bring the UE into connected mode for DL signalling messages, the failure in paging will prolong the attach procedure.

#### 6.37.3.3	UE Reachability Estimation (Satellite Location/Coverage Information)

##### 6.37.3.3.1	UE reachability estimation

The URE is a new network function located on ground responsible for estimating which non-terrestrial network element can reach the UE, according to defined criteria, such as in shortest amount of time possible. The following figure shows a possible architecture, including the URE.

Figure 6.37.3.3-1: Architecture 5GS including UE Reachability Estimator

The UE reachability is a function of both:

1.	UE location (and optionally trajectory and velocity, if available); and

2.	Satellite (NTNFs) location, velocity, and ephemeris. This includes when there is a feeder link between the ground and a SAT and when there is Uu radio connectivity between the SAT and the UE.

For a static UE, URE estimates reachability based on the satellite's ephemeris. A precise "Reachability Schedule" (RS) can be calculated. This RS precisely contains the time and duration during which a UE is reachable by the satellite, through which the UE is reachable in each duration, and the latest time this satellite has reached the terrestrial core network before covering the UE.

For UEs in motion, if the trajectory and velocity of the UE are known, the URE takes these into account when constructing the RS. In this case, the RS may include reachability probabilities with each entry. The RS can also be more precisely constructed if the UE trip plan is known to the URE.

##### 6.37.3.3.2	URE-TNFs interaction and use-cases

A TNF that would like to know when a given UE is reachable queries the URE for this UE. The URE responds with the earliest possible "UE Reachability Time" and "UE Reachability Duration" and candidate Satellites. There can be several use cases where a TNF may interrogate the URE, for example:

1.	to set a timer for a NAS procedure of a satellite-served UE;

2.	to determine when to send messages (signalling, data, SMS) to the UE; or

3.	during NAS procedures, the AMF may take "UE Reachability Time" into account when assigning the NAS timer values (such as Periodic Update Timer).

As a response to a TNF interrogation, the URE replies with the earliest possible "UE Reachability Time" and "UE Reachability Duration". As an option, more than one alternative might be provided, e.g. "UE Reachability Time 1" and "UE Reachability Duration 1" via satellite 1, "UE Reachability Time 2" and "UE Reachability Duration 2" via satellite 2, etc.

Based on network configuration/policies and the interrogator TNF, the URE may include additional information in its response, such as the SAT-ID and/or the IDs of some or all NTNFs serving the UE during the provided reachability time and duration.

##### 6.37.3.3.3	Reachability Modes

The information provided in the URE response to the TNF varies, depending on a selected "Reachability Mode" (RM). The RM determines how the UE is reachable; for example, "Same SAT Mode" (SSM), "Any SAT Mode" (ASM) or "Fastest Reach Mode" (FRM) (via inter-SAT link).

"Same SAT Mode" (SSM) assumes that the same satellite that has received a request from a UE would be the same satellite to reply to the UE. It can also be assumed that the same satellite through which the UE registered to the network would be the satellite to use for MT signalling or MT data.

The "Any SAT Mode" (ASM), assumes that the UE will be reached by the first satellite that can cover this UE and that will have a feeder link to the terrestrial core network between the time of interrogation and before the satellite covers the UE.

The "Fastest Reachable Mode" (FRM) seeks to reach the UE and deliver the message in the fastest possible mean, including the utilisation of inter-satellite links for delivering the MT service.

If the user location is not available in the request to the URE, the URE service needs to determine the user location, e.g. by querying the UDM. The UDM provides an optional satellite constellation ID or satellite operator ID if the serving AMF of the UE has registered this information onto UDM. Assuming the last known TAI: registration area (set of TAI(s)) and gets also registered in UDM, for UEs registered via a S&F NTN RAT -Radio Access Type- (this may correspond to UE(s) with specific subscriptions that may benefit from satellite with S&F feature), the URE gets the location information from UDM.

Otherwise, the URE may need to fetch location information from AMF. If the user location is not provided by the UDM but only the registered AMF, URE queries the UE location from any AMF in the AMF set. This registration area will help in getting constellation/satellite information that is currently covering that region. The URE can query UE's last registration area and Last TAI from any AMF, in AMF set which is currently reachable. NTNF AMF can also provide expected UE behaviour information (which is currently used for paging). URE can take that into consideration to assume UE's current location.

The URE based on the satellite constellation ID or satellite operator ID, on the UE location (last known TA/ registration area) and on its knowledge of the satellite ephemeris determines:

1.	The time and duration (time window TW1) at which NTNF(s) (capable of) serving a given UE is reachable (contactable via feeder link).

2.	A subsequent time window (TW2) through which the UE is reachable by this NTNF(s) (when the UE has satellite coverage).

Editor's note:	How does the URE obtains the UE trajectory or velocity, is FFS.

Editor's note:	How the ephemeris data on satellite movement is provisioned in URE is FFS.

### 6.37.4	Impacts on services, entities and interfaces

UE:

-	NAS impact to handle larger delay due to S&F mode of operation.

-	NAS impact due to new MM capability handling (S&F).

-	NAS impact due to new interim GUTI allocation.

AMF-T:

-	Buffer/Store N2 messages for transmission towards the UE.

-	Determine when buffered N2 message should be transmitted to the relevant AMF-NT.

-	Transmission (reception) of ciphered and plain text NAS messages to (from) AMF-TsAMF-NT:

-	Buffer/Store N2 messages for transmission towards a ground station.

-	Determine when buffered N2 message should be transmitted to the AMF-T, once connected to a ground station.

-	Transmission (reception) of ciphered and plain text NAS messages to (from) AMF-TsHandling of plain text NAS and NGAP messages on behalf of AMF-T.

URE:

-	New NF estimating when UE is reachable and by which non-terrestrial network element (i.e. satellite) according to request criteria.

-	Provide/expose information about when a UE is reachable via a satellite, and also about the non-terrestrial network element (i.e. satellite) that can reach the UE.

NRF:

-	New URE profile in NRF, including Constellations IDs supported.

UDM:

-		possible impact due to new subscription data.

## 6.38	Solution #38: Exposure of Registration in S&F Mode

### 6.38.1	Description

This solution addresses following aspect of KI #2:

-	What enhancements on the related UE and network procedures are needed to support S&F Satellite Operation?

When a UE registers to the network in S&F mode and is available to send/receive data on specific occasions only (e.g. when satellites fly-over and later connect to ground station), it may be desirable to expose information such as below to an application function. For e.g. today the AF may send a message to the UE and expect UE will reply in certain time, monitored by a guard timer. AF may need to re-adjust all such applicable timer if the UE is in the S&F mode, Thus such information will help an AF adjust its logic accordingly as described.

1)	UE registered in S&F Mode: If the UE is registered in S&F mode, the AF may want to adjust the frequency, size of data, message acknowledgement handling, ack timers etc.

2)	When the Feeder-Link is available: This will help AF schedule delivery as and when Feeder-Link is available. Additionally feeder link availability duration. This can help AF to know till when a data packet can be delivered to the UE in the current pass.

3)	Expected delivery time: The delivery of data will require following events:

a)	Feeder-Link Availability to deliver data to EPC/5GC and further to the Satellite

b)	Service-Link Availability to deliver data to UE and receive acknowledgement

c)	Feeder-Link availability to deliver acknowledgement to EPC/5GC and further back to the AF

It may be useful to provide expected delivery time, potential round trip(if UE sends an ack on reception of this data) to AF so that retry etc. can be scheduled accordingly

Accordingly, it is proposed to define new monitoring events and expose those via NEF/SCEF to AF.

### 6.38.2	Procedures

It is proposed to add following events to be exposed by AMF/MME, UDM/HSS via NEF/SCEF to AF:

6.38.2-1

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Registration in S&F Mode | Detected when the UE registers in S&F Mode. The event report may include an indication of S&F Mode. | Ground-AMF/Ground-MME/UDM/HSS |
| Feeder Link Available | Network detects that Feeder Link is available for AF to send data to a UE. The event report may include duration for which the feeder link is available. Alternatively, the AF may query the network to determine when/frequency the feeder-link will be available next. | Ground-AMF/Ground-MME |
| Expected Delivery/Delay Time | The expected delay to deliver the data to the UE. This information may be informed to the AF based on query, or when data sent by AF is cached into EPC/5GC, or along with the "Feeder Link Available" event. The potential round trip time if UE acknowledges to the sent data packet | Ground-AMF/Ground-MME |

Editor's note:	Whether any existing monitoring events can be used or enhanced to achieve the events described in this solution is FFS.

### 6.38.3	Impacts on services, entities and interfaces

AF/UDM/AMF/HSS/MME/SCEF/NEF: Support of new monitoring events.

## 6.39	Solution #39: Session creation and parameters provisioning for S&F

### 6.39.1	Key Issue mapping

This solution is related to Key Issue #2 (Support of Store and Forward Satellite operation).

### 6.39.2	Description

Considering the limited storage capability of the MME/SGW on satellite, which performs S&F operation, the MME/SGW may configure storage quota for each UE, and may provision the parameter to UE for assisting UE to manage UL data sending to avoid exceeding the quota.

The Initial Attach / Initial Registration procedure is assumed to be resolved in other solution (e.g. Sol#12, Sol#13, Sol#16). The Session create is embedded in the attach procedure or uses the UE requested PDN connectivity after the attach procedure.

In this solution, the Session create procedure is designed in different framework which includes following 3 aspects for Control Plane CIoT EPS Optimisation:

-	Session create for S&F with MME, SGW and PGW onboard.

-	Session create for S&F with MME onboard.

-	Session create for S&F with split MME (as designed in Sol#11).

Editor's note:	It is FFS that if the satellite serving the UE has passed away, how to deal with the parameters provisioned to UE.

### 6.39.3	MME, SGW and PGW onboard

Figure 6.39.3-1 depicts the Session create procedure for S&F operation with MME, SGW and PGW onboard, which can be during attach procedure or UE requested PDN connectivity procedure. Other attach procedures (e.g. the Authentication, security setup) can be resolved in other solutions (e.g. Sol#16).

In this procedure, the MME performs the S&F operation, or the S-GW performs the S&F operation instructed by the MME.

Figure 6.39.3-1: Attach/PDN connectivity procedure with MME, SGW and PGW onboard

1.	When PDN Connectivity Request with the necessary info for the Control Plane CIoT EPS Optimisation is received, or during attach procedure, for example, ESM message container is not omitted in the attach request, then the MME initiates the session create procedure. MME sends a Create Session Request to SGW. The Create Session Request includes a S&F indication if MME decides to let the SGW store the data when feeder link is not available.

2.	SGW sends a Create session request to the PGW.

3.	If dynamic PCC is not deployed, step is skipped. If the dynamic PCC is deployed, the PGW initiates the IP-CAN session establishment until the feeder link is available.

4.	The PGW returns a Create Session Response to SGW.

5.	SGW returns a Create Session Response to MME, if indicated in step1 that the SGW stores the data, considering that there may be limited storage space in SGW, the SGW may additionally provide S&F parameters (e.g. allowed Storage space, retention period) to UE for UL data sending. The allowed Storage space is used to restrict the quota of UL data sending one time. The retention period may help UE be aware of how long the data is allowed to be stored in SGW.

6.	When the service link is available, MME sends the attach accept or PDN connectivity accept to the UE.

-	If it is the SGW to store UL data, the MME includes the S&F parameters provided by SGW as described in step 5 in the attach accept message to UE.

-	If it is the MME to store UL data ( i.e. the MME doesn't indicate SGW S&F operation in step 1), considering that there may be limited storage space in MME, the MME may additionally provide S&F parameters (e.g. allowed Storage space, retention period) to UE for UL data sending. The allowed Storage space is used to restrict the quota of UL data sending one time. The retention period may help UE be aware of how long the data is allowed to be stored in MME.

7.	UE sends the attach complete or PDN connectivity complete message to MME.

8.	After sending the complete message, if UE has the UL data transmission requirement, the UE shall ensure that the uplink data will not exceed the Storage space allowed for the UE mentioned in the attach accept message.

9.	MME sends the UL data to SGW if MME indicates the SGW to store the UL data, otherwise the MME store the UL data.

In the case of SGW and PGW not onboard (i.e. only MME onboard), the MME performs the S&F operation and will not include the S&F indication in the Create Session Request in step 1.

### 6.39.4	Impacts on services, entities and interfaces

MME:

-	Indicates the SGW to store the data.

-	May allocate a S&F parameters (e.g. allowed Storage space, retention period) to UE.

SGW:

-	May allocate a S&F parameters (e.g. allowed Storage space, retention period) to UE.

## 6.40	Solution #40: User plane offload from P-CSCF/IMS-AGW located on the ground

### 6.40.1	Description

#### 6.40.1.1	General

This solution addresses Key Issue #3 Support of UE-satellite-UE communication.

KI#3 refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network. In this case the UPFs need to be on the satellites to switch the media flows locally. However the IMS network entities including P-CSCF and IMS-AGW are normally located on the ground to avoid occupation of on-site resources.

As specified in TS 23.228 [9] and TS 24.229 [11], P-CSCF in IMS network acts as SIP proxy or IMS-AGW rules to provide features including SDP negotiation, Policy control, P-CSCF initiated dialog-release , transcoding, user plane security, etc. These features requires the IMS-AGW to anchor the user plane of the IMS session. In this case when the P-CSCF/IMS-AGW is located on the ground, the user plane traffic of IMS session has to be routed down to the ground network and back to the satellite after handling by the originating and terminating P-CSCFs and other optional IMS network functional entities, e.g. MRF, which means current IMS procedure can not fulfil the requirement of KI#3.

This solution aims at this use case that PSA UPFs and the IMS entities including P-CSCF/IMS-AGW are located on the ground, while ULCL/BP/local PSA UPFs are inserted during the session establishment procedure to bypass PSA UPFs and IMS-AGW on the ground if the condition for P-CSCF/IMS-AGW offload is met. The solution supports both the scenarios that the UEs are connected to the same or different satellite.

Editor's note:	Whether and how to support LI for the case IMS-AGW is not onboard is FFS.

In SIP level, this solution proposes enhancements to SDP negotiation procedures on the P-CSCF to allow the user plane traffic to be offloaded from IMS-AGW so that it can be routed directly between the on-board ULCL/BP/local PSA UPFs serving the originating and terminating UEs.

In 5GC level, in this solution the P-CSCF indicates SMF via PCF to insert on-board ULCL/BP/local PSA UPFs for IMS PDU session when it determines to use P-CSCF/IMS-AGW offload.

The UE in this solution provides the access information about the satellite and satellite system during the registration or session initiation procedures, which is used by P-CSCF to determine if the P-CSCF/IMS-AGW offload is possible and take action to offload the user plane traffic accordingly. The access information includes satellite identity, satellite constellation identity. In option 1 and 2 below, when the access information is sent to P-CSCF are the only difference between two options. In option 1, the access information is sent during registration procedure, while in option2, it is sent during session initiation procedure.

Editor's note:	The details of the access information is FFS.

#### 6.40.1.2	Architecture assumption

The demonstrating architecture for this solution is shown in Figure 6.40.1.2-1.

Figure 6.40.1.2-1: Demonstrating architecture

The UE1 and UE2 are connected to the different satellites which belong to the same satellite system with ISL connection. It is assumed the ISL is always available during the session. Each satellite has a local selected UPF serving the connected UE.

When the UEs establish PDU sessions for IMS, the SMFs serving different UE are assumed to select different P-CSCFs for the UEs. In the assumption the P-CSCF1/IMS-AGW and P-CSCF2/IMS-AGW are selected and serves UE1 and UE2 accordingly.

### 6.40.2	Procedures

#### 6.40.2.1	Session establishment procedure

##### 6.40.2.1.1	Procedure of option 1

The figure 6.40.2.1.1-1 depicts the procedures in which the UE includes satellite access information in REGISTER request and INVITE request.

Figure 6.40.2.1.1-1: Procedure of option 1

The procedure of option 1 is as follows:

1.	UE1 and UE2 has established PDU sessions for IMS. The SMF is on the ground and SMF has selected P-CSCF for their IMS registration. The PSA UPFs for both PDU sessions are on the ground. During the session, the SMF and PSA UPF are not changed.

2.	UE1 and UE2 registers to IMS network via P-CSCF including in the REGISTER request their satellite access information, e.g. satellite identification or satellite system information.

3.	P-CSCF1 and P-CSCF2 stores the satellite access information during this registration period accordingly.

4.	The UE1 initiates an IMS voice session with its SDP offer. The UE1 may also include its satellite access information if the information has changed after step 2.

5.	The P-CSCF1 stores the SDP offer from UE1 for possible further usage before continuing the normal session control and SDP negotiation. It may update the SDP offer to anchor the user plane to IMS-AGW1 for further handling of voice media, e.g. transcoding or NAT traverse.

6.	The P-CSCF1 sends INVITE request to terminating IMS network, in which the satellite access information of UE1 is included.

7.	When P-CSCF2 receives INVITE request, it stores the satellite access information of UE1 and continue the normal session control and SDP negotiation.

8-9.	The UE2 responses with its SDP answer in 18X response.

10.	The P-CSCF2 determines whether the user traffic is able to be offloaded based on the received SDP answer and the satellite access information of UE1 and UE2. If the UE1 and UE2 are connected to the same satellite or two satellites who belong to the same satellite system and the user plane does not need transcoding or NAT traverse, the P-CSCF decides to offload the user plane traffic from itself, i.e. the P-CSCF2 will not use IP address and port of IMS-AGW2 to anchor media plane transfer, instead it uses IP address and port of UE2 in SDP answer so that the media plane will be bypassed from the P-CSCF2.

11.	The P-CSCF2 uses the received UE2 SDP answer in the 18X response, which contains IP address and port of UE2 for media plane transfer.

12.	The P-CSCF2 also includes stored satellite access information of UE2 in the 18X response.

13.	When the P-CSCF1 receives the 18X response with satellite access information of UE2, it determines whether the user traffic is able to be offloaded with the same logic in step 10, i.e. the P-CSCF1 will not use IP address and port of IMS-AGW1 to anchor media plane transfer, instead it uses IP address and port of UE1 in another SDP offer so that the media plane will be bypassed from the P-CSCF1.

14.	The P-CSCF invokes Npcf_Policy Authorization_Create service operation to indicate PCF to insert ULCL/BP/local PSA UPF1 for IMS PDU session. UE1 IP address is included to correlate IMS PDU session. DNAI is generated based on satellite access information and is included for on-board UPF selection.

15.	The PCF invokes Npcf_SMPolicyControl_UpdateNotify to notify SMF to insert ULCL/BP/local PSA UPF for IMS PDU session. UE1 IP address and DNAI are included.

16.	The SMF inserts ULCL/BP/local PSA UPF for IMS PDU session as specified in clause 4.3.5.4 of TS 23.502 [3]. A UPF on-board is selected based on DNAI provided by P-CSCF.

17-18.	Responses are sent back to P-CSCF.

19.	The P-CSCF1sends UPDATE request using previously stored SDP offer from UE1 to re-negotiate the SDP for this IMS session to offload the user plane from IMS-AGW1. It includes an indication of P-CSCF/IMS-AGW offload to indicate the P-CSCF2 that the session will be offloaded.

20.	When the P-CSCF2 receives the UPDATE request with the indication of P-CSCF offload, it knows that P-CSCF offload is used for this session.

21-25.	Similar to steps 14-18, an on-board ULCL/BP/local PSA UPF2 for IMS PDU session is selected for UE2.

26.	The P-CSCF uses the received SDP offer, i.e. UE1 SDP offer, for SDP re-negotiation to UE2.

27.	The UE2 responses with its SDP answer in 200 OK response to UPDATE request.

28.	The 200 OK response is send to P-CSCF1.

29-30.	The P-CSCF1 includes received UE2 SDP answer in 18X response to UE1. After this step, UE1 and UE2 actually negotiate SDP directly so that the media plane will be transferred directly between two UEs.

31-32.	The UE2 is alerted and the IMS session is established. The user plane traffic is established between UE1 and UE2 via UPF1 and UPF2.

Editor's note:	How the connection between onboard UPFs (ULCL/BP/local PSA UPF1 and ULCL/BP/local PSAUPF2) is established is FFS.

The media traffic of the session is transported via on-board UPFs.

##### 6.40.2.1.2	Procedure of option 2

The figure 6.40.2.1.2-1 depicts the procedures in which the UE includes satellite access information only in INVITE request and response.

Figure 6.40.2.1.2-1: Procedure of option 2

The procedure of option 2 is as follows:

1.	UE1 and UE2 has established PDU sessions for IMS. The SMF is on the ground and SMF has selected P-CSCF for their IMS registration. The PSA UPFs for both PDU sessions are on the ground. During the session, the SMF and PSA UPF are not changed.

2.	UE1 and UE2 registers on the IMS network.

3.	The UE1 initiates an IMS voice session with its SDP offer. The UE1 includes its satellite access information, e.g. satellite identification or satellite system information.

4.	The P-CSCF1 stores the SDP offer from UE1 for possible further usage before continuing the normal session control and SDP negotiation. It may update the SDP offer to anchor the user plane to IMS-AGW1 for further handling of voice media, e.g. transcoding or NAT traverse.

5.	The P-CSCF1 sends INVITE request to terminating IMS network, in which the satellite access information of UE1 is included.

6-7.	When P-CSCF2 receives INVITE request, it stores the satellite access information of UE1 and continue the normal session control and SDP negotiation.

8.	The UE2 responses with its SDP answer in 18X response. In the response the UE2 includes its satellite access information.

9-31.	Same as step 10-32 in Figure 6.40.2.1.1-1.

#### 6.40.2.2	Satellite handover procedure

The figure 6.40.2.2-1 depicts the procedure when the on-board gNB and UPF change due to satellite moving.

Figure 6.40.2.2-1: Procedure of satellite handover

1.	UE1 and UE2 has established PDU sessions for IMS. The media traffic of IMS session is transferred via gNB and ULCL/BP/local PSA UPF on the source satellite.

2.	When the satellite moves, the gNB and ULCL/BP/local PSA UPF serving UE1 change to the target satellite.

3.	After gNB handover, the target gNB sends N2 Path Switch Request to AMF1, indicating the switch is for IMS session.

4.	The AMF invokes Nsmf_PDUSession_UpdateSMContext to provide SMF with N2 SM information.

5.	The SMF knows the satellite change so it initiates insertion of new ULCL/BP/local PSA UPF on the target satellite.

6.	After new ULCL/BP/local PSA UPF on the target satellite is inserted, the SMF release old ULCL/BP/local PSA UPF on source satellite.

7-8.	The SMF responds to AMF, which further responds to source gNB.

9.	The target gNB release the resource on the source gNB.

The media traffic is transferred via new ULCL/BP/local PSA UPF after the handover.

### 6.40.3	Impacts on existing services, entities and interfaces

UE impacts:

-	UE includes satellite access information in REGISTER, INVITE request and response message.

P-CSCF impacts:

-	P-CSCF stores the satellite access information and includes the information in REGISTER, INVITE request and response message.

-	P-CSCF determines whether the user traffic is able to be offloaded based on the received SDP answer and the satellite access information of originating and terminating UEs.

-	After determining to offload user plane, P-CSCF uses SDP offer and SDP answer from UE to negotiate the user plane instead of anchoring user plane on the P-CSCF.

-	P-CSCF initiates insertion of ULCL/BP/local PSA UPF for IMS session.

PCF:

-	PCF indicates SMF to insert ULCL/BP/local PSA UPF for IMS session.

SMF:

-	SMF selects UPF on-board based on DNAI and inserts ULCL/BP/local PSA UPF for IMS session;

-	SMF supports change of ULCL/BP/local PSA UPF when the serving satellite changes.

IMS-AGW:

-	no impact is seen.

## 6.41	Solution #41: Support of UE-satellite-UE communication for IMS services with UPF/AGW on board

### 6.41.1	Description

This solution resolves the Key Issue #3 to support UE-satellite-UE communication for IMS services.

This solution assumes that the UPF/AGW on board of the satellite, and the ground PSA UPF was selected when the PDU Session is established, the UPF on board satellite is inserted into the data path to act as UL CL and Local PSA when the voice call is established, as shown in the Figure 6.41.1-1.

Figure 6.41.1-1: IMS Signalling path and Data path for UE-satellite-UE communication

When the AGW is on board of the satellite, the media packets (e.g. RTP packets) may be transferred between AGWs of different satellite, how to perform handover between satellites is introduced in this solution.

#### 6.41.1.1	Reference Architecture

For this solution, gNB, UPF and AGW are deployed on the satellite, as shown in the Figure 6.41.1-1.

Figure 6.41.1.1-1: Network architecture with IMS AGW deployed on each satellite

A satellite may have multiple ISLs with multiple satellites, when an AGW receives a media packet from UE, it is not clear which ISL should be used to send the received media packet. The simplest method is trial and error method, that is, the AGW sends the media packet to each ISL it has. This method will waste the ISL resources, which is not acceptable.

AS depicts in clause G.4.3 of TS 23.228 [9], AGW will allocate IP address to the serving UE, and this IP address is used by peer UE side to use as target IP address. To resolve the problem, it suggests that each AGW on board satellite(s) is allocated a specific IP range, and the relationship between the IP range information and satellite ID are pre-configured to each AGWs on board satellite(s). When one AGW receives a media packet from UE, the AGW determines target satellite ID based on pre-configured information and the target IP address, then forwards the media packets to the target satellite based on the target satellite ID directly, as shown in the Figure 6.41.1.1-2.

Figure 6.41.1.1-2: IMS Data path for UE-satellite-UE communication

Editor's note:	The user of dynamic IP addresses is FFS.

For example, when the AGW receives a media packet from UE-1, the AGW determines the target IP address is 10.10.6.5 which is in the IP range of SAT-1, then the AGW can determine the satellite ID of SAT-1, and use the ISL with SAT-1 to forward the media packet.

### 6.41.2	Procedures

#### 6.41.2.1	Procedure when serving satellite changes

Figure 6.41.2.2-1 depicts the example scenario of the handover procedure: UE#1 handover from SAT-1 to SAT-2, and UE-2 states in SAT-2 unchanged.

Figure 6.41.2.2-1: Example scenario of the handover procedure

Based on the procedure of clause 4.3.5.7 "Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session" in TS 23.502 [3]. When the Xn or N2 based handover is performed for UE-1, the UL CL and local PSA are also changed to the target satellite. Please refer to Figure 6.41.2.2-2.

Editor's note:	Whether the procedures for the UL CL insertion need upgrading or not is FFS.

Figure 6.41.2.2-2: Media path based on existing procedure

The AGW-1 is unchanged, as a result, the media packets between UE-1 and UE-2 still need to be routed back to the SAT-1, this will waste the ISL resources.

Therefore, it suggests that the AGW should also be changed to SAT-2, please refer to Figure 6.41.2.2-3.

Figure 6.41.2.2-3: media path after AGW relocation

Figure 6.41.2.2-4 depicts how to relocate the AGW to SAT-2 after the handover procedure.

Figure 6.41.2.2-4: Handover procedure when serving satellite changes

0.	UE#1 and UE#2 is in UE-SAT-UE communication. UE#1 is served by satellite SAT#1 and UE#2 is served by satellite SAT#2. The media path between the UEs is:

UE#1↔SAT#1(gNB#1+UL CL#1+PSA2#1+AGW#1) ↔SAT#2(AGW#2+UL CL#2+PSA2#2+gNB#2) ↔UE#2.

1.	Handover procedure is performed as specified in clause 4.9.1 of TS 23.502 [3], and Simultaneous change of UL CL and local PSA is performed as specified in clause 4.3.5.7 of TS 23.502 [3].

2.	The uplink media path between the UEs is:

UE#1→SAT#2(gNB#2+UL CL#2)→SAT#1(UL CL#1+PSA2#1+AGW#1)→SAT2(AGW#2+PSA2#2+UL CL#2+gNB#2)→UE#2.

3.	The downlink media path between the UEs is:

UE#2→SAT#2(gNB#2+UL CL#2+PSA2#2+AGW#2) →SAT#1(AGW#1+PSA2#1+UL CL#1) →SAT#2(UL CL#2+ gNB#2)→UE#1

4.	PCF#1 obtains UE#1's latest Access Network Information from SMF#1 as specified in TS 23.501 [2].

The Access Network Information includes UE#1's latest satellite ID.

5.	The PCF sends Npcf_PolicyAuthorization_Notify with SAT#2's satellite ID to P-CSCF as specified in clause 5.2.5.3 of TS 23.502 [3] when determining UE#1's satellite ID is changed.

P-CSCF subscribes the Access Network Information from PCF during the UE-SAT-UE communication establishment procedure

6.	P-CSCF determines whether the USU communication can be continued based on UE#1's new satellite ID and stored UE#2's satellite ID, if yes performs step 6.

For example, based on satellite ID of UE1 and UE2, P-CSCF#2 can, based on configuration, determine whether the two UEs are connected to the same satellite or with interconnected satellites (through ISL), if yes, the USU communication can be continued.

Editor's note:	It's FFS whether and how the P-CSCF determines the UE-Satellite-UE communication cannot be continued.

7.	The P-CSCF sends a SIP MESSAGE request to the S-CSCF to request the S-CSCF to notify UE#1 to send a SIP re-Invite to perform SDP re-negotiation.

8.	S-CSCF sends a SIP NOTIFY to UE#1 indicating that UE#1 should send a re-INVITE request.

9.	UE#1 sends SIP re-Invite request contains SDP offer[c=IP-1].

IP-1 is the IP address of UE#1.

10-11.	The P-CSCF#1 selects the AGW#2 onboard SAT#2 based the satellite ID received in step 5 and performs the procedure of clause G.4.3 in TS 23.228 [9] to obtain the transport address: Tr_IP-1.

Editor's note:	How the media path is transferred from AGW-1 to AGW-2 is FFS.

12-13.	The P-CSCF#1 changes the IP-1 in the SDP offer to Tr_IP-1 received from the AGW#2, and forwards the SIP message with the modified SDP offer to UE#2.

14-15.	P-CSCF#1 receives 200 OK with SDP answer[c=IP-2]

IP-2 is the IP address allocated by AGW#2 for UE#2.

16-17.	The P-CSCF#1 requests a transport address from AGW#2 as specified in clause G.4.3 of TS 23.228 [9] and AGW#2 returns the transport address: Tr_IP-2.

18.	P-CSCF#1 trigger PCF#1 and SMF#1 to perform the PDU session modification procedure

P-CSCF#1 informs PCF#1 the Tr_IP-2 via "fDescs" attribute, and PCF#1 provides to SMF#1 via PCC rule. The SMF#1 sends a new UL CL rule contains Tr_IP-2 to UL CL#2, so that the UL CL#2 will sends the media packets from UE#1 to PSA2#2, then PSA2#2 forwards to AGW#2.

The N9 forwarding tunnel between UL CL#1 and UL CL#2 can be released by SMF#1 when there are no active traffic as specified in clause 4.3.5.7 of TS 23.502 [3].

19.	The P-CSCF#1 changes the IP-2 in the SDP offer to Tr_IP-2 received from the AGW#2, and forwards the 200 OK with the modified SDP offer to UE#1.

20.	After step 19, the media path between the UEs is:

UE#1↔SAT#2(gNB#2+ UL CL#2+PSA2#2+ AGW#2+AGW#2+PSA#2+UL CL#2+gNB#2) ↔UE#2.

### 6.41.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

SMF:

-	Provision to PCF the serving satellite ID for Regenerative-based satellite access.

PCF:

-	Provision to P-CSCF serving satellite ID for Regenerative-based satellite access.

P-CSCF:

-	Selection of AGW onboard satellite.

-	Determines whether the USU communication can be continued.

-	Request S-CSCF to trigger UE to send SIP re-Invite message.

-	connection to different PSA-UPF than AGW.

S-CSCF:

-	Trigger UE to send SIP re-Invite message.

AGW:

-	connection to different PSA-UPF than P-CSCF.

NOTE:	This solution assumes that P-CSCF and AGW are not co-located.

## 6.42	Solution #42: UE-satellite-UE communication in the context of LEO satellite constellations with ISL

### 6.42.1	Description

This solution resolves Key Issue #3: Support of UE-satellite-UE communication and also Key Issue #1. The focus is on architectural enhancements to support UE-satellite-UE communication particularly for IMS voice and video service via a satellite constellation of LEO satellites. Multiple LEO satellites form a satellite constellation and are connected via ISL. To achieve this, it is necessary from an architectural perspective to place gNB, UPF and IMS AGW onboard the satellites. Additionally, their representative Relay entity is ground-based. This entity refers to the satellite astronomical calendar based on satellite ephemeris to determine the opposing nodes. This solution is based on the following assumptions:

-	The satellite onboard entities i.e. onboard gNB, UPF, and IMS AGW are owned and operated by a Satellite network operator and are used by multiple MNOs in shared manner.

-	The ground-based Relay entity is owned and operated by a Satellite network operator and is used by multiple MNOs in shared manner.

AMF and SMF treats theRelay entity as a usual gNB and UPF. The P-CSCF treats the Relay as a usual IMS AGW. Satellite constellations are identified by Satellite constellation IDs.

Similarly, the Relay entity behaves as an AMF to onboard gNB, as SMF to onboard UPF, and as a P-CSCF to onboard IMS AGW.

NOTE 1:	These ground-based representative entity serves as control plane entity and hence the routing of the user plane traffic remains in the satellite.

Architectural assumptions explained above is depicted based on Figure C-3 as follows:

Figure 6.42.1-1: Basic concept of architectural assumptions of this solution

This solution is based on the Rel-18 approach agreed in TS 23.228 [9], TS 24.229 [11] and TS 23.502 [3] related to PDU Session Establishment, IMS registration signalling, and the IEs included in the messages.

NOTE 2:	All the messages for communication described in the procedures are standardized messages with potential new elements such as "satellite constellation ID".

Editor's note:	The HO procedures when the satellite moves is FFS.

### 6.42.2	Procedures

#### 6.42.2.1	Association of Relay entity representative of satellite constellations

The registration procedure for the Relay entity with AMF, SMF, and P-CSCF respectively is as illustrated in Figure 6.42.2.1-1. The identifier for the constellations "satellite constellation ID" is used.

The ground-based representative Relay entity of satellite constellations establish a connection with each of the counterpart nodes (onboard) deployed on each of the LEO satellites that form a part of the satellite constellation and that can be connected from the ground at a specific time period derived from the satellite astronomical calendar based on satellite ephemeris.

Figure 6.42.2.1-1: Procedure for association of Relay entity representative of satellite constellations

1.	The ground-based Relay sends the NG setup request including the satellite constellation ID to the AMF.

2.	This is followed by the response message.

3.	The ground-based Relay sends the N4 Association setup request message including the satellite constellation ID to the SMF.

4.	This is followed by the response message.

5.	The ground-based Relay sends the IMS AGW Register message including the satellite constellation ID to the P-CSCF. This is according to the procedure in TS 23.334 [21].

6.	This is followed by the acknowledgement message.

#### 6.42.2.2	Registration Procedure

Figure 6.42.2.2-1: Registration for UE-satellite-UE communication.

1.	When a UE initiates a call, an RRC connection is set up between the UE and its serving gNB (onboard). The gNB then refers to the list of [start time, end time, to-be-connected Relay] created based on the satellite ephemeris and selects the appropriate Relay entity.

Editor's note:	How the list of [start time, end time, to-be-connected Relay] is created based on satellite ephemeris is FFS.

2.	The gNB forwards the GUAMI in its Initial UE message. The inclusion of GUAMI in the Initial UE message is essential for the Relay to select the appropriate AMF.

3.	The ground-based Relay sends Initial UE message which includes the Registration request to the selected AMF.. Registration procedure continues according to clause 4.2.2.2.2 of TS 23.502 [3].

4.	The AMF sends the Registration accept response via Downlink NAS Transport message to the Relay entity.

5.	Relay sends the Registration accept response via Downlink NAS Transport message to the onboard gNB.

6.	The onboard gNB sends the Registration accept message to the UE.

#### 6.42.2.3	IMS PDU Session Establishment Procedure

Figure 6.42.2.3-1: IMS PDU session establishment procedure.

1-3.	AMF receives PDU session establishment request for IMS DNN via the Relay. Based on the locally stored information created by the procedure in Clause 6.42.2.1, the AMF recognizes that the request comes from a UE served by a particular satellite constellation.

4-5.	AMF sends the Nsmf_PDUSession_CreateSMContext request message including a satellite constellation ID to SMF. SMF selects a Relay corresponding to the satellite constellation ID

6-8.	SMF sends N4 session establishment request to the Relay. This includes the PDU session ID, which is used so that the context in UPF related to the PDU session ID can be easily found. The Relay entity refers to the list of [start time, end time, to-be-connected UPF] created based on the satellite ephemeris and selects the onboard UPF. N4 Session Establishment request/response is exchanged between the Relay and onboard UPF followed by N4 Session Establishment response sent to SMF.

NOTE 1:	PDU session ID to find context in UPF can be used in the procedure for possible HO procedures between LEO satellites.

Editor's note:	How the list of [start time, end time, to-be-connected UPF] is created based on satellite ephemeris is FFS.

Upon session establishment, the SMF sets IP address of P-CSCF tailored to treat satellite communication in Extended protocol configuration options (ePCO). The SMF may allocate an IP address to the UE from the IP address range of that particular satellite constellation ID.

NOTE 2:	Since the SMF allocates IP address to UE from a pool of IP address range belonging to a satellite constellation ID, the IP address of the UE does not change during HO process when a satellite moves.

9.	Namf_Communication_N1N2MessageTransfer including the P-CSCF address in ePCO is sent from SMF to AMF.

10-12.	PDU Session Establishment accept is sent from AMF to onboard gNB via the Relay.

11.	PDU Session Establishment accept is sent to UE.

#### 6.42.2.4	IMS registration and voice call setup procedure

##### 6.42.2.4.1	IMS registration

Figure 6.42.2.4.1-1: IMS registration

1.	The UE sends Register to the P-CSCF IP address available to it from the ePCO (see Clause 6.42.2.3).

2.	P-CSCF sends Register to S-CSCF.

3.	Registration of S-CSCF in HSS.

4.	Nhss_ImsSDM_Get request/response between S-CSCF and HSS.

5-6.	200 OK messages from S-CSCF to P-CSCF and from P-CSCF to UE.

##### 6.42.2.4.2	Voice call setup procedure - MO

Figure 6.42.2.4.2-1: Voice call setup procedure - MO

1.	A voice establishment request message (SIP INVITE) is received at the P-CSCF.

The P-CSCF selects the Relay belonging to the satellite constellation ID based on UE's IP address and the corresponding satellite constellation ID.

2-4.	Message for controlling the access of the voice media is sent to the selected Relay. The Relay refers to the list of [start time, end time, to-be-connected IMS AGW] created based on the satellite ephemeris and selects the onboard IMS AGW. Allocation request is sent to reserve a resource of a terminating point in its own equipment for data communication with the callee side, followed by response.

Editor's note:	How the list of [start time, end time, to-be-connected IMS AGW] is created based on satellite ephemeris is FFS.

5.	SIP INVITE is transferred to S-CSCF. Interaction with the terminating network continues.

6.	SIP 183 Session Progress response is sent to S-CSCF.

7-9.	Request/response to set a terminating point (Terminating UE IP address).

10-12.	Allocation request to set a terminating point in the caller and to reserve a resource of a terminating point in its own equipment with regards to data communication with the caller.

13.	SIP 183 Session Progress message is sent to UE.

14.	Npcf_PolicyAuthorization_Create request message is sent from the P-CSCF to the PCF. The PCF which recognized the PDU session ID from PDU session establishment procedure sends the PDU session ID in response to the P-CSCF.

15-17.	Context update request/response messages exchanged between P-CSCF and IMS AGW via the Relay. This includes the PDU session ID.

NOTE:	This is a newly introduced message exchange. PDU session ID can be used in possible procedures for HO between LEO satellites.

Editor's note:	The media path needs to be added to the procedure description and figure.

##### 6.42.2.4.3	Voice call setup procedure - MT

Figure 6.42.2.4.3-1: Voice call setup procedure - MT

1.	SIP INVITE is received at I-CSCF.

2-3.	I-CSCF looks for S-CSCF to forward SIP INVITE.

4-5.	SIP INVITE is forwarded to S-CSCF then to P-CSCF.

6.	P-CSCF selects the Ip intermediary belonging to the satellite constellation ID based on the terminating UE's IP address and the corresponding satellite constellation ID. A message for controlling the access of the voice media is sent to the Ip intermediary.

7-9.	The Ip intermediary refers to the list of [start time, end time, to-be-connected IMS AGW] created based on the satellite ephemeris and selects the onboard IMS AGW. Allocation request is sent, followed by response.

10.	SIP INVITE is transferred to UE.

11.	UE sends SIP 183 Session Progress to P-CSCF.

12-15.	Modification request is sent, followed by response.

16-19.	Allocation request is sent, followed by response.

20-22.	SIP 183 Session Progress is sent to I-CSCF.

23-24.	Npcf_PolicyAuthorization_Create request message is sent from P-CSCF to PCF. The PCF which recognized the PDU session ID from PDU session establishment procedure sends the PDU session ID in response to the P-CSCF.

25-28.	Context update request/response messages exchanged between P-CSCF and IMS AGW via the Ip intermediary. This includes the PDU session ID.

Editor's note:	The media path needs to be added to the procedure description and figure.

### 6.42.3	Impacts on services, entities, and interfaces

gNB on board:

-	Capability to use satellite ephemeris and to discover an appropriate ground-based Relay.

Relay:

-	A new network node that behaves as a gNB to AMF on the ground and as an AMF to gNB on board.

-	Capability to use satellite ephemeris and to discover an appropriate gNB on board.

-	Behaves as a UPF to SMF on the ground and as an SMF to UPF on board.

-	Capability to use satellite ephemeris and to discover an appropriate UPF on board.

-	Behaves as an IMS AGW to P-CSCF on the ground and as a P-CSCF to IMS AGW on board.

-	Capability to use satellite ephemeris and to discover an appropriate IMS AGW on board.

AMF:

-	Capability to identify a request coming from Relay entity and to set satellite constellation ID to a message to SMF.

SMF:

-	Capability to select Relay entity based on satellite constellation ID.

-	Capability to assign IP address to UE from a pool of IP address range belonging to a satellite constellation ID.

P-CSCF:

-	Capability to identify the need to select Relay based on UE IP address.

-	Capability to select Relay entity based on satellite constellation ID.

-	Capability to retrieve PDU session ID from PCF and to send it to Relay entity.

## 6.43	Solution #43: UE-Sat-UE communication using resource allocation on-board satellite

### 6.43.1	Key Issue mapping

This solution aims to resolve Key Issue #3, " Support of UE-satellite-UE communication".

### 6.43.2	Description

This solution aims to ensure that the routing of user plane traffic for IMS (MMTEL) remains in the satellite(s) when both parties are served by a satellite of the same constellation. It considers that both parties could be served by the same or different satellites involving same or different SMF, UPF, PCF, P-CSCF.

This solution considers potential impacts on IMS procedures and functions to manage UE-satellite-UE communication, trying to minimize these IMS impacts: for this purpose, the solution assumes all the IMS subsystems remain on the ground with the possible exception of IMS-AGW that may be deployed on satellite that serve jurisdictions where LI may apply (See clause 6.43.3.5).

Figure 6.43.2-1 UE-Sat-UE IMS user plane offload

NOTE 1: The C-plane line shows the entities which may be involved in signalling, not necessarily the exact signaling path. The IBCF is not used when calling and the called party correspond to the same IMS Home network.

This solution is based on the following principles:

- For the IMS related PDU Session, UE(s) get an IP address from a PSA UPF located on the ground. Thus, their IP address is not changed when they change satellite.

- UE(s) register onto IMS with this IP address.

NOTE 2: 	Thus UE(s) keep their IP address constant and don’t need to reregister when there is change of serving satellite.

- Upon UE registration, the P-CSCF may subscribe to the PCF for the UE connectivity via satellite constellation: at subscription, it gets notified (whether/that) the UE is served by which satellite constellation and then gets notified when the serving satellite constellation changes for the UE.

This assumes that the AMF notifies the SMF (that notifies the PCF) about whether a UE is served by a given satellite constellation and that such notifications are also sent when the UE changes satellite constellation.

- The solution assumes, ISL(s) can be set-up within the same satellite constellation or across different constellations depending on satellite operator’s deployments (SLA). The set of ISL(s) build up an IP network which is outside of the scope of 3GPP.

- The fact that a UE is served by a satellite of a constellation may be carried in a (e.g PANI P-Access-Network-Info defined in clause 4.4 of RFC 3455) SIP header that is sent (for an IMS call) between the originating and the terminating party’s P-CSCF but shall never be sent to the other UE. For example, this information will be added by the originating P-CSCF and removed by the terminating P-CSCF at the border of the trust domain.

- Based on SIP header information (e.g. PANI) associated with the remote party of a call and on the information received from the PCF for the local UE, a P-CSCF may detect that both parties of a call are served by the same satellite constellation or served by satellite constellation that supports inter constellation ISL (N6 termination towards each other). This applies for both originating and terminating P-CSCF. In this case, P-CSCF will not anchor the media in an IMS-AGW.

NOTE 3: The PANI header is not changed in this proposal. The P-CSCF uses both PANI and PCF notified access information to determine the possibility of UE-Sat-UE call.

- An ISL IP network using ISL links is used for UE-Sat-UE user plane communication where UPF (I-UPF) onboard satellite can route U-plane packets to other user’s satellite’s UPF (I-UPF). PtP tunnelling can be applicable for I-UPF to I-UPF packet forwarding where the media flows with inner source/destination IP address of both UE(s) are sent in an IP over IP tunnel where the outer source/destination IP addresses correspond to the I-UPF(s) serving each of the UE(s). It is assumed that inter I-UPF in satellite data traffic use a dedicated PtP tunnel over ISL IP network.

- NOTE 4: The PtP tunnelling mechanism follows the same concept as described in clause 9.2 in TS 29.561. The PtP tunnel information can be preconfigured per ISL communication path. The Satellite identification can be mapped to each PtP tunnel endpoint to enable I-UPF (UL-CL) to select the N6 forwarding endpoint based on the other party’s satellite access (i.e. PtP tunnel endpoint pointing to other UEs N6 endpoint). As described in clause 9.3 in TS 29.561 and clause 4.3.17.8.3.3.3 of 3GPP TS 23.401, the PtP tunnelling setup via other mechanism such as GRE/PMIP, L2TP are not precluded in this solution proposal. if the PtP tunnel information is not configured against the ISL where other UE is present, then the call setup needs to be done via ground network and I-UPF allocation is no needed. This check against the presence of PtP towards the other’s UE satellite is done also during HO procedure to make sure, call continuity is maintained, if not the call reestablishment via ground network needs to be carried out. The PtP tunnel can be per Sat-Sat pair and there can be multiple PtP tunnel between 2 satellite and it is up to the UPF to choose which PtP tunnel to use. The PtP information over the satellite ISL is exchanged via P-CSCF after collecting the information from I-UPF->SMF->PCF during allocation of resources for IMS PDU, as described in step 10-12 in 6.43.3.2 and step 10-13 in 6.43.3.3

The description below applies for the P-CSCF / PCF / SMF at the first side of an IMS call where QoS resources are reserved:

NOTE 5: 	this corresponds for example, to the procedure in clause 6.43.3.2.

- When the P-CSCF invokes PCF services to request Authorization of QoS resources and to fetch the user location (ANI) from the access network (as defined in TS 23.228 e.g. step 6b of Figure R.3-1, step 5 of Figure R.4-1), the P-CSCF may ask to insert a local N6 breakout point for the corresponding QoS resources and to report addressing information for this local N6 breakout point.

NOTE 6: 	This is done using Rx/N5 services and not Nnef Traffic Influence API. Using Nnef Traffic Influence API would be possible but would require more API invocation between P-CSCF (acting as an AF) and the PCF which in turn would have induced more N7 and N4 signalling. This would increase the call set-up delay.

- The PCF via PCC rules, transfers the request to the SMF; the SMF:

b1)	 (based on the PCF request to ask to insert a local N6 breakout point) allocates for the Session (a) I-UPF(s) on the satellite currently serving the UE and configures the UPF(s) to act as UL CL (to break out only traffic corresponding to the SDF indicated by the PCF) and/or as L-PSA (local N6 breakout point); it retrieves from the L-PSA UPF the local (outer IP) addressing information corresponding to the local N6 breakout point;

b2) requests QoS resources and location information from the RAN;

b3) reports to the PCF ANI that contains both the location information received from the RAN and (outer IP) address information corresponding to the local N6 breakout point (and retrieved from the L-PSA).

- The PCF reports the information provided by the SMF to the P-CSCF that ensures that the (outer IP) addressing information corresponding to the local N6 breakout point is transferred via SIP header information (e.g. PANI) or via SDP to the P-CSCF handling the other party.

- When the P-CSCF receives (outer IP) addressing information corresponding to the N6 breakout point of the other party it transfers the information to the PCF that updates the corresponding PCC rule to the SMF. The SMF updates the PSA UPF with the received (outer IP) addressing information to be used for UL media traffic to be sent on the ISL IP network.

The description below applies for the P-CSCF / PCF / SMF at the second side of the call where QoS resources are reserved. The description for the first side of the call where QoS resources are reserved above applies to the other side / party of the call where QoS resources are reserved with the following modification:

- If Step a) and Step d) take place at the same time, the P-CSCF, having received remote (outer IP) addressing information from the remote party before it has requested Authorization of QoS resources, can at the same time request local traffic switching and provide remote (outer IP) addressing information.

- this corresponds to the procedure in clause 6.43.3.3

### 6.43.3	Procedures

#### 6.43.3.1	Overview

The call flows of clause 6.43.3.2 describe the case of an SDP Offer-answer scenario where resources are first reserved when the SDP Answer is processed by the local P-CSCF and then resources are reserved at the side of the SDP offer.

#### 6.43.3.2	PCC and SMF Procedures for IMS Session Establishment at terminating P-CSCF and PCF

This clause details the changes to be made to TS 29.513 Annex B.2.2. it uses the same step numbers.

Figure 6.43.3.2-1: PCC Procedures for IMS Session Establishment at terminating P-CSCF and PCF

0.	at IMS initial registration onto the P-CSCF the P-CSCF subscribes (Npcf_PolicyAuthorization_Subscribe targeting the UE IP address in the contact header) onto the PCF about (Event = [Access Network Information Reporting-> “Current Satellite Constellation ID”]) the UE connectivity via satellite connection: at subscription time (via immediate reporting) it gets notified (whether/that) the UE has changed the satellite constellation (Npcf_PolicyAuthorization_Notify). The P-CSCF stores the association between the UE and a satellite constellation.

1.	The P-CSCF receives the SDP parameters defined by the originator together with the originated side IMS core generated SIP header (e.g. PANI), indicating that the originator is served by a satellite of a given constellation (Satellite ID+ Satellite constellation ID). The following assumes both UEs involved in the call are served by the same satellite constellation or satellites across constellation having ISL (detected based on a match between the remote UE information received in the SIP header (e.g. PANI) and the local UE information retrieved at step 0)

2.	The P-CSCF identifies the connection information needed (IP address of the up-link IP flow(s), port numbers to be used, etc.).

3.	The P-CSCF sends the SDP offer to the UE (removing any SIP header info (e.g. PANI) received from the other party).

4.	The P-CSCF receives the negotiated SDP parameters from the UE.

5.	The P-CSCF identifies the connection information needed (IP address of the downlink IP flow(s), port numbers to be used, etc.) to request resource reservation.

6.	The P-CSCF invokes the Npcf_PolicyAuthorization_Create service operation to forward the derived service information to the PCF.

6a.	(if Rx applies instead of N5) The P-CSCF forwards the derived service information to the PCF by sending a Diameter AAR for a new Rx Diameter session.

In both cases (6. and 6a), the P-CSCF uses the same request to also fetch the local user location (Access Network Information ANI) and/or UE Time Zone information from the access network (as defined in TS 23.228 e.g. step 6b of Figure R.3-1, step 5 of Figure R.4-1), and asks to insert a local N6 breakout point for the corresponding QoS resources and to report addressing information for this local N6 breakout point.

If the P-CSCF assigned IMS-AGW resources before in the context of step 2, it releases the resources.

7.	The PCF stores the received session information and performs Session binding. For the N5 interface, the PCF creates an "Individual Application Session Context" resource to store the received application session information.

8.	The PCF sends a [an HTTP "201 Created"]  Npcf_PolicyAuthorization_Create response to the P-CSCF [and includes the URI of the "Individual Application Session Context" resource in the Location header field. (8a.) In case 6a applies The PCF sends a Diameter AAA to the P-CSCF].

9.	If the P-CSCF did not request “access network information” in step 6 (or step 6a for the Rx case), upon reception of the acknowledgement from the PCF, the SDP parameters in the SDP answer are passed to the originator.

10.	The PCF executes interactions according to clause 5.2.2.2.1 in TS 29.513. This step implies provisions related PCC rules corresponding to the information received in step 6/6a. This is executed in parallel with steps 8 (or step 8a for the Rx case) and 9.

Upon reception of the PCC rules the SMF:

b1)	 (based on the PCF request to insert a local N6 breakout point) allocates for the PDU Session (a) (an) I-UPF(s) on the satellite currently serving the UE and configures the UPF(s) to act as UL CL (to break out only U-plane traffic corresponding to the SDF indicated by the PCF) and/or as L-PSA (local N6 breakout point); the SMF creates a PDR/FAR referring to the network of ISL(inter-satellite links) and the UPF answers back with addressing information corresponding to the local N6 breakout point (termination point at the L-PSA of the N6 tunnel over the ISL network)

b2) requests QoS resources and ANI (Access Network Information) from the RAN

b3) reports ANI to the PCF that contains both the information received from the RAN and address information corresponding to the local N6 breakout point (termination point at the L-PSA of the N6 tunnel over the ISL network).

NOTE 1: The N6 termination point routing/next hop information is provided by PCF based on satellite ID/Satellite constellation ID.

NOTE 2: If the N6 termination information cannot be determined at this time, the PCF shall wait for step 14 to receive the same from the originating side once the originating side setups up its own U-plane resources.

11.	If the P-CSCF requested access network information and/or EPS fallback indication in step 6, the PCF invokes the Npcf_PolicyAuthorization_Notify service operation to forward EPS fallback indication, if received in step 10, and/or the ANI received in step 10 [by sending an HTTP POST request to the Notification URI received in step 6].

11a.	Rx equivalent of the N5 procedure described in step 11.

12.	If step 11 occurs, the P-CSCF acknowledges the receipt of the notification request with an [HTTP "204 No Content"] Npcf_PolicyAuthorization_Notify response.

12a.	If step 11a occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

13.	If step 11 occurs (or step 11a for the Rx case), the P-CSCF forwards the SDP answer and includes the network-provided location information (ANI) in the next SIP message the P-CSCF sends towards the IMS core network.

As a difference with R18 behaviour, the IMS Core of the local UE ensures that information corresponding to the local N6 breakout point is transferred via the SIP header (e.g. PANI) towards the P-CSCF handling the other party (here originator of the call).

14. The terminating P-CSCF receives addressing information corresponding to the remote outer tunnel N6 breakout point transferred via SIP header (e.g. PANI) (and ensures this information is not sent to the UE). This step can correspond to SIP UPDATE and SIP 200 OK (UPDATE) carrying remote end information.

15.	The P-CSCF provides (via PCF) the SMF with addressing information corresponding to the remote outer tunnel N6 breakout point. This information is sent to the SMF via traffic steering information in a PCC rule (refer to clause 6.3.1 in TS 23.503 for “Application Function influence on traffic routing Enforcement Control”)

16.	the SMF Sends a corresponding N4 PDR/FAR to the UPF in such a way that media from the local UE to the remote UE is sent by the local PSA (I-UPF) (inserted in step 10 b1) in a tunnel whose destination address corresponds to the addressing information of the remote outer tunnel N6 breakout point: the SMF updates the corresponding FAR/Forwarding Parameters / Outer Header Creation accordingly.

If the addressing information of the remote outer tunnel N6 breakout point corresponds to the local PSA (i.e. if both the UEs are served by the same satellite), then local traffic switching at the L-PSA I-UPF needs to take place.

NOTE 3: The local switching can be done by UPF based on its internal mechanism or by keeping a dedicated PtP tunnel for loopback routing.

#### 6.43.3.3	PCC and SMF Procedures for IMS Session Establishment at Originating P-CSCF and PCF

This clause details the changes to be made to TS 29.513 Annex B.2.1. it uses the same steps numbers.

Figure 6.43.3.3-1: PCC Procedures for IMS Session Establishment at originating P-CSCF and PCF

1.	The P-CSCF receives the SDP parameters defined by the originator within an SDP offer in SIP signalling.

2.	The P-CSCF identifies the connection information needed (IP address of the down link IP flow(s), port numbers to be used, etc.).

3.	The P-CSCF forwards the SDP offer in SIP signalling.

4.	The P-CSCF gets the negotiated SDP parameters from the terminating side through SIP signalling interaction together with terminating side IMS core generated SIP header (e.g. PANI) indicating that the terminating side is served by a satellite of a given constellation (Satellite constellation ID) and/or I-UPF’s local termination information. (This corresponds to Step13 in Figure 6.43.3.2-1 from terminating side)

5.	The P-CSCF identifies the connection information needed (IP address of the up-link media IP flow(s), port numbers to be used, etc.).

6.	The P-CSCF invokes the Npcf_PolicyAuthorization_Create service operation to forward the derived session information to the PCF by sending an HTTP POST request to the "Application Sessions" resource.

6a.	The P-CSCF provides session information to the PCF by sending a Diameter AAR for a new Rx Diameter session.

7.	The PCF stores application session information and performs session binding. For N5 interface, the PCF creates an "Individual Application Session Context" resource to store the received application session information.

8.	The PCF replies to the P-CSCF with a HTTP "201 Created" response and includes the URI of the "Individual Application Session Context" resource in the Location header field.

8a.	The PCF sends a Diameter AAA to the P-CSCF.

9.	Upon reception of the acknowledgement from the PCF, the SDP parameters are passed to the UE in SIP signalling.

10.	The PCF executes interactions according to figure 5.2.2.2-1 in TS 29.513. This step implies provisioning of PCC rules and is executed in parallel with steps 8 and 9 (steps 8a and 9a for Rx case).

Upon reception of the PCC rules the SMF:

b1)	 (based on the PCF request to insert a local N6 breakout point) allocates for the PDU Session (a) (an) I-UPF(s) on the satellite currently serving the UE and configures the UPF(s) to act as UL CL (to break out only U-plane traffic corresponding to the SDF indicated by the PCF) and/or as L-PSA (local N6 breakout point); the SMF creates a PDR/FAR referring to the network of ISL(inter satellite links) and the I-UPF answers back with addressing information corresponding to the local N6 breakout point (termination point at the L-PSA of the N6 tunnel over the ISL network of the terminating side)

b2) requests QoS resources and ANI (Access Network Information) from the RAN

b3) reports ANI to the PCF that contains both the information received from the RAN and addressing information corresponding to the local N6 breakout point (termination point at the L-PSA of the local outer tunnel N6 tunnel over the ISL network).

11.	If the P-CSCF requested ANI and/or EPS fallback indication in step 6, the PCF invokes the Npcf_PolicyAuthorization_Notify service operation to forward the EPS fallback indication, if received in step 10, and/or the ANI received in step 10 in an HTTP POST request sent to the Notification URI received in step 6.

11a.	If the P-CSCF requested ANI and/or EPS fallback indication in step 6a, the PCF forwards the EPS fallback indication, if received in step 10, and the ANI received in step 10 in a Diameter RAR.

12.	If step 11 occurs, the P-CSCF acknowledges the receipt of the notification request with an [HTTP "204 No Content"] Npcf_PolicyAuthorization_Notify response to the PCF.

12a.	If step 11a occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

13.	If step 11 occurs (step 11a for Rx case), the P-CSCF forwards the network provided location information in a subsequent SIP message to IMS core network together a SIP header (e.g. PANI) indicating that the terminating side is served by a satellite of a given constellation (Satellite constellation ID) and/or PSA I-UPF’s local outer tunnel N6 breakout point information. The P-CSCF, based on local configuration, may also include the EPS fallback indication, if received. (This step corresponds to step 14 in clause 6.43.3.2)

#### 6.43.3.4 Change of Satellite in UE-Sat-UE scenario (mobility)

Figure 6.43.3.41 Change of Satellite in UE-Sat-UE

Figure 6.43.3.42 Change of Satellite using simultaneous change of UL CL

The above call flow is based on clause 4.3.6.7 from TS 23.502.

UE has an established PDU Session with a UPF, including the PDU Session Anchor (Ground UPF). The PDU Session user plane involves at least the Source (R)AN, Source UL CL and the ground UPF (PDU Session Anchor, PSA), where Source UL CL and the terminating side are communicating over PtP tunnel.

- SMF decides to change the UL CL due to UE mobility (change of satellite).

- The SMF selects a UPF and using N4 establishes the Target UL CL for the PDU Session. SMF provides the necessary (for SIP signalling) uplink forwarding rule towards the PSA, including the tunnel info for the ground UPF. If session continuity upon UL CL relocation is used, the SMF also uses N4 to establish an N9 forwarding tunnel between the Source UL CL and Target UL CL, including the Tunnel Info for each UPF. In addition, the AN Tunnel Info to target (R)AN is provided for downlink forwarding to the target UL CL. In the case of UL CL, the SMF provides traffic filters indicating what traffic shall be forwarded towards PSA and Source UL CL, respectively. The target UL CL is configured with UL PDR/FAR to point to source UL CL and the source UL CL is configured with DL PDR/FAR to point to target UL CL. The SMF also fetches the N6 endpoint information from the target UL CL and informs P-CSCF via PCF (as described in clause 6.43.3.3 step 10-12)

NOTE 1:	When session continuity upon UL CL relocation is used, the downlink traffic at this point goes through Source UL CL, Target UL CL and Target (R)AN.

- The SMF updates the PSA via N4. It provides the PDU Session CN Tunnel Info for the downlink traffic. The Ground UPF now points to target UL-CL (for SIP signalling).

- The SMF updates (R)AN via N2 SM information over N11. It provides the new CN Tunnel Info corresponding to the Target UL CL.

NOTE 2:	When session continuity upon UL CL relocation is used, the uplink traffic destined to Source AGW-NT at this point goes through Target (R)AN, Target UL CL and Source UL CL and then to Source AGW-NT.

5.	When the P-CSCF receives the reinvite with new P-ANI header due to a change of satellite and sends the reinvite along with an additional header as described in step 13 of Figure 6.43.3.3-1 and receives the 200 OK along with terminating side N6 endpoint information. The P-CSCF shall send policy_authorization update by including the new PCC rules based on terminating side N6 information to PCF. PCF shall send the PCC rule update to SMF. SMF configures the target UL-CL with the filter rules and forwarding rules to point to new N6 of the terminating side. The target UL-CL will then continue to forward the packets belonging to source N6 terminating PtP via source UL-CL (N9), and now start forwarding the packet belonging to target N6 terminating PtP via N6.

6.  When session continuity upon UL CL relocation is used, detection of no active traffic over the N9 forwarding tunnel is performed during a time interval provisioned by SMF for User Plane inactivity report in order to release the N9 forwarding tunnel. The detection can be done by Source UL CL, which notifies the SMF of no active traffic over the N9 forwarding tunnel. The SMF releases the Source Branching Point or the Source UL CL.

NOTE 3:	It is up to network configuration whether the detection of no active traffic is performed by the Source UL CL or the Target UL CL. As an alternative to the detection of no active traffic, the P-CSCF can send an explicit notification (traffic influence) to the SMF when traffic to/from this UE ceases to exist, leading the SMF to release the Source UL CL. The notification can be sent after step 29 in the above diagram.

#### 6.43.3.4 Roaming case in UE-Sat-UE scenario

Figure 6.43.3.4-1: Voice roaming architecture using LBO

- For IMS roaming to work, the P-CSCF and S-CSCF exchange and record each other’s Uniform Resource Identifiers (URIs) during IMS registration as specified in 3GPP TS 24.229. The recorded S-CSCF URI is added as an SIP route header during the session setup by P-CSCF to route the originated sessions to the S-CSCF, and similarly, the S-CSCF adds the recorded P-CSCF URI as a SIP route header to route terminated sessions to the P-CSCF as specified in 3GPP TS 24.229.

- The rest of the procedure will remain the same as described in clause 6.43.3.2 and 6.43.3.3. The IPX network performs routing based exclusively upon the topmost SIP Route header that must contain the address of the destination network.

#### 6.43.3.5 Support for jurisdictions where LI can apply

When the satellite serves an area where LI may apply, the architecture supports an IMS-AGW on board of the satellite, as depicted in Figure 6.43.3.5-1.

This figure depicts a change of satellite at the side of the left UE where S-RAN / S-I-UPF and S-AGW-NT represent the RAN, the intermediate UPF (supporting both a UL CL and a local PSA) and the IMS AGW on the source (initial) satellite serving the UE;

The routing between AGW-NTs across satellite via ISL is assumed to be based on IP routing and the AGWs in satellite needs to have non-conflicting IP address during resource allocation. It is up to the deployment to manage IP routing across ISL links.

Figure 6.43.3.5-1: AGW resource allocation in satellite

Figure 6.43.3.5-2: P-CSCF and IMS-AGW interaction

The call flow follows clause 6.2.1, Gate Control & Local NA(P)T procedure in TS 23.334. The IMS-AGW is replaced by IMS-AGW-NT. In the above figure, the AGW-NT1 is the source AGW-NT, and AGW-NT2 is the target AGW-NT, as represented in Figure 6.43.3.5-1.

Upon receipt of a session initiation request, the IMS-ALG shall extract the offeror’s destination network address(es) and port number(s) from the signalling message body received from the calling party endpoint. It shall then request the IMS-AGW-NT1 to allocate transport resources (T2) via the Reserve AGW Connection Point procedure. Upon receipt of the response from the IMS-AGW-NT1, the IMS-ALG shall modify the offeror’s destination address(es) and/or port(s) contained in the application signalling message body and propagate the session establishment toward the terminating party.

On receipt of the terminating end SDP in the session establishment response, the IMS-ALG shall pass the information to the IMS-AGW-NT1 in the Configure AGW Connection Point procedure and shall request the IMS-AGW-NT1 to allocate transport resources (T1) via the Reserve and Configure AGW Connection Point. Upon receiving the response from the IMS-AGW-NT1, the IMS-ALG shall modify the answerer's destination address(es) and/or port(s) contained in the application signalling message body and pass the information to the originating party.

On session termination, the IMS-ALG shall request the IMS-AGW-NT1 to release its transport resources via the Release AGW Termination procedure.

- Upon receipt of a session initiation request, the P-CSCF(IMS-ALG) shall extract the offeror’s destination network address(es) and port number(s) from the signalling message body received from the calling party endpoint.

- It shall then request the IMS-AGW-NT1 to allocate transport resources (T2) via the Reserve AGW Connection Point procedure. The request optionally can include local endpoint information to identify which PtP tunnel to use for forwarding user plane traffic to the terminating end.

- AGW-NT1 reserve resources (T2)

- Once AGW-NT1 reserves resources (T2), it shall indicate the reserved resources to P-CSCF and optionally the local user plane traffic endpoint PtP tunnel information (IP and Port).

- Upon receipt of the response from the IMS-AGW-NT1, the P-CSCF(IMS-ALG) shall modify the offeror’s destination address(es) and/or port(s) contained in the application signalling message body.

- P-CSCF propagate the session establishment toward the terminating party. It shall also include an additional SIP header (e.g., a P-ANI header) to indicate its satellite access information, such as satellite ID and optional local tunnel endpoint info.

- P-CSCF receives the terminating end SDP with an additional SIP header containing information about terminating point satellite access (e.g. P-ANI header) indicating its satellite access information, such as satellite ID and optionally remote tunnel endpoint info.

- On receipt of the terminating end SDP in the session establishment response, the P-CSCF(IMS-ALG) shall pass the information to the IMS-AGW-NT1 in the Configure AGW Connection Point procedure and shall request the IMS-AGW-T to configure transport resources (T2) via the Configure AGW Connection Point. It can optionally include the terminating point satellite access information (remote tunnel endpoint info).

- AGW-NT1 shall configure the terminating side transport endpoint (T2).

- AGW-T sends ack for T2 configuration complete.

- The P-CSCF(IMS-ALG) shall pass the information (originating side) to the IMS-AGW-NT1 in the Configure AGW Connection Point procedure and shall request the IMS-AGW-NT1 to allocate transport resources (T1) via the Reserve and Configure AGW Connection Point.

- AGW-NT1 reserve resources for T1.

- Once the AGW-NT1 configures itself (T1), it acknowledges to P-CSCF.

- Based on the T1 resource reservation, P-CSCF modifies the SDP.

- P-CSCF sends modified SDP answer to the originating side UE.

NOTE: The mobility procedure in 5GS is explained in figure 6.43.3.5-3.

- Due to change of satellite (mobility), the UE shall send re-invite with new P-ANI header indicating new satellite information. The P-CSCF shall determine the new AGW based on satellite information received in P-ANI header.

- P-CSCF shall then request the IMS-AGW-NT2 to allocate transport resources (T2) via the Reserve AGW Connection Point procedure. The request optionally can include local endpoint information to identify which PtP tunnel to use for forwarding user plane traffic to terminating end.

- AGW-NT2 reserve resources (T2).

- Once AGW-NT2 reserves resources (T2), it shall indicate the reserved resources to P-CSCF and optionally the local user plane traffic end point PtP tunnel information (IP and Port).

- Upon receipt of the response from the IMS-AGW-NT2, the P-CSCF(IMS-ALG) shall modify the offeror’s destination address(es) and/or port(s) contained in the application signalling message body.

- P-CSCF propagate the session establishment toward the terminating party. It shall also include additional SIP header (e.g. P-ANI header) to indicate its satellite access information such as satellite ID and optionally local tunnel endpoint info.

- P-CSCF receives the terminating end SDP with additional SIP header containing information about terminating point satellite access (e.g. P-ANI header) indicating its satellite access information such as satellite ID and optionally remote tunnel endpoint info. The remote tunnel endpoint might change for different satellite.

- On receipt of the terminating end SDP in the session establishment response, the P-CSCF(IMS-ALG) shall pass the information to the IMS-AGW-NT2 in the Configure AGW Connection Point procedure and shall request the IMS-AGW-NT2 to configure transport resources (T2) via the Configure AGW Connection Point. It can optionally include the terminating point satellite access information (remote tunnel endpoint info).

- AGW-NT2 shall configure the terminating side transport endpoint (T2).

- AGW-NT2 sends ack for T2 configuration complete.

- The P-CSCF(IMS-ALG) shall pass the information (originating side) to the IMS-AGW-NT2 in the Configure AGW Connection Point procedure and shall request the IMS-AGW-NT2 to allocate transport resources (T1) via the Reserve and Configure AGW Connection Point.

- AGW-NT2 reserve resources for T1.

- Once the AGW-NT2 configures itself (T1), it acknowledges to P-CSCF.

- Based on the T1 resource reservation, P-CSCF modifies the SDP and sends a modified SDP answer in 200 OK to the originating side UE.

- The P-CSCF then releases the resources from the (source) IMS-AGW-NT1.

Editor’s Note: SA3-LI WG is input is needed to validate, if the relocation of AGW has any impact on the LI architecture.

Figure 6.43.3.5-3: Simultaneous change of UL CL for session continuity

The above call flow is based on clause 4.3.6.7 from TS 23.502.

UE has an established PDU Session with a UPF, including the PDU Session Anchor (Ground UPF). The PDU Session user plane involves at least the Source (R)AN, Source UL CL and the ground UPF (PDU Session Anchor, PSA), where Source UL CL and AGW-NT may be directly connected (no routing necessary), but routing can be supported.

The below steps, except for steps 5 onwards, are executed between steps 15 and 16 in the above diagram. The step 5 onwards are executed after step 28 in the above diagram.

1.	SMF decides to change the UL CL due to UE mobility (change of satellite).

2.	The SMF selects a UPF and, using N4 establishes the Target UL CL for the PDU Session. SMF provides the necessary (for SIP signalling) uplink forwarding rule towards the PSA, including the tunnel info for the ground UPF. If session continuity upon UL CL relocation is used, the SMF also uses N4 to establish an N9 forwarding tunnel between the Source UL CL and Target UL CL, including the Tunnel Info for each UPF. In addition, the AN Tunnel Info to target (R)AN is provided for downlink forwarding to the target UL CL. In the case of UL CL, the SMF provides traffic filters indicating what traffic shall be forwarded towards PSA and Source UL CL, respectively. The target UL CL is configured with UL PDR/FAR to point to source UL CL and the source UL CL is configured with DL PDR/FAR to point to target UL CL.

NOTE 1:	When session continuity upon UL CL relocation is used, the downlink traffic at this point goes through Source UL CL, Target UL CL and Target (R)AN.

3.	The SMF updates the PSA via N4. It provides the PDU Session CN Tunnel Info for the downlink traffic. The Ground UPF now points to target UL-CL (for SIP signalling).

4.	The SMF updates (R)AN via N2 SM information over N11. It provides the new CN Tunnel Info corresponding to the Target UL CL.

NOTE 2:	When session continuity upon UL CL relocation is used, the uplink traffic destined to Source AGW-NT at this point goes through Target (R)AN, Target UL CL and Source UL CL and then to Source AGW-NT.

5.	When the P-CSCF receives the resource allocation information in step 28 in Figure 6.43.3.5-2, the P-CSCF shall send a policy_authorization update by including the new PCC rules based on target AGW resource allocation to PCF. PCF shall send the PCC rule update to SMF. SMF configures the target UL-CL with the filter rules and forwarding rules to point to target AGW-NT. The target UL-CL will then continue to forward the packet belonging to source AGW-NT via source UL-CL (N9) and now start forwarding the packet belonging to target AGW via N6.

6. When session continuity upon UL CL relocation is used, detection of no active traffic over the N9 forwarding tunnel is performed during a time interval provisioned by SMF for User Plane inactivity report in order to release the N9 forwarding tunnel. The detection can be done by Source UL CL, which notifies the SMF of no active traffic over the N9 forwarding tunnel. The SMF releases the Source Branching Point or the Source UL CL.

NOTE 3:	It is up to network configuration whether the detection of no active traffic is performed by the Source UL CL or the Target UL CL. As an alternative to the detection of no active traffic.

### 6.43.4	Impacts on services, entities and interfaces

UE:

- The UE needs to invoke reinvite based its own implementation of detecting change of serving satellite. The UE will use the existing P-ANI header information.

IMS

-	IBCF: Allows PANI information be sent to the other party of the call but with only (outer IP) addressing information

-	P-CSCF

-	Upon UE registration subscribe onto the PCF for the UE connectivity via satellite access capable of UE-Sat-UE.

-	May support generating/receiving a PANI related to satellite access information.

-	Based on PANI associated with the remote party of a call and on the information received from the PCF for the local UE detect that both parties of a call are served by the same satellite constellation.

-	When invoking PCF for Authorization of QoS resources and fetching the user location (ANI), request to insert a local N6 breakout point for the corresponding QoS resources and to report addressing information for this local N6 breakout point.

-	Send addressing information for the local N6 breakout point (received from PCF) to the other end of the call via SIP signalling.

-	The identification and insertion of AGW onboard satellite, where LI is applicable.

PCF

- Support subscription for the UE connectivity via satellite constellation

- Support a request to insert a local N6 breakout point for the corresponding QoS resources and to report (outer IP) addressing information for this local N6 breakout point and transform this request into relevant PCC rules. Furthermore, notifies the consumer (P-CSCF) when receiving from the SMF (outer IP) addressing information for the local N6 breakout point.

SMF

- based on the PCF request (PCC rule) to insert a local N6 breakout point,

  - allocates for the Session (a) I-UPF(s) on the satellite currently serving the UE.

  - configures the UPF(s) to act as UL CL (to break out only traffic corresponding to the SDP indicated by the PCF) and/or as L-PSA (local N6 breakout point);

  - retrieve from the L-PSA UPF the local (outer IP) addressing information corresponding to the local N6 breakout point.

  - Provides the remote (outer IP) addressing information to the L-PSA UPF.

- reports to the PCF ANI that contains both the location information received from the RAN and (outer IP) addressing information corresponding to the local N6 breakout point (and retrieved from the L-PSA).

UPF

- Provides to SMF the local (outer IP) addressing information corresponding to the network instance in a PDR.

- Detects based on the remote (outer IP) addressing information that some traffic is to be locally switched at the UPF (based on FAR/Forwarding Rules/ Outer Header creation containing the L-PSA own outer IP address)

## 6.44	Solution #44: eNB buffered signalling and CP data transfer with eNB monitoring list

### 6.44.1	Description

This solution resolves Key Issue #2.

Taking into account the intermittent availability of the service link and ground link the solution enhances Store and Forward operation while improving the UE power saving.

The enhanced attach procedure is used as example to illustrate the solution concept.

-	The attach procedure is modified including when there is a service link between the eNodeB on board the satellite and the UE, the eNodeB generates a next accessing time based on ephemeris and sends it to the UE. The UE uses this value to know when to access an eNodeB in the future to resume the attach procedure and repeats the process until the procedure is completed.

-	The data transport of MO and MT data is modified by introducing a mechanism to activate the S&F mode in EPC, in the RAN or both. The use of S&F mode results in a more efficient data transmission: first the most suitable eNodeB is selected to deliver data to the UE; second, the UE is instructed to stop listening if no more data needs to be transferred.

In addition to the S&F buffering/delivery, it additionally provides the following aspects which can be applied to any S&F solution (i.e. they are not dependent on the architecture in this solutions):

-	To further enhance power saving, the UE can request to listen to a single or multiple satellites by taking into account UE power saving requirements and delay sensitivity of the data to be transferred.

-	When a UE is released from connected the UE can be informed that the eNodeB that released the UE will not page the UE, enabling the UE to save power by not monitoring paging for the remaining time the UE camps on the eNodeB’s cell.

-	Since multiple UEs may share the same satellite or satellite constellation, the storage resources of the satellites may be insufficient. Thus, UE level storage space reservation should be supported to enable satellite selection of serving satellites taking storage resources into consideration.

NOTE 1:	For power saving and storage resources discussed above is general and decoupled with architecture options.

NOTE 2: 	The solution can also be applied to 5GS, with replacing the EPS nodes with the appropriate 5GS nodes (e.g. MME with AMF, eNodeB with gNB etc).

NOTE 3:	The solution assumes the use of control plane data transfer and therefore AS security is not required.

Editor’s Note: 	Whether the solution needs to support user plane data transfer is FFS.

Editor’s Note:	The handling of TAIs for the eNodeB onboard requires clarification.

The HSS can contain the S&F data quota which is provided to the MME.

### 6.44.2	Architecture

EPC

UE

eNB

MME

Feeder link

Service link

Figure 6.44.2-1: eNodeB on board architecture with S&F operation

### 6.44.2	Procedures

#### 6.44.2.1	Attach Procedure

UE

MME

EPC nodes

SAT eNodeB 1

SAT eNodeB 2

1. RRC message (attach request)

Service link available

2. eNodeB determines target eNodeB

4a. Store UE context

3. RRC connection released and next access timer indication

4b. Store UE NAS Messages, UE context and UE location

6. Processing of uplink NAS messages (authentication request)

Feeder link available to source eNodeB

5. Uplink NAS messages and target eNodeB

8. RRC connection resumed (next NAS message)

Service link available

9. Repeat steps 2-4

Feeder link available to source eNodeB

10. Repeat steps 5-6

7. DL NAS messages, UE context and UE location

Feeder link available to target eNodeB

Repeat N times until procedure is completed

11. DL NAS messages, UE context and UE location

Feeder link available to target eNodeB

The enhanced attach procedure is shown in Figure 6.44.2.1-1. The eNodeB calculates a next accessing time value that indicates when the service link will become available again. The UE uses this time to resume the attach procedure.

Figure 6.44.2.1-1: high-level procedure for S&F attach

1.	When service link is available, the UE sends RRC message including Attach Request to the eNodeB. The eNodeB may provide an indication of S&F operation by e.g. an indication in SIB.

NOTE 1:	The timeout of NAS messages should be adjusted to when the procedure uses S&F (e.g. several hours). This will be determined by CT1.

2.	The eNodeB providing the service link (source eNodeB) may select a target eNodeB which is may be used by the UE to continue the procedure and exchange subsequent NAS message(s). The selection of the target eNodeB is based on ephemeris and UE location, and other parameters. The (source) eNodeB may select itself as target eNodeB.

When there is only one eNodeB in the satellite constellation can provide the service link towards the UE, that eNodeB (source eNodeB) always selects itself as the target eNodeB.

After selecting the target eNodeB, the (source) eNodeB determines the next access timer for the UE, which indicates when the target eNodeB provides service link to the UE.

Editor’s Note: How the eNodeB determines the target eNodeB is FFS.

3.	The eNodeB provides the next access timer to the UE. Until the next access timer expires, the UE can stop monitoring broadcast and paging messages to enable power saving.

4a.	The eNodeB and UE stores UE context, including at least a UE ID identifying the UE performing the attach procedure, such as the combination of C-RNTI and global eNodeB ID. The UE context is used to resume the RRC connection between the eNodeB providing the service link and the UE (see step 7).

Editor’s Note: The details of the UE context information used to resume the RRC connection in the target eNB is FFS and requires coordination with RAN.

4b.	The eNodeB stores the UE context as well as uplink NAS message (i.e. Attach Request) and the UE location information. The ULI should be generated by the eNodeB when the UE performs the RACH procedure to create or resume a connection (e.g. to send an UL NAS message such as Attach Request).

NOTE 2:	Details of the UE context used for resuming a connection will be determined by RAN.

5.	When a feeder link is available, the eNodeB sends the uplink NAS message (i.e. Attach Request) and related information as described in clause 5.3.2.1 of TS 23.401 [5] to the MME including UE location information, the UE context collected in Step 4b as well as the identity of the target eNodeB if included in Step 2.

6.	The MME interacts with other EPC elements (e.g. SGW, HSS) to progress the procedure as described in clause 5.3.2 of TS 23.401 [5]. As an example, the MME interacts with the HSS to perform authentication.

7.	When the feeder link to the target eNodeB is available, the MME sends the downlink NAS message to the target eNodeB. The MME should also start specific NAS timers applicable for S&F, considering e.g. the delays for S&F operation.

For multi-satellite case, the S1AP message also includes UE context and UE location information from the MME. The UE location information can be used by the (target) eNodeB to page the UE in step 8.

8.	When the service link is available (i.e. the next access timer stored in the UE expires and the target eNodeB is available to the UE), the RRC connection between the UE and target eNodeB is resumed by paging the UE trigging the RRC connection to be resumed based on the UE ID and C-RNTI stored in UE context and the next NAS uplink message is sent to the eNodeB.

9.	Steps 2~4 are repeated, for the next uplink NAS message in the NAS procedure.

10.	When the feeder link becomes available for the source eNodeB carrying the uplink NAS messages, step 5 and step 6 are repeated.

11.	When the feeder link becomes available to the target eNodeB, step 7 is repeated for the next downlink NAS message is the procedure.

Steps 8~11 are repeated as many times as required by the procedure.

As described in clause 6.44.2.4, the MME constructs and sends a S&F Monitoring List to the UE if the Attach is accepted. After the UE receives the S&F Monitoring List, the UE starts to listen to paging only from an eNodeB included in the S&F Monitoring List.

Editor's note:	How the handle the UE falling back from S&F to none S&F operation is FFS.

Editor’s note: 	How to handle the case of the UE attaching to a TN while performing S&F attach procedure is FFS.

#### 6.44.2.2	MT control plane data transfer in S&F operation mode

The call flow below illustrates how MT data is transmitted to UE in S&F mode.

SAT eNodeB

UE

MME

2. eNodeB selection, data encryption and integrity protection

Feeder link available

SGW

1. Downlink Data

P-GW

3. NAS PDU

Service link available

4. RRC connection establishment and NAS PDU delivery

5. RRC Release and Sleep Indicator

6. Stop listening to paging during the remaining time the satellite is providing coverage

7. NAS delivery notification

Feeder link available

Figure 6.44.2.2-1: High-level procedure for MT data transport in S&F mode

Precondition: the SAT eNodeB is in the S&F Monitoring List and has obtained the UE related context or needs to obtain it in step 3 from MME, e.g., AS security (if is use for a UE) and S&F data quota reservation.

1.	When the S-GW receives downlink data, it forwards the data to MME, as described in steps 1, step 2 and step 7 to step 11 of TS 23.401 [5] clause 5.3.4B.3, assuming the RAT Type is unchanged from last access.

2.	The MME determines that the UE is working in S&F mode (e.g. based on subscription data and how it attached), and it selects the most suitable eNodeB on board to deliver the downlink NAS PDU. The eNodeB selection is based on the S&F Monitoring List, e.g., the first satellite in the S&F Monitoring List which can reach the UE in shortest amount of time.

The MME prepares the downlink NAS PDU to send to the UE, e.g. encrypts and integrity protects downlink data, performs IP header compression, as per step 12 of TS 23.401 [5] clause 5.3.4B.3.

3.	The MME sends the downlink NAS PDU to the selected eNodeB.

4.	When the service link between the selected eNodeB and the UE becomes available, and the RRC connection is established, the eNodeB delivers the downlink NAS PDU to the UE.

5.	If the eNodeB determines all stored downlink for the UE has been delivered, it may release the RRC Connection and send the UE a Sleep indicator, which informs the UE that it will not be paged by the eNodeB during the remaining time while the current eNodeB provides coverage, enabling additional UE power saving.

6.	The UE may stop listening to paging during the remaining time when the current eNodeB provides coverage.

7.	When the feeder link becomes available, the eNodeB sends a NAS Delivery indication to the MME if requested, as described in step 15 of TS 23.401 [5] clause 5.3.4B.3.

#### 6.44.2.3	MO control plane data transfer in S&F operation mode

SAT eNodeB

UE

MME

4. Check integrity and decrypt data

Feeder link available

SGW

P-GW

3. NAS PDU

Service link available

1. RRC connection establishment or RRC early data request (NAS PDU)

2. RRC connection release and sleep indicator

5. Uplink Data

6. Downlink NAS/data delivery

The call flow below illustrates how MO data is transmitted from the UE in S&F mode.

Figure 6.44.2.3-1: High-level procedure for MO data transport in S&F mode

Precondition: the SAT eNodeB is in the S&F Monitoring List and has obtained the UE related context or needs to obtain it in step 3 from MME, e.g., AS security (if is use for a UE) and S&F data quota reservation.

1.	Similar to step 1 of TS 23.401 [5] clause 5.3.4B.2, the UE establishes an RRC connection with an on-board eNodeB which is present in the S&F Monitoring List, and sends an uplink NAS PDU containing the Uplink Data.

2.	As the feeder link is disconnected, the eNodeB stores the uplink NAS PDU. The eNodeB may release the RRC Connection and may send the UE a Sleep indicator, which informs the UE that it will not be paged by the eNodeB during the remaining time while this eNodeB provides coverage.

3.	When the feeder link becomes available, the NAS PDU sent in step 1 is sent to the MME.

4.	Following step 3 of TS 23.401 [5] clause 5.3.4B.2, the uplink NAS PDU is decrypted, verified and any header decompression applied as needed.

5.	As per step 4 to step 8 of TS 23.401 [5] clause 5.3.4B.2 the uplink data is sent to the P-GW via the S-GW.

6.	If there is any downlink data to be delivered to the UE, then the procedure in clause 6.44.2.2 can be performed.

#### 6.44.2.4	Negotiation of S&F Monitoring List

The call flow below illustrates the S&F Monitoring List negotiation procedure in S&F operation mode.

UE

eNodeB

MME

eNBs in the monitoring list

1. Attach/TAU Request

(Requested S&F Monitoring

Mode)

2. Attach/TAU Request

(Requested S&F Monitoring

Mode)

3. Attach/TAU Accept

(S&F Monitoring List)

4. Attach/TAU Accept

(S&F Monitoring List)

3a. [UE context synchronization], S&F data quota

5b. Listen to paging under coverage of particular eNodeB/SAT(s)

5a. Select eNodeB/SAT(s) included in S&F Monitoring List

Figure 6.44.2.4 -1: High-level procedure for S&F Monitoring List negotiation

Precondition: When the S1-AP connection between eNodeB and MME is established, the eNodeB sends its remaining storage space size to the MME.

1.	The UE sends Attach/TAU Request including the Requested S&F Monitoring Mode (e.g., single satellite, a subset of satellites, all satellite) to the eNodeB on-board, when the service link is available, using the mechanism described in clause 6.44.2.1. The UE is aware that the procedure may be completed over more than one RRC connection, and considers that accordingly (e.g. adjusts timers etc...).

The UE may determine its preferred S&F Monitoring Mode considering both power saving and data delivery delay requirements.

2.	The eNodeB sends Attach/TAU Request message to the MME, when the feeder link is available, as per clause 6.44.2.1 or clause 6.44.2.3 using NAS signalling.

3.	The MME obtains the UE subscription data including S&F data quota from the HSS, if not obtained yet. The MME determines UE’s S&F Monitoring List which indicates the eNodeB(s) the UE should listen to (e.g., based on UE context, UE preference, subscription data including S&F data quota, latest reported remaining storage space size of the eNodeB(s), and recommended satellites list from a new network element (e.g., URE in solution#25) which is responsible for estimating which satellites can reach the UE in short amount of time possible, etc.) and sends it to the eNodeB in the Attach/TAU Accept message as per clause 6.44.2.1 or step 6 of clause 6.44.2.3. The UE listens to paging only from an eNodeB included in the S&F Monitoring List.

3a. As long as the S&F Monitoring List is determined, when satellites in the S&F Monitoring List established S1-AP connection with MME, the MME should synchronize UE context with UE S&F data quota to them.

4.	The eNodeB sends the Attach/ TAU Accept message including the S&F Monitoring List for the UE, when the service link is available, as per clause 6.44.2.1.

5.	The MME selects a suitable eNodeB included in the S&F Monitoring List when sending DL message to the UE. The UE starts to listen to paging only when an eNodeB included in the S&F Monitoring List provides service link.

### 6.44.3	Impacts on services, entities and interfaces

MME:

-	Determines UE’s S&F Monitoring List for power saving and S&F data quota reservation during Attach/TAU procedure.

-	Activates S&F mode base on UE’s subscription.

-	Sends UE context and NAS PDU Data to RAN before UE is reachable.

RAN:

-	Determines target RAN based on ephemeris and UE location.

-	Generates a next access timer based on ephemeris and target RAN.

-	Stores UE context for RRC connection resume during initial attach procedure.

-	Resumes the RRC connection with the UE when the UE is in coverage again with downlink NAS message.

-	Sends UE context and, target RAN ID and UE location information to MME when feeder link is available.

-	Stores the UL and DL NAS PDU Data, until relevant service/feeder link is available.

-	Performs Paging when reaching UE’s location area.

-	Sends Sleep Indicator to UE.

UE:

-	Stores information for RRC connection resume during initial attach procedure.

-	After sending attach request, it can suspend and resume RRC connection to complete the exchange of NAS messages.

-	Can request its preferred S&F Monitoring Mode during Attach/TAU procedure.

-	If S&F Monitoring mode for power saving is agreed with the network, only listens to paging of specific RAN/SAT(s) providing the service link.

-	May stop listening to paging when service link is still available.

## 6.45	Solution#45: Support SMS in S&F satellite operation

### 6.45.1	Introduction

This solution corresponds to KI#2 to enable SMS services with store and forward satellite operation.

### 6.45.2	Description

In this solution, the SMS delivery over satellite which support store and forward feature is supported based on the enhanced architecture of SMS in MME described in Annex C of TS 23.272 [18], and based on the enhanced procedures of short message transfer described in Figure 17 a) of TS 23.040 [19]. The enhanced architecture is described in Figure 6.45.2-1, and the enhanced procedure is described in Figure 6.45.3.1-1.

Figure 6.45.2-1: Architecture enhancement for SMS support in S&F satellite operation

The UE attaches to the EPS network that the E-UTRAN, MME, and the SMS-GMSC (SAT) are on-boarded to the store and forward feature supporting satellite. In the ground, the SMS-GMSC (Ground), SC, HSS are located.

It is assumed that the MME is aware of the service link connectivity and the feeder link connectivity, based on the Unavailability Period of the satellite that the MME is on-boarded. Based on the connectivity of the service link and the feeder link, the MME provides the time information to the SMS-GMSC (SAT) when to retransmit the SMS message, and when to transfer the Delivery Report of the SMS message. Subsequently, the SMS-GMSC (SAT) stores and transfers the SMS message and the Delivery Report until the time indicated by the MME is reached.

By the above mechanism, the MME can keep its responsibility of satellite link connectivity determination and retransmission time indication, and the SMS-GMSC can keep its responsibility of storing the Short Message with minimum impacts to existing mechanism for SMS.

NOTE:	SMS-GMSC (SAT) and SMS-GMSC (Ground) can exchange signalling based on the implementations, and the detail is out of scope of 3GPP.

### 6.45.3	Procedures

Editor’s Note: MO SMS Delivery procedure and support for multiple satellites are FFS.

#### 6.45.3.1	MT SMS Delivery via SMS-GMSC on satellite

Based on Figure 17a) of TS 23.040 [19], the MT SMS message is delivered as described in Figure 6.45.3.1-1 with following differences:

- The SMS-GMSC is located both in the ground and in the satellite.

- The MS SMS message and the Delivery Report of MT SMS are stored in SMS-GMSC until the service link or the feeder link becomes available.

Figure 6.45.3.1-1: Procedure for MT SMS Delivery via SMS-GMSC on satellite

The UE attaches to EPS via Regenerative satellite access, which supports store and forward operation. E-UTRAN, MME, and SMS-GSMC (SAT) are on the same satellite, and SMS-GMSC (Ground), SC, and HSS are on the ground, as described in Figure 6.45.2-1.

- The SC initiates transfer of MT SMS. The SMS-GMSC (Ground) retrieves the address of MME or SMS-GMSC (SAT) from the HSS.

- The SMS-GMSC (Ground) forwards the MT SMS message to SMS-GMSC (SAT).

- The SMS-GMSC (SAT) forwards the MT SMS message to the MME, and indicates it is capable to retransmit the Short Message until a maximum retransmission time.

- The MME checks whether the service link is connected.

Case A: Service link is not available

- If the service link between the UE and the satellite is not connected as determined in step 4, then the MME sends a Delivery Report with a cause indicating that it is temporarily unreachable and with an indication requesting the SMS-GMSC (SAT) to retransmit the Short Message at a later requested retransmission time prior to the maximum retransmission time. The MME determines the requested retransmission time based on the Unavailability Period of the satellite that MME is on-boarded.

- The SMS-GMSC (SAT) stores the MT SMS message until the requested retransmission time received from the MME.

Case B: Service link is available

- If the service link is connected as determined in step 4 and/or the requested retransmission time received in step 5 is reached, the SMS-GMSC (SAT) retransmits the MT SMS message to the MME.

- The MME pages the UE. If the UE is in the ECM-IDLE state, the UE triggers Service Request procedure upon reception of paging message from MME.

- The MT SMS message is delivered to the UE.

- The MT SMS message is transferred to the UE, and the Delivery Report of MT SMS is returned to the MME.

- The MME checks whether the feeder link is connected.

Case C: Feeder link is not available.

- If the feeder link between the MME and the SMS-GMSC (SAT) is not connected as determined in step 11, then the MME transfers the Delivery Report of MT SMS to the SMS-GMSC (SAT) with an information about the available time for feeder link.

- The SMS-GMSC (SAT) stores the Delivery Report of MT SMS until the available time for feeder link indicated by the MME.

Case D: Feeder link is available.

- If the feeder link is connected as determined in step 11 and the indicated available time for feeder link is reached, the SMS-GMSC (SAT) returns the Delivery Report of MT SMS message to the SMS-GMSC (Ground), and it is transferred to the SC.

### 6.45.3	Impacts on Existing Nodes and Functionality

The following impacts are foreseen by this solution:

MME:

-	The MME checks the service link connectivity, and determines and provides the requested retransmission time to SMS-GMSC (SAT).

-	The MME checks the feeder link connectivity, and determines and provides the available time for feeder link to SMS-GMSC (SAT).

SMS-GMSC:

-	Upon reception of the available time for feeder link from MME, the SMS-GMSC (SAT) stores and forwards the Delivery Report of MT SMS message.

HSS:

-	The HSS provides to SMS-GMSC (Ground) the address of the satellite on-boarded MME or the SMS-GMSC (SAT).

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.1	Solution grouping for Key Issue #1: Support of Regenerative-based satellite access

There are 14 candidate solutions (solutions #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #34, #35, #36, #42) for this KI, they can be divided into two categories based on the architecture impact:

-	Cat 1: solutions without any IWK/AMF agent/Proxy over NG/S1 interface (#1, #2, #3, #4, #5, #6, #7, #8, #34, #35, and #36)

-	Cat 2: solutions with an IWK/AMF agent/Proxy over NG/S1 interface (#9, #10, and #42)

For Cat1 solutions, they can be further classified into six categories based on the sub-issues being addressed:

-	Solutions for N2/S1 connections management considering RAN node mobility, the solutions can be grouped as followed:

-	N2/S1 connection setup/disconnect: #1

-	N2/S1 connection suspend/resume: #2, #3, #4

-	Solutions for enhanced TA/Cell management considering the change of supported TA list for a RAN node on-board, the solutions can be grouped as followed:

-	TA "timetable“ is calculated per RAN node by AMF/MME based on satellite ephemeris: #2

-	TA "timetable“ is provided to AMF/MME per RAN node ID: #4

-	Solutions for handling feeder link switchover with/without AMF/MME change, the solutions can be grouped as followed:

-	TAU triggered due to feed link switchover: #5, #6

-	two (logical) gNBs/eNBs connecting to both old and new CN nodes simultaneously: #35

-	different TAI configuration for different AMF/MME: #35

-	Solutions for user Plane Management considering RAN node mobility, the solutions can be grouped as followed:

-	explicit indication from RAN to AMF and finally to SMF regarding feeder link change, then the UPF buffers DL data: #7

-	gNB detects TNL IP address change and notifies the SMF, so that the SMF can instruct the UPF to update the DL tunnel: #35

-	AMF notifies the SMF of the gNB leaving its service area, so that SMF can treat GTP-U error notifications correctly: #36

-	Solutions for PCC/QoS control enhancement considering Regenerative-based satellite access:

-	two new RAT type are introduced for regenerative mode NGSO satellite access: #8

-	Solutions for AMF discovery:

-	gNB queries AMF agent on the ground to find the target AMF to be connected: #34

## 7.2	Key Issue #2 Principles for evaluation

It is proposed to evaluate the support of Store & Forward Satellite operation (key issue#2) following the principles below:

- Support of single and multi-satellites.

- Support of Roaming (the definition of roaming in S&F Satellite operation shall be clarified).

- Support of MO/MT SMS service, CIoT CP/UP.

- UE location verification

- Security considerations (in coordination with SA3)

- UE power consumption

- Delay for a UE to complete the attach procedure and for a registered UE to access service

- Support of Legacy UEs (Rel-17 & Rel-18 UEs)

- Minimal impact on the existing 3GPP procedures

- Whether Proprietary NFs are needed to support S&F

- Compute and storage requirements on satellite considering limitation of satellite payload, which can be estimated based on NFs deployed on the satellite.

- Management of the transition between phases where feeder link is available or not.

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Example network scenarios for regenerative payload generic architecture (e/gNB on LEO satellite)

The following diagrams are examples of scenarios and situations that the 3GPP standards should handle in order to offer good service to UEs when an eNB/gNB is deployed on a LEO satellite.

These examples are not intended to be an exhaustive set.

Figure A-1: Basic concept of eNB/gNB on LEO satellite

Figure A-2: Handover between satellites with common earth station location

In figure A-2, the UE is basically stationary; the satellite with eNB X is going down in elevation angle, but before X becomes unusable, the satellite with eNB Y rises above the horizon and becomes usable. Again, using methods already discussed in RAN WGs during Release 17/18, the UE can be handed over (e.g. from cell X-a to cell Y-c) during the time that the UE's area is in coverage of both satellites. This handover could be either an X2 or an S1 handover, but note that NB-IoT devices do not support handover.

Figure A-3: Feeder link switch with 2 eNBs/gNBs on LEO satellite

In figure A-3, the UE is basically stationary, but as the satellite flies over, the satellite is moving out of range of one earth station (ES1) and into range of another earth station (ES2). At some time before time T, the satellite system's O&M activates a second eNB (eNB-Z2) on the satellite and the S1 interfaces from eNB-Z through ES2 are established at time T. Using methods already discussed in RAN WGs during Release 17/18, the UE can be handed over (e.g. from cell X1-a to cell Z2-b) in the period between time T and time T+k. This handover could be an S1 handover, or, if the eNBs know that they are connected to the same MME, it could be an X2 handover. Note that NB-IoT devices do not support handover.

Figure A-4: Handover between satellites using different earth station location

Figure A-4 is a combination of the preceding scenarios but needs to be supported.

Annex B:
Store and Forward Satellite operation

The Store and Forward Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

An example of "S&F Satellite operation" is illustrated in Figure B-1, in contrast to what could be considered the current assumption for the "normal/default Satellite operation" of a 5G system with satellite access.

As shown in Figure B-1:

-	Under "normal/default Satellite operation" mode, signalling and data traffic exchange between a UE with satellite access and the remote ground network requires the service and feeder links to be active simultaneously, so that, at the time that the UE interacts over the service link with the satellite, there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network.

-	In contrast, under "S&F Satellite operation" mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (step A and B in Figure B-1). In step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B.

| "Normal/default Satellite operation" mode |  |
| --- | --- |
| "S&F Satellite operation" mode |  |

Figure B-1: Illustration of "normal/default operation" and "S&F Satellite operation" modes in a 5G system with satellite access

The concept of "S&F" service is widely used in the fields of delay-tolerant networking and disruption-tolerant networking. In 3GPP context, a service that could be assimilated to an S&F service is SMS, for which there is no need to have an end-to-end connectivity between the end-points (e.g. an end-point can be a UE and the other an application server) but only between the end-points and the SMSC which acts as an intermediate node in charge of storing and relying.

The support of S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time IoT satellite services with NGSO satellites.

Annex C:
Example of network scenarios for UE-Sat-UE generic architecture (regenerative payload capabilities on LEO satellite)

The following diagrams are examples of scenarios and situations that the 3GPP standards should handle to deliver good service for UEs - satellites - UEs over LEO constellations.

These examples are not intended to be an exhaustive set.

For sake of clarity of figures:

-	Only 2 UEs are represented to be in communication, but session of communication may involve more than 2 UEs. Note also in case satellite(s) serves more than one cell, the UEs may be in different cells.

-	Inter Satellite link is represented as a transport bearer between 2 satellites but can involve more than 2 satellites.

-	AMF is the only represented NF for 5GC but other NFs might be implied (e.g.: SMF).

Figure C-1: Basic concept of UEs- SAT- UEs communications on LEO satellite in same cell without ISL

In the case where satellites are connected with Inter Satellite Links, ISL may ensure that ground connectivity is always available.

Figure C-2: Basic concept of UEs- SAT- UEs communications on LEO satellite in same cell with ISL

In the case local switching capabilities are spread over more than one satellite, UE-Sat-UE communication can be extended to the coverage of the more than one satellites, and ISL may ensure that ground connectivity is always available.

Figure C-3: Basic concept of UEs- SAT- UEs communications on LEO satellite in cells from different satellites connected with ISL

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | SA2#159 | S2-2311913 | - | - | - | Skeleton TR 23.700-29 | 0.0.0 |
| 2023-10 | SA2#159 | S2-2311555 | - | - | - | Example network and satellite configurations for WT1 in FS_5GSAT_ARCH_Ph3 | 0.1.0 |
| 2023-10 | SA2#159 | S2-2311602 | - | - | - | FS_5GSAT_ARCH_Ph3 TR Definitions | 0.1.0 |
| 2023-10 | SA2#159 | S2-2311603 | - | - | - | FS_5GSAT_ARCH_Ph3 TR Scope | 0.1.0 |
| 2023-10 | SA2#159 | S2-2311604 | - | - | - | FS_5GSAT_ARCH_Ph3 Architecture Assumptions | 0.1.0 |
| 2023-11 | SA2#160 | S2-2313369 | - | - | - | Update of Terms and abbreviations | 0.2.0 |
| 2023-11 | SA2#160 | S2-2313623 | - | - | - | KI for WT#1 | 0.2.0 |
| 2023-11 | SA2#160 | S2-2313702 | - | - | - | KI for WT#2 | 0.2.0 |
| 2023-11 | SA2#160 | S2-2313703 | - | - | - | Architecture assumption for UE-satellite-UE communication | 0.2.0 |
| 2023-11 | SA2#160 | S2-2313704 | - | - | - | KI for WT#3 | 0.2.0 |
| 2023-11 | SA2#160 | S2-2313861 | - | - | - | Example network and satellite configurations for WT#3 in FS_5GSAT_ARCH_Ph3 | 0.2.0 |
| 2024-01 | SA2#160AHE |  | - | - | - | Implementation of approved pCRs (S2-2401776, S2-2401777, S2-2401778, S2-2401779, S2-2401780, S2-2401781, S2-2401782, S2-2401783, S2-2401784, S2-2401785, S2-2401786, S2-2401787, S2-2401788, S2-2401789, S2-2401790, S2-2401791, S2-2401792, S2-2401793, S2-2401794, S2-2401795, S2-2401796, S2-2401797, S2-2401798, S2-2401799, S2-2401800, S2-2401801, S2-2401802, S2-2401803, S2-2401804, S2-2401805, S2-2401806, S2-2401807, S2-2401837,) | 0.3.0 |
| 2024-03 | SA2#161 |  |  |  |  | Implementation of approved pCRs (S2-2403129, S2-2403134, S2-2403296, S2-2403573, S2-2403574, S2-2403575, S2-2403576, S2-2403578, S2-2403579, S2-2403581, S2-2403589) | 0.4.0 |
| 2024-04 | SA2#162 |  |  |  |  | Implementation of approved pCRs (S2-2405623, S2-2405324, S2-2405325, S2-2405010, S2-2405011, S2-2405012, S2-2405013, S2-2405014, S2-2405015, S2-2405016, S2-2405017, S2-2405018, S2-2405620, S2-2405621) | 0.5.0 |