| 3GPP TR 23.700-37 V0.3.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on discreet monitoring of mission critical services, Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Scenarios	8

4.1	General	8

4.2	Scenario 1: Discreet monitoring of MCPTT and MCVideo private calls	8

4.3	Scenario 2: Discreet monitoring of MCPTT and MCVideo group calls	9

4.3.1	Discreet monitoring of an MC service user in a group call	9

4.3.2	Discreet monitoring of an MC group	10

4.4	Scenario 3:  Discreet monitoring of one to one MCVideo pull	11

4.5	Scenario 4: Discreet monitoring of one to one MCVideo push	11

4.6	Scenario 5: Discreet monitoring of remotely initiated MCVideo push	12

4.7	Scenario 6: Discreet monitoring of MCData SDS (using media plane)	13

4.8	Scenario 7: Discreet monitoring of one to one file distribution (using media plane)	13

4.9	Scenario 8: Discreet monitoring of one from server MCVideo pull	14

4.10	Scenario 9: Discreet monitoring of one to server MCVideo push	15

4.11	Scenario 10: Discreet monitoring of MCData SDS (using signalling control plane)	15

4.12	Scenario 11: Discreet monitoring of file upload and file download	16

4.13	Scenario 12: Discreet monitoring of one to one file distribution (using HTTP)	17

4.14	Scenario 13: Discreet monitoring of migrated users	17

4.15	Scenario 14: Discreet monitoring with regulatory constraints on private communication transmissions	18

4.16	Scenario 15: Discreet monitoring with regulatory constraints on group communication transmissions	19

4.x	Scenario x: <Write the title here>	20

5	Key Issues	20

5.1	Key issue 1: Functional architecture for Discreet monitoring	20

5.2	Key issue 2: Discreet monitoring for migrated MC service user	20

5.x	Key issue x: <Write KI title here>	21

6	Solutions	21

6.1 Solution 1 (for KI#1): Functional architecture	21

6.1.1	Description	21

6.1.2	Impacts on existing functional entities and reference points	24

6.1.3	Solution evaluation	24

6.x	Solution x: <Write solution title here>	24

6.x.1	Description	24

6.x.2	Impacts on existing functional entities and reference points	24

6.x.3	Solution evaluation	24

7	Overall evaluation	24

8	Conclusions	25

A.1	Requirements for discreet monitoring	26

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies solutions to satisfy the requirements for discreet monitoring of mission critical services. It identifies enhancements to be included in the technical specifications for MC Common functional architecture (3GPP TS 23.280 [2]), MCPTT (3GPP TS 23.379 [3]), MCVideo (3GPP TS 23.281 [4]), MCData (3GPP TS 23.282 [5]), MC Interworking with LMR (3GPP TS 23.283 [6]), and Mission Critical services over 5G System (3GPP TS 23.289 [7]). The requirements for this study are taken from the Stage 1 requirements in 3GPP TS 22.280 [8].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.283: " Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[8]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.280 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] or in 3GPP TS 23.280 [2].

Discreet Listening: Access, by an authorized user, to all target user’s voice media in realtime without noticeable impact on or knowledge of any unauthorized user in the communication.

Discreet Monitoring: Access, by an authorized user, to all target user’s media (voice, video, data) in realtime without noticeable impact on or knowledge of any unauthorized user in the communication.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Scenarios

< Scenarios should be short. Sometimes even just a title is enough. As a starting point, all scenarios in TR 23.784 are still valid, but DO NOT copy paste the text/figures into this TR. Scenarios in TR 23.784 have way too much information (which belongs to KIs and even solutions) and are written based on one specific functional model for discreet monitoring. Leave details to the KIs and solutions. >

## 4.1	General

The following clauses describe scenarios that will arise from a demand from an authorized MC user for discreet monitoring of the on-network communications of a target MC user within the authority of the authorized MC user. Use cases for MCPTT, MCVideo and MCData are captured.

In each case, the authorized MC user will be presented with media and metadata which has originated from or is destined to the target MC user of the discreet monitoring request.

## 4.2	Scenario 1:	Discreet monitoring of MCPTT and MCVideo private calls

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving target MC user B where MC user A and MC user B are served by the same MC system, MC system A and target MC service user B is within the authority of authorized MC user A. The scenario is illustrated in figure 4.2-1 below.

Figure 4.2-1: Discreet monitoring of MCPTT or MCVideo private call in single MC system

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B places or receives an MCPTT or MCVideo private call to or from MC user C.

-	MC system A provides MC service client A with information concerning the initiation of the call, including the identity of MC user C.

-	MC system A provides the media exchanged between the target MC user B and MC user C in the private call.

-	When the private call completes, the call completion is conveyed to MC service client A.

NOTE 1:	The scenario also applies to emergency private calls.

NOTE 2:	The scenario also applies to first-to-answer call.

NOTE 3:	The scenario also applies to ambient listening call.

NOTE 4:	The scenario also applies to forwarded private calls.

NOTE 5:	The scenario also applies to transferred private calls.

## 4.3	Scenario 2:	Discreet monitoring of MCPTT and MCVideo group calls

### 4.3.1	Discreet monitoring of an MC service user in a group call

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC service user B is within the authority of authorized MC user A, where MC user A and MC user B have the same primary MC system, MC system A. Target MC user B is a member of and affiliated to MC service group Y together with MC users C to N, and therefore the communications in MC service group Y are to be provided to MC user A. The scenario is illustrated in figure 4.3.1-1 below.

Figure 4.3.1-1: Discreet monitoring of an MC user in a group call

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A identifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC system A identifies target MC user B as a member of MC service group Y

-	An affiliated group member in MC service group Y (one of MC users B or C to N) makes an MCPTT or MCVideo group call.

-	MC system A provides MC service client A with information concerning the initiation of the call and provides the media exchanged throughout the group call, including the identity of the calling MC users.

-	When the group call completes, the call completion is conveyed to MC service client A.

NOTE 1:	The scenario does not determine whether authorized MC user A is a member of MC service group Y or not.

NOTE 2:	The scenario also applies to other variants of group call, such as emergency group call, imminent peril group call, broadcast group call, ad hoc group call, and calls to regrouped groups where MC user B is a group member.

### 4.3.2	Discreet monitoring of an MC group

This scenario describes the case where authorized MC user A requests discreet monitoring for communications within an MC group X that is within the authority of authorized MC user A, where MC user A and the MC service group X have the same primary MC system, MC system A. MC user C is a member of and affiliated to target MC service group X together with MC users D to N. The primary MC system of MC service group X is also MC system A. The scenario is illustrated in figure 4.3.2-1 below.

Figure 4.3.2-1: Discreet monitoring of an MC group

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC group X as the target for discreet monitoring. MC system A identifies that MC user A is authorized to perform discreet monitoring on MC group X. This aspect takes place before any communications involving MC group X can be subject to discreet monitoring.

-	An affiliated group member in target MC service group X (one of MC users C or D to N) makes an MCPTT or MCVideo group call.

-	MC system A provides MC service client A with information concerning the initiation of the call and provides the media exchanged throughout the group call, including the identity of the calling MC users.

-	When the group call completes, the call completion is conveyed to MC service client A.

NOTE 1:	The scenario does not determine whether authorized MC user A is a member of target MC service group X or not.

NOTE 2:	The scenario also applies to other variants of group call, such as emergency group call, imminent peril group call and broadcast group call in MC group X, to calls to in regrouped groups where MC service group X is part of.

NOTE 3:	It is FFS if and how the scenario applies to ad hoc group call.

## 4.4	Scenario 3:	 Discreet monitoring of one to one MCVideo pull

This scenario describes the case where authorized MC user A requests discreet monitoring for MC user B where MC user B is within the authority of authorized MC user A, and authorized MC user A receives MCVideo streams where target MC user B is the initiator of the MCVideo pull request to a third MC user, or is the recipient of an MCVideo pull request from a third MC user and so provides video as a result of this request. As a one to one MCVideo pull is procedurally identical to an MCVideo private call, the scenarios are the same as those described within the present document in subclause 4.2 for MCVideo private call.

## 4.5	Scenario 4:	Discreet monitoring of one to one MCVideo push

This scenario describes the case where authorized MC user A requests discreet monitoring for MC user B where MC user B is within the authority of authorized MC user A, and authorized MC user A receives MCVideo streams initiated or received by target MC user B. As a one to one MCVideo push is procedurally identical to an MCVideo private call, the scenarios are the same as those described within the present document in subclause 4.2 for MCVideo private call.

## 4.6	Scenario 5: Discreet monitoring of remotely initiated MCVideo push

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user C causes a remotely initiated MCVideo push from target MC user B to MC user D. The scenario is illustrated in figure 4.6-1 below.

Figure 4.6-1: Discreet monitoring of remotely initiated MCVideo push

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC user C initiates a remotely initiated MCVideo push procedure, wherein target MC user B pushes video to MC user D.

-	MC system A provides MC service client A with information concerning the initiation of the MCVideo push including the identities of MC users C and D, and provides the MCVideo media sent from target MC user B to MC user D.

NOTE 1:	Authorized MC user A may receive separate notifications when the MCVideo push is initiated, and when the MCVideo is provided.

NOTE 2:	The provision of MCVideo media to authorized MC user A may be in real time, or on demand with intermediate storage in the MCVideo server. This aspect may need an additional requirement on media storage server.

NOTE 3:	The scenario is the same where MC user C causes MC user D to push MCVideo media towards target MC user B; MC system A also sends the pushed video from MC user D to authorized MC user A.

NOTE 4:	The scenario is the same where MC user C remotely initiates a private call between target MC user B and MC user D.

## 4.7	Scenario 6:	Discreet monitoring of MCData SDS (using media plane)

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. The scenario is illustrated in figure 4.7-1 below.

Figure 4.7-1: Discreet monitoring of one to one MCData SDS (using media plane)

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends or receives an MCData SDS data session request to or from MC user C.

-	MC system A provides MC service client A with the MCData SDS data session request, including metadata information.

-	If the recipient sends one or more disposition notifications, these are also sent to MC user A.

NOTE 1:	The scenario also applies to enhanced status sent over SDS.

NOTE 2:	If target MC user B is sending or receiving SDS within a group, the scenario is similar to that described in subclause 4.3 for MCPTT and MCVideo group call.

## 4.8	Scenario 7: Discreet monitoring of one to one file distribution (using media plane)

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. This scenario focusses on the case where the file is distributed over the media plane. The scenario is illustrated in figure 4.8-1 below.

Figure 4.8-1: Discreet monitoring of one to one MCData file distribution (using media plane)

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends or receives an MCData file distribution request to or from MC user C.

-	MC system A provides MC service client A with the transferred file, including metadata information.

-	The MCData download completed report is also sent to MC service client A.

## 4.9	Scenario 8:	Discreet monitoring of one from server MCVideo pull

This scenario describes the case where authorized MC user A requests discreet monitoring for MC user B where MC user B is within the authority of authorized MC user A, and where authorized MC user A receives MCVideo pulls from the MC system initiated by target MC user B. In figure 4.9-1 below, MC user A and target MC user B have the same primary MC system, MC system A.

Figure 4.9-1: Discreet monitoring of MCVideo pull from server

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B initiates an MCVideo pull from the MCVideo server.

-	MC system A provides MC service client A with information concerning the initiation of the MCVideo pull, and provides the MCVideo media sent from the MCVideo server to target MC user B.

## 4.10	Scenario 9:	Discreet monitoring of one to server MCVideo push

This scenario describes the case where authorized MC user A requests discreet monitoring for MC user B where MC user B is within the authority of authorized MC user A, and where authorized MC user A receives MCVideo pushes to the MCVideo server that have been initiated by target MC user B. In figure 4.10-1 below, MC user A and target MC user B have the same primary MC system, MC system A.

Figure 4.10-1: Discreet monitoring of one to server MCVideo push

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B initiates an MCVideo push to the MCVideo server.

-	MC system A provides MC service client A with information concerning the initiation of the MCVideo push, and provides the MCVideo media sent from target MC user B to the MCVideo server.

## 4.11	Scenario 10:	Discreet monitoring of MCData SDS (using signalling control plane)

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. The scenario is illustrated in figure 4.11-1 below.

Figure 4.11-1: Discreet monitoring of one to one MCData SDS (using signalling control plane)

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends or receives an MCData SDS data request to or from MC user C.

-	MC system A provides MC service client A with the MCData SDS data request, including metadata information.

-	If the recipient sends one or more disposition notifications, these are also sent to MC user A.

NOTE 1:	The scenario applies when the data is sent over the signalling plane.

NOTE 2:	The scenario applies when the data is sent in standalone transactions.

NOTE 3:	The scenario applies to enhanced status sent over SDS.

## 4.12	Scenario 11:	Discreet monitoring of file upload and file download

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. This scenario focusses on the case where target MC user B is uploading or downloading from/to the Content Server. The scenario is illustrated in figure 4.12-1 below.

Figure 4.12-1: Discreet monitoring of MCData file upload and download

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on target MC user B. This aspect takes place before any communications involving target MC user B can be subject to discreet monitoring.

-	Target MC user B uploads or downloads a file to or from the Content Server.

-	MC system A provides MC service client A with the transferred file, including metadata information.

## 4.13	Scenario 12: Discreet monitoring of one to one file distribution (using HTTP)

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user A and target MC user B have the same primary MC system. This scenario focusses on the case where the file is distributed over HTTP. The scenario is illustrated in figure 4.13-1 below.

Figure 4.13-1: Discreet monitoring of one to one MCData file distribution (using HTTP)

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends or receives an MCData file distribution request to or from MC user C.

-	MC system A provides MC service client A with the transferred file, including metadata information.

-	The MCData download completed report is also sent to MC service client A.

NOTE 1:	Authorized MC user A may receive separate notifications when the MCData file transfer is initiated, and when the MCData file is provided.

NOTE 2:	The provision of the MCData file to authorized MC user A may be in real time, or on demand with intermediate storage in the MC system.

## 4.14	Scenario 13:	Discreet monitoring of migrated users

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC service user B is within the authority of authorized MC user A and where target MC user B has migrated to a partner system.

Discreet monitoring of migrated users is applicable to all the services that are illustrated in clause 4.

As a general example a private call MCPTT scenario is illustrated in figure 4.14-1 below.

Figure 4.14-1: Discreet monitoring of MCPTT private call with target served by partner MC system

The scenario consists of the following aspects:

-	MC user B has migrated to the partner system.

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. The primary MC system verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	The primary MC system identifies partner MC system as the MC system serving target MC user B. The partner MC system is aware that MC user B is target for discreet monitoring from the primary MC system. This aspect takes place before any communications involving target MC user B can be subject to discreet monitoring.

-	Target MC user B places or receives an MCPTT private call to or from MC user C.

-	The primary MC system provides MC service client A with information concerning the initiation of the call, including the identity of MC user C. The partner MC system and the primary MC system share the relevant information for this to take place.

-	The primary MC system provides the media exchanged between the target MC user B and MC user C in the call. The partner MC system and the primary MC system share the relevant information for this to take place.

-	When the private call completes, the call completion is conveyed to MC service client A.

NOTE:	The scenario is the same whether MC user C, the other party in the private call, is receiving MC service from the primary MC system or the partner MC system.

## 4.15	Scenario 14: Discreet monitoring with regulatory constraints on private communication transmissions

This scenario describes the case where due to regulatory constraints or operator security policies only the transmissions from the target user of discreet monitoring shall be made available.

The authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A. MC user C can be, but does not have to be within the authority of the authorized MC user A. The MC user C is not the target of the discreet monitoring request executed by the authorized MC user A. The scenario would allow MC user A to receive the communication coming from target MC user B, but not the communication coming from MC user C. The scenario is illustrated in figure 4.15-1 below.

Figure 4.15-1: Discreet monitoring with regulatory constraints on private communication transmissions

NOTE 1:	As the communication originated from MC user C is restricted, authorized user A is not able to receive the full context of the communications from target MC user B which may limit the usefulness of the discreet monitoring of target MC user B.

## 4.16	Scenario 15: Discreet monitoring with regulatory constraints on group communication transmissions

This scenario describes the case where due to regulatory constraints or operator security policies only the transmissions from the target user of discreet monitoring shall be made available.

The authorized MC user A requests discreet monitoring for communications involving MC user B being part of group communications, where MC user B is within the authority of authorized MC user A. All other participants on these group communications can, but do not have to be within the authority of the authorized MC user A.

The scenario would allow MC user A to be receive the communications from target MC user B to the MC service group, but not the communications coming from MC user C, MC user D and MC user E to the MC service group. The scenario is illustrated in figure 4.16-1 below.

Figure 4.16-1: Discreet monitoring with regulatory constraints on user who is part of group communication transmissions

The scenario consists of the following aspect:

-	Authorized MC user A is requesting discreet monitoring identifying MC user B who is part of group communications.

NOTE 1:	As the communication originated from MC user C, MC user D and MC user E is restricted, authorized user A is not able to receive the full context of the communications in the rest of the group which may limit the usefulness of the discreet monitoring service capability.

## 4.x	Scenario x: <Write the title here>

< Short description. You may use references to clauses in TS 23.xxx. Leave details to KIs (and solutions). Describe which SA1 requirements (if any) are related to this scenario. >

# 5	Key Issues

## 5.1	Key issue 1: Functional architecture for Discreet monitoring

Functional architecture for Discreet monitoring (DM) will have impact to all scenarios, key issues and solutions in this TR. It would make sense to study, evaluate and decide on the functional architecture as the first step of this study item.

There are fundamentally two options:

1)	Use the assumptions in TR 23.784, especially clause 6.1.1.1. The main motivation/benefit for this solution is to cause no changes to the functional model. However, there are surely also drawbacks that were not mentioned/analysed in the Rel-16 TR 23.784.

2)	Reuse/copy the functionality and reference points from the Recording and replay feature. This includes setting of (DM) targets and activating the DM process in the MC service servers and potentially even using same protocols (REC-4) to transfer media and metadata to the authorized MCX user (discreet listener). This solution would also add a “Discreet monitoring admin user” (similar to “Recording admin user”) and “DM admin client” into the selected functional model.

a.	Consider if this “DM admin” is enough i.e. same user/client has the authority and functionality to set the targets and (discreet) listen to them

or

b.	A more complex architecture (equivalent to recording/replay) is necessary, i.e. separate “DM admin” and “DM listener”.

Both options should be analysed (at least briefly).

## 5.2	Key issue 2: Discreet monitoring for migrated MC service user

An MC service user that has migrated to another MC system receives MC service within the security domain of that MC system.

Issues:

-	The primary MC system of the MC service user and the migrated MC system of the MC service user may both wish to authorize discreet monitoring of the migrated MC service user by the primary MC system of that migrated MC service user.

NOTE 1:	Subject to the trust relationship and operator security policies, the involvement of  (an authorized user of) the partner system for discreet monitoring for migrated MC service users may be in conflict with the requirement that discreet monitoring takes place without the knowledge of other unauthorized MC service users.

-	The primary MC system of the migrated MC service users and the migrated MC system of the migrated MC service user may both wish to authorize any discreet monitoring of the migrated MC service user by the migrated MC system.

NOTE 2:	Subject to the trust relationship and operator security policies, the involvement of  (an authorized user of) the primary system for discreet monitoring for migrated MC service users may be in conflict with the requirement that discreet monitoring takes place without the knowledge of other unauthorized MC service users.

-	There may be a need to restrict the provided content of communications to that which is originated by the migrated MC service user, and not to provide content which originates from other users receiving service within the migrated MC system.

-	There may be a need to restrict metadata relating to the communications to hide the identities and other information relating to MC service users and MC service groups whose primary MC system is the migrated MC system of the MC service user who is the target of the discreet monitoring.

## 5.x	Key issue x: <Write KI title here>

< First describe the key issue in general level. If the KI is related to scenario(s) in clause 4, it must be mentioned here. If possible, use references or mention also the related SA1 requirement(s). These are essential pieces of information to justify the KI and to enable mapping between requirements, scenarios and Kis. See examples of general descriptions e.g. from TR 23.784. >

Issues:

-	< Write issues into a bullet list, one issue per bullet. Each issue must be described preferably by one sentence. Try to be extremely clear and unambiguous. See examples from TR 23.784. >

# 6	Solutions

## 6.1 Solution 1 (for KI#1): Functional architecture

### 6.1.1	Description

#### 6.1.1.1	General

This solution re-uses functionality of the Recording feature as much as feasible. New functional entities “Discreet monitoring server” and “Discreet monitoring client” are added, as well as a number of new reference points. All new configuration parameters are mimicking those already existing for the Recording feature.

#### 6.1.1.2	Functional model and reference points

##### 6.1.1.2.1	Functional model

Functional model of MC system including Discreet Monitoring functionality is shown in figure 6.1.1.2.1-1. Note that this figure shows only a single MC system, interconnection aspects are not (yet) included.

Figure 6.1.1.2.1-1: Functional model of MC system including Discreet Monitoring functionality

NOTE 1: 	The functional architecture is only describing the required functionality for DM. How this functionality is implemented is out of scope of this document. E.g. DM client may be implemented as add-on functionality to the MC service clients and DM server may be implemented as add-on functionality to the recording server.

NOTE 2: 	The dashed lines between DM client and other clients in an MS service UE are only for illustration purposes. They will not be included into the normative TS.

##### 6.1.1.2.2	Functional entity description – DM server

The discreet monitoring (DM) server is a functional entity that receives communications metadata and media of DM target users and target groups, from MC service servers. When media from MC service servers is received in e2ee format, the DM server is capable of deciphering the media. The DM server forwards the media, using secure DM-1 reference point, to the authorized DM clients who have subscribed to receive the target user/group communications.

The target users and/or groups for DM are set in the MC service user profile configuration data (A.3) and in the group configuration data (A.4). This can be done by an authorized DM client utilizing the CSC-4 reference point (user profiles) and CSC-2 reference point (group profiles).

##### 6.1.1.2.3	Functional entity description – DM client

The discreet monitoring client is a functional entity that acts as the application user agent for discreet monitoring services. These services include:

-	Setting or modifying target users and target groups for discreet monitoring. This can be done by interacting with the configuration management server via configuration management client and with the group management server via group management client.

-	Subscribing to the DM server for receiving media of target users and/or groups.

-	Receiving the user/group media from the DM server and providing the media to the MC user.

##### 6.1.1.2.4	Reference point DM-1 (between the DM client and server)

Editor's note:	The contents of this clause are FFS.

##### 6.1.1.2.5	Reference point DM-2 (between the DM server and KMS)

Editor's note:	The contents of this clause are FFS.

##### 6.1.1.2.6	Reference point DM-3 (between the DM server MC Service Servers)

Editor's note:	The contents of this clause are FFS.

##### 6.1.1.2.7	Reference point DM-4 (between the DM server and GMS)

Editor's note:	The contents of this clause are FFS.

##### 6.1.1.2.8	Reference point DM-5 (between the DM server and CMS)

Editor's note:	The contents of this clause are FFS.

#### 6.1.1.3	Configurations

Configuration parameters for DM include:

-	DM user (authorized user) profile:

i.	Authorizations for Discreet Monitoring of users and groups – the “Recording admin and/or replay service user profile configuration data (on-network)” – Table A.11-1 in 3GPP TS 23.280 – can be used as a starting point. If we end up with only one DM client (i.e. no separate ‘admin’ and ‘replay’ clients like for the recording), the number of parameters will be smaller.

-	Target users/groups for DM:

i.	For groups, new parameters shall be added to Table A.4 in 3GPP TS 23.280 (Group configuration data) – “Group is a target for DM” and “DM server address”.

ii.	For individual users, similar parameters to the MCPTT, MCData and MCVideo user profiles as already exists for Recording i.e. “User is a target for DM” and “DM server address”.

- Editor's note:	It is FFS how these parameters can be hidden from the target users.

#### 6.1.1.4	Procedures

For configurations, the existing procedures can be used (just like for the Recording feature).

Procedures in the new reference points will mainly mimic those for Recording, but these need to be studied in more detail.

Procedures between DM client and DM server are also FFS. A secure interface is required, can use same security mechanisms as any MC client-server interface --> SA3.

MC service UE internal procedures between DM client and other clients (see figure above) are implementation specific and out of scope of 3GPP.

### 6.1.2	Impacts on existing functional entities and reference points

See figure above.

### 6.1.3	Solution evaluation

Editor's note:	The contents of this clause are FFS.

## 6.x	Solution x: <Write solution title here>

### 6.x.1	Description

#### 6.x.1.1	Functional model and reference points

#### 6.x.1.2	Configurations

#### 6.x.1.3	Procedures

### 6.x.2	Impacts on existing functional entities and reference points

### 6.x.3	Solution evaluation

# 7	Overall evaluation

# 8	Conclusions

Annex A (informative): 
List of stage 1 requirements

# A.1	Requirements for discreet monitoring

The requirements in 3GPP TS 22.280 [8] that apply for discreet monitoring are provided in table A.1-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.1-1: Discreet monitoring requirements

| Requirement | Description |
| --- | --- |
| [R-6.15.1-001a] | The MCX Service shall provide discreet listening capabilities without noticeable impact on or knowledge of the target MCX User, or the members of the target MCX Service Group including Ad hoc groups, and all other unauthorized MCX Users. |
| [R-6.15.1-001] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Private Communication transmissions to and from a specific target MCX User that is within the authority of the authorized MCX User. |
| [R-6.15.1-002] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Group Communication transmissions to and from a specific target MCX User that is within the authority of the authorized MCX User. |
| [R-6.15.1-002a] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Ad hoc Group Communication transmissions to and from a specific target MCX User that is within the authority of the authorized MCX User. |
| [R-6.15.1-003] | Subject to regulatory constraints and operator security policies, the MCX Service shall allow to be configured not to provide transmissions from MCX Users who are communicating with the discreet listening target MCX User, and who are not themselves targets of discreet listening. |
| [R-6.15.1-004] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Group transmissions from a specific MCX Service Group that is within the authority of the authorized MCX User. |
| NOTE: Permission to activate discreet listening can include multiple levels of authorization, but this is outside the scope of 3GPP. |  |

Annex B:
Mapping of stage 1 requirements to solutions

| Requirement | Scenarios | Key Issues | Solutions | NOTE |
| --- | --- | --- | --- | --- |
| [R-6.15.1-001a] [R-6.15.1-001] [R-6.15.1-002] [R-6.15.1-002a] [R-6.15.1-003] [R-6.15.1-004] | All scenarios in clause 4. | 1 |  |  |

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 | S6-251273 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | SA6#66 | S6-251274 S6-251275 S6-251276 |  |  |  | Scope References and annex A Terms | 0.1.0 |
| 2025-05 | SA6#67 | S6-252328 |  |  |  | Key issue 1 | 0.2.0 |
| 2025-09 | SA6#68 | S6-253406 S6-253407 S6-253408 S6-253409 S6-253410 S6-253411 S6-253412 S6-253413 S6-253414 S6-253415 S6-253416 S6-253417 S6-253418 S6-253419 S6-253420 S6-253421 S6-253422 S6-253423 |  |  |  | Scenario general Scenario 1 Scenario 2 Scenario 3 Scenario 4 Scenario 5 Scenario 6 Scenario 7 Scenario 8 Scenario 9 Scenario 10 Scenario 11 Scenario 12 Scenario 13 Scenario 14 Scenario 15 Key issue 2 Solution 1 | 0.3.0 |