| 3GPP TR 23.700-39 V0.2.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on logging and recording of mission critical services, Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Scenarios	7

4.1	On-network communications	7

4.1.1	Recording of MCVideo one to one pull and push	7

4.2	Off-network communications	8

4.2.x	<title>	8

5	Key Issues	8

5.1	Key Issue 1: Recording of MCVideo one to one pull and push	8

5.2	Key Issue 2: Recording target setting of MCData users and groups	8

5.3	Key Issue 3: Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets	8

5.4	Key Issue 4: Recording and replay of e2e encrypted one-to-one communication	9

5.5	Key Issue 5: Recording and replay of e2e encrypted group communication	10

6	Solutions	11

6.1	Solution 1: Recording of MCVideo one to one pull and push	11

6.1.1	Description	11

6.1.1.1	Functional model and reference points	11

6.1.1.2	Configurations	11

6.1.1.3	Procedures	11

6.1.2	Impacts on existing functional entities and reference points	11

6.1.3	Solution evaluation	12

6.2	Solution 2: Recording target setting of MCData users and groups	12

6.2.1	Description	12

6.2.1.1	Functional model and reference points	12

6.2.1.2	Configurations	12

6.2.1.3	Procedures	12

6.2.2	Impacts on existing functional entities and reference points	12

6.2.3	Solution evaluation	12

7	Overall evaluation	12

8	Conclusions	12

Annex A (informative): List of stage 1 requirements	13

A.1	Requirements for on-network recording and audit	13

A.2	Requirements for off-network recording and audit	13

Annex B (informative): Mapping of stage 1 requirements to solutions	15

Annex C (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies solutions to satisfy the requirements for logging and recording of mission critical services. It identifies enhancements to be included in the technical specifications for MC Common functional architecture (3GPP TS 23.280 [2]), MCPTT (3GPP TS 23.379 [3]), MCVideo (3GPP TS 23.281 [4]), MCData (3GPP TS 23.282 [5]), MC Interworking with LMR (3GPP TS 23.283 [6]), and Mission Critical services over 5G System (3GPP TS 23.289 [7]). The requirements for this study are taken from the Stage 1 requirements in 3GPP TS 22.280 [8].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.283: " Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[8]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[9]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.280 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] or 3GPP TS 23.280 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Scenarios

< Scenarios should be short. Sometimes even just a title is enough. As a starting point, all scenarios in TR 23.784 are still valid, but DO NOT copy paste the text/figures into this TR. Scenarios in TR 23.784 have way too much information (which belongs to KIs and even solutions) and are written based on one specific functional model for discreet monitoring. Leave details to the KIs and solutions. >

## 4.1	On-network communications

### 4.1.1	Recording of MCVideo one to one pull and push

Figure 4.1.1-1: Functional entities relevant for recording of MCVideo one to one pull and push

This scenario describes the recording of MCVideo pull or push between MC user A and MC user B.

MC user A and/or MC user B (MC Video ID A and/or MC Video ID B) has/have been set as the target for recording by a Recording admin user. The parameter for target setting is in the MCVideo user profile, see A.3 of 3GPP TS23.281 [4]. The procedure for updating the target user’s profile in the CMS is described in 3GPP TS 23.280 [2], clause 10.18.2.2.

One-to-one Video push procedure is described in 3GPP TS 23.281 [4] clause 7.4.2.3.

One-to-one Video pull procedure is described in 3GPP TS 23.281 [4] clause 7.3.2.3.

Procedure for recording communication sessions of an MC service user is described in 3GPP TS 23.280 [2], clause 10.18.3.2.

## 4.2	Off-network communications

### 4.2.x	<title>

< Short description. You may use references to clauses in TS 23.xxx. Leave details to KIs (and solutions). Describe which SA1 requirements (if any) are related to this scenario. >

# 5	Key Issues

## 5.1	Key Issue 1: Recording of MCVideo one to one pull and push

MC Video one to one pull and push procedures are MC Video private call with specific parameter values. As described in the scenario (4.1.1), the required procedures, information flows and parameters already exist in the Rel-19 MCX specifications.

To fully support this scenario, the following aspects need to be studied:

- Consider adding into 3GPP TS 23.280 [2] an explicit list of services that can be recorded with the existing procedures and reference points i.e. REC-4 and SIPREC. Remove all Rel-19 specific notes, see e.g. 3GPP TS 23.280 [2], clause 7.5.2.34.

## 5.2	Key Issue 2: Recording target setting of MCData users and groups

Functionality required for recording of MCData services was not completed in Rel-19. Those parts that were ready, including setting MCData users/groups as targets, were removed from the Rel-19 TSs, to avoid any misunderstandings of the release content.

Now this functionality needs to be added back.

Issues:

- Revert the changes (i.e. removals) done for the Rel-19 TSs

- Study if some additional parameters for MCData recording targets are required

## 5.3	Key Issue 3: Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets

This key issue focuses on the case when, in a MCPTT or MCVideo private call, MCData one-to-one communication or MCData IP connectivity, both parties are targets for recording.

The issues / alternatives to be studied are:

- Starting only one recording session

- Starting a separate recording session for each MCPTT or MCVideo private call, MCData one-to-one communication or point-to-point participant

## 5.4	Key Issue 4: Recording and replay of e2e encrypted one-to-one communication

Figure 5.4-1: Functional elements relevant for recording and replay of e2e encrypted one-to-one communication

This Key issue focuses on the case when a private call or one-to-one communication between MC client A and MC client B is e2e encrypted.

A basic assumption is that the Private Communication Key (PCK) is known only to the communication parties (client A and B) and, at some point of time, also to the Recording server. To ensure e2e confidentiality, as required in [R-6.15.4-005] of TS 22.280 [2], the PCK shall never be revealed to any external entity, including the Replay clients.

There are two options to record e2e encrypted media:

- Decrypt the media in the Recording server before storage. This option requires that the media, before storage, is also re-encrypted with some implementation specific mechanisms.

- Store the recorded media as e2e encrypted. This option requires that, when a replay is requested by an authorized user at some later point of time, the Recording server shall be able to fetch/generate the used PCK to decrypt the media.

In both alternatives, for the replay of the recordings, a secure connection between the Recording server and the Replay client is required (REC-1), but this connection and the security mechanisms used are outside the scope of 3GPP.

Issues:

- Study clause 5.6 of 3GPP TS 33.180 [9]. Is there something that should be referred to in 3GPP TS 23.280 [2], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] or 3GPP TS 23.379 [3]?

- How does the Recording server get the PCK? Study both options 1 and 2 (see above) and define – if needed - the required procedures, information flows and parameters.

NOTE: This KI will require co-operation with SA3 and the solutions may require changes also to TS 33.180 [9].

## 5.5	Key Issue 5: Recording and replay of e2e encrypted group communication

Figure 5.5-1: Functional elements relevant for recording and replay of e2e encrypted group communication

This Key issue focuses on the case when group communication is e2e encrypted.

A basic assumption is that the Group Master Key (GMK) is known only to the group members and, at some point of time, also to the Recording server. To ensure e2e confidentiality, as required in [R-6.15.4-005] of TS 22.280 [2], the GMK shall never be revealed to any external entity, including the Replay clients.

There are two alternatives to record e2e encrypted group media:

- Decrypt the media in the Recording server before storage. This option requires that the media, before storage, is re-encrypted with some implementation specific mechanisms.

- Store the recorded media as e2e encrypted. This option requires that, when a replay is requested by an auditor at some later point of time, the Recording server shall be able to fetch/generate the used GMK and decrypt the media.

In both alternatives, for the replay of recordings, a secure connection between the Recording server and the Replay client is required (REC-1), but this connection and the security mechanisms used there are outside the scope of 3GPP.

Issues:

- Study clause 5.7 of 3GPP TS 33.180 [9]. Is there something that should be referred to in 3GPP TS 23.280 [2], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] or 3GPP TS 23.379 [3]?

- How does the Recording server get the GMK? Study both options 1 and 2 (see above) and define – if needed - the required procedures, information flows and parameters

NOTE: This KI will require co-operation with SA3 and the solutions may require changes also to 3GPP TS 33.180 [9].

# 6	Solutions

## 6.1	Solution 1: Recording of MCVideo one to one pull and push

### 6.1.1	Description

#### 6.1.1.1	Functional model and reference points

As described in Key issue 1 and Scenario 4.1.1, MC Video one to one pull and push procedures are MC Video private call with specific parameter values. I.e. the existing functionality in the Rel-19 TSs can be utilized for this service.

#### 6.1.1.2	Configurations

None.

#### 6.1.1.3	Procedures

None.

### 6.1.2	Impacts on existing functional entities and reference points

The required changes/amendments for the Rel-19 TSs are clarifications. This solution proposes the following changes to TS 23.280 clause 7.5.2.34.

- - - - -

7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point for transport and routing of signalling and communication sessions related metadata.

MC service server and recording server shall use Session Recording Protocol as specified in IETF RFC-7866 [40] for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. IETF RFC-7866 [40] specifies the use of SIP, Session Description Protocol (SDP), and RTP to establish a recording session.

- - - - -

### 6.1.3	Solution evaluation

TBD.

## 6.2	Solution 2: Recording target setting of MCData users and groups

### 6.2.1	Description

#### 6.2.1.1	Functional model and reference points

This solution has no impact on the existing functional model or reference points.

#### 6.2.1.2	Configurations

The removal of MCData recording target configurations from Rel-19 - S6-250334 (TS 23.282) and S6-250335 (TS 23.280) - shall be reverted.

#### 6.2.1.3	Procedures

This solution has no impact on the existing procedures.

### 6.2.2	Impacts on existing functional entities and reference points

This solution has no impact on the existing functional model or reference points.

### 6.2.3	Solution evaluation

TBD.

# 7	Overall evaluation

# 8	Conclusions

Annex A (informative):
List of stage 1 requirements

# A.1	Requirements for on-network recording and audit

Requirements for on-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.1-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.1-1: Requirements for on-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-6.15.4-001] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of the MCX Service Group Communications and MCX Service Private Communications under the organization's authority. |
| [R-6.15.4-002] | Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-6.15.4-003] | The MCX Service shall provide a mechanism for a Mission Critical Organization to record the transmissions of the Group Communications and Private Communications under the organization's authority. |
| [R-6.15.4-004] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per communication: depending on service this may include; start time, date, MCX User ID, functional alias(es), MCX Group ID, Location information of the transmitting Participant, end time or duration, end reason, type of communication (e.g., MCX Service Emergency, regroup, private) and success/failure indication. |
| [R-6.15.4-005] | If an MCX Service Group Communication or MCX Service Private Communication uses end-to-end confidentiality, the MCX Service shall provide a mechanism for a Mission Critical Organization to maintain the end-to-end confidentiality when the MCX Service Group Communication or MCX Service Private Communication is logged. |
| [R-6.15.4-006] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of non-communication related user activities under the agency's authority. |
| [R-6.15.4-007] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following non-communication activity types: MCX Service Emergency Alert, MCX Service Emergency Alert cancellation, In-progress Emergency cancellation, registration state change, overridden event, user remote logout, changing another user's affiliations, affiliation change, change of Selected MCX Service Group, (de)activation of functional alias(es) and User and Group Regroup operations. |
| [R-6.15.4-008] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per non-communication activity: time, date, MCX Service User identity, activity type and pertinent information about the activity (e.g. MCX User/Group ID(s) included in the temporary group created by the User/Group Regroup operation). The following metadata should be logged if applicable to the activity type: MCX Service Group ID, Location information of the MCX User, affiliation list, target MCX Service User ID and success/failure indication. |
| [R-6.15.4-009] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log metadata for all failed authorization attempts (e.g., invalid login password) by an MCX User. |
| [R-6.15.4-010] | The MCX Service shall provide a mechanism to collect metadata for network access events (e.g., pre-emption of the 3GPP system bearer services, loss of signal, failed registration attempts). |
| [R-6.15.4-011] | The MCX Service shall provide recording and audit functions for all types of MCX Service group communication (e.g., normal group, regroup group, Ad hoc group). |

# A.2	Requirements for off-network recording and audit

Requirements for off-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.2-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.2-1: Requirements for off-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-7.14-001] | The Off-Network MCX Service shall provide a mechanism to collect metadata for MCX Service Group Communications and MCX Service Private Communications (e.g., initiating MCX Service User ID, MCX Service Group ID) and non-communication activities (e.g., changing group settings) from MCX UEs operating in off-network mode. Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-7.14-001a] | The Off-Network MCX Service shall provide a mechanism for a Mission Critical Organization to record the media content of the transmitting Participant of Group Communications and Private Communications under the organization's authority from MCX UEs operating in off-network mode. NOTE: The handling of storage limitation of the UE is out of scope of 3GPP. |
| [R-7.14-002] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve communication and non-communication activity metadata from an MCX UE that has collected such metadata while operating in off-network mode. |
| [R-7.14-002a] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve the media content of the transmitting Participant of Group Communications and Private Communications from an MCX UE that has recorded such media content while operating in off-network mode. |

Annex B (informative):
Mapping of stage 1 requirements to solutions

| Requirement | Scenarios | Key Issues | Solutions | NOTE |
| --- | --- | --- | --- | --- |
| [R-6.15.4-001] [R-6.15.4-002] [R-6.15.4-003] [R-6.15.4-004] | 4.1.1 | 1 | 1 |  |
| [R-6.15.4-001] [R-6.15.4-002] [R-6.15.4-003] [R-6.15.4-004] | All MCData scenarios | 2 | 2 |  |
| [R-6.15.4-001] [R-6.15.4-002] [R-6.15.4-003] [R-6.15.4-004] | All private call / one-to-one session scenarios | 3 |  |  |
| [R-6.15.4-003] [R-6.15.4-005] | All private call and one to one scenarios. | 4 |  |  |
| [R-6.15.4-003] [R-6.15.4-005] [R-6.15.4-011] | All group communication scenarios. | 5 |  |  |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 | S6-251269 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | SA6#66 | S6-251270 S6-251271 S6-251272 |  |  |  | Scope References and Annex A Terms | 0.1.0 |
| 2025-05 | SA6#67 | S6-252330 |  |  |  | Restructuring the TR | 0.2.0 |
| 2025-05 | SA6#67 | S6-252320 |  |  |  | Scenario: Recording MCVideo one to one pull and push | 0.2.0 |
| 2025-05 | SA6#67 | S6-252321 |  |  |  | Key Issue: Recording MCVideo one to one pull and push | 0.2.0 |
| 2025-05 | SA6#67 | S6-252322 |  |  |  | Solution: Recording MCVideo one to one pull and push NOTE: content of S6-252322 adapted to the clause 6 structure. | 0.2.0 |
| 2025-05 | SA6#67 | S6-252323 |  |  |  | Key Issue: Setting recording targets for MCData | 0.2.0 |
| 2025-05 | SA6#67 | S6-252324 |  |  |  | Solution: Setting recording targets for MCData | 0.2.0 |
| 2025-05 | SA6#67 | S6-252325 |  |  |  | Key Issue: Both parties of a private call or one-to-one session as recording targets | 0.2.0 |
| 2025-05 | SA6#67 | S6-252326 |  |  |  | Key Issue: Recording and audit of e2e encrypted one-to-one communication | 0.2.0 |
| 2025-05 | SA6#67 | S6-252327 |  |  |  | Key Issue: Recording and audit of e2e encrypted group communication | 0.2.0 |