| 3GPP TR 23.700-79 V0.2.0 (2020-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Support of a Gateway UE Function for Mission Critical Communication; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles – Sophia Antipolis Valbonne – FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Scenarios	6

4.1	MC gateway UE services and the relationship to MC service users/clients	6

4.1.1	General	6

5	Key issues	6

5.1	Key issue 1 – Functional Architecture for an MC gateway UE	6

5.2	Key issue 2 – Authorisation for connection of non-3GPP devices with an MC gateway UE	7

5.3	Key issue 3 – Identification of MC service users behind an MC gateway UE residing on non-3GPP devices	7

5.4	Key issue 4 – MBMS support	7

5.5	Key issue 5 – User traffic handling	8

6	Architectural requirements	8

7	Solutions	8

7.1	Functional architecture	8

7.1.1	General	8

7.1.2	Solution description	8

7.1.3	Solution evaluation	9

7.2	Connection authorisation with the MC gateway UE	9

7.2.1	General	9

7.2.2	Information flows	9

7.2.2.1	Connection authorization request	9

7.2.2.2	Connection authorization response	10

7.2.3	Procedure	10

7.2.4	Solution evaluation	11

7.3	Connection authorisation with an MC server via an MC gateway UE	11

7.3.1	General	11

7.3.2	Information flows	11

7.3.2.1	Connection authorization request	11

7.3.2.2	Connection authorization response	11

7.3.3	Procedure	11

7.3.4	Solution evaluation	12

7.4	Using IMS identities behind the MC gateway UE	13

7.4.1	General	13

7.4.2	Solution description	13

7.4.3	Solution evaluation	14

8	Overall evaluation	14

9	Conclusions	14

Annex A (informative): Change history	15

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

An MC gateway UE has the functionality of providing service access with the MC service system for multiple MC service clients. The MC gateway UE enables MC service access for those MC service clients operating on devices that have no MC service capabilities (incl. 3GPP transport). This technical report identifies the key issues and corresponding solutions with recommendations for the normative work.

# 1	Scope

The present document studies solutions to satisfy the requirements for a Gateway UE function. It identifies enhancements to be included in the technical specifications for MCPTT, MCVideo, MCData and in the common functional architecture to support mission critical communications. Requirements for this study are taken from stage 1 requirements, including 3GPP TS 22.179 [2] and 3GPP TS 22.280 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe)".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MC gateway UE: A UE that can be simultaneously shared between multiple MC service clients using the same or different MC services.

MC client: A client that represents a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: A server that represents a set of servers (i.e. MC service server and servers of the Common services core).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Scenarios

## 4.1	MC gateway UE services and the relationship to MC service users/clients

### 4.1.1	General

The MC gateway UE service may provide the following necessary functions:

-	Allowing an MC service client to connect to a MC gateway UE.

-	Ensure that the content of communications between the MC system and MC service client connected to an MC gateway UE is unaltered.

-	Ensure that the communication traffic attributes, e.g. priority and QoS, of an MC service client connected to an MC gateway UE remains independent from other MC service clients concurrently connected to the same MC gateway UE.

-	Independent MC service client identification, authorisation and service profile association using the gateway MC UE functionality.

-	Sharing of an MC gateway UE transport resources with multiple connected MC service clients.

-	Indication of potential MC gateway UE anomalies sent towards the connected MC service clients.

-	MC service client is serving an MC service user on a device which is connected to the MC gateway UE via non-3GPP transport.

# 5	Key issues

## 5.1	Key issue 1 – Functional Architecture for an MC gateway UE

The MC gateway UE may provide several sub-functions like an MC gateway UE user device connecting function, an MC gateway UE network function, an MC gateway UE user QoS handling function, an MC gateway UE exposure function, an MC gateway UE MBMS support function and other functions.

A functional architecture for the MC gateway UE may help to develop proper mechanisms and procedures.

List of key issues:

-	Clarify the need to define a functional architecture for the MC gateway UE.

-	Describe the overall functional architecture and the details of the single elements from that architecture.

## 5.2	Key issue 2 – Authorisation for connection of non-3GPP devices with an MC gateway UE

A control mechanism for connecting non-3GPP devices with an MC gateway UE is needed as not all devices should be connected with all MC gateway UEs. An authorized usage of an MC gateway UE by clients and users is needed as well as a control mechanism on how many MC gateway UEs can be used by the same MC service user/client. If a connection related temporary identifier and an explicit login procedure is required (e.g. for traffic routing) needs to be clarified.

List of key issues:

-	Clarify how to authorize the use of non-3GPP devices by the MC gateway UE.

-	Clarify how to control MC service user/client use of multiple MC gateway UEs and to control the related traffic aggregation.

-	Clarify whether there is a need for an explicit login procedure and assignment of a connection related temporary non-3GPP device ID.

## 5.3	Key issue 3 – Identification of MC service users behind an MC gateway UE residing on non-3GPP devices

The mission critical user identity (or MC ID) is the identity that an MC service user presents to the identity management server during a user authentication transaction (not necessarily tied to a single mission critical service).

The MC service user identity (or MC service ID) is used as a globally unique identifier within the MC service that identifies an MC service user.

The MC ID and the MC service ID may be the same.

The SIP signalling control plane uses a private user identity to authenticate the signalling user agent and one or more public user identities for routing of signalling messages in the SIP core.

There are several relationships between the MC service ID(s) and the public user identity(ies).

List of key issues:

-	Clarify whether the MC ID can be used on non-3GPP devices connected to the MC system via an MC gateway UE. Identify impacts to existing authentication procedures.

-	Clarify whether the MC service ID can be used on non-3GPP devices connected to the MC system via an MC gateway UE. Identify impacts or limitations on existing procedures in the application plane.

-	Elaborate how and if SIP signalling plane identities can be used by non-3GPP devices connected to the MC system via an MC gateway UE.

-	Elaborate the impacts regarding the current relationships between SIP signalling plane identities and application plane identities.

-	Investigate whether there is a need for additional application plane identities to support non-3GPP devices connected to network via an MC gateway UE.

## 5.4	Key issue 4 – MBMS support

If non-3GPP devices are connected to the MC system via an MC gateway UE, then MBMS support should be supported. MBMS support is required for both, MC service clients residing on a UE acting as MC gateway UE and for MC service clients residing on a non-3GPP device. The MBMS use via non-3GPP devices should be transparent for network, i.e. the network is unaware whether the MC gateway UE is currently in use or not.

List of key issues:

-	Clarify how MBMS transmissions can be supported for MC service clients operating on non-3GPP devices via an MC gateway UE.

-	Clarify whether there any implications on switching between unicast and multicast.

-	Clarify how the MC gateway UE forwards the MBMS downlink traffic to the corresponding MC service clients behind the MC gateway UE.

## 5.5	Key issue 5 – User traffic handling

User data traffic and signalling information needs to be routed to/from user/clients residing on non-3GPP devices. The MC gateway UE needs to deal with multiple non-3GPP devices on one side and with multiple bearers (LTE) and 5G QFIs on the network side. The communication content should be unchanged, E2E encryption should be supported.

List of key issues:

-	Clarify how the MC gateway UE routes and maps the traffic data and signalling information between non-3GPP devices and the network.

-	Clarify whether there are any limitations in the MC gateway UE network connectivity aspects like max. number of PDU sessions, max. QFIs per PDU session etc.

-	Elaborate how independent QoS and priority treatment of each communication per MC service client is enabled for a MC service user behind an MC gateway UE.

-	Clarify how the content of a MC service user/client communication beyond an MC gateway UE remains unchanged.

-	Clarify how end-to-end encryption (E2EE) is enabled for MC service users/clients residing on non-3GPP devices.

# 6	Architectural requirements

Editor's Note:	This clause will provide architectural requirements.

# 7	Solutions

## 7.1	Functional architecture

### 7.1.1	General

This solution addresses the key issue 1 described in clause 5.1 on defining a functional architecture when using an MC gateway UE.

### 7.1.2	Solution description

The MC gateway UE offers access to the MC server for several MC clients (see Figure 7.1.2-1). The MC clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access.

For non-3GPP devices which can host an MC client, the MC gateway UE enables connectivity to the MC server. For non-3GPP devices which cannot host the MC client, the MC gateway UE hosts the instantiation of the MC client for the non-3GPP device.

Figure 7.1.2-1: Functional architecture

The MC gateway UE provides MC service capabilities and 3GPP access capabilities using 3GPP network credentials for authorized access with an MC server.

For non-3GPP devices which cannot host MC clients, the MC gateway UE instantiates an MC client which is acting on behalf of the non-3GPP device to enable connection with an MC server.

NOTE:	MC clients residing on a non-3GPP device cannot use UICC credentials to perform authorisation with the 3GPP transport system.

For MC clients getting MC service access via the MC gateway UE, the MC gateway UE forwards (unmodified) signalling and media from the individual MC clients to the MC server and vice versa.

Editor's Note:	Further study is needed to elaborate the details about user traffic handling in order to support e.g. different QoS types and various latency needs, which cannot be provided by basic IP relay functionality.

Editor's Note:	Further study is needed to elaborate single elements from that baseline functional architecture (e.g. device connecting function, traffic handling function, MBMS support function).

Editor's Note:	The support of using multiple MC gateway UEs simultaneously by an MC client is FFS.

### 7.1.3	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.2	Connection authorisation with the MC gateway UE

### 7.2.1	General

This solution addresses the key issue 2 described in clause 5.2 on authorisation for connection of non-3GPP devices with an MC gateway UE. The solution only applies to non-3GPP devices which can host an MC client.

### 7.2.2	Information flows

#### 7.2.2.1	Connection authorization request

Table 7.2.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE.

Table 7.2.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |

#### 7.2.2.2	Connection authorization response

Table 7.2.2.2-1 describes the information flow connection authorization response sent from the MC gateway UE to the MC client.

Table 7.2.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request. |

### 7.2.3	Procedure

The procedure for connection authorisation with an MC gateway UE is shown in figure 7.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client has been provided with an appropriate MC service ID.

Editor's Note:	Whether separate MC gateway UE authentication with the MC server and specific MC gateway UE configuration is needed requires further study.

Editor's Note:	How the MC client discovers the MC gateway UE is FFS.

Figure 7.2.3-1: Connection authorisation with the MC gateway UE

1.	The MC client requests connection authorization with the MC gateway UE. The MC client of the MC service user provides the MC service ID.

2.	The MC gateway UE performs an authorization check by using the provided MC service ID to verify that the MC service user is permitted to use the MC gateway UE to access MC services.

NOTE:	The criteria for the authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request connection authorization with the MC gateway UE.

3.	The MC gateway UE sends the connection authorization response to the MC client, confirming successful connection with the MC gateway UE.

The MC client has now access to an MC server and may continue with user authentication and service authorization.

### 7.2.4	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.3	Connection authorisation with an MC server via an MC gateway UE

### 7.3.1	General

This solution addresses the key issue 2 described in clause 5.2 on authorisation for connection of non-3GPP devices with an MC gateway UE. The solution only applies to non-3GPP devices which can host an MC client.

### 7.3.2	Information flows

#### 7.3.2.1	Connection authorization request

Table 7.3.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the MC server.

Table 7.3.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |

#### 7.3.2.2	Connection authorization response

Table 7.3.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC gateway UE, and from the MC gateway UE to the MC client residing on a non-3GPP device.

Table 7.3.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request. |

### 7.3.3	Procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 7.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client has been provided with an appropriate MC service ID.

Editor's Note:	Whether separate MC gateway UE authentication with the MC server and specific MC gateway UE configuration is needed requires further study.

Editor's Note:	How the MC client discovers the MC gateway UE is FFS.

Figure 7.3.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The MC client requests connection authorization via the MC gateway UE with an MC server. The client of the MC service user provides the MC service ID.

2.	The MC gateway UE performs an authorization check to verify that the MC service user is permitted to use the MC gateway UE to access the MC server.

NOTE:	The criteria for the authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request connection authorization via the MC gateway UE with the MC server.

3.	The MC gateway UE sends the connection authorization request to the MC server.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE is permitted.

Editor's Note:	The configuration of the MC server to perform the authorisation check needs further study.

Editor's Note:	Whether there is a need for control when the MC service user/client uses multiple MC gateway UEs simultaneously is FFS.

5.	The MC server marks the MC service user as allowed to have access via the MC gateway UE.

Editor's Note: It is FFS whether and how the MC server can mark the MC service user as allowed to have access before user authentication and service authorisation has been performed.

6.	The MC server sends the connection authorization response to the MC gateway UE.

7.	The MC gateway UE sends the connection authorization response to the MC client, confirming that successful connection via the MC gateway UE and connection with the MC server is enabled.

The MC client has now access to the MC server and may continue with user authentication and service authorization.

Editor's Note:	Whether this procedure can be combined with existing mission critical communication procedures (e.g. user authentication, service authentication) needs further study.

### 7.3.4	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.4	Using IMS identities behind the MC gateway UE

### 7.4.1	General

This solution addresses the key issue 3 described in clause 5.3 on identification of MC service users behind an MC gateway UE residing on non-3GPP devices.

### 7.4.2	Solution description

The MC gateway UE offers access to the MC system for several MC service clients. The MC service clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access. Simultaneous sharing of an MC gateway UE by various MC service clients requires a relationship between the MC service identities used by the MC service clients with media streams passing the MC gateway UE towards the MC system to share the network transport resources of the MC gateway UE.

It is assumed that the MC service user behind the MC gateway UE has no 3GPP transport access credentials (i.e. no UICC), whereas the MC system relies on IMS identities (i.e. IMPUs/IMPIs) needed for authorisation with the IMS. In addition, in MC systems there exist a one-to-one correspondence of MC service IDs with IMPUs for enabling routing of signalling traffic between the MC service server and the MC service client.

Taking the above into account two options are considered.

Option 1 (see Figure 7.4.2-1) relies on MC service IDs provided by the MC gateway UE. The MC service clients behind the MC gateway UE share the credentials from the IMS subscription of the MC gateway UE. The number of MC service clients behind the MC gateway UE is determined by the number of subscribed MC service IDs that are associated with the IMS subscription of the MC gateway UE.

Figure 7.4.2-1: Sharing the MC gateway UE's IMS subscription

Editor's Note:	It is FFS whether the solution supports the simultaneous use of multiple MC gateway UEs by an MC service client.

Option 2 (figure 7.4.2-2) relies on the use of the IMS Credentials (IMC) application specified in 3GPP TS 23.228 [4]. The MC service clients behind the MC gateway UE use their own IMS subscription and do not rely on the IMS subscription from the MC gateway UE. The number of MC service clients behind the MC gateway UE is independent from the number of subscribed MC service IDs that are associated with the IMS subscription of the MC gateway UE.

Figure 7.4.2-2: Using independent IMS subscriptions

Both options support an unambiguous identification of MC service clients behind the MC gateway UE. Option 2 also has the degree of freedom that the user profiles associated with the devices behind the MC gateway UE are independent of the MC gateway user profile.

### 7.4.3	Solution evaluation

Both options support an unambiguous identification of MC service clients behind the MC gateway UE. Option 2 offers the degree of freedom that the user profiles used on the non -3GPP devices behind the MC gateway UE are independent from those of the MC gateway user profile.

Editor's Note:	Further evaluation of both options is needed before making a final assessment.

# 8	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

# 9	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the study item activities.

# Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | SA6#37-e |  |  |  |  | TR skeleton | 0.0.0 |
| 2020-06 | SA6#37-e |  |  |  |  | S6-200655, S6-200656, S6-200657 | 0.0.1 |
| 2020-08 | SA6#38-e |  |  |  |  | S6-201195, S6-201196, S6-201257, S6-201198, S6-201199, S6-201200 | 0.1.0 |
| 2020-10 | SA6#39-BIS-e |  |  |  |  | S6-201738, S6-201888, S6-201889, S6-201890, S6-202022 | 0.2.0 |