| 3GPP TR 23.700-83 V0.2.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application layer support for AI/ML services Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Architectural Assumptions and Principles	10

4.1	Analysis of relation between AIMLE and ADAE services	10

4.2	Architectural Assumptions	10

5	Key issues	11

5.1	Key issue #1: Key issue on support for ML model inference	11

5.1.1	Description	11

5.1.2	Open Issues	11

5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer	11

5.2.1	Description	11

5.2.2	Open Issues	12

5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios	12

5.3.1	Description	12

5.3.2	Open Issues	12

5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers	13

5.4.1	Description	13

5.4.2	Open Issues	13

5.5	Key issue #5: Support for AI Inference Exposure	14

5.5.1	Description	14

5.5.2	Open Issues	14

5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios	14

5.6.1	Description	14

5.6.2	Open Issues	14

5.7	Key Issue #7: VFL sample alignment across VAL servers	15

5.7.1	Description	15

5.7.2	Open Issues	15

5.8	Study on Support for AI Inference service performance management and feedback	15

5.8.1	Description	15

5.8.2	Open Issues	15

5.x	Key issue #x: <Title>	16

5.x.1	Description	16

5.x.2	Open Issues	16

6	Enhanced architecture for enabling AI/ML services	16

6.1	General	16

6.2	Enhanced AI/ML enablement functional model	16

6.3	Functional Elements	16

6.4	Reference Points	16

7	Solutions	16

7.0	Mapping of solutions to key issues	16

7.1	Solution #1: Enhancement of AIMLE to support model inference	17

7.1.1	Solution description	17

7.1.2	Architecture Impacts	17

7.1.3	Corresponding APIs	17

7.1.4	Solution evaluation	18

7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection	18

7.2.1	Solution Description	18

7.2.1.1	Impact to existing AIMLE procedures	19

7.2.2	Architecture Impacts	23

7.2.3	Corresponding APIs	23

7.2.4	Solution evaluation	23

7.3	Solution #3: New AIMLE Service on ML Model Maintenance	23

7.3.1	Solution Description	23

7.3.1.1	Procedure	24

7.3.1.2	Information flows	25

7.3.1.2.1	ML model maintenance subscription request	25

7.3.1.2.2	ML model maintenance subscription response	25

7.3.1.2.3	ML model maintenance notification	25

7.3.2	Architecture Impacts	26

7.3.3	Corresponding APIs	26

7.3.4	Solution evaluation	26

7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.	26

7.4.1	Solution description	26

7.4.1.1	Introduction	26

7.4.1.2	ML model evaluation information storage	26

7.4.1.3	ML model evaluation information retrieval	27

7.4.1.4	ML model evaluation information removal	28

7.4.2	Architecture Impacts	29

7.4.3	Corresponding APIs	29

7.4.3.1	ML Evaluation Profile	29

7.4.3.2	ML Evaluation Information Storage	29

7.4.3.3	ML Evaluation Information Retrieval	30

7.4.3.4	ML Evaluation Information Removal	30

7.4.4	Solution evaluation	31

7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness	31

7.5.1	Solution description	31

7.5.2	Architecture Impacts	33

7.5.3	Corresponding APIs	33

7.5.4	Solution evaluation	33

7.6	Solution #6: Enhancements to performance monitoring	33

7.6.1	Solution description	33

7.6.1.1	Enhancements to ML model performance monitoring	33

8.22.2	Procedure	34

7.6.1.2	Enhancements to HFL training	35

8.12.2	Procedure	36

7.6.2	Architecture Impacts	37

7.6.3	Corresponding APIs	37

7.6.4	Solution evaluation	37

7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios	37

7.7.1	Solution description	37

7.7.2	Architecture Impacts	39

7.7.3	Corresponding APIs	39

7.7.4	Solution evaluation	39

7.8 Solution #8: Multi-Client Split AI/ML Operations Pipeline	40

7.8.1 Solution Description	40

7.8.1.1	General	40

7.8.1.2	Multi-client pipeline operation	41

7.8.1.3	Multi-client pipeline service continuity	42

7.8.2 Architecture Impacts	43

7.8.3 Corresponding APIs	43

7.8.4 Solution Evaluation	43

7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment	43

7.9.1	Solution description	43

7.9.1.1 AIMLE server registration	44

7.9.1.2 AIMLE server registration update	44

7.9.1.3 AIMLE server de-registration	45

7.9.2	Architecture impacts	46

7.9.3	Corresponding APIs	46

7.9.3.1	AIMLE server registration request	46

7.9.3.2	AIMLE server registration response	46

7.9.3.3	AIMLE server registration update request	47

7.9.3.4	AIMLE server registration update response	47

7.9.3.5	AIMLE server de-registration request	47

7.9.3.6	AIMLE server de-registration response	47

7.9.4 Solution evaluation	48

7.x	Solution #x: <title>	48

7.x.1	Solution description	48

7.x.2	Architecture Impacts	48

7.x.3	Corresponding APIs	48

7.x.4	Solution evaluation	48

8	Deployment scenarios	48

8.1	General	48

8.x	Deployment model #x: <Title>	48

9	Business Relationships	48

10	Overall evaluation	49

11	Conclusions	49

11.1	General conclusions	49

11.2	Conclusions of key issue #x	49

Annex A (informative): Change history	50

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

As part of Rel-19, SA6 normatively specified the application layer architecture to enable AI/ML services which introduced as a new SEAL service, aka AIMLE, as well as a new ML repository, in 3GPP TS 23.482 [2]. Furthermore, as part of Rel-19 work, in 3GPP TS 23.436 [3] enhancements have been specified for supporting ML-enabled analytics.

In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. In this direction, some already defined concepts need further study related to application layer ML model performance monitoring and correctness evaluation, extensions to AIML split operation, enhancements in VFL support considering also VFL participants at the edge/cloud; and enhancements for supporting different deployments and federation/multi-operator scenarios.

This technical report identifies the key issues and corresponding application architecture enhancements and related solutions with recommendations for the normative work.

# 1	Scope

The present document is a technical report which identifies enhancements to the application enablement layer architecture, capabilities, and services to support AI/ML services at the application layer.

The aspects of the study include the continuation of the investigation of application enablement impacts, the application enablement layer architecture enhancements and solutions needed to provide assistance in AI/ML operations at the VAL layer as well as at the application enablement layer (e.g., SEAL ADAES, EDGEAPP).

The study takes into consideration the work done for AI/ML in 3GPP TS 23.436 [3] and 3GPP TS 23.482 [2] and can consider other AI/ML related works within 3GPP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service	".

[3]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Principles

## 4.1	Analysis of relation between AIMLE and ADAE services

In this clause an analysis of the relation of ADAE analytics and AIMLE potential service on ML model inference is discussed. Initially a definition is provided about the possible role of AIMLE vs ADAES in inference and analytics:

1.	Analytics as exposed by ADAE, does not necessarily rely on AI/ML models. The consumer simply requests analytics and receives insights—without needing to know or specify how those insights are derived. Thus, when requesting analytics a consumer does not provide any information in relation to the ML model even if such model is used as it is abstracted away from it.

2.	Inference, on the other hand, is fundamentally tied to the use of a specific ML model. The consumer must either provide a model, model information or define criteria for selecting a model. This makes the ML model an integral part of the inference service. While inference can be used to support analytics, it does not itself expose analytics as a service.

## 4.2	Architectural Assumptions

The following high-level assumptions and principles will be considered in this study for ADAES:

1.	ADAES is the entity providing analytics which can be either ML-enabled or not.

2.	If ADAES analytics is ML-enabled, the ML training capability is provided by AIMLE (Rel-19 AIMLAPP).

3.	If ADAES analytics is ML-enabled, the ML inference capability can be provided as part of the analytics functionality inside ADAES (internal to ADAES logic). ML model inference is one of methods utilized by ADAE for Data analytics.

The following high-level assumptions and principles will be considered in this study for AIMLE:

1.	AIMLE provides ML support capabilities as of Rel-19 for both VAL and other SEAL servers (Rel-19 AIMLAPP).

2.	AIMLE provides ML model training capability for both VAL task and ADAE analytics (Rel-19 AIMLAPP).

3.	AIMLE is not expected to provide ML model inference for tasks related to ADAE analytics.

4.	AIMLE can only be seen as a candidate entity for performing ML model inference for VAL/ASP automation tasks (needs to be discussed further within Rel-20 study).

The following high-level assumptions and principles will be considered in this study for ML repository:

1.	The ML repository is the unique repository / registry per SEAL provider for the application-layer ML models and the parameters required for the AIMLE operations (Rel-19 AIMLAPP).

2.	Different deployments and cardinalities of ML repository within the SEAL provider’s domain will be studied as part of Rel-20 work.

# 5	Key issues

## 5.1	Key issue #1: Key issue on support for ML model inference

### 5.1.1	Description

ML model inference, according to 3GPP TS 28.105, is a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

ML model inference may be provided by an analytics entity (e.g. ADAES) or may be provided by VAL server (in case of VAL-provided analytics) or potentially by AIMLE if the scenario involves the VAL server to request from AIMLE to provide ML model inference functionality (e.g. using network data or edge platform data).

In Rel-19 AIML_App, there was some basic support for operations related to split ML model inference; however, the use of ML model inference for supporting ADAE-layer analytics as well as the AIMLE support for enabling ML model inference  (as part of the ML model lifecycle operations) was not discussed.

In this key issue, further enhancements in both AIMLE and ADAES will be explored for supporting ML model inference in ADAES and AIMLE, considering also impacts on the interfaces among AIMLE and ADAE entities in distributed AIMLE / ADAES deployments.

### 5.1.2	Open Issues

The following open issues need to be studied:

1.	Identify deployment models for supporting ML model inference in enablement layer.

2.	Whether and how ADAES needs be enhanced or leverage AIMLE for supporting ML model inference for ADAE analytics.

3.	Whether and how AIMLE needs to be enhanced to enable ML model inference to address VAL request (e.g. for VAL automation task or analytics).

4.	Whether and how to enhance AIMLE to support model inference in edge scenarios.

5.	Whether and how to enhance AIMLE architecture and functions to support collaborative inference between AIMLE client and edge AIMLE server.

NOTE:	Solutions for the above key issue should not introduce any inference services exposed by AIMLE (e.g., producing prediction result to third party).

## 5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

### 5.2.1	Description

KI #2 focuses on providing enhanced means to evaluate and determine ML model performance and correctness in the AIML Enablement Layer.

The evaluation of performance and correctness of a ML model may depend on the ML model, and the means for evaluating performance and correctness of a ML model may vary depending on the ML model usage and purpose.

ML model performance and correctness evaluation may require evaluating different aspects of the ML model. For example, the performance and correctness of a ML model may be related to its robustness (e.g., capability for handling unseen or drifting underlying data) or may be associated with its stability (e.g., small changes in input not leading to drastic prediction change).

A procedure to monitor ML model performance degradation by VAL server has been defined in clause 8.22 of 3GPP TS 23.482 [2]. However, this procedure is limited to evaluating performance degradation at the AIMLE client for ML model training and HFL use cases and does not clearly define the information needed for the AIML enablement layer to perform performance and correctness evaluation.

### 5.2.2	Open Issues

This key issue will study the following AIML enablement layer aspects:

1.	How to define ML model performance and ML model correctness in the context of the AIML enablement layer.

2.	Whether and how to evaluate ML model performance and correctness considering application specific ML models.

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

4.	Whether and how to trigger corrective actions on ML model performance and correctness degradation detection (e.g., model maintenance).

## 5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

### 5.3.1	Description

As part of Rel-19, AIMLE was introduced as an enabler for application-layer AI/ML services, supporting model training, distribution primarily within a single operator’s network domain. However, as AI/ML services increasingly involve VAL UEs moving across network domains (e.g., cross-PLMN mobility, UE roaming, federation across operators), and AI/ML services becoming increasingly distributed and dynamic, the ability to ensure service continuity and mobility of AIMLE functions becomes a critical requirement.

This key issue aims to study how AIMLE services can support mobility, including roaming scenarios where UEs access AI/ML services in visited networks, and cross-PLMN environments where AI/ML operations involve entities from different operators. This will address both the continuity of AI/ML services and the federation across MNOs and CSPs/ECSPs.

In particular, as ML model inference increasingly moves toward the network edge to meet ultra-low latency and bandwidth efficiency demands (e.g., in real-time video analytic, industrial automation), maintaining inference continuity across mobility events becomes critical. When VAL UEs move across network boundaries or service areas, edge AIMLE instances may need to be reselected, dynamically discovered, or seamlessly replaced to ensure uninterrupted AI/ML inference execution and delivery.

The current AIMLE specifications and ongoing studies focus on static deployments and assume the VAL UE remains within its home network. However, real-world deployments often require:

1.	VAL UEs to maintain access to AIMLE services when roaming into visited PLMNs or NPNs,

2.	Cross-border AI/ML service collaboration (e.g. in eV2X or multiplayer gaming scenarios),

3.	Edge inference or AI model access to continue seamlessly even as the UE moves between coverage areas.

### 5.3.2	Open Issues

Therefore, the following aspects need to be studied:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

NOTE: Open issues 2, 3 and 4 will be based on the identified use cases in open issue 1.

## 5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers

### 5.4.1	Description

Split AI/ML operations have been introduced in 3GPP TS 23.482 [2]. The current framework is for single AIMLE client-to-network interaction and does not consider multiple AIMLE clients collaborating in a split AI/ML scenario. If  multiple AIMLE clients were to collaboratively execute split AIML operations within the same pipeline involving an Edge Server, the operation continuity may be disrupted if one client moves out of the service area during execution. Therefore, it is important to study the service continuity and optimization for split AIML operations involving multiple AIML clients.

In addition to the above multi-client scenario, emerging AI/ML applications require collaborative inference across multiple computing nodes (UE(s), edge(s), and cloud(s)) rather than a single operation. For example, in an autonomous driving or V2X scenario, a VAL UE (e.g. a connected vehicle) may only execute part of a model locally and rely on a roadside edge server or even a cloud server to complete the remaining inference. This is due to the vehicle’s limited computing resources and the need for low-latency results to ensure safe and efficient driving decisions.

In Release-19, basic support for split ML model inference between an AIMLE client and an AIMLE server was introduced in 3GPP TS 23.482 [2]). However, those solutions assumed a relatively static allocation of inference tasks and did not fully explore more dynamic or collaborative inference involving multiple nodes. Therefore, it is necessary to study whether and how to enhance the AIML application enablement architecture and functions to support cloud-edge-device collaborative inference considering the conditions changes on these computing nodes (e.g. due to variations in edge server load).

### 5.4.2	Open Issues

This key issue will study:

1.	Define the use cases supported in the AIML enablement layer involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers requiring concurrent usage of the same split AI/ML operation pipeline.

2.	Whether and how the AIML enablement layer can support split AI/ML operations involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers.

3.	Whether and how the AIMLE enablement layer can support service continuity during split AIML operation involving multiple AIMLE clients.

4. Whether and how to support dynamic inference partitioning and coordinating across AIMLE clients, edge AIMLE servers and central AIMLE servers in AIMLE layer.

NOTE:	This open issue may involve the ability to split an AI/ML inference pipeline into segments that can be executed on different nodes based on real-time conditions such as latency requirements, processing capabilities, network bandwidth, and current load (which may be provided by VAL UE or 5GC).

## 5.5	Key issue #5: Support for AI Inference Exposure

### 5.5.1	Description

With the increasing deployment of ML model at the edge and cloud, the need for reusable, secure, and discoverable inference capabilities is becoming critical for enabling intelligent applications in verticals such as industrial automation, automotive, etc.

Currently, AI/ML inference is typically tightly coupled to the operator’s own applications or specific vertical services. There is a growing demand to open up these capabilities as a standardized inference service that can be consumed by third-party applications or other service enablers. By exposing inference functions through standardized APIs, operators can enable new use cases, for example, an enterprise application could send sensor data to a mobile edge and request an AI-based analysis (inference) result in real time based on a suitable ML model. This requires exposing the inference capabilities of ADAES with the support from AIMLE services Therefore, enhancements of ADAES is needed to support the exposure of inference services based on the suitable ML model provided by AIMLE service.

### 5.5.2	Open Issues

Therefore, in order to support ML Model inference services exposure, the following aspects need to be studied:

1.	Whether and how to enhance ADAES service capabilities to expose ML model inference result based on the suitable ML model provided by AIMLE service in both central and edge scenarios.

## 5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios

### 5.6.1	Description

SA6 has specified the application layer architecture to enable AI/ML services. In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. It requires for more investigation on the implications when supporting AIMLE capabilities in scenarios where the AIMLE is deployed in distributed or hierarchical fashion as indicated in Annex A.3 and A.4 of 3GPP TS 23.482 [2]. In this study, it is expected to explore potential enhancement for supporting such deployments, considering also different implementations of the ML repository in edge or cloud platforms, and different assumptions on the access of ML repositories by VAL and other enablers. In this regard, considering the various AIML capabilities provided by the VAL UE, the collaboration procedures among AIMLE Clients, edge AIMLE servers, and central AIMLE servers need to be enhanced in terms of data management and AIML operation.

### 5.6.2	Open Issues

This key issue will study:

1.	Study and identify the use cases and definition of hierarchical and distributed AIMLE architecture.

2.	Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

3.	Whether and how to enhance the architecture and related functions to support for AIMLE services, AIML operation assuming diverse (e.g., hierarchical and distributed) deployments scenarios.

4.	Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

## 5.7	Key Issue #7: VFL sample alignment across VAL servers

### 5.7.1	Description

In the Vertical Federated Learning (VFL) process, the active party or coordinating server collects partial results from other participants (passive parties) to obtain a global model. VFL participants can be VAL servers, AIMLE servers, and/or AIMLE clients. VFL participants share the same sample space but have different feature sets. Therefore, to achieve VFL process, these different VFL participants require sample alignment to participate in a VFL operation. In some cases, how these samples are identified by different participants can also differ. Sample alignment is crucial for this process as it allows matching corresponding data entries across different datasets in a privacy-preserving manner.

The AIMLE server, which has the ability to know the identity of VAL UEs, can accomplish sample alignment securely and in a privacy-preserving way without revealing the VAL UE identity.

### 5.7.2	Open Issues

This key issue will study:

1.	Whether and how to enhance VFL operation considering also VAL servers and AIMLE servers as VFL participants.

2.	Whether and how to support sample alignment also considering VAL servers / AIMLE servers as VFL participants.

## 5.8	Study on Support for AI Inference service performance management and feedback

### 5.8.1	Description

AIMLE architecture currently provides mechanisms to request ML inference but lacks support for managing and observing the runtime quality of inference services. For example, in a vehicle-road collaboration scenario, a vehicle may request a semantic perception result that must be returned within 50ms. If that is not achieved, the application should be notified. Similarly, in scientific computing, model inference may degrade in accuracy due to hardware limitations, the feedback to the control plane is needed. Therefore, AIMLE architecture should consider providing:

-	A way for VAL UEs or AIMLE clients to express desired service performance parameters when requesting inference (e.g., max latency, expected confidence level),

-	Runtime monitoring of whether these conditions are met, and notification or reporting mechanisms for inference failures, degradation, or exceptions,

Although AI models are central to inference, the service performance of inference delivery depends on more than the model itself. The same model may produce different service experiences depending on runtime conditions (e.g., server load, network delays). The QoS of a model is generally static unless the model is retrained, whereas the performance of inference services may vary depending on resource conditions. Even if the model degrades, the inference service may still meet the required service performance.AI inference service consumers care about whether the results are delivered on time, not just whether the model is accurate. Thus, service performance observability for inference is a distinct layer of functionality, necessary to make AIMLE more reliable and useful in dynamic and distributed deployments. This is particularly important in multi-edge deployments, and industrial scenario.

### 5.8.2	Open Issues

Therefore, the following aspects need to be studied:

1.	Whether and how AIMLE should support expressing inference service performance parameters (e.g., expected latency, accuracy, confidence level) in service requests.

## 2.	Whether AIMLE requires mechanisms for real-time feedback from clients (e.g., inference result too late, failed, or inaccurate).

## 5.x	Key issue #x: <Title>

Provide a suitable title for the key issue.

### 5.x.1	Description

This subclause will provide the description of the key issue, including also the gap analysis against Rel-19 AI/ML support capabilities.

### 5.x.2	Open Issues

This subclause will list the open issues related to this key issue.

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

This subclause will illustrate the high-level architecture enhancements over Rel-19 AIMLE architecture for supporting AI/ML services.

## 6.3	Functional Elements

The functional elements corresponding to the architecture will be presented in this clause.

## 6.4	Reference Points

The reference points corresponding to the architecture will be presented in this clause.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |

## 7.1	Solution #1: Enhancement of AIMLE to support model inference

### 7.1.1	Solution description

This clause describes the procedure for supporting the ML model inference by the AIMLE/ADAE server. The VAL server requests the AIMLE/ADAE server for ML model inference by specifying the details about the ML model or requirements to select a ML model for inference.

Pre-condition:

-	The ML model needed to be used for this ML model inference is available by the AIMLE/ADAE server.

Figure 7.1.1-1 illustrates the procedure for AIMLE / ADAE server to support ML model inference based on the request from the VAL server.

Figure 7.1.1-1: ML model inference

1.	The VAL server sends an ML model inference request to AIMLE / ADAE server, requesting the inference result from the AIMLE / ADAE server. This request consists of ML model information or ML model inference requirement information, etc.

2.	The AIMLE / ADAE server checks whether the VAL server is authorized to perform the ML model inference request. If no model information is provided but only the model inference requirement information is provided in step 1, the AIMLE / ADAE server identifies and selects the appropriate ML model for inference based on the ML model inference requirement information using the procedure as specified in clause 8.11.3. The AIMLE / ADAE server handles the request and execute the inference task.

3.	If the VAL server is authorized, AIME server returns the inference response, otherwise a failure response indication the reason for failure.

### 7.1.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.1.3	Corresponding APIs

Table 7.1.3.1-1 describes the information flow from the VAL server to the AIMLE / ADAE server for requesting the ML model inference.

Table 7.1.3.1-1: ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Inference data (NOTE 1) | O | Original data for inference that may include text, images, audio, numerical data, etc. This IE may be included in the request in the scenario, e.g. the data is small. |
| Inference data index (NOTE 1) | O | An index of the data for inference. It is used to indicate where the ADAE function can get the original data for inference. This IE may be included in the request in the scenario, e.g. the data is big, or stored in different servers. |
| Inference Parameters | O | Additional inference parameters that may be needed, such as maximum output length, temperature, batch size, etc., depending on the specific requirements of the model. |
| ML model information (NOTE 2) | O | Identifies the ML model that VAL server specified to perform inference. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model inference requirement information (NOTE 2) | O | Identity the type or goal of the inference task, such as 'text classification,' 'image recognition,' 'sentiment analysis,' etc |
| NOTE 1: Only one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

Table 7.1.3.2-1 describes the information flow of ML model inference response from the AIMLE / ADAE server to the VAL server.

Table 7.1.3.2-2: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference response | O | Indicates that the ML model inference request was successful or not. |
| >Inference result (NOTE) | O | If the inference ML model inference request is successful, the Inference result is included in the response. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE / ADAE server for inference. |
| > Cause (NOTE) | M | If the inference ML model inference request is failed, the reason for the failure is included in the response. |
| NOTE: Only one of these information elements shall be present |  |  |

### 7.1.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection

### 7.2.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model (e.g. the distribution of the features have changed over time). If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift, that is data drift refers to changes in the statistical properties of data over time leading to a decline in model performance.

Therefore, it is essential that the data management system procedure provides the capability to detect whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance.

Data drift detection procedure allows to assist ML model maintenance procedures. To support ML model management, e.g. assist ML model maintenance, the AIMLE data management assistance service need be enhanced by adding drift detection functionality.

The following clauses specify the impact to existing AIMLE procedures for the enhancements of AIMLE data management assistance service.

#### 7.2.1.1	Impact to existing AIMLE procedures

The AIMLE procedures and information flows in 3GPP TS 23.482 can be enhanced (highlighted in bold italics) as follows.

8.15 AIMLE data management assistance 8.15.1 General AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation, and data analysis, and data drift detection. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation, and data analysis, and data drift detection operations and send the outputs to the AIMLE server for aggregation. The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance. 8.15.2 Procedure Pre-conditions: 1. AIMLE clients have registered with AIMLE server. 2. UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10]. Figure 8.15.2-1: AIMLE data management assistance 1. An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1. 2. The AIMLE server authenticates and authorizes the request. If authorized, the AIMILE server assigns an identifier for the subscription. 3. The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1. 4. The AIMLE server sends Client data processing trigger requests to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.15.3.4-1. 5. Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally. NOTE: The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training. 6. After the data operation completes, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.15.3.5-1. 7. The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations. 8. The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1. 8.15.3 Information flows 8.15.3.1 AIMLE data management assistance subscription request Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure. Table 8.15.3.1-1: AIMLE data management assistance subscription request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management operations; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE 1); Data preparation requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE 1); Data analysis requirements as detailed in Table 8.15.3.1-3.. > Data drift detection requirements; O (NOTE 1); Data drift detection requirements in Table 8.15.3.1-2.. AIMLE clients list; O (NOTE 2); A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier.. AIMLE client selection criteria; O (NOTE 2); Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided.. Table 8.15.3.1-2: Data preparation requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data preparation requirements; M; Requirements for data preparation.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature to process.. > Data preparation function; M; This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. Table 8.15.3.1-3: Data analysis requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Dataset analysis requirements; M; Requirements for data analysis.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data analysis function; M; This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. Table 8.15.3.1-x: Data drift detection requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data drift detection requirements; M; Requirements for data drift detection.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data drift detection function; O; This indicates the function which performs the drift detection, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. 8.15.3.3 AIMLE data management assistance notify Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure. Table 8.15.3.3-1: AIMLE data management assistance notify Information element; Status; Description. Status; M; The status for the data management operation. Aggregated data preparation outputs; O (NOTE 1, NOTE 2); Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output.. Aggregated data analysis outputs; O (NOTE 1, NOTE 2); Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information.. Aggregated data drift detection outputs; O (NOTE 1, NOTE 2); Provides outputs for data drift detection: dataset identifier, data drift detection status (e.g. detected data drift of the dataset features).. Timestamp; O; Timestamp of the data management notification. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal.. 8.15.3.4 Client data processing trigger request Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure. Table 8.15.3.4-1: Client data processing trigger request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management type; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-3.. > Data drift detection; O (NOTE); Data drift detection requirements as defined in Table 8.15.3.1-x.. Operational schedule; O; A schedule to perform the requested data management operation.. NOTE: At least one of the information elements shall be provided.. 8.15.3.5 Client data processing trigger response Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure. Table 8.15.3.5-1: Client data processing trigger response Information element; Status; Description. Status; M; The status for the data management operation. Data preparation outputs; O (NOTE 1); The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2). Data analysis outputs; O (NOTE 1); The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2). Data drift detection outputs; O; The output of data drift detection status (e.g. detected data drift of the dataset features). (NOTE 2). Timestamp; O; Timestamp of the data management operation. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data..

### 7.2.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.2.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.2.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.3	Solution #3: New AIMLE Service on ML Model Maintenance

### 7.3.1	Solution Description

It is important to continuously monitor the model's performance and check for any changes in the statistical nature of the data that is used for training the ML model or be used by the ML model for its task. In this scenario, ML model maintenance is needed to check e.g., whether the model’s performance aligns with expectations or data drift appears. Then, the AIMLE server may initiate model update process.

AIMLE service provides model maintenance service to the consumer (e.g. VAL server) based on request, e.g., to detect data drift, and initiate ML model update process. The information of data drift in specific dataset provided to the AIMLE server may be the data drift analysis output from the AIMLE client, and/or the data drift detection outputs from the VAL client via the AIMLE client.

The following clauses specify procedures, information flows and APIs for Key Issue #2 to support new AIMLE service on ML model maintenance.

#### 7.3.1.1	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

Figure 7.3.1.1-1: AIMLE support ML model maintenance procedure

1.	An AIMLE service consumer (e.g., VAL server) sends a ML model maintenance subscription request to the AIMLE server. The request includes information as described in Table 7.3.1.2.1-1.

2.	The AIMLE server authenticates and authorizes the request. If authorized, the AIMLE server assigns an identifier for the subscription.

3.	The AIMLE server sends an ML model maintenance subscription response to the AIMLE service consumer. The response includes information as described in Table 7.3.1.2.2-1.

4.	The AIMLE server performs operations for ML model performance evaluation to check the ML model performance. If the performance of the ML model is not as expected as request in step 1, the AIMLE server determines date drift detection. The ML model performance evaluation can use the ML model performance monitoring service as described in 3GPP TS 23.482 [2] clause 8.22 or use offline ML model evaluation procedure.

Editor's Note:	The detail procedure for offline ML model performance evaluation is FFS.

5.	The AIMLE server request and get client data processing trigger response as described in 3GPP TS 23.482 [2] clause 8.15 with data drift detection outputs from the AIMLE client, by using the enhanced AIMLE data management assistance service as described in Solution #2 steps 4-6 with enhanced information flows.

6.	Based on the data drift information received in step 5, the AIMLE server determines if actions are required (e.g. model replacement (e.g. select other ML model) or transfer learning (e.g. re-train the ML model based on the original ML model)) to keep the ML model performance within the performance requirements received in step 1.

7. The AIMLE server sends an ML model maintenance notification based on the notification requirements received in step 1 to the AIMLE service consumer with information as described in Table 7.3.1.2.3-1.

#### 7.3.1.2	Information flows

##### 7.3.1.2.1	ML model maintenance subscription request

Table 7.3.1.2.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the ML model maintenance subscription.

Table 7.3.1.2.1-1: ML model maintenance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| ML model ID | M | The identifier of the model that will be maintained. |
| ML model retrieval endpoint | O (NOTE 1) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| ML model | O (NOTE 1) | The ML model that will be maintained. |
| ML model maintenance requirements | M | Requirements for the ML model maintenance. |
| > ML model performance requirements | M | ML model performance requirements for the maintenance. |
| > Dataset ID | O (NOTE 2) | An identifier for the dataset, which is used for training the ML model, or be used by the ML model for its task. |
| > AIMLE clients list | O (NOTE 2) | A list of AIMLE clients from which the datasets for the ML model maintenance applied for. |
| > Maintenance mode | M | The maintenance mode, e.g., periodically or event triggered (e.g. ML model performance degradation detected, data drift detected). |
| Notification requirements | M | Requirements for notification, e.g., periodically or event triggered (e.g. ML model updated). Notify with e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Notification endpoint | O | The notification endpoint where the notifications should be sent to. |
| Expiration time | O | The proposed expiration time of the subscription. |
| NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided. |  |  |

##### 7.3.1.2.2	ML model maintenance subscription response

Table 7.3.1.2.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.2-1: ML model maintenance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model maintenance subscription request was successful. |
| > Subscription identifier | M | An identifier for the subscription. |
| > Expiration time | O | Expiration time for the subscription |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the information elements shall be present. |  |  |

##### 7.3.1.2.3	ML model maintenance notification

Table 7.3.1.2.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.3-1: ML model maintenance notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Model maintenance outputs | M | Provides outputs for model maintenance based on request: e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Timestamp | O | Timestamp of the data management notification. |

### 7.3.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.3.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.3.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.

### 7.4.1	Solution description

#### 7.4.1.1	Introduction

This solution proposes enhancements to address Key Issue #2 – Open Issue #2 and provides procedures required to enable evaluation of ML model performance and correctness considering application specific ML models.

This solution proposes to:

- define new procedures and information flows to enable management (e.g., storage, retrieval, removal) of ML model evaluation information in the ML model repository;

- update information flows to link the ML model evaluation information with ML model(s).

The solution allows the owner of ML model(s) to store and manage the evaluation information associated with ML model(s). The evaluation information is information that can be retrieved to perform the evaluation of performance and correctness of ML model(s). This solution can be combined with existing and/or new procedures when evaluation of a ML model performance and correctness is required.

ML models that can be stored and managed in the AIML enablement layer are application specific ML models. Such ML models can be retrieved by the AIMLE client or VAL server to perform inferencing at the VAL layer.

ML model specific evaluation information is required for a consumer of a ML model to perform evaluation of a ML model performance and correctness. Thus, this solution proposes to enable storage and retrieval of ML model evaluation information in the ML model repository for enabling the performance and correctness evaluation of application specific ML models.

#### 7.4.1.2	ML model evaluation information storage

Figure 7.4.1.2-1 describes procedures for storing ML model evaluation information in the ML model repository.

Figure 7.4.1.2-1: ML evaluation information storage

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information storage request to the AIMLE server. The request includes the requestor identifier and ML evaluation information.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information storage request to store the ML evaluation information in the ML repository. The request includes the requestor identifier and ML evaluation information.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository stores the ML evaluation information, creates an ML evaluation profile including a ML evaluation profile identifier and sends an ML evaluation information storage response to the AIMLE server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information storage response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

#### 7.4.1.3	ML model evaluation information retrieval

Figure 7.4.1.3-1 describes procedures for retrieving ML model evaluation information from the ML model repository.

Figure 7.4.1.3-1: ML evaluation information retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information retrieval request to the AIMLE server. The request includes the requestor identifier and ML evaluation information retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information retrieval request to obtain the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information retrieval filters.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository identifies ML evaluation profiles based on the ML evaluation information retrieval filters and sends a ML evaluation information retrieval response to the requestor. The response includes an indication of success for the request and the identified ML evaluation profiles if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information retrieval response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile(s) if the request was successful.

#### 7.4.1.4	ML model evaluation information removal

Figure 7.4.1.4-1 describes procedures for removal of ML model evaluation information from the ML model repository.

Figure 7.4.1.4-1: ML evaluation information removal

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information removal request to the AIMLE server. The request includes the requestor identifier and ML evaluation information identifier(s) to be removed.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information removal request to remove the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information identifier(s).

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository removes the ML evaluation information based on the ML evaluation information identifier(s) and sends a ML evaluation information removal response to the requestor. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information removal response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

### 7.4.2	Architecture Impacts

No architecture impact has been identified.

### 7.4.3	Corresponding APIs

#### 7.4.3.1	ML Evaluation Profile

Table 7.4.3.1-1 describes the ML evaluation profile and Table 7.4.3.1-2 describes the ML evaluation which is used in the ML evaluation information management procedures between the AIMLE client, VAL server, AIMLE server and ML model repository.

Table 7.4.3.1-1: ML Evaluation Profile

| Information element | Status | Description |
| --- | --- | --- |
| ML evaluation profile identifier | M | The identifier of the ML evaluation profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML evaluation information. |
| ML repository identifier | M | The identifier of the ML repository where the ML evaluation information is stored. |
| List of ML evaluations | M | The list of ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.1-2: ML Evaluation Information

| Information element | Status | Description |
| --- | --- | --- |
| Evaluation identifier | M | The identifier of the evaluation information used for evaluating ML model(s). |
| List of ML models identifiers | M | The list of ML models identifiers to which the evaluation applies. |
| Source identifier | O | The identifier of the source (e.g., VAL server ID, VAL client ID) that stored the ML evaluation information in the ML repository. |
| Description | M | The description of the ML model evaluation. |
| Metric information | M | The metrics measured by the performing the ML model evaluation. |
| Input information | M | Represents the input information which is aligned with the associated ML model input parameters. The format of input information for application specific ML models is out of scope. |
| Output information | M | Represents the expected output information which is expected as a result of processing the input information by the ML model. The format of output information for application specific ML models is out of scope. |
| Grading information | M | The information for evaluating and grading the performance and correctness of a ML model (e.g., grading the obtained evaluation output based on the expected output). |
| Execution information | O | The execution information which is required to evaluate performance and correctness of a ML model. The information indicates whether the evaluation is executed once, repeatedly in a time window, has its output averaged, etc. If not included, the evaluation is evaluated and graded once. |

#### 7.4.3.2	ML Evaluation Information Storage

Table 7.4.3.2-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for storing ML evaluation information in the ML repository.

Table 7.4.3.2-1: ML Evaluation Information Storage Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation information | M | The list of ML evaluation information to be stored in the ML repository as described in Table 7.4.3.1-2. |

Table 7.4.3.2-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information storage request.

Table 7.4.3.2-2: ML Evaluation Information Storage Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > ML evaluation profile identifier | M | The ML evaluation profile identifier stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.3	ML Evaluation Information Retrieval

Table 7.4.3.3-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for retrieving ML evaluation information from the ML repository.

Table 7.4.3.3-1: ML Evaluation Information Retrieval Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Evaluation information filters | M | The evaluation information filter used to identify the retrieved ML evaluation information. |
| > Evaluation profile identifier | O | Represent the filtering criteria which is a ML evaluation profile identifier. |
| > Evaluation information filter | O | Represents the filtering criteria, which can be any of the ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.3-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information retrieval request.

Table 7.4.3.3-2: ML Evaluation Information Retrieval Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profiles | M | The list of retrieved ML evaluation profiles stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.4	ML Evaluation Information Removal

Table 7.4.3.4-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for removing ML evaluation information from the ML repository.

Table 7.4.3.4-1: ML Evaluation Information Removal Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation profile identifiers | M | The list of evaluation profile identifiers to be removed from the ML model repository. |

Table 7.4.3.4-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information removal request.

Table 7.4.3.4-2: ML Evaluation Information Removal Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profile identifiers | M | The list of ML evaluation profile identifiers removed from the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.4.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

## 7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness

### 7.5.1	Solution description

This solution addresses Key Issue #2 and open issue 3.

In this solution, the ADAES is providing the analytics performance monitoring where the AIMLE server detects an expected or predicted model degradation performance based on the ML-enabled analytics performance feedback. Such ADAE analytics (as captured in 3GPP TS 23.436) can be about the edge load or VAL server or session performance analytics which utilizes ML methods (training by AIMLE) for predicting the end-to-end performance.

Pre-conditions:

- One or more ADAE services (at the ADAE server or clients) using the given ML model training service from AIMLE server are ongoing.

- The AIMLE server (or VAL server) detects a requirement to monitor the analytics performance. Such trigger may be based on VAL server request for the ML model performance monitoring capability as specified in 3GPP TS 23.482 clause 8.22.2.

Figure 7.5.1-1: Procedure for ML-enabled analytics monitoring

1.	The AIMLE server sends an ADAE analytics performance monitoring subscription request to the ADAE server (ADAES). The subscription request includes the following parameters: an ADAE analytics ID or type, a VAL server ID, an area of interest and time of validity, a monitoring reporting configuration (thresholds for triggering a monitoring event, e.g. minimum accuracy, confidence level, whether the reporting is one time or periodical or event-based), a KPI for the analytics service, VAL policies for triggering an action based on a monitoring event (e.g. if analytics degradation is detected, re-start the analytics process).

2.	The ADAE server authorizes the subscription request and sends an ADAE analytics performance monitoring subscription response to the AIMLE server. The response is a positive or negative acknowledgement of the subscription and includes a subscription ID in case of success and a cause in case of failure (e.g. failure to authorize).

3.	The ADAE server discovers based on the analytics ID the ADAECs or other entities (e.g. edge ADAES) which are undertaking an analytics task for the given analytics ID (e.g. local prediction at UE side or ADAES side).

4.	The ADAES server sends a request to the discovered entities (ADAEC or edge ADAES) indicating a monitoring requirement for the analytics task. This request includes the analytics ID, the requestor ID, the event trigger criteria for providing a monitoring report (e.g. reaching a threshold), the target KPIs to be monitored (e.g. accuracy), the area and time of interest, whether this is a predicted parameter or actual parameter to be monitored. Such request can be in form of an analytics correctness feedback request either real time or when the analytics task finishes.

5. The discovered ADAEC(s) and/or edge ADAES start monitoring the performance of the analytics task and in particular the attributes which are requested (accuracy, delay, evaluation vs ground truth data). This involves the continuous checking of the performance (accuracy, latency) of the analytics service for providing a notification / trigger event to the AIMLE server identifying a deviation in terms of ADAE analytics performance. Such identification can be a detection of a mismatch in the analytics performance with a possible cause (e.g. data producer, ML model, UE/ channel conditions).

6. The discovered ADAEC(s) and/or edge ADAES based on the trigger criteria, send to the ADAES a monitoring response, indicating an expected or predicted degradation of the performance of the analytics task. Such message may also include the possible cause as in step 6 (e.g. data producer, ML model, UE/ channel conditions).

7. The ADAES identifies or predicts an ADAE service degradation or performance (against the KPIs for that analytics ID) and sends a notification to the AIMLE server detecting a degradation / mismatch at the target performance for the analytics task.

8. The AIMLE server based on the received monitoring notification from the one or more AIMLE clients, it identifies an expected or predicted ML model degradation using the AIMLE capability for ML model performance monitoring as specified in 3GPP TS 23.482 clause 8.22.2 and can optionally use the ADAE capability for ML model degradation detection in clause 8.19.2.

### 7.5.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.5.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.5.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.6	Solution #6: Enhancements to performance monitoring

### 7.6.1	Solution description

#### 7.6.1.1	Enhancements to ML model performance monitoring

The ML model performance monitoring procedure as specified in clause 8.22 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

### 8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. The request also includes baseline validation dataset information which is the baseline reference for evaluation of HFL model after each round of parameter aggregation. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. In order to get the information, the request is enhanced with following IEs: Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Aggregated AI/ML model parameters; M; Information about the AI/ML model and model parameters for use in HFL training.. Performance Metrics; M; The performance metrics that are used to evaluate the performance of model (e.g. accuracy, latency). each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters and Performance metrics. Each AIMLE client sends the performance report of aggregated global model on the local data as the response message to AILMLE Server. The performance report response message includes information as described in the following table. The response is enhanced with following IEss: Information element; Status; Description. AIMLE Client ID; M; The identifier of the AIMLE Client.. AIMLE service ID; M; The service identifier for the AIMLE HFL training operation.. Performance records; M; The evaluation performance records (e.g. on accuracy, on latency) of received aggregated global model from AIMLE Server on local data.. 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service as indicated in step 4 . 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Further, AIMLE Server sends a configuration request to increase training collaboration to only AIMLE Clients that have degraded performance records. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service.

#### 7.6.1.2	Enhancements to HFL training

As per step 4 of Clause 8.22 of 3GPP TS 23.482 [2], for performance monitoring of AIMLE clients clause 8.12.2 is reused, quote:

“the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service.”

The current solution proposes to enhance clause 8.12 as follows (new text in bold italics):

### 8.12.2 Procedure Pre-conditions: 1. AIMLE client discovery and selection have been performed. 2. Datasets are available and prepared for AI/ML training at the AIMLE clients. The datasets are assigned identifiers for use in HFL training. Figure 8.12.2-1: HFL training 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2. 2. The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1. 3. For the HFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training. 4. Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the HFL training as described in the clause 8.10. 5. The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1. The subscription request is further enhanced to include following: Information element; Status; Description. AI/ML model information; M; Information about the AI/ML model (e.g. Model size, Model architecture, etc.). Current HFL training iteration; O; The number of aggregations that have been done during HFL training. Performance Tracker; O; The KPI records (i.e. performance metrics, such as accuracy, etc.) over different subscriptions rounds.. Editor's Note: Whether performance tracker IE is needed or not is FFS. The Current HFL training iteration informs the client about current training iteration ongoing. 6. Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 8. 7. Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule. 8. Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an VAL service ID for the HFL training operation. 9. If errors were encountered, the procedure skips to step 10. If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 8 are repeated for the next training round. When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository. 10. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

### 7.6.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.6.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.6.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios

### 7.7.1	Solution description

This solution addresses key issue #3 open issues 1), 3), 4).

According to 3GPP TS 23.434, one of the business models is related to the SEAL provider having PLMN operator service arrangements with multiple home PLMN operators. Additionally, according to [AR-4.2.2-b], the SEAL shall support deployments in which SEAL services are deployed only outside of PLMN network.

This solution corresponds to the following use case:

- An AI/ML service runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator AIMLE/VAL service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN. The AI/ML service can be an application enablement service (AIMLE client to server) where AIMLE-UU can be over multiple MNO networks.

- In Figure 7.7.1-1, an example is shown where UE#1 is connected to PLMN#1 and UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. Different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

- One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks.

Figure 7.7.1-1 use case for multi-operator AIMLE scenario

The solution provides a mechanism for discovering AI/ML participants (AIMLE clients, VAL clients) in an AIMLE-assisted operation (e.g. ML model training, FL, TL) in a given AIMLE service area. Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

In cross-PLMN scenario as in Figure 7.7.1-2, the following enhancements are provided (over AIMLE-S or SEAL-X interface if the AIMLE consumer is another SEAL server, e.g. ADAES).

8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

This request also includes information on the serving and/or preferred PLMN for the communication of the VAL service, as well as the allowed MNO info from other MNO network from which the AIMLE consumer / VAL server can discover AIMLE clients / VAL UEs.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients for the VAL UEs connected to the serving PLMN. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs connected to the serving PLMN from the ML repository.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on supported services and AIMLE client profiles.

The AIMLE server determines whether the discovered UEs fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those UEs that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1.

If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML service consumer may discover the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients. The response may also include information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML) or the AIMLE clients are not available or capable for undertaking the operation, then the AIMLE server determines to discover and discovers the remaining required AIMLE clients connected to other PLMNs based on the allowed and/or preferred MNO information and the permissions as in step 1. The AIMLE server also obtains candidate UEs connected to the target/destination PLMN (other than the serving PLMN) from the ML repository.

Editor's Note: Enhancements on the information flows related to AIMLE client discovery procedure for cross-PLMN scenario is FFS

### 7.7.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.7.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.7.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.8 Solution #8: Multi-Client Split AI/ML Operations Pipeline

### 7.8.1	Solution Description

#### 7.8.1.1	General

This solution addresses key issues related to split AI/ML operations involving multiple AIMLE clients. The proposed solution enhances split AI/ML operations by facilitating multiple AIMLE client collaboration within the same pipeline. It allows the creation of multi-client AI/ML split operation pipeline when such a pipeline is not yet created and allows joining the pipeline when such a pipeline has already been created. The solution also keeps AIMLE clients in sync with the latest status of the pipeline through notifications as more AIMLE clients join and the pipeline is adjusted according to the resources of the newly joining AIMLE clients. The solution ensures efficient resource utilization and continuous service, even with mobility and varying device constraints.

Editor's Note: The feasibility of sharing a pipeline among AIMLE clients is FFS

Use Cases:

- Enhanced Smart City Surveillance with Mobility: Surveillance cameras, both stationary and mobile, capture video footage and process raw data on the cameras, sending subsequent layers to an edge server for immediate processing. AIML split operations enable real-time improved surveillance and threat detection for public safety through a shared pipeline that allows collaborative data processing. The pipeline needs to be re-adjusted based on computing resource powers for the surveillance cameras. Pipeline re-adjustment is also needed as the mobile surveillance cameras change service areas. This results in efficient resource utilization, energy saving, and larger surveillance.

- Collaborative Autonomous Driving: A group of autonomous vehicles collaborate by sharing and processing data together. A more concreate example is vehicles utilizing shared Local Dynamic Maps (LDM) which is collaboratively updated and used by each vehicle locally. The raw data of each vehicle is processed locally and it is sent for subsequent layers in the edge server for immediate processing. The shared AIML split operation enable real-time collaboration to optimize driving conditions (e.g., efficient routes) and reduces computational burden in the vehicles. Pipeline re-adjustment is needed based on the computing resource power of the participants. Pipeline-readjustment is also needed as the vehicles change service area during mobility. This results in a larger collaboration coverage.

#### 7.8.1.2	Multi-client pipeline operation

Figure 7.8.1.2-1: Multi-client AI/ML split operation

The solution involves a multi-client split operations pipeline facilitated by AIMLE clients and servers. Initially, an AIMLE client initiates a discovery request to the AIMLE server to participate in a split operation pipeline, as illustrated in Block A of the figure. This is similar to clause 8.14.2.2 in 3GPP TS 23.482 split operation pipeline discovery. A key addition for this solution is an expansion of the split operation discovery filters in clause 8.14.3.2-2 with additional criteria for multi-client split operations within the pipeline. Following the discovery request, the split operation profile in the discovery response, as in clause 8.14.3.3 Table 8.14.3.3-2, is also expanded to include information about a pipeline with multi-client split operation pipeline support, the usage of the multi-client split operation pipeline (in usage information), and whether such a pipeline has already been created or not.

If in the discovery stage, as in Block B, the discovered instance of the split operation pipeline is a pipeline that has not yet been created, the AIMLE client will then request the creation of a new pipeline, explicitly mentioning the option for multi-client operations in the split operation profile, as shown in Block B, and subscribes for the split operation event. This is similar to clause 8.14.2.3 in 3GPP TS 23.482 and clause 8.14.2.5 in 3GPP TS 23.482, with additional expansion of clause 8.14.3.4-1 in 3GPP TS 23.482 to include multi-client support in the pipeline, expansion of the usage information to include further details on the multi-client pipeline, and also the computing resources of the AIMLE client to be able to efficiently adjust the pipeline later on as more AIMLE clients join the pipeline. Furthermore, instead of 3GPP TS 23.482, which has two separate procedures for pipeline creation and subscription, this solution proposes a split operation pipeline create subscription, which will allow creation and subscription together. Although this is a useful procedure for a single AIMLE client case as well, it becomes more important for split AI/ML operations involving multiple AIMLE clients. This is because as other AIMLE clients join or leave and the pipeline is modified, notifications need to be sent to the AIMLE clients so that proper client-side adjustments are made as needed.

Editor's Note: The procedure of split operation pipeline create subscription and notification is FFS to be detailed in clause 7.8.2.

Subsequently, once a multi-client AI/ML split operation pipeline has been created, other AIMLE clients who seek to also engage in the multi-client AI/ML split operation pipeline begin by initiating a discovery procedure as in Block C, which is similar to Block A. If a pipeline is already created according to Block B, it identifies the existing created pipeline that supports multi-client AI/ML split operation, fulfilling the respective split operation discovery filters. Therefore, instead of a split operation pipeline create subscription, it will proceed with a split operation pipeline join subscription, which will allow the AIMLE client to join the created pipeline as in Block D. To enable adjustment of the pipeline considering the newly joining AIMLE clients, each client indicates its respective computing resources during the join. This process can be scaled up to accommodate an n-number of AIMLE clients, allowing for extensive multi-client collaboration in split AI/ML operations.

Editor's Note: The procedure of split operation pipeline join subscription and notification is FFS to be detailed in clause 7.8.2.

As AIMLE clients with different computing resources join the split operation pipeline, the pipeline may need to be adjusted, and the participating AIMLE clients need to be notified of changes to the pipeline so that they can make client-side adjustments. For this purpose, as shown in Block E, a split AI/ML operation notification is sent to all AIMLE clients as per clause 8.14.2.5.3 of 3GPP TS 23.482.

#### 7.8.1.3	Multi-client pipeline service continuity

A multi-client AI/ML split operation pipeline should be resilient to the mobility of the AIMLE clients. When edges are involved in the split operation pipeline, not only should the pipeline be adjusted, but AIMLE clients should also be enabled to collaborate in the pipeline even if they are in different service areas. This will enable better service continuity support. When an AIMLE client leaves a pipeline, the pipeline also needs to be adjusted to stay optimal.

This solution achieves service continuity in two ways:

1.	First, the pipeline is re-adjusted according to the computing resources of the AIMLE clients to obtain matching layers in the pipeline. For example, when AIMLE clients with low computing resources join, the pipeline is adjusted so that more parts of the pipeline are offloaded to the nodes. Conversely, when an AIMLE client with low resources leaves, the pipeline can be re-adjusted so that more parts of the pipeline can be handled by the remaining AIMLE clients.

2.	Second, when AIMLE clients change service areas, the pipeline can still be adjusted so that the common layers in the pipeline remain matched. When an AIMLE client moves to a different service area, adjustments are made based on the available AIMLE clients in the current service area. When the AIMLE client moves back to the original service area, re-adjustment is performed to keep the common layers in the pipeline matched. For example, when an AIMLE client with low computing resources moves to a different service area, part of the pipeline in the current service area can be relaxed for the other AIMLE clients so that more of the pipeline can be on the client side. The edges in the service area of the low-resource AIMLE clients can be part of the pipeline in line with the computing resources of the AIMLE clients in the service area to which it moved. The source and target AIMLE servers can continue to communicate as in step 1 of 8.24.2 TS 23.482.

### 7.8.2	Architecture Impacts

Editor's Note: The new and updated procedures corresponding to this solution is FFS.

### 7.8.3	Corresponding APIs

Editor's Note: The corresponding APIs are FFS.

### 7.8.4	Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

## 7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment

### 7.9.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes AIMLE server registration procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 as shown on Figure 7.9.1-1. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

Figure 7.9.1-1: Example hierarchical AIMLE deployment

The registration of an edge AIMLE server with a central AIMLE server enables the central AIMLE server to be aware of the edge AIMLE server capabilities and of the clients registered with the edge AIMLE server.

A central AIMLE server needs information about AIMLE clients available in different EDN service areas when performing operations requiring the selection of AIMLE clients. The AIMLE clients in an EDN service area may register to the edge AIMLE server, therefore, registration of the edge AIMLE server with the central AIMLE server are needed to support hierarchical AIMLE deployment scenarios.

#### 7.9.1.1 AIMLE server registration

Figure 7.9.1.1-1 describes a procedure for an AIMLE server to register with another AIMLE server. In such a scenario, an AIMLE server deployed in an EDN registers to a central AIMLE server.

Pre-conditions:

1.	AIMLE clients have registered with an edge deployed AIMLE server as described in clause 8.7.2.2 of 3GPP TS 23.482. The edge deployed AIMLE server receives AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE server has been pre-configured with endpoint information of a central AIMLE server.

Figure 7.9.1.1-1: AIMLE server registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration request to a central AIMLE server. The registration request includes the requestor identifier, security credentials, endpoint information and a list of AIMLE clients registered with the AIMLE server. The request may include EDN information and AIMLE server capabilities.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server stores the registration information and creates a registration identifier. The central AIMLE server sends a AIMLE server registration response to the requestor. If the central AIMLE server has registered the requestor, the response includes an indication of success, a registration identifier and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

#### 7.9.1.2 AIMLE server registration update

Pre-conditions:

- The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.2-1: AIMLE server registration update procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration update request to the central AIMLE server. The request includes the registration identifier, security credentials and may include updated AIMLE server information as described in Table 7.9.3.3-1. If the request is to renew the registration, only the registration identifier is included in the request.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server updates the information associated with the registration identifier and may update the registration expiration time if the request only includes the registration identifier. The central AIMLE server sends a AIMLE server registration update response to the requestor. If the central AIMLE server has updates the registration, the response includes an indication of success and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

#### 7.9.1.3 AIMLE server de-registration

Pre-conditions:

- The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.3-1: AIMLE server de-registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server de-registration request to the central AIMLE server. The de-registration request includes the registration identifier and security credentials.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server terminates the registration indicated in the request. The central AIMLE server sends a AIMLE server de-registration response to the requestor. If the central AIMLE server has de-registered the requestor, the response includes an indication of success. If the requestor was not de-registered, the response includes an indication of failure and may include a reason for failure.

### 7.9.2	Architecture impacts

No architectural impact has been identified.

### 7.9.3	Corresponding APIs

#### 7.9.3.1	AIMLE server registration request

Table 7.9.3.1-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for registering AIMLE server and clients in the EDN service area.

Table 7.9.3.1-1: AIMLE server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the registering AIMLE server. |
| AIMLE server endpoint | M | The endpoint information (e.g., URL, URI, IP address, port) of the registering AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registering AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O (NOTE) | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O (NOTE) | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registering AIMLE server. |
| List of registered AIMLE clients | M | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registering AIMLE server. |
| NOTE: At least one of the information elements is present. |  |  |

#### 7.9.3.2	AIMLE server registration response

Table 7.9.3.2-1 shows the response sent by the central AIMLE server to the edge deployed AIMLE server in response to the registration request.

Table 7.9.3.2-1: AIMLE server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the registration. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration failed. |
| > Cause | O | Indicates the cause of the failure |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.3	AIMLE server registration update request

Table 7.9.3.3-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server registration update request.

Table 7.9.3.3-1: AIMLE server registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of an existing registration. |
| AIMLE server endpoint | O | The endpoint information (e.g., URL, URI, IP address, port) of the registered AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registered AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O | The identifier of the EDN service area associated with the registered AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registered AIMLE server. |
| List of registered AIMLE clients | O | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registered AIMLE server. |

#### 7.9.3.4	AIMLE server registration update response

Table 7.9.3.4-1 shows the AIMLE server registration update response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.4-1: AIMLE server registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update failed. |
| > Cause | O | Indicates the cause the failure. |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.5	AIMLE server de-registration request

Table 7.9.3.5-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server de-registration request.

Table 7.9.3.5-1: AIMLE server de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the existing registration. |

#### 7.9.3.6	AIMLE server de-registration response

Table 7.9.3.6-1 shows the AIMLE server de-registration response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.6-1: AIMLE server de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration failed. |
| > Cause | O | Indicates the cause of the failure. |
| NOTE: One of the information elements must be present. |  |  |

### 7.9.4 Solution evaluation

## Editor's Note:		The solution evaluation is FFS.

## 7.x	Solution #x: <title>

Provide a suitable title for the solution.

### 7.x.1	Solution description

This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

### 7.x.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 8	Deployment scenarios

## 8.1	General

This clause will provide a general description of the deployment scenarios.

## 8.x	Deployment model #x: <Title>

Provide a description of the deployment scenarios.

# 9	Business Relationships

Provide a description of the involved business relationships.

# 10	Overall evaluation

This clause will provide evaluation of different solutions.

# 11	Conclusions

## 11.1	General conclusions

This clause will provide general conclusions for the study.

## 11.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 |  |  |  |  | TS skeleton | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-251333, S6-251334, S6-251443, S6-251459, S6-251480, S6-251481, S6-251482, S6-251491 | 0.1.0 |
| 2025-05 | SA6#67 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-252512, S6-252427, S6-252494, S6-252509, S6-252430, S6-252495, S6-252480, S6-252536, S6-252434, S6-252537, S6-252437, S6-252520,S6-252438, S6-252439 | 0.2.0 |