| 3GPP TR 23.700-95 V0.4.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement aspects for subscriber-aware northbound API access; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Key issues	7

4.1	Key Issue #1: UE-originated API invocation	7

4.1.1	Description	7

4.2	Key Issue #2: AF-originated API invocation	8

4.2.1	Description	8

4.3	Key Issue #3: Providing and revoking user consent upon invoking APIs	8

4.3.1	Description	8

4.4	Key Issue #4: Discovery of target API information	9

4.4.1	Description	9

5	Architectural requirements	9

5.1	General requirements	9

6	Solutions	9

6.0	Mapping of Solutions to Key Issues	9

6.1	Solution #1: Business relationship in SNA	10

6.1.1	Solution description	10

6.1.2	Solution evaluation	10

6.2	Solution #2: Functional model description for SNA	10

6.2.1	Solution description	10

6.2.2	Solution evaluation	11

6.3	Solution #3: Obtaining user consent via CAPIF-8	11

6.3.1	Solution description	11

6.3.1.1	Resource owner registration	11

6.3.1.2	Obtaining user consent	12

6.3.1.3	Updating user consent	14

6.3.2	Solution evaluation	14

6.X	Solution #X: <Title>	14

6.X.1	Solution description	14

6.X.2	Solution evaluation	14

7	Overall evaluation	14

8	Conclusions	14

Annex A (informative): Change history	15

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document studies the potential enhancements in CAPIF and application enablement frameworks (e.g. SEAL, EDGEAPP, vertical enabler layers) to support the subscriber-aware northbound API access (SNA), whose requirements are specified in TS 22.261 [2].

This study takes into consideration the existing works for CAPIF (3GPP TS 23.222 [3]), SEAL (3GPP TS 23.434 [4]), and EDGEAPP (3GPP TS 23.558 [5]).

Any potential enhancements to security procedures related to CAPIF and application enablement frameworks (e.g. EDGEAPP, SEAL, vertical enabler layers) to support SNA are not in the scope of this study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1"

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows"

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[6]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping"

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

resource owner: An entity capable of granting access to a protected resource.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

AF	Application Function

CAPIF	Common API Framework

CCF	CAPIF Core Function

SNA	Subscriber-aware northbound API access

# 4	Key issues

## 4.1	Key Issue #1: UE-originated API invocation

### 4.1.1	Description

The UE-originated API invocation scenario corresponds to the following requirement specified in TS 22.261 [2]:

-	The 5G system shall be able to provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE.

In this scenario, the application on the UE invokes the northbound APIs. The scenario is illustrated in figure 4.1.1-1.

From CAPIF point of view, the application on the UE in this scenario plays the role of the API invoker, as defined in TS 23.222 [3].

Figure 4.1.1-1: UE-originated API invocation

Open issues:

1)	Whether the existing CAPIF functional model is sufficient for the UE-originated API invocation.

a)	If the existing CAPIF functional model is insufficient, how it must be modified to accommodate for the API invoker in the UE.

## 4.2	Key Issue #2: AF-originated API invocation

### 4.2.1	Description

In the AF-originated API invocation scenario, the AF invokes the northbound APIs, and the application on the UE consumes the service from the AF. The scenario is illustrated in figure 4.2.1-1.

Figure 4.2.1-1: AF-originated API invocation

Open issues:

1)	Whether the existing CAPIF functional model is sufficient for the AF-originated API invocation.

a)	If the existing CAPIF functional model is insufficient, how it should be updated.

2)	Study the business relationship between the user (UE), the AF and the northbound API provider.

## 4.3	Key Issue #3: Providing and revoking user consent upon invoking APIs

### 4.3.1	Description

Invocation of some northbound APIs may require obtaining user consent (e.g. API to retrieve the location of the UE).

In the SNA context, it is for further study how to obtain user consent when the API invocation is related to the resource owner (e.g. the user).

In addition, the system should ensure that there are mechanisms for the resource owner to revoke the user consent provided previously for the API invocation.

Open issues:

1)	Whether and how CAPIF functions can determine the resource owner upon API invocation.

2)	Whether and how CAPIF can support obtaining user consent from resource owner.

3)	Whether and how CAPIF can support revoking user consent by the resource owner.

NOTE:	This document studies the high-level architecture to solve this issue. The detailed procedure for providing the user consent will be specified in SA3.

## 4.4	Key Issue #4: Discovery of target API information

### 4.4.1	Description

As per the below requirement from 3GPP TS 22.261 [2] (clause 6.10.2), the third party needs to be provided with information to locate the target APIs in the network.

-	provide a third-party with information to identify networks and APIs on those networks.

An AF may serve UEs that belong to different PLMNs. When the AF is not aware of the PLMN provider of the UEs and has to discover the service API in relation to such UEs, then it is not clear how to discover the service APIs. How the AF can determine the right AEF instance, which is exposing the required Northbound API for which the AF wants to consume that Northbound API with the UE(s) information.

Open Issue:

-	Whether and how to support discovery of the target Service API information in CAPIF with respect to UE(s) information, especially when:

-	AF is serving UEs belonging to multiple PLMNs; and

-	There are multiple instances of the AEF within a PLMN.

# 5	Architectural requirements

## 5.1	General requirements

[AR-5.1-a] The confidentiality of the UE's external identity (e.g. MSISDN) shall be preserved during the API invocation supported by CAPIF.

[AR-5.1-b] The CAPIF shall support the authentication of the resource owner.

[AR-5.1-c] The CAPIF shall enable the resource owner(s) to provide and revoke user consent.

# 6	Solutions

Editor's Note:	This clause will provide descriptions for the different solutions.

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key issue 1 | Key issue 2 | Key issue 3 | Key issue 4 | … |
| --- | --- | --- | --- | --- | --- |
| Solution 1 | X | X |  |  |  |
| Solution 2 |  |  | X |  |  |
| Solution 3 |  |  | X |  |  |

## 6.1	Solution #1: Business relationship in SNA

### 6.1.1	Solution description

This solution addresses the key issue #2 with regard to the business relationship between the user (UE), the AF and the northbound API provider in the AF-originated API invocation scenario. Considering the business relationship, the resource owner (which is a UE-side entity) is a new entity that has not been in the existing CAPIF business relationship, thus the business relationship should be updated to include the resource owner.

Figure 6.1.1-1 shows the typical business relationship in SNA. This business relationship can be applied to both AF- and UE-originated API invocation scenario, as the API invoker in figure 6.1.1-1 can either be an application on the UE or the AF.

Figure 6.1.1-1: Business relationships in SNA

The API invoker has service agreement with a CAPIF provider, and the API provider provides APIs associated with the resource owner. The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), as described in 3GPP TS 23.222 [3] clause 5. When the CAPIF provider is a PLMN operator, the resource owner may be a subscriber of the PLMN.

### 6.1.2	Solution evaluation

This solution enhances the existing CAPIF business relationship by introducing the resource owner, which is viable.

## 6.2	Solution #2: Functional model description for SNA

### 6.2.1	Solution description

This solution addresses the key issue #3 with regard to obtaining and revoking user consent from the resource owner. As the resource owner is a new entity for CAPIF, the functional model for the CAPIF should be updated including the resource owner. Figure 6.2.1-1 shows the reference point based functional model for the CAPIF with enhancements to support SNA.

Figure 6.2.1-1: Functional model for the CAPIF

The resource owner resides within the PLMN trust domain and interacts with the API exposing function via CAPIF-8. The resource owner communicates with the API exposing function to provide and revoke user consent.

NOTE:	Security aspects including specification of the authorisation procedures are to be decided in SA3.

### 6.2.2	Solution evaluation

This solution enhances the existing CAPIF functional model by introducing the resource owner, which is viable.

## 6.3	Solution #3: Obtaining user consent via CAPIF-8

### 6.3.1	Solution description

This solution addresses the key issue #3 with regard to obtaining user consent. When the CAPIF obtain user consent upon service API invocation, the API exposing function informs the resource owner that the user consent is needed and obtains the user consent response. For the resource owner and the API exposing function interaction, the resource owner registration may optionally be performed prior to the service API invocation.

Every API exposing function deployed by the service provider does not require to support the resource owner registration. It is up to the service provider to enable this functionality.

Clause 6.3.1.1 shows the procedure for resource owner registration and clause 6.3.1.2 shows the procedure for obtaining user consent.

#### 6.3.1.1	Resource owner registration

Figure 6.3.1.1-1 shows the procedure for resource owner registration.

Editor's Note:	Which CAPIF entity (e.g. CAPIF core function, API exposing function) acts as the resource owner registration handling function in the procedure below is FFS.

Pre-conditions:

1.	The resource owner is authorised to access the resource owner registration handling function; and

2.	The resource owner has information for accessing the resource owner registration handling function.

Figure 6.3.1.1-1: Procedure for resource owner registration

1.	The resource owner sends resource owner registration request to the resource owner registration handling function, including the resource owner identity (e.g. GPSI) and security credentials.

2.	The resource owner registration handling function initiates the verification procedure and validates the resource owner registration request.

3.	The resource owner registration handling function sends resource owner registration response.

#### 6.3.1.2	Obtaining user consent

Figure 6.3.1.2-1 shows the procedure to obtain user consent upon service API invocation.

Pre-conditions:

1.	The resource owner is registered to the API exposing function and can be accessed by the API exposing function.

Figure 6.3.1.2-1: Procedure for obtaining user consent

1.	The API invoker sends service API invocation request to the API exposing function, which requires user consent of the resource owner.

2.	The API exposing function determine if the user consent is required to execute the service API. If the user consent is not required for the API invocation, the steps 3-5 will be skipped.

NOTE:	The API exposing function determines if the user consent is required based on implemented API logic.

The API exposing function identifies the resource owner by one of the following processes:

-	If the service API invocation indicates the resource owner by GPSI, the API exposing function identifies the resource owner by retrieving the resource owner information from the resource owner registration handling function, which has received the resource owner information in resource owner registration, as shown in clause 6.3.1.1.

-	If the service API invocation indicates the resource owner by UE (IP or Ethernet) address, the API exposing function identifies the resource owner by interacting with BSF, as specified in TS 29.513 [6].

-	If the service API invocation indicates the resource owner by external group identifier, the API exposing function identifies the individual resource owners by interacting with HSS/UDM, as specified in TS 23.682 [7].

-	If the service API invocation indicates the resource owner by "any UE" identification, the API exposing function regards all the available resource owners as the target resource owners.

3.	The API exposing function obtains the user consent from the resource owner.

NOTE:	The detailed procedure to obtain the user consent will be specified in SA3.

4.	If the API invocation is allowed, the process for service API execution is continued. If the API invocation is denied, the service API execution is rejected. The result of the user consent response is stored in the API exposing function.

NOTE:	How the stored user consent is maintained or cleared is up to implementation and is out of scope of this document.

5.	The API exposing function sends service API invocation response to the API invoker based on the result of the service API execution.

#### 6.3.1.3	Updating user consent

The resource owner may update user consent anytime to allow or deny future API invocation.

NOTE:	The detailed procedure to update the user consent will be specified in SA3.

### 6.3.2	Solution evaluation

Editor's Note:	This clause will provide the evaluation for the solution.

## 6.X	Solution #X: <Title>

### 6.X.1	Solution description

### 6.X.2	Solution evaluation

# 7	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the study item activities.

# Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211548 |  |  |  | TR skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | S6-211821, S6-211852, S6-211853, S6-211854, S6-211855, S6-211856 | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | S6-211871, S6-212075, S6-212076, S6-212165 | 0.2.0 |
| 2021-10 | SA6#45-bis-e |  |  |  |  | S6-212242, S6-212475 | 0.3.0 |
| 2021-11 | SA6#46-e |  |  |  |  | S6-212809, S6-212810, S6-212811, S6-212812, S6-212826 | 0.4.0 |