| 3GPP TR 23.700-95 V1.1.0 (2022-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement aspects for subscriber-aware northbound API access; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Key issues	8

4.1	Key Issue #1: UE-originated API invocation	8

4.1.1	Description	8

4.2	Key Issue #2: AF-originated API invocation	9

4.2.1	Description	9

4.3	Key Issue #3: Providing and revoking user consent upon invoking APIs	9

4.3.1	Description	9

4.4	Key Issue #4: Discovery of target API information	10

4.4.1	Description	10

5	Architectural requirements	10

5.1	General requirements	10

6	Solutions	10

6.0	Mapping of Solutions to Key Issues	10

6.1	Solution #1: Business relationship in SNA	11

6.1.1	Solution description	11

6.1.2	Solution evaluation	11

6.2	Solution #2: Functional model description for SNA	11

6.2.1	Solution description	11

6.2.1.1	General	11

6.2.1.2	Functional model	12

6.2.1.3	Deployment models	12

6.2.2	Solution evaluation	12

6.3	Solution #3: Obtaining resource owner consent upon service API invocation	13

6.3.1	Solution description	13

6.3.1.1	General	13

6.3.1.2	Resource owner registration	13

6.3.1.3	Obtaining resource owner consent	13

6.3.1.4	Updating resource owner consent	15

6.3.2	Solution evaluation	15

6.4	Solution #4: API invoker obtaining resource owner consent	15

6.4.1	Solution description	15

6.4.1.1	API invoker obtaining resource owner consent prior to the service API invocation	15

6.4.2	Solution evaluation	16

6.5	Solution #5: UE-originated API invocation within CAPIF	16

6.5.1	Description	16

6.5.2	Solution evaluation	17

6.6	Solution #6: Discover a proper AEF with owner information	17

6.6.1	Description	17

6.6.2	Evaluation	19

6.X	Solution #X: <Title>	19

6.X.1	Solution description	19

6.X.2	Solution evaluation	19

7	Overall evaluation	19

8	Conclusions	20

Annex A (informative): Change history	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document studies the potential enhancements in CAPIF and application enablement frameworks (e.g. SEAL, EDGEAPP, vertical enabler layers) to support the subscriber-aware northbound API access (SNA), whose requirements are specified in TS 22.261 [2].

This study takes into consideration the existing works for CAPIF (3GPP TS 23.222 [3]), SEAL (3GPP TS 23.434 [4]), and EDGEAPP (3GPP TS 23.558 [5]).

Any potential enhancements to security procedures related to CAPIF and application enablement frameworks (e.g. EDGEAPP, SEAL, vertical enabler layers) to support SNA are not in the scope of this study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1"

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows"

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[6]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping"

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

resource owner: An entity capable of granting access to a protected resource.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

AF	Application Function

CAPIF	Common API Framework

CCF	CAPIF Core Function

SNA	Subscriber-aware northbound API access

# 4	Key issues

## 4.1	Key Issue #1: UE-originated API invocation

### 4.1.1	Description

The UE-originated API invocation scenario corresponds to the following requirement specified in TS 22.261 [2]:

-	The 5G system shall be able to provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE.

In this scenario, the application on the UE invokes the northbound APIs. The scenario is illustrated in figure 4.1.1-1.

From CAPIF point of view, the application on the UE in this scenario plays the role of the API invoker, as defined in TS 23.222 [3].

Figure 4.1.1-1: UE-originated API invocation

Open issues:

1)	Whether the existing CAPIF functional model is sufficient for the UE-originated API invocation.

a)	If the existing CAPIF functional model is insufficient, how it must be modified to accommodate for the API invoker in the UE.

## 4.2	Key Issue #2: AF-originated API invocation

### 4.2.1	Description

In the AF-originated API invocation scenario, the AF invokes the northbound APIs, and the application on the UE consumes the service from the AF. The scenario is illustrated in figure 4.2.1-1.

Figure 4.2.1-1: AF-originated API invocation

Open issues:

1)	Whether the existing CAPIF functional model is sufficient for the AF-originated API invocation.

a)	If the existing CAPIF functional model is insufficient, how it should be updated.

2)	Study the business relationship between the user (UE), the AF and the northbound API provider.

## 4.3	Key Issue #3: Providing and revoking user consent upon invoking APIs

### 4.3.1	Description

Invocation of some northbound APIs may require obtaining user consent (e.g. API to retrieve the location of the UE).

In the SNA context, it is for further study how to obtain user consent when the API invocation is related to the resource owner (e.g. the user).

In addition, the system should ensure that there are mechanisms for the resource owner to revoke the user consent provided previously for the API invocation.

Open issues:

1)	Whether and how CAPIF functions can determine the resource owner upon API invocation.

2)	Whether and how CAPIF can support obtaining user consent from resource owner.

3)	Whether and how CAPIF can support revoking user consent by the resource owner.

NOTE:	This document studies the high-level architecture to solve this issue. The detailed procedure for providing the user consent will be specified in SA3.

## 4.4	Key Issue #4: Discovery of target API information

### 4.4.1	Description

As per the below requirement from 3GPP TS 22.261 [2] (clause 6.10.2), the third party needs to be provided with information to locate the target APIs in the network.

-	provide a third-party with information to identify networks and APIs on those networks.

An AF may serve UEs that belong to different PLMNs. When the AF is not aware of the PLMN provider of the UEs and has to discover the service API in relation to such UEs, then it is not clear how to discover the service APIs. How the AF can determine the right AEF instance, which is exposing the required Northbound API for which the AF wants to consume that Northbound API with the UE(s) information.

Open Issue:

-	Whether and how to support discovery of the target Service API information in CAPIF with respect to UE(s) information, especially when:

-	AF is serving UEs belonging to multiple PLMNs; and

-	There are multiple instances of the AEF within a PLMN.

# 5	Architectural requirements

## 5.1	General requirements

[AR-5.1-a] The confidentiality of the UE's external identity (e.g. MSISDN) shall be preserved during the API invocation supported by CAPIF.

[AR-5.1-b] The CAPIF shall support the authentication of the resource owner.

[AR-5.1-c] The CAPIF shall enable the resource owner(s) to provide and revoke user consent.

# 6	Solutions

Editor's Note:	This clause will provide descriptions for the different solutions.

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key issue 1 | Key issue 2 | Key issue 3 | Key issue 4 | … |
| --- | --- | --- | --- | --- | --- |
| Solution 1 | X | X |  |  |  |
| Solution 2 |  |  | X |  |  |
| Solution 3 |  |  | X |  |  |
| Solution 4 |  |  | X |  |  |
| Solution 5 | X |  |  |  |  |
| Solution 6 |  |  |  | X |  |

## 6.1	Solution #1: Business relationship in SNA

### 6.1.1	Solution description

This solution addresses the key issue #2 with regard to the business relationship between the user (UE), the AF and the northbound API provider in the AF-originated API invocation scenario. Considering the business relationship, the resource owner (which is a UE-side entity) is a new entity that has not been in the existing CAPIF business relationship, thus the business relationship should be updated to include the resource owner.

Figure 6.1.1-1 shows the typical business relationship in SNA. This business relationship can be applied to both AF- and UE-originated API invocation scenario, as the API invoker in figure 6.1.1-1 can either be an application on the UE or the AF.

Figure 6.1.1-1: Business relationships in SNA

The API invoker has service agreement with a CAPIF provider, and the API provider provides APIs associated with the resource owner. The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), as described in 3GPP TS 23.222 [3] clause 5. When the CAPIF provider is a PLMN operator, the resource owner may be a subscriber of the PLMN.

### 6.1.2	Solution evaluation

This solution enhances the existing CAPIF business relationship by introducing the resource owner, which is viable.

## 6.2	Solution #2: Functional model description for SNA

### 6.2.1	Solution description

#### 6.2.1.1	General

This solution addresses the key issue #3 with regard to obtaining and revoking resource owner consent from the resource owner client. As the resource owner client is a new entity for CAPIF, the functional model for the CAPIF should be updated including the resource owner client and resource owner authorization function.

#### 6.2.1.2	Functional model

Figure 6.2.1.2-1 shows the reference point based functional model for the CAPIF with enhancements to support SNA.

Figure 6.2.1.2-1: Functional model for the CAPIF with SNA enhancements

The resource owner(s) are end-users or subscribers of the API provider domain's service provider as specified in 3GPP TS 33.501 [8].

The resource owner(s) reside within the API provider domain and interacts with the resource owner authorization function using resource owner clients(s) via CAPIF-8. The resource owner communicates with the resource owner authorization function to provide and revoke resource owner consent. The resource owner interactions are supported via a resource owner client, which is a client-side entity.

Editor's Note:	The location of the resource owner authorization function in the CAPIF functional model is FFS.

The API exposing function (e.g NEF) acts as a resource owner consent enforcement point as specified in 3GPP TS 33.501 [8] and interacts with the resource owner authorization function via CAPIF-9. The API exposing function shall support to retrieve the resource owner consent parameters from.the resource owner authorization function.

The API invoker interacts with resource owner authorization function via CAPIF-10/CAPIF-10e. The API invoker shall support to retrieve the resource owner consent parameters from the resource owner authorization function.

The API invoker interacts with CAPIF core function via CAPIF-1/CAPIF-1e to discover and obtain API access information. Further, the API invoker interacts with API exposing function via CAPIF-2/CAPIF-2e to perform service API invocations.

NOTE:	Security aspects including specification of the authorisation procedures are to be decided in SA3.

#### 6.2.1.3	Deployment models

Editor's note:	The deployment models for the functional model in clause 6.2.1.2 are FFS.

### 6.2.2	Solution evaluation

This solution enhances the existing CAPIF functional model by introducing the resource owner client and the resource owner authorization function, which is viable.

## 6.3	Solution #3: Obtaining resource owner consent upon service API invocation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution addresses the key issue #3 with regard to obtaining resource owner consent. When the AEF acting as user consent enforcement point determines to obtain resource owner consent upon service API invocation, the API exposing function interacts with the resource owner authorization function to obtain the resource owner consent information. For the interaction between the resource owner authorization function and the API exposing function, the resource owner registration may optionally be performed prior to the service API invocation.

Clause 6.3.1.2 shows the procedure for resource owner registration and clause 6.3.1.3 shows the procedure for obtaining resource owner consent.

#### 6.3.1.2	Resource owner registration

Figure 6.3.1.2-1 shows the procedure for resource owner registration.

Pre-conditions:

1.	The resource owner client is authorised to access the resource owner authorization function; and

2.	The resource owner client has information for accessing the resource owner authorization function.

Figure 6.3.1.2-1: Procedure for resource owner registration

1.	The resource owner client sends resource owner registration request to the resource owner authorization function, including the resource owner identity (e.g. GPSI) and security credentials.

2.	The resource owner authorization function initiates the verification procedure and validates the resource owner registration request.

3.	The resource owner registration function sends resource owner registration response.

#### 6.3.1.3	Obtaining resource owner consent

Figure 6.3.1.3-1 shows the procedure to obtain resource owner consent upon service API invocation.

Pre-conditions:

1.	The resource owner client is registered to the resource owner authorization function and can be accessed by the API exposing function.

Figure 6.3.1.3-1: Procedure for obtaining resource owner consent upon service API invocation

1.	The API invoker sends service API invocation request to the API exposing function via CAPIF-2.

2.	The API exposing function determine if the resource owner consent verification is required to execute the service API. If the resource owner consent verification is not required for the API invocation, the steps 3-5 will be skipped.

NOTE:	The API exposing function determines if the resource owner consent verification is required based on implemented API logic.

The API exposing function identifies the resource owner client in 3GPP network context by one of the following processes:

-	If the service API invocation indicates the resource owner client by GPSI, the API exposing function identifies the resource owner client by retrieving the resource owner information from the resource owner authorization function, which has received the resource owner information in resource owner registration, as shown in clause 6.3.1.2.

-	If the service API invocation indicates the resource owner client by UE (IP or Ethernet) address, the API exposing function identifies the resource owner client by interacting with BSF, as specified in TS 29.513 [6].

-	If the service API invocation indicates the resource owner client by external group identifier, the API exposing function identifies the individual resource owner clients by interacting with HSS/UDM, as specified in TS 23.682 [7].

-	If the service API invocation indicates the resource owner client by "any UE" identification, the API exposing function regards all the available resource owner clients as the target resource owner clients.

3.	The API exposing function obtains the resource owner consent from the resource owner client. The result of the resource owner consent response may be stored in the API exposing function.

NOTE:	The detailed procedure to obtain the resource owner consent and manage such resource owner consent information will be specified in SA3.

4.	If the API invocation is allowed, the process for service API execution is continued. If the API invocation is denied, the service API execution is rejected.

5.	The API exposing function sends service API invocation response to the API invoker based on the result of the service API execution.

#### 6.3.1.4	Updating resource owner consent

The resource owner client may update resource owner consent anytime to allow or deny future API invocation.

NOTE:	The detailed procedure to update the resource owner consent will be specified in SA3.

### 6.3.2	Solution evaluation

Editor's Note:	This clause will provide the evaluation for the solution.

NOTE:	The API exposing function may need a solution to contact UE through firewall in Step 3 of clause 6.3.1.2.

## 6.4	Solution #4: API invoker obtaining resource owner consent

### 6.4.1	Solution description

This solution addresses the key issue #3 with regard to obtaining user consent. CAPIF may authorize the API invoker to invoke the service API based on the user consent slightly before the API invocation. The procedures to obtain the user consent may reuse well-known authorization procedures such as OAuth 2.0 [9].

Clause 6.4.1.1 shows the procedure for obtaining user consent in a near real-time manner.

#### 6.4.1.1	API invoker obtaining resource owner consent prior to the service API invocation

Figure 6.4.1.1-1 shows the procedure to obtain resource owner consent prior to the service API invocation, which is used in the near real-time consent scenario.

Pre-conditions:

1.	The resource owner can communicate with the API invoker.

Figure 6.4.1.1-1: Procedure for API invoker obtaining resource owner consent prior to the service API invocation

1.	The API invoker requests authorization grant and access token to invoke the service API.

NOTE:	The detailed procedure to obtain user consent and provide an access token is studied in SA3.

2.	The API invoker sends service API invocation request to the API exposing function with the access token received in step 1.

### 6.4.2	Solution evaluation

Editor's Note:	This clause will provide the evaluation for the solution.

## 6.5	Solution #5: UE-originated API invocation within CAPIF

### 6.5.1	Description

KI#1 describes the scenario of UE application client (AC) invokes 3GPP northbound APIs (e.g. NEF location API). In figure 6.5.1-1, the AC plays the role of the API invoker, and triggers a number of service operations with CCF and AEF (e.g. onboarding, security, API discovery).

Figure 6.5.1-1: Overview of CAPIF operations

The existing CAPIF model and procedures for UE-originated API invocation with CAPIF may be used but it is not so efficient since UE may be installed with many application clients (e.g. 100-200 apps). To reduce the number of CAPIF interactions per UE, a CAPIF client agent in the UE can be used as a common API invoker. Such CAPIF client agent executes the onboarding, authentication and authorization for security, service API discovery (discovered result can be re-used by all ACs), CAPIF event subscription with CCF.

NOTE:	Security aspects including specification of the authentication and authorisation procedures for UE-originated API invocation within CAPIF are to be decided in SA3.

In short, the CAPIF client agent (as an application in the UE) is seen as an API invoker (see figure 6.5.1-2).

Figure 6.5.1-2: CAPIF client agent functionality in the UE

NOTE:	Reference point detail between AC and CAPIF client agent is out of 3GPP scope.

### 6.5.2	Solution evaluation

This solution addresses KI#1. It introduces an option to have a common API invoker (i.e. CAPIF client agent) for the UE so that the signalling over CAPIF reference points can be optimized.

Editor's Note:	The CAPIF operations by CAPIF client agent (API invoker) considering per application granularity is FFS.

## 6.6	Solution #6: Discover a proper AEF with owner information

### 6.6.1	Description

The API provider hosts one or more service APIs and has a service API arrangement with CAPIF provider to offer the service APIs to the API invoker, as described in 3GPP TS 23.222 [3]. A CAPIF provider may have business relationships with multiple API providers (e.g. MNOs).

The AF knows UE IP address from the Application traffic. If the AF wants to contact with an API exposing function (AEF) provided by MNO but does not know which MNO the UE belongs to, the AF can utilize the information provided by Regional Internet Registry (RIR). e.g. Asia-Pacific Network Information Centre (APNIC) is responsible for providing allocation and administration services to Asia-Pacific's Internet service providers (ISPs) and Internet users. IP "whois" service can be used to retrieve IP address related information, for instance, with input of IP address 223.167.255.255 in https://wq.apnic.net/apnic-bin/whois.pl it gives the following result including the owner for such IP address (only partial result is listed here for relevant info):

% APNIC found the following authoritative answer from: whois.apnic.net

% [whois.apnic.net]

% Whois data copyright terms    http://www.apnic.net/db/dbcopyright.html

% Information related to '223.166.0.0 - 223.167.255.255'

% Abuse contact for '223.166.0.0 - 223.167.255.255' is 'hqs-ipabuse@chinaunicom.cn'

inetnum:        223.166.0.0 - 223.167.255.255

netname:        UNICOM-SH

descr:          CHINA UNICOM Shanghai city network

descr:          China Unicom

descr:          No.21,Jin Rong Street,Beijing,100033

descr:          P.R.China

country:        CN

admin-c:        CH1302-AP

tech-c:         CH1302-AP

remarks:        service provider

mnt-by:         APNIC-HM

mnt-lower:      MAINT-CNCGROUP-SH

mnt-routes:     MAINT-CNCGROUP-RR

status:         ALLOCATED PORTABLE

mnt-irt:        IRT-CU-CN

last-modified:  2013-08-08T23:17:38Z

source:         APNIC

Noting that such interaction with IP address administrative authority is out of the scope of 3GPP.

Once the AF knows which MNO it should contact for the API invocation related to a UE, the AF queries the CAPIF core function (CCF) with required API provider name (e.g. China Mobile Beijing) and CCF responds with the AEF information matching the required API provider name. The CCF knows the API provider name either via API management function triggered Registration procedure for the whole API provider domain functions or via the API publish function triggered service API information publish procedure for the AEF offered API(s). The impact for CAPIF procedures is illustrated in figure 6.6.1-1, and tables 6.6.1-1, table 6.6.1-2 and 6.6.1-3 describe the information flow impact with highlighted IEs.

Figure 6.6.1-1: API provider name used in CAPIF procedures

Table 6.6.1-1: Registration request

| Information element | Status | Description |
| --- | --- | --- |
| List of API provider domain functions | M | List of API provider domain functions including role (e.g. AEF, APF, AMF) and, if required, specific security information. |
| API provider name | O | The API provider name uniquely identifies an API provider (e.g. Internet Service Provider). |
| Security information | M | Information for CAPIF core function to validate the registration request |

Table 6.6.1-2: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

Table 6.6.1-3: Service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), interfaces, protocols) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

The API provider name can be shared between CCFs over CAPIF-6/6e reference point and the interconnection service API discovery may also include the required API provider name as query parameter.

Alternatively, to reduce the burden on API invoker side, API invoker may include subscriber IP address in the service API discover request in step 6 above and CCF uses implementation specific means (e.g. whois) to resolve the subscriber IP address to its owner information (e.g. API provider name) and then find the matching service API information with the owner information.

### 6.6.2	Evaluation

This solution addresses KI#4, it enables the API invoker to discover a proper AEF with API provider name or subscriber IP address for invoking an API related to UE. It is assumed that all AEF instances provided by the API provider know their owner's information and API invoker knows subscriber IP address. This solution is a viable CAPIF solution to be considered.

## 6.X	Solution #X: <Title>

### 6.X.1	Solution description

### 6.X.2	Solution evaluation

# 7	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the study item activities.

# Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211548 |  |  |  | TR skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | S6-211821, S6-211852, S6-211853, S6-211854, S6-211855, S6-211856 | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | S6-211871, S6-212075, S6-212076, S6-212165 | 0.2.0 |
| 2021-10 | SA6#45-bis-e |  |  |  |  | S6-212242, S6-212475 | 0.3.0 |
| 2021-11 | SA6#46-e |  |  |  |  | S6-212809, S6-212810, S6-212811, S6-212812, S6-212826 | 0.4.0 |
| 2021-11 | SA#94-e | SP-211508 |  |  |  | Presentation for information at SA#94-e | 1.0.0 |
| 2022-02 | SA6#47-e |  |  |  |  | S6-220449, S6-220450, S6-220451, S6-220452, S6-220453 | 1.1.0 |