| 3GPP TR 23.700-98 V0.6.0 (2022-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Enhanced architecture for enabling Edge Applications; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

Introduction	9

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Key issues	11

4.1	Key issue #1: Enhanced notification service to the EEC	11

4.2	Key issue #2: Enablement of Service APIs exposed by EAS	12

4.3	Key issue #3: Enhancements to service continuity planning	12

4.4	Key issue #4: EDGE-5	13

4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC	14

4.6	Key issue #6: Edge services support across ECSPs	14

4.7	Key issue #7: Application traffic filter exposure	15

4.8	Key issue #8: EAS selection synchronization	16

4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering	17

4.10	Key issue #10: Support for roaming UEs	17

4.11	Key issue #11: ACR between EAS and Cloud Application Server	18

4.12	Key issue #12: EEL service differentiation	18

4.13	Key issue #13: Edge enabler layer support for EAS synchronization	18

4.14	Key issue #14: Application traffic influence for initially selected EAS	19

4.15	Key issue #15: Support of constrained devices for Edge	19

4.16	Key issue #16: support of NAT deployed within the edge data network	20

4.17	Key issue #17: Discovery of a common EAS	20

4.18	Key issue #18: Linkage between EASs	21

4.19	Key issue #19: ACR scenario combination	22

4.20	Key issue #20: Method of supporting federated EAS service	22

4.21	Key issue #21: Simultaneously EAS connectivity in ACR	22

4.22	Key issue #22: EAS discovery in Edge Node sharing scenario	24

4.x	Key issue #x: <title>	24

5	Architectural requirements	24

5.1	General requirements	24

5.1.1	General	24

5.1.2	Requirements	24

5.2	Enablement of Service APIs exposed by EAS	25

5.2.1	General	25

5.2.2	Requirements	25

5.3	ECS discovery	25

5.3.1	General	25

5.3.2	Requirements	25

5.x	<specific requirements>	25

5.x.1	General	25

5.x.2	Requirements	26

6	Enhanced Application Architecture	26

6.1	Option #1: Roaming architecture	26

6.1.1	Architecture enhancements	26

6.1.1.1	Local breakout roaming architecture: Local breakout to access H-ECS	26

6.1.1.2	Home-routed EDGE-4 access to H-ECS	26

6.1.2	Identities	27

6.1.3	Cardinality rules	27

6.2	Option #2: Non-roaming architecture	27

6.2.1	Architecture enhancements	27

6.2.2	Identities	28

6.2.3	Cardinality rules	28

6.3	Option #3: Edge Notification Server architecture	28

6.3.1	Architecture enhancements	28

6.3.1.1	Edge Notification Server (ENS)	29

6.3.1.2	ENS Discovery	30

6.4	Option #4: Constrained devices with limited capabilities	30

6.4.1	Architecture enhancements	30

6.4.1.1	General	30

6.4.1.2	Architecture	30

6.4.2	Identities	30

6.4.3	Cardinality rules	31

6.5	Option #5: Architecture with Cloud Application Server (CAS)	31

6.5.1	Architecture enhancements	31

6.5.2	Identities	32

6.5.3	Cardinality rules	32

6.x	Option #x: <title>	32

6.x.1	Architecture enhancements	32

6.x.2	Identities	32

6.x.3	Cardinality rules	32

7	Solutions	32

7.0	Mapping of solutions to key issues	32

7.1	Solution #1: Service provisioning via push notification	34

7.1.1	Architecture enhancements	34

7.1.2	Solution description	34

7.1.2.1	General	34

7.1.2.2	Procedure	34

7.1.3	Solution evaluation	35

7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection	35

7.2.1	Architecture enhancements	35

7.2.2	Solution description	35

7.2.3	Solution evaluation	36

7.3	Solution #3: Service provisioning triggering via SMS over NAS	37

7.3.1	Architecture enhancements	37

7.3.2	Solution description	37

7.3.2.1	General	37

7.3.2.2	Procedure	37

7.3.3	Solution evaluation	38

7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs	39

7.4.1	Architecture enhancements	39

7.4.2.1	General	39

7.4.2.2	Procedure	39

7.4.3	Solution evaluation	41

7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs	41

7.5.1	Architecture enhancements	41

7.5.2	Solution description	41

7.5.2.1	General	41

7.5.2.2	Procedure	41

7.5.3	Solution evaluation	43

7.6	Solution #6: ACR update in service continuity planning	43

7.6.1	Architecture enhancements	43

7.6.2	Solution description	43

7.6.2.1	ACR modification solution	44

7.6.2.1.1	EEC-based ACR modification procedure	44

7.6.2.1.2	EES-based modification procedure	44

7.6.2.1.3	ACR modification execution procedure	45

7.6.3	Solution evaluation	46

7.7	Solution #7: EES monitors UE mobility for service continuity planning	46

7.7.1	Architecture enhancements	46

7.7.2	Solution description	47

7.7.2.1	General	47

7.7.2.2	Procedure	47

7.7.3	Solution evaluation	49

7.8	Solution #8: EAS Service API enablement using CAPIF	50

7.8.1	Architecture enhancements	50

7.8.2	Solution description	50

7.8.2.1	General	50

7.8.2.2	CAPIF operations in Edge Enabler Layer	50

7.8.2.3	Service KPIs in CAPIF for EAS Service APIs	51

7.8.3	Solution evaluation	54

7.9	Solution #9: Application traffic influence trigger from EAS	54

7.9.1	Architecture enhancements	54

7.9.2	Solution description	55

7.9.2.1	General	55

7.9.2.2	procedure	55

7.10	Solution #10: low power mode support	55

7.10.1	Architecture enhancements	55

7.10.2	Solution description	55

7.10.2.1	General	55

7.10.2.2	Procedure	56

7.11	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF	57

7.11.1	Architecture enhancements	57

7.11.2	Solution description	57

7.11.2.1	General	57

7.11.2.2	Procedure	59

7.11.3	Solution evaluation	59

7.12	Solution #12: Service continuity planning allowance	59

7.12.1	Architecture enhancements	59

7.12.2	Solution description	59

7.12.2.1	General	59

7.12.2.2	Procedure	59

7.12.3	Solution evaluation	60

7.13	Solution #13: Update ECS configuration information	60

7.13.1	Architecture enhancements	60

7.13.2	Solution description	60

7.13.3	Solution evaluation	61

7.14	Solution #14: V-ECS Discovery via the H-ECS	61

7.14.1	Architecture enhancements	61

7.14.2	Solution description	61

7.14.2.1	General	61

7.14.2.2	Procedure	61

7.14.3	Solution evaluation	63

7.15	Solution #15: Initial EAS selection declaration	63

7.15.1	Architecture enhancements	63

7.15.2	Solution description	63

7.15.3	Solution evaluation	65

7.16	Solution #16: EAS discovery for different users	65

7.16.1	Architecture enhancements	65

7.16.2	Solution description	65

7.16.2.1	General	65

7.16.2.2	Procedure	65

7.16.3	Solution evaluation	65

7.17	Solution #17: Traffic influence for initial EAS discovery	66

7.17.1	Architecture enhancements	66

7.17.2	Solution description	66

7.17.2.1	General	66

7.17.2.2	Procedure	66

7.17.3	Solution evaluation	66

7.18	Solution #18: Constraint device in EDGEAPP	67

7.18.1	Architecture enhancements	67

7.18.2	Solution description	67

7.18.3	Solution evaluation	69

7.19	Solution #19: EES determines the selected ACR scenario	69

7.19.1	Architecture enhancements	69

7.19.2	Solution description	69

7.19.2.1	General	69

7.19.2.2	Procedure	69

7.19.3	Solution evaluation	70

7.20	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server	70

7.20.1	Architecture enhancements	70

7.20.2	Solution description	70

7.20.2.1	General	70

7.20.2.2	Notification delivery over a direct Notification Channel Procedure	70

7.20.2.3	Notification delivery using a Push Server Procedure (indirect Notification Channel)	72

7.20.3	Solution evaluation	74

7.21	Solution #21: Prediction expiration time for service continuity planning enhancement	74

7.21.1	Architecture enhancements	74

7.21.2	Solution description	74

7.21.2.1	General	74

7.21.2.2	Procedure	74

7.21.3	Solution evaluation	76

7.22	Solution #22: Support simultaneous EAS connectivity in ACR	76

7.22.1	Architecture enhancements	76

7.22.2	Solution description	76

7.22.2.1	Solution for traffic influence	76

7.22.3	Solution evaluation	79

7.23	Solution #23: UE identification with NAT	79

7.23.1	Architecture enhancements	79

7.23.2	Solution description	79

7.23.2.1	General	79

7.23.2.2	Procedure	80

7.23.3	Solution evaluation	81

7.24	Solution #24: ACR between CAS and EAS	81

7.24.1	Architecture enhancements	81

7.24.2	Solution description	81

7.24.3	Solution evaluation	82

7.25	Solution #x25: ACR between EAS and Cloud Application Server	82

7.25.1	Architecture enhancements	82

7.25.2	Solution description	82

7.25.2.1	General	82

7.25.2.2	Procedure	82

7.25.2.2.1	Updated 3GPP TS 23.558 clause 8.8.2.2 Initiation by EEC using regular EAS Discovery	82

7.25.3	Solution evaluation	84

7.26	Solution #26: Bundled EASs	84

7.26.1	Architecture enhancements	84

7.26.2	Solution description	84

7.26.2.1	General	84

7.26.2.2	Handling of EAS bundle information by the EEC	86

7.26.2.3	Handling of EAS bundle information and EAS bundle requirements by the EES	86

7.26.3	Solution evaluation	87

7.27	Solution #27: Enabling AC Association Aware services by selecting common EASs	87

7.27.1	Architecture enhancements	87

7.27.2	Solution description	87

7.27.2.1	General	87

7.27.2.2	New Information Elements	87

7.27.2.3	Enhancements to existing Information Elements	88

7.27.2.4	Procedure for determining common EES	89

7.27.2.5	Discovery of common EAS Procedure	89

7.27.2.6	ACR with common EAS Procedure	90

7.27.3	Solution evaluation	90

7.28	Solution #28: Common EAS discovery using EAS selection information	90

7.28.1	Architecture enhancements	90

7.28.2	Solution description	90

7.28.2.1	General	90

7.28.2.2	Procedure	91

7.28.3	Solution evaluation	92

7.29	Solution #29: Discovery of a common EAS	92

7.29.1	Architecture enhancements	92

7.29.2	Solution description	92

7.29.2.1	General	92

7.29.2.2	Procedure	94

7.29.3	Solution evaluation	96

7.x	Solution #x: <title>	96

7.x.1	Architecture enhancements	96

7.x.2	Solution description	96

7.x.3	Solution evaluation	96

8	Deployment scenarios	96

8.1	<Title>	96

9	Involved entities and relationships	96

9.1	Federation and Roaming	96

9.1.1	General	96

10	Overall evaluation	97

10.1	Architecture enhancements	98

10.2	Key issue evaluations	98

10.2.1	General	98

10.2.2	Key issue #14: Application traffic influence for initially selected EAS	98

10.2.x	Key issue #x	98

11	Conclusions	98

11.1	Architecture enhancements	99

11.2	Solutions	99

Annex A (Informative): ETSI MEC and EDGEAPP system comparison	100

A.1	General	100

A.2	Service consumer and service provider	100

A.3	EAS/MEC application profile provisioning	100

A.4	EAS registration and EAS discovery	101

Annex B (Informative): Deployment and Evolution options of EDGEAPP and ETSI MEC platforms	102

B.1	General	102

B.2	Deployment options	103

B.2.1	Deployment Option-1: Collocated Platforms	103

B.2.2	Deployment Option-2: Converged architecture	103

B.2.2.1	General	103

B.2.2.2	Evolution Options	104

B.2.2.2.1	General	104

B.2.2.2.2	Evolution Option #1- Enhancement of a deployed MEP to support the functionality of EES	104

B.2.2.2.3	Evolution Option #2 Enhancement of a deployed EES to support the functionality of MEP	105

B.2.3	Deployment Option-3: non-Collocated Platforms	105

Annex C (informative): Change history	106

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Rel-17 TS 23.558 [2] defines the application layer architecture to enable edge applications over 3GPP networks. It includes features such as ECS discovery, service provisioning, EAS discovery, EEC/EAS/EES registration, network and Edge Enabler Layer capability exposure, service continuity support with seamless service continuity and EEC context continuity etc., along with cardinality rules, deployment options, involved relationships and mapping with ETSI MEC [3] and GSMA OP [4] architectures.

This TR documents a study on architecture and procedure enhancements to improve the Rel-17 architecture for enabling edge applications, and to support emerging industry requirements.

# 1	Scope

The present document is a technical report capturing the study on enhanced architecture for enabling edge applications over 3GPP networks. The study bases the enhancements on the work done in 3GPP TS 23.558 [2] and takes into consideration other related work done within and outside 3GPP i.e. ETSI MEC [3] and GSMA OP [4].

The aspects of the study include identifying new key issues, architecture requirements, related architecture enhancements and solutions, cardinality rules, deployment options, and involved entities and relationships to enhance the Rel-17 architecture for enabling edge applications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications".

[3]	ETSI ISG MEC ETSI GS MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[4]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[7]	3GPP TR 23.721: "Study on Sponsored Data Connectivity Improvements".

[8]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[10]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[11]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[12]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[13]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[14]	ETSI GS MEC 011 v3.0.5: " Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[15]	ETSI GS MEC 001: " Multi-access Edge Computing (MEC); Terminology ".

[16] 	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[17]	3GPP TS 29.522: "Network Exposure Function Northbound APIs; Stage 3".

[18]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[19]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key issue #1: Enhanced notification service to the EEC

The EEC can be notified with the updated information for edge computing service by EES and ECS over EDGE-1 and EDGE-4, respectively. The notification services available to the EEC (i.e., service provisioning notification, EAS discovery notification, ACR information notification) are specified in release 17, but the details on the notification mechanism is not addressed. In this regard, additional ways (e.g. push notification mechanism and application triggering specified in 3GPP TS 23.501 [5], etc.) of providing the updates to the EEC in real time need to be studied.

Open issues:

1.	Whether and how the EEC acquires the notification target address or a notification channel URI to receive the notifications?

2.	Whether and how EEC, ECS and EES support push notification mechanism. Whether and what additional functional entity is necessary for this?

3.	How are the EEC subscriptions and/or notification targets treated in mobility scenario (e.g. during ACR scenario)?

4.	Whether and how to utilize application triggering method specified in 3GPP TS 23.501 [5] to provide notifications to the EEC?

## 4.2	Key issue #2: Enablement of Service APIs exposed by EAS

As specified in 3GPP TS 23.558 [2] (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES as specified in the clause 8.6 of 3GPP TS 23.558 [2] (Rel-17) and the capabilities provided by the 3GPP core network as specified in the clause 8.7 of 3GPP TS 23.558 [2] (Rel-17).

However, there are several use cases and requirements to exploit Service APIs exposed by EASs (provided by Application service providers) which can be invoked by the other EASs such as Cloud/Edge/Split Rendering in AR/VR use cases; and video content delivery exploiting separate services such as content caching, video encoding/decoding, and video analytics.

With the enablement of Service APIs exposed by EASs in the Edge Enabler Layer, there may expect several benefits to the involved business roles in the edge computing services. For example, Application service providers may simplify the Edge Application logic by composing the service components provided by the other EASs at the network edge; and open a new business to provide one or more service components to be executed and invoked by the other EASs at the network edge.

In order to enable the use of Service APIs exposed by EASs in the Edge Enabler Layer, there have been relevant studies in 3GPP TR 23.758 [6] (Rel-17), which result in Sol#15 as follows:

-	Solution #15: Edge Application Server's service APIs publish and discovery using CAPIF:

-	The Edge Enabler Server can support edge application (owned by 3rd party or by PLMN operator) access to the service APIs offered by other Edge Application Servers within and across the Edge Data Network by providing CAPIF functions in a distributed or centralized manner.

Based on the Rel-17 study result, this key issue focuses on addressing the following open issues.

Open issues:

1.	Identify any gaps in CAPIF to enable EAS Service APIs in the EDGEAPP architecture based on the Sol#15 of 3GPP TR 23.758 [6] (Rel-17).

-	In terms of e.g., service-specific attributes for API publish/discovery, API availability subscription/notification across EES/CCF

2.	If any, whether and how to enhance CAPIF capabilities to address the gap identified as above?

## 4.3	Key issue #3: Enhancements to service continuity planning

In 3GPP TS 23.558 [2], the service continuity planning was specified as part of the Edge Enabler Layer value-add features for supporting seamless service continuity, when information about planned, projected, or anticipated behaviour is available at EESs or provided by EECs.

This key issue studies potential enhancements to the service continuity planning feature, based on additional criteria for detecting a planned ACR, e.g. the network conditions / analytics monitoring, the DN performance monitoring, the expected/predicted UE route etc. This key issue also studies potential enhancements to the service continuity planning feature to allow the EEC to send a timely ACR request. Finally, the key issue studies scenarios when the planned UE behavior changes after the launch of the service and the ACR needs to be modified due to these changes, e.g. due to UE mobility change.

Open Issues:

1.	How to rely on the capability of EES/EEC to detect whether the UE moves to the predicted location or not for service continuity planning?

2.	Whether and how the EEL can support the determination of the ACR request trigger timing in case of service continuity planning?

3.	How to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility?

4.	Whether and what additional capability exposure is required from the 5GS (e.g. NWDAF, OAM) to enhance the service continuity planning?

5.	Potential impact on information exchanged between EAS and EEL.

6.	Potential impact on information to communicate within the EEL.

## 4.4	Key issue #4: EDGE-5

According to the latest version of the EDGEAPP specification in 3GPP TS 23.558 [2]:

"EDGE-5 reference point enables interactions between AC(s) and the EEC."

That specification also notes:

"NOTE:	Detailed specification of this reference point is out of scope of this release of this specification."

According to the latest version of the FS_EDGEAPP specification in 3GPP TR 23.758 [6]:

"The interactions between Application Client(s) and the Edge Enabler Client in the UE are supported by the EDGE-5 reference point. This reference point supports:

-	Obtaining information about Edge Application Servers that Application Client require to connect;

-	Notifications about events related to the connection between Application Clients and their corresponding Edge Application Servers, such as: when an Application Client needs to reconnect to a different Edge Application Server;

-	Providing Application Client information (such as its profile) to be used for various tasks such as, identifying the appropriate Edge Application Server instance to connect to; and

-	Provide the identity of the desired Edge Application Server to the Edge Enabler Client to enable it to use that identity as a filter when requesting information about Edge Application Servers."

The necessary functionality of the EDGE-5 interface needs to be studied for Release 18:

-	What functionality should an EEC provide to an AC (see the interactions copied from 3GPP TR 23.758 [6] and the procedures and information flows specified in 3GPP TS 23.558 [2])?

The following aspects may also need to be studied for Release 18 based upon the provided functionality of the EDGE-5 interface:

1.	Whether the cardinality as currently captured in 3GPP TS 23.558 [2] is to be modified:

"The following cardinality rules apply for EDGE-5:

a)	One AC may communicate with only one EEC; and

b)	One EEC may communicate with one or more AC(s) concurrently."

2.	Whether and how an AC can discover available EEC(s)?

3.	Whether mutual authentication and authorization between an AC and an EEC is necessary, and if so, how is that accomplished?

4.	What APIs should be exposed from an EEC to an AC to support that functionality?

5.	Whether a notification mechanism is necessary from an EEC to an AC?

6.	Whether and how an AC registers to an EEC?

7.	Whether and how an AC de-registers from an EEC?

8.	Whether and how an AC detects an abnormal termination of an EEC?

9.	Whether and how an EEC detects an abnormal termination of an AC?

10.	Whether user's consent is necessary to either AC or EEC operation, and if so, how is it provided?

11.	Whether and how can EDGE-5 support constrained devices with limited capabilities (such as a terminal equipment as defined in TR 21.905) over EDGE-5?

NOTE 1:	The aspects of defining end-user consent/authorization over APIs and aspects of mutual authentication and authorization between an AC and an EEC are in the scope of SA3.

NOTE 2:	The aspects of the usage of end-user consent/authorization over APIs is in the scope of SA6.

## 4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC

As described in Annex C of 3GPP TS 23.558 [2] (Rel-17), both EDGEAPP and ETSI MEC can provide support for hosting different edge applications. According to Annex C: "Both EAS and MEC application are application servers and can provide similar application specific functionalities. EAS utilizes the services of EES as specified in this document whereas MEC application utilizes the services provided by MEC platform as specified in ETSI GS MEC 003." As discussed in Annex B.2 of draft GS MEC 003 [3] (v3.0.4), the EES and MEC platform can also be collocated in an implementation.

While the 3GPP TS 23.558 [2] and ETSI GS MEC 003 [3] provide an initial view about the alignment of the two platforms, this KI intends to address the following:

1.	Study and analyse different deployment options of EDGEAPP and ETSI MEC platforms.

2.	Functional architecture and gap analysis between EDGEAPP and ETSI MEC to determine complementary and possibly overlapping APIs and other related functionalities. Annex A captures a comparison of the architectures to facilitate the gap analysis.

3.	Recommendation and enhancements based upon the outcome of (1) and (2).

NOTE:	Backward compatibility is an important aspect of any recommendations & enhancements and will be considered during the study of this KI.

## 4.6	Key issue #6: Edge services support across ECSPs

An edge service or an EAS (e.g. V2X server) can be provided via different EDNs deployed by different ECSPs. Each ECSP may not have the required infrastructure to install the EAS in every EDN due to financial, regulatory and operation constraints. It is assumed that a UE can access the same edge service served by different EASs which are registered to different EESs and deployed by different ECSPs, which have a service level agreement to share edge services. These ECSPs can deploy EESs to serve different PLMNs or different coverages of the same PLMN. A typical scenario is depicted in Figure 4.6-1.

Figure 4.6-1 EAS deployed by different ECSPs

In Figure 4.6-1, the EAS2 resident in EDN1 and EDN2 provides the same service. The UE may be configured with the ECS1 configuration information (e.g., if the UE is a subscriber of ECSP_1). It is not clear how to provision the ECS2 configuration information, deployed by ECSP_2 (a partner of ECSP_1), to the UE when the UE is out of the service area of EAS2 in ECSP1 and cannot find a suitable EES within ECSP1 to discover and connect to EAS2.  The same issue exists when EAS2 becomes unavailable due to other reasons, e.g. overload, or in cases where ECSP_1 does not deploy EAS2 at all and relies on partner ECSP_2 to provide the edge service.

Besides, the UE may have already accessed the EAS2 in the EDN1 and is getting service from that EAS. In that case, it is not clear how to support service continuity due to UE mobility when the UE moves out of the service area of the EAS2 in EDN1 and goes to the service area of the EAS2 in EDN2.

Furthermore, the target EDN and source EDN are operated by different ECSP which may not have SLA with each other, then the S-EES may not be able to communicate with a T-EES (discovered from ECS) due to lack of SLA. Unfortunately, in Rel17 this failure may only be detect upon EDGE-9 interaction.

The following study is needed:

1.	Identify potential enhancements to the existing architecture defined in Rel-17 to enable inter-ECSP interactions.

2.	Study potential impact to support ECS discovery and service provisioning based on UE location.

3.	Whether and how EEC registers with an EES deployed by a partner ECSP?

4.	Study potential impact to support service continuity.

5.	How is EEC context continuity maintained across ECSPs with or without ACR?

6.	How the ECS can discover a T-EES having SLA with S-EES based on the federation agreements between ECSPs before EDGE-9 interaction?

## 4.7	Key issue #7: Application traffic filter exposure

3GPP TS 23.558 [2] has specified EDGE-3 exposure with different APIs. The session with QoS API provides the capability for the EAS to influence the QoS for the application traffic via EES. The ACR management event API supports "User plane path change", "ACR monitoring" and "ACR facilitation" and all events support to detect user plane path change for the application traffic.

The current definition of session with QoS API and ACR management event API only supports simple IP flow description. E.g. in 3GPP TS 23.558 [2], table 8.6.6.3.2-1 IP flow description identifies the application traffic by 3 or 5 tuples.

Table 8.6.6.3.2-1: Session with QoS create request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| UE IP address (NOTE 1) | O | The UE IP address. |
| UE ID (NOTE 1) | O | The identifier of the UE (i.e. GPSI) |
| UE Group ID (NOTE 1) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| IP flow description | M | The IP flow description for the application traffic. |
| Requested QoS reference (NOTE 2) | O | Refers to pre-defined QoS information for the data session between AC and EAS (NOTE 3). |
| … | … | … |

Only supporting IP flow description in EDGE-3 exposure APIs is not enough. EPS already supported the application traffic (e.g. encrypted application traffic) detection by more filters than IP flow description for the application (e.g. considering in Rel-14 study 3GPP TR 23.721 [7]) and more traffic filters are supported via PFD management procedure as described in 3GPP TS 23.682 [10] and 3GPP TS 23.203 [9], and such capability is also supported in 5GS in 3GPP TS 23.502 [8] and 3GPP TS 23.503 [11] correspondingly. The EEL should provide the same level of traffic filter for identifying the application traffic.

Open issues:

1.	How to support more application traffic filter for session with QoS API.

2.	How to support more application traffic filter for ACR management event API.

## 4.8	Key issue #8: EAS selection synchronization

Currently, EAS discovery may result in multiple EASs being discovered for a specific AC. The discovery request may trigger at the EES operations such as dynamic instantiation (3GPP TS 23.558 [2] clause 8.12). However, the EEC may select only one or some of the discovered EASs to enable AC communications or may not begin communications right after a discovery. For a discovered but unselected EAS, operations such as EAS instantiation or state change are unnecessary and inefficient,

From a different perspective, the registration procedure results in EEC context establishment at EES, with associated service session management. The EES process for determining EAS selection for service sessions is not specified, and there is no method to synchronize this information with the EEC. Moreover, following a registration the EEC does not have EAS endpoint information which could be leveraged for initiating service sessions, although the registration results in reservation of resources in the EDN.

In another case, many IoT devices are configured for bursty communications at large intervals of time, without service continuity requirements and with many semi-static (e.g. provisioned over-the-top) parameters. For example, a set of EASs may be maintained at the EEC with the expectation of providing signalling optimizations. However, this EEC information cannot be currently leveraged by the system as intended.

While basic edge functionality can be enabled assuming ideal implementations of an algorithm through which the EES determines that a registered EAS is providing services to an AC, inefficiencies in EAS capability use and enabling service session functionality immediately after registration remain. Hence, it is required to study:

1.	How to enable the EES to accurately determine the EAS(s) capabilities needed by EEC for service sessions in order to perform optimal EAS instantiation operations.

2.	Whether and how the service session communications between ACs and EASs can be enabled by the EEC as soon as the EDN capabilities are available after registration.

3.	Whether and how to enable the EES to leverage pre-existent EAS information at the EEC in order to enable service session communications efficiently for IoT devices.

## 4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering

In order to ensure efficient utilization of EDN resources for EAS deployment, it should be possible to have the proper number of EAS instances in the EDN to accommodate the load for applications. The dynamic EAS instantiation triggered by the EES is supported in release 17, but further details are not addressed. The EES may invoke EAS dynamic instantiation triggering to the EAS management system, e.g,, for considering the service load/capacity of EAS (e.g., number of service session); and for considering the EEC's requesting service characteristics (e.g., location, latency). In this regard, the followings need to be studied further.

Open issues:

1.	What kind of information can be acquired by edge enabling layer and utilized by an EES to decide to trigger dynamic EAS instantiation and which entities can provide such information to an EES

2.	Whether and how to support dynamic EAS termination triggering in order to enable dynamic scaling of EAS (i.e., scale in as needed).

Editor's note:	The aspects of the interaction between EES and EAS management system should be consulted with SA5.

## 4.10	Key issue #10: Support for roaming UEs

When a UE is roaming in VPLMN, EDN configuration information for edge computing service in VPLMNs may not be available at all ECS deployed in HPLMN (termed H-ECS. The EEC in the UE thus needs to obtain information for V-ECSs (ECS available in VPLMN) to obtain service provisioning information in VPLMN based on the business relationship between HPLMN operator, VPLMN operator, and related ECSP(s).

In one scenario, the EEC in the roaming UE needs to discover the availability of edge computing services via ECS(s) available in VPLMN.

In Rel-17, ECS discovery based on VPLMN ID addressed in clause 8.3.2 of 3GPP TS 23.558 [2] do not cover some cases, e.g. when there are multiple available ECSs via the VPLMN for the roaming UE. Therefore, it is required to study ECS discovery in VPLMN and subsequent service provisioning for all relevant deployment models.

Additionally, it is required to clarify how an EEC hosted in the roaming UE can be authenticated and authorized to access the edge computing services available in the VPLMN. The related requirement is described in GSMA OPG as follows: "Access of roaming subscribers to edge applications in the visited network shall be subject to authorisation by the subscriber's Home OP and the Visited OP". Note that EEC authentication/authorization in Rel-17 is not clarified in roaming situation. It is thus required to study the architectural support necessary for SA3-defined procedures for EEC authentication/ authorization in roaming scenarios.

The following aspects shall be studied to support roaming UEs:

1.	Roaming-related deployment scenarios (if any) to be supported, and which may be differentiated by: (i) the relationship between the ECS provider and the PLMN operators, (ii) whether connectivity to an ECS can be established in both involved PLMNs.

2.	How the EEC in the roaming UE knows the availability of ECS(s) and/or EES(s) and discovers them in the VPLMN?

3.	Whether and how edge computing service continuity is supported when transitioning between an HPLMN and VPLMN.

4.	How to support authentication and authorization for an EEC hosted in the roaming UE.

5.	Whether and how to support topology hiding on inter-PLMN/ECSP interfaces on the edge enabler layer.

## 4.11	Key issue #11: ACR between EAS and Cloud Application Server

When a UE moves to a new location, different EASs or Cloud Application Server (CAS) can be more suitable for serving the ACs in the UE. Such transitions can result from a non-mobility event also, requiring support to maintain the continuity of the service.

This key issue is to support service continuity for ACs in the UE to minimize service interruption while switching the application server between Edge and Cloud. To support service continuity, the application context is transferred between EAS and CAS.

Rel-17 Edge Computing work is limited to the service continuity between the EAS(s) and identified several scenarios for service continuity. Detailed study is required to enable service continuity between EAS and CAS, covering the following open issues:

1.	Whether and how to detect that ACR is required between EAS and CAS

2.	Whether and how to decide that ACR is required between EAS and CAS

3.	Whether and how to perform ACR between EAS and CAS

4.	Whether and how to perform post ACR actions

5.	Whether EEL is required on the cloud deployment and what are the potential impacts to the CAS architecture

6.	Whether and what are the impacts of CAS initiated ACR.

## 4.12	Key issue #12: EEL service differentiation

The service differentiation in Rel-17 is very general and not detailed (e.g. ECS use local policy to determine service provisioning).  Details to enable an ECSP to provide different service quality levels should be specified. Such as a principle based on the role of service consumer. For example, a premium user must get the nearest available edge, or there should be certain applications available only to the premium users and so on.

Open issues:

-	What service differentiation should be enabled by the EEL?

-	Which procedure should be enhanced to support service differentiation?

-	Which functional entity is responsible for defining or managing the service differentiation?

## 4.13	Key issue #13: Edge enabler layer support for EAS synchronization

It is possible for the ASP to provide EAS (with same service) in different EDNs. For example, a gaming service provider may have deployed game servers in different EDNs to serve its users. It is required for an EAS to synchronize the particular communication session among other EAS(s). In figure 4.13-1, consider:

1)	a multi-user gaming session-1 is active between 5 gaming users where 2 users are in EDN1 served by EAS-1X (ASP1), while other 3 users are in EDN2 served by EAS-2X (ASP1);

2)	at same time (when gaming session-1 is active), another gaming session-2 is also active between different set of users, where some users are in EDN2 served by EAS-2X (ASP1), while other users are in EDN3 served by EAS-3X (ASP1).

It is required for EAS(s) to synchronize among each other for specific communication session (e.g. multi-user game session) in order to serve the users properly. It is required to study how Edge enabler layer can support EAS from EDN1 to find other EAS(s) in other EDN(s) to synchronize with.

Figure 4.13-1 EAS (with same service) deployed by same ASP in different EDNs

Further, referring to 3GPP TS 23.558 Annex A.4, the application architecture supports SEAL application server functions and Application Enabler Server functions available at the edge. While SEAL application server functions can be made available as an EAS at the edge, it is also possible that certain SEAL application server functions are available either or both at the edge and at the cloud. When the server functions of an application are available both at the edge and at the cloud, there may be a need for interaction between the two corresponding application servers.

This key issue is to support interaction between the two application server functions deployed at the edge or both at the edge and at the cloud.

Open issues:

1)	Whether and how to enable EAS to find other EAS(s) with multi-user communication session to synchronize?

2)	Whether and how to enable EAS to find other interested EAS(s) with specific service to synchronize?

3)	Whether and how to enable EAS to discover and interact with another application server function deployed on the cloud for context synchronization?

4)	Whether and how edge enabler layer could provide support to EAS synchronization?

## 4.14	Key issue #14: Application traffic influence for initially selected EAS

The application traffic is between the AC and EAS and 3GPP CN provides the underlying connectity for the application traffic. The EEC starts with EAS discovery first in order to offer EAS(s) to AC to start communication with the initially selected EAS. Later on, if the EAS relocation criteria is met, the EAS relocation procedure happens, which consists of EAS discovery, application traffic influence, application context transfer and AC communication with the new EAS.

Currently, it is not clear in the specification how to influence the application traffic with best optimal user plane routing when initial EAS discovery is completed in the application layer.

Open issues:

-	Whether and how the EEL can be involved to influence the application traffic routing in AC communication with the initially selected EAS.

-	Whether and how EEL configuration information that may influence application traffic routing.

-	Which entities can configure or provide information for application traffic influence routing.

## 4.15	Key issue #15: Support of constrained devices for Edge

Energy efficiency requirements are relevant for battery driven low-power IoT devices. Those devices may require edge computing services in a local DN for low-latency and employ the SA6 specified edge enablers.

Clause 6.15 of 3GPP TS 22.261 [12] has provided requirements for energy efficiency for devices, for instance: the 5G system shall support UEs using small rechargeable and single coin cell batteries; and shall support mechanisms to improve battery life for a UE over what is possible in EPS.

In the existing EDGEAPP architecture as described in TS 23.558 [2], many procedures and services are defined for EEC to interaction with EES/ECS. It is needed to study what impact the support for constrained UE may have on EDGEAPP architecture and what procedure can be improved.

Open issues:

-	Whether there are any impacts on the EDGEAPP architecture for constrained UE.

-	Whether and how the existing EDGEAPP architecture and procedures, for constrained UE to network communication (i.e. EDGE-1 and EDGE-4), e.g. can be improved to reduce power consumption.

## 4.16	Key issue #16: support of NAT deployed within the edge data network

In operational deployments of cloud infrastructures including at the edge, Network Address Translation (NAT) are oftentimes deployed.

Such an operational constraint will prevent the EES to have an IP address of the UE that is known by the underlying 3GPP network.

Open issue:

-	How the EES can access 3GPP network services pertaining to a UE when the edge data network employs Network Address Translators (NATs).

-	How AF specific and temporary UE IDs can be managed at the Edge Enabler Layer?

Editor's Note: Whether EAS requires to be addressed in the key issue is FFS

## 4.17	Key issue #17: Discovery of a common EAS

An ASP can deploy several EASs providing the same service in different locations within the EDN.

For certain use cases involving real-time communication in a multi-user session, both between AC and EAS and between different ACs via the EAS, it may be necessary or beneficial to use services from a single common EAS to meet the strict latency requirements and to avoid the need for inter-EAS synchronization. The use cases may include, for example, a team of robots coordinating together on a manufacturing floor, a team of surgeons using VR headsets and robotic surgery equipment to operate together on a patient, or a group of trucks using V2X for platooning.

Dependent on the use case, the EEL may apply different additional criteria to determine this common EAS. E.g. it could be desirable to determine the EAS so that the latency for all the ACs in the session is approximately the same or that the latency for a specific AC is minimized.

Figure 4.17-1 Several EASs (with same service) deployed in different locations in the same EDN

Open issues:

1)	Whether and how the ACs/EECs of different users can select or be provisioned the same EAS within an EDN?

NOTE:	This open issue is dealing with the issue how different EECs can perform EAS discovery so that they select the same EAS within an EDN, whereas KI#13 is dealing with the issue how, after different EECs have selected different EASs located in different EDNs, these EASs can synchronize their contexts.

2)	Whether and how the ACs/EECs of different users can select or be provisioned a common EAS, even if initially the EECs are communicating with different EDNs?

3)	Whether and how the EEL can support service continuity to ensure that when ACs require the use of service from a common EAS and an ACR operation is needed, ACR operations can be coordinated so that upon completion of the ACR operations the ACs again have services provided by a common EAS.

## 4.18	Key issue #18: Linkage between EASs

Rel-17 EEL procedures are designed such that services like EAS discovery and service continuity support are performed per EAS, where individual EASs are uniquely identified using EAS endpoints (e.g., URI, FQDN, IP address).

However, to provide services to the end user a typical AC communicates with multiple endpoints i.e., multiple EASs. This creates a linkage between these EASs, which impacts the support provided by the Edge Enabler Layer. Taking an example of an online game where to support large number of users, different game functions are split across multiple servers; like, a game engine for game state and user input management, in-game chat server for communication between players and a capture server for capturing rendered images, encoding, and transporting them to the player's device. If each of these EASs are discovered, controlled, and relocated individually, it may impact the overall quality of service. For e.g. ACR failing for the game engine should cancel the ACR of the capture server to maintain their proximity.

This key issue is to study the impacts on the EEL's support functions created by such linkages or dependencies between the EASs.

NOTE:	This KI does not focus on enabling communication between the EASs, rather, it focuses on coordination at the EEL.

Open issue:

-	How can the EEL identify linkages between the EASs?

-	What are the impacts on EEL procedures due to linkage between the EASs e.g. when the linked EASs are served by the same EES and require ACR due to UE mobility?

## 4.19	Key issue #19: ACR scenario combination

Multiple ACR scenarios are specified in 3GPP TS 23.558[2] clause 8.8. Applications can utilize one or more ACR scenarios. Different combinations of utilizing ACR scenarios by Applications should be enabled by the Edge Enabler Layer (e.g. only one ACR scenario allowed or several ACR scenarios allowed).

Open issues:

-	Whether and how the EEL can support the determination of the ACR scenario for one AC?

## 4.20	Key issue #20: Method of supporting federated EAS service

In order for EAS to provide services (weather, transportation, maps, etc.) in partnership with other EASs, EAS context processing and federated EAS support may be required at edge-compatible layers. When ACR occurs due to UE mobility, a method of rearranging the federated EAS context may be required to provide continuous service of the federated EAS. In addition, there may be a need for a method for finding an EAS that provides a federated EAS service within the EDN in which the UE has moved.

Although EAS can discover and communicate other EAS APIs through CAPIF's functions, but for service continuity, it may be necessary to discover EASs providing federated EASs and relocation the context of EASs that provided federated capabilities.

NOTE:	This KI focuses on coordination at the EEL when the federated EAS provides services to the AC on a UE.

Open issues:

-	Whether and how the EEL can support to federated EAS context management.

-	Whether and how the EEL can support the relocation of the federated EAS context for service continuity.

-	Whether and how the EEL can discover EAS that provides the services of the federated EASs.

## 4.21	Key issue #21: Simultaneously EAS connectivity in ACR

In 3GPP TS 23.548 [19], there is a use case where application client needs to connect to both S-EAS and T-EAS during service continuity. Details are specified in clause 6.3.4 and Annex F of 3GPP TS 23.548 [19].

For AC triggered application context relocation, clause 8.8.2.2 of 3GPP TS 23.558 [19] describes the scenario for the service continuity initiated by EEC using regular EAS discovery. In step 5, the EEC and AC jointly decides the T-EAS to be used and in step 8 the AC is triggered by the EEC to start ACT.

Figure 4.21-1: ACR initiated by the EEC and ACs

Figure 4.21-1 illustrates one ACR scenario example in SA6 EDGEAPP. If the AC needs to connect to the T-EAS first to trigger ACT, the AC has two application sessions simultaneously during the service continuity. There could be more ACR scenarios applicable for this type of ACT. There are potential improvements in EEL to facilitate such simultaneous connectivity. For example, whether and how to influence the application traffic to maintain both S-PSA and T-PSA during the service continuity and the feature interaction between simultaneous EAS connectivity and service continuity. NOTE:	In Rel-17, EES can optimize the user plane routing before AC starts the application signalling with the T-EAS, but that does not consider the need for simultaneous EAS connectivity.

Open issues:

-	Whether and how EEL can influence the application traffic routing considering the need to maintain both S-PSA and T-PSA for simultaneous connectivity with both S-EAS and T-EAS, during the service continuity.-	What the feature interaction is for simultaneous EAS connectivity and service continuity (including planning).

## 4.22	Key issue #22: EAS discovery in Edge Node sharing scenario

Based on OPG.02 CR1001 Operator Platform Telco Edge Requirements, Edge Node sharing scenario has been identified in GSMA OPG.02 CR1001 Operator Platform Telco Edge Requirements clause 3.3.5.

The deployment case is as follows:

1.	OP B deploys application in the OP A (partner OP). OP B wants to scale its services for the region covered by OP A by using OP A's edge infrastructure.

2.	User belongs to the OP B.

3.	If OP B finds that the most suitable application that can serve the user is available in OP A (partner OP), then OP B requests the edge computing service from OP A (partner OP).

NOTE:	The user is referred to the subscribers who have edge service authorizations.

Based on the deployment case, it is not clear how to discover and determine the EAS(s) deployed in OP A for OP B users.

Figure 4.x-1 Edge Node sharing scenario

The following study is needed:

1.	How can EES discover and determine a EAS which allows (subscribers of OP B) to avail its services?

NOTE:	The key issue assumes OP A and OP B has the same Edge Computing Architecture (i.e. EDGEAPP).

## 4.x	Key issue #x: <title>

This clause describes the key issue with a suitable title. Please provide a high-level description of the key issue along with a list of open issues.

# 5	Architectural requirements

## 5.1	General requirements

### 5.1.1	General

This clause provides a basic description of the general architectural requirements.

### 5.1.2	Requirements

This clause lists the general requirements.

## 5.2	Enablement of Service APIs exposed by EAS

### 5.2.1	General

This clause specifies the requirements for EAS Service API enablement in the EDGEAPP architecture to address the KI#2 in the clause 4.2.

### 5.2.2	Requirements

The following are the architectural requirements to support EAS Service APIs in the EDGEAPP Rel-18 architecture.

1)	EAS capability exposure

-	The application layer architecture shall support exposure of EAS's capabilities to the other EASs.

2)	EAS Service API publication

-	The application layer architecture shall support EAS to publish its exposing Service API information to EES

-	The application layer architecture shall support EAS to update the published EAS Service API information on the EES.

-	The application layer architecture shall provide mechanisms for the EAS to publish and update KPIs of its Service APIs when available.

3)	EAS Service API discovery

-	The application layer architecture shall provide mechanisms for an EAS to discover available EAS Service APIs.

4)	Subscription service

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in dynamic information of EAS Service APIs from an EES.

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in availability of EAS Service APIs from an EES.

## 5.3	ECS discovery

### 5.3.1	General

This clause specifies the requirements for ECS discovery to address the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs in clause 4.10.

### 5.3.2	Requirements

The following is the architectural requirement to support ECS discovery in the EDGEAPP Rel-18 architecture.

-	The application layer architecture shall provide mechanisms for ECS to discover available ECS which may have suitable EES, to support UE mobility between ECSPs.

## 5.x	<specific requirements>

### 5.x.1	General

This clause provides a basic description of specific architectural requirements.

### 5.x.2	Requirements

This clause lists specific requirements.

# 6	Enhanced Application Architecture

## 6.1	Option #1: Roaming architecture

This clause describes the architecture for roaming UEs, addressing Key Issue #10.

### 6.1.1	Architecture enhancements

#### 6.1.1.1	Local breakout roaming architecture: Local breakout to access H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. In this architecture, the H-ECS is associated with HPLMN, while the V-ECS and the EDN which the UE accesses is associated with VPLMN. A new reference point EDGE-10 is defined between ECSs. Figure 8.1.2-1 shows the architecture for this model.

NOTE:	H-ECS and V-ECS can be provided by the same ECSP.

Figure 6.1.1.1-1: Local breakout roaming architecture: Local breakout to access H-ECS

#### 6.1.1.2	Home-routed EDGE-4 access to H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. Figure 6.1.1.2-1 shows the architecture for home routed roaming architecture for this model.

Figure 6.1.1.2-1: Home-routed access to H-ECS

In the HR roaming scenario, the roaming architecture is valid if the UE is supported to access the EDN in the VPLMN (i.e., the local access to the EDN of the VPLMN is supported in the HR roaming scenario). The traffic toward the EDN in the VPLMN (i.e., EDGE-1 traffic and application data traffic between AC and EAS) is not home routed to the HPLMN while the traffic between the EEC and H-ECS over is home routed via VPLMN and HPLMN.

Editor's note:	 Whether and how to support local access to the EDN in the VPLMN for a HR roaming UE is SA2's responsibility.

### 6.1.2	Identities

None.

### 6.1.3	Cardinality rules

None.

## 6.2	Option #2: Non-roaming architecture

This clause describes the architecture for non-roaming UEs.

### 6.2.1	Architecture enhancements

Compared with the Rel-17 EDGEAPP architecture for the non-roaming scenario, a new reference point EDGE-10 is defined between ECSs. Figure 6.2.1-1 shows the architecture for this model.

NOTE:	ECSs communicating via EDGE-10 may be provided by different ECSPs.

Figure 6.2.1-1: Non-roaming architecture

### 6.2.2	Identities

None.

### 6.2.3	Cardinality rules

None.

## 6.3	Option #3: Edge Notification Server architecture

This clause provides an enhanced application architecture based on Rel-17 architecture by incorporating an Edge Notification Server addressing Key Issue #1, "Enhanced notification service to the EEC".

### 6.3.1	Architecture enhancements

This clause describes the new Edge Notification Server (ENS) functional element and the new interfaces (i.e EDGE-11, EDGE-12 and EDGE-13) needed to enable interactions in between EEC-ENS, EES-ENS and ECS-ENS respectively.

Figure 6.3.1-1 illustrates the reference point representation of the architecture for Edge Enabling Application with the inclusion of the Edge Notification Server and the reference points EDGE-11, EDGE-12 and EDGE-13. The Figure also shows an OEM Push sever which is outside of the PLMN.

NOTE 1: The OEM Push sever and the Push Function in the UE, as shown in Figure 6.3.1-1, are outside the scope of this TR and SA6.

NOTE 2: The details of communication between the ENS and the OEM Push sever is outside the scope of this TR and SA6.

Figure 6.3.1-1: Enhanced architecture using an Edge Notification Server

Editor's Note: Whether ENS should be an optional EDGEAPP architectural enhancement is FFS.

Editor's Note: Whether ENS should be addressed as a generic function provided as part of SEAL is FFS.

#### 6.3.1.1	Edge Notification Server (ENS)

ENS is the central notification server which receives notifications from EES (EDGE-12) and ECS (EDGE-13) and based on the preferred notification delivery method indicated by EEC (over EDGE-11), delivers the notifications to EEC through either a Pull or a Push delivery method.

Functionalities of ENS are:

a)	Enabling EEC to request for a Callback URL to be used in its event subscription creation with EES and ECS;

b)	Enabling EEC to request for an optional Channel URL to receive notifications from the ENS directly (e.g. via Long-polling or WebSocket);

c)	Enabling EEC to request receiving notifications from the ENS indirectly through a preferred Push server (e.g. FCM, APNS, OMA Push);

d)	Setting up a Pull or Push notification channel with EEC based on EEC's preferred notification delivery method (e.g. Long-polling, WebSocket) requested;

e)	Receiving Notifications from EES and ECS at the CallBack URL and passing them onto the EEC either over a notification Channel (e.g. Long-polling, WebSocket) which is setup directly with the EEC or indirectly (i.e. an implicit notification channel) via a Push server (e.g. FCM, APNS, OMA Push);

#### 6.3.1.2	ENS Discovery

For a given EEC, the associated ENS's information (e.g. URI(s), FQDN(s), IP address(es)) and optionally the ENS Provider Identifier are obtained from the ECS as part of the initial provisioning activity (see clause 8.3 in 23.558).

This approach ensures regardless of how many EECs a UE has or how many ENSs are deployed in the network (e.g. one ENS per ECSP or a single ENS by MNO), a given EEC would always receive the appropriate ENS endpoint to communicate and open a notification channel with.

NOTE 1: Depending on the number of EECs (in the UE) and the number of associated ENS(s) in the network (i.e. one ENS per ECSP or a single ENS by MNO), an UE may simultaneously interact with multiple ENSs (one per ECSP) or a single ENS deployed by the MNO.

## 6.4	Option #4: Constrained devices with limited capabilities

### 6.4.1	Architecture enhancements

#### 6.4.1.1	General

This architecture option adds support for certain constrained devices which either don't have enough capabilities to execute its own EEC (e.g., the constrained device may not have a Mobile Termination entity) or do not execute its own EEC to save essential resources such as processing power and battery. Such constrained devices (e.g., terminal equipments as defined in TR 21.905) will benefit by being able to utilize services of an EEC running on a different UE, using EDGE-5.

#### 6.4.1.2	Architecture

Figure 6.4.1.2-1: Architecture supporting constrained devices

Editor's Note:	Whether this figure should be depicted as an architecture update or as a deployment option is FFS.

NOTE 1:	The architecture option should utilize existing functionalities specified by SA2 and avoid creating new requirements for system architecture.

NOTE 2:	How the TE communicates with the UE (e.g., Bluetooth, Wi-Fi, Cellular etc.) is out of scope of SA6.

### 6.4.2	Identities

None.

### 6.4.3	Cardinality rules

None.

## 6.5	Option #5: Architecture with Cloud Application Server (CAS)

This clause describes the architecture for enabling interactions between EAS and Cloud Application Server (CAS), addressing Key Issue #11.

### 6.5.1	Architecture enhancements

Figure 6.5.1-1 shows the architecture enabling interactions between EAS and CAS. Compared to the EDGEAPP (Rel-17) architecture, new entity Cloud Application Server is proposed along with the new reference points EDGE-14 (between EES and CAS), EDGE-15 (between ECS and CAS) and EDGE-16 (between 3GPP Core Network and CAS).

Figure 6.5.1-1: Architecture with Cloud Application Server (CAS)

In this solution, the Cloud Application Server (CAS) interaction with EES is supported via EDGE-14 reference point and CAS is supported by ECS via EDGE-15 reference point. The CAS and EAS interaction is supported as Application Data Traffic, which is out-of-scope of this specification. The CAS interaction with the 3GPP core network happens over EDGE-16 reference point, which is similar to EDGE-7 reference point.

Editor's note:	What functionalities of EDGE-9 and EDGE-6 are to be reused for EDGE-14 and EDGE-15 respectively is FFS.

Editor's Note:	It is FFS whether new Cloud Enabler Server (CES) entity is needed for interacting with CAS.

Since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to connect to either another EAS in new EDN or the CAS.

The architecture supports ACR between edge and cloud deployments for the following conditions:

-	Condition 1: For the locations where EDN is not available, the ACR support is based on the failed Service provisioning response (i.e. the non-availability of the EDN at a particular location) from the ECS.

-	Condition 2: When AC profiles are sent in Service provisioning request and particular EAS is not available then EDN non-availability for that EAS is inferred through Service provisioning response.

-	Condition 3: For the locations where EDN is available but the EAS is not available, the ACR support is based on the indication from the EES (in the EAS discovery response) about the non-availability of the EAS at that particular location.

-	Condition 4: EAS and EDN are available but EAS is overloaded or not in a position to serve the EEC/UE due to any reason.

-	Condition 5: For a UE location, when CAS is serving and a suitable EAS is available at the edge, ACR can be initiated for continuing the service delivery via EAS.

### 6.5.2	Identities

None.

### 6.5.3	Cardinality rules

None.

## 6.x	Option #x: <title>

This clause provides an enhanced application architecture based on Rel-17 architecture.

### 6.x.1	Architecture enhancements

This clause describes the new functional elements and interfaces proposed by this option.

### 6.x.2	Identities

This clause describes the new identities defined for the functional enhancements proposed by this option.

### 6.x.3	Cardinality rules

This clause describes the new cardinality rules defined for the functional enhancements proposed by this option.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # 6 | KI # 7 | KI # 8 | KI # 9 | KI # 10 | KI # 11 | KI # 12 | KI # 13 | KI # 14 | KI # 15 | KI # 16 | KI # 17 | KI # 18 | KI # 19 | KI # 20 | KI # 21 | KI # 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #8 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #11 |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #12 |  |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #16 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Sol #20 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #21 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| Sol #23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sol #24 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Sol #25 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Sol #26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| Sol #27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.1	Solution #1: Service provisioning via push notification

### 7.1.1	Architecture enhancements

None.

### 7.1.2	Solution description

#### 7.1.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC in clause 4.1.

In this solution, push notification mechanism is utilized to enhance the service provisioning procedure. It is assumed that at least one push server is available to ECS and the UE has push function supporting the interaction with the push server.

#### 7.1.2.2	Procedure

Pre-conditions:

1.	The UE supports push notification service and the associated push server can be accessed by the ECS.

2.	The address of push server is pre-configured in the Push function.

Figure 7.1.2.2-1: Service provisioning via push notification

1.	The EEC registers with the push function within the UE. The EEC acquires a push token and push server information from the push function.

NOTE:	The push server provides the push function in the UE with a push token, which is delivered to the EEC.

2.	The EEC sends a service provisioning subscribe request to the ECS. The service provisioning request includes push token, push server information (e.g. address) in addition to information elements in clause 8.3.3.3.4 of 3GPP TS 23.558 [2] v17.0.0. The push server address is included as Notification Target Address.

3.	Upon receiving the request, the ECS performs an authorization check as in clause 8.3.3.2.3.2 of 3GPP TS 23.558 [2] and further verify if push notification can be used. If the request is authorized and push server can be used for notification, the ECS creates and stores subscription resource for service provisioning.

4.	If the processing of the request was successful, the ECS responds with a service provisioning subscription response.

5.	If an event occurs at the ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription is for push notification, the ECS sends push notification request to the push server that is identified by the push server information provided by the EEC in the step 2. The push notification request sent from the ECS to the push server contains the push token, EEC information (e.g. identification or port ID) and service provisioning notification message.

6.	The push server sends the service provisioning notification message to the EEC via the push function, which in turn and delivers the notification message to the corresponding EEC.

### 7.1.3	Solution evaluation

The proposed solution addresses Key Issue #1. The solution is based on Rel-17 service provisioning subscribe procedure and the push notification mechanism supported in a UE. This approach does not require EEC to persistently keep the session with ECS for receiving notification message. Instead, the UE reuses the session kept by the push function and push notification server.

This solution does not introduce impact on Rel-17 architecture.

## 7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

### 7.2.1	Architecture enhancements

None.

### 7.2.2	Solution description

The EAS can provide the domain name to the EES as traffic descriptor and EES can further provision the domain name to the 3GPP CN. The following figure 7.2.2-1 depicts the solution sketch for the Session with QoS create operation with bold text showing the enhancement to the existing procedure in 3GPP TS 23.558 [2], clause 8.6.6.2. Such solution allows the EAS to provide fruitful traffic filters to be applied for the intended session with requested QoS.

Figure 7.2.2-1: Session with QoS API: create operation

In step 1, the EAS requests establishment of a data session between the AC and the EAS with a specific QoS, the EAS sends the domain name as traffic descriptor which also indicates the applicable protocol and matching criteria (e.g. TLS SNI). In step 2a, the EES invokes the PFD management procedure with the 3GPP CN as described in 3GPP TS 23.682 [10] and 3GPP TS 23.502 [8] with an application id that may be derived from the EASID according to local policy. Further the EES provides the same application id for requesting data session with specific QoS in step 2c or step 4.

NOTE:	PFD management is optionally supported in MNO, if EES cannot invoke step 2a, it responds EAS with appropriate error.

If the EAS provides only IP flow description as the traffic filter, the EES can provide the application id (which may be derived from the EASID) to the 3GPP CN when requesting data session with specific QoS and rely on further traffic classification mechanism in MNO to apply specific QoS for the application traffic. In that case, the EES skips the PFD management procedure in step 2a.

Similar approach can be applied for the ACR management event API for EAS to consume EES service.

The EAS can also send other specific filters (e.g. URI) to the EEL allowing a fruitful application traffic detection in the 3GPP CN, such filters will be used by the EES in the PFD management procedure in step 2a.

### 7.2.3	Solution evaluation

This solution address KI#7. It enhances the existing Session with QoS API and ACR management event API in 3GPP TS 23.558 [2] with support for traffic filters more than IP flow description.

## 7.3	Solution #3: Service provisioning triggering via SMS over NAS

### 7.3.1	Architecture enhancements

None.

### 7.3.2	Solution description

#### 7.3.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC.

In this solution, application triggering (device triggering) via SMS over NAS method specified in 3GPP TS 23.501 [5] is utilized to provide the updated EDN configuration information to the EEC.

The EEC indicates to the ECS that SMS over NAS is supported. Then, the ECS checks if the SMS over NAS can be utilized and perform EEC triggering by using SMS over NAS

#### 7.3.2.2	Procedure

Pre-conditions:

1.	The EEC is able to check if the UE supports the SMS over NAS;

2.	The EEC is able to decode the need to trigger the operation indicated in the SMS; and

3.	The ECS is allowed to use Nnef_Trigger_Delivery service.

Figure 7.3.2.2-1: EEC triggering via SMS over NAS to perform service provisioning

1.	The EEC determines that the UE supports the SMS over NAS by checking if the UE capability support the SMS delivery over NAS and if the UE is allowed by the core network to use SMS delivery over NAS (e.g., whether the UE is indicated by the core network that SMS over NAS is allowed during Registration procedure or UE Configuration Update procedure as specified in TS 23.502 [8]). If the SMS over NAS is supported and allowed in the UE, the EEC sends an indication of SMS over NAS supported to the ECS. This indication may be included in the service provisioning request message and sent to the ECS.

2.	The ECS acknowledges to the EEC by sending a message including whether the EEC triggering via SMS over NAS is available for service provisioning or not. When the EDN configuration information in the ECS is updated, the ECS triggers EEC to initiate service provisioning request by utilizing the SMS over NAS as described in the below figure 7.3.2.2-2.

Figure 7.3.2.2-2: EEC triggering via SMS over NAS to perform service provisioning

1.	The ECS determines to perform EEC triggering via SMS over NAS for service provisioning (e.g. when the EDN configuration information is updated).

2.	The ECS invokes Nnef_Trigger_Delivery service to send a trigger delivery request message to the NEF as described in clause 4.13.2 of 3GPP TS 23.502 [8]. The message includes indication to trigger service provisioning, EEC ID, UE ID, and ECS information (e.g. address).

3.	The NEF delivers the triggering information via MT SMS Delivery procedure as specified in TS 23.502 [8]. The indication to trigger service provisioning, EEC ID, ECS information are included as the triggering payload in the triggering information

4.	The NEF sends Nnef_Trigger_Delivery service response message to the ECS to inform that the ECS-provided information in step 2 is successfully delivered to the SMS-SC as in clause 4.13.2 of 3GPP TS 23.502 [8].

5.	The NEF sends Nnef_Trigger_Delivery Nofity message to the ECS to inform that the SMS message is successfully delivered to the UE as in clause 4.13.2 of 3GPP TS 23.502 [8].

6.	The UE receives the SMS message containing the ECS-provided triggering information (indication to trigger service provisioning, EEC ID, ECS information) from the 5GC via Mobile Terminated SMS Delivery as in clause 4.13.3 of 3GPP TS 23.502 [8] and the EEC is triggered to perform service provisioning request toward the ECS identified by the ECS information contained in the SMS message.

7.	The EEC may send a service provisioning request to the ECS identified by the triggering information contained in the SMS.

Editor's note:	It is FFS if this solution conforms to the notion of a notification service to the EEC.

### 7.3.3	Solution evaluation

TBD.

## 7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs

### 7.4.1	Architecture enhancements

For a roaming scenario, Option #1 in the clause 6.1 is the basis for this solution.7.4.2	Solution description

#### 7.4.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES based on UE location and provides it to the requesting EEC or EES.

In a roaming scenario, after a UE selects a PLMN and performs the registration procedure, the EEC gets ECS information to use edge computing service in the VPLMN as follows:

-	The EEC may have been pre-configured or provisioned with V-ECS information.

-	The EEC may try to send a service provisioning request message including service PLMN ID to the H-ECS and retrieves V-ECS information or, if possible, EDN configuration information for VPLMN. The EEC may perform service provisioning procedure with the V-ECS if the H-ECS provides V-ECS information to the EEC as show in figure 7.4.2.2-1.

-	The EEC may try to derive V-ECS address information from the VPLMN identifier. For example, an EEC can combine the VPLMN identifier and an ECSP code (or ECSP ID) assigned to an edge computing service provider (e.g. "ECS <ECSP-ID/CODE>.ECS.MNC <MNC> .mcc <MCC> .3gppNetwork.org ").

Editor's note:	It is FFS whether and how the precedence among the above method for ECS discovery is defined.

#### 7.4.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC or source EES, the ECS1 discovers another ECS2 which may have suitable EES and responds with the ECS2 information. After that, the EEC or source EES sends the request of EES to the ECS2.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2indicate the H-ECS and the V-ECS, respectively).

Figure 7.4.2.2-1: Solution X for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers potential ECSs which may have suitable EES based on the information such as the UE location.

The ECS1 may determine a potential ECS information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16].

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the potential ECSs, e.g. ECS2 which have suitable EES based on the serving PLMN ID. Optionally, the ECS1 may send a request to ECS2 to verify whether the ECS2 is available. If yes, ECS2 returns the success response to the ECS1. Otherwise, a failure response/code is returned.

Editor's note:	In the roaming scenario, it is FFS how an H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the related roaming agreement on edge computing service.

Editor's note:	It is FFS whether and how ECS1 filters the list of the discovered ECSs.

3. ECS1 sends a response message including the list of ECSs information and failure cause indicating redirection to another ECS (for service provisioning request or retrieve EES request) to the requester of step 1 if the ECS1 cannot discover a EES:

3a.	In response to the request in step 1a, the ECS1 sends the response message to the EEC.

3b.	In response to the request in step 1b, the ECS1 sends the response message to the S-EES.

4.	The EEC or source EES can send the request (e.g. resends a service provisioning request or retrieve EES request) to one of the ECSs received from the ECS1 e.g. the ECS2. Correspondingly, the EEC or source EES receives response from the ECS2.

### 7.4.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

Solution 4 solves the problems of KI#10 and KI#6. After discovering another ECS2 which may have suitable EES, the ECS1 sends respond to the EEC or source EES with the ECS2 information. Then the EEC or source EES can send the request to the ECS2 directly to retrieve suitable EES. It is possible that some enhancements on ECS are needed to support ECS discovery.

Editor's note:	How the EDGEAPP architecture works in different roaming scenarios is FFS.

## 7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs

### 7.5.1	Architecture enhancements

A new reference point is required between ECSs, which is described as EDGE-10 in the clause 6.1.1.1.

For a roaming scenario, Option #1 in the clause 6.1 is the basis for this solution.

### 7.5.2	Solution description

#### 7.5.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs in clause 4.10.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2.

#### 7.5.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC / source EES, the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2. The ECS1 then provides the EES information to the EEC / source EES in the response.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2 indicate the H-ECS and the V-ECS respectively).

Figure 7.5.2.2-1: Solution X for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the Retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable T-EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES based on the information such as the UE location. The ECS1 may determine the ECS2 information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16].

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the ECS2 which have suitable EES based on the serving PLMN ID.

Editor's note:	In the roaming scenario, it is FFS how an H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the related roaming agreement on edge computing service.

3.	The ECS1 further discovers a target EES whose service area can cover the UE location via the ECS2. The ECS1 may interact with the ECS2 directly or indirectly via e.g. a Federation manager.

4.	ECS sends the response to the requester of step 1:

4a.	In response to the request in step 1a, ECS1 sends the EES information to the EEC.

4b. In response to the request in step 1b, ECS1 sends the Retrieve EES response to the requesting EES (EES1).

### 7.5.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

This solution addresses KI#6 and KI#10. The EEC or the EES gets the requested EES information from ECS2 via ECS1. It is possible that some enhancements on ECS are needed to support service provisioning and EES retrieval request across the ECSPs.

Editor's note:	How the EDGEAPP architecture works in different roaming scenarios is FFS.

## 7.6	Solution #6: ACR update in service continuity planning

### 7.6.1	Architecture enhancements

Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 7.6.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 7.6.1-1: high level illustration of proposed service continuity planning enhancements

### 7.6.2	Solution description

The solution addresses the key issue #3 and the open issue on how to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility.

The solution can have two different solution variants based on the type of service continuity planning update. One possible solution is to modify the ACR service after the launch (described in 7.6.2.1) and another possible solution is the pause of the ACR services for a pre-defined time or till the detection entity decides to resume the ACR.

Editor's note:	It is FFS to capture the ACR pause solution variant and whether this can be a standalone procedure or incorporated within the ACR modification procedure.

#### 7.6.2.1	ACR modification solution

##### 7.6.2.1.1	EEC-based ACR modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.1-1: EEC-based ACR modification procedure

1.	The EEC detects a change of the expected UE behaviour. In particular, it may receive from the AC (over EDGE-5) an indication that the expected/predicted UE location and/or mobility changed. Such message can include the current and new expected/predicted location or current and new expected mobility/speed/direction/velocity, and/or an expected change of the UE route/trajectory.

2.	The EEC identifies that one or more ACR updates are needed based on the change of the UE behaviour and decides the type of the ACR update to be an ACR modification.

3.	The EEC sends an ACR modification request to the S-EES or T-EES (for EEC executed ACR via T-EES scenario) to indicate an ACR modification and to provide the updated parameters, such as the expected completion time. The request also includes the necessary parameters (e.g. IDs) to indicate the ACR that is requested to be updated.

4.	S-EES or T-EES determines the ACR to be modified based on the request in step 3. The ACR modification, is executed by the execution entity as described in clause 7.6.2.1.3.

5.  The S-EES (or T-EES for EEC executed ACR via T-EES scenario) sends an ACR modification response to the EEC to notify on the result.

6.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR modification result.

##### 7.6.2.1.2	EES-based modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.2-1: S-EES-based ACR modification procedure

1.	The S-EES detects a change of the expected UE behaviour. In particular, S-EES acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that S-EES has subscribed to consume 5GC services like LCS or NEF monitoring events). Such UE location report or monitoring event report may help indicating that the UE is not going to be at the predicted location at the given time and is expected to deviate by the original planning.

2.	The S-EES identifies that one or more ACR updates are needed based on the information on the change of the UE behaviour. The S-EES then decides for each ACR, the type of the ACR update to be an ACR modification and the parameters that need to be updated, such as the expected completion time.

3.	S-EES or T-EES determines the ACR(s) to be modified based on the decision in step 2. For each ACR identified to be updated in step 2, the ACR modification is executed by the execution entity as described in clause 7.6.2.1.3.

4.  The S-EES sends an ACR modification notification to EEC to notify on the result.

5.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR that is modified.

##### 7.6.2.1.3	ACR modification execution procedure

Figure 7.6.2.1.3-1 illustrates the ACR modification execution procedure as used in the procedures in 7.6.2.1.1 and 7.6.2.1.2.

Pre-condition:

1.	An ACR has been launched between EEC, S-EES, S-EAS, T-EES and S-EAS, where S-EES and T-EES can be the same or different; and one of the following preconditions holds:

1.1	The S-EES or T-EES has received an ACR modification request from EEC, e.g. according to the step 3 of the EEC-based ACR modification procedure in solution 6 in clause 7.6.2.1.1; or

1.2	The S-EES has decided an ACR modification, e.g. according to step 3 of EES-based modification procedure in solution 6 in clause 7.6.2.1.2.

Figure 7.6.2.1.3-1: ACR modification execution procedure

1.	If the EAS has subscribed to receive ACR notify for modification, the EES shall notify the EAS about the need to update the ACR. The EES shall include the updated parameters in the notification to the EAS.

Depending on the ACR scenario, the EES can be either of S-EES or T-EES. If it is S-EES, the EAS will be S-EAS. Otherwise if the EES is T-EES, the EAS will be T-EAS.

2.	The EES processes the updated parameters. The EES sends the updated parameters to the remote EES, if the updated parameter can be used by the remote EES to handle the messages from the corresponding EAS, e.g. expected completion time if provided in step 3 of clause 7.6.2.1.1 for the waiting time for the ACR status update.

When EES from step 1 is S-EES the remote EES will be the T-EES. Otherwise if the EES is T-EES, the remote EES will be S-EES.

3.	In response to the message in step 2, the remote EES responds and confirms the reception and acceptance of the updated parameters.

Editor's Note: It is FFS whether there is a need to update EEC context due to ACR modification.

4.	If in step 1 the S-EAS receives the ACR notify for modification, it may initiate ACT update process with the T-EAS. Otherwise if in step 1, T-EAS receives the ACR notify for modification, it may initiate the ACT update process. S-EAS and T-EAS may perform step 4 in an application specific time and manner any time after the notification in step 1.

### 7.6.3	Solution evaluation

This solution provides the procedures to update the parameters of an ACR in service continuity planning. It addresses the third open issue of the KI#3, to handle scenarios when the ACR needs to be modified, e.g. due to UE mobility. The solution does not require enhancement to the existing architecture in Release 17.

## 7.7	Solution #7: EES monitors UE mobility for service continuity planning

### 7.7.1	Architecture enhancements

None.

### 7.7.2	Solution description

#### 7.7.2.1	General

The following solution corresponds to the key issue#3 on enhancements to service continuity planning in clause 4.3

#### 7.7.2.2	Procedure

In this solution, the EES is responsible to identify the ACR type in ACR detection part or receive the ACR type from the EAS and the EEC. When application context transmission is complete, the EAS will send ACR status update message to the EES, once EES detects that UE moves to the predicted/expected location, the EES will notify EEC ACR complete message, then the EEC can be aware of  the compeletion of ACR. With this solution the EAS will not need to monitor the UE mobility and determine when to send the ACR status update message to the EES.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.5, the following differences are captured below:

2.	The detection entities (EEC, S-EAS, S-EES) detect the ACR may be required and identify the ACID and related ACR type (normal ACR or service continuity planning).

3.	If the EEC or S-EAS detect the ACR event, the EEC or S-EAS should inform S-EES with ACID, ACR type and predicted/expected UE location or EAS service area in the ACR launching procedure.

10. when S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

11. Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.3, the following differences are captured below:

4.	The EEC performs ACR lauching procedure indicating S-EES with ACR type (normal ACR or service continuity planning) and predicted/expected UE location or EAS service area.7.	When S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

9. 	Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC indicating that UE has moved to the predicted location.

Enhancements to 3GPP TS 23.558 clause 8.8.3.4	ACR launching procedure

Figure 8.8.3.4-1 illustrates the ACR launching procedure by the EEC or the S-EAS. If this procedure is triggered by the EEC, depending on the ACR action indicated in the ACR request, the procedure is used for either ACR initiation or ACR determination. If this procedure is triggered by the S-EAS, the procedure is used for ACR determination.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES as specified in clause 8.11, if the procedure is triggered by the EEC; and

2.	Information related to the S-EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.4-1: ACR launching procedure

1.	The EEC or the S-EAS sends an ACR request message to the EES in order to start ACR. The ACR request message includes ACR type to indicate whether the ACR is for normal ACR or service continuity planning. The ACR request message includes Predicted/Expected UE location or EAS service area to indicate that the EES should detect whether the UE has moves to the Predicted/Expected UE location or EAS service area or not in ACR clean-up phase. The ACR request message includes ACR action to indicate either ACR initiation request or ACR determination request. If the procedure is triggered by the S-EAS, the ACR request message is only for ACR determination.

An ACR request for ACR initiation:

-	includes an indication of whether the EEC requests the EES to perform EAS notification; and

-	provides information used by EES to perform AF traffic influence as in 3GPP TS 23 501 [2].

An ACR request for ACR determination informs the EES that the need for ACR has been detected at EEC.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

If the request in step 1 is for ACR initiation:

-	the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in 3GPP TS 23.501 [2], clause 5.6.7.1; and

-	if the EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the need to start ACR.

If the request in step 1 is for ACR determination, the EES decides to execute ACR as described in clause 8.8.2.5.

3.	The EES responds to the requestor's request with an ACR response message.

Enhancements to 3GPP TS 23.558 clause 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent from the EEC either to the S-EES or T-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE identifier (NOTE 4) | O | The identifier of the UE (i.e. GPSI). |
| ACR type | M | Indicates whether the ACR is for normal ACR or service continuity planning |
| Predicted/Expected UE location or EAS service area (NOTE 5) | O | The predicted/expected location information of the UE. The UE location is described in clause 7.3.2 or the predicted/expected EAS service area as described in clause 7.3.3.3 |
| Prediction expiration time (NOTE 6) | O | The time the UE reaches the Predicted/Expected UE location or EAS service area at the latest |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: This IE shall be present if the request originates from the EEC. NOTE 5: This IE shall be present if the ACR type indicates the ACR procedure is for service continuity planning. NOTE 6: This IE may be present if the request is from EEC and the ACR type indicates the ACR procedure is for service continuity planning. |  |  |

Editor's note: It is FFS whether "Prediction expiration time" and "expected completion time" mentioned in Solution #6 clause 7.6 are the same IEs.

### 7.7.3	Solution evaluation

This clause provides an evaluation of the solution.

This solution address KI#3. It enhances the existing service continuity procedure in 3GPP TS 23.558 [2] with support for the EES monitoring UE mobility for service continuity planning. With this solution, the EAS will not need to monitor the UE mobility and determine when to send the ACR status update message to the EES.

Editor's note: It is FFS for the handling when UE doesn't move to the location monitored by EES

## 7.8	Solution #8: EAS Service API enablement using CAPIF

### 7.8.1	Architecture enhancements

None.

### 7.8.2	Solution description

#### 7.8.2.1	General

This solution addresses the Key issue #2: Enablement of Service APIs exposed by EAS as specified in the clause 4.2 by supporting for an EAS to expose its Service APIs towards the other EASs.

As specified in TS 23.558 (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES (clause 8.6 of TS 23.558); the capabilities provided by the 3GPP core network (clause 8.7 of TS 23.558); and SEAL service APIs (clause A.4 of TS 23.558).

In this solution, the Edge Enabler Layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs in order to fulfil the architectural requirements specified in the clause 5.2. This solution exploits CAPIF specified in 3GPP TS 23.222 [16] to support publication/discovery, and change subscription of EAS Service APIs as studied in Sol#15 of TR 23.758 (Rel-17) with the following architectural assumptions within the CAPIF architecture:

-	An EAS may act as an API provider by implementing API provider domain functions (i.e., API exposing function, API publishing function, and API management function)

-	An EAS may act as an API invoker

-	An EES may act as a CAPIF provider by implementing CAPIF core function (CCF)

Note:

-	EES provides support for the logging and audit of EAS API invocation as described in 3GPP TS 23.222[16], clause 8.19 and clause 8.22.

-	EES control the service API access as specified in clause 8.12 of 3GPP TS 23.222 [16].

Based on the architectural assumption above, the essential operations regarding EAS Service APIs complying with CAPIF are as follows:

-	An EAS (acting as API provider) may publish its EAS Service APIs to EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may discover EAS Service APIs from EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may subscribe to be notified of dynamic information or availability of EAS Service APIs from EES (acting as CAPIF provider)

-	As EAS1(acting as API invoker) discovers EAS2 (acting as API provider) from EES, it can invoke the Service APIs directly from EAS2

#### 7.8.2.2	CAPIF operations in Edge Enabler Layer

The Figure 7.8.2.2-1 depicts the essential operational steps for EAS Service API enablement using the CAPIF operations as shown in Annex A of TS 23.222 [16].

Pre-conditions:

1.	The EAS #A-1 and EAS #A-2 have completed the EAS registration with the EES #A.

Figure 7.8.2.2-1: CAPIF operations in Edge Enabler Layer for EAS Service API enablement

1.	The EAS #A-2 (AMF) registers its API provider domain functions to the EES #A (CCF) via CAPIF-3.

2.	The EAS #A-2 (APF) publishes its exposing Service API(s) to the EES #A (CCF) via CAPIF-4.

3.	The EAS #A-1 (API invoker) performs onboarding process with the EES #A (CCF) via CAPIF-1.

4.	The EAS #A-1 (API invoker) discovers from the EES #A (CCF) a Service API required to run via CAPIF-1.

5.	The EAS #A-1 (API invoker) invokes the Service API provided by EAS #A-2 (AEF) via CAPIF-2 as discovered from the EES #A (CCF).

6. 	The EAS #A-1 (API invoker) subscribes to notifications of any updates of target Service APIs on the EES #A (CCF) via CAPIF-1.

7.	The EES #A (CCF) and EES #B (CCF) inter-operate with each other via CAPIF-6 for interconnection operations for publication and discovery of Service APIs managed by each EES.

#### 7.8.2.3	Service KPIs in CAPIF for EAS Service APIs

In TS 23.558, "Service KPI" IEs are specified to provide information about service characteristics provided by EASs; or required by ACs. This is used for discovery or provisioning of EASs which meet the Service KPIs required by ACs.

In the similar manner, Service KPIs of EASs (as API Providers) need to be specified in CAPIF to be used for discovery or provisioning of EAS Service APIs which meet the Service KPIs required by EASs (as API Invokers).

The proposed IEs in CAPIF as specified in 3GPP TS 23.222 [16] related to Service KPIs can be summarized as follows:

1)	Service KPIs provided by EAS as API Provider

a.	Service API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API; can be mapped to EAS Service KPIs in EAS Profile [TS 23.558] of the EAS providing the Service API

2)	Service KPIs required by EAS as API Invoker

a.	Onboard API invoker response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API which is allowed to access

b.	Service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs required by the API invoker

c.	Service API discover response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API corresponding to the discovery request

3)	Service KPIs for CAPIF interconnection between EESs

a.	Interconnection API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API which is published across CCFs (i.e., implemented in EESs)

b.	Interconnection service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs across CCFs (i.e., implemented in EESs)

The relevant information elements of CAPIF are listed as follows. The highlighted with bold text is proposed to add for supporting the Service KPIs.

Table 7.8.2.3-1: Service API publish request (Table 8.3.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

Table 7.8.2.3-2: Onboard API invoker response (Table 8.1.2.2-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the API invoker is granted permission otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs |
| Service API information | O (see NOTE 2) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

Table 7.8.2.3-3: Service API discover request (Table 8.7.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), interfaces, protocols, (new) Service KPI) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

Table 7.8.2.3-4: Service API discover response (Table 8.7.2.2-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including API description such as service API name, service API type, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, (new) Service KPI |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |

Table 7.8.2.3-5: Interconnection API publish request (Table 8.25.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O (see NOTE 2) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE 1: At least one of the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

Table 7.8.2.3-6: Interconnection service API discover request (Table 8.25.2.3-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | Identity information of the CAPIF core function discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs or CAPIF core function (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), interfaces, protocols, service API category, (new) Service KPI) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

In TS 23.222 [16], the Service KPIs can be specified as a part of Query information and Service API information in abstract, so that the Service KPI IE can be implemented as one or more new attributes of ServiceAPIDescription data type in TS 29.222 (but it's up to the CT3's work).

### 7.8.3	Solution evaluation

This solution allows for an EAS to expose its Service APIs towards the other EASs. It exploits CAPIF as specified in 3GPP TS 23.222 [16] for publication/discovery, and change subscription of EAS Service APIs.

This solution addresses the key issue #2: enablement of service APIs exposed by EAS as specified in the clause 4.2; and fulfils the architectural requirements for enablement of service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended capabilities of EAS and EES for publication/discovery of EAS Service APIs but there is no impact to the APIs and procedures of EDGAPP since the extended capabilities can be realized using CAPIF.

This solution relies on the CAPIF as specified in TS 23.222 [16] with some updates on the information elements of the CAPIF APIs for publish/discovery.

## 7.9	Solution #9: Application traffic influence trigger from EAS

### 7.9.1	Architecture enhancements

None.

### 7.9.2	Solution description

#### 7.9.2.1	General

This solution addresses key issue 14. An EAS can explicitly request EES including necessary information to influence the EAS traffic from UE(s). Then the EES can trigger the AF request to influence traffic routing towards the 3GPP CN for one or more UE(s) accessing the EAS.

#### 7.9.2.2	procedure

Figure 7.9.2.2:  AF influence the traffic for EAS

1.	The EAS sends an EAS traffic influence request, the request consists of necessary parameters such as N6 Traffic Routing requirements, the Target UE Identifier(s), Temporal Validity Condition.

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence.

3.	Upon successful authorization, the EES sends the traffic influence request to 5GC to influence the traffic for EAS as described in 3GPP TS 23.501, clause 5.6.7.1.

4.	The EES sends the EAS traffic influence response.

Editor's note: The information flow details corresponding to step 1 and 4 is FFS.

## 7.10	Solution #10: low power mode support

### 7.10.1	Architecture enhancements

None.

### 7.10.2	Solution description

#### 7.10.2.1	General

The following solution corresponds to the key issue #15 on supporting of constrained devices for Edge.

For the constrained UEs, there are two aspects should be considered to reduce the power consumption, one is help UE turn into idle mode or DRX mode when there is no traffic transmitting between UE and network, another one is help to reduce the traffic data rate when UE is connected to the network with transmitting packets.

EEC can get the knowledge about the constrained UE. When the applications are running with high power consumption, or the UE decides that the battery level is low and should start power saving, the EEC should send messages to EES with carrying such indication for low-power consumption requirement and specific application identifier.

When EES gets low-power consumption indication from EEC, the EES should trigger two methods to satisfy the constrained UE requirement.

Method1: EES should send the message to corresponding EAS through EDGE-3 that the data rate should be reduced, so that the downlink data will be reduced and UE do not need to process high-bandwidth traffic. How to balance the low-power consumption requirement and the UE experience is based on the EAS implementation.

Method2: EES should send the UE's low-power consumption indication to 3GPP core network. The 3GPP system may trigger some specific policies or treatment for such applications and UEs e.g. changing the AM policies e.g. update UE-AMBR, UE- slice MBR, RFSP index and/or service area restriction, for those UEs as specified in TS 23.502 clause 4.15.6.9, and changing the SM policies e.g. downgrade the QoS as specified in TS 23.502 clause 4.15.6.6.

Editor's note: It is FFS whether AM policy change can satisfy the need for all applications in the UE.

#### 7.10.2.2	Procedure

Pre-condition:

1.	The AC-EAS cannot handle the negotiation at the application layer.

Figure 7.10.2.2-1: low power mode support

1.	The EEC sends low-power consumption indication to EES with UE Identifier, EEC ID, AC ID, low-power consumption indicator, which indicating a constrained UE, and triggers the low power consumption enhancement.

2a.	When EES gets the low-power consumption indication, the EES may send low power indication to EAS to request corresponding EAS to reduce the downlink packets, which can help constrained UE process the application with constrained power consumption. The request could provide the UE Identifier(s).

2b. When EES gets the low-power consumption indication, the EES may request 3GPP core network to update the UE policy or QoS policy to support constrained UE, e.g. EES could act as an AF to set up a session with required QoS procedure as specified in TS 23.502 clause 4.15.6.6.

Editor's note: How much benefit does QoS downgrade have for energy saving is FFS.

3a. The EAS responds to EES to indicate the low-power processing is successful or failure.

3b. the 3GPP core network responds to EES with successful or failure.

4. EES response to EEC.

### 7.10.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.11	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF

### 7.11.1	Architecture enhancements

None.

### 7.11.2	Solution description

#### 7.11.2.1	General

This solution addresses "KI#5: Alignment of EDGEAPP and ETSI MEC" in alignment with ETSI MEC using CAPIF for exposing and invoking Service APIs. It specifies a CAPIF deployment option for:

1.	allowing EASs to invoke MEC services, and

2.	exposing Service APIs provided by EES or EAS (as described in KI#2) to MEC Applications

The following architecture is proposed to use CAPIF for alignment with ETSI MEC in terms of exposing EAS|EES Service APIs and invoking MEC Services as a deployment option.

Based on the proposed deployment option, the alignment of EDGEAPP and ETSI MEC could be done by mapping or complementing their own APIs to CAPIF APIs, especially for invoking/exposing Service APIs and MEC Services.

Note that Figure 7.11.2.1-1 below is to illustrate what EDGEAPP and ETSI MEC could look like when they exploit CAPIF for invoking and exposing their APIs. There is no such architecture defined in EDGEAPP or ETSI MEC as shown in Figure 7.11.2.1-1.

Figure 7.11.2.1-1: A deployment option for alignment with ETSI MEC using CAPIF

The proposed deployment option can be realized with the following implementations:

1)	3GPP EDGEAPP

a.	EES may implement CAPIF Core Function to manage the Service APIs exposed by EAS and EES;

b.	EES may implement CAPIF Core Function to interconnect with CCF of ETSI MEC via CAPIF-6

c.	EAS may implement CAPIF API Provider functions in order to expose EAS Service APIs to EDGEAPP EASs or ETSI MEC Applications

d.	EAS may implement CAPIF API Invoker function to invoke EAS Service APIs or ETSI MEC Services

e.	EAS may invoke EES Service APIs via EDGE-3

f.	EES may implement CAPIF API Provider functions in order to expose EES Service APIs (e.g. UE location API, AC information exposure API, etc.) to API invokers (e.g. EASs, ETSI MEC Applications)

-	i.e. EES Service APIs (as well as EAS Service APIs) need to be published to CAPIF Core Function

2)	ETSI MEC (out of the SA6 scope)

a.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to manage the MEC Services exposed by MEP and MEC App.

b.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to interconnect with CCF of EDGEAPP EES via CAPIF-6

c.	ETSI MEC (e.g. MEP, MEC App.) may implement CAPIF API Provider functions in order to enable EDGEAPP EASs to invoke MEC Services via CAPIF-2

d.	ETSI MEC (e.g. MEC App.) may implement CAPIF API Invoker function to invoke EDGEAPP EAS|EES Service APIs

#### 7.11.2.2	Procedure

(TBD)

### 7.11.3	Solution evaluation

(TBD)

## 7.12	Solution #12: Service continuity planning allowance

### 7.12.1	Architecture enhancements

TBD.

### 7.12.2	Solution description

#### 7.12.2.1	General

This solution addresses the key issue #3 on enhancements to service continuity planning and the key issue #12 on EEL service differentiation.

In this solution, the EEC provides its capability to perform service continuity planning and the EES can provide information about allowance of the service continuity planning.

#### 7.12.2.2	Procedure

Pre-conditions:

1.	EEC knows whether the planned or predicted UE mobility behaviour is available.

Figure 7.12.2.2-1: Service continuity planning allowance during EEC Registration procedure

1.	The EEC provides EEC Service Continuity Support or indication to request service continuity planning allowance when sending EEC Registration request. The request message may contain the capability of EEC to detect and provide information about planned or predicted UE mobility behaviour (e.g. including whether the UE moves to the expected/predicted location or not).

Editor's note:	It is FFS whether the EEC can provide the EES with its capability to obtain and provide information about planned, projected, or anticipated UE mobility behaviour.

2.	If the EEC Registration request message includes EEC Service Continuity Support or indication to request service continuity planning allowance, the EES verifies if the service continuity planning is allowed or not for the EEC based on the ECSP policy or the request from the EAS provider. If the EEC indicates to the EES in step1 that it cannot detect information about predicted UE mobility behaviour, then the EES monitors predicted UE mobility (e.g. including whether the UE moves to the expected/predicted location or not for service continuity planning.).

If AC profile(s) is provided in step 1, the EES may verify whether the service continuity planning is allowed or not for each EAS(s) matched with the AC profile(s) based on the ECSP policy or the request from the EAS provider.

Editor's note:	It is FFS how the EES verifies if the service continuity planning is allowed.

3.	The EES sends EEC Registration response including indication on whether service continuity planning is allowed or not.

If the EEC receives the indication that service continuity planning is allowed in step 3, the EEC detects or acquires the planned or predicted UE mobility behaviour. If the EES verifies and indicates the service continuity planning allowance per application to the EEC in step 2-3, the EEC performs the required detection or acquisition of the planned or predicted UE mobility behaviour for the EASs allowed for service continuity planning.

If the EEC is indicated that service continuity planning is not allowed, the EEC does not issue the application context relocation request for service continuity planning.

NOTE:	If the EEC is indicated that service continuity planning is not allowed, the EEC does not perform detecting or acquiring the planned or predicted UE mobility behaviour for the purpose of service continuity planning.

Editor's note:	It is FFS whether response message contains that the EES will monitoring whether the UE moves to the expected/predicted location or not, when the EEC cannot detect UE mobility.

### 7.12.3	Solution evaluation

TBD

Editor's note:	Potential backward compatibility issue should be evaluated for this solution

## 7.13	Solution #13: Update ECS configuration information

### 7.13.1	Architecture enhancements

None.

### 7.13.2	Solution description

This solution addresses key issue #6 on edge services support across ECSPs and key issue #10 on support for roaming UEs.

This solution proposes to include supported PLMN ID(s) in the ECS configuration information as described in the following table:

Table 7.13.2-1: ECS configuration information provided by ECS to 5GC

| Information element | Status | Description |
| --- | --- | --- |
| … | … | … |
| Supported PLMN ID(s) | O | The identifier of the PLMNs of which EDN configuration information can be provided by the ECS |

NOTE:	Except in LBO roaming scenario, when the ECS configuration in Table 7.x13.2-1 is successfully stored in the UDM, the H-SMF can provide ECS Configuration Information including a VPLMN ID. In this case, UE can use the ECS Configuration Information to get EDN configuration information of the VPLMN only when the UE is registered via a cell of the VPLMN.

The above ECS configuration information may be provisioned to the 5GC procedure (see 3GPP TS 23.502 [8]) or configured in an ECS so that it can be used to select proper other ECS in solution #4 of clause 7.4.

Editor's Note: How to provision the updated ECS configuration information via the 5GC is SA2's responsibility.

Editor's Note: It is FFS whether additional IE such as supported ECSP ID(s) is included in the ECS configuration information.

Editor's Note: It is FFS whether the Supported PLMN ID(s) needs to be provided to the EEC.

Editor's Note: Relationship between the Spatial Validity Condition defined in 3GPP TS 23.502 [8] and Supported PLMN ID is FFS.

### 7.13.3	Solution evaluation

TBD

## 7.14	Solution #14: V-ECS Discovery via the H-ECS

### 7.14.1	Architecture enhancements

None.

### 7.14.2	Solution description

#### 7.14.2.1	General

This solution addresses aspects of Key Issue #10. Specifically, the solution explains how the EEC in the roaming UE knows the availability of ECS(s) and/or EES(s) and discovers them in the VPLMN.

The principle of this solution is that, when roaming, the H-ECS provides information to the UE so that the EEC can communicate with an ECS in the VPLMN (i.e. a V-ECS). The UE can then use a home routed PDU Session to communicate with an H-ECS and an LBO PDU Session to communicate with a V-ECS. In this solution, the home network does not need to provide V-EES information to the EEC. The EEC can discover V-EES information by interacting directly with the V-ECS.

As explained in 3GPP TS 23.501 [5]: "The HPLMN can control via subscription data per DNN and per S-NSSAI whether a PDU Session is to be set-up in HR or in LBO mode." This solution explains how the H-ECS can provide information to the UE that can be used to establish an LBO PDU Session that is used to reach a V-ECS.

As one option, the H-ECS can provide an FQDN or an IP Address of a V-ECS to the EEC. When the UE accesses the provided FQDN or IP Address, URSP rules may steer the UE to use a DNN/S-NSSAI combination that can be used to reach the V-ECS (e.g. an LBO Session).

As a second option, the H-ECS can provide a DNN / S-NSSAI combination to the EEC. In this case, the DNN / S-NSSAI combination may be used to send a PDU Session Establishment Request that will result in an LBO PDU Session. The SMF in the VPLMN may then send ECS Address Configuration Information to the UE as described in 3GPP TS 23.548 [19]. The EEC may then use the ECS FQDN or IP Address from the ECS Address Configuration Information to reach the V-ECS.

#### 7.14.2.2	Procedure

This procedure presents a high-level overview of Solution #14.

Figure 7.14.2.2-1: V-ECS Discovery via the H-ECS

1.	The EEC sends a Service Provisioning Request to an ECS in the home network via a home routed PDU session. As described in 3GPP TS 23.558 [2], clause 8.3.3.3.7, the Service Provisioning Request may include the identity of the PLMN that the UE is currently registered to.

2.	If the Service Provisioning Request from the EEC did not include a PLMN ID, then the ECS may obtain the PLMN ID by invoking the NEF's monitoring event API with the monitoring type set to ROAMING_STATUS and the plmnIndication set to TRUE as described in 3GPP TS 29.522 [17] and 3GPP TS 29.122 [18]. The ECS receives a response from the NEF. The response indicates if the UE is roaming and the identity of the UE's serving PLMN.

3.	The ECS sends a Service Provisioning Response. The Service Provisioning Response may include a new information element called ECS Discovery Information. The ECS Discovery Information may include ECS Contact Information (O), ECS Provider ID (O), DNN (O), and S-NSSAI (O).

NOTE:	The solution requires that the deployment allows for the ECS to be provisioned with the ECS Discovery information (e.g. via OAM).

4.	The UE establishes an LBO PDU Session that will be used to communicate with the V-ECS and/or to obtain ECS Contact Information from the VPLMN.

The ECS Discovery Information from step 3 may include only ECS Contact Information (i.e. an FQDN of an ECS in the VPLMN). In this case, URSP rules may steer the UE to use a DNN/S-NSSAI combination that can be used to reach the ECS (e.g. an LBO Session).

The ECS Discovery Information from step 3 may include a DNN / S-NSSAI combination. In this case, the combination may be used to send a PDU Session Establishment Request that will result in an LBO session. The SMF in the VPLMN may then send ECS Address Configuration Information to the UE as described in 3GPP TS 23.548 [19].

5.	The EEC sends a Service Provisioning Request to an ECS in the visited network. The ECS address was obtained in step 3 or step 4.

6.	The EEC receives a Service Provisioning Response via the LBO PDU session that was established in step 4. As described in step 3, the ECS service provisioning response may include ECS Contact Information for other ECS(s) in the VPLMN. For example, the ECS that was contacted in step 5 may be a "primary" ECS of the VPLMN and the "primary" ECS of the VPLMN may provide the UE with ECS Contact Information for other ECS(s) in this step.

Editor's note:	Whether H-ECS can provide V-ECS address information directly to the UE is FFS.

### 7.14.3	Solution evaluation

Solution #4 and this solution share the principle that the home network can provide the UE with information that is used to contact a V-ECS. In solution #4 the V-ECS information may be an address, endpoint or service API information. In this solution, the V-ECS information can include a DNN (O) and/or S-NSSAI (O). The DNN / S-NSSAI can be used to establish an LBO PDU Session. Once the UE establishes an LBO PDU Session, Rel-17 procedures can be used to discover the V-ECS address.

NOTE: 	Solution #4 includes an option where EDGE-10 is used between the H-ECS and V-ECS. EDGE-10 is not used in this solution; however, this solution does not preclude the addition of EDGE-10.

## 7.15	Solution #15: Initial EAS selection declaration

### 7.15.1	Architecture enhancements

None.

### 7.15.2	Solution description

This solution addresses KI#8 and KI#14. In this solution, the EES can know the selected EAS and the EES is enabled to trigger the EAS traffic influence after initial EAS is being determined.

Figure 7.15.2-1: Initial EAS selection declaration

In Figure 7.15.2-1 step 1 and 2, the EEC performs the start-up procedures for initial service provisioning and EAS discovery. EEC may send EAS discovery to multiple EESs. If registration is required by an EES, EEC registers into the EES before EAS discovery.

In step 3, the EEC (or AC and EEC) selects the initial EAS from the discovered EAS candidates.

In step 4, the EEC sends Selected EAS declaration request with AC ID, EAS ID, EAS endpoint and UE ID to the selected EES (which is determined based on the selected EAS).

The EES, in step 5:

-	may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before.

NOTE 1:	EES can also influence the EAS traffic in advance.

The EEC is then responded by the selected EES with success/failure of the request in step 6.

Editor's Note: It is FFS whether EES can perform more actions (e.g. store SSC) and the timing for AC to connect to selected EAS.

NOTE 3:	The AC is not depicted in above figure for simplicity and solution to address interaction between AC and EEC is related to KI#4 for step 1 to 3.

Table 7.15.2.1-1 describes information elements for the selected EAS announcement request sent from the EEC to the serving EES.

Table 7.15.2.1-1: Selected EAS announcement request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of the UE. |
| AC ID | O | The Application Client ID. |
| Security credentials | M | Security credentials. |
| Selected EAS ID | M | Selected EAS identifier. |
| Selected EAS Endpoint | M | Endpoint of the selected EAS. |

### 7.15.3	Solution evaluation

This solution addresses KI#14. It allows the EES to be aware of the selected EAS in initial service start so that the EES can control application traffic influence for the initial application traffic. It is a viable solution.

Editor's note: It is FFS whether and how management of SSC required by this solution can be provided if the EEC does not register to EES before EAS discovery.

Editor's note:	Whether a new selected EAS declaration announcement API is needed or existing API (e.g. EEC registration) can be re-used is FFS.

## 7.16	Solution #16: EAS discovery for different users

### 7.16.1	Architecture enhancements

None.

### 7.16.2	Solution description

#### 7.16.2.1	General

The following solution corresponds to the key issue #12 on service differentiation for users.

In this solution, the User information corresponding to service permission level is introduced to achieve service differentiation.

The User information corresponding to service permission level could be a list of UE identifier(s) associated with service permission level. It enables EES support to handle the information of user priority or something like "allowed-list", or EAS deployment segmentation in EDN. And it is provided to EES by EAS during the EAS registration or EAS registration update.

Editor's note:	It is FFS where the User information can be stored and how to use the User information.

#### 7.16.2.2	Procedure

In this solution, when EES receives the EAS discovery request, in addition to service permission level filter, the EES identifies the EAS based on received User information.

Editor's note:	It is FFS if User information is not received in the EAS discovery request.

Editor's note: It is FFS when large set of users are present, where the User information can be stored and how to use the User information.

### 7.16.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.17	Solution #17: Traffic influence for initial EAS discovery

### 7.17.1	Architecture enhancements

None.

### 7.17.2	Solution description

#### 7.17.2.1	General

The following solution corresponds to the key issue#14 on application traffic influence for initial EAS discovery clause 4.14.

#### 7.17.2.2	Procedure

In this solution, the EES will decide whether to perform traffic influce for the AC before the real data transmission between the AC and the EAS considering the AC type, requirement of the AC and so on, since AC may not connect to the EAS immediately after obtain the EAS information, in this case, the execution of traffic influence is unnecessary.

Figure 7.17.2.2-1: Traffic influence for initial EAS discovery

1.	The EEC determines to perform EAS discovery before the real data transmission between the AC and the EAS based on the response time in the AC profile. The EEC sends the EAS discovery request to the EES. The request contains the AC type, AC Service KPIs.

2.	Upon receiving the request from the EEC, if the AC has high requirement on response time, then the EES will select one EAS and determine to perform application traffic influence for this AC before the real data transmission between the AC and the EAS, based on the AC type, AC Service KPIs, response time. If the AC does not have high requirement on response time, then the EES will not perform application traffic influence immediately.

3.	The EES sends the AF request to the 5GC optimizing the user plane path between the UE and the EAS, if the EES determines to perform application traffic influence immediately.

4.	The EES sends the EAS discovery response to the EEC. The response contains the discovered EAS list with EAS being traffic influenced. The EEC sends the EAS discovery response to the AC.

### 7.17.3	Solution evaluation

This clause provides an evaluation of the solution.

The solution #17 is applicable to the scenario where the EES selects the EAS and performs the traffic influence immediately for the selected EAS after EEC sent EAS discovery request. It is a viable solution.

NOTE: The EES may only be able to use the Service KPIs to determine whether to perform traffic influence immediately or not.

## 7.18	Solution #18: Constraint device in EDGEAPP

### 7.18.1	Architecture enhancements

None.

### 7.18.2	Solution description

This solution addresses KI#15. In this solution, in order to reduce power consumption in the UE, it specifies the EEC with Reduced Capabilities (RedEEC). The RedEEC skips EEC registration and the EAS discovery procedure is enhanced to delegate the EAS selection to EES. Figure 7.18.2-1 illustrates the details interactions.

Figure 7.18.2-1: service provisioning and EAS discovery for constraint device

In step 1, EEC performs service provisioning. For EAS discovery, the EEC sends the request to a selected EES from the candidate EES(s) and indicates UE type to EES in step 2. In step 3, the EES performs EAS discovery as described in clause 8.5.2.2 of 3GPP TS 23.558 [2] and in addition selects a suitable EAS from discovered candidate EAS(s) if UE type indicates constraint device, then the EES sends EAS discovery response to the EEC including the selected EAS information in the Discovered EAS list. In step 5, if the EAS discovery response contains successful result, the EEC sends to AC the received selected EAS information and the EAS discovery and selection procedure ends; otherwise, the EEC repeats step 2 with next selected candidate EES.

NOTE:	The EEC can interact with each candidate EES if no appropriate EAS can be selected by EES.

Table 7.18.2-1 and table 7.18.2-1 below show the detailed impact (highlighted with bold text) for EAS discovery request and response.

Table 7.18.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Target DNAI (NOTE) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| UE type | O | Indicates UE or device type (e.g. constraint device) |
| NOTE: This IE shall not be included when the request originates from the EEC. |  |  |

Table 7.18.2-2: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. Based on UE type, only one EAS may be included. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |

In service continuity, to offload UE from monitoring triggers (e.g. location change) to start ACR, EEC can announce its support for EDN side decided ACR scenarios (clause 8.8.2.4 and clause 8.8.2.5 of 3GPP TS 23.558 [2]) so that EEC only need to passively receive ACR information notifications.

Editor's Note: Whether or not EEL can provide ACR support for ACR scenarios (clause 8.8.2.4 and clause 8.8.2.5 of 3GPP TS 23.558 [2]) is FFS

### 7.18.3	Solution evaluation

This solution addresses KI#15 with EAS discovery and selection procedure for constrained device using EDGEAPP. The EEC relies on EES to select an appropriate EAS (the EEC does not need to select an EAS among discovered EAS list from candidate EESs). When (or since) no explicit registration request is used in conjunction with this solution, EEL support for ACR scenarios in clause 8.8.2.4 and clause 8.8.2.5 of 3GPP TS 23.558 [2] requires further clarifications.

## 7.19	Solution #19: EES determines the selected ACR scenario

### 7.19.1	Architecture enhancements

None.

### 7.19.2	Solution description

#### 7.19.2.1	General

The following solution corresponds to the key issue#19 on overlapping of ACR scenario in clause 4.19

#### 7.19.2.2	Procedure

In this solution, the EES is responsible to determine the selected ACR scenario for each AC based on the ACR scenarios supported by AC, EEC, EES and EAS. When the EES determines the selected ACR scenario for one AC, the EES will send the selected ACR scenario for the AC to the EEC and the EAS, then the EEC will notify the AC with the selected ACR scenario. With this solution the EES can determine which ACR approach can be enabled for the AC and EAS considering the detailed information (e.g EAS features, KPIs) of both the AC and the EAS and thus also avoiding the overlapping of ACR initiations by multiple entities which introduce unnecessary signalling to resolve co-existence issues.

Figure 7.19.2.2-1: EES determines the selected ACR scenario

1.	The EAS sends an AC information subscription request to the EES.

2.	The EES sends an AC information subscription response to the EAS with the subscription result.

3.	The EEC sends the EAS discovery request to the EES, the request contains the AC service continuity support and EEC service continuity support indicating which ACR scenarios are supported by the AC and the EEC.

Editor's note:	It is FFS what information and message are required to trigger EES to make ACR scenario determination.

4.	Upon the EES obtains the AC service continuity support and EEC service continuity support from UE and EAS service continuity support, then the EES determines one suitable ACR scenario for the AC based on the ACR scenarios supported by AC, EEC, EES and EAS. The EES can select an appropriate ACR scenario for the AC from the intersection of the ACR scenarios supported by AC, EEC, EES and EAS.

NOTE:	The ACR scenario supported by the EAS is available in the EAS service continuity support of the EAS profile.

Editor's note: It is FFS how the EES can know the supported service continuity scenarios of the T-EES and T-EAS.

5.	The EES sends the AC information notification to the EAS, the notification contains the selected ACR scenario and related ACID.

6.	The EES sends the EAS discovery response to the EEC including the selected ACR scenario and related ACID, then the EEC may notify the AC with the selected ACR scenario.

Editor's note: It is FFS whether the existing EES-provided service capability API is enhanced to support this ACR scenario selection or the new service API is defined.

Editor's note: It is FFS how to utilize multiple ACR scenarios to detect ACR timely.

### 7.19.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.20	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server

### 7.20.1	Architecture enhancements

Architecture enhancement in clause 6.3 is the basis for this solution.

### 7.20.2	Solution description

#### 7.20.2.1	General

The following solution corresponds to the key issue #1, "enhanced notification service to the EEC" as described in clause 4.1.

In this solution, ENS is used as a centralized notification server enhancing EEL's notifications delivery mechanism to EEC. Such an architectural enhancement allows the EEC to inform EEL of its preferred method (e.g. Long-polling, WebSocket, FCM, APNS, OMA Push) of receiving notifications and enables Edge Application architecture to flexibly provide a diverse set of methods for notifications delivery to EEC.

#### 7.20.2.2	Notification delivery over a direct Notification Channel Procedure

In this procedure, based on EEC's request on the preferred method of notification delivery (i.e. Log-polling or WebSocket), a direct notification channel in between the EEC and ENS is established.

NOTE:	Notification delivery method using a Push server is described in clause 7.20.2.3.

Pre-conditions:

1.	EEC is aware of the ENS's endpoint through provisioning

2.	EES and ECS are authorized to interact with the ENS

3.	EEC is authorized to interact with the ENS, EES and ECS

Figure 7.20.2.2-1: Notification delivery over a direct Notification Channel

1.	The EEC sends a request to ENS for a direct notification channel (e.g. by indicating either a Long-polling (Pull) or a WebSocket (Push) notification delivery method).

2.	Upon receiving the request, the ENS performs an authorization check and further verifies if the requested notification delivery method (e.g. Long-polling or WebSocket ) can be used. If the request is authorized and the requested notification delivery method can be used, the ENS assigns a CallBack URL and an associated Channel URL for the EEC and stores the information for its subsequent use (i.e. any notification received at the assigned Callback URL will be made available over the associated Channel for consumption by the EEC).

3.	If the processing of the request was successful, the ENS responds with the assigned Callback URL and Channel URL

4.	EEC, upon receiving the Callback URL and Channel URL, based on the type of direct notification channel (e.g. Long-polling or WebSocket) it wishes to have with ENS, would either starts polling the Channel URL (e.g. HTTP GET Channel URL)f or events or uses the Channel URL and prepares it for the Push delivery (e.g. uses the channel URL and upgrades it to a WebSocket notification channel as per WebSocket protocol and in essence opens the notification channel for the flow of notifications as they arrive at ENS).

5.	The EEC sends a EAS discovery subscription request and/or ACR information subscription request (as described in clauses 8.5.3.4 and 8.8.4.8 of 3GPP TS 23.558 v17.0.0 respectively) to the EES with the Notification Target Address IE set to the CallBack URL received from the ENS in step 3 above.

6.	Upon receiving the request, the EES performs an authorization check (as described in clauses 8.5.2.3.2 or 8.8.3.5.2 of TS 23.558 depending on the subscription request in step 5) and if the request is authorized, the EES creates the associated subscription resource.

7	EES responds to EEC with the subscriptionId of the created resource.

8	The EEC sends a service provisioning subscribe request (as described in clause 8.3.3.3.4 of 3GPP TS 23.558 v17.0.0) to the ECS with the Notification Target Address IE set to the CallBack URL received from the ENS in step 3 above.

9	Upon receiving the request, the ECS performs an authorization check as described in clause 8.3.3.2.3.2 of TS 23.558 and if the request is authorized, the ECS creates the associated subscription resource.

10.	ECS responds to EEC with the subscriptionId of the created resource.

11.	Some events of interest to EEC occurs in ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription directs the ECS to push the event to the CallBack URL which is terminated at the ENS.

12.	ECS pushes the notification to the CallBack URL

13.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Channel URL (which it created and assigned to the EEC in step 2 above) and determines which notification delivery method was requested by the EEC (as per step 1 above). ENS forwards the notification to EEC either by responding to the Long-polling request (if EEC is using the Long-polling method) or pushes the notification to the EEC (if EEC is using a Push method such as the WebSocket delivery method),

14.	Some events of interest to EEC occurs in EES that satisfies trigger conditions for updating a subscribed EEC with the EAS discovery information or ACR related information (e.g. ACR complete event) and the corresponding subscription directs the EES to push the event to the CallBack URL which is terminated at the ENS.

15.	EES pushes the notification to the CallBack URL

16.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Channel URL (which it created and assigned to the EEC in step 2 above) and determines which notification delivery method was requested by the EEC (as per step 1 above). ENS forwards the notification to EEC either by responding to the Long-polling request (if EEC is using the Long-polling method) or pushes the notification to the EEC (if EEC is using a Push method such as the WebSocket delivery method).

#### 7.20.2.3	Notification delivery using a Push Server Procedure (indirect Notification Channel)

In this procedure, based on EEC's request that the preferred method of notification delivery being via a Push server, there is no need for a direct notification channel between the EEC and ENS. Instead, ENS forwards the notifications through an identified Push server to the EEC. In other words, based on EEC's request, an indirect notification channel between the EEC and ENS with the Push server as an intermediary is established.

NOTE: This procedure reuses the Push function setup steps 1a, 1b and 1c described in solution #1 (see clause 7.1.2.2).

Pre-conditions:

1.	EEC is aware of the ENS's endpoint through provisioning

2.	EES and ECS are authorized to interact with the ENS

3.	EEC is authorized to interact with the ENS, EES and ECS

4.	ENS is aware of the Push server endpoint and is authorized to interact with it

Figure 7.20.2.3-1: Notification delivery using a Push Server

1.	The EEC registers with the push function within the UE. The EEC acquires a push token and push server information from the push function. This step is depicted as sub-steps 1a, 1b and 1c in Figure 7.20.2.2-1.

NOTE:	The push server provides the push function in the UE with a push token, which is delivered to the EEC.

2.	The EEC sends a request to ENS for an indirect notification channel via the identified Push server. The request contains the Push server information as well as the EEC's push token (see step 1)

3.	Upon receiving the request, the ENS performs an authorization check and further verifies if the requested notification delivery method (i.e. Push server) can be used. If the request is authorized and the requested notification delivery method can be used, the ENS assigns a CallBack URL for the EEC and stores the information for its subsequent use (i.e. any notification received at the assigned Callback URL will be forwarded to the associated Push server for consumption by the EEC).

4.	If the processing of the request was successful, the ENS responds with the assigned Callback URL.

5.	The EEC sends a EAS discovery subscription request and/or ACR information subscription request (as described in clauses 8.5.3.4 and 8.8.4.8 of 3GPP TS 23.558 v17.0.0 respectively) to the EES with the Notification Target Address IE set to the CallBack URL received from the ENS in step 4 above.

6.	Upon receiving the request, the EES performs an authorization check (as described in clauses 8.5.2.3.2 or 8.8.3.5.2 of TS 23.558 depending on the subscription request in step 5) and if the request is authorized, the EES creates the associated subscription resource.

7.	EES responds to EEC with the subscriptionId of the created resource.

8.	The EEC sends a service provisioning subscribe request (as described in clause 8.3.3.3.4 of 3GPP TS 23.558 v17.0.0) to the ECS with the Notification Target Address IE set to the CallBack URL received from the ENS in step 4 above.

9.	Upon receiving the request, the ECS performs an authorization check as described in clause 8.3.3.2.3.2 of TS 23.558 and if the request is authorized, the ECS creates the associated subscription resource.

10.	ECS responds to EEC with the subscriptionId of the created resource.

11.	Some events of interest to EEC occurs in ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription directs the ECS to push the event to the CallBack URL which is terminated at the ENS.

12.	ECS pushes the notification to the CallBack URL

13.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Push server and the EEC's push token (which it received from EEC in step 2 above) and pushes the notification to the identified Push server using the given EEC's push token notification.

14.	Push server forwards the notification to the EEC via the push function.

15.	Some events of interest to EEC occurs in EES that satisfies trigger conditions for updating a subscribed EEC with the EAS discovery information or ACR related information (e.g. ACR complete event) and the corresponding subscription directs the EES to push the event to the CallBack URL which is terminated at the ENS.

16.	ECS pushes the notification to the CallBack URL

17.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Push server and the EEC's push token (which it received from EEC in step 2 above) and pushes the notification to the identified Push server using the given EEC's push token notification.

18.	Push server forwards the notification to the EEC via the push function.

### 7.20.3	Solution evaluation

TBD

## 7.21	Solution #21: Prediction expiration time for service continuity planning enhancement

### 7.21.1	Architecture enhancements

None.

### 7.21.2	Solution description

#### 7.21.2.1	General

The following solution corresponds to the key issue #3 Enhancements to service continuity planning in clause 4.3.

In service continuity planning, the ACR is performed for a predicted or expected location of the UE in the future. Since the prediction is for a future location in a future time, a prediction expiration time is introduced to take account for the future time window in which the prediction is expected to happen. Therefore, in this solution, EEC includes its expectation or knowledge of the time that it expects the prediction to happen within the ACR request at latest. This is the time that the EEC requests the T-EAS to wait for the AC to connect at latest. The EESs can consider this time in deciding the time that they will wait for the ACR status update from the corresponding EASs.

EES can take into account the prediction expiration time in processing the ACR request message, e.g. accepting or rejecting the request. In case that the request is authorized and accepted by the EES, it also includes it in the notification (if any) to the EAS. Similarly, EAS can take into account the prediction expiration time in performing ACT. T-EAS and T-EES will receive the prediction expiration time and may consider it in deciding whether and for how long to wait for the UE to connect to them.

#### 7.21.2.2	Procedure

Figure 7.21.2.2-1 illustrates the EEC triggered ACR scenario for service continuity planning.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES.

2.	EEC has detected the need for ACR for an expected/predicted location in the future.

Figure 7.21.2.2-1: EEC triggered ACR scenario for service continuity planning

1.	The EEC sends an ACR request message to the EES with initiation or determination action. Depending on the ACR scenario, the EES can be either of S-EES or T-EES.

In case of service continuity planning the ACR request message may include also the prediction expiration time to indicate the time that the EEC expects the prediction takes place at latest. This is the time that the EEC requests the T-EAS to wait for the AC to connect at latest.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

2a.	If authorized and if EES decides to continue the ACR, if the EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the need to start ACR. In that case, if the request in step 1 includes a prediction expiration time, the EES includes it in the notification to EAS.

If the request in step 1 is to S-EES this EAS is the S-EAS. Otherwise if the request of step 1 is to T-EES, this EAS is the T-EAS.

3.	The EES responds to the requestor's request, i.e. EEC, with an ACR response message.

4.	(a) If the action in the request in step 1 is initiation, the EES sends the information and parameters of the ACR to the remote EES. Otherwise if the action is determination the EES first discovers the T-EES and T-EAS, and then sends the information and parameters of the ACR to the T-EES.

(b) If the remote EAS has subscribed to receive such notification, the remote EES shall notify the EAS about the need to ACR and includes corresponding parameters of the ACR, e.g. prediction expiration time if it is provided in the request in step 1.

(c) The remote EES sends the response back to the EES to confirm it has received the ACR parameters.

If the request in step 1 is to S-EES the remote EES is the T-EES. Otherwise if the request of step 1 is to T-EES, the remote EES is the S-EES.

5.	The ACR is executed depending on the ACR scenario. S-EAS and T-EAS perform application context transfer (ACT) in an application specific time and manner. The EAS may use the prediction expiration time received in step 2a to decide on whether and how to perform the ACT. The T-EAS may also consider it in deciding whether and for how long to wait for the AC of the UE to connect to it.

6.	The EAS sends the failure or success message in the ACR status update based on the results of step 5b. EAS can consider the prediction expiration time received in step 2a, if provided, to send the message in step 6.

If the request in step 1 is to S-EES this EAS is the S-EAS. Otherwise if the request of step 1 is to T-EES, this EAS is the T-EAS.

7.	The EES sends the ACR complete notify to the EEC with failure or success result. The EES also considers the failure or success message received in step 6 in the ACR complete notification.

Editor'note: It is FFS how the EES and EAS can consider and process the "Prediction expiration time" IE in deciding details of ACR operations.

Editor's note: How the EES or EAS may react when the requested "Prediction expiration time" is not acceptable (e.g. based on ECSP policy, resource status) is FFS.

Editor's note: Limiting the "Prediction expiration time" before the first ACR request and avoiding multiple ACR requests in case of failure is FFS.

### 7.21.3	Solution evaluation

Editor's note: Solution evaluation is FFS.

## 7.22	Solution #22: Support simultaneous EAS connectivity in ACR

### 7.22.1	Architecture enhancements

None.

### 7.22.2	Solution description

#### 7.22.2.1	Solution for traffic influence

To solve the issue about traffic influence to maintain both S-PSA and T-PSA to support simultaneous connectivity with both S-EAS and T-EAS during the service continuity, the AC includes the need for simultaneous EAS connectivity in the AC profile and the AC profile is sent to EEC via EDGE-5 reference point so that the EEC can request such need in the ACR request sent to EES. Then the EES, at the time of requesting traffic influence towards 3GPP CN, provides additional requirement for simultaneous PSA connectivity as described in clause 6.3.4 of 3GPP TS 23.548 [19].

Figure 7.22.2.1-1 describes the detailed interaction between the AC and EEC for AC registration procedure (a possible way to provide AC profile to EEC).

Figure 7.22.2.1-1: AC Registration procedure

1.	The AC sends AC registration request to EEC, the request includes the AC profile (see table 7.22.2.1-1).

2.	The EEC authorizes the AC registration request and stores the AC profile.

3.	The AC is responded with AC registration response (success/failure).

Table 7.22.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS |

Figure 7.22.2.1-2 describes the detailed interaction between the EEC and EES during service continuity (i.e. ACR).

Figure 7.22.2.1-2: ACR initiation procedure

0.	The EEC is informed about AC profiles (e.g. during AC registration procedure as depicted in Figure 7.22.2.1-1).

1.	The EEC triggers ACR request (type: initiation) to EES, the request includes the simultaneous EAS connectivity information in service continuity (see table 7.22.2.1-2) which was previously received as part of the AC profile (see table 7.22.2.1-1).

2.	The EES authorizes the ACR request. Then the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS and Simultaneous EAS connectivity information in the 3GPP Core Network (if applicable), as described in clause 5.6.7.1 of 3GPP TS 23.501 [5] and clause 6.3.4 of 3GPP TS 23.548 [19].

Editor's Note:	Since the 3GPP CN only supports simultaneous PSA connectivity in SSC mode 3 or session breakout, it is FFS whether EES should firstly know PDU session capability before invoking AF traffic influence API.

3.	The EEC is responded with ACR response (success/failure).

Table 7.22.2.1-2: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| UE identifier | O | The identifier of the UE (i.e. GPSI). |
| ACID | O | The identifier of the AC. |
| ACR action | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > Simultaneous EAS connectivity information | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| … | … | … |
| > S-EAS endpoint | O | Endpoint information of the S-EAS |
| ACR determination data | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |

### 7.22.3	Solution evaluation

This solution addresses KI#4 about "Whether and how an AC registers to an EEC". During the AC registration, the AC is able to indicate its desire for keeping Simultaneous EAS Connectivity. This solution addresses KI#21 about traffic influence. The EES interacts with 3GPP CN to satisfy AC need during ACR request processing.

## 7.23	Solution #23: UE identification with NAT

### 7.23.1	Architecture enhancements

None.

### 7.23.2	Solution description

#### 7.23.2.1	General

This solution corresponds to KI#16 on support of NAT deployed within the edge data network.

In this solution, the EEC provides the CN network assigned IP address i.e., the private IP address of the UE to the EES to obtain the UE identifier from the EES. EES uses the private IP address of the UE provided by the EEC to invoke the CN capability to translate UE's IP address to its UE ID (e.g., UE's GPSI).

Once obtained, the EEC passes the UE ID to the AC, which provides it to the EAS for use over EDGE-3 interface. With EAS and EES both being aware of the UE ID, issues pertaining to NAT and UE identification are resolved.

#### 7.23.2.2	Procedure

Figure 7.23.2.2 illustrates the procedure for the EEC to obtain the UE ID from the EES and provide to the AC.

Figure 7.23.2.2-1: EEC obtaining UE ID from the EES

1.	AC sends an Edge UE ID request to the EEC. The request may include the list of EASIDs for which the AC is requesting the Edge UE ID information.

2.	The EEC upon receiving the request validates if AC is authorized to request this information. If AC is authorised, the EEC sends the Edge UE ID request to the EES. The request includes the CN assigned private IP address of the UE and may include the list of EASIDs if provided by the AC.

NOTE 1:	EEC can also send this request without receiving a request from AC in step 1.

NOTE 2:	Private IP address used by AC and private IP address used by EEC can be the same or different (if different PDU sessions are used). The request from EEC can include either of the IP addresses.

3.	Upon receiving the request from EEC, the EES authorizes the EEC. If authorized the EES invokes the CN capability Nnef_UEId_Get for translating the UE's IP address to its UE ID as defined in 3GPP TS 23.502 [08]. If the request from EEC includes a list of EASIDs, the EES may invoke the Nnef_UEId_Get API for each EAS individually to obtain EAS specific UE ID(s). The EES generates temporary Edge UE ID(s) which may be the same as the 3GPP CN provided UE ID or may be assigned by the EES itself.

4.	The EES sends the Edge UE ID response to the EEC including the Edge UE ID(s).

5.	Upon receiving the response from the EES, the EEC provides the Edge UE ID information to the AC by sending the Edge UE ID response.

6.	The AC provides the Edge UE ID information to the EAS.

NOTE 3:	Details on how the AC provides the information to the EAS is out of scope.

7.	Once received from the AC, the EAS uses the Edge UE ID to invoke the APIs provided by the EES over EDGE-3 interface (e.g., T-EAS Discovery, UE location request, ACR request, and EELManangedACR services).

8.	Once received from the EES, the EEC uses the Edge UE ID to invoke API provided by the EES over EDGE-1 interface (e.g., EAS Discovery and ACR request services).

9.	The EES uses the UE ID obtained from the CN in step 3 to invoke the 3GPP CN capabilities as in TS 23.558, Clause 8.10.3. This step can be performed following triggers that require 3GPP CN capabilities to be invoked (e.g., on receiving a request over EDGE-1 or EDGE-3), in which case, to invoke the 3GPP CN capabilities the EES uses the UE ID associated with the Edge UE ID included in the trigger.

Editor's Note:	Whether coordination with SA2 for this solution to work as intended in case of private IP overlap issue in large network deployments is FFS.

### 7.23.3	Solution evaluation

## 7.24	Solution #24: ACR between CAS and EAS

### 7.24.1	Architecture enhancements

### 7.24.2	Solution description

Figure 7.24.2-1: Illustration of application architecture with Edge and Central server deployment

In this solution, the Central Application Server (CAS) is supported by Central Enabler Server (CES) via EDGE-3' reference point and CES is supported by ECS via EDGE-6' reference point. The CES communicates with EES or another CES via EDGE-9' reference point. The EEC utilizes EDGE-1' reference point to communicate with the CES. The CES and CAS interact with 3GPP Core Network via EDGE-7' reference point and EDGE-2' reference point, respectively.

Editor's note:	It is FFS whether the "C" in CAS and CES stands for "Central" or "Cloud".

Editor's note:	It is FFS whether CES is needed and if it can be co-located with CAS

From concept wise, the CAS and CES are servers deployed in a central cloud and EAS and EES are servers deployed in an edge cloud. If certain EAS and EES in the edge cloud are capable of supporting more UEs than regular edge server and can serve UE from anywhere (N6 routable), their role becomes central server.

Editor's note:	It is FFS whether EDGE-1' is needed. And it is FFS for the detailed function differences between EDGE-1 and EDGE-1' (if needed), EDGE-3 and EDGE-3', between EDGE-6 and EDGE-6' and between EDGE-9 and EDGE-9'.

Since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to connect to either another EAS in new EDN or the CAS. For the latter case, the EES interacts with the CES via EDGE-9' reference point and application context is transferred between the EAS and CAS. Later, if the UE is moving to an area with edge coverage, the CES interacts with the EES via EDGE-9' reference point and application context is transferred between the CAS and EAS.

### 7.24.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.25	Solution #x25: ACR between EAS and Cloud Application Server

### 7.25.1	Architecture enhancements

Architecture enhancements in clause 6.5 is the basis for this solution.

### 7.25.2	Solution description

#### 7.25.2.1	General

The following solution addresses open issues of key issue #11, ACR between EAS and Cloud Application Server.

#### 7.25.2.2	Procedure

The scenarios specified in 3GPP TS 23.558 (Rel-17) clause 8.8 have been updated to consider the ACR between EAS and Cloud Application Server.

##### 7.25.2.2.1	Updated 3GPP TS 23.558 clause 8.8.2.2 Initiation by EEC using regular EAS Discovery

The scenario handles ACR as a result of the UE moving to, or the UE expecting to move to, a new location which is outside the service area of the serving EAS. It further relies on the EEC being triggered as a result of the UE's movement.

This scenario is based on Service Provisioning (as specified in TS 23.558) and DNS procedures to discover the CAS that shall serve the AC as a result of the UE's new location, and that shall receive the Application Context from the serving EASs. The scenario below describes the relocation of a single application context to a CAS. However, it should be repeated for each active AC in the UE for which EAS or EDN is not available on that UE location.

The pre-conditions:

1.	The AC in the UE already has a connection to a corresponding S-EAS;

2.	The preconditions for the Service provisioning - Request/Response model as specified in TS 23.558 with regards to the EEC are fulfilled; and

3.	The EEC is triggered when it obtains the UE's new location or is triggered by another entity such as an ECS notification.

Phase I: ACR Detection

1.	The EEC detects the UE location update as a result of a UE mobility event and is provided with the UE's new location as described in TS 23.558. The EEC can also detect an expected or predicted UE location in the future as described in TS 23.558.

NOTE 1:	If the EEC is triggered by an external entity such as by a notification from the ECS, unavailability of new EESs (to be used as T-EESs) is provided by that notification and step 3 below is skipped.

Phase II: ACR Decision

2.	Either the AC or the EEC makes the decision to perform the ACR.

NOTE 2:	Which applications require ACR can be decided based on the application profile, e.g. requirement of service continuity of the application.

Phase III: ACR Execution

3.	The EEC performs Service Provisioning (as specified in TS 23.558) for all active applications that require ACR. Since the location of the UE has changed, this procedure results in unavailability of T-EESs that are relevant to the supplied applications and the new location of the UE. If the service provisioning is done without supplying the application information but EES information is provisioned, the EEC attempts discovering relevant T-EAS with the EES provisioned in the service provisioning response, if any. Service provisioning or discovery of relevant T-EAS may not result in EES configuration or T-EAS is not discovered respectively.

If the change in UE's location does not trigger a need to change the serving EAS, the subsequent steps will not take place. The EEC remains connected to the serving EESs and the ACs remain connected to their corresponding serving EASs.

4.	The AC triggers the UE to perform DNS resolution for the CAS relevant for the AC. The UE may need to establish a new PDU connection to the CAS.

Editor's Note:	Further clarifications on how the AC is triggered to perform DNS query is FFS

5.	The EEC performs ACR launching procedure (as described in TS 23.558) to the S-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (along with the details of the CAS and without the need to notify the EAS). The S-EES may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable), as described in TS 23.558.

6.	The AC is triggered by the EEC to start ACT. The AC decides to initiate the transfer of application context from the S-EAS to the CAS.

After the ACT is completed, the AC remains connected to the CAS and disconnects from the S-EAS; the EEC is informed of the completion of the ACT.

The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context (e.g. based on monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV: Post-ACR Clean up

7.	The S-EAS sends the ACR status update message to the S-EES as specified in TS 23.558.

8.	If the status in step 7 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in TS 23.558.

The CAS can perform the required CN capability exposure subscriptions upon receiving the application context.

Editor's note:	Updated 3GPP TS 23.558 for other scenarios is FFS.

### 7.25.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.26	Solution #26: Bundled EASs

### 7.26.1	Architecture enhancements

None.

### 7.26.2	Solution description

#### 7.26.2.1	General

This solution corresponds to KI#18 on linkages between EASs.

Editor's Note:	The solution assumes that the EASs belonging to a bundle is part of the same EDN. Support for EASs spread across EDNs is FFS.

The solution extends the AC profile and EAS profile to introduce the following new IEs and their handling by different EEL functions:

-	EAS bundle information: EAS bundle information (e.g., EAS bundle ID, information of associated EASs) establishes an association between the EASs. Edge Enabler Layer handles the Edge Application Servers belonging to the same bundle as required by the bundle requirements. When included in the EAS profile, EAS Bundle information denotes the bundle to which the EAS belongs. When included in the AC profile EAS Bundle information is used to perform different Edge Enabler Layer operations, such as EAS discovery.

-	EAS bundle requirements: This IE provides the Edge Enabler Layer the requirements that apply to the bundle of EAS. The requirements may include combined discovery and combined ACR.

Both, EAS bundle information and EAS bundle requirements, are provided by the ASP. EAS bundle information can be a list of EASs or a bundle ID, however, detailed format of the EAS bundle ID is out of scope.

Table 7.26.2.1-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| EAS bundle information | O | Identifier of the bundle of EASs of which AC wants services. |

Editor's Note:	Including the EAS bundle requirement (including Coordinated ACR discovery and Coordinated ACR) in the AC profile is FFS.

Table 7.26.2.1-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > EAS bundle information | M | Identifier of the bundle to which EAS belongs. |
| > EAS bundle requirements | O | Requirements associated with the bundle |
| >> Coordinated EAS discovery | O | Indicates if EAS discovery request for one of the bundled EAS is received, then EAS discovery response should include information of all the EASs belonging to the bundle. |
| >> Coordinated ACR | O | Indicates if EAS ACR is initiated for one of the bundled EAS, then ACR should be initiated for all the EASs belonging to the bundle. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g., ACR for all other EAS that are part of the bundle must be cancelled or not. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

#### 7.26.2.2	Handling of EAS bundle information by the EEC

If AC profile contains an EAS bundle information, the EEC uses it while performing operations on EDGE-1 interface.

#### 7.26.2.3	Handling of EAS bundle information and EAS bundle requirements by the EES

The EES may receive the EAS bundle information from the EEC, EAS or another EES and the EAS bundle requirements from the EAS.

Upon receiving the EAS bundle information and EAS bundle requirements as part of the EAS registration request from the EAS, the EES stores the information and associates the EAS with other EASs providing the same EAS bundle information. The EAS bundle requirements are used by the EES while providing services on EDGE-1, EDGE-3 and EDGE-9 interfaces.

Upon receiving the EAS bundle information as part of the:

-	EAS discovery request from the EEC, EAS or another EES, and depending on the EAS bundle requirements the EES provides in the EAS discovery response information of all the EASs which are part of the EAS bundle.

Editor's Note: How EES provides EAS discovery response for all EASs in a bundle is FFS.

-	ACR request from the EEC or the EAS, and depending on the EAS bundle requirements the EES takes appropriate ACR related action (e.g., initiate ACR, cancel ACR etc.) for all the EASs which are part of the EAS bundle.

### 7.26.3	Solution evaluation

## 7.27	Solution #27: Enabling AC Association Aware services by selecting common EASs

### 7.27.1	Architecture enhancements

None.

### 7.27.2	Solution description

#### 7.27.2.1	General

This solution addresses key issue #17 on discovery of a common EAS in clause 4.17.

The solution describes enhancements to several edge procedures that enable ACs/EECs of different UEs to share AC association information with the EEL. This AC association is formed such that the services for the associated ACs are optimized by selecting a common EAS.

NOTE:	In the following descriptions within this solution, the term "Associated ACs" is used to denote a set of ACs on different UEs for which the association described above has been configured via an AC association Profile. How the AC association Profile is determined is out of scope of the current specification.

#### 7.27.2.2	New Information Elements

The AC association Profile is proposed as a new IE in TS 23.558:

Table 7.27.2.2-1: AC association Profile

| Information element | Status | Description |
| --- | --- | --- |
| Association ID | M | Identity of the Association |
| AC association type | M | Choice of multi-use or multi-session |
| List of associated UEs characteristics | O | Information for determining the UEs with associated ACs |
| > Associated UE group ID | O | Group ID pre-provisioned (i.e. as External Group ID) to the UEs with ACs in the associated group. If this IE is not present, |
| > Associated UEs service area | O | Service area for determining other UEs in the association. The UE location is described in clause 7.3.2. The optional additional EAS selection criteria describe criteria for the EAS selection (e.g., "same latency for all" or "lowest latency for the own UE location"). |
| List of associated ACs characteristics (NOTE 1) | M | Information for determining the ACs on the associated UEs |
| > common ACID | O | If provided, all ACs in the association have the same AC ID as indicated |
| > common ACs Type | O | If provided, all ACs in the association have the same AC Type as indicated |
| > common ACs Schedule | O | If provided, all ACs in the association have the same AC Schedule as indicated |
| List of common EAS characteristics (NOTE 1) | M | Information for determining the common EAS |
| >Common EAS discovery filter (NOTE 2) | O | Describes the characteristics of a common EASs using the EAS discovery filter described in Table 8.5.3.2-2. |
| > Common EAS aggregate Service KPIs | O | Service characteristics provided by the common EAS, detailed in Table 8.2.5-1. The characteristics are described to meet the requirements for the AC association. |
| NOTE 1: Either "List of associated AC characteristics" or "List of common EAS characteristics" shall be present. NOTE 2: Only the "List of EAS characteristics" IE from Table 8.5.3.2-2 is included. The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

Editor's Note. The list of IEs in Table 7.27.2.2-1 and whether AC association information should be provided in a new Profile IE, or via existing ones, is FFS.

Editor's Note. Whether the ACs in an AC association information may have different ACIDs is FFS.

#### 7.27.2.3	Enhancements to existing Information Elements

The following enhancements (highlighted with bold text) are proposed to tables in TS 23.558:

Table 7.27.2.3-1: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| AC association profile | O | Describes an association between ACs and an association between ACs and a common EAS. |
| NOTE 1: Either "List of AC characteristics" ,"List of EAS characteristics" or AC association profile shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

#### 7.27.2.4	Procedure for determining common EES

Editor's Note. The procedure for determining common EES is FFS.

#### 7.27.2.5	Discovery of common EAS Procedure

In this procedure, the EEC sends an EAS discovery request to an EES including AC association Profile. Based the AC association Profile, the EES determines an EAS discovery result that includes a common EAS for the AC association.

Pre-conditions:

1.	An AC association Profile has been provided to the EEC.

Figure 7.27.2.5-1: Common EAS discovery with AC association Profile

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes an AC association Profiles. Included in the AC association Profile is information regarding an association between AC(s) on this UE  and other ACs  indicating that the use of a common EAS is required.

2.	Upon receiving the request, the EES determines whether a common EAS is available to provide services to the associated ACs that meet the criteria specified in the AC association Profile

3.	If the processing of the request was successful, the EES responds to the EEC with an EAS discovery response which includes information about the discovered common EAS able to provide services to the associated ACs.

4.	If the EES is unable to determine a common EAS for the associated ACs, the EES responds with an appropriate failure cause.

#### 7.27.2.6	ACR with common EAS Procedure

Editor's Note. The ACR with common EAS procedure is FFS.

### 7.27.3	Solution evaluation

Editor's Note. Further investigation is required if this solution is feasible.

## 7.28	Solution #28: Common EAS discovery using EAS selection information

### 7.28.1	Architecture enhancements

None

### 7.28.2	Solution description

#### 7.28.2.1	General

The following solution corresponds to the key issue #17 on Discovery of a common EAS.

#### 7.28.2.2	Procedure

This solution is based on the following principles:

1.	Each EAS instance can be identified uniquely across EDNs.

2.	EAS Selection Information can be stored in the AC profile as the EAS selection information.

3.	For service provisioning, the EEC can include EAS selection information in the AC profile to discover the EES / EDN where a common EAS is available

4.	For EAS discovery, the EEC can include the EAS selection information in the AC profile to discover a common EAS.

This solution assumes that an EAS can only register to a single EES within an EDN, as per Rel-17 version of TS 23.558 [2] clause 6.6.3.3 on reference point cardinality for EDGE-3. Therefore, it is assumed that all UEs discovering the common EAS must do so via the EES where the common EAS is registered. Additionally, requirements such as EEC registration must be followed by the UEs discovering a common EAS.

Figure 7.28.2.2-1 presents an overview of the procedures for discovery of a common EAS; the scenario represented involves a first UE (e.g. UE-1/AC-1/EEC-1) discovering and using an EAS and a second UE (e.g. UE-2/AC-2/EEC-2) discovering the same EAS used by the first UE-1.

Figure 7.28.2.2-1: Common EAS discovery using EAS selection information

Pre-conditions:

-	The EAS has registered to the EES

1.	The AC-1 registers to EEC-1, and according to Rel-17 procedures defined in TS 23.558, the EEC-1 performs service provisioning with the ECS and performs EAS discovery with the EES. Additionally, the EEC can perform EAS selection declaration if required.

2.	The EEC-1 selects one of the discovered EAS to be used as the common EAS. The EEC-1 uses the EAS Selection Information to form the EAS selection information which can be used to uniquely identify the common EAS within and across EDNs. The EEC-1 can provide the EAS selection information to AC-1.

3.	Not shown on figure 7.28.2.2-1, AC-2 residing on UE-2 can be provided with EAS selection information. How AC-2 obtains the EAS selection information is out of SA6 scope, for example it may be obtained via direct interaction between AC-1 and AC-2 or interaction between AC-2 and the service provider of the EAS.

The AC-2 includes the EAS selection information in its AC profile and then registers with EEC-2, or alternatively updates its existing AC registration with EAS selection information.

NOTE:	If an EAS was discovered prior to receiving EAS selection information, the AC can update its AC profile to trigger step 4 and step 5 to discover the common EAS.

4.	The EEC-2 can perform a service provisioning procedure and includes the AC profile that contains the EAS selection information. The ECS considers the EAS selection information (e.g. the EESID) when selecting the EES and chooses the EES with a matching EESID. The service provisioning response includes the EDN configuration information that contains the EES where the common EAS is registered. Step 4 is optional if the EESID included in the EAS selection information is already known by EEC-2, in such case the EEC-2 can proceed to step 5. Not shown on the figure, the EEC may need to register to the EES if required by service provisioning.

5.	The EEC-2 selects the EES according to the EAS selection information. If the EAS profile of the common EAS is not known at the EEC-2, the EEC-2 can perform the EAS discovery with the selected EES, and includes the AC profile in the EAS discovery request. The EEC-2 performs the EAS selection declaration with the EES; the EES considers if the EEC-2 can use the common EAS. The EAS selection declaration response received at the EEC-2 indicates success/failure; if successful, the AC-2 can access the same common EAS as AC-1.

Editor's Note: Whether AC-1 and AC-2 always share the same ACID is FFS.

### 7.28.3	Solution evaluation

This clause provides an evaluation of the solution.

Editor's Note:	Further investigation is required if this solution is feasible.

## 7.29	Solution #29: Discovery of a common EAS

### 7.29.1	Architecture enhancements

None.

### 7.29.2	Solution description

#### 7.29.2.1	General

Editor's note:	To analyse the feasibility of this solution, further enhancements and clarifications to the description are required.

The following solution addresses open issues 1 and 2 of key issue #17, discovery of a common EAS.

Editor's note:	The solution for open issue 3, support of service continuity, is FFS.

Note:	The communication between AC and AS and between ACs is out of scope of SA6. In the following, some of this communication is only described to enable a better understanding of how the procedures under SA6 responsibility may be embedded in the overall procedure. The description is not intended to exclude scenarios using a different application level communication.

It is assumed that when users want to participate in a real-time multi-user session, their ACs will register with an application server (AS) in the cloud. The AS configures the ACs to form an AC group to which the AS assigns a globally unique Group ID (e.g., a UUID as described in RFC 4122).

During the registration with the AS, users will provide their UE location information to the AS. Dependent on the use case, the location information can be given, e.g., in the format of GPS coordinates, a Cell Identity or a Tracking Area Identity. Furthermore, dependent on the application, the users may be asked to consent to exchanging the location of their UEs with each other. If users consent, then the UE location information is exchanged between the ACs via application level signalling via the AS in the cloud.

Based on the UE location information received from the ACs, the AS determines a Requested EES geographical service area. Dependent on the use case this can be a single coordinate, e.g., the center of mass of the UE/AC locations or the location of a UE/AC taking a specific role in the group (e.g., in a V2X scenario the first truck in platoon), or it can be a geographical area including the current UE/AC locations (without disclosing individual UE/AC locations). The AS provides Group ID, Requested EES geographical service area and, if applicable, the locations of the other ACs to each of the ACs.

Dependent on the application, the distribution of the Group ID and the other parameters by the AS can be triggered by different events, e.g., upon explicit request of a user or when a certain minimum number of users have registered.

Upon receipt of the Group ID, together with the other parameters, each AC requests its EEC to perform EAS discovery.

During service provisioning, the Requested EES geographical service area is received by the ECS and used by it to determine a common EES, by comparing it with the EES geographical service area received from each EES during EES registration.

EAS discovery based on individual UE locations

Later, during EAS discovery, the EEC-1 of user 1 provides the common EES with its own location and with the locations of the other UEs/EECs. Based on the location information of all EECs involved in the session and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria, the EES determines a common EAS and indicates it to the EEC-1.

When EEC-2 of user 2 provides the EES with the same set of location information for all the EECs, then the EES will determine the same EAS again.

Dependent on the use case, the EES may apply different additional EAS selection criteria signalled by the EECs to determine the common EAS. E.g., it may be desirable to determine the EAS so that the latency for all the ACs in the session is approximately the same or that the latency for a specific AC is minimized.

EAS discovery based on other information

If users do not consent to share their individual UE location information with other users, then only the Requested EES geographical service area is distributed by the AS to the ACs via application level signalling. Additionally, the AS may send a "group size for EAS discovery" parameter to each AC.

Later, during EAS discovery, each EEC provides the common EES with its own location and optionally, in the Group profile IE, with the Expected Group geographical service area set to the Requested EES geographical service area or with the "group size for EAS discovery" parameter or both.

The EES may initiate determination of a common EAS for the EECs based on different criteria, dependent the received parameters and the application, e.g.:

-	immediately upon receipt of the first EAS discovery request, based on the Expected Group geographical service area, if available;

-	when the number of EAS discovery request is greater or equal to the "group size for EAS discovery" parameter; or

-	a certain time after receipt of the first EAS discovery request,

taking into account the UE location information received from the EECs by this point in time and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria.

Editor's Note:	Whether the group size should be "minimum group size" is FFS.

Handling of "late-coming" ACs

If a late-coming AC wants to join the multi-user group session after the initial EAS discovery was initiated, it may register with the AS. After registration it will receive from another AC in application layer signalling, e.g., via the AS, the EDN configuration information necessary to connect to the common EES. The details of this signalling are out-of-scope of SA6. The AC can then register with the EES and perform EAS discovery, providing its Group ID. The EES determines the EAS based on the Group ID.

#### 7.29.2.2	Procedure

The following procedure demonstrates the signalling for 2 UEs only, but the same principles can be applied to a larger number of UEs.

Pre-conditions:

1.	ACs have registered with an AS in the cloud to participate in a real-time multi-user session. During the registration, the AS provided the ACs with the Group ID and the Requested EES geographical service area (which may be a single coordinate or an area including the UE locations, see clause 7.29.2.1), and optionally with additional EAS selection criteria for the EES. Dependent on user consent, the AS also forwards the location information of the other UE(s) to each of the ACs. If user consent is not available, the AS may send a "Minimum group size for EAS discovery" parameter to the ACs.

1.-2.	Each AC requests its EEC to perform EAS discovery. The request includes the AC profile, the Group ID, the Requested EES geographical service area, and optionally the location information of the other UEs involved in the multi-user session in the request and the additional EAS selection criteria for the EES.

3.-4.	EEC1 and EEC2 initiate service provisioning, including the AC profile, the Group ID, the Group profile IE including the Requested EES geographical service area, and optionally a Group KPI IE.

Editor's Note:	Whether the Group profile IE and Group KPI IE are required is FFS

5.-6.	Based on the AC profile, the Requested EES geographical service area and the other information provided by the EEC, the ECS determines a suitable common EES and includes it in the service provisioning response. If more than one suitable EES is available, the ECS can use additional information as input parameter to determine a unique EES.

Note 1:	For example, the ECSs can use the Group ID and an algorithm shared between the ECSs to derive the common EES from the list of available EESs.

If the ECS wants to provision the EECs with additional EESs, e.g., for service continuity, then the common EES is included as first EES in the first entry of the list of EDN configuration information.

7.-8.	EEC1 and EEC2 initiate EAS discovery towards the common EES, including the AC profile, the Group ID, the Group profile IE which optionally includes the requested EES geographical service area (set to the Requested EES geographical service area) or the location information of all UEs involved in the multi-user session, the additional EAS selection criteria for the EES, and a Group KPI IE.

9.-10.	The EES initiates determination of a common EAS for the EECs based on different criteria, dependent the received parameters and the application, e.g.:

i)	immediately upon receipt of the first EAS discovery request, based on the Expected Group geographical service area or the location information of all UEs involved in the multi-user session, if received;

ii)	when the number of EAS discovery request is greater or equal to the "group size for EAS discovery" parameter; or

iii)	a pre-defined time after receipt of the first EAS discovery request,

taking into account the Expected Group geographical service area or the UE location information received by this point in time and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria.

Note 2:	The EES can also utilize the capabilities of the 3GPP core network to determine a UE location.

The EES includes the determined common EAS, e.g. EAS-1(A), in the EAS discovery response.

### 7.29.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.x	Solution #x: <title>

This clause describes the solution and its evaluation with a suitable title. Please also update Table 7.0-1 and Table 10.2.1-1.

### 7.x.1	Architecture enhancements

This clause summarizes the architecture enhancements required by this solution – please mention the option from clause 6 which is the basis for this solution. Also list any specific functional enhancements, new identities and cardinality rules that are required for this solution. If no new enhancements are required compared to the Rel-17 architecture, please mention 'None'.

### 7.x.2	Solution description

This clause describes the solution in detail. Please provide description, flow diagram, information flows and APIs for the new solution.

### 7.x.3	Solution evaluation

This clause provides an evaluation of the solution.

# 8	Deployment scenarios

## 8.1	<Title>

Provide an illustrative description of the deployment scenario.

# 9	Involved entities and relationships

## 9.1	Federation and Roaming

### 9.1.1	General

This clause describes the relationship of edge computing service providers, PLMN operators, application service providers and end users, taking federation and roaming into account.

Figure 9.1.1: Relationships involved in edge computing service – federation and roaming

The end user is the consumer of the applications provided by the application service provider (ASP). End user:

-	can have ASP service agreement with a single or multiple application service providers.

-	has a PLMN subscription arrangement with a PLMN operator (HPLMN). The UE used by the end user can register on the HPLMN network and network of its roaming partners.

-	can have authorization to access edge services of a single or multiple ECSPs.

The ASP consumes the edge services (e.g., infrastructure, platform) provided by the ECSP. ASP:

-	can have edge computing service provider service agreement with a single or multiple ECSPs.

The PLMN operator provides connectivity between the end user and the edge services provided by the ECSP. PLMN operator:

-	can have the PLMN operator service agreement with a single or multiple ECSPs.

-	can have service agreement for roaming including agreements for Edge Computing services, and/or federation with a single or multiple PLMN operators.

The ECSP provides the edge services. ECSP:

-	can have PLMN operator service agreement with a single or multiple PLMN operators which provide edge computing support.

-	can have federation partnership to share edge services with a single or multiple ECSPs.

The ECSP and the PLMN operator can be part of the same organization.

# 10	Overall evaluation

This clause provides a summary of architecture enhancements and solution evaluations.

## 10.1	Architecture enhancements

## 10.2	Key issue evaluations

### 10.2.1	General

All the key issues and solutions specified in this technical report are listed in Table 10.2.1-1.

The table provides a mapping of the key issues to the related solutions. It also indicates whether the solution requires enhancement to the Release-17 architecture, identities or cardinalities, and lists the dependencies on other working groups.

Table 10.2.1-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Solution (clause reference) | Enhancements required | Dependency on other working groups |
| --- | --- | --- | --- | --- |
| #1: <title> | #x: <title> | 6.x | Architecture / Identities / Cardinalities / None | <WG> |
|  | #y: <title> | 6.y | Architecture / Identities / Cardinalities / None | <WG> |

### 10.2.2	Key issue #14: Application traffic influence for initially selected EAS

This clause provides an overall evaluation of the key issue #14 Application traffic influence for initially selected EAS. The solution #9, solution #15 and solution #17 are complementary to each other.

The solution #9 is applicable to the scenario where the EAS triggers the EES to perform traffic influence without necessitating requests to be made on a per UE basis.

The solution #15 is applicable to the scenario where the EEC (or AC and EEC) selects the EAS after EAS discovery and then provides a selected EAS declaration request to the EES, which may be used by the EES to make traffic influence decisions if those haven't already been performed.

The solution #17 is applicable to the scenario where the EES selects the EAS and performs traffic influence immediately for the selected EAS after EEC sent EAS discovery request based on attributes in the EEC provided AC profile, providing the selected EAS to the EEC in the EAS discovery response.

All 3 solutions result in network resources being reserved to varying degrees before actual use, where this may be lower in the UE specific solutions of solution 15 and 17.

Editor's note:	Further evaluation is FFS according to the resolution of the ENs in clause 7.9 and clause 7.15.

### 10.2.x	Key issue #x

This clause provides an overall evaluation of all the solutions defined for Key Issue #x.

# 11	Conclusions

This clause provides conclusions of the study.

## 11.1	Architecture enhancements

## 11.2	Solutions

Annex A (Informative):
ETSI MEC and EDGEAPP system comparison

## A.1	General

This Annex compares EDGEAPP architecture R17 as defined in 3GPP TS 23.558 [2] and ETSI MEC architecture [3] and provides a gap analysis, addressing the objective in Key issue #5 in clause 4.5.

## A.2	Service consumer and service provider

The functionalities enabled via the Mp1 reference point between MEC applications and MEC platform is mainly described in ETSI GS MEC011 [14]. The related functionality includes MEC service registration/deregistration, MEC service discovery and event notifications. Other functionality includes MEC service availability, traffic rules, DNS and time of day.

From ETSI MEC's perspective, there are two types of MEC Applications, i.e., MEC Application that consumes MEC Services and MEC Application that provides MEC service(s). For the MEC Application that provides MEC service(s), the MEC Application sends a service registration request to the MEC platform to register the MEC service during the MEC Application start-up procedure. As for the MEC Application that consumes MEC Services, the MEC Application can send a service query request to the MEC platform to discover a MEC service. It should be noted that the API of registration and discovery is defined for MEC service.

In R17 of SA6, the EES can take the role of the CAPIF core function, and the vertical application enabler server acting the AEF and publish the vertical application enabler server APIs to the EES. Further, the vertical application enabler server APIs is discovered by the EASs acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [16].

In clause 4.2, the Key issue #2 plans to study Enablement of Service APIs exposed by EAS. The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC:

[Observation A.2-1] The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC. According to the Key issue #2 in clause 4.2, The EAS acting as a service provider is expected to be defined in R18 and expose service APIs.

## A.3	EAS/MEC application profile provisioning

ETSI MEC and EDGEAPP defined different style of EAS/MEC application profile provisioning. The information flows for lifecycle management of MEC applications is described in ETSI GS MEC010-2 [13]. The informational flows for the optional MEC Application registration are described in ETSI GS MEC 011 [14]. The MEC application can start producing or consuming MEC Services after the MEC Application is instantiated and running. The application information (AppInfo), which can be regarded as the MEC application profile, represents the information provided by the MEC application instance as part of the "application registration request" message. The attributes of the AppInfo are available from the clause 7.1.2.6 of ETSI GS MEC011 [14]:

Some fields in AppInfo are intentionally not duplicating the EAS profile (if present) with conflicting parameters but should be consistent with them. This is highlighted in NOTE 1 and NOTE 2, for example. It can be seen that unlike AppD [13], which is mainly used in the management plane for instantiating an application, and is static in nature, AppInfo carries the runtime information about the MEC application instance.

In EDGEAPP, the EAS profile is provided in the EAS registration request. According to clause 8.2.4 of 3GPP TS 23.558 [2] the information element of the EAS profile is listed as below:

Table A.3-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [14]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Comparison of AppInfo in 7.1.2.6 of ETSI GS MEC 011 [14] and EAS Profile in Table A.3.2 shows that:

[Observation A.3-1] Both ETSI MEC and EDGEAPP provides similar types of EAS/MEC application profile provisioning. Both the EAS Profile and MEC Application Instance (AppInfo) is provided during the application registration request.

[Observation A.3-2] Some IEs of the EAS profile may overlap with the ones defined in AppInfo (e.g., EAS ID vs. appName, EAS Provider Identifier vs. appProvider, EAS Endpoint vs. endpoint). Both AppInfo and EAS Profile has many optional IEs, whether and how to address their differences in SA6 is FFS.

## A.4	EAS registration and EAS discovery

In R17 of EDGEAPP, the EAS Registration procedure is defined to allow an EAS to provide its information to an EES in order to enable its discovery as defined in clause 8.4.3 of 3GPP TS 23.558 [2]. The EAS discovery procedure is used to provide EAS information to the EEC. After the EEC is provisioned with the EAS information, it can establish a connection to the EAS. Besides, in the service continuity scenario, the source EAS may send an EAS discovery request to the EES to discover a target EAS (providing same functionality as the source EAS) to serve the UE as defined in clause 8.8.3.2 of 3GPP TS 23.558 [2].

However, in current ETSI MEC specification, no APIs for MEC Application registration is defined because it is assumed that all MEC Application are on-boarded and managed by MEC Orchestrator, which was specified in ETSI GS MEC 010-2 [13]. API for MEC Application discovery is not defined since the existing MEC service is either defined from the MEC Application's perspective or it is consumed by the MEC Application rather than the UE.

Therefore, the comparison EAS registration and EAS discovery of EDGEAPP [2] and ETSI MEC specification [13] shows that:

[Observation A.4-1] The EAS registration and EAS discovery mechanism is defined in R17 of SA6 and while it is not defined in ETSI MEC introduced MEC application registration (ETSI GS MEC 011 v3.0.6). It is FFS whether and how to address such differences in SA6, e.g., in support of ETSI MEC.

[Observation A.4-2] ETSI MEC platform(MEP) only supports service registration. In the registration parameter "ServiceInfo"，there is a mandatory field "consumedLocalOnly" used to indicate whether the service can only be consumed by the MEC applications located in the same locality, which means ETSI MEC services (produced by Authorized MEC APPs) registered and exposed on MEP can be invoked by MEC consumer APPs deployed on the same or another MEC host.

Annex B (Informative): Deployment and Evolution options of EDGEAPP and ETSI MEC platforms

## B.1	General

This clause provides the analysis to address the first open issue of KI#5, which intends to study and analyse different deployment options of EDGEAPP and ETSI MEC platforms. In that regard, this clause describes the following foreseen types of deployment and evolution scenarios for deployment of EDGEAPP and ETSI MEC.

Annex C of TS 23.558 provides a relationship between EDGEAPP and ETSI MEC architectures as in figure below.

Figure B.1-1: Relationship in EDGEAPP and ETSI MEC architecture

Editor's note: Whether and how to enhance EDGE-9 or Mp3 is FFS.

Editor's Note: Refining overlapping content in this clause is FFS.

## B.2	Deployment options

### B.2.1	Deployment Option-1: Collocated Platforms

Based on Figure Y.1-1, the two platforms (EES and MEC Platform) are co-located, and made by a single (unique) equipment, which is compliant with both standards. However, the actual deployment details of two platforms is implementation specific (Figure B.2.1-1).

Figure B.2.1-1: EES and MEC Platform as two different AFs on a single Physical/NFV Infrastructure

### B.2.2	Deployment Option-2: Converged architecture

#### B.2.2.1	General

From the practical and business perspective, it is possible that an operator has deployed ETSI MEC architecture in its MEC sites to provide edge service since the stage 1 work of ETSI MEC has been already finished for a period of time. At the same time, the operator still cannot deploy EDGEAPP architecture since the stage 3 work of EDGEAPP is still not completed at the time being.

On the other hand, it is assumed that an enhanced architecture, including a converged architecture as depicted in Figure B.2.2-1, will be introduced after completion of release 18. The converged architecture is expected to satisfy the following requirement:

-	The MEP+EES is able to satisfy all the functionalities of MEP defined in ETSI and EES defined in SA6.

-	A uniform API is defined for the EAS and MEC app, i.e., EDGE-3 and Mp1 are unified into one interface and the EAS and MEC app will consume the same service from the MEP+EES.

-	EDGE-9 and Mp3 are unified into one interface.

Editor's note: Management of MEP+EES is under the scope of SA5 and the related reference is FFS.

Figure B.2.2.1-1 Converged architecture for EDGEAPP and ETSI MEC alignment

The two platforms (EES and MEC Platform) are co-located which can be implemented as a single AF (e.g. realized as one VNF) compliant with both standards (Figure B.2.2-2).

Figure B.2.2.1-2: EES and MEC Platform as a single AF on a single NFV Infrastructure (NFVI)

#### B.2.2.2	Evolution Options

##### B.2.2.2.1	General

In this clause the converged architecture is the architecture described in clause B.2.2 and satisfies the requirements listed in that clause.

##### B.2.2.2.2	Evolution Option #1- Enhancement of a deployed MEP to support the functionality of EES

After the completion of Release 18, an operator may determine to upgrade/evolve the ETSI MEC architecture to the converged architecture if the operator deployed ETSI MEC architecture in its MEC sites to provide edge service in the early stage. The MEP needs to be upgraded to the MEP+EES for supporting the functionality of EES defined in 3GPP SA6. The deployment and evolution scenario is depicted in Figure B.2.2.2.2-1.

Figure B.2.2.2.2-1 Evolution Option #1- An early stage deployed MEP is enhanced to support the functionality of EES in a later stage.

In Figure B.2.2.2.2-1, the EDN_1 and EDN_2 are deployed by the same operator.

##### B.2.2.2.3	Evolution Option #2 Enhancement of a deployed EES to support the functionality of MEP

After the completion of Release 18, an operator may opt to upgrade/evolve the EDGEAPP architecture to the converged architecture if the operator deployed EDGEAPP architecture in its EDNs to provide edge service in the early stage. The deployment and evolution scenario is depicted in Figure B.2.2.2.3-1.

Figure B.2.2.2.3-1  Evolution Option #2- Enhancement of a deployed EES to support the functionality of MEP

### B.2.3	Deployment Option-3: non-Collocated Platforms

The two platforms are non-collocated, and reside in two different data networks, where EES is in the Mobile Network Operator (MNO) domain while the ETSI MEC platform is in another MNO domain (Figure B.2.3-1).

Figure B.2.3-1: EES and MEC Platform as two different AFs in two different EDNs

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211758 |  |  |  | TR Skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | S6-211754, S6-211759, S6-211760 | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | S6-212048, S6-212054, S6-212087, S6-212103, S6-212115, S6-212142, S6-212166, S6-212167, S6-212168, S6-212169, S6-212170, S6-212171 | 0.2.0 |
| 2021-10 | SA6#45-BIS-e |  |  |  |  | S6-212478, S6-212402, S6-212479, S6-212480, S6-212481, S6-212482, S6-212207, S6-212411, S6-212206, S6-212403, S6-212483, S6-212401, S6-212484, S6-212486, S6-212456 | 0.3.0 |
| 2021-12 | SA6#46-BIS-e |  |  |  |  | S6-212720, S6-212730, S6-212748, S6-212796, S6-212807, S6-212813, S6-212829, S6-212830, S6-212831, S6-212832 | 0.4.0 |
| 2022-02 | SA6#47-e |  |  |  |  | S6-220077, S6-220457, S6-220458, S6-220459, S6-220460, S6-220340, S6-220461, S6-220274, S6-220420, S6-220462, S6-220479, S6-220299, S6-220102, S6-220463, S6-220464, S6-220465, S6-220466, S6-220468, S6-220480, S6-220469, S6-220470, S6-220349, S6-220454, S6-220455. | 0.5.0 |
| 2022-03 | SA6#47-e |  |  |  |  | Missing clause 7 from S6-220274 implemented. | 0.5.1 |
| 2022-04 | SA6#48-e |  |  |  |  | S6-220632, S6-220730, S6-220769, S6-220864, S6-220868, S6-220913, S6-220914, S6-220951, S6-220952, S6-220953, S6-220954, S6-220955, S6-220956, S6-220957, S6-220958, S6-220959, S6-220960, S6-220961, S6-220962, S6-220963, S6-220966, S6-220977, S6-220978. | 0.6.0 |