| 3GPP TS 24.559 V0.4.0 (2024-01) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals Application Data Analytics Enablement Service; Stage 3; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Abbreviations	10

4	General description	10

4.1	Overview	10

5	Functional entities	10

5.1	Application data analytics enablement server (ADAES)	10

5.2	Application data analytics enablement client (ADAEC)	11

6	Application data analytics enablement service API	11

6.1	General	11

6.2	Application performance analytics	11

6.2.1	Service description	11

6.2.1.1	Overview	11

6.2.2	Service Operations	11

6.2.2.1	Introduction	11

6.2.2.2	Subscribe_VAL_Performance_Analytics	12

6.2.2.2.1	General	12

6.2.2.2.2	Subscribing to VAL performance analytics event using Subscribe_VAL_Performance_Analytics service operation	12

6.2.2.3	Notify_VAL_Performance_Analytics	12

6.2.2.3.1	General	12

6.2.2.3.2	Notifying VAL performance analytics event using Notify_VAL_Performance_Analytics service operation	12

6.2.2.4	Unsubscribe_VAL_Performance_Analytics	12

6.2.2.4.1	General	12

6.2.2.4.2	Unsubscribing from VAL performance analytics event using Unsubscribe_VAL_Performance_Analytics service operation	12

6.3	UE-to-UE session performance analytics	13

6.3.1	Service description	13

6.3.1.1	Overview	13

6.3.2	Service Operations	13

6.3.2.1	Introduction	13

6.3.2.2	Fetch_UE2UE_Session_Performance_Analytics	13

6.3.2.2.1	General	13

6.3.2.2.2	Obtaining UE-to-UE session performance analytics using Fetch_UE2UE_Session_Performance_Analytics service operation	13

6.4	Edge load data collection	14

6.4.1	Service description	14

6.4.2	Service Operations	14

6.4.2.1	Introduction	14

6.4.2.2	Subscribe_Edge_Load_Data_Collection	14

6.4.2.2.1	General	14

6.4.2.2.2	Subscribing to edge load data collection event using Subscribe_Edge_Load_Data_Collection service operation	14

6.4.2.3	Notify_Edge_Load_Data_Collection	15

6.4.2.3.1	General	15

6.4.2.3.2	Notifying edge load data collection event using Notify_Edge_Load_Data_Collection service operation	15

6.4.2.4	Unsubscribe_Edge_Load_Data_Collection	15

6.4.2.4.1	General	15

6.4.2.4.2	Unsubscribing from edge load data collection event using Unsubscribe_Edge_Load_Data_Collection service operation	15

6.5	Service experience performance analytics	16

6.5.1	General	16

6.5.2	Service Operations	16

6.5.2.1	Introduction	16

6.5.2.2	Configure_Triggers_Service_Information_Experience_Report	16

6.5.2.2.1	General	16

6.5.2.2.2	Configuring service experience information reporting using Configure_Triggers_Service_Information_Experience_Report service operation	16

6.5.2.3	Service_Experience_Information_Report	17

6.5.2.3.1	General	17

6.5.2.3.2	Reporting service experience information using Service_Experience_Infomation_Report service operation	17

6.5.2.4	Abolish_Triggers_Service_Experience_Information_Report	17

6.5.2.4.1	General	17

6.5.2.4.2	Abolishing service experience information reporting using Abolish_Triggers_Service_Experience_Information_Report service operation	17

6.5.2.5	Push_Service_Experience_Information_Report	18

6.5.2.5.1	General	18

6.5.2.5.2	Pushing service experience information report using Push_Service_Experience_Information_Report service operation	18

6.5.2.6	Pull_Service_Experience_Information_Report	18

6.5.2.6.1	General	18

6.5.2.6.2	Pulling service experience information report using Pull_Service_Experience_Information_Report service operation	18

7	API Definitions	19

7.1	ADAE_ServiceConfiguration API	19

7.1.1	Introduction	19

7.1.2	Usage of HTTP 	19

7.1.2.1	General	19

7.1.2.2	Content type	19

7.1.3	Resources	19

7.1.3.1	Overview	19

7.1.3.2	Resource: Application performance event subscription	21

7.1.3.2.1	Description	21

7.1.3.2.2	Resource definition	21

7.1.3.2.3	Resource standard methods	21

7.1.3.2.3.1	POST	21

7.1.3.2.4	Resource custom operations	22

7.1.3.3	Resource: Individual application performance event subscription	22

7.1.3.3.1	Description	22

7.1.3.3.2	Resource Definition	22

7.1.3.3.3	Resource Standard Methods	22

7.1.3.3.3.1	DELETE	22

7.1.3.3.4	Resource Custom Operations	23

7.1.3.4	Resource: UE-to-UE session performance analytics	23

7.1.3.4.1	Description	23

7.1.3.4.2	Resource definition	23

7.1.3.4.3	Resource standard methods	23

7.1.3.4.4	Resource custom operations	23

7.1.3.4.4.1	Overview	23

7.1.3.4.4.2	Fetch	23

7.1.3.5	Resource: Edge load data collection event subscription	24

7.1.3.5.1	Description	24

7.1.3.5.2	Resource definition	24

7.1.3.5.3	Resource standard methods	24

7.1.3.5.3.1	POST	24

7.1.3.5.4	Resource custom operations	25

7.1.3.6	Resource: Individual edge load event subscription	25

7.1.3.6.1	Description	25

7.1.3.6.2	Resource Definition	25

7.1.3.6.3	Resource Standard Methods	25

7.1.3.6.3.1	DELETE	25

7.1.3.6.4	Resource Custom Operations	26

7.1.3.7	Resource: Service experience information	26

7.1.3.7.1	Description	26

7.1.3.7.2	Resource definition	26

7.1.3.7.3	Resource standard methods	26

7.1.3.7.3.1	POST	26

7.1.3.7.4	Resource custom operations	27

7.1.3.7.4.1	Overview	27

7.1.3.7.4.2	Operation: PUSH Service Experience Information	27

7.1.3.7.4.3	Operation: PULL Service Experience Information	28

7.1.3.8	Resource: Individual trigger configuration for service experience information reporting	28

7.1.3.8.1	Description	28

7.1.3.8.2	Resource Definition	28

7.1.3.8.3	Resource Standard Methods	29

7.1.3.8.3.1	DELETE	29

7.1.3.8.4	Resource Custom Operations	29

7.1.4	Notifications	29

7.1.4.1	General	29

7.1.4.2	Application performance event notification	29

7.1.4.2.1	Description	29

7.1.4.2.2	Notification definition	29

7.1.4.3	Edge load event notification	30

7.1.4.3.1	Description	30

7.1.4.3.2	Notification definition	30

7.1.4.4	Service experience information report event notification	31

7.1.4.4.1	Description	31

7.1.4.4.2	Notification definition	31

7.1.5	Data model	31

7.1.5.1	General	31

7.1.5.2	Structured data types	32

7.1.5.2.1	Introduction	32

7.1.5.2.2	Type: Ue2UePerfReq	33

7.1.5.2.3	Type: Ue2UePerfResp	33

7.1.5.2.4	Type: ConfigRepTrigger	33

7.1.5.2.5	Type: ValServSpecCrit	34

7.1.5.2.6	Type: PullSrvExpInfo	34

7.1.5.2.7	Type: SrvExpInfoRep	34

7.1.5.3	Simple data types and enumerations	34

7.1.5.3.1	Introduction	34

7.1.5.3.2	Simple data types	34

7.1.5.3.3	Enumeration: SrvExpRepCrit	35

7.1.6	Error Handling	35

7.1.6.1	General	35

7.1.6.2	Protocol Errors	35

7.1.6.3	Application Errors	35

7.1.7	Feature Negotiation	35

8	Usage of common API framework	35

8.1	General	35

9	Security	36

9.1	General	36

Annex <A> (normative): OpenAPI specification	37

A.1	General	37

A.2	ADAE_ServiceConfiguration API	37

Annex <C> (informative): Change history	46

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects of ADAE of SEAL services. The protocol aspects specify the UE supporting the client functionality of the ADAE SEAL services and the network supporting the server functionality of ADAE SEAL services, where the client functionality and server functionality are specified in 3GPP TS 23.436 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.900: "Technical Specification Group working methods".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[4]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service".

[7]	3GPP TS 29.549:" Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

[9]	3GPP TS 33.434: "Service Enabler Architecture Layer for Verticals (SEAL); Security Aspects".

[10]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[11]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[12]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[13]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[14]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[15]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[16]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[17]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[18]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[19]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[20]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [2].

ADAE client: An entity that provides the client side functionalities corresponding to the ADAE.

ADAE server: An entity that provides the server side functionalities corresponding to the ADAE.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.436 [3] apply:

ADAE service

SEAL server

SEAL service

VAL application

VAL server

VAL service

VAL client

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

ADAE	Application Data Analytics Enablement

ADAE-C	Application Data Analytics Enablement Client

ADAE-S	Application Data Analytics Enablement Server

API	Application Programming Interface

SEAL	Service Enabler Architecture Layer

UE	User Equipment

VAL	Vertical Application Layer

# 4	General description

## 4.1	Overview

Application data analytics enablement service enables an application data analytics enablement client (ADAEC) and a vertical application layer (VAL) server to communicate with an application data analytics enablement server (ADAES).

# 5	Functional entities

## 5.1	Application data analytics enablement server (ADAES)

The ADAES is a functional entity with a unique identity, ADAES ID, in the PLMN and provides data analytics to administer the operation and performance of one or more VAL applications.

To be compliant with the HTTP procedures in the present document the ADAES:

a)	shall support the role of XCAP server as specified in IETF RFC 4825 [10]; and

b)	shall support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [19].

## 5.2	Application data analytics enablement client (ADAEC)

The ADAEC functional entity with a unique identity, ADAEC ID, and act as the VAL application client which provides data analytics for managing network slice capabilities.

To be compliant with the HTTP procedures in the present document the ADAEC:

a)	shall support the role of XCAP client as specified in IETF RFC 4825 [10]; and

b)	shall support the role of XDMC as specified in OMAOMA-TS-XDM_Core-V2_1 [19].

# 6	Application data analytics enablement service API

## 6.1	General

The clause describes the procedures of the application data analytics enablement service API.

## 6.2	Application performance analytics

### 6.2.1	Service description

#### 6.2.1.1	Overview

The ADAE_ServiceConfiguration API, as defined 3GPP TS 23.436 [3], allows the ADAES via ADAE-UU reference point to subscribe to ADAEC to the event of the VAL performance analytics.

### 6.2.2	Service Operations

#### 6.2.2.1	Introduction

The service operation defined for ADAE_ServiceConfiguration API for application performance analytics is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations for application performance analytics

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_VAL_Performance_Analytics | This service operation is used by ADAES to subscribe to the event of the VAL performance analytics. | ADAES |
| Notify_VAL_Performance_Analytics | This service operation is used by ADAEC to notify about the VAL performance analytics. | ADAEC |
| Unsubscribe_VAL_Performance_Analytics | This service operation is used by ADAES to unsubscribe from the event of the VAL performance analytics. | ADAES |

#### 6.2.2.2	Subscribe_VAL_Performance_Analytics

##### 6.2.2.2.1	General

This service operation is used by the ADAES for VAL performance analytics event subscription to the ADAEC.

##### 6.2.2.2.2	Subscribing to VAL performance analytics event using Subscribe_VAL_Performance_Analytics service operation

To subscribe to VAL performance analytics event, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/application-performance" and with a body containing data type AppPerfSub as defined in clause 7.10.1.4.2.2 of 3GPP TS 29.549 [7].

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to subscribe to the VAL performance analytics event; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall create a new "Individual application performance event subscription" resource and respond to the ADAES with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual application performance event subscription" and the response body including the AppPerfSub data structure containing a representation of the created resource as defined in clause 7.1.3.

#### 6.2.2.3	Notify_VAL_Performance_Analytics

##### 6.2.2.3.1	General

This service operation is used by the ADAEC to send notification to the ADAES with the VAL performance analytics event subscription to the ADAEC.

##### 6.2.2.3.2	Notifying VAL performance analytics event using Notify_VAL_Performance_Analytics service operation

To notify VAL performance analytics event, the ADAEC shall send an HTTP POST request with a Request-URI according to the pattern "{notifUri}" and with a body containing data type AppPerfNotif as defined in clause 7.10.1.4.2.3 of 3GPP TS 29.549 [7].

Upon receipt of the HTTP POST request, the ADAES shall respond to the ADAEC with:

a)	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

b)	if errors occur when processing the request, an appropriate error response as specified in clause 7.1.6.

#### 6.2.2.4	Unsubscribe_VAL_Performance_Analytics

##### 6.2.2.4.1	General

This service operation is used by the ADAES to unsubscribe from the VAL performance analytics event.

##### 6.2.2.4.2	Unsubscribing from VAL performance analytics event using Unsubscribe_VAL_Performance_Analytics service operation

To unsubscribe from VAL performance analytics event, the ADAES shall send an HTTP DELETE request to the resource representing the event in the ADAES as specified in clause 7.1.3.3.

Upon receiving the HTTP DELETE request:

a)	the ADAEC shall verify the identity of the ADAES and check if the ADAES is authorized to unsubscribe from the VAL performance analytics event associated with the resource URI "{apiRoot}/adae-sc/<apiVersion>/application-performance/{appPerfId}";

b)	if the ADAES is authorized to unsubscribe from the VAL performance analytics event, the ADAEC shall delete the resource pointed by the resource URI "{apiRoot}/adae-sc/<apiVersion>/application-performance/{appPerfId}";

c)	if the request is successfully processed, the ADAEC shall respond to the ADAES with a "204 No Content" status code; and

d)	if errors occur when processing the request, the ADAEC shall respond to the ADAES with an appropriate error response as specified in clause 7.1.6.

## 6.3	UE-to-UE session performance analytics

### 6.3.1	Service description

#### 6.3.1.1	Overview

The ADAE_ServiceConfiguration API, as defined 3GPP TS 23.436 [3], allows the ADAES via ADAE-UU reference point, to obtain the UE-to-UE session performance analytics from the ADAEC.

### 6.3.2	Service Operations

#### 6.3.2.1	Introduction

The service operation defined for ADAE_ServiceConfiguration API for UE-to-UE session performance analytics is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations for UE-to-UE session performance analytics

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Fetch_UE2UE_Session_Performance_Analytics | This service operation is used by ADAES to obtain the UE-to-UE session performance analytics. | ADAES |

#### 6.3.2.2	Fetch_UE2UE_Session_Performance_Analytics

##### 6.3.2.2.1	General

This service operation is used by the ADAES for obtaining the UE-to-UE session performance analytics from the ADAEC.

##### 6.3.2.2.2	Obtaining UE-to-UE session performance analytics using Fetch_UE2UE_Session_Performance_Analytics service operation

To obtain the UE-to-UE session performance analytics, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance/fetch" and with a body containing data type Ue2UePerfReq as defined in clause 7.1.5.2.2.

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to obtain the UE-to-UE session performance analytics; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall respond to the ADAES with an HTTP "200 OK" status code with the response body including the Ue2UePerfResp as defined in clause 7.1.3.3.4.2 with the following attributes:

i)	UE-to-UE session performance analytics;

ii)	one or more VAL UEs; and

iii)	identity of the UE-to-UE session performance analytics.

## 6.4	Edge load data collection

### 6.4.1	Service description

The ADAE_ServiceConfiguration API, as defined 3GPP TS 23.436 [3], allows the ADAES via ADAE-UU reference point to subscribe to ADAEC to the event of the edge load data collection.

### 6.4.2	Service Operations

#### 6.4.2.1	Introduction

The service operation defined for ADAE_ServiceConfiguration API for edge load data collection is shown in the table 6.4.2.1-1.

Table 6.4.2.1-1: Operations for edge load data collection

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Edge_Load_Data_Collection | This service operation is used by ADAES to subscribe to the event of the edge load data collection. | ADAES |
| Notify_Edge_Load_Data_Collection | This service operation is used by ADAEC to notify about the edge load data collection. | ADAEC |
| Unsubscribe_Edge_Load_Data_Collection | This service operation is used by ADAES to unsubscribe from the edge load data collection. | ADAES |

#### 6.4.2.2	Subscribe_Edge_Load_Data_Collection

##### 6.4.2.2.1	General

This service operation is used by the ADAES for edge load data collection event subscription to the ADAEC.

##### 6.4.2.2.2	Subscribing to edge load data collection event using Subscribe_Edge_Load_Data_Collection service operation

To subscribe to edge load data collection event, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/edge-load" and with a body containing data type EdgeSubs as defined in clause 7.10.7.4.2.2 of 3GPP TS 29.549 [7].

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to subscribe to the edge load data collection event; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall create a new "Individual edge load event subscription" resource and respond to the ADAES with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual edge load event subscription" and the response body including the EdgeSubs data structure containing a representation of the created resource as defined in clause 7.1.3.

#### 6.4.2.3	Notify_Edge_Load_Data_Collection

##### 6.4.2.3.1	General

This service operation is used by the ADAEC to send notification to the ADAES with the edge load data collection event subscription to the ADAEC.

##### 6.4.2.3.2	Notifying edge load data collection event using Notify_Edge_Load_Data_Collection service operation

To notify edge load data collection event, the ADAEC shall send an HTTP POST request with a Request-URI according to the pattern "{notifUri}" and with a body containing data type EdgeNotif as defined in clause 7.10.7.4.2.3 of 3GPP TS 29.549 [7];

Upon receipt of the HTTP POST request, the ADAES shall respond to the ADAEC with:

a)	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

b)	if errors occur when processing the request, an appropriate error response as specified in clause 7.1.6.

#### 6.4.2.4	Unsubscribe_Edge_Load_Data_Collection

##### 6.4.2.4.1	General

This service operation is used by the ADAES to unsubscribe from the edge load data collection event.

##### 6.4.2.4.2	Unsubscribing from edge load data collection event using Unsubscribe_Edge_Load_Data_Collection service operation

To unsubscribe from edge load data collection event, the ADAES shall send an HTTP DELETE request to the resource representing the event in the ADAES as specified in clause 7.1.3.6.

Upon receiving the HTTP DELETE request:

a)	the ADAEC shall verify the identity of the ADAES and check if the ADAES is authorized to unsubscribe from the edge load data collection event associated with the resource URI "{apiRoot}/adae-sc/<apiVersion>/edge-load/{edgeLdId}";

b)	if the ADAES is authorized to unsubscribe from the edge load data collection event, the ADAEC shall delete the resource pointed by the resource URI "{apiRoot}/adae-sc/<apiVersion>/edge-load/{edgeLdId}";

c)	if the request is successfully processed, the ADAEC shall respond to the ADAES with a "204 No Content" status code; and

d)	if errors occur when processing the request, the ADAEC shall respond to the ADAES with an appropriate error response as specified in clause 7.1.6.

## 6.5	Service experience performance analytics

### 6.5.1	General

The ADAE_ServiceConfiguration API, as defined 3GPP TS 23.436 [3], allows the ADAES via ADAE-UU reference point to:

-	configure the ADAEC with triggers for reporting service experience information;

-	configure the ADAEC to push the service experience information report; and

-	pull from the ADAEC, the service experience information report.

### 6.5.2	Service Operations

#### 6.5.2.1	Introduction

The service operation defined for ADAE_ServiceConfiguration API for service experience information is shown in the table 6.5.2.1-1.

Table 6.5.2.1-1: Operations for service experience information

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Configure_Triggers_Service_Experience_Information_Report | This service operation is used by ADAES to configure triggers for service experience information report. | ADAES |
| Service_Experience_Information_Report | This service operation is used by ADAEC to service experience information report. | ADAEC |
| Abolish_Triggers_Service_Experience_Information_Report | This service operation is used by ADAES to abolish already configured triggers for service experience information report. | ADAES |
| Push_Service_Experience_Information_Report | This service operation is used by ADAEC to push service experience information report. | ADAEC |
| Pull_Service_Experience_Information_Report | This service operation is used by ADAES to pull service experience information report. | ADAES |

#### 6.5.2.2	Configure_Triggers_Service_Information_Experience_Report

##### 6.5.2.2.1	General

This service operation is used by the ADAES to configure the ADAEC with triggers for reporting the service experience information.

##### 6.5.2.2.2	Configuring service experience information reporting using Configure_Triggers_Service_Information_Experience_Report service operation

To configure triggers for reporting service experience information, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/service-experience" and with a body containing data type ConfigRepTrigger as defined in clause 7.1.5.2.4.

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to configure triggers for service experience information report; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall create a new "Individual service experience information" resource and respond to the ADAES with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual service experience information " and the response body including the ConfigRepTrigger data structure containing a representation of the created resource as defined in clause 7.1.3.

#### 6.5.2.3	Service_Experience_Information_Report

##### 6.5.2.3.1	General

This service operation is used by the ADAEC which is configured by VAL client or other triggers to send service experience information report to the ADAES.

##### 6.5.2.3.2	Reporting service experience information using Service_Experience_Infomation_Report service operation

To report service experience information, the ADAEC shall send an HTTP POST request with a Request-URI according to the pattern "{notifUri}" and with a body containing data type SrvExpInfoRep as defined in clause 7.1.5.2.7.

Upon receipt of the HTTP POST request, the ADAES shall respond to the ADAEC with:

a)	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

b)	if errors occur when processing the request, an appropriate error response as specified in clause 7.1.6.

Editor's note: 	Whether the data type SrvExpInfoRep can be used by the ADAEC which is configured by triggers for service experience information reporting, is FFS.

#### 6.5.2.4	Abolish_Triggers_Service_Experience_Information_Report

##### 6.5.2.4.1	General

This service operation is used by the ADAES to abolish the configured triggers on the ADAEC for the service experience information reporting.

##### 6.5.2.4.2	Abolishing service experience information reporting using Abolish_Triggers_Service_Experience_Information_Report service operation

To abolish triggers from the configured service experience information reporting, the ADAES shall send an HTTP DELETE request to the resource representing the event in the ADAES as specified in clause 7.1.3.8.

Upon receiving the HTTP DELETE request:

a)	the ADAEC shall verify the identity of the ADAES and check if the ADAES is authorized to abolish triggers from the configured service experience information reporting associated with the resource URI "{apiRoot}/adae-sc/<apiVersion>/service-experience/{srvTrigId}";

b)	if the ADAES is authorized to abolish triggers from the configured service experience information reporting, the ADAEC shall delete the resource pointed by the resource URI "{apiRoot}/adae-sc/<apiVersion>/service-experience/{srvTrigId}";

c)	if the request is successfully processed, the ADAEC shall respond to the ADAES with a "204 No Content" status code; and

d)	if errors occur when processing the request, the ADAEC shall respond to the ADAES with an appropriate error response as specified in clause 7.1.6.

#### 6.5.2.5	Push_Service_Experience_Information_Report

##### 6.5.2.5.1	General

This service operation is used by the ADAEC to push the service experience information report to the ADAES.

Editor's note:	Whether the mechanism for PUSH experience information report is the same as the mechanism for service experience information report in clause 6.5.2.3, is FFS.

##### 6.5.2.5.2	Pushing service experience information report using Push_Service_Experience_Information_Report service operation

To push the service experience information report to the ADAES, the ADAES shall configure the ADAEC by sending an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/service-experience/push" and with a body containing data type Uri as defined in clause 5.2.2 of 3GPP TS 29.571 [8].

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to configure push for service experience information report; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall respond to the ADAES with an HTTP "200 OK" status code.

Editor's note:	Whether the body of the HTTTP "200 OK" contains data type SrvExpInfoRep as defined in clause 7.1.5.2.7 for transport of the information from ADAEC to ADAES or the information is transported by means described in 3GPP TS 26.531 and 3GPP TS 26.532.

#### 6.5.2.6	Pull_Service_Experience_Information_Report

##### 6.5.2.6.1	General

This service operation is used by the ADAES to pull the service experience information report from the ADAEC.

##### 6.5.2.6.2	Pulling service experience information report using Pull_Service_Experience_Information_Report service operation

To pull the service experience information report from the ADAEC, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/service-experience/pull" and with a body containing data type PullSrvExpInfo as defined in clause 7.1.5.2.6.

Upon receipt of the HTTP POST request:

a)	the ADAEC shall verify the identity of the ADAES and determine if the ADAES is authorized to pull the service experience information report; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall respond to the ADAES with an HTTP "200 OK" status code and with a body containing data type SrvExpInfoRep as defined in clause 7.1.5.2.7.

Upon receipt of the HTTP POST request, the ADAES shall respond to the ADAEC with a "204 No Content" status code and process the report.

# 7	API Definitions

## 7.1	ADAE_ServiceConfiguration API

### 7.1.1	Introduction

The HTTP URIs used in HTTP protocol for the ADAE service shall have the resource URI structure as defined in clause TBD:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

where:

a)	{apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4];

b)	<apiName> shall be "adae-sc";

c)	<apiVersion> shall be "v1"; and

d)	The <apiSpecificSuffixes> shall be set as described in clause 7.1.3.

### 7.1.2	Usage of HTTP

#### 7.1.2.1	General

For ADAE service configuration API, support of HTTP/1.1 (IETF RFC 7230 [11], IETF RFC 7231 [12], IETF RFC 7232 [13], IETF RFC 7233 [14], IETF RFC 7234 [15] and IETF RFC 7235 [16]) over TLS is mandatory and support of HTTP/2 (IETF RFC 7540 [17]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [17].

#### 7.1.2.2	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 8259 [18]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 8259 [18].

### 7.1.3	Resources

#### 7.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.3.1-1 depicts the resource URI structure of the ADAE_ServiceConfiguration API.

Figure 7.1.3.1-1: Resource URI structure of the ADAE_ServiceConfiguration API

Table 7.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Application performance event subscription | /application-performance | POST | Subscription to the VAL performance analytics event |
| Individual application performance event subscription | /application-performance/{appPerfId} | DELETE | Deletes an individual VAL performance analytics event |
| Edge load event subscription | /edge-load | POST | Subscription to the edge load data collection event |
| Individual edge load event subscription | /edge-load/{edgeLdId} | DELETE | Deletes an individual edge load data collection subscription |
| Service experience | /service-experience | POST | Configure triggers for reports on the service experience information |
| Individual service experience | /service-experience/{srvTrigId} | DELETE | Delete an individual trigger-configuration for service experience report |

#### 7.1.3.2	Resource: Application performance event subscription

##### 7.1.3.2.1	Description

Application performance event subscription is used by the ADAES to subscribe to the ADAEC for the event of the VAL performance analytics.

##### 7.1.3.2.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/application-performance

This resource shall support the resource URI variables defined in the table 7.1.3.2.2-1.

Table 7.1.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |

##### 7.1.3.2.3	Resource standard methods

###### 7.1.3.2.3.1	POST

This operation is for subscription to the VAL application performance analytics and shall support the URI query parameters specified in table 7.1.3.2.3.1-1.

Table 7.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.3.1-2and the response data structures and response codes specified in table 7.1.3.2.3.1-3.

Table 7.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfSub | M | 1 | Subscription to the VAL performance analytics event |

Table 7.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppPerfSub | M | 1 | 201 Created | Subscription to the VAL performance analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 7.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adae-sc/<apiVersion>/application-performance/{appPerfId} |

##### 7.1.3.2.4	Resource custom operations

None.

#### 7.1.3.3	Resource: Individual application performance event subscription

##### 7.1.3.3.1	Description

The individual application performance event subscription resource represents an individual event subscription of the ADAES.

##### 7.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/application-performance/{appPerfId}

This resource shall support the resource URI variables defined in the table 7.1.3.3.2-1.

Table 7.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |
| appPerfId | string | Identifies an application performance event subscription |

##### 7.1.3.3.3	Resource Standard Methods

##### 7.1.3.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.1.3.3.3.1-1.

Table 7.1.3.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.3.1-2 and the response data structures and response codes specified in table 7.1.3.3.3.1-3.

Table 7.1.3.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual application performance event subscription matching the appPerfId is deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 7.1.3.3.4	Resource Custom Operations

None.

#### 7.1.3.4	Resource: UE-to-UE session performance analytics

##### 7.1.3.4.1	Description

This resource is used by the ADAES to request the ADAEC for the UE-to-UE session performance analytics.

##### 7.1.3.4.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance

This resource shall support the resource URI variables defined in the table 7.1.3.4.2-1.

Table 7.1.3.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |

##### 7.1.3.4.3	Resource standard methods

None

##### 7.1.3.4.4	Resource custom operations

###### 7.1.3.4.4.1	Overview

Table 7.1.3.4.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /ue2ue-session-performance/fetch | POST | Request for the UE-to-UE session performance analytics |

###### 7.1.3.4.4.2	Fetch

This custom operation is for the ADAES to request the ADAEC the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.1.3.4.4.2-1.

Table 7.1.3.4.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This custom operation shall support the request data structures specified in table 7.1.3.4.4.2-2 and the response data structures and response codes specified in table 7.1.3.4.4.2-3.

Table 7.1.3.4.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Ue2UePerfReq | M | 1 | ADAES requests ADAEC for the UE-to-UE session performance analytics |

Table 7.1.3.4.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Ue2UePerfResp | M | 1 | 200 OK | ADAEC responses ADAES the UE-to-UE session performance analytics |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

#### 7.1.3.5	Resource: Edge load data collection event subscription

##### 7.1.3.5.1	Description

Edge load data collection event subscription is used by the ADAES to subscribe to the ADAEC for the event of the edge load data collection.

##### 7.1.3.5.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/edge-load

This resource shall support the resource URI variables defined in the table 7.1.3.5.2-1.

Table 7.1.3.5.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |

##### 7.1.3.5.3	Resource standard methods

###### 7.1.3.5.3.1	POST

This method is the ADAES to subscribe to the ADAEC for the event of the edge-load data collection and shall support the URI query parameters specified in table 7.1.3.5.3.1-1.

Table 7.1.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.5.3.1-2 and the response data structures and response codes specified in table 7.1.3.5.3.1-3.

Table 7.1.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeSubs | M | 1 | Subscription to the edge load data collection event |

Table 7.1.3.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EdgeSubs | M | 1 | 201 Created | Subscription to the edge load data collection is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 7.1.3.5.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adae-sc/<apiVersion>/edge-load/{edgeLdId} |

##### 7.1.3.5.4	Resource custom operations

None.

#### 7.1.3.6	Resource: Individual edge load event subscription

##### 7.1.3.6.1	Description

The individual edge load event subscription resource represents an individual event subscription of the ADAE server.

##### 7.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/edge-load/{edgeLdId}

This resource shall support the resource URI variables defined in the table 7.1.3.6.2-1.

Table 7.1.3.6.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |
| edgeLdId | string | Identifies an edge load data collection event subscription |

##### 7.1.3.6.3	Resource Standard Methods

##### 7.1.3.6.3.1	DELETE

This method shall support the URI query parameters specified in table 7.1.3.6.3.1-1.

Table 7.1.3.6.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.6.3.1-2 and the response data structures and response codes specified in table 7.1.3.6.3.1-3.

Table 7.1.3.6.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.6.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual edge load data collection event subscription matching the edgeLdId is deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 7.1.3.6.4	Resource Custom Operations

None.

#### 7.1.3.7	Resource: Service experience information

##### 7.1.3.7.1	Description

The resource is used by the ADAES to:

a)	configure the ADAEC with triggers for reporting service experience information;

b)	configure the ADAEC to push the service experience information report to the ADAES; and

c)	pull the service experience information report from the ADAEC.

##### 7.1.3.7.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/service-experience

This resource shall support the resource URI variables defined in the table 7.1.3.7.2-1.

Table 7.1.3.7.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |

##### 7.1.3.7.3	Resource standard methods

###### 7.1.3.7.3.1	POST

This operation is used by the ADAES to configure ADAEC with triggers for reporting the service experience information and shall support the URI query parameters specified in table 7.1.3.7.3.1-1.

Table 7.1.3.7.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.7.3.1-2 and the response data structures and response codes specified in table 7.1.3.7.3.1-3.

Table 7.1.3.7.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConfigRepTrigger | M | 1 | Configure triggers for service experience information reporting |

Table 7.1.3.7.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ConfigRepTrigger | M | 1 | 201 Created | Configuration of service experience information reporting was successful. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 7.1.3.7.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adea-sc/<apiVersion>/service-experience /{srvTrigId}. |

##### 7.1.3.7.4	Resource custom operations

###### 7.1.3.7.4.1	Overview

Table 7.1.3.7.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Push | /service-experience/push | POST | Push a service experience information report |
| Pull | /service-experience/pull | POST | Pull a service experience information report |

###### 7.1.3.7.4.2	Operation: PUSH Service Experience Information

This method is used by the ADAEC to push the service experience information report to the ADAES and shall support the URI query parameters specified in table 7.1.3.7.4.2-1.

Table 7.1.3.7.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.7.4.2-2 and the response data structures and response codes specified in table 7.1.3.7.4.2-3.

Table 7.1.3.7.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Uri | M | 1 | Configure to report the service experience information |

Table 7.1.3.7.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvExpInfoRep | M | 1 | 200 OK | Configuration to report on the service experience information is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

###### 7.1.3.7.4.3	Operation: PULL Service Experience Information

This operation is used by the ADAES to pull the service experience information report from the ADAEC and shall support the URI query parameters specified in table 7.1.3.7.4.3-1.

Table 7.1.3.7.4.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.7.4.3-2 and the response data structures and response codes specified in table 7.1.3.7.4.3-3.

Table 7.1.3.7.4.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PullSrvExpInfo | M | 1 | Request for the report on the service experience information |

Table 7.1.3.7.4.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvExpInfoRep | M | 1 | 200 OK | Successfully obtaining the report on the service experience information |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

#### 7.1.3.8	Resource: Individual trigger configuration for service experience information reporting

##### 7.1.3.8.1	Description

The individual trigger configuration for service experience information reporting resource represents an individual event trigger configuration of the ADAES.

##### 7.1.3.8.2	Resource Definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/service-experience/{srvTrigId}

This resource shall support the resource URI variables defined in the table 7.1.3.8.2-1.

Table 7.1.3.6.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |
| srvTrigId | string | Identifies a trigger configuration for service experience information reporting |

##### 7.1.3.8.3	Resource Standard Methods

##### 7.1.3.8.3.1	DELETE

This method shall support the URI query parameters specified in table 7.1.3.8.3.1-1.

Table 7.1.3.8.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.8.3.1-2 and the response data structures and response codes specified in table 7.1.3.8.3.1-3.

Table 7.1.3.8.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.8.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual trigger-configured service experience information reporting matching the srvTrigId is deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 7.1.3.8.4	Resource Custom Operations

None.

### 7.1.4	Notifications

#### 7.1.4.1	General

Table 7.1.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method | Description (service operation) |
| --- | --- | --- | --- |
| Application performance event notification | {notificationUri} | POST | Notification for the VAL performance analytics event |
| Edge load event notification | {notificationUri} | POST | Notification for the edge load data collection event |
| Service experience report event notification | {notificationUri} | POST | Notification for the service experience report event |

#### 7.1.4.2	Application performance event notification

##### 7.1.4.2.1	Description

Application performance event notification is by the ADAEC to notify the ADAES, the VAL performance analytics.

##### 7.1.4.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.1.4.2.2-1.

Table 7.1.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the VAL performance analytics, this method shall support the request data structures specified in table 7.1.4.2.2-2 and the response data structures and response codes specified in table 7.1.4.2.2-3.

Table 7.1.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfNotif | M | 1 | Notification information of the VAL performance analytics. |

Table 7.1.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the VAL performance analytics event is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

#### 7.1.4.3	Edge load event notification

##### 7.1.4.3.1	Description

The edge load event notification is used by the ADAEC to notify the ADAES, the edge load data collection.

##### 7.1.4.3.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.1.4.3.2-1.

Table 7.1.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the edge load data collection, this method shall support the request data structures specified in table 7.1.4.3.2-2 and the response data structures and response codes specified in table 7.1.4.3.2-3.

Table 7.1.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeNotif | M | 1 | Notification information of edge load data collection event |

Table 7.1.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the edge load data collection event is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

#### 7.1.4.4	Service experience information report event notification

##### 7.1.4.4.1	Description

The service experience information report event notification is used by the ADAEC to notify the ADAES, the service experience information.

##### 7.1.4.4.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.1.4.4.2-1.

Table 7.1.4.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the service experience information, this method shall support the request data structures specified in table 7.1.4.4.2-2 and the response data structures and response codes specified in table 7.1.4.4.2-3.

Table 7.1.4.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SrvExpInfoRep | M | 1 | Notification of service experience information report |

Table 7.1.4.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 (No Content) | Notification of the service experience information report is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

### 7.1.5	Data model

#### 7.1.5.1	General

This clause specifies the application data model supported by the API.

Table 7.1.5.1-1 specifies the data types defined for the ADAE_ServiceConfiguration API.

Table 7.1.5.1-1: ADAE_ServiceConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| Ue2UePerfReq | 7.1.5.2.2 | Request for the UE-to-UE session performance analytics |  |
| Ue2UePerfResp | 7.1.5.2.3 | Response for the UE-to-UE session performance analytics |  |
| ConfigRepTrigger | 7.1.5.2.4 | Configure triggers for reports on the service experience information |  |
| PullSrvExpInfo | 7.1.5.2.5 | Pull an individual service experience information |  |
| SrvExpInfoRep | 7.1.5.2.6 | Response to pull an individual service experience information |  |

Table 7.1.5.1-2 specifies data types re-used by the ADAE_ServiceConfiguration API service.

Table 7.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AppPerfSub | 3GPP TS 29.549 [7] | Subscription to the VAL application performance analytics |  |
| AppPerfNotif | 3GPP TS 29.549 [7] | Notification information of the application performance analytics. |  |
| DurationSec | 3GPP TS 29.571 [8] | Represent the time interval between successive location reports. |  |
| EdgeSubs | 3GPP TS 29.549 [7] | Subscription to the edge load analytics event |  |
| EdgeNotif | 3GPP TS 29.549 [7] | Notification information of the edge load analytics event. |  |
| LocationArea | 3GPP TS 29.122 [4] | Represents location information. |  |
| Pc5QoSPara | 3GPP TS 29.571 [8] | Represents policy data on the PC5 QoS parameters. |  |
| ReportingInformation | 3GPP TS 29.523 [6] | Indicates the reporting requirement. |  |
| ValTargetUe | 3GPP TS 29.549 [7] | Used to indicate either VAL User ID or VAL UE ID. |  |
| Uri | 3GPP TS 29.571 [8] | Used to indicate the notification URI. |  |

#### 7.1.5.2	Structured data types

##### 7.1.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 7.1.5.2.2	Type: Ue2UePerfReq

Table 7.1.5.2.2-1: Definition of type Ue2UePerfReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | string | M | 1 | Identity of the ADAES |  |
| analyticsId | string | M | 1 | Identity of the UE-to-UE session analytics |  |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which the UE-to-UE session performance analytics, is requested. |  |
| pc5Qos | Pc5QoSPara | M | 1 | The QoS attributes to be analyzed at the ADAEC during UE-to-UE session. |  |
| reportConfig | ReportingInformation | O | 0..1 | The configuration of UE-to-UE session performance analytics reporting. |  |
| area | LocationArea | O | 0..1 | The geographical or service area, for which the UE-to-UE session peformance analytics is requested. |  |
| timeWindow | DurationSec | O | 0..1 | The time window as the start time point and the end time point, for which the UE-to-UE session performance analytics is applied. |  |

##### 7.1.5.2.3	Type: Ue2UePerfResp

Table 7.1.5.2.-1: Definition of type Ue2UePerfResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataOutputs | array(string) | M | 1..N | UE-to-UE session performance analytics for prediction or statistics depending on the type and on the requested QoS parameter based on the analytics event. |  |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which the UE-to-UE session performance analytics applies. |  |
| analyticsId | string | M | 1 | Identity of the UE-to-UE session analytics |  |

##### 7.1.5.2.4	Type: ConfigRepTrigger

Table 7.1.5.2.4-1: Definition of type ConfRepTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServSpecCrit | ValServSpecCrit | M | 1 | Identity of the VAL server, requesting service experience information report. |  |
| commonTriggCrit | SrvExpRepCrit | O | 0..1 | Information criteria about the triggers (applicable to all VAL servers) on which the service experience information is fetched. |  |
| srvExpMeas | DurationSec | O | 0..1 | Information about the service experience information measurements which needs to be fetched and included in the report. If not present, by default end-to-end response time is measured. |  |
| notifyTarget | string | O | 0..1 | The target address which is notified. |  |

##### 7.1.5.2.5	Type: ValServSpecCrit

Table 7.1.5.2.5-1: Definition of type ValServSpecCrit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerIds | array(string) | M | 1..N | Identities of one or more VAL servers, for which the configuration of the service experience information report applies. |  |
| triggCrit | SrvExpRepCrit | M | 1 | Information criteria about the triggers on which the service experience is information to be reported for the VAL server. |  |

##### 7.1.5.2.6	Type: PullSrvExpInfo

Table 7.1.5.2.6-1: Definition of type PullSrvExpInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Identity of the VAL server, for which the service experience information report is requested. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service |  |

##### 7.1.5.2.7	Type: SrvExpInfoRep

Table 7.1.5.2.7-1: Definition of type SrvExpInfoRep

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeId | ValTargetUe | M | 1 | Identity of VAL UE |  |
| valServerId | string | M | 1 | Identity of the VAL server, for which the service experience information report is requested. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service |  |
| timeStamp | DurationSec | O | 0..1 | Timestamp as start time and end time of the collected report |  |
| valSrvExpRep | ReportingInformation | O | 0..1 | Report on the VAL service experience information |  |

#### 7.1.5.3	Simple data types and enumerations

##### 7.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 7.1.5.3.2	Simple data types

None.

##### 7.1.5.3.3	Enumeration: SrvExpRepCrit

Table 7.1.5.3.3-1: Enumeration SrvExpRepCrit

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRIGGER_CRITERIA | Information criteria that can trigger service experience information reporting to a VAL server. |  |
| COMMON_TRIGGER_CRITERIA | Information criteria that can trigger service experience information reporting to all VAL servers. |  |

### 7.1.6	Error Handling

#### 7.1.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [x].

In addition, the requirements in the following clauses shall apply.

#### 7.1.6.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the ADAE_ServiceConfiguration API.

#### 7.1.6.3	Application Errors

The application errors defined for ADAE_ServiceConfiguration API are listed in table 7.1.6.3-1.

Table 7.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 7.1.7	Feature Negotiation

General feature negotiation procedures are defined in clause 5.2.7 of 3GPP TS 29.122 [4]. Table 7.1.7-1 lists the supported features for ADAE_ServiceConfiguration API.

Table 7.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 8	Usage of common API framework

## 8.1	General

Usage of common API framework shall be supported by the ADAE service configuration API as described in clause 8 in 3GPP TS 29.549 [7].

# 9	Security

## 9.1	General

Usage of HTTP over TLS and the TLS profiles shall be as specified in clause 5.1.1.4 of 3GPP TS 33.434 [9].

Annex <A> (normative):
OpenAPI specification

# A.1	General

This annex is based on the OpenAPI Specification [20] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [1] and clause 5.3.1 of the 3GPP TS 29.501 [5] for further information).

# A.2	ADAE_ServiceConfiguration API

openapi: 3.0.0

info:

title: ADAE_ServiceConfiguration

version: 1.0.0-alpha.1

description: |

API for ADAE service configuration.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.559 V18.0.0 Appliction Data Analytics Enablement Service; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.559/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/adae-sc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/application-performance:

post:

description: >

Creates a new individual VAL performance analytics event subscription.

operationId: VALPerformanceSubscription

tags:

- VAL performance event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_VALPerformanceAnalytics.yaml#/components/schemas/AppPerfSubs'

callbacks:

notificationUri:

'{request.body#/notificationUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_VALPerformanceAnalytics.yaml#/components/schemas/AppPerfNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: VAL performance event subscription resource created successfully.

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_VALPerformanceAnalytics.yaml#/components/schemas/AppPerfSubs'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/application-performance/{appPerfId}:

delete:

description: Deletes an individual VAL performance event subscription.

operationId: DeleteIndValPerfEventSubsc

tags:

- Individual VAL performance event subscription

parameters:

- name: appPerfId

in: path

description: Identifier of an individual VAL performance event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual VAL performance subscription matching the appPerfId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/ue2ue-session-performance/fetch:

post:

description: >

Obtain the UE-to-UE session performance analytics.

operationId: FetchUe2UeSessionPerformance

tags:

- Fetch UE-to-UE session performance analytics

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Ue2UePerfReq'

responses:

'200':

description: >

Successful case. The UE-to-UE session performance information is returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/Ue2UePerfResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/edge-load:

post:

description: >

Creates a new individual edge load data collection event subscription.

operationId: EdgeLoadDataCollectionSubscription

tags:

- Edge load data collection event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_EdgeLoadAnalytics.yaml#/components/schemas/EdgeSubs'

callbacks:

notificationUri:

'{request.body#/notificationUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_EdgeLoadAnalytics.yaml#/components/schemas/EdgeNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Edge load data collection event subscription resource created successfully.

content:

application/json:

schema:

$ref: 'TS29549_SS_AEDA_EdgeLoadAnalytics.yaml#/components/schemas/EdgeSubs'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/edge-load/{edgeLdId}:

delete:

description: Deletes an individual edge load data collection event subscription.

operationId: DeleteIndEdgeLdDataCollectEventSubsc

tags:

- Individual edge load data collection event subscription

parameters:

- name: edgeLdId

in: path

description: Identifier of an individual edge load data collection event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual edge load data collection subscription matching the edgeLdId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/service-experience:

post:

description: >

Configures the ADAEC triggers for service experience reporting.

operationId: ConfigTriggerServExpReporting

tags:

- Configuration for service experience reporting (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Config	RepTrigger'

callbacks:

notificationUri:

'{request.body#/notificationUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SrvExpInfoRep'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Triggers for service experience reporting configured successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ConfigRepTrigger'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/service-experience/{srvTrigId}:

delete:

description: Abolish configured triggers for service experience reporting.

operationId: DeleteTriggerSrvExpReporting

tags:

- Individual configured trigger for service experience reporting

parameters:

- name: srvTrigId

in: path

description: Identifier of an individual triger-configured service experience reporting.

required: true

schema:

type: string

responses:

'204':

description: >

The individual trigger-configured service experiment reporting matching

the srvTrigId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/service-experience/push:

post:

description: >

Configure ADAE client to push service experience report to the ADAE server.

operationId: PushSrvExpReport

tags:

- Push service experienec report

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

responses:

'200':

description: >

Successful case. The ADAE client pushes service experience reporting to the ADAE server.

content:

application/json:

schema:

$ref: '#/components/schemas/SrvExpInfoRep'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/service-experience/pull:

post:

description: >

ADAE server pulls service experience report from the ADAE client.

operationId: PullSrvExpReport

tags:

- Pull service experienec report

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PullSrvExpInfo'

responses:

'200':

description: >

Successful case. The ADAE client provides service experience reporting to

the ADAE server.

content:

application/json:

schema:

$ref: '#/components/schemas/SrvExpInfoRep'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

Ue2UePerfReq:

description: ADAES requests ADAEC for the UE-to-UE session performance analytics.

type: object

properties:

serverId:

type: string

description: String identifying the ADAE server

analyticsId:

type: string

description: String identifying the UE-to-UE session analytics

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

One or more VAL UE IDs whose UE-to-UE session performance is requested.

pc5Qos:

$ref: 'TS29571_CommonData.yaml'#/components/schemas/Pc5QoSPara'

description: QoS for the UE-to-UE session performance analytics.

reportConfig:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

description: The configuration of UE-to-UE session performance analytics reporting.

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea'

description: Area for the UE-to-UE session performance analytics.

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

description: Start and end of period for the UE-to-UE session performance analytics.

required:

- serverId

- analyticsId

- valUeIds

- pc5Qos

Ue2UePerfResp:

description: >

ADAEC responds to ADAES with the UE-to-UE session performance analytics information.

type: object

properties:

dataOutputs:

type: array

items:

type: string

minItems: 1

description: >

UE-to-UE session performance analytics for prediction or statistics.

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

One or more VAL UE IDs whose UE-to-UE session performance has been requested.

analyticsId:

type: string

description: String identifying the UE-to-UE session analytics

required:

- dataOutputs

- valUeIds

- analyticsId

ConfigRepTrigger:

description: Configures the ADAEC triggers for service experience reporting.

type: object

properties:

valServSpecCrit:

$ref: '#/components/schemas/ValServSpecCrit          description: String identifying the ADAE server

commonTriggCrit:

$ref: '#/components/schemas/SrvExpRepCrit

srvExpMeas:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

notifyTarget:

type: string

description: the target address which is notified.

required:

- valServSpecCrit

ValServSpecCrit

type: object

properties:

valServerIds:

type: array

description: VAL servers for which configuration of service experience report applies.

items:

type: string

minItems: 1

triggCrit:

$ref: '#/components/schemas/SrvExpRepCrit

required:

- valServerIds

- triggCrit

PullSrvExpInfo

type: object

properties:

valServerId:

type: string

valServiceId:

type: string

required:

- valServerId

SrvExpInfoRep:

description: Allows ADAEC to provide the service experience report to the ADAES.

type: object

properties:

valUeId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valServerId:

type: string

description: String identifying the VAL server the service experience report applies.

valServiceId:

type: string

description: String identifying the VAL service

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

valSrvExpRep:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

required:

- valUeId

- valServerId

# Simple data types and Enumerations

SrvExpRepCrit:

anyOf:

- type: string

enum:

- TRIGGER_CRITERIA

- COMMON_TRIGGER_CRITERIA

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents information criteria to trigger service experience reporting.

Possible values are:

- TRIGGER_CRITERIA: Information criteria that can trigger service experience

reproting to a VAL server.

- COMMON_TRIGGER_CRITERIA: Information criteria that can trigger service experience

reproting to all VAL servers.

Annex <C> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | CT1#140 | C1-230160 |  |  |  | Skeleton for Application Data Analytics Enablement Service | 0.0.0 |
| 2023-05 | CT1#142 | C1-233310 |  |  |  | Application data analytics enablement service abbreviations and refences | 0.1.0 |
| 2023-05 | CT1#142 | C1-234031 |  |  |  | Application data analytics enablement service functional entities | 0.1.0 |
| 2023-05 | CT1#142 | C1-234032 |  |  |  | Application data analytics enablement service procedures | 0.1.0 |
| 2023-05 | CT1#142 | C1-234033 |  |  |  | Application data analytics enablement service procedures | 0.1.0 |
| 2023-10 | CT1#144 | C1-237009 |  |  |  | Correction of general description | 0.2.0 |
| 2023-10 | CT1#144 | C1-237962 |  |  |  | ADAES configuration API | 0.2.0 |
| 2023-10 | CT1#144 | C1-237963 |  |  |  | Application performance analytics configuration API | 0.2.0 |
| 2023-10 | CT1#144 | C1-237964 |  |  |  | Procedure for application performance analytics | 0.2.0 |
| 2023-11 | CT1#145 | C1-239589 |  |  |  | Resource review for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239666 |  |  |  | Application performance event subscription for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239672 |  |  |  | UE-to-UE session performance for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239673 |  |  |  | Edge load event subscription for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239588 |  |  |  | Service experience for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239674 |  |  |  | Application performance event notification for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239675 |  |  |  | Edge load event notification for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239586 |  |  |  | Data model for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239587 |  |  |  | Error handling for ADAE services | 0.3.1 |
| 2023-11 | CT1#145 | C1-239477 |  |  |  | Notification overview for ADAE services | 0.3.1 |
| 2024-01 | CT1#146 | C1-240300 |  |  |  | Add references | 0.4.0 |
| 2024-01 | CT1#146 | C1-240301 |  |  |  | Corrections and removal of some titles | 0.4.0 |
| 2024-01 | CT1#146 | C1-240302 |  |  |  | Restructuring of resource URI for ADAES | 0.4.0 |
| 2024-01 | CT1#146 | C1-240303 |  |  |  | Service description and operations for application performance analytics | 0.4.0 |
| 2024-01 | CT1#146 | C1-240304 |  |  |  | Service description and operations for UE-to-UE session performance analytics | 0.4.0 |
| 2024-01 | CT1#146 | C1-240305 |  |  |  | Service description and operations for edge load data collection | 0.4.0 |
| 2024-01 | CT1#146 | C1-240306 |  |  |  | Service description and operations for service-experiment | 0.4.0 |
| 2024-01 | CT1#146 | C1-240307 |  |  |  | Usage of HTTP, common API framework, and security | 0.4.0 |
| 2024-01 | CT1#146 | C1-240308 |  |  |  | ADAE service configuration OpenAPI | 0.4.0 |