3GPP TR 24.890 V0.2.0 (2017-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System – Phase 1;

CT WG1 Aspects

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	10

2	References	10

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Architecture	13

4.1	General	13

4.2	Architecture reference model	13

4.3	Architecture for interworking with E-UTRAN connected to EPC	14

5	Network selection procedures	14

5.1	Concepts	14

5.2	Procedures	15

5.2.1	General	15

5.2.2	Procedures for 3GPP radio access networks	15

5.2.3	Procedures for non-3GPP access networks	15

6	Stage 3 protocol, general aspects	16

6.1	General	16

6.2	Protocols and peer to peer communication	16

6.3	Basic groups of functions	16

6.4	Protocol architecture	16

6.5	Standard L3 messages	16

7	Coordination between the protocols for 5GS mobility management and 5GS session management	16

8	5GS mobility management	16

8.1	Overview	16

8.1.1	General	16

8.1.2	Types of 5GMM procedures	17

8.1.3	5GMM sublayer states	18

8.1.3.1	General	18

8.1.3.2	5GMM sublayer states over 3GPP access	18

8.1.3.2.1	General	18

8.1.3.2.2	5GMM sublayer states in the UE	18

8.1.3.2.2.1	General	18

8.1.3.2.2.2	Main states	18

8.1.3.2.2.2.1	5GMM-NULL	18

8.1.3.2.2.2.2	5GMM-DEREGISTERED	19

8.1.3.2.2.2.3	5GMM-REGISTERED-INITIATED	19

8.1.3.2.2.2.4	5GMM-REGISTERED	19

8.1.3.2.2.2.5	5GMM-DEREGISTERED-INITIATED	19

8.1.3.2.3	5GMM sublayer states in the network	19

8.1.3.2.3.1	General	19

8.1.3.2.3.2	5GMM-DEREGISTERED	19

8.1.3.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED	20

8.1.3.2.3.4	5GMM-REGISTERED	20

8.1.3.2.3.5	5GMM-DEREGISTERED-INITIATED	20

8.1.4	Mobile initiated connection only mode	20

8.2	5GS mobility functions in IDLE and CONNECTED mode	20

8.2.1	Registration areas in the 5GS	20

8.2.1.1	General	20

8.2.1.2	Open issues on registration areas in the 5GS	21

8.2.2	UE NAS mobility functions in 5GMM-IDLE mode	21

8.2.3	UE NAS mobility functions in 5GMM-CONNECTED mode	22

8.3	IP address allocation	22

8.3.1	General	22

8.3.2	IP address allocation via NAS signalling	23

8.4	5GS mobility management procedures	23

8.4.1	5GMM common procedures	23

8.4.1.1	NAS transport procedures	23

8.4.1.1.1	General	23

8.4.1.1.2	Alternative 1 for NAS transport	23

8.4.1.1.1.1	SM transport procedures	23

8.4.1.1.1.1.1	UE-initiated SM message transport procedure	23

8.4.1.1.1.1.1.1	General	23

8.4.1.1.1.1.1.2	UE-initiated SM message transport initiation	24

8.4.1.1.1.1.1.3	UE-initiated SM message transport accepted by the network	24

8.4.1.1.1.1.1.4	Abnormal cases on the network side	25

8.4.1.1.1.1.3	Network-initiated SM message transport procedure	25

8.4.1.1.1.1.3.1	General	25

8.4.1.1.1.1.3.2	Network-initiated SM message transport initiation	25

8.4.1.1.1.1.3.3	Network-initiated SM message transport accepted by the UE	26

8.4.1.1.1.2	Non-SM message transport procedures	26

8.4.1.1.1.2.1	General	26

8.4.1.1.1.2.2	UE-initiated non-SM message transport procedure	26

8.4.1.1.1.2.2.1	General	26

8.4.1.1.1.2.2.2	UL GENERIC TRANSPORT message sending	26

8.4.1.1.1.2.2.3	UL GENERIC TRANSPORT message receiving	27

8.4.1.1.1.2.3	Network-initiated non-SM message transport procedure	27

8.4.1.1.1.2.3.1	General	27

8.4.1.1.1.2.3.2	DL GENERIC TRANSPORT message sending	27

8.4.1.1.1.2.3.3	DL GENERIC TRANSPORT message receiving	28

8.4.1.1.3	Alternative 2 for NAS Transport	28

8.4.1.1.3.1	General	28

8.4.1.1.3.2	UE-initiated NAS transport procedure	28

8.4.1.1.3.2.1	General	28

8.4.1.1.3.2.2	NAS transport procedure initiation	28

8.4.1.1.3.2.3	NAS transport accepted by the network	29

8.4.1.1.3.3	Network-initiated NAS transport procedure	29

8.4.1.1.3.3.1	General	29

8.3.1.1.3.3.2	NAS transport procedure initiation	29

8.4.1.1.3.3.3	NAS transport procedure accepted by UE	30

8.4.1.2	Generic UE configuration update procedure	30

8.4.1.2.1	General	30

8.4.1.2.2	UE Configuration update procedure initiated by the network	30

8.4.1.2.3	UE Configuration update accepted by the UE	31

8.4.1.2.4	UE Configuration update completion by the network	31

8.4.1.2.5	Abnormal cases in the UE	31

8.4.1.2.6	Abnormal cases on the network side	31

8.4.2	5GMM specific procedures	31

8.4.2.1	Registration procedure	31

8.4.2.1.1	General	31

8.4.2.1.2	Registration procedure for initial registration	31

8.4.2.1.2.1	General	31

8.4.2.1.2.2	Initial registration initiation	31

8.4.2.1.2.3	5GMM common procedure initiation	32

8.4.2.1.2.4	Initial registration accepted by the network	32

8.4.2.1.2.5	Initial registration not accepted by the network	32

8.4.2.1.2.6	Initial registration for emergency services not accepted by the network	32

8.4.2.1.2.7	Abnormal cases in the UE	32

8.4.2.1.2.8	Abnormal cases on the network side	33

8.4.2.1.3	Registration procedure for mobility and periodic registration update	33

8.4.2.1.3.1	General	33

8.4.2.1.3.2	Mobility and periodic registration update initiation	33

8.4.2.1.3.3	5GMM common procedure initiation	33

8.4.2.1.3.4	Mobility and periodic registration update accepted by the network	34

8.4.2.1.3.5	Mobility and periodic registration update not accepted by the network	34

8.4.2.1.3.6	Abnormal cases in the UE	34

8.4.2.1.3.7	Abnormal cases on the network side	34

8.4.3	5GMM connection management procedures	34

8.4.3.1	Service request procedure	34

8.4.3.1.1	General	34

8.4.3.1.2	Service request procedure initiation	35

8.4.3.1.3	Common procedure initiation	35

8.4.3.1.4	Service request procedure accepted by the network	35

8.4.3.1.5	Service request procedure not accepted by the network	36

8.4.3.1.6	Abnormal cases in the UE	36

8.4.3.1.7	Abnormal cases on the network side	36

8.4.3.2	Paging procedure	36

8.4.3.2.1	General	36

8.4.3.2.2	Paging for 5GS services	36

8.4.3.2.2.1	General	36

8.4.3.2.2.2	Abnormal cases on the network side	36

8.5	5GS mobility management message coding	37

8.5.1	Registration request	37

8.5.1.1	Message definition	37

8.5.2	Registration accept	37

8.5.2.1	Message definition	37

8.5.3	Registration complete	37

8.5.3.1	Message definition	37

8.5.4	Registration reject	37

8.5.4.1	Message definition	37

8.5.5	NAS SM message transport	37

8.5.5.1	Message definition	37

8.5.6	38

8.5.6.1	Message definition	38

8.5.7	UL NAS TRANSPORT for Alternative 2	38

8.5.7.1	Message definition	38

8.5.8	DL GENERIC TRANSPORT for Alternative 1	38

8.5.8.1	Message definition	38

8.5.9	DL NAS TRANSPORT for Alternative 2	38

8.5.9.1	Message definition	38

8.6	Timers of 5GS mobility management	38

9	5GS session management	39

9.1	General	39

9.2	Overview	39

9.3	5GSM sublayer states	40

9.3.1	General	40

9.3.2	5GSM sublayer states in the UE	40

9.3.2.1	Overview	40

9.3.2.2	PDU SESSION INACTIVE	41

9.3.2.3	PDU SESSION ACTIVE PENDING	41

9.3.2.4	PDU SESSION ACTIVE	41

9.3.2.5	PDU SESSION INACTIVE PENDING	41

9.3.2.6	PDU SESSION MODIFICATION PENDING	41

9.3.2.7	PDU SESSION ACTIVE PENDING	41

9.3.2.8	PDU SESSION ACTIVE	41

9.3.2.9	PDU SESSION INACTIVE PENDING	41

9.3.2.10	PDU SESSION MODIFICATION PENDING	41

9.3.3	5GSM sublayer states in the network	41

9.3.3.1	Overview	41

9.3.3.2	PDU SESSION INACTIVE	42

9.3.3.3	PDU SESSION ACTIVE	42

9.3.3.4	PDU SESSION INACTIVE PENDING	42

9.3.3.5	PDU SESSION MODIFICATION PENDING	42

9.4	5GS session management procedures	42

9.4.1	UE-requested PDU session establishment procedure	42

9.4.1.1	General	43

9.4.1.2	UE-requested PDU session establishment procedure initiation	43

9.4.1.3	UE-requested PDU session establishment procedure accepted	44

9.4.1.4	UE requested PDU session establishment procedure rejected by SMF	45

9.4.1.5	Abnormal cases in the UE	45

9.4.1.6	Abnormal cases on the network side	45

9.4.2	PDU session establishment authentication and authorization procedure	45

9.4.2.1	General	45

9.4.2.2	PDU session establishment authentication and authorization procedure initiation	45

9.4.2.3	PDU session establishment authentication and authorization procedure accepting	46

9.4.2.4	PDU session establishment authentication and authorization procedure rejecting	46

9.4.2.5	Abnormal cases on the network side	47

9.4.2.6	Abnormal cases in the UE	47

9.4.3	UE-requested PDU session modification procedure	47

9.4.3.1	General	47

9.4.3.2	UE-requested PDU session modification procedure initiation	47

9.4.3.3	UE-requested PDU session modification procedure accepting	47

9.4.3.4	UE-requested PDU session modification procedure rejecting	48

9.4.3.5	Abnormal cases in the UE	48

9.4.3.6	Abnormal cases on the network side	48

9.4.4	Network-requested PDU session modification procedure	48

9.4.4.1	General	48

9.4.4.2	Network-requested PDU session modification procedure initiation	48

9.4.4.3	Network-requested PDU session modification procedure accepting	49

9.4.4.4	Abnormal cases on the network side	49

9.4.4.5	Abnormal cases in the UE	49

9.4.5	UE-requested PDU session release procedure	49

9.4.5.1	General	49

9.4.5.2	UE-requested PDU session release procedure initiation	50

9.4.5.3	UE-requested PDU session release procedure accepting	50

9.4.5.4	UE-requested PDU session release procedure rejecting	50

9.4.5.5	Abnormal cases in the UE	50

9.4.5.6	Abnormal cases on the network side	51

9.4.6	Network-requested PDU session release procedure	51

9.4.6.1	General	51

9.4.6.2	Network-requested PDU session release procedure initiation	51

9.4.6.3	Network-requested PDU session release procedure accepting	52

9.4.6.4	N1 SM delivery skipped	52

9.4.6.5	Abnormal cases on the network side	52

9.4.6.6	Abnormal cases in the UE	52

9.5	5GS session management coding	52

9.5.1	PDU session establishment request	52

9.5.1.1	Message definition	52

9.5.2	PDU session establishment accept	52

9.5.2.1	Message definition	52

9.5.3	 PDU session establishment reject	53

9.5.3.1	Message definition	53

9.5.4	PDU session authentication request	53

9.5.4.1	Message definition	53

9.5.5	PDU session authentication accept	53

9.5.5.1	Message definition	53

9.5.6	PDU session authentication reject	54

9.5.6.1	Message definition	54

9.5.7	PDU session modification request	54

9.5.7.1	Message definition	54

9.5.8	PDU session modification reject	54

9.5.8.1	Message definition	54

9.5.9	PDU session modification command	54

9.5.9.1	Message definition	54

9.5.10	PDU session modification accept	55

9.5.10.1	Message definition	55

9.5.11	PDU session release request	55

9.5.11.1	Message definition	55

9.5.12	PDU session release reject	55

9.5.12.1	Message definition	55

9.5.13	PDU session release command	55

9.5.13.1	Message definition	55

9.5.14	PDU session release accept	56

9.5.14.1	Message definition	56

9.6	Timers of 5GS session management	56

10	General message format and information elements coding	57

10.1	Overview	57

10.2	Extended protocol discriminator	58

10.3	Security header type	58

10.4	Spare half octet	58

11	Access to the 5G core network via non-3GPP access networks	58

11.1	General	58

11.2	Protocol stack	59

11.3	NAS over non-3GPP access	59

11.3.1	General	59

11.3.2	Mobility management over non-3GPP access	59

11.3.3	Session management over non-3GPP access	59

11.4	Registration over non-3GPP access	59

11.4.1	General	59

11.4.2	Registration over non-3GPP access	60

11.4.3	Registration over non-3GPP access and 3GPP access	60

11.4.4	De-registration over non-3GPP access	60

11.5	Selection of N3IWF	60

11.6	Procedures between UE and N3IWF	60

11.7.1	IKE SA creation procedure for UE registration via untrusted non-3GPP access	61

11.7.2	Child SA creation procedure for UE-requested PDU session establishment	61

11.7.3	Child SA change procedure for PDU session modification	61

11.7.4	Child SA deletion procedure for PDU session release	61

11.7.5	IKE SA deletion for UE deregistration via untrusted non-3GPP access	61

11.8	IKEv2 payload extensions	61

12	Interworking with E-UTRAN connected to EPC	61

12.1	General	61

12.2	Single-registration mode	61

12.2.1	General	61

12.2.2	Coordination between 5GMM and EMM	61

12.3	Dual-registration mode	62

13	5G System core network impact on services, network functions and capabilities	62

13.1	Specific services	62

13.1.1	Public warning system	62

13.1.2	SMS over NAS	62

13.1.3	Emergency services	62

13.1.3.1	Indication of support of emergency eervices	62

13.1.3.2	Handling of emergency service for the UE in limited service state	62

13.1.3.3	Emergency PDU session setup	62

13.1.3.4	Handling of local emergency numbers	63

13.1.3.5	Security procedures for handling emergency services	63

13.1.4	Location services	63

13.1.5	Multimedia priority services	63

13.2	Access control	63

13.3	Security	63

13.4	Quality of service (QoS)	63

13.4.1	General	63

13.4.1.1	QoS rules	63

13.4.1.2	Session-AMBR	64

13.4.1.3	QoS flow template	64

13.4.1.4	Packet matching	64

11.4.1.5	Reflective QoS	65

11.4.1.5.1	Reflective QoS activation via the control plane	65

11.4.1.5.2	Reflective QoS activation via the user plane	65

13.4.1.5.3	QoS flow templated derived by the UE for reflective QoS	65

13.4.1.5.4	Reflective QoS deactivation	66

13.4.2	QoS coding	66

13.4.2.1	QoS rules coding	66

13.4.2.2	Session-AMBR coding	66

13.5	IM CN subsystem	66

13.5.1	IMS voice over PS Session supported Indication	66

13.5.2	P-CSCF address delivery	66

13.6	Network sharing	67

13.7	Congestion and overload control	67

13.8	Charging	67

14	Network slicing	67

14.1	General	67

14.2	Mobility management aspects	67

14.2.1	General	67

14.2.2	NSSAI storage	68

14.3	Session management aspects	68

15	Conclusions and recommendations	68

Annex A: Impacts to specifications	68

Annex B: Change history	69

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document discusses and describes the CT1 aspects of the 5G System phase 1. In particular, these CT1 aspects include the non-access-stratum (NAS) functions to be performed by the user equipment (UE) in idle mode and the NAS signalling procedures between the UE and the 5G System (5GS) phase 1.

The present document also considers the requirements on the NAS between the UE and the core network which arise from specific services such as public warning system (PWS), SMS over NAS, emergency services, location services (LCS), multimedia priority services (MPS), and also network functions such as access control, security, QoS, IMS, network sharing, charging, congestion and overload control. In addition, the document considers the case of network slicing as well as interworking with E-UTRAN connected to EPC.

The present document is intended as a holding place for CT1 5GS material until it stabilises sufficiently to be moved to appropriate 3GPP technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[8]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[9]	3GPP TS 38.304: "New Generation Radio Access Network; User Equipment (UE) procedures in idle mode".

[10]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[11]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[12]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[13]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

N1 mode: A mode of a UE allowing access to the 5G core network via the 5G access network.

N1 NAS signalling connection: A peer to peer N1 mode connection between UE and AMF. An N1 NAS signalling connection is either the concatenation of an NR RRC connection via the Uu reference point and an NG connection via the N2 reference point for 3GPP access, or the concatenation of an IPsec tunnel via the NWu reference point and an NG connection via the N2 reference point for non-3GPP access.

5GMM-IDLE mode: A UE is in 5GMM-IDLE mode when no N1 NAS signalling connection between UE and network exists. The term 5GMM-IDLE mode used in the present document corresponds to the term CM-IDLE state used in 3GPP TS 23.501 [5].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [5] apply:

5G access network

5G core network

5G QoS flow

5G QoS indicator

5G System

5G-RAN

Allowed NSSAI

Configured NSSAI

Network slice

PDU session

Requested NSSAI

For the purposes of the present document, the following terms and definitions given in 3GPP TS 38.413 [10] apply:

NG connection

Editor's note:	The term (NG connection) can be modified according to 3GPP TS 38.413 [10].

Editor's note:	Definition on the term NR RRC connection (or the equivalent) will be updated if defined by RAN2.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GCN	5G Core Network

5GS	5G System

5GMM	5GS Mobility Management

5GSM	5GS Session Management

5GQI	5G QoS Indicator

DL	Downlink

MICO	Mobile Initiated Connection Only

N3IWF	Non-3GPP InterWorking Function

NSSAI	Network Slice Selection Assistance Information

NSSP	Network Slice Selection Policy

QFI	QoS Flow Identity

QoS	Quality of Service

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EPD	Extended Protocol Discriminator

EMM	EPS Mobility Management

EPC	Evolved Packet Core Network

EPS	Evolved Packet System

GFBR	Guaranteed Flow Bit Rate

HRPD	High Rate Packet Data

MFBR	Maximum Flow Bit Rate

RQI	Reflective QoS Indication

S-NSSAI	Single NSSAI

SA	Security Association

UL	Uplink

TA	Tracking Area

TAC	Tracking Area Code

TAI	Tracking Area Identity

# 4	Architecture

## 4.1	General

The 5GS architecture is specified in 3GPP TS 23.501 [5] and the stage 2 procedures and flows for 5GS are specified in 3GPP TS 23.502 [6].

## 4.2	Architecture reference model

The 5GS architecture reference model can be represented in two ways; a service-based representation and a reference point representation (see figures 4.2.1, 4.2.2). The service-based representation also includes point-to-point reference points where necessary. The 5GS architecture reference model covers both roaming and non-roaming scenarios in all aspects, including interworking between 5GS and EPS, mobility within 5GS, policy control and charging, and authentication.

Figure 4.2.1: Non-roaming 5GS architecture in reference point representation

Figure 4.2.2: Non-roaming architecture for 5GCN with untrusted non-3GPP access

A UE, which supports 5GS connectivity, can access the 5GCN via a 5G-RAN or a standalone non-3GPP access or both. The NAS signalling with the 5GCN control-plane functions is over the N1 reference point. The N11 reference point is used to connect the core access and mobility management function (AMF) and the session management function (SMF). The SMF handles the session management part of the NAS signalling exchanged with the UE. The NWu reference point is used for establishing secure tunnel(s) between the UE and the N3IWF so that control-plane and user-plane exchanged between the UE and the 5GCN is transferred securely over untrusted non-3GPP access. CT1 will specify the N1 and NWu interfaces.

## 4.3	Architecture for interworking with E-UTRAN connected to EPC

Interworking between the 5GS and the EPC through E-UTRAN is supported and specified in 3GPP TS 23.501 [2] as illustrated by the figure 4.3.1. The system interworking procedures are specified in 3GPP TS 23.502 [6].

Figure 4.3.1: Non-roaming architecture for interworking between the 5GS and the EPC/E-UTRAN

The Nx interface is an inter-CN interface between the MME and the AMF used to enable interworking between the EPC and the 5GCN. Support of the Nx interface is optional. However, single-registration mode (see 3GPP TS 23.501 [5]) requires support for Nx interface between the AMF and the MME to enable seamless session continuity (e.g. for voice services) for N1 mode to S1 mode inter-system change and vice versa.

Editor's note:	The details of the N1 mode to S1 mode inter-system change and S1 mode to N1 mode inter-system change is FFS and will be captured by CT1 based on stage 2 requirements (e.g., UE behaviour, mapping of identities, mapping of contexts).

# 5	Network selection procedures

## 5.1	Concepts

The 5GS supports both 3GPP and non-3GPP access networks as specified in 3GPP TS 22.261 [3].

As per 3GPP TS 22.261 [3] the 5G system supports the concept of "network slices" where different 5G radio access networks potentially are connected to network slices of different SSTs.

Editor's note:	It is for further study how to support the concept of "network slices" of 5G radio access networks.

Requirements on PLMN selection for 3GPP access in 3GPP TS 22.011 [2], clause 3.2 apply together with new requirements specified by 3GPP TS 22.261 [3] subclause 6.19.2.

PLMN selection for 3GPP access does not depend on PLMN selection for non-3GPP access. If a UE is registered over non-3GPP access, the UE performs PLMN selection for 3GPP access independently of the PLMN to which the N3IWF belongs.

Editor's note:	It is FFS whether PLMN selection for non-3GPP access depends on PLMN selection for 3GPP access, e.g. PLMN for 3GPP access can be taken into account for PLMN selection for non-3GPP access.

PLMNs selected for 3GPP access and non-3GPP access may be the same or different.

PLMN selection defines a UE-based procedure, whereby candidate PLMNs are selected, one at a time, for attempted registration. Once the PLMN selection has been performed, the UE follows the signalling procedures defined for the selected RAT.

Editor's note:	It is for further study whether a common PLMN selection procedure can apply to both 3GPP access and non-3GPP access.

## 5.2	Procedures

### 5.2.1	General

The Radio Access Technology (RAT) identifier associated with each entry in the "User controlled PLMN selector with Access Technology", "Operator controlled PLMN selector with Access Technology" and "HPLMN selector with Access Technology" data files can indicate any combination of the values: GSM, GSM COMPACT, UTRAN, E-UTRAN, cdma2000® 1xRTT, cdma2000® HRPD and 5G-RAN access technologies.

Editor’s note:	It is FFS whether E-UTRA connected to 5GC access should be distinguished from E-UTRA connected to EPC and whether E-UTRA connected to 5GC should be distinguished from NR.

NOTE:	cdma2000® is a registered trademark of the Telecommunications Industry Association (TIA-USA).

Any signalling procedures after PLMN selection are defined in the specifications related with the radio access technology of the selected PLMN.

### 5.2.2	Procedures for 3GPP radio access networks

The same procedures for PLMN selection applicable for the GSM, UTRAN and E-UTRAN access technologies as described in 3GPP TS 22.011 [2] and 3GPP TS 23.122 [4] also apply for 5G-RAN, with the following additions and exceptions:

-	when the UE is informed that an area is forbidden, the tracking area identity for this area is added to a list of "forbidden areas" which is stored in the UE. The UE shall not send any signalling or user data in this area and shall search for a suitable cell in the same PLMN but belonging to a tracking area which is not in the list of "forbidden areas". The list of "forbidden areas" is deleted when the UE is switched off or the USIM is removed; and

Editor's note:	Whether the existing list of "forbidden tracking areas for roaming" can be re-used for the list "forbidden areas" is FFS.

-	one single list of forbidden PLMNs ("forbidden PLMNs" list) is sufficient for 5GS. The existing "forbidden PLMNs for GPRS service" and "forbidden PLMNs for attach in S101 mode" lists are not used for 5GS.

Editor's note:	It is FFS how RAT restriction is achieved.

### 5.2.3	Procedures for non-3GPP access networks

Editor's note:	Currently there are no new requirements available.

## 6	Stage 3 protocol, general aspects

## 6.1	General

This subclause defines the principal architecture of the 5GS NAS layer 3 protocol and its sublayers, including the message format applied by layer 3.

Editor's note:	5GS layer protocol and sublayer definition is FFS.

## 6.2	Protocols and peer to peer communication

Editor's note:	Definition of 5GS sublayers and functional blocks instances should be captured in this subclause.

## 6.3	Basic groups of functions

Editor's note:	5GS identifiers used for routing of 5GS NAS messages should be captured in this subclause.

## 6.4	Protocol architecture

Editor's note:	Figures for 5GS protocol architecture should be captured in this subclause.

## 6.5	Standard L3 messages

Editor's note:	5GS NAS message IE applicability, 5GS NAS message standard L3 header, 5GS Protocol discriminator values, Possible new 5GS identities for routing and 5GS message type should be captured in this subclause.

# 7	Coordination between the protocols for 5GS mobility management and 5GS session management

A 5GS session management (5GSM) message is piggybacked in a 5GS mobility management (5GMM) message. To this purpose, the 5GSM messages can be transmitted in an information element in the 5GMM messages. In this case, the UE, the AMF and the SMF execute the 5GMM procedure and the 5GSM procedure in parallel. The success of the 5GMM procedure is not dependent on the success of the piggybacked 5GSM procedure.

Editor's note:	It is FFS whether an initial registration request procedure can be used to piggyback 5GSM messages.

The UE can only initiate the 5GSM procedure when there is a 5GMM context established at the UE and the AMF can only forward the 5GSM message for the 5GSM procedure initiated by the SMF when there is a 5GMM context established at the AMF.

Except for the 5GMM procedures piggybacking 5GSM messages, during 5GMM procedures the UE and the AMF shall suspend the transmission of 5GSM messages.

Editor's note:	Which type of procedure will be included as 5GMM procedure is FFS.

# 8	5GS mobility management

Editor’s note:	The names of messages are informational and used for work in progress. Final 5GMM names are TBD.

## 8.1	Overview

### 8.1.1	General

The main function of the 5GS mobility management (5GMM) sublayer is to support the identification, security, mobility of a UE as well as generic message transport.

A further function of the 5GMM sublayer is to provide connection management services to the other sublayer(s).

Editor's note:	Sublayer design can be revisited after the protocol framework is identified.

### 8.1.2	Types of 5GMM procedures

Depending on how they can be initiated, three types of 5GMM procedures can be distinguished:

1)	5GMM common procedures

Editor's note:	More procedures (e.g. related to security, identification) will be updated according to the results of the work in SA2 and SA3.

Initiated by the network or the UE and used to carry a 5GMM message and/or any other encapsulated message(s) e.g. SMS or when the AMF wants to change UE configuration. The procedures belonging to this type are:

Initiated by the network:

network-initiated SM message transport.

-	network-initiated non-SM message transport.

-	network-initiated NAS transport.

- 	generic UE Configuration update

Initiated by the UE:

-	UE-initiated SM message transport.

-	UE-initiated non-SM message transport.

-	UE-initiated NAS transport.

2)	5GMM s pecific procedures:

At any time only one UE initiated 5GMM specific procedure can be running for each of the access network(s) that the UE is camping in. The procedures belonging to this type are:

Initiated by the UE and used e.g. to register to the network for 5GS services and establish a 5GMM context, to update the location/parameter(s) of the UE:

-	registration.

Initiated by the UE or the network and used to deregister from the network for 5GS services and to release a 5GMM context:

-	de-registration.

3)	5GMM c onnection management procedures:

Initiated by the UE and used to establish a secure connection to the network or to request the resource reservation for sending data, or both:

-	service request.

The service request procedure can only be initiated if no UE initiated 5GMM specific procedure is ongoing for each of the access network(s) that the UE is camping in.

Initiated by the network and used to request the establishment of an N1 NAS signalling connection or to prompt the UE to perform re-registration if necessary as a result of a network failure; not applicable for the non-3GPP access network:

-	paging.

Editor's note:	Alternative 1 is to define network/UE-initiated SM/non-SM message transport procedures (see subclause 8.4.2.2.1.1) and Alternative 2 is to define network/UE-initiated NAS transport procedures (see subclause 8.4.2.2.3.2). CT1 will choose a single alternative.

### 8.1.3	5GMM sublayer states

#### 8.1.3.1	General

In the following subclauses, the 5GS mobility management (5GMM) sublayer of the UE and the network is described by means of different state machines. . The 5GMM sublayer states is managed per access type independently, i.e. 3GPP access or non-3GPP access. In subclause 8.1.3.2, the states of the 5GMM sublayer over 3GPP access are introduced. The states of the 5GMM sublayer over non-3GPP access are described in subclause 11.1.3.3.

#### 8.1.3.2	5GMM sublayer states over 3GPP access

##### 8.1.3.2.1	General

In the following subclauses, the possible 5GMM sublayer states over 3GPP access are described. In subclause 8.1.3.2.2, the states of the 5GMM sublayer in the UE are introduced. The states for the network side are described in subclause 8.1.3.2.3.

##### 8.1.3.2.2	5GMM sublayer states in the UE

###### 8.1.3.2.2.1	General

In the following subclauses, the possible 5GMM sublayer states of the UE are described and shown in Figure 8.1.3.2.2.1.1.

Editor's note:	The substates of the main states and the detailed behaviour of the UE in the single states and substates are FFS.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

# Figure 8.1.3.2.2.1.1: 5GMM main states in the UE (3GPP access)

###### 8.1.3.2.2.2	Main states

8.1.3.2.2.2.1	5GMM-NULL

5GS services are disabled in the UE. No 5GS mobility management function shall be performed in this state.

8.1.3.2.2.2.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established and the UE location is unknown to the network and hence it is unreachable by a network. In order to establish a 5GMM context, the UE shall start the initial registration procedure.

8.1.3.2.2.2.3	5GMM-REGISTERED-INITIATED

A UE enters the state 5GMM-REGISTERED-INITIATED after it has started the initial registration procedure, the non-initial registration procedure (including the normal regstriation update and periodic regstriation update) or the service request procedure and is waiting for a response from the network.

8.1.3.2.2.2.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the UE. The UE may initiate the non-initial registration procedure (including the normal regstriation update and periodic regstriation update) and the service request procedure.

8.1.3.2.2.2.5	5GMM-DEREGISTERED-INITIATED

A UE enters the state 5GMM-DEREGISTERED-INITIATED after it has requested release of the 5GMM context by starting the deregistration procedure and is waiting for a response from the network.

##### 8.1.3.2.3	5GMM sublayer states in the network

###### 8.1.3.2.3.1	General

In the following subclauses, the possible 5GMM sublayer states of the network are described and shown in Figure 8.1.3.2.3.1.1.

Editor's note:	The detailed behaviour of the network in the single states is FFS.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

# Figure 8.1.3.2.3.1.1: 5GMM main states in the network (3GPP access)

###### 8.1.3.2.3.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established or the 5GMM context is marked as deregistered. The UE is deregistered. The network may answer to an initial registration procedure initiated by the UE. The network may also answer to a deregistration procedure initiated by the UE.

###### 8.1.3.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED

The network enters the state 5GMM-COMMON-PROCEDURE-INITIATED, after it has started a common 5GMM procedure and is waiting for a response from the UE.

###### 8.1.3.2.3.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the network.

###### 8.1.3.2.3.5	5GMM-DEREGISTERED-INITIATED

The network enters the state 5GMM-DEREGISTERED-INITIATED after it has started a deregistration procedure and is waiting for a response from the UE.

## 8.1.4	Mobile initiated connection only mode

The UE can request the use of mobile initiated connection only (MICO) mode during the registration procedure (see 3GPP TS 23.501 [5] and 3GPP TS 23.502[6]).

The network accepts the use of MICO mode by providing a MICO indication when accepting the registration procedure. The UE may use MICO mode only if the network has provided the MICO indication IE during the last registration procedure.

If the network accepts the use of MICO mode, the UE may deactivate the AS layer and activate MICO mode by entering the state 5GMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is in 5GMM-IDLE mode; and

b)	in the 5GMM-REGISTERED.NORMAL-SERVICE state.

When MICO mode is activated all NAS timers are stopped and associated procedures aborted except for periodic registration update timer, any backoff timers, and the timer T controlling the periodic search for HPLMN or EHPLMN or higher prioritized PLMNs (see 3GPP TS 23.122 [4]).

The UE may deactivate MICO mode and activate the AS layer at any time. Upon deactivating MICO mode, the UE may initiate 5GMM procedures (e.g. for the transfer of mobile originated signalling or user data).

If the AMF accepts the use of MICO mode, the AMF starts the implicit deregistration timer when entering 5GMM-IDLE mode. In this case, the value of the implicit detach timer is 4 minutes greater than the periodic registration update timer.

## 8.2	5GS mobility functions in IDLE and CONNECTED mode

### 8.2.1	Registration areas in the 5GS

#### 8.2.1.1	General

Within the 5GS, the registration area is managed independently per access type, i.e., 3GPP access or non-3GPP access. The AMF assigns a registration area to the UE during the registration procedure. A registration area is defined as a set of tracking areas and each of these tracking areas consists of one or more cells that cover a geographical area. Tracking areas cannot overlap each other. Within the 5GS, the concept of "registration to multiple tracking areas" applies:

-	A tracking area is identified by a TAI which is broadcast in the cells of the tracking area. The TAI is constructed from a TAC and a PLMN identifier. In case of a shared network, a single TAC and multiple PLMN identifiers are broadcast.

-	In order to reduce the tracking area update signalling within the 5GS, the AMF can assign several tracking areas to the UE. These tracking areas construct a list of tracking areas which is identified by a TAI list. When generating the TAI list, the AMF shall include only TAIs that are applicable on the access where the TAI list is sent. The AMF shall be able to allocate a TAI List over different 5G-RAN access technologies.

-	The UE considers itself registered to a list of tracking areas and does not need to trigger mobility registration update procedure (i.e. registration procedure with registration type set to "mobility registration update") as long as the UE stays in one of the tracking areas of the list of tracking areas received from the AMF.

Editor's note: 	The maximum number of tracking areas which can be allocated per UE needs to be defined which can be as in EPS or different.

-	The UE will consider the TAI list as valid, until it receives a new TAI list in the next mobily registration update or periodic registration upate procedure, or the UE  is commanded by the network to delete the TAI list by a reject message or it is detached from the 5GS. If the registration request is accepted or the TAI list is reallocated by the AMF, the AMF shall provide at least one entry in the TAI list. If the new and the old TAI list are identical, the AMF does not need to provide the new TAI list to the UE during mobily registration update or periodic registration update.

-	The TAI list can be reallocated by the AMF.

-	When the UE is detached from the 5GS, the TAI list in the UE is invalid.

-	The AMF allocates one globally unique temporary identity, which is common between 3GPP and non3GPP, to the UE.

Editor's note: 	The details about the handling of the temporary identity are FFS.

#### 8.2.1.2	Open issues on registration areas in the 5GS

-	CT1 need to study whether the periodic registration update timer for the periodic registration updating procedure and the periodic tracking area updating procedure in EPS can have different values, related values, or the same value.

-	CT1 need to study the impacts on the periodic registration update timer when the mobile initiated connection only (MICO) mode (see 3GPP TS 23.501 [5]) is used.

## 8.2.2	UE NAS mobility functions in 5GMM-IDLE mode

In 5GMM-IDLE mode a UE with a valid USIM enabled will:

-	perform cell selection/reselection procedure according to 3GPP TS 38.304 [9] and PLMN selection according to 3GPP TS 23.122 [4];

-	when not registered to the 5GCN:

-	perform registration procedure with registration type set to "initial registration" in order to receive services that require registration in the 5GS when not registered to the EPS;

-	perform registration procedure with registration type set to "mobility registration update" when registered to the EPS;

-	when registered to the 5GCN:

-	perform registration procedure with registration type set to "mobility registration update" when:

-	entering a tracking area not in the list of assigned tracking areas, in order to maintain the registration and enable the AMF to keep track of the UE; or

-	the UE needs to update its capabilities towards the 5GCN;

-	perform registration procedure with registration type set to "periodic registration update" to periodically notify the 5GS that the UE is available;

-	answer to paging from the AMF by performing a service request procedure;

-	perform the service request procedure in order to establish a N1 NAS signalling connection to the network or to request the establishment of radio bearers when uplink user data is to be sent, or both;

-	perform de-registration procedure when the UE is switched off, the USIM is removed or disabled, or the 5GS capability of the UE is disabled.

Editor's note: 	Whether the de-registration procedure is associated with an indication of whether it applies only to the access on which the de-registration procedure is run or to all accesses of the UE (3GPP and non-3GPP access) is FFS.

In 5GMM-IDLE mode a UE without valid USIM enabled will:

-	perform cell selection/reselection according to 3GPP TS 38.304 [9], but not PLMN selection.

Editor's note: 	How the UE without a valid USIM registers to the 5GCN for unauthenticated emergency services is FFS.

## 8.2.3	UE NAS mobility functions in 5GMM-CONNECTED mode

In 5GMM-CONNECTED mode a UE with a valid USIM enabled will:

-	respond to identification requests from the AMF;

-	perform authentication when requested by the AMF;

-	initiate the ciphering and integrity protection for NAS signalling when requested by the AMF;

-	not perform periodic registration update procedure (i.e. registration procedure with registration type set to "periodic registration update");

Editor's note: 	The need of a procedure to reallocate a new temporary identity to the UE is FFS.

-	when attached to the 5GCN:

-	perform mobility registration update procedure (i.e. registration procedure with registration type set to "mobility registration update") when after handover the UE detects that it has entered a tracking area not in the assigned list of tracking areas;

-	perform the service request procedure in order to request the establishment of the radio bearers when uplink user data is to be sent;

-	perform de-registration procedure when the UE is switched off, the USIM is removed or disabled, the 5GS capability of the UE is disabled, or when requested by the AMF.

Editor's note: 	The case of the UE without a valid USIM registered to the 5GCN and using emergency services is FFS.

## 8.3	IP address allocation

### 8.3.1	General

This clause specifies IP address allocation for the PDU session.

In this release of specification, PDU sessions for PDU type IP shall contain only one IP version, i.e. IPv4 PDU type or IPv6 PDU type.

IP address allocation to the UE shall be performed by SMF based on the selected IP version. If IPv4 PDU type is selected, an IPv4 address is allocated to the UE. If IPv6 PDU type is selected, an IPv6 prefix is allocated to the UE.

For IPv4 PDU type, the UE can obtain an IPv4 address and IPv4 configuration parameter (e.g. DNS server address)

-	via NAS signalling as specified in subclause 8.3.2; or

-	using DHCPv4.

For IPv6 PDU type, the UE can obtain /64 IPv6 prefix via IPv6 stateless address autoconfiguration as specified in RFC 4862 [12] and IPv6 configuration parameters via stateless DHCPv6 as specified in RFC 3736 [11].

### 8.3.2	IP address allocation via NAS signalling

The UE shall set the PDU session type IE in the PDU SESSION ESTABLISHMENT REQUEST message, based on its IP stack configuration if the UE requests IP connectivity as follows:

a)	A UE, which is IPv6 and IPv4 capable and:

-	has not been allocated an IP address for this DNN, shall set the PDU session type IE to IPv6.

-	the UE is requesting an IPv6 address for this DNN, shall set the PDU session type IE to IPv6.

-	the UE is requesting an IPv4 address for this DNN, shall set the PDN type IE to IPv4.

b)	A UE, which is only IPv4 capable, shall set the PDU session type IE to IPv4.

c)	A UE, which is only IPv6 capable, shall set the PDU session type IE to IPv6.

d)	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall set the PDU session type IE to IPv6.

Editor's note:	Whether a 5G UE will only support IPv4 is FFS.

Editor's note:	FFS whether the UE can request an IP PDU session type without indicating a particular IP version, i.e. the UE would indicate IP and network would select either IPv4 or IPv6 (but not both).

If the UE wants to use DHCPv4 for IPv4 address assignment, it shall indicate that to the network within the extended protocol configuration options IE in the PDU SESSION ESTABLISHMENT REQUEST.

## 8.4	5GS mobility management procedures

### 8.4.1	5GMM common procedures

### 8.4.1.1	NAS transport procedures

### 8.4.1.1.1	General

The purpose of the NAS transport procedures is to provide a transport of payload between the UE to the AMF. Two alternatives are described: subclause 8.4.1.1.2 describes Alternative 1 where there is a dedicated set of procedures for each of SM transport and non-SM transport; and subclause 8.4.1.1.3 describes Alternative 2 where there is a single set of procedures for both SM and non-SM transport.

### 8.4.1.1.2	Alternative 1 for NAS transport

### 8.4.1.1.1.1	SM transport procedures

Editor's note:	It is FFS whether to extend the UE-initiated SM message transport procedure and the network-initiated SM message transport procedure so that those procedures can transport any type of payload.

Editor's note:	It is FFS whether to extend the NAS SM MESSAGE TRANSPORT can be an initial 5GMM message.

Editor's note:	Transport between SMF and AMF depends on CT4 decisions.

#### 8.4.1.1.1.1.1	UE-initiated SM message transport procedure

##### 8.4.1.1.1.1.1.1	General

The purpose of the UE-initiated SM transport message procedure is to provide transport of:

-	an SM message; and

-	associated routing information (the PDU session ID, the S-NSSAI, the DNN, the request type);

from the UE to the AMF in the 5GMM message.

5GSM procedures specified in clause 9 describe conditions for inclusion of the S-NSSAI, and the DNN.

The request type is not provided along SM messages other than the PDU SESSION ESTABLISHMENT REQUEST message.

##### 8.4.1.1.1.1.1.2	UE-initiated SM message transport initiation

The UE shall set the N1 SM information IE of the NAS SM MESSAGE TRANSPORT message to the SM message.

The UE shall set the PDU session ID IE of the NAS SM MESSAGE TRANSPORT message to the PDU session ID.

If an S-NSSAI is to be included, the UE shall set the S-NSSAI IE of the NAS SM MESSAGE TRANSPORT message to the S-NSSAI.

If a DNN is to be included, the UE shall set the DNN IE of the NAS SM MESSAGE TRANSPORT message to the DNN.

If a request type is to be included, the UE shall set the request type IE of the NAS SM MESSAGE TRANSPORT message to the request type.

The UE shall send the NAS SM MESSAGE TRANSPORT message to the AMF (see example in figure 8.3.1.1.1.1.1.2.1).

# Figure 8.4.1.1.1.1.1.2.1: UE-initiated SM transport procedure

##### 8.4.1.1.1.1.1.3	UE-initiated SM message transport accepted by the network

Upon reception of an NAS SM MESSAGE TRANSPORT message, the AMF looks up a PDU session routing context for the PDU session ID of the NAS SM MESSAGE TRANSPORT message and the UE and::

a)	if the AMF has a PDU session routing context for the PDU session ID and the UE, and the request type IE is not included in the NAS SM MESSAGE TRANSPORT message, the AMF shall forward the SM message, and the PDU session ID of the NAS SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context;

b)	if the AMF has a PDU session routing context for the PDU session ID and the UE, and the request type IE is included in the NAS SM MESSAGE TRANSPORT message and is set to "existing PDU session", the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the NAS SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context;

c)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE,  and the request type IE is included in the NAS SM MESSAGE TRANSPORT message and is set to "initial request":

1)	the AMF shall select an SMF. If the DNN is not included in the NAS SM MESSAGE TRANSPORT message, the AMF shall use the default DNN as the DNN. If the S-NSSAI is not included in the NAS SM MESSAGE TRANSPORT message, the AMF may determine a default S-NSSAI according to the subscription context from UDM and, if determined, the AMF may use the default S-NSSAI as the S-NSSAI; and

NOTE:	SMF selection is out of scope of CT1.

2)	if the SMF selection is successful:

A)	the AMF shall store a PDU session routing context for the PDU session ID and the UE and shall set the SMF ID in the stored PDU session routing context to the SMF ID of the selected SMF; and

B)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the NAS SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context; and

d)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the request type IE of the NAS SM MESSAGE TRANSPORT message is included and is set to "existing PDU session", and the subscription context from UDM contains an SMF ID corresponding to:

1)	the DNN of the NAS SM MESSAGE TRANSPORT message, if the DNN is included in the NAS SM MESSAGE TRANSPORT message; or

2)	the default DNN, if the DNN is not included in the NAS SM MESSAGE TRANSPORT message;

Editor's note: FFS how to obtain the correct SMF ID when the UE has several PDU sessions with a given DNN, each PDU session is provided by a different SMF, and the UE attempts to handover one of those PDU sessions.

then:

1)	the AMF shall store a PDU session routing context for the PDU session ID and the UE, shall set the SMF ID in the stored PDU session routing context to the SMF ID corresponding to the DNN in the subscription context from UDM; and

2)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the NAS SM MESSAGE TRANSPORT message towards the SMF ID of with the PDU session routing context.

Editor's note:	FFS what to do when the AMF has a PDU session routing context for the PDU session ID and the UE, and the request type IE is included in the NAS SM MESSAGE TRANSPORT message and is set to "initial request". This can be a UE retransmission of an earlier PDU SESSION ESTABLISHMENT REQUEST message establishing a new PDU session, or it can be an error case.

##### 8.4.1.1.1.1.1.4	Abnormal cases on the network side

The following abnormal cases in AMF are identified:

a)	the AMF does not have a PDU session routing context for the PDU session ID of the NAS SM MESSAGE TRANSPORT message, the request type IE of the NAS SM MESSAGE TRANSPORT message is set to "initial request", and the SMF selection fails.

Editor's note:	Handling of this abnormal case is FFS

b)	the AMF does not have a PDU session routing context for the PDU session ID of the NAS SM MESSAGE TRANSPORT message, the request type IE of the NAS SM MESSAGE TRANSPORT message is set to "existing PDU session", and the subscription context from UDM does not contain an SMF ID corresponding to.

1)	the DNN of the NAS SM MESSAGE TRANSPORT message, if the DNN is included in the NAS SM MESSAGE TRANSPORT message; or

2)	the default DNN, if the DNN is not included in the NAS SM MESSAGE TRANSPORT message.

Editor's note:	Handling of this abnormal case is FFS

#### 8.4.1.1.1.1.3	Network-initiated SM message transport procedure

##### 8.4.1.1.1.1.3.1	General

The purpose of the network-initiated SM message transport procedure is to provide transport of an SM message and a PDU session ID from the AMF to the UE in a 5GMM message.

##### 8.4.1.1.1.1.3.2	Network-initiated SM message transport initiation

Upon:

a)	reception of an SM message without an N1 SM delivery skip allowed indication for a UE served from an SMF; or

b)	reception of an SM message, and an N1 SM delivery skip allowed indication for a served UE from an SMF and if the UE is in the CM-CONNECTED state;

the AMF shall create a NAS SM MESSAGE TRANSPORT message.

The AMF shall set the N1 SM information IE of the NAS SM MESSAGE TRANSPORT message to the SM message from the SMF.

The AMF shall set the PDU session ID IE of the NAS SM MESSAGE TRANSPORT message to the PDU session ID.

Editor's note:	FFS how AMF determines the PDU session ID - (a) the AMF can receive the PDU session ID along the SM message in the N11 message from the SMF, or (b) if there is one N11 connection for each PDU session then the AMF can locally derive the PDU session ID from identifier of the N11 connection via which the N11 message is received.

The AMF shall send the NAS SM MESSAGE TRANSPORT message to the UE (see example in figure 8.4.1.1.1.1.3.2.1).

# Figure 8.4.1.1.1.1.3.2.1: UE-initiated SM transport procedure

##### 8.4.1.1.1.1.3.3	Network-initiated SM message transport accepted by the UE

The SM message and the PDU session ID of a received NAS SM MESSAGE TRANSPORT message is handled in the 5GSM procedures specified in clause 9.

###### 8.4.1.1.1.2	Non-SM message transport procedures

#### 8.4.1.1.1.2.1	General

The purpose of the procedures is to provide a transport of non-SM messages between the UE and the AMF. The type of the non-SM message is identified by the message container type IE and corresponds to one of the followings:

-	SMS.

Editor's note:	Other types, such as location-services, are FFS.

Editor's note:	It is FFS whether a message belonging to the procedures could be an initial 5GMM message.

#### 8.4.1.1.1.2.2	UE-initiated non-SM message transport procedure

##### 8.4.1.1.1.2.2.1	General

The purpose of the UE-initiated non-SM message transport procedure is to provide a transport of:

-	SMS

from the UE to the AMF in a 5GMM message.

##### 8.4.1.1.1.2.2.2	UL GENERIC TRANSPORT message sending

The UE shall set the message container IE of the UL GENERIC TRANSPORT message to the non-SM message.

The UE shall set the message container type IE of the UL GENERIC TRANSPORT message to the type of the non-SM message.

The UE shall send the UL GENERIC TRANSPORT message to the AMF (see example in figure 8.4.1.1.1.2.2.2.1).

Figure 8.4.1.1.1.2.2.2.1: UE-initiated non-SM message transport procedure

##### 8.4.1.1.1.2.2.3	UL GENERIC TRANSPORT message receiving

Upon reception of an UL GENERIC TRANSPORT message with the message container type IE set to "SMS":

a)	if the AMF needs to select an SMSF, i.e. the AMF does not have SMSF information associated with the UE:

1)	the AMF shall select an SMSF; and

2)	if the SMSF selection is successful, the AMF shall associate the selected SMSF with the UE; and

Editor's note: FFS what to do if the SMSF selection fails.

b)	the AMF shall forward the SMS message of the UL GENERIC TRANSPORT message towards the SMSF associated with the UE.

#### 8.4.1.1.1.2.3	Network-initiated non-SM message transport procedure

##### 8.4.1.1.1.2.3.1	General

The purpose of the network-initiated non-SM message transport procedure is to provide a transport of:

-	SMS

from the AMF to the UE in a 5GMM message.

##### 8.4.1.1.1.2.3.2	DL GENERIC TRANSPORT message sending

Upon reception of a non-SM message for a UE from the other network function (e.g. SMSF), the AMF shall create a DL GENERIC TRANSPORT message.

The AMF shall set the message container IE of the DL GENERIC TRANSPORT message to the non-SM message from the other network function.

The AMF shall set the message container type IE of the DL GENERIC TRANSPORT message to the type of the non-SM message.

The AMF shall select the acces network (either 3GPP or non-3GPP) through which the DL GENERIC TRANSPORT message is sent according to information received from the other network function that provided the non-SM message.

The AMF shall send the DL GENERIC TRANSPORT message to the UE (see example in figure 8.4.1.1.1.2.3.2.1).

Figure 8.4.1.1.1.2.3.2.1: Network-initiated non-SM message transport procedure

##### 8.4.1.1.1.2.3.3	DL GENERIC TRANSPORT message receiving

Upon reception of a DL GENERIC TRANSPORT message, if the message container type IE is set to:

a) "SMS", the UE shall forward the content of the message container IE to the SMS entity.

#### 8.4.1.1.3	Alternative 2 for NAS Transport

##### 8.4.1.1.3.1	General

The purpose of the NAS transport procedures is to provide a transport of payload between the UE to the AMF. The type of the payload is identified by the Payload type information element and includes one or more of the following:

-	a single 5GSM message; or

-	SMS.

Editor’s note: Other types of payload, such as generic application payload, location-services related payload, are FFS.

Along with the payload, the NAS transport procedure may transport the associated payload information (e.g. PDU session information for 5GSM message payload etc.).

Editor's note:	It is FFS whether the DL NAS TRANSPORT and UL NAS TRANSPORT could be initial 5GMM messages.

##### 8.4.1.1.3.2	UE-initiated NAS transport procedure

###### 8.4.1.1.3.2.1	General

The purpose of the UE-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message; or

b)	SMS

and optional associated payload routing information from the UE to the AMF in a 5GMM message.

NOTE:	For case a) the procedures specified below may be supplemented by the related aspects of the procedures in subclause 8.4.1.1.1.1.3.2.1 during the normative phase.

###### 8.4.1.1.3.2.2	NAS transport procedure initiation

In the connected mode, the UE initiaties the NAS transport procedure by sending the UL NAS TRANSPORT message, as shown in figure 8.4.1.1.3.2.2.1.

In case a) in subclause 8.4.1.1.3.2.1, the UE shall:

-	include the PDU session information (PDU session ID, S-NSSAI, DNN, Request type) in the Payload information IE.

-	set the Payload type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

5GSM procedures specified in clause 8 describe conditions for inclusion of the S-NSSAI, and the DNN.

In case b) in subclause 8.4.1.1.3.2.1, the UE shall:

-	set the Payload type IE to "SMS"; and

-	set the Payload container IE to the SMS payload.

Editor’s note: Whether Payload information IE is needed for SMS payload or not is FFS.

# Figure 8.4.1.1.3.2.2.1: UE-initiated NAS transport procedure

###### 8.4.1.1.3.2.3	NAS transport accepted by the network

Upon reception of an UL NAS TRANSPORT message, if the Payload type IE is set to:

a)	"N1 SM information":

i)	if the AMF needs to select an SMF:

Editor's note: FFS under which condition the AMF needs to select an SMF.

1)	the AMF shall select an SMF. If the DNN is not included in Payload information IE, the AMF shall use the default DNN as the DNN; and

Editor's note: SMF selection is to be specified by CT4.

2)	if the SMF selection is successful, the AMF shall associate the selected SMF with the PDU session ID; and

Editor's note: It is FFS what to do if the SMF address selection fails

ii)	the AMF shall forward the PDU session ID, the 5GSM message in the Payload container IE, the S-NSSAI (if received), and the DNN (if received) of the UL NAS TRANSPORT message towards the SMF associated with the PDU session ID:

b)	"SMS", the AMF shall forward the content of the Payload container IE to the SMSF or UDM.

##### 8.4.1.1.3.3	Network-initiated NAS transport procedure

###### 8.4.1.1.3.3.1	General

The purpose of the network-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message; or

b)	SMS

from the AMF to the UE in a 5GMM message.

NOTE:	For case a) the procedures specified below may be supplemented by the related aspects of the procedures in subclause 8.4.1.1.3.1 during the normative phase.

###### 8.3.1.1.3.3.2	NAS transport procedure initiation

In connected mode, the AMF initiaties the NAS transport procedure by sending the DL NAS TRANSPORT message, as shown in figure 8.4.1.1.3.3.2.1.

In case a) in subclause 8.4.1.1.3.3.1, the AMF shall:

-	include the PDU session information (PDU session ID) in the Payload information IE;

-	set the Payload type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

In case b) in subclause 8.4.1.1.3.3.1, the AMF shall:

-	set the Payload type IE to "SMS"; and

-	set the Payload container IE to the SMS payload.

# Figure 8.4.1.1.3.3.2.1: Network-initiated NAS transport procedure

###### 8.4.1.1.3.3.3	NAS transport procedure accepted by UE

Upon reception of a DL NAS TRANSPORT message, if the Payload type IE is set to:

a)	 "N1 SM information":

The 5GSM message in the Payload container IE is handled in the 5GSM procedures specified in clause 8.

b)	"SMS", the UE shall forward the content of the Payload container IE to the SMS stack entity.

### 8.4.1.2	Generic UE configuration update procedure

#### 8.4.1.2.1	General

The purpose of this procedure is to allow the AMF to update UE configuration. The procedure may be initiated by the network and can only be used when the UE has an established 5GMM context, and the UE is in 5GMM-CONNECTED mode. The AMF may require a confirmation response in order to ensure that the parameter has been updated by the UE.

Editor’s note:	How the AMF requires confirmation from the UE is FFS.

Editor's note:	It is FFS which configuration parameters are updated by this procedure (e.g. Mobility Restrictions, Temporary User ID, TAI list, UE NSSAI information, and policy information).

Editor's note:	It is FFS whether the case when configuration provided by a different NF than the AMF is covered by this or a different NAS procedure (e.g. UE route selection policies (URSP) provided by PCF).

Figure 8.4.1.2.1.1: UE configuration update procedure

#### 8.4.1.2.2	UE Configuration update procedure initiated by the network

The AMF shall initiate the UE configuration procedure by sending the CONFIGURATION UPDATE COMMAND to the UE. If the AMF requests a confirmation it shall start timer Tx.

#### 8.4.1.2.3	UE Configuration update accepted by the UE

Upon receiving the CONFIGURATION UPDATE COMMAND, the UE shall store the received parameters. If the AMF requires a confirmation response, the UE shall send a CONFIGURATION UPDATE COMPLETE message.

Editor's note:	Further details on handling of specific IEs are FFS.

#### 8.4.1.2.4	UE Configuration update completion by the network

Upon receipt of the CONFIGURATION UPDATE ACCEPT message, the AMF shall stop the timer Tx and consider information provided in the CONFIGURATION UPDATE COMMAND message successfully accepted by the UE.

Editor's note:	Further details on handling of specific IEs are FFS.

#### 8.4.1.2.5	Abnormal cases in the UE

The following abnormal cases may occur

- Transmission failure of the CONFIGURATION UPDATE COMPLETE message.

Editor's note:	Further abnormal cases in the UE are FFS.

#### 8.4.1.2.6	Abnormal cases on the network side

- Expiry of timer Tx.

Editor's note:	Further abnormal cases in the network are FFS.

### 8.4.2	5GMM specific procedures

### 8.4.2.1	Registration procedure

### 8.4.2.1.1	General

The registration procedure is always initiated by the UE and can be used for the following purposes:

-	initial registration for 5GS services;

-	mobility registration updating to update the registration of the actual tracking area of a UE in the network;

-	periodic registration updating to periodically notify the availability of the UE to the network;

-	initial registration for emergency services.

Editor's note:	Further purposes are FFS.

##### 8.4.2.1.2	Registration procedure for initial registration

##### 8.4.2.1.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

When the UE initiates the registration procedure for initial registration, the UE shall indicate "initial registration" in the 5GS registration type IE.

###### 8.4.2.1.2.2	Initial registration initiation

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer Ta. If timer Tb is currently running, the UE shall stop timer Tb. If timer Tc is currently running, the UE shall stop timer Tc.

# Figure 8.4.2.1.2.2.1: Registration procedure for initial registration

###### 8.4.2.1.2.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

###### 8.4.2.1.2.4	Initial registration accepted by the network

##### If the initial registration request is accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE and start timer Td.

##### Editor's note:	Further details on initial registration accepted by the network are FFS.

###### 8.4.2.1.2.5	Initial registration not accepted by the network

###### If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed);

Editor's note:	Further details on initial registration not accepted by the network are FFS.

###### 8.4.2.1.2.6	Initial registration for emergency services not accepted by the network

Editor's note:	Possible handling of initial registration for emergency services by the network is FFS.

###### 8.4.2.1.2.7	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.4.2.1.2.8	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

##### 8.4.2.1.3	Registration procedure for mobility and periodic registration update

###### 8.4.2.1.3.1	General

This procedure can be used by a UE for mobility and periodic registration update for 5GS services.

When the UE initiates the registration procedure for mobility and periodic registration update, the UE shall indicate "mobility registration update" or "periodic registration update" respectively in the 5GS registration type IE.

The periodic registration updating procedure is controlled in the UE by timer Te. When timer Te expires, the periodic tracking area updating procedure is started. Start and reset of timer Te is described in subclause 8.5.

###### 8.4.2.1.3.2	Mobility and periodic registration update initiation

The UE in state 5GMM-REGISTERED shall initiate the mobility and periodic registration updating procedure by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the AMF; or

b)	when the periodic registration updating timer Te expires.

Editor's note:	Further cases for initiation of mobility and periodic registration update are FFS.

After sending the REGISTRATION REQUEST message to the AMF the UE shall start timer Ta. If timer Tb is currently running, the UE shall stop timer Tb. If timer Tc is currently running, the UE shall stop timer Tc.

# Figure 8.4.2.1.3.2.1: Registration procedure for mobility and periodic registration update

###### 8.4.2.1.3.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

###### 8.4.2.1.3.4	Mobility and periodic registration update accepted by the network

If the registration update request has been accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE. If the AMF assigns a new temporary user ID for the UE, a temporary user ID shall be included in the REGISTRATION ACCEPT message. In this case, the AMF shall start timer Td.

Editor's note:	Further details on mobility and periodic registration update accepted by the network are FFS.

###### 8.4.2.1.3.5	Mobility and periodic registration update not accepted by the network

If the mobility and periodic registration update request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed);

Editor's note:	Further details on mobility and periodic registration update not accepted by the network are FFS.

###### 8.4.2.1.3.6	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.4.2.1.3.7	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

### 8.4.3	5GMM connection management procedures

### 8.4.3.1	Service request procedure

#### 8.4.3.1.1	General

The purpose of the service request procedure is to change the 5GMM mode from 5GMM-IDLE to 5GMM-CONNECTED mode, and/or to request establishment of user-plane radio resources for PDU sessions which are activated without radio resources. In latter case, the 5GMM mode can be the 5GMM-IDLE mode or the 5GMM-CONNECTED mode if the UE requires radio resources. This procedure is used when:

-	the network has downlink signalling pending;

-	the UE has uplink signalling pending;

-	the network has user data pending and the UE is in 5GMM-IDLE mode; or

-	the UE has user data pending and the UE is in 5GMM-IDLE or 5GMM-CONNECTED mode.

The service request procedure is initiated by the UE, however, for the downlink transfer of signalling or user data in 5GMM-IDLE mode, the trigger is given by the network by means of the paging procedure (see subclause 8.4.3.2).

Editor's note:	The definition of 5GMM-IDLE mode and 5GMM-CONNECTED mode are FFS.

# Figure 8.4.3.1.1.1: Service Request procedure

#### 8.4.3.1.2	Service request procedure initiation

The UE initiates the service request procedure by sending a SERVICE REQUEST message to the AMF and starts timer Tx.

If the service request is triggered for user data pending, the PDU session ID(s) shall be included in the SERVICE REQUEST message to indicate the PDU session(s) that the UE needs to use. If the service request is triggered for signalling pending only, the PDU session ID(s) is not included in the SERVICE REQUEST message. When this procedure is triggered for paging response, if the UE needs to use some PDU session(s), the PDU session ID(s) shall be included in the SERVICE REQUEST message to indicate the PDU session(s) that the UE needs to use.

The PDU session status information element may be included in the SERVICE REQUEST message to indicate the PDU session(s) available in the UE.

Editor's note:	The content of the SERVICE REQUEST message is FFS.

#### 8.4.3.1.3	Common procedure initiation

Upon receipt of the SERVICE REQUEST message, the AMF may initiate the common procedures e.g. the authentication procedure and security procedure.

Editor's note:	Common procedures will be updated according to the results of the works in SA2 and SA3.

#### 8.4.3.1.4	Service request procedure accepted by the network

If the SERVICE REQUEST message was sent, then the UE shall treat the reception of the SERVICE ACCEPT message as successful completion of the procedure and stop timer Tx.

The PDU session status information element may be included in the SERVICE ACCEPT message to indicate the PDU session(s) available in the network.

Editor's note:	The content of the SERVICE ACCEPT message is FFS.

Editor's note:	Whether the indication from the lower layers that the user-plane radio resources are set up needs to be considered as a successful completion of the procedure is FFS.

Editor's note:	Timers are FFS.

#### 8.4.3.1.5	Service request procedure not accepted by the network

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate 5GMM cause value and stop timer Tx.

Editor's note:	The content of the SERVICE REJECT message is FFS.

Editor's note:	Timers are FFS.

#### 8.4.3.1.6	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

#### 8.4.3.1.7	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

### 8.4.3.2	Paging procedure

#### 8.4.3.2.1	General

The paging procedure is used by the network to request the establishment of an NAS signalling connection to the UE.

#### 8.4.3.2.2	Paging for 5GS services

##### 8.4.3.2.2.1	General

The network shall initiate the paging procedure for 5GS services when NAS signalling messages or user data is pending to be sent to the UE when no NAS signalling connection exists (see example in figure 8.4.3.2.2.1).

Figure 8.4.3.2.2.1: Paging procedure

To initiate the procedure the 5GMM entity in the network requests the lower layer to start paging and shall start timer Ty. Upon reception of a paging indication, the UE shall initiate a service request procedure to respond to the paging (see 3GPP TS 23.502 [6].

Editor's note:	It is FFS how the paging procedure is performed depending on different UE Identity of the paging message.

The network shall stop timer Ty for the paging procedure when an integrity-protected response is received from the UE and successfully integrity checked by the network. If the response received is not integrity protected, or the integrity check is unsuccessful, timer Ty for the paging procedure shall be kept running.

Upon expiry of timer Ty, the network may reinitiate paging.

Editor's note:	Timers are FFS.

##### 8.4.3.2.2.2	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

## 8.5	5GS mobility management message coding

### 8.5.1	Registration request

#### 8.5.1.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the network.

Message type:	REGISTRATION REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the REGISTRATION REQUEST message is FFS.

### 8.5.2	Registration accept

#### 8.5.2.1	Message definition

The REGISTRATION ACCEPT message is sent by the network to the UE.

Message type:	REGISTRATION ACCEPT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the REGISTRATION ACCEPT message is FFS.

### 8.5.3	Registration complete

#### 8.5.3.1	Message definition

The REGISTRATION COMPLETE message is sent by the UE to the network.

Message type:	REGISTRATION COMPLETE

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the REGISTRATION COMPLETE message is FFS.

### 8.5.4	Registration reject

#### 8.5.4.1	Message definition

The REGISTRATION REJECT message is sent by the network to the UE.

Message type:	REGISTRATION REJECT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the REGISTRATION REJECT message is FFS.

### 8.5.5	NAS SM message transport

#### 8.5.5.1	Message definition

NAS SM MESSAGE TRANSPORT message conveys an SM message and associated information.

# Message type:	NAS SM MESSAGE TRANSPORT

Significance:		dual

Direction:			both

Editor's note:	Content of the NAS SM MESSAGE TRANSPORT message is FFS.

### 8.5.6	UL GENERIC TRANSPORT for Alternative 1

#### 8.5.6.1	Message definition

UL GENERIC TRANSPORT message transports a non-SM message and associated information from the UE to the network.

Message type:	UL GENERIC TRANSPORT

Significance:		dual

Direction:			UE to network

### 8.5.7	UL NAS TRANSPORT for Alternative 2

#### 8.5.7.1	Message definition

UL NAS TRANSPORT message transports message payload and associated information.

# Message type:	UL NAS TRANSPORT

Significance:		dual

Direction:			UE to AMF

Editor's note: content of UL NAS TRANSPORT message is FFS.

### 8.5.8	DL GENERIC TRANSPORT for Alternative 1

#### 8.5.8.1	Message definition

DL GENERIC TRANSPORT message transports a non-SM message and associated information from the network to the UE.

Message type:	DL GENERIC TRANSPORT

Significance:		dual

Direction:			Network to UE

### 8.5.9	DL NAS TRANSPORT for Alternative 2

#### 8.5.9.1	Message definition

DL NAS TRANSPORT message transports message payload and associated information.

# Message type:	DL NAS TRANSPORT

Significance:		dual

Direction:			AMF to UE

Editor's note: content of DL NAS TRANSPORT message is FFS.

##### 8.6	Timers of 5GS mobility management

Timers of 5GS mobility management are shown in table 8.6.1 and table 8.6.2

# Table 8.6.1: Timers of 5GS mobility management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Ta | TBD | TBD | Transmission of REGISTRATION REQUEST message. | REGISTRATION ACCEPT message received or REGISTRATION REJECT message received. | Retransmission of REGISTRATION REQUEST message. |
| Tb | TBD | TBD | At registration failure and the attempt counter is equal to 5. | REGISTRATION REQUEST sent | Initiation of the registration procedure, if still required |
| Tc | TBD | TBD |  | REGISTRATION REQUEST sent | Retransmission of the REGISTRATION REQUEST, if still required |
| Te | TBD | TBD | In 5MM-REGISTERED, when 5MM-CONNECTED mode is left. | When entering state 5MM-DEREGISTERED or when entering 5MM-CONNECTED mode. | Initiation of the periodic registration procedure |

# Table 8.6.2: Timers of 5GS mobility management – AMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Td | TBD | TBD | REGISTRATION ACCEPT message sent at initial registration. REGISTRATION ACCEPT message sent with Temporary User ID at mobility or periodic registration. | REGISTRATION COMPLETE received | Retransmission of REGISTRATION ACCEPT message. |

# 9	5GS session management

Editor’s note:	The names of messages are informational and used for work in progress. Final 5GSM names are TBD.

## 9.1	General

The 5GS architecture reference model defined in 3GPP TS 23.501 [5] shows that the N11 reference point is used to connect the core access and mobility management function (AMF) and the session management function (SMF). The SMF handles the session management part of the NAS signalling exchanged with the UE. 3GPP TS 23.502 [6] specifies that session management procedures (e.g., PDU session establishment) are not performed between the AMF and the SMF but the UE and the SMF.

CT1 will specify the control-plane 5G session management (5GSM) towards the UE and the SMF. This includes the procedures, messages, information elements and the necessary error handling for the session management part of the NAS signalling.

## 9.2	Overview

The SMF is responsible for the session management functions to provide the PDU connectivity service to the UE via the SM signalling between UE and SMF.  The session management procedures includes:

a)	the UE-requested PDU session establishment procedure;

b)	the PDU session establishment authentication and authorization procedure;

c)	the UE-requested PDU session modification procedure;

d)	the network-requested PDU session modification procedure; and

e)	the UE-requested PDU session release procedure; and

f)	the network-requested PDU session release procedure.

A UE may establish multiple PDU sessions, to the same data network or to different data networks, via 3GPP and via and Non-3GPP access networks at the same time.

The session management messages between UE and SMF are transferred via AMF as specified in subclause 9.5.

## 9.3	5GSM sublayer states

#### 9.3.1	General

In the following subclauses, the possible states of a 5G PDU session context in the UE and the network side are described.

Editor's note:	The procedure transaction states for 5GSM are FFS.

#### 9.3.2	5GSM sublayer states in the UE

##### 9.3.2.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the UE are described and shown in figure 9.3.2.1.1.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

# Figure 9.3.2.1.1: The 5GSM sublayer states for PDU session handling in the UE (overview)

##### 9.3.2.2	PDU SESSION INACTIVE

No PDU session context exists.

##### 9.3.2.3	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

##### 9.3.2.4	PDU SESSION ACTIVE

The PDU session context is active in the UE.

##### 9.3.2.5	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

##### 9.3.2.6	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

##### 9.3.2.7	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

##### 9.3.2.8	PDU SESSION ACTIVE

The PDU session context is active in the UE.

##### 9.3.2.9	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

##### 9.3.2.10	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

#### 9.3.3	5GSM sublayer states in the network

#### 9.3.3.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the network are described and shown in Figure 9.3.3.1.1.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

Figure 9.3.3.1.1: The 5GSM sublayer states for PDU session handling in the network (overview)

##### 9.3.3.2	PDU SESSION INACTIVE

No PDU session context exists.

##### 9.3.3.3	PDU SESSION ACTIVE

The PDU session context is active in the network.

##### 9.3.3.4	PDU SESSION INACTIVE PENDING

The network has initiated a PDU session release procedure towards the UE and is waiting for a response from the UE.

##### 9.3.3.5	PDU SESSION MODIFICATION PENDING

The network has initiated a PDU session modification procedure towards the UE and is waiting for a response from the UE.

## 9.4	5GS session management procedures

### 9.4.1	UE-requested PDU session establishment procedure

Editor's note:	Transport between SMF and AMF depends on CT4 decisions.

#### 9.4.1.1	General

The purpose of the UE-requested PDU session establishment procedure is to establish a new PDU session with a DN or to perform handover of an existing PDU session between 3GPP access and non-3GPP access. If accepted by the network, the PDU session enables exchange of PDUs between the UE and the DN.

#### 9.4.1.2	UE-requested PDU session establishment procedure initiation

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

If the UE requests to establish a new PDU session with a DN and requests a PDU session type, the UE shall set the PDU session type IE of the PDU SESSION ESTABLISHMENT REQUEST message to the IP version capability as specified in subclause 9.3.2, "ethernet" or "unstructured".

If the UE requests to establish a new PDU session with a DN and the UE requests an SSC mode, the UE shall set the SSC mode IE of the PDU SESSION ESTABLISHMENT REQUEST message to the SSC mode.

If the UE requests to establish a new PDU session with a DN and needs to provide information for the PDU session authentication and authorization by the external DN, the UE shall include the PDU DN request container with the information for the PDU session authentication and authorization by the external DN.

Editor's note:	FFS whether CT1 need to give more details about coding of the information for the PDU session authentication and authorization by the external DN.

Editor's note:	Further he contents of the PDU SESSION ESTABLISHMENT REQUEST are FFS.

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID;

c)	the S-NSSAI, if the UE requests a S-NSSAI other than default S-NSSAI;

d)	and the requested DNN, if the UE requests a connectivity to a DNN other than the default DNN; and

e)	the request type set:

1)	to "initial request", if the UE requests to establish a new PDU session; or

2)	to "existing PDU session", if the UE requests handover of an existing PDU session between 3GPP access and non-3GPP access;

using the UE-initiated SM message transport procedure as specified in subclause 8.4.2.2.1.1 or the UE-initiated NAS transport procedure as specified in subclause 8.4.1.1.3.2, and the UE shall start timer Tx (see example in figure 9.4.1.2.1).

# Figure 9.4.1.2.1: UE-requested PDU session establishment procedure

Upon receipt of a PDU SESSION ESTABLISHMENT REQUEST message, a PDU session ID, optionally a S-NSSAI, and optionally a DNN, and the request type, the SMF checks whether connectivity with the requested DN can be established. If the requested DNN is not included, the SMF shall use the default DNN. . If the S-NSSAI is not included, the SMF may determine a default S-NSSAI according to the subscription context from UDM and, if determined, the SMF may use the default S-NSSAI as the S-NSSAI.

#### 9.4.1.3	UE-requested PDU session establishment procedure accepted

If the connectivity with the requested DN is accepted by the network, the SMF shall create a PDU SESSION ESTABLISHMENT ACCEPT message.

The SMF shall set the authorized QoS rules IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the authorized QoS rules of the PDU session.

The SMF shall set the selected SSC mode IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected SSC mode of the PDU session.

The SMF shall set the selected S-NSSAI IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected S-NSSAI of the PDU session.

If the PDU SESSION ESTABLISHMENT REQUEST message does not include a PDU session type IE, the SMF shall set the selected PDU session type IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected PDU session type of the PDU session.

If the selected PDU session type is "IPv4", the SMF shall include the PDU address IE in the PDU SESSION ESTABLISHMENT ACCEPT message and shall set the PDU address IE to an IPv4 address is allocated to the UE in the PDU session.

The SMF shall set the selected DNN IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected DNN of the PDU session.

The SMF shall set the selected Session-AMBR IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the Session-AMBR of the PDU session.

Editor's note:	Further contents of the PDU SESSION ESTABLISHMENT ACCEPT are FFS.

The SMF shall send the PDU SESSION ESTABLISHMENT ACCEPT message.

Upon receipt of a PDU SESSION ESTABLISHMENT ACCEPT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE shall stop timer Tx and shall consider that the PDU session was established.

#### 9.4.1.4	UE requested PDU session establishment procedure rejected by SMF

If the connectivity with the requested DN is rejected by the network, the SMF shall create a SM PDU SESSION ESTABLISHMENT REJECT message.

The SMF shall set the SM cause IE of the PDU SESSION ESTABLISHMENT REJECT message to indicate the reason for rejecting the PDU session establishment.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note: SM causes are FFS.

Editor's note:	Further contents of the SM PDU SESSION ESTABLISHMENT REJECT are FFS.

The SMF shall send the SM PDU SESSION ESTABLISHMENT REJECT message.

Upon receipt of a PDU SESSION ESTABLISHMENT REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE shall stop timer Tx and shall consider that the PDU session was not established.

#### 9.4.1.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tx expired

Editor's note:	Further abnormal cases in the UE are FFS.

#### 9.4.1.6	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network side are FFS.

### 9.4.2	PDU session establishment authentication and authorization procedure

#### 9.4.2.1	General

The purpose of the PDU session establishment authentication and authorization procedure is to enable the DN:

1)	to authenticate the UE establishing the PDU session;

2)	to authorize the UE establishing the PDU session; or

3)	both of the above.

The PDU session establishment authentication and authorization procedure can be performed only during the UE-requested PDU session establishment procedure.

Editor's note:	FFS whether a dedicated messages for PDU session establishment authentication and authorization procedure are needed or whether some kind of a generic SM message can be defined and used.

#### 9.4.2.2	PDU session establishment authentication and authorization procedure initiation

In order to initiate the PDU session establishment authentication and authorization procedure, the SMF shall create a PDU SESSION AUTHENTICATION REQUEST message.

The SMF shall set the authentication information IE of the PDU SESSION AUTHENTICATION REQUEST message to the authentication information provided by the DN.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION REQUEST message are FFS.

The SMF shall send the PDU SESSION AUTHENTICATION REQUEST message, and the SMF shall start timer Tm (see example in figure 9.4.2.2.1).

# Figure 9.4.2.2.1: PDU session establishment authentication and authorization procedure

Upon receipt of a PDU SESSION AUTHENTICATION REQUEST message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE passes the authentication information IE of the PDU SESSION AUTHENTICATION REQUEST message to the upper layers.

#### 9.4.2.3	PDU session establishment authentication and authorization procedure accepting

If the upper layers provide a responding authentication information, and the UE shall create a PDU SESSION AUTHENTICATION ACCEPT message.

The UE shall set the authentication information IE of the PDU SESSION AUTHENTICATION ACCEPT message to the responding authentication information.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION ACCEPT message are FFS.

The UE shall transport the PDU SESSION AUTHENTICATION ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.

Upon receipt of a PDU SESSION AUTHENTICATION ACCEPT message, the SMF shall stop timer Tm and provides the authentication information to the DN.

#### 9.4.2.4	PDU session establishment authentication and authorization procedure rejecting

If the upper layers does not accept or does not provide a responding authentication information, the UE shall create a PDU SESSION AUTHENTICATION REJECT message.

The UE shall set the SM cause IE of the PDU SESSION AUTHENTICATION REJECT message to indicate the reason for rejecting the PDU SESSION AUTHENTICATION REQUEST message.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note:	SM causes are FFS.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION REJECT message are FFS.

The UE shall transport the PDU SESSION AUTHENTICATION REJECT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.2.2.1.1.

Upon receipt of a PDU SESSION AUTHENTICATION REJECT message, the SMF shall stop timer Tm.

#### 9.4.2.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Tm expired

Editor's note:	Further abnormal cases are FFS

#### 9.4.2.6	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS

### 9.4.3	UE-requested PDU session modification procedure

#### 9.4.3.1	General

The purpose of the UE-requested PDU session modification procedure is to enable the UE to request modification of a PDU session.

Editor’s note:	It is FFS whether the PDU Session ID is also included in the PDU SESSION MODIFICATION REQUEST message and the PDU SESSION MODIFICATION REJECT message.

#### 9.4.3.2	UE-requested PDU session modification procedure initiation

In order to initiate the UE-requested PDU session modification procedure, the UE shall create a PDU SESSION MODIFICATION REQUEST message.

The UE shall set the PTI IE of the PDU SESSION MODIFICATION REQUEST message to an allocated PTI.

Editor's note:	Further contents of the PDU SESSION MODIFICATION REQUEST message are FFS.

The UE shall transport the PDU SESSION MODIFICATION REQUEST message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1, and the UE shall start timer Tk (see example in figure 9.4.3.2.1).

# Figure 9.4.3.2.1: UE-requested PDU session modification procedure

#### 9.4.3.3	UE-requested PDU session modification procedure accepting

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF accepts the request to modify the PDU session, the SMF shall perform the network-requested PDU session modification procedure as specified in subclause 9.4.4.

#### 9.4.3.4	UE-requested PDU session modification procedure rejecting

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF does not accepts the request to modify the PDU session, the SMF shall create a PDU SESSION MODIFICATION REJECT message.

The SMF shall set the SM cause IE of the PDU SESSION MODIFICATION REJECT message to indicate the reason for rejecting the PDU session modification.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note:	SM causes are FFS.

Editor's note:	Further contents of the PDU SESSION MODIFICATION REJECT message are FFS.

The SMF shall send the PDU SESSION MODIFICATION REJECT message.

Upon receipt of a PDU SESSION MODIFICATION REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE shall stop timer Tk and shall consider that the PDU session is not modified.

#### 9.4.3.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tk expired

Editor's note:	Further abnormal cases are FFS.

#### 9.4.3.6	Abnormal cases on the network side

Editor's note:	Abnormal cases are FFS.

### 9.4.4	Network-requested PDU session modification procedure

#### 9.4.4.1	General

The purpose of the network-requested PDU session modification procedure is to enable the network to modify a PDU session.

Editor's note:	The solution for situation when the UE is in CM-IDLE state and the AMF decides to invoke asynchronous type communication is FFS.

Editor's note:	It is FFS whether there are cases where the UE can validly reject the PDU SESSION MODIFICATION COMMAND message.

Editor's note:	It is FFS whether PDU session ID is included in the PDU SESSION MODIFICATION REQUEST message and PDU SESSION MODIFICATION REJECT message or not.

#### 9.4.4.2	Network-requested PDU session modification procedure initiation

In order to initiate the network-requested PDU session modification procedure, the SMF shall create a PDU SESSION MODIFICATION COMMAND message.

If the authorized QoS rules of the PDU session is modified, the SMF shall set the authorized QoS rules IE of the PDU SESSION MODIFICATION COMMAND message to the authorized QoS rules of the PDU session.

If the session-AMBR of the PDU session is modified, the SMF shall set the selected Session-AMBR IE of the PDU SESSION MODIFICATION COMMAND message to the session-AMBR of the PDU session.

If the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure, the SMF shall set the PTI IE of the PDU SESSION MODIFICATION COMMAND message to the PTI of the PDU SESSION MODIFICATION REQUEST message received as part of the UE-requested PDU session modification procedure.

Editor's note:	Further contents of the PDU SESSION MODIFICATION COMMAND message are FFS.

The SMF shall send the PDU SESSION MODIFICATION COMMAND message, and the SMF shall start timer Tm (see example in figure 9.4.4.2.1).

# Figure 9.4.4.2.1: Network-requested PDU session modification procedure

#### 9.4.4.3	Network-requested PDU session modification procedure accepting

Upon receipt of a PDU SESSION MODIFICATION COMMAND message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, if the UE accepts the PDU SESSION MODIFICATION COMMAND message, the UE considers the PDU session as modified and the UE shall create a PDU SESSION MODIFICATION ACCEPT message.

Editor's note:	Further contents of the PDU SESSION MODIFICATION ACCEPT message are FFS.

If the PDU SESSION MODIFICATION COMMAND message contains a PTI IE, the UE shall release the PTI indicated by the PTI IE and shall stop the timer Tk.

The UE shall transport the PDU SESSION MODIFICATION ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.

#### Upon receipt of a PDU SESSION MODIFICATION ACCEPT message, the SMF shall stop timer Tm and shall consider the PDU session as modified.

#### 9.4.4.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Tm expired

Editor's note:	Further abnormal cases are FFS.

#### 9.4.4.5	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS.

### 9.4.5	UE-requested PDU session release procedure

#### 9.4.5.1	General

The purpose of the UE-requested PDU session release procedure is to enable by the UE to request a release of a PDU session.

Editor’s note:	It is FFS whether the PDU Session ID is also included in the PDU SESSION RELEASE REQUEST message and PDU SESSION MODIFICATION REJECT message.

#### 9.4.5.2	UE-requested PDU session release procedure initiation

In order to initiate the UE-requested PDU session release procedure, the UE shall create an PDU SESSION RELEASE REQUEST message.

The UE shall set the PTI IE of the PDU SESSION RELEASE REQUEST message to an allocated PTI.

Editor's note:	Further contents of the PDU SESSION RELEASE REQUEST message are FFS.

The UE shall transport the PDU SESSION RELEASE REQUEST message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1, and the UE shall start timer Tz (see example in figure 9.4.5.2.1).

# Figure 9.4.5.2.1: UE-requested PDU session release procedure

#### 9.4.5.3	UE-requested PDU session release procedure accepting

Upon receipt of an PDU SESSION RELEASE REQUEST message and an PDU session ID, if the SMF accepts the request to release the PDU session, the SMF shall perform the network-requested PDU session release procedure as specified in subclause 9.4.6.

#### 9.4.5.4	UE-requested PDU session release procedure rejecting

Upon receipt of an PDU SESSION RELEASE REQUEST message, if the SMF does not accept the request to release the PDU session, the SMF shall create an PDU SESSION RELEASE REJECT message.

The SMF shall set the SM cause IE of the PDU SESSION RELEASE REJECT message to indicate the reason for rejecting the PDU session release.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note:	SM causes are FFS.

Editor's note:	Further contents of the PDU SESSION RELEASE REJECT message are FFS.

The SMF shall send the PDU SESSION RELEASE REJECT message.

Upon receipt of an PDU SESSION RELEASE REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE shall stop timer Tz and shall consider that the PDU session is not released.

#### 9.4.5.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tz expired

Editors' note:	Further abnormal cases are FFS.

#### 9.4.5.6	Abnormal cases on the network side

Editors' note:	Abnormal cases are FFS.

### 9.4.6	Network-requested PDU session release procedure

#### 9.4.6.1	General

The purpose of the network-requested PDU session release procedure is to enable the network to release a PDU session.

Editor’s note:	It is FFS whether the PDU Session ID is also included in the PDU SESSION RELEASE COMMAND message and PDU SESSION MODIFICATION ACCEPT message.

#### 9.4.6.2	Network-requested PDU session release procedure initiation

In order to initiate the network-requested PDU session release procedure, the SMF shall create an PDU SESSION RELEASE COMMAND message.

The SMF shall set the SM cause IE of the PDU SESSION RELEASE COMMAND message to indicate the reason for releasing the PDU session.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note:SM causes are FFS.

Editor's note:	Further contents of the PDU SESSION RELEASE COMMAND message are FFS.

If the network-requested PDU session release procedure is triggered by a UE-requested PDU session release procedure, the SMF shall set the PTI IE of the PDU SESSION RELEASE COMMAND message to the PTI of the PDU SESSION RELEASE REQUEST message received as part of the UE-requested PDU session release procedure.

The SMF shall send:

a)	the PDU SESSION RELEASE COMMAND message; and

b)	if the SMF allows the AMF to skip sending of the N1 SM container to the UE, the N1 SM delivery skip allowed indication;

towards the AMF, and the SMF shall start timer Ty (see example in figure 9.4.6.2.1).

# Figure 9.4.6.2.1: Network-requested PDU session release procedure

#### 9.4.6.3	Network-requested PDU session release procedure accepting

Upon receipt of an PDU SESSION RELEASE COMMAND message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.3, the UE considers the PDU session as released and the UE shall create an PDU SESSION RELEASE ACCEPT message.

If the PDU SESSION RELEASE COMMAND message contains a PTI IE, the UE shall release the PTI indicated by the PTI IE and shall stop the timer Tz.

Editor's note:	Further contents of the PDU SESSION RELEASE ACCEPT message are FFS.

The UE shall transport the PDU SESSION RELEASE ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.4.1.1.1.1.

Upon receipt of an PDU SESSION RELEASE ACCEPT message, the SMF shall stop timer Ty and shall consider the PDU session as released.

#### 9.4.6.4	N1 SM delivery skipped

If the PDU SESSION RELEASE COMMAND message was sent along the N1 SM delivery skip allowed indication towards AMF, then upon receipt of an indication that N1 SM delivery was skipped, the SMF shall stop timer Ty and shall consider the PDU session as released.

#### 9.4.6.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Ty expired

Editors' note:	Further abnormal cases are FFS.

#### 9.4.6.6	Abnormal cases in the UE

Editors' note:	Abnormal cases are FFS.

## 9.5	5GS session management coding

### 9.5.1	PDU session establishment request

#### 9.5.1.1	Message definition

The PDU SESSION ESTABLISHMENT REQUEST message is sent by the UE to the network to initiate establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the SM PDU SESSION ESTABLISHMENT REQUEST message is FFS. At least the following is needed: the PDU session type IE (optional), the SSC mode IE (optional), the extended protocol configuration options (optional), SM PDU DN request container (optional).

### 9.5.2	PDU session establishment accept

#### 9.5.2.1	Message definition

The PDU SESSION ESTABLISHMENT ACCEPT message is sent by the network to the UE is sent in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates successful establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT ACCEPT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the PDU SESSION ESTABLISHMENT ACCEPT message is FFS. At least the following is needed: the authorized QoS rules IE (mandatory), the selected SSC mode IE (mandatory), the selected S-NSSAI (mandatory), the PDU session type (conditional on not inclusion by the UE in the PDU SESSION ESTABLISHMENT REQUEST message), PDU address (conditional on PDU session type being IPv4), the extended protocol configuration options (optional), the selected DNN (mandatory). session-AMBR (mandatory).

### 9.5.3	 PDU session establishment reject

#### 9.5.3.1	Message definition

The PDU SESSION ESTABLISHMENT REJECT message is sent by the network to the UE is sent in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates unsuccessful establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT REJECT

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION ESTABLISHMENT REJECT message is FFS. At least the following is needed: the SM cause (mandatory), the extended protocol configuration options (optional).

### 9.5.4	PDU session authentication request

#### 9.5.4.1	Message definition

The PDU SESSION AUTHENTICATION REQUEST message is sent by the network to the UE for authentication of the UE establishing the PDU session.

Message type:	PDU SESSION AUTHENTICATION REQUEST

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION AUTHENTICATION REQUEST message is FFS. At least the following is needed: the authentication information (mandatory), extended protocol configuration options (optional).

### 9.5.5	PDU session authentication accept

#### 9.5.5.1	Message definition

The PDU SESSION AUTHENTICATION ACCEPT message is sent by the UE to the network in response to the PDU SESSION AUTHENTICATION REQUEST message and indicates acceptance of the PDU SESSION AUTHENTICATION REQUEST message.

Message type:	PDU SESSION AUTHENTICATION ACCEPT

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION AUTHENTICATION ACCEPT message is FFS. At least the following is needed: the authentication information (mandatory), extended protocol configuration options (optional).

### 9.5.6	PDU session authentication reject

#### 9.5.6.1	Message definition

The PDU SESSION AUTHENTICATION REJECT message is sent by the UE to the network in response to the PDU SESSION AUTHENTICATION REQUEST message and indicates rejection of the PDU SESSION AUTHENTICATION REQUEST message.

Message type:	PDU SESSION AUTHENTICATION REJECT

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION AUTHENTICATION REJECT message is FFS. At least the following is needed: SM cause (mandatory), extended protocol configuration options (optional).

### 9.5.7	PDU session modification request

#### 9.5.7.1	Message definition

The PDU SESSION MODIFICATION REQUEST message is sent by the UE to the network to request a modification of a PDU session.

Message type:	PDU SESSION MODIFICATION REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION MODIFICATION REQUEST message is FFS. At least the following is needed: PTI (mandatory), the extended protocol configuration options (optional).

### 9.5.8	PDU session modification reject

#### 9.5.8.1	Message definition

The PDU SESSION MODIFICATION REJECT message is sent by the network to the UE to indicate rejection of the PDU SESSION MODIFICATION REQUEST.

Message type:	PDU SESSION MODIFICATION REJECT

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION MODIFICATION REJECT message is FFS. At least the following is needed: SM cause (mandatory), the extended protocol configuration options (optional).

### 9.5.9	PDU session modification command

#### 9.5.9.1	Message definition

The PDU SESSION MODIFICATION COMMAND message is sent by the network to the UE to indicate a modification of a PDU session.

Message type:	PDU SESSION MODIFICATION COMMAND

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION MODIFICATION COMMAND message is FFS. At least the following is needed: PTI (conditional), authorized QoS rules (conditional on change of the QoS rules), Session-AMBR (conditional on change of the session-AMBR), the extended protocol configuration options (optional).

### 9.5.10	PDU session modification accept

#### 9.5.10.1	Message definition

The PDU SESSION MODIFICATION ACCEPT message is sent by the UE to the network in response to the PDU SESSION MODIFICATION COMMAND message and indicates an acceptance of the PDU SESSION MODIFICATION COMMAND message.

Message type:	PDU SESSION MODIFICATION ACCEPT

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION MODIFICATION ACCEPT message is FFS. At least the following is needed: the extended protocol configuration options (optional).

### 9.5.11	PDU session release request

#### 9.5.11.1	Message definition

The PDU SESSION RELEASE REQUEST message is sent by the UE to the network to request a release of a PDU session.

Message type:	PDU SESSION RELEASE REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION RELEASE REQUEST message is FFS. At least the following is needed: PTI (mandatory), the extended protocol configuration options (optional).

### 9.5.12	PDU session release reject

#### 9.5.12.1	Message definition

The PDU SESSION RELEASE REJECT message is sent by the network to the UE to indicate rejection of request a release of a PDU session.

Message type:	PDU SESSION RELEASE REJECT

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION RELEASE REJECT message is FFS. At least the following is needed: SM cause (mandatory), the extended protocol configuration options (optional).

### 9.5.13	PDU session release command

#### 9.5.13.1	Message definition

The PDU SESSION RELEASE COMMAND message is sent by the network to the UE to indicate a release of a PDU session.

Message type:	PDU SESSION RELEASE COMMAND

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION RELEASE COMMAND message is FFS. At least the following is needed: the cause IE (mandatory) and PTI (conditional), the extended protocol configuration options (optional).

### 9.5.14	PDU session release accept

#### 9.5.14.1	Message definition

The PDU SESSION RELEASE ACCEPT message is sent by the UE to the network in response to the PDU SESSION RELEASE COMMAND message and indicates an acceptance of a release of the PDU session.

Message type:	PDU SESSION RELEASE ACCEPT

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the PDU SESSION RELEASE ACCEPT message is FFS. At least the following is needed: the extended protocol configuration options (optional).

#### 9.6	Timers of 5GS session management

Timers of 5GS session management are shown in table 9.6.1 and table 9.6.2.

# Table 9.6.1: Timers of 5GS session management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Tx | TBD | TBD | Transmission of PDU SESSION ESTABLISHMENT REQUEST message. | PDU SESSION ESTABLISHMENT ACCEPT message received or PDU SESSION ESTABLISHMENT REJECT message received. | TBD |
| Tm | TBD | TBD | Transmission of PDU SESSION AUTHENTICATION REQUEST message. | PDU SESSION AUTHENTICATION ACCEPT message received or PDU SESSION AUTHENTICATION REJECT message received. | TBD |
| Tk | TBD | TBD | Transmission of PDU SESSION MODIFICATION REQUEST message. | PDU SESSION MODIFICATION COMMAND message with the same PTI is received or PDU SESSION MODIFICATION REJECT message received. | TBD. |
| Tz | TBD | TBD | Transmission of PDU SESSION RELEASE REQUEST message. | PDU SESSION RELEASE COMMAND message with the same PTI is received or PDU SESSION RELEASE REJECT message received. | TBD |

# Table 9.6.2: Timers of 5GS session management – SMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Tm | TBD | TBD | Transmission of PDU SESSION MODIFICATION COMMAND message. | PDU SESSION MODIFICATION ACCEPT message received or PDU SESSION MODIFICATION REJECT message received. | TBD |
| Ty | TBD | TBD | Transmission of PDU SESSION RELEASE COMMAND message. | PDU SESSION RELEASE ACCEPT message received or N1 SM delivery skipped indication received. | TBD |

### 10	General message format and information elements coding

## 10.1	Overview

Within the protocols defined in the present document, every 5G NAS message, is a standard L3 message as defined in 3GPP TS 24.007 [7]. This means that the message consists of the following parts:

Editor's note:	The required updates on a standard L3 message definition in 3GPP TS 24.007 [7] are FFS.

1)	if the message is a plain 5G NAS message:

a)	extended protocol discriminator;

b)	security header type;

c)	procedure transaction identity;

d)	message type;

e)	other information elements, as required.

2)	if the message is a security protected 5G NAS message:

Editor's note:	The message parts of a security protected 5G NAS message are FFS.

The organization of a plain 5G NAS message is illustrated in the example shown in figure 10.1.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Spare half octet |  |  |  | Security header type |  |  |  | octet 2 |
| Procedure transaction identity |  |  |  |  |  |  |  | octet 3 |
| Message type |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet 5 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

# Figure 10.1.1: General message organization example for a plain 5G NAS message

Editor's note:	The organization of a security protected 5G NAS message is FFS.

Unless specified otherwise in the message descriptions of subclause 8 and subclause 9, a particular information element shall not be present more than once in a given message.

When a field extends over more than one octet, the order of bit values progressively decreases as the octet number increases. The least significant bit of the field is represented by the lowest numbered bit of the highest numbered octet of the field.

#### 10.2	Extended protocol discriminator

Bits 1 to 8 of the first octet of every 5GMM message contain the Extended protocol discriminator (EPD) IE. The EPD and its use are defined in 3GPP TS 24.007 [7]. Bits 1 to 4 of the extended protocol discriminator in the header of a plain 5GMM message is encoded as "1110".

Editor's note:	The definition of the extended protocol discriminator in 3GPP TS 24.007 [7] is FFS.

Editor's note:	Whether the Extended protocol discriminator IE is included in the header of 5GSM message is FFS.

#### 10.3	Security header type

Bits 1 to 4 of the second octet of every 5GMM message contain the Security header type IE. This IE includes control information related to the security protection of a 5GMM message. The total size of the Security header type IE is 4 bits.

Editor's note:	The values of the Security header type are FFS.

Editor's note:	Whether the PDU session identity IE is included in the header of every 5GSM message is FFS.

#### 10.4	Spare half octet

Bits 5 to 8 of the second octet of every 5G NAS message contains the spare half octet which is filled with spare bits set to zero.

# 11	Access to the 5G core network via non-3GPP access networks

## 11.1	General

In this release, the 5GS supports untrusted non-3GPP accesses and does not support trusted non-3GPP accesses (see 3GPP TS 23.501 [5]).

Access to the 5GCN via non-3GPP access networks shall be supported by the non-3GPP interworking function (N3IWF). Between the UE and the N3IWF, an IPsec SA shall be established. During the IPsec SA establishment, the UE shall be authenticated by and registered to the 5G core network. The N3IWF interfaces to 5G core network via N2 and N3 reference points.

Editor's note:	How to transmit a registration request message is FFS.

If the UE is authenticated by the 5G core network, secure NAS message exchange between the UE and the 5G core network shall be supported using the N1 reference point over the N3IWF.

When a UE is registered via 3GPP and non-3GPP access networks, two N1 NAS signalling connections shall exist for the UE.

A UE simultaneously registered to a single PLMN over 3GPP and non-3GPP access networks, shall be served by a single AMF. In this case, two N1 NAS signalling connections shall exist for the AMF. Otherwise a UE registered to two different PLMNs over 3GPP and non-3GPP access networks, shall be served by two AMFs in two different PLMNs. In this case, single N1 NAS signalling connections shall exist for each of the AMFs.

During the IPsec SA establishment between the UE and the N3IWF, the N3IWF retrieves the NAI of the UE and the NAI is used to identify the UE in the 5G core network. If the UE is registered to the 5G core network via non-3GPP access network, the UE shall be assigned a temporary ID from the 5G core network. If a UE is registered to a single PLMN over 3GPP and non-3GPP access networks simultaneously, the AMF shall ensure that the same temporary ID is assigned to the UE via both access networks. Otherwise if a UE is registered to two different PLMNs over 3GPP and non-3GPP access networks, each of the AMFs shall allocate a temporary ID independently to the UE.

## 11.2	Protocol stack

Editor's note:	This subclause will specify the control-plane and user-plane protocol stacks of untrusted non-3GPP access to 5GCN.

## 11.3	NAS over non-3GPP access

### 11.3.1	General

From the UE's NAS perspective, the N1 interface is indistiguishable over the 3GPP access or the non-3GPP access in so much as the peer-to-peer signalling procedures and messages defined for UE NAS to 5GCN NAS over 3GPP access will be used for same purposes for UE NAS to 5GCN NAS over non-3GPP access. The 5GS NAS state machines of both 3GPP access or non-3GPP access should likewise have minimum differences. Maximum reuse should be the objective with any deviations or exceptions adopted if without such deviations or exceptions, signalling procedures and intended services will fail or malfunction, or service or archtitectural (or both) requirments cannot be met.

### 11.3.2	Mobility management over non-3GPP access

The mobility management procedures defined for N1 over 3GPP access will be re-used for N1 over non-3GPP access with the following exceptions:

-	the status of the UE's non-3GPP 5GMM instance and the status of the UE's 3GPP 5GMM instance are independent and can be different.

-	single registration mode does not apply for 5GMM over non-3GPP access.

Editor's note:	It is FFS if dual registration mode applies for non-3GPP access.

-	there will be one instance of 5GMM for 3GPP access and one instance of 5GMM for non-3GPP access. The RPLMN over non-3GPP access can be different from the RPLMN over 3GPP access. The MCC of the RPLMN over 3GPP access and the MCC of the RPLMN over the non-3GPP access can also be different.

-	as over non-3GPP access the 5GS operates one common registration area for an entire PLMN, list management of registration areas and resultant registration updating due to registration area change with the registered PLMN is not required. Registration updating at change of PLMN is still required.

Editor's note:	The support of periodic registration is FFS.

-	the 5GMM over non-3GPP access considers that the N1 NAS signalling connection is established once the IKE_SA association is established succcessfully.

Editor's note:	Further exceptions are possible and are FFS.

### 11.3.3	Session management over non-3GPP access

The session management procedures defined for N1 over 3GPP access will be re-used for N1 over non-3GPP access with the following exceptions:

-	a PDU session established over non-3GPP access cannot be access transferred for use upon an inter-system change to EPS.

Editor's note:	Further exceptions are possible and are FFS.

## 11.4	Registration over non-3GPP access

### 11.4.1	General

The UE registers separately for 3GPP access and non-3GPP access, respectively.

The UE shall not perform periodic registration update over a non-3GPP access.

Editor's note:	It is FFS whether a registration timer is used for the registration over non-3GPP access

The UE cannot be paged over a non-3GPP access.

#### 11.4.2	Registration over non-3GPP access

When the UE is registered with a PLMN over a non-3GPP access, the AMF and the UE maintain:

-	Registration state and state machine over non-3GPP access;

-	Security context;

-	Temporary UE identity; and

-	Registration area for non-3GPP access, which is associated with a fixed well-known N3GPP TAI.

Editor’s note:	What information the N3IWF maintains for a registered UE is FFS.

### 11.4.3	Registration over non-3GPP access and 3GPP access

When the UE is registered with a PLMN over a non-3GPP access and over a 3GPP access, the AMF and the UE maintain:

-	Separate registration states and separate instances of state machines over 3GPP access and non-3GPP access, respectively;

Editor’s note:	The state machine over non-3GPP access is FFS.

-	Separate registration areas; and

-	Same security context and Temporary UE identity, only if the UE is registered with the same PLMN over both accesses. Otherwise, the security contexts and the temporary UE identity shall be separate.

Editor’s note:	To what extent the security context is the same when the UE is registered with the same PLMN over both accesses is FFS.

When the UE initiates registration over a non-3GPP access to a PLMN for which it has a valid temporary UE identity, the UE shall use this temporary UE identity during the registration. When the UE is currently registered with the same PLMN over a 3GPP access, the temporary UE identity is considered as valid.

Editor’s note:	Other conditions for considering a temporary UE identity as valid are FFS.

#### 11.4.4	De-registration over non-3GPP access

The UE and the AMF may initiate de-registration for a non-3GPP access by:

-	Initiating a de-registration request over a non-3GPP access; or

-	Initiating a de-registration request over a 3GPP access with an indication that it applies to all accesses.

When the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

## 11.5	Selection of N3IWF

Editor's note:	How the UE selects the N3IWF is FFS.

## 11.6	Procedures between UE and N3IWF

Editor's note:	This subclause specifies the IKEv2 procedures between the UE and N3IWF due to UE registration/de-registration, PDU session setup, PDU session modification and PDU session release.

### 11.7.1	IKE SA creation procedure for UE registration via untrusted non-3GPP access

Editor's note:	This subclause specifies IKE SA creation procedure for UE registration via untrusted non-3GPP access.

### 11.7.2	Child SA creation procedure for UE-requested PDU session establishment

Editor's note:	This subclause specifies child SA creation procedure for PDU session setup via untrusted non-3GPP access.

### 11.7.3	Child SA change procedure for PDU session modification

Editor's note:	This subclause specifies child SA change procedure for PDU session modification via untrusted non-3GPP access.

### 11.7.4	Child SA deletion procedure for PDU session release

Editor's note:	This subclause specifies child SA deletion procedure for PDU session release via untrusted non-3GPP access.

### 11.7.5	IKE SA deletion for UE deregistration via untrusted non-3GPP access

Editor's note:	This subclause specifies IKE SA deletion procedure for UE deregistration via untrusted non-3GPP access.

## 11.8	IKEv2 payload extensions

Editor's note:	This subclause defines the coding of IKEv2 payload for untrusted non-3GPP access to 5GCN.

# 12	Interworking with E-UTRAN connected to EPC

## 12.1	General

In order to interwork with E-UTRAN connected to EPC, the UE supporting both S1 mode and N1 mode can operate in single-registration mode or dual-registration mode (see 3GPP TS 23.501 [5]).

Editor’s note:	The condition(s) for which the UE operates in the single-registration mode and/or dual-registration mode is FFS.

## 12.2	Single-registration mode

### 12.2.1	General

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode in the single-registration mode, shall maintain one common registration for 5GMM and EMM.

### 12.2.2	Coordination between 5GMM and EMM

A UE that is not registered shall be in state EMM-DEREGISTERED and in state 5GMM-DEREGISTERED.

In N1 mode, upon successful registration procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NORMAL-SERVICE and EMM-REGISTERED.NO-CELL-AVAILABLE.

In S1 mode, upon successful attach or tracking area updating procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NO-CELL-AVAILABLE and EMM-REGISTERED.NOMRAL-SERVICE.

Editor’s note:	The substate 5GMM-REGISTERED.NORMAL-SERVICE is comparable to the substate EMM-REGISTERED.NORMAL-SERVICE. Detailed description is FFS.

Editor’s note:	The substate 5GMM-REGISTERED.NO-CELL-AVAILABLE is comparable to the substate EMM-REGISTERED.NO-CELL-AVAILABLE. Detailed description is FFS.

## 12.3	Dual-registration mode

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode in the dual-registration mode, shall maintain independent registrations for 5GMM and EMM independently and coordination between 5GMM and EMM is not needed.

# 13	5G System core network impact on services, network functions and capabilities

This clause will describe the impacts on services, network functions and capabilities, e.g., specific services (PWS, SMS over NAS, emergency services, LCS, MPS), access control, security, IMS, QoS, network sharing, congestion and overload control, charging.

## 13.1	Specific services

### 13.1.1	Public warning system

This sub-clause will describe the necessary work to support PWS (see 3GPP TS 23.501 [5] subclause 5.16.1 and 3GPP TS 23.502 [6] subclause 4.13.2).

### 13.1.2	SMS over NAS

This sub-clause will describe the necessary work to support SMS over NAS (see 3GPP TS 23.501 [5] subclause 5.16.2 and 3GPP TS 23.502 [6] subclause 4.13.3).

### 13.1.3	Emergency services

#### 13.1.3.1	Indication of support of emergency eervices

During the registration procedure the AMF includes the emergency PDU session service support indicator in the REGISTRATION ACCEPT message (see subclause 8.5.2).

Editor’s note:	The emergency PDU session service support indicator needs to be aligned with the stage 2 as soon as procedural requirements are in place.

Editor’s note:	The emergency service support indicator needs to be added in the subclause 8.5.2.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency service support indicator is included.

The received emergency PDU session service support indicator shall be provided to the upper layers in the UE.

Editor’s note:	The use of the emergency PDU session service support indicator by upper layers is FFS.

#### 13.1.3.2	Handling of emergency service for the UE in limited service state

Editor's note:	This subclause describes how to provide emergency bearer services to the UE in limited services state, especially how the UE in limited service state registers to the 5GC

#### 13.1.3.3	Emergency PDU session setup

Editor's note:	This subclause describes the procedure of PDU session setup.

#### 13.1.3.4	Handling of local emergency numbers

The following mechanism of handling of local emergency numbers applies when the UE accesses the 5GCN via 3GPP access and non-3GPP access.

Editor’s note:	The handling of the case where the UE registers to different PLMNs via 3GPP access and non-3GPP access simultaneously and receive the emergency number list from different PLMNs is FFS.

The local emergency numbers can be downloaded by the network to the UE at successful completion of the registration procedure. The UE shall store and use the local emergency numbers received from the network in addition to the emergency numbers stored on the UICC or user equipment to detect that the number dialled is an emergency number.

The network may send local emergency numbers in the REGISTRATION ACCEPT message (see subclause 8.5.2).

Editor’s note:	The emergency number list needs to be added in the subclause 8.5.2.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency number list is included.

The local emergency numbers received from the network are valid only in networks in the same country as where the UE received them.

#### 13.1.3.5	Security procedures for handling emergency services

Editor's note:	This subclause describes the difference on the security aspect between emergency services procedure and normal procedure.

### 13.1.4	Location services

This sub-clause will describe the necessary work to support LCS (see 3GPP TS 23.501 [5] Annex B and 3GPP TS 23.502 [6] subclause 4.13.7).

### 13.1.5	Multimedia priority services

This sub-clause will describe the necessary work to support MPS (see 3GPP TS 23.501 [5] subclause 5.16.5 and 3GPP TS 23.502 [6] subclause 4.13.6).

## 13.2	Access control

This sub-clause will describe the CT1 relevant access control aspects to support 5GS.

## 13.3	Security

This sub-clause will describe the CT1 relevant security aspects to support 5GS.

## 13.4	Quality of service (QoS)

### 13.4.1	General

#### 13.4.1.1	QoS rules

The NAS protocol enables the network to provide the UE with QoS rules associated with a PDU session.

The network can provide the UE with one or more QoS rules associated with a PDU session at the PDU session establishment or at QoS flow establishment.

Each QoS rule contains:

a)	a QoS rule identifier (QRI);

b)	a QoS flow identity (QFI);

c)	optionally a QoS flow template containing a set of packet filters and corresponding precedence values;

d)	whether reflective QoS function is activated using control plane; and

e)	if the reflective QoS function is activated using control plane, the precedence value for a derived QoS rule.

Additionally, the network provides the UE with the following QoS parameters for each provided QoS rule:

a)	if the flow is a GBR QoS flow:

1)	Guaranteed flow bit rate (GFBR) for UL;

2)	Guaranteed flow bit rate (GFBR) for DL;

3) Maximum flow bit rate (MFBR) for UL; and

4)	Maximum flow bit rate (MFBR) for DL.

Editors' note:	To provide the 5G QoS indicator (5QI) to the UE is FFS.

Editor's note:	The length of the 5QI, QFI and QRI is FFS.

#### 13.4.1.2	Session-AMBR

The NAS protocol enables the network to provide the UE with the session-AMBR associated with a PDU session.

### 13.4.1.3	QoS flow template

A QoS flow template consists of one or more packet filters, each associated with a packet filter identifier and an evaluation precedence index. The evaluation precedence index shall be unique among all packet filters associated with a given PDU session.

For PDU sessions of type IP, the format of the packet filters is the same as that specified in 3GPP TS 24.008 [8] subclause 10.5.6.12.

Editor's note:	Whether packet filters are applicable to PDU sessions of type Ethernet, and if so what the format of the packet filters should be, are FFS.

Editor's note:	Whether packet filters are applicable to PDU sessions of type unstructured, and if so what the format of the packet filters should be, are FFS.

### 13.4.1.4	Packet matching

The UE shall associate each UL user-data packet in a PDU session with a QoS flow by performing the following steps in order:

-	the UE evaluates for a match between the UL user-data packet and UL packet filter which has the lowest evaluation precedence index amongst all packet filters in the QoS rules stored in the UE for the PDU session; and

-	if no match is found, the UE proceeds with the evaluation of UL packet filters in increasing order of their evaluation precedence index value.

The steps above shall be executed until a match is found or all UL packet filters have been evaluated. Then the UE shall:

-	if a match is found, pass the QFI value associated with the QoS rule containing the packet filter for which there was a match along with the UL user-data packet to the lower layers for transmission; and

NOTE:	Marking of the UL user-data packet with the QFI is performed by the lower layers.

-	if no match is found, discard the UL user-data packet.

### 11.4.1.5	Reflective QoS

### 11.4.1.5.1	Reflective QoS activation via the control plane

To activate reflective QoS via the control plane, the network sends a QoS rule to the UE with an RQI indicating that reflective QoS for this QoS flow is activated, and an evaluation precedence index to be associated with the packet filters self-constructed by the UE as specified in subclause 11.4.1.5.3.

#### 11.4.1.5.2	Reflective QoS activation via the user plane

To activate reflective QoS via the user plane, the network includes an RQI in the encapsulation header added to the DL end-to-end user-data packet indicating that reflective QoS is activated.

#### 13.4.1.5.3	QoS flow templated derived by the UE for reflective QoS

When:

-	the UE receives a DL user-data packet marked with a QFI associated with a QoS rule for which reflective QoS is activated; or

-	the UE is notified by the lower layers that reflective QoS is activated for a DL user-data packet;

the UE derives the QoS flow template to be applied to UL user-data packets based on DL user-data packets as follows:

-	the QoS flow template contains one or more UL packet filters, each associated with:

1)	the evaluation precedence index signalled to the UE by the network if reflective QoS was activated via the control plane; or

2)	the standardized evaluation precedence index value mapping to the QFI of the associated QoS flow if reflective QoS was activated via the user plane; and

Editor's note:	The mapping of standardized evaluation precedence index to QFI is FFS.

-	the UL packet filters are constructed as follows:

1)	if the QoS flow belongs to a PDU session of type IP:

i)	when the UE receives a DL user-data packet, the UE shall check whether this packet maps to an existing UL packet filter in the QoS flow template; and

ii)	if no matching UL packet filter in the QoS flow template is found, the UE shall create a new UL packet filter with the following components:

a)	for IPv4:

I)	an IPv4 remote address component set to the source IP address of the received DL packet;

II)	an IPv4 local address component set to the destination IP address of the received DL packet; and

III)	a Protocol identifier/Next header type component set to the value of the last received protocol type field;

b)	for IPv6:

I)	an IPv6 remote address set to the source IP address of the received DL packet;

II)	an IPv6 local address/prefix length component set to the destination IP address of the received DL packet; and

III)	a Protocol identifier/Next header type component set to the value of the last received next header field;

c)	a Single local port type component set to value of the destination port in the received DL packet; and

d)	a Single remote port type component set to the value of the source port in the received DL packet;

Editor's note:	The need to associate a time stamp with each UL packet filter is FFS.

2) if the QoS flow belongs to a PDU session of type Ethernet:

Editor's note:	How the UE constructs UL packet filters for a PDU session of type Ethernet when reflective QoS is activated is FFS.

3)	if the QoS flow belongs to a PDU session of type unstructured:

Editor's note:	How the UE constructs UL packet filters for a PDU session of type unstructured when reflective QoS is activated is FFS.

#### 13.4.1.5.4	Reflective QoS deactivation

Editor's note:	The procedure to deactive reflective QoS is FFS.

### 13.4.2	QoS coding

#### 13.4.2.1	QoS rules coding

Editor's note:	The exact coding of the QoS rules is FFS.

#### 13.4.2.2	Session-AMBR coding

Editor's note:	The exact coding of the session-AMBR is FFS.

## 13.5	IM CN subsystem

### 13.5.1	IMS voice over PS Session supported Indication

The network informs the UE about the support of IMS voice over PS session In a UE, the IMS voice over PS session indicator (IMS VoPS) shall be provided to the upper layers. When initiating a voice call, the upper layers take the IMS voice over PS session support indicator into account.

Editor’s note:	The use of the IMS voice over PS session by upper layers is FFS.

Editor's note:	It is FFS if there are other methods indicating that the UE can expect a successful IMS voice over PS session.

During the registration procedure the AMF includes the IMS voice over PS session supported indicator in the REGISTRATION ACCEPT message (see subclause 8.5.2).

Editor’s note:	The emergency PDU session service support indicator needs to be aligned with the stage 2 as soon as procedural requirements are in place.

Editor’s note:	The emergency service support indicator needs to be added in the subclause 8.5.2.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency service support indicator is included.

When indicating support of IMS voice over PS session to the UE, the AMF should take into account the following factors:

1)	the local policy of the PLMN to which the AMF belongs;

2)	the capabilities of the network;

3)	the capabilities of the UE; and

4)	the roaming agreement between the VPLMN and the HPLMN.

### 13.5.2	P-CSCF address delivery

When the UE requests to establish a PDU sesssion, which can be used for IMS services, the UE includes in the ePCO IE contained in the PDU SESSION ESTABLISHMENT REQUEST message (see subclause 9.5.3) the request for P-CSCF IPv4 address or P-CSCF IPv6 address or both. The AMF forwards the ePCO IE transparently via N11 interface towards the SMF.

When receiving an 5GSM request message containing a P-CSCF address request on the N11 interface, the SMF includes the locally configured P-CSCF address(es) in the ePCO IE of the corresponding 5GSM response message.

## 13.6	Network sharing

This sub-clause will describe the CT1 relevant network sharing aspects to support 5GS (see 3GPP TS 23.501 [5] subclause 5.18).

## 13.7	Congestion and overload control

This sub-clause will describe the CT1 relevant congestion and overload control aspects to support 5GS (see 3GPP TS 23.501 [5] subclause 5.19).

## 13.8	Charging

This sub-clause will describe the CT1 relevant charging aspects to support 5GS.

# 14	Network slicing

## 14.1	General

The 5GS supports network slicing as described in 3GPP TS 23.501 [5]. Within a PLMN, a network slice is identified by an S-NSSAI, which is comprised of a slice/service type (SST) and a slice differentiator (SD). Inclusion of an SD in an S-NSSAI is optional.

Editor's note:	Terms on network slicing can be revisited.

## 14.2	Mobility management aspects

### 14.2.1	General

A set of one or more S-NSSAIs is called the NSSAI. There are multiple types of the NSSAI (see 3GPP TS 23.501 [5]):

-	configured NSSAI;

-	allowed NSSAI; and

-	requested NSSAI.

The HPLMN may configure a UE with the configured NSSAI per PLMN.

The AMF may provide the UE with the allowed NSSAI for the PLMN. See subclause 8.4.2.1 for further details.

Upon registration to a PLMN, the UE shall send to the AMF the requested NSSAI including one or more S-NSSAIs corresponding to the slice(s) to which the UE wishes to register if the UE has a configured NSSAI and/or an allowed NSSAI for the PLMN. If the UE has neither a configured NSSAI nor an allowed NSSAI for a PLMN, the UE does not send a requested NSSAI when requesting registration towards the PLMN. See subclause 8.4.2.1for further details.

The set of network slice(s) for a UE can be changed at any time while the UE is registered to a PLMN, and may be initiated by the network, or the UE. In this case, the allowed NSSAI and associated registration area may be changed during the registration procedure. The network may notify the UE of the change of the supported network slice(s) in order to trigger the registration procedure. Change in the allowed NSSAI may lead to AMF relocation subject to operator policy.

Editor's note:	The subclause for the notification procedure will be included in the TR and the subclause will be referred.

### 14.2.2	NSSAI storage

Configured NSSAI as well as allowed NSSAI is a set composed of one or more S-NSSAIs associated with a PLMN identity. There shall be no duplicated PLMN identities in each of the list of configured NSSAI(s) and the list of allowed NSSAI(s).

If available, the configured NSSAI(s) and/or allowed NSSAI(s) shall be stored in a non-volatile memory in the ME together with the SUPI from the USIM per PLMN. The configured NSSAI(s) and/or allowed NSSAI(s) can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME; else the UE shall delete them.

## 14.3	Session management aspects

In order for data transmission in a network slice, the UE may request establishment of a PDU session in a network slice towards a data network (DN) which is associated with an S-NSSAI and a data network name (DNN). See subclause 9.4.1 for further details.

The network may provision the UE with NSSP. The UE selects a PDU session taking into account NSSP (if provisioned by the network) and other conditions in the UE (if any).

Editor's note:	Whether the UE configuration update procedure can be used to provision NSSP is FFS.

# 15	Conclusions and recommendations

This clause will contain the conclusions and recommendations of the study including the organization of normative specifications for 5G Phase 1.

Annex A:
Impacts to specifications

This clause will describe the impacts to existing specifications and potential new specifications.

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | CT1#103 | C1-171624 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2017-04 | CT1#103 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-171512, C1-171516, C1-171623, C1-171632, C1-171634, C1-171638, C1-171959, C1-171960, C1-171662, C1-171971, C1-171973, C1-171974, C1-171975. | 0.1.0 |
| 2017-05 | CT1#104 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-172298, C1-172410, C1-172411, C1-172412, C1-172628, C1-172643, C1-172416, C1-172427, C1-172429, C1-172430, C1-172431, C1-172432, C1-172630, C1-172632, C1-172654, C1-172655, C1-172769, C1-172770, C1-172771, C1-172772, C1-172773, C1-172780, C1-172786, C1-172633, C1-172634, C1-172635, C1-172636, C1-172637, C1-172645, C1-172646, C1-172647, C1-172656, C1-172775, C1-172472, C1-172777, C1-172640, C1-172659, C1-172661, C1-172662, C1-172776, C1-172778, C1-172779 | 0.2.0 |