| 3GPP TR 26.996 V0.0.1 (2024-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Immersive Audio for Split Rendering Scenarios; Performance characterization (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Editor’s note: This document represents the initial draft of this TR by the rapporteur. More edits are expected prior to and during the SA4#128 meeting. It is expected that company contributions providing further performance evaluations results for track-A solutions and for track-B need to be integrated during the SA4#128 meeting.

An essential architectural characteristic of XR clients is the reliance on a functional split between a set of composite pre-renderers that are implemented as parts of a presentation engine and a set of post-rendering operations implemented on an End Device prior to final output. Split rendering may be a necessity if the End Device is power constrained or limited in computational power. However, split rendering is not precluded from other End Devices that do not have such constraints. A discussion of relevant split rendering scenarios is provided in TR 26.865 [2], together with general design guidelines for immersive audio split rendering systems and specific design constraints and performance requirements for split rendering solutions for the 3GPP IVAS codec [3]. The latter are the basis for the split rendering feature of the IVAS codec.

# 1	Scope

This TR presents a characterization of the ISAR split rendering solutions specified in TS 26.249 [4] in terms of audio quality performance and technical properties. The characterization is done both for the ISAR baseline solution that is defined as a feature of the IVAS codec and additional features that were developed and contributed under track b of the ISAR work item.

Note: As the ISAR split rendering baseline solution is also incorporated as a feature into the IVAS codec, some of the characterization results of the present document may also appear in similar form in the IVAS characterization TR 26.997 [5]. Both representations are equally valid.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 26.865: "Immersive Audio for Split Rendering Scenarios; Requirements ".

[3]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[4]	3GPP TS 26.249: “Immersive Audio for Split Rendering Scenarios“.

[5]	3GPP TR 26.997: “IVAS codec performance characterization”.

[6]	3GPP TS 26.119: “Media Capabilities for Augmented Reality”.

[7]	3GPP TR 26.998: “Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.

[8]	3GPP TR 26.806: “Study on Tethering AR Glasses – Architectures, QoS and Media Aspects”.

[9]	3GPP TR 26.928: “Extended Reality (XR) in 5G”.

[10]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	General

## 4.1	Project History

The work item “Immersive Audio for Split Rendering Scenarios (ISAR)” started early 2023 with the target to provide split immersive audio rendering solutions within Rel-18 timeframe. Essential milestones of this work were:

- Provisioning of requirements underlying the work in for of TR 26.865 [2]

- Provisioning of TS 26.249 describing split immersive audio rendering solutions meeting the requirements

- Provisioning of CRs to IVAS codec specifications [10]-[12] adding the ISAR split rendering feature to IVAS

- Provisioning of this TR documenting quality performance and technical properties of the specified split immersive audio rendering solutions.

An important split in the work was to develop IVAS specific solutions ISAR baseline solutions under a work track A and to develop and specify codec/renderer agnostic solutions under a work track B. This split is reflected in both the ISAR TS [4] and this TR where baseline solutions in the context of the IVAS codec and additional ISAR features are described separately.

The selection of IVAS specific solutions to be standardized as ISAR baseline followed a rigorous selection process within 3GPP SA4. The selection process applied the following selection rules:

Rule 1: Provision of full set of selection phase deliverables

The proponents of a candidate solution shall provide all required items listed in PD on ISAR/IVAS selection deliverables in due time in order to be considered further in the selection process of the split rendering solution of the IVAS codec standard.

Rule 2: Compliance with design constraints

The proponents of a candidate solution shall report on compliance of the candidate solution with the IVAS related ISAR design constraints in TR 26.865.

Rule 3: Performance

The performance of the candidate solution(s) will be analysed against the IVAS related ISAR performance requirements in TR 26.865.

The selection procedure involved the following steps:

The selection procedure will consist of the following pre-selection steps for a given candidate solution:

- The selection deliverables associated with the candidate solution according to rule 1 are reviewed by SA4 and a determination will be made if they meet the requirements.

- The compliance with design constraints of the candidate solution is evaluated based on the report provided according to rule 2 and a determination will be made if the design constraints are met.

- The performance of the candidate solution will be analysed against the IVAS related ISAR performance requirements in TR 26.865 and a determination will be made if the performance requirements are met.

- Based on the outcome of steps 1-3, SA4 will discuss and determine whether the candidate solution is eligible to be adopted as split rendering solution of the IVAS codec standard. This ends the pre-selection steps for the candidate.

The selection procedure will further consist of the following main selection steps for the pre-selected candidate solution(s):

- If there is only a single pre-selected candidate solution, its status will be elevated to selected candidate solution.

- If there are more than a single pre-selected candidate solution, SA4 will enter a discussion which of the pre-selected candidate solutions has the highest merit in terms of meeting or exceeding ISAR WID objectives and particularly the relevant IVAS specific performance requirements and design constraints. SA4 will then seek agreement on the most meritful solution and elevate its status to selected candidate solution.

- Agreement will be declared on the selection.

- SA will be requested to approve the selection and the relevant associated deliverables such as CRs to IVAS specifications.

The actual selection considered only a single candidate solution.

## 4.2	Overview of the ISAR Work Item

### 4.2.1	Work item justification

TS 26.119 [6] assumes a common XR Baseline Client architecture. An essential characteristic is that a functional split is envisioned between a Presentation Engine comprising a set of composite renderers that are controlled by a Scene Manager and an XR Runtime performing a set of functions that interface with a platform to perform commonly required operations, e.g. post-rendering, prior to final output. The relevant interface between Presentation engine and end device may be a 5G physical interface between, e.g., between a smartphone or 5G EDGE and a lightweight device (AR glasses) like those considered in 5G EDGe-Dependent AR (EDGAR) and 5G Wireless Tethered AR UEs as described in TR 26.998 [7] or those considered in TR 26.806 [8].

The functional split assumed in split renderer architectures is a result of stringent implementation and operational requirements applicable for rendering of XR media on XR devices. For head-tracked immersive audio, the need to rely on a split renderer architecture, may depend on various factors among which the round-trip latency between the renderer in the presentation engine and the lightweight device is a decisive parameter. There are scenarios where this latency may be substantial which may prefer a split rendering approach with pose correction in the end device for binaural audio in a similar way as for video unless decoding and head-tracked binaural audio rendering on the lightweight device does not exceed its strict complexity constraints. In other scenarios, that latency may be sufficiently low, in which case the head-tracked binaural rendering can exclusively be done in the presentation engine. It is notable that the transmission over the interface may generally be bit rate constrained and dependent on the specific physical interface.

Binaural audio rendering comprises of signal processing functionalities that may include:

- Binauralization of audio input based on head rotation (3DoF),

- Binauralization of audio input based on listener position and head rotation (6DoF),

- Room acoustics synthesis.

Audio input to be rendered may be a combination of diegetic immersive (3D audio) and non-diegetic sounds. The diegetic immersive sounds need to be binauralized using the up-to-date head rotation data. The head rotation data is typically originating from the head-tracker available from the lightweight end device. The room acoustic synthesis can be performed using room impulse response data or parametric representation thereof, typically supplied to the Presentation Engine.

Depending on constraints and design preferences of the lightweight device (AR glasses, earbuds, etc.) and the properties of the interface between Presentation Engine and end device, solutions are needed that among more are compliant with TRs 26.928 [9] and 26.998 [7]. The solutions shall address given interface characteristics and not impose any new requirements for them.

Another aspect is the currently ongoing standardization of the EVS Codec Extension for Immersive Voice and Audio Services (IVAS) codec. While low complex rendering for lightweight devices is not a specific design objective, the IVAS codec work item should ideally provide solutions that would enable using IVAS services over head-tracked lightweight clients meeting relevant requirements.

Bearing in mind the evolution of the AR/XR technologies, it would be desirable to design low complex solutions for head-tracked binaural audio rendering on lightweight devices that under certain limitations are agnostic in a sense that the pre-renderer component in the presentation engine could be connected with any immersive binaural audio framework through suitable APIs.

The solutions to be specified are intended to add to the number of rendering options to enable immersive audio services on a broad range of devices, including light-weight AR glasses or earbuds. The pre-rendering part of the solutions is expected to become non-mandatory but shall fulfill the relevant requirements set out under this work item. It should interface through a fully specified intermediate bitstream with a fully specified split rendering decoder. For end device implementations claiming support of a specific solution, a fully compliant implementation of at least the split rendering decoder shall be required. Other end device implementations not claiming support of a specific solution remain at the discretion of the implementor.

### 4.2.1	Work item objectives

The overall objective of this work item is to develop solutions for immersive binaural audio on head-tracked devices that are compatible with the envisaged split architectures (MeCAR, 26.998). The solutions should consider low-complex and lightweight devices and demonstrate operational benefits over solutions with full decoding and rendering in the end device. The following objectives should be achieved with the work item:

- Provide format specification for intermediate representation(s).

  - Provide functional requirements for (pre-)renderer operations to be carried out by Presentation Engine.

  - Define suitable APIs.

- Provide encoder, bitstream and decoder specification for intermediate representations including audio with and without post-rendering control metadata.

- Provide a specification for decoded intermediate representations to provide binaural audio output with and without head-tracker input and post-rendering control metadata.

- Consider potential solutions offered by the IVAS work item, and specify the necessary interfaces.

The work item shall in a first phase identify and agree relevant requirements to be documented in a TR. This shall cover:

- Design constraints related to complexity and memory as well as constraints related to relevant interfaces between presentation engine and end device such as bit rate, latency, down- and upstream traffic characteristics.

- Design constraints related to functional capability requirements such as rendering of non-diegetic sounds, 3DoF rendering of diegetic immersive sounds, 6DoF rendering of diegetic immersive sounds, including simultaneous rendering of different sound categories, and room acoustics synthesis.

- Performance requirements.

The solution(s) are characterized for the range of relevant interface characteristics between presentation engine and lightweight device. The case where the immersive audio is decoded and rendered within the end device should be considered as a reference.

The requirements will be documented in a first technical report. The developments under this work item shall lead to a new specification defining among others textual descriptions of the involved renderers and codec (incl. frame loss concealment) of the intermediate representation(s). The performance of the developed solutions in relation to the requirements will be documented in a second technical report. Solutions meeting the ISAR split rendering requirement may be added to the set of IVAS codec specifications (by means of CRs) if they are found suitable for IVAS.  The developed solutions should also be referenced in the MeCAR specification.

Specific split rendering solutions for IVAS should comprise a non-mandatory default split rendering encoder for the specified internal and stand-alone IVAS renderers. In addition, for a given specific solution there should be specified interfaces offering the possibility either to connect a given (proprietary) renderer for IVAS to the intermediate representation encoder or to use proprietary pre-renderers/intermediate encoders to produce compliant intermediate bitstreams. Such proprietary solutions shall be compliant with the relevant requirements documented in the first technical report. ISAR end device implementations for IVAS claiming support of a specific solution shall be required to have at least a fully compliant split rendering decoder. Other decoder/post-renderer implementations not claiming support of a specific ISAR solution for IVAS remain at the discretion of the implementor.

# 5	Terms of Reference

The design constraints and performance requirements defined in the requirements TR [2] constitute the Terms of Reference for the ISAR work. Notably, specific design constraints and performance requirements could only be defined given IVAS as target codec/renderer of the baseline ISAR solutions. For codec/renderer agnostic solutions, the TR provides guidelines that can be turned into specific design constraints and performance requirements once a specific target system is defined.

# 6	Audio Quality evaluations

## 6.1	ISAR Baseline quality

### 6.1.1	Selection tests

#### 6.1.1.1	Test plan

The selection tests evaluating the performance of the single ISAR candidate solution for IVAS was carried out according to a permanent document on Testing Aspects for Phase/Track 2/a. Annex A provides the main part of this document to provide the context within which the test results provided below were obtained. The complete document is provided for reference in the electronic attachment of this TR.

#### 6.1.1.2	Test results

Editor’s note: Add results from lab reports Tdocs 572, 574, 584, 593, 727, 724 in a consistent layout

### 6.1.2	Further tests

### 6.1.1.x	Test report x

Editor’s note: To be added at SA4#128.

## 6.2	Quality of additional ISAR features

## 6.2.x	Test report x

Editor’s note: To be added at SA4#128.

# 7	ISAR solution properties

## 7.1	ISAR Baseline properties

Editor’s note: the following contents is taken from Tdoc S4-240713. Some text adjustments are planned.

[

### 7.1.1	Introduction

This contribution reports on how the split rendering candidate solution for IVAS Codec by the IVAS Codec Public Collaboration complies with the design constraints defined for ISAR phase 1/track a solutions. The design constraints are specified in TR 26.865 clauses 6 (Physical Design Constraints) and 7 (Functional Design Constraints). Reference is made to these.

lists the provided selection deliverables associated with the selection of the split rendering candidate solution for IVAS Codec by the IVAS Codec PC.

### 7.1.2	CMD lines used for complexity and memory measurements

This section provides the command lines of the IVAS/ISAR candidate solution executables used for the complexity and memory measurements. The command lines also show the used test vectors, defined under the IVAS codec work. The measurements were done by using the ITU-T WMC Tool also used for IVAS codec complexity and memory measurements by the IVAS codec PC.

|  | HOA3 + LCLD | MC714 + LC3plus | ISM + LC3plus | MASA + LCLD |
| --- | --- | --- | --- | --- |
| IVAS ENC | ./IVAS_cod -sba 3 512000 48 ../testv/stv3OA48c.wav enc_out.pkt | ./IVAS_cod -mc 7_1_4 512000 48 ../testv/stv714MC48c.wav enc_out_mc.pkt | ./IVAS_cod -ism 4 .. /testv/stvISM1.csv .. /testv/stvISM2.csv .. /testv/stvISM3.csv .. /testv/stvISM4.csv 512000 48 .. /testv/stv4ISM48s.wav enc_out_ism4.pkt | ./IVAS_cod -masa 2 .. /testv/stv2MASA2TC48c.met 512000 48 .. /testv/stv2MASA2TC48c.wav enc_out_masa.pkt |
| IVAS DEC | ./IVAS_dec -render_config rc.txt -t ../trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out.pkt split_out_sba_lcld_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config rc.txt -t ../trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_mc.pkt split_out_mc_lc3plus_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config ./Rc.txt -t .. /trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_ism4.pkt split_out_ism4_lc3plus_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config ./Rc.txt -t .. /trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_masa.pkt split_out_masa_lcld_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; |
| ISAR POST REND | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_sba_lcld_20ms.pkt -o split_out_sba_lcld_20ms_out.wav -fs 48 -t ../trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_mc_lc3plus_20ms.pkt -o split_out_mc_lc3plus_20ms_out.wav -fs 48 -t ../trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_ism4_lc3plus_20ms.pkt -o split_out_ism4_lc3plus_20ms_out.wav -fs 48 -t .. /trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_masa_lcld_20ms.pkt -o split_out_masa_lcld_20ms_out.wav -fs 48 -t .. /trajectories/full_circle_in_15s.csv |

### 7.1.3	Physical design constraints applicable for IVAS split rendering scenarios

The following physical design constraints apply for split rendering solutions involving the IVAS codec:

| Physical attribute | Constraint | Values based on “isar_selection_branch” |
| --- | --- | --- |
| Complexity of operation in end-rendering lightweight device | The complexity of operation in end-rendering lightweight device shall not exceed 80+(DOF*20) wMOPS. 0DOF: 80 1DOF: 100 2DOF: 120 3DOF: 140 | 3DOF (with LC3plus): max 73.66, avg 70.7 3DOF (with LCLD): max 60.96, avg 51.41 |
| Complexity of operation at capable device/node | The complexity of operation at pre-rendering device/node shall be characterized. | IVAS decoder + 3DOF split pre-rendering with LC3plus (MC714): max 1063.97, avg 1037.15 IVAS decoder + 3DOF split pre-rendering with LC3plus (ISM4): max 864.88, avg 854.2 IVAS decoder + 3DOF split pre-rendering with LCLD (HOA3): max 925.5, avg 837.0 IVAS decoder + 3DOF split pre-rendering with LCLD (MASA2): max 662.12, avg 653.06 |
| Memory footprint of operation in end-rendering lightweight device | The RAM consumption shall not exceed 100+(DOF*50) kWords. 0DOF: 100 kWords 1DOF: 150 kWords 2DOF: 200 kWords 3DOF: 250 kWords The ROM (PROM and table ROM) shall not exceed 150 kWords. | (word = 4 Bytes) RAM (heap + stack): 3DOF (with LC3plus): 67.51 kWords 3DOF (with LCLD): 67.87 kWords PROM (lc3plus + lib_isar + lcld): 20.04 kWords TROM (lc3plus + lib_isar + lcld): 42.14 kWords |
| Memory footprint of operation at pre-rendering device/node | The memory footprint of operation at pre-rendering device/node shall be characterized. | RAM: IVAS decoder + 3DOF split pre-rendering with LC3plus (MC714): 586.751 kWords IVAS decoder + 3DOF split pre-rendering with LC3plus (ISM4): 393.24 kWords IVAS decoder + 3DOF split pre-rendering with LCLD (HOA3): 467.63 kWords IVAS decoder + 3DOF split pre-rendering with LCLD (MASA2): 290.39 kWords PROM (lib_com + lib_dec + lib_rend):172.6 kWords TROM (lib_com + lib_dec + lib_rend):156.4 kWords |
| Algorithmic motion-to-sound latency in head-tracked rendering operation | 0-DOF: no constraint 1-DOF: 30 ms for rotations around corrected axis (in post rendering), for other axes no constraints 2-DOF: 30 ms for rotations around corrected axes (in post rendering), for the remaining axis no constraint 3-DOF: 30 ms for rotations around all axes (in post rendering) | 2.5ms <= Algorithmic motion-to-sound latency <= 22.5 ms |
| Algorithmic audio delay | The total algorithmic end-to-end audio delay including IVAS algorithmic delay shall not exceed 50 ms. | LC3plus; LCLD; DOF. SBA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. MC714@512kbps; 32+5+2.5 = 39.5ms; 32+5 = 37ms; 3. ISM@512kbps; 32+5+2.5 = 39.5ms; 32+5 = 37ms; 3. MASA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. OMASA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. OSBA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. LC3plus; LCLD; DOF. SBA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. MC714@512kbps; 32+2.5 = 34.5ms; 32+5 = 37ms; 0. ISM@512kbps; 32+2.5 = 34.5ms; 32+5 = 37ms; 0. MASA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. OMASA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. OSBA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. NOTE: The additional delay for 1-3DOF on top of IVAS is 5ms for pose correction with LCLD coding, 7.5ms (5+2.5ms) for pose correction and LC3plus coding. The additional delay on top of IVAS for 0-DOF operation is 2.5ms for LC3plus coding and 0 or, respectively, 5ms for LCLD coding. In case of pre-rendering in CLDFB domain additional delay by LCLD coding can be circumvented. |
| Bit rate of coded intermediate representation | The Split Rendering solution should offer 3-DOF operation at multiple bit rates with rate switching support and shall at least offer operation at 768 kbps. For 3-DOF operation, 384 kbps and 512 kbps should be offered in addition to the required 768 kbps. For 1-DOF operation, 256 kbps should be offered. For 0-DOF operation, 256 kbps and even lower bit rates should be offered. For cases with high transmission channel capacities, high bit rate operation modes > 768 kbps may be offered if QoE gains can be demonstrated. | DOF; Bitrates supported (kbps). 0; 256, 384, 512. 1; 384, 512, 768. 2; 384, 512, 768. 3; 384, 512, 768. |

### 7.1.4	Functional design constraints applicable for IVAS split rendering scenarios

The following functional design constraints apply for split rendering solutions involving the IVAS codec:

| Functional attribute | Constraint |  |
| --- | --- | --- |
| Immersive audio formats of native coding format | All required immersive IVAS encoder input formats according to Pdoc IVAS-4 shall be supported. IVAS stereo input format should be supported for 0-DOF operation. | SBA: Done MC: Done ISM: Done MASA: Done OSBA: Done OMASA: Done Stereo: Not done |
| Bit rates of required immersive audio coding modes of native coding format | All required bit rates of IVAS immersive operation modes according to Pdoc IVAS-4 shall be supported. All IVAS stereo bit rates should be supported for 0-DOF operation. | All bitrates supported for immersive formats Stereo: Not done |
| Head-trackability of immersive audio formats | The head-trackability of the immersive audio formats of the native coding format shall be retained. Preservation of the DOF level is the objective, reduced DOF levels may be provided. Note: an audio format supported by the native coding system may be 3-DOF head-trackable, i.e., around 3 axes (yaw, pitch, roll). The Split Rendering system should retain this possibility. Complexity or bit rate reduced variants may though reduce this to lower DOF levels like yaw-only correction (1-DOF). | Done |
| Packet loss concealment (PLC) | A PLC solution shall be provided. | Done |
| Non-diegetic audio support | The solution shall support (1DOF – 3DOF) diegetic audio and (0-DOF) one-channel non-diegetic audio and two-channel (stereo or binaural) non-diegetic audio. It shall be possible to overlay post rendered audio obtained from instances operated with diegetic and non-diegetic audio. | Done |

### 7.1.5	Conclusion

The comparative evaluation of the properties of the split rendering candidate solution for IVAS Codec by the IVAS Codec Public Collaboration shows that all design constraints defined in TR 26.865 are met.

## ]

## 7.2	Properties of additional ISAR features

Annex A (informative):
ISAR Pdoc on Testing Aspects for Phase/Track 2/a

Organization of tests

Overview of the Selection Test Process

The selection tests of the IVAS specific ISAR solution will be organized as in-house tests by Solution Proponents Laboratories. The execution of these subjective tests is under the responsibility of the solution proponents participating in the selection and other volunteering organizations.

The selection test experiments will be duplicated and additionally run by suitable cross-checkers, Cross-check Listening Laboratories, with no stake in the candidate solutions under test.

The selection test results will be reported by the testing organizations to SA4 with suggested statistical result analysis. SA4 will review these analyses and, if found valid, confirm them to base its selection decision on them.

The processing of the selection test material is under the responsibility of the solution proponents participating in the selection. It is based on commonly available processing scripts, solution candidate executables, original sound material and head-tracker trajectories available to the solution proponents and other organizations who volunteer to carry out cross-checks.

Responsibilities

Solution Proponent (SP)

The specific responsibilities of the SP are:

Make executables of solution candidate publicly available.

Develop common processing scripts using the condition lists defined in this document and the processing steps defined in the processing plan.

Process the test material using commonly available processing scripts, the shared solution candidate executables, original sound material and head-tracker trajectories.

Communicate with Volunteering Processing Cross-check Organizations to verify correct processing.

Solution Proponents Laboratory (SPL)

Carry out selection tests according to the requirements of this test plan.

Carry out statistical result analysis according to the requirements of this test plan and provide test report including analysis to SA4.

Obligations as SPL:

The testing shall be caried out in a blinded fashion not revealing the conditions to the subjects.

No test subjects must be used that were actively involved in developing the split rendering features in the systems under test that are exposed by the experiments.

The test report shall describe how the listening lab ensured unbiased testing.

Volunteering Cross-check Listening Laboratories (CLL)

Carry out selection tests according to the requirements of this test plan.

Carry out statistical result analysis according to the requirements of this test plan and provide test report including analysis to SA4.

Obligations as CLL:

The testing shall be caried out in a blinded fashion not revealing the conditions to the subjects.

The CLL shall not be contributor of the split rendering features in the systems under test that are exposed by the experiments.

The test report shall contain a statement confirming that the listening lab has met the obligations.

Listening Laboratories (LL) (both SPL and CLL)

Provide a listening environment meeting the listening conditions for BS.1534 testing [4].

Volunteering Processing Cross-check Organizations (PCO)

Process the test material using commonly available processing scripts, the shared solution candidate executables, original sound material and head-tracker trajectories.

Communicate with Solution Proponent to verify correct processing.

SA4

Review selection test analyses received from LLs and determine their validity.

Selection of Candidate Solution according to selection rules for IVAS specific ISAR solutions targeted in Phase/Track 2/a of the ISAR work [1].

Statistical analysis of results

The statistical result analysis reports shall present the results of the Terms of Reference (ToR) tests using Student’s Dependent Groups t-test (single-sided at 95% confidence level). Results of the Requirement ToR tests for each experiment shall be presented containing all relevant data allowing to verify proper execution of the Student’s Dependent Groups t-test.

In the for Requirement ToR tests this should lead to the following indications:

Requirement ToR tests that are passed, (i.e., CuT “not worse than” Requirement) are indicated by CuT NWT Ref.

Requirement ToR tests that are exceeded, (i.e., CuT “better than” Requirement) are indicated by CuT BT Ref.

Requirement ToR tests that are failed (i.e., CuT “worse than” Requirement) are indicated by CuT WT Ref.

Identities

In the following, the identities of the involved organizations are listed:

SP (proponent of CuT):

Proponent companies:
Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Main contributor(s) to CuT whose split rendering features are exposed in the experiments:
Dolby Sweden AB, Fraunhofer IIS

SPLs:

Dolby Sweden AB, Fraunhofer IIS

CLLs:

Qualcomm, Nokia, Bytedance, Ittiam

Processing Cross-check Organizations (PCO)

Fraunhofer IIS, Dolby

LL assignment

| - Experiment | - SPL | - CCL/other SPL |
| --- | --- | --- |
| - BS1534-1: SBA (HOA3) | - Dolby | - Qualcomm |
| - BS1534-2: Multi-channel 7.1+4 | - Fraunhofer IIS | - Ittiam |
| - BS1534-3: Objects | - Fraunhofer IIS | - Nokia |
| - BS1534-4: MASA | - Dolby | - Bytedance |

Information relevant to all Experiments

General Technical Notes

Any and all deviations from the specifications contained in this document must be documented and submitted to SA4 along with the test reports.

General Consideration of Experiments

Difference scenario between assumed and actual end-device poses

- For the evaluation of ISAR split rendering solution, a primary focus should be the testing with relevant difference scenarios between assumed and actual end-device poses. To cover relevant cases, the used head-tracker trajectory files should be taken from the following categories:

Static within range: +-20 degrees

Dynamic within range: +-20 degrees

Sinusoidal: 0.25 Hz

Triangular: 0.5 Hz

Real, i.e., derived from real head tracker trajectories with movements giving rise to substantial differences (>15 degrees) between assumed and actual end-device poses and exposing the tested methods to a sufficient degree.

DOF

- Another aspect of interest is the ability to deal with differences of assumed and actual end-device poses around different axes, i.e., the number of degrees of freedom (1-3) which the candidate solutions can cope with. Accordingly, the head-tracker trajectory files shall cover the following DOF scenarios:

1-DOF with pose deviations in yaw

2-DOF with pose deviations in yaw and pitch

3-DOF with pose deviations in yaw, pitch and roll

Rendering simulation

- Two different rendering simulation methodologies shall be covered in the tests.

Trajectory nullification [2]
This simulation methodology is based on the concept that an immersive audio scene is pre-rotated prior to IVAS encoding while the head-tracked rendering compensates for the pre-rotation. In the ideal case and under certain conditions, this compensation can be perfect. This simulation methodology exposes the ability of a split rendering system to compensate for the pre-rotation despite the differences between assumed and actual end-device poses.

Unguided end-device pose
This simulation methodology is based on rendering a decoded an immersive audio scene according to a given head-tracker trajectory. This simulation methodology exposes the ability of a split rendering system to follow the actual head-tracker trajectory despite the mere availability of the divergent assumed head-tracker trajectory available at the pre-renderer.

Input formats

- According to the ISAR requirements, the tests shall cover the following IVAS codec input formats:

SBA (HOA3)

MASA (2 TCs)

Multi-channel (7.1.4)

Objects (ISM-4)

Methodology

BS.1534 test methodologies shall be used in the ISAR selection tests. High-level configuration of the experiments is outlined below.

BS.1534

Number of items per experiment: 12

10 experienced listeners

Total number of conditions: 4

Number of anchor conditions: 2

Native reference system

7 kHz low-pass anchor

Head-tracker trajectories

All head-tracker trajectory files shall follow the convention imposed by the IVAS specifications [8], i.e., shall be usable by the IVAS decoder/renderer.

Head-tracker trajectory categories and DOF

- Head-tracker trajectories shall meet the above-defined category and DOF specifications.

Head-tracker trajectory availability and selection

- Head-tracker trajectories of the above-defined categories and DOF will be publicly collected from volunteering organizations. After collection and checking suitability, a list of available trajectories will be generated. The collection and checking will be done jointly by the involved organizations of the ISAR selection, i.e., the SP, the LLs and the PCOs.

- In a second step, these organizations will jointly select up to 6 suitable trajectories for each test. In case this number of trajectories is not available, a smaller number is selected where a given trajectory may be reused across different tests or within a test.

- The involved organizations will document their trajectory selection and assignment to tests for inclusion of this information into this document.

Audio Material

All audio material shall be sampled at 48 kHz with Full Band (FB) content and formatted as 16-bit little endian WAVE format files [9].

Audio categories

- To cover a broad range of conceivable audio categories, the test items should be taken from the categories clean and noisy speech, music, critical audio. However, the ability of the system to deal with different audio categories is only a secondary focus of the selections tests and full coverage of these categories may not be possible.

Test Item availability and selection

- Audio material of the above categories has been collected as part of the IVAS codec selection phase. Details of this material are available in the IVAS test plan [3]. A subset of this material is either publicly available or at least available to the involved organizations of the ISAR selection, i.e., the SP, the LLs and the PCOs. These organizations will in a first step create a list of commonly available test items.

- In a second step, these organizations will jointly select up to 12 suitable original test items for each test. In case this number of test items is not available, a smaller number is selected where a given test item may be reused across different tests or even within a test if the applied head-tracker trajectories, DOF or simulation methodology is different.

- The involved organizations will document their test item selection and assignment to tests for inclusion of this information into this document.

Training material

No dedicated training material will be made available for use in a potential training phase in which the subjects may familiarize with the testing methodology and environment.

Such a training phase is voluntary and upon the own responsibility of the involved LLs. No items from the main tests shall be used for training. A training phase shall be executed as a separate short BS.1534 session.

Listening Systems and Listening Environments

The ISAR Selection Test will use the following listening systems:

High-quality stereo headphones for binaural listening, e.g.:

Sennheiser HD 650

Subjective Experiments

The purpose of the 4 experiments (Experiments BS1534-1 – BS1534-4) is to evaluate the performances of the IVAS specific ISAR solution candidate with respect to the performance requirements and objectives defined in ISAR TR 26.865 [5].

The details provided in this section and in corresponding Annexes are those that are specific to each particular experiment. Generic information can be found in Section 4. Therefore, the LLs should use the information in Section 4 in conjunction with the information given in this section and Annexes.

Table 4 shows high-level overview of the experiments.

Detailed conditions for each subjective experiment are defined in Annex B.

Table 4: High-level overview of ISAR selection experiments

| Exp | Input format | Source material | Listening environment | Bitrates kbps |
| --- | --- | --- | --- | --- |
| BS1534-1 | SBA (HOA3) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-2 | Multi-channel 7.1+4 | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-3 | Objects (ISM-4) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-4 | MASA (2 TC) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |

Annex B (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-05 | SA4#128 | S4-241071 |  |  |  | Initial version | 0.0.1 |