| 3GPP TS 29.552 V0.5.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Network Data Analytics signalling flows; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Reference Architecture for Data Analytics	9

5	Signalling Flows for the Network Data Analytics Framework	9

5.1	General	9

5.2	Analytics Exposure Procedures	9

5.2.1	General	9

5.2.2	Network data analytics Subscribe/Unsubscribe/Notify	9

5.2.2.1	Analytics Subscribe/Unsubscribe/Notify initiated by 5GC NFs, OAM or AFs	9

5.2.2.2	Analytics Subscribe/Unsubscribe/Notify initiated by AFs via the NEF	10

5.2.3	Network data analytics information request	11

5.2.3.1	Analytics information request initiated by 5GC NFs, OAM or AFs	11

5.2.3.2	Analytics information request initiated by AFs via the NEF	12

5.2.4	Analytics Exposure via DCCF	13

5.2.5	Analytics Exposure via DCCF and MFAF	17

5.3	Analytics Aggregation from Multiple NWDAFs	17

5.4	Analytics Context Information Transferring	17

5.5	Data Collection	17

5.6	ML Model provisioning procedures	17

5.6.1	General	17

5.6.2	ML Model Subscribe/Unsubscribe/Notify procedure	18

5.7	Procedures for Specific Network Data Analytics	19

5.7.1	General	19

5.7.2	Network Slice (Instance) load level Analytics	19

5.7.3	Observed Service Experience Analytics	22

5.7.4	NF load Analytics	22

5.7.5	Network Performance Analytics	22

5.7.6	UE Mobility Analytics	24

5.7.7	UE Communication Analytics	27

5.7.8	Expected UE behavioural Analytics	30

5.7.9	Abnormal UE behavioural Analytics	31

5.7.10	User Data Congestion Analytics	33

5.7.11	QoS Sustainability Analytics	33

5.7.12	Dispersion Analytics	33

5.7.13	WLAN Performance Analytics	33

5.7.14	Session Management Congestion Control Experience Analytics	33

5.7.15	Redundant Transmission Experience Analytics	33

5.7.16	DN Performance Analytics	33

5.8	Procedures for NWDAF Discovery and Selection	35

5.8.1	General	35

5.8.2	Procedures related to NRF	36

5.8.2.1	General	36

5.8.2.2	 NWDAF Registration in NRF	36

5.8.2.3	Consumer discovery and selection of NWDAF in NRF	36

5.8.3	Procedures related to UDM	36

5.8.3.1	General	36

5.8.3.2	 NWDAF containing AnLF Registration/Deregistration in UDM	36

5.8.3.2.1	NWDAF containing AnLF Registration in UDM	36

5.8.3.2.2	NWDAF containing AnLF Update of Registration in UDM	37

5.8.3.2.3	NWDAF containing AnLF De-Registration in UDM	37

5.8.3.3	Consumer discovery and selection of NWDAF containing AnLF in UDM	38

5.8.4	Procedures for PCF learning NWDAF IDs for served UEs	39

Annex A (informative):	Change history	40

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies detailed call flows of Network Data Analytics over the Nnwdaf, Nsmf, Npcf, Nbsf, Nnef, Naf, Ndccf, Nadrf, Nmfaf and Nudm service-based interfaces and their relationship with the flow level signalling in 5G system.

NOTE:	The call flows depicted in this Technical Specification do not cover all traffic cases.

The stage 2 definition and procedures of Network Data Analytics are contained in 3GPP TS 23.288 [2] and 3GPP TS 23.502 [3]. The 5G System Architecture is defined in 3GPP TS 23.501 [4].

Detailed definitions of the involved services are provided in 3GPP TS 29.520 [5], 3GPP TS 29.508 [6], 3GPP TS 29.523 [7], 3GPP TS 29.554 [8], 3GPP TS 29.521 [9], 3GPP TS 29.522 [10], 3GPP TS 29.591 [11], 3GPP TS 29.517 [12], 3GPP TS 29.574 [15], 3GPP TS 29.575 [16], 3GPP TS 29.576 [17], 3GPP TS 29.503 [22], 3GPP TS 29.510 [23], 3GPP TS 29.507 [24] and 3GPP TS 29.512 [25].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition of the 5G System are specified in 3GPP TS 29.500 [13] and 3GPP TS 29.501 [14].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[6]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[7]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[8]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[12]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[13]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[15]	3GPP TS 29.574: "5G System; Data Collection Coordination Services; Stage 3".

[16]	3GPP TS 29.575: "5G System; Analytics Data Repository Services; Stage 3".

[17]	3GPP TS 29.576: "5G System; Messaging Framework Adaptor Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[20] 	3GPP TS 29.536: "5G System: Network Slice Admission Control Services; Stage 3".

[21] 	3GPP TS 29.531: "5G System: Network Slice Selection Services; Stage 3".

[22]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[23]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3"

[24]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3"

[25]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3"

[26] 	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADRF	Analytics Data Repository Function

AF	Application Function

AMF 	Access and Mobility Management Function

AnLF	Analytics Logical Function

DCCF	Data Collection Coordination Function

MFAF	Messaging Framework Adaptor Function

ML	Machine Learning

MTLF	Model Training Logical Function

NEF	Network Exposure Function

NRF	Network Repository Function

NSACF	Network Slice Admission Control Function

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

SMF	Session Management Function

UDM	Unified Data Management

# 4	Reference Architecture for Data Analytics

This clause will provide the reference architecture for data analysis.

# 5	Signalling Flows for the Network Data Analytics Framework

## 5.1	General

This clause will provide a general introduction for Signalling Flows for the Network Data Analytics Framework.

## 5.2	Analytics Exposure Procedures

### 5.2.1	General

The analytics exposure procedures allow the NF service consumers (i.e. NFs, OAM and AFs) to obtain the analytics information from the NWDAF.

### 5.2.2	Network data analytics Subscribe/Unsubscribe/Notify

#### 5.2.2.1	Analytics Subscribe/Unsubscribe/Notify initiated by 5GC NFs, OAM or AFs

This procedure is used by the NF service consumers (i.e. NFs, OAM and AFs) to subscribe to/unsubscribe from analytics information directly from the NWDAF, it is also used by the NWDAF to notify the observed analytics event(s) to the NF service consumer if subscribed before.

Figure 5.2.2.1-1: Analytics Subscribe/Unsubscribe/Notify initiated by 5GC NFs, OAM or AFs

1.	In order to subscribe to notification(s) of analytics information from the NWDAF, the NF service consumer invokes Nnwdaf_EventsSubscription_Subscribe service operation by sending an HTTP POST request targeting the resource "NWDAF Events Subscriptions". The request includes the subscribed events and may include event filter information.

In order to update the existing subscription, the NF service consumer invokes Nnwdaf_EventsSubscription_Subscribe service operation by sending an HTTP PUT request with Resource URI of the resource "Individual NWDAF Event Subscription".

2.	The NWDAF responds to the Nnwdaf_EventsSubscription_Subscribe service operation. Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the NWDAF responds to the NF service consumer with "201 Created", and the URI of the created subscription is included in the Location header field.

Upon receipt of the HTTP PUT request, if the subscription is accepted to be updated, the NWDAF responds to the NF service consumer with "200 OK" or "204 No Content"

3.	If the NWDAF observes the subscribed event(s), the NWDAF invokes Nnwdaf_EventsSubscription_Notify service operation to report the event(s) by sending an HTTP POST request with {notificationURI} as Notification URI.

4.	The NF service consumer sends an HTTP "204 No Content" response to the NWDAF.

5.	In order to unsubscribe to the notification(s) of analytics information from the NWDAF, the NF service consumer invokes Nnwdaf_EventsSubscription_Unsubscribe service operation by sending an HTTP DELETE request targeting the resource "Individual NWDAF Event Subscription", to the NWDAF to unsubscribe from analytics information. The request includes the event subscriptionId of the existing subscription that is to be deleted.

6.	The NWDAF responds to the Nnwdaf_EventsSubscription_Unsubscribe service operation. If the unsubscription is accepted, the NWDAF responds with "204 No Content".

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify service operations refer to 3GPP TS 29.520 [5].

#### 5.2.2.2	Analytics Subscribe/Unsubscribe/Notify initiated by AFs via the NEF

This procedure is used by the AF to subscribe to/unsubscribe from analytics information from the NWDAF via the NEF, it is also used by the NWDAF to notify the analytics event(s) to the AF via the NEF, if subscribed before.

Figure 5.2.2.2-1: Analytics Subscribe/Unsubscribe/Notify initiated by AFs via the NEF

1.	In order to subscribe to notification(s) of analytics exposure via the NEF, the AF invokes the Nnef_AnalyticsExposure_Subscribe request by sending an HTTP POST request message targeting the resource "Analytics Exposure Subscriptions" as defined in subclause 4.4.14.1 of 3GPP TS 29.522 [10].

In order to update an existing analytics exposure subscription, the AF shall send an HTTP PUT request message to the NEF to the resource "Individual Analytics Exposure Subscription" requesting to change the subscription.

2.	Upon receipt of the HTTP request from the AF, if the AF is authorized with the requested analytics event(s) and the requested parameters comply with the inbound restriction in the analytics exposure mapping, the NEF shall invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in step 1 in subclause 5.2.2.1.

3.	The NWDAF responds to the Nnwdaf_EventsSubscription_Subscribe service operation as described in step 2 in subclause 5.2.2.1.

4.	Upon receipt of the HTTP request response from the NWDAF, the NEF shall invoke the Nnef_AnalyticsExposure_Subscribe response message by mapping and forwarding the response to the AF.

5.	If the NWDAF observes the subscribed event(s), the NWDAF invokes Nnwdaf_EventsSubscription_Notify service operation as described in step 3 in subclause 5.2.2.1 to the NEF.

6.	If the NEF receives an analytics information notification from the NWDAF indicating that the subscribed analytics event has been detected, the NEF shall invoke the Nnef_AnalyticsExposure_Notify request by sending HTTP POST request message provide a notification to the AF request including the AnalyticsEventNotification data structure at least with the detected analytics event identified by the notification URI together with the notification correlation identifier received during creation of the Individual Analytics Exposure Subscription.

7.	Upon receipt of the analytics event notification, the AF shall respond the NEF with a "204 No Content" status code to confirm the received notification in Nnef_AnalyticsExposure_Notify response message.

8.	The NEF shall forward the HTTP "204 No Content" response to the NWDAF.

9.	In order to delete an existing analytics exposure subscription, the AF shall invoke the Nnef_AnalyticsExposure_Unsubscribe request by sending an HTTP DELETE request message to the NEF to the resource "Individual Analytics Exposure Subscription".

10.	If the NEF receives an HTTP DELETE request from the AF, the NEF shall invoke the Nnwdaf_EventsSubscription_Unsubscribe service operation as described in step 5 in subclause 5.2.2.1.

11.	The NWDAF responds to the Nnwdaf_EventsSubscription_Unsubscribe service operation as described in step 6 in subclause 5.2.2.1.

12.	The NEF shall forward the HTTP "204 No Content" response to the AF.

NOTE:	Details of AnalyticsExposure API refer to subclause 4.4.14 and subclause 5.6 of 3GPP TS 29.522 [4].

### 5.2.3	Network data analytics information request

#### 5.2.3.1	Analytics information request initiated by 5GC NFs, OAM or AFs

This procedure is used by the NF service consumers (i.e. 5GC NFs, OAM and AFs) to retrieve analytics information directly from the NWDAF.

Figure 5.2.3.1-1: Network data analytics info request procedure

1.	The NF Service Consumer invokes Nnwdaf_AnalyticsInfo_Request service operation by sending an HTTP GET request targeting the resource "NWDAF Analytics", to the NWDAF to request the analytics information. The request includes analytics identifier and related event filter information.

2.	The NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation. If the request is accepted, the response includes the requested analytics information with "200 OK".

NOTE:	For details of Nnwdaf_AnalyticsInfo_Request service operation refer to 3GPP TS 29.520 [5].

#### 5.2.3.2	Analytics information request initiated by AFs via the NEF

This procedure is used by the AFs to retrieve analytics information from the NWDAF via the NEF.

Figure 5.2.3.2-1: Analytics Request initiated by AFs via the NEF

1.	In order to fetch analytics information, the AF invoke the Nnef_AnalyticsExposure_Fetch request by sending an HTTP POST request message to the NEF to the customized operation URI "{apiRoot}/3gpp-analyticsexposure/v1/fetch" as defined in subclause 4.4.14.2 of 3GPP TS 29.522 [10].

2.	Upon receipt of the HTTP request from the AF, if the AF is authorized with the requested analytics event(s) and the requested parameters comply with the inbound restriction in the analytics exposure mapping, the NEF shall invoke the Nnwdaf_AnalyticsInfo_Request service operation as described in step 1 in subclause 5.2.3.1.

3.	The NWDAF responds with the analytics information as described in step 2 in subclause 5.2.3.1 to the NEF.

4.	The NEF responds with the analytics information to the AF.

NOTE:	Details of AnalyticsExposure API refer to subclause 4.4.14 and subclause 5.6 of 3GPP TS 29.522 [4].

### 5.2.4	Analytics Exposure via DCCF

This procedure is used by NF service consumer(s) (e.g. NFs/OAM) based on local configuration, to subscribe/unsubscribe to NWDAF analytics event(s) via the DCCF, and upon the delivery option "Delivery via DCCF" configured on the DCCF, also used by DCCF to notify the NF service consumer(s) of the analytics information via the DCCF if subscribed before.

Figure 5.2.4-1: Analytics Exposure via DCCF

1.	In order to subscribe to notification(s) of analytics exposure via the DCCF based on local configuration, the NF service consumer invokes the Ndccf_DataManagement_Subscribe service operation by sending an HTTP POST request message targeting the resource "undefined resource name", the HTTP POST message shall include the undefined name of data structure as request body with parameters as defined in subclause 5.7.4 of 3GPP TS 29.574 [15].

2.	The DCCF determine whether the historical data handling is applicable or not.

If the historical data handling is not applicable or not supported, the DCCF shall proceed step 3a, skip step 3b, step 4b, step 5b, step 3c, step 4c and step 5c.

If the historical data is available in an ADRF, the DCCF shall proceed step 3a and step 3b, skip step 3c, step 4c and step 5c.

If the historical data is available in an NWDAF, the DCCF shall proceed step 3a and step 3c, skip step 3b, step 4b and step 5b.

Editor's Note:	It is FFS that how the DCCF determines whether the historical data handling is applicable, based on some input data which will be defined later.

3a.	The DCCF shall determine whether the analytics requested are already being collected.

If the requested analytics are already being collected by an NF service consumer, the DCCF adds the new NF service consumer to the list of NF service consumers that are subscribed for these analytics.

4a	If the analytics requested at step 1 are not already available yet, the DCCF shall invoke the Nnwdaf_EventsSubscription_Subscribe service operation by sending an HTTP POST request message request to the NWDAF targeting the resource "NWDAF Events Subscriptions" to subscribe to a new analytics exposure subscription, or if the analytics subscribed in step 1 partially matches an analytics that is already being collected by the DCCF from an NWDAF, and a modification of this subscription to the NWDAF would satisfy both the existing analytics subscriptions as well as the newly requested analytics, by sending an HTTP PUT request to the resource "Individual Analytics Exposure Subscription " to replace an existing analytics exposure subscription. The request includes the subscribed event(s) and event filter information received from the NF service consumer, mapping to the parameters as defined in subclause 5.1 of 3GPP TS 29.520 [5].

NOTE:	If the NWDAF instance or NWDAF Set is not identified by the NF service consumer, the DCCF determines the NWDAF instances that can provide analytics. If the consumer requested storage of analytics in an ADRF but an ADRF ID is not provided by the NF service consumer, or the collected analytics is to be stored in an ADRF according to configuration on the DCCF, the DCCF selects an ADRF to store the collected data.

5a	The NWDAF responds to the Nnwdaf_EventsSubscription_Subscribe service operation.

Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the NWDAF responds to the DCCF with "201 Created", and the URI of the created subscription is included in the Location header field.

Upon receipt of the HTTP PUT request, if the subscription is accepted to be updated, the NWDAF responds to the DCCF with "200 OK" or "204 No Content".

3b	If the historical data handling is applicable, and the DCCF determine to retrieve analytics data from the ADRF, the DCCF shall determine which ADRF instances might provide the analytics.

4b	In order to retrieve the historical analytics data from the ADRF, the DCCF shall invoke the Nadrf_DataManagement_RetrievalSubscribe service operation by sending an HTTP POST request message targeting the resource "undefined resource name", the HTTP POST message shall include the undefined name of data structure as request body with parameters as defined in subclause 5.1.6 of 3GPP TS 29.575 [16].

5b	The ADRF responds to the Nadrf_DataManagement_RetrievalSubscribe service operation.

Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the ADRF responds to the DCCF with "201 Created", and the URI of the created subscription is included in the Location header field.

3c	If the historical data handling is applicable, and the DCCF determine to retrieve analytics data from the NWDAF, the DCCF shall determine which NWDAF instances might provide the analytics as described and proceed step 4b.

4c	In order to retrieve the historical analytics data from the NWDAF, the DCCF shall invoke the Nnwdaf_DataManagement_Subscribe service operation by sending an HTTP POST request message targeting the resource " NWDAF Data Management Subscriptions", the HTTP POST message shall include the undefined name of data structure as request body with parameters as defined in subclause 5.3.6 of 3GPP TS 29.520 [5].

5c	The NWDAF responds to the Nnwdaf_DataManagement_Subscribe service operation.

Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the NWDAF responds to the DCCF with "201 Created", and the URI of the created subscription is included in the Location header field.

6.	The DCCF responds to the Ndccf_DataManagement_Subscribe service operation with HTTP "204 No Content" status code.

7a.	(conditional)When the analytics are available, the NWDAF invokes the Nnwdaf_EventsSubscription_Notify service operation by sending an HTTP POST request message to notify the analytics information to the DCCF.

8a.	The DCCF responds to the Nnwdaf_EventsSubscription_Notify service operation with HTTP "204 No Content" status code.

7b.	(conditional)When the historical analytics data are available in the ADRF, the ADRF shall invoke the Nadrf_DataManagement_RetrievalNotify service operation by sending an HTTP POST request message to notify the historical data to the DCCF.

8b.	The DCCF responds to the Nadrf_DataManagement_RetrievalNotify service operation with HTTP "204 No Content" status code.

7c.	(conditional)When the historical analytics data are available in the NWDAF, the NWDAF shall invoke the Nnwdaf_DataManagement_Notify service operation by sending an HTTP POST request message to notify the historical data to the DCCF.

8c.	The DCCF responds to the Nnwdaf_DataManagement_Notify service operation with HTTP "204 No Content" status code.

9.	Upon the delivery option "Delivery via DCCF" configured on the DCCF. the DCCF invoke the Ndccf_DataManagement_Notify service operation by sending HTTP POST request message(s) to send the analytics data to all notification endpoints indicated in step 1. Analytics sent to notification endpoints may be processed and formatted by the DCCF so they conform to delivery requirements for each NF service consumer or notification endpoint.

NOTE:	According to Formatting Instructions provided by the NF service consumer, multiple notifications from a NWDAF can be combined in a single Ndccf_DataManagement_Notify so many notifications from an NWDAF results in fewer notifications (or one notification) to the NF service consumer. Alternatively, a notification can instruct the analytics notification endpoint to fetch the analytics from the DCCF.

10.		The NF service consumer responds to the Ndccf_DataManagement_Notify service operation with HTTP "204 No Content" status code.

11.	(conditional) The NF service consumer invoke the Ndccf_DataManagement_Fetch service operation by sending and HTTP GET request method to fetch the analytics from the DCCF before an expiry time, if received the fetch instruction in NdccfDataManagement_Notify service operation in step 9.

12.	The NF service consumer responds to the Ndccf_DataManagement_Fetch service operation with HTTP "204 No Content" status code.

13.	When the NF service consumer no longer need the subscription to the analytics requested in step 1, shall invoke the Ndccf_DataManagement_Unsubscribe service operation by sending an HTTP DELETE request message with "{xxx}" as Resource URI, where "{subscriptionId}" is the event subscriptionId of the existing subscription that is to be deleted., using the Subscription Correlation Id received in response to its subscription in step 1. The DCCF removes the NF service consumer from the list of NF service consumers that are subscribed for these analytics.

14.	The DCCF responds to the Ndccf_DataManagement_Unsubscribe service operation with HTTP "204 No Content" status code, upon removed the NF service consumer from the list of NF service consumers that are subscribed for these analytics

15a.	If there are no other NF service consumers subscribed to the analytics, the DCCF invoke the Nnwdaf_EventsSubscription_Unsubscribe service operation by sending a HTTP DELETE request message to the NWDAF.

16a.	The NWDAF responds to the Nnwdaf_EventsSubscription_Unsubscribe service operation with HTTP "204 No Content" status code, upon the analytics event(s) subscription is removed.

15b.	If DCCF determine that no other NF service consumers requiring the historical analytics data from the ADRF, the DCCF may invoke the Nadrf_DataManagement_RetrievalUnSubscribe service operation by sending a HTTP DELETE request message to the ADRF.

16b.	The ADRF responds to the Nadrf_DataManagement_RetrievalUnSubscribe service operation with HTTP "204 No Content" status code, upon the analytics data retrieval subscription is removed.

15c.	If DCCf determine that no other NF service consumers requiring the historical analytics data from the NWDAF, the DCCF may invoke the Nnwdaf_DataManagement_UnSubscribe service operation by sending a HTTP DELETE request message to the NWDAF.

16c.	The NWDAF responds to the Nnwdaf_DataManagement_UnSubscribe service operation with HTTP "204 No Content" status code, upon the analytics data subscription is removed.

Editor's Note:	It is FFS on all the undefined resource names and undefined data type names, which will be defined later.

### 5.2.5	Analytics Exposure via DCCF and MFAF

Editor’s Note:	detailed procedure for this clause will be updated later.

## 5.3	Analytics Aggregation from Multiple NWDAFs

This clause provides signalling flows to support Analytics Aggregation from Multiple NWDAFs.

## 5.4	Analytics Context Information Transferring

This clause provides signalling flows to support Analytics Context Information Transferring.

## 5.5	Data Collection

This clause provides signalling flows to support Data Collection.

## 5.6	ML Model provisioning procedures

### 5.6.1	General

The ML Model provisioning procedures allow the NF service consumers (i.e. NWDAF (MTLF+AnLF), NWDAF (AnLF)) to obtain the ML model information on the related Analytics from another NWDAF (i.e. an NWDAF containing MTLF).

### 5.6.2	ML Model Subscribe/Unsubscribe/Notify procedure

The procedure is used by an NF service consumer to subscribe to/unsubscribe from the ML model information on the related Analytics on NWDAF containing MTLF, and also used by the NWDAF containing MTLF to notify the ML model information to the NF service consumer if it subscribed to the ML model information previously.

Figure 5.6.2-1: ML Model Subscribe/Unsubscribe/Notify procedure

1.	In order to subscribe to ML model information, the NF service consumer invokes Nnwdaf_MLModelProvision_Subscribe service operation by sending an HTTP POST request targeting the resource "NWDAF ML Model Provision Subscriptions". The request shall include the event ID(s) and the notification URI, and may include the event filter information, the target UE information, the expected time interval during which the ML model shall be reported and the expiry time.

In order to modify the existing subscription, the NF service consumer invokes Nnwdaf_MLModelProvision_Subscribe service operation by sending an HTTP PUT request with Resource URI of the resource "Individual NWDAF ML Model Provision Subscription".

2.	The NWDAF containing MTLF responds to the Nnwdaf_MLModelProvision_Subscribe service operation. Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the NWDAF containing MTLF responds to the NF service consumer with "201 Created", and the URI of the created subscription is included in the Location header field.

Upon receipt of the HTTP PUT request, if the subscription is accepted to be updated, the NWDAF containing MTLF responds to the NF service consumer an HTTP "200 OK" with a response body containing a representation of the updated subscription or "204 No Content".

3.	If the NWDAF containing MTLF determines that the subscribed ML model information is available, the NWDAF containing MTLF invokes MLModelProvision_Notify service operation to report the ML model information by sending an HTTP POST request to the NF service consumer identified by the notification URI received during the creation/modification of the subscriptions. The request shall include the event ID, the subscriptionId and either the address (e.g. URL or FQDN) of ML Model file or the ADRF ID, and may include the period of validity that indicates the time period when the provided ML model applies and spatial validity that indicates the area where the provided ML model applies.

4.	The NF service consumer responds to the NWDAF containing MTLF with an HTTP "204 No Content" message.

5.	In order to unsubscribe from the notification(s) of the ML model information, the NF service consumer invokes Nnwdaf_MLModelProvision_Unsubscribe service operation by sending an HTTP DELETE request, which targets the resource "Individual NWDAF ML Model Provision Subscription", to the NWDAF containing MTLF.

6.	If the request is accepted, the NWDAF containing MTLF deletes the subscription and responds to the NF service consumer with an HTTP "204 No Content" message.

NOTE:	For details of Nnwdaf_MLModelProvision_Subscribe /Unsubscribe/Notify service operations refer to 3GPP TS 29.520 [5].

## 5.7	Procedures for Specific Network Data Analytics

### 5.7.1	General

### 5.7.2	Network Slice (Instance) load level Analytics

This procedure is used by the NF to obtain the network slice (instance) load level analytics which are calculated by the NWDAF based on the information collected from the NSACF, NRF, AMF, SMF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.2-1: Procedure for Network Slice (Instance) load level Analytics

1a.	In order to obtain the network slice (instance) load level analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain the network slice (instance) load level analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a-2b.	If the event is set to "LOAD_LEVEL_INFORMATION" or "NSI_LOAD_LEVEL", the NWDAF invokes Nnrf_NFDiscovery_Request service operation as described in clause 5.3.2.2 of 3GPP TS 29.510 [26] to discover the AMF, SMF and NSSF instance(s) relevant to the analytics filters provided in the subscription request. The NRF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	(Only for "NSI_LOAD_LEVEL")The NWDAF may invoke Nnssf_NSSelection_Get service operation as described in clause 5.2.2.2 of 3GPP TS 29.531 [21] to obtain the NSI ID(s) corresponding to the S-NSSAI in the subscription request. The NRF responds to the NWDAF an HTTP "201 Created" response.

4a-4b.	(Only for "NSI_LOAD_LEVEL")The NWDAF may invoke Nnrf_NFManagement_NFStatusSubscribe service operation as described in clause 5.2.2.5 of 3GPP TS 29.510 [26] to subscribe to the resource usage information of a network slice instance obtained from its constituent NF instances. The NRF responds to the NWDAF an HTTP "201 Created" response.

5a-5b. (Only for "NSI_LOAD_LEVEL")	If step 4a and step 4b are performed, the NRF may invoke Nnrf_NFManagement_NFStatusNotify service operation as described in clause 5.2.2.6 of 3GPP TS 29.510 [26]. The NWDAF responds to the NRF an HTTP "204 No Content" response.

6a-6b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of individual UE registration/deregistration registered to an S-NSSAI or to an S-NSSAI and NSI ID, or request the total number of UEs served by the AMF per S-NSSAI or per S-NSSAI and NSI ID. The AMF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

8a-8b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to subscribe to the notification of individual PDU session established or PDU session released in an S-NSSAI or request the total number of PDU Sessions established in an S-NSSAI. The SMF responds to the NWDAF an HTTP "201 Created" response.

9a-9b.	If step 8a and step 8b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

10.	The NWDAF may invoke "Data collection service" to the OAM to get the mean number of UEs registered, mean number of PDU sessions established and/or the resource usage information of a network slice instance obtained from its constituent NF instances. (Obtaining the resource usage information of a network slice instance is only applicable for "NSI_LOAD_LEVEL" event).

Editor's Note:	Which service operation of the OMA may be invoked is FFS.

11a-11b.	The NWDAF may invoke Nnsacf_SliceEventExposure_Subscribe service operation as described in clause 5.3.2.2 of 3GPP TS 29.536 [20] to request the number of UEs registered to the network slice and/or the number of PDU sessions established to the network slice.

12a-12b.	The NSACF may invoke Nnsacf_SliceEventExposure_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.536 [20]. The NWDAF responds to the NSACF an HTTP "204 No Content" response.

13.	The NWDAF calculates the network slice (instance) load level analytics based on the data collected from AMF, SMF, NRF, NSACF and/or OAM.

14a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

14b-14c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

15a-15b.	The same as step 5a and step 5b.

16a-16b.	The same as step 7a and step 7b.

17a-17b.	The same as step 9a and step 9b.

18.	The same as step 10.

19a-19b.	The same as step 12a and step 12b.

20.	The same as step 13.

21a-21b.	The same as step 14b and step 14c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.3	Observed Service Experience Analytics

This clause provides signalling flows to support Observed Service Experience Analytics.

### 5.7.4	NF load Analytics

This clause provides signalling flows to support NF load analytics.

### 5.7.5	Network Performance Analytics

This procedure is used by the NF to obtain the network performance analytics which are calculated by the NWDAF based on the information collected from the AMF, NRF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.5-1: Procedure for Network Performance Analytics

1a.	In order to obtain the network performance analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain the network performance analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a-2b.	If the event is set to "NETWORK_PERFORMANCE", the NWDAF may invoke Nnrf_NFManagement_NFStatusSubscribe service operation as described in clause 5.2.2.5 of 3GPP TS 29.510 [26] to subscribe to NF (e.g. SMF, UPF) load and status information. The NRF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the NRF may invoke Nnrf_NFManagement_NFStatusNotify service operation as described in clause 5.2.2.6 of 3GPP TS 29.510 [26]. The NWDAF responds to the NRF an HTTP "204 No Content" response.

4a-4b.	If the event is set to "NETWORK_PERFORMANCE" and the type of network performance is set to "NUM_OF_UE", the NWDAF invokes Nnrf_NFDiscovery_Request service operation as described in clause 5.3.2.2 of 3GPP TS 29.510 [26] to discover the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest. The NRF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to collect the number of UEs located in the Area of Interest from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

6a-6b.	If step 5a and step 5b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

7.	The NWDAF may invoke "Performance threshold monitoring service" to the OAM as described in clause 12.3.1.2 of 3GPP TS 28.532 [19] to get the status and load information and the resource usage on the Area of Interest

Editor's Note:	Which service operation of the OMA may be invoked is FFS.

8.	The NWDAF calculates the network performance analytics based on the data collected from AMF, NRF and/or OAM.

9a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

9b-9c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

10a-10b.	The same as step 3a and step 3b.

11a-11b.	The same as step 6a and step 6b.

12.	The same as step 7.

13.	The same as step 8.

14a-14b.	The same as step 9b and step 9c.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.6	UE Mobility Analytics

This procedure is used by the NF to obtain UE mobility analytics, which is calculated by the NWDAF based on the information collected from the AMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.6-1: Procedure for UE Mobility analytics

1a.	In order to obtain the UE mobility analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain the UE mobility analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18]. This step may be skipped when e.g. the UE mobility information is available. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.518 [18]. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation to the AF directly by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the AF an HTTP "204 No Content" response.

6a-6d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

7a-7d.	If step 6a to step 6d are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 6b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

8.	The NWDAF may invoke "streaming data reporting service reportStreamData" service operation to the OAM as described in clause 12.5.1.1.5 of 3GPP TS 28.532 [19].

9.	The NWDAF may invoke the "File data reporting service subscribe" service operation to the OAM as described in clause 12.6.1.1.3 of 3GPP TS 28.532 [19] or invoke "File data reporting service listAvailableFiles" service operation to the OAM as described in clause 12.6.1.1.2 of 3GPP TS 28.532 [19].

10. The NWDAF calculates the requested UE mobility analytics based on the data collected from AMF, AF and/or OAM.

11a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

11b-11c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

12a-12b.	The same as step 3a and step 3b.

13a-13b.	The same as step 5a and step 5b.

14a-14d.	The same as step 7a and step 7d.

15.	The same as step 9.

16.	The same as step 10.

17a-17b.	The same as step 11b and step 11c.

NOTE 1:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 2:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 3:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.7	UE Communication Analytics

This procedure is used by the NF to obtain UE communication analytics, which is calculated by the NWDAF based on the information collected from the AMF, SMF and/or AF. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.7-1: Procedure for UE Communication analytics

1a.	In order to obtain the UE communication analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain the UE communication analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve one or more Type Allocation codes, UE connection management state, UE access behaviour trends and UE location trends from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to request the information of the UE and/or N4 session related data to calculate the analytics. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to request the service data from AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 8b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10. The NWDAF calculates the requested UE communication analytics based on the data collected from AMF, SMF and/or AF.

11a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

11b-11c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

12a-12b.	The same as step 3a and step 3b.

13a-13b.	The same as step 5a and step 5b.

14a-14b.	The same as step 7a and step 7b.

15a-15d.	The same as step 9a and step 9d.

16.	The same as step 10.

17a-17b.	The same as step 11b and step 11c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 3:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 4:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.8	Expected UE behavioural Analytics

This procedure is used by the NF to obtain the expected UE behavioural parameters, which are calculated by the NWDAF based on the information collected from the AMF, SMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.8-1: Procedure for Expected behavioural Analytics

1a.	In order to obtain the expected UE behavioural parameters, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain expected UE behavioural parameters, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a.	If the event is set to "UE_MOBILITY", the NWDAF collects data from AMF, AF and/or OAM as described in clause 5.7.6 from step 2a to step 9.

2b.	If the event is set to "UE_COMM", the NWDAF collects data from AMF, SMF and/or AF as described in clause 5.7.7 from step 2a to step 9d.

3. The NWDAF calculates the expected UE behavioural parameters based on the collected data from AMF, SMF, AF and/or OAM.

4a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

4b-4c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

5a-5b.	The AMF, SMF, AF and/or OAM send the notifications to the NWDAF if it has subscribed to the related events in step 2a or step 2b.

6.	The same as step 3.

7a-7b.	The same as step 4b and step 4c.

NOTE 1:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.9	Abnormal UE behavioural Analytics

This procedure is used by the NF to obtain the abnormal UE behavioural analytics which are calculated by the NWDAF based on the information collected from the AMF, SMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.9-1: Procedure for Abnormal UE behavioural Analytics

1a.	In order to obtain the abnormal UE behavioural parameters, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain abnormal UE behavioural parameters, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a.	If the event is set to "ABNORMAL_BEHAVIOUR" and the expected UE analytics type is set to "MOBILITY" or "MOBILITY_AND_COMMUN", or the list of exception IDs includes "UNEXPECTED_UE_LOCATION", "PING_PONG_ACROSS_CELLS", "UNEXPECTED_RADIO_LINK_FAILURES", or "UNEXPECTED_WAKEUP", the NWDAF collects data from AMF, AF and/or OAM as described in clause 5.7.6 from step 2a to step 9.

2b.	If the event is set to "ABNORMAL_BEHAVIOUR" and the expected UE analytics type is set to "COMMUN" or "MOBILITY_AND_COMMUN", or the list of exception IDs includes "SUSPICION_OF_DDOS_ATTACK", "UNEXPECTED_LONG_LIVE_FLOW", "UNEXPECTED_LARGE_RATE_FLOW", "WRONG_DESTINATION_ADDRESS", "TOO_FREQUENT_SERVICE_ACCESS" or "UNEXPECTED_WAKEUP", the NWDAF collects data from AMF, SMF and/or AF as described in clause 5.7.7 from step 2a to step 9d.

3. The NWDAF calculates the abnormal UE behavioural parameters based on the collected data from AMF, SMF, AF and/or OAM.

4a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

4b-4c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

5a-5b.	The AMF, SMF, AF and/or OAM send the notifications to the NWDAF if it has subscribed to the related events in step 2a or step 2b.

6.	The same as step 3.

7a-7b.	The same as step 4b and step 4c.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

### 5.7.10	User Data Congestion Analytics

This clause provides signalling flows to support User Data Congestion Analytics.

### 5.7.11	QoS Sustainability Analytics

This clause provides signalling flows to support QoS Sustainability Analytics.

### 5.7.12	Dispersion Analytics

This clause provides signalling flows to support Dispersion Analytics.

### 5.7.13	WLAN Performance Analytics

This clause provides signalling flows to support WLAN Performance Analytics.

### 5.7.14	Session Management Congestion Control Experience Analytics

This clause provides signalling flows to support Session Management Congestion Control Experience Analytics.

### 5.7.15	Redundant Transmission Experience Analytics

This clause provides signalling flows to support Redundant Transmission Experience Analytics.

### 5.7.16	DN Performance Analytics

This procedure is used by the NF to obtain DN performance analytics, which is calculated by the NWDAF based on the information collected from the AMF, SMF, AF, UPF and/or OAM. If the NF is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.3.2.

Figure 5.7.16-1: Procedure for DN Performance Analytics

1a.	In order to obtain the DN performance analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

1b-1c.	In order to obtain the DN performance analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1.

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve the UE location information and SUPI(s) from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to request DNN, S-NSSAI, application ID, UPF info, DNAI, IP filter information and QFI. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6.	The NWDAF may collect performance data from AF as described in clause 5.7.7 from step 6a to step 9d.

7.	The NWDAF may collect QoS flow bit rate, QoS flow packet delay, packet transmission and packet retransmission information from UPF.

Editor's Note:	How NWDAF collects data from UPF is not defined in stage 2.

8.	The NWDAF may collect Reference Signal Received Power, Reference Signal Received Quality, Signal-to-noise and interference ratio, the mapping information between cell ID and frequency and Cell Energy Saving State data from OAM.

Editor's Note:	Which service operation of the OMA may be invoked is FFS.

9. The NWDAF calculates the DN performance analytics based on the data collected from AMF, SMF, AF, UPF and/or OAM.

10a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

10b-10c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

11a-11b.	The same as step 3a and step 3b.

12a-12b.	The same as step 5a and step 5b.

13.	The same as step 6.

14.	The same as step 9.

15a-15b.	The same as step 10b and step 10c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

## 5.8	Procedures for NWDAF Discovery and Selection

### 5.8.1	General

NWDAF may be deployed in multiple configuration options. Not all these configurations require the same registration, discovery and selection procedures. The discovery and selection may depend on the NWDAF deployment option and the types of analytics it procedures, (i.e., whether it is related to Network Functions or to UEs) or the data that is able to collect from NFs.

### 5.8.2	Procedures related to NRF

#### 5.8.2.1	General

#### 5.8.2.2		NWDAF Registration in NRF

NWDAF registers in NRF as a regular network function. See TS 29.510 [23] for details.

#### 5.8.2.3	Consumer discovery and selection of NWDAF in NRF

A consumer of analytics services may use the NRF for discovering an NWDAF containing AnLF.

If the NWDAF service consumer needs to discover an NWDAF containing AnLF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF with the Nnrf_NFDiscovery_NFDiscover service operation, providing the NF Set IDs (serving-nf-set-id attribute) or NF types (serving-nf-type attribute) in the discovery request. See TS 29.510 [23] for details.

### 5.8.3	Procedures related to UDM

#### 5.8.3.1	General

#### 5.8.3.2		NWDAF containing AnLF Registration/Deregistration in UDM

##### 5.8.3.2.1	NWDAF containing AnLF Registration in UDM

For UE-related analytics, an NWDAF containing AnLF may register in UDM using the Nudm_UECM_Register service operation (see TS 29.503 [22] clause 5.3.2.2.8). This is useful especially in scenarios of co-location of the NWDAF containing AnLF with other network functions (e.g., AMF, SMF) and in certain deployment models. Figure 5.8.3.2.1-1 illustrates the signalling flow.

Figure 5.8.3.2.1-1: NWDAF containing AnLF registration in UDM

1.	NWDAF containing AnLF triggers a registration in UDM, e.g. based on local configuration in the NWDAF containing AnLF, the reception of a new Analytics subscription request, start of collection of UE related data or an OAM configuration action.

2.	The NWDAF containing AnLF registers into UDM for the served UE, by sending Nudm_UECM_Registration request (UE ID of the served user, NWDAF Instance ID, NF Set ID, Analytics ID(s)).

3.	UDM sends a response to NWDAF containing AnLF. A successful response consists of a "200 OK" or "201 Created". If the operation is unsuccessful, UDM responds with proper error response code. See TS 29.503 [22] for details.

##### 5.8.3.2.2	NWDAF containing AnLF Update of Registration in UDM

If an NWDAF containing AnLF has previously registered in UDM for one or more analytics IDs for a given UE, the NWDAF containing AnLF may modify the registration at any time. This may be due to, e.g., the NWDAF containing AnLF adds one or more Analytic IDs for the UE, or the NWDAF containing AnLF removes one or more Analytics IDs, but keep registered in UDM for at least one Analytic ID. The NWDAF containing AnLF sends an Nudm_UECM_Update service operation (NWDAF registration ID, added or removed Analytics ID(s)), see TS 29.503 [22] clause 5.3.2.6.4. Figure 5.8.3.2.2-1 illustrates the signalling flow.

Figure 5.8.3.2.2-1: NWDAF containing AnLF registration update in UDM

1.	If an NWDAF containing AnLF has previously registered one or more Analytics IDs for a UE in UDM, the NWDAF containing AnLF triggers an update of the registration in UDM, e.g. based on the NWDAF containing AnLF serving the UE of one or more additional Analytic ID(s) or based on the NWDAF containing AnLF not serving the UE for one or more Analytic ID(s) but serving the UE for at least one Analytic ID.

2.	The NWDAF containing AnLF updates its registration in UDM for the served UE, by sending Nudm_UECM_Update request (NWDAF Registration Id, added or removed Analytic ID(s)).

3.	Upon success UDM sends a "204 No Content" response to the NWDAF containing AnLF. If the operation is unsuccessful, UDM responds with proper error response code. See TS 29.503 [22] for details.

##### 5.8.3.2.3	NWDAF containing AnLF De-Registration in UDM

If an NWDAF containing AnLF has previously registered in UDM for one or more analytics ID for a given UE, when the NWDAF containing AnLF no longer serves the UE, e.g., it does not collect data for the UE for that Analytics ID or does no keep produce analytic reports or does not keep any data related to the UE, the NWDAF containing AnLF should de-register from UDM by invoking the Nudm_UECM_Deregistration service operation (NWDAF registration ID, Analytics ID(s)), see TS 29.503 [22] clause 5.3.2.4.8). Figure 5.8.3.2.3-1 illustrates the signalling flow.

Figure 5.8.3.2.3-1: NWDAF containing AnLF deregistration from UDM

1.	If an NWDAF containing AnLF has previously registered one or more Analytics IDs for a UE in UDM, the NWDAF containing AnLF triggers a complete deregistration from UDM, e.g. based on the NWDAF containing AnLF not collecting data any longer for the UE, or the NWDAF containing AnLF not producing analytics reports for the UE, or the NWDAF containing AnLF not keeping analytics data for the UE.

2.	The NWDAF containing AnLF deregisters from UDM for the served UE, by sending Nudm_UECM_Deregistration request (NWDAF Registration Id).

3.	UDM sends a "204 No Content" response to the NWDAF containing AnLF. See TS 29.503 [22] for details.

#### 5.8.3.3	Consumer discovery and selection of NWDAF containing AnLF in UDM

For UE related analytics, if the NWDAF service consumer needs to discover an NWDAF containing AnLF that is serving or holds data for a UE, the NWDAF serving consumer may query UDM with the target SUPI and the Analytics ID. If the response from UDM indicates multiple candidate NWDAFs, the NWDAF service consumer may, e.g., determine an instance of an NWDAF containing AnLF that has registered in UDM (registrationTime attribute) around the time of interest of the data or analytics, and may also query NRF, as specified in clause 5.8.2.3 in order to determine the capabilities of these NWDAFs and select the most appropriate one. Figure 5.8.3.3-1 illustrates the signalling flow.

Figure 5.8.3.3-1: Discovery of NWDAF containing AnLF in UDM

1.	The NWDAF service consumer sends an Nudm_UECM_Get request to the UDM (UE ID, Analytics ID(s), NF type = NWDAF).

2.	The NWDAF containing AnLF deregisters from UDM for the served UE, by sending Nudm_UECM_Deregistration request (NWDAF Registration Id).

3.	Upon success, UDM sends a "200 OK" response to the service consumer including the NWDAF information registered for the UE ID. If the operation is unsuccessful, UDM responds with proper error response code. See TS 29.503 [22] for details.

### 5.8.4	Procedures for PCF learning NWDAF IDs for served UEs

PCF may receive from AMF and SMF the IDs of the NWDAFs containing AnLF that are used by AMF, SMF, and UPF, so that the PCF may select the same NWDAF containing AnLF being used for a specific UE.

When the AMF creates or updates an Access and Mobility Policy Association for a UE, the AMF may include the IDs of the NWDAF containing AnLF that are used by the AMF, along with their Analytic ID(s), in the Npcf_AMPolicyControl_Create/Update service operations, as illustrated in Figure 5.8.4-1. See details in TS 29.507 [24].

Figure 5.8.4-1: PCF leaning from AMF NWDAFs containing AnLF

The PCF may select those NWDAF instances discovered through the Npcf_AMPolicyControl_Create/Update request as the ones to use for their associated analytic ID(s) for the UE for which those AM Policy Associations are related to.

When the SMF creates or updates a Session Management Policy Association for a UE, the SMF may include the IDs of the NWDAF containing AnLF that are used by the SMF and UPF, along with their Analytic ID(s), in the Npcf_SMPolicyControl_Create/Update service operations, as illustrated in Figure 5.8.4-2. See details in TS 29.512 [25].

Figure 5.8.4-2: PCF leaning from SMF and UPF NWDAFs containing AnLF

The PCF may select those NWDAF instances discovered through the Npcf_SMPoliycControl_Create/Update request as the ones to subscribe for their associated analytic ID(s) for the UE for which those SM Policy Associations are related to.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | CT3#115e |  |  |  |  | TS skeleton for TS 29.552. | 0.0.0 |
| 2021-04 | CT3#115e |  |  |  |  | Inclusion of documents agreed in CT3#115e: C3-212120, C3-212389 | 0.1.0 |
| 2021-05 | CT3#116e |  |  |  |  | Inclusion of document agreed in CT3#116e: C3-213373 | 0.2.0 |
| 2021-08 | CT3#117e |  |  |  |  | Inclusion of document agreed in CT3#117e: C3-214475, C3-214476, C3-214162 | 0.3.0 |
| 2021-10 | CT3#118e |  |  |  |  | Inclusion of document agreed in CT3#118e: C3-215481, C3-215482, C3-215220, C3-215221. | 0.4.0 |
| 2021-11 | CT3#119e |  |  |  |  | Inclusion of document agreed in CT3#119e: C3-216463,C3-216428,C3-216241,C3-216242,C3-216243,C3-216244,C3-216464 | 0.5.0 |