| 3GPP TR 29.866 V0.1.0 (2023-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on IMS Disaster Prevention and Restoration Enhancement (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Baseline	7

5	Key Issues	7

5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload	8

5.1.1	Description of the use case	8

5.1.2	Key issue definition	8

5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users	8

5.2.1	Description of the use case	8

5.2.2	Key issue definition	8

5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails	9

5.3.2	Key issue definition	9

6	Solutions	9

6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload	9

6.1.1	Description	9

6.1.1.1	General	9

6.1.1.2	(Re)-registration procedure for routing without I-CSCF	10

6.1.1.3	(Re)-registration procedure for routing with I-CSCF	10

6.1.2	Impacts on services, entities and interfaces	11

6.1.2.1	Routing without I-CSCF	11

6.1.2.2	Routing with I-CSCF	11

6.1.3	Pros	12

6.1.4	Cons	12

6.2	Solution#2: TBD	12

6.2.1	Description	12

6.2.2	Impacts on services, entities and interfaces	12

6.2.3	Pros	12

6.2.4	Cons	12

7	Evaluations	12

7.1	Evaluation of Solutions for Key Issue#1	12

7.2	Evaluation of Solutions for Key Issue#2	12

8	Conclusions	12

8.1	Conclusion of Solutions for Key Issue#1	13

8.2	Conclusion of Solutions for Key Issue#2	13

Annex A (informative):  Change history	14

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This Technical Report aims to study potential enhancements on IMS Disaster Prevention and Restoration, to increase the stability of the IMS system.

# 1	Scope

The present document identifies IMS restoration mechanisms as specified in 3GPP TS 23.380 [2] and studies potential optimization solutions. The document will focus on the following aspects:

-	Create/enhance corresponding the restoration mechanism to support the normal use of voice, video and SMS services in the IMS network. Specifically, it adopts the context data of registered online users stored by HSS/UDM, S-CSCF, P-CSCF and MMTEL to bypass the involved faulty network functions or links. That supports the following procedures:

-	Re-registration procedure for registered online users;

-	Mobile terminated procedure for registered online users;

-	EPS fallback procedure for registered online users;

-	Initial registration procedure for registered online users after deregistration.

-	Create/enhance corresponding network disaster-prevention mechanisms, including:

-	UE specific registration analysis mechanism, to prevent registration with missing parameter or incorrect parameters repeatedly.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.380: "IMS Restoration Procedures".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 23.007: "Restoration procedures".

[5]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents ".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

AV	Authentication Vector

AVP	Attribute-Value Pair

CSCF	Call Server Control Function

HSS	Home Subscriber Server

I-CSCF	Interrogating-CSCF

IMS	IP Multimedia Subsystem

MAA	Multimedia-Auth-Answer

MAR	Multimedia-Auth-Request

MMTEL	Multimedia Telephony

P-CSCF	Proxy-CSCF

S-CSCF	Serving CSCF

SAA	Server-Assignment-Answer

SAR	Server-Assignment-Request

UDA	User Data Answer

UDM	Unified Data Management

UDR	User Data Request

# 4	Baseline

TS 23.380 [2] has specified the procedures required in IMS to handle a S-CSCF or a P-CSCF service interruption scenario with minimum impact to the service to the end user. But IMS Restoration Procedures covering service interruption of other network elements (e.g. UDM, DRA, etc.) are not defined. So it is proposed to create/enhance corresponding restoration mechanism to network elements in the IMS network, to enhance the stability of IMS system.

Besides this, UE registration with missing parameter or incorrect parameters repeatedly may also lead to HSS/UDM overload, and thus make troubles to IMS. So it is proposed to create/enhance UE specific registration analysis for these abnormal behaviours and help to prevent IMS disasters.

# 5	Key Issues

This clause describes the issues that needs to be considered for IMS Disaster Prevention and Restoration Enhancement.

## 5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload

### 5.1.1	Description of the use case

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF / S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, P-CSCF/S-CSCF restoration procedures via HSS increases the HSS load which may further cause HSS overload.

HSS/UDM stores authentication data and subscription data of IMS subscribers. When one HSS/UDM instance fails, other HSS/UDM instances in the same set could provide services to I-/S-CSCF and IMS AS. In this case, there is no impact on IMS services. But once all the HSS/UDM instances in the set are unavailable, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results the service failure. Similar situation happens to HSS/UDM overload scenario, since at that time the HSS/UDM is hard to handle the related procedures.

Even if HSS/UDM failure or overload, operators have strong requirement to continue IMS service for those users already registered to IMS, so that to minimize the service interruption due to HSS/UDM failure or overload. How to enhance existing procedure to support the operator requirement should be considered.

### 5.1.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all HSS/UDM instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users

### 5.2.1	Description of the use case

The reliability of PS domain is the prerequisite of the availability of IMS voice, video and SMS service. In 4G network, CSFB solution could be used to provide voice call service for subscribers when EPC network failure causes that VoLTE is unavailable. But CSFB solution could not be used anymore after 2/3G sunset. Therefore, it’s important to study how to minimize the impact to IMS services when some network function in EPC/5GC network fails, especially when all network functions of this kind are faulty.

PCRF/PCF provides QoS guarantee to IMS services, which are defined in 3GPP TS 23.503 [7]. When one PCRF/PCF instance fails, the other PCRF/PCF instances in the set could provide services to P-CSCF and there is no impact on IMS services. But once all the PCRF/PCF instances in the set are unavailable, the dedicated bearers of voice/video media could not be established which causes that all the newly initiated calls fail. This key issue will study the solution addressing this case.

### 5.2.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all PCRF/PCF instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails

### 5.3.1	Description of the use case

In most of EPC/5GC NF failure scenarios, the UE could invoke re-attach and re-registration procedures quickly if

-	the UE initiates a new IMS session and some SIP error code is sent by the IMS network; or

-	the EPC/5GC network detects the failure and kicks off the related UEs to re-attach.

Then the UE’s service will be restored soon.

But in some cases, the re-attach and re-registration procedures will not be invoked immediately. In that case, the UE’s terminating session will be unavailable until its IMS registration is expired and it invokes the re-registration procedure. If the UE’s registration expiration time is 6000 seconds, the unavailable time is 50 minutes in the worst case. It’s important to find a solution to speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

### 5.3.2	Key issue definition

This key issue will study the following aspects:

-	Specify the mechanism that speeds up the restoration procedure of the terminating session when EPC/5GC NF fails.

NOTE:	The mechanism to speed up the restoration procedure should not impact the UE and introduce overload of EPC/5GC.

# 6	Solutions

This clause describes the solutions for key issues.

## 6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload

### 6.1.1	Description

#### 6.1.1.1	General

The solution is to address the Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload.

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF/S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, the existing P-CSCF/S-CSCF restoration procedures via HSS/UDM increases the HSS/UDM load which may further cause HSS/UDM overload.

In the case of HSS/UDM failure, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results in the service failure.

This solution considers to continue IMS service for registered UE, in the following procedures:

-	(re)-registration procedure for routing without I-CSCF;

-	(re)-registration procedure for routing with I-CSCF;

Selection of routing without I-CSCF or routing with I-CSCF in the scenario of HSS/UDM failure or overload is based on operation policy.

#### 6.1.1.2	(Re)-registration procedure for routing without I-CSCF

The below Figure 6.1.1.2-1 illustrates the Re-registration procedure for routing without I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.2-1: Re-registration procedure for routing without I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, it forwards the re-registration message directly to the S-CSCF.

3-4. Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

5-6. According to the strategy in 3GPP TS 23.228 [3], step 5-6 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS/UDM and return 200 OK to the UE.

7-8. The S-CSCF returns 200 OK to the UE.

#### 6.1.1.3	(Re)-registration procedure for routing with I-CSCF

The below Figure 6.1.1.3-1 illustrates the Re-registration procedure for routing with I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.3-1: Re-registration procedure for routing with I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, the P-CSCF adds the address of S-CSCF to a new header field and forwards the re-registration message to the I-CSCF.

3.	The I-CSCF identifies the new header field within the message. If it presents, I-CSCF forwards the re-registration message to the S-CSCF address obtained from the new header field.

4-5.	Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

6-7. According to the strategy in 3GPP TS 23.228 [3], step 6-7 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS and return 200 OK to the UE.

8-10. The S-CSCF returns 200 OK to the UE.

### 6.1.2	Impacts on services, entities and interfaces

If HSS/UDM failure or overload is detected, some enhancements are needed for the optimization.

#### 6.1.2.1	Routing without I-CSCF

The requirements for the modification of P-CSCF:

-	Add functionality for directly routing re-registration messages to S-CSCF.

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

#### 6.1.2.2	Routing with I-CSCF

The requirements for the modification of P-CSCF:

-	Including the address of S-CSCF into a new header field.

The requirements for the modification of I-CSCF:

-	Identifying the new header field, if included, forwarding the re-registration message to the S-CSCF address obtained from the new header field;

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

### 6.1.3	Pros

This solution provides different efficient ways to bypass the HSS/UDM if HSS/UDM failure or overload is detected.

### 6.1.4	Cons

The cons of the solution:

-	Enhancements to P-CSCF, I-CSCF and S-CSCF are required.

-	The method of routing without I-CSCF deviates from the signaling flows specified in 3GPP TS 23.228 [3], so this approach needs to be discussed in Stage 2

## 6.2	Solution#2: TBD

### 6.2.1	Description

### 6.2.2	Impacts on services, entities and interfaces

### 6.2.3	Pros

### 6.2.4	Cons

# 7	Evaluations

This clause describes evaluations of the solutions.

## 7.1	Evaluation of Solutions for Key Issue#1

## 7.2	Evaluation of Solutions for Key Issue#2

# 8	Conclusions

This clause describes conclusions.

## 8.1	Conclusion of Solutions for Key Issue#1

## 8.2	Conclusion of Solutions for Key Issue#2

Annex A (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | CT4#118 | C4-234587 | - | - | - | Implementation of C4-234059, C4-234525, C4-234526, C4-234527, C4-234529, C4-234569, C4-234570, C4-234571 and C4-234577 in CT4#118 | 0.1.0 |