Annex A (informative):
Connection Diagrams

Definition of Terms

System Simulator or SS – A device or system, that is capable of generating simulated Node B signalling and analysing UE signalling responses on one or more RF channels, in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Measurement and control of the UE Tx output power through TPC commands

2.	Measurement of Throughput

3.	Measurement of signalling timing and delays

4.	Ability to simulate UTRAN and/or E-UTRAN and/or GERAN signalling

GNSS System Simulator or GSS: A device or system, that is capable of generating simulated GNSS satellite transmissions in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual satellites and the simulation of atmospheric delays.

2.	Ability to synchronize with E-UTRAN timing in the SS.

Test System – A combination of devices brought together into a system for the purpose of making one or more measurements on a UE in accordance with the test case requirements. A test system may include one or more System Simulators if additional signalling is required for the test case. The following diagrams are all examples of Test Systems.

NOTE 1:	The above terms are logical definitions to be used to describe the test methods used in the documents TS36.521-1, TS 36.523-1 and TS36.521-3 in practice, real devices called 'System Simulators' may also include additional measurement capabilities or may only support those features required for the test cases they are designed to perform.

NOTE 2:	Components in the connection diagrams:
The components in the connection diagrams represent ideal components. They are intended to display the wanted signal flow. They don’t mandate real implementations. An alternative to Figure A3 is shown below as an example: It is nearer to real implementations. The signal levels are the same as in Figure A3. The signal flow cannot be displayed as detailed as in Figure A.3.

Alternative to Figure A.3

Connection: Each connection is displayed as a one or two sided arrow, showing the intended signal flow. In some cases, for some tests, some connections shown may not be necessary (for example UL RX connection for a second cell).

Circulator: The signal, entering one port, is conducted to the adjacent port, indicated by the arrow. The attenuation among the above mentioned ports is ideally 0 and the isolation among the other ports is ideally ∞.

Splitter: a splitter has one input and 2 or more outputs. The signal at the input is equally divided to the outputs. The attenuation from input to the outputs is ideally 0 and the isolation between the outputs is ideally ∞.

Combiner: a combiner has one output and 2 or more inputs. The signals at the inputs are conducted to the output, all with the same, ideally 0 attenuation. The isolation between the inputs is ideally ∞.

Switch: contacts a sink (or source) alternatively to two or more sources (or sinks).

Fader: The fader has one input and one output. The MIMO fading channel is represented by several single faders (e.g. 8 in case of a MIMO antenna configuration 4x2) The correlation among the faders is described in TS 36.521-1 clause B.2.2. In some cases, for some tests, diagrams with fader(s) are referenced when no fading is required; in this case the fader(s) is omitted.

Attenuator: TBD

General considerations on connections for CA testing

Figures A.32<x> to A.37<x> and A.41<x> to A.46<x> contain connection diagrams for CA testing with 2 CCs. Different options (<x> = a, b, c) have been defined for each connection, to consider different UE employments with respect to UE antenna connectors and signal mapping:

-	a: Separate antenna connectors for each CC. An optional Combiner may be used in case the UE employs one common wide band antenna only for the receive diversity for both CC-s.

-	b: Common antenna connectors for both CC-s with same UL transmit antenna connector.

-	c: Common antenna connectors for both CC-s with different UL transmit antenna connectors.

The connections are referred in tests as Figure group which imply figures with the same Arabic numeral in the number, but different Latin letter extension (e.g. Figure A.32a, A.32b etc.). The selection of the connection option for testing is done according to the UE employment as appropriate.

Connection diagrams for CA testing with ≥3CCs have been defined in other figures.

-	The connections represent the case of a DUT with separate antenna connectors per CC. For other DUT antenna connector configurations, the necessary signal combining is done accordingly.

-	The number of secondary CC is n with n = 2, 3, ... as applicable to the individual test case for n+1 CCs testing.

Following symbolic is used in the connection diagrams to distinguish CC-s and paths:

-	p: primary CC

-	s: secondary CC (in case of CA with 2CCs)

-	s1: first secondary CC (in case of CA with >2CCs)

-	 sn: n-th secondary CC with n = 2, 3, ... (in case of CA with >2CCs)

-	<j>: (integer) defines the receive antenna and the path to it <j> for a given CC

-	<i><j>: (integer) defines the path between transmit antenna <i> and receive antenna <j> for a given CC

The connections are general and support CA both in DL and UL. According to the test conditions, if the UL CA is not used, the "secondary CCs of UL" in the connections may be ignored.

For contiguous CA, single AWGN_k sources with k = {p, s, s1, s2, ...} affecting the same UE antenna connector can be unified to AWGN sources with higher bandwidth.

Connections for UE category 0 and category M1 testing

For UE category 0 and category M1, if the connection is referred with the additional clarification “using only main UE Tx/Rx antenna”, then all the links and blocks in the connection serving to supply other UE Rx antennas with signal, are not used.

The example below in Figure A.0 shows the resulting connection to be used, after removing the unnecessary parts (red coloured) to connection of Figure A.10, assuming it is referred in a UE category 0 or a category M1 test as mentioned above.

Connections for ProSe testing

TBD

Connections for 4 Rx antenna ports in transmitter tests

For UE supporting 4 Rx antenna ports on at least one of the bands under test, in transmitter tests the connection is referred to connection diagrams showing only 2 UE Rx antenna ports. For such cases, the SS fed only two of the four antennas from the UE, which are connected depending on UE capability. The remaining two antenna ports of the UE are not connected to the SS.

Considerations for CA testing on 4Rx capable UEs

Figures A.80, A.82, A.85 to A.87, A.90, A.91, A.94 and A.95 contain connection diagrams that shall be used to enable CA testing on 4Rx capable UEs. Such connection diagrams are only applicable to UEs that support 4Rx on at least one of the bands under test. In such cases, the following rules apply to the connection diagram:

1	For nDL CA with x CCs on a 4Rx supported band and (n-x) CCs on a 2Rx supported band, the 4Rx Module shall be replicated x times and 2Rx Module shall be replicated (n-x) times. These modules are specified on the connection diagrams.

2	UE is assumed to support 2Rx in {Rx1, Rx3} and 4Rx in {Rx1, Rx2, Rx3, Rx4}. Depending on UE capability these ports can be interchanged.

3	For all 2Rx supported band the 2-Rx CCs  are fed only on two ports and the remaining two ports have zero-input in this band.

4	SS shall configure Rx1, Rx2, Rx3, Rx4 based on UE capability.

5	UE Tx will be placed in one of the 4 UE antenna ports and shall feed either SS_4Rx or SS_2Rx depending on UE capability. SS_4Rx and SS_2Rx are specified on the connection diagrams.

The connections represent the case of a DUT with common antenna connectors for both CC-s with same UL transmit antenna connector. For other DUT antenna connector configurations, the necessary signal combining is done accordingly.

Figure A.0: Reduced implementation of connections for UE category 0 and category M1 testing 
(example of Figure A.10)

Figure A.1: Void

Figure A.2: Connection for Transmitter Intermodulation tests

Figure A.3: Connection for basic single cell, RX and TX tests

Figure A.4: Connection for Receiver tests with E-UTRA-Interference

Figure A.5: Connection for Receiver tests with CW interferer

Figure A.6: Connection for Receiver tests with both E-UTRA Interference and additional CW signal

Figure A.7: Connection for TX-tests with additional Spectrum Analyzer

Figure A.8: Connection for RX-tests with additional Spectrum Analyzer

Figure A.8a: Connection for RX tests for CA (≥ 2CCs) with additional Spectrum Analyzer

Figure A.9: Connection for RX performance tests with antenna configuration 1x2 (single antenna port)

Figure A.9a: Connection for RX performance tests with antenna configuration 1x2 (single antenna port) for 4Rx capable UEs without any 2Rx RF bands

Figure A.10: Connection for RX performance tests with antenna configuration 2x2

Figure A.10a: Connection for RX performance tests with antenna configuration 2x2 for 4Rx capable UEs without any 2Rx RF bands

Figure A.11: Connection for RX performance tests with antenna configuration 4x2

Figure A.11a: Connection for RX performance tests with antenna configuration 4x2 for 4Rx capable UEs without any 2Rx RF bands

Figure A.12: Void

Figure A.13: Void

Figure A.14: Connection for 2 cells with static propagation and receive diversity

Figure A.15: Connection for 2 cells with multipath fading propagation and receive diversity

Figure A.16: Connection for single cell Signalling tests
(Note: only one Rx/Tx antenna port in case of NB-IoT UE)

Figure A.16a: Connection for single cell Signalling tests for 4Rx Capable UE

Figure A.17: Connection for multiple cells Signalling tests
(Note: only one Rx/Tx antenna port in case of NB-IoT UE)

Figure A.17a: Connection for multiple cells Signalling tests for 4Rx capable UE

Figure A.18: Connection for 1 cell with antenna configuration 1x2 in static propagation conditions

Figure A.19: Connection for 3 cells with antenna configuration 1x2 in static (cell 1) and multipath fading (cell 2 and 3) propagation conditions and receive diversity

Figure A.20: Connection for 2 cells with static propagation and receive diversity with phase rotator (The frequency offset used in phase rotator is 5 Hz)

Figure A.21: Connection for frequency-selective interference with multipath fading propagation and receive diversity

Figure A.21a: Connection for frequency-selective interference with multipath fading propagation and receive diversity for 4Rx capable UEs without any 2Rx RF bands

Figure A.22: Connection for 2 cells with static propagation condition. Origin (E-UTRAN) and target cell (≠ E-UTRAN) received with RX diversity

Figure A.23: Connection for 2 cells with multipath fading propagation condition. Origin (E-UTRAN) and target cell (≠ E-UTRAN) received with RX diversity

Figure A.24: Connection for 3 cells with antenna configuration 1x2 in static (cell 1) and multipath fading conditions (cell 2 and 3) and multiple RAT and receive diversity

Figure A.25: Connection for 3 cells with antenna configuration 1x2 in multipath fading (cell 1 and 2) and multiple RAT (cell 3 static) and receive diversity

Figure A.26: Connection for 2 cells. Origin (E-UTRAN, multipath fading) and target cell (≠ E-UTRAN, static) received with RX diversity

Figure A.27: Connection for 3 cells with static propagation and receive diversity

Figure A.27a: Connection for 3 cells with static propagation and 1x4 receive diversity

Figure A.28: Connection for basic UL MIMO with receive diversity

Figure A.29: Connection for UL MIMO Receiver tests with E-UTRA-Interference

Figure A.30: Connection for UL MIMO Receiver tests with CW Interference

Figure A.31: Connection for UL MIMO Receiver tests with both E-UTRA-Interference and additional CW signal

Figure group A.32: Connections for basic Tx and Rx tests for CA

Figure A.32a: Connection for basic Tx and Rx tests for CA (separate connectors)

Figure A.32b: Connection for basic Tx and Rx tests for CA (common connectors, same UL antenna)

Figure A.32c: Connection for basic Tx and Rx tests for CA (common connectors, different UL antennas)

Figure group A.33: Connection for Tx tests for CA with additional Spectrum Analyzer

Figure A.33a: Connection for Tx tests for CA with additional Spectrum Analyzer (separate connectors)

Figure A.33b: Connection for Tx tests for CA with additional Spectrum Analyzer (common connectors, same UL antenna)

Figure A.33c: Connection for Tx tests for CA with additional Spectrum Analyzer (common connectors, different UL antennas)

Figure group A.34: Connection for Rx tests for CA with additional Interferer / CW

Figure A.34a: Connection for Rx tests for CA with additional Interferer / CW (separate connectors)

Figure A.34b: Connection for Rx tests for CA with additional Interferer / CW (common connectors, same UL antenna)

Figure A.34c: Connection for Rx tests for CA with additional Interferer / CW (common connectors, different UL antennas)

Figure group A.35: Connection for Rx performance tests for CA with antenna configuration 1x2

Figure A.35a: Connection for Rx performance tests for CA with antenna configuration 1x2 (separate connectors)

Figure A.35b: Connection for Rx performance tests for CA with antenna configuration 1x2 (common connectors, same UL antenna)

Figure A.35c: Connection for Rx performance tests for CA with antenna configuration 1x2 (common connectors, different UL antennas)

Figure group A.36: Connection for Rx performance tests for CA with antenna configuration 2x2

Figure A.36a: Connection for Rx performance tests for CA with antenna configuration 2x2 (separate connectors)

Figure A.36b: Connection for Rx performance tests for CA with antenna configuration 2x2 (common connectors, same UL antenna)

Figure A.36c: Connection for Rx performance tests for CA with antenna configuration 2x2 (common connectors, different UL antennas)

Figure group A.37: Connection for Tx tests for CA with additional CW and Spectrum Analyzer

Figure A.37a: Connection for Tx tests for CA with additional CW and Spectrum Analyzer (separate connectors)

Figure A.37b: Connection for Tx tests for CA with additional CW and Spectrum Analyzer (common connectors, same UL antenna)

Figure A.37c: Connection for Tx tests for CA with additional CW and Spectrum Analyzer (common connectors, different UL antennas)

Figure A.38: Connection for UL MIMO tests with additional Spectrum Analyzer

Figure A.39: Connection for Transmitter Intermodulation tests for UL-MIMO

Figure A.40: Connection for 2 cells, antenna configuration 2x2, multipath fading and receive diversity

Figure A.40a: Connection for 2 cells, antenna configuration 2x2, multipath fading and receive diversity for 4Rx capable UEs without any 2Rx RF bands

Figure group A.41: Connection for CA with additional intra-frequency cell and static propagation channel

Figure A.41a: Connection for CA with additional intra-frequency cell and static propagation channel (separate connectors) (The frequency offset used in phase rotator is 5 Hz)

Figure A.41b: Connection for CA with additional intra-frequency cell and static propagation channel (common connectors, same UL antenna) (The frequency offset used in phase rotator is 5 Hz)

Figure A.41c: Connection for CA with additional intra-frequency cell and static propagation channel (common connectors, different UL antennas) (The frequency offset used in phase rotator is 5 Hz)

Figure group A.42: Connection for CA with additional intra-frequency cell and multipath fading propagation

Figure A.42a: Connection for CA with additional intra-frequency cell and multipath fading propagation (separate connectors)

Figure A.42b: Connection for CA with additional intra-frequency cell and multipath fading propagation (common connectors, same UL antenna)

Figure A.42c: Connection for CA with additional intra-frequency cell and multipath fading propagation (common connectors, different UL antennas)

Figure group A.43: Connection for CA with additional inter-frequency cell and multipath fading propagation

Figure A.43a: Connection for CA with additional inter-frequency cell and multipath fading propagation (separate connectors)

Figure A.43b: Connection for CA with additional inter-frequency cell and multipath fading propagation (common connectors, same UL antenna)

Figure A.43c: Connection for CA with additional inter-frequency cell and multipath fading propagation (common connectors, different UL antennas)

Figure group A.44: Connection for CA with additional inter-RAT cell and multipath fading propagation

Figure A.44a: Connection for CA with additional inter-RAT cell and multipath fading propagation (separate connectors)

Figure A.44b: Connection for CA with additional inter-RAT cell and multipath fading propagation (common connectors, same UL antenna)

Figure A.44c: Connection for CA with additional inter-RAT cell and multipath fading propagation (common connectors, different UL antennas)

Figure group A.45: Connection for 2 cells with antenna configuration 1x2 and static propagation

Figure A.45a: Connection for 2 cells with antenna configuration 1x2 and static propagation (separate connectors)

Figure A.45b: Connection for 2 cells with antenna configuration 1x2 and static propagation (common connectors, same UL antenna)

Figure A.45c: Connection for 2 cells with antenna configuration 1x2 and static propagation (common connectors, different UL antennas)

Figure group A.46: Connection for Rx performance tests for CA with antenna configuration 4x2

Figure A.46a: Connection for Rx performance tests for CA with antenna configuration 4x2 (separate connectors)

Figure A.46b: Connection for Rx performance tests for CA with antenna configuration 4x2 (common connectors, same UL antenna)

Figure A.46c: Connection for Rx performance tests for CA with antenna configuration 4x2 (common connectors, different UL antennas)

Figure A.47: Connection for RX performance tests with antenna configuration 8x2

Figure A.47a: Connection for RX performance tests with antenna configuration 8x2 for 4Rx capable UEs without any 2Rx RF bands

Figure A.48: Connection for 3 Cells (TPs) with antenna configuration 2x2 and fading

Figure A.49: Connection for 2 Cells (TPs) with antenna configuration 4x2 and fading

Figure A.50: Connection for 2 Cells (TPs) with antenna configuration 1x2 and fading for one Cell (TP)

Figure A.51: Connection for 2 Cells (TPs) with antenna configuration 2x2 / 1x2 and fading for the 2x2 Cell (TP)

Figure A.51a: Connection for 3 Cells (TPs) with antenna configuration 2x2 / 1x2 / 1x2 and fading for the 2x2 Cell (TP)

Figure A.52: Connection for 2 Cells (TPs) with antenna configuration 4x2 / 2x2 and fading

Figure A.52a: Connection for 2 Cells (TPs) with antenna configuration 4x2 / 2x2 and fading for 4Rx capable UEs without any 2Rx RF bands

Figure A.53: Connection for 2 Cells (TPs) with antenna configuration 8x2 / 2x2 and fading

Figure A.53a: Connection for 2 Cells (TPs) with antenna configuration 8x2 / 2x2 and fading for 4Rx capable UEs without any 2Rx RF bands

Figure A.54: Connection for 3 or more Cells (TPs) with antenna configuration 1x2 and fading

Figure A.55: Connection for basic Tx and Rx tests for CA (≥ 3CCs)

Figure A.56: Connection for Tx tests for CA (≥ 3CCs) with additional Spectrum Analyzer

Figure A.57: Connection for Tx tests for CA (≥ 3CCs) with additional CW and Spectrum Analyzer

Figure A.58: Connection for Rx tests for CA (≥ 3CCs) with additional Interferer / CW

Figure A.59: Connection for Rx performance tests for CA (≥ 3CCs) with antenna configuration 1x2

Figure A.60: Connection for Rx performance tests for CA (≥ 3CCs) with antenna configuration 2x2

Figure A.61: Connection for Rx performance tests for CA (≥ 3CCs) with antenna configuration 4x2

Figure A.62: Connection for 2 Cells with antenna configuration 2x2 / 1x2 and fading

Figure A.63: Connection for CA testing (≥ 3CCs) with antenna configuration 1x2 and static propagation conditions

Figure A.64: Connection for CA(≥ 3CCs) with static PCC propagation channel, SCCs multipath fading propagation channels and additional intra-frequency cell on one SCC

Figure A.64a: Void

Figure A.65: Connection for 3DL CA with static propagation channels and additional intra-frequency cell on one SCC (the frequency offset used in phase rotator is 5 Hz)

Figure A.66: Connection for 3DL CA with static propagation channels and additional intra-frequency cell(s) on both SCCs (the frequency offset used in phase rotators is 5 Hz)

Figure A.66a: Connection for more than 3DL CA with static propagation channels and additional intra-frequency cell(s) on each SCC (the frequency offset used in phase rotators is 5 Hz)

Figure A.67: Connection for Sidelink basic Tx/Rx testing

Figure A.68: Connection for 3DL CA with PCC and SCCs multipath fading propagation channels and additional intra-frequency cell on one SCC

Figure A.69: Connection for Receiver test with 4 Rx antenna ports

Figure A.70: Connection for Sidelink Receiver tests with E-UTRA-Interference

Figure A.71: Connection for Sidelink Receiver tests with CW interferer

Figure A.72: Connection for Sidelink Receiver tests with both E-UTRA Interference and additional CW interferer

Figure A.73: Connection for 2 Cells (TPs) with antenna configuration 2x4 and fading

Figure A.74: Connection for RX performance tests with antenna configuration 4x4

Figure A.75: Connection for RX performance tests with antenna configuration 1x4

Figure A.76: Connection for RX performance tests with antenna configuration 2x4

Figure A.77: Connection for RX performance tests with antenna configuration 8x4

Figure A.78: Connection for 2 Cells (TPs) with antenna configuration 1x4 and fading

Figure A.78a: Connection for 2 Cells (TPs) with antenna configuration 1x4 and phase rotators (The frequency offset used in the phase rotators is 5, 10 and 15 Hz respectively)

Figure A.79: Connection for 2 Cells (TPs) with antenna configuration 2x4 / 1x4 and fading

Figure A.80: Connection to enable Receiver tests for CA on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported RF bands

Figure A.81: Connection for Receiver tests with interferer(s) and 4 Rx antenna ports

Figure A.82: Connection to enable Receiver tests for CA with additional Interferer on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported RF bands

Figure A.83: Connection for Receiver tests with 4 Rx antenna ports and additional spectrum analyzer

Figure A.84: Void

Figure A.84a: Void

Figure A.85: Connection Diagram to enable Carrier Aggregation tests (with 2x2 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands

Figure A.86: Connection Diagram to enable Carrier Aggregation tests (with 1x2 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands

Figure A.87: Connection Diagram to enable Carrier Aggregation tests (with 4x2 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands

Figure A.88: Connection for 3 Cells (TPs) with antenna configuration 4x2 / 2x2 / 2x2 and fading

Figure A.89: Connection for CA with Fading and additional intra-frequency cell for 4Rx antenna ports

Figure A.89a: Connection for V2X Communication / Intra-band contiguous multi-carrier operation (common connectors, same UL antenna)

Figure A.90: Connection for CA with Fading and additional Static intra-frequency cell with phase rotators for 4Rx antenna ports

Figure A.91: Connection for CA SDR tests on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported RF bands

Figure A.92: Connection for basic Tx/Rx testing of V2X sidelink transmissions non-concurrent with E-UTRAN uplink transmissions

Figure A.92a: Connection for basic Tx/Rx testing of V2X sidelink transmissions non-concurrent with E-UTRAN uplink transmission with E-UTRA-Interference

Figure A.92b: Connection for basic Tx/Rx testing of V2X sidelink transmissions non-concurrent with E-UTRAN uplink transmission with CW Interference

Figure A.92c: Connection for basic Tx/Rx testing of V2X sidelink transmissions non-concurrent with E-UTRAN uplink transmission with both E-UTRA Interference and additional CW signal

Figure A.93: Void

Figure A.92d: Connection for V2X Communication / Intra-band contiguous multi-carrier operation with additional Interferer / CW (common connectors, same UL antenna)

Figure A.93: Connection for RRM tests with antenna configuration 2x1 and 2 Cells

Figure A.93a: Connection for basic Tx/Rx testing of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission

Figure A.93b: Connection for V2X testing of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission with additional CW and Spectrum Analyzer

Figure A.93c: Connection for V2X testing of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission with additional Interferer / CW

Figure A.93d: Connection for V2X Tx testing of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission with additional Spectrum Analyzer

Figure A.93e: Connection for V2X Rx testing of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission with additional Spectrum Analyzer

Figure A.94: Connection Diagram to enable Carrier Aggregation tests (with 2x4 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands

Figure A.95: Connection Diagram to enable Carrier Aggregation tests (with 4x4 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands

Figure A.96: Connection for performance testing of V2V/V2X sidelink communications in 1x2 faded channel and non-concurrent with E-UTRAN uplink transmission

Figure A.97: Connection for performance decoding capability testing of V2V/V2X sidelink communications in 1x2 static channel and non-concurrent with E-UTRAN uplink transmission

Figure A.98: Connection for power imbalance performance testing of V2V sidelink communications with two links in 1x2 AWGN channel and non-concurrent with E-UTRAN uplink transmission

Figure A.99: Connection for more than 1 Cell with multipath fading propagation for 4Rx Capable UE

Figure A.100: Connection for RX performance tests with antenna configuration 12x2

Figure A.101: Connection for RX performance tests with antenna configuration 12x2 for 4Rx capable UEs without any 2Rx RF bands

Figure A.102: Connection for RX performance tests with antenna configuration 16x2

Figure A.103: Connection for RX performance tests with antenna configuration 16x2 for 4Rx capable UEs without any 2Rx RF bands

Annex B (informative):
Change history

| Meeting-1st-Level |  | Doc-1st-Level |  | CR |  | Rev |  | Subject |  | Cat |  | Version-Current |  | Version-New |  | Doc-2nd-Level |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAN5 #37 |  | R5-073107 |  |  |  |  |  | Skeleton proposed for RAN5#37 Jeju |  |  |  |  |  | 0.0.1 |  |  |  |
| RAN5 LTE workshop |  | R5w080007 |  |  |  |  |  | Proposed for RAN5 LTE workshop, Sophia Antipolis |  |  |  | 0.0.1 |  | 0.0.2 |  |  |  |
| RAN5 #39 |  | R5-081167 |  |  |  |  |  | Following proposals have been incorporated: R5w080046 R5w080026 R5w080036 |  |  |  | 0.0.2 |  | 0.0.3 |  |  |  |
| RAN5 #39 |  | R5-081615 |  |  |  |  |  | Following proposals and many editorial corrections have been incorporated: R5-081564, R5-081561, R5-081248, R5-081530, R5-081126, R5-081443, R5-081382, R5-081200 |  |  |  | 0.0.3 |  | 0.1.0 |  |  |  |
| RAN5 #39bis |  | R5-082141 |  |  |  |  |  | Following proposals and many editorial corrections have been incorporated: R5-082149, "Updates of reference test conditions for TS 36.508" R5-082148, "Addition of E-UTRA TDD Test frequencies for TS36.508" R5-082150, "Default downlink signal channel powers for LTE UE test" R5-082146, "Addition of Cell Environment for multi Cell Configuration" R5-082140, "Proposal of LTE reference system configurations for TS 36.508" R5-082204, "Addition of Cell and UE configuration for TS 36.508" R5-082090, "Update of default RRC message contents" R5-082100, "Proposal on Structure of Default Message Contents for TS 36.508" R5-082091, "Addition of SRB and DRB radio bearer combinations to 36.508" R5-082173, "Connection Diagrams for TX and RX tests" |  |  |  | 0.1.0 |  | 0.2.0 |  |  |  |
| RAN5 #40 |  | R5-083399 |  |  |  |  |  | Following proposals have been incorporated: R5-083800, "Mapping of DL physical channels to physical resources for TS 36.508", NEC R5-083403, "Addition of New Cell Environment for multi Cell Configuration", NTT DOCOMO R5-083529, "Proposal on default system information contents for TS 36.508", NTT DOCOMO R5-083395, "Corrections to generic procedures in TS 36.508", NTT DOCOMO R5-083623, "Update of RRC default message contents and RB combination parameters", Ericsson R5-083622, "Radio Resource Configuration specification for TS 36.508", NEC R5-083397, "Addition of Default NAS message contents in TS 36.508", NTT DOCOMO |  |  |  | 0.2.0 |  | 1.0.0 |  |  |  |
| RAN5 #40bis |  | R5-084102 |  |  |  |  |  | Following proposals have been incorporated: R5-084101, "Missing corrections to TS 36.508" R5-084110, "Updates of Test frequencies for TS 36.508" R5-084144, "The mapping of DL physical channels to physical resources for TS 36.508" R5-084198, "Update of RA and RB power ratios definition in TS 36.508" R5-084199, "Update of Reference System Configuration in 36.508" R5-084109, "Addition of default RRC message contents to TS 36.508" R5-084202, "Update of RRC Message Contents and RB Configurations in 36.508" R5-084265, "Addition of default RRC message contents for handover" R5-084162, "Connection Diagrams for performance tests" |  |  |  | 1.0.0 |  | 1.1.0 |  |  |  |
| RAN5 #41 |  | R5-085145 |  |  |  |  |  | Following proposals have been incorporated: R5-085087, "Updates of Test frequencies for TS 36.508" R5-085701, "Cleaning up section 5 in TS 36.508" R5-085252, "Correction to Section 4.3.3.2 of TS 36.508" R5-085315, "Connection Diagrams: delete the editorial note" R5-085454, "Addition of timer tolerances" R5-085566, "Addition of default settings of suitable / non-suitable cells in TS 36.508" R5-085541, "Update to default configurations of simulated cells in TS 36.508" R5-085514, "Update to default configurations of system information blocks in TS 36.508" R5-085472, "Addition of default settings of suitable / non-suitable cells in TS 36.508" R5-085394, "Update of Reference system configurations in 36.508" R5-085457, "Update to generic procedure in TS 36.508" R5-085458, "Addition of new generic procedure to check the UE does not answer to paging" R5-085523, "Update of default RRC message contents" R5-085381, "Addition to default RRC IE contents for measurement configuration" R5-085469, "Update to default NAS message contents in TS 36.508" R5-085451, "Parameter settings for reference RB configurations" R5-085556, "Common test USIM parameters for EPS testing" |  |  |  | 1.1.0 |  | 2.0.0 |  |  |  |
| RAN#42 |  | RP-085145 |  |  |  |  |  | Approval of version 2.0.0 at RAN#42, then updated to v 8.0.0. |  |  |  | 2.0.0 |  | 8.0.0 |  |  |  |
|  |  |  |  |  |  |  |  | Editorial corrections. |  |  |  | 8.0.0 |  | 8.0.1 |  |  |  |
| RAN5 #41bis |  | R5-086021 |  | 0001 |  | - |  | Introduction of half cell configurations in eUTRA SS |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086166 |  | 0002 |  | - |  | Removal of Redundant Environmental Conditions |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086221 |  | 0003 |  | - |  | CR to 36.508: correction of EARFCN |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086226 |  | 0004 |  | - |  | Correction to the default system informations in TS 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086236 |  | 0005 |  | - |  | Connection diagrams for RRM |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086346 |  | 0006 |  | - |  | Update of the default message AUTHENTICATION FAILURE |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086362 |  | 0007 |  | - |  | update of reference configuration systems for CDMA2000 in 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086363 |  | 0008 |  | - |  | Updated of common and default parameters for CDMA2000 cells |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086364 |  | 0009 |  | - |  | Update of SystemInformationBlockType8 in 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086369 |  | 0010 |  | - |  | Addition of reference EPS bearer contexts |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086370 |  | 0011 |  | - |  | Mapping of default DL Physical Channels for TDD in 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #41bis |  | R5-086400 |  | 0012 |  | - |  | Addition of RS_EPRE powers to default DL signal levels |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090084 |  | 0013 |  | - |  | Test procedure to verify that an EPS bearer context is active |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090362 |  | 0014 |  | - |  | Correction to the definition of simulated NAS cells in TS 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090464 |  | 0015 |  | - |  | Clean up the test algorithm for authentication |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090586 |  | 0016 |  | - |  | Add specific information elements for RRC reconfiguration |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090630 |  | 0017 |  | - |  | Introduction of alternative DRX configurations |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090681 |  | 0018 |  | - |  | Correction to the default NAS message contents in TS 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090682 |  | 0019 |  | - |  | Correction to the definition of simulated cells in TS 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090698 |  | 0020 |  | - |  | Update of 4.5 generic procedures in 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090699 |  | 0021 |  | - |  | TDD RTT correction for timer tolerance |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-090759 |  | 0022 |  | - |  | Correction to the default RRC message contents in TS 36.508 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-091000 |  | 0023 |  | - |  | Correction to clause 4.3.3.3 |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #42 |  | R5-091001 |  | 0024 |  | - |  | LTE-RF: Clarification to 36.508 Simulated Cells for RF tests |  |  |  | 8.0.1 |  | 8.1.0 |  |  |  |
| RAN5 #43 |  | RP-090447 |  | 0025 |  | - |  | Correction to Cell off power |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092086 |  |
| RAN5 #43 |  | RP-090447 |  | 0026 |  | - |  | LTE Signalling Tests: UE Rx antenna connection |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092087 |  |
| RAN5 #43 |  | RP-090448 |  | 0027 |  | - |  | CR to 36.508 for subclause 4.3.1 channel bandwidth clarification for RF tests (re-submit no changes) |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092124 |  |
| RAN5 #43 |  | RP-090448 |  | 0028 |  | - |  | Text for 4.2.2: Minimum functional requirements |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092128 |  |
| RAN5 #43 |  | RP-090448 |  | 0029 |  | - |  | Annex A: transition from 1 to 2 RX antenna |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092132 |  |
| RAN5 #43 |  | RP-090447 |  | 0030 |  | - |  | Update of SN length in UM RLC default configuration |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092202 |  |
| RAN5 #43 |  | RP-090448 |  | 0031 |  | - |  | TP for simulated UTRA TDD cell parameter |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092275 |  |
| RAN5 #43 |  | RP-090447 |  | 0032 |  | - |  | Correction to specific message contents in setup procedure in TS 36.508 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092349 |  |
| RAN5 #43 |  | RP-090447 |  | 0033 |  | - |  | Correction to the definition of simulated NAS cells in TS 36.508 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092352 |  |
| RAN5 #43 |  | RP-090447 |  | 0034 |  | - |  | CR on 6.7 TDD Timer Tolerance in 36.508 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092363 |  |
| RAN5 #43 |  | RP-090448 |  | 0044 |  | - |  | Update of 4.5.2A in 36.508 (Re-submit not change) |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092457 |  |
| RAN5 #43 |  | RP-090448 |  | 0035 |  | - |  | Default value of q-RxLevMin for RF TCs |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092458 |  |
| RAN5 #43 |  | RP-090598 |  | 0045 |  | - |  | CR to 36.508 Addition of test frequencies for band 18 and band 19 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092535 |  |
| RAN5 #43 |  | RP-090447 |  | 0036 |  | - |  | Update of the default NAS message contents in TS 36.508 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092708 |  |
| RAN5 #43 |  | RP-090447 |  | 0037 |  | - |  | Correction to reference radio bearer configurations |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092721 |  |
| RAN5 #43 |  | RP-090447 |  | 0038 |  | - |  | Definition of default Test Control (TC) messages |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092734 |  |
| RAN5 #43 |  | RP-090448 |  | 0039 |  | - |  | Modification of procedures in section 4.5.2.3 /4.5.2A |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092735 |  |
| RAN5 #43 |  | RP-090447 |  | 0040 |  | - |  | Addition of default physical layer parameters |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092736 |  |
| RAN5 #43 |  | RP-090447 |  | 0041 |  | - |  | Correction to default RRC message contents |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092738 |  |
| RAN5 #43 |  | RP-090447 |  | 0042 |  | - |  | Introduction in 36.508 of a common tracking/routing area update procedure for Idle mode and RRC connection release test cases |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092765 |  |
| RAN5 #43 |  | RP-090447 |  | 0043 |  | - |  | Corrections to default system configurations in TS 36.508 |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092773 |  |
| RAN5 #43 |  | RP-090447 |  | 0046 |  | - |  | Max. resources for signalling test cases |  |  |  | 8.1.0 |  | 8.2.0 |  | R5-092723 |  |
| - |  | - |  | - |  | - |  | Editorial corrections and merging of all sections together |  |  |  | 8.2.0 |  | 8.2.1 |  | - |  |
| RAN5 #44 |  | RP-090802 |  | 0047 |  | - |  | Correction to the default value of ul-Bandwidth in TS 36.508 |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094059 |  |
| RAN5 #44 |  | RP-090802 |  | 0048 |  | - |  | UTRAN SIB scheduling for LTE interRAT test |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094072 |  |
| RAN5 #44 |  | RP-090802 |  | 0049 |  | - |  | Correction to the default NAS message contents |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094141 |  |
| RAN5 #44 |  | RP-090802 |  | 0050 |  | - |  | TDD fields in default physical layer parameters |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094279 |  |
| RAN5 #44 |  | RP-090802 |  | 0051 |  | - |  | Addition of UTRA reference radio bearer parameters and GERAN reference PDP context parameters for E-UTRA Inter-RAT testing |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094304 |  |
| RAN5 #44 |  | RP-090801 |  | 0052 |  | - |  | System information scheduling for RF testing |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094311 |  |
| RAN5 #44 |  | RP-090801 |  | 0053 |  | - |  | Connection for 1 cell with antenna configuration 1x2 in static propagation conditions |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094364 |  |
| RAN5 #44 |  | RP-090801 |  | 0054 |  | - |  | Correction to 4.3.1.2.5 TDD reference test frequencies for Operating Band 37 |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094373 |  |
| RAN5 #44 |  | RP-090801 |  | 0055 |  | - |  | LTE RF: Physical Layer configurations for RF/RRM testing |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094421 |  |
| RAN5 #44 |  | RP-090802 |  | 0056 |  | - |  | Update of SN length in PDCP default configuration |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094533 |  |
| RAN5 #44 |  | RP-090802 |  | 0057 |  | - |  | Corrections to default RRC message and IE contents |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094639 |  |
| RAN5 #44 |  | RP-090801 |  | 0058 |  | - |  | Update of TDD reference test frequencies for operating band 40 |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094786 |  |
| RAN5 #44 |  | RP-090810 |  | 0059 |  | - |  | TDD special subframe pattern update |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-094901 |  |
| RAN5 #44 |  | RP-090802 |  | 0060 |  | - |  | corrections to reference RB configurations |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095064 |  |
| RAN5 #44 |  | RP-090802 |  | 0061 |  | - |  | Correction of test procedure 6.4.2.7 in TS 36.508 |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095093 |  |
| RAN5 #44 |  | RP-090802 |  | 0062 |  | - |  | Adding new elementary files to the default USIM settings |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095100 |  |
| RAN5 #44 |  | RP-090802 |  | 0063 |  | - |  | Correction to the Test procedure to check RRC_IDLE state |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095102 |  |
| RAN5 #44 |  | RP-090802 |  | 0064 |  | - |  | Introduction of UE mode of operation into NAS default message contents |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095138 |  |
| RAN5 #44 |  | RP-090802 |  | 0065 |  | - |  | TDD ACK/NACK feedback mode update |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095152 |  |
| RAN5 #44 |  | RP-090802 |  | 0066 |  | - |  | Corrections to default signal levels |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095206 |  |
| RAN5 #44 |  | RP-090802 |  | 0067 |  | - |  | Update to default messages in regard to IP address allocation |  | F |  | 8.2.1 |  | 8.3.0 |  | R5-095218 |  |
| RAN5 #45 |  | RP-091121 |  | 0068 |  | - |  | Correction to 4.3.1.2.6 TDD reference test frequencies for Operating Band 38 |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-095486 |  |
| RAN5 #45 |  | RP-091121 |  | 0069 |  | - |  | Correction CR to 36.508: Set the default parameter for offsetFreq in MeasObjectGERAN Information Element |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-095514 |  |
| RAN5 #45 |  | RP-091470 |  | 0070 |  | - |  | Addition of HSPA UTRA reference radio bearer parameters for E-UTRA Inter-RAT testing |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-095555 |  |
| RAN5 #45 |  | RP-091122 |  | 0071 |  | - |  | Introduction of RS power boosting to reduce interference |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-095594 |  |
| RAN5 #45 |  | RP-091122 |  | 0072 |  | - |  | Corrections to default RRC message contents |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-095651 |  |
| RAN5 #45 |  | RP-091122 |  | 0073 |  | - |  | Clarification for Cell Configuration Identifiers in 36.508 |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096005 |  |
| RAN5 #45 |  | RP-091122 |  | 0074 |  | - |  | Correction to the generic procedure for IP allocation and more |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096114 |  |
| RAN5 #45 |  | RP-091122 |  | 0075 |  | - |  | Addition of new generic procedure for TAU after inter-RAT HO from UTRA |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096115 |  |
| RAN5 #45 |  | RP-091122 |  | 0076 |  | - |  | Update of header chapter 5.2 in 36.508 |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096202 |  |
| RAN5 #45 |  | RP-091122 |  | 0077 |  | - |  | Correction to the default NAS message contents |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096403 |  |
| RAN5 #45 |  | RP-091122 |  | 0078 |  | - |  | cell frequency allocation |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096440 |  |
| RAN5 #45 |  | RP-091122 |  | 0079 |  | - |  | Correction for IP address allocation |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096447 |  |
| RAN5 #45 |  | RP-091122 |  | 0080 |  | - |  | LAI and TMSI in ATTACH ACCEPT and TAU ACCEPT messages |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096449 |  |
| RAN5 #45 |  | RP-091122 |  | 0081 |  | - |  | Addition of generic procedure for IP address allocation / configuration in U-plane |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096455 |  |
| RAN5 #45 |  | RP-091122 |  | 0082 |  | - |  | Correction of test procedures in TS 36.508 |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096456 |  |
| RAN5 #45 |  | RP-091122 |  | 0083 |  | - |  | Update of MAC configuration for disabling PHR and BSR for L2 test cases |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096458 |  |
| RAN5 #45 |  | RP-091122 |  | 0084 |  | - |  | Addition of default UTRA message contents to TS 36.508 |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096461 |  |
| RAN5 #45 |  | RP-091122 |  | 0085 |  | - |  | Clarification to the mapping of GERAN cells and the default parameter values |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096462 |  |
| RAN5 #45 |  | RP-091122 |  | 0086 |  | - |  | Correction of TFTs for reference dedicated EPS bearer contexts |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096464 |  |
| RAN5 #45 |  | RP-091122 |  | 0087 |  | - |  | Cleanup of default NAS message contents |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096465 |  |
| RAN5 #45 |  | RP-091122 |  | 0088 |  | - |  | LTE-Sig: Assignment of different rootSequenceIndex for cells at the same frequency |  | F |  | 8.3.0 |  | 8.4.0 |  | R5-096641 |  |
| RAN5 #46 |  | RP-100143 |  | 0089 |  | - |  | Correction of SIB19 scheduling position in the neighbouring UTRA cell |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100086 |  |
| RAN5 #46 |  | RP-100143 |  | 0091 |  | - |  | Correction of Quantity Configuration for EUTRA |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100110 |  |
| RAN5 #46 |  | RP-100143 |  | 0092 |  | - |  | Correction for the offset value of RSRP in EUTRA |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100111 |  |
| RAN5 #46 |  | RP-100143 |  | 0093 |  | - |  | SIB10 and SIB11 periodicity |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100112 |  |
| RAN5 #46 |  | RP-100143 |  | 0094 |  | - |  | Assignment of rootSequenceIndex for simulated NAS cells in different PLMNs |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100260 |  |
| RAN5 #46 |  | RP-100143 |  | 0095 |  | - |  | Editorial correction to the default value of 'p-a' |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100261 |  |
| RAN5 #46 |  | RP-100142 |  | 0096 |  | - |  | New chapter: Test environment for RRM tests |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100396 |  |
| RAN5 #46 |  | RP-100143 |  | 0097 |  | - |  | Remove UM in DRB reconfiguration |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100487 |  |
| RAN5 #46 |  | RP-100143 |  | 0098 |  | - |  | Addition of default power allocation for two TX antennas |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100518 |  |
| RAN5 #46 |  | RP-100143 |  | 0099 |  | - |  | Correction to TFT parameters used in ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100771 |  |
| RAN5 #46 |  | RP-100143 |  | 0100 |  | - |  | Update to RRC common messages for support of test cases for MIMO |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100786 |  |
| RAN5 #46 |  | RP-100152 |  | 0101 |  | - |  | Adding band 20 in 36.508 |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100846 |  |
| RAN5 #46 |  | RP-100154 |  | 0102 |  | - |  | CR to 36.508: Update test frequencies with extended LTE1500 operating bands |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-100847 |  |
| RAN5 #46 |  | RP-100143 |  | 0103 |  | - |  | Defining default message contents for ATTACH/TAU REQUEST/ACCEPT messages according to UE capability |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101021 |  |
| RAN5 #46 |  | RP-100143 |  | 0104 |  | - |  | Limitation of simultaneous co-existence of intra-freq cells to reduce interference |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101029 |  |
| RAN5 #46 |  | RP-100143 |  | 0105 |  | - |  | An additional option for IP address allocation in test cases using UE test mode |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101045 |  |
| RAN5 #46 |  | RP-100143 |  | 0106 |  | - |  | Specify default UL NAS check |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101051 |  |
| RAN5 #46 |  | RP-100143 |  | 0107 |  | - |  | Correct default requirement for some mandatory information elements |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101052 |  |
| RAN5 #46 |  | RP-100142 |  | 0108 |  | - |  | Addition of Tracking area updating procedure |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101136 |  |
| RAN5 #46 |  | RP-100143 |  | 0109 |  | - |  | Clarification of Security Protection for NAS Messages |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101147 |  |
| RAN5 #46 |  | RP-100143 |  | 0110 |  | - |  | Update to layer 2 UM test cases to increase the drx-Inactivity Timer to psf200 |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101178 |  |
| RAN5 #46 |  | RP-100143 |  | 0111 |  | - |  | Addition of new generic procedure for bearer establishment for MO call. |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101187 |  |
| RAN5 #46 |  | RP-100143 |  | 0112 |  | - |  | update of default bandwidth configuration for signalling |  | F |  | 8.4.0 |  | 8.5.0 |  | R5-101207 |  |
| RP#47 |  | - |  | - |  | - |  | Moved to v9.0.0 with no change |  | - |  | 8.5.0 |  | 9.0.0 |  | - |  |
| RP#48 |  | RP-100510 |  | 0143 |  | - |  | Clarification of security protection when NAS security mode procedure has taken place outside of a TC sequence |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103085 |  |
| RP#48 |  | RP-100523 |  | 0113 |  | - |  | CR to 36.508: Update of EARFCN for band 21 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103101 |  |
| RP#48 |  | RP-100510 |  | 0114 |  | - |  | Correction to the default message contents of EXTENDED SERVICE REQUEST |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103128 |  |
| RP#48 |  | RP-100510 |  | 0115 |  | - |  | Removal of unrealistic network behaviour from generic procedures |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103223 |  |
| RP#48 |  | RP-100510 |  | 0116 |  | - |  | Correction of table numbers in clause 6.2.3.1 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103232 |  |
| RP#48 |  | RP-100510 |  | 0117 |  | - |  | Correction to default values for PhysicalConfigDedicated and MIMO |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103287 |  |
| RP#48 |  | RP-100510 |  | 0118 |  | - |  | Correction to SR-ConfigIndex for LTE TDD signalling test cases |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103290 |  |
| RP#48 |  | RP-100510 |  | 0119 |  | - |  | Correction to MME Group ID to set MSB to 1 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103298 |  |
| RP#48 |  | RP-100500 |  | 0144 |  | - |  | Connection diagram for test 8.11.2 (3 cells) |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103311 |  |
| RP#48 |  | RP-100510 |  | 0120 |  | - |  | New combination of system information blocks for CSG Cell in TS 36.508 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103363 |  |
| RP#48 |  | RP-100510 |  | 0121 |  | - |  | Clarification of security protection for detach request message |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103368 |  |
| RP#48 |  | RP-100510 |  | 0122 |  | - |  | Corrections to cell numbers for 3GPP2 Inter-RAT network scenarios |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103374 |  |
| RP#48 |  | RP-100510 |  | 0140 |  | - |  | Clarification to default message content for Modify EPS Bearer Context Request message |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103625 |  |
| RP#48 |  | RP-100510 |  | 0141 |  | - |  | Clarification to default message content for RRC Connection Reconfiguration message |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103626 |  |
| RP#48 |  | RP-100524 |  | 0123 |  | - |  | Addition of WLAN test cell |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103647 |  |
| RP#48 |  | RP-100524 |  | 0124 |  | - |  | Addition of default message contents for mobility management based on DSMIPv6 testing |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103648 |  |
| RP#48 |  | RP-100510 |  | 0125 |  | - |  | Update default message contents for EPS attach conditions |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103673 |  |
| RP#48 |  | RP-100510 |  | 0126 |  | - |  | Update default message with network support for IMS voice |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103674 |  |
| RP#48 |  | RP-100510 |  | 0127 |  | - |  | Correction to Generic Test Procedure in TS 36.508 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103675 |  |
| RP#48 |  | RP-100510 |  | 0128 |  | - |  | Default settings of suitable - non-suitable cells for UTRAN/GERAN |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103677 |  |
| RP#48 |  | RP-100510 |  | 0129 |  | - |  | Correction to IE schedulingRequestConfig during Handover |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103678 |  |
| RP#48 |  | RP-100510 |  | 0130 |  | - |  | Update generic procedures for IMS |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103679 |  |
| RP#48 |  | RP-100510 |  | 0131 |  | - |  | Aligning E-UTRAN USIM parameters for multi-RAT devices |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103680 |  |
| RP#48 |  | RP-100510 |  | 0132 |  | - |  | Adding Additional Update Result handling to the default messages |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103681 |  |
| RP#48 |  | RP-100509 |  | 0133 |  | - |  | Connection diagram update for intra-freq measurement with phase rotator |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103770 |  |
| RP#48 |  | RP-100509 |  | 0134 |  | - |  | Update of default bandwidth configuration for Signalling for Band 38 |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103813 |  |
| RP#48 |  | RP-100524 |  | 0135 |  | - |  | Addition of generic procedures for mobility management based on DSMIPv6 testing |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103858 |  |
| RP#48 |  | RP-100510 |  | 0136 |  | - |  | Addition of new generic procedure for MO SMS over SGs and clarifications |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103869 |  |
| RP#48 |  | RP-100510 |  | 0137 |  | - |  | Addition of generic procedures for HRPD and 1xRTT pre-registration |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103870 |  |
| RP#48 |  | RP-100510 |  | 0138 |  | - |  | Introduction of reference information for test case postambles |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103875 |  |
| RP#48 |  | RP-100509 |  | 0139 |  | - |  | Physical layer parameter correction to DCI formats used in RF tests |  | F |  | 9.0.0 |  | 9.1.0 |  | R5-103885 |  |
| RP#49 |  | RP-100812 |  | 0145 |  | - |  | Update of tested channel bandwidths for Bands 13, 14 and 17 |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104089 |  |
| RP#49 |  | RP-100816 |  | 0146 |  | - |  | Correction to remove special configurations for UM Bearer test cases |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104106 |  |
| RP#49 |  | RP-100816 |  | 0147 |  | - |  | Clarification to the procedure: UE triggered establishment of a default EPS bearer context associated with an additional PDN |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104128 |  |
| RP#49 |  | RP-100816 |  | 0148 |  | - |  | Correction to Inter-frequency carrier frequency list in SIB5 and E-UTRA carrier frequency list in SIB19 |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104169 |  |
| RP#49 |  | RP-100816 |  | 0149 |  | - |  | Update of 6.2.3 of 36.508 - description of default frequency for the single cell signalling test |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104220 |  |
| RP#49 |  | RP-100831 |  | 0150 |  | - |  | Update of default message contents for DSMIPv6 testing |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104392 |  |
| RP#49 |  | RP-100837 |  | 0151 |  | - |  | Addition of UE test state model for HRPD |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104454 |  |
| RP#49 |  | RP-100812 |  | 0152 |  | - |  | Correction to Qrxlevmin in SIB 1 for RF and RRM |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104503 |  |
| RP#49 |  | RP-100816 |  | 0153 |  | - |  | Specification of HRPD specific values in SIB8 |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104547 |  |
| RP#49 |  | RP-100816 |  | 0154 |  | - |  | Update of default bandwidth configuration for signalling |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104685 |  |
| RP#49 |  | RP-100816 |  | 0155 |  | - |  | Clarification to packet filter identifier and precedence in TFT |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104702 |  |
| RP#49 |  | RP-100816 |  | 0156 |  | - |  | Add P-CSCF method II for IMS |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104703 |  |
| RP#49 |  | RP-100816 |  | 0157 |  | - |  | Update generic procedures for IMS |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104704 |  |
| RP#49 |  | RP-100816 |  | 0158 |  | - |  | Correction of clause 4.3.3.4 |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104705 |  |
| RP#49 |  | RP-100816 |  | 0159 |  | - |  | Correction to Downlink Frequency for N_DL 4850 |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104706 |  |
| RP#49 |  | RP-100816 |  | 0160 |  | - |  | Default message contents for UTRAN and GERAN |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104707 |  |
| RP#49 |  | RP-100812 |  | 0161 |  | - |  | Addition of the new connection diagram for the CSI test case |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-104884 |  |
| RP#49 |  | RP-100816 |  | 0162 |  | - |  | Correction to reference end states |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-105003 |  |
| RP#49 |  | RP-100837 |  | 0163 |  | - |  | Correction for Timer Tolerances |  | F |  | 9.1.0 |  | 9.2.0 |  | R5-105050 |  |
| RP#49 |  | RP-100885 |  | 0165 |  | - |  | Introduction of default message contents for HRPD overhead messages |  | F |  | 9.1.0 |  | 9.2.0 |  | - |  |
| GP#49 |  | GP-101743 |  | 0166 |  | - |  | Update of Common parameters for simulated GERAN cells for supporting GERAN-EUTRAN Inter-RAT cell reselection |  | F |  | 9.2.0 |  | 9.3.0 |  | GP-101743 |  |
| RP#50 |  | RP-101138 |  | 0167 |  | - |  | MBSFN configuration for RRM tests using E-UTRA FDD cells |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106070 |  |
| RP#50 |  | RP-101155 |  | 0169 |  | - |  | CR to 36.508: Update test frequencies for EUTRA TDD LTE band 41. |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106105 |  |
| RP#50 |  | RP-101142 |  | 0168 |  | - |  | CR to 36.508: Correction to HRPD Overhead messages (subclause 4.4.7.1) |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106116 |  |
| RP#50 |  | RP-101142 |  | 0170 |  | - |  | Addition of test frequencies for LTE-C2k interworking test cases |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106298 |  |
| RP#50 |  | RP-101142 |  | 0172 |  | - |  | Addition of UTRA reference radio parameters and combination for PS RB and Speech |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106383 |  |
| RP#50 |  | RP-101142 |  | 0175 |  | - |  | Correction to EUTRA carrier frequency list in SIB19 |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106421 |  |
| RP#50 |  | RP-101142 |  | 0171 |  | - |  | Correction to the IDENTITY RESPONSE (with IMSI) message |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106551 |  |
| RP#50 |  | RP-101142 |  | 0178 |  | - |  | Correction for DRX offset start time |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106552 |  |
| RP#50 |  | RP-101142 |  | 0177 |  | - |  | Correction of specific message content for generic procedures (state 2 and state 2A) |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106596 |  |
| RP#50 |  | RP-101138 |  | 0180 |  | - |  | LTE-RF state 3A |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106597 |  |
| RP#50 |  | RP-101142 |  | 0176 |  | - |  | Correction to security protection header for Identity Request message |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106611 |  |
| RP#50 |  | RP-101142 |  | 0173 |  | - |  | Addition of new system information combinations |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106616 |  |
| RP#50 |  | RP-101142 |  | 0174 |  | - |  | Remove SS requirement for IMS in UTRA |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106690 |  |
| RP#50 |  | RP-101159 |  | 0179 |  | - |  | Corrections to default settings for Elementary Files (EFs) on Test USIM |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106819 |  |
| RP#50 |  | RP-101138 |  | 0181 |  | - |  | Update of the RF exceptional RRC message |  | F |  | 9.2.0 |  | 9.3.0 |  | R5-106820 |  |
| RP#51 |  | RP-110161 |  | 0182 |  | - |  | Update of HRPD overhead message parameters AccessSignature, SectorSignature |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110069 |  |
| RP#51 |  | RP-110161 |  | 0183 |  | - |  | Add a new eUTRA sub-end state E2_T3440 |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110105 |  |
| RP#51 |  | RP-110161 |  | 0184 |  | - |  | Correction for NAS message NOTE |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110229 |  |
| RP#51 |  | RP-110161 |  | 0185 |  | - |  | Addition of CSIM default contents |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110331 |  |
| RP#51 |  | RP-110161 |  | 0186 |  | - |  | Update of Table 4.4.2-1 with HRPD/1xRTT frequency range info |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110332 |  |
| RP#51 |  | RP-110157 |  | 0187 |  | - |  | Update of the RF exceptional RRC message |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110410 |  |
| RP#51 |  | RP-110161 |  | 0188 |  | - |  | Editorial correction for IMS signalling |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110433 |  |
| RP#51 |  | RP-110161 |  | 0189 |  | - |  | Correction to SIB combinations related to HeNB Cells |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110471 |  |
| RP#51 |  | RP-110161 |  | 0190 |  | - |  | Correction to default message content for Detach Request message |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110472 |  |
| RP#51 |  | RP-110157 |  | 0191 |  | - |  | Removal of Lower Humidity Limit in Normal Conditions |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110534 |  |
| RP#51 |  | RP-110157 |  | 0192 |  | - |  | Correction of EARFCN numbers for band 41 |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110542 |  |
| RP#51 |  | RP-110161 |  | 0193 |  | - |  | Removal of "Modified contents of the EFs at the ISIM ADF (application DF) level" |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110593 |  |
| RP#51 |  | RP-110161 |  | 0194 |  | - |  | Correction of the IEs for compressed mode in table 4.7B.1-5 |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110601 |  |
| RP#51 |  | RP-110161 |  | 0195 |  | - |  | Update to default message content for TRACKING AREA UPDATE REQUEST message |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110703 |  |
| RP#51 |  | RP-110161 |  | 0196 |  | - |  | Add default APN for IMS |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110708 |  |
| RP#51 |  | RP-110161 |  | 0197 |  | - |  | Introduction of over head messages for CDMA2000 1XRTT |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110710 |  |
| RP#51 |  | RP-110161 |  | 0198 |  | - |  | Addition of default SMS over SGs message contents |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110875 |  |
| RP#51 |  | RP-110161 |  | 0199 |  | - |  | Correction to 'Test procedure to check that UE is camped on E-UTRAN cell upon mobility from another RAT' |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110746 |  |
| RP#51 |  | RP-110161 |  | 0200 |  | - |  | Correction of frequency allocations |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110788 |  |
| RP#51 |  | RP-110161 |  | 0201 |  | - |  | Update of Reference packet filters contents |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110789 |  |
| RP#51 |  | RP-110157 |  | 0202 |  | - |  | Correction to FDD Reference Test Frequencies for Operating Band 12 |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110846 |  |
| RP#51 |  | RP-110157 |  | 0203 |  | - |  | LTE RF: state 3A-RF update |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110937 |  |
| RP#51 |  | RP-110172 |  | 0204 |  | - |  | Add test frequencies for bands 42, 43 (3500MHz) |  | F |  | 9.3.0 |  | 9.4.0 |  | R5-110968 |  |
| RP#52 |  | RP-110647 |  | 0205 |  | - |  | Correction to default message content of LOCATION UPDATING REQUEST message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112114 |  |
| RP#52 |  | RP-110647 |  | 0206 |  | - |  | Correction to default message content of TRACKING AREA UPDATE REQUEST message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112120 |  |
| RP#52 |  | RP-110643 |  | 0207 |  | - |  | PRACH-Config-DEFAULT for RF-tests TDD: Correction to derivation path |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112146 |  |
| RP#52 |  | RP-110643 |  | 0208 |  | - |  | Correction to connection diagram for CQI with uneven interference test (A.21) |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112147 |  |
| RP#52 |  | RP-110647 |  | 0209 |  | - |  | Update to Common contents of system information blocks |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112161 |  |
| RP#52 |  | RP-110647 |  | 0210 |  | - |  | Addition of missing labels in Figure 4.5.1-1 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112278 |  |
| RP#52 |  | RP-110647 |  | 0211 |  | - |  | Corrections to Table 4.3.7-6 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112279 |  |
| RP#52 |  | RP-110647 |  | 0212 |  | - |  | Update of CSIM default contents |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112285 |  |
| RP#52 |  | RP-110660 |  | 0213 |  | - |  | Add emergency bearer support |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112289 |  |
| RP#52 |  | RP-110667 |  | 0214 |  | - |  | Band 24 Addition to TS 36.508 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112381 |  |
| RP#52 |  | RP-110643 |  | 0215 |  | - |  | New connection diagrams to Annex A |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112458 |  |
| RP#52 |  | RP-110647 |  | 0216 |  | - |  | Correction to Derivation Path for RB Setup on UTRA side (condition UTRA PS RB) |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112571 |  |
| RP#52 |  | RP-110647 |  | 0217 |  | - |  | Update APN check at attach |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112598 |  |
| RP#52 |  | RP-110647 |  | 0218 |  | - |  | Update of SIB7 default message contents |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112599 |  |
| RP#52 |  | RP-110647 |  | 0219 |  | - |  | Clarification to conditions for IP address configuration over user plane |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112600 |  |
| RP#52 |  | RP-110666 |  | 0220 |  | - |  | Introduction of UE Test Loop Mode C for LTE MBMS testing |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112675 |  |
| RP#52 |  | RP-110647 |  | 0222 |  | - |  | Update 36.508 QoS definition for InterRat test cases |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112698 |  |
| RP#52 |  | RP-110666 |  | 0221 |  | - |  | Addition of some MBMS related message definitions in TS36.508 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112748 |  |
| RP#52 |  | RP-110647 |  | 0223 |  | - |  | Introduction of generic CS fall back procedures for UTRAN and GERAN |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112751 |  |
| RP#52 |  | RP-110643 |  | 0224 |  | - |  | Default Bandwidth Configuration for RF Testing |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112760 |  |
| RP#52 |  | RP-110643 |  | 0225 |  | - |  | Update for PRACH-Config-DEFAULT for the default TDD RRM message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112872 |  |
| RP#52 |  | RP-110647 |  | 0205 |  | - |  | Correction to default message content of LOCATION UPDATING REQUEST message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112114 |  |
| RP#52 |  | RP-110647 |  | 0206 |  | - |  | Correction to default message content of TRACKING AREA UPDATE REQUEST message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112120 |  |
| RP#52 |  | RP-110643 |  | 0207 |  | - |  | PRACH-Config-DEFAULT for RF-tests TDD: Correction to derivation path |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112146 |  |
| RP#52 |  | RP-110643 |  | 0208 |  | - |  | Correction to connection diagram for CQI with uneven interference test (A.21) |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112147 |  |
| RP#52 |  | RP-110647 |  | 0209 |  | - |  | Update to Common contents of system information blocks |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112161 |  |
| RP#52 |  | RP-110647 |  | 0210 |  | - |  | Addition of missing labels in Figure 4.5.1-1 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112278 |  |
| RP#52 |  | RP-110647 |  | 0211 |  | - |  | Corrections to Table 4.3.7-6 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112279 |  |
| RP#52 |  | RP-110647 |  | 0212 |  | - |  | Update of CSIM default contents |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112285 |  |
| RP#52 |  | RP-110660 |  | 0213 |  | - |  | Add emergency bearer support |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112289 |  |
| RP#52 |  | RP-110667 |  | 0214 |  | - |  | Band 24 Addition to TS 36.508 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112381 |  |
| RP#52 |  | RP-110643 |  | 0215 |  | - |  | New connection diagrams to Annex A |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112458 |  |
| RP#52 |  | RP-110647 |  | 0216 |  | - |  | Correction to Derivation Path for RB Setup on UTRA side (condition UTRA PS RB) |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112571 |  |
| RP#52 |  | RP-110647 |  | 0217 |  | - |  | Update APN check at attach |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112598 |  |
| RP#52 |  | RP-110647 |  | 0218 |  | - |  | Update of SIB7 default message contents |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112599 |  |
| RP#52 |  | RP-110647 |  | 0219 |  | - |  | Clarification to conditions for IP address configuration over user plane |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112600 |  |
| RP#52 |  | RP-110647 |  | 0222 |  | - |  | Update 36.508 QoS definition for InterRat test cases |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112698 |  |
| RP#52 |  | RP-110666 |  | 0221 |  | - |  | Addition of some MBMS related message definitions in TS36.508 |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112748 |  |
| RP#52 |  | RP-110647 |  | 0223 |  | - |  | Introduction of generic CS fall back procedures for UTRAN and GERAN |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112751 |  |
| RP#52 |  | RP-110643 |  | 0224 |  | - |  | Default Bandwidth Configuration for RF Testing |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112760 |  |
| RP#52 |  | RP-110643 |  | 0225 |  | - |  | Update for PRACH-Config-DEFAULT for the default TDD RRM message |  | F |  | 9.4.0 |  | 9.5.0 |  | R5-112872 |  |
| RP#53 |  | RP-111138 |  | 0226 |  | - |  | Correction for generic CS fallback procedures for UTRAN |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113033 |  |
| RP#53 |  | RP-111138 |  | 0227 |  | - |  | Correction to paging test procedure |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113163 |  |
| RP#53 |  | RP-111138 |  | 0228 |  | - |  | Deletion of 'EPSOnlyAttachForced' in common part |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113195 |  |
| RP#53 |  | RP-111135 |  | 0229 |  | - |  | Deletion of 'EPSOnlyAttachForced' in RF part |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113213 |  |
| RP#53 |  | RP-111135 |  | 0230 |  | - |  | Correction to PhysicalConfigDedicated-DEFAULT for HO case |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113450 |  |
| RP#53 |  | RP-111138 |  | 0231 |  | - |  | Addition of the 'Expanded 1900 MHz band' as operating band 25 to TS 36.508 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113500 |  |
| RP#53 |  | RP-111153 |  | 0232 |  | - |  | Band 24 Details for Signalling part of 36.508 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113521 |  |
| RP#53 |  | RP-111138 |  | 0233 |  | - |  | Add new SI combination |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113658 |  |
| RP#53 |  | RP-111138 |  | 0234 |  | - |  | Update of HRPD/1xRTT test frequencies, pre-registration procedures and message contents |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113659 |  |
| RP#53 |  | RP-111138 |  | 0235 |  | - |  | Correction to test procedures specific message contents |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113660 |  |
| RP#53 |  | RP-111138 |  | 0236 |  | - |  | Correction to 'Test procedure to check that UE is camped on E-UTRAN cell upon mobility from another RAT' |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113661 |  |
| RP#53 |  | RP-111138 |  | 0237 |  | - |  | Correction for the default NAS message contents |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113662 |  |
| RP#53 |  | RP-111138 |  | 0238 |  | - |  | Update UTRA RRC messages |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113663 |  |
| RP#53 |  | RP-111138 |  | 0239 |  | - |  | Update UTRA NAS messages |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113664 |  |
| RP#53 |  | RP-111138 |  | 0240 |  | - |  | Addition of default GERAN message PS HANDOVER COMMAND |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113665 |  |
| RP#53 |  | RP-111138 |  | 0241 |  | - |  | Update test procedure 6.4.3.7.6 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113666 |  |
| RP#53 |  | RP-111138 |  | 0242 |  | - |  | Update test procedure 6.4.3.7.5 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113667 |  |
| RP#53 |  | RP-111155 |  | 0243 |  | - |  | Update test frequencies for FDD LTE Band 23 in 36.508 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113749 |  |
| RP#53 |  | RP-111138 |  | 0244 |  | - |  | Correction on the IE ReportConfigEUTRA-PERIODICAL definition |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113751 |  |
| RP#53 |  | RP-111148 |  | 0245 |  | - |  | Correction to TS36.508 subclause 4.6.1 |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113761 |  |
| RP#53 |  | RP-111145 |  | 0246 |  | - |  | Combined parallel procedures between EUTRA/EPC and IMS emergency call |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-113801 |  |
| RP#53 |  | RP-111135 |  | 0247 |  | - |  | RF/RRM State 3A-RF: Editors note |  | F |  | 9.5.0 |  | 9.6.0 |  | R5-114037 |  |
| RP#54 |  | RP-111579 |  | 0248 |  | - |  | Correction of the default message contents of Transaction Identifiers in Activate Default EPS Bearer Context and Activate Dedicated EPS Bearer Context messages |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115091 |  |
| RP#54 |  | RP-111596 |  | 0250 |  | - |  | Adding band 22 (3500MHz FDD) to 36.508 |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115185 |  |
| RP#54 |  | RP-111579 |  | 0251 |  | - |  | Update of UE Registration pre-registration on 1xRTT registrationPeriod |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115264 |  |
| RP#54 |  | RP-111579 |  | 0252 |  | - |  | Update UTRA RRC message for handover to UTRAN |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115533 |  |
| RP#54 |  | RP-111579 |  | 0253 |  | - |  | Add reference default EPS bearer context for QCI 5 |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115537 |  |
| RP#54 |  | RP-111579 |  | 0254 |  | - |  | Correction of EPS Bearer Contexts |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115585 |  |
| RP#54 |  | RP-111579 |  | 0255 |  | - |  | Correction to ESM default messages |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115618 |  |
| RP#54 |  | RP-111579 |  | 0256 |  | - |  | Introduction of Combined Generic test procedure for IMS Speech call |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115619 |  |
| RP#54 |  | RP-111576 |  | 0257 |  | - |  | default band configuration for RF testing in band 25 |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115633 |  |
| RP#54 |  | RP-111579 |  | 0258 |  | - |  | Complete the system configuration for dual mode network scenario |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115694 |  |
| RP#54 |  | RP-111579 |  | 0259 |  | - |  | Correction to IE PhysicalConfigDedicated-DEFAULT definition in Table 4.8.2.1.6-1 |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115696 |  |
| RP#54 |  | RP-111579 |  | 0260 |  | - |  | Updates for 1xRTT pre-registration scenario |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115708 |  |
| RP#54 |  | RP-111579 |  | 0261 |  | - |  | Update of UE 1xRTT registrationPeriod in SystemInformationBlock type8 |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115750 |  |
| RP#54 |  | RP-111579 |  | 0262 |  | - |  | Correction to 1xRTT Overhead Message contents |  | F |  | 9.6.0 |  | 9.7.0 |  | R5-115791 |  |
| RP#55 |  | RP-120176 |  | 0263 |  | - |  | Correction to measurement control and report in default UTRA message |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120311 |  |
| RP#55 |  | RP-120176 |  | 0264 |  | - |  | Update of E-UTRAN_QRXLEVMIN in SIB19 and SI2 Quater |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120312 |  |
| RP#55 |  | RP-120176 |  | 0265 |  | - |  | Update of RF Reference system configurations |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120313 |  |
| RP#55 |  | RP-120179 |  | 0266 |  | - |  | Correction of Physical Layer configurations |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120497 |  |
| RP#55 |  | RP-120179 |  | 0267 |  | - |  | Correction of E2_T3440 state definition |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120566 |  |
| RP#55 |  | RP-120179 |  | 0268 |  | - |  | Correction of default PCO value in some ESM messages |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120572 |  |
| RP#55 |  | RP-120179 |  | 0269 |  | - |  | Correction to the default message content of GERAN carrier frequency group list for E-UTRA cells |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120604 |  |
| RP#55 |  | RP-120179 |  | 0270 |  | - |  | Update of SystemInformationBlockType1 |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120616 |  |
| RP#55 |  | RP-120179 |  | 0271 |  | - |  | Update the default configuration of channel bandwidth for Band 39 for signalling testing |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120617 |  |
| RP#55 |  | RP-120179 |  | 0272 |  | - |  | Correction for UE pre-registration on CDMA2000 system |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120618 |  |
| RP#55 |  | RP-120179 |  | 0273 |  | - |  | Update to Reference default EPS bearer context #2 |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120619 |  |
| RP#55 |  | RP-120179 |  | 0274 |  | - |  | Correction to the default (UTRA) Physical Channel Reconfiguration message |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120620 |  |
| RP#55 |  | RP-120179 |  | 0275 |  | - |  | Correction of default measurement gap offset |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120621 |  |
| RP#55 |  | RP-120179 |  | 0276 |  | - |  | Correction to carrier bandwidth |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120677 |  |
| RP#55 |  | RP-120192 |  | 0277 |  | - |  | Update generic procedure 4.5A.4 |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120693 |  |
| RP#55 |  | RP-120192 |  | 0278 |  | - |  | Update generic procedure 4.5A.5 |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120694 |  |
| RP#55 |  | RP-120200 |  | 0280 |  | - |  | Addition of the default value of Carrier Aggregation parameters |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120726 |  |
| RP#55 |  | RP-120179 |  | 0282 |  | - |  | Correction of UE Release in UE capability |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120752 |  |
| RP#55 |  | RP-120179 |  | 0283 |  | - |  | Introduction of generic test procedure for SRVCC call handover to UTRA |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120753 |  |
| RP#55 |  | RP-120179 |  | 0284 |  | - |  | Correction to test procedure sequence 6.4.2.7A for check that UE is camped on E-UTRAN cell upon mobility from another RAT |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120754 |  |
| RP#55 |  | RP-120179 |  | 0285 |  | - |  | Addition of Default UTRA RRC Connection Request message |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120756 |  |
| RP#55 |  | RP-120176 |  | 0286 |  | - |  | TS 36.508: Band 23 test frequencies correction |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120800 |  |
| RP#55 |  | RP-120179 |  | 0287 |  | - |  | Remove IPv4viaNAS_TestMode |  | F |  | 9.7.0 |  | 9.8.0 |  | R5-120908 |  |
| RP#55 |  | GP-120009 |  | 0288 |  | - |  | Section 4.4.5 Common parameters for simulated GERAN cells – Correction to SI2 Quarter |  | F |  | 9.7.0 |  | 9.8.0 |  | GP-120009 |  |
| RP#55 |  | RP-120203 |  | 0279 |  | - |  | Addition of two MBMS counting related messages in TS 36.508 |  | F |  | 9.8.0 |  | 10.0.0 |  | R5-120723 |  |
| RP#55 |  | RP-120199 |  | 0281 |  | - |  | Introduction of new RRC default messages for Rel-10 and MDT |  | F |  | 9.8.0 |  | 10.0.0 |  | R5-120737 |  |
| RP#56 |  | RP-120644 |  | 0290 |  | - |  | Correction of power ratio allocation on PDSCH for common logical channels |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121089 |  |
| RP#56 |  | RP-120644 |  | 0291 |  | - |  | Correction to default mobilityParameters message contents |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121121 |  |
| RP#56 |  | RP-120644 |  | 0292 |  | - |  | Corrections to default overhead messages for HRPD |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121122 |  |
| RP#56 |  | RP-120644 |  | 0293 |  | - |  | Corrections to default overhead messages for 1xRTT |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121123 |  |
| RP#56 |  | RP-120644 |  | 0294 |  | - |  | Update of CSFBParametersRequest/ResponseCDMA2000 process in Registration |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121276 |  |
| RP#56 |  | RP-120641 |  | 0295 |  | - |  | RRM: Definition of parameters for simulated GSM cells |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121357 |  |
| RP#56 |  | RP-120662 |  | 0296 |  | - |  | SIB Combination for CA test scenarios |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121383 |  |
| RP#56 |  | RP-120644 |  | 0297 |  | - |  | Addition of default value of RSRQ Cell Reselection parameters |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121384 |  |
| RP#56 |  | RP-120641 |  | 0298 |  | - |  | Test frequencies for inter-band cells in RRM tests |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121407 |  |
| RP#56 |  | RP-120644 |  | 0299 |  | - |  | Update generic procedure 4.5A.6 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121413 |  |
| RP#56 |  | RP-120644 |  | 0300 |  | - |  | Update generic procedure 4.5A.7 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121414 |  |
| RP#56 |  | RP-120644 |  | 0301 |  | - |  | Update generic procedure 4.5A.3 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121439 |  |
| RP#56 |  | RP-120644 |  | 0302 |  | - |  | Correction to RadioResourceConfigCommon-DEFAULT |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121447 |  |
| RP#56 |  | RP-120641 |  | 0304 |  | - |  | Removing 5.2A.3 for State 2A |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121521 |  |
| RP#56 |  | RP-120641 |  | 0305 |  | - |  | Addition of A-GNSS testing to Test frequencies clause 4.3.1 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121543 |  |
| RP#56 |  | RP-120641 |  | 0306 |  | - |  | Ensuring all SS TX antennas are in use for whole test sequence |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121544 |  |
| RP#56 |  | RP-120641 |  | 0307 |  | - |  | Connection diagram for RRM 3 cell TCs with static propagation |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121556 |  |
| RP#56 |  | RP-120644 |  | 0308 |  | - |  | Correction of Physical Layer configurations tables |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121665 |  |
| RP#56 |  | RP-120644 |  | 0309 |  | - |  | Update of default bandwidth configuration for E-UTRA FDD band 11 and 18 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121712 |  |
| RP#56 |  | RP-120644 |  | 0310 |  | - |  | Correction to default SIB4 contents for CSG cells |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121733 |  |
| RP#56 |  | RP-120644 |  | 0311 |  | - |  | Update of CDMA2000 Band Class data and addition of CDMA2000 Band Class 10 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121734 |  |
| RP#56 |  | RP-120644 |  | 0312 |  | - |  | Update to Reference dedicated EPS bearer context #3 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121735 |  |
| RP#56 |  | RP-120644 |  | 0313 |  | - |  | Add generic procedure MO video call |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121736 |  |
| RP#56 |  | RP-120644 |  | 0314 |  | - |  | Add generic procedure MT video call |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121737 |  |
| RP#56 |  | RP-120644 |  | 0315 |  | - |  | Clarifications to UICC requirements for LTE-C2K testing |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121809 |  |
| RP#56 |  | RP-120644 |  | 0316 |  | - |  | Update UE capability information |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121847 |  |
| RP#56 |  | RP-120658 |  | 0317 |  | - |  | Introduction of System information for PWS |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121854 |  |
| RP#56 |  | RP-120662 |  | 0318 |  | - |  | Update of the default value of Carrier Aggregation parameters |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121856 |  |
| RP#56 |  | RP-120644 |  | 0319 |  | - |  | Correction to reference system configurations for dual mode scenarios |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121888 |  |
| RP#56 |  | RP-120667 |  | 0320 |  | - |  | Adding operating band 26 to TS 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121905 |  |
| RP#56 |  | RP-120641 |  | 0321 |  | - |  | Addition of State 3B-RF in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121919 |  |
| RP#56 |  | RP-120641 |  | 0322 |  | - |  | Correction to Tracking area updating procedure in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121936 |  |
| RP#56 |  | RP-120662 |  | 0323 |  | - |  | Addition of Test mode call procedure for CA test cases |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121982 |  |
| RP#56 |  | RP-120641 |  | 0303 |  | - |  | Correction to Handover to UTRAN commands for UTRA RRC messages in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-122018 |  |
| RP#56 |  | RP-120662 |  | 0324 |  | - |  | 36.508 - Test frequencies for CA_1C and CA_40C |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-122134 |  |
| RP#56 |  | RP-120649 |  | 0289 |  | - |  | Removal of technical content in 36.508 v9.8.0 and substitution with pointer to the next Release |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121078 |  |
| RP#56 |  | RP-120644 |  | 0290 |  | - |  | Correction of power ratio allocation on PDSCH for common logical channels |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121089 |  |
| RP#56 |  | RP-120644 |  | 0291 |  | - |  | Correction to default mobilityParameters message contents |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121121 |  |
| RP#56 |  | RP-120644 |  | 0292 |  | - |  | Corrections to default overhead messages for HRPD |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121122 |  |
| RP#56 |  | RP-120644 |  | 0293 |  | - |  | Corrections to default overhead messages for 1xRTT |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121123 |  |
| RP#56 |  | RP-120644 |  | 0294 |  | - |  | Update of CSFBParametersRequest/ResponseCDMA2000 process in Registration |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121276 |  |
| RP#56 |  | RP-120641 |  | 0295 |  | - |  | RRM: Definition of parameters for simulated GSM cells |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121357 |  |
| RP#56 |  | RP-120662 |  | 0296 |  | - |  | SIB Combination for CA test scenarios |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121383 |  |
| RP#56 |  | RP-120644 |  | 0297 |  | - |  | Addition of default value of RSRQ Cell Reselection parameters |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121384 |  |
| RP#56 |  | RP-120641 |  | 0298 |  | - |  | Test frequencies for inter-band cells in RRM tests |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121407 |  |
| RP#56 |  | RP-120644 |  | 0299 |  | - |  | Update generic procedure 4.5A.6 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121413 |  |
| RP#56 |  | RP-120644 |  | 0300 |  | - |  | Update generic procedure 4.5A.7 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121414 |  |
| RP#56 |  | RP-120644 |  | 0301 |  | - |  | Update generic procedure 4.5A.3 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121439 |  |
| RP#56 |  | RP-120644 |  | 0302 |  | - |  | Correction to RadioResourceConfigCommon-DEFAULT |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121447 |  |
| RP#56 |  | RP-120641 |  | 0304 |  | - |  | Removing 5.2A.3 for State 2A |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121521 |  |
| RP#56 |  | RP-120641 |  | 0305 |  | - |  | Addition of A-GNSS testing to Test frequencies clause 4.3.1 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121543 |  |
| RP#56 |  | RP-120641 |  | 0306 |  | - |  | Ensuring all SS TX antennas are in use for whole test sequence |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121544 |  |
| RP#56 |  | RP-120641 |  | 0307 |  | - |  | Connection diagram for RRM 3 cell TCs with static propagation |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121556 |  |
| RP#56 |  | RP-120644 |  | 0308 |  | - |  | Correction of Physical Layer configurations tables |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121665 |  |
| RP#56 |  | RP-120644 |  | 0309 |  | - |  | Update of default bandwidth configuration for E-UTRA FDD band 11 and 18 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121712 |  |
| RP#56 |  | RP-120644 |  | 0310 |  | - |  | Correction to default SIB4 contents for CSG cells |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121733 |  |
| RP#56 |  | RP-120644 |  | 0311 |  | - |  | Update of CDMA2000 Band Class data and addition of CDMA2000 Band Class 10 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121734 |  |
| RP#56 |  | RP-120644 |  | 0312 |  | - |  | Update to Reference dedicated EPS bearer context #3 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121735 |  |
| RP#56 |  | RP-120644 |  | 0313 |  | - |  | Add generic procedure MO video call |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121736 |  |
| RP#56 |  | RP-120644 |  | 0314 |  | - |  | Add generic procedure MT video call |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121737 |  |
| RP#56 |  | RP-120644 |  | 0315 |  | - |  | Clarifications to UICC requirements for LTE-C2K testing |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121809 |  |
| RP#56 |  | RP-120644 |  | 0316 |  | - |  | Update UE capability information |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121847 |  |
| RP#56 |  | RP-120658 |  | 0317 |  | - |  | Introduction of System information for PWS |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121854 |  |
| RP#56 |  | RP-120662 |  | 0318 |  | - |  | Update of the default value of Carrier Aggregation parameters |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121856 |  |
| RP#56 |  | RP-120644 |  | 0319 |  | - |  | Correction to reference system configurations for dual mode scenarios |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121888 |  |
| RP#56 |  | RP-120667 |  | 0320 |  | - |  | Adding operating band 26 to TS 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121905 |  |
| RP#56 |  | RP-120641 |  | 0321 |  | - |  | Addition of State 3B-RF in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121919 |  |
| RP#56 |  | RP-120641 |  | 0322 |  | - |  | Correction to Tracking area updating procedure in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121936 |  |
| RP#56 |  | RP-120662 |  | 0323 |  | - |  | Addition of Test mode call procedure for CA test cases |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-121982 |  |
| RP#56 |  | RP-120641 |  | 0303 |  | - |  | Correction to Handover to UTRAN commands for UTRA RRC messages in 36.508 |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-122018 |  |
| RP#56 |  | RP-120662 |  | 0324 |  | - |  | 36.508 - Test frequencies for CA_1C and CA_40C |  | F |  | 10.0.0 |  | 10.1.0 |  | R5-122134 |  |
| RP#57 |  | RP-121098 |  | 0325 |  | - |  | Correction to default message content of UTRA NAS attach accept message |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123114 |  |
| RP#57 |  | RP-121098 |  | 0326 |  | - |  | Correction default message content for Tracking Area Update Request message |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123117 |  |
| RP#57 |  | RP-121095 |  | 0327 |  | - |  | RF: Addition of messages and SIB combination informations for RF MBMS tests |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123211 |  |
| RP#57 |  | RP-121098 |  | 0328 |  | - |  | Clarify requirements for ROHC |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123259 |  |
| RP#57 |  | RP-121098 |  | 0329 |  | - |  | Updates to cl 6.x regarding use of MIMO |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123304 |  |
| RP#57 |  | RP-121113 |  | 0330 |  | - |  | Update of default parameters for Carrier Aggregation |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123305 |  |
| RP#57 |  | RP-121098 |  | 0331 |  | - |  | Addition of default value of Additional update parameters |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123306 |  |
| RP#57 |  | RP-121098 |  | 0332 |  | - |  | Addition of default value of Explicit Signalling Indication parameters |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123307 |  |
| RP#57 |  | RP-121095 |  | 0333 |  | - |  | Update of NeighCellConfig for RRM tests |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123328 |  |
| RP#57 |  | RP-121095 |  | 0334 |  | - |  | Correction to HO commands for UTRAN TDD |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123329 |  |
| RP#57 |  | RP-121116 |  | 0335 |  | - |  | Addition of new connection diagrams for UL-MIMO Testing |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123390 |  |
| RP#57 |  | RP-121098 |  | 0336 |  | - |  | Correction to generic test procedure CS fallback to GERAN with redirection or CCO / MT call(DTM not supported) |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123629 |  |
| RP#57 |  | RP-121098 |  | 0337 |  | - |  | Correction to RRC Connection Reconfiguration message during Handover from UTRA to EUTRA |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123630 |  |
| RP#57 |  | RP-121098 |  | 0338 |  | - |  | Update to Reference dedicated EPS bearer context #3 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123632 |  |
| RP#57 |  | RP-121098 |  | 0339 |  | - |  | Update generic procedure 4.5A.6 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123633 |  |
| RP#57 |  | RP-121098 |  | 0340 |  | - |  | Update generic procedure 4.5A.7 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123634 |  |
| RP#57 |  | RP-121098 |  | 0341 |  | - |  | Update generic procedure 4.5A.8 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123635 |  |
| RP#57 |  | RP-121098 |  | 0342 |  | - |  | Update generic procedure 4.5A.9 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123636 |  |
| RP#57 |  | RP-121098 |  | 0343 |  | - |  | Update the default value of IE RadioResourceConfigCommon and IE additionalSpectrumEmission |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123712 |  |
| RP#57 |  | RP-121098 |  | 0344 |  | - |  | Correction to test procedure 6.4.2.10 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123720 |  |
| RP#57 |  | RP-121098 |  | 0345 |  | - |  | Corrections to CDMA2000 message sequences |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123728 |  |
| RP#57 |  | RP-121098 |  | 0346 |  | - |  | Update of UE Capability Information |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123729 |  |
| RP#57 |  | RP-121098 |  | 0347 |  | - |  | Update of Paging test procedure (for NAS test cases) |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123730 |  |
| RP#57 |  | RP-121098 |  | 0348 |  | - |  | Addition of guard timer to the procedure for IMS signalling |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123731 |  |
| RP#57 |  | RP-121098 |  | 0349 |  | - |  | Update to cl. 6.4.2.7A and 7.2B.1 |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123734 |  |
| RP#57 |  | RP-121113 |  | 0351 |  | - |  | Extension of cell configurations for Carrier Aggregation |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123751 |  |
| RP#57 |  | RP-121113 |  | 0352 |  | - |  | Maximum number of cells simultaneously used in Carrier Aggregation test cases |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123752 |  |
| RP#57 |  | RP-121098 |  | 0353 |  | - |  | Update of default MSPL and MLPL contents in Test UICC |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123767 |  |
| RP#57 |  | RP-121113 |  | 0354 |  | - |  | Correction to the CA test state references |  | F |  | 10.1.0 |  | 10.2.0 |  | R5-123949 |  |
| RP#58 |  | RP-121681 |  | 0356 |  | - |  | Addition of new connection diagram for UL-MIMO testing |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125107 |  |
| RP#58 |  | RP-121659 |  | 0357 |  | - |  | Update to test procedure sequence 6.4.2.7A for check that UE is camped on E-UTRAN cell upon mobility from another RAT |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125116 |  |
| RP#58 |  | RP-121677 |  | 0358 |  | - |  | Introduction of connection diagrams for CA tests |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125187 |  |
| RP#58 |  | RP-121685 |  | 0359 |  | - |  | Addition of default message contents for ESM NOTIFICATION message |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125219 |  |
| RP#58 |  | RP-121659 |  | 0360 |  | - |  | Modification of measurement configuration for UTRAN FDD SIG |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125283 |  |
| RP#58 |  | RP-121659 |  | 0361 |  | - |  | Clarification of table 6.6.2-1 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125287 |  |
| RP#58 |  | RP-121656 |  | 0362 |  | - |  | Addition of channel bandwidth tested for E-UTRA FDD band 19 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125303 |  |
| RP#58 |  | RP-121656 |  | 0363 |  | - |  | Correction of circulator in connection diagrams |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125370 |  |
| RP#58 |  | RP-121656 |  | 0364 |  | - |  | Correction to test channel numbers for Band 26 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125372 |  |
| RP#58 |  | RP-121656 |  | 0365 |  | - |  | Update of Chw Bw Parameters for 1.4MHz and 3MHz |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125373 |  |
| RP#58 |  | RP-121659 |  | 0366 |  | - |  | Update generic procedure 4.5A.7 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125530 |  |
| RP#58 |  | RP-121659 |  | 0367 |  | - |  | Update of default ROUTING AREA UPDATE REQUEST message |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125545 |  |
| RP#58 |  | RP-121690 |  | 0368 |  | - |  | CA_38: Addition of new test frequency for CA band 38 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125573 |  |
| RP#58 |  | RP-121689 |  | 0369 |  | - |  | Addition of Band 28 definition |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125778 |  |
| RP#58 |  | RP-121685 |  | 0370 |  | - |  | Add generic procedure for IMS MO speech and aSRVCC |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125792 |  |
| RP#58 |  | RP-121677 |  | 0371 |  | - |  | Modification to state 3A-RF-CA initial conditions of clause 5.2A.4 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125800 |  |
| RP#58 |  | RP-121688 |  | 0372 |  | - |  | Introduction of Band 27 to TS 36.508 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125832 |  |
| RP#58 |  | RP-121677 |  | 0373 |  | - |  | Update of RF Reference system configurations in 36.508 |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-125933 |  |
| RP#58 |  | RP-121659 |  | 0374 |  | - |  | Using not null integrity protection and ciphering algorithms by default |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-126020 |  |
| RP#58 |  | RP-121677 |  | 0375 |  | - |  | Introducing default channel bandwidth for CA signalling testing |  | F |  | 10.2.0 |  | 10.3.0 |  | R5-126053 |  |
| RP#59 |  | RP-130156 |  | 0377 |  | - |  | Addition of new connection diagram for UL-MIMO testing |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130136 |  |
| RP#59 |  | RP-130145 |  | 0378 |  | - |  | AWGN Level uncertainty for Signalling test cases |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130201 |  |
| RP#59 |  | RP-130144 |  | 0379 |  | - |  | Correction to CDMA2000Parameters message contents |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130202 |  |
| RP#59 |  | RP-130167 |  | 0380 |  | - |  | Addition of SCell configuration type |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130311 |  |
| RP#59 |  | RP-130145 |  | 0382 |  | - |  | Addition of default message contents of measurement information elements for event A4 and A5 measurement test cases |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130363 |  |
| RP#59 |  | RP-130144 |  | 0383 |  | - |  | Correction to ncc-Permitted value to TS 36.508 |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130453 |  |
| RP#59 |  | RP-130167 |  | 0384 |  | - |  | Correction to EARFCN definition for CA_40C |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130477 |  |
| RP#59 |  | RP-130144 |  | 0385 |  | - |  | Addition of combination A2+A11 and condition "UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH in Table 4.7B.1-1: HANDOVER TO UTRAN COMMAND |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130561 |  |
| RP#59 |  | RP-130144 |  | 0386 |  | - |  | Update of Band 18 configuration |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130619 |  |
| RP#59 |  | RP-130144 |  | 0387 |  | - |  | Update generic procedure 4.5.2 |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130620 |  |
| RP#59 |  | RP-130144 |  | 0388 |  | - |  | Update generic procedure 4.5.2A |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130621 |  |
| RP#59 |  | RP-130144 |  | 0389 |  | - |  | Add generic procedure MO add video |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130622 |  |
| RP#59 |  | RP-130144 |  | 0390 |  | - |  | Add generic procedure MT add video |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130623 |  |
| RP#59 |  | RP-130167 |  | 0391 |  | - |  | Correction of test frequencies for CA signalling tests |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130692 |  |
| RP#59 |  | RP-130146 |  | 0395 |  | - |  | Addition of simulated cell for E-UTRA dual mode multi cell network scenarios |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130709 |  |
| RP#59 |  | RP-130167 |  | 0397 |  | - |  | Corrections to Annex A general considerations on connections for CA testing |  | F |  | 10.3.0 |  | 10.4.0 |  | R5-130900 |  |
| RP#59 |  | RP-130165 |  | 0376 |  | - |  | Addition of test frequencies of CA_1A-19A and CA_1A-21A for CA signalling testing |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130087 |  |
| RP#59 |  | RP-130165 |  | 0381 |  | - |  | Addition of test frequencies for CA_7C and CA_41C signalling test |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130343 |  |
| RP#59 |  | RP-130165 |  | 0392 |  | - |  | Addition of test frequencies of CA_38, CA_3-7 and CA_7-20 for CA signalling test |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130694 |  |
| RP#59 |  | RP-130165 |  | 0393 |  | - |  | Addition of test frequencies of CA_4A-5A and CA_4A-13A for CA signalling testing |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130695 |  |
| RP#59 |  | RP-130165 |  | 0394 |  | - |  | Updates of 6.2.3.2 Test frequency for CA_1A-18A and 11A-18A |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130696 |  |
| RP#59 |  | RP-130164 |  | 0396 |  | - |  | Adding operating band 44 to TS36.508 |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130710 |  |
| RP#59 |  | RP-130165 |  | 0398 |  | - |  | Addition of new test frequencies for CA band 7 and band 41 |  | F |  | 10.4.0 |  | 11.0.0 |  | R5-130955 |  |
| RP#60 |  | RP-130609 |  | 0399 |  | - |  | Update of CDMA2000 specification references: TS 36.508 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131067 |  |
| RP#60 |  | RP-130609 |  | 0401 |  | - |  | Editorial Fix Hysteris values for event B1&B2 ReportConfigInterRAT for GERAN |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131090 |  |
| RP#60 |  | RP-130622 |  | 0402 |  | - |  | eICIC: Connection diagram for 2x2 antenna configuration scenarios |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131116 |  |
| RP#60 |  | RP-130636 |  | 0403 |  | - |  | CA: Connection diagrams for RRM tests |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131117 |  |
| RP#60 |  | RP-130609 |  | 0404 |  | - |  | Clarification to ncc-Permitted value interpretation in TS 36.508 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131322 |  |
| RP#60 |  | RP-130626 |  | 0405 |  | - |  | Maintenance of Band 23 Requirements in TS 36.508 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131462 |  |
| RP#60 |  | RP-130626 |  | 0406 |  | - |  | Adding IE T3412 extended value |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131474 |  |
| RP#60 |  | RP-130611 |  | 0408 |  | - |  | Addition of exception for p-MaxGERAN of SIB7 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131583 |  |
| RP#60 |  | RP-130627 |  | 0409 |  | - |  | Correction of test bandwidth for band 44 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131611 |  |
| RP#60 |  | RP-130631 |  | 0410 |  | - |  | Addition of default parameters for ePDCCH |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131638 |  |
| RP#60 |  | RP-130630 |  | 0411 |  | - |  | Addition of default parameters for CoMP |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131667 |  |
| RP#60 |  | RP-130628 |  | 0412 |  | - |  | Addition of test frequencies for CA_2A-17A and CA_4A-17A for signalling testing |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131704 |  |
| RP#60 |  | RP-130611 |  | 0413 |  | - |  | Editorial update of generic procedure 4.5.2 |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131804 |  |
| RP#60 |  | RP-130611 |  | 0414 |  | - |  | Editorial update of generic procedure 4.5.2A |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131805 |  |
| RP#60 |  | RP-130609 |  | 0415 |  | - |  | Correction of DRB Logical Channel configuration |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131806 |  |
| RP#60 |  | RP-130611 |  | 0416 |  | - |  | Add generic procedure for XCAP establishment |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131807 |  |
| RP#60 |  | RP-130611 |  | 0417 |  | - |  | Corrections to default messages for eMBMS testing |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131809 |  |
| RP#60 |  | RP-130611 |  | 0418 |  | - |  | Complete the default MEASUREMENT CONTROL MESSAGE for LCR_TDD_UTRAN |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131872 |  |
| RP#60 |  | RP-130636 |  | 0419 |  | - |  | Carrier Aggregation: PCC and SCC Configuration Update |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131885 |  |
| RP#60 |  | RP-130609 |  | 0420 |  | - |  | Update of UECapabilityInformation for Rel-11 UEs |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131888 |  |
| RP#60 |  | RP-130636 |  | 0421 |  | - |  | Modifications to Common Section CA settings |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132002 |  |
| RP#60 |  | RP-130636 |  | 0422 |  | - |  | Correction to test frequencies for CA signalling test |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132027 |  |
| RP#60 |  | RP-130629 |  | 0423 |  | - |  | Modification of default contents for UEInformationRequest message |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132029 |  |
| RP#60 |  | RP-130628 |  | 0424 |  | - |  | Addition of test frequencies of CA_4-12, CA_5-12 for CA signalling test |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132041 |  |
| RP#60 |  | RP-130611 |  | 0426 |  | - |  | Addition of generic procedure for IMS call release |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132066 |  |
| RP#60 |  | RP-130609 |  | 0425 |  | - |  | Update of CSIM and USIM EFs |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132082 |  |
| RP#60 |  | RP-130609 |  | 0427 |  | - |  | Corrections to setup of dedicated EPS bearer contexts in IMS generic procedures |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132085 |  |
| RP#60 |  | RP-130636 |  | 0428 |  | - |  | Modifications to RF Section CA settings |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-132106 |  |
| RP#60 |  | RP-130610 |  | 0429 |  | - |  | Correction to generic MO CSFB procedures |  | F |  | 10.4.0 |  | 11.1.0 |  | R5-131496 |  |
| RP#60 |  | - |  | - |  | - |  | Correction in history table from v11.0.0 to v11.1.0 |  | F |  | 11.0.0 |  | 11.1.1 |  |  |  |
| RP#61 |  | RP-131115 |  | 0460 |  | - |  | Addition of frequency f2 to band combinations CA_2A-17A and CA_4A-17A |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133094 |  |
| RP#61 |  | RP-131115 |  | 0430 |  | - |  | Addition of test frequencies of CA_3-8 for CA signaling test |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133113 |  |
| RP#61 |  | RP-131103 |  | 0431 |  | - |  | Addition of option to use IP addresses from PCO IE in ESM INFORMATION RESPONSE |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133116 |  |
| RP#61 |  | RP-131101 |  | 0432 |  | - |  | Update of CSIM Elementary File for EPRL |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133118 |  |
| RP#61 |  | RP-131103 |  | 0433 |  | - |  | Correction to SystemInformationBlockType13 message (eMBMS testing) |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133159 |  |
| RP#61 |  | RP-131114 |  | 0434 |  | - |  | Correction to default MBMS Counting messages |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133160 |  |
| RP#61 |  | RP-131111 |  | 0435 |  | - |  | CA RRM: Phase rotator for intra-frequency static scenarios |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133224 |  |
| RP#61 |  | RP-131125 |  | 0436 |  | - |  | Addition of Band 31 to 36.508 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133246 |  |
| RP#61 |  | RP-131111 |  | 0437 |  | - |  | Test frequencies for RF intra-band contiguous CA tests |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133270 |  |
| RP#61 |  | RP-131115 |  | 0438 |  | - |  | Update of PUCCH-ConfigDedicated-v1020-DEFAULT for TDD |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133275 |  |
| RP#61 |  | RP-131101 |  | 0439 |  | - |  | Default Bandwidth Configuration for E-UTRA Band 2 RF Testing |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133306 |  |
| RP#61 |  | RP-131116 |  | 0440 |  | - |  | Modification of default contents for UEInformationRequest message |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133321 |  |
| RP#61 |  | RP-131115 |  | 0441 |  | - |  | Correction of reference to cell frequencies for CA signalling test cases |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133348 |  |
| RP#61 |  | RP-131101 |  | 0442 |  | - |  | Correction to SID value |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133365 |  |
| RP#61 |  | RP-131101 |  | 0443 |  | - |  | Update of generic procedure 4.5A.7 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133366 |  |
| RP#61 |  | RP-131101 |  | 0444 |  | - |  | Update of generic procedure 4.5A.14 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133367 |  |
| RP#61 |  | RP-131111 |  | 0445 |  | - |  | Update of Generic RRM procedures for CA |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133451 |  |
| RP#61 |  | RP-131113 |  | 0446 |  | - |  | Default Message Contents for eICIC PCell Pattern |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133453 |  |
| RP#61 |  | RP-131103 |  | 0447 |  | - |  | Corrections to the reference dedicated EPS bearer contexts |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133509 |  |
| RP#61 |  | RP-131101 |  | 0448 |  | - |  | Extension of default contents of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUESTS message |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133554 |  |
| RP#61 |  | RP-131101 |  | 0449 |  | - |  | Corrections to allow ISIM or USIM to be used in test cases using IMS |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133582 |  |
| RP#61 |  | RP-131101 |  | 0450 |  | - |  | Correction of Feature Group Indicators in Table 4.6.1-23 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133583 |  |
| RP#61 |  | RP-131101 |  | 0451 |  | - |  | Correction to ROUTING AREA UPDATE ACCEPT default message contents |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133584 |  |
| RP#61 |  | RP-131115 |  | 0452 |  | - |  | Addition of test frequencies of CA_3A-5A for CA signalling test |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133614 |  |
| RP#61 |  | RP-131115 |  | 0453 |  | - |  | Extension of EUTRA CA test frequency configurations |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133615 |  |
| RP#61 |  | RP-131113 |  | 0454 |  | - |  | Update of Measurement information element |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133645 |  |
| RP#61 |  | RP-131117 |  | 0455 |  | - |  | Introduction of new default messages for eMBMS service continuity testing |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133661 |  |
| RP#61 |  | RP-131117 |  | 0456 |  | - |  | Introduction of system information combinations for eMBMS service continuity testing |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133662 |  |
| RP#61 |  | RP-131118 |  | 0457 |  | - |  | Addition of specific message formats for eDDA test cases. |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133674 |  |
| RP#61 |  | RP-131101 |  | 0458 |  | - |  | Missing agreed change in 36.508 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133704 |  |
| RP#61 |  | RP-131101 |  | 0459 |  | - |  | Clean up of 36.508 |  | F |  | 11.1.1 |  | 11.2.0 |  | R5-133714 |  |
| RP#62 |  | RP-131864 |  | 0461 |  | - |  | Correction of MBSFNAreaConfiguration and SIB combination 15 |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134152 |  |
| RP#62 |  | RP-131874 |  | 0465 |  | - |  | Correction to MeasConfig-DEFAULT |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134459 |  |
| RP#62 |  | RP-131876 |  | 0466 |  | - |  | General clarification to CA test frequency tables for signalling |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134642 |  |
| RP#62 |  | RP-131861 |  | 0467 |  | - |  | Update to MeasConfig-DEFAULT |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134645 |  |
| RP#62 |  | RP-131861 |  | 0469 |  | - |  | Editorial correction to generic MO CSFB procedures |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134726 |  |
| RP#62 |  | RP-131864 |  | 0470 |  | - |  | Corrections to default message contents of MBMSCountingRequest |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134727 |  |
| RP#62 |  | RP-131876 |  | 0471 |  | - |  | Extension of default settings for the Elementary Files on Test UICC |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134730 |  |
| RP#62 |  | RP-131877 |  | 0472 |  | - |  | Addition of CA band combinations CA_2A_29A, CA_4A_29A and CA_5A_17A |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134786 |  |
| RP#62 |  | RP-131873 |  | 0475 |  | - |  | Addition of connection diagrams for CA CSI tests |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134803 |  |
| RP#62 |  | RP-131873 |  | 0477 |  | - |  | Correction to PhysicalConfigDedicated-DEFAULT for SCell_AddMod |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134903 |  |
| RP#62 |  | RP-131873 |  | 0478 |  | - |  | Clarification of multi antenna ports for all neighboring cells |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134904 |  |
| RP#62 |  | RP-131877 |  | 0480 |  | - |  | Addition of B29 test frequencies to 36.508 |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-134986 |  |
| RP#62 |  | RP-131861 |  | 0481 |  | - |  | Corrections to procedures for State 3A-RF |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-135013 |  |
| RP#62 |  | RP-131864 |  | 0468 |  | - |  | Correction to Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-135069 |  |
| RP#62 |  | RP-131864 |  | 0479 |  | - |  | Correction of generic test procedure for IMS Emergency Call establishment in EUTRA limited service |  | F |  | 11.2.0 |  | 11.3.0 |  | R5-135070 |  |
| RP#62 |  | RP-131890 |  | 0462 |  | - |  | Addition of test frequencies for CA_3C |  | F |  | 11.3.0 |  | 12.0.0 |  | R5-134336 |  |
| RP#62 |  | RP-131890 |  | 0463 |  | - |  | Updates of 6.2.3.2 Test frequencies for CA_1A-26A |  | F |  | 11.3.0 |  | 12.0.0 |  | R5-134368 |  |
| RP#62 |  | RP-131890 |  | 0464 |  | - |  | Addition of CA band combination CA_2A_5A |  | F |  | 11.3.0 |  | 12.0.0 |  | R5-134426 |  |
| RP#62 |  | RP-131873 |  | 0473 |  | - |  | Addition of test frequencies of CA_3A-19A and CA_19A-21A for CA signalling testing |  | F |  | 11.3.0 |  | 12.0.0 |  | R5-134790 |  |
| RP#62 |  | RP-131890 |  | 0474 |  | - |  | Addition of test frequencies for CA_3C signalling test |  | F |  | 11.3.0 |  | 12.0.0 |  | R5-134791 |  |
| RP#63 |  | RP-140329 |  | 0482 |  | - |  | Adding default message content for SystemInformationBlockType14 |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140075 |  |
| RP#63 |  | RP-140307 |  | 0484 |  | - |  | Correction of UTRA cell carrier frequency in Table 6.3.1.3-2 |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140136 |  |
| RP#63 |  | R5-140319 |  | 0485 |  | - |  | CA RF: Adding connection diagram for CA 4x2 MIMO with fading scenarios |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140242 |  |
| RP#63 |  | R5-140319 |  | 0486 |  | - |  | eDL-MIMO RF: Adding connection diagram for 8x2 MIMO scenarios |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140243 |  |
| RP#63 |  | RP-140307 |  | 0487 |  | - |  | Correction to Table 6.4.2.7A.1-1 for TAU Request |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140325 |  |
| RP#63 |  | RP-140308 |  | 0488 |  | - |  | Update of MBMS configurations |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140394 |  |
| RP#63 |  | RP-140332 |  | 0489 |  | - |  | Addtion of test frequencies for CA_1A-8A |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140406 |  |
| RP#63 |  | R5-140319 |  | 0490 |  | - |  | Correction to UL CA message exceptions |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140503 |  |
| RP#63 |  | RP-140308 |  | 0492 |  | - |  | Correction to Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140536 |  |
| RP#63 |  | R5-140319 |  | 0493 |  | - |  | Update to QuantityConfig in MeasConfig DEFAULT |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140743 |  |
| RP#63 |  | RP-140331 |  | 0494 |  | - |  | Correction of CA band combinations CA_1A_18A and CA_11A_18A |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140744 |  |
| RP#63 |  | R5-140319 |  | 0495 |  | - |  | Adding test frequencies for CA reverse inter band combination for bands 4 and 13 |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140745 |  |
| RP#63 |  | RP-140307 |  | 0496 |  | - |  | Update to Generic test procedure in TS 36.508 Table 6.4.2.7A-2 |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140746 |  |
| RP#63 |  | RP-140307 |  | 0497 |  | - |  | Addition of procedure for EPS Bearer Deactivaton and deletion of procedure for MT release of IMS call |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140747 |  |
| RP#63 |  | RP-140332 |  | 0498 |  | - |  | Addition of Rel12 reverse band combination CA_2A-5A |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140789 |  |
| RP#63 |  | RP-140325 |  | 0499 |  | - |  | Addition of neighbour cell CRS and dedicated SIB-1 IE for fEICIC test cases |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140830 |  |
| RP#63 |  | R5-140319 |  | 0500 |  | - |  | Addition of transmissionMode-r10 for SCC |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140928 |  |
| RP#63 |  | RP-140331 |  | 0501 |  | - |  | Addition of Rel11 reverse band combinations CA_2A-29A, CA_4A-5A, CA_4A-29A, CA_5A-17A |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-140998 |  |
| RP#63 |  | RP-140302 |  | 0502 |  | - |  | Change of test frequencies for Band 28 |  | F |  | 12.0.0 |  | 12.1.0 |  | R5-141039 |  |
| RP#64 |  | RP-140838 |  | 0503 |  | - |  | Addition of CA 3A-28A to 36.508 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142114 |  |
| RP#64 |  | RP-140812 |  | 0504 |  | - |  | Update to MEASUREMENT CONTROL |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142126 |  |
| RP#64 |  | RP-140815 |  | 0505 |  | - |  | Update to default non-MBSFNregionLength in SystemInformationBlockType13 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142127 |  |
| RP#64 |  | RP-140815 |  | 0506 |  | - |  | Editorial Update to Table 4.6.1-4A: MBSFNAreaConfiguration |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142128 |  |
| RP#64 |  | RP-140837 |  | 0507 |  | - |  | Correction to default SystemInformationBlockType15 message for eMBMS testing |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142254 |  |
| RP#64 |  | RP-140838 |  | 0508 |  | - |  | Updates of 6.2.3.2 Test frequency for CA_3A-26A and CA_3A-27A |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142299 |  |
| RP#64 |  | RP-140832 |  | 0509 |  | - |  | Adding connection diagramms for RF/RRM Rel-11 WI-s tests |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142318 |  |
| RP#64 |  | RP-140832 |  | 0510 |  | - |  | Addition of exceptions for feICIC RRM test cases |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142336 |  |
| RP#64 |  | RP-140812 |  | 0511 |  | - |  | Correction to NAS UTRA Routing Area Update Accept message |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142397 |  |
| RP#64 |  | RP-140812 |  | 0512 |  | - |  | Correction to the exception of SIB2 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142602 |  |
| RP#64 |  | RP-140812 |  | 0513 |  | - |  | Addition of 4Tx for RF demodulation test cases |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142603 |  |
| RP#64 |  | RP-140838 |  | 0514 |  | - |  | Addtion of test frequencies for CA_2A-4A and CA_5A-7A |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142768 |  |
| RP#64 |  | RP-140837 |  | 0515 |  | - |  | Correction to system information combination 16 and 19 for eMBMS testing |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142844 |  |
| RP#64 |  | RP-140817 |  | 0516 |  | - |  | Update to ri-ConfigIndex in Table 4.6.3-2AC CQI-ReportPeriodic-r10-DEFAULT |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142845 |  |
| RP#64 |  | RP-140815 |  | 0517 |  | - |  | Correction to MFBI Frequencies in 36.508 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142846 |  |
| RP#64 |  | RP-140838 |  | 0518 |  | - |  | Addition of test frequencies of CA_39A-41A for CA signalling testing in TS 36.508 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142928 |  |
| RP#64 |  | RP-140838 |  | 0519 |  | - |  | Updates to TS 36.508 for CA_27B sianlling test frequencies defined in section 6.2.3.2 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-142930 |  |
| RP#64 |  | RP-140817 |  | 0520 |  | - |  | Clarification of RRC message definitions |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-143001 |  |
| RP#64 |  | RP-140817 |  | 0521 |  | - |  | Corrections to MBMS information elements in SIB2 and SIB13 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-143002 |  |
| RP#64 |  | RP-140838 |  | 0522 |  | - |  | Addition of new test frequency for CA_39C |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-143132 |  |
| RP#64 |  | RP-140838 |  | 0523 |  | - |  | Updates to TS 36.508 for CA_27B test frequencies defined in section 4.3.1.1 |  | F |  | 12.1.0 |  | 12.2.0 |  | R5-143171 |  |
| RP#65 |  | RP-141573 |  | 0524 |  | - |  | Correction to notificationSF-Index-r9 in Sib13 |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144185 |  |
| RP#65 |  | RP-141571 |  | 0525 |  | - |  | IMS APN pre-configured IR.92 devices |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144370 |  |
| RP#65 |  | RP-141571 |  | 0526 |  | - |  | Removal of transition period for Band 28 test frequency definition in 36.508 |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144459 |  |
| RP#65 |  | RP-141593 |  | 0527 |  | - |  | Addition of Rel11 reverse band combinations CA_1A-18A, CA_11A-18A |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144543 |  |
| RP#65 |  | RP-141594 |  | 0528 |  | - |  | Addition of Rel12 reverse band combination CA_1A-26A |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144544 |  |
| RP#65 |  | RP-141593 |  | 0529 |  | - |  | Editorial correction to Table 6.2.3.2-2 |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144644 |  |
| RP#65 |  | RP-141594 |  | 0530 |  | - |  | Addition of reverse band combinations, CA_1A-19A, CA_1A-21A, CA_3A-19A and CA_19A-21A |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144685 |  |
| RP#65 |  | RP-141587 |  | 0531 |  | - |  | Addition of default parameters for Enhanced downlink control channel(s) for LTE Advanced |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144711 |  |
| RP#65 |  | RP-141593 |  | 0532 |  | - |  | Removal of FFS from IEs triggered by CoMP related TCs specification |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144712 |  |
| RP#65 |  | RP-141571 |  | 0533 |  | - |  | Clarification on the default setting of Security header type in SECURITY PROTECTED NAS MESSAGE |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144750 |  |
| RP#65 |  | RP-141571 |  | 0534 |  | - |  | Support of 2nd PDN connectivity at UE registration |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144797 |  |
| RP#65 |  | RP-141575 |  | 0535 |  | - |  | CA: New structure for test frequencies for intra-band contiguous CA |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144890 |  |
| RP#65 |  | RP-141575 |  | 0536 |  | - |  | Correction to 7.2B.1 Tracking Area Updating procedure |  | F |  | 12.2.0 |  | 12.3.0 |  | R5-144929 |  |
| RP#65 |  | - |  | - |  | - |  | Correction concerning R5-144797 to resolve the 4.5A.X, 4.5A.Y, 4.5A.Z reference in Table 4.5.2.3-1 |  | - |  | 12.3.0 |  | 12.3.1 |  | - |  |
| RP#66 |  | RP-142075 |  | 0538 |  | - |  | New CA band combination CA_1A-3A - Updates of 6.2.3.2 Test Frequency |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145181 |  |
| RP#66 |  | RP-142076 |  | 0539 |  | - |  | Addition of E-UTRA FDD Band 30 to TS 36.508, subclause 6.2.3.1. |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145606 |  |
| RP#66 |  | RP-142054 |  | 0540 |  | - |  | Correction to SIB8 Default Message Contents for LTE<>1XRTT |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145657 |  |
| RP#66 |  | RP-142057 |  | 0541 |  | - |  | Correction to Test frequencies for E-UTRA FDD(5MHz) for MFBI |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145658 |  |
| RP#66 |  | RP-142057 |  | 0542 |  | - |  | Updating dataCodingScheme of CMAS TC in 36.508 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145659 |  |
| RP#66 |  | RP-142074 |  | 0543 |  | - |  | Correction of default parameters for Enhanced downlink control channel(s) for LTE Advanced |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145661 |  |
| RP#66 |  | RP-142074 |  | 0544 |  | - |  | Correction to eMBMS multiband test cases |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145662 |  |
| RP#66 |  | RP-142059 |  | 0545 |  | - |  | Harmonization of CA terminology (36.508) |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145663 |  |
| RP#66 |  | RP-142054 |  | 0546 |  | - |  | Correction to Test Frequencies for FDD Band 12->17 for 5MHZ |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145686 |  |
| RP#66 |  | RP-142075 |  | 0547 |  | - |  | Addition of CA_1A-28A in 36.508 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145715 |  |
| RP#66 |  | RP-142075 |  | 0548 |  | - |  | Introduction of 1+11 and 8+11 in 36.508 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145716 |  |
| RP#66 |  | RP-142075 |  | 0549 |  | - |  | Addition of Intra-Band Non-Contiguous Default Frequencies to 36.508 Chapter 6 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145717 |  |
| RP#66 |  | RP-142078 |  | 0550 |  | - |  | Addition of CA_18A-28A in 36.508 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145718 |  |
| RP#66 |  | RP-142075 |  | 0551 |  | - |  | Existing CA band combination CA_39C: update test frequencies for CA_39C signalling test |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145739 |  |
| RP#66 |  | RP-142054 |  | 0552 |  | - |  | Correction to generic procedure for EPS Bearer Deactivation |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145774 |  |
| RP#66 |  | RP-142054 |  | 0553 |  | - |  | Corrections for Generic Test Procedure for IMS MO/MT add video establishment in E-UTRA (clause 4.5A.11 and 4.5A.12) |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145779 |  |
| RP#66 |  | RP-142054 |  | 0554 |  | - |  | Support of 2nd PDN connectivity at UE registration - XCAP |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145787 |  |
| RP#66 |  | RP-142054 |  | 0555 |  | - |  | Correction of CSIM Elementary File for EPRL |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145792 |  |
| RP#66 |  | RP-142059 |  | 0556 |  | - |  | Correction of SCell_AddMod condition in MAC Configurations |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145793 |  |
| RP#66 |  | RP-142059 |  | 0557 |  | - |  | Addition of CA_4A-7A and CA_3A-20A Test Frequencies |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145799 |  |
| RP#66 |  | RP-142076 |  | 0558 |  | - |  | Addition of operating band 30 to TS 36.508, subclause 4.3.1.1.30 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145850 |  |
| RP#66 |  | RP-142059 |  | 0559 |  | - |  | Addition of editor's note for MFBI Test frequencies |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145879 |  |
| RP#66 |  | RP-142079 |  | 0560 |  | - |  | Clarifications to connection diagrams |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145897 |  |
| RP#66 |  | RP-142059 |  | 0562 |  | - |  | Correction to message exceptions for feICIC tests |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145916 |  |
| RP#66 |  | RP-142075 |  | 0563 |  | - |  | CA: Test frequencies for intra-band non-contiguous |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145923 |  |
| RP#66 |  | RP-142075 |  | 0564 |  | - |  | Introduction of CA_42C for TS36.508 |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145959 |  |
| RP#66 |  | RP-142059 |  | 0565 |  | - |  | TS 36.508: Correction to CA intra-band contiguous test frequencies |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145962 |  |
| RP#66 |  | RP-142054 |  | 0566 |  | - |  | Correction to generic procedure for UE Registration, pre-registration on 1xRTT (State 2C) |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145971 |  |
| RP#66 |  | RP-142054 |  | 0561 |  | - |  | UE registration for RF and RRM testing |  |  |  | 12.3.1 |  | 12.4.0 |  | R5-145979 |  |
| - |  | - |  | - |  | - |  | correction of closing bracket in Table 5.2A.5.1.1-2 of R5-145916 |  |  |  | 12.4.0 |  | 12.4.1 |  | - |  |
| - |  | - |  | - |  | - |  | correction of missing changes of R5-145787 |  |  |  | 12.4.0 |  | 12.4.1 |  | - |  |
| RP#67 |  | RP-150342 |  | 0567 |  | - |  | Correction to CA_19A-21A, CA_39A-41A |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150103 |  |
| RP#67 |  | RP-150323 |  | 0568 |  | - |  | Support of 2nd PDN connectivity at UE registration - Corrections for C2K |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150127 |  |
| RP#67 |  | RP-150326 |  | 0570 |  | - |  | Correction of MFBI Test Frequencies for Band 38->41 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150273 |  |
| RP#67 |  | RP-150341 |  | 0571 |  | - |  | Addition of CA_8A-20A to Clause 6.2.3.2 of TS 36.508 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150367 |  |
| RP#67 |  | RP-150343 |  | 0572 |  | - |  | Addition of CA_1A-20A to Clause 6.2.3.2 of TS 36.508 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150480 |  |
| RP#67 |  | RP-150342 |  | 0575 |  | - |  | Correction to TDD CA contiguous Intra-band setting for signalling test |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150679 |  |
| RP#67 |  | RP-150342 |  | 0576 |  | - |  | Addition of Signalling test frequencies to 36.508 for CA_1A-7A |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150680 |  |
| RP#67 |  | RP-150343 |  | 0577 |  | - |  | Addition of CA_2A-30A to Clause 6.2.3.2 of TS 36.508. |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150682 |  |
| RP#67 |  | RP-150343 |  | 0578 |  | - |  | Addition of CA_4A-30A to Clause 6.2.3.2 of TS 36.508. |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150683 |  |
| RP#67 |  | RP-150343 |  | 0579 |  | - |  | Addition of CA_5A-30A to Clause 6.2.3.2 of TS 36.508. |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150684 |  |
| RP#67 |  | RP-150342 |  | 0580 |  | - |  | Addition of Signalling test frequencies to 36.508 for CA_5A-25A and CA_12A-25A |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150722 |  |
| RP#67 |  | RP-150343 |  | 0581 |  | - |  | Correction to CA_18A-28A SIG test frequency in 36.508 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150743 |  |
| RP#67 |  | RP-150342 |  | 0582 |  | - |  | Addition of test frequency table for Band 23B to 36.508 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150859 |  |
| RP#67 |  | RP-150343 |  | 0583 |  | - |  | Addition of test frequencies for CA_18A-28A in 36.508 |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150864 |  |
| RP#67 |  | RP-150343 |  | 0584 |  | - |  | Addition of 3DL Intra-band Contiguous and Non-contiguous Test Frequencies |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150895 |  |
| RP#67 |  | RP-150342 |  | 0585 |  | - |  | Addition of further test frequencies for Intra-band Non-contiguous 2DL CA |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150896 |  |
| RP#67 |  | RP-150343 |  | 0586 |  | - |  | Update of SCC configuration for 3DL CA RF/RRM tests |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150912 |  |
| RP#67 |  | RP-150345 |  | 0569 |  | - |  | Addition of UE Positioning testing to State 3A |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150933 |  |
| RP#67 |  | RP-150342 |  | 0573 |  | - |  | Addition of Signalling test frequencies to 36.508 for CA_23B |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150936 |  |
| RP#67 |  | RP-150342 |  | 0574 |  | - |  | Addition of Signalling test frequencies to 36.508 for CA_23A-29A |  | F |  | 12.4.1 |  | 12.5.0 |  | R5-150937 |  |
| RP#68 |  | RP-150886 |  | 0587 |  | - |  | APN IE requirements for IMS emergency PDN |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151104 |  |
| RP#68 |  | RP-150900 |  | 0588 |  | - |  | CA RF: Connection diagrams for 3DL CA (RF) testing |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151113 |  |
| RP#68 |  | RP-150905 |  | 0592 |  | - |  | Addition of band 32 to 36.508 |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151327 |  |
| RP#68 |  | RP-150912 |  | 0594 |  | - |  | Updates to 36.508 for CA_7C, CA_40C, CA_41C, CA_41D |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151365 |  |
| RP#68 |  | RP-150888 |  | 0595 |  | - |  | Change of default Network Signalling Value for Uplink Carrier aggregation test cases |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151423 |  |
| RP#68 |  | RP-150900 |  | 0602 |  | - |  | Addition of test frequencies for CA_1A-18A-28A in 36 508 |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151582 |  |
| RP#68 |  | RP-150900 |  | 0604 |  | - |  | Correction to test frequencies for CA_18A-28A in 36.508 |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151626 |  |
| RP#68 |  | RP-150900 |  | 0607 |  | - |  | 36.508: Test frequencies definition for CA_12B |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151658 |  |
| RP#68 |  | RP-150900 |  | 0608 |  | - |  | 36.508: Signalling test frequencies definition for CA_12B |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151660 |  |
| RP#68 |  | RP-150888 |  | 0601 |  | 1 |  | Addition of Carrier Aggregation band combination 2 to 13 |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151737 |  |
| RP#68 |  | RP-150912 |  | 0603 |  | 1 |  | Removal of SIB1 transmission in 5.2A.5 Exceptions for feICIC tests |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151801 |  |
| RP#68 |  | RP-150888 |  | 0600 |  | 1 |  | Addition of exception of RRC Connection Setup(UTRA) |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151804 |  |
| RP#68 |  | RP-150900 |  | 0591 |  | 1 |  | Addition of 2DL CA and 3DL CA Frequencies to 36.508 Chapter 6 |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151967 |  |
| RP#68 |  | RP-150909 |  | 0590 |  | 1 |  | Update of default SIB1 for Low Cost MTC testing |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151975 |  |
| RP#68 |  | RP-150906 |  | 0596 |  | 2 |  | Add generic procedure for IMS MO speech for EVS |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151976 |  |
| RP#68 |  | RP-150903 |  | 0598 |  | 1 |  | Introduction of EUTRA-WLAN interworking test environment parameters and message contents |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-151995 |  |
| RP#68 |  | RP-150908 |  | 0589 |  | 1 |  | Addition of Device to Device Proximity Services Test Environment |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-152058 |  |
| RP#68 |  | RP-150903 |  | 0599 |  | 1 |  | Introduction of generic procedures for EUTRA-WLAN interworking |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-152117 |  |
| RP#68 |  | RP-150888 |  | 0609 |  | 1 |  | Correction to default E-UTRA channel bandwidth combination for CA_39C |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-152140 |  |
| RP#68 |  | RP-150903 |  | 0610 |  | 1 |  | Update to registration procedure to support multi PDN for RAN assisted WLAN interworking |  | F |  | 12.5.0 |  | 12.6.0 |  | R5-152141 |  |
| RP#69 |  | RP-151409 |  | 0612 |  | - |  | Prevent unnecessary IMS signalling over GERAN for non-IMS related test cases |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153114 |  |
| RP#69 |  | RP-151409 |  | 0613 |  | - |  | Correct the IMS APN format |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153149 |  |
| RP#69 |  | RP-151429 |  | 0614 |  | - |  | Introduction of new RRC default message - SidelinkUEInformation |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153188 |  |
| RP#69 |  | RP-151412 |  | 0623 |  | - |  | Correction to Table 4.4.2-1A to change root sequence index for Cell 30 |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153408 |  |
| RP#69 |  | RP-151427 |  | 0631 |  | - |  | Add generic procedure for IMS MT speech for EVS |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153539 |  |
| RP#69 |  | RP-151412 |  | 0625 |  | 1 |  | Update the default configuration of channel bandwidth for Band 42 for signalling testing |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153715 |  |
| RP#69 |  | RP-151409 |  | 0628 |  | 1 |  | New generic procedure 4.5A.21 Generic Test Procedure for IMS MO Customized Alerting Tones and speech establishment in E-UTRA |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153716 |  |
| RP#69 |  | RP-151412 |  | 0630 |  | 1 |  | Correction to PhysicalConfigDedicated-DEFAULT |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153717 |  |
| RP#69 |  | RP-151423 |  | 0624 |  | 1 |  | Addition and modification of 2DL CA Frequencies and CA Channel Bandwidth combination |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153749 |  |
| RP#69 |  | RP-151423 |  | 0640 |  | - |  | Introduction of test environment information for intra-band Carrier Aggregation combinations CA_2C |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153751 |  |
| RP#69 |  | RP-151423 |  | 0636 |  | 1 |  | Introduction of Default E-UTRA channel bandwidth combinations for CA Signalling testing with three component carriers |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153752 |  |
| RP#69 |  | RP-151423 |  | 0638 |  | 1 |  | Introduction of test environment and test frequency information for several new 3DL inter-band Carrier Aggregation combinations with two different bands to 36.508. |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153753 |  |
| RP#69 |  | RP-151423 |  | 0627 |  | 1 |  | Addition of CA_2C to Chapter 4 |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153820 |  |
| RP#69 |  | RP-151433 |  | 0629 |  | 1 |  | Introduce connection diagram for multi-cell Enhanced requirements Type C tests |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153835 |  |
| RP#69 |  | RP-151430 |  | 0616 |  | 1 |  | UE Cat 0: Connection diagrams for RF/RRM testing |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153850 |  |
| RP#69 |  | RP-151423 |  | 0611 |  | 1 |  | CA RF: Update of connection diagrams for 3CCs (RF) testing for more than 3CCs |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153875 |  |
| RP#69 |  | RP-151423 |  | 0621 |  | 1 |  | Update of 4.4.2 Simulated cells for CA RRM test cases |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153876 |  |
| RP#69 |  | RP-151409 |  | 0618 |  | 1 |  | Update to Loopback activation procedure for IMS enabled UE |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153971 |  |
| RP#69 |  | RP-151429 |  | 0615 |  | 2 |  | Update to Device to Device Proximity Services Test Environment |  | F |  | 12.6.0 |  | 12.7.0 |  | R5-153998 |  |
| RP#69 |  | - |  | - |  | - |  | update of the "non-specific references" in section 2 according to the approved R5-153582 and an action point on ETSI MCC |  | - |  | 12.6.0 |  | 12.7.0 |  | - |  |
| RP#70 |  | RP-151703 |  | 0641 |  | - |  | Update generic procedures 4.5A.19 and 4.5A.20 |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155083 |  |
| RP#70 |  | RP-151685 |  | 0642 |  | - |  | Editor notes correction in clause 5.2A.1A |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155111 |  |
| RP#70 |  | RP-151705 |  | 0643 |  | - |  | Editorial correction to Device to Device Proximity Services Test Environment |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155130 |  |
| RP#70 |  | RP-151710 |  | 0644 |  | - |  | Introduction of combinations of system information blocks for Dual Connectivity |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155234 |  |
| RP#70 |  | RP-151710 |  | 0651 |  | - |  | Introduction of generic test procedures for Dual Connectivity for signalling testing |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155419 |  |
| RP#70 |  | RP-151710 |  | 0652 |  | - |  | Introduction of default messages for Dual Connectivity |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155434 |  |
| RP#70 |  | RP-151685 |  | 0655 |  | - |  | Correction to procedure for UE triggered establishment of a default EPS bearer context associated with an additional PDN |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155490 |  |
| RP#70 |  | RP-151705 |  | 0659 |  | - |  | Addition of default Test UICC contents for DF_ProSe |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155647 |  |
| RP#70 |  | RP-151713 |  | 0662 |  | 1 |  | Removal of incorrect test frequencies for CA_12B |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155800 |  |
| RP#70 |  | RP-151700 |  | 0654 |  | 1 |  | Connections for 3DL CA RRM test cases |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155865 |  |
| RP#70 |  | RP-151700 |  | 0658 |  | 1 |  | 3DL carrier aggregation combination corrections to Table 6.2.3.2-2 “Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation (two bands)” |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155910 |  |
| RP#70 |  | RP-151705 |  | 0663 |  | 1 |  | Clarification of SIB18/19 Table |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155922 |  |
| RP#70 |  | RP-151699 |  | 0661 |  | 1 |  | Add generic procedures for IMS over WLAN |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155958 |  |
| RP#70 |  | RP-151701 |  | 0650 |  | 1 |  | Addition of new generic procedures to check UE does not offload |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155961 |  |
| RP#70 |  | RP-151701 |  | 0657 |  | 1 |  | Update of WLAN Offload procedures to and from EUTRA |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-155962 |  |
| RP#70 |  | RP-151705 |  | 0648 |  | 1 |  | ProSe: Connection diagram for basic Tx/Rx testing |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-156012 |  |
| RP#70 |  | RP-151712 |  | 0645 |  | 1 |  | Adding default message contents for NAICS test cases |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-156088 |  |
| RP#70 |  | RP-151685 |  | 0653 |  | 1 |  | Correction to UE test states for RF/RRM testing |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-156094 |  |
| RP#70 |  | RP-151705 |  | 0647 |  | 1 |  | Add generic procedures for ProSe UE registration |  | F |  | 12.7.0 |  | 12.8.0 |  | R5-156159 |  |
| RP#71 |  | RP-160107 |  | 0675 |  | - |  | Correction of generic setup procedures for Dual Connectivity protocol testing |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160406 |  |
| RP#71 |  | RP-160107 |  | 0676 |  | - |  | Introduction of generic RF procedures for Dual Connectivity |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160407 |  |
| RP#71 |  | RP-160111 |  | 0678 |  | - |  | CA_20A-67A: Add test frequencies to sub-clause 6.2.3.2 |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160413 |  |
| RP#71 |  | RP-160099 |  | 0679 |  | - |  | Addition of default power level for WLAN AP beacon RSSI |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160471 |  |
| RP#71 |  | RP-160110 |  | 0680 |  | - |  | Addition of reference message definitions for eIMTA |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160476 |  |
| RP#71 |  | RP-160103 |  | 0682 |  | - |  | Addition of ProSe Test Frequencies and update to Band and Bandwidth capabilities |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160503 |  |
| RP#71 |  | RP-160117 |  | 0683 |  | - |  | Addition of missing E-UTRA manual selection (E5) -end state |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160562 |  |
| RP#71 |  | RP-160105 |  | 0686 |  | - |  | Correction of PhysicalConfigDedicated-DEFAULT |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160646 |  |
| RP#71 |  | RP-160128 |  | 0687 |  | - |  | Correction of test frequencies for CA_3C and CA_7C |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160647 |  |
| RP#71 |  | RP-160103 |  | 0689 |  | - |  | Addition of default Test UICC contents for DF_ProSe |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160699 |  |
| RP#71 |  | RP-160117 |  | 0673 |  | 1 |  | UTRAN Registration procedure for LTE-IRAT test cases |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160730 |  |
| RP#71 |  | RP-160117 |  | 0684 |  | 1 |  | Correction to CDMA Frequency range for E25-CDMA BC1 band combination |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160731 |  |
| RP#71 |  | RP-160103 |  | 0670 |  | 1 |  | Update generic procedures 4.5A.22 |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160765 |  |
| RP#71 |  | RP-160103 |  | 0685 |  | 1 |  | Update for SIB18 and SIB19 |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160767 |  |
| RP#71 |  | RP-160117 |  | 0690 |  | - |  | Correction to UE registration procedure State 2 |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160784 |  |
| RP#71 |  | RP-160098 |  | 0691 |  | - |  | Add new antenna diagram for 3DL CA RSTD test cases |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160789 |  |
| RP#71 |  | RP-160111 |  | 0672 |  | 1 |  | Addition of Test frequency for CA_19A-28A and CA_1A-19A-28A |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160809 |  |
| RP#71 |  | RP-160111 |  | 0677 |  | 1 |  | CA_20A-67A: Add test frequencies to sub-clause 4.3.1 |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160810 |  |
| RP#71 |  | RP-160109 |  | 0664 |  | 1 |  | Adding new specific message content to release NAICS configuration |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160848 |  |
| RP#71 |  | RP-160112 |  | 0667 |  | 1 |  | Connection diagram for Receiver test with 4 Rx antenna ports |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160895 |  |
| RP#71 |  | RP-160107 |  | 0674 |  | 1 |  | Corrections to default RRC messages for Dual Connectivity |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160950 |  |
| RP#71 |  | RP-160117 |  | 0671 |  | 1 |  | Corrections to bit rates for video calls |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-160980 |  |
| RP#71 |  | RP-160128 |  | 0681 |  | 1 |  | Correction to extendedPHR-r10 for UL CA |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-161003 |  |
| RP#71 |  | RP-160106 |  | 0669 |  | 1 |  | Addition of the default messages in 36.508 for the small cell enhancements for physical layer test |  | F |  | 12.8.0 |  | 12.9.0 |  | R5-161057 |  |
| RP#72 |  | RP-160858 |  | 0717 |  | 1 |  | Defining SIB combination for TDD-FDD CA |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163012 |  |
| RP#72 |  | RP-160843 |  | 0708 |  | 2 |  | Addition of test frequencies for band 66 |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163193 |  |
| RP#72 |  | RP-160835 |  | 0734 |  | - |  | Addition of test frequencies for intra-band non-contiguous Low WGap test points |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162889 |  |
| RP#72 |  | RP-160837 |  | 0695 |  | 1 |  | Correction to generic RF procedures for Dual Connectivity |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162883 |  |
| RP#72 |  | RP-160833 |  | 0692 |  | - |  | Clarifications to the D2D ProSe test environment for signalling tests |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162028 |  |
| RP#72 |  | RP-160833 |  | 0698 |  | - |  | Update generic procedures 4.5A.22 |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162133 |  |
| RP#72 |  | RP-160833 |  | 0723 |  | - |  | Update of 36508 USIM for D2D |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162611 |  |
| RP#72 |  | RP-160833 |  | 0699 |  | 1 |  | Add TLS default messages |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162902 |  |
| RP#72 |  | RP-160833 |  | 0719 |  | 1 |  | Update of 36508 adding Default ProSe messages D2D |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162903 |  |
| RP#72 |  | RP-160833 |  | 0720 |  | 1 |  | Update of 36508 References for D2D |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162904 |  |
| RP#72 |  | RP-160833 |  | 0721 |  | 1 |  | Update of 36508 SIB19 D2D |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162906 |  |
| RP#72 |  | RP-160833 |  | 0722 |  | 1 |  | Update of 36508 SIB18 D2D |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162907 |  |
| RP#72 |  | RP-160837 |  | 0694 |  | - |  | Correction to generic default messages for Dual Connectivity |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162073 |  |
| RP#72 |  | RP-160837 |  | 0710 |  | - |  | Introduction of test frequencies for protocol testing of Dual Connectivity |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162394 |  |
| RP#72 |  | RP-160837 |  | 0704 |  | 1 |  | Correction to generic procedures for protocol testing of Dual Connectivity |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163025 |  |
| RP#72 |  | RP-160831 |  | 0700 |  | 1 |  | Add IKEv2 default messages |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163077 |  |
| RP#72 |  | RP-160839 |  | 0724 |  | 1 |  | Modification of reference message definitions for eIMTA |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163081 |  |
| RP#72 |  | RP-160846 |  | 0693 |  | - |  | Removing outdated Editor’s Notes |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162042 |  |
| RP#72 |  | RP-160858 |  | 0718 |  | - |  | New CA band combination CA_8A-40A – Updates of 6.2.3.2 Test Frequency |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162558 |  |
| RP#72 |  | RP-160858 |  | 0705 |  | 1 |  | Correction to Tracking area update request message (mobility from another RAT) |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162764 |  |
| RP#72 |  | RP-160846 |  | 0730 |  | 1 |  | Updates to bitrate values for the dedicated EPS video bearer |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162766 |  |
| RP#72 |  | RP-160858 |  | 0732 |  | - |  | Editorial correction to the test procedure of 6.4.3.10 |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-162767 |  |
| RP#72 |  | RP-160858 |  | 0709 |  | 2 |  | Updates to handle IMS registration in WLAN offloading procedures |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163053 |  |
| RP#72 |  | RP-160847 |  | 0729 |  | 1 |  | Introduction of Generic Test Procedure for optional UE initiated detaching |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163079 |  |
| RP#72 |  | RP-160858 |  | 0703 |  | 1 |  | [Common] Updates to WLAN offloading procedures to allow IMS PDN |  | F |  | 12.9.0 |  | 12.10.0 |  | R5-163080 |  |
| RP#72 |  | RP-160841 |  | 0716 |  | - |  | Add test frequencies for CA_42D in 36.508 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162541 |  |
| RP#72 |  | RP-160841 |  | 0711 |  | 1 |  | Addition of RF test frequencies for CA_5B, CA_5A-5A & CA_3C-5A to Section 4.3.1 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-163117 |  |
| RP#72 |  | RP-160863 |  | 0727 |  | 1 |  | Band 65 test frequencies for 36.508 clause 4.3 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162972 |  |
| RP#72 |  | RP-160841 |  | 0701 |  | - |  | Updates of 6.2.3.2 Test Frequency for CA_3A-41A for CA signalling test |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162170 |  |
| RP#72 |  | RP-160841 |  | 0702 |  | - |  | Updates of CA Channel Bandwidth combination for CA signalling test in sub-clause 4.3.1 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162171 |  |
| RP#72 |  | RP-160841 |  | 0712 |  | 1 |  | Addition of signalling test frequencies for CA_5B, CA_5A-5A & CA_3C-5A to Section 6.2.3.2 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162795 |  |
| RP#72 |  | RP-160841 |  | 0733 |  | - |  | Update of Rel-13 CA test frequency CA_19A-28A and CA_21A-42C |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-163052 |  |
| RP#72 |  | RP-160863 |  | 0728 |  | 1 |  | Band 65 signalling test frequencies for 36.508 clause 6.2.3 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162796 |  |
| RP#72 |  | RP-160861 |  | 0726 |  | 2 |  | Introduction of default messages for testing of CAT-M1 UE and UE in enhanced coverage |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-163156 |  |
| RP#72 |  | RP-160864 |  | 0714 |  | 1 |  | Addition of some SC-PTM related message definitions in TS36.508 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-163038 |  |
| RP#72 |  | RP-160862 |  | 0697 |  | 1 |  | Updates to Common parameter in EUTRA (SIB1) and NAS Default Messages and informational elements contents to include Extended DRX parameters |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-162797 |  |
| RP#72 |  | RP-160866 |  | 0713 |  | 1 |  | Addition of LAA related message definitions in TS36.508 |  | F |  | 12.10.0 |  | 13.0.0 |  | R5-163001 |  |
| RP#72 |  | - |  | - |  | - |  | restoring deleted Table 4.3.1.1.66A-3 of R5-163193 |  | F |  | 13.0.0 |  | 13.0.1 |  | - |  |
| RP#73 |  | RP-161427 |  | 0749 |  | - |  | Correction to DRB configuration |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165110 |  |
| RP#73 |  | RP-161407 |  | 0754 |  | - |  | Introduction of test frequencies for CA_8A-42A and editorial corrections to clause 6.2.3.2 |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165242 |  |
| RP#73 |  | RP-161427 |  | 0755 |  | - |  | Prevent unnecessary IMS signalling over GERAN for non-IMS test cases |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165266 |  |
| RP#73 |  | RP-161399 |  | 0756 |  | - |  | Update to generic test procedure 4.5A.22 |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165267 |  |
| RP#73 |  | RP-161407 |  | 0761 |  | - |  | TS36.508_Addition of signalling test frequencies for CA_8B and CA_8A-41C |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165448 |  |
| RP#73 |  | RP-161399 |  | 0762 |  | - |  | Update of 36508 adding Default ProSe messages D2D |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165457 |  |
| RP#73 |  | RP-161399 |  | 0763 |  | - |  | Update of 36508 SIB19 D2D |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165458 |  |
| RP#73 |  | RP-161399 |  | 0764 |  | - |  | Update of 36508 SIB18 D2D |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165459 |  |
| RP#73 |  | RP-161400 |  | 0765 |  | - |  | Correction to PhysicalConfigDedicated-DEFAULT IE for HalfDuplex Type B mode operation |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165478 |  |
| RP#73 |  | RP-161410 |  | 0771 |  | - |  | Addition of LAA CA band 46 combination test frequencies for signalling test cases |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165519 |  |
| RP#73 |  | RP-161410 |  | 0772 |  | - |  | Addition of LAA band 46 test frequencies |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165521 |  |
| RP#73 |  | RP-161407 |  | 0773 |  | - |  | Update of Rel-13 CA test frequency and CA Channel Bandwidth combination |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165609 |  |
| RP#73 |  | RP-161428 |  | 0758 |  | 1 |  | Update SystemInformationBlockType1 for band 65 and higher |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165892 |  |
| RP#73 |  | RP-161439 |  | 0759 |  | 1 |  | Introduction of test frequencies for MFBI enhancement |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165893 |  |
| RP#73 |  | RP-161437 |  | 0769 |  | 1 |  | Correction of test frequencies for CA intra band combinations CA_2C, CA_3C, CA_7C and CA_12B |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165894 |  |
| RP#73 |  | RP-161407 |  | 0746 |  | 1 |  | Updates of 6.2.3.2 Test Frequency for CA_2A-28A for CA signalling test |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165921 |  |
| RP#73 |  | RP-161407 |  | 0752 |  | 1 |  | New CA band combination CA_1A-40A and CA_3A-40A - Updates of 6.2.3.2 Test Frequency |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165922 |  |
| RP#73 |  | RP-161407 |  | 0783 |  | 1 |  | Updates of 6.2.3.2 Test Frequency for CA_4A-28A and CA_20A-31A for CA signalling test |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165923 |  |
| RP#73 |  | RP-161413 |  | 0747 |  | 1 |  | Correction of default messages for testing of CAT-M1 UE and UE in enhanced coverage |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165932 |  |
| RP#73 |  | RP-161399 |  | 0753 |  | 1 |  | Addition of default content for KEY_REQUEST, KEY_RESPONSE and MIKEY messages |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165960 |  |
| RP#73 |  | RP-161421 |  | 0777 |  | 1 |  | NB-IoT test frequencies definition |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165985 |  |
| RP#73 |  | RP-161421 |  | 0779 |  | 1 |  | Addition basic NB-IoT UE test states to TS 36.508 |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165986 |  |
| RP#73 |  | RP-161421 |  | 0780 |  | 1 |  | Introduction default NAS messages and Reference bearer configurations for NB-IoT |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165987 |  |
| RP#73 |  | RP-161421 |  | 0781 |  | 1 |  | Introduction default RRC messages for NB-IoT |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165988 |  |
| RP#73 |  | RP-161421 |  | 0784 |  | 1 |  | Definition of NB-IoT environmental conditions and NB-IoT common test requirements of test equipment |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165989 |  |
| RP#73 |  | RP-161421 |  | 0785 |  | 1 |  | Definition of NB-IoT physical layer parameters |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165990 |  |
| RP#73 |  | RP-161421 |  | 0786 |  | 1 |  | Definition of NB-IoT signal levels and standard test signals |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165991 |  |
| RP#73 |  | RP-161421 |  | 0787 |  | 1 |  | Definition of NB-IoT radio conditions and physical channel allocation |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-165992 |  |
| RP#73 |  | RP-161437 |  | 0796 |  | - |  | 36.508: Addition of test frequencies for band CA_7B |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166015 |  |
| RP#73 |  | RP-161402 |  | 0775 |  | 1 |  | Addition of message exceptions for DL 256QAM |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166021 |  |
| RP#73 |  | RP-161407 |  | 0760 |  | 1 |  | TS36.508_Additional new Test Frequency for CA_8B |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166026 |  |
| RP#73 |  | RP-161409 |  | 0795 |  | 1 |  | Addition of connection diagram for 4Rx test cases |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166095 |  |
| RP#73 |  | RP-161421 |  | 0782 |  | 1 |  | Introduction Test environment for RF test for NB-IoT |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166124 |  |
| RP#73 |  | RP-161419 |  | 0770 |  | 1 |  | Introduction of Band 45 into 36.508 |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166128 |  |
| RP#73 |  | RP-161411 |  | 0766 |  | 1 |  | Correction to test frequencies for Band 66 |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166131 |  |
| RP#73 |  | RP-161411 |  | 0776 |  | 1 |  | Correction to Band 66 test frequencies |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166132 |  |
| RP#73 |  | RP-161399 |  | 0767 |  | 1 |  | Addition of connection diagrams for ProSe |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166140 |  |
| RP#73 |  | RP-161437 |  | 0791 |  | 1 |  | 36.508 UL CA correction for RF test cases |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166142 |  |
| RP#73 |  | RP-161423 |  | 0789 |  | 2 |  | Introduction of Indoor Positioning Enhancements (MBS) |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166184 |  |
| RP#73 |  | RP-161397 |  | 0778 |  | 1 |  | Correction to Default IKEv2 message definitions. |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166260 |  |
| RP#73 |  | RP-161413 |  | 0748 |  | 1 |  | Introduction of test environment for testing of CAT-M1 UE and UE in enhanced coverage |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166261 |  |
| RP#73 |  | RP-161421 |  | 0790 |  | 1 |  | Addition of definition of simulated network cells for NB-IOT |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166285 |  |
| RP#73 |  | RP-161421 |  | 0797 |  | 1 |  | Add default NAS messages for NB-IoT and Rel-13 updates |  | F |  | 13.0.1 |  | 13.1.0 |  | R5-166331 |  |
| RP#74 |  | RP-162076 |  | 0798 |  | - |  | Update of default message contents for ProSe Direct Communication |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168072 |  |
| RP#74 |  | RP-162075 |  | 0801 |  | - |  | Update IKEv2 default messages |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168094 |  |
| RP#74 |  | RP-162085 |  | 0802 |  | - |  | Addition of connection diagrams for new SC demod and CSI Test Cases for 4Rx |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168120 |  |
| RP#74 |  | RP-162083 |  | 0805 |  | - |  | Editorial correction to aggregated number of resource blocks for CA Inter-band combination CA_8A-42A |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168319 |  |
| RP#74 |  | RP-162083 |  | 0811 |  | - |  | Addition of new test frequencies for intra-band non-contiguous CA |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168367 |  |
| RP#74 |  | RP-162097 |  | 0812 |  | - |  | Common test frequencies for Band 70 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168398 |  |
| RP#74 |  | RP-162104 |  | 0815 |  | - |  | Update SCellToAddMod for band 65 and higher |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168447 |  |
| RP#74 |  | RP-162104 |  | 0816 |  | - |  | Update MobilityControlInfo for band 65 and higher |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168454 |  |
| RP#74 |  | RP-162091 |  | 0819 |  | - |  | Addition of Procedure for IP address allocation in the CP CIoT |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168472 |  |
| RP#74 |  | RP-162079 |  | 0820 |  | - |  | Correction of ReportConfigEUTRA-C2 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168567 |  |
| RP#74 |  | RP-162096 |  | 0824 |  | - |  | CA_20A-28A: Add test frequencies to sub-clause 6.2.3.2 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168618 |  |
| RP#74 |  | RP-162103 |  | 0834 |  | - |  | Correction to EUTRA band 2 and band 25 <> CDMA BC 1 for LTE<>HRPD/1xRTT |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168775 |  |
| RP#74 |  | RP-162113 |  | 0835 |  | - |  | Addition of test frequencies for CA_7C |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168778 |  |
| RP#74 |  | RP-162091 |  | 0837 |  | - |  | Addition NB-IoT test environment for RRM test |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168833 |  |
| RP#74 |  | RP-162097 |  | 0840 |  | - |  | Signalling Test environment for CA_70C |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168843 |  |
| RP#74 |  | RP-162087 |  | 0845 |  | - |  | Add generic procedure for control plane CIoT connection request |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168885 |  |
| RP#74 |  | RP-162087 |  | 0846 |  | - |  | Add generic procedure for user plane CIoT connection request |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168886 |  |
| RP#74 |  | RP-162080 |  | 0847 |  | - |  | Correction to Table 6.2.3.2-2 for DC_7A-20A |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168890 |  |
| RP#74 |  | RP-162080 |  | 0848 |  | - |  | Editorial correction to clause 5.2A.2A.1 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168891 |  |
| RP#74 |  | RP-162083 |  | 0851 |  | - |  | Add test frequencies to sub-clause 6.2.3.2 for CA_7A-22A, CA_7B and CA_20A-40A |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-168922 |  |
| RP#74 |  | RP-162091 |  | 0806 |  | 1 |  | Addition of various CIoT generic procedures for PDN establishment-release |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169009 |  |
| RP#74 |  | RP-162087 |  | 0844 |  | 1 |  | Update UE test states |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169032 |  |
| RP#74 |  | RP-162104 |  | 0814 |  | 1 |  | Update SystemInformationBlockType5 for band 65 and higher |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169034 |  |
| RP#74 |  | RP-162103 |  | 0843 |  | 1 |  | EPS bearer mapping updates |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169035 |  |
| RP#74 |  | RP-162097 |  | 0813 |  | 1 |  | Signalling Test environment for Band 70 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169047 |  |
| RP#74 |  | RP-162075 |  | 0800 |  | 1 |  | Update generic procedure 4.5A.23 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169074 |  |
| RP#74 |  | RP-162087 |  | 0825 |  | 1 |  | Correction of default DCI formats and messages for testing of CAT-M1 UE and UE in enhanced coverage |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169087 |  |
| RP#74 |  | RP-162091 |  | 0838 |  | 1 |  | Addition default values in NB-IoT RRC messages and IEs |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169088 |  |
| RP#74 |  | RP-162104 |  | 0817 |  | 1 |  | Update measurement information for band 65 and higher |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169116 |  |
| RP#74 |  | RP-162083 |  | 0832 |  | 1 |  | Introduction of test frequencies for CA_5A-29A |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169125 |  |
| RP#74 |  | RP-162091 |  | 0799 |  | 1 |  | Update to default NAS messages for NB-IoT |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169126 |  |
| RP#74 |  | RP-162091 |  | 0807 |  | 1 |  | Update of NB-IoT Generic procedures |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169127 |  |
| RP#74 |  | RP-162091 |  | 0818 |  | 1 |  | TAU Procedure for NB-IoT UE’s |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169128 |  |
| RP#74 |  | RP-162091 |  | 0833 |  | 1 |  | Addition of Default configurations for NB-IoT NAS and Idle Mode test cases |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169129 |  |
| RP#74 |  | RP-162091 |  | 0808 |  | 1 |  | Correction to NB-IOT System configuration IE |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169144 |  |
| RP#74 |  | RP-162103 |  | 0827 |  | 1 |  | Correction to Procedure for IMS signalling |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169150 |  |
| RP#74 |  | RP-162091 |  | 0852 |  | 1 |  | Default NB-IoT uplink configuration |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169164 |  |
| RP#74 |  | RP-162091 |  | 0841 |  | 1 |  | Update of common test environment sections and Annex for NB-IoT UEs |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169169 |  |
| RP#74 |  | RP-162113 |  | 0830 |  | 1 |  | Correction to test frequencies for CA_66C |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169500 |  |
| RP#74 |  | RP-162115 |  | 0831 |  | 1 |  | Correction to default messages for TDD FDD CA |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169501 |  |
| RP#74 |  | RP-162113 |  | 0810 |  | 1 |  | New connection diagram for TC7.9A |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169503 |  |
| RP#74 |  | RP-162113 |  | 0829 |  | 1 |  | Correction to test frequencies for CA_12B |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169504 |  |
| RP#74 |  | RP-162101 |  | 0836 |  | 1 |  | Correction of lower test frequencies for Band 26 |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169505 |  |
| RP#74 |  | RP-162085 |  | 0809 |  | 1 |  | New connection diagrams for 4Rx Receiver TCs |  | F |  | 13.1.0 |  | 13.2.0 |  | R5-169618 |  |
| RP#74 |  | RP-162085 |  | 0826 |  | 1 |  | Addition of 4-Rx connection diagrams |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169619 |  |
| RP#74 |  | RP-162098 |  | 0821 |  | 1 |  | Addition of V2X Sidelink Communication Test Environment |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169647 |  |
| RP#74 |  | RP-162097 |  | 0839 |  | 1 |  | Common test frequencies for CA_70C |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169649 |  |
| RP#74 |  | RP-162114 |  | 0822 |  | 1 |  | Addition of UECapabilityEnquiry message for RF tests |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169661 |  |
| RP#74 |  | RP-162085 |  | 0803 |  | 1 |  | Addition of connection diagram for new CA SDR Test Case for 4Rx |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169673 |  |
| RP#74 |  | RP-162085 |  | 0804 |  | 1 |  | Addition of connection diagrams for 2Rx Test Cases with 4Rx capable UEs |  | F |  | 13.2.0 |  | 14.0.0 |  | R5-169674 |  |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | RAN#75 | RP-170082 | 0856 | - | F | Update to default NAS messages for NB-IoT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0857 | - | F | To add Figure A.64a for 4DL CA connection with static PCC propagation channel, SCCs multipath fading propagation channels and additional intra-frequency cell on one SCC. | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0858 | - | F | Update generic procedure control plane CIoT connection request | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0859 | - | F | Corrections for TA and ul-carrierFreq-r13 in SIB2 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0862 | - | F | Add generic procedure to check RRC Connected state for NB-IoT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170097 | 0872 | - | F | Update SystemInformationBlockType5 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170070 | 0875 | - | F | NAICS: Addtion of new conncetion diagram | 14.1.0 |
| 2017-03 | RAN#75 | RP-170107 | 0876 | - | F | New generic procedure for IMS emergency call in EPC / WLAN | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0879 | - | F | NPDCCH config dedicated update for RF test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170106 | 0880 | - | F | Addition of test frequencies for 10+15MHz and 15+10MHz in CA_41C | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0882 | - | F | Correction to default values of NB-IoT RRC UECapabilityInformation-NB message | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0883 | - | F | Update of Rel-13 LAA related IE content in MeasObjectEURA | 14.1.0 |
| 2017-03 | RAN#75 | RP-170091 | 0884 | - | F | Update of V2X Sidelink Communication Test Environment | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0886 | - | F | Update of various CIoT generic procedures | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0888 | - | F | Correction to test frequencies for CA_5A-5A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0889 | - | F | NPRACH-ConfigSIB-NB-DEFAULT updated | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0895 | - | F | Correction to NB-IoT Physical layer parameters for DCI | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0896 | - | F | Correction to Generic Test Procedure for network initiated release of additional PDN connectivity | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0897 | - | F | Correction to generic procedure for preventing IMS signalling over GERAN | 14.1.0 |
| 2017-03 | RAN#75 | RP-170108 | 0900 | - | F | Add missing condition description in Table 4.6.3-9A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0902 | 1 | F | Update MobilityControlInfo | 14.1.0 |
| 2017-03 | RAN#75 | RP-170097 | 0903 | 1 | F | Update MeasConfig-DEFAULT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0864 | 1 | F | Introduction of test frequencies for CA_3A-11A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0865 | 1 | F | Introduction of test frequencies for CA_8A-28A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0866 | 1 | F | Introduction of test frequencies for CA_11A-28A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0893 | 1 | F | CA_29A-66A, CA_29A-66A-66A, CA_29A-66C, CA_46A-66A test frequencies for signalling test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170108 | 0869 | 1 | F | Introduction of new combination of SIBs for OTDOA 3DL CA test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0854 | 1 | F | [Common] Correction to default parameters for Ncells and SIB-NB | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0905 | - | F | Correction to eMTC generic procedures | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0885 | 1 | F | Correction of default messages for testing of CAT-M1 UE and UE in enhanced coverage | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0899 | 1 | F | TAU Procedure for NB-IoT UEs performing Suspend-Resume | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0898 | 1 | F | Correction to test frequencies for Band 46 and Test frequencies for CA signalling test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0874 | 1 | F | Updates to Band 66 test frequencies | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0894 | 1 | F | Update of Rel-13 LAA related IE content in the test environment for RF and RRM test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0873 | 1 | F | LAA: Test frequencies for Band 46 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0863 | 1 | F | Correction of PRACH configuration for RF tests | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0887 | 1 | F | Addition of Low Wgap frequencies for CA_4A-4A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170093 | 0853 | 1 | F | Update TS 36.508 with Addition of LTE Band 48 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0871 | 1 | F | MBSFN information in SIB2 should not be mandatory for eMTC RRM test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0877 | 2 | F | Correction to NB-IoT test environment for SIG | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0904 | 1 | F | Updates to NB-IoT generic procedures to add User-Plane mode support | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0878 | 3 | F | Correction to default message contents for eMTC testing | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0856 | - | F | Update to default NAS messages for NB-IoT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0857 | - | F | To add Figure A.64a for 4DL CA connection with static PCC propagation channel, SCCs multipath fading propagation channels and additional intra-frequency cell on one SCC. | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0858 | - | F | Update generic procedure control plane CIoT connection request | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0859 | - | F | Corrections for TA and ul-carrierFreq-r13 in SIB2 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0862 | - | F | Add generic procedure to check RRC Connected state for NB-IoT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0880 | - | F | Update SystemInformationBlockType5 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170070 | 0875 | - | F | NAICS: Addtion of new conncetion diagram | 14.1.0 |
| 2017-03 | RAN#75 | RP-170107 | 0876 | - | F | New generic procedure for IMS emergency call in EPC / WLAN | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0879 | - | F | NPDCCH config dedicated update for RF test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170106 | 0880 | - | F | Addition of test frequencies for 10+15MHz and 15+10MHz in CA_41C | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0882 | - | F | Correction to default values of NB-IoT RRC UECapabilityInformation-NB message | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0883 | - | F | Cor Update of Rel-13 LAA related IE content in MeasObjectEURA | 14.1.0 |
| 2017-03 | RAN#75 | RP-170091 | 0884 | - | F | Update of V2X Sidelink Communication Test Environment | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0886 | - | F | Update of various CIoT generic procedures | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0888 | - | F | Correction to test frequencies for CA_5A-5A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0889 | - | F | NPRACH-ConfigSIB-NB-DEFAULT updated | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0895 | - | F | Correction to NB-IoT Physical layer parameters for DCI Renumbering of clauses 8.1.3.6.1.5 (and associated tables) not implemented: clause renumbering is strongly deprecated; instead, empty padding clauses 8.1.3.6.1.3 & 8.1.3.6.1.4 added. | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0896 | - | F | Correction to Generic Test Procedure for network initiated release of additional PDN connectivity | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0897 | - | F | Correction to generic procedure for preventing IMS signalling over GERAN | 14.1.0 |
| 2017-03 | RAN#75 | RP-170108 | 0900 | - | F | Add missing condition description in Table 4.6.3-9A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170095 | 0902 | - | F | Update MobilityControlInfo | 14.1.0 |
| 2017-03 | RAN#75 | RP-170097 | 0903 | - | F | Update MeasConfig-DEFAULT | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0864 | - | F | Introduction of test frequencies for CA_3A-11A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0865 | - | F | Introduction of test frequencies for CA_8A-28A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0866 | - | F | Introduction of test frequencies for CA_11A-28A to section 6 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0893 | - | F | CA_29A-66A, CA_29A-66A-66A, CA_29A-66C, CA_46A-66A test frequencies for signalling test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170108 | 0869 | 1 | F | Introduction of new combination of SIBs for OTDOA 3DL CA test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170108 | 0854 | 1 | F | [Common] Correction to default parameters for Ncells and SIB-NB | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0905 | 1 | F | Correction to eMTC generic procedures | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0885 | 1 | F | Correction of default messages for testing of CAT-M1 UE and UE in enhanced coverage | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0899 | 1 | F | TAU Procedure for NB-IoT UEs performing Suspend-Resume | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0898 | 1 | F | Correction to test frequencies for Band 46 and Test frequencies for CA signalling test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170089 | 0874 | 1 | F | Updates to Band 66 test frequencies | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0894 | 1 | F | Update of Rel-13 LAA related IE content in the test environment for RF and RRM test | 14.1.0 |
| 2017-03 | RAN#75 | RP-170076 | 0873 | 1 | F | LAA: Test frequencies for Band 46 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0863 | 1 | F | Correction of PRACH configuration for RF tests | 14.1.0 |
| 2017-03 | RAN#75 | RP-170072 | 0887 | - | F | Addition of Low Wgap frequencies for CA_4A-4A | 14.1.0 |
| 2017-03 | RAN#75 | RP-170093 | 0853 | 1 | F | Update TS 36.508 with Addition of LTE Band 48 | 14.1.0 |
| 2017-03 | RAN#75 | RP-170093 | 0871 | 1 | F | MBSFN information in SIB2 should not be mandatory for eMTC RRM test cases | 14.1.0 |
| 2017-03 | RAN#75 | RP-170093 | 0872 | 2 | F | Correction to NB-IoT test environment for SIG | 14.1.0 |
| 2017-03 | RAN#75 | RP-170077 | 0878 | 3 | F | Correction to default message contents for eMTC testing | 14.1.0 |
| 2017-03 | RAN#75 | RP-170082 | 0904 | 1 | F | Updates to NB-IoT generic procedures to add User-Plane mode support | 14.1.0 |
| 2017-06 | RAN#76 | R5-172049 | 0908 |  | F | Updates to D2D ProSe default message contents | 14.2.0 |
| 2017-06 | RAN#76 | R5-172061 | 0910 |  | F | Editorial update to references to PhysicalConfigDedicated-DEFAULT for SIG | 14.2.0 |
| 2017-06 | RAN#76 | R5-172115 | 0915 |  | F | NB-IoT test frequencies for Band 70 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172227 | 0925 |  | F | CA_46C-66A test frequencies for signalling test and correction to CA_46A-66A | 14.2.0 |
| 2017-06 | RAN#76 | R5-172324 | 0933 |  | F | MobilityControInfo-HO correction for bands > 64 (Editorial) | 14.2.0 |
| 2017-06 | RAN#76 | R5-172427 | 0938 |  | F | Update to e-MTC SIB-2 Signalling | 14.2.0 |
| 2017-06 | RAN#76 | R5-172507 | 0944 |  | F | Correction to PhysicalConfigdedicated Parameters for LAA | 14.2.0 |
| 2017-06 | RAN#76 | R5-172561 | 0947 |  | F | Update of test frequencies for 2DL/1UL of CA_3A-3A and CA_7A-7A due to new REFSENS test points addition. | 14.2.0 |
| 2017-06 | RAN#76 | R5-172567 | 0948 |  | F | Addition of SRS switching common test environment | 14.2.0 |
| 2017-06 | RAN#76 | R5-172582 | 0950 |  | F | Addition of test frequencies with Low Wgap for CA_66A-66A | 14.2.0 |
| 2017-06 | RAN#76 | R5-172636 | 0952 |  | F | Correction to scheduling request parameters for CatM1 using HD-FDD | 14.2.0 |
| 2017-06 | RAN#76 | R5-172672 | 0953 |  | F | Introduction of test frequencies for Band 48 for protocol testing | 14.2.0 |
| 2017-06 | RAN#76 | R5-172756 | 0955 |  | F | Addition of Rel-14 NB-IoT operating bands | 14.2.0 |
| 2017-06 | RAN#76 | R5-172776 | 0957 |  | F | NB-IoT test frequencies update for inband operation mode | 14.2.0 |
| 2017-06 | RAN#76 | R5-172786 | 0959 |  | F | Editorial correction to Clause and Table numbers in generic procedure 4.5A.3B | 14.2.0 |
| 2017-06 | RAN#76 | R5-172850 | 0906 | 1 | F | Update SRB reconfiguration for NB-IoT | 14.2.0 |
| 2017-06 | RAN#76 | R5-172851 | 0909 | 1 | F | NB-IoT: Add new generic procedure for generic RB establishment for UP | 14.2.0 |
| 2017-06 | RAN#76 | R5-172853 | 0963 |  | F | Correction to EPS network feature support for ATTACH ACCEPT and TRACKING AREA UPDATE ACCEPT | 14.2.0 |
| 2017-06 | RAN#76 | R5-172875 | 0911 | 1 | F | Update SystemInformationBlockType2 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172876 | 0916 | 1 | F | Update generic procedure control plane CIoT connection request | 14.2.0 |
| 2017-06 | RAN#76 | R5-172877 | 0917 | 1 | F | Add generic procedure control plane CIoT with test mode | 14.2.0 |
| 2017-06 | RAN#76 | R5-172878 | 0964 |  | F | Update SystemInformationBlockType3-NB | 14.2.0 |
| 2017-06 | RAN#76 | R5-172879 | 0919 | 1 | F | Update generic procedure user plane CIoT connection request | 14.2.0 |
| 2017-06 | RAN#76 | R5-172880 | 0922 | 1 | F | Add messages for resuming an RRC connection | 14.2.0 |
| 2017-06 | RAN#76 | R5-172895 | 0926 | 1 | F | Addition of Band 66 Frequencies | 14.2.0 |
| 2017-06 | RAN#76 | R5-172896 | 0927 | 1 | F | Update System Information Block type 19 for Band > 64 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172941 | 0934 | 1 | F | Addition of CA_2A-66A, CA_5A-66A and CA_13A-66A test frequencies for signalling test | 14.2.0 |
| 2017-06 | RAN#76 | R5-172944 | 0940 | 1 | F | Correction of test frequency selection of band 20 in Table 6.2.3.2-2 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172946 | 0965 |  | F | Introduction of test frequencies for 2UL/2DL of DC_41A-42A to section 6 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172961 | 0961 | 1 | F | Update for Antenna configuration for 4 Rx UE antenna | 14.2.0 |
| 2017-06 | RAN#76 | R5-172962 | 0966 |  | F | Correction to RACH-ConfigCommon-DEFAULT | 14.2.0 |
| 2017-06 | RAN#76 | R5-172981 | 0949 | 1 | F | Update of V2X related message contents in 36.508 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172999 | 0913 | 1 | F | Update RRCConnectionSetupComplete | 14.2.0 |
| 2017-06 | RAN#76 | R5-173000 | 0912 | 1 | F | Update ATTACH ACCEPT | 14.2.0 |
| 2017-06 | RAN#76 | R5-173001 | 0918 | 1 | F | Update RRCConnectionRelease | 14.2.0 |
| 2017-06 | RAN#76 | R5-173002 | 0920 | 1 | F | Add generic procedure user plane CIoT with test mode | 14.2.0 |
| 2017-06 | RAN#76 | R5-173003 | 0921 | 1 | F | Update UE test state | 14.2.0 |
| 2017-06 | RAN#76 | R5-173006 | 0960 | 1 | F | Correction to DRB logical channel configurations | 14.2.0 |
| 2017-06 | RAN#76 | R5-173028 | 0943 | 1 | F | LAA band 46 clarification on DL only operation | 14.2.0 |
| 2017-06 | RAN#76 | R5-173035 | 0929 | 1 | F | Correction to SIB21 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173227 | 0942 | 1 | F | Addition of Figure Connection for more than 3DL CA with static propagation channels and additional intra-frequency cell(s) on each SCC | 14.2.0 |
| 2017-06 | RAN#76 | R5-173315 | 0932 | 1 | F | RadioResourceConfigCommon-DEFAULT correction for eMTC (Editorial) | 14.2.0 |
| 2017-06 | RAN#76 | R5-173352 | 0931 | 1 | F | Introduction of the generic procedures for V2X Communication | 14.2.0 |
| 2017-06 | RAN#76 | R5-173353 | 0956 | 1 | F | Addition of reference test frequencies for V2X 47B | 14.2.0 |
| 2017-06 | RAN#76 | R5-173406 | 0923 | 1 | F | Update to NB-IOT RRM cell configuration | 14.2.0 |
| 2017-06 | RAN#76 | R5-173408 | 0936 | 1 | F | Separation of Narrowband Index for SI message and Data (PDSCH/MPDCCH/PUSCH) for eMTC test cases | 14.2.0 |
| 2017-06 | RAN#76 | R5-173416 | 0924 | 1 | F | New connection diagrams for RX tests with 2Rx and 4Rx and clarification for TX tests | 14.2.0 |
| 2017-09 | RAN#77 | R5-173636 | 0974 | - | F | Remove editor’s note for ATTACH ACCEPT | 14.3.0 |
| 2017-09 | RAN#77 | R5-173669 | 0976 | - | F | Update to SI2Quater for LTE Bands>64 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173670 | 0977 | - | F | Updates to UTRAN MEASUREMENT CONTROL & REPORT for LTE Bands>64 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173686 | 0978 | - | F | Clarify the use of system information in generic procedures | 14.3.0 |
| 2017-09 | RAN#77 | R5-173765 | 0983 | - | F | Add new Ncells for RRM Test Cases | 14.3.0 |
| 2017-09 | RAN#77 | R5-173783 | 0984 | - | F | Correction to 4Rx connection diagram for CSI 9.3.3 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173794 | 0985 | - | F | Correction to default messages for Cat M1 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173805 | 0986 | - | F | Correction to System Information Block Type 19 for Band > 64 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173839 | 0987 | - | F | Remove editor’s note for RRC Connection Release | 14.3.0 |
| 2017-09 | RAN#77 | R5-173842 | 0988 | - | F | Add generic procedure to check that UE is camped on a new E-UTRAN cell / UP CIoT | 14.3.0 |
| 2017-09 | RAN#77 | R5-173859 | 0991 | - | F | Addition of Test Frequencies for NB-IoT FDD MFBI (180 kHz) | 14.3.0 |
| 2017-09 | RAN#77 | R5-173914 | 0996 | - | F | Introduction of default message configurations for eHST | 14.3.0 |
| 2017-09 | RAN#77 | R5-173927 | 0999 | - | F | Editorial: Update of NB-IOT RadioResourceConfigCommonSIB-NB message in 36.508 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173973 | 1004 | - | F | Introduction of test frequencies for CA configuration CA_2A-7A | 14.3.0 |
| 2017-09 | RAN#77 | R5-174071 | 1008 | - | F | Addition of Physical layer parameters for DCI format 0C | 14.3.0 |
| 2017-09 | RAN#77 | R5-174151 | 1010 | - | F | Addition of 1.4 and 3 MHz test frequencies for Band 65 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174173 | 1014 | - | F | Introduction of test frequencies for CA_3A-32A to section 6.2.3.2 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174266 | 1016 | - | F | Addition of FDD reference test frequencies for band 69 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174291 | 1017 | - | F | Update to UECapabilityInformation-NB message | 14.3.0 |
| 2017-09 | RAN#77 | R5-174299 | 1018 | - | F | Update to V2X message contents | 14.3.0 |
| 2017-09 | RAN#77 | R5-174403 | 1027 | - | F | Correction ro generic procedures for Rel-12 DC | 14.3.0 |
| 2017-09 | RAN#77 | R5-174456 | 1019 | 1 | F | Correction to Generic Test Procedure for NB-IoT Control Plane CIoT | 14.3.0 |
| 2017-09 | RAN#77 | R5-174457 | 1028 | 1 | F | Addition of test loop mode I | 14.3.0 |
| 2017-09 | RAN#77 | R5-174498 | 0969 | 1 | F | New generic test procedure for XCAP establishment in EPC / WLAN | 14.3.0 |
| 2017-09 | RAN#77 | R5-174499 | 0973 | 1 | F | WLAN power level | 14.3.0 |
| 2017-09 | RAN#77 | R5-174500 | 0975 | 1 | F | Corrections to UTRAN SIB19 for LTE Bands>64 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174501 | 1024 | 1 | F | Update REJECT message security header | 14.3.0 |
| 2017-09 | RAN#77 | R5-174502 | 0980 | 1 | F | Update of Default ProSe messages 36508 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174565 | 1011 | 1 | F | CA_29A-70A, CA_46A-66C, CA_46A-70A signalling test frequencies | 14.3.0 |
| 2017-09 | RAN#77 | R5-174580 | 1006 | 2 | F | Correction to RadioResourceConfigCommonSIB-DEFAULT IE | 14.3.0 |
| 2017-09 | RAN#77 | R5-174593 | 0979 | 1 | F | New default ProSe messages Rel-13 36508 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174594 | 0994 | 1 | F | 36.508 Section 6.6.1 - Update for QCI 69 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174595 | 0995 | 1 | F | 36.508 Section 6.6.2 - Update for QCI 65 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174640 | 0990 | 1 | F | Update UP CIoT for ATTACH ACCEPT | 14.3.0 |
| 2017-09 | RAN#77 | R5-174649 | 0968 | 1 | F | Update IMS speech procedures for UE category M1 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174660 | 0971 | 2 | F | Addition of the SIB1 eCall over IMS Release 14 related IEs and condition | 14.3.0 |
| 2017-09 | RAN#77 | R5-174666 | 0997 | 1 | F | Addition of PUCCH SCell common test environment | 14.3.0 |
| 2017-09 | RAN#77 | R5-174667 | 1009 | 1 | F | Update of CIoT conditions in NAS messages | 14.3.0 |
| 2017-09 | RAN#77 | R5-174681 | 0972 | 1 | F | Addition of Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service | 14.3.0 |
| 2017-09 | RAN#77 | R5-175016 | 1003 | 1 | F | Updates to test frequencies for CA combinations | 14.3.0 |
| 2017-09 | RAN#77 | R5-175050 | 1007 | 1 | F | Sending of MaxLayersMimo-r10 IE for 4Rx TCs with more than 2 Layers | 14.3.0 |
| 2017-09 | RAN#77 | R5-175051 | 1023 | 1 | F | Addition of connection diagrams for 4Rx test cases | 14.3.0 |
| 2017-09 | RAN#77 | R5-175074 | 1005 | 1 | F | New connection diagram for SDR tests with 2Rx and 4Rx | 14.3.0 |
| 2017-09 | RAN#77 | R5-175083 | 1000 | 1 | F | Update of V2V related connection diagram | 14.3.0 |
| 2017-09 | RAN#77 | R5-175099 | 0970 | 1 | F | Addition of SL-V2X-Preconfiguration for V2X MCC case in 4.6.3 and 6.8.2.1 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175100 | 1002 | 1 | F | Update of V2X related message contents in 36.508 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175101 | 1020 | 1 | F | Addition of connection diagram for V2X Communication / Intra-band contiguous MCC operation | 14.3.0 |
| 2017-09 | RAN#77 | R5-175110 | 1012 | 1 | F | Correction of NB-IOT MasterInformationBlock | 14.3.0 |
| 2017-09 | RAN#77 | R5-175112 | 0967 | 2 | F | Correct srs-Bandwidth comment for Frequency Hopping tests (Editorial Change) | 14.3.0 |
| 2017-09 | RAN#77 | R5-175124 | 1025 | 1 | F | Addition of rel-14 NB-IoT test frequencies | 14.3.0 |
| 2017-09 | RAN#77 | R5-175159 | 1013 | 1 | F | Correction to PRACH-Config-v1310-DEFAULT for eMTC TC | 14.3.0 |
| 2017-09 | RAN#77 | R5-175218 | 1001 | 2 | F | Addition of new generic test procedures State 5-V2X and State 5A-V2X | 14.3.0 |
| 2017-12 | RAN#78 | R5-176083 | 1033 | - | F | Addition of MFBI Test Frequencies for operation Bands 4 and 10 overlapping with Band 66 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176104 | 1034 | - | F | New Connection diagram for RRM eMTC MISO tests | 14.4.0 |
| 2017-12 | RAN#78 | R5-176106 | 1036 | - | F | Correction of SIB2-NB (complete implementation of CR0859) | 14.4.0 |
| 2017-12 | RAN#78 | R5-176132 | 1037 | - | F | Correction to eCalloverIMS condition in SysinfoType1 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176136 | 1038 | - | F | Correction to support of User Plane CIoT Optimisation and S1-U Data Transfer | 14.4.0 |
| 2017-12 | RAN#78 | R5-176137 | 1039 | - | F | Corrections to NB-IoT generic test procedure to establish RB(s) in UP | 14.4.0 |
| 2017-12 | RAN#78 | R5-176145 | 1040 | - | F | Correction to RRC Connection Release message | 14.4.0 |
| 2017-12 | RAN#78 | R5-176146 | 1041 | - | F | Update to Tracking Area Update procedure for UP CIoT | 14.4.0 |
| 2017-12 | RAN#78 | R5-176421 | 1058 | - | F | Addition of test frequencies for CA signalling test | 14.4.0 |
| 2017-12 | RAN#78 | R5-176450 | 1060 | - | F | Update of PUCCH SCell common test environment | 14.4.0 |
| 2017-12 | RAN#78 | R5-176487 | 1064 | - | F | Correction to test frequencies for CA_41D-41A | 14.4.0 |
| 2017-12 | RAN#78 | R5-176491 | 1065 | - | F | Correction to NB-IoT Test frequencies for in-band mode | 14.4.0 |
| 2017-12 | RAN#78 | R5-176575 | 1071 | - | F | Update to SECURITY REJECT message security header | 14.4.0 |
| 2017-12 | RAN#78 | R5-176607 | 1074 | - | F | Editorial correction to the FDD reference test frequencies for operating band 30 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176680 | 1076 | - | F | Correction to startSymbolBR-r13 IE in SystemInformationBlockType1-BR-r13 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176683 | 1078 | - | F | Correction to PRACH-ConfigSIB-v1310-DEFAULT IE | 14.4.0 |
| 2017-12 | RAN#78 | R5-176703 | 1079 | - | F | New Connection diagram for RRM 4Rx | 14.4.0 |
| 2017-12 | RAN#78 | R5-176758 | 1080 | - | F | Allow retransmissions in IMS registration procedure over UTRA | 14.4.0 |
| 2017-12 | RAN#78 | R5-176802 | 1085 | - | F | Correction of Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service | 14.4.0 |
| 2017-12 | RAN#78 | R5-176872 | 1044 | 1 | F | Updates to ATTACH REQUEST and TRACKING AREA REQUEST for CIoT Optimisation | 14.4.0 |
| 2017-12 | RAN#78 | R5-176887 | 1042 | 1 | F | Corrections to IMS over WLAN procedures and default messages | 14.4.0 |
| 2017-12 | RAN#78 | R5-176888 | 1061 | 1 | F | Addition of 4 antenna ports default message contents | 14.4.0 |
| 2017-12 | RAN#78 | R5-176892 | 1083 | 1 | F | Addition of default IE contents for MeasGapConfig-CE | 14.4.0 |
| 2017-12 | RAN#78 | R5-176950 | 1032 | 1 | F | Addition of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support | 14.4.0 |
| 2017-12 | RAN#78 | R5-176958 | 1043 | 1 | F | Introduction of default message configuration for LTE_VoLTE_ViLTE_enhhancement | 14.4.0 |
| 2017-12 | RAN#78 | R5-176961 | 1030 | 1 | F | Correction to 4.5.9.4 and 4.7I | 14.4.0 |
| 2017-12 | RAN#78 | R5-176962 | 1054 | 1 | F | Correct the order of Table 4.6.3-20J and Table 4.6.3-20K | 14.4.0 |
| 2017-12 | RAN#78 | R5-176965 | 1059 | 1 | F | Update to V2V and V2X default message definitions. | 14.4.0 |
| 2017-12 | RAN#78 | R5-176991 | 1082 | 1 | F | Corrections for NPRACH-ConfigSIB-NB-DEFAULT | 14.4.0 |
| 2017-12 | RAN#78 | R5-176998 | 1031 | 1 | F | Correction to raster offset in MIB | 14.4.0 |
| 2017-12 | RAN#78 | R5-177040 | 1051 | 1 | F | Update of default ProSe discovery messages | 14.4.0 |
| 2017-12 | RAN#78 | R5-177041 | 1052 | 1 | F | Update of USIM for ProSe discovery | 14.4.0 |
| 2017-12 | RAN#78 | R5-177053 | 1089 | - | F | Addition of new generic procedure for User or Network Initiated EPS Bearer Deactivation | 14.4.0 |
| 2017-12 | RAN#78 | R5-177058 | 1077 | 1 | F | Corrections to Generic Test Procedure for user initiated release of additional PDN connectivity | 14.4.0 |
| 2017-12 | RAN#78 | R5-177062 | 1090 | - | F | Correction to RadioResourceConfigCommon-DEFAULT IE | 14.4.0 |
| 2017-12 | RAN#78 | R5-177084 | 1050 | 1 | F | Adding generic procedures for Converged IP Communications | 14.4.0 |
| 2017-12 | RAN#78 | R5-177090 | 1068 | 1 | F | Correction to MeasConfig-DEFAULT for Band > 64 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177128 | 1073 | 2 | F | Correction to UE registration procedure for an IMS enabled multi-PDN UE when Network does not support IMS VoPS | 14.4.0 |
| 2017-12 | RAN#78 | R5-177334 | 1088 | 1 | F | Sending of MaxLayersMimo-r10 IE on SCC for 4Rx TCs with more than 2 Layers | 14.4.0 |
| 2017-12 | RAN#78 | R5-177351 | 1056 | 1 | F | Addition of connection diagrams for V2V RX test cases | 14.4.0 |
| 2017-12 | RAN#78 | R5-177361 | 1057 | 1 | F | Addition of connection diagrams for V2V RX Intra-band contiguous MCC operation | 14.4.0 |
| 2017-12 | RAN#78 | R5-177378 | 1035 | 1 | F | Correction of MIB for NB-IOT InBand | 14.4.0 |
| 2017-12 | RAN#78 | R5-176210 | 1046 | - | F | Signaling Test environment for Band 72 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176216 | 1047 | - | F | Common test frequencies for Band 68 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176220 | 1048 | - | F | Signaling Test environment for Band 68 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176557 | 1069 | - | F | Addition of test frequencies for Band 71 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176817 | 1087 | - | F | Band 71 Signaling Test environment | 15.0.0 |
| 2017-12 | RAN#78 | R5-176919 | 1072 | 2 | F | Update RRCConnectionReconfiguration for 5GS | 15.0.0 |
| 2017-12 | RAN#78 | R5-177316 | 1045 | 1 | F | Common test frequencies for Band 72 | 15.0.0 |
| 2017-12 | RAN#78 | R5-177375 | 1075 | 1 | F | Correction to SystemInformationBlockType2 for NS value > 32 | 15.0.0 |
| 2018-03 | RAN#79 | R5-180317 | 1093 | - | F | New Cells for NB-IOT OTDOA tests - LTE Cell | 15.1.0 |
| 2018-03 | RAN#79 | R5-180377 | 1095 | - | F | Correction to DRX onDurationTimer for FDD-HD operation in MAC configuration | 15.1.0 |
| 2018-03 | RAN#79 | R5-180395 | 1097 | - | F | Addition of default configuration of DCI 3B to TS 36.508 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180396 | 1098 | - | F | Update of SRS switching test environment | 15.1.0 |
| 2018-03 | RAN#79 | R5-180407 | 1101 | - | F | Addition of connection diagram of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | 15.1.0 |
| 2018-03 | RAN#79 | R5-180715 | 1111 | - | F | Addition of new Enhanced LAA IE in PhysicalConfigDedicatedSCell-r10-DEFAULT message | 15.1.0 |
| 2018-03 | RAN#79 | R5-180749 | 1116 | - | F | Added R14 CA combination to Test frequencies for CA signalling test | 15.1.0 |
| 2018-03 | RAN#79 | R5-180753 | 1117 | - | F | Added R15 CA combination to Test frequencies for CA signalling test | 15.1.0 |
| 2018-03 | RAN#79 | R5-180780 | 1118 | - | F | Addition of CA_29A-70C, CA_66A-70A, CA_66A-70C, CA_66C-70A, CA_66C-70C to 36.508 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180918 | 1120 | - | F | Addition of Band 74 Signaling Test environment | 15.1.0 |
| 2018-03 | RAN#79 | R5-180934 | 1121 | - | F | Correction to test frequencies for CA_7A-7A | 15.1.0 |
| 2018-03 | RAN#79 | R5-180939 | 1122 | - | F | Correction to interval-ULHoppingConfigCommonModeA/B-r13 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180940 | 1123 | - | F | Correction to sr-ConfigIndex for CatM1 with 3MHz BW | 15.1.0 |
| 2018-03 | RAN#79 | R5-181054 | 1126 | - | F | Addition of the Band 74 information into TS 36.508 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181066 | 1127 | - | F | Correction to test frequency f4 for E-UTRA FDD bands 25 and 26 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181101 | 1130 | - | F | Connection diagrams for new 4Rx CA demodulation test cases | 15.1.0 |
| 2018-03 | RAN#79 | R5-181141 | 1096 | 1 | F | Corrections to the generic test procedures for IMS MT calls in E-UTRA | 15.1.0 |
| 2018-03 | RAN#79 | R5-181142 | 1128 | 1 | F | Correction to MeasConfig-DEFAULT | 15.1.0 |
| 2018-03 | RAN#79 | R5-181143 | 1129 | 1 | F | Correction to UE registration with test mode activation procedure 4.5.2A.3 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181196 | 1103 | 1 | F | Addition of default USIM settings for IMS eCall test cases | 15.1.0 |
| 2018-03 | RAN#79 | R5-181197 | 1113 | 1 | F | Update to generic procedure 36.508 4.5A.27 for eCall Only Supporting UE | 15.1.0 |
| 2018-03 | RAN#79 | R5-181216 | 1099 | 1 | F | Update the USIM EFs configuration for V2X. | 15.1.0 |
| 2018-03 | RAN#79 | R5-181217 | 1100 | 1 | F | Editorial, clean up of V2X default message definitions. | 15.1.0 |
| 2018-03 | RAN#79 | R5-181218 | 1119 | 1 | F | Changes and additions to GNSS requirements for V2X testing | 15.1.0 |
| 2018-03 | RAN#79 | R5-181293 | 1104 | 1 | F | Addition of new Generic Test Procedure for IPSec Tunnel Disconnection in EPC / WLAN | 15.1.0 |
| 2018-03 | RAN#79 | R5-181295 | 1106 | 1 | F | Updates to default NB-IoT messages to support the in-band operation mode in protocol test cases | 15.1.0 |
| 2018-03 | RAN#79 | R5-181309 | 1131 | - | F | Correction to RRCConnectionResume-NB specific message contents. | 15.1.0 |
| 2018-03 | RAN#79 | R5-181601 | 1094 | 1 | F | New Cells for NB-IOT OTDOA tests - NB-IOT Cell | 15.1.0 |
| 2018-03 | RAN#79 | R5-181607 | 1102 | 1 | F | Include AdditionalSpectrumEmission for V2X as per RAN2 spec | 15.1.0 |
| 2018-03 | RAN#79 | R5-181616 | 1114 | 1 | F | Test frequencies for Rel-13 LTE CA configurations | 15.1.0 |
| 2018-03 | RAN#79 | R5-181357 | 1115 | 1 | F | Test frequencies for Rel-14 LTE CA configurations | 15.1.0 |
| 2018-03 | RAN#79 | R5-181666 | 1112 | 1 | F | Addition of Connection Diagrams for V2X and V2V Sidelink Communication Performance Test Cases | 15.1.0 |
| 2018-06 | RAN#80 | R5-182376 | 1142 | - | F | Removal of square brackets for GNSS for V2X setups | 15.2.0 |
| 2018-06 | RAN#80 | R5-182415 | 1144 | - | F | Addition of GNSS information for MCS tests | 15.2.0 |
| 2018-06 | RAN#80 | R5-182425 | 1145 | - | F | Update of default NAS messages to add ENDC IEs | 15.2.0 |
| 2018-06 | RAN#80 | R5-182460 | 1146 | - | F | Correction to Attach request and Tracking Area Update request message | 15.2.0 |
| 2018-06 | RAN#80 | R5-182608 | 1154 | - | F | Addition of connection diagrams for power imbalance performance V2V test | 15.2.0 |
| 2018-06 | RAN#80 | R5-182636 | 1155 | - | F | Update SIB2 message for NR | 15.2.0 |
| 2018-06 | RAN#80 | R5-182654 | 1156 | - | F | Addition of SIB22-NB definition | 15.2.0 |
| 2018-06 | RAN#80 | R5-182660 | 1157 | - | F | Update of V2X USIM configuration to add service ID | 15.2.0 |
| 2018-06 | RAN#80 | R5-182666 | 1158 | - | F | Addition of test frequency of CA_40E | 15.2.0 |
| 2018-06 | RAN#80 | R5-182668 | 1159 | - | F | Addition of test frequency of CA_41F | 15.2.0 |
| 2018-06 | RAN#80 | R5-182811 | 1163 | - | F | Addition of test frequencies for CA signalling test | 15.2.0 |
| 2018-06 | RAN#80 | R5-182878 | 1167 | - | F | Exceptions for DL Control Channel Interference Mitigation (Type A and Type B) Tests | 15.2.0 |
| 2018-06 | RAN#80 | R5-183038 | 1173 | - | F | Addition of test frequency of CA_66A-66A-66A | 15.2.0 |
| 2018-06 | RAN#80 | R5-183055 | 1174 | - | F | Corrections to 4.5A.24 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183056 | 1139 | 1 | F | Update to UECapabilityInformation-NB message | 15.2.0 |
| 2018-06 | RAN#80 | R5-183057 | 1150 | 1 | F | Correction to t-PollRetransmit value in Table 4.6.3-19A | 15.2.0 |
| 2018-06 | RAN#80 | R5-183058 | 1170 | 1 | F | Correction to PRACH Config for CAT-M1 UEs | 15.2.0 |
| 2018-06 | RAN#80 | R5-183076 | 1172 | 1 | F | Correction to Band 66 CA band combinations | 15.2.0 |
| 2018-06 | RAN#80 | R5-183091 | 1166 | 1 | F | Addition of NR and MRDC details for UECapabilityEnquiry RRC message | 15.2.0 |
| 2018-06 | RAN#80 | R5-183092 | 1169 | 1 | F | Add ULInformationTransferMRDC message contents | 15.2.0 |
| 2018-06 | RAN#80 | R5-183125 | 1136 | 1 | F | Addition of MeasObject and MeasReport for NR | 15.2.0 |
| 2018-06 | RAN#80 | R5-183170 | 1161 | 1 | F | Introduction of DCI0A/0B for eLAA | 15.2.0 |
| 2018-06 | RAN#80 | R5-183171 | 1162 | 1 | F | Introduction of DCI4A/4B for eLAA | 15.2.0 |
| 2018-06 | RAN#80 | R5-183182 | 1149 | 1 | F | Addition of V2XoverLTEUu in V2X management data object configuration | 15.2.0 |
| 2018-06 | RAN#80 | R5-183238 | 1138 | 1 | F | Update RRCConnectionReconfiguration message for EN-DC | 15.2.0 |
| 2018-06 | RAN#80 | R5-183239 | 1147 | 1 | F | Update TFT configuration in Dedicated Bearer to Allow IP Ping Operation | 15.2.0 |
| 2018-06 | RAN#80 | R5-183247 | 1140 | 1 | F | Update to the generic registration procedure for Multi-PDNs UEs | 15.2.0 |
| 2018-06 | RAN#80 | R5-183786 | 1153 | 1 | F | Update on AT commands for V2V/X Sidelink communications | 15.2.0 |
| 2018-06 | RAN#80 | R5-183864 | 1151 | 1 | F | Definition of additional simulated cells for IncMon RRM Test Cases | 15.2.0 |
| 2018-06 | RAN#80 | R5-183867 | 1141 | 1 | F | Addition of Rel-15 NB-IoT test frequencies | 15.2.0 |
| 2018-06 | RAN#80 | R5-183894 | 1143 | 1 | F | Addition of WLAN and BLE to test frequencies | 15.2.0 |
| 2018-09 | RAN#81 | R5-184054 | 1177 | - | F | Add SRB with NR PDCP | 15.3.0 |
| 2018-09 | RAN#81 | R5-184075 | 1178 | - | F | NB-IoT timer tolerances: extension of timer value range | 15.3.0 |
| 2018-09 | RAN#81 | R5-184110 | 1180 | - | F | Correction to NB-IOT In-Band Test Frequencies | 15.3.0 |
| 2018-09 | RAN#81 | R5-184234 | 1182 | - | F | Correction to message contents for UL64QAM | 15.3.0 |
| 2018-09 | RAN#81 | R5-184235 | 1183 | - | F | Correction to default message for UL CA tests | 15.3.0 |
| 2018-09 | RAN#81 | R5-184236 | 1184 | - | F | Editorial correction to FDD Mode test frequencies | 15.3.0 |
| 2018-09 | RAN#81 | R5-184268 | 1186 | - | F | Cleaning up the default message contents and test states of V2X | 15.3.0 |
| 2018-09 | RAN#81 | R5-184282 | 1187 | - | F | Update of test frequencies for CA_42A-42A | 15.3.0 |
| 2018-09 | RAN#81 | R5-184569 | 1193 | - | F | Add Band 31 Test Frequencies for NB-IoT Inter-frequency Test cases | 15.3.0 |
| 2018-09 | RAN#81 | R5-184668 | 1195 | - | F | Editorial correction of reference document | 15.3.0 |
| 2018-09 | RAN#81 | R5-184771 | 1197 | - | F | Addition of test frequencies for CA_66A-71A, CA_66C-71A, CA_70A-71A, CA_70C-71A to 36.508 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184820 | 1199 | - | F | Addition of default laa-SCellConfiguration setting for eLAA | 15.3.0 |
| 2018-09 | RAN#81 | R5-184884 | 1200 | - | F | Update to NB-IoT test conditions for in-band / guard band | 15.3.0 |
| 2018-09 | RAN#81 | R5-185002 | 1179 | 1 | F | Correction to ATTACH ACCEPT message for SMS only | 15.3.0 |
| 2018-09 | RAN#81 | R5-185003 | 1190 | 1 | F | Correction of 6.2.3.2 Test Frequency for CA_3A-27A | 15.3.0 |
| 2018-09 | RAN#81 | R5-185023 | 1189 | 1 | F | Updates of 6.2.3.2 Test Frequency for CA_8A-27A | 15.3.0 |
| 2018-09 | RAN#81 | R5-185103 | 1196 | 1 | F | Specifying content for SCGFailureInformationNR | 15.3.0 |
| 2018-09 | RAN#81 | R5-185104 | 1201 | 1 | F | Addition of reference dedicated EPS bearer context #7 for V2X testing | 15.3.0 |
| 2018-09 | RAN#81 | R5-185113 | 1198 | 1 | F | Addition of NB_IOTenh-UEConTest test case scenarios and default SIB content | 15.3.0 |
| 2018-09 | RAN#81 | R5-185158 | 1176 | 1 | F | Update Reference Table 6.6.2-1A dedicated EPS bearer contexts | 15.3.0 |
| 2018-09 | RAN#81 | R5-185164 | 1175 | 1 | F | Update RRCConnectionReconfiguration message for EN-DC | 15.3.0 |
| 2018-09 | RAN#81 | R5-185166 | 1188 | 1 | F | Modification of EPS & Data Radio Bearer ID mapping for EN-DC Test cases | 15.3.0 |
| 2018-09 | RAN#81 | R5-185409 | 1181 | 1 | F | Addition of NB-IOT Guardband Test Frequencies for 10 MHz | 15.3.0 |
| 2018-09 | RAN#81 | R5-185449 | 1185 | 1 | F | Correction to Figure A.64 to cater for CC >= 3 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185454 | 1191 | 1 | F | Addition of antenna diagrams for IncMon RRM test cases | 15.3.0 |
| 2018-09 | RAN#81 | R5-185455 | 1192 | 1 | F | Addition of message content for RRM IncMon test cases | 15.3.0 |
| 2018-12 | RAN#82 | R5-186472 | 1203 | - | F | NB-IoT Band70 In-band frequency for 10Mhz Mid range | 15.4.0 |
| 2018-12 | RAN#82 | R5-186473 | 1204 | - | F | New NCell for NB-IOT RRM Intra-Freq Cell reselection | 15.4.0 |
| 2018-12 | RAN#82 | R5-186631 | 1207 | - | F | Correction to GNSS scenario #3 for V2X test cases | 15.4.0 |
| 2018-12 | RAN#82 | R5-186768 | 1212 | - | F | Modify the values of IEs in sps-AssistanceInformation-r14 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187002 | 1214 | - | F | Correction to Table 8.1.6.1-15a SCPTMConfiguration-NB | 15.4.0 |
| 2018-12 | RAN#82 | R5-187005 | 1215 | - | F | Adding connection diagram for eNB-IoT RRM test case 6.2.18 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187058 | 1216 | - | F | Addition of test frequency for CA_48C and CA_48A-48A | 15.4.0 |
| 2018-12 | RAN#82 | R5-187059 | 1217 | - | F | Addition of test frequency for CA_40A-40A and CA_40A-40C | 15.4.0 |
| 2018-12 | RAN#82 | R5-187114 | 1218 | - | F | Correction to RRCConnectionReconfiguration | 15.4.0 |
| 2018-12 | RAN#82 | R5-187447 | 1222 | - | F | Addition of Rel-13 CA configurations | 15.4.0 |
| 2018-12 | RAN#82 | R5-187612 | 1225 | - | F | Addition of the message content of PDCP-config for UDC | 15.4.0 |
| 2018-12 | RAN#82 | R5-187639 | 1213 | 1 | F | Update NB-IoT Rel-14 DCI format N0/N1 with HARQ process number | 15.4.0 |
| 2018-12 | RAN#82 | R5-187640 | 1226 | - | F | Clarification to inband testing for signalling test cases | 15.4.0 |
| 2018-12 | RAN#82 | R5-187643 | 1219 | 1 | F | New feMTC IEs | 15.4.0 |
| 2018-12 | RAN#82 | R5-187646 | 1220 | 1 | F | Addition of support for LWA/LWIP Test Cases in generic procedures | 15.4.0 |
| 2018-12 | RAN#82 | R5-187719 | 1224 | 1 | F | Correction to Table 6.6.2-1 in 36.508 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187729 | 1221 | 1 | F | Addition of ReportConfigInterRAT for NR | 15.4.0 |
| 2018-12 | RAN#82 | R5-187730 | 1227 | 1 | F | Updating UE registration procedure to handle UE capability in 2 steps | 15.4.0 |
| 2018-12 | RAN#82 | R5-187739 | 1210 | 1 | F | Addition of SRB4 configuration to RRC connection | 15.4.0 |
| 2018-12 | RAN#82 | R5-187740 | 1211 | 1 | F | Addition of Condition QMC for LTE QMC test | 15.4.0 |
| 2018-12 | RAN#82 | R5-187923 | 1205 | 1 | F | Definition of additional cells for IncMon RRM Test Cases | 15.4.0 |
| 2018-12 | RAN#82 | R5-187924 | 1206 | 1 | F | Message content for IncMon RRM Test Cases | 15.4.0 |
| 2018-12 | RAN#82 | R5-187976 | 1208 | 1 | F | Introduction of the support of ProSe for B72 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187978 | 1209 | 1 | F | Introduction of B68 in ProSe test frequencies table | 15.4.0 |
| 2019-03 | RAN#83 | R5-191522 | 1232 | - | F | Update RRCConnectionReconfiguration message with table references | 15.5.0 |
| 2019-03 | RAN#83 | R5-191553 | 1234 | - | F | Correction to 36.508 Table 8.3.2.3.1-1a | 15.5.0 |
| 2019-03 | RAN#83 | R5-191645 | 1235 | - | F | Updating p-MaxEUTRA-r15 IE part of RRCConnectionReconfiguration | 15.5.0 |
| 2019-03 | RAN#83 | R5-191735 | 1236 | - | F | Addition of CA_21A_42A | 15.5.0 |
| 2019-03 | RAN#83 | R5-191768 | 1241 | - | F | Correction to CA_42E channel definition | 15.5.0 |
| 2019-03 | RAN#83 | R5-191769 | 1242 | - | F | Addition of test frequency for CA_40C-40C | 15.5.0 |
| 2019-03 | RAN#83 | R5-191770 | 1243 | - | F | Addition of test frequency for CA_48D | 15.5.0 |
| 2019-03 | RAN#83 | R5-192268 | 1247 | - | F | Addition of NB-IoT DCI Physical layer parameters for SC-PTM | 15.5.0 |
| 2019-03 | RAN#83 | R5-192344 | 1231 | 1 | F | Addition of support for LWA/LWIP Test Cases in generic procedures | 15.5.0 |
| 2019-03 | RAN#83 | R5-192713 | 1244 | 1 | F | Correction to SIB2 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192859 | 1248 | 1 | F | Addition of SystemInformationBlockType24 message | 15.5.0 |
| 2019-03 | RAN#83 | R5-192860 | 1249 | 2 | F | Addition of Combination 31 of system information blocks | 15.5.0 |
| 2019-03 | RAN#83 | R5-192336 | 1233 | 1 | F | Band 53 introduction in TS 36.508 | 16.0.0 |
| 2019-03 | RAN#83 | - | - | - | - | Editorial updates of table numbering | 16.0.0 |
| 2019-06 | RAN#84 | R5-193579 | 1250 | - | F | Introduction of default message content for ReportConfigEUTRA-H1 and ReportConfigEUTRA-H2 | 16.1.0 |
| 2019-06 | RAN#84 | R5-193851 | 1255 | - | F | Update of Aerial vehicle specific IE in the ReportConfigEUTRA default message | 16.1.0 |
| 2019-06 | RAN#84 | R5-193961 | 1258 | - | F | Addition of test frequencies for signalling testing of CA_3A-38A | 16.1.0 |
| 2019-06 | RAN#84 | R5-193982 | 1259 | - | F | Corrections to dedicated EPS bearer context #6 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194641 | 1268 | - | F | Addition of test frequencies for signalling testing of CA_5A-40A and CA_5A-40C | 16.1.0 |
| 2019-06 | RAN#84 | R5-194736 | 1269 | - | F | Resubmission of Addition of default IE contents for MeasGapConfig-CE | 16.1.0 |
| 2019-06 | RAN#84 | R5-194770 | 1260 | 1 | F | Introduction of GNSS requirements for aerial testing | 16.1.0 |
| 2019-06 | RAN#84 | R5-194820 | 1272 | - | F | Introducing conditions for EN-DC PSCell handover in RRCConnectionReconfiguration | 16.1.0 |
| 2019-06 | RAN#84 | R5-194875 | 1252 | 1 | F | Update LTE MeasObjectNR | 16.1.0 |
| 2019-06 | RAN#84 | R5-194876 | 1266 | 1 | F | Correction to SMTC periodicity | 16.1.0 |
| 2019-06 | RAN#84 | R5-194877 | 1267 | 1 | F | Update of EMM messages for NR | 16.1.0 |
| 2019-06 | RAN#84 | R5-194880 | 1251 | 1 | F | Introduction of test frequencies for LTE Band 85 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194960 | 1271 | 1 | F | Common test frequencies for Band 73 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195069 | 1270 | 1 | F | Addition of connection diagrams for FD-MIMO | 16.1.0 |
| 2019-06 | RAN#84 | R5-195308 | 1257 | 1 | F | Correction to 4.5A.16.3 procedure for UEs supporting Multi-PDN and Internet configured as secondary PDN | 16.1.0 |
| 2019-06 | RAN#84 | R5-195321 | 1263 | 2 | F | Update of Aerial vehicle specific default message | 16.1.0 |
| 2019-09 | RAN#85 | R5-195800 | 1273 | - | F | Addition of test frequencies for signalling testing of CA_11A_41A, CA_11A_41C, CA_11A_42A and CA_11A_42C | 16.2.0 |
| 2019-09 | RAN#85 | R5-196004 | 1276 | - | F | Update to 4.5A.16 generic procedure for UEs supporting Multi-PDN and Internet configured as secondary PDN with MobileData off | 16.2.0 |
| 2019-09 | RAN#85 | R5-196599 | 1284 | - | F | Correction SIB5-NB for RRM inter-frequency tests | 16.2.0 |
| 2019-09 | RAN#85 | R5-196647 | 1285 | - | F | Update of default messages in regard to NR relevant content | 16.2.0 |
| 2019-09 | RAN#85 | R5-196697 | 1287 | - | F | Update of RRC message SCGFailureInformationNR | 16.2.0 |
| 2019-09 | RAN#85 | R5-196725 | 1288 | - | F | Signalling Test environment for Band 73 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196760 | 1289 | - | F | Addition of TDD reference test frequencies for CA in operating band 41 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196777 | 1290 | - | F | Correction to NOTE on MCC/MNC setting for operation in EUTRA band 6 | 16.2.0 |
| 2019-09 | RAN#85 | R5-196838 | 1291 | - | F | Typo corrections on NB-IoT test frequencies | 16.2.0 |
| 2019-09 | RAN#85 | R5-196971 | 1274 | 1 | F | Update to dataCodingScheme in SystemInformationBlockType11, required for ETWS test case | 16.2.0 |
| 2019-09 | RAN#85 | R5-196977 | 1280 | 1 | F | Correction of USIM file format for V2X | 16.2.0 |
| 2019-09 | RAN#85 | R5-197178 | 1277 | 1 | F | Update of ReportConfigEUTRA default message | 16.2.0 |
| 2019-09 | RAN#85 | R5-197251 | 1278 | 1 | F | Update to Bluetooth and WLAN cells configuration | 16.2.0 |
| 2019-09 | RAN#85 | R5-197257 | 1293 | 1 | F | Title: Update EN-DC PSCell handover condition in RRCConnectionReconfiguration | 16.2.0 |
| 2019-09 | RAN#85 | R5-197453 | 1283 | 1 | F | Addition of test freq for CA_1A-1A to 36.508 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197474 | 1281 | 1 | F | Addition of NB-IOT test frequencies for band FDD85 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197488 | 1292 | 1 | F | Removing duplicate Table 4.3.1.2.9A-10: Test frequencies for CA_41F | 16.2.0 |
| 2019-12 | RAN#86 | R5-198034 | 1294 | - | F | Modification of GNSS Scenarios for aerial testing | 16.3.0 |
| 2019-12 | RAN#86 | R5-198066 | 1296 | - | F | Correction to GNSS Scenario for aerial testing | 16.3.0 |
| 2019-12 | RAN#86 | R5-198514 | 1302 | - | F | Update uplinkPowerControlDedicatedSCell-v1530 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198995 | 1298 | 1 | F | Update upperLayerIndication-r15 in SystemInformationBlockType2 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198996 | 1301 | 1 | F | Addition of N1 interworking to ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST messages | 16.3.0 |
| 2019-12 | RAN#86 | R5-199411 | 1300 | 1 | F | Addition of new SIB combination for RRM EN-DC tests | 16.3.0 |
| 2019-12 | RAN#86 | R5-199469 | 1297 | 1 | F | Addition of TDD reference test frequencies for CA_48C | 16.3.0 |
| 2020-03 | RAN#87 | R5-200648 | 1308 |  | F | Updates to PDN Connectivity Request message | 16.4.0 |
| 2020-03 | RAN#87 | R5-200649 | 1309 |  | F | Updates to ESM Information Response message | 16.4.0 |
| 2020-03 | RAN#87 | R5-200704 | 1310 |  | F | Correction to QuantityConfig-DEFAULT | 16.4.0 |
| 2020-03 | RAN#87 | R5-200917 | 1306 | 1 | F | Adding missing R15 IEs into RRCConnectionReconfiguration | 16.4.0 |
| 2020-03 | RAN#87 | R5-200989 | 1305 | 1 | F | Configuring dualConnectivityPHR IE for EN-DC scenarios | 16.4.0 |
| 2020-03 | RAN#87 | R5-201008 | 1304 | 1 | F | Updates to condition for UEs indicating support of N1 | 16.4.0 |
| 2020-03 | RAN#87 | R5-201110 | 1307 | 1 | F | Addition of new Generic Test Procedure for IMS Re-registration in E-UTRAN and updates to clause 4.5A.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201426 | 1311 | - | F | Correction to Table 4.4.3.3-20-SIB24 of LTE | 16.5.0 |
| 2020-06 | RAN#88 | R5-201485 | 1312 | - | F | Update table 4.6.6-8A | 16.5.0 |
| 2020-06 | RAN#88 | R5-201497 | 1313 | - | F | Updates to RRCConnectionReconfiguration for NR SCell addition, modification and release | 16.5.0 |
| 2020-06 | RAN#88 | R5-202441 | 1321 | - | F | Correction of Test Frequencies for Band 73 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202694 | 1317 | 1 | F | Correction to mobility control information elements | 16.5.0 |
| 2020-06 | RAN#88 | R5-202695 | 1320 | 1 | F | Addition of Default configuration of IE ReportConfigInterRAT for SFTD measurement | 16.5.0 |
| 2020-06 | RAN#88 | R5-202784 | 1319 | 1 | F | Addition of NB-IoT test frequencies of TDD band 41 | 16.5.0 |
| 2020-06 | RAN#88 | R5-202792 | 1314 | 1 | F | Addition of NB-IOT test frequencies for band 73 | 16.5.0 |
| 2020-06 | RAN#88 | R5-203049 | 1318 | 1 | F | Addition of Common Test Environment for short TTI and short processing time | 16.5.0 |
| 2020-09 | RAN#89 | R5-203494 | 1322 | - | F | Update to ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message | 16.6.0 |
| 2020-09 | RAN#89 | R5-203495 | 1323 | - | F | Correction to USIM content for eCall over IMS test cases | 16.6.0 |
| 2020-09 | RAN#89 | R5-203922 | 1326 | - | F | Update of common test environment for short TTI and short processing time TCs | 16.6.0 |
| 2020-09 | RAN#89 | R5-204348 | 1332 | - | F | Adding signalling test frequency for 1A-38A | 16.6.0 |
| 2020-09 | RAN#89 | R5-204488 | 1325 | 1 | F | Corrections to Default contents of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST MESSAGE for S1 to N1 mode | 16.6.0 |
| 2020-09 | RAN#89 | R5-204496 | 1328 | 1 | F | Update to Common contents of system information messages for NB_IOTenh2 | 16.6.0 |
| 2020-09 | RAN#89 | R5-204499 | 1329 | 1 | F | Updates to IEs for Early Data Transmission test cases | 16.6.0 |
| 2020-09 | RAN#89 | R5-204521 | 1324 | 1 | F | Update the related RRC messages to support Inter-RAT handover from NR to EN-DC | 16.6.0 |
| 2020-09 | RAN#89 | R5-204966 | 1330 | 1 | F | Updates to IEs for RF eMTC TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205078 | 1333 |  | F | Introduction of test frequencies for LTE Bands 87 and 88 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205109 | 1334 |  | F | Addition of 41A-48A to 36.508 Table 6.2.3.2-2 | 16.7.0 |
| 2020-12 | RAN#90 | R5-205143 | 1335 |  | F | Update of for UECapabilityEnquiry | 16.7.0 |
| 2020-12 | RAN#90 | R5-205761 | 1336 |  | F | Correction to default physical layer parameters for DCI format 7 and default downlink TM for sTTI | 16.7.0 |
| 2020-12 | RAN#90 | R5-205890 | 1338 |  | F | Addition of IE for WUS into SIB22-NB | 16.7.0 |
| 2020-12 | RAN#90 | R5-206392 | 1339 | 1 | F | Correction to RACH-ConfigCommon IE | 16.7.0 |
| 2021-03 | RAN#91 | R5-210201 | 1341 | - | F | MCPTT Packet Filter | 16.8.0 |
| 2021-03 | RAN#91 | R5-210342 | 1342 | - | F | Correction to NB-IoT Common contents of system information messages | 16.8.0 |
| 2021-03 | RAN#91 | R5-210399 | 1343 | - | F | Inclusion of SK-Counter IE in RRCConnectionReconfiguration | 16.8.0 |
| 2021-03 | RAN#91 | R5-211127 | 1347 | - | F | Minor corrections of 4.1 for test environment conditions | 16.8.0 |
| 2021-03 | RAN#91 | R5-211450 | 1348 | 1 | F | Editorial updates to RACH-ConfigCommon IE | 16.8.0 |
| 2021-03 | RAN#91 | R5-211505 | 1340 | 1 | F | Correction to emergency bearer service over S1 for eCall test cases | 16.8.0 |
| 2021-03 | RAN#91 | R5-211506 | 1349 | 1 | F | Editorial for Table 6.6.2-10 Reference packet filter #9 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211507 | 1350 | 1 | F | Correction to Test frequencies for NB-IoT FDD MFBI | 16.8.0 |
| 2021-03 | RAN#91 | R5-211891 | 1346 | 1 | F | Updating the value of PLTE for EN-DC test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212179 | 1352 | - | F | Change to the Geographical Area for V2X and MCS testing | 16.9.0 |
| 2021-06 | RAN#92 | R5-212271 | 1353 | - | F | Addition of common aspects of MTC wake-up signal | 16.9.0 |
| 2021-06 | RAN#92 | R5-212273 | 1354 | - | F | Update to RRCConnectionReconfiguration in Table 4.6.1-8 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212280 | 1355 | - | F | Addition of common aspects of narrowband wake-up signal | 16.9.0 |
| 2021-06 | RAN#92 | R5-212391 | 1356 | - | F | MCPTT Packet Filter | 16.9.0 |
| 2021-06 | RAN#92 | R5-212614 | 1359 | - | F | Addition of TDD 46 reference test frequencies for CA | 16.9.0 |
| 2021-06 | RAN#92 | R5-213122 | 1361 | - | F | Updates to default contents of RRC messages for RACS | 16.9.0 |
| 2021-06 | RAN#92 | R5-213124 | 1362 | - | F | Updates to default contents of NAS messages for RACS | 16.9.0 |
| 2021-06 | RAN#92 | R5-213546 | 1364 | 1 | F | Update to NB-IoT Common test environment parameters | 16.9.0 |
| 2021-06 | RAN#92 | R5-213549 | 1357 | 1 | F | Correction to RRC messages for LTE CHO | 16.9.0 |
| 2021-06 | RAN#92 | R5-213939 | 1360 | 1 | F | Correction to the condition of p-MaxEUTRA-r15 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214013 | 1365 | 1 | F | Addition of NB-IoT test frequencies for band 7, 42, 43 and 65 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214031 | 1366 | 1 | F | Correction to band 71 guard-band test frequencies for NB-IoT | 16.9.0 |
| 2021-06 | RAN#92 | R5-214032 | 1367 | 1 | F | Adding Test frequencies for NB-IoT FDD MFBI for band 4, 10 and 66 | 16.9.0 |
| 2021-06 | RAN#92 | R5-214043 | 1358 | 1 | F | LTE_Test frequencies correction for CA_66A-66A | 16.9.0 |
| 2021-09 | RAN#93 | R5-214623 | 1368 | - | F | Update to generic procedure 4.5A.5 for IMS Emergency call establishment in EUTRA: Limited Service | 16.10.0 |
| 2021-09 | RAN#93 | R5-214938 | 1369 | - | F | Addition of test frequency of CA_66A-66A-66A | 16.10.0 |
| 2021-09 | RAN#93 | R5-214976 | 1370 | - | F | Addition of NB-IoT common configuration of System Information Block 23 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215574 | 1376 | - | F | Correction to LTE CA_41E channel spacing | 16.10.0 |
| 2021-09 | RAN#93 | R5-215823 | 1375 | 1 | F | NB-IoT Test Frequency Corrections to align with RAN4 and US FCC | 16.10.0 |
| 2021-09 | RAN#93 | R5-216003 | 1371 | 1 | F | Update to GNSS nominal start time for V2X testing | 16.10.0 |
| 2021-09 | RAN#93 | R5-216280 | 1372 | 1 | F | Updates to default contents of NAS messages for RACS | 16.10.0 |
| 2021-09 | RAN#93 | R5-216082 | 1373 | 1 | F | Updates to test frequencies for LTE band 24 | 17.0.0 |
| 2021-09 | RAN#93 | R5-216335 | 1374 | 1 | F | Updates to signalling test frequencies for LTE band 24 | 17.0.0 |