# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is the first part of a multi-part conformance specification. 3GPP TS 36.523-2 [18] contains a pro-forma for the Implementation Conformance Statement (ICS) and an applicability table, indicating the release from which each test case is applicable. 3GPP TS 36.523-3 [19] contains a TTCN-3 design frame work, test model and Test Suites (the detailed test specifications in TTCN-3).

For at least a minimum set of services, the prose descriptions of test cases will have a matching detailed test case implemented in TTCN [19].

The present document may contain descriptions of tests for additional services, but these tests may not have matching TTCN test cases.

The present document will not contain any tests on the USIM, or the interface between the UE and the USIM. These tests are documented elsewhere.

# 1	Scope

The present document specifies the protocol conformance testing for the 3rd Generation E-UTRAN User Equipment (UE).

This is the first part of a multi-part test specification. The following information can be found in this part:

-	the overall test structure;

-	the test configurations;

-	the conformance requirement and reference to the core specifications;

-	the test purposes; and

-	a brief description of the test procedure, the specific test requirements and short message exchange table.

The following information relevant to testing could be found in accompanying specifications:

-	the default setting of the test parameters [18];

-	the applicability of each test case [19].

A detailed description of the expected sequence of messages could be found in the 3rd part of this test specification.

The Implementation Conformance Statement (ICS) pro-forma could be found in the 2nd part of the present document.

The present document is valid for UE implemented according to 3GPP releases starting from Release 8 up to the Release indicated on the cover page of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[3]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[4]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[5]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE) Conformance Testing".

[6]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[7]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[8]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[9]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[10]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[11]	3GPP TS 36.302: "Services provided by the physical layer for E-UTRA".

[12]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA) User Equipment (UE) Procedures in idle mode ".

[13]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA) User Equipment (UE) Radio Access capabilities ".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA) Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Link Control (RLC) protocol specification".

[16]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA) Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC) Protocol Specification".

[18]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Common Test Environments for User Equipment (UE) Conformance Testing".

[19]	3GPP TS 36.523-2: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[20]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[21]	3GPP TR 24.801: "3GPP System Architecture Evolution; CT WG1 Aspects".

[22]	3GPP TS 23.401: "3GPP System Architecture Evolution; GPRS enhancements for E-UTRAN access".

[23]	3GPP TS 51.010-1: "Mobile Station (MS) conformance specification; Part 1: Conformance specification".

[24]	ISO/IEC 9646 (all parts): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework".

[25]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Special conformance testing functions for User Equipment (UE)".

[26]	3GPP TS 33.102: "3G Security; Security architecture"

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[29]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[30]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"

[31]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[32]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[33]	3GPP2 A.S0008-C v4.0: "Interoperability Specification (IOS) for High Rate Packet Data (HRPD) Radio Access Network Interfaces with Session Control in the Access Network"

[34]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management"

[35]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification ".

[36]	3GPP TS 45.008: "Technical Specification Group GSM/EDGE Radio Access Network; Radio subsystem link control"

[37]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control (RLC) / Medium Access Control (MAC) protocol"

[38]	3GPP TS 44.018: "Technical Specification Group GSM/EDGE Radio Access Network; Mobile radio interface layer 3 specification; Radio Resource Control Protocol"

[39]	3GPP TS 43.022: "Functions related to Mobile Station (MS) in idle mode and group receive mode"

[40]	3GPP TS 25.133: "Requirements for support of Radio Resource Management (FDD)"

[41]	3GPP TS 25.123: "Requirements for support of Radio Resource Management (TDD)"

[42]	3GPP TS 25.304: "UE Procedures in Idle Mode and Procedures for Cell Reselection in Connected Mode"

[43]	3GPP TS 25.367: "Mobility Procedures for Home NodeB; Overall Description; Stage 2"

[44]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation"

[45]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)"

[46]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3"

[47]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[48]	3GPP2 C.S0005- Fv1.0: "Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems - Release F".

[49]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)"

[50]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs"

[51]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects"

[52]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3"

[53]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects"

[54]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface"

[55]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)"

[56]	IETF RFC 1035: "DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION"

[57]	IETF RFC 5996: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[58]	3GPP TS 27.007: "AT command set for User Equipment (UE)"

[59]	3GPP TS 36.579-1: "Mission Critical (MC) services over LTE; Part 1: Common test environment".

[60]	3GPP TS 36.579-2: "Mission Critical (MC) services over LTE; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification".

[61]	3GPP TS 22.280: "Common functional architecture to support mission critical services; Stage 2".

[62]	3GPP TS 23.203: "Policy and charging control architecture".

[63]	3GPP TS 36.579-5: "Mission Critical (MC) services over LTE; Part 5: Abstract test suite (ATS)".

[64]	3GPP TS 36.579-6: "Mission Critical (MC) services over LTE; Part 6: Mission Critical Video (MCVideo) User Equipment (UE) Protocol conformance specification".

[65]	3GPP TS 36.579-7: "Mission Critical (MC) services over LTE; Part 7: Mission Critical Data (MCData) User Equipment (UE) Protocol conformance specification".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Floor: Floor(x) is the largest integer smaller than or equal to x.

Ceil: Ceil (x) is the smallest integer larger than or equal to x.

sTTI: A transmission time interval (TTI) of either one slot or one subslot as defined in TS 36.211 [44] on either uplink or downlink.

For the purposes of the present document in regard to terminology used for ProSe conformance testing terms and definitions given in 3GPP TS 23.303 [51] apply.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ENB	Evolved Node B

FDD	Frequency Division Duplex

FFS	For Further Study

ICS 	Implementation Conformance Statement

ROHC	RObust Header Compression

SPDCCH	Short Physical Downlink Control channel

SPUCCH	Short Physical Uplink Control channel

USD	User Service Description

# 4	Overview

## 4.1	Test methodology

### 4.1.1	Testing of optional functions and procedures

Any function or procedure which is optional, as indicated in the present document, may be subject to a conformance test if it is implemented in the UE.

A declaration by the apparatus supplier (ICS) is used to determine whether an optional function/procedure has been implemented.

### 4.1.2	Test interfaces and facilities

Detailed descriptions of the UE test interfaces and special facilities for testing are provided in 3GPP TS 36.509 [25].

## 4.2	Implicit testing

For some 3GPP signalling and protocol features conformance is not verified explicitly in the present document. This does not imply that correct functioning of these features is not essential, but that these are implicitly tested to a sufficient degree in other tests.

## 4.3	Repetition of tests

As a general rule, the test cases specified in the present document are highly reproducible and don't need to be repeated unless otherwise stated. However, the rate of correct UE behaviour such as cell re-selection, measurement and handover is specified statistically, e.g. "at least 90%" [34]. Additionally, in some of the test cases , presented in TS 36.523-3 clause 7.3.2, HARQ retransmissions are not tolerated, because of characteristics of the test case [20]. It is the case that for those tests expecting such UE behaviour (stated above) are not 100% reproducible, therefore if a UE does not pass the test after its first trial and the tester suspects the reason is due to the statistical nature of the test, then the UE should be re-tested one or more times. For non-HARQ retransmission tolerant test cases re-testing applies, when test case does not pass, which is caused by unspecified HARQ retransmission.

## 4.4	Handling of differences between conformance requirements in different releases of cores specifications

The conformance requirements which determine the scope of each test case are explicitly copy-pasted from relevant core specifications in the especially dedicated for this section of each test with the title 'Conformance requirements'.

When differences between conformance requirements in different releases of the cores specifications have impact on the Pre-test conditions, Test procedure sequence or/and the Specific message contents, the Conformance requirements related to different releases are specified separately with clear indication of the Release of the spec from which they were copied.

When there is no Release indicated for a conformance requirement text, this should be understood either as the Conformance requirements in the latest version of the spec with release = the TC Applicability release (which can be found in TS 36.523-2, Table 4-1: Applicability of tests and additional information for testing, column 'Release'), or, as the Conformance requirements in the latest version of the spec of the release when the feature was introduced to the core specs.

# 5	Reference conditions

The reference environments used by all signalling and protocol tests will be specified in TS 36.508 [18]. Where a test requires an environment that is different, this will be specified in the test itself.

## 5.1	Generic setup procedures

A set of basic generic procedures for radio resource signalling, and generic setup procedures for layer 3 NAS signalling will be described in TS 36.508 [18]. These procedures will be used in numerous test cases throughout the present document.

# 6	Idle mode operations

## 6.0	Introduction

### 6.0.1	PLMN and TAC

The PLMN numbers indicated in Table 6.0.1-1 are used in test cases to associate a cell with an MCC and MNC for that cell. If no PLMN is explicitly specified, the default value is PLMN 1.

Table 6.0.1-1: Tracking Area Identity (TAI) in System Information Block Type 1
broadcast on the BCCH (E-UTRAN)

| PLMN | MCC1 | MCC2 | MCC3 | MNC1 | MNC2 | MNC3 | TAC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 0 | 1 | Not present | See Table 6.0.1-2 |
| 2 | 0 | 0 | 2 | 1 | 1 | Not present |  |
| 3 | 0 | 0 | 3 | 2 | 1 | Not present |  |
| 4 | 0 | 0 | 4 | 3 | 1 | Not present |  |
| 5 | 0 | 0 | 5 | 4 | 1 | Not present |  |
| 6 | 0 | 0 | 6 | 5 | 1 | Not present |  |
| 7 | 0 | 0 | 7 | 6 | 1 | Not present |  |
| 8 | 0 | 0 | 8 | 7 | 1 | Not present |  |
| 9 | 0 | 0 | 9 | 0 | 2 | Not present |  |
| 10 | 0 | 1 | 0 | 1 | 2 | Not present |  |
| 11 | 0 | 1 | 1 | 2 | 2 | Not present |  |
| 12 | 0 | 1 | 2 | 3 | 2 | Not present |  |
| 13 | 0 | 0 | 2 | 8 | 1 | Not present |  |
| 14 | 0 | 0 | 2 | 9 | 1 | Not present |  |
| 15 | 0 | 0 | 1 | 1 | 1 | Not present |  |
| 16 | 0 | 0 | 1 | 2 | 1 | Not present |  |
| 17 | 0 | 0 | 1 | 3 | 1 | Not present |  |
| 18 | 0 | 0 | 1 | 4 | 1 | Not present |  |

NOTE:	The Location Area Information (LAI) for UTRA and GSM could be re-used from TS 34.123-1 clause 6.

If no TAC for E-UTRAN cell is explicitly specified in the test case description, the TACs for E-UTRAN cells indicated in Table 6.0.1-2 are used in the test case.

Table 6.0.1-2: Tracking Area Code (TAC) for E-UTRAN cells

| cell ID | TAC |
| --- | --- |
| Cell 1 | 1 |
| Cell 2 | 2 |
| Cell 3 | 3 |
| Cell 4 | 4 |
| Cell 6 | 6 |
| Cell 10 | 10 |
| Cell 11 | 11 |
| Cell 12 | 12 |
| Cell 13 | 13 |
| Cell 14 | 14 |
| Cell 23 | 23 |
| Cell 28 | 28 |
| Cell 29 | 29 |
| Cell 30 | 30 |

If no RAI for UTRAN cell is explicitly specified in the test case description, the RAI for UTRAN cells indicated in Table 4.4.4-2 in [18] are used in the test case.

### 6.0.2	Other

#### 6.0.2.1	Values of parameters calculated by the UE

Values of some parameters calculated by the UE, such as Srxlev, are provided in the test case description without explicitly taking into account SS signal generation and UE measurement uncertainties. These values are for information purpose only and therefore shall not be directly used to determine the UE behaviour; instead a range of values resulting from uncertainties specific for given test case conditions shall be considered.

## 6.1	In a pure E-UTRAN environment

### 6.1.1	PLMN selection

#### 6.1.1.1	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode

6.1.1.1.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and RPLMN, HPLMN, UPLMN and OPLMN E-UTRAN cells available and UE is fitted with a USIM indicating RPLMN should be selected }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the RPLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE camped on an E-UTRAN VPLMN cell and cells of a higher priority E-UTRAN PLMN available }

ensure that {

when { higher priority PLMN search timer T expires }

then { UE selects and camps on a cell of the highest priority PLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(3)

with { UE in Automatic network selection mode and HPLMN, UPLMN and OPLMN E-UTRAN cells available and UE is fitted with a USIM with Access Technology data files for each PLMN and there are no equivalent HPLMNs defined}

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the highest priority PLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(4)

with { UE camped on an E-UTRAN VPLMN cell and cells of a E-UTRAN HPLMN available }

ensure that {

when { higher priority PLMN search timer T expires }

then { UE selects and camps on a cell of HPLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1, 4.4.3.1.1 and 4.4.3.3.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see clause 4.5.2) attempts to perform a Location Registration.

...

If successful registration is achieved, the MS indicates the selected PLMN.

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then this operating mode shall be used.

...

NOTE 1: 	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	…

v)	...

When following the above procedure the following requirements apply:

a)	…

b)	...

c)	In ii and iii, the MS should limit its search for the PLMN to the access technology or access technologies associated with the PLMN in the appropriate PLMN Selector with Access Technology list (User Controlled or Operator Controlled selector list). An MS using a SIM without access technology information storage (i.e. the "User Controlled PLMN Selector with Access Technology" and the "Operator Controlled PLMN Selector with Access Technology" data files are not present) shall instead use the "PLMN Selector" data file, for each PLMN in the "PLMN Selector" data file, the MS shall search for all access technologies it is capable of and shall assume GSM access technology as the highest priority radio access technology.

d)	...

e)	...

f)	In i, the MS shall search for all access technologies it is capable of. No priority is defined for the preferred access technology and the priority is an implementation issue, but "HPLMN Selector with Access Technology" data file on the SIM may be used to optimise the procedure.

g)	...

h)	...

NOTE 1:	...

NOTE 2:	...

NOTE 3:	High quality signal is defined in the appropriate AS specification.

If successful registration is achieved, the MS indicates the selected PLMN.

[TS 23.122, clause 4.4.3.3]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value of timer T may be stored in the SIM. The interpretation of the stored value depends on the radio capabilities supported by the MS:

-	For an MS that does not only support any of the following or a combination of EC-GSM-IoT or Category M1 or Category NB1(as defined in 3GPP TS 36.306 [54]): T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 60 minutes is used for T.

-	For an MS that only supports any of the following or a combination of EC-GSM-IoT or Category M1 or Category NB1(as defined in 3GPP TS 36.306 [54]): T is either in the range 2 hours to 240 hours, using 2 hour steps from 2 hours to 80 hours and 4 hour steps from 84 hours to 240 hours, or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 72 hours is used.

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

The MS does not stop timer T, as described in 3GPP TS 24.008 [23] and 3GPP TS 24.301 [23A], when it activates power saving mode (PSM) (see 3GPP TS 23.682 [27A]).

The MS can be configured for Fast First Higher Priority PLMN search as specified in 3GPP TS 31.102 [40] or 3GPP TS 24.368 [50]. Fast First Higher Priority PLMN search is enabled if the corresponding configuration parameter is present and set to enabled. Otherwise, Fast First Higher Priority PLMN search is disabled.

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode when the MS is roaming, and not while the MS is attached for emergency bearer services or has a PDN connection for emergency bearer services;

b)	The MS shall make the first attempt after a period of at least 2 minutes and at most T minutes:

-	only after switch on if Fast First Higher Priority PLMN search is disabled; or

-	after switch on or upon selecting a VPLMN if Fast First Higher Priority PLMN search is enabled.

c)	The MS shall make the following attempts if the MS is on the VPLMN at time T after the last attempt;

d)	Periodic attempts shall only be performed by the MS while in idle mode;

d1)	periodic attempts may be postponed while the MS is in power saving mode (PSM) (see 3GPP TS 23.682 [27A]).

e)	If the HPLMN (if the EHPLMN list is not present or is empty) or a EHPLMN (if the list is present) or a higher priority PLMN is not found, the MS shall remain on the VPLMN.

f)	In steps i), ii) and iii) of subclause 4.4.3.1.1 the MS shall limit its attempts to access higher priority PLMN/access technology combinations to PLMN/access technology combinations of the same country as the current serving VPLMN, as defined in Annex B.

g)	Only the priority levels of Equivalent PLMNs of the same country as the current serving VPLMN, as defined in Annex B, shall be taken into account to compare with the priority level of a selected PLMN.

h)	If the PLMN of the highest priority PLMN/access technology combination available is the current VPLMN, or one of the PLMNs in the "Equivalent PLMNs" list, the MS shall remain on the current PLMN/access technology combination.

6.1.1.1.3	Test description

6.1.1.1.3.1	Pre-test conditions

System Simulator:

-	Four inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting PLMNs as indicated in Table 6.1.1.1.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.1.3.1-1.

Table 6.1.1.1.3.1–1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |
| 14 | PLMN3 | 001 | 31 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.1.3.1-2.

Table 6.1.1.1.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 3 | Default PLMN3 PLMN2 Remaining mandatory entries use default values | Default All specified E-UTRAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) 1 (2 hours) | E-UTRAN Category M1 of E-UTRAN enhanced-MTC |

Preamble:

-	The UE is made to camp on Cell 12 and then Switched OFF (State 1).

6.1.1.1.3.2	Test procedure sequence

Table 6.1.1.1.3.2 – 1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.1.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.1.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Cell 14 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | “Off” | -85 | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | -85 | -85 | -85 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” | -85 | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.1.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 12? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | Check: Does the UE send an RRCConnectionRequest on Cell 1 after 120 seconds, but before 660 / 7500 (for Category M1) seconds (Note 2) from power on? | --> | RRCConnectionRequest | 4 | P |
| 21-25 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 26 | Void | - | - | - | - |
| - | EXCEPTION: Step 26Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 26Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 26B | Check: Is PLMN4 indicated by the UE? | - | - | 2 | P |
| 27 | Void | - | - | - | - |
| 28 | SS adjusts cell levels according to row T2 of table 6.1.1.1.3.2-1 | - | - | - | - |
| 29 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 13? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | - |
| - | EXCEPTION: Step 29Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 29Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 30 | SS adjusts cell levels according to row T3 of table 6.1.1.1.3.2-1 | - | - | - | - |
| 31 | Check: Is PLMN2 indicated by the UE? | - | - | 3 | P |
| 32 | Check: Does the UE send an RRCConnectionRequest on Cell 14 after 120 seconds, but before 660 / 7500 (for Category M1) seconds after step29 ? (Note 1 and 2) | --> | RRCConnectionRequest | 2 | P |
| 33-37 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 38 | Check: Is PLMN3 indicated by the UE? | - | - | 2 | P |
| 39 | Void | - | - | - | - |
| Note 1: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-12). Hence, window between 120s to T+Tolerance is being used,, where the high priority PLMN search timer T defined by EFHPPLMN. Note 2: Tolerance of 5min is added to allow time for the UE to find the proper PLMN |  |  |  |  |  |

6.1.1.1.3.3	Specific message contents

None

#### 6.1.1.1a	PLMN selection / Automatic mode/between FDD and TDD

6.1.1.1a.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and camp on a cell of a PLMN with FDD mode }

ensure that {

when { UE is powered on or return to coverage }

then { UE selects a cell of a PLMN with TDD mode and UE attempts registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and camp on a cell of a PLMN with TDD mode}

ensure that {

when { UE is powered on or return to coverage }

then { UE selects a cell of a PLMN with FDD mode and UE attempts registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1, 4.4.3.1.1 and 4.4.3.3.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see clause 4.5.2) attempts to perform a Location Registration.

...

If successful registration is achieved, the MS indicates the selected PLMN.

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then this operating mode shall be used.

...

NOTE 1: 	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

...

NOTE2:	…

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	…

v)	...

When following the above procedure the following requirements apply:

a)	…

b)	...

c)	In ii and iii, the MS should limit its search for the PLMN to the access technology or access technologies associated with the PLMN in the appropriate PLMN Selector with Access Technology list (User Controlled or Operator Controlled selector list). An MS using a SIM without access technology information storage (i.e. the "User Controlled PLMN Selector with Access Technology" and the "Operator Controlled PLMN Selector with Access Technology" data files are not present) shall instead use the "PLMN Selector" data file, for each PLMN in the "PLMN Selector" data file, the MS shall search for all access technologies it is capable of and shall assume GSM access technology as the highest priority radio access technology.

d)	...

e)	...

f)	In i, the MS shall search for all access technologies it is capable of. No priority is defined for the preferred access technology and the priority is an implementation issue, but "HPLMN Selector with Access Technology" data file on the SIM may be used to optimise the procedure.

g)	...

h)	...

NOTE 1:	...

NOTE 2:	...

NOTE 3:	High quality signal is defined in the appropriate AS specification.

If successful registration is achieved, the MS indicates the selected PLMN.

...

6.1.1.1a.3	Test description

6.1.1.1a.3.1	Pre-test conditions

System Simulator

-	Two FDD and TDD inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.3 are configured broadcasting default PLMNs as indicated in TS 36.508 Table 4.4.2-2.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.1a.3.1-1.

Table 6.1.1.1a.3.1-1: PLMN identifiers

| Cell | PLMN name | MODE |
| --- | --- | --- |
| 1 | PLMN1 | FDD |
| 28 | PLMN2 | TDD |

UE

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.1a.3.1-2.

-	The UE is registered to PLMN1 before it is switched off.

Table 6.1.1.1a.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN2. |  |
| EFPLMNwAcT | 1 2 | Default PLMN2 Remaining mandatory entries use default values | Default E-UTRAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN2 | E-UTRAN |

Preamble

-	The UE is in state Switched OFF (State 1).

6.1.1.1a.3.2	Test procedure sequence

Table 6.1.1.1a.3.2 – 1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.1a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.1a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 28 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.1a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2-17 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 18 | PLMN1 is indicated by the UE. | - | - | - | - |
| 19 | Power off the UE, and wait [15 seconds]. |  |  | - | - |
| 20 | SS adjusts cell levels according to row T1 of table 6.1.1.1a.3.2-1 | - | - | - | - |
| 21 | Power on the UE |  |  |  |  |
| 22 | Check: Does the UE send an RRCConnectionRequest on Cell 28? | - | - | 1 | P |
| 23-37 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 28. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 38 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 39 | Power off the UE, and wait [15 seconds]. | - | - | - | - |
| 40 | SS adjusts cell levels according to row T2 of table 6.1.1.1a.3.2-1 | - | - | - | - |
| 41 | Power on the UE | - | - | - | - |
| 42 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | - | - | 2 | P |
| 43-47 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 48 | Check: Is PLMN1 indicated by the UE? | - | - | 2 | P |

6.1.1.1a.3.3	Specific message contents

None

#### 6.1.1.1b	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation

6.1.1.1b.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.1.1

6.1.1.1b.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.1.2

6.1.1.1b.3	Test description

6.1.1.1b.3.1	Pre-test conditions

System Simulator:

-	Four intra-frequency multi-PLMN cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.1b.3.1-1.

Table 6.1.1.1b.3.1–1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 2 | PLMN1 | 001 | 11 |
| 4 | PLMN2 | 001 | 21 |
| 11 | PLMN3 | 001 | 31 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.1b.3.1-2.

Table 6.1.1.1b.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 3 | Default PLMN3 PLMN2 Remaining mandatory entries use default values | Default All specified E-UTRAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) 1 (2 hours) | E-UTRAN Category M1 of E-UTRAN enhanced-MTC |

Preamble:

-	The UE is made to camp on Cell 2 and then Switched OFF (State 1).

6.1.1.1b.3.2	Test procedure sequence

Table 6.1.1.1b.3.2 – 1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.1b.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.1b.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | “Off” | -85 | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | -85 | -79 | -85 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | -79 | -85 | -85 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” | -85 | -79 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T4 | RS EPRE | dBm/15kHz | “Off” | -85 | -85 | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.1b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.1b.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 2. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 19A | SS adjusts cell levels according to row T2 of table 6.1.1.1b.3.2-1 | - | - | - | - |
| 20 | Check: Does the UE send an RRCConnectionRequest on Cell 1 after 120 seconds, but before 660 / 7500 (for Category M1) seconds (Note 2) from power on? | --> | RRCConnectionRequest | 4 | P |
| 21-25 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 26 | Void | - | - | - | - |
| - | EXCEPTION: Step 26Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 26Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 26B | Check: Is PLMN4 indicated by the UE? | - | - | 4 | P |
| 27 | Void | - | - | - | - |
| 28 | SS adjusts cell levels according to row T3 of table 6.1.1.1b.3.2-1 | - | - | - | - |
| 29 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 4? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | - |
| - | EXCEPTION: Step 29Aa1 describe optional behaviour that depend on the UE capability | - | - | - | - |
| 29Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 30 | SS adjusts cell levels according to row T4 of table 6.1.1.1b.3.2-1 | - | - | - | - |
| 31 | Check: Is PLMN2 indicated by the UE? | - | - | 3 | P |
| 32 | Check: Does the UE send an RRCConnectionRequest on Cell 11 after 120 seconds, but before 660 / 7500 (for Category M1) seconds after step 29? (Note 1 and 2) | --> | RRCConnectionRequest | 2 | P |
| 33-37 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 38 | Check: Is PLMN3 indicated by the UE? | - | - | 2 | P |
| 39 | Void | - | - | - | - |
| Note 1: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-12). Hence, window between 120s to T+Tolerance is being used, where the high priority PLMN search timer T defined by EFHPPLMN Note 2: Tolerance of 5min is added to allow time for the UE to find the proper PLMN |  |  |  |  |  |

6.1.1.1b.3.3	Specific message contents

Table 6.1.1.1b.3.3-1: SystemInformationBlockType3 for cells 2 and 4 (preamble and all steps, table 6.1.1.1b.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst |  | dB24 |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.1.2	PLMN selection of "Other PLMN/access technology combinations" / Automatic mode

6.1.1.2.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and EHPLMN, UPLMN and OPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the EHPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and UPLMN, OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the UPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(3)

with { UE in Automatic network selection mode and OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the OPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(4)

with { UE in Automatic network selection mode and other PLMN/access technology combinations cells not belonging to any of EHPLMN, UPLMN or OPLMN available }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of other PLMN/access technology combinations and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1.1.

[TS 23.122, clause 4.4.3.1.1]

4.4.3.1.1	Automatic Network Selection Mode Procedure

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present);

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

When following the above procedure the following requirements apply:

a)	An MS with voice capability shall ignore PLMNs for which the MS has identified at least one GSM COMPACT.

b)	In A/Gb mode or GSM COMPACT, an MS with voice capability, or an MS not supporting packet services shall not search for CPBCCH carriers.

c)	In ii and iii, the MS should limit its search for the PLMN to the access technology or access technologies associated with the PLMN in the appropriate PLMN Selector with Access Technology list (User Controlled or Operator Controlled selector list).

An MS using a SIM without access technology information storage (i.e. the "User Controlled PLMN Selector with Access Technology" and the "Operator Controlled PLMN Selector with Access Technology" data files are not present) shall instead use the "PLMN Selector" data file, for each PLMN in the "PLMN Selector" data file, the MS shall search for all access technologies it is capable of. The priority ordering amongst the access technologies is implementation dependent.

d)	In iv and v, the MS shall search for all access technologies it is capable of, before deciding which PLMN to select.

e)	In ii, and iii, a packet only MS which supports GSM COMPACT, but using a SIM without access technology information storage (i.e. the "User Controlled PLMN Selector with Access Technology" and the "Operator Controlled PLMN Selector with Access Technology" data files are not present) shall instead use the "PLMN Selector" data file, for each PLMN in the "PLMN Selector" data file, the MS shall search for all access technologies it is capable of and shall assume GSM COMPACT access technology as the lowest priority radio access technology.

f)	In i, the MS shall search for all access technologies it is capable of. No priority is defined for the preferred access technology and the priority is an implementation issue, but "HPLMN Selector with Access Technology" data file on the SIM may be used to optimise the procedure.

g)	In i, an MS using a SIM without access technology information storage (i.e. the "HPLMN Selector with Access Technology" data file is not present) shall search for all access technologies it is capable of. The priority ordering amongst the access technologies is implementation dependent. A packet only MS which supports GSM COMPACT using a SIM without access technology information storage shall also assume GSM COMPACT access technology as the lowest priority radio access technology.

h)	In v, the MS shall order the PLMN/access technology combinations in order of decreasing signal quality within each access technology. The order between PLMN/access technology combinations with different access technologies is an MS implementation issue.

NOTE 1:	Requirements a) and b) apply also to requirement d), so a GSM voice capable MS should not search for GSM COMPACT PLMNs, even if capable of GSM COMPACT.

NOTE 2:	Requirements a) and b) apply also to requirement f), so a GSM voice capable MS should not search for GSM COMPACT PLMNs, even if this is the only access technology on the "HPLMN Selector with Access Technology" data file on the SIM.

NOTE 3:	High quality signal is defined in the appropriate AS specification.

If successful registration is achieved, the MS indicates the selected PLMN.

If registration cannot be achieved because no PLMNs are available and allowable, the MS indicates "no service" to the user, waits until a new PLMN is available and allowable and then repeats the procedure.

If there were one or more PLMNs which were available and allowable, but an LR failure made registration on those PLMNs unsuccessful or an entry in any of the lists "forbidden LAs for roaming", "forbidden TAs for roaming", "forbidden LAs for regional provision of service" or "forbidden TAs for regional provision of service" prevented a registration attempt, the MS selects the first such PLMN again and enters a limited service state.

6.1.1.2.3	Test description

6.1.1.2.3.1	Pre-test conditions

System Simulator:

-	Four inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default PLMNs as indicated in TS 36.508 Table 4.4.2-2.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.2.3.1-1.

Table 6.1.1.2.3.1–1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |
| 13 | PLMN3 |
| 14 | PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.2.3.1-2.

Table 6.1.1.2.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN3 Remaining defined entries use default values | E-UTRAN |
| EFUST |  | Services 20, 42 and 71 are supported. |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.2.3.2	Test procedure sequence

Table 6.1.1.2.3.2 – 1 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” “T3” “T4”etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.2.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.2.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Cell 14 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | -85 | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” | ”Off” | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T4 | RS EPRE | dBm/15kHz | “Off” | ”Off” | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.2.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | SS adjusts cell levels according to row T2 of table 6.1.1.2.3.2-1 | - | - | - | - |
| 21 | Check: Does the UE send an RRCConnectionRequest on Cell 12? | --> | RRCConnectionRequest | 2 | P |
| 22-26 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 12. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 27 | Void | - | - | - | - |
| - | EXCEPTION: Step 27Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 27Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.. | - | - | - | - |
| 27B | Check: Is PLMN2 indicated by the UE? | - | - | 2 | P |
| 28 | SS adjusts cell levels according to row T3 of table 6.1.1.2.3.2-1 | - | - | - | - |
| 29 | Check: Does the UE send an RRCConnectionRequest on Cell 13? | --> | RRCConnectionRequest | 3 | P |
| 30-34 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 13. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 35 | Check: Is PLMN3 indicated by the UE? | - | - | 3 | P |
| 36 | SS adjusts cell levels according to row T4 of table 6.1.1.2.3.2-1 | - | - | - | - |
| 37 | Check: Does the UE send an RRCConnectionRequest on Cell 14? | --> | RRCConnectionRequest | 4 | P |
| 38-42 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 14. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 43 | Check: Is PLMN4 indicated by the UE? | - | - | 4 | P |

6.1.1.2.3.3	Specific message contents

None

#### 6.1.1.2a	PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation

6.1.1.2a.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and EHPLMN and UPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the EHPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and UPLMN and OPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the UPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(3)

with { UE in Automatic network selection mode and OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the OPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(4)

with { UE in Automatic network selection mode and other PLMN/access technology combinations cells not belonging to any of EHPLMN, UPLMN or OPLMN available }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of other PLMN/access technology combinations and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.2a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.2.2

6.1.1.2a.3	Test description

6.1.1.2a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency multi-PLMN cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.2a.3.1-1.

Table 6.1.1.2a.3.1–1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 11 | PLMN2 |
| 4 | PLMN3 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.2a.3.1-2.

Table 6.1.1.2a.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN3 Remaining defined entries use default values | E-UTRAN |
| EFUST |  | Services 20, 42 and 71 are supported. |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.2a.3.2	Test procedure sequence

Table 6.1.1.2a.3.2 – 1 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” “T3” “T4”etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.2a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.2a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -79 | "Off" | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” | -79 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T4 | RS EPRE | dBm/15kHz | “Off” | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T5 | RS EPRE | dBm/15kHz | -79 | "Off" | "Off" | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.2a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | SS adjusts cell levels according to row T2 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 21 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 2 | P |
| 22-26 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 27 | Void | - | - | - | - |
| - | EXCEPTION: Step 27AAa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 27AAa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 27Aa1 | Void | - | - | - | - |
| 27AB | Check: Is PLMN2 indicated by the UE? | - | - | 2 | P |
| 27A | SS adjusts cell levels according to row T5 of table 6.1.1.2a.3.2-1 and change PLMN identifiers of Cell 11 to PLMN4 | - | - | - | - |
| 27B | The generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. |  |  |  |  |
| - | EXCEPTION: Step 27Ca1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 27Ca1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 28 | SS adjusts cell levels according to row T3 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 29 | Check: Does the UE send an RRCConnectionRequest on Cell 4? | --> | RRCConnectionRequest | 3 | P |
| 30-34 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 34Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 34Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 35 | Check: Is PLMN3 indicated by the UE? | - | - | 3 | P |
| 36 | SS adjusts cell levels according to row T4 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 37 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 38-42 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 43 | Check: Is PLMN4 indicated by the UE? | - | - | 4 | P |

6.1.1.2a.3.3	Specific message contents

None

#### 6.1.1.3	Cell reselection of ePLMN in manual mode

6.1.1.3.1	Test Purpose (TP)

(1)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Attach procedure }

ensure that {

when { higher ranked cell is a cell of a PLMN in the downloaded equivalent PLMN list }

then { UE reselects to the equivalent PLMN cell. }

}

(2)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Tracking Area Update procedure }

ensure that {

when { highest ranked cell is a cell of a PLMN not in the downloaded equivalent PLMN list }

then { UE does not reselect to the cell. }

}

6.1.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clause 4.4.3.1.2.

[TS 23.122, clause 4.4.3.1.2]

…

Once the UE has registered on a PLMN selected by the user, the UE shall not automatically register on a different PLMN unless:

i)	the new PLMN is declared as an equivalent PLMN by the registered PLMN; or

ii)	the user selects automatic mode.

…

6.1.1.3.3	Test description

6.1.1.3.3.1	Pre-test conditions

System Simulator

-	Three inter-frequency multi-PLMN cells.

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 6.1.1.3.3.1-1.

Table 6.1.1.3.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |
| 13 | PLMN3 |

- System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE

-	The UE is in Manual PLMN selection mode.

-

Preamble

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 6.1.1.3.3.3-2.

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to TS 36.508[18];

6.1.1.3.3.2	Test procedure sequence

Table 6.1.1.3.3.2-1: Time instances of cell power level changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -97 | -82 | -67 |  |
| T2 | RS EPRE | dBm/15kHz | -85 | -97 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.3.3.2-1 | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 12? NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | 1,2 | P |
| 3 | Check: Is PLMN2 indicated by the UE? | - | - | 1,2 | P |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 and 13 within 60s? | - | - | 2 | F |
| 5 | Set UE to Automatic PLMN selection mode. | - | - | - | - |
| 6 | SS adjusts cell levels according to row T2 of table 6.1.1.3.3.2-1 | - | - | - | - |
| 7 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| Note 1: In Step 4 no further cell reselections are expected. Note 2: Cell 13 is the highest ranked cell, but never appears in the ePLMN list. Note 3: Steps 5 - 7 are to ensure UE is set back to automatic PLMN selection mode for the next test case. |  |  |  |  |  |

6.1.1.3.3.3	Specific message contents

Table 6.1.1.3.3.3-0: Conditions for specific message contents
in Tables 6.1.1.3.3.3-1 and 6.1.1.3.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.1.3.3.3-1: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.1.3.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[2] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.3.3.3-2: SystemInformationBlockType5 for Cell 12 (preamble and all steps, Table 6.1.1.3.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[2] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.3.3.3-3: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  | Cell 1 |

Table 6.1.1.3.3.3-4: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 2, Table 6.1.1.3.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  | Cell 12 |

#### 6.1.1.3a	Cell reselection of ePLMN in manual mode / between FDD and TDD

6.1.1.3a.1	Test Purpose (TP)

(1)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Attach procedure }

ensure that {

when { higher ranked cell is a cell of a PLMN in the downloaded equivalent PLMN list }

then { UE reselects to the equivalent PLMN cell. }

}

(2)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Tracking Area Update procedure }

ensure that {

when { highest ranked cell is a cell of a PLMN not in the downloaded equivalent PLMN list }

then { UE does not reselect to the cell. }

}

6.1.1.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clause 4.4.3.1.2.

[TS 23.122, clause 4.4.3.1.2]

…

Once the UE has registered on a PLMN selected by the user, the UE shall not automatically register on a different PLMN unless:

i)	the new PLMN is declared as an equivalent PLMN by the registered PLMN; or

ii)	the user selects automatic mode.

…

6.1.1.3a.3	Test description

6.1.1.3a.3.1	Pre-test conditions

System Simulator

-	Four multi-PLMN cells. Cell 1 and Cell 2 are set to LTE-FDD mode while Cell 28 and Cell 29 are set to LTE-TDD mode.

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 6.1.1.3a.3.1-1. Note Cell 2 is in a different PLMN to Cell 1.

Table 6.1.1.3a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 28 | PLMN2 |
| 2 | PLMN3 |
| 29 | PLMN3 |

- System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE

-	The UE is in Manual PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 6.1.1.3a.3.1-2.

Table 6.1.1.3a.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See pre-amble) |  |

Preamble

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 6.1.1.3a.3.3-1.

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to TS 36.508[18];

6.1.1.3a.3.2	Test procedure sequence

Table 6.1.1.3a.3.2-1: Time instances of cell power level changes

|  | Parameter | Unit | Cell 1 | Cell 28 | Cell 2 | Cell 29 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -97 | -82 | -67 | OFF |  |
| T2 | RS EPRE | dBm/15kHz | -82 | -97 | OFF | -67 |  |
| T3 | RS EPRE | dBm/15kHz | -97 | -82 | OFF | OFF |  |
| T4 | RS EPRE | dBm/15kHz | -82 | -97 | OFF | OFF |  |

Table 6.1.1.3a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.3a.3.2-1 | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 28? NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | 1,2 | P |
| 3 | Check: Is PLMN2 indicated by the UE? | - | - | 1,2 | P |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 60s? | - | - | 2 | F |
| 5 | SS adjusts cell levels according to row T2 of table 6.1.1.3a.3.2-1 | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The TAU is accepted with PLMN2 listed as an Equivalent PLMN | - | - | 1,2 | P |
| 7 | Check: Does the UE send an RRCConnectionRequest on Cell 29 within 60s? | - | - | 2 | F |
| 8 | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 1. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. |  |  |  |  |
| 9 | SS adjusts cell levels according to row T3 of table 6.1.1.3a.3.2-1 | - |  | - | - |
| 10 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra CarrierFreq of Cell 28). | <-- | RRCConnectionRelease |  |  |
| 11 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on cell 28. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | - | - | - | - |
| 12 | Set UE to Automatic PLMN selection mode. | - | - | - | - |
| 13 | SS adjusts cell levels according to row T4 of table 6.1.1.3a.3.2-1 | - | - | - | - |
| 14 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on cell 1. | - | - | - | - |
| Note 1: In Step 4 no further cell reselections are expected. Note 2: Cell 3 and Cell 29 are the highest ranked cell, but never appears in the ePLMN list. Note 3: Steps 8 -14 are to ensure UE is set back to automatic PLMN selection mode for the next test case. |  |  |  |  |  |

6.1.1.3a.3.3	Specific message contents

Table 6.1.1.3a.3.3-0: Conditions for specific message contents
in Tables 6.1.1.3a.3.3-1 and 6.1.1.3a.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.1.3a.3.3-1: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.1.3a.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 28 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 28 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.3a.3.3-2: SystemInformationBlockType5 for Cell 28 (preamble and all steps, Table 6.1.1.3a.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.3a.3.3-3: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 6.1.1.3a.3.3-4: TRACKING AREA UPDATE ACCEPT for Cell 28 (step 2, Table 6.1.1.3a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 6.1.1.3a.3.3-5: TRACKING AREA UPDATE ACCEPT for Cell 1 (step 6, Table 6.1.1.3a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 6.1.1.3a.3.3-6: RRCConnectionRelease message (step 10, Table 6.1.1.3a.3.2-2)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 28 |  |  |
| } |  |  |  |
| redirectedCarrierInfo | Not present |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 28 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.1.3b	Cell reselection of ePLMN in manual mode / Single Frequency operation

6.1.1.3b.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.3.1

6.1.1.3b.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.3.2

6.1.1.3b.3	Test description

6.1.1.3b.3.1	Pre-test conditions

System Simulator

-	Two intra-frequency multi-PLMN cells.

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 6.1.1.3b.3.1-1.

Table 6.1.1.3b.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 2 | PLMN2 |

UE

-	The UE is in Manual PLMN selection mode.

Preamble

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 6.1.1.3b.3.3-2.

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to TS 36.508[18];

6.1.1.3b.3.2	Test procedure sequence

Table 6.1.1.3b.3.2-1: Time instances of cell power level changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -88 | -82 |  |
| T2 | RS EPRE | dBm/15kHz | -76 | -82 |  |

Table 6.1.1.3b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.3b.3.2-1 | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The TAU is accepted with PLMN3 listed as an Equivalent PLMN. Note 2. | - | - | 1,2 | P |
| 3 | Void | - | - |  |  |
| - | EXCE-TION: Step 3AAa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 3AAa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 3AB | Check: Is PLMN2 indicated by the UE? | - | - | 1,2 | P |
| 3A | SS adjusts cell levels according to row T2 of table 6.1.1.3b.3.2-1 | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within 60s? | - | - | 2 | F |
| Note 1: In Step 4 no further cell reselections are expected. Note 2: PLMN3 is dummy PLMN, which is not associated on any cell, to able to satisfy test purpose. |  |  |  |  |  |

6.1.1.3b.3.3	Specific message contents

Table 6.1.1.3b.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  | Cell 1 |

Table 6.1.1.3b.3.3-2: TRACKING AREA UPDATE ACCEPT for Cell 2 (step 2, Table 6.1.1.3b.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN3 |  | Cell 2 |

#### 6.1.1.4	PLMN selection in shared network environment / Automatic mode

6.1.1.4.1	Test Purpose (TP)

(1)

with { the UE is in automatic network selection mode and there is a suitable cell with multiple PLMN identities among which the HPLMN but not the registered PLMN }

ensure that {

when { the UE is switched on }

then { the UE attaches to the HPLMN on the shared cell  }

}

(2)

with { the UE in automatic network selection mode and there is a suitable cell with multiple PLMN identities among which the registered PLMN }

ensure that {

when { the UE returns to coverage }

then { the UE performs a location registration to the registered PLMN on the shared cell }

}

6.1.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.1.2.2, TS 23.122 clauses 4.4.3 and 4.4.3.1.1, TS 24.301 clauses 5.5.1.1 and 5.5.3.1, and TS 36.331 clause 5.3.3.4.

[TS 36.304 clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value),

…

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 23.122 clause 4.4.3]

When the MS reselects to a cell in a shared network, the AS may indicate multiple PLMN identities to the NAS according to 3GPP TS 25.304 [32]. The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

[TS 23.122 clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

[TS 24.301 clause 5.5.1.1]

...

In a shared network, the UE shall choose one of the PLMN identities as specified in 3GPP TS 23.122 [6]. The UE shall construct the TAI of the cell from this chosen PLMN identity and the TAC received as part of the broadcast system information. The chosen PLMN identity shall be indicated to the E-UTRAN (see 3GPP TS 36.331 [22]).

...

[TS 24.301 clause 5.5.3.1]

...

In a shared network, the UE shall choose one of the PLMN identities as specified in 3GPP TS 23.122 [6]. The UE shall construct the TAI of the cell from this chosen PLMN identity and the TAC received on the broadcast system information. The chosen PLMN identity shall be indicated to the E-UTRAN (see 3GPP TS 36.331 [22]).

...

[TS 36.331 clause 5.3.3.4]

The UE shall:

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

...

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

6.1.1.4.3	Test description

6.1.1.4.3.1	Pre-test conditions

System Simulator:

-	Cells 1 and 2, as specified in  TS 36.508 clause 4.4.1.2 are configured according to Table 4.4.2-2 in [18] except for multiple broadcasted PLMN identities as shown in Table 6.1.1.4.3.1–1: PLMN identifiers broadcasted by cells in shared network.

Table 6.1.1.4.3.1-1: PLMN identifiers

| Cell | PLMN names |
| --- | --- |
| 1 | PLMN15, PLMN1 |
| 2 | PLMN15, PLMN1, PLMN16 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.4.3.1-2.

- 	The UE is registered to PLMN4 before it is switched off.

Table 6.1.1.4.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service 71 is not supported (Note). Service 74 is supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN |  | 0xFF..FF (Note) |  |
| Note: Either EFUST or EFEHPLMN or both can be set to the specified value in Table 6.1.1.4.3.1–2 |  |  |  |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.1.4.3.2	Test procedure sequence

Table 6.1.1.4.3.2–1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configuration marked “T1”is applied at the points indicated in the Main behaviour description in Table 6.1.1.4.3.2-2. Cell powers are chosen for a serving cell and a non-suitable cell as defined in  TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.4.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | Off | The power level values are assigned to such that camping on Cell 1 is guaranteed. Cell 2 is “off”. |
| T1 | RS EPRE | dBm/15kHz | Off | -85 | Cell 1 becomes “off”. UE shall select Cell 2. |

Table 6.1.1.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T0 of table 6.1.1.4.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE transmits an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup | <-- | RRCConnectionSetup | - | - |
| 5 | Check; Does the UE transmit an RRCConnectionSetupComplete message indicating the HPLMN (second PLMN in the list)? Note: this message contains an ATTACH REQUEST and a PDN CONNECTIVITY REQUEST message according to default message contents. | --> | RRCConnectionSetupComplete | 1 | P |
| 6 -18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | The SS adjusts cell levels according to row T1 of table 6.1.1.4.3.2-1 | - | - | - | - |
| 21 | Check: Does the UE transmit an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 2 | P |
| 22 - 26 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 27 | Check: Is the same PLMN as indicated by UE in step 19 again indicated by the UE? | - | - | 2 | P |

6.1.1.4.3.3	Specific message contents

Table 6.1.1.4.3.3-1: RRCConnectionSetupComplete (step 5 and 23, Table 6.1.1.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 2 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.1.4a	PLMN selection in shared network environment / Automatic mode / Between FDD and TDD

6.1.1.4a.1	Test Purpose (TP)

(1)

with { the UE is in automatic network selection mode and there is a suitable cell with multiple PLMN identities among which the EHPLMN list is present }

ensure that {

when { the UE returns to coverage }

then { the UE performs a location registration to the highest priority EHPLMN that is available with FDD mode on the shared cell  }

}

(2)

with { the UE in automatic network selection mode and there is a suitable cell with multiple PLMN identities among which the EHPLMN list is present }

ensure that {

when { the UE is powered on }

then { the UE performs a location registration to the highest priority EHPLMN that is available with TDD mode on the shared cell }

}

6.1.1.4a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.1.2.2, TS 23.122 clauses 4.4.3 and 4.4.3.1.1, TS 24.301 clauses 5.5.1.1 and 5.5.3.1, and TS 36.331 clause 5.3.3.4.

[TS 36.304 clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value),

…

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 23.122 clause 4.4.3]

When the MS reselects to a cell in a shared network, the AS may indicate multiple PLMN identities to the NAS according to 3GPP TS 25.304 [32]. The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

[TS 23.122 clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

[TS 24.301 clause 5.5.1.1]

...

In a shared network, the UE shall choose one of the PLMN identities as specified in 3GPP TS 23.122 [6]. The UE shall construct the TAI of the cell from this chosen PLMN identity and the TAC received as part of the broadcast system information. The chosen PLMN identity shall be indicated to the E-UTRAN (see 3GPP TS 36.331 [22]).

...

[TS 24.301 clause 5.5.3.1]

...

In a shared network, the UE shall choose one of the PLMN identities as specified in 3GPP TS 23.122 [6]. The UE shall construct the TAI of the cell from this chosen PLMN identity and the TAC received on the broadcast system information. The chosen PLMN identity shall be indicated to the E-UTRAN (see 3GPP TS 36.331 [22]).

...

[TS 36.331 clause 5.3.3.4]

The UE shall:

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

...

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

6.1.1.4a.3	Test description

6.1.1.4a.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2, Cell 28 and Cell 30 as specified in TS 36.508 clause 4.4.1.3 are configured according to Table 4.4.2-2 in [18] except for multiple broadcasted PLMN identities as shown in Table 6.1.1.4a.3.1–1.

Table 6.1.1.4a.3.1-1: PLMN identifiers

| Cell | PLMN names | MODE |
| --- | --- | --- |
| 1 | PLMN2,PLMN15 | FDD |
| 28 | PLMN4,PLMN16 | TDD |
| 30 | PLMN3,PLMN17 | TDD |
| 2 | PLMN1,PLMN18 | FDD |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.4a.3.1-2.

Table 6.1.1.4a.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFUST |  | Service 71 and 74 are supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN | 1 2 3 | PLMN2 PLMN1 PLMN4 Remaining mandatory entries use default values |  |

Preamble:

-	The UE is registered on PLMN4 (Cell28) using the procedure described in TS 36.508 clause 4.5.2.

The UE is in state Registered, Idle mode (state 2) according to [18].

6.1.1.4a.3.2	Test procedure sequence

Table 6.1.1.4a.3.2–1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configuration marked “T1”is applied at the points indicated in the Main behaviour description in Table 6.1.1.4a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.4a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 28 | Cell 30 | Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | ”Off” | -85 | ”Off” | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | -79 | ”Off” | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | ”Off” | -85 | -73 | ”Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.4a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 6.1.1.4a.3.2-1 | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 3 – 7 | Steps 3 to 7 of the generic test procedure in TS 36.508 subclause 4.5A.2.1-1 are performed on Cell 1. NOTE: The UE performs TAU procedure and the RRC connection is released. | - | - | - | - |
| 8 – 17 | VOID | - | - | - | - |
| 18 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 19 | Power off the UE. |  |  |  |  |
| 20 | The SS adjusts cell levels according to row T2 of table 6.1.1.4a.3.2-1 | - | - | - | - |
| 21 | Power on the UE. |  |  |  |  |
| 22 | Check: Does the UE transmit an RRCConnectionRequest on Cell 28? | --> | RRCConnectionRequest | 2 | P |
| 23 – 37 | Steps 3 to 17 of the generic test procedure in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 38 | Check: Is PLMN4 indicated by the UE? | - | - | 2 | P |

6.1.1.4a.3.3	Specific message contents

Table 6.1.1.4a.3.3-1: RRCConnectionSetupComplete (step 4, Table 6.1.1.4a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 | PLMN2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.1.5	Void

#### 6.1.1.6	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection

6.1.1.6.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode registered to UPLMN and RPLMN, UPLMN and OPLMN E-UTRAN cells available }

ensure that {
  when { UE is requested to initiate reselection and registration onto an available PLMN }

then { UE reselects to the cell which belongs to higher priority OPLMN }

}

(2)

with { UE in Automatic network selection mode registered to OPLMN and only RPLMN E-UTRAN cell available }

ensure that {
  when { UE is requested to initiate reselection and registration onto an available PLMN }

then { UE remains on the current cell which belongs to RPLMN. }

}

(3)

with { UE in Automatic network selection mode registered to OPLMN and RPLMN, UPLMN and OPLMN E-UTRAN cells available }

ensure that {
  when { UE is requested to initiate reselection and registration onto an available PLMN }

then { UE reselects to the cell which belongs to UPLMN }

}

(4)

with { UE in Automatic network selection mode registered to UPLMN and RPLMN, UPLMN, OPLMN and HPLMN E-UTRAN cells available }

ensure that {
  when { UE is requested to initiate reselection and registration onto an available PLMN }

then { UE reselects to the cell which belongs to HPLMN }

}

6.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.2 and 4.4.3.2.1.

[TS 23.122, clause 4.4.3.2]

At any time the user may request the MS to initiate reselection and registration onto an available PLMN, according to the following procedures, dependent upon the operating mode.

[TS 23.122, clause 4.4.3.2.1]

The MS selects and attempts registration on PLMNs, if available and allowable, in all of its bands of operation in accordance with the following order:

i)	the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present);

ii)	PLMN/access technology combinations contained in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) excluding the previously selected PLMN/access technology combination;

iii)	PLMN/access technology combinations contained in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) excluding the previously selected PLMN/access technology combination;

iv)	other PLMN/access technology combinations with the received high quality signal in random order excluding the previously selected PLMN/access technology combination;

v)	other PLMN/access technology combinations, excluding the previously selected PLMN/access technology combination in order of decreasing signal quality or, alternatively, the previously selected PLMN/access technology combination may be chosen ignoring its signal quality;

vi)	The previously selected PLMN/access technology combination.

The previously selected PLMN/access technology combination is the PLMN/access technology combination which the MS has selected prior to the start of the user reselection procedure.

NOTE 1:	 If the previously selected PLMN is chosen, and registration has not been attempted on any other PLMNs, then the MS is already registered on the PLMN, and so registration is not necessary.

The equivalent PLMNs list shall not be applied to the user reselection in Automatic Network Selection Mode.

6.1.1.6.3	Test description

6.1.1.6.3.1	Pre-test conditions

System Simulator:

-	Four inter-frequency multi-PLMN cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.6.3.1-1.

Table 6.1.1.6.3.1-1: PLMN identifiers

| - Cell | - PLMN name |
| --- | --- |
| - 1 | - PLMN1 |
| - 12 | - PLMN2 |
| - 13 | - PLMN3 |
| - 14 | - PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.6.3.1-2.

Table 6.1.1.6.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 2 | PLMN2 PLMN4 Remaining defined entries use default values | E-UTRAN E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN3 | E-UTRAN |
| EFUST |  | Service 71 is not supported |  |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.1.6.3.2	Test procedure sequence

Table 6.1.1.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Cell 14 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | "Off" | -85 | The power level values are assigned to satisfy RCell 1 = RCell 12 = RCell 14. (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" | "Off" | Only Cell 12 is available. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -85 | -85 | The power level values are assigned to satisfy RCell 1 = RCell 13 = RCell 14. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 6.1.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row “T1” in table 6.1.1.6.3.2-1 | - | - | - | - |
| 2 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 12? | --> | RRCConnectionRequest | 1 | P |
| 4-8 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 12. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 9 | Check: Is PLMN 2 indicated by the UE? | - | - | 1 | P |
| 10 | The SS changes the power level setting according to the row “T2” in table 6.1.1.6.3.2-1. | - | - | - | - |
| 11 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 12 | Check: Does the UE send an RRCConnectionRequest on Cell 12 within 90 s? | --> | RRCConnectionRequest | 2 | F |
| 13 | Check: Is PLMN 2 indicated by the UE? | - | - | 2 | P |
| 14 | The SS changes the power level setting according to the row “T1” in table 6.1.1.6.3.2-1. | - | - | - | - |
| 15 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 16 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 3 | P |
| 17-21 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22 | Check: Is PLMN 1 indicated by the UE? | - | - | 3 | P |
| 23 | The SS changes the power level setting according to the row “T3” in table 6.1.1.6.3.2-1. | - | - | - | - |
| 24 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 25 | Check: Does the UE send an RRCConnectionRequest on Cell 13? | --> | RRCConnectionRequest | 4 | P |
| 26-30 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 13. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 31 | Void | - | - | - |  |
| - | EXCEPTION: Step 32a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 32a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 33 | Check: Is PLMN 3 indicated by the UE? | - | - | 4 | P |
| Note: MMI command “TRIGGER_USER_RESELECTION” to be used. AT command AT+COPS is not suitable to achieve the test purpose. |  |  |  |  |  |

6.1.1.6.3.3	Specific message contents

None

#### 6.1.1.6a	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation

6.1.1.6a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.6.1

6.1.1.6a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.6.2

6.1.1.6a.3	Test description

6.1.1.6a.3.1	Pre-test conditions

System Simulator:

-	Four intra-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.6a.3.1-1.

Table 6.1.1.6a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 2 | PLMN2 |
| 4 | PLMN3 |
| 11 | PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.6a.3.1-2.

Table 6.1.1.6a.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 2 | PLMN2 PLMN3 Remaining defined entries use default values | E-UTRAN E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Service 71 is not supported |  |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.1.6a.3.2	Test procedure sequence

Table 6.1.1.6a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -85 | "Off" | (NOTE 1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" | "Off" | Only Cell 2 is available. (NOTE 1) |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -85 | -85 | "Off" | (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -85 | -79 | (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 6.1.1.6a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row “T1” in table 6.1.1.6a.3.2-1 | - | - | - | - |
| 2 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 4-8 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 9 | Check: Is PLMN 2 indicated by the UE? | - | - | 1 | P |
| 10 | The SS changes the power level setting according to the row “T2” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 11 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 12 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 90 s? | --> | RRCConnectionRequest | 2 | F |
| 13 | Check: Is PLMN 2 indicated by the UE? | - | - | 2 | P |
| 14 | The SS changes the power level setting according to the row “T3” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 15 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 16 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 3 | P |
| 17-21 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22 | Check: Is PLMN 1 indicated by the UE? | - | - | 3 | P |
| 23 | The SS changes the power level setting according to the row “T4” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 24 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 25 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 26-30 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 31 | Void | - | - | - | - |
| - | EXCEPTION: Step 32a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 32a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 33 | Check: Is PLMN 4 indicated by the UE? | - | - | 4 | P |
| Note: MMI command “TRIGGER_USER_RESELECTION” to be used. AT command AT+COPS is not suitable to achieve the test purpose.. |  |  |  |  |  |

6.1.1.6a.3.3	Specific message contents

Table 6.1.1.6a.3.3-1: SystemInformationBlockType3 for cells 1 and 2 (preamble and all steps, table 6.1.1.6a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst |  | dB24 |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.1.6b	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation

6.1.1.6b.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.6a.1.

6.1.1.6b.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.6a.2.

6.1.1.6b.3	Test description

6.1.1.6b.3.1	Pre-test conditions

System Simulator:

Same with Pre-test conditions for system simulator in 6.1.1.6a.2 except:

-	Four inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

-	Cell 2 is replaced by Cell 3,

-	Cell 4 is replaced by Cell 11,

-	Cell 11 is replaced by Cell 12.

UE:

Same with Pre-test conditions for UE in 6.1.1.6a.2.

Preamble:

Same with Pre-test conditions for Preamble in 6.1.1.6a.2.

6.1.1.6b.3.2	Test procedure sequence

Same with Test procedure in 6.1.1.6a.3.2 except:

-	Cell 2 is replaced by Cell 3,

-	Cell 4 is replaced by Cell 11,

-	Cell 11 is replaced by Cell 12.

6.1.1.6b.3.3	Specific message contents

Same with Specific message contents in 6.1.1.6a.3.3 except:

-	Cell 2 is replaced by Cell 3.

#### 6.1.1.7	PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer

6.1.1.7.1	Test Purpose (TP)

(1)

with { UE configured with “MinimumPeriodicSearchTimer“ and camped on an E-UTRAN VPLMN cell and cells of a higher priority E-UTRAN PLMN available }

ensure that {

when { the higher priority PLMN search timer T stored in the USIM  or the default value for T is less than the MinimumPeriodicSearchTimer }

then { T shall be set to the MinimumPeriodicSearchTimer. After the first attempt to the higher priority PLMN cell is made, the UE shall not use a value for T that is less than the MinimumPeriodicSearchTimer. When the T expires the UE selects and camps on a cell of the highest priority PLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1 and 4.4.3.3.1.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see clause 4.5.2) attempts to perform a Location Registration.

...

If successful registration is achieved, the MS indicates the selected PLMN.

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then this operating mode shall be used.

...

NOTE 1:	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.3.1]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value T minutes may be stored in the SIM, T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value is stored in the SIM, a default value of 60 minutes is used for T.

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode when the MS is roaming, and not while the MS is attached for emergency bearer services or has a PDN connection for emergency bearer services;

b)	After switch on a period of at least 2 minutes and at most T minutes shall elapse before the first attempt is made;

….

6.1.1.7.3	Test description

6.1.1.7.3.1	Pre-test conditions

System Simulator:

-	Three multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting PLMNs as indicated in Table 6.1.1.7.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.7.3.1-1.

Table 6.1.1.7.3.1-1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 7 minutes

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.7.3.1-2.

Table 6.1.1.7.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 | Default PLMN2 Remaining mandatory entries use default values | Default E-UTRAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to 7 minutes |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.7.3.2	Test procedure sequence

Table 6.1.1.7.3.2 – 1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” is applied at the point indicated in the Main behaviour description in Table 6.1.1.7.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in  TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.7.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | “Off” | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | RS EPRE | dBm/15kHz | “Off” | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | -85 | -85 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.7.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-18 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | UE selects PLMN1 | - | - | - | - |
| 20 | Check: Does the UE send an RRCConnectionRequest on Cell 13 after 120 seconds, but before 420 seconds from power on? | --> | RRCConnectionRequest | 1 | P |
| 21-25 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 13. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 26 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 27 | SS adjusts cell levels according to row T2 of table 6.1.1.7.3.2-1 | - | - | - | - |
| 28 | Check: Does the UE send an RRCConnectionRequest on Cell 1 after 420 seconds from step 20? | --> | RRCConnectionRequest | 1 | P |
| 29-33 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 34 | Check: Is PLMN4 indicated by the UE? | - | - | 1 | P |
| Note 1: Timers in Steps 20 and 28 are derived from the value defined by the MinimumPeriodicSearchTimer |  |  |  |  |  |

6.1.1.7.3.3	Specific message contents

None

#### 6.1.1.7a	PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer / Single Frequency operation

6.1.1.7a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.7.1

6.1.1.7a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.7.2

6.1.1.7a.3	Test description

6.1.1.7a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency multi-PLMN cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting PLMNs as indicated in TS 36.508 Table 4.4.2-2.

System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.7a.3.1-1.

Table 6.1.1.7a.3.1-1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 2 | PLMN1 | 001 | 11 |
| 4 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 before it is switched off.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 7 minutes

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.7a.3.1-2.

Table 6.1.1.7a.3.1-2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 | Default PLMN2 Remaining mandatory entries use default values | Default E-UTRAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to 7 minutes |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.7a.3.2	Test procedure sequence

Table 6.1.1.7a.3.2 – 1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” is applied at the point indicated in the Main behaviour description in Table 6.1.1.7a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in  TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.7a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | "Off" | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.7a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.7a.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-18 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 2. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | UE selects PLMN1 | - | - | - | - |
| 19A | SS adjusts cell levels according to row T2 of table 6.1.1.7a.3.2-1 |  |  |  |  |
| 20 | Check: Does the UE send an RRCConnectionRequest on Cell 4 after 120 seconds, but before 420 seconds from power on? | --> | RRCConnectionRequest | 1 | P |
| 21-25 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 26 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 27 | SS adjusts cell levels according to row T3 of table 6.1.1.7a.3.2-1 | - | - | - | - |
| 28 | Check: Does the UE send an RRCConnectionRequest on Cell 1 after 420 seconds from step 20? | --> | RRCConnectionRequest | 1 | P |
| 29-33 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 34 | Check: Is PLMN4 indicated by the UE? | - | - | 1 | P |
| Note 1: Timers in Steps 20 and 28 are derived from the value defined by the MinimumPeriodicSearchTimer |  |  |  |  |  |

6.1.1.7a.3.3	Specific message contents

None

#### 6.1.1.8	PLMN selection of RPLMN or (E)HPLMN; Automatic mode

6.1.1.8.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and RPLMN, EHPLMN and HPLMN cells available and UE is fitted with a USIM containing the EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or EHPLMN and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and RPLMN, HPLMN and VPLMN cells available and  UE is fitted with a USIM not containing or containing empty EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or HPLMN and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.1.2.2, TS 23.122 clauses 1.2, 4.4.3.1 and 4.4.3.1.1.

[TS 36.304 clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value),

…

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 23.122, clause 1.2]

Equivalent HPLMN list: To allow provision for multiple HPLMN codes, PLMN codes that are present within this list shall replace the HPLMN code derived from the IMSI for PLMN selection purposes. This list is stored on the USIM and is known as the EHPLMN list. The EHPLMN list may also contain the HPLMN code derived from the IMSI. If the HPLMN code derived from the IMSI is not present in the EHPLMN list then it shall be treated as a Visited PLMN for PLMN selection purposes.

…

[TS 23.122 clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see subclause 4.5.2) attempts to perform a Location Registration.

EXCEPTION: As an alternative option to this, if the MS is in automatic network selection mode and it finds coverage of an EHPLMN, the MS may register to that EHPLMN and not return to the registered PLMN or equivalent PLMN. If the EHPLMN list is not present or is empty, and the HPLMN is available, the MS may register on the HPLMN and not return to the registered PLMN or equivalent PLMN. The operator shall be able to control by SIM configuration whether an MS that supports this option is permitted to perform this alternative behaviour.

...

[TS 23.122 clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

...

6.1.1.8.3	Test description

6.1.1.8.3.1	Pre-test conditions

System Simulator:

-	Cells 1, 12, 13 and 14, as specified in TS 36.508 clause 4.4.1.2 are configured as shown in Table 6.1.1.8.3.1–1.

Table 6.1.1.8.3.1-1: PLMN identifiers

| Cell | PLMN names |
| --- | --- |
| 1 | PLMN4 |
| 12 | PLMN1 |
| 13 | PLMN2 |
| 14 | PLMN3 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN4 before switch off.

-	Two USIMs containing default values (as per TS 36.508) except for those listed in Table 6.1.1.8.3.1-2 and Table 6.1.1.8.3.1-3 will be used.

Table 6.1.1.8.3.1-2: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 2 | PLMN2 PLMN1 |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.1.1.8.3.1-3: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.1.8.3.2	Test procedure sequence

Table 6.1.1.8.3.2–1 shows the cell configurations used during the test. Subsequent configuration marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.1.1.8.3.2-2. Cell powers are chosen for a serving cell and a non-suitable cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.8.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Cell 14 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | -85 | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | -85 | -85 | “Off” | -85 |  |

Table 6.1.1.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 6.1.1.8.3.2-1. | - | - | - | - |
| 2 | Power on the UE with USIM A inserted. | - | - | - | - |
| 3 | Check: Does the UE transmits an RRCConnectionRequest on Cell 1 or Cell 13? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup. | <-- | RRCConnectionSetup | - | - |
| 5 | Check; Does the UE transmit an RRCConnectionSetupComplete message indicating the RPLMN (PLMN4) or EHPLMN (PLMN2)? Note: this message contains an ATTACH REQUEST and a PDN CONNECTIVITY REQUEST message according to default message contents. | --> | RRCConnectionSetupComplete | 1 | P |
| 6-18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1 or Cell 13. NOTE: The UE performs registration with valid stored security context (see preamble) and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN4 or PLMN 2 indicated by the UE? | - | - | 1 | P |
| 20 | If possible switch off is performed or USIM is removed, otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 20A1 to 20A4 describes behaviour that depends on the UE capability. | - | - | - | - |
| 20A1-20A4 | If pc_SwitchOnOff or pc_USIM_Removal then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 21 | The SS adjusts cell levels according to row T2 of table 6.1.1.8.3.2-1. | - | - | - | - |
| 22 | The UE is brought back to operation with USIM B inserted. | - | - | - | - |
| 23 | Check: Does the UE transmit an RRCConnectionRequest on Cell 1 or Cell 12? | --> | RRCConnectionRequest | 2 | P |
| 24-38 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1 or Cell 12. | - | - | - | - |
| 39 | Check: Is PLMN4 or PLMN 1 indicated by the UE? | - | - | 2 | P |

6.1.1.8.3.3	Specific message contents

None

#### 6.1.1.9	PLMN selection of RPLMN or (E)HPLMN; Manual mode

6.1.1.9.1	Test Purpose (TP)

(1)

with { UE in Manual network selection mode and EHPLMN and HPLMN cells available and (E)RPLMN cell is not available and UE is fitted with a USIM containing the EHPLMN list and the UE supports the exception to manual mode selection mode }

ensure that {

when { UE is switched on }

then { UE selects a cell of the highest priority EHPLMN and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Manual network selection mode and HPLMN and VPLMN cells available and (E)RPLMN cell is not available and  UE is fitted with a USIM not containing or containing empty EHPLMN list and the UE supports the exception to manual mode selection mode }

ensure that {

when { UE is switched on }

then { UE selects a cell of the HPLMN and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.1.2.2, TS 23.122 clauses 1.2, 4.4.3.1 and 4.4.3.1.2.

[TS 36.304 clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value),

…

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 23.122, clause 1.2]

Equivalent HPLMN list: To allow provision for multiple HPLMN codes, PLMN codes that are present within this list shall replace the HPLMN code derived from the IMSI for PLMN selection purposes. This list is stored on the USIM and is known as the EHPLMN list. The EHPLMN list may also contain the HPLMN code derived from the IMSI. If the HPLMN code derived from the IMSI is not present in the EHPLMN list then it shall be treated as a Visited PLMN for PLMN selection purposes.

…

[TS 23.122 clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see subclause 4.5.2) attempts to perform a Location Registration.

…

EXCEPTION: At switch on, if the MS is in manual mode and neither registered PLMN nor PLMN that is equivalent to it is available but EHPLMN is available, then instead of performing the manual network selection mode procedure of subclause 4.4.3.1.2 the MS may select and attempt registration on the highest priority EHPLMN. If the EHPLMN list is not available or is empty and the HPLMN is available, then the MS may select and attempt registration on the HPLMN. The MS shall remain in manual mode.

...

[TS 23.122 clause 4.4.3.1.2]

The MS indicates whether there are any PLMNs, which are available using all supported access technologies. This includes PLMNs in the "forbidden PLMNs" list, "forbidden PLMNs for GPRS service" list and PLMNs which only offer services not supported by the MS. An MS which supports GSM COMPACT shall also indicate GSM COMPACT PLMNs (which use PBCCH).

If displayed, PLMNs meeting the criteria above are presented in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or, if one or more of the EHPLMNs are available then based on an optional data field on the SIM either only the highest priority available EHPLMN is to be presented to the user or all available EHPLMNs are presented to the user in priority order. If the data field is not present on the SIM, then only the highest priority available EHPLMN is presented;

ii)	PLMN/access technology combinations contained in the " User Controlled PLMN Selector with Access Technology " data file in the SIM (in priority order);

iii)	PLMN/access technology combinations contained in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

...

6.1.1.9.3	Test description

6.1.1.9.3.1	Pre-test conditions

System Simulator:

-	Cells 1, 2 and 4, as specified in TS 36.508 clause 4.4.1.2 are configured as shown in Table 6.1.1.9.3.1–1.

Table 6.1.1.9.3.1-1: PLMN identifiers

| Cell | PLMN names |
| --- | --- |
| 1 | PLMN1 (during TC body); PLMN4 (during preamble) |
| 2 | PLMN2 |
| 4 | PLMN3 |

UE:

-	The UE is in Manual PLMN selection mode.

-	The UE is registered to PLMN4 before switch off.

-	Two USIMs containing default values (as per TS 36.508) except for those listed in Table 6.1.1.9.3.1-2 and Table 6.1.1.9.3.1-3 will be used.

Table 6.1.1.9.3.1-2: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 2 | PLMN2 PLMN1 |  |

Table 6.1.1.9.3.1-3: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.1.9.3.2	Test procedure sequence

Table 6.1.1.9.3.2–1 shows the cell configurations used during the test. Subsequent configuration marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.1.1.9.3.2-2. Cell powers are chosen for a serving cell and a non-suitable cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.9.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | -85 | “Off” | -85 |  |

Table 6.1.1.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 6.1.1.9.3.2-1. | - | - | - | - |
| 2 | Power on the UE with USIM A inserted. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup. | <-- | RRCConnectionSetup | - | - |
| 5 | Check; Does the UE transmit an RRCConnectionSetupComplete message indicating the EHPLMN (PLMN2)? Note: this message contains an ATTACH REQUEST and a PDN CONNECTIVITY REQUEST message according to default message contents. | --> | RRCConnectionSetupComplete | 1 | P |
| 6 - 18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 2. NOTE: The UE performs registration with valid stored security context (see preamble) and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN 2 indicated by the UE? | - | - | 1 | P |
| 20 | If possible switch off is performed or the USIM is removed, otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 20B1 to 20 B4 describe behaviour that depends on the UE capability. | - | - | - | - |
| 20A1-20A6 | Void | - | - | - | - |
| 20B1-20B4 | If pc_SwitchOnOff or pc_USIM_Removal then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 21 | The SS adjusts cell levels according to row T2 of table 6.1.1.9.3.2-1. | - | - | - | - |
| 22 | The UE is brought back to operation with USIM B inserted. | - | - | - | - |
| 23 | Check: Does the UE transmit an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 24 - 38 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| 39 | Check: Is PLMN 1 indicated by the UE? | - | - | 2 | P |

6.1.1.9.3.3	Specific message contents

None

#### 6.1.1.10	eMTC / NTN

6.1.1.10.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED/RRC-IDLE and UE finds a cell which provides access by E-UTRA NTN RAT }

ensure that {

when { UE has no valid GNSS position }

then { UE shall not establish RRC connection to perform an attach procedure }

}

(2)

with { UE in EMM-DEREGISTERED/RRC-IDLE and UE finds a cell which provides access by E-UTRA NTN RAT }

ensure that {

when { Cell broadcasts systemInformationBlockType31 and UE has a valid GNSS position }

then { UE establishes RRC connection, and includes gnss-validityDuration in RRCConnectionSetupComplete-NB message, and successfully performs an attach procedure }

}

6.1.1.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clauses 4.12, 23.21.1 and 23.21.6, TS 36.331 clauses 5.2.2.3, 5.2.2.4, 5.3.3.1a and 5.3.3.4. Unless otherwise stated these are Rel-17 requirements.

[TS 36.300, clause 4.12]

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figure 4.12-1 below illustrates an example of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Figure 4.12-1: Overall illustration of an NTN

NOTE:	Figure 4.12-1 illustrates an NTN; RAN4 aspects are out of scope.

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The following connectivity is supported by the NTN payload:

-	An eNB may serve multiple NTN payloads;

-	An NTN payload may be served by multiple eNBs.

NOTE:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

[TS 36.300, clause 23.21.1]

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers.

UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment.

[TS 36.300, clause 23.21.6]

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.

The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

For a BL UE or a UE in enhanced coverage or a NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS optimisation, the eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., depending on operator's policy) or up to implementation.

NOTE 2:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

[TS 36.331, clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

…

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockType21, SystemInformationBlockType26 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), and SystemInformationBlockType28 (if UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication), SystemInformationBlockType30 (if UE is configured by upper layers to report disaster roaming related information);

…

3>	the UE is NTN capable:

4>	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT), if scheduled;

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

2>	if the UE supports discontinuous coverage; and

2>	if schedulingInfoList indicates that SystemInformationBlockType32 (SystemInformationBlockType32-NB in NB-IoT) is present and the UE does not have a valid version of this system information block:

3>	acquire SystemInformationBlockType32 (SystemInformationBlockType32-NB in NB-IoT);

[TS 36.331, clause 5.3.3.1d]

If systemInformationBlockType31 (systemInformationBlockType31-NB in NB-IoT) is broadcast, a RRC connection is initiated only if the UE has a valid GNSS position.

NOTE:	The UE may need to re-acquire the GNSS position before establishing the connection to avoid interruption during the connection.

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to NTN:

3>	include gnss-validityDuration in accordance with the remaining time of the GNSS validity duration;

…

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

6.1.1.10.3	Test description

6.1.1.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508 [18] clause 4.4.1.1 is configured.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE's positioning engine (e.g., standalone GNSS receiver) shall be disabled or no UE location is preconfigured.

Preamble:

-	The UE is in state Switched OFF (State 1) according to TS 36.508 [18] clause 4.5.1.

6.1.1.10.3.2	Test procedure sequence

Table 6.1.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 1A | Check: Does the UE send an RRCConnectionRequest on Ncell 1 within 6 minutes? | - | - | 1 | F |
| 1B | Power off the UE. | - | - | - | - |
| 1C | Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within 6 minutes? | --> | RRCConnectionRequest | 2 | P |
| 3-17 | Steps 3 to 17 of the registration procedure described in TS 36.508 [18] subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| Note 1: This could be done by use of AT command and the detailed UE location is defined in TS 36.508 [18] Clause 4.13. |  |  |  |  |  |

6.1.1.10.3.3	Specific message contents

Table 6.1.1.10.3.3-1: RRCConnectionSetupComplete (step 4, table 6.1.1.10.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.6.1-18 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| gnss-ValidityDuration-r17 |  | Present but contents not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 6.1.1.11	eMTC / NTN / Multi-TAC

6.1.1.11	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED/RRC-IDLE and UE finds a cell which provides access by E-UTRA RAT }

ensure that {

when { systemInformationBlockType1-BR broadcast more than one TAC per PLMN }

then { UE forwards the trackingAreaList to upper layers for mobility management procedure }

}

6.1.1.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 23.21.4.1 and TS 36.331 clauses 5.2.2.7 and 6.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 36.300, clause 23.21.4.1]

The network may broadcast more than one TAC per PLMN in a cell in order to reduce the signalling load at cell edge in NTN, in particular for Earth-moving cell coverage. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. UEs may by UE implementation also check whether a TAC has been removed.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

…

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

3>	forward the trackingAreaList to upper layers, if present;

3>	forward the PLMN identity to upper layers;

[TS 36.331, clause 6.2.2]

trackingAreaList A list of tracking area codes for the PLMN listed. For the first entry in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area code in trackingAreaCode (without suffix) and the tracking area codes in trackingAreaList. If this field is absent, trackingAreaCode (without suffix) applies. For other entries in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area codes in trackingAreaList. If this field is absent, the list of tracking area codes of the preceding entry in plmn-IdentityList-v1700 applies. The total number of signalled tracking area codes across all PLMNs cannot be more than maxTAC-r17.

6.1.1.11.3	Test description

6.1.1.11.3.1	Pre-test conditions

System Simulator:

-	Cell 1 (belongs to TAI-1, TAI-2) and Cell 11.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1-1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to TS 36.508 [18].

6.1.1.11.3.2	Test procedure sequence

Table 6.1.1.11.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.1.11.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 6.1.1.11.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T2 of table 6.1.1.11.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 11 within 3 minutes? | --> | RRCConnectionRequest | 1 | F |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 [18] subclause 6.4.2.2 indicate that the UE is camped on Cell 11? | - | - | 1 | - |

6.1.1.11.3.3	Specific message contents

Table 6.1.1.11.3.3-1: SystemInformationBlockType1-BR-r13 on Cell 1 (preamble and all steps, Table 6.1.1.11.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-v1700-IEs SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred |  |  |
| PLMN-IdentityList-v1700 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1700 SEQUENCE { | 1 entry |  |  |
| PLMN-IdentityInfo-v1700 SEQUENCE { |  | Entry 1 |  |
| TrackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode SEQUENCE { | 2 entries |  |  |
| TrackingAreaCode[1] | 1 |  |  |
| TrackingAreaCode[2] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.11.3.3-2: SystemInformationBlockType31 on Cell 11 (preamble and all steps, Table 6.1.1.11.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -23550424 |  |  |
| positionY-r17 | 22301093 |  |  |
| positionZ-r17 | 0 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 6.1.2	Cell selection and reselection

#### 6.1.2.1	Void

#### 6.1.2.2	Cell selection / Qrxlevmin

6.1.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria which are not fulfilled (S<0)}

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell including the cell selection criteria for a cell which are also fulfilled (S>0)}

then { the UE considers the cell suitable and camps on it }

}

6.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.1.2.2, 5.2.3.1, 5.2.3.2 and 5.2.8.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

[TS 36.304, clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value), provided that the following high quality criterion is fulfilled:

1.	For an E-UTRAN cell, the measured RSRP value shall be greater than or equal to -110 dBm.

Found PLMNs that do not satisfy the high quality criterion, but for which the UE has been able to read the PLMN identities are reported to the NAS together with the RSRP value. The quality measure reported by the UE to NAS shall be the same for each PLMN found in one cell.

...

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 36.304, clause 5.2.3.1]

The UE shall use one of the following two cell selection procedures:

a)	Initial Cell Selection

This procedure requires no prior knowledge of which RF channels are E-UTRA carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.8]

In this state, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high quality cell, as defined in subclause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state until an acceptable cell is found.

6.1.2.2.3	Test description

6.1.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.2.2.3.2	Test procedure sequence

Table 6.1.2.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.2.3.2-2

Table 6.1.2.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -95 | The power level value is such to satisfy SrxlevCell 1 < 0 but the UE is able to read the PLMN identity |
|  | Qrxlevmin | dBm | -84 |  |
|  | Qrxlevminoffset | dB | 0 |  |
|  | Pcompensation | dB | 0 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -75 | The power level is such that SrxlevCell 1 > 0 |
| Note: The downlink signal level uncertainty is specified in TS 36.508 section 4.3.4.1 |  |  |  |  |

Table 6.1.2.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T1" in table 6.1.2.2.3.2-1. | - | - | - | - |
| 0B | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 3 | SS re-adjusts the cell-specific reference signal level of Cell 1 level according to row "T2" in table 6.1.2.2.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 5-19 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.1.2.2.3.3	Specific message contents

Table 6.1.2.2.3.3-1: SystemInformationBlockType1 for cell 1 (all steps)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -42 (-84 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.2a	Cell selection / Qqualmin

6.1.2.2a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria which are not fulfilled (Srxlev>0 AND Squal<0)}

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell including the cell selection criteria for a cell which are also fulfilled (Srxlev>0 AND Squal>0)}

then { the UE considers the cell suitable and camps on it }

}

6.1.2.2a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.1.2.2, 5.2.3.1, 5.2.3.2 and 5.2.8.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn (“initial cell selection”) or make use of stored information to shorten the search (“stored information cell selection”).

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of “forbidden tracking areas for roaming”;

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

[TS 36.304, clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value), provided that the following high quality criterion is fulfilled:

1.	For an E-UTRAN cell, the measured RSRP value shall be greater than or equal to -110 dBm.

Found PLMNs that do not satisfy the high quality criterion, but for which the UE has been able to read the PLMN identities are reported to the NAS together with the RSRP value. The quality measure reported by the UE to NAS shall be the same for each PLMN found in one cell.

...

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 36.304, clause 5.2.3.1]

The UE shall use one of the following two cell selection procedures:

a)	Initial Cell Selection

This procedure requires no prior knowledge of which RF channels are E-UTRA carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation Squal = Qqualmeas – (Qqualmin + Qqualminoffset)

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX –PPowerClass, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.8]

In this state, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high quality cell, as defined in subclause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state until an acceptable cell is found.

6.1.2.2a.3	Test description

6.1.2.2a.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.2.2a.3.2	Test procedure sequence

Table 6.1.2.2a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.2a.3.2-2

Table 6.1.2.2a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -95 | The power level value is such to satisfy SrxlevCell 1 > 0 and SqualCell 1 < 0 but the UE is able to read the PLMN identity |
|  | RSRQ | dB | -32 |  |
|  | Noc | dBm/15kHz | -75 |  |
|  | Qrxlevmin | dBm | -106 | (default value in TS 36.508 Table 4.4.3.2-3) |
|  | Qrxlevminoffset | dB | 0 |  |
|  | Qqualmin | dB | -18 |  |
|  | Qqualminoffset | dB | 0 |  |
|  | Pcompensation | dB | 0 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -65 | The power level is such that SrxlevCell 1 > 0 and SqualCell 1 > 0 |
|  | RSRQ | dB | -5 |  |
|  | Noc | dBm/15kHz | -75 |  |
| Note: The downlink signal level uncertainty is specified in TS 36.508 section 4.3.4.1 |  |  |  |  |

Table 6.1.2.2a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T1" in table 6.1.2.2a.3.2-1. | - | - | - | - |
| 2 | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 5 | SS re-adjusts the cell-specific reference signal level of Cell 1 level according to row "T2" in table 6.1.2.2a.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 7-21 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.1.2.2a.3.3	Specific message contents

Table 6.1.2.2a.3.3-1: SystemInformationBlockType1 for Cell 1 (all steps)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -18 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.2b	Cell selection / UE Cat 0 not allowed

6.1.2.2b.1	Test Purpose (TP)

(1)

with { Cat 0 UE in E-UTRA RRC_IDLE state on a cell broadcasting SystemInformationBlockType1 with IE category0Allowed included }

ensure that {

when { another cell not including the IE category0Allowed in broadcasted SystemInformationBlockType1 becomes better }

then { the UE considers the other cell as barred and does not reselect the cell }

}

6.1.2.2b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 5.2.8 and TS 36.331, clause 5.2.2.7.

[TS 36.304, clause 5.2.8]

In this state, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high quality cell, as defined in subclause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state until an acceptable cell is found.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1>	if the UE is a category 0 UE according to 36.306 [5]; and

1>	if category0Allowed is not included in SystemInformationBlockType1:

2>	consider the cell as barred in accordance with TS 36.304 [4];

6.1.2.2b.3	Test description

6.1.2.2b.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.2b.3.2	Test procedure sequence

Table 6.1.2.2b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.2b.3.2-2.

Table 6.1.2.2b.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | SrxlevCell 1 > 0, SrxlevCell 2 > 0 , RCell 1 < RCell 2 |
|  | Srxlev* | dB | 15 | 21 | Cell 2 becomes stronger than Cell 1 |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.1.2.2b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1and Cell 2 level according to the row "T1" in table 6.1.2.2b.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 10s? | --> | RRCConnectionRequest | 1 | F |

6.1.2.2b.3.3	Specific message contents

Table 6.1.2.2b.3.3-1: SystemInformationBlockType1 for Cell 2 (preamble and all steps)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-v12xy SEQUENCE { |  |  |  |
| category0Allowed-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.2c	Cell selection / Qrxlevmin / Enhanced Coverage

6.1.2.2c.1	Test Purpose (TP)

(1)

with { Enhanced Coverage Capable UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria (Srxlev > 0) for neither normal nor enhanced coverage are fulfilled }

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(2)

with { Enhanced Coverage Capable UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria (Srxlev > 0) for normal coverage is not fulfilled and the cell selection criteria for enhanced coverage is fulfilled }

then { the UE considers itself to be in enhanced coverage and camps on the cell }

}

6.1.2.2c.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 23.7b, and TS 36.304, clause 5.2.3.2. Unless otherwise stated these are Rel-13.

[TS 36.300, clause 23.7b]

A UE in enhanced coverage is a UE that requires the use of enhanced coverage functionality to access the cell.

A UE may access a cell using enhanced coverage functionality only if the MIB of the cell indicates that access of UEs in enhanced coverage is supported. System information procedures for UEs in enhanced coverage are identical to the system information procedures for bandwidth reduced low complexity UEs. A UE capable of enhanced coverage acquires, if needed, and uses legacy system information when in normal coverage if it is not a BL UE. A UE capable of enhanced coverage acquires, if needed, and uses system information specific for UEs in enhanced coverage. A UE in enhanced coverage is not required to detect SIB change when in RRC_CONNECTED.

A set of PRACH resources (e.g. time, frequency, preamble); each associated with a coverage enhancement level, is provided in SIB. Number of PRACH repetitions and number of maximum preamble transmission attempts per coverage enhancement level are provided in SIB. UEs in same enhanced coverage level use random access resources associated with the same enhanced coverage level. Time/frequency resources and repetition factor for random access response messages for UEs in enhanced coverage are derived from the used PRACH resources.

A UE in enhanced coverage is paged using the same mechanism for paging BL UEs. The starting subframe of a paging occasion and the repetition pattern (in both time and frequency domain for downlink common control signalling) of that paging occasion are determined irrespective of the UEs enhanced coverage level.

The paging request from the MME for a UE supporting enhanced coverage functionality may contain enhanced coverage level related information and corresponding cell ID.

A UE in RRC_IDLE does not inform the network when it changes the enhanced coverage level.

A UE in enhanced coverage camps in a suitable cell where S criterion for UEs in enhanced coverage is fulfilled. The UE shall re-select to inter-frequency cells in which it is able to operate in normal coverage over cells in which it has to be in enhanced coverage.

Connected mode mobility mechanisms such as measurement reporting, network controlled handover etc., are supported for UEs in enhanced coverage. No additional mechanisms are introduced to support the use of enhanced coverage functionality to access an E-UTRA cell during inter-RAT handovers.

[TS 36.304, clause 5.2.3.2]

For NB-IoT the cell selection criterion is defined in sub-clause 5.2.3.2.a.

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

6.1.2.2c.3	Test description

6.1.2.2c.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.2.2c.3.2	Test procedure sequence

Table 6.1.2.2c.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.2c.3.2-2

Table 6.1.2.2c.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -95 | The power level value is such to satisfy SrxlevCell 1 < 0 but the UE is able to read the PLMN identity |
|  | Qrxlevmin_CE | dBm | -82 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -69 | The power level is such that SrxlevCell 1 > 0 |
| Note: The total test tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and absolute UE measurement accuracy of RSRP (TS 36.133 clause 9.1.21.1 in CEModeA). |  |  |  |  |

Table 6.1.2.2c.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts the cell levels according to row T1 of table 6.1.2.2c.3.2-1. | - | - | - | - |
| 2 | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 5 | SS re-adjusts the cell-specific reference signal level of Cell 1 level according to row "T2" in table 6.1.2.2c.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 7-21 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.1.2.2c.3.3	Specific message contents

Table 6.1.2.2c.3.3-1: CellSelectionInfoCE-r13 for SystemInformationBlockType1-BR-r13 on Cell 1 (all steps)

| Derivation Path: 36.508 Table 4.6.3-1AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CellSelectionInfoCE-r13::= SEQUENCE { |  |  |  |
| q-RxLevMinCE-r13 | -41 (-82 dBm) |  |  |
| } |  |  |  |

#### 6.1.2.2d	Cell selection / Qqualmin / Enhanced Coverage

6.1.2.2d.1	Test Purpose (TP)

(1)

with { Enhanced Coverage Capable UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria (Srxlev>0 AND Squal<0) for neither normal nor enhanced coverage are fulfilled }

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(2)

with { Enhanced Coverage Capable UE in E-UTRA RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell including the cell selection criteria (Srxlev>0 AND Squal>0) for enhanced coverage but not for normal coverage}

then { the UE considers the cell suitable and camps on it }

}

6.1.2.2d.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 23.7b, and TS 36.304, clause 5.2.3.2. Unless otherwise stated these are Rel-13.

[TS 36.300, clause 23.7b]

A UE in enhanced coverage is a UE that requires the use of enhanced coverage functionality to access the cell. In this release of specification two enhanced coverage modes (mode A, mode B) are supported. The support of enhanced coverage mode A is mandatory for a BL UE.

A UE may access a cell using enhanced coverage functionality only if the MIB of the cell indicates that scheduling information for SIB1 specific for BL UEs is scheduled. System information procedures for UEs in enhanced coverage are identical to the system information procedures for bandwidth reduced low complexity UEs. A UE capable of enhanced coverage acquires, if needed, and uses legacy system information when in normal coverage if it is not a BL UE. A UE capable of enhanced coverage acquires, if needed, and uses system information specific for UEs in enhanced coverage. A UE in enhanced coverage is not required to detect SIB change when in RRC_CONNECTED.

A set of PRACH resources (e.g. time, frequency, preamble); each associated with a coverage enhancement level, is provided in SIB. Number of PRACH repetitions and number of maximum preamble transmission attempts per coverage enhancement level are provided in SIB. UEs in same enhanced coverage level use random access resources associated with the same enhanced coverage level. Time/frequency resources and repetition factor for random access response messages for UEs in enhanced coverage are derived from the used PRACH resources.

A UE in enhanced coverage is paged using the same mechanism for paging BL UEs. The starting subframe of a paging occasion and the repetition pattern (in both time and frequency domain for downlink common control signalling) of that paging occasion are determined irrespective of the UEs enhanced coverage level.

The paging request from the MME for a UE supporting enhanced coverage functionality may contain enhanced coverage level related information and corresponding cell ID.

A UE in RRC_IDLE does not inform the network when it changes the enhanced coverage level.

A UE in enhanced coverage camps on a suitable cell where S criterion for UEs in enhanced coverage is fulfilled. The UE shall re-select to inter-frequency cells in which it is able to operate in normal coverage over cells in which it has to be in enhanced coverage.

Connected mode mobility mechanisms such as measurement reporting, network controlled handover etc., are supported for UEs in enhanced coverage. No additional mechanisms are introduced to support the use of enhanced coverage functionality to access an E-UTRA cell during inter-RAT handovers.

[TS 36.304, clause 5.2.3.2]

For NB-IoT the cell selection criterion is defined in sub-clause 5.2.3.2.a.

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

6.1.2.2d.3	Test description

6.1.2.2d.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.1.2.2d.3.2	Test procedure sequence

Table 6.1.2.2d.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.2d.3.2-2.

Table 6.1.2.2d.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -73 | The power level value is such to satisfy SrxlevCell 1 > 0 and SqualCell 1 < 0 but the UE is able to read the PLMN identity |
|  | RSRQ* | dB | -10 |  |
|  | Noc | dBm/15kHz | -75 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | The power level is such that SrxlevCell 1 > 0 and SqualCell 1 > 0 for enhanced coverage, and SrxlevCell 1 > 0 and SqualCell 1 < 0 for normal coverage |
|  | RSRQ* | dB | -4 |  |
|  | Noc | dBm/15kHz | -75 |  |
| Note: The downlink signal level uncertainty is specified in TS 36.508 section 4.3.4.1 |  |  |  |  |

Table 6.1.2.2d.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts the cell levels according to row T1 of table 6.1.2.2d.3.2-1. | - | - | - | - |
| 2 | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 5 | SS re-adjusts the cell-specific reference signal level of Cell 1 level according to row "T2" in table 6.1.2.2d.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 7-21 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.1.2.2d.3.3	Specific message contents

Table 6.1.2.2d.3.3-1: Void

Table 6.1.2.2d.3.3-2: SystemInformationBlockType1-BR-r13 for Cell 1 (Step 1 onwards)

| Derivation Path: 36.508 table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -3 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| tdd-Config-v1130 | Not present |  |  |
| cellSelectionInfo-v1130 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-v1250 SEQUENCE { |  |  |  |
| category0Allowed-r12 | Not Present |  |  |
| } |  |  |  |
| cellSelectionInfo-v1250 | Not present |  |  |
| freqBandIndicatorPriority-r12 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r13 | Not present |  |  |
| eDRX-Allowed-r13 | Not present |  |  |
| eDRX-Allowed-r13 | True |  |  |
| cellSelectionInfoCE-r13::= SEQUENCE { |  |  |  |
| q-RxLevMinCE-r13 | -53 (-106 dBm) |  |  |
| q-QualMinRSRQ-CE-r13 | -8 dB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.3	Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (S<0 or barred)

6.1.2.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (S<0)and there is a suitable neighbour cell (S>0) }

then { UE selects the suitable neighbour cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable neighbour cell}

then { UE selects the suitable neighbour cell }

}

6.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, 5.2.3.2, 5.2.4.6 and 5.3.1.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.4.6]

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1 sharing, this IE is common for all PLMNs

...

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

...

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

6.1.2.3.3	Test description

6.1.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 have different tracking areas according to table 6.0.1-2.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.3.3.2	Test procedure sequence

Table 6.1.2.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.3.3.2-2.

Table 6.1.2.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE (FDD/TDD) | dBm/15kHz | "Off" | -85 | SrxlevCell 1 < 0 (NOTE 1) |
|  | Srxlev* | dB | - | 25 | Cell 11 becomes the strongest cell |
| T2 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | SrxlevCell 11 > 0, SrxlevCell 1 > 0 , RCell 1 < RCell 11 |
|  | Srxlev* (FDD) | dB | 19 | 25 |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -89 | -85 |  |
|  | Srxlev* (TDD) | dB | 21 | 25 | Cell 1 becomes suitable |
|  | cellBarred | - | notBarred | barred | Serving cell becomes barred |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

NOTE: If the UE fails the test because of a failure to detect and reselect to a right cell, then the operator may re-run the test.

Table 6.1.2.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 and Cell 11 according to row "T1" in table 6.1.2.3.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 11? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | SS changes Cell 1 signal level and SIB1 IE cellBarred according to row "T2" in table 6.1.2.3.3.2-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |

6.1.2.3.3.3	Specific message contents

Table 6.1.2.3.3.3-1: Void

Table 6.1.2.3.3.3-2: SystemInformationBlockType1 for cells 1 and 11 (preamble and all steps, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| } |  |  |  |
| systemInfoValueTag | The value is increased in step 3 |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-2A: SystemInformationBlockType1-BR-r13 for cells 1 and 11 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| } |  |  |  |
| systemInfoValueTag | The value is increased in step 3 |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-3: SystemInformationBlockType3 for cells 1 and 11 (Preamble and all steps, table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-4: SystemInformationBlockType1 for cell 11 (step 3, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.3.3.3-4A: SystemInformationBlockType1-BR-r13 for cell 11 (step 3 when UE under test is CAT M1, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.3.3.3-5: Paging (step 3, Table 6.1.2.3.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | True |  |  |
| etws-PrimaryNotificationIndication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### 6.1.2.3a	Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (Srxlev > 0 and Squal < 0)

6.1.2.3a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (Srxlev > 0 and Squal < 0)and there is a suitable neighbour cell (Srxlev > 0 and Squal > 0) }

then { UE selects the suitable neighbour cell }

}

6.1.2.3a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, 5.2.3.2 and 5.2.4.6.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn (“initial cell selection”) or make use of stored information to shorten the search (“stored information cell selection”).

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of “forbidden tracking areas for roaming”;

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in [10].

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation Squal = Qqualmeas – (Qqualmin + Qqualminoffset)

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX –PPowerClass, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.4.6]

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.3a.3	Test description

6.1.2.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.3a.3.2	Test procedure sequence

Table 6.1.2.3a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.1.2.3a.3.2-2.

Table 6.1.2.3a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -97 | -85 | SrxlevCell 1 > 0 and SqualCell 1 < 0 |
|  | RSRQ | dB | -15.28 | -3.28 |  |
|  | Qrxlevmin | dBm | -106 | -92 |  |
|  | Qrxlevminoffset | dB | 0 | 0 |  |
|  | Qqualmin | dB | -5 | -20 |  |
|  | Qqualminoffset | dB | 0 | 0 |  |
|  | Pcompensation | dB | 0 | 0 |  |
|  | Srxlev* | dB | 9 | 7 | Cell 2 is suitable cell |
|  | Squal* | dB | -10.28 | 16.72 | Squal = Qqualmeas – (Qqualmin + Qqualminoffset) |
|  | Noc | dBm/15kHz | Off | Off |  |

Table 6.1.2.3a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 and Cell 2 according to row "T1" in table 6.1.2.3a.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.3a.3.3	Specific message contents

Table 6.1.2.3a.3.3-0: Conditions for table 6.1.2.3a.3.3-1, 6.1.2.3a.3.3-2

Condition descriptions

Cell 1 This condition applies to system information transmitted on Cell 1.

Cell 2 This condition applies to system information transmitted on Cell 2.

Table 6.1.2.3a.3.3-1: SystemInformationBlockType1 for Cell 1 and 2 (all steps, Table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3a.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 1 and 2 (all steps when UE under test is CAT M1, Table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3a.3.3-2: SystemInformationBlockType3 for Cell 1 and 2 (all steps, table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| threshServingLowQ-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.4	Cell reselection

6.1.2.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell ranked as the best cell }

then { UE reselects the new cell }

}

6.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2 and 5.2.4.6.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.4.3	Test description

6.1.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.4.3.2	Test procedure sequence

Table 6.1.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.4.3.2-2.

Table 6.1.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 2. |

Table 6.1.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 2 level according to the row "T1" in table 6.1.2.4.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? | - | - | 1 | - |

6.1.2.4.3.3	Specific message contents

Table 6.1.2.4.3.3-1: Void

#### 6.1.2.5	Cell reselection for interband operation

6.1.2.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell on the different frequency band }

then { UE reselects the new cell }

}

6.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

6.1.2.5.3	Test description

6.1.2.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1, cell 10.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.5.3.2	Test procedure sequence

Table 6.1.2.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.5.3.2-2.

Table 6.1.2.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | Cell 10 become stronger than Threshx, high. |

Table 6.1.2.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 10 power level according to the row "T1" in table 6.1.2.5.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |

6.1.2.5.3.3	Specific message contents

Table 6.1.2.5.3.3-0: Conditions for specific message contents
in Tables 6.1.2.5.3.3-1 and 6.1.2.5.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.5.3.3-1: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5.3.3-2: SystemInformationBlockType5 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  | Cell 1 |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 10 |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 10 | 20dB |  |
| cellReselectionPriority | 5 |  | Cell 1 |
|  | 1 |  | Cell 10 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  | Cell 1 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.5a	Cell reselection for interband operation/ Power Class 2 UE operation/ Between FDD and TDD

6.1.2.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state on a FDD band}

ensure that {

when { UE detects the power class 2 cell re-selection criteria is met for the cell on a TDD band }

then { UE reselects to the new cell }

}

6.1.2.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.3.2, 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0  AND  Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp

Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

6.1.2.5a.3	Test description

6.1.2.5a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is set to LTE-FDD mode and Cell 10 is set to LTE-TDD mode.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.5a.3.2	Test procedure sequence

Table 6.1.2.5a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.5a.3.2-2.

Table 6.1.2.5a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -71 | Cell 10 becomes stronger than Threshx, high. |

Table 6.1.2.5a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 10 power level according to the row "T1" in table 6.1.2.5a.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |
| Note: Pcompensation is used while determining Srxlev of Cell 10. Pcomp for PC2 UE = max(Pmax – Ppowerclass, 0) i.e. max(26dBm – 26dBm, 0) which comes to 0. Pcomp for PC3 UE = max(Pmax – Ppowerclass, 0) i.e. max(26dBm – 23dBm, 0) which comes to 3. Srxlev of Cell 10 using Pcomp of PC2 is 35dB and using Pcomp of PC3 is 32dB. Threshx, high is set to 32dB in SIB5 of Cell 1, such that Srxlev of Cell 10 using Pcomp for PC2 UE is greater than that and Srxlev of Cell 10 using Pcomp of PC3 is lesser. i.e. Srxlev with PC3 ≤ Threshx, high < Srxlev with PC2. This is to ensure that only PC2 UE will be able to do reselection. |  |  |  |  |  |

6.1.2.5a.3.3	Specific message contents

Table 6.1.2.5a.3.3-0: Conditions for specific message contents
in Tables 6.1.2.5a.3.3-1, 6.1.2.5a.3.3-2, 6.1.2.5a.3.3-3 and 6.1.2.5a.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.5a.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| p-Max | 23dBm | Default | Cell 1 |
|  | 26dBm |  | Cell 10 |
| } |  |  |  |

Table 6.1.2.5a.3.3-2: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5a.3.3-3: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| p-max | 26dBm |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 16 | 32dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5a.3.3-4: SystemInformationBlockType5 for Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 16 | 32dB |  |
| cellReselectionPriority | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.5b	Cell reselection for interband operation using Pcompensation / Between FDD and TDD

6.1.2.5b.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state on a FDD band}

ensure that {

when { UE detects the re-selection criteria using Pcompensation is met for the cell on a TDD band }

then { UE reselects to the new cell }

6.1.2.5b.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.3.2, 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

6.1.2.5b.3	Test description

6.1.2.5b.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is set to LTE-FDD mode and Cell 10 is set to LTE-TDD mode.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.5b.3.2	Test procedure sequence

Table 6.1.2.5b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.5b.3.2-2.

Table 6.1.2.5b.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | Cell 1 becomes lower than Threshserving,low |

Table 6.1.2.5b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the cell power levels according to the row "T1" in table 6.1.2.5b.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |

6.1.2.5b.3.3	Specific message contents

Table 6.1.2.5b.3.3-0: Conditions for specific message contents
in Tables 6.1.2.5b.3.3-1, 6.1.2.5b.3.3-2, 6.1.2.5b.3.3-3 and 6.1.2.5b.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.5b.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5b.3.2-2)

| Derivation path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| p-Max | 33dBm |  |  |
| } |  |  |  |

Table 6.1.2.5b.3.3-1A: SystemInformationBlockType1-BR-r13  for Cell 1 and Cell 10 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.5b.3.2-2)

| Derivation path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| p-Max | 33dBm |  |  |
| } |  |  |  |

Table 6.1.2.5b.3.3-2: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 6 | 12dB |  |
| cellReselectionPriority | 5 |  | Cell 1 |
|  | 1 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5b.3.3-3: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.5b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| p-max | 33dBm |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 10 | 20dB |  |
| cellReselectionPriority | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5b.3.3-4: SystemInformationBlockType5 for Cell 10 (preamble and all steps, Table 6.1.2.5b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 10 | 20dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.5c	Inter-band Cell reselection / Extended frequency list

6.1.2.5c.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell on the different frequency band and the inter-frequency cell in interFreqCarrierFreqListExt-r12 is the highest ranked cell }

then { UE reselects the new cell }

}

6.1.2.5.2c	Conformance requirements

Same as 6.1.2.5.2.

6.1.2.5c.3	Test description

6.1.2.5c.3.1	Pre-test conditions

Same as 6.1.2.5.3.1 and

System Simulator:

-	  In addition to the 2 configured cells (Cell 1 and Cell 10), a non-configured inter-frequency Cell 29 shall be referred to only in SIB5 for Signalling purpose to achieve the test purpose.

6.1.2.5c.3.2	Test procedure sequence

Same as 6.1.2.5.3.2.

6.1.2.5c.3.3	Specific message contents

Table 6.1.2.5c.3.3-0: Conditions for specific message contents
in Tables 6.1.2.5c.3.3-1 and 6.1.2.5c.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.5c.3.3-1: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5c.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5c.3.3-2: SystemInformationBlockType5 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5c.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq [0] | EARFCN of Cell 29 | Non used cell |  |
| dl-CarrierFreq [0] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | EARFCN of Cell 29 | Non used cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| interFreqCarrierFreqListExt-r12 SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-r12 | EARFCN of Cell 10 |  | Cell 1 |
|  | EARFCN of Cell 1 |  | Cell 10 |
| q-RxLevMin-r12 | -53 (-106 dBm) |  |  |
| t-ReselectionEUTRA-r12 | 7 |  |  |
| threshX-High-r12 | 10 (20 dB) |  |  |
| threshX-Low-r12 | 1 (2 dB) |  |  |
| allowedMeasBandwidth-r12 | See subclause 4.4.3.4 |  |  |
| presenceAntennaPort1-r12 | FALSE |  |  |
| cellReselectionPriority-r12 | 5 |  | Cell 1 |
|  | 1 |  | Cell 10 |
| neighCellConfig-r12 | '01'B |  |  |
| q-OffsetFreq-r12 | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.6	Cell reselection using Qhyst, Qoffset and Treselection

6.1.2.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Qhyst is non-zero or its value changes in system information }

then { UE reselects the highest ranked cell taking the actual Qhyst value into account}

}

(2)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { cell reselection criteria are fulfilled during a time interval Treselection }

then { UE reselects the highest ranked cell }

}

(3)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Qoffset is non-zero or its value changes in system information }

then { UE reselects the highest ranked cell taking the actual Qoffset value into account }

}

6.1.2.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.2 and TS 36.304, clauses 5.2.1 and 5.2.4.6 and TS 36.331 clause 6.3.1.

[TS 36.300, clause 10.1.1.2]

UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following:

-	The UE makes measurements of attributes of the serving and neighbour cells to enable the reselection process:

-	There is no need to indicate neighbouring cell in the serving cell system information to enable the UE to search and measure a cell i.e. E-UTRAN relies on the UE to detect the neighbouring cells;

-	For the search and measurement of inter-frequency neighbouring cells, only the carrier frequencies need to be indicated;

-	Measurements may be omitted if the serving cell attribute fulfils particular search or measurement criteria.

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	Intra-frequency reselection is based on ranking of cells;

...

-	Cell reselection parameters are applicable for all UEs in a cell, ...

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell.

...

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT,

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

The IE SystemInformationBlockType4 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as exclude-listed cells.

6.1.2.6.3	Test description

6.1.2.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.6.3.2	Test procedure sequence

Table 6.1.2.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2", "T3", "T4", "T5", "T6" and "T7" are applied at the points indicated in the Main behaviour description in Table 6.1.2.6.3.2-2.

Table 6.1.2.6.3.2-1: Time instances of cell power level and parameter change

| Ti | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | Cell 2 becomes stronger than Cell 1 but Cell 1 remains the highest ranked one due to QhystsCell1 |
|  | Cell-specific RS EPRE (TDD) |  | -89 | -85 |  |
| T2 | Qhysts | dB | 0 | 0 | QhystsCell1 change causes Cell 2 to become highest ranked cell |
| T3 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | Cell 1 becomes the strongest and highest ranked one due to |
|  | Cell-specific RS EPRE (TDD) |  | -85 | -89 |  |
|  | Qoffsets,n | dB | 24 | 0 | Qoffsets,nCell2 remains zero |
| T4 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | Cell 1 becomes weaker but it remains the highest ranked one due to Qoffsets,nCell 1 |
|  | Cell-specific RS EPRE (TDD) |  | -89 | -85 |  |
| T5 | Qoffsets,n | dB | 0 | 0 | Cell 2 becomes the highest ranked one due to Qoffsets,nCell1 change |
| T6 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | Cell 1 becomes the highest ranked one |
|  | Cell-specific RS EPRE (TDD) |  | -85 | -89 |  |
|  | TreselectionEUTRAN | s | 7 | 0 |  |
| T7 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | Cell 2 becomes the highest ranked cell |
|  | Cell-specific RS EPRE (TDD) |  | -89 | -85 |  |
| Note: The total test tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and relative UE measurement accuracy of RSRP (TS 36.133 clause 9.1.2.2). |  |  |  |  |  |

Table 6.1.2.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 1 second to allow UE to enter RRC_IDLE state on cell 1. | - | - | - | - |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.6.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 10s? | --> | RRCConnectionRequest | 1 | F |
| 3 | SS resets QhystsCell1 according to row "T2" in table 6.1.2.6.3.2-1 and notifies UE of the system information change. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? (Note 1) NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 5 | Void | - | - | - | - |
| 5A | SS changes Qoffsets,nCell1 according to rows "T3" in table 6.1.2.6.3.2-1 and sends notification of the system information change. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 5B | Wait for 2.1* modification period (Note 2) to allow the new system information to take effect. | - | - | - | - |
| 5C | SS re-adjusts cell-specific reference signal levels according to rows "T3" in table 6.1.2.6.3.2-1. | - | - | - | - |
| 6 | Wait until there is any RRCConnectionRequest send from the UE on Cell 1. | --> | RRCConnectionRequest | - | - |
| 7-11 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 11A | Wait for 1 second to allow UE to enter RRC_IDLE state on cell 1. | - | - | - | - |
| 12 | SS re-adjusts cell-specific reference signal levels according to row "T4" in table 6.1.2.6.3.2-1. | - | - | - | - |
| 13 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 10s? | --> | RRCConnectionRequest | 3 | F |
| 14 | SS resets Qoffsets,nCell1 according to row "T5" in table 6.1.2.6.3.2-1 and notifies UE of the system information change. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? (Note 1) NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | - |
| 16 | Void | - | - | - | - |
| 16A | SS changes TreselectionEUTRAN Cell 1 according to rows "T6" in table 6.1.2.6.3.2-1 and sends notification of the system information change. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 16B | Wait for 2.1* modification period (Note 2) to allow the new system information to take effect. | - | - | - | - |
| 16C | SS re-adjusts cell-specific reference signal levels according to rows "T6" in table 6.1.2.6.3.2-1. | - | - | - | - |
| 17 | Wait until there is any RRCConnectionRequest send from the UE on Cell 1 | --> | RRCConnectionRequest | - | - |
| 18-22 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22A | Wait for 1 second to allow UE to enter RRC_IDLE state on cell 1. | - | - | - | - |
| 23 | SS re-adjusts cell-specific reference signal levels according to rows "T7" in table 6.1.2.6.3.2-1. | - | - | - | - |
| 24 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 6s? | --> | RRCConnectionRequest | 2 | F |
| 25 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 12s? | --> | RRCConnectionRequest | 2 | P |
| 26-29 | Steps 2 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure. | - | - | - | - |

NOTE 1:	The time interval in step 4 and 15 is set to cover TevaluateFDD,intra (TS 36.133 clause 4.2.2.3) + 1280 ms for DRX cycle + 1280 ms for system information block type scheduling. The timer tolerances are taken into account according to [18].

NOTE 2:	The wait time of 2.1* modification period in step 5B and 16B is to allow for the network to paging the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period.

6.1.2.6.3.3	Specific message contents

Table 6.1.2.6.3.3-1: Void

Table 6.1.2.6.3.3-2: SystemInformationBlockType1 for cells 1 (preamble and all steps, Table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| schedulingInformation ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} |  |  |  |
| systemInfoValueTag | The value is increased by 1 in step 3, step 5A, step 14 and step 16A |  |  |
| } |  |  |  |

Table 6.1.2.6.3.3-3: SystemInformationBlockType3 for cell 1 (preamble, table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |  |
| q-Hyst |  | dB24 | QhystsCell1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.6.3.3-4: SystemInformationBlockType3 for cell 1 (step 3, table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |  |
| q-Hyst |  | dB0 | QhystsCell1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.6.3.3-5: SystemInformationBlockType3 for cell 1 (step 16A, table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |  |
| q-Hyst |  | dB0 | QhystsCell1 |  |
| } |  |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |  |
| t-ReselectionEUTRA |  | 7 | seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.6.3.3-6: SystemInformationBlockType4 for cell 1 (step 5A, table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |  |
| physCellId [1] |  | Physical cell identity of Cell 2 |  |  |
| q-OffsetCell [1] |  | dB24 | Qoffsets,nCell 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.6.3.3-7: SystemInformationBlockType4 for cell 1 (step 14, table 6.1.2.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |  |
| physCellId [1] |  | Physical cell identity of Cell 2 |  |  |
| q-OffsetCell [1] |  | dB0 | Qoffsets,nCell 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 6.1.2.6a	Cell reselection using Treselection / Enhanced Coverage

6.1.2.6a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and camped on a cell in enhanced coverage, and the UE is not in high mobility state }

ensure that {

when { cell reselection criteria are fulfilled during a time interval TreselectionEUTRA_CE }

then { UE reselects the highest ranked cell }

}

6.1.2.6a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 23.7b, and TS 36.304, clause 5.2.3.2, 5.2.4.6a and 5.2.4.7. Unless otherwise stated these are Rel-13.

[TS 36.300, clause 23.7b]

A UE in enhanced coverage is a UE that requires the use of enhanced coverage functionality to access the cell. In this release of specification two enhanced coverage modes (mode A, mode B) are supported. The support of enhanced coverage mode A is mandatory for a BL UE.

A UE may access a cell using enhanced coverage functionality only if the MIB of the cell indicates that scheduling information for SIB1 specific for BL UEs is scheduled. System information procedures for UEs in enhanced coverage are identical to the system information procedures for bandwidth reduced low complexity UEs. A UE capable of enhanced coverage acquires, if needed, and uses legacy system information when in normal coverage if it is not a BL UE. A UE capable of enhanced coverage acquires, if needed, and uses system information specific for UEs in enhanced coverage. A UE in enhanced coverage is not required to detect SIB change when in RRC_CONNECTED.

A set of PRACH resources (e.g. time, frequency, preamble); each associated with a coverage enhancement level, is provided in SIB. Number of PRACH repetitions and number of maximum preamble transmission attempts per coverage enhancement level are provided in SIB. UEs in same enhanced coverage level use random access resources associated with the same enhanced coverage level. Time/frequency resources and repetition factor for random access response messages for UEs in enhanced coverage are derived from the used PRACH resources.

A UE in enhanced coverage is paged using the same mechanism for paging BL UEs. The starting subframe of a paging occasion and the repetition pattern (in both time and frequency domain for downlink common control signalling) of that paging occasion are determined irrespective of the UEs enhanced coverage level.

The paging request from the MME for a UE supporting enhanced coverage functionality may contain enhanced coverage level related information and corresponding cell ID.

A UE in RRC_IDLE does not inform the network when it changes the enhanced coverage level.

A UE in enhanced coverage camps on a suitable cell where S criterion for UEs in enhanced coverage is fulfilled. The UE shall re-select to inter-frequency cells in which it is able to operate in normal coverage over cells in which it has to be in enhanced coverage.

Connected mode mobility mechanisms such as measurement reporting, network controlled handover etc., are supported for UEs in enhanced coverage. No additional mechanisms are introduced to support the use of enhanced coverage functionality to access an E-UTRA cell during inter-RAT handovers.

[TS 36.304, clause 5.2.3.2]

For NB-IoT the cell selection criterion is defined in sub-clause 5.2.3.2.a.

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

[TS 36.304, clause 5.2.4.6]

…

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

…

[TS 36.304, clause 5.2.4.6a]

Ranking  as defined in sub-clause 5.2.4.6 is applied for intra-frequency and inter-frequency cell reselection (irrespective of configured frequency priorities, if any) while the UE is in enhanced coverage.

[TS 36.304, clause 5.2.4.7]

…

TreselectionEUTRA_ CE

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN  when a neighbour cell is evaluated for camping in enhanced coverage. The parameter can be set per E-UTRAN frequency.

…

6.1.2.6a.3	Test description

6.1.2.6a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	SystemInformationBlockType3 using parameters as specified in Table 6.1.2.6a.3.3-1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.6a.3.2	Test procedure sequence

Table 6.1.2.6a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.6a.3.2-2.

Table 6.1.2.6a.3.2-1: Time instances of cell power level and parameter change

| Ti | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | Cell 1 remains the highest ranked |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | Cell 2 becomes the highest ranked cell |
| Note: The total test tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and relative UE measurement accuracy of RSRP (TS 36.133 clause 9.1.21.2 in CEModeA). |  |  |  |  |  |

Table 6.1.2.6a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for 1 second to allow UE to enter RRC_IDLE state on Cell 1. | - | - | - | - |
| 2 | SS adjusts the cell levels according to row T1 of table 6.1.2.6a.3.2-1. | - | - | - | - |
| 3 | Wait for 60s to ensure UE detects Cell 2 | - | - | - | - |
| 4 | SS re-adjusts cell-specific reference signal levels according to rows "T2" in table 6.1.2.6a.3.2-1. | - | - | - | - |
| 5 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 15s? | --> | RRCConnectionRequest | 1 | F |
| 6 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within the next 30s? | --> | RRCConnectionRequest | 1 | P |
| 7-10 | Steps 2 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure. | - | - | - | - |

6.1.2.6a.3.3	Specific message contents

Table 6.1.2.6a.3.3-1: SystemInformationBlockType3 for cell 1 (preamble and all steps, Table 6.1.2.6a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 with condition CEmodeA |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |
| q-RxLevMin | -54 dBm |  |
| } |  |  |
| lateNonCriticalExtension { |  |  |
| t-ReselectionEUTRA-CE-r13 | 15 | seconds |
| } |  |  |
| } |  |  |

Table 6.1.2.6a.3.3-2: SystemInformationBlockType4 for cell 1 (preamble and all steps 6.1.2.6a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |  |
| physCellId [1] |  | Physical cell identity of Cell 2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 6.1.2.6b	Cell reselection from cell in enhanced coverage to inter-frequency cell in normal coverage

6.1.2.6b.1	Test Purpose (TP)

(1)

with { a non-CAT M1 UE in E-UTRA RRC_IDLE state, and camped on a cell in enhanced coverage }

ensure that {

when { another inter-frequency cell meeting the normal cell reselection criteria becomes higher ranked }

then { UE reselects to the inter-frequency cell in normal coverage}

}

6.1.2.6b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 23.7b, and TS 36.304, clause 5.2.3.2, 5.2.4.6a and 5.2.4.7. Unless otherwise stated these are Rel-13.

[TS 36.300, clause 23.7b]

A UE in enhanced coverage is a UE that requires the use of enhanced coverage functionality to access the cell. In this release of specification two enhanced coverage modes (mode A, mode B) are supported. The support of enhanced coverage mode A is mandatory for a BL UE.

A UE may access a cell using enhanced coverage functionality only if the MIB of the cell indicates that scheduling information for SIB1 specific for BL UEs is scheduled. System information procedures for UEs in enhanced coverage are identical to the system information procedures for bandwidth reduced low complexity UEs. A UE capable of enhanced coverage acquires, if needed, and uses legacy system information when in normal coverage if it is not a BL UE. A UE capable of enhanced coverage acquires, if needed, and uses system information specific for UEs in enhanced coverage. A UE in enhanced coverage is not required to detect SIB change when in RRC_CONNECTED.

A set of PRACH resources (e.g. time, frequency, preamble); each associated with a coverage enhancement level, is provided in SIB. Number of PRACH repetitions and number of maximum preamble transmission attempts per coverage enhancement level are provided in SIB. UEs in same enhanced coverage level use random access resources associated with the same enhanced coverage level. Time/frequency resources and repetition factor for random access response messages for UEs in enhanced coverage are derived from the used PRACH resources.

A UE in enhanced coverage is paged using the same mechanism for paging BL UEs. The starting subframe of a paging occasion and the repetition pattern (in both time and frequency domain for downlink common control signalling) of that paging occasion are determined irrespective of the UEs enhanced coverage level.

The paging request from the MME for a UE supporting enhanced coverage functionality may contain enhanced coverage level related information and corresponding cell ID.

A UE in RRC_IDLE does not inform the network when it changes the enhanced coverage level.

A UE in enhanced coverage camps on a suitable cell where S criterion for UEs in enhanced coverage is fulfilled. The UE shall re-select to inter-frequency cells in which it is able to operate in normal coverage over cells in which it has to be in enhanced coverage.

Connected mode mobility mechanisms such as measurement reporting, network controlled handover etc., are supported for UEs in enhanced coverage. No additional mechanisms are introduced to support the use of enhanced coverage functionality to access an E-UTRA cell during inter-RAT handovers.

[TS 36.304, clause 5.2.3.2]

…

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

[TS 36.304, clause 5.2.4.6a]

Ranking with cell selection criterion S for enhanced coverage is applied for intra-frequency and inter-frequency cell reselection when the current serving cell can only be accessed using enhanced coverage.

6.1.2.6b.3	Test description

6.1.2.6b.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 have different tracking areas according to table 6.0.1-2. Cell 1 supports CE and Cell 3 does not support CE.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	The UE is in state Registered, Idle mode (State 2-CE) in Cell 1 according to [18].

6.1.2.6b.3.2	Test procedure sequence

Table 6.1.2.6b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.6a.3.2-2.

Table 6.1.2.6b.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -115 | The power levels, Qrxlevmin and Qrxlevmin_CE for Cell 1 and Cell 3 are set such that: Srxlev > 0 for Cell 1 in enhanced coverage is met, but not for normal coverage; and Srxlev > 0 for Cell 3 is non-suitable cell both for normal or enhanced coverage |
|  | Qrxlevmin | dBm | -54dBm | -106 dBm |  |
|  | Qrxlevmin_CE | dBm | -106 dBm | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power levels are such that Srxlev > 0 for normal coverage is met for Cell 3 and that Cell 3 becomes the highest ranked cell |
| Note: The total test tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and relative UE measurement accuracy of RSRP (TS 36.133 clause 9.1.21.2 in CEModeA). |  |  |  |  |  |

Table 6.1.2.6b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell-specific reference signal levels according to rows "T1" in table 6.1.2.6b.3.2-1. | - | - | - | - |
| 2 | Wait for 60s to ensure UE detects Cell 3 | - | - | - | - |
| 3 | SS adjusts cell-specific reference signal levels according to rows "T2" in table 6.1.2.6b.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 3 within the next 30s? | --> | RRCConnectionRequest | 1 | P |
| 5-9 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 3. NOTE: The UE performs a TAU procedure. | - | - | - | - |

6.1.2.6b.3.3	Specific message contents

Table 6.1.2.6b.3.3-1: SystemInformationBlockType1 for Cell 3 (preamble and all steps in Table 6.1.2.6b.3.2-2)

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |
| cellSelectionInfo SEQUENCE { |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |
| } |  |  |
| } |  |  |

Table 6.1.2.6b.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 3 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.6b.3.2-2)

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |
| cellSelectionInfo SEQUENCE { |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |
| } |  |  |
| } |  |  |

#### 6.1.2.7	Cell reselection / Equivalent PLMN

6.1.2.7.1	Test Purpose (TP)

(1)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Location Registration procedure }

ensure that {

when { a suitable neighbour cell on an equivalent PLMN becomes better ranked than the suitable Serving cell }

then { UE reselects to this equivalent PLMN cell. }

}

(2)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Location Registration procedure }

ensure that {

when { the Serving cell becomes unsuitable due to S<0 and a suitable cell exists on an equivalent PLMN }

then { UE reselects to this equivalent PLMN cell. }

}

(3)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Location Registration procedure }

ensure that {

when { the UE evaluates cells for reselection }

then { UE ignores suitable cells for which the UE has no reselection priority provided. }

}

6.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.2.4.1, TS 23.122 clause 4.4.3.

[TS 36.304, clause 5.2.4.1]

UE shall only perform reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

[TS 23.122, clause 4.4.3]

The Mobile Equipment stores a list of "equivalent PLMNs". This list is replaced or deleted at the end of each location update procedure, routing area update procedure and GPRS attach procedure. The stored list consists of a list of equivalent PLMNs as downloaded by the network plus the PLMN code of the registered PLMN that downloaded the list. All PLMNs in the stored list, in all access technologies supported by the PLMN, are regarded as equivalent to each other for PLMN selection, cell selection/re-selection and handover.

6.1.2.7.3	Test description

6.1.2.7.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.2.7.3.1-1.

Table 6.1.2.7.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |
| 13 | PLMN3 |

-	All cells are high quality.

-	All cells are suitable cells.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508 clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN3 in the Equivalent PLMN list as described in Table 6.1.2.7.3.3-1.

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.7.3.2	Test procedure sequence

Table 6.1.2.7.3.2–1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.1.2.7.3.2-2.

Table 6.1.2.7.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -67 | -82 | -97 | All cells S>0 |
| T1 | RS EPRE | dBm/15kHz | -115 | -82 | -97 | Cell 1 S < 0 as described in TS 36.508 clause 6.2.2.1 |
| T2 | RS EPRE | dBm/15kHz | -67 | -97 | -82 | All cells S>0 |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3. |  |  |  |  |  |  |

Table 6.1.2.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE send an RRCConnectionRequest within the next 100 seconds? | --> | RRCConnectionRequest | 1 | F |
| 2 | SS adjusts cell levels according to row T1 of table 6.1.2.7.3.2-1. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 13? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN2 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 2 | P |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 12? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN3 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 1 | P |
| 6 | Void | - | - | - | - |
| 7 | SS adjusts cell levels according to row T0 of table 6.1.2.7.3.2-1. | - | - | - | - |
| 8 | Check: Does the UE send an RRCConnectionRequest within the next 100 seconds? | --> | RRCConnectionRequest | 1,3 | F |
| 9 | SS adjusts cell levels according to row T2 of table 6.1.2.7.3.2-1. | - | - | - | - |
| 10 | Void | - | - | - | - |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 13? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN2 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 3 | P |
| Note 1: In Steps 1 and 8, a wait time long enough to ensure that the UE has detected, measured and evaluated the neighbour inter-frequency cells is provided, see 36.133 clause 4.2.2.4. Note 2: In Step 11, the UE moves to Cell 13 because no reselection priority is defined for Cell 1, see Table 6.1.2.7.3.3-6. |  |  |  |  |  |

6.1.2.7.3.3	Specific message contents

Table 6.1.2.7.3.3-0: Conditions for specific message contents
in Table 6.1.2.7.3.3-6 and Table 6.1.2.7.3.3-7

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.7.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN3 |  |  |

Table 6.1.2.7.3.3-2: TRACKING AREA UPDATE ACCEPT for Cell 13 (steps 3 & 11 , Table 6.1.2.7.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 2 |  |  |

Table 6.1.2.7.3.3-3: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 5, Table 6.1.2.7.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 3 |  |  |

Table 6.1.2.7.3.3-4: Void

Table 6.1.2.7.3.3-5: Void

Table 6.1.2.7.3.3-6: SystemInformationBlockType5 for Cell 12 (preamble and all steps, Table 6.1.2.7.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | Not present | UE does not have a priority for frequency of Cell 1 |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.7.3.3-7: SystemInformationBlockType5 for Cell 13 (preamble and all steps, Table 6.1.2.7.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | Not present | UE does not have a priority for frequency of Cell 1 |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 12 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.7a	Cell reselection / Equivalent PLMN / Single Frequency operation

6.1.2.7a.1	Test Purpose (TP)

(1)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Location Registration procedure }

ensure that {

when { a suitable neighbour cell on an equivalent PLMN becomes better ranked than the suitable Serving cell }

then { UE reselects to this equivalent PLMN cell. }

}

(2)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Location Registration procedure }

ensure that {

when { the Serving cell becomes unsuitable due to S<0 and a suitable cell exists on an equivalent PLMN }

then { UE reselects to this equivalent PLMN cell. }

}

6.1.2.7a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.2.7.2

6.1.2.7a.3	Test description

6.1.2.7a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency multi-PLMN cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.2.7a.3.1-1.

Table 6.1.2.7a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 2 | PLMN2 |
| 4 | PLMN3 |

-	All cells are high quality.

-	All cells are suitable cells.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508 clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN3 in the Equivalent PLMN list as described in Table 6.1.2.7a.3.3-1.

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.7a.3.2	Test procedure sequence

Table 6.1.2.7a.3.2–1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.1.2.7a.3.2-2. `

Table 6.1.2.7a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -67 | -73 | -79 | All cells S>0 |
| T1 | RS EPRE | dBm/15kHz | -115 | -88 | -82 | Cell 1 S < 0 as described in TS 36.508 clause 6.2.2.1 |
| T2 | RS EPRE | dBm/15kHz | -115 | -82 | -88 |  |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3. |  |  |  |  |  |  |

Table 6.1.2.7a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE send an RRCConnectionRequest within the next 100 seconds? | --> | RRCConnectionRequest | 1 | F |
| 2 | SS adjusts cell levels according to row T1 of table 6.1.2.7a.3.2-1. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 4? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN2 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 2 | P |
| 4 | SS adjusts cell levels according to row T2 of table 6.1.2.7a.3.2-1. | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN3 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 1 | P |
| 6 | SS adjusts cell levels according to row T1 of table 6.1.2.7a.3.2-1. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 4? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN2 listed as an Equivalent PLMN in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 1 | P |
| Note 1: In Step 1, a wait time long enough to ensure that the UE has detected, measured and evaluated the neighbour intra-frequency cells is provided, see 36.133 clause 4.2.2.3. |  |  |  |  |  |

6.1.2.7a.3.3	Specific message contents

Table 6.1.2.7a.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN3 |  |  |

Table 6.1.2.7a.3.3-2: TRACKING AREA UPDATE ACCEPT for Cell 4 (steps 3 & 7 , Table 6.1.2.7a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 2 |  |  |

Table 6.1.2.7a.3.3-3: TRACKING AREA UPDATE ACCEPT for Cell 2 (step 5, Table 6.1.2.7a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 3 |  |  |

#### 6.1.2.8	Cell reselection using cell status and cell reservations / Access control class 0 to 9

6.1.2.8.1	Test Purpose (TP)

(1)

with { UE camped normally in state E-UTRA RRC_IDLE and UE fitted with a USIM with access class 0..9}

ensure that {

when { a higher ranked cell is found with cell status "barred" }

then { UE does not attempt to reselect to the higher ranked cell }

}

(2)

with { UE camped normally in state E-UTRA RRC_IDLE and UE fitted with a USIM with access class 0..9}

ensure that {

when { a higher ranked cell is found "reserved" for Operator use }

then { UE does not attempt to reselect to the higher ranked cell }

}

6.1.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clauses 5.2.4.4 and 5.3.1.

[TS 36.304, clause 5.2.4.4]

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.4.6, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
…

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
…

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	...

-	UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 and UEs with AC 11 or 15 not operating in theirHLPMN/EHPLMN shall behave as if the cell status is “barred” in case the cell is “reserved for operator use” for the PLMN the UE is currently registered with.

NOTE 1:	...

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	...

-	else

-	If the IE intraFrequencyReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

6.1.2.8.3	Test description

6.1.2.8.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2, except that TAC values use the codes in Table 6.0.1-2.

-	SIB 1 of Cell 3 and Cell 6 indicate cellBarred=barred.

- Each cell has only a single PLMN identity.

- All cells are high quality.

- The cell power levels are configured as shown in Table 6.1.2.8.3.1-1.

- System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

Table 6.1.2.8.3.1-1: Cell power configuration

| Parameter | Unit | Cell 1 | Cell 3 | Cell 6 (min UL) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RS EPRE | dBm/15kHz | -97 | -82 | -67 | S>0 for all cells |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3 |  |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 6.1.2.8.3.1-2.

Table 6.1.2.8.3.1–2: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFACC | Type “A” as defined in TS 34.108 clause 8.3.2.15 |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.8.3.2	Test procedure sequence

Table 6.1.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SIB1 of Cell 3 to indicate cellBarred=notBarred | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 3 to indicate cellBarred=barred. (Cell 3 and Cell 6 are now both barred). | <-- | Paging | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 6 | SS adjusts SIB1 of both Cell 3 and Cell 6: Cell 3 indicates cellBarred=notBarred; Cell 6 indicates cellBarred=notBarred and cellReservedForOperatorUse = reserved. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |
| 8 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 3 to indicate cellReservedForOperatorUse = reserved. | <-- | Paging | - | - |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |

6.1.2.8.3.3	Specific message contents

Table 6.1.2.8.3.3-0: Conditions for specific message contents
in Tables 6.1.2.8.3.3-1, 6.1.2.8.3.3-2, 6.1.2.8.3.3-4 and 6.1.2.8.3.3-5

| Condition | Explanation |
| --- | --- |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 6 | This condition applies to system information transmitted on Cell 6. |

Table 6.1.2.8.3.3-1: SystemInformationBlockType1 for Cell 3 and Cell 6 (pre-test conditions, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.8.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (pre-test conditions when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.8.3.3-2: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 |
|  | barred |  | Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-2AA: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 1 when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 |
|  | barred |  | Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-2A: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 3, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-2AB: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 3 when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-3: Paging (steps 3 & 8, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.8.3.3-4: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 6, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 |
|  | reserved |  | Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-4A: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 6 when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 |
|  | reserved |  | Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-5: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 8, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | reserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 4 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.8.3.3-5A: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 8 when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | reserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 4 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

#### 6.1.2.8a	Cell reselection using cell status and cell reservations / Access control class 0 to 9 / Single Frequency operation

6.1.2.8a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.2.8.1

6.1.2.8a.2	Conformance requirements

Same Conformance requirement as in clause 6.1.2.8.2

6.1.2.8a.3	Test description

6.1.2.8a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency cells Cell 1, Cell 2 and Cell 4 as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2, except that TAC values use the codes in Table 6.0.1-2.

-	SIB 1 of Cell 4 indicates cellBarred=barred.

- Each cell has only a single PLMN identity.

- All cells are high quality.

- The cell power levels are configured as shown in Table 6.1.2.8a.3.1-1.

Table 6.1.2.8a.3.1-1: Cell power configuration

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -73 | -67 | -97 | Cell 2 is best ranked cell |
| T2 | RS EPRE | dBm/15kHz | -97 | -73 | -67 | Cell 4 is best ranked cell |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3 |  |  |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 6.1.2.8a.3.1-2.

Table 6.1.2.8a.3.1–2: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFACC | Type “A” as defined in TS 34.108 clause 8.3.2.15 |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.8a.3.2	Test procedure sequence

Table 6.1.2.8a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell power levels according to row T1 of table 6.1.2.8a.3.1–1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 2 to indicate cellBarred=barred. (Cell 2 and Cell 4 are now both barred). | <-- | Paging | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within 30 s? | - | - | 1 | F |
| 5 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.2.5 indicate that the UE does not respond to paging in cell 2 when paged with S-TMSI allocated to UE with CN domain indicator set to "PS"? | - | - | 1 | - |
| 6 | SS adjusts SIB1 of both Cell 2 and Cell 4: Cell 2 indicates cellBarred=notBarred; Cell 4 indicates cellBarred=notBarred and cellReservedForOperatorUse = reserved. SS adjusts cell levels according to row T2 of table 6.1.2.8a.3.1–1 | - | - | - | - |
| 7 | Check: Does the UE send an RRCConnectionRequest on Cell 4 within 30 s? | - | - | 2 | F |
| 9 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.2.5 indicate that the UE does not respond to paging in cell 2 when paged with S-TMSI allocated to UE with CN domain indicator set to "PS"? | - | - | 2 | - |
| 10 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 4 to indicate cellReservedForOperatorUse = notReserved. | <-- | Paging | - | - |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 4? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |
| Note : In steps 6-11, UE will have barred all the cells for 300 seconds hence may take more than 300 seconds to initiate the TAU procedure. The default value of intraFreqReselection is 'not allowed' in TS 36.508 table 4.4.3.2-3. |  |  |  |  |  |

6.1.2.8a.3.3	Specific message contents

Table 6.1.2.8a.3.3-0: Conditions for specific message contents
in Tables 6.1.2.8a.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |
| Cell 4 | This condition applies to system information transmitted on Cell 4. |

Table 6.1.2.8a.3.3-1: SystemInformationBlockType1 for Cell 4 (pre-test conditions, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 4 (pre-test conditions when UE under test is CAT M1, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-2: Void

Table 6.1.2.8a.3.3-2A: SystemInformationBlockType1 for Cell 2 (step 3, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-2B: SystemInformationBlockType1-BR-r13 for Cell 2 (step 3 when UE under test is CAT M1, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-3: Paging (steps 3 & 10, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-4: SystemInformationBlockType1 for Cell 2 and Cell 4 (step 6, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 |
|  | reserved |  | Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 2 |
|  | 1 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.8a.3.3-4A: SystemInformationBlockType1-BR-r13 for Cell 2 and Cell 4 (step 6 when UE under test is CAT M1, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 |
|  | reserved |  | Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 2 |
|  | 1 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.8a.3.3-5: SystemInformationBlockType1 for Cell 4 (step 10, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | notBarred |  |  |
| } |  |  |  |
| systemInfoValueTag | 2 |  |  |
| } |  |  |  |

Table 6.1.2.8a.3.3-5A: SystemInformationBlockType1-BR-r13 for Cell 4 (step 10 when UE under test is CAT M1, Table 6.1.2.8a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| cellBarred | notBarred |  |  |
| } |  |  |  |
| systemInfoValueTag | 2 |  |  |
| } |  |  |  |

#### 6.1.2.9	Cell reselection using cell status and cell reservations / Access control class 11 to 15

6.1.2.9.1	Test Purpose (TP)

(1)

with { UE camped normally in state E-UTRA RRC_IDLE and UE fitted with a USIM with access class 0..9 and access classes 11..15 inclusive }

ensure that {

when { a higher ranked cell is found with cell status "barred" }

then { UE does not attempt to reselect to the higher ranked cell }

}

(2)

with { UE camped normally in state E-UTRA RRC_IDLE and UE fitted with a USIM with access class 0..9 and access classes 11..15 inclusive }

ensure that {

when { a higher ranked cell is found "reserved" for Operator use }

then { UE re-selects to the higher ranked cell }

}

6.1.2.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clauses 5.2.4.4 and 5.3.1.

[TS 36.304, clause 5.2.4.4]

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.4.6, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
…

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
…

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	UEs assigned to Access Class 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the IE cellReservedForOperatorUse for that PLMN set to “reserved”.

-	UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 and UEs with AC 11 or 15 not operating in theirHLPMN/EHPLMN shall behave as if the cell status is “barred” in case the cell is “reserved for operator use” for the PLMN the UE is currently registered with.

NOTE 1:	...

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	...

-	else

-	If the IE intraFrequencyReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if selection/re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

6.1.2.9.3	Test description

6.1.2.9.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2, except that TAC values use the codes in Table 6.0.1-2.

-	SIB 1 of Cell 3 and Cell 6 indicate cellBarred=barred

-	Each cell has only a single PLMN identity.

-	All cells are high quality.

-	The cell power levels are configured as shown in Table 6.1.2.9.3.1-1.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

Table 6.1.2.9.3.1–1: Cell power configuration

| Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remarks |
| --- | --- | --- | --- | --- | --- |
| RS EPRE | dBm/15kHz | -97 | -82 | -67 | S>0 for all cells |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3 |  |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 6.1.2.9.3.1-2.

Table 6.1.2.9.3.1–2: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFACC | Type “B” as defined in TS 34.108 clause 8.3.2.15 |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.9.3.2	Test procedure sequence

Table 6.1.2.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SIB1 of Cell 3 to indicate cellBarred=notBarred | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure with establishment cause ‘high priority access' and the RRC connection is released. | - | - | 1 | - |
| 3 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 3 to indicate cellBarred=barred. (Cell 3 and Cell 6 are now both barred) . | <-- | Paging | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure with establishment cause ‘high priority access' and the RRC connection is released. | - | - | 1 | - |
| 6 | SS adjusts SIB1 of both Cell 3 and Cell 6: Cell 3 indicates cellBarred=notBarred; Cell 6 indicates cellBarred=notBarred and cellReservedForOperatorUse = reserved | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 6? NOTE: The UE performs a TAU procedure with establishment cause ‘high priority access’ and the RRC connection is released. | - | - | 2 | - |

6.1.2.9.3.3	Specific message contents

Table 6.1.2.9.3.3-0: Conditions for specific message contents
in Tables 6.1.2.9.3.3-1, 6.1.2.9.3.3-2 and 6.1.2.8.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 6 | This condition applies to system information transmitted on Cell 6. |

Table 6.1.2.9.3.3-1: SystemInformationBlockType1 for Cell 3 and Cell 6 (pre-test conditions, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.9.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (pre-test conditions when UE under test is CAT M1, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.9.3.3-2: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 1, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 |
|  | barred |  | Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.9.3.3-2AA: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 1 when UE under test is CAT M1, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 |
|  | barred |  | Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 3 |
|  | 0 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.9.3.3-2A: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 3, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 3 Cell 6 |
|  | 0 |  |  |
| } |  |  |  |

Table 6.1.2.9.3.3-2AB: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 3 when UE under test is CAT M1, Table 6.1.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 Cell 6 |
| } |  |  |  |
| cellBarred | barred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 3 Cell 6 |
|  | 0 |  |  |
| } |  |  |  |

Table 6.1.2.9.3.3-3: Paging (step 3, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.9.3.3-4: SystemInformationBlockType1 for Cell 3 and Cell 6 (step 6, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 |
|  | reserved |  | Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.9.3.3-4A: SystemInformationBlockType1-BR-r13 for Cell 3 and Cell 6 (step 6 when UE under test is CAT M1, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 3 |
|  | reserved |  | Cell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 3 Cell 6 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 3 |
|  | 1 |  | Cell 6 |
| } |  |  |  |

Table 6.1.2.9.3.3-5: RRCConnectionRequest (step2, step5 and step7, Table 6.1.2.9.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.9a	Cell reselection using cell status and cell reservations / Access control class 11 to 15 / Single Frequency operation

6.1.2.9a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.2.9.1.

6.1.2.9a.2	Conformance requirements

Same Conformance requirement as in clause 6.1.2.9.2.

6.1.2.9a.3	Test description

6.1.2.9a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency cells as specified in  TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2, except that TAC values use the codes in Table 6.0.1-2.

-	SIB 1 of Cell 2 and Cell 4 indicate cellBarred=barred

-	Each cell has only a single PLMN identity.

-	All cells are high quality.

-	The cell power levels are configured as shown in Table 6.1.2.9a.3.1-1.

Table 6.1.2.9a.3.1–1: Cell power configuration

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -73 | -67 | -97 | Cell 2 is best ranked cell |
| T2 | RS EPRE | dBm/15kHz | -97 | -73 | -67 | Cell 4 is best ranked cell |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 4.4.3.2 and 4.4.3.3 |  |  |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 6.1.2.9a.3.1-2.

Table 6.1.2.9a.3.1–2: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFACC | Type “B” as defined in TS 34.108 clause 8.3.2.15 |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.2.9a.3.2	Test procedure sequence

Table 6.1.2.9a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SIB1 of Cell 2 to indicate cellBarred=notBarred and cell levels according to row T1 of table 6.1.2.9a.3.1–1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure with establishment cause ‘high priority access' and the RRC connection is released. | - | - | - | - |
| 3 | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 2 to indicate cellBarred=barred. (Cell 2 and Cell 4 are now both barred). | <-- | Paging | - | - |
| 3A | Wait for 2.1* modification period (Note 2) to allow the new system information to take effect. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.2.5 indicate that the UE does not respond to paging in Cell 2 when paged with S-TMSI allocated to UE with CN domain indicator set to "PS"? | - | - | 1 | - |
| 5A | Check: Does the UE send an RRCConnectionRequest on Cell 1, Cell 2 or Cell 4 within 300 s of step 3? | - | - | 1 | F |
| 6 | SS adjusts SIB1 of both Cell 2 and Cell 4: Cell 2 indicates cellBarred=notBarred; Cell 4 indicates cellBarred=notBarred and cellReservedForOperatorUse = reserved. SS adjusts cell levels according to row T2 of table 6.1.2.9a.3.1–1. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 4? NOTE: The UE performs a TAU procedure with establishment cause ‘high priority access’ and the RRC connection is released. | - | - | 2 | - |
| Note 1: In steps 3-5A , UE will have barred all the cells for 300 seconds. The default value of intraFreqReselection is 'not allowed' in TS 36.508 table 4.4.3.2-3. Note 2: The wait time of 2.1* modification period in step 3A is to allow for the network to paging the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. |  |  |  |  |  |

6.1.2.9a.3.3	Specific message contents

Table 6.1.2.9a.3.3-0: Conditions for specific message contents
in Tables 6.1.2.9a.3.3-1, 6.1.2.9a.3.3-2, 6.1.2.9a.3.3-2A and 6.1.2.9a.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |
| Cell 4 | This condition applies to system information transmitted on Cell 4. |

Table 6.1.2.9a.3.3-1: SystemInformationBlockType1 for Cell 2 and Cell 4 (pre-test conditions, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | barred |  | Cell 2 Cell 4 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.9a.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 2 and Cell 4 (pre-test conditions when UE under test is CAT M1, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | barred |  | Cell 2 Cell 4 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.9a.3.3-2: SystemInformationBlockType1 for Cell 2 and Cell 4 (step 1, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 |
|  | barred |  | Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 2 |
|  | 0 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.9a.3.3-2A: SystemInformationBlockType1-BR-r13 for Cell 2 and Cell 4 (step 1 when UE under test is CAT M1, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 |
|  | barred |  | Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 1 |  | Cell 2 |
|  | 0 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.9a.3.3-2A: SystemInformationBlockType1 for Cell 2 and Cell 4 (step 3, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | barred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 2 Cell 4 |
|  | 0 |  |  |
| } |  |  |  |

Table 6.1.2.9a.3.3-2B: SystemInformationBlockType1-BR-r13 for Cell 2 and Cell 4 (step 3 when UE under test is CAT M1, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 Cell 4 |
| } |  |  |  |
| cellBarred | barred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 2 |  | Cell 2 Cell 4 |
|  | 0 |  |  |
| } |  |  |  |

Table 6.1.2.9a.3.3-3: Paging (step 3, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.9a.3.3-4: SystemInformationBlockType1 for Cell 2 and Cell 4 (step 6, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 |
|  | reserved |  | Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 2 |
|  | 1 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.9a.3.3-4A: SystemInformationBlockType1-BR-r13 for Cell 2 and Cell 4 (step 6 when UE under test is CAT M1, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Cell 2 |
|  | reserved |  | Cell 4 |
| } |  |  |  |
| cellBarred | notBarred |  | Cell 2 Cell 4 |
| } |  |  |  |
| systemInfoValueTag | 3 |  | Cell 2 |
|  | 1 |  | Cell 4 |
| } |  |  |  |

Table 6.1.2.9a.3.3-5: RRCConnectionRequest (step2, step5 and step7, Table 6.1.2.9a.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.10	Cell reselection in shared network environment

6.1.2.10.1	Test Purpose (TP)

with { the UE is in E-UTRA RRC_Idle and registered on the HPLMN }

ensure that {

when { a cell of a different PLMN but shared with the HPLMN becomes highest ranked cell }

then { the UE reselects the cell shared with the HPLMN }

6.1.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.2.4.2 and TS 23.122 clause 4.4.3.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 23.122, clause 4.4.3]

...

When the MS reselects to a cell in a shared network, the AS may indicate multiple PLMN identities to the NAS according to 3GPP TS 25.304 [32]. The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

...

6.1.2.10.3	Test description

6.1.2.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1 (HPLMN)

-	Cell 2 (primary PLMN: same MCC like HPLMN but different MNC, secondary PLMN: HPLMN)

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.10.3.2	Test procedure sequence

Table 6.1.2.10.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.10.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are assigned to satisfy SrxlevCell 2 > SrxlevCell 1 |

Table 6.1.2.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 2level according to the row "T1" in table 6.1.2.10.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup | <-- | RRCConnectionSetup | - | - |
| 4 | Check; Does the UE transmit an RRCConnectionSetupComplete message indicating the HPLMN (second PLMN in the list)? Note: this message contains an TRACKING AREA UPDATE REQUEST message according to default message contents. | --> | RRCConnectionSetupComplete | 1 | P |
| 5 - 7 | Steps 4 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |

6.1.2.10.3.3	Specific message contents

Table 6.1.2.10.3.3-1: SystemInformationBlockType1 (Cell 1, all steps, Table 6.1.2.10.3.2-2)

| Derivation path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.10.3.3-1A: SystemInformationBlockType1-BR-r13 (Cell 1, all steps when UE under test is CAT M1, Table 6.1.2.10.3.2-2)

| Derivation path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.10.3.3-2: SystemInformationBlockType1 (Cell 2, all steps, Table 6.1.2.10.3.2-2)

| Derivation path: 36.508 clause 4.4.3.2,Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code stored in EFIMSI on the test USIM, MNC=02 |  |  |
| plmn-Identity[2] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card | This is the same PLMN as Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.10.3.3-2A: SystemInformationBlockType1-BR-r13 (Cell 2, all steps, Table 6.1.2.10.3.2-2)

| Derivation path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code stored in EFIMSI on the test USIM, MNC=02 |  |  |
| plmn-Identity[2] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card | This is the same PLMN as Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.10.3.3-3: RRCConnectionSetupComplete (step 4, Table 6.1.2.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 2 | HPLMN |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.11	Inter-frequency Cell reselection

6.1.2.11.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects both intra-frequency and equal priority inter-frequency neigbour cells and the inter-frequency cell is the highest ranked cell }

then { UE reselects the inter-frequency cell }

}

6.1.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5 and 5.2.4.6.

[TS 36.304, clause 5.2.4.5]

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.11.3	Test description

6.1.2.11.3.1	Pre-test conditions

System Simulator:

-	Cell 1,Cell 2 and Cell 3 have different tracking areas according to table 6.0.1-2.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.11.3.2	Test procedure sequence

Table 6.1.2.11.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.11.3.2-2.

Table 6.1.2.11.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -73 | The power level values are set so that RCell 2 < RCell 1 < RCell 3. |

Table 6.1.2.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.11.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.11.3.3	Specific message contents

Table 6.1.2.11.3.3-1: Void

Table 6.1.2.11.3.3-2: Void

Table 6.1.2.11.3.3-3: Void

#### 6.1.2.11a	Inter-frequency Cell reselection / Extended frequency list

6.1.2.11a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects both intra-frequency and equal priority inter-frequency neighbour cells and the inter-frequency cell in interFreqCarrierFreqListExt-r12 is the highest ranked cell }

then { UE reselects the inter-frequency cell }

}

6.1.2.11a.2	Conformance requirements

Same as 6.1.2.11.2.

6.1.2.11a.3	Test description

6.1.2.11a.3.1	Pre-test conditions

Same as 6.1.2.11.3.1 and

System Simulator:

-	 In addition to the 3 configured cells (Cell 1, Cell 2 and Cell 3), a non-configured inter-frequency Cell 6 shall be referred to only in SIB5 for Signalling purpose to achieve the test purpose.

6.1.2.11a.3.2	Test procedure sequence

Same as 6.1.2.11.3.2.

6.1.2.11a.3.3	Specific message contents

Table 6.1.2.11a.3.3-1: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.11a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[0] | EARFCN of Cell 6 | Non used cell |  |
| dl-CarrierFreq[0] | 65535 | maxEARFCN | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| interFreqCarrierFreqList-v9e0 SEQUENCE { (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[0] | Downlink EARFCN of Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| interFreqCarrierFreqListExt-r12 SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-r12 | EARFCN of Cell 3 |  |  |
| q-RxLevMin-r12 | -53 (-106 dBm) |  |  |
| t-ReselectionEUTRA-r12 | 0 |  |  |
| threshX-High-r12 | 2 (4 dB) |  |  |
| threshX-Low-r12 | 1 (2 dB) |  |  |
| allowedMeasBandwidth-r12 | See subclause 4.4.3.4 |  |  |
| presenceAntennaPort1-r12 | FALSE |  |  |
| cellReselectionPriority-r12 | 4 |  |  |
| neighCellConfig-r12 | '01'B |  |  |
| q-OffsetFreq-r12 | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.12	Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list

6.1.2.12.1	Test Purpose

(1)

with { the UE is in RRC_IDLE and SystemInformationBlockType4 contain a cell-specific Qoffset for a neighbour intra frequency cell }

ensure that {

when { the neighbour cell has lower power than the serving cell but it is higher ranked due to the cell-specific Qoffset }

then { the UE reselects the neighbour cell with cell-specific Qoffset }

}

(2)

with { the UE is in RRC_IDLE and SystemInformationBlockType4 contain a exclude-listed cell }

ensure that {

when { a exclude-listed cell becomes higher ranked than the serving cell }

then { the UE remains camped on the serving cell }

}

6.1.2.12.2	Conformance Requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.304, clauses 5.2.4.1 and 5.2.4.6.

[TS 36.304 clause 5.2.4.1]

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

[TS 36.304 clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.12.3	Test description

6.1.2.12.3.1	Pre-test conditions

System Simulator:

-	cells 1, 4, and 11 on the same E-UTRA frequency and in different tracking areas

-	only 2 cells are simultaneously active, Cell 4 can be implemented as minimum uplink cell

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	cell 1 is transmitting SystemInformationBlockType4 according to specific message contents (Qoffset1,11 is -24dB)

-	cell 11 is transmitting SystemInformationBlockType4 according to specific message contents (Qoffset11,1 is 24dB, cell 4 is on excluded list)

NOTE:	Qoffset11,1 does not serve any test purpose, but symmetrical Qoffset values may be more realistic.

UE:

None.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.12.3.2	Test procedure sequence

Table 6.1.2.12.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Rows marked "T0" denote the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.12.3.2-1: Time instances of cell power level and parameter change

| Ti | Parameter | Unit | Cell 1 | Cell 11 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | Off | Only Cell 1 is on |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | Off | Cell 11 has lower power but is higher ranked due to cell-specific Qoffset1,11 |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 | Off |  |
| T2 | Cell-specific RS EPRE (FDD) | dBm/15kHz | Off | -91 | Off |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | Off | -89 | Off |  |
| T3 | Cell-specific RS EPRE (FDD) | dBm/15kHz | Off | -91 | -85 | Cell 4 has higher power than Cell 11 but is exclude-listed |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | Off | -89 | -85 |  |

Table 6.1.2.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.12.3.2-1. | - | - | - | - |
| 2 | Void |  |  |  |  |
| 2A-2E | Check: Does the test result of test steps 1 to 5 of generic procedure 6.4.2.7 indicate that the UE is camped on Cell 11? | - | - | 1 | - |
| 2F | The SS re-adjusts the cell-specific reference signal levels according to row "T2" in table 6.1.2.12.3.2-1 | - | - | - | - |
| 2G | The SS waits for 1s. Note: Delay is added to ensure cell power levels set at T2 has settled on correct level (36.523-3 cl. 7.4.2) before RRC connection is released. | - | - | - | - |
| 2H | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal levels according to row "T3" in table 6.1.2.12.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE initiate a random access procedure on Cell 4 within the next 120s? | - | - | 2 | F |

6.1.2.12.3.3	Specific message contents

Table 6.1.2.12.3.3-1: SystemInformationBlockType4 for cell 1 (all steps, Table 6.1.2.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellID of Cell 11 |  |  |
| q-OffsetCell[1] | dB-24 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.12.3.3-2: SystemInformationBlockType4 for cell 11 (all steps, Table 6.1.2.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList { | 1 entry |  |  |
| physCellId[1] | PhysicalCellID of Cell 1 |  |  |
| q-OffsetCell[1] | dB24 |  |  |
| } |  |  |  |
| intraFreqExcludedCellList SEQUENCE { | 1 entry |  |  |
| start[1] | PhysicalCellID of Cell 4 |  |  |
| range[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.13	Cell reselection, Sintrasearch, Snonintrasearch

6.1.2.13.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Sintrasearch is non-zero in system information }

then { UE perform measurement and reselects the highest ranked cell upon Srxlev < Sintrasearch }

}

(2)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Snonintrasearch is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the equal priority frequency cell upon Srxlev < Snonintrasearch }

}

(3)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Snonintrasearch is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the high priority frequency cell upon Srxlev > Snonintrasearch }

}

6.1.2.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.304, clauses 5.2.4.2, 5.2.4.5 and 5.2.4.6;  TS 36.331, clause 6.3.1;  TS 36.133, clause 4.2.2.4.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

.....

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

.....

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell.

...

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT,

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

[TS 36.133, clause 4.2.2.4]

…

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in  TS 36.304 within Kcarrier * Tdetect,EUTRAN_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5dB for reselections based on ranking or 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

6.1.2.13.3	Test description

6.1.2.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 3 have different tracking areas according to table 6.0.1-2.

- 	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.13.3.2	Test procedure sequence

Table 6.1.2.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.13.3.2-2.

Table 6.1.2.13.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | "Off" | Srxlev of Cell 1 is less than Sintrasearch. |
|  | Qrxlevmin | dBm | -106 | -106 | -106 |  |
|  | Sintrasearch | dB | 22 | 22 | 22 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -97 | -79 | Srxlev of Cell 2 is less than Snonintrasearch. |
|  | Snonintrasearch | dB | 16 | 16 | 16 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -79 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -79 | Srxlev of Cell 3 is greater than Snonintrasearch but Cell 1 is high priority cell. |
| Note: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 2 | Check: Does UE transmit an RRCConnectionRequest message on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 3 - 7 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 8 | The SS re-adjusts the cell-specific reference signal levels according to row "T2" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 9 | Check: Does UE transmit an RRCConnectionRequest message on Cell 3? | --> | RRCConnectionRequest | 2 | P |
| 10 - 14 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 3. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 14A | The SS re-adjusts the cell-specific reference signal levels according to row "T3" in table 6.1.2.13.3.2-1. |  |  |  |  |
| 15 | The SS notifies the UE of change of System Information and changes the system information for Cell 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 16 | Wait for 15 s for the UE to receive system information. | - | - | - | - |
| 16A | SS modifies SIB3 in Cell 1 and the systemInfoValueTag in the SystemInformationBlockType1 is increased. | - | - | - | - |
| 17 | The SS re-adjusts the cell-specific reference signal levels according to row " T4" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 18 | Check: Does the test result of steps 1 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? (Note1) NOTE: The UE performs a TAU procedure. Note: UE uses the new SIB3 modified in step 16A. | - | - | 3 | - |
| Note1: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.1.2.13.3.3	Specific message contents

Table 6.1.2.13.3.3-0: Conditions for specific message contents
in Table 6.1.2.13.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.13.3.3-1: Void

Table 6.1.2.13.3.3-2: SystemInformationBlockType3 for Cells 1, 2 and 3 (preamble and all steps, table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 8 (16 dB) |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 11 (22 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-3: SystemInformationBlockType1 for Cell 3 (step 15, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.13.3.3-3A: SystemInformationBlockType1-BR-r13 for Cell 3 (step 15 when UE under test is CAT M1, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 3 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType5. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| SystemInfoValueTagSI-r13[3] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-4: SystemInformationBlockType5 for Cell 3 (step 15, Table 6.1.2.13.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 5 | 10 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-5: Paging (step 15, Table 6.1.2.13.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-6: SystemInformationBlockType3 for Cell 1 (step 16A, table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 8 (16 dB) |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 11 (22 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.13.3.3-7: SystemInformationBlockType1 for Cell 1 (step 16A, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.13.3.3-7A: SystemInformationBlockType1-BR-r13  for Cell 1 (step 16A when UE under test is CAT M1, Table 6.1.2.13.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 3 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType3. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| SystemInfoValueTagSI-r13[3] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.14	Speed-dependent Cell reselection

6.1.2.14.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and in high mobility state }

ensure that {
  when { UE detects the cell ranked as the best cell taking into account scaling factor }

then { UE reselects the new cell }

}

6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.3 and 5.2.4.3.1. TS 36.133, clause 4.2.2.3.

[TS 36.304, clause 5.2.4.3]

Besides Normal-mobility state a High-mobility and a Medium-mobility state are applicable if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are sent in the system information broadcast of the serving cell.

State detection criteria:

...

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax exceeds NCR_H

The UE shall not count consecutive reselections between same two cells into mobility state detection criteria if same cell is reselected just after one other reselection.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

...

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in subclause 5.2.4.3.1.

[TS 36.304, clause 5.2.4.3.1]

UE shall apply the following scaling rules:

-	If neither Medium- nor Highmobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst” to Qhyst if sent on system information

-	For E-UTRAN cells multiply TreselectionEUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionEUTRA” if sent on system information

-	For UTRAN cells multiply TreselectionUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionUTRA” if sent on system information

-	For GERAN cells multiply TreselectionGERA by the sf-High of "Speed dependent ScalingFactor for TreselectionGERA state” if sent on system information

-	For CDMA2000 HRPD cells Multiply TreselectionCDMA_HRPD by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_HRPD” if sent on system information

-	For CDMA2000 1xRTT cells Multiply TreselectionCDMA_1xRTT by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_1xRTT” if sent on system information

...

In case scaling is applied to any TreselectionRAT parameter the UE shall round up the result after all scalings to the nearest second.

[TS 36.133, clause 4.2.2.3]

The UE shall be able to identify new intra-frequency cells and perform RSRP and RSRQ measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in  TS 36.304 within Tdetect,EUTRAN_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 for a corresponding Band.

The UE shall measure RSRP and RSRQ at least every Tmeasure,EUTRAN_Intra (see table 4.2.2.3-1) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter RSRP and RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_Intra/2

The UE shall not consider a E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,E-UTRAN_intra when Treselection = 0 as specified in table 4.2.2.3-1 provided that the cell is at least 3dB better ranked. When evaluating cells for reselection, the side conditions for RSRP and SCH apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

Table 4.2.2.3-1: Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate, E-UTRAN_intra

| DRX cycle length [s] | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

6.1.2.14.3	Test description

6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2, and Cell 4.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.14.3.2	Test procedure sequence

Table 6.1.2.14.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | -91 | Cell 2 becomes the highest ranked cell. |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -89 | -85 | -89 |  |
| T2 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -91 | -85 | Cell 4 becomes the highest ranked cell. |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -89 | -89 | -85 |  |
| T3 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | -91 | Cell 1 becomes the highest ranked cell. |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 | -89 |  |

Table 6.1.2.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row "T1" in table 6.1.2.14.3.2-1. | - | - | - | - |
| 2 | Wait for 40s for the UE to perform cell reselection procedure.(Note 1, Note 5) | - | - | - | - |
| 3 | The SS changes the power level setting according to the row “T2” in table 6.1.2.14.3.2-1. | - | - | - | - |
| 4 | Wait for 14s for the UE to perform cell reselection procedure.(Note 2, Note 6) | - | - | - | - |
| 5 | The SS changes the power level setting according to the row “T3” in table 6.1.2.14.3.2-1. | - | - | - | - |
| 6 | Wait for 14s for the UE to perform cell reselection procedure. (Note 3, Note 6) | - | - | - | - |
| 7 | The SS changes the power level setting according to the row “T1” in table 6.1.2.14.3.2-1. | - | - | - | - |
| 8 | Wait for 14s for the UE to perform cell reselection procedure. (Note 4, Note 6) | - | - | - | - |
| 9 | The SS changes the TAC for Cell 4 to 2. | - | - | - | - |
| 10 | The SS changes the power level setting according to the row “T2” in table 6.1.2.14.3.2-1. | - | - | - | - |
| 11 | Check: Does the UE send an RRCConnectionRequest on Cell 4 within 2s? (Note 7) | --> | RRCConnectionRequest | 1 | F |
| 12 | Check: Does the UE send an RRCConnectionRequest on Cell 4 within 7s? (Note 8) | --> | RRCConnectionRequest | 1 | P |
| 13-17 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. | - | - | 1 | - |
| Note 1: NCR (number of cell reselection) is 1. Note 2: NCR is 2. Note 3: NCR is 3. Note 4: NCR is 4 and the UE enters high mobility state because number of cell reselections during time period TCRmax exceeds NCR_H. Note 5: The wait time for reselection to a newly detected intra frequency cell is selected to cover Tdetect,EUTRAN_Intra(25*1280ms=32s) + Treselection (7s) + 320 ms system information block type scheduling = 39.32s rounded up to 40s. Note 6: The wait time for reselection to an already detected intra frequency cell is selected to cover Tevaluate,E-UTRAN_intra(6.4s) + Treselection (7s) + 320 ms system information block type scheduling = 13.72s rounded up to 14s. Note 7: The wait time for evaluating an intra-frequency cell is Treselection after scaling (2s) + 320 ms system information block type scheduling. This wait time is rounded down to nearest second (i.e. 2s in this case). Note 8: The wait time for cell reselection procedure is selected to cover Tevaluate,E-UTRAN_intra (6.4s) + Treselection after scaling (7*0.25) + 320 ms system information block type scheduling = 8.47s rounded up to 9s. The actual wait time is 7s which covers 9s - 2s (wait time in step 11). |  |  |  |  |  |

6.1.2.14.3.3	Specific message contents

Table 6.1.2.14.3.3-1: SystemInformationBlockType1 for Cell 2, and Cell 4 (preamble, Table 6.1.2.14.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.14.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 2, and Cell 4 (preamble when UE under test is CAT M1, Table 6.1.2.14.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.14.3.3-2: SystemInformationBlockType3 for Cell 1, Cell 2, and Cell 4 (preamble and all steps, Table 6.1.2.14.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| speedStateReselectionPars SEQUENCE { |  |  |  |
| mobilityStateParameters SEQUENCE { |  |  |  |
| t-Evalulation | s240 |  |  |
| t-HystNormal | s240 |  |  |
| n-CellChangeMedium | 3 |  |  |
| n-CellChangeHigh | 3 |  |  |
| } |  |  |  |
| q-HystSF SEQUENCE { |  |  |  |
| sf-Medium | dB0 |  |  |
| sf-High | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRAN | 7 |  |  |
| t-ReselectionEUTRA-SF SEQUENCE { |  |  |  |
| sf-Medium | oDot25 |  |  |
| sf-High | oDot25 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.14.3.3-3: SystemInformationBlockType1 for Cell 4 (step 9, Table 6.1.2.14.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 2 |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 6.1.2.14.3.3-3A: SystemInformationBlockType1-BR-r13 for Cell 4 (step 9 when UE under test is CAT M1, Table 6.1.2.14.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 2 |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

#### 6.1.2.15	Inter-frequency Cell reselection according to cell reselection priority provided by SIBs

6.1.2.15.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the equal priority frequency }

then { UE reselects the cell which belongs to the equal priority frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority frequency }

then { UE reselects the cell which belongs to the higher priority frequency }

}

(3)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority frequency }

then { UE reselects the cell which belongs to the lower priority frequency }

}

6.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2, 5.2.4.5 and 5.2.4.6.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and [T3230, FFS] in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

or if:

-	there are no candidate cells for reselection on serving frequency according to 5.2.4.4 or 5.3.1.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8].For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.15.3	Test description

6.1.2.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 6.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.15.3.2	Test procedure sequence

Table 6.1.2.15.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.15.3.2-2.

Table 6.1.2.15.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | ”Off” | The power level values are assigned to satisfy RCell 1 < RCell 3. (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -73 | -73 | The power level values are assigned to satisfy SrxlevCell 1 < 0 and SrxlevCell 6 > ThreshCell 6, high. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -73 | -95 | The power level values are assigned to satisfy SrxlevCell 6 < Threshserving, low and SrxlevCell 3 > ThreshCell 3, low, SrxlevCell 1< 0. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.15.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 3 power level according to the row "T1" in table 6.1.2.15.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 1 | - |
| 3 | The SS changes Cell 1 and Cell 6 power level according to the row "T2" in table 6.1.2.15.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 6? | - | - | 2 | - |
| 5 | The SS changes Cell 6 power level according to the row "T3" in table 6.1.2.15.3.2-1. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 3 | - |

6.1.2.15.3.3	Specific message contents

Table 6.1.2.15.3.3-0: Conditions for specific message contents
in Table 6.1.2.15.3.3-4 and Table 6.1.2.15.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.15.3.3-1: Void

Table 6.1.2.15.3.3-2: Void

Table 6.1.2.15.3.3-3: SystemInformationBlockType3 for Cell 6 (preamble and all steps, Table 6.1.2.15.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15.3.3-4: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.15.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[0] | EARFCN of Cell 6 |  |  |
| dl-CarrierFreq[0] | maxEARFCN |  | Band > 64 |
| threshX-High[0] | 10 | 20 dB |  |
| cellReselectionPriority[0] | 5 |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[0] | Same downlink EARFCN as used for Cell 6 |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15.3.3-5: SystemInformationBlockType5 for Cell 3 (preamble and all steps, Table 6.1.2.15.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 6 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.15a	Inter-frequency Cell reselection according to cell reselection priority provided by SIBs / Between FDD and TDD

6.1.2.15a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in FDD mode}

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the equal priority TDD frequency  }

then { UE reselects the cell which belongs to the equal priority TDD frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state in TDD mode}

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority FDD frequency }

then { UE reselects the cell which belongs to the higher priority FDD frequency }

}

(3)

with { UE in E-UTRA RRC_IDLE state in FDD mode}

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority TDD frequency }

then { UE reselects the cell which belongs to the lower priority TDD frequency }

}

6.1.2.15a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2, 5.2.4.5 and 5.2.4.6.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and [T3230, FFS] in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

or if:

-	there are no candidate cells for reselection on serving frequency according to 5.2.4.4 or 5.3.1.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8].For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.15a.3	Test description

6.1.2.15a.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 10 and Cell 3 have different tracking areas according to table 6.0.1-2.

-	Cell 1 and Cell 3 are set to LTE-FDD mode and Cell 10 is set to LTE-TDD mode.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.15a.3.2	Test procedure sequence

Table 6.1.2.15a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.15a.3.2-2.

Table 6.1.2.15a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | ”Off” | The power level values are assigned to satisfy RCell 1 < RCell 10. (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -73 | -73 | The power level values are assigned to satisfy SrxlevCell 1 < 0 and SrxlevCell 3 > ThreshCell 10, high. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -73 | -95 | The power level values are assigned to satisfy SrxlevCell 3 < Threshserving, low and SrxlevCell 10 > ThreshCell 3, low, SrxlevCell 1< 0. (NOTE 1). |
| Note 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.15a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 10 power level according to the row "T1" in table 6.1.2.15a.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | P |
| 3 | The SS changes Cell 1 and Cell 3 power level according to the row "T2" in table 6.1.2.15a.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 2 | P |
| 5 | The SS changes Cell 3 power level according to the row "T3" in table 6.1.2.15a.3.2-1. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 3 | P |

6.1.2.15a.3.3	Specific message contents

Table 6.1.2.15a.3.3-0: Conditions for specific message contents
in Table 6.1.2.7.3.3-2 and Table 6.1.2.7.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.15a.3.3-1: SystemInformationBlockType3 for Cell 3 (preamble and all steps, Table 6.1.2.15a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15a.3.3-2: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.15a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[0] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[0] | maxEARFCN |  | Band > 64 |
| threshX-High[0] | 10 | 20 dB |  |
| cellReselectionPriority[0] | 5 |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 10 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[0] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15a.3.3-3: SystemInformationBlockType5 for Cell 10 (preamble and all steps, Table 6.1.2.15a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.15b	Inter-band Cell reselection according to cell reselection priority provided by SIBs

6.1.2.15b.1	Test Purpose (TP)

Same Test Purposes as in clause 6.1.2.15.1

6.1.2.15b.2	Conformance requirements

Same Conformance requirements as in clause 6.1.2.15.2

6.1.2.15b.3	Test description

6.1.2.15b.3.1	Pre-test conditions

System Simulator:

-	Cell 1, cell 10 and  ‘If (px_SinglePLMN_Tested = Multi PLMN in primary band)’ Cell 3 ‘ELSE If (px_SinglePLMN_Tested  = Multi PLMN in Secondary band)’ Cell 28.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.15b.3.2	Test procedure sequence

Table 6.1.2.15b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.15b.3.2-2.

Table 6.1.2.15b.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 or Cell 28 (If Present) | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | ”Off” | The power level values are assigned to satisfy RCell 1 < RCell 3 or 28 (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -73 | -73 | The power level values are assigned to satisfy SrxlevCell 1 < 0 and SrxlevCell 10 > ThreshCell 10, high. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -73 | ”Off” | -95 | The power level values are assigned to satisfy SrxlevCell 10 < Threshserving, low and SrxlevCell 1 > ThreshCell 1, low, SrxlevCell 3 or 28< 0. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.15b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | Exception: Steps 1a1 and 1a2 are executed if cell 3 or cell 28 are configured | - | - | - | - |
| 1a1 | The SS changes Cell 3/ Cell 28 power level according to the row "T1" in table 6.1.2.15b.3.2-1. | - | - | - | - |
| 1a2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3/ Cell 28 (if present)? | - | - | 1 | - |
| 2 | The SS changes Cell 1 and Cell10 power level according to the row "T2" in table 6.1.2.15b.3.2-1. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 2 | - |
| 4 | The SS changes Cell 10 and cell 1 power level according to the row "T3" in table 6.1.2.15b.3.2-1. | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 3 | - |

6.1.2.15b.3.3	Specific message contents

Table 6.1.2.15b.3.3-0: Conditions for specific message contents
in Table 6.1.2.15b.3.3-2 and Table 6.1.2.15b.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.15b.3.3-1: SystemInformationBlockType3 for Cell 10 (preamble and all steps, Table 6.1.2.15b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15b.3.3-2: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.15b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[0] | EARFCN of Cell 10 |  |  |
| dl-CarrierFreq[0] | maxEARFCN |  | Band > 64 |
| threshX-High[0] | 10 | 20 dB |  |
| cellReselectionPriority[0] | 5 |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3/Cell 28 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[0] | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3/28 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15b.3.3-3: SystemInformationBlockType5 for Cell 3/Cell 28 (preamble and all steps, Table 6.1.2.15b.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 10 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 10 | 20 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.16		Cell reselection / interband operation / Between FDD and TDD

6.1.2.16.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in FDD mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell on the different TDD frequency band }

then { UE reselects the new cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state in TDD mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell on the different FDD frequency band }

then { UE reselects the new cell }

}

6.1.2.16.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

6.1.2.16.3	Test description

6.1.2.16.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 28 and Cell 30.

- 	Cell 1 is FDD mode, Cell 28,Cell30 are TDD mode in the same TDD band.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.2.16.3.1-1.

Table 6.1.2.13.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 28 | PLMN2 |
| 30 | PLMN3 |

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2/PLMN3 in the Equivalent PLMN list as described in Table 6.1.2.16.3.3-5.

6.1.2.16.3.2	Test procedure sequence

Table 6.1.2.16.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.16.3.2-2.

Table 6.1.2.16.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 28 | Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -73 | Off | -73 | Cell 1 and Cell 30 becomes stronger than Threshx, high., but Cell 30 has the higher priority (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -73 | -73 | -85 | Cell 1 and Cell 28 become stronger than Threshx, high., but Cell 1 has the higher priority(NOTE 1). |
| Note 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.16.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 30 power level according to the row "T1" in table 6.1.2.16.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 30? NOTE: The UE performs a TAU procedure with PLMN1 and PLMN2 listed as Equivalent PLMN list in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 1, |  |
| - | The SS changes the SystemInformationBlockType3 and SystemInformationBlockType5 in the system information and transmits a Paging message including a systemInfoModification. | - | - | - | - |
| 3 | The SS changes Cell 1, Cell 28 and Cell 30 power level according to the row "T2" in table 6.1.2.16.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure with PLMN2 and PLMN3 listed as Equivalent PLMN list in the TRACKING AREA UPDATE ACCEPT message, and the RRC connection is released. | - | - | 2 |  |

6.1.2.16.3.3	Specific message contents

Table 6.1.2.16.3.3-0: Conditions for specific message contents
in Tables 6.1.2.16.3.3-1 and 6.1.2.16.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 28 | This condition applies to system information transmitted on Cell 28. |
| Cell 30 | This condition applies to system information transmitted on Cell 30. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.16.3.3-1: SystemInformationBlockType3 for Cell 1 and Cell 30 (preamble and step 1 and 2, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  | Cell 1 |
|  | 5 |  | Cell 30 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.16.3.3-2: SystemInformationBlockType3 for Cell 1, Cell 28 and Cell 30 (step 3 and step 4, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  | Cell 1 |
|  | 4 |  | Cell 28 |
|  | 3 |  | Cell 30 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.16.3.3-3: SystemInformationBlockType5 for Cell 1 and Cell 30 (preamble and step 1 and 2, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 30 |  | Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 30 |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 5 |  | Cell 1 |
|  | 5 |  | Cell 30 |
| threshX-High | 10 | 20dB |  |
| cellReselectionPriority | 5 |  | Cell 1 |
|  | 3 |  | Cell 30 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 30 |  | Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.16.3.3-4: SystemInformationBlockType5 for Cell 1, Cell 28, and Cell 30 (step 3 and 4, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 28 |  | Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 28, Cell 30 |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA[n] | 7 |  | Cell 1 |
|  | 5 |  | Cell 30 |
| threshX-High[n] | 10 | 20dB |  |
| cellReselectionPriority[n] | 4 |  | Cell 1 |
|  | 5 |  | Cell 28, Cell 30 |
| dl-CarrierFreq[m] | Same downlink EARFCN as used for Cell 30 |  | Cell 1, Cell 28 |
|  | Same downlink EARFCN as used for Cell 28 |  | Cell 30 |
| dl-CarrierFreq[m] | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA[m] | 5 |  | Cell 1 |
|  | 7 |  | Cell 30 |
| threshX-High[m] | 10 | 20dB |  |
| cellReselectionPriority[m] | 3 |  | Cell 1, Cell 28 |
|  | 4 |  | Cell 30 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 28 |  | Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 28, Cell 30 |
| dl-CarrierFreq-v9e0[m] | Same downlink EARFCN as used for Cell 30 |  | Cell 1, Cell 28 |
| } | Same downlink EARFCN as used for Cell 28 |  | Cell 30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.16.3.3-5: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2,3 |  |  |

Table 6.1.2.16.3.3-6: TRACKING AREA UPDATE ACCEPT for Cell 30 (step 2, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 2 |  |  |

Table 6.1.2.16.3.3-7: TRACKING AREA UPDATE ACCEPT for Cell 1 (step 4, Table 6.1.2.16.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2, 3 |  |  |

#### 6.1.2.17	Cell reselection for Squal to check against SIntraSearchQ and SnonIntraSearchQ

6.1.2.17.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { SIntraSearchQ is non-zero in system information }

then { UE perform measurement and reselects the highest ranked cell upon Squal < SIntraSearchQ }

}

(2)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { SnonIntraSearchQ is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the equal priority frequency cell upon Squal < SnonIntraSearchQ }

}

(3)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { SnonIntraSearchQ is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the high priority frequency cell upon Squal > SnonIntraSearchQ }

}

6.1.2.17.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.304, clauses 5.2.4.2, 5.2.4.5 and 5.2.4.6;  TS 36.331, clause 6.3.1.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

…..

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

6.1.2.17.3	Test description

6.1.2.17.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 3 have different tracking areas according to table 6.0.1-2.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.17.3.2	Test procedure sequence

Table 6.1.2.17.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.17.3.2-2.

Table 6.1.2.17.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | -115 | Squal of Cell 1 is less than SIntraSearchQ. |
|  | RSRQ | dB | -11.68 | -5.68 | -30.8 |  |
|  | Qrxlevmin | dBm | -106 | -106 | -106 |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Noc | dBm/15kHz | -95 | -95 | -95 |  |
|  | Srxlev | dB | 15 | 21 | -9 |  |
|  | Squal | dB | 8.32 | 14.32 | -10.8 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 | -91 | -85 | Squal of Cell 2 is less than SnonIntraSearchQ. |
|  | RSRQ | dB | -32.32 | -8.32 | -5.05 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Srxlev | dB | -9 | 15 | 21 |  |
|  | Squal | dB | -12.32 | 11.68 | 14.95 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -115 | -85 | Squal of Cell 3 is greater than SnonIntraSearchQ but Cell 1 is high priority cell. |
|  | RSRQ | dB | [-6] | [-25] | [-6] |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 2 |  |
|  | Srxlev | dB | 21 | -9 | 21 |  |
|  | Squal | dB | 14.95 | -15.05 | 14.95 |  |
| Note: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 6.1.2.17.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.17.3.2-1. | - | - | - | - |
| 2 | Check: Does UE transmits an RRCConnectionRequest message on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 3 - 7 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 8 | The SS re-adjusts the cell-specific reference signal levels according to row "T2" in table 6.1.2.17.3.2-1. | - | - | - | - |
| 9 | Check: Does UE transmits an RRCConnectionRequest message on Cell 3? | --> | RRCConnectionRequest | 2 | P |
| 10 - 14 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 3. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 15 | The SS waits 1 second. | - | - | - | - |
| 16 | The SS notifies the UE of change of System Information and changes the system information for Cell 1, 2 & 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 17 | Void | - | - | - | - |
| 18 | The SS re-adjusts the cell-specific reference signal levels according to row "T3" in table 6.1.2.13.3.2-1. | - | - | - | - |
| 19 | Check: Does the test result of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? (Note1) NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | - |
| Note1: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.1.2.17.3.3	Specific message contents

Table 6.1.2.17.3.3-0: Conditions for specific message contents
in Table 6.1.2.17.3.3-2A and Table 6.1.2.17.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.17.3.3-1: SystemInformationBlockType1 for Cells 1, 2 and 3 (preamble, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-1A: SystemInformationBlockType1-BR-r13 for Cells 1, 2 and 3 (preamble when UE under test is CAT M1, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-2: SystemInformationBlockType3 for Cells 1, 2 and 3 (preamble, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |  |  |
| s-IntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-IntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-IntraSearchQ-r9 |  |  | 20 dB |  |  |
| } |  |  |  |  |  |
| s-NonIntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-NonIntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-NonIntraSearchQ-r9 |  |  | 20 dB |  |  |
| } |  |  |  |  |  |
| q-QualMin-r9 |  |  | -20 dB |  |  |
| } |  |  |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-2A: SystemInformationBlockType5 for Cell 2 (preamble, Table 6.1.2.17.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-3: SystemInformationBlockType1 for Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-3AA: SystemInformationBlockType1-BR-r13 for Cell 3 (step 16 when UE under test is CAT M1, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3A: SystemInformationBlockType3 for Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |  |  |
| s-NonIntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-NonIntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-NonIntraSearchQ-r9 |  |  | 2 dB |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-4: SystemInformationBlockType5 for Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| threshX-High[1] | 7 | 14 dB |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-4A: SystemInformationBlockType3 for Cell 1 and 2 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 0 (0 dB) |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-5: Paging (step 16, Table 6.1.2.17.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-6: SystemInformationBlockType1 for Cell 1, Cell 2 and Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| systemInfoValueTag | 1 | Previous value is 0 |  |

Table 6.1.2.17.3.3-6A: SystemInformationBlockType1-BR-r13 for Cell 1, Cell 2 and Cell 3 (step 16 when UE under test is CAT M1, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| systemInfoValueTag | 1 | Previous value is 0 |  |

#### 6.1.2.18	Inter-frequency Cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ

6.1.2.18.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority frequency }

then { UE reselects the cell which belongs to the higher priority frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority frequency }

then { UE reselects the cell which belongs to the lower priority frequency }

}

6.1.2.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

…..

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

6.1.2.18.3	Test description

6.1.2.18.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.18.3.2	Test procedure sequence

Table 6.1.2.18.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.18.3.2-2.

Table 6.1.2.18.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | -85 | The power level values are assigned to satisfy SrxlevCell 1 > 0, SrxlevCell 3 < ThreshCell 3, highP, SqualCell 1 > 0 and SqualCell 3 > ThreshCell 3, highQ. |
|  | RSRQ | dB | -3 | -3 |  |
|  | Qqualmin | dB | -20 | -20 |  |
|  | Qrxlevmin | dBm | -100 | -100 |  |
|  | Noc | dBm/15kHz | off | off |  |
| T2 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | -91 | The power level values are assigned to satisfy SrxlevCell 3 > Threshserving, lowP, SqualCell 3 < Threshserving, lowQ and SqualCell 1 > ThreshCell 1, lowQ. |
|  | RSRQ | dB | -3 | -3 |  |

Table 6.1.2.18.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS changes Cell 3 power level according to the row "T1" in table 6.1.2.18.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 1 | - |
| 2A | The SS notifies the UE of change of System Information and changes the system information 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 3 | The SS changes Cell 3 power level according to the row "T2" in table 6.1.2.18.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.1.2.18.3.3	Specific message contents

Table 6.1.2.18.3.3-1: Conditions for specific message contents
in Tables 6.1.2.18.3.3-2 and 6.1.2.18.3.3-3

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.18.3.3-2: SystemInformationBlockType3 for Cell 1 and Cell 3 (preamble, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 1 | 2 dB | Cell 3 |
| cellReselectionPriority | 5 |  | Cell 3 |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -50 (-100 dBm) |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  | Cell 1 |
|  | 4 dB |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-3: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| q-RxLevMin[1] | -50 (-100 dBm) |  |  |
| t-ReselectionEUTRA[1] | 7 |  |  |
| threshX-High[1] | 10 (20 dB) |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| q-QualMin-r9[1] | -20 dB |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9[1] | 4 | 4 dB |  |
| threshX-LowQ-r9[1] | 2 | 2 dB |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | EARFCN of Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-3a: SystemInformationBlockType5 for Cell 3 (preamble and all steps, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 1 |  |  |
|  | maxEARFCN |  | Band > 64 |
| q-RxLevMin[1] | -50 (-100 dBm) |  |  |
| t-ReselectionEUTRA[1] | 7 |  |  |
| cellReselectionPriority[1] | 4 |  |  |
| q-QualMin-r9[1] | -20 dB |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9[1] | 4 | 4 dB |  |
| threshX-LowQ-r9[1] | 2 | 2 dB |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | EARFCN of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-4: SystemInformationBlockType3 for Cell 3 (Step 2A, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-5: SystemInformationBlockType1 for Cell 3 (Step 2A, Table 6.1.2.18.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| systemInfoValueTag | 1 | Previous value is 0 |  |

#### 6.1.2.19	Intra-frequency Cell reselection / MFBI

6.1.2.19.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN Intra-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving

cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

6.1.2.19.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2 and 5.2.4.6, TS 36.331, clause 5.2.2.7 and 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell, x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

…

SystemInformationBlockType2 field descriptions

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1, listed in the same order.

6.1.2.19.3	Test description

6.1.2.19.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2 and are MFBI capable cells.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	Cell 2 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.19.3.2	Test procedure sequence

Table 6.1.2.19.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.19.3.2-2.

Table 6.1.2.19.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are set so that RCell 1 < RCell 2. |

Table 6.1.2.19.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.19.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.19.3.3	Specific message contents

Table 6.1.2.19.3.3-0: Conditions for specific message contents
in Table 6.1.2.19.3.3-1

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |
| mFBI band > 64 | mFBI band > 64 is selected |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.19.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.1.2.19.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Cell 1 Cell 2 |
| freqBandIndicator |  | 64 | Band > 64 Cell 1 Cell 2 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| freqBandIndicator |  | 64 | mFBI band > 64 Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Band > 64 Cell 1 Cell 2 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  | mFBI band > 64 |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.19.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 1 and Cell 2 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.19.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Cell 1 Cell 2 |
| freqBandIndicator |  | 64 | Band > 64 Cell 1 Cell 2 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| freqBandIndicator |  | 64 | mFBI band > 64 Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Band > 64 Cell 1 Cell 2 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  | mFBI band > 64 |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.19.3.3-2: SystemInformationBlockType2 for Cell 1 and Cell 2 (preamble and all steps, Table 6.1.2.19.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.20	Inter-frequency Cell reselection / MFBI

6.1.2.20.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN inter-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving }

then { the UE reselects the new cell }

}

6.1.2.20.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.6 and TS 36.331, clause 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

…

SystemInformationBlockType5 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common. E-UTRAN indicates at most maxMultiBands frequency bands (i.e. the total number of entries across both multiBandInfoList and multiBandInfoList-v9e0 is below this limit).

6.1.2.20.3	Test description

6.1.2.20.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 have different tracking areas according to table 6.0.1-2 and are MFBI capable cells.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	Cell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.20.3.2	Test procedure sequence

Table 6.1.2.20.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.20.3.2-2.

Table 6.1.2.20.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are set so that RCell 1 < RCell 2. |

Table 6.1.2.20.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.20.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.20.3.3	Specific message contents

Table 6.1.2.20.3.3-0: Conditions for specific message contents
in Tables 6.1.2.19.3.3-1 and 6.1.2.19.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| mFBI band > 64 | mFBI band > 64 is selected |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.20.3.3-1: SystemInformationBlockType1 for cell 1 and cell 3 (preamble)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Cell 1 Cell 3 |  |
| freqBandIndicator |  |  | 64 |  | Band > 64 Cell 1 Cell 3 |  |
| lateNonCriticalExtension |  |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An Overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 1 Cell 3 |  |
| freqBandIndicator |  |  | 64 |  | mFBI band > 64 Cell 1 Cell 3 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Band > 64 Cell 1 Cell 3 |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |  | mFBI band > 64 |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_MFBI_FrequencyBand) |  | Cell 1 Cell 3 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 6.1.2.20.3.3-1AA: SystemInformationBlockType1-BR-r13 for cell 1 and cell 3 (preamble when UE under test is CAT M1)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Cell 1 Cell 3 |  |
| freqBandIndicator |  |  | 64 |  | Band > 64 Cell 1 Cell 3 |  |
| lateNonCriticalExtension |  |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An Overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 1 Cell 3 |  |
| freqBandIndicator |  |  | 64 |  | mFBI band > 64 Cell 1 Cell 3 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Band > 64 Cell 1 Cell 3 |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |  | mFBI band > 64 |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_MFBI_FrequencyBand) |  | Cell 1 Cell 3 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 6.1.2.20.3.3-1A: SystemInformationBlockType2 for Cell 1 and Cell3 (preamble and all steps, Table 6.1.2.20.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.20.3.3-2: SystemInformationBlockType5 for cell 1 and cell 3 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq |  | 65535 | mFBI band > 64 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList-v8h0 SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v8h0 SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An Overlapping Bandunder test (px_MFBI_FrequencyBand) | Cell 1 Cell 3 |
| freqBandIndicator |  | 64 | mFBI band > 64 Cell 1 Cell 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v9e0-IEs SEQUENCE { |  |  | mFBI band > 64 |
| interFreqCarrierFreqList-v9e0 SEQUENCE { |  |  |  |
| interFreqCarrierFreqInfo-v9e0 SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 |  | Downlink EARFCN under test | Cell 1 Cell 3 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand) | Cell 1 Cell 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.1.2.21	Inter-band Cell reselection / MFBI

6.1.2.21.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN another band neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

6.1.2.21.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5 and 5.2.4.6; TS 36.331, clause 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.5]

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

…

SystemInformationBlockType5 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common. E-UTRAN indicates at most maxMultiBands frequency bands (i.e. the total number of entries across both multiBandInfoList and multiBandInfoList-v9e0 is below this limit).

6.1.2.21.3	Test description

6.1.2.21.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10 have different tracking areas according to table 6.0.1-2.

-	Cell 10 is MFBI capable cell.

-	The E-UTRA Primary frequency band under test (px_ePrimaryFrequencyBand) is supported by the UE.

-	Cell 10 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.21.3.2	Test procedure sequence

Table 6.1.2.21.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.21.3.2-2.

Table 6.1.2.21.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | Cell 10 become stronger than Threshx, high. |

Table 6.1.2.21.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.21.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10 belonging to the MFBI band (px_MFBI_FrequencyBand)? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.21.3.3	Specific message contents

Table 6.1.2.21.3.3-0: Conditions for specific message contents
in Tables 6.1.2.21.3.3-1, 6.1.2.21.3.3-3 and 6.1.2.21.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| mFBI band > 64 | mFBI band > 64 is selected |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.21.3.3-1: SystemInformationBlockType1 for Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  | Cell 10 |  |
| freqBandIndicator |  | 64 |  | Band > 64 Cell 10 |  |
| lateNonCriticalExtension |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 10 |  |
| freqBandIndicator |  | 64 |  | mFBI band > 64 Cell 10 |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Band > 64 Cell 10 |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  | mFBI band > 64 |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand) |  | Cell 10 |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 6.1.2.21.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 10 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  | Cell 10 |  |
| freqBandIndicator |  | 64 |  | Band > 64 Cell 10 |  |
| lateNonCriticalExtension |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 10 |  |
| freqBandIndicator |  | 64 |  | mFBI band > 64 Cell 10 |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI) |  | Band > 64 Cell 10 |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  | mFBI band > 64 |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand) |  | Cell 10 |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 6.1.2.21.3.3-2: SystemInformationBlockType2 for Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.21.3.3-3: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.21.3.3-4: SystemInformationBlockType5 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| dl-CarrierFreq |  | Same downlink EARFCN as used for Cell 10 |  | Cell 1 |
|  |  | Same downlink EARFCN as used for Cell 1 |  | Cell 10 |
|  |  | 65535 |  | mFBI band > 64 |
| t-ReselectionEUTRA |  | 7 |  |  |
| threshX-High |  | 10 | 20dB |  |
| cellReselectionPriority |  | 5 |  | Cell 1 |
|  |  | 1 |  | Cell 10 |
| } |  |  |  |  |
| lateNonCriticalExtension |  |  |  |  |
| SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { |  |  |  |  |
| InterFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| InterFreqCarrierFreqInfo-v8h0 [n] SEQUENCE { |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |
| freqBandIndicator |  | An overlapping Band for Cell 10 under test (px_MFBI_FrequencyBand). |  | Cell 10 |
| freqBandIndicator |  | 64 |  | mFBI band > 64 Cell 10 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| SystemInformationBlockType5-v9e0-IEs SEQUENCE { |  |  |  | mFBI band > 64 |
| interFreqCarrierFreqList-v9e0 SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqInfo-v9e0 SEQUENCE { |  |  |  |  |
| dl-CarrierFreq-v9e0 |  | Downlink EARFCN under test |  | Cell 10 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand) |  | Cell10 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 6.1.2.22	Cell reselection / MFBI / UE does not support multiBandInfoList

6.1.2.22.1	Test Purpose (TP)

(1)

with { an MFBI non-capable UE in E-UTRA RRC_IDLE state }

ensure that {

when { upon reception of the SIB1 from MFBI cell the frequency band indicated in freqBandIndicator is not supported by UE and the frequency band indicated in freqBandIndicator under multiBandInfoList is supported by UE}"

then { the UE does not reselect to the cell}

}

6.1.2.22.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2 and 5.2.4.6, TS 36.331, clause 5.2.2.7 and 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 message the UE shall:

1>	if in RRC_CONNECTED while T311 is not running and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4] and;

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

6.1.2.22.3	Test description

6.1.2.22.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 have different tracking areas according to table 6.0.1-2.

-	Cell 3 is a MFBI capable cell.

-	Cell 3 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.22.3.2	Test procedure sequence

Table 6.1.2.22.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.22.3.2-2.

Table 6.1.2.22.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are set so that RCell 1 < RCell 3. |

Table 6.1.2.22.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.22.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 3 within the next 60 s? | - | - | 1 | F |

6.1.2.22.3.3	Specific message contents

Table 6.1.2.22.3.3-0: Conditions for specific message contents
in Table 6.1.2.21.3.3-1

| Condition | Explanation |
| --- | --- |
| mFBI band > 64 | mFBI band > 64 is selected |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.22.3.3-1: SystemInformationBlockType1 for cell 3 (preamble)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |  |
|  |  |  | 64 |  | Band > 64 |  |
| lateNonCriticalExtension |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  |  | An Overlapping Band under test (px_MFBI_FrequencyBand). |  |  |
|  |  |  | 64 |  | mFBI_Band > 64 |
| } |  |  |  |  |  |
| nonCriticalExtension { |  |  |  |  | Band > 64 |
| SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |  |  |
| MultiBandInfo-v9e0 SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  | mFBI_Band > 64 |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension |  |  | Not present |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 6.1.2.22.3.3-1A: SystemInformationBlockType1-BR-r13 for cell 3 (preamble when UE under test is CAT M1)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |  |
|  |  |  | 64 |  | Band > 64 |  |
| lateNonCriticalExtension |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  |  | An Overlapping Band under test (px_MFBI_FrequencyBand). |  |  |
|  |  |  | 64 |  | mFBI_Band > 64 |
| } |  |  |  |  |  |
| nonCriticalExtension { |  |  |  |  | Band > 64 |
| SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |  |  |
| MultiBandInfo-v9e0 SEQUENCE { |  |  |  |  |  |
| freqBandIndicator-v9e0 |  |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  | mFBI_Band > 64 |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension |  |  | Not present |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

#### 6.1.2.23	Inter-band cell reselection / MFBI frequency band priority adjustment/Inter-band CA

6.1.2.23.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and support MFBI frequency band priority adjustment }

ensure that {

when { UE detects the cell re-selection criteria are met for the MFBI cell on the different frequency band, and both the bands in the FreqBandIndicator and the multiBandInfoList are supported by UE and the freqBandIndicatorPriority in SystemInformationBlockType1 is set to true }

then { the UE camped on the new cell on the band indicated by MultiBandInfoList}

}

6.1.2.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5, TS 36.331, clause 5.2.2.7 and 6.3.1.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1>	if the UE is a category 0 UE according to TS 36.306 [5]; and

1>	if category0Allowed is not included in SystemInformationBlockType1:

2>	consider the cell as barred in accordance with TS 36.304 [4];

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

3>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

5>	apply the additionalPmax;

4>	else:

5>	apply the p-Max;

3>	else:

4>	apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

…

SystemInformationBlockType1 field descriptions

freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList IE in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator IE. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList.

multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator IE it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList IE. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE.

6.1.2.23.3	Test description

6.1.2.23.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10 have different tracking areas according to table 6.0.1-2.

-	Cell 1 is a SCell but not a MFBI capable cell, Cell 10 is a MFBI capable cell and a PCell also.

-	Cell 1 belongs to the absolute centre frequency which is not overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI.

-	Cell 10 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI.

-	Cell 1 uses the E-UTRA Primary frequency band under test (px_ePrimaryFrequencyBand) which together with px_OverlappingSupportedFrequencyBand_MFBI belongs to an E-UTRA CA Configuration for CA Inter-band operation supported by the UE, px_ePrimaryFrequencyBand and different is value from px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI.

-	Cell 1 frequency f1 and Cell 10 frequency f5 are initialised as defined in TS 36.508[18] Table6.2.3.4-1.

-	It is ensured that there is no CA band combination with bands px_ePrimaryFrequencyBand and px_MFBI_FrequencyBand.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	UE support px_ePrimaryFrequencyBand, px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI bands.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.23.3.2	Test procedure sequence

Table 6.1.2.23.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.23.3.2-2.

Table 6.1.2.23.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are set so that RCell 1 < RCell 10. |

Table 6.1.2.23.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.23.3.2-1. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message on cell 10. | --> | RRC: RRCConnectionRequest | - | - |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |
| 6 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer, | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of data radio bearer. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 10 | The UE send a TRACKING AREA UPDATE COMPLETE on the cell 10. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with SCell Cell 1 addition. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message?(Note1) | --> | RRCConnectionReconfigurationComplete | 1 | P |
| Note1: For steps 2-10, a conformant UE having selected the band in the MultiBandInfoList does the inter-Band CA at step12, which a non-conformant UE will not. |  |  |  |  |  |

6.1.2.23.3.3	Specific message contents

Table 6.1.2.23.3.3-0: Conditions for specific message contents
in Tables 6.1.2.23.3.3-1, 6.1.2.23.3.3-1B, 6.1.2.23.3.3-3 and 6.1.2.23.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.23.3.3-1: SystemInformationBlockType1 for Cell 10 (preamble and all steps, Table 6.1.2.23.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand)). | Cell 10 |
| freqBandIndicator |  | 64 | Band > 64 Cell 10 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE (SIZE (1..maxMultiBands)) OF SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_OverlappingSupportedFrequencyBand_MFBI). | Cell 10 |
| freqBandIndicator |  | 64 | mFBI_Band > 64 Cell 10 |
| } |  |  |  |
| SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Band > 64 Cell 10 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |
| MultiBandInfo-v9e0 SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingSupportedFrequencyBand_MFBI). | mFBI_Band > 64 Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v920-IEs ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v1250-IEs ::= SEQUENCE { |  |  |  |
| freqBandIndicatorPriority-r12 |  | true | Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.23.3.3-1A: SystemInformationBlockType2 for Cell 10 (preamble and all steps, Table 6.1.2.23.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |
| lateNonCriticalExtension |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |
| multiBandInfoList SEQUENCE { |  |  |
| AdditionalSpectrumEmission | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |
| } |  |  |
| } |  |  |

Table 6.1.2.23.3.3-1B: SystemInformationBlockType5 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.23.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  | Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Cell 10 |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 Cell 1 |
|  | maxEARFCN |  | Band > 64 Cell 10 |
| } |  |  |  |
| lateNonCriticalExtension |  |  | Cell 1 |
| SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { |  |  | Cell 1 |
| InterFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Cell 1 |
| InterFreqCarrierFreqInfo-v8h0 [n] SEQUENCE { |  |  | Cell 1 |
| multiBandInfoList SEQUENCE { |  |  | Cell 1 |
| freqBandIndicator | An overlapping Band for Cell 10 under test (px_OverlappingSupportedFrequencyBand_MFBI). |  | Cell 1 |
| freqBandIndicator | maxFBI |  | Band > 64 Cell 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| InterFreqCarrierFreqList-v9e0 SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  | Band > 64 Cell 1 |
|  | Same downlink EARFCN as used for Cell 1 |  | Band > 64 Cell 10 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  |  |
| MultiBandInfo-v9e0 SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 | An overlapping Band for Cell 10 under test (px_OverlappingSupportedFrequencyBand_MFBI). |  | Band > 64 Cell 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.23.3.3-1C: RRCConnectionReconfiguration (step 7, Table 6.1.2.23.3.2-2)

Derivation Path: TS 36.508 Table 4.6.1-8, condition DRB(1, 0)

Table 6.1.2.23.3.3-2: RRCConnectionReconfiguration (step 11, Table 6.1.2.23.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sCellToReleaseList-r10 |  | Not present |  |  |
| sCellToAddModList-r10 |  | SCellToAddMod-r10-f2-Add | SCell addition for Cell 1 |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.23.3.3-3: SCellToAddMod-r10-f2-Add (Table 6.1.2.23.3.3-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10 | 1 |  |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell1 |  |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  |  | Band > 64 |
| } |  |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  | Band > 64 |
| } |  |  |  |  |  |

Table 6.1.2.23.3.3-4: RadioResourceConfigCommonSCell-r10-f2 (Table 6.1.2.23.3.3-3)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r10 | Same uplink EARFCN as used for Cell 1 | OP | FDD |
|  | maxEARFCN |  | FDD and Band > 64 |
|  | Not present |  | TDD |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 1 | optional | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-CarrierFreq-v1090 | Same uplink EARFCN as used for Cell 1 |  | FDD and Band > 64 |
| } |  |  |  |

Table 6.1.2.23.3.3-5: RadioResourceConfigDedicatedSCell-r10-f2 (Table 6.1.2.23.3.3-3)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 6.2	Multi-mode environment (E-UTRAN, UTRAN, GERAN, CDMA2000)

### 6.2.1	Inter-RAT PLMN selection

#### 6.2.1.1	Inter-RAT PLMN Selection / Selection of correct RAT for OPLMN / Automatic mode

6.2.1.1.1	Test Purpose (TP)

(1)

with { UE pre-set in Automatic network selection mode }

ensure that {

when { UE is switched on and there are suitable E-UTRAN and UTRAN cells some on the OPLMN list and some not on the OPLMN list, none of them being part of the RPLMN }

then { UE selects the highest priority OPLMN and RAT combination, attaches on the selected cell and when successfully registered indicates the PLMN to the user }

}

(2)

with { UE in Automatic network selection mode }

ensure that {

when { UE returns to coverage and there are suitable E-UTRAN and GERAN cells some on the OPLMN list and some not on the OPLMN list , none of them being part of the RPLMN  }

then { UE selects the highest priority OPLMN and RAT combination, attaches on the selected cell and when successfully registered indicates the PLMN to the user }

}

(3)

with { UE in Automatic network selection mode }

ensure that {

when { UE returns to coverage and there are suitable E-UTRAN and UTRAN cells some on the OPLMN list and some not on the OPLMN list , none of them being part of the RPLMN  }

then { UE selects the highest priority OPLMN and RAT combination, attaches on the selected cell and when successfully registered indicates the PLMN to the user }

}

6.2.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1 and 4.4.3.1.1.

[TS 23.122, clause 4.4.3.1]

If successful registration is achieved, the MS indicates the selected PLMN.

...

NOTE 1: 	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

6.2.1.1.3	Test description

6.2.1.1.3.1	Pre-test conditions

System Simulator

5 cells are used maximum 3 are operating at the same time:

-	3 cells belong to E-UTRAN (Cell 1, Cell 2 and Cell 3)

-	IF pc_GERAN 1 cell belongs to GERAN (Cell 24)

-	1 cell belongs to UTRAN (Cell 9).

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE

- The UE is in Automatic PLMN selection mode.

- The HPLMN is PLMN1

- There is no RPLMN.

Table 6.2.1.1.3.1-1: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT |  | 3GPP TS 31.102, Annex E |  | The EF is empty. |
| EFOPLMNwAcT | 1 | PLMN2 | E-UTRAN |  |
|  | 2 | PLMN13 | GERAN |  |
|  | 3 | PLMN13 | UTRAN |  |
|  | 4 | PLMN13 | E-UTRAN |  |

Table 6.2.1.1.3.1-2: Void

Preamble

-	The UE is in state Switched OFF (State 1)

6.2.1.1.3.2	Test procedure sequence

Tables 6.2.1.1.3.2-1 and 2 show the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configuration marked "T1" to "T5" are applied at the point indicated in the Main behaviour description in Table 6.2.1.1.3.2-3. Cell powers for the E-UTRAN, UTRAN and GERAN cells are defined in TS 36.508 Table 6.2.2.1-1.

Table 6.2.1.1.3.2-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN2 |
| 9 | PLMN13 |
| 3 | PLMN14 |
| 24 | PLMN13 |
| 2 | PLMN13 |

Table 6.2.1.1.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1 | Cell 2* | Cell 3* | Cell 9 | Cell 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | Serving cell | OFF | Serving cell |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | Serving cell |  |
|  | RF Signal Level | dBm |  |  |  |  | OFF |
| T1 | RS EPRE | dBm/15kHz | OFF | Serving cell | Serving cell |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | Serving cell |  |
|  | RF Signal Level | dBm |  |  |  |  | OFF |
| T2 | RS EPRE | dBm/15kHz | Serving cell | OFF | OFF |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | OFF |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | OFF |  |
|  | RF Signal Level | dBm |  |  |  |  | OFF |
| T3 | RS EPRE | dBm/15kHz | Serving cell | OFF | Serving cell |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | OFF |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | OFF |  |
|  | RF Signal Level | dBm |  |  |  |  | Serving cell |
| T4 | RS EPRE | dBm/15kHz | OFF | Serving cell | Serving cell |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | OFF |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | OFF |  |
|  | RF Signal Level | dBm |  |  |  |  | Serving cell |
| T5 | RS EPRE | dBm/15kHz | OFF | -115 | Serving cell |  |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  |  | OFF |  |
|  | P-CCPCH | dBm/1.28 MHz |  |  |  | OFF |  |
|  | RF Signal Level | dBm |  |  |  |  | Serving cell |
| *Only minimum uplink cell configuration required (See 6.3.3.2 in TS 36.508) |  |  |  |  |  |  |  |

Table 6.2.1.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 3-17 | Steps 3-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1 take place. All messages are observed on Cell 1. | - | - | - | - |
| 18 | Check: Is PLMN2 indicated by the UE as the PLMN on which the UE is camped? | - | - | 1 | P |
| 19 | SS adjusts cell levels according to row T1 of table 6.2.1.1.3.2-2. | - | - | - | - |
| 19A | Check: Does the UE send RRC CONNECTION REQUEST on Cell 9? | --> | RRC CONNECTION REQUEST | 3 | P |
| 19B-19J | Steps 2-10 of the routing area update procedure described in TS 36.508 Table 6.4.2.8-1 take place. All messages are observed on Cell 9. | - | - | - | - |
| - | Exception: Steps 19JAa1 to 19JAa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 19JAa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 19JAa1a1-19JAa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 19JAa1a4 | SS releases RRC Connection | - | - | - | - |
| 19JAa1a5 | SS Stops 6s timer | - | - | - | - |
| 19JAa1b1 | The 6s timer expires | - | - | - | - |
| 19K | Check: Is PLMN13 indicated by the UE as the PLMN on which the UE is camped? | - | - | 3 | P |
| - | EXCEPTION: Steps 20a1 to 20a32 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place if the UE supports GERAN. |  |  |  |  |
| 20a1 | IF pc_GERAN THEN SS adjusts cell levels according to row T2 of table 6.2.1.1.3.2-2. | - | - | - | - |
| 20a2 | The UE sends RRCConnectionRequest on Cell 1 | --> | RRCConnectionRequest | - | - |
| 20a3-20a9 | Steps 2-8 of generic test procedure in TS 36.508 Table 6.4.2.7A-1 take place (the UE performs a TAU procedure and the RRC connection is released). All messages are observed on Cell 1. | - | - | - | - |
| 20a10 – 20a17 | Void | - | - | - | - |
| 20a18 | SS adjusts cell levels according to row T3 of table 6.2.1.1.3.2-2. | - | - | - | - |
| 20a19 | Wait for 1s to allow the stable cell condition before the next cell adjustments and catch any not allowed UE behaviour (the UE is expected not to do anything). | - | - | - | - |
| 20a20 | SS adjusts cell levels according to row T4 of table 6.2.1.1.3.2-2. | - | - | - | - |
| 20a21 | Check: Does the UE send a CHANNEL REQUEST on Cell 24? | --> | CHANNEL REQUEST | 2 | P |
| 20a21A | SS adjusts cell levels according to row T5 of table 6.2.1.1.3.2-2. | - | - | - | - |
| 20a22-20a31 | Steps 2-11 of the routing area update procedure described in TS 36.508 table 6.4.2.9-1 take place. All messages are observed on Cell 24. | - | - | - | - |
| 20a32 | Check: Is PLMN13 indicated by the UE? | - | - | 2 | P |
| 21-46 | Void | - | - | - | - |

6.2.1.1.3.3	Specific message contents

Table 6.2.1.1.3.3-1: System Information Block type 19 for Cell 9 (preamble and all steps from step 1 up to step 20a1, Table 6.2.1.1.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |  |
| v920NonCriticalExtensions | Not present |  |  |
| } |  |  |  |

Table 6.2.1.1.3.3-2: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 (step 20a2 onwards, Table 6.2.1.1.3.2-3)

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Repeated E-UTRAN Neighbour Cells | 0 | Not Present |  |

#### 6.2.1.2	Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode

6.2.1.2.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and UPLMN and OPLMN cells available on E-UTRAN and UTRAN}

ensure that {

when { UE is switched on}

then { UE selects a the highest priority UPLMN and RAT combination and UE attempts to attach on the selected cell and when successfully registered indicates the PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and UPLMN and OPLMN cells available on E-UTRAN and UTRAN}

ensure that {

when { UE returns to coverage}

then { UE selects a the highest priority VPLMN and RAT combination and UE attempts to attach on the selected cell and when successfully registered indicates the PLMN to the user. }

}

6.2.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1 and 4.4.3.1.1.

[TS 23.122, clause 4.4.3.1]

If successful registration is achieved, the MS indicates the selected PLMN.

...

NOTE 1: 	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

6.2.1.2.3	Test description

6.2.1.2.3.1	Pre-test conditions

System Simulator

-	3 cells are used:  2 cells belong to E-UTRAN  and 1 cell belongs to UTRAN. The cells are configured as specified in TS 36.508 clause 4.4.1.4 and Table 4.4.2-2.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE

- The UE is in Automatic PLMN selection mode.

- The HPLMN is PLMN1

- There is no RPLMN.

Table 6.2.1.2.3.1–1: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN13 | E-UTRAN |  |
|  | 2 | PLMN14 | UTRAN |  |
| EFOPLMNwAcT | 1 | PLMN2 | All |  |
|  | 2 | PLMN14 | UTRAN |  |
|  | 3 | PLMN13 | E-UTRAN |  |

Preamble

-	The UE is in state Switched OFF (State 1)

-	Cell levels are set according to row T0 of table 6.2.1.2.3.2-2.

6.2.1.2.3.2	Test procedure sequence

Tables 6.2.1.2.3.2 – 1 and 2 show the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configuration marked “T1” is applied at the point indicated in the Main behaviour description in Table 6.2.1.2.3.2-3. Cell powers for the E-UTRAN cell are defined in  TS 36.508 Table 6.2.2.1-1 and cell powers for the UTRAN cells are defined in TS 36.508 Table 6.2.2.1-1.

Table 6.2.1.2.3.2-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN2 |
| 9 | PLMN14 |
| 12 | PLMN13 |

Table 6.2.1.2.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1* | Cell 9 | Cell 12 |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | Serving cell |  | Serving cell |
|  | CPICH_Ec | dBm/3.84 MHz |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  | Serving cell |  |
| T1 | RS EPRE | dBm/15kHz | Serving cell |  | Non-suitable “Off” |
|  | CPICH_Ec | dBm/3.84 MHz |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  | Serving cell |  |
| *Only minimum uplink cell configuration required (See 6.3.3.2 in TS 36.508) |  |  |  |  |  |

Table 6.2.1.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send RRCConnectionRequest on Cell 12? | --> | RRCConnectionRequest | 1 | P |
| 3-17 | The UE performs Steps 3-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1. | - | - | - | - |
| 18 | Check: Is PLMN13 indicated by the UE? | - | - | 1 | P |
| 19 | SS adjusts cell levels according to row T1 of table 6.2.1.2.3.2-2. | - | - | - | - |
| 20 | Check: Does the UE send a RRC CONNECTION REQUEST on Cell 9? | --> | RRC CONNECTION REQUEST | 2 | P |
| 21-29 | The UE performs Steps 2-10 of the routing area update procedure described in TS 36.508 Table 6.4.2.8-1. | - | - | - | - |
| - | Exception: Steps 29Aa1 to 29Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 29Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 29Aa1a1-29Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 29Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 29Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 29Aa1b1 | The 6s timer expires | - | - | - | - |
| 30 | Check: Is PLMN14 indicated by the UE? | - | - | 2 | P |

6.2.1.2.3.3	Specific message contents

Table 6.2.1.2.3.3-1: System Information Block type 19 for Cell 9 (preamble and all steps, Table 6.2.1.2.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |  |

#### 6.2.1.3	Inter-RAT PLMN Selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode

6.2.1.3.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and shared OPLMN cells available on E-UTRAN and UTRAN }

ensure that {

when { UE is switched on}

then { UE selects a the highest priority OPLMN and RAT combination and UE attempts to attach on the selected cell and PLMN and when successfully registered indicates the correct PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and shared OPLMN cells available on E-UTRAN and UTRAN }

ensure that {

then { UE selects a the highest priority OPLMN and RAT combination and UE attempts to attach on the selected cell and PLMN and when successfully registered indicates the correct PLMN to the user. }

6.2.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3, 4.4.3.1  and 4.4.3.1.1.

[TS 23.122, clause 4.4.3]

When the MS reselects to a cell in a shared network, the AS may indicate multiple PLMN identities to the NAS according to 3GPP TS 25.304 [32]. The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see subclause 4.5.2) attempts to perform a Location Registration.

…

If successful registration is achieved, the MS indicates the selected PLMN.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

6.2.1.3.3	Test description

6.2.1.3.3.1	Pre-test conditions

System Simulator

- 3 cells are used:  2 cells belong to E-UTRAN  and 1 cell belongs to UTRAN. The cells are configured as specified in TS 36.508 clause 4.4.1.4 and Table 4.4.2-2.

UE

- The UE is in Automatic PLMN selection mode.

- The HPLMN is PLMN1.

- There is no RPLMN.

Table 6.2.1.3.3.1–1: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | E-UTRAN |  |
|  | 2 | PLMN2 | UTRAN |  |
|  | 3 | PLMN14 | UTRAN |  |
|  | 4 | PLMN13 | E-UTRAN |  |

Preamble

-	The UE is in state Switched OFF (State 1)

-	Cell levels are set according to row T0 of table 6.2.1.3.3.2-2.

6.2.1.3.3.2	Test procedure sequence

Tables 6.2.1.3.3.2 – 1 and 2 show the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configuration marked “T1” is applied at the point indicated in the Main behaviour description in Table 6.2.1.3.3.2-3. Cell powers for the E-UTRAN cell are defined in TS 36.508 Table 6.2.2.1-1 and cell powers for the UTRAN cells are defined in TS 36.508 Table 6.2.2.1-1.

Table 6.2.1.3.3.2-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN13,PLMN2 |
| 9 | PLMN14, PLMN13 |
| 12 | PLMN13, PLMN14 |

Table 6.2.1.3.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1 | Cell 9 | Cell 12* |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | Serving cell |  | Serving cell |
|  | CPICH_Ec | dBm/3.84 MHz |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  | Serving cell |  |
| T1 | RS EPRE | dBm/15kHz | Non-suitable “Off” |  | Serving cell |
|  | CPICH_Ec | dBm/3.84 MHz |  | Serving cell |  |
|  | P-CCPCH | dBm/1.28 MHz |  | Serving cell |  |
| *Only minimum uplink cell configuration required (See 6.3.3.2 in TS 36.508) |  |  |  |  |  |

Table 6.2.1.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 3-17 | The UE performs Steps 3-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1. | - | - | - | - |
| 18 | Check: Is PLMN2 indicated by the UE? | - | - | 1 | P |
| 19 | SS adjusts cell levels according to row T1 of table 6.2.1.3.3.2-2. | - | - | - | - |
| 20 | Check: Does the UE send a RRC CONNECTION REQUEST on Cell 9? | --> | RRC CONNECTION REQUEST | 2 | P |
| 20A | Void | - | - | - | - |
| 20B | Void | - | - | - | - |
| 21-29 | The UE performs Steps 2-10 of the routing area update procedure described in TS 36.508 Table 6.4.2.8-1. | - | - | - | - |
| - | Exception: Steps 29Aa1 to 29Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 29Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 29Aa1a1-29Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 29Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 29Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 29Aa1b1 | The 6s timer expires | - | - | - | - |
| 30 | Check: Is PLMN14 indicated by the UE? | - | - | 2 | P |

6.2.1.3.3.3	Specific message contents

Table 6.2.1.3.3-0: Conditions for specific message contents
in Table 6.2.1.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.1.3.3.3-1: SystemInformationBlockType1 for Cell 1(all steps, Table 6.2.1.3.3.2-3)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | PLMN13. |  |  |
| plmn-Identity[2] | PLMN2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.3.3.3-2: MasterInformationBlock for Cell 9 (all steps, Table 6.2.1.3.3.2-3)

| Derivation path: 34.108 Section 6.1.0a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| - Supported PLMN types |  |  |  |
| - PLMN type | GSM-MAP |  |  |
| - PLMN identity | PLMN14 |  |  |
| - Multiple PLMN List - MIB PLMN Identity | TRUE |  |  |
| - PLMN identity with Optional MCC[1] | PLMN13 |  |  |

Table 6.2.1.3.3.3-3: SystemInformationBlockType1 for Cell 12 (all steps, Table 6.2.1.3.3.2-3)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | PLMN13 |  |  |
| plmn-Identity[2] | PLMN14 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.3.3-4: RRCConnectionSetupComplete (Step 5, Table 6.2.1.3.3.2-3)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 2 | PLMN2 |  |

Table 6.2.1.3.3-5: INITIAL DIRECT TRANSFER (Step 23, Table 6.2.1.3.3.2-3)

| Derivation Path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PLMN identity |  | PLMN14 |  |  |
| Intra Domain NAS Node Selector - CHOICE version - CHOICE CN type - CHOICE Routing basis |  | R99 GSM-MAP (P)TMSI of different PLMN |  |  |

Table 6.2.1.3.3-6: System Information Block type 19 for Cell 9 (all steps, Table 6.2.1.3.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 12 |  |  |
| priority [n] | 2 |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 12 |  |  |
| priority [n] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.1.4	Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode

6.2.1.4.1	Test Purpose (TP)

(1)

with { UE in Manual network selection mode and OPLMN cells available on E-UTRAN and GERAN}

ensure that {

when { the USER selects an OPLMN}

then { UE selects a the highest priority RAT for the OPLMN and UE attempts to attach on the selected cell and when successfully registered indicates the PLMN to the user. }

}

(2)

with { UE in Manual network selection mode and OPLMN cells available on E-UTRAN and GERAN}

ensure that {

when { the USER selects an OPLMN and RAT combination}

then {UE attempts to attach on the selected OPLMN and RAT combination and when successfully registered indicates the PLMN to the user. }

}

6.2.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1.2.

[TS 23.122, clause 4.4.3.1.2]

The MS indicates whether there are any PLMNs, which are available using all supported access technologies. This includes PLMNs in the "forbidden PLMNs" list and PLMNs which only offer services not supported by the MS. An MS which supports GSM COMPACT shall also indicate GSM COMPACT PLMNs (which use PBCCH).

If displayed, PLMNs meeting the criteria above are presented in the following order:

i)-	either the HPLMN (if the EHPLMN list is not present or is empty) or, if one or more of the EHPLMNs are available then based on an optional data field on the SIM either only the highest priority available EHPLMN is to be presented to the user or all available EHPLMNs are presented to the user in priority order. If the data field is not present on the SIM, then only the highest priority available EHPLMN is presented;

ii)-	PLMN/access technology combinations contained in the " User Controlled PLMN Selector with Access Technology " data file in the SIM (in priority order);

iii)- PLMN/access technology combinations contained in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)- other PLMN/access technology combinations with received high quality signal in random order;

v)-	other PLMN/access technology combinations in order of decreasing signal quality.

…

The user may select his desired PLMN and the MS then initiates registration on this PLMN using the access technology chosen by the user for that PLMN or using the highest priority available access technology for that PLMN, if the associated access technologies have a priority order. (This may take place at any time during the presentation of PLMNs). For such a registration, the MS shall ignore the contents of the "forbidden LAs for roaming", "forbidden TAs for roaming", "forbidden LAs for regional provision of service", "forbidden TAs for regional provision of service", "forbidden PLMNs for GPRS service" and "forbidden PLMNs" lists.

NOTE 1:	It is an MS implementation option whether to indicate access technologies to the user. If the MS does display access technologies, then the access technology selected by the user is only used for initial registration on the selected PLMN. If the MS does not display access technologies, then the access technology chosen for a particular PLMN should be the highest priority available access technology for that PLMN, if the associated access technologies have a priority order, and is only used for initial registration.

6.2.1.4.3	Test description

6.2.1.4.3.1	Pre-test conditions

System Simulator

- 3 cells are used:  2 cells belong to E-UTRAN and 1 cell belongs to GERAN. The cells are configured as specified in TS 36.508 clause 4.4.1.4 and Table 4.4.2-2.

UE

- The HPLMN is PLMN1.

Table 6.2.1.4.3.1–1: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | E-UTRAN |  |
|  | 2 | PLMN2 | GERAN |  |
|  | 3 | PLMN2 | UTRAN |  |
|  | 4 | PLMN13 | E-UTRAN |  |

Preamble

- The UE is equipped with a USIM with default settings except for the ones specified in table 6.2.1.4.3.1–1 then the UE is attached to PLMN1 and the UE is set to Manual PLMN selection mode before it is switched off.

-	Cell levels are set according to row T0 of table 6.2.1.4.3.2-2.

6.2.1.4.3.2	Test procedure sequence

Tables 6.2.1.4.3.2 – 1 and 2 show the cell configurations used during the test. The configuration T0 indicates the initial conditions after preamble. Cell powers for the E-UTRAN cell are defined in  TS 36.508 Table 6.2.2.1-1 and cell powers for the  GERAN cell are defined in TS 36.508 Table 6.2.2.1-1.

Table 6.2.1.4.3.2-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN2 |
| 12 | PLMN13 |
| 24 | PLMN2 |

Table 6.2.1.4.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1 | Cell 12* | Cell 24 |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | Serving cell | Serving cell |  |
|  | RF Signal Level | dBm |  |  | Serving cell |
| *Only minimum uplink cell configuration required (See 6.3.3.2 in TS 36.508) |  |  |  |  |  |

Table 6.2.1.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Make the UE display the list of available PLMNs. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5-19 | Void | - | - | - | - |
| 20 | Void | - | - | - | - |
| - | EXCEPTION: Steps 21a1 to 21b18 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that takes place if the condition is met. | - | - | - | - |
| 21a1 | IF pc_Available_PLMNs_AcT_Ind (Support of Access Technology Indication in available PLMNs list) THEN PLMN2 (GERAN) is selected | - | - | - | - |
| 21a2 | The UE completes the registration procedure on GERAN cell 24 as specified in TS 51.010-1 clause 41.2.8.1.1. | - | - | - | - |
| 21a2A | The UE may attempt the procedure for IMS signalling on GERAN as specified in TS 36.508 clause 4.5A.3B | - | - | - | - |
| 21a3 | Check: Is PLMN2 indicated by the UE as the PLMN on which the UE is registered? | - | - | 2 | P |
| 21b1 | ELSE (No Access Technology shown to the User) PLMN2 is selected |  |  |  |  |
| 21b2 | Check: Does the UE send an RRCConnectionRequest on cell 1? | --> | RRCConnectionRequest | 1 | P |
| 21b3-21b17 | The UE performs Steps 3-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1. | - | - | - | - |
| 21b18 | Check: Is PLMN2 indicated by the UE as the PLMN on which the UE is registered? | - | - | 1 | P |

6.2.1.4.3.3	Specific message contents

Table 6.2.1.4.3.3-1: Serving Cell Priority Parameters Description struct of SI2Quater for Cell 24

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 5 |  |  |
| THRESH_Priority_Search | 0 |  |  |
| } |  |  |  |

#### 6.2.1.5

#### 6.2.1.6	Inter-RAT Background HPLMN Search / Search for correct RAT for HPLMN / Automatic Mode

6.2.1.6.1	Test Purpose (TP)

with { UE in Automatic network selection mode is camped on a GERAN VPLMN cell and HPLMN cell available on E-UTRAN}

ensure that {

when { higher priority PLMN search timer T expires }

then { UE detects E_UTRA cell and camps on the E-UTRA cell }

}

6.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.3.1.

[TS 23.122, clause 4.4.3.3.1]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value T minutes may be stored in the SIM, T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value is stored in the SIM, a default value of 60 minutes is used.

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode when the MS is roaming, and not while attached for emergency bearer services;

b)	After switch on a period of at least 2 minutes and at most T minutes shall elapse before the first attempt is made;

c)	The MS shall make the following attempts if the MS is on the VPLMN at time T after the last attempt;

d)	Periodic attempts shall only be performed by the MS while in idle mode;

e)	If the HPLMN (if the EHPLMN list is not present or is empty) or a EHPLMN (if the list is present) or a higher priority PLMN is not found, the MS shall remain on the VPLMN.

f)	In steps i), ii) and iii) of subclause 4.4.3.1.1 the MS shall limit its attempts to access higher priority PLMN/access technology combinations to PLMN/access technology combinations of the same country as the current serving VPLMN, as defined in Annex B.

g)	Only the priority levels of Equivalent PLMNs of the same country as the current serving VPLMN, as defined in Annex B, shall be taken into account to compare with the priority level of a selected PLMN.

h)	If the PLMN of the highest priority PLMN/access technology combination available is the current VPLMN, or one of the PLMNs in the "Equivalent PLMNs" list, the MS shall remain on the current PLMN/access technology combination.

6.2.1.6.3	Test description

          - Pre-test conditions

System Simulator

-	GERAN VPLMN Cell 24 serving cell

-	E-UTRAN HPLMN Cell 1 off

-	Table 6.2.1.6.3.1-1 shows the cell configuration used during the test

Table 6.2.1.6.3.1-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 24 | PLMN15 |

UE

Table 6.2.1.6.3.1–2: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN1 | E-UTRAN |  |
|  | 2 | PLMN15 | GERAN |  |
| EFHPPLMN |  | 1(=6 min) |  | The HPLMN Search Period on the USIM shall be set to 6 minutes. |

Preamble

-	The UE is equipped with a USIM with default settings except for the ones specified in table 6.2.1.6.3.1–2.

-	The UE is switched off..

6.2.1.6.3.2	Test procedure sequence

Tables 6.2.1.6.3.2 – 1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Cell powers for the E-UTRAN cell are defined in  TS 36.508 Table 6.2.2.1-1 and cell powers for the GERAN cell are defined in TS 36.508 Table 6.2.2.1-1.

Table 6.2.1.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1 | Cell 24 |
| --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -60 | Same as before |
| *Only minimum uplink cell configuration required (See 6.3.3.2 in TS 36.508) |  |  |  |  |

Table 6.2.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 0A | Power on the UE. | - | - | - | - |
| 1 | SS starts timer of tmin = 2 minutes and tmax =(6 minutes + cell selection time) (Note 1) | - | - | - | - |
| 1A | The UE completes the registration procedure on GERAN cell as specified in TS 51.010-1 clause 41.2.8.1.1. | - | - | - | - |
| 1B | The UE may attempt the procedure for IMS signalling on GERAN as specified in TS 36.508 clause 4.5A.3B | - | - | - | - |
| 2 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.1.6.3.2-1 | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on cell 1 after tmin but before tmax expires? (Note 1) | --> | RRC: RRCConnectionRequest | 1 | P |
| 4-10 | The steps from step 2 to step 8 as specified in TS 36.508 Table 6.4.2.7A-2 shall take place | - | - | - | - |
| Note 1: Timers tmin and tmax in step 1 and 3 are derived from the high priority PLMN search timer T defined by EFHPPLMN |  |  |  |  |  |

### 6.2.2	Inter-RAT cell selection

#### 6.2.2.1	Inter-RAT Cell selection / From E-UTRA RRC_IDLE to UTRA_Idle / Serving cell becomes non-suitable

6.2.2.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable and there is a suitable UTRA neighbour cell  }

then { UE selects the suitable UTRA neighbour cell }

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable UTRA neighbour cell}

then { UE selects the suitable UTRA neighbour cell }

}

Note: (SServingCell<0 or barred) condition is for Rel 8; (Srxlev<0 or Squal<0 or barred) condition is for Rel 9.

6.2.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.304, clause 4.1, 4.3,5.2.1, 5.2.3.2 and 5.3.1. TS 25.304, clause 5.2.3.2.

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

- The cell is part of either:

- the selected PLMN, or:

- the registered PLMN, or:

- a PLMN of the Equivalent PLMN list

according to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

-	For a CSG cell, the CSG ID is part of the allowed CSG list of the UE.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [3].

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1, this IE is common for all PLMNs

…

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

…

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

[TS 25.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

| Squal | Cell Selection quality value (dB) Applicable only for FDD cells. |
| --- | --- |
| Srxlev | Cell Selection RX level value (dB) |
| Qqualmeas | Measured cell quality value. The quality of the received signal expressed in CPICH Ec/N0 (dB) for FDD cells. CPICH Ec/N0 shall be averaged as specified in [10]. Applicable only for FDD cells. |
| Qrxlevmeas | Measured cell RX level value. This is received signal, CPICH RSCP for FDD cells (dBm) and P-CCPCH RSCP for TDD cells (dBm). |
| Qqualmin | Minimum required quality level in the cell (dB). Applicable only for FDD cells. |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Pcompensation | max(UE_TXPWR_MAX_RACH – P_MAX, 0) (dB) |
| UE_TXPWR_MAX_RACH | Maximum TX power level an UE may use when accessing the cell on RACH (read in system information) (dBm) |
| P_MAX | Maximum RF output power of the UE (dBm) |

6.2.2.1.3	Test description

6.2.2.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.2.1.3.2	Test procedure sequence

Tables 6.2.2.1.3.2-1 & 6.2.2.1.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and " T3" are applied at the points indicated in the Main behaviour description in Table 6.2.2.1.3.2-3.

Table 6.2.2.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm | -120 | SrxlevCell 1 < 0 |
|  | Srxlev* | dB | -10 | Cell 5 becomes the strongest cell |
| T2 | Cell-specific RS EPRE | dBm | -60 |  |
| T3 | Cell-specific RS EPRE | dBm | -60 | SrxlevCell 1 > SrxlevCell 5 > 0 |
|  | Srxlev* | dB | 50 |  |
|  | cellBarred | - | Barred | Serving cell becomes barred |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.2.1.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -70 | SrxlevCell 5 > 0,for FDD |
|  | P-CCPCH | dBm/1.28 MHz | -72 | SrxlevCell 5 > 0,for TDD |
|  | Srxlev* | dB | 9 | Cell 5 becomes the strongest cell or the suitable cell |
| T2 | CPICH_Ec | dBm/3.84 MHz | OFF |  |
|  | P-CCPCH | dBm/1.28 MHz | OFF |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -70 | SrxlevCell 5 > 0,for FDD |
|  | P-CCPCH | dBm/1.28 MHz | -72 | SrxlevCell 5 > 0,for TDD |
|  | Srxlev* | dB | 9 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T1" in table 6.2.2.1.3.2-1 & Cell 5 power level is set according to row "T1" in table 6.2.2.1.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | 1 | - |
| - | Exception: Steps 2Aa1 to 2Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 2Aa1a1-2Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 2Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 2Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 2Aa1b1 | The 6s timer expires | - | - | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T2" in table 6.2.2.1.3.2-1 & Cell 5 power level is set according to row "T2" in table 6.2.2.1.3.2-2. | - | - | - | - |
| 4A-4G | Generic test procedure in TS 36.508 Table 6.4.2.7A-1 steps 1-7 are performed and the UE will camped on E-UTRAN Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 4Ha1 to 4Ha2b1 describe optional behaviour if UE sends IMS deregistration at steps 2Aa1 to 2Aa1b1 | - | - | - | - |
| 4Ha1-4Ha2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 4I | The SS releases RRC Connection |  |  |  |  |
| 5 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 6 | SS changes Cell 5 power level according to row "T3" in table 6.2.2.1.3.2-2 and modifies System Information so that Cell 1 becomes barred according to row "T3" in table 6.2.2.1.3.2-1. The SS transmits a Paging message including systemInfoModification. | <-- | Paging | - | - |
| 6A | Void | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs a RAU procedure. | - | - | 2 | - |

6.2.2.1.3.3	Specific message contents

Table 6.2.2.1.3.3-1: SystemInformationBlockType1 for Cell 1 (preamble and steps 1 to 5, Table 6.2.2.1.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.3.3-2: Paging for Cell 1 (step 6, Table 6.2.2.1.3.2-3)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.2.1.3.3-3: SystemInformationBlockType1 for Cell 1 (step 6, Table 6.2.2.1.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

#### 6.2.2.2	Inter-RAT Cell selection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_idle / Serving cell becomes non-suitable

6.2.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable and there is a suitable GSM neighbour cell  }

then { UE selects the suitable GSM neighbour cell }

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable GSM neighbour cell}

then { UE selects the suitable GSM neighbour cell }

}

NOTE: (SServingCell<0 or barred) conidtion is for Rel 8; (Srxlev<0 or Squal<0 or barred) condition is for Rel 9.

6.2.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.304, clause 4.1, 4.3,5.2.1, 5.2.3.2, 5.3.1, 43.022, clause 3.6 and TS 45.008, clause 6.4.

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

- The cell is part of either:

- the selected PLMN, or:

- the registered PLMN, or:

- a PLMN of the Equivalent PLMN list

according to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

-	For a CSG cell, the CSG ID is part of the allowed CSG list of the UE.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [3].

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1, this IE is common for all PLMNs

…

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

…

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

[TS 43.022, clause 3.6]

The MS uses a "path loss criterion" parameter C1 to determine whether a cell is suitable to camp on. C1 depends on 4 parameters:

i)	The received signal level (suitably averaged);

ii)	The parameter RXLEV_ACCESS_MIN, which is broadcast as system information, and is related to the minimum signal that the operator wants the network to receive when being initially accessed by an MS;

iii)	The parameter MS_TXPWR_MAX_CCH, which is also broadcast as system information, and is the maximum power that an MS may use when initially accessing the network;

iv)	The maximum power of the MS.

The formula for determining C1 is given in 3GPP TS 45.008.

Use of the parameter C1 enables the MS to determine whether communication is possible with the network in the absence of interference. However because of the possibility of interference degrading the communications, an additional safeguard is used. This is to monitor the success rate of the MS in decoding signalling blocks of the paging subchannel. If there is a downlink signalling failure, i.e. the success rate drops too low, this indicates probable interference on the downlink, and the MS attempts to find another suitable cell. Downlink signalling failure monitoring is specified in 3GPP TS 45.008.

In order to optimize cell reselection, additional cell reselection parameters can be broadcast as system information of each cell. The cell reselection process employs a parameter C2 which depends on these parameters.

The parameters used to calculate C2 are as follows:

i)	CELL_RESELECT_OFFSET (see subclause 3.5.2.2);

ii)	PENALTY_TIME;

When the MS places the cell on the list of the strongest carriers as specified in 3GPP TS 45.008, it starts a timer which expires after the PENALTY_TIME. This timer will be reset when the cell is taken off the list. For the duration of this timer, C2 is given a negative offset. This will tend to prevent fast moving MSs from selecting the cell.

iii)	TEMPORARY_OFFSET;

This is the amount of the negative offset described in (ii) above. An infinite value can be applied, but a number of finite values are also possible.

The permitted values of these parameters and the way in which they are combined to calculate C2 are defined in 3GPP TS 45.008.

Instead of the parameter C2, a GPRS MS applies the corresponding GPRS parameter if provided.

[TS 45.008, clause 6.4]

The path loss criterion parameter C1 used for cell selection and reselection is defined by:

C1 = (A - Max(B,0))

where

A									=	RLA_C - RXLEV_ACCESS_MIN
B									=	MS_TXPWR_MAX_CCH - P

except for the class 3 DCS 1 800 MS where:

B									=	MS_TXPWR_MAX_CCH + POWER OFFSET - P
RXLEV_ACCESS_MIN		=	Minimum received signal level at the MS required for access to the system.
MS_TXPWR_MAX_CCH		=	Maximum TX power level an MS may use when accessing the system 											until otherwise commanded.

POWER OFFSET				=	The power offset to be used in conjunction with the MS TXPWR 
										MAX CCH parameter by the class 3 DCS 1 800 MS.
P									=	Maximum RF output power of the MS.

All values are expressed in dBm.

The path loss criterion (3GPP TS 43.022) is satisfied if C1 > 0.

6.2.2.2.3	Test description

6.2.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 24

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.2.2.3.2	Test procedure sequence

Tables 6.2.2.2.3.2-1 & 6.2.2.2.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.2.2.3.2-3.

Table 6.2.2.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm | -120 | SrxlevCell 1 < 0 |
|  | Srxlev* | dB | -10 | Cell 24 becomes the strongest cell |
| T2 | Cell-specific RS EPRE | dBm | -60 |  |
| T3 | Cell-specific RS EPRE | dBm | -60 | SrxlevCell 1 > SrxlevCell 24 > 0 |
|  | Srxlev* | dB | 50 |  |
|  | cellBarred | - | Barred | Serving cell becomes barred |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.2.2.3.2-2: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 24 | Remark |
| --- | --- | --- | --- | --- |
| T1 | RF Signal Level | dBm | -80 | SrxlevCell 24 > 0 |
|  | C1* | dB | 20 | Cell 24 becomes the strongest cell |
| T2 | RF Signal Level | dBm | OFF |  |
| T3 | RF Signal Level | dBm | -80 | SrxlevCell 24 > 0 |
|  | C1* | dB | 20 |  |
| Note: C1 is calculated in the UE |  |  |  |  |

Table 6.2.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Void | - | - | - | - |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T1" in table 6.2.2.2.3.2-1 & cell 24 power level is set according to row "T1" in table 6.2.2.2.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE performs a RAU procedure. | - | - | 1 | - |
| 3 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T2" in table 6.2.2.2.3.2-1 & Cell 24 power level is set according to row "T2" in table 6.2.2.2.3.2-2. | - | - | - | - |
| 4 | Generic test procedure in TS 36.508 Table 6.4.2.7A-2 is performed and the UE will be camped on E-UTRAN Cell 1. | - | - | - | - |
| 5 | Void | - | - | - | - |
| 6 | SS changes Cell 24 power level according to row "T3" in table 6.2.2.2.3.2-2 and modifies System Information so that Cell 1 becomes barred according to row "T3" in table 6.2.2.2.3.2-1. SI2Quater for cell 24 is updated according to table 6.2.2.2.3.3-4 to reflect the cell barring. The SS transmits a Paging message including systemInfoModification. | <-- | Paging | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE performs a RAU procedure. The UE may but does not need to reset LLC. | - | - | 2 | - |

6.2.2.2.3.3	Specific message contents

Table 6.2.2.2.3.3-1: SystemInformationBlockType1 for Cell 1 (preamble and steps 1 to 5, Table 6.2.2.2.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.3.3-2: SystemInformationBlockType1 for Cell 1 (step 6, Table 6.2.2.2.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.2.2.2.3.3-3: Paging for Cell 1 (step 6, Table 6.2.2.2.3.2-3)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| etws-Indication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.2.2.3.3-4: Repeated E-UTRAN Neighbour Cells struct of SI2Quarter for Cell 24 (step 6, Table 6.2.2.3.2-3)

| Derivation Path: 36.508 Table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Repeated E-UTRAN Not Allowed Cells struct > ::= { |  |  |  |
| Not Allowed Cells : < PCID Group IE >>{ |  |  |  |
| PCID} | ‘000000000’B | PCID=0, Meaning that the Cell 1 which have PCID=0 will not be allowed |  |
| E-UTRAN_FREQUENCY_INDEX | ‘000’B | Cell 1 |  |
| } |  |  |  |

#### 6.2.2.3	Inter-RAT Cell selection / From E-UTRA RRC_IDLE to HRPD Idle / Serving cell becomes non-suitable

6.2.2.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable and there is a suitable neighbour HRPD cell (S>0) }

then { UE selects the suitable neighbour HRPD cell }

}

Note: (SServingCell<0 or barred) conidtion is for rel 8; (Srxlev<0 or Squal<0 or barred) condition is for rel 9.

6.2.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, and 5.2.3.2.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | [FFS] |

6.2.2.3.3	Test description

6.2.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is a cell on E-UTRA

-	Cell 15 is a cell on HRPD

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.2.3.3.2	Test procedure sequence

Table 6.2.2.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and “T2” are applied at the points indicated in the Main behaviour description in Table 6.2.2.3.3.2-2.

Table 6.2.2.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | -5 | Cell 15 is on |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -6 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 |  |  |
| T2 | Cell-specific RS EPRE | dBm/15 kHz | -120 | - | SrxlevCell 1 <0 and Cell 15 becomes the strongest cell |
|  | Srxlev* | dB | -10 |  |  |
|  | Îor/Ioc | dB | - | -5 |  |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -6 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.2.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 15 according to "T1" in 6.2.2.3.3.2-1. | - | - | - | - |
| 1A | SS waits [10s] to allow the UE to read the system information | - | - | - | - |
| 2 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to "T2" in 6.2.2.3.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE transmit an Access Probe on Cell 15? | --> | Access Probe | 1 | P |

6.2.2.3.3.3	Specific message contents

Table 6.2.2.3.3.3-1 SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.2.3.3.2-2)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.3.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.2.3.3.2-2)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE { |  |  |  |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured HRPD carriers |  |  |
| bandClass | Band Class of frequency under test | [Set according to specific test case] |  |
| cellReselectionPriority | 5 | [3 is applicable when HRPD is lower priority than E-UTRA. 5 is applicable when HRPD is higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.2.4	Inter-RAT Cell selection / From E-UTRAN RRC_IDLE to 1xRTT idle / Serving cell becomes non-suitable

6.2.2.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable and

there is a suitable neighbour 1xRTT cell (S>0) }

then { UE selects the suitable neighbour 1xRTT cell }

}

Note: (SServingCell<0 or barred) conidtion is for rel 8; (Srxlev<0 or Squal<0 or barred) condition is for rel 9.

6.2.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, and 5.2.3.2.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on  that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN.

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | [FFS] |

6.2.2.4.3	Test description

6.2.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is a cell on E-UTRA

-	Cell 19 is a cell on 1xRTT

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.2.4.3.2	Test procedure sequence

Table 6.2.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and “T2” are applied at the point indicated in the Main behaviour description in Table 6.2.2.4.3.2-2

Table 6.2.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | 0 | Cell 19 is on |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | CPICH_Ec/Io (Note 1) | dB | - | -10 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -75 | - |  |
| T2 | Îor/Ioc | dB | - | 0 | SrxlevCell 1 <0 and Cell 19 becomes the strongest cell |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | CPICH_Ec/Io (Note 1) | dB | - | -10 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -120 | - |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row “T1” in Table 6.2.2.4.3.2-1. | - | - | - | - |
| 1A | SS waits [10s] to allow the UE to read the system information. | - | - | - | - |
| 2 | SS re-adjusts the cell-specific reference signal levels according to row "T2" in Table 6.2.2.4.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 1 | P |

6.2.2.4.3.3	Specific message contents

Table 6.2.2.4.3.3-1 SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.2.4.3.2-1)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.2.5	Cell selection / No USIM

6.2.2.5.1	Test Purpose (TP)

(1) Void

(2)

with { UE supporting E-UTRA and UTRAN and supporting Voice services, but not IMS speech and fitted with no SIM/USIM }

ensure that {
  when { UE is switched on and there are E-UTRA and UTRAN cells available with the E-UTRA cell preferable from cell power point of view and UE is requested to make an emergency call }

then { UE camps on the UTRAN cell and initiates an Emergency call }

}

6.2.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 22.101 clause 10.1 and TS 23.221, clause 7.2a.

[TS 22.101, clause 10.1]

Emergency calls shall be supported by the UE without a SIM/USIM/ISIM being present. No other type than emergency calls shall be accepted without a SIM/USIM/ISIM.

[TS 23.221, clause 7.2a]

In limited service state, a UE should always camp on a RAT which is likely to support emergency calls.

The TS 23.167 [33] describes when an emergency call is to be initiated in the CS domain or using IMS. When the CS domain is to be used the following applies:

-	A CSFB/IMS enabled UE which is registered in the CS domain, e.g. using CSFB procedures or using above-mentioned selection principles, shall initiate emergency call attempts in the CS domain by using available CS domain procedures.

-	A CS Voice capable UE which is not registered in the CS domain shall initiate emergency calls in the CS domain by autonomously switching to a RAT which supports the CS domain (e.g. GERAN or UTRAN).

6.2.2.5.3	Test description

6.2.2.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

Table 6.2.2.5.3.1-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm | -70 | - |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -75 |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -77 |

UE:

-	The UE is not equipped with a USIM.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.2.2.5.3.2	Test procedure sequence

Table 6.2.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait for 60s for the UE to complete cell selection. E-UTRA cell is more powerful than UTRA cell | - | - | - | - |
| 3 | Cause the UE to originate Emergency call. (see Note 1) | - | - | - | - |
| 4 | Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5? | --> | RRC CONNECTION REQUEST | 2 | P |
| 5 | The SS transmits an RRC CONNECTION SETUP message on Cell 5. | <-- | RRC CONNECTION SETUP | - | - |
| 6 | The UE transmits an RRC CONNECTION SETUP COMPLETE message on Cell 5. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 7 | The UE transmits an INITIAL DIRECT TRANSFER message on Cell 5. This message includes a CM SERVICE REQUEST message. | --> | INITIAL DIRECT TRANSFER | - | - |
| 8 | The SS transmits a DOWNLINK DIRECT TRANSFER message on Cell 5. This message includes a CM SERVICE ACCEPT message. | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 9 | The UE transmits an UPLINK DIRECT TRANSFER message on Cell 5. This message includes an EMERGENCY SETUP message. | --> | UPLINK DIRECT TRANSFER | - | - |
| 10 | The SS transmits a DOWNLINK DIRECT TRANSFER message on Cell 5. This message includes a CALL PROCEEDING message. | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 11 | The SS transmits a RADIO BEARER SETUP message on Cell 5. | <-- | RADIO BEARER SETUP | - | - |
| 12 | The UE transmits a RADIO BEARER SETUP COMPLETE message on Cell 5. | --> | RADIO BEARER SETUP COMPLETE | - | - |
| 13 | The SS transmits a DOWNLINK DIRECT TRANSFER message on Cell 5. This message includes an ALERTING message. | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 14 | The SS transmits a DOWNLINK DIRECT TRANSFER message on Cell 5. This message includes a CONNECT message. | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 15 | The UE transmits an UPLINK DIRECT TRANSFER message on Cell 5. This message includes a CONNECT ACKNOWLEDGE message. | --> | UPLINK DIRECT TRANSFER | - | - |
| Note 1: The request to originate an emergency call may be performed by MMI or AT command. |  |  |  |  |  |

6.2.2.5.3.3	Specific message contents

None.

#### 6.2.2.6	Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA_RRC_IDLE / Serving cell becomes non-suitable

6.2.2.6.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { the serving cell becomes non-suitable and there is a suitable E-UTRA neighbour cell  }

then { UE selects the suitable E-UTRA neighbour cell }

6.2.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.4 .

...

[TS 45.008, clause 6.4]

The path loss criterion parameter C1 used for cell selection and reselection is defined by:

C1 = (A - Max(B,0))

where

A									=	RLA_C - RXLEV_ACCESS_MIN
B									=	MS_TXPWR_MAX_CCH - P

except for the class 3 DCS 1 800 MS where:

B									=	MS_TXPWR_MAX_CCH + POWER OFFSET - P
RXLEV_ACCESS_MIN		=	Minimum received signal level at the MS required for access to the system.
MS_TXPWR_MAX_CCH		=	Maximum TX power level an MS may use when accessing the system 											until otherwise commanded.

POWER OFFSET				=	The power offset to be used in conjunction with the MS TXPWR 
										MAX CCH parameter by the class 3 DCS 1 800 MS.
P									=	Maximum RF output power of the MS.

All values are expressed in dBm.

The path loss criterion (3GPP TS 43.022) is satisfied if C1 > 0.

6.2.2.6.3	Test description

6.2.2.6.3.1	Pre-test conditions

System Simulator:

- Two cells:

- One GERAN cell, Cell 24 is serving cell.

- One E-UTRAN cell, Cell 1 is Suitable neighbour inter-RAT cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3B [18] on Cell 24.

6.2.2.6.3.2	Test procedure sequence

Tables 6.2.2.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.2.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -120 | - | SrxlevCell 1 < 0 |
|  | Qrxlevmin | dBm | -110 | - | - |
|  | Srxlev* | dB | N/A | - | - |
|  | RF Signal level | dBm | - | -80 | - |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | - |
|  | Srxlev* | dB | 30 | - | SrxlevCell 1 >THRESH_E-UTRAN_high |
|  | RF Signal level | dBm | - | OFF | - |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.2.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.2.2.6.3.2-1 | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 A-2 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.2.2.6.3.3	Specific message contents

None.

#### 6.2.2.7	Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA_RRC_IDLE ,when the serving cell is barred.

6.2.2.7.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { the serving cell becomes barred and there is a suitable E-UTRA neighbour cell }

then { UE selects the suitable E-UTRA neighbour cell }

}

6.2.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause  6.4 and 6.6.1, TS 43.022, clause 3.2.1.

[TS 45.008, clause 6.4]

The path loss criterion parameter C1 used for cell selection and reselection is defined by:

C1 = (A - Max(B,0))

where

A									=	RLA_C - RXLEV_ACCESS_MIN
B									=	MS_TXPWR_MAX_CCH - P

except for the class 3 DCS 1 800 MS where:

B									=	MS_TXPWR_MAX_CCH + POWER OFFSET - P
RXLEV_ACCESS_MIN		=	Minimum received signal level at the MS required for access to the system.
MS_TXPWR_MAX_CCH		=	Maximum TX power level an MS may use when accessing the system 											until otherwise commanded.

POWER OFFSET				=	The power offset to be used in conjunction with the MS TXPWR 
										MAX CCH parameter by the class 3 DCS 1 800 MS.
P									=	Maximum RF output power of the MS.

All values are expressed in dBm.

The path loss criterion (3GPP TS 43.022) is satisfied if C1 > 0.

[TS 45.008, clause 6.6.1]

The MS shall attempt to decode the full BCCH data of the serving cell at least every 30 seconds or at least as often as possible in the case that system information scheduling period exceeds 30 seconds.

If SI13 is broadcast, the MS supporting change mark in SI13 (See 3GPP TS 44.018) is only required to confirm system information on the BCCH of the serving cell if indicated by change mark in SI13.

[TS 43.022, clause 3.2.1]

The choice of such a suitable cell for the purpose of receiving normal service is referred to as "normal camping". There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

i)	It should be a cell of the selected PLMN or, if the selected PLMN is equal to the last registered PLMN, an equivalent PLMN;

ii)	It should not be "barred" (see subclause 3.5.1);

6.2.2.7.3	Test description

6.2.2.7.3.1	Pre-test conditions

System Simulator:

- Two cells:

- One GERAN cell, Cell 24 is serving cell.

- One E-UTRAN cell, Cell 1 is Suitable neighbour inter-RAT cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3B [18] on Cell 24

6.2.2.7.3.2	Test procedure sequence

Tables 6.2.2.7.3.2-1  illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.2.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -120 | - | SrxlevCell 1 < 0 |
|  | Qrxlevmin | dBm | -110 | - |  |
|  | Srxlev* | dB | N/A | - |  |
|  | RF Signal Level | dBm | - | -80 |  |
|  | RXLEV_ACCESS_MIN | dBm | - | -101 |  |
|  | C1* | dB | - | 21 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - |  |
|  | Srxlev* | dB | 30 | - | SrxlevCell 1 >THRESH_E-UTRAN_high |
|  | RF Signal Level | dBm | - | -80 |  |
|  | RXLEV_ACCESS_MIN | dBm | - | 21 |  |
|  | C1* | dB | - | Barred |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.2.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cells according to row T1 of table 6.2.2.7.3.2-1 and SI2 IE cellBarred according to row "T1" in table 6.2.2.7.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |

6.2.2.7.3.3	Specific message contents

Table 6.2.2.7.3.3-1: RACH Control Parameters of SI2, SI3 and SI4 for Cell 24(step 1, Table 6.2.2.7.3.2-3)

| Derivation Path: 51.010-1 clause 40.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH Control Parameters{ |  |  |  |
| Cell Barred for Access | Barred |  |  |
| } |  |  |  |

Table 6.2.2.7.3.3-2: SI 13 Rest Octets of , SI13 for Cell 24(step 1, Table 6.2.2.7.3.2-3)

| Derivation Path: 51.010-1 clause 40.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BCCH_CHANGE_MARK { | 001 |  |  |
| SI_CHANGE_FIELD | 0000 |  |  |
| } |  |  |  |

#### 6.2.2.8	Inter-RAT Cell selection / From UTRA_Idle to E-UTRA RRC_IDLE / Serving cell becomes non-suitable

6.2.2.8.1	Test Purpose (TP)

(1)

with { UE in UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable and there is a suitable E-UTRA neighbour cell  }

then { UE selects the suitable E-UTRA neighbour cell }

(2)

with { UE in UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable E-UTRA neighbour cell}

then { UE selects the suitable E-UTRA neighbour cell }

}

Note: (SServingCell<0 or barred) conidtion is for rel 8; (Srxlev<0 or Squal<0 or barred) condition is for rel 9.

6.2.2.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 25.304, clause 5.2.1, 4.3, 5.2.1.4, 5.2.3.2. TS 36.304, clause 4.1, 4.3,5.2.1 and 5.2.3.2.

[TS 25.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 25.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

-	The cell shall be part of either:

- the selected PLMN, or:

- the registered PLMN, or:

- a PLMN of the Equivalent PLMN list

according to the latest information provided by the NAS.

-	The cell is not barred, see subclause 5.3.1.1;

-	The cell is part of at least one LA that is not part of the list of "forbidden LAs for roaming" [9], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.1.2.

-	For a CSG cell, the cell is a CSG member cell for the UE.

If the IE “Multiple PLMN List” [4] is broadcast in the cell, the cell is considered to be part of all LAs with LAIs constructed from the PLMN identities in the “Multiple PLMN List” and the LAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [4].

[TS 25.304, clause 5.2.5.1]

When camped normally, the UE shall perform the following tasks:

-	execute the cell reselection evaluation process on the following occasions/triggers:

1)	UE internal triggers, so as to meet performance as specified in [10] and [11];

2)	When information on the BCCH used for the cell reselection evaluation procedure has been modified

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

- The cell is part of either:

- the selected PLMN, or:

- the registered PLMN, or:

- a PLMN of the Equivalent PLMN list

according to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

-	For a CSG cell, the CSG ID is part of the allowed CSG list of the UE.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 25.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

| Squal | Cell Selection quality value (dB) Applicable only for FDD cells. |
| --- | --- |
| Srxlev | Cell Selection RX level value (dB) |
| Qqualmeas | Measured cell quality value. The quality of the received signal expressed in CPICH Ec/N0 (dB) for FDD cells. CPICH Ec/N0 shall be averaged as specified in [10]. Applicable only for FDD cells. |
| Qrxlevmeas | Measured cell RX level value. This is received signal, CPICH RSCP for FDD cells (dBm) and P-CCPCH RSCP for TDD cells (dBm). |
| Qqualmin | Minimum required quality level in the cell (dB). Applicable only for FDD cells. |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Pcompensation | max(UE_TXPWR_MAX_RACH – P_MAX, 0) (dB) |
| UE_TXPWR_MAX_RACH | Maximum TX power level an UE may use when accessing the cell on RACH (read in system information) (dBm) |
| P_MAX | Maximum RF output power of the UE (dBm) |

6.2.2.8.3	Test description

6.2.2.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.2.2.8.3.2	Test procedure sequence

Tables 6.2.2.8.3.2-1 & 6.2.2.8.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and " T3" are applied at the points indicated in the Main behaviour description in Table 6.2.2.8.3.2-3.

Table 6.2.2.8.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | Cell 1 becomes the strongest cell or the suitable cell |
|  | Srxlev* | dB | 36 | SrxlevCell 1 > 0 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.2.8.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -90 | SrxlevCell 5 < 0,for FDD |
|  | P-CCPCH | dBm/1.28 MHz | -92 | SrxlevCell 5 < 0,for TDD |
|  | Srxlev* | dB | -11 |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -50 |  |
|  | P-CCPCH | dBm/1.28 MHz | -52 |  |
|  | Srxlev* | dB | 29 | SrxlevCell 5 > SrxlevCell 1 > 0 Serving cell becomes barred |
| Note : Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.2.8.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T1" in table 6.2.2.8.3.2-1 & Cell 5 power level is set according to row "T1" in table 6.2.2.8.3.2-2. | - | - | - | - |
| - | EXCEPTION : Step 2a1 to 2b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN |  |  |  |  |
| 2 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 is performed and the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |
| 2b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? |  |  |  |  |
| 2A | Wait for [5] s for UE to receive system information. | - | - | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T2" in table 6.2.2.8.3.2-1 & Cell 5 power level is set according to row "T2" in table 6.2.2.8.3.2-2. | - | - | - | - |
| 4 | Generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| 5 | SS changes Cell 1 power level according to row "T3" in table 6.2.2.8.3.2-1 and modifies System Information so that Cell 5 becomes barred according to row "T3" in table 6.2.2.8.3.2-2. The SS transmits a PAGING TYPE 1 message including "BCCH Modification Information”. | <-- | PAGING TYPE 1 | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 2 | - |

6.2.2.8.3.3	Specific message contents

Table 6.2.2.8.3.3-2: PAGING TYPE 1 for Cell 5 (step 5, Table 6.2.2.8.3.2-3)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.2.8.3.3-3: SystemInformationBlockType3 for Cell 5 (step 5, Table 6.2.2.8.3.2-3)

| Derivation Path: 34.108 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Cell Access Restriction |  |  |  |
| - - Cell barred | barred |  |  |
| - - Intra-frequency cell re-selection indicator | notAllowed |  |  |
| - Tbarred | - s1280 |  |  |

### 6.2.3	Inter-RAT Cell reselection

#### 6.2.3.1	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle

6.2.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT(GERAN) frequency for TreselectionRAT }

then { UE reselects the cell which belongs to the higher priority inter-RAT(GERAN) frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT(GERAN) frequency for TreselectionRAT}

then { UE reselects the cell which belongs to the lower priority inter-RAT(GERAN) frequency }

}

6.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and  5.2.4.5.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). The UE shall delete priorities provided by dedicated signalling when:

- the UE enters RRC_CONNECTED state; or

- the optional validity time of dedicated priorities (T320) expires; or

- a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and [T3230, FFS] in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on serving frequency or on an equal priority E-UTRAN frequency or on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low  during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

or if:

-	there are no candidate cells for reselection on serving frequency according to 5.2.4.4 or 5.3.1.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured From the evaluated cell.

6.2.3.1.3	Test description

6.2.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 24 and Cell 25.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.1.3.2	Test procedure sequence

Tables 6.2.3.1.3.2-1 & 6.2.3.1.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.1.3.2-3.

Table 6.2.3.1.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cells

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | SrxlevCell 1 >0 |
|  | Srxlev* | dB | 36 | SrxlevCell 1 > SrxlevCell 24 > 0 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | SrxlevCell 1 >20 |
|  | Srxlev* | dB | 46 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -100 | SrxlevCell 1 <20 [threshServingLow] |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.1.3.2-2: Time instances of cell power level and parameter changes for GERAN cells

|  | Parameter | Unit | Cell 24 | Cell 25 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | RF Signal Level | dBm | -80 | OFF | SrxlevCell 24 > 4[threshX-High] |
|  | C1* | dB | 21 | N/A |  |
| T2 | RF Signal Level | dBm | OFF | OFF |  |
|  | C1* | dB | N/A | 21 |  |
| T3 | RF Signal Level | dBm | OFF | -80 | SrxlevCell 25 > 4[threshX-Low] |
| Note: C1 is calculated in the UE |  |  |  |  |  |

Table 6.2.3.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 according to the row "T1" in table 6.2.3.1.3.2-1and Cell 24 level according to the row "T1" in table 6.2.3.1.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE performs a RAU procedure. | - | - | 1 | - |
| 3 | Void | - | - | - | - |
| 4 | Wait for [5 s]. | - | - | - | - |
| 5 | The SS changes Cell 1 according to the row "T2" in table 6.2.3.1.3.2-1and Cell 24 level according to the row "T2" in table 6.2.3.1.3.2-2 | - | - | - | - |
| 6 | UE reselects Cell 1 and completes tracking area update procedure. Steps 1 to 22 of the generic test procedure described in TS 36.508 Table 6.4.2.7A-2 are performed on Cell 1. | - | - | - | - |
| 7 | Wait for [5 s]. | - | - | - | - |
| 8 | The SS changes Cell 1 according to the row "T3" in table 6.2.3.1.3.2-1 and Cell 25 level according to the row "T3" in table 6.2.3.1.3.2-2. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 25? NOTE: The UE performs a RAU procedure. | - | - | 2 | - |

6.2.3.1.3.3	Specific message contents

Table 6.2.3.1.3.3-1:  SystemInformationBlockType3 cell 1 (preamble and all steps, Table 6.2.3.1.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 10 | Actual value =20 dB |  |
| cellReselectionPriority | 4 | A middle value in the range has been selected. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-2: SystemInformationBlockType7 cell 1 (preamble and all steps, Table 6.2.3.1.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 | INTEGER (0..7) |  |
| speedDependentScalingParameters SEQUENCE {} | Not present |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqsGERAN SEQUENCE [n] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Empty |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Greater than EUTRA Freq |  |
| ncc-Permitted | ‘01000000’B |  |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| carrierFreqsGERAN SEQUENCE [m] { |  |  |  |
| startingARFCN | Same as cell 25 |  |  |
| bandIndicator | Same as cell 25 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Empty |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 | Less than EUTRA Freq |  |
| ncc-Permitted | ‘01000000’B |  |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-3: System Information 2 Quater for Cell 24 in Preamble and all test steps

| Derivation Path: 36.508 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 5 | Greater than EUTRA freq |  |
| } |  |  |  |

Table 6.2.3.1.3.3-4: System Information 2 Quater for Cell 25 in Preamble and all test steps

| Derivation Path: 36.508 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 3 | Less than EUTRA freq |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells | 1 | Present |  |
| { |  |  |  |
| EARFCN | Same as cell 1 |  |  |
| EARFCN | 65535 |  | Band > 64 |
| E-UTRAN_QRXLEVMIN | 21 (-98 dBm) |  |  |
| } ** 0 |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 6.2.3.1a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle (Squal < ThreshServing, LowQ, Srxlev > ThreshX, LowP and Srxlev > ThreshX, HighP)

6.2.3.1a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria(Srxlev > ThreshX, HighP) is met for the cell which belong to the higher priority inter-RAT(GERAN) frequency for TreselectionRAT }

then { UE reselects the cell which belongs to the higher priority inter-RAT(GERAN) frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria(Squal < ThreshServing, LowQ and Srxlev > ThreshX, LowP) is met for the cell which belong to the lower priority inter-RAT(GERAN) frequency for TreselectionRAT }

then { UE reselects the cell which belongs to the lower priority inter-RAT(GERAN) frequency }

}

6.2.3.1a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.304, clause 5.2.4.5.

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

6.2.3.1a.3	Test description

6.2.3.1a.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 24 and Cell 25.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.1a.3.2	Test procedure sequence

Tables 6.2.3.1a.3.2-1 & 6.2.3.1a.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.1a.3.2-3.

Table 6.2.3.1a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cells

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | SrxlevCell 1 >0 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -100 |  |
|  | RSRQ | dB | -3.01 | Make SqualCell 1 < threshServingLowQ |
|  | Noc | dBm/15kHz | off |  |
| Note: Srxlev and Squal is calculated in the UE. |  |  |  |  |

Table 6.2.3.1a.3.2-2: Time instances of cell power level and parameter changes for GERAN cells

|  | Parameter | Unit | Cell 24 | Cell 25 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | RF Signal Level | dBm | -80 | OFF | SrxlevCell 24 > 4[threshX-High] |
|  | C1* | dB | 21 | N/A |  |
| T2 | RF Signal Level | dBm | OFF | OFF |  |
|  | C1* | dB | N/A | N/A |  |
| T3 | RF Signal Level | dBm | OFF | -80 | SrxlevCell 25 > 4[threshX-Low] |
| Note: C1 is calculated in the UE. |  |  |  |  |  |

Table 6.2.3.1a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 according to the row "T1" in table 6.2.3.1a.3.2-1and Cell 24 level according to the row "T1" in table 6.2.3.1a.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE performs a RAU procedure. | - | - | 1 | - |
| 3 | The SS send CHANNEL RELEASE message to UE. | <-- | CHANNEL RELEASE | - | - |
| 4 | The SS changes Cell 1 according to the row "T2" in table 6.2.3.1a.3.2-1and Cell 24 level according to the row "T2" in table 6.2.3.1a.3.2-2 | - | - | - | - |
| 5 | UE performs generic test procedure in TS 36.508 subclause 6.4.2.7A-2 and camp on E-UTRA Cell 1. | - | - | - | - |
| 7 | The SS changes Cell 1 according to the row "T3" in table 6.2.3.1a.3.2-1 and Cell 25 level according to the row "T3" in table 6.2.3.1a.3.2-2. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 25? NOTE: The UE performs a RAU procedure. | - | - | 2 | - |
| 9 | The SS send CHANNEL RELEASE message to UE. | <-- | CHANNEL RELEASE | - | - |

6.2.3.1a.3.3	Specific message contents

Table 6.2.3.1a.3.3-1: SystemInformationBlockType1 cell 1 (preamble and all steps, Table 6.2.3.1a.3.2-3)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |  |
| q-QualMin-r9 | -20 |  |  |  |
| q-QualMinOffset-r9 | 1 |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |

Table 6.2.3.1a.3.3-2: SystemInformationBlockType3 cell 1 (preamble and all steps, Table 6.2.3.1a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 0 | Actual value =0 dB |  |
| cellReselectionPriority | 4 | A middle value in the range has been selected. |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1a.3.3-3: SystemInformationBlockType7 cell 1 (preamble and all steps, Table 6.2.3.1a.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 | INTEGER (0..7) |  |
| t-ReselectionGERAN-SF | Not present |  |  |
| speedDependentScalingParameters SEQUENCE {} | Not present |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqsGERAN SEQUENCE [n] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 24 |  |  |
| numberOfFollowingARFCNs | Same as cell 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Greater than EUTRA Freq |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| carrierFreqsGERAN SEQUENCE [m] { |  |  |  |
| startingARFCN | Same as cell 25 |  |  |
| bandIndicator | Same as cell 25 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 25 |  |  |
| numberOfFollowingARFCNs | Same as cell 25 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 2 | Less than EUTRA Freq |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1a.3.3-4: System Information 2 Quater for Cell 24 (Preamble and all test steps, , Table 6.2.3.1a.3.2-3)

| Derivation Path: 36.508 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 5 | Greater than EUTRA freq |  |
| } |  |  |  |

Table 6.2.3.1a.3.3-5: System Information 2 Quater for Cell 25 (Preamble and all test steps, , Table 6.2.3.1a.3.2-3)

| Derivation Path: 36.508 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 2 | Less than EUTRA freq |  |  |
| 1 | Present |  |  |
| Same as cell 1 |  |  |  |
| 21 (-98 dBm) |  |  |  |
| } ** 0 |  |  |  |

#### 6.2.3.2	Void

#### 6.2.3.3	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE

6.2.3.3.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a.

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

6.2.3.3.3	Test description

6.2.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.2.3.3.3.2	Test procedure sequence

Tables 6.2.3.3.3.2-1 & Table 6.2.3.3.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA S nonServingCell, Cell1 > Thresh Cell1,low |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -80 |  |
|  | Srxlev* | dB | 26 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA S nonServingCell, Cell1 > Thresh Cell1,high |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.3.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -90 |  |
|  | P-CCPCH | dBm/1.28 MHz | -92 |  |
|  | Srxlev* | dB | -11 | SrxlevServingCell < Thresh Serving,low |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | -72 |  |
|  | Srxlev* | dB | 9 |  |
| Note : Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.3.3.2-1 and table 6.2.3.3.3.2-2. | - | - | - | - |
| - | EXCEPTION : Step 2a1 to 2b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN |  |  |  |  |
| 2 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |
| 2b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? |  |  |  |  |
| 2A | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.3.3.2-1 and table 6.2.3.3.3.2-2. | - | - | - | - |
| 4 | Generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| 5 | The SS changes the E-UTRA cell priority broadcast in system information. | - | - | - | - |
| 6 | Notify UE change of System Information. | <-- | PAGING TYPE 1 | - | - |
| 7 | The SS changes Cell 1 and Cell 5 levels according to the row "T3" in table 6.2.3.3.3.2-1 and table 6.2.3.3.3.2-2. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.3.3.3	Specific message contents

Table 6.2.3.3.3.3-0: Conditions for specific message contents
in Table 6.2.3.3.3.3-1 and Table 6.2.3.3.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.3.3.3-1: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.3.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| s-PrioritySearch1 | 8 (16 dB) |  |  |
| s-PrioritySearch2 | Not present |  |  |
| threshServingLow | 4 (8 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 3 |  |  |
| qRxLevMinEUTRA [n] | -60 (-120 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 10 (20 dB) |  |  |
| eutra-exclude-ListedCellList [n] | Not present |  |  |
| eutraDetection [n] | TRUE |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 3 |  |  |
| qRxLevMinEUTRA [n] | -60 (-120 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 10 (20 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.3.3.3-2: System Information Block type 19 for Cell 5 (step 5, Table 6.2.3.3.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| s-PrioritySearch1 | 8 (16 dB) |  |  |
| s-PrioritySearch2 | Not present |  |  |
| threshServingLow | 4 (8 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| qRxLevMinEUTRA [n] | -60 (-120 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 10 (20 dB) |  |  |
| eutra-exclude-ListedCellList [n] | Not present |  |  |
| eutraDetection [n] | TRUE |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| qRxLevMinEUTRA [n] | -60 (-120 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 10 (20 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.3.3.3-3: PAGING TYPE 1 (step 6, Table 6.2.3.3.3.2-3)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-4:SystemInformationBlockType3 for Cell 1(preamble, Table 6.2.3.3.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-5:SystemInformationBlockType3 for Cell 1(step 5, Table 6.2.3.3.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-6:SystemInformationBlockType6 for Cell 1(preamble, Table 6.2.3.3.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.3.3.3-7:SystemInformationBlockType1 for Cell 1(step 5, Table 6.2.3.3.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | The value is increased |  |  |
| } |  |  |  |

#### 6.2.3.3a	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2)

6.2.3.3a.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a.

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority inform69ation for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= 0) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: (SqualServingCell < Threshserving,low2 or SrxlevServingCell <= 0) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

...

Cell reselection from UTRAN to E-UTRAN, for which Squal (RSRQ) based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria (criterion 4 or 5) if the UE supports Squal based cell reselection from E-UTRAN to UTRAN. Otherwise, cell reselection to E-UTRAN shall be performed based on Srxlev criteria.

6.2.3.3a.3	Test description

6.2.3.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.2.3.3a.3.2	Test procedure sequence

Tables 6.2.3.3a.3.2-1 & Table 6.2.3.3a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.3a.3.2-3.

Table 6.2.3.3a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Squal nonServingCell, Cell1 > Thresh Cell1,low2 (4 dB) |
|  | RSRQ | dB | -3 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | Off |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 |  |
|  | RSRQ | dB | -3 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | RSRQ | dB | -3 | S nonServingCell, Cell1 > Thresh Cell1,high2 |

Table 6.2.3.3a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -80 | SqualServingCell < ThreshServing,low2 (25 dB) |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
|  | Qrxlevmin | dBm | -99 |  |
|  | Qqualmin | dB | -20 |  |
|  | Ioc | dBm/3.84MHz | off |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -80 |  |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |

Table 6.2.3.3a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| - | EXCEPTION : Step 2a1 to 2b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN |  |  |  |  |
| 2 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |
| 2b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? |  |  |  |  |
| 3 | Void | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 5 | The UE selects UTRAN cell 5 and performs the generic test procedure in TS 36.508 subclause 6.4.2.8. | - | - | - | - |
| 6 | The SS changes the E-UTRA cell priority broadcast in system information. | - | - | - | - |
| 7 | Notify UE change of System Information. | <-- | PAGING TYPE 1 | - | - |
| 8 | The SS changes Cell 1 and Cell 5 levels according to the row "T3" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.3a.3.3	Specific message contents

Table 6.2.3.3a.3.3-0: Conditions for specific message contents
in Table 6.2.3.3a.3.3-1 and Table 6.2.3.3a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.3a.3.3-1: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.3a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark | Comment |  |  | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |  |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |  |  |  |  |
| priority |  |  | 4 |  |  |  |  |
| s-PrioritySearch1 |  |  | 12 (24 dB) |  |  |  |  |
| s-PrioritySearch2 |  |  | 0 (0 dB) |  |  |  |  |
| threshServingLow |  |  | 2 (4 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn [n] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  |  | Not present |  |  |  |  |
| priority [n] |  |  | 3 |  |  |  |  |
| threshXhigh [n] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| threshServingLow2 | 25 (25dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| threshXhigh2[n] | 30 (30dB) |  |  |  |  |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn [n] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  |  | Not present |  |  |  |  |
| priority [n] |  |  | 3 |  |  |  |  |
| threshXhigh [n] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |  |  |  |  |

Table 6.2.3.3a.3.3-2: System Information Block type 19 for Cell 5 (step 6, Table 6.2.3.3a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | 4 (4dB) |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.3a.3.3-3: PAGING TYPE 1 (step 7, Table 6.2.3.3a.3.2-3)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-4:SystemInformationBlockType3 for Cell 1(preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-5:SystemInformationBlockType3 for Cell 1(step 6, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-6:SystemInformationBlockType6 for Cell 1(preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| q-RxLevMin[n] | -36 (-71 dBm) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| q-RxLevMin[n] | -36 (-71 dBm) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 6.2.3.4	Inter-RAT Cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE

6.2.3.4.1	Test Purpose (TP)

(1)

with { UE in UTRA_CELL_PCH state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a; TS 25.331, clause 8.3.9.2, 8.3.9.3, Annex E.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states.

...

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

...

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

[TS 25.331, 8.3.9.2]

This procedure is applicable in states CELL_FACH, CELL_PCH or URA_PCH. Inter-RAT cell reselection to E-UTRAN in CELL_FACH state is not supported in this version of the specification.

When the UE based on received system information makes a cell reselection to a radio access technology other than UTRAN, e.g. GSM/GPRS or E-UTRA, according to the criteria specified in [4], the UE shall:

...

[TS 25.331, 8.3.9.3]

When the UE has succeeded in reselecting a cell in the target radio access technology, the UE shall:

1>	release all UTRAN specific resources.

UTRAN should:

1>	release all UE dedicated resources upon indication that the UE has completed a connection establishment to the other radio access technology.

[TS 25.331, Annex E]

This annex contains the definitions of the bits in EUTRA Feature Group Indicators.

In this release of the specification the UE shall include the optional EUTRA Feature Group Indicators IE in the UE multi-mode/multi RAT capability IE of the RRC message. For a specific indicator, if all functionalities for a feature group listed in Table E.1 have been implemented and tested, the UE shall set the indicator as "true" (as one), else (i.e. if any one of the functionalities in a feature group listed in Table E.1 have not been implemented or tested), the UE shall set the indicator as zero.

The UE shall set all indicators, which do not have a definition in the table E.1, as "false" (as zero).

If the optional EUTRA Feature Group Indicators IE is not included by a UE supporting a future release of the specification, the network may assume the UE supports all features in the feature groups listed in table E.1 and deployed in the network.

Table E.1: Definitions of feature group indicators

| Index of indicator (bit number) | Definition (description of the supported functionality, if indicator set to ‘true’) | Notes |
| --- | --- | --- |
| 1 (leftmost bit) | - UTRA CELL_PCH to EUTRA RRC_IDLE cell reselection - UTRA URA_PCH to EUTRA RRC_IDLE cell reselection |  |
| 2 | EUTRAN measurements and reporting in connected mode |  |
| 3 | Undefined |  |
| 4 | Undefined |  |

Clarification for mobility to EUTRAN

For mobility to E-UTRAN, it is assumed that we have 6 main "functions":

A.	Support of measurements and cell reselection procedure in idle mode

B.	Support of measurements and cell reselection procedure in CELL/URA_PCH

C.	Support of RRC release with redirection procedure

D.	Support of RRC reject with redirection procedure

E.	Support of EUTRAN measurements and reporting in connected mode

F.	Support of handover procedure in connected mode

Of the above, all UEs that indicate support of E-UTRA in UE capability signalling "Support of E-UTRA FDD" or "Support of E-UTRA TDD" support A), C) and D) EUTRA bands.

For B) above, UEs indicate support by the Group 1 indicator bit (if Group 1 is set to "true", it is supported for all EUTRA bands the UE supports).

For E) above, UEs indicate support by the Group 2 indicator bit (if Group 2 is set to "true", it is supported for all EUTRA bands the UE supports). The compressed mode capability for supported E-UTRA frequency bands ("Need for compressed mode") in "Measurement capability extension" is ignored by the network when Group 2 is set to "false".

For F) above, UEs indicate support by the separate UE capability signalling "Support of Inter-RAT PS Handover to E-UTRA FDD/TDD" defined in TS 25.306 (if this bit is set to "true", PS handover is supported for all EUTRA bands the UE supports). This bit can only be set to "true" if the UE has set the Group 2 indicator bit to "true".

6.2.3.4.3	Test description

6.2.3.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	Cell 1 suitable neighbour E-UTRA cell

-	Cell 5 UTRA serving cell

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 and moves to state CELL_PCH (state 6-12) according to [5].

The UE is in state

-	The cell power level and parameter for Cell 5 is as follow:

| Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- |
| CPICH_Ec | dBm/3.84 MHz | -60 |  |
| P-CCPCH | dBm/1.28 MHz | -62 |  |
| Qrxlevmin (FDD) | dBm | -79 | Default value |
| Qrxlevmin(TDD) | dBm | -81 | Default value |

6.2.3.4.3.2	Test procedure sequence

Table 6.2.3.4.3.2-1: Time instance of cell power level and parameter changes for E-UTRA cell and UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 |  |  |
|  | Srxlev* | dB | 26 |  | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA S nonServingCell, Cell1 > Thresh Cell1,high |
|  | CPICH_Ec | dBm/3.84 MHz |  | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz |  | -72 |  |
|  | Srxlev* | dB |  | 9 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.3.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.4.3.2-1. (Note 1) | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? (Note 2) | - | - | 1 | - |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |
| Note 1: SS should not send any RRC message after UE has successfully proceed to CELL_PCH state in preamble. Note 2: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.2.3.4.3.3	Specific message contents

Table 6.2.3.4.3.3-0: Conditions for specific message contents
in Table 6.2.3.4.3.3-1

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.4.3.3-1: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.4.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { | 1 entry |  |  |
| earfcn [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [1] | 5 |  |  |
| qRxLevMinEUTRA [1] | -53 (-106 dBm) |  |  |
| threshXhigh [1] | 10 (20 dB) |  |  |
| threshXlow [1] | 5 (10 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [n] | 5 |  |  |
| qRxLevMinEUTRA [1] | -53 (-106 dBm) |  |  |
| threshXhigh [1] | 10 (20 dB) |  |  |
| threshXlow [1] | 5 (10 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-2: Void

Table 6.2.3.4.3.3-3:SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| CellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-4:SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same as Cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same as Cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 6.2.3.4a	Inter-RAT Cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE based on RSRQ+RSRP evaluation

6.2.3.4a.1	Test Purpose (TP)

(1)

with { UE in UTRA_CELL_PCH state }

ensure that {

when { UE detects the cell re-selection criteria ( RSRQ => Squal >0 , RSRP => Srxlev >0 )are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.4a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a; TS 25.331, clause 8.3.9.2, 8.3.9.3, Annex E.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states.

...

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

...

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= 0) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: (SqualServingCell < Threshserving,low2 or SrxlevServingCell <= 0) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

…

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

...

Cell reselection from UTRAN to E-UTRAN, for which Squal (RSRQ) based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria (criterion 4 or 5) if the UE supports Squal based cell reselection from E-UTRAN to UTRAN. Otherwise, cell reselection to E-UTRAN shall be performed based on Srxlev criteria.

[TS 25.331, 8.3.9.2]

This procedure is applicable in states CELL_FACH, CELL_PCH or URA_PCH. Inter-RAT cell reselection to E-UTRAN in CELL_FACH state is not supported in this version of the specification.

When the UE based on received system information makes a cell reselection to a radio access technology other than UTRAN, e.g. GSM/GPRS or E-UTRA, according to the criteria specified in [4], the UE shall:

...

[TS 25.331, 8.3.9.3]

When the UE has succeeded in reselecting a cell in the target radio access technology, the UE shall:

1>	release all UTRAN specific resources.

UTRAN should:

1>	release all UE dedicated resources upon indication that the UE has completed a connection establishment to the other radio access technology.

[TS 25.331, Annex E]

This annex contains the definitions of the bits in EUTRA Feature Group Indicators.

In this release of the specification the UE shall include the optional EUTRA Feature Group Indicators IE in the UE multi-mode/multi RAT capability IE of the RRC message. For a specific indicator, if all functionalities for a feature group listed in Table E.1 have been implemented and tested, the UE shall set the indicator as "true" (as one), else (i.e. if any one of the functionalities in a feature group listed in Table E.1 have not been implemented or tested), the UE shall set the indicator as zero.

The UE shall set all indicators, which do not have a definition in the table E.1, as "false" (as zero).

If the optional EUTRA Feature Group Indicators IE is not included by a UE supporting a future release of the specification, the network may assume the UE supports all features in the feature groups listed in table E.1 and deployed in the network.

Table E.1: Definitions of feature group indicators

| Index of indicator (bit number) | Definition (description of the supported functionality, if indicator set to ‘true’) | Notes |
| --- | --- | --- |
| 1 (leftmost bit) | - UTRA CELL_PCH to EUTRA RRC_IDLE cell reselection - UTRA URA_PCH to EUTRA RRC_IDLE cell reselection |  |
| 2 | EUTRAN measurements and reporting in connected mode |  |
| 3 | Undefined |  |
| 4 | Undefined |  |

Clarification for mobility to EUTRAN

For mobility to E-UTRAN, it is assumed that we have 6 main "functions":

A.	Support of measurements and cell reselection procedure in idle mode

B.	Support of measurements and cell reselection procedure in CELL/URA_PCH

C.	Support of RRC release with redirection procedure

D.	Support of RRC reject with redirection procedure

E.	Support of EUTRAN measurements and reporting in connected mode

F.	Support of handover procedure in connected mode

Of the above, all UEs that indicate support of E-UTRA in UE capability signalling "Support of E-UTRA FDD" or "Support of E-UTRA TDD" support A), C) and D) EUTRA bands.

For B) above, UEs indicate support by the Group 1 indicator bit (if Group 1 is set to "true", it is supported for all EUTRA bands the UE supports).

For E) above, UEs indicate support by the Group 2 indicator bit (if Group 2 is set to "true", it is supported for all EUTRA bands the UE supports). The compressed mode capability for supported E-UTRA frequency bands ("Need for compressed mode") in "Measurement capability extension" is ignored by the network when Group 2 is set to "false".

For F) above, UEs indicate support by the separate UE capability signalling "Support of Inter-RAT PS Handover to E-UTRA FDD/TDD" defined in TS 25.306 (if this bit is set to "true", PS handover is supported for all EUTRA bands the UE supports). This bit can only be set to "true" if the UE has set the Group 2 indicator bit to "true".

6.2.3.4a.3	Test description

6.2.3.4a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	Cell 1 suitable neighbour E-UTRA cell

-	Cell 5 UTRA serving cell

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

-	The cell power level and parameter for Cell 5 is as follow:

| Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- |
| CPICH_Ec | dBm/3.84 MHz | -60 |  |
| P-CCPCH | dBm/1.28 MHz | -62 |  |
| Qrxlevmin (FDD) | dBm | -79 | Default value |
| Qrxlevmin(TDD) | dBm | -81 | Default value |

6.2.3.4a.3.2	Test procedure sequence

Tables 6.2.3.3a.3.2-1 & Table 6.2.3.3a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells applied at the points indicated in the Main behaviour description in Table 6.2.3.4a.3.2-3.

Table 6.2.3.3a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -75 | Steps 1-3, SqualnonServingCell,1 < Threshx,high2(26 dB) Steps 4-7, SqualnonServingCell,1 >Threshx,high2(4 dB) |
|  | RSRQ | dB | -3 |  |
|  | Qrxlevmin | dBm | -84 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | off |  |
| Note: The downlink signal level uncertainty is specified in TS 36.508 section 4.3.4.1 |  |  |  |  |

Table 6.2.3.3a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | -72 |  |
|  | CPICH_Ec/Io | dB | -2.89 |  |
|  | QqualminFDD | dB | -20 |  |
|  | Ioc | dBm/3.84MHz | off |  |

Table 6.2.3.4a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.4a.3.2-1 and 6.2.3.4a.3.2-2. | - | - | - | - |
| 2 | The UE is brought to state CELL_PCH (state 6-12) on Cell 5 according to [5] with modified SIB-19 info so that Cell 1 Squal cell 1 < threshXhigh2 according to table 6.2.3.4a.3.2-1 | - | - | - |  |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is NOT camped on E-UTRAN Cell 1? (Note 2) | - | - | - | - |
| 4 | The SS changes the SIB-19 info so that for Cell 1 Squalcell1> threshXhigh2 according to table 6.2.3.4a.3.2-2. | - | - | - | - |
| 5 | Notify UE change of System Information. | <- | - PAGING TYPE 1 |  |  |
| 6 | Wait for 6 s for UE to receive system information. | - | - | - |  |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? (Note 2) | - | - | 1 | - |
| - | Exception: Steps 8 to 8a5/8b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 8 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 8a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 8a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 8a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 8a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 8a5 | The SS releases the RRC connection | - | - | - | - |
| 8b1 | The 10s timer expires | - | - | - | - |
| Note 1: Void Note 2: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.2.3.4a.3.3	Specific message contents

Table 6.2.3.4a.3.3-0: Conditions for specific message contents
in Table 6.2.3.4a.3.3-1 and Table 6.2.3.4a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.4a.3.3-1: System Information Block type 19 for Cell 5 ( at preamble, Table 6.2.3.4a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |  | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |  |  |  |
| Priority |  | 3 |  |  |  |  |
| s-PrioritySearch1 |  | 2 (4 dB) |  |  |  |  |
| s-PrioritySearch2 |  | 0 (0 dB) |  |  |  |  |
| threshServingLow |  | 2 (4 dB) |  |  |  |  |
| } |  |  |  |  |  |  |
| utran-FDD-FrequencyList |  | Not present |  |  |  | UTRA-FDD |
| utran-TDD-FrequencyList |  | Not present |  |  |  | UTRA-TDD |
| } |  |  |  |  |  |  |
| gsm-PriorityInfoList |  | Not present |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |  |  |  |
| earfcn [n] |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  | Not present |  |  |  |  |
| priority [n] |  | 5 |  |  |  |  |
| qRxlevMinEUTRA [n] |  | -84 (-168 dBm) |  |  |  |  |
| qQualminEUTRA [n] |  | -20 |  |  |  |  |
| eutra-exclude-ListedCellList [n] |  | Not present |  |  |  |  |
| eutraDetection [n] |  | TRUE |  |  |  |  |
| } |  |  |  |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |  |  |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |  |  |  |  |  |
| threshServingLow2 |  | 8 (8dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |  |
| qQualMinEUTRA[n] |  | -20 (-20dB) |  |  |  |  |  |  |
| threshXhigh2[n] |  | 26 (26dB) |  |  |  |  |  |  |
| threshXlow2[n] |  | 4 (4dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| nonCriticalExtensions SEQUENCE {} |  | Not present |  |  |  |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |
| earfcn [n] |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  | Not present |  |  |  |  |
| priority [n] |  | 5 |  |  |  |  |
| qRxlevMinEUTRA [n] |  | -84 (-168 dBm) |  |  |  |  |
| qQualminEUTRA [n] |  | -20 |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.4a.3.3-2: System Information Block type 19 for Cell 5 ( at step 4, Table 6.2.3.4a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| qRxlevMinEUTRA [n] | -84 (164 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| qQualminEUTRA [n] | -20 |  |  |
| eutra-exclude-ListedCellList [n] | Not present |  |  |
| eutraDetection [n] | TRUE |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | -20 (-20dB) |  |  |
| threshXhigh2[n] | 4 (4dB) |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| qRxlevMinEUTRA [n] | -84 (164 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| qQualminEUTRA [n] | -20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.4a.3.3-3: PAGING TYPE 1 for Cell 5 (step 5, Table 6.2.2.4a.3.2-3)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.3.4a.3.3-4:SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.4a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4a.3.3-5:SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.4a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same as Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same as Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 6.2.3.5	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle

6.2.3.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the neighbour cell which belongs to the higher priority UTRAN frequency }

then { UE reselects the UTRA cell }

}

6.2.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

…

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. ...

6.2.3.5.3	Test description

6.2.3.5.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell :

-	Cell 1 is a E-UTRA cell

-	Cell 5 is a UTRA cell

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.5.3.2	Test procedure sequence

Table 6.2.3.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.2.3.5.3.2-2.

Table 6.2.3.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | -65 | S nonServingCell, Cell5 > Thresh Cell 5,high |
|  | P-CCPCH | dBm/1.28 MHz |  | -67 |  |

Table 6.2.3.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.5.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | - |

6.2.3.5.3.3	Specific message contents

Table 6.2.3.5.3.3-1: Void

Table 6.2.3.5.3.3-2: Void

Table 6.2.3.5.3.3-3: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.5.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.5.3.3-4: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.5.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.5a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle (Squal > ThreshX, HighQ, Squal < ThreshServing, LowQ, Squal > ThreshX, LowQ and SnonIntraSearchQ)

6.2.3.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT UTRA cell}

then { UE reselects the cell which belongs to the lower priority inter-RAT UTRA cell }

}

(2)

with { UE in E-UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT UTRA cell }

}

(3)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { threshX-HighQ-r9 is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the high priority UTRA cell upon Squal > threshX-HighQ-r9 }

}

6.2.3.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

…

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

...

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

…

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

...

Cell reselection to another RAT, for which Squal based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria if the UE supports Squal (RSRQ) based cell reselection to E-UTRAN from all the other RATs provided by system information which UE supports. Otherwise, cell reselection to another RAT shall be performed based on Srxlev criteria.

6.2.3.5a.3	Test description

6.2.3.5a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.5a.3.2	Test procedure sequence

Tables 6.2.3.5a.3.2-1 & Table 6.2.3.5a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.5a.3.2-3.

Table 6.2.3.5a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Squal ServingCell, Cell1 < ThreshServing, LowQ |
|  | RSRQ | dB | -3 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | off |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -80 |  |
|  | RSRQ | dB | -3 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | RSRQ | dB | -3 |  |

Table 6.2.3.5a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 | Note 1 |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Ioc | dBm/3.84MHz | Off |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -90 | Note 2 |
|  | P-CCPCH | dBm/1.28 MHz | -92 |  |
|  | CPICH_Ec/No | dB | off |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -65 | Note 3 |
|  | P-CCPCH | dBm/1.28 MHz | -67 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
| Note 1: Since no interference is occurred, SqualnonServingCell, Cell 5 > ThreshCell5, LowQ is satisfied, Note 2: Since no interference is occurred, SqualServingCell, Cell 5 < 0 is satisfied. Note 3: Since no interference is occurred, Squal nonServingCell, Cell5 > Thresh Cell5,highQ is satisfied, |  |  |  |  |

Table 6.2.3.5a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | Wait for 6 s for UE to receive system information. | - | - | - | - |  |
| 2 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.5a.3.2-1 and table 6.2.3.5a.3.2-2. | - | - | - | - |  |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs a RAU procedure and the RRC Connection is released. | - | - | 1 | - |  |
| - | Exception: Steps 3Aa1 to 3Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - |  | - |  |
| 3Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - |  | - |  |
| 3Aa1a1-3Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - |  | - |  |
| 3Aa1a4 | SS releases RRC Connection | - | - | - |  | - |  |
| 3Aa1a5 | SS Stops 6s timer | - | - | - |  | - |  |
| 3Aa1b1 | The 6s timer expires | - | - | - |  | - |  |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.5a.3.2-1 and table 6.2.3.5a.3.2-2. | - | - | - | - |  |
| 5A-5G | The UE selects E-UTRAN Cell 1 and performs the generic test procedure in TS 36.508 Table 6.4.2.7A-1 from steps 1-7. | - | - | - | - |  |
| - | EXCEPTION: Steps 5Ha1 to 5Ha2b1 describe optional behaviour if UE sends IMS deregistration at steps 3Aa1 to 3Aa1b1 | - | - | - |  | - |  |
| 5Ha1-5Ha2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - |  | - |  |
| 5I | The SS releases RRC Connection |  |  |  |  |  |  |
| 6 | The SS changes the UTRA cell priority broadcast in system information. | - | - | - | - |  |
| 7 | Notify UE change of System Information. | <-- | Paging | - | - |  |
| 8 | Wait for 6 s for UE to receive system information. | - | - | - | - |  |
| 9 | The SS changes Cell 1 and Cell 5 levels according to the row "T3" in table 6.2.3.5a.3.2-1 and table 6.2.3.5a.3.2-2. | - | - | - | - |  |
| 10-17 | Check: Does the test result of steps 1 to 8 generic test procedure in TS 36.508 Table 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs a RAU procedure | - | - | 2, 3 | - |  |

6.2.3.5a.3.3	Specific message contents

Table 6.2.3.5a.3.3-0: Conditions for specific message contents
in Table 6.2.3.5a.3.3-3 and Table 6.2.3.5a.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.5a.3.3-1:SystemInformationBlockType3 for Cell 1 (preamble, Table 6.2.3.5a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.5a.3.3-2:SystemInformationBlockType6 for Cell 1 (preamble, Table 6.2.3.5a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| threshX-High[n] | 13 (26 dB) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 24 (24 dB) |  |  |
| threshX-LowQ-r9 | 9 (9 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.5a.3.3-3: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.5a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 4 |  |  |
| qRxlevMinEUTRA [n] | -53 (-106 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| eutra-exclude-ListedCellList [n] | Not present |  |  |
| eutraDetection [n] | [TRUE] |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | -20 (-20dB) |  |  |
| threshXhigh2[n] | 24 (24dB) |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 4 |  |  |
| qRxlevMinEUTRA [n] | -53 (-106 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.5a.3.3-4: Void

Table 6.2.3.5a.3.3-5:SystemInformationBlockType6 for Cell 1 (step 6, Table 6.2.3.5a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| threshX-High[n] | 10 (20 dB) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 9 (9 dB) |  |  |
| threshX-LowQ-r9 | 24 (24 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.5a.3.3-6: System Information Block type 19 for Cell 5 (step 6, Table 6.2.3.5a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 4 |  |  |
| qRxlevMinEUTRA [n] | -53 (106 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| eutra-exclude-ListedCellList [n] | Not present |  |  |
| eutraDetection [n] | [TRUE] |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | -20 (-20dB) |  |  |
| threshXhigh2[n] | 8 (8dB) |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 4 |  |  |
| qRxlevMinEUTRA [n] | -53 (106 dBm) |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.5a.3.3-7: Paging (step 7, Table 6.2.3.5a.3.2-3)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | True |  |  |
| etws-PrimaryNotificationIndication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### 6.2.3.6	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle according to RAT priority provided by dedicated signalling

6.2.3.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority RAT }

then { UE reselects the cell which belongs to the higher priority RAT }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority RAT }

then { UE reselects the cell which belongs to the lower priority RAT }

}

(3)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { T320 expires }

then { UE reselects a cell by applying the cellReselectionPriority broadcast in the system information }

}

(4)

with { UE in E-UTRA RRC_IDLE state, and reselection priorities have been provided in dedicated signalling }

ensure that {

when { T320 has not expired }

then { The UE shall ignore all the priorities provided in system information, and not perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies for which the UE doesn’t have a priority provided }

}

6.2.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1 and 5.2.4.5 and TS 36.331, clause 5.3.8.3 and 5.3.8.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

- the UE enters RRC_CONNECTED state; or

- the optional validity time of dedicated priorities (T320) expires; or

- a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfills the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low  during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

...

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates 'loadBalancingTAURequired':

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[TS 36.331, clause 5.3.8.4]

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information;

6.2.3.6.3	Test description

6.2.3.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1(E-UTRA) and Cell 5(UTRA).

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

6.2.3.6.3.2	Test procedure sequence

Table 6.2.3.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3", "T4" and "T5" are to be applied subsequently. The exact instants on which these values are applied are described in the texts in this clause.

Table 6.2.3.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are assigned to satisfy SrxlevCell 1 > 0 and SrxlevCell 5 < 0. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | - | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that camping on Cell 1 is guaranteed. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power level values are assigned to satisfy Srxlevcell 1 < Threshserving, low and Threshx,low < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that camping on Cell 1 is guaranteed. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T5 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are assigned to satisfy Srxlevcell 1 > Threshserving, low and Threshx,high < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |

Table 6.2.3.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message with an idleModeMobilityControlInfo including a cellReselectionPriority on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | The SS changes Cell 5 level according to row "T1" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRC CONNECTION REQUEST message within the next 30s on Cell 5? | --> | RRC CONNECTION REQUEST | 4 | F |
| 4 | The SS transmits a Paging message including matched identity on Cell 1. | <-- | Paging | - | - |
| 5 | UE transmit an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 6 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 7 | The UE transmits an RRCConnectionSetupComplete on Cell 1. | --> | RRCConnectionSetupComplete | - | - |
| 8 | The SS transmits an RRCConnectionRelease message with an idleModeMobilityControlInfo including a cellReselectionPriority on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | - |
| - | Exception: Steps 9Aa1 to 9Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 9Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 9Aa1a1-9Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 9Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 9Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 9Aa1b1 | The 6s timer expires | - | - | - | - |
| 10 | The SS changes Cell 1 and Cell 5 parameters according to the row "T2" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 11A-11G | Generic test procedure in TS 36.508 subclause 6.4.2.7 A-1 from steps 1-7 is performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | Exception: Steps 11Ha1 to 11Ha2b1 describe optional behaviour if UE sends optional IMS deregistration at steps 9Aa1 to 9Aa1b1 | - | - | - | - |
| 11Ha1-11Ha2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 11I | The SS releases RRC Connection |  |  |  |  |
| 12 | The SS changes Cell 1 and Cell 5 parameters according to the row "T3" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 2 | - |
| - | Exception: Steps 13Aa1 to 13Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 13Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 13Aa1a1-13Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 13Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 13Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 13Aa1b1 | The 6s timer expires | - | - | - | - |
| 14 | The SS changes Cell 1 and Cell 5 parameters according to the row "T4" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 15A-15G | Generic test procedure in TS 36.508 subclause 6.4.2.7 A-1 from steps 1-7 are performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 15Ha1 to 15Ha2b1 describe optional behaviour if UE sends IMS deregistration at steps 13Aa1 to 13Aa1b1 | - | - | - | - |
| 15Ha1-15Ha2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed NOTE: Timer_1 value of 6s used in Step 10a1 of procedure. | - | - | - | - |
| 15I | The SS releases RRC Connection |  |  |  |  |
| 16 | The SS changes Cell 1 and Cell 5 parameters according to the row "T5" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 17 | Wait for 5 minutes from step 15 to ensure that T320 expires. | - | - | - | - |
| 18 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 3 | - |

6.2.3.6.3.3	Specific message contents

Table 6.2.3.6.3.3-1: Void

Table 6.2.3.6.3.3-2: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.6.3.3-3: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.6.3.3-4: RRCConnectionRelease (Step 1, Table 6.2.3.6.3.2-2)

| Derivation path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| IdleModeMobilityControlInfo ::= SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  | UTRA-TDD |
| freqPriorityListUTRA-TDD | Not present |  | UTRA-FDD |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-5: RRCConnectionRelease (step 8, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-6: RRCConnectionRelease (step 11, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-7: RRCConnectionRelease (step 15, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | min5 |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-8: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |

#### 6.2.3.7	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA

6.2.3.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and UE detects the cell re-selection criteria that HRPD cell is higher reselection priority than E-UTRAN serving cell }

ensure that {  when { UE searches for and measures HRPD cell at least every Thigher_priority_search

and Srxlev,HRPD > ThreshX, HighP }

then { UE reselects the HRPD cell }

}

6.2.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information , in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

[TS 36.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

If ThreshX-Q is provided, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If ThreshX-Q is provided, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

[TS 36.133, clause 4.2.2.5]

If the SServingCellof the E-UTRA serving cell is greater than Snonintrasearch, then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2.

If the SServingCellof the E-UTRA serving cell is less than or equal to Snonintrasearchthen the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.4]

In order to perform measurement and cell reselection to HRPD cell, the UE shall acquire the timing of HRPD cells.

When the measurement rules indicate that HRPD cells are to be measured, the UE shall measure CDMA2000 HRPD Pilot Strength of HRPD cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of HRPD Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all HRPD cells in the neighbour cell list.

When the RSRP of the E-UTRA serving cell (or other cells on the same frequency layer) is lower than ‘HRPD Start Measuring E-UTRAN Rx Power Strength Threshold’ and HRPD is of lower priority than the currently selected E-UTRAN frequency layer, the UE shall measure CDMA2000 HRPD Pilot Strength of the HRPD cells at least every (Number of HRPD Neighbour Frequency)*TmeasureHRPD.

The UE shall be capable of evaluating that the HRPD cell has met cell reselection criterion defined in [1] within TevaluateHRPD.

Table 4.2.2.5.4-1 gives values of TmeasureHRPD and TevaluateHRPD.

Table 4.2.2.5.4-1: TmeasureHRPD and TevaluateHRPD

| DRX cycle length [s] | TmeasureHRPD [s] (number of DRX cycles) | TevaluateHRPD [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

6.2.3.7.3	Test description

6.2.3.7.3.1	Pre-test conditions

System Simulator:

- Cell 1 for E-UTRAN

- Cell 15 for HRPD with higher reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.7.3.2	Test procedure sequence

Table 6.2.3.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and “T2” are applied at the point indicated in the Main behaviour description in Table 6.2.3.7.3.2-2.

Table 6.2.3.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | -20 | Cell 15 is on Srxlev,HRPD of Cell 15 < ThreshX, HighP |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -20 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 |  |  |
| T2 | Îor/Ioc | dB | - | 0 | Increase pilot power of HRPD cell such that Srxlev,HRPD of Cell 15 > ThreshX, HighP |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -3 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 |  |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.3.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | The SS changes Cell 15 level according to row "T1" in table 6.2.3.7.3.2-1. Cell 15 is on and it has higher reselection priority than Cell1. | - | - | - | - |
| 3 | UE performs measurement for Cell 15 at every Thigher_priority_search | - | - | - | - |
| 4 | Check: Does the UE transmit an Access Probe on Cell 15 within the next [60s]? | --> | Access Probe | 1 | F |
| 5 | SS adjust HRPD cell signal level to make sure SnonServingCell,HRPD of Cell 15 > ThreshHRPD, high, according to “T2” in table 6.2.3.7.3.2-2 | - | - | - | - |
| 6 | Check: Does the UE transmit an Access Probe on Cell 15? | --> | Access Probe | 1 | P |

6.2.3.7.3.3	Specific message contents

Table 6.2.3.7.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.7.3.2-1)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |

Table 6.2.3.7.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.7.3.2-1)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured HRPD carriers |  |  |
| bandClass | Band Class of frequency under test | [Set according to specific test case] |  |
| cellReselectionPriority | 5 | [3 is applicable when HRPD is lower priority than E-UTRA. 5 is applicable when HRPD is higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.7a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA (Srxlev > ThreshHRPD, HighP)

6.2.3.7a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE }

ensure that {

when { UE detects a HRPD cell with higher reselection priority than the E-UTRAN serving cell }

then { UE does not reselect the HRPD cell }

}

Note:   The UE is expected during this time to do measurements on the HRPD cell at least every Thigher_priority_search but this cannot be easily verified

(2)

with { UE in E-UTRA RRC_IDLE having detected a HRPD cell with higher reselection priority than the E-UTRAN serving cell } ensure that {

when { Srxlev,HRPD > ThreshHRPD, HighP }

then { UE reselects the HRPD cell }

}

6.2.3.7a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

- the UE enters RRC_CONNECTED state; or

- the optional validity time of dedicated priorities (T320) expires; or

- a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.4]

In order to perform measurement and cell reselection to HRPD cell, the UE shall acquire the timing of HRPD cells.

When the measurement rules indicate that HRPD cells are to be measured, the UE shall measure CDMA2000 HRPD Pilot Strength of HRPD cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of HRPD Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all HRPD cells in the neighbour cell list.

When the E-UTRA serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for CDMA2000 HRPD layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 HRPD cells which have been detected, the UE shall measure CDMA2000 HRPD Pilot Strength at least every (Number of HRPD Neighbour Frequency)*TmeasureHRPD, when the E-UTRA serving cell Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

The UE shall be capable of evaluating that the CDMA2000 HRPD cell has met cell reselection criterion defined in [1] within TevaluateHRPD.

Table 4.2.2.5.4-1 gives values of TmeasureHRPD and TevaluateHRPD.

Table 4.2.2.5.4-1: TmeasureHRPD and TevaluateHRPD

| DRX cycle length [s] | TmeasureHRPD [s] (number of DRX cycles) | TevaluateHRPD [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 HRPD cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this CDMA2000 HRPD cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

6.2.3.7a.3	Test description

6.2.3.7a.3.1	Pre-test conditions

System Simulator:

- Cell 1 for E-UTRAN

- Cell 15 for HRPD with higher reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.7a.3.2	Test procedure sequence

Table 6.2.3.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and “T2” are applied at the point indicated in the Main behaviour description in Table 6.2.3.7.3.2-2.

Table 6.2.3.7a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | -20 | Cell 15 is on Srxlev,HRPD of Cell 15 < ThreshHRPD,HighP |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -20 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 |  |  |
| T2 | Îor/Ioc | dB | - | 0 | Increase pilot power of HRPD cell such that Srxlev,HRPD of Cell 15 > ThreshHRPD,HighP |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -3 |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 |  |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.3.7a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 15 level according to row "T1" in table 6.2.3.7a.3.2-1. Cell 15 is on and it has higher reselection priority than Cell1. | - | - | - | - |
| 2 | UE performs measurement for Cell 15 at every Thigher_priority_search | - | - | - | - |
| 3 | Check: Does the UE transmit an Access Probe on Cell 15 within the next [60s]? | --> | Access Probe | 1 | F |
| 4 | SS adjust HRPD cell signal level to make sure Srxlev of Cell 15 > ThreshHRPD, HighP, according to “T2” in table 6.2.3.7a.3.2-1 | - | - | - | - |
| 5 | Check: Does the UE transmit an Access Probe on Cell 15? | --> | Access Probe | 2 | P |
| Note 1: The UE is expected during this time to do measurements on the HRPD cell at least every Thigher_priority_search but this cannot be easily verified. |  |  |  |  |  |

6.2.3.7a.3.3	Specific message contents

Table 6.2.3.7a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.7.3.2-2)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |

Table 6.2.3.7a.3.3-2: Void

Table 6.2.3.7a.3.3-3: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.7a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured HRPD carriers |  |  |
| bandClass | Band Class of frequency under test | [Set according to specific test case] |  |
| cellReselectionPriority | 5 | [3 is applicable when HRPD is lower priority than E-UTRA. 5 is applicable when HRPD is higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.8	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD is lower reselection priority than E-UTRA

6.2.3.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and UE detects the cell re-selection criteria that HRPD cell is equial to or lower reselection priority than E-UTRAN serving cell }

ensure that {

When{ SServingCell <= ThreshServing, LowP and Srxlev > ThreshHRPD, low }

then { UE searches for and measures HRPD cell at least every TmeasureHRPD and reselects the HRPD cell }

}

6.2.3.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information or in the RRC message releasing the RRC connection. If priorities are assigned via dedicated signalling, the UE shall ignore all the priorities provided in system information. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

UE shall only perform reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

[TS 36.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

-	If SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with a equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequencies of equal or lower priority.

-	If SServingCell <= Snonintrasearch  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequencies cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

If ThreshX-Q is provided, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If ThreshX-Q is provided, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

[TS 36.133, clause 4.2.2.5]

If the SServingCellof the E-UTRA serving cell (or other cells on the same frequency layer) is greater than Snonintrasearch, then

-	the UE may not search for, or measure inter-RAT layers of equal or lower priority.

-	the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2.

If the SServingCellof the E-UTRA serving cell is less than or equal to Snonintrasearch, then the UE shall search for and measure inter-RAT layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure such layers is not reduced and shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.4]

In order to perform measurement and cell reselection to HRPD cell, the UE shall acquire the timing of HRPD cells.

When the measurement rules indicate that HRPD cells are to be measured, the UE shall measure CDMA2000 HRPD Pilot Strength of HRPD cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of HRPD Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all HRPD cells in the neighbour cell list.

When the RSRP of the E-UTRA serving cell (or other cells on the same frequency layer) is lower than ‘HRPD Start Measuring E-UTRAN Rx Power Strength Threshold’ and HRPD is of lower priority than the currently selected E-UTRAN frequency layer, the UE shall measure CDMA2000 HRPD Pilot Strength of the HRPD cells at least every (Number of HRPD Neighbour Frequency)*TmeasureHRPD. In case HRPD is of higher priority than the currently selected E-UTRAN frequency layer the UE shall measure HRPD cells at least every (Number of HRPD Neighbour Frequency)* Thigher_proirty_search. The parameter Thigher_proirty_search is defined in section 4.2.2.

The UE shall be capable of evaluating that the HRPD cell has met cell reselection criterion defined in [1] within TevaluateHRPD.

Table 4.2.2.5.4-1 gives values of TmeasureHRPD and TevaluateHRPD.

Table 4.2.2.5.4-1: TmeasureHRPD and TevaluateHRPD

| DRX cycle length [s] | TmeasureHRPD [s] (number of DRX cycles) | TevaluateHRPD [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

6.2.3.8.3	Test description

6.2.3.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1 for E-UTRAN

-	Cell 15 for HRPD with lower reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.8.3.2	Test procedure sequence

Table 6.2.3.8.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and “T2” are applied at the point indicated in the Main behaviour description in Table 6.2.3.8.3.2-2.

Table 6.2.3.8.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | -20 | Cell 15 is on |
|  | Ioc | dBm/ 1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -20 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -70 | - | SServingCellof the E-UTRA > ThreshServing, LowP and Srxlev,HRPD of Cell 15 < Threshx, low |
|  | Srxlev (Note 1) | dB | 40 | -40 |  |
| T2 | Îor/Ioc | dB | - | -5 | Increase pilot power of HRPD cell such that SnonServingCell,HRPD of Cell 15 > Threshx, low |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -6.2 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -120 | - |  |
|  | Srxlev (Note 1) | dB | -10 | -12 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. Note 2: Default value of ThreshServing, LowP is 0. |  |  |  |  |  |

Table 6.2.3.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 15 is on and it has lower reselection priority than Cell 1, according to “T1” in 6.2.3.8.3-1. SS adjusts the serving cell signal level so that SServingCellof the E-UTRA <= Snonintrasearch.2 | - | - | - | - |
| 2 | Void |  |  |  |  |
| 3 | UE performs measurement for Cell 15 at every TmeasureHRPD | - | - | - | - |
| 4 | Check: Does the UE transmit an Access Probe on Cell 15 within the next [60s]? | --> | Access Probe | 1 | F |
| 5 | SS adjust HRPD cell signal level to make sure SnonServingCell,HRPD of Cell 15 > ThreshHRPD, low according to “T2” in 6.2.3.8.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit an Access Probe on Cell 15? | --> | Access Probe | 1 | P |

6.2.3.8.3.3	Specific message contents

Table 6.2.3.8.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.8.3.2-1)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.8.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.8.3.2-1)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured HRPD carriers |  |  |
| bandClass | Band Class of frequency under test | [Set according to specific test case] |  |
| cellReselectionPriority | 3 | [3 is applicable when HRPD is lower priority than E-UTRA. 5 is applicable when HRPD is higher priority than E-UTRA] |  |
| threshX-High | 30[-30] | INTEGER (0..63) |  |
| threshX-Low | 32[-32] | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.8a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > ThreshHRPD, LowP

6.2.3.8a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE }

ensure that {

when { UE detects a HRPD cell with equal or lower reselection priority than the E-UTRAN serving cell }

then { UE does not reselect the HRPD cell }

}

Note:   The UE is expected during this time to do measurements on

the HRPD cell at least every TmeasureHRPD but this cannot be easily verified

(2)

with { UE in E-UTRA RRC_IDLE having detected a HRPD cell with equal or lower reselection priority than the E-UTRAN serving cell }

ensure that {

when { Srxlev, HRPD > ThreshHRPD, LowP }

then { UE reselects the HRPD cell }

}

6.2.3.8a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.4]

In order to perform measurement and cell reselection to HRPD cell, the UE shall acquire the timing of HRPD cells.

When the measurement rules indicate that HRPD cells are to be measured, the UE shall measure CDMA2000 HRPD Pilot Strength of HRPD cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of HRPD Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all HRPD cells in the neighbour cell list.

When the E-UTRA serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for CDMA2000 HRPD layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 HRPD cells which have been detected, the UE shall measure CDMA2000 HRPD Pilot Strength at least every (Number of HRPD Neighbour Frequency)*TmeasureHRPD, when the E-UTRA serving cell Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

The UE shall be capable of evaluating that the CDMA2000 HRPD cell has met cell reselection criterion defined in [1] within TevaluateHRPD.

Table 4.2.2.5.4-1 gives values of TmeasureHRPD and TevaluateHRPD.

Table 4.2.2.5.4-1: TmeasureHRPD and TevaluateHRPD

| DRX cycle length [s] | TmeasureHRPD [s] (number of DRX cycles) | TevaluateHRPD [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 HRPD cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this CDMA2000 HRPD cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

6.2.3.8a.3	Test description

6.2.3.8a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 for E-UTRAN

-	Cell 15 for HRPD with lower reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.8a.3.2	Test procedure sequence

Table 6.2.3.8a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and “T2” are applied at the point indicated in the Main behaviour description in Table 6.2.3.8.3.2-2.

Table 6.2.3.8a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Îor/Ioc | dB | - | -20 | Cell 15 is on |
|  | Ioc | dBm/ 1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -20 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -70 | - | SServingCellof the E-UTRA > ThreshServing, LowP and Srxlev,HRPD of Cell 15 < ThreshHRPD, LowP |
|  | Srxlev (Note 1) | dB | 40 | -40 |  |
| T2 | Îor/Ioc | dB | - | -5 | Increase pilot power of HRPD cell such that SnonServingCell,HRPD of Cell 15 > ThreshHRPD,LowP |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -6.2 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -120 | - |  |
|  | Srxlev (Note 1) | dB | -10 | -12 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. Note 2: Default value of ThreshServing, LowP is 0. |  |  |  |  |  |

Table 6.2.3.8a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 15 is on and it has lower reselection priority than Cell 1, according to “T1” in 6.2.3.8a.3.2-1. SS adjusts the serving cell signal level so that E-UTRA serving cell Squal <= SnonIntraSearchQ | - | - | - | - |
| 3 | UE performs measurement for Cell 15 at every TmeasureHRPD | - | - | - | - |
| 4 | Check: Does the UE transmit an Access Probe on Cell 15 within the next [60s]? | --> | Access Probe | 1 | F |
| 5 | SS adjust HRPD cell signal level to make sure Srxlev of Cell 15 > ThreshHRPD, LowP according to “T2” in 6.2.3.8a.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit an Access Probe on Cell 15? | --> | Access Probe | 2 | P |
| Note 1: The UE is expected during this time to do measurements on the HRPD cell at least every TmeasureHRPD but this cannot be easily verified. |  |  |  |  |  |

6.2.3.8a.3.3	Specific message contents

Table 6.2.3.8a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.8a.3.2-2)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.8a.3.3-2: SystemInformationBlockType3 for Cell 1 (Preamble and all steps, table 6.2.3.8a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 (10dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.8a.3.3-3: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.8a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured HRPD carriers |  |  |
| bandClass | Band Class of frequency under test | [Set according to specific test case] |  |
| cellReselectionPriority | 3 | [3 is applicable when HRPD is lower priority than E-UTRA. 5 is applicable when HRPD is higher priority than E-UTRA] |  |
| threshX-High | 30[-30] | INTEGER (0..63) |  |
| threshX-Low | 32[-32] | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.9	Inter-RAT Cell reselection: from E-UTRA RRC_IDLE to CDMA2000 1xRTT Dormant- When CDMA2000 1xRTT cell is higher reselection priority than E-UTRA

6.2.3.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE and UE detects a 1xRTT cell with higher reselection priority than E-UTRAN serving cell }

ensure that {

when { UE determines that SnonServingCell,CDMA2000_1x <= Thresh1xRTT, high }

then { UE does not reselect the CDMA2000 1xRTT cell }

}

(2)

with { UE in E-UTRA RRC_IDLE and UE detects a 1xRTT cell with higher reselection priority than E-UTRAN serving cell }

ensure that {

when { SnonServingCell,CDMA2000_1x > Thresh1xRTT, high }

then { UE reselects the CDMA2000 1xRTT cell }

}

6.2.3.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

If ThreshX-Q is provided, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If ThreshX-Q is provided, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.5]

In order to perform measurement and cell reselection to cdma2000 1X cell, the UE shall acquire the timing of cdma2000 1X cells.

When the measurement rules indicate that cdma2000 1X cells are to be measured, the UE shall measure cdma2000 1x RTT Pilot Strength of cdma2000 1X cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of CDMA2000 1X Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all cdma2000 1X cells in the neighbour cell list.

If SServingCell of the E-UTRA serving cell is greater than Snonintrasearch, the UE shall search for cdma2000 1X layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 1X cells which have been detected, the UE shall measure CDMA2000 1xRTT Pilot Strength at least every (Number of CDMA2000 1X Neighbour Frequency)*TmeasureCDMA2000_1X, when the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch.

The UE shall be capable of evaluating that the cdma2000 1X cell has met cell reselection criterion defined in [1] within TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1 gives values of TmeasureCDMA2000_1X and TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1: TmeasureCDMA2000 1X and TevaluateCDMA2000 1X

| DRX cycle length [s] | TmeasureCDMA2000_1X [s] (number of DRX cycles) | TevaluateCDMA2000_1X [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 1X cell is better ranked than the serving cell, the UE shall evaluate this CDMA2000 1X cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

6.2.3.9.3	Test description

6.2.3.9.3.1	Pre-test conditions

System Simulator:

- Cell 1 for E-UTRAN

- Cell 19 for CDMA2000 1xRTT with higher reselection priority than Cell 1

-	System information combination 46 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.2.3.9.3.2	Test procedure sequence

Table 6.2.3.9.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.9.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15 KHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
| T1 | Cell-specific RS EPRE | dBm/15 KHz | -75 |  | Cell 19 is on |
|  | Îor/Ioc | dB |  | -15 |  |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -22 |  |
| T2 | Cell-specific RS EPRE | dBm/15 KHz | -75 |  | Increase pilot power of 1xRTT cell such that SnonServingCell, of Cell 19 > Thresh1xRTT, high |
|  | Îor/Ioc | dB |  | 0 |  |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -10 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.3.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 19 is on and it has higher reselection priority than Cell1, according to “T1” in 6.2.3.9.3.2-1 | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE transmit an Access Probe on Cell 19 within the next 60s? | --> | Access Probe | 1 | F |
| 4 | SS adjust CDMA2000 1xRTT cell signal level to make sure SnonServingCell, 1xRTT of Cell 19 > Thresh1xRTT, high, according to “T2” in 6.2.3.9.3.2-1 | - | - | - | - |
| 5 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 2 | P |

6.2.3.9.3.3	Specific message contents

Table 6.2.3.9.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.9.3.2-1)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.9.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.9.3.2-1)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured 1xRTT carriers |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 5 | [3 is applicable when 1xRTT is lower priority than E-UTRA. 5 is applicable when 1xRTTis higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.9a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is higher reselection priority than E-UTRA (Srxlev > Thresh1xRTT, HighP)

6.2.3.9a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE }

ensure that {

when { UE detects a 1xRTT cell with higher reselection priority than the E-UTRAN serving cell }

then { UE does not reselect the 1xRTT cell }

}

Note:   The UE is expected during this time to do measurements on the 1xRTT cell at least every Thigher_priority_search but this cannot be easily verified.

(2)

with { UE in E-UTRA RRC_IDLE having detected a 1xRTT cell with higher reselection priority than the E-UTRAN serving cell } ensure that {

when { Srxlev,CDMA2000_1x > Thresh1xRTT, HighP }

then { UE reselects the CDMA2000 1xRTT cell }

}

6.2.3.9a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.5]

In order to perform measurement and cell reselection to cdma2000 1X cell, the UE shall acquire the timing of cdma2000 1X cells.

When the measurement rules indicate that cdma2000 1X cells are to be measured, the UE shall measure cdma2000 1x RTT Pilot Strength of cdma2000 1X cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of CDMA2000 1X Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all cdma2000 1X cells in the neighbour cell list.

When the E-UTRA serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for cdma2000 1X layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 1X cells which have been detected, the UE shall measure CDMA2000 1xRTT Pilot Strength at least every (Number of CDMA2000 1X Neighbour Frequency)*TmeasureCDMA2000_1X, when the E-UTRA serving cell Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. The UE shall be capable of evaluating that the cdma2000 1X cell has met cell reselection criterion defined in [1] within TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1 gives values of TmeasureCDMA2000_1X and TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1: TmeasureCDMA2000 1X and TevaluateCDMA2000 1X

| DRX cycle length [s] | TmeasureCDMA2000_1X [s] (number of DRX cycles) | TevaluateCDMA2000_1X [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 1X cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this CDMA2000 1X cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

6.2.3.9a.3	Test description

6.2.3.9a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 for E-UTRAN

-	Cell 19 for CDMA2000 1xRTT with higher reselection priority than Cell 1

-	System information combination 46 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.2.3.9a.3.2	Test procedure sequence

Table 6.2.3.9.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.9a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15 KHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
| T1 | Cell-specific RS EPRE | dBm/15 KHz | -75 |  | Cell 19 is on |
|  | Îor/Ioc | dB |  | -15 |  |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -22 |  |
| T2 | Cell-specific RS EPRE | dBm/15 KHz | -75 |  | Increase pilot power of 1xRTT cell such that SnonServingCell, of Cell 19 > Thresh1xRTT, HighP |
|  | Îor/Ioc | dB |  | 0 |  |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -10 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.2.3.9a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 19 is on and it has higher reselection priority than Cell1, according to “T1” in 6.2.3.9a.3.2-1 | - | - | - | - |
| 2 | UE performs measurement for Cell 19 at every Thigher_priority_search | - | - | - | - |
| 3 | Check: Does the UE transmit an Access Probe on Cell 19 within the next [60s]? | --> | Access Probe | 1 | F |
| 4 | SS adjust CDMA2000 1xRTT cell signal level to make sure Srxlev of Cell 19 > Thresh1xRTT, HighP, according to “T2” in 6.2.3.9a.3.2-1 | - | - | - | - |
| 5 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 2 | P |
| Note 1: The UE is expected during this time to do measurements on the HRPD cell at least every Thigher_priority_search but this cannot be easily verified. |  |  |  |  |  |

6.2.3.9a.3.3	Specific message contents

Table 6.2.3.9a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.9a.3.2-2)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.9a.3.3-2: Void

Table 6.2.3.9a.3.3-3: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.9a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured 1xRTT carriers |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 5 | [3 is applicable when 1xRTT is lower priority than E-UTRA. 5 is applicable when 1xRTTis higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.10	Inter-RAT Cell reselection: from E-UTRA RRC_IDLE to CDMA2000 1xRTT Idle - When CDMA2000 1xRTT is lower reselection priority than E-UTRA

6.2.3.10.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and UE detects the cell re-selection criteria that CDMA2000 1xRTT cell is equial to or lower reselection priority than E-UTRAN serving cell }

ensure that {

When{ UE determines that SServingCell <= Snonintrasearch }

then { UE searches for and measures CDMA2000 1xRTT cell at least every TmeasureCDMA2000_1X }

when { UE determines that SnonServingCell,1xRTT > Thresh1xRTT, low }

then { UE reselects the CDMA2000 1xRTT cell }

}

6.2.3.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

If ThreshX-Q is provided, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If ThreshX-Q is provided, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If the SServingCellof the E-UTRA serving cell (or other cells on the same frequency layer) is greater than Snonintrasearch, then

-	the UE may not search for, or measure inter-RAT layers of equal or lower priority.

-	the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2.

If the SServingCellof the E-UTRA serving cell is less than or equal to Snonintrasearch, then the UE shall search for and measure inter-RAT layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure such layers is not reduced and shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.5]

In order to perform measurement and cell reselection to cdma2000 1X cell, the UE shall acquire the timing of cdma2000 1X cells.

When the measurement rules indicate that cdma2000 1X cells are to be measured, the UE shall measure cdma2000 1x RTT Pilot Strength of cdma2000 1X cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of CDMA2000 1X Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all cdma2000 1X cells in the neighbour cell list.

If SServingCell of the E-UTRA serving cell is greater than Snonintrasearch, the UE shall search for cdma2000 1X layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 1X cells which have been detected, the UE shall measure CDMA2000 1xRTT Pilot Strength at least every (Number of CDMA2000 1X Neighbour Frequency)*TmeasureCDMA2000_1X, when the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch.

The UE shall be capable of evaluating that the cdma2000 1X cell has met cell reselection criterion defined in [1] within TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1 gives values of TmeasureCDMA2000_1X and TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1: TmeasureCDMA2000 1X and TevaluateCDMA2000 1X

| DRX cycle length [s] | TmeasureCDMA2000_1X [s] (number of DRX cycles) | TevaluateCDMA2000_1X [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 1X cell is better ranked than the serving cell, the UE shall evaluate this CDMA2000 1X cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

6.2.3.10.3	Test description

6.2.3.10.3.1	Pre-test conditions

System Simulator:

- Cell 1 for E-UTRAN

- Cell 19 for CDMA2000 1xRTT with lower reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.2.3.10.3.2	Test procedure sequence

Table 6.2.3.10.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.10.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15 KHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
| T1 | Cell-specific RS EPRE | dBm/15 KHz | -100 | - | Cell 19 is on and SnonServingCell, of Cell 19 < Thresh1xRTT, Low |
|  | Îor/Ioc | dB | - | -15 |  |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -22 |  |
| T2 | Cell-specific RS EPRE | dBm/15 KHz | -120 | - | Increase pilot power of 1xRTT cell such that SnonServingCell, of Cell 19 > Thresh1xRTT, Low and SServingCellof the E-UTRA < ThreshServing, LowP |
|  | Îor/Ioc | dB | - | 0 |  |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -10 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. Note 2: Default value of ThreshServing, LowP is 0. |  |  |  |  |  |

Table 6.2.3.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 19 is on and it has lower reselection priority than Cell 1, according to “T1” in 6.2.3.10.3.2-1 SS adjust the serving cell signal level so that SServingCellof the E-UTRA <= Snonintrasearch | - | - | - | - |
| 2 | UE performs measurement for Cell 19 at every TmeasureCDMA2000_1X | - | - |  |  |
| 3 | Check: Does the UE transmit an Access Probe on Cell 19 within the next [60s]? | --> | Access Probe | 1 | F |
| 4 | SS adjust CDMA2000 1xRTT Cell 19 signal level to make sure SnonServingCell,1xRTT of Cell 19 > Thresh1xRTT, low, according to “T2” in 6.2.3.10.3.2-1 |  |  |  |  |
| 5 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 1 | P |

6.2.3.10.3.3	Specific message contents

Table 6.2.3.10.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.10.3.2-1)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.10.3.2-1)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured 1xRTT carriers |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 3 | [3 is applicable when 1xRTT is lower priority than E-UTRA. 5 is applicable when 1xRTTis higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.10a	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > Thresh1xRTT, LowP)

6.2.3.10a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE }

ensure that {

when { UE detects a CDMA2000 1xRTT cell with equal or lower reselection priority than the E-UTRAN serving cell }

then { UE does not reselect the a CDMA2000 1xRTT  cell }

}

Note:   The UE is expected during this time to do measurements on the CDMA2000 1xRTT  cell at least every TmeasureCDMA2000_1X but this cannot be easily verified

(2)

with { UE in E-UTRA RRC_IDLE having detected a HRPD cell with equal or lower reselection priority than the E-UTRAN serving cell } ensure that {

when { Srxlev,1xRTT > Thresh1xRTT, LowP }

then { UE reselects the CDMA2000 1xRTT  cell }

}

6.2.3.10a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5 and TS 36.133, clause 4.2.2.5 and 4.2.2.5.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 36.133, clause 4.2.2.5]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

[TS 36.133, clause 4.2.2.5.5]

In order to perform measurement and cell reselection to cdma2000 1X cell, the UE shall acquire the timing of cdma2000 1X cells.

When the measurement rules indicate that cdma2000 1X cells are to be measured, the UE shall measure cdma2000 1x RTT Pilot Strength of cdma2000 1X cells in the neighbour cell list at the minimum measurement rate specified in this section.

The parameter ‘Number of CDMA2000 1X Neighbour Frequency’, which is transmitted on E-UTRAN BCCH, is the number of carriers used for all cdma2000 1X cells in the neighbour cell list.

When the E-UTRA serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for cdma2000 1X layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is defined in section 4.2.2.

For CDMA2000 1X cells which have been detected, the UE shall measure CDMA2000 1xRTT Pilot Strength at least every (Number of CDMA2000 1X Neighbour Frequency)*TmeasureCDMA2000_1X, when the E-UTRA serving cell Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. The UE shall be capable of evaluating that the cdma2000 1X cell has met cell reselection criterion defined in [1] within TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1 gives values of TmeasureCDMA2000_1X and TevaluateCDMA2000_1X.

Table 4.2.2.5.5-1: TmeasureCDMA2000 1X and TevaluateCDMA2000 1X

| DRX cycle length [s] | TmeasureCDMA2000_1X [s] (number of DRX cycles) | TevaluateCDMA2000_1X [s] (number of DRX cycles) |
| --- | --- | --- |
| 0.32 | 5.12 (16) | 15.36 (48) |
| 0.64 | 5.12 (8) | 15.36 (24) |
| 1.28 | 6.4 (5) | 19.2 (15) |
| 2.56 | 7.68 (3) | 23.04 (9) |

If Treselection timer has a non zero value and the CDMA2000 1X cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this CDMA2000 1X cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

6.2.3.10a.3	Test description

6.2.3.10a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 for E-UTRAN

-	Cell 19 for CDMA2000 1xRTT with lower reselection priority than Cell 1

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.2.3.10a.3.2	Test procedure sequence

Table 6.2.3.10.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.10a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15 KHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
| T1 | Cell-specific RS EPRE | dBm/15 KHz | -100 | - | Cell 19 is on and SnonServingCell, of Cell 19 < Thresh1xRTT, LowP |
|  | Îor/Ioc | dB | - | -15 |  |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -22 |  |
| T2 | Cell-specific RS EPRE | dBm/15 KHz | -120 | - | Increase pilot power of 1xRTT cell such that SnonServingCell, of Cell 19 > Thresh1xRTT, LowP and SServingCellof the E-UTRA < ThreshServing, LowP |
|  | Îor/Ioc | dB | - | 0 |  |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | Pilot Ec/ Io (Note 1) | dB | - | -10 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. Note 2: Default value of ThreshServing, LowP is 0. |  |  |  |  |  |

Table 6.2.3.10a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cell 19 is on and it has lower reselection priority than Cell 1, according to “T1” in 6.2.3.10a.3.2-1 SS adjust the serving cell signal level so that E-UTRA serving cell Squal <= SnonIntraSearchQ | - | - | - | - |
| 2 | UE performs measurement for Cell 19 at every TmeasureCDMA2000_1X | - | - |  |  |
| 3 | Check: Does the UE transmit an Access Probe on Cell 19 within the next [60s]? | --> | Access Probe | 1 | F |
| 4 | SS adjust CDMA2000 1xRTT Cell 19 signal level to make sure Srxlev of Cell 19 > Thresh1xRTT, low, according to “T2” in 6.2.3.10a.3.2-1 |  |  |  |  |
| 5 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 2 | P |
| Note 1: The UE is expected during this time to do measurements on the HRPD cell at least every TmeasureCDMA2000_1X but this cannot be easily verified. |  |  |  |  |  |

6.2.3.10a.3.3	Specific message contents

Table 6.2.3.10a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.10.3.2-2)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10a.3.3-2: SystemInformationBlockType3 for Cell 1 (Preamble and all steps, table 6.2.3.10a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 (10dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10a.3.3-3: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.3.10a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured 1xRTT carriers |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 3 | [3 is applicable when 1xRTT is lower priority than E-UTRA. 5 is applicable when 1xRTTis higher priority than E-UTRA] |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 7 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.11	Void

#### 6.2.3.12	Void

#### 6.2.3.13	Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE according to RAT priority provided by dedicated signalling

6.2.3.13.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell based on the configured RAT priority provided by dedicated signaling }

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state having inherit RAT priority provided by dedicated signaling with the remaining validity time }

ensure that {

when { UE discard the inherit RAT priority upon connection establishment }

then { UE reselects a cell by applying the cellReselectionPriority broadcast in the system information }

}

6.2.3.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a; TS 25.331, clause 8.3.3.3 and 8.6.7.23; TS 36.331, clause 5.3.3.4; TS 25.133, clause 4.2.2.5a

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

…

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and [T3230, FFS] in GERAN), if configured, at inter-RAT cell (re)selection.

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

…

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 25.331, clause 8.3.3.3]

When the UE receives a UTRAN MOBILITY INFORMATION message, it shall:

1>	act on received information elements as specified in subclause 8.6;

1>	if the IE "Dedicated Priority Information" is present:

2>	act upon the received IE as described in subclause 8.6.7.23.

[TS 25.331, clause 8.6.7.23]

If the CHOICE "Action" has the value "Configure dedicated priorities", the UE shall:

1>	clear the variable PRIORITY_INFO_LIST;

1>	stop timer T322, if it is running;

1>	set the value of IE "Priority status" in the variable PRIORITY_INFO_LIST to "dedicated_priority";

1>	for each occurrence of the IE "Priority Level List":

2>	create a new entry in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST, and in that new entry:

3>	set the CHOICE "Radio Access Technology" to the value received in the IE "Priority Level List";

3>	set the IE "priority" to the value received in the IE "Priority Level List";

3>	set the values in IE "Frequency List" or "BCCH ARFCN List" to the values received in the IE "Priority Level List".

1>	set the IE "E-UTRA detection" to the value received in the IE "Priority Level List".

1>	if the IE "T322" is present:

2>	start timer T322 using the value signalled in this IE.

1>	if the UE is not in CELL_DCH state:

2>	take the actions as described in subclause 8.1.1.6.19 using stored System information Block type 19.

[TS 36.331, clause 5.3.3.4]

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

...

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 25.133, clause 4.2.2.5a]

…

The UE shall be able to evaluate whether a new detectable lower priority inter-RAT E-UTRA cell meets the reselection criteria defined in [1] within Kcarrier * TdetectE-UTRA, where TdetectE-UTRA is given in Table 4.2, if E-UTRA carrier frequency information is provided in the inter-RAT measurement control system information when Treselection=0 provided that the reselection criteria is met by at least 6dB. The parameter Kcarrier is the number of E-UTRA carrier frequencies indicated in the inter-RAT measurement control system information. An inter RAT E-UTRAN cell is considered to be detectable if:

-	RSRP|dBm according to Annex B.1.1 for a corresponding Band

…

For an inter-RAT E-UTRA cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the E-UTRA cell has met reselection criterion defined TS 36.304 within Kcarrier * TevaluateEUTRA as specified in table 4.2 provided that the reselection criteria is met by at least 6dB and the Treselection timer is set to zero.

6.2.3.13.3	Test description

6.2.3.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	Cell 1 suitable neighbour E-UTRA cell

-	Cell 5 UTRA serving cell

NOTE: Common Priority information for Cell 1 & Cell 5: UTRA priority = 3; E-UTRA priority = 4.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

- 	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH_DCH (state 6-10) according to [5].

6.2.3.13.3.2	Test procedure sequence

Tables 6.2.3.13.3.2-1 and 6.2.3.13.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while row marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.13.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -97 |  |
|  | Qrxlevmin | dBm | -106 | Default value |
|  | Srxlev* | dB | 9 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA S nonServingCell, Cell1 > Thresh Cell1,low |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -97 |  |
|  | Qrxlevmin | dBm | -106 | Default value |
|  | Srxlev* | dB | 9 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.13.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 | The power levels are such that camping on Cell 5 is guaranteed. |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin | dBm | -79 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -75 |  |
|  | P-CCPCH | dBm/1.28 MHz | -77 |  |
|  | Srxlev* | dB | 4 | SrxlevServingCell < Thresh Serving,low |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.13.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS provides dedicated priority information (UTRA priority = 5; E-UTRA priority = 4) and validity timer to the UE. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2 | The SS receives the UTRAN MOBILITY INFOMRATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 3 | The SS releases the UE connection. | <-- | RRC CONNECTION RELEASE | - | - |
| 4 | The SS receives UE release complete message. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 5 | The SS receives UE release complete message. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 6 | The SS receives UE release complete message. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 7 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.13.3.2-1 and table 6.2.3.13.3.2-2. | - | - | - | - |
| 8 | Check: does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |
| - | Exception: Steps 8A to 8Aa5/8Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 8A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 8Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 8Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 8Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 8Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 8Aa5 | The SS releases the RRC connection | - | - | - | - |
| 8Ab1 | The 10s timer expires | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 11 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.13.3.2-1 and table 6.2.3.13.3.2-2. | - | - | - | - |
| 12 | Check: does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 2 | - |

6.2.3.13.3.3 		Specific message contents

Table 6.2.3.13.3.3-0: Conditions for specific message contents
in tables 6.2.3.13.3.3-1 and 6.2.3.13.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.13.3.3-1: UTRAN MOBILITY INFORMATION for Cell 5 (step 1, Table 6.2.3.13.3.2-3)

| Derivation Path: 34.108, clause 9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UTRANMobilityInformation ::= CHOICE { |  |  |  |
| later-than-r3 SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| r7 SEQUENCE { |  |  |  |
| v860NonCriticalExtensions SEQUENCE { |  |  |  |
| utranMobilityInformation-v860ext SEQUENCE { |  |  |  |
| dedicatedPriorityInformation SEQUENCE { |  |  |  |
| action CHOICE { |  |  |  |
| configureDedicatedPriorities SEQUENCE { |  |  |  |
| t-322 | 5 | Time in minutes |  |
| priorityLevelList SEQUENCE (SIZE (1..maxPrio)) OF SEQUENCE { | 2 entry |  |  |
| priority[1] | 5 |  |  |
| radioAccessTechnology[1] CHOICE { |  |  |  |
| utraFDD SEQUENCE (SIZE (1..maxNumFDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| utraTDD SEQUENCE (SIZE (1..maxNumTDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| priority[2] | 4 |  |  |
| radioAccessTechnology[2] CHOICE { |  |  |  |
| eutra SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { | 1 entry |  |  |
| earfcn | Same downlink EARFCN as used for Cell 1 |  |  |
| earfcn | 65535 |  | Band>64 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| E-UTRA detection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vb50NonCriticalExtentions SEQUENCE { |  |  | Band > 64 |
| utranMobilityInformation-vb50ext SEQUENCE { |  |  |  |
| dedicatedPriorityInformation SEQUENCE { |  |  |  |
| action CHOICE { |  |  |  |
| configureDedicatedPriorities SEQUENCE { |  |  |  |
| t-322 | 5 |  |  |
| priorityLevelList SEQUENCE (SIZE (1..maxPrio)) OF SEQUENCE { | 2 entries |  |  |
| priority[1] | 5 |  |  |
| radioAccessTechnology[1] CHOICE { |  |  |  |
| utraFDD SEQUENCE (SIZE (1..maxNumFDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| utraTDD SEQUENCE (SIZE (1..maxNumTDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| priority[2] | 4 |  |  |
| radioAccessTechnology[2] CHOICE { |  |  |  |
| eutra SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF EARFCNRange { | 1 entry |  |  |
| earfcn | 65535 |  |  |
| earfcnEXT | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.13.3.3-2: RRC CONNECTION RELEASE for Cell 5 (step 3, Table 6.2.3.13.3.2-3)

| Derivation Path: 34.108, clause 9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= CHOICE { |  |  |  |
| later-than-r3 SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| r4 SEQUENCE { |  |  |  |
| rrcConnectionRelease-r4 SEQUENCE { |  |  |  |
| n-308 | 2 | 2 (for CELL_DCH state). Not Present (for UE in other connected mode states). |  |
| releaseCause | normalEvent |  |  |
| rplmn-information | Not Present |  |  |
| } |  |  |  |
| v4d0NonCriticalExtensions SEQUENCE {} | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.13.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.13.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| s-PrioritySearch1 | 8 (16 dB) |  |  |
| threshServingLow | 4 (8 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| qRxLevMinEUTRA [n] | -53 (-106 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 6 (12 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| qRxLevMinEUTRA [n] | -53 (-106 dBm) |  |  |
| threshXhigh [n] | 16 (32 dB) |  |  |
| threshXlow [n] | 6 (12 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.13.3.3-4: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.13.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 (20dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.13.3.3-5: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.13.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 6.2.3.14	Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority of E-UTRA cells are higher than the serving cell)

6.2.3.14.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Priority of E-UTRA neighbour cell is set lower than GERAN cell }

then { UE does not reselects the lower priority E-UTRA neighbour cell }

}

(2)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Priority of E-UTRA neighbour cell is set higher than GERAN cell }

then { UE reselects the higher priority inter-RAT E-UTRA neighbour cell }

}

6.2.3.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 10.1.3.3 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 10.1.3.3]

Cell reselection to a cell of another inter-RAT frequency shall be performed if any of the conditions below (to be evaluated in the order shown) is satisfied:

-	The S_non-serving_XXX of one or more cells of a higher priority inter-RAT frequency is greater than THRESH_XXX_high (or, in case of an E-UTRAN target, THRESH_E-UTRAN_high_Q, if provided) during a time interval T_reselection; in that case, the mobile station shall consider the cells for reselection in decreasing order of priority and, for cells of the same inter-RAT frequency or of inter-RAT frequencies of equal priority, in decreasing order of S_non-serving_XXX, and reselect the first cell that satisfies the conditions above;

...

If the mobile station applies either common priorities or individual priorities received through dedicated signalling and priorities are available only for some inter-RAT frequencies, cells belonging to frequencies for which no priority is available or no threshold is provided by the serving cell shall not be considered for measurement and for cell re-selection.

If a mobile station in camped normally state (see 3GPP TS 43.022) applies individual priorities received through dedicated signalling and no priority is available for the serving cell, the mobile station shall consider any GSM cell (including the serving cell) to have lowest priority (i.e. lower than the eight network configured values).

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.14.3	Test description

6.2.3.14.3.1	Pre-test conditions

System Simulator:

-	Two cells:

-	One GERAN cell, Cell 24 is serving cell.

-	One E-UTRAN cell, Cell 1 is Suitable neighbour inter-frequency cell.

UE:

None.

Preamble:

-	The UE is in state Registered, IDLE (GPRS) on Cell 24 (3GPP TS 51.010-1 clause 41.2.8.1.1) and 36.508[18] clause 4.5A.3B

6.2.3.14.3.2	Test procedure sequence

Table 6.2.3.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 75 s (70 s + T_reselect)? | --> | RRCConnectionRequest | 1 | F |
| 2 | The SS changes SI2Quater in Cell 24 according to table 6.2.3.14.3.3-2. | - | - | - | - |
| 3 | The SS changes SI13 in Cell 24 according to table 6.2.3.14.3.3-3. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.14.3.3	Specific message contents

Table 6.2.3.14.3.3-1: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in Preamble

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 5 |  |  |
| THRESH_Priority_Search | 0 |  |  |
| THRESH_GSM_low | 0 |  |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | 4 |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value =4 dB |  |
| THRESH_E-UTRAN_low | ‘00010’B | Actual value =4 dB |  |
| } |  |  |  |

Table 6.2.3.14.3.3-2: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in table 6.2.3.14.3.2-1 step 2

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 2QUATER REST OCTETS |  |  |  |
| MP Change Mark | 1 |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘110’B | 6 |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value =4 dB |  |
| THRESH_E-UTRAN_low | ‘00010’B | Actual value =4 dB |  |
| } |  |  |  |

Table 6.2.3.14.3.3-3: SI13 for Cell 24 in table 6.2.3.14.3.2-1 step 3

| Derivation Path: 51.010-1 clause 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets ::= { |  |  |  |
| BCCH_CHANGE_MARK | ‘001’B |  |  |
| SI_CHANGE_FIELD | ‘0010’B | Update of SI2, SI2 bis or SI2 ter message or any instance of SI2quater messages. |  |
| } |  |  |  |

#### 6.2.3.15	Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority of E-UTRA cells are lower than the serving cell)

6.2.3.15.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Srxleveutran < THRESH_E-UTRAN_low for the lower priority inter-RAT E-UTRA cell }

then { UE does not reselects the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Srxleveutran > THRESH_E-UTRAN_low for the lower priority inter-RAT E-UTRA cell }

then { UE reselects the lower priority inter-RAT E-UTRA cell }

}

6.2.3.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 10.1.3.3 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 10.1.3.3]

Cell reselection to a cell of another inter-RAT frequency shall be performed if any of the conditions below (to be evaluated in the order shown) is satisfied:

...

-	The value of S_serving is lower than THRESH_serving_low for the serving cell and all measured GSM cells; in this case, the mobile station shall consider for reselection the inter-RAT cells in the following order, and reselect the first one that satisfies the following criteria:

-	cells of a lower priority inter-RAT frequency whose S_non-serving_XXX is greater than THRESH_XXX_low during a time interval T_reselection; these cells shall be considered in decreasing order of priority and, for cells of the same RAT, in decreasing order of S_non-serving_XXX;

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.15.3	Test description

6.2.3.15.3.1	Pre-test conditions

System Simulator:

- Two cells:

- One GERAN cell, Cell 24 is serving cell.

- One E-UTRAN cell, Cell 1 is Suitable neighbour inter-frequency cell.

None.

Preamble:

-	The UE is in state Registered, IDLE (GPRS) on Cell 24 (3GPP TS 51.010-1 clause 41.2.8.1.1 and TS 36.508 [18] clause 4.5A.3B).

6.2.3.15.3.2	Test procedure sequence

Table 6.2.3.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 30 s (25 s+T_reselect)? | --> | RRCConnectionRequest | 1 | F |
| 2 | The SS changes SI2Quater in Cell 24 according to table 6.2.3.15.3.3-2. SrxlevCell 1 > THRESH_E-UTRAN_low. | - | - | - | - |
| 3 | The SS changes SI13 in Cell 24 according to table 6.2.3.15.3.3-3. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.15.3.3	Specific message contents

Table 6.2.3.15.3.3-0: Band > 64 Condition Table

| Condition | Explanation |
| --- | --- |
| Band > 64 | If Band > 64 is selected |

Table 6.2.3.15.3.3-1: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in Preamble

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= {8 |  |  |  |
| GERAN_PRIORITY | 7 |  |  |
| THRESH_Priority_Search | 0 |  |  |
| THRESH_GSM_low | 15 | MS is always allowed to reselect to lower priority cells |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as Cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘001’B |  |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value = 4 dB |  |
| THRESH_E-UTRAN_low | ‘10010’B | Actual value = 36 dB |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104 |  |
| } |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.15.3.3-2: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in table 6.2.3.15.3.2-1 step 2

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 2QUATER REST OCTETS |  |  |  |
| 3G BA Indicator | 1 |  |  |
| MP Change Mark | 1 |  |  |
| Serving Cell Priority Parameters Description ::= { |  |  |  |
| THRESH GSM low | 15 | MS is always allowed to reselect to lower priority cells |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as Cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘001’B |  |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value = 4 dB |  |
| THRESH_E-UTRAN_low | ‘00010’B | Actual value = 4 dB |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.15.3.3-3: SI13 for Cell 24 in table 6.2.3.15.3.2-1 step 3

| Derivation Path: 51.010-1 clause 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets ::= { |  |  |  |
| BCCH_CHANGE_MARK | ‘001’B |  |  |
| SI_CHANGE_FIELD | ‘0010’B | Update of SI2, SI2 bis or SI2 ter message or any instance of SI2quater messages. |  |
| } |  |  |  |

#### 6.2.3.16	Inter-RAT Cell reselection / from GSM_Idle to E-UTRAN /based on H_PRIO criteria

6.2.3.16.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Srxleveutran < THRESH_E-UTRAN_low for the lower priority inter-RAT E-UTRA cell and H_PRIO is off }

then { UE does not reselects the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Srxleveutran < THRESH_E-UTRAN_low for the lower priority inter-RAT E-UTRA cell and when Srxleveutran is higher than ServingCell by at least H_PRIO}

then { UE reselects the  inter-RAT E-UTRA cell based on H_PRIO}

}

6.2.3.16.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.6.6 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 6.6.6]

Cell reselection to a cell of another inter-RAT frequency shall be performed if any of the conditions below (to be evaluated in the order shown) is satisfied:

...

-	The value of S_GSM is lower than THRESH_GSM_low for the serving cell and all measured GSM cells during a time interval T_reselection; in this case, the mobile station shall consider for reselection the inter-RAT cells in the following order, and reselect the first one that satisfies the following criteria:

-	cells of a lower priority inter-RAT frequency whose S_non-serving_XXX is greater than THRESH_XXX_low (or, in case of an E-UTRAN target, THRESH_E-UTRAN_low_Q, if provided) during a time interval T_reselection; these cells shall be considered in decreasing order of priority and, for cells of the same RAT, in decreasing order of S_non-serving_XXX;

-	if no cells satisfy the criterion above, inter-RAT cells for which, during a time interval T_reselection, S_non-serving_XXX is higher than S_GSM for the serving cell by at least a specific hysteresis H_PRIO; these cells shall be considered in decreasing order of S_non-serving_XXX.

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.16.3	Test description

6.2.3.16.3.1	Pre-test conditions

System Simulator:

- Two cells:

- One GERAN cell, Cell 24 is serving cell.

- One E-UTRAN cell, Cell 1.

None.

Preamble:

-	The UE is in state Registered, IDLE (GPRS) on Cell 24 (3GPP TS 51.010-1 clause 41.2.8.1.1 and TS 36.508 [18] clause 4.5A.3B).

6.2.3.16.3.2	Test procedure sequence

Table 6.2.3.16.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -80 | - |  |
|  | Qrxlevmin | dBm | -112 | - |  |
|  | Srxlev* | dB | 32 | - | Srxlev should be at least 4dB > C1 |
|  | RF Signal level | dBm | - | -80 |  |
|  | RXLEV_ACCESS_MIN | dBm | - | -101 |  |
|  | C1* | dB | - | 21 |  |
| Note: Srxlev and C1 is calculated in the UE |  |  |  |  |  |

Table 6.2.3.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 30s (25 s+T_reselect)? | --> | RRCConnectionRequest | 1 | F |
| 2 | The SS changes SI2Quater in Cell 24 according to table 6.2.3.16.3.3-2. H_PRIO=2 (4dB) | - | - | - | - |
| 3 | The SS changes SI13 in Cell 24 according to table 6.2.3.16.3.3-3. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | p |

6.2.3.16.3.3	Specific message contents

Table 6.2.3.16.3.3-1: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in Preamble

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 7 |  |  |
| THRESH_Priority_Search | 15 |  |  |
| THRESH_GSM_low | 15 | MS is always allowed to reselect to lower priority cells |  |
| H_PRIO | 0 | Rule disabled |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| E-UTRAN_PRIORITY | ‘001’B |  |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value = 4 dB |  |
| THRESH_E-UTRAN_low | ‘11110’B | Actual value = 62 dB |  |
| E-UTRAN_QRXLEVMIN | 14 (-112 dBm) |  |  |
| } |  |  |  |

Table 6.2.3.16.3.3-2: Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24 in table 6.2.3.16.3.2-1 step 2

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 2QUATER REST OCTETS |  |  |  |
| MP Change Mark | 1 |  |  |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 7 |  |  |
| THRESH_Priority_Search | 15 |  |  |
| THRESH_GSM_low | 15 | MS is always allowed to reselect to lower priority cells |  |
| H_PRIO | 2 | Reselect to inter-RAT cell which is at least 4dB higher than Serving cell |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| E-UTRAN_PRIORITY | ‘001’B |  |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value = 4 dB |  |
| THRESH_E-UTRAN_low | ‘11110’B | Actual value = 62 dB |  |
| E-UTRAN_QRXLEVMIN | 14 (-112 dBm) |  |  |
| } |  |  |  |

Table 6.2.3.16.3.3-3: SI13 for Cell 24 in table 6.2.3.16.3.2-1 step 3

| Derivation Path: 51.010-1 clause 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets ::= { |  |  |  |
| BCCH_CHANGE_MARK | ‘001’B |  |  |
| SI_CHANGE_FIELD | ‘0010’B | Update of SI2, SI2 bis or SI2 ter message or any instance of SI2quater messages. |  |
| } |  |  |  |

#### 6.2.3.17	Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority E-UTRA cells)

6.2.3.17.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which has priority}

then { UE does not reselects the inter-RAT E-UTRA cell without priority}

}

6.2.3.17.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.6.6 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 6.6.6]

The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list; the network may provide priority information if only UTRAN frequencies are included in the neighbour cell list. If priority information is available to the mobile station and the mobile station supports priority based inter-RAT cell re-selection, the algorithm in this subclause shall be used for inter-RAT reselection towards all RATs. The rules regarding which set of priorities is valid at any given time are defined in 3GPP TS 44.018.

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.17.3	Test description

6.2.3.17.3.1	Pre-test conditions

System Simulator:

-	Cell 24 is serving cell.

-	Cell 1 and Cell 3 off cells.

UE:

None.

Preamble:

-	The UE is in state Registered, IDLE (GPRS) on Cell 24 (3GPP TS 51.010-1 clause 41.2.8.1.1 and TS 36.508 [18] clause 4.5A.3B).

6.2.3.17.3.2	Test procedure sequence

Table 6.2.3.17.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell3 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | -80 | - |  |
|  | Qrxlevmin | dBm | -110 | -110 | - |  |
|  | Srxlev* | dB | 50 | 30 | - |  |
|  | RF Signal level | dBm |  |  | -80 |  |
|  | C1 | dB |  |  | 21 |  |
| Note: Srxlev and C1 is calculated in the UE |  |  |  |  |  |  |

Table 6.2.3.17.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.2.2.17.3.2-1 | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 1 | - |

6.2.3.17.3.3	Specific message contents

Table 6.2.3.17.3.3-1 Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24

| Derivation Path: 36.508 Table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 7 |  |  |
| THRESH_GSM_low | 15 | MS is always allowed to reselect to lower priority cells |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as Cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘0’ | Not Present |  |
| E-UTRAN_QRXLEVMIN | 15 (-110 dBm) |  |  |
| } { |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104 |  |
| } |  | End of Repeated EARFCN_extended |  |
| EARFCN | Same as Cell 3 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth} | Same as Cell 3 |  |  |
| E-UTRAN_PRIORITY | 2 |  |  |
| THRESH_E-UTRAN_low | ‘01010’B | Actual value = 20 dB |  |
| E-UTRAN_QRXLEVMIN | 15 (-110 dBm) |  |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as Cell 3 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104 |  |
| } |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 6.2.3.18	Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (Not allowed E-UTRA cells)

6.2.3.18.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Priority of E-UTRA neighbour cells are set higher than GERAN cell. Among two E-UTRA neighbor cells, the cell with higher priority is Not allowed }

then { UE ignores Not allowed E-UTRA neighbour cell

}

6.2.3.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.6.6 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 6.6.6]

Cell reselection to a cell of another inter-RAT frequency shall be performed if any of the conditions below (to be evaluated in the order shown) is satisfied:

-	The S_non-serving_XXX of one or more cells of a higher priority inter-RAT frequency is greater than THRESH_XXX_high (or, in case of an E-UTRAN target, THRESH_E-UTRAN_high_Q, if provided) during a time interval T_reselection; in that case, the mobile station shall consider the cells for reselection in decreasing order of priority and, for cells of the same inter-RAT frequency or of inter-RAT frequencies of equal priority, in decreasing order of S_non-serving_XXX, and reselect the first cell that satisfies the conditions above;

….

E-UTRAN cells which are included in the list of not allowed cells shall not be considered as candidates for cell reselection. If the strongest cells on a E-UTRAN frequency are included in the list of not allowed cells, the mobile station may reselect the strongest valid cell (see subclause 8.4.7) on that frequency.

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.18.3	Test description

6.2.3.18.3.1	Pre-test conditions

System Simulator:

- Cell 24 is serving cell.

- Cell 1 and Cell 3 off cells

None.

Preamble:

-	The UE is in state Registered, IDLE (GPRS) on Cell 24 (3GPP TS 51.010-1 clause 41.2.8.1.1 and TS 36.508 [18] clause 4.5A.3B).

6.2.3.18.3.2	Test procedure sequence

Table 6.2.3.18.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell3 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | OFF | - |  |
|  | Qrxlevmin | dBm | -110 | - | - |  |
|  | Srxlev* | dB | 50 | - | - |  |
|  | RF Signal level | dBm | - | - | -80 |  |
|  | C1 | - | - | - | 21 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | -80 | - |  |
|  | Qrxlevmin | dBm | -110 | -110 | - |  |
|  | Srxlev* | dB | 50 | 30 | - |  |
|  | RF Signal level | dBm | - | - | -80 |  |
|  | C1 | dB | - | - | 21 |  |
| Note: Srxlev and C1 is calculated in the UE |  |  |  |  |  |  |

Table 6.2.3.18.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.2.2.18.3.2-1 | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 75 s (70 s+T_reselect)? | --> | RRCConnectionRequest | 1 | F |
| 3 | SS adjusts cell levels according to row T2 of table 6.2.2.18.3.2-1 | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 3? | - | - | 1 | - |

6.2.3.18.3.3	Specific message contents

Table 6.2.3.18.3.3-1 Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24

| Derivation Path: 36.508 Table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| THRESH_Priority_Search | ‘0000’B | Actual value=0 |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as Cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘111’B | Actual value=7 |  |
| E-UTRAN_QRXLEVMIN | 15 (-110 dBm) |  |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as Cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104 |  |
| } |  | End of Repeated EARFCN_extended |  |
| { |  |  |  |
| EARFCN | Same as Cell 3 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as Cell 3 |  |  |
| E-UTRAN_PRIORITY | ‘101’B | Actual value=5 |  |
| E-UTRAN_QRXLEVMIN | 15 (-110 dBm) |  |  |
| } |  |  |  |
| Repeated E-UTRAN Not Allowed Cells struct > ::= { |  |  |  |
| Not Allowed Cells : < PCID Group IE >>{ |  |  |  |
| PCID} | ‘000000000’B | PCID=0, Meaning that the Cell 1 which have PCID=0 will not be allowed |  |
| E-UTRAN_FREQUENCY_INDEX | ‘000’B | Cell 1 |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as Cell 3 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 6.2.3.19	Redirection to E-UTRA upon the release of the CS connection

6.2.3.19.1	Test Purpose (TP)

(1)

with { UE in CS Active state }

ensure that {

when { UE receives CHANNEL RELEASE message including an IE Cell selection indicator after release of all TCH and SDCCH, or IE Individual Priorities including E-UTRAN carrier frequency of Cell 1}

then { UE enters RRC_IDLE state on E-UTRAN Carrier included in IE Cell selection indicator info }

}

6.2.3.19.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 44.018, clause 3.4.13.1.1, and TS 45.008, clause 6.7.1.

[TS 44.018, clause 3.4.13.1.1]

The CHANNEL RELEASE message may include the information element "Cell selection indicator after release of all TCH and SDCCH" which shall be used by the mobile station in its cell selection algorithm after release of all TCH and SDCCH (see 3GPP TS 45.008).

The CHANNEL RELEASE message may include Individual priorities IE to convey individual priorities information to the MS (see subclause 3.2.4). When the MS receives an Individual priorities IE it shall start an instance of timer T3230 with the value supplied in the Individual priorities IE.

If the network redirects a mobile station towards E-UTRAN, it proceeds as follows:

-	The network shall use the information element “Cell selection indicator after release of all TCH and SDCCH” in the channel release message for redirection towards an EARFCN in the E-UTRAN band numbered less than 65.

-	The network shall use the information element “individual priorities” in the channel release message for redirection towards an EARFCN in the E-UTRAN band numbered greater than 64.

[TS 45.008, clause 6.7.1]

In A/Gb mode, when the MS releases all TCHs or SDCCH and returns to idle mode, packet idle mode or MAC-Idle state, it shall, as quickly as possible, camp on the cell whose channel has just been released. Similarly in Iu mode, when the MS releases all DBPSCHs and returns to MAC-Idle state, it shall, as quickly as possible, camp on the cell whose channel has just been released. However, in both modes (A/Gb mode or Iu mode), if the CHANNEL RELEASE message contains a “cell selection indicator after release of all TCH and SDCCH” or “Individual priorities” (see TS 44.018), the MS shall as quickly as possible camp on an indicated GSM, UTRAN or E-UTRAN cell that has been identified by the CHANNEL RELEASE message. If UTRAN or E-UTRAN frequency only is indicated the MS shall as quickly as possible camp on a suitable cell of this frequency.

In case the “cell selection indicator after release of all TCHs and SDCCH” or “Individual priorities” is not present, then if the full (P)BCCH data for that cell was not decoded in the preceding 30s, the MS shall attempt to decode the full (P)BCCH data. Until the MS has decoded the (P)BCCH data required for determining the paging group, it shall also monitor all paging blocks on timeslot 0 of the BCCH carrier or, for GPRS if PCCCH exists and for Iu mode, on the PDCH indicated on BCCH for possible paging messages that might address it. If the MS receives a page before having decoded the full (P)BCCH data for the cell, the MS shall store the page and respond once the relevant (P)BCCH data has been decoded, provided that the cell is not barred and the MS's access class is allowed. Reception of full BCCH(BA) information is not required before responding to the page.

If the CHANNEL RELEASE does not contain a “cell selection indicator after release of all TCH and SDCCH” or “Individual priorities”, and the MS has the knowledge that the cell whose channel is being released is not suitable (see 3GPP TS 43.022 [11]), the MS is allowed to camp on any suitable cell.

If the CHANNEL RELEASE contains a “cell selection indicator after release of all TCH and SDCCH” or “Individual priorities” and

-	the MS cannot find a suitable cell from the indicated ones within 10 s, or

-	none of the indicated cells are suitable,

the MS is allowed to camp on any suitable cell.

NOTE:	The received signal level measurements on surrounding cells made during the last 5 seconds on the TCH or SDCCH in A/Gb mode, or on the DBPSCH in Iu mode, may be averaged and used, where possible, to speed up the process. However, it should be noted that the received signal level monitoring while on the TCH or SDCCH in A/Gb mode, or on the DBPSCH in Iu mode, is on carriers in BA (SACCH), while the carriers to be monitored for cell reselection are in BA (BCCH) or BA (GPRS).

After decoding the relevant (P)BCCH data the MS shall perform cell reselection as specified in 3GPP TS 43.022 [11].

6.2.3.19.3	Test description

6.2.3.19.3.1	Pre-test conditions

System Simulator:

-	2 cells, one GSM and one E-UTRA cell:

-	Cell 24 GSM serving cell

-	Cell 1 suitable neighbour E-UTRA cell

UE:

None

Preamble:

U10 Active state on cell 24.as per TS 51.010-1 clause 40.4.3.22 and 36.508 [18] clause 4.5A.3B.

6.2.3.19.3.2	Test procedure sequence

Table 6.2.3.19.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS initiate a disconnect call | <-- | DISCONNECT | - | - |
| 2 | UE sends RELEASE message | --> | RELEASE | - | - |
| 3 | SS sends RELEASE COMPLETE message | <-- | RELEASE COMPLETE | - | - |
| 4 | The SS transmits an CHANNEL RELEASE message with either IE, Cell selection indicator after release of all TCH and SDCCH or IE Individual Priorities including E-UTRAN carrier frequency of Cell 1 | <-- | CHANNEL RELEASE | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |

6.2.3.19.3.3	Specific message or IE contents

Table 6.2.3.19.3.3-1: Channel Release message (step 4, Table 6.2.3.19.3.2-1]

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Protocol Discriminator | 0110 | RR Management |  |
| Skip Indicator | 0000 |  |  |
| Message Type | 00001101 |  |  |
| RR Cause |  |  |  |
| - RR Cause Value | Normal event. |  |  |
| <Cell Selection Indicator after release of all TCH and SDCCH IE > ::= |  |  | Band <= 64 |
| E-UTRAN Description | 011 | { 1 <E-UTRAN Description : < E-UTRAN Description struct >> } |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| TARGET_PCID | Same as cell 1 |  |  |
| <Individual Priorities IE> ::= |  |  | Band > 64 |
| provide individual priorities | 1 |  |  |
| GERAN_PRIORITY | 000 |  |  |
| E-UTRAN IPP with extended EARFCNs Description | 00011 | { Additions in Rel-11, E-UTRAN IPP with extended EARFCNs Description struct} |  |
| EARFCN_extended |  | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| E-UTRAN_PRIORITY | 000 |  |  |

| Condition | Explanation |
| --- | --- |
| Band <= 64 | If band <= 64 is selected |
| Band > 64 | If band > 64 is selected |

#### 6.2.3.20	Void

#### 6.2.3.21	Inter-RAT Cell reselection / From GPRS Packet_Transfer (NC0 mode) to E-UTRA

6.2.3.21.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC0 mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.21.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 10.1.3.3 & TS 44.018, clause 3.4.1.2.1.1a.

[TS 45.008, clause 10.1.3.3]

The algorithm in this subclause shall be used for inter-RAT cell reselection if priority information is available to the MS and threshold information is provided by the network. The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list. The mobile station may apply either priorities broadcast in system information or individual priorities received through dedicated signalling. The rules regarding which set of priorities shall apply are defined in 3GPP TS 44.018 and 3GPP TS 44.060.

NOTE:	Throughout the specification, the phrase “neighbour cell list” will include also the E-UTRAN Neighbour Cell list and/or the GPRS E-UTRANNeighbour Cell list where appropriate.

If the GPRS 3G Cell Reselection list or the GPRS E-UTRAN Neighbour Cell list include frequencies of other radio access technologies, the MS shall, at least every 5 seconds update the value RLA_P for the serving cell and each of the at least 6 strongest non serving GSM cells.

The MS shall then reselect a suitable (see 3GPP TS 25.304 for UTRAN and 3GPP TS 36.304 for E-UTRAN) cell of another radio access technology if the criteria below are satisfied. S_non-serving_XXX is the measurement quantity of a non-serving inter-RAT cell and XXX indicates the other radio access technology/mode. S_non-serving_XXX is defined in subclause 6.6.6.

Cell reselection to a cell of another inter-RAT frequency shall be performed if any of the conditions below (to be evaluated in the order shown) is satisfied:

-	The S_non-serving_XXX of one or more cells of a higher priority inter-RAT frequency is greater than THRESH_XXX_high during a time interval T_reselection; in that case, the mobile station shall consider the cells for reselection in decreasing order of priority and, for cells of the same inter-RAT frequency, in decreasing order of S_non-serving_XXX, and reselect the first cell that satisfies the conditions above;

-	The value of S_serving is lower than THRESH_serving_low for the serving cell and all measured GSM cells; in this case, the mobile station shall consider for reselection the inter-RAT cells in the following order, and reselect the first one that satisfies the following criteria:

-	cells of a lower priority inter-RAT frequency whose S_non-serving_XXX is greater than THRESH_XXX_low during a time interval T_reselection; these cells shall be considered in decreasing order of priority and, for cells of the same RAT, in decreasing order of S_non-serving_XXX;

-	if no cells satisfy the criterion above, inter-RAT cells for which, during a time interval T_reselection, S_non-serving_XXX is higher than S_serving by at least a specific hysteresis H_PRIO; these cells shall be considered in decreasing order of S_non-serving_XXX.

A UTRAN FDD cell shall only be reselected if, in addition to the criteria above, its measured Ec/No value is equal to or greater than FDD_Qmin - FDD_Qmin_Offset..

Cell reselection to a cell of another radio access technology (e.g. UTRAN or E-UTRAN) shall not occur within [5] seconds after the MS has reselected a GSM cell from an inter-RAT cell if a suitable GSM cell can be found.

If the mobile station applies individual priorities received through dedicated signalling and priority information is available only for some inter-RAT frequencies, cells belonging to frequencies for which no individual priority is available or no threshold is broadcast in system information shall not be considered for measurement and for cell re-selection.

If a mobile station in camped normally state (see 3GPP TS 43.022) applies individual priorities received through dedicated signalling and no priority is available for the serving cell, the mobile station shall consider any GSM cell (including the serving cell) to have lowest priority (i.e. lower than the eight network configured values).

A mobile station in camped on any cell state (see 3GPP TS 43.022) shall ignore individual priorities received through dedicated signalling and shall apply priorities received from the system information of the serving cell while attempting to find a suitable cell. If the mobile station supports CS voice services, the MS shall avoid reselecting acceptable (but not suitable) E-UTRA cells regardless of the priorities provided in system information.

NOTE:	If the MS is camping on an acceptable cell, individual priorities are not discarded until an event leading to their deletion occurs.

In case of a reselection attempt towards a barred UTRAN cell, the MS shall abandon further reselection attempts towards this UTRAN cell as defined by the Tbarred value on the barred UTRAN cell (see 3GPP TS 25.331).

NOTE:	It is FFS whether a similar requirement should be added in case of a reselection attempt towards a barred E-UTRAN cell.

NOTE:	Requirements for cells belonging to “forbidden LAs for roaming” should be included here.

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The E-UTRAN Neighbour Cell list may contain up to 8 E-UTRAN frequencies. For each E-UTRAN frequency, zero or more E-UTRAN neighbour cells may be specified that are not allowed for cell reselection. The list of not allowed cells is defined in the Not Allowed Cells IEs.

Each EARFCN in each instance of the Repeated E-UTRAN Neighbour Cells IE is added to the E-UTRAN Neighbour Cell list in the order in which it is received.

6.2.3.21.3	Test description

6.2.3.21.3.1	Pre-test conditions

System Simulator:

2 cells, one GSM and one E-UTRA cell:

-	Cell 24 GSM serving cell

-	Cell 1 suitable neighbour E-UTRA cell Cell 1 is off.

UE:

None.

Preamble:

-	The UE is GPRS attached and the PDP context 2 activated according to [23]. and 36.508 [18] clause 4.5A.3B.

6.2.3.21.3.2	Test procedure sequence

Table 6.2.3.21.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.21.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 |  |
|  | Srxlev* | dB | 30 | SrxlevCell 1 >THRESH_E-UTRAN_high |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.21.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U -- S | Message |  |  |
| 1 | UE is brought into downlink packet transfer mode | - | - | - | - |
| - | EXCEPTION: Step 2 is repeated 5 times. | - | - | - | - |
| 2 | The SS sends one downlink data block | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.21.3.2-1 | - | - | - | - |
| - | EXCEPTION: Step 5 is repeated until the parallel behaviour in table 6.2.3.21.3.2-3 takes place. | - | - | - | - |
| 5 | The SS sends one downlink data block | - | - | - | - |

Table 6.2.3.21.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U -- S | Message |  |  |  |
| 1 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |  |
| - | Exception: Steps 2a1 to 2a1b1 describe optional behaviour that depend on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 2a1 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 2a1a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 2a1a2 | The 10s timer is stopped. | - | - | - | - |
| 2a1a3 | The SS establishes SRB2 and DRB | - | - | - | - |
| 2a1a4 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 2a1a5 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 2a1a6 | The SS releases the RRC connection | - | - | - | - |
| 2a1b1 | The 10s timer expires | - | - | - | - |

6.2.3.21.3.3	Specific message contents

Table 6.2.3.21.3.3-1: Void

#### 6.2.3.22	Void

#### 6.2.3.23	Inter-RAT Cell reselection from GPRS Packet transfer to E-UTRA in CCN Mode (PACKET CELL CHANGE CONTINUE)

6.2.3.23.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC1 mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell and in CCN mode }

then { UE leaves CCN mode, reselects the E-UTRA cell }

}

6.2.3.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008 section 6.6.6, 10.1.3.3,10.1.4 and  TS 44.060 section 5.5.1.1a.2 and 5.5.2.3

[TS 45.008, section 6.6.6 and 10.1.3.3]

The algorithm in this subclause shall be used for inter-RAT cell reselection if priorities are available to the MS and thresholds are provided by the network, and if the mobile station supports priority based inter-RAT cell re-selection and priority information for the serving cell is provided by the network. A mobile station supporting E-UTRAN shall support priority based inter-RAT cell re-selection towards all the supported RATs. A mobile station not supporting E-UTRAN and supporting UTRAN and supporting priority based reselection from UTRAN to GERAN shall support priority based inter-RAT cell re-selection towards UTRAN

The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list. If priority information is available to the mobile station and the mobile station supports priority based inter-RAT cell re-selection, the algorithm in this subclause shall be used for inter-RAT reselection towards all RATs.

If the 3G Cell Reselection list or the E-UTRAN Neighbour Cell list include frequencies of other radio access technologies, the MS shall, at least every 5 s update the value RLA_C for the serving cell and each of the at least 6 strongest non serving GSM cells.

The MS shall then reselect a suitable (see 3GPP TS 25.304 for UTRAN and 3GPP TS 36.304 for E-UTRAN) cell of another radio access technology if the criteria below are satisfied. S_non-serving_XXX is the measurement quantity of a non-serving inter-RAT cell and XXX indicates the other radio access technology/mode and is defined as follows:

-	for a E-UTRAN cell, is the measured RSRP value for the cell minus E-UTRAN_QRXLEVMIN for the cell’s frequency if THRESH_E-UTRAN_high_Q is not provided; otherwise, if THRESH_E-UTRAN_high_Q is provided, is the measured RSRQ value for the cell minus E-UTRAN_QQUALMIN for the cell’s frequency.

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control 	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060 section 5.5.1.1a.2 and 5.5.2.3.

6.2.3.23.3	Test description

6.2.3.23.3.1	Pre-test conditions

System Simulator:

-	Cell 24 is serving GERAN Cell

-	Cell 1 is OFF

UE:

None.

Preamble:

- The UE is GPRS attached and the PDP context 2 activated according to section 41.2.8.1.1 and 40.4.3.15 of TS 51010-1 and 36.508[18] clause 4.5A.3B.

6.2.3.23.3.2	Test procedure sequence

Table 6.2.3.23.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.23.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | No change | The power level is such that SrxlevCell 1 > 0 |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.3.23.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Uplink dynamic allocation two phase access according to TS 51.010 clause 40.4.3.9 using n=30000 octets of data | - | - | - | - |
| 2 | EXCEPTION: MS continues to transfer data and send measurement reports for cell 1 in PACKET MEASUREMENT REPORT in parallel to steps 3 to 4 | - | - | - | - |
| 3 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.23.3.2-1 | - | - | - | - |
| 4 | The UE transmits PACKET CELL CHANGE NOTIFICATION to E_UTRA cell on 24 | --> | PACKET CELL CHANGE NOTIFICATION |  |  |
| 5 | The SS sends PACKET CELL CHANGE CONTINUE for cell 1 as the target cell on cell 24 | <-- | PACKET CELL CHANGE CONTINUE | - | - |
| 6 | Check: Does UE send RRC CONNECTION REQUEST on cell1? | --> | RRCConnectionRequest | 1 | P |
| 7 | SS sends RRCConnectionSetup to the UE | <-- | RRCConnectionSetup | - | - |
| 8 | Check: Does the UE send RRCConnectionSetupComplete message on cell 1? | --> | RRCConnectionSetupComplete | 1 | P |
| Note: Uplink data transfer can be completed in any step after step 2 and before step 5 in Cell 24, or any step after step 5 in Cell 1. During step 2 to 5 new resources for uplink transfer may be request from UE according to 51.010-1 section 40.4.3.9. |  |  |  |  |  |

6.2.3.23.3.3	Specific message contents

Table 6.2.3.23.3.3-1: SI2Quater for Cell 24[Preamble]

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘111’B | 61.44 sec |  |
| } |  |  |  |
| Serving Cell Priority Parameters ::= { |  |  |  |
| GERAN_PRIORITY | ‘001’B | This filed specifies GERAN cell priority |  |
| } |  |  |  |
| GPRS E-UTRAN Measurement Parameters Description ::= { |  |  |  |
| Qsearch_P_E-UTRAN_Initial | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm,  (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm,  (never). Default value =  (never). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| E-UTRAN Parameters Description struct ::= { |  |  |  |
| E-UTRAN_CCN_ACTIVE | 1 | CCN is enabled in the cell |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.23.3.3-2: PACKET CELL CHANGE CONTINUE (step 5, Table 6.2.3.23.3.3-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Packet Cell Change Continue message content ::= |  |  |  |
| PAGE_MODE | ‘00’B | Normal Paging |  |
| Global TFI | TFI of the uplink TBF |  |  |

Table 6.2.3.23.3.3-3: SI 13 for Cell 24 (Preamble)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |

#### 6.2.3.24	Inter-RAT Cell reselection from GPRS Packet transfer to E-UTRA in CCN Mode (PACKET CELL CHANGE ORDER)

6.2.3.24.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC0 mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell and in CCN mode }

then { UE leaves CCN mode,reselects the E-UTRA cell }

}

6.2.3.24.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008 section 6.6.6, 10.1.3.3,10.1.4 and TS 44.060 section 5.5.1.1a.2 and 5.5.2.3

[TS 44.060, section 5.5.1.1a.2]

A mobile station, which has CCN Enabled, can enter CCN Mode.

The mobile station shall enable CCN when the following criteria are fulfilled:

-	the mobile station is camping on a cell (see 3GPP TS 45.008); and

-	the network indicates CCN ACTIVE/3G CCN ACTIVE/E-UTRAN CCN ACTIVE either in system information to all mobile stations in the cell or in an individual order to a certain mobile station; and

-	the mobile station is neither in dedicated mode nor Dual Transfer Mode; and

-	the mobile station is in NC0 or in NC1 mode; and

-	the mobile station is in Packet Transfer mode.

The CCN procedures and the criteria for entering and leaving CCN mode are specified in sub-clauses 8.8.2 and 8.8.3.

[TS 45.008, section 6.6.6 and 10.1.3.3]

The algorithm in this subclause shall be used for inter-RAT cell reselection if priorities are available to the MS and thresholds are provided by the network, and if the mobile station supports priority based inter-RAT cell re-selection and priority information for the serving cell is provided by the network. A mobile station supporting E-UTRAN shall support priority based inter-RAT cell re-selection towards all the supported RATs. A mobile station not supporting E-UTRAN and supporting UTRAN and supporting priority based reselection from UTRAN to GERAN shall support priority based inter-RAT cell re-selection towards UTRAN

The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list. If priority information is available to the mobile station and the mobile station supports priority based inter-RAT cell re-selection, the algorithm in this subclause shall be used for inter-RAT reselection towards all RATs.

If the 3G Cell Reselection list or the E-UTRAN Neighbour Cell list include frequencies of other radio access technologies, the MS shall, at least every 5 s update the value RLA_C for the serving cell and each of the at least 6 strongest non serving GSM cells.

The MS shall then reselect a suitable (see 3GPP TS 25.304 for UTRAN and 3GPP TS 36.304 for E-UTRAN) cell of another radio access technology if the criteria below are satisfied. S_non-serving_XXX is the measurement quantity of a non-serving inter-RAT cell and XXX indicates the other radio access technology/mode and is defined as follows:

-	for a E-UTRAN cell, is the measured RSRP value for the cell minus E-UTRAN_QRXLEVMIN for the cell’s frequency if THRESH_E-UTRAN_high_Q is not provided; otherwise, if THRESH_E-UTRAN_high_Q is provided, is the measured RSRQ value for the cell minus E-UTRAN_QQUALMIN for the cell’s frequency.

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control 	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060 section 5.5.1.1a.2 and 5.5.2.3.

6.2.3.24.3	Test description

6.2.3.24.3.1	Pre-test conditions

System Simulator:

-	Cell 24 is serving GERAN Cell

-	Cell 1 is OFF

UE:

None.

Preamble:

- The UE is GPRS attached and the PDP context 2 activated according to section 41.2.8.1.1 and 40.4.3.15 of TS 51010-1 and TS 36.508 [18] clause 4.5A.3B.

6.2.3.24.3.2	Test procedure sequence

Table 6.2.3.23.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.24.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | Same as before | The power level is such that SrxlevCell 1 > 0 |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.3.24.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Uplink dynamic allocation two phase access according to TS 51.010 clause 40.4.3.9 using n=30000 octets of data | - | - | - | - |
| - | EXCEPTION: MS continues to transfer data in parallel to steps 2 to 4 | - | - | - | - |
| 2 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.24.3.2-1 | - | - | - | - |
| 3 | The UE transmits PACKET CELL CHANGE NOTIFICATION to E_UTRA cell on 24 | --> | PACKET CELL CHANGE NOTIFICATION |  |  |
| 4 | The SS sends PACKET CELL CHANGE ORDER for cell 1 as the target cell on cell 24 | <-- | PACKET CELL CHANGE ORDER | - | - |
| 5 | Check: Does UE send RRC CONNECTION REQUEST on cell1? | --> | RRCConnectionRequest | 1 | P |
| 6 | SS sends RRCConnectionSetup to the UE | <-- | RRCConnectionSetup | - | - |
| 7 | Check: Does the UE send RRCConnectionSetupComplete message on cell 1? | --> | RRCConnectionSetupComplete | 1 | P |
| Note: Uplink data transfer can be completed in any step after step 2 and before step 4 in Cell 24, or any step after step 4 in Cell 1. During step 2 to 4 new resources for uplink transfer may be request from UE according to 51.010-1 section 40.4.3.9. |  |  |  |  |  |

6.2.3.24.3.3	Specific message contents

Table 6.2.3.24.3.3-0: Conditions for specific message contents
in Tables 6.2.3.24.3.3-1 and 6.2.3.24.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.24.3.3-1 Repeated E-UTRAN Neighbour Cells struct of SI2Quater for Cell 24[Preamble]

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters ::= { |  |  |  |
| GERAN_PRIORITY | ‘001’B | This filed specifies GERAN cell priority |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as cell 1 |  |  |
| } |  |  |  |
| E-UTRAN Parameters Description struct ::= { |  |  |  |
| E-UTRAN_CCN_ACTIVE | 1 | CCN is enabled in the cell |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.24.3.3-2: PACKET CELL CHANGE ORDER (step 5, Table 6.2.3.24.3.2-2)

| Information element | Value/remark | Condition |
| --- | --- | --- |
| < PAGE_MODE : bit (2) > | 00 (Normal Paging) |  |
| 0 ¦ 10 | 0 |  |
| < GLOBAL_TFI : Global TFI IE > | <5 bit Uplink TFI> |  |
| 0 ¦ 1 | 1 |  |
| Message Escape < IMMEDIATE_REL > 0¦1<UTRAN FDD Target cell IE> 0¦1<UTRAN TDD Target cell IE> Additions in Rel-5 0 ¦ 1 < G-RNTI extension Additions in Rel-8 0¦1<E-UTRAN Target cell IE> 0¦1<E-UTRAN Target cell IE> EARFCN 0 ¦ 1 < Measurement Bandwidth Physical Layer Cell identity 0 ¦ 1 < Individual Priorities Additions in Rel-11 0 ¦ 1 <E-UTRAN Target cell with extended EARFCN IE > EARFCN_extended 0 ¦ 1 < Measurement Bandwidth Physical Layer Cell Identity 0 ¦ 1 <E-UTRAN IPP with extended EARFCNs IE> | 00 1 (Immediate abort of operation in the old cell is required) 0 (not present ) 0 (not present ) 1 0 (not present ) 1 0 1 EARFCN of the cell 1 0 (not present ) PCID of the cell 1 0 (not present ) 1 1 EARFCN of the cell 1 0 (not present ) PCID of the cell 1 0 (not present ) | Band>64 Band>64 |

#### 6.2.3.25	Void

#### 6.2.3.26	Inter-RAT Autonomous Cell reselection GPRS Packet_transfer to E-UTRA (NC1 mode)

6.2.3.26.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC1 mode }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell }

then { UE performs autonomous,reselection to the E-UTRA cell }

}

6.2.3.26.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008 section 6.6.6, 10.1.3.3 and 10.1.4:

[TS 45.008, section 6.6.6 and 10.1.3.3]

The algorithm in this subclause shall be used for inter-RAT cell reselection if priorities are available to the MS and thresholds are provided by the network, and if the mobile station supports priority based inter-RAT cell re-selection and priority information for the serving cell is provided by the network. A mobile station supporting E-UTRAN shall support priority based inter-RAT cell re-selection towards all the supported RATs. A mobile station not supporting E-UTRAN and supporting UTRAN and supporting priority based reselection from UTRAN to GERAN shall support priority based inter-RAT cell re-selection towards UTRAN

The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list. If priority information is available to the mobile station and the mobile station supports priority based inter-RAT cell re-selection, the algorithm in this subclause shall be used for inter-RAT reselection towards all RATs.

If the 3G Cell Reselection list or the E-UTRAN Neighbour Cell list include frequencies of other radio access technologies, the MS shall, at least every 5 s update the value RLA_C for the serving cell and each of the at least 6 strongest non serving GSM cells.

The MS shall then reselect a suitable (see 3GPP TS 25.304 for UTRAN and 3GPP TS 36.304 for E-UTRAN) cell of another radio access technology if the criteria below are satisfied. S_non-serving_XXX is the measurement quantity of a non-serving inter-RAT cell and XXX indicates the other radio access technology/mode and is defined as follows:

-	for a E-UTRAN cell, is the measured RSRP value for the cell minus E-UTRAN_QRXLEVMIN for the cell’s frequency if THRESH_E-UTRAN_high_Q is not provided; otherwise, if THRESH_E-UTRAN_high_Q is provided, is the measured RSRQ value for the cell minus E-UTRAN_QQUALMIN for the cell’s frequency.

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060 section 5.5.1.1a.2 and 5.5.2.3.

6.2.3.26.3	Test description

6.2.3.26.3.1	Pre-test conditions

System Simulator:

-	Cell 24 is serving GERAN Cell

-	Cell 1 is suitable E-UTRAN Cell

UE:

None.

Preamble:

- The UE is GPRS attached and the PDP context 2 activated according to section 41.2.8.1.1 and 40.4.3.15 of TS 51.010-1 and TS 36.508 [18] clause 4.5A.3B.

6.2.3.26.3.2	Test procedure sequence

Tables 6.2.3.26.3.2-1& 6.2.3.26.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.26.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | OFF |  |
|  | Qrxlevmin | dBm | -106 |  |
|  | Srxlev* | dB | N/A |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 |  |
|  | Srxlev* | dB | 30 | SrxlevCell 1 >THRESH_E-UTRAN_high |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.26.3.2-2: Time instances of cell power level and parameter changes for GERAN cells

|  | Parameter | Unit | Cell 24 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RF Signal Level | dBm | -80 | Camping on Cell 24 is guaranteed |
|  | RXLEV_ACCESS_MIN | dBm | -101 |  |
|  | C1* | dB | 21 |  |
| T1 | RF Signal Level | dBm |  | Same as before |
|  | RXLEV_ACCESS_MIN | dBm |  | Sa me as before |
|  | C1* | dB |  | Same as before |
| Note: C1 is calculated in the UE |  |  |  |  |

Table 6.2.3.26.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Downlink TBF is established according to TS 51.010 clause 40.4.3.14 | - | - | - | - |
| - | EXCEPTION: Steps 2 and 3 are repeated until measurement results for cell 1 are included in PACKET MEASUREMENT REPORT | - | - | - | - |
| 2 | SS sends downlink data | <-- |  |  |  |
| 3 | UE sends PACKET MEASUREMENT REPORT | --> | PACKET MEASUREMENT REPORT |  |  |
| 4 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.26.3.2-1 | - | - | - | - |
| - | EXCEPTION: Steps 5 and 6 are repeated until the parallel behaviour in table 6.2.3.26.3.2-3 takes place | - | - | - | - |
| 5 | SS sends downlink data |  |  |  |  |
| 6 | UE sends PACKET MEASUREMENT REPORT | --> | PACKET MEASUREMENT REPORT |  |  |

Table 6.2.3.26.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |

6.2.3.26.3.3	Specific message contents

Table 6.2.3.26.3.3-1: Message SI2quater for Cell 24

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |
| NC_NON_DRX_PERIOD | ‘000’B | No non DRX mode |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘011’B | 3.84 sec |  |
| } |  |  |  |
| Serving Cell Priority Parameters ::= { |  |  |  |
| GERAN_PRIORITY | ‘001’B | This field specifies GERAN cell priority |  |
| } |  |  |  |
| GPRS E-UTRAN Measurement Parameters Description ::= { |  |  |  |
| Qsearch_P_E-UTRAN_Initial | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm,  (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm,  (never). Default value =  (never). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value =4 dB |  |
| THRESH_E-UTRAN_low | ‘00010’B | Actual value =4 dB |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.26.3.3-2: SYSTEM INFORMATION TYPE 13 for Cell 24

| Derivation Path: 51.010-1 section 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets |  |  |  |
| -NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |

#### 6.2.3.27	Inter-RAT Cell selection from GPRS Packet_transfer to E-UTRA (NC2 Mode)

6.2.3.27.1	Test Purpose (TP)

(1) with { UE in GPRS Registered state with active packet data transfer in NC2 mode }

ensure that {

when { UE detects network re-selection criteria are met on the E-UTRA cell}

then { UE performs cell change to E-UTRA cell and the data transfer is continued and completed in E-UTRA cell }

}

6.2.3.27.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.4, 10.1.4, 10.1.4.1 and 10.1.4.2, TS 44.018, clause 3.4.1.2.1.1a:

[TS 45.008, clause 6.4]

The path loss criterion parameter C1 used for cell selection and reselection is defined by:

C1 = (A - Max(B,0))

where

A									=	RLA_C - RXLEV_ACCESS_MIN
B									=	MS_TXPWR_MAX_CCH - P

except for the class 3 DCS 1 800 MS where:

B									=	MS_TXPWR_MAX_CCH + POWER OFFSET - P
RXLEV_ACCESS_MIN		=	Minimum received signal level at the MS required for access to the system.
MS_TXPWR_MAX_CCH		=	Maximum TX power level an MS may use when accessing the system 											until otherwise commanded.

POWER OFFSET				=	The power offset to be used in conjunction with the MS TXPWR 
										MAX CCH parameter by the class 3 DCS 1 800 MS.
P									=	Maximum RF output power of the MS.

All values are expressed in dBm.

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control 	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

A list given by Packet Cell Change Order applies in the new cell. This list may also include cells with other radio access technologies.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060.

If the MS operates in NC2 mode, the following rules shall be applied:

-	The network controls the measurements of UTRAN cells by the parameter Qsearch_P. The network controls the measurements of E-UTRAN cells by the parameter Qsearch_P_E-UTRAN.

-	The network may control UTRAN measurements per individual UTRAN frequency by the parameters Measurement_Control_UTRAN and E-UTRAN measurements per individual E-UTRAN frequency by the parameters Measurement_Control_E-UTRAN.

[TS 45.008, clause 10.1.4.1]

When ordered to send measurement reports, the MS shall continuously monitor all carriers in BA(GPRS) or as indicated by the parameter NC_FREQUENCY_LIST and the BCCH carrier of the serving cell. The measurement requirements are defined in subclause 10.1.1 for the actual packet mode

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The fast acquisition of system information procedure, as defined in subclause 3.4.1.2.1.11, shall be used to acquire E-UTRAN measurement parameters and neighbour cell information from SI2quater and MEASUREMENT INFORMATION.

6.2.3.27.3	Test description

6.2.3.27.3.1	Pre-test conditions

System Simulator:

-	2 Cells, one GSM and one E-UTRA:

-Cell 24 GSM serving cell

-Cell 1 non-suitable “Off” cell

UE:

- None

Preamble:

- The UE is GPRS attached to the home PLMN and the PDP context 2 activated according to TS 51.010-1 clause 40.4.3.15 and TS 36.508 [18] clause 4.5A.3B.

6.2.3.27.3.2	Test procedure sequence

Table 6.2.3.27.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.27.3.2-1: Time instances of cell power level and parameter changes (E-UTRA cell)

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | SrxlevCell1 > 0 |
|  | Qrxlevmin | dBm | -106 | - | - |
|  | Qrxlevminoffset | dB | 0 | - | - |
|  | Pcompensation | dB | 0 | - | - |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.3.27.3.2-2: Time instances of cell power level and parameter changes for the GERAN cell

|  | Parameter | Unit | Cell 24 | Remark |
| --- | --- | --- | --- | --- |
| T1 | RF Signal Level | dBm | -90 |  |
|  | C1* | dB | <0 |  |
| Note: C1 is calculated in the UE |  |  |  |  |

Table 6.2.3.27.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Uplink dynamic allocation two phase access according to TS 51.010 clause 40.4.3.9 using n=3000 octets of data. | --> | - | - | - |
| 2 | The UE transmits PACKET MEASUREMENT REPORT | --> | PACKET MEASUREMENT REPORT | - | - |
| 3 | SS adjusts cell levels according to row T1 of table 6.2.3.27.3.2-1 and 6.2.3.27.3.2-2. | - | - | - | - |
| - | EXCEPTION: Step 2 is repeated and MS continues to transfer data. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does UE send RRC CONNECTION REQUEST on cell 1? | --> | RRCConnectionRequest | 1 | P |
| 6 | SS sends RRCCconnectionSetup to the UE | <-- | RRCCconnectionSetup | - | - |
| 7 | Check: Does the UE send RRCCconnectionSetupComplete message on cell 1? | --> | RRCCconnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 8-12 | Steps 4 to 8 from generic prodedure 36.508 Table 6.4.2.7A-1 are executed | - | - | - | - |
| Note: Uplink data transfer can be completed in any step after step 2 and before step 4 in Cell 24, or any step after step 20 in Cell 1. During step 2 to 4 new resources for uplink transfer may be request from UE according to 51.010-1 section 40.4.3.9. |  |  |  |  |  |

6.2.3.27.3.3	Specific message contents

Table 6.2.3.27.3.3-1: SI2quater for Cell 24

| Derivation Path: 44.018 Table 10.5.2.33b.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘10’B | NC2 |  |
| NC_NON_DRX_PERIOD | ‘000’B | No non DRX mode |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘010’B | 1.92 sec |  |
| } |  |  |  |
| Serving Cell Priority Parameters Description::= SEQUENCE { |  |  |  |
| GERAN_PRIORITY | 7 |  |  |
| THRESH_Priority_Search | 0 |  |  |
| THRESH_GSM_low | 0(-98 dBm) |  |  |
| H_PRIO | 0 | Hysteresis used in the priority reselection algorithm |  |
| T_Reselection | 3(=20 seconds) | Time hysteresis in the reselection algorithm |  |
| } |  |  |  |
| GPRS E-UTRAN Measurement Parameters Description ::= { |  |  |  |
| Qsearch_P_E-UTRAN | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm,  (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm,  (never). Default value =  (never). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.27.3.3-2: SYSTEM INFORMATION TYPE 13 for Cell 24

| Derivation Path: 51.010-1 section 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets |  |  |  |
| -NETWORK_CONTROL_ORDER | ‘10’B | NC2 |  |

Table 6.2.3.27.3.3-3: RRCConnectionRequest (step 5, Table 6.2.3.27.3.2-2)

| Derivation Path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ue-Identity | Any allowed value |  |  |

#### 6.2.3.28	Inter-RAT Cell reselection from GPRS Packet_transfer to E-UTRA (Network Assisted Cell Change)

6.2.3.28.1	Test Purpose (TP)

(1) with { UE in GPRS Registered state with active packet data transfer in NC2 mode }

ensure that {

when { UE detects network assisted cell change and the target E-UTRA cell}

then { UE reselects for the target E-UTRA cell}

}

6.2.3.28.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 10.1.4, 10.1.4.1 and 10.1.4.2, TS 44.018, clause 3.4.1.2.1.1a:

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control 	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

A list given by Packet Cell Change Order applies in the new cell. This list may also include cells with other radio access technologies.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060.

If the MS operates in NC2 mode, the following rules shall be applied:

-	The network controls the measurements of UTRAN cells by the parameter Qsearch_P. The network controls the measurements of E-UTRAN cells by the parameter Qsearch_P_E-UTRAN.

-	The network may control UTRAN measurements per individual UTRAN frequency by the parameters Measurement_Control_UTRAN and E-UTRAN measurements per individual E-UTRAN frequency by the parameters Measurement_Control_E-UTRAN.

[TS 45.008, clause 10.1.4.1]

When ordered to send measurement reports, the MS shall continuously monitor all carriers in BA(GPRS) or as indicated by the parameter NC_FREQUENCY_LIST and the BCCH carrier of the serving cell. The measurement requirements are defined in subclause 10.1.1 for the actual packet mode

[TS 45.008, clause 10.1.4.2]

A cell re-selection command may be sent from the network to an MS. When the MS receives the command, it shall re-select the cell according to the included cell description and change the network control mode according to the command (see 3GPP TS 44.060). The command may include re-selection of another radio access technology/mode.

[TS 44.018, clause 3.4.1.2.1.1a]

This applies only to a multi-RAT MS supporting E-UTRAN. One or more instances of the Measurement Information message or SI2quater message may provide E-UTRAN Neighbour Cell Description information in one or more instances of the Repeated E-UTRAN Neighbour Cells IE. This is used to build the E-UTRAN Neighbour Cell list. The fast acquisition of system information procedure, as defined in subclause 3.4.1.2.1.11, shall be used to acquire E-UTRAN measurement parameters and neighbour cell information from SI2quater and MEASUREMENT INFORMATION.

6.2.3.28.3	Test description

6.2.3.28.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 24.

UE:

- The HPLMN is PLMN1.

Preamble:

- The UE is GPRS attached to the home PLMN and the PDP context 2 activated according to [23] and TS 36.508 [18] clause 4.5A.3B.

Tables 6.2.3.28.3.2-1 & 6.2.3.28.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.28.3.2-1: Time instances of cell power level and parameter changes for the E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | OFF |  |
|  | Qrxlevmin | dBm | -106 |  |
|  | Qrxlevminoffset | dB | 0 |  |
|  | Pcompensation | dB | 0 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | SrxlevCell1 > 0 |
|  | Qrxlevmin | dBm | -106 |  |
|  | Qrxlevminoffset | dB | 0 |  |
|  | Pcompensation | dB | 0 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.28.3.2-2: Time instances of cell power level and parameter changes for the GERAN cell

|  | Parameter | Unit | Cell 24 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RF Signal Level | dBm | -80 | Camping on Cell 24 is guaranteed |
|  | RXLEV_ACCESS_MIN | dBm | -101 |  |
|  | C1* | dB | 21 |  |
| T1 | RF Signal Level | dBm | -80 |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 |  |
|  | C1* | dB | 21 |  |
| Note: C1 is calculated in the UE |  |  |  |  |

Table 6.2.3.28.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is brought into downlink packet transfer mode. | - | - | - | - |
| 2 | SS sends PACKET MEASUREMENT ORDER | <-- | PACKET MEASUREMENT ORDER | - | - |
| 3 | SS sends downlink data | <-- | - | - | - |
| 4 | The UE transmits PACKET MEASUREMENT REPORT | --> | PACKET MEASUREMENT REPORT |  |  |
| 5 | SS adjusts cell levels according to row T1 of tables 6.2.3.28.3.2-1 and 6.2.3.28.3.2-2. | - | - | - | - |
| 6 | EXCEPTION: Step 3 to 4 are repeated until measurement results for cell 1 are included in the PACKET MEASUREMENT REPORT message | - | - | - | - |
| 7 | SS sends PACKET CELL CHANGE ORDER for cell 1 as the target cell | <-- | PACKET CELL CHANGE ORDER | - | - |
| 8 | Check: Does UE send RRC CONNECTION REQUEST on cell 1? | --> | RRCConnectionRequest | 1 | P |
| 9 | SS sends RRCCconnectionSetup to the UE | <-- | RRCCconnectionSetup | - | - |
| 10 | Check: Does the UE send RRCCconnectionSetupComplete message on cell 1? | --> | RRCCconnectionSetupComplete | 1 | P |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 1 to 8. |  |  |  |  |  |

6.2.3.28.3.3	Specific message contents

Table 6.2.3.28.3.3-0: Conditions for specific message contents
in Tables 6.2.3.28.3.3-1 and 6.2.3.28.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.28.3.3-1: Message PACKET MEASUREMENT ORDER (step 2, Table 6.2.3.28.3.2-3)

| Derivation Path: 44.060 Table 11.2.9b.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MESSAGE TYPE | ‘000011’ B |  |  |
| PAGE MODE | ‘00’ B |  |  |
| PMO_INDEX | ‘000’B |  |  |
| PMO_COUNT | ‘000’B |  |  |
| Global TFI | TFI of the uplink TBF |  |  |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘10’B | NC2 |  |
| NC_NON_DRX_PERIOD | ‘000’B | No non DRX mode |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘010’B | 1.92 sec |  |
| } |  |  |  |
| NC_FREQUENCY_LIST | 0 | Not present |  |
| 3G_BA_IND | 0 | Present |  |
| PMO_IND | 0 | Present |  |
| E-UTRAN Parameters Description ::= { |  |  |  |
| E-UTRAN_CCN_ACTIVE | 1 | CCN is enabled in the cell |  |
| GPRS E-UTRAN Measurement Parameters struct ::= { |  |  |  |
| Qsearch_P_E-UTRAN | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm, (never). Default value = (never). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { | Not Present |  | Band > 64 |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| E-UTRAN NC with extended EARFCNs ::= SEQUENCE { |  |  | Band > 64 |
| Repeated E-UTRAN NC with extended EARFCNs ::= SEQUENCE [ |  |  |  |
| EARFCN extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.28.3.3-2: Message PACKET CELL CHANGE ORDER (step 7, Table 6.2.3.28.3.2-3)

| Derivation Path: 51.010-1 clause 42.4.3.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Global TFI | TFI of the uplink TBF |  |  |
| IMMEDIATE_REL | 1 |  |  |
| E-UTRAN Target cell IE |  |  |  |
| EARFCN | 0 |  | Band > 64 |
| EARFCN | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |
| E-UTRAN Target cell with extended EARFCN IE |  |  | Band > 64 |
| EARFCN_extended | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |

#### 6.2.3.29	Inter-RAT Cell reselection from GPRS packet_transfer to E-UTRA in CCN mode (PACKET MEASUREMENT ORDER)

6.2.3.29.1	Test Purpose (TP)

(1)

with { UE in GPRS packet_transfer state with active data transfer in NC1 mode }

ensure that {

when { UE with CCN enable detects E-UTRA cell, enters CCN mode, receives PACKET MEASUREMENT ORDER to enter NC2 and subsequently receives PACKET CELL CHANGE ORDER }

then { UE does not perform autonomous reselection, but performs cell change to E-UTRA cell only on receipt of PACKET CELL CHANGE ORDER }

}

6.2.3.29.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 6.6.6, 10.1.4, 10.1.4.1, 10.1.4.2, TS 44.060 clause 5.5.1.1a.2, and TS 36.331 clause 5.4.6.2:

[TS 45.008, clause 6.6.6]

The algorithm in this subclause shall be used for inter-RAT cell reselection if priorities are available to the MS and thresholds are provided by the network, and if the mobile station supports priority based inter-RAT cell re-selection and priority information for the serving cell is provided by the network. A mobile station supporting E-UTRAN shall support priority based inter-RAT cell re-selection towards all the supported RATs. A mobile station not supporting E-UTRAN and supporting UTRAN and supporting priority based reselection from UTRAN to GERAN shall support priority based inter-RAT cell re-selection towards UTRAN.

The network shall provide priority information if E-UTRAN frequencies are included in the neighbour cell list; the network may provide priority information if only UTRAN frequencies are included in the neighbour cell list. If priority information is available to the mobile station and the mobile station supports priority based inter-RAT cell re-selection, the algorithm in this subclause shall be used for inter-RAT reselection towards all RATs. The rules regarding which set of priorities is valid at any given time are defined in 3GPP TS 44.018.

NOTE 1:	“Priority information” includes priorities and thresholds which are related to each UTRAN or E-UTRAN frequency (e.g. UTRAN_PRIORITY, E-UTRAN_PRIORITY, THRESH_UTRAN_high, THRESH_E-UTRAN_high) and information related to the serving cell (e.g. GERAN_PRIORITY, THRESH_GSM_low).

NOTE 2:	Throughout the specification, the phrase “neighbour cell list” will include also the E-UTRAN Neighbour Cell list where appropriate.

NOTE 3:	Priorities need to be provided also for frequencies of networks that do not support priority-based cell reselection.

If the 3G Cell Reselection list or the E-UTRAN Neighbour Cell list include frequencies of other radio access technologies, the MS shall, at least every 5 s update the value RLA_C for the serving cell and each of the at least 6 strongest non serving GSM cells.

The MS shall then reselect a suitable (see 3GPP TS 25.304 for UTRAN and 3GPP TS 36.304 for E-UTRAN) cell of another radio access technology if the criteria below are satisfied. S_non-serving_XXX is the measurement quantity of a non-serving inter-RAT cell and XXX indicates the other radio access technology/mode and is defined as follows:

-	for a UTRAN cell, is the measured RSCP value for the cell minus UTRAN_QRXLEVMIN for the cell’s frequency;

-	for a E-UTRAN cell, is the measured RSRP value for the cell minus E-UTRAN_QRXLEVMIN for the cell’s frequency if THRESH_E-UTRAN_high_Q is not provided; otherwise, if THRESH_E-UTRAN_high_Q is provided, is the measured RSRQ value for the cell minus E-UTRAN_QQUALMIN for the cell’s frequency.

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

A list given by Packet Cell Change Order applies in the new cell. This list may also include cells with other radio access technologies.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060.

[TS 45.008, clause 10.1.4.1]

When ordered to send measurement reports, the MS shall continuously monitor all carriers in BA(GPRS) or as indicated by the parameter NC_FREQUENCY_LIST and the BCCH carrier of the serving cell. The measurement requirements are defined in subclause 10.1.1 for the actual packet mode

[TS 45.008, clause 10.1.4.2]

A cell re-selection command may be sent from the network to an MS. When the MS receives the command, it shall re-select the cell according to the included cell description and change the network control mode according to the command (see 3GPP TS 44.060). The command may include re-selection of another radio access technology/mode.

[TS 44.060, clause 5.5.1.1a.2]

A mobile station, which has CCN Enabled, can enter CCN Mode.

The mobile station shall enable CCN when the following criteria are fulfilled:

-	the mobile station is camping on a cell (see 3GPP TS 45.008); and

-	the network indicates CCN ACTIVE/3G CCN ACTIVE/E-UTRAN CCN ACTIVE either in system information to all mobile stations in the cell or in an individual order to a certain mobile station; and

-	the mobile station is neither in dedicated mode nor Dual Transfer Mode; and

-	the mobile station is in NC0 or in NC1 mode; and

-	the mobile station is in Packet Transfer mode.

The CCN procedures and the criteria for entering and leaving CCN mode are specified in sub-clauses 8.8.2 and 8.8.3.

[TS 36.331, clause 5.4.6.2]

The procedure is initiated when a radio access technology other than E-UTRAN, e.g. GSM/GPRS, using procedures specific for that RAT, orders the UE to change to an E-UTRAN cell. In response, upper layers request the establishment of an RRC connection as specified in subclause 5.3.3.

NOTE:	Within the message used to order the UE to change to an E-UTRAN cell, the source RAT should specify the identity of the target E-UTRAN cell as specified in the specifications for that RAT.

The UE shall:

1>	upon receiving an RRCConnectionSetup message:

2>	consider the inter-RAT cell change order procedure to have completed successfully;

6.2.3.29.3	Test description

6.2.3.29.3.1	Pre-test conditions

System Simulator:

- GERAN Cell 24 serving cell

- E-UTRAN Cell 1 off

UE:

None

Preamble:

-	The UE is GPRS attached and the PDP context 2 activated according to section 40.4.3.15 of TS 51010-1 and TS 36.508 [18] clause 4.5A.3B.

6.2.3.29.3.2	Test procedure sequence

Table 6.2.3.29.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.29.3.2-1: Time instances of cell power level and parameter changes (GERAN and E-UTRA cells)

|  | Parameter | Unit | Cell 1 | Cell24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | No change | The power level is such that SrxlevCell 1 > 0 |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.2.3.29.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Uplink dynamic allocation two phase access according to TS 51.010 clause 40.4.3.9 using n=30000 octets of data | - | - | - | - |
| - | UE continues to transfer data. See Note. | - | - | - | - |
| 2 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.29.3.2-1 | - | - | - | - |
| 3 | The UE transmits PACKET CELL CHANGE NOTIFICATION to E_UTRA cell on cell 24 | --> | PACKET CELL CHANGE NOTIFICATION |  |  |
| 4 | SS sends PACKET MEASUREMENT ORDER activating NC2 | <-- | PACKET MEASUREMENT ORDER |  |  |
| 5 | The UE moves out of CCN mode and does not perform cell reselection Check: Does UE send RRCConnectionRequest on cell 1? | --> | RRCConnectionRequest | 1 | F |
| 6 | The UE sends PACKET MEASUREMENT REPORT including the Cell 1 information in the message. | --> | PACKET MEASUREMENT REPORT | - | - |
| 7 | SS sends PACKET CELL CHANGE ORDER for cell 1 as the target cell | <-- | PACKET CELL CHANGE ORDER |  |  |
| 8 | Check: Does UE send RRCConnectionRequest on cell 1? | --> | RRCConnectionRequest | 1 | P |
| 9 - 14 | Steps 2 to 7 from generic prodedure 36.508 Table 6.4.2.7A-1 are executed | - | - | - | - |
| Note: Uplink data transfer can be completed in any step after step 4 and before step 7 in Cell 24, or any step after step 7 in Cell 1. During step 2 to 7 new resources for uplink transfer may be request from UE according to 51.010-1 section 40.4.3.9. |  |  |  |  |  |

Table 6.2.3.29.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Before step 3 Main behaviour: The UE sends PACKET MEASUREMENT REPORT | - | - | - | - |

6.2.3.29.3.3	Specific message contents

Table 6.2.3.29.3.3-1: Message SI2quater for Cell 24

| Derivation Path: 36.508 table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |
| NC_NON_DRX_PERIOD | ‘000’B | No non DRX mode |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘111’B | 61.44 sec |  |
| } |  |  |  |
| Serving Cell Priority Parameters ::= |  |  |  |
| GERAN_PRIORITY | ‘001’B | This field specifies GERAN cell priority |  |
| E-UTRAN Parameters Description ::= |  |  |  |
| E-UTRAN_CCN_ACTIVE | 1 | CCN is enabled in the cell |  |
| GPRS E-UTRAN Measurement Parameters Description ::= { |  |  |  |
| Qsearch_P_E-UTRAN_Initial | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm,  (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm,  (never). Default value =  (never). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= |  |  |  |
| { |  |  |  |
| Measurement Bandwidth | Same as cell 1 |  |  |
| } |  |  |  |

Table 6.2.3.29.3.3-2: Message PACKET MEASUREMENT ORDER (step 4, Table 6.2.3.29.3.2-2)

| Derivation Path: 51.010-1 clause 42.4.3.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Global TFI | TFI of the uplink TBF |  |  |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘10’B | NC2 |  |
| NC_NON_DRX_PERIOD | ‘000’B | No non DRX mode |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘010’B | 1.92 sec |  |
| } |  |  |  |
| NC_FREQUENCY_LIST | 0 | Not present |  |

Table 6.2.3.29.3.3-3: Message PACKET CELL CHANGE ORDER (step 7, Table 6.2.3.29.3.2-2)

| Derivation Path: 51.010-1 clause 42.4.3.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Global TFI | TFI of the uplink TBF |  |  |
| IMMEDIATE_REL | 1 |  |  |
| E-UTRAN Target cell IE | 0 |  | Band > 64 |
| E-UTRAN Target cell IE |  |  |  |
| EARFCN | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |
| E-UTRAN Target cell with extended EARFCN IE |  |  | Band > 64 |
| EARFCN_extended | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.29.3.3-4: SYSTEM INFORMATION TYPE 13 for Cell 24

| Derivation Path: 51.010-1 section 40.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 13 Rest Octets |  |  |  |
| -NETWORK_CONTROL_ORDER | ‘01’B | NC1 |  |

#### 6.2.3.30	Inter-RAT Cell reselection failure from GPRS Packet transfer to E-UTRA (Network Assisted Cell Change)

6.2.3.30.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC2 mode }

ensure that {

when { UE detects network assisted cell change and cell re-selection failure for the target E-UTRA cell}

then { UE reselects back the GERAN cell and downlink data transfer is continued and completed in GERAN cell }

}

6.2.3.30.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 45.008, clause 10.1.4, and 10.1.4.2:

[TS 45.008, clause 10.1.4]

The network may request measurement reports from the MS and control its cell re-selection. This is indicated by the parameter NETWORK_CONTROL_ORDER. The meaning of the different parameter values is specified as follows:

NC0	Normal MS control 	
The MS shall perform autonomous cell re-selection.

NC1	MS control with measurement reports 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall perform autonomous cell re-selection.

NC2	Network control 	
The MS shall send measurement reports to the network as defined in subclause 10.1.4.1.
The MS shall only perform autonomous cell re-selection when the reselection is triggered by a downlink signalling failure as defined in subclause 6.5 or a random access failure as defined in 3GPP TS 44.018 and 3GPP TS 44.060 or if the cell is barred or the C1 criterion falls below zero. The MS shall only determine whether the cell is barred once camped on the cell.

RESET	The MS shall return to the broadcast parameters. Only sent on PCCCH or PACCH.

A list given by Packet Cell Change Order applies in the new cell. This list may also include cells with other radio access technologies.

All signalling for support of network controlled cell re-selection and measurement reports are defined in 3GPP TS 44.060.

If the MS operates in NC2 mode, the following rules shall be applied:

-	The network controls the measurements of UTRAN cells by the parameter Qsearch_P. The network controls the measurements of E-UTRAN cells by the parameter Qsearch_P_E-UTRAN.

-	The network may control UTRAN measurements per individual UTRAN frequency by the parameters Measurement_Control_UTRAN and E-UTRAN measurements per individual E-UTRAN frequency by the parameters Measurement_Control_E-UTRAN.

[TS 45.008, clause 10.1.4.2]

A cell re-selection command may be sent from the network to an MS. When the MS receives the command, it shall re-select the cell according to the included cell description and change the network control mode according to the command (see 3GPP TS 44.060). The command may include re-selection of another radio access technology/mode.

If a UTRAN capable MS receives a cell re-selection command towards a not known UTRAN cell (see 3GPP TS 25.133 and 3GPP TS 25.123), or if a E-UTRAN capable MS receives a cell re-selection command towards a not known E-UTRAN cell (see 3GPP TS 36.133), then the MS shall search for synchronisation information up to 800 ms. In case of failure, the MS shall return to the old cell and indicate a packet cell change failure (see 3GPP TS 44.060).

6.2.3.30.3	Test description

6.2.3.30.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 24.

UE:

The HPLMN is PLMN1.

Preamble:

- The UE is GPRS attached to the home PLMN and the PDP context 2 activated according to [23] and 36.508 [18] clause 4.5A.3B.

6.2.3.30.3.2	Test procedure sequence

Tables 6.2.3.30.3.2-1 & 6.2.3.30.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.30.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit |  | Cell 1 |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | OFF |  |  |  |
|  | Qrxlevmin | dBm |  | -106 |  |  |  |
|  | Qrxlevminoffset | dB |  | 0 |  |  |  |
|  | Pcompensation | dB |  | 0 |  |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz |  | -70 |  | The power level is such that SrxlevCell 1 > 0 |  |
| T2 | Cell-specific RS EPRE |  | dBm/15kHz |  | -115 |  | The power level is such that reselection back to Cell 1 should not occur |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |  |  |

Table 6.2.3.30.3.2-2: Time instances of cell power level and parameter changes for GERAN cells

|  | Parameter | Unit | Cell 24 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RF Signal Level | dBm | -80 | Camping on Cell 24 is guaranteed |
|  | RXLEV_ACCESS_MIN | dBm | -101 |  |
|  | C1* | dB | 21 |  |
| T1 | RF Signal Level | dBm | -80 |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 |  |
|  | C1* | dB | 21 |  |
| Note: C1 is calculated in the UE |  |  |  |  |

Table 6.2.3.30.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Downlink TBF is established according to TS 51.010 clause 40.4.3.14 |  |  | - | - |
| 2 | SS sends PACKET MEASUREMENT ORDER | <-- | PACKET MEASUREMENT ORDER | - | - |
| 3 | SS sends downlink data | <-- | - | - | - |
| 4 | The UE transmits PACKET MEASUREMENT REPORT | --> | PACKET MEASUREMENT REPORT |  |  |
| 5 | SS adjusts cell levels for Cell 1 according to row T1 of table 6.2.3.30.3.2-1 | - | - | - | - |
| 6 | EXCEPTION: Step 3 to 4 are repeated until measurement results for cell 1 are included in the PACKET MEASUREMENT REPORT message | - | - | - | - |
| 7 | SS sends PACKET CELL CHANGE ORDER for cell 1 as the target cell | <-- | PACKET CELL CHANGE ORDER | - | - |
| 8 | UE sends RRCConnectionRequest to cell 1 | --> | RRCConnectionRequest | - | - |
| 9 | SS sends RRCConnectionReject to the UE | <-- | RRCConnectionReject | - | - |
| 10 | All the UL/DL messages sent in steps 11-20 shall be sent on cell 24. | - |  | - | - |
| 11 | Check: Does the UE send CHANNEL REQUEST ? | --> | CHANNEL REQUEST | 1 | P |
| 11A | SS adjusts cell levels for Cell 1 according to row T2 of table 6.2.3.30.3.2-1 | - | - | - | - |
| 12 | SS sends IMMEDIATE ASSIGNMENT | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 13 | Check: Does the UE send PACKET CELL CHANGE FAILURE? | --> | PACKET CELL CHANGE FAILURE | 1 | P |
| - | EXCEPTION: Steps 14a to 14c describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place. This is checked for a period of 1s | - | - | - | - |
| 14a | the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 14b | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 14c | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 15 | Downlink TBF is established according to TS 51.010 clause 40.4.3.14 |  |  |  |  |
| 16 | SS sends downlink data until downlink data transfer is complete | <-- | - | - | - |
| Note 1: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 1 to 8. |  |  |  |  |  |

6.2.3.30.3.3	Specific message contents

Table 6.2.3.30.3.3-0: Conditions for specific message contents
in Tables 6.2.3.30.3.3-1 and 6.2.3.30.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.30.3.3-1: Message SI2Quater for Cell 24 (Preamble)

| Derivation Path: 44.018 Table 10.5.2.33b.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| E-UTRAN Measurement Parameters Description ::= { |  |  |  |
| Qsearch_C_E-UTRAN_Initial | ‘0111’ B | This filed indicates the Qsearch value to be used in dedicated / dual transfer mode before Qsearch_C_E-UTRAN is received, Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells ::= SEQUENCE { |  |  |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Same as cell 1 |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) |  |  |
| } |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.30.3.3-2: Message ATTACH REQUEST (Preamble)

| Derivation Path: Table 9.4.1/3GPP TS 24.008 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS Radio Access capability |  |  |  |
| E-UTRA FDD support | ‘0’B or ‘1’B |  | C1 |
| E-UTRA TDD support | ‘0’B or ‘1’B |  | C1 |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘10’B or ‘11’B | UTRAN Neighbour Cell measurements and MS autonomous cell reselection to E-UTRAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN |  |

| C1 | At least one of these fields shall be set to ‘1’B |
| --- | --- |

Table 6.2.3.30.3.3-3: Message PACKET MEASUREMENT ORDER (step 2, Table 6.2.3.30.3.2-3)

| Derivation Path: 51.010-1 clause 42.4.3.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Global TFI | TFI of the uplink TBF |  |  |
| NC Measurement parameters ::={ |  |  |  |
| NETWORK_CONTROL_ORDER | ‘10’B | NC2 |  |
| NC_REPORTING_PERIOD_I | ‘111’B | 61.44 sec |  |
| NC_REPORTING_PERIOD_T | ‘010’B | 1.92 sec |  |
| } |  |  |  |
| NC_FREQUENCY_LIST | 0 | Not present |  |
| 3G_BA_IND | 0 | Present |  |
| PMO_IND | 0 | Present |  |
| E-UTRAN Parameters Description ::= { |  |  |  |
| E-UTRAN_CCN_ACTIVE | 1 | CCN is enabled in the cell |  |
| GPRS E-UTRAN Measurement Parameters struct ::= { |  |  |  |
| Qsearch_P_E-UTRAN | ‘0111’ B | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) or above threshold (8-15). |  |
| E-UTRAN_REP_QUANT | ‘0’ B | This filed indicates the reporting quantity for E-UTRAN cells |  |
| E-UTRAN_MULTIRAT_REPORTING | ‘01’ B | This field indicates the number of cells that shall be included in the list of strongest cells or in the measurement report. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.30.3.3-4: Message PACKET CELL CHANGE ORDER (step 7, Table 6.2.3.30.3.2-3)

| Derivation Path: 51.010-1 clause 42.4.3.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Global TFI | TFI of the uplink TBF |  |  |
| IMMEDIATE_REL | 1 |  |  |
| E-UTRAN Target cell IE | 0 |  | Band > 64 |
| E-UTRAN Target cell IE |  |  |  |
| EARFCN | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |
| E-UTRAN Target cell with extended EARFCN IE |  |  | Band > 64 |
| EARFCN_extended | specified for cell 1 |  |  |
| Physical Layer Cell Identity | specified for cell 1 |  |  |

Table 6.2.3.30.3.3-5: RRCConnectionRequest (step 8, Table 6.2.3.30.3.2-3)

| Derivation Path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ue-Identity | Any allowed value |  |  |

Table 6.2.3.30.3.3-6: Message PACKET CELL CHANGE FAILURE (step 13, Table 6.2.3.30.3.2-3)

| Derivation Path: 51.010-1 clause 42.4.3.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CAUSE | ‘0010’B |  |  |

#### 6.2.3.31	Inter-RAT Cell reselection / From UTRA_Idle (low priority) to E-UTRA RRC_IDLE (high priority) according to RAT priority provided by dedicated signalling

6.2.3.31.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the high priority inter-RAT E-UTRA cell based on the configured RAT priority provided by dedicated signaling }

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.31.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a; TS 25.331, clause 8.3.3.3 and 8.6.7.23; TS 36.331, clause 5.3.3.4.

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

…

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

…

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 25.331, clause 8.3.3.3]

When the UE receives a UTRAN MOBILITY INFORMATION message, it shall:

1>	act on received information elements as specified in subclause 8.6;

1>	if the IE "Dedicated Priority Information" is present:

2>	act upon the received IE as described in subclause 8.6.7.23.

[TS 25.331, clause 8.6.7.23]

If the CHOICE "Action" has the value "Configure dedicated priorities", the UE shall:

1>	clear the variable PRIORITY_INFO_LIST;

1>	stop timer T322, if it is running;

1>	set the value of IE "Priority status" in the variable PRIORITY_INFO_LIST to "dedicated_priority";

1>	for each occurrence of the IE "Priority Level List":

2>	create a new entry in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST, and in that new entry:

3>	set the CHOICE "Radio Access Technology" to the value received in the IE "Priority Level List";

3>	set the IE "priority" to the value received in the IE "Priority Level List";

3>	set the values in IE "Frequency List" or "BCCH ARFCN List" to the values received in the IE "Priority Level List".

1>	set the IE "E-UTRA detection" to the value received in the IE "Priority Level List".

1>	if the IE "T322" is present:

2>	start timer T322 using the value signalled in this IE.

1>	if the UE is not in CELL_DCH state:

2>	take the actions as described in subclause 8.1.1.6.19 using stored System information Block type 19.

[TS 36.331, clause 5.3.3.4]

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

...

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

6.2.3.31.3	Test description

6.2.3.31.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	Cell 1 suitable neighbour E-UTRA cell

-	Cell 5 UTRA serving cell

NOTE: Common Priority information for Cell 1 & Cell 5: UTRA priority = 3; E-UTRA priority = 4.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH_DCH (state 6-10) according to [5].

6.2.3.31.3.2	Test procedure sequence

Tables 6.2.3.31.3.2-1 and 6.2.3.31.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.31.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -97 |  |
|  | Qrxlevmin | dBm | -106 | Default value |
|  | Srxlev* | dB | 9 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -82 |  |
|  | Srxlev* | dB | 24 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA S nonServingCell, Cell1 > Thresh Cell1,high |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.31.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 | The power levels are such that camping on Cell 5 is guaranteed. |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin | dBm | -79 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -75 |  |
|  | P-CCPCH | dBm/1.28 MHz | -77 |  |
|  | Srxlev* | dB | 4 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.31.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS provides dedicated priority information (UTRA priority = 3; E-UTRA priority = 4) and validity timer to the UE. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2 | The UE transmits UTRAN MOBILITY INFOMRATION CONFIRM message on Cell 5. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 3 | The SS transmits an RRC CONNECTION RELEASE message to release UE connection. | <-- | RRC CONNECTION RELEASE | - | - |
| 4 | The UE transmits an RRC CONNECTION RELEASE COMPLETE message on Cell 5. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 5 | The UE transmits an RRC CONNECTION RELEASE COMPLETE message on Cell 5. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 6 | The UE transmits an RRC CONNECTION RELEASE COMPLETE message on Cell 5. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 7 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.31.3.2-1 and table 6.2.3.31.3.2-2. | - | - | - | - |
| 8-14 | Check: Does the test result of steps 1 to 7 of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |
| - | Exception: Steps 15 to 15a5/15b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 15 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 15a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 15a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 15a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 15a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 15a5 | The SS releases the RRC connection | - | - | - | - |
| 15b1 | The 10s timer expires | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

6.2.3.31.3.3	Specific message contents

Table 6.2.3.31.3.3-0: Conditions for specific message contents
in tables 6.2.3.31.3.3-1 and 6.2.3.31.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.31.3.3-1: UTRAN MOBILITY INFORMATION for Cell 5 (step 1, Table 6.2.3.31.3.2-3)

| Derivation Path: 34.108, clause 9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UTRANMobilityInformation ::= CHOICE { |  |  |  |
| later-than-r3 SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| r7 SEQUENCE { |  |  |  |
| v860NonCriticalExtensions SEQUENCE { |  |  |  |
| utranMobilityInformation-v860ext SEQUENCE { |  |  |  |
| dedicatedPriorityInformation SEQUENCE { |  |  |  |
| action CHOICE { |  |  |  |
| configureDedicatedPriorities SEQUENCE { |  |  |  |
| t-322 | 5 | Time in minutes |  |
| priorityLevelList SEQUENCE (SIZE (1..maxPrio)) OF SEQUENCE { | 2 entry |  |  |
| priority[1] | 3 |  |  |
| radioAccessTechnology[1] CHOICE { |  |  |  |
| utraFDD SEQUENCE (SIZE (1..maxNumFDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| utraTDD SEQUENCE (SIZE (1..maxNumTDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| priority[2] | 4 |  |  |
| radioAccessTechnology[2] CHOICE { |  |  |  |
| eutra SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { | 1 entry |  |  |
| earfcn | Same downlink EARFCN as used for Cell 1 |  |  |
| earfcn | 65535 |  | Band>64 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| e-UTRA detection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vb50NonCriticalExtentions SEQUENCE { |  |  | Band > 64 |
| utranMobilityInformation-vb50ext SEQUENCE { |  |  |  |
| dedicatedPriorityInformation SEQUENCE { |  |  |  |
| action CHOICE { |  |  |  |
| configureDedicatedPriorities SEQUENCE { |  |  |  |
| t-322 | 5 |  |  |
| priorityLevelList SEQUENCE (SIZE (1..maxPrio)) OF SEQUENCE { | 2 entries |  |  |
| priority[1] | 5 |  |  |
| radioAccessTechnology[1] CHOICE { |  |  |  |
| utraFDD SEQUENCE (SIZE (1..maxNumFDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| utraTDD SEQUENCE (SIZE (1..maxNumTDDFreqs)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| uarfcn | Same downlink UARFCN as used for Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| priority[2] | 4 |  |  |
| radioAccessTechnology[2] CHOICE { |  |  |  |
| eutra SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF EARFCNRange { | 1 entry |  |  |
| earfcn | 65535 |  |  |
| earfcnEXT | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.31.3.3-2: RRC CONNECTION RELEASE for Cell 5 (step 3, Table 6.2.3.31.3.2-3)

| Derivation Path: 34.108, clause 9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= CHOICE { |  |  |  |
| later-than-r3 SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| r4 SEQUENCE { |  |  |  |
| rrcConnectionRelease-r4 SEQUENCE { |  |  |  |
| n-308 | 2 | 2 (for CELL_DCH state). Not Present (for UE in other connected mode states). |  |
| releaseCause | normalEvent |  |  |
| rplmn-information | Not Present |  |  |
| } |  |  |  |
| v4d0NonCriticalExtensions SEQUENCE {} | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.31.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.31.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| s-PrioritySearch1 | 8 (16 dB) |  |  |
| threshServingLow | 4 (8 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { | 1 entry |  |  |
| earfcn [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [1] | Not present |  |  |
| qRxLevMinEUTRA [1] | -53 (-106 dBm) |  |  |
| threshXhigh [1] | 9 (18 dB) |  |  |
| threshXlow [1] | 15 (30 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [1] | Not present |  |  |
| qRxLevMinEUTRA [1] | -53 (-106 dBm) |  |  |
| threshXhigh [1] | 9 (18 dB) |  |  |
| threshXlow [1] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.31.3.3-4: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.31.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 (10dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.31.3.3-5: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.31.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| threshX-High[1] | 5 (10dB) |  |  |
| threshX-Low[1] | 5 (10dB) |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| threshX-High[1] | 5 (10dB) |  |  |
| threshX-Low[1] | 5 (10dB) |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 6.2.3.32	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle, Snonintrasearch

6.2.3.32.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { Snonintrasearch is non-zero in system information }

then { UE perform measurement and reselects the cell which belong to the high priority UTRA cell upon Srxlev > Snonintrasearch }

}

6.2.3.32.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.304, clauses 5.2.4.2, 5.2.4.5 and 5.2.4.6;  TS 36.331, clause 6.3.1.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

.....

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell.

...

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT,

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

[TS 36.133, clause 4.2.2.5.1]

…

The UE shall evaluate whether newly detectable UTRA FDD cells have met the reselection criteria in TS 36.304 within time (NUTRA_carrier) * TdetectUTRA_FDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when TreselectionRAT = 0 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io.

…

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met reselection criterion defined in 3GPP TS 36.304 [1] within (NUTRA_carrier) * TevaluateUTRA_FDD when Treselection = 0 as speficied in table 4.2.2.5.1-1 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io.

[TS 36.133, clause 4.2.2.5.2]

…

The UE shall evaluate whether newly detectable UTRA TDD cells have met the reselection criteria in TS 36.304 within time (NUTRA_carrier_TDD) * TdetectUTRA_TDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ  when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB.

…

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met reselection criterion defined in [1] within NUTRA_carrier_TDD *TevaluateUTRA_TDD when Treselection = 0 as specified in table 4.2.2.5.2-1 provided that the reselection criteria is met by a margin of at least 6dB.

6.2.3.32.3	Test description

6.2.3.32.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1(serving cell) according to [18].

6.2.3.32.3.2	Test procedure sequence

Table 6.2.3.32.3.2-1 and 6.2.3.32.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.2.3.32.3.2-3.

Table 6.2.3.32.3.2-1: Time instances of cell power level and parameter change for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Srxlev of Cell 1 is greater than Snonintrasearch |
|  | Qrxlevmin | dBm | -106 |  |
|  | Snonintrasearch | dB | [14] |  |
|  | Srxlev* | dB | 21 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.32.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 | SrxlevnonServingCell, Cell 5 > Thresh Cell 5,high |
| Note: Srxlev is calculated in the UE |  |  |  |  |

Table 6.2.3.32.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.2.3.32.3.2-1 and 6.2.3.32.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of steps 1 to 8 of the generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? (Note 1) NOTE: The UE performs a RAU procedure. | - | - | 1 | - |
| Note 1: UE shall search higher priority layer (i.e. UTRA carrier frequency of Cell 5) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in [4.2.2, 34]. |  |  |  |  |  |

6.2.3.32.3.3	Specific message contents

Table 6.2.3.32.3.3-1: SystemInformationBlockType3 for Cell 1 (preamble and all steps, table 6.2.3.32.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 7 (14 dB) |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 11 (22 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.32.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.32.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| threshX-High[n] | 5 | 10 dB |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| threshX-High[n] | 5 | 10 dB |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.32.3.3-3: System Information Block Type 19 for Cell 5 (preamble and all steps, Table 6.2.3.32.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.33	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle / Squal based cell reselection parameters are broadcasted in E-UTRAN / UE does not support Squal based cell reselection in UTRAN

6.2.3.33.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state not supporting Squal based inter RAT cell reselection from UTRAN to E-UTRAN and having received SystemInformationBlockType3 containing Squal based cell reselection parameters from serving E-UTRAN cell }

ensure that {
  when { UE detects the cell re-selection criteria based on Srxlev are met for the neighbour UTRA cell }

then { UE reselects the UTRA cell }

}

6.2.3.33.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5.

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

...

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

...

Cell reselection to another RAT, for which Squal based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria if the UE supports Squal (RSRQ) based cell reselection to E-UTRAN from all the other RATs provided by system information which UE supports. Otherwise, cell reselection to another RAT shall be performed based on Srxlev criteria.

6.2.3.33.3	Test description

6.2.3.33.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.2.3.33.3.2	Test procedure sequence

Table 6.2.3.33.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.2.3.33.3.2-2.

Table 6.2.3.33.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -97 | - | The power level value is assigned to satisfy SrxlevCell 1 < ThreshServing, LowP. (20 dB) SqualCell 1 > ThreshServing, LowQ(4 dB) |
|  | Noc | dBm/15kHz | Off | - |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | -65 | The power level value is assigned to satisfy SrxlevCell 5 > Threshx, LowP. (2 dB) |
|  | P-CCPCH | dBm/1.28 MHz | - | -67 |  |
|  | Ioc | dBm/3.84kHz | - | Off |  |
|  | CPICH_Ec/No | dB | - | -2.89 |  |

Table 6.2.3.33.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 2 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 6.2.3.33.3.2-1. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 1 | - |

6.2.3.33.3.3	Specific message contents

Table 6.2.3.33.3.3-0: Conditions for specific message contents
in Table 6.2.3.33.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.33.3.3-1: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.33.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present | Default of infinity applied. |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present | Default 0 dB applied |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.33.3.3-2: Void

Table 6.2.3.33.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.33.3.2-2)

| Derivation Path: 36.508, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn[n] | Same downlink EUARFCN as used for Cell 1 |  |  |
| qRxLevMinEUTRA[n] | -53 | -106 dBm |  |
| threshXhigh[n] | 15 | 30 dB |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| qRxLevMinEUTRA[n] | -53 | -106 dBm |  |
| threshXhigh[n] | 15 | 30 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.3.34	Inter-RAT Cell reselection from E-UTRA to UTRA / MFBI

6.2.3.34.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an higher priority UTRAN neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { UE reselects the UTRA cell }

}

6.2.3.34.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

…

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. ...

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType6

The IE SystemInformationBlockType6 contains information relevant only for inter-RAT cell re-selection i.e. information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

…

SystemInformationBlockType6 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq in the CarrierFreqUTRA-FDD for which UTRA cell reselection parameters are common.

6.2.3.34.3	Test description

6.2.3.34.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell:

-	Cell 1 is a E-UTRA cell and is an MFBI capable cell

-	Cell 5 is a UTRA cell and is an MFBI capable cell

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

-	Cell 5 belongs to absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_UTRAN_FrequencyBand and px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in the E-UTRA cell.

UE:

-	UE does not support both of the px_OverlappingNotSupportedFrequencyBand_MFBI band and the px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.34.3.2	Test procedure sequence

Table 6.2.3.34.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.2.3.34.3.2-2.

Table 6.2.3.34.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | -65 | S nonServingCell, Cell5 > Thresh Cell 5,high |
|  | P-CCPCH | dBm/1.28 MHz |  | -67 |  |

Table 6.2.3.34.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.34.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | - |

6.2.3.34.3.3	Specific message contents

Table 6.2.3.34.3.3-1: SystemInformationBlockType1 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |
| freqBandIndicator |  | 64 | Band > 64 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | Overlapping Bands for Cell 1 under test (px_MFBI_FrequencyBand). | Cell 1 |
| freqBandIndicator |  | 65535 | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Band > 64 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.34.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  |  |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType6-v8h0-IEs ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD-v8h0 SEQUENCE { |  |  |  |
| CarrierFreqInfoUTRA-FDD-v8h0 SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| FreqBandIndicator-UTRA-FDD | Overlapping Bands for Cell 5 under test (px_MFBI_UTRAN_FrequencyBand). | Cell 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.34.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| Priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.34.3.3-4: System Information Block type 5 for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 25.331 clause 10.2.48.8.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Frequency Band Indicator | Not Present |  | A1 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A2 |
|  | Extension indicator |  | A3 or A4 |
| Frequency Band Indicator 2 | Not Present |  | A1 or A2 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A3 |
|  | Extension indicator |  | A4 |
| Frequency Band Indicator 3 | Not Present |  | A1 or A2 or A3 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A4 |
| Multiple Frequency Band indicator list | px_MFBI_UTRAN_FrequencyBand |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Band I, Band II, Band III |
| A2 | Band V, Band VI, Band VII |
| A3 | Band VIII & bands from Band X to Band XXII & |
| A4 | Band XXV and Band XXVIII |

Table 6.2.3.34.3.3-5: System Information Block type 5bis for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 25.331 clause 10.2.48.8.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Frequency Band Indicator | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A1 |
|  | Extension indicator |  | A2 |
| Frequency Band Indicator 2 | Not Present |  | A1 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A2 |
| Multiple Frequency Band indicator list | px_MFBI_UTRAN_FrequencyBand |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Band IV |
| A2 | Band IX and Band X |
| Band > 64 | If band > 64 is selected |

#### 6.2.3.35	Inter-RAT Cell reselection from UTRA to E-UTRA / MFBI

6.2.3.35.1	Test Purpose (TP)

(1)

with { UE in UTRA RRC_IDLE state }

ensure that {

when { the cell re-selection criteria is met for the E-UTRA cell included in the Multiple E-UTRA frequency info list }

then { the UE performs cell reselection to the E-UTRA cell }

}

6.2.3.35.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.331, clause 8.6.7.3c. Unless otherwise stated these are Rel-10 requirements.

[TS 25.331, clause 8.6.7.3c]

If the IE "E-UTRA frequency and priority info list" is received in System Information Block Type 19, the UE shall:

1>	for each occurrence of the IE "E-UTRA frequency and priority":

2>	create a new entry in the IE "Frequency Info List" in the variable EUTRA_FREQUENCY_INFO_LIST, and in that new entry:

3>	store the IEs "EARFCN" and "Measurement bandwidth";

3>	if the IE " Exclude-listed cells per freq list" is present in the IE "E-UTRA frequency and priority":

4>	store the exclude-listed cells information.

2>	if the UE supports multi-band signalling and the UE does not support the E-UTRA Frequency Band associated with the EARFCN in the IE "EARFCN", but does understand the EARFCN downlink (Nd):

3>	if the IE "Multiple E-UTRA frequency info list" is present for this occurrence of "E-UTRA frequency and priority":

4>	for the first entry in the IE "Multiple E-UTRA frequency band indicator list" which the UE supports, the UE shall:

5>	extract the supported band and compute the corresponding EARFCN downlink (Nd) value;

5>	overwrite the previously stored EARFCN value with the computed EARFCN downlink (Nd) value.

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

…

SystemInformationBlockType2 field descriptions

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1, listed in the same order.

6.2.3.35.3	Test description

6.2.3.35.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is a E-UTRA cell and is an MFBI capable cell.

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBand_MFBI.

UE:

-	UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5.

6.2.3.35.3.2	Test procedure sequence

Table 6.2.3.35.3.2-1 and Table 6.2.3.35.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.35.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.3.35.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -90 |  |
|  | P-CCPCH | dBm/1.28 MHz | -92 |  |
|  | Srxlev | dB | -11 |  |

Table 6.2.3.35.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change power levels according to the row "T1" in tables 6.2.3.35.3.2-1 and 6.2.3.35.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |

6.2.3.35.3.3	Specific message contents

Table 6.2.3.35.3.3-0: Conditions for specific message contents
in Tables 6.2.3.35.3.3-1 and 6.2.3.35.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.35.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.3.35.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE { |  |  |  |
| earfcn | Associated with an overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  | Band > 64 |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE {} |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE { |  |  |  |
| MultipleEutraFrequencyInfoList SEQUENCE { |  |  |  |
| MultipleEUTRAFrequencyBandInfo SEQUENCE { |  |  |  |
| MultipleEUTRAFrequencyBandIndicatorList SEQUENCE { |  |  |  |
| RadioFrequencyBandEUTRA | An overlapping Band under test (px_MFBI_FrequencyBand). |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MultipleEutraFrequencyInfoList | Not present |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE {} |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn | Associated with an overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |  |
| } |  |  |  |
| multipleEutraFrequencyInfoExtensionList SEQUENCE (SIZE (1.. maxNumEUTRAFreqs)) OF SEQUENCE (SIZE (1.. maxMultipleFrequencyBandsEUTRA)) OF SEQUENCE { |  |  |  |
| RadioFrequencyBandEUTRAExt | An overlapping Band under test (px_MFBI_FrequencyBand). |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.35.3.3-2: SystemInformationBlockType1 for cell 1 (preamble and all steps, Table 6.2.3.35.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBand_MFBI). |  |
| freqBandIndicator |  | 64 | Band > 64 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | Overlapping Bands for Cell 1 under test (px_MFBI_FrequencyBand). | Cell 1 |
| freqBandIndicator |  |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType1-v9e0-IEs SEQUENCE { |  |  |  |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_OverlappingNotSupportedFrequencyBand_MFBI). | Band > 64 |
| multiBandInfoList-v9e0 SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.35.3.3-3: SystemInformationBlockType2 for Cell 1 (preamble and all steps, Table 6.2.3.35.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 6.2.4	Inter-RAT absolute priority based reselection in UTRA CELL_FACH

#### 6.2.4.1	Inter-RAT absolute priority based reselection in UTRA CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Srxlev,x > Threshx,high and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2)

6.2.4.1.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and absolute priority measurement indicated }

ensure that {

when { Threshx,high2 or Threshx,low2 are not provided and the Srxlev for a higher absolute priority E-UTRA cell is greater than Threshx,high }

then { the UE performs cell reselection to the E-UTRA cell }

}

6.2.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements  are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

...Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.1.3	Test description

6.2.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 3.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 and moves to state PS-DCCH+DTCHFACH (state 6-11) according to [5].

6.2.4.1.3.2	Test procedure sequence

Table 6.2.4.1.3.2-1 and  Table 6.2.4.1.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.1.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.4.1.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.2.4.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.1.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.1.3.3	Specific message contents

Table 6.2.4.1.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.1.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | 31 (31dB) |  |  |
| threshXlow2[n] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | high-priority-layers |  |  |
| vb50NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.1.3.3-2: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.1.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.2	Inter-RAT absolute priority based reselection in UTRA CELL_FACH (Higher Priority Layers, no cell reselection to E-UTRA RRC_IDLE when Srxlev,serv < Sprioritysearch1)

6.2.4.2.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and absolute priority measurement indicated }

ensure that {

when { Srxlev for serving cell is lower than Sprioritysearch1 }

then { the UE performs no cell reselection to the E-UTRA cell }

}

6.2.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.2.3	Test description

6.2.4.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.2.3.2	Test procedure sequence

Table 6.2.4.2.3.2-1 and Table 6.2.4.2.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.2.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.4.2.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.2.4.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.2.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 10s? | - | - | 1 | F |

6.2.4.2.3.3	Specific message contents

Table 6.2.4.2.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.2.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| s-PrioritySearch1 | 31 (62 dB) | Highest |  |
| } |  |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE {} |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | high-priority-layers |  |  |
| vb50NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.2.3.3-2: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.2.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.3	Inter-RAT absolute priority based reselection in UTRA _CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Squal,x > Threshx,high2 and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2)

6.2.4.3.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and absolute priority measurement indicated }

ensure that {

when { the Squal for a higher absolute priority E-UTRA cell is greater than Threshx,high2 }

then { the UE performs cell reselection to the E-UTRA cell }

}

6.2.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements  are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;...Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.3.3	Test description

6.2.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.3.3.2	Test procedure sequence

Table 6.2.4.3.3.2-1 and Table 6.2.4.3.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.3.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
|  | Squal* | dB | 17 | Squal,x = RSRQ - qQualminEUTRA |

Table 6.2.4.3.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
|  | Squal* | dB | 21 |  |

Table 6.2.4.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.3.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | P |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.3.3.3	Specific message contents

Table 6.2.4.3.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.3.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh[n] | 31 (62 dB) | Highest value, so threshXhigh2 is applicable. |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE {} |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | high-priority-layers |  |  |
| vb50NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.3.3.3-2: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.3.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.4	Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Srxlev,x > Threshx,high)

6.2.4.4.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and Cell FACH absolute priority measurement indicated as All Layers }

ensure that {

when { Threshx,high2 or Threshx,low2 are not provided and the Srxlev for a higher absolute priority E-UTRA cell is greater than Threshx,high }

then { the UE performs cell reselection to the higher priority E-UTRA cell }

}

6.2.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements  are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

...Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.4.3	Test description

6.2.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.4.3.2	Test procedure sequence

Table 6.2.4.4.3.2-1 and Table 6.2.4.4.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.4.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.4.4.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.2.4.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.4.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.4.3.3	Specific message contents

Table 6.2.4.4.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.4.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | 31 (31dB) |  |  |
| threshXlow2[n] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | all-layers |  |  |
| vb50NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.4.3.3-2: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.4.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.5	Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Squal,x >ThreshX,high2)

6.2.4.5.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and Cell FACH absolute priority measurement indicated as All Layers }

ensure that {

when { UE determines that Threshx,high2 and Threshx,low2 are both provided and the Squal for a higher absolute priority E-UTRA cell is greater than Threshx,high2 }

then { UE performs cell reselection to the higher priority E-UTRA cell }

}

6.2.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements  are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

...Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.5.3	Test description

6.2.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 with QBASED is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.5.3.2	Test procedure sequence

Table 6.2.4.5.3.2-1 and Table 6.2.4.5.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.5.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RSRQ | dB | -16 |  |
|  | Noc | dBm/15kHz | -90 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -95 |  |
|  | Qrxlevmin | dBm | -106 | (default value in TS 36.508 Table 4.4.3.2-3) |
|  | Qqualmin | dB | -22 |  |
|  | Squal* | dB | 4 | Squal,x = RSRQ - qQualminEUTRA |
| T1 | RSRQ | dB | -4 |  |
|  | Noc |  | -90 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -65 |  |
|  | Squal* | dB | 17 | Squal,x = RSRQ - qQualminEUTRA |

Table 6.2.4.5.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Threshx,high | dB | 62 |  |
|  | Threshx,high2 | dB | 5 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.2.4.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.5.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | P |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.5.3.3	Specific message contents

Table 6.2.4.5.3.3-1: SystemInformationBlockType1 for Cell 1 (Pre-test conditions and all steps, Table 6.2.4.5.3.2-3)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -22 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.5.3.3-2: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.5.3.2-3)

| Derivation Path: 36.508, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| threshXhigh [n] | 31 (62 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} | Not present |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | all-layers |  |  |
| } |  |  |  |
| vb50NonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.6.3.3-3: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.6.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.6	Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Srxlev,serv <Thresh serv,low and Srxlev,x > Threshx,low)

6.2.4.6.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and Cell FACH absolute priority measurement indicated as All Layers }

ensure that {

when { Threshx,high2 or Threshx,low2 are not provided, Srxlev < Sprioritysearch1 and Srxlev < Thresh serv,low for serving UTRA cell, and the Srxlev for a lower absolute priority E-UTRA cell is greater than Threshx,low  }

then { the UE performs cell reselection to the lower priority E-UTRA cell }

}

6.2.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements  are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

…

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

...

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= Threshserving,low2) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

... Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.6.3	Test description

6.2.4.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.6.3.2	Test procedure sequence

Table 6.2.4.6.3.2-1 and Table 6.2.4.6.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.6.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev* | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.4.6.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.2.4.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change Cell 1 level according to the row "T1" in table 6.2.4.6.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | P |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.6.3.3	Specific message contents

Table 6.2.4.6.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.6.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| s-PrioritySearch1 | 13 (26dB) |  |  |
| threshServingLow | 13 (26dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | The same number of entries as the configured eutra carriers For Signalling test cases, see table 6.3.1.7-1 | n denotes the index of the entry |  |
| priority[n] | 2 |  |  |
| threshXhigh[n] | 2 (4 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} | Not present |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | all-layers |  |  |
| } |  |  |  |
| vb50NonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.6.3.3-2: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.6.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.7	Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Squal,serv <Thresh serv,low2 and Squal,x > ThreshX,low2)

6.2.4.7.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell FACH state and Cell FACH absolute priority measurement indicated as All Layers }

ensure that {

when { Threshx,high2 and Threshx,low2 are both provided and the Squal for a lower absolute priority E-UTRA cell is greater than Threshx,low2 and the Srxlev and Squal for the serving cell are lower than Sprioritysearch1 and Thresh serv,low2 respectively }

then { the UE performs cell reselection to the lower priority E-UTRA cell }

}

6.2.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.1.2a, 5.2.6.1.4a and TS 25.331, 8.1.1.6.19.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

The UE is required to perform measurements of inter-frequency and inter-RAT cells listed in system information according to Measurement requirements are specified in [10]. UE specific priorities are not applied in camped on any cell state.

…

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	FFS

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

-	…

-	In CELL_FACH state: If there are UTRAN inter-frequency, GERAN and E-UTRAN neighbours configured and required to be measured according to the measurement rules above, then the UE shall perform measurements on UTRAN inter-frequency and E-UTRAN layers (i.e. the UE is not required to measure GERAN, regardless of whether GERAN has absolute priorities assigned).

[TS 25.304, clause 5.2.6.1.4a]

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

- 	Criterion 5: for FDD, (SrxlevServingCell < =Threshserving,low or SqualServingCell < Threshserving,low2) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection; for 1.28 Mcps TDD, (SrxlevServingCell < Threshserving,low) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

[TS 25.331, clause 8.1.1.6.19]

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" and "EUTRA-RSRQ-offsetWB" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

1>	if the IE "GSM priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3b.

1>	if the IE "E-UTRA frequency and priority info list" is present:

2>	act upon the received IE as described in subclause 8.6.7.3c.

1>	if the IE "E-UTRA frequency RACH reporting information" is present; and

1>	if the UE supports E-UTRA RACH reporting:

2>	store the IE "E-UTRA frequency RACH reporting information" into the variable EUTRA_FREQUENCY_INFO_LIST_FACH and act upon the received IE as described in subclause 8.5.23.

6.2.4.7.3	Test description

6.2.4.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 with QBASED is used in E-UTRA Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 and moves to state PS-DCCH+DTCH FACH (state 6-11) according to [5].

6.2.4.7.3.2	Test procedure sequence

Table 6.2.4.7.3.2-1 and Table 6.2.4.7.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.4.7.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RSRQ | dB | -16 |  |
|  | Noc | dBm/15kHz | -80 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Qrxlevmin | dBm | -106 | (default value in TS 36.508 Table 4.4.3.2-3) |
|  | Qqualmin | dB | -22 |  |
|  | Squal* | dB | 6 | Squal,x = RSRQ - qQualminEUTRA |
| T1 | RSRQ | dB | -4 |  |
|  | Noc |  | -80 |  |
|  | Cell-specific RS EPRE | dBm/15kHz | -65 |  |
|  | Squal* | dB | 18 | Squal,x = RSRQ - qQualminEUTRA |

Table 6.2.4.7.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | CPICH Ec/N0 | dB | -7 |  |
|  | Ioc (FDD) | dBm/3.84MHz | -55 |  |
|  | Ioc (TDD) | dBm/1.28MHz | -57 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin (TDD) | dBm | -81 | Default value |
|  | Qqualmin | dB | -24 | Default value |
|  | Sprioritysearch1 | dB | 24 |  |
|  | Threshserving,low | dB | 0 | Default value |
|  | Threshserving,low2 | dB | 12 |  |
|  | QqualminEUTRA | dB | -22 |  |
|  | Threshx,high2 | dB | 31 |  |
|  | Threshx,low2 | dB | 10 | Default value |
|  | Srxlev* | dB | 19 |  |
|  | Squal * | dB | 17 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -73 |  |
|  | CPICH Ec/N0 | dB | -18 |  |
|  | Ioc (FDD) | dBm/3.84MHz | -55 |  |
|  | Ioc (TDD) | dBm/1.28MHz | -57 |  |
|  | P-CCPCH | dBm/1.28 MHz | -75 |  |
|  | Srxlev* | dB | 6 |  |
|  | Squal * | dB | 6 |  |

Table 6.2.4.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Check: Does the UE send an RRCConnectionRequest on Cell 1 within the next 60 s? | --> | RRCConnectionRequest | 1 | F |
| 1 | The SS change Cell 1 and Cell 5 level according to the row "T1" in table 6.2.4.7.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1 NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | P |
| - | Exception: Steps 3 to 3a5/3b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 3 | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 3a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 3a2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 3a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 3a4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 3a5 | The SS releases the RRC connection | - | - | - | - |
| 3b1 | The 10s timer expires | - | - | - | - |

6.2.4.7.3.3	Specific message contents

Table 6.2.4.7.3.3-1: SystemInformationBlockType1 for Cell 1 (Pre-test conditions and all steps, Table 6.2.4.7.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -22 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.7.3.3-2: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.7.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1, condition QBASED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| s-PrioritySearch1 | 12 (24dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 12 (12dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[0] | -22 (-22dB) |  |  |
| threshXhigh2[0] | 31 (31dB) |  |  |
| threshXlow2[0] | 10 (10 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE {} | Not present |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | all-layers |  |  |
| } |  |  |  |
| vb50NonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.4.2.7.3-3: System Information Block type 11 for Cell 5 (Pre-test conditions and all steps, Table 6.2.4.7.3.2-3

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType11 ::= SEQUENCE { |  |  |  |
| fach-MeasurementOccasionInfo SEQUENCE { |  |  |  |
| fACH-meas-occasion-coeff | 3 |  |  |
| inter-freq-FDD-meas-ind | FALSE |  |  |
| inter-RAT-meas-ind (SIZE (1.. maxOtherRAT)) OF SEQUENCE {} | { gsm } |  |  |
| } |  |  |  |
| } |  |  |  |

## 6.3	Closed Subscriber Group cells

### 6.3.1	Inter-frequency Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE CSG cell

6.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { Manual CSG ID selection is requested }

then { UE selects the requested CSG cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state on a non-CSG cell and UE’s Allowed CSG list is not empty }

ensure that {

when { a previously visited allowed E-UTRA CSG cell becomes the strongest cell on its frequency }

then { UE reselects the E-UTRA CSG cell }

}

6.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.8.1, and 36.133, clauses 4.2.2.10 and 4.2.2.10.1.

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

[TS 36.133, clause 4.2.2.10]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list.  The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

[TS 36.133, clause 4.2.2.10.1]

The UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.10.1-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.1-1: Parameters for CSG inter-frequency reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| EARFCN Note1 |  | Channel 1 | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | 2 |
| CSG identity |  | Not sent | Already stored in UE allow-list from previous visit |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 | -140 |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | -110 | -110 |
| Note 1: For this requirement to be applicable, the EARFCN and physical cell identity for cell 1 and cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

6.3.1.3	Test description

6.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3, Cell 11 and Cell 23 have different tracking areas according to table 6.0.1-2.

-	Cell 1, Cell 11 and Cell 23 are not CSG cells.

-	Cell 3 is a CSG cell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1,Cell 11 and Cell 23.

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 3.

UE:

-	The UE’ s allowed CSG list shall not contain CSG ID 3.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.3.1.3.2	Test procedure sequence

Table 6.3.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.3.1.3.2-2.

Table 6.3.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 11 | Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | Off | Off | The power levels are such that camping on Cell 1 is guaranteed. |
|  | Qrxlevmin | dBm | -140 | -140 | -140 | -140 |  |
|  | Srxlev* | dB | 75 | - | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -65 | -85 | Off | -91 |  |
|  | Srxlev* | dB | 75 | 55 | - | 49 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Off | Off | -65 | Off | The power levels are such that camping on Cell 11 is guaranteed. |
|  | Srxlev* | dB | - | - | 75 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | -91 | -85 |  |
|  | Srxlev* | dB | 75 | - | 49 | 55 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -65 | -85 | Off | -91 |  |
|  | Srxlev* | dB | 75 | 55 | - | 49 |  |

Table 6.3.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-17 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: the UE performs registration and the RRC connection is released. | - | - | - | - |
| 18 | The SS changes Cell 3 and Cell 23 levels according to row "T1" in table 6.3.1.3.2-1. | - | - | - | - |
| 19 | The UE is made to perform manual CSG ID selection and select Cell 3. | - | - | - | - |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 21 | The SS changes Cell1, Cell 3, Cell 11 and Cell 23 levels according to row "T2" in table 6.3.1.3.2-1. | - | - | - | - |
| 22-27 | Steps 1 to 6 of the TAU procedure described in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: the UE performs a TAU and the RRC connection is released. | - | - | - | - |
| 28 | The SS changes Cell 1, Cell 11 and Cell 23 levels according to row "T3" in table 6.3.1.3.2-1. | - | - | - | - |
| 29-34 | Steps 1 to 6 of the TAU procedure described in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: the UE performs a TAU and the RRC connection is released. | - | - | - | - |
| 35 | The SS changes Cell 3, Cell 11 and Cell 23 levels according to row "T4" in table 6.3.1.3.2-1. | - | - | - | - |
| 36-40 | Check: Does the test result of Steps 1 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3 within 6 mins from "T4" to the time RRCConnectionRequest received by SS? NOTE: The UE performs a TAU procedure. | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

6.3.1.3.3	Specific message contents

Table 6.3.1.3.3-1: Conditions for table 6.3.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 11 | This condition applies to system information transmitted on Cell 11. |
| Cell 23 | This condition applies to system information transmitted on Cell 23. |

Table 6.3.1.3.3-2: SystemInformationBlockType1 for Cell 1, 3, 11 and 23 (preamble and all steps, Table 6.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  | Cell 1 Cell 11 Cell 23 |
|  | TRUE |  | Cell 3 |
| csg-Identity | Not present |  | Cell 1 Cell 11 Cell 23 |
|  | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.1.3.3-3: SystemInformationBlockType4 for Cell 3 (Pre-test conditions and all steps, table 6.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 3 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.2	Inter-RAT Cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA idle CSG cell

6.3.2.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Manual CSG ID selection is requested }

then { UE select the suitable E-UTRAN CSG cell }

}

(2)

with { UE in GSM/GPRS Registered state and no RR connection and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRAN CSG cell previously visited }

then { UE reselects the suitable previously visited E-UTRAN CSG cell irrespective of the EUTRAN cell reselection priority }

}

6.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.2.4, TS 45.008, clause 6.6.7.1, TS 45.008, clause 6.6.7.2 and TS 36.304, clause 10.1.3.4.

[TS 24.301, clause 5.5.3.2.4]

If the UE has initiated the tracking area updating procedure due to manual CSG selection and receives a TRACKING AREA UPDATE ACCEPT message, the UE shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSD ID to the Allowed CSG list.

[TS 45.008, clause 6.6.7.1]

If a mobile station is a member of at least one Closed Subscriber Group, i.e. at least one CSG ID is included in the MS’s "Allowed CSG list", then, in addition to normal cell reselection, the MS shall use an autonomous search function to detect UTRAN and/or E-UTRAN CSG cells. The autonomous search function shall at least detect previously visited allowed CSG cells.

NOTE 1:	The autonomous search function is implementation dependent and controls when and/or where to search for allowed CSG cells.

…

If the strongest cell (see 3GPP TS 25.304 and 3GPP TS 36.304 for the definition of the strongest cell) which the MS has detected on a UTRAN or E-UTRAN frequency during a time interval T_reselection is a suitable CSG cell (see 3GPP TS 25.304 and 3GPP TS 36.304 for suitability criteria for UTRAN and E-UTRAN CSG cells respectively), the MS shall reselect to this cell irrespective of the cell reselection rules applicable for the cell on which the MS is currently camped.

The following requirements are valid for reselection to allowed CSG cells previously visited by the MS when the radio configuration parameters, including the carrier frequency and PSC/PCI (whichever and if it is applicable) of the CSG cell, non CSG cell and other neighbour cells, are unchanged from the most recent previous visit. The autonomous search and cell re-selection to a previously visited allowed CSG cell shall meet the performance requirements defined as follows:

- the MS shall perform search and re-selection to a previously visited allowed UTRAN FDD CSG cell, that has met the CSG cell re-selection criterion defined above, within 6 minutes in the radio conditions specified for an UTRAN FDD CSG cell in 3GPP TS 36.133 in terms of parameters for CSG inter-RAT UTRAN FDD reselection; UARFCN and PSC shall be unchanged from the most recent previous visit of the UTRAN FDD CSG cell; Serving GSM cell at RXLEV= -70 dBm, with 6 GSM neighbours at RXLEV= -75 dBm;

- the MS shall perform search and re-selection to a previously visited allowed E-UTRAN CSG cell, that has met the CSG cell re-selection criterion defined above, within 6 minutes in the radio conditions specified for an E-UTRAN CSG cell in 3GPP TS 25.133 in terms of parameters for CSG inter-RAT E-UTRA reselection; E-UARFCN and PCI shall be unchanged from the most recent previous visit of the E-UTRAN CSG cell; Serving GSM cell at RXLEV= -70 dBm, with 6 GSM neighbours at RXLEV= -75 dBm.

NOTE 4:	The above performance requirements are minimum requirements defined to ensure the testability of autonomous CSG search.

[TS 45.008, clause 6.6.7.2]

If NAS requests AS to search for available CSG IDs, the MS shall perform the search and report the results to NAS as described in 3GPP TS 25.304 for UTRAN and as described in 3GPP TS 36.304 for E-UTRAN.

If a CSG ID is manually selected by NAS, the MS shall behave as specified in 3GPP TS 25.304 or in 3GPP TS 36.304, depending on the RAT type of the selected CSG cell.

[TS 45.008, clause 10.1.3.4]

If a mobile station is a member of at least one Closed Subscriber Group, i.e. at least one CSG ID is included in the MS’s "Allowed CSG list", then, in addition to normal cell reselection, the MS shall use an autonomous search function to detect UTRAN and/or E-UTRAN CSG cells. The autonomous search function shall at least detect previously visited allowed CSG cells.

6.3.2.3	Test description

6.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	Cell 24 and Cell 25.

-	Cell 1, Cell 24 and Cell 25 are not CSG cells.

-	Cell 2 is CSG cell.

-	Cell 24 and Cell 25 have different Routing Area.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 14 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2.

UE:

-	The UE’s allowed CSG list shall not contain CSG ID 1.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.3.2.3.2	Test procedure sequence

Tables 6.3.2.3.2-1 and 6.3.2.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.3.2.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Srxlev* | dB | “Off” | “Off” | S nonServingCell, Cell7 > Thresh Cell7,high |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.2.3.2-2: Time instances of cell power level and parameter changes for GERAN cell

|  | Parameter | Unit | Cell 24 | Cell 25 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | RF Signal Level | dBm | -80 | “Off” | The power levels are such that camping on Cell 24 is guaranteed. |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| T1 | RF Signal Level | dBm | -80 | “Off” |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| T2 | RF Signal Level | dBm | “Off” | -80 | The power levels are such that camping on Cell 25 is guaranteed. |
|  | RXLEV_ACCESS_MIN | dBm | “Off” | -101 |  |
|  | C1* | dB | “Off” | 21 |  |
| T3 | RF Signal Level | dBm | -80 | -90 |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | -101 |  |
|  | C1* | dB | 21 | 11 |  |
| T4 | RF Signal Level | dBm | -80 | “Off” |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| Note: C1 is calculated in the UE |  |  |  |  |  |

Table 6.3.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Call the registration procedure described in TS 34.123-3 subclause 6.10.2.7.1.1 performed on Cell 24. NOTE: The UE performs registration and the RR connection is released. | - | - | - | - |
| 2A | The UE may attempt the procedure for IMS signalling on GERAN as specified in TS 36.508 clause 4.5A.3B | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.3.2.3.2-1. | - | - | - | - |
| 4 | The UE is made to perform manual CSG ID selection and select Cell 2. | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 2? | - | - | 1 | - |
| 5A | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 6 | The SS changes Cell 1, Cell 2, Cell 24 and Cell 25 levels according to row "T2" in table 6.3.2.3.2-1 and table 6.3.2.3.2-2. | - | - | - | - |
| 7-17 | Steps 1 to 11 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.9 are performed on Cell 25. NOTE: The UE performs RAU and the RR connection is released. | - | - | - | - |
| 18 | The SS changes Cell 1, Cell 24 and Cell 25 levels according to row "T3" in table 6.3.2.3.2-1 and table 6.3.2.3.2-2. | - | - | - | - |
| 19-29 | Steps 1 to 11 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.9 are performed on Cell 24. NOTE: The UE performs RAU and the RR connection is released. | - | - | - | - |
| 30 | The SS changes Cell 1, Cell 2 and Cell 25 level according to row "T4" in table 6.3.2.3.2-1 and table 6.3.2.3.2-2. | - | - | - | - |
| 31 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 2 within 6 mins? | - | - | 2 | - |

        - Specific message contents

Table 6.3.2.3.3-1: SystemInformationBlockType1 for Cell 2 (preamble and all steps, Table 6.3.2.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  |  |
| csg-Identity | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.3.3-2: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.3.2.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange SEQUENCE { |  |  |  |
| start | 2 |  |  |
| range | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.3.3-3: SYSTEM INFORMATION TYPE 2 Quarter for Cell 24 and Cell 25 (Pre-test conditions and all steps, Table 6.3.2.3.2-3)

| Derivation Path: 36.508 clause 4.4.5, Table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 2QUATER REST OCTETS |  |  |  |
| Additions in release 8 | H |  |  |
| Priority and E-UTRAN Parameters Description | 1 | Present |  |
| Serving Cell Priority Parameters Description | 1 | Present |  |
| GERAN_PRIORITY | 5 | Higher priority than E-UTRA |  |

Table 6.3.2.3.3-4: SystemInformationBlockType7 for Cell 1 and Cell 2 (Pre-test conditions and all steps, Table 6.3.2.3.2-3)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 | INTEGER (0..7) |  |
| speedDependentScalingParameters SEQUENCE {} | Not present |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqsGERAN SEQUENCE [1] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 24 |  |  |
| numberOfFollowingARFCNs | Same as cell 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Higher priority than EUTRA Freq |  |
| } |  |  |  |
| carrierFreqsGERAN SEQUENCE [2] { |  |  |  |
| startingARFCN | Same as cell 25 |  |  |
| bandIndicator | Same as cell 25 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 25 |  |  |
| numberOfFollowingARFCNs | Same as cell 25 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Higher priority than EUTRA Freq |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.3	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE CSG cell

6.3.3.1	Test Purpose (TP)

(1)

with { UE in UTRA IDLE state }

ensure that {

when { Manual CSG ID selection is requested }

then { UE select the suitable E-UTRAN CSG cell }

}

(2)

with { UE in UTRA IDLE state and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRAN CSG cell previously visited }

then { UE reselects the suitable previously visited E-UTRAN CSG cell irrespective of the EUTRAN cell reselection priority }

}

6.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.6.1, 5.2.6.4.1, TS 36.304 clause 5.2.4.8.1, 5.2.4.6, 5.2.4.8.2, 5.2.4.1 and TS 25.133, clause 4.2.2.11.2.

[TS 24.301, clause 5.5.3.2.4]

If the UE has initiated the tracking area updating procedure due to manual CSG selection and receives a TRACKING AREA UPDATE ACCEPT message, the UE shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSD ID to the Allowed CSG list.

[TS 25.304, clause 5.6.1]

In the UE on request of NAS, the AS shall scan all RF channels in the UTRA bands according to its capabilities to find available CSG IDs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) together with their “HNB name” (if broadcast) and PLMN(s) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 25.304, clause 5.2.6.4.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE is required to perform autonomous search function in Idle, Cell_PCH and URA_PCH states. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty. If "Dedicated CSG frequency(ies) " IE is present, the UE may use the autonomous search function only on these dedicated frequencies and on the other frequencies listed in the system information.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

When the UE has no or an empty Permitted CSG list, the UE may ignore cells with PSC in the stored range "CSG PSC Split Information" [4] reserved for CSG cells for intra-frequency and inter-frequency measurements and cell re-selections.

…

If the UE detects a suitable CSG cell on a different frequency it shall reselect this cell irrespective of the cell reselection rules applicable for the cell the UE is currently camped on, if the detected suitable CSG cell is the strongest cell on that frequency. If suitable CSG cells are detected on different frequencies and these are the strongest cells on their frequencies, then the UE shall reselect to any one of them.

If the UE detects one or more suitable CSG cell on another RAT, the UE shall reselect to one of them according to [18].

[TS 25.133, clause 4.2.2.11.2]

The UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.11.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.2-1: Parameters for CSG inter-RAT E-UTRA reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| EARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_RSCPNote2 | dBm | -100 | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | -110 |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the EARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG listUE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG listUE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

If the UE detects a suitable CSG cell on the same frequency, it shall reselect to this cell as per normal reselection rules (5.2.4.6.).

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.2.4.8.2]

While camped on a suitable CSG cell, the UE shall apply the normal cell reselection rules as defined in subclause 5.2.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection.

…

While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency.

6.3.3.3	Test description

6.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	Cell 5 and Cell 7.

-	Cell 1, Cell 5 and Cell 7 are not CSG cells.

-	Cell 2 is CSG cell.

-	Cell 5 and Cell 7 have different Routing Area.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 11 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2.

UE:

-	The UEs allowed CSG list shall not contain CSG ID 1.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 (serving cell).

6.3.3.3.2	Test procedure sequence

Tables 6.3.3.3.2-1 and 6.3.3.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.3.3.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Srxlev* | dB | N/A | N/A | S nonServingCell, Cell7 > Thresh Cell7,high |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.3.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 | “Off” | The power levels are such that camping on Cell 5 is guaranteed. |
|  | P-CCPCH | dBm/1.28 MHz | -62 | “Off” | The power levels are such that camping on Cell 5 is guaranteed. |
|  | Qrxlevmin (FDD) | dBm | -79 | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | -81 | Default value |
|  | Srxlev* | dB | 19 | “Off” |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 | “Off” |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 | “Off” |  |
|  | Srxlev* | dB | 19 | “Off” |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | “Off” | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | “Off” | -62 |  |
|  | Srxlev* | dB | “Off” | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -60 | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 | -72 |  |
|  | Srxlev* | dB | 19 | 9 |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | -60 | “Off” |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 | “Off” |  |
|  | Srxlev* | dB | 19 | “Off” |  |
| Note : Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void |  |  |  |  |
| 2-13 | Void |  |  |  |  |
| 14 | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.3.3.3.2-1. | - | - | - | - |
| 15 | The UE is made to perform manual CSG ID selection and select Cell 2. | - | - | - | - |
| - | EXCEPTION : Step 16a1 to 16b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN |  |  |  |  |
| 16 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 2? | - | - | 1 | - |
| 16b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 2? |  |  |  |  |
| 16A | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 17 | The SS changes Cell 1, Cell 2, Cell 5 and Cell 7 levels according to row "T2" in table 6.3.3.3.2-1 and table 6.3.3.3.2-2. | - | - | - | - |
| 18-27 | Steps 1 to 10 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 7. NOTE: The UE performs RAU and the RRC connection is released. | - | - | - | - |
| 28 | The SS changes Cell 1, Cell 5 and Cell 7 levels according to row "T3" in table 6.3.3.3.2-1 and table 6.3.3.3.2-2. | - | - | - | - |
| 29-38 | Steps 1 to 10 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs RAU and the RRC connection is released. | - | - | - | - |
| 39 | The SS changes Cell 1, Cell 2 and Cell 7 level according to row "T4" in table 6.3.3.3.2-1 and table 6.3.3.3.2-2. | - | - | - | - |
| 40-47 | Check: Does the test result of Steps 1 to 8 of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 2 within 6 mins from "T4" to the time RRCConnectionRequest received by SS? NOTE: The UE performs a TAU procedure. | - | - | 2 | - |

        - Specific message contents

Table 6.3.3.3.3-1: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.3.3.3.3-2: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.3.3.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  | Cell 1 |
|  | TRUE |  | Cell 2 |
| csg-Identity | Not present |  | Cell 1 |
|  | 1 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.3.3-3: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.3.3.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange SEQUENCE { |  |  |  |
| start | 2 |  |  |
| range | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.3.3-4: System Information Block type 19 for Cell 5 and Cell 7 (Pre-test conditions and all steps, Table 6.3.3.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn[n] | Same downlink EARFCN as used for cell 1 |  |  |
| priority[n] | 4 |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  | Band > 64 |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [n] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.3.3-5: SystemInformationBlockType6 for Cell 1 and Cell 2 (Pre-test conditions and all steps, Table 6.3.3.3.2-3

| Derivation Path: 36.508 Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.4	Inter-RAT Cell reselection / From UTRA CELL_PCH state to E-UTRA RRC_IDLE CSG cell

6.3.4.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell PCH state and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable CSG cell that is not the strongest cell on that frequency }

then { UE does not reselect to the CSG cell }

}

(2)

with { UE in UTRA Cell PCH state and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRAN CSG cell previously visited }

then { UE reselects the suitable previously visited E-UTRAN CSG cell irrespective of the EUTRAN cell reselection priority }

}

6.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.4.1, TS 36.304 clause 5.2.4.8.1, 5.2.4.6, 5.2.4.8.2, 5.2.4.1, TS 25.133 clause 4.2.2.11.2 and TS 25.331, Annex E.

[TS 25.304, clause 5.2.6.4.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE is required to perform autonomous search function in Idle, Cell_PCH and URA_PCH states. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty. If "Dedicated CSG frequency(ies) " IE is present, the UE may use the autonomous search function only on these dedicated frequencies and on the other frequencies listed in the system information.

NOTE: The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

When the UE has no or an empty Permitted CSG list, the UE may ignore cells with PSC in the stored range "CSG PSC Split Information" [4] reserved for CSG cells for intra-frequency and inter-frequency measurements and cell re-selections.

…

If the UE detects a suitable CSG cell on a different frequency it shall reselect this cell irrespective of the cell reselection rules applicable for the cell the UE is currently camped on, if the detected suitable CSG cell is the strongest cell on that frequency. If suitable CSG cells are detected on different frequencies and these are the strongest cells on their frequencies, then the UE shall reselect to any one of them.

If the UE detects one or more suitable CSG cell on another RAT, the UE shall reselect to one of them according to [18].

[TS 25.133, clause 4.2.2.11]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list. The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

NOTE:	According to [1], the UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

[TS 25.133, clause 4.2.2.11.2]

The UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.11.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.2-1: Parameters for CSG inter-RAT E-UTRA reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| EARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_ RSCPNote2 | dBm | -100 | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | -110 |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the EARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s CSG whitelist. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s CSG whitelist is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

If the UE detects a suitable CSG cell on the same frequency, it shall reselect to this cell as per normal reselection rules (5.2.4.6.).

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.2.4.8.2]

While camped on a suitable CSG cell, the UE shall apply the normal cell reselection rules as defined in subclause 5.2.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection.

…

While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency.

[TS 25.331, Annex E]

This annex contains the definitions of the bits in EUTRA Feature Group Indicators.

In this release of the specification the UE shall include the optional EUTRA Feature Group Indicators IE in the UE multi-mode/multi RAT capability IE of the RRC message. For a specific indicator, if all functionalities for a feature group listed in Table E.1 have been implemented and tested, the UE shall set the indicator as "true" (as one), else (i.e. if any one of the functionalities in a feature group listed in Table E.1 have not been implemented or tested), the UE shall set the indicator as zero.

The UE shall set all indicators, which do not have a definition in the table E.1, as "false" (as zero).

If the optional EUTRA Feature Group Indicators IE is not included by a UE supporting a future release of the specification, the network may assume the UE supports all features in the feature groups listed in table E.1 and deployed in the network.

Table E.1: Definitions of feature group indicators

| Index of indicator (bit number) | Definition (description of the supported functionality, if indicator set to ‘true’) | Notes |
| --- | --- | --- |
| 1 (leftmost bit) | - UTRA CELL_PCH to EUTRA RRC_IDLE cell reselection - UTRA URA_PCH to EUTRA RRC_IDLE cell reselection |  |
| 2 | EUTRAN measurements and reporting in connected mode |  |
| 3 | Undefined |  |
| 4 | Undefined |  |

Clarification for mobility to EUTRAN

For mobility to E-UTRAN, it is assumed that we have 6 main "functions":

A.	Support of measurements and cell reselection procedure in idle mode

B.	Support of measurements and cell reselection procedure in CELL/URA_PCH

C.	Support of RRC release with redirection procedure

D.	Support of RRC reject with redirection procedure

E.	Support of EUTRAN measurements and reporting in connected mode

F.	Support of handover procedure in connected mode

Of the above, all UEs that indicate support of E-UTRA in UE capability signalling "Support of E-UTRA FDD" or "Support of E-UTRA TDD" support A), C) and D) EUTRA bands.

For B) above, UEs indicate support by the Group 1 indicator bit (if Group 1 is set to "true", it is supported for all EUTRA bands the UE supports).

For E) above, UEs indicate support by the Group 2 indicator bit (if Group 2 is set to "true", it is supported for all EUTRA bands the UE supports). The compressed mode capability for supported E-UTRA frequency bands ("Need for compressed mode") in "Measurement capability extension" is ignored by the network when Group 2 is set to "false".

For F) above, UEs indicate support by the separate UE capability signalling "Support of Inter-RAT PS Handover to E-UTRA FDD/TDD" defined in TS 25.306 (if this bit is set to "true", PS handover is supported for all EUTRA bands the UE supports). This bit can only be set to "true" if the UE has set the Group 2 indicator bit to "true".

6.3.4.3	Test description

6.3.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	Cell 5.

-	Cell 1, Cell 5 are not CSG cells.

-	Cell 2 is CSG cell.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 11 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2.

UE:

-	The UE’s Allowed CSG list  shall not contain CSG ID 1.

Preamble:

-	 The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.3.4.3.2	Test procedure sequence

Table 6.3.4.3.2-1 and  Table 6.3.4.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.3.4.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
|  | Srxlev* | dB | NA | NA |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
|  | Srxlev* | dB | 21 | 15 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.4.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T4 | “Off” | Srxlev | -60 |  |
|  | “Off” | dBm/1.28 MHz | -62 |  |
|  | Srxlev | dB | 19 |  |

Table 6.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | Void |  |  |  |  |
| 0B-0M | Void |  |  |  | - |
| 0N | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.3.4.3.2-1. | - | - | - | - |
| 0O | The UE is made to perform manual CSG ID selection and select Cell 2. | - | - | - | - |
|  | NOTE: The UE is camped on E-UTRAN Cell 2. | - | - | - | - |
| 0P | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 0Q | The SS changes Cell 1, Cell 2 and Cell 5 levels according to row "T2" in table 6.3.4.3.2-1 and table 6.3.4.3.2-2. | - | - | - | - |
| 0R | Steps 1 to 8 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs RAU. | - | - | - | - |
| 0S | The Procedure for bringing the UE to cell_PCH state defined in TS34.108 clause 7.4.2.7 takes place and the UE is moved to CELL-PCH state (Note 2) | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 2 levels according to the row "T3" in table 6.3.4.3.2-1. (Note 1) | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 6 minutes? | - | - | 1 | F |
| 3 | The SS changes Cell 1 and Cell 2 levels according to the row "T4" in table 6.3.4.3.2-1. | - | - | - | - |
| 4-11 | Check: Does the test result of steps 1 to 8 of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 2 within 6 minutes from "T4" to the time RRCConnctionRequest received by SS? NOTE: The UE performs a TAU procedure | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS should not send any RRC message after UE has successfully proceed to CELL_PCH state. Note 2: At the step 0S UE already has a PDP context therefore RadioBearerSetup message should be used based on reported NSAPI from activate PDP context message instead of default value. |  |  |  |  |  |

6.3.4.3.3	Specific message contents

Table 6.3.4.3.3-1: Conditions for specific message contents in Table 6.3.4.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |

Table 6.3.4.3.3-2: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.3.4.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  | Cell 1 |
|  | TRUE |  | Cell 2 |
| csg-Identity | Not present |  | Cell 1 |
|  | 1 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.3.3-3: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.3.4.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange SEQUENCE { |  |  |  |
| start | 2 |  |  |
| range | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.4.3.3-4: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.3.4.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.4.3.3-5: Void

Table 6.3.4.3.3-6: SystemInformationBlockType6 for Cell 1 and Cell 2 (Pre-test conditions and all steps, Table 6.3.4.3.2-3

| Derivation Path: 36.508 Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.3.4.3.3-7: Physical Channel Reconfiguration for Cell 5 (Pre-test conditions Table 6.3.4.3.2-3)

| Derivation Path: 34.108 sub clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| > RRC State indicator | CELL_PCH |  | UTRA-PCH-A9 |
| > UTRAN DRX cycle length coefficient | 8 |  | UTRA-PCH-A9 |

| Condition | Explanation |
| --- | --- |
| UTRA-PCH-A9 | "Packet to CELL_PCH from CELL_FACH in PS" |

### 6.3.5	Manual support for CSG ID selection

6.3.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an intra-frequency CSG ID cell in the CSG subscription data for the UE at the network side is manually selected }

then { UE camps on a cell with the selected CSG ID }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an inter-frequency CSG ID cell in the UE’s Allowed CSG list but not in the CSG subscription data for the UE at the network side is CSG autonomously selected, and when trying to camp on the selected cell the UE is rejected by the network by sending TRACKING AREA UPDATE REJECT with cause #25 }

then { the UE updates the Allowed CSG list by deleting this CSG ID from the UE’s Allowed CSG list }

}

(3)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an inter-frequency CSG ID cell not in the UE’s Allowed CSG list but in the CSG subscription data for the UE at the network side is manually selected }

then { UE camps on the strongest cell with the selected CSG ID, performs a TAU procedure and updates the UE’s Allowed CSG list by adding this CSG ID into the UE’s Allowed CSG list }

}

6.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.5.1, clause 5.2.4.8.2, TS 23.122, clause 3.1A and clause 4.4.3.1.3.

[TS 36.304, clause 5.5.1]

In the UE on request of NAS, the AS shall scan all RF channels in the E-UTRA bands according to its capabilities to find available CSG IDs of available CSG cells. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) belonging to the registered PLMN  together with their “HNB name” (if broadcast) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 36.304, clause 5.2.4.8.2]

While camped on a suitable CSG cell, the UE shall apply the normal cell reselection rules as defined in subclause 5.2.4.

[TS 23.122, clause 3.1A ]

If the MS supports CSG, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM if the list is available in the USIM or as described in 3GPP TS 24.285 [47] if the list is not available in the USIM. This list has zero or more entries. There are two modes of CSG selection:

…

-	Manual mode: In this mode, the MS indicates to the user the list of available CSGs in the currently registered PLMN. The list of CSGs presented to the user is not restricted by the Allowed CSG list stored in the MS. After the user makes a selection, the MS camps on a cell with the selected CSG identity and may attempt to register with the associated PLMN (see 3GPP TS 24.008 [23] and 3GPP TS 24.301 [23A]).

[TS 23.122, clause 4.4.3.1.3 ]

The MS displays to the user the CSGs that are available with the currently registered PLMN using all access technologies which support CSGs and which are supported by the MS. For each entry in the list, an indication is provided whether that CSG identity is in the Allowed CSG list stored in the UE for this PLMN. Additional requirements for the display, including for the display of HNB name, can be found in 3GPP TS 22.011 [9].

The user may select a CSG from the available ones and the MS then camps on a cell with the selected CSG identity. The MS attempts to perform a Location Registration on the currently registered PLMN if the selected CSG Identity is not in the allowed CSG list stored in the MS.

6.3.5.3	Test description

6.3.5.3.1	Pre-test conditions

System Simulator:

- 	Cell 1, Cell 6, Cell 3 and Cell 4.

- 	Cell 1 is not a CSG cell.

- 	CSG 4 (Cell 4), CSG 2 (Cell 6), and CSG 3 (Cell 3)

-	CSG 4 and CSG 3 are in the CSG subscription data for the UE, CSG 2 is not.

-	Cell 3 and Cell 4 have the same tracking areas

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 6 and Cell 3.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1.

-	System information combination 7 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 4.

UE:

-	CSG 2 is in the UE’s Allowed CSG list, while CSG 3 and CSG 4 are not. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Registered, Idle mode (State 2) according to [18] on Cell 1.

6.3.5.3.2	Test procedure sequence

Table 6.3.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0","T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.3.5.3.2-2.

Table 6.3.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 6 | Cell 3 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | Off | Off | Cell 1 serving cell |
|  | Srxlev* | dB | 75 | - | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -71 | Off | Off | -65 |  |
|  | Srxlev* | dB | 69 | - | - | 75 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | Off | -95 |  |
|  | Srxlev* | dB | 75 | - | - | 45 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -71 | -65 | Off | Off |  |
|  | Srxlev* | dB | 69 | 75 | - | - |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -71 | Off | -65 | -95 |  |
|  | Srxlev* | dB | 69 | - | 75 | 45 |  |

Table 6.3.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 4 levels according to row "T1" in table 6.3.5.3.2-1. | - | - | - | - |
| 2 | The UE is made to perform manual CSG ID selection and select CSG 4. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 4? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | P |
| 4 | The SS changes Cell1 and Cell 4 levels according to row "T2" in table 6.3.5.3.2-1. | - | - | - | - |
| 5-10 | Steps 1 to 6 of the procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 11 | The SS changes Cell 1, Cell 6 and Cell 4 levels according to row "T3" in table 6.3.5.3.2-1. | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | The UE transmits an RRCConnectionRequest on Cell 6 within the next 6 minutes [Note] | --> | RRCConnectionRequest | - | - |
| 14 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 15 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 16 | The SS transmits a TRACKING AREA UPDATE REJECT message with cause #25 (not authorized for this CSG) to fail the UE’s TAU on Cell 6. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE REJECT | - | - |
| 17 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 18 | Void | - | - | - | - |
| 18 A-18F | Check: Does the test result of generic test procedure (steps 1 to 6) in TS 36.508 subclause 6.4.2.7indicate that the UE is camped on Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | P |
| 18G | Check: Does the UE send an RRCConnectionRequest on Cell 6 within the next 6 minutes [Note] | --> | RRCConnectionRequest | 2 | F |
| 19 | The SS changes Cell1, Cell 6, Cell 3 and Cell 4 levels according to row "T4" in table 6.3.5.3.2-1. | - | - | - | - |
| 20 | The UE is made to perform manual CSG ID selection and select CSG 3. | - | - | - | - |
| 21-26 | Check: Does the test result of generic test procedure (steps 1 to 6) in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 3 | P |
| 27 | Check: Is CSG 3 added in the UE’s Allowed CSG list? | - | - | 3 | P |
| Note: 6 minutes is allowed for automatic cell reslection to inter-frequency CSG cell [TS 36.133, clause 4.2.2.10.1] |  |  |  |  |  |

6.3.5.3.3	Specific message contents

Table 6.3.5.3.3-1: Conditions for table 6.3.5.3.3-2, 6.3.5.3.3-3 and 6.3.5.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 6 | This condition applies to system information transmitted on Cell 6. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 4 | This condition applies to system information transmitted on Cell 4 |

Table 6.3.5.3.3-2: SystemInformationBlockType1 for Cell 6, 3 and 4 (preamble and all steps except step 11, Table 6.3.5.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell 6 Cell 3 Cell 4 |
| csg-Identity | '000 0000 0000 0000 0000 0000 0010'B |  | Cell 6 |
|  | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 3 |
|  | '000 0000 0000 0000 0000 0000 0100'B |  | Cell 4 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.5.3.3-3: SystemInformationBlockType3 for Cell 4 (preamble and all steps, table 6.3.5.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB20 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.5.3.3-4: SystemInformationBlockType4 for Cell 6, 3 and 4 (preamble and all steps, table 6.3.5.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 0 | Indicates the lowest physical cell identity in the range |  |
| range | n12 | Indicates the number of physical cell identities in the range (including start). |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.5.3.3-5: SystemInformationBlockType9 for Cell 6, 3 and 4 (preamble and all steps except for step 11, table 6.3.5.3.2-2)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType9 ::= SEQUENCE { |  |  |  |
| hnb-Name | “3gppTest_CSG2” |  | Cell 6 |
|  | “3gppTest_CSG3” |  | Cell 3 |
|  | “3gppTest_CSG4” |  | Cell 4 |
| } |  |  |  |

Table 6.3.5.3.3-6: TRACKING AREA UPDATE REJECT (step 16, Table 6.3.5.3.2-2)

| Derivation path: 36.508 table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | ‘00011001’ B | Not authorized for this CSG |  |

Table 6.3.5.3.3-7: TRACKING AREA UPDATE REQUEST (step 18C, Table 6.3.5.3.2-2)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | '000'B | “TA updating” | TA_only |
| EPS update type | ‘010’B | "combined TA/LA updating with IMSI attach" | combined_TA_LA |

### 6.3.6	Ignoring CSG cells in cell selection/reselection when allowed CSG list is empty or not supported

6.3.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and UE does not support Allowed CSG list or UE’s Allowed CSG list is empty }

ensure that {

when { Srxlev < 0 for non-CSG cell and Srxlev > 0 for intra-frequency CSG cell and Srxlev > 0 for inter-frequency CSG cell }

then { UE does not select the intra-freq CSG cell nor the inter-frequency CSG cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state and UE does not support Allowed CSG list or UE’s Allowed CSG list is empty }

ensure that {

when { Srxlev > 0 for non-CSG cell and Srxlev of non-CSG cell > Srxlev of intra-frequency CSG cell and Srxlev of non-CSG cell < Srxlev of inter-frequency CSG cell }

then { UE selects the non-CSG cell }

}

(3)

with { UE in E-UTRA RRC_IDLE state and UE does not support Allowed CSG list or UE’s Allowed CSG list is empty }

ensure that {

when { UE detects an intra-frequency CSG cell with Rn higher than Rs }

then { UE does not reselect to the intra-frequency CSG cell }

}

(4)

with { UE in E-UTRA RRC_IDLE state and UE does not support Allowed CSG list or UE’s Allowed CSG list is empty }

ensure that {

when { UE detects an inter-frequency CSG cell with Rn higher than Rs }

then { UE does not reselect to the inter-frequency CSG cell }

}

6.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.3.2, 5.2.4.6 and TS 36.331, clause B.2.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [3]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause B.2]

In this release of the protocol, it is mandatory for the UE to support a minimum set of CSG functionality consisting of:

-	Identifying whether a cell is CSG or not;

-	Ignoring CSG cells in cell selection/reselection.

6.3.6.3	Test description

6.3.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 3.

-	Cell 1 is not a CSG cell.

-	Cell 2 and Cell 3 are CSG cells.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2 and Cell 3.

UE:

-	If pc_Allowed_CSG_list, The UE’ s allowed CSG list shall not contain CSG IDs 2 and 3. This is realized by using either manual CSG selection or a USIM with field EFACSGL preconfigured.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.3.6.3.2	Test procedure sequence

Table 6.3.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.3.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 |  | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -115 | -97 |  | -85 | SrxlevCell 1 < 0 and SrxlevCell 3 > SrxlevCell 2 > 0. |
|  | Qrxlevmin | dBm | -106 | -106 |  | -106 |  |
|  | Qhyst | dB | 0 | 0 |  | 0 |  |
|  | Srxlev* | dB | -9 | 9 |  | 21 | Cell 3 is the strongest cell |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  | -73 | SrxlevCell 3 > SrxlevCell 1 > SrxlevCell 2 > 0. |
|  | Srxlev* | dB | 21 | 15 |  | 33 | Cell 3 is the strongest cell |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 |  | -79 | -115 | The power level values are assigned to satisfy RCell 1 < RCell 2. |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  | -115 | -73 | The power level values are assigned to satisfy RCell 1 < RCell 3. |

Table 6.3.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 2 or Cell 3 within 60 seconds? | --> | RRCConnectionRequest | 1 | F |
| 3 | The SS changes Cell 1, Cell 2 and Cell 3 levels according to row "T1" in table 6.3.6.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 5-19 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 20 | The SS changes Cell 1, Cell 2 and Cell 3 levels according to row "T2" in table 6.3.6.3.2-1. | - | - | - | - |
| 21 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 60 seconds? | --> | RRCConnectionRequest | 3 | F |
| 22 | The SS changes Cell 1, Cell 2 and Cell 3 levels according to row "T3" in table 6.3.6.3.2-1. | - | - | - | - |
| 23 | Check: Does the UE send an RRCConnectionRequest on Cell 3 within 60 seconds? | --> | RRCConnectionRequest | 4 | F |

6.3.6.3.3	Specific message contents

Table 6.3.6.3.3-1: Conditions for tables 6.3.6.3.3-2 and 6.3.6.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |

Table 6.3.6.3.3-2: SystemInformationBlockType1 for Cell 1, 2 and 3 (Pre-test conditions and all steps, Table 6.3.6.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  | Cell 1 |
|  | TRUE |  | Cell 2 Cell 3 |
| csg-Identity | Not present |  | Cell 1 |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell 2 |
|  | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.6.3.3-3: SystemInformationBlockType3 for Cell 1, 2 and 3 (Pre-test conditions and all steps, table 6.3.6.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.6.3.3-4: SystemInformationBlockType4 for Cell 2 and 3 (Pre-test conditions and all steps, table 6.3.6.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 2 |  | Cell 2 |
|  | 3 |  | Cell 3 |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.7	Inter-RAT Cell reselection from E-UTRA idle non-CSG cell to a UTRA CSG cell

6.3.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable UTRA CSG cell previously visited }

then { UE selects the suitable UTRA CSG cell irrespective of its lower priority than EUTRA cells}

}

6.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.8, 4.3, TS 25.304, clause 5.6.1, 5.2.3.1.2, TS 25.367, clause 6.1, 7.2.3 and TS 36.133, clause 4.2.2.10.2.

[TS 36.304, clause 5.2.4.8]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

If the UE detects one or more suitable CSG cells on another RAT, the UE shall reselect to one of them, if allowed according to [19].

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

- The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

according to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

-	For a CSG cell, the CSG ID is part of the allowed CSG list of the UE.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

[TS 25.304, clause 5.6.1]

In the UE on request of NAS, the AS shall scan all RF channels in the UTRA bands according to its capabilities to find available CSG IDs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) belonging to the registered PLMN together with their “HNB name” (if broadcast) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 25.304, clause 5.2.3.1.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled values QqualminOffset and QrxlevminOffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Squal | Cell Selection quality value (dB) Applicable only for FDD cells. |
| --- | --- |
| Srxlev | Cell Selection RX level value (dB) |
| Qqualmeas | Measured cell quality value. The quality of the received signal expressed in CPICH Ec/N0 (dB) for FDD cells. CPICH Ec/N0 shall be averaged as specified in [10]. Applicable only for FDD cells. |
| Qrxlevmeas | Measured cell RX level value. This is received signal, CPICH RSCP for FDD cells (dBm) and P-CCPCH RSCP for TDD cells (dBm). |
| Qqualmin | Minimum required quality level in the cell (dB). Applicable only for FDD cells. |
| QqualminOffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| QrxlevminOffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(UE_TXPWR_MAX_RACH – P_MAX, 0) (dB) |
| UE_TXPWR_MAX_RACH | Maximum TX power level an UE may use when accessing the cell on RACH (read in system information) (dBm) |
| P_MAX | Maximum RF output power of the UE (dBm) |

[TS 25.367, clause 6.1]

During manual CSG ID selection a UE is allowed to perform Location Registration procedure on a CSG cell whose CSD ID is not in the Permitted CSG list.

Based on the outcome of a Location Registration procedure initiated on a CSG cell, the UE’s Permitted CSG list is updated.

[TS 25.367, clause 7.2.3]

Inter-RAT reselection to an allowed CSG cell is supported when the UE is camped on another RAT. The UE requirements are defined in the specifications of the concerned RAT.

[TS 36.133, clause 4.2.2.10.2]

The UE shall perform search and reselection to an allowed inter-RAT UTRAN FDD CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.10.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.2-1: Parameters for CSG inter-RAT UTRAN FDD reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| EARFCN Note1 |  | Channel 1 | N/A |
| UARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | N/A |
| Primary scrambling code Note1 |  | N/A | Scrambling code 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | N/A |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | -110 |  |
| CPICH_RSCPNote2 | dBm | N/A | -100 |
| CPICH_Ec/Ior | dB |  | -10 |
| PCCPCH_Ec/Ior | dB |  | -12 |
| SCCPCH_Ec/Ior | dB |  | -12 |
| AICH_Ec/Ior | dB |  | -15 |
| SCH_Ec/Ior | dB |  | -15 |
| PICH_Ec/Ior | dB |  | -15 |
|  | dBm/3.84 MHz |  | Off |
| Note 1: For this requirement to be applicable, the EARFCN and physical cell identity for cell 1 and the UARFCN and scrambling code for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

6.3.7.3	Test description

6.3.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN cells

-	Cell 5 and Cell 7 are UTRAN cells

-	Cell 5 is a UTRA CSG cell

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE’ s allowed CSG lst shall not contain CSG ID 2.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) according to [18] in Cell 1.

6.3.7.3.2	Test procedure sequence

Tables 6.3.7.3.2-1 & 6.3.7.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1","T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.3.7.3.2-1: Time instances of cell power level and parameter changes for E-UTRA Cell 1 and Cell 2

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Qrxlevmin | dBm | -106 | -106 | Default value |
|  | Qrxlevminoffset | dB | 0 | 0 | Default value |
|  | Qhyst | dB | 0 | 0 | Default value |
|  | Srxlev* | dB | 21 | N/A | Cell 1 is the strongest cell |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | OFF | -85 |  |
|  | Srxlev* | dB | N/A | 21 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
|  | Srxlev* | dB | 21 | 15 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.7.3.2-2: Time instances of cell power level and parameter changes for UTRA Cell 5 and Cell 7

|  | Parameter | Unit | Cell 5 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | Qrxlevmin (FDD) | dBm | -79 | -79 | Default value |
|  | Qrxlevmin (TDD) | dBm | -81 | -81 | Default value |
|  | Srxlev* | dB | N/A | N/A |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 | -66 | SrxlevCell 5 > 0, for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -68 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 13 | Cell 5 becomes the suitable cell |
| T2 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF |  |
|  | Srxlev* | dB | N/A | N/A |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | OFF | -66 |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | -68 |  |
|  | Srxlev* | dB | N/A | 13 |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | -60 | -66 | SrxlevCell 5 > 0,for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -68 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 13 | Cell 5 becomes the suitable cell |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.3.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 5 and Cell 7 levels according to the row "T1" in table 6.3.7.3.2-2. | - | - | - | - |
| 2 | UE performs manual CSG ID selection. CSG Identity ('000 0000 0000 0000 0000 0000 0010'B) is selected manually and added in UE’s Allowed CSG list. | - | - | - | - |
| 3-12 | Steps 1 to 10 of the generic test procedure described in TS 36.508 subclause 6.4.2.8 are performed on UTRA CSG Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 12Aa1 to 12Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 12Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 12Aa1a1-12Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 12Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 12Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 12Aa1b1 | The 6s timer expires | - | - | - | - |
| 13 | The SS changes Cell 1, Cell 2, Cell 5 and Cell 7 levels according to the row "T2" in table 6.3.7.3.2-1 and table 6.3.7.3.2-2. | - | - | - | - |
| 14-21 | Steps 1 to 8 of the procedure described in TS 36.508 Table 6.4.2.7A-1 is performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22 | The SS changes Cell 1, Cell 2 and Cell 7 levels according to the row "T3" in table 6.3.7.3.2-1 and table 6.3.7.3.2-2. | - | - | - | - |
| 23-28 | Steps 1 to 6 of the procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 28AAa1 to 28AAa2b1 describe optional behaviour if UE sends IMS deregistration at steps 12Aa1 to 12Aa1b1 | - | - | - | - |
| 28AAa1-28AAa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 28AB | The SS releases RRC Connection |  |  |  |  |
| 28A | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 29 | The SS changes Cell 2 and Cell 5 levels according to the row "T4" in table 6.3.7.3.2-1 and table 6.3.7.3.2-2. | - | - | - | - |
| 30-37 | Check: Does the test result of steps 1 to 8 of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN CSG Cell 5 in 6 mins from "T4" to the time RRC CONNECTION REQUEST received by SS? NOTE: The UE performs a RAU procedure | - | - | 1 | - |

6.3.7.3.3	Specific message contents

Table 6.3.7.3.3-1: Void

Table 6.3.7.3.3-2:  Void

Table 6.3.7.3.3-3: SystemInformationBlockType6 for Cell 1 and Cell 2 (preamble and all steps, Table 6.3.7.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.7.3.3-4: Master Information Block for Cell 5 (preamble and all steps, Table 6.3.7.3.2-3)

| Derivation Path: 34.108 clause 6.1.0a.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Indicator | TRUE |  |  |

Table 6.3.7.3.3-5: System Information Block type 3 for Cell 5 (preamble and all steps, Table 6.3.7.3.2-3)

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Identity | '000 0000 0000 0000 0000 0000 0010'B |  |  |
| - CSG PSC Split Information |  |  |  |
| - Start PSC | 104 |  |  |
| - Number of PSCs | 5 |  |  |
| - PSC Range 2 Offset | Not present |  |  |

Table 6.3.7.3.3-6: System Information Block type 19 for Cell 5 and Cell 7 (preamble and all steps, Table 6.3.7.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.8	Void

### 6.3.9	Manual CSG ID selection across PLMNs

6.3.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { a CSG cell not belonging to the Registered PLMN is manually selected }

then { UE selects the requested CSG cell }

}

(2)

with { UE having selected a CSG cell not belonging to the initial Registered PLMN }

ensure that {

when { the UE is no longer within coverage of the CSG cell }

then { UE returns to the stored duplicate PLMN selection mode and selects a cell on the initial Registered PLMN }

}

6.3.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.3.3.

[TS 23.122, clause 4.4.3.1.3.3]

If the user selects a CSG in a PLMN that is different from the RPLMN, then the following applies:

i)	The MS shall store a duplicate of the RPLMN and a duplicate of the current PLMN selection mode;

ii)	The MS shall enter into Manual mode of PLMN selection in state M4 (Trying PLMN) as defined in subclause 4.3.1.2;

iii)	The MS shall select the PLMN corresponding to the CSG and attempt to register on the selected CSG cell in the PLMN; and

iv)	If the registration fails or the MS is no longer in the coverage of the CSG, then the MS shall return to the stored duplicate PLMN selection mode and use the stored duplicate value of RPLMN for further action.

6.3.9.3	Test description

6.3.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 12.

-	Cell 1 and Cell 12 are not CSG cells.

-	Cell 3 is a CSG cell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 12.

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 3.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.9.1.3.1-1.

Table 6.3.9.1.3.1–1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 3 | PLMN2 |
| 12 | PLMN2 |

UE:

-	The UE’ s allowed CSG list shall not contain CSG ID 3

-	The UE’s Operator CSG list is empty.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.3.9.3.2	Test procedure sequence

Table 6.3.9.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.3.9.3.2-2.

Table 6.3.9.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | Off | The power levels are such that camping on Cell 1 is guaranteed. |
|  | Srxlev* | dB | 41 | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -65 | -85 | Off |  |
|  | Srxlev* | dB | 41 | 21 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | -65 |  |
|  | Srxlev* | dB | 21 | - | 41 |  |

Table 6.3.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-17 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: the UE performs registration and the RRC connection is released. | - | - | - | - |
| 18 | The SS changes Cell 3 level according to row "T1" in table 6.3.9.3.2-1. | - | - | - | - |
| 19 | The UE is made to perform manual CSG ID selection and select Cell 3. | - | - | - | - |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 21 | The SS changes Cell 1, Cell 3 and Cell 12 levels according to row "T2" in table 6.3.9.3.2-1. | - | - | - | - |
| 22-26 | Check: Does the test result of Steps 1 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 1? NOTE: The UE performs a TAU procedure. | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

6.3.9.3.3	Specific message contents

Table 6.3.9.3.3-1: Conditions for table 6.3.9.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 12 | This condition applies to system information transmitted on Cell 12. |

Table 6.3.9.3.3-2: SystemInformationBlockType1 for Cell 1, 3 and 12 (preamble and all steps, Table 6.3.9.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  | Cell 1 Cell 12 |
|  | TRUE |  | Cell 3 |
| csg-Identity | Not present |  | Cell 1 Cell 12 |
|  | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.9.3.3-3: SystemInformationBlockType4 for Cell 3 (Pre-test conditions and all steps, table 6.3.9.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 3 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

### 6.3.10	Void

### 6.3.11	Void

### 6.3.12	Void

## 6.4	Hybrid cells

### 6.4.1	Manual CSG ID selection / Hybrid cell whose CSG ID is not in the Allowed CSG list nor Operator’s list

6.4.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { a hybrid cell whose CSG ID is not included in the Allowed CSG list nor Operator CSG List is manually selected }

then { UE selects the requested hybrid cell }

}

(2)

with { UE camping on a non-CSG cell in which the UE previously selected a hybrid cell manually whose CSG ID is not included in the Allowed CSG list nor Operator CSG List }

ensure that {
  when { the hybrid cell the UE previously selected is activated }

then { UE does not reselect the hybrid cell against normal cell reselection criteria }

}

6.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.3.1 and 4.4.3.1.3.2, TS 23.401, clause 5.3.3.2, TS 36.133, clause 4.2.2.10.1 and TS 36.304, clause 5.4.2.9.

[TS 23.122, clause 4.4.3.1.3.1]

The HPLMN may configure the MS whether to provide to the user CSGs for a certain PLMN without any restriction or to provide to the user only CSGs in the Operator CSG List for that PLMN. This configuration may be done either:

-	 in the USIM if the Operator CSG list is available in the USIM; or

-	as described in 3GPP TS 24.285 [47].

In the absence of  such configuration from the HPLMN for a certain PLMN, the MS shall by default provide to the user CSGs for that PLMN without any restriction.

For PLMNs where no restriction is configured, the MS provides to the user the CSGs that are available and the associated PLMNs using all access technologies which support CSGs (see 3GPP TS 23.003 [22A]) and which are supported by the MS. For each entry in the list, an indication is provided whether that CSG identity is in the Allowed CSG list or in the Operator CSG List stored in the MS for this PLMN.

For PLMNs where the MS is configured to provide to the user only CSGs in the Operator CSG List, the MS provides to the user the CSGs that are available and in the Operator CSG list, using all access technologies which support CSGs (see 3GPP TS 23.003 [22A]) and which are supported by the MS. For each entry in the list, the MS provides to the user the associated PLMN and an indication that the CSG identity is in the Operator CSG List stored in the MS for this PLMN.

Additional requirements for the display, including for the display of HNB name, can be found in 3GPP TS 22.220 [49].

The user may select a CSG from the indicated CSGs.

If the MS has a PDN connection for emergency bearer services, manual CSG selection shall not be performed.

[TS 23.122, clause 4.4.3.1.3.2]

If the user selects a CSG whose CSG identity is not included in the Allowed CSG list or Operator CSG list, then the MS shall attempt to register on a cell that corresponds to the CSG.

[TS 23.401, clause 5.3.3.2]

NOTE 7:	If the UE receives a TAU Accept message via a hybrid cell, the UE does not add the corresponding CSG ID to its Allowed CSG list. Adding a CSG ID to the UE's local Allowed CSG list for a hybrid cell is performed only by OTA or OMA DM procedures.

[TS 36.133, clause 4.2.2.10.1]

The UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.10.1-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.1-1: Parameters for CSG inter-frequency reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UARFCN Note1 |  | Channel 1 | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 | -140 |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | [≥TBD] | [≥TBD] |
| Note 1: For this requirement to be applicable, the E-UARFCN and physical cell identity for cell 1 and cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE. |  |  |  |

[TS 36.304, clause 5.2.4.9]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

6.4.1.3	Test description

6.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 are E-UTRAN cells and have different tracking areas according to table 6.0.1-2.

-	Cell 1 is a non-CSG cell.

-	Cell 3 is a hybrid cell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 3.

UE:

-	The UE’s allowed CSG list shall not contain CSG ID 3. This is realized by using manual CSG selection.

-	the UE’s Operator CSG list is empty. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.4.1.3.2	Test procedure sequence

Table 6.4.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.4.1.3.2-2.

Table 6.4.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -97 | -85 | The power level values are assigned so that the UE can detect Cell 3. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | The power level values are assigned so that camping on Cell 1 is guaranteed. |

Table 6.4.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 3 levels according to row "T1" in Table 6.4.1.3.2-1. | - | - | - | - |
| 2 | The UE is made to perform manual CSG ID selection and select Cell 3. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 4 | The SS changes Cell 1 and Cell 3 levels according to row "T2" in Table 6.4.1.3.2-1. | - | - | - | - |
| 5 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 6 | The SS changes Cell 1 and Cell 3 levels according to row "T1" in Table 6.4.1.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 3 within 6 minutes? | --> | RRCConnectionRequest | 2 | F |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 2 | - |

6.4.1.3.3	Specific message contents

Table 6.4.1.3.3-0: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.4.1.3.3-1: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | 3 | lower priority than the frequency for Cell 1 |  |
| } |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | EARFCN of Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.1.3.3-2: SystemInformationBlockType1 for Cell 3 (preamble and all steps, Table 6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Identity | '000 0000 0000 0000 0000 0000 0011'B |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.1.3.3-3: SystemInformationBlockType3 for Cell 3 (preamble and all steps, Table 6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 | lower priority than the frequency for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.1.3.3-4: SystemInformationBlockType5 for Cell 3 (preamble and all steps, Table 6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | EARFCN of Cell 1 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| threshX-High[n] | 10 | 20 dB |  |
| } |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | EARFCN of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.1.3.3-5: SystemInformationBlockType4 for Cell 3 (preamble and all steps, Table 6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange SEQUENCE { |  |  |  |
| start | 4 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

### 6.4.2	Inter-frequency Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE member hybrid cell

6.4.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { Manual CSG ID selection is requested }

then { UE selects the requested hybrid cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state on a non-CSG cell and UE’s Allowed CSG list is not empty }

ensure that {

when { a previously visited E-UTRA member hybrid cell becomes the strongest cell on its frequency }

then { UE reselects the E-UTRA member hybrid cell }

}

6.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clauses 5.2.4.9 and 5.2.4.8.1, and 36.133, clauses 4.2.2.10 and 4.2.2.10.1.

[TS 36.304, clause 5.2.4.9]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

[TS 36.133, clause 4.2.2.10]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list. The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

[TS 36.133, clause 4.2.2.10.1]

The UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met reselection criterion defined in [1] and that is in its allow-list, within [6] minutes in the conditions shown in table 4.2.2.10.1-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.1-1: Parameters for CSG inter-frequency reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UARFCN Note1 |  | Channel 1 | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | 2 |
| CSG identity |  | Not sent | Already stored in UE allow-list from previous visit |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 | -140 |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | [≥TBD] | [≥TBD] |
| Note 1: For this requirement to be applicable, the E-UARFCN and physical cell identity for cell 1 and cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

6.4.2.3	Test description

6.4.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3, Cell 11 and Cell 23.

-	Cell 1, Cell 11 and Cell 23 are not CSG cells.

-	Cell 3 is a hybrid cell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 11 and Cell 23.

-	System information combination 13 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 3.

-	Cell 1, Cell 3, Cell 11 and Cell 23 have different tracking areas according to table 6.0.1-2.

UE:

-	The UE’s Allowed CSG list contains the CSG ID of Cell 3. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.4.2.3.2	Test procedure sequence

Table 6.4.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.4.2.3.2-2.

Table 6.4.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 11 | Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | Off | Off | The power levels are such that camping on Cell 1 is guaranteed. |
|  | Srxlev* | dB | 41 | - | - | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -65 | -85 | Off | -91 |  |
|  | Srxlev* | dB | 41 | 21 | - | 15 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Off | Off | -65 | Off | The power levels are such that camping on Cell 11 is guaranteed. |
|  | Srxlev* | dB | - | - | 41 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -65 | Off | -91 | -85 |  |
|  | Srxlev* | dB | 41 | - | 15 | 21 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -65 | -85 | Off | -91 |  |
|  | Srxlev* | dB | 41 | 21 | - | 15 |  |

Table 6.4.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-17 | Steps 2 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: the UE performs registration and the RRC connection is released. | - | - | - | - |
| 18 | The SS changes Cell 3 and Cell 23 levels according to row "T1" in table 6.4.2.3.2-1. | - | - | - | - |
| 19 | The UE is made to perform manual CSG ID selection and select Cell 3. | - | - | - | - |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 21 | The SS changes Cell1, Cell 3, Cell 11 and Cell 23 levels according to row "T2" in table 6.4.2.3.2-1. | - | - | - | - |
| 22-27 | Steps 1 to 6 of the TAU procedure described in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: the UE performs a TAU and the RRC connection is released. | - | - | - | - |
| 28 | The SS changes Cell 1, Cell 11 and Cell 23 levels according to row "T3" in table 6.4.2.3.2-1. | - | - | - | - |
| 29-34 | Steps 1 to 6 of the TAU procedure described in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: the UE performs a TAU and the RRC connection is released. | - | - | - | - |
| 35 | The SS changes Cell 3, Cell 11 and Cell 23 levels according to row "T4" in table 6.4.2.3.2-1. | - | - | - | - |
| 36-40 | Check: Does the test result of Steps 1 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on Cell 3 within [6] min from T4 to the time the RRCConnectionRequest is received by the SS? NOTE: The UE performs a TAU procedure. | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

6.4.2.3.3	Specific message contents

Table 6.4.2.3.3-1: Conditions for table 6.4.2.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Cell 11 | This condition applies to system information transmitted on Cell 11. |
| Cell 23 | This condition applies to system information transmitted on Cell 23. |

Table 6.4.2.3.3-2: SystemInformationBlockType1 for Cell 1, 3, 11 and 23 (preamble and all steps, Table 6.4.2.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  | Cell 1 Cell 11 Cell 23 |
|  | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |

Table 6.4.2.3.3-3: SystemInformationBlockType4 for Cell 3 (Pre-test conditions and all steps, table 6.4.2.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 3 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

### 6.4.3	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to UTRA_Idle member hybrid cell

6.4.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state on a non-CSG cell and UE’s Allowed CSG list is not empty }

ensure that {

when { a previously visited UTRA member hybrid cell becomes the strongest cell on its frequency }

then { UE reselects the suitable UTRA member hybrid cell irrespective of its lower priority than EUTRA cell }

}

6.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.6.1; TS 36.304, clauses 5.2.4.8.1 and 5.2.4.9; and TS 36.133, clauses 4.2.2.10 and 4.2.2.10.2.

[TS 25.304, clause 5.6.1]

In the UE on request of NAS, the AS shall scan all RF channels in the UTRA bands according to its capabilities to find available CSG IDs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) together with their “HNB name” (if broadcast) and PLMN(s) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

[TS 36.304, clause 5.2.4.9]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 36.133, clause 4.2.2.10]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list.allow-list The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

[TS 36.133, clause 4.2.2.10.2]

The UE shall perform search and reselection to an allowed inter-RAT UTRAN FDD CSG cell that has met CSG reselection criterion defined in [1] and that is in its , within 6 minutes in the conditions shown in table 4.2.2.10.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.2-1: Parameters for CSG inter-RAT UTRAN FDD reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UARFCN Note1 |  | Channel 1 | N/A |
| UARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | N/A |
| Primary scrambling code Note1 |  | N/A | Scrambling code 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | N/A |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | [≥TBD] |  |
| CPICH_EcNote2 | dBm | N/A | [≥TBD] |
| CPICH_Ec/Ior | dB |  | -10 |
| PCCPCH_Ec/Ior | dB |  | -12 |
| SCCPCH_Ec/Ior | dB |  | -12 |
| AICH_Ec/Ior | dB |  | -15 |
| SCH_Ec/Ior | dB |  | -15 |
| PICH_Ec/Ior | dB |  | -15 |
|  | dBm/3.84 MHz |  | Off |
| Note 1: For this requirement to be applicable, the E-UARFCN and physical cell identity for cell 1 and the UARFCN and scrambling code for cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by the UE. |  |  |  |

6.4.3.3	Test description

6.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN cells and have different tracking areas according to table 6.0.1-2.

-	Cell 5 and Cell 7 are UTRAN cells

-	Cell 1, Cell 2 and Cell 7 are not CSG cells.

-	Cell 5 is a hybrid cell.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- The UE’s Allowed CSG list contains the CSG ID of Cell 5. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) according to [18] in Cell 1.

6.4.3.3.2	Test procedure sequence

Table 6.4.3.3.2-1 & 6.4.3.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.4.3.3.2-3.

Table 6.4.3.3.2-1: Time instances of cell power level and parameter changes for E-UTRA Cell 1 and Cell 2

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Qrxlevmin | dBm | -106 | -106 | Default value |
|  | Qrxlevminoffset | dB | 0 | 0 | Default value |
|  | Qhyst | dB | 0 | 0 | Default value |
|  | Srxlev* | dB | 21 | N/A | Cell 1 is the strongest cell |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | OFF | -85 |  |
|  | Srxlev* | dB | N/A | 21 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
|  | Srxlev* | dB | 21 | 15 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.3.3.2-2: Time instances of cell power level and parameter changes for UTRA Cell 5 and Cell 7

|  | Parameter | Unit | Cell 5 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | Qrxlevmin (FDD) | dBm | -79 | -79 | Default value |
|  | Qrxlevmin (TDD) | dBm | -81 | -81 | Default value |
|  | Srxlev* | dB | N/A | N/A |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 | -64 | SrxlevCell 5 > 0, for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -66 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 15 | Cell 5 becomes the suitable cell |
| T2 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF |  |
|  | Srxlev* | dB | N/A | N/A |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | OFF | -64 |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | -66 |  |
|  | Srxlev* | dB | N/A | 15 |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | -60 | -64 | SrxlevCell 5 > 0,for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -66 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 15 | Cell 5 becomes the suitable cell |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 5 and Cell 7 levels according to the row "T1" in table 6.4.3.3.2-2. | - | - | - | - |
| 2 | UE performs manual CSG ID selection and select Cell 5. | - | - | - | - |
| 3-12 | Steps 1 to 10 of the generic test procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 12Aa1 to 12Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 12Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 12Aa1a1-12Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 12Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 12Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 12Aa1b1 | The 6s timer expires | - | - | - | - |
| 13 | The SS changes Cell 1, Cell 2, Cell 5 and Cell 7 levels according to the row "T2" in table 6.4.3.3.2-1 and table 6.4.3.3.2-2. | - | - | - | - |
| 14-20 | Steps 1 to 8 of the procedure described in TS 36.508 Table 6.4.2.7A-1 is performed on Cell 2. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 21a1 to 21a2b1 describe optional behaviour if UE sends IMS deregistration at steps 12Aa1 to 12Aa1b1 | - | - | - | - |
| 21a1-21a2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 21A | The SS releases RRC Connection |  |  |  |  |
| 22 | The SS changes Cell 1, Cell 2 and Cell 7 levels according to the row "T3" in table 6.4.3.3.2-1 and table 6.4.3.3.2-2. | - | - | - | - |
| 23-28 | Steps 1 to 6 of the procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 29 | The SS changes Cell 2 and Cell 5 levels according to the row "T4" in table 6.4.3.3.2-1 and table 6.4.3.3.2-2. | - | - | - | - |
| 30-37 | Check: Does the test result of steps 1 to 8 of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on Cell 5 within 6 mins from "T4" to the time RRC CONNECTION REQUEST received by SS? NOTE: The UE performs a RAU procedure | - | - | 1 | - |

6.4.3.3.3	Specific message contents

Table 6.4.3.3.3-1: SystemInformationBlockType6 for Cell 1 and Cell 2 (preamble and all steps, Table 6.4.3.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.4.3.3.3-2: Master Information Block for Cell 5 (preamble and all steps, Table 6.4.3.3.2-3)

| Derivation Path: 34.108 clause 6.1.0a.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Indicator | Not present |  |  |

Table 6.4.3.3.3-3: System Information Block type 3 for Cell 5 (preamble and all steps, Table 6.4.3.3.2-3)

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Identity | '000 0000 0000 0000 0000 0000 0010'B |  |  |
| - CSG PSC Split Information |  |  |  |
| - Start PSC | 104 |  |  |
| - Number of PSCs | 5 |  |  |
| - PSC Range 2 Offset | Not present |  |  |

Table 6.4.3.3.3-4: System Information Block type 19 for Cell 5 and Cell 7 (preamble and all steps, Table 6.4.3.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| Utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.4.4	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE non-member hybrid cell to UTRA_Idle member hybrid cell

6.4.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state on a non-member hybrid cell and UE’s Allowed CSG list is not empty }

ensure that {

when { a previously visited UTRA member hybrid cell becomes the strongest cell on its frequency }

then { UE reselects the suitable UTRA member hybrid cell irrespective of its lower priority than EUTRA cell }

}

6.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.6.1; TS 36.304, clauses 5.2.4.8.1 and 5.2.4.9; and TS 36.133, clauses 4.2.2.10 and 4.2.2.10.2.

[TS 25.304, clause 5.6.1]

In the UE on request of NAS, the AS shall scan all RF channels in the UTRA bands according to its capabilities to find available CSG IDs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) together with their “HNB name” (if broadcast) and PLMN(s) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

[TS 36.304, clause 5.2.4.9]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 36.133, clause 4.2.2.10]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list.  The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

[TS 36.133, clause 4.2.2.10.2]

The UE shall perform search and reselection to an allowed inter-RAT UTRAN FDD CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.10.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.10.2-1: Parameters for CSG inter-RAT UTRAN FDD reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UARFCN Note1 |  | Channel 1 | N/A |
| UARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Physical cell identityNote1 |  | 1 | N/A |
| Primary scrambling code Note1 |  | N/A | Scrambling code 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| PBCH_RA | dB | 0 | N/A |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | Off |  |
| RSRP Note2 | dBm/15 KHz | [≥TBD] |  |
| CPICH_EcNote2 | dBm | N/A | [≥TBD] |
| CPICH_Ec/Ior | dB |  | -10 |
| PCCPCH_Ec/Ior | dB |  | -12 |
| SCCPCH_Ec/Ior | dB |  | -12 |
| AICH_Ec/Ior | dB |  | -15 |
| SCH_Ec/Ior | dB |  | -15 |
| PICH_Ec/Ior | dB |  | -15 |
|  | dBm/3.84 MHz |  | Off |
| Note 1: For this requirement to be applicable, the E-UARFCN and physical cell identity for cell 1 and the UARFCN and scrambling code for cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by the UE. |  |  |  |

6.4.4.3	Test description

6.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN cells and have different tracking areas according to table 6.0.1-2.

-	Cell 5 and Cell 7 are UTRAN cells

-	Cell 1 and Cell 7 are not CSG cells.

-	Cell 2 and Cell 5 are hybrid cells.

-	System information combination 11 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

- The UE’s Allowed CSG list contains the CSG ID of Cell 5. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) according to [18] in Cell 1.

6.4.4.3.2	Test procedure sequence

Table 6.4.4.3.2-1 & 6.4.4.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1", "T2", "T3", "T4" and "T5" are applied at the points indicated in the Main behaviour description in Table 6.4.4.3.2-3.

Table 6.4.4.3.2-1: Time instances of cell power level and parameter changes for E-UTRA Cell 1 and Cell 2

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Qrxlevmin | dBm | -106 | -106 | Default value |
|  | Qrxlevminoffset | dB | 0 | 0 | Default value |
|  | Qhyst | dB | 0 | 0 | Default value |
|  | Srxlev* | dB | 21 | N/A | Cell 1 is the strongest cell |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | OFF | -85 |  |
|  | Srxlev* | dB | N/A | 21 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 |  |
| T5 | Cell-specific RS EPRE | dBm/15kHz | OFF | -85 |  |
|  | Srxlev* | dB | N/A | 21 |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.4.3.2-2: Time instances of cell power level and parameter changes for UTRA Cell 5 and Cell 7

|  | Parameter | Unit | Cell 5 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF | Camping on Cell 1 is guaranteed |
|  | Qrxlevmin (FDD) | dBm | -79 | -79 | Default value |
|  | Qrxlevmin (TDD) | dBm | -81 | -81 | Default value |
|  | Srxlev* | dB | N/A | N/A |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF |  |
|  | Srxlev* | dB | N/A | N/A |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 | -64 | SrxlevCell 5 > 0, for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -66 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 15 | Cell 5 becomes the suitable cell |
| T3 | CPICH_Ec | dBm/3.84 MHz | OFF | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | OFF |  |
|  | Srxlev* | dB | N/A | N/A |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | OFF | -64 |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | -66 |  |
|  | Srxlev* | dB | N/A | 15 |  |
| T5 | CPICH_Ec | dBm/3.84 MHz | -60 | -64 | SrxlevCell 5 > 0,for FDD |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -66 | SrxlevCell 5 > 0, for TDD |
|  | Srxlev* | dB | 19 | 15 | Cell 5 becomes the suitable cell |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.4.4.3.2-1. | - | - | - | - |
| 2-8 | Steps 1 to 6 of the procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 9 | The SS changes Cell 1, Cell 5 and Cell 7 levels according to the row "T2" in table 6.4.4.3.2-1 and table 6.4.4.3.2-2. | - | - | - | - |
| 10 | UE performs manual CSG ID selection and select Cell 5. | - | - | - | - |
| - | Exception: Steps 20Aa1 to 20Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 20Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 20Aa1a1-20Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 20Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 20Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 20Aa1b1 | The 6s timer expires | - | - | - | - |
| 11-20 | Steps 1 to 10 of the generic test procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| 21 | The SS changes Cell 1, Cell 2, Cell 5 and Cell 7 levels according to the row "T3" in table 6.4.4.3.2-1 and table 6.4.4.3.2-2. | - | - | - | - |
| 22-28 | Steps 1 to 7 of the procedure described in TS 36.508 Table 6.4.2.7A-1 is performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 29a1 to 29a2b1 describe optional behaviour if UE sends IMS deregistration at steps 20Aa1 to 20Aa1b1 | - | - | - | - |
| 29a1-29a2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 29A | The SS releases RRC Connection |  |  |  |  |
| 30 | The SS changes Cell 1, Cell 2 and Cell 7 levels according to the row "T4" in table 6.4.4.3.2-1 and table 6.4.4.3.2-2. | - | - | - | - |
| 31-36 | Steps 1 to 6 of the procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 37 | The SS changes Cell 1 and Cell 5 levels according to the row "T5" in table 6.4.4.3.2-2. | - | - | - | - |
| 38-45 | Check: Does the test result of steps 1 to 8 of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on Cell 5 within 6 mins from "T5" to the time RRC CONNECTION REQUEST received by SS? NOTE: The UE performs a RAU procedure | - | - | 1 | - |

6.4.4.3.3	Specific message contents

Table 6.4.4.3.3-1: Conditions for table 6.4.4.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |

Table 6.4.4.3.3-2: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.4.4.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  | Cell 1 |
| csg-Identity | '000 0000 0000 0000 0000 0000 0011'B |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |

Table 6.4.4.3.3-3: SystemInformationBlockType4 for Cell 2 (preamble and all steps, table 6.4.4.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 4 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.4.3.3-4: SystemInformationBlockType6 for Cell 1 and Cell 2 (preamble and all steps, Table 6.4.4.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.4.4.3.3-5: Master Information Block for Cell 5 (preamble and all steps, Table 6.4.4.3.2-3)

| Derivation Path: 34.108 clause 6.1.0a.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Indicator | Not present |  |  |

Table 6.4.4.3.3-6: System Information Block type 3 for Cell 5 (preamble and all steps, Table 6.4.4.3.2-3)

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - CSG Identity | '000 0000 0000 0000 0000 0000 0010'B |  |  |
| - CSG PSC Split Information |  |  |  |
| - Start PSC | 104 |  |  |
| - Number of PSCs | 5 |  |  |
| - PSC Range 2 Offset | Not present |  |  |

Table 6.4.4.3.3-7: System Information Block type 19 for Cell 5 and Cell 7 (preamble and all steps, Table 6.4.4.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.4.5	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE member hybrid cell

6.4.5.1	Test Purpose (TP)

(1)

with { UE in UTRA IDLE state and UE’s Allowed CSG list is not empty }

ensure that {

when { a previously visited E-UTRA member hybrid cell becomes the strongest cell on its frequency }

then { UE reselects the suitable E-UTRA member hybrid cell irrespective of its lower priority than UTRA cell }

}

6.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clauses 5.2.6.4.1 and 5.2.6.5; TS 25.133, clauses 4.2.2.11 and 4.2.2.11.2; TS 36.304, clause 5.5.1.

[TS 25.304, clause 5.2.6.4.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE is required to perform autonomous search function in Idle, Cell_PCH and URA_PCH states. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty. If "Dedicated CSG frequency(ies) " IE is present, the UE may use the autonomous search function only on these dedicated frequencies and on the other frequencies listed in the system information.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

When the UE has no or an empty Permitted CSG listUE’s Permitted CSG list, the UE may ignore cells with PSC in the stored range "CSG PSC Split Information" [4] reserved for CSG cells for intra-frequency and inter-frequency measurements and cell re-selections.

If the UE detects a suitable CSG cell on the same frequency it shall reselect this cell if the concerned CSG cell is the highest ranked cell, according to 5.2.6.1.4.

If the UE detects a suitable CSG cell on a different frequency it shall reselect this cell irrespective of the cell reselection rules applicable for the cell the UE is currently camped on, if the detected suitable CSG cell is the strongest cell on that frequency. If suitable CSG cells are detected on different frequencies and these are the strongest cells on their frequencies, then the UE shall reselect to any one of them.

If the UE detects one or more suitable CSG cell on another RAT, the UE shall reselect to one of them according to [18].

[TS 25.304, clause 5.2.6.5]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the  according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 36.304, clause 5.5.1]

In the UE on request of NAS, the AS shall scan all RF channels in the E-UTRA bands according to its capabilities to find available CSG IDs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CSG ID(s) together with their “HNB name” (if broadcast) and PLMN(s) to the NAS. The search for available CSG IDs may be stopped on request of the NAS.

If NAS has selected a CSG ID and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CSG ID to camp on.

[TS 25.133, clause 4.2.2.11]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list.  The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

[TS 25.133, clause 4.2.2.11.2]

The UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.11.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.2-1: Parameters for CSG inter-RAT E-UTRA reselection

| - Parameter | - Unit | - Cell 1 | - Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| E-UARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_EcNote2 | dBm | [≥TBD] | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | [≥TBD] |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the E-UARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by the UE. |  |  |  |

6.4.5.3	Test description

6.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN cells.

-	Cell 5 and Cell 7 are UTRAN cells and have different routing areas.

-	Cell 1, Cell 5 and Cell 7 are not CSG cells.

-	Cell 2 is a hybrid cell.

-	System information combination 11 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 2.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

- The UE’s Allowed CSG list contains the CSG ID of Cell 2. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5.

6.4.5.3.2	Test procedure sequence

Table 6.4.5.3.2-1 & 6.4.5.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T0", "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.4.5.3.2-3.

Table 6.4.5.3.2-1: Time instances of cell power level and parameter changes for E-UTRA Cell 1 and Cell 2

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -OFF | OFF |  |
|  | Qrxlevmin | dBm | -106 | -106 | Default value |
|  | Qrxlevminoffset | dB | 0 | 0 | Default value |
|  | Qhyst | dB | 0 | 0 | Default value |
|  | Srxlev* | dB | N/A | N/A |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | SrxlevCell 2 > 0 |
|  | Srxlev* | dB | 21 | 27 | Cell 2 becomes the suitable cell |
| T2 | Cell-specific RS EPRE | dBm/15kHz | OFF | OFF |  |
|  | Srxlev* | dB | N/A | N/A |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | OFF |  |
|  | Srxlev* | dB | 21 | N/A |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | SrxlevCell 2 > 0 |
|  | Srxlev* | dB | 21 | 27 | Cell 2 becomes the suitable cell |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.5.3.2-2: Time instances of cell power level and parameter changes for UTRA Cell 5 and Cell 7

|  | Parameter | Unit | Cell 5 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 | OFF | Camping on Cell 5 is guaranteed |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | OFF | Camping on Cell 5 is guaranteed |
|  | Qrxlevmin (FDD) | dBm | -79 | -79 | Default value |
|  | Qrxlevmin (TDD) | dBm | -81 | -81 | Default value |
|  | Srxlev* | dB | 19 | N/A | Cell 5 is the strongest cell |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | OFF |  |
|  | Srxlev* | dB | 19 | N/A |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | OFF | -60 |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | OFF | -62 |  |
|  | Srxlev* | dB | N/A | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -60 | -64 |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | -66 |  |
|  | Srxlev* | dB | 19 | 15 |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | -60 | OFF |  |
|  | PCCPCH RSCP | dBm/1.28 MHz | -62 | OFF |  |
|  | Srxlev* | dB | 19 | N/A |  |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.4.5.3.2-1. | - | - | - | - |
| 2 | UE performs manual CSG ID selection and select Cell 2. | - | - | - | - |
| 3-24 | Steps 1 to 22 of the generic test procedure described in TS 36.508 Table 6.4.2.7A-2 are performed on Cell 2. | - | - | - | - |
| 25 | The SS changes Cell 1, Cell 2, Cell 5 and Cell 7 levels according to the row "T2" in table 6.4.5.3.2-1 and table 6.4.5.3.2-2. | - | - | - | - |
| 26-35 | Steps 1 to 10 of the procedure described in TS 36.508 subclause 6.4.2.8 is performed on Cell 7. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| 36 | The SS changes Cell 1, Cell 5 and Cell 7 levels according to the row "T3" in table 6.4.5.3.2-1 and table 6.4.5.3.2-2. | - | - | - | - |
| 37-46 | Steps 1 to 10 of the procedure described in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. | - | - | - | - |
| 47 | The SS changes Cell 2 and Cell 7 levels according to the row "T4" in table 6.4.5.3.2-1 and table 6.4.5.3.2-2. | - | - | - | - |
| 48-54 | Check: Does the test result of steps 1 to 7 of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on Cell 2 within 6 mins from "T4" to the time RRCConnectionRequest received by SS? NOTE: The UE performs a TAU procedure | - | - | 1 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

6.4.5.3.3	Specific message contents

Table 6.4.5.3.3-1: Conditions for specific message contents
in Table 6.4.5.3.3-3

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |

Table 6.4.5.3.3-2: System Information Block type 19 for Cell 5 and Cell 7 (preamble and all steps, Table 6.4.5.3.2-3)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.5.3.3-3: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.4.5.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |  |
| csg-Indication | FALSE |  |  |  |
| csg-Identity | Not present |  | Cell 1 |  |
| csg-Identity | '000 0000 0000 0000 0000 0000 0010'B |  | Cell 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.4.5.3.3-4: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.4.5.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | 4 |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.5.3.3-5: SystemInformationBlockType6 for Cell 1 and Cell 2 (preamble and all steps, Table 6.4.5.3.2-3)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

### 6.4.6	Inter-RAT Cell reselection / From UTRA CELL_PCH to E-UTRA RRC_IDLE member hybrid cell

6.4.6.1	Test Purpose (TP)

(1)

with { UE in UTRA Cell PCH state and UE’s Permitted CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRA member hybrid cell that is not the strongest cell on that frequency }

then { UE does not reselect to the E-UTRA member hybrid cell }

}

(2)

with { UE in UTRA Cell PCH state and UE’s Permitted CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRA member hybrid cell previously visited }

then { UE reselects the suitable previously visited E-UTRA member hybrid cell irrespective of the EUTRAN cell reselection priority }

}

6.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, 5.2.6.5, 5.2.6.4.1, TS 36.304 clause 5.2.4.9, 5.2.4.8.1, 5.2.4.6, 5.2.4.8.2, 5.2.4.1 and TS 25.133 clause 4.2.2.11.2.

[TS 25.304, clause 5.2.6.5]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 25.304, clause 5.2.6.4.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE is required to perform autonomous search function in Idle, Cell_PCH and URA_PCH states. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty. If "Dedicated CSG frequency(ies) " IE is present, the UE may use the autonomous search function only on these dedicated frequencies and on the other frequencies listed in the system information.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

When the UE has no or an empty Permitted CSG list, the UE may ignore cells with PSC in the stored range "CSG PSC Split Information" [4] reserved for CSG cells for intra-frequency and inter-frequency measurements and cell re-selections.

…

If the UE detects a suitable CSG cell on a different frequency it shall reselect this cell irrespective of the cell reselection rules applicable for the cell the UE is currently camped on, if the detected suitable CSG cell is the strongest cell on that frequency. If suitable CSG cells are detected on different frequencies and these are the strongest cells on their frequencies, then the UE shall reselect to any one of them.

If the UE detects one or more suitable CSG cell on another RAT, the UE shall reselect to one of them according to [18].

[TS 25.133, clause 4.2.2.11]

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s Permitted CSG list.  The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

NOTE:	According to [1], the UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

[TS 25.133, clause 4.2.2.11.2]

The UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its allow-list, within 6 minutes in the conditions shown in table 4.2.2.11.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.2-1: Parameters for CSG inter-RAT E-UTRA reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| E-UARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE allow-list from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_EcNote2 | dBm | [≥TBD] | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | [≥TBD] |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the E-UARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

[TS 36.304, clause 5.2.4.9]

In addition to normal cell reselection rules, the UE shall use an autonomous search function to detect at least previously visited hybrid cells whose CSG IDs are in the UE’s Permitted CSG list according to the performance requirements specified in [10]. The UE shall treat detected hybrid cells as CSG cells if the CSG ID of the hybrid cell is in the UE’s Permitted CSG list and as normal cells otherwise.

[TS 36.304, clause 5.2.4.8.1]

In addition to normal cell reselection, the UE shall use an autonomous search function to detect at least previously visited allowed CSG cells on non-serving frequencies, including inter-RAT frequencies, according to the performance requirements specified in [10], when at least one CSG ID is included in the UE’s Permitted CSG list. The UE may also use autonomous search on the serving frequency. The UE shall disable the autonomous search function for CSG cells if the UE’s Permitted CSG list is empty.

NOTE:	The UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

If the UE detects one or more suitable CSG cells on different frequencies, then the UE shall reselect to one of the detected cells irrespective of the  frequency priority of the cell the UE is currently camped on, if the concerned CSG cell is the highest ranked cell on that frequency.

If the UE detects a suitable CSG cell on the same frequency, it shall reselect to this cell as per normal reselection rules (5.2.4.6.).

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.2.4.8.2]

While camped on a suitable CSG cell, the UE shall apply the normal cell reselection rules as defined in subclause 5.2.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection.

…

While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency.

6.4.6.3	Test description

6.4.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN Cells.

-	Cell 5 is UTRAN Cell.

-	Cell 1, Cell 5 are not CSG or Hybrid CSG cells.

-	Cell 2 is CSG cell.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell 1.

-	System information combination 11 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell 2.

UE:

-	The UE’s Allowed CSG list shall not contain CSG ID 2.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.4.6.3.2	Test procedure sequence

Table 6.4.6.3.2-1 and  Table 6.4.6.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.4.6.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
|  | Srxlev* | dB | NA | NA | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
|  | Srxlev* | dB | 21 | 15 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.6.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev* | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |
| T4 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Srxlev* | dB | 19 |  |

Table 6.4.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | Void | - | - | - | - |
| 0B | Void | - | - | - | - |
| 0C | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.4.6.3.2-1. | - | - | - | - |
| 0D | The UE is made to perform manual CSG ID selection and select Cell 2. | - | - | - | - |
|  | NOTE: The UE is camped on E-UTRAN Cell 2. | - | - | - | - |
| 0E | The SS notifies the UE of change of System Information. SS adjusts SIB1 of Cell 2 to indicate csg-Indication =false. Wait for 6 s for UE to receive system information. | - | - | - | - |
| 0F | The SS changes Cell 1, Cell 2 and Cell 5 levels according to row "T2" in table 6.4.6.3.2-1 and table 6.4.6.3.2-2. | - | - | - | - |
| 0G | Steps 1 to 8 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs RAU | - | - | - | - |
| 0H | The Procedure for bringing the UE to cell_PCH state defined in TS34.108 clause 7.4.2.7 takes place and the UE is moved to CELL-PCH state | - |  | - | - |
| 1 | The SS changes Cell 1 and Cell 2 levels according to the row "T3" in table 6.4.6.3.2-1. (Note 1) | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 6 minutes? | - | - | 1 | F |
| 3 | The SS changes Cell 1 and Cell 2 levels according to the row "T4" in table 6.4.6.3.2-1. | - | - | - | - |
| 4-10 | Check: Does the test result of steps 1 to 7 of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 2 within 6 minutes from T4 to the time RRCConnctionRequest received by SS? NOTE: The UE performs a TAU procedure | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note 1: SS should not send any RRC message after UE has successfully proceed to CELL_PCH state. |  |  |  |  |  |

6.4.6.3.3	Specific message contents

Table 6.4.6.3.3-1: Conditions for specific message contents
in Table 6.4.6.3.3-2

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 2 | This condition applies to system information transmitted on Cell 2. |

Table 6.4.6.3.3-2: SystemInformationBlockType1 for Cell 1 (preamble and all steps, Table 6.4.6.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not Present |  |  |
| } |  |  |  |

Table 6.4.6.3.3-2A: SystemInformationBlockType1 for Cell 2 (preamble, Table 6.4.6.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  |  |
| csg-Identity | '000 0000 0000 0000 0000 0000 0001'B |  |  |
| } |  |  |  |

Table 6.4.6.3.3-2B: SystemInformationBlockType1 for Cell 2 (step 0E, Table 6.4.6.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | '000 0000 0000 0000 0000 0000 0001'B |  |  |
| } |  |  |  |

Table 6.4.6.3.3-3: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.4.6.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csg-PhysCellIdRange |  |  |  |
| Start | 4 |  |  |
| Range | Not present |  |  |
| } |  |  |  |

Table 6.4.6.3.3-4: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.4.6.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| Priority | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry | the first entry only |  |
| priority[1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.6.3.3-5: SystemInformationBlockType6 for Cell 1 and Cell 2 (Pre-test conditions and all steps, Table 6.4.6.3.2-3

| Derivation Path: 36.508 Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-FDD |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry | the first entry only | UTRA-TDD |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

### 6.4.7	Inter-RAT Cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA RRC_IDLE member hybrid cell

6.4.7.1	Test Purpose (TP)

(1)

with { UE in GSM/GPRS Registered state and no RR connection }

ensure that {

when { Manual CSG ID selection is requested }

then { UE select the suitable E-UTRAN CSG cell }

}

(2)

with { UE in GSM/GPRS Registered state and no RR connection and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects a suitable E-UTRA member hybrid cell previously visited }

then { UE reselects the suitable previously visited E-UTRAN CSG cell irrespective of the EUTRAN cell reselection priority }

}

6.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.2.4, TS 45.008, clause 6.6.7.1, TS 45.008, clause 6.6.7.2 and TS 36.304, clause 10.1.3.4.

[TS 24.301, clause 5.5.3.2.4]

If the UE has initiated the tracking area updating procedure due to manual CSG selection and receives a TRACKING AREA UPDATE ACCEPT message, and the UE sent the TRACKING AREA UPDATE REQUEST message in a CSG cell, the UE shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSD ID to the Allowed CSG list and the UE may add the HNB Name (if provided by lower layers) to the Allowed CSG list if the HNB Name is present in neither the Operator CSG list nor the Allowed CSG list.

[TS 45.008, clause 6.6.7.1a]

If a mobile station is a member of at least one Closed Subscriber Group then, in addition to normal cell reselection, the MS shall use an autonomous search function to detect hybrid cells. The autonomous search function shall at least detect previously visited hybrid cells whose CSG IDs are included in the Permitted CSG list of the MS.

NOTE:	The autonomous search for hybrid cells does not imply that the MS needs to constantly check the CSG IDs of all cells it sees, and the impact on battery consumption should be minimised.

If a neighbour cell has been detected as a hybrid cell and the CSG ID of the hybrid cell is included in the Permitted CSG list of the MS, reselection to that cell shall follow the rules for CSG cells in subclause 6.6.7.1. Otherwise normal cell reselection rules (see subclause 6.6.5 and 6.6.6) shall apply.

[TS 45.008, clause 6.6.7.1]

If a mobile station is a member of at least one Closed Subscriber Group, i.e. at least one CSG ID is included in the MS’s Permitted CSG list, then, in addition to normal cell reselection, the MS shall use an autonomous search function to detect UTRAN and/or E-UTRAN CSG cells. The autonomous search function shall at least detect previously visited allowed CSG cells.

NOTE 1:	The autonomous search function is implementation dependent and controls when and/or where to search for allowed CSG cells.

…

If the strongest cell (see 3GPP TS 25.304 and 3GPP TS 36.304 for the definition of the strongest cell) which the MS has detected on a UTRAN or E-UTRAN frequency during a time interval T_reselection is a suitable CSG cell (see 3GPP TS 25.304 and 3GPP TS 36.304 for suitability criteria for UTRAN and E-UTRAN CSG cells respectively), the MS shall reselect to this cell irrespective of the cell reselection rules applicable for the cell on which the MS is currently camped.

The following requirements are valid for reselection to allowed CSG cells previously visited by the MS when the radio configuration parameters, including the carrier frequency and PSC/PCI (whichever and if it is applicable) of the CSG cell, non CSG cell and other neighbour cells, are unchanged from the most recent previous visit. The autonomous search and cell re-selection to a previously visited allowed CSG cell shall meet the performance requirements defined as follows:

- the MS shall perform search and re-selection to a previously visited allowed UTRAN FDD CSG cell, that has met the CSG cell re-selection criterion defined above, within 6 minutes in the radio conditions specified for an UTRAN FDD CSG cell in 3GPP TS 36.133 in terms of parameters for CSG inter-RAT UTRAN FDD reselection; UARFCN and PSC shall be unchanged from the most recent previous visit of the UTRAN FDD CSG cell; Serving GSM cell at RXLEV= -70 dBm, with 6 GSM neighbours at RXLEV= -75 dBm;

- the MS shall perform search and re-selection to a previously visited allowed E-UTRAN CSG cell, that has met the CSG cell re-selection criterion defined above, within 6 minutes in the radio conditions specified for an E-UTRAN CSG cell in 3GPP TS 25.133 in terms of parameters for CSG inter-RAT E-UTRA reselection; E-UARFCN and PCI shall be unchanged from the most recent previous visit of the E-UTRAN CSG cell; Serving GSM cell at RXLEV= -70 dBm, with 6 GSM neighbours at RXLEV= -75 dBm.

NOTE 4:	The above performance requirements are minimum requirements defined to ensure the testability of autonomous CSG search.

[TS 45.008, clause 6.6.7.2]

If NAS requests AS to search for available CSG IDs, the MS shall perform the search and report the results to NAS as described in 3GPP TS 25.304 for UTRAN and as described in 3GPP TS 36.304 for E-UTRAN.

If a CSG ID is manually selected by NAS, the MS shall behave as specified in 3GPP TS 25.304 or in 3GPP TS 36.304, depending on the RAT type of the selected CSG cell.

[TS 45.008, clause 10.1.3.4]

If a mobile station is a member of at least one Closed Subscriber Group, i.e. at least one CSG ID is included in the MS’s Permitted CSG list, then, in addition to normal cell reselection, the MS shall use an autonomous search function to detect UTRAN and/or E-UTRAN CSG cells and hybrid cells. The autonomous search function shall at least detect previously visited allowed CSG cells and previously visited hybrid cells whose CSG IDs are included in the Permitted CSG list of the MS.

6.4.7.3	Test description

6.4.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are E-UTRAN Cells.

-	Cell 24 and Cell 25 are GERAN Cells.

-	Cell 1, Cell 24 and Cell 25 are not CSG cells or hybrid cells.

-	Cell 2 is E-UTRAN Hybrid Cell.

-	Cell 24 and Cell 25 have different Routing Area.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell 1.

-	System information combination 14 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell 2.

UE:

-	The UE’s Permitted CSG list contains the CSG ID of Cell 2. This is realized by using manual CSG selection.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

6.4.7.3.2	Test procedure sequence

Tables 6.4.7.3.2-1 and 6.4.7.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.4.7.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Srxlev* | dB | “Off” | “Off” | S nonServingCell, Cell7 > Thresh Cell7,high |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | Srxlev* | dB | 15 | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 6.4.7.3.2-2: Time instances of cell power level and parameter changes for GERAN cell

|  | Parameter | Unit | Cell 24 | Cell 25 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | RF Signal Level | dBm | -80 | “Off” | The power levels are such that camping on Cell 24 is guaranteed. |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| T1 | RF Signal Level | dBm | -80 | “Off” |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| T2 | RF Signal Level | dBm | “Off” | -80 | The power levels are such that camping on Cell 25 is guaranteed. |
|  | RXLEV_ACCESS_MIN | dBm | “Off” | -101 |  |
|  | C1* | dB | “Off” | 21 |  |
| T3 | RF Signal Level | dBm | -80 | -90 |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | -101 |  |
|  | C1* | dB | 21 | 11 |  |
| T4 | RF Signal Level | dBm | -80 | “Off” |  |
|  | RXLEV_ACCESS_MIN | dBm | -101 | “Off” |  |
|  | C1* | dB | 21 | “Off” |  |
| Note: C1 is calculated in the UE |  |  |  |  |  |

Table 6.4.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Call the registration procedure described in TS 34.123-3 subclause 6.10.2.7.1.1 performed on Cell 24. NOTE: The UE performs registration and the RR connection is released. | - | - | - | - |
| 2A | The UE may attempt the procedure for IMS signalling on GERAN as specified in TS 36.508 clause 4.5A.3B | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 6.4.7.3.2-1. | - | - | - | - |
| 4 | The UE is made to perform manual CSG ID selection and select Cell 2. | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 2? | - | - | 1 | - |
| 6 | The SS changes Cell 1, Cell 2, Cell 24 and Cell 25 levels according to row "T2" in table 6.4.7.3.2-1 and table 6.4.7.3.2-2. | - | - | - | - |
| 7-17 | Steps 1 to 11 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.9 are performed on Cell 25. NOTE: The UE performs RAU and the RR connection is released. | - | - | - | - |
| 18 | The SS changes Cell 1, Cell 24 and Cell 25 levels according to row "T3" in table 6.4.7.3.2-1 and table 6.4.7.3.2-2. | - | - | - | - |
| 19-29 | Steps 1 to 11 of the Routing Area Update procedure described in TS 36.508 subclause 6.4.2.9 are performed on Cell 24. NOTE: The UE performs RAU and the RR connection is released. | - | - | - | - |
| 30 | The SS changes Cell 1, Cell 2 and Cell 25 level according to row "T4" in table 6.4.7.3.2-1 and table 6.4.7.3.2-2. | - | - | - | - |
| 31 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 2 within 6 mins? | - | - | 2 | - |

6.4.7.3.3		Specific message contents

Table 6.4.7.3.3-1: SystemInformationBlockType1 for Cell 2 (preamble and all steps, Table 6.4.7.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | '000 0000 0000 0000 0000 0000 0001'B |  |  |
| } |  |  |  |

Table 6.4.7.3.3-2: SystemInformationBlockType4 for Cell 2 (preamble and all steps, Table 6.4.7.3.2-3)

| Derivation Path: 36.508 Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csg-PhysCellIdRange |  |  |  |
| Start | 4 |  |  |
| Range | Not present |  |  |
| } |  |  |  |

Table 6.4.7.3.3-3: SYSTEM INFORMATION TYPE 2 Quarter for Cell 24 and Cell 25 (Pre-test conditions and all steps, Table 6.4.7.3.2-3

| Derivation Path: 36.508 clause 4.4.5, Table 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI 2QUATER REST OCTETS |  |  |  |
| Additions in release 8 | H |  |  |
| Priority and E-UTRAN Parameters Description | 1 | Present |  |
| Serving Cell Priority Parameters Description | 1 | Present |  |
| GERAN_PRIORITY | 5 | Higher priority than E-UTRA |  |

Table 6.4.7.3.3-4: SystemInformationBlockType7 for Cell 1 and Cell 2 (Pre-test conditions and all steps, Table 6.4.7.3.2-3

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 | INTEGER (0..7) |  |
| speedDependentScalingParameters SEQUENCE {} | Not present |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqsGERAN SEQUENCE [1] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 24 |  |  |
| numberOfFollowingARFCNs | Same as cell 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Greater than EUTRA Freq |  |
| } |  |  |  |
| carrierFreqsGERAN SEQUENCE [2] { |  |  |  |
| startingARFCN | Same as cell 25 |  |  |
| bandIndicator | Same as cell 25 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 25 |  |  |
| numberOfFollowingARFCNs | Same as cell 25 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Greater than EUTRA Freq |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 6.5	WLAN Offload for E-UTRA RRC Idle

### 6.5.1	WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qrxlevmeas, BeaconRSSI, WLAN identifier no match/match)

6.5.1.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_IDLE state, and UE receives SystemInformationBlockType17 including RAN-assisted WLAN interworking data and configured with ThreshServingOffloadWLAN, LowP   and ThreshBeaconRSSIWLAN, High }

ensure that {

when { UE measures that Qrxlevmeas is less than ThreshServingOffloadWLAN, LowP and BeaconRSSI is greater than ThreshBeaconRSSIWLAN, High}

then { UE initiates handover attach to WLAN after TsteeringWLAN  }

}

(2)

with { UE is associated with WLAN AP and attached to WLAN ePDG and configured with ThreshServingOffloadWLAN, HighP   and ThreshBeaconRSSIWLAN, Low }

ensure that {

when { UE measures that Qrxlevmeas is greater than ThreshServingOffloadWLAN, HighP and BeaconRSSI is less than ThreshBeaconRSSIWLAN, Low}

then { UE initiates handover attach to EUTRA after TsteeringWLAN }

}

(3)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { The EUTRA cell and WLAN AP with a non matching WLAN identifier, meet steering conditions to offload from EUTRA to WLAN}

then { UE does not offload PDN connection to WLAN }

}

6.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.6.2.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1. In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2. In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

3.In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

4.In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

6.5.1.3	Test description

6.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN AP 1

-	System information combination 22   as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The UE supporting ANDSF is pre-provisioned with WLANSP ANDSF MO with at least one WLAN identifier and with ForServiceBased ANDSF MO with RANValidityCondition allowing at least one APN to use both WLAN and LTE and at least one APN to use LTE only.

-	The UE supporting RAN rules is configured with at least two PDN connections, one PDN connection offloadable to WLAN and one PDN connection not offloadable to WLAN.

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.5.1.3.2	Test procedure sequence

Table 6.5.1.3.2-1 illustrates the downlink power levels to be applied for Cell 1and WLAN AP 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.5.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 (WLAN) | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | E-UTRA Cell 1 is a suitable cell and WLAN is off. |
|  | BeaconRSSI | dBm | - | off |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for steering from E-UTRA to WLAN are satisfied. Qrxlevmeas < ThreshServingOffloadWLAN, LowP and BeaconRSSI > ThreshBeaconRSSIWLAN, High |
|  | BeaconRSSI | dBm | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -79 | -60 | The power level values are such that conditions for steering from WLAN to E-UTRA are satisfied. Qrxlevmeas > ThreshServingOffloadWLAN, HighP and BeaconRSSI > ThreshBeaconRSSIWLAN, Low |

Table 6.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row “T1” in Table 6.5.1.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE perform Offload to WLAN procedure in TS 36.508 Table 6.4.3.10-1)? | - | - | 1 | P |
| 3 | The SS changes Cell 1 Cell 27 and parameters according to the row "T2" in table 6.5.1.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE perform Offload from WLAN procedure in TS 36.508 Table 6.4.3.11-1)? | - | - | 2 | P |
| 5 | The SS release the RRC connection. | <-- | RRC: RRCConnectionRelease |  |  |
| 6 | The SS notifies the UE of change of System Information. SS adjusts SIB17 of Cell 1 to indicate WLAN SSID to a value different from that of cell 27 | <-- | Paging | - | - |
| 7 | The SS changes Cell 1 and Cell 27 parameters according to the row "T1" in table 6.5.1.3.2-1. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.12-1 indicate that the PDN connection is not offloaded from E-UTRAN to the WLAN? | - | - | 3 | - |

6.5.1.3.3	Specific message contents

Table 6.5.6.3.3-1: SystemInformationBlockType17 (Preamble)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 6.5.1.3.3-1: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRP and BeaconRSSI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| thresholdRSRP-High-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| } |  |  |  |
| thresholdRSRQ-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbolsWithWB-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbols-r12 | Not Present |  |  |
| thresholdRSRQ-WB-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12 | Not Present |  |  |
| thresholdBackhaul-Bandwidth-r12 | Not Present |  |  |
| thresholdBeaconRSSI-r12::= SEQUENCE { |  |  |  |
| thresholdBeaconRSSI-Low-r12 | 50 | Value in dBm =-78 |  |
| thresholdBeaconRSSI-High-r12 | 60 | Value in dBm =-68 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.13.3-3: SystemInformationBlockType17 (step 6 of table 6.5.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT(Table 6.5.1.3.3-1) |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 1 Entry |  |
| wlan-Identifiers SEQUENCE{ |  |  |  |
| ssid-r12 | Set to a different value than transmitted by cell 27 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

### 6.5.2	WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qrxlevmeas, BackhaulRateDlWLAN)

6.5.2.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_Idle state and configured with thresholdRSRP-r12 and BackhaulRateDlWLAN in RRCConnectionReconfiguration }

ensure that {

when { UE detects the condition for steering traffic from E-UTRA to WLAN is met for a time interval TsteeringWLAN }

then { UE performs offload from E-UTRAN to WLAN }

}

(2)

with { UE connected in WLAN and configured with thresholdRSRP-r12 and BackhaulRateDlWLAN }

ensure that {

when { UE detects the condition (RSRPmeas > ThreshServingOffloadWLAN, HighP )for steering traffic from WLAN to E-UTRA is met for a time interval TsteeringWLAN }

then { UE performs offload from WLAN to E-UTRAN }

}

6.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.12.2 and TS 24.302, clause 6.10.4 and TS 36.304, clauses 5.6.1, 5.6.2 and 5.6.3. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

[TS 24.302, clause 6.10.4]

the UE:

a)	shall perform the procedure in subclause 5.1.3.2.3 and in subclause 5.2.3.2 to select the selected WLAN and the NAI for authentication;

b)	if not authenticated yet with the selected WLAN using the NAI for authentication in subclause 6.4, shall authenticate with the selected WLAN using the NAI for authentication in subclause 6.4. During authentication, if the selected WLAN is a trusted WLAN, SCM is supported by both UE and network, MCM is not supported by UE, network or both, and if:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and the UE has only one PDN connection;

shall handover one offloadable PDN connection from 3GPP access to the WLAN access using procedures in subclause 6.4.2.6.2;

NOTE:	When the UE already has one PDN connection established via WLAN in SCM, and if move-traffic-to-WLAN indication is received, it is up to the UE implementation to determine whether to offload a PDN connection from 3GPP access to WLAN. In that case, it is also up to the UE implementation to determine which one of the offloadable PDN connections will be offloaded.

c)	if the selected WLAN is a trusted WLAN, and MCM is supported by both UE and network, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures of 3GPP TS 24.244 [56];

d)	if the selected WLAN is an untrusted WLAN, and if the UE supports access to EPC via untrusted WLAN, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures in subclause 7.2.1 and subclause 7.2.2; and

e)	if the UE has a valid IARP rule for APN, shall use the IARP for APN using the procedures in subclause 6.8.2.2.4.5.

Upon receiving move-traffic-from-WLAN indication, and if the user preferences are not present, the UE shall handover all the PDN connections established in (or previously handed over to) WLAN access to the 3GPP access using procedures in 3GPP TS 24.301 [10].

[TS 36.304, clause 5.6.1]

RAN assistance parameters may be provided to the UE in SystemInformationBlockType17 or in the RRCConnectionReconfiguration message. RAN assistance parameters are used only if the UE is camped normally.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1.	In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2.	In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

4.	In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

3.	In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only.

[TS 36.304, clause 5.6.2]

The following RAN assistance parameters for RAN-assisted WLAN interworking may be provided:

ThreshServingOffloadWLAN, LowP

This specifies the RSRP threshold (in dBm) used by the UE for traffic steering to from E-UTRAN to WLAN.

ThreshServingOffloadWLAN, HighP

This specifies the RSRP threshold (in dBm) used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshServingOffloadWLAN, LowQ

This specifies the RSRQ threshold (in dB) used by the UE for traffic steering from E-UTRAN to WLAN.

ThreshServingOffloadWLAN, HighQ

This specifies the RSRQ threshold (in dB) used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshChUtilWLAN, Low

This specifies the WLAN channel utilization (BSS load) threshold used by the UE for traffic steering from E-UTRAN to WLAN.

ThreshChUtilWLAN, High

This specifies the WLAN channel utilization (BSS load) threshold used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshBackhRateDLWLAN, Low

This specifies the backhaul available downlink bandwidth threshold used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshBackhRateDLWLAN, High

This specifies the backhaul available downlink bandwidth threshold used by the UE for traffic steering from E-UTRAN to WLAN.

ThreshBackhRateULWLAN, Low

This specifies the backhaul available uplink bandwidth threshold used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshBackhRateULWLAN, High

This specifies the backhaul available uplink bandwidth threshold used by the UE for traffic steering from E-UTRAN to WLAN.

ThreshBeaconRSSIWLAN, Low

This specifies the Beacon RSSI threshold used by the UE for traffic steering from WLAN to E-UTRAN.

ThreshBeaconRSSIWLAN, High

This specifies the Beacon RSSI threshold used by the UE for traffic steering from E-UTRAN to WLAN.

TsteeringWLAN

This specifies the timer value TsteeringWLAN during which the rules should be fulfilled before starting traffic steering between E-UTRAN and WLAN.

WLAN identifiers

Only the SSIDs, BSSIDs and HESSIDs which are provided in this parameter shall be considered for traffic steering between E-UTRAN and WLAN based on the rules in this subclause.

6.5.2.3	Test description

6.5.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 27

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to TS 36.508 [18] clause 4.5.2D.

6.5.2.3.2	Test procedure sequence

Table 6.5.2.3.2-1 illustrates the downlink power levels/parameters to be applied for Cell 1and WLAN at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.5.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | WLAN | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | Conditions RSRPmeas < ThreshServingOffloadWLAN, LowP and BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High are satisfied |
|  | BackhaulRateDlWLAN | kbps | - | r16384 |  |
|  | BackhaulRateUlWLAN | kbps | - | r8192 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | Condition BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low is satisfied and RSRPmeas > ThreshServingOffloadWLAN, HighP is not satisfied |
|  | BackhaulRateDlWLAN | kbps | - | r2048 |  |

Table 6.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and WLAN AP parameters according to the row "T0" in table 6.5.2.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.10-1 indicate that the PDN connection is offloaded from E-UTRAN to the WLAN? | - | - | 1 | P |
| 3 | The SS changes Cell 1 and WLAN parameters according to the row "T1" in table 6.5.5.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicate that the PDN connection is offloaded from the WLAN access to E-UTRAN? | - | - | 2 | P |

6.5.2.3.3	Specific message contents

Table 6.5.2.3.3-1: SystemInformationBlockType17 (Preamble and all steps of table 6.5.2.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 6.5.2.3.3-2: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRP and BackhaulBW |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| thresholdRSRP-High-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| } |  |  |  |
| thresholdRSRQ-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbolsWithWB-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbols-r12 | Not Present |  |  |
| thresholdRSRQ-WB-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12 | Not Present |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r4096 kbps |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r8192 kbps |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r0 kbps | Value makes it insignificant for offload |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r4294967296 kbps | Value makes it insignificant for offload |  |
| } |  |  |  |
| thresholdBeaconRSSI-r12 | Not Present |  |  |
| } |  |  |  |

### 6.5.3	WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qqualmeas, BackhaulRateUlWLAN)

6.5.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects that Qqualmeas < ThreshServingOffloadWLAN, LowQ and BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High }

then { UE offload PDN connections to WLAN }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects that Qqualmeas > ThreshServingOffloadWLAN, HighQ and BackhaulRateUlWLAN > ThreshBackhRateULWLAN, Low }

then { UE offload PDN connections to E-UTRAN }

}

6.5.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.12.2 and TS 24.302, clause 6.10.4 and TS 36.304 clauses 5.6.1 and 5.6.2.

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

[TS 24.302, clause 6.10.4]

….

the UE:

a)	shall perform the procedure in subclause 5.1.3.2.3 and in subclause 5.2.3.2 to select the selected WLAN and the NAI for authentication;

b)	if not authenticated yet with the selected WLAN using the NAI for authentication in subclause 6.4, shall authenticate with the selected WLAN using the NAI for authentication in subclause 6.4. During authentication, if the selected WLAN is a trusted WLAN, SCM is supported by both UE and network, MCM is not supported by UE, network or both, and if:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and the UE has only one PDN connection;

shall handover one offloadable PDN connection from 3GPP access to the WLAN access using procedures in subclause 6.4.2.6.2;

NOTE:	When the UE already has one PDN connection established via WLAN in SCM, and if move-traffic-to-WLAN indication is received, it is up to the UE implementation to determine whether to offload a PDN connection from 3GPP access to WLAN. In that case, it is also up to the UE implementation to determine which one of the offloadable PDN connections will be offloaded.

c)	if the selected WLAN is a trusted WLAN, and MCM is supported by both UE and network, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures of 3GPP TS 24.244 [56];

d)	if the selected WLAN is an untrusted WLAN, and if the UE supports access to EPC via untrusted WLAN, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures in subclause 7.2.1 and subclause 7.2.2; and

e)	if the UE has a valid IARP rule for APN, shall use the IARP for APN using the procedures in subclause 6.8.2.2.4.5.

Upon receiving move-traffic-from-WLAN indication, and if the user preferences are not present, the UE shall handover all the PDN connections established in (or previously handed over to) WLAN access to the 3GPP access using procedures in 3GPP TS 24.301 [10].

[TS 36.304, clause 5.6.1]

RAN assistance parameters may be provided to the UE in SystemInformationBlockType17 or in the RRCConnectionReconfiguration message. RAN assistance parameters are used only if the UE is camped normally.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1. In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2. In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

3. In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

4. In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

6.5.3.3	Test description

6.5.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.5.3.3.2	Test procedure sequence

Table 6.5.3.3.2-1 illustrates the downlink power levels to be applied for Cell 1and WLAN at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.5.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | WLAN | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | E-UTRA Cell 1 is a suitable cell and WLAN is off. |
|  | BeaconRSSI | - | - | off |  |
|  | RSRQ | dB | -4.15 | - |  |
|  | Noc | dBm/15kHz | -73 |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied. Qqualmeas < ThreshServingOffloadWLAN, LowQ and BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; |
|  | BackhaulRateDlWLAN | kbps | - | r8192 |  |
|  | BackhaulRateUlWLAN | kbps | - | r16384 |  |
|  | RSRQ | dB | -13.23 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA are satisfied. Qqualmeas > ThreshServingOffloadWLAN, HighQ; and BackhaulRateUlWLAN > ThreshBackhRateULWLAN, Low; |
|  | BackhaulRateUlWLAN | kbps | - | r8192 |  |
|  | RSRQ | dB | -4.15 | - |  |

Table 6.5.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and WLAN parameters according to the row "T1" in table 6.5.3.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.10-1 indicate that the PDN connection is offloaded from E-UTRAN to the WLAN? | - |  | 1 | P |
| 3 | The SS changes Cell 1 and WLAN parameters according to the row "T2" in table 6.5.3.3.2-1 | - |  |  |  |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicate that the PDN connection is offloaded from the WLAN access to E-UTRAN? | - |  | 2 | P |

6.5.3.3.3	Specific message contents

Table 6.5.3.3.3-1: SystemInformationBlockType17 (Preamble and all steps of table 6.5.3.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 6.5.3.3.3-2: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRQ and BackhaulBW |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRQ-r12::= SEQUENCE { |  |  |  |
| thresholdRSRQ-Low-r12 | 17 | -11.5dB RSRQ < -11dB |  |
| thresholdRSRQ-High-r12 | 26 | -7dB RSRQ < -6.5dB |  |
| } |  |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r0 |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r4294967296 |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r4096 |  |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r8192 |  |  |
| } |  |  |  |
| thresholdBeaconRSSI-r12 | Not Present |  |  |
| } |  |  |  |

### 6.5.4	WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qqualmeas, ChannelUtilizationWLAN)

6.5.4.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_IDLE state, and UE receives SystemInformationBlockType17 including RAN-assisted WLAN interworking data and configured with ThreshServingOffloadWLAN, LowQ and ThreshChUtilWLAN, Low }

ensure that {

when { UE measures that Qrxlevmeas is less than ThreshServingOffloadWLAN, LowQ and ChannelUtilizationWLAN is less than ThreshChUtilWLAN, Low }

then { UE initiates handover attach to WLAN after TsteeringWLAN  }

}

(2)

with { UE is associated with WLAN AP and attached to WLAN ePDG and configured with ThreshServingOffloadWLAN, HighQ   and ThreshChUtilWLAN, High }

ensure that {

when { UE measures that Qrxlevmeas is greater than ThreshServingOffloadWLAN, HighQ and ChannelUtilizationWLAN is less than ThreshChUtilWLAN, High }

then { UE initiates handover attach to EUTRA after TsteeringWLAN }

}

6.5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.6.2.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1.	In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2.	In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

3.	In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

4.	In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

6.5.4.3	Test description

6.5.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN AP 1

-	System information combination 22   as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The UE supporting ANDSF is pre-provisioned with WLANSP ANDSF MO with at least one WLAN identifier and with ForServiceBased ANDSF MO with RANValidityCondition allowing at least one APN to use both WLAN and LTE and at least one APN to use LTE only.-	The UE supporting RAN rules is configured with at least two PDN connections, one PDN connection offloadable to WLAN and one PDN connection not offloadable to WLAN.

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.5.4.3.2	Test procedure sequence

Table 6.5.4.3.2-1 illustrates the downlink power levels to be applied for Cell 1and WLAN AP 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.5.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Noc | dBm/15kHz | -73 | off | E-UTRA Cell 1 is a suitable cell and WLAN AP 1 is off. |
|  | RSRQ (Note 1) | dB | -4.15 | - |  |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 | - |  |
|  | ChannelUtilizationWLAN |  | - | off |  |
| T1 | RSRQ (Note 1) |  | -13.23 | - | The power level values are such that conditions for steering from E-UTRA to WLAN are satisfied. Qqualmeas < ThreshServingOffloadWLAN, LowQ and ChannelUtilizationWLAN < ThreshChUtilWLAN, Low |
|  | Cell-specific RS EPRE | dBm/15 KHz | -75 | - |  |
|  | ChannelUtilizationWLAN |  | - | 50 |  |
| T2 | RSRQ (Note 1) |  | -4.15 | - | The power level values are such that conditions for steering from WLAN to E-UTRA are satisfied. Qqualmeas > ThreshServingOffloadWLAN, HighQ and ChannelUtilizationWLAN < ThreshChUtilWLAN, High |
|  | Cell-specific RS EPRE | dBm/15 KHz | -60 | - |  |
|  | ChannelUtilizationWLAN |  | - | 150 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 6.5.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row “T1” in Table 6.5.4.3.2-1. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE perform Offload to WLAN procedure in TS 36.508 Table 6.4.3.10-1? | - | - | 1 | P |
| 4 | The SS changes Cell 1 and WLAN AP 1 parameters according to the row "T2" in table 6.5.4.3.2-1. | - | - |  |  |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE perform Offload from WLAN procedure in TS 36.508 Table 6.4.3.11-1? | - | - | 2 | P |

6.5.4.3.3	Specific message contents

Table 6.5.4.3.3-1: SystemInformationBlockType17 (Preamble and all steps of table 6.5.4.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| } |  |  |  |

Table 6.5.4.3.3-2: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRQ and ChannelUtilization |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRQ-r12::= SEQUENCE { |  |  |  |
| thresholdRSRQ-Low-r12 | 17 | -11.5dB RSRQ < -11dB |  |
| thresholdRSRQ-High-r12 | 26 | -7dB RSRQ < -6.5dB |  |
| } |  |  |  |
| thresholdChannelUtilization-r12::= SEQUENCE { |  |  |  |
| thresholdChannelUtilizationLow-r12 | 80 |  |  |
| thresholdChannelUtilizationHigh-r12 | 200 |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.5.5	WLAN offload / Cell selection / EUTRA RRC_Idle to/from WLAN (ANDSF and RAN rules co-existence)

6.5.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on RAN rules is satisfied but based on stored ISRP ANDSF rule is not }

then { UE does not offload PDN connections to WLAN }

}

(2)

with { UE in E-UTRA RRC_IDLE state, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on stored ISRP ANDSF rule is satisfied }

then { UE offload PDN connections to WLAN }

}

(3)

with { UE in E-UTRA RRC_IDLE state and A PDN offloaded to WLAN, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on RAN rules is satisfied but based on stored ISRP ANDSF rule is not }

then { UE does not offload PDN connections to EUTRAN }

}

(4)

with { UE in E-UTRA RRC_IDLE state and A PDN offloaded to WLAN, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on stored ISRP ANDSF rule is satisfied }

then { UE offload PDN connections to EUTRAN }

}

6.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.12.3 and TS 24.302, clause 6.10.2 and TS 36.304 clauses 5.6.1 and 5.6.2.

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

[TS 24.302, clause 6.10.2]

The WLAN access selection and traffic routing can be controlled either by ANDSF rules or by RAN rules.

The ANDSF rules control the WLAN access selection and traffic routing if:

a)	the UE has ANDSF rules but no RAN rules; or

b)	the UE has both ANDSF rules and RAN rules; and:

1)	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN; and:

A)	the UE is not roaming and the UE has at least one ISMP rule from HPLMN;

B)	the UE is roaming in a VPLMN contained in the visited PLMNs with preferred rules and the UE has at least one ISMP rule from VPLMN; or

C)	the UE is roaming in a VPLMN not contained in the visited PLMNs with preferred rules and the UE has at least one ISMP rule from HPLMN; or

2)	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; and:

A)	the UE is not roaming and the UE has an valid ISRP rule from HPLMN;

B)	the UE is roaming in a VPLMN contained in the visited PLMNs with preferred rules and the UE has a valid ISRP rule from VPLMN; or

C)	the UE is roaming in a VPLMN not contained in the visited PLMNs with preferred rules and the UE has a valid ISRP rule from HPLMN.

[TS 36.304, clause 5.6.1]

RAN assistance parameters may be provided to the UE in SystemInformationBlockType17 or in the RRCConnectionReconfiguration message. RAN assistance parameters are used only if the UE is camped normally.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1. In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2. In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

3. In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

4. In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

6.5.5.3	Test description

6.5.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 27

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

UE has configured a ISRP ANDSF rule using RAN thresholds of BackhaulRateDlWLAN.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to TS 36.508 [18] clause 4.5.2D.

6.5.5.3.2	Test procedure sequence

Table 6.5.5.3.2-1 illustrates the downlink power levels to be applied for Cell 1and WLAN AP (Cell 27)at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while rows marked "T1",”T2”, “T3” and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.5.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | WLAN | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | E-UTRA Cell 1 is a suitable cell and WLAN is switched off. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied for RSRP and UL Backhaul BW (RAN) and not satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r2048 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r16384 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are not satisfied for RSRP and UL Backhaul BW (RAN) and satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r16384 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r2048 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA are satisfied for UL Backhaul BW(RAN) and not satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r16384 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r2048 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA satisfied for DL Backhaul BW(ANDSF/RAN). |
|  | BackhaulDL-Bandwidth | kbps | - | r2084 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r16384 |  |

Table 6.5.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and WLAN parameters according to the row "T1" in table 6.5.5.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.12-1 indicate that the PDN connection is not offloaded from E-UTRAN to the WLAN? | - |  | 1 | - |
| 3 | The SS changes Cell 1 and WLAN parameters according to the row "T2" in table 6.5.5.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.10-1 indicate that the PDN connection is offloaded from E-UTRAN to the WLAN? | - | - | 2 | P |
| 5 | The SS changes Cell 1 and WLAN parameters according to the row "T3" in table 6.5.5.3.2-1 | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.13-1 indicate that the PDN connection is not offloaded from the WLAN access to E-UTRAN? | - | - | 3 | - |
| 7 | The SS changes Cell 1 and WLAN parameters according to the row "T3" in table 6.5.5.3.2-1 | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicate that the PDN connection is offloaded from the WLAN access to E-UTRAN? | - | - | 4 | P |

6.5.5.3.3	Specific message contents

Table 6.5.5.3.3-1: SystemInformationBlockType17 (Preamble and all steps of table 6.5.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| } |  |  |  |

Table 6.5.5.3.3-2: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRP and BeaconRSSI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| thresholdRSRP-High-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| } |  |  |  |
| thresholdRSRQ-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbolsWithWB-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbols-r12 | Not Present |  |  |
| thresholdRSRQ-WB-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12 | Not Present |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r4096 |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r8192 |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r4096 |  |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r8192 |  |  |
| } |  |  |  |
| thresholdBeaconRSSI-r12 | Not Present |  |  |
| } |  |  |  |