## 8.4	Inter-RAT handover

### 8.4.1	Inter-RAT handover E-UTRA to UTRA

#### 8.4.1.1	Void

#### 8.4.1.2	Inter-RAT handover / From E-UTRA to UTRA PS / Data

8.4.1.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and a DPCH PS RAB combination is configured for an UTRA cell}

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the utra cell}

}

8.4.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

8.4.1.2.3	Test description

8.4.1.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE is previously registered on cell 5.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.1.2.3.2	Test procedure sequence

Table 8.4.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures UTRA cell 5 to reference configuration according 36.508 table 4.8.3-1, condition UTRA PS RB. | - | - | - | - |
| 1A | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 1B | The UE transmit a UECapabilityInformation message on Cell 1. NOTE: The start-PS values received, should be used to configure ciphering on cell 5. | --> | UECapabilityInformation | - | - |
| 2 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 3 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: Step 3Aa1 depend on UE implementation | - | - | - | - |
| 3Aa1 | The UE sends a ROUTING AREA UPDATE REQUEST message within [2] seconds. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a SECURITY MODE COMMAND message on Cell 5 in order to activate integrity protection. | <-- | SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 5Aa1 to 5Aa2 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE sent a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 5Aa1 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 5Aa2 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 6 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information on Cell 5. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 7 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message on Cell 5. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| - | EXCEPTION: In parallel to the events described in steps 7A to 9 and depending on the UE implementation the steps defined in Table 8.4.1.2.3.2-2 may take place. | - | - | - | - |
| 7A | IF pc_IMS = TRUE THEN SS starts 6s timer |  |  |  |  |
| - | EXCEPTION: Steps 8a1 to 8a3 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE did not send a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 8a1 | The UE sends a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 8a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 9 | IF pc_IMS = TRUE THEN SS waits until the 6s timer expires |  |  |  |  |
| - | Note : At the end of the test procedure sequence, the UE is in end state UTRA connected (U2) according to 36.508 | - | - | - | - |

Table 8.4.1.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 [35] takes place. | - | - | - | - |

8.4.1.2.3.3	Specific message contents

Table 8.4.1.2.3.3-1: MobilityFromEUTRACommand (step 2, Table 8.4.1.2.3.2-1)

| Derivation Path: 36.508 table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.2.3.3-2: HANDOVER TO UTRAN COMMAND (Table 8.4.1.2.3.3-1)

Derivation Path: 36.508 table 4.7B.1-1, condition UTRA PS RB

Table 8.4.1.2.3.3-3: SECURITY MODE COMMAND (step 4, Table 8.4.1.2.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (SECURITY MODE COMMAND message) |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

Table 8.4.1.2.3.3-4: UTRAN MOBILITY INFORMATION (step 6, Table 8.4.1.2.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (UTRAN MOBILITY INFORMATION message) |  |
| --- | --- |
| Information Element | Value/remark |
| CN information info |  |
| - PLMN identity |  |
| - MCC | 001 |
| - MNC | 01 |
| - CN common GSM-MAP NAS system information | 00 01H |
| - CN domain information list full |  |
| - CN domain identity | PS |
| - CN domain specific NAS system information | 01 00H |
| - DRX cycle length coefficient | 7 |
| - CN domain identity | CS |
| - CN domain specific NAS system information | 1E 01H |
| - DRX cycle length coefficient | 7 |

Table 8.4.1.2.3.3-5: UECapabilityEnquiry (step 1A, Table 8.4.1.2.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entry |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.2.3.3-6: ROUTING AREA UPDATE ACCEPT (step 5Aa1, step 8a2 Table 8.4.1.2.3.2-2)

| Derivation Path: 36.508 Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update result | 0 ‘Follow-on proceed’ |  |  |

#### 8.4.1.3	Void

#### 8.4.1.4	Inter-RAT handover / From E-UTRA to UTRA HSDPA / Data

8.4.1.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and a DPCH and HS-PDSCH PS RAB combination is configured for an UTRA cell}

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the utra cell}

}

8.4.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

8.4.1.4.3	Test description

8.4.1.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE is previously registered on cell 5.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.1.4.3.2	Test procedure sequence

Table 8.4.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures UTRA cell 5 to reference configuration according 36.508 table 4.8.3-1, condition UTRA HSDPA RB.. | - | - | - | - |
| 1A | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 1B | The UE transmit a UECapabilityInformation message on Cell 1. NOTE: The start-PS values received, should be used to configure ciphering on cell 5. | --> | UECapabilityInformation | - | - |
| 2 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 3 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: Step 3Aa1 depend on UE implementation | - | - | - | - |
| 3Aa1 | The UE sends a ROUTING AREA UPDATE REQUEST message within [2] seconds. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a SECURITY MODE COMMAND message on Cell 5 in order to activate integrity protection. | <-- | SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 5Aa1 to 5Aa2 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE sent a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 5Aa1 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 5Aa2 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 6 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information on Cell 5. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 7 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message on Cell 5. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| - | EXCEPTION: In parallel to the events described in steps 7A to 9 and depending on the UE implementation the steps defined in Table 8.4.1.4.3.2-2 may take place. | - | - | - | - |
| 7A | IF pc_IMS = TRUE THEN SS starts 6s timer |  |  |  |  |
| - | EXCEPTION: Steps 8a1 to 8a3 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE did not send a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 8a1 | The UE sends a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 8a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 9 | IF pc_IMS = TRUE THEN SS waits until the 6s timer expires |  |  |  |  |
| - | Note : At the end of the test procedure sequence, the UE is in end state UTRA connected (U2) according to 36.508 | - | - | - | - |

Table 8.4.1.4.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 [35] takes place. | - | - | - | - |

8.4.1.4.3.3	Specific message contents

Table 8.4.1.4.3.3-1: MobilityFromEUTRACommand (step 2, Table 8.4.1.4.3.2-1)

| Derivation Path: 36.508t able 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.4.3.3-2: HANDOVER TO UTRAN COMMAND (Table 8.4.1.4.3.3-1)

Derivation Path: 36.508 table 4.7B.1-1, condition UTRA HSDPA RB

Table 8.4.1.4.3.3-3: SECURITY MODE COMMAND (step 4, Table 8.4.1.4.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (SECURITY MODE COMMAND message) |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

Table 8.4.1.4.3.3-4: UTRAN MOBILITY INFORMATION (step 6, Table 8.4.1.4.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (UTRAN MOBILITY INFORMATION message) |  |
| --- | --- |
| Information Element | Value/remark |
| CN information info |  |
| - PLMN identity |  |
| - MCC | 001 |
| - MNC | 01 |
| - CN common GSM-MAP NAS system information | 00 01H |
| - CN domain information list full |  |
| - CN domain identity | PS |
| - CN domain specific NAS system information | 01 00H |
| - DRX cycle length coefficient | 7 |
| - CN domain identity | CS |
| - CN domain specific NAS system information | 1E 01H |
| - DRX cycle length coefficient | 7 |

Table 8.4.1.4.3.3-5: UECapabilityEnquiry (step 1A, Table 8.4.1.4.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entry |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.4.3.3-6: ROUTING AREA UPDATE ACCEPT (step 5Aa1, step 8a2 Table 8.4.1.4.3.2-2)

| Derivation Path: 36.508 Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update result | 0 ‘Follow-on proceed’ |  |  |

#### 8.4.1.5	Inter-RAT Handover / from E-UTRA to UTRA(HSUPA/HSDPA) / Data

8.4.1.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and a E-DCH and HS-DSCH PS RAB combination is configured for an UTRA cell}

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the utra cell}

}

8.4.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

8.4.1.5.3	Test description

8.4.1.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE is previously registered on cell 5.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.1.5.3.2	Test procedure sequence

Table 8.4.1.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures UTRA cell 5 to reference configuration according 36.508 table 4.8.3-1, condition UTRA HSUPA/HSDPA RB. | - | - | - | - |
| 1A | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 1B | The UE transmit a UECapabilityInformation message on Cell 1. NOTE: The start-PS values received, should be used to configure ciphering on cell 5. | --> | UECapabilityInformation | - | - |
| 2 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 3 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: Step 3Aa1 depend on UE implementation | - | - | - | - |
| 3Aa1 | The UE sends a ROUTING AREA UPDATE REQUEST message within [2] seconds. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a SECURITY MODE COMMAND message on Cell 5 in order to activate integrity protection. | <-- | SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 5Aa1 to 5Aa2 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE sent a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 5Aa1 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 5Aa2 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 6 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information on Cell 5. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 7 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message on Cell 5. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| - | EXCEPTION: In parallel to the events described in steps 7A to 9 and depending on the UE implementation the steps defined in Table 8.4.1.5.3.2-2 may take place. | - | - | - | - |
| - | EXCEPTION: Steps 8a1 to 8a3 depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the UE did not send a ROUTING AREA UPDATE REQUEST message at step 3a | - | - | - | - |
| 7A | IF pc_IMS = TRUE THEN SS starts 6s timer |  |  |  |  |
| 8a1 | The UE sends a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 8a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 9 | IF pc_IMS = TRUE THEN SS waits until the 6s timer expires |  |  |  |  |
| - | Note : At the end of the test procedure sequence, the UE is in end state UTRA connected (U2) according to 36.508 | - | - | - | - |

Table 8.4.1.5.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 [35] takes place. | - | - | - | - |

8.4.1.5.3.3	Specific message contents

Table 8.4.1.5.3.3-1: MobilityFromEUTRACommand (step 2, Table 8.4.1.5.3.2-1)

| Derivation Path: 36.508t able 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.5.3.3-2: HANDOVER TO UTRAN COMMAND (Table 8.4.1.5.3.3-1)

Derivation Path: 36.508 table 4.7B.1-1, condition UTRA HSUPA/HSDPA RB

Table 8.4.1.5.3.3-3: SECURITY MODE COMMAND (step 4, Table 8.4.1.5.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (SECURITY MODE COMMAND message) |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

Table 8.4.1.5.3.3-4: UTRAN MOBILITY INFORMATION (step 6, Table 8.4.1.5.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (UTRAN MOBILITY INFORMATION message) |  |
| --- | --- |
| Information Element | Value/remark |
| CN information info |  |
| - PLMN identity |  |
| - MCC | 001 |
| - MNC | 01 |
| - CN common GSM-MAP NAS system information | 00 01H |
| - CN domain information list full |  |
| - CN domain identity | PS |
| - CN domain specific NAS system information | 01 00H |
| - DRX cycle length coefficient | 7 |
| - CN domain identity | CS |
| - CN domain specific NAS system information | 1E 01H |
| - DRX cycle length coefficient | 7 |

Table 8.4.1.5.3.3-5: UECapabilityEnquiry (step 1A, Table 8.4.1.5.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entry |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.5.3.3-6: ROUTING AREA UPDATE ACCEPT (step 5Aa1, step 8a2 8.4.1.5.3.2-2)

| Derivation Path: 36.508 Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update result | 0 ‘Follow-on proceed’ |  |  |

### 8.4.2	Inter-RAT handover UTRA to E-UTRA

#### 8.4.2.1	Void

#### 8.4.2.2	Inter-RAT handover / From UTRA PS to E-UTRA / Data

8.4.2.2.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_DCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state }

}

8.4.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.2.3.

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target cell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the configuration that do not require the UE to know the SFN of the target cell;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

8.4.2.2.3	Test description

8.4.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

8.4.2.2.3.2	Test procedure sequence

Table 8.4.2.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.2.3.2-2.

Table 8.4.2.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | - | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.4.2.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 8.4.2.2.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Ba1 to 2Ba1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1-2Aa3Aa6 | Void | - | - | - | - |
| 2Ba1 | IF pc_IMS = TRUE THEN SS starts 6s timer |  |  |  |  |
| 2Ba1a1-2Ba1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed |  |  |  |  |
| 2Ba1a4 | SS releases RRC Connection |  |  |  |  |
| 2Ba1a5 | Stop 6s timer |  |  |  |  |
| 2Ba1b1 | The 6s timer expires |  |  |  |  |
| 3-4 | Void | - | - | - | - |
| 4A-4E | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 5a1 to 5a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto step8. | - | - | - | - |
| 5a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 5a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 6-7 | Void | - | - | - | - |
| 8 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 9 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.4.2.2.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 11 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using the security key derived from the new KeNB? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 12A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| 13-19 | Void | - | - | - | - |
|  | EXCEPTION: Steps 19Aa1 to 19Aa9 describe optional behaviour that depend on the UE capability | - | - | - | - |
| 19Aa1-19Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a2a1-10a2b1 is performed |  |  |  |  |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.2.3.3	Specific message contents

Table 8.4.2.2.3.3-1: Void

Table 8.4.2.2.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[1] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.2.3.3-3: Void

Table 8.4.2.2.3.3-4: Void

Table 8.4.2.2.3.3-5: HANDOVER FROM UTRAN COMMAND (step 11, Table 8.4.2.2.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.2.3.3-6: RRCConnectionReconfiguration (Table 8.4.2.2.3.3-5)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.4.2.2.3.3-7: MobilityControlInfo (Table 8.4.2.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.4.2.2.3.3-8: SecurityConfigHO (Table 8.4.2.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.2.3.3-9: MEASUREMENT CONTROL (step 8, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -80 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.4.2.2.3.3-10: System Information Block type 19 for Cell 5 (preamble, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| Priority | 5 | higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.2.3	Void

#### 8.4.2.4	Inter-RAT handover / From UTRA HSPA to E-UTRA / Data

8.4.2.4.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_HS-DSCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state }

}

8.4.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.2.3.

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target cell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the configuration that do not require the UE to know the SFN of the target cell;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

8.4.2.4.3	Test description

8.4.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

8.4.2.4.3.2	Test procedure sequence

Table 8.4.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.4.3.2-2.

Table 8.4.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | - | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH Ec(UTRA LCR TDD) | dBm/1.28 MHz | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.4.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 8.4.2.4.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Aa1 to 2Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 2Aa1a1-2Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 2Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 2Aa1a5 | Stop 6s timer | - | - | - | - |
| 2Aa1b1 | The 6s timer expires | - | - | - | - |
| 3-4 | Void | - | - | - | - |
| 4A-4E | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA HSDPA RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | UTRAN FDD: EXCEPTION: Steps 5a1 to 5a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. UTRAN TDD : go to step 8 | - | - | - | - |
| 5a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 5a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 6-7 | Void | - | - | - | - |
| 8 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 9 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.4.2.4.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 11 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using the security key derived from the new KeNB? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 12A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| 13-19 | Void | - | - | - | - |
|  | EXCEPTION: Steps 19Aa1 to 19Aa9 describe optional behaviour that depend on the UE capability | - | - | - | - |
| 19Aa1-19Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a2a1-10a2b1 is performed |  |  |  |  |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.4.3.3	Specific message contents

Table 8.4.2.4.3.3-1: Void

Table 8.4.2.4.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.4.3.3-3: Void

Table 8.4.2.4.3.3-4: Void

Table 8.4.2.4.3.3-5: HANDOVER FROM UTRAN COMMAND (step 11, Table 8.4.2.4.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.4.3.3-6: RRCConnectionReconfiguration (Table 8.4.2.4.3.3-5)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.4.2.4.3.3-7: MobilityControlInfo (Table 8.4.2.4.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.4.2.4.3.3-8: SecurityConfigHO (Table 8.4.2.4.3.3-5)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 set to 11223344. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are set to the NAS key set identifier of the current security context. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value and are arbitrarily selected. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.4.3.3-9: PHYSICAL CHANNEL RECONFIGURATION (step 5a1, Table 8.4.2.4.3.2-2)

| Derivation Path: 34.108 clause 9.1.1 (PHYSICAL CHANNEL RECONFIGURATION message) |  |
| --- | --- |
| Information Element | Value/remark |
| - CHOICE mode | FDD |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | deactivate |
| - TGCFN | Not Present |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | Infinity |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | Not Present |
| - TGD | 270 |
| - TGPL1 | 12 |
| - TGPL2 | Not Present |
| - RPP | mode 0 |
| - ITP | mode 0 |
| - CHOICE UL/DL Mode | UL and DL, UL only, or DL only, depending on UE capability |
| - Downlink compressed mode method | SF/2 or Not present depending on UE capability |
| - Uplink compressed mode method | Higher Layer Scheduling or Not Present depending on UE capability |
| - Downlink frame type | B |
| - DeltaSIR1 | 20 (2.0) |
| - DeltaSIRAfter1 | 10 (1.0) |
| - DeltaSIR2 | Not Present |
| - DeltaSIRAfter2 | Not Present |
| - N identify abort | Not Present |
| - T Reconfirm abort | Not Present |

Table 8.4.2.4.3.3-10: System Information Block type 19 for Cell 5 (preamble, Table 8.4.2.4.3.2-2)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| Priority | 5 | higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.2.5	Void

#### 8.4.2.6	Void

#### 8.4.2.7	CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition

##### 8.4.2.7.1	CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Intra-band Contiguous CA

8.4.2.7.1.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_HS-DSCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message with RRCConnectionReconfiguration including the sCellToAddModList }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state with both PCell and SCell }

}

8.4.2.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: Reference TS 36.331 clause 5.4.2.3, and 5.3.10.3b]

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target PCell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target PCell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition as specified in 5.3.10.3b;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication configuration in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	if the RRCConnectionReconfiguration message does not include rlf-TimersAndConstants set to setup:

2>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 1:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.4.2.7.1.3	Test description

8.4.2.7.1.3.1	Pre-test conditions

System Simulator:

-	Cell 5 is UTRAN Cell

-	Cell 1 is the PCell, Cell 3 is SCell to be added

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 9 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

8.4.2.7.1.3.2	Test procedure sequence

Table 8.4.2.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.7.1.3.2-2.

Table 8.4.2.7.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | -90 |  | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | - | -65 |  |
|  | PCCPCH Ec(UTRA LCR TDD) | dBm/1.28 MHz | - | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -85 |  |

Table 8.4.2.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 5, Cell 1 and Cell 3 level according to the row "T1" in table 8.4.2.7.1.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Aa1 to 2Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 2Aa1a1-2Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 2Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 2Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 2Aa1b1 | The 6s timer expires | - | - | - | - |
| 3-7 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA HSDPA RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | EXCEPTION: Steps 8a1 to 8a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 8a1 | IF pc_FDD AND pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 8a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 9 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 10 | The SS changes Cell 5, Cell 1 and Cell 3 level according to the row "T2" in table 8.4.2.7.1.3.2-1. | - | - | - | - |
| 11 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 12 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the successful completion of the handover and SCell addition of Cell 3? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 13A – 13H | Step 1 to 8 of test procedure in TS36.508 Table 6.4.2.10-1 is performed on Cell1 | - | - | - | - |
| - | EXCEPTION: Steps 13Ia1 to 13Ia2b1 describe optional behaviour if UE sends IMS deregistration at steps 2Aa1 to 2Aa1b1 | - | - | - | - |
| 13Ia1-13Ia2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.7.1.3.3	Specific message contents

Table 8.4.2.7.1.3.3-0: Conditions for specific message contents
in Tables 8.4.2.7.1.3.3-5 and 8.4.2.7.1.3.3-7

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.4.2.7.1.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[1] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.7.1.3.3-2: MEASUREMENT CONTROL (step 9, Table 8.4.2.7.1.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -55 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.4.2.7.1.3.3-3: HANDOVER FROM UTRAN COMMAND (step 12, Table 8.4.2.7.1.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.7.1.3.3-4: RRCConnectionReconfiguration (Table 8.4.2.7.1.3.3-3)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0) and SCell_AddMod

Table 8.4.2.7.1.3.3-5: MobilityControlInfo (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.4.2.7.1.3.3-6: SecurityConfigHO (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.7.1.3.3-7: SCellToAddMod-r10-f2 (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10 | 1 |  |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  |  | Band > 64 |
| } |  |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  |  | Band > 64 |
| } |  |  |  |  |  |

Table 8.4.2.7.1.3.3-8: RadioResourceConfigCommonSCell-r10-f2 (Table 8.4.2.7.1.3.3-7)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.4.2.7.2	CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Inter-band CA

The scope and description of the present TC is the same as test case 8.4.2.7.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

##### 8.4.2.7.3	CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.4.2.7.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

### 8.4.3	Inter-RAT mobility E-UTRA to GERAN

#### 8.4.3.1	Inter-RAT handover / From E-UTRA to GPRS / PS HO

8.4.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA (data) RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message including radio resources that have been allocated for the UE in the target cell }

then { UE successfully completes the handover and leaves the RRC_CONNECTED, enter the GPRS_Packet_Transfer_Mode}

}

8.4.3.1.2	Conformance requirements [D]

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.4.3.3, 5.4.3.4.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

3>	if the targetRAT-Type is set to ‘geran’:

4>	use the contents of systemInformation, if provided for PS Handover, as the system information to begin access on the target GERAN cell;

NOTE 1:	If there are DRBs for which no radio bearers are established in the target RAT as indicated in the targetRAT-MessageContainer in the message, the E-UTRA RRC part of the UE does not indicate the release of the concerned DRBs to the upper layers. Upper layers may derive which bearers are not established from information received from the AS of the target RAT.

[TS 36.331, clause 5.4.3.4]

Upon successfully completing the handover or the cell change order, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’;

1>	stop timer T304, if running;

8.4.3.1.3	Test description

8.4.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1and Cell 24 - Cell 1 is an E-UTRAN cell, Cell 24 is a GERAN cell.

-	All cells belong to the same PLMN.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	UE is in state3 (Generic RB Established) in cell 1 as specified in clause 4.5.3 of TS 36.508.

8.4.3.1.3.2	Test procedure sequence

Table 8.4.3.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial condition after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.3.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - |
|  | RSSI | dBm | - | [-85] |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | - |
|  | RSSI | dBm | - | [-65] |  |

Table 8.4.3.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures cell 1 and GERAN cell 24 according to the row “T1” in Table 8.4.3.1.3.2-1 | - | - | - | - |
| 2 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 3 | Check: Does the UE transmit a PS HANDOVER ACCESS message on cell 24? | --> | PS HANDOVER ACCESS | 1 | P |

8.4.3.1.3.3	Specific message contents

Table 8.4.3.1.3.3-1: MobilityFromEUTRACommand (step 2, Table 8.4.1.2.3.2-1)

| Derivation Path: 36.508 table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| CS-FallbackIndicator | false | Applies only for Rel.9 |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | geran |  |  |
| targetRAT-MessageContainer | PS HANDOVER COMMAND |  |  |
| nas-SecurityParamFromEUTRA |  |  | UTRAGERAN |
| systemInformation | PSI-GERAN |  | PS HO |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.3.1.3.3-2: PS HANDOVER COMMAND (Table 8.4.3.1.3.3-1)

| Derivation Path: TS 36.508, Table 4.7D.1-1: PS HANDOVER COMMAND |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS Container for PS Handover | Present |  |  |

Table 8.4.3.1.3.3-3: NAS Container for PS Handover (Table 8.4.3.1.3.3-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| NAS_CONTAINER_LENGTH | Present | Set accordingly |  |
| Type of ciphering algorithm | Present |  |  |
| old XID | Present | Reset with the old XID parameters |  |
| IOV-UI value | ‘00000000’O |  |  |

#### 8.4.3.2	Inter-RAT cell change order / From E-UTRA data RRC_CONNECTED to GPRS / Without NACC

8.4.3.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA (data) RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message including information facilitating access of the target cell }

then { UE successfully completing the cell change order and leaves the RRC_CONNECTED,enter the GPRS_Paccket_Idle}

}

8.4.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.4.3.3, 5.4.3.4.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

3>	if the targetRAT-Type is set to ‘geran’:

4>	use the contents of systemInformation, if provided for PS Handover, as the system information to begin access on the target GERAN cell;

NOTE 1:	If there are DRBs for which no radio bearers are established in the target RAT as indicated in the targetRAT-MessageContainer in the message, the E-UTRA RRC part of the UE does not indicate the release of the concerned DRBs to the upper layers. Upper layers may derive which bearers are not established from information received from the AS of the target RAT.

2>	else if the targetRAT-Type is set to ‘cdma2000-1XRTT’ or ‘cdma2000-HRPD’:

3>	forward the targetRAT-Type and the targetRAT-MessageContainer to the CDMA2000 upper layers for the UE to access the cell indicated in the inter-RAT message in accordance with the specifications of the CDMA2000 target-RAT;

1>	else if the MobilityFromEUTRACommand message includes the purpose set to ‘cellChangeOrder’:

2>	start timer T304 with the timer value set to t304, as included in the MobilityFromEUTRACommand message;

2>	if the targetRAT-Type is set to ‘geran’:

3>	if networkControlOrder is included in the MobilityFromEUTRACommand message:

4>	apply the value as specified in TS 44.060 [36];

3>	else:

4>	acquire networkControlOrder and apply the value as specified in TS 44.060 [36];

3>	use the contents of systemInformation, if provided, as the system information to begin access on the target GERAN cell;

NOTE 2:	The systemInformation is constructed in the same way as in 2G to 2G NACC, i.e. the PSI messages are encoded as such, whereas the SI messages exclude 2 octets of headers, see TS 44.060[36].

2>	establish the connection to the target cell indicated in the CellChangeOrder;

NOTE 3:	The criteria for success or failure of the cell change order to GERAN are specified in TS 44.060[36].

[TS 36.331, clause 5.4.3.4]

Upon successfully completing the handover or the cell change order, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’;

1>	stop timer T304, if running;

8.4.3.2.3	Test description

8.4.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1and Cell 24 - Cell 1 is an E-UTRAN cell, Cell 24 is a GERAN cell.

-	All cells belong to the same PLMN.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	UE is in state3 (Generic RB Established) in cell 1 as specified in clause 4.5.3 of TS 36.508.

8.4.3.2.3.2	Test procedure sequence

Table 8.4.3.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial condition after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.3.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - |
|  | RSSI | dBm | - | -85 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | - |
|  | RSSI | dBm | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power levels are such that reselection back to cell 1 should not occur |

Table 8.4.3.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of inter RAT measurement on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1and Cell 24 parameters according to the row "T1" in table 8.4.3.2.3.2-1. | - | - | - | - |
| 4 | The UE transmit a MeasurementReport message to report the event B2 for Cell 24. | --> | MeasurementReport | - | - |
| 5 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 5A | The UE transmits a CHANNEL REQUEST message on Cell24. | --> | CHANNEL REQUEST | - | - |
| 5B | The SS changes cell 1 power level according to the row “T2” | - | - | - | - |
| 6-15 | Check: Does the test result of steps 2-11 in generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | P |

8.4.3.2.3.3	Specific message contents

Table 8.4.3.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.3.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.4.3.2.3.3-2: MeasConfig (step 1, Table 8.4.3.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| measObject[2] | MeasObjectGERAN-GENERIC(f11) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN(-69, -79) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigGERAN SEQUENCE { |  |  |  |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.4.3.2.3.3-3: MobilityFromEUTRACommand (step 5, Table 8.4.3.2.3.2-2)

| Derivation Path: 36.508 table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| mobilityFromEUTRACommand-r8::=SEQUENCE { |  |  |  |  |
| csFallbackIndicator |  | False |  |  |
| purpose CHOICE { |  |  |  |  |
| cellChangeOrder::= SEQUENCE { |  |  |  |  |
| t304 |  | ms8000 |  |  |
| targetRAT-Type CHOICE{ |  |  |  |  |
| geran::= SEQUENCE { |  |  |  |  |
| physCellId |  | 0001H |  |  |
| carrierFreq ::= SEQUENCE { |  |  |  |  |
| arfcn |  | Downlink ARFCN of Cell 24 |  |  |
| bandIndicator |  | The same band indicator of the Cell 24 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.4.3.3	Inter-RAT cell change order / From E-UTRA data to GPRS / With NACC

8.4.3.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA (data) RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message including information facilitating access of the target cell }

then { UE successfully completing the cell change order and leaves the RRC_CONNECTED,enter the GPRS_Paccket_Idle}

}

8.4.3.3.2	Conformance requirements[D]

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.4.3.3, 5.4.3.4.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

3>	if the targetRAT-Type is set to ‘geran’:

4>	use the contents of systemInformation, if provided for PS Handover, as the system information to begin access on the target GERAN cell;

NOTE 1:	If there are DRBs for which no radio bearers are established in the target RAT as indicated in the targetRAT-MessageContainer in the message, the E-UTRA RRC part of the UE does not indicate the release of the concerned DRBs to the upper layers. Upper layers may derive which bearers are not established from information received from the AS of the target RAT.

2>	else if the targetRAT-Type is set to ‘cdma2000-1XRTT’ or ‘cdma2000-HRPD’:

3>	forward the targetRAT-Type and the targetRAT-MessageContainer to the CDMA2000 upper layers for the UE to access the cell indicated in the inter-RAT message in accordance with the specifications of the CDMA2000 target-RAT;

1>	else if the MobilityFromEUTRACommand message includes the purpose set to ‘cellChangeOrder’:

2>	start timer T304 with the timer value set to t304, as included in the MobilityFromEUTRACommand message;

2>	if the targetRAT-Type is set to ‘geran’:

3>	if networkControlOrder is included in the MobilityFromEUTRACommand message:

4>	apply the value as specified in TS 44.060 [36];

3>	else:

4>	acquire networkControlOrder and apply the value as specified in TS 44.060 [36];

3>	use the contents of systemInformation, if provided, as the system information to begin access on the target GERAN cell;

NOTE 2:	The systemInformation is constructed in the same way as in 2G to 2G NACC, i.e. the PSI messages are encoded as such, whereas the SI messages exclude 2 octets of headers, see TS 44.060[36].

2>	establish the connection to the target cell indicated in the CellChangeOrder;

NOTE 3:	The criteria for success or failure of the cell change order to GERAN are specified in TS 44.060[36].

[TS 36.331, clause 5.4.3.4]

Upon successfully completing the handover or the cell change order, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’;

1>	stop timer T304, if running;

8.4.3.3.3	Test description

8.4.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1and Cell 24 - Cell 1 is an E-UTRAN cell, Cell 24 is a GERAN cell.

-	All cells belong to the same PLMN.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	UE is in state3 (Generic RB Established)in cell 1 as specified in clause 4.5.3 of TS 36.508.

8.4.3.3.3.2	Test procedure sequence

Table 8.4.3.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.3.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - |
|  | RSSI | dBm | - | [-85] |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | - |
|  | RSSI | dBm | - | [-65] |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power levels are such that reselection back to cell 1 should not occur |

Table 8.4.3.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of inter RAT measurement on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1and Cell 24 parameters according to the row "T1" in table 8.4.3.3.3.2-1. |  |  | - | - |
| 4 | The UE transmit a MeasurementReport message to report the event B2 for Cell 24. | --> | MeasurementReport | - | - |
| 5 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 5A | Check: Does the UE send a CHANNEL REQUEST message on the cell(s) specified in the test case? | --> | CHANNEL REQUEST | 1 | P |
| 5AA | The SS changes cell 1 power level according to the row “T2” | - | - | - | - |
| 5B | An uplink TBF is established in order to allow the UE to transmit a ROUTING AREA UPDATE REQUEST message signalling. | - | - | - | - |
| 5C | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATING REQUEST | - | - |
| 5D | the UE transmits a PACKET SI STATUS message | --> | PACKET SI STATUS |  |  |
| 6 -13 | The reminder (steps 4 – 11) of generic test procedure in TS 36.508 subclause 6.4.2.9 is performed. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - |  |  |

8.4.3.3.3.3	Specific message contents

Table 8.4.3.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.3.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.4.3.3.3.3-2: MeasConfig (step 1, Table 8.4.3.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| measObject[2] | MeasObjectGERAN-GENERIC(f11) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN(-69, [-79]) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigGERAN SEQUENCE { |  |  |  |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.4.3.3.3.3-3: MobilityFromEUTRACommand (step 5, Table 8.4.3.3.3.2-2)

| Derivation Path: 36.508 table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| mobilityFromEUTRACommand-r8::=SEQUENCE { |  |  |  |  |
| csFallbackIndicator |  | False |  |  |
| purpose CHOICE { |  |  |  |  |
| cellChangeOrder::= SEQUENCE { |  |  |  |  |
| t304 |  | ms2000 |  |  |
| targetRAT-Type CHOICE{ |  |  |  |  |
| geran::= SEQUENCE { |  |  |  |  |
| physCellId |  | 0001H |  |  |
| carrierFreq ::= SEQUENCE { |  |  |  |  |
| arfcn |  | Downlink ARFCN of Cell 24 |  |  |
| bandIndicator |  | The same band indicator of the Cell 24 |  |  |
| } |  |  |  |  |
| networkControlOrder |  | 00 | NC0: MS controlled cell re-selection, no measurement reporting |  |
| SI-OrPSI-GERAN::= CHOICE{ |  |  |  | PS HO |
| SystemInfoListGERAN ::= SEQUENCE{ |  |  |  |  |
| SYSTEM INFORMATION TYPE 1 |  | Set according to clause 4.4.5 in TS 36.508. |  |  |
| SYSTEM INFORMATION TYPE 3 |  | Set according to clause 4.4.5 in TS 36.508. |  |  |
| SYSTEM INFORMATION TYPE 13 |  | Set according to clause 4.4.5 in TS 36.508. |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.4.3.3.3.3-4: System Information 13 (Preamble onwards)

| Derivation Path: 51.010, Clause 40.2.1.1.1 |  |
| --- | --- |
| SI 13 Rest Octets - SI_STATUS_IND bit | 1 PACKET SI STATUS message supported |

Table 8.4.3.3.3.3-5: Packet SI Status

| Information element | Value/remark |
| --- | --- |
| < GLOBAL_TFI : Global TFI IE > | Present, any Value |
| < BCCH_CHANGE_MARK : bit (3) > | Present, any Value |
| < Received SI Message List : { 1 < SI_MESSAGE_TYPE : bit (8) > { < MESS_REC : bit (2) } SI_MESSAGE_TYPE : bit (8) > { < MESS_REC : bit (2) } } ** 0 < ADDITIONAL_MSG_TYPE : bit > ; | 1 00011001 (SI1) 01 ( Message type supported and received, single instance) 1 00011011 (SI3) 1 Not Checked, The UE may include additional information Present, any Value |
| < Received Unknown SI Message List : { 1 < SI_MESSAGE_TYPE : bit (8) > } ** 0 < ADDITIONAL_MSG_TYPE : bit > ; | 0 (not present) Present, any Value |
| ¦ 1 -- Additions for REL-6 : < PSCSI_SUPPORT : bit > < PS_REL_REQ : bit > | Not Checked Not Checked Not Checked |

### 8.4.4	Void

#### 8.4.4.1	Void

#### 8.4.4.2	Void

#### 8.4.4.3	Void

### 8.4.5	Inter-RAT handover E-UTRA to HRPD

#### 8.4.5.1	Void

#### 8.4.5.2	Void

#### 8.4.5.3	Void

#### 8.4.5.4	Pre-registration at HRPD and inter-RAT handover / From E-UTRA to HRPD Active / Data

8.4.5.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and UE has performed pre-registration on HRPD neighbour cell }

ensure that {

when { UE receives a HandoverFromEUTRAPreparationRequest message with cdma2000-type set to ‘HRPD’ }

then { UE transmits a ULHandoverPreparationTransfer message containing tunnelled HRPD ConnectionRequest and RouteUpdate messages }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has performed pre-registration on HRPD neighbour cell }

ensure that {

when { UE receives a MobilityFromEUTRACommand message containing tunnelled HRPD TrafficChannelAssignment, HRPDSilenceParameters and HRPDOpenLoopParameters messages }

then { UE transmits a TrafficChannelComplete message on the target HRPD cell }

}

8.4.5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.4.3.3, 5.4.4.3, 5.4.5.1 and 5.4.5.3, TS 23.402, clause 9.3.2 and 3GPP2 X.S0057- B v1.0, clause 13.1.2.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to ‘handover’:

2>	if the targetRAT-Type is set to ‘utra’ or ‘geran’:

…

2>	else if the targetRAT-Type is set to ‘cdma2000-1XRTT’ or ‘cdma2000-HRPD’:

3>	forward the targetRAT-Type and the targetRAT-MessageContainer to the CDMA2000 upper layers for the UE to access the cell indicated in the inter-RAT message in accordance with the specifications of the CDMA2000 target-RAT;

[TS 36.331, clause 5.4.4.3]

Upon reception of the HandoverFromEUTRAPreparationRequest message, the UE shall:

1>	indicate the request to prepare handover and forward the cdma2000-Type to the CDMA2000 upper layers;

1>	if cdma2000-Type is set to ‘type1XRTT’:

2>	forward the rand and the mobilityParametersCDMA2000 to the CDMA2000 upper layers;

[TS 36.331, clause 5.4.5.1]

Figure 5.4.5.1-1: UL handover preparation transfer

The purpose of this procedure is to tunnel the handover related CDMA2000 dedicated information from UE to E-UTRAN when requested by the higher layers. The procedure is triggered by the higher layers on receipt of HandoverFromEUTRAPreparationRequest message. This procedure applies to CDMA2000 capable UEs only.

[TS 36.331, clause 5.4.5.3]

The UE shall set the contents of the ULHandoverPreparationTransfer message as follows:

1>	include the cdma2000-Type and the dedicatedInfoCDMA2000;

1>	if the cdma2000-Type is set to ‘type1XRTT’:

2>	include the meid and set it to the value received from the CDMA2000 upper layers;

1>	submit the ULHandoverPreparationTransfer message to lower layers for transmission, upon which the procedure ends;

[TS 23.402, clause 9.3.2]

Figure 9.3.2-1 illustrates a high-level call flow for the optimised E-UTRAN to HRPD handover procedure, Handover phase. The prerequisite of the handover phase is the successfully performed Pre-registration phase as it is specified in clause 9.3.1.

Figure 9.3.2-1: E-UTRAN to HRPD handover

[3GPP2 X.S0057- B v1.0, clause 13.1.2]

Figure 9.3.2-2

8.4.5.4.3	Test description

8.4.5.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 15.

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3B) on Cell 1 according to [18].

-	The UE has performed HRPD pre-registration on Cell 15.

8.4.5.4.3.2	Test procedure sequence

Table 8.4.5.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.5.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power level values are such that camping on Cell 1 is guaranteed. |
|  | Îor/Ioc | dB | - | -20 |  |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -20 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | The power level values are such that entering conditions for event B2 on Cell 15 are satisfied. |
|  | Îor/Ioc | dB | - | -5 |  |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - | -6 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 8.4.5.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter RAT measurement on Cell 15. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of inter RAT measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 15 parameters according to row “T1” in table 8.4.5.4.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event B2 for Cell 15, | --> | MeasurementReport | - | - |
| 5 | The SS transmits a HandoverFromEUTRAPreparationRequest on Cell 1. | <-- | HandoverFromEUTRAPreparationRequest | - | - |
| 6 | Check: Does the UE transmit tunnelled HRPD Connection Request and Route Update messages contained in an ULHandoverPreparationTransfer message on Cell 1? | --> | ULHandoverPreparationTransfer | 1 | P |
| 7 | The SS transmits tunnelled HRPD Traffic Channel Assignment, HRPD Silence Parameters and HRPD Open Loop Parameters messages contained in a MobilityFromEUTRACommand on Cell1 to order the UE to perform inter RAT handover to Cell 15. | <-- | MobilityFromEUTRACommand | - | - |
| 8 | The UE tunes to HRPD radio. | - | - | - | - |
| 9 | Check: Does the UE transmit a Traffic Channel Complete message on Cell 15? | --> | Traffic Channel Complete | 2 | P |

8.4.5.4.3.3	Specific message contents

Table 8.4.5.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.5.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.4.5.4.3.3-2: MeasConfig (step 1, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f14 |  |  |
| measObject[2] | MeasObjectCDMA2000-GENERIC |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-CDMA2000 |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-CDMA2000(-69, -18) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f14 |  |  |
| reportConfigId[1] | IdReportConfig-B2-CDMA2000 |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigCDMA2000 SEQUENCE { |  |  |  |
| measQuantityCDMA2000 | pilotStrength |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp1 | 30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.4.5.4.3.3-3: MeasObjectCDMA2000-GENERIC (step 1, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCDMA2000-GENERIC ::= SEQUENCE { |  |  |  |
| cdma2000-Type | TypeHRPD |  |  |
| CarrierFreqCDMA2000 SEQUENCE { |  |  |  |
| bandClass | Band Class of frequency under test |  |  |
| arfcn | f14 |  |  |
| } |  |  |  |
| SearchWindowSize | 15 |  |  |
| offsetFreq | db0 |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList CHOICE {} | Cell 15 | Listed cell parameters to be reported |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

Table 8.4.5.4.3.3-4: MeasurementReport (step 4, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultsCDMA2000 :=SEQUENCE { |  |  |  |
| preRegistrationStatusHRPD | TRUE |  |  |
| measResultListCDMA2000 ::=SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry | Note 1 |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.5.4.3.3-5: HandoverFromEUTRAPreparationRequest (step 5, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| HandoverFromEUTRAPreparationRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| handoverFromEUTRAPreparationRequest-r8 SEQUENCE { |  |  |  |  |
| cdma2000-Type |  | typeHRPD |  |  |
| rand |  | Not present |  |  |
| mobilityParameters |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.4.5.4.3.3-6: ULHandoverPreparationTransfer (step 6, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULHandoverPreparationTransfer ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ulHandoverPreparationTransfer-r8 SEQUENCE { |  |  |  |
| cdma2000-Type | typeHRPD |  |  |
| meid | Not present |  |  |
| dedicatedInfo | Set according to Table 8.4.5.4.3.3-6A | HRPD Connection Request and Route Update |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.5.4.3.3-6A: dedicatedInfo in ULHandoverPreparationTransfer (step 6, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| SAPState | ‘0’B | SAP Header |  |
| SessionConfigurationToken | 16 bits, Set by UE |  |  |
| ConnectionLayerFormat | 1 bit, Set by UE |  |  |
| ATI Record | 34 bits, Set based on UATI assigned to UE |  |  |
| Reserved | ‘0000’B |  |  |
| Length | Length of HRPD Route Update message (Table 8.4.5.4.3.3-7A) + length of StreamHeader + length of SLPHeader + length of SNPHeader below, Set by SS | Connection Layer Header |  |
| StreamHeader | ‘00’B | Stream Layer header. Stream 0 is assigned to the Default Signalling Application |  |
| SLPHeader | Set by the UE | Signalling Link Protocol SLP-D and SLP-F headers. |  |
| SNPHeader | ‘00001110’ | Signalling Network Protocol header. InConfigurationProtocol=0, Type=Route Update. |  |
| SessionLayerPacket | HRPD Route Update message (Table 8.4.5.4.3.3-7A) |  |  |
| Length | Length of HRPD Connection Request message (Table 8.4.5.4.3.3-7) + length of StreamHeader + length of SLPHeader + length of SNPHeader below, Set by SS | Connection Layer Header |  |
| StreamHeader | ‘00’B | Stream Layer header. Stream 0 is assigned to the Default Signalling Application |  |
| SLPHeader | Set by the UE | Signalling Link Protocol SLP-D and SLP-F headers. |  |
| SNPHeader | ‘00001100’ | Signalling Network Protocol header. InConfigurationProtocol=0, Type=Idle State. |  |
| SessionLayerPacket | HRPD Connection Request message (Table 8.4.5.4.3.3-7) |  |  |

Table 8.4.5.4.3.3-7: HRPD Connection Request (step 6, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘0000001’ | Connection Request | this value shall be verified by TTCN |
| TransactionID | Any allowed value | 8 bit field |  |
| RequestReason | ‘0000’ | Access Terminal Initiated |  |

Table 8.4.5.4.3.3-7A: HRPD Route Update (step 6, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘0000000’B | Route Update | this value shall be verified by TTCN |
| MessageSequence | 8 bits, Set by UE |  |  |
| ReferencePilotPN | 9 bits, Set by UE |  |  |
| ReferencePilotStrength | 6 bits, Set by UE |  |  |
| ReferenceKeep | ‘1’B |  |  |
| NumPilots | ‘0000’B |  |  |
| CompatibleReserved | ‘0’B |  |  |
| ReferencePilotChannelIncluded | ‘1’B |  |  |
| ReferencePilotChannel | 24 bits, Set by UE |  |  |
| ReferencePilotArrivalIncluded | ‘1’B |  |  |
| ReferencePilotArrival | 15 bits, Set by UE |  |  |
| Reserved | 0-7 bits, Set all 0s by UE |  |  |

Table 8.4.5.4.3.3-8: MobilityFromEUTRACommand (step 7, Table 8.4.5.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| csFallbackIndicator | False |  |  |
| purpose CHOICE{ |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | cdma2000-HRPD |  |  |
| targetRAT-MessageContainer | Set according to Table 8.4.5.4.3.3-8 A | HRPD Silence Parameters and HRPD Open Loop Parameters, HRPD Traffic Channel Assignment |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.5.4.3.3-8A: targetRAT-MessageContainer in MobilityFromEUTRACommand (step 7, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| SAPState | ‘1’B | SAP Header |  |
| SessionConfigurationToken | ‘0’B |  |  |
| ConnectionLayerFormat | 1 bit, Set by SS |  |  |
| ATI Record | 34 bits, Set based on UATI assigned to UE |  |  |
| Reserved | ‘0000’B |  |  |
| Length | Length of HRPD Silence Parameters message (Table 8.4.5.4.3.3-9A) + length of StreamHeader + length of SLPHeader + length of SNPHeader below, Set by SS | Connection Layer Header (Note 1) |  |
| StreamHeader | ‘00’B | Stream Layer header. Stream 0 is assigned to the Default Signalling Application |  |
| SLPHeader | Set by the SS | Signalling Link Protocol SLP-D and SLP-F headers. |  |
| SNPHeader | ‘00001111’ | Signalling Network Protocol header. InConfigurationProtocol=0, Type=Overhead Messages. |  |
| SessionLayerPacket | HRPD Silence Parameters message (Table 8.4.5.4.3.3-9A) | (Note 1) |  |
| Length | Length of HRPD Open Loop Parameters message (Table 8.4.5.4.3.3-9B) + length of StreamHeader + length of SLPHeader + length of SNPHeader below, Set by SS | Connection Layer Header (Note 1) |  |
| StreamHeader | ‘00’B | Stream Layer header. Stream 0 is assigned to the Default Signalling Application |  |
| SLPHeader | Set by the SS | Signalling Link Protocol SLP-D and SLP-F headers. |  |
| SNPHeader | ‘00000100’ | Signalling Network Protocol header. InConfigurationProtocol=0, Type=Reverse Traffic Channel MAC. |  |
| SessionLayerPacket | HRPD Open Loop Parameters message (Table 8.4.5.4.3.3-9B) | (Note 1) |  |
| Length | Length of HRPD Traffic Channel Assignment message (Table 8.4.5.4.3.3-9) + length of StreamHeader + length of SLPHeader + length of SNPHeader below, Set by SS | Connection Layer Header |  |
| StreamHeader | ‘00’B | Stream Layer header. Stream 0 is assigned to the Default Signalling Application |  |
| SLPHeader | Set by the SS | Signalling Link Protocol SLP-D and SLP-F headers. |  |
| SNPHeader | ‘00001110’ | Signalling Network Protocol header. InConfigurationProtocol=0, Type=Route Update. |  |
| SessionLayerPacket | HRPD Traffic Channel Assignment message (Table 8.4.5.4.3.3-9) |  |  |

Note 1: HRPD Silence Parameters message and HRPD Open Loop Parameters message can be sent in any order.

Table 8.4.5.4.3.3-9: HRPD Traffic Channel Assignment (step 7, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000001’B | Traffic Channel Assignment |  |
| MessageSequence | Set by SS | 8 bit field |  |
| ChannelIncluded | ‘1’B | Channel record included |  |
| Channel | ‘000000000000000001111010’B | channel record for Cell 15 |  |
| FrameOffset | ‘1010’B | frame offset for Cell 15 |  |
| DRCLength | ‘01’B | DRCLength for Cell 15 |  |
| DRCChannelGainBase | ‘111101’B | ratio of the power level of the DRC Channel (when it is transmitted) to the power level of the Reverse Traffic Pilot Channel expressed as 2's complement value in units of 0.5 dB |  |
| ACKChannelGain | ‘000110’B | ratio of the power level of the Ack Channel (when it is transmitted) to the power level of the Reverse Traffic Pilot Channel expressed as 2's complement value in units of 0.5 dB |  |
| NumPilots | ‘1’B |  |  |
| PilotPN | ‘000110010’B | PN Offset of target sector (Cell 15) |  |
| SofterHandoff | ‘0’B | Set to ‘0’since only 1 pilot included in message |  |
| MACIndexLSBs | Set by SS | 6 least significant bits of the MACIndex assigned to UE |  |
| DRCCover | ‘001’B | index of the DRC cover associated with target sector (Cell 15) |  |
| RABLength | ‘01’B | 2 bit field |  |
| RABOffset | ‘010’B | 3 bit field |  |

Table 8.4.5.4.3.3-9A: HRPD Silence Parameters (step 7, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000010’B |  |  |
| ReverseLinkSilenceDuration | 2 bits, Set by SS |  |  |
| ReverseLinkSilencePeriod | 2 bits, Set by SS |  |  |
| Reserved | 0-7 bits, Set all 0s by SS |  |  |

Table 8.4.5.4.3.3-9B: HRPD Open Loop Parameters (step 7, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000111’B |  |  |
| NumPilots | ‘0001’B |  |  |
| PilotPN | 9 bits, Set by SS |  |  |
| OpenLoopAdjust | 8 bits, Set by SS |  |  |
| InitialAdjust | 5 bits, Set by SS |  |  |
| PilotStrengthIncluded | 1 bit, Set by SS |  |  |
| PilotStrengthNominal | 3 bits, Set by SS |  |  |
| PilotStrengthCorrectionMin | 3 bits, Set by SS |  |  |
| PilotStrengthCorrectionMax | 3 bits, Set by SS |  |  |
| Reserved | 0-7 bits, Set all 0s by SS |  |  |

Table 8.4.5.4.3.3-10: HRPD Traffic Channel Complete (step 9, Table 8.4.5.4.3.2-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000010’ |  | this value shall be verified by TTCN |
| MessageSequence | Same value as MessageSequence in HRPD Traffic Channel Assignment message (Table 8.4.5.4.3.3-9) |  |  |

### 8.4.6	Inter-RAT handover HRPD to E-UTRA

### 8.4.7	Inter-RAT mobility E-UTRA to 1xRTT

#### 8.4.7.1	Void

#### 8.4.7.3	Void

#### 8.4.7.4	Void

#### 8.4.7.5	Void

#### 8.4.7.6	Void

#### 8.4.7.7	Void

#### 8.4.7.8	Void

#### 8.4.7.9	Void

#### 8.4.7.10	Void

### 8.4.8	WLAN Offload EUTRA RRC Connected

#### 8.4.8.1	WLAN Offload / Offload Success / EUTRA RRC_Connected to/from WLAN (Qrxlevmeas, BackhaulRateUlWLAN)

8.4.8.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing wlan-OffloadConfigDedicated with thresholdRSRP-Low, thresholdRSRP-High, thresholdBackhaulUL-BandwidthLow and thresholdBackhaulUL-BandwidthHigh }

then { UE configures dedicated WLAN offload and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { RSRPmeas < ThreshServingOffloadWLAN, LowP and BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High }

then { UE offload PDN connections to WLAN }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { RSRPmeas > ThreshServingOffloadWLAN, HighP and BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low }

then { UE offload PDN connections to 3GPP }

}

8.4.8.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.6.12.2, 5.6.12.3, TS 36.304, clause 5.6.2 and TS 24.302, clause 6.10.4.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadInfo is set to release:

2> release wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

1>	else:

2>	apply the received wlan-OffloadConfigDedicated:

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304;

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only.

[TS 24.302, clause 6.10.4]

This subclause applies if the RAN rules control the WLAN access selection and traffic routing as described in subclause 6.10.2.

The access stratum layer of the 3GPP access can provide:

1)	move-traffic-to-WLAN indication, along with list of WLAN identifiers. An entry in the list of the WLAN identifiers consists of SSID, BSSID, HESSID, or any combination of them; and

2)	move-traffic-from-WLAN indication.

The user preferences take precedence over the indications provided by the access stratum layer of the 3GPP access.

Upon:

-	receiving move-traffic-to-WLAN indication, along with the list of the WLAN identifiers, if the user preferences are not present; or

-	establishment of a new PDN connection in 3GPP access, if the PDN connection is an offloadable PDN connection, the access stratum indicated move-traffic-to-WLAN, the access stratum has not indicated the move-traffic-from-WLAN indication after indicating of the move-traffic-to-WLAN indication and the user preferences are not present;

and:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and all the PDN connections of the UE in 3GPP access are offloadable PDN connections;

the UE:

a)	shall perform the procedure in subclause 5.1.3.2.3 and in subclause 5.2.3.2 to select the selected WLAN and the NAI for authentication;

b)	if not authenticated yet with the selected WLAN using the NAI for authentication in subclause 6.4, shall authenticate with the selected WLAN using the NAI for authentication in subclause 6.4. During authentication, if the selected WLAN is a trusted WLAN, SCM is supported by both UE and network, MCM is not supported by UE, network or both, and if:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and the UE has only one PDN connection;

shall handover one offloadable PDN connection from 3GPP access to the WLAN access using procedures in subclause 6.4.2.6.2;

NOTE:	When the UE already has one PDN connection established via WLAN in SCM, and if move-traffic-to-WLAN indication is received, it is up to the UE implementation to determine whether to offload a PDN connection from 3GPP access to WLAN. In that case, it is also up to the UE implementation to determine which one of the offloadable PDN connections will be offloaded.

c)	if the selected WLAN is a trusted WLAN, and MCM is supported by both UE and network, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures of 3GPP TS 24.244];

d)	if the selected WLAN is an untrusted WLAN, and if the UE supports access to EPC via untrusted WLAN, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures in subclause 7.2.1 and subclause 7.2.2; and

e)	if the UE has a valid IARP rule for APN, shall use the IARP for APN using the procedures in subclause 6.8.2.2.4.5.

Upon receiving move-traffic-from-WLAN indication, and if the user preferences are not present, the UE shall handover all the PDN connections established in (or previously handed over to) WLAN access to the 3GPP access using procedures in 3GPP TS 24.301.

8.4.8.1.3	Test Description

8.4.8.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	WLAN.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

8.4.8.1.3.2	Test procedure sequence

Table 8.4.8.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while the configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 8.4.8.1.3.2-2.

Table 8.4.8.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | WLAN | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | Off | The power level values are such that camping on Cell 1 is guarantee. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
|  | BackhaulRateDLWLAN | kbps | - | r8192 |  |
|  | BackhaulRateULWLAN | kbps | - | r16384 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -79 |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy the leave condition for event A3 (M3 < M1). |
|  | BeaconRSSI | dBm | - | off |  |

Table 8.4.8.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup WLAN offload configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS changes WLAN parameters according to the row "T1" in table 8.4.8.1.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.3.10 indicate that the PDN connections are offloaded from 3GPP access to the WLAN? | - | - | 2 | P |
| 5 | The SS changes WLAN parameters according to the row "T2" in table 8.4.8.1.3.2-1. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.3.11 indicate that the PDN connections are offloaded from the WLAN access to 3GPP? | - | - | 3 | P |

8.4.8.1.3.3	Specific message contents

Table 8.4.8.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.8.1.3.2-2)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition WLAN-OffloadSetup

Table 8.4.8.1.3.3-2: wlan-OffloadInfo-r12 (step 1, Table 8.4.8.1.3.2-2)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-30, condition RSRP and BackhaulBW |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 50 |  |  |
| thresholdRSRP-High-r12 | 50 |  |  |
| } |  |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r0 |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r2048 |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r2048 |  |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r4096 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.8.2	WLAN Offload / Offload Success / EUTRA RRC_Connected to/from WLAN (Qrxlevmeas, ChannelUtilizationWLAN)

8.4.8.2.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_CONNECTED state, and UE is configured with ThreshServingOffloadWLAN, LowP, ChannelUtilizationWLAN and ThreshChUtilWLAN, Low }

ensure that {

when { UE measures that Qrxlevmeas is less than ThreshServingOffloadWLAN, LowP and ChannelUtilizationWLAN is less than ThreshChUtilWLAN, Low}

then { UE offloads to WLAN AP after TsteeringWLAN }

}

(2)

with { UE is associated with WLAN AP and attached to WLAN ePDG and configured with ThreshChUtilWLAN, High   and ChannelUtilizationWLAN }

ensure that {

when { UE measures that Qrxlevmeas is lower than ThreshServingOffloadWLAN, HighP and ChannelUtilizatioWLAN is greater than ThreshChUtilWLAN, High}

then { UE offloads to EUTRA Cell after TsteeringWLAN }

}

8.4.8.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 5.6.2, TS 36.331 clause 5.6.12.2 and 5.6.12.3.

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadInfo is set to release:

2> release wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

1>	else:

2>	apply the received wlan-OffloadConfigDedicated:

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

8.4.8.2.3	Test description

8.4.8.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN AP 1

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The UE supporting ANDSF is pre-provisioned with WLANSP ANDSF MO with at least one WLAN identifier and with ForServiceBased ANDSF MO with RANValidityCondition allowing at least one APN to use both WLAN and LTE and at least one APN to use LTE only.

-	The UE supporting RAN rules is configured with at least two PDN connections, one PDN connection offloadable to WLAN and one PDN connection not offloadable to WLAN.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.8.2.3.2	Test procedure sequence

Table 8.4.8.2.3.2-1 illustrates the downlink power levels to be applied for Cell 1and WLAN AP 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.8.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | E-UTRA Cell 1 is a suitable cell and WLAN AP Cell 27 is off. |
|  | ChannelUtilizatinWLAN | dBm | - | Off |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied. Qrxlevmeas < ThreshServingOffloadWLAN, LowP and ChannelUtilizationWLAN < ThreshChUtilWLAN, Low |
|  | ChannelUtilizatinWLAN | - | - | 120 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied. ChannelUtilizationWLAN > ThreshChUtilWLAN, High and Qrxlevmeas < ThreshServingOffloadWLAN, HighP |
|  | ChannelUtilizatinWLAN | dBm | - | Off |  |

Table 8.4.8.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup WLAN offload configurations on Cell 1 with WLAN offload thresholds. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal levels according to row “T1” in Table 8.4. 8.2.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE perform Offload to WLAN procedure in TS 36.508 Table 6.4.3.10-1)? | - | - | 1 | P |
| 5 | The SS changes Cell 1 and WLAN AP 1 parameters according to the row "T2" in table 8.4.8.2.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE perform Offload from WLAN procedure in TS 36.508 Table 6.4.3.11-1)? | - | - | 2 | P |

8.4.8.2.3.3	Specific message contents

Table 8.4.8.2.3.3-2: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508 table 4.6.3-30 with conditions RSRP and ChannelUtilization |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| thresholdRSRP-High-r12 | 55 | Actual value = Thres+140 => Threshold =-85 dB |  |
| } |  |  |  |
| thresholdRSRQ-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbolsWithWB-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbols-r12 | Not Present |  |  |
| thresholdRSRQ-WB-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12::= SEQUENCE { | Not Present |  |  |
| thresholdChannelUtilizationLow-r12 | 50 |  |  |
| thresholdChannelUtilizationHigh-r12 | 80 |  |  |
| } |  |  |  |
| thresholdBackhaul-Bandwidth-r12 | Not Present |  |  |
| } |  |  |  |

#### 8.4.8.3	WLAN Offload/ Offload Success/ EUTRA RRC_Connected to/from WLAN (Qqualmeas, BeaconRSSI)

8.4.8.3.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_CONNECTED state}

ensure that {

when { UE receives RRCConnectionReconfiguration including wlan-OffloadDedicated-r12 to setup WLAN offload configuration }

then { UE sends RRCConnectionReconfigurationComplete  }

}

(2)

with { UE in EUTRA RRC_CONNECTED state and configured with thresholdRSRQ-r12  and thresholdBeaconRSSI-r12 in RRCConnectionReconfiguration }

ensure that {

when { UE detects the condition for steering traffic from E-UTRA to WLAN is met for a

time interval TsteeringWLAN }

then { UE performs offload from E-UTRAN to WLAN}

}

(3)

with { UE connected in WLAN  and configured with thresholdRSRQ-r12  and thresholdBeaconRSSI-r12 }

ensure that {

when { UE detects the condition for steering traffic from WLAN to E-UTRA is met for a

time interval TsteeringWLAN }

then { UE performs offload from WLAN to E-UTRAN }

}

8.4.8.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.12.2 and 5.6.12.3 and TS 36.304 clause 5.6.2.

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadDedicated is set to release:

…

1>	else:

…

2>	if the received wlan-OffloadDedicated includes thresholdRSRQ:

3>	apply the received thresholdRSRQ;

3>	forward the received thresholdRSRQ to upper layers;

…

2>	if the received wlan-OffloadDedicated includes thresholdBeaconRSSI:

3>	apply the received thresholdBeaconRSSI;

3>	forward the received thresholdBeaconRSSI to upper layers;

…

2>	if the received wlan-OffloadDedicated includes t-SteeringWLAN:

3>	apply the received t-SteeringWLAN;

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

8.4.8.3.3	Test description

8.4.8.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 27

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.8.3.3.2	Test procedure sequence

Table 8.4.8.3.3.2-1 illustrates the downlink power levels to be applied for Cell 1and Cell 27 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.8.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | E-UTRA Cell 1 is a suitable cell and WLAN Cell 27 is off. |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | off |  |
|  | Noc | dBm/15kHz | -73 |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied. - Qqualmeas < ThreshServingOffloadWLAN, LowQ - BeaconRSSI > ThreshBeaconRSSIWLAN, High |
|  | RSRQ | dB | -13.23 | - |  |
|  | BeaconRSSI |  | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -65 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA are satisfied. - Qqualmeas < ThreshServingOffloadWLAN, HighQ - BeaconRSSI < ThreshBeaconRSSIWLAN, Low |
|  | RSRQ | dB | -5 | - |  |
|  | BeaconRSSI |  | - | -86 |  |

Table 8.4.8.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup WLAN offload configurations on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS changes Cell 1 and Cell 27 parameters according to the row "T1" in table 8.4.8.3.3.2-1. | - | - | - | - |
| 4 | Wait for TsteeringWLAN expiry | - | - | - | - |
| 5 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.10 indicate that the UE has successfully offloaded to WLAN Cell 27 after TsteeringWLAN expiry? | - | - | 2 | P |
| 6 | Wait for 5 seconds | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 27 parameters according to the row "T2" in table 8.4.8.3.3.2-1 | - | - | - | - |
| 8 | Wait for TsteeringWLAN expiry | - | - | - | - |
| 9 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.11 indicate that the UE has successfully offloaded to EUTRA Cell 1 after TsteeringWLAN expiry? | - | - | 3 | P |

8.4.8.3.3.3	Specific message contents

Table 8.4.8.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.8.3.3.2-2)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition WLAN-OffloadSetup

Table 8.4.8.3.3.3-2: wlan-OffloadInfo-r12 (step 1, Table 8.4.8.3.3.2-2)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| wlan-OffloadInfo-r12::= CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| wlan-OffloadConfigDedicated-r12SEQUENCE { |  |  |  |
| thresholdRSRP-r12 | Not Present |  |  |
| thresholdRSRQ-r12 SEQUENCE { |  |  |  |
| thresholdRSRQ-Low-r12 | 17 | -11.5dB ≤ RSRQ < -11dB |  |
| thresholdRSRQ-High-r12 | 32 | -4 dB ≤ RSRQ < -3.5dB |  |
| } |  |  |  |
| thresholdBeaconRSSI-r12 SEQUENCE { |  |  |  |
| thresholdBeaconRSSI-Low-r12 | 50 | Value in dBm =-78 |  |
| thresholdBeaconRSSI-High-r12 | 60 | Value in dBm =-68 |  |
| } |  |  |  |
| t-SteeringWLAN-r12 | 7 | 7 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.8.4	WLAN Offload / Offload Success / EUTRA RRC_Connected to/from WLAN (Qqualmeas, BackhaulRateDlWLAN) / CA

8.4.8.4.1	Test Purpose (TP)

(1)

with { UE in EUTRA RRC_CONNECTED state with SCell added and configured with thresholdRSRQ-r12 and ThreshBackhRateDLWLAN in RRCConnectionReconfiguration }

ensure that {

when { UE detects the SCell Qqualmeas < ThreshServingOffloadWLAN, LowQ and BackhaulRateDlWL> ThreshBackhRateDLWLAN, High }

then { UE does not performs offload from E-UTRAN to WLAN }

}

(2)

with { UE in EUTRA RRC_CONNECTED state with SCell added and configured with thresholdRSRQ-r12 and ThreshBackhRateDLWLAN in RRCConnectionReconfiguration }

ensure that {

when { UE detects the PCell Qqualmeas < ThreshServingOffloadWLAN, LowQ and BackhaulRateDlWL> ThreshBackhRateDLWLAN, High }

then { UE performs offload from E-UTRAN to WLAN }

}

(3)

with { UE in EUTRA RRC_CONNECTED state with SCell added, a PDN offloaded to WLAN and configured with thresholdRSRQ-r12 and ThreshBackhRateDLWLAN in RRCConnectionReconfiguration }

ensure that {

when { UE detects the SCell Qqualmeas < ThreshServingOffloadWLAN, HighQ for a time interval TsteeringWLAN }

then { UE does not performs offload from WLAN to E-UTRAN }

}

(4)

with { UE in EUTRA RRC_CONNECTED state with SCell added, a PDN offloaded to WLAN and configured with thresholdRSRQ-r12 and ThreshBackhRateDLWLAN in RRCConnectionReconfiguration }

ensure that {

when { UE detects the PCell Qqualmeas < ThreshServingOffloadWLAN, HighQ for a time interval TsteeringWLAN }

then { UE does not performs offload from WLAN to E-UTRAN }

}

8.4.8.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.12.2 and 5.6.12.3 and TS 36.304 clause 5.6.2. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadDedicated is set to release:

…

1>	else:

…

2>	if the received wlan-OffloadDedicated includes thresholdRSRQ:

3>	apply the received thresholdRSRQ;

3>	forward the received thresholdRSRQ to upper layers;

…

2>	if the received wlan-OffloadDedicated includes thresholdBeaconRSSI:

3>	apply the received thresholdBeaconRSSI;

3>	forward the received thresholdBeaconRSSI to upper layers;

…

2>	if the received wlan-OffloadDedicated includes t-SteeringWLAN:

3>	apply the received t-SteeringWLAN;

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 [66] to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304 [4];

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402 [25]. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302 [28]) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

1.	In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

2.	In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302 [28]) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

3.	In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

4.	In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only.

8.4.8.4.3	Test description

8.4.8.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 (PCell), if intra band CA Cell 3(SCell) else if inter band CA Cell 10(Scell) and Cell 27

- Cell 3/Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to TS 36.508 [18].

8.4.8.4.3.2	Test procedure sequence

Table 8.4.8.4.3.2-1 illustrates the downlink power levels to be applied for Cell 1and Cell 27 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.4.8.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3/10 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | -60 | - | E-UTRA Cell 1 Cell 3/10 are suitable cell and WLAN Cell 27 is off. |
|  | RSRQ | dB | -4.15 | -4.15 | - |  |
|  | Noc | dBm/15kHz | -73 | -73 |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | -75 | - | The power level values are such that conditions SCell Qqualmeas < ThreshServingOffloadWLAN, LowQ & BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High are satisfied. Condition PCell Qqualmeas < ThreshServingOffloadWLAN, LowQ is not satisfied |
|  | RSRQ | dB | -4.15 | -13.23 | - |  |
|  | BackhaulRateDlWLAN | kbps | - | - | r16384 |  |
|  | BackhaulRateUlWLAN | kbps | - | - | r8192 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -75 | -75 | - | The power level values are such that conditions PCell Qqualmeas < ThreshServingOffloadWLAN, LowQ & BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High are satisfied. |
|  | RSRQ | dB | -13.23 | -13.23 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -75 | -65 |  | The power level values are such that condition Qqualmeas < ThreshServingOffloadWLAN, HighQ for SCell is satisfied and not satisfied for PCell |
|  | RSRQ | dB | -13.23 | -5 |  |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -65 | -75 | - | The power level values are such that condition Qqualmeas < ThreshServingOffloadWLAN, HighQ for PCell is satisfied |
|  | RSRQ | dB | -5 | -13.23 | - |  |

Table 8.4.8.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message with SCell (Cell 3/10) addition. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm SCell (Cell 3/10) addition. | --> | RRCConnectionReconfiguration Complete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup WLAN offload configurations on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmit RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1, Cell 3/10 and Cell 27 parameters according to the row "T1" in table 8.4.8.4.3.2-1. | - | - | - | - |
| 6 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.12 indicate that the UE does not offload to WLAN Cell 27 after TsteeringWLAN expiry? | - | - | 1 | - |
| 7 | The SS changes Cell 1, Cell 3/10 and Cell 27 parameters according to the row "T2" in table 8.4.8.4.3.2-1. | - | - | - | - |
| 8 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.10 indicate that the UE has successfully offloaded to WLAN Cell 27 after TsteeringWLAN expiry? | - | - | 2 | P |
| 9 | The SS changes Cell 1, Cell 3/10 and Cell 27 parameters according to the row "T3" in table 8.4.8.4.3.2-1. | - | - | - | - |
| 10 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.13 indicate that the UE does not offload to EUTRA Cell 1 after TsteeringWLAN expiry? | - | - | 3 | - |
| 11 | The SS changes Cell 1, Cell 3/10 and Cell 27 parameters according to the row "T4" in table 8.4.8.4.3.2-1. | - | - | - | - |
| 12 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.3.11 indicate that the UE has successfully offloaded to EUTRA Cell 1 after TsteeringWLAN expiry? | - | - | 4 | P |

8.4.8.4.3.3	Specific message contents

Table 8.4.8.4.3.3-1: RRCConnectionReconfiguration (step 3, Table 8.4.8.4.3.2-2)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition WLAN-OffloadSetup

Table 8.4.8.4.3.3-2: wlan-OffloadInfo-r12 (Table 8.4.8.4.3.3-1)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-29 with condition RSRQ and BackhaulBW |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| wlan-OffloadInfo-r12::= CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| wlan-OffloadConfigDedicated-r12SEQUENCE { |  |  |  |
| thresholdRSRP-r12 | Not Present |  |  |
| thresholdRSRQ-r12 SEQUENCE { |  |  |  |
| thresholdRSRQ-Low-r12 | 17 | -11.5dB ≤ RSRQ < -11dB |  |
| thresholdRSRQ-High-r12 | 32 | -4 dB ≤ RSRQ < -3.5dB |  |
| } |  |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r4096 kbps |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r8192 kbps |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r0 kbps | Value makes it insignificant for offload |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r4294967296 kbps | Value makes it insignificant for offload |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.8.5	WLAN Offload / T350 expiry

8.4.8.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state receives an RRCConnectionReconfiguration message containing wlan-OffloadConfigDedicated with t350 }

ensure that {

when { UE moves to RRC Idle and in RRC_IDLE state the timer t350 expires  }

then { UE offloads to WLAN AP after TsteeringWLAN as per wlan-OffloadConfigCommon not per wlan-OffloadConfigDedicated once the offload conditions are met }

}

8.4.8.5.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.6.12.2, 5.6.12.3, 5.6.12.4, TS 36.304, clause 5.6.2 and TS 24.302, clause 6.10.4.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadInfo is set to release:

2> release wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

1>	else:

2>	apply the received wlan-OffloadConfigDedicated:

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304;

[TS 36.331, clause 5.6.12.4]

The UE shall:

1>	if T350 expires or is stopped:

2>	release the wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only.

[TS 24.302, clause 6.10.4]

This subclause applies if the RAN rules control the WLAN access selection and traffic routing as described in subclause 6.10.2.

The access stratum layer of the 3GPP access can provide:

1)	move-traffic-to-WLAN indication, along with list of WLAN identifiers. An entry in the list of the WLAN identifiers consists of SSID, BSSID, HESSID, or any combination of them; and

2)	move-traffic-from-WLAN indication.

The user preferences take precedence over the indications provided by the access stratum layer of the 3GPP access.

Upon:

-	receiving move-traffic-to-WLAN indication, along with the list of the WLAN identifiers, if the user preferences are not present; or

-	establishment of a new PDN connection in 3GPP access, if the PDN connection is an offloadable PDN connection, the access stratum indicated move-traffic-to-WLAN, the access stratum has not indicated the move-traffic-from-WLAN indication after indicating of the move-traffic-to-WLAN indication and the user preferences are not present;

and:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and all the PDN connections of the UE in 3GPP access are offloadable PDN connections;

the UE:

a)	shall perform the procedure in subclause 5.1.3.2.3 and in subclause 5.2.3.2 to select the selected WLAN and the NAI for authentication;

b)	if not authenticated yet with the selected WLAN using the NAI for authentication in subclause 6.4, shall authenticate with the selected WLAN using the NAI for authentication in subclause 6.4. During authentication, if the selected WLAN is a trusted WLAN, SCM is supported by both UE and network, MCM is not supported by UE, network or both, and if:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and the UE has only one PDN connection;

shall handover one offloadable PDN connection from 3GPP access to the WLAN access using procedures in subclause 6.4.2.6.2;

NOTE:	When the UE already has one PDN connection established via WLAN in SCM, and if move-traffic-to-WLAN indication is received, it is up to the UE implementation to determine whether to offload a PDN connection from 3GPP access to WLAN. In that case, it is also up to the UE implementation to determine which one of the offloadable PDN connections will be offloaded.

c)	if the selected WLAN is a trusted WLAN, and MCM is supported by both UE and network, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures of 3GPP TS 24.244];

d)	if the selected WLAN is an untrusted WLAN, and if the UE supports access to EPC via untrusted WLAN, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures in subclause 7.2.1 and subclause 7.2.2; and

e)	if the UE has a valid IARP rule for APN, shall use the IARP for APN using the procedures in subclause 6.8.2.2.4.5.

Upon receiving move-traffic-from-WLAN indication, and if the user preferences are not present, the UE shall handover all the PDN connections established in (or previously handed over to) WLAN access to the 3GPP access using procedures in 3GPP TS 24.301.

8.4.8.5.3	Test Description

8.4.8.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN AP 1.

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The UE supporting ANDSF is pre-provisioned with WLANSP ANDSF MO with at least one WLAN identifier and with ForServiceBased ANDSF MO with RANValidityCondition allowing at least one APN to use both WLAN and LTE and at least one APN to use LTE only.

-	The UE supporting RAN rules is configured with at least two PDN connections, one PDN connection offloadable to WLAN and one PDN connection not offloadable to WLAN.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.8.5.3.2	Test procedure sequence

Table 8.4.8.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while the configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 8.4.8.5.3.2-2.

Table 8.4.8.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | E-UTRA Cell 1 is a suitable cell and WLAN AP Cell 27 is off. |
|  | BeaconRSSI | dBm | - | Off |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied. Qrxlevmeas < ThreshServingOffloadWLAN, LowP and ChannelUtilizationWLAN < ThreshChUtilWLAN, Low |
|  | ChannelUtilizationWLAN | - | - | 55 |  |
|  |  |  | - |  |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 |  | The power level values are such that conditions for traffic steering from WLAN to E-UTRA are satisfied |
|  | ChannelUtilizationWLAN | - | - | 100 |  |

Table 8.4.8.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup WLAN offload configurations on Cell 1 with WLAN offload thresholds and t350. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 4 | The SS changes WLAN parameters according to the row "T1" in table 8.4.8.5.3.2-1. | - | - | - | - |
| 5 | The UE transitions to state RRC_IDLE which starts timer t350. | - | - | - | - |
| 6 | Wait for 300 seconds to allow t350 expiry. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.3.10 indicate that the PDN connections are offloaded from 3GPP access to the WLAN with offload parameters as per wlan-OffloadConfigCommon? | - | - | 1 | P |
| 8 | The SS changes WLAN parameters according to the row "T2" in table 8.4.8.5.3.2-1. | - | - | - | - |
| 9 | The test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicates that the PDN connection is offloaded from the WLAN access to E-UTRAN | - | - | - | - |

8.4.8.5.3.3	Specific message contents

Table 8.4.8.5.3.3-1: SystemInformationBlockType17 (Preamble and all steps of table 8.4.8.5.2.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT | as per Table 8.4.8.5.3.3-2 |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.4.8.5.3.3-2: WLAN-OffloadConfig-DEFAULT(Preamble and all steps of table 8.4.8.5.2.2-2)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-30, condition RSRP and ChanUtilization |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 75 |  |  |
| thresholdRSRP-High-r12 | 85 |  |  |
| } |  |  |  |
| thresholdChannelUtilization-r12::= SEQUENCE { |  |  |  |
| thresholdChannelUtilizationLow-r12 | 65 |  |  |
| thresholdChannelUtilizationHigh-r12 | 75 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.8.5.3.3-3: RRCConnectionReconfiguration (step 1, Table 8.4.8.5.3.2-2)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition WLAN-OffloadSetup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT | as per Table 8.4.8.5.3.3-4 |  |
| t350-r12 | min5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.8.5.3.3-4: WLAN-OffloadConfig-DEFAULT (step 1, Table 8.4.8.5.3.2-2)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-30, condition RSRP and ChanUtilization |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-Low-r12 | 75 |  |  |
| thresholdRSRP-High-r12 | 85 |  |  |
| } |  |  |  |
| thresholdChannelUtilization-r12::= SEQUENCE { |  |  |  |
| thresholdChannelUtilizationLow-r12 | 50 |  |  |
| thresholdChannelUtilizationHigh-r12 | 75 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.4.8.6	WLAN Offload / Offload Success / EUTRA RRC_Connected to/from WLAN (ANDSF and RAN rules co-existence)

8.4.8.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on RAN rules is satisfied but based on stored ISRP ANDSF rule is not }

then { UE does not offload PDN connections to WLAN }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on stored ISRP ANDSF rule is satisfied }

then { UE offload PDN connections to WLAN }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and A PDN offloaded to WLAN, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on RAN rules is satisfied but based on stored ISRP ANDSF rule is not }

then { UE does not offload PDN connections to EUTRAN }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and A PDN offloaded to WLAN, with valid ISRP ANDSF rule using RAN thresholds }

ensure that {

when { Offload based on stored ISRP ANDSF rule is satisfied }

then { UE offload PDN connections to EUTRAN }

}

8.4.8.6.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.6.12.2, 5.6.12.3, TS 36.304, clause 5.6.2 and TS 24.302, clause 6.10.4.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.6.12.2]

The UE shall:

1>	if the received wlan-OffloadInfo is set to release:

2> release wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

1>	else:

2>	apply the received wlan-OffloadConfigDedicated:

[TS 36.331, clause 5.6.12.3]

The UE shall:

1>	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated:

2>	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in TS 24.312 to upper layers;

2>	evaluate the network selection and traffic steering rules as defined in TS 36.304;

[TS 36.304, clause 5.6.2]

The rules in this sub-clause are only applicable for WLANs for which identifiers has been signalled to the UE by E-UTRAN and the UE is capable of RAN-assisted WLAN interworking based on access network selection and traffic steering rules. Coexistence with ANDSF based WLAN selection and traffic steering methods on the UE is based on mechanism described in TS 23.402. The rules refer to the following quantities:

| ChannelUtilizationWLAN | WLAN channel utilization as defined in subclause 8.4.2.30 in [26]. |
| --- | --- |
| BackhaulRateDlWLAN | WLAN DLBandwidth as defined in subclause 9.1.2 in [27]. |
| BackhaulRateUlWLAN | WLAN ULBandwidth as defined in subclause 9.1.2 in [27]. |
| BeaconRSSI | WLAN Beacon RSSI as defined in [7]. |
| RSRPmeas | Qrxlevmeas in RRC_IDLE, and PCell RSRP in RRC_CONNECTED as defined in TS 36.331 [3]. |
| RSRQmeas | Qqualmeas in RRC_IDLE, and PCell RSRQ in RRC_CONNECTED as defined in TS 36.331 [3]. |

The upper layers in the UE shall be notified (see TS 24.302) when and for which WLAN(s), that matches all the provided identifiers (in subclause 5.6.3) for a specific entry in the list, the following conditions 1 and 2 for steering traffic from E-UTRAN to WLAN are satisfied for a time interval TsteeringWLAN:

In the E-UTRAN serving cell:

RSRPmeas < ThreshServingOffloadWLAN, LowP; or

RSRQmeas < ThreshServingOffloadWLAN, LowQ;

In the target WLAN:

ChannelUtilizationWLAN < ThreshChUtilWLAN, Low; and

BackhaulRateDlWLAN > ThreshBackhRateDLWLAN, High; and

BackhaulRateUlWLAN > ThreshBackhRateULWLAN, High; and

BeaconRSSI > ThreshBeaconRSSIWLAN, High;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only. If not all metrics related to the provided thresholds can be acquired for a WLAN BSS, the UE shall exclude that WLAN BSS from the evaluation of the above rule.

The upper layers in the UE shall be notified (see TS 24.302) when the following conditions 3 or 4 for steering traffic from WLAN to E-UTRAN are satisfied for a time interval TsteeringWLAN:

In the source WLAN:

ChannelUtilizationWLAN > ThreshChUtilWLAN, High; or

BackhaulRateDlWLAN < ThreshBackhRateDLWLAN, Low; or

BackhaulRateUlWLAN < ThreshBackhRateULWLAN, Low; or

BeaconRSSI < ThreshBeaconRSSIWLAN, Low;

In the target E-UTRAN cell:

RSRPmeas > ThreshServingOffloadWLAN, HighP; and

RSRQmeas > ThreshServingOffloadWLAN, HighQ;

The UE shall not consider the metrics for which a threshold has not been provided. The UE shall evaluate the E-UTRAN conditions on PCell only.

[TS 24.302, clause 6.10.4]

This subclause applies if the RAN rules control the WLAN access selection and traffic routing as described in subclause 6.10.2.

The access stratum layer of the 3GPP access can provide:

1)	move-traffic-to-WLAN indication, along with list of WLAN identifiers. An entry in the list of the WLAN identifiers consists of SSID, BSSID, HESSID, or any combination of them; and

2)	move-traffic-from-WLAN indication.

The user preferences take precedence over the indications provided by the access stratum layer of the 3GPP access.

Upon:

-	receiving move-traffic-to-WLAN indication, along with the list of the WLAN identifiers, if the user preferences are not present; or

-	establishment of a new PDN connection in 3GPP access, if the PDN connection is an offloadable PDN connection, the access stratum indicated move-traffic-to-WLAN, the access stratum has not indicated the move-traffic-from-WLAN indication after indicating of the move-traffic-to-WLAN indication and the user preferences are not present;

and:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and all the PDN connections of the UE in 3GPP access are offloadable PDN connections;

the UE:

a)	shall perform the procedure in subclause 5.1.3.2.3 and in subclause 5.2.3.2 to select the selected WLAN and the NAI for authentication;

b)	if not authenticated yet with the selected WLAN using the NAI for authentication in subclause 6.4, shall authenticate with the selected WLAN using the NAI for authentication in subclause 6.4. During authentication, if the selected WLAN is a trusted WLAN, SCM is supported by both UE and network, MCM is not supported by UE, network or both, and if:

-	the UE is capable to simultaneously route IP traffic to both 3GPP access and WLAN; or

-	the UE is not capable to simultaneously route IP traffic to both 3GPP access and WLAN, and the UE has only one PDN connection;

shall handover one offloadable PDN connection from 3GPP access to the WLAN access using procedures in subclause 6.4.2.6.2;

NOTE:	When the UE already has one PDN connection established via WLAN in SCM, and if move-traffic-to-WLAN indication is received, it is up to the UE implementation to determine whether to offload a PDN connection from 3GPP access to WLAN. In that case, it is also up to the UE implementation to determine which one of the offloadable PDN connections will be offloaded.

c)	if the selected WLAN is a trusted WLAN, and MCM is supported by both UE and network, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures of 3GPP TS 24.244];

d)	if the selected WLAN is an untrusted WLAN, and if the UE supports access to EPC via untrusted WLAN, shall handover all the offloadable PDN connections from 3GPP access to the WLAN access using procedures in subclause 7.2.1 and subclause 7.2.2; and

e)	if the UE has a valid IARP rule for APN, shall use the IARP for APN using the procedures in subclause 6.8.2.2.4.5.

Upon receiving move-traffic-from-WLAN indication, and if the user preferences are not present, the UE shall handover all the PDN connections established in (or previously handed over to) WLAN access to the 3GPP access using procedures in 3GPP TS 24.301.

8.4.8.6.3	Test Description

8.4.8.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and WLAN AP 1.

-	System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The UE supporting ANDSF is pre-provisioned with WLANSP ANDSF MO with at least one WLAN identifier and with ForServiceBased ANDSF MO with RANValidityCondition allowing at least one APN to use both WLAN and LTE and at least one APN to use LTE only.

-	The UE supporting RAN rules is configured with at least two PDN connections, one PDN connection offloadable to WLAN and one PDN connection not offloadable to WLAN.

-	The UE has a ISRP ANDSF rule using RAN thresholds of BackhaulRateDlWLAN.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.4.8.6.3.2	Test procedure sequence

Table 8.4.8.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while the configuration marked "T1", "T2", “T3” and “T4” are applied at the point indicated in the Main behaviour description in Table 8.4.8.6.3.2-2.

Table 8.4.8.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | WLAN | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | E-UTRA Cell 1 is a suitable cell and WLAN is switched off. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are satisfied for RSRP and UL Backhaul BW (RAN) and not satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r2048 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r16384 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -79 | - | The power level values are such that conditions for traffic steering from E-UTRA to WLAN are not satisfied for RSRP and UL Backhaul BW (RAN) and satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r16384 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r2048 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA are satisfied for UL Backhaul BW(RAN) and not satisfied for DL Backhaul BW(ANDSF). |
|  | BackhaulDL-Bandwidth | kbps | - | r16384 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r2048 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -91 | - | The power level values are such that conditions for traffic steering from WLAN to E-UTRA satisfied for DL Backhaul BW(ANDSF/RAN). |
|  | BackhaulDL-Bandwidth | kbps | - | r1024 |  |
|  | BackhaulUL-Bandwidth | kbps | - | r16384 |  |

Table 8.4.8.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and WLAN parameters according to the row "T1" in table 8.4.8.6.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.10-1 indicate that the PDN connection is offloaded from E-UTRAN to the WLAN? | - |  | 1 | F |
| 3 | The SS changes Cell 1 and WLAN parameters according to the row "T2" in table8.4.8.6.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.10-1 indicate that the PDN connection is offloaded from E-UTRAN to the WLAN? | - | - | 2 | P |
| 5 | The SS changes Cell 1 and WLAN parameters according to the row "T3" in table 8.4.8.6.3.2-1 | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicate that the PDN connection is offloaded from the WLAN access to E-UTRAN? | - | - | 3 | F |
| 7 | The SS changes Cell 1 and WLAN parameters according to the row "T4" in table 8.4.8.6.3.2-1 | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.3.11-1 indicate that the PDN connection is offloaded from the WLAN access to E-UTRAN? | - | - | 4 | P |

8.4.8.6.3.3	Specific message contents

Table 8.4.8.6.3.3-1: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.3-30, condition RSRP and BackhaulBW |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  |  |  |
| thresholdRSRP-Low-r12 | 75 |  |  |  |
| thresholdRSRP-High-r12 | 85 |  |  |  |
| } |  |  |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  |  |  |
| thresholdBackhaulDL-BandwidthLow-r12 | r2048 |  |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | r4096 |  |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | r2048 |  |  |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | r4096 |  |  |  |
| } |  |  |  |  |
| t-SteeringWLAN-r12 | 7 | Typical value in real networks |  |  |
| } |  |  |  |  |

## 8.5	RRC others

### 8.5.1	Radio link failure

#### 8.5.1.1	Radio link failure / RRC connection re-establishment success

8.5.1.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE detecting physical layer problems}

then { UE shall start timer T310 and UE does not initiate any RRC Connection re-establishment procedure before expiring of timer T310}

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE detecting radio link failure on expiring of timer T310 }

then { UE starts timer T311 and UE initiates the RRC Connection re-establishment procedure }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE successfully completes the RRC Connection re-establishment procedure }

then { UE is in E-UTRA RRC_CONNECTED state }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and detecting radio link failure on expiring of timer T310 }

ensure that {

when { An exclude-listed neighbour cell satisfies the re-establishment condition, i.e. it has the UE context }

then { UE starts timer T311 and initiates the RRC Connection re-establishment procedure to the exclude-listed cell }

}

8.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.7.2, 5.3.10.1, 5.3.10.3 and 5.3.10.4 TS 36.304, clause 5.2.3.1.

[TS 36.331 clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	upon integrity check failure indication from lower layers; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5;

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1> start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331 clause 5.3.11.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications from lower layers while neither T300, T301, T304 nor T311 is running:

2>	start timer T310;

[TS 36.331 clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’;

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.304 clause 5.2.3.1]

The UE shall use one of the following two cell selection procedures:

…

b)	Stored Information Cell Selection

This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells. Once the UE has found a suitable cell the UE shall select it. If no suitable cell is found the Initial Cell Selection procedure shall be started.

NOTE:	Priorities between different frequencies or RATs provided to the UE by system information or dedicated signalling are not used in the cell selection process.

8.5.1.1.3	Test description

8.5.1.1.3.1	Pre-test conditions

System Simulator:

-	3 cells on same E-UTRA frequency:

-	Cell 1 (default parameters) serving cell

-	Cell 2 and Cell 4 are intra-frequency neighbour cells

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18] on cell 1.

8.5.1.1.3.2	Test procedure sequence

Table 8.5.1.1.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -85 | -97 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” | -85 |
| Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1, Cell 2 and Cell 4 parameters according to the row "T1" in table 8.5.1.1.3.2-0 in order that the radio link quality of Cell 1 is degraded and cell 2 is suitable for camping. | - | - | - | - |
| 2 | Check: Does the UE initiate an RRC connection re-establishment procedure on Cell 1 or Cell 2. This is checked during the time T=T310. | - | - | 1 | F |
| 3 | Check: Does the UE send RRCConnectionReestablishmentRequest message on Cell 2? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 4 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 5 | The UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - |  | 3 | P |
| 9 | The SS changes Cell 2 and Cell 4 parameters according to the row "T2" in table 8.5.1.1.3.2-0 in order that the radio link quality of Cell 2 is degraded and cell 4 is suitable for camping. | - | - | - | - |
| 10 | Check: Does the UE send RRCConnectionReestablishmentRequest message on Cell 4? | --> | RRCConnectionReestablishmentRequest | 4 | P |
| 11 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 4? | - |  | 3 | - |

8.5.1.1.3.3	Specific message contents

Table 8.5.1.1.3.3-0: SystemInformationBlockType4 for Cell 2 (Preamble and all steps, Table 8.5.1.1.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqExcludedCellList SEQUENCE (SIZE (1..maxExcludedCell)) OF SEQUENCE { |  |  |  |
| PhysCellIdRange[1] SEQUENCE { |  |  |  |
| start | physicalCellIdentity-Cell4 |  |  |
| range | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.1.3.3-1: RRCConnectionReestablishmentRequest (step 3, Table 8.5.1.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.1.3.3-2: RRCConnectionReconfiguration (step 6 and step 13, Table 8.5.1.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.1.3.3-3: RRCConnectionReestablishmentRequest (step 10, Table 8.5.1.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.1.2	Radio link failure / T301 expiry

8.5.1.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE having sent an RRCConnectionReestablishmentRequest message on starting of timer T301 }

then { UE goes to RRC_IDLE state after timer T301 is expired and trigger TAU procedure in order to recover RRC connection}

}

8.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.7.2, 5.3.7.3, 5.3.7.7, 5.3.11.1 and 5.3.11.3. The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331 clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	upon integrity check failure indication from lower layers; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5;

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331 clause 5.3.7.3]

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	stop timer T311;

1>	start timer T301;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source cell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331 clause 5.3.7.7]

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 36.304 [4]:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331 clause 5.3.11.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications from lower layers while neither T300, T301, T304 nor T311 is running:

2>	start timer T310;

[TS 36.331 clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

8.5.1.2.3	Test description

8.5.1.2.3.1	Pre-test conditions

System Simulator:

- 2 cells on same E-UTRA frequency:

-	Cell 1 (default parameters) serving cell

-	Cell 2 intra-frequency cell

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.1.2.3.2	Test procedure sequence

Table 8.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level of Cell 1 to non-suitable “Off” and changes the power level of Cell 2 to suitable according to TS 36.508 subclause 6.2.2.1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 2 | The UE sends RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 3 | The SS does not respond to any RRCConnectionReestablishmentRequest message for 2s (T301). | - | - | - | - |
| 4-8 | The UE will perform TAU procedure based on steps 1 to 5 of subclause 6.4.2.7 in TS 36.508 on Cell 2. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

8.5.1.2.3.3	Specific message contents

Table 8.5.1.2.3.3-1: SystemInformationBlockType2 for Cell 2 (all steps)

| Derivation path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t301 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.2.3.3-2: RRCConnectionReestablishmentRequest (step 2, Table 8.5.1.2.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.1.3	Radio link failure / T311 expiry

8.5.1.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with default bearer established and radio link failure was detected and UE attempts to select a suitable E-UTRA cell to re-establish the RRC connection }

ensure that {
  when { UE can not find a suitable cell within T311 }

then { UE does not try to re-establish the RRC connection and goes to RRC_IDLE state after T311 expired }

}

8.5.1.3.2	Conformance requirements

The conformance requirements covered in the current test case are specified in TS 36.331 clause 5.3.7.2, 5.3.7.6 and 5.3.12.

[TS 36.331 clause 5.3.7.2]

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

...

[TS 36.331 clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘RRC connection failure’.

[TS 36.331 clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

8.5.1.3.3	Test description

8.5.1.3.3.1	Pre-test conditions

System simulator:

2 cells on same E-UTRA frequency:

-	Cell 1 (default parameters) serving cell

-	Cell 11 intra-frequency cell

UE:

None.

Preamble:

-	The UE is in Generic RB Established (state 3) according to [18] on Cell 1.

-	DRX is not configured

8.5.1.3.3.2	Test procedure sequence

Table 8.5.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level of Cell 1 to non-suitable “Off” according to TS 36.508 subclause 6.2.2.1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 2 | Wait for 12s (T311 (10s) is transmitted in SIB2). | - | - | - | - |
| 3 | The SS changes the power level of Cell 11 to “Serving Cell” according to TS 36.508 subclause 6.2.2.1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 11? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

8.5.1.3.3.3	Specific message contents

Table 8.5.1.3.3.3-1: RRCConnectionReconfiguration (preamble)

| Derivation path: 36.508 table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.1.4	Radio link failure / RRC connection re-establishment reject

8.5.1.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with default bearer established and radio link failure was detected and UE initiates the re-establishment procedure}

ensure that {
  when { the UE receives a RRCConnectionReestablishmentReject message }

then { UE goes to RRC_IDLE and trigger TAU procedure in order to recover RRC connection }

}

8.5.1.4.2	Conformance requirements

The conformance requirements covered in the current test case are specified in TS 36.331 clause 5.3.7.8 and 5.3.12.

[TS 36.331 clause 5.3.7.8]

Upon receiving the RRCConnectionReestablishmentReject message, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘RRC connection failure’.

[TS 36.331 clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4];

8.5.1.4.3	Test description

8.5.1.4.3.1	Pre-test conditions

System simulator:

-	2 cells on same E-UTRA frequency:

-	Cell 1(default parameters) serving cell

-	Cell 2 intra-frequency cell

UE:

None.

Preamble:

-	The UE is in Generic RB Established (state 3) according to [18] on Cell 1.

8.5.1.4.3.2	Test procedure sequence

Table 8.5.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level of Cell 1 to non-suitable “Off” cell according to TS 36.508 subclause 6.2.2.1 in order that the radio link quality of Cell 1 is degraded and set the power level of Cell 2 to suitable cell. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionRestablishmentRequest | - | - |
| 3 | The SS transmits a RRCConnectionReestablishmentReject message | <-- | RRCConnectionReestablishmentReject | - | - |
| 4-8 | Check: Does the UE perform TAU procedure based on steps 1 to 5 of subclause 6.4.2.7 in TS 36.508 on Cell 2. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery. | - | - | 1 | - |
| 8A | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 9 | Void | - | - | - | - |

8.5.1.4.3.3	Specific message contents

None.

#### 8.5.1.5	Radio link failure / Radio link recovery while T310 is running

8.5.1.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE detecting physical layer recovery while T310 was running }

then { the UE resumes the RRC connection without explicit signalling }

}

8.5.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.11.1 and 5.3.11.2.

[TS 36.331, clause 5.3.11.1]

The UE shall:

1>	upon receiving N310 consecutive "out of sync" indications from lower layers while neither T300, T301, T304 nor T311 is running:

2>	start timer T310.

[TS 36.331, clause 5.3.11.2]

Upon receiving N311 consecutive "in-sync" indications from lower layers while T310 is running, the UE shall:1>

stop timer T310.

NOTE 1:		In this case, the UE resumes the RRC connection without explicit signalling, i.e. the UE resumes the entire radio resource configuration.

NOTE 2:		Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications

8.5.1.5.3	Test description

8.5.1.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.5.1.5.3.2	Test procedure sequence

Table 8.5.1.5.3.2-1 illustrates the downlink power level to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial condition, while column marked "T1"is applied according the procedure.

Table 8.5.1.5.3.2-1: Time instances of cell power level

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | Pdefault | Power level from 36.508 clause 6.2.2.1. Pdefault as serving cell. |
| T1 | RS EPRE | dBm/15kHz | Poff | Poff as non-suitable “Off” cell. |

Table 8.5.1.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | SS wait for the 660ms to ensure that DL Timing Advance is sent at least once and UE apply the value of timeAlignmentTimerDedicated which is updated during the preamble. | - | - | - | - |
| 1 | The SS changes Cell 1 level according to the row "T1" in table 8.5.1.5.3.2-1. | - | - | - | - |
| 2 | SS waits for 1.5s. The T310 is 2s. | - | - | - | - |
| 3 | The SS changes Cell 1 level according to the row "T0" in table 8.5.1.5.3.2-1. | - | - | - | - |
| 4 | SS waits for 5s. Check: Does the UE transmit any signalling message? | - | - | 1 | F |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.5.1.5.3.3		Specific message contents

Table 8.5.1.5.3.3-1: SystemInformationBlockType2 (preamble and all steps, Table 8.5.1.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.5.3.3-2: RRCConnectionReconfiguration (preamble: Table 4.5.3.3-1 [18], step 8)

| Derivation path: 36.508 table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

NOTE: As per test model SS is not configured to transmit PDCCH orders and it expects UE to be PUCCH synchronized throughout the test sequence.

#### 8.5.1.6	Radio link failure / T311 expiry / Dedicated RLF timer

8.5.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message containing an rlf-TimersAndConstants-r9 set to setup }

then { UE uses timer value received in the RRCConnectionReconfiguration message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and having received an RRCConnectionReconfiguration message containing an rlf-TimersAndConstants-r9 set to setup }

ensure that {
  when { UE receives SystemInformationBlockType2 containing different timer value from RRCConnectionReconfiguration message }

then { UE continues to use timer value received in the RRCConnectionReconfiguration message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and having received an RRCConnectionReconfiguration message containing an rlf-TimersAndConstants-r9 set to setup }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message containing an rlf-TimersAndConstants-r9 set to release }

then { UE does not use timer value received in the RRCConnectionReconfiguration message }

}

8.5.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.2.2.9, 5.3.7.2, 5.3.7.6, 5.3.10.0, 5.3.10.7 and 5.3.12.

[TS 36.331 clause 5.2.2.9]

Upon receiving SystemInformationBlockType2, the UE shall:

...

1>	if in RRC_CONNECTED and UE has previously received rlf-TimersAndConstants:

2>	The UE shall not update its values of the timers and constants in UE-TimersAndConstants except for the value of timer T300.

[TS 36.331 clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

[TS 36.331 clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331 clause 5.3.10.0]

The UE shall:

...

1>	if the received radioResourceConfigDedicated includes the rlf-TimersAndConstants:

2>	reconfigure the values of timers and constants as specified in 5.3.10.7;

[TS 36.331 clause 5.3.10.7]

The UE shall:

1>	if the received rlf-TimersAndConstants is set to ‘release’:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2;

1>	else:

2>	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants;

[TS 36.331 clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE  and perform procedures as specified in TS 36.304 [4, 5.2.7];

8.5.1.6.3	Test description

8.5.1.6.3.1	Pre-test conditions

System simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.1.6.3.2	Test procedure sequence

Table 8.5.1.6.3.2-1 illustrates the downlink power levels to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.1.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | ”Off” | No Cells are available. (NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -85 | Only Cell 2 is available. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.5.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 levels according to row "T1" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 2 | Wait for 10s (see NOTE). | - | - | - | - |
| 3 | The SS changes Cell 2 levels according to row "T2" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 5-9 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 10 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 11-18 | Steps 2 to 9 of the generic test procedure in TS 36.508 subclause 4.5.3.3 are performed on Cell 2. NOTE: The UE performs the establishment of the data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 19 | The SS transmits a Paging message including systemInfoModification on Cell 2. | <-- | Paging | - | - |
| 20 | Wait for 2.1* modification period for the UE to receive system information. | - | - | - | - |
| 21 | The SS changes Cell 2 levels according to row "T1" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 22 | Wait for 10s (see NOTE). | - | - | - | - |
| 23 | The SS changes Cell 1 levels according to row "T3" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 24 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 25-29 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 30 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 31-38 | Steps 2 to 9 of the generic test procedure in TS 36.508 subclause 4.5.3.3 are performed on Cell 1. NOTE: The UE performs the establishment of the data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 39 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 40 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 41 | The SS changes Cell 1 levels according to row "T1" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 42 | Wait for 10s (see NOTE). | - | - | - | - |
| 43 | The SS changes Cell 2 levels according to row "T2" in Table 8.5.1.6.3.2-1. | - | - | - | - |
| 44 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 2? | --> | RRCConnectionReestablishmentRequest | 3 | P |
| 45 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 46 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 47 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 48 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 49 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 3 | - |
| NOTE: The delay of 10 sec is more than 8.4 sec (= time for evaluation of the out-of-sync situation + T310 + T311) plus one additional DRX cycle of 1.28 sec. |  |  |  |  |  |

8.5.1.6.3.3	Specific message contents

Table 8.5.1.6.3.3-1: SystemInformationBlockType2 for Cell 1 (preamble and all steps, Table 8.5.1.6.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t311 | ms30000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.6.3.3-2: RRCConnectionReconfiguration (preamble, step 17 and 37 Table 8.5.1.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition SRB2-DRB(1, 0)

Table 8.5.1.6.3.3-3: RadioResourceConfigDedicated (Table 8.5.1.6.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| rlf-TimersAndConstants-r9 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| t301-r9 | ms1000 |  |  |
| t310-r9 | ms1000 |  |  |
| n310-r9 | n1 |  |  |
| t311-r9 | ms1000 |  |  |
| n311-r9 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.6.3.3-4: Paging (step 19, Table 8.5.1.6.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

Table 8.5.1.6.3.3-5: SystemInformationBlockType2 for Cell 2 (step 20, Table 8.5.1.6.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t311 | ms30000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.6.3.3-6: RRCConnectionReconfiguration (step 39, Table 8.5.1.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated SEQUENCE { | RadioResourceConfigDedicated-NON-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.6.3.3-7: RadioResourceConfigDedicated-NON-DEFAULT (Table 8.5.1.6.3.3-6)

| Derivation Path: 36.508, Table 4.6.3-18A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NON-DEFAULT ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| rlf-TimersAndConstants-r9 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.1.7	CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell

##### 8.5.1.7.1	CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band Contiguous CA

8.5.1.7.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED, security activated and Scell configured }

ensure that {

when { UE drops out out service on Scell }

then { UE does not consider radio link failure to be detected and stay in RRC_CONNECTED state }

}

8.5.1.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.10.7, 5.3.11.1 and 5.3.11.3.

[TS 36.331, clause 5.3.10.7]

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2;

1>	else:

2>	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants;

[TS 36.331, clause 5.3.11.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers while neither T300, T301, T304 nor T311 is running:

2>	start timer T310;

NOTE:	Physical layer monitoring and related autonomous actions do not apply to SCells.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48, hours after the radio link failure is detected, upon power off or upon detach.

8.5.1.7.1.3	Test description

8.5.1.7.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, and Cell 3

-	Cell 1 is PCell

-	Cell 3 is SCell

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.1.7.1.3.2	Test procedure sequence

Table 8.5.1.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.1.7.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | Cell1 and Cell 3 are available |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | Only Cell 1 is available. . |

Table 8.5.1.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to configure Scell. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 2A | Wait for 1s. | - | - | - | - |
| 3 | The SS changes power levels according to row "T1" in Table 8.5.1.7.1.3.2-1. | - | - | - | - |
| 4 | Wait for 1s. | - | - | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 1? | --> | RRCConnectionReestablishmentRequest | 1 | F |
| 6 | Check: Does the test result of CALL generic test procedure in 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.5.1.7.1.3.3	Specific message contents

Table 8.5.1.7.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.5.1.7.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.5.1.7.1.3.3-2: SCellToAddMod-r10 (Table 8.5.1.7.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.5.1.7.1.3.3-3: RadioResourceConfigCommonSCell-r10 (Table 8.5.1.7.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

##### 8.5.1.7.2	CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Inter-band CA

The scope and description of the present TC is the same as test case 8.5.1.7.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

##### 8.5.1.7.3	CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 8.5.1.7.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

#### 8.5.1.8	Dual Connectivity / Radio link failure

##### 8.5.1.8.1	Radio link failure on PSCell / UE supports SCG DRB

8.5.1.8.1.1	Test Purpose (TP)

(1)

With { UE in connected mode with SCG activated with a SCG DRB established }

ensure that {

when { T313 expires }
  then { the UE transmits a SCGFailureInformation message with failureType set to ‘t313-Expiry’ }

}

(2)

With { UE in connected mode with SCG activated with a SCG DRB established }
ensure that {
  when { random access problem indication from SCG MAC }
  then { the UE transmits a  SCGFailureInformation message with failureType set to ‘randomAccessProblem’ }
            }

(3)

With { UE in connected mode with SCG activated with a MCG DRB and a SCG DRB established }
ensure that {
  when { indication from SCG RLC that the maximum number of retransmissions has been reached for a SCG DRB }
  then { the UE shall transmit a  SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’ }
            }

(4)

With { UE in connected mode with SCG activated with a SCG DRB established }
ensure that {
  when { T307 expires }
  then { the UE shall transmit a  SCGFailureInformation message with failureType set to ‘scg-ChangeFailure’ }
            }

(5)

With { UE in connected mode with SCG activated }
ensure that {
  when { UE detects radio link failure of the PSCell and has PDCP SDUs available for transmission on the MCG DRB }
  then { the UE transmits the uplink data on the MCG DRB }
            }

8.5.1.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.7a, 5.3.11.3, 5.6.13.2, and 5.6.13.3. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.7a]

The UE shall:

1>	if T307 expires:

NOTE 1:	Following T307 expiry any dedicated preamble, if provided within the rach-ConfigDedicatedSCG, is not available for use by the UE anymore.

2>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG change failure;

[TS 36.331, clause 5.3.11.3]

…

The UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached for an SCG or split DRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.13.2]

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a;

Upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

[TS 36.331, clause 5.6.13.3]

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2>	include failureType and set it to the trigger for detecting SCG radio link failure;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

2>	include failureType and set it to scg-ChangeFailure;

1>	set the measResultServFreqList to include for each SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16];

1>	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

8.5.1.8.1.3	Test description

8.5.1.8.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 10 is the PSCell1 and Cell 28 is the PSCell2.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1,  Cell 10 and Cell 28.

UE:

Preamble:

-	The UE is in state DC MCG/SCG DRB Loopback Activated (state 6A with Test loop Mode B) on Cell 1 and Cell 10 according to [18].

8.5.1.8.1.3.2	Test procedure sequence

Table 8.5.1.8.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.1.8.1.3.2-1: Time instances of cell power level and parameter changes

|  |  | Unit | Cell 1 | Cell 10 | Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | -85 | Power level values are such that PCell and PSCell1 are used. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | -85 | The power level value of the PSCell1 is changed to create detection of physical layer problems for the PSCell1 i.e. N313 consecutive out-of-sync indications from UE lower layers. |
| Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.5.1.8.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS stops sending SCG RLC acknowledgments | - | - | - | - |
| 2 | The SS transmits one IP packet to the UE on Cell 10 on SCG DRB. | <-- | IP packet | - | - |
| 3 | UE loops back the IP packet on Cell 10 on SCG DRB | --> | IP packet | - | - |
| 4 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’? | --> | SCGFailureInformation | 3 | P |
| 5 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS transmits a PDCCH order from the PSCell1 providing Random Access Preamble. | <-- | (PDCCH Order) | - | - |
| 8 | The UE transmits a preamble on PRACH to the PSCell1 using the same preamble index as given in step 7 | --> | (PRACH Preamble) | - | - |
| 9 | The SS waits for 10 seconds NOTE: The SS does not transmit Random Access Response to the UE. | - | - | - | - |
| 10 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘randomAccessProblem’? | --> | SCGFailureInformation | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message including the IE scg-Configuration that includes the IEs sCellToReleaseListSCG, pSCellToAddMod and mobilityControlInfoSCG modifying the SCG. The SS does not respond to the UE transmitted RACH preambles on Cell 28. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘scg-ChangeFailure’? | --> | SCGFailureInformation | 4 | P |
| 14A | The SS transmits an RRCConnectionReconfiguration message containing the IE scg-Configuration that includes the IEs sCellToReleaseListSCG mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 15A | The SS transmits a CLOSE UE TEST LOOP message with Test loop Mode B and IP PDU delay =10 seconds. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 15B | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 16 | The SS transmits one IP packet to the UE on the DRB associated with the MCG DRB. NOTE: The SS does not provide uplink grants to the UE until cell power levels have been set according to the row “T1” in step 17, to ensure UE does not loop back IP packet before RLF has happened. | <-- | IP packet | - | - |
| 17 | The SS changes Cell 10 parameters according to the row "T1" in table 8.5.1.8.1.3.2-1. | - | - | - | - |
| 18 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘t313-Expiry’? | --> | SCGFailureInformation | 1 | P |
| 19 | The SS changes Cell 10 parameters according to the row "T0" in table 8.5.1.8.1.3.2-1. | - | - | - | - |
| 20 | Check: Does the UE loops back the IP packet on Cell 1 (PCell) on the MCG DRB? | --> | IP packet | 5 | P |

8.5.1.8.1.3.3	Specific message contents

Table 8.5.1.8.1.3.3-1: RRCConnectionReconfiguration (step 5& 11Table 8.5.1.8.1.3.2-2)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE {} |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | v890 |  |  |
| nonCriticalExtension SEQUENCE { |  | v920 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1020 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1130 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1250 |  |  |
| scg-Configuration-r12 CHOICE { | SCG-Configuration-r12- MobSCG |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.5.1.8.1.3.3-2: SCG-Configuration-r12-MobSCG (Table 8.5.1.8.1.3.3-1)

| Derivation Path: 36.508, Clause 4.6.3 Table 4.6.3-19F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 | Not Present |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 | Not Present |  |  |
| sCellToReleaseListSCG-r12 | Not Present |  |  |
| pSCellToAddMod-r12 | Not Present |  |  |
| sCellToAddModListSCG-r12 | Not present |  |  |
| mobilityControlInfoSCG-r12 | mobilityControlInfoSCG-r12 (Table 8.5.1.8.1.3.5) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.1.3.3-3: RRCConnectionReconfiguration (step 13, Table 8.5.1.8.1.3.2-2)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE {} |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRBRECONFIG |  |  |  |
| nonCriticalExtension SEQUENCE { |  | v890 |  |  |
| nonCriticalExtension SEQUENCE { |  | v920 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1020 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1130 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1250 |  |  |
| scg-Configuration-r12 CHOICE { | SCG-Configuration-r12- sCellRelease-pSCellAdd-ModSCG |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.5.1.8.1.3.3-4: SCG-Configuration-r12-sCellRelease-pSCellAdd-ModSCG (Table 8.5.1.8.1.3.3-3)

| Derivation Path: 36.508, Clause 4.6.3 Table 4.6.3-19F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 | Not present |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 | Not present |  |  |
| sCellToReleaseListSCG-r12 | sCellToReleaseListSCG-r12 |  |  |
| pSCellToAddMod-r12 | pSCellToAddMod-r12 |  |  |
| sCellToAddModListSCG-r12 | Not present |  |  |
| mobilityControlInfoSCG-r12 | mobilityControlInfoSCG-r12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.1.3.3-5: sCellToReleaseListSCG-r12 (Table 8.5.1.8.1.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| sCellToReleaseListSCG-r12 |  |  |  |
| { sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 | PSCell release for Cell 10 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.1.3.3-6: pSCellToAddMod-r12 (Table 8.5.1.8.1.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PSCellToAddMod-r12 SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| PSCellToAddMod-r12 SEQUENCE { |  |  |  |
| cellIdentification-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Physical Cell Identity of cell 28 |  |  |
| dl-CarrierFreq-r12 | Same downlink EARFCN as used for Cell 28 |  | Step 13 |
|  | Same downlink EARFCN as used for Cell 28 |  | Step 14A |
| } |  |  |  |
| RadioResourceConfigCommonPSCell-r12 | Same value as 36.508 Clause 4.6.3 Table 4.6.3-19F |  |  |
| RadioResourceConfigDedicatedPSCell-r12 | Same value as 36.508 Clause 4.6.3 Table 4.6.3-19F |  |  |
| AntennaInfoDedicated-v10i0 | Not present |  |  |
| } |  |  |  |

Table 8.5.1.8.1.3.3-7: mobilityControlInfoSCG-r12 (Table 8.5.1.8.1.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| mobilityControlInfoSCG-r12 SEQUENCE { |  | Modifies the SCG |  |
| T307-r12 | ms2000 |  |  |
| C-RNTI | Any allowed value different than previously assigned |  |  |
| RACH-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| CipheringAlgorithm-r12 | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| } |  |  |  |

##### 8.5.1.8.2	Radio link failure on PSCell / UE supports Split DRB

8.5.1.8.2.1	Test Purpose (TP)

(1)

With { UE in connected mode with SCG activated with a Split DRB established }

ensure that {

when { T313 expires }

then { the UE transmits a SCGFailureInformation message with failureType set to ‘t313-Expiry’ }

}

(2)

With { UE in connected mode with SCG activated with a Split DRB established }

ensure that {

when { random access problem indication from SCG MAC }

then { the UE transmits a  SCGFailureInformation message with failureType set to ‘randomAccessProblem’ }

}

(3)

With { UE in connected mode with SCG activated with a Split DRB established }

ensure that {

when { indication from SCG RLC that the maximum number of retransmissions has been reached for a Split DRB }

then { the UE shall transmit a  SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’ }

}

(4)

With { UE in connected mode with SCG activated with a Split DRB established }

ensure that {

when { T307 expires }

then { the UE shall transmit a  SCGFailureInformation message with failureType set to ‘scg-ChangeFailure’ }

}

(5)

With { UE in connected mode with SCG activated with a Split DRB established and with ul-DataSplitDRB-ViaSCG set to FALSE }

ensure that {

when { UE detects radio link failure of the PSCell and has PDCP SDUs available for transmission }

then { the UE transmits the uplink data on the MAC entity configured for MCG }

}

8.5.1.8.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.7a, 5.3.11.3, 5.6.13.2, and 5.6.13.3. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.7a]

The UE shall:

1>	if T307 expires:

NOTE 1:	Following T307 expiry any dedicated preamble, if provided within the rach-ConfigDedicatedSCG, is not available for use by the UE anymore.

2>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG change failure;

[TS 36.331, clause 5.3.11.3]

…

The UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached for an SCG or split DRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.13.2]

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a;

Upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

[TS 36.331, clause 5.6.13.3]

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2>	include failureType and set it to the trigger for detecting SCG radio link failure;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

2>	include failureType and set it to scg-ChangeFailure;

1>	set the measResultServFreqList to include for each SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16];

1>	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

8.5.1.8.2.3	Test description

8.5.1.8.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 10 is the PSCell1 and Cell 28 is the PSCell2.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 10 and Cell 28.

UE:

Preamble:

-	The UE is in state DC Split DRB Loopback Activated (state 6B with Test loop Mode B) on Cell 1 and Cell 10 according to [18].

8.5.1.8.2.3.2	Test procedure sequence

Table 8.5.1.8.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.1.8.2.3.2-1: Time instances of cell power level and parameter changes

|  |  | Unit | Cell 1 | Cell 10 | Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | -85 | Power level values are such that PCell and PSCell1 are used. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | -85 | The power level value of the PSCell1 is changed to create detection of physical layer problems for the PSCell1 i.e. N313 consecutive out-of-sync indications from UE lower layers. |
| Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 8.5.1.8.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS stops sending SCG RLC acknowledgments | - | - | - | - |
| 2 | The SS transmits one IP packet to the UE on Cell 10 on split DRB. | <-- | IP packet | - | - |
| 3 | UE loops back the IP packet on Cell 10 on SCG DRB | --> | IP packet | - | - |
| 4 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘rlc-MaxNumRetx’? | --> | SCGFailureInformation | 3 | P |
| 5 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS transmits a PDCCH order from the PSCell1 providing Random Access Preamble. | <-- | (PDCCH Order) | - | - |
| 8 | The UE transmits a preamble on PRACH to the PSCell1 using the same preamble index as given in step 7 | --> | (PRACH Preamble) | - | - |
| 9 | The SS waits for 10 seconds NOTE: The SS does not transmit Random Access Response to the UE. | - | - | - | - |
| 10 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘randomAccessProblem’ ? | --> | SCGFailureInformation | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message including the IE scg-Configuration that includes the IEs sCellToReleaseListSCG, pSCellToAddMod and mobilityControlInfoSCG modifying the SCG. The SS does not respond to the UE transmitted RACH preambles. on Cell 28 | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘scg-ChangeFailure’? | --> | SCGFailureInformation | 4 | P |
| 14A | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfoSCG-r12 | <-- | RRCConnectionReconfiguration | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 15A | The SS transmits a CLOSE UE TEST LOOP message with Test loop Mode B and IP PDU delay =10 seconds. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 15B | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 16 | The SS transmits one IP packet to the UE on the DRB associated with the MCG DRB. NOTE: The SS does not provide uplink grants to the UE until cell power levels have been set according to the row “T1” in step 17, to ensure UE does not loop back IP packet before RLF has happened. | <-- | IP packet | - | - |
| 17 | The SS changes Cell 10 parameters according to the row "T1" in table 8.5.1.8.1.3.2-1. | - | - | - | - |
| 18 | Check: Does the UE transmit an SCGFailureInformation message with failureType set to ‘t313-Expiry’ ? | --> | SCGFailureInformation | 1 | P |
| 19 | The SS changes Cell 10 parameters according to the row "T0" in table 8.5.1.8.1.3.2-1. | - | - | - | - |
| 20 | Check: Does the UE loops back the IP packet on Cell 1 (PCell) on the MCG DRB? | --> | IP packet | 5 | P |

8.5.1.8.2.3.3	Specific message contents

Table 8.5.1.8.2.3.3-1: RRCConnectionReconfiguration (step 5 & 11 Table 8.5.1.8.2.3.2-2)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE {} |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRBRECONFIG |  |  |  |
| nonCriticalExtension SEQUENCE { |  | v890 |  |  |
| nonCriticalExtension SEQUENCE { |  | v920 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1020 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1130 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1250 |  |  |
| scg-Configuration-r12 CHOICE { | SCG-Configuration-r12- MobSCG |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.5.1.8.2.3.3-2: SCG-Configuration-r12-MobSCG (Table 8.5.1.8.2.3.3-1)

| Derivation Path: 36.508, Clause 4.6.3 Table 4.6.3-19F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 | Not Present |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 | Not Present |  |  |
| sCellToReleaseListSCG-r12 | Not Present |  |  |
| pSCellToAddMod-r12 | Not Present |  |  |
| sCellToAddModListSCG-r12 | Not present |  |  |
| mobilityControlInfoSCG-r12 | mobilityControlInfoSCG-r12 (Table 8.5.1.8.1.3.7) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.2.3.3-3: RRCConnectionReconfiguration (step 13 & 14A, Table 8.5.1.8.2.3.2-2)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE {} |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRBRECONFIG |  |  |  |
| nonCriticalExtension SEQUENCE { |  | v890 |  |  |
| nonCriticalExtension SEQUENCE { |  | v920 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1020 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1130 |  |  |
| nonCriticalExtension SEQUENCE { |  | v1250 |  |  |
| scg-Configuration-r12 CHOICE { | SCG-Configuration-r12- sCellRelease-pSCellAdd-ModSCG |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.5.1.8.2.3.3-4: SCG-Configuration-r12-sCellRelease-pSCellAdd-ModSCG (Table 8.5.1.8.2.3.3-3)

| Derivation Path: 36.508, Clause 4.6.3 Table 4.6.3-19F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 | Not present |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 | Not present |  |  |
| sCellToReleaseListSCG-r12 | sCellToReleaseListSCG-r12 |  |  |
| pSCellToAddMod-r12 | pSCellToAddMod-r12 |  |  |
| sCellToAddModListSCG-r12 | Not present |  |  |
| mobilityControlInfoSCG-r12 | mobilityControlInfoSCG-r12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.2.3.3-5: sCellToReleaseListSCG-r12 (Table 8.5.1.8.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| sCellToReleaseListSCG-r12 |  |  |  |
| { sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 | PSCell release for Cell 10 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.1.8.2.3.3-6: pSCellToAddMod-r12 (Table 8.5.1.8.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PSCellToAddMod-r12 SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| PSCellToAddMod-r12 SEQUENCE { |  |  |  |
| cellIdentification-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Physical Cell Identity of cell 28 |  |  |
| dl-CarrierFreq-r12 | Same downlink EARFCN as used for Cell 28 |  | Step 13 |
| dl-CarrierFreq-r12 | Same downlink EARFCN as used for Cell 10 |  | Step 14A |
| } |  |  |  |
| RadioResourceConfigCommonPSCell-r12 | Same value as 36.508 Clause 4.6.3 Table 4.6.3-19F |  |  |
| RadioResourceConfigDedicatedPSCell-r12 | Same value as 36.508 Clause 4.6.3 Table 4.6.3-19F |  |  |
| AntennaInfoDedicated-v10i0 | Not present |  |  |
| } |  |  |  |

Table 8.5.1.8.2.3.3-7: mobilityControlInfoSCG-r12 (Table 8.5.1.8.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| mobilityControlInfoSCG-r12 SEQUENCE { |  | Modifies the SCG |  |
| T307-r12 | ms2000 |  |  |
| C-RNTI | Any allowed value different than previously assigned |  |  |
| RACH-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| CipheringAlgorithm-r12 | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| } |  |  |  |

#### 8.5.1.9	Radio link failure / RRC connection re-establishment success/ Release configured UDC

##### 8.5.1.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with UDC configuration}

ensure that {

when { UE successfully completes the RRC Connection re-establishment procedure }

then { UE release the configured UDC }

}

##### 8.5.1.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.7.2.

[TS 36.331 clause 5.3.7.2]

The UE shall only initiate the procedure either when AS security has been activated or for a NB-IoT UE supporting RRC connection re-establishment for the Control Plane CIoT EPS optimisation. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5; or

1>	upon an RRC connection reconfiguration failure, in accordance with TS38.331 [82, 5.3.5.5].

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T313, if running;

1>	stop timer T307, if running;

1>	start timer T311;

1>	stop timer T370, if running;

1>	release uplinkDataCompression, if configured;

##### 8.5.1.9.3	Test description

8.5.1.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

-	The condition UDC is used for PDCP-Config in step 8 in 4.5.3.3 according to [18].

8.5.1.9.3.2	Test procedure sequence

Table 8.5.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 3 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. After having sent a PDU, the SS set Next_PDCP_TX_SN= 1. | <-- | PDCP PDU DATA #0 | - | - |
| 3 | The UE sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. | --> | PDCP PDU DATA #0 | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. After having sent a PDU, the SS set Next_PDCP_TX_SN= 2. | <-- | PDCP PDU DATA #1 | - | - |
| 5 | The UE sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 1). Data is previously received data from PDU #1 after decompression. | --> | PDCP PDU DATA #1 | - | - |
| 6 | The SS changes the power level of Cell 1 to non-suitable “Off” according to TS 36.508 subclause 6.2.2.1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 7 | Wait for 12s (T311 (10s) is transmitted in SIB2). | - | - | - | - |
| 8 | The SS changes the power level of Cell 1 to “Serving Cell” according to TS 36.508 subclause 6.2.2.1. | - | - | - | - |
| 9 | The UE send RRCConnectionReestablishmentRequest message on Cell 1? | --> | RRCConnectionReestablishmentRequest | - | - |
| 10 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 11 | The UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 14 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU DATA #0 | - | - |
| 15 | Check: Does the UE sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 1, Checksum = 1111. Data is previously received data from PDU #0 after decompression. | --> | PDCP PDU DATA #0 | 1 | P |
| Note 1: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

8.5.1.9.3.3	Specific message contents

None.

### 8.5.2	Redirection to E-UTRAN

#### 8.5.2.1	Redirection to E-UTRAN / From UTRAN upon reception of RRC CONNECTION REJECT

8.5.2.1.1	Test Purpose (TP)

(1)

with { UE in UTRA Idle state }

ensure that {

when { UE is requested to make an outgoing PS call }

then { UE includes in the RRC CONNECTION REQUEST the IE Pre-Redirection info" }

}

(2)

with { UE in UTRA CELL_DCH state }

ensure that {

when { UE receives an RRC CONNECTION REJECT message including an IE Redirection info with E-UTRA target info E-UTRA frequency }

then { UE enters RRC_IDLE state on E-UTRAN Carrier included in IE Redirection info }

}

8.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 25.331, clause  8.1.3.3, 8.1.4.3 and clause 8.5.2.

[TS 25.331, clause 8.1.3.3]

The UE shall, in the transmitted RRC CONNECTION REQUEST message:

…

1>	if the UE supports E-UTRA:

2>	if the variable EUTRA_FREQUENCY_INFO_LIST contains no E-UTRA frequencies:

3>	include the IE "Pre-Redirection info";

3>	if the UE supports E-UTRA FDD:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports E-UTRA TDD:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

2>	if the UE supports any of the bands that the E-UTRA frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

3>	include the IE "Pre-Redirection info";

3>	if the UE supports any of the bands that the E-UTRA FDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports any of the bands that the E-UTRA TDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

[TS 25.331, clause 8.1.3.9]

When the UE receives an RRC CONNECTION REJECT message on the downlink CCCH, it shall compare the value of the IE "Initial UE identity" in the received RRC CONNECTION REJECT message with the value of the variable INITIAL_UE_IDENTITY:

…

1>	if the IE "inter-RAT info" is present:

2>	if the IE "wait time" = '0':

3>	the UE behaviour is not specified.

2>	if V300 is equal to or smaller than N300:

3>	if the IE "GSM target cell info" is present:

4>	attempt to camp on a suitable cell of the list of cells indicated for that RAT;

4>	if the UE selects and camps on one of the cells indicated for that RAT:

5>	disable cell reselection to the original RAT until the time stated in the IE "wait time" has elapsed.

4>	if the UE cannot find any suitable cell from the indicated ones within 10s, the UE is allowed to camp on any suitable cell on that RAT.

5>	after having selected and camped on a suitable cell on the designated RAT:

6>	the UE may disable cell reselection to the original RAT until the time stated in the IE "wait time" has elapsed.

3>	if the IE "E-UTRA target info" is present:

4>	attempt to camp on a suitable cell on one of the frequencies indicated for that RAT, excluding any cell indicated in the list of not allowed cells for that RAT (e.g. the " exclude-listed cells per freq list " for E-UTRA), if present;

4>	if the UE selects and camps on one such cell:

5>	disable cell reselection to the original RAT until the time stated in the IE "wait time" has elapsed.

4>	if the UE cannot find any suitable cell on the indicated frequencies within 10s, the UE is allowed to camp on any suitable cell on that RAT:

5>	after having selected and camped on a suitable cell on the designated RAT:

6>	disable cell reselection to the original RAT until the time stated in the IE "wait time" has elapsed.

8.5.2.1.3	Test description

8.5.2.1.3.1	Pre-test conditions

System Simulator:

-	2 cells, one UTRA and one E-UTRA cell:

-	Cell 5 UTRA serving cell (priority 4 default)

-	Cell 1 suitable neighbour E-UTRA cell (priority 3)

UE:

UTRAN Idle state

Preamble:

The UE is in state Registered, Idle mode according to section 4.5A.3A [18] on Cell 5 (serving cell).

8.5.2.1.3.2	Test procedure sequence

Table 8.5.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE initiate an outgoing PS call. | - | - | - | - |
| 2 | Check: does the UE include the IE Pre-redirection info with Support of E-UTRA set to TRUE? | --> | RRC CONNECTION REQUEST | 1 | P |
| 3 | The SS transmit a RRC CONNECTION REJECT | <-- | RRC CONNECTION REJECT | - | - |
| - | EXCEPTION : Step 4a1 to 4b6 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN | - | - | - | - |
| 4 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |
| 4b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? | - | - | - | - |
| 4b2 | The UE transmits a RRCConnectionRequest message on E-UTRAN Cell1 | --> | RRC: RRCConnectionRequest | - | - |
| 4b3 | SS transmit an RRCConnectionSetup message | <-- | RRC: RRCConnectionSetup | - | - |
| 4b4 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a EMM SERVICE REQUEST message | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | - | - |
| 4b5 | SS transmits a SERVICE REJECT with cause Protocol error, unspecified | <-- | SERVICE REJECT | - | - |
| 4b6 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |

8.5.2.1.3.3	Specific message or IE contents

Table 8.5.2.1.3.3-0: Conditions for specific message contents
in Table 8.5.2.1.3.3-1

| Condition | Explanation |
| --- | --- |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 8.5.2.1.3.3-1 System Information Block type 19 for cell 5 (preamble and all steps, Table 8.5.2.1.3.2-1)

| Derivation Path: 36.508 Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry |  |  |
| earfcn[1] | Downlink EARFCN of Cell 1 |  |  |
| priority[1] | 3 |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| earfcn [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [1] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.2.1.3.3-2: RRC CONNECTION REQUEST (UTRA Rel-8)

| Derivation path: 34.108 default RRC CONNECTION REQUEST in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info |  | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to |  |
| Support of E-UTRA FDD | TRUE |  | E-UTRA-FDD |
| Support of E-UTRA TDD | TRUE |  | E-UTRA-TDD |
| Domain indicator | PS domain |  |  |

Table 8.5.2.1.3.3-3: RRC CONNECTION REJECT (UTRA Rel-8)

| Derivation path: 34.108 default RRC CONNECTION REJECT in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Wait Time | 15 |  |  |
| Redirection info |  |  |  |
| Inter-RAT info | E-UTRA |  |  |
| E-UTRA target info |  |  |  |
| E-UTRA Target Frequency Info List | 1 Entry |  |  |
| DL Carrier frequency | EARFCN of the downlink Cell 1 carrier frequency |  |  |
| DL Carrier frequency | 65535 |  | Band > 64 |
| E-UTRA Target Frequency Info extension List |  |  | Band > 64 |
| EARFCN extension | EARFCN of the downlink Cell 1 carrier frequency |  |  |

Table 8.5.2.1.3.3-4: SERVICE REJECT (step 4b5, Table 8.5.2.1.3.2-1)

| Derivation Path: 36.508 table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | ‘01101111’B | Protocol error, unspecified |  |

### 8.5.3	Void

### 8.5.4	UE capability transfer

#### 8.5.4.1	UE capability transfer / Success

8.5.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message before AS security is activated }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message after AS security is activated }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.5.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.3.3, 5.6.22.2, 5.6.22.3 and TS 25.331, clause 8.1.16.3.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes ‘eutra’:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to ‘eutra’;

2>	if the ue-CapabilityRequest includes ‘geran-cs’ and if the UE supports GERAN CS domain:

3>	include the UE radio access capabilities for GERAN CS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-cs’;

2>	if the ue-CapabilityRequest includes ‘geran-ps’ and if the UE supports GERAN PS domain:

3>	include the UE radio access capabilities for GERAN PS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-ps’;

2>	if the ue-CapabilityRequest includes ‘utra’ and if the UE supports UTRA:

3>	include the UE radio access capabilities for UTRA within a ueCapabilityRAT-Container and with the rat-Type set to ‘utra’;

2>	if the ue-CapabilityRequest includes ‘cdma2000-1XRTT’ and if the UE supports CDMA2000-1xRTT:

3>	include the UE radio access capabilities for CDMA 2000 within a ueCapabilityRAT-Container and with the rat-Type set to ‘cdma2000-1XRTT’;

…

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

1> submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 25.331, clause 8.1.16.3]

The UE shall:

1>	if the UE supports both UMTS TDD and FDD:

2>	determine whether capabilities are being requested for FDD or TDD mode; and

2>	use the capabilities associated with that mode for the remainder of this procedure.

1>	include the IE "UE security information", and the IE "UE security information2" if inter-RAT PS handover is supported by the UE; and

1>	not include the IE "UE Specific Behaviour Information 1 interRAT";

1>	in case support for the compressed version of the inter RAT handover info is indicated via the other radio access technology:

2>	if the other radio access technology is not E-UTRA:

3>	include of the following IEs the IE that after encoding has the smallest size: IE "Predefined configuration status information compressed" or the IE "Predefined configuration status information".

2>	else:

3>	exclude the IE "Predefined configuration status information" and "Predefined configuration status information compressed".

2>	include the IE "UE radio access capability compressed".

1>	else:

2>	if the other radio access technology is not E-UTRA:

3>	include the IE "Predefined configuration status information".

2>	else:

3>	exclude the IE "Predefined configuration status information".

2>	include the IE "UE capability container", containing the IE "UE radio access capability" and the IE "UE radio access capability extension", in accordance with the following:

3>	if the UE supports multiple UTRA FDD Frequency Bands; or

3>	if the UE supports a single UTRA FDD Frequency Band different from Band I [21]; or

3>	if the UE supports E-UTRA:

4>	include the IE "UE radio access capability", excluding IEs "RF capability FDD" and "Measurement capability" for FDD and including the IE "Measurement capability TDD" for TDD;

4>	include the IE "UE radio access capability extension", including the IEs "RF capability FDD extension", the "Measurement capability extension", the "Additional Secondary Cells" and the "Non-contiguous multi-cell" associated with each supported UTRA FDD frequency band indicated in the IE "Frequency band", but may omit all or part of these IEs for supported inter-RAT bands.

3>	else:

4>	include the IE "UE radio access capability", including the IEs "RF capability FDD" and "Measurement capability" associated with the Band I [21] for FDD and excluding the IE "Measurement capability TDD" for TDD;

4>	include the IE "UE radio access capability extension", including the IEs "RF capability FDD extension", the "Measurement capability extension", the "Additional Secondary Cells" and the "Non-contiguous multi-cell" associated with each supported UTRA FDD frequency band indicated in the IE "Frequency band".

1>	For FDD, include the IE "UE radio access capability comp 2";

1>	For 1.28 Mcps TDD, include the IE "UE radio access capability comp for 1.28 Mcps TDD";

1>	initiate the transfer of the INTER RAT HANDOVER INFO message via the other radio access technology, using radio access technology-specific procedures;

1>	store the following in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED if they were included in the INTER RAT HANDOVER INFO message:

2>	the IE "Predefined configuration status information";

2>	the IE "Predefined configuration status information compressed";

2>	the IE "UE security information";

2>	the IE "UE security information2";

2>	the IE "UE radio access capability";

2>	the IE "UE radio access capability extension"; and

2>	the IE "UE radio access capability compressed";

2>	if the IE "UE radio access capability compressed" were included in the INTER RAT HANDOVER INFO message:

3>	set the IE "Security Capability" to the mandatory R99 algorithms.

1>	and the procedure ends.

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.5.4.1.3	Test description

8.5.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

8.5.4.1.3.2	Test procedure sequence

Table 8.5.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-8 | Steps 2-8 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 10a1 and 10a2 will be performed else step 10b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 10a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 10a1 | The UE sends segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 10a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 10b1 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 1 | P |
| 11 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | SecurityModeCommand | - | - |
| 12 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | SecurityModeComplete | - | - |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred after SECURITY MODE COMPLETE message. | - | - | - | - |
| 13a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits a DLInformationTransfer message. This message includes an ESM INFORMATION REQUEST message. | <-- | DLInformationTransfer | - | - |
| 13a2 | The UE transmits a ULInformationTransfer message. This message includes an ESM INFORMATION RESPONSE message. | --> | ULInformationTransfer | - | - |
| 14-16c1 | Steps 14-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 17 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 18a1 and 18a2 will be performed else step 18b1 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 18a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 18a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? | --> | ULDedicatedMessageSegment | - | - |
| 18a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 2 | P |
| 18b1 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 2 | P |
| - | EXCEPTION: Steps 19a1 to 19a2 describe behaviour that depends on the UE capability. | - | - | - | - |
| 19a1 | IF pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR, pc_GERAN, pc_1xRTT or pc_HRPD THEN the SS transmits a UECapabilityEnquiry message to request UE radio access capability information for every other supported RATs. | <-- | UECapabilityEnquiry | - | - |
| 19a2 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 2 | P |
| 20-22 | IF MULTI_PDN = TRUE (NOTE) THEN steps 10-12 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| NOTE: MULTI_PDN as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

8.5.4.1.3.3	Specific message contents

Table 8.5.4.1.3.3-1: UEcapabilityEnquiry (step 9 and 17, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type[1] | eutra |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-2: UECapabilityInformation (step 10a2 and 18b1, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment |  | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry only |  |  |  |
| rat-Type[1] | eutra | E-UTRA only |  |  |
| ueCapabilitiesRAT-Container[1] OCTET STRING { |  |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |  |
| accessStratumRelease | Not checked | Value should be based on Rel of Access stratum supported |  |  |
| ue-Category | Checked against UE Category indications in the PICS |  |  | Rel-8 or Rel-9 |
| pdcp-Parameters | Not checked |  |  | Rel-8 |
| pdcp-Parameters SEQUENCE { |  |  |  | > Rel-8 |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |
| profile0x0001 | Checked |  |  | pc_ROHC_profile0x0001 |
| profile0x0002 | Checked |  |  | pc_ROHC_profile0x0002 |
| profile0x0003 | Not checked |  |  |  |
| profile0x0004 | Not checked |  |  |  |
| profile0x0006 | Not checked |  |  |  |
| profile0x0101 | Not checked |  |  |  |
| profile0x0102 | Not checked |  |  |  |
| profile0x0103 | Not checked |  |  |  |
| profile0x0104 | Not checked |  |  |  |
| } |  |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |  |
| } |  |  |  |  |
| phyLayerParameters SEQUENCE { |  |  |  |  |
| ul-AntennaSelectionSupported | Not checked |  |  |  |
| ue-SpecificRefSigsSupported | Not checked |  |  |  |
| } |  |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 1 to 64, n3 is the sum of reported bands without pc_eBandα_Supp for α= 1 to 64, and n4 is the number of extended bands indicated as maxFBI (=64) corresponding to the extended bands in supportedBandListEUTRA_v9e0 (Note 14). | n1= n2+n3+n4 |  | not pc_FDD_TypeB_HalfDuplex |
| BandListEUTRA SEQUENCE { |  |  |  |  |
| bandEUTRA [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α – 1. Except bands indicated as extended bands. |  |  |  |
| halfDuplex[α = 1..n2] | False |  |  | FDD |
| } |  |  |  |  |
| } |  |  |  |  |
| supportedBandListEUTRA (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 1 to 64, n3 is the sum of reported bands without pc_eBandα_Supp for α= 1 to 64, and n4 is the number of extended bands indicated as maxFBI (=64) corresponding to the extended bands in supportedBandListEUTRA_v9e0 (Note 14). | n1=n2+n3+n4 |  | pc_FDD_TypeB_HalfDuplex |
| BandListEUTRA SEQUENCE { |  |  |  |  |
| bandEUTRA [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α – 1. . Except bands indicated as extended bands. |  |  |  |
| halfDuplex[α = 1..n2] | true |  |  | FDD |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measurementParameters SEQUENCE { |  |  |  |  |
| BandListEUTRA(SIZE (1..maxBands)) OF SEQUENCE { | same number of entries like in SupportedBandListEUTRA |  |  |  |
| BandInfoEUTRA SEQUENCE { |  |  |  |  |
| InterFreqBandList (SIZE (1..maxBands)) OF SEQUENCE { | m entries (contents not checked, but m shall be equal to the number of bands listed in each IE present in interRAT-Parameters) |  |  |  |
| InterFreqBandInfo |  |  |  |  |
| } |  |  |  |  |
| InterRAT-BandList (SIZE (1..maxBands)) OF SEQUENCE { |  |  |  |  |
| InterRAT-BandInfo | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureGroupIndicators | shall be set according to the corresponding PICS items |  |  |  |
| -- FGI 1 | Checked |  |  |  |
| -- FGI 2 | Checked |  |  |  |
| -- FGI 3 | Checked |  |  |  |
| -- FGI 4 | Checked |  |  |  |
| -- FGI 5 | Checked |  |  |  |
| -- FGI 6 | Checked |  |  |  |
| -- FGI 7 | Checked |  |  |  |
| -- FGI 8 | Checked |  |  |  |
| -- FGI 9 | Checked |  |  |  |
| -- FGI 10 | Checked |  |  |  |
| -- FGI 11 | Checked |  |  |  |
| -- FGI 12 | Checked |  |  |  |
| -- FGI 13 | Checked |  |  |  |
| -- FGI 14 | Checked |  |  |  |
| -- FGI 15 | Checked |  |  |  |
| -- FGI 16 | Checked |  |  |  |
| -- FGI 17 | Checked |  |  |  |
| -- FGI 18 | Checked |  |  |  |
| -- FGI 19 | Checked |  |  |  |
| -- FGI 20 | Checked |  |  |  |
| -- FGI 21 | Checked |  |  |  |
| -- FGI 22 | Checked |  |  |  |
| -- FGI 23 | Checked |  |  |  |
| -- FGI 24 | Checked |  |  |  |
| -- FGI 25 | Checked |  |  |  |
| -- FGI 26 | Checked |  |  |  |
| -- FGI 27 | Checked |  |  |  |
| -- FGI 28 | Checked |  |  |  |
| -- FGI 29 | Checked |  |  |  |
| -- FGI 30 | Checked |  |  |  |
| -- FGI 31 | Checked |  |  |  |
| -- FGI 32 | ‘0’B (Undefined) |  |  |  |
| interRAT-Parameters SEQUENCE { | m elements are present |  |  |  |
| utraFDD | Present but value not checked | See Note 11 |  | pc_FDD AND (px_ReportUTRA_RAT=fdd OR px_ReportUTRA_RAT=both) |
| utraTDD128 | Present but value not checked | See Note 11 |  | pc_TDD_LCR AND Not px_NoTDD_LCR_onLTE AND (px_ReportUTRA_RAT=tdd OR px_ReportUTRA_RAT=both) Note 4 |
| utraTDD384 | Present but value not checked | See Note 11 |  | pc_TDD_HCR AND (px_ReportUTRA_RAT=tdd OR px_ReportUTRA_RAT=both) |
| utraTDD768 | Present but value not checked | See Note 11 |  | pc_TDD_VHCR AND (px_ReportUTRA_RAT=tdd OR pc_ReportUTRA_RAT=both) |
| geran | Present but value not checked |  |  | pc_GERAN |
| cdma2000-HRPD | Present but value not checked |  |  | pc_HRPD |
| cdma2000-1xRTT | Present but value not checked |  |  | pc_1xRTT |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| phyLayerParameters-v920 | Not checked |  |  |  |
| interRAT-ParametersGERAN-v920 | Not checked |  |  |  |
| interRAT-ParametersUTRA-v920 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-v920 | Not checked |  |  |  |
| deviceType-r9 | Not checked |  |  |  |
| csg-ProximityIndicationParameters-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| son-Parameters-r9 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |  |
| featureGroupIndRel9Add-r9 | If present, shall be set according to the corresponding PICS items |  |  |  |
| -- FGI 33 | Checked |  |  |  |
| -- FGI 34 | Checked |  |  |  |
| -- FGI 35 | Checked |  |  |  |
| -- FGI 36 | Checked |  |  |  |
| -- FGI 37 | Checked |  |  |  |
| -- FGI 38 | Checked |  |  |  |
| -- FGI 39 | Checked |  |  |  |
| -- FGI 40 | Checked |  |  |  |
| -- FGI 41 | Checked |  |  |  |
| -- FGI 42-64 | ‘0’B (Undefined) |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |
| phyLayerParameters-r9 | Not checked |  |  |  |
| featureGroupIndicators-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 1_F | Checked |  |  |  |
| -- FGI 2_F | Checked |  |  |  |
| -- FGI 3_F | Checked |  |  | FDD = TDD |
| -- FGI 4_F | Checked |  |  |  |
| -- FGI 5_F | Checked |  |  | FDD = TDD |
| -- FGI 6_F | Checked |  |  | FDD = TDD |
| -- FGI 7_F | Checked |  |  | FDD = TDD |
| -- FGI 8_F | Checked |  |  |  |
| -- FGI 9_F | Checked |  |  |  |
| -- FGI 10_F | Checked |  |  |  |
| -- FGI 11_F | Checked |  |  |  |
| -- FGI 12_F | Checked |  |  |  |
| -- FGI 13_F | Checked |  |  | FDD = TDD |
| -- FGI 14_F | Checked |  |  | FDD = TDD |
| -- FGI 15_F | Checked |  |  |  |
| -- FGI 16_F | Checked |  |  |  |
| -- FGI 17_F | Checked |  |  | FDD = TDD |
| -- FGI 18_F | Checked |  |  | FDD = TDD |
| -- FGI 19_F | Checked |  |  |  |
| -- FGI 20_F | Checked |  |  | FDD = TDD |
| -- FGI 21_F | Checked |  |  | FDD = TDD |
| -- FGI 22_F | Checked |  |  |  |
| -- FGI 23_F | Checked |  |  |  |
| -- FGI 24_F | Checked |  |  |  |
| -- FGI 25_F | Checked |  |  | FDD = TDD |
| -- FGI 26_F | Checked |  |  |  |
| -- FGI 27_F | Checked |  |  |  |
| -- FGI 28_F | Checked |  |  |  |
| -- FGI 29_F | Checked |  |  |  |
| -- FGI 30_F | Checked |  |  | FDD = TDD |
| -- FGI 31_F | Checked |  |  |  |
| -- FGI 32_F | ‘0’B (Undefined) |  |  |  |
| featureGroupIndRel9Add-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 33_F | Checked |  |  |  |
| -- FGI 34_F | Checked |  |  |  |
| -- FGI 35_F | Checked |  |  |  |
| -- FGI 36_F | Checked |  |  |  |
| -- FGI 37_F | Checked |  |  |  |
| -- FGI 38_F | Checked |  |  |  |
| -- FGI 39_F | Checked |  |  |  |
| -- FGI 40_F | Checked |  |  |  |
| -- FGI 41_F | Checked |  |  |  |
| -- FGI 42_F | Checked |  |  |  |
| -- FGI 43-64_F | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersUTRA-r9 | Not checked |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |
| phyLayerParameters-r9 | Not checked |  |  |  |
| featureGroupIndicators-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 1_T | Checked |  |  |  |
| -- FGI 2_T | Checked |  |  |  |
| -- FGI 3_T | Checked |  |  | FDD = TDD |
| -- FGI 4_T | Checked |  |  |  |
| -- FGI 5_T | Checked |  |  | FDD = TDD |
| -- FGI 6_T | Checked |  |  | FDD = TDD |
| -- FGI 7_T | Checked |  |  | FDD = TDD |
| -- FGI 8_T | Checked |  |  |  |
| -- FGI 9_T | Checked |  |  |  |
| -- FGI 10_T | Checked |  |  |  |
| -- FGI 11_T | Checked |  |  |  |
| -- FGI 12_T | Checked |  |  |  |
| -- FGI 13_T | Checked |  |  | FDD = TDD |
| -- FGI 14_T | Checked |  |  | FDD = TDD |
| -- FGI 15_T | Checked |  |  |  |
| -- FGI 16_T | Checked |  |  |  |
| -- FGI 17_T | Checked |  |  | FDD = TDD |
| -- FGI 18_T | Checked |  |  | FDD = TDD |
| -- FGI 19_T | Checked |  |  |  |
| -- FGI 20_T | Checked |  |  | FDD = TDD |
| -- FGI 21_T | Checked |  |  | FDD = TDD |
| -- FGI 22_T | Checked |  |  |  |
| -- FGI 23_T | Checked |  |  |  |
| -- FGI 24_T | Checked |  |  |  |
| -- FGI 25_T | Checked |  |  | FDD = TDD |
| -- FGI 26_T | Checked |  |  |  |
| -- FGI 27_T | Checked |  |  |  |
| -- FGI 28_T | Checked |  |  |  |
| -- FGI 29_T | Checked |  |  |  |
| -- FGI 30_T | Checked |  |  | FDD = TDD |
| -- FGI 31_T | Checked |  |  |  |
| -- FGI 32_T | ‘0’B (Undefined) |  |  |  |
| featureGroupIndRel9Add-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 33_T | Checked |  |  |  |
| -- FGI 34_T | Checked |  |  |  |
| -- FGI 35_T | Checked |  |  |  |
| -- FGI 36_T | Checked |  |  |  |
| -- FGI 37_T | Checked |  |  |  |
| -- FGI 38_T | Checked |  |  |  |
| -- FGI 39_T | Checked |  |  |  |
| -- FGI 40_T | Checked |  |  |  |
| -- FGI 41_T | Checked |  |  |  |
| -- FGI 42_T | Checked |  |  |  |
| -- FGI 43-64_T | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersUTRA-r9 | Not checked |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| interRAT-ParametersUTRA-v9c0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| phyLayerParameters-v9d0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| rf-Parameters-v9e0 SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA-v9e0 (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 65 to 256 and n3 is the sum of reported bands without pc_eBandα_Supp for α= 65 to 256 | n1= n2+n3 |  |  |
| BandListEUTRA-v9e0 SEQUENCE { |  |  |  |  |
| bandEUTRA-v9e0 [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α - 1 | band 65 onwards, up to 64 bands |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| ue-Category-v1020 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| phyLayerParameters-v1020 | Not checked |  |  |  |
| rf-Parameters-v1020 { | Not checked |  |  |  |
| supportedBandCombination-r10 { | For as many band combinations as supported |  |  | Various conditions (Note 5) |
| BandCombinationParameters-r10 { |  |  |  |  |
| BandParameters-r10 { |  |  |  |  |
| bandEUTRA-r10 { | Not checked |  |  |  |
| } |  |  |  |  |
| BandParameterUL-r10 { |  |  |  |  |
| CA-MIMO-ParametersUL-r10 { | For as many bandwidth classes as supported |  |  |  |
| ca-BandwidthClassUL-r10 | Not checked |  |  |  |
| supportedMIMO-CapabilityUL-r10 | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| BandParameterDL-r10 { | For as many bandwidth classes as supported |  |  |  |
| CA-MIMO-ParametersDL-r10 { |  |  |  |  |
| ca-BandwidthClassDL-r10 | Not checked |  |  |  |
| supportedMIMO-CapabilityDL-r10 | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measParameters-v1020 { | Not checked |  |  |  |
| bandCombinationListEUTRA-r10 { | For as many band combinations as supported |  |  |  |
| BandInfoEUTRA { |  |  |  |  |
| interFreqBandList { |  |  |  |  |
| InterFreqBandInfo { |  |  |  |  |
| interFreqNeedForGaps | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| interRAT-BandList { |  |  |  |  |
| InterRAT-BandInfo { |  |  |  |  |
| interRAT-NeedForGaps | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureGroupIndRel10-r10 | Shall be set according to the corresponding PICS items. Checked. | BITSTRING 32 |  |  |
| -- FGI 101 | Checked |  |  |  |
| -- FGI 102 | Checked |  |  |  |
| -- FGI 103 | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104 | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105 | Checked |  |  |  |
| -- FGI 106 | Checked |  |  |  |
| -- FGI 107 | Checked |  |  |  |
| -- FGI 108 | Checked |  |  |  |
| -- FGI 109 | Checked |  |  |  |
| -- FGI 110 | Checked |  |  |  |
| -- FGI 111 | Checked |  |  |  |
| -- FGI 112 | Checked |  |  |  |
| -- FGI 113 | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114 | Checked |  |  |  |
| -- FGI 115 | Checked |  |  |  |
| -- FGI 116 | Checked |  |  |  |
| -- FGI 117-132 | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersCDMA2000-v1020 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-r10 | Not checked |  |  |  |
| interRAT-ParametersUTRA-TDD-v1020 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1060 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 101_F | Checked |  |  |  |
| -- FGI 102_F | Checked |  |  |  |
| -- FGI 103_F | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104_F | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105_F | Checked |  |  |  |
| -- FGI 106_F | Checked |  |  |  |
| -- FGI 107_F | Checked |  |  |  |
| -- FGI 108_F | Checked |  |  |  |
| -- FGI 109_F | Checked |  |  |  |
| -- FGI 110_F | Checked |  |  |  |
| -- FGI 111_F | Checked |  |  |  |
| -- FGI 112_F | Checked |  |  |  |
| -- FGI 113_F | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114_F | Checked |  |  |  |
| -- FGI 115_F | Checked |  |  |  |
| -- FGI 116_F | Checked |  |  |  |
| -- FGI 117-132_F | ‘0’B (Undefined) |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1060 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 101_T | Checked |  |  |  |
| -- FGI 102_T | Checked |  |  |  |
| -- FGI 103_T | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104_T | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105_T | Checked |  |  |  |
| -- FGI 106_T | Checked |  |  |  |
| -- FGI 107_T | Checked |  |  |  |
| -- FGI 108_T | Checked |  |  |  |
| -- FGI 109_T | Checked |  |  |  |
| -- FGI 110_T | Checked |  |  |  |
| -- FGI 111_T | Checked |  |  |  |
| -- FGI 112_T | Checked |  |  |  |
| -- FGI 113_T | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114_T | Checked |  |  |  |
| -- FGI 115_T | Checked |  |  |  |
| -- FGI 116_T | Checked |  |  |  |
| -- FGI 117-132_T | ‘0’B (Undefined) |  |  |  |
| rf-Parameters-v1060 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1090 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| pdcp-Parameters-v1130 | Not checked |  |  |  |
| phyLayerParameters-v1130 | Not checked |  |  | Rel-8 to Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  | > Rel-10 |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| rf-Parameters-v1130 | Not checked |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1130 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1130 |  |  |  | > Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  |  |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1130 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1130 |  |  |  | > Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  |  |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| phyLayerParameters-v1170 | Not checked |  |  |  |
| ue-Category-v1170 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1180 | Not checked |  |  |  |
| mbms-Parameters-r11 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1180 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1180 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-Category-v11a0 | Not checked |  |  |  |
| measParameters-v11a0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| phyLayerParameters-v1250 | Not checked |  |  |  |
| rf-Parameters-v1250 SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA-v1250 | Not checked |  |  |  |
| supportedBandCombination-v1250 | Not checked |  |  |  |
| supportedBandCombinationAdd-v1250 | Not checked |  |  |  |
| freqBandPriorityAdjustment-r12 | Checked |  | pc_freqBandPriorityAdjustment |  |
| } |  |  |  |  |
| rlc-Parameters-r12 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1250 | Not checked |  |  |  |
| ue-CategoryDL-r12 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| ue-CategoryUL-r12 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| wlan-IW-Parameters-r12 | Not checked |  |  |  |
| measParameters-v1250 SEQUENCE { |  |  |  |  |
| timerT312-r12 | Not checked |  |  |  |
| alternativeTimeToTrigger-r12 | Not checked |  |  |  |
| incMonEUTRA-r12 | Not checked |  |  |  |
| incMonUTRA-r12 | Not checked |  |  |  |
| extendedMaxMeasId-r12 | Not checked |  |  |  |
| extendedRSRQ-LowerRange-r12 | Not checked |  |  |  |
| rsrq-OnAllSymbols-r12 | Not checked |  |  |  |
| crs-DiscoverySignalsMeas-r12 | Not checked |  |  |  |
| csi-RS-DiscoverySignalsMeas-r12 | Checked |  | pc_CSI-RS_DS_Meas |  |
| } |  |  |  |  |
| dc-Parameters-r12 | Not checked |  |  |  |
| mbms-Parameters-v1250 | Not checked |  |  |  |
| mac-Parameters-r12 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1250 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1250 | Not checked |  |  |  |
| sl-Parameters-r12 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1260 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1270 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| phyLayerParameters-v1280 |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1310 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 3 | pc_ue_CategoryDL_M1 |  |
| ue-CategoryUL-v1310 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 3 | pc_ue_CategoryUL_M1 |  |
| pdcp-Parameters-v1310 | Not checked |  |  |  |
| rlc-Parameters-v1310 | Not checked |  |  |  |
| mac-Parameters-v1310 SEQUENCE { |  |  |  |  |
| extendedMAC-LengthField-r13 | Not checked |  |  |  |
| extendedLongDRX-r13 | Checked |  | pc_extendedlongDRX |  |
| } |  |  |  |  |
| phyLayerParameters-v1310 | Not checked |  |  |  |
| rf-Parameters-v1310 | Not checked |  |  |  |
| measParameters-v1310 SEQUENCE { |  |  |  |  |
| rs-SINR-Meas-r13 | Not checked |  |  |  |
| allowedCellList-r13 | Not checked |  |  |  |
| extendedMaxObjectId-r13 | Not checked |  |  |  |
| ul-PDCP-Delay-r13 | Not checked |  |  |  |
| extendedFreqPriorities-r13 | Not checked |  |  |  |
| multiBandInfoReport-r13 | Not checked |  |  |  |
| rssi-AndChannelOccupancyReporting-r13 | Checked |  | pc_rssiAndChannelOccupancyReporting |  |
| } |  |  |  |  |
| dc-Parameters-v1310 | Not checked |  |  |  |
| sl-Parameters-v1310 | Not checked |  |  |  |
| scptm-Parameters-r13 | Not checked |  |  |  |
| ce-Parameters-r13 SEQUENCE { |  |  |  |  |
| ce-ModeA-r13 | Checked |  | pc_CEmodeA |  |
| ce-ModeB-r13 | Checked |  | pc_CEmodeB |  |
| } |  |  |  |  |
| interRAT-ParametersWLAN-r13 | Not checked |  |  |  |
| laa-Parameters-r13 SEQUENCE { |  |  |  |  |
| crossCarrierSchedulingLAA-DL-r13 | Not checked |  |  |  |
| csi-RS-DRS-RRM-MeasurementsLAA-r13 | Not checked |  |  |  |
| downlinkLAA-r13 | Checked |  | pc_downlink_LAA |  |
| endingDwPTS-r1310 | Not checked |  |  |  |
| secondSlotStartingPosition- r13 | Not checked |  |  |  |
| tm9-LAA-r13 | Not checked |  |  |  |
| tm10-LAA-r13 | Not checked |  |  |  |
| } |  |  |  |  |
| lwa-Parameters-r13 | Not checked |  |  |  |
| wlan-IW-Parameters-v1310 | Not checked |  |  |  |
| lwip-Parameters-r13 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1310 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1310 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ce-Parameters-v1320 SEQUENCE { |  |  |  |  |
| intraFreqA3-CE-ModeA-r13 | Checked |  | pc_IntraFreqA3_CE_ModeA |  |
| intraFreqA3-CE-ModeB-r13 | Not checked |  |  |  |
| intraFreqHO-CE-ModeA-r13 | Checked |  | pc_IntraFreqHO_CE_ModeA |  |
| intraFreqHO-CE-ModeB-r13 | Not checked |  |  |  |
| } |  |  |  |  |
| phyLayerParameters-v1320 | Not checked |  |  |  |
| rf-Parameters-v1320 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1320 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1320 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryDL-v1330 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| phyLayerParameters-v1330 | Not checked |  |  |  |
| ue-CE-NeedULGaps-r13 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryUL-v1340 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCritialExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryDL-v1350 | Checked | See Note 7 | pc_ue_CategoryDL_1bis |  |
| ue-CategoryUL-v1350 | Checked | See Note 7 | pc_ue_CategoryUL_1bis |  |
| ce-Parameters-v1350 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| other-Parameters-v1360 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| phyLayerParameters-v1430 | Not checked |  |  |  |
| ue-CategoryDL-v1430 | Not checked | See Note 8 |  |  |
| ue-CategoryUL-v1430 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 9 |  |  |
| ue-CategoryUL-v1430b | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  | pc_ue_CategoryUL_21 |  |
| mac-Parameters-v1430 | Not checked |  |  |  |
| measParameters-v1430 | Not checked |  |  |  |
| pdcp-Parameters-v1430 | Not checked |  |  |  |
| rlc-Parameters-v1430 | Not checked |  |  |  |
| rf-Parameters-v1430 | Not checked |  |  |  |
| laa-Parameters-v1430 | Not checked |  |  |  |
| lwa-Parameters-v1430 | Not checked |  |  |  |
| lwip-Parameters-v1430 | Not checked |  |  |  |
| otherParameters-v1430 | Not checked |  |  |  |
| mmtel-Parameters-r14 | Not checked |  |  |  |
| mobilityParameters-r14 | Not checked |  |  |  |
| ce-Parameters-v1430 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1430 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1430 | Not checked |  |  |  |
| mbms-Parameters-v1430 | Not checked |  |  |  |
| sl-Parameters-v1430 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1430 | Not checked |  |  |  |
| highSpeedEnhParameters-r14 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lwa-Parameters-v1440 | Not checked |  |  |  |
| mac-Parameters v1440 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| phyLayerParameters-v1450 | Not checked |  |  |  |
| rf-Parameters-v1450 | Not checked |  |  |  |
| ue-CategoryDL-v1450 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1460 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  | pc_ue_CategoryDL_21 |  |
| otherParameters-v1460 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| irat-ParametersNR- r15 | Not checked |  |  |  |
| featureSetsEUTRA- r15 | Not checked |  |  |  |
| pdcp-ParametersNR- r15 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities- v1510 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities- v1510 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measParameters- v1520 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measParameters- v1530 SEQUENCE { |  |  |  |  |
| qoe-MeasReport-r15 | Not checked |  |  |  |
| qoe-MTSI-MeasReport-r15 | Not checked |  |  |  |
| ca-IdleModeMeasurements-r15 | Not checked |  |  |  |
| ca-IdleModeValidityArea-r15 | Not checked |  |  |  |
| heightMeas-r15 | Checked |  | pc_heightMeas |  |
| multipleCellsMeasExtension-r15 | Checked |  | pc_Multiple_Cells_Meas_Ext |  |
| } |  |  |  |  |
| otherParameters- v1530 SEQUENCE { |  |  |  |  |
| assistInfoBitForLC-r15 | Not checked |  |  |  |
| timeReferenceProvision-r15 | Not checked |  |  |  |
| flightPathPlan-r15 | Checked |  | pc_FlightPathPlan |  |
| neighCellSI-AcquisitionParameters- v1530 | Not checked |  |  |  |
| mac-Parameters- v1530 | Not checked |  |  |  |
| phyLayerParameters- v1530 SEQUENCE { |  |  |  |  |
| stti-SPT-Capabilities-r15 | Not checked |  |  |  |
| ce-Capabilities-r15 SEQUENCE { |  |  |  |  |
| ce-CRS-IntfMitig-r15 | Not checked |  |  |  |
| ce-CQI-AlternativeTable-r15 | Not checked |  |  |  |
| ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15 | supported |  | pc_FlexibleStartPRB_PDSCH |  |
| ce-PDSCH-FlexibleStartPRB-CE-ModeB-r15 | Not checked |  |  |  |
| ce-PDSCH-64QAM-r15 | Not checked |  |  |  |
| ce-PUSCH-FlexibleStartPRB-CE-ModeA-r15 | supported |  | pc_FlexibleStartPRB_PUSCH |  |
| ce-PUSCH-FlexibleStartPRB-CE-ModeB-r15 | Not checked |  |  |  |
| ce-PUSCH-SubPRB-Allocation-r15 | Not checked |  |  |  |
| ce-UL-HARQ-ACK-Feedback-r15 | Not checked |  |  |  |
| shortCQI-ForSCellActivation-r15 | Not checked |  |  |  |
| mimo-CBSR-AdvancedCSI-r15 | Not checked |  |  |  |
| crs-IntfMitig-r15 | Not checked |  |  |  |
| ul-PowerControlEnhancements-r15 | Not checked |  |  |  |
| urllc-Capabilities-r15 | Not checked |  |  |  |
| altMCS-Table-r15 | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rf-Parameters- v1530 | Not checked |  |  |  |
| pdcp-Parameters-v1530 | Not checked |  |  |  |
| ue-CategoryDL- v1530 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| ue-BasedNetwPerfMeasParameters- v1530 | Not checked |  |  |  |
| rlc-Parameters- v1530 | Not checked |  |  |  |
| sl-Parameters- v1530 | Not checked |  |  |  |
| extendedNumberOfDRBs- v1530 | Not checked |  |  |  |
| reducedCP-Latency- v1530 | Not checked |  |  |  |
| laa-Parameters- v1530 | Not checked |  |  |  |
| ue-CategoryUL- v1530 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities- v1530 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities- v1530 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE | UE-EUTRA-Capability-v1540-IEs |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Rel-8 | Only for Rel-8 |
| >Rel-8 | For Rel-9 or later Releases |
| >Rel-10 | For Rel-11 or later Releases |
| FDD = TDD | UE is not allowed to signal different values for FDD and TDD |
| Note 1: For Rel-9 UEs: If FDD resp. TDD specific FGI values are reported then the FDD resp. TDD specific FGI PICS are checked against the reported FDD resp. TDD specific FGI values, else the FDD resp. TDD specifc FGI PICS are both checked against the reported common FGI values. For Rel-8 UEs the FDD resp. TDD specific FGI PICS are both checked against the reported common FGI values. For Rel-10 and higher release versions the same holds as stated above for Rel-9, except that the common values may be assumed if not reported. See 36.331 Annex B.1. Note 2: The present table shows the checking which is performed if a UE of the highest defined access stratum release indicator is tested. For UEs of lower release versions information related to higher release versions may be present if a UE has selective higher release features implemented. Such information is tolerated but not used for checking against any PICS items. Note 3: Only checked against pc_ue_CategoryDL_M1 and pc_ue_CategoryUL_M1. Note 4: Depending on UE implementation/configuration, the UE may not report certain RAT support based on information received from the network. Note 5: List of Conditions: pc_DL_intraBand_contCaBWclassB, pc_DL_intraBand_contCaBWclassC, pc_UL_intraBand_contCaBWclassB, pc_UL_intraBand_contCaBWclassC, pc_DL_intraBand_nonContCaBwClassComb_AA, pc_UL_intraBand_nonContCaBwClassComb_AA, pc_DL_interBand_CaBwClassComb_AA, pc_UL_SupportedInAllBandsInCAComb, pc_UL_interBand_CaBwClassComb_AA Checkpoints: pc_DL_intraBand_contCaBWclassB shall be set to true if at least one DL Intra-band contiguous CA BW Class B CA configuration is reported; pc_DL_intraBand_contCaBWclassC shall be set to true if at least one DL Intra-band contiguous CA BW ClassC CA configuration is reported; pc_UL_intraBand_contCaBWclassB shall be set to true if at least one UL Intra-band contiguous CA BW Class B CA configuration is reported; pc_UL_intraBand_contCaBWclassC shall be set to true if at least one UL Intra-band contiguous CA BW Class C CA configuration is reported; pc_DL_intraBand_nonContCaBwClassComb_AA shall be set to true if at least one DL_intraBand_nonContCaBwClassComb_AA configuration is reported; pc_UL_intraBand_nonContCaBwClassComb_AA shall be set to true if at least one UL_intraBand_nonContCaBwClassComb_AA configuration is reported; pc_DL_interBand_CaBwClassComb_AA shall be set to true if at least one DL_DL_interBand_CaBwClassComb_AA configuration is reported; pc_UL_SupportedInAllBandsInCAComb shall be set to true if UL_SupportedInAllBandsInCAComb configuration is reported for the CA band combination identified by px_EUTRA_CA_BandCombination; pc_UL_interBand_CaBwClassComb_AA shall be set to true if at least one UL_interBand_CaBwClassComb_AA configuration is reported. The UE shall include in supportedBandCombination-r10 as many as possible of the band combinations the UE supports. For each checkpoint listed above, in case none of the listed configuration is reported, the associated PICS item is assumed to be set correctly. Note 6: If the UE is Single Mode (FDD or TDD) support of ss-CCH-InterfHandl-r11 will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the support of ss-CCH-InterfHandl-r11 is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1130 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes (i.e. “supported” on both FDD and TDD). The IE may be signaled independently for either FDD or TDD or both. Note 7: Only check against pc_ue_CategoryDL_1bis and pc_ue_CategoryUL_1bis. Note 8: Not checked as no PICS are defined yet. Note 9: Checked as far as PICS are defined. Note 10: If the UE is Single Mode (FDD or TDD) support of crs-InterfHandl-r11 will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the support of crs-InterfHandl -r11 is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1130 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes (i.e. “supported” on both FDD and TDD). The IE may be signaled independently for either FDD or TDD or both. Note 11: Depending on UE implementation, the UE may not report both UTRA RATs supported. Note 12: The applicable categories for FGI 103/104 checking are (UE category = 8 OR UE category >=11 OR UE DL Category = 11 OR UE DL Category = 12 OR UE DL Category >= 14) according to TS 36.331. The UE DL Categories higher than 14 are not explicitly listed in the condition because in this version of core specification UE DL Category > 14 requires supporting of UE Category 11 or 12. Note 13: Void. Note 14: The extended bands represented by their placeholders need to be related to the bands reported in supportedBandListEUTRA-v9e0. |  |

Table 8.5.4.1.3.3-2A: UE-EUTRA-Capability-v1540-IEs (Table 8.5.4.1.3.3-2)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-EUTRA-Capability-v1540-IEs SEQUENCE { |  |  |  |
| phyLayerParameters-v1540 | Not checked |  |  |
| otherParameters-v1540 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1540 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1540 | Not checked |  |  |
| sl-Parameters-v1540 | Not checked |  |  |
| irat-ParametersNR-v1540 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1550-IEs |  |
| neighCellSI-AcquisitionParameters-v1550 | Not checked |  |  |
| phyLayerParameters-v1550 | Not checked |  |  |
| mac-Parameters-v1550 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1550 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1550 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1560-IEs |  |
| pdcp-ParametersNR-v1560 | Not checked |  |  |
| irat-ParametersNR-v1560 | Not checked |  |  |
| appliedCapabilityFilterCommon-r15 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1560 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1560 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1570-IEs |  |
| rf-Parameters-v1570 | Not checked |  |  |
| irat-ParametersNR-v1570 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v15a0-IEs |  |
| neighCellSI-AcquisitionParameters-v15a0 | Not checked |  |  |
| eutra-5GC-Parameters-r15 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v15a0 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v15a0 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1610-IEs |  |
| highSpeedEnhParameters-v1610 | Not checked |  |  |
| neighCellSI-AcquisitionParameters-v1610 | Not checked |  |  |
| mbms-Parameters-v1610 | Not checked |  |  |
| pdcp-Parameters-v1610 | Not checked |  |  |
| mac-Parameters-v1610 | Not checked |  |  |
| phyLayerParameters-v1610 | Not checked |  |  |
| measParameters-v1610 | Not checked |  |  |
| pur-Parameters-r16 | Not checked |  |  |
| eutra-5GC-Parameters-v1610 | Not checked |  |  |
| otherParameters-v1610 | Not checked |  |  |
| dl-DedicatedMessageSegmentation-r16 | Not checked |  |  |
| mmtel-Parameters-v1610 | Not checked |  |  |
| irat-ParametersNR-v1610 | Not checked |  |  |
| rf-Parameters-v1610 | Not checked |  |  |
| mobilityParameters-v1610 | Not checked |  |  |
| ue-BasedNetwPerfMeasParameters-v1610 | Not checked |  |  |
| sl-Parameters-v1610 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1610 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1610 | Not checked |  |  |
| nonCriticalExtension SEQUENCE | UE-EUTRA-Capability-v1630-IEs |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-2B: UE-EUTRA-Capability-v1630-IEs (Table 8.5.4.1.3.3-2A)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-EUTRA-Capability-v1630-IEs SEQUENCE { |  |  |  |
| rf-Parameters-v1630 | Not checked |  |  |
| sl-Parameters-v1630 | Not checked |  |  |
| earlySecurityReactivation-r16 | Not checked |  |  |
| mac-Parameters-v1630 | Not checked |  |  |
| measParameters-v1630 | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1630 | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1630 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1650-IEs |  |
| other-Parameters-v1650 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1660-IEs |  |
| irat-ParametersNR-v1660 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1690-IEs |  |
| other-Parameters-v1690 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1700-IEs |  |
| measParameters-v1700 | Not checked |  |  |
| ue-BasedNetwPerfMeasParameters-v170 | Not checked |  |  |
| phyLayerParameters-v1700 | Not checked |  |  |
| ntn-Parameters-r17 | Not checked |  |  |
| irat-ParametersNR-v1700 | Not checked |  |  |
| mbms-Parameters-v1700 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1710-IEs |  |
| irat-ParametersNR-v1710 | Not checked |  |  |
| neighCellSI-AcquisitionParameters-v1710 | Not checked |  |  |
| sl-Parameters-v1710 | Not checked |  |  |
| sidelinkRequested-r17 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-EUTRA-Capability-v1720-IEs |  |
| ntn-Parameters-v1720 | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-3: UEcapabilityEnquiry (step 19a1, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { | Numbering of entries is according to conditions met | According to inter-RAT capabilities of UE |  |
| ue-RadioAccessCapRequest[i1] | utran | This entry is present if the UE is capable of any mode (FDD/TDD) in UMTS. | pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR |
| ue-RadioAccessCapRequest[i2] | geran-cs |  | pc_GERAN and pc_CS |
| ue-RadioAccessCapRequest[i3] | geran-ps |  | pc_GERAN and pc_PS |
| ue-RadioAccessCapRequest[i4] | cdma2000-1XRTT |  | pc_1xRTT,pc_HRPD |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-4: UECapabilityInformation (step 19a2, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { |  | Stated capability shall be compatible with 3GPP TS 36.523-2 (ICS statements) and the user settings |  |
| rat-Type[i1] | utran |  | pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR |
| ueCapabilitiesRAT-Container[i1] OCTET STRING {} | ueCapabilitiesRAT-Container-UTRAN | Encoded as an INTER RAT HANDOVER INFO messagewith UMTS TDD capabilities as defined in 3GPP TS 25.331 [17]. | pc_TDD_HCR OR pc_TDD_LCR OR pc_TDD_VHCR |
|  |  |  | (pc_TDD_LCR AND pc_FDD) OR (pc_TDD_HCR AND pc_FDD) OR (pc_TDD_VHCR AND pc_FDD)) (Note 1) |
|  |  | Encoded as an INTER RAT HANDOVER INFO message with UMTS FDD capabilities as defined in 3GPP TS 25.331 [17]. | pc_FDD |
|  |  |  | (pc_TDD_LCR AND pc_FDD) OR (pc_TDD_HCR AND pc_FDD) OR (pc_TDD_VHCR AND pc_FDD)) (Note 1) |
| rat-Type[i2] | geran-cs |  | pc_GERAN and pc_CS |
| ueCapabilitiesRAT-Container [i2] OCTET STRING {} | ueCapabilitiesRAT-Container-GERAN-CS | Encoded as the concatenation of IEs MS classmark 2 and MS classmark 3 as defined in 3GPP TS 24.008 [32]. | pc_GERAN and pc_CS |
| rat-Type[i3] | geran-ps |  | pc_GERAN and pc_PS |
| ueCapabilitiesRAT-Container [ i3] OCTET STRING {} | ueCapabilitiesRAT-Container-GERAN-PS | Encoded as MS radio access capability IE as defined in 3GPP TS 24.008 [32]. | pc_GERAN and pc_PS |
| rat-Type[i4] | cdma2000-1XRTT |  | pc_1xRTT, pc_HRPD |
| ueCapabilitiesRAT-Container [ i4] OCTET STRING {} | Not checked | Encoded as A21 Mobile Subscription Information as defined in 3GPP2 A.S0008-C v4.0 [33]. | pc_1xRTT, pc_HRPD |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-5: ueCapabilitiesRAT-Container-UTRAN

| Derivation path: 25.331 clause 11.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark | Comment | Condition |
| ueCapabilitiesRAT-Container-UTRAN ::= SEQUENCE { |  |  |  |  |
| predefinedConfigStatusList CHOICE { |  |  |  |  |
| absent |  | NULL |  |  |
| } |  |  |  |  |
| uE-SecurityInformation |  | Not checked | The value of start CS is not used for LTE to UMTS handover in Rel-8 |  |
| ue-CapabilityContainer CHOICE { |  |  |  |  |
| present |  | Not checked | Container including UE radio access capability |  |
| } |  |  |  |  |
| v390NonCriticalExtensions CHOICE { |  |  |  |  |
| present SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v390ext |  | Not checked | Positioning capability and dummy field |  |
| v3a0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3a0ext |  | Not checked | Positioning capability |  |
| laterNonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3d0ext |  | Not checked | Deprecated information |  |
| interRATHandoverInfo-r3-add-ext |  | Not checked if present | UE radio access capability for bands VIII to XIV, UE radio access capability extension, support of 2 DRX schemes in CELL_PCH, support of E-DPDCH power interpolation |  |
| v3g0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3g0ext |  | Not checked | Positioning capability extension |  |
| v4b0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v4b0ext |  | Checked | Access Stratum Release indicator |  |
| v4d0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v4d0ext |  | Not checked if present | LCR TDD UE capability |  |
| v590NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v590ext |  | Not checked if present | Predefined configuration status information compressed, UE radio access capability compressed |  |
| v690NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v690ext SEQUENCE { |  |  |  |  |
| ue-SecurityInformation2 |  | Present but value not checked | START PS | pc_Featr Grp_8 |
| ue-SecurityInformation2 |  | Not present |  | NOT pc_FeatrGrp_8 |
| ue-RadioAccessCapabilityComp |  | Not checked | RF capability for bands VIII to XIV |  |
| ue-RadioAccessCapabilityComp2 |  | Present but value not checked | UE radio access capability comp 2 | pc_FDD AND UE reportsF DD capabilities (Note1) |
| ue-RadioAccessCapabilityComp2 |  | Not checked if present | UE radio access capability comp 2 | NOT (pc_FDD AND UE reportsF DD capabilities) (Note 1) |
| } |  |  |  |  |
| v6b0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v6b0ext |  | Not checked if present | Support for SIB11bis |  |
| v6e0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v6e0ext |  | Not checked if present | Support of FDPCH |  |
| v770NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v770ext |  | Not checked if present | TDD RF and physical channel capability extensions in Rel-7, support of GANSS, support of MAC-ehs, LCR TDD UE specific capability information |  |
| v790nonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v790ext |  | Not checked if present | Support of E-DPCCH power boosting |  |
| v860NonCriticalExtensions0 SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v860ext |  | Not checked if present | UE radio access capability for additional bands, Rel-8 HS-DSCH physical layer category, support of MAC-iis |  |
| v880NonCriticalExtensions SEQUENCE { |  | Present but value not checked |  | pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE (Note 1) |
| v880NonCriticalExtensions SEQUENCE { |  | Not checked if present |  | NOT (pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE ) (Note 1) |
| v920NonCritical ExtensionsinterRATHandoverInfo-v880ext |  | Not checked | Support for priority reselection in UTRAN, Rel-8 radio access capability extensions for LCR TDD (e.g. multi-carrier operation) |  |
| SEQUENCE { |  | Not checked if present |  |  |
| interRATHandoverInfo-v920ext |  | Not checked if present |  |  |
| v8b0NonCritical Extensions SEQUENCE { |  | Present but value not checked |  | pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE (Note 1) |
| v8b0NonCritical Extensions SEQUENCE { |  | Not checked if present |  | NOT (pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE ) (Note 1) |
| interRATHandoverInfo-v8b0ext |  | Not checked if present |  |  |
| v950NonCritical Extensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v950ext |  | Not checked if present |  |  |
| va40NonCritical Extensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-va40ext |  | Not checked if present |  |  |
| va80NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-va80ext | Not checked if present |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vb50ext | Not checked if present |  |  |  |
| vb70NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vb70ext | Not checked if present |  |  |  |
| vbc0NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vbc0ext | Not checked if present |  |  |  |
| vc50NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vc50ext | Not checked if present |  |  |  |
| nonCritical Extensions SEQUENCE {} |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| Note 1: When the UE supports more than one mode (e.g. pc_FDD and pc_TDD_HCR) then the UE is expected to report in the UTRA Container the capabilities associated with one of the supported modes. |  |  |  |  |

Table 8.5.4.1.3.3-6: ueCapabilitiesRAT-Container-GERAN-CS

| Derivation path: 25.331 clause 11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Mobile Station Classmark 2 | First byte is 33H Second byte is 3. Third, Fourth and Fifth bytes are ignored. |  |  |
| Mobile Station Classmark 3 | CSN.1 decoding shall be successful and the contents shall indicate that E-UTRA FDD or EUTRA TDD or both is supported. Other values are not checked. |  |  |

Table 8.5.4.1.3.3-7: ueCapabilitiesRAT-Container-GERAN-PS

| Derivation path: 25.331 clause 11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access Capability | CSN.1 decoding shall be succesful and the contents shall indicate that E-UTRA FDD or EUTRA TDD or both is supported. Other values are not checked. |  |  |

#### 8.5.4.2	Network-requested CA Band Combination Capability Signalling / Number of UE supported CA band combinations less than or equal to 128

8.5.4.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE supporting 128 or less CA band combinations receives a UECapabilityEnquiry message with IE requestedFrequencyBands indication prioritizing frequency band X}

then { UE transmits a UECapabilityInformation message with IE supportedBandCombination with the first 2DL+1UL CA band combination containing band X and including IE requestedBands with the same content as in the IE requestedFrequencyBands received in the UECapabilityEnquiry message }

}

8.5.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.6.3.3, 5.6.22.2, 5.6.22.3.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes eutra:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to eutra;

….

3>	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

4>	create a set of band combinations supported by the UE, including non-CA combinations, target for being included in supportedBandCombination while observing the following order (i.e. listed in order of decreasing priority):

-	include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	the UE supports TM10 with one or more CSI processes;

-	include all 2DL+1UL CA band combinations, only consisting of bands included in requestedFrequencyBands;

-	include all other 2DL+1UL CA band combinations;

-	include all other CA band combinations, only consisting of bands included in requestedFrequencyBands, and prioritized in the order of requestedFrequencyBands, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

4>	include in supportedBandCombination as many of the target band combinations as possible, determined according to the above, while observing the priority order;

4>	include in supportedBandCombinationAdd as many of the remaining target band combinations as possible, i.e. the target band combinations the UE was not able to include in supportedBandCombination, and limited to those consisting of bands included in requestedFrequencyBands, while observing the priority order;

4> indicate in requestedBands the same bands and in the same order as included in the received requestedFrequencyBands;

3>	else

4>	create a set of band combinations supported by the UE, including non-CA combinations, target for being included in supportedBandCombination:

-	include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	the UE supports TM10 with one or more CSI processes;

-	include all 2DL+1UL CA band combinations;

-	include all other CA band combinations;

4>	include in supportedBandCombination as many of the target band combinations as possible, determined according to the above;

4>	if the number of non-CA and CA band combinations supported by UE exceeds the maximum number of band combinations of supportedBandCombination, the selection of subset of band combinations is up to UE implementation;

NOTE:	If the UECapabilityEnquiry message does not include requestedFrequencyBands, UE does not include supportedBandCombinationAdd.

….

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.5.4.2.3	Test description

8.5.4.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.4.2.3.2	Test procedure sequence

Table 8.5.4.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. The IE requestedFrequencyBands is not included | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 will be performed else step 2b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | UE transmits segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 2b1 | UE transmits a UECapabilityInformation message. | --> | UECapabilityInformation | - | - |
| - | The reference to supportedBandCombinations and supportedBandListEUTRA below refers to the IEs received by the SS in the UECapabilityInformation message in step 2. The SS locates the first 2DL+1UL CA band combination in the supportedBandCombination-r10 sequence. If the first CA band combination is an intra-band combination the SS stores the value of the IE bandEUTRA-r10 in the test variables B1 and B2. If the first CA band combination is an inter-band combination the SS stores the value of the IE bandEUTRA-r10 for each of the bands in the test variables B1 and B2 respectively. The SS searches in the supportedBandCombination-r10 sequence for a 2DL+1UL CA band combination where the band or bands are different from the bands stored in the test variables B1 and B2. The SS stores the value of the IE bandEUTRA-r10 of the band (intra-band case) or of one of the bands (inter-band case) in the test variable B3. | - | - | - | - |
| 3 | The SS transmits a UECapabilityEnquiry message including the IE requestedFrequencyBands-r11 with the first requested frequency band entry set to the value of the test variable B3, the second entry to the value of the test variable B1 and the successive entries 3 to n set to frequency bands different from B1 and B3 derived from the UE indicated supported bands in IE supportedBandListEUTRA received by the SS in the UECapabilityInformation message in step 2 and where n is MIN(16, number of UE supported bands) and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 4a1 and 4a2 will be performed else step 4b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 4a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 4a1 | UE transmits segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 4a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 4 | Check: Does the UE transmit a UECapabilityInformation message with IE supportedBandCombination-r10 where the first listed 2DL+1UL CA band combination contains a band equal to the band stored in test variable B3; and where IE requestedBands have the same content as the IE requestedFrequencyBands-r11 sent by the SS in step 3? (Note 1) | --> | UECapabilityInformation | 1 | P |
| Note 1: Acc. to TS 36.331 (ASN.1), each CA band combination listed in IE supportedBandCombination-r10 is denominated with the contained bandEUTRA-r10 element (in case of Intra-band) resp. bandEUTRA-r10 elements (in case of inter-band). Extended bands (i.e. band > 64) were defined from TS 36.331 v1090 (ASN.1) onwards and the respective IE bandEUTRA-r10 is always encoded as '64'. Which means that it is not possible to distinguish among multiple reported extended band candidates in IE supportedBandCombination-r10. As a consequence, the Check procedure in step 4 cannot distinguish between extended bands in case that the UE supports mulitple CA band combinations containing different extended bands or a CA band combination involving more than one extended band. In such case, the test procedure shall issue an INCONC (= inconclusive) verdict with the following error message: Ambiguity due to more than one extended band reported in the supported CA band combinations. |  |  |  |  |  |

8.5.4.2.3.3	Specific message contents

Table 8.5.4.2.3.3-1: UECapabilityEnquiry (step 1, Table 8.5.4.2.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type[1] | Eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.2.3.3-2: UECapabilityInformation (steps 2a2 and 2b1, Table 8.5.4.2.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-Capabilities)) of SEQUENCE { | 1 entry |  |  |
| rat-Type[1] | Eutra | E-UTRA only |  |
| ueCapabilityRAT-Container[1] OCTET STRING { |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1020 { |  |  |  |
| supportedBandCombination-r10 { |  | See Table 8.5.4.2.3.2-1 for SS actions to set test variables B1, B2 and B3 based on the content in the IE supportedBandCombination-r10. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1090 { |  |  |  |
| supportedBandCombination-v1090 { |  | See Table 8.5.4.2.3.2-1 for SS actions to set test variables B1, B2 and B3 based on the content in the IE supportedBandCombination-r10 and supportedBandCombination-v1090. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1180 { |  |  |  |
| freqBandRetrieval-r11 | Supported |  |  |
| requestedBands-r11 | Not present |  |  |
| supportedBandCombinationAdd-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.2.3.3-3: UECapabilityEnquiry (step 3, Table 8.5.4.2.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type[1] | Eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 | n entries, where n is equal to UE supported bands or 16 if number of UE supported bands exceeds 16. |  |  |
| FreqBandIndicator-r11 [1] | B3 | See Table 8.5.4.2.3.2-1 for definition of B3. |  |
| FreqBandIndicator-r11 [2] | B1 | See Table 8.5.4.2.3.2-1 for definition of B1. |  |
| FreqBandIndicator-r11 [k] | For k=3 to n set to value different from values set in preceding entries FreqBandIndicator-r11 [1] to FreqBandIndicator-r11 [k-1], |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.2.3.3-4: UECapabilityInformation (steps 4a2 and 4b1, Table 8.5.4.2.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-Capabilities)) of SEQUENCE { | 1 entry |  |  |
| rat-Type[1] | Eutra | E-UTRA only |  |
| ueCapabilitiesRAT-Container[1] OCTET STRING { |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1020 { |  |  |  |
| supportedBandCombination-r10 { | See step 4 in Table 8.5.4.2.3.2-1 for SS actions for how to check content |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1090 { |  |  |  |
| supportedBandCombination-v1090 { | See step 4 in Table 8.5.4.2.3.2-1 for SS actions for how to check content |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| .. rf-Parameters-v1180 { |  |  |  |
| .. freqBandRetrieval-r11 | Supported |  |  |
| .. requestedBands-r11 | Same number of entries and content as SS sent in IE requestedFrequencyBands in the UEcapabilityEnquiry message in step 3 |  |  |
| supportedBandCombinationAdd-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.4.3	Network-requested CA Band Combination Capability Signalling / Number of UE supported CA band combinations exceeds 128

8.5.4.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE supporting more than 128 CA band combinations receives a UECapabilityEnquiry message with IE requestedFrequencyBands indication prioritizing frequency band X}

then { UE transmits a UECapabilityInformation message with IE supportedBandCombination with the first 2DL+1UL CA band combination containing band X and including IEs supportedBandCombinationAdd and requestedBands where IE requestedBands contains the same content as in the IE requestedFrequencyBands received in the UECapabilityEnquiry message }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE supporting more than 128 CA band combinations receives a UECapabilityEnquiry message not including IE requestedFrequencyBands }

then { UE transmits a UECapabilityInformation message including IE supportedBandCombination, but not including IE supportedBandCombinationAdd }

}

8.5.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.6.3.3, 5.6.22.2, 5.6.22.3.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes eutra:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to eutra;

….

3>	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

4>	create a set of band combinations supported by the UE, including non-CA combinations, target for being included in supportedBandCombination while observing the following order (i.e. listed in order of decreasing priority):

-	include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	the UE supports TM10 with one or more CSI processes;

-	include all 2DL+1UL CA band combinations, only consisting of bands included in requestedFrequencyBands;

-	include all other 2DL+1UL CA band combinations;

-	include all other CA band combinations, only consisting of bands included in requestedFrequencyBands, and prioritized in the order of requestedFrequencyBands, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

4>	include in supportedBandCombination as many of the target band combinations as possible, determined according to the above, while observing the priority order;

4>	include in supportedBandCombinationAdd as many of the remaining target band combinations as possible, i.e. the target band combinations the UE was not able to include in supportedBandCombination, and limited to those consisting of bands included in requestedFrequencyBands, while observing the priority order;

4> indicate in requestedBands the same bands and in the same order as included in the received requestedFrequencyBands;

3>	else

4>	create a set of band combinations supported by the UE, including non-CA combinations, target for being included in supportedBandCombination:

-	include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	the UE supports TM10 with one or more CSI processes;

-	include all 2DL+1UL CA band combinations;

-	include all other CA band combinations;

4>	include in supportedBandCombination as many of the target band combinations as possible, determined according to the above;

4>	if the number of non-CA and CA band combinations supported by UE exceeds the maximum number of band combinations of supportedBandCombination, the selection of subset of band combinations is up to UE implementation;

NOTE:	If the UECapabilityEnquiry message does not include requestedFrequencyBands, UE does not include supportedBandCombinationAdd.

….

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.5.4.3.3	Test description

8.5.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.4.3.3.2	Test procedure sequence

Table 8.5.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. The IE requestedFrequencyBands is not included | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 and 2a2 will be performed else step 2b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | UE transmits segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 2a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 2 | P |
| 2b1 | Check: Does the UE transmit a UECapabilityInformation message including IE supportedBandCombination, but not including IE supportedBandCombinationAdd? | --> | UECapabilityInformation | 2 | P |
| - | The reference to supportedBandCombinations and supportedBandListEUTRA below refers to the IEs received by the SS in the UECapabilityInformation message in step 2. The SS locates the first 2DL+1UL CA band combination in supportedBandCombination-r10 sequence. If the first CA band combination is an intra-band combination the SS stores the value of the IE bandEUTRA-r10 in the test variables B1 and B2. If the first CA band combination is an inter-band combination the SS stores the value of the IE bandEUTRA-r10 for each of the bands in the test variables B1 and B2 respectively. The SS searches in the supportedBandCombination-r10 sequence for a 2DL+1UL CA band combination where the band or bands are different from the bands stored in the test variables B1 and B2. The SS stores the value of the IE bandEUTRA-r10 of the band (intra-band case) or of one of the bands (inter-band case) in the test variable B3. | - | - | - | - |
| 3 | The SS transmits a UECapabilityEnquiry message including the IE requestedFrequencyBands with the first requested frequency band entry set to the value of the test variable B3, the second entry to the value of the test variable B1 and the successive entries 3 to n set to frequency bands different from B1 and B3 derived from the UE indicated supported bands in IE supportedBandListEUTRA received by the SS in the UECapabilityInformation message in step 2 and where n is MIN(16, number of UE supported bands) and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 4a1 and 4a2 will be performed else step 4b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 4a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 4a1 | UE transmits segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 4a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 4b1 | Check: Does the UE transmit a UECapabilityInformation message with IE supportedBandCombination-r11 where the first listed 2DL+1UL CA band combination contains a band equal to the band stored in test variable B3; and is including IEs supportedBandCombinationAdd and requestedBand; and where IE requestedBands have the same content as the IE requestedFrequencyBands-r11 sent by the SS in step 3? (Note 1) | --> | UECapabilityInformation | 1 | P |
| Note 1: Acc. to TS 36.331 (ASN.1), each CA band combination listed in IE supportedBandCombination-r10 is denominated with the contained bandEUTRA-r10 element (in case of Intra-band) resp. bandEUTRA-r10 elements (in case of inter-band). Extended bands (i.e. band > 64) were defined from TS 36.331 v1090 (ASN.1) onwards and the respective IE bandEUTRA-r10 is always encoded as '64'. Which means that it is not possible to distinguish among multiple reported extended band candidates in IE supportedBandCombination-r10. As a consequence, the Check procedure in step 4 cannot distinguish between extended bands in case that the UE supports multiple CA band combinations containing different extended bands or a CA band combination involving more than one extended band. In such case, the test procedure shall issue an INCONC (= inconclusive) verdict with the following error message: Ambiguity due to more than one extended band reported in the supported CA band combinations. |  |  |  |  |  |

8.5.4.3.3.3	Specific message contents

Table 8.5.4.3.3.3-1: UECapabilityEnquiry (step 1, Table 8.5.4.3.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type[1] | Eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.3.3.3-2: UECapabilityInformation (steps 2a2 and 2b1, Table 8.5.4.3.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-Capabilities)) of SEQUENCE { | 1 entry |  |  |
| rat-Type[1] | Eutra | E-UTRA only |  |
| ueCapabilityRAT-Container[1] OCTET STRING { |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1020 { |  |  |  |
| supportedBandCombination-r10 { |  | See Table 8.5.4.3.3.2-1 for SS actions to set test variables B1, B2 and B3 based on the content in the IE supportedBandCombination-r10. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1090 { |  |  |  |
| supportedBandCombination-v1090 { |  | See Table 8.5.4.3.3.2-1 for SS actions to set test variables B1, B2 and B3 based on the content in the IE supportedBandCombination-r10 and supportedBandCombination-v1090. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| .. rf-Parameters-v1180 { |  |  |  |
| .. freqBandRetrieval-r11 | Supported |  |  |
| .. requestedBands-r11 | Not present |  |  |
| .. supportedBandCombinationAdd-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.3.3.3-3: UECapabilityEnquiry (step 3, Table 8.5.4.3.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type[1] | Eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 | n entries, where n is equal to UE supported bands or 16 if number of UE supported bands exceeds 16. |  |  |
| FreqBandIndicator-r11 [1] | B3 | See Table 8.5.4.3.3.2-1 for definition of B3. |  |
| FreqBandIndicator-r11 [2] | B1 | See Table 8.5.4.3.3.2-1 for definition of B1. |  |
| FreqBandIndicator-r11 [k] | For k=3 to n set to value different from values set in preceding entries FreqBandIndicator-r11 [1] to FreqBandIndicator-r11 [k-1], |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.3.3.3-4: UECapabilityInformation (steps 4a2 and 4b1, Table 8.5.4.3.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-Capabilities)) of SEQUENCE { | 1 entry |  |  |
| rat-Type[1] | Eutra | E-UTRA only |  |
| ueCapabilityRAT-Container[1] OCTET STRING { |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1020 { |  |  |  |
| supportedBandCombination-r10 { | See step 4 in Table 8.5.4.3.3.2-1 for SS actions for how to check content |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rf-Parameters-v1090 { |  |  |  |
| supportedBandCombination-v1090 { | See step 4 in Table 8.5.4.3.3.2-1 for SS actions for how to check content |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| .. rf-Parameters-v1180 { |  |  |  |
| .. freqBandRetrieval-r11 | Supported |  |  |
| .. requestedBands-r11 | Same number of entries and content as SS sent in IE requestedFrequencyBands in the UECapabilityEnquiry message in step 3 |  |  |
| .. supportedBandCombinationAdd-r11 | Any value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.4.4	UE Capability Transfer/ Success/ UE Cat 0/ UE Paging Info

8.5.4.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE supporting Cat 0 receives an UECapabilityEnquiry message }

then { UE transmits UECapabilityInformation message with IE UE-RadioPagingInfo including ue-Category }

}

8.5.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.3.3

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes eutra:

…

3>	if the UE is a category 0 UE according to TS 36.306 [5]:

4>	include ue-RadioPagingInfo including ue-Category;

…

8.5.4.4.3	Test description

8.5.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.5.4.4.3.2	Test procedure sequence

Table 8.5.4.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message | <-- | UECapabilityEnquiry | - | - |
| 2 | Check: Does the UE transmit a UECapabilityInformation message with IE UE-RadioPagingInfo including ue-Category ? | --> | UECapabilityInformation | 1 | P |

8.5.4.4.3.3	Specific message contents

Table 8.5.4.4.3.3-1: UEcapabilityEnquiry (step 1 Table 8.5.4.4.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry | E-UTRA only |  |
| RAT-Type | eutra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.4.3.3-2: UECapabilityInformation (step 2, Table 8.5.4.4.3.2-1)

| Derivation Path: 36.508, clause 4.6.1, Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-RadioPagingInfo-r12 SEQUENCE { |  |  |  |
| ue-Category-v1250 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.5.5	Message Segment transfer

#### 8.5.5.1	RACS / UL Message Segment transfer / UECapabilityInformation / Success

8.5.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a UECapabilityEnquiry message with rrc-SegAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends ULDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.5.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.6.3, 5.6.22.2, 5.6.22.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.6.3]

The UE shall:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 36.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	release uplinkDataCompression, if configured;

2>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

2>	reset MAC;

8.5.5.1.3	Test description

8.5.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.5.5.1.3.2	Test procedure sequence

Table 8.5.5.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 |
| --- | --- | --- | --- |
| T1 | Cell-specific RS RPRE | dBm/15kHz | “Off” |
| T2 | Cell-specific RS RPRE | dBm/15kHz | -85 |

Table 8.5.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Steps 2a1 to 2b7 describe behaviour that depends on the size of UEs capability container; the "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else steps 2b1 to 2b7 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b1 | The UE sends UECapabilityInformation Message. | --> | UECapabilityInformation | - | - |
| 3a1-4a11 | Void | - | - | - | - |
| 2b2 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b3 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 2b4 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 2b5 is repeated a maximum of 2 times. | - | - | - | - |
| 2b5 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b6 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b7 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| 5 | The SS allocates 17 UL grants periodically from 15 (FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. (Note 1) | - | - | - | - |
| 6 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 7 | The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 8 | Wait for T310 to expire | - | - | - | - |
| 9 | The SS changes Cell 1 parameters according to the row "T2" in table 8.5.5.1.3.2-1. | - | - | - | - |
| 10 | The UE sends an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1? | --> | RRCConnectionReestablishmentComplete | - | - |
| 13 | SS resumes regular UL grant transmission. | - | - | - | - |
| 14 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |
| NOTE 1: 17 UL grants of default size (501 bytes, as per 36.523-3[20] Table 7.2.0-1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 8188 bytes. |  |  |  |  |  |

8.5.5.1.3.3	Specific message contents

Table 8.5.5.1.3.3-1: UECapabilityEnquiry (steps 1, 2b4, 4, Table 8.5.5.1.3.2-2)

| Derivation Path: TS 36.508 [7], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT TYPE { |  |  |  |
| RAT-Type [1] | eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 SEQUENCE { | n entries, where n is equal to UE supported bands or 16 if number of UE supported bands exceeds 16. |  |  |
| FreqBandIndicator-r11 [1] | px_ePrimaryFrequencyBand | E-UTRA primary frequency band under test. |  |
| FreqBandIndicator-r11 [k] | For k=2 to n set to value different from values set in preceding entries FreqBandIndicator-r11 [1] to FreqBandIndicatorr11 [k-1] | Frequency bands supported by UE. Any value β such that pc_eBandβ_Supp is TRUE |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-2: ULDedicatedMessageSegment (steps 2a1, 2b5 and step 6, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508 [18], Table 4.6.1-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including a segment of the encoded DL DCCH message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-3: RRCConnectionReestablishmentRequest (step 11, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-4: UECapabilityInformation (step 2b1, Table 8.5.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry |  |  |
| rat-Type | eutra | E-UTRA only |  |
| ueCapabilityRAT-Container | UE-EUTRA-Capability | Encoded as per TS 36.331 [17] clause 5.6.3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.5.5.2	DL Message Segment transfer / RRC connection reconfiguration / RLF / Success

8.5.5.2.1	Test Purpose (TP)

(1)

with { UE has previously received and stored a segment of a segmented RRCConnectionReconfiguration message }

ensure that {

when { UE detects radio link failure }

then { UE discards all the segments of the RRCConnectionReconfiguration message }

}

(2)

with { UE has previously received and stored segment(s) of a segmented RRCConnectionReconfiguration message }

ensure that {

when { UE receives all remaining segment(s) of the RRCConnectionReconfiguration message }

then { UE successfully performs the RRC connection reconfiguration procedure }

}

8.5.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.11.3 and 5.6.25.3.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon T318 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

[TS 36.331, clause 5.6.25.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5 for the RRCConnectionReconfiguration message or 5.3.3.4a for the RRCConnectionResume message;

2>	discard all segments.

8.5.5.2.3	Test description

8.5.5.2.3.1	Pre-test conditions

System Simulator:

- Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) on Cell 1 according to [18] using the UE TEST LOOP MODE B.

8.5.5.2.3.2	Test procedure sequence

Table 8.5.5.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after Preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.5.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level value shall be such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 + Hys < M1). |

Table 8.5.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message | - | - |
| 1 | The SS transmits a DLDedicatedMessageSegment containing the first segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 2 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.5.5.2.3.2-1 | - | - | - | - |
| 3 | T310 seconds after Step 2 the SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.5.5.2.3.2-1 | - | - | - | - |
| - | The following messages are sent on Cell 1 unless otherwise stated. | - | - | - | - |
| 4 | The UE sends RRCConnectionReestablishmentRequest message | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 6 | The UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 9 | The SS transmits a DLDedicatedMessageSegment containing the second and last segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 in the next 1s? | --> | RRCConnectionReconfigurationComplete | 1 | F |
| 11 | The SS transmits a DLDedicatedMessageSegment containing the first segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 12 | The SS transmits a DLDedicatedMessageSegment containing the second (last) segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 14 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 2. | <-- | IP packet | - | - |
| 15 | Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context on Cell 2? | --> | IP packet | 2 | P |

8.5.5.2.3.3	Specific message contents

Table 8.5.5.2.3.3-1: ACTIVATE TEST MODE (preamble)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE B

Table 8.5.5.2.3.3-2: DLDedicatedMessageSegment (Steps 1, 9, 11 and 12, Table 8.5.5.2.3.2-2)

| Derivation Path: 36.331, Clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 |  | Step 1, Step 11 |
| segmentNumber-r16 | 1 |  | Step 12 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING First segment of the RRCConnectionReconfiguration message) | Table 8.5.5.2.3.3-3 | Step 1, Step 11 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Second segment of the RRCConnectionReconfiguration message) | Table 8.5.5.2.3.3-3 | Step 9, 12 |
| lateNonCriticalExtension | Not present |  |  |
| rrc-MessageSegmentType-r16 | notLastSegment |  | Step 1, Step 11 |
| rrc-MessageSegmentType-r16 | lastSegment |  | Step 9, 12 |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |

Table 8.5.5.2.3.3-3: RRCConnectionReconfiguration (Steps 1, 9, 11 and 12 Table 8.5.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING RRCConnectionReconfiguration-v8m0-IEs) |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.2.3.3-4: MobilityControlInfo (steps 1, 9, 11 and 12, Table 8.5.5.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.5.6	Non-Terrestrial Networks

#### 8.5.6.1	eMTC / NTN / Ephemeris information update / T317 Expiry / T318 Expiry

8.5.6.1.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType31 and starts timer T317 with the duration ul-SyncValidityDuration }

ensure that {

when { T317 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronisation is lost and start T318 }

}

(2)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { SystemInformationBlockType31 is acquired successfully }

then { UE stops T318 and informs lower layers that the UL synchronization is restored }

}

(3)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { T318 expires }

then { UE considers radio link failure and initiates the connection re-establishment procedure }

}

8.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.1.3, 5.2.2.4, 5.2.2.39, 5.3.18 and 5.3.11.3 Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.1.3]

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

[TS 36.331, clause 5.2.2.39]

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

1>	start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

[TS 36.331, clause 5.3.18]

The UE shall:

1>	if in RRC_CONNECTED:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

3>	stop timer T318;

3>	inform lower layers that the UL synchronisation is restored;

NOTE:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

[TS 36.331, clause 5.3.11.3]

The UE shall:

…

- upon T318 expiry; or

…

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

8.5.6.1.3	Test description

8.5.6.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508 [18] clause 4.4.1.1 is configured.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in in state Switched OFF (State 1) according to TS 36.508 [18] clause 4.5.1.

8.5.6.1.3.2	Test procedure sequence

Table 8.5.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType31 and start T317 = 60 seconds. | - | - | - | - |
| 3-20 | Steps 1 to 18 of the registration and test mode activated procedure described in TS 36.508 [18] subclause 4.5.2A.3 with TEST LOOP MODE A are performed on Cell 1. Note: After receiving the message in step3, SS start a timer T1 with value = T317. | - | - | - | - |
| 20A-20B | Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 4.5.4.3 are performed on Cell 1. | - | - | - | - |
| 21 | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 22 | Wait until expiry of T1. Note: UE starts timer T318. SS start a timer T2 with value = T318. | - | - | - | - |
| 23-24 | Void | - | - | - | - |
| 25 | The SS transmits a MAC PDU containing an RLC SDU. | <-- | MAC PDU (RLC SN=0) | - | - |
| 26 | UL grants assigned at the next available USS before expiry of T2, allowing the UE to return the RLC SDU as received in step 25. | <-- | (UL Grant (known C-RNTI)) | - | - |
| 26A | SS resumes broadcasting SystemInformationBlockType31. Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. | - | - | - | - |
| 26B | Check: Does the UE transmit a MAC PDU corresponding to grant in step 26? | --> | MAC PDU | 2 | P |
| 26C | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 26D | Wait until expiry of T3. Note: UE starts timer T318. SS starts a timer T4 with value = T318 | - | - | - | - |
| 26E | The SS transmits a MAC PDU containing an RLC SDU. | <-- | MAC PDU (RLC SN=1) | - | - |
| 27 | Check: Does the UE transmit a MAC PDU corresponding to grant in step 26? | --> | MAC PDU | 1 | F |
| 28 | Wait until expiry of T4. | - | - | - | - |
| 28A | SS resumes broadcasting SystemInformationBlockType31. | - | - | - | - |
| 29 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on SRB0? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 30 | The SS transmits a RRCConnectionReestablishment message on SRB0 | <-- | RRCConnectionReestablishment | - | - |
| 31 | The UE transmits RRCConnectionReestablishmentComplete message on SRB1. | --> | RRCConnectionReestablishmentComplete | - | - |

8.5.6.1.3.3	Specific message contents

Table 8.5.6.1.3.3-1: Void

Table 8.5.6.1.3.3-2: SystemInformationBlockType31 (steps 2, 27A and 28A, table 8.5.6.1.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ul-SyncValidityDuration-r17 | s60 | Value of T317 |  |
| } |  |  |  |
| } |  |  |  |

## 8.6	Minimization of Drive Test Specific Procedures

### 8.6.1	Immediate MDT

#### 8.6.1.1	Immediate MDT / Reporting / Location information

8.6.1.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement with event A2 configured with includeLocationInfo included in the reportConfig }

ensure that {

when { Entry condition for event A2 is met and detailed location information that has not been reported is available }

then { UE sends MeasurementReport message with locationInfo included }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A2 }

ensure that {

when { Exit condition for event A2 is met }

then { UE stops sending MeasurementReport message }

}

8.6.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.3 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

...

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else:

3>	if the corresponding measObject concerns EUTRA:

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	if the triggerType is set to ‘event’ and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to ‘event’ and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>		if the triggerType is set to ‘event’ and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during  timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

…

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

…

[TS 36.331, clause 5.5.4.3]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

InequalityA2-1 (Entering condition)

InequalityA2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within the reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within the reportConfigEUTRA for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

...

1>	if the includeLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of the locationInfo as follows:

2>	include the locationCoordinates;

2>	if available, include the gnss-TOD-msec;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId :

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

...

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

8.6.1.1.3	Test description

8.6.1.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position.  This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) according to [18].

8.6.1.1.3.2	Test procedure sequence

Table 8.6.1.1.3.2-1 illustrates the downlink power levels to be applied for Cell 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.1.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 |  |  | -70 | Power level is such that Ms > Thresh + Hys |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 | Power level is such that entry condition for event A2 is satisfied Ms + Hys < Thresh |
| T2 |  |  | -70 | Power level is such that exit condition for event A2 is satisfied Ms > Thresh + Hys |
| Note: The total tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and absolute UE measurement accuracy (TS 36.133 clause 9). |  |  |  |  |

Table 8.6.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A2 with includeLocationInfo configured. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.6.1.1.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE locationInfo included? | --> | MeasurementReport | 1 | P |
| 5 | The SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.6.1.1.3.2-1. | - | - | - | - |
| 6 | Wait and ignore MeasurementReport messages for 5 s to allow change of power levels for Cell 1. | - | - | - | - |
| 7 | Check: Does the UE attempt to transmit an uplink message within the next 10s? | - |  | 2 | F |

8.6.1.1.3.3	Specific message contents

Table 8.6.1.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.1.1.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.6.1.1.3.3-2: MeasConfig (step 1, Table 8.6.1.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| reportConfig[1] | ReportConfig-A2-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.1.1.3.3-3: ReportConfig-A2-H (step 1, Table 8.6.1.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-5 ReportConfigEUTRA-A2(-83) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| hysteresis | 6 | 3 dB |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| includeLocationInfo-r10 | true |  |  |
| } |  |  |  |

Table 8.6.1.1.3.3-4: MeasurementReport (step 4, Table 8.6.1.1.3.2-2)

| Derivation path: 36.508 table clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 SEQUENCE { |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |
| ellipsoid-Point-r10 | Any allowed value |  |  |
| ellipsoidPointWithAltitude-r10 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.1.2	Immediate MDT / Reporting / Location information / Request from eNB / Event A2

8.6.1.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A2 with the includeLocationInfo set to true and the obtainLocation set to setup }

ensure that {

when { Serving cell becomes worse than absolute threshold minus hysteresis }

then { UE sends MeasurementReport message with the locationInfo obtained after receiving RRCConnectionReconfiguration message }

}

8.6.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.10.9 and 5.5.5. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.10.9]

The UE shall:

…

1>	if the received otherConfig includes the obtainLocation:

2>	attempt to have detailed location information available for any subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

[TS 36.331, clause 5.5.5]

The purpose of this procedure is to transfer measurement results from the UE to E-UTRAN.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if the includeLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of the locationInfo as follows:

2>	include the locationCoordinates;

2>	if available, include the gnss-TOD-msec;

8.6.1.2.3	Test description

8.6.1.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.1.2.3.2	Test procedure sequence

Table 8.6.1.2.3.2-1 illustrates the downlink power levels to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.1.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 | The power level value is such that measurement result for Cell 1 satisfies leaving condition for event A2 (Ms - Hys > Thresh). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 | The power level value is such that measurement result for Cell 1 satisfies entering condition for event A2 (Ms + Hys < Thresh). |

Table 8.6.1.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement for event A2 with includeLocationInfo set to true. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 power level according to the row "T1" in table 8.6.1.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A2 with locationInfo. (Note 1) | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 power level according to the row "T0" in table 8.6.1.2.3.2-1. | - | - | - | - |
| 6 | The SS waits 10s. | - | - | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to modify intra-frequency measurement for event A2 with includeLocationInfo set to true and obtainLocation set to setup. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS waits [60s] to ensure that the UE has detailed location information available. (NOTE 2) | - | - | - | - |
| 10 | The SS changes Cell 1 power level according to the row "T1" in table 8.6.1.2.3.2-1. | - | - | - | - |
| 11 | Check: Does the UE transmit a MeasurementReport message to report event A2 with locationInfo? | --> | MeasurementReport | 1 | P |
| NOTE 1: The UE reports the detailed location information available stored in the UE. This intends to check that detailed location information reported in step 11 is obtained after receiving the RRCConnectionReconfiguration message in step 7. NOTE 2: Depending on UE's positioning engine used in this test case, any suitable method (e.g. test function Update UE Location Information defined in TS 36.509 [25]) may be used in this step to provide the position to the UE. |  |  |  |  |  |

8.6.1.2.3.3	Specific message contents

Table 8.6.1.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.1.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8 with condition MEAS

Table 8.6.1.2.3.3-2: MeasConfig (Table 8.6.1.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A2-RECONF |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.1.2.3.3-3: ReportConfigEUTRA-A2-RECONF (Table 8.6.1.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-5 ReportConfigEUTRA-A2(-83) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| hysteresis | 6 | 3dB |  |
| } |  |  |  |
| } |  |  |  |
| si-RequestForHO-r9 | Not present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not present |  |  |
| includeLocationInfo-r10 | true |  |  |
| reportAddNeighMeas-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.1.2.3.3-4: MeasurementReport (steps 4 and 11, Table 8.6.1.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells | Not present |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Any allowed value |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.2.3.3-5: RRCConnectionReconfiguration (step 7, Table 8.6.1.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 SEQUENCE { |  |  |  |
| reportProximityConfig-r9 | Not present |  |  |
| idc-Config-r11 | Not present |  |  |
| powerPrefIndicationConfig-r11 | Not present |  |  |
| obtainLocationConfig-r11 SEQUENCE { |  |  |  |
| obtainLocation-r11 | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.2.3.3-6: MeasConfig (Table 8.6.1.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A2-RECONF2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.2.3.3-7: ReportConfigEUTRA-A2-RECONF2 (Table 8.6.1.2.3.3-6)

| Derivation Path: 36.508, Table 4.6.6-5 ReportConfigEUTRA-A2(-83) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| hysteresis | 4 | 2dB |  |
| } |  |  |  |
| } |  |  |  |
| si-RequestForHO-r9 | Not present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not present |  |  |
| includeLocationInfo-r10 | true |  |  |
| reportAddNeighMeas-r10 | Not present |  |  |
| } |  |  |  |

#### 8.6.1.3	Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per QCI

8.6.1.3.1	Test Purpose (TP)

(1)

with {UE supporting UL PDCP Packet Delay per QCI measurement and in RRC_CONNECTED state}

ensure that {

when {UE receives RRCConnectionReconfiguration containing ReportConfigEUTRA IE with ul-DelayConfig IE set to “setup” and triggerType set to “periodical” and purpose set to “reportStrongestCells”}

then {UE sends the RRCConnectionReconfigurationComplete message}

}

(2)

with {UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per QCI and having DRB established with QCI1, QCI2, QCI3 or QCI4}

ensure that {
  when {UE determines that UL PDCP SDUs per QCI are exceeding the configured delay threshold }

then {UE sends the MeasurementReport message with ul-PDCP-DelayResultList for corresponding QCI}

}

(3)

with { UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per QCI}

ensure that {

when {UE receives RRCConnectionReconfiguration containing ReportConfigEUTRA IE with ul-DelayConfig IE set to “release”)

then {UE stops monitoring the UL PDCP Packet Delay per QCI}

}

8.6.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.5.3.1, 5.5.4.1, 5.5.5 & TS 36.314, clause 4.2.1.1

[TS 36.331, clause 5.5.3.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the purpose for the associated reportConfig is set to reportCGI:

2>	if the ul-DelayConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	configure the PDCP layer to perform UL PDCP Packet Delay per QCI measurement;

[TS 36.331, clause 5.5.4.1]

1>	for each measId included in the measIdList within VarMeasConfig:

2>	else if the purpose is included and set to reportStrongestCells or to reportStrongestCellsForSON and if a (first) measurement result is available:

3>	if the purpose is set to reportStrongestCells and reportStrongestCSI-RSs is not included:

4>	if the triggerType is set to periodical and the corresponding reportConfig includes the ul-DelayConfig:

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided by lower layers;

[TS 36.331, clause 5.5.5]

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	if uplink PDCP delay results are available:

2>	set the ul-PDCP-DelayResultList to include the uplink PDCP delay results available;

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

[TS 36.314, clause 4.2.1.1]

The objective of this measurement performed by UE is to measure Excess Packet Delay Ratio in Layer PDCP for QoS verification of MDT.

Table 4.2.1.1-0: Protocol Layer: RLC, PDCP

| Definition | PDCP Packet Delay in the UL per QCI. This measurement refers to packet delay for DRBs, which captures the delay from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.2.1.1-1 below. |
| --- | --- |

Table 4.2.1.1-1

|  | Ratio of packets in UL per QCI exceeding the configured delay threshold among the UL PDCP SDUs transmitted. |
| --- | --- |
|  | Number of PDCP SDUs of a data radio bearer with QCI = ,for which ULdelay exceeded the configured delayThreshold as defined in TS 36.331 [5] during the time period T. |
|  | Number of PDCP SDUs of a data radio bearer with QCI = , for which at least a part of SDU was transmitted during the time period T. |
|  | Queuing delay observed at the UE PDCP layer from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC, the packet belongs to a data radio bearer with QCI = . |
|  | The point in time when the PDCP SDU i of a data radio bearer with QCI =was delivered to lower layers. |
|  | The point in the time when the PDCP SDU i of a data radio bearer with QCI = arrives at PDCP upper SAP. |
|  | Index of PDCP SDU that arrives at the PDCP upper SAP during time period . |
|  | Time period during which the measurement is performed. |

Table 4.2.1.1.1-1: EXCESS DELAY RATIO measurement report mapping (5 –bit value)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| EXCESS DELAY RATIO_00 | 0 < EXCESS DELAY RATIO  0,079 | % |
| EXCESS DELAY RATIO_01 | 0,079 < EXCESS DELAY RATIO  0,100 | % |
| EXCESS DELAY RATIO_02 | 0,100 < EXCESS DELAY RATIO 0,126 | % |
| EXCESS DELAY RATIO_03 | 0,126 < EXCESS DELAY RATIO  0,158 | % |
| EXCESS DELAY RATIO_04 | 0,158 < EXCESS DELAY RATIO  0,199 | % |
| EXCESS DELAY RATIO_05 | 0,199 < EXCESS DELAY RATIO  0,251 | % |
| EXCESS DELAY RATIO_06 | 0,251 < EXCESS DELAY RATIO  0,316 | % |
| EXCESS DELAY RATIO_07 | 0,316 < EXCESS DELAY RATIO  0,398 | % |
| EXCESS DELAY RATIO_08 | 0,398 < EXCESS DELAY RATIO  0,501 | % |
| EXCESS DELAY RATIO_09 | 0,501 < EXCESS DELAY RATIO  0,631 | % |
| EXCESS DELAY RATIO_10 | 0,631 < EXCESS DELAY RATIO  0,794 | % |
| EXCESS DELAY RATIO_11 | 0,794 < EXCESS DELAY RATIO  1,000 | % |
| EXCESS DELAY RATIO_12 | 1,000 < EXCESS DELAY RATIO  1,259 | % |
| EXCESS DELAY RATIO_13 | 1,259 < EXCESS DELAY RATIO 1,585 | % |
| EXCESS DELAY RATIO_14 | 1,585 < EXCESS DELAY RATIO 1,995 | % |
| EXCESS DELAY RATIO_15 | 1,995 < EXCESS DELAY RATIO 2,511 | % |
| EXCESS DELAY RATIO_16 | 2,511 < EXCESS DELAY RATIO 3,161 | % |
| EXCESS DELAY RATIO_17 | 3,161 < EXCESS DELAY RATIO 3,980 | % |
| EXCESS DELAY RATIO_18 | 3,980 < EXCESS DELAY RATIO 5,011 | % |
| EXCESS DELAY RATIO_19 | 5,011 < EXCESS DELAY RATIO 6,309 | % |
| EXCESS DELAY RATIO_20 | 6,309 < EXCESS DELAY RATIO 7,943 | % |
| EXCESS DELAY RATIO_21 | 7,943 < EXCESS DELAY RATIO 10,00 | % |
| EXCESS DELAY RATIO_22 | 10,00 < EXCESS DELAY RATIO 12,589 | % |
| EXCESS DELAY RATIO_23 | 12,589 < EXCESS DELAY RATIO 15,849 | % |
| EXCESS DELAY RATIO_24 | 15,849 < EXCESS DELAY RATIO 19,953 | % |
| EXCESS DELAY RATIO_25 | 19,953 < EXCESS DELAY RATIO 25,119 | % |
| EXCESS DELAY RATIO_26 | 25,119 < EXCESS DELAY RATIO 31,623 | % |
| EXCESS DELAY RATIO_27 | 31,623 < EXCESS DELAY RATIO 39,811 | % |
| EXCESS DELAY RATIO_28 | 39,811 < EXCESS DELAY RATIO 50,119 | % |
| EXCESS DELAY RATIO_29 | 50,119 < EXCESS DELAY RATIO 63,096 | % |
| EXCESS DELAY RATIO_30 | 63,096 < EXCESS DELAY RATIO 79,433 | % |
| EXCESS DELAY RATIO_31 | 79,433 < EXCESS DELAY RATIO 100 | % |

8.6.1.3.3	Test description

8.6.1.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

-	None

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) according to [18].

8.6.1.3.3.2	Test procedure sequence

Table 8.6.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS performs the generic procedure to get UE in Loopback Activated (state 4) with condition UE TEST LOOP MODE B with a Reference default EPS bearer context #1 and Reference dedicated EPS bearer context #1 and Reference default EPS bearer #2 and Reference dedicated EPS bearer context #3 according to [18] on Cell A | - | - | - | - |
| 1 | The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup UL-DelayConfig | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #1 | - | - | - | - |
| 4 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #3 | - | - | - | - |
| 5 | SS provides sufficient UL grants to ensure UE can loopback IP packets without delay and waits for duration indicated in the IE reportInterval | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 6 UE transmits IP packets on the data radio bearers associated with the dedicated EPS bearer contexts #1 and #3. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayResultList for QCI 1 and/or QCI 2 | --> | MeasurementReport | 2 | F |
| 7 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #1 |  |  |  |  |
| 8 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #3 |  |  |  |  |
| 9 | SS doesn’t provide UL grants to the UE for the duration indicated by IE delayThreshold-r13 and waits for duration indicated in the IE reportInterval (Note 1) |  |  |  |  |
| - | EXCEPTION: In parallel to the event described in step 10 UE transmits IP packets on the data radio bearers associated with the dedicated EPS bearer contexts #1 and #3. | - | - | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayResultList for QCI 1 and QCI 2 | --> | MeasurementReport | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message including MeasConfig to release the UL-DelayConfig measurement | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete |  |  |
| 13 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #1 | - | - | - | - |
| 14 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated EPS bearer contex #3 | - | - | - | - |
| 15 | SS doesn’t provide UL grants to the UE for the duration indicated by IE delayThreshold-r13 and waits for duration indicated in the IE reportInterval (Note 1) | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 16 UE transmits IP packets on the data radio bearers associated with the dedicated EPS bearer contexts #1 and #3. | - | - | - | - |
| 16 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayResultList for QCI 1 and QCI 2 | --> | MeasurementReport | 3 | F |
| Note 1: dsr-TransMax shall be set to 64 to ensure UE doesn’t initiate PRACH preamble before delayThreshold-r13 (ms750) expires. |  |  |  |  |  |

8.6.1.3.3.3	Specific message contents

Table 8.6.1.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.1.3.3.2-1)

| Derivation Path: TS 36.508 clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.3.3.3-2: MeasConfig (step 1, Table 8.6.1.3.3.2-1)

| Derivation Path: TS 36.508, clause 4.6.6 Table 4.6.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  | Band > 64 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  |  |  |
| } |  |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE{ |  |  |  |  |
| reportConfigId[1] | IdReportConfig- PERIODICAL-CSI-RS |  |  |  |
| reportConfig[1] | reportConfigEUTRA- PERIODICAL-CSI-RS |  |  |  |
| } |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE{ | 1 entry |  |  |  |
| measId[1] | 1 |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |
| reportConfigId[1] | IdReportConfig- PERIODICAL-CSI-RS |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |  |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.1.3.3.3-3: reportConfigEUTRA- PERIODICAL-CSI-RS (step 1, Table 8.6.1.3.3.2-1)

| Derivation path: TS 36.508 clause 4.6.6 table 4.6.6-12 ReportConfigEUTRA-PERIODICAL-CSI-RS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms5120 |  |  |
| reportAmount | r64 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | false |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useAllowedCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| delayThreshold-r13 | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.3.3.3-3A: SchedulingRequest-Configuration (RRCConnectionReconfiguration, step 1 table 8.6.1.3.3.2-1)

| Derivation Path: 36.508 clause 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Configuration ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| dsr-TransMax | n64 | Max value allowed |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.3.3.3-4: MeasurementReport (step 10, Table 8.6.1.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| measId-v1250 | Not present |  |  |
| measResultPCell-v1250 | Not present |  |  |
| measResultCSI-RS-List-r12 | Not present |  |  |
| measResultForRSSI-r13 | Not present |  |  |
| measResultServFreqListExt-r13 | Not present |  |  |
| measResultSSTD-r13 | Not present |  |  |
| measResultPCell-v1310 | Not present |  |  |
| ul-PDCP-DelayResultList-r13 SEQUENCE (SIZE (1..maxQCI-r13)) OF SEQUENCE { | 2 entries |  |  |
| qci-Id-r13[1] | qci1 |  |  |
| excessDelay-r13[1] | 31 |  |  |
| qci-Id-r13[2] | qci2 |  |  |
| excessDelay-r13[2] | 31 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.3.3.3-5: RRCConnectionReconfiguration (step 11, Table 8.6.1.3.3.2-1)

| Derivation Path: TS 36.508 clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.1.3.3.3-6: MeasConfig (step 11, Table 8.6.1.3.3.2-1)

| Derivation Path: TS 36.508, clause 4.6.6 Table 4.6.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| measObjectToAddModList SEQUENCE {} | Not Present |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE{ |  |  |  |  |
| reportConfigId[1] | IdReportConfig- PERIODICAL-CSI-RS |  |  |  |
| reportConfig[1] | reportConfigEUTRA- PERIODICAL-CSI-RS |  |  |  |
| } |  |  |  |  |
| measIdToAddModList SEQUENCE {} | Not Present |  |  |  |
| } |  |  |  |  |

Table 8.6.1.3.3.3-7: reportConfigEUTRA- PERIODICAL-CSI-RS (step 11, Table 8.6.1.3.3.2-1)

| Derivation path: TS 36.508 clause 4.6.6 table 4.6.6-12 ReportConfigEUTRA-PERIODICAL-CSI-RS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms10240 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | false |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useAllowedCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.1.4	Void

#### 8.6.1.5	Void

### 8.6.2	Logged MDT

#### 8.6.2.1	Logged MDT / Intra-frequency measurement, logging and reporting

8.6.2.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured without areaConfiguration and the UE is able to detect an E-UTRA intra-frequency cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and Intra-frequency neighbouring cell measurements }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has one or more logged Intra-frequency neighbouring cell measurement entries stored in VarLogMeasReport stored and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving RRCConnectionSetup message }

then { UE includes the logMeasAvailable IE in the RRCConnectionSetupComplete message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged Intra-frequency neighbouring cell measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with Intra-frequency neighbouring cell measurements }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with absoluteTimeStamp set equal to the value configured when the logged measurement configuration was received and a relativeTimeStamp for each logged measurement indicating the elapsed time since the logged measurement configuration was received }

}

(5)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with traceReference, traceRecordingSessionRef and tce-Id being set to same values as configured when the logged measurement configuration was received }

}

8.6.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.2 and 5.5.5.

[TS 36.304, clause 8 (TP1)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.3.3.4 (TP2)]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include logMeasAvailable;

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3 (TP3, TP4, TP5)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.3 (TP1, TP4, TP5)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1, TP4)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.2.2 (TP1)]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				,

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.2.1.3	Test description

8.6.2.1.3.1	Pre-test conditions

System Simulator:

-	Two intra-frequency cells belonging to the same PLMN, but to different tracking areas: Cell 1, Cell 11

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of intra-frequency neighbouring cell list in SIB4 set as defined in TS 36.508 [18] table 6.3.1.1-1.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.2.1.3.2	Test procedure sequence

Table 8.6.2.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.6.2.1.3.2-2.

Table 8.6.2.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.6.2.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.6.2.1.3.2-1. (Note 1) | - | - | - | - |
| 5 | The UE transmits an RRCConnectionRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) | --> | RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCConnectionSetupComplete message? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 2 | P |
| 8-10 | Steps 4 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 11-19 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 20 | The SS transmits a UEInformationRequest message on Cell 11. | <-- | UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with the IEs absoluteTimeStamp, traceReference-r10, traceRecordingSessionRef and tce-Id set to same value as received in the LoggedMeasurementConfiguration message in step 1; and a LogMeasInfoList with at least two entries with serving cell idle mode measurements and where in at least one of the entries the IE measResultListEUTRA include a neighbouring cell measurement of Cell 11; and where the relativeTimeStamp is increased between the subsequent LogMeasInfoList entries by at least the value of configured loggingInterval in the received LoggedMeasurementConfiguration message in step 1. | --> | UEInformationResponse | 1,3,4,5 | P |
| Note 1: The change of power levels is to trigger a cell re-selection procedure to make sure that the UE is logging neighbouring cell measurements (logging interval=2.56 seconds) of Cell 11 while t-Reselection timer is running (7 seconds). |  |  |  |  |  |

8.6.2.1.3.3	Specific message contents

Table 8.6.2.1.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA |  | 7 | seconds |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.1.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.1.3.3-3: RRCConnectionSetupComplete (step 7, Table 8.6.2.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.1.3.4: UEInformationRequest (step 20, Table 8.6.2.1.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.1.3.3-5: UEInformationResponse (step 21, Table 8.6.2.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same EARFCN as Cell 11 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 11 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 11 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 8.6.2.2	Logged MDT / Inter-frequency measurement, logging and reporting

8.6.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where loggedmeasurement is configured without areaConfiguration and the UE is able to detect an E-UTRA inter-frequency cell }

ensure that {
  when { T330 is running }

then { UE is logging Inter-frequency neighbouring cell measurements }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has one or more logged Inter-frequency neighbouring cell measurement entries stored in VarLogMeasReport stored and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with Inter-frequency neighbouring cell measurements }

}

8.6.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.6.5.3, 5.6.6.3 and 5.6.8.2.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

….

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.2.2.3	Test description

8.6.2.2.3.1	Pre-test conditions

System Simulator:

-	Two inter-frequency cells belonging to the same PLMN, but to different tracking areas: Cell 1 and Cell 23

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of inter-frequency neighbouring cell list in SIB5 set as defined in TS 36.508 [18] table 6.3.1.2-1.

UE

None.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.2.2.3.2	Test procedure sequence

Table 8.6.2.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.6.2.2.3.2-2.

Table 8.6.2.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 23. |

Table 8.6.2.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 23 levels according to the row "T1" in table 8.6.2.2.3.2-1. (Note 1) | - | - | - | - |
| 5-10 | Steps 1 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 23. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 11-19 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 20 | SS transmits a UEInformationRequest message. | <-- | UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least one neighbouring cell measurement of Cell 23. | --> | UEInformationResponse | 1,2 | P |
| Note 1: The change of power levels is to trigger the cell re-selection procedure to make sure that the UE is logging neighbouring cell measurements (logging interval=2.56 seconds) of Cell 23 while t-Reselection timer is running (7 seconds). |  |  |  |  |  |

8.6.2.2.3.3	Specific message contents

Table 8.6.2.1.3.3-1: SystemInformationBlockType5 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |
| t-ReselectionEUTRA[1] |  | 7 | Seconds |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.2.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.2.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | Ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.2.3.3-3: UEInformationRequest (step 20, Table 8.6.2.2.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A, condition “Logged MDT”

Table 8.6.2.2.3.3-4: UEInformationResponse (step 21, Table 8.6.2.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value | The relativeTimeStamp is increased between the subsequent LogMeasInfoList entries by at least the value of configured loggingInterval in the received LoggedMeasurementConfiguration message in step 1. |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same EARFCN as Cell 23 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 23 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 23 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 8.6.2.3	Logged MDT / Logging and reporting / Limiting area scope

8.6.2.3.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a LTE cell}

ensure that {

when { UE camps on a LTE cell in the cellGlobalIdList}

then { UE performs logged measurements}

}

(2)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a LTE cell}

ensure that {
  when { UE camps on a LTE cell not in the cellGlobalIdList}

then { UE does not perform logged measurements}

}

(3)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a LTE cell}

ensure that {
  when { UE camps on a LTE cell in the trackingAreaCodeList}

then { UE performs logged measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a LTE cell}

ensure that {
  when { UE camps on a LTE cell not in the trackingAreaCodeList}

then { UE does not perform logged measurements }

}

8.6.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331 clauses 5.6.6.3, 5.6.8.2, 6.3.4 and 6.3.6.

[TS 36.304, clause 8 (TP1,TP2,TP3,TP4)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.6.3 (TP1,TP2,TP3,TP4)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1,TP2,TP3,TP4)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.3.6 (TP1, TP2, TP3, TP4)]

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging applies in the entire RPLMN of the UE at the point of receiving the configuration

AreaConfiguration information element

-- ASN1START

AreaConfiguration-r10 ::=	CHOICE {

cellGlobalIdList-r10			CellGlobalIdList-r10,

trackingAreaCodeList-r10		TrackingAreaCodeList-r10

}

CellGlobalIdList-r10 ::=				SEQUENCE (SIZE (1..32)) OF CellGlobalIdEUTRA

TrackingAreaCodeList-r10 ::=			SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

-- ASN1STOP

[TS 36.331, clause 6.3.4 (TP1,TP2)]

The IE CellGlobalIdEUTRA specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA.

CellGlobalIdEUTRA information element

-- ASN1START

CellGlobalIdEUTRA ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellIdentity							CellIdentity

}

-- ASN1STOP

[TS 36.331, clause 6.3.4 (TP3,TP4)]

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.301 [35].

TrackingAreaCode information element

-- ASN1START

TrackingAreaCode ::=				BIT STRING (SIZE (16))

-- ASN1STOP

8.6.2.3.3	Test description

8.6.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 11.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18]

8.6.2.3.3.2	Test procedure sequence

Table 8.6.2.3.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2”, “T3”and “T4” are applied at the points indicated in the Main behaviour description in Table 8.6.2.3.3.2-2.

Table 8.6.2.3.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 11 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | -91 | -91 | Configure logged measurement with limiting area scope using cellGlobalIdList |
| T1 | RS EPRE | dBm/15kHz | -91 | -91 | -85 | Verify that UE is perform logging of Cell 11 (TP1, Cell 11 is configured in cellGlobalIdList) |
| T2 | RS EPRE | dBm/15kHz | -91 | -85 | -91 | Verify that UE is not logging Cell 2 (TP2, Cell 2 is not configured in cellGlobalIdList) and then re-configure logged measurement with limiting area scope using trackingAreaCodeList |
| T3 | RS EPRE | dBm/15kHz | -91 | -91 | -85 | Verify that UE is not perform logging of Cell 11 (TP4, Cell 11 is not in configured trackingAreaCodeList) |
| T4 | RS EPRE | dBm/15kHz | -85 | -91 | -91 | Verify that UE is logging Cell 1 (TP3, Cell 2 is in configured trackingAreaCodeList) |

Table 8.6.2.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message with a cellGlobalIdList on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T1" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 4-9 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 11. | - | - | - | - |
| 10 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 11-18 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 19 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 20 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for Cell 11? | --> | UEInformationResponse | 1 | P |
| 21 | SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 22 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T2" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 23-28 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 2. | - | - | - | - |
| 29 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 30 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 31 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 32 | SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 2 then the steps 33a1-33a7 are executed. |  |  |  |  |
| 33a1 | The UE transmit an RRCConnectionSetupComplete message including logMeasAvailable IE set it to true. | --> | RRCConnectionSetupComplete | - | - |
| 33a2-33a5 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 2. | - | - | - | - |
| 33a6 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 33a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 2? | --> | UEInformationResponse | 2 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 2 then the step 33b1 is executed. | - | - | - | - |
| 33b1 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 2 | P |
| 34 | The SS transmits a LoggedMeasurementConfiguration message with a trackingAreaCodeList on Cell 2. | <-- | LoggedMeasurementConfiguration | - | - |
| 35 | The SS transmits a RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on Cell 2. | <-- | RRCConnectionRelease | - | - |
| 36 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T3" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 37-42 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 11. | - | - | - | - |
| 43 | The SS transmits a Paging message on Cell 11. | <-- | Paging | - | - |
| 44 | The UE transmits an RRCConnectionRequest message on Cell 11. | --> | RRCConnectionRequest | - | - |
| 45 | The SS transmit an RRCConnectionSetup message on Cell 11. | <-- | RRCConnectionSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 11 then the steps 46a1-46a7 are executed. |  |  |  |  |
| 46a1 | The UE transmit an RRCConnectionSetupComplete message including logMeasAvailable IE set it to true. | --> | RRCConnectionSetupComplete | - | - |
| 46a2-46a5 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 46a6 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 46a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 11? | --> | UEInformationResponse | 4 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 11 then the step 46b1 is executed. | - | - | - | - |
| 46b1 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 4 | P |
| 47 | The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 48 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T4" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 49-54 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 1. | - | - | - | - |
| 55 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 56-63 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 64 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 65 | Check: Does the UE send an UEInformationResponse include logMeasReport with logged serving cell measurements of Cell 1? | --> | UEInformationResponse | 3 | P |

8.6.2.3.3.3	Specific message contents

Table 8.6.2.3.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA | 7 | seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| areaConfiguration-r10 CHOICE { |  |  |  |  |
| cellGlobalIdList-r10 SEQUENCE { |  | 1 entry |  |  |
| plmn-Identity[1] |  | plmn-Identity of Cell 11 |  |  |
| cellIdentity[1] |  | cellIdentity of Cell 11 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-3: UEInformationRequest (step 19, 33a6, 46a6 and 64, Table 8.6.2.3.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.3.3.3-4: UEInformationResponse (step 20 and 33a7, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 11 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-5: RRCConnectionSetupComplete (step 33a1 and 46a1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.3.3.3-6: RRCConnectionSetupComplete (step 33b1 and 46b1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.3.3.3-7: LoggedMeasurementConfiguration (step 34, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| areaConfiguration-r10 CHOICE { |  |  |  |  |
| trackingAreaCodeList -r10 SEQUENCE { |  | 1 entry |  |  |
| trackingAreaCode [1] |  | TAC = 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-8: UEInformationResponse (step 46a7, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 2 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-9: UEInformationResponse (step 65, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.2.3a	Logged MDT / Logging and reporting / Limiting area scope / TAC list with PLMN identity

8.6.2.3a.1	Test Purpose (TP)

(1)

with { UE is in E-UTRA RRC_IDLE state and the RPLMN is included in plmn-IdentityList received in LoggedMeasurementConfiguration message with trackingAreaCodeList and plmn-Identity-perTAC-List on a LTE cell }

ensure that {
  when { UE camps on a LTE cell in the trackingAreaCodeList and plmn-Identity-perTAC-List }

then { UE performs logged measurements }

}

8.6.2.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331, clauses 5.3.3.4, 5.6.5.3 and 5.6.8.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	The RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception, or is present in the plmn-IdentityList (see TS 36.331 [3]) if configured;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.3.3.4]

The UE shall:

...

1>	set the content of RRCConnectionSetupComplete message as follows:

...

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include logMeasAvailable;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

8.6.2.3a.3	Test description

8.6.2.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.2.3a.3.1-1.

Table 8.6.2.3a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.2.3a.3.3-5

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.2.3a.3.2	Test procedure sequence

Table 8.6.2.3a.3.2-1 illustrates the downlink power levels to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.2.3a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | Only Cell 1 is available. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 12. |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.2.3a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including on Cell 1 to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 3 | The SS changes Cell 1 and Cell 12 levels according to the row "T1" in table 8.6.2.3a.3.2-1. | - | - | - | - |
| 4 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on Cell 12. NOTE: The UE performs a TAU procedure. The TAU is accepted with PLMN1 listed as an Equivalent PLMN and the RRC connection is released. | - | - | - | - |
| 5 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 12. | - | - | - | - |
| 14 | The SS transmits a UEInformationRequest message on Cell 12. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message with LogMeasInfoList on Cell 12? | --> | UEInformationResponse | 1 | P |

8.6.2.3a.3.3	Specific message contents

Table 8.6.2.3a.3.3-0: Conditions for specific message contents
in Tables 8.6.2.3a.3.3-4 and 8.6.2.3a.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.2.3a.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| areaConfiguration-r10 CHOICE { |  |  |  |
| trackingAreaCodeList-r10 SEQUENCE (SIZE (1..8)) OF { | 1 entry |  |  |
| TrackingAreaCode [1] | TAC = 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF { | 1 entry |  |  |
| PLMN-Identity [1] | PLMN2 |  |  |
| } |  |  |  |
| areaConfiguration-v1130 SEQUENCE { |  |  |  |
| trackingAreaCodeList-v1130 SEQUENCE { |  |  |  |
| plmn-Identity-perTAC-List-r11 SEQUENCE (SIZE (1..8)) OF { | 1 entry |  |  |
| PLMN-Identity [1] | PLMN2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3a.3.3-1A: RRCConnectionSetupComplete (step 4, Table 8.6.2.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | Not present or any allowed value |  |  |
| rlf-InfoAvailable-r10 | Not present or any allowed value |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| rn-SubframeConfigReq-r10 | Not present or any allowed value |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3a.3.3-1B: Message TRACKING AREA UPDATE ACCEPT (step 4, Table 8.6.2.3a.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.2.3a.3.3-2: RRCConnectionSetupComplete (step9, Table 8.6.2.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | Not present or any allowed value |  |  |
| rlf-InfoAvailable-r10 | Not present or any allowed value |  |  |
| logMeasAvailable-r10 | true |  |  |
| rn-SubframeConfigReq-r10 | Not present or any allowed value |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3a.3.3-3: UEInformationRequest (step 14, Table 8.6.2.3a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A condition Logged MDT

Table 8.6.2.3a.3.3-4: UEInformationResponse (step 15, Table 8.6.2.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-MNC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE { | All entries comply to entry with index ‘x’ below. |  |  |
| locationInfo-r10 [x] | Not present or any allowed value |  |  |
| relativeTimeStamp-r10 [x] | Any allowed value |  |  |
| servCellIdentity-r10 [x] SEQUENCE { |  |  |  |
| plmn-Identity [x] | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity [x] | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |
| rsrpResult-r10 [x] | (0..97) |  |  |
| rsrqResult-r10 [x] | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r10 [x] | Not present or any value |  | If it has been included in any other LogMeasInfo-R10 instance |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  | The IE shall be present at least in 1 instance of LogMeasInfo-R10. |
| measResultListEUTRA-r10 [x] SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9 [1] SEQUENCE { |  |  |  |
| physCellId [1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info [1] | Not present |  |  |
| measResult [1] SEQUENCE { |  |  |  |
| rsrpResult [1] | Not present or (0..97) |  |  |
| rsrqResult [1] | Not present or (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r10 | Not present |  |  |
| measResultListGERAN-r10 | Not present |  |  |
| measResultListCDMA2000-r10 | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3a.3.3-5: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.2.3a.3.3-6: SystemInformationBlockType5 for Cell 12 (preamble and all steps, Table 8.6.2.3a.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.2.4	Logged MDT / Logging and reporting / Indication of logged measurements at E-UTRA handover

8.6.2.4.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo}

then { UE transmits an RRCConnectionReconfigurationComplete message with logMeasAvailable is true}

}

(2)

with { UE having indicated availability of logged measurements in RRCConnectionReconfiguration message }

ensure that {

when { UE has completed the E-UTRA handover procedure and receives a UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message including logMeasReport}

8.6.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4 and 5.6.5.3.

[TS 36.331, clause 5.3.5.4 (TP1)]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

….

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.2.4.3	Test description

8.6.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 4.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of intra-frequency neighbouring cell list in SIB4 set as defined in TS 36.508 [18] table 6.3.1.1-1.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.2.4.3.2	Test procedure sequence

Table 8.6.2.4.3.2-1shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” are applied at the points indicated in the Main behaviour description in Table 8.6.2.4.3.2-2.

Table 8.6.2.4.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy exit condition for event A3 (M4 < M1). |
| T1 | RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1). |

Table 8.6.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4-11 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS changes Cell 1 and Cell 4 parameters according to the row "T1" in table 8.6.2.4.3.2-1. | - | - | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4 with logMeasAvailable is true confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 15 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 16 | Check: Does the UE send an UEInformationReponse message include logMeasReport. | --> | UEInformationResponse | 2 | P |

8.6.2.4.3.3	Specific message contents

Table 8.6.2.4.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA | 7 | seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.4.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.4.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.4.3.3-3: RRCConnectionReconfigurationComplete (step 14, Table 8.6.2.4.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.4.3.3-4: UEInformationRequest (step 15, Table 8.6.2.4.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.4.3.3-5: UEInformationResponse (step 16, Table 8.6.2.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.2.5	Logged MDT / Logging and reporting / Indication of logged measurements at E-UTRA re-establishment

8.6.2.5.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE has initiated a re-establishment procedure and receives an RRCConnectionReestablishment message}

then { UE transmits an RRCConnectionReestablishmentComplete message with logMeasAvailable is true}

}

(2)

with { UE indicated availability of logged measurements in RRCConnectionReestablishmentComplete message }

ensure that {

when { UE has successfully completes the re-establishment procedure and resume the existing radio bearer; and has receive a UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message including logMeasReport}

8.6.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.7.5 (TP1)]

The UE shall:

1>	stop timer T301;

…

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.2.5.3	Test description

8.6.2.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of intra-frequency neighbouring cell list in SIB4 set as defined in TS 36.508 [18] table 6.3.1.1-1.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18]

8.6.2.5.3.2	Test procedure sequence

Table 8.6.2.5.3.2-1shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” are applied at the points indicated in the Main behaviour description in Table 8.6.2.5.3.2-2.

Table 8.6.2.5.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | Off | -85 |

Table  8.6.2.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4-12 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 13 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.6.2.5.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 2 is suitable for camping. | - | - | - | - |
| 14 | The UE send RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 15 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 16 | Check: Does the UE transmit RRCConnectionReestablishmentComplete message with logMeasAvailable set as true? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 17 | The SS transmits an RRCConnectionReconfiguration message to resume the existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 18 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 19 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE send an UEInformationResponse message include logMeasReport. | --> | UEInformationResponse | 2 | P |

8.6.2.5.3.3	Specific message contents

Table 8.6.2.5.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA | 7 | seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.5.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.5.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.5.3.3-2A: RRCConnectionReestablishmentRequest (step 14, Table 8.6.2.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.5.3.3-3: RRCConnectionReestablishmentComplete (step 16, Table 8.6.2.5.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-11 RRCConnectionReestablishmentComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.5.3.3-4: UEInformationRequest (step 19, Table 8.6.2.5.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.5.3.3-5: UEInformationResponse (step 20, Table 8.6.2.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.2.6	Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer

8.6.2.6.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {
  when { The logging duration timer T330 expires}

then { UE release VarLogMeasConfig and will not perform logged measurements }

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}}

ensure that {

when { UE has released VarlogMeasConfig due to timer T330 has expired and UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message including logMeasReport}

(3)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE has transmitted all the stored logMeasReport and UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message without logMeasReport}

8.6.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.6.5.3, 5.6.6.3 and 5.6.6.4.

[TS 36.331, clause 5.6.5.3 (TP2, TP3)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.4 (TP1)]

Upon expiry of T330 the UE shall:

1>	release VarLogMeasConfig;

The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport 48 hours after T330 expiry.

8.6.2.6.3	Test description

8.6.2.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18]

8.6.2.6.3.2	Test procedure sequence

Table 8.6.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4-12 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 13 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 14 | The UE send an UEInformationResponse message include logMeasReport-r10. | --> | UEInformationResponse | - | - |
| - | EXCEPTION: In case the logMeasAvailable-r10 in logMeasReport-r10 is True, steps 14a1 and 14a2 will be executed. | - | - | - | - |
| 14a1 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 14a2 | The UE sends an UEInformationResponse message include logMeasReport-r10. | --> | UEInformationResponse | - | - |
| - | EXCEPTION: In case the logMeasAvailable-r10 in logMeasReport-r10 is True, steps 14a1 and 14a2 will be executed. | - | - | - | - |
| 15 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE send an UEInformationResponse message not include logMeasReport? | --> | UEInformationResponse | 3 | P |
| 17 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 18 | Wait 10 minutes for UE performing the logging at regular time intervals as to ensure timer T330 has expired. | - | - | - | - |
| 19-27 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 28 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 29 | Check: Does the UE send an UEInformationResponse message include logMeasReport? | --> | UEInformationResponse | 2 | P |
| - | EXCEPTION: In case the logMeasAvailable-r10 in logMeasReport-r10 is True, steps 29a1 and 29a2 will be executed. | - | - | - | - |
| 29a1 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 29a2 | UE sends an UEInformationResponse message include logMeasReport. | --> | UEInformationResponse | - | - |
| - | EXCEPTION: In case the logMeasAvailable-r10 in logMeasReport-r10 is True, steps 29a1 and 29a2 will be executed. | - | - | - | - |
| 30 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 31 | Wait 30 seconds. |  |  |  |  |
| 32 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 33 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 34 | The SS transmit an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 35 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 1 | P |

8.6.2.6.3.3	Specific message contents

Table 8.6.2.6.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.6.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingDuration-r10 |  | min10 | 10 minutes |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.6.3.3-2: RRCConnectionSetupComplete (step 8 and step 23, Table 8.6.2.6.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.6.3.3-3: UEInformationRequest (step 13, step 14a1, step 15, step 28 and step 29a1, Table 8.6.2.6.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.6.3.3-4: UEInformationResponse (step 14, step 14a2, step 29 and step 29a2, Table 8.6.2.6.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | A least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not present |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present or True |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.6.3.3-5: UEInformationResponse (step 16, Table 8.6.2.6.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReportReq-r10 SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.6.3.3-6: RRCConnectionSetupComplete (step 35, Table 8.6.2.6.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

#### 8.6.2.7	Logged MDT / Release of logged MDT measurement configuration / Reception of new logged measurement configuration, Detach or UE power off

8.6.2.7.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE receive a new LoggedMeasurementConfiguration message }

then { UE discards the stored logMeasReport }

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE receive a new LoggedMeasurementConfiguration message }

then { UE discards the old logged measurement configuration }

}

(3)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE is switched off or detaches from the EPS service }

then { UE discards the stored logMeasReport }

}

(4)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE is switched off or detaches from the EPS service }

then { UE discards the logged measurement configuration }

}

8.6.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.6.5.3, and 5.6.7.2.

[TS 36.331, clause 5.6.5.3 (TP1, TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

1>	if rach-ReportReq is set to true, set the contents of the rach-Report in the UEInformationResponse message as follows:

2>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the last successfully completed random access procedure;

2>	if contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the last successfully completed random access procedure:

3>	set the contentionDetected to true;

2>	else:

3>	set the contentionDetected to false;

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.7.2 (TP1, TP2, TP3, TP4)]

The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or detach.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport;

8.6.2.7.3	Test description

8.6.2.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None

Preamble:

-	-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18]

8.6.2.7.3.2	Test procedure sequence

Table 8.6.2.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4-12 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 13 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 14 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 15 | Check: Does the UE send an UEInformationResponse message include logMeasReport? | --> | UEInformationResponse | 1 | F |
| 16 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 17 | Wait 30 seconds for UE performing the logging at regular time intervals |  |  |  |  |
| 18-26 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 27 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 28 | Check: Does the UE send an UEInformationResponse message include logMeasReport based on the LoggedMeasurementConfiguration as in step 1? | --> | UEInformationResponse | 2 | F |
| 29 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 30 | The UE is switched off, or detaches from the network. | - | - | - | - |
| 31 | The UE is switched on, or attaches to the network. | - | - | - | - |
| 32-47 | The UE performs steps 2-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1. | - | - | - | - |
| 48 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 49 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 50 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 51 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 52 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 3,4 | P |

8.6.2.7.3.3	Specific message contents

Table 8.6.2.7.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.7.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.7.3.3-2: RRCConnectionSetupComplete (step 8 and 22, Table 8.6.2.7.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.7.3.3-3: LoggedMeasurementConfiguration (step 13, Table 8.6.2.7.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms5120 | 5.12 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.7.3.3-4: UEInformationRequest (step 14 and 27, Table 8.6.2.7.3.2-1)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.7.3.3-5: UEInformationResponse (step 15, Table 8.6.2.7.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.7.3.3-6: UEInformationResponse (step 28, Table 8.6.2.7.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not present |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.7.3.3-7: RRCConnectionSetupComplete (step 52, Table 8.6.2.7.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

#### 8.6.2.8	Logged MDT / Maintaining logged measurement configuration / UE state transitions and mobility

8.6.2.8.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE reselected to a cell belong to non MDT PLMN}

then { UE does not indicate availability of Logged MDT measurements}

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE reselects to a cell belong to non MDT PLMN}

then { UE suspends Logged MDT measurements}

}

(3)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN}

then { UE indicate availability of Logged MDT measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN}

then { UE resumes Logged MDT measurements }

}

(5)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE moves to RRC_CONNECTED state }

then { UE maintains the Logged measurement configurations and logged measurement reports}

}

(6)

with { UE received LoggedMeasurementConfiguration message}

ensure that {

when { UE is not in ‘camped normally’ state on an E-UTRA cell }

then { UE stop performing logged measurements and T330 keep running}

}

(7)

with { UE received LoggedMeasurementConfiguration message and is not in ‘camped normally’ state on an E-UTRA cell}

ensure that {

when { UE is again in ‘camped normally’ state on an E-UTRA cell }

then { UE resumes logged MDT measurements }

}

8.6.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.6.5.3, 5.6.6.3 and 5.6.6.4.

[TS 36.304, clause 8 (TP1, TP2, TP5, TP6)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.3.3.4 (TP1, TP3)]

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>		if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

2>	if connecting as an RN:

3>	include the rn-SubframeConfigReq;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include logMeasAvailable;

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.8.2 (TP4, TP5, TP7)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

8.6.2.8.3	Test description

8.6.2.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell2,  Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	When Cell 3 is used its PLMN will be set to PLMN2

-	Cell 2 TAC is set to 0002

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18]

8.6.2.8.3.2	Test procedure sequence

Table 8.6.2.8.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2”, “T3”and “T4” are applied at the points indicated in the Main behaviour description in Table 8.6.2.8.3.2-2.

Table 8.6.2.8.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | Off | Off |
| T1 | RS EPRE | dBm/15kHz | Off | Off | -85 |
| T2 | RS EPRE | dBm/15kHz | Off | -85 | Off |
| T3 | RS EPRE | dBm/15kHz | Off | Off | Off |
| T4 | RS EPRE | dBm/15kHz | -85 | Off | Off |

Table 8.6.2.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 10 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 3 level according to the row "T1" in Table 8.6.2.8.3.2-1. | - | - | - | - |
| 5-10 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 3. | - | - | - | - |
| 11 | Wait 10 seconds for UE to perform the logging at regular time intervals. | - | - | - | - |
| 12 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 13 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 14 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 15 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 1 | P |
| 16 | The SS transmits a RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on Cell 3. | <-- | RRCConnectionRelease | - | - |
| 17 | The SS changes Cell 1, Cell 2 and Cell 3 level according to the row "T2" in Table 8.6.2.8.3.2-1. | - | - | - | - |
| 18-23 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. | - | - | - | - |
| 24 | Wait 10 seconds for UE performing the logging at regular time intervals |  |  |  |  |
| 25 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 26 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 27 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 28 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable set as true? | --> | RRCConnectionSetupComplete | 3 | P |
| 29-32 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 33 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 34 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for Cell 1 and Cell 2 and without serving cell measurements for Cell 3? | --> | UEInformationResponse | 2, 4, 5 | P |
| 35 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on Cell 2. | <-- | RRCConnectionRelease | - | - |
| 36 | Wait 10 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 37 | The SS changes Cell 2 and Cell 3 level according to the row "T3" in Table 8.6.2.8.3.2-1. | - | - | - | - |
| 38 | Wait 15 seconds. (Note 1) | - | - | - | - |
| 39 | The SS changes Cell 1 level according to the row "T4" in Table 8.6.2.8.3.2-1. | - | - | - | - |
| 39A1-39A6 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. | - | - | - | - |
| 40 | Wait 10 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 41-48 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 49 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 50 | Check: Does the UE send a UEInformationResponse message including logMeasReport-r10 with serving cell measurements for Cell 1 and Cell 2? Note: The number of entries in logMeasInfoList should not be more than the expected logged measurement result entries within 30 seconds of logging periods. | --> | UEInformationResponse | 5,6, 7 | P |
| Note 1: It is checked that UE does not perform the logging while not in ‘camped normally’ state on an E-UTRA cell. |  |  |  |  |  |

8.6.2.8.3.3	Specific message contents

Table 8.6.2.8.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA | 7 | seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.8.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.8.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingDuration-r10 |  | min10 | 10 minutes |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.8.3.3-3: RRCConnectionSetupComplete (step 15, Table 8.6.2.8.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.8.3.3-4: RRCConnectionSetupComplete (step 28, Table 8.6.2.8.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.8.3.3-5: UEInformationRequest (step 33 and 49, Table 8.6.2.8.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.8.3.3-6: UEInformationResponse (step 34, Table 8.6.2.8.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least 1 entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 2 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.8.3.3-7: UEInformationResponse (step 50, Table 8.6.2.8.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least 1 entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry. |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 2 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10{} |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.2.9	Logged MDT / Location information

8.6.2.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged measurements with detailed

location information available for E-UTRA and plmn-Identity stored in VarLogMeasReport

is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with locationCoordinates in the locationInfo in the logged measurement entries }

}

8.6.2.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.306, clause 4.3.13.2; TS 36.331, clause 5.6.8.2.

[TS 36.306, clause 4.3.13.2 (TP1)]

standaloneGNSS-Location

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

8.6.2.9.3	Test description

8.6.2.9.3.1	Pre-test conditions

System Simulator:

-	Two intra-frequency cells belonging to the same PLMN, but to different tracking areas: Cell 1, Cell 11

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of SIB4 set as defined in TS 36.508 [18] table 6.3.1.1-1.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.2.9.3.2	Test procedure sequence

Table 8.6.2.9.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.6.2.9.3.2-2.

Table 8.6.2.9.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.6.2.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.6.2.9.3.2-1. (Note 1) | - | - | - | - |
| 5 | The UE transmits an RRCConnectionRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) | --> | RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCConnectionSetupComplete message? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 8-10 | Steps 4 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 11-19 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 20 | The SS transmits a UEInformationRequest message on Cell 11. | <-- | UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least two entries with serving cell idle mode measurements and where in at least one of the entries the IE locationCoordinates-r10 is present. | --> | UEInformationResponse | 1 | P |
| Note 1: The change of power levels is to trigger a cell re-selection procedure to make sure that the UE is logging neighbouring cell measurements (logging interval=2.56 seconds) of Cell 11 while t-Reselection timer is running (7 seconds). |  |  |  |  |  |

8.6.2.9.3.3	Specific message contents

Table 8.6.2.9.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA |  | 7 | seconds |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.9.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.9.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.9.3.3-3: RRCConnectionSetupComplete (step 7, Table 8.6.2.9.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.9.3.4: UEInformationRequest (step 20, Table 8.6.2.9.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.9.3.3-5: UEInformationResponse (step 21, Table 8.6.2.9.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Not checked |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Not checked |  |  |
| tce-Id-r10 |  | Not checked |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] SEQUENCE { |  |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |  |
| ellipsoid-Point-r10 |  | Any allowed value |  |  |
| ellipsoidPointWithAltitude-r10 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| relativeTimeStamp-r10 [x] |  | Not checked |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] |  | Not checked |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same as Cell 11 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 11 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 11 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### 8.6.2.10	Logged MDT / Logging and reporting / Reporting at RRC connection establishment / PLMN list

8.6.2.10.1	Test Purpose (TP)

(1)

with { UE is in E-UTRA RRC_IDLE state, UE has logged measurements available and the RPLMN is included in plmn-IdentityList received in LoggedMeasurementConfiguration message }

ensure that {

when { UE performs an RRC Connection establishment procedure }

then { UE sends an RRCConnectionSetupComplete message with logMeasAvailable }

}

(2)

with { UE is in E-UTRA RRC_CONNECTED state, UE has logged measurements available and the RPLMN is included in plmn-IdentityList received in LoggedMeasurementConfiguration message }

ensure that {

when { UE receives UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with a logMeasReport }

}

8.6.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331, clauses 5.3.3.4, 5.6.5.3 and 5.6.8.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	The RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception, or is present in the plmn-IdentityList (see TS 36.331 [3]) if configured;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.3.3.4]

The UE shall:

...

1>	set the content of RRCConnectionSetupComplete message as follows:

...

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include logMeasAvailable;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

8.6.2.10.3	Test description

8.6.2.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.2.10.3.1-1.

Table 8.6.2.10.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.2.10.3.3-5

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.2.10.3.2	Test procedure sequence

Table 8.6.2.10.3.2-1 illustrates the downlink power levels to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.2.10.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | Only Cell 1 is available. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 12. |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.2.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message on Cell 1 to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 3 | The SS changes Cell 1 and Cell 12 levels according to the row "T1" in table 8.6.2.10.3.2-1. | - | - | - | - |
| 4 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on Cell 12. NOTE: The UE performs a TAU procedure. The TAU is accepted with PLMN1 listed as an Equivalent PLMN and the RRC connection is released. | - | - | - | - |
| 5 | Wait [5s] to allow UE to activate logging. | - | - | - | - |
| 6 | The SS transmits a Paging message on Cell 12. | <-- | Paging | - | - |
| 7 | The UE transmits an RRCConnectionRequest message on Cell 12. | --> | RRCConnectionRequest | - | - |
| 8 | The SS transmits an RRCConnectionSetup message on Cell 12. | <-- | RRCConnectionSetup | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionSetupComplete message with logMeasAvailable on Cell 12? | --> | RRCConnectionSetupComplete | 1 | P |
| 10-13 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 12. | - | - | - | - |
| 14 | The SS transmits a UEInformationRequest message on Cell 12. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message with LogMeasInfoList on Cell 12? | --> | UEInformationResponse | 2 | P |

8.6.2.10.3.3	Specific message contents

Table 8.6.2.10.3.3-0: Conditions for specific message contents
in Tables 8.6.2.10.3.3-4 and 8.6.2.10.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.2.10.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| loggingInterval-r10 | ms1280 | 1.28 seconds |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| PLMN-Identity [1] | PLMN2 |  |  |
| } |  |  |  |
| areaConfiguration-v1130 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.10.3.3-2: RRCConnectionSetupComplete (steps 4 and 9, Table 8.6.2.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | Not present or any allowed value |  |  |
| rlf-InfoAvailable-r10 | Not present or any allowed value |  |  |
| logMeasAvailable-r10 | true |  |  |
| rn-SubframeConfigReq-r10 | Not present or any allowed value |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.10.3.3-2A: Message TRACKING AREA UPDATE ACCEPT (step 4, Table 8.6.2.10.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.2.10.3.3-3: UEInformationRequest (step 14, Table 8.6.2.10.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A condition Logged MDT

Table 8.6.2.10.3.3-4: UEInformationResponse (step 15, Table 8.6.2.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-MNC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE { | One or more entries where at least one entry complies to entry with index ‘x’ below. |  |  |
| locationInfo-r10 [x] | Not present or any allowed value |  |  |
| relativeTimeStamp-r10 [x] | Any allowed value |  |  |
| servCellIdentity-r10 [x] SEQUENCE { |  |  |  |
| plmn-Identity [x] | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity [x] | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |
| rsrpResult-r10 [x] | (0..97) |  |  |
| rsrqResult-r10 [x] | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r10 SEQUENCE { |  |  |  |
| measResultListEUTRA-r10 SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9 [1] SEQUENCE { |  |  |  |
| physCellId [1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info [1] | Not present |  |  |
| measResult [1] SEQUENCE { |  |  |  |
| rsrpResult [1] | Not present or (0..97) |  |  |
| rsrqResult [1] | Not present or (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r10 | Not present |  |  |
| measResultListGERAN-r10 | Not present |  |  |
| measResultListCDMA2000-r10 | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.10.3.3-5: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.2.10.3.3-6: SystemInformationBlockType5 for Cell 12 (preamble and all steps, Table 8.6.2.10.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.2.11	Logged MDT / Logging and reporting / Reporting at intra LTE handover / PLMN list

8.6.2.11.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with plmn-IdentityList-r11 configured and UE has logged measurements available with the RPLMN included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo }

then { UE transmits an RRCConnectionReconfigurationComplete message with logMeasAvailable included }

}

(2)

with { UE has indicated the availability of logged measurements in RRCConnectionReconfigurationComplete message to network }

ensure that {

when { UE receives a UEInformationRequest message with logMeasReportReq present and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE transmits an UEInformationResponse message with logMeasReport included }

}

8.6.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.4 (TP1)]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

….

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFail-Report and if the RPLMN is equal to plmn-Identity stored in VarConnEstFail-Report:

3>	include connEstFailInfoAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.2.11.3	Test description

8.6.2.11.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 12.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.2.11.3.1-1.

Table 8.6.2.11.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1, 2 | PLMN1 |
| 12 | PLMN2 |

UE:

None

Preamble:

-	The UE is registered on PLMN2 (Cell 12) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN1 in the Equivalent PLMN list as described in Table 8.6.2.11.3.3-5

-	The UE is in state Generic RB Established (state 3) on Cell 12 according to [18].

8.6.2.11.3.2	Test procedure sequence

Table 8.6.2.11.3.2-1shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” ,“T2” and “T3” are applied at the points indicated in the Main behaviour description in Table 8.6.2.11.3.2-2.

Table 8.6.2.11.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 12 | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | "Off" | "Off" |  |
| T1 | RS EPRE | dBm/15kHz | -85 | -91 | "Off" |  |
| T2 | RS EPRE | dBm/15kHz | -85 | -79 | "Off" |  |
| T3 | RS EPRE (FDD) | dBm/15kHz | -85 | -91 | -79 |  |
|  | RS EPRE (TDD) | dBm/15kHz | -85 | -87 | -83 |  |

Table 8.6.2.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS changes Cell 1 and Cell 12 levels according to the row "T1" in table 8.6.2.11.3.2-1. | - | - | - | - |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 12. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals on Cell 12. | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 12 levels according to the row "T2" in table 8.6.2.11.3.2-1. | - | - | - | - |
| 5 | The generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 6 | Wait 30 seconds for UE performing the logging at regular time intervals on Cell 1 | - | - | - | - |
| 7-14 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 15 | The SS changes Cell 1 and Cell 2 parameters according to the row "T3" in table 8.6.2.11.3.2-1. | - | - | - | - |
| 16 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 with logMeasAvailable set true to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 18 | The SS sends a UEInformationRequest message with logMeasReportReq present. | <-- | UEInformationRequest |  |  |
| 19 | Check: Does the UE send a UEInformationReponse message with logMeasReport including logged measurements did on Cell 1 and Cell 12? | --> | UEInformationResponse | 2 | P |

8.6.2.11.3.3	Specific message contents

Table 8.6.2.11.3.3-0: Conditions for specific message contents
in Tables 8.6.2.11.3.3-4 and 8.6.2.11.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.2.11.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF { | 1 entry |  |  |
| PLMN-Identity [1] | PLMN1 |  |  |
| PLMN-Identity [2] | PLMN2 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.11.3.3-2: RRCConnectionReconfigurationComplete (step 17, Table 8.6.2.11.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.11.3.3-3: UEInformationRequest (step 18, Table 8.6.2.11.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.11.3.3-4: UEInformationResponse (step 19, Table 8.6.2.11.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as absoluteTimeInfo sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | SS records the relativeTimeStamp value for each entry. At least 2 entries where one entry complies to entry with index ‘x’ and one entry complies to entry with index ‘y’ below. |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Cell 12 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10[x] SEQUENCE { |  | Not checked |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF { |  |  |  |  |
| carrierFreq-r9[1] |  | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF { |  |  |  |  |
| physCellId |  | Cell 1 |  |  |
| cgi-Info |  | Not present |  |  |
| measResult SEQUENCE { |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |
| rsrqResult |  | (0..34) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| locationInfo-r10[y] |  | Not checked |  |  |
| relativeTimeStamp-r10 [y] |  | SS record the value |  |  |
| servCellIdentity-r10 [y] |  | Cell 1 |  |  |
| measResultServCell-r10 [y] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10[y] SEQUENCE { |  | Not checked |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF { |  |  |  |  |
| carrierFreq-r9[1] |  | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF { |  |  |  |  |
| physCellId |  | Cell 12 |  |  |
| cgi-Info |  | Not present |  |  |
| measResult SEQUENCE { |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |
| rsrqResult |  | (0..34) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.11.3.3-5: ATTACH ACCEPT for Cell 12 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.2.11.3.3-6: SystemInformationBlockType5 for Cell 12 (preamble and all steps, Table 8.6.2.11.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 2 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.2.12	Logged MDT / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list

8.6.2.12.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with plmn-IdentityList-r11 configured and UE has logged measurements available with the RPLMN included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { UE has initiated a re-establishment procedure and receives an RRCConnectionReestablishment message }

then { UE transmits an RRCConnectionReestablishmentComplete message with logMeasAvailable is true }

}

(2)

with { UE indicated the availability of logged measurements in RRCConnectionReestablishmentComplete message }

ensure that {

when { UE has successfully completed the re-establishment procedure and resume the existing radio bearer; and has received a UEInformationRequest message with logMeasReportReq present }

then { UE transmits a UEInformationResponse message including logMeasReport }

}

8.6.2.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.7.5 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.7.5 (TP1)]

The UE shall:

1>	stop timer T301;

…

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include the rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include the connEstFailInfoAvailable;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.2.12.3	Test description

8.6.2.12.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 12.

-	The power level of Cell 1 is the Serving Cell level and Cell 12 is the Non-suitable Off level defined in table 6.2.2.1-1 of TS 36.508 [18].

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.2.12.3.1-1.

Table 8.6.2.12.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.2.12.3.3-6.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.2.12.3.2	Test procedure sequence

Table 8.6.2.12.3.2-1 shows the cell configurations used during the test. The configurations marked "T0" and "T1" are applied at the points indicated in the Main behaviour description in Table 8.6.2.12.3.2-2.

Table 8.6.2.12.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 12 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | Off | -85 |

Table 8.6.2.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS changes Cell 1 and Cell 12 levels according to the row "T0" in table 8.6.2.12.3.2-1. | - | - | - | - |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 30 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4-12 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 13 | The SS changes Cell 1 and Cell 12 parameters according to the row "T1" in table 8.6.2.12.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 12 is suitable for camping. | - | - | - | - |
| 14 | The UE sends RRCConnectionReestablishmentRequest message on Cell 12. | --> | RRCConnectionReestablishmentRequest | - | - |
| 15 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 16 | Check: Does the UE transmit RRCConnectionReestablishmentComplete message with logMeasAvailable set to true? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 17 | The SS transmits an RRCConnectionReconfiguration message to resume the existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 18 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 18A | The UE transmits a Tracking Area Update Request message on Cell 12. | --> | NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 18B | The SS transmits a Tracking Area Update Accept. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | <-- | NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 18C | The UE transmits a Tracking Area Update Complete | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 19 | The SS sends a UEInformationRequest message with logMeasReportReq present. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE send a UEInformationResponse message including logMeasReport? | --> | UEInformationResponse | 2 | P |

8.6.2.12.3.3	Specific message contents

Table 8.6.2.12.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.2.12.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF { |  |  |  |
| PLMN-Identity [1] | PLMN1 |  |  |
| PLMN-Identity [2] | PLMN2 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.12.3.3-2: RRCConnectionReestablishmentComplete (step 16, Table 8.6.2.12.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-11 RRCConnectionReestablishmentComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| rlf-InfoAvailable-r9 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.12.3.3-3: RRCConnectionReconfiguration (step 17, Table 8.6.2.12.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.12.3.3-3A: Message TRACKING AREA UPDATE ACCEPT (step 18B, Table 8.6.2.12.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.2.12.3.3-4: UEInformationRequest (step 19, Table 8.6.2.12.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.2.12.3.3-5: UEInformationResponse (step 20, Table 8.6.2.12.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as absoluteTimeInfo sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | SS records the relativeTimeStamp value for each entry. At least1 entry where at least one entry complies to entry with index ‘x’ below |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10[x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF { |  |  |  |  |
| carrierFreq-r9[1] |  | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF { |  |  |  |  |
| physCellId |  | Cell 12 |  |  |
| cgi-Info |  | Not present |  |  |
| measResult SEQUENCE { |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |
| rsrqResult |  | (0..34) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| measResultListEUTRA-v1090 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Band > 64 |
| carrierFreq-v9e0[1] |  | Same EARFCN as Cell 12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.2.12.3.3-6: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.6.2.13	Void

#### 8.6.2.14	Void

#### 8.6.2.15	Void

### 8.6.3	Inter-RAT Logged MDT

#### 8.6.3.1	Logged MDT / UTRAN Inter-RAT measurement, logging and reporting

8.6.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured without areaConfiguration and the UE is able to detect a UTRA cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and UTRA neighbouring cell measurements }

}

(2)

with { UE in UTRA RRC_CONNECTED state and UE has logged UTRA neighbouring cell measurements available from earlier measurements performed while in E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { handover from UTRA to E-UTRA cell successfully and T330 is running}

then { UE includes the logMeasAvailable IE in the RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged UTRA neighbouring cell measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with UTRA neighbouring cell measurements }

}

8.6.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.4.2.3, 5.6.5.3, 5.6.6.3, 5.6.8.2, 6.2.2.

[TS 36.304, clause 8 (TP1,TP2)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.4.2.3 (TP2)]

The UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include logMeasAvailable;

- submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

…

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.5.3 (TP3)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.3 (TP1)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 3:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.2.2 (TP1)]

-	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

Table 8.6.3.1.2-1: LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.3.1.3	Test description

8.6.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 5 - Cell 1 is E-UTRAN cell, Cell 5 is a UTRA cell.

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of UTRA carrier frequency list in SIB6 set as defined in TS 36.508 [18] table 6.3.1.3-1.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.3.1.3.2	Test procedure sequence

Table 8.6.3.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 8.6.3.1.3.2-2.

Table 8.6.3.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -97 | - |  |
|  | CPICH_Ec(UTRA FDD) | dBm/3.84 MHz | - | -60 | S nonServingCell, Cell1 > Thresh Cell1,high |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH_Ec(UTRA FDD) | dBm/3.84 MHz | - | -100 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -100 |  |

Table 8.6.3.1.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. |  | <-- | LoggedMeasurementConfiguration | - | - |  |
| 2 | The SS transmits an RRCConnectionRelease message. |  | <-- | RRCConnectionRelease | - | - |  |
| 3 | Wait 5s to allow UE to activate logging |  | - | - | - | - |  |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 8.6.3.1.3.2-1. (Note 1) |  | - | - | - | - |  |
| 5 | The UE transmits an RRC CONNECTION REQUEST message on Cell 5 to initiate a routing area update procedure. (Note 1) |  | --> | RRC CONNECTION REQUEST | - | - |  |
| 6 | The SS transmit an RRC CONNECTION SETUP message. |  | <-- | RRC CONNECTION SETUP | - | - |  |
| 7 | The UE transmit an RRC CONNECTION SETUP COMPLETE message. |  | --> | RRC CONNECTION SETUP COMPLETE | - | - |  |
| 8-14 | Steps 4 to 10 of the generic test procedure in TS 36.508 subclause 6.4.2.8 are performed on Cell 5. NOTE: The UE performs a RAU procedure and the RRC connection is released. |  | - | - | - | - |  |
| - | Exception: Steps 14AAa1 to 14AAa1b1 describe optional behaviour that depend on the UE capability. |  | - | - | - | - |  |
| 14AAa1 | IF pc_IMS = TRUE THEN SS starts 6s timer |  | - | - | - | - |  |
| 14AAa1a1-14AAa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed |  | - | - | - | - |  |
| 14AAa1a4 | SS releases RRC Connection |  | - | - | - | - |  |
| 14AAa1a5 | Stop 6s timer |  | - | - | - | - |  |
| 14AAa1b1 | The 6s timer expires |  | - | - | - | - |  |
| 14A | The SS waits for 5 s to ensure the UE is in service |  | - | - | - | - |
| 15-19 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. |  | - | - | - | - |  |
| - | For UTRAN FDD, EXCEPTION: Steps 20a1 to 20a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto step 21. |  | - | - | - | - |  |
| 20a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. |  | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |  |
| 20a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. |  | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |  |
| 21 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. |  | <-- | MEASUREMENT CONTROL | - | - |  |
| 22 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.6.3.1.3.2-1. |  | - | - | - | - |  |
| 23 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. |  | --> | MEASUREMENT REPORT | - | - |  |
| 24 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. |  | <-- | HANDOVER FROM UTRAN COMMAND | - | - |  |
| 25 | Check: Does the UE include the IE logMeasAvailable in the RRCConnectionReconfigurationComplete message on Cell 1? |  | --> | RRCConnectionReconfigurationComplete | 2 | P |  |
| 25A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - |  | - | - | - |  |
|  | EXCEPTION: Steps 25Ba1 to 25Ba2b1 describe optional behaviour that depend on the UE capability | - |  | - | - | - |  |
| 25Ba1-25Ba2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed |  |  |  |  |  |  |
| 26 | The SS transmits a UEInformationRequest message on Cell 1. | <-- |  | UEInformationRequest | - | - |  |
| 27 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least one inter-RAT neighbouring cell measurement of Cell 5. | --> |  | UEInformationResponse | 1, 3 | P |  |
| Note 1: The change of power levels is to trigger an inter-RAT cell re-selection procedure to make sure that the UE is logging inter-RAT neighbouring cell measurements (logging interval=2.56 seconds) of Cell 5 while t-ReselectionUTRA timer is running (7 seconds). |  |  |  |  |  |  |  |

8.6.3.1.3.3	Specific message contents

Table 8.6.3.1.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, 8.6.3.1.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| t-ReselectionUTRA | 7 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.3.1.3.3-2: System Information Block type 19 for Cell 5 (preamble, Table 8.6.3.1.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.1.3.3-3: LoggedMeasurementConfiguration (step 1, Table 8.6.3.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.1.3.3-4: RRCConnectionReconfigurationComplete (step 25, Table 8.6.3.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| nonCritriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.1.3.3-5: UEInformationRequest (step 26, Table 8.6.3.1.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.1.3.3-6: UEInformationResponse (step 27, Table 8.6.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  | At least1 entry where at least one entry complies to entry with index ‘x’ below |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 |  | Not present |  |  |
| measResultListUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same as Cell 5 |  |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 5 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| utra-RSCP |  | (-5..91) |  |  |
| utra-EcN0 |  | (0..49) |  |  |
| additionalSI-Info-r9 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.3.2	Logged MDT / GERAN Inter-RAT measurement, logging and reporting

8.6.3.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured without areaConfiguration and the UE is able to detect a GERAN cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and GERAN neighbouring cell measurements

}

}

(2)

with { UE in GPRS Registered state with active packet data transfer in NC2 mode and UE has logged GERAN neighbouring cell measurements available from earlier measurements performed while in E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { UE handover from GERAN to E-UTRA cell successfully and T330 is running }

then { UE includes the logMeasAvailable IE in the RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged GERAN neighbouring cell measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with GERAN neighbouring cell measurements }

}

8.6.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.4.2.3, 5.6.5.3, 5.6.6.3, 5.6.8.2, 6.2.2.

[TS 36.304, clause 8 (TP1)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.4.2.3 (TP2)]

The UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include logMeasAvailable;

- submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

…

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.5.3 (TP3)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.3 (TP1)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 3:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.2.2 (TP1)]

-	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.3.2.3	Test description

8.6.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 24 - Cell 1 is E-UTRAN cell, Cell 24 is a GERAN cell.

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of GERAN carrier frequency group list in SIB7 set as defined in TS 36.508 [18] table 6.3.1.4-1.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.3.2.3.2	Test procedure sequence

Table 8.6.3.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently.

Table 8.6.3.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -80 | - |  |
|  | RSSI | dBm | - | [-85] |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -100 | - |  |
|  | RSSI | dBm | - | [-60] |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -80 | - |  |
|  | RSSI | dBm | - | [-85] |  |

Table 8.6.3.2.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. |  | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. |  | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging |  | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 24 levels according to the row "T1" in table 8.6.3.2.3.2-1. (Note 1) |  | - | - | - | - |
| 5 | Generic test procedure in TS 36.508 Table 6.4.2.9 is performed and the UE will camp on GERAN Cell 24. |  |  |  |  |  |
| 6 | UE is brought into downlink packet transfer mode according to TS 51.010 clause 40.4.3.14 |  |  |  |  |  |
| 7 | The SS changes Cell 1 and Cell 24 levels according to the row "T2" in table 8.6.3.2.3.2-1. |  | - | - | - | - |
| 8 | The SS transmits PS HANDOVER COMMAND on Cell24 |  | <-- | PS HANDOVER COMMAND | - | - |
| 9 | Check: Does the UE include the IE logMeasAvailable in the RRCConnectionReconfigurationComplete message on Cell 1? |  | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 10 | The SS transmits a UEInformationRequest message on Cell 1. | <-- |  | UEInformationRequest | - | - |
| 11 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least one inter-RAT neighbouring cell measurement of Cell 24. | --> |  | UEInformationResponse | 1,3 | P |
| Note 1: The change of power levels is to trigger an inter-RAT cell re-selection procedure to make sure that the UE is logging GERAN neighbouring cell measurements (logging interval=2.56 seconds) of Cell 5 while t-ReselectionGERAN timer is running (7 seconds). |  |  |  |  |  |  |

8.6.3.2.3.3	Specific message contents

Table 8.6.3.2.3.3-1: SystemInformationBlockType7 for cell 1 (preamble and all steps, 
Table 8.6.3.2.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 |  |  |
| carrierFreqsInfoListcarrierFreqsInfoListSEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqs carrierFreqs[n] SEQUENCE { |  |  |  |
| startingARFCN[n] | Same starting ARFCN used for cell 24 |  |  |
| bandIndicator[n] | same band used for GERAN cell24 |  |  |
| followingARFCNs[n] CHOICE { |  |  |  |
| explicitListOfARFCNs[n] | Same ARFCN used for cell24 |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo[n] SEQUENCE { |  |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.2.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.3.2.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.2.3.3-3: RRCConnectionReconfigurationComplete (step 9, Table 8.6.3.2.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| nonCritriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | TRUE |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.2.3.3-4: UEInformationRequest (step 10, Table 8.6.3.2.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.2.3.3-5: UEInformationResponse (step 11, Table 8.6.3.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRAN-r10 |  | Not present |  |  |
| measResultListUTRAN-r10 |  | Not present |  |  |
| measResultListGERAN-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same as Cell 24 |  |  |
| physCellId [1] |  | Same as Cell 24 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| rssi |  | (0..63) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.3.3	Logged MDT / CDMA2000 Inter-RAT measurement, logging and reporting

8.6.3.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured without areaConfiguration and the UE is able to detect a CDMA2000 cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and CDMA2000 neighbouring cell measurements }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has logged CDMA2000 neighbouring cell measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with Inter-RAT neighbouring cell measurements }

}

8.6.3.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.6.6.3, 5.6.8.2, 5.6.5.3 and 6.2.2.

[TS 36.304, clause 8 (TP1)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and the plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 5.6.6.3 (TP1)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 3:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.2.2 (TP1)]

-	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

Table 8.6.3.3.2-1: LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

And the procedure ends.

8.6.3.3.3	Test description

8.6.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 15

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of CDMA2000 HRPD carrier frequency list in SIB8 set as defined in TS 36.508 [18] table 6.3.1.5-1.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.3.3.3.2	Test procedure sequence

Table 8.6.3.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.6.3.3.3.2-2.

Table 8.6.3.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that entering conditions for event B2 are not satisfied: Inequality B2-1 (Entering condition 1) Ms Hys > Thresh1 Inequality B2-2 (Entering condition 2) Mn Ofn Hys Thresh2 |
|  | Îor/Ioc | dB | - | -20 |  |
|  | Ioc | dBm/1.23MHz | - | -55 |  |
|  | Pilot_Ec/Io (Note 1) | dB | - | -20 |  |
|  | Îor/Ioc | dB | - | -5 |  |
|  | Ioc | dBm/1.23MHz | - | -55 |  |
|  | Pilot_Ec/Io (Note 1) | dB | - | -6 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS |  |  |  |  |  |

Table 8.6.3.3.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message including to configure the UE how to perform logging of measurement results while in RRC_IDLE. |  | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. |  | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging |  | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 15 levels according to the row "T1" in table 8.6.3.3.3.2-1. |  | - | - | - | - |
| 5 | The SS transmits a RRC CONNECTION SETUP message. |  | <-- | RRC CONNECTION SETUP | - | - |
| 6 | The UE include the IE “Logged Meas Available” in the RRC CONNECTION SETUP COMPLETE message. |  | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 7 | The SS transmits a UEInformationRequest message on Cell 1. | <-- |  | UEInformationRequest | - | - |
| 8 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least one inter-RAT neighbouring cell measurement of Cell 1. | --> |  | UEInformationResponse | 1,2 | P |

8.6.3.3.3.3	Specific message contents

Table 8.6.3.3.3.3-3: LoggedMeasurementConfiguration (step 1, Table 8.6.3.3.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.3.3.3-4: UEInformationRequest (step 7, Table 8.6.3.3.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.3.3.3-5: UEInformationResponse (step 8, Table 8.6.3.3.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 |  | Not present |  |  |
| measResultListUTRA-r10 |  | Not present |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry if available |  |  |
| physCellId[1] |  | PhysicalCellIdentity of Cell 15 if available |  |  |
| cgi-Info[1] CHOICE { |  |  |  |  |
| cellGlobalIdHRPD |  | cellGlobalId of Cell 15 if available |  |  |
| } |  |  |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| pilotStrength |  | (0..63) if available |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physCellId |  | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I |  | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |  |
| reestablishmentCause |  | otherFailure |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.3.4	Logged MDT / Logging and reporting / Reporting at UTRAN Inter-RAT handover / PLMN list

8.6.3.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state camping normally on an E-UTRA cell where logged measurement is configured with the RPLMN included in plmn-IdentityList and the UE is able to detect a UTRA cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and UTRA neighbouring cell measurements }

}

(2)

with { UE in UTRA RRC_CONNECTED state and UE has logged E-UTRA measurements and UTRA neighbouring cell measurements available from earlier measurements performed while in E-UTRA }

ensure that {

when { UE handovers from UTRA to E-UTRA cell successfully and T330 is running and the current RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCConnectionReconfigurationComplete message with logMeasAvailable IE set to true }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and has logged measurements available for E-UTRA and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits UEInformationResponse messages with logMeasReport included }

}

8.6.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 34.304, clause 8; TS 36.331, clauses 5.4.2.3, 5.6.5.3, 5.6.6.3, 5.6.8.2, 6.2.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304, clause 8 (TP1, TP2)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception, or is present in the plmn-IdentityList (see TS 36.331 [3]) if configured;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.4.2.3 (TP2)]

The UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

…

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.6.6.3 (TP1)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 5.6.5.3 (TP3)]

Upon receiving the UEInformationRequest message, the UE shall

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[TS 36.331, clause 6.2.2 (TP1)]

-	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			LoggedMeasurementConfiguration-v1080-IEs	OPTIONAL	-- Need OP

}

LoggedMeasurementConfiguration-v1080-IEs ::= SEQUENCE {

lateNonCriticalExtension-r10	OCTET STRING						OPTIONAL,	-- Need OP

nonCriticalExtension			LoggedMeasurementConfiguration-v1130-IEs	OPTIONAL	-- Need OP

}

LoggedMeasurementConfiguration-v1130-IEs ::= SEQUENCE {

plmn-IdentityList-r11			PLMN-IdentityList3-r11		OPTIONAL,	-- Need OR

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,	-- Need OR

nonCriticalExtension			SEQUENCE {}					OPTIONAL	-- Need OP

}

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ identities.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

8.6.3.4.3	Test description

8.6.3.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 9 and Cell 12 - Cell 1 and Cell 12 are E-UTRAN cell, Cell 9 is a UTRA cell.

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of UTRA carrier frequency list in SIB6 set as defined in TS 36.508 [18] table 6.3.1.3-1.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.3.4.3.1-1.

Table 8.6.3.4.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

Preamble:

- The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.3.4.3.3-7

- The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.6.3.4.3.2	Test procedure sequence

Table 8.6.3.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 8.6.3.4.3.2-2.

Table 8.6.3.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Cell 9 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | - | SnonServingCell, Cell9 > Thresh Cell9,high |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -65 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -67 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -100 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -100 |  |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.3.4.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in E-UTRA RRC_IDLE. |  | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. |  | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging |  | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 9 levels according to the row "T1" in table 8.6.3.4.3.2-1. (Note 1) |  | - | - | - | - |
| 5 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 9. NOTE: The UE performs an RAU procedure and the RRC connection is released. |  | - | - | - | - |
| - | Exception: Steps 5Aa1 to 5Aa1b1 describe optional behaviour that depend on the UE capability. |  | - | - | - | - |
| 5Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer |  | - | - | - | - |
| 5Aa1a1-5Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed |  | - | - | - | - |
| 5Aa1a4 | SS releases RRC Connection |  | - | - | - | - |
| 5Aa1a5 | Stop 6s timer |  | - | - | - | - |
| 5Aa1b1 | The 6s timer expires |  | - | - | - | - |
| 6-10 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 9 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. |  | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 11a1 to 11a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto step 12. |  | - | - | - | - |
| 11a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 9 including the DPCH compressed mode info. |  | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 11a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 9. |  | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 12 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 9. |  | <-- | MEASUREMENT CONTROL | - | - |
| 13 | The SS changes Cell 12 and Cell 9 levels according to the row "T2" in table 8.6.3.4.3.2-1. |  | - | - | - | - |
| 14 | The UE transmits a MEASUREMENT REPORT message on Cell 9 including the E-UTRA event results. |  | --> | MEASUREMENT REPORT | - | - |
| 15 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 9. |  | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with the IE logMeasAvailable set to true on Cell 12? |  | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 17 | Void |  | - | - | - | - |
| 17A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 12. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN |  | - | - | - | - |
|  | EXCEPTION: Steps 17Ba1 to 17Ba2b1 describe optional behaviour that depend on the UE capability |  | - | - | - | - |
| 17Ba1-17Ba2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed |  | - | - | - | - |
| 18 | Void |  |  |  |  |  |
| 19 | Void |  |  |  |  |  |
| 20 | The SS transmits a UEInformationRequest message on Cell 12 with logMeasReportReq present. | <-- |  | UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with logMeasReport included? | --> |  | UEInformationResponse | 1, 3 | P |
| Note 1: The change of power levels is to trigger an inter-RAT cell re-selection procedure to make sure that the UE is logging inter-RAT neighbouring cell measurements (logging interval=2.56 seconds) of Cell 9 while t-ReselectionUTRA timer is running (7 seconds). |  |  |  |  |  |  |

8.6.3.4.3.3	Specific message contents

Table 8.6.3.4.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, 8.6.3.4.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Downlink UARFCN of Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Downlink UARFCN of Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| t-ReselectionUTRA | 7 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.3.4.3.3-2: System Information Block type 19 for Cell 9 (preamble, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.4.3.3-3: LoggedMeasurementConfiguration (step 1, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |
| loggingInterval-r10 |  |  |  |
| nonCriticalExtension SEQUENCE { | ms2560 | 2.56 seconds |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| plmn-IdentityList-r11 SEQUENCE (SIZE (1..16)) OF { |  |  |  |
| PLMN-Identity [1] | PLMN1 |  |  |
| PLMN-Identity [2] | PLMN2 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.3.4.3.3-4: RRCConnectionReconfigurationComplete (step 16, Table 8.6.3.4.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-9 RRCConnectionReconfigurationComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| nonCritriticalExtension SEQUENCE { |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.4.3.3-5: UEInformationRequest (step 20, Table 8.6.3.4.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.4.3.3-6: UEInformationResponse (step 21, Table 8.6.3.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 [x] SEQUENCE { |  |  |  |  |
| measResultListEUTRA-r10 |  | Not present |  |  |
| measResultListUTRA-r10 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r9[1] |  | Same as Cell 9 |  |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId [1] |  | Same as Cell 9 |  |  |
| cgi-Info [1] |  | Not checked |  |  |
| measResult [1] SEQUENCE { |  |  |  |  |
| utra-RSCP |  | (-5..91) |  |  |
| utra-EcN0 |  | (0..49) |  |  |
| additionalSI-Info-r9 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListGERAN-r10 |  | Not present |  |  |
| measResultListCDMA2000-r10 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.4.3.3-7: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.3.4.3.3-8: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 17A, Table 8.6.3.4.3.2-2) (step 2 in TS 36.508 subclause 6.4.2.10)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  | Cell 28 |

#### 8.6.3.5	Logged MDT / Logging and reporting / Bluetooth measurement collection

8.6.3.5.1	Test Purpose (TP)

(1)

with { UE received RRCConnectionSetup message }

ensure that {

when { UE has Bluetooth logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableBT in the RRCConnectionSetupComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasRepor }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 including logMeasResultListBT }

}

(3)

with { UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a Bluetooth beacon is not included in bt-NameList }

ensure that {

when { UE in E-UTRA RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 not including the measurement result of the Bluetooth beacon not included in bt-NameList }

}

8.6.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331 clauses 5.3.3.4, 5.6.5.3, 5.6.6.3, 5.6.8.2.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.6.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	store the received targetMBSFN-AreaList, if included, in VarLogMeasConfig;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

…

1>	if not suspended, perform the logging in accordance with the following:

…

2>	else if:

3>	if the UE is in any cell selection state (as specified in TS 36.304 [4]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

3>	else if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

…

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	if wlan-NameList is included in VarLogMeasConfig:

4>	if detailed WLAN measurements are available:

5>	include logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs;

3>	if bt-NameList is included in VarLogMeasConfig:

4>	if detailed Bluetooth measurements are available:

5>	include logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons;

…

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

4>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

…

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

1>	the procedure ends:

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.3.5.3	Test description

8.6.3.5.3.1	Pre-test conditions

System Simulator:

- Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 36.508 cl 4.4.9

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.3.5.3.2	Test procedure sequence

Table 8.6.3.5.3.2-1 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.6.3.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.6.3.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionSetupComplete message with logMeasAvailableBT on Cell 1? | --> | RRCConnectionSetupComplete | 1 | P |
| 8-11 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r10 with one entry (Bluetooth beacon 1) measurement result in the IE logMeasResultListBT. | --> | UEInformationResponse | 2,3 | P |

8.6.3.5.3.3	Specific message contents

Table 8.6.3.5.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.3.5.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension-r10 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| plmn-IdentityList-r11 |  | Not present |  |  |
| areaConfiguration-v1130 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| targetMBSFN-AreaList-r12 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| bt-NameList-r15 SEQUENCE { SIZE (1..maxBT-Name-r15)) OF |  | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r15[1] |  | Set as per Table 4.4.9-1 of [18] | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |  |
| wlan-NameList-r15 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.3.5.3.3-2: RRCConnectionSetupComplete (step 7, Table 8.6.3.5.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailableBT-r15 |  | TRUE |  |  |  |  |  |
| logMeasAvailableWLAN-r15 |  | Not checked |  |  |  |  |  |
| idleMeasAvailable-r15 |  | Not checked |  |  |  |  |  |
| flightPathInfoAvailable-r15 |  | Not checked |  |  |  |  |  |
| connectTo5GC-r15 |  | Not checked |  |  |  |  |  |
| registeredAMF-r15 |  | Not checked |  |  |  |  |  |
| s-NSSAI-list-r15 SEQUENCE {} |  | Not checked |  |  |  |  |  |
| ng-5G-S-TMSI-Bits-r15 CHOICE {} |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.5.3.3-3: UEInformationRequest (step 12, Table 8.6.3.5.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.5.3.3-4: UEInformationResponse (step 13, Table 8.6.3.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Not checked |  |  |
| traceReference-r10 |  | Not checked |  |  |
| traceRecordingSessionRef-r10 |  | Not checked |  |  |
| tce-Id-r10 |  | Not checked |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10 |  | Not checked |  |  |
| relativeTimeStamp-r10 |  | Not checked |  |  |
| servCellIdentity-r10 |  | Not checked |  |  |
| measResultServCell-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultNeighCells-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultListEUTRA-v1090 |  | Not checked |  |  |
| measResultListMBSFN-r12 |  | Not checked |  |  |
| measResultServCell-v1250 |  | Not checked |  |  |
| servCellRSRQ-Type-r12 |  | Not checked |  |  |
| measResultListEUTRA-v1250 |  | Not checked |  |  |
| inDeviceCoexDetected-r13 |  | Not checked |  |  |
| measResultServCell-v1360 |  | Not checked |  |  |
| logMeasResultListBT-r15 SEQUENCE (SIZE (1..maxBT-IdReport-r15)) OF SEQUENCE { |  | 1 entry | Report Bluetooth beacon 1 |  |
| bt-Addr-r15 |  | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r15 |  | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |  |
| logMeasResultListWLAN-r15 |  | Not checked |  |  |
| anyCellSelectionDetected-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.3.6	Logged MDT / Logging and reporting / WLAN measurement collection

8.6.3.6.1	Test Purpose (TP)

(1)

with { UE received RRCConnectionSetup message }

ensure that {

when { UE has WLAN logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableWLAN in the RRCConnectionSetupComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 including logMeasResultListWLAN }

}

(3)

with { UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a WLAN node is not included in wlan-NameList }

ensure that {

when { UE in E-UTRA RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 not including the measurement result of the WLAN node not included in wlan-NameList }

}

8.6.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331 clauses 5.3.3.4, 5.6.5.3, 5.6.6.3, 5.6.8.2.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.6.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	store the received targetMBSFN-AreaList, if included, in VarLogMeasConfig;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

…

1>	if not suspended, perform the logging in accordance with the following:

…

2>	else if:

3>	if the UE is in any cell selection state (as specified in TS 36.304 [4]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

3>	else if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

…

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	if wlan-NameList is included in VarLogMeasConfig:

4>	if detailed WLAN measurements are available:

5>	include logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs;

3>	if bt-NameList is included in VarLogMeasConfig:

4>	if detailed Bluetooth measurements are available:

5>	include logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons;

…

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

4>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

…

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

1>	the procedure ends

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.3.6.3	Test description

8.6.3.6.3.1	Pre-test conditions

System Simulator:

- Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell27b are configured as per 36.508 cl 4.4.8 with the OFFLOAD condition

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.3.6.3.2	Test procedure sequence

Table 8.6.3.6.3.2-1 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.6.3.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.6.3.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionSetupComplete message with logMeasAvailableWLAN on Cell 1? | --> | RRCConnectionSetupComplete | 1 | P |
| 8-11 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r10 with one entry (WLAN AP 1) measurement result in the IE logMeasResultListWLAN. | --> | UEInformationResponse | 2,3 | P |

8.6.3.6.3.3	Specific message contents

Table 8.6.3.6.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.3.6.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension-r10 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| plmn-IdentityList-r11 |  | Not present |  |  |
| areaConfiguration-v1130 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| targetMBSFN-AreaList-r12 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| bt-NameList-r15 |  | Not present |  |  |
| wlan-NameList-r15 SEQUENCE { |  | 1 entry | WLAN AP 1 |  |
| WLAN-Name-r15[1] |  | Set as per Table 4.4.8-1 of [18] | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

-	The specific message content for the SystemInformationBlockType17 message in Table 8.6.3.6.3.3-1A.

Table 8.6.3.6.3.3-1A: SystemInformationBlockType17 (Preamble and all steps of Table 8.6.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 2 Entry |  |
| wlan-Identifiers[1] SEQUENCE{ |  |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[1] | Not present |  |  |
| hessid-r12[1] | Not present |  |  |
| } |  |  |  |
| wlan-Identifiers[2] SEQUENCE{ |  |  |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[2] | Not present |  |  |
| hessid-r12[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.6.3.6.3.3-2: RRCConnectionSetupComplete (step 7, Table 8.6.3.6.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |  |  |  |
| logMeasAvailableWLAN-r15 |  | TRUE |  |  |  |  |  |
| idleMeasAvailable-r15 |  | Not checked |  |  |  |  |  |
| flightPathInfoAvailable-r15 |  | Not checked |  |  |  |  |  |
| connectTo5GC-r15 |  | Not checked |  |  |  |  |  |
| registeredAMF-r15 |  | Not checked |  |  |  |  |  |
| s-NSSAI-list-r15 SEQUENCE {} |  | Not checked |  |  |  |  |  |
| ng-5G-S-TMSI-Bits-r15 CHOICE {} |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.6.3.3-3: UEInformationRequest (step 12, Table 8.6.3.6.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.6.3.3-4: UEInformationResponse (step 13, Table 8.6.3.6.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Not checked |  |  |
| traceReference-r10 |  | Not checked |  |  |
| traceRecordingSessionRef-r10 |  | Not checked |  |  |
| tce-Id-r10 |  | Not checked |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10 |  | Not checked |  |  |
| relativeTimeStamp-r10 |  | Not checked |  |  |
| servCellIdentity-r10 |  | Not checked |  |  |
| measResultServCell-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultNeighCells-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultListEUTRA-v1090 |  | Not checked |  |  |
| measResultListMBSFN-r12 |  | Not checked |  |  |
| measResultServCell-v1250 |  | Not checked |  |  |
| servCellRSRQ-Type-r12 |  | Not checked |  |  |
| measResultListEUTRA-v1250 |  | Not checked |  |  |
| inDeviceCoexDetected-r13 |  | Not checked |  |  |
| measResultServCell-v1360 |  | Not checked |  |  |
| logMeasResultListBT-r15 |  | Not checked |  |  |
| logMeasResultListWLAN-r15 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF SEQUENCE { |  | 1 entry | Report WLAN AP1 |  |
| wlan-Identifiers-r15[1] SEQUENCE { |  |  |  |  |
| ssid-r12 |  | ssid for WLAN AP 1 |  |  |
| bssid-r12 |  | Not present |  |  |
| hessid-r12 |  | Not present |  |  |
| } |  |  |  |  |
| rssiWLAN-r15[1] |  | Any allowed value |  |  |
| rtt-WLAN-r15[1] |  | Any allowed value |  |  |
| } |  |  |  |  |
| anyCellSelectionDetected-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |
| logMeasAvailableWLAN-r15 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 8.6.4	Logged Radio Link Failure

#### 8.6.4.1	Radio Link Failure logging / Reporting of Intra-frequency measurements

8.6.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell }

}

8.6.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.7.5, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.1.3	Test description

8.6.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.1.3.2	Test procedure sequence

Table 8.6.4.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | Only Cell 2 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 2 parameters according to the row "T1" in Table 8.6.4.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 parameter according to the row "T2" in Table 8.6.4.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.6.4.1.3.3	Specific message contents

Table 8.6.4.1.3.3-0: Conditions for specific message contents
in Tables 8.6.4.1.3.3-2 and 8.6.4.1.3.3-8

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.4.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.1.3.3-2: MeasConfig (Table 8.6.4.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObect[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-3: MeasurementReport (step 4, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPcell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-4: RRCConnectionReestablishmentRequest (step 6, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-5: RRCConnectionReestablishmentComplete (step 8, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-6: RRCConnectionReconfiguration (step 9, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-7: UEInformationRequest (step 11, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.1.3.3-8: UEInformationResponse (step 12, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | Physical cell Identity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |  |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.4.2	Radio Link Failure logging / Reporting of Inter-frequency measurements

8.6.4.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the inter-frequency measurement and reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for inter-frequency neighbour cell }

}

8.6.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.7.5, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.2.3	Test description

8.6.4.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.2.3.2	Test procedure sequence

Table 8.6.4.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -73 | Only Cell 3 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 3 parameters according to the row "T1" in Table 8.6.4.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 parameter according to the row "T2" in Table 8.6.4.2.3.2-1. | - | - | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 3. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 3. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 3. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message on Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS transmits a UEInformationRequest message on Cell 3. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit a UEInformationResponse message on Cell 3? | --> | UEInformationResponse | 1 | P |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 3? | - | - | 1 | - |

8.6.4.2.3.3	Specific message contents

Table 8.6.4.2.3.3-0: Conditions for specific message contents
in Tables 8.6.4.2.3.3-2 and 8.6.4.2.3.3-8

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.4.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.2.3.3-2: MeasConfig (Table 8.6.4.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-3: MeasurementReport (step 4, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPcell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-4: RRCConnectionReestablishmentRequest (step 6, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-5: RRCConnectionReestablishmentComplete (step 8, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-6: RRCConnectionReconfiguration (step 9, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-7: UEInformationRequest (step 11, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.2.3.3-8: UEInformationResponse (step 12, Table 8.6.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.4.3	Radio Link Failure logging / Reporting at RRC connection establishment and reestablishment

8.6.4.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { T301 expires after UE having sent an RRCConnectionReestablishmentRequest message, the UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCConnectionSetupComplete message with rlf-InfoAvailable included when UE performs TAU procedure }

}

(2)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCConnectionReestablishmentComplete message with rlf-InfoAvailable included

}

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {
  when { UE receives the UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

(4)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with eestablishmentCellId set to the global cell identity of the selected cell }

}

(5)

with { UE in RRC_CONNECTED state with successful delivery of the UEInformationResponse message confirmed by lower layer }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message without rlf-Report included }

}

8.6.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.7.5, 5.3.11.3, 5.3.7.4 and 5.6.5.3.

[TS 36.331, clause 5.3.3.4 (TP1)]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

…

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.7.5 (TP2)]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.11.3 (TP1, TP2)]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.3.7.4 (TP4)]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.6.5.3 (TP4,TP5)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.3.3	Test description

8.6.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 6

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.3.3.2	Test procedure sequence

Table 8.6.4.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | "Off" |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -85 |  |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.4.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.6.4.3.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 3 is suitable for camping. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 3. | --> | RRCConnectionReestablishmentRequest | - | - |
| 3 | The SS does not respond to any RRCConnectionReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to RRC_IDLE state on Cell 3. |  |  |  |  |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 3 | --> | RRC: RRCConnectionRequest | - | - |
| 5 | The SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment with rlf-InfoAvailable included? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 6A | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand | - | - |
| 6B | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete | - | - |
| 7 | The SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 8 | The UE sends a TRACKING AREA UPDATE COMPLETE on Cell 3 to finish the TAU procedure | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 9 | The SS changes Cell 3 and Cell 6 parameters according to the row "T2" in Table 8.6.4.3.3.2-1 in order that the radio link quality of Cell 3 is degraded and Cell 6 is suitable for camping. | - | - | - | - |
| 10 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 6. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message on Cell 6. | <-- | RRCConnectionReestablishment | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message on Cell 6 with rlf-InfoAvailable included? | --> | RRCConnectionReestablishmentComplete | 2 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 6. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 6. | --> | RRCConnectionReconfigurationComplete | - | - |
| 15 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 6. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 6 with rlf-Report included? Check: Does the reestablishmentCellId included in the UEInformationResponse message equal to the global cell identity of Cell 6? | --> | UEInformationResponse | 3, 4 | P |
| 17 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 6. | <-- | UEInformationRequest | - | - |
| 18 | Check: Does the UE transmit a UEInformationResponse message on Cell 6 without rlf-Report included? | --> | UEInformationResponse | 5 | P |
| 19 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 6? | - | - | 1 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

8.6.4.3.3.3	Specific message contents

Table 8.6.4.3.3.3-1: RRCConnectionReestablishmentRequest (step 2, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.3.3.3-2: RRCConnectionSetupComplete (step 6, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r10 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.4.3.3.3-3: RRCConnectionReestablishmentRequest (step 10, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 3 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.3.3.3-4: RRCConnectionReestablishmentComplete (step 12, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.3.3.3-5: RRCConnectionReconfiguration (step 13, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.3.3.3-6: UEInformationRequest (steps 15 and 17, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.3.3.3-7: UEInformationResponse (step 16, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 3 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 6 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 6 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| previousPCellId-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.3.3.3-8: UEInformationResponse (step 18, Table 8.6.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.4.4	Radio Link Failure logging / Reporting at E-UTRA handover

8.6.4.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with T304 expiry and handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN for Rel-10 UE, or the RPLMN is included in plmn-IdentityList stored in VarRLF-Report for Rel-11 and onwards UE }

ensure that {

when { UE receives the RRCConnectionReestablishment message and completes the RRC Connection Reestablishment successfully }

then { UE sends the RRCConnectionReestablishmentComplete message including rlf-InfoAvailable }

(2)

with { UE in RRC_CONNECTED state with handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN for Rel-10 UE, or the RPLMN is included in plmn-IdentityList stored in VarRLF-Report for Rel-11 and onwards UE }

ensure that {

when { UE receives RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message }

then { UE sends the RRCConnectionReconfigurationComplete message with rlf-InfoAvailable included }

}

(3)

with { UE in RRC_CONNECTED state with the handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN for Rel-10 UE, or the RPLMN is included in plmn-IdentityList stored in VarRLF-Report for Rel-11 and onwards UE }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

(4)

with { UE in RRC_CONNECTED state with the handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN for Rel-10 UE, or the RPLMN is included in plmn-IdentityList stored in VarRLF-Report for Rel-11 and onwards UE }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message with eestablishmentCellId set to the global cell identity of the selected cell }

}

(5)

with { Rel-10 UE in RRC_CONNECTED state with the handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is not equal to the RPLMN }

ensure that {
  when { UE receives the UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message without rlf-Report included }

}

8.6.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.4, 5.3.5.6, 5.3.7.4 and 5.6.5.3. Unless otherwise stated these are Rel-10 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

[TS 36.331, clause 5.3.5.6]

[Rel-10]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[Rel-11]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

…

1>	set the reestablishmentCause as follows:

…

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.6.5.3]

[Rel-10]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

[Rel-11]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.4.3	Test description

8.6.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 4 and Cell 12

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 8.6.4.4.3.1-1.

Table 8.6.4.4.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1,4 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table  8.6.4.4.3.3-19

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.4.3.2	Test procedure sequence

Table 8.6.4.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1",  "T2",  "T3", "T4" and "T5" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy exit condition for event A3 (M12< M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | Off | The power level values are assigned to satisfy SrxlevCell 12 < 0 such that selecting Cell 1 is guaranteed (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | Off | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | Off | -85 | -73 | The power level values are such that measurement results for Cell 4 (M4) and Cell 12 (M12) satisfy entry condition for event A3 (M12> M4). (NOTE 1). |
| T5 | Cell-specific RS EPRE | dBm/15kHz | Off | Off | -73 | The power level values are assigned to satisfy SrxlevCell 4 < 0 such that selecting Cell 12 is guaranteed (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.4.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra and inter frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 12 parameter according to the row "T1" in table 8.6.4.4.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 12. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the event described in step 6 the steps specified in Table 8.6.4.4.3.2-3 should take place. | - | - | - | - |
| 6 | Wait for 1 s to ensure that T304 expires. The SS changes Cell 12 parameter according to the row "T2" in table 8.6.4.4.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1 | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable included? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 10 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 12 | The SS changes Cell 4 parameter according to the row "T3" in table 8.6.4.4.3.2-1. | - | - | - | - |
| 13 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 4. | --> | MeasurementReport | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 15 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4 with rlf-InfoAvailable included? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 16 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 4. | <-- | UEInformationRequest | - | - |
| 17 | Check: Does the UE transmit a UEInformationResponse message on Cell 4 with rlf-Report included? Check: Does the reestablishmentCellId included in the UEInformationResponse message equal to the global cell identity of Cell 1? | --> | UEInformationResponse | 3,4 | P |
| 18 | The SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 4 | <-- | RRCConnectionReconfiguration | - | - |
| 19 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 4. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 20 | The SS changes Cell 1, Cell 4 and Cell - 12 parameters according to the row "T4" in table 8.6.4.4.3.2-1. | - | - | - | - |
| 21 | The UE transmits a MeasurementReport message on Cell 4 to report event A3 with the measured RSRP, RSRQ value for Cell 12. | --> | MeasurementReport | - | - |
| 22 | The SS transmits an RRCConnectionReconfiguration message on Cell 4 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 23 the steps specified in Table 8.6.4.4.3.2-3 should take place. | - | - | - | - |
| 23 | The SS changes Cell 4 power level according to the row "T5" in table 8.6.4.4.3.2-1. | - | - | - | - |
| 24 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 12 | --> | RRCConnectionReestablishmentRequest | - | - |
| 25 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 12. | <-- | RRCConnectionReestablishment | - | - |
| 26 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable included? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 27 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 28 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 12. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 29 | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 12. | - | - | - | - |
| 30 | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 31 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| - | EXCEPTION: Steps 31Aa1 and 31Aa2 depend on the UE implementation. The lower letter indicates a branch that will take place depending on the UE release. | - | - | - | - |
| 31Aa1 | IF Rel-10 UE THEN the SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 12. | <-- | UEInformationRequest | - | - |
| 31Aa2 | Check: Does the UE transmit a UEInformationResponse message on Cell 12 without rlf-Report included? | --> | UEInformationResponse | 5 | P |
| 32 | Void |  |  |  |  |
| 33 | Void |  |  |  |  |
| 34 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 12? | - | - | 1 | - |

Table 8.6.4.4.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on Cell 12. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

Table 8.6.4.4.3.2-4: Void

8.6.4.4.3.3	Specific message contents

Table 8.6.4.4.3.3-0: SystemInformationBlockType2 for Cell 1, Cell 4 and Cell 12 (preamble and all the steps in Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-0: Conditions for specific message contents
in Tables 8.6.4.4.3.3-2, 8.6.4.4.3.3-5, 8.6.4.4.3.3-14 and 8.6.4.4.3.3-18

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.4.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.4.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.4.3.3-2: MeasConfig (Table 8.6.4.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObect[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObect[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| reportConfigId[2] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-3: MeasurementReport (step 4, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPcell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-4: RRCConnectionReconfiguration (step 5, step 14 and step 22, Table 8.6.4.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.4.4.3.3-5: MobilityControlInfo (step 5, Table 8.6.4.4.3.3-4)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-7: RRCConnectionReestablishment (step 8, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-8: RRCConnectionReestablishmentComplete (step 9 and step 26, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-9: RRCConnectionReconfiguration (step 10, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-10: MeasurementReport (step 13, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPcell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-11: MobilityControlInfo (step 14, Table 8.6.4.4.3.3-4)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 4 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-12: RRCConnectionReconfigurationComplete (step 15, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r10 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.4.4.3.3-13: UEInformationRequest (step 16 and step 31Aa1, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-14: UEInformationResponse (step 17, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE(SIZE(1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 | The ARFCN of Cell 12 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId | Phys cell id of cell 12 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | Not present or (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. | Target PCell |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 | “Verify setting of reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell at RRC connection re-establishment due to handover failure” |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not Checked/Present |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  | Source PCell |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  | plmn-Identity |
| cellIdentity | cellIdentity of Cell 1 |  | cellIdentity |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-15: RRCConnectionReconfiguration (step 18, Table 8.6.4.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.4.3.3-16: MeasConfig (Table 8.6.4.4.3.3-15)

Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ

Table 8.6.4.4.3.3-17: MeasurementReport (step 21, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPcell SEQUENCE { |  | Cell 4 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-18: MobilityControlInfo (step 22, Table 8.6.4.4.3.3-4)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-19: RRCConnectionReestablishmentRequest (step 24, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 4 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.4.3.3-20: UEInformationResponse (step 31Aa2, Table 8.6.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| rlf-Report-r9 SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.4.4.3.3-21: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.4.4.3.3-22: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 30, Table 8.6.4.4.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

#### 8.6.4.5	Radio Link Failure logging / Reporting of ECGI of the PCell

8.6.4.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed intra-frequency handover and reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing previousPCellId }

}

8.6.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.7.5, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48, hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.5.3	Test description

8.6.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.5.3.2	Test procedure sequence

Table 8.6.4.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | No Cells are available. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | Only Cell 2 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 2 parameter according to the row "T1" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra-frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6A | Wait for 1s to ensure that the UE completes the acquisition of essential SI | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 2 parameter according to the row "T2" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 8 | Wait for 3s to ensure that the UE detects T310 expiry. | - | - | - | - |
| 9 | The SS changes Cell 2 parameter according to the row "T3" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 10 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits an RRCConnectionReestablishmentComplete message with radio link failure information on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 15 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 17 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.6.4.5.3.3	Specific message contents

Table 8.6.4.5.3.3-0: Conditions for specific message contents
in Tables 8.6.4.5.3.3-2 and 8.6.4.5.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.5.3.3-2: MeasConfig (Table 8.6.4.5.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObect[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-3: MeasurementReport (step 4, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.6.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.4.5.3.3-5: MobilityControlInfo (Table 8.6.4.5.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-6: RRCConnectionReestablishmentRequest (step 10, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-7: RRCConnectionReestablishmentComplete (step 12, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-8: RRCConnectionReconfiguration (step 13, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-9: UEInformationRequest (step 15, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| logMeasReportReq-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-10: UEInformationResponse (step 16, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 | Not present |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.4.6	Void

#### 8.6.4.7	Radio Link Failure logging / Location information

8.6.4.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has radio link failure information with location information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell including locationCoordinates }

}

8.6.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 34.306, clause 4.3.13.2; TS 36.331, clause 5.3.11.3 and 5.6.5.3.

[TS 36.306, clause 4.3.13.2 (TP1)]

standaloneGNSS-Location

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.7.3	Test description

8.6.4.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE: None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.7.3.2	Test procedure sequence

Same test procedure as specified in 8.6.4.1.3.2 with the following exception:

-	The specific message content for the UEInformationResponse message in Table 8.6.4.1.3.3-8 is replaced by the specific message content in Table 8.6.4.7.3.3-1.

8.6.4.7.3.3	Specific message contents

Table 8.6.4.7.3.3-1: UEInformationResponse (step 12, Table 8.6.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |  |  |  |
| rsrpResult-r9 |  | (0..97) |  |  |  |  |
| rsrqResult-r9 |  | Not present or (0..34) |  |  |  |  |
| } |  |  |  |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |  |  |
| carrierFreq-r9[1] |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  |  |  | Band > 64 |  |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |  |  |
| physCellId[1] |  | Physical cell Identity of Cell 2 |  |  |  |  |
| cgi-Info[1] |  | Not present |  |  |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |  |  |
| rsrqResult |  | (0..34) |  |  |  |  |
| additionalSI-Info-r9 |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| measResultListUTRA-r9 |  | Not present |  |  |  |  |
| measResultListGERAN-r9 |  | Not present |  |  |  |  |
| measResultsCDMA2000-r9 |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |
| locationInfo-r10 SEQUENCE { |  |  |  |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |  |  |  |  |
| ellipsoid-Point-r10 |  | Any allowed value |  |  |  |  |  |
| ellipsoidPointWithAltitude-r10 |  | Any allowed value |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| failedPCellId-r10 CHOICE { |  | cellGlobalId-r10 or pci-arfcn-r10 |  | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |  |  |  |
| plmn-Identity |  | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |
| cellIdentity |  | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |
| } |  |  |  |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |  |  |  |
| physCellId-r10 |  | Physical cell Identity of Cell 1 |  |  |  |  |
| carrierFreq-r10 |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| carrierFreq-r10 |  | maxEARFCN |  |  |  | Band > 64 |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |  |  |  |
| plmn-Identity |  | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |  |  |
| cellIdentity |  | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |  |  |
| } |  |  |  |  |  |  |
| timeConnFailure-r10 |  | Not present |  |  |  |  |
| connectionFailureType-r10 |  | rlf |  |  |  |  |
| previousPCellId-r10 |  | Not present |  |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  |  |  |  | Band > 64 |  |  |
| carrierFreq-v1090 |  | Not present or the same downlink EARFCN as used for Cell 1 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | Band > 64 |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |  |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |  |  |  |
| carrierFreq-v9e0[1] |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

#### 8.6.4.8	Radio Link Failure logging / Logging and reporting / Reporting at RRC connection establishment / PLMN list

8.6.4.8.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {

when { UE detects radio link failure and T311 is expired and camp on a cell belonging to equivalent PLMN }

then { UE performs RRC Connection Establishment procedure and sends an RRCConnectionSetupComplete message with rlf-InfoAvailable }

}

(2)

with { UE having sent an RRCConnectionSetupComplete message with rlf-InfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends a UEInformationResponse message with rlf-Report }

}

8.6.4.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.11.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

…

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

3>	set the c-RNTI to the C-RNTI used in the PCell;

3>	set the rlf-Cause to the trigger for detecting radio link failure;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.8.3	Test description

8.6.4.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.4.8.3.1-1.

Table 8.6.4.8.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.4.8.3.3-4

-	The UE is in state Generic RB Established (State 3) on Cell 1 according to [18].

-	DRX is not configured

8.6.4.8.3.2	Test procedure sequence

Table 8.6.4.8.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T1" and "T2" are to be applied. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.8.3.2-1: Time instances of cell power level

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | "Off" | "Off" | No cell is available (NOTE 1) |
| T2 | RS EPRE | dBm/15kHz | "Off" | -85 | (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.4.8.3.2-1. | - | - | - | - |
| 2 | The SS waits for 12s to ensure that the UE detects T311 expiry. | - | - | - | - |
| 3 | The SS changes Cell 12 parameter according to the row "T2" in Table 8.6.4.8.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on cell 12 to initiate a tracking area update procedure. | --> | RRCConnectionRequest | - | - |
| 5 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 6 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. Check: Does the UE send an RRCConnectionSetupComplete message with rlf-InfoAvailable on Cell 12? | --> | RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 7-9 | Steps 4 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 12. NOTE: The UE performs a TAU procedure. The TAU is accepted with PLMN1 listed as an Equivalent PLMN and the RRC connection is released. | - | - | - | - |
| 10-18 | Steps 1 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 12. | - | - | - | - |
| 19 | The SS sends a UEInformationRequest message to get rlf-ReportReq on Cell 12. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE send a UEInformationResponse message with rlf-Report on Cell 12? | --> | UEInformationResponse | 2 | P |

8.6.4.8.3.3	Specific message contents

Table 8.6.4.8.3.3-1: RRCConnectionSetupComplete (step 6, Table 8.6.4.8.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.8.3.3-1A: Message TRACKING AREA UPDATE ACCEPT (step 7, 8.6.4.8.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.4.8.3.3-2: UEInformationRequest (step 19, Table 8.6.4.8.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF Report

Table 8.6.4.8.3.3-3: UEInformationResponse (step 20, Table 8.6.4.8.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-Report-r9 ::= SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not Present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |  |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 1 |  |  |  |
| } |  |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.8.3.3-4: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.4.8.3.3-5: RRCConnectionReconfiguration (preamble)

| Derivation path: 36.508 table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.4.9	Radio Link Failure logging / Logging and reporting / Reporting at intra LTE handover / PLMN list

8.6.4.9.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {

when { UE performs RRC connection re-establishment procedure and intra-LTE handover procedure}

then { UE sends an RRCConnectionReconfigurationComplete message with rlf-InfoAvailable }

}

(2)

with { UE having sent an RRCConnectionReconfigurationComplete message with rlf-InfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends a UEInformationResponse message with rlf-Report and PLMN list }

}

8.6.4.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.4, 5.3.11.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

…

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

3>	set the c-RNTI to the C-RNTI used in the PCell;

3>	set the rlf-Cause to the trigger for detecting radio link failure;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.9.3	Test description

8.6.4.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.4.9.3.1-1.

Table 8.6.4.9.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.4.9.3.3-8.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.9.3.2	Test procedure sequence

Table 8.6.4.9.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T1", "T2" and "T3" are to be applied. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.9.3.2-1: Time instances of cell power level

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | "Off" | "Off" | No cell is available (NOTE 1) |
| T2 | RS EPRE | dBm/15kHz | -85 | "Off" | (NOTE 1) |
| T3 | RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.4.9.3.2-1. | - | - | - | - |
| 2 | The SS waits for 3s to ensure that the UE detects T310 expiry. | - | - | - | - |
| 3 | The SS changes Cell 1 parameter according to the row "T2" in Table 8.6.4.9.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmit an RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1. | --> | RRCConnectionReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1 | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS changes Cell 1 and Cell 12 parameter according to the row "T3" in Table 8.6.4.9.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 12. | --> | MeasurementReport | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does the UE send an RRCConnectionReconfigurationComplete message with rlf-InfoAvailable on Cell 12? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 13-14A | Steps 3 to 5 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 12. NOTE: The UE performs a TAU procedure. The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | - | - | - | - |
| 15 | The SS sends a UEInformationRequest message to get rlf-ReportReq on Cell 12. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE send a UEInformationResponse message with rlf-Report on Cell 12? | --> | UEInformationResponse | 2 | P |

8.6.4.9.3.3	Specific message contents

Table 8.6.4.9.3.3-0: Conditions for specific message contents
in Tables 8.6.4.9.3.3-3, 8.6.4.9.3.3-4b and 8.6.4.9.3.3-7

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.4.9.3.3-1: RRCConnectionReestablishmentRequest (step 4, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-2: RRCConnectionReconfiguration (step 7, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-3: MeasConfig (step 7, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObect[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObect[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-4: MeasurementReport (step 10, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-4a: RRCConnectionReconfiguration (step 11, Table 8.6.4.9.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.4.9.3.3-4b: MobilityControlInfo (Table 8.6.4.9.3.3-4a)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-5: RRCConnectionReconfigulationComplete (step 12, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigulationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions SEQUENCE{ |  |  |  |
| rrcConnectionReconfigulationComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-5A: Message TRACKING AREA UPDATE ACCEPT (step 14, 8.6.4.9.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.4.9.3.3-6: UEInformationRequest (step 15, Table 8.6.4.9.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF Report

Table 8.6.4.9.3.3-7: UEInformationResponse (step 16, Table 8.6.4.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not Present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| basicFields-r11SEQUENCE { |  |  |  |
| c-RNTI-r11 | Any allowed value |  |  |
| rlf-Cause-r11 | t310-Expiry |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.9.3.3-8: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.6.4.10	Radio Link Failure logging / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list

8.6.4.10.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {

when { UE detects T310 is expired and performs RRC Connection re-establishment procedure on a cell belonging to equivalent PLMN  }

then { UE sends an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE having sent an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE sends a UEInformationResponse message with rlf-Report and PLMN list }

}

8.6.4.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.5, 5.3.11.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include the rlf-InfoAvailable;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

…

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

3>	set the c-RNTI to the C-RNTI used in the PCell;

3>	set the rlf-Cause to the trigger for detecting radio link failure;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.10.3	Test description

8.6.4.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.4.8.3.1-1.

Table 8.6.4.10.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.4.10.3.3-4

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.10.3.2	Test procedure sequence

Table 8.6.4.8.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T1" and "T2" are to be applied. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.10.3.2-1: Time instances of cell power level

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | -91 |  |
| T2 | RS EPRE | dBm/15kHz | "Off" | -85 | (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes cell parameters according to the row "T1" in Table 8.6.4.10.3.2-1. | - | - | - | - |
| 2 | The SS waits for 60s to ensure that the UE detects the present of Cell12. | - | - | - | - |
| 3 | The SS changes cell parameters according to the row "T2" in Table 8.6.4.10.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 12. | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmit an RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 6 | Check: Does the UE send an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 12? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 12 | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 12. | --> | RRCConnectionReconfigurationComplete | - | - |
| 8A | The UE transmits a Tracking Area Update Request message on Cell 12. | --> | NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 8B | The SS transmits a Tracking Area Update Accept Note: The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | <-- | NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 8C | The UE transmits a Tracking Area Update Complete | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 9 | The SS sends a UEInformationRequest message to get rlf-ReportReq on Cell 12. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE send a UEInformationResponse message with rlf-Report on Cell 12? | --> | UEInformationResponse | 2 | P |

8.6.4.10.3.3	Specific message contents

Table 8.6.4.10.3.3-1: RRCConnectionReestablishmentComplete (step 6, Table 8.6.4.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.10.3.3-1A: Message TRACKING AREA UPDATE ACCEPT (step 8B, 8.6.4.10.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.4.10.3.3-2: UEInformationRequest (step 9, Table 8.6.4.10.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF

Table 8.6.4.10.3.3-3: UEInformationResponse (step 10, Table 8.6.4.10.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| RLF-Report-r9 ::= SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not Present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present or any allowed value |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| basicFields-r11SEQUENCE { |  |  |  |
| c-RNTI-r11 | Any allowed value |  |  |
| rlf-Cause-r11 | t310-Expiry |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.10.3.3-4: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.6.4.11	Radio Link Failure logging / Logging and reporting / Dropped QCI

8.6.4.11.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state having a DRB with QCI1 and detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCConnectionReestablishmentComplete message with rlf-InfoAvailable included

}

}

(2)

with { UE in RRC_CONNECTED having a DRB with QCI1 and reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report included  with IE drb-EstablishedWithQCI-1-r13}

}

8.6.4.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.7.5, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

Except for NB-IoT, if the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	if the UE is not a NB-IoT UE:

2>	set the content of RRCConnectionReestablishmentComplete message as follows:

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4>	include the rlf-InfoAvailable;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached for an SRB or for an MCG or split DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2> except for NB-IoT, store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

...

3>	if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

4>	include the drb-EstablishedWithQCI-1;

3>	set the connectionFailureType to rlf;

3>	set the c-RNTI to the C-RNTI used in the PCell;

3>	set the rlf-Cause to the trigger for detecting radio link failure;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.11.3	Test description

8.6.4.11.3.1	Pre-test conditions

System Simulator:

-Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18]

8.6.4.11.3.2	Test procedure sequence

Table 8.6.4.11.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.11.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | Only Cell 1 is available. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | Only Cell 2 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-14 | Steps 1 to 14 of the generic test procedure for IMS MO speech call (TS 36.508 4.5A.6.3-1). | - | - | - | - |
| 15 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in Table 8.6.4.11.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 2 is suitable for camping. | - | - | - | - |
| 16 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 17 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 18 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 2 including the IE rlf-InfoAvailable. | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 19 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message on Cell 2 including IE drb-EstablishedWithQCI-1-r13? | --> | UEInformationResponse | 2 | P |

8.6.4.11.3.3	Specific message contents

Table 8.6.4.11.3.3-1: RRCConnectionReestablishmentRequest (step 16, Table 8.6.4.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.11.3.3-2: RRCConnectionReestablishmentComplete (step 18, Table 8.6.4.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | True |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.11.3.3-3: UEInformationRequest (step 19, Table 8.6.4.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | True |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.11.3.3-4: UEInformationResponse (step 20, Table 8.6.4.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |  |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  |  |
| } |  |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | Any allowed value |  |  |
| rlf-Cause-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| drb-EstablishedWithQCI-1-r13 | qci1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

#### 8.6.4.12	Void

#### 8.6.4.13	Void

### 8.6.5	Inter-RAT Logged Radio Link Failure

#### 8.6.5.1	Radio Link Failure logging / Reporting at UTRAN Inter-RAT handover

8.6.5.1.1	Test Purpose (TP)

(1)

with { UE selects the UTRAN cell and enters UTRA CELL_DCH(PS-DCCH+DTCH_DCH) after detection of radio link failure in an E-UTRAN cell }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message }

then { UE transmits an RRCConnectionReconfigurationComplete message containing rlf-InfoAvailable and enters E-UTRA RRC_CONNECTED state }

}

(2)

with { UE in RRC_CONNECTED having reported that the UE has radio link failure information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for UTRA neighbour cell }

}

8.6.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.11.3, 5.4.2.3 and 5.6.5.3.

[TS 36.331, clause 5.3.11.3 (TP1, TP2)]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48, hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.4.2.3 (TP1)]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

...

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.5.1.3	Test description

8.6.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.5.1.3.2	Test procedure sequence

Table 8.6.5.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.6.5.1.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -70 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -70 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | Only Cell 5 is available. (NOTE 1) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -70 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -70 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 5 parameters according to the row "T1" in Table 8.6.5.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 parameter according to the row "T2" in Table 8.6.5.1.3.2-1. | - | - | - | - |
| 6 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 6AAa1 to 6AAa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 6AAa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 6AAa1a-6AAa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 6AAa1a4 | SS releases RRC Connection | - | - | - | - |
| 6AAa1a5 | SS Stops 6s timer | - | - | - | - |
| 6AAa1b1 | The 6s timer expires | - | - | - | - |
| 6A | The SS waits for 5 s to ensure the UE is in service | - | - | - | - |
| 7-11 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 12a1 to 12a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto Step 13. | - | - | - | - |
| 12a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 12a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 13 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 14 | The SS changes Cell 1 and Cell 5 parameters according to the row "T3" in Table 8.6.5.1.3.2-1. | - | - | - | - |
| 15 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 16 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 17 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with radio link failure information on Cell 1? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 18 | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| - | EXCEPTION: Steps 18Aa1 to 18Aa2b1 describe optional behaviour if UE sends IMS deregistration at steps 6AAa1 to 6AAa1b1 | - | - | - | - |
| 18Aa1-18Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 19 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message on Cell 1? | --> | UEInformationResponse | 2 | P |
| 21 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 2 | - |

8.6.5.1.3.3	Specific message contents

Table 8.6.5.1.3.3-0: Conditions for specific message contents
in Tables 8.6.5.1.3.3-2, 8.6.5.1.3.3-8 and 8.6.5.1.3.3-12

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.5.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.5.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.5.1.3.3-2: MeasConfig (Table 8.6.5.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f8 |  |  |
| measObject[2] | MeasObjectUTRA-f8 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-UTRA |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-UTRA(-75, -82) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| reportConfigId[1] | IdReportConfig-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1.3.3-3: QuantityConfig (Table 8.6.5.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-3A, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig ::= SEQUENCE { |  |  |  |
| quantityConfigUTRA SEQUENCE { |  |  |  |
| measQuantityUTRA-FDD | cpich-RSCP |  | UTRA-FDD |
| measQuantityUTRA-TDD | pccpch-RSCP |  | UTRA-TDD |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| quantityConfigUTRA-v1020 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1.3.3-4: MeasObjectUTRA-f8 (Table 8.6.5.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Same downlink ARFCN as used for Cell 5 |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListUTRA-FDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-FDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| cellsToAddModListUTRA-TDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-TDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| csg-allowedReportingCells-v930 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1.3.3-5: MeasurementReport (step 4, Table 8.6.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 5 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 5 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1.3.3-6: HANDOVER FROM UTRAN COMMAND (step 16, Table 8.6.5.1.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.6.5.1.3.3-7: RRCConnectionReconfiguration (Table 8.6.5.1.3.3-6)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.6.5.1.3.3-8: MobilityControlInfo (Table 8.6.5.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 1 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 1 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.6.5.1.3.3-9: SecurityConfigHO (Table 8.6.5.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.1.3.3-10: RRCConnectionReconfigurationComplete (step 17, Table 8.6.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present or any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.1.3.3-11: UEInformationRequest (step 19, Table 8.6.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.1.3.3-12: UEInformationResponse (step 20, Table 8.6.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 | Same downlink ARFCN as used for Cell 5 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 5 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 5 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| utra-RSCP | Not present or (-5..91) |  |  |
| utra-EcN0 | Not present or (0..49) |  | UTRA-FDD |
|  | Not present |  | UTRA-TDD |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 | Not present |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 8.6.5.1a	Radio Link Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list

8.6.5.1a.1	Test Purpose (TP)

(1)

with { UE selects the UTRAN cell and enters UTRA CELL_DCH(PS-DCCH+DTCH_DCH) after detection of radio link failure in an E-UTRAN cell }

ensure that {
  when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message and UE selects the EPLMN which is not the RPLMN }

then { UE transmits an RRCConnectionReconfigurationComplete message containing rlf-InfoAvailable and enters E-UTRA RRC_CONNECTED state }

}

(2)

with { UE in RRC_CONNECTED having reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for UTRA neighbour cell }

}

8.6.5.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.11.3, 5.4.2.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.11.3 (TP1, TP2)]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

3>	set the c-RNTI to the C-RNTI used in the PCell;

3>	set the rlf-Cause to the trigger for detecting radio link failure;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.4.2.3 (TP1)]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

...

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

...

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.5.1a.3	Test description

8.6.5.1a.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 9 and Cell 12

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

Table 8.6.5.1a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 9 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.5.1a.3.3-1

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.5.1a.3.2	Test procedure sequence

Table 8.6.5.1a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.6.5.1a.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.1a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 9 | Cell12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - | "Off" | The power level values are such that entering conditions for event B2 are satisfied. (NOTE2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -70 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -70 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | "Off" | Only Cell 9 is available. (NOTE 1, NOTE 2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -70 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -70 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | “Off” | - | -70 | The power level values are such that entering conditions for event 3a are satisfied. (NOTE 1) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -85 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 | - |  |
| NOTE 1: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. NOTE 2: Power level "Off" for UTRA cell is defined in TS 34.108 Table 6.1.4 and Table 6.1.9. |  |  |  |  |  |  |

Table 8.6.5.1a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 9 parameters according to the row "T1" in Table 8.6.5.1a.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 parameter according to the row "T2" in Table 8.6.5.1a.3.2-1. | - | - | - | - |
| 6 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 9. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 6Aa1 to 6Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 6Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 6Aa1a1-6Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 6Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 6Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 6Aa1b1 | The 6s timer expires | - | - | - | - |
| 7-11 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 9 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 12a1 to 12a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto Step 13. | - | - | - | - |
| 12a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 9 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 12a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 9. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 13 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 9. | <-- | MEASUREMENT CONTROL | - | - |
| 14 | The SS changes Cell 9 and Cell 12 parameters according to the row "T3" in Table 8.6.5.1a.3.2-1. | - | - | - | - |
| 15 | The UE transmits a MEASUREMENT REPORT message on Cell 9 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 16 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 9. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 17 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with radio link failure information on Cell 12? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 18 | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 12. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA The TAU is accepted with PLMN1 listed as an Equivalent PLMN.. | - | - | - | - |
| - | EXCEPTION: Steps 18Aa1 to 18Aa2b1 describe optional behaviour if UE sends IMS deregistration at steps 6Aa1 to 6Aa1b1 | - | - | - | - |
| 18Aa1-18Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 19 | The SS transmits a UEInformationRequest message on Cell 12. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message on Cell 12? | --> | UEInformationResponse | 2 | P |
| 21 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 12? | - | - | 2 | - |

8.6.5.1a.3.3	Specific message contents

Table 8.6.5.1a.3.3-0: Conditions for specific message contents
in Tables 8.6.5.1a.3.3-3, 8.6.5.1a.3.3-10 and 8.6.5.1a.3.3-14

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.5.1a.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.5.1a.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.5.1a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.5.1a.3.3-3: MeasConfig (Table 8.6.5.1a.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f10 |  |  |
| measObject[2] | MeasObjectUTRA-f10 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-UTRA |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-UTRA(-75, -82) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f10 |  |  |
| reportConfigId[1] | IdReportConfig-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1a.3.3-4: QuantityConfig (Table 8.6.5.1a.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-3A, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig ::= SEQUENCE { |  |  |  |
| quantityConfigUTRA SEQUENCE { |  |  |  |
| measQuantityUTRA-FDD | cpich-RSCP |  | UTRA-FDD |
| measQuantityUTRA-TDD | pccpch-RSCP |  | UTRA-TDD |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| quantityConfigUTRA-v1020 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1a.3.3-5: MeasObjectUTRA-f10 (Table 8.6.5.1a.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Same downlink ARFCN as used for Cell 9 |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListUTRA-FDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-FDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 9 |  |  |
| } |  |  |  |
| cellsToAddModListUTRA-TDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-TDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| csg-allowedReportingCells-v930 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1a.3.3-6: MeasurementReport (step 4, Table 8.6.5.1a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 9 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 9 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.5.1a.3.3-7: MEASUREMENT CONTROL (step 13, Table 8.6.5.1a.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -80 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.6.5.1a.3.3-8: HANDOVER FROM UTRAN COMMAND (step 16, Table 8.6.5.1a.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.6.5.1a.3.3-9: RRCConnectionReconfiguration (Table 8.6.5.1a.3.3-8)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.6.5.1a.3.3-10: MobilityControlInfo (Table 8.6.5.1a.3.3-9)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 12 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 12 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.6.5.1a.3.3-11: SecurityConfigHO (Table 8.6.5.1a.3.3-8)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.1a.3.3-12: RRCConnectionReconfigurationComplete (step 17, Table 8.6.5.1a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present or any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.1a.3.3-12A: Message TRACKING AREA UPDATE ACCEPT (step 18, 8.6.5.1a.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.5.1a.3.3-13: UEInformationRequest (step 19, Table 8.6.5.1a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.5.1a.3.3-14: UEInformationResponse (step 20, Table 8.6.5.1a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 | Not present |  |  |
| measResultListUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 | Same downlink ARFCN as used for Cell 9 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 9 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 9 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| utra-RSCP | Not present or (-5..91) |  |  |
| utra-EcN0 | Not present or (0..49) |  | UTRA-FDD |
|  | Not present |  | UTRA-TDD |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 | Not present |  |  |
| timeConnFailure-r10 | Not present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| previousPCellId-r10 | Not present |  |  |
| basicFields-r11SEQUENCE { |  |  |  |
| c-RNTI-r11 | Any allowed value |  |  |
| rlf-Cause-r11 | t310-Expiry |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 SEQUENCE { |  |  |  |
| carrierFreq-r11 | Same downlink ARFCN as used for Cell 12 |  |  |
| physCellId-r11 CHOICE { |  |  |  |
| fdd-r11 | PhysicalCellIdentity of Cell 12 |  | UTRA-FDD |
| tdd-r11 | PhysicalCellIdentity of Cell 12 |  | UTRA-TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 8.6.5.2	Radio Link Failure logging / Reporting at GERAN Inter-RAT handover

8.6.5.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC_CONNECTED state with radio link failure information available in VarRLF-Report }

ensure that {
  when { UE handovers back to E-UTRAN from GERAN with the RPLMN equalling to the plmn-Identity stored in VarRLF-Report }

then { UE sends the RRCConnectionReconfigurationComplete message with rlf-InfoAvailable included }

}

(2)

with { UE in E-UTRAN RRC_CONNECTED state with the Inter-RAT GERAN neighbour cell measurement information configured and the radio link failure information available in VarRLF-Report }

ensure that {
  when { UE receives the UEInformationRequest message with rlf-ReportReq set to true and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the UEInformationResponse message with GERAN neighbour cell information (measResultListGERAN) included in measResultNeighCells }

}

8.6.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.11.3, 5.4.2.3 and 5.6.5.3.

[TS 36.331, clause 5.3.11.3 (TP1, TP2)]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.4.2.3 (TP1)]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.5.2.3	Test description

8.6.5.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 24 - Cell 1 and Cell 2 are E-UTRAN cell, Cell 24 is a GERAN cell.

-	All cells belong to the same PLMN.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.5.2.3.2	Test procedure sequence

Table 8.6.5.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1","T2"and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | - |  |
|  | RSSI | dBm | - | [-65] |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | - |  |
|  | RSSI | dBm | - | [-65] |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - |  |
|  | RSSI | dBm | - | [-85] |  |

Table 8.6.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 24 parameters according to the row "T1" in table 8.6.5.2.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event B2 with the measured rssi value for Cell 24. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 1 and Cell 24 parameters according to the row "T2" in Table 8.6.5.2.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 24 is suitable for camping. | - | - | - | - |
| 6 | The UE executes the generic test procedure in TS 36.508 subclause 6.4.2.9 to make sure that the UE is camped on GERAN Cell 24 | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 24 parameters according to the row "T3" in table 8.6.5.2.3.2-1. | - | - | - | - |
| 8 | The SS transmits PS HANDOVER COMMAND message on Cell 24 to order the UE to perform inter-RAT handover to Cell 1. | <-- | PS HANDOVER COMMAND | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 with rlf-InfoAvailable included? | --> | RRCConnectionReconfigurationtComplete | 1 | P |
| 10 | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 1. | - | - | - | - |
| 11 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security (mapped security context) | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 12 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| 13 | SS responds with a TRACKING AREA UPDATE ACCEPT message. | - | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 14 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 15 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 1. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 1 with the GERAN neighbour Cell 24 measurement information (measResultListGERAN) included in rlf-Report? | --> | UEInformationResponse | 2 | P |
| 17 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.6.5.2.3.3	Specific message contents

Table 8.6.5.2.3.3-0: Conditions for specific message contents
in Tables 8.6.5.2.3.3-2, 8.6.5.2.3.3-5 and 8.6.5.2.3.3-9

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.5.2.3.3-1: RRCConnectionReconfiguration (step 2, Table 8.6.5.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.5.2.3.3-2: MeasConfig (Table 8.6.5.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| measObject[2] | MeasObjectGERAN-GENERIC(f11) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN(-69, [-79]) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigGERAN SEQUENCE { |  |  |  |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.2.3.3-3: MeasurementReport (step 4, Table 8.6.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultsNeighCells CHOICE { |  |  |  |
| measResultListGERAN SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] SEQUENCE { |  |  |  |
| arfcn | Not checked |  |  |
| bandIndicator | Not checked |  |  |
| } |  |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 24 |  |  |
| cgi-info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.2.3.3-4: PS HANDOVER COMMAND (step 8, Table 8.6.5.2.3.3-2)

| Derivation Path: 44.060, Table 11.2.43.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PAGE MODE | ‘00’B | Normal Paging |  |
| Global TFI | TFI of the downlink TBF |  |  |
| CONTAINER_ID | 0 |  |  |
| PS Handover to E-UTRAN Payload | ‘10’B |  |  |
| RRC Container IE |  |  |  |
| RRC_CONTAINER_LENGTH | Length of the container data |  |  |
| RRC_CONTAINER_DATA |  |  |  |
| RRCConnectionReconfiguration message |  |  | HO-TO-EUTRA |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  | Derivation Path: 36.331 clause 6.2.2 |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  | HO-TO-EUTRA Ref Table 8.6.5.2.3.3-5 |
| dedicatedInfoNASList | Not present |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  | HO-TO-EUTRA(n,m) |
| securityConfigHO | SecurityConfigHO |  | HO-TO-EUTRA Ref Table 8.6.5.2.3.3-6 |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.2.3.3-5: MobilityControlInfo (Table 8.6.5.2.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
|  | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.6.5.2.3.3-6: SecurityConfigHO (Table 8.6.5.2.3.3-4)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO SEQUENCE { |  |  |  |
| handoverType CHOICE{ |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.2.3.3-7: RRCConnectionReconfigurationComplete (step 9, Table 8.6.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r10 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.5.2.3.3-8: UEInformationRequest (step 15, Table 8.6.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.2.3.3-9: UEInformationResponse (step 16, Table 8.6.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListGERAN-r9 SEQUENCE(SIZE(1.. maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 SEQUENCE { |  |  |  |
| arfcn | Downlink ARFCN of Cell 24 |  |  |
| bandIndicator | The same band indicator of the Cell 24 |  |  |
| } |  |  |  |
| physCellId | 0001H |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rssi | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not Present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not Present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.5.3	Radio Link Failure logging / Reporting CDMA2000 neighbour cell information

8.6.5.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC_CONNECTED state with the Inter-RAT cdma2000 neighbour cell measurement information configured and the radio link failure information available in VarRLF-Report }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq set to true and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the UEInformationResponse message with CDMA2000 neighbour cell information (measResultsCDMA2000) included }

}

8.6.5.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

…

1>	set the reestablishmentCause as follows:

…

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

…

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48 hours after the radio link failure is detected.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.5.3.3	Test description

8.6.5.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 15 - Cell 1 and Cell 2 are E-UTRAN cell, Cell 15 is a CDMA2000 cell.

-	All cells belong to the same PLMN.

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3B) on Cell 1 according to [18].

-	The UE has performed HRPD pre-registration on Cell 15.

8.6.5.3.3.2	Test procedure sequence

Table 8.6.5.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 15 |  |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | “Off” | - | The power level values are such that camping on Cell 1 is guaranteed (Note 2) |
|  | Îor/Ioc | dB | - |  | -20 |  |
|  | Ioc | dBm/1.23 MHz | - |  | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - |  | -20 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -75 | - | Radio link failure occurred in Cell 1 and UE re-establish on Cell 2 |
|  | Îor/Ioc | dB | - |  | -20 |  |
|  | Ioc | dBm/1.23 MHz | - |  | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - |  | -20 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -80 | - | The power level values are such that entering conditions for event B2 on Cell 15 are satisfied |
|  | Îor/Ioc | dB | - |  | -5 |  |
|  | Ioc | dBm/1.23 MHz | - |  | -55 |  |
|  | Pilot Ec/Io (Note 1) | dB | - |  | -6 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. Note 2: Power level "Off" for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.5.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in Table 8.6.5.3.3.2-1 in order that the radio link quality of Cell 1 is degraded and Cell 2 is suitable for camping. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2 | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable included | --> | RRCConnectionReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 9 | The SS changes Cell 2 and Cell 15 parameter according to the row "T2" in table 8.6.5.3.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message on Cell 2 to report event B2 with the measured pilotStrength value for Cell 15. | --> | MeasurementReport | - | - |
| 11 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true on Cell 2. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit a UEInformationResponse message on Cell 2 with the CDMA2000 neighbour Cell 15 measurement information (measResultsCDMA2000) included in rlf-Report? | --> | UEInformationResponse | 1 | P |

8.6.5.3.3.3	Specific message contents

Table 8.6.5.3.3.3-0: Conditions for specific message contents
in Tables 8.6.5.3.3.3-2 and 8.6.5.3.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.5.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.5.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.5.3.3.3-2: MeasConfig (Table 8.6.5.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f14 |  |  |
| measObject[2] | MeasObjectCDMA2000-GENERIC |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-CDMA2000 |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-CDMA2000(-69, -18) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f14 |  |  |
| reportConfigId[1] | IdReportConfig-B2-CDMA2000 |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigCDMA2000 SEQUENCE { |  |  |  |
| measQuantityCDMA2000 | pilotStrength |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig SEQUENCE { |  |  |  |
| gapActivation CHOICE { |  |  |  |
| activate SEQUENCE { |  |  |  |
| gapPattern CHOICE { |  |  |  |
| gp1 SEQUENCE { |  |  |  |
| gapOffset | 30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-3: MeasObjectCDMA2000-GENERIC (Table 8.6.5.3.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCDMA2000-GENERIC ::= SEQUENCE { |  |  |  |
| cdma2000-Type | TypeHRPD |  |  |
| CarrierFreqCDMA2000 SEQUENCE { |  |  |  |
| bandClass | Band Class of frequency under test |  |  |
| arfcn | f14 |  |  |
| } |  |  |  |
| SearchWindowSize | 15 |  |  |
| offsetFreq | db0 |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList CHOICE {} | Cell 15 | Listed cell parameters to be reported |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-4: RRCConnectionReestablishmentRequest (step 4, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-5: RRCConnectionReestablishment (step 5, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-6: RRCConnectionReestablishmentComplete (step 6, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-7: RRCConnectionReconfiguration (step 7, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-8: MeasurementReport (step 10, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultsCDMA2000 :=SEQUENCE { |  |  |  |
| preRegistrationStatusHRPD | TRUE |  |  |
| measResultListCDMA2000 ::=SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry | Note 1 |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-9: UEInformationRequest (step 11, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.5.3.3.3-10: UEInformationResponse (step 12, Table 8.6.5.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultsCDMA2000-r9 SEQUENCE(SIZE(1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9 SEQUENCE { |  |  |  |
| bandClass |  |  |  |
| arfcn | The ARFCN of Cell 15 |  |  |
| } |  |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| preRegistrationStatusHRPD | true |  |  |
| measResultListCDMA2000 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId | Phys cell id of cell 15 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 | “Verify setting of reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell at RRC connection re-establishment due to handover failure” |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Not Present |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 | Not Present |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.5.4	Void

#### 8.6.5.5	Radio Link Failure logging / Logging and reporting /Bluetooth measurement collection

8.6.5.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCConnectionReestablishmentComplete message with logMeasAvailableBT included }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r9 including logMeasResultListBT }

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a Bluetooth beacon is not included in bt-NameListConfig }

then { UE sends the UEInformationResponse message not including the measurement result of the Bluetooth beacon not included in bt-NameListConfig }

}

8.6.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.5, 5.3.10.9, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.10.9]

The UE shall:

…

1>	if the received otherConfig includes the bt-NameListConfig:

2>	if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report;

1>	if the received otherConfig includes the wlan-NameListConfig:

2>	if wlan-NameListConfig is set to setup, attempt to have WLAN measurement results available for subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

…

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	except for NB-IoT, store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

…

2>	if AS security has not been activated:

3>	if the UE is a NB-IoT UE:

4>	if the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation:

5>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	except for a NB-IoT UE for which AS security has not been activated:

…

2>	if the UE is not a NB-IoT UE:

3>	if the UE is connected to EPC:

4>	set the content of RRCConnectionReestablishmentComplete message as follows:

5>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

6>	include the rlf-InfoAvailable;

5>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

6>	include logMeasAvailableMBSFN;

5>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailable;

5>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailableBT;

5>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailableWLAN;

5>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

6>	include the connEstFailInfoAvailable;

5>	if the UE has flight path information available:

6>	include flightPathInfoAvailable;

3>	perform the measurement related actions as specified in 5.5.6.1;

3>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

2>	else:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2a:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

…

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

8.6.5.5.3	Test description

8.6.5.5.3.1	Pre-test conditions

System Simulator:

- Cell 1, Cell 2, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 36.508 cl 4.4.9

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.5.5.3.2	Test procedure sequence

Table 8.6.5.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 8.6.5.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.4.13.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message on Cell 2 with logMeasAvailableBT included? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report-r9 with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2,3 | P |

8.6.5.5.3.3	Specific message contents

Same Specific message contents as specified in 8.6.4.1.3.3 with the following exception:

-	The specific message content for the RRCConnectionReconfiguration message in Table 8.6.5.5.3.3-1.

Table 8.6.5.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.5.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 SEQUENCE { |  |  |  |
| powerPrefIndicationConfig-r11 CHOICE {} | Not present |  |  |
| bw-PreferenceIndicationTimer-r14 | Not present | ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} |  |
| rlm-ReportConfig-r14 CHOICE {} | Not present |  |  |
| overheatingAssistanceConfig-r14 CHOICE {} | Not present |  |  |
| ailc-BitConfig-r15 | Not present | BOOLEAN |  |
| bt-NameListConfig-r15 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..maxBT-Name-r15)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r15[1] | Set as per Table 4.4.9-1 of [18] | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| wlan-NameListConfig-r15 CHOICE {} | Not present |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	The specific message content for the RRCConnectionReestablishmentComplete message in Table 8.6.4.1.3.3-5 is replaced by the specific message content in Table 8.6.5.5.3.3-2.

Table 8.6.5.5.3.3-2: RRCConnectionReestablishmentComplete (step 6, Table 8.6.5.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailable-r10 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableMBSFN-r12 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableBT-r15 | True |  |  |
| logMeasAvailableWLAN-r15 | Not checked |  |  |
| flightPathInfoAvailable-r15 | Not checked |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	The specific message content for the UEInformationResponse message in Table 8.6.4.1.3.3-8 is replaced by the specific message content in Table 8.6.5.5.3.3-3.

Table 8.6.5.5.3.3-3: UEInformationResponse (step 10, Table 8.6.5.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE {} | Not Checked |  |  |
| locationInfo-r10 | Not Checked |  |  |
| failedPCellId-r10 CHOICE {} | Not Checked |  |  |
| reestablishmentCellId-r10 SEQUENCE {} | Not Checked |  |  |
| timeConnFailure-r10 | Not Checked |  |  |
| connectionFailureType-r10 | Not Checked |  |  |
| previousPCellId-r10 | Not Checked |  |  |
| failedPCellId-v1090 SEQUENCE { } | Not Checked |  |  |
| basicFields-r11 SEQUENCE { } | Not Checked |  |  |
| previousUTRA-CellId-r11 SEQUENCE { } | Not Checked |  |  |
| selectedUTRA-CellId-r1 SEQUENCE { } | Not Checked |  |  |
| failedPCellId-v1250 SEQUENCE { } | Not Checked |  |  |
| measResultLastServCell-v1250 | Not Checked |  |  |
| lastServCellRSRQ-Type-r12 | Not Checked |  |  |
| measResultListEUTRA-v1250 | Not Checked |  |  |
| drb-EstablishedWithQCI-1-r13 | Not Checked |  |  |
| measResultLastServCell-v1360 | Not Checked |  |  |
| logMeasResultListBT-r15 SEQUENCE (SIZE (1..maxBT-IdReport-r15)) OF SEQUENCE { | 1 entry | Report Bluetooth beacon 1 |  |
| bt-Addr-r15 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |  |
| rssi-BT-r15 | Any allowed value | INTEGER (-128..127) |  |  |
| } |  |  |  |  |
| logMeasResultListWLAN-r15 | Not Checked |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.5.6	Radio Link Failure logging / Logging and reporting / WLAN measurement collection

8.6.5.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCConnectionReestablishmentComplete message with logMeasAvailableWLAN included }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r9 including logMeasResultListWLAN }

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a WLAN node is not included in wlan-NameListConfig }

then { UE sends the UEInformationResponse message not including  the measurement result of the WLAN node not included in wlan-NameListConfig}

}

8.6.5.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.5, 5.3.10.9, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.10.9]

The UE shall:

…

1>	if the received otherConfig includes the bt-NameListConfig:

2>	if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report;

1>	if the received otherConfig includes the wlan-NameListConfig:

2>	if wlan-NameListConfig is set to setup, attempt to have WLAN measurement results available for subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

…

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	except for NB-IoT, store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

…

2>	if AS security has not been activated:

3>	if the UE is a NB-IoT UE:

4>	if the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation:

5>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	except for a NB-IoT UE for which AS security has not been activated:

…

2>	if the UE is not a NB-IoT UE:

3>	if the UE is connected to EPC:

4>	set the content of RRCConnectionReestablishmentComplete message as follows:

5>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

6>	include the rlf-InfoAvailable;

5>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

6>	include logMeasAvailableMBSFN;

5>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailable;

5>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailableBT;

5>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

6>	include the logMeasAvailableWLAN;

5>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

6>	include the connEstFailInfoAvailable;

5>	if the UE has flight path information available:

6>	include flightPathInfoAvailable;

3>	perform the measurement related actions as specified in 5.5.6.1;

3>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

2>	else:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2a:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

…

[TS 36.331, clause 5.6.5.3

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

8.6.5.6.3	Test description

8.6.5.6.3.1	Pre-test conditions

System Simulator:

- Cell 1, Cell 2, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell27b are configured as per 36.508 cl 4.4.8 with the OFFLOAD condition

- System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.5.6.3.2	Test procedure sequence

Table 8.6.5.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.5.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 8.6.5.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.5.6.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message on Cell 2 with logMeasAvailableWLAN included? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report-r9 with logMeasResultListWLAN including one entry(WLAN AP 1) measurement result? | --> | UEInformationResponse | 2,3 | P |

8.6.5.6.3.3	Specific message contents

Same Specific message contents as specified in 8.6.4.1.3.3 with the following exception:

-	The specific message content for the RRCConnectionReconfiguration message in Table 8.6.5.6.3.3-1.

Table 8.6.5.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.5.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 with condition OtherConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 SEQUENCE { |  |  |  |
| powerPrefIndicationConfig-r11 CHOICE {} | Not present |  |  |
| bw-PreferenceIndicationTimer-r14 | Not present |  |  |
| rlm-ReportConfig-r14 CHOICE {} | Not present |  |  |
| overheatingAssistanceConfig-r14 CHOICE {} | Not present |  |  |
| ailc-BitConfig-r15 | Not present |  |  |
| bt-NameListConfig-r15 CHOICE {} | Not present |  |  |
| wlan-NameListConfig-r15 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..maxWLAN-Name-r15)) OF | 1 entry | WLAN AP 1 |  |
| WLAN-Name-r15[1] | Set as per Table 4.4.8-1 of [18] | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	The specific message content for the SystemInformationBlockType17 message in Table 8.6.5.6.3.3-1A.

Table 8.6.5.6.3.3-1A: SystemInformationBlockType17 (Preamble and all steps of Table 8.6.5.6.3.2-2

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 2 Entry |  |
| wlan-Identifiers[1] SEQUENCE{ |  |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[1] | Not present |  |  |
| hessid-r12[1] | Not present |  |  |
| } |  |  |  |
| wlan-Identifiers[2] SEQUENCE{ |  |  |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[2] | Not present |  |  |
| hessid-r12[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	The specific message content for the RRCConnectionReestablishmentComplete message in Table 8.6.4.1.3.3-5 is replaced by the specific message content in Table 8.6.5.6.3.3-2.

Table 8.6.5.6.3.3-2: RRCConnectionReestablishmentComplete (step 6, Table 8.6.5.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailable-r10 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableMBSFN-r12 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableBT-r15 | Not checked |  |  |
| logMeasAvailableWLAN-r15 | True |  |  |
| flightPathInfoAvailable-r15 | Not checked |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	The specific message content for the UEInformationResponse message in Table 8.6.4.1.3.3-8 is replaced by the specific message content in Table 8.6.5.6.3.3-3.

Table 8.6.5.6.3.3-3: UEInformationResponse (step 10, Table 8.6.5.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE {} | Not Checked |  |  |
| locationInfo-r10 | Not Checked |  |  |
| failedPCellId-r10 CHOICE {} | Not Checked |  |  |
| reestablishmentCellId-r10 SEQUENCE {} | Not Checked |  |  |
| timeConnFailure-r10 | Not Checked |  |  |
| connectionFailureType-r10 | Not Checked |  |  |
| previousPCellId-r10 | Not Checked |  |  |
| failedPCellId-v1090 SEQUENCE { } | Not Checked |  |  |
| basicFields-r11 SEQUENCE { } | Not Checked |  |  |
| previousUTRA-CellId-r11 SEQUENCE { } | Not Checked |  |  |
| selectedUTRA-CellId-r1 SEQUENCE { } | Not Checked |  |  |
| failedPCellId-v1250 SEQUENCE { } | Not Checked |  |  |
| measResultLastServCell-v1250 | Not Checked |  |  |
| lastServCellRSRQ-Type-r12 | Not Checked |  |  |
| measResultListEUTRA-v1250 | Not Checked |  |  |
| drb-EstablishedWithQCI-1-r13 | Not Checked |  |  |
| measResultLastServCell-v1360 | Not Checked |  |  |
| logMeasResultListBT-r15 | Not Checked |  |  |
| logMeasResultListWLAN-r15 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF SEQUENCE { | 1 entry | Report WLAN AP1 |  |  |
| wlan-Identifiers-r15[1] ::= SEQUENCE { |  |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |  |
| bssid-r12 | Not present |  |  |  |
| hessid-r12 | Not present |  |  |  |
| } |  |  |  |  |
| rssiWLAN-r15[1] | Any allowed value |  |  |  |
| rtt-WLAN-r15[1] | Any allowed value |  |  |  |
| } |  |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.6.6	Logged Handover Failure

#### 8.6.6.1	Handover Failure logging / Reporting of Intra-frequency measurements

8.6.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell }

}

8.6.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.4, 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report 48 hours after the failure is detected.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.1.3	Test description

8.6.6.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.1.3.2	Test procedure sequence

Table 8.6.6.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.6.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | Only Cell 1 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.6.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 2 parameters according to the row "T1" in Table 8.6.6.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.6.1.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 2 parameter according to the row "T2" in Table 8.6.6.1.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS transmits an RRCConnectionReestablishment message on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1. | --> | RRCConnectionReestablishmentComplete | - | - |
| 10 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message on Cell 1? | --> | UEInformationResponse | 1 | P |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

Table 8.6.6.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra-frequency handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.6.1.3.3	Specific message contents

Table 8.6.6.1.3.3-0: Condition for specific message contents in Table 8.6.6.1.3.3-2 and Table 8.6.6.1.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.6.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.6.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.6.1.3.3-2: MeasConfig (Table 8.6.6.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-3: MeasurementReport (step 4, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.6.6.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.1.3.3-5: MobilityControlInfo (Table 8.6.6.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-7: RRCConnectionReestablishmentComplete (step 9, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-8: RRCConnectionReconfiguration (step 10, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-9: UEInformationRequest (step 12, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.1.3.3-10: UEInformationResponse (step 13, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.6.2	Handover Failure logging / Reporting of Inter-frequency measurements

8.6.6.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the inter-frequency measurement and reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for inter-frequency neighbour cell }

}

8.6.6.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.4, 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report 48 hours after the failure is detected.

NOTE 2:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.2.3	Test description

8.6.6.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.2.3.2	Test procedure sequence

Table 8.6.6.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.6.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | Only Cell 1 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.6.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 3 parameters according to the row "T1" in Table 8.6.6.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.6.2.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 3 parameter according to the row "T2" in Table 8.6.6.2.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS transmits an RRCConnectionReestablishment message on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1. | --> | RRCConnectionReestablishmentComplete | - | - |
| 10 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message on Cell 1? | --> | UEInformationResponse | 1 | P |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

Table 8.6.6.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-frequency handover using MAC Random Access Preamble on Cell 3. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.6.2.3.3	Specific message contents

Table 8.6.6.2.3.3-0: Condition for specific message contents in Table 8.6.6.2.3.3-2, Table 8.6.6.2.3.3-5 and Table 8.6.6.2.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.6.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.6.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.6.2.3.3-2: MeasConfig (Table 8.6.6.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-3: MeasurementReport (step 4, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.6.6.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.2.3.3-5: MobilityControlInfo (Table 8.6.6.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-7: RRCConnectionReestablishmentComplete (step 9, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-8: RRCConnectionReconfiguration (step 10, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-9: UEInformationRequest (step 12, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.2.3.3-10: UEInformationResponse (step 13, Table 8.6.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 3 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.6.3	Void

#### 8.6.6.4	Handover Failure logging / Location information

8.6.6.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has handover failure information with location information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell including locationCoordinates }

}

8.6.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 34.306, clause 4.3.13.2; TS 36.331, clause 5.3.5.6 and 5.6.5.3.

[TS 36.306, clause 4.3.13.2 (TP1)]

standaloneGNSS-Location

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report 48 hours after the failure is detected.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.4.3	Test description

8.6.6.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.4.3.2	Test procedure sequence

Same test procedure as specified in 8.6.6.1.3.2 with the following exception:

- 	The specific message content for the UEInformationResponse message in Table 8.6.6.1.3.3-10 is replaced by the specific message content in Table 8.6.6.4.3.3-1.

8.6.6.4.3.3	Specific message contents

Table 8.6.6.4.3.3-0: Condition for specific message contents in Table 8.6.6.4.3.3-1

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.6.4.3.3-1: UEInformationResponse (step 13, Table 8.6.6.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |  |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |  |  |  |  |
| rsrpResult-r9 |  | (0..97) |  |  |  |  |  |
| rsrqResult-r9 |  | Not present or (0..34) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |  |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |  |  |  |
| carrierFreq-r9[1] |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |  |
| carrierFreq-r9[1] |  | maxEARFCN |  |  |  | Band > 64 |  |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |  |  |  |
| physCellId[1] |  | PhysicalCellIdentity of Cell 2 |  |  |  |  |  |
| cgi-Info[1] |  | Not present |  |  |  |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |  |  |  |
| rsrqResult |  | (0..34) |  |  |  |  |  |
| additionalSI-Info-r9 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measResultListUTRA-r9 |  | Not present |  |  |  |  |  |
| measResultListGERAN-r9 |  | Not present |  |  |  |  |  |
| measResultsCDMA2000-r9 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| locationInfo-r10 SEQUENCE { |  |  |  |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |  |  |  |  |
| ellipsoid-Point-r10 |  | Any allowed value |  |  |  |  |  |
| ellipsoidPointWithAltitude-r10 |  | Any allowed value |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| failedPCellId-r10 CHOICE { |  | cellGlobalId-r10 or pci-arfcn-r10 |  | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |  |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |  |  |  |  |
| plmn-Identity |  | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |  |  |  |
| cellIdentity |  | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |  |  |  |  |
| physCellId-r10 |  | PhysicalCellIdentity of Cell 2 |  |  |  |  |  |
| carrierFreq-r10 |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |  |
| carrierFreq-r10 |  | maxEARFCN |  |  |  | Band > 64 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |  |  |  |  |
| plmn-Identity |  | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |  |
| cellIdentity |  | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| timeConnFailure-r10 |  | Any allowed value |  |  |  |  |  |
| connectionFailureType-r10 |  | hof |  |  |  |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |  |  |  |  |
| plmn-Identity |  | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |  |
| cellIdentity |  | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  |  |  |  | Band > 64 |  |
| carrierFreq-v1090 |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | Band > 64 |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |  |  |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |  |  |  |  |
| carrierFreq-v9e0[1] |  | Same downlink EARFCN as used for Cell 2 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

#### 8.6.6.5	Handover Failure logging / Logging and reporting / Reporting at RRC connection establishment / PLMN list

8.6.6.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having detected handover failure }

ensure that {
  when { UE reselects to a cell that belongs to a PLMN which is different from the PLMN where the handover failure was detected but included in the plmn_IdentityList stored in VarRLF-Report }

then { UE transmits the RRCConnectionSetupComplete with IE rlf-InfoAvailable included }

}

(2)

with { UE indicated the availability of handover failure information in RRCConnectionSetupComplete message and the RPLMN is included in plmn-IdentityList stored in VarRLF-Report: }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE transmits a UEInformationResponse message including rlf-Report }

}

8.6.6.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.5.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.4 (TP1, TP2)]

The UE shall:

1>	set the content of RRCConnectionSetupComplete message as follows:

...

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

...

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.5.6 (TP1, TP2)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.5.3	Test description

8.6.6.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.6.5.3.1-1.

Table 8.6.6.5.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1, 2 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

- The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.6.5.3.3-13.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.5.3.2	Test procedure sequence

Table 8.6.6.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.6.6.5.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.6.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | "Off" | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). (NOTE 1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | "Off" | (NOTE 1) |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | -73 | The power level values are assigned to satisfy RCell 2 < RCell 12. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.6.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in Table 8.6.6.5.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 5A | The SS changes Cell 1 parameters according to the row "T2" in Table 8.6.6.5.3.2-1. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.6.5.3.2-3 should take place. | - | - | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS transmits an RRCConnectionRelease message on Cell 2. | <-- | RRCConnectionRelease | - | - |
| 12 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 13 | The SS changes Cell 1, Cell 2 and Cell 12 parameters according to the row "T3" in Table 8.6.6.5.3.2-1. | - | - | - | - |
| 14-15 | Steps 1 to 2 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 12. | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionSetupComplete message with rlf-InfoAvailable on Cell 12? | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 17-19 | Steps 4 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 12. Note: The UE performs a TAU procedure, The TAU is accepted with PLMN1 listed as an Equivalent PLMN and the RRC connection is released. | - | - | - | - |
| 20-27 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 12. | - | - | - | - |
| 28 | The SS sends a UEInformationRequest message with rlf-ReportReq set to true. | <-- | UEInformationRequest | - | - |
| 29 | Check: Does the UE transmit a UEInformationResponse message including rlf-Report? | --> | UEInformationResponse | 2 | P |

Table 8.6.6.5.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra-frequency handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.6.5.3.3	Specific message contents

Table 8.6.6.5.3.3-0: Condition for specific message contents in Table 8.6.6.5.3.3-3 and Table 8.6.6.5.3.3-12

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.6.5.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.6.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.6.5.3.3-3: MeasConfig (Table 8.6.6.5.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-4: MeasurementReport (step 4, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-5: RRCConnectionReconfiguration (step 5, Table 8.6.6.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.5.3.3-6: MobilityControlInfo (Table 8.6.6.5.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-7: RRCConnectionReestablishmentRequest (step 6, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the C-RNTI used in the Cell 1 |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-8: RRCConnectionReestablishmentComplete (step 8, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-9: RRCConnectionReconfiguration (step 9, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-10: RRCConnectionSetupComplete (step 16, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | PLMN2 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | native |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| rn-SubframeConfigReq-r10 | Not present |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-10A: Message TRACKING AREA UPDATE ACCEPT (step 17, 8.6.6.5.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.6.5.3.3-11: UEInformationRequest (step 28, Table 8.6.6.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.6.5.3.3-12: UEInformationResponse (step 29, Table 8.6.6.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value | Time from UE receive HO command to the failure |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | C-RNTI used in Cell 1 |  |  |
| rlf-Cause-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value | Time elapsed from connection failure |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.5.3.3-13: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.6.6.6	Handover Failure logging / Logging and reporting / Reporting at intra LTE handover / PLMN list

8.6.6.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having detected handover failure }

ensure that {
  when { UE handovers to another cell that belongs to a PLMN which is different from the PLMN where the handover failure was detected but included in the plmn_IdentityList stored in VarRLF-Report }

then { UE transmits the RRCConnectionReconfigurationComplete with IE rlf-InfoAvailable included }

}

(2)

with { UE indicated the availability of handover failure information in RRCConnectionReconfigurationComplete message and the RPLMN is included in plmn-IdentityList stored in VarRLF-Report }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE transmits a UEInformationResponse message including rlf-Report }

}

8.6.6.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.4, 5.3.5.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.4 (TP1, TP2)]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

[TS 36.331, clause 5.3.5.6 (TP1, TP2)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.6.3	Test description

8.6.6.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.6.6.3.1-1.

Table 8.6.6.6.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1, 2 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

- The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.6.6.3.3-14.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.6.3.2	Test procedure sequence

Table 8.6.6.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.6.6.6.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.6.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | "Off" | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). (NOTE 1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | "Off" | (NOTE 1) |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | -73 | The power level values are such that measurement results for Cell 2 (M2) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M2). (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.6.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra and inter frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in Table 8.6.6.6.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 5A | The SS changes Cell 1 parameters according to the row "T2" in Table 8.6.6.5.3.2-1. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.6.6.3.2-3 should take place. | - | - | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell 1, Cell 2 and Cell 12 parameters according to the row "T3" in Table 8.6.6.6.3.2-1. | - | - | - | - |
| 12 | The UE transmits a MeasurementReport message on Cell 2 to report event A3 with the measured RSRP, RSRQ value for Cell 12. | --> | MeasurementReport | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message on Cell 2 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 12 with rlf-InfoAvailable included? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 14A | The UE transmits a Tracking Area Update Request message on Cell 12. | --> | NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 14B | The SS transmits a Tracking Area Update Accept. Note: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | <-- | NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 14C | The UE transmits a Tracking Area Update Complete | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 15 | The SS sends a UEInformationRequest message with rlf-ReportReq set to true. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE send a UEInformationResponse message including rlf-Report? | --> | UEInformationResponse | 2 | P |

Table 8.6.6.6.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra-frequency handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.6.6.3.3	Specific message contents

Table 8.6.6.6.3.3-0: Condition for specific message contents in Table 8.6.6.6.3.3-3, Table 8.6.6.6.3.3-10b and Table 8.6.6.6.3.3-13

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.6.6.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.6.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.6.6.3.3-3: MeasConfig (Table 8.6.6.6.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| reportConfigId[2] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-4: MeasurementReport (step 4, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-5: RRCConnectionReconfiguration (step 5, Table 8.6.6.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.6.3.3-6: MobilityControlInfo (Table 8.6.6.6.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-7: RRCConnectionReestablishmentRequest (step 6, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the C-RNTI used in the Cell 1 |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-8: RRCConnectionReestablishmentComplete (step 8, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-9: RRCConnectionReconfiguration (step 9, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-10: MeasurementReport (step 12, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPCell SEQUENCE { |  | Cell 2 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-10a: RRCConnectionReconfiguration (step 13, Table 8.6.6.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.6.3.3-10b: MobilityControlInfo (Table 8.6.6.6.3.3-10a)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-11: RRCConnectionReconfigurationComplete (step 14, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r10 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.6.6.3.3-11A: Message TRACKING AREA UPDATE ACCEPT (step 14B, 8.6.6.6.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.6.6.3.3-12: UEInformationRequest (step 15, Table 8.6.6.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.6.6.3.3-13: UEInformationResponse (step 16, Table 8.6.6.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value | Time from UE receive HO command to the failure |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | C-RNTI used in Cell 1 |  |  |
| rlf-Cause-r11 | Any value |  |  |
| timeSinceFailure-r11 | Any allowed value | Time elapsed from connection failure |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.6.3.3-14: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.6.6.7	Handover Failure logging / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list

8.6.6.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having detected handover failure }

ensure that {
  when { UE re-establishes to a cell that belongs to a PLMN which is different from the PLMN where the handover failure was detected but included in the plmn_IdentityList stored in VarRLF-Report }

then { UE transmits the RRCConnectionReestablishmentComplete with IE rlf-InfoAvailable included }

}

(2)

with { UE indicated the availability of handover failure information in RRCConnectionReestablishmentComplete message and the RPLMN is included in plmn-IdentityList stored in VarRLF-Report }

ensure that {

when { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

then { UE transmits a UEInformationResponse message including rlf-Report }

}

8.6.6.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.5 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.6 (TP1, TP2)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.5 (TP1)]

The UE shall:

…

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include the rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include the connEstFailInfoAvailable;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

1>	if SystemInformationBlockType15 is broadcast by the PCell:

2>	if the UE has transmitted an MBMSInterestIndication message during the last 1 second preceding detection of radio link failure:

3>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

3>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

3>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

1>	the procedure ends;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.6.7.3	Test description

8.6.6.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 12

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.6.6.7.3.1-1.

Table 8.6.6.7.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.6.7.3.3-12.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.6.7.3.2	Test procedure sequence

Table 8.6.6.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1"are applied at the points indicated in the Main behaviour description in Table 8.6.6.7.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.6.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 8.6.6.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 12 parameters according to the row "T1" in Table 8.6.6.7.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 5A | The SS changes Cell 1 parameters according to the row "T2" in Table 8.6.6.7.3.2-1 | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.6.7.3.2-3 should take place. | - | - | - | - |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 12. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 12. | <-- | RRCConnectionReestablishment | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 12? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 9 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 12. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10A | The UE transmits a Tracking Area Update Request message on Cell 12. | --> | NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 10B | The SS transmits a Tracking Area Update Accept. Note: The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | <-- | NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 10C | The UE transmits a Tracking Area Update Complete | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 11 | The SS sends a UEInformationRequest message with rlf-ReportReq set to true. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit a UEInformationResponse message including rlf-Report? | --> | UEInformationResponse | 2 | P |

Table 8.6.6.7.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-frequency handover using MAC Random Access Preamble on Cell 12. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.6.7.3.3	Specific message contents

Table 8.6.6.7.3.3-0: Condition for specific message contents in Table 8.6.6.7.3.3-3, Table 8.6.6.7.3.3-6 and Table 8.6.6.7.3.3-11

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.6.7.3.3-1: SystemInformationBlockType2 for Cell 12 (preamble and all steps, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.6.7.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.6.7.3.3-3: MeasConfig (Table 8.6.6.7.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-4: MeasurementReport (step 4, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 12 | Cell 12 |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-5: RRCConnectionReconfiguration (step 5, Table 8.6.6.7.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.6.7.3.3-6: MobilityControlInfo (Table 8.6.6.7.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12. |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-7: RRCConnectionReestablishmentRequest (step 6, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the C-RNTI used in the Cell 1 |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-8: RRCConnectionReestablishmentComplete (step 8, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-9: RRCConnectionReconfiguration (step 9, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-9A: Message TRACKING AREA UPDATE ACCEPT (step 10B, 8.6.6.7.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.6.7.3.3-10: UEInformationRequest (step 11, Table 8.6.6.7.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.6.7.3.3-11: UEInformationResponse (step 12, Table 8.6.6.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) | Cell 1 |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value | Time from UE receive HO command to the failure |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | C-RNTI used in Cell 1 |  |  |
| rlf-Cause-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value | Time elapsed from connection failure |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.6.7.3.3-12: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

### 8.6.7	Inter-RAT Logged Handover Failure

#### 8.6.7.1	Handover Failure logging / Reporting of UTRAN Inter-RAT measurements

8.6.7.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the UTRA measurement and reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for UTRA neighbour cell }

}

8.6.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.4, 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.7.1.3	Test description

8.6.7.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.7.1.3.2	Test procedure sequence

Table 8.6.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause

Table 8.6.7.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | -63 | - | The power level values are such that entering conditions for event A3 and event B2 are not satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -88 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -88 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -84 | -87 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -64 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -64 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -80 | -74 | - | The power level values are such that entering conditions for event A3 are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | -64 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -64 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -74 | - | Only Cell 2 is available. (NOTE 1,NOTE 2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | - | "Off" |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | "Off" |  |
| NOTE 1: Power level "Off" for UTRA cell is defined in TS 34.108 Table 6.1.4. NOTE 2: Power level "Off" for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 5 parameters according to the row "T1" in Table 8.6.7.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 2 and Cell 5 parameters according to the row "T2" in Table 8.6.7.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.6.7.1.3.2-3 should take place. | - | - | - | - |
| 8 | The SS changes Cell 1 parameter according to the row "T3" in Table 8.6.7.1.3.2-1. | - | - | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 10 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 11 | The UE transmits an RRCConnectionReestablishmentComplete message with handover failure information on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 16 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

Table 8.6.7.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.7.1.3.3	Specific message contents

Table 8.6.7.1.3.3-0: Conditions for specific message contents
in Tables 8.6.7.1.3.3-3, 8.6.7.1.3.3-9 and 8.6.7.1.3.3-14

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.7.1.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.7.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.7.1.3.3-3: MeasConfig (Table 8.6.7.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f8 |  |  |
| measObject[2] | MeasObjectUTRA-f8 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 2 entries |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| reportConfigId[2] | IdReportConfig-B2-UTRA |  |  |
| reportConfig[2] | ReportConfigInterRAT-B2-UTRA(-72, -76) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f8 |  |  |
| reportConfigId[2] | IdReportConfig-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.7.1.3.3-4: QuantityConfig (Table 8.6.7.1.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-3A, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig SEQUENCE { |  |  |  |
| quantityConfigUTRA SEQUENCE { |  |  |  |
| measQuantityUTRA-FDD | cpich-RSCP |  | UTRA-FDD |
| measQuantityUTRA-TDD | pccpch-RSCP |  | UTRA-TDD |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| quantityConfigUTRA-v1020 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.7.1.3.3-5: MeasObjectUTRA-f8 (Table 8.6.7.1.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Same downlink ARFCN as used for Cell 5 |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListUTRA-FDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-FDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| cellsToAddModListUTRA-TDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-TDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| csg-allowedReportingCells-v930 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.7.1.3.3-6: MeasurementReport (step 4, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 5 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 5 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | Not present or (-5..91) |  |  |
| utra-EcN0 | Not present |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.7.1.3.3-7: MeasurementReport (step 6, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCellSEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-8: RRCConnectionReconfiguration (step 7, Table 8.6.7.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.7.1.3.3-9: MobilityControlInfo (Table 8.6.7.1.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| carrierFreq-v9e0[1] | Not present |  | Band > 64 |
| } |  |  |  |

Table 8.6.7.1.3.3-10: RRCConnectionReestablishmentRequest (step 9, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-11: RRCConnectionReestablishmentComplete (step 11, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-12: RRCConnectionReconfiguration (step 12, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-13: UEInformationRequest (step 14, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.1.3.3-14: UEInformationResponse (step 15, Table 8.6.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] CHOICE { |  |  |  |
| fdd | Physical cell Identity of Cell 5 |  | UTRA-FDD |
| tdd | Physical cell Identity of Cell 5 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | Not present or (-5..91) |  |  |
| utra-EcN0 | Not present or (0..49) |  | UTRA-FDD |
|  | Not present |  | UTRA-TDD |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| basicFields-r11 | Not present or any allowed value |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### 8.6.7.2	Handover Failure logging / Reporting of GERAN Inter-RAT measurements

8.6.7.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the GERAN measurement and reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for GERAN neighbour cell }

}

8.6.7.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.4, 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.5.6(TP1)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 2:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.4(TP1)]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

...

1>	set the reestablishmentCause as follows:

...

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value handoverFailure;

[TS 36.331, clause 5.3.7.5(TP1)]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3(TP1)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.7.2.3	Test description

8.6.7.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 24 - Cell 1 and Cell 2 are E-UTRAN cell, Cell 24 is a GERAN cell.

-	All cells belong to the same PLMN.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.7.2.3.2	Test procedure sequence

Table 8.6.7.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause

Table 8.6.7.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | -63 | - | The power level values are such that entering conditions for event A3 and event B2 are not satisfied. |
|  | RSSI | dBm |  |  | -85 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | -83 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | RSSI | dBm |  |  | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -80 | -74 | - | The power level values are such that entering conditions for event A3 are satisfied. (NOTE 1) |
|  | RSSI | dBm | - | - | "Off" |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -74 | - | Only Cell 2 is available. (NOTE 1,NOTE 2) |
|  | RSSI | dBm | - | - | "Off" |  |
| NOTE 1: Power level "Off" for GERAN cell is defined in TS 36.508 Table 6.2.2.1-1. NOTE 2: Power level "Off" for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.7.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 5 seconds to allow UE to measure serving and neighbour cells correctly after SS changes Cell 1, Cell 2 and Cell 24 parameters according to the row "T0" in Table 8.6.7.2.3.2-1 | - | - | - | - |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 24 parameters according to the row "T1" in Table 8.6.7.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 2 and Cell 24 parameters according to the row "T2" in Table 8.6.7.2.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.6.7.2.3.2-3 should take place. | - | - | - | - |
| 8 | The SS changes Cell 1 parameter according to the row "T3" in Table 8.6.7.2.3.2-1. | - | - | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 10 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 11 | The UE transmits an RRCConnectionReestablishmentComplete message with handover failure information on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 16 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

Table 8.6.7.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.7.2.3.3	Specific message contents

Table 8.6.7.2.3.3-0: Conditions for specific message contents
in Tables 8.6.7.2.3.3-3, and 8.6.7.2.3.3-14

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.7.2.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.7.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.7.2.3.3-3: MeasConfig (Table 8.6.7.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| measObject[2] | MeasObjectGERAN-GENERIC(f11) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 2 entries |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| reportConfigId[2] | IdReportConfig-B2- GERAN |  |  |
| reportConfig[2] | ReportConfigInterRAT-B2-GERAN(-69, [-79]) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| reportConfigId[2] | IdReportConfig-B2-GERAN |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Derivation Path: 36.508, Table 4.6.6-1, condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObjectId[2] | IdMeasObject-f11 |  |  |
| measObject[2] | MeasObjectGERAN-GENERIC(f11) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN(-69, [-79]) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfig-B2-GERAN |  |  |
| } |  |  |  |
| quantityConfig SEQUENCE { |  |  |  |
| quantityConfigGERAN SEQUENCE { |  |  |  |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GERAN | For inter-RAT measurements with GERAN |

Table 8.6.7.2.3.3-5: MeasObjectGERAN-GENERIC(f11) (Table 8.6.7.2.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-2A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectGERAN-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreqs SEQUENCE { |  |  |  |
| startingARFCN | Same downlink ARFCN as used for Cell 24 |  |  |
| bandIndicator | The same band indicator of the Cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Same ARFCN used for Cell24 |  |  |
| } |  |  |  |
| } |  |  |  |
| offsetFreq | Not present |  |  |
| ncc-Permitted | Not present |  |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-6: MeasurementReport (step 4, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListGERAN SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] SEQUENCE { |  |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 24 |  |  |
| } |  |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | (0..63) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-7: MeasurementReport (step 6, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCellSEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-8: RRCConnectionReconfiguration (step 7, Table 8.6.7.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.7.2.3.3-9: MobilityControlInfo (Table 8.6.7.2.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-10: RRCConnectionReestablishmentRequest (step 9, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-11: RRCConnectionReestablishmentComplete (step 11, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-12: RRCConnectionReconfiguration (step 12, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-13: UEInformationRequest (step 14, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.2.3.3-14: UEInformationResponse (step 15, Table 8.6.7.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN--r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink ARFCN as used for Cell 24 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | Physical cell Identity of Cell 24 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | (0..63) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultList UTRA-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.7.3	Handover Failure logging / Reporting of CDMA2000 Inter-RAT measurements

8.6.7.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the CDMA2000 measurement and reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for CDMA neighbour cell

}

}

8.6.7.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.3.7.4, 5.3.7.5 and 5.6.5.3.

[TS 36.331, clause 5.3.5.6(TP1)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 2:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.4(TP1)]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

...

1>	set the reestablishmentCause as follows:

...

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value handoverFailure;

...

[TS 36.331, clause 5.3.7.5(TP1)]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.5.3(TP1)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.7.3.3	Test description

8.6.7.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 15- Cell 1 and Cell 2 are E-UTRAN cell, Cell 15 is a HRPD cell.

-	All cells belong to the same PLMN.

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.7.3.3.2	Test procedure sequence

Table 8.6.7.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.7.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | -63 | - | The power level values are such that entering conditions for event A3 and event B2 are not satisfied. |
|  | Îor/Ioc | dB | - | - | -20 |  |
|  | Ioc | dBm/1.23 MHz | - | - | -55 |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | - | -20 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -80 | -83 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | Îor/Ioc | dB | - | - | -5 |  |
|  | Ioc | dBm/1.23 MHz | - | - | -55 |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | - | -6 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -80 | -74 | - | The power level values are such that entering conditions for event A3 are satisfied. |
|  | Îor/Ioc | dB | - | - | "Off" |  |
|  | Ioc | dBm/1.23 MHz | - | - | "Off" |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | - | "Off" |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -74 | - | Only Cell 2 is available. (NOTE 2) |
|  | Îor/Ioc | dB | - | - | "Off" |  |
|  | Ioc | dBm/1.23 MHz | - | - | "Off" |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | - | "Off" |  |
| NOTE 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. NOTE 2: Power level "Off" for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.7.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 15 parameters according to the row "T1" in Table 8.6.7.3.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS changes Cell 2 and Cell 15 parameters according to the row "T2" in Table 8.6.7.3.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.6.7.3.3.2-3 should take place. | - | - | - | - |
| 8 | The SS changes Cell 1 parameter according to the row "T3" in Table 8.6.7.3.3.2-1. | - | - | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 10 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 11 | The UE transmits an RRCConnectionReestablishmentComplete message with handover failure information on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 16 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

Table 8.6.7.3.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.7.3.3.3	Specific message contents

Table 8.6.7.3.3.3-0: Conditions for specific message contents
in Tables 8.6.7.3.3.3-3, 8.6.7.3.3.3-9 and 8.6.7.3.3.3-14

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.7.3.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.6.7.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.7.3.3.3-3: MeasConfig (Table 8.6.7.3.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f14 |  |  |
| measObject[2] | MeasObjectCDMA2000-f14 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 2 entries |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| reportConfigId[2] | IdReportConfig-B2-CDMA2000 |  |  |
| reportConfig[2] | ReportConfigInterRAT-B2-CDMA2000(-69, -18) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f14 |  |  |
| reportConfigId[2] | IdReportConfig-B2-CDMA2000 |  |  |
| } |  |  |  |
| quantityConfig |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-4: QuantityConfig (Table 8.6.7.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-3A, condition CDMA2000 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig SEQUENCE { |  |  |  |
| quantityConfigCDMA2000 SEQUENCE { |  |  |  |
| measQuantityCDMA2000 | pilotStrength |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-5: MeasObjectCDMA2000-f14 (Table 8.6.7.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCDMA2000 ::= SEQUENCE { |  |  |  |
| cdma2000-Type | typeHRPD |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| bandClass | Band Class of f14 |  |  |
| arfcn | f14 |  |  |
| } |  |  |  |
| searchWindowSize | 15 |  |  |
| offsetFreq | 0dB |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListCDMA2000 SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  |  |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| } |  |  |  |
| } |  |  |  |
| cellForWhichToReportCGI | 50 |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-6: MeasurementReport (step 4, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultsCDMA2000 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| preRegistrationStatusHRPD |  |  |  |
| measResultListCDMA2000 ::=SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-7: MeasurementReport (step 6, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCellSEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-8: RRCConnectionReconfiguration (step 7, Table 8.6.7.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.7.3.3.3-9: MobilityControlInfo (Table 8.6.7.3.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| carrierFreq-v9e0[1] | Not present |  | Band > 64 |
| } |  |  |  |

Table 8.6.7.3.3.3-10: RRCConnectionReestablishmentRequest (step 9, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-11: RRCConnectionReestablishmentComplete (step 11, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-12: RRCConnectionReconfiguration (step 12, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-13: UEInformationRequest (step 14, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.3.3.3-14: UEInformationResponse (step 15, Table 8.6.7.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsCDMA2000-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink ARFCN as used for Cell 15 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| preRegistrationStatusHRPD |  |  |  |
| measResultListCDMA2000 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.7.4	Handover Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list

8.6.7.4.1	Test Purpose (TP)

(1)

with { UE selects the UTRAN cell and enters UTRA CELL_DCH(PS-DCCH+DTCH_DCH) after detection of handover failure in an E-UTRAN cell in RRC_CONNECTED }

ensure that {
  when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message and UE selects the EPLMN which is not the RPLMN }

then { UE transmits an RRCConnectionReconfigurationComplete message containing rlf-InfoAvailable and enters E-UTRA RRC_CONNECTED state }

}

(2)

with { UE in RRC_CONNECTED having reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq set to true }

then { UE transmits a UEInformationResponse message including rlf-Report }

}

8.6.7.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.4.2.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.6 (TP1, TP2)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.4.2.3 (TP1)]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

...

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

...

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.7.4.3	Test description

8.6.7.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 9 and Cell 12

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

Table 8.6.7.4.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 9 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.7.4.3.3-1

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.7.4.3.2	Test procedure sequence

Table 8.6.7.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.6.7.4.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.7.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 9 | Cell12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M1). (NOTE 1) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | "Off" | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | "Off" | - |  |
| T1A | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | -79 | Only Cell 12 is available. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | "Off" | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | "Off" | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | -85 | SnonServingCell, Cell9 > Thresh Cell9,high (NOTE 2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -65 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -67 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | “Off” | - | -70 | The power level values are such that entering conditions for event 3a are satisfied. (NOTE 2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -85 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 | - |  |
| NOTE 1: Power level "Off" for UTRA cell is defined in TS 34.108 Table 6.1.4 and Table 6.1.9. NOTE 2: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.7.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 12 parameters according to the row "T1" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 5A the steps specified in Table 8.6.7.4.3.2-3 should take place. | - | - | - | - |
| 5A | The SS changes Cell 1 and Cell 12 parameter according to the row "T1A" in Table 8.6.7.4.3.2-1. |  |  |  |  |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 12. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 12. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 12. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 12. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 12. | - | - | - | - |
| 10B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 10C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 11 | The SS transmits an RRCConnectionRelease message on Cell 12. | <-- | RRCConnectionRelease | - | - |
| 12 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 13 | The SS changes Cell 1, Cell 12 and Cell 9 parameters according to the row "T2" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 14 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 9. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 14Aa1 to 14Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 14Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 14Aa1a1-14Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 14Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 14Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 14Aa1b1 | The 6s timer expires | - | - | - | - |
| 15-19 | Step 15 to 19 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 9 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 20a1 to 20a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, go to Step 21. | - | - | - | - |
| 20a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 9 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 20a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 9. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 21 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 9. | <-- | MEASUREMENT CONTROL | - | - |
| 22 | The SS changes Cell 9 and Cell 12 parameters according to the row "T3" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 23 | The UE transmits a MEASUREMENT REPORT message on Cell 9 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 24 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 9. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 25 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with handover failure information on Cell 12? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 26 | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 12. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | - | - | - | - |
| - | EXCEPTION: Steps 26Aa1 to 26Aa2b1 describe optional behaviour if UE sends IMS deregistration at steps 14Aa1 to 14Aa1b1 | - | - | - | - |
| 26Aa1-26Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 27 | The SS transmits a UEInformationRequest message on Cell 12. | <-- | UEInformationRequest | - | - |
| 28 | Check: Does the UE transmit a UEInformationResponse message on Cell 12? | --> | UEInformationResponse | 2 | P |
| 29 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 12? | - | - | 2 | - |

Table 8.6.7.4.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-frequency handover using MAC Random Access Preamble on Cell 12. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.7.4.3.3	Specific message contents

Table 8.6.7.4.3.3-0: Conditions for specific message contents
in Tables 8.6.7.4.3.3-4, 8.6.7.4.3.3-7, 8.6.7.4.3.3-14 and Table 8.6.7.4.3.3-18

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.7.4.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.7.4.3.3-2: SystemInformationBlockType2 for Cell 12 (preamble and all steps, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-3: RRCConnectionReconfiguration (step 1, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.7.4.3.3-4: MeasConfig (Table 8.6.7.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-5: MeasurementReport (step 4, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 12 | Cell 12 |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-6: RRCConnectionReconfiguration (step 5, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.7.4.3.3-7: MobilityControlInfo (Table 8.6.7.4.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| … |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-8: RRCConnectionReestablishmentRequest (step 6, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the C-RNTI used in the Cell 1 |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-9: RRCConnectionReestablishmentComplete (step 8, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-10: RRCConnectionReconfiguration (step 9, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-11: MEASUREMENT CONTROL (step 21, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -80 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.6.7.4.3.3-12: HANDOVER FROM UTRAN COMMAND (step 24, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.6.7.4.3.3-13: RRCConnectionReconfiguration (Table 8.6.7.4.3.3-12)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.6.7.4.3.3-14: MobilityControlInfo (Table 8.6.7.4.3.3-13)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 12 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 12 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.6.7.4.3.3-15: SecurityConfigHO (Table 8.6.7.4.3.3-14)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-16: RRCConnectionReconfigurationComplete (step 25, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present or any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-16A: Message TRACKING AREA UPDATE ACCEPT (step 26, 8.6.7.4.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.7.4.3.3-17: UEInformationRequest (step 27, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.7.4.3.3-18: UEInformationResponse (step 28, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) | Cell 1 |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value | Time from UE receive HO command to the failure |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | C-RNTI used in Cell 1 |  |  |
| rlf-Cause-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any value | Time elapsed from connection failure |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-19: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 10B, Table 8.6.7.4.3.2-2))

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.7.4.3.3-20: SystemInformationBlockType6 for Cell12 (preamble, and all steps, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

### 8.6.8	Connection Establishment Failure

#### 8.6.8.1	Connection Establishment Failure logging / Logging and reporting / T300 expiry

8.6.8.1.1	Test Purpose (TP)

(1)

with { UE having sent an RRCConnectionRequest message }

ensure that {
  when { T300 is expired and random access procedure failed }

then { UE stores the connection establishment failure information }

}

(2)

with { UE having sent an RRCConnectionSetupComplete message with connEstFailInfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with connEstFailReport }

}

8.6.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4 and 5.3.3.6. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

8.6.8.1.3	Test description

8.6.8.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.8.1.3.2	Test procedure sequence

Table 8.6.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 1A | The SS is configured to not transmit contention resolution in step 2 RACH procedure. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 6 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | 1 | P |
| 8-11 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE send a UEInformationResponse message with connEstFailReport? | --> | UEInformationResponse | 2 | P |

8.6.8.1.3.3	Specific message contents

Table 8.6.8.1.3.3-1: RRCConnectionSetupComplete (step 7, Table 8.6.8.1.3.2-1)

| Derivation Path: TS 36.508 Table 4.6.1-18 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |  |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| connEstFailInfoAvailable-r11 |  | true |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.8.1.3.3-2: UEInformationRequest (step 12, Table 8.6.8.1.3.2-1)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.8.1.3.3-3: UEInformationResponse (step 13, Table 8.6.8.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.8.2	Connection Establishment Failure logging / Logging and reporting / Reporting at intra-LTE handover

8.6.8.2.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an Handover procedure }

then { UE sends an RRCConnectionReconfigurationComplete message with connEstFailInfoAvailable }

}

8.6.8.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.3.5.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

8.6.8.2.3	Test description

8.6.8.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.8.2.3.2	Test procedure sequence

Table 8.6.8.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.8.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 |  |

Table 8.6.8.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 1A | The SS is configured to not transmit contention resolution in step 2 RACH procedure. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - |  | - | - |
| 4 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) | - | - |
| 5 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 7 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | - | - |
| 8-11 | Steps 6 to 9 the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - |  | - | - |
| 12 | The SS transmits a RRCConnectionReconfiguration message | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits a RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | The SS changes cell 1 and cell 3 parameters according to the row "T1"in table 8.6.8.2.3.2-1 | - |  | - | - |
| 15 | The UE transmits a MeasurementReport message. | --> | MeasurementReport | - | - |
| 16 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | The UE transmit an RRCConnectionReconfigurationComplete message containing connEstFailInfoAvailable to Cell 3. | --> | RRCConnectionReconfigurationCcomplete | 1 | P |

8.6.8.2.3.3	Specific message contents

Table 8.6.8.2.3.3-0: Condition for specific message contents in Table 8.6.8.2.3.3-3 and Table 8.6.6.1.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.8.2.3.3-1: RRCConnectionSetupComplete (step 7, Table 8.6.8.2.3.2-2)

| Derivation Path: TS 36.508 Table 4.6.1-18 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |  |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| connEstFailInfoAvailable-r11 |  | true |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.8.2.3.3-2: RRCConnectionReconfiguration (step 12, Table 8.6.8.2.3.2-2)

Derivation Path: 36.508 clause 4.6.1-8 Condition MEAS

Table 8.6.8.2.3.3-3: MeasConfig (Table 8.6.8.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0[2] | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.2.3.3-4: MeasurementReport (step 15, Table 8.6.8.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.2.3.3-5: RRCConnectionReconfiguration (step 16, Table 8.6.8.2.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8 Condition HO

Table 8.6.8.2.3.3-6: MobilityControlInfo (Table 8.6.8.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.2.3.3-7: RRCConnectionReconfigurationComplete (step 17, Table 8.6.8.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-v1130-IEs SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| connEstFailInfoAvailable-r11 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.8.3	Connection Establishment Failure logging / Logging and reporting / Reporting at RRC connection re-establishment

8.6.8.3.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an RRC Connection re-establishment procedure }

then { UE sends an RRCConnectionReestablishmentComplete message with connEstFailInfoAvailable }

}

8.6.8.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.3.7.5. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.3.7.5]

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

…

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include the rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include the connEstFailInfoAvailable;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

8.6.8.3.3	Test description

8.6.8.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

8.6.8.3.3.2	Test procedure sequence

Table 8.6.8.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.8.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -80 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -80 | Only Cell 2 is available. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.8.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 1A | The SS is configured to not transmit contention resolution in step 2 RACH procedure. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 4-11 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 12 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.8.3.3.2-1. | - | - | - | - |
| 13 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 14 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 15 | Check: Does the UE send an RRCConnectionReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? | --> | RRCConnectionReestablishmentComplete | 1 | P |
| 16 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |

8.6.8.3.3.3	Specific message contents

Table 8.6.8.3.3.3-1: RRCConnectionSetupComplete (step 7, Table 8.6.8.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.3.3.3-2: RRCConnectionReestablishmentRequest (step 13, Table 8.6.8.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.3.3.3-3: RRCConnectionReestablishmentComplete (step 15, Table 8.6.8.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.8.4	Connection Establishment Failure logging / Logging and reporting / Location Information

8.6.8.4.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with location information }

ensure that {
  when { UE receives the UEInformationRequest message containing connEstFailReportReq }

then { UE sends the UEInformationResponse message containing connEstFailReport with locationCoordinates }

}

8.6.8.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.306, clause 4.3.13.2; TS 36.331, clauses 5.3.3.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.306, clause 4.3.13.2]

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.8.4.3	Test description

8.6.8.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25], if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.8.4.3.2	Test procedure sequence

Table 8.6.8.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 1A | The SS is configured to not transmit contention resolution in step 2 RACH procedure. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 4-11 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE send a UEInformationResponse message with connEstFailReport with the IE locationInfo-r11 is present? | --> | UEInformationResponse | 1 | P |

8.6.8.4.3.3	Specific message contents

Table 8.6.8.4.3.3-1: RRCConnectionSetupComplete (step 7, Table 8.6.8.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.4.3.3-2: UEInformationRequest (step 12, Table 8.6.8.4.3.2-1)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.8.4.3.3-3: UEInformationResponse (step 13, Table 8.6.8.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 SEQUENCE { |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |
| ellipsoid-Point-r10 | Any allowed value |  |  |
| ellipsoidPointWithAltitude-r10 | Any allowed value |  |  |
| ellipsoidPointWithUncertaintyCircle-r11 | Any allowed value |  |  |
| ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11 | Any allowed value |  |  |
| ellipsoidArc-r11 | Any allowed value |  |  |
| polygon-r11 | Any allowed value |  |  |
| } |  |  |  |
| horizontalVelocity-r10 | Any allowed value |  |  |
| gnss-TOD-msec-r10 | Any allowed value |  |  |
| } |  |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.8.5	Connection Establishment Failure logging / Logging and reporting / Reporting of Intra-frequency measurements

8.6.8.5.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the intra-frequency measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for intra-frequency neighbouring cell }

}

8.6.8.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.8.5.3	Test description

8.6.8.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

8.6.8.5.3.2	Test procedure sequence

Table 8.6.8.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.8.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE (FDD/TDD) | dBm/15kHz | -85 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 2. |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 |  |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.8.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.8.5.3.2-1. | - | - | - | - |
| 2 | The SS waits for 40s to ensure that the UE detects intra-frequency cell. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 3A | The SS is configured to not transmit contention resolution in step 4 RACH procedure. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq on Cell 1. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on Cell 1? | --> | UEInformationResponse | 1 | P |

8.6.8.5.3.3	Specific message contents

Table 8.6.8.5.3.3-0: Condition for specific message contents in Table 8.6.8.5.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.8.5.3.3-1: RRCConnectionSetupComplete (step 9, Table 8.6.8.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.5.3.3-2: UEInformationRequest (step 14, Table 8.6.8.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.8.5.3.3-3: UEInformationResponse (step 15, Table 8.6.8.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 SEQUENCE { |  |  |  |
| measResultListEUTRA-r11 SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same as Cell 2 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | Same as Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | Not present or (0..97) |  |  |
| rsrqResult | Not present or (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r11 | Not present |  |  |
| measResultListGERAN-r11 | Not present |  |  |
| measResultsCDMA2000-r11 | Not present |  |  |
| } |  |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.8.6	Connection Establishment Failure logging / Logging and reporting / Reporting of Inter-frequency measurements

8.6.8.6.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the inter-frequency measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for inter-frequency neighbouring cell }

}

8.6.8.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.8.6.3	Test description

8.6.8.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

8.6.8.6.3.2	Test procedure sequence

Table 8.6.8.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.8.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are assigned to satisfy RCell 1 > RCell 3. |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.8.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.8.6.3.2-1. | - | - | - | - |
| 2 | The SS waits for 40s to ensure that the UE detects inter-frequency cell. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 3A | The SS is configured to not transmit contention resolution in step 4 RACH procedure. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq on Cell 1. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on Cell 1? | --> | UEInformationResponse | 1 | P |

8.6.8.6.3.3	Specific message contents

Table 8.6.8.6.3.3-0: Condition for specific message contents in Table 8.6.8.2.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.8.6.3.3-1: RRCConnectionSetupComplete (step 9, Table 8.6.8.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.8.6.3.3-2: UEInformationRequest (step 14, Table 8.6.8.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.8.6.3.3-3: UEInformationResponse (step 15, Table 8.6.8.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 SEQUENCE { |  |  |  |
| measResultListEUTRA-r11 SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same as Cell 3 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | Same as Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | Not present or (0..97) |  |  |
| rsrqResult | Not present or (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r11 | Not present |  |  |
| measResultListGERAN-r11 | Not present |  |  |
| measResultsCDMA2000-r11 | Not present |  |  |
| } |  |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.8.7	Void

#### 8.6.8.8	Void

### 8.6.9	Inter-RAT Connection Establishment Failure

#### 8.6.9.1	Connection Establishment Failure logging / Logging and reporting / Reporting at UTRAN Inter-RAT handover

8.6.9.1.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an RRC Connection reconfiguration procedure at UTRAN Inter-RAT handover }

then { UE sends an RRCConnectionReconfigurationComplete message with connEstFailInfoAvailable }

}

(2)

with { UE having sent an RRCConnectionReconfigurationComplete message with connEstFailInfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with connEstFailReport }

}

8.6.9.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.4.2.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

…

8.6.9.1.3	Test description

8.6.9.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.9.1.3.2	Test procedure sequence

Table 8.6.9.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.6.9.1.3.2-2.

Table 8.6.9.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | - | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH Ec(UTRA LCR TDD) | dBm/1.28 MHz | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.6.9.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 1A | The SS is configured to not transmit contention resolution in step 2 RACH procedure. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 8.6.9.1.3.2-1. | - | - | - | - |
| 5 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 5Aa1 to 5Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 5Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 5Aa1a1-5Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 5Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 5Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 5Aa1b1 | The 6s timer expires | - | - | - | - |
| 6-10 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA HSDPA RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | UTRAN FDD: EXCEPTION: Steps 11a1 to 11a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. UTRAN TDD : go to step 12 | - | - | - | - |
| 11a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 11a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 12 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 13 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.6.9.1.3.2-1. | - | - | - | - |
| 14 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 15 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with connEstFailInfoAvailable on Cell 1 using the security key derived from the new KeNB? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 17 | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| - | EXCEPTION: Steps 17Aa1 to 17Aa2b1 describe optional behaviour if UE sends IMS deregistration at steps 5Aa1 to 5Aa1b1 | - | - | - | - |
| 17Aa1-17Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 18 | The SS transmits a UEInformationRequest message to get connEstFailReportReq on Cell 1. | <-- | UEInformationRequest |  |  |
| 19 | Check: Does the UE transmit a UEInformationResponse message with connEstFailReport on Cell 1? | --> | UEInformationResponse | 2 | P |

8.6.9.1.3.3	Specific message contents

Table 8.6.9.1.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.6.9.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.9.1.3.3-2: Void

Table 8.6.9.1.3.3-3: RRCConnectionReconfigurationComplete (step 16, Table 8.6.9.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| connEstFailInfoAvailable-r11 |  | true |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.9.1.3.3-4: UEInformationRequest (step 18, Table 8.6.9.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.9.1.3.3-5: UEInformationResponse (step 19, Table 8.6.9.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present or any allowed value |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.9.2	Connection Establishment Failure logging / Logging and reporting / Reporting of UTRAN Inter-RAT measurements

8.6.9.2.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the UTRA measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for UTRA neighbouring cell }

}

8.6.9.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.9.2.3	Test description

8.6.9.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.9.2.3.2	Test procedure sequence

Table 8.6.9.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.9.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | Only Cell 1 is available. (NOTE 1) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | "Off" |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | "Off" |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | Cell 1 and Cell 5 are available. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -75 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -78 |  |
| NOTE 1: Power level "Off" for UTRA cell is defined in TS 34.108 Table 6.1.4 and Table 6.1.9. |  |  |  |  |  |

Table 8.6.9.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.9.2.3.2-1. | - | - | - | - |
| 2 | The SS waits for [90s] to ensure that the UE detects UTRA cell. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 3A | The SS is configured to not transmit contention resolution in step 4 RACH procedure. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5 | The SS waits for 2sec (T300 exiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq on Cell 1. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on Cell 1? | --> | UEInformationResponse | 1 | P |

8.6.9.2.3.3	Specific message contents

Table 8.6.9.2.3.3-0 A: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 8.6.9.2.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 26 (52 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.2.3.3-0 B: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 8.6.9.2.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Downlink UARFCN of Cell 5 |  |  |
| cellReselectionPriority[n] | 3 |  |  |
| threshX-High[n] | 5 (10dB) |  |  |
| threshX-Low[n] | 5 (10dB) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.2.3.3-1: RRCConnectionSetupComplete (step 9, Table 8.6.9.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.2.3.3-2: UEInformationRequest (step 14, Table 8.6.9.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.9.2.3.3-3: UEInformationResponse (step 15, Table 8.6.9.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 SEQUENCE { |  |  |  |
| measResultListEUTRA-r11 | Not present |  |  |
| measResultListUTRA-r11 SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same as Cell 5 |  |  |
| measResultList-r9 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | Same as Cell 5 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | Not present or (-5..91) |  |  |
| utra-EcN0 | Not present or (0..49) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListGERAN-r11 | Not present |  |  |
| measResultsCDMA2000-r11 | Not present |  |  |
| } |  |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.9.3	Connection Establishment Failure logging / Logging and reporting / Reporting of GERAN Inter-RAT measurements

8.6.9.3.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the GERAN measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for GERAN neighbouring cell }

}

8.6.9.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.6. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

8.6.9.3.3	Test description

8.6.9.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 24.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.9.3.3.2	Test procedure sequence

Table 8.6.9.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.9.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | Only Cell 1 is available. (NOTE 1) |
|  | RSSI | dBm | - | "Off" |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | Cell 1 and Cell 24 are available. |
|  | RSSI | dBm | - | -85 |  |
| NOTE 1: Power level "Off" for GERAN cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.9.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 parameter according to the row "T1" in Table 8.6.9.3.3.2-1. | - | - | - | - |
| 2 | The SS waits for [30s] to ensure that the UE detects GERAN cell. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 3A | The SS is configured to not transmit contention resolution in step 4 RACH procedure. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq on Cell 1. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on Cell 1? | --> | UEInformationResponse | 1 | P |

8.6.9.3.3.3	Specific message contents

Table 8.6.9.3.3.3-1: RRCConnectionSetupComplete (step 9, Table 8.6.9.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.3.3.3-2: UEInformationRequest (step 14, Table 8.6.9.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.9.3.3.3-3: UEInformationResponse (step 15, Table 8.6.9.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 SEQUENCE { |  |  |  |
| measResultListEUTRA-r11 | Not present |  |  |
| measResultListGERAN-r11 SEQUENCE { | 1 entry |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| arfcn | Same as Cell 24 |  |  |
| bandIndicator | dcs1800 or pcs1900, Same as Cell 24 |  |  |
| } |  |  |  |
| physCellId SEQUENCE { |  |  |  |
| networkColourCode | Same as Cell 24 |  |  |
| baseStationColourCode | Same as Cell 24 |  |  |
| } |  |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsCDMA2000-r11 | Not present |  |  |
| } |  |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| measResultFailedCell-v1250 | Not present or any allowed value |  |  |
| failedCellRSRQ_Type_r12 | Not present or any allowed value |  |  |
| measResultListEUTRA-v1250 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.9.4	Connection Establishment Failure logging / Logging and reporting / Reporting of CDMA2000 Inter-RAT measurements

8.6.9.4.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state with connection establishment failure information available }

ensure that {
  when { UE successfully performs a RRC connection establishment procedure and the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport }

then { UE transmits the RRCConnectionSetupComplete with IE connEstFailInfoAvailable present }

}

(2)

with { UE has connection establishment failure information available with the CDMA2000 measurement result and the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with connEstFailReport containing the measurement result for CDMA2000 neighbouring cell }

}

8.6.9.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.3.6 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.3.6]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	clear the information included in VarConnEstFailReport, if any;

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3>	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3>	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3>	set contentionDetected to indicate whether contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3>	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.9.4.3	Test description

8.6.9.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 15- Cell 1 is E-UTRAN cell, Cell 15 is a HRPD cell.

-	All cells belong to the same PLMN.

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells with content of CDMA2000 HRPD carrier frequency list in SIB8 set as defined in TS 36.508 [18] table 6.3.1.5-1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

8.6.9.4.3.2	Test procedure sequence

Table 8.6.9.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.9.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power level values are such that camping on Cell 1 is guaranteed |
|  | Îor/Ioc | dB | - | "Off" |  |
|  | Ioc | dBm/1.23 MHz | - | "Off" |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | "Off" |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | Cell 15 is detectable |
|  | Îor/Ioc | dB | - | -20 |  |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec/Io (NOTE 1) | dB | - | -20 |  |

Table 8.6.9.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 15 parameter according to the row "T1" in Table 8.6.9.4.3.2-1. | - | - | - | - |
| 2 | The SS waits for 20s to ensure that the UE detects CDMA2000 Cell 15. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 3A | The SS is configured to not transmit contention resolution in step 4 RACH procedure. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 6-13 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | 1 | - |
| 14 | The SS sends a UEInformationRequest message with connEstFailReportReq-r11 set to true on Cell 1. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport-r11 on Cell 1? | --> | UEInformationResponse | 2 | P |

8.6.9.4.3.3	Specific message contents

Table 8.6.9.4.3.3-1: RRCConnectionSetupComplete (step 9, Table 8.6.9.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.4.3.3-2: UEInformationRequest (step 14, Table 8.6.9.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition ConEstFail

Table 8.6.9.4.3.3-3: UEInformationResponse (step 15, Table 8.6.9.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  | Cell 1 |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 SEQUENCE { |  |  |  |
| measResultListEUTRA-r11 | Not present |  |  |
| measResultListUTRA-r11 | Not present |  |  |
| measResultListGERAN-r11 | Not present |  |  |
| measResultsCDMA2000-r11 SEQUENCE (SIZE (1..maxFreq)) OF { | 1 entry |  |  |
| carrierFreq-r9[1] SEQUENCE { | Same as Cell 15 |  |  |
| bandClass | Operating band class under test |  |  |
| arfcn | f14 |  |  |
| } |  |  |  |
| measResultList-r9[1] SEQUENCE { |  |  |  |
| preRegistrationStatusHRPD | false |  |  |
| measResultListCDMA2000 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 15 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| pilotStrength | (0..63) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| numberOfPreamblesSent-r11 | Any allowed value |  |  |
| contentionDetected-r11 | Any allowed value |  |  |
| maxTxPowerReached-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any allowed value |  |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.9.5	Connection Establishment Failure logging / Logging and reporting / Bluetooth measurement collection

8.6.9.5.1	Test Purpose (TP)

(1)

with { UE detects connection establishment failure }

ensure that {

when { UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCConnectionSetupComplete message with logMeasAvailableBT included }

}

(2)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListBT }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends the UEInformationResponse message containing connEstFailReport with Bluetooth measurement collection }

}

(3)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListBT }

ensure that {

when { UE receives the UEInformationRequest message containing connEstFailReportReq and a Bluetooth beacon is not included in bt-NameListConfig }

then { UE sends the UEInformationResponse message not including measurement result of this Bluetooth beacon which is not included in bt-NameListConfig }

}

8.6.9.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.3.6, 5.3.10.9, and 5.6.5.3.

[TS 36.331, clause 5.3.10.9 Other configuration]

The UE shall:

…

1>	if the received otherConfig includes the bt-NameListConfig:

2>	if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

[TS 36.331, clause 5.3.3.6 T300 expiry]

The UE shall:

1> if timer T300 expires:

…

2> except for NB-IoT, store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

[TS 36.331, clause 5.3.3.4 Reception of the RRCConnectionSetup by the UE]

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

…

1> set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

…

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.6.9.5.3	Test description

8.6.9.5.3.1	Pre-test conditions

System Simulator:

- Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 36.508 cl 4.4.9

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.9.5.3.2	Test procedure sequence

Table 8.6.9.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRCConnectionRelease | - | - |
| 4 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 4A | The SS is configured to not transmit contention resolution in step 5 RACH procedure. | - | - | - | - |
| 5 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 6 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 8 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 9 | Check : Does the UE transmits an RRCConnectionSetupComplete message including connEstFailInfoAvailable with logMeasAvailableBT. UE initiates the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC : RRCConnectionSetupComplete NAS : SERVICE REQUEST | 1 | P |
| 10-13 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? Check: Does the UE send a UEInformationResponse message with connEstFailReport without measurement result of Bluetooth beacon not including in bt-NameListConfig (Bluetooth beacon 2)? | --> | UEInformationResponse | 2,3 | P |

8.6.9.5.3.3	Specific message contents

Table 8.6.9.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| OtherConfig-r9 ::= SEQUENCE { |  |  |  |
| powerPrefIndicationConfig-r11 CHOICE {} | Not present |  |  |
| bw-PreferenceIndicationTimer-r14 | Not present | ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} |  |
| rlm-ReportConfig-r14 CHOICE {} | Not present |  |  |
| overheatingAssistanceConfig-r14 CHOICE {} | Not present |  |  |
| ailc-BitConfig-r15 | Not present | BOOLEAN |  |
| bt-NameListConfig-r15 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..maxBT-Name-r15)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r15[1] | Set as per Table 4.4.9-1 of [18] | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| wlan-NameListConfig-r15 CHOICE {} | Not present |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.5.3.3-2: RRCConnectionSetupComplete (step 9, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableBT-r15 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.5.3.3-3: UEInformationRequest (step 14, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23A with condition ConnEstFail |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |

Table 8.6.9.5.3.3-4: UEInformationResponse (step 15, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present |  |  |
| numberOfPreamblesSent-r11 | Any allowed value | INTEGER (1..200) |  |
| contentionDetected-r11 | Any allowed value | BOOLEAN |  |
| maxTxPowerReached-r11 | Any allowed value | BOOLEAN |  |
| timeSinceFailure-r11 | Any allowed value | INTEGER (0..172800) |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| measResultFailedCell-v1250 | Not present |  |  |
| failedCellRSRQ-Type-r12 | Not present |  |  |
| measResultListEUTRA-v1250 | Not present |  |  |
| measResultFailedCell-v1360 | Not present |  |  |
| logMeasResultListBT-r15 SEQUENCE { | 1 entry | Report Bluetooth beacon 1 |  |
| LogMeasResultBT-r15 SEQUENCE { |  |  |  |
| bt-Addr-r15 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r15 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.6.9.6	Connection Establishment Failure logging / Logging and reporting / WLAN measurement collection

8.6.9.6.1	Test Purpose (TP)

(1)

with { UE detects connection establishment failure }

ensure that {

when { UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCConnectionSetupComplete message with logMeasAvailableWLAN included }

}

(2)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListWLAN }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends the UEInformationResponse message containing connEstFailReport with WLAN measurement collection }

}

(3)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListWLAN }

ensure that {

when { UE receives the UEInformationRequest message containing connEstFailReportReq and a WLAN AP is not included in wlan-NameListConfig }

then { UE sends the UEInformationResponse message not including measurement result of this WLAN AP which is not included in wlan-NameListConfig }

}

8.6.9.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.3.6, 5.3.10.9, and 5.6.5.3.

[TS 36.331, clause 5.3.10.9 Other configuration]

The UE shall:

…

1>	if the received otherConfig includes the wlan-NameListConfig:

2>	if wlan-NameListConfig is set to setup, attempt to have WLAN measurement results available for subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

[TS 36.331, clause 5.3.3.6 T300 expiry]

The UE shall:

1> if timer T300 expires:

…

2> except for NB-IoT, store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	f available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

[TS 36.331, clause 5.3.3.4 Reception of the RRCConnectionSetup by the UE]

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

…

1> set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

…

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.6.9.6.3	Test description

8.6.9.6.3.1	Pre-test conditions

System Simulator:

- Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell 27b are configured as per 36.508 cl 4.4.8 with the OFFLOAD condition

- System information combination 22 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.9.6.3.2	Test procedure sequence

Table 8.6.9.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRCConnectionRelease | - | - |
| 4 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 4A | The SS is configured to not transmit contention resolution in step 5 RACH procedure. | - | - | - | - |
| 5 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 6 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 7 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 8 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 9 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 10 | Check: Does the UE transmits an RRCConnectionSetupComplete message including connEstFailInfoAvailable with logMeasAvailableWLAN. UE initiates the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC : RRCConnectionSetupComplete NAS : SERVICE REQUEST | 1 | P |
| 11-14 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 15 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE send a UEInformationResponse message with connEstFailReport with logMeasResultListWLAN including one entry (WLAN AP 1) measurement result? Check: Does the UE send a UEInformationResponse message with connEstFailReport without measurement result of WLAN AP not including in wlan-NameListConfig (WLAN AP 2)? | --> | UEInformationResponse | 2,3 | P |

8.6.9.6.3.3	Specific message contents

Table 8.6.9.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.9.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 with condition OtherConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| OtherConfig-r9 ::= SEQUENCE { |  |  |  |
| powerPrefIndicationConfig-r11 CHOICE {} | Not present |  |  |
| bw-PreferenceIndicationTimer-r14 | Not present |  |  |
| rlm-ReportConfig-r14 CHOICE {} | Not present |  |  |
| overheatingAssistanceConfig-r14 CHOICE {} | Not present |  |  |
| ailc-BitConfig-r15 | Not present |  |  |
| wlan-NameListConfig-r15 CHOICE { |  |  |  |
| setup | WLAN-NameList-r15 |  |  |
| WLAN-NameList-r15 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r15)) OF WLAN-Name-r15 | 1 entry | WLAN AP 1 |  |
| WLAN-Name-r15 ::= | Set as per Table 4.4.8-1 of [18] | OCTET STRING (SIZE (1..32)) Refers to SSID defined in IEEE 802.11-2012 |  |
| } |  |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	The specific message content for the SystemInformationBlockType17 message in Table 8.6.9.6.3.3-1A.

Table 8.6.9.6.3.3-1A: SystemInformationBlockType17 (Preamble and all steps of Table 8.6.9.6.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 2 Entry |  |
| wlan-Identifiers[1] SEQUENCE{ |  |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[1] | Not present |  |  |
| hessid-r12[1] | Not present |  |  |
| } |  |  |  |
| wlan-Identifiers[2] SEQUENCE{ |  |  |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[2] | Not present |  |  |
| hessid-r12[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.6.9.6.3.3-2: RRCConnectionSetupComplete (step 9, Table 8.6.9.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableWLAN-r15 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.6.3.3-3: UEInformationRequest (step 14, Table 8.6.9.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23A with condition ConnEstFail |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |

Table 8.6.9.6.3.3-4: UEInformationResponse (step 15, Table 8.6.9.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | Any allowed value if present |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present |  |  |
| numberOfPreamblesSent-r11 | Any allowed value | INTEGER (1..200) |  |
| contentionDetected-r11 | Any allowed value | BOOLEAN |  |
| maxTxPowerReached-r11 | Any allowed value | BOOLEAN |  |
| timeSinceFailure-r11 | Any allowed value | INTEGER (0..172800) |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| measResultFailedCell-v1250 | Not present |  |  |
| failedCellRSRQ-Type-r12 | Not present |  |  |
| measResultListEUTRA-v1250 | Not present |  |  |
| measResultFailedCell-v1360 | Not present |  |  |
| logMeasResultListBT-r15 | Not checked |  |  |
| logMeasResultListWLAN-r15 ::= SEQUENCE (SIZE (1..maxWLAN-IdReport-r14)) OF LogMeasResultWLAN-r15 | 1 entry | Report WLAN AP 1 |  |
| LogMeasResultWLAN-r15 ::= SEQUENCE { |  |  |  |
| wlan-Identifiers-r15 SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 | OCTET STRING (SIZE (1..32)) |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r15 | Any allowed value | WLAN-RSSI-Range-r13 INTEGER(0..141) |  |
| rtt-WLAN-r15 | Any allowed value if present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 8.6.10	Inter-RAT Immediate MDT

#### 8.6.10.1	Inter-RAT Immediate MDT / Reporting / Location information / Event B2

8.6.10.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement with event B2 configured with includeLocationInfo included in the reportConfig }

ensure that {
  when { Entry condition for event B2 is met and detailed location information that has not been reported is available }

then { UE sends MeasurementReport message with locationInfo included }

}

8.6.10.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.306, clause 4.3.13.2; TS 36.331, clauses 5.5.5 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.306, clause 4.3.13.2]

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

[TS 36.331, clause 5.5.5]

The purpose of this procedure is to transfer measurement results from the UE to E-UTRAN.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if the includeLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of the locationInfo as follows:

2>	include the locationCoordinates;

2>	if available, include the gnss-TOD-msec;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

…

1>	if the logMeasReport is included in the UEInformationResponse:

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.10.1.3	Test description

8.6.10.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 7.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 36.509 [25] , if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.10.1.3.2	Test procedure sequence

Table 8.6.10.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.10.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 7 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -88 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -88 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -84 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -64 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |

Table 8.6.10.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-RAT measurement with includeLocationInfo on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of inter-RAT measurement on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 7 parameters according to the row "T1" in table 8.3.2.3.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report the event B2 for Cell 7 with locationInfo? | --> | MeasurementReport | 1 | P |

8.6.10.1.3.3	Specific message contents

Table 8.6.10.1.3-0: Condition for specific message contents in Table 8.6.10.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.10.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.10.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.10.1.3.3-2: MeasConfig (Table 8.6.10.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f8 |  |  |
| measObject[2] | MeasObjectUTRA-f8 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE{ | 1 entry |  |  |
| reportConfigId | IdReportConfig-B2-UTRA |  |  |
| reportConfig | ReportConfigInterRAT-B2-UTRA(-72, -76) |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId | 1 |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| reportConfigId[1] | IdReportConfig-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.10.1.3.3-2A: QuantityConfig (Table 8.6.10.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-3A, condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig ::= SEQUENCE { |  |  |  |
| quantityConfigUTRA SEQUENCE { |  |  |  |
| measQuantityUTRA-FDD | cpich-RSCP |  | UTRA-FDD |
| measQuantityUTRA-TDD | pccpch-RSCP |  | UTRA-TDD |
| filterCoefficient | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.10.1.3.3-2B: MeasObjectUTRA-f8 (Table 8.6.10.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-3 MeasObjectUTRA-GENERIC(f8) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA -GENERIC(f8) ::= SEQUENCE { |  |  |  |
| carrierFreq | UTRA DL carrier frequency of the cell 7 |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListUTRA-FDD ::= SEQUENCE (SIZE (1.. maxCellMeas)) OF SEQUENCE { |  |  | UTRA-FDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | physicalCellIdentity – Cell 7 |  |  |
| } |  |  |  |
| cellsToAddModListUTRA-TDD ::= SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  | UTRA-TDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | physicalCellIdentity – Cell 7 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.6.10.1.3.3-2C: ReportConfigInterRAT-B2-UTRA (step 1, Table 8.6.1.10.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-8 ReportConfigInterRAT-B2-UTRA(-72, -76) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| includeLocationInfo-r11 | True |  |  |
| } |  |  |  |

Table 8.6.10.1.3.3-3: MeasurementReport (step 4, Table 8.6.10.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE{ |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 7 |  |  |
| cgi-info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r10 SEQUENCE { |  |  |  |
| locationCoordinates-r10 CHOICE { |  |  |  |
| ellipsoid-Point-r10 | Any allowed value |  |  |
| ellipsoidPointWithAltitude-r10 | Any allowed value |  |  |
| ellipsoidPointWithUncertaintyCircle-r11 | Any allowed value |  |  |
| ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11 | Any allowed value |  |  |
| ellipsoidArc-r11 | Any allowed value |  |  |
| polygon-r11 | Any allowed value |  |  |
| } |  |  |  |
| horizontalVelocity-r10 | Not present or ny allowed value |  |  |
| gnss-TOD-msec-r10 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.10.2	Inter-RAT Immediate MDT / Reporting /Bluetooth measurement collection

8.6.10.2.1	Test Purpose (TP)

(1)

with { UE is in RRC_CONNECTED state and supports Bluetooth measurements and receives the ReportConfigEUTRA in RRCConnectionReconfiguration message with includeBT-Meas and measurement configured for event A2 and a Bluetooth beacon is included in BT-NameListConfig }

ensure that {

when { Entry condition for event A2 is met and Bluetooth measurement results that has not been reported is available }

then { UE sends MeasurementReport message with LogMeasResultListBT including Bluetooth measurement results corresponding to BT-NameListConfig included in includeBT-Meas }

}

(2)

with { UE is in RRC_CONNECTED state and supports Bluetooth measurements and receives the ReportConfigEUTRA in RRCConnectionReconfiguration message with includeBT-Meas and measurement configured for event A2 and a Bluetooth beacon is not included in BT-NameListConfig }

ensure that {

when { Entry condition for event A2 is met }

then { UE sends MeasurementReport message with LogMeasResultListBT not including Bluetooth measurement results of the Bluetooth beacon not included in BT-NameListConfig }

}

8.6.10.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.3 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

[TS 36.331, clause 5.5.4.1]

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical is configured in the corresponding reportConfig:

5>	consider only the PCell to be applicable;

4>	else if the reportSSTD-Meas is set to true in the corresponding reportConfig:

5>	consider the PSCell to be applicable;

4>	else if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else if eventC1 or eventC2 is configured in the corresponding reportConfig; or if reportStrongestCSI-RSs is included in the corresponding reportConfig:

5>	consider a CSI-RS resource on the associated frequency to be applicable when the concerned CSI-RS resource is included in the measCSI-RS-ToAddModList defined within the VarMeasConfig for this measId;

4>	else if measRSSI-ReportConfig is configured in the corresponding reportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	else:

5>	if useAllowedCellList is set to TRUE:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the ExcludedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

4>	if the corresponding reportConfig includes alternativeTimeToTrigger and if the UE supports alternativeTimeToTrigger:

5>	use the value of alternativeTimeToTrigger as the time to trigger instead of the value of timeToTrigger in the corresponding reportConfig for cells included in the altTTT-CellsToAddModList of the corresponding measObject;

…

2>	if the triggerType is set to event, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4>	if T312 is not running:

5>	start timer T312 with the value configured in the corresponding measObject;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4>	if T312 is not running:

5>	start timer T312 with the value configured in the corresponding measObject;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCell:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

…

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

…

[TS 36.331, clause 5.5.4.3]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the primary or secondary cell that is configured on the frequency indicated in the associated measObjectEUTRA to be the serving cell;

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within the reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within the reportConfigEUTRA for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the measResults as follows:

2>	if available, include the logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the measResults as follows:

2>	if available, include the logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons;

…

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

8.6.10.2.3	Test description

8.6.10.2.3.1	Pre-test conditions

System Simulator:

- Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 36.508 cl 4.4.9

UE:

None

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18].

8.6.10.2.3.2	Test procedure sequence

Table 8.6.10.2.3.2-1 illustrates the downlink power levels to be applied for Cell 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while row marked "T1"is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.10.2.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 |  |  | Power level is such that Ms > Thresh + Hys |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 |  |  | Power level is such that entry condition for event A2 is satisfied Ms + Hys < Thresh |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |
| Note: The total tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and absolute UE measurement accuracy (TS 36.133 clause 9). |  |  |  |  |  |  |

Table 8.6.10.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A2 with includeLocationInfo configured. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.6.10.2.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A2 with the LogMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | MeasurementReport | 1,2 | P |

8.6.10.2.3.3	Specific message contents

Table 8.6.10.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.10.2.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.6.10.2.3.3-2: MeasConfig (step 1, Table 8.6.10.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| reportConfig[1] | ReportConfig-A2-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.10.2.3.3-3: ReportConfig-A2-H (step 1, Table 8.6.10.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-5 ReportConfigEUTRA-A2(-83) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| hysteresis | 6 | 3 dB |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| includeLocationInfo-r10 | Not present |  |  |
| includeBT-Meas-r15 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..maxBT-Name-r15)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r15[1] | Set as per Table 4.4.9-1 of [18] | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| includeWLAN-Meas-r15 | Not present |  |  |
| } |  |  |  |

Table 8.6.10.2.3.3-4: MeasurementReport (step 4, Table 8.6.10.2.3.2-2)

| Derivation path: 36.508 table clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| logMeasResultListBT-r15 SEQUENCE (SIZE (1..maxBT-IdReport-r15)) OF SEQUENCE { | 1 entry | Report Bluetooth beacon 1 |  |
| bt-Addr-r15 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r15 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| logMeasResultListWLAN-r15 | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.6.10.3	Inter-RAT Immediate MDT / Reporting /WLAN measurement collection

8.6.10.3.1	Test Purpose (TP)

(1)

with { UE is in RRC_CONNECTED state and supports WLAN measurements and receives the ReportConfigEUTRA in RRCConnectionReconfiguration message with includeWLAN-Meas and measurement configured for event A2 and a WLAN node is included in WLAN-NameListConfig}

ensure that {

when { Entry condition for event A2 is met and WLAN measurement results that has not been reported is available }

then { UE sends MeasurementReport message with LogMeasResultListWLAN including WLAN measurement results corresponding to WLAN-NameListConfig included in includeWLAN-Meas }

}

(2)

with { UE is in RRC_CONNECTED state and supports WLAN measurements and receives the ReportConfigEUTRA in RRCConnectionReconfiguration message with includeWLAN-Meas and measurement configured for event A2 and a WLAN node is not included in WLAN-NameListConfig }

ensure that {

when { Entry condition for event A2 is met }

then { UE sends MeasurementReport message with LogMeasResultListWLAN not including WLAN measurement results of the WLAN node not included in WLAN-NameListConfig }

}

8.6.10.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.3 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

[TS 36.331, clause 5.5.4.1]

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical is configured in the corresponding reportConfig:

5>	consider only the PCell to be applicable;

4>	else if the reportSSTD-Meas is set to true in the corresponding reportConfig:

5>	consider the PSCell to be applicable;

4>	else if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else if eventC1 or eventC2 is configured in the corresponding reportConfig; or if reportStrongestCSI-RSs is included in the corresponding reportConfig:

5>	consider a CSI-RS resource on the associated frequency to be applicable when the concerned CSI-RS resource is included in the measCSI-RS-ToAddModList defined within the VarMeasConfig for this measId;

4>	else if measRSSI-ReportConfig is configured in the corresponding reportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	else:

5>	if useAllowedCellList is set to TRUE:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the ExcludedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

4>	if the corresponding reportConfig includes alternativeTimeToTrigger and if the UE supports alternativeTimeToTrigger:

5>	use the value of alternativeTimeToTrigger as the time to trigger instead of the value of timeToTrigger in the corresponding reportConfig for cells included in the altTTT-CellsToAddModList of the corresponding measObject;

…

2>	if the triggerType is set to event, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4>	if T312 is not running:

5>	start timer T312 with the value configured in the corresponding measObject;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4>	if T312 is not running:

5>	start timer T312 with the value configured in the corresponding measObject;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCell:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3> if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

…

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

…

[TS 36.331, clause 5.5.4.3]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the primary or secondary cell that is configured on the frequency indicated in the associated measObjectEUTRA to be the serving cell;

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within the reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within the reportConfigEUTRA for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the measResults as follows:

2>	if available, include the logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the measResults as follows:

2>	if available, include the logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons;

…

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

8.6.10.3.3	Test description

8.6.10.3.3.1	Pre-test conditions

System Simulator:

- Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell27b are configured as per 36.508 cl 4.4.8 with the OFFLOAD condition

UE:

None

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18].

8.6.10.3.3.2	Test procedure sequence

Table 8.6.10.3.3.2-1 illustrates the downlink power levels to be applied for Cell 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.10.3.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 |  |  | Power level is such that Ms > Thresh + Hys |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 |  |  | Power level is such that entry condition for event A2 is satisfied Ms + Hys < Thresh |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |
| Note: The total tolerance used is the sum of downlink signal level uncertainty (TS 36.508 clause 6.2.2.1) and absolute UE measurement accuracy (TS 36.133 clause 9). |  |  |  |  |  |  |

Table 8.6.10.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A2 with includeLocationInfo configured. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.6.10.3.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A2 with the LogMeasResultListWLAN including one entry (WLAN AP 1) measurement result? | --> | MeasurementReport | 1,2 | P |

8.6.10.3.3.3	Specific message contents

Table 8.6.10.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.10.3.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

The specific message content for the SystemInformationBlockType17 message in Table 8.6.10.3.3.3-1A.

Table 8.6.10.3.3.3-1A: SystemInformationBlockType17 (Preamble and all steps of Table 8.6.10.3.3.2-2

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 2 Entry |  |
| wlan-Identifiers[1] SEQUENCE{ |  |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[1] | Not present |  |  |
| hessid-r12[1] | Not present |  |  |
| } |  |  |  |
| wlan-Identifiers[2] SEQUENCE{ |  |  |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[2] | Not present |  |  |
| hessid-r12[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.6.10.3.3.3-2: MeasConfig (step 1, Table 8.6.10.3.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| reportConfig[1] | ReportConfig-A2-H |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A2 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.6.10.3.3.3-3: ReportConfig-A2-H (step 1, Table 8.6.10.3.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-5 ReportConfigEUTRA-A2(-83) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| hysteresis | 6 | 3 dB |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| includeWLAN-Meas-r15 CHOICE { |  |  |  |
| setup SEQUENCE ( SIZE (1..maxWLAN-Name-r15)) OF | 1 entry | WLAN AP 1 |  |
| WLAN-Name-r15[1] | Set the value as per Cell 27 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| ..} |  |  |  |
| } |  |  |  |

Table 8.6.10.3.3.3-4: MeasurementReport (step 4, Table 8.6.10.3.3.2-2)

| Derivation path: 36.508 table clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| logMeasResultListWLAN-r15 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF SEQUENCE { | 1 entry | Report WLAN AP1 |  |
| wlan-Identifiers-r15[1] SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r15[1] | Any allowed value |  |  |
| rtt-WLAN-r15[1] | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.6.11	RACH Optimisation

#### 8.6.11.1	RACH Optimisation

8.6.11.1.1	Test Purpose (TP)

(1)

with { UE is in E-UTRA RRC_CONNECTED state and no contention resolution failure occurred for the last successfully completed random access procedure }

ensure that {
  when { UE receives UEInformationRequest message with rach-ReportReq set to true }

then { UE transmits UEInformationResponse message with a rach-Report including contentionDetected set to false}

}

(2)

with { UE is in E-UTRA RRC_CONNECTED state and contention resolution was not successful for at least one of the transmitted preambles for the last successfully completed random access procedure}

ensure that {
  when { UE receives UEInformationRequest message with rach-ReportReq set to true }

then { UE transmits UEInformationResponse message with a rach-Report including contentionDetected set to true }

}

8.6.11.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

1>	if rach-ReportReq is set to true, set the contents of the rach-Report in the UEInformationResponse message as follows:

2>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the last successfully completed random access procedure;

2>	if contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the last successfully completed random access procedure:

3>	set the contentionDetected to true;

2>	else:

3>	set the contentionDetected to false;

8.6.11.1.3	Test description

8.6.11.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.11.1.3.2	Test procedure sequence

Table 8.6.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UEInformationRequest message. | <-- | UEInformationRequest | - | - |
| 2 | Check: Does the UE transmit a UEInformationResponse message with rach-Report? | --> | UEInformationResponse | 1 | P |
| 3 | SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease |  |  |
| 4 | SS waits for 5s. | - | - | - | - |
| 4A | SS is configured to transmit MAC control element with non-matched UE Contention Resolution Identity for first RACH procedure and responds with matched UE Contention Resolution from second RACH procedure | - | - | - | - |
| 5 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | Paging |  |  |
| 6 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest |  |  |
| 7 | TheUEtransmitsanRRCConnectionRequestmessage. | --> | RRCConnectionRequest- | - | - |
| 7A | Void |  |  |  |  |
| 8 | Void |  |  |  |  |
| 9-16 | Steps 3 to 10 of of generic test procedure in TS 36.508 subclause 6.4.2.2 are performed on Cell 1? | - | - | - | - |
| 17 | The SS transmits a UEInformationRequest message. | <-- | UEInformationRequest | - | - |
| 18 | Check: Does the UE transmit a UEInformationResponse message with rach-Report? | --> | UEInformationResponse | 2 | P |

8.6.11.1.3.3	Specific message contents

Table 8.6.11.1.3.3-1: UEInformationRequest (step 1 and step 17, Table 8.6.11.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rach-ReportReq-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.11.1.3.3-2: UEInformationResponse (step 2, Table 8.6.11.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rach-Report-r9 SEQUENCE { |  |  |  |
| numberOfPreamblesSent-r9 | Any allowed value |  |  |
| contentionDetected-r9 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.11.1.3.3-3: UEInformationResponse (step 18, Table 8.6.11.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rach-Report-r9 SEQUENCE { |  |  |  |
| numberOfPreamblesSent-r9 | Any allowed value |  |  |
| contentionDetected-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 8.7	Automatic Neighbour Relation (ANR) for UTRAN Specific Procedures

### 8.7.1	Inter-RAT / UTRAN ANR measurement, logging and reporting / E-UTRAN cell

8.7.1.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH state }

ensure that {

when { UE receives a LOGGING MEASUREMENT CONFIGURATION message containing Logged ANR configuration Info with E-UTRA Indicator set to TRUE }

then { UE reads “Logged ANR configuration info” and configure UE to perform inter-RAT ANR measurements for E-UTRAN to be reported in the logged ANR report provided to the network in the UE INFORMATION RESPONSE message }

}

(2)

with { UE in E-UTRA RRC IDLE state and T327 timer is running }

ensure that {

when { UE performs cell reselection to a UTRAN cell belonging to the PLMN or the list of Equivalent PLMNs where the Logging Measurement Configuration was received and source E-UTRAN cell is not included in the exclude-list for the E-UTRAN frequency in SIB 19 on target UTRAN cell }

then { UE performs inter-RAT ANR logging for the E-UTRAN cell to be reported in the logged ANR report provided to the network in the UE INFORMATION RESPONSE message }

}

(3)

with { UE in UTRA IDLE state and UE has a Inter-RAT ANR logging measurement stored for E-UTRAN cell and the registered PLMN is the same as the IE "PLMN Identity" stored in LOG_ANR_REPORT_VARIABLE }

ensure that {

when { receiving RRC CONNECTION SETUP message }

then { UE includes the ANR Logging Results Available IE in the RRC CONNETION SETUP COMPLETE message }

}

(4)

with { UE in UTRA CELL_DCH state and UE has inter-RAT ANR logging results available for E-UTRAN and the registered PLMN is the same as the IE "PLMN Identity" stored in variable LOG_ANR_REPORT_VARIABLE }

ensure that {

when { UE receives an UE INFORMATION REQUEST message with Logged ANR Report Request IE which is asking for ANR log from UE }

then { UE sends an UE INFORMATION RESPONSE message with ANR logged data for E-UTRAN }

}

8.7.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 25.304, clauses 5.8.2.2; TS 25.331, clauses 8.1.3.6, 8.5.63.3, 8.5.64.3 and 8.5.67.2

[TS 25.304, clause 5.8.2.2 (TP2)]

If configured to perform inter-RAT ANR via the Logging Measurement Configuration message, the UE may perform inter-RAT ANR logging only when:

-	after inter-RAT cell reselection from E-UTRAN or GSM to a normal UTRAN cell belonging to the PLMN or the list of Equivalent PLMNs where the Logging Measurement Configuration is received.

During the inter-RAT ANR process, the UE may log the corresponding information of the previously camped E-UTRAN or GSM cell as specified in TS 25.331.

[TS 25.331, clause 7.2.1 (TP2)]

The UE shall perform ANR measurements and logging as specified in [4], when logged ANR measurement is configured.

[TS 25.331, clause 8.1.3.6 (TP3)]

The UE shall compare the value of the IE "Initial UE identity" in the received RRC CONNECTION SETUP message with the value of the variable INITIAL_UE_IDENTITY.

…

If the values are identical, the UE shall:

…

1> submit an RRC CONNECTION SETUP COMPLETE message to the lower layers on the uplink DCCH after successful state transition per subclause 8.6.3.3, with the contents set as specified below:

2>	if an IE "Logged ANR Report Info" in variable LOG_ANR_REPORT_VARIABLE is present and the registered PLMN is the same as the IE "PLMN Identity" stored in variable LOG_ANR_REPORT_VARIABLE:

3>	include IE "ANR Logging Results Available".

[TS 25.331, clause 8.5.63.3 (TP1)]

Upon receiving the LOGGING MEASUREMENT CONFIGURATION message the UE shall:

1>	if IE "Logged ANR configuration Info" is present:

2>	if variable LOG_ANR_CONFIG was already stored, discard the existing logged measurement configuration for ANR purpose as well as the associated logged measurement information as specified in 8.5.66;

2>	store the received IEs in the IE "Logged ANR configuration Info" in variable LOG_ANR_CONFIG;

2>	store the current Registered PLMN in the IE "PLMN Identity" in variable LOG_ANR_REPORT_VARIABLE;

2>	start timer T327 with the timer value set to the IE "Logging Duration" included in IE "Logged ANR configuration Info".

[TS 25.331, clause 8.5.64.3 (TP4)]

The UE shall:

1>	if IE "Logged ANR Report Request" is present:

2>	if Registered PLMN is the same as the IE "PLMN Identity" stored in variable LOG_ANR_REPORT_VARIABLE:

3>	if IE "Logged ANR Report Info" in variable LOG_ANR_REPORT_VARIABLE is present:

4> set IEs "Logged ANR Report Info" in the UE INFORMATION RESPONSE as follows:

5>	include the IEs "Logged ANR Report Info List" and set it to include entries from LOG_ANR_REPORT_VARIABLE;

5>	clear the logged measurement results included in the list of IEs "Logged ANR Report Info List" from the LOG_ANR_REPORT_VARIABLE;

5>	clear the variable LOG_ANR_CONFIG and stop timer T327.

2>	transmit a UE INFORMATION RESPONSE message on the uplink DCCH using AM RLC.

[TS 25.331, clause 8.5.67.2 (TP2,TP4)]

While T327 is running, the UE shall:

1>	perform the ANR measurements and evaluation on UTRAN, E-UTRAN or GERAN cells in accordance with the following:

2>	if IE “Inter-RAT ANR for E-UTRA Indicator” is included in variable LOG_ANR_CONFIG:

3>	if the UE reselected from a E-UTRA cell to an UTRA cell (serving cell) that is part of the PLMN which is the same PLMN as the IE “PLMN Identity” stored in variable LOG_ANR_REPORT_VARIABLE; and

3>	if the previously camped E-UTRAN cell is not included in the exclude-list for the EUTRAN frequency in SIB19 of the serving cell; and

3>	if both the previously camped E-UTRAN cell and serving cell are not CSG cells:

4>	log the ANR information into the variable LOG_ANR_REPORT_VARIABLE, if E-UTRA related ANR information has not been logged before, as follows:

5>		set the IEs "Serving PLMN Identity" and "Serving Cell" to indicate cell identity of the serving cell;

5>	set the IE "Cell Identity" to indicate cell identity of this previously camped E-UTRAN cell;

5>	set the IE "PLMN Identity" to indicate the Primary PLMN which this previously camped E-UTRAN cell belongs to;

5>	set the IE "Tracking Area Code" to indicate the TAC which this previously camped E-UTRAN cell belongs to;

5>	set the IE "EARFCN" and "Physical Cell Identity" of this previously camped E-UTRAN cell.

8.7.1.3	Test Description

8.7.1.3.1	Pre-test conditions

System Simulator:

-	SS shall use Rel-10 branch of UTRAN ASN.1 for downlink messages.

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell: Cell 1.

-	System Information Block type 19 as defined in TS 36.508 [18] clause 4.4.4.1 is used in UTRA cell: Cell 5.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A [18] on Cell 5 and moves to state PS-DCCH+DTCH_DCH (state 6-10) according to [5].

8.7.1.3.2	Test procedure sequence

Table 8.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values are applied are described in the texts in this clause.

Table 8.7.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -115 | - |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -62 |  |
| T1 | RS EPRE | dBm/15kHz | -60 | - | UE performs cell reselection from Cell 5 to Cell 1 |
|  | CPICH_Ec | dBm/3.84 MHz | - | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -72 |  |
| T2 | RS EPRE | dBm/15kHz | -115 | - | UE performs cell reselection from Cell 1 to Cell 5 |
|  | CPICH_Ec | dBm/3.84 MHz | - | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -62 |  |

Table 8.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LOGGING MEASUREMENT CONFIGURATION message including to configure the UE to perform inter-RAT ANR logging for E-UTRA on Cell 5. | <-- | LOGGING MEASUREMENT CONFIGURATION | - | - |
| 2 | The SS transmits an RRC CONNECTION RELEASE message on CCCH. | <-- | RRC CONNECTION RELEASE | - | - |
| 3 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 8.7.1.3.2-1. | - | - | - | - |
| 4 | Generic test procedure in TS 36.508 Table 6.4.2.7A is performed on Cell 1. | - | - | - | - |
| 5 | Wait for 6 s for UE to receive system information. |  |  |  |  |
| 6 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 8.7.1.3.2-1. | - | - | - | - |
| 7 | Generic test procedure in TS 36.508 Table 6.4.2.8 is performed on Cell 5. And the UE move to idle mode on Cell 5. | - | - | - | - |
| 8 | Wait 10 seconds to allow UE to re-activate ANR logging after returning from Cell 1. | - | - | - | - |
| 9 | The SS transmits a Paging message to the UE. | <-- | PAGING | - | - |
| 10 | The UE sends an RRC CONNECTION REQUEST message. | --> | RRC CONNECTION REQUEST | - | - |
| 11 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 12 | Check: Does the UE sends an RRC CONNECTION SETUP COMPLETE with the IE “ANR Logging Results Available”. | --> | RRC CONNECTION SETUP COMPLETE | 3 | P |
| 12A | The UE transmits a SERVICE REQUEST message on Cell 5. | --> | SERVICE REQUEST | - | - |
| 12B | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand | - | - |
| 12C | The UE transmits a SecurityModeComplete message and establishes the security configuration. | --> | RRC: SecurityModeComplete | - | - |
| 13 | The SS transmits a UE INFORMATION REQUEST message on Cell 5. | <-- | UE INFORMATION REQUEST | - | - |
| 14 | Check: Does the UE send UE INFORMATION RESPONSE with the IE “Logged ANR Report Info” on SRB4. | --> | UE INFORMATION RESPONSE | 1,2,4 | P |

8.7.1.3.3	Specific message contents

Table 8.7.1.3.3-1: LOGGING MEASUREMENT CONFIGURATION (step 1, Table 8.7.1.3.2-2)

| Derivation path: 34.108 default LOGGING MEASUREMENT CONFIGURATION in section 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Logged Measurements Configuration Info |  |  |  |
| Logged ANR configuration Info |  |  |  |
| - Logging Duration | 1 hour |  |  |
| - Inter-RAT ANR for E-UTRA Indicator | TRUE |  |  |

Table 8.7.1.3.3-2: RRC CONNECTION SETUP COMPLETE (step 12, Table 8.7.1.3.2-2)

| Derivation path: 34.108 default RRC CONNECTION SETUP COMPLETE in section 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Other information elements |  |  |  |
| Deferred measurement control reading |  |  |  |
| -ANR Logging Results Available | TRUE |  |  |

Table 8.7.1.3.3-3: UE INFORMATION REQUEST (step 13, Table 8.7.1.3.2-2)

| Derivation path: 34.108 default UE INFORMATION REQUEST in section 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Logged ANR Report Request | TRUE |  |  |

Table 8.7.1.3.3-4: UE INFORMATION RESPONSE (step 14, Table 8.7.1.3.2-2)

| Derivation path: 34.108 default UE INFORMATION RESPONSE in section 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Logged ANR Report Info List |  |  |  |
| -Serving PLMN Identity | PLMN Identity of Cell 5 |  |  |
| -Serving Cell | Cell Identity of Cell 5 |  |  |
| -CHOICE logged cell info |  |  |  |
| -E-UTRA |  |  |  |
| - PLMN Identity | PLMN Identity within System Information Bloc k Type1 broadcasted in Cell 1 |  |  |
| - Tracking Area Code | Tracking Area Code of Cell 1 |  |  |
| - Cell Identity | Cell Identity within System Information Bloc k Type1 broadcasted in Cell 1 |  |  |
| - EARFCN | Same downlink EARFCN as used in Cell 1 |  |  |
| - Physical Cell identity | Physical Cell Identity of Cell 1 |  |  |

## 8.8	Void

## 8.9	Aerial UE communication

### 8.9.1	Aerial UE / UE has flight path information available / UE information

8.9.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and UE has flight path information available }

ensure that {

when { receiving UEInformationRequest message including flightPathInfoReport }

then { UE transmits UEInformationResponse messages with flightPathInfoReport}

}

8.9.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.6.5.3.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if flightPathInfoReq field is present and the UE has flight path information available:

2>	include the flightPathInfoReport and set it to include the list of waypoints along the flight path;

2>	if the includeTimeStamp is set to TRUE:

3>	set the field timeStamp to the time when UE intends to arrive to each waypoint if this information is available at the UE;

8.9.1.3	Test description

8.9.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

-	The user shall programme into the UE a flightpath containing one waypoint. Note that the method for performing this programming is implementation specific.

8.9.1.3.2	Test procedure sequence

Table 8.9.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UEInformationRequest message including flightPathInfoReport and TimeStamp set to TRUE on Cell 1. | <-- | UEInformationRequest | - | - |
| 2 | Check: Does the UE transmit a UEInformationResponse message with flightPathInfoReport and timestamp? | --> | UEInformationResponse | 1 | P |

8.9.1.3.3	Specific message contents

Table 8.9.1.3.3-1: UEInformationRequest (step 1, Table 8.9.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r15 |  | Not present |  |  |
| flightPathInfoReq-r15 SEQUENCE { |  |  |  |  |
| maxWayPointNumber-r15 |  | 1 |  |  |
| includeTimeStamp-r15 |  | true |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.9.1.3.3-2: UEInformationResponse (step 2, Table 8.9.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measResultListIdle-r15 |  | Not checked |  |  |
| flightPathInfoReport-r15 SEQUENCE { |  |  |  |  |
| flightPath-r15 SEQUENCE { |  | 1 entry |  |  |
| wayPointLocation-r15 SEQUENCE { |  |  |  |  |
| locationCoordinates-r10 |  | Any allowed value |  |  |
| horizontalVelocity-r10 |  | Not checked |  |  |
| gnss-TOD-msec-r10 |  | Not checked |  |  |
| verticalVelocityInfo-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| timeStamp-r15 |  | Any allowed value |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 8.9.2	Aerial UE / Measurement configuration control and reporting / Event H1

8.9.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event H1 with event based periodical reporting }

ensure that {

when { Aerial UE height becomes bigger than the absolute treshold plus hysteresis }

then { UE sends MeasurementReport message at regular intervals while entering conditions for event H1 are satisfied }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and periodical measurement reporting triggered by event H1 ongoing}

ensure that {

when { Aerial UE height becomes lower than the absolute treshold minus hysteresis }

then { UE stops sending MeasurementReport message }

}

8.9.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.16 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

...

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the triggerType is set to event and if the eventId is set to eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined within the VarMeasConfig for this event:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

…

[TS 36.331, clause 5.5.4.16]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H1-2, as specified below, is fulfilled;

Inequality H1-1 (Entering condition)

Inequality H1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the Aerial UE height, not taking into account any offsets.

Hys is the hysteresis parameter (i.e. h1-Hysteresis as defined within ReportConfigEUTRA) for this event.

Thresh is the reference threshold parameter for this event given in MeasConfig(i.e. heightThreshRef as defined within MeasConfig).

Offset is the offset value to heightThreshRef to obtain the absolute threshold for this event. (i.e. h1-ThresholdOffset as defined within ReportConfigEUTRA)

Ms is expressed in meters.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

...

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId;

2>	start the periodical reporting timer with the value of reportInterval as defined within the reportConfig for this measId;

…

1>	if the triggerType is set to event; and if eventId is set to eventH1 or eventH2:

2>	set the heightUE to include the altitude of the UE;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the triggerType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

…

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

8.9.2.3	Test description

8.9.2.3.1	Pre-test conditions

System Simulator:

- Cell 1

- 1 GNSS simulator

UE:

None.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18].

8.9.2.3.2	Test procedure sequence

Table 8.9.2.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.9.2.3.2-1: Simulated aerial height

| Time | Parameter | Unit | Value (Note 2) | Remark (Note 1) |
| --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 500 | Simulated aerial height above sea level (Ms) is such that entry condition for event H1 is not satisfied: Ms - Hys – Offset < Thresh |
| T1 |  |  | 680 | Simulated aerial height above sea level (Ms) is such that entry condition for event H1 is satisfied: Ms - Hys – Offset > Thresh |
| T2 |  |  | 480 | Simulated aerial height above sea level (Ms) is such that exit condition for event H1 is satisfied: Ms + Hys – Offset < Thresh |
| Note 1: Value of Hys and Offset correspond to values of h1-Hysteresis-15 and h1-ThresholdOffset-r15 in Table 8.9.2.3.3-3. Value of Thresh corresponds to value of heightThreshRef-r15 in Table 8.9.2.3.3-2. Note 2: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. |  |  |  |  |

Table 8.9.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 00 | Trigger the UE to reset UTC time in order that the UE accepts the time generated by the GNSS simulator and also uses only GNSS for position calculation in the subsequent test procedure, as defined in TS 36.509 [25] clause 5.5.3. NOTE: The UTC time reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 0 | SS configures the GNSS simulator for step 1 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.9.2.3.2-1. | - | - | - | - |
| 1 | SS transmits a RRCConnectionReconfiguration message including MeasConfig to setup intra E-UTRAN measurement and reporting for event H1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits a RCConnectionReconfigrationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T1 in Table 8.9.2.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event H1 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 5 below is repeated until 3 MeasurementReport messages are received from the UE. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message to report event H1 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 6 | SS configures GNSS simulator for step 4 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T2 in Table 8.9.2.3.2-1. | - | - | - | - |
| 7 | Wait and ignore MeasurementReport messages for 50 s (as detailed in TS 36.508 [18] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 8 | Check: Does the UE attempt to transmit an uplink message within the next 10s? | - | - | 2 | F |

8.9.2.3.3	Specific message contents

Table 8.9.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.9.2.3.2-2)

| Derivation Path: TS 36.508 clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.9.2.3.3-2: MeasConfig-DEFAULT (step 1, Table 8.9.2.3.2-2)

| Derivation Path: TS 36.508, clause 4.6.6 Table 4.6.6-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasConfig-DEFAULT ::= CHOICE { |  |  |  |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  |  |  | Band > 64 |  |
| } |  |  |  |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE{ |  |  |  |  |  |  |  |
| reportConfigId[1] |  | IdReportConfig-H1 |  |  |  |  |  |
| reportConfig[1] |  | reportConfigEUTRA-H1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |  |  |  |
| measId[1] | 1 |  |  |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |  |  |
| reportConfigId[1] | IdReportConfig-H1 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |  | Band > 64 |  |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| heightThreshRef-r15 CHOICE { |  |  |  |  |  |  |  |
| Setup |  | 3 (480m) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.9.2.3.3-3: reportConfigEUTRA-H1 (step 1, Table 8.9.2.3.2-2)

| Derivation path: TS 36.508 clause 4.6.6 table 4.6.6-15 ReportConfigEUTRA-H1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventID CHOICE { |  |  |  |
| eventH1-r15 SEQUENCE { |  |  |  |
| h1-ThresholdOffset-r15 | 50 (100m) |  |  |
| h1-Hysteresis-15 | 10 (10m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |

Table 8.9.2.3.3-4: MeasurementReport (steps 4 and 5, Table 8.9.2.3.2-2)

| Derivation path: 36.508 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| heightUE-r15 | (-400..8880) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.9.3	Aerial UE / Measurement configuration control and reporting / Event H2

8.9.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event H2 with event based periodical reporting }

ensure that {

when { Aerial UE height becomes lower than the absolute treshold minus hysteresis }

then { UE sends MeasurementReport message at regular intervals while entering conditions for event H2 are satisfied }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and periodical measurement reporting triggered by event H2 ongoing}

ensure that {

when { Aerial UE height becomes higher than the absolute treshold plus hysteresis }

then { UE stops sending MeasurementReport message }

}

8.9.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.17 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

...

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the triggerType is set to event and if the eventId is set to eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined within the VarMeasConfig for this event:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

…

[TS 36.331, clause 5.5.4.17]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H2-2, as specified below, is fulfilled;

Inequality H2-1 (Entering condition)

Inequality H2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the Aerial UE height, not taking into account any offsets.

Hys is the hysteresis parameter (i.e. h2-Hysteresis as defined within ReportConfigEUTRA) for this event.

Thresh is the reference threshold parameter for this event given in MeasConfig(i.e. heightThreshRef as defined within MeasConfig).

Offset is the offset value to heightThreshRef to obtain the absolute threshold for this event. (i.e. h2-ThresholdOffset as defined within ReportConfigEUTRA)

Ms is expressed in meters.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

...

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId;

2>	start the periodical reporting timer with the value of reportInterval as defined within the reportConfig for this measId;

…

1>	if the triggerType is set to event; and if eventId is set to eventH1 or eventH2:

2>	set the heightUE to include the altitude of the UE;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the triggerType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

…

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

8.9.3.3	Test description

8.9.3.3.1	Pre-test conditions

System Simulator:

- Cell 1

- 1 GNSS simulator

UE:

None.

Preamble:

- The UE is in state Generic RB Established (state 3) according to [18].

8.9.3.3.2	Test procedure sequence

Table 8.9.3.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.9.3.3.2-1: Simulated aerial height

| Time | Parameter | Unit | Value (Note 2) | Remark (Note 1) |
| --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 490 | Simulated aerial height above sea level (Ms) is such that entry condition for event H2 is not satisfied: Ms + Hys – Offset > Thresh |
| T1 |  |  | 310 | Simulated aerial height above sea level (Ms) is such that entry condition for event H2 is satisfied: Ms + Hys – Offset < Thresh |
| T2 |  |  | 510 | Simulated aerial height above sea level (Ms) is such that exit condition for event H2 is satisfied: Ms - Hys – Offset > Thresh |
| Note 1: Value of Hys and Offset correspond to values of h2-Hysteresis-15 and h2-ThresholdOffset-r15 in Table 8.9.3.3.3-3. Value of Thresh corresponds to value of heightThreshRef-r15 in Table 8.9.3.3.3-2. Note 2: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. |  |  |  |  |

Table 8.9.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | SS configures the GNSS simulator for step 1 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.9.3.3.2-1. | - | - | - | - |
| 1 | SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra E-UTRAN measurement and reporting for event H2. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RCConnectionReconfigrationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T1 in Table 8.9.3.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 5 below is repeated until 3 MeasurementReport messages are received from the UE. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message to report event H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 6 | SS configures GNSS simulator for step 4 of Scenario #1 as defined in TS 36.508 [18] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T2 in Table 8.9.3.3.2-1. | - | - | - | - |
| 7 | Wait and ignore MeasurementReport messages for 50 s (as detailed in TS 36.508 [18] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 8 | Check: Does the UE attempt to transmit an uplink message within the next 10s? | - | - | 2 | F |

8.9.3.3.3	Specific message contents

Table 8.9.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.9.3.3.2-2)

| Derivation Path: TS 36.508 clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig | MeasConfig-DEFAULT |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.9.3.3.3-2: MeasConfig-DEFAULT (step 1, Table 8.9.3.3.2-2)

| Derivation Path: TS 36.508, clause 4.6.6 Table 4.6.6-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment | Condition |
| MeasConfig-DEFAULT ::= CHOICE { |  |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  |  |  | Band > 64 |
| } |  |  |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE{ |  |  |  |  |  |
| reportConfigId[1] |  | IdReportConfig-H2 |  |  |  |
| reportConfig[1] |  | reportConfigEUTRA-H2 |  |  |  |
| } |  |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |  |  |
| measId[1] | 1 |  |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |  |
| reportConfigId[1] | IdReportConfig-H2 |  |  |  |  |
| } |  |  |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| heightThreshRef-r15 CHOICE { |  |  |  |  |  |
| Setup |  | 2 (180m) |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 8.9.3.3.3-3: reportConfigEUTRA-H2 (step 1, Table 8.9.3.3.2-2)

| Derivation path: TS 36.508 clause 4.6.6 table 4.6.6-15 ReportConfigEUTRA-H1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventID CHOICE { |  |  |  |
| eventH2-r15 SEQUENCE { |  |  |  |
| h2-ThresholdOffset-r15 | 115 (230m) |  |  |
| h2-Hysteresis-15 | 10 (10m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| } |  |  |  |

Table 8.9.3.3.3-4: MeasurementReport (steps 4 and 5, Table 8.9.3.3.2-2)

| Derivation path: 36.508 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| heightUE-r15 | (-400..8880) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.9.4	Aerial UE / Measurement configuration control and reporting / numberOfTriggeringCells configured / Event A3

8.9.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A3 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A3 is not satisfied as the number of cells meeting the criteria is less than numberOfTriggeringCells }

then { UE does not send MeasurementReport message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A3 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A3 is satisfied as the number of cells meeting the criteria is same or more than numberOfTriggeringCells }

then { UE sends MeasurementReport message at regular intervals }

}

8.9.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.4 and 5.5.5. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical, eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCell:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

...

2>	upon expiry of the T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to event or to periodical while the corresponding measurement is not performed due to the PCell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

[TS 36.331, clause 5.5.4.4]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	if usePSCell of the corresponding reportConfig is set to true:

2>	use the PSCell for Mp, Ofp and Ocp;

1>	else:

2>	use the PCell for Mp, Ofp and Ocp;

NOTE	The cell(s) that triggers the event is on the frequency indicated in the associated measObject which may be different from the frequency used by the PCell/ PSCell.

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the PCell/ PSCell, not taking into account any offsets.

Ofp is the frequency specific offset of the frequency of the PCell/ PSCell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the PCell/ PSCell).

Ocp is the cell specific offset of the PCell/ PSCell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the PCell/ PSCell), and is set to zero if not configured for the PCell/ PSCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigEUTRA for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

[TS 36.331, clause 5.5.5]

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to 'event':

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to 'event'; or the purpose is set to 'reportStrongestCells' or to 'reportStrongestCellsForSON':

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

...

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

...

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.9.4.3	Test description

8.9.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 4.

-	Cell 1 is the serving cell.

-	Cell 2 and Cell 4 are intra-frequency neighbour cells of Cell 1.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.9.4.3.2	Test procedure sequence

Table 8.9.4.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 2 and Cell 4 at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while rows marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.9.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 (DL only) | Cell 4 (DL only) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -91 | Power levels are such that entry condition for event A3 is not satisfied for both the Cells. Mn2 + Ofn2 + Ocn2 + Hys < Ms + Ofs + Ocs + Off and Mn4 + Ofn4 + Ocn4 + Hys < Ms + Ofs + Ocs + Off |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied only for Cell 2: Mn2 + Ofn2 + Ocn2 - Hys > Ms + Ofs + Ocs + Off and Mn4 + Ofn4 + Ocn4 + Hys < Ms + Ofs + Ocs + Off |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied only for Cell 4: Mn2 + Ofn2 + Ocn2 + Hys < Ms + Ofs + Ocs + Off and Mn4 + Ofn4 + Ocn4 - Hys > Ms + Ofs + Ocs + Off |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -79 | Power levels are such that entry condition for event A3 is satisfied for both Cell 2 and Cell 4: Mn2 + Ofn2 + Ocn2 - Hys > Ms + Ofs + Ocs + Off and Mn4 + Ofn4 + Ocn4 - Hys > Ms + Ofs + Ocs + Off |

Table 8.9.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A3 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP value for Cell 2 and/or Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 4 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.9.4.3.2-1. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP value for Cell 2 within next 10s? | --> | MeasurementReport | 1 | F |
| 6 | SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.9.4.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message on Cell 1, with the measured RSRP value for Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 8 | SS re-adjusts the cell-specific reference signal level according to row "T3" in table 8.9.4.3.2-1. | - | - | - | - |
| - | EXCEPTION: Step 9 below is repeated until 4 MeasurementReport messages are received from the UE | - | - | - | - |
| 9 | Check: Does the UE transmit a MeasurementReport message on Cell 1, with the measured RSRP value for Cell 2 and Cell 4? | --> | MeasurementReport | 2 | P |

8.9.4.3.3	Specific message contents

Table 8.9.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.9.4.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8 with condition MEAS

Table 8.9.4.3.3-2: MeasConfig (step 1, Table 8.9.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-RECONF |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-Need ForGaps to TRUE. |

Table 8.9.4.3.3-3: ReportConfigEUTRA-A3 (step 1, Table 8.9.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-6AA, Condition Aerial UE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| maxReportCells | 2 |  |  |
| reportAmount | r4 |  |  |
| } |  |  |  |

Table 8.9.4.3.3-4: MeasurementReport (step 9, Table 8.9.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 2 entries | Report Cell 2 and Cell 4 |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| physCellId[2] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[2] | Not present |  |  |
| measResult[2] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.9.4a	Aerial UE / Measurement configuration control and reporting / numberOfTriggeringCells configured / Event A3 (Inter-frequency measurement)

8.9.4a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and inter-frequency measurement configured for event A3 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A3 is not satisfied as the number of cells meeting the criteria is less than numberOfTriggeringCells }

then { UE does not send MeasurementReport message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and inter-frequency measurement configured for event A3 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A3 is satisfied as the number of cells meeting the criteria is same or more than numberOfTriggeringCells }

then { UE sends MeasurementReport message at regular intervals }

}

8.9.4a.2	Conformance requirements

Same as test case 8.9.4 with the following differences:

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

…

4>	else:

…

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

...

8.9.4a.3	Test description

8.9.4a.3.1	Pre-test conditions

Same as test case 8.9.4 with the following differences:

-	Cells configuration: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

8.9.4a.3.2	Test procedure sequence

Same as test case 8.9.4 with the following differences:

-	Cells configuration: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

8.9.4a.3.3	Specific message contents

Same as test case 8.9.4 with the following differences:

-	In Table 8.9.4.3.3.-4: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

Table 8.9.4a.3.3-1: MeasConfig (step 1, Table 8.9.4a.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 2 entries |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | 2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-RECONF |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

### 8.9.5	Aerial UE / Measurement configuration control and reporting / numberOfTriggeringCells configured / Event A4

8.9.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A4 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A4 is not satisfied as the number of cells meeting the criteria is less than numberOfTriggeringCells }

then { UE does not send MeasurementReport message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A4 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A4 is satisfied as the number of cells meeting the criteria is same or more than numberOfTriggeringCells }

then { UE sends MeasurementReport message at regular intervals }

}

8.9.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.5 and 5.5.5. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical, eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCell:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

...

2>	upon expiry of the T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to event or to periodical while the corresponding measurement is not performed due to the PCell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

[TS 36.331, clause 5.5.4.5]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled;

Inequality A4-1 (Entering condition)

Inequality A4-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigEUTRA for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.5.5]

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to 'event':

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to 'event'; or the purpose is set to 'reportStrongestCells' or to 'reportStrongestCellsForSON':

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

...

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

...

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.9.5.3	Test description

8.9.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 4.

-	Cell 1 is the serving cell.

-	Cell 2 and Cell 4 are intra-frequency neighbour cells of Cell 1.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.9.5.3.2	Test procedure sequence

Table 8.9.5.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 2 and Cell 4 at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while rows marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.9.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 (DL only) | Cell 4 (DL only) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -97 | Power levels are such that entry condition for event A4 is not satisfied for both the Cells. Mn2 + Ofn2 + Ocn2 - Hys < Thresh and Mn4 + Ofn4 + Ocn4 - Hys < Thresh |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -97 | Power levels are such that entry condition for event A4 is satisfied only for Cell 2: Mn2 + Ofn2 + Ocn2 - Hys > Thresh and Mn4 + Ofn4 + Ocn4 - Hys < Thresh |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -79 | Power levels are such that entry condition for event A4 is satisfied only for Cell 4: Mn2 + Ofn2 + Ocn2 - Hys < Thresh and Mn4 + Ofn4 + Ocn4 - Hys > Thresh |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -79 | Power levels are such that entry condition for event A4 is satisfied for both Cell 2 and Cell 4: Mn2 + Ofn2 + Ocn2 - Hys > Thresh and Mn4 + Ofn4 + Ocn4 - Hys > Thresh |

Table 8.9.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A4 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A4 with the measured RSRP value for Cell 2 and/or Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 4 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.9.5.3.2-1. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A4 with the measured RSRP value for Cell 2 within next 10s? | --> | MeasurementReport | 1 | F |
| 6 | SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.9.5.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message on Cell 1, with the measured RSRP value for Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 8 | SS re-adjusts the cell-specific reference signal level according to row "T3" in table 8.9.5.3.2-1. | - | - | - | - |
| - | EXCEPTION: Step 9 below is repeated until 4 MeasurementReport messages are received from the UE | - | - | - | - |
| 9 | Check: Does the UE transmit a MeasurementReport message on Cell 1, with the measured RSRP value for Cell 2 and Cell 4? | --> | MeasurementReport | 2 | P |

8.9.5.3.3	Specific message contents

Table 8.9.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.9.5.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8 with condition MEAS

Table 8.9.5.3.3-2: MeasConfig (step 1, Table 8.9.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 6 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A4-RECONF |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 6 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.9.5.3.3-3: ReportConfigEUTRA-A4 (-88) (step 1, Table 8.9.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-6AA, Conditions NOT RSRQ, Aerial UE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| maxReportCells | 2 |  |  |
| reportAmount | r4 |  |  |
| } |  |  |  |

Table 8.9.5.3.3-4: MeasurementReport (step 9, Table 8.9.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 2 entries | Report Cell 2 and Cell 4 |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| physCellId[2] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[2] | Not present |  |  |
| measResult[2] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.9.5a	Aerial UE / Measurement configuration control and reporting / numberOfTriggeringCells configured / Event A4 (inter-frequency measurements)

8.9.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and inter-frequency measurement configured for event A4 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A4 is not satisfied as the number of cells meeting the criteria is less than numberOfTriggeringCells }

then { UE does not send MeasurementReport message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and inter-frequency measurement configured for event A4 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {

when { Entry condition for event A4 is satisfied as the number of cells meeting the criteria is same or more than numberOfTriggeringCells }

then { UE sends MeasurementReport message at regular intervals }

}

8.9.5a.2	Conformance requirements

Same as test case 8.9.5 with the following differences:

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

…

4>	else:

…

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

...

8.9.5a.3	Test description

8.9.5a.3.1	Pre-test conditions

Same as test case 8.9.5 with the following differences:

-	Cells configuration: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

8.9.5a.3.2	Test procedure sequence

Same as test case 8.9.5 with the following differences:

-	Cells configuration: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

8.9.5a.3.3	Specific message contents

Same as test case 8.9.5 with the following differences:

-	In Table 8.9.5.3.3.-4: Cell 3 replaces Cell 2, Cell 6 replaces Cell 4

Table 8.9.5a.3.3-1: MeasConfig (step 1, Table 8.9.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 2 entries |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | 2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 6 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A4-RECONF |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 6 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

### 8.9.6	Aerial UE / Measurement configuration control and reporting / numberOfTriggeringCells configured / Event A5

8.9.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A5 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {
  when { Entry condition for event A5 is not satisfied as the number of cells meeting the criteria is less than numberOfTriggeringCells }

then { UE does not send MeasurementReport message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for event A5 with event based periodical reporting and reportconfig includes numberOfTriggeringCells }

ensure that {
  when { Entry condition for event A5 is satisfied as the number of cells meeting the criteria is same or more than numberOfTriggeringCells }

then { UE sends MeasurementReport message at regular intervals }

}

8.9.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.6 and 5.5.5. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

...

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical, eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	if the triggerType is set to event, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4>	if T312 is not running:

5>	start timer T312 with the value configured in the corresponding measObject;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCell:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

2>	upon expiry of the T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to event or to periodical while the corresponding measurement is not performed due to the PCell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

NOTE 3:	If the UE is configured with DRX, the UE may delay the measurement reporting for event triggered and periodical triggered measurements until the Active Time, which is defined in TS 36.321 [6].

[TS 36.331, clause 5.5.4.6]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both conditions A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality A5-1 (Entering condition 1)

Inequality A5-2 (Entering condition 2)

Inequality A5-3 (Leaving condition 1)

Inequality A5-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigEUTRA for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigEUTRA for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Ms.

Thresh2 is expressed in the same unit as Mn.

[TS 36.331, clause 5.5.5]

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to 'event':

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to 'event'; or the purpose is set to 'reportStrongestCells' or to 'reportStrongestCellsForSON':

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

...

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

...

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.9.6.3	Test description

8.9.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 4.

-	Cell 1 is the serving cell.

-	Cell 2 and Cell 4 are intra-frequency neighbour cells of Cell 1.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.9.6.3.2	Test procedure sequence

Table 8.9.6.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 2 and Cell 4 at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while rows marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.9.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -65 | -97 | -97 | Power levels are such that no cell satisfies entry condition for event A5: Ms - Hys > Thresh1 and Mn1 + Ofn + Ocn + Hys < Thresh2 and Mn2 + Ofn + Ocn + Hys < Thresh2 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -79 | Power levels are such that Cell 1 and Cell 4 satisfy but Cell 2 does not satisfy the entry condition for event A5: Ms - Hys < Thresh1 and Mn1 + Ofn + Ocn + Hys < Thresh2 and Mn2 + Ofn + Ocn + Hys > Thresh2 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -97 | Power levels are such that Cell1 and Cell 2 satisfy but Cell 4 does not satisfy the entry condition for event A5: Ms - Hys < Thresh1 and Mn1 + Ofn + Ocn + Hys > Thresh2 and Mn2 + Ofn + Ocn + Hys < Thresh2 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -79 | Power levels are such that all 3 cells satisfy the entry condition for event A5: Ms + Hys < Thresh1 and Mn1 + Ofn + Ocn - Hys > Thresh2 and Mn2 + Ofn + Ocn - Hys > Thresh2 |

Table 8.9.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurement and reporting for event A5 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A5 with the measured RSRP value for Cell 2 and/or Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 4 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.9.6.3.2-1. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A5 with the measured RSRP value for Cell 4 within next 10s? | --> | MeasurementReport | 1 | F |
| 6 | SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.9.6.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A5 with the measured RSRP value for Cell 2 within next 10s? | --> | MeasurementReport | 1 | F |
| 8 | SS re-adjusts the cell-specific reference signal level according to row "T3" in table 8.9.6.3.2-1. | - | - | - | - |
| - | EXCEPTION: Step 9 below is repeated until 4 MeasurementReport messages are received from the UE | - | - | - | - |
| 9 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A5 with the measured RSRP value for Cell 2 and Cell 4? | --> | MeasurementReport | 2 | P |

8.9.6.3.3	Specific message contents

Table 8.9.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.9.6.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8 with condition MEAS

Table 8.9.6.3.3-2: MeasConfig (step 1, Table 8.9.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE{ | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 7 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A5 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 7 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.9.6.3.3-3: ReportConfigEUTRA-A5 (-76, -88) (step 1, Table 8.9.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-6AB, Conditions NOT RSRQ, Aerial UE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| maxReportCells | 2 |  |  |
| reportAmount | r4 |  |  |
| } |  |  |  |

Table 8.9.6.3.3-4: MeasurementReport (step 9, Table 8.9.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 2 entries | Report Cell 2, Cell 4 |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| physCellId[2] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[2] SEQUENCE { |  |  |  |
| rsrpResult | (52..97) |  |  |
| rsrqResult | (23..34) |  |  |
| additionalSI-Info-r9 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |