# 9	EPS mobility managements

## 9.1	EMM common procedures

### 9.1.1	Void

#### 9.1.1.1	Void

#### 9.1.1.2	Void

### 9.1.2	Authentication procedure

#### 9.1.2.1	Void

NOTE:	The present test (Authentication accepted) is superseded by test case 9.1.2.3 (i.e. all requirements which the present test verified are verified in test case 9.1.2.3).

#### 9.1.2.2	Void

#### 9.1.2.3	Authentication not accepted by the network / GUTI used / Authentication reject and re-authentication

9.1.2.3.1	Test Purpose (TP)

(1)

with { UE having sent an initial NAS message with type of identity GUTI }

ensure that {

when { UE receives an AUTHENTICATION REJECT message as a result of failure of an Authentication procedure initiated by the network  }

then { UE shall set the update status to EU3 ROAMING NOT ALLOWED, delete the stored GUTI, TAI list, last visited registered TAI and KSIASME and enter state EMM-DEREGISTERED }

}

(2)

with { UE having a NAS signalling connection existing }

ensure that {

when { UE receives an AUTHENTICATION REQUEST message }

then { UE responds with a correct AUTHENTICATION RESPONSE message and establishes correct EPS security context }

}

9.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.4.2.1, 5.4.2.3 and 5.4.2.5 and TS 36.331, clause 5.3.8.3. Unless otherwise stated these are Rel-8 requirements.

[TS 24.301, clause 5.4.2.5]

Upon receipt of an AUTHENTICATION REJECT message, the UE shall set the update status to EU3 ROAMING NOT ALLOWED, delete the stored GUTI, TAI list, last visited registered TAI and KSIASME. The USIM shall be considered invalid until switching off the UE or the UICC containing the USIM is removed.

If the AUTHENTICATION REJECT message is received by the UE, the UE shall abort any EMM signalling procedure, stop any of the timers T3410, T3417 or T3430 (if running) and enter state EMM-DEREGISTERED.

[TS 24.301, clause 5.4.2.1]

The UE shall support the EPS authentication challenge only if a USIM is present.

An EPS security context is established in the UE and the network when an EPS authentication is successfully performed. During a successful EPS authentication, the CK and IK keys are computed. CK and IK are then used as key material to compute a new key, KASME. KASME is stored in the EPS security contexts (see 3GPP TS 33.401 [19]) of both the network and the UE, and is the root for the EPS integrity protection and ciphering key hierarchy.

[TS 24.301, clause 5.4.2.3]

The UE shall respond to an AUTHENTICATION REQUEST message. With the exception of the cases described in subclause 5.4.2.6, the UE shall process the authentication challenge data and respond with an AUTHENTICATION RESPONSE message to the network.

Upon a successful EPS authentication challenge, the new KASME calculated from the authentication challenge data shall be stored in a new EPS security context.

[TS 33.401, clause 6.1.1]

UE shall compute KASME from CK, IK, and serving network's identity (SN id) using the KDF as specified in Annex A. SN id binding implicitly authenticates the serving network's identity when the derived keys from KASME are successfully used.

...

UE shall respond with User authentication response message including RES in case of successful AUTN verification as described in TS 33.102[4] and successful AMF verification as described above. Otherwise UE shall send User authentication reject message with a proper CAUSE value.

9.1.2.3.3	Test description

9.1.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.1.2.3.3.2	Test procedure sequence

Table 9.1.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a GUTI and a PDN CONNECTIVITY REQUEST message | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message without integrity protection and ciphering | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits an AUTHENTICATION REJECT message without integrity protection and ciphering | <-- | AUTHENTICATION REJECT | - | - |
| 6 | SS releases the RRC connection | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8 | Check: Does the test result of the generic procedure defined in clause 6.4.2.5 of 36.508 [18] indicates that the UE responds to paging when paged with S-TMSI include GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
| 9 | Check: Does the test result of the generic procedure defined in clause 6.4.2.5 of 36.508 [18] indicates that the UE responds to paging when paged with IMSI and with CN domain indicator set to "PS"? | - | - | 1 | - |
| 10 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 11 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 12 | Check: Does the UE transmit a NOT integrity protected ATTACH REQUEST message including IMSI and a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 13 | The SS transmits an AUTHENTICATION REQUEST message without integrity protection and ciphering | <-- | AUTHENTICATION REQUEST | - | - |
| 14 | The UE transmits an AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | 2 | P |
| 15 | SS transmits a NAS SECURITY MODE COMMAND message including the KSIASME of the new EPS security context (as provided in step 3) | <-- | SECURITY MODE COMMAND | - | - |
| 16 | Check: Does the UE respond with NAS SECURITY MODE COMPLETE message integrity protected and ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 5 | --> | SECURITY MODE COMPLETE | 2 | P |
| 17a1-26b1 | The attach procedure is completed by executing steps 9a1 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.1.2.3.3.3	Specific message contents

Table 9.1.2.3.3.3-1a: ATTACH REQUEST (step 2, Table 9.1.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 | GUTI allocated in pre-test conditions. |  |

Table 9.1.2.3.3.3-1: ATTACH REQUEST (step 12, Table 9.1.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier | '111'B | no key is available |  |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not Present |  |  |

Table 9.1.2.3.3.3-3: AUTHENTICATION RESPONSE (step 14, Table 9.1.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication response parameter | RES equal to the XRES calculated in the SS with the parameters provided/indicated in the AUTHENTICATION REQUEST |  |  |

#### 9.1.2.4	Authentication not accepted by the UE / MAC code failure

9.1.2.4.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { the UE receives an AUTHENTICATION REQUEST message with invalid MAC code }

then { the UE shall send an AUTHENTICATION FAILURE message to the network, with the reject cause #20 "MAC failure" }

}

9.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.4.2.6.

[TS 24.301, clause 5.4.2.6]

In an EPS authentication challenge, the UE shall check the authenticity of the core network by means of the AUTN parameter received in the AUTHENTICATION REQUEST message. This enables the UE to detect a false network.

During an EPS authentication procedure, the UE may reject the core network due to an incorrect AUTN parameter (see 3GPP TS 33.401 [19]). This parameter contains three possible causes for authentication failure:

a)	MAC code failure:

If the UE finds the MAC code (supplied by the core network in the AUTN parameter) to be invalid, the UE shall send an AUTHENTICATION FAILURE message to the network, with the EMM cause #20 "MAC failure". The UE shall then follow the procedure described in subclause 5.4.2.7, item c.

[TS 24.301, clause 5.4.2.7]

c)	Authentication failure (EMM cause #20 "MAC failure"):

The UE shall send an AUTHENTICATION FAILURE message, with EMM cause #20 "MAC failure" according to subclause 5.4.2.6, to the network and start timer T3418 (see example in figure 5.4.2.7.1). Furthermore, the UE shall stop any of the retransmission timers that are running (e.g. T3410, T3417, T3421 or T3430). Upon the first receipt of an AUTHENTICATION FAILURE message from the UE with EMM cause #20 "MAC failure", the network may initiate the identification procedure described in subclause 5.4.4. This is to allow the network to obtain the IMSI from the UE. The network may then check that the GUTI originally used in the authentication challenge corresponded to the correct IMSI. Upon receipt of the IDENTITY REQUEST message from the network, the UE shall send the IDENTITY RESPONSE message.

...

If the GUTI/IMSI mapping in the network was incorrect, the network should respond by sending a new AUTHENTICATION REQUEST message to the UE. Upon receiving the new AUTHENTICATION REQUEST message from the network, the UE shall stop the timer T3418, if running, and then process the challenge information as normal.

9.1.2.4.3	Test description

9.1.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 1) according to TS 36.508 [18].

9.1.2.4.3.2	Test procedure sequence

Table 9.1.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message | --> | ATTACH REQUEST | - | - |
| 3 | SS transmits an AUTHENTICATION REQUEST message which contains an invalid MAC code | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | Check: Does the UE respond with an AUTHENTICATION FAILURE message, with reject cause "MAC failure"? | --> | AUTHENTICATION FAILURE | 1 | P |
| 5 | SS transmits an IDENTITY REQUEST message requesting IMSI in the IE Identity type | <-- | IDENTITY REQUEST | - | - |
| 6 | The UE responds with a correct IDENTITY RESPONSE message providing its IMSI in the IE Mobile Identity | --> | IDENTITY RESPONSE | - | - |
| 7 | SS transmits a correct AUTHENTICATION REQUEST message, RAND different to the one send in Step 3 | <-- | AUTHENTICATION REQUEST | - | - |
| 8 | Check: Does the UE respond with a correct AUTHENTICATION RESPONSE message with RES that is equal to the XRES calculated in the SS? | --> | AUTHENTICATION RESPONSE | 1 | P |
| 9 | SS transmits a NAS SECURITY MODE COMMAND message including the KSIASME of the new EPS security context (as provided in step 8) | <-- | SECURITY MODE COMMAND | - | - |
| 10 | UE transmits a NAS SECURITY MODE COMPLETE message integrity protected and ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 9 | --> | SECURITY MODE COMPLETE | - | - |
| 11a1-20b1 | Steps 9a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.1.2.4.3.3	Specific message contents

Table 9.1.2.4.3.3-1: AUTHENTICATION REQUEST (step 3, Table 9.1.2.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication parameter AUTN | Invalid MAC | SS shall calculate the correct MAC value as specified in TS 33.102 and use any different value, e.g. correct_MAC+5. |  |

Table 9.1.2.4.3.3-2: AUTHENTICATION RESPONSE (step 8, Table 9.1.2.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication response parameter | RES equal to the XRES calculated in the SS with the parameters provided/indicated in the AUTHENTICATION REQUEST |  |  |

#### 9.1.2.5	Authentication not accepted by the UE / SQN failure

9.1.2.5.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when { the UE receives an AUTHENTICATION REQUEST message with SQN out of range }

then { the UE sends an AUTHENTICATION FAILURE message to the network, with EMM cause "synch failure" and a re-synchronization token }

}

(2)

with { UE having sent an AUTHENTICATION FAILURE message to the network, with EMM cause "synch failure" }

ensure that {
  when { the UE receives a new correct AUTHENTICATION REQUEST message while T3420 is running }

then { the UE sends a correct AUTHENTICATION RESPONSE message }

}

9.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.4.2.6 and 5.4.2.7.

[TS 24.301, clause 5.4.2.6]

In an EPS authentication challenge, the UE shall check the authenticity of the core network by means of the AUTN parameter received in the AUTHENTICATION REQUEST message. This enables the UE to detect a false network.

During an EPS authentication procedure, the UE may reject the core network due to an incorrect AUTN parameter (see 3GPP TS 33.401 [19]). This parameter contains three possible causes for authentication failure:

...

c)	SQN failure:

If the UE finds the SQN (supplied by the core network in the AUTN parameter) to be out of range, the UE shall send an AUTHENTICATION FAILURE message to the network, with the EMM cause #21 "synch failure" and a re-synchronization token AUTS provided by the USIM (see 3GPP TS 33.102 [18]). The UE shall then follow the procedure described in subclause 5.4.2.7, item e.

[TS 24.301, clause 5.4.2.7]

e)	Authentication failure (EMM cause #21 "synch failure"):

The UE shall send an AUTHENTICATION FAILURE message, with EMM cause #21 "synch failure", to the network and start the timer T3420 (see example in figure 5.4.2.7.2). Furthermore, the UE shall stop any of the retransmission timers that are running (e.g. T3410, T3417, T3421 or T3430). Upon the first receipt of an AUTHENTICATION FAILURE message from the UE with the EMM cause #21 "synch failure", the network shall use the returned AUTS parameter from the authentication failure parameter IE in the AUTHENTICATION FAILURE message, to re-synchronise. The re-synchronisation procedure requires the MME to delete all unused authentication vectors for that IMSI and obtain new vectors from the HSS. When re-synchronisation is complete, the network shall initiate the authentication procedure. Upon receipt of the AUTHENTICATION REQUEST message, the UE shall stop the timer T3420, if running.

...

If the network is validated successfully (a new AUTHENTICATION REQUEST is received which contains a valid SQN and MAC) while T3420 is running, the UE shall send the AUTHENTICATION RESPONSE message to the network and shall start any retransmission timers (e.g. T3410, T3417, T3421 or T3430), if they were running and stopped when the UE received the first failed AUTHENTICATION REQUEST message.

9.1.2.5.3	Test description

9.1.2.5.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Switched OFF (State 1) according to TS 36.508 [18].

9.1.2.5.3.2	Test procedure sequence

Table 9.1.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message | --> | ATTACH REQUEST | - | - |
| 3 | SS transmits AUTHENTICATION REQUEST message with the AMF field in the IE "Authentication parameter AUTN" set to "AMFRESYNCH" value to trigger SQN re-synchronisation procedure in test USIM | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | Check: Does the UE respond with an AUTHENTICATION FAILURE message, with EMM cause "synch failure"? | --> | AUTHENTICATION FAILURE | 1 | P |
| 5 | SS transmits an IDENTITY REQUEST message requesting IMSI in the IE Identity type | <-- | IDENTITY REQUEST | - | - |
| 6 | The UE responds with IDENTITY RESPONSE message providing its IMSI in the IE Mobile Identity | --> | IDENTITY RESPONSE | - | - |
| 7 | SS transmits AUTHENTICATION REQUEST message (Note 1) | <-- | AUTHENTICATION REQUEST | - | - |
| 8 | Check: Does the UE respond with AUTHENTICATION RESPONSE message with RES that is equal to the XRES calculated in the SS? | --> | AUTHENTICATION RESPONSE | 2 | P |
| 9 | SS transmits a NAS SECURITY MODE COMMAND message including the KSIASME of the new EPS security context (as provided in step 8) | <-- | SECURITY MODE COMMAND | - | - |
| 10 | UE transmits a NAS SECURITY MODE COMPLETE message integrity protected and ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 9 | --> | SECURITY MODE COMPLETE | - | - |
| 10Aa1-19b1 | Steps 9a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: The SS shall ensure that the AUTHENTICATION REQUEST message sent in step 7 is sent less than (T3420-10%) sec after the message sent in step 4 otherwise it cannot be ensured that the UE will behave as specified in step 8. |  |  |  |  |  |

9.1.2.5.3.3	Specific message contents

Table 9.1.2.5.3.3-1: AUTHENTICATION REQUEST (step 3, Table 9.1.2.5.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication parameter AUTN | AMF field set to "AMFRESYNCH" |  |  |

Table 9.1.2.5.3.3-2: AUTHENTICATION FAILURE (step 4, Table 9.1.2.5.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0001 0101'B | Synch failure |  |
| Authentication failure parameter | '1111 1111 1111 1111'B | AMFRESYNCH see TS 34.108, 8.1.2.2 |  |

Table 9.1.2.5.3.3-3: AUTHENTICATION RESPONSE (step 8, Table 9.1.2.5.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication response parameter | RES equal to the XRES calculated in the SS with the parameters provided/indicated in the AUTHENTICATION REQUEST |  |  |

#### 9.1.2.6	Abnormal cases / Network failing the authentication check

9.1.2.6.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state / EMM-CONNECTED mode}

ensure that {

when { UE receives an AUTHENTICATION REQUEST message but UE deems that the network failed the authentication check }

then { UE locally release the RRC connection and treat the active cell as barred }

}

9.1.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.4.2.7.

[TS 24.301, clause 5.4.2.7]

It can be assumed that the source of the authentication challenge is not genuine (authentication not accepted by the UE) if any of the following occur:

-	after sending the AUTHENTICATION FAILURE message with the EMM cause #20 "MAC failure" the timer T3418 expires;

…

When it has been deemed by the UE that the source of the authentication challenge is not genuine (i.e. authentication not accepted by the UE), the UE shall proceed as described in item f.

…

f)	Network failing the authentication check:

If the UE deems that the network has failed the authentication check, then it shall request RRC to locally release the RRC connection and treat the active cell as barred (see 3GPP TS 36.331 [22]). The UE shall start any retransmission timers (e.g. T3410, T3417, T3421 or T3430), if they were running and stopped when the UE received the first AUTHENTICATION REQUEST message containing an invalid MAC or SQN.

9.1.2.6.3	Test description

9.1.2.6.3.1	Pre-test conditions

System Simulator:

-	Cell A and Cell B are configured according to table 6.3.2.2-1 in TS 36.508 [18]

UE:

none.

Preamble:

-	the UE is in state Switched OFF (state 1) according to clause [18].

9.1.2.6.3.2	Test procedure sequence

Table 9.1.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a " Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 4 | SS transmits an AUTHENTICATION REQUEST message which contains an invalid MAC code | <-- | AUTHENTICATION REQUEST | - | - |
| 5 | UE responds with an AUTHENTICATION FAILURE message, with reject cause "MAC failure". | --> | AUTHENTICATION FAILURE | - | - |
| 6 | SS responds nothing and waits for the expiration of T3418. |  |  |  |  |
| 6A | The SS configures: - Cell B as the "Serving cell". - Cell A as a " Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 8-21b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.1.2.6.3.3	Specific message contents

Table 9.1.2.6.3.3-1: AUTHENTICATION REQUEST (step 3, Table 9.1.2.6.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication parameter AUTN | Invalid MAC | SS shall calculate the correct MAC value as specified in TS 33.401 and use any different value, e.g. correct_MAC+5. |  |

Table 9.1.2.6.3.3-2: AUTHENTICATION FAILURE (step 5, Table 9.1.2.6.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0001 0100'B | MAC failure |  |
| Authentication failure parameter | Not present |  |  |

Table 9.1.2.6.3.3-3: SystemInformationBlockType1(Cell A, Preamble and all steps)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | notBarred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.2.6.3.3-4: SystemInformationBlockType1-BR-r13 (Cell A, Preamble and all steps when UE under test is CAT M1)

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | notBarred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

#### 9.1.2.7	Authentication not accepted by the UE/ non-EPS authentication unacceptable

9.1.2.7.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { the UE receives an AUTHENTICATION REQUEST message with "separation bit" in the AMF field is 0 }

then { the UE shall send an AUTHENTICATION FAILURE message to the network, with the reject cause #26 " non-EPS authentication unacceptable " }

}

9.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.4.2.6 and 5.4.2.7.

[TS 24.301, clause 5.4.2.6]

During an EPS authentication procedure, the UE may reject the core network due to an incorrect AUTN parameter (see 3GPP TS 33.401 [19]). This parameter contains three possible causes for authentication failure:

…

b)	Non-EPS authentication unacceptable:

If the UE finds that the "separation bit" in the AMF field of AUTN supplied by the core network is 0, the UE shall send an AUTHENTICATION FAILURE message to the network, with the EMM cause #26 "non-EPS authentication unacceptable" (see subclause 6.1.1 in 3GPP TS 33.401 [19]). The UE shall then follow the procedure described in subclause 5.4.2.7, item d.

…

[TS 24.301, clause 5.4.2.7]

d)	Authentication failure (EMM cause #26 "non-EPS authentication unacceptable"):

The UE shall send an AUTHENTICATION FAILURE message, with EMM cause #26 "non-EPS authentication unacceptable", to the network and start the timer T3418 (see example in figure 5.4.2.7.1). Furthermore, the UE shall stop any of the retransmission timers that are running (e.g. T3410, T3417, T3421 or T3430). Upon the first receipt of an AUTHENTICATION FAILURE message from the UE with EMM cause #26 "non-EPS authentication unacceptable", the network may initiate the identification procedure described in subclause 5.4.4. This is to allow the network to obtain the IMSI from the UE. The network may then check that the GUTI originally used in the authentication challenge corresponded to the correct IMSI. Upon receipt of the IDENTITY REQUEST message from the network, the UE shall send the IDENTITY RESPONSE message.

…

9.1.2.7.3	Test description

9.1.2.7.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Switched OFF (State 1) according to TS 36.508 [18].

9.1.2.7.3.2	Test procedure sequence

Table 9.1.2.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message | --> | ATTACH REQUEST | - | - |
| 3 | SS transmits an AUTHENTICATION REQUEST message with "separation bit" in the AMF field is 0. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | Check: Does the UE respond with an AUTHENTICATION FAILURE message, with reject cause "non-EPS authentication unacceptable"? | --> | AUTHENTICATION FAILURE | 1 | P |
| 5 | SS transmits an IDENTITY REQUEST message requesting IMSI in the IE Identity type | <-- | IDENTITY REQUEST | - | - |
| 6 | The UE responds with a correct IDENTITY RESPONSE message providing its IMSI in the IE Mobile Identity | --> | IDENTITY RESPONSE | - | - |
| 7 | SS transmits a correct AUTHENTICATION REQUEST message, RAND different to the one send in Step 3 | <-- | AUTHENTICATION REQUEST | - | - |
| 8 | Check: Does the UE respond with a correct AUTHENTICATION RESPONSE message with RES that is equal to the XRES calculated in the SS? | --> | AUTHENTICATION RESPONSE | 1 | P |
| 9 | SS transmits a NAS SECURITY MODE COMMAND message including the KSIASME of the new EPS security context (as provided in step 8) | <-- | SECURITY MODE COMMAND | - | - |
| 10 | UE transmits a NAS SECURITY MODE COMPLETE message integrity protected and ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 9 | --> | SECURITY MODE COMPLETE | - | - |
| 10Aa1-19b1 | Steps 9a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.1.2.7.3.3	Specific message contents

Table 9.1.2.7.3.3-1: AUTHENTICATION REQUEST (step 3, Table 9.1.2.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication parameter AUTN | "separation bit"=0 | The "separation bit" in the AMF field of AUTN supplied by the core network is 0. |  |

Table 9.1.2.7.3.3-2: AUTHENTICATION FAILURE (step 4, Table 9.1.2.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0001 1010'B | non-EPS authentication unacceptable |  |

Table 9.1.2.7.3.3-3: AUTHENTICATION RESPONSE (step 8, Table 9.1.2.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Authentication response parameter | RES equal to the XRES calculated in the SS with the parameters provided/indicated in the AUTHENTICATION REQUEST |  |  |

### 9.1.3	Security mode control procedure

#### 9.1.3.1	NAS security mode command accepted by the UE

9.1.3.1.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives an integrity protected SECURITY MODE COMMAND message including replayed security capabilities and IMEISV request }

then { UE sends an integrity protected and ciphered SECURITY MODE COMPLETE message including IMEISV and starts applying the NAS Security in both UL and DL }

}

(2)

with { NAS Security Activated and EPS Authentication and key agreement procedure is executed for new Key generation }

ensure that {

when { UE receives an integrity protected SECURITY MODE COMMAND message corresponding to NAS count reset to zero including replayed security capabilities and IMEISV request }

then { UE sends integrity protected and ciphered SECURITY MODE COMPLETE message with NAS count set to zero including IMEISV and starts applying the NAS Security in both UL and DL }

}

9.1.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 clause 4.4.3.1, 5.4.3.1, 5.4.3.2 and 5.4.3.3.

[TS 24.301, clause 4.4.3.1]

Each EPS NAS security context shall be associated with two separate counters NAS COUNT: one related to uplink NAS messages and one related to downlink NAS messages. The NAS COUNT counters use 24 bit internal representation and are independently maintained by UE and MME. The NAS COUNT shall be constructed as a NAS sequence number (8 least significant bits) concatenated with a NAS overflow counter (16 most significant bits).

When NAS COUNT is input to NAS ciphering or NAS integrity algorithms it shall be considered to be a 32-bit entity which shall be constructed by padding the 24-bit internal representation with 8 zeros in the most significant bits.

During the handover from UTRAN/GERAN to E-UTRAN, if the mapped EPS security context is taken into use, the NAS COUNT values for this EPS security context shall be initialized to zero in the UE and the network for uplink and downlink NAS messages.

The NAS sequence number part of the NAS COUNT shall be exchanged between the UE and the MME as part of the NAS signalling. After each new or retransmitted outbound security protected NAS message, the sender shall increase the NAS COUNT number by one. Specifically, on the sender side, the NAS sequence number shall be increased by one, and if the result is zero (due to wrap around), the NAS overflow counter shall also be incremented by one (see subclause 4.4.3.5). The receiving side shall estimate the NAS COUNT used by the sending side. Specifically, if the estimated NAS sequence number wraps around, the NAS overflow counter shall be incremented by one.

[TS 24.301, clause 5.4.3.1]

The purpose of the NAS security mode control procedure is to take an EPS security context into use, and initialise and start NAS signalling security between the UE and the MME with the corresponding NAS keys and security algorithms.

[TS 24.301, clause 5.4.3.2]

The MME initiates the NAS security mode control procedure by sending a SECURITY MODE COMMAND message to the UE and starting timer T3460 (see example in figure 5.4.3.2.1).

If the security mode control procedure is initiated further to a successful execution of the authentication procedure, the MME shall use the reset downlink NAS COUNT to integrity protect the SECURITY MODE COMMAND message.

The MME shall send the SECURITY MODE COMMAND message unciphered, but shall integrity protect the message with the NAS integrity key based on KASME or mapped K'ASME indicated by the eKSI included in the message. The MME shall set the security header type of the message to "integrity protected with new EPS security context".

...

The MME shall include the replayed security capabilities of the UE (including the security capabilities with regard to NAS, RRC and UP (user plane) ciphering as well as NAS, RRC integrity, and other possible target network security capabilities, i.e. UTRAN/GERAN if UE included them in the message to network), the replayed nonceUE if the UE included it in the message to the network, the selected NAS ciphering and integrity algorithms and the Key Set Identifier (eKSI).

Additionally, the MME may request the UE to include its IMEISV in the SECURITY MODE COMPLETE message.

NOTE:	The AS and NAS security capabilities will be the same, i.e. if the UE supports one algorithm for NAS it is also be supported for AS.

[TS 24.301, clause 5.4.3.3]

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received UE security capabilities and the received nonceUE have not been altered compared to what the UE provided in the initial layer 3 message that triggered this procedure.

If the type of security context flag is set to "native security context" and if the KSI matches a valid native EPS security context held in the UE while the UE has a mapped EPS security context as the current security context, the UE shall take the native EPS security context into use. The UE shall store the native EPS security context, as specified in annex C.

If the security mode command can be accepted, the UE shall reset the uplink NAS COUNT and the UE shall take the new EPS security context into use when:

a)	the SECURITY MODE COMMAND message is received further to a successful execution of the authentication procedure; or

b)	the type of security context flag is set to "mapped security context" in the NAS KSI IE included in the SECURITY MODE COMMAND message.

If the security mode command can be accepted and the eKSI was included in the SECURITY MODE COMMAND message, the UE shall send a SECURITY MODE COMPLETE message integrity protected with the selected NAS integrity algorithm and the NAS integrity key based on the KASME or mapped K'ASME if the type of security context flag is set to "mapped security context" indicated by the eKSI. If the SECURITY MODE COMMAND message includes the type of security context flag set to "mapped security context" in the NAS KSI IE, nonceMME and nonceUE, the UE shall generate K'ASME from both nonces as indicated in 3GPP TS 33.401 [19] and reset the downlink NAS COUNT to check whether the SECURITY MODE COMMAND can be accepted or not. The UE shall cipher the SECURITY MODE COMPLETE message with the selected NAS ciphering algorithm and the NAS ciphering key based on the KASME or mapped K'ASME indicated by the eKSI. The UE shall set the security header type of the message to "integrity protected and ciphered with new EPS security context".

From this time onwards the UE shall cipher and integrity protect all NAS signalling messages with the selected NAS ciphering and NAS integrity algorithms.

If the MME indicated in the SECURITY MODE COMMAND message that the IMEISV is requested, the UE shall include its IMEISV in the SECURITY MODE COMPLETE message.

9.1.3.1.3	Test description

9.1.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.1.3.1.3.2	Test procedure sequence

Table 9.1.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. It is integrity protected and includes request to include IMEISV (Note 1). | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a SECURITY MODE COMPLETE message and does it establish the initial security configuration? | --> | SECURITY MODE COMPLETE | 1 | P |
| 6Aa1-8Dc1 | Steps 9a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS transmits an IDENTITY REQUEST message ( Security protected as per the algorithms specified in step 5) | <- | IDENTITY REQUEST | - | - |
| 10 | Check: Does the UE transmit an IDENTIY RESPONSE message (Security Protected as per the algorithms specified in step 5)? | -> | IDENTITY RESPONSE | 1 | P |
| 11 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure for new key set generation. | <-- | AUTHENTICATION REQUEST | - | - |
| 12 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 13 | SS resets UL and DL NAS Count to zero | - | - | - | - |
| 14 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. It is integrity protected and includes request to include IMEISV | <-- | SECURITY MODE COMMAND | - | - |
| 15 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE | 2 | P |
| - | Exception: Steps 16 and 17 are executed 100 times to check UE is applying security correctly. | - | - | - | - |
| 16 | The SS transmits an IDENTITY REQUEST message (Security protected as per the algorithms specified in step 14) | <- | IDENTITY REQUEST | - | - |
| 17 | Check: Does the UE transmit an IDENTIY RESPONSE message (Security Protected as per the algorithms specified in step 14)? | -> | IDENTITY RESPONSE | 2 | P |
| 18 A-18D | IF MULTI_PDN = TRUE (Note 2) THEN steps 10-13 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| 19 | The UE is brought in state Switched OFF (state 1) according to TS 36.508 [18] | - | - | - | - |
| 20-29 | Steps 1 to 10 above are executed again null ciphering algorithm requested in step 5. (Note 1) | - | - | - | - |
| 30-33 | IF MULTI_PDN = TRUE (Note 2) THEN steps 10-13 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: This TC verifies the usage of a not null and the null ciphering algorithms. The type of algorithm is specified in the SECURITY MODE COMMAND and then is applied in the messages that follow accordingly. Note 2: MULTI_PDN as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.1.3.1.3.3	Specific message contents

Table 9.1.3.1.3.3-1: SECURITY MODE COMMAND (Steps 5 and 14, Table 9.1.3.1.3.2-1)

| Derivation path: 36.508 [18], table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| Type of ciphering algorithm | Set according to PIXIT parameter for default ciphering algorithm if it is set to a value different to EEA0, or, set to any value different to EEA0 otherwise | Non-zero ciphering algorithm |  |
| IMEISV request | Present |  |  |

Table 9.1.3.1.3.3-2: SECURITY MODE COMPLETE (Steps 6, 15 and 25, Table 9.1.3.1.3.2-1)

| Derivation path: 36.508 [18], table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| IMEISV | Present |  |  |

Table 9.1.3.1.3.3-3: SECURITY MODE COMMAND (Step 24, Table 9.1.3.1.3.2-1)

| Derivation path: 36.508 [18], table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| Type of ciphering algorithm | EEA0 | Zero ciphering algorithm |  |
| IMEISV request | Present |  |  |

#### 9.1.3.2	NAS security mode command not accepted by the UE

9.1.3.2.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure[ }

ensure that {

when { UE receives an integrity protected SECURITY MODE COMMAND message including not matching replayed security capabilities}

then { UE sends SECURITY MODE REJECT and does not start applying the NAS security in both UL and DL}

}

9.1.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 clause 5.4.3.1, 5.4.3.2, 5.4.3.3 and 5.4.3.5.

[TS 24.301, clause 5.4.3.1]

The purpose of the NAS security mode control procedure is to take an EPS security context into use, and initialise and start NAS signalling security between the UE and the MME with the corresponding NAS keys and security algorithms.

[TS 24.301, clause 5.4.3.2]

The MME initiates the NAS security mode control procedure by sending a SECURITY MODE COMMAND message to the UE and starting timer T3460 (see example in figure 5.4.3.2.1).

If the security mode control procedure is initiated further to a successful execution of the authentication procedure, the MME shall use the reset downlink NAS COUNT to integrity protect the SECURITY MODE COMMAND message.

The MME shall send the SECURITY MODE COMMAND message unciphered, but shall integrity protect the message with the NAS integrity key based on KASME or mapped K'ASME indicated by the eKSI included in the message. The MME shall set the security header type of the message to "integrity protected with new EPS security context".

...

The MME shall include the replayed security capabilities of the UE (including the security capabilities with regard to NAS, RRC and UP (user plane) ciphering as well as NAS, RRC integrity, and other possible target network security capabilities, i.e. UTRAN/GERAN if UE included them in the message to network), the replayed nonceUE if the UE included it in the message to the network, the selected NAS ciphering and integrity algorithms and the Key Set Identifier (eKSI).

Additionally, the MME may request the UE to include its IMEISV in the SECURITY MODE COMPLETE message.

NOTE:	The AS and NAS security capabilities will be the same, i.e. if the UE supports one algorithm for NAS it is also be supported for AS.

[TS 24.301, clause 5.4.3.3]

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received UE security capabilities and the received nonceUE have not been altered compared to what the UE provided in the initial layer 3 message that triggered this procedure.

[TS 24.301, clause 5.4.3.5]

If the security mode command cannot be accepted, the UE shall send a SECURITY MODE REJECT message, which shall not be integrity protected. The SECURITY MODE REJECT message contains an EMM cause that typically indicates one of the following cause values:

#23:	UE security capabilities mismatch;

#24:	security mode rejected, unspecified.

.

Upon receipt of the SECURITY MODE REJECT message, the MME shall stop timer T3460. The MME shall also abort the ongoing procedure that triggered the initiation of the NAS security mode control procedure.

9.1.3.2.3	Test description

9.1.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.1.3.2.3.2	Test procedure sequence

Table 9.1.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. It is integrity protected and includes un matched replayed security capabilities. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a NAS SECURITY MODE REJECT message with cause’#23: UE security capabilities mismatch’? | --> | SECURITY MODE REJECT | 1 | P |
| 7 | The SS Transmits an IDENTITY REQUEST message for IMSI (Security not applied) | <- | IDENTITY REQUEST | - | - |
| 8 | Check: Does the UE transmit a non security protected IDENTIY RESPONSE message? | -> | IDENTITY RESPONSE | 1 | P |
| 9 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. It is integrity protected and includes request to include IMEISV | <-- | SECURITY MODE COMMAND | - | - |
| 10 | The UE transmits a SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE | - | - |
| 10Aa1-19b1 | Steps 9a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.1.3.2.3.3	Specific message contents

Table 9.1.3.2.3.3-1: SECURITY MODE COMMAND (Step 5)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Replayed UE security capabilities | Set to mismatch the security capability of UE under test |  |  |

Table 9.1.3.2.3.3-2: SECURITY MODE REJECT (Step 6)

| Derivation path: 36.508 table 4.7.2-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | #23 |  |  |

#### 9.1.3.3	No emergency bearer service / NAS security mode command with EIA0 not accepted by the UE

9.1.3.3.1	Test Purpose (TP)

(1)

with { UE not having a PDN connection for emergency bearer services established or not establishing a PDN connection for emergency bearer }

ensure that {

when { UE receives a SECURITY MODE COMMAND message indicating the "null integrity protection algorithm" EIA0 }

then { UE sends SECURITY MODE REJECT and does not start applying the "null integrity protection algorithm" EIA0 }

}

9.1.3.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 clause 4.4.4.1, 4.4.4.2, 5.4.3.3 and 5.4.3.5.

[TS 24.301, clause 4.4.4.1]

For the UE, integrity protected signalling is mandatory for the NAS messages once a valid EPS security context exists and has been taken into use. For the network, integrity protected signalling is mandatory for the NAS messages once a secure exchange of NAS messages has been established for the NAS signalling connection. Integrity protection of all NAS signalling messages is the responsibility of the NAS. It is the network which activates integrity protection.

The use of "null integrity protection algorithm" EIA0 (see subclause 9.9.3.23) in the current security context is only allowed for an unauthenticated UE. For setting the security header type in outbound NAS messages, the UE and the MME shall apply the same rules irrespective of whether the "null integrity protection algorithm" or any other integrity protection algorithm is indicated in the security context.

[TS 24.301, clause 4.4.4.2]

Except the messages listed below, no NAS signalling messages shall be processed by the receiving EMM entity in the UE or forwarded to the ESM entity, unless the network has established secure exchange of NAS messages for the NAS signalling connection:

-	EMM messages:

-	IDENTITY REQUEST (if requested identification parameter is IMSI);

-	AUTHENTICATION REQUEST;

...

NOTE:	These messages are accepted by the UE without integrity protection, as in certain situations they are sent by the network before security can be activated.

All ESM messages are integrity protected.

Once the secure exchange of NAS messages has been established, the receiving EMM or ESM entity in the UE shall not process any NAS signalling messages unless they have been successfully integrity checked by the NAS. If NAS signalling messages, having not successfully passed the integrity check, are received, then the NAS in the UE shall discard that message. The processing of the SECURITY MODE COMMAND message that has not successfully passed the integrity check is specified in subclause 5.4.3.5. If any NAS signalling message is received as not integrity protected even though the secure exchange of NAS messages has been established by the network, then the NAS shall discard this message.

[TS 24.301, clause 5.4.3.3]

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received replayed UE security capabilities and the received nonceUE have not been altered compared to what the UE provided in the initial layer 3 message that triggered this procedure. However, the UE is not required to perform the checking of the received nonceUE if the UE does not want to re-generate the K'ASME (i.e. the SECURITY MODE COMMAND message is to derive and take into use a mapped EPS security context and the eKSI matches the current EPS security context, if it is a mapped EPS security context). When the UE has a PDN connection for emergency bearer services established or the UE is establishing a PDN connection for emergency bearer services, the UE is not required to locally re-generate the KASME (i.e. the SECURITY MODE COMMAND message is used to derive and take into use a native EPS security context where the KSI value "000" is included in the NAS key set identifier IE and the EIA0 and EEA0 are included as the selected NAS security algorithms).

The UE shall accept a SECURITY MODE COMMAND message indicating the "null integrity protection algorithm" EIA0 as the selected NAS integrity algorithm only if the message is received for a UE that has a PDN connection for emergency bearer services established or a UE that is establishing a PDN connection for emergency bearer services.

[TS 24.301, clause 5.4.3.5]

If the security mode command cannot be accepted, the UE shall send a SECURITY MODE REJECT message. The SECURITY MODE REJECT message contains an EMM cause that typically indicates one of the following cause values:

#23:	UE security capabilities mismatch;

#24:	security mode rejected, unspecified.

Upon receipt of the SECURITY MODE REJECT message, the MME shall stop timer T3460. The MME shall also abort the ongoing procedure that triggered the initiation of the NAS security mode control procedure.

Both the UE and the MME shall apply the EPS security context in use before the initiation of the security mode control procedure, if any, to protect the SECURITY MODE REJECT message and any other subsequent messages according to the rules in subclauses 4.4.4 and 4.4.5.

9.1.3.3.3	Test description

9.1.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

NOTE: The preamble is executed with non-null NAS security algorithms indicated in the PIXT.

9.1.3.3.3.2	Test procedure sequence

Table 9.1.3.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message; EIA0 (NULL integrity), EEA0 (NULL ciphering), matched replayed security capabilities. Note: 'matched replayed security capabilities' shall be sent to ensure that the SECURITY MODE REJECT is not sent due to problem with this information. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a NAS SECURITY MODE REJECT message, integrity protected with previous security context? | --> | SECURITY MODE REJECT | 1 | P |
| 7 | The SS transmits an IDENTITY REQUEST message for IMSI (Security not applied) | <- | IDENTITY REQUEST | - | - |
| 8 | The UE transmits an integrity protected only IDENTITY RESPONSE message. | -> | IDENTITY RESPONSE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - | - | - |
| 9a1 | The SS transmits an ESM INFORMATION REQUEST message - no integrity protection applied - to initiate exchange of protocol configuration options and/or APN. | <-- | ESM INFORMATION REQUEST | - | - |
| 9a2 | Check: Does the UE transmit an ESM INFORMATION RESPONSE message? Note: The UE is expected to discard the ESM INFORMATION REQUEST message without security protection. | --> | ESM INFORMATION RESPONSE | 1 | F |
| 10 | The SS transmits an ATTACH ACCEPT message- no integrity protection applied. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: Steps 11a1 to 25a2 describe behaviour that depends on the UE action; the "lower case letter" identifies a step sequence that take place if a particular sequential line of behaviour is manifested. | - | - | - | - |
| 11a1 | Check: Does the UE transmit an ATTACH COMPLETE message? Note: The UE is expected to discard the ATTACH ACCEPT message without security protection. | --> | ATTACH COMPLETE | 1 | F |
| 11b1 | Check: Does the UE transmit an ATTACH REQUEST message? Note 1: After timer T3411 expire the UE is expected to re-attempt to attach. Note 2: Steps 11b1 to 11b13 are executed with non-null NAS security algorithms indicated in the PIXIT. | --> | ATTACH REQUEST | 1 | P |
| 12-25b1- | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 [18] sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508 [18]. | - | - | - | - |

9.1.3.3.3.3	Specific message contents

Table 9.1.3.3.3.3-1: Void

Table 9.1.3.3.3.3-2: Void

Table 9.1.3.3.3.3-3: SECURITY MODE COMMAND (Step 5, Table 9.1.3.3.3.2-1)

| Derivation path: 36.508 [18], table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| Type of integrity protection algorithm | EIA0 |  |  |
| Type of ciphering algorithm | EEA0 |  |  |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '000'B |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Spare half octet | '0000'B |  |  |

Table 9.1.3.3.3.3-4: SECURITY MODE REJECT (Step 6, Table 9.1.3.3.3.2-1)

| Derivation path: 36.508 [18], table 4.7.2-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | #23 | Note 1 |  |
|  | #24 | Note 1 |  |
| Note 1: Any of these two values is allowed. |  |  |  |

NOTE:	This message is sent within SECURITY PROTECTED NAS MESSAGE message with previous security context.

### 9.1.4	Identification procedure

#### 9.1.4.1	Void

#### 9.1.4.2	Identification procedure / IMEI / IMEISV requested

9.1.4.2.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state / EMM-CONNECTED mode}

ensure that {

when { UE receives an IDENTITY REQUEST message with IMEI in the IE Identity type }

then { UE sends an IDENTITY RESPONSE message providing its IMEI }

}

(2)

with { UE in EMM-REGISTERED state / EMM-CONNECTED mode}

ensure that {

when { UE receives an IDENTITY REQUEST message with IMEISV in the IE Identity type }

then { UE sends an IDENTITY RESPONSE message providing its IMEISV }

}

9.1.4.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.4.4.3.

[TS 24.301, clause 5.4.4.3]

A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in EMM-CONNECTED mode.

Upon receipt of the IDENTITY REQUEST message the UE shall send an IDENTITY RESPONSE message to the network. The IDENTITY RESPONSE message shall contain the identification parameters as requested by the network.

9.1.4.2.3	Test description

9.1.4.2.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.1.4.2.3.2	Test procedure sequence

Table 9.1.4.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an IDENTITY REQUEST message requesting IMEI in the IE Identity type. | <-- | IDENTITY REQUEST | - | - |
| 2 | Check: Does the UE respond with an IDENTITY RESPONSE message providing its IMEI? | --> | IDENTITY RESPONSE | 1 | P |
| 3 | SS transmits an IDENTITY REQUEST message requesting the international mobile equipment identity together with the software version number (IMEISV) in the IE Identity type. | <-- | IDENTITY REQUEST | - | - |
| 4 | Check: Does the UE respond with an IDENTITY RESPONSE message providing its IMEISV? | --> | IDENTITY RESPONSE | 2 | P |

9.1.4.2.3.3	Specific message contents

Table 9.1.4.2.3.3-1: Message IDENTITY REQUEST (step 1, Table 9.1.4.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identity Type | 0010 | IMEI |  |

Table 9.1.4.2.3.3-2: IDENTITY RESPONSE (step 2, Table 9.1.4.2.3.2-1)

| Derivation path: 36.508, Table 4.7.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Mobile Identity |  |  |  |
| Type of identity | 010 | IMEI |  |
| Identity digits | UE's IMEI |  |  |

Table 9.1.4.2.3.3-3: Message IDENTITY REQUEST (step 3, Table 9.1.4.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identity Type | 0011 | IMEISV |  |

Table 9.1.4.2.3.3-4: IDENTITY RESPONSE (step 4, Table 9.1.4.2.3.2-1)

| Derivation path: 36.508, Table 4.7.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Mobile Identity |  |  |  |
| Type of identity | 011 | IMEISV |  |
| Identity digits | UE's IMEISV |  |  |

### 9.1.5	EMM information procedure

#### 9.1.5.1	EMM information procedure

9.1.5.1.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state and UE supporting the EMM information message }

ensure that {

when { UE receives an EMM Information message }

then { UE accepts the message and uses the contents to update appropriate information stored within the UE }

}

9.1.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.4.5.3.

[TS 24.301, clause 5.4.5.3]

When the UE (supporting the EMM information message) receives an EMM INFORMATION message, it shall accept the message and optionally use the contents to update appropriate information stored within the UE.

9.1.5.1.3	Test description

9.1.5.1.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 9.1.5.1.3.1-1

Table 9.1.5.1.3.1-1: USIM configuration

| USIM field | Priority | Value |
| --- | --- | --- |
| EFUST |  | Services 19 and 51 are not supported |

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.1.5.1.3.2	Test procedure sequence

Table 9.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Void | - | - | - | - |
| - | EXCEPTION: Steps 0Aa1 and 0Ab1 describe behaviour that depends on the UE release; the "lower case letter" identifies a step sequence that take place if the UE is of particular release. | - | - | - | - |
| 0Aa1 | IF the UE is (Rel-11 or higher) and (pc_DaylightSavingTime or pc_UniversalAndLocalTimeZone) THEN Switch on Time Zone reporting on the UE (Note 3) | - | - | - | - |
| 0Ab1 | IF the UE is lower than Rel-11 THEN configure the UE with automatic time zone update (Note 0) | - | - | - | - |
| 1 | The SS transmits an EMM INFORMATION message. | <-- | EMM INFORMATION | - | - |
| 2 | Check: Does the UE transmit in the next 5 seconds an EMM STATUS message with cause #97 "message type non-existent or not implemented"? | --> | EMM STATUS | 1 | F |
| - | EXCEPTION: Steps 2Aa1 to 3d2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 2Aa1 | IF ( pc_DaylightSavingTime AND pc_ProvideDST_inUse ) THEN Check: Does the UE assume that this daylight saving time applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? (Note 8) Operator Action: The use of the supported Fields is checked: DaylightSavingTime: 1 (Note 7) | - | - | 1 | P |
| 2Aa2 | Void | - | - | - | - |
| 3a1 | IF pc_FullNameNetwork THEN Check: Does the UE associate the "full length name of the network" with the MCC and MNC contained in the last visited tracking area identification and is presented to the MS user at the earliest opportunity? Operator Action: “FullName12345678” is presented to the MS user. (Note 0, 1) | - | - | 1 | P |
| 3a2 | Void | - | - | - | - |
| 3b1 | IF pc_ShortNameNetwork THEN Check: Does the UE associate the "abbreviated name of the network" with the MCC and MNC contained in the last visited tracking area identification and is presented to the MS user at the earliest opportunity? Operator Action: "SName123" is presented to the MS user (Note 1) | - | - | 1 | P |
| 3b2 | Void | - | - | - | - |
| 3c1 | IF pc_LocalTimeZone THEN Check: Does the UE assume that this time zone applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? Operator Action: The use of the supported Fields is checked: Timezone: GMT+1 (Note 2) | - | - | 1 | P |
| 3c2 | Void | - | - | - | - |
| 3d1 | IF pc_UniversalAndLocalTimeZone THEN Check: Does the UE assume that this time applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? (Note 3, Note 8) Operator Action: The use of the supported Fields is checked: Local Time: Year: <Current Year> Month: December Day: 31st Hour: 14 Hours (Note 4) | - | - | 1 | P |
| 3d2 | Void | - | - | - | - |
| Note 0: AT command +CTZU is assumed to be used. Note 1: AT command +COPS is assumed to be used for check. Note 2: AT command +CCLK is assumed to be used for check. Note 3: AT command +CTZR is assumed to be used for check for Rel-11 onwards. Note 4: Current Year is derived by the SS and the minutes and seconds are not checked. Note 5: Void. Note 6: Void Note 7: If (Rel-11 or higher Release) and Time Zone reporting is enabled at Step 0, the UE returns the result code +CTZE. Note 8: In case of Rel-10 or lower releases: CTZU has been used to trigger UE report. A time zone report in the CTZE structure is expected. Triggering a time zone report in the CTZE structure is only required if pc_DaylightSavingTime or pc_UniversalAndLocalTimeZone are supported. |  |  |  |  |  |

9.1.5.1.3.3	Specific message contents

Table 9.1.5.1.3.3-1: EMM INFORMATION (step 1, Table 9.1.5.1.3.2-1)

| Derivation Path: 36.508 table 4.7.2-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Full name for network | "C63A9BED0CB7CB31D98C56B3DD70" O | "FullName12345678", Note 1 |  |
| Short name for network | "5367B85D8EC966" O | "SName123", Note 1 |  |
| Local time zone | "40" O | "GMT+1", Note 1, Note 2 |  |
| Universal time and local time zone | "xx211331832540" O | "<Current Year> 31 December 13:38:52 GMT+1", Note 1, Note 2 |  |
| Network daylight saving time | "01" O | "+1 hour adjustment for Daylight Saving Time", Note 1 |  |
| Note 1: Hard coded values have been chosen to allow for consistent/comparable SS behaviour. Note 2: Daylight Saving Time is included in the Local Time Zone. |  |  |  |

Table 9.1.5.1.3.3-2: Message EMM STATUS (step 2, Table 9.1.5.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '0110 0001'B | Message type non-existent or not implemented |  |

#### 9.1.5.2	EMM information procedure not supported by the UE

9.1.5.2.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { UE receives an EMM Information message }

then { UE ignore the contents of the message and return an EMM STATUS message with cause #97 "message type non-existent or not implemented" }

}

9.1.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.4.5.3.

[TS 24.301, clause 5.4.5.3]

If the UE does not support the EMM information message the UE shall ignore the contents of the message and return an EMM STATUS message with cause #97 "message type non-existent or not implemented".

9.1.5.2.3	Test description

9.1.5.2.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.1.5.2.3.2	Test procedure sequence

Table 9.1.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an EMM INFORMATION message. | <-- | EMM INFORMATION | - | - |
| 2 | Check: Does the UE transmit an EMM STATUS message with cause #97 "message type non-existent or not implemented". | --> | EMM STATUS | 1 | P |

9.1.5.2.3.3	Specific message contents

Table 9.1.5.2.3.3-1: EMM INFORMATION (step 1, Table 9.1.5.2.3.2-1)

| Derivation Path: 36.508 table 4.7.2-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Full name for network |  | Not present |  |  |
| Short name for network |  | Not present |  |  |
| Local time zone |  | Not present |  |  |
| Universal time and local time zone |  | Not present |  |  |
| Network daylight saving time |  | ‘00’B | No adjustment for Daylight Saving Time |  |

Table 9.1.5.2.3.3-2: Message EMM STATUS (step 2, Table 9.1.5.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '0110 0001'B | Message type non-existent or not implemented |  |

## 9.2	EMM specific procedures

### 9.2.1	Attach procedure

#### 9.2.1.1	Attach procedure for EPS services

##### 9.2.1.1.0	General

NOTE:	Although section 9.2.1.1 is specifically dedicated to Attach for EPS services, it contains also a number of TCs that verify UE behaviour in EPS and Combined attach environments. The extension of the scope of these TCs was decided on a later stage and they were kept in the present section to avoid problems with changing TC numbers which would have been the result if the TCs were moved to another section.

##### 9.2.1.1.1	Attach / Success / Valid GUTI

9.2.1.1.1.1	Test Purpose (TP)

(1)

with { the UE is switched-off with a valid USIM inserted and the USIM contains a valid GUTI and last visited registered TAI }

ensure that {

when { UE is powered on in a cell not belonging to the last visited registered TAI and in a different PLMN }

then { the UE establishes the RRC connection without S-TMSI, with registeredMME and with the RRC establishmentCause set to 'mo-Signalling' }

}

(2)

with { UE is switched-off with a valid USIM inserted and the USIM contains a valid GUTI and last visited registered TAI }

ensure that {

when { UE is powered on  in a cell not belonging to the last visited registered TAI }

then { the UE transmits an ATTACH REQUEST message with the EPS attach type set to "EPS attach", including the GUTI and last visited registered TAI copied from the USIM, and, a PDN CONNECTIVITY REQUEST message with the request type set to "initial request" and not including APN }

}

(3)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH ACCEPT message with EPS attach result matching the requested service(s), the TAI list the UE is registered to and including an ACTIVATE DEFAULT EPS CONTEXT BEARER message with IE EPS Bearer Identity for the default EPS bearer context activated for the UE }

then { UE accepts the allocated GUTI, deletes the old TAI list and transmits an ATTACH COMPLETE message, together with ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT  message and enters EMM-REGISTERED state }

}

9.2.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.1.1, 5.5.1.2.1, 5.5.1.2.2, 5.5.1.2.4, 6.2.2, 6.4.1.3, 6.5.1.2 and Annex D, and TS 36.331, clauses 5.3.3.3 and 5.3.3.4.

[TS 24.301, clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

...

For the routing of the initial NAS message to the appropriate MME, the UE NAS provides the lower layers with either the S-TMSI or the registered globally unique MME identifier (GUMMEI) that consists of the PLMN ID, the MME group ID, and the MME code (see 3GPP TS 23.003 [2]) according to the following rules:

-	When the UE is registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the S-TMSI, but shall not provide the registered MME identifier to the lower layers. Exceptionally, when the UE in EMM-IDLE mode initiates a tracking area updating or combined tracking area updating procedure for load balancing purposes, the UE NAS shall provide the lower layers with neither S-TMSI nor registered MME identifier.

-	When the UE is not registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS does not provide the lower layers with the S-TMSI.  Instead,

a)	if the TIN indicates "GUTI" or "RAT-related TMSI", or the TIN is not available, and the UE holds a valid GUTI, the UE NAS shall provide the lower layers with the MME identifier part of the valid GUTI; or

b)	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE NAS shall provide the lower layers with the MME identifier part of the mapped GUTI, which is generated from the P-TMSI and RAI.

[TS 24.301, clause 5.5.1.1]

The attach procedure is used to attach to an EPC for packet services in EPS.

The attach procedure is used for two purposes:

-	by a UE in PS mode of operation to attach for EPS services only; or

-	by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

With a successful attach procedure, a context is established for the UE in the MME, and a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure.

...

[TS 24.301, clause 5.5.1.2.1]

This procedure is used by a UE to attach for EPS services only. When the UE initiates the EPS attach procedure, the UE shall indicate "EPS attach" in the EPS attach type IE.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

...

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

...

If a valid NAS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message. When the UE does not have a valid NAS security context, the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message is not integrity protected.

...

[TS 24.301, clause 5.5.1.2.4]

...

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

...

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

...

[TS 24.301, clause 6.2.2]

The UE shall set the PDN type IE in the PDN CONNECTIVITY REQUEST message based on its IP stack configuration as follows:

a)	A UE, which is IPv6 and IPv4 capable and

-	has not been allocated an IP address for this APN, shall set the PDN type IE to IPv4v6.

-	has been allocated an IPv4 address for this APN and received the ESM cause #52, "single address bearers only allowed", and is requesting an IPv6 address, shall set the PDN type IE to IPv6.

-	has been allocated an IPv6 address for this APN and received the ESM cause #52, "single address bearers only allowed", and is requesting an IPv4 address, shall set the PDN type IE to IPv4.

b)	A UE, which is only IPv4 capable, shall set the PDN type IE to IPv4.

c)	A UE, which is only IPv6 capable, shall set the PDN type IE to IPv6.

d)	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall set the PDN type IE to IPv4v6.

...

[TS 24.301, clause 6.4.1.3]

Upon receipt of the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, the UE shall send an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message and enter the state BEARER CONTEXT ACTIVE. When the default bearer is activated as part of the attach procedure, the UE shall send the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message together with ATTACH COMPLETE message. When the default bearer is activated as the response to the stand-alone PDN CONNECTIVITY REQUEST message, the UE shall send the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message alone.

The UE checks the PTI in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to identify the UE requested PDN connectivity procedure to which the default bearer context activation is related (see subclause 6.5.1).

...

[TS 24.301, clause 6.5.1.2]

When the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message, the UE shall not include the APN.

NOTE:	If the UE needs to provide PCO which require ciphering or provide an APN, or both, during the attach procedure, the ESM information transfer flag is included in the PDN CONNECTIVITY REQUEST. The MME then at a later stage in the PDN connectivity procedure initiates the ESM information request procedure in which the UE can provide the MME with PCO or APN or both.

...

The UE shall set the request type to "initial request" when the UE is establishing connectivity to a PDN for the first time, i.e. when it is an initial attach to that PDN. The UE shall set the request type to "handover" when the connectivity to a PDN is established upon handover from a non-3GPP access network and the UE was connected to that PDN before the handover to the 3GPP access network.

...

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| Attach | MO signalling (See Note 1) | "originating signalling" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or (if the EHPLMN list is not present or is empty) EHPLMN, (if the EHPLMN list is present) the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3 “Actions related to transmission of RRCConnectionRequest message”]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

[TS 36.331 clause 5.3.3.4]

...

The UE shall:

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, include and set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

...

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

9.2.1.1.1.3	Test description

9.2.1.1.1.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

- 	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell H using default message contents according to TS 36.508 [18].

NOTE: For cell A, (MCC, MNC, TAI) is (MCC stored in EFIMSI, 02, TAI-8).

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.1.3.2	Test procedure sequence

Table 9.2.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’? | - | - | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including PLMN ID, MME group ID and MME code with ATTACH REQUEST message including a GUTI and a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 5-15 | Steps 5-15 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 16 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 2 | P |
| 16a1-18b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 19 | Check: Does the test result of CALL generic procedure [18] clause 6.4.2.4 indicate that the UE is in E-UTRA EMM-REGISTERED state on Cell A? | - | - | 3 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.1.3.3	Specific message contents

Table 9.2.1.1.1.3.3-1: Message RRCConnectionRequest (step 2, Table 9.2.1.1.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| randomValue | Not checked |  |  |
| } |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.1.3.3-2: Message RRCConnectionSetupComplete (step 4, Table 9.2.1.1.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| registeredMME { |  |  |  |
| plmn-Identity | PLMN ID(MCC as stored in EFIMSI on the test USIM card and MNC 02) |  |  |
| mmegi | 1000000000001000 | Bit 0 is LSB |  |
| mmec | 00000001 | Bit 0 is LSB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.1.3.3-3: Message ATTACH REQUEST (step 4, Table 9.2.1.1.1.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI(belonging to PLMN with same MCC as stored in EFIMSI on the test USIM card and MNC 02 ) | GUTI copied from USIM Old and valid GUTI is included by the UE |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | TAI8 | GUTI copied from USIM If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |

Table 9.2.1.1.1.3.3-4: Void

Table 9.2.1.1.1.3.3-5: Void

Table 9.2.1.1.1.3.3-6: Void

Table 9.2.1.1.1.3.3-7: Void

##### 9.2.1.1.1a	Attach Procedure / Success / Last visited TAI, TAI list and equivalent PLMN list handling

9.2.1.1.1a.1	Test Purpose (TP)

(1)

with { UE  attached to the network with a valid USIM inserted and a valid GUTI}

ensure that {

when { UE is powered off and then powered on }

then { the UE transmits an ATTACH REQUEST message with the EPS attach type set to "initial EPS attach", including GUTI and last visited registered TAI and a PDN CONNECTIVITY REQUEST message with the request type set to "initial attach" and not including APN }

}

(2)

with { UE having a valid NAS security context and the UE switched-off }

ensure that {

when { UE is powered on}

then { the UE transmits an integrity protected ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message}

}

(3)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH ACCEPT message with EPS attach result matching the requested service(s), the TAI list the UE is registered to, a set of equivalent PLMNs matching the PLMNs within the TAI list, and including an ACTIVATE DEFAULT EPS CONTEXT BEARER message with IE EPS Bearer Identity for the default EPS bearer context activated for the UE }

then { UE deletes the old TAI list, stores the new TAI list, and does not perform a TAU while moving within this set of TAs }

}

(4)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH ACCEPT message with EPS attach result matching the requested service(s), the TAI list the UE is registered to, a set of equivalent PLMNs matching the PLMNs within the TAI list, and including an ACTIVATE DEFAULT EPS CONTEXT BEARER message with IE EPS Bearer Identity for the default EPS bearer context activated for the UE }

then { UE deletes the old TAI list, stores the new TAI list, and performs a TAU when moving out of this set of TAs}

}

(5)

with { UE has received a set of equivalent PLMNs in an ATTACH ACCEPT message }

ensure that {

when { the UE has been switched off; then switched on; and then the UE receives an ATTACH_ACCEPT message with a new set of equivalent PLMNs}

then { UE deletes the old equivalent PLMN list, and uses the new equivalent PLMN list}

}

9.2.1.1.1a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.3, 5.5.1.2.2, 5.5.1.2.4, 6.5.1.2 and 9.9.3.33, and TS 36.304 clause 4.3.

[TS 24.301, clause 5.3.3]

The UE shall store a list of equivalent PLMNs. These PLMNs shall be regarded by the UE as equivalent to each other for PLMN selection and cell selection/re-selection. The same list is used by EMM, GMM and MM.

The UE shall update or delete this list at the end of each attach or tracking area updating procedure. The stored list consists of a list of equivalent PLMNs as downloaded by the network plus the PLMN code of the registered PLMN that downloaded the list. When the UE is switched off, it shall keep the stored list so that it can be used for PLMN selection after switch on. The UE shall delete the stored list if the USIM is removed. The maximum number of possible entries in the stored list is 16.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see figure 5.5.1.2.2.1).

...

The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN (see subclause 6.5.1).

...

If a valid NAS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message.

[TS 24.301, clause 5.5.1.2.4]

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

…

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity and set its TIN to "GUTI". The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

...

The MME may also include a list of equivalent PLMNs in the ATTACH ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, after having removed from the list any PLMN code that is already in the list of forbidden PLMNs. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the ATTACH ACCEPT message. If the ATTACH ACCEPT message does not contain a list, then the UE shall delete the stored list.

[TS 24.301, clause 5.5.3.2.2, “Normal and periodic tracking area updating procedure initiation”]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

[TS 24.301, clause 6.5.1.2, “UE requested PDN connectivity procedure initiation”]

In order to request connectivity to the default PDN, the UE shall not include any APN in the PDN CONNECTIVITY REQUEST message.

[TS 24.301, clause 9.9.3.33, “Tracking area identity list”]

The Tracking area identity list is a type 4 information element, with a minimum length of 8 octets and a maximum length of 98 octets. The list can contain a maximum of 16 different tracking area identities.

…

The value part of the Tracking area identity list information element consists of one or several partial tracking area identity lists. The length of each partial tracking area identity list can be determined from the 'type of list' field and the 'number of elements' field in the first octet of the partial tracking area identity list.

…

Partial tracking area identity list:

Type of list (octet 1)

Bits

7 6

0 0 list of TACs belonging to one PLMN, with non-consecutive TAC values

0 1 list of TACs belonging to one PLMN, with consecutive TAC values

1 0 list of TAIs belonging to different PLMNs

…

For type of list = "001" and number of elements = k:

octet 2 to 4 contain the MCC+MNC, and

octet 5 and 6 contain the TAC of the first TAI belonging to the partial list.

The TAC values of the other k-1 TAIs are TAC+1, TAC+2, …, TAC+k-1.

…

The MNC shall consist of 2 or 3 digits.

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

- The cell is part of either:

- the selected PLMN, or:

- the registered PLMN, or:

- a PLMN of the Equivalent PLMN list

according to the latest information provided by NAS:

9.2.1.1.1a.3	Test description

9.2.1.1.1a.3.1	Pre-test conditions

System Simulator:

NOTE:	while this test describes the uses of 8 cells, it is intended that this test only requires 2 cells to be active at any one instant.

Table 9.2.1.1.1a-1: Cell TAI values

| Cell | MCC | MNC | TAC (hex) | Remark | Freq | List of frequencies in SIB5 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | 001 | 01 | 0002 | 2 digit MNC | f1 | f2, f3 | HPLMN |
| I | 005 | 102 | 0002 | 3 digit MNC | f3 | f1, f2 | See Note 1 |
| B | 001 | 01 | 0001 |  | f1 | f2, f3 | HPLMN |
| C | 001 | 01 | 0027 |  | f1 | f2, f3 | HPLMN |
| G | 004 | 07 | fff0 |  | f2 | f1, f3 | See Note 2 |
| H | 004 | 07 | fff9 |  | f2 | f1, f3 | See Note 2 |
| K | 006 | 002 | 0003 | 3 digit MNC | f3 | f1, f2 | See Note 1 |
| E | 004 | 02 | 0003 |  | f2 | f1, f3 | See Note 2 |
| Note 1: Cell I and Cell K do not co-exist; the same frequency f3 is used. Note 2: Cell E and Cells G and H do not co-exist; the same frequency f2 is used. |  |  |  |  |  |  |  |

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

-	with the exception of the Physical Cell Identity and the list of frequencies in SIB5, all other parameters for these cells are the same as defined for cell 1 in TS 36.508 [18];

-	The power level of cell A is the Serving Cell level defined in table 6.2.2.1-1 of TS 36.508 [18];

-	The power levels of cells B to K are set to the Non-suitable Off level defined in table 6.2.2.1-1 of TS 36.508 [18].

Table 9.2.1.1.1a-2: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell A | Cell I | Cell B | Cell C | Cell G | Cell H | Cell K | Cell E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | Off | Off | Off | Off | Off | Off | Off |
| T1 | Cell-specific RS EPRE | dBm/ 15kHz | -97 | -85 | Off | Off | Off | Off | Off | Off |
| T2 | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | -85 | Off | Off | Off | Off | Off |
| T3 (N=3) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | -97 | -85 | Off | Off | Off | Off |
| T3 (N=4) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | -97 | -85 | Off | Off | Off |
| T3 (N=5) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | -97 | -85 | Off | Off |
| T3 (N=6) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | Off | -97 | -85 | Off |
| T3 (N=7) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | Off | Off | -97 | -85 |
| T4 | Cell-specific RS EPRE | dBm/ 15kHz | Off | -85 | Off | Off | Off | Off | Off | -97 |
| T5 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | Off | Off | Off | Off | Off | Off | -97 |

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.1a.3.2	Test procedure sequence

Table 9.2.1.1.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |  |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |  |
| 1 | The UE is switched on. | - | - | - | - |  |
| 2-7C | Steps 4-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |  |
| 8 | SS responds with ATTACH ACCEPT message including a valid TAI list containing the TAIs of Cell A and Cell I; with PLMN ID of Cell A included in the GUTI; and with the PLMN ID of Cell I included in the Equivalent PLMNs IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |  |
| 8A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 9-11Ab1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |  |
| 12 | SS adjusts the cell power levels according to row T1 in table 9.2.1.1.1a-2. Note: Cell A is still suitable but the UE shall select Cell I | - | - | - | - |  |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |  |
| 13 | Wait 70 seconds for mobile to camp on Cell I | - | - | - | - |  |
| 14 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |  |
| - | EXCEPTION: Step 14A and 15 describe behaviour that depends on the UE capability. | - | - | - | - |  |
| 14A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 15 | If pc_SwitchOnOff or pc_USIM_Removal (see step 14) then the UE sends DETACH REQUEST message. | --> | DETACH REQUEST | - | - |  |
| 16 | SS adjusts the cell power levels according to row T2 in table 9.2.1.1.1a-2. | - | - | - | - |  |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |  |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |  |
| 18 | Check: Does the UE send an integrity protected ATTACH REQUEST message (including a PDN CONNECTIVITY REQUEST message) with the last visited TAI correctly indicating the TAI of cell I; the GUTI allocated in step 8 and the KSIASME allocated in step 3? | --> | ATTACH REQUEST | 1, 2 | P |  |
| 18A | Void | - | - | - | - |  |
| 18B | Void | - | - | - | - |  |
| - | EXCEPTION: Steps 18a1 to 18a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - | - | - |  |
| 18a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | ESM INFORMATION REQUEST | - | - |  |
| 18a2 | The UE transmits the ESM INFORMATION REQUEST message to transfer protocol configuration options and/or APN. | --> | ESM INFORMATION RESPONSE | - | - |  |
| 19 | The SS sends an ATTACH ACCEPT message allocating 16 TAIs and an aligned set of equivalent PLMNs. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |  |
| - | EXCEPTION: In parallel to the event described in step 20 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |  |
| 20-22b1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |  |
| 23 | The SS waits 5seconds | - | - | - | - |  |
| - | EXCEPTION steps 24 to 26 are repeated for N = 3 to N = 7 with cells C, G, H, K, E according to T3 in table 9.2.1.1.1a-2. | - | - | - | - |  |
| 24 | Cell power levels are set according to T3 and the value of N. | - | - | - | - |  |
| 25 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message in the next 70 seconds? | --> | TRACKING AREA UPDATE REQUEST | 3 | F |  |
| 26 | Check: Does the UE camp on the strongest cell for each T3(x) in table 9.2.1.1.1a-2 being applied by using the procedure in clause 6.4.2.2 of TS 36.508 [18]? | - | - | 3 | - |  |
| 27 | SS adjusts the cell power levels according to row T4 in table 9.2.1.1.1a-2. Note: the new list of equivalent PLMNs allocated in step 19 means that list of equivalent PLMNs allocated in step 8 should have been deleted. Hence the PLMN of Cell I shall not be selected by a cell reselection process, and the UE shall remain camped on Cell E. | - | - | - | - |  |
| 28 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell I in the next 70 seconds? | --> | TRACKING AREA UPDATE REQUEST | 5 | F |  |
| 29 | Check: Does the UE camp on cell E by using the procedure in clause 6.4.2.2 of TS 36.508 [18]? | - | - | 5 | - |  |
| 30 | SS adjusts the cell power levels according to row T5 in table 9.2.1.1.1a-2. | - | - | - | - |  |
| 31 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell A with the last visited TAI set to the TAI of cell E; the GUTI allocated in step 8 and the KSIASME allocated in step 3? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |  |
| 32 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |  |
| 33 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |  |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |  |

9.2.1.1.1a.3.3	Specific message contents

Table 9.2.1.1.1a.3.3-1: Message ATTACH ACCEPT (step 8, Table 9.2.1.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| TAI list |  | List of 2 TAIs |  |
| Length of tracking area identity list contents | 11 | The value in the length field |  |
| Number of elements | 00001 |  |  |
| Type of list | 010 | More than one PLMN |  |
| Partial tracking area identity list | First TAI = TAI of Cell I; Second TAI = TAI of Cell A |  |  |
| GUTI | MCC=001, MNC=01, MMEGI = 1, MMEC= 1, M-TMSI arbitrarily allocated but compliant to rules of TS 23.003 sub clause 2.8 | Includes PLMN ID of cell A |  |
| Equivalent PLMNs | MCC=005, MNC=102 | PLMN ID of cell I |  |

Table 9.2.1.1.1a.3.3-2: Message ATTACH REQUEST (step 18, Table 9.2.1.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Sent in SECURITY PROTECTED NAS MESSAGE with valid integrity check |  |  |  |
| Old GUTI or IMSI | GUTI allocated in step 8 |  |  |
| NAS key set identifier | KSI allocated in step 3 |  |  |
| Last visited registered TAI | TAI of cell I |  |  |

Table 9.2.1.1.1a.3.3-3: Message ATTACH ACCEPT (step 19, Table 9.2.1.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| TAI list |  | Contains 3 separate partial tracking area ID lists |  |
| Length of tracking area identity list contents | 32 | The decimal value of the value in the length field |  |
| Type of first partial tracking area identity list | 010 | More than one PLMN |  |
| Number of elements | 00010 | 3 elements |  |
| First TAI | MCC = 004, MNC = 02, TAC = 0003 |  |  |
| Second TAI | MCC = 005, MNC = 002, TAC = 0003 |  |  |
| Third TAI | MCC = 006, MNC = 002, TAC = 0003 |  |  |
| Type of second partial tracking area identity list | 001 | Consecutive TACs on same PLMN |  |
| Number of consecutive TACS | 01001 | 10 elements |  |
| TAI | MCC = 004 MNC = 07 TAC = fff0 | TAI with lowest numbered TAC |  |
| Type of third partial tracking area identity list | 000 | Individual TACs on same PLMN |  |
| Number of elements | 00010 | 3 |  |
| MCC | MCC = 001 |  |  |
| MNC | MNC = 01 |  |  |
| First TAC | TAC = 0001 |  |  |
| Second TAC | TAC = 0005 |  |  |
| Third TAC | TAC = 0027 |  |  |
| GUTI | MCC=001, MNC = 01, MMEGI = 64000, MMEC= 127, M-TMSI arbitrarily allocated but compliant to rules of TS 23.003 sub clause 2.8 | Includes PLMN ID of cell B. |  |
| Equivalent PLMNs | MCC=004, MNC=02; MCC=004, MNC=03; MCC=004, MNC=07; MCC=006, MNC=002; | 4 equivalent PLMNs |  |

Table 9.2.1.1.1a.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 31, Table 9.2.1.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI allocated in step 19 |  |  |
| NAS key set identifier ASME | Same as allocated in step 3 |  |  |
| Last visited registered TAI | TAI of cell E |  |  |

##### 9.2.1.1.1b	Attach Procedure / Success / Last visited TAI, TAI list and equivalent PLMN list handling / Single Frequency operation

9.2.1.1.1b.1	Test Purpose (TP)

Same Test Purpose as in clause 9.2.1.1.1a.1

9.2.1.1.1b.2	Conformance requirements

Same Conformance requirements as in clause 9.2.1.1.1a.2

9.2.1.1.1b.3	Test description

9.2.1.1.1b.3.1	Pre-test conditions

System Simulator:

-	Eight intra-frequency cells with the PLMNs identified in the test by the identifiers in Table 9.2.1.1.1b-1.

NOTE:	while this test describes the uses of 8 cells, it is intended that this test only requires 2 cells to be active at any one instant.

Table 9.2.1.1.1b-1: Cell TAI values

| Cell | MCC | MNC | TAC (hex) | Remark | Freq | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| A | 001 | 01 | 0002 | 2 digit MNC | f1 | HPLMN |
| L | 005 | 102 | 0002 | 3 digit MNC | f1 |  |
| B | 001 | 01 | 0001 |  | f1 | HPLMN |
| C | 001 | 01 | 0027 |  | f1 | HPLMN |
| G | 004 | 07 | fff0 |  | f1 |  |
| H | 004 | 07 | fff9 |  | f1 |  |
| K | 006 | 002 | 0003 | 3 digit MNC | f1 |  |
| E | 004 | 02 | 0003 |  | f1 |  |

-	System information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

-	with the exception of the Physical Cell Identity, all other parameters for these cells are the same as defined for cell 1 in TS 36.508 [18];

-	the power level of cell A is the Serving Cell level defined in table 6.2.2.1-1 of TS 36.508 [18];

-	the power levels of cells B to K are set to the Non-suitable Off level defined in table 6.2.2.1-1 of TS 36.508 [18].

Table 9.2.1.1.1b-2: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell A | Cell L | Cell B | Cell C | Cell G | Cell H | Cell K | Cell E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | Off | Off | Off | Off | Off | Off | Off |
| T1 | Cell-specific RS EPRE | dBm/ 15kHz | -91 | -85 | Off | Off | Off | Off | Off | Off |
| T2 | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | -85 | Off | Off | Off | Off | Off |
| T3 (N=3) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | -91 | -85 | Off | Off | Off | Off |
| T3 (N=4) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | -91 | -85 | Off | Off | Off |
| T3 (N=5) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | -91 | -85 | Off | Off |
| T3 (N=6) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | Off | -91 | -85 | Off |
| T3 (N=7) | Cell-specific RS EPRE | dBm/ 15kHz | Off | Off | Off | Off | Off | Off | -91 | -85 |
| T4 | Cell-specific RS EPRE | dBm/ 15kHz | Off | -91 | Off | Off | Off | Off | Off | -85 |
| T5 | Cell-specific RS EPRE | dBm/ 15kHz | -85 | Off | Off | Off | Off | Off | Off | Off |

UE:

-	ehe UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.1b.3.2	Test procedure sequence

Same Test procedure sequence as in clause 9.2.1.1.1a.3.2, except for the check in step 28 which cannot be covered due to the specific cell power levels at T4 in Table 9.2.1.1.1b-2. Cell I shall be replaced by Cell L. At step 27 Cell K is set to the Non-suitable “Off” level. The signal strength of Cell L is set to that of a Suitable Neighbour Cell and that of Cell E is set to the Serving Cell level. At step 30 Cells E and L are set to the Non-suitable “Off” level, the signal strength of Cell A is set to the Serving Cell level.

In all steps other than those mentioned before the references to table 9.2.1.1.1a-2 are to be replaced by references to table 9.2.1.1.1b-2.

9.2.1.1.1b.3.3	Specific message contents

Same Specific message contents as in clause 9.2.1.1.1a.3.3. Cell I shall be replaced by Cell L.

##### 9.2.1.1.2	Attach Procedure / Success / With IMSI / GUTI reallocation

9.2.1.1.2.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED state }

ensure that {

when { there is no valid GUTI available in UE }

then { UE sends ATTACH REQUEST message, containing IMSI as the EPS mobile identity }

}

(2)

with { UE having received reallocated GUTI in the ATTACH ACCEPT message }

ensure that {

when { UE detaches from the EPS services }

then { UE sends DETACH REQUEST message, containing GUTI as the EPS mobile identity }

}

9.2.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

[TS 24.301, clause 5.5.1.2.4]

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450. The MME shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

…

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI, the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

For a shared network, the TAIs included in the TAI list can contain different PLMN identities. The MME indicates the selected core network operator PLMN identity to the UE in the GUTI (see 3GPP TS 23.251).If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

9.2.1.1.2.3	Test description

9.2.1.1.2.3.1	Pre-test conditions

System Simulator:

-	cell A (HPLMN).

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (State 2) according to TS 36.508 [18].

9.2.1.1.2.3.2	Test procedure sequence

Table 9.2.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits Paging on cell A with IMSI. Upon reception of paging with IMSI the UE shall locally deactivate any EPS bearer context(s), locally detach from EPS and delete the GUTI-1. After local detach the UE shall perform an EPS attach procedure. | - | - | - | - |
|  | EXCEPTION: Steps 1a describes a behaviour which depends on the UE capability | - | - | - | - |
| 1a | IF NOT pc_Automatic_EPS_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 2 | Check: Does the UE transmit an ATTACH REQUEST message including IMSI in the EPS mobile identity IE including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 3-6Aa2 | Void | - | - | - | - |
| 6B-6H | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 7 | SS responds with ATTACH ACCEPT message with a new GUTI-2 included in the EPS mobile identity IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 8 | Void | - | - | - | - |
| 8A-8Ac1 | Steps 16-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 8B-8D | IF MULTI_PDN = TRUE (NOTE) THEN steps 10-12 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| 9 | Cause UE to detach from the EPS services | - | - | - | - |
| 10 | Check: Does the UE transmit a DETACH REQUEST message including GUTI-2 in the EPS mobile identity IE? | --> | DETACH REQUEST | 2 | P |
| 11 | SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| NOTE: MULTI_PDN as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.1.1.2.3.3	Specific message contents

Table 9.2.1.1.2.3.3-1: ATTACH REQUEST (step 2, Table 9.2.1.1.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.1.1.2.3.3-2: DETACH REQUEST (step 10, Table 9.2.1.1.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI or IMSI | GUTI-2 |  |  |

##### 9.2.1.1.2a	Attach Procedure / AttachWithIMSI configured / Selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs / Success

9.2.1.1.2a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and configured for "AttachWithIMSI" }

ensure that {

when { UE enters a tracking area in a new PLMN that is neither the registered PLMN nor in the list of equivalent PLMNs }

then { UE sends ATTACH REQUEST message, containing IMSI as the EPS mobile identity }

}

9.2.1.1.2a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.2, and, TS 24.368 clauses 4 and 5.4.

[TS 24.301, clause 5.5.1.2.2]

If the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] and the selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs, the UE shall include the IMSI in the EPS mobile identity IE in the ATTACH REQUEST message.

[TS 24.368, clause 4]

The NAS configuration MO is used to manage configuration parameters related to NAS functionality for a UE supporting provisioning of such information. The presence and format of the non access stratum configuration file on the USIM is specified in 3GPP TS 31.102 [6].

...

The following nodes and leaf objects are possible in the NAS configuration MO as described in figure 4-1:

Figure 4-1: The NAS configuration Management Object

[TS 24.368, clause 5.4]

The AttachWithIMSI leaf indicates whether attach with IMSI is performed when moving to a non-equivalent PLMN as specified in 3GPP TS 24.008 [4] and 3GPP TS 24.301 [5].

-	Occurrence: ZeroOrOne

-	Format: bool

-	Access Types: Get, Replace

-	Values: 0, 1

0	Indicates that normal behaviour is applied.

1	Indicates that attach with IMSI is performed when moving to a non-equivalent PLMN.

The default value 0 applies if this leaf is not provisioned.

9.2.1.1.2a.3	Test description

9.2.1.1.2a.3.1	Pre-test conditions

System Simulator:

-	cell A (HPLMN), cell B a different PLMN.

UE:

-	The UE is configured to do "AttachWithIMSI".

The UE is equipped with a USIM containing values shown in Table 9.2.1.1.2a.3.1–1.

Table 9.2.1.1.2a.3.1–1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 is supported. |
| EFNASCONFIG | “Attach with IMSI is performed when moving to a non-equivalent PLMN” as defined in TS 24.368, clause 5.4. |

-	The UE is configured to initiate EPS attach.

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) according to TS 36.508 [18] on cell A. During the registration a list with 2 equivalent PLMNs is provided. The PLMN of cell B is not among them.

9.2.1.1.2a.3.2	Test procedure sequence

Table 9.2.1.1.2a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell B as the "Serving cell". - Cell A as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: Does the UE transmit an ATTACH REQUEST message including IMSI as the EPS mobile identity? The UE includes a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | 1 | P |
| 3-14Bb1 | Steps 5-18b1 from the UE Registration (State 2) procedure described in TS 36.508 [18], Table 4.5.2.3-1 take place. | - | - | - | - |
| 15 | Void | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508 [18]. | - | - | - | - |

9.2.1.1.2a.3.3	Specific message contents

Table 9.2.1.1.2a.3.3-1: ATTACH REQUEST (step 2, Table 9.2.1.1.2a.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |

##### 9.2.1.1.3	Attach Procedure / Success / Request for obtaining the IPv6 address of the home agent

9.2.1.1.3.1	Test Purpose (TP)

(1)

with { UE is configured to request the IPv6 address of the Home Agent during Attach procedure }

ensure that {

when { UE is switched on and has established the RRC connection }

then { UE transmits an ATTACH REQUEST message and a PDN CONNECTIVITY REQUEST message indicating a request for DSMIPv6 Home Agent Address in the protocol configuration options }

}

(2)

with { UE having transmitted an ATTACH REQUEST message and a PDN CONNECTIVITY REQUEST message indicating a request for DSMIPv6 Home Agent Address in the protocol configuration options }

ensure that {

when { the SS responds to the ATTACH REQUEST with an IPv6 Home Agent address }

then { the UE transmits an ATTACH COMPLETE message }

}

9.2.1.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.1 and 6.5.1.2, and TS 24.008, clause 10.5.6.3.

[24.301 clause 5.5.1.1]

During the attach procedure, the UE may also obtain the home agent IPv4 and IPv6 addresses.

[24.301 clause 6.5.1.2]

If the UE supports DSMIPv6, the UE may include a request for obtaining the IPv6 address and optionally the IPv4 address of the home agent in the Protocol configuration options IE in the PDN CONNECTIVITY REQUEST message. The UE may also include a request for obtaining the IPv6 Home Network Prefix. The UE shall request the IPv6 Home Network Prefix only if the UE has requested the home agent IPv6 address. The requested home agent address(es) and the Home Network Prefix are related to the APN the UE requested connectivity for.

[24.008 clause 10.5.6.3]

Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element

Additional parameters list (octets w+1 to z) The additional parameters list is included when special parameters and/or requests (associated with a PDP context) need to be transferred between the MS and the network. These parameters and/or requests are not related to a specific configuration protocol (e.g. PPP), and therefore are not encoded as the "Packets" contained in the configuration protocol options list. The additional parameters list contains a list of special parameters, each one in a separate container. The type of the parameter carried in a container is identified by a specific container identifier. In this version of the protocol, the following container identifiers are specified: MS to network direction: - 0001H (P-CSCF Address Request); - 0002H (IM CN Subsystem Signalling Flag); - 0003H (DNS Server Address Request); - 0004H (Not Supported); - 0005H (MS Support of Network Requested Bearer Control indicator); - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address Request; - 0008H (DSMIPv6 Home Network Prefix Request); - 0009H (DSMIPv6 IPv4 Home Agent Address Request); - 000AH (IP address allocation via NAS signalling); and - 000BH (IPv4 address allocation via DHCPv4). Network to MS direction: - 0001H (P-CSCF Address); - 0002H (IM CN Subsystem Signalling Flag); - 0003H (DNS Server Address); - 0004H (Policy Control rejection code); - 0005H (Selected Bearer Control Mode; - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address) ; - 0008H (DSMIPv6 Home Network Prefix); and - 0009H (DSMIPv6 IPv4 Home Agent Address). If the additional parameters list contains a container identifier that is not supported by the receiving entity the corresponding unit shall be discarded. The container identifier field is encoded as the protocol identifier field and the length of container identifier contents field is encoded as the length of the protocol identifier contents field. When the container identifier indicates P-CSCF Address Request or DNS Server Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IM CN Subsystem Signalling Flag (see 3GPP TS 24.229 [95]), the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. In Network to MS direction this information may be used by the MS to indicate to the user whether the requested dedicated signalling PDP context was successfully established. When the container identifier indicates P-CSCF Address, the container identifier contents field contains one IPv6 address corresponding to a P-CSCF address (see 3GPP TS 24.229 [95]). This IPv6 address is encoded as a 128-bit address according to RFC 3513 [99]. When there is need to include more than one P-CSCF address, then more logical units with container identifier indicating P-CSCF Address are used. When the container identifier indicates DNS Server Address, the container identifier contents field contains one IPv6 DNS server address (see 3GPP TS 27.060 [36a]). This IPv6 address is encoded as a 128-bit address according to RFC 3513 [99]. When there is need to include more than one DNS server address, then more logical units with container identifier indicating DNS Server Address are used. When the container identifier indicates Policy Control rejection code, the container identifier contents field contains a Go interface related cause code from the GGSN to the UE (see 3GPP TS 29.207 [100]). The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates MS Support of Network Requested Bearer Control indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates Selected Bearer Control Mode, the container identifier contents field contains the selected bearer control mode, where ‘01H’ indicates that ‘MS only’ mode has been selected and ‘02H’ indicates that ‘MS/NW’ mode has been selected. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates DSMIPv6 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Network Prefix Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Agent Address, the container identifier contents field contains one IPv6 address corresponding to a DSMIPv6 HA address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 3513 [99]. When the container identifier indicates DSMIPv6 Home Network Prefix, the container identifier contents field contains one IPv6 Home Network Prefix (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 prefix is encoded as an IPv6 address according to RFC 3513 [99] followed by 8 bits which specifies the prefix length. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address, the container identifier contents field contains one IPv4 address corresponding to a DSMIPv6 IPv4 Home Agent address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). When the container identifier indicates IP address allocation via NAS signalling, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IP address allocation DHCPv4, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. NOTE 1: The additional parameters list and the configuration protocol options list are logically separated since they carry different type of information. The beginning of the additional parameters list is marked by a logical unit, which has an identifier (i.e. the first two octets) equal to a container identifier (i.e. it is not a protocol identifier).

9.2.1.1.3.3	Test description

9.2.1.1.3.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to [18];

NOTE:	Any type of attach is acceptable.

-	the UE is configured to request the IPv6 address of the Home Agent in the protocol configuration options IE as part of the Attach / PDN connectivity request procedure.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.3.3.2	Test procedure sequence

Table 9.2.1.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message with IE PROTOCOL CONFIGURATION OPTIONS indicating a DSMIPv6 Home Agent Address Request? | --> | ATTACH REQUEST | 1 | P |
| 3-11 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 12 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message, including the IE PROTOCOL CONFIGURATION OPTIONS indicating a DSMIPv6 Home Agent Address. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 13 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 2 | P |
| 14 | The SS releases the RRC connection. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.3.3.3	Specific message contents

Table 9.2.1.1.3.3.3-1: Message ATTACH REQUEST (step 2, Table 9.2.1.1.3.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ESM message container | PDN CONNECTIVITY REQUEST message as specified in table 9.2.1.1.3.3.3-2. |  |  |

Table 9.2.1.1.3.3.3-2: PDN CONNECTIVITY REQUEST (Table 9.2.1.1.3.3.3-1)

| Derivation path: TS 36.508 table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | The IE Protocol configuration options contains a configuration protocol option = ‘0007H’ (“DSMIPv6 Home Agent Address Request”, length of contents = 0). |  |  |

Table 9.2.1.1.3.3.3-3: Message ATTACH ACCEPT (step 12, Table 9.2.1.1.3.3.2-1)

| Derivation path: TS 36.508 table 4.7.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ESM message container | Contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message specified in table 9.2.1.1.3.3.3-4. |  |  |

Table 9.2.1.1.3.3.3-4: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Table 9.2.1.1.3.3.3-3)

| Derivation path: TS 36.508 table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | The IE Protocol configuration options contains a configuration protocol option = ‘0007H’ (“DSMIPv6 Home Agent Address”, non-zero length), with the value set to the IPv6 address of the Home Agent. |  |  |

##### 9.2.1.1.4	Attach Procedure / Success / Request for obtaining the IPv4 address of the home agent

9.2.1.1.4.1	Test Purpose (TP)

(1)

with { UE is configured to request the DSMIPv6 IPv4 Home Agent Address }

ensure that {

when { UE is switched on and has established the RRC connection }

then { UE transmits an ATTACH REQUEST and a PDN CONNECTIVITY REQUEST message indicating a request for DSMIPv6 IPv4 Home Agent Address in the protocol configuration options }

}

(2)

with { UE having transmitted an ATTACH REQUEST and a PDN CONNECTIVITY REQUEST message indicating a request for DSMIPv6 IPv4 Home Agent Address in the protocol configuration options }

ensure that {

when { SS responds to the ATTACH REQUEST with a DSMIPv6 IPv4 Home Agent Address }

then { UE transmits an ATTACH COMPLETE message }

}

9.2.1.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.1.1, and 6.5.1.2, and  TS 24.008, clause 10.5.6.3.

[24.301 clause 5.5.1.1]

During the attach procedure, the UE may also obtain the home agent IPv4 and IPv6 addresses.

[24.301 clause 6.5.1.2]

If the UE supports DSMIPv6, the UE may include a request for obtaining the IPv6 address and optionally the IPv4 address of the home agent in the Protocol configuration options IE in the PDN CONNECTIVITY REQUEST message. The UE may also include a request for obtaining the IPv6 Home Network Prefix. The UE shall request the IPv6 Home Network Prefix only if the UE has requested the home agent IPv6 address. The requested home agent address(es) and the Home Network Prefix are related to the APN the UE requested connectivity for.

[24.008 clause 10.5.6.3]

Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element

Additional parameters list (octets w+1 to z) The additional parameters list is included when special parameters and/or requests (associated with a PDP context) need to be transferred between the MS and the network. These parameters and/or requests are not related to a specific configuration protocol (e.g. PPP), and therefore are not encoded as the "Packets" contained in the configuration protocol options list. The additional parameters list contains a list of special parameters, each one in a separate container. The type of the parameter carried in a container is identified by a specific container identifier. In this version of the protocol, the following container identifiers are specified: MS to network direction: - 0001H (P-CSCF Address Request); - 0002H (IM CN Subsystem Signalling Flag); - 0003H (DNS Server Address Request); - 0004H (Not Supported); - 0005H (MS Support of Network Requested Bearer Control indicator); - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address Request; - 0008H (DSMIPv6 Home Network Prefix Request); - 0009H (DSMIPv6 IPv4 Home Agent Address Request); - 000AH (IP address allocation via NAS signalling); and - 000BH (IPv4 address allocation via DHCPv4). Network to MS direction: - 0001H (P-CSCF Address); - 0002H (IM CN Subsystem Signalling Flag); - 0003H (DNS Server Address); - 0004H (Policy Control rejection code); - 0005H (Selected Bearer Control Mode; - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address) ; - 0008H (DSMIPv6 Home Network Prefix); and - 0009H (DSMIPv6 IPv4 Home Agent Address). If the additional parameters list contains a container identifier that is not supported by the receiving entity the corresponding unit shall be discarded. The container identifier field is encoded as the protocol identifier field and the length of container identifier contents field is encoded as the length of the protocol identifier contents field. When the container identifier indicates P-CSCF Address Request or DNS Server Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IM CN Subsystem Signalling Flag (see 3GPP TS 24.229 [95]), the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. In Network to MS direction this information may be used by the MS to indicate to the user whether the requested dedicated signalling PDP context was successfully established. When the container identifier indicates P-CSCF Address, the container identifier contents field contains one IPv6 address corresponding to a P-CSCF address (see 3GPP TS 24.229 [95]). This IPv6 address is encoded as a 128-bit address according to RFC 3513 [99]. When there is need to include more than one P-CSCF address, then more logical units with container identifier indicating P-CSCF Address are used. When the container identifier indicates DNS Server Address, the container identifier contents field contains one IPv6 DNS server address (see 3GPP TS 27.060 [36a]). This IPv6 address is encoded as a 128-bit address according to RFC 3513 [99]. When there is need to include more than one DNS server address, then more logical units with container identifier indicating DNS Server Address are used. When the container identifier indicates Policy Control rejection code, the container identifier contents field contains a Go interface related cause code from the GGSN to the UE (see 3GPP TS 29.207 [100]). The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates MS Support of Network Requested Bearer Control indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates Selected Bearer Control Mode, the container identifier contents field contains the selected bearer control mode, where ‘01H’ indicates that ‘MS only’ mode has been selected and ‘02H’ indicates that ‘MS/NW’ mode has been selected. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates DSMIPv6 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Network Prefix Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Agent Address, the container identifier contents field contains one IPv6 address corresponding to a DSMIPv6 HA address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 3513 [99]. When the container identifier indicates DSMIPv6 Home Network Prefix, the container identifier contents field contains one IPv6 Home Network Prefix (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 prefix is encoded as an IPv6 address according to RFC 3513 [99] followed by 8 bits which specifies the prefix length. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address, the container identifier contents field contains one IPv4 address corresponding to a DSMIPv6 IPv4 Home Agent address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). When the container identifier indicates IP address allocation via NAS signalling, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IP address allocation DHCPv4, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. NOTE 1: The additional parameters list and the configuration protocol options list are logically separated since they carry different type of information. The beginning of the additional parameters list is marked by a logical unit, which has an identifier (i.e. the first two octets) equal to a container identifier (i.e. it is not a protocol identifier).

9.2.1.1.4.3	Test description

9.2.1.1.4.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

NOTE:	Any type of attach is acceptable.

- 	the UE is configured to request the DSMIPv6 IPv4 Home Agent Address in the protocol configuration options IE as part of the Attach / PDN connectivity request procedure.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.4.3.2	Test procedure sequence

Table 9.2.1.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message with IE PROTOCOL CONFIGURATION OPTIONS indicating a DSMIPv6 IPv4 Home Agent Address Request? | --> | ATTACH REQUEST | 1 | P |
| 5-13 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 14 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message, including the IE PROTOCOL CONFIGURATION OPTIONS indicating a DSMIPv6 IPv4 Home Agent Address | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 11 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| 15 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 2 | P |
| 16 | The SS releases the RRC connection. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.4.3.3	Specific message contents

Table 9.2.1.1.4.3.3-1: Void

Table 9.2.1.1.4.3.3-2: Message ATTACH REQUEST (step 4, Table 9.2.1.1.4.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ESM message container | PDN CONNECTIVITY REQUEST message as specified in table 9.2.1.1.4.3.3-3. |  |  |

Table 9.2.1.1.4.3.3-3: PDN CONNECTIVITY REQUEST (Table 9.2.1.1.4.3.3-2)

| Derivation path: TS 36.508 table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | The IE Protocol configuration options contains a configuration protocol option = ‘0009H’ (“DSMIPv6 IPv4 Home Agent Address Request”, length of contents = 0). |  |  |

Table 9.2.1.1.4.3.3-4: Message ATTACH ACCEPT (step 11, Table 9.2.1.1.4.3.2-1)

| Derivation path: TS 36.508 table 4.7.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ESM message container | Contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message specified in table 9.2.1.1.4.3.3-5. |  |  |

Table 9.2.1.1.4.3.3-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Table 9.2.1.1.4.3.3-4)

| Derivation path: TS 36.508 table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | The IE Protocol configuration options contains a configuration protocol option = ‘0009H’ (“DSMIPv6 IPv4 Home Agent Address”, non-zero length), with the value set to the IPv4 address of the Home Agent. |  |  |

##### 9.2.1.1.5	Void

##### 9.2.1.1.6	Void

##### 9.2.1.1.7	Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message

9.2.1.1.7.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED-INITIATED state }

ensure that {

when { the UE receives ATTACH ACCEPT message including a list of equivalent PLMNs }

then { the UE stores correctly the list and does not consider forbidden PLMNs as equivalent PLMNs }

}

(2)

with { UE in EMM-REGISTERED-INITIATED state }

ensure that {

when { the UE receives ATTACH ACCEPT message without a list of equivalent PLMNs }

then { the UE deletes the stored list and applies a normal PLMN selection process }

}

9.2.1.1.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.4.

[TS 24.301, clause 5.5.1.2.4]

The MME may also include a list of equivalent PLMNs in the ATTACH ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, after having removed from the list any PLMN code that is already in the list of forbidden PLMNs. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the ATTACH ACCEPT message. If the ATTACH ACCEPT message does not contain a list, then the UE shall delete the stored list.

9.2.1.1.7.3	Test description

9.2.1.1.7.3.1	Pre-test conditions

System Simulator:

-	Cell A (PLMN1, HPLMN), cell G (PLMN2, visited PLMN), cell I (PLMN3, another visited PLMN) and cell J (PLMN4, another visited PLMN, different than cell I and switched OFF) are configured according to table 6.3.2.2-1 in TS 36.508 [18];

-	At most 3 cells are active simultaneously.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell A

UE:

- 	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE last attempted to register on cell I and received reject cause "forbidden PLMN".

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7.3.2	Test procedure sequence

Table: 9.2.1.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell G as a "Non-Suitable cell". - Cell I as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3-9Jb1 | Steps 2-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 10 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. |  |  | - | - |
| - | EXCEPTION: Step 10a1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 10a | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST |  |  |
| 11 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell G as a "Suitable cell". - Cell I as a "Serving cell". Note: Cell I belongs to the forbidden PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 12 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.1.1.7.3.2-2 occurs in parallel with step 13. |  |  |  |  |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 13A-15AKb1 | Steps 5-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 15A | The SS configures: - Cell A as a "Suitable cell". - Cell G as a "Non-Suitable cell". - Cell I as a "Non-suitable "Off” cell". - Cell J as a "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.1.1.7.3.2-3 occurs in parallel with step 15B. |  |  |  |  |
| 15B | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 15C | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15D | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE |  |  |
| 15E | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 15Fa1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 15Fa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 16-34 | Void | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle (E1) according to TS 36.508. | - | - | - | - |

Table: 9.2.1.1.7.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an ATTACH REQUEST message on Cell I (PLMN3)? | --> | ATTACH REQUEST | 1 | F |

Table: 9.2.1.1.7.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell J (PLMN4)? | --> | TRACKING AREA UPDATE REQUEST | 2 | F |

,

9.2.1.1.7.3.3	Specific message contents

Table 9.2.1.1.7.3.3-1: SystemInformationBlockType5 for Cell A

| Derivation path: 36.508 Table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 3 entries |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell G |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell I |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[3] | Same downlink EARFCN as used for Cell J |  |  |
| dl-CarrierFreq[3] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell G |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell I |  |  |
| dl-CarrierFreq-v9e0[3] | Same downlink EARFCN as used for Cell J |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 9.2.1.1.7.3.3-2: Message ATTACH ACCEPT (step 8, Table 9.2.1.1.7.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN2, PLMN3 and PLMN4. |  |

Table 9.2.1.1.7.3.3-3: Message ATTACH ACCEPT (step 14, Table 9.2.1.1.7.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | Not present |  |  |

##### 9.2.1.1.7a	Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Single Frequency operation

9.2.1.1.7a.1	Test Purpose (TP)

Same Test Purpose as in clause 9.2.1.1.7.1

9.2.1.1.7a.2	Conformance requirements

Same Conformance requirements as in clause 9.2.1.1.7.2

9.2.1.1.7a.3	Test description

9.2.1.1.7a.3.1	Pre-test conditions

System Simulator:

-	Cell A (PLMN1, HPLMN), cell B (PLMN2, visited PLMN), and cell C (PLMN3, another visited PLMN)

-	The cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 36.508 [18], and are configured on same frequency f1;

-	System information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell A

UE:

- 	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE last attempted to register on cell C and received reject cause "forbidden PLMN".

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7a.3.2	Test procedure sequence

Table: 9.2.1.1.7a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Non-Suitable cell". - Cell C as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3-9Ib1 | Steps 2-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 10 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. |  |  | - | - |
| - | EXCEPTION: Step 10a1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 10a | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST |  |  |
| 11 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Non-Suitable cell ". - Cell C as a "Serving cell". Note: Cell C belongs to the forbidden PLMN. | - | - | - | - |
| 12 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 12a | Check: Does the UE transmit an ATTACH REQUEST message in the next 60 seconds on Cell C | - | - | 1 | F |
| 12b | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Serving cell". - Cell C as a "Non-Suitable cell". |  |  |  |  |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| - | EXCEPTION: Steps 13Aa1 to 13Aa2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - | - | - |
| 13Aa1-15AGb1 | Steps 9a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 15A | The SS configures: - Cell A as a " Serving cell ". - Cell B as a "Non-Suitable cell". - Cell C as a "Non-suitable "Off” cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 15B | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 15C | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15D | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE |  |  |
| 15 E | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 15Fa1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 15 Fa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 16-34 | Void | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.7a.3.3	Specific message contents

Table 9.2.1.1.7a.3.3-1: Message ATTACH ACCEPT (step 8, Table 9.2.1.1.7a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN2, PLMN3 and PLMN4. |  |

Table 9.2.1.1.7a.3.3-2: Message ATTACH ACCEPT (step 14, Table 9.2.1.1.7a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | Not present |  |  |

##### 9.2.1.1.7b	Attach / Success / native GUMMEI

9.2.1.1.7b.1	Test Purpose (TP)

(1)

with { the UE is switched-off with a valid USIM inserted and the USIM contains a valid GUTI and last visited registered TAI }

ensure that {

when { UE is powered on in a cell not belonging to the last visited registered TAI and in a different PLMN }

then { the UE establishes the RRC connection without S-TMSI, with registeredMME and with the RRC establishmentCause set to 'mo-Signalling' }

}

(2)

with { UE is switched-off with a valid USIM inserted and the USIM contains a valid GUTI and last visited registered TAI }

ensure that {

when { UE is powered on  in a cell not belonging to the last visited registered TAI }

then { the UE completes the RRC establishment with an indication that the MME identifier is a native GUMMEI and the UE transmits an ATTACH REQUEST message with the EPS attach type set to "EPS attach", including the GUTI and last visited registered TAI copied from the USIM }

}

9.2.1.1.7b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.1.1, 5.5.1.2.1, 5.5.1.2.2, 5.5.1.2.4 and Annex D, and TS 36.331, clause 5.3.3.4.

[TS 24.301, clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

...

For the routing of the initial NAS message to the appropriate MME, the UE NAS provides the lower layers with either the S-TMSI or the registered globally unique MME identifier (GUMMEI) that consists of the PLMN ID, the MME group ID, and the MME code (see 3GPP TS 23.003 [2]) according to the following rules:

-	When the UE is registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the S-TMSI, but shall not provide the registered MME identifier to the lower layers. Exceptionally, when the UE in EMM-IDLE mode initiates a tracking area updating or combined tracking area updating procedure for load balancing purposes, the UE NAS shall provide the lower layers with neither S-TMSI nor registered MME identifier.

-	When the UE is not registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS does not provide the lower layers with the S-TMSI. Instead,

a)	if the TIN indicates "GUTI" or "RAT-related TMSI", or the TIN is not available, and the UE holds a valid GUTI, the UE NAS shall provide the lower layers with the MME identifier part of the valid GUTI with an indication that the identifier is a native GUMMEI; or

b)	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE NAS shall provide the lower layers with the MME identifier part of the mapped GUTI, which is generated from the P-TMSI and RAI with an indication that the identifier is a mapped GUMMEI.

[TS 24.301, clause 5.5.1.1]

The attach procedure is used to attach to an EPC for packet services in EPS.

The attach procedure is used for two purposes:

-	by a UE in PS mode of operation to attach for EPS services only; or

-	by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

With a successful attach procedure, a context is established for the UE in the MME, and a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure.

...

[TS 24.301, clause 5.5.1.2.1]

This procedure is used by a UE to attach for EPS services only. When the UE initiates the EPS attach procedure, the UE shall indicate "EPS attach" in the EPS attach type IE.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

...

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

...

If a valid NAS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message. When the UE does not have a valid NAS security context, the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message is not integrity protected.

...

[TS 24.301, clause 5.5.1.2.4]

...

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

...

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

...

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| Attach | MO signalling (See Note 1) | "originating signalling" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or (if the EHPLMN list is not present or is empty) EHPLMN, (if the EHPLMN list is present) the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.4 “Reception of the RRCConnectionSetup by the UE”]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

…

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>		if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

…

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

9.2.1.1.7b.3	Test description

9.2.1.1.7b.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell E, using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7b.3.2	Test procedure sequence

Table 9.2.1.1.7b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’? | - | - | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including PLMN ID, MME group ID and MME code with ATTACH REQUEST message including a GUTI and a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 5-19 | Steps 5-19, from Table 9.2.1.1.1.3.2-1 in test case 9.2.1.1.1, are performed. |  |  |  |  |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.7b.3.3	Specific message contents

Table 9.2.1.1.7b.3.3-1: Message RRCConnectionRequest (step 2, Table 9.2.1.1.7b.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| randomValue | Not checked |  |  |
| } |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.7b.3.3-2: Message RRCConnectionSetupComplete (step 4, Table 9.2.1.1.7b.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| registeredMME { |  |  |  |
| plmn-Identity | PLMN ID(MCC and MNC as stored in EFIMSI on the test USIM card) |  |  |
| mmegi | 1000000000001001 | Bit 0 is LSB |  |
| mmec | 00000001 | Bit 0 is LSB |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | native |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.7b.3.3-3: Message ATTACH REQUEST (step 4, Table 9.2.1.1.7b.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI(belonging to PLMN with same MCC/MNC as stored in EFIMSI on the test USIM card) | GUTI copied from USIM Old and valid GUTI is included by the UE |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | TAI12 | GUTI copied from USIM If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |

##### 9.2.1.1.7c	Attach / Success / PSM

9.2.1.1.7c.1	Test Purpose (TP)

(1)

with { the UE is switched-off with a valid USIM inserted and the UE is configured to attach with PSM }

ensure that {

when { UE is powered on }

then { the UE transmits an ATTACH REQUEST message including the T3324 IE set to a configured value }

}

(2)

with { the UE in IDLE mode }

ensure that {

when { UE receives a paging message before timer T3324 is expired }

then { the UE responds to the paging request }

}

9.2.1.1.7c.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.11, 5.5.1.2.2.

[TS 24.301, clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

NOTE:	Timer T3324 is specified in 3GPP TS 24.008 [13].

Upon expiry of the timer T3324 or if the T3324 value provided by the network is zero, the UE may deactivate the AS layer and activate PSM by entering the state EMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is not attached for emergency bearer services;

b)	the UE has no PDN connection for emergency bearer services;

c)	the UE is in EMM-IDLE mode; and

d)	in the EMM-REGISTERED.NORMAL-SERVICE state.

If conditions a, b and c are fulfilled, but the UE is in a state other than EMM-REGISTERED.NORMAL-SERVICE when timer T3324 expires, the UE may activate PSM when the MS returns to state EMM-REGISTERED.NORMAL-SERVICE.

A UE that has already been allocated timer T3324 with a value different from "deactivated" and the timer T3324 has expired, may activate PSM if it receives an "Extended wait time" from lower layers.

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

The UE may deactivate PSM at any time (e.g. for the transfer of mobile originated signalling or user data), by activating the AS layer before initiating the necessary EMM procedures. When PSM is activated all NAS timers are stopped and associated procedures aborted except for T3412, T3346 and T3396.

[TS 24.301, clause 5.5.1.2.2]

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

9.2.1.1.7c.3	Test description

9.2.1.1.7c.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode

-	The UE is configured to use a specific value of T3324

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7c.3.2	Test procedure sequence

Table 9.2.1.1.7c.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | - | - | - | - |
| 3 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including the T3324 IE set to two minutes? | --> | ATTACH REQUEST | 1 | P |
| 5 -13Eb1 | Steps 5-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
|  | The SS shall wait for 1 minute and then execute the following steps before timer T3324 expires. |  |  |  |  |
| 14 - 24 | Check: Does the UE accept the paging request? Steps 1-10, from Table 4.5.3.3-1 in TS 36.508, are performed. | - | - | 2 | P |

9.2.1.1.7c.3.3	Specific message contents

Table 9.2.1.1.7c.3.3-1: Void

Table 9.2.1.1.7c.3.3-2: Message ATTACH REQUEST (step 4, Table 9.2.1.1.7c.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI(belonging to PLMN with same MCC as stored in EFIMSI on the test USIM card) | GUTI copied from USIM Old and valid GUTI is included by the UE |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | TAI1 | GUTI copied from USIM If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| T3324 | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.1.1.7c.3.3-4: Void

Table 9.2.1.1.7c.3.3-5: Message ATTACH ACCEPT (step 10, Table 9.2.1.1.7c.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3324 | T3324 timer value | Timer set to the same value as in table 9.2.1.1.7c.3.3-2 |  |
| T3412 extended value | Value received in ATTACH REQUEST | Included only if the IE 'T3412 extended value' was provided in the ATTACH REQUEST |  |

##### 9.2.1.1.7d	Attach / Success / DCN

9.2.1.1.7d.1	Test Purpose (TP)

(1)

with { the UE is switched-off with a valid USIM inserted and the UE is pre-configured with a Default DCN-ID}

ensure that {

when { UE is powered on }

then { the UE transmits an RRCConnectionSetupComplete message including the dcn-ID IE set to the Default DCN-ID }

}

(2)

with { the UE in IDLE mode having received a DCN-ID in a previous ATTACH ACCEPT message}

ensure that {

when { UE receives a paging message }

then { the UE transmits an RRCConnectionSetupComplete message including the dcn-ID IE set to the received DCN-ID }

}

9.2.1.1.7d.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.1.1 and 5.5.1.2.4 and in TS 36.331, clause 5.3.3.4.

[TS 24.301, clause 5.3.1.1]

…

When an ATTACH REQUEST message, or a TRACKING AREA UPDATE REQUEST message when the TAI of the current cell is not included in the TAI list, is sent to establish a signalling connection, the UE NAS also provides the lower layers with the DCN-ID according to the following rules:

a)	if a DCN-ID for the PLMN code of the selected PLMN is available in the UE, the UE NAS shall provide this DCN-ID to the lower layers; or

b)	if no DCN-ID for the PLMN code of the selected PLMN is available but a Default_DCN_ID value is available in the UE, as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17], the UE NAS shall provide this DCN-ID to the lower layers.

…

[TS 24.301, clause 5.5.1.2.4]

…

If the ATTACH ACCEPT message contains the DCN-ID IE, then the UE shall store the included DCN-ID value together with the PLMN code of the registered PLMN in a DCN-ID list in a non-volatile memory in the ME as specified in annex C.

…

[TS 36.331, clause 5.3.3.4]

The UE shall:

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

2>	discard the stored UE AS context and resumeIdentity;

2>	indicate to upper layers that the RRC connection resume has been fallbacked;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	for NB-IoT, if stored, discard the dedicated frequency offset provided by the redirectedCarrierOffsetDedicated;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI:

4>	set the s-TMSI to the value received from upper layers;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

2>	if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>	if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

2>	if the UE supports CIoT EPS optimisation(s):

3>	include attachWithoutPDN-Connectivity if received from upper layers;

3>	include up-CIoT-EPS-Optimisation if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

2>	if connecting as an RN:

3>	include the rn-SubframeConfigReq;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	except for NB-IoT:

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4>	include rlf-InfoAvailable;

3>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailableMBSFN;

3>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailable;

3>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4>	include connEstFailInfoAvailable;

3>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include the mobilityHistoryAvail;

2>	include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers;

2>	if UE needs UL gaps during continuous uplink transmission:

3>	include ue-CE-NeedULGaps;

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

9.2.1.1.7d.3	Test description

9.2.1.1.7d.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	the UE is configured to initiate EPS attach;

The UE is equipped with a USIM containing values shown in Table 9.2.1.1.7d.3.1-1.

Table 9.2.1.1.7d.3.1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 is supported. |
| EFNASCONFIG | “Default_DCN_ID value” provisioned with the value 0, as defined in TS 24.368, clause 5.10e. |

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7d.3.2	Test procedure sequence

Table 9.2.1.1.7d.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmit an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’? | - | - | - | - |
| 3 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including the dcn-ID IE set to the default DCN ID provisioned in the UE? | --> | ATTACH REQUEST | 1 | P |
| 5 -13Eb1 | Steps 5-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 14 - 18 | Steps 1-5, from Table 4.5.3.3-1 in TS 36.508, are performed. Check: Does the UE transmit an RRCConnectionSetupComplete message including the dcn-ID IE set to the DCN ID received in step 10? | - | - | 2 | P |
| 19 | The SS transmits a SERVICE REJECT message with the EMM cause set to 'Illegal UE'. | <-- | SERVICE REJECT | - | - |
| 20 | The SS releases the RRC connection. | - | - | - | - |

9.2.1.1.7d.3.3	Specific message contents

Table 9.2.1.1.7d.3.3-1: Message RRCConnectionSetupComplete (step 2, Table 9.2.1.1.7d.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| dcn-ID-r14 | Integer value | The Default DCN-ID as defined in the USIM |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.7d.3.3-2: Message ATTACH ACCEPT (step 10, Table 9.2.1.1.7d.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DCN-ID | Integer value | Any allowed value different from the Default DCN-ID in the USIM |  |

Table 9.2.1.1.7d.3.3-3: Message RRCConnectionSetupComplete (step 18, Table 9.2.1.1.7d.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| dcn-ID-r14 | Integer value | The same DCN-ID value as used in Table 9.2.1.1.7d.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.1.1.8	Void

##### 9.2.1.1.9	Attach / Rejected / IMSI invalid

9.2.1.1.9.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Illegal UE" }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE receives an ATTACH REJECT message with the reject cause set to "Illegal UE" }

ensure that {

when { the UE has been switched off, then switched on }

then { the UE sends an ATTACH REQUEST message with IMSI, including a PDN CONNECTIVITY REQUEST message }

}

9.2.1.1.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.2.5.

[TS 24.301, clause 5.5.1.2.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the reject cause value received.

#3		(Illegal UE); or

...

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall consider the USIM as invalid for EPS services and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

NOTE:	The possibility to configure a UE so that the radio transceiver for a specific RAT is not active, although it is implemented in the UE, is out of scope of the present specification.

9.2.1.1.9.3	Test description

9.2.1.1.9.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and If (px_SinglePLMN_Tested = Multi PLMN) cell G:

-	Cell A and Cell B (home PLMN, different TAs),

-	If (px_SinglePLMN_Tested = Multi PLMN), Cell G (another PLMN);

-	the different cells may not be simultaneously activated (at most 2 cells are active simultaneously).

UE:

-	the UE is configured to initiate EPS attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.9.3.2	Test procedure sequence

Table 9.2.1.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - cell A as the "Serving cell". - cell B as a "Non-Suitable cell". - If present, Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on cell A. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "Illegal UE" as specified. | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? Note: Cell B belongs to the same PLMN where the UE was rejected but a different TA | --> | ATTACH REQUEST | 1 | F |
| 8 | The operator initiates an attach by MMI or by AT command. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 10 | Void | - | - | - | - |
| 11 | Check: Does the test result of the generic procedure defined in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging on cell B for PS domain with S-TMSI included in GUTI-1? | - | - | 1 | - |
| - | EXCEPTION: Steps 12a1 to 12a4 describe behaviour that depends on the network capability / preference; the "lower case letter" identifies a step sequence that takes place if the network is capable or prefers. | - | - | - | - |
| 12 a1 | IF (px_SinglePLMN_Tested = Multi PLMN) The SS configures: - Cell A as a “Non-Suitable Off cell”. - Cell B as a "Non-Suitable cell". - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 12a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? Note: Cell G belongs to a PLMN which is not the same like the one on which the UE was rejected. | --> | ATTACH REQUEST | 1 | F |
| 12a3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 12a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 13-15 | Void | - | - | - | - |
| 16 | if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. |  |  |  |  |
| 16A | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". -If present Cell G as a "Non-Suitable off cell". | - | - | - | - |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 17A | The following messages are to be observed on Cell B unless explicitly stated otherwise | - | - | - | - |
| 18 | Void | - | - | - | - |
| 19 | Check: Does the UE transmit an ATTACH REQUEST message with IMSI-1? | --> | ATTACH REQUEST | 2 | P |
| 20-33 b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.9.3.3	Specific message contents

Table 9.2.1.1.9.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00000011 | #3 "Illegal UE" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.9.3.3-2: Message ATTACH REQUEST (step 19, Table 9.2.1.1.9.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 |  |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | Not present |  |  |
| Old LAI | Not present |  |  |
| TMSI status | Not present |  |  |

##### 9.2.1.1.10	Attach / Rejected / Illegal ME

9.2.1.1.10.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Illegal ME" }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE receives an ATTACH REJECT message with the reject cause set to "Illegal ME" }

ensure that {

when { the UE has been switched off, then switched on }

then { the UE sends an ATTACH REQUEST message with IMSI, including a PDN CONNECTIVITY REQUEST message }

}

9.2.1.1.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.2.5.

[TS 24.301, clause 5.5.1.2.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the reject cause value received.

#6		(Illegal ME);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall consider the USIM as invalid for EPS services and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

9.2.1.1.10.3	Test description

The test description is identical to the one of subclause 9.2.1.1.9 except that in the test procedure and in the specific message contents, the reject cause #3 "Illegal UE" is replaced with the reject cause #6 "Illegal ME".

##### 9.2.1.1.11	Attach / Rejected / EPS services and non-EPS services not allowed

9.2.1.1.11.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "EPS services and non-EPS services not allowed" }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE receives an ATTACH REJECT message with the reject cause set to "EPS services and non-EPS services not allowed" }

ensure that {

when { the UE has been switched off, then switched on }

then { the UE sends an ATTACH REQUEST message with IMSI, including a PDN CONNECTIVITY REQUEST message }

}

(3)

with { UE receives an ATTACH REJECT message with the reject cause set to "EPS services and non-EPS services not allowed" }

ensure that {

when { the UE has been switched off, then switched on and a UMTS or GSM cell is found }

then { the UE sends an ATTACH REQUEST message with IMSI }

}

9.2.1.1.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.2.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.5.1.2.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the reject cause value received.

#8		(EPS services and non-EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall consider the USIM as invalid for EPS services and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

[TS 24.008, clause 4.7.3.2.4]

The MS shall then take one of the following actions depending upon the reject cause:

# 8		(GPRS services and non-GPRS services not allowed);

...

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (shall store it according to subclause 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The new GMM state is GMM-DEREGISTERED. The new MM state is MM IDLE.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The SIM/USIM shall be considered as invalid for GPRS and non-GPRS services until switching off or the SIM/USIM is removed.

9.2.1.1.11.3	Test description

The test description is identical to the one of subclause 9.2.1.1.9 with the following exceptions:

9.2.1.1.11.3.1	Pre-test conditions

System Simulator:

-	cell A, Cell B (home PLMN, different TAs) and if (px_SinglePLMN_Tested = Multi PLMN) Cell G TS 36.508 (another PLMN)

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN) ;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (is configured);

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (is configured);

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 9 nor cell 24 is configured;

- 	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 9 using default message contents according to TS 36.508 [18].

- 	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.11.3.2	Test procedure sequence

Table 9.2.1.1.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Non-Suitable cell". - If present, Cell G as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services and non-EPS services not allowed" as specified. | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? Note: Cell B belongs to the same PLMN where the UE was rejected but a different TA | --> | ATTACH REQUEST | 1 | F |
| 8 | The operator initiates an attach by MMI or by AT command. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 10 | Void | - | - | - | - |
| 11 | Check: Does the test result of the generic procedure defined in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging on cell B for PS domain with S-TMSI included in GUTI-1? | - | - | 1 | - |
| - | EXCEPTION: Steps 12a1 to 12a4 describe behaviour that depends on the network capability / preference; the "lower case letter" identifies a step sequence that takes place if the network is capable or prefers. | - | - | - | - |
| 12a1 | IF (px_SinglePLMN_Tested = Multi PLMN) The SS configures: - Cell A as a "Non-Suitable “off" cell". - Cell B as a "Non-Suitable cell". - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 12a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? Note: Cell G belongs to a PLMN which is not the same like the one on which the UE was rejected. | --> | ATTACH REQUEST | 1 | F |
| 12a3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 12a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 13-15 | Void | - | - | - | - |
| 16 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 16A | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". -If present, Cell G as a "Non-Suitable off cell". | - | - | - | - |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Void | - | - | - | - |
| - | The following messages are sent on Cell B. | - | - | - | - |
| 19 | Check: Does the UE transmit an ATTACH REQUEST message with IMSI-1? | --> | ATTACH REQUEST | 2 | P |
| 19A-19D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 20 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services and non-EPS services not allowed" as specified. | <-- | ATTACH REJECT | - | - |
| 20A | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Steps 21a1 to 21a10 describe behaviour that depends on the UE capability. | - | - | - | - |
| 21a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS sets - Cell B as a "Non-Suitable cell". - Cell 24 (GERAN) or Cell 9 (UTRAN) as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 24 or Cell 9 (depending on which cell was set as "Serving cell" in the preceding step) unless explicitly stated otherwise. | - | - | - | - |
| 21a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 21a3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 21a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 21a5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 21a6 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | EXCEPTION: Steps 21a6a1, 21a6a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that take place if pc_AutomaticAttachSwitchON is NOT supported | - | - | - | - |
| 21a6Aa1 | IF NOT pc_AutomaticAttachSwitchON | - | Registration on CS | - | - |
| 21a6Aa2 | IF NOT pc_AutomaticAttachSwitchON the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 21a7 | Void | - | - | - | - |
| 21a8 | Check: Does the UE transmit an ATTACH REQUEST message with IMSI-1? | --> | ATTACH REQUEST | 3 | P |
| 22 | The attach procedure is completed | - | - | - | - |
| 23 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA | - | - | - | - |
| 23a1 | The SS releases the RRC connection |  |  |  |  |
| 23a2 | The generic test procedure in TS 36.508 subclause 4.5A.3A.2 is performed | - | - | - | - |
| - | If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN); At the end of this test procedure sequence, the UE is in end state either U1_IDLE or G1_IDLE E-UTRA connected (E2) according to TS 36.508. Else: At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

9.2.1.1.11.3.3	Specific message contents

Table 9.2.1.1.11.3.3-1: Message ATTACH REJECT (steps 4 and 20, Table 9.2.1.1.11.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001000 | #8 "EPS services and non-EPS services not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.11.3.3-2: Message ATTACH REQUEST (step 19, Table 9.2.1.1.11.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 |  |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | Not present |  |  |
| Old LAI | Not present |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.1.11.3.3-3: Message ATTACH REQUEST (step 21a8, Table 9.2.1.1.11.3.2-1)

| Derivation path: TS 24.008 table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS network capability | Not checked |  |  |
| Attach type | Not checked |  |  |
| GPRS ciphering key sequence number | 111 | No key is available |  |
| DRX parameter | Not checked |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | Deleted RAI-1 | The MNC and the MCC in the coding of the RAI are replaced by the RAC of RAC-1 |  |
| MS Radio Access capability | Not checked |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer Value | Not checked |  |  |
| TMSI status | Not checked |  |  |
| PS LCS Capability | Not checked |  |  |
| Mobile station classmark 2 | Not checked |  |  |
| Mobile station classmark 3 | Not checked |  |  |
| Supported Codecs | Not checked |  |  |
| UE network capability | Not checked |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

##### 9.2.1.1.12	Attach / Rejected / EPS services not allowed

9.2.1.1.12.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "EPS services not allowed" }

then { UE deletes the GUTI and the last visited registered TAI and KSI and considers the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed and deletes the list of equivalent PLMNs and UE enters state EMM-DEREGISTERED }

}

9.2.1.1.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.2.2 and 5.5.1.2.5.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

…

[TS 24.301, clause 5.5.1.2.5]

…

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the reject cause value received.

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [6] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

9.2.1.1.12.3	Test description

9.2.1.1.12.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell G:

-	cell A and cell B (HPLMN, different TAs), if (px_SinglePLMN_Tested = Multi PLMN) cell G (another PLMN);

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN) ;

-	if pc_UTRA, AND px_RATComb_Tested = EUTRA_UTRA cell 9 (HPLMN, UTRAN);

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (HPLMN, GERAN);

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 9 nor cell 24 is configured;

- 	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	the different cells may not be simultaneously activated (at most 2 cells are active simultaneously);

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 9 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.12.3.2	Test procedure sequence

Table 9.2.1.1.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a " Suitable neighbour cell". - If present, Cell G as a "Non-Suitable Off cell". - If present, Cell 9 or 24 as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The user switches the UE on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services not allowed". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS reconfigures: Cell A as a " Suitable neighbour cell". Cell B as the "Serving cell". Note: Cell A and Cell B are in different TAIs – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| - | EXCEPTION: Steps 10a1 to 10a6 describe behaviour that depends on the UE capability. | - | - | - | - |
| 10a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS configures - Cell A as "Non-suitable Off cell" - Cell B as a " Suitable neighbour cell". - Cell 24 (GERAN) or 9 (UTRAN) as the "Serving cell". Note: Cell B and Cell 24 (GERAN) or 9 (UTRAN) are in different PLMNs | - | - | - | - |
| 10a2 | Void | - | - | - | - |
| - | EXCEPTION: The following messages are to be observed on Cell 24 or Cell 9 (depending on which cell was set as "Serving cell" in the preceding step) unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: Depending on UE capabilities the behaviour in table 9.2.1.1.12.3.2-2 occurs in parallel with step 10a3. | - | - | - | - |
|  | EXCEPTION: The behaviour in table 9.2.1.1.12.3.2-2 occurs in parallel with step 10a3. |  |  |  |  |
| 10a3 | Check: Does the UE transmit an ATTACH REQUEST message in the next 2 minutes? | --> | ATTACH REQUEST | 1 | F |
| 10a4 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 10a5 | Check : Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 10a6 | The SS configures cell 24 or cell 9 as " Non-Suitable Off cell". | - | - | - | - |
| - | EXCEPTION: Steps 11a1 to 11a4 describe behaviour that depends on the network capability / preference; the "lower case letter" identifies a step sequence that takes place if the network is capable or prefers. | - | - | - | - |
| 11a1 | If (px_SinglePLMN_Tested = Multi PLMN) The SS sets: - Cell B as a "Non-Suitable Off cell". - Cell G as the "Serving cell". Note: Cell B and Cell G are different PLMNs. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 11a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 60 seconds? | --> | ATTACH REQUEST | 1 | F |
| 11a3 | The operator initiates an attach by MMI or by AT command. | - | - | - | - |
| 11a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

Table 9.2.1.1.12.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a 5 describe a behaviour which depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_CS THEN the UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 1a2a1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| 1a2a2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 1a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a5 | The SS transmits a LOCATION UPDATING ACCEPT message including IMSI-1 | <-- | LOCATION UPDATING ACCEPT | - | - |

9.2.1.1.12.3.3	Specific message contents

Table 9.2.1.1.12.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.1.12.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00000111 | #7 "EPS services not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.12.3.3-2: LOCATION UPDATING ACCEPT (step 5, Table 9.2.1.1.12.3.2-2)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity |  |  |  |
| IMSI | IMSI-1 |  |  |

##### 9.2.1.1.13	Attach / Rejected / PLMN not allowed

9.2.1.1.13.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { the UE receives an ATTACH REJECT message with the reject cause set to "PLMN not allowed" }

then { the UE deletes the GUTI, the last visited registered TAI, KSI, the list of equivalent PLMNs and UE enters state EMM-DEREGISTERED.PLMN-SEARCH and UE stores the PLMN in the "forbidden PLMN list" in the USIM }

}

(2)

with { the UE is switched off and a PLMN is stored in the "forbidden PLMN list" in the USIM }

ensure that {

when { the UE is switched on  }

then { the UE doesn't  attempt to attach on this PLMN }

}

(3)

with { the UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { the UE detects a cell belonging to a PLMN which is not in the "forbidden PLMN list" }

then { the UE attaches to this PLMN }

}

(4)

with {the  UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { the forbidden PLMN is selected manually }

then { the UE attaches to the forbidden PLMN and deletes this PLMN from the USIM}

}

9.2.1.1.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.2.2 and 5.5.1.2.5.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI or IMSI IE as follows:

...

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI or IMSI IE.

...

-	Otherwise the UE shall include the IMSI in the Old GUTI or IMSI IE.

...

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

...

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

...

#11	(PLMN not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. Additionally, the UE shall delete the list of equivalent PLMNs and reset the attach attempt counter, and enter state EMM-DEREGISTERED.PLMN-SEARCH.

In S1 mode, the UE shall store the PLMN identity in the "forbidden PLMN list" and enter state EMM-DEREGISTERED.PLMN-SEARCH. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

...

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value and no RR connection exists.

9.2.1.1.13.3	Test description

9.2.1.1.13.3.1	Pre-test conditions

System Simulator:

-	cell G, cell H (VPLMN, same MCC like HPLMN, different TAs) and cell I (VPLMN, different MCC from HPLMN);

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1;

-	the cells may not be simultaneously activated.

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell G using default message contents according to TS 36.508 [18];

-	the "forbidden PLMN list" is empty;

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.13.3.2	Test procedure sequence

Table 9.2.1.1.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell G as the "Serving cell". - Cell H as a " Suitable Neighbour cell". - Cell I as a "Non-Suitable Off cell". Note: Cell G and Cell H are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message including EMM cause = "PLMN not allowed". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message on cell G or H in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 10 | The SS configures: - Cell G as the "Serving cell" - Cell H as a "Non-Suitable Off cell" - Cell I as a " Suitable neighbour cell". | - | - | - | - |
| 11 | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1, 3 | P |
| 12A-12M | The attach procedure is completed and the RRC connection is released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - |  |
| 13 - 19 | Void | - | - | - | - |
| 20 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 21 describes behaviour that depends on the UE capability. |  |  |  |  |
| 21 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 22 | The SS configures: - Cell G as the "Serving cell" - Cell I as a "Non-suitable Off cell". Note: Cell G belongs to the forbidden PLMN. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 24 | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 25 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. | - | - | - | - |
| 26 | The user selects PLMN of cell G. | - | - | - | - |
| 27 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 4 | P |
| 28-40 | The attach procedure is completed and the RRC connection is released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA manual selection (E5) according to TS 36.523-3. | - | - | - | - |

9.2.1.1.13.3.3	Specific message contents

Table 9.2.1.1.13.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001011 | #11 "PLMN not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.13.3.3-2: Message ATTACH REQUEST (step 12, Table 9.2.1.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 5; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 5. |  |

Table 9.2.1.1.13.3.3-3: Message ATTACH REQUEST (step 27, Table 9.2.1.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI allocated in step 18 |  |  |
| Last visited registered TAI | TAI-9 | TAI 9 is allocated on Cell I according to 36.508 table 6.3.2.2-1 |  |

##### 9.2.1.1.13a	Attach / Rejected / PLMN not allowed / Single Frequency operation

9.2.1.1.13a.1	Test Purpose (TP)

Same test purpose as in clause 9.2.1.1.13.1

9.2.1.1.13a.2	Conformance requirements

Same conformance requirements as in clause 9.2.1.1.13.2

9.2.1.1.13a.3	Test description

9.2.1.1.13a.3.1	Pre-test conditions

System Simulator:

-	cell A(belongs to TAI-7, visited PLMN, MCC = MCC in USIM  MNC=02),

-	cell B (belongs to TAI-8, visited PLMN, MCC = MCC in USIM  MNC=02)) and

-	cell C (belongs to TAI-9, visited PLMN, MCC=002 MNC=101);

-	the cells may not be simultaneously activated.

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

-	the "forbidden PLMN list" is empty;

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.13a.3.2	Test procedure sequence

Table 9.2.1.1.13a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a " Suitable Neighbour cell". - Cell C as a "Non-Suitable Off cell". Note: Cell A and Cell B are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message including EMM cause = "PLMN not allowed". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message on cell A or B in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 10 | The SS configures: - Cell A as the " Suitable neighbour intrafrequency cell" - Cell B as a "Non-Suitable Off cell" - Cell C as a " Serving cell". | - | - | - | - |
| 11 | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1, 3 | P |
| 12A-12M | The attach procedure is completed and the RRC connection is released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - |  |
| 13-19 | Void | - | - | - | - |
| 20 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 21 describes behaviour that depends on the UE capability. |  |  |  |  |
| 21 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 22 | The SS configures: - Cell A as the "Serving cell" - Cell C as a "Non-suitable Off cell". Note: Cell A belongs to the forbidden PLMN. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 24 | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 25 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. | - | - | - | - |
| 26 | The user selects PLMN of cell A. | - | - | - | - |
| 27 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 4 | P |
| 28-40 | The attach procedure is completed and the RRC connection is released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.13a.3.3	Specific message contents

Table 9.2.1.1.13a.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.1.13a.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001011 | #11 "PLMN not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.13a.3.3-2: Message ATTACH REQUEST (step 12, Table 9.2.1.1.13a.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 5; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 5. |  |

Table 9.2.1.1.13a.3.3-3: Message ATTACH REQUEST (step 27, Table 9.2.1.1.13a.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI allocated in step 18 |  |  |
| Last visited registered TAI | TAI-9 | TAI 9 is allocated on Cell C according to 36.508 table 6.3.2.2-1 |  |

##### 9.2.1.1.14	Attach / Rejected / Tracking area not allowed

9.2.1.1.14.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Tracking area not allowed" }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, UE deletes the GUTI, last visited registered TAI and KSI, UE enters the state EMM-DEREGISTERED.LIMITED-SERVICE and UE stores the current TAI in the list of "forbidden tracking areas for regional provision of service" }

}

(2)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { serving cell belongs to TAI where UE was rejected }

then { UE does not attempt to attach on any other cell }

}

(3)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE re-selects a new cell in the same TAI it was already rejected }

then { UE does not attempt to attach }

}

(4)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE enters a cell belonging to a tracking area not in the list of "forbidden tracking areas for regional provision of service"}

then { UE attempts to attach with IMSI }

}

(5)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the list of "forbidden tracking areas for regional provision of service" contains more than one TAI}

ensure that {

when { UE re-selects a cell belonging to one of the TAIs in the list of "forbidden tracking areas for regional provision of service" }

then { UE does not attempt to attach }

}

(6)

with { UE is switched off }

ensure that {

when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for regional provision of service" before the UE was switched off }

then { UE performs registration on that cell }

}

9.2.1.1.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.2, 5.5.1.2.2, 5.5.1.2.5, 5.2.2.3.2, Annex C and TS 36.304 clause 4.3.

[TS 24.301, clause 5.3.2]

The UE shall store a list of "forbidden tracking areas for roaming", as well as a list of "forbidden tracking areas for regional provision of service". These lists shall be erased when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours).

...

In S1 mode, the UE shall update the suitable list whenever an ATTACH REJECT, TRACKING AREA UPDATE REJECT, SERVICE REJECT or DETACH REQUEST message is received with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area".

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

...

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI or IMSI IE as follows:

...

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI or IMSI IE.

...

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

...

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

...

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

...

#12	(Tracking area not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. Additionally, the UE shall reset the attach attempt counter.

In S1 mode, the UE shall store the current TAI in the list of "forbidden tracking areas for regional provision of service" and enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

...

[TS 24.301, clause 5.2.2.3.2]

The UE shall perform an attach or combined attach procedure when entering a cell which provides normal service.

[TS 24.301, Annex C (normative)]

The following EMM parameters shall be stored on the USIM if the corresponding file is present:

-	GUTI;

-	last visited registered TAI;

-	EPS update status;

-	Allowed CSG list; and

-	EPS security context parameters from a full native EPS security context (see 3GPP TS 33.401 [19]).

The presence and format of corresponding files on the USIM is specified in 3GPP TS 31.102 [17].

If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM. The allowed CSG list is stored in a non-volatile memory in the ME if the UE supports CSG selection. These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

…

[TS 36.304, clause 4.3]

…

suitable cell:

…

Following exceptions to these definitions are applicable for UEs:

-	camped on a cell that belongs to a registration area that is forbidden for regional provision of service; a cell that belongs to a registration area that is forbidden for regional provision service ([5], [16]) is suitable but provides only limited service.

…

9.2.1.1.14.3	Test description

9.2.1.1.14.3.1	Pre-test conditions

System Simulator:

-	cell A (home PLMN) and cell B (home PLMN, another TA) are configured according to table 6.3.2.2-1 in TS 36.508 [18] and belong to the same frequency as specified in TS 36.523-3 [20].

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.14.3.2	Test procedure sequence

Table 9.2.1.1.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Suitable Neighbour cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message, EMM cause = "Tracking area not allowed". (The list of "forbidden tracking areas for regional provision of service " in the UE should now contain TAI-1) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds on Cell A or Cell B? | --> | ATTACH REQUEST | 1, 2 | F |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 9 | The SS reconfigures: Cell A as a "Suitable Neighbour cell", Cell B as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 10 | Check: Does the UE transmit the ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1,4 | P |
| 10A-10D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 11 | The SS transmits an ATTACH REJECT message, EMM cause = "Tracking area not allowed". (The list of "forbidden tracking areas for regional provision of service " in the UE should now contain TAI-1 and TAI-2) | <-- | ATTACH REJECT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| 13 | The SS reconfigures: Cell A as the "Serving cell". Cell B as a " Suitable Neighbour cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 14 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1, 3, 5 | F |
| 15 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 16 | The SS reconfigures: Cell A as the "Serving cell", Cell B as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Void | - | - | - | - |
| 19 | Check: Does the UE transmit the ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1, 6 | P |
| 20-32 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.14.3.3	Specific message contents

Table 9.2.1.1.14.3.3-1: Message ATTACH REJECT (steps 4 and 11, Table 9.2.1.1.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001100 | #12 "Tracking area not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.14.3.3-2: Message ATTACH REQUEST (steps 10 and 18 Table 9.2.1.1.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 4; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 4. |  |

##### 9.2.1.1.15	Attach / Rejected / Roaming not allowed in this tracking area

9.2.1.1.15.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { the UE receives an ATTACH REJECT message with the reject cause set to "roaming not allowed in this tracking area" }

then { the UE sets the EPS update status to EU3 ROAMING NOT ALLOWED and the UE deletes the GUTI, the last visited registered TAI and KSI and the UE enters the state EMM-DEREGISTERED.LIMITED-SERVICE or optionally EMM-DEREGISTERED.PLMN-SEARCH and the UE stores the current TAI in the list of "forbidden tracking areas for roaming" }

}

(2)

with { the UE is in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-DEREGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE enters a cell belonging to a tracking area not in the list of "forbidden tracking areas for roaming"}

then { the UE attempts to attach with IMSI }

}

(3)

with { the UE is in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-DEREGISTERED.PLMN-SEARCH state and the list of "forbidden tracking areas for roaming" contains more than one TAI}

ensure that {

when { the UE selects a cell belonging to one of the TAIs in the list of "forbidden tracking areas for roaming" }

then { the UE does not attempt to attach }

}

(4)

with { the UE is switched off or the UICC containing the USIM is removed }

ensure that {

when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for roaming" before the UE was switched off or the USIM is inserted again on that cell }

then { UE performs registration on that cell }

}

(5)

with { the UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { the UE receives an ATTACH REJECT message with the reject cause set to "roaming not allowed in this tracking area" }

then { the UE performs a PLMN selection }

}

9.2.1.1.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.2, 5.5.1.2.2, 5.5.1.2.5 and in TS 36.304 clause 5.2.4.4.

[TS 24.301, clause 5.3.2]

The UE shall store a list of "forbidden tracking areas for roaming", as well as a list of "forbidden tracking areas for regional provision of service". These lists shall be erased when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours).

...

In S1 mode, the UE shall update the suitable list whenever an ATTACH REJECT, TRACKING AREA UPDATE REJECT, SERVICE REJECT or DETACH REQUEST message is received with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area".

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

...

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message. If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI or IMSI IE as follows:

...

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI or IMSI IE.

...

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

...

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

...

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

...

#13	(Roaming not allowed in this tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall delete the list of equivalent PLMNs and reset the attach attempt counter.

In S1 mode, the UE shall store the current TAI in the list of "forbidden tracking areas for roaming". Additionally, the UE shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE or optionally EMM-DEREGISTERED.PLMN-SEARCH. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

...

[TS 36.304 subclause 5.2.4.4]

...

If the highest ranked cell is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

...

9.2.1.1.15.3	Test description

9.2.1.1.15.3.1	Pre-test conditions

System Simulator:

-	cell C (home PLMN), cell I (visited PLMN) and cell E (same visited PLMN, another TA), but at most two cells are simultaneously activated.

NOTE 1:	Cell E shall not use the same frequency as cell I.

NOTE 2:	The requirement in 3GPP TS 24.301 to store at least 40 entries in the list of "forbidden tracking areas for roaming" is not fully tested.

NOTE 3:	Different types of UE may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12 am) for roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell I using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.15.3.2	Test procedure sequence

Table 9.2.1.1.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell I as the "Serving cell". cell E as a " Non-Suitable Off cell", cell C as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on cell I unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area ". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1,5 | F |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1,5 | F |
| - | The following messages are to be observed on Cell E unless explicitly stated otherwise. | - | - | - | - |
| 8A | The SS configures: Cell I as the "Serving cell". Cell E as a "Suitable Neighbour cell", Cell C as a "Non-Suitable Off cell". | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on Cell E as specified? Note: according to TS 24.301, the UE has the choice to enter "LIMITED-SERVICE" or "PLMN- SEARCH" state. But in any case it shall do a PLMN selection. In the first option, the UE shall apply reselection so it will select cell E and then attempt to attach; in the second option it will select the same PLMN again and exclude cells from forbidden TAs so it will select cell E. | --> | ATTACH REQUEST | 1, 2 | P |
| 9A-9D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 10 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9 and TAI-12) | <-- | ATTACH REJECT | - | - |
| 11 | The SS releases the RRC connection. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message in the next 60 seconds on Cell I or Cell E? | --> | ATTACH REQUEST | 1, 3 | F |
| 13 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 14 | The SS reconfigures: Cell I as the "Serving cell", Cell E as a "Non-Suitable Off cell", Cell C as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 15 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 16 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified. | --> | ATTACH REQUEST | 4 | P |
| 16A-16D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 17 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area ". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 18 | The SS reconfigures: Cell I as the "Serving cell", Cell E as a "Non-Suitable Off cell", Cell C as a "Suitable neighbour cell". | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 19 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 5 | P |
| 20-32 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.15.3.3	Specific message contents

Table 9.2.1.1.15.3.3-1: Message ATTACH REJECT (steps 4, 10 and 17 in table 9.2.1.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001101 | #13 " roaming not allowed in this tracking area " |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.15.3.3-2: Message ATTACH REQUEST (steps 9,12, 16 and 19 in table 9.2.1.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI1 | GUTI has been deleted after receiving ATTACH REJECT at step 4; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 4. |  |

##### 9.2.1.1.15a	Attach / Rejected / Roaming not allowed in this tracking area / Single Frequency operation

9.2.1.1.15a.1	Test Purpose (TP)

Same test purpose as in clause 9.2.1.1.15.1.

9.2.1.1.15a.2	Conformance requirements

Same conformance requirements as in clause 9.2.1.1.15.2

9.2.1.1.15a.3	Test description

9.2.1.1.15a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency cells.

-	cell C (home PLMN), cell B (visited PLMN, TAI-9) and cell A (same visited PLMN, TAI-12), but at most two cells are simultaneously activated.

NOTE 1:	The requirement in 3GPP TS 24.301 to store at least 40 entries in the list of "forbidden tracking areas for roaming" is not fully tested.

NOTE 2:	Different types of UE may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12 am) for roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

UE:

-	the UE is configured to initiate EPS attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell B using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.15a.3.2	Test procedure sequence

Table 9.2.1.1.15a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell B as the "Serving cell". cell A as a " Non-Suitable Off cell", cell C as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area ". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 1,5 | F |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 1,5 | F |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 8A | The SS configures: Cell A as the "Serving cell". Cell B as a "Suitable Neighbour intrafrequency cell", Cell C as a "Non-Suitable Off cell". | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on Cell A as specified? Note: according to TS 24.301, the UE has the choice to enter "LIMITED-SERVICE" or "PLMN- SEARCH" state. But in any case it shall do a PLMN selection. In the first option, the UE shall apply reselection so it will select cell A and then attempt to attach; in the second option it will select the same PLMN again and exclude cells from forbidden TAs so it will select cell A. | --> | ATTACH REQUEST | 1, 2 | P |
| 9A-9D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 10 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9 and TAI-12) | <-- | ATTACH REJECT | - | - |
| 11 | The SS releases the RRC connection. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Cell B or Cell A? | --> | ATTACH REQUEST | 1, 3 | F |
| 13 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 14 | The SS reconfigures: Cell B as the "Serving cell", Cell A as a "Non-Suitable Off cell", Cell C as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 15 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 16 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified. | --> | ATTACH REQUEST | 4 | P |
| 16A-16D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 17 | The SS transmits an ATTACH REJECT message, EMM cause = "roaming not allowed in this tracking area ". (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 18 | The SS reconfigures: Cell B as the " Suitable neighbour intrafrequency cell", Cell A as a "Non-Suitable Off cell", Cell C as a " Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 19 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 5 | P |
| 20-32 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.15a.3.3	Specific message contents

Same specific message contents as clause 9.2.1.1.15.3.3

##### 9.2.1.1.16	Attach / Rejected / EPS services not allowed in this PLMN

9.2.1.1.16.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "EPS services not allowed in this PLMN" }

then { UE deletes any GUTI, last visited registered TAI, KSI and enters EMM-DEREGISTERED.PLMN-SEARCH state}

}

(2)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE detects a cell which belongs to a PLMN which is in the "forbidden PLMNs for GPRS service list" }

then { UE doesn’t perform an attach procedure }

}

(3)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE detects a cell which belongs to a PLMN which is not in the "forbidden PLMNs for GPRS service list" }

then { UE performs an attach procedure }

}

(4)

with { UE is switched off when a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE is powered on a cell which belongs to this PLMN }

then { UE performs an attach procedure }

}

(5)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE is in the cell which belongs to the rejected PLMN and when that PLMN is selected manually }

then { UE performs an attach procedure }

9.2.1.1.16.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.2.5 and TS 23.122, clause 3.1.

[TS 24.301 clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value. If the attach procedure fails due to a default EPS bearer setup failure, an ESM procedure failure, or operator determined barring is applied on default EPS bearer context activation during attach procedure, the MME shall combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message contained in the ESM message container information element. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19 "ESM failure".

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

…

#14	(EPS services not allowed in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI.

In S1 mode, the UE shall store the PLMN identity in the "forbidden PLMNs for GPRS service" list. Additionally, the UE shall enter state EMM-DEREGISTERED.PLMN-SEARCH. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6]:

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

…

[TS 23.122 clause 3.1]

…

If a message with cause value "GPRS services not allowed in this PLMN" is received by an MS in response to an GPRS attach, GPRS detach, routing area update, attach or tracking area update request (see 3GPP TS 24.008 [23] and 3GPP TS 24.301 [23A]) from a VPLMN, that VPLMN is added to a list of "forbidden PLMNs for GPRS service" which is stored in the MS and thereafter that VPLMN will not be accessed by the MS for GPRS service when in automatic mode. This list is deleted when the MS is switched off or when the SIM is removed. A PLMN is removed from the list of "forbidden PLMNs for GPRS service" if, after a subsequent manual selection of that PLMN, there is a successful GPRS attach. The maximum number of possible entries in this list is implementation dependant, but must be at least one entry. The HPLMN (if the EHPLMN list is not present or is empty) or an EHPLMN (if the EHPLMN list is present) shall not be stored on the list of "forbidden PLMNs for GPRS service".

…

9.2.1.1.16.3	Test description

9.2.1.1.16.3.1	Pre-test conditions

System Simulator:

-	cell G, Cell H and cell I are configured according to Table 6.3.2.2-1 in; [18].

-	cell G and Cell H with MCC-1/MNC-2 (visited PLMN, different TAs)

-	Cell I with MCC-2/MNC-101 (visited PLMN)

-	the cells may not be simultaneously activated;

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1.

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell G using default message contents according to TS 36.508 [18];

-	the "forbidden PLMNs for GPRS service list" is empty.

-	the different cells may not be simultaneously activated (at most 2 cells are active simultaneously).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

            - Test procedure sequence

Table 9.2.1.1.16.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell G as the "Serving cell". - Cell H as a "Non-Suitable cell". - Cell I as a "Non-Suitable off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 5 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8 | The SS configures: Cell G as a "Non-Suitable cell". Cell H as the "Serving cell". Note: Cell G and Cell H are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell H unless explicitly stated otherwise. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 11 | The SS configures: Cell H as a "Non-Suitable off cell". Cell I as the "Serving cell". Note: Cell G and Cell I are different PLMNs. | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message including and a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 3 | P |
| 13A | The SS completes the attach procedure successfully and then releases the RRC -connection by executing steps 5 to 17 of UE registration procedure in TS 36.508 clause 4.5.2.3. |  |  |  |  |
| 14 - 20 | Void | - | - | - | - |
| 21 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 22 | The SS configures - Cell I as a "Non-Suitable cell". - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. |  |  |  |  |
| 24 | Void | - | - | - | - |
| 25 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 25A-25D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 26 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 26A | SS releases the RRC connection. | - | - | - | - |
| 27 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. |  |  |  |  |
| 28 | The user selects PLMN (MCC-1/MNC-2) |  |  |  |  |
| 29 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 5 | P |
| 30-42 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.16.3.3	Specific message contents

Table 9.2.1.1.16.3.3-1: Message ATTACH REJECT (step 5, Table 9.2.1.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 (Plain NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001110 | #14 "EPS services not allowed in this PLMN " |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.16.3.3-2: Message ATTACH REQUEST (step 13 and step 29, Table 9.2.1.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 5 and step25; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 5 and step26. |  |

Table 9.2.1.1.16.3.3-3: Message ATTACH REJECT (step 26, Table 9.2.1.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001110 | #14 "EPS services not allowed in this PLMN " |  |
| ESM message container | Not present |  |  |

##### 9.2.1.1.16a	Attach / Rejected / EPS services not allowed in this PLMN / Single Frequency operation

9.2.1.1.16a.1	Test Purpose (TP)

Same test purpose as in clause 9.2.1.1.16.1

9.2.1.1.16a.2	Conformance requirements

Same conformance requirements as in clause 9.2.1.1.16.2

9.2.1.1.16a.3	Test description

9.2.1.1.16a.3.1	Pre-test conditions

System Simulator:

-	cell A(TAI-7), Cell B(TAI-8) and intra-frequency cells cell C(TAI-9) are configured according to Table 6.3.2.2-1 in; [18].

-	cell A and Cell B with MCC-1/MNC-2 (visited PLMN, different TAs)

-	Cell C with MCC-2/MNC-101 (visited PLMN)

-	the cells may not be simultaneously activated.

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

-	the "forbidden PLMNs for GPRS service list" is empty.

-	the different cells may not be simultaneously activated (at most 2 cells are active simultaneously).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.16a.3.2	Test procedure sequence

Table 9.2.1.1.16a.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Non-Suitable cell". - Cell C as a "Non-Suitable off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 5 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8 | The SS configures: Cell A as a "Non-Suitable cell". Cell B as the "Serving cell". Note: Cell A and Cell B are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 11 | The SS configures: Cell B as a "Non-Suitable off cell". Cell C as the "Serving cell". Note: Cell A and Cell C are different PLMNs. | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message including and a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 3 | P |
| 13A | The SS completes the attach procedure successfully and then releases the RRC -connection by executing steps 5 to 17 of UE registration procedure in TS 36.508 clause 4.5.2.3. |  |  |  |  |
| 14 - 20 | Void | - | - | - | - |
| 21 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 22 | The SS configures - Cell C as a "Non-Suitable cell". - Cell A as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. |  |  |  |  |
| 24 | Void | - | - | - | - |
| 25 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 25A-25D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 26 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 26A | SS releases the RRC connection. | - | - | - | - |
| 27 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. |  |  |  |  |
| 28 | The user selects PLMN of cell A |  |  |  |  |
| 29 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 5 | P |
| 30-42 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.16a.3.3	Specific message contents

Same specific message contents as in clause 9.2.1.1.16.3.3

##### 9.2.1.1.17	Attach / Rejected / No suitable cells in tracking area

9.2.1.1.17.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { the UE receives an ATTACH REJECT message with the EMM cause set to "No suitable cells in tracking area" }

then { the UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, UE deletes any GUTI, last visited registered TAI and KSI and the UE enters the state EMM-DEREGISTERED.LIMITED-SERVICE and the UE stores the current TAI in the list of "forbidden tracking areas for roaming" }

}

(2)

with { the UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE re-selects a cell that belongs to the TAI where UE was rejected }

then { the UE does not attempt to attach }

}

(3)

with { the UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for roaming" and KSI was deleted }

ensure that {

when { in the same PLMN, the UE enters a cell which provides normal service and belongs to a tracking area not in the list of "forbidden tracking areas for roaming" }

then { the UE attempts to attach with IMSI }

}

(4)

with { the UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for roaming"}

ensure that {

when { there are cells in the same PLMN and other PLMN that provide normal service and belong to tracking areas not in the list of "forbidden tracking areas for roaming" }

then { UE attempts to attach to the cell in the same PLMN }

}

NOTE: TP 4 is applicable only for Multi PLMN environment

(5)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the list of "forbidden tracking areas for roaming" contains more than one TAI}

ensure that {

when { UE re-selects a cell that belongs to one of the TAIs in the list of "forbidden tracking areas for roaming" }

then { UE does not attempt to attach }

}

(6)

with { UE is switched off }

ensure that {

when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for roaming" before the UE was switched off }

then { UE attempts to attach }

}

9.2.1.1.17.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.2, 5.5.1.2.2 and 5.5.1.2.5, and in TS 36.304 clause 5.2.4.4.

[TS 24.301, clause 5.3.2]

The UE shall store a list of "forbidden tracking areas for roaming", as well as a list of "forbidden tracking areas for regional provision of service". These lists shall be erased when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours).

...

In S1 mode, the UE shall update the suitable list whenever an ATTACH REJECT, TRACKING AREA UPDATE REJECT, SERVICE REJECT or DETACH REQUEST message is received with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area".

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

...

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

...

If the UE supports neither A/Gb mode nor Iu mode, the UE shall handle the Old GUTI or IMSI IE in the ATTACH REQUEST message as follows:

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI or IMSI IE as follows:

...

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI or IMSI IE.

...

-	Otherwise the UE shall include the IMSI in the Old GUTI or IMSI IE.

…

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

...

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

...

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

...

#15	(No suitable cells in tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. Additionally, the UE shall reset the attach attempt counter.

In S1 mode, the UE shall store the current TAI in the list of "forbidden tracking areas for roaming" and enter the state EMM-DEREGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area or in another location area in the same PLMN according to 3GPP TS 36.304 [21].

...

[TS 36.304 subclause 5.2.4.4]

...

If the highest ranked cell is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

...

9.2.1.1.17.3	Test description

9.2.1.1.17.3.1	Pre-test conditions

System Simulator:

-	cell I, , cell K, cell L and if (px_SinglePLMN_Tested = MultiPLMN) cell J are configured according to table 6.3.2.2-1 in TS 36.508 [18] (maximum 3 cells are simultaneously active):

-	cell I and cell K (visited PLMN, same TA);

- 	cell L (same visited PLMN, another TA);

-	If (px_SinglePLMN_Tested = Multi PLMN) cell J (another VPLMN).

NOTE:	Different types of UE may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12 am) for roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

UE:

- 	the UE is configured to initiate EPS attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell I using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.17.3.2	Test procedure sequence

Table 9.2.1.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell I as the "Serving cell", Cell K as a "Suitable Neighbour intrafrequency cell", Cell L as a "Non-Suitable Off cell", If present, Cell J as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message, EMM cause = “No suitable cells in tracking area”. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell I or Cell K? | --> | ATTACH REQUEST | 1 | F |
| 7 | The SS reconfigures: Cell I as a "Suitable neighbour intrafrequency cell", Cell K as the "Serving cell", Cell L as a "Non-Suitable Off cell", If present, Cell J as a "Non-Suitable Off cell ". | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell K or Cell I? | --> | ATTACH REQUEST | 2 | F |
| 8A | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 9 | The SS reconfigures: Cell I as a " Serving cell", Cell K is the " Non-Suitable Off cell ", Cell L as a "Non-Suitable Off cell", If present, Cell J as a "Non-Suitable Off cell". | - | - | - | - |
| 9A | The UE is brought back to operation or the USIM is inserted. |  |  |  |  |
| 9B | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 9BA-9BD | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 9C | The SS reconfigures: Cell I as a "Suitable Neighbour intrafrequency cell", Cell L as a " Serving cell", If present, Cell J as a "Suitable Neighbour intrafrequency cell”. | - | - |  |  |
| 9D | The SS transmits an ATTACH REJECT message, EMM cause = “No suitable cells in tracking area”. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 9E | The SS releases the RRC connection. | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified on Cell L? | --> | ATTACH REQUEST | 3, 4 | P |
| 10A-10D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 11 | The SS transmits an ATTACH REJECT message, EMM cause = “No suitable cells in tracking area”. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9 and TAI-11) | <-- | ATTACH REJECT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| 13 | The SS reconfigures: Cell I as the "Serving cell". Cell K as a “Non-Suitable Off cell”, Cell L as a " Suitable Neighbour intrafrequency cell", If present, Cell J as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 14 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 5 | F |
| 15 | If possible (see ICS) switch off is performed. Otherwise the power is removed. | - | - | - | - |
| 16 | The UE is brought back to operation. | - | - | - | - |
| 17 | Void | - | - | - | - |
| 18 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 6 | P |
| 19-31 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.17.3.3	Specific message contents

Table 9.2.1.1.17.3.3-1: Message ATTACH REJECT (step 4 and 11 Table 9.2.1.1.17.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | 0000 1111 | #15 “No suitable cells in tracking area” |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.17.3.3-2: Message ATTACH REQUEST (step 10 Table 9.2.1.1.17.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier | 111 | "No key is available" |  |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 4; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 4. |  |

##### 9.2.1.1.18	Attach / Rejected / Not authorized for this CSG

9.2.1.1.18.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to " Not authorized for this CSG " with integrity protection }

then { UE shall remove the CSG ID from the allowed CSG list and search for a suitable cell in the same PLMN }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and the CSG ID is removed from the Allowed CSG list }

ensure that {
  when { UE detects entering new tracking areas not included in the TAI list }

then { UE attempts to enter a normal cell and does not choose a cell which not included in the allowed CSG list }

}

9.2.1.1.18.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.5.

[TS 24.301, clause 5.5.1.2.5]

#25	(Not authorized for this CSG);

Cause #25 is only applicable when received from a CSG cell. Cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.6.

If the ATTACH REJECT message with cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). Additionally, the UE shall reset the attach attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall remove the CSG ID of the cell where the UE has sent the ATTACH REQUEST message from the Allowed CSG list.

The UE shall search for a suitable cell in the same PLMN according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

9.2.1.1.18.3	Test description

9.2.1.1.18.3.1	Pre-test conditions

System Simulator:

-	cell A (TAC 1, frequency 1, not a CSG cell);

-	cell B (TAC 2, frequency 1, is a CSG cell);

-	cell D (TAC 3, frequency 1, not a CSG cell).

-	System information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells A and D;

-	System information combination 7 as defined in TS 36.508[18] clause 4.4.3.1 is used in cell B;

UE:

-	the UE is configured to initiate EPS attach;

-	the UE is previously registered on cell B using either manual CSG selection or a USIM with field EFACSGL preconfigured (so the allowed CSG list includes CSG ID of cell B);

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered in E-UTRAN using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.18.3.2	Test procedure sequence

Table 9.2.1.1.18.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". - Cell D as a "Not Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | UE initiate attach procedure and send ATTACH REQUEST including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "Not authorized for this CSG". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". - Cell D as a "Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on Cell D unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message? Note: The UE shall search for a suitable cell in the same PLMN | --> | ATTACH REQUEST | 1 | P |
| 8-14A | Void | - | - | - | - |
| 14B-14Ob1 | Steps 5-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 15 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". - Cell D as a "Not Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 16 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message in the next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 2 | F |
| 17 | The SS configures: - Cell A as a "Serving cell". - Cell B as the "Not Suitable cell". - Cell D as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 18 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 19 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.1.1.18.3.3	Specific message contents

Table 9.2.1.1.18.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.1.18.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00011001 | #25 " Not authorized for this CSG " |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.18.3.3-2: SystemInformationBlockType1 for Cell A, B, D (Pre-test conditions and all steps in Table 9.2.1.1.18.3.2-1)

| Derivation Path: 36.508 clause 4.4.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell B |
|  | FALSE |  | Cell A |
|  | FALSE |  | Cell D |
| csg-Identity | Not present |  | Cell A |
|  | Not present |  | Cell D |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell B |

##### 9.2.1.1.19	Attach / Abnormal case / Failure due to non integrity protection

9.2.1.1.19.1	Test Purpose (TP)

(1)

with { UE having been initiated an Attach }

ensure that {

when { UE receives an ATTACH ACCEPT messages without NAS integrity protection before NAS security mode control procedure being performed }

then { UE discards this message }

}

(2)

with { a valid NAS security context exists and the NAS security mode control procedure has been successfully completed in the network and the UE }

ensure that {

when { UE receives a valid NAS signalling message without integrity protection }

then { UE discards this NAS signalling message }

}

(3)

with { a valid NAS security context exists and the NAS security mode control procedure has been successfully completed in the network and the UE }

ensure that {

when { UE receives a valid security protected NAS signalling message with the Message authentication code set to an incorrect value  }

then { UE discards this NAS signalling message }

}

(4)

with { a valid NAS security context exists and the NAS security mode control procedure has been successfully completed in the network and the UE }

ensure that {

when { UE receives a valid NAS signalling message with integrity protection which require a response from the UE }

then { UE sends the response as a security protected NAS message }

}

9.2.1.1.19.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 4.4.4.1, 4.4.4.2 and 5.5.2.2.

[TS 24.301, clause 4.4.4.1]

For the UE, integrity protected signalling is mandatory for the NAS messages once a valid EPS security context exists and has been taken into use. For the network, integrity protected signalling is mandatory for the NAS messages once a secure exchange of NAS messages has been established for the NAS signalling connection. Integrity protection of all NAS signalling messages is the responsibility of the NAS. It is the network which activates integrity protection.

…

[TS 24.301, clause 4.4.4.2]

Except the messages listed below, no NAS signalling messages shall be processed by the receiving EMM entity in the UE or forwarded to the ESM entity, unless the secure exchange of NAS messages has been established for the NAS signalling connection:

-	EMM messages:

-	IDENTITY REQUEST (if requested identification parameter is IMSI);

-	AUTHENTICATION REQUEST;

-	AUTHENTICATION REJECT;

-	ATTACH REJECT;

-	DETACH REQUEST;

-	DETACH ACCEPT (for non switch off);

-	TRACKING AREA UPDATE REJECT;

-	SERVICE REJECT.

NOTE:	These messages are accepted by the UE without integrity protection, as in certain situations they are sent by the network before security can be activated.

All ESM messages are integrity protected.

Once the secure exchange of NAS messages has been established, the receiving EMM or ESM entity in the UE shall not process any NAS signalling messages unless they have been successfully integrity checked by the NAS. If NAS signalling messages, having not successfully passed the integrity check, are received, then the NAS in the UE shall discard that message. If any NAS signalling message is received as not integrity protected even though the secure exchange of NAS messages has been established by the network, then the NAS shall discard this message.

[TS 24.301, clause 5.5.2.2.1]

...

If the UE is to be switched off, the UE shall:

-	delete the current EPS security context stored in the UE as specified in annex C, if it is a mapped EPS security context;

-	store the native EPS security context (if it is valid), as specified in annex C; and

-	try for a period of 5 seconds to send the DETACH REQUEST message. During this period, the UE may be switched off as soon as the DETACH REQUEST message has been sent.

...

9.2.1.1.19.3	Test description

9.2.1.1.19.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

NOTE:	Any type of attach is acceptable.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

- 	The NAS integrity algorithm shall be set to a different value than 'EPS integrity algorithm' EIA0 throughout the whole duration of the test.

9.2.1.1.19.3.2	Test procedure sequence

Table 9.2.1.1.19.3.2-1: Main behaviour

| St | Procedure |  | Message Sequence |  |  |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S |  | Message |  |  |  |
| 1 | The UE is switched on. |  | - |  | - |  | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. Note: The ATTACH REQUEST message shall be sent as a security protected NAS message (see TS 24.301 – clause 9.1). |  | --> |  | ATTACH REQUEST |  | - | - |
| 2Aa1-2Aa2 | Void |  | - |  | - |  | - | - |
| 2Ba1-2D | Steps 9a1-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  | - |  | - |  | - | - |
| 3 | The SS transmits an ATTACH ACCEPT although UE has not successfully completed any NAS security mode control procedure. Note: The ATTACH ACCEPT message is sent as a plain NAS message (see TS 24.301 – clause 9.1). |  | <-- |  | ATTACH ACCEPT |  | - | - |
| 4 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message without security protection. |  | --> |  | ATTACH COMPLETE |  | 1 | F |
| 5 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. |  | <-- |  | AUTHENTICATION REQUEST |  | - | - |
| 6 | The UE transmits an AUTHENTICATION RESPONSE message to establish mutual authentication. |  | --> |  | AUTHENTICATION RESPONSE |  | - | - |
| 7 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. |  | <-- |  | SECURITY MODE COMMAND |  | - | - |
| 8 | The UE transmits a SECURITY MODE COMPLETE message and establishes the initial security configuration. |  | --> |  | SECURITY MODE COMPLETE |  | - | - |
| 9 | The SS transmits an ATTACH ACCEPT without integrity protection. Note: The ATTACH ACCEPT message is sent as a plain NAS message (see TS 24.301 – clause 9.1). |  | <-- |  | ATTACH ACCEPT |  | - | - |
| 10 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message without security protection |  | --> |  | ATTACH COMPLETE |  | 2 | F |
| 11 | The SS transmits an ATTACH ACCEPT with integrity protection with the Message authentication code set to an incorrect value. Note: The ATTACH ACCEPT message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). |  | <-- |  | ATTACH ACCEPT |  | - | - |
| 12 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message because the integrity check is failed. |  | --> |  | ATTACH COMPLETE |  | 3 | F |
| 13 | The SS transmits an ATTACH ACCEPT message including an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. Note: The ATTACH ACCEPT message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). Note 1: SS allocates a PDN address of a PDN type which is compliant with the PDN type requested by the UE. |  | <-- |  | ATTACH ACCEPT |  | - | - |
| - | EXCEPTION: In parallel to the event described in step 14 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. |  | - |  | - |  | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 16 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE |  | - |  | - |  | - | - |
| 14 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified? Note: The ATTACH COMPLETE message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). |  | --> |  | ATTACH COMPLETE |  | 4 | P |
| 14A1a1-14A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - |  | - |  | - |  | - |
| 15 | Void |  | - |  | - |  | - | - |
| 16 | Check: Does the test results of test procedure in 36.508 clause 6.4.2.4 indicate that the UE is in E-UTRA EMM-REGISTERED state with S-TMSI-2? Note: This step verifies that the UE has correctly stored the GUTI-4 which was included in the protected ATTACH ACCEPT messages. |  | - |  | - |  | 4 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. |  | - |  | - |  | - | - |
| Note: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |  |  |  |

9.2.1.1.19.3.3	Specific message contents

Table 9.2.1.1.19.3.3-1: Message ATTACH ACCEPT (steps 3 and 9, Table 9.2.1.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 (Plain NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 | The SS chooses a value different from GUTI-1. |  |

Table 9.2.1.1.19.3.3-2: Message SECURITY PROTECTED NAS MESSAGE (step 11, Table 9.2.1.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.1-1 with condition CIPHERED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Message authentication code | Incorrect value | The SS chooses an incorrect value which fails integrity checks. (e.g. 00000000) |  |
| NAS message | ATTACH ACCEPT (see table 9.2.1.1.19.3.3-3) |  |  |

Table 9.2.1.1.19.3.3-3: Message ATTACH ACCEPT (step 11, Table 9.2.1.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 (Security protected NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-3 | The SS chooses a value different from GUTI-1 and GUTI-2. |  |

Table 9.2.1.1.19.3.3-4: Message ATTACH ACCEPT (step 13, Table 9.2.1.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 (Security protected NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-4 | The SS chooses a value different from GUTI-1, GUTI-2 and GUTI-3. |  |

##### 9.2.1.1.20	Attach / Abnormal case / Access barred because of access class barring or NAS signalling connection establishment rejected by the network

9.2.1.1.20.1	Test Purpose (TP)

(1)

with { UE switched-on, and not yet attached to EPS }

ensure that {

when { Access is barred for signalling in the cell UE is camping [Access Class barred in System information] }

then { the UE will not initiate any Attach procedure on the current cell }

}

(2)

with { UE switched-on, and not yet attached to EPS }

ensure that {

when { Access is barred for signalling in the cell UE is camping [T302 running due to RRCConnectionReject message reception] }

then { the UE will not initiate any Attach procedure on the current cell }

}

(3)

with { UE switched-on, and not yet attached to EPS }

ensure that {

when { Access is not barred for signalling in the cell UE is camping }

then { the UE will initiate Attach procedure on the current cell }

}

(4)

with { UE switched-on, and not yet attached to EPS }

ensure that {

when { Access was barred for signalling in the cell and UE has reselected an new cell where access for "signalling" is granted }

then { the UE will initiate Attach procedure on the new cell }

}

9.2.1.1.20.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.6 and TS 36.331, clause 5.3.3.2

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

a)	Access barred because of access class barring or NAS signalling connection establishment rejected by the network

If access is barred for "signalling" (see 3GPP TS 36.331 [22]), the attach procedure shall not be started. The UE stays in the current serving cell and applies the normal cell reselection process. The attach procedure is started as soon as possible, i.e. when access for "signalling" is granted on the current cell or when the UE moves to a cell where access for "signalling" is granted.

[TS 36.331, clause 5.3.3.2]

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInformation and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number ‘rand’ uniformly distributed in the range: 0 ≤ rand < 1;

4>	if ‘rand’ is lower than the value indicated by accessProbabilityFactor included in accessBarringForSignalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

9.2.1.1.20.3	Test description

9.2.1.1.20.3.1	Pre-test conditions

System Simulator:

-	cell I and cell K are configured according to table 6.3.2.2-1 in TS 36.508 [18].

-	cell I and cell K belong to TAI-9 (home PLMN)

UE:

- 	the UE is configured to initiate EPS attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell K using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.20.3.2	Test procedure sequence

Table 9.2.1.1.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sets the cell type of cell I to the ''Serving cell'', set the cell type of cell K to the '' Non- Suitable cell'', and sets SystemInformationBlockType2 parameters as described below. The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: For 90 seconds if UE initiates Attach procedure and hence transmits RRC Connection Request? | - | - | 1 | F |
| 3 | The SS transmits a Paging message including systemInfoModification. | - | - | - | - |
| 4 | The SS changes SystemInformationBlockType2 parameters to default parameters defined in [18]. | - | - | - | - |
| 5 | The UE transmits RRC Connection Request | - | - | - | - |
| 6 | SS responds with RRCConnectionReject message with IE waitTime set to 10 seconds (Max Value). | - | - | - | - |
| 7 | Check: For 10 seconds if UE initiates Attach procedure and hence transmits RRC Connection Request? | - | - | 2 | F |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 3 | P |
| 8A-8Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 9-15 | Void |  |  |  |  |
| - | EXCEPTION: Step 16a describes behaviour that depends on the UE capability. |  |  |  |  |
| 16a | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST |  |  |
| 17 | The SS sets the cell type of cell I to the ''Serving cell'', sets the cell type of cell K to the '' Non- Suitable cell'', and sets SystemInformationBlockType1 and SystemInformationBlockType2 parameters as described below. The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Check: for 90 seconds if UE initiates Attach procedure and hence transmits RRC Connection Request? | - | - | 1 | F |
| 19 | The SS sets the cell type of cell K to the ''serving cell'' and cell I to “suitable Cell”. |  |  |  |  |
| - | The following messages are to be observed on Cell K unless explicitly stated otherwise. | - | - | - | - |
| 20 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 21-34 b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.20.3	Specific message contents

Table 9.2.1.1.20.3.3-0: SystemInformationBlockType1 for Cell I (step 4, Table 9.2.1.1.20.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 9.2.1.1.20.3.3-0A: SystemInformationBlockType1-BR-r13 for Cell I (step 4 when UE under test is CAT-M1, Table 9.2.1.1.20.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.20.3.3-1: SystemInformationBlockType2 for Cell I (step 1 and 17)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| accessBarringInformation SEQUENCE { |  |  |  |
| accessBarringForEmergencyCalls | FALSE |  |  |
| accessBarringForSignalling SEQUENCE { |  |  |  |
| accessProbabilityFactor | p00 |  |  |
| accessBarringTime | s4 |  |  |
| accessClassBarringList SEQUENCE (SIZE (maxAC)) OF SEQUENCE { | 5 entries |  |  |
| accessClassBarring[1] | TRUE |  |  |
| accessClassBarring[2] | TRUE |  |  |
| accessClassBarring[3] | TRUE |  |  |
| accessClassBarring[4] | TRUE |  |  |
| accessClassBarring[5] | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| accessBarringForOriginatingCalls | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.20.3.3-2: SystemInformationBlockType1 for Cell I (step 17)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.20.3.3-3: SystemInformationBlockType1-BR-r13 for Cell I (step 17 when UE under test is CAT M1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.1.1.21	Void

NOTE:	The present test (Attach / Abnormal case / Success after several attempts due to no network response) is superseded by test case 9.2.1.1.22 (i.e. all requirements which the present test verified are verified in test case 9.2.1.1.22).

##### 9.2.1.1.22	Attach / Abnormal case / Unsuccessful attach after 5 attempts

9.2.1.1.22.1	Test Purpose (TP)

(0)

with { UE has sent an ATTACH REQUEST message and started T3410 timer}

ensure that {

when { T3410 timer expires }

then { the UE release NAS signalling connection locally }

}

(0A)

with { UE has sent an ATTACH REQUEST message and T3410 timer expired}

ensure that {

when { T3411 timer expires and attach attempt counter is less than 5 }

then { the UE restarts the attach procedure }

}

(0B)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { Lower Layer failure (RRC Connection is released) before the ATTACH ACCEPT or ATTACH REJECT message is received, T3411 has expired and attach attempt counter is less than 5}

then { the UE restarts the attach procedure }

}

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { T3411 has expired and attach attempt counter is equal to 5}

then { the UE stops attach attempts and starts timer T3402 }

}

9.2.1.1.22.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.6 & 10.2

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

…

b)	Lower layer failure or release of the NAS signalling connection before the ATTACH ACCEPT or ATTACH REJECT message is received

The attach procedure shall be aborted, and the UE shall proceed as described below.

c)	T3410 timeout

The UE shall abort the attach procedure and proceed as described below. The NAS signalling connection shall be released locally.

…

For the cases b, c, and d the UE shall proceed as follows:

-	Timer T3410 shall be stopped if still running. The attach attempt counter shall be incremented, unless it was already set to 5.

If the attach attempt counter is less than 5:

- timer T3411 is started and the state is changed to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH. When timer T3411 expires the attach procedure shall be restarted.

If the attach attempt counter is equal to 5:

-	the UE shall delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs and KSI, shall set the update status to EU2 NOT UPDATED, and shall start timer T3402. The state is changed to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH or optionally to EMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal attach procedure fails and the attach attempt counter is equal to 5.

[TS 24.301, clause 10.2]

Table 10.2.1: EPS mobility management timers – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3402 | Default 12 min. NOTE 1 | EMM-DEREGISTERED EMM-REGISTERED | At attach failure and the attempt counter is equal to 5. At tracking area updating failure and the attempt counter is equal to 5. | ATTACH REQUEST sent TRACKING AREA UPDATE REQUEST sent | Initiation of the attach procedure or TAU procedure |
| … |  |  |  |  |  |
| T3410 | 15s | EMM-REGISTERED-INITIATED | ATTACH REQUEST sent | ATTACH ACCEPT received ATTACH REJECT received | Start T3411 or T3402 as described in subclause 5.5.1.2.6 |
| T3411 | 10s | EMM-DEREGISTERED. ATTEMPTING-TO-ATTACH EMM-REGISTERED. ATTEMPTING-TO-UPDATE | At attach failure due to lower layer failure, T3410 timeout or attach rejected with other EMM cause values than those treated in subclause 5.5.1.2.5. At tracking area updating failure due to lower layer failure, T3430 timeout or TAU rejected with other EMM cause values than those treated in subclause 5.5.3.2.5. | ATTACH REQUEST sent TRACKING AREA UPDATE REQUEST sent | Retransmission of the ATTACH REQUEST or TRACKING AREA UPDATE REQUEST |
| … |  |  |  |  |  |
| Note 1: The default value of this timer is used if the network does not indicate another value in an EMM signalling procedure. |  |  |  |  |  |

9.2.1.1.22.3	Test description

9.2.1.1.22.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is configured to initiate EPS attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.22.3.2	Test procedure sequence

Table 9.2.1.1.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. (Attach attempt counter = 1) | --> | ATTACH REQUEST | - | - |
| 3 | The SS waits 25 seconds (15 seconds T3410 and 10 seconds T3411). | - | - | - | - |
| 3A | Check: Does the UE transmit an RRCConnectionRequest message? | - | - | 0 | P |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 2) | --> | ATTACH REQUEST | 0A | P |
| 5 | The SS releases the RRC connection. |  |  |  |  |
| 6 | The SS waits 10 seconds (T3411). |  |  |  |  |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 3) | --> | ATTACH REQUEST | 0B | P |
| 8 | The SS waits 25 seconds (15 seconds T3410 and 10 seconds T3411). | - |  | - | - |
| 9 | The UE transmits an ATTACH REQUEST message. (Attach attempt counter = 4) | --> | ATTACH REQUEST | - | - |
| 10 | The SS releases the RRC connection. |  |  |  |  |
| 11 | The SS waits 10 seconds (T3411). |  |  |  |  |
| 12 | The UE transmits an ATTACH REQUEST message. (Attach attempt counter = 5) | --> | ATTACH REQUEST |  |  |
| 13 | The SS releases the RRC connection. |  |  |  |  |
| 14 | Check: Does the UE transmit an ATTACH REQUEST message after 12 minutes (the default value of T3402), after step 13? | --> | ATTACH REQUEST | 1 | P |
| 15-28 b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.22.3.3	Specific message contents

Table 9.2.1.1.22.3.3-1: Message ATTACH REQUEST (step 14, Table 9.2.1.1.22.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier | 111 | "No key is available" |  |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted. |  |

##### 9.2.1.1.23	Attach / Abnormal case / Repeated rejects for network failures

9.2.1.1.23.1	Test Purpose (TP)

(1)

with { UE having valid GUTI, has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to #17 or #22 and attach attempt counter is less than 5}

then { UE starts timer T3411 and shall not delete stored GUTI }

when { Timer T3411 expires}

then { UE restarts attach procedure }

}

(2)

with { UE having valid GUTI, has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to #22 and attempt counter is set to 5}

then { the UE stops attach attempts and starts timer T3402, shall delete stored GUTI }

}

NOTE: Only representative coverage for various abnormal reject causes is provided.

9.2.1.1.23.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.5, 5.5.1.2.6, 9.9.3.9 & 10.2

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value. If the attach procedure fails due to a default EPS bearer setup failure, an ESM procedure failure, or operator determined barring is applied on default EPS bearer context activation during attach procedure, the MME shall combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message contained in the ESM message container information element. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19 "ESM failure".

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410 and take the following actions depending on the EMM cause value received.

...

Other values are considered as abnormal cases. The behaviour of the UE in those cases is specified in subclause 5.5.1.2.6.

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

…

d)	ATTACH REJECT, other EMM cause values than those treated in subclause 5.5.1.2.5

Upon reception of the EMM cause #19, "ESM failure", the UE may set the attach attempt counter to 5. Upon reception of the EMM causes #95, #96, #97, #99 and #111 the UE should set the attach attempt counter to 5.

The UE shall proceed as described below.

…

For the cases b, c, and d the UE shall proceed as follows:

-	Timer T3410 shall be stopped if still running. The attach attempt counter shall be incremented, unless it was already set to 5.

If the attach attempt counter is less than 5:

- timer T3411 is started and the state is changed to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH. When timer T3411 expires the attach procedure shall be restarted.

If the attach attempt counter is equal to 5:

-	the UE shall delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs and KSI, shall set the update status to EU2 NOT UPDATED, and shall start timer T3402. The state is changed to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH or optionally to EMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal attach procedure fails and the attach attempt counter is equal to 5.

[TS 24.301, clause 9.9.3.9]

Table 9.9.3.9.1: EMM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  |  | ... |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | Network failure |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  | CS domain not available |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |  | ESM failure |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | MAC failure |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | Synch failure |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  | Congestion |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |  | UE security capabilities mismatch |
|  |  |  |  |  |  |  |  |  | ... |
| Any other value received by the mobile station shall be treated as 0110 1111, "protocol error, unspecified". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

[TS 24.301, clause 10.2]

Table 10.2.1: EPS mobility management timers – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3402 | Default 12 min. NOTE 1 | EMM-DEREGISTERED EMM-REGISTERED | At attach failure and the attempt counter is equal to 5. At tracking area updating failure and the attempt counter is equal to 5. | ATTACH REQUEST sent TRACKING AREA UPDATE REQUEST sent | Initiation of the attach procedure or TAU procedure |
| … |  |  |  |  |  |
| T3410 | 15s | EMM-REGISTERED-INITIATED | ATTACH REQUEST sent | ATTACH ACCEPT received ATTACH REJECT received | Start T3411 or T3402 as described in subclause 5.5.1.2.6 |
| T3411 | 10s | EMM-DEREGISTERED. ATTEMPTING-TO-ATTACH EMM-REGISTERED. ATTEMPTING-TO-UPDATE | At attach failure due to lower layer failure, T3410 timeout or attach rejected with other EMM cause values than those treated in subclause 5.5.1.2.5. At tracking area updating failure due to lower layer failure, T3430 timeout or TAU rejected with other EMM cause values than those treated in subclause 5.5.3.2.5. | ATTACH REQUEST sent TRACKING AREA UPDATE REQUEST sent | Retransmission of the ATTACH REQUEST or TRACKING AREA UPDATE REQUEST |
| … |  |  |  |  |  |
| Note 1: The default value of this timer is used if the network does not indicate another value in an EMM signalling procedure. |  |  |  |  |  |

9.2.1.1.23.3	Test description

9.2.1.1.23.3.1	Pre-test conditions

System Simulator:

-	cell A.

NOTE:	T3402 is set to default (12 min.).

UE:

-	the UE is configured to initiate EPS attach

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.23.3.2	Test procedure sequence

Table 9.2.1.1.23.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. (Attach attempt counter = 1) | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an ATTACH REJECT message, EMM cause = Network failure (#17) | <-- | ATTACH REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 2) | --> | ATTACH REQUEST | 1 | P |
| 7 | The SS transmits an ATTACH REJECT message, EMM cause = Network failure (#17) | <-- | ATTACH REJECT | - | - |
| 8 | The SS releases the RRC connection. | - | - | - | - |
| 9 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 3) | --> | ATTACH REQUEST | 1 | P |
| 11 | The SS transmits an ATTACH REJECT message, EMM cause = Congestion (#22) | <-- | ATTACH REJECT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| 13 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 14 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 4) | --> | ATTACH REQUEST | - | - |
| 15 | The SS transmits an ATTACH REJECT message, EMM cause = Congestion (#22) | <-- | ATTACH REJECT | - | - |
| 16 | The SS releases the RRC connection. | - | - | - | - |
| 17 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 18 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 5) | --> | ATTACH REQUEST | 1 | P |
| 19 | The SS transmits an ATTACH REJECT message, EMM cause = Congestion (#22) | <-- | ATTACH REJECT | - | - |
| 20 | The SS releases the RRC connection. | - | - | - | - |
| 21 | The SS waits 12 minutes (default value of T3402). | - | - | - | - |
| 22 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 23-36 b1 | The attach procedure is completed by executing steps 5 to 18b1 ofthe UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.23.3.3	Specific message contents

Table 9.2.1.1.23.3.3-1: Message ATTACH REQUEST (step 2, 6, 10 &14 Table 9.2.1.1.23.2-1)

| Derivation path: TS 36.508 table 4.7.2.-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier | Any allowed value other than '111'B | "Stored key is available" |  |
| Old GUTI or IMSI | GUTI-1 | As stored in USIM |  |
| Last visited registered TAI | TAI-1 | Stored TAI. |  |

Table 9.2.1.1.23.3.3-2: Message ATTACH REJECT (steps 3 & 7 Table 9.2.1.1.23.3.2-1)

| Derivation path: 36.508 table 4.7.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Security header type | 0000 | " Plain NAS message, not security protected " |  |
| EMM cause | 00010001 | #17 " Network failure" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.23.3.3-3: Message ATTACH REJECT (steps 11, 15 & 19 Table 9.2.1.1.23.3.2-1)

| Derivation path: 36.508 table 4.7.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Security header type | 0000 | " Plain NAS message, not security protected " |  |
| EMM cause | 00010110 | #22 "Congestion " |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.23.3.3-4: Message ATTACH REQUEST (step 22, Table 9.2.1.1.23.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier | 111 | "No key is available" |  |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted. |  |

##### 9.2.1.1.24	Attach / Abnormal case / Change of cell into a new tracking area

9.2.1.1.24.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH Request message }

ensure that {

when { cell change into a new tracking area occurs before the ATTACH procedure is completed }

then { the UE aborts the ATTACH procedure and re-initiates it immediately in the new tracking area }

}

(2)

with { UE has sent an ATTACH REQUEST message and received ATTACH ACCEPT message containing GUTI }

ensure that {

when { UE reselects a cell belonging to a new tracking area }

then { the UE restarts the attach procedure }

}

9.2.1.1.24.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.6 and 5.5.1.3.6.

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

…

e)	Change of cell into a new tracking area

If a cell change into a new tracking area occurs before the attach procedure is completed, the attach procedure shall be aborted and re-initiated immediately. If a tracking area border is crossed when the ATTACH ACCEPT message has been received but before an ATTACH COMPLETE message is sent, the attach procedure shall be re-initiated. If a GUTI was allocated during the attach procedure, this GUTI shall be used in the attach procedure.

[TS 24.301, clause 5.5.1.3.6]

The UE shall proceed as follows:

...

-	otherwise, the abnormal cases specified in subclause 5.5.1.2.6 apply with the following modification.

If the attach attempt counter is incremented according to subclause 5.5.1.2.6 the next actions depend on the value of the attach attempt counter:

9.2.1.1.24.3	Test description

9.2.1.1.24.3.1	Pre-test conditions

System Simulator:

-	Cells A and B:

-	Cell A is the serving cell with TAI 1 (PLMN1+TAC1);

-	Cell B is the non-suitable cell with TAI 2 (PLMN1+TAC2).

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

NOTE:	Any type of attach is acceptable.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.24.3.2	Test procedure sequence

Table 9.2.1.1.24.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS configures: - Cell B as a "serving cell" - Cell A as a "non-suitable cell". | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message in Cell B including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3 | SS does not send ATTACH ACCEPT to the UE and update TAC value in SystemInformationBlockType1. | - |  | - | - |
| - | EXCEPTION: IF pc_ue_CategoryDL_M1 THEN execute 3Ab1 to 3Ab3 ELSE execute step 3Aa1. | - | - | - | - |
| 3A a1 | The SS transmits a Paging message paging occasion including a systemInfoModification. From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1 as specified. | <-- | Paging | - | - |
| 3Ab1 | The SS transmits an RRCConnectionRelease message in Cell B. | - | - | - | - |
| 3Ab2 | Void | - | - | - | - |
| 3Ab3 | The SS indicates a new system information by Direct Indication information with bit 1 and transmitted on MPDCCH using P-RNTI. From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1-BR-r13. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message in Cell B including a PDN CONNECTIVITY REQUEST message in the next 12 seconds? Note: Wait time is more than 2.1* modification period for the UE to receive system information and inferior to T3410. | --> | ATTACH REQUEST | 1 | P |
| 4A-8b | Void | - | - | - | - |
| 8A-8G | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS configures: - Cell A as a “serving cell” | - | - | - | - |
| 10 | SS is configured to not allocate any UL grant or respond to any PRACH preambles for ATTACH COMPLETE in Cell B. | - | - | - | - |
| 11 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. GUTI-1 is allocated. | <-- | ATTACH ACCEPT | - | - |
| 12 | The SS configures: - Cell B as a "non-suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 13 | UE transmits RRCConnectionReestablishmentRequest message in Cell A | - | - | - | - |
| 14 | SS Sends RRCConnectionReestablishment | - | - | - | - |
| 15 | The UE transmits a RRCConnectionReestablishmentComplete message | - | - | - | - |
| 16 | Check: Does the UE transmit an ATTACH REQUEST message, containing GUTI-1? | --> | ATTACH REQUEST | 2 | P |
| 17 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 18 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (NOTE 2) THEN in parallel to the event described in step 18 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 18 | The UE transmits an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | ATTACH COMPLETE | - | - |
| 18A1a1-18A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 19 | Void | - | - | - | - |
| NOTE 1: UE already has NAS security context in step 17 and just sending a DL message with Integrity and Ciphering will start enable NAS security. NOTE 2: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.1.1.24.3.3	Specific message contents

Table 9.2.1.1.24.3.3-1: Message ATTACH ACCEPT (step 11, Table 9.2.1.1.24.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| TAI list |  |  |  |  |  |
| Length of tracking area identity list contents | '0000 0110'B |  |  |  |  |
| Partial tracking area identity list 1 |  |  |  |  |  |
| Number of elements | '0 0000'B |  |  |  |  |
| Type of list | '00'B |  |  |  |  |
| MCC | MCC of cell B |  |  |  |  |
| MNC | MNC of cell B |  |  |  |  |
| TAC 1 | TAC-4 |  |  |  |  |
| GUTI |  | GUTI-1 |  |  |  |

Table 9.2.1.1.24.3.3-2: SystemInformationBlockType1 for Cell B (From step 3 in Table 9.2.1.1.24.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| trackingAreaCode | TAC-4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.24.3.3-2A: SystemInformationBlockType1-BR-r13 for Cell B (From step 3 in Table 9.2.1.1.24.3.2-1 when UE under test is CAT M1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| trackingAreaCode | TAC-4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.1.24.3.3-3: Paging (step 3Aa1, Table 9.2.1.1.24.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

Table 9.2.1.1.24.3.3-4: Message ATTACH REQUEST (step 16, Table 9.2.1.1.24.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI of cell B with TAC-4 |  |  |

##### 9.2.1.1.25	Attach / Abnormal case / Mobile originated detach required

9.2.1.1.25.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED-INITIATED state }

ensure that {

when { the UE initiates mobile originated detach }

then { the UE aborts the attach procedure }

}

9.2.1.1.25.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.6 and 5.5.1.3.6

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

f)	Mobile originated detach required

The attach procedure shall be aborted, and the UE initiated detach procedure shall be performed.

[TS 24.301, clause 5.5.1.3.6]

The UE shall proceed as follows:

...

-	otherwise, the abnormal cases specified in subclause 5.5.1.2.6 apply with the following modification.

If the attach attempt counter is incremented according to subclause 5.5.1.2.6 the next actions depend on the value of the attach attempt counter:

9.2.1.1.25.3	Test description

9.2.1.1.25.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.25.3.2	Test procedure sequence

Table 9.2.1.1.25.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. (NOTE 2) | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE | - | - |
| 7 | The SS does not respond to ATTACH REQUEST message. | - | - | - | - |
| 7A | Cause UE to initiate detach before the T3410 times out. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE initiate mobile originated detach and abort the attach procedure while T3410 is running? | --> | DETACH REQUEST | 1 | P |
| 9 | The SS transmits DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 9A | The SS releases the RRC connection. | - | - | - | - |
| 10 | Check: Does the test result of the generic procedure defined in clause 6.4.2.5 of 36.508 [18] indicates that the UE responds to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
| 11 | Void | - | - | - | - |
| NOTE 1: T3410 value is specified as 15s in TS 24.301. NOTE 2: Any type of attach is acceptable. |  |  |  |  |  |

9.2.1.1.25.3.3	Specific message contents

Table 9.2.1.1.25.3.3-1: Message DETACH REQUEST (step 8, Table 9.2.1.1.25.3.2-1)

| Derivation path: 36.508 table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | Normal detach |  |

##### 9.2.1.1.26	Attach / Abnormal case / Detach procedure collision

9.2.1.1.26.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a DETACH REQUEST message and detach type indicates “re-attach not required” }

then { the UE sends DETACH ACCEPT }

}

(2)

with { UE in EMM-REGISTERED-INITIATED  state }

ensure that {

when { UE receives a DETACH REQUEST message and detach type  indicates “re-attach  required” }

then { the UE continues with ATTACH procedure or the UE sends DETACH ACCEPT and re-initiates the ATTACH procedure}

}

9.2.1.1.26.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 [28], clause 5.5.1.2.6, 5.5.1.3.6 and 5.5.3.2.

[TS 24.301 [28], clause 5.5.1.2.6]

[Rel-14]

g)	Detach procedure collision

If the UE receives a DETACH REQUEST message from the network in state EMM-REGISTERED-INITIATED and the detach type indicates "re-attach not required" and no EMM cause IE, or "re-attach not required" and the EMM cause value is not #2 "IMSI unknown in HSS", the detach procedure shall be progressed and the attach procedure shall be aborted. Otherwise the attach procedure shall be progressed and the DETACH REQUEST message shall be ignored.

[Rel-15]

The following abnormal cases can be identified:

…

g)	Detach procedure collision

If the UE receives a DETACH REQUEST message from the network in state EMM-REGISTERED-INITIATED and the detach type indicates "re-attach not required", and no EMM cause IE, or "re-attach not required" and the EMM cause value is not #2 "IMSI unknown in HSS", the detach procedure shall be progressed and the attach procedure shall be aborted. If the UE receives a DETACH REQUEST message from the network in state EMM-REGISTERED-INITIATED and the detach type indicates "re-attach required", the detach procedure shall be progressed and the UE shall locally release the NAS signalling connection, before re-initiating the attach procedure. Otherwise the attach procedure shall be progressed and the DETACH REQUEST message shall be ignored.

[TS 24.301 [28], clause 5.5.1.3.6]

[Rel-14]

The UE shall proceed as follows:

...

3)	otherwise, the abnormal cases specified in subclause 5.5.1.2.6 apply with the following modification.

If the attach attempt counter is incremented according to subclause 5.5.1.2.6 the next actions depend on the value of the attach attempt counter:

-	if the attach attempt counter is less than 5, the UE shall set the update status to U2 NOT UPDATED but shall not delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs;

[TS 24.301 [28], clause 5.5.3.2][Rel-15]

When receiving the DETACH REQUEST message and the detach type indicates "re-attach required", the UE shall deactivate the EPS bearer context(s), if any, including the default EPS bearer context locally without peer-to-peer signalling between the UE and the MME. The UE shall stop the timer T3346, if it is running. The UE shall also stop timer(s) T3396, if it is running. The UE shall send a DETACH ACCEPT message to the network and enter the state EMM-DEREGISTERED. Furthermore, the UE shall, after the completion of the detach procedure, and the release of the existing NAS signalling connection, initiate an attach or combined attach procedure. The UE should also re-establish any previously established PDN connection(s).

9.2.1.1.26.3	Test description

9.2.1.1.26.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.26.3.2	Test procedure sequence

Table 9.2.1.1.26.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. (NOTE 2) | --> | ATTACH REQUEST | - | - |
| 3 | The SS initiates Detach procedure with the Detach Type IE "re-attach not required" | <-- | DETACH REQUEST | - | - |
| 4 | Check: Does the UE send DETACH ACCEPT message? | --> | DETACH ACCEPT | 1 | P |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 5A | The SS waits 5 seconds (NOTE 1) | - | - | - | - |
| 6 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 6A | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 7 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. (NOTE 2) | --> | ATTACH REQUEST | - | - |
| 8-12a2 | Void | - | - | - | - |
| 12A-12G | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 13 | The SS initiates Detach procedure with the Detach Type IE "re-attach required". Note: A pre Release 15 UE ignores the DETACH REQUEST. | <-- | DETACH REQUEST | - | - |
| 13A | Start a 5 second timer. |  |  |  |  |
| - | EXCEPTION: Steps 13Ba1 to 13Bb1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if UE have particular implementation. | - | - | - | - |
| 13Ba1 | Check: Does the UE send DETACH ACCEPT message. | --> | DETACH ACCEPT | 2 | P |
| 13Ba2 | Cancel the timer started in step 13A. |  |  |  |  |
| 13Ba3 | The UE locally releases the NAS signalling connection. The SS locally releases the RRC connection. |  |  |  |  |
| 13Ba4 | Check: Does the UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. (NOTE 2) | --> | ATTACH REQUEST | 2 | P |
| 13Ba4-13Ba10 | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 13Bb1 | The timer started in step 13A expires. |  |  |  |  |
| 14 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. GUTI-1 is allocated. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in steps 15 to 16 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (NOTE 3) THEN in parallel to the event described in steps 15 to 16 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 15 | The UE transmits an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 2 | P |
| 15Aa1-15Ac1 | Steps 16a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  |  |  |  |
| 16 | Void | - | - | - | - |
| 16A1a1-16A3b1 | Void | - | - | - | - |
| 16B-16Cb1 | Steps 17-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 17 | Void | - | - | - | - |
| NOTE 1: The time delay is added to additionally guarantee UE has respected the content of the DETACH REQUEST message. NOTE 2: Any type of attach is acceptable. NOTE 3: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

Table 9.2.1.1.26.3.2-2: Void

9.2.1.1.26.3.3	Specific message contents

Table 9.2.1.1.26.3.3-1: Message DETACH REQUEST (step 3, Table 9.2.1.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | '010'B | "re-attach not required" |  |
| EMM cause | ‘07’H | EPS services not allowed |  |

Table 9.2.1.1.26.3.3-2: Message DETACH REQUEST (step 13, Table 9.2.1.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | ‘001'B | "re-attach required" |  |
| EMM cause | NOT PRESENT |  |  |

##### 9.2.1.1.27	Attach / Abnormal case / Network reject with Extended Wait Timer

9.2.1.1.27.1	Test Purpose (TP)

(1)

with { UE configured for low access priority, has sent an ATTACH REQUEST message with NAS low priority signalling indication }

ensure that {

when { UE receives “Extended Wait Time” in the RRCConnectionRelease message }

then { UE starts timer T3346 and abort the attach procedure }

}

(2)

with { UE having aborted attach procedure and with timer T3346 running }

ensure that {

when { Timer T3346 expires }

then { UE restarts attach procedure }

}

9.2.1.1.27.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.6

[TS 24.301, clause 5.5.1.2.6]

The following abnormal cases can be identified:

…

l)	"Extended wait time" from the lower layers

If the ATTACH REQUEST message contained the NAS signalling low priority indicator set to "MS is configured for NAS signalling low priority", the UE shall start timer T3346 with the "Extended wait time" value.

In other cases the UE shall ignore the "Extended wait time".

The UE shall abort the attach procedure, reset the attach attempt counter, stay in the current serving cell, change the state to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH and apply the normal cell reselection process.

The UE shall proceed as described below.

m)	Timer T3346 is running

The UE shall not start the attach procedure unless the UE is accessing the network with access class 11 – 15 or needs to attach for emergency bearer services. The UE stays in the current serving cell and applies the normal cell reselection process.

NOTE:	It is considered an abnormal case if the UE needs to initiate an attach procedure while timer T3346 is running independent on whether timer T3346 was started due to an abnormal case or a non successful case.

The UE shall proceed as described below.

For the cases b, c, and d the UE shall proceed as follows:

-	Timer T3410 shall be stopped if still running. The attach attempt counter shall be incremented, unless it was already set to 5.

If the attach attempt counter is less than 5:

-	for the cases l and m, the attach procedure is started, if still necessary, when timer T3346 expires or is stopped;

-	for all other cases, timer T3411 is started and the state is changed to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH. When timer T3411 expires the attach procedure shall be restarted, if still required by ESM sublayer.

9.2.1.1.27.3	Test description

9.2.1.1.27.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is configured to initiate EPS attach

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	the UE is configured for NAS signalling low priority

The UE is equipped with a USIM containing values shown in Table 9.2.1.1.27.3.1-1.

Table 9.2.1.1.27.3.1-1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 is supported. |
| EFNASCONFIG | “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. |
| EFNASCONFIG | “ExtendedAccessBarring is set to extended access barring is applied for the UE” as defined in TS 24.368, clause 5.8 |
| Note: As per TS 23.401 [22] clause 4.3.17.4, UE’s configuration of low access priority and Extended Access Barring shall match each other. |  |

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.27.3.2	Test procedure sequence

Table 9.2.1.1.27.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. (Device properties IE included) | --> | ATTACH REQUEST | - | - |
| 3 | The SS releases the RRC connection. | - | The SS includes the IE “Extended Wait Time” in the RRCConnectionRelease message. UE starts timer T3346 with the value 5 seconds | - | - |
| 4 | The SS waits 5 seconds (T3346). | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 1,2 | P |
| 619b1 | The attach procedure is completed by executing steps 5 to 18b1of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.1.27.3.3	Specific message contents

Table 9.2.1.1.27.3.3-1: Message ATTACH REQUEST (step 2 and 5 Table 9.2.1.1.27.2-1)

| Derivation path: TS 36.508 table 4.7.2.-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier | Any allowed value other than '111'B | "Stored key is available" |  |
| Old GUTI or IMSI | GUTI-1 | As stored in USIM |  |
| Last visited registered TAI | TAI-1 | Stored TAI. |  |
| Device properties | 1 | “MS is configured for NAS signalling low priority” |  |

Table 9.2.1.1.27.3.3-2: RRCConnectionRelease (step 3 Table 9.2.1.1.27.2-1)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-v890-IEs |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-v920-IEs |  |
| cellInfoList-r9 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-v1020-IEs |  |
| extendedWaitTime-r10 | 5 seconds |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.1.1.27a	Attach Procedure / EAB broadcast handling / ExtendedAccessBarring configured in the UE

9.2.1.1.27a.1	Test Purpose (TP)

(1)

with { UE being configured for ExtendedAccessBarring (EAB) and not configured for Override_ExtendedAccessBarring and accessing the network with an access class different to classes 11 - 15 }

ensure that {

when { the UE finds a suitable cell to attach for services, and, the cell is broadcasting EAB information effectively barring the cell for the UE }

then { the UE will not initiate Attach procedure on the cell }

}

9.2.1.1.27a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.3.10 and D.1, TS 24.368 clauses 4 and 5.8, and, TS 36.331, clauses 5.2.2.4, 5.3.3.2 and 5.3.3.12. Unless otherwise stated these are Rel-11 requirements.

[TS 24.301, clause 5.3.10]

The network can restrict the access for certain groups of UEs by means of barring their access class.

The UE shall evaluate the access control information for Access Class Barring, Access Control for CSFB and Extended Access Barring (EAB) as specified in 3GPP TS 36.331 [22].

[TS 24.301, clause D.1]

When EMM requests the establishment of a NAS-signalling connection, the RRC establishment cause used by the UE shall be selected according to the NAS procedure as specified in table D.1.1. The EMM shall also indicate to the lower layer for the purpose of access control, the call type associated with the RRC establishment cause as specified in table D.1.1. If the UE is configured for EAB (see the "ExtendedAccessBarring" leaf of NAS configuration MO in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]), the EMM shall indicate to the lower layer for the purpose of access control that EAB applies for this request except for the following cases:

-	the UE is accessing the network with one of the access classes 11 – 15;

-	the UE is answering to paging;

-	the RRC Establishment cause is set to "Emergency call"; or

-	the UE is configured to allow overriding EAB (see the "Override_ExtendedAccessBarring" leaf of the NAS configuration MO as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) and receives an indication from the upper layers to override EAB.

[TS 24.368, clause 4]

The NAS configuration MO is used to manage configuration parameters related to NAS functionality for a UE supporting provisioning of such information. The presence and format of the non-access stratum configuration file on the USIM is specified in 3GPP TS 31.102 [6].

The MO identifier is: urn:oma:mo:ext-3gpp-nas-config:1.0.

The OMA DM Access Control List (ACL) property mechanism (see OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the NAS configuration MO.

The following nodes and leaf objects are possible in the NAS configuration MO as described in figure 4-1:

Figure 4-1: The NAS configuration Management Object

[TS 24.368, clause 5.8]

The ExtendedAccessBarring leaf indicates whether the extended access barring is applicable for the UE as specified in 3GPP TS 24.008 [4] and 3GPP TS 24.301 [5].

-	Occurrence: ZeroOrOne

-	Format: bool

-	Access Types: Get, Replace

-	Values: 0, 1

0	Indicates that the extended access barring is not applied for the UE.

1	Indicates that the extended access barring is applied for the UE.

The default value 0 applies if this leaf is not provisioned.

[TS 36.331, clause 5.2.2.4]

The UE shall:

...

1>	not initiate the RRC connection establishment subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

...

1>	if the UE is EAB capable:

2>	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification or upon entering a cell during RRC_IDLE:

3>	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4>	start acquiring SystemInformationBlockType14 immediately;

3>	else:

4>	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that EAB is applicable, upon which the procedure ends;

[TS 36.331, clause 5.3.3.12]

The UE shall:

1>	if SystemInformationBlockType14 is present and includes the eab-Param:

2>	if the eab-Common is included in the eab-Param:

3>	if the UE belongs to the category of UEs as indicated in the eab-Category contained in eab-Common; and

3>	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the eab-BarringBitmap contained in eab-Common is set to one:

4>	consider access to the cell as barred;

3>	else:

4>	consider access to the cell as not barred due to EAB;

9.2.1.1.27a.3	Test description

9.2.1.1.27a.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is configured to respect "ExtendedAccessBarring" as per TS 24.368 [49]. This is achieved by equipping the UE with a USIM containing values shown in Table 9.2.1.1.27a.3.1-1.

Table 9.2.1.1.27a.3.1-1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 'Non-Access Stratum configuration by USIM' is supported. |
| EFNASCONFIG | - Extended access barring activated: UE shall apply extended access barring - Override Extended access barring not activated |
| EFNASCONFIG | - “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. - Override_NAS_SignallingLowPriority not activated |

-	The UE is configured to initiate EPS attach.

Preamble:

-	The UE is first registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18]. During the registration, the SS is not transmitting SystemInformationBlockType14.

-	The UE is Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.27a.3.2	Test procedure sequence

Table 9.2.1.1.27a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts transmitting SystemInformationBlockType14 Extended access barring activated on Cell A. System information combination 21 as defined in TS 36.508[18] clause 4.4.3.1.2 is used. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE send an ATTACH REQUEST message in the next 60 sec? | - | ATTACH REQUEST | 1 | F |
| 4 | The SS stops transmitting SystemInformationBlockType14 on Cell A. | - | - | - | - |
| 5 | Check: Does the result of generic procedure UE Registration (State 2) as described in TS 36.508 [18] clause 4.5.2.3 indicate that UE is registered on Cell A? | - | - | 1 | P |
| - | At the end of the present test procedure sequence, the UE is in end state E-UTRA RRC_IDLE (State 2) according to TS 36.508 [18]. | - | - | - | - |

9.2.1.1.27a.3.3	Specific message contents

Table 9.2.1.1.27a.3.3-1: Message ATTACH REQUEST (preamble and all steps)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Device properties | 1 | “MS is configured for NAS signalling low priority” |  |

##### 9.2.1.1.27b	Attach / EAB / CE-level based access barring

9.2.1.1.27b.1	Test Purpose (TP)

(1)

with { UE configured for CE-level based ExtendedAccessBarring(EAB) }

ensure that {

when { the UE finds a suitable cell to attach for services, and the RSRP level is below the CE level access barring RSRP threshold }

then { the UE does not initiate Attach procedure on the cell }

}

(2)

with { UE configured for CE-level based ExtendedAccessBarring(EAB) }

ensure that {

when { the UE finds a suitable cell to attach for services, and the RSRP level is above the CE level access barring RSRP threshold }

then { the UE initiates Attach procedure on the cell }

}

9.2.1.1.27b.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clause 5.3.10, and TS 36.331, clause 5.3.3.12, 5.2.1.6, 5.3.2.3 and 5.3.3.2.

[TS 24.301, clause 5.3.10]

The network can restrict the access for certain groups of UEs by means of barring their access class.

The UE shall evaluate the access control information as specified in 3GPP TS 36.331 [22] for:

-	Access Class Barring;

-	Access Control for CSFB and Extended Access Barring (EAB);

-	Access Control for Application specific Congestion control for Data Communication (ACDC), if the UE supports ACDC; and

-	Access Barring.

[TS 36.331, clause 5.3.3.12]

The UE shall:

1>	if SystemInformationBlockType14 is present:

2>	if eab-PerRSRP is included:

3>	if the establishmentCause received from higher layers is set to a value other than emergency; and

3>	if the UE has no Access Class, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11] :

4>	if eab-PerRSRP is set to thresh0:

5>	consider access to the cell as barred when in enhanced coverage as specified in TS 36.304 [4];

4>	else if eab-PerRSRP is set to thresh1:

5>	if the measured RSRP is less than the first entry in rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first CE level are configured;

4>	else if eab-PerRSRP is set to thresh2:

5>	if the measured RSRP is less than the second entry in rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first and second CE levels are configured;

4>	else if eab-PerRSRP is set to thresh3:

5>	if the measured RSRP is less than the third entry rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first, second, and third CE levels are configured;

[TS 36.331, clause 5.2.1.6]

Change of EAB parameters can occur at any point in time. The EAB parameters are contained in SystemInformationBlockType14. The Paging message is used to inform EAB capable UEs in RRC_IDLE about a change of EAB parameters or that SystemInformationBlockType14 is no longer scheduled. If the UE receives a Paging message including the eab-ParamModification, it shall acquire SystemInformationBlockType14 according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives a Paging message including the eab-ParamModification while it is acquiring SystemInformationBlockType14, the UE shall continue acquiring SystemInformationBlockType14 based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

…

1>	if in RRC_IDLE, the eab-ParamModification is included and the UE is EAB capable:

2>	consider previously stored SystemInformationBlockType14 as invalid;

2>	re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.6;

2>	re-acquire SystemInformationBlockType14 using the system information acquisition procedure as specified in 5.2.2.4;

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE or when upper layers request resume of an RRC connection or RRC layer requests resume of an RRC connection for, e.g. RNAU or reception of RAN paging while the UE is in RRC_INACTIVE.

Except for NB-IoT, upon initiation of the procedure, if the UE is connected to EPC, the UE shall:

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that EAB is applicable, upon which the procedure ends;

9.2.1.1.27b.3	Test description

9.2.1.1.27b.3.1	Pre-test conditions

System Simulator:

-	Cell A supports CE.

UE:

-	The UE is configured to respect "ExtendedAccessBarring" as per TS 24.368 [49]. This is achieved by equipping the UE with a USIM containing values shown in Table 9.2.1.1.27a.3.1-1.

Table 9.2.1.1.27b.3.1-1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 'Non-Access Stratum configuration by USIM' is supported. |
| EFNASCONFIG | - Extended access barring activated: UE shall apply extended access barring - Override Extended access barring not activated |
| EFNASCONFIG | - “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. - Override_NAS_SignallingLowPriority not activated |

-	The UE is configured to initiate EPS attach.

Preamble:

-	The UE is first registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18]. During the registration, the SS is not transmitting SystemInformationBlockType14.

-	The UE is Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.27b.3.2	Test procedure sequence

Table 9.2.1.1.27b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 9.2.1.1.27b.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell A | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -102 | The power level value is such that the measured RSRP is higher than the third entry in rsrp-ThresholdsPrachInfoList but lower than the first and second entries in rsrp-ThresholdsPrachInfoList |

Table 9.2.1.1.27b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts transmitting SystemInformationBlockType14. System information combination 21 as defined in TS 36.508[18] clause 4.4.3.1.2 is used. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE send an ATTACH REQUEST message in the next 90 sec? | --> | ATTACH REQUEST | 1 | F |
| 4 | The SS changes Cell A parameters according to the row "T1" in table 9.2.1.1.27b.3.2-1 | - | - | - | - |
| 5 | SS sends Paging with eab-ParamModification. | <-- | Paging | - | - |
| 6 | SS starts transmitting modified SIB14 with eab-PerRSRP set to thresh1 | - | - | - | - |
| 7 | Check: Does the UE send an ATTACH REQUEST message in the next 90 sec? | --> | ATTACH REQUEST | 1 | F |
| 8 | SS sends Paging with eab-ParamModification. | <-- | Paging | - | - |
| 9 | SS starts transmitting modified SIB14 with eab-PerRSRP set to thresh2 | - | - | - | - |
| 10 | Check: Does the UE send an ATTACH REQUEST message in the next 90 sec? | --> | ATTACH REQUEST | 1 | F |
| 11 | SS sends Paging with eab-ParamModification. | <-- | Paging | - | - |
| 12 | SS starts transmitting modified SIB14 with eab-PerRSRP set to thresh3 | - | - | - | - |
| 13 | Check: Does the result of generic procedure UE Registration (State 2) as described in TS 36.508 [18] clause 4.5.2.3 indicate that UE is registered on Cell A? | - | - | 2 | P |
| - | At the end of the present test procedure sequence, the UE is in end state E-UTRA RRC_IDLE (State 2) according to TS 36.508 [18]. | - | - | - | - |

9.2.1.1.27b.3.3	Specific message contents

Table 9.2.1.1.27b.3.3-1: SystemInformationBlockType2 (all steps, Table 9.2.1.1.27b.3.2-2)

| Derivation Path: 36.508-1 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommonSIB-RSRPList |  |  |
| } |  |  |  |

Table 9.2.1.1.27b.3.3-2: RadioResourceConfigCommonSIB-RSRPList (Table 9.2.1.1.27b.3.3-1)

| Derivation Path: 36.508-1 Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB::= SEQUENCE { |  |  |  |
| prach-ConfigCommon-v1310 | PRACH-ConfigSIB-v1310-DEFAULT with condition CEmodeB PRACH_4CE | RSRP-Range[1] = 49 (-92 dBm) RSRP-Range[2] = 42 (-99 dBm) RSRP-Range[3] = 34 (-107dBm) |  |
| } |  |  |  |

Table 9.2.1.1.27b.3.3-3: SystemInformationBlockType14 (step 1 in 9.2.1.1.27b.3.2-2 )

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 | Not Present |  |  |
| eab-PerRSRP-r15 | thresh0 |  |  |
| } |  |  |  |

Table 9.2.1.1.27b.3.3-4: Paging (steps 5, 8 and 11 in table 9.2.1.1.27b.3.2-2)

| Derivation Path: 36.508-1 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| eab-ParamModification-r11 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.1.1.27b.3.3-5: SystemInformationBlockType14 (step 6 in table 9.2.1.1.27b.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 | Not Present |  |  |
| eab-PerRSRP-r15 | thresh1 |  |  |
| } |  |  |  |

Table 9.2.1.1.27b.3.3-6: SystemInformationBlockType14 (step 9 in table 9.2.1.1.27b.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 | Not Present |  |  |
| eab-PerRSRP-r15 | thresh2 |  |  |
| } |  |  |  |

Table 9.2.1.1.27b.3.3-7: SystemInformationBlockType14 (step 12 in table 9.2.1.1.27b.3.2-2)

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 | Not Present |  |  |
| eab-PerRSRP-r15 | thresh3 |  |  |
| } |  |  |  |

##### 9.2.1.1.28	Attach / Success / IMS

9.2.1.1.28.1	Test Purpose (TP)

(1)

with { the UE is switched-off and supports method II for P-CSCF discovery }

ensure that {

when { UE is powered on }

then { the UE transmits either an ATTACH REQUEST message and a PDN CONNECTIVITY REQUEST message or an ESM Information Transfer response with method II for P-CSCF discovery }

}

(2)

with { the UE supports conversational speech }

ensure that {

when { UE is available for calls }

then { the UE performs an initial IMS registration }

}

(3)

with { the UE is switched-off and IPv6 and IPv4 capable }

ensure that {

when { UE is powered on }

then { the UE transmits either an ATTACH REQUEST message and a PDN CONNECTIVITY REQUEST message or an ESM Information Transfer response with IPv4v6 for the PDN type }

9.2.1.1.28.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.229, clauses L.2.2.1, L.3.1.2 and TS 23.401 clause 5.3.1.1. Unless otherwise stated these are Rel-8 requirements.

[TS 24.229, clause L.2.2.1 “EPS bearer context activation and P-CSCF discovery”]

Prior to communication with the IM CN subsystem, the UE shall:

…

c)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

I.	When using IPv4, employ the Dynamic Host Configuration Protocol (DHCP) RFC 2132 [20F], the DHCPv4 options for SIP servers RFC 3361 [35A], and RFC 3263 [27A] as described in subclause 9.2.1. When using IPv6, employ Dynamic Host Configuration Protocol for IPv6 (DHCPv6) RFC 3315 [40], the DHCPv6 options for SIP servers RFC 3319 [41] and DHCPv6 options for Domain Name Servers (DNS) RFC 3646 [56C] as described in subclause 9.2.1.

II.	Transfer P-CSCF address(es) within the EPS bearer context activation procedure.

The UE shall indicate the request for a P-CSCF address to the network within the Protocol Configuration Options information element of the PDN CONNECTIVITY REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message, the UE shall assume that the list is prioritised with the first address within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

III.	The UE selects a P-CSCF from the list (see 3GPP TS 31.103 [15B]) stored in the ISIM.

IV.	The UE selects a P-CSCF from the list in IMS management object.

[TS 24.229, clause L.3.1.2 “Availability for calls”]

A UE shall perform an initial registration as specified in subclause 5.1.1.2, if all the following conditions are met:

1)	if the UE is operating in one of the following modes of operation (see 3GPP TS 24.301 [8J]):

a)	PS mode 1;

b)	CS/PS mode 1 and the UE is attached for EPS-Services only;

2)	if the UE is capable of receiving any (but not necessarily all) of the media types which the CS domain supports, such that the media type can also be used when accessing the IM CN subsystem using the current IP-CAN;

3)	if the media type of item 2 is an "audio" media type, and the UE supports codecs suitable for (conversational) speech;

4)	if the UE determines that its contact has not been bound to a public user identity using the IP-CAN, such that the contact is expected to be used for the delivery of incoming requests in the IM CN subsystem relating to the media of item 2 and item 3;

5)	if the IMSVoPS indicator, provided by the lower layers (see 3GPP TS 24.301 [8J]), indicates voice is supported; and

6)	if the procedures to perform the initial registration are enabled (see 3GPP TS 24.305 [8T]).

NOTE:	Regardless of any of the above conditions, a UE might attempt to register with the IM CN subsystem at any time.

[TS 23.401, clause 5.3.1.1]

…

The UE sets the PDN type during the Attach procedure based on its IP stack configuration as follows:

-	A UE which is IPv6 and IPv4 capable shall request for PDN type IPv4v6.

-	A UE which is only IPv4 capable shall request for PDN type IPv4.

-	A UE which is only IPv6 capable shall request for PDN type IPv6.

-	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall request for PDN type IPv4v6.

9.2.1.1.28.3	Test description

9.2.1.1.28.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.28.3.2	Test procedure sequence

Table 9.2.1.1.28.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4 | Steps 1 to 3 of the generic test procedure for UE registration (TS 36.508 4.5.2.3-1). | - | - | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1, 3 | P |
| 6-16 | Check: Does the UE transmit steps 5 to 15 of the generic test procedure for UE registration (TS 36.508 4.5.2.3-1). | - | - | 1 | P |
| - | EXCEPTION: In parallel to the event described in step 17 below the generic procedure for IMS signalling in the U-plane specified in table 9.2.1.1.28.3.2-2 takes place. | - | - | - | - |
| 17-19b1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |

Table 9.2.1.1.28.3.2-2: Procedure for IMS signalling in the U-plane

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-9 | Check: Does the UE transmit the registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11) | - | - | 2 | P |

9.2.1.1.28.3.3	Specific message contents

Table 9.2.1.1.28.3.3-1: Message PDN CONNECTIVITY REQUEST (step 5, table 9.2.1.1.28.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDN type | '011'B (IPv4v6) |  |  |
| Protocol configuration options |  |  |  |
| Container ID n | ‘0001’H (P-CSCF IPv6) | At least one value shall be present. |  |
| Container ID n | ‘000C’H (P-CSCF IPv4) |  |  |

Table 9.2.1.1.28.3.3-2: Message ESM INFORMATION TRANSFER RESPONSE (step 9a2 of the generic test procedure referenced in steps 6 -16, table 9.2.1.1.28.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol configuration options |  |  |  |
| Container ID n | ‘0001’H (P-CSCF IPv6) | At least one value shall be present. |  |
| Container ID n | ‘000C’H (P-CSCF IPv4) |  |  |

NOTE:	The containers can be transferred in PDN CONNECTIVITY REQUEST and/or in ESM INFORMATION TRANSFER RESPONSE message.

##### 9.2.1.1.28a	Attach / Success / IMS / Second PDN

9.2.1.1.28a.1	Test Purpose (TP)

(1)

with { the UE is switched-off and supports method II for P-CSCF discovery }

ensure that {

when { UE is powered on }

then { the UE transmits a PDN CONNECTIVITY REQUEST message with method II for P-CSCF discovery }

}

(2)

with { the UE supports conversational speech }

ensure that {

when { UE is available for calls }

then { the UE performs an initial IMS registration }

}

(3)

with { the UE is switched-off and IPv6 and IPv4 capable }

ensure that {

when { UE is powered on }

then { the UE transmits a PDN CONNECTIVITY REQUEST message with IPv4v6 for the PDN type }

9.2.1.1.28a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.229, clauses L.2.2.1, L.3.1.2 and TS 23.401 clause 5.3.1.1. Unless otherwise stated these are Rel-8 requirements.

[TS 24.229, clause L.2.2.1 “EPS bearer context activation and P-CSCF discovery”]

Prior to communication with the IM CN subsystem, the UE shall:

…

c)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

I.	When using IPv4, employ the Dynamic Host Configuration Protocol (DHCP) RFC 2132 [20F], the DHCPv4 options for SIP servers RFC 3361 [35A], and RFC 3263 [27A] as described in subclause 9.2.1. When using IPv6, employ Dynamic Host Configuration Protocol for IPv6 (DHCPv6) RFC 3315 [40], the DHCPv6 options for SIP servers RFC 3319 [41] and DHCPv6 options for Domain Name Servers (DNS) RFC 3646 [56C] as described in subclause 9.2.1.

II.	Transfer P-CSCF address(es) within the EPS bearer context activation procedure.

The UE shall indicate the request for a P-CSCF address to the network within the Protocol Configuration Options information element of the PDN CONNECTIVITY REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message, the UE shall assume that the list is prioritised with the first address within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

III.	The UE selects a P-CSCF from the list (see 3GPP TS 31.103 [15B]) stored in the ISIM.

IV.	The UE selects a P-CSCF from the list in IMS management object.

[TS 24.229, clause L.3.1.2 “Availability for calls”]

A UE shall perform an initial registration as specified in subclause 5.1.1.2, if all the following conditions are met:

1)	if the UE is operating in one of the following modes of operation (see 3GPP TS 24.301 [8J]):

a)	PS mode 1;

b)	CS/PS mode 1 and the UE is attached for EPS-Services only;

2)	if the UE is capable of receiving any (but not necessarily all) of the media types which the CS domain supports, such that the media type can also be used when accessing the IM CN subsystem using the current IP-CAN;

3)	if the media type of item 2 is an "audio" media type, and the UE supports codecs suitable for (conversational) speech;

4)	if the UE determines that its contact has not been bound to a public user identity using the IP-CAN, such that the contact is expected to be used for the delivery of incoming requests in the IM CN subsystem relating to the media of item 2 and item 3;

5)	if the IMSVoPS indicator, provided by the lower layers (see 3GPP TS 24.301 [8J]), indicates voice is supported; and

6)	if the procedures to perform the initial registration are enabled (see 3GPP TS 24.305 [8T]).

NOTE:	Regardless of any of the above conditions, a UE might attempt to register with the IM CN subsystem at any time.

[TS 23.401, clause 5.3.1.1]

…

The UE sets the PDN type during the Attach procedure based on its IP stack configuration as follows:

-	A UE which is IPv6 and IPv4 capable shall request for PDN type IPv4v6.

-	A UE which is only IPv4 capable shall request for PDN type IPv4.

-	A UE which is only IPv6 capable shall request for PDN type IPv6.

-	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall request for PDN type IPv4v6.

9.2.1.1.28a.3	Test description

9.2.1.1.28a.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.28a.3.2	Test procedure sequence

Table 9.2.1.1.28a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-17 | Steps 1 to 16 of the generic test procedure for UE registration (TS 36.508 subclause 4.5.2.3-1). | - | - | - | - |
| 18 | Check: Does the UE transmit step 1 of the generic test procedure to establish additional PDN connectivity (TS 36.508 subclause 4.5A.16.3-1)? | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST | 1, 3 | P |
| 19 | Step 2 of the generic test procedure to establish additional PDN connectivity (TS 36.508 subclause 4.5A.16.3-1). | - | - | - | - |
| 20 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of additional default bearer | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the event described in step 21 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 21 below the generic procedure for IMS signalling in the U-plane specified in table 9.2.1.1.28a.3.2-2 takes place. | - | - | - | - |
| 21 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 22-23 | Void | - | - | - | - |
| 24-27 | Steps 10 to 13 of the generic test procedure for network initiated release of additional PDN connectivity (TS 36.508 subclause 4.5A.18.3) are executed. The internet PDN is released. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508 Table 6.4.3A.2-1. | - | - | - | - |

Table 9.2.1.1.28a.3.2-2: Procedure for IMS signalling in the U-plane

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-9 | Check: Does the UE transmit the registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11)? | - | - | 2 | P |

9.2.1.1.28a.3.3	Specific message contents

Table 9.2.1.1.28a.3.3-1: Message PDN CONNECTIVITY REQUEST (step 18, table 9.2.1.1.28a.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDN type | '011'B (IPv4v6) |  |  |
| Protocol configuration options |  |  |  |
| Container ID n | ‘0001’H (P-CSCF IPv6) | At least one value shall be present. |  |
| Container ID n | ‘000C’H (P-CSCF IPv4) |  |  |

##### 9.2.1.1.28b	Attach / Success / IMS / New P-CSCF Discovery using PCO

9.2.1.1.28b.1	Test Purpose (TP)

(1)

with { the UE is switched-off and supports method II for P-CSCF discovery }

ensure that {

when { UE is powered on }

then { the UE transmits either an ATTACH REQUEST message and a PDN CONNECTIVITY REQUEST message or an ESM Information Transfer response with method II for P-CSCF discovery }

}

(2)

with { the UE sends SIP REGISTER to perform a SIP registration }

ensure that {

when { IM CN responds with 305 Use Proxy }

then { the UE re-establishes an IMS PDN to get a new P-CSCF address using method II and performs a SIP registration using the new P-CSCF}

}

9.2.1.1.28b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.229, clauses 5.1.1.2.1, 9.2.1, L.2.2.1.

[TS 24.229, clause 5.1.1.2.1]:

On receiving a 305 (Use Proxy) response to the unprotected REGISTER request, unless otherwise specified in access specific annexes (as described in annex B, annex L or annex U), the UE shall:

a)	ignore the contents of the Contact header field if it is included in the received message;

NOTE 17:	The 305 response is not expected to contain a Contact header field.

b)	release all IP-CAN bearers used for the transport of media according to the procedures in subclause 9.2.2;

c)	initiate either a new P-CSCF discovery procedure as described in subclause 9.2.1, or select a new P-CSCF, if the UE was pre-configured with more than one P-CSCF's IP addresses or domain names;

d)	select a P-CSCF address, which is different from the previously used address, from the address list; and

e)	perform the procedures for initial registration as described in subclause 5.1.1.2.

[TS 24.229, clause 9.2.1]:

Prior to communication with the IM CN subsystem, the UE shall:

a)	establish a connection with the IP-CAN;

b)	obtain an IP address using either the standard IETF protocols (e.g., DHCP or IPCP) or a protocol that is particular to the IP-CAN technology that the UE is utilising. The UE shall fix the obtained IP address throughout the period the UE is connected to the IM CN subsystem, i.e. from the initial registration and at least until the last deregistration; and

c)	acquire a P-CSCF address(es).

The UE may acquire an IP address via means other than the DHCP. In this case, upon acquiring an IP address, the UE shall request the configuration information (that includes the DNS and P-CSCF addresses) from the DHCP server.

The methods for acquiring a P-CSCF address(es) are:

I.	Employ Dynamic Host Configuration Protocol for IPv4 RFC 2131 [40A] or for IPv6 (DHCPv6) RFC 3315 [40]. Employ the DHCP options for SIP servers RFC 3319 [41] or, for IPv6, RFC 3361 [35A]. Employ the DHCP options for Domain Name Servers (DNS) RFC 3646 [56C].

The UE shall either:

-	in the DHCP query, request a list of SIP server domain names of P-CSCF(s) and the list of Domain Name Servers (DNS); or

-	request a list of SIP server IP addresses of P-CSCF(s).

II.	Obtain the P-CSCF address(es) by employing a procedure that the IP-CAN technology supports. (e.g. GPRS).

III.	The UE may use pre-configured P-CSCF address(es) (IP address or domain name). For example:

a.	The UE selects a P-CSCF from the list stored in ISIM or IMC;

b.	The UE selects a P-CSCF from the list in IMS management object.

NOTE 1:	Access-specific annexes provide additional guidance on the method to be used by the UE to acquire P-CSCF address(es).

When acquiring a P-CSCF address(es), the UE can freely select either method I or II or III.

NOTE 2:	In case a P-CSCF address is provisioned or received as a FQDN, procedures according to RFC 3263 [27A] will provide the resolution of the FQDN.

The UE may also request a DNS Server IP address(es) as specified in RFC 3315 [40] and RFC 3646 [56C] or RFC 2131 [40A].

[TS 24.229, clause L.2.2.1]:

Prior to communication with the IM CN subsystem, the UE shall:

…

c)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

…

II.	Transfer P-CSCF address(es) within the EPS bearer context activation procedure.

The UE shall indicate the request for a P-CSCF address to the network within the Protocol Configuration Options information element of the PDN CONNECTIVITY REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message, the UE shall assume that the list is ordered top-down with the first P-CSCF address within the Protocol Configuration Options information element as the P-CSCF address having the highest preference and the last P-CSCF address within the Protocol Configuration Options information element as the P-CSCF address having the lowest preference.

...

If the UE is configured to use Option II above and detects that all P-CSCFs known by the UE have been used when the UE selects a different P-CSCF as a result of:

-	receiving 305 (Use Proxy) to the REGISTER request;

-	receiving 504 (Server Time-out); or

-	expiration of the timer F at the UE,

then if there are more than one PDN connection that UE is connected to and unless the IP-CAN bearer is in use by other applications, the UE shall:

1)	release IP-CAN bearer that is used only for the transport of SIP signalling and that are not used for other non-IMS applications, but shall not release emergency IP-CAN bearers; and

2)	unless the UE decides the service is no longer needed,

a)	perform a new P-CSCF discovery procedure as described in subclause 9.2.1; and

b)	perform the procedures for initial registration as described in subclause 5.1.1.2.

9.2.1.1.28b.3	Test description

9.2.1.1.28b.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	None

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.28b.3.2	Test procedure sequence

Table 9.2.1.1.28b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4 | Steps 1 to 3 of the generic test procedure for UE registration (TS 36.508 4.5.2.3-1). | - | - | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-16 | Check: Does the UE transmit steps 5 to 15 of the generic test procedure for UE registration (TS 36.508 4.5.2.3-1)? | - | - | 1 | P |
| - | EXCEPTION: In parallel to the event described in step 17 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |  |  |
| - | EXCEPTION: In parallel to the event described in step 17 below the procedure for IMS signalling specified in Table 9.2.1.1.28b.3.2-2 takes place | - | - |  |  |
| 17 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |  |
| 17Aa1-17Ac1 | Steps 16a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3-1 are performed. | - | - |  |  |
| 18 | Check: After the parallel behaviour in Table 9.2.1.1.28b.3.2-2, does the UE transmit a PDN DISCONNECT REQUEST? | --> | RRC: ULInformationTransfer PDN DISCONNECT REQUEST | 2 | P |
| 19-20 | Steps 1 to 2 of the generic test procedure for EPS Bearer Deactivation (TS 36.508 4.5A.15A.3-1). | - | - | - | - |
| 21 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST message to request an IMS PDN? | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST | 2 | P |
| 22 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to TS 36.508 subclause 4.8.2.2.1.1. This message includes The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |  |  |
| 23 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |  |  |
| - | EXCEPTION: In parallel to the event described in step 24 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |  |  |
| - | EXCEPTION: In parallel to the event described in step 24 below the procedure for IMS signalling specified in Table 9.2.1.1.28b.3.2-3 takes place | - | - |  |  |
| 24 | This message includes the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | 2 | P |

Table 9.2.1.1.28b.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | Step 4 defined in annex C.2 of TS 34.229-1 [35]. | --> | REGISTER | - | - |
| 3 | SS responds with 305 Use Proxy defined in annex A.4.8 of TS 34.229-1 [35] | <-- | 305 Use Proxy | - | - |

Table 9.2.1.1.28b.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | Check: Does the UE send REGISTER request as defined in Step4 in TS 34.229-1 [43] subclause C.2 for SIP registration? | --> | REGISTER | 2 | P |
| 3-9 | Steps 5 to 11 defined in TS 34.229-1 [43] subclause C.2 for SIP registration | - | - | - | - |

9.2.1.1.28b.3.3	Specific message contents

Table 9.2.1.1.28b.3.3-1: PDN CONNECTIVITY REQUEST (step 5, table 9.2.1.1.28b.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDN type | '011'B (IPv4v6) |  |  |
| Protocol configuration options |  |  |  |
| Container ID 1 | ‘0001’H (P-CSCF IPv6) | At least one value shall be present. |  |
| Container ID 2 | ‘000C’H (P-CSCF IPv4) |  |  |

Table 9.2.1.1.28b.3.3-2: ESM INFORMATION TRANSFER RESPONSE (step 9a2 of the generic test procedure referenced in steps 6 -16, table 9.2.1.1.28b.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol configuration options |  |  |  |
| Container ID 1 | ‘0001’H (P-CSCF IPv6) | At least one value shall be present. |  |
| Container ID 2 | ‘000C’H (P-CSCF IPv4) |  |  |

NOTE:	The containers can be transferred in PDN CONNECTIVITY REQUEST and/or in ESM INFORMATION TRANSFER RESPONSE message.

Table 9.2.1.1.28b.3.3-3: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 22, table 9.2.1.1.28b.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol configuration options |  |  |  |
| Container ID 1 | ‘0001’H (P-CSCF IPv6) |  | P-CSCF IPv6 |
| Container ID 1 contents | IPv6 address different from the value in step 5 | P-CSCF IPv6 Address |  |
| Container ID 2 | ‘000C’H (P-CSCF IPv4) |  | P-CSCF IPv4 |
| Container ID 2 contents | IPv4 address different from the value in step 5 | P-CSCF IPv4 Address |  |

##### 9.2.1.1.29	Attach / Rejected / IMEI not accepted

9.2.1.1.29.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST for emergency bearer services }

ensure that {

when { the UE receives an ATTACH REJECT message with the EMM cause set to “IMEI not accepted” }

then { UE enters state EMM-DEREGISTERED }

}

9.2.1.1.29.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.2.5A.

[TS 24.301, clause 5.5.1.2.5A]

If the attach request for emergency bearer services cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including EMM cause #5 "IMEI not accepted" or one of the EMM cause values as described in subclause 5.5.1.2.5.

NOTE:	If EMM cause #11 is sent to a UE of a roaming subscriber attaching for emergency bearer services and the UE is in automatic network selection mode, it cannot obtain normal service provided by this PLMN.

Upon receiving the ATTACH REJECT message including EMM cause #5, the UE shall enter the state EMM-DEREGISTERED.NO-IMSI.

…

9.2.1.1.29.3	Test description

9.2.1.1.29.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B:

-	Cell A and Cell B (home PLMN, different TAs),

UE:

- Without a valid USIM inserted

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

9.2.1.1.29.3.2	Test procedure sequence

Table 9.2.1.1.29.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - cell A as the "Serving cell". - cell B as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on without a USIM inserted. | - | - | - | - |
| 3 | Causes the UE originate an emergency bearer service (Note1). | - | - | - | - |
| 4 | The UE transmits an ATTACH REQUEST message to attach for emergency bearer services, including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 5 | The SS transmits an ATTACH REJECT message with EMM cause = " IMEI not accepted " as specified. | <-- | ATTACH REJECT | - | - |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? Note: Cell B belongs to the same PLMN where the UE was rejected but a different TA | --> | ATTACH REQUEST | 1 | F |
| Note 1: The request to originate an emergency service may be performed by MMI or AT command. |  |  |  |  |  |

9.2.1.1.29.3.3	Specific message contents

Table 9.2.1.1.29.3.3-1: Message ATTACH REJECT (steps 5, Table 9.2.1.1.29.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Security header type | 0000 | "Plain NAS message, not security protected " |  |
| EMM cause | 00000101 | "#5 IMEI not accepted " |  |
| ESM message container | Not present |  |  |

##### 9.2.1.1.30	Void

##### 9.2.1.1.31	Attach / Success / Extended and spare fields in UE Network Capability

9.2.1.1.31.1	Test Purpose (TP)

(1)

with { the UE is powered on and has transmitted an ATTACH REQUEST message }

ensure that {

when { the UE receives additional octets in ATTACH ACCEPT that it does not comprehend (those added in later versions of the protocols without using the mechanisms defined in TS 24.007 c) or d)) }

then { the UE ignores the contents of these octets and transmits ATTACH COMPLETE }

}

9.2.1.1.31.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.007, clause 11.2.2.1, and TS 24.301 clauses 5.5.1.2.2, 5.5.1.2.3, and 5.5.1.2.4.

[TS 24.007, clause 11.2.2.1 “Description methods for IE structures, Tables”]

According to this description method, the IE is presented in its maximum format, i.e., T, TV, TLV or TLV-E, in a picture representing the bits in a table, each line representing an octet. Bits appear in the occidental order, i.e., from left of the page to right of the page, and from top of the page to bottom of the page.

Boxes so delimited contains typically the field name, possibly an indication of which bits in the field are in the box, and possibly a value (e.g., for spare bits).

A specific method can be used in the IE description to describe a branching structure, i.e., a structure variable according to the value of particular fields in the IE. This design is unusual outside type 4 and type 6 IEs, and as, a design rule, should be used only in type 4 and type 6 IEs.

a)	The octet number of an octet within the IE is defined typically in the table. It consists of a positive integer, possibly of an additional letter, and possibly of an additional asterisk, see clause f). The positive integer identifies one octet or a group of octets.

b)	Each octet group is a self-contained entity. The internal structure of an octet group may be defined in alternative ways.

c)	An octet group is formed by using some extension mechanism. The preferred extension mechanism is to extend an octet (N) through the next octet(s) (Na, Nb, etc.) by using bit 8 in each octet as an extension bit.

-	The bit value "0" indicates that the octet group continues through to the next octet. The bit value "1" indicates that this octet is the last octet of the group. If one octet (Nb) is present, the preceding octets (N and Na) shall also be present.

-	In the format descriptions of the individual information elements, bit 8 is marked "0/1 ext" if another octet follows. Bit 8 is marked "1 ext" if this is the last octet in the extension domain.

-	Additional octets may be defined in later versions of the protocols ("1 ext" changed to "0/1 ext") and equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in the formal description of the messages and of the individual information elements only takes into account this version of the protocols.

d)	In addition to the extension mechanism defined above, an octet (N) may be extended through the next octet(s) (N+1, N+2 etc.) by indications in bits 7-1 (of octet N).

e)	The mechanisms in c) and d) may be combined.

f)	Optional octets are marked with asterisks (*). As a design rule, the presence or absence of an optional octet should be determinable from information in the IE and preceding the optional octet. Care should be taken not to introduce ambiguities with optional octets.

g)	At the end of the IE, additional octets may be added in later versions of the protocols also without using the mechanisms defined in c) and d). Equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in the formal description of the messages and of the individual information elements only takes into account this version of the protocols.

[TS 24.301, clause 5.5.1.2.2 “Attach procedure initiation”]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

The UE shall include the IMSI in the EPS mobile identity IE in the ATTACH REQUEST message if the selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs and:

a)	the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]; or

b)	the UE is in NB-S1 mode.

For all other cases, the UE shall handle the EPS mobile identity IE in the ATTACH REQUEST message as follows:

a)	if the UE operating in the single-registration mode is performing an inter-system change from N1 mode to S1 mode or the UE was previously registered in N1 mode before entering state 5GMM-DEREGISTERED and:

1)	the UE has received the interworking without N26 interface indicator set to "interworking without N26 interface supported" from the network and:

i)	if the UE holds a valid GUTI, the UE shall include the valid GUTI into the EPS mobile identity IE, include Old GUTI type IE with GUTI type set to "native GUTI" and include the UE status IE with a 5GMM registration status set to:

-	"UE is in 5GMM-REGISTERED state" if the UE is in 5GMM-REGISTERED state; or

-	"UE is in 5GMM-DEREGISTERED state" if the UE is in 5GMM-DEREGISTERED state; or

ii)	if the UE does not hold a valid GUTI, the UE shall include the IMSI in the EPS mobile identity IE; or

2)	the UE has received the interworking without N26 interface indicator set to "interworking without N26 interface not supported" from the network and:

i)	if the UE holds a valid 5G-GUTI, the UE shall include a GUTI, mapped from 5G-GUTI into the EPS mobile identity IE, include Old GUTI type IE with GUTI type set to "native GUTI" and include the UE status IE with a 5GMM registration status set to "UE is in 5GMM-DEREGISTERED state";

ii)	if the UE holds a valid GUTI and does not hold a valid 5G-GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE and include Old GUTI type IE with GUTI type set to "native GUTI"; or

iii)	if the UE holds neither a valid GUTI nor a valid 5G-GUTI, the UE shall include the IMSI in the EPS mobile identity IE; or

NOTE 1:	The value of the EMM registration status included by the UE in the UE status IE is not used by the MME.

b)	otherwise:

1)	if the UE supports neither A/Gb mode nor Iu mode, the UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. In addition, the UE shall include Old GUTI type IE with GUTI type set to "native GUTI". If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message; or

2)	If the UE supports A/Gb mode or Iu mode or both and:

i)	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE;

NOTE 2:	The mapping of the P-TMSI and the RAI to the GUTI is specified in 3GPP TS 23.003 [2].

ii)	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI";

iii)	if the TIN is deleted and:

-	the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI";

-	the UE does not hold a valid GUTI but holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE; or

-	the UE does not hold a valid GUTI, P-TMSI or RAI, the UE shall include the IMSI in the EPS mobile identity IE; or

iv)	otherwise the UE shall include the IMSI in the EPS mobile identity IE.

If the UE is operating in the dual-registration mode and it is in 5GMM state 5GMM-REGISTERED, the UE shall include the UE status IE with the 5GMM registration status set to "UE is in 5GMM-REGISTERED state".

NOTE 3:	The value of the EMM registration status included by the UE in the UE status IE is not used by the MME.

If the UE is attaching for emergency bearer services and does not hold a valid GUTI, P-TMSI or IMSI as described above, the IMEI shall be included in the EPS mobile identity IE.

If the UE in limited service state is attaching for access to RLOS and does not hold a valid GUTI, P-TMSI or IMSI as described above, the IMEI shall be included in the EPS mobile identity IE.

If the UE supports A/Gb mode or Iu mode or if the UE needs to indicate its UE specific DRX parameter to the network, the UE shall include the UE specific DRX parameter in the DRX parameter IE in the ATTACH REQUEST message. If the UE in NB-S1 mode needs to indicate the UE specific DRX parameter in NB-S1 mode to the network, it shall include the UE specific DRX parameter in NB-S1 mode in the DRX parameter in NB-S1 mode IE in the ATTACH REQUEST message.

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

If the UE supports WUS assistance, then the UE shall set the WUSA bit to "WUS assistance supported" in the UE network capability IE, and if the UE is not attaching for emergency bearer services, the UE may include its UE paging probability information in the Requested WUS assistance information IE of the ATTACH REQUEST message.

If the UE supports SRVCC to GERAN/UTRAN, the UE shall set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported".

If the UE supports vSRVCC from S1 mode to Iu mode, then the UE shall set the H.245 after handover capability bit to "H.245 after SRVCC handover capability supported" and additionally set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported" in the ATTACH REQUEST message.

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

If the UE supports ProSe direct discovery, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct discovery bit to "ProSe direct discovery supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports ProSe direct communication, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct communication bit to "ProSe direct communication supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports acting as a ProSe UE-to-network relay, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe UE-to-network relay bit to "acting as a ProSe UE-to-network relay supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports NB-S1 mode, Non-IP or Ethernet PDN type, N1 mode, or if the UE supports DNS over (D)TLS (see 3GPP TS 33.501 [24]), then the UE shall support the extended protocol configuration options IE.

If the UE supports the extended protocol configuration options IE, then the UE shall set the ePCO bit to "extended protocol configuration options supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports the restriction on use of enhanced coverage, then the UE shall set the RestrictEC bit to "Restriction on use of enhanced coverage supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports the control plane data back-off timer T3448, the UE shall set the CP backoff bit to "back-off timer for transport of user data via the control plane supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, then the UE shall set the Control plane CIoT EPS optimization bit to "Control plane CIoT EPS optimization supported" in the UE network capability IE of the ATTACH REQUEST message. If the UE is capable of NB-N1 mode, then the UE shall set the Control plane CIoT 5GS optimization bit to "Control plane CIoT 5GS optimization supported" in the N1 UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, supports NB-S1 mode only, and requests to attach for EPS services and "SMS only", the UE shall indicate the SMS only requested bit to "SMS only" in the additional update type IE and shall set the EPS attach type IE to "EPS attach" in the ATTACH REQUEST message.

If the UE supports CIoT EPS optimizations, it shall indicate in the UE network capability IE of the ATTACH REQUEST message whether it supports EMM-REGISTERED without PDN connection.

If the UE supports S1-U data transfer and multiple user plane radio bearers (see 3GPP TS 36.306 [44], 3GPP TS 36.331 [22]) in NB-S1 mode, then the UE shall set the Multiple DRB support bit to "Multiple DRB supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports control plane MT-EDT, then the UE shall set the CP-MT-EDT bit to "Control plane Mobile Terminated-Early Data Transmission supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports user plane MT-EDT, then the UE shall set the UP-MT-EDT bit to "User plane Mobile Terminated-Early Data Transmission supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports V2X communication over E-UTRA-PC5, then the UE shall set the V2X PC5 bit to "V2X communication over E-UTRA-PC5 supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports V2X communication over NR-PC5, then the UE shall set the V2X NR-PC5 bit to "V2X communication over NR-PC5 supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports service gap control, then the UE shall set the SGC bit to "service gap control supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports dual connectivity with New Radio (NR), then the UE shall set the DCNR bit to "dual connectivity with NR supported" in the UE network capability IE of the ATTACH REQUEST message and shall include the UE additional security capability IE in the ATTACH REQUEST message.

If the UE supports N1 mode, the UE shall set the N1mode bit to "N1 mode supported" in the UE network capability IE of the ATTACH REQUEST message and shall include the UE additional security capability IE in the ATTACH REQUEST message.

If the UE supports signalling for a maximum number of 15 EPS bearer contexts, then the UE shall set the 15 bearers bit to "Signalling for a maximum number of 15 EPS bearer contexts supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports ciphered broadcast assistance data and needs to obtain new ciphering keys, the UE shall include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the ATTACH REQUEST message.

If EMM-REGISTERED without PDN connection is not supported by the UE or the MME, or if the UE wants to request PDN connection with the attach procedure, the UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container IE.

If EMM-REGISTERED without PDN connection is supported by the UE and the MME, and the UE does not want to request PDN connection with the attach procedure, the UE shall send the ATTACH REQUEST message together with an ESM DUMMY MESSAGE contained in the ESM message container information element.

In WB-S1 mode, if the UE supports RACS, the UE shall:

a)	set the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST message; and

b)	if the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected PLMN, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE of the ATTACH REQUEST message.

If the attach procedure is initiated following an inter-system change from N1 mode to S1 mode in EMM-IDLE mode or the UE which was previously registered in N1 mode before entering state 5GMM-DEREGISTERED initiates the attach procedure:

a)	if the UE has received an "interworking without N26 interface not supported" indication from the network and a valid 5G NAS security context exists in the UE, the UE shall integrity protect the ATTACH REQUEST message combined with the message included in the ESM message container IE using the 5G NAS security context;

b)	otherwise:

1)	if a valid EPS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the message included in the ESM message container IE using the EPS security context; or

2)	if the UE does not have a valid EPS security context, the ATTACH REQUEST message combined with the message included in the ESM message container IE is not integrity protected.

Figure 5.5.1.2.2.1: Attach procedure and combined attach procedure

[TS 24.301, clause 5.5.1.2.3 “EMM common procedure initiation”]

The network may initiate EMM common procedures, e.g. the identification, authentication and security mode control procedures during the attach procedure, depending on the information received in the ATTACH REQUEST message (e.g. IMSI, GUTI and KSI).

If the network receives an ATTACH REQUEST message containing the Old GUTI type IE and the EPS mobile identity IE with type of identity indicating "GUTI", and the network does not follow the use of the most significant bit of the <MME group id> as specified in 3GPP TS 23.003 [2], subclause 2.8.2.2.2, the network shall use the Old GUTI type IE to determine whether the mobile identity included in the EPS mobile identity IE is a native GUTI or a mapped GUTI.

During an attach for emergency bearer services or an attach for access to RLOS, the MME may choose to skip the authentication procedure even if no EPS security context is available and proceed directly to the execution of the security mode control procedure as specified in subclause 5.4.3.

[TS 24.301, clause 5.5.1.2.4 “Attach accepted by the network”]

During an attach for emergency bearer services, if not restricted by local regulations, the MME shall not check for mobility and access restrictions, regional restrictions, subscription restrictions, or perform CSG access control when processing the ATTACH REQUEST message. The network shall not apply subscribed APN based congestion control during an attach procedure for emergency bearer services.

During an attach for access to RLOS, the MME shall not check for access restrictions, regional restrictions and subscription restrictions when processing the ATTACH REQUEST message.

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450.

If the attach request included the PDN CONNECTIVITY REQUEST message in the ESM message container information element to request PDN connectivity, the MME when accepting the attach request shall:

-	send the ATTACH ACCEPT message together with an ESM DUMMY MESSAGE contained in the ESM message container information element and discard the ESM message container information element included in the attach request if:

-	the UE indicated support of EMM-REGISTERED without PDN connection in the UE network capability IE of the ATTACH REQUEST message;

-	the MME supports EMM-REGISTERED without PDN connection and PDN connection is restricted according to the user's subscription data;

-	the attach type is not set to "EPS emergency attach" or "EPS RLOS attach"; and

-	the request type of the UE requested PDN connection is not set to "emergency" or "RLOS";

-	otherwise, send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). In WB-S1 mode, the network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2). In NB-S1 mode the network shall not initiate the activation of dedicated bearers.

If EMM-REGISTERED without PDN connection is supported by the UE and the MME, and the UE included an ESM DUMMY MESSAGE in the ESM message container information element of the ATTACH REQUEST message, the MME shall send the ATTACH ACCEPT message together with an ESM DUMMY MESSAGE contained in the ESM message container information element.

If the attach request is accepted by the network, the MME shall delete the stored UE radio capability information or the UE radio capability ID, if any.

In NB-S1 mode, if the attach request is accepted by the network, the MME shall set the EMC BS bit to zero in the EPS network feature support IE included in the ATTACH ACCEPT message to indicate that support of emergency bearer services in NB-S1 mode is not available.

If the UE has included the UE network capability IE or the MS network capability IE or both in the ATTACH REQUEST message, the MME shall store all octets received from the UE, up to the maximum length defined for the respective information element.

NOTE 1:	This information is forwarded to the new MME during inter-MME handover or to the new SGSN during inter-system handover to A/Gb mode or Iu mode.

NOTE 2:	For further details concerning the handling of the MS network capability and UE network capability in the MME see also 3GPP TS 23.401 [10].

If the UE specific DRX parameter was included in the DRX Parameter IE in the ATTACH REQUEST message, the MME shall replace any stored UE specific DRX parameter with the received parameter and use it for the downlink transfer of signalling and user data in WB-S1 mode.

In NB-S1 mode, if the DRX parameter in NB-S1 mode IE was included in the ATTACH REQUEST message, the MME shall provide to the UE the Negotiated DRX parameter in NB-S1 mode IE in the ATTACH ACCEPT message. The MME shall replace any stored UE specific DRX parameter in NB-S1 mode with the negotiated DRX parameter and use it for the downlink transfer of signalling and user data in NB-S1 mode.

NOTE 3:	In NB-S1 mode, if a DRX parameter was included in the Negotiated DRX parameter in NB-S1 mode IE in the ATTACH ACCEPT message, then the UE stores and uses the received DRX parameter in NB-S1 mode (see 3GPP TS 36.304 [21]). If the UE did not receive a DRX parameter in the Negotiated DRX parameter in NB-S1 mode IE, or if the Negotiated DRX parameter in NB-S1 mode IE was not included in the ATTACH ACCEPT message, then the UE uses the cell specific DRX value in NB-S1 mode (see 3GPP TS 36.304 [21]).

In NB-S1 mode, if the UE requested "SMS only" in the Additional update type IE, supports NB-S1 mode only and the MME decides to accept the attach request for EPS services and "SMS only", the MME shall indicate "SMS only" in the Additional update result IE and shall set the EPS attach result IE to "EPS only" in the ATTACH ACCEPT message.

The MME shall include the extended DRX parameters IE in the ATTACH ACCEPT message only if the extended DRX parameters IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of eDRX.

If

-	the UE supports WUS assistance; and

-	the MME supports and accepts the use of WUS assistance,

then the MME shall determine the negotiated UE paging probability information for the UE, store it in the EMM context of the UE, and if the UE is not attaching for emergency bearer services, the MME shall include it in the Negotiated WUS assistance information IE in the ATTACH ACCEPT message. The MME may take into account the UE paging probability information received in the Requested WUS assistance information IE when determining the negotiated UE paging probability information for the UE.

NOTE 4:	Besides the UE paging probability information requested by the UE, the MME can take local configuration or previous statistical information for the UE into account when determining the negotiated UE paging probability information for the UE (see 3GPP TS 23.401 [10]).

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The MME shall not assign a TAI list containing both tracking areas in NB-S1 mode and tracking areas in WB-S1 mode. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

NOTE 5:	When assigning the TAI list, the MME can take into account the eNodeB's capability of support of CIoT EPS optimization.

The MME may include T3412 extended value IE in the ATTACH ACCEPT message only if the UE indicates support of the extended periodic timer T3412 in the MS network feature support IE in the ATTACH REQUEST message.

The MME shall include the T3324 value IE in the ATTACH ACCEPT message only if the T3324 value IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of PSM.

If the MME supports and accepts the use of PSM, and the UE included the T3412 extended value IE in the ATTACH REQUEST message, then the MME shall take into account the T3412 value requested when providing the T3412 value IE and the T3412 extended value IE in the ATTACH ACCEPT message.

NOTE 6:	Besides the value requested by the UE, the MME can take local configuration or subscription data provided by the HSS into account when selecting a value for T3412 (3GPP TS 23.401 [10] subclause 4.3.17.3).

If the UE indicates support for EMM-REGISTERED without PDN connection in the ATTACH REQUEST message and the MME supports EMM-REGISTERED without PDN connection, the MME shall indicate support for EMM-REGISTERED without PDN connection in the EPS network feature support IE of the ATTACH ACCEPT message. The UE and the MME shall use the information whether the peer entity supports EMM-REGISTERED without PDN connection as specified in the present clause 5 and in clause 6.

If the UE requests "control plane CIoT EPS optimization" in the Additional update type IE, indicates support of control plane CIoT EPS optimization in the UE network capability IE and the MME decides to accept the requested CIoT EPS optimization and the attach request, the MME shall indicate "control plane CIoT EPS optimization supported" in the EPS network feature support IE.

If the MME supports NB-S1 mode, Non-IP or Ethernet PDN type, inter-system change with 5GS or the network wants to enforce the use of DNS over (D)TLS (see 3GPP TS 33.501 [24]), then the MME shall support the extended protocol configuration options IE.

NOTE 7: 	Support of DNS over (D)TLS is based on the informative requirements as specified in 3GPP TS 33.401 [19] and it is implemented based on the operator requirement.

If the MME supports the extended protocol configuration options IE and the UE indicated support of the extended protocol configuration options IE, then the MME shall set the ePCO bit to "extended protocol configuration options supported" in the EPS network feature support IE of the ATTACH ACCEPT message.

If the UE indicates support for restriction on use of enhanced coverage in the ATTACH REQUEST message, and the network decides to restrict the use of enhanced coverage for the UE, then the MME shall set the RestrictEC bit to "Use of enhanced coverage is restricted" in the EPS network feature support IE of the ATTACH ACCEPT message.

If the UE has indicated support for the control plane data back-off timer, and the MME decides to activate the congestion control for transport of user data via the control plane, then the MME shall include the T3448 value IE in the ATTACH ACCEPT message.

If the UE indicates support for dual connectivity with NR in the ATTACH REQUEST message, and the MME decides to restrict the use of dual connectivity with NR for the UE, then the MME shall set the RestrictDCNR bit to "Use of dual connectivity with NR is restricted" in the EPS network feature support IE of the ATTACH ACCEPT message.

If the UE indicates support for N1 mode in the ATTACH REQUEST message and the MME supports inter-system interworking with 5GS, the MME may set the IWK N26 bit to either:

-	"interworking without N26 interface not supported" if the MME supports N26 interface; or

-	"interworking without N26 interface supported" if the MME does not support N26 interface

in the EPS network feature support IE in the ATTACH ACCEPT message.

If the UE requests ciphering keys for ciphered broadcast assistance data in the ATTACH REQUEST message and the MME has valid ciphering key data applicable to the UE's subscription, then the MME shall include the ciphering key data in the Ciphering key data IE of the ATTACH ACCEPT message.

If due to operator policies unsecured redirection to a GERAN cell is not allowed in the current PLMN, the MME shall set the redir-policy bit to "Unsecured redirection to GERAN not allowed" in the Network policy IE of the ATTACH ACCEPT message.

The MME may include the T3447 value IE set to the service gap time value in the ATTACH ACCEPT message if:

-	the UE has indicated support for service gap control; and

-	a service gap time value is available in the EMM context.

If the network supports signalling for a maximum number of 15 EPS bearer contexts and the UE indicated support of signalling for a maximum number of 15 EPS bearer contexts in the ATTACH REQUEST message, then the MME shall set the 15 bearers bit to "Signalling for a maximum number of 15 EPS bearer contexts supported" in the EPS network feature support IE of the ATTACH ACCEPT message.

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI or IMEI, or the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

For a shared network, the TAIs included in the TAI list can contain different PLMN identities. The MME indicates the selected core network operator PLMN identity to the UE in the GUTI (see 3GPP TS 23.251 [8B]).

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

If the ATTACH ACCEPT message contains the T3412 extended value IE, then the UE shall use the value in T3412 extended value IE as periodic tracking area update timer (T3412). If the ATTACH ACCEPT message does not contain T3412 extended value IE, then the UE shall use the value in T3412 value IE as periodic tracking area update timer (T3412).

If the ATTACH ACCEPT message contains the T3324 value IE, then the UE shall use the included timer value for T3324 as specified in 3GPP TS 24.008 [13], subclause 4.7.2.8.

If the ATTACH ACCEPT message contains the DCN-ID IE, then the UE shall store the included DCN-ID value together with the PLMN code of the registered PLMN in a DCN-ID list in a non-volatile memory in the ME as specified in annex C.

The MME may also include a list of equivalent PLMNs in the ATTACH ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if the attach procedure is neither for emergency bearer services nor for access to RLOS, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs" or in the list of "forbidden PLMNs for GPRS service". In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the ATTACH ACCEPT message. If the ATTACH ACCEPT message does not contain a list, then the UE shall delete the stored list.

If the attach procedure is neither for emergency bearer services nor for access to RLOS, and if the PLMN identity of the registered PLMN is a member of the list of "forbidden PLMNs" or the list of "forbidden PLMNs for GPRS service", any such PLMN identity shall be deleted from the corresponding list(s).

The network informs the UE about the support of specific features, such as IMS voice over PS session, location services (EPC-LCS, CS-LCS), emergency bearer services, or CIoT EPS optimizations, in the EPS network feature support information element. In a UE with IMS voice over PS capability, the IMS voice over PS session indicator and the emergency bearer services indicator shall be provided to the upper layers. The upper layers take the IMS voice over PS session indicator into account as specified in 3GPP TS 23.221 [8A], subclause 7.2a and subclause 7.2b, when selecting the access domain for voice sessions or calls. When initiating an emergency call, the upper layers also take both the IMS voice over PS session indicator and the emergency bearer services indicator into account for the access domain selection. In a UE with LCS capability, location services indicators (EPC-LCS, CS-LCS) shall be provided to the upper layers. When MO-LR procedure is triggered by the UE's application, those indicators are taken into account as specified in 3GPP TS 24.171 [13C].

If the RestrictDCNR bit is set to "Use of dual connectivity with NR is restricted" in the EPS network feature support IE of the ATTACH ACCEPT message, the UE shall provide the indication that dual connectivity with NR is restricted to the upper layers.

The UE supporting N1 mode shall operate in the mode for inter-system interworking with 5GS as follows:

-	if the IWK N26 bit in the EPS network feature support IE is set to "interworking without N26 interface not supported", the UE shall operate in single-registration mode;

-	if the IWK N26 bit in the EPS network feature support IE is set to "interworking without N26 interface supported" and the UE supports dual-registration mode, the UE may operate in dual-registration mode; or

NOTE 8:	The registration mode used by the UE is implementation dependent.

-	if the IWK N26 bit in the EPS network feature support IE is set to "interworking without N26 interface supported" and the UE only supports single-registration mode, the UE shall operate in single-registration mode.

The UE shall treat the interworking without N26 interface indicator as valid in the entire PLMN and equivalent PLMNs. The interworking procedures required for coordination between 5GMM and EMM without N26 interface are specified in 3GPP TS 24.501 [54].

If the redir-policy bit is set to "Unsecured redirection to GERAN not allowed" in the Network policy IE of the ATTACH ACCEPT message, the UE shall set the network policy on unsecured redirection to GERAN for the current PLMN to "Unsecured redirection to GERAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is not allowed. If the redir-policy bit is set to "Unsecured redirection to GERAN allowed" or if the Network policy IE is not included in the ATTACH ACCEPT message, the UE shall set the network policy for the current PLMN to "Unsecured redirection to GERAN allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is allowed. The UE shall set the network policy on unsecured redirection to GERAN to "Unsecured redirection to GERAN not allowed" and indicate this to the lower layers when any of the following events occurs:

-	the UE initiates an EPS attach or tracking area updating procedure in a PLMN different from the PLMN where the UE performed the last successful EPS attach or tracking area updating procedure;

-	the UE is switched on; or

-	the UICC containing the USIM is removed.

If the UE has initiated the attach procedure due to manual CSG selection and receives an ATTACH ACCEPT message; and the UE sent the ATTACH REQUEST message in a CSG cell, the UE shall check if the CSG ID and associated PLMN identity of the cell are contained in the Allowed CSG list. If not, the UE shall add that CSG ID and associated PLMN identity to the Allowed CSG list and the UE may add the HNB Name (if provided by lower layers) to the Allowed CSG list if the HNB Name is present in neither the Operator CSG list nor the Allowed CSG list.

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, and if the UE has requested PDN connectivity the UE shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

Additionally, the UE shall reset the attach attempt counter, enter state EMM-REGISTERED, and set the EPS update status to EU1 UPDATED.

If EMM-REGISTERED without PDN connection is supported by the UE and the MME, and the UE receives the ATTACH ACCEPT message combined with an ESM DUMMY MESSAGE, the UE shall send an ATTACH COMPLETE message together with an ESM DUMMY MESSAGE contained in the ESM message container information element to the network.

If the UE receives the ATTACH ACCEPT message from a PLMN for which a PLMN-specific attempt counter or PLMN-specific PS-attempt counter is maintained (see subclause 5.3.7b), then the UE shall reset these counters. If the UE maintains a counter for "SIM/USIM considered invalid for GPRS services", then the UE shall reset this counter.

When the UE receives any ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST messages during the attach procedure, and if the UE has requested PDN connectivity the UE shall forward the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message(s) to the ESM sublayer. The UE shall send a response to the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message(s) after successful completion of the attach procedure.

If the attach procedure was initiated in S101 mode, the lower layers are informed about the successful completion of the procedure.

Upon receiving an ATTACH COMPLETE message, the MME shall stop timer T3450, enter state EMM-REGISTERED and consider the GUTI sent in the ATTACH ACCEPT message as valid.

If the T3448 value IE is present in the received ATTACH ACCEPT message, the UE shall:

-	stop timer T3448 if it is running; and

-	start timer T3448 with the value provided in the T3448 value IE.

If the UE is using EPS services with control plane CIoT EPS optimization, the T3448 value IE is present in the ATTACH ACCEPT message and the value indicates that this timer is either zero or deactivated, the UE shall consider this case as an abnormal case and proceed as if the T3448 value IE is not present.

If the UE has indicated "service gap control supported" in the ATTACH REQUEST message and:

-	the ATTACH ACCEPT message contains the T3447 value IE, then the UE shall store the new T3447 value, erase any previous stored T3447 value if exists and use the new T3447 value with the T3447 timer next time it is started; or

-	the ATTACH ACCEPT message does not contain the T3447 value IE, then the UE shall erase any previous stored T3447 value if exists and stop the T3447 timer if running.

In WB-S1 mode, if the UE has set the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST message, the MME may include a UE radio capability ID IE or a UE radio capability ID deletion indication IE in the ATTACH ACCEPT message.

In WB-S1 mode, if the UE has set the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST message and the ATTACH ACCCEPT message includes:

-	a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested", the UE shall delete any network-assigned UE radio capability IDs associated with the registered PLMN stored at the UE, then the UE shall, after the completion of the ongoing attach procedure, initiate a tracking area updating procedure as specified in subclause 5.5.3; and

-	a UE radio capability ID IE, the UE shall store the UE radio capability ID as specified in annex C.

9.2.1.1.31.3	Test description

9.2.1.1.31.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.31.3.2	Test procedure sequence

Table 9.2.1.1.31.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message on Cell A. | --> | RRCConnectionRequest | - | - |
| 3 | The SS transmits an RRCConnectionSetup message on Cell A. | <-- | RRCConnectionSetup | - | - |
| 4 | The UE transmits an RRCConnectionSetupComplete message with an accompanying ATTACH REQUEST message. | --> | RRCConnectionSetupComplete ATTACH REQUEST | - | - |
| 5-14 | Steps 5-14 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 15 | The SS transmits ATTACH ACCEPT with additional octets included that the UE does not comprehend. | <-- | ATTACH ACCEPT | - | - |
| 16 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 1 | P |

9.2.1.1.31.3.3	Specific message contents

Table 9.2.1.1.31.3.3-1: Message ATTACH ACCEPT (step 15, table 9.2.1.1.31.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | ‘0000 0001 1111 1111’B | The third octet indicates that IMS voice over PS session in S1 mode is supported. The fourth octet is to be interpreted by the UE as additional bits that it does not comprehend. | Rel-8 to Rel-12 |

##### 9.2.1.1.32	Attach / Success / MUSIM

9.2.1.1.32.1	Test Purpose (TP)

(1)

with { MUSIM UE is switched-off with two valid USIMs inserted}

ensure that {

when { UE is powered on }

then { the UE transmits ATTACH REQUEST message  and sets one or more MUSIM feature bits in UE network capability IE to "supported"  individually with cell A and cell G which belong to different PLMNs (i.e. NAS signalling connection release, paging indication for voice services, reject paging request, paging restriction the paging timing collision control) }

}

9.2.1.1.32.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 4.10, 5.4.4.1 and 5.5.1.2.2 and in TS 36.331, clause 5.3.3.4.

[TS 24.301, clause 4.10]

…

A network and a MUSIM UE may support one or more of the MUSIM features (i.e. the NAS signalling connection release, the paging indication for voice services, the reject paging request, the paging restriction and the paging timing collision control).

If MUSIM UE supports one or more MUSIM features, the UE indicates support of one or more MUSIM features during the attach procedure and the normal tracking area updating procedure. If the UE has indicated support of the NAS signalling connection release or the reject paging request or both and the UE supports the paging restriction, the UE indicates support of the paging restriction.

If the UE indicates support of one or more MUSIM features and the network decides to accept one or more MUSIM features, the network indicates the support of one or more MUSIM features during the attach procedure and the normal tracking area updating procedure. The network only indicates the support of the paging restriction together with the support of either NAS signalling connection release or the reject paging request.

The network does not indicate support for any MUSIM feature to the UE during the attach for emergency bearer services.

…

[TS 24.301, clause 5.4.4.1]

…

If the UE is a MUSIM UE, the UE uses a separate IMEI for each USIM the UE operates.

…

[TS 24.301, clause 5.5.1.2.2]

…

If the MUSIM UE supports the NAS signalling connection release, then the UE shall set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the ATTACH REQUEST message otherwise the UE shall not set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the ATTACH REQUEST message.

If the MUSIM UE supports the paging indication for voice services, then the UE shall set the paging indication for voice services bit to "paging indication for voice services supported" in the UE network capability IE of the ATTACH REQUEST message otherwise the UE shall not set the paging indication for voice services bit to "paging indication for voice services supported" in the UE network capability IE of the ATTACH REQUEST message.

If the MUSIM UE supports the reject paging request, then the UE shall set the reject paging request bit to "reject paging request supported" in the UE network capability IE of the ATTACH REQUEST message otherwise the UE shall not set the reject paging request bit to "reject paging request supported" in the UE network capability IE of the ATTACH REQUEST message.

If the MUSIM UE sets:

-	the reject paging request bit to "reject paging request supported";

-	the NAS signalling connection release bit to "NAS signalling connection release supported"; or

-	both of them;

and supports the paging restriction, then the UE shall set the paging restriction bit to "paging restriction supported" in the UE network capability IE of the ATTACH REQUEST message otherwise the UE shall not set the paging restriction bit to "paging restriction supported" in the UE network capability IE of the ATTACH REQUEST message.

If the MUSIM UE supports the paging timing collision control, then the UE shall set the paging timing collision control bit to "paging timing collision control supported" in the UE network capability IE of the ATTACH REQUEST message otherwise the UE shall not set the paging timing collision control bit to "paging timing collision control supported" in the UE network capability IE of the ATTACH REQUEST message.

…

9.2.1.1.32.3	Test description

9.2.1.1.32.3.1	Pre-test conditions

System Simulator:

-	Cell A(home PLMN1) and Cell G(home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

UE:

-	the UE is a MUSIM UE;

-	each USIM of the UE is configured to initiate EPS attach;

-	the UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.32.3.2	Test procedure sequence

Table 9.2.1.1.32.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE perform the attach procedure by executing steps 1 to 31 of the MUSIM UE registration procedure as described in TS 36.508 [18] clause 4.5.2E? | - | - | 1 | - |

9.2.1.1.32.3.3	Specific message contents

Table 9.2.1.1.32.3.3-1: Void

Table 9.2.1.1.32.3.3-2: Void

##### 9.2.1.1.33	Attach / Success / MUSIM / IMSI offset

9.2.1.1.33.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message containing the Requested IMSI offset IE }

ensure that {

when { the UE receives the Negotiated IMSI offset IE in the ATTACH ACCEPT message }

then { the UE receives a paging message in a valid new paging occasion which calculated based on the Negotiated IMSI offset }

}

(2)

with { UE has transmitted ATTACH REQUEST message containing the Requested IMSI offset IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message without Negotiated IMSI offset IE }

then { the UE receives a paging message in a valid old paging occasion which calculated without any IMSI offset }

}

9.2.1.1.33.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.1.2.4 and in TS 23.401, clause 4.3.33.5.

[TS 24.301, clause 5.5.1.2.4]

…

If the MUSIM UE has included a Requested IMSI offset IE in the ATTACH REQUEST message and if the MME supports paging timing collision control, the MME shall include the Negotiated IMSI offset IE in the ATTACH ACCEPT message, and the MME shall set the IMSI offset value to:

-	A value that is different than what the UE has provided, if the MME has a different value; or

-	A value that is same as what the UE has provided, if the MME does not have a different value;

and the MME shall calculate an alternative IMSI value using the IMSI offset value and store it in the UE context as specified in 3GPP TS 23.401 [10]. The alternative IMSI value is used for deriving the paging occasion as specified in 3GPP TS 36.304 [21].

…

If the ATTACH ACCEPT message contains Negotiated IMSI offset IE, the MUSIM UE shall forward the IMSI offset value to lower layers. If the ATTACH ACCEPT message does not contain Negotiated IMSI offset IE, the MUSIM UE shall indicate to lower layers to erase any IMSI offset value, if available.

…

[TS 23.401, clause 4.3.33.5]

To avoid possible paging occasion collision and to enhance the likelihood that paging is received successfully for different USIMs, a Multi-USIM UE may provide, for at least one USIM, a Requested IMSI Offset value that is used for the determination of paging occasions. Upon reception of a Requested IMSI Offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an Accepted IMSI Offset value to the UE in the Attach Accept or Tracking Area Update Accept message to acknowledge it supports the feature and provide the accepted value. The Accepted IMSI Offset Value may be different from the Requested IMSI Offset provided by the UE. The Alternative IMSI value, determined as below, is stored in the UE context in the MME. If the UE does not provide any Requested IMSI Offset value in Attach Request or Tracking Area Request, the MME removes any stored Alternative IMSI value in the UE context. The UE and the network use the Accepted IMSI Offset to determine the paging occasion. The UE and MME use the Accepted IMSI Offset value to calculate the Alternative IMSI value that is determined based on UE's IMSI as follows:

Alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)]

where: the MCC, MNC and MSIN value are the fields of the UE's IMSI as defined in TS 23.003 [9].

The MME uses the Alternative IMSI value to compute the UE Identity Index Value. The MME sends the UE Identity Index Value to RAN in the Paging message (see TS 36.413 [36]) for RAN to derive the paging occasions according to TS 36.304 [34].

The UE uses the Alternative IMSI value for the determination of paging occasions as specified in TS 36.304 [34].

NOTE 1:	It is recommended to avoid excessive signalling load from UE due to this procedure.

NOTE 2:	The MME does not remove Alternative IMSI value if the Tracking Area Update Request is for periodic Tracking Area Update.

9.2.1.1.33.3	Test description

9.2.1.1.33.3.1	Pre-test conditions

System Simulator:

-	Cell A(home PLMN1) and Cell G(home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

UE:

-	The UE is a MUSIM UE;

-	Each USIM of the UE is configured to initiate EPS attach;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.33.3.2	Test procedure sequence

Table 9.2.1.1.33.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Configure the UE to perform ATTACH including a Requested IMSI offset IE. (Note1) | - | - | - | - |
| 2 | The SS configures: - Cell A as the "Serving cell". - Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’. | - | - | - | - |
| 5 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionSetupComplete message with ATTACH REQUEST message including a Requested IMSI offset IE? (Note1) | --> | ATTACH REQUEST | - | - |
| 7-15 | Steps 5-13 of the generic procedure of UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 16 | The SS transmits an ATTACH ACCEPT with the Negotiated IMSI offset IE. | <-- | ATTACH ACCEPT |  |  |
| 17-20b1 | Steps 15-18b1 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 21 | The SS transmits a Paging message in a PO which calculated based on the Negotiated IMSI offset. | - | - | - | - |
| 22-31 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 1 | P |
| 32 | The SS configures: - Cell G as a "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 33 | The UE transmits an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’. | - | - | - | - |
| 34 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 35 | Check: Does the UE transmit an RRCConnectionSetupComplete message with ATTACH REQUEST message including a Requested IMSI offset IE? (Note1) | --> | ATTACH REQUEST | - | - |
| 36-44 | Steps 5-13 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 45 | The SS transmits an ATTACH ACCEPT without the Negotiated IMSI offset IE. | <-- | ATTACH ACCEPT | - | - |
| 46-49b1 | Steps 15-18b1 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 49 | The SS transmits a Paging message in a PO which calculated without the Negotiated IMSI offset. | - | - | - | - |
| 50-59 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 2 | P |
| Note 1: The ATTACH REQUEST message including a Requested IMSI offset IE may be performed by MMI or AT command. |  |  |  |  |  |

9.2.1.1.33.3.3	Specific message contents

Table 9.2.1.1.33.3.3-1: Message ATTACH REQUEST (step 6 and 35, Table 9.2.1.1.33.3.2-1)

| Derivation path: TS 36.508 table 4.5.2E.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested IMSI offset | Any allowed value |  |  |

Table 9.2.1.1.33.3.3-2: Message ATTACH ACCEPT (step 16, Table 9.2.1.1.33.3.2-1)

| Derivation path: TS 36.508 table 4.5.2E.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated IMSI offset | Same as what the Requested IMSI offset IE |  |  |

##### 9.2.1.1.34	eMTC / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location"

9.2.1.1.34.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "PLMN not allowed to operate at the present UE location" }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, UE deletes the GUTI, last visited registered TAI, TAI list and eKSI, UE resets the registration attempt counter, UE enters the state EMM-DEREGISTERED.PLMN-SEARCH and perform a PLMN selection }

}

9.2.1.1.34.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clauses 4.11.2 and 5.5.1.2.5. Unless otherwise stated these are Rel-17 requirements.

[TS 24.301 clause 4.11.2]

The UE may be rejected with EMM cause #78 in ATTACH REJECT message, TRACKING AREA UPDATE REJECT message or DETACH REQUEST message. The EMM cause #78 is applicable for the UE only when the UE is accessing a PLMN using a satellite E-UTRAN access.

For the satellite E-UTRAN access the UE shall store a list of "PLMN not allowed to operate at the present UE location". Each entry in the list consists of:

a)	PLMN identity of the PLMN which sent a message including EMM cause value #78 "PLMN not allowed to operate at the present UE location" via the satellite E-UTRAN access technology;

b)	geographical location, if known by the UE, where the EMM cause value #78 was received over the satellite E-UTRAN access technology; and

c)	if the geographical location exists, a UE implementation specific distance value.

Before storing a new entry in the list, the UE shall delete any existing entry with the same PLMN identity. Upon storing a new entry, the UE starts a timer instance associated with the entry with an implementation specific value that shall not be set to a value smaller than the timer value indicated by the network in the Lower bound timer value IE, if any. If the Lower bound timer value IE was not provided by the network, the value of the timer shall be set based on the UE implementation.

The UE is allowed to attempt to access a PLMN via the satellite E-UTRAN access technology which is part of the list of "PLMNs not allowed to operate at the present UE location" if:

a)	the current UE location is known, a geographical location is stored for the entry of this PLMN, and the distance from location where EMM cause value #78 was received to the current UE location is larger than a UE implementation specific value:

b)	the timer associated with the entry of this PLMN has expired; or

c)	the access is for emergency services (see 3GPP TS 23.122 [5] for further details).

NOTE:	When the UE is accessing network for emergency services, it is up to operator and regulatory policies whether the network needs to determine if the UE is in a location where network is not allowed to operate.

The list shall accommodate three or more entries. The maximum number of entries is an implementation decision. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

Each entry shall be removed if for the entry:

a)	the UE successfully registers via the satellite E-UTRAN access technology to the PLMN stored in the entry; or

b)	the timer instance associated with the entry expires.

The UE may remove an entry in the list, if the UE's current location is known, a geographical is stored for the entry of this PLMN, and the distance from location where EMM cause value #78 was received to the current UE location is larger than the UE implementation specific value.

If the UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and an entry from the list of "PLMNs not allowed to operate at the present UE location" is removed, the UE shall perform PLMN selection according to 3GPP TS 23.122 [6].

When the UE is switched off, the UE shall keep the list of "PLMNs not allowed to operate at the present UE location" in its non-volatile memory together with the SUPI from the USIM. The UE shall delete the list of "PLMNs not allowed to operate at the present UE location" if the USIM is removed.

If the UE is switched off when the timer instance associated with the entry in the list is running, the UE shall behave as follows when the UE is switched on and the USIM in the UE remains the same:

let t1 be the time remaining for timer instance associated with the entry in the list to timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted and considered expired. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

[TS 24.301 clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

…

In NB-S1 mode or WB-S1 mode via satellite E-UTRAN access, if the attach request is from a UE via a satellite E-UTRA cell and the network using the User Location Information provided by the eNodeB (see 3GPP TS 36.413 [23]), is able to determine that the UE is in a location where the network is not allowed to operate, the network shall set the EMM cause value in the ATTACH REJECT message to #78 "PLMN not allowed to operate at the present UE location".

Upon receiving the ATTACH REJECT message, if the message is integrity protected or contains a reject cause other than EMM cause value #25, the UE shall stop timer T3410.

…

Furthermore, the UE shall take the following actions depending on the EMM cause value received in the ATTACH REJECT message.

…

#78	(PLMN not allowed to operate at the present UE location).

This cause value received from a non-satellite E-UTRA cell is considered as an abnormal case and the behaviour of the UE is specified in clause 5.5.1.2.6.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to clause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall reset the registration attempt counter. The UE shall store the PLMN identity and, if it is known, the current geographical location in the list of "PLMNs not allowed to operate at the present UE location", start a corresponding timer instance (see clause 4.11.2), enter state EMM-DEREGISTERED.PLMN-SEARCH and perform a PLMN selection according to 3GPP TS 23.122 [6].

If the UE is operating in single-registration mode, the UE shall in addition handle the 5GMM parameters 5GMM state, 5GS update status and registration attempt counter as specified in 3GPP TS 24.501 [54] for the case when the initial registration procedure performed over 3GPP access is rejected with the 5GMM cause with the same value.

9.2.1.1.34.3	Test description

9.2.1.1.34.3.1	Pre-test conditions

System Simulator:

-	Cell A (PLMN1, HPLMN) and cell N (belongs to TAI-7, visited PLMN, MCC=001 MNC=02) are configured according to Table 6.3.2.2-1 in TS 36.508 [18].

-	System information combination 33 as defined in TS 36.508 [18] clause 4.4.3.1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is previously registered on Cell A and the UE is last authenticated and registered using default message contents according to TS 36.508 [18].

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

Table 9.2.1.1.34.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell N as a "Non-Suitable off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 4-7 | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508 [18]. | - | - | - | - |
| 8 | The SS transmits an ATTACH REJECT message including EMM cause = "PLMN not allowed to operate at the present UE location". | <-- | ATTACH REJECT | - | - |
| 9 | The SS releases the RRC connection. | - | - | - | - |
| - | The SS configures: - Cell A as a "Suitable cell". - Cell N as a "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell N unless explicitly stated otherwise. | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 11-23 | Steps 5 to 17 of the registration procedure described in TS 36.508 [18] subclause 4.5.2.3 are performed on Cell N. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 24 | Make the UE delete the list of PLMNs not allowed to operate at the present UE location. (Note 1) | - | - | - | - |
| Note 1: This could be done by an MMI/AT command. |  |  |  |  |  |

9.2.1.1.34.3.3	Specific message contents

Table 9.2.1.1.34.3.3-1: Message ATTACH REJECT (step 8, Table 9.2.1.1.34.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01001110 | #78 "PLMN not allowed to operate at the present UE location" |  |
| ESM message container | Not present |  |  |

Table 9.2.1.1.34.3.3-2: Message ATTACH REQUEST (step 10, Table 9.2.1.1.34.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 8; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 8. |  |

Table 9.2.1.1.34.3.3-3: SystemInformationBlockType31 on Cell N (preamble and all steps, Table 9.2.1.1.34.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-21 with condition NGSO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 9.2.1.2	Combined attach procedure for EPS services and non-EPS services

##### 9.2.1.2.1	Combined attach procedure / Success / EPS and non-EPS services

9.2.1.2.1.1	Test Purpose (TP)

(1)

with { UE in state EMM-DEREGISTERED and is switched off }

ensure that {

when { UE is powered up or switched on}

then { UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach' }

}

(2)

with { UE in state EMM-REGISTERED-INITIATED}

ensure that {

when { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' }

then { UE sends ATTACH COMPLETE message and enters EMM state EMM-REGISTERED and MM state MM-IDLE }

}

(3)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE enters a new tracking area already included in the TAI list }

then { UE does not send TRACKING AREA UPDATE REQUEST message }

}

(4)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE enters a new tracking area not included in the TAI list }

then { UE sends TRACKING AREA UPDATE REQUEST message with 'EPS update type = combined TA/LA updated '}

}

(5)

with { UE in state EMM-DEREGISTERED and is switched off and has a valid TAI value in 'Last visited registered TAI' IE and a valid GUTI}

ensure that {

when { UE is powered up or switched on}

then { UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach' and with the value in 'Last visited registered TAI' and the valid GUTI}

}

9.2.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.1.3.1, 5.5.1.3.4.1 and 5.5.1.3.4.2.

[TS 24.301 clause5.5.1.3.1]

The combined EPS attach procedure is used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

The combined EPS attach procedure is also used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for EPS services if it is already attached for non-EPS services.

When the UE initiates a combined EPS attach procedure, the UE shall indicate "combined EPS/IMSI attach" in the EPS attach type IE.

The combined EPS attach procedure follows the attach procedure for EPS described in subclause 5.5.1.2.

[TS 24.301 clause5.5.1.3.2]

If the UE is in EMM state EMM-DEREGISTERED, the UE initiates the combined attach procedure by sending an ATTACH REQUEST message to the network, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

[TS 24.301 clause5.5.1.3.4.1]

Depending on the value of the EPS attach result IE received in the ATTACH ACCEPT message, two different cases can be distinguished:

1)	The EPS attach result IE value indicates "combined EPS/IMSI attach": attach for EPS and non-EPS services have been successful.

…

[TS 24.301 clause5.5.1.3.4.2]

The description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

The TMSI reallocation may be part of the combined attach procedure. The TMSI allocated is then included in the ATTACH ACCEPT message, together with the location area identification (LAI). In this case the MME shall start timer T3450 and enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

The UE, receiving an ATTACH ACCEPT message, stores the received location area identification, stops timer T3410, resets the location update attempt counter and sets the update status to U1 UPDATED. If the message contains an IMSI, the UE is not allocated any TMSI, and shall delete any TMSI accordingly. If the message contains a TMSI, the UE shall use this TMSI as the new temporary identity. The UE shall delete its old TMSI and shall store the new TMSI. If neither a TMSI nor an IMSI has been included by the network in the ATTACH ACCEPT message, the old TMSI, if any available, shall be kept. The UE, when receiving the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, shall send an ATTACH COMPLETE message combined with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message to the network after which it shall enter state EMM-REGISTERED and MM state MM-IDLE and set the EPS update status to EU1 UPDATED.

9.2.1.2.1.3	Test description

9.2.1.2.1.3.1	Pre-test conditions

System Simulator:

-	Cell A, cell B and cell C are configured according to Table 6.3.2.2-1 in [18]:

-	Cell A belongs to TAI-1 (home PLMN);

-	Cell B belongs to TAI-2 (home PLMN, another TAC);

-	Cell C belongs to TAI-3 (home PLMN, another TAC).

-	The different cells may not be simultaneously activated  (at most 2 cells are active simultaneously).

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

UE:

- 	The UE is configured to initiate combined EPS/IMSI attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.1.3.2	Test procedure sequence

Table 9.2.1.2.1.3.2-1: Main Behaviour

| St | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S |  | Message |  |  |  |  |  |
| 1 | The SS configures: - Cell A as the “Serving cell”. - Cell B as a “Non-Suitable cell”. - Cell C as a “Non-Suitable Off cell”. |  | - |  | - |  | - |  | - |  |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. |  | - |  | - |  | - |  | - |  |
| 2 | The UE is powered up or switched on. |  |  |  |  |  | - |  | - |  |
| 3 | Check: Does the UE send an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? |  | --> |  | ATTACH REQUEST |  | 1 |  | P |  |
| 4-7a2 | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - |  | - |  | - |  | - |  |  |
| 8 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. |  | <-- |  | ATTACH ACCEPT |  | - |  | - |  |
| 8A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> |  | RRC: RRCConnectionReconfigurationComplete |  | - |  | - |  |
| 9 | Check: Does the UE send ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? |  | --> |  | ATTACH COMPLETE |  | 2 |  | P |  |
| 9A1a1-9A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  |  |  |  |  |  |  |  |  |
| 10-15 | Void |  |  |  |  |  |  |  |  |  |
| 16 | The SS configures: - Cell A as a “Non-Suitable cell”. - Cell B as the “Serving cell”. - Cell C as a “Non-Suitable Off cell”. |  | - |  | - |  | - |  | - |  |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. |  | - |  | - |  | - |  | - |  |
| 17 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message in the next 30 seconds? |  | - |  | - |  | 3 |  | F |  |
| 18-19E | Void |  |  |  |  |  |  |  |  |  |
| 20 | The SS configures : - Cell A as a “Non-Suitable Off cell”. - Cell B as a “Non-Suitable cell”. - Cell C as the “Serving cell”. |  | - |  | - |  | - |  | - |  |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. |  | - |  | - |  | - |  | - |  |
| 21 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? |  | --> |  | TRACKING AREA UPDATE REQUEST |  | 4 |  | P |  |
| 22 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. |  | <-- |  | TRACKING AREA UPDATE ACCEPT |  | - |  | - |  |
| 23 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? |  | --> |  | TRACKING AREA UPDATE COMPLETE |  | - |  | - |  |
| 24 | The SS releases the RRC connection. |  | - |  | - |  | - |  | - |  |
| 25 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. |  | - |  | - |  | - |  | - |  |
| - | EXCEPTION: Step 26 describes behaviour that depends on the UE capability. |  |  |  |  |  |  |  |  |  |
| 26 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. |  | --> |  | DETACH REQUEST |  | - |  | - |  |
| 27 | The UE is brought back to operation or the USIM is inserted. |  | - |  | - |  | - |  | - |  |
| 28 | Check: Does the UE send ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. |  | --> |  | ATTACH REQUEST |  | 5 |  | P |  |
| 29-37 | Steps 5 to 13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  | - |  | - |  | - |  | - |  |
| 38 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. |  | <-- |  | ATTACH ACCEPT |  | - |  | - |  |
| 38A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> |  | RRC: RRCConnectionReconfigurationComplete |  | - |  | - |  |
| 39-41b1 | Steps 16 to 18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - |  | - |  | - |  | - |  |  |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. |  | - |  | - |  | - |  | - |  |

9.2.1.2.1.3.3	Specific message contents

Table 9.2.1.2.1.3.3-1: Message ATTACH REQUEST (step 3, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach type | '0010'B |  | ''combined EPS/IMSI attach'' |  |  |
| Old GUTI or IMSI | GUTI-1 |  |  |  |  |
| Last visited registered TAI | TAI-1 |  |  |  |  |
| Old location area identification | LAI-1 |  |  |  |  |
| TMSI status | Not Present |  |  |  |  |

Table 9.2.1.2.1.3.3-2: Message ATTACH ACCEPT (step 8, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach result | '010'B |  | ''Combined EPS/IMSI attach'' |  |  |
| TAI list |  |  |  |  |  |
| Number of elements | '00001'B |  |  |  |  |
| Type of list | '00'B |  | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |  |
| MCC MNC TAC 1 TAC 2 | PLMN= MCC/MNC TAC 1=1 TAC 2=2 |  | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-1'' ''TAI-2'' |  |  |
| GUTI | GUTI-2 |  |  |  |  |
| LAI | LAI-1 |  |  |  |  |
| MS identity | TMSI-1 |  |  |  |  |

Table 9.2.1.2.1.3.3-3: Void

Table 9.2.1.2.1.3.3-4: Void

Table 9.2.1.2.1.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 21, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '001'B | ''Combined TA/LA updating'' |  |
| Old GUTI | GUTI-2 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not Present |  |  |  |

Table 9.2.1.2.1.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 22, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | '001'B | ''combined TA/LA updated'' |  |
| GUTI | GUTI-3 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-3 |  |  |
| Location area identification | LAI-3 |  |  |
| MS identity | TMSI-3 |  |  |

Table 9.2.1.2.1.3.3-7: Message ATTACH REQUEST (step 28, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach type | '0010'B | ''combined EPS/IMSI attach'' |  |
| Old GUTI or IMSI | GUTI-3 |  |  |
| Last visited registered TAI | TAI-3 |  |  |
| Old location area identification | LAI-3 |  |  |
| TMSI status | Not Present |  |  |

Table 9.2.1.2.1.3.3-8: Message ATTACH ACCEPT (step 38, Table 9.2.1.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-3 |  |  |
| GUTI | GUTI-3 |  |  |

##### 9.2.1.2.1b	Combined attach procedure / Success / SMS only

9.2.1.2.1b.1	Test Purpose (TP)

(1)

with { UE operating in CS/PS mode 1, in state EMM-REGISTERED-INITIATED}

ensure that {

when { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported }

then { UE automatically disables the E-UTRA capability and performs a registration to UTRAN cell or GERAN cell(depending on the UE capability)}

}

(2)

with { UE operating in CS/PS mode 2, in state EMM-REGISTERED and state MM-IDLE}

ensure that {

when { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported and SS sends Paging message with PS domain }

then { UE sends SERVICE REQUEST message }

}

(3)

with { UE operating in CS/PS mode 2 and configured to use IMS voice, in state EMM-REGISTERED and state MM-IDLE}

ensure that {

when { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported and the user initiates an IMS voice call }

then { UE does not initiate an IMS voice call}

(4)

with { UE operating in CS/PS mode 1" }

ensure that {

when { receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported }

then { UE does not set the E-UTRA support bit in the relevant RRC messages}

9.2.1.2.1b.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 4.3.1, 4.5, 5.5.1.3.1, 5.5.1.3.4.1 5.5.1.3.4.2 and 5.6.2.4.

[TS 24.301 clause 4.3.1]

The behaviour of the UE in CS/PS mode 1 of operation, upon failure to access the CS domain or upon reception of a "CS fallback not preferred" or "SMS only" indication, will depend on the availability of voice over IMS. In the present document, "IMS voice not available" refers to one of the following conditions:

-	the UE is not configured to use IMS;

-	the UE is not configured to use IMS voice, i.e. when the voice domain preference for E-UTRAN, as defined in 3GPP TS 24.167 [13B], indicates that voice communication services are allowed to be invoked only over the CS domain;

-	the UE is configured to use IMS voice, but the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are not supported; or

-	the UE is configured to use IMS voice, the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are supported, but the upper layers indicate that the UE is not available for voice calls in the IMS.

[TS 24.301 clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN; and

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability.

NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

The UE shall enable the E-UTRA capability again in the following cases:

-	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation;

-	the UE powers off and powers on again; or

- 	for the PLMN selection purpose.

[TS 24.301 clause5.5.1.3.1]

The combined attach procedure is used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services, or both EPS services and "SMS only".

The combined attach procedure is also used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for EPS services if it is already IMSI attached for non-EPS services.

When the UE initiates a combined attach procedure, the UE shall indicate "combined EPS/IMSI attach" in the EPS attach type IE.

The combined attach procedure follows the attach procedure for EPS described in subclause 5.5.1.2.

[TS 24.301 clause5.5.1.3.2]

If the UE is in EMM state EMM-DEREGISTERED, the UE initiates the combined attach procedure by sending an ATTACH REQUEST message to the network, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

If the UE initiates a combined attach procedure for EPS services and "SMS only", the UE shall indicate "SMS only" in the Additional update type IE.

[TS 24.301 clause5.5.1.3.4.1]

Depending on the value of the EPS attach result IE received in the ATTACH ACCEPT message, the following different cases can be distinguished:

1)	The EPS attach result IE value indicates "combined EPS/IMSI attach": attach for EPS and non-EPS services, or for EPS services and "SMS only" have been successful.

…

[TS 24.301 clause5.5.1.3.4.2]

The description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services or "SMS only" applies.

…

If the UE requested "SMS only" in the Additional update type IE, the network shall indicate "SMS only" in the Additional update result IE.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only" or "CS Fallback not preferred", a UE operating in CS/PS mode 1 with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only", a UE operating in CS/PS mode 2 shall not attempt to use CS fallback for mobile originating services.

….

[TS 24.301 clause5.6.2.4]

The network shall initiate the paging procedure when it receives an incoming mobile terminating SMS to the UE that is IMSI attached for non-EPS services or for "SMS only", and no NAS signalling connection exists.

To initiate the procedure for SMS when no NAS signalling connection exists, the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.413 [23]). The paging message shall include a CN domain indicator set to "PS". The paging procedure is performed according to subclause 5.6.2.2.1. The MME shall not start timer T3413 for this procedure.

9.2.1.2.1b.3	Test description

9.2.1.2.1b.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18].

-	If pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, Cell 5;

-	If pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN then, Cell 24;

-	Cell 24 belongs to LAI-1 and RAI-1(home PLMN)

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	The UE is configured to initiate combined EPS/IMSI attach and to be voice centric (UE operates in CS/PS mode 1). If the UE does not support to be configured voice centric initiate combined EPS/IMSI attach and to be data centric (UE operates in CS/PS mode 2).

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

9.2.1.2.1b.3.2	Test procedure sequence

Table 9.2.1.2.1b.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell A. |  |  |  |  |
| 1 | Set the cell type of cell A to the ''Serving cell''. Set the cell type of cell 5 or cell 24 to the '' Non-Suitable cell ''. | - | - | - | - |
| 2 | The UE is powered up or switched on. |  |  | - | - |
| 2A1 | UE sends RRCConnectionRequest message | - | - | - | - |
| 2A2 | SS sends RRCConnectionSetup message | - | - | - | - |
| 2A3 | Set the cell type of cell 5 or cell 24 to ''Suitable neighbour cell''. | - | - | - | - |
| 3 | UE sends ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach". | --> | ATTACH REQUEST | - | - |
| 4-7Aa2 | Void | - | - | - | - |
| 7B-7H | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 8 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | ATTACH ACCEPT | - | - |
| 9 | Void | - | - | - | - |
| 10 | Void | - | - | - | - |
| - | EXCEPTION: Steps 11a1 to 11b3a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 11a1 | IF the UE is configured in CS/PS mode 1 of operation THEN perform actions specified in Table 9.2.1.2.1b.3.2-2. |  |  |  |  |
| 11a2a1-11a2a4 | Void | - | - | - | - |
| 11a3-11a6 | Void | - | - | - | - |
| 11b0 | ELSE The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 11b1a1-11b3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 11b2 | Check: Does the UE respond to paging on cell A with S-TMSI in GUTI-2 for PS domain?(Generic Procedure TS 36.508 6.4.2.4) | - | - | 2 | P |
| - | EXCEPTION: Steps 11b3a1 to 11b3a2 describe behaviour that depends on the UE capability. |  |  |  |  |
| 11b3 a1 | IF pc_VoLTE THEN an IMS voice call is initiated | - | - | - | - |
| 11b3a2 | Check: Does the UE send RRC CONNECTION REQUEST message? | --> | RRC CONNECTION REQUEST | 3 | F |
| Note 1: Void |  |  |  |  |  |

Table 9.2.1.2.1b.3.2-2: CS/PS mode 1 of operation with “IMS voice not available” behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1c2 depend on UE release. IF UE is Release 8, Release 9 or Release 10steps 1a1 to 1b3 are considered. Steps 1a1 to 1b3 are optional and depend on UE implementation. They shall be executed if the UE sends an ATTACH COMPLETE with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message OR a DETACH REQUEST message during [2] seconds. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met. Note: UE may also locally release the established NAS signalling connection and start 2G/3G search immediately. ELSE IF UE is Release 11 onward, steps 1c1 and 1c2 are executed. | - | - | - | - |
| 1a1 | IF UE sent an ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | ATTACH COMPLETE | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2a2 are optional and depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the condition is met. | - | - | - | - |
| 1a2a1 | IF UE sends a DETACH REQUEST message during [2] seconds. | --> | DETACH REQUEST | - | - |
| 1a2a2 | SS sends a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 1a3 | SS releases the RRC connection | - | - | - | - |
| 1b1 | IF UE sent a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 1b2 | SS sends a DETACH ACCEPT message. Note: It is not explicitly defined what the network should do if the UE did not send ATTACH COMPLETE. | <-- | DETACH ACCEPT | - | - |
| 1b3 | SS releases the RRC connection | - | - | - | - |
| e | EXCEPTION: In parallel to the event described in step 1c1 below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| 1c1 | The UE transmits an ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | ATTACH COMPLETE | - | - |
| 1c2 | SS releases the RRC connection | - | - | - | - |
| 2 | Void | - | - | - | - |
| - | The following messages shall be received on Cell 5 or Cell 24 | - | - | - | - |
| - | EXCEPTION: Steps 3a1 to 3a3 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 3a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN Check: Does the UE send RRC CONNECTION REQUEST on Cell 5? | --> | RRC CONNECTION REQUEST | 4 | P |
| 3a2 | The SS transmits an RRC CONNECTION SETUP message on Cell 5. | <-- | RRC CONNECTION SETUP | - | - |
| 3a3 | Check: Does the UE transmit a RRC CONNECTION SETUP COMPLETE message on Cell 5? | --> | RRC CONNECTION SETUP COMPLETE | 4 | P |
| - | EXCEPTION: Steps 4a1 to 4b5 depend on UE implementation; the "lower case letter" identifies a step sequence that takes place if the condition is met. | - | - | - | - |
| 4a1 | IF UE did not sent DETACH REQUEST in step 1a2a1 or 1b1 AND UE transmits a ROUTING AREA UPDATE REQUEST message during [2] seconds THEN check contents of ROUTING AREA UPDATE REQUEST | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| - | EXCEPTION: Steps 4a1Aa1 to 4a2 Steps 4a1Aa1 to 4a2 A depend on the UE implementation. Step 4a1Aa1 to 4a1A a3 take place if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN |  | - | - | - |
| 4a1Aa1 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. from Cell 24 | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 4a1Aa2 | The UE transmits a AUTHENTICATION AND CIPHERING RESPONSE message on Cell 24 | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 4a1Aa3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 4a2 | Void. | - | - | - | - |
| 4a2A | If px_RATComb_Tested = EUTRA_UTRA the SS transmits a ROUTING AREA UPDATE ACCEPT message with “Follow-on proceed” set to 0 if UE set the “Follow-on request” to 1 in the RAU REQUEST at step 4a1. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 4a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: steps 4a4, 4a4A to 4a4C take place if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA AND UE set the “Follow-on request” to 1 in the RAU REQEST at step 4a1 (see NOTE 1) | - | - | - | - |
| 4a4 | The UE transmits a SERVICE REQUEST message on Cell 5 | --> | SERVICE REQUEST | - | - |
| 4a4A | The SS transmits a SERVICE ACCEPT message on Cell 5 | <-- | SERVICE ACCEPT | - | - |
| 4a4B | The SS transmits RadioBearerSetup message to Setup PS RAB on Cell 5 | <-- | RadioBearerSetup | - | - |
| 4a4C | The UE transmits RadioBearerSetupComplete message on Cell 5 | --> | radioBearerSetupComplete | - | - |
| 4b1 | ELSE check: Does the UE transmit an ATTACH REQUEST message?. | --> | ATTACH REQUEST | 1 | P |
| 4b2 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 4b3 | The UE transmits a AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 4b4 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 4b5 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 4b6 | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) |  |  |  |  |
| 5 | SS releases the RRC connection | - | - | - | - |
| NOTE 1: The SERVICE REQUEST procedure may be triggered by the need for the UE to send an UL IPv6 packet (e.g. for ICMPv6 Router Solicitation) |  |  |  |  |  |

9.2.1.2.1b.3.3	Specific message contents

Table 9.2.1.2.1b.3.3-1: Message ATTACH REQUEST (step 3, Table 9.2.1.2.1b.3.2-1)

| Derivation path: 36.508 table 4.7.2.-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach type | '0010'B |  | ''combined EPS/IMSI attach'' |  |  |
| Old GUTI or IMSI | GUTI-1 |  |  |  |  |
| Last visited registered TAI | TAI-1 |  |  |  |  |
| Old location area identification | LAI-1 |  |  |  |  |
| TMSI status | Not Present |  |  |  |  |

Table 9.2.1.2.1b.3.3-2: Message ATTACH ACCEPT (step 8, Table 9.2.1.2.1b.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach result | '010'B |  | ''Combined EPS/IMSI attach'' |  |  |
| GUTI | GUTI-2 |  |  |  |  |
| EPS network feature support | '0000 0000'B |  | IMS voice over PS session in S1 mode NOT supported |  |  |
| Additional update result | '10'B0000 |  | SMS only |  |  |

Table 9.2.1.2.1b.3.3-3: Message RRC Connection Request (step 3 a1, Table 9.2.1.2.1b.3.2-2)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info | Not present OR value as specified below |  |  |
| > Support of E-UTRA FDD | False | E-UTRA FDD not supported |  |
| > Support of E-UTRA TDD | False | E-UTRA TDD not supported |  |

Table 9.2.1.2.1b.3.3-4: Message RRC Connection Setup Complete (step 3 a3, Table 9.2.1.2.1b.3.2-2)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio access capability |  | The UE shall not indicate support for E-UTRAN |  |
| >UE multi-mode/multi-RAT capability | Not present OR value as specified below |  |  |
| >> Multi-RAT capability |  |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA FDD | Not present |  |  |
| >>> Support of E-UTRA TDD | Not present |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA TDD | Not present |  |  |
| >>> EUTRA Feature Group Indicators | Not checked |  |  |

Table 9.2.1.2.1b.3.3-5: Message ROUTING AREA UPDATE REQUEST (step4 a1, Table 9.2.1.2.1b.3.2-2)

| Derivation path: 36.508 table 4.7B.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |
| MS RA capability value part |  |  |  |
| Access capabilities |  |  |  |
| E-UTRA FDD support : | ‘0’B |  |  |
| E-UTRA TDD support | ‘0’B |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.1.2.1b.3.3-5A: ROUTING AREA UPDATE ACCEPT (step4a1Aa3, Table 9.2.1.2.1b.3.2-2)

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0000000000000000 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

Table 9.2.1.2.1b.3.3-6: Message ATTACH REQUEST (step4 b1, Table 9.2.1.2.1b.3.2-2)

| Derivation path: 36.508 table 4.7B.2-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |
| MS RA capability value part |  |  |  |
| Access capabilities |  |  |  |
| E-UTRA FDD support : | ‘0’B |  |  |
| E-UTRA TDD support | ‘0’B |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

##### 9.2.1.2.1c	Combined attach procedure / Success / EPS and CS Fallback not preferred

9.2.1.2.1c.1	Test Purpose (TP)

(1)

with { UE operating in CS/PS mode 1 in state EMM-REGISTERED-INITIATED}

ensure that {

when { UE receives ATTACH ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred”}

then { UE attempts to register on UTRAN cell and automatically disables its E-UTRAN capability.}

}

(2)

with { UE operating in CS/PS mode 1 in OFF state after having its E-UTRAN capability disabled}

ensure that {

when { UE is switched on }

then { UE re-enables its E-UTRA capability and registers on the E-UTRAN cell}

}

(3)

with { UE operating in CS/PS mode 1 }

ensure that {

when { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE ‘CS Fallback not preferred’ and with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported”}

then { UE does not set the E-UTRA support bit in the relevant NAS and AS messages}

9.2.1.2.1c.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 4.3.1, 4.5 and 5.5.1.3.4.2.

[TS 24.301 clause 4.3.1]

The behaviour of the UE in CS/PS mode 1 of operation, upon failure to access the CS domain or upon reception of a “CS fallback not preferred” or “SMS only” indication, will depend on the availability of voice over IMS. In the present document, “IMS voice not available” refers to one of the following conditions:

-	the UE is not configured to use IMS;

-	the UE is not configured to use IMS voice, i.e. when the voice domain preference for E-UTRAN, as defined in 3GPP TS 24.167 [13B], indicates that voice communication services are allowed to be invoked only over the CS domain;

-	the UE is configured to use IMS voice, but the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are not supported; or

-	the UE is configured to use IMS voice, the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are supported, but the upper layers indicate that the UE is not available for voice calls in the IMS.

[TS 24.301 clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN; and

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability.

NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

The UE shall enable the E-UTRA capability again in the following cases:

- 	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation;

-	the UE powers off and powers on again; or

- 	for the PLMN selection purpose.

[TS 24.301 clause 5.5.1.3.4.2]

If the UE requested "SMS only" in the Additional update type IE, the network shall indicate "SMS only" in the Additional update result IE.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only" or "CS Fallback not preferred", a UE operating in CS/PS mode 1 with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only", a UE operating in CS/PS mode 2 shall not attempt to use CS fallback for mobile originating services.

If the ATTACH ACCEPT message includes the Additional update result IE with value "CS Fallback not preferred", this indicates to a UE operating in CS/PS mode 2 that it is attached for EPS and non-EPS services and that it can use CS fallback.

9.2.1.2.1c.3	Test description

9.2.1.2.1c.3.1	Pre-test conditions

System Simulator:

-	cell A and cell 5 and both belong to PLMN1;

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is configured to be voice centric

- The HPLMN is PLMN1

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.1c.3.2	Test procedure sequence

Table 9.2.1.2.1c.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| - | Cell A is set to “Serving cell” and Cell 5 is set to “Non suitable cell” | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 1 | The UE is powered up or switched on. | - | - | - | - |
| 2 | UE sends RRCConnectionRequest message | - | - | - | - |
| 3 | SS sends RRCConnectionSetup message | - | - | - | - |
| 4 | Set the cell type of cell 5 to ''Suitable neighbour cell''. | - | - | - | - |
| 5 | UE sends ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach". | --> | ATTACH REQUEST | - | - |
| 6 | The SS starts an authentication procedure | <-- | AUTHENTICATION REQUEST | - | - |
| 7 | The UE responds properly to the authentication procedure | --> | AUTHENTICATION RESPONSE | - | - |
| 8 | The SS starts a NAS security mode command procedure to perform NAS integrity protection. | <-- | SECURITY MODE COMMAND | - | - |
| 9 | The UE responds properly to the NAS security mode command procedure | --> | SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | ESM INFORMATION REQUEST | - | - |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | ESM INFORMATION RESPONSE | - | - |
| 10 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | ATTACH ACCEPT | - | - |
| 11 | Perform actions specified in Table 9.2.1.2.1c.3.2-2 | - | - | - | - |
| - | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 12 describes behaviour that depends on the UE capability. | - | - |  |  |
| 12a | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 13 | Set the cell type of cell 5 to ''Non suitable''. |  |  |  |  |
| 14 | The UE is brought back to operation or the USIM is inserted. | - | - |  |  |
| 15 | Check: Does the UE send RRCConnectionRequest on Cell A? | - | - | 2 | P |
| 16-30 | Steps 3-17 of the registration procedure described in TS 36.508 table 4.5.2.3-1 are performed. | - | - | - | - |

Table 9.2.1.2.1c.3.2-2: CS/PS mode 1 of operation with “IMS voice not available” behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 to 1d6 are optional and depend on UE implementation. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met within [2] seconds. | - | - | - | - |
| 1a1 | IF UE is Release 8, 9 or 10 and sent an ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | ATTACH COMPLETE | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2a2 are optional and depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the condition is met.. | - | - | - | - |
| 1a2a1 | IF UE sends a DETACH REQUEST message during [2] seconds. | --> | DETACH REQUEST | - | - |
| 1a2a2 | SS sends a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 1a3 | SS releases the RRC connection | - | - | - | - |
| 1a4 | Check: Does the UE send RRC CONNECTION REQUEST on Cell 5? | - | - | 1,3 | P |
| 1a5 | The SS sends RRC CONNECTION SETUP | - |  | - | - |
| 1a6 | The UE sends RRC CONNECTION SETUP COMPLETE | - |  | 3 | - |
| 1a7 | Perform actions specified in Table 9.2.1.2.1c.3.2-3 |  |  |  |  |
| 1b1 | IF UE is Release 8, 9 or 10 and sent a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 1b2 | SS sends a DETACH ACCEPT message. Note: It is not explicitly defined what the network should do if the UE did not send ATTACH COMPLETE. | <-- | DETACH ACCEPT | - | - |
| 1b3 | SS releases the RRC connection | - | - | - | - |
| 1b4 | Check: Does the UE send RRC CONNECTION REQUEST on Cell 5? | - | - | 1,3 | P |
| 1b5 | The SS sends RRC CONNECTION SETUP | - |  | - | - |
| 1b6 | The UE sends RRC CONNECTION SETUP COMPLETE | - |  | 3 | - |
| 1b7 | Perform actions specified in Table 9.2.1.2.1c.3.2-3 |  |  |  |  |
| 1c1 | IF the UE is Release 8, 9 or 10 and moved straight to. Check: Does the UE send RRC CONNECTION REQUEST on Cell 5? | - | - | 3 | P |
| 1c2 | The SS sends RRC CONNECTION SETUP | - |  | - | - |
| 1c3 | The UE sends RRC CONNECTION SETUP COMPLETE | - |  | 3 | - |
| 1c4 | Perform actions specified in Table 9.2.1.2.1c.3.2-3 |  |  |  |  |
| - | EXCEPTION: In parallel to the event described in step 1d1 below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| 1d1 | IF the UE is Release 11 onwards, it sends an ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | ATTACH COMPLETE | - | - |
| 1d2 | SS releases the RRC connection | - | - | - | - |
| 1d3 | Check: Does the UE send RRC CONNECTION REQUEST on Cell 5? | - | - | 1,3 | P |
| 1d4 | The SS sends RRC CONNECTION SETUP | - |  | - | - |
| 1d5 | The UE sends RRC CONNECTION SETUP COMPLETE | - |  | 3 | - |
| 1d6 | Perform actions specified in Table 9.2.1.2.1c.3.2-3 |  |  |  |  |

Table 9.2.1.2.1c.3.2-3: Registration on UTRAN cell behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b5 depend on UE implementation; the "lower case letter" identifies a step sequence that takes place if the condition is met.. | - | - | - | - |
| 1a1 | IF UE did not sent DETACH REQUEST in step 1a2a1 or 1b1 in Table 9.2.1.2.1c.3.2-2 AND UE transmits a ROUTING AREA UPDATE REQUEST message during [2] seconds THEN check contents of ROUTING AREA UPDATE REQUEST | --> | ROUTING AREA UPDATE REQUEST | 3 | P |
| 1a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 1a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: steps 1a4, 1a4A to 1a4C take place if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA AND UE set the “Follow-on request” to 1 in the RAU REQUEST at step 1a1 (see NOTE 1) | - | - | - | - |
| 1a4 | The UE transmits a SERVICE REQUEST message on Cell 5 | --> | SERVICE REQUEST | - | - |
| 1a4A | The SS transmits a SERVICE ACCEPT message on Cell 5 | <-- | SERVICE ACCEPT | - | - |
| 1a4B | The SS transmits RadioBearerSetup message to Setup PS RAB on Cell 5 | <-- | RadioBearerSetup | - | - |
| 1a4C | The UE transmits RadioBearerSetupComplete message on Cell 5 | --> | radioBearerSetupComplete | - | - |
| 1b1 | ELSE check: Does the UE transmit an ATTACH REQUEST message?. | --> | ATTACH REQUEST | 3 | P |
| 1b2 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 1b3 | The UE transmits a AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 1b4 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 1b5 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 2 | SS releases the RRC connection | - | - | - | - |

NOTE 1:	The SERVICE REQUEST procedure may be triggered by the need for the UE to send an UL IPv6 packet (e.g. for ICMPv6 Router Solicitation).

9.2.1.2.1c.3.3	Specific message contents

Table 9.2.1.2.1c.3.3-1: Message ATTACH ACCEPT (step 10, Table 9.2.1.2.1c.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| ESM message container | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to activate the default bearer | There is no message piggybacked in it. |  |
| EPS network feature support | '0000 0000'B | IMS voice over PS session in S1 mode NOT supported |  |
| Additional update result | ‘01’b | “CS Fallback not preferred” |  |

Table 9.2.1.2.1c.3.3-2: Message RRC Connection Request (steps 1a4, 1b4, 1c1 and 1d3 Table 9.2.1.2.1c.3.2-2)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info |  | Optional IE, but if transmitted it should have the below values |  |
| > Support of E-UTRA FDD | False | E-UTRA FDD not supported |  |
| > Support of E-UTRA TDD | False | E-UTRA TDD not supported |  |

Table 9.2.1.2.1c.3.3-3: Message RRC Connection Setup Complete (steps 1b6 , 1c3 and 1d5 Table 9.2.1.2.1c.3.2-2)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio access capability |  | The UE shall not indicate support for E-UTRAN |  |
| >UE multi-mode/multi-RAT capability |  |  |  |
| >> Multi-RAT capability |  |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA FDD | Not present |  |  |
| >>> Support of E-UTRA TDD | Not present |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA TDD | Not present |  |  |
| >>> EUTRA Feature Group Indicators | Not checked |  |  |

Table 9.2.1.2.1c.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 1a1, Table 9.2.1.2.1c.3.2-3)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | Any Value | The setting of Follow-on request (FOR) will have impact on the setting of 'Update Results in Table 9.2.1.2.1c.3.3-6 |  |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |
| MS RA capability value part |  |  |  |
| Access capabilities |  |  |  |
| E-UTRA FDD support | ‘0’B |  |  |
| E-UTRA TDD support | ‘0’B |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.1.2.1c.3.3-5: Message ATTACH REQUEST (step 1b1, Table 9.2.1.2.1c.3.2-3)

| Derivation path: 24.008 table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |
| MS RA capability value part |  |  |  |
| Access capabilities |  |  |  |
| E-UTRA FDD support | ‘0’B |  |  |
| E-UTRA TDD support | ‘0’B |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |

Table 9.2.1.2.1c.3.3-6: Message ROUTING AREA UPDATE ACCEPT (step 1a2, Table 9.2.1.2.1c.3.2-3)

| Derivation path: 36.508, table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update result | 1 ‘No follow-on proceed’ | If UE set the “Follow-on request” to 0 in the RAU REQUEST Table 9.2.1.2.1c.3.3-4 ( step 1a1 Table 9.2.1.2.1c.3.2-3) |  |
|  | 0 ‘Follow-on proceed’ | If UE set the “Follow-on request” to 1 in the RAU REQUEST Table 9.2.1.2.1c.3.3-4 ( step 1a1 Table 9.2.1.2.1c.3.2-3) |  |

##### 9.2.1.2.1d	Combined attach procedure / Success / EPS and CS Fallback not preferred/data centric UE

9.2.1.2.1d.1	Test Purpose (TP)

(1)

with { UE operating in CS/PS mode 2 in state EMM-REGISTERED-INITIATED}

ensure that {

when { UE receives ATTACH ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred”}

then { UE stays on the E-UTRAN cell}

}

(2)

with { UE operating in CS/PS mode 2}

ensure that {

when receives a TRACKING AREA UPDATE ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred” }

then { The UE is able to initiate an MO-CS fallback call}

}

(3)

with { UE operating in CS/PS mode 2}

ensure that {

when receives a ATTACH ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred” }

then { The UE is able to receive MT CS fallback call}

}

9.2.1.2.1d.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 4.3.1, 4.5 and 5.5.1.3.4.2.

[TS 24.301 clause 4.3.1]

The behaviour of the UE in CS/PS mode 1 of operation, upon failure to access the CS domain or upon reception of a "CS fallback not preferred" or "SMS only" indication, will depend on the availability of voice over IMS. In the present document, "IMS voice not available" refers to one of the following conditions:

-	the UE is not configured to use IMS;

-	the UE is not configured to use IMS voice, i.e. when the voice domain preference for E-UTRAN, as defined in 3GPP TS 24.167 [13B], indicates that voice communication services are allowed to be invoked only over the CS domain;

-	the UE is configured to use IMS voice, but the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are not supported; or

-	the UE is configured to use IMS voice, the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are supported, but the upper layers indicate that the UE is not available for voice calls in the IMS.

[TS 24.301 clause 4.5]

The UE shall enable the E-UTRA capability again in the following cases:

- 	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation;

-	the UE powers off and powers on again; or

- 	for the PLMN selection purpose.

[TS 24.301 clause 5.5.1.3.4.2]

If the UE requested "SMS only" in the Additional update type IE, the network shall indicate "SMS only" in the Additional update result IE.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only" or "CS Fallback not preferred", a UE operating in CS/PS mode 1 with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

If the ATTACH ACCEPT message includes the Additional update result IE with value "SMS only", a UE operating in CS/PS mode 2 shall not attempt to use CS fallback for mobile originating services.

If the ATTACH ACCEPT message includes the Additional update result IE with value "CS Fallback not preferred", this indicates to a UE operating in CS/PS mode 2 that it is attached for EPS and non-EPS services and that it can use CS fallback.

9.2.1.2.1d.3	Test description

9.2.1.2.1d.3.1	Pre-test conditions

System Simulator:

-	cell A and cell 5

UE:

-	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is configured to be data centric

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Table 9.2.1.2.1d.3.1–1: Void

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.1d.3.2	Test procedure sequence

Table 9.2.1.2.1d.3.2-1: Void

Table 9.2.1.2.1d.3.2-2: Void

Table 9.2.1.2.1d.3.2-3: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. Cell A is configured as the Serving Cell and Cell 5 is configured as a Non-Suitable Cell | - | - | - | - |
| 1 | The UE is powered up or switched on. |  |  | - | - |
| 2-16 | Steps 2-16 of the registration procedure described in TS 36.508 table 4.5.2.3-1 are performed. |  |  | - | - |
| 16A | Cell 5 is configured as a Suitable Neighbour Cell | - | - | - | - |
| 17 | Check: Does the UE try to access the UTRAN cell (cell 5) in the next 90s? |  |  | 1 | F |
| 18 | The SS transmits a Paging message to the UE on Cell A using S-TMSI with CN domain indicator set to “CS” | - | - | - | - |
| - | EXCEPTION: Step 19a1 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 19a1 | IF the UE needs to request upper layer input before accepting the CS fallback, the incoming CS call is accepted at the UE through MMI or AT command. | - | - | - | - |
| 20 | The UE transmits an RRCConnectionRequest message on Cell A. | - | - | - | - |
| 21 | The SS transmits an RRCConnectionSetup message on Cell A. | - | - | - | - |
| 22 | Check: Does the UE transmit an RRCConnectionSetupComplete message containing an EXTENDED SERVICE REQUEST? | --> | EXTENDED SERVICE REQUEST | 3 | P |
| 23 | The SS transmits an RRCConnectionRelease message on Cell A with IE redirectionInformation including UTRA-CarrierFreq of Cell 5. | - | - | - | - |
| 23A | Cell A is configured as the Non-Suitable Cell | - | - | - | - |
| 24-42 | Steps 1-19 of the procedure described in TS 36.508 table 6.4.3.7.1-1 TS 36.508. A CS call is set up in the UTRAN cell. | - |  | - | - |
| 42A-42F | The CS call is terminated. CC Disconnect procedure described in TS 36.5.23-3 table 10.1.3.1-1 | - | - | - | - |
| 42G | Cell A is configured as the Serving Cell | - | - | - | - |
| 43-48 | Steps 2-7 of the tracking area updating procedure on cell A as described in TS 36.508 table 4.5A.2.1-1 are performed |  |  |  |  |
| - | A CS call is initiated. | - | - | - | - |
| 49 | The UE transmits an RRCConnectionRequest message on Cell A. | - | - | - | - |
| 50 | The SS transmits an RRCConnectionSetup message on Cell A. | - | - | - | - |
| 51 | Check: Does the UE transmit an RRCConnectionSetupComplete message containing an EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 2 | P |
| 52 | The SS transmits an RRCConnectionRelease message on Cell A with IE redirectionInformation including UTRA-CarrierFreq of Cell 5. | - | - | - | - |
| 53-71 | Steps 1-19 of the procedure described in TS 36.508 table 6.4.3.7.2-1. A CS call is set up in the UTRAN cell. | - | - | - | - |

9.2.1.2.1d.3.3	Specific message contents

Table 9.2.1.2.1d.3.3-1: Message ATTACH ACCEPT (step 14, Table 9.2.1.2.1d.3.2-3)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| ESM message container | Empty | There is no message piggybacked in it. |  |
| EPS network feature support | '0000 0000'B | IMS voice over PS session in S1 mode NOT supported |  |
| Additional update result | ‘01’b | “CS Fallback not preferred” |  |

Table 9.2.1.2.1d.3.3-2: Message Paging (step 18, Table 9.2.1.2.1d.3.2-3)

| Derivation Path: 36.508 clause 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| pagingRecordList SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { |  | 1 entry |  |  |
| ue-Identity[1] CHOICE { |  |  |  |  |
| s-TMSI |  | Set to the value of the S-TMSI of the UE |  |  |
| } |  |  |  |  |
| cn-Domain[1] |  | cs |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.1.2.1d.3.3-3: RRCConnectionRelease (steps 23 and 44 Table 9.2.1.2.1d.3.2-3)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.2.1d.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 45, Table 9.2.1.2.1d.3.2-3)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | “001”B | “Combined TA/LA updating'” |  |
| EPS network feature support | '0000 0000'B | IMS voice over PS session in S1 mode NOT supported |  |
| Additional update result | ‘01’b | “CS Fallback not preferred” |  |

Table 9.2.1.2.1d.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 44, Table 9.2.1.2.1d.3.2-3)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | "Combined TA/LA updating with IMSI attach” |  |
| Old P-TMSI Signature | Any Value |  |  |
| Additional GUTI | Any Value |  |  |
| Nonce | Any Value |  |  |

##### 9.2.1.2.2	Combined attach procedure / Success / EPS services only / IMSI unknown in HSS

9.2.1.2.2.1	Test Purpose (TP)

(1)

with { UE in state EMM-DEREGISTERED and is switched off }

ensure that {

when { the UE is powered up or switched on }

then { the UE transmits an ATTACH REQUEST message with the EPS attach type set to "combined EPS/IMSI attach" and enters EMM-REGISTERED-INITIATED state }

}

(2)

with { UE in state EMM-REGISTERED-INITIATED }

ensure that {

when { the UE receives an ATTACH ACCEPT message with EPS attach result set to "EPS only" and EMM cause set to "IMSI unknown in HSS" }

then { the UE transmits an ATTACH COMPLETE message, the UE sets the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number and enters EMM-REGISTERED.NORMAL-SERVICE state and MM IDLE state. The USIM shall be considered as invalid for non-EPS services until switching off or the UICC containing the USIM is removed }

}

(3)

with { UE in E-UTRA EMM-REGISTERED.NORMAL-SERVICE state and MM IDLE state and USIM is invalidated by network for non-EPS services }

ensure that {

when { SS sends DETACH REQUEST message with the Detach type IE "re-attach required" }

then { the UE sends DETACH ACCEPT message, and then the UE sends ATTACH REQUEST message with EPS attach type set to 'EPS attach', including the value in 'Last visited registered TAI' and the valid GUTI and initiates an attach procedure }

}

(4)

with { UE in EMM-REGISTERED.NORMAL-SERVICE state and MM IDLE state and USIM is invalidated by network for non-EPS services }

ensure that {

when { the UE is switch off and then powered up or switched on }

then { the UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach' }

}

(5)

with { UE in EMM-REGISTERED-INITIATED state }

ensure that {

when { the UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' }

then { UE sends ATTACH COMPLETE message and enters EMM-REGISTERED.NORMAL-SERVICE state and MM IDLE state }

}

9.2.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 4.3, 5.5.1.2.2, 5.5.1.2.4, 5.5.1.3.1, 5.5.1.3.2, 5.5.1.3.4.1, 5.5.1.3.4.2, and 5.5.1.3.4.3.

[TS 24.301 clause 4.3]

A UE attached for EPS services may operate in one of the following operation modes:

...

-	CS/PS mode 1 of operation: the UE registers to both EPS and non-EPS services, and UE's usage setting is "voice centric"; and

-	CS/PS mode 2 of operation: the UE registers to both EPS and non-EPS services, and UE's usage setting is "data centric" or the UE has no CS voice capability.

A UE configured to use CS fallback, shall operate in CS/PS mode 1 or CS/PS mode 2. Such UE may also be configured to use IMS, in which case the voice domain preference as defined in 3GPP TS 24.167 [13B] shall be used for the selection of the domain for originating voice communication services.

...

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

…

-	The UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

...

[TS 24.301, clause 5.5.1.2.4]

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450. The MME shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

...

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI, or the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

...

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

...

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

Additionally, the UE shall reset the attach attempt counter and tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED.

…

Upon receiving an ATTACH COMPLETE message, the MME shall stop timer T3450, enter state EMM-REGISTERED and consider the GUTI sent in the ATTACH ACCEPT message as valid.

[TS 24.301, clause 5.5.1.3.1]

The combined attach procedure is used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

The combined attach procedure is also used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for EPS services if it is already IMSI attached for non-EPS services.

When the UE initiates a combined attach procedure, the UE shall indicate "combined EPS/IMSI attach" in the EPS attach type IE.

The combined attach procedure follows the attach procedure for EPS described in subclause 5.5.1.2.

[TS 24.301, clause 5.5.1.3.2]

If the UE is in EMM state EMM-DEREGISTERED, the UE initiates the combined attach procedure by sending an ATTACH REQUEST message to the network, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

[TS 24.301, clause 5.5.1.3.4.1]

Depending on the value of the EPS attach result IE received in the ATTACH ACCEPT message, two different cases can be distinguished:

...

2)	The EPS attach result IE value indicates "EPS only": attach for EPS services has been successful but attach for non-EPS services has failed.

[TS 24.301, clause 5.5.1.3.4.2]

The description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

...

[TS 24.301, clause 5.5.1.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

The UE receiving the ATTACH ACCEPT message takes one of the following actions depending on the EMM cause value:

#2		(IMSI unknown in HSS)

The UE shall stop T3410 if still running and shall reset the tracking area updating attempt counter. The UE shall set the update status to U3 ROAMING NOT ALLOWED and shall delete any TMSI, LAI and ciphering key sequence number. The UE shall enter state EMM-REGISTERED.NORMAL-SERVICE. The new MM state is MM IDLE. The USIM shall be considered as invalid for non-EPS services until switching off or the UICC containing the USIM is removed.

...

Other EMM cause values and the case that no EMM cause IE was received are considered as abnormal cases. The combined attach procedure shall be considered as failed for EPS and non-EPS services. The behaviour of the UE in those cases is specified in subclause 5.5.1.3.6.

[TS 24.301, clause 5.5.2.3.2]

When receiving the DETACH REQUEST message and the detach type indicates "re-attach required", the UE shall deactivate the EPS bearer context(s) including the default EPS bearer context locally without peer-to-peer signalling between the UE and the MME. The UE shall then send a DETACH ACCEPT message to the network and enter state EMM-DEREGISTERED. Furthermore, the UE shall, after the completion of the detach procedure, and the existing NAS signalling connection has been released, initiate an attach or combined attach or combined attach procedure

NOTE 1:	When detach type indicates "re-attach required", user interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

A UE which receives a DETACH REQUEST message with detach type indicating "re-attach required" or "re-attach not required" and no EMM cause IE, is detached only for EPS services.

…

If the detach type indicates "IMSI detach" or "re-attach required", then the UE shall ignore the EMM cause IE if received.

9.2.1.2.2.3	Test description

9.2.1.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

- 	The UE is configured to initiate combined EPS/IMSI attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.2.3.2	Test procedure sequence

Table 9.2.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is powered up or switched on. | - | - | - | - |
| 2 | The UE transmits ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN with EPS attach type set to "combined EPS/IMSI attach" | --> | ATTACH REQUEST | 1 | P |
| 3-7D | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 7E | The SS transmits ATTACH ACCEPT message with EPS Attach result IE set to “EPS only” and EMM cause set to "IMSI unknown in HSS", including the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message as specified. | <-- | ATTACH ACCEPT | - | - |
| 7F | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the event described in step 8 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. s | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 8 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified? | --> | ATTACH COMPLETE | 2 | P |
| 8A1a1-8A1c1 | Steps 16a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 8A2-12 | Void | - | - | - | - |
| 13 | The SS sends DETACH REQUEST message with Detach Type set to "re-attach required" | <-- | DETACH REQUEST | - | - |
| 14 | Check: Does the UE send DETACH ACCEPT message? | --> | DETACH ACCEPT | 3 | P |
| 15 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 15a describes a behaviour which depends on the UE capability | - | - | - | - |
| 15a | IF NOT pc_Automatic_Re_Attach, the user initiates an Attach by MMI or by AT command. | - | - | - | - |
| 16 | Check: Does the UE send ATTACH REQUEST message with EPS attach type set to “EPS attach”, including the PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 3 | P |
| 16Aa1-16Aa2 | Void | - | - | - | - |
| 16Ba1-16D | Steps 9a1-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 17 | The SS sends ATTACH ACCEPT message including the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 15 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 16 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 18 | Check: Does the UE send ATTACH COMPLETE message? | --> | ATTACH COMPLETE | 3 | P |
| 18A1a1-18A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 19 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 19a describes behaviour that depends on the UE capability. |  |  |  |  |
| 19a | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST |  |  |
| 20 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 21 | Check: Does the UE transmit ATTACH REQUEST message with EPS attach type set to "combined EPS/IMSI attach", including PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 22 | The SS transmits ATTACH ACCEPT message with EPS Attach result IE set to “combined EPS/IMSI attach” , including the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 23 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 16 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 23 | Check: Does the UE transmit an ATTACH COMPLETE message including the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 5 | P |
| 24a1-26b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

NOTE:	It is assumed in the test procedure sequence that the UE initially has a valid GUTI, hence it is included in ATTACH REQUEST message in step 2. However, it is not important for the test procedure sequence.

9.2.1.2.2.3.3	Specific message contents

Table 9.2.1.2.2.3.3-1: Message ATTACH REQUEST (step 2, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 |  |  |
| EPS attach type | 010 | “combined EPS/IMSI attach” |  |
| ESM message container | PDN CONNECTIVITY REQUEST message |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.2.3.3-2: Message ATTACH ACCEPT (step 7, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" The SS accepts Combined attach for EPS services only. |  |
| ESM message container | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |  |
| GUTI | GUTI-2 | The SS assigns a new GUTI |  |
| LAI | Not present |  |  |
| MS identity | Not Present | No TMSI is assigned |  |
| EMM cause | 00000010 | #2 “IMSI unknown in HSS” |  |

Table 9.2.1.2.2.3.3-3: Message DETACH REQUEST (step 13, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | 001 | "re-attach required" |  |

Table 9.2.1.2.2.3.3-4: Message ATTACH REQUEST (step 16, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-2 |  |  |
| EPS attach type | 001 | “EPS attach” |  |
| ESM message container | PDN CONNECTIVITY REQUEST message |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.2.3.3-5: Message ATTACH ACCEPT (step 17, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | 001 | '' EPS only '' |  |
| ESM message container | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |  |
| GUTI | Not present | The SS doesn’t assign a new GUTI |  |

Table 9.2.1.2.2.3.3-6: Message ATTACH REQUEST (step 21, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-2 |  |  |
| EPS attach type | 010 | “combined EPS/IMSI attach” |  |
| ESM message container | PDN CONNECTIVITY REQUEST message |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | 0 | no valid TMSI available |  |

Table 9.2.1.2.2.3.3-7: Message ATTACH ACCEPT (step 22, Table 9.2.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | 010 | ''Combined EPS/IMSI attach'' |  |
| ESM message container | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |  |
| GUTI | Not present | The SS doesn’t assign a new GUTI |  |

##### 9.2.1.2.3	Successful combined attach procedure / EPS service only / MSC temporarily not reachable

9.2.1.2.3.1	Test Purpose (TP)

(1)

with { the UE has sent a combined ATTACH REQUEST message }

ensure that {

when { the UE receives an ATTACH ACCEPT message with EPS attach result set to "EPS only" and EMM reject cause set to "MSC temporarily not reachable" or "Network failure" and including an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with IE EPS Bearer Identity matching the ATTACH REQUEST message and including a PDN address, an APN and an uplink TFT }

then { UE transmits an ATTACH COMPLETE message, containing the EPS bearer identity, together with ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message and the UE resets the attach attempt counter and the UE increments tracking area updating attempt counter and starts timer T3411 and enters EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM state }

}

(2)

with { the UE is in E-UTRA EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM state }

ensure that {

when { timer T3411 expires or timer T3402 expires }

then { the UE initiates a combined tracking area update procedure indicating "combined TA/LA updating with IMSI attach" }

}

(3)

with { the UE is in E-UTRA EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM state }

ensure that {

when { UE receives a TRACKING AREA UPDATE ACCEPT with EPS update result set to "EPS only" and EMM reject cause set to "MSC temporarily not reachable" or "Network failure" }

then { UE increments tracking area updating attempt counter unless it was already set to 5 and starts timer T3411 if tracking area updating attempt counter is less than 5 or starts timer T3402 if tracking area updating attempt counter is equal to 5 }

}

9.2.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.2.4, 5.5.1.3.1, 5.5.1.3.2, 5.5.1.3.4.1, 5.5.1.3.4.2, 5.5.1.3.4.3, and 5.5.3.3.4.3.

[TS 24.301, clause 5.5.1.2.4]

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450. The MME shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

If the attach request is accepted by the network, the MME shall delete the stored UE radio capability information, if any.

If the UE has included the UE network capability IE or the MS network capability IE or both in the ATTACH REQUEST message, the MME shall store all octets received from the UE, up to the maximum length defined for the respective information element.

NOTE:	This information is forwarded to the new MME during inter-MME handover or to the new SGSN during inter-system handover to A/Gb mode or Iu mode.

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI, or the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

…

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

…

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

Additionally, the UE shall reset the attach attempt counter and tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED.

…

Upon receiving an ATTACH COMPLETE message, the MME shall stop timer T3450, enter state EMM-REGISTERED and consider the GUTI sent in the ATTACH ACCEPT message as valid.

[TS 24.301, clause 5.5.1.3.1]

The combined attach procedure is used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

The combined attach procedure is also used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for EPS services if it is already IMSI attached for non-EPS services.

When the UE initiates a combined attach procedure, the UE shall indicate "combined EPS/IMSI attach" in the EPS attach type IE.

The combined attach procedure follows the attach procedure for EPS described in subclause 5.5.1.2.

[TS 24.301, clause 5.5.1.3.2]

If the UE is in EMM state EMM-DEREGISTERED, the UE initiates the combined attach procedure by sending an ATTACH REQUEST message to the network, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

[TS 24.301, clause 5.5.1.3.4.1]

Depending on the value of the EPS attach result IE received in the ATTACH ACCEPT message, two different cases can be distinguished:

1)	The EPS attach result IE value indicates "combined EPS/IMSI attach": attach for EPS and non-EPS services have been successful.

2)	The EPS attach result IE value indicates "EPS only": attach for EPS services has been successful but attach for non-EPS services has failed.

[TS 24.301, clause 5.5.1.3.4.2]

The description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

…

[TS 24.301, clause 5.5.1.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

The UE receiving the ATTACH ACCEPT message takes one of the following actions depending on the EMM cause value:

…

#16	(MSC temporarily not reachable);

#17	(Network failure); or

The UE shall stop timer T3410 if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5:

-	the UE shall start timer T3411, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3411 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered.

If the tracking area updating attempt counter is equal to 5:

-	a UE operating in CS/PS mode 2 of operation shall start timer T3402, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3402 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered;

-	a UE operating in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures. The UE shall disable the E-UTRA capability (see subclause 4.5).

NOTE 1:	It is up to the UE implementation when to enable E-UTRAN radio access technology selection.

…

Other EMM cause values and the case that no EMM cause IE was received are considered as abnormal cases. The combined attach procedure shall be considered as failed for EPS and non-EPS services. The behaviour of the UE in those cases is specified in subclause 5.5.1.3.6.

[TS 24.301, clause 5.5.3.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for tracking area for EPS services as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location updating for non-EPS services applies.

The UE receiving the TRACKING AREA UPDATE ACCEPT message takes one of the following actions depending on the EMM cause value:

…

#16	(MSC temporarily not reachable);

#17	(Network failure); or

The UE shall stop timer T3430 if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5:

-	the UE shall start timer T3411, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3411 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again.

If the tracking area updating attempt counter is equal to 5:

-	a UE operating in CS/PS mode 2 of operation shall start timer T3402, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3402 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again;

-	a UE operating in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures. The UE shall disable the E-UTRA capability (see subclause 4.5).

NOTE 1:	It is up to the UE implementation when to enable E-UTRAN radio access technology selection.

…

9.2.1.2.3.3	Test description

9.2.1.2.3.3.1	Pre-test conditions

System Simulator:

-	cell A.

NOTE:	T3402 is set to default (12 min.).

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.3.3.2	Test procedure sequence

The sequence is executed for execution counter k = 1, 2.

Table 9.2.1.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits a combined ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified. | --> | ATTACH REQUEST | - | - |
| 3-6Aa2 | Void | - | - | - | - |
| 6B-6H | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 7 | The SS transmits an ATTACH ACCEPT message with EPS attach result indicating "EPS only" and EMM cause indicating according to specific message contents and with IE EPS Bearer Identity set to default EPS bearer context. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. Note 1: SS allocates a PDN address of a PDN type which is compliant with from the PDN type requested by the UE. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 8 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH COMPLETE message including a ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified? | --> | ATTACH COMPLETE | 1 | P |
| - | EXCEPTION: Steps 8a1 to 8c1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place when the UE is configured in a certain way. | - | - |  |  |
| 8a1-8c1 | Steps 16a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | EXCEPTION: Steps 8Aa1 to 8Ab5 describe behaviour that depends on UE capabilities/configuration and test requirements; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. |  |  |  |  |
| 8Aa1-8Aa3 | IF MULTI_PDN AND NOT pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in steps 10-12 of TS 36.508 subclause 4.5A.18 | - | - | - | - |
| 8Ab1 | IF MULTI_PDN AND pc_UE_supports_user_initiated_PDN_disconnect THEN Cause the UE to request disconnection from the additional PDN(Note 1) | - | - | - | - |
| 8Ab2-8Ab5 | IF MULTI_PDN AND pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in steps 9-12 of TS 36.508 subclause 4.5A.17 | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Check1: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with "combined TA/LA updating with IMSI attach"? Check2: Is the time between the previous ATTACH ACCEPT and TRACKING AREA UPDATE REQUEST equal to T3411? Note: Tracking area updating attempt counter=1. | --> | TRACKING AREA UPDATE REQUEST | 1,2 | P |
| 11 | SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "TA updated" and EMM cause indicating according to specific message contents. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 11A | The SS releases the RRC connection. Note: Tracking area updating attempt counter=2 | - | - | - | - |
| 12 | Check1: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with "combined TA/LA updating with IMSI attach"? Check2: Is the time between the previous TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE REQUEST equal to T3411? | --> | TRACKING AREA UPDATE REQUEST | 2,3 | P |
| 13 | SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "TA updated" and EMM cause indicating according to specific message contents. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13A | The SS releases the RRC connection. Note: Tracking area updating attempt counter=3 | - | - | - | - |
| 14 | Check1: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with "combined TA/LA updating with IMSI attach"? Check2: Is the time between the previous TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE REQUEST equal to T3411? | --> | TRACKING AREA UPDATE REQUEST | 2,3 | P |
| 15 | SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "TA updated" and EMM cause indicating according to specific message contents | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15A | The SS releases the RRC connection. Note: Tracking area updating attempt counter=4 | - | - | - | - |
| 16 | Check1: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with "combined TA/LA updating with IMSI attach"? Check2: Is the time between the previous TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE REQUEST equal to T3411? | --> | TRACKING AREA UPDATE REQUEST | 2,3 | P |
| - | EXCEPTION: Steps 17a1 to 17b2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place according to UE mode of operation | - | - | - | - |
| 17 a1 | IF the UE is configured to operate in CS/PS mode 2, then SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "TA updated" and EMM cause indicating according to specific message contents. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17a2 | The SS releases the RRC connection. Note: Tracking area updating attempt counter=5 | - | - | - | - |
| 17a3 | Check1: Does the UE send TRACKING AREA UPDATE REQUEST message with "combined TA/LA updating with IMSI attach"? Check2: Is the time between the previous TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE REQUEST equal to T3402? Note: Tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE REQUEST | 2,3 | P |
| 17a4 | SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "combined TA/LA updated " according to default message contents. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17a5 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 17a6 | The SS releases the RRC connection. | - | - | - | - |
| 17a7 | Check: Does the result of test procedure in 36.508 clause 6.4.2.4 indicate that the UE answer to paging with S-TMSI-1 (associated with GUTI-1) for PS domain? | - | - | 3 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| 17b1 | ELSE SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating "combined TA/LA updated". | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17b2 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. Note: Tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| 18-20 | Void | - | - | - | - |
| NOTE 1: The request to disconnect from a PDN may be performed by MMI or AT command; in any case the EPS bearer identity of the Default EPS Bearer of the PDN to be released needs to be handed over to the UE. |  |  |  |  |  |

9.2.1.2.3.3.3	Specific message contents

Table 9.2.1.2.3.3.3-1: Message ATTACH REQUEST (step 2, Table 9.2.1.2.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach type | 010 | "combined EPS/IMSI attach" |  |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.3.3.3-2: Message ATTACH ACCEPT (step 7, Table 9.2.1.2.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" |  |
| GUTI | Not present |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010000 for k=1 or 00010001 for k=2 | #16 (MSC temporarily not reachable) for k=1 #17 (Network failure) for k=2 |  |

Table 9.2.1.2.3.3.3-3: Message TRACKING AREA UPDATE REQUEST (steps 10-12-14-16-17a3, Table 9.2.1.2.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type | 010 | "combined TA/LA updating with IMSI attach" |  |
| Old GUTI | GUTI-1 |  |  |
| Old P-TMSI signature | Not present | This IE is included when the UE holds a valid P-TMSI signature. |  |
| Additional GUTI | Not present | TIN = 'GUTI' |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.3.3.3-4: Message TRACKING AREA UPDATE ACCEPT (steps 11-13-15, Table 9.2.1.2.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | 000 | "TA updated" SS accepts Combined TAU for EPS services only. |  |
| GUTI | Not present |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010000 for k=1 or 00010001 for k=2 | #16 (MSC temporarily not reachable) for k=1 #17 (Network failure) for k=2 |  |

Table 9.2.1.2.3.3.3-5: Message TRACKING AREA UPDATE ACCEPT (steps 17a1, Table 9.2.1.2.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | 000 | "TA updated" SS accepts Combined TAU for EPS services only. |  |
| GUTI | Not present |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010000 for k=1 or 00010001 for k=2 | #16 (MSC temporarily not reachable) for k=1 #17 (Network failure) for k=2 |  |
| T3402 | ‘000 01111’B | 30 seconds |  |

##### 9.2.1.2.4	Successful combined attach procedure / EPS service only / CS domain not available

9.2.1.2.4.1	Test Purpose (TP)

(1)

with { UE is switched-off }

ensure that {

when { UE is powered on and a valid GUTI is available }

then { the UE transmits an ATTACH REQUEST message with the EPS attach type set to "combined EPS/IMSI attach", including GUTI, last visited registered TAI and a PDN CONNECTIVITY REQUEST message with the request type set to "initial attach" }

}

(2)

with { UE has sent a combined ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH ACCEPT message including EPS attach result set to "EPS only" and EMM reject cause set to "CS domain not available" and including an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with IE EPS Bearer Identity matching the PDN CONNECTIVITY REQUEST message }

then { UE transmits ATTACH COMPLETE message, containing the EPS bearer identity, including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message and sets the update status to U2 NOT UPDATED and enters EMM-REGISTERED state }

}

(3)

with { The UE received an ATTACH ACCEPT message with EPS attach result set to "EPS only" and EMM cause set to "CS domain not available" }

ensure that {

when { UE enters a TA where it is not registered and which belongs to the PLMN where the UE received the EMM cause "CS domain not available" }

then { UE initiates a normal tracking area update procedure indicating "TA updating" without valid LAI, TMSI, GSM ciphering key, UMTS integrity key, UMTS ciphering key or ciphering key sequence number }

}

(4)

with { The UE received an ATTACH ACCEPT message with EPS attach result set to "EPS only" and EMM cause set to "CS domain not available" }

ensure that {

when { UE is powered up or switched on }

then { UE initiates a combined attach procedure indicating "combined EPS/IMSI attach" }

}

9.2.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.1.2.4, 5.5.1.3.1, 5.5.1.3.2, 5.5.1.3.4.1, 5.5.1.3.4.2 and 5.5.1.3.4.3, and TS 24.008, clause 4.1.2.2. Unless otherwise stated these are Rel-8 requirements.

[TS 24.301, clause 5.5.1.2.4]

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450. The MME shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

If the attach request is accepted by the network, the MME shall delete the stored UE radio capability information, if any.

If the UE has included the UE network capability IE or the MS network capability IE or both in the ATTACH REQUEST message, the MME shall store all octets received from the UE, up to the maximum length defined for the respective information element.

NOTE:	This information is forwarded to the new MME during inter-MME handover or to the new SGSN during inter-system handover to A/Gb mode or Iu mode.

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI, or the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

...

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

...

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

Additionally, the UE shall reset the attach attempt counter and tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED.

…

Upon receiving an ATTACH COMPLETE message, the MME shall stop timer T3450, enter state EMM-REGISTERED and consider the GUTI sent in the ATTACH ACCEPT message as valid.

[TS 24.301, clause 5.5.1.3.1]

The combined attach procedure is used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services.

The combined attach procedure is also used by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for EPS services if it is already IMSI attached for non-EPS services.

When the UE initiates a combined attach procedure, the UE shall indicate "combined EPS/IMSI attach" in the EPS attach type IE.

The combined attach procedure follows the attach procedure for EPS described in subclause 5.5.1.2.

[TS 24.301, clause 5.5.1.3.2]

...

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

[TS 24.301, clause 5.5.1.3.4.1]

Depending on the value of the EPS attach result IE received in the ATTACH ACCEPT message, two different cases can be distinguished:

1)	The EPS attach result IE value indicates "combined EPS/IMSI attach": attach for EPS and non-EPS services have been successful.

2)	The EPS attach result IE value indicates "EPS only": attach for EPS services has been successful but attach for non-EPS services has failed.

[TS 24.301, clause 5.5.1.3.4.2]

The description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

...

[TS 24.301, clause 5.5.1.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

The UE receiving the ATTACH ACCEPT message takes one of the following actions depending on the EMM cause value:

...

#18	(CS domain not available)

The UE shall stop timer T3410 if still running, shall reset the tracking area updating attempt counter, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.NORMAL-SERVICE.

The UE shall set the update status to U2 NOT UPDATED.

A UE in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

A UE in CS/PS mode 2 of operation may provide a notification to the user or the upper layers that the CS domain is not available.

…

The UE shall not attempt combined attach or combined tracking area update procedure with current PLMN until switching off the UE or the UICC containing the USIM is removed.

Other EMM cause values and the case that no EMM cause IE was received are considered as abnormal cases. The combined attach procedure shall be considered as failed for EPS and non-EPS services. The behaviour of the UE in those cases is specified in subclause 5.5.1.3.6.

[TS 24.008, clause 4.1.2.2]

In parallel with the sublayer states described in subclause 4.1.2.1 and which control the MM sublayer protocol, an update status exists.

The update status pertains to a specific subscriber embodied by a SIM/USIM. This status is defined even when the subscriber is not activated (SIM/USIM removed or connected to a switched-off ME). It is stored in a non volatile memory in the SIM/USIM. The update status is changed only as a result of a location updating procedure attempt (with the exception of an authentication failure and of some cases of CM service rejection). In some cases, the update status is changed as a result of a GPRS attach, GPRS routing area update, service request or network initiated GPRS detach procedure.

...

U2 NOT UPDATED

The last location updating attempt made failed procedurally (no significant answer was received from the network, including the cases of failures or congestion inside the network).

[Rel-8]

-	For this status, the SIM/USIM does not contain any valid LAI, TMSI, GSM ciphering key, UMTS integrity key, UMTS ciphering key or ciphering key sequence number. For compatibility reasons, all these fields must be set to the "deleted" value at the moment the status is set to NOT UPDATED. However the presence of other values shall not be considered an error by the mobile station. The "Location update status" stored on the SIM/USIM shall be "not updated".

[Rel-11]

-	For this status, the SIM/USIM can contain a valid LAI of the location area to which the subscriber was registered, and possibly also a valid TMSI, GSM ciphering key, UMTS integrity key, UMTS ciphering key and ciphering key sequence number. For compatibility reasons, all these fields shall be set to the "deleted" value if the LAI is deleted. However the presence of other values shall not be considered an error by the mobile station. Furthermore, if the ME supports any A5 ciphering algorithm that requires a 128-bit ciphering key and a USIM is in use, then the ME shall delete the GSM Kc128 stored if the LAI is deleted. The "Location update status" stored on the SIM/USIM shall be "not updated".

...

9.2.1.2.4.3	Test description

9.2.1.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell A (TAI-1, home PLMN) and cell B (TAI-2, home PLMN).

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells;

UE:

- 	The UE is configured to initiate combined EPS/IMSI attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS.36.508 [18].

9.2.1.2.4.3.2	Test procedure sequence

Table 9.2.1.2.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the "Serving cell" Set the cell type of cell B to the "Non-suitable cell" | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE transmit a combined ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 4- 7E | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 8 | SS responds with ATTACH ACCEPT message with IE EPS Bearer Identity set to default EPS bearer context. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | ATTACH ACCEPT | - | - |
| 8A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the event described in step 9 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 9 below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH COMPLETE message including a ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified? | --> | ATTACH COMPLETE | 2 | P |
| 9Aa1-9 Cb1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 10 | Void | - | - | - | - |
| 10A | Wait 15 seconds for UE to camp on Cell A (UE might attempt to select GERAN or UTRAN cells) | - | - | - | - |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell A? | - | - | 2 | - |
| 12 | Set the cell type of cell B to the "Serving cell" Set the cell type of cell A to the "Non-suitable cell" | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 13 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 14 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15 | UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 16 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 16a describes behaviour that depends on the UE capability. |  |  |  |  |
| 16a | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST |  |  |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Check: Does the UE transmit a combined ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 18Aa1-18C | Steps 9a1-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 19 | SS responds with ATTACH ACCEPT message with IE EPS Bearer Identity set to default EPS bearer context. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 20 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (Note) THEN in parallel to the event described in step 20 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 20 | Check: Does the UE transmit an ATTACH COMPLETE message including a ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified? | --> | ATTACH COMPLETE | - | - |
| 20A1a1-20A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508 but attached for EPS services only. | - | - | - | - |
| Note: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.1.2.4.3.3	Specific message contents

Table 9.2.1.2.4.3.3-1: Message ATTACH REQUEST (step 3, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach type | 010 | "combined EPS/IMSI attach" |  |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.4.3.3-2: Message ATTACH ACCEPT (step 8, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" |  |
| GUTI | Not Present |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010010 | #18 "CS domain not available" |  |

Table 9.2.1.2.4.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 13, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 | If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| Old location area identification | Not present | Not present as this is not a combined TAU. |  |
| TMSI status | Not present | Not present as this in not a combined TAU |  |

Table 9.2.1.2.4.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 14, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-2 |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | Not Present | Not present as this is not a combined TAU |  |

Table 9.2.1.2.4.3.3-5: Message ATTACH REQUEST (step 18, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach type | 010 | "combined EPS/IMSI attach" |  |
| Old GUTI or IMSI | GUTI-2 |  |  |
| Last visited registered TAI | TAI-2 |  |  |
| Old location area identification | Not present |  | Rel-8 to Rel-10 |
|  | Not present or any allowed value |  | Rel-11 onwards |
| TMSI status | 0 | “no valid TMSI available” | Rel-8 to Rel-10 |
|  | Set to 0 or not present | “0: no valid TMSI available” “ Not present: valid TMSI available” | Rel-11 onwards |

Table 9.2.1.2.4.3.3-6: Message ATTACH ACCEPT (step 19, Table 9.2.1.2.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 010 | ''Combined EPS/IMSI attach'' |  |
| GUTI | GUTI-2 |  |  |
| Location area identification | LAI-2 |  |  |
| MS identity | TMSI-2 |  |  |

##### 9.2.1.2.4a	Successful combined attach procedure / EPS service only / Congestion

9.2.1.2.4a.1	Test Purpose (TP)

(1)

with { UE having initiated a Combined attach procedure by sending an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH ACCEPT message with EPS attach result set to 'EPS only' and EMM cause set to #22 'Congestion' }

then { the UE initiates a Combined tracking area procedure indicating 'combined TA/LA updating with IMSI attach' after time=T3402 }

}

9.2.1.2.4a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.1.3.4.3 and 5.2.3.2.7. Unless otherwise stated these are Rel-11 requirements.

[TS 24.301, clause 5.5.1.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for attach for EPS services as specified in subclause 5.5.1.2.4 shall be followed. In addition, the following description for attach for non-EPS services applies.

The UE receiving the ATTACH ACCEPT message takes one of the following actions depending on the EMM cause value:

...

#22	(Congestion)

The UE shall stop the timer T3410 if still running. The tracking area updating attempt counter shall be set to 5. The UE shall start the timer T3402, shall set the EPS update status to EU1 UPDATED, shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, and shall enter state MM IDLE.

[TS 24.301, clause 5.2.3.2.7]

The UE:

...

-	shall initiate combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" on the expiry of timers T3411 or T3402 or when the UE enters a tracking area not in the list of registered tracking areas.

9.2.1.2.4a.3	Test description

9.2.1.2.4a.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

-	The UE is configured to initiate combined EPS/IMSI attach

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.4a.3.2	Test procedure sequence

Table 9.2.1.2.4a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits a combined ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3-8 C | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 8D | SS responds with ATTACH ACCEPT message with EPS attach result set to 'EPS only' and EMM cause set to #22 (Congestion). The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | ATTACH ACCEPT | - | - |
| 8E | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 9 | Void | - | - | - | - |
| 9A-10Ab1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 11 | Wait for time=T3402. Note: The value of T3402 is provided in ATTACH ACCEPT message in step 8. | - | - | - | - |
| 12 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message indicating 'combined TA/LA updating with IMSI attach'? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 13 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 14 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of the test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.1.2.4a.3.3	Specific message contents

Table 9.2.1.2.4a.3.3-1: Message ATTACH REQUEST (step 2, Table 9.2.1.2.4a.3.2-1)

| Derivation Path: 36.508, table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach type | '010'B | "combined EPS/IMSI attach" |  |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.4a.3.3-2: Message ATTACH ACCEPT (step 8D, Table 9.2.1.2.4a.3.2-1)

| Derivation Path: 36.508, table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | '001'B | "EPS only" |  |
| GUTI | Not Present |  |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | '00010110'B | "Congestion" |  |
| T3402 | '00001111'B | 30 seconds |  |

Table 9.2.1.2.4a.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 12, Table 9.2.1.2.4a.3.2-1)

| Derivation Path: 36.508, table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | "combined TA/LA updating with IMSI attach" |  |

Table 9.2.1.2.4a.3.3-4: TRACKING AREA UPDATE ACCEPT (step 13, Table 9.2.1.2.4a.3.2-1)

Derivation Path: 36.508, table 4.7.2-24 with condition combined TA/LA updated

##### 9.2.1.2.5	Combined attach / Rejected / IMSI invalid

9.2.1.2.5.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'Illegal UE' }

then { UE deletes GUTI, last visited registered TAI and KSI and considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'Illegal UE' }

then { UE deletes P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number, TMSI, LAI and ciphering key sequence number }

}

9.2.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.5.1.3.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

#3		(Illegal UE);

…

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause5.1.3.3) and shall delete any GUTI, last visited registered TAI and eKSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

[TS 24.008, clause 4.7.3.2.4]

The MS shall then take one of the following actions depending upon the reject cause:

# 3		(Illegal MS);

...

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (shall store it according to subclause 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The new GMM state is GMM-DEREGISTERED. The new MM state is MM IDLE.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The SIM/USIM shall be considered as invalid for GPRS and non-GPRS services until switching off or the SIM/USIM is removed.

9.2.1.2.5.3	Test description

9.2.1.2.5.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (, LAI-1 and RAI-1;home PLMN) ;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, Cell 24 (, LAI-1 and RAI-1.home PLMN);

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.5.3.2	Test procedure sequence

Table 9.2.1.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell", - Cell B as a "Non-Suitable cell", - Cell 5 or Cell 24 as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with the EMM cause set to 'Illegal UE'. | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as the "Serving cell". | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell B or on Cell A? | --> | ATTACH REQUEST | 1 | F |
| - | EXCEPTION: Steps 8a1 to 8a6 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 8a1 | The SS configures: - Cell B as a "Non-Suitable cell", - Cell 5 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 8a2 | Void | - | - | - | - |
| 8a3 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8a4 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 8a5 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | EXCEPTION: Steps 8a5a1, 8a5a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that take place if pc_AutomaticAttachSwitchON is NOT supported | - | - | - | - |
| 8a5Aa1 | IF NOT pc_AutomaticAttachSwitchON | - | Registration on CS | - | - |
| 8a5Aa2 | IF NOT pc_AutomaticAttachSwitchON the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8a6 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 8b1 | Void | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS configures: - Cell A as the "Serving cell", - Cell B as a "Non-Suitable cell", - Cell 5 or Cell 24 as a " Cell Off ". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| - | Void |  |  |  |  |
| 11 | Void | - | - | - | - |
| 11a1 | Void | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 13-14 | Void | - | - | - | - |
| 15a1-15a9 | Void | - | - | - | - |
| 16-29b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: Void. Note 2: Switching off, USIM removal, or power removal shall be done before T3310 and T3311 expire (30 seconds) so that the UE does not retransmit ATTACH REQUEST message. |  |  |  |  |  |

9.2.1.2.5.3.3	Specific message contents

Table 9.2.1.2.5.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.2.5.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 0011'B | Illegal UE |  |

Table 9.2.1.2.5.3.3-2: Message ATTACH REQUEST (step 12, Table 9.2.1.2.5.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | '0'B | no valid TMSI available |  |

Table 9.2.1.2.5.3.3-3: Message ATTACH REQUEST (step 8a6, Table 9.2.1.2.5.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | '011'B | Combined GPRS/IMSI attach |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | '0'B | no valid TMSI available |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

##### 9.2.1.2.6	Combined attach / Rejected / Illegal ME

9.2.1.2.6.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'Illegal ME' }

then { UE deletes GUTI, last visited registered TAI and KSI and considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'Illegal ME' }

then { UE deletes P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number, TMSI, LAI and ciphering key sequence number }

}

9.2.1.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.5.1.3.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

…

#6		(Illegal ME); or

…

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause5.1.3.3) and shall delete any GUTI, last visited registered TAI and eKSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

[TS 24.008, clause 4.7.3.2.4]

The MS shall then take one of the following actions depending upon the reject cause:

...

# 6		(Illegal ME), or

...

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (shall store it according to subclause 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The new GMM state is GMM-DEREGISTERED. The new MM state is MM IDLE.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The SIM/USIM shall be considered as invalid for GPRS and non-GPRS services until switching off or the SIM/USIM is removed.

9.2.1.2.6.3	Test description

The test description is identical to the one of subclause 9.2.1.2.5 except that the reject cause #3 “illegal UE” is replaced with reject cause #6 “Illegal ME”

##### 9.2.1.2.7	Combined attach / Rejected / EPS services and non-EPS services not allowed

9.2.1.2.7.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'EPS services and non-EPS services not allowed' }

then { UE deletes GUTI, last visited registered TAI and KSI and considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE has sent an ATTACH REQUEST }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'EPS services and non-EPS services not allowed' }

then { UE deletes P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number, TMSI, LAI and ciphering key sequence number }

}

9.2.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.5.1.3.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

…

#8		(EPS services and non-EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause5.1.3.3) and shall delete any GUTI, last visited registered TAI and eKSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

[TS 24.008, clause 4.7.3.2.4]

The MS shall then take one of the following actions depending upon the reject cause:

...

# 8		(GPRS services and non-GPRS services not allowed);

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (shall store it according to subclause 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The new GMM state is GMM-DEREGISTERED. The new MM state is MM IDLE.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. The SIM/USIM shall be considered as invalid for GPRS and non-GPRS services until switching off or the SIM/USIM is removed.

9.2.1.2.7.3	Test description

The test description is identical to the one of subclause 9.2.1.2.5 except that the reject cause #3 “illegal UE” is replaced with reject cause #8 “EPS services and non-EPS services not allowed”

##### 9.2.1.2.8	Combined attach / Rejected / EPS services not allowed

9.2.1.2.8.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the reject cause set to "EPS services not allowed" }

then { UE considers the USIM as invalid for EPS services and enters state EMM-DEREGISTERED and UE does not attempt to attach on any other cell }

}

9.2.1.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5.

[TS 24.301, clause 5.5.1.3.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and eKSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

A UE which is not yet IMSI attached for non-EPS services shall select GERAN or UTRAN radio access technology and perform an IMSI attach for non-EPS services, using the MM IMSI attach procedure as described in 3GPP TS 24.008 [13]. The UE shall not reselect E-UTRAN radio access technology until switching off or the UICC containing the USIM is removed.

A UE which is already IMSI attached for non-EPS services is still IMSI attached for non-EPS services in the network. The UE shall select GERAN or UTRAN radio access technology and shall proceed with the appropriate MM specific procedure according to the MM service state. The UE shall not reselect E-UTRAN radio access technology until switching off or the UICC containing the USIM is removed.

NOTE:	Some interaction is required with the access stratum to disable E-UTRAN cell reselection.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

9.2.1.2.8.3	Test description

9.2.1.2.8.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (home PLMN);

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (home PLMN);

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach.

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.8.3.2	Test procedure sequence

Table 9.2.1.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Suitable neighbour cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services not allowed" as specified. | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 5A | The SS configures Cell 5 or Cell 24 as a "Suitable neighbour cell". | - | - | - | - |
| - | EXCEPTION: Steps 6a1 to 6a5 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 6a1 | The following messages are sent and shall be received on cell 5 or Cell 24. | - | - | - | - |
| 6a2 | Check: Does the UE transmit a LOCATION UPDATING REQUEST message with Location updating type set to "IMSI attach" on Cell 5 or Cell 24? | --> | LOCATION UPDATING REQUEST | 1 | P |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 6a2Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |  |  |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 6a2Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |  |  |
| 6a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 6a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 6a5 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 7 | The SS configures: - Cell 5 or Cell 24 as a "Non-Suitable cell". - Cell A as the "Suitable neighbour cell". - Cell B as the "Suitable neighbour cell". | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell A or on Cell B? | --> | ATTACH REQUEST | 1 | F |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

9.2.1.2.8.3.3	Specific message contents

Table 9.2.1.2.8.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.2.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 0111'B | EPS services not allowed |  |

Table 9.2.1.2.8.3.3-2: LOCATION UPDATING REQUEST (step 6a2, Table 9.2.1.2.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Location updating type | IMSI attach Note 1 | Rel8, Rel9, Rel10 |  |
|  | Normal attach | Note 1 |  |
| Note 1: Depending on the Release, one or any of these values is allowed. |  |  |  |

Table 9.2.1.2.8.3.3-3: LOCATION UPDATING ACCEPT (step 6a5, Table 9.2.1.2.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 9.2.1.2.9	Combined attach / Rejected / PLMN not allowed

9.2.1.2.9.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the reject cause set to "PLMN not allowed" }

then { UE deletes the GUTI, the last visited registered TAI and KSI and UE deletes the list of equivalent PLMNs and UE enters state EMM-DEREGISTERED.PLMN-SEARCH and UE stores the PLMN in the "forbidden PLMN list" }

}

(2)

with { UE is switched off and a PLMN is stored in the "forbidden PLMN list" }

ensure that {
  when { UE is powered on this PLMN }

then { UE does not attempt to attach on the cell }

}

(3)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {
  when { UE enters a PLMN which is not in the "forbidden PLMN list" }

then { UE attempts to attach on the cell }

}

(4)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { the forbidden PLMN is selected manually }

then { UE attaches to the forbidden PLMN and deletes this PLMN from the forbidden PLMN list on the USIM}

}

9.2.1.2.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5.

[TS 24.301, clause 5.5.1.3.5]

...

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#11	(PLMN not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, and KSI, and reset the attach attempt counter. The UE shall delete the list of equivalent PLMNs and enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMN list".

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value and no RR connection exists.

...

9.2.1.2.9.3	Test description

9.2.1.2.9.3.1	Pre-test conditions

System Simulator:

-	cell G, cell H (VPLMN, same MCC like HPLMN, different TAs) and cell I (VPLMN, different MCC from HPLMN);

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1;

-	if pc_UTRA AND , px_RATComb_Tested = EUTRA_UTRA N, cell 9 (belongs to RAI-1, same PLMN as cell G, Non-suitable off);

-	if pc_GERAN px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-1, same PLMN as cell G, Non-suitable off); -	the cells may not be simultaneously activated.

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell G using default message contents according to TS 36.508 [18];

-	the "forbidden PLMN list" is empty.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.9.3.2	Test procedure sequence

Table 9.2.1.2.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell G as the "Serving cell". - Cell H as a " Suitable Neighbour cell". - Cell I as a "Non-Suitable Off cell". Note: Cell G and Cell H are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "PLMN not allowed". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | - | ATTACH REQUEST | 1 | F |
| 7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 8A | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 2 | F |
| - | EXCEPTION: Steps 9a1 to 9a3 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 9a1 | The SS configures: - Cell H as a "Non-Suitable off cell". - Cell G as a "Non-Suitable cell". - Cell 9 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) as the "Serving cell". Note: Cell G, Cell 9 and Cell 24 are in the same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell 9 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 9a2 | Void | - | - | - | - |
| 9a3 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 2 | F |
| 10 | The SS configures: Cell 9 or Cell 24 as a "Non-Suitable off cell". Cell I as a " Serving cell". Note: Cell G belongs to PLMN different from PLMN belonging to Cell I. | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1,3 | P |
| 12-23, 23A | The attach procedure is completed by executing steps 5 to 17 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| 24 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 25 describes behaviour that depends on the UE capability. |  |  |  |  |
| 25 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 26 | The SS configures: - Cell G as the "Serving cell" - Cell I as a "Non-suitable cell". Note: Cell G belongs to the forbidden PLMN. | - | - | - | - |
| 27 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| 28 | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 29 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. | - | - | - | - |
| 30 | The user selects PLMN of cell G. | - | - | - | - |
| 31 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 4 | P |
| 32-44 | The attach procedure is completed and the RRC connection is released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.9.3.3	Specific message contents

Table 9.2.1.2.9.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.2.9.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1011'B | PLMN not allowed |  |

Table 9.2.1.2.9.3.3-2: Message ATTACH REQUEST (step 11, Table 9.2.1.2.9.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI1 |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.1.2.9.3.3-3: Message ATTACH REQUEST (step 31, Table 9.2.1.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI allocated in step 21 |  |  |
| Last visited registered TAI | TAI allocated in step 21 |  |  |

##### 9.2.1.2.10	Combined attach / Rejected / Tracking area not allowed

9.2.1.2.10.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the reject cause set to "Tracking area not allowed" }

then { UE deletes the GUTI, last visited registered TAI and KSI, UE enters the state EMM-DEREGISTERED.LIMITED-SERVICE and UE stores the current TAI in the list of "forbidden tracking areas for regional provision of service" }

}

(2)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service" }

ensure that {
  when { serving cell belongs to TAI where UE was rejected }

then { UE does not attempt to attach on any cell }

}

(3)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service" }

ensure that {
  when { UE re-selects a new cell in the same TAI it was rejected }

then { UE does not attempt to attach on the cell }

}

(4)

with { UE is switched off }

ensure that {
  when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for regional provision of service" before the UE was switched off }

then { UE attempts to attach on the cell }

}

9.2.1.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.3.5.

[TS 24.301, clause 5.5.1.3.5]

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#12	(Tracking area not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall reset the attach attempt counter and enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for regional provision of service".

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

9.2.1.2.10.3	Test description

9.2.1.2.10.3.1	Pre-test conditions

System Simulator:

-	Cell A, Cell B and Cell M.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.10.3.2	Test procedure sequence

Table 9.2.1.2.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell B as a "Suitable neighbour cell". - Cell M as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "Tracking area not allowed". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds on Cell A or Cell B? | - | ATTACH REQUEST | 1,2 | F |
| 7 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Non-Suitable cell". - Cell M as a "Serving cell". | - | - | - | - |
| 8 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds on Cell M? | - | ATTACH REQUEST | 3 | F |
| - | The following messages are to be observed on Cell M unless explicitly stated otherwise. | - | - | - | - |
| 9 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 10 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1,4 | P |
| 12-25b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.10.3.3	Specific message contents

Table 9.2.1.2.10.3.3-0: SystemInformationBlockType1 of Cell A (all steps, Table 9.2.1.2.10.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.1.2.10.3.3-1: Message ATTACH REJECT (step 4, Table 9.2.1.2.10.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1100'B | Tracking Area not allowed |  |

Table 9.2.1.2.10.3.3-2: Message ATTACH REQUEST (step 11, Table 9.2.1.2.10.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI1 |  |  |
| Last visited registered TAI | Not present |  |  |

##### 9.2.1.2.11	Combined attach / Rejected / Roaming not allowed in this tracking area

9.2.1.2.11.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to "Roaming not allowed in this tracking area" }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED and UE deletes the GUTI, the last visited registered TAI and KSI and UE enters the state EMM-DEREGISTERED.LIMITED-SERVICE or optionally EMM-DEREGISTERED.PLMN-SEARCH and UE stores the current TAI in the list of "forbidden tracking areas for roaming" and deletes the TMSI, the LAI and the ciphering key sequence number }

}

(2)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-DEREGISTERED.PLMN-SEARCH state and the current TAI is in the list of "forbidden tracking areas for roaming" }

ensure that {
  when { UE re-selects a new cell in the same TA where it was rejected }

then { UE does not attempt to attach }

}

(3)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-DEREGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming" }

ensure that {
  when { UE enters a cell belonging to a tracking area not in the list of "forbidden tracking areas for roaming" }

then { UE attempts to attach with IMSI }

}

(4)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE or EMM-DEREGISTERED.PLMN-SEARCH state and the list of "forbidden tracking areas for roaming" contains more than one TAI }

ensure that {
  when { UE selects a cell belonging to one of the TAIs in the list of "forbidden tracking areas for roaming" }

then { UE does not attempt to attach }

}

(5)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to "Roaming not allowed in this tracking area" }

then { UE deletes RAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number and sets the GPRS update status to GU3 ROAMING NOT ALLOWED }

}

(6)

with { UE is switched off or the UICC containing the USIM is removed }

ensure that {
  when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for roaming" before UE was switched off or the USIM is inserted again on that cell }

then { UE performs registration on that cell }

}

(7)

with { a cell of the HPLMN is available }

ensure that {
  when { UE performs a PLMN selection }

then { UE returns to a cell of the HPLMN }

}

9.2.1.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.2 and 5.5.1.3.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.3.2]

The UE shall store a list of "forbidden tracking areas for roaming", as well as a list of "forbidden tracking areas for regional provision of service". These lists shall be erased when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours). One or more tracking areas is removed from the list of "forbidden tracking areas for roaming" in the UE, as well as the list of "forbidden tracking areas for regional provision of service" if, after a subsequent procedure e.g. attach procedure, tracking area updating procedure and GUTI reallocation procedure, one or more tracking areas in the lists is received from the network.

In S1 mode, the UE shall update the suitable list whenever an ATTACH REJECT, TRACKING AREA UPDATE REJECT, SERVICE REJECT or DETACH REQUEST message is received with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area".

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

[TS 24.301, clause 5.5.1.3.5]

If the attach request can neither be accepted by the network for EPS nor for non-EPS services, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value. If the attach procedure fails due to a default EPS bearer setup failure or an ESM procedure failure, the MME shall combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19, "ESM failure".

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#13	(Roaming not allowed in this tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. The UE shall delete the list of equivalent PLMNs and reset the attach attempt counter. Additionally the UE enter the state EMM-DEREGISTERED.LIMITED-SERVICE or optionally EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming".

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

...

[TS 24.008, clause 4.7.3.2.4]

If the attach request can neither be accepted by the network for GPRS nor for non-GPRS services, an ATTACH REJECT message is transferred to the MS. The MS receiving the ATTACH REJECT message stops timer T3310, and for all causes except #12, #14, #15 and #25 deletes the list of "equivalent PLMNs".

The MS shall then take one of the following actions depending upon the reject cause:

...

# 13	(Roaming not allowed in this location area);

The MS shall delete any RAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number, shall set the GPRS update status to GU3 ROAMING NOT ALLOWED(and shall store it according to clause 4.1.3.2) and shall reset the GPRS attach attempt counter. The state is changed to GMM-DEREGISTERED.LIMITED-SERVICE or optionally to GMM-DEREGISTERED.PLMN-SEARCH.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, reset the location update attempt counter and shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM IDLE.

...

9.2.1.2.11.3	Test description

9.2.1.2.11.3.1	Pre-test conditions

System Simulator:

-	cell I and cell K (visited PLMN, same TA);

-	cell L (same visited PLMN, another TA);

-	cell C (home PLMN);

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f2;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (only active when stated):

-	same PLMN like visited PLMN above;

-	RAI-1 (RAC & LAC values chosen by SS);

-	System information indicate that NMO 1 is used;

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (only active when stated):

-	same PLMN like visited PLMN above;

-	RAC-1 (RAC & LAC values chosen by SS);

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE 1: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

NOTE 2:	Cell K is present to confirm that UE does not attempt attach to the cell in same TAI after reject from the SS.

NOTE 3:	Cell C is present to confirm that UE does attempt attach to the cell in HPLMN after reject from the SS.

NOTE 4:	The requirement in 3GPP TS 24.301 to store at least 40 entries in the list of "forbidden tracking areas for roaming" is not fully tested.

NOTE 5:	Different types of UE may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12 am) for roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell I using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 9 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.11.3.2	Test procedure sequence

Table 9.2.1.2.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell I as the "Serving cell". Cell K as a "Non-Suitable cell", Cell L as a "Non-Suitable “off” cell", Cell C as a "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with the EMM cause set to "Roaming not allowed in this tracking area" on Cell I. (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell I? | --> | ATTACH REQUEST | 1,4 | F |
| 7 | The SS reconfigures: Cell I as a "Suitable cell", Cell K as the "Serving cell", Cell L as a "Non-Suitable “off” cell", Cell C as a "Non-Suitable cell". | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on any cell? | --> | ATTACH REQUEST | 2 | F |
| 9 | The SS reconfigures: Cell I as a "Non-Suitable “off” cell", Cell K as a "Suitable cell", Cell L as the "Serving cell", Cell C as a "Non-Suitable cell". | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on Cell L? | --> | ATTACH REQUEST | 3 | P |
| 10A-10D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 11 | The SS transmits an ATTACH REJECT message with the EMM cause set to "Roaming not allowed in this tracking area" on Cell L. (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9 and TAI-11) | <-- | ATTACH REJECT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell L or Cell K? | --> | ATTACH REQUEST | 1,4 | F |
| 14 | The SS reconfigures: Cell I as a "Serving cell", Cell K as a " Non-Suitable “off” cell", Cell L as a "Suitable cell", Cell C as the "Non-Suitable cell". | - | - | - | - |
| 15 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on any cell? | --> | ATTACH REQUEST | 2,4 | F |
| - | EXCEPTION: Steps 16a1 to 16a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 16a1 | The SS reconfigures: Cell I as "Serving cell", Cell L as "Non-suitable “off" cell", Cell 9 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) as "Suitable cell". | - | - | - | - |
| 16a2 | Check: Does the UE transmit an ATTACH REQUEST message without P-TMSI, P-TMSI signature, RAI, TMSI on Cell 9 or Cell 24? | --> | ATTACH REQUEST | 5 | P |
| 16a3 | The SS transmits an ATTACH REJECT message with the GMM cause set to "Roaming not allowed in this location area" on Cell 9 or Cell 24. | <-- | ATTACH REJECT | - | - |
| 17 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 18 | The SS reconfigures: Cell I as the "Serving cell", Cell K as a "Non-Suitable cell", Cell 9 or Cell 24 as a "Non-Suitable “off” cell", Cell C as a "Non-Suitable cell". | - | - | - | - |
| 19 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 20 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on Cell I? | --> | ATTACH REQUEST | 6 | P |
| 20A-20D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 21 | The SS transmits an ATTACH REJECT message with the EMM cause set to "Roaming not allowed in this tracking area" on Cell I. (The list of "forbidden tracking areas for roaming" in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 21A | The SS releases the RRC connection. | - | - | - | - |
| 22 | The SS reconfigures: Cell I as the "Serving cell", Cell K as a "Non-Suitable cell", Cell C as a "Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 23 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 7 | P |
| 24-37b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.11.3.3	Specific message contents

Table 9.2.1.2.11.3.3-1: Message ATTACH REJECT (step 4, step 11 and step 21, Table 9.2.1.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1101'B | Roaming not allowed in this tracking area |  |

Table 9.2.1.2.11.3.3-2: Message ATTACH REQUEST (step 10, step 20 and step23, Table 9.2.1.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | '0'B | no valid TMSI available |  |

Table 9.2.1.2.11.3.3-3: Message ATTACH REQUEST (step 16a2, Table 9.2.1.2.11.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | '011'B | Combined GPRS/IMSI attach |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | '0'B | no valid TMSI available |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.1.2.11.3.3-4: Message ATTACH REJECT (step 16a3, Table 9.2.1.2.11.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GMM cause | '0000 1101'B | Roaming not allowed in this location area |  |
| T3302 value | Not present |  |  |

##### 9.2.1.2.12	Combined attach / Rejected / EPS services not allowed in this PLMN

9.2.1.2.12.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "EPS services not allowed in this PLMN" }

then { UE deletes any GUTI, last visited registered TAI, KSI and enters EMM-DEREGISTERED.PLMN-SEARCH state}

}

(2)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE detects a cell which belongs to a PLMN which is in the "forbidden PLMNs for GPRS service list" }

then { UE doesn’t perform an attach procedure }

}

(3)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE detects a cell which belongs to a PLMN which is not in the "forbidden PLMNs for GPRS service list" }

then { UE performs an attach procedure }

}

(4)

with { UE is switched off when a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE is powered on a cell which belongs to this PLMN }

then { UE performs an attach procedure }

}

(5)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state, and a PLMN is stored in the "forbidden PLMNs for GPRS service list" }

ensure that {

when { UE is in the cell which belongs to the rejected PLMN and when that PLMN is selected manually }

then { UE performs an attach procedure }

9.2.1.2.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.4.4.3.

[TS 24.301, clause5.5.1.3.5]

If the attach request can neither be accepted by the network for EPS nor for non-EPS services, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value. If the attach procedure fails due to a default EPS bearer setup failure or an ESM procedure failure, the MME shall combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19, "ESM failure".

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#14	(EPS services not allowed in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. Additionally the UE shall reset the attach attempt counter and enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMNs for GPRS service" list.

A UE operating in CS/PS mode 1 which is not yet IMSI attached for non-EPS services may select GERAN or UTRAN radio access technology and perform an IMSI attach for non-EPS services, using the MM IMSI attach procedure as described in 3GPP TS 24.008 [13]. In this case the UE shall not reselect E-UTRAN radio access technology for the duration the UE is on the PLMN or an equivalent PLMN.

A UE operating in CS/PS mode 1 which is already IMSI attached for non-EPS services in the network is still IMSI attached for non-EPS services in the network. The UE may select GERAN or UTRAN radio access technology and proceed with the appropriate MM specific procedure according to the MM service state. In this case the UE shall not reselect E-UTRAN radio access technology for the duration the UE is on the PLMN or an equivalent PLMN.

A UE in CS/PS mode 1 of operation may perform a PLMN selection according to 3GPP TS 23.122 [6].

A UE operating in CS/PS mode 2 which is already IMSI attached for non-EPS services in the network is still IMSI attached for non-EPS services in the network.

A UE operating in CS/PS mode 2 of operation shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

9.2.1.2.12.3	Test description

9.2.1.2.12.3.1	Pre-test conditions

System Simulator:

-	cell G, Cell H and cell I are configured according to Table 6.3.2.2-1 in; [18].

-	cell G and Cell H with MCC-1/MNC-2 (visited PLMN, different TAs)

-	Cell I with MCC-2/MNC-101 (visited PLMN);

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 36.508 [18], except replacing f3 with f1

-	the cells may not be simultaneously activated.

UE:

- 	the UE is configured to initiate Combined attach;

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell G using default message contents according to TS 36.508 [18];

-	the "forbidden PLMNs for GPRS service list" is empty.

-	the different cells may not be simultaneously activated (at most 2 cells are active simultaneously).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.12.3.2	Test procedure sequence

Table 9.2.1.2.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell G as the "Serving cell". - Cell H as a "Non-Suitable cell". - Cell I as a "Non-Suitable off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 1 | F |
| 7 | The SS configures: Cell G as a "Non-Suitable cell". Cell H as the "Serving cell". Note: Cell G and Cell H are in the different TAI – same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell H unless explicitly stated otherwise. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 9 | The SS configures: Cell H as a "Non-Suitable off cell". Cell I as the "Serving cell". Note: Cell G and Cell I are different PLMNs. | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 1,3 | P |
| 11-23 | The SS completes the attach procedure successfully and then releases the RRC -connection by executing steps 5 to 17 of UE registration procedure in TS 36.508 clause 4.5.2.3. |  |  |  |  |
| 24 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 25 | The SS configures - Cell I as a "Non-Suitable cell". - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 26 | The UE is brought back to operation or the USIM is inserted. |  |  |  |  |
| 27 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 27A-27D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 28 | The SS transmits an ATTACH REJECT message including EMM cause = "EPS services not allowed in this PLMN". | <-- | ATTACH REJECT | - | - |
| 29 | SS releases the RRC connection. | - | - | - | - |
| 30 | The user sets the UE in manual PLMN selection mode or requests a PLMN search. The user selects PLMN (MCC-1/MNC-2) |  |  |  |  |
| 31 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message as specified? | --> | ATTACH REQUEST | 5 | P |
| 32-44 | The attach procedure is completed and the RRC connection released by executing steps 5 to 17 of the UE registration procedure in TS 36.508 clause 4.5.2.3. | - | - | - | - |
| 45 | At the end of this test procedure sequence, the UE is in end state E-UTRA manual selection (E5) according to TS 36.508. | - | - | - | - |

9.2.1.2.12.3.3	Specific message contents

Table 9.2.1.2.12.3.3-1: Message ATTACH REQUEST (step 3, Table 9.2.1.2.12.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 |  |  |  |  |
| Last visited registered TAI | TAI-1 |  |  |  |  |
| Old location area identification | LAI-1 |  |  |  |  |
| TMSI status | Not Present |  |  |  |  |

Table 9.2.1.2.12.3.3-2: Message ATTACH REJECT (step 4 and 28, Table 9.2.1.2.12.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 (Plain NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001110 | #14 "EPS services not allowed in this PLMN " |  |
| ESM message container | Not present |  |  |

Table 9.2.1.2.12.3.3-3: Message ATTACH REQUEST (step 10 and step 31, Table 9.2.1.2.12.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 5 and step28; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 5 and step29. |  |

##### 9.2.1.2.13	Combined attach / Rejected / No suitable cells in tracking area

9.2.1.2.13.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'No Suitable Cells In tracking area' }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, deletes any GUTI, last visited registered TAI and KSI, resets the attach attempt counter, enters the state EMM-DEREGISTERED.LIMITED-SERVICE and stores the current TAI in the list of "forbidden tracking areas for roaming" }

}

(2)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI is in the list of "forbidden tracking areas for roaming" }

ensure that {
  when { UE re-selects a cell that belongs to the TAI where UE was rejected }

then { UE does not attempt to attach }

}

(3)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI is in the list of "forbidden tracking areas for roaming" and KSI was deleted }

ensure that {
  when { in the same PLMN, UE enters a cell which provides normal service and belongs to the tracking area not in the list of "forbidden tracking areas for roaming" }

then { UE attempts to attach with IMSI indicated that no key is available }

}

(4)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI is in the list of "forbidden tracking areas for roaming" }

ensure that {
  when { there are cells in the same PLMN and other PLMN that provide normal service and belong to the tracking area not in the list of "forbidden tracking areas for roaming" }

then { UE attempts to attach to the cell in the same PLMN }

}

(5)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the list of "forbidden tracking areas for roaming" contains more than one TAI }

ensure that {
  when { UE re-selects a cell that belongs to one of the TAIs in the list of "forbidden tracking areas for roaming" }

then { UE does not attempt to attach }

}

(6)

with { UE has sent an ATTACH REQUEST message }

ensure that {
  when { UE receives an ATTACH REJECT message with the EMM cause set to 'No Suitable Cells In tracking area' }

then { UE deletes RAI, P-TMSI, P-TMSI signature, GPRS ciphering key sequence number, TMSI, LAI and ciphering key sequence number }

}

(7)

with { UE is switched off }

ensure that {
  when { UE is powered on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for roaming" before UE was switched off }

then { UE attempts to attach }

}

9.2.1.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.2 and 5.5.1.3.5 and TS 24.008, clause 4.7.3.2.4.

[TS 24.301, clause 5.3.2]

The UE shall store a list of "forbidden tracking areas for roaming", as well as a list of "forbidden tracking areas for regional provision of service". These lists shall be erased when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours). One or more tracking areas is removed from the list of "forbidden tracking areas for roaming" in the UE, as well as the list of "forbidden tracking areas for regional provision of service" if, after a subsequent procedure e.g. attach procedure, tracking area updating procedure and GUTI reallocation procedure, one or more tracking areas in the lists is received from the network.

In S1 mode, the UE shall update the suitable list whenever an ATTACH REJECT, TRACKING AREA UPDATE REJECT, SERVICE REJECT or DETACH REQUEST message is received with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area".

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

[TS 24.301, clause 5.5.1.3.5]

If the attach request can neither be accepted by the network for EPS nor for non-EPS services, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value. If the attach procedure fails due to a default EPS bearer setup failure or an ESM procedure failure, the MME shall combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19, "ESM failure".

Upon receiving the ATTACH REJECT message, the UE shall stop timer T3410, enter MM state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#15	(No suitable cells in tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI and KSI. Additionally the UE shall reset the attach attempt counter and enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming".

The UE shall search for a suitable cell in another tracking area or in another location area in the same PLMN according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined attach procedure is rejected with the GMM cause with the same value.

...

[TS 24.008, clause 4.7.3.2.4]

...

The MS shall then take one of the following actions depending upon the reject cause:

...

# 15	(No Suitable Cells In Location Area);

The MS shall delete any RAI, P-TMSI, P-TMSI signature and GPRS ciphering key sequence number, shall set the GPRS update status to GU3 ROAMING NOT ALLOWED(and shall store it according to clause 4.1.3.2) and shall reset the GPRS attach attempt counter. The state is changed to GMM-DEREGISTERED.LIMITED-SERVICE.

The MS shall set the update status to U3 ROAMING NOT ALLOWED, reset the location update attempt counter and shall delete any TMSI, LAI and ciphering key sequence number. The new MM state is MM IDLE.

...

9.2.1.2.13.3	Test description

9.2.1.2.13.3.1	Pre-test conditions

System Simulator:

-	cell I and cell K (visited PLMN, same TA);

- 	cell E (same visited PLMN, another TA);

-	cell J (another VPLMN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 8 (only active when stated):

-	same PLMN like visited PLMN above;

-	RAI-1 (RAC & LAC values chosen by SS);

-	system information indicates that NMO 1 is used;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (only active when stated:

-	same PLMN like visited PLMN above;

-	RAC-1 (RAC & LAC values chosen by SS);

-	system information indicates that NMO 1 is used;

-	maximum 3 cells are simultaneously active.

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

-     the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 36.508 [18], except replacing f4 with f1.

NOTE 1: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

NOTE 2:	Cell E is present to confirm that UE searches in the same PLMN after reject from the SS.

NOTE 3:	Cell K is present to confirm that UE shall not attempt attach to the cell in same TAI it was once rejected from.

NOTE 4:	The requirement in 3GPP TS 24.301 to store at least 40 entries in the list of "forbidden tracking areas for roaming" is not fully tested.

NOTE 5:	Different types of UE may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12 am) for roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell I using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 8 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.13.3.2	Test procedure sequence

Table 9.2.1.2.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell I as the "Serving cell", Cell K as a "Suitable cell", Cell E as a "Non-Suitable cell", Cell J as a "Non-Suitable" off” cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3A-3D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 4 | The SS transmits an ATTACH REJECT message with the EMM cause set to 'No suitable cells in tracking area' on Cell I. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS reconfigures: Cell I as a "Suitable cell", Cell K as the "Serving cell", Cell E as a "Non-Suitable cell", Cell J as a "Non-Suitable" off” cell". | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Cell K or Cell I? | --> | ATTACH REQUEST | 2 | F |
| 7A | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 7B | The SS configures: Cell I as the "Serving cell", Cell K as a "Suitable cell", | - | - | - | - |
| 7C | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 7D | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 7DA-7DD | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 7E | The SS transmits an ATTACH REJECT message with the EMM cause set to 'No suitable cells in tracking area' on Cell I. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9) | <-- | ATTACH REJECT | - | - |
| 8 | The SS reconfigures: Cell K as a “Non-Suitable “off” cell”, Cell E as a "Suitable cell", Cell J as a "Suitable cell". | - | - | - | - |
| 8A | The SS releases the RRC connection. (IE redirectedCarrierInfo includes eutra CarrierFreq of Cell E) | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on Cell E? Note: Cell J is present to ensure the UE doesn’t attempt to attach in a cell with a different PLMN | --> | ATTACH REQUEST | 1, 3, 4, 6 | P |
| 9A-9D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 9A | The SS reconfigures: Cell J as a "Non-Suitable “off” cell". | - | - | - | - |
| 10 | The SS transmits an ATTACH REJECT message with the EMM cause set to 'No suitable cells in tracking area' on Cell E. (The list of “forbidden tracking areas for roaming” in the UE should now contain TAI-9 and TAI-12) | <-- | ATTACH REJECT | - | - |
| 11 | The SS releases the RRC connection | - | - | - | - |
| 12 | The SS reconfigures: Cell I as the "Serving cell". Cell K as a “Non-Suitable cell”, Cell E as a "Suitable cell". | - | - | - | - |
| 13 | Check: Does the UE transmit the ATTACH REQUEST message in the next 30 seconds on Cell I? | --> | ATTACH REQUEST | 5 | F |
| - | EXCEPTION: Steps 14a1 to 14a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 14a1 | The SS configures: Cell I as a "Non-Suitable cell", Cell E as a "Non-Suitable cell", Cell K as a "Non-Suitable “off” cell". Cell 8 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 8 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 14a2 | Void | - | - | - | - |
| 14a3 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 6 | P |
| 14a4 | The SS transmits an ATTACH REJECT message with the GMM cause set to 'No Suitable Cells In Location Area'. | <-- | ATTACH REJECT | - | - |
| 14a5 | The SS releases the RRC connection | - | - | - | - |
| 14A | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 15 | The SS reconfigures: Cell I as the "Serving cell". Cell K as a “Non-Suitable “off” cell”, Cell E as a "Non-Suitable cell", Cell 8 or Cell 24 as the "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 16 | Void | - | - | - | - |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 7 | P |
| 19-32b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.13.3.3	Specific message contents

Table 9.2.1.2.13.3.3-1: Message ATTACH REJECT (step 4 and step 10, Table 9.2.1.2.13.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1111'B | No Suitable Cells In tracking area |  |

Table 9.2.1.2.13.3.3-2: Message ATTACH REQUEST (step 9 and step 18, Table 9.2.1.2.13.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | '0'B | no valid TMSI available |  |

Table 9.2.1.2.13.3.3-3: Message ATTACH REQUEST (step 14a3, Table 9.2.1.2.13.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | '011'B | Combined GPRS/IMSI attach |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | '0'B | no valid TMSI available |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.1.2.13.3.3-4: Message ATTACH REJECT (step 14a4, Table 9.2.1.2.13.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GMM cause | '0000 1111'B | No Suitable Cells In Location Area |  |
| T3302 value | Not present |  |  |

Table 9.2.1.2.13.3.3-5: Message RRCConnectionRelease (step 8A, Table 9.2.1.2.13.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell E |  |  |
| } |  |  |  |
| redirectedCarrierInfo | Not present |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell E |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

##### 9.2.1.2.14	Combined attach / Rejected / Not authorized for this CSG

9.2.1.2.14.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on a CSG cell }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Not authorized for this CSG" and without integrity protection }

then { UE discards this message }

}

(2)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on a CSG cell which is contained in the Allowed CSG list }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE removes the CSG ID from the Allowed CSG list }

(3)

with { UE has sent an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message on a CSG cell }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE searches for a suitable cell }

9.2.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.5 , and TS 36.304, clause 4.3.

[TS 24.301, clause 5.5.1.2.5]

[Rel-8]#25	(Not authorized for this CSG);

Cause #25 is only applicable when received from a CSG cell. Cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.6.

If the ATTACH REJECT message with cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). Additionally, the UE shall reset the attach attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall remove the CSG ID of the cell where the UE has sent the ATTACH REQUEST message from the Allowed CSG list.

The UE shall search for a suitable cell in the same PLMN according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

[Rel-12]

If the ATTACH REJECT message with EMM cause #25 was received without integrity protection, then the UE shall discard the message.

…

#25	(Not authorized for this CSG);

EMM cause #25 is only applicable when received from a CSG cell. EMM cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.6.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and store it according to subclause 5.1.3.3). Additionally, the UE shall reset the attach attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

If the CSG ID and associated PLMN identity of the cell where the UE has sent the ATTACH REQUEST message are contained in the Allowed CSG list, the UE shall remove the entry corresponding to this CSG ID and associated PLMN identity from the Allowed CSG list.

If the CSG ID and associated PLMN identity of the cell where the UE has sent the ATTACH REQUEST message are contained in the Operator CSG list, the UE shall apply the procedures defined in 3GPP TS 23.122 [6] subclause 3.1A.

The UE shall search for a suitable cell according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

[TS 36.304, clause 4.3]

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements.

-	The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

9.2.1.2.14.3	Test description

9.2.1.2.14.3.1	Pre-test conditions

System Simulator:

-	cell A (TAC-1, frequency 1, not a CSG cell);

-	cell B (TAC-2, frequency 1, a CSG cell whose CSG Identity is included in Allowed CSG list);

-	cell G (another PLMN, frequency 2 and not a CSG cell).

-	System information combination 7 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

- 	the UE is previously registered on cell B using either manual CSG selection or a USIM with field EFACSGL preconfigured (so the allowed CSG list includes CSG ID of cell B);

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.14.3.2	Test procedure sequence

Table 9.2.1.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". - Cell G as a "Not Suitable cell". | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | UE initiates attach procedure and sends ATTACH REQUEST including a PDN CONNECTIVITY REQUEST message on cell B. | --> | ATTACH REQUEST | - | - |
| 4 | The SS transmits an ATTACH REJECT message with EMM cause = "Not authorized for this CSG" without integrity protection. | <-- | ATTACH REJECT | - | - |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message on Cell B after the expiry of timer T3410 and T3411? Note 1: IF UE initiate ATTACH procedure again, it can prove that this UE has discard the unprotected ATTACH REJECT message. Note 2: Default value of T3410 is 15s; default value of T3411 is 10s. In this TC, the network will wait for total 30 seconds. | --> | ATTACH REQUEST | 1 | P |
| 7 | The SS transmits an ATTACH REJECT message with EMM cause = "Not authorized for this CSG" with integrity protection. | <-- | ATTACH REJECT | - | - |
| 8 | The SS releases the RRC connection. | - | - | - | - |
| 9 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message on Cell B in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 11 | The SS configures: - Cell B as a "Not Suitable cell". - Cell G as a "Serving cell". - Cell A as a "Suitable cell". |  |  |  |  |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message on Cell A? | --> | ATTACH REQUEST | 3 | P |
| 13-26b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.14.3.3	Specific message contents

Table 9.2.1.2.14.3.3-1: Message ATTACH REQUEST (step 3, step 6, step 12, Table 9.2.1.2.14.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.1.2.14.3.3-2: Message ATTACH REJECT (step 4, Table 9.2.1.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Security header type | '0000'B | " Plain NAS message, not security protected " |  |
| EMM cause | '00011001'B | #25 " Not authorized for this CSG" |  |

Table 9.2.1.2.14.3.3-3: Message ATTACH REJECT (step 7, Table 9.2.1.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00011001'B | #25 " Not authorized for this CSG" |  |

Table 9.2.1.2.14.3.3-4: SystemInformationBlockType1 for Cell A, B, G (Pre-test conditions and all steps in Table 9.2.1.2.14.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell B |
|  | FALSE |  | Cell G |
|  | FALSE |  | Cell A |
| csg-Identity | Not present |  | Cell A |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell B |
|  | Not present |  | Cell G |

##### 9.2.1.2.15	Combined attach / Abnormal case / Handling of the EPS attach attempt counter

9.2.1.2.15.1	Test Purpose (TP)

(1)

with { UE has detected T3410 expiry after sending an ATTACH REQUEST message and has the attach attempt counter set to the value less than five }

ensure that {
  when { UE detects T3411 expiry }

then { UE restarts the attach procedure }

}

(2)

with { UE has sent an ATTACH REQUEST message and has the attach attempt counter set to five }

ensure that {
  when { UE detects T3410 expiry }

then { UE deletes GUTI, TAI list, last visited registered TAI and KSI }

}

(3)

with { UE has sent an ATTACH REQUEST message and has the attach attempt counter set to five }

ensure that {
  when { UE detects T3410 expiry }

then { UE deletes LAI, TMSI, ciphering key sequence number, RAI, P-TMSI, P-TMSI signature, and GPRS ciphering key sequence number }

}

9.2.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.1.1, 5.5.1.2.6 and 5.5.1.3.6 and TS 24.008, clause 4.7.3.1.5.

[TS 24.301, clause 5.5.1.1]

...

An attach attempt counter is used to limit the number of subsequently rejected attach attempts. The attach attempt counter shall be incremented as specified in subclause 5.5.1.2.6. Depending on the value of the attach attempt counter, specific actions shall be performed. The attach attempt counter shall be reset when:

-	the UE is powered on;

-	a USIM is inserted;

-	an attach or combined attach procedure is successfully completed;

-	a combined attach procedure is completed for EPS services only with cause #2, #16, #17, #18 or #22;

-	an attach or combined attach procedure is rejected with cause #11, #12, #13, #14, #15 or #25; or

-	a network initiated detach procedure is completed with cause #11, #12, #13, #14, #15 or #25.

Additionally the attach attempt counter shall be reset when the UE is in sub state EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH and:

-	a new tracking area is entered; or

-	T3402 expires.

[TS 24.301, clause 5.5.1.3.2]

If the UE is in EMM state EMM-DEREGISTERED, the UE initiates the combined attach procedure by sending an ATTACH REQUEST message to the network, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1).

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the ATTACH REQUEST message.

If the UE initiates a combined attach procedure for EPS services and "SMS only", the UE shall indicate "SMS only" in the Additional update type IE.

[TS 24.301, clause 5.5.1.3.6]

...

If the attach attempt counter is incremented according to subclause 5.5.1.2.6 the next actions depend on the value of the attach attempt counter:

-	if the update status is U1 UPDATED and the attach attempt counter is less than 5, then the UE shall keep the update status to U1 UPDATED, the new MM state is MM IDLE sub state NORMAL SERVICE;

-	if the attach attempt counter is less than 5 and, additionally, the update status is different from U1 UPDATED, then the UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. The MM state remains MM LOCATION UPDATING PENDING; or

-	if the attach attempt counter is equal to 5, then the UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. A UE operating in CS/PS mode 1 of operation shall select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures.

NOTE:	It is up to the UE implementation when to enable E-UTRAN radio access technology selection.

[TS 24.008, clause 4.7.3.1.5]

...

If the GPRS attach attempt counter is greater than or equal to 5:

-	the MS shall delete any RAI, P-TMSI, P-TMSI signature, list of equivalent PLMNs, and GPRS ciphering key sequence number, shall set the GPRS update status to GU2 NOT UPDATED, shall start timer T3302. The state is changed to GMM-DEREGISTERED. ATTEMPTING-TO-ATTACH or optionally to GMM-DEREGISTERED.PLMN-SEARCH (see subclause 4.2.4.1.2) in order to perform a PLMN selection according to 3GPP TS 23.122 [14].

...

9.2.1.2.15.3	Test description

9.2.1.2.15.3.1	Pre-test conditions

System Simulator:

-	cell A;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (only active when stated):

-	same PLMN like visited PLMN above;

-	RAI-1 (RAC & LAC values chosen by SS);

-	System information indicates that NMO 1 is used;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (only active when stated;

-	same PLMN like visited PLMN above;

-	RAC-1 (RAC & LAC values chosen by SS);

-	System information indicates that NMO 1 is used;

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach.

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.2.15.3.2	Test procedure sequence

Table 9.2.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS reconfigures: Cell A as "Serving cell", Cell 5 or Cell 24 as "Non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is powered on or switched on. | - | - | - | - |
| 3 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 4 | Wait for 25s to ensure that T3410 and T3411 expire and the UE releases locally the NAS signalling connection. NOTE 1: The attach attempt counter is 1. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 6 | Wait for 25s to ensure that T3410 and T3411 expire and the UE releases locally the NAS signalling connection. NOTE 2: The attach attempt counter is 2. | - | - | - | - |
| 7 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 8 | Wait for 25s to ensure that T3410 and T3411 expire and the UE releases locally the NAS signalling connection. NOTE 3: The attach attempt counter is 3. | - | - | - | - |
| 9 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 10 | Wait for 25s to ensure that T3410 and T3411 expire and the UE releases locally the NAS signalling connection. NOTE 4: The attach attempt counter is 4. | - | - | - | - |
| 11 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 12 | Wait for 15s to ensure that T3410 expire and the UE releases locally the NAS signalling connection. NOTE 5: The attach attempt counter is 5. | - | - | - | - |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 13a1 | The SS reconfigures: Cell A as "Non-suitable cell", Cell 5 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) as "Serving cell". | - | - | - | - |
| 13a2 | Check: Does the UE transmit an ATTACH REQUEST message without P-TMSI, P-TMSI signature, RAI, TMSI on Cell 5 or Cell 24? | --> | ATTACH REQUEST | 3 | P |
| 13A1 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
|  | EXCEPTION: Step 13Aa1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 13Aa1 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message | --> | DETACH REQUEST | - | - |
| 14 | The SS reconfigures: Cell A as "Serving cell", Cell 5 or Cell 24 as "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 15 | Void | - | - | - | - |
| 16 | The UE is brought back to operation or the USIM is inserted. NOTE 6: The attach attempt counter is reset. | - | - | - | - |
| 17-26 | Void | - | - | - | - |
| 27 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST? | --> | ATTACH REQUEST | 2 | P |
| 28-41b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.1.2.15.3.3	Specific message contents

Table 9.2.1.2.15.3.3-1: Message ATTACH REQUEST (step 13a2, Table 9.2.1.2.15.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | '011'B | Combined GPRS/IMSI attach |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | '0'B | no valid TMSI available |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.1.2.15.3.3-2: Message ATTACH REQUEST (step 27, Table 9.2.1.2.15.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | '0'B | no valid TMSI available |  |

### 9.2.2	Detach procedure

#### 9.2.2.1	UE initiated detach procedure

##### 9.2.2.1.1	UE initiated detach / UE switched off

9.2.2.1.1.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { the UE is switched off }

then { the UE establishes the RRC connection with the RRC establishmentCause set to ‘mo-Signalling’ and sends DETACH REQUEST message, keeps the native security context, and deactivates the EPS bearer context(s) locally }

}

9.2.2.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.1.1, 5.5.2.2.1, 5.5.2.2.2 and Annex D and TS 36.331 clause 5.3.3.3.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a NAS signalling connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

[TS 24.301 clause 5.5.2.2.1]

The detach procedure is initiated by the UE by sending a DETACH REQUEST message. The Detach type IE included in the message indicates whether detach is due to a "switch off" or not. The Detach type IE also indicates whether the detach is for EPS services only, for non-EPS services only, or for both. If the UE has a mapped EPS security context as the current EPS security context, the UE shall set the type of security context flag to "mapped security context". Otherwise, the UE shall set the type of security context flag to "native security context".

...

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DETACH REQUEST message. During this period, the UE may be switched off as soon as the DETACH REQUEST message has been sent.

After the last DETACH REQUEST message is sent, the UE shall proceed as follows:

-	if the current EPS security context is a native EPS security context, then the UE shall store the current EPS security context as specified in annex C and mark it as valid;

-	else if the current EPS security context is a mapped EPS security context and a non-current full native EPS security context exists, then the UE shall store the non-current EPS security context as specified in annex C and mark it as valid, and finally the UE shall delete any mapped EPS security context or partial native EPS security context.

[TS 24.301 clause 5.5.2.2.2]

When the DETACH REQUEST message is received by the network, the network shall send a DETACH ACCEPT message to the UE and store the current EPS security context, if the Detach type IE does not indicate "switch off". Otherwise, the procedure is completed when the network receives the DETACH REQUEST message. On reception of a DETACH REQUEST message indicating "switch off", the MME shall delete the current EPS security context, if it is a mapped EPS security context.

The network and the UE shall deactivate the EPS bearer context(s) for this UE locally without peer-to-peer signalling between the UE and the MME.

The UE, when receiving the DETACH ACCEPT message, shall stop timer T3421.

The UE is marked as inactive in the network for EPS services. State EMM-DEREGISTERED is entered in the network.

The UE in PS mode of operation shall enter the EMM-DEREGISTERED state.

The UE in CS/PS mode 1 or CS/PS mode 2 of operation shall set the update status to U2 NOT UPDATED, disable E-UTRAN and select GERAN or UTRAN access technology and enter the EMM-NULL state.

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Detach | MO signalling (See Note 1) | "originating signalling" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

...

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

9.2.2.1.1.3	Test description

9.2.2.1.1.3.1	Pre-test conditions

System Simulator:

-	cell A (HPLMN).

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (State 2) according to TS 36.508 [18].

9.2.2.1.1.3.2	Test procedure sequence

Table 9.2.2.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause switch off | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message with establishmentCause set to ‘mo-Signalling’ followed by a DETACH REQUEST with the Detach Type IE indicating "switch off"? | --> | DETACH REQUEST | 1 | P |
| 2A | The SS releases the RRC connection. | - | - | - | - |
| 3-7 | Void | - | - | - | - |
| 8 | The UE is switched on | - | - | - | - |
| 9 | Check: Does the UE transmit ATTACH REQUEST message using KSIASME of the previously allocated EPS native security context? | --> | ATTACH REQUEST | 1 | P |
| 10-23b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.2.1.1.3.3	Specific message contents

Table 9.2.2.1.1.3.3-1: Message RRCConnectionRequest (step 2, Table 9.2.2.1.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.2.1.1.3.3-2: Message ATTACH REQUEST (step 9, Table 9.2.2.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 (Security protected NAS message) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| TSC | native security context (for KSIASME) |  |  |

##### 9.2.2.1.2	UE initiated detach / USIM removed from the UE

9.2.2.1.2.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED }

ensure that {

when { the USIM is removed from the UE }

then { the UE sends DETACH REQUEST message and indicates that the detach is for both EPS services and non-EPS services or for EPS services depending on the EPS attach type used }

}

9.2.2.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.2.2.1 and 5.5.2.2.3.

[TS 24.301 clause 5.5.2.2.1]

The detach procedure is initiated by the UE by sending a DETACH REQUEST message. The Detach type IE included in the message indicates whether detach is due to a "switch off" or not. The Detach type IE also indicates whether the detach is for EPS services only, for non-EPS services only, or for both. If the UE has a mapped EPS security context as the current EPS security context, the UE shall set the type of security context flag to "mapped security context". Otherwise, the UE shall set the type of security context flag to "native security context".

If the UE has a valid GUTI, the UE shall populate the GUTI or IMSI IE with the valid GUTI. If the UE does not have a valid GUTI, the UE populates the GUTI or IMSI IE with its IMSI.

If the detach is not due to switch off and the UE is in the state EMM-REGISTERED or EMM-REGISTERED-INITIATED, timer T3421 shall be started in the UE after the DETACH REQUEST message has been sent. If the detach type indicates that the detach is for non-EPS services only the UE shall enter the state EMM-REGISTERED.IMSI-DETACH-INITIATED, otherwise the UE shall enter the state EMM-DEREGISTERED-INITIATED. If the detach type indicates that the detach is for non-EPS services or both EPS and non-EPS services, the UE shall enter the state MM IMSI DETACH PENDING.

[TS 24.301 clause 5.5.2.2.3]

When the DETACH REQUEST message is received by the network, a DETACH ACCEPT message shall be sent to the UE, if the Detach type IE value indicates that the detach request has not been sent due to switching off. Depending on the value of the Detach type IE the following applies:

-	combined EPS/IMSI detach:

The UE is marked as inactive in the network for EPS and for non-EPS services. The states EMM-DEREGISTERED and MM-NULL are entered in both the UE and the network.

9.2.2.1.2.3	Test description

9.2.2.1.2.3.1	Pre-test conditions

System Simulator:

-	cell A (HPLMN).

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (State 2) according to TS 36.508 [18]

9.2.2.1.2.3.2	Test procedure sequence

Table 9.2.2.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause removal of USIM from the UE without powering down | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. |  |  |  |  |
| 2 | Check: Does the UE transmit a DETACH REQUEST with the Detach Type IE indicating "normal detach" or “power off” detach and "combined EPS/IMSI detach" or "EPS detach" depending on the UE configuration? | --> | DETACH REQUEST | 1 | P |
| - | EXCEPTION: Step 2a describe behaviour that depends on the UE implementation | - | - | - | - |
| 2a | If in step 2 normal detach is performed SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 3 | Check: Does the test result of generic procedure in TS 36.508 subclause 6.4.2.5 indicate that the UE does not respond to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |

9.2.2.1.2.3.3	Specific message contents

Table 9.2.2.1.2.3.3-1: DETACH REQUEST (step 2, Table 9.2.2.1.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | EPS detach | EPSOnlyAttach |
|  | 011 | combined EPS/IMSI detach | CombinedAttach |
| Switch off | Not checked |  |  |
| GUTI or IMSI | GUTI-1 |  |  |

##### 9.2.2.1.3	UE initiated detach / EPS capability of the UE is disabled

9.2.2.1.3.1	Test Purpose (TP)

(1)

with { CS/PS mode 1 UE or CS/PS mode 2 UE in EMM-REGISTERED state }

ensure that {

when { EPS capability of the UE is disabled by disabling it's EPS services }

then { UE sends the DETACH REQUEST message on the cell registered and indicates that the detach is for EPS services }

}

(2)

with { CS/PS mode 1 UE or CS/PS mode 2 UE }

ensure that {

when { EPS capability of the UE is disabled by disabling it's EPS services and the UE detaches from EPS services }

then { UE selects GERAN or UTRAN radio access technology indicating E-UTRA not supported in order to enable the UE to continue to obtain CS services }

}

9.2.2.1.3.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.301 clauses 5.5.2.1, 5.5.2.2.1, 5.5.2.2.2, 3.1 and 4.5, Unless otherwise stated these are Rel-8 requirements.

[TS 24.301, clause 5.5.2.1]

The detach procedure with appropriate detach type shall be invoked by the UE if the UE is switched off, the USIM card is removed from the UE or the UE has disabled EPS services or the UE wishes to detach for non-EPS services.

If the detach procedure for EPS services is performed, the EPS bearer context(s) for this particular UE are deactivated locally without peer-to-peer signalling between the UE and the MME.

Upon successful completion of the detach procedure, if the UE and the MME enter the EMM-DEREGISTERED state, the UE and the MME shall delete any mapped EPS security context or partial native EPS security context.

If the UE supports A/Gb mode or Iu mode, the UE shall store the TIN in the non-volatile memory in the ME, as described in annex C, for a subsequent attach procedure.

[TS 24.301, clause 5.5.2.2.1]

The detach procedure is initiated by the UE by sending a DETACH REQUEST message. The Detach type IE included in the message indicates whether detach is due to a "switch off" or not. The Detach type IE also indicates whether the detach is for EPS services only, for non-EPS services only, or for both. If the UE has a mapped EPS security context as the current EPS security context, the UE shall set the type of security context flag to "mapped security context". Otherwise, the UE shall set the type of security context flag to "native security context".

If the UE has a valid GUTI, the UE shall populate the GUTI or IMSI IE with the valid GUTI. If the UE does not have a valid GUTI, the UE populates the GUTI or IMSI IE with its IMSI.

If the detach is not due to switch off and the UE is in the state EMM-REGISTERED or EMM-REGISTERED-INITIATED, timer T3421 shall be started in the UE after the DETACH REQUEST message has been sent. If the detach type indicates that the detach is for non-EPS services only the UE shall enter the state EMM-REGISTERED.IMSI-DETACH-INITIATED, otherwise the UE shall enter the state EMM-DEREGISTERED-INITIATED. If the detach type indicates that the detach is for non-EPS services or both EPS and non-EPS services, the UE shall enter the state MM IMSI DETACH PENDING.

[TS 24.301, clause 5.5.2.2.2]

The network and the UE shall deactivate the EPS bearer context(s) for this UE locally without peer-to-peer signalling between the UE and the MME.

The UE, when receiving the DETACH ACCEPT message, shall stop timer T3421.

The UE is marked as inactive in the network for EPS services. State EMM-DEREGISTERED is entered in the network.

The UE in PS mode of operation shall enter the EMM-DEREGISTERED state.

The UE in CS/PS mode 1 or CS/PS mode 2 of operation shall set the update status to U2 NOT UPDATED, disable E-UTRAN and select GERAN or UTRAN access technology and enter the EMM-NULL state.

[TS 24.301, clause 3.1]

EPS services: Within the context of this specification, EPS services is used as a synonym for GPRS services in 3GPP TS 24.008 [13].

[TS 24.301, clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN; and

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability.

NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

9.2.2.1.3.3	Test description

9.2.2.1.3.3.1	Pre-test conditions

System Simulator:

-	Cell A.

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (home PLMN, RAI-1, NMO 1) is set to "Suitable cell";

-	 if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (home PLMN, RAI-1, NMO 1) is set to "Suitable cell";

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.2.1.3.3.2	Test procedure sequence

Table 9.2.2.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause UE to disable the EPS services. | - | - | - | - |
| 2 | Check: Does the UE transmit a DETACH REQUEST message with the Detach type IE indicating “EPS detach” and “normal detach”. | --> | DETACH REQUEST | 1 | P |
| 3 | The SS transmits a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 3A | The SS releases the RRC connection. | - | - | - | - |
| 4 | Void | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5b5A describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 5a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN | - | - | - | - |
| - | The following messages are to be observed on Cell 5 unless explicitly stated otherwise. | - | - | - | - |
| 5a2 | Check: Does the UE send RRC CONNECTION REQUEST indicating that it does not support E-UTRA? | --> | - | 2 | P |
| 5a3 | The SS sends RRC CONNECTION SETUP | <-- | - | - | - |
| 5a4 | Check: Does the UE send RRC CONNECTION SETUP COMPLETE message indicating that it does not support E-UTRA? | --> | - | 2 | P |
| 5a5 | Check: Does the UE transmit a LOCATION UPDATING REQUEST on Cell 5? | --> | LOCATION UPDATING REQUEST | 2 | P |
| 5a6 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 5a7 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 5a8 | The SS transmits a LOCATION UPDATING ACCEPT message on Cell 5. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 5a9 | The UE transmits an TMSI REALLOCATION COMPLETE message | --> | TMSI REALLOCATION COMPLETE | - | - |
| 5b1 | ELSE IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN | - | - | - | - |
| - | The following messages are to be observed on Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 5b2 | Check: Does the UE transmit a LOCATION UPDATING REQUEST? | --> | LOCATION UPDATING REQUEST | 2 | P |
| - | EXCEPTION: Steps 5b2AAa1 to 5b2AAb1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 5b2AAa1 | IF pc_Dynamic_GERAN_Rel_downgrade=FALSE THEN Check: Does the UE transmit a Classmark Change message indicating that it does not support E-UTRA? | --> | CLASSMARK CHANGE | 2 | P |
| 5b2AAb1 | IF pc_Dynamic_GERAN_Rel_downgrade=TRUE THEN the UE transmit a Classmark Change message. | --> | CLASSMARK CHANGE | - | - |
| 5b2A | Void | - | - | - | - |
| - | EXCEPTION: Step 5b2B describes behaviour that depends on UE capability. | - | - | - | - |
| 5b2B | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 5b3 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 5b4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 5b5 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 5b5A | The UE transmits an TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 5b6 | Void | - | - | - | - |

Table: 9.2.2.1.3.3.2-2: Void

9.2.2.1.3.3.3	Specific message contents

Table 9.2.2.1.3.3.3-1: DETACH REQUEST (steps 2 in Table 9.2.2.1.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | EPS detach only |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

Table 9.2.2.1.3.3.3-2: Message RRC Connection Request (step 5a2 in Table 9.2.2.1.3.3.2-1)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info |  | Optional IE, but if transmitted it should have the below values |  |
| > Support of E-UTRA FDD | False | E-UTRA FDD not supported |  |
| > Support of E-UTRA TDD | False | E-UTRA TDD not supported |  |

Table 9.2.2.1.3.3.3-3: Message RRC Connection Setup Complete (step 5a4 in Table 9.2.2.1.3.3.2-1)

| Derivation path: 34.108 sub-clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio access capability |  | The UE shall not indicate support for E-UTRAN |  |
| >UE multi-mode/multi-RAT capability |  |  |  |
| >> Multi-RAT capability |  |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA FDD | Not present |  |  |
| >>> Support of E-UTRA TDD | Not present |  |  |
| >>> Support of Inter-RAT PS Handover to E-UTRA TDD | Not present |  |  |
| >>> EUTRA Feature Group Indicators | Not checked |  |  |

Table 9.2.2.1.3.3.3-4: CLASSMARK CHANGE (step 5b2A in Table 9.2.2.1.3.3.2-1)

| Derivation path: 44.018 sub-clause 9.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Mobile Station Classmark |  | CM3 bit is set to 1 |  |
| Additional Mobile Station Classmark Information |  | Mobile Station Classmark 3 |  |

Table 9.2.2.1.3.3.3-5: Mobile Station Classmark 3 (Table 9.2.2.1.3.3.3-4)

| Derivation path: 24.008 sub-clause 10.5.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ... |  | Only the following values should be checked (Note 1) |  |
| E-UTRA FDD support | '0'B | Not supported | pc_Dynamic_GERAN_Rel_downgrade=FALSE |
|  | Not present |  | pc_Dynamic_GERAN_Rel_downgrade=TRUE |
| E-UTRA TDD support | '0'B | Not supported | pc_Dynamic_GERAN_Rel_downgrade=FALSE |
|  | Not present |  | pc_Dynamic_GERAN_Rel_downgrade=TRUE |
| Note 1: When EPS capability is disabled, a UE may dynamically switch its GERAN Release to a lower release than GERAN Rel-8, and hence any Classmark 3 structures relevant for Rel-8 and above (inc. E-UTRA FDD support and E-UTRA FDD support) may not be present. |  |  |  |

##### 9.2.2.1.4	UE initiated detach / detach for non-EPS services

9.2.2.1.4.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { UE is detached for non-EPS services }

then { UE sends the DETACH REQUEST message on the cell registered and indicates a IMSI detach }

}

9.2.2.1.4.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.301 clauses 5.5.2.1, 5.5.2.2.1 and 5.5.2.2.3.

[TS 24.301, clause 5.5.2.1]

…

The detach procedure with appropriate detach type shall be invoked by the UE if the UE is switched off, the USIM card is removed from the UE or the UE has disabled EPS services or the UE wishes to detach for non-EPS services.

[TS 24.301, clause 5.5.2.2.1]

The detach procedure is initiated by the UE by sending a DETACH REQUEST message (see example in figure 5.5.2.2.1.1). The Detach type IE included in the message indicates whether detach is due to a "switch off" or not. The Detach type IE also indicates whether the detach is for EPS services only, for non-EPS services only, or for both. If the UE has a mapped EPS security context as the current EPS security context, the UE shall set the type of security context flag to "mapped security context". Otherwise, the UE shall set the type of security context flag to "native security context".

If the UE has a valid GUTI, the UE shall populate the GUTI or IMSI IE with the valid GUTI. If the UE does not have a valid GUTI, the UE populates the GUTI or IMSI IE with its IMSI.

If the detach is not due to switch off and the UE is in the state EMM-REGISTERED or EMM-REGISTERED-INITIATED, timer T3421 shall be started in the UE after the DETACH REQUEST message has been sent. If the detach type indicates that the detach is for non-EPS services only the UE shall enter the state EMM-REGISTERED.IMSI-DETACH-INITIATED, otherwise the UE shall enter the state EMM-DEREGISTERED-INITIATED. If the detach type indicates that the detach is for non-EPS services or both EPS and non-EPS services, the UE shall enter the state MM IMSI DETACH PENDING.

[TS 24.301, clause 5.5.2.2.3]

When the DETACH REQUEST message is received by the network, a DETACH ACCEPT message shall be sent to the UE, if the Detach type IE value indicates that the detach request has not been sent due to switching off. Depending on the value of the Detach type IE the following applies:

-	combined EPS/IMSI detach:

The UE is marked as inactive in the network for EPS and for non-EPS services. The states EMM-DEREGISTERED and MM-NULL are entered in both the UE and the network.

-	IMSI detach:

The UE is marked as inactive in the network for non-EPS services. The states MM-NULL and EMM-REGISTERED are entered in both the UE and the network.

9.2.2.1.4.3	Test description

9.2.2.1.4.3.1	Pre-test conditions

System Simulator:

-	Cell A is set to ''Serving cell'';

-	Cell B is set to ''Non-Suitable cell'

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.2.1.4.3.2	Test procedure sequence

Table 9.2.2.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause UE to initiate detach for non-EPS services. | - | - | - | - |
| 2 | Check: Does the UE transmit a DETACH REQUEST message with the Detach type IE indicating “IMSI detach” and “normal detach”. | --> | DETACH REQUEST | 1 | P |
| 3 | The SS responds with DETACH ACCEPT message. | - | DETACH ACCEPT | - | - |
| 3A | The SS releases the RRC Connection. | - | - | - | - |
| 4 | Set the cell type of cell A to the "Non-Suitable cell ". Set the cell type of cell B to the " Serving cell" | - | - | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 6 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.2.1.4.3.3	Specific message contents

Table 9.2.2.1.4.3.3-1: DETACH REQUEST (step 2 in Table 9.2.2.1.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 010 | IMSI detach |  |
| Switch off | 0 | normal detach |  |

Table 9.2.2.1.4.3.3-2: TRACKING AREA UPDATE REQUEST (step 5 in Table 9.2.2.1.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '000'B | TA updating |  |

##### 9.2.2.1.5	Void

##### 9.2.2.1.6	UE initiated detach / Abnormal case / Local detach after 5 attempts due to no network response

9.2.2.1.6.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { the UE receives no response to the UE initiated DETACH REQUEST }

then { the UE re-transmits the DETACH REQUEST up to 4 times on the expiry of timer T3421 }

}

(2)

with { UE in EMM-REGISTERED state }

ensure that {

when { the UE receives no response to the UE initiated DETACH REQUEST }

then { the UE aborts the detach procedure and perform local detach on the 5th expiry of timer T3421 }

}

9.2.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.2.2.

[TS 24.301, clause 5.5.2.2.1]

The detach procedure is initiated by the UE by sending a DETACH REQUEST message. The Detach type IE included in the message indicates whether detach is due to a "switch off" or not. The Detach type IE also indicates whether the detach is for EPS services only, for non-EPS services only, or for both. If the UE has a mapped EPS security context as the current EPS security context, the UE shall set the type of security context flag to "mapped security context". Otherwise, the UE shall set the type of security context flag to "native security context".

If the UE has a valid GUTI, the UE shall populate the GUTI or IMSI IE with the valid GUTI. If the UE does not have a valid GUTI, the UE populates the GUTI or IMSI IE with its IMSI.

If the detach is not due to switch off and the UE is in the state EMM-REGISTERED or EMM-REGISTERED-INITIATED, timer T3421 shall be started in the UE after the DETACH REQUEST message has been sent. If the detach type indicates that the detach is for non-EPS services only the UE shall enter the state EMM-REGISTERED.IMSI-DETACH-INITIATED, otherwise the UE shall enter the state EMM-DEREGISTERED-INITIATED. If the detach type indicates that the detach is for non-EPS services or both EPS and non-EPS services, the UE shall enter the state MM IMSI DETACH PENDING.

[TS 24.301, clause 5.5.2.2.4 c)]

The following abnormal cases can be identified:

…

c)	T3421 timeout

On the first four expiries of the timer, the UE shall retransmit the DETACH REQUEST message and shall reset and restart timer T3421. On the fifth expiry of timer T3421, the detach procedure shall be aborted and the UE shall change to state:

-	EMM-REGISTERED.NORMAL-SERVICE and MM-NULL if "IMSI detach" was requested;

-	EMM-DEREGISTERED if "EPS detach" was requested;

-	EMM-DEREGISTERED and MM-NULL if "combined EPS/IMSI detach" was requested.

[TS 24.301, clause 5.1.3.2.2.2]

In the state EMM-DEREGISTERED, no EMM context has been established and the UE location is unknown to an MME and hence it is unreachable by an MME. In order to establish an EMM context, the UE shall start the attach or combined attach procedure (see subclause 5.5.1).

9.2.2.1.6.3	Test description

9.2.2.1.6.3.1	Pre-test conditions

System Simulator:

-	cell A (HPLMN).

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (State 2) according to TS 36.508 [18].

9.2.2.1.6.3.2	Test procedure sequence

Table 9.2.2.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause UE to initiate detach. | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. |  |  |  |  |
| 2 | The UE transmits a DETACH REQUEST message? The UE starts timer T3421. | --> | DETACH REQUEST | - | - |
| 3 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 4 | Check: When the timer T3421 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (1st expiry). | --> | DETACH REQUEST | 1 | P |
| 5 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 6 | Check: When the timer T3421 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (2nd expiry). | --> | DETACH REQUEST | 1 | P |
| 7 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 8 | Check: When the timer T3421 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (3rd expiry). | --> | DETACH REQUEST | 1 | P |
| 9 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 10 | Check: When the timer T3421 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (4th expiry). | --> | DETACH REQUEST | 1 | P |
| 11 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 12 | When the timer T3421 expires the UE aborts the detach procedure and performs a local detach (5th expiry). | - | - | 2 | P |
| 13 | The SS starts the EPS bearer context modification procedure using the previously allocated EPS bearer identity | <-- | MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 14 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message within the next 10s? | --> | MODIFY EPS BEARER CONTEXT ACCEPT | 2 | F |
| 15 | Void | - | - | - | - |
| 16a1 | Void | - | - | - | - |
| Note: T3421 value is specified as 15s in TS 24.301. If the UE supports CE mode B and UEs usage setting is not voice centric, T3421 takes a value of 45s (see TS 24.301 Table 10.2.1) |  |  |  |  |  |

9.2.2.1.6.3.3	Specific message contents

Table 9.2.2.1.6.3.3-1: DETACH REQUEST (steps 2, 4, 6, 8 and 10 Table 9.2.2.1.6.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

Table 9.2.2.1.6.3.3-2:  Void

##### 9.2.2.1.7	UE initiated detach / Abnormal case / Detach procedure collision

9.2.2.1.7.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED-INITIATED state }

ensure that {

when { the UE receives the DETACH REQUEST message from the network }

then { the UE aborts the UE initiated detach procedure and completes the network initiated detach procedure }

}

(2)

with { UE in EMM-DEREGISTERED state }

ensure that {

when { the UE has completed the network initiated detach procedure with re-attach required and the UE supports re-attach after DETACH collision }

then { the UE initiates the attach procedure }

}

(3)

with { UE in EMM-DEREGISTERED state }

ensure that {

when { the UE has completed the network initiated detach procedure with re-attach required and the UE does not support re-attach after DETACH collision }

then { the UE does not initiate the attach procedure }

}

9.2.2.1.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.2.2.4 and 5.5.2.3.2.

[TS 24.301, clause 5.5.2.2.4]

The following abnormal cases can be identified:

…

d)	Detach procedure collision

If the UE receives a DETACH REQUEST message before the UE initiated detach procedure has been completed, it shall treat the message as specified in subclause 5.5.2.3.2 with the following modification: if the DETACH REQUEST message received by the UE contains detach type "re-attach required", and the UE initiated detach procedure is with detach type "EPS detach" or "combined EPS/IMSI detach", the UE need not initiate the attach or combined attach procedure.

[TS 24.301, clause 5.5.2.3.2]

When receiving the DETACH REQUEST message and the detach type indicates "re-attach required", the UE shall deactivate the EPS bearer context(s) including the default EPS bearer context locally without peer-to-peer signalling between the UE and the MME. The UE shall then send a DETACH ACCEPT message to the network and enter state EMM-DEREGISTERED. Furthermore, the UE shall, after the completion of the detach procedure, and the existing NAS signalling connection has been released, initiate an attach or combined attach procedure.

NOTE 1:	When detach type indicates "re-attach required", user interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

9.2.2.1.7.3	Test description

9.2.2.1.7.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (State 2) according to TS 36.508 [18].

9.2.2.1.7.3.2	Test procedure sequence

Table 9.2.2.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause UE to initiate detach. | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 2 | The UE transmits a DETACH REQUEST message. The UE starts timer T3421. | --> | DETACH REQUEST | - | - |
| 3 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 4 | With T3421 still running the SS shall send DETACH REQUEST message with the Detach type IE indicating "re-attach required". | <-- | DETACH REQUEST | - | - |
| 5 | Check: Does the UE transmit a DETACH ACCEPT message? | --> | DETACH ACCEPT | 1 | P |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 7a1-7b1 describe a behaviour which depends on the UE capability | - | - | - | - |
| 7a1 | Void |  |  |  |  |
| 7a2 | IF pc_Re_Attach_AfterDetachColl THEN Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 7a3-7a14 | The attach procedure is completed by executing steps 5 to 16 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| 7a15 | Cause UE to initiate detach. | - | - | - | - |
| 7a15A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 7a16 | The UE transmits a DETACH REQUEST with the Detach Type IE indicating "normal detach" and "combined EPS/IMSI detach" or "EPS detach" depending on the UE configuration. | --> | DETACH REQUEST | - | - |
| 7a17 | The SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 7a18 | The SS releases the RRC connection | - | - | - | - |
| 7b1 | ELSE Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message in the next 30s? | --> | ATTACH REQUEST | 3 | F |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

9.2.2.1.7.3.3	Specific message contents

Table 9.2.2.1.7.3.3-1: DETACH REQUEST (steps 2 and 7a16 Table 9.2.2.1.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

Table 9.2.2.1.7.3.3-2: DETACH REQUEST (step 4 Table 9.2.2.1.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | re-attach required |  |

Table 9.2.2.1.7.3.3-3: Message ATTACH REQUEST (step 7a2, Table 9.2.2.1.7.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach type | 0001 or 0010 |  | EPS attach or combined EPS/IMSI attach |  |  |

##### 9.2.2.1.8	UE initiated detach / Abnormal case / Detach and EMM common procedure collision

9.2.2.1.8.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED-INITIATED state due to switch off }

ensure that {

when { the UE receives an EMM common procedure message from the network }

then { the UE ignores the message and continues the detach procedure }

}

(2)

with { UE in EMM-DEREGISTERED-INITIATED state due to normal detach }

ensure that {

when { the UE receives GUTI REALLOCATION COMMAND from the network }

then { the UE ignores the message and continues the detach procedure }

}

(3)

with { UE in EMM-DEREGISTERED-INITIATED state due to normal detach }

ensure that {

when { the UE receives AUTHENTICATION REQUEST, SECURITY MODE COMMAND or IDENTITY REQUEST message from the network }

then { the UE responds to the message and then continues the detach procedure }

}

9.2.2.1.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.2.2.4.

[TS 24.301, clause 5.5.2.2.4]

The following abnormal cases can be identified:

…

e)	Detach and EMM common procedure collision

Detach containing cause "switch off":

-	If the UE receives a message used in an EMM common procedure before the detach procedure has been completed, this message shall be ignored and the detach procedure shall continue

Detach containing other causes than "switch off":

-	If the UE receives a GUTI REALLOCATION COMMAND, an EMM STATUS or an EMM INFORMATION message before the detach procedure is completed, this message shall be ignored and the detach procedure shall continue.

-	If the UE receives an AUTHENTICATION REQUEST, SECURITY MODE COMMAND or IDENTITY REQUEST message before the detach procedure has been completed, the UE shall respond to it as described in subclause 5.4.2, 5.4.3 and 5.4.4 respectively and the detach procedure shall continue.

9.2.2.1.8.3	Test description

9.2.2.1.8.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (State 2) according to TS 36.508[18].

9.2.2.1.8.3.2	Test procedure sequence

Table 9.2.2.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause switch off. | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 2 | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “switch off”. | --> | DETACH REQUEST | - | - |
| 3 | The SS transmits a GUTI REALLOCATION COMMAND message. | <-- | GUTI REALLOCATION COMMAND | - | - |
| 4 | Check: Does the UE transmit an GUTI REALLOCATION COMPLETE message? | --> | GUTI REALLOCATION COMPLETE | 1 | F |
| 5 | Check: Does the test result of the generic procedure in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
| 6 | The UE is switched on and performs an ATTACH procedure. | - | - | - | - |
| 7 | Cause UE to initiate detach. | - | - | - | - |
| 7A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 8 | The UE transmits a DETACH REQUEST message? ((The UE starts timer T3421) | --> | DETACH REQUEST | 2 | P |
| 9 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 10 | With T3421 still running the SS shall send GUTI REALLOCATION COMMAND. | <-- | GUTI REALLOCATION COMMAND | - | - |
| 11 | Check: Does the UE transmit a GUTI REALLOCATION COMPLETE message? | --> | GUTI REALLOCATION COMPLETE | 2 | F |
| 12 | SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Cause UE to initiate attach. The generic procedure in 36.508 clause 4.5.2.3-1 is executed so as to complete the attach procedure (Note). | - | - | - | - |
| 15 | Cause UE to initiate detach. | - | - | - | - |
| 15A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 16 | The UE transmits a DETACH REQUEST message? (The UE starts timer T3421. ) | --> | DETACH REQUEST | - | - |
| 17 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 18 | With T3421 still running the SS shall send AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 19 | Check: Does the UE transmit an AUTHENTICATION RESPONSE message? | --> | AUTHENTICATION RESPONSE | 3 | P |
| 20 | SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 21 | The SS releases the RRC connection. | - | - | - | - |
| 22 | Cause UE to initiate attach. The generic procedure in 36.508 clause 4.5.2.3-1 is executed so as to complete the attach procedure (Note). | - | - | - | - |
| 23 | Cause UE to initiate detach. | - | - | - | - |
| 23A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 24 | The UE transmits a DETACH REQUEST message? (The UE starts timer T3421. ) | --> | DETACH REQUEST | - | - |
| 25 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 26 | With T3421 still running the SS shall send SECURITY MODE COMMAND. | <-- | SECURITY MODE COMMAND | - | - |
| 27 | Check: Does the UE transmit a SECURITY MODE COMPLETE message? | --> | SECURITY MODE COMPLETE | 3 | P |
| 28 | SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 29 | The SS releases the RRC connection. | - | - | - | - |
| 30 | Cause UE to initiate attach. The generic procedure in 36.508 clause 4.5.2.3-1 is executed so as to complete the attach procedure (Note). | - | - | - | - |
| 31 | Cause UE to initiate detach. | - | - | - | - |
| 31A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 32 | The UE transmits a DETACH REQUEST message? (The UE starts timer T3421) | --> | DETACH REQUEST | - | - |
| 33 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 34 | With T3421 still running the SS shall send IDENTITY REQUEST. | <-- | IDENTITY REQUEST | - | - |
| 35 | Check: Does the UE transmit an IDENTITY RESPONSE message? | --> | IDENTITY RESPONSE | 3 | P |
| 36 | SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 37 | The SS releases the RRC connection. | - | - | - | - |
| Note: The request is triggered by either of the following AT commands, depending on UE capability: - IF pc_KeepEpsBearerParametersAfterNormalDetach THEN AT command AT+CGATT=1, or - ELSE AT command AT+CGDCONT=1,”IP” followed by AT+CGACT=1 |  |  |  |  |  |

9.2.2.1.8.3.3	Specific message contents

Table 9.2.2.1.8.3.3-1: DETACH REQUEST (steps 8, 16, 24 and 32 Table 9.2.2.1.8.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |

Table 9.2.2.1.8.3.3-2: GUTI REALLOCATION COMMAND (step 3 Table 9.2.2.1.8.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list | Not present |  |  |

Table 9.2.2.1.8.3.3-3: GUTI REALLOCATION COMMAND (step 10 Table 9.2.2.1.8.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-3 |  |  |
| TAI list | Not present |  |  |

Table 9.2.2.1.8.3.3-4: Message ATTACH REQUEST (steps 14, 22 and 30, Table 9.2.2.1.8.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach type | 0001 or 0010 |  | EPS attach or combined EPS/IMSI attach |  |  |

##### 9.2.2.1.9	UE initiated detach / Abnormal case / Change of cell into a new tracking area

9.2.2.1.9.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED-INITIATED state }

ensure that {

when { the UE changes into a new tracking area that is not in the stored TAI list }

then { the UE aborts the detach procedure and initiates a Tracking Area Updating procedure }

}

(2)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {

when { the UE receives TRACKING AREA UPDATE ACCEPT message }

then { the UE re-initiates the detach procedure after completing the Tracking Area Updating procedure }

}

9.2.2.1.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.2.2.4.

[TS 24.301, clause 5.5.2.2.4]

The following abnormal cases can be identified:

…

b)	Lower layer failure or release of the NAS signalling connection before reception of DETACH ACCEPT message

The detach procedure shall be aborted, and the UE shall enter state:

-	EMM-REGISTERED.NORMAL-SERVICE and MM-NULL if "IMSI detach" was requested;

-	EMM-DEREGISTERED if "EPS detach" was requested;

-	EMM-DEREGISTERED and MM-NULL if "combined EPS/IMSI detach" was requested.

…

f)	Change of cell into a new tracking area

If a cell change into a new tracking area that is not in the stored TAI list occurs before the UE initiated detach procedure is completed, the detach procedure shall be aborted and re-initiated after successfully performing a tracking area updating procedure. If the detach procedure was initiated due to removal of the USIM, the UE shall abort the detach procedure and enter the state EMM-DEREGISTERED.

9.2.2.1.9.3	Test description

9.2.2.1.9.3.1	Pre-test conditions

System Simulator:

-	cell A belongs to TAI-1 (home PLMN) is set to ''Serving cell''

-	cell B belongs to TAI-2 (home PLMN) is set to ''Non-Suitable cell''.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) according to TS 36.508 [18].

9.2.2.1.9.3.2	Test procedure sequence

Table 9.2.2.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 1 | Cause UE to initiate detach. | - | - | - | - |
| 1A | UE may optionally perform IMS de-registration using the generic procedure defined in 34.229-1 [35] Annex C.30. | - | - | - | - |
| 2 | The UE transmits a DETACH REQUEST message. (The UE starts timer T3421) | --> | DETACH REQUEST | - | - |
| 3 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 4 | Set the cell type of cell A to the ''Suitable neighbour cell''. Set the cell type of cell B to the ''Serving cell''. Note: T3421 value is specified as 15s in TS 24.301 and it is assumed that SS can configure cells within this time. | - | - | - | - |
| 4A | The SS transmits an RRCConnectionReconfiguration message on Cell A to order the UE to perform intra frequency handover to Cell B. | <-- | - | - | - |
| 4B | The UE transmits an RRCConnectionReconfigurationComplete message on Cell B to confirm the successful completion of the intra frequency handover. | --> | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 6 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 8 | The UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | 2 | P |
| 9 | SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 10 | Check: Does the test result of the generic procedure in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging when paged with S-TMSI included in GUTI-2 and with CN domain indicator set to "PS"? | - | - | 2 | - |

9.2.2.1.9.3.3	Specific message contents

Table 9.2.2.1.9.3.3-1: DETACH REQUEST (step 2, Table 9.2.2.1.9.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

Table 9.2.2.1.9.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 6, Table 9.2.2.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |

Table 9.2.2.1.9.3.3-3: RRCConnectionReconfiguration (step 4a, Table 9.2.2.1.9.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo SEQUENCE { | MobilityControlInfo-HO |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell B (see 36.508 clause 4.6.5) |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell B |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.2.1.9.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 5, Table 9.2.2.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| Active Flag | Any allowed value |  |  |
| EPS bearer context status | Not present or (octet 3 = '00100000'B and octet 4 = '00000000'B) or (octet 3 = '00000000'B and octet 4 = '00000000'B) |  |  |

Table 9.2.2.1.9.3.3-5: DETACH REQUEST (step 8, Table 9.2.2.1.9.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 or 011 | EPS detach or combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-2 |  |  |

##### 9.2.2.1.10	UE initiated detach / Mapped security context

9.2.2.1.10.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { the UE sent the last DETACH REQUEST }

then { the UE delete the mapped security context }

}

9.2.2.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.2.1 and 5.5.3.3.2. Unless otherwise stated these are rel-8 requirements.

[TS 24.301, clause 5.5.2.1]

Upon successful completion of the detach procedure, if the UE and the MME enter the EMM-DEREGISTERED state, the UE and the MME shall delete any mapped EPS security context or partial native EPS security context.

[Rel-12]

[TS 24.301, clause 5.5.3.3.2]

The UE operating in CS/PS mode 1 or CS/PS mode 2, in state EMM-REGISTERED, shall initiate the combined tracking area updating procedure:

…

d)	when the UE performs an intersystem change from A/Gb or Iu mode to S1 mode, and the UE previously either performed a combined GPRS attach procedure, an IMSI attach procedure, a location area updating procedure or a combined routing area updating procedure, in A/Gb or Iu mode, or moved to A/Gb or Iu mode from S1 mode through an SRVCC handover or moved to Iu mode from S1 mode through an vSRVCC handover. In this case the EPS update type IE shall be set to "combined TA/LA updating with IMSI attach";

9.2.2.1.10.3	Test description

9.2.2.1.10.3.1	Pre-test conditions

System Simulator:

-	Cell A and Cell 5.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell;

UE:

None.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.2.1.10.3.2	Test procedure sequence

Table 9.2.2.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U -S | Message |  |  |
| - | The SS configures: - Cell 5 as a "Non-Suitable Off cell". - Cell A as the "Serving cell". | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. | --> | ATTACH REQUEST | - | - |
| 2A-2D | Steps 5 to 8 from procedure 4.5.2.3 in TS 36.508. |  |  |  |  |
| 3 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services and non-EPS services not allowed" as specified. | <-- | ATTACH REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 6 | The SS configures: - Cell A as a "Non-Suitable Off cell". - Cell 5 as the "Serving cell". | - | - | - | - |
| 7 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 8 | The UE transmits an ATTACH REQUEST message on Cell 5 | --> | ATTACH REQUEST |  |  |
| 9 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST |  |  |
| 10 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE |  |  |
| 11 | SS responds with ATTACH ACCEPT message including P-TMSI-1 and RAI-1. | <-- | ATTACH ACCEPT | - | - |
| 12 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 12A | The SS releases the RRC connection. | - | - | - | - |
| 12B | Generic test procedure in TS 36.508 subclause 4.5A.3A.2 is performed | - | - | - | - |
| 13 | The activation of a PDP context is triggered by MMI or AT command. | - | - | - | - |
| 14 | Generic test procedure in TS 34.108 subclause 7.2.4.2.3 is performed on Cell 5. NOTE: The UE performs Radio Bearer Establishment in a UTRAN cell. | - | - | - | - |
| 15 | The SS releases the RRC connection. | - | - | - | - |
| 16 | The SS configures: - Cell A as the "Serving cell". - Cell 5 as a "Non-Suitable cell". | - | - | - | - |
| 17 - 24 | Generic test procedure in TS 36.508 subclause 6.4.2.7A-1 is performed. (step 1 to 8) | - | - | - | - |
| - | Exception: Steps 24A to 24Aa5/24Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 24A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 24Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - | - |
| 24Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 24Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 24Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 24Aa5 | The SS releases the RRC connection | - | - | - | - |
| 24Ab1 | The 10s timer expires | - | - | - | - |
| 25 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 25 describes behaviour that depends on the UE capability. |  |  |  |  |
| 26 | If pc_SwitchOnOff or pc_USIM_Removal then UE send DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 27 | The SS configures: - Cell A as the "Serving cell". - Cell 5 as a "Non-Suitable Off cell". | - | - | - | - |
| 28 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | The following messages shall be received on cell A. | - | - | - | - |
| 29 | Cause UE to initiate attach Check: Does the UE send ATTACH REQUEST message | --> | ATTACH REQUEST | 1 | P |
| 30-43b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.2.1.10.3.3	Specific message contents

Table 9.2.1.1.10.3.3-1: Message ATTACH REJECT (step 3, Table 9.2.2.1.10.3.2-1)

| Derivation path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00001000 | #8 "EPS services and non-EPS services not allowed" |  |
| ESM message container | Not present |  |  |

Table 9.2.2.1.10.3.3-2: ATTACH REQUEST (step 29, Table 9.2.2.1.10.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| NAS key set identifier | ‘111’ | no key is available |  |

#### 9.2.2.2	Network initiated detach procedure

##### 9.2.2.2.1	NW initiated detach / Re-attach required

9.2.2.2.1.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {
  when { SS sends DETACH REQUEST message with the Detach type IE "re-attach required" }

then { UE sends DETACH ACCEPT message and UE initiates an attach procedure with the current EPS security context}

}

9.2.2.2.1.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.301 clauses 5.5.2.3.2.

[TS 24.301 clause5.5.2.3.2]

When receiving the DETACH REQUEST message and the detach type indicates "re-attach required", the UE shall deactivate the EPS bearer context(s) including the default EPS bearer context locally without peer-to-peer signalling between the UE and the MME. The UE shall then send a DETACH ACCEPT message to the network and enter state EMM-DEREGISTERED. Furthermore, the UE shall, after the completion of the detach procedure, and the existing NAS signalling connection has been released, initiate an attach or combined attach procedure.

NOTE 1:	When detach type indicates "re-attach required", user interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

A UE which receives a DETACH REQUEST message with detach type indicating "re-attach required" or "re-attach not required" and no EMM cause IE, is detached only for EPS services.

...If the detach type indicates "IMSI detach" or "re-attach required" then the UE shall ignore the EMM cause IE if received.

9.2.2.2.1.3	Test description

9.2.2.2.1.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.2.2.2.1.3.2	Test procedure sequence

Table 9.2.2.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS initiates Detach procedure with the Detach Type IE "re-attach required" | <-- | DETACH REQUEST | - | - |
| 2 | Check: Does the UE send DETACH ACCEPT message? | --> | DETACH ACCEPT | 1 | P |
| 3 | The SS releases RRC connection. |  |  |  |  |
| - | EXCEPTION: Step 3a describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a | IF NOT pc_Automatic_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE send ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 4Aa1-4Aa2 | Void | - | - | - | - |
| 4Ba1-4D | Steps 9a1-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 5 | The SS sends ATTACH ACCEPT to assign the new GUTI (GUTI-2). The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 6 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. |  |  |  |  |
| - | EXCEPTION: IF PDN1_IMS THEN in parallel to the event described in step 6 below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE |  |  |  |  |
| 6 | Check: Does the UE send ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 1 | P |
| 7a1 – 9b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |

9.2.2.2.1.3.3	Specific message contents

Table 9.2.2.2.1.3.3-1: Message DETACH REQUEST (step 1, Table 9.2.2.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | '001'B | "re-attach required" |  |

Table 9.2.2.2.1.3.3-2: Message ATTACH REQUEST (step 4, Table 9.2.2.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| TSC | '0'B | native security context |  |
| NAS key set identifier | The value is a same value to be allocated by SS in Preamble. |  |  |
| Old GUTI or IMSI | GUTI-1 |  |  |

NOTE: This message is sent within the SECURITY NAS PROTECTED MESSAGE in Table 9.2.2.2.1.3.3-3.

Table 9.2.2.2.1.3.3-3: Message SECURITY PROTECTED NAS MESSAGE (step 4, Table 9.2.2.2.1.3.2-1)

| Derivation Path: 36.508 Table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Security header type |  | '0001'B | Integrity protected |  |
| NAS message |  | '01000001'B | ''ATTACH REQUEST'' |  |

Table 9.2.2.2.1.3.3-4: Message ATTACH ACCEPT (step 5, Table 9.2.2.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| TAI list |  |  |  |
| Length of tracking area identity list contents | ‘00000110’B | 6 octets |  |
| Number of elements | ‘00000’B | 1 element |  |
| Type of list | ‘00’B | "list of TACs belonging to one PLMN, with non-consecutive TAC values" |  |
| Partial tracking area identity list | TAI-1 |  |  |
| GUTI | GUTI-2 |  |  |

##### 9.2.2.2.2	NW initiated detach / IMSI detach

9.2.2.2.2.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {
  when { UE receives DETACH REQUEST message with the Detach type IE = “IMSI detach” }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type IE = “Combined TA/LA updating with IMSI attach” }

}

(2)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {
  when { UE receives TRACKING AREA UPDATE ACCEPT message }

then { UE enters EMM-REGISTERED and enters MM IDLE and sends TRACKING AREA UPDATE COMPLETE message and re-attaches for non-EPS services }

}

9.2.2.2.2.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.301 clauses 5.5.2.3.2. Unless otherwise stated these are Rel-8 requirements.

[TS 24.301 clause 5.5.2.3.2]

When receiving the DETACH REQUEST message and the Detach type IE indicates "IMSI detach", the UE shall not deactivate the EPS bearer context(s) including the default EPS bearer context. The UE shall set the MM update status to U2 NOT UPDATED. A UE may send a DETACH ACCEPT message to the network, and shall re-attach to non-EPS services by performing the combined tracking area updating procedure according to subclause 5.5.3.3, sending a TRACKING AREA UPDATE REQUEST message with EPS update type IE indicating "combined TA/LA updating with IMSI attach".

9.2.2.2.2.3	Test description

9.2.2.2.2.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.2.2.2.2.3.2	Test procedure sequence

Table 9.2.2.2.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS sends DETACH REQUEST with the Detach Type IE “IMSI detach”. | <-- | DETACH REQUEST | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported.. | - | - | - | - |
| 2 a1 | IF the UE supports indication that the detach procedure has been accepted THEN the UE sends DETACH ACCEPT message. | --> | DETACH ACCEPT | - | - |
| 3 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| - 4-5 | - Void | - - | - - | - - | - - |
| 6 | The SS sends TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | Check: Does the UE send TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 2 | P |
| 8 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 9a1 to 9a6 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 9 a1 | IF pc_CS_fallback THEN SS pages the UE with S-TMSI2 for CS domain. | - | - | - | - |
| - | EXCEPTION: Step 9 a2a1 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 9 a2a1 | IF the UE needs to request upper layer input before accepting the CS fallback (see ICS), THEN the incoming CS call is accepted at the UE through MMI or AT command. | - | - | - | - |
| 9 a3 | Check: Does the UE respond to paging by sending an EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 2 | P |
| 9 a4 | The SS transmits SERVICE REJECT message. | <-- | SERVICE REJECT | - | - |
| 9 a5 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.2.2.2.3.2-2 may occur in parallel with step 9 a6. | - | - | - | - |
| 9 a6 | The SS waits for 60 seconds (T3442 value provided in step 9a4. Note: The value 60 sec is chosen to allow sufficient time if the parallel behaviour occurs to be handled before going to the next step.) | - | - | - | - |
| 10 | Check: Does the UE respond to paging with S-TMSI2 for PS domain? Generic Procedure (TS 36.508 subclause 6.4.2.4) | - | - | 2 | - |

Table 9.2.2.2.2.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |

9.2.2.2.2.3.3	Specific message contents

Table 9.2.2.2.2.3.3-1: Message DETACH REQUEST (step 1, Table 9.2.2.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | ‘011’B | “IMSI detach” |  |
| EMM cause | Not present |  |  |

Table 9.2.2.2.2.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 3, Table 9.2.2.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | ‘010’B | “Combined TA/LA updating with IMSI attach” |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.2.2.2.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 6, Table 9.2.2.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | ‘00000110’B |  |  |
| Partial tracking area identity list 1 |  | 1 element, TAI-1 |  |
| Location area identification | LAI-1 |  |  |
| MS identity | TMSI-1 |  |  |

Table 9.2.2.2.2.3.3-4: Message EXTENDED SERVICE REQUEST (step 9a3, Table 9.2.2.2.2.3.2-1)

| Derivation Path: 36.508 table 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| M-TMSI |  | M-TMSI1 | ''M-TMSI1 is a part of GUTI-1'' |  |

Table 9.2.2.2.2.3.3-5: Message SERVICE REJECT (step 9a4, Table 9.2.2.2.2.3.2-1)

| Derivation Path: 36.508 table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | '0010 0111'B | #39 'CS domain temporarily not available' |  |
| T3442 value |  | '0010 0001'B | 60 sec |  |

Table 9.2.2.2.2.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 2, Table 9.2.2.2.2.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

##### 9.2.2.2.3	Void

##### 9.2.2.2.4	Void

##### 9.2.2.2.5	Void

##### 9.2.2.2.6	Void

##### 9.2.2.2.7	Void

##### 9.2.2.2.8	Void

##### 9.2.2.2.9	Void

##### 9.2.2.2.10	Void

##### 9.2.2.2.11	Void

##### 9.2.2.2.12	Void

##### 9.2.2.2.13	Void

##### 9.2.2.2.14	NW initiated detach / Abnormal case / EMM cause not included

9.2.2.2.14.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state }

ensure that {

when { UE receives a DETACH REQUEST message with the Detach type IE “re-attach not required” and with no EMM cause IE included }

then { UE delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, KSI, set the update status to EU2 NOT UPDATED and start timer T3402. when T3402 expired, the UE attempts to attach again }

}

9.2.2.2.14.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.301 clauses 5.5.2.3.4.

[TS 24.301 clause5.5.2.3.4]

The following abnormal cases can be identified:

…

b)	DETACH REQUEST, other EMM cause values than those treated in subclause 5.5.2.3.2 or no EMM cause IE is included, and the Detach type IE indicates "re-attach not required".

The UE shall delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, KSI, shall set the update status to EU2 NOT UPDATED and shall start timer T3402. The UE may enter the state EMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6]; otherwise the UE shall enter the state EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH.

9.2.2.2.14.3	Test description

9.2.2.2.14.3.1	Pre-test conditions

System Simulator:

-	cell I (VPLMN and set as a serving cell) configured according to table 6.3.2.2-3 in TS 36.508 [18].

UE:

None.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell I according to TS 36.508 [18];

-	T3402 in the UE is set to 2 minutes.

9.2.2.2.14.3.2	Test procedure sequence

Table 9.2.2.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS initiates Detach procedure with the Detach Type IE “re-attach not required” and no EMM cause IE included | <-- | DETACH REQUEST | - | - |
| 2 | The UE transmits a DETACH ACCEPT message on Cell I. Note: Now UE should start timer T3402 | --> | DETACH ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |
| 4 | Check: When the timer T3402 expires does the UE transmit ATTACH REQUEST message on cell I? | --> | ATTACH REQUEST | 1 | P |
| 5-18b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2) according to TS 36.508. | - | - | - | - |

9.2.2.2.14.3.3	Specific message contents

Table 9.2.2.2.14.3.3-1: Message DETACH REQUEST (step 1, Table 9.2.2.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | ‘010’B | “re-attach not required” |  |
| EMM cause | Not present |  |  |

Table 9.2.2.2.14.3.3-2: Message ATTACH REQUEST (step 4, Table 9.2.2.2.14.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI1 |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.2.2.14.3.3-3: Message ATTACH ACCEPT (preamble, 9.2.2.2.14.3.1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3402 value | 2 minute | The default value is 12 minutes, use 2 minute to shorten the whole TC execute time |  |

### 9.2.3	Tracking area updating procedure (S1 mode only)

#### 9.2.3.1	Normal and periodic tracking area updating

##### 9.2.3.1.1	Normal tracking area update / Accepted

9.2.3.1.1.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE detects entering a new tracking area already included in the TAI list }

then { UE does not send TRACKING AREA UPDATE REQUEST message }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE detects entering a new tracking area not included in the TAI list }

then { UE sends TRACKING AREA UPDATE REQUEST message with 'EPS update type = TA updating'}

}

(3)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and has a valid TAI value }

ensure that {

when { UE detects entering a new tracking area not included in the TAI list }

then { UE sends TRACKING AREA UPDATE REQUEST message with the TAI value in 'Last visited registered TAI' IE }

}

9.2.3.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.1, 5.5.3.2.2 and 5.5.3.2.4.

[TS 24.301 clause5.5.3.1]

The tracking area updating procedure is always initiated by the UE and is used for the following purposes:

- normal tracking area updating to update the registration of the actual tracking area of a UE in the network;

...

[TS 24.301 clause5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

After sending the TRACKING AREA UPDATE REQUEST message to the MME, the UE shall start timer T3430 and enter state EMM-TRACKING-AREA-UPDATING-INITIATED (see example in figure 5.5.3.2.2). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411. If timer T3442 is currently running, the UE shall stop timer T3442.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

…

[TS 24.301 clause5.5.3.2.4]

...

Upon receiving a TRACKING AREA UPDATE ACCEPT message, the UE shall stop timer T3430, reset the routing area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED. If the message contains a GUTI, the UE shall use this GUTI as new temporary identity for EPS services and shall store the new GUTI. If no GUTI was included by the MME in the TRACKING AREA UPDATE ACCEPT message, the old GUTI shall be used. If the UE receives a new TAI list in the TRACKING AREA UPDATE ACCEPT message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

...

If the TRACKING AREA UPDATE ACCEPT message contained a GUTI, the UE shall return a TRACKING AREA UPDATE COMPLETE message to the MME to acknowledge the received GUTI.

9.2.3.1.1.3	Test description

9.2.3.1.1.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

-	cell D (belongs to TAI-4, home PLMN) is set to ''Non- Suitable cell''.

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.1.3.2	Test procedure sequence

Table 9.2.3.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 1 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the " Serving cell" | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2, 3 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. |  |  | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on cell B with Paging UE-Identity =S-TMSI2 and with CN domain indicator set to "PS"? | - | - | 2 | - |
| 7 | Set the cell type of cell B to the "Non-Suitable cell ". Set the cell type of cell D to the " Serving cell" | - | - | - | - |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message in the next 30 seconds? | - | - | 1 | F |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on cell D with Paging UE-Identity =S-TMSI1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
|  | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 10 | Set the cell type of cell D to the "Non-Suitable cell". Set the cell type of cell A to the " Serving cell" | - | - | - | - |
| 11 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2, 3 | P |
| 12 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 14 | The SS releases the RRC connection. | - | - | - | - |

9.2.3.1.1.3.3	Specific message contents

Table 9.2.3.1.1.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.1.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00001000'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | ‘00001’B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 TAC 2 | PLMN= MCC/MNC TAC 1=2 TAC 2=4 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-2'' ''TAI-4'' |  |

Table 9.2.3.1.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 11, Table 9.2.3.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-2 |  |  |
| Last visited registered TAI | TAI-4 |  |  |

Table 9.2.3.1.1.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 12, Table 9.2.3.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-3 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00001000'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | ’00001’B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 TAC 2 | PLMN= MCC/MNC TAC 1=1 TAC 2=2 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-1'' ''TAI-2'' |  |

##### 9.2.3.1.1a	Normal tracking area update / Accepted / PSM

9.2.3.1.1a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { PSM is activated }

then { UE send TRACKING AREA UPDATE REQUEST message with the T3324 value set to 2 minutes }

}

(2)

with { UE in state EMM-REGISTERED.NO-CELL-AVAILABLE }

ensure that {

when { the SS sends a Paging message }

then { the UE does not answer the Paging message }

}

(3)

with { UE in state EMM-REGISTERED.NO-CELL-AVAILABLE }

ensure that {

when { PSM is deactivated }

then { UE sends TRACKING AREA UPDATE REQUEST message }

}

9.2.3.1.1a.2	Conformance requirements

[TS 24.301, clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

NOTE:	Timer T3324 is specified in 3GPP TS 24.008 [13].

Upon expiry of the timer T3324 or if the T3324 value provided by the network is zero, the UE may deactivate the AS layer and activate PSM by entering the state EMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is not attached for emergency bearer services;

b)	the UE has no PDN connection for emergency bearer services;

c)	the UE is in EMM-IDLE mode; and

d)	in the EMM-REGISTERED.NORMAL-SERVICE state.

If conditions a, b and c are fulfilled, but the UE is in a state other than EMM-REGISTERED.NORMAL-SERVICE when timer T3324 expires, the UE may activate PSM when the MS returns to state EMM-REGISTERED.NORMAL-SERVICE.

A UE that has already been allocated timer T3324 with a value different from "deactivated" and the timer T3324 has expired, may activate PSM if it receives an "Extended wait time" from lower layers.

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

The UE may deactivate PSM at any time (e.g. for the transfer of mobile originated signalling or user data), by activating the AS layer before initiating the necessary EMM procedures. When PSM is activated all NAS timers are stopped and associated procedures aborted except for T3412, T3346 and T3396.

[TS 24.301, clause 5.5.1.2.2]

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

[TS 24.301, clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

……..

t)	when the UE needs to request the use of PSM or needs to stop the use of PSM; or

9.2.3.1.1a.3	Test description

9.2.3.1.1a.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	the UE is configured to initiate EPS attach.

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.1a.3.2	Test procedure sequence

Table 9.2.3.1.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The user requests PSM by MMI or by AT command. The requested value of T3324 is 2 minutes. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. |  |  | - | - |
| 6 | When the T3324 timer expires the SS send Paging message including a matched identity | <-- | Paging | - | - |
| 7 | Check: Does the UE respond to the paging message? |  |  | 2 | F |
| 8 | The user requests to deactivate PSM. This can be initiated by MMI or AT command. | - | - | - | - |
| 9 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 10 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 11 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.1a.3.3	Specific message contents

Table 9.2.3.1.1a.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.3.1.1a.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | “TA only” |  |
| GUTI | GUTI-2 |  |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Value received in TRACKING UPDATE REQUEST | Included only if the IE 'T3412 extended value' was provided in the TRACKING UPDATE REQUEST |  |

Table 9.2.3.1.1a.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 9, Table 9.2.3.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-2 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |

Table 9.2.3.1.1a.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 10, Table 9.2.3.1.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-3 |  |  |

##### 9.2.3.1.1b	Normal tracking area update / Accepted / DCN

9.2.3.1.1b.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and the UE is pre-configured with a Default DCN-ID }

ensure that {

when { UE detects entering a new tracking area already included in the TAI list }

then { the UE transmits an RRCConnectionSetupComplete message including the dcn-ID IE set to the Default DCN-ID }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having received a DCN-ID in a previous GUTI REALLOCATION message }

ensure that {

when { UE detects entering a new tracking area already included in the TAI list }

then { the UE transmits an RRCConnectionSetupComplete message including the dcn-ID IE set to the received DCN-ID }

}

9.2.3.1.1b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.1.1, 5.5.3.2.4 and 5.4.1 and in TS 36.331, clause 5.3.3.4.

[TS 24.301, clause 5.3.1.1]

…

When an ATTACH REQUEST message, or a TRACKING AREA UPDATE REQUEST message when the TAI of the current cell is not included in the TAI list, is sent to establish a signalling connection, the UE NAS also provides the lower layers with the DCN-ID according to the following rules:

a)	if a DCN-ID for the PLMN code of the selected PLMN is available in the UE, the UE NAS shall provide this DCN-ID to the lower layers; or

b)	if no DCN-ID for the PLMN code of the selected PLMN is available but a Default_DCN_ID value is available in the UE, as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17], the UE NAS shall provide this DCN-ID to the lower layers.

…

[TS 24.301, clause 5.5.3.2.4]

…

If the TRACKING AREA UPDATE ACCEPT message contains the DCN-ID IE, then the UE shall store the included DCN-ID value together with the PLMN code of the registered PLMN in a DCN-ID list in a non-volatile memory in the ME as specified in annex C.

…

[TS 24.301, clause 5.4.1]

Upon receipt of the GUTI REALLOCATION COMMAND message, the UE shall store the GUTI and the TAI list and the DCN-ID, if provided, and send a GUTI REALLOCATION COMPLETE message to the MME. The UE considers the new GUTI as valid and the old GUTI as invalid. If the UE receives a new TAI list in the GUTI REALLOCATION COMMAND message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

If the GUTI REALLOCATION COMMAND message contains the DCN-ID IE, then the UE shall store the included DCN-ID value together with the PLMN code of the registered PLMN in a DCN-ID list in a non-volatile memory in the ME as specified in annex C.

[TS 36.331, clause 5.3.3.4]

The UE shall:

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

2>	discard the stored UE AS context and resumeIdentity;

2>	indicate to upper layers that the RRC connection resume has been fallbacked;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	for NB-IoT, if stored, discard the dedicated frequency offset provided by the redirectedCarrierOffsetDedicated;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI:

4>	set the s-TMSI to the value received from upper layers;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

2>	if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>	if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

2>	if the UE supports CIoT EPS optimisation(s):

3>	include attachWithoutPDN-Connectivity if received from upper layers;

3>	include up-CIoT-EPS-Optimisation if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

2>	if connecting as an RN:

3>	include the rn-SubframeConfigReq;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	except for NB-IoT:

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4>	include rlf-InfoAvailable;

3>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailableMBSFN;

3>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailable;

3>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4>	include connEstFailInfoAvailable;

3>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include the mobilityHistoryAvail;

2>	include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers;

2>	if UE needs UL gaps during continuous uplink transmission:

3>	include ue-CE-NeedULGaps;

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

9.2.3.1.1b.3	Test description

9.2.3.1.1b.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

The UE is equipped with a USIM containing values shown in Table 9.2.3.1.1b.3.1-1.

Table 9.2.3.1.1b.3.1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 is supported. |
| EFNASCONFIG | “Default_DCN_ID value” provisioned the value 0, as defined in TS 24.368, clause 5.10e. |

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.1b.3.2	Test procedure sequence

Table 9.2.3.1.1b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 1 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the " Serving cell" | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA REQUEST COMPLETE message. Check: Does the UE transmit an RRCConnectionSetupComplete message including the dcn-ID IE set to the default DCN ID provisioned in the UE? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | Void |  |  | - | - |
| 6 | The SS transmits a GUTI REALLOCATION COMMAND message including the DCN-ID IE. | <-- | GUTI REALLOCATION COMMAND | - | - |
| 7 | The UE transmit a GUTI REALLOCATION COMPLETE message | --> | GUTI REALLOCATION COMPLETE | - | - |
| 7A | The SS releases the RRC connection | - | - | - | - |
|  | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 8 | Set the cell type of cell B to the "Non-Suitable cell". Set the cell type of cell A to the " Serving cell" | - | - | - | - |
| 9 | The UE transmits a TRACKING AREA REQUEST COMPLETE message. Check: Does the UE transmit an RRCConnectionSetupComplete message including the dcn-ID IE set to the DCN ID received in step 6? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 10 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 11 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |

9.2.3.1.1b.3.3	Specific message contents

Table 9.2.3.1.1b.3.3-1: Message RRCConnectionSetupComplete (step 2, Table 9.2.3.1.1b.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| dcn-ID-r14 | Integer value | The Default DCN-ID as defined in the USIM |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.1b.3.3-2:: GUTI REALLOCATION COMMAND (step 6, Table 9.2.2.1.8.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| DCN-ID | Integer value | Any allowed value different from the Default DCN-ID in the USIM |  |

Table 9.2.3.1.1b.3.3-3: Message RRCConnectionSetupComplete (step 9, Table 9.2.3.1.1b.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| dcn-ID-r14 | Integer value | The same DCN-ID value as used in Table 9.2.3.1.1b.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.3.1.2	Void

##### 9.2.3.1.3	Void

##### 9.2.3.1.4	Normal tracking area update / List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message

9.2.3.1.4.1	Test Purpose (TP)

(1)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {

when { the UE receives TRACKING AREA UPDATE ACCEPT message including a list of equivalent PLMNs }

then { the UE stores correctly the list and considers a forbidden PLMN if the forbidden PLMN is included in the equivalent list }

}

(2)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {

when { the UE receives TRACKING AREA UPDATE ACCEPT message without a list of equivalent PLMNs }

then { the UE deletes the stored list and applies a normal PLMN selection process }

}

9.2.3.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.2.4.

[TS 24.301, clause 5.5.3.2.4]

The MME may also include of list of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, after having removed from the list any PLMN code that is already in the list of forbidden PLMNs. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the TRACKING AREA UPDATE ACCEPT message. If the TRACKING AREA UPDATE ACCEPT message does not contain a list, then the UE shall delete the stored list.

9.2.3.1.4.3	Test description

9.2.3.1.4.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, PLMN1);

-	cell B (belongs to TAI-2, PLMN1);

-	cell C (belongs to TAI-3, PLMN2);

-	cell D (belongs to TAI-4, PLMN3);

- system information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell B and C.

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE last attempted to register on cell D and received reject cause "forbidden PLMN" (so the "forbidden PLMN list" contains PLMN3).

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508[18].

9.2.3.1.4.3.2	Test procedure sequence

Table 9.2.3.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. Set the cell type of cell C to the '' Non-Suitable off cell''. Set the cell type of cell D to the '' Non-Suitable off cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | SS responds with a TRACKING AREA UPDATE ACCEPT message including PLMN2 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 6A | SS Releases the RRC Connection. | - | - | - | - |
| 7 | Set the cell type of cell A to the ''Non-Suitable off cell''. Set the cell type of cell B to the ''Non-Suitable cell''. Set the cell type of cell C to the '' Suitable cell''. | - | - | - | - |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell C (PLMN2)? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 9 | Void | - | - | - | - |
| 10 | Void | - | - | - | - |
| 11 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including PLMN1 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 12 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 12A | SS Releases the RRC Connection. | - | - | - | - |
| 12Aa1 | Void | - | - | - | - |
| 13 | Set the cell type of cell C to the ''Non-Suitable cell''. Set the cell type of cell D to the '' Serving cell''. Note: Cell D (PLMN3) belongs to the forbidden PLMN. | - | - | - | - |
| 14 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell D (PLMN3) in next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 15 | The UE is switched to manual PLMN selection mode and is made to select PLMN3 in order to remove PLMN3 in the forbidden PLMN list in the UE. | - | - | - | - |
| 16 | The UE transmits a TRACKING AREA UPDATE REQUEST message on cell D (PLMN3). | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 17 | Void | - | - | - | - |
| 18 | Void | - | - | - | - |
| 19 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 20A | SS releases the RRC connection | - | - | - | - |
|  | EXCEPTION: Step 20Aa1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 20Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 21 | The UE is switched back to automatic PLMN selection mode. | - | - | - | - |
| 21A | SS waits for 5 minutes to allow UE to complete automatic PLMN search |  |  |  |  |
| 22 | Set the cell type of cell D to ''Non-Suitable cell''. Set the cell type of cell B to ”Serving cell”. Set the cell type of cell C to ”Suitable neighbour intra-frequency cell”. | - | - | - | - |
| 23 | Void | - | - | - | - |
| 24 | Void | - | - | - | - |
| 24A | Void | - | - | - | - |
| 25 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell B (PLMN1)? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 26 | Void | - | - | - | - |
| 27 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 28 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 29 | SS Releases the RRC Connection. | - | - | - | - |
|  | EXCEPTION: Step 29a1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 29a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle ( E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.4.3.3	Specific message contents

Table 9.2.3.1.4.3.3-1: Void

Table 9.2.3.1.4.3.3-2: Void

Table 9.2.3.1.4.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 5, Table 9.2.3.1.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN2 and PLMN3. |  |

Table 9.2.3.1.4.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 11, Table 9.2.3.1.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN1 and PLMN3. |  |

Table 9.2.3.1.4.3.3-5: Void

Table 9.2.3.1.4.3.3-6: Void

##### 9.2.3.1.5	Periodic tracking area update / Accepted

9.2.3.1.5.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { the periodic tracking area updating timer T3412 expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = ’Periodic updating '}

}

(2)

with { UE in 'out of E-UTRAN coverage' and the periodic tracking area updating timer T3412 expires }

ensure that {

when { the UE enters E-UTRAN coverage }

then { UE configured for EPS attach sends TRACKING AREA UPDATE REQUEST message with EPS update type = 'Periodic updating' and the UE configured for combined EPS / IMSI attach sends TRACKING AREA UPDATE REQUEST message with EPS update type = ‘combined TA/LA updating with IMSI attach’}

}

9.2.3.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 23.401 clause 4.3.5.2 and TS 24.301 clauses 5.3.5, 5.5.3.2.1, 5.5.3.2.2 and 5.5.3.2.4.

[TS 23.401 clause 4.3.5.2]

…

An EMM-REGISTERED UE performs periodic Tracking Area Updates with the network after the expiry of the periodic TAU timer.

If the UE is out of E-UTRAN coverage (including the cases when the UE is camped on 2G/3G cells) when its periodic TAU update timer expires, and ISR is activated the UE shall start the E-UTRAN Deactivate ISR timer. After the E-UTRAN Deactivate ISR timer expires the UE shall deactivate ISR by setting its TIN to "P-TMSI". The EMM-REGISTERED UE shall remember it has to perform a Tracking Area Update when it next returns to E-UTRAN coverage.

[TS 24.301 clause5.3.5]

Periodic tracking area updating is used to periodically notify the availability of the UE to the network. The procedure is controlled in the UE by the periodic tracking area update timer (timer T3412). The value of timer T3412 is sent by the network to the UE in the ATTACH ACCEPT message and can be sent in the TRACKING AREA UPDATE ACCEPT message. The UE shall apply this value in all tracking areas of the list of tracking areas assigned to the UE, until a new value is received.

The timer T3412 is reset and started with its initial value, when the UE goes from EMM-CONNECTED to EMM-IDLE mode. The timer T3412 is stopped when the UE enters EMM-CONNECTED mode or EMM-DEREGISTERED state.

When timer T3412 expires, the periodic tracking area updating procedure shall be started and the timer shall be set to its initial value for the next start.

…

If the UE is attached to both EPS and non-EPS services, and if timer T3412 expires or timer T3423 expires when the UE is in EMM-REGISTERED.NO-CELL-AVAILABLE state, then the UE shall initiate the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" when the UE returns to EMM-REGISTERED.NORMAL-SERVICE state.

…

If the UE is in another state than EMM-REGISTERED.NORMAL-SERVICE when the timer expires the periodic tracking area updating procedure is delayed until the UE returns to EMM-REGISTERED.NORMAL-SERVICE.

…

The mobile reachable timer shall be reset and started with its initial value, when the MME releases the NAS signalling connection for the UE. The mobile reachable timer shall be stopped when a NAS signalling connection is established for the UE.

….

[TS 24.301 clause5.5.3.2.1]

The periodic tracking area updating procedure is controlled in the UE by timer T3412. When timer T3412 expires, the periodic tracking area updating procedure is started. Start and reset of timer T3412 is described in subclause 5.5.3.2.

[TS 24.301 clause5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

...

b)	when the periodic tracking area updating timer T3412 expires;

...

After sending the TRACKING AREA UPDATE REQUEST message to the MME, the UE shall start timer T3430 and enter state EMM-TRACKING-AREA-UPDATING-INITIATED (see example in figure 5.5.3.2.2). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411. If timer T3442 is currently running, the UE shall stop timer T3442.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

…

When the tracking area updating procedure is initiated in EMM-IDLE mode, the UE may also include an EPS bearer context status IE in the TRACKING AREA UPDATE REQUEST message, indicating which EPS bearer contexts are active in the UE.

[TS 24.301 clause5.5.3.2.4]

...

Upon receiving a TRACKING AREA UPDATE ACCEPT message, the UE shall stop timer T3430, reset the routing area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED. If the message contains a GUTI, the UE shall use this GUTI as new temporary identity for EPS services and shall store the new GUTI. If no GUTI was included by the MME in the TRACKING AREA UPDATE ACCEPT message, the old GUTI shall be used. If the UE receives a new TAI list in the TRACKING AREA UPDATE ACCEPT message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

...

If the TRACKING AREA UPDATE ACCEPT message contained a GUTI, the UE shall return a TRACKING AREA UPDATE COMPLETE message to the MME to acknowledge the received GUTI.

9.2.3.1.5.3	Test description

9.2.3.1.5.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.1.5.3.2	Test procedure sequence

Table 9.2.3.1.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is powered up or switched on. | - |  | - | - |
| 2 | The UE transmits an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN | --> | ATTACH REQUEST | - | - |
| 3-6Aa2 | Void | - | - | - | - |
| 6B-6J | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 7 | The SS transmits an ATTACH ACCEPT message with GUTI-1 and with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | ATTACH ACCEPT | - | - |
| 8-9 | Void | - | - | - | - |
| 9A-9Cb1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  |  | - | - |
| 10 | The SS waits 6minutes. (Expire of T3412) | - |  |  |  |
| 11 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 12 | The SS sends TRACKING AREA UPDATE ACCEPT. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 14 | The SS releases the RRC connection. |  |  | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on cell A with PagingUE-Identity = S-TMSI2 and with CN domain indicator set to "PS"? | - |  | 1 | - |
| 16 | Set the cell type of cell A to the ''non-Suitable cell''. | - |  | - | - |
| 17 | The SS waits 8minutes. | - |  | - | - |
| 18 | Set the cell type of cell A to the ''Serving cell''. | - |  | - | - |
| 19 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 20 | The SS sends TRACKING AREA UPDATE ACCEPT. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 21 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 22 | The SS releases the RRC connection. | - | - | - | - |
| 23 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state in cell A with PagingUE-Identity = S-TMSI3 and with CN domain indicator set to "PS"? | - | - | 2 | - |

9.2.3.1.5.3.3	Specific message contents

Table 9.2.3.1.5.3.3-1: Message ATTACH ACCEPT (step 7, Table 9.2.3.1.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '00001' | ''6 minutes'' |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-1 |  |  |
| GUTI | GUTI-1 |  |  |

Table 9.2.3.1.5.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 11, Table 9.2.3.1.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-1 |  |  |

Table 9.2.3.1.5.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 12, Table 9.2.3.1.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 with condition TA_only. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment |  | Condition |
| T3412 value |  |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |  |
| Timer value | '00001' | ''6 minutes'' |  |  |
| GUTI | GUTI-2 |  |  |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |  |
| Partial tracking area identity list |  |  |  |  |
| Number of elements | '00000'B |  |  |  |
| Type of list | '00'B |  | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-1 |  |  |  |

Table 9.2.3.1.5.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 19, Table 9.2.3.1.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' | TA_only |
|  | ‘010’B | “combined TA/LA updating with IMSI attach” | combined_TA_LA |
| Old GUTI | GUTI-2 |  |  |

Table 9.2.3.1.5.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 20, Table 9.2.3.1.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment |  | Condition |  |
| GUTI |  | GUTI-3 |  |  |  |  |  |
| TAI list |  |  |  |  |  |  |  |
| Length of tracking area identity list contents |  | '00000110'B |  |  |  |  |  |
| Partial tracking area identity list |  |  |  |  |  |  |
| Number of elements | '00000'B |  |  |  |  |  |
| Type of list | '00'B |  | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |  |  |
| MCC MNC TAC 1 | TAI-1 |  |  |  |  |  |

##### 9.2.3.1.5a	Periodic tracking area update / Accepted / Per-device timer

9.2.3.1.5a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode with timer T3412 “normal” and extended values being allocated by the SS during attach procedure }

ensure that {

when { timer T3412 extended value expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = “Periodic updating” }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode with timer T3412 “normal” and extended values being allocated by the SS during TAU procedure }

ensure that {

when { timer T3412 extended value expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = “Periodic updating” }

}

9.2.3.1.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.5, 5.5.1.2.4 and 5.5.3.2.4.

[TS 24.301 clause 5.3.5]

Periodic tracking area updating is used to periodically notify the availability of the UE to the network. The procedure is controlled in the UE by the periodic tracking area update timer (timer T3412). The value of timer T3412 is sent by the network to the UE in the ATTACH ACCEPT message and can be sent in the TRACKING AREA UPDATE ACCEPT message. The UE shall apply this value in all tracking areas of the list of tracking areas assigned to the UE, until a new value is received.

The UE indicates in the MS network feature support IE whether it supports the T3412 extended value.

...

When a UE is not attached for emergency bearer services, and timer T3412 expires, the periodic tracking area updating procedure shall be started and the timer shall be set to its initial value for the next start.

…

If the network includes T3412 extended value IE in the ATTACH ACCEPT message or TRACKING AREA UPDATE ACCEPT message, the network shall use T3412 extended value IE as the value of timer T3412.

[TS 24.301 clause 5.5.1.2.4]

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450. The MME shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

...

If the ATTACH ACCEPT message contains a T3412 extended value IE, then the UE shall use the value in T3412 extended value IE as periodic tracking area update timer (T3412). If the ATTACH ACCEPT message does not contain T3412 extended value IE, then the UE shall use the value in T3412 value IE as periodic tracking area update timer (T3412).

[TS 24.301 clause 5.5.3.2.4]

If the tracking area update request has been accepted by the network, the MME shall send a TRACKING AREA UPDATE ACCEPT message to the UE. If the MME assigns a new GUTI for the UE, a GUTI shall be included in the TRACKING AREA UPDATE ACCEPT message. In this case, the MME shall start timer T3450 and enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1. The MME may include a new TAI list for the UE in the TRACKING AREA UPDATE ACCEPT message.

...

If the TRACKING AREA UPDATE ACCEPT message contains T3412 extended value IE, then the UE shall use the T3412 extended value IE as periodic tracking area update timer (T3412). If the TRACKING AREA UPDATE ACCEPT contains T3412 value IE, but not T3412 extended value IE, then the UE shall use value in T3412 value IE as periodic tracking area update timer (T3412). If neither T3412 value IE nor T3412 extended value IE is included, the UE shall use the value currently stored, e.g. from a prior ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT message.

9.2.3.1.5a.3	Test description

9.2.3.1.5a.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

The UE indicates in the MS network feature support IE that it supports the T3412 extended value..

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.1.5a.3.2	Test procedure sequence

Table 9.2.3.1.5a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is powered up or switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN | --> | ATTACH REQUEST | - | - |
| 3-6Aa2 | Void | - | - | - | - |
| 6B-6J | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 7 | The SS transmits an ATTACH ACCEPT message with GUTI-1 and with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. SS assigns both a "normal" and an extended value for the T3412 timer. | <-- | ATTACH ACCEPT | - | - |
| 8-9 | Void | - | - | - | - |
| 9A-9Cb1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 10 | The SS waits 6minutes. (Expiry of T3412 extended value) | - | - | - | - |
| 11 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 12 | The SS sends TRACKING AREA UPDATE ACCEPT. SS assigns both a "normal" and an extended value for the T3412 timer. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 14 | The SS releases the RRC connection. | - | - | - | - |
| 15 | The SS waits 8 minutes. (Expiry of T3412 extended value) | - | - | - | - |
| 16 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 17 | The SS sends TRACKING AREA UPDATE ACCEPT. The SS assigns T3412 ”normal” and extended values. The extended value is different from the one assigned in the ATTACH ACCEPT message in step 7. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 18 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 19 | The SS releases the RRC connection. | - | - | - | - |

9.2.3.1.5a.3.3	Specific message contents

Table 9.2.3.1.5a.3.3-0: Message ATTACH REQUEST (steps 2, Table 9.2.3.1.5a.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS network feature support | 1 | MS supports the extended periodic timer in this domain |  |

Table 9.2.3.1.5a.3.3-1: Message ATTACH ACCEPT (step 7, Table 9.2.3.1.5a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  |  |  |
| Unit | '001'B | value is incremented in multiples of 1 minute |  |
| Timer value | '00100'B | ''4 minutes'' |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-1 |  |  |
| GUTI | GUTI-1 |  |  |
| T3412 extended value |  |  |  |
| Unit | '101'B | value is incremented in multiples of 1 minute |  |
| Timer value | '00110'B | ''6 minutes'' |  |

Table 9.2.3.1.5a.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 11, Table 9.2.3.1.5a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-1 |  |  |
| MS network feature support | 1 | MS supports the extended periodic timer in this domain |  |

Table 9.2.3.1.5a.3.3-3: Message TRACKING AREA UPDATE ACCEPT (steps 12 and 17, Table 9.2.3.1.5a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment |  | Condition |
| T3412 value |  |  |  |  |
| Unit | '001'B | value is incremented in multiples of 1 minute |  |  |
| Timer value | '00100'B | ''4 minutes'' |  |  |
| GUTI | GUTI-2 |  |  |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |  |
| Partial tracking area identity list |  |  |  |  |
| Number of elements | '00000'B |  |  |  |
| Type of list | '00'B |  | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-1 |  |  |  |
| T3412 extended value |  |  |  |  |
| Unit | '101'B |  | value is incremented in multiples of 1 minute |  |
| Timer value | '01000'B |  | ''8 minutes'' |  |

Table 9.2.3.1.5a.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 16, Table 9.2.3.1.5a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-2 |  |  |
| MS network feature support | 1 | MS supports the extended periodic timer in this domain |  |

##### 9.2.3.1.5b	Periodic tracking area update / Accepted / PSM / T3412 Extended Value

9.2.3.1.5b.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { PSM is activated }

then { UE send TRACKING AREA UPDATE REQUEST message with the T3324 value set to 2 minutes and T3412 extended value set to 4 minutes  }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode with timer T3412 extended value active }

ensure that {

when { timer T3412 extended value expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = “Periodic updating” }

}

9.2.3.1.5b.2	Conformance requirements

[TS 24.301, clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

NOTE:	Timer T3324 is specified in 3GPP TS 24.008 [13].

Upon expiry of the timer T3324 or if the T3324 value provided by the network is zero, the UE may deactivate the AS layer and activate PSM by entering the state EMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is not attached for emergency bearer services;

b)	the UE has no PDN connection for emergency bearer services;

c)	the UE is in EMM-IDLE mode; and

d)	in the EMM-REGISTERED.NORMAL-SERVICE state.

If conditions a, b and c are fulfilled, but the UE is in a state other than EMM-REGISTERED.NORMAL-SERVICE when timer T3324 expires, the UE may activate PSM when the MS returns to state EMM-REGISTERED.NORMAL-SERVICE.

A UE that has already been allocated timer T3324 with a value different from "deactivated" and the timer T3324 has expired, may activate PSM if it receives an "Extended wait time" from lower layers.

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

The UE may deactivate PSM at any time (e.g. for the transfer of mobile originated signalling or user data), by activating the AS layer before initiating the necessary EMM procedures. When PSM is activated all NAS timers are stopped and associated procedures aborted except for T3412, T3346 and T3396.

[TS 24.301, clause 5.5.1.2.2]

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

9.2.3.1.5b.3	Test description

9.2.3.1.5b.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	the UE is configured to initiate EPS attach.

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.5b.3.2	Test procedure sequence

Table 9.2.3.1.5b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The user requests PSM by MMI or by AT command. The requested value of T3324 is 2 minutes and T3412 extended value is 4 minutes. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. |  |  | - | - |
| 6 | The SS waits 4 minutes. (Expiry of T3412 extended value) | - | - | - | - |
| 7 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.5b.3.3	Specific message contents

Table 9.2.3.1.5b.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| MS network feature support | 1 | MS supports the extended periodic timer in this domain |  |
| T3412 extended value | ‘1010 0100’B or ‘1000 1000’B | 4 minutes or 240 seconds |  |

Table 9.2.3.1.5b.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-2 |  |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | ‘1010 0100’B or ‘1000 1000’B (same value as received in TAU REQUEST message at Step 2) | 4 minutes or 240 seconds |  |

Table 9.2.3.1.5b.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 011 | periodic updating' |  |
| Old GUTI | GUTI-2 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.3.1.5b.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 8, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-3 |  |  |
| T3324 value | ’0010 0100’B | 4 minutes |  |
| T3412 extended value | Value received in TRACKING AREA UPDATE REQUEST (Step 7) | Included only if the IE 'T3412 extended value' was provided in the TRACKING AREA UPDATE REQUEST |  |

##### 9.2.3.1.6	Normal tracking area update / UE with ISR active moves to E-UTRAN

9.2.3.1.6.1	Test Purpose (TP)

(1)

with { ISR activated UE in state EMM-REGISTERED and ECM_IDLE mode}

ensure that {

when { ISR activated UE reselects from E-UTRAN to UTRAN/GERAN cell belonging to the RA where the UE is registered }

then { UE does not send ROUTING AREA UPDATE REQUEST message }

}

(2)

with { ISR activated UE in state EMM-REGISTERED and ECM_IDLE mode }

ensure that {

when { ISR activated UE reselects from E-UTRAN to UTRAN/GERAN cell belonging to a RA which is not the RA where the UE is registered }

then { UE sends ROUTING AREA UPDATE REQUEST message }

}

(3)

with { ISR activated UE in state GMM-REGISTERED and PMM_IDLE mode }

ensure that {

when { ISR activated UE reselects from UTRAN/GERAN to E-UTRAN cell belonging to one of the TAs of the list of TAs where the UE is registered }

then { UE does not send TRACKING AREA UPDATE REQUEST message }

}

(4)

with { ISR activated UE in state GMM-REGISTERED and PMM_IDLE mode }

ensure that {

when { ISR activated UE reselects from UTRAN/GERAN to E-UTRAN cell belonging to a TA which is not in the list of TAs where the UE is registered }

then { UE sends TRACKING AREA UPDATE REQUEST message }

}

(5)

with { ISR activated UE has a stored TIN value = "RAT-related TMSI" }

ensure that {

when { SS sends a Paging with "P-TMSI" to ISR activated UE on UTRAN/GERAN cell }

then { UE responds to the paging message }

}

(6)

with { ISR activated UE has a stored TIN value = "RAT-related TMSI" }

ensure that {

when { SS sends a Paging with "GUTI" to ISR activated UE on E-UTRAN cell }

then { UE responds to the paging message }

}

9.2.3.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 23.401 clause 4.3.5.6, 3GPP TS 24.008 clauses 4.7.5.1.1 and 4.7.5.1.3 and 3GPP TS 24.301 clauses 5.5.3.2.2, 5.5.3.2.4 and clause 5.5.3.3.2. Unless otherwise stated these are Rel-8 requirements.

[TS 23.401 clause 4.3.5.6]

The Idle mode Signalling Reduction (ISR) function provides a mechanism to limit signalling during inter-RAT cell-reselection in idle mode (ECM-IDLE, PMM-IDLE, GPRS STANDBY states).

NOTE:	The Idle mode Signalling Reduction function is mandatory for E-UTRAN UEs that support GERAN and/or UTRAN and optional for core network. The UE's ISR capability in the UE Network Capability element is for test purpose.

ISR is activated by decision of the CN nodes and shall be explicitly signalled to the UE as "ISR activation" in the RAU and TAU signalling. The UE may have valid MM parameters both from MME and from SGSN. The "Temporary Identity used in Next update" (TIN) is a parameter of the UE's MM context, which identifies the UE identity that the UE shall indicate in the next RAU Request or TAU Request message. The TIN also identifies the status of ISR activation in the UE.

The TIN can take one of the three values, "P-TMSI", "GUTI" or "RAT-related TMSI". The UE shall set the TIN when receiving an Attach Accept, a TAU Accept or RAU Accept message according to the rules in table 4.3.5.6-1.

Table 4.3.5.6-1: Setting of the TIN

| Message received by UE | Current TIN value stored by UE | TIN value to be set by the UE when receiving message |
| --- | --- | --- |
| Attach Accept via E-UTRAN (never indicates ISR activation) | Any value | GUTI |
| Attach Accept via GERAN/UTRAN (never indicates ISR activation) | Any value | P-TMSI |
| TAU Accept not indicating ISR | Any value | GUTI |
| TAU Accept indicating ISR | GUTI P-TMSI or RAT-related TMSI | GUTI RAT-related TMSI |
| RAU Accept not indicating ISR | Any value | P-TMSI |
| RAU Accept indicating ISR | P-TMSI GUTI or RAT-related TMSI | P-TMSI RAT-related TMSI |

When ISR activation is indicated by the RAU/TAU Accept message but the UE shall not set the TIN to "RAT-related TMSI" is a special situation. Here the UE has deactivated ISR due to special situation handling. By maintaining the old TIN value the UE remembers to use the RAT specific TMSI indicated by the TIN when updating with the CN node of the other RAT.

Only if the TIN is set to "RAT-related TMSI" ISR behaviour is enabled for the UE, i.e. the UE can change between all registered areas and RATs without any update signalling and it listens for paging on the RAT it is camped on. If the TIN is set to "RAT-related TMSI", the UE's P-TMSI and RAI as well as its GUTI and TAI(s) shall remain registered with the network and shall remain valid in the UE.

Table 4.3.5.6-2: Old temporary UE Identity that the UE shall indicate in TAU/RAU Request (as old GUTI or as old P-TMSI/RAI)

| Message to be sent by UE | TIN value: P-TMSI | TIN value: GUTI | TIN value: RAT-related TMSI |
| --- | --- | --- | --- |
| TAU Request | GUTI mapped from P-TMSI/RAI | GUTI | GUTI |
| RAU Request | P-TMSI/RAI | P-TMSI/RAI mapped from GUTI | P-TMSI/RAI |
| Attach Request via E-UTRAN | GUTI mapped from P-TMSI/RAI | GUTI | GUTI |
| Attach Request via GERAN/UTRAN | P-TMSI/RAI | P-TMSI/RAI mapped from GUTI | P-TMSI/RAI |

Table 4.3.5.6-2 shows which temporary identity the UE shall indicate in a Tracking or Routing Area Update Request of in an Attach Request message, when the UE stores these as valid parameters.

Situations may occur that cause unsynchronized state information in the UE, MME and SGSN. Such special situations trigger a deactivation of ISR locally in the UE.

…

[TS 24.008 clause 4.7.5.1.1]

…

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

-	If the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and RAI, the MS shall indicate the RAI in the Old routing area identification IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the P-TMSI in the P-TMSI IE.

…

[TS 24.008 clause 4.7.5.1.3]

…

In order to indicate to the MS that the GUTI and TAI list assigned to the MS remain registered with the network and are valid in the MS, the network shall indicate in the Update result IE in the ROUTING AREA UPDATE ACCEPT message that ISR is activated.

If the ROUTING AREA UPDATE ACCEPT message contains

i)	no indication that ISR is activated, an MS supporting S1 mode shall set the TIN to "P-TMSI"; or

ii)	an indication that ISR is activated, the MS shall regard the available GUTI and TAI list as valid and registered with the network. If the TIN currently indicates "GUTI", the MS shall set the TIN to "RAT-related TMSI".

[TS 24.301 clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

…

If the UE supports neither A/Gb mode nor Iu mode, the UE shall include a valid GUTI in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

…

[TS 24.301 clause5.5.3.2.4]

The network may also indicate in the EPS update result IE in the TRACKING AREA UPDATE ACCEPT message that ISR is active. If the TRACKING AREA UPDATE ACCEPT message contains:

i)	no indication that ISR is activated, the UE shall set the TIN to "GUTI";

ii)	an indication that ISR is activated, the UE shall regard a previously assigned P-TMSI and RAI as valid and registered with the network. If the TIN currently indicates "P-TMSI", the UE shall set the TIN to "RAT-related TMSI".

9.2.3.1.6.3	Test description

9.2.3.1.6.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving  cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 and cell 7 are configured;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 and cell 25 are configured;

-	cell 5 / cell 24 belong to RAI-1 (home PLMN) as specified in TS 34.123-1 clause 12 and are set to '' Non-Suitable cell'';

-	cell 7 / cell 25 belong to RAI-2 (home PLMN) as specified in TS 34.123-1 clause 12 and are set to ''Non- Suitable “off" cell'';

-	system information indicate that NMO 1 is used.

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.6.3.2	Test procedure sequence

Table 9.2.3.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Void | - | - | - | - |
| 1A | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell 5 or 24 to the ''Serving cell''. | - | - | - | - |
| - | The following messages are sent on Cell 5 or 24 | - | - | - | - |
| 2-8 | Void | - | - | - | - |
| 8A | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message with P-TMSI and RAI on cell 5 or 24? | --> | ROUTING AREA UPDATE REQUEST |  |  |
| - | EXCEPTION: Steps 8AAa1 to 8AAa2 describe the behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 8AAa1 | IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS transmits an AUTHENTICATION AND CIPHERING REQUEST message on Cell 24. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 8AAa2 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message on Cell 24. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 8B | The SS sends ROUTING AREA UPDATE ACCEPT message with P-TMSI-1 and RAI-1. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8C | The UE sends ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 9 | Set the cell type of cell 5 or 24 to the ''Non-Suitable cell''. Set the cell type of cell A to the ''Serving cell''. | - | - | - | - |
| 10 | The UE sends TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 11 | The SS sends TRACKING AREA UPDATE ACCEPT including GUTI-1(TIN set to RAT-related TMSI) and TAI-1. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 12 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 12A | The SS releases the RRC connection. | - | - | - | - |
| 12A1-12A1b1 | Void | - | - | - | - |
| 13 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell 5 or 24 to the ''Serving cell''. | - | - | - | - |
| 14 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message in the next 90 seconds on cell 5 or 24? | --> | ROUTING AREA UPDATE REQUEST | 1 | F |
| 15 | SS transmits Paging for PS domain on Cell 5/24. | <-- | Paging | - | - |
|  | EXCEPTION: Steps 15Ca1 to 15Cb2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 15A-15b2 | Void | - | - | - | - |
| 15Ca1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN Check: Does the UE respond to the Paging request? | --> | RRCConnectionRequest | 5 | P |
| 15Ca2 | SS sends RRCConnectionReject | <-- | RRCConnectionReject | - | - |
| 15Cb1 | ELSE IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN Check: Does the UE respond to the Paging request? | --> | CHANNEL REQUEST | 5 | P |
| 15Cb2 | SS sends Immediate Assignment Reject | <-- | IMMEDIATE ASSIGNMENT REJECT | - | - |
| 16 | Set the cell type of cell 5 or 24 to the ''Non-Suitable cell''. Set the cell type of cell A to the ''Serving cell''. | - | - | - | - |
| 17 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message in the next 90 seconds. | --> | TRACKING AREA UPDATE REQUEST | 3 | F |
| 18 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on cell A with PagingUE-Identity = S-TMSI1 and with CN domain indicator set to "PS"? | - | - | 6 | P |
| 18A | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell 5 or 24 to the ''Serving cell''. | - | - | - | - |
| 18B | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message in the next 90 seconds on cell 5 or 24? | --> | ROUTING AREA UPDATE REQUEST | 1 | F |
| 19 | Set the cell type of cell 5 or 24 to the ''Non-Suitable off cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 20 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 21 | The SS sends TRACKING AREA UPDATE ACCEPT including GUTI-2 and TAI-2. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 22A | The SS releases the RRC connection. | - | - | - | - |
| 23 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on cell B with PagingUE-Identity = S-TMSI2 and with CN domain indicator set to "PS"? | - | - | 6 | - |
| 24 | Set the cell type of cell A to the ''Non-Suitable “off" cell''. Set the cell type of cell B to the ''Non-Suitable cell''. Set the cell type of cell 7 or 25 to the ''Serving cell''. | - | - | - | - |
| 25 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message with P-TMSI and RAI on cell 7 or 25? | --> | ROUTING AREA UPDATE REQUEST | 2 | P |
| 26 | The SS sends ROUTING AREA UPDATE ACCEPT message with P-TMSI-2 and RAI-2. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 27 | The UE sends ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

9.2.3.1.6.3.3	Specific message contents

Table 9.2.3.1.6.3.3-1A: Message ROUTING AREA UPDATE ACCEPT (step 8B, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508, table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Periodic RA update timer | ‘01001001’B | Activated with default T3312 value of 54 min as TS 24.008, Table 11.3a (Note) |  |
| PDP Context Status | Same value as in ROUTING AREA UPDATE REQUEST message |  |  |
| Note: The timer is specified to avoid Rel-10 and higher releases impact on the UE behaviour which is not part of the TPs. |  |  |  |

Table 9.2.3.1.6.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 10, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating | TA_only |
|  | '001'B or ‘010’B | Combined TA/LA updating or combined TA/LA updating with IMSI attach | No_LAU_CombinedRAU_GERAN_UTRA |
|  | ‘010’B | Combined TA/LA updating with IMSI attach | LAU_CombinedRAU_GERAN_UTRA |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old GUTI | Mapped from the P-TMSI and RAI allocated in step 5 |  |  |
| Old P-TMSI signature | Any allowed value |  |  |
| NonceUE | Any allowed value |  |  |
| EPS bearer context status | (octet 3 = '00100000'B and octet 4 = '00000000'B) OR (octet 3 = '01100000'B and octet 4 = '00000000'B |  |  |

Table 9.2.3.1.6.3.3-1A: Message TRACKING AREA UPDATE REQUEST Conditions (Table 9.2.3.1.6.3.3-1)

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below 36.508 table 4.7.2-24. |
| No_LAU_CombinedRAU_GERAN_UTRA | UE has not performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |
| LAU_CombinedRAU_GERAN_UTRA | UE has performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |

Table 9.2.3.1.6.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 11, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | '100'B | ''TA updated and ISR activated'' | TA_only |
|  | '101'B | ''combined TA/LA updated and ISR activated'' | combined_TA_LA |
| T3412 value | Not present | (Note) |  |
| GUTI | GUTI-1 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAI-1 |  |  |
| Note: This is to allow the UE to apply the default timer, and avoid Rel-10 and higher releases impact on the UE behaviour which is not part of the TPs. |  |  |  |

Table 9.2.3.1.6.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 20, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |

Table 9.2.3.1.6.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 21, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | '100'B | ''TA updated and ISR activated'' | TA_only |
|  | '101'B | ''combined TA/LA updated and ISR activated'' | combined_TA_LA |
| T3412 value | Not present | (Note) |  |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 2 | TAI-2 |  |  |
| Note: This is to allow the UE to apply the default timer, and avoid Rel-10 and higher releases impact on the UE behaviour which is not part of the TPs. |  |  |  |

Table 9.2.3.1.6.3.3-4A: Message ROUTING AREA UPDATE REQUEST (step 25, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508, table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update type | ‘000’B | “RA updating” |  |
|  | ‘001’B | “combined RA/LA updating” |  |
| P-TMSI | P-TMSI-1 |  |  |

Table 9.2.3.1.6.3.3-5: Message ROUTING AREA UPDATE ACCEPT (step 26, Table 9.2.3.1.6.3.2-1)

| Derivation path: 36.508, table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update result | 100 | “RA updated and ISR activated” |  |
|  | 101 | “combined RA/LA updated and ISR activated” |  |
| Periodic RA update timer | ‘01001001’B | Activated with default T3312 value of 54 min as TS 24.008, Table 11.3a (Note) |  |
| Note: The timer is specified to avoid Rel-10 and higher releases impact on the UE behaviour which is not part of the TPs. |  |  |  |

Table 9.2.3.1.6.3.3-6: Void

##### 9.2.3.1.7	Void

##### 9.2.3.1.8	UE receives an indication that the RRC connection was released with cause "load balancing TAU required"

9.2.3.1.8.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-CONNECTED mode}

ensure that {

when { UE receives RRC CONNECTION RELEASE message with cause "load balancing TAU required" and enters EMM-REGISTERED and EMM-IDLE mode}

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = "TA updating"}

}

(2)

with { UE in state EMM-REGISTERED and EMM-CONNECTED mode}

ensure that {

when { UE receives RRC CONNECTION RELEASE message with cause "load balancing TAU required" and enters EMM-REGISTERED and EMM-IDLE mode}

then { the UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set random-Value; and, in the RRCConnectionSetupComplete message the registeredMME is not included) }

}

9.2.3.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 23.401 clause 4.3.7.3, 3GPP TS 24.301 clauses 5.3.1.1, 5.5.3.1, 5.5.3.2.2 and 5.5.3.2.4, and 3G PP TS 36.331 clauses and 5.3.3.3 and 5.3.3.4].

[TS 23.401 clause 4.3.7.3]

…

To off-load ECM-CONNECTED mode UEs, the MME initiates the S1 Release procedure with release cause "load balancing TAU required" (clause 5.3.5). The S1 and RRC connections are released and the UE initiates a TAU but provides neither the S-TMSI nor the GUMMEI to eNodeB in the RRC establishment.

…

[TS 24.301 clause 5.3.1.1]

…

When the UE is registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the S-TMSI, but shall not provide the registered MME identifier to the lower layers. Exceptionally, when the UE in EMM-IDLE mode initiates a tracking area updating or combined tracking area updating procedure for load balancing purposes, the UE NAS shall provide the lower layers with neither S-TMSI nor registered MME identifier.

…

[TS 24.301 clause5.5.3.1]

The tracking area updating procedure is always initiated by the UE and is used for the following purposes:

…

-	MME load balancing;

…

[TS 24.301 clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

…

e)	when the UE receives an indication from the lower layers that the RRC connection was released with cause "load balancing TAU required";

…

…

[TS 24.301 clause 5.5.3.2.4]

If the tracking area update request has been accepted by the network, the MME shall send a TRACKING AREA UPDATE ACCEPT message to the UE. If the MME assigns a new GUTI for the UE, a GUTI shall be included in the TRACKING AREA UPDATE ACCEPT message. In this case, the MME shall start timer T3450 and enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1. The MME may include a new TAI list for the UE in the TRACKING AREA UPDATE ACCEPT message.

…

Upon receiving a TRACKING AREA UPDATE ACCEPT message, the UE shall stop timer T3430, reset the tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED. If the message contains a GUTI, the UE shall use this GUTI as new temporary identity for EPS services and shall store the new GUTI. If no GUTI was included by the MME in the TRACKING AREA UPDATE ACCEPT message, the old GUTI shall be used. If the UE receives a new TAI list in the TRACKING AREA UPDATE ACCEPT message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

…

If the TRACKING AREA UPDATE ACCEPT message contained a GUTI, the UE shall return a TRACKING AREA UPDATE COMPLETE message to the MME to acknowledge the received GUTI.

[TS 36.331 clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else:

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

…

[TS 36.331 clause 5.3.3.4]

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>		if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>		if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

…

9.2.3.1.8.3	Test description

9.2.3.1.8.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.2.3.1.8.3.2	Test procedure sequence

Table 9.2.3.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCConnectionRelease with cause "load balancing TAU required". | - | - | - | - |
| 1A | Check: Does the UE send an RRCConnectionRequest with the ue-Identity set to “random-Value”? | --> | RRCConnectionRequest | 1 | P |
| 1B | The SS responds with RRCConnectionSetup. | <-- | RRCConnectionSetup | - | - |
| 1C | Check: Does the UE send an RRCConnectionSetupComplete without the registeredMME IE? | --> | RRCConnectionSetupComplete | 1 | P |
| 2 | Check: Does the UE send TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS sends TRACKING AREA UPDATE ACCEPT. (Note 1) | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note 1: The SS assigns a different MME Identifier (MMEI) value in a GUTI. |  |  |  |  |  |

9.2.3.1.8.3.3	Specific message contents

Table 9.2.3.1.8.3.3-1: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.8.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |

Table 9.2.3.1.8.3.3-2: Message RRCConnectionRequest (step 1A, Table 9.2.3.1.8.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| random-Value | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.8.3.3-3: Message RRCConnectionSetupComplete (step 1C, Table 9.2.3.1.8.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| registeredMME | Not present |  |  |
| dedicatedInfoNAS | See table 9.2.3.1.8.3.3-3 |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.3.1.8a	Normal tracking area update / low priority override

9.2.3.1.8a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and UE configured for low priority NAS signalling and low priority NAS signalling override }

ensure that {

when { UE detects entering a new tracking area not included in the TAI list }

then { UE sends TRACKING AREA UPDATE REQUEST message with the low priority indicator set to "MS is not configured for NAS signalling low priority"}

}

9.2.3.1.8a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 4.2A

[TS 24.301 clause 4.2A]

...

A UE configured for NAS signalling low priority (see 3GPP TS 24.368 [15A], 3GPP TS 31.102 [17]) indicates this by including the Device properties IE in the appropriate NAS message and setting the low priority indicator to "MS is configured for NAS signalling low priority", except for the following cases in which the UE shall set the low priority indicator to "MS is not configured for NAS signalling low priority":

…

-	the UE configured for dual priority is requested by the upper layers to establish a PDN connection with the low priority indicator set to "MS is not configured for NAS signalling low priority";

-	the UE configured for dual priority is performing EPS session management procedures related to the PDN connection established with low priority indicator set to "MS is not configured for NAS signalling low priority";

-	the UE configured for dual priority has a PDN connection established by setting the low priority indicator to "MS is not configured for NAS signalling low priority" and is performing EPS mobility management procedures;

-	the UE is accessing the network with access class 11 – 15; or

-	the UE is responding to paging.

The network may use the NAS signalling low priority indication for NAS level mobility management congestion control and APN based congestion control.

If the NAS signalling low priority indication is provided in a PDN CONNECTIVITY REQUEST message, the MME stores the NAS signalling low priority indication within the default EPS bearer context activated due to the PDN connectivity request procedure.

9.2.3.1.8a.3	Test description

9.2.3.1.8a.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is configured to initiate EPS attach.

-	the UE is configured for low priority NAS signalling

-	the UE is configured for low priority NAS signalling override

The UE is equipped with a USIM containing values shown in Table 9.2.3.1.8a.3.1-1.

Table 9.2.3.1.8a.3.1-1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Service 96 is supported. |
| EFNASCONFIG | “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. |
| EFNASCONFIG | “Override_NAS_SignallingLowPriority is set to UE can override the NAS signalling low priority indicator” as defined in TS 24.368, clause 5.9. |
| EFNASCONFIG | “ExtendedAccessBarring is set to extended access barring is applied for the UE” as defined in TS 24.368, clause 5.8 |
| EFNASCONFIG | “Override_ExtendedAccessBarring is set to UE can override extended access barring” as defined in TS 24.368, clause 5.10. |
| Note: As per TS 23.401 [22] clause 4.3.17.4, UE’s configuration of low access priority and Extended Access Barring shall match each other and so do their corresponding override configuration. |  |

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.8a.3.2	Test procedure sequence

Table 9.2.3.1.8a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | Cause the UE to request connectivity to an additional PDN with the Device properties IE set to “MS is not configured for NAS signalling low priority” (see Note) | - | - |  |  |
| 0B | The UE transmits an RRCConnectionRequest message with establishmentCause set to ‘mo-Data’ followed by a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 0C | The SS establishes SRB2 and DRB associated with default EPS bearer context (a first PDN obtained during the attach procedure). | - | - | - | - |
| 0D | UE should include the Device properties IE set to “MS is not configured for NAS signalling low priority” | --> | PDN CONNECTIVITY REQUEST | - | - |
| 0E | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with IE EPS Bearer Identity set to new EPS bearer context. | <-- | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: In parallel to the event described in step 0F below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| 0F | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message for the additional default EPS Bearer. | --> | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
|  | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 1 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the " Serving cell" | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - |  | - |
| Note: The trigger in step 0A is the same as in the generic procedure in 36.508 clause 6.4.3.2. The request of connectivity to an additional PDN may be performed by MMI or AT command. |  |  |  |  |  |

9.2.3.1.8a.3.3	Specific message contents

Table 9.2.3.1.8a.3.3-1: Message PDN CONNECTIVITY REQUEST (step 0D, Table 9.2.3.1.8a.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Procedure transaction identity | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |
| ESM information transfer flag | Not present | This IE is only needed during an attach procedure. |  |
| Access point name | APN-1 | The requested PDN is different from default PDN |  |
| Device properties | 0 | ‘MS is not configured for NAS signalling low priority’ |  |

Table 9.2.3.1.8a.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.8a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Device properties | 0 | MS is not configured for NAS signalling low priority |  |
| Last visited registered TAI | TAI-1 |  |  |

##### 9.2.3.1.8b	Normal tracking area update / EAB broadcast handling / ExtendedAccessBarring configured in the UE / ExtendedAccessBarring and Override_ExtendedAccessBarring configured in the UE

9.2.3.1.8b.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and UE being configured for ExtendedAccessBarring (EAB) and not configured for Override_ExtendedAccessBarring and accessing the network with an access class different to classes 11 - 15 }

ensure that {

when { UE detects entering a new tracking area, and, the cell is broadcasting EAB information effectively barring the cell for the UE }

then { the UE does not initiate Normal tracking area update on the cell }

}

(2)

with { UE is switched off and powered off and UE being configured for ExtendedAccessBarring (EAB) and configured for Override_ExtendedAccessBarring and accessing the network with an access class different to classes 11 - 15 }

ensure that {

when { UE is powered; then switched on and the cell is broadcasting EAB information effectively barring the cell for the UE and an indication from the upper layers to override EAB is received }

then { the UE initiates Attach procedure on the cell }

}

9.2.3.1.8b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.3.10 and D.1, TS 24.368 clauses 4, 5.8 and 5.10, and, TS 36.331, clauses 5.2.2.4, 5.3.3.2 and 5.3.3.12. Unless otherwise stated these are Rel-11 requirements.

[TS 24.301, clause 5.3.10]

The network can restrict the access for certain groups of UEs by means of barring their access class.

The UE shall evaluate the access control information for Access Class Barring, Access Control for CSFB and Extended Access Barring (EAB) as specified in 3GPP TS 36.331 [22].

[TS 24.301, clause D.1]

When EMM requests the establishment of a NAS-signalling connection, the RRC establishment cause used by the UE shall be selected according to the NAS procedure as specified in table D.1.1. The EMM shall also indicate to the lower layer for the purpose of access control, the call type associated with the RRC establishment cause as specified in table D.1.1. If the UE is configured for EAB (see the "ExtendedAccessBarring" leaf of NAS configuration MO in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]), the EMM shall indicate to the lower layer for the purpose of access control that EAB applies for this request except for the following cases:

-	the UE is accessing the network with one of the access classes 11 – 15;

-	the UE is answering to paging;

-	the RRC Establishment cause is set to "Emergency call"; or

-	the UE is configured to allow overriding EAB (see the "Override_ExtendedAccessBarring" leaf of the NAS configuration MO as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) and receives an indication from the upper layers to override EAB.

[TS 24.368, clause 4]

The NAS configuration MO is used to manage configuration parameters related to NAS functionality for a UE supporting provisioning of such information. The presence and format of the non-access stratum configuration file on the USIM is specified in 3GPP TS 31.102 [6].

The MO identifier is: urn:oma:mo:ext-3gpp-nas-config:1.0.

The OMA DM Access Control List (ACL) property mechanism (see OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the NAS configuration MO.

The following nodes and leaf objects are possible in the NAS configuration MO as described in figure 4-1:

Figure 4-1: The NAS configuration Management Object

[TS 24.368, clause 5.8]

The ExtendedAccessBarring leaf indicates whether the extended access barring is applicable for the UE as specified in 3GPP TS 24.008 [4] and 3GPP TS 24.301 [5].

-	Occurrence: ZeroOrOne

-	Format: bool

-	Access Types: Get, Replace

-	Values: 0, 1

0	Indicates that the extended access barring is not applied for the UE.

1	Indicates that the extended access barring is applied for the UE.

The default value 0 applies if this leaf is not provisioned.

[TS 24.368, clause 5.10]

The Override_ExtendedAccessBarring leaf indicates whether the UE can override ExtendedAccessBarring leaf node configured to extended access barring.

The handling of extended access barring for the UE when the Override_ExtendedAccessBarring leaf exists is specified in 3GPP TS 24.008 [4] and 3GPP TS 24.301 [5].

-	Occurrence: ZeroOrOne

-	Format: bool

-	Access Types: Get, Replace

-	Values: 0, 1

0	Indicates that the UE cannot override extended access barring

1	Indicates that the UE can override extended access barring

The default value 0 applies if this leaf is not provisioned.

[TS 36.331, clause 5.2.2.4]

The UE shall:

...

1>	not initiate the RRC connection establishment subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

...

1>	if the UE is EAB capable:

2>	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification or upon entering a cell during RRC_IDLE:

3>	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4>	start acquiring SystemInformationBlockType14 immediately;

3>	else:

4>	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that EAB is applicable, upon which the procedure ends;

[TS 36.331, clause 5.3.3.12]

The UE shall:

1>	if SystemInformationBlockType14 is present and includes the eab-Param:

2>	if the eab-Common is included in the eab-Param:

3>	if the UE belongs to the category of UEs as indicated in the eab-Category contained in eab-Common; and

3>	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the eab-BarringBitmap contained in eab-Common is set to one:

4>	consider access to the cell as barred;

3>	else:

4>	consider access to the cell as not barred due to EAB;

9.2.3.1.8b.3	Test description

9.2.3.1.8b.3.1	Pre-test conditions

System Simulator:

-	Cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	Cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell''; When active, the SS is transmitting SystemInformationBlockType14 Extended access barring activated on this cell. System information combination 21 as defined in TS 36.508[18] clause 4.4.3.1.2 is used in E-UTRA cell B.

UE:

-	The UE is configured to respect "ExtendedAccessBarring" as per TS 24.368 [49]. This is achieved by equipping the UE with a USIM containing values shown in Table 9.2.3.1.8b.3.1-1.

Table 9.2.3.1.8b.3.1-1: USIM A Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 'Non-Access Stratum configuration by USIM' is supported. |
| EFNASCONFIG | - Extended access barring activated: UE shall apply extended access barring - Override Extended access barring not activated |
| EFNASCONFIG | - “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. - Override_NAS_SignallingLowPriority not activated |

-	The UE is configured to initiate EPS attach.

-	The UE is capable to be configured for Override Extended access barring as described in TS 24.368 [49]. This is achieved by equipping the UE with a USIM containing values shown in Table 9.2.3.1.8b.3.1-2.

Table 9.2.3.1.8b.3.1-2: USIM B Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Services 96 'Non-Access Stratum configuration by USIM' is supported. |
| EFNASCONFIG | - Extended access barring activated: UE shall apply extended access barring - Override Extended access barring activated |
| EFNASCONFIG | - “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. - Override_NAS_SignallingLowPriority activated |
| Note: As per TS 23.401 [22] clause 4.3.17.4, UE’s configurations of low access priority and Extended Access Barring shall match each other. |  |

Preamble:

-	The UE is equipped with USIM A, and is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.8b.3.2	Test procedure sequence

Table 9.2.3.1.8b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Non-Suitable cell". - Cell B as "Serving cell" (the cell is transmitting SystemInformationBlockType14 Extended access barring activated) | - | - | - | - |
| - | The following messages are sent and shall be received on cell B | - | - | - | - |
| 2 | Check: Does the UE send a TRACKING AREA UPDATE REQUEST message in the next 60 sec? | -> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 3 | Void | - | - | - | - |
| 3A | The UE is switched off and powered off. | - | - | - | - |
| 3B | The UE is equipped with USIM B containing values shown in Table 9.2.3.1.8b.3.1-2. | - | - | - | - |
| 3C | The UE is powered on and switched on. | - | - | - | - |
| 3D | Cause the UE to initiate Attach procedure with the Device properties IE set to “MS is not configured for NAS signalling low priority” by MMI or AT command. |  |  |  |  |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7-11 | The attach procedure is completed by executing steps 5 to 8 of the UE registration procedure in TS 36.508 subclause 4.5.2.3. | - | - | - | - |
| 12-20b1 | Void | - | - | - | - |
| 21 | The SS transmits an ATTACH REJECT message, EMM cause = EPS services and non-EPS services not allowed (#8) | <-- | ATTACH REJECT | - | - |
| 22 | The SS releases the RRC connection. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

9.2.3.1.8b.3.3	Specific message contents

Table 9.2.3.1.8b.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Device properties | 1 | “MS is configured for NAS signalling low priority” |  |

Table 9.2.3.1.8b.3.3-2: Message ATTACH REQUEST (Step4)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Device properties | 0 | “MS is not configured for NAS signalling low priority” |  |

##### 9.2.3.1.9	Normal tracking area update / Correct handling of CSG list

9.2.3.1.9.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { Manual CSG ID selection is requested }

then { UE sends TRACKING AREA UPDATE REQUEST message with 'EPS update type = TA updating'}

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode; and UE’s Allowed CSG list is not empty }

ensure that {

when { UE detects suitable E-UTRAN CSG cell previously visited }

then { UE reselects the suitable previously visited E-UTRAN CSG cell and sends TRACKING AREA UPDATE REQUEST message with 'EPS update type = TA updating'}

}

9.2.3.1.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.3.1, 5.5.3.2.2 and 5.5.3.2.4.

[TS 24.301, clause 5.5.3.1]

The tracking area updating procedure is always initiated by the UE and is used for the following purposes:

-	normal tracking area updating to update the registration of the actual tracking area of a UE in the network;

...

-	to indicate to the network that the UE has selected a CSG cell whose CSG identity is not included in the UE's Allowed CSG list.

[TS 24.301, clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

k)	when due to manual CSG selection the UE has selected a CSG cell whose CSG identity is not included in the UE's Allowed CSG list;

...

After sending the TRACKING AREA UPDATE REQUEST message to the MME, the UE shall start timer T3430 and enter state EMM-TRACKING-AREA-UPDATING-INITIATED (see example in figure 5.5.3.2.2). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411. If timer T3442 is currently running, the UE shall stop timer T3442.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall include a valid GUTI in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

.....

[TS 24.301, clause 5.5.3.2.4]

If the tracking area update request has been accepted by the network, the MME shall send a TRACKING AREA UPDATE ACCEPT message to the UE. If the MME assigns a new GUTI for the UE, a GUTI shall be included in the TRACKING AREA UPDATE ACCEPT message. In this case, the MME shall start timer T3450 and enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1. The MME may include a new TAI list for the UE in the TRACKING AREA UPDATE ACCEPT message.

...

Upon receiving a TRACKING AREA UPDATE ACCEPT message, the UE shall stop timer T3430, reset the tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED. If the message contains a GUTI, the UE shall use this GUTI as new temporary identity for EPS services and shall store the new GUTI. If no GUTI was included by the MME in the TRACKING AREA UPDATE ACCEPT message, the old GUTI shall be used. If the UE receives a new TAI list in the TRACKING AREA UPDATE ACCEPT message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

...

If the UE has initiated the tracking area updating procedure due to manual CSG selection and receives a TRACKING AREA UPDATE ACCEPT message, the UE shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSD ID to the Allowed CSG list.

If the TRACKING AREA UPDATE ACCEPT message contained a GUTI, the UE shall return a TRACKING AREA UPDATE COMPLETE message to the MME to acknowledge the received GUTI.

Upon receiving a TRACKING AREA UPDATE COMPLETE message, the MME shall stop timer T3450, and shall consider the GUTI sent in the TRACKING AREA UPDATE ACCEPT message as valid.

...

9.2.3.1.9.3	Test description

9.2.3.1.9.3.1	Pre-test conditions

System Simulator:

-	Cell A (belongs to TAI-1, home PLMN, not a CSG cell) is set to ''Serving cell'';

-	Cell B (belongs to TAI-3, home PLMN, is a CSG cell) is set to ''Non- Suitable cell'';

-	Cell D (belongs to TAI-4, home PLMN, not a CSG cell) is set to ''Non- Suitable cell''.

-	System information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells A and D

-	System information combination 7 as defined in TS 36.508[18] clause 4.4.3.1 is used in cell B;

UE:

- the UE is configured to initiate EPS attach;

- pc_Allowed_CSG_list, The UE’s allowed CSG li st shall not contain CSG ID 2.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on Cell A according to TS 36.508 [18].

9.2.3.1.9.3.2	Test procedure sequence

Table 9.2.3.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". - Cell D as a "Not Suitable “Off” cell". | - | - | - | - |
| 2 | The UE performs manual CSG ID selection and CSG Identity ('000 0000 0000 0000 0000 0000 0010'B) is selected manually. | - | - | - | - |
| 3 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell B? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 4 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. NOTE: UE shall add the selected CSG ID as indicate in step 2 to UE's Allowed CSG list. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message on Cell B? | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | The SS configures: - Cell A as a "Not Suitable “Off” cell". - Cell B as a "Not Suitable cell". - Cell D as a "Serving cell". | - | - | - | - |
| 8 | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell D. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 9 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 10 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell D. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 11 | The SS releases the RRC connection. | - | - | - | - |
| 12 | The SS configures: - Cell B as a "Serving cell". - Cell D as a "Not Suitable cell". | - | - | - | - |
| 13 | Check: Does the UE camped on Cell B within 6 min and transmits a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 14 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message on Cell B? | --> | TRACKING AREA UPDATE COMPLETE | 2 | P |

9.2.3.1.9.3.3	Specific message contents

Table 9.2.3.1.9.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 3, Table 9.2.3.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.9.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 4, Table 9.2.3.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | ‘00000’B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | PLMN= MCC/MNC TAC 1=2 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-2'' |  |

Table 9.2.3.1.9.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 8, Table 9.2.3.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-2 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| Last visited registered TAI | TAI-2 |  |  |

Table 9.2.3.1.9.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 9, Table 9.2.3.1.9.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-3 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | ‘00000’B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | PLMN= MCC/MNC TAC 1=4 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-4'' |  |

Table 9.2.3.1.9.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 13, Table 9.2.3.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-3 |  |  |
| Last visited registered TAI | TAI-4 |  |  |

Table 9.2.3.1.9.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 14, Table 9.2.3.1.9.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | ’00000’B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | PLMN= MCC/MNC TAC 1=2 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-2'' |  |

##### 9.2.3.1.9a	Normal tracking area update / NAS signalling connection recovery

9.2.3.1.9a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-CONNECTED }

ensure that {
  when { UE receives an indication of "RRC Connection failure" from the lower layers }

then { UE initiates the tracking area updating procedure }

}

9.2.3.1.9a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.1, 5.5.3.2.2 and 5.5.3.3.2 and TS 36.331, clause 5.3.7.8 and 5.3.12.

[TS 24.301, clause 5.5.3.1]

The tracking area updating procedure is always initiated by the UE and is used for the following purposes:

...

-	recovery from certain error cases (for details see subclauses 5.5.3.2.2 and subclause 5.5.3.3.2);

[TS 24.301, clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

...

i)	when the UE receives an indication of "RRC Connection failure" from the lower layers and has no signalling or user uplink data pending (i.e. when the lower layer requests NAS signalling connection recovery);

...

For all cases except case b, the UE shall set the EPS update type IE to "TA updating". For case b, the UE shall set the EPS update type IE to "periodic updating".

[TS 24.301, clause 5.5.3.3.2]

The UE operating in CS/PS mode 1 or CS/PS mode 2, in state EMM-REGISTERED, shall initiate the combined tracking area updating procedure:

...

j)	when the UE receives an indication of "RRC Connection failure" from the lower layers and has no signalling or user uplink data pending (i.e. when the lower layer requests NAS signalling connection recovery);

...

To initiate a combined tracking area updating procedure the UE sends the message TRACKING AREA UPDATE REQUEST to the network, starts timer T3430 and changes to state EMM-TRACKING-AREA-UPDATING-INITIATED. The value of the EPS update type IE in the message shall indicate "combined TA/LA updating" unless explicitly specified otherwise.

[TS 36.331, clause 5.3.7.8]

Upon receiving the RRCConnectionReestablishmentReject message, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

...

1>	indicate the release of the RRC connection to upper layers together with the release cause;

9.2.3.1.9a.3	Test description

9.2.3.1.9a.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B (same TA); Cell B is configured to belong to TAI-1;

-	cell A is "Serving cell" and cell B is "non-Suitable cell".

UE:

none.

Preamble:

-	the UE is in state Generic RB Established (state 3) on cell A according to TS 36.508 [18].

9.2.3.1.9a.3.2	Test procedure sequence

Table 9.2.3.1.9a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the “Non-Suitable "Off” cell”. Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionReestablishmentRequest. | - | - | - | - |
| 3 | The SS transmits an RRCConnectionReestablishmentReject. | - | - | - | - |
| 4 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST with the EPS update type set to 'TA updating' or ‘combined TA/LA updating’ in the next 10 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.9a.3.3	Specific message contents

Table 9.2.3.1.9a.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 4, Table 9.2.3.1.9a.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.9a.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 5, Table 9.2.3.1.9a.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

##### 9.2.3.1.10	Normal tracking area update / Rejected / IMSI invalid

9.2.3.1.10.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to ''Illegal UE'' }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up }

then { UE send ATTACH REQUEST message with Old GUTI or IMSI IE = ''IMSI''}

9.2.3.1.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

#3		(Illegal UE); or

….

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and the MM parameters update status, TMSI, LAI and ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed.

9.2.3.1.10.3	Test description

9.2.3.1.10.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'';

-	If (px_SinglePLMN_Tested = Multi PLMN) cell G (belongs to TAI-7 ,visited PLMN) is set to ''Non-suitable cell'';

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (UTRAN, belong to RAI-1) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (GERAN, belong to RAI-1) is set to "Non-suitable “off” cell;

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 9 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells.

UE:

- 	the UE is configured to initiate EPS attach;

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.10.3.2	Test procedure sequence

Table 9.2.3.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = "Illegal UE" as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Set the cell type of cell A to the ''Serving cell''. Set the cell type of cell B to the ''Non-Suitable cell''. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? Note: Cell A belongs to the same PLMN where the UE was rejected but a different TAC | --> | ATTACH REQUEST | 1 | F |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? | --> | ATTACH REQUEST | 1 | F |
| - | EXCEPTION: Steps 9a to 12a describe behaviour that depends on the network capability / preference; the "lower case letter" identifies a step sequence that takes place if the network is capable or prefers. | - | - | - | - |
| 9a | Set the cell type of cell A to the '' Non-Suitable cell''. Set the cell type of cell G to the ''Serving cell''. | - | - | - | - |
| 10a | If (px_SinglePLMN_Tested = Multi PLMN) Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell G? Note: Cell G belongs to a PLMN which is not the same like the one on which the UE was rejected. | --> | ATTACH REQUEST | 1 | F |
| 11a | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 12a | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell G? | --> | ATTACH REQUEST | 1 | F |
| 13 | If present, set the cell type of cell G to the ''Non-Suitable “off” cell''. If present, set the cell type of cell 9 or 24 to the ''Serving cell''. |  |  |  |  |
|  | EXCEPTION: Steps 14a1 to 14a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 14a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 14a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell 9 or 24? | --> | ATTACH REQUEST | 1 | F |
| 15 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 16 | Set the cell type of cell A to the ''Serving cell''. If present, set the cell type of cell G to the ''Non-Suitable cell''. If present, set the cell type of cell 9 or 24 to the ''Non-Suitable “off” cell''. | - | - | - | - |
| 17 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 18 | Check: Does the UE transmit an ATTACH REQUEST message on cell A? | --> | ATTACH REQUEST | 2 | P |
| 19-32b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.10.3.3	Specific message contents

Table 9.2.3.1.10.3.3-1: Void

Table 9.2.3.1.10.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.10.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 00000011 | #3 "Illegal UE" |  |

Table 9.2.3.1.10.3.3-3: Message ATTACH REQUEST (step 18, Table 9.2.3.1.10.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |

##### 9.2.3.1.11	Normal tracking area update / Rejected / Illegal ME

9.2.3.1.11.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "Illegal ME" }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up }

then { UE send ATTACH REQUEST message with Old GUTI or IMSI IE = ''IMSI''}

9.2.3.1.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#6		(Illegal ME);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and the MM parameters update status, TMSI, LAI and ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed.

9.2.3.1.11.3	Test description

The test description is identical to the one of subclause 9.2.3.1.10 except that the reject cause #3 "Illegal UE" is replaced with the reject cause #6 "Illegal ME".

##### 9.2.3.1.12	Normal tracking area update / Rejected / EPS service not allowed

9.2.3.1.12.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to ''EPS service not allowed'' }

then { UE considers the USIM as invalid for EPS services and enters state EMM-DEREGISTERED }

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up or switched on }

then { UE sends ATTACH REQUEST message with 'Old GUTI or IMSI IE = 'IMSI''}

9.2.3.1.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.12.3	Test description

The test description is identical to the one of subclause 9.2.3.1.10 except that the reject cause #3 “illegal UE” is replaced with reject cause #7 “EPS services not allowed”.

9.2.3.1.12.3.2	Test procedure sequence

Same test procedure as in clause 9.2.3.1.10.3.2 with the following exceptions:

Depending on UE capabilities the behaviour in table 9.2.3.1.12.3.2-1 occurs in parallel with step 14a2

Table 9.2.3.1.12.3.2-1: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a 5 describe a behaviour which depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_CS THEN the UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 1a2a1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| 1a2a2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 1a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a5 | The SS transmits a LOCATION UPDATING ACCEPT message including IMSI-1 | <-- | LOCATION UPDATING ACCEPT | - | - |

Depending on UE capabilities the behaviour in table 9.2.3.1.12.3.2-2 occurs between step 15 and 16

Table 9.2.3.1.12.3.2-2: IMSI detach behaviour

| 1 | IF pc_CS THEN the UE optionally transmits an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION | - | - |
| --- | --- | --- | --- | --- | --- |

9.2.3.1.12.3.3	Specific message contents

Table 9.2.3.1.12.3.3-1: LOCATION UPDATING ACCEPT (step 5, Table 9.2.3.1.12.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity |  |  |  |
| IMSI | IMSI-1 |  |  |

##### 9.2.3.1.13	Normal tracking area update / Rejected / UE identity cannot be derived by the network

9.2.3.1.13.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to ''UE identity cannot be derived by the network'' }

then { UE deletes any GUTI, last visited registered TAI, TAI list and KSI and enters the state EMM-DEREGISTERED and subsequently, UE automatically initiates the attach procedure}

9.2.3.1.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#9		(UE identity cannot be derived by the network);

The UE shall set the EPS update status to EU2 NOT UPDATED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

Subsequently, the UE shall automatically initiate the attach procedure.

NOTE 2:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.13.3	Test description

9.2.3.1.13.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'';

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.13.3.2	Test procedure sequence

Table 9.2.3.1.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = '' UE identity cannot be derived by the network '' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 4-19a2-4a1 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 4a1 describes a behaviour which depends on the UE capability | - | - | - | - |
| 4a1 | IF NOT pc_Automatic_EPS_Re_Attach , the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message on cell B? | --> | ATTACH REQUEST | 1 | P |
| 6-19b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second then the existing RRC Connection is released. |  |  |  |  |  |

9.2.3.1.13.3.3	Specific message contents

Table 9.2.3.1.13.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Old P-TMSI signature | Absent or any allowed value |  |  |

Table 9.2.3.1.13.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00001001'B | #9 ''UE identity cannot be derived by the network'' |  |

Table 9.2.3.1.13.3.3-3: Message ATTACH REQUEST (step 5, Table 9.2.3.1.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Old P-TMSI signature | Not present |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.1.13.3.3-4: Message ATTACH REJECT (step 6, Table 9.2.3.1.13.3.2-1)

| Derivation Path: 36.508 table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | '0000 1111'B | #15 ''No suitable cells in tracking area'' |  |

##### 9.2.3.1.14	Normal tracking area update / Rejected / UE implicitly detached

9.2.3.1.14.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to ''UE implicitly detached'' }

then { UE enters the state EMM-DEREGISTERED.NORMAL-SERVICE and sends ATTACH REQUEST message}

9.2.3.1.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#10	(Implicitly detached);

The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.NORMAL-SERVICE. The UE shall delete any mapped EPS security context or partial native EPS security context. The UE shall perform a new attach procedure.

NOTE 3:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM state as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.14.3	Test description

9.2.3.1.14.3.1	Pre-test conditions

System Simulator:

-	cell A belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B belongs to TAI-2, home PLMN is set to ''Non- Suitable cell'';

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.14.3.2	Test procedure sequence

Table 9.2.3.1.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''UE implicitly detached'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 3a1-3a2 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 3a1 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 3a2 describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a2 | IF NOT pc_Automatic_EPS_Re_Attach , the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message on cell B? | --> | ATTACH REQUEST | 1 | P |
| 5-18b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. |  |  |  |  |  |

9.2.3.1.14.3.3	Specific message contents

Table 9.2.3.1.14.3.3-1:  Void

Table 9.2.3.1.14.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00001010'B | #10 ''UE implicitly detached'' |  |

Table 9.2.3.1.14.3.3-3: Message ATTACH REQUEST (step 4, Table 9.2.3.1.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

##### 9.2.3.1.15	Normal tracking area update / Rejected / PLMN not allowed

9.2.3.1.15.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "PLMN not allowed" }

then { UE deletes the GUTI, the last visited registered TAI and KSI and UE deletes the list of equivalent PLMNs and UE enters state EMM-DEREGISTERED.PLMN-SEARCH and UE stores the PLMN in the "forbidden PLMN list" }

}

(2)

with { UE is switched off having a PLMN stored in the "forbidden PLMN list" }

ensure that {

when { UE is powered up on this PLMN }

then { UE doesn’t perform an attach procedure }

}

(3)

with { UE in EMM-DEREGISTERED.PLMN-SEARCH state having a PLMN stored in the "forbidden PLMN list" }

ensure that {

when { UE enters a cell which is not in the "forbidden PLMN list" }

then { UE initiates an attach procedure }

}

(4)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state having a PLMN stored in the "forbidden PLMN list" }

ensure that {

when { UE is in a forbidden PLMN cells and when the PLMN is selected manually }

then { UE initiates an attach procedure }

}

9.2.3.1.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#11	(PLMN not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall reset the tracking area updating attempt counter, delete the list of equivalent PLMNs and enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMN list".

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter and the MM parameters update status, TMSI, LAI, ciphering key sequence number and the location update attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value and no RR connection exists.

9.2.3.1.15.3	Test description

9.2.3.1.15.3.1	Pre-test conditions

System Simulator:

-	cell G (belongs to TAI-7, visited PLMN) and is set to ''Serving cell'';

-	cell H (belongs to TAI-8, visited PLMN) is set to ''Non-suitable cell";

-	cell I (belongs to TAI-9, visited PLMN) is set to ''Non-suitable “off” cell'';

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1;

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (UTRAN, belongs to RAI-1) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (GERAN, belongs to RAI-1) is set to "Non-suitable “off” cell".

-	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 9 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell G according to TS 36.508 [18].

9.2.3.1.15.3.2	Test procedure sequence

Table 9.2.3.1.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell G to the ''Non-Suitable cell''. Set the cell type of cell H to the ''Serving cell''. Note: cell G and cell H are in the same PLMN. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell H. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''PLMN not allowed''. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell H? | --> | ATTACH REQUEST | 1 | F |
| 6 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 7 | Set the cell type of cell G to the ''Serving cell''. Set the cell type of cell H to the ''Non-Suitable cell''. Note: cell G and cell H are in the same PLMN. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell G? | --> | ATTACH REQUEST | 2 | F |
| 10 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? | --> | ATTACH REQUEST | 2 | F |
| 12 | Set the cell type of cell G to the ''Non-Suitable cell''. If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 9 to the ''Serving cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Serving cell''. Note: Cell G and Cell 9 or 24 are in the same PLMN. | - | - | - | - |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 13a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 13a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell 9 or 24? | --> | ATTACH REQUEST | 1 | F |
| 14 | The following messages are sent and shall be received on cell I. | - | - | - | - |
| 15 | If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 9 to the ''Non-Suitable cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Non-Suitable cell''. Set the cell type of cell I to the ''Serving cell'', Set the cell type of cell H to the ''Non-Suitable “off” cell''. Note: cell 9 or 24 and cell I are in different PLMNs. | - | - | - | - |
| 16 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 3 | P |
| 16A-16Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 17-23 | Void | - | - | - | - |
| 24 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 25 describes behaviour that depends on the UE capability. |  |  |  |  |
| 25 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 26 | The following messages are sent and shall be received on cell G. | - | - | - | - |
| 27 | Set the cell type of cell G to the '' Serving cell''. Set the cell type of cell I to the '' Non-Suitable cell''. Note: Cell G belongs to the forbidden PLMN. | - | - | - | - |
| 28 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 29 | The UE is switched to manual PLMN selection mode and is made to select the forbidden PLMN. | - | - | - | - |
| 30 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 4 | P |
| 31-43 | The attach procedure is completed by executing steps 5 to 17 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.15.3.3	Specific message contents

Table 9.2.3.1.15.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-7 |  |  |

Table 9.2.3.1.15.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00001011'B | #11 '' PLMN not allowed '' |  |

Table 9.2.3.1.15.3.3-3: Message ATTACH REQUEST (step 16, Table 9.2.3.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Old P-TMSI signature | Not present |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.1.15.3.3-4: Message ATTACH ACCEPT (step 21, Table 9.2.3.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark |  | Comment | Condition |
| TAI list |  |  |  |  |
| Number of elements | '00000'B |  |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |  |
| MCC | '002'B | ''TAI-9'' |  |  |
| MNC | '101'B | ''TAI-9'' |  |  |
| TAC 1 | '1'B | ''TAI-9'' |  |  |
| GUTI | GUTI-9 |  |  |  |

Table 9.2.3.1.15.3.3-5: Message ATTACH REQUEST (step 30, Table 9.2.3.1.15.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-9 |  |  |
| Last visited registered TAI | TAI-9 |  |  |

##### 9.2.3.1.15a	Normal tracking area update / Rejected / PLMN not allowed / Single Frequency operation

9.2.3.1.15a.1	Test Purpose (TP)

Same test purpose as in clause 9.2.3.1.15.1

9.2.3.1.15a.2	Conformance requirements

Same conformance requirements as in clause 9.2.3.1.15.2

9.2.3.1.15a.3	Test description

9.2.3.1.15a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency E-UTRA cells Cell A, Cell B, and Cell C;

-	cell A (belongs to TAI-7, visited PLMN, MCC = MCC in USIM  MNC=02) and is set to ''Serving cell'';

-	cell B (belongs to TAI-8, visited PLMN, MCC = MCC in USIM  MNC=02) is set to ''Non-suitable cell";

-	cell C (belongs to TAI-9, visited PLMN, MCC=002 MNC=101) is set to ''Non-suitable “off” cell'';

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (UTRAN, belongs to RAI-1) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (GERAN, belongs to RAI-1) is set to "Non-suitable “off” cell".

-	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 5 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.15a.3.2	Test procedure sequence

Table 9.2.3.1.15a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. Note: cell A and cell B are in the same PLMN. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''PLMN not allowed''. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell B? | --> | ATTACH REQUEST | 1 | F |
| 6 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 7 | Set the cell type of cell A to the ''Serving cell''. Set the cell type of cell B to the ''Non-Suitable cell''. Note: cell A and cell B are in the same PLMN. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? | --> | ATTACH REQUEST | 2 | F |
| 10 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? | --> | ATTACH REQUEST | 2 | F |
| 12 | Set the cell type of cell A to the ''Non-Suitable cell''. If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 5 to the ''Serving cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Serving cell''. Note: Cell A and Cell 5 or 24 are in the same PLMN. | - | - | - | - |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 13a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 13a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell 5 or 24? | --> | ATTACH REQUEST | 1 | F |
| 14 | The following messages are sent and shall be received on cell C. | - | - | - | - |
| 15 | If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 5 to the ''Non-Suitable cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Non-Suitable cell''. Set the cell type of cell C to the ''Serving cell'', Set the cell type of cell B to the ''Non-Suitable “off” cell''. Note: cell 5 or 24 and cell C are in different PLMNs. | - | - | - | - |
| 16 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 3 | P |
| 16A-16Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 17-23 | Void | - | - | - | - |
| 24 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 25 describes behaviour that depends on the UE capability. |  |  |  |  |
| 25 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 26 | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 27 | Set the cell type of cell A to the '' Serving cell''. Set the cell type of cell C to the '' Non-Suitable cell''. Note: Cell A belongs to the forbidden PLMN. | - | - | - | - |
| 28 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 29 | The UE is switched to manual PLMN selection mode and is made to select the forbidden PLMN. | - | - | - | - |
| 30 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 4 | P |
| 31-42 | The attach procedure is completed by executing steps 5 to 17 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.15a.3.3	Specific message contents

Same specific message contents as in clause 9.2.3.1.15.3.3

##### 9.2.3.1.16	Normal tracking area update / Rejected / Tracking area not allowed

9.2.3.1.16.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to ''Tracking area not allowed '' }

then { shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE and store the current TAI in the list of "forbidden tracking areas for regional provision of service" }

(2)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and has a TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE is in the serving cell which the UE is rejected }

then { UE does not attempt an attach procedure on any other cell}

}

(3)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE enters a new cell in the same TAI it was rejected }

then { UE does not initiate an attach procedure}

}

(4)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE enters a new cell with different TAI without in the list of "forbidden tracking areas for regional provision of service"}

then { UE initiates attach procedure with IMSI }

}

(5)

with { UE is switched off }

ensure that {

when { UE is powered on and enters the cell with "forbidden tracking areas for regional provision of service" before the UE was switched off }

then { UE initiates attach procedure on the cell }

9.2.3.1.16.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#12	(Tracking area not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for regional provision of service".

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.16.3	Test description

9.2.3.1.16.3.1	Pre-test conditions

System Simulator:

-	cell A (belong to TAI-1, home PLMN) are set to ''Non-suitable cell';'

-	cell B (belongs to TAI-2, home PLMN) is set to ''Serving cell'';

-	cell  C (belongs to TAI-3, visited PLMN) is set to ''Non-suitable cell'';

-	cell M (belongs to TAI-1, home PLMN) are set to ''Non-suitable “off" cell''

UE:

- 	The UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell B according to TS 36.508 [18].

9.2.3.1.16.3.2	Test procedure sequence

Table 9.2.3.1.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to ''Serving cell''. Set the cell type of cell B to ''Non-Suitable cell''. Set the cell type of cell C to ”Suitable neighbour intra-frequency cell”. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''Tracking area not allowed'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell C? | --> | ATTACH REQUEST | 2 | F |
| 5A | Set the cell type of cell B to the ''Non-Suitable “off" cell''. Set the cell type of cell M to the '' Non-Suitable cell ''. Set the cell type of cell C to the '' Non-Suitable cell ''. |  |  |  |  |
| 6 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? | --> | ATTACH REQUEST | 1 | F |
| 8 | Set the cell type of cell A to the '' Non-Suitable cell ''. Set the cell type of cell M to the ''Serving cell''. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell M? | --> | ATTACH REQUEST | 3 | F |
| 10 | Set the cell type of cell C to the ''Serving cell''. Set the cell type of cell M to the ''Non-Suitable cell ''. | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST on cell C including a PDN CONNECTIVITY REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |
| 11A-11Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 12-18 | Void |  |  |  |  |
| 19 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 20 describes behaviour that depends on the UE capability. |  |  |  |  |
| 20 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 21 | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 22 | Set the cell type of cell A to the '' Serving cell''. Set the cell type of cell C to the '' Non-Suitable cell''. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 24 | The UE transmits ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. | --> | ATTACH REQUEST | 5 | P |
| 25-38b1 | The attach procedure is completed executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 clause 4.5.2.3 | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.16.3.3	Specific message contents

Table 9.2.3.1.16.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-7 |  |  |

Table 9.2.3.1.16.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00001100'B | #12 ''Tracking area not allowed'' |  |

Table 9.2.3.1.16.3.3-3: Message ATTACH REQUEST (step 11, Table 9.2.3.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.1.16.3.3-4: Message ATTACH ACCEPT (step 16, Table 9.2.3.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark |  | Comment | Condition |
| TAI list |  |  |  |  |
| Number of elements | '00000'B |  |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |  |
| MCC MNC TAC 1 | PLMN= MCC/02 TAC 1=1 | ''MCC is set to the same MCC stored in EFIMSI'' ''TAI-7'' |  |  |
| GUTI | GUTI-7 |  |  |  |

Table 9.2.3.1.16.3.3-5: Message ATTACH REQUEST (step 24, Table 9.2.3.1.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-7 |  |  |
| Last visited registered TAI | TAI-7 |  |  |

##### 9.2.3.1.17	Normal tracking area update / Rejected / Roaming not allowed in this tracking area

9.2.3.1.17.1	Test Purpose (TP)

(1)

with { the UE has sent TRACKING AREA UPDATE REQUEST message }

ensure that {

when { the UE receives TRACKING AREA UPDATE REJECT message with the reject cause set to "roaming not allowed in this tracking area" }

then { the UE sets the EPS update status to EU3 ROAMING NOT ALLOWED and the UE deletes the last visited registered TAI and the UE enters the state EMM-REGISTERED.PLMN-SEARCH and the UE stores the current TAI in the list of "forbidden tracking areas for roaming" }

}

(2)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the current TAI in the list of "forbidden tracking areas for roaming"}

ensure that {

when { the serving cell belongs to TAI where UE was rejected }

then { the UE does not attempt to send TRACKING AREA UPDATE REQUEST message }

}

(3)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE enters a cell belonging to same PLMN and TAI not in the list of "forbidden tracking areas for roaming"}

then { the UE sends TRACKING AREA UPDATE REQUEST message }

}

(4)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE enters a cell belonging to another PLMN }

then { the UE sends TRACKING AREA UPDATE REQUEST message }

}

(5)

with { the UE, which A/Gb mode or Iu mode is supported by the UE, in EMM-DEREGISTERED.PLMN-SEARCH state and the list of "forbidden tracking areas for roaming" contains more than one TAI }

ensure that {

when { the UE move to UTRAN or GERAN cell }

then { the UE sends ROUTING AREA UPDATE REQUEST message }

9.2.3.1.17.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.2.5.

[TS 24.301, clause 5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

...

#13	(Roaming not allowed in this tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete the list of equivalent PLMNs. The UE shall reset the tracking area updating attempt counter and shall change to state EMM-REGISTERED.PLMN-SEARCH.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list if present.

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

...

9.2.3.1.17.3	Test description

9.2.3.1.17.3.1	Pre-test conditions

System Simulator:

-	cell K(belongs to TAI-9, visited PLMN) is set to ''Serving cell'';

-	cell L(belongs to TAI-11, same visited PLMN) is set to ''Non-suitable cell'';

-	If (px_SinglePLMN_Tested = Multi PLMN) cell J (belongs to TAI-10, another visited PLMN) is set to ''Non-suitable “off” cell'';

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN) ;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA,  cell 9 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

-	system information indicate that NMO 1 is used;

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither  cell 9 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell K according to TS 36.508 [18].

9.2.3.1.17.3.2	Test procedure sequence

Table 9.2.3.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell K to the '' Suitable neighbour intra-frequency cell ''. Set the cell type of cell L to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits TRACKING AREA UPDATE REQUEST on Cell L. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2A | Set the cell type of cell K to the ''Serving cell''. Set the cell type of cell L to the “Suitable neighbour intra-frequency cell''. |  |  |  |  |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''Roaming not allowed in this tracking area'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. |  |  |  |  |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message on cell K? | --> | TRACKING AREA UPDATE REQUEST | 1, 3 | P |
| 8 | The SS transmits TRACKING AREA UPDATE REJECT message with EMM cause = ''Roaming not allowed in this tracking area'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 9 | The SS releases the RRC connection. |  |  |  |  |
| 10 | Set the cell type of cell K to the ''Non-Suitable cell''. If present, set the cell type of cell 9 or 24 to the ''Serving cell''. Note: Cell K and cell 9 or 24 are in the same PLMN. | - | - | - | - |
|  | EXCEPTION: Steps 11a1 to 11a3 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 11a1 | Check: Does the UE transmit ROUTING AREA UPDATE REQUEST message on cell 9 or 24? | --> | ROUTING AREA UPDATE REQUEST | 5 | P |
| 11a2 | The SS transmits a ROUTING AREA UPDATE REJECT message with cause = '' Roaming not allowed in this tracking area '' as specified. | <-- | ROUTING AREA UPDATE REJECT | - | - |
| 11a3 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Steps 12a1 to 12a4 describe behaviour that depends on the network capability / preference; the "lower case letter" identifies a step sequence that takes place if the network is capable or prefers. | - | - | - | - |
| 12 a1 | If (px_SinglePLMN_Tested = Multi PLMN) If present, set the cell type of cell 9 or 24 to the '' Non-Suitable cell''. Set the cell type of cell L to the ''Non-Suitable “off” cell''. Set the cell type of cell J to the ''Serving cell''. Note: cell 9 or 24 and cell J are in different PLMNs. | - | - | - | - |
| 12a2 | Check: Does the UE transmit TRACKING AREA REQUEST message on cell J? | --> | TRACKING AREA UPDATE REQUEST | 1, 4 | P |
| 12a3 | The SS sends TRACKING AREA ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 12a4 | The UE transmits TRACKING AREA COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | If (px_SinglePLMN_Tested = Multi PLMN): At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. Else: At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |

9.2.3.1.17.3.3	Specific message contents

Table 9.2.3.1.17.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, 7 and 12a2, Table 9.2.3.1.17.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-9 |  |  |

Table 9.2.3.1.17.3.3-2: TRACKING AREA UPDATE REJECT (step 3 and 8, Table 9.2.3.1.17.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1101'B | Roaming not allowed in this tracking area |  |

##### 9.2.3.1.18	Normal tracking area update / Rejected / EPS services not allowed in this PLMN

9.2.3.1.18.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to 'EPS services not allowed in this PLMN' }

then { UE deletes the GUTI, the last visited registered TAI and KSI and UE deletes the list of equivalent PLMNs and UE enters state EMM-DEREGISTERED.PLMN-SEARCH and UE stores the PLMN in the "forbidden PLMNs for GPRS service" }

(2)

with { UE in E-UTRA EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMNs for GPRS service" }

ensure that {

when { UE enters a cell which is in the "forbidden PLMNs for GPRS service" }

then { UE doesn’t perform an attach procedure }

}

(3)

with { UE in EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMNs for GPRS service" }

ensure that {

when { UE enters a cell which is not in the "forbidden PLMNs for GPRS service" }

then { UE initiates an attach procedure }

}

(4)

with { UE is switched off and a PLMN is stored in the 'forbidden PLMNs for GPRS service' }

ensure that {

when { UE is power ON in a cell with forbidden PLMNs for GPRS service }

then { UE initiates an attach procedure }

9.2.3.1.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.2.5.

[TS 24.301, clause 5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

...

#14	(EPS services not allowed in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). Furthermore the UE shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMNs for GPRS service" list.

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

...

9.2.3.1.18.3	Test description

9.2.3.1.18.3.1	Pre-test conditions

System Simulator:

-	cell G belongs to TAI-7(visited PLMN) and is set to ''Serving cell'';

-	cell H belongs to TAI-8(visited PLMN, another TAC) and is set to ''Non-suitable cell'';

-	cell I belongs to TAI-9(visited PLMN, another PLMN) and is set to ''Non-suitable “off” cell'';

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1;

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA,  cell 9 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	system information indicate that NMO 1 is used;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither  cell 9 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell G according to TS 36.508 [18].

9.2.3.1.18.3.2	Test procedure sequence

Table 9.2.3.1.18.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell G to the ''Non-Suitable cell''. Set the cell type of cell H to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell H. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''EPS services not allowed in this PLMN'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell H? | --> | ATTACH REQUEST | 1 | F |
| 6 | Set the cell type of cell G to the ''Serving cell''. Set the cell type of cell H to the ''Non-Suitable cell''. Note: cell G and cell H are in the same PLMN. | - | - | - | - |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? | --> | ATTACH REQUEST | 2 | F |
| 9 | Set the cell type of cell G to the ''Non-Suitable cell''. Set the cell type of cell H to the ''Non-Suitable “off” cell''. If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 9 to the ''Serving cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Serving cell''. Note: Cell G and Cell 9 or 24 are in the same PLMN. | - | - | - | - |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 10a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| - | EXCEPTION: Depending on UE capabilities the behaviour in table 9.2.3.1.18.3.2-2 occurs in parallel with step 10a2. | - | - | - | - |
| 10a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell 9 or 24? | --> | ATTACH REQUEST | 1 | F |
| - | The following messages are sent and shall be received on cell I. | - | - | - | - |
| 11 | If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 9 to the ''Non-Suitable cell' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Non-Suitable cell''. Set the cell type of cell I to the ''Serving cell''. Note: cell 9 or 24 and cell I are in different PLMNs. | - | - | - | - |
| 12 | Check: Does the UE send ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 3 | P |
| 12A-12Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 13-20 | Void | - | - | - | - |
| 21 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 22 describes behaviour that depends on the UE capability. | - | - | - | - |
| 22 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| - | The following messages are sent and shall be received on cell G. | - | - | - | - |
| 23 | Set the cell type of cell G to the '' Serving cell''. Set the cell type of cell I to the '' Non-Suitable cell''. Note: Cell G belongs to the forbidden PLMNs for GPRS service. | - | - | - | - |
| 24 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 25 | Check: Does the UE send ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 4 | P |
| 26-39b137 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

Table 9.2.3.1.18.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a 5 describe a behaviour which depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_CS THEN the UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 1a2a1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| 1a2a2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 1a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a5 | The SS transmits a LOCATION UPDATING ACCEPT message including IMSI-1 | <-- | LOCATION UPDATING ACCEPT | - | - |

9.2.3.1.18.3.3	Specific message contents

Table 9.2.3.1.18.3.3-1: Void

Table 9.2.3.1.18.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.18.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '0000 1110'B | #14 ''EPS services not allowed in this PLMN'' |  |

Table 9.2.3.1.18.3.3-3: Message ATTACH REQUEST (step 12, Table 9.2.3.1.18.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.1.18.3.3-4: Message ATTACH ACCEPT (step 18, Table 9.2.3.1.18.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark |  | Comment | Condition |
| TAI list |  |  |  |  |
| Number of elements | '00000'B |  |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |  |
| MCC MNC TAC 1 | PLMN= 002/101 TAC 1=1 | ''TAI-9'' |  |  |
| GUTI | GUTI-9 |  |  |  |

Table 9.2.3.1.18.3.3-5: Message ATTACH REQUEST (step 25, Table 9.2.3.1.18.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-9 |  |  |
| Last visited registered TAI | TAI-9 |  |  |

Table 9.2.3.1.18.3.3-6: LOCATION UPDATING ACCEPT (step 5, Table 9.2.3.1.18.3.2-2)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity |  |  |  |
| IMSI | IMSI-1 |  |  |

##### 9.2.3.1.18a	Normal tracking area update / Rejected / EPS services not allowed in this PLMN / Single Frequency operation

9.2.3.1.18a.1	Test Purpose (TP)

Same test purpose as in clause 9.2.3.1.18.1

9.2.3.1.18a.2	Conformance requirements

Same conformance requirements as in clause 9.2.3.1.18.2

9.2.3.1.18a.3	Test description

9.2.3.1.18a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency E-UTRA cells Cell A, Cell B, and Cell C;

-	cell A belongs to TAI-7(visited PLMN, MCC = MCC in USIM  MNC=02) and is set to ''Serving cell'';

-	cell B belongs to TAI-8(visited PLMN, MCC = MCC in USIM  MNC=02 another TAC) and is set to ''Non-suitable cell'';

-	cell C belongs to TAI-9(visited PLMN, MCC=002 MNC=101) and is set to ''Non-suitable “off” cell'';

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-8, visited PLMN) is set to "Non-suitable “off” cell";

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	system information indicate that NMO 1 is used;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 5 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.18a.3.2	Test procedure sequence

Table 9.2.3.1.18a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''EPS services not allowed in this PLMN'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. |  |  |  |  |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell B? | --> | ATTACH REQUEST | 1 | F |
| 6 | Set the cell type of cell A to the ''Serving cell''. Set the cell type of cell B to the ''Non-Suitable cell''. Note: cell A and cell B are in the same PLMN. | - | - | - | - |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on cell A? | --> | ATTACH REQUEST | 2 | F |
| 9 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Non-Suitable “off” cell''. If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 5 to the ''Serving cell'' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Serving cell''. Note: Cell A and Cell 5 or 24 are in the same PLMN. | - | - | - | - |
|  | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 10a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| - | EXCEPTION: Depending on UE capabilities the behaviour in table 9.2.3.1.18a.3.2-2 occurs in parallel with step 10a2 |  |  |  |  |
| 10a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell 5 or 24? | --> | ATTACH REQUEST | 1 | F |
|  | The following messages are sent and shall be received on cell C. | - | - | - | - |
| 11 | If px_RATComb_Tested = EUTRA_UTRA set the cell type of cell 5 to the ''Non-Suitable cell' or if px_RATComb_Tested = EUTRA_GERAN set cell 24 to the ''Non-Suitable cell''. Set the cell type of cell C to the ''Serving cell''. Note: cell 5 or 24 and cell C are in different PLMNs. | - | - | - | - |
| 12 | Check: Does the UE send ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 3 | P |
| 12A-12Nb1 | The attach procedure is completed by executing steps 5 to 18b1 the UE registration procedure in TS 36.508 sub clause 4.5.2.3 | - | - | - | - |
| 13-20 | Void | - | - | - | - |
| 21 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 22 describes behaviour that depends on the UE capability. |  |  |  |  |
| 22 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
|  | The following messages are sent and shall be received on cell A. |  |  |  |  |
| 23 | Set the cell type of cell A to the '' Serving cell''. Set the cell type of cell C to the '' Non-Suitable cell''. Note: Cell A belongs to the forbidden PLMNs for GPRS service. |  |  |  |  |
| 24 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 25 | Check: Does the UE send ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN? | --> | ATTACH REQUEST | 4 | P |
| 26-39b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

Table 9.2.3.1.18a.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a 5 describe a behaviour which depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_CS THEN the UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 1a2a1 | IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| - | EXCEPTION: Step 1a2a2 describes behaviour that depends on the UE capability. | - | - | - | - |
| 1a2a2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 1a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a5 | The SS transmits a LOCATION UPDATING ACCEPT message including IMSI-1 | <-- | LOCATION UPDATING ACCEPT | - | - |

9.2.3.1.18a.3.3	Specific message contents

Same specific message contents as in clause 9.2.3.1.18.3.3

Table 9.2.3.1.18a.3.3-1: LOCATION UPDATING ACCEPT (step 1a5, Table 9.2.3.1.18.3.2-2)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity |  |  |  |
| IMSI | IMSI-1 |  |  |

##### 9.2.3.1.19	Normal tracking area update / Rejected / No suitable cells in tracking Area

9.2.3.1.19.1	Test Purpose (TP)

(1)

with { UE is sending a TRACKING AREA UPDATE REQUEST message}

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'No Suitable Cells In tracking area' }

then { UE selects a suitable cell in another tracking area in the same PLMN and performs the tracking area updating procedure and UE does not select a suitable cell in another PLMN}

}

9.2.3.1.19.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.2.5.

[TS 24.301, clause 5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

...

#15	(No suitable cells in tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-REGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list if present.

The UE shall search for a suitable cell in another tracking area or in another location area in the same PLMN according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

...

9.2.3.1.19.3	Test description

9.2.3.1.19.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'';

-	If (px_SinglePLMN_Tested = Multi PLMN) cell G (belongs to TAI-7, visited PLMN) is set to ''Non-suitable cell''.

UE:

- 	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.19.3.2	Test procedure sequence

Table 9.2.3.1.19.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". If present, set the cell type of Cell G to the "Serving cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'No Suitable Cells In tracking area'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 3A | Set the cell type of Cell A to the " Serving cell ". Set the cell type of Cell B to the "Suitable neighbour Intra Frequency cell". |  |  |  |  |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 4A | Void |  |  |  |  |
| - | The following messages are sent and shall be received on Cell A. | - | - | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 6 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.19.3.3	Specific message contents

Table 9.2.3.1.19.3.3-1: TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.19.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1111'B | No Suitable Cells In tracking area |  |

Table 9.2.3.1.19.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 5, Table 9.2.3.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |

##### 9.2.3.1.20	Normal tracking area update / Rejected / Not authorized for this CSG

9.2.3.1.20.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE removes the CSG ID from the Allowed CSG list }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE searches for a suitable cell in the same PLMN and sent a TRACKING AREA UPDATE REQUEST message }

}

9.2.3.1.20.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.2.5]

#25	(Not authorized for this CSG);

EMM cause #25 is only applicable when received from a CSG cell. EMM cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.3.2.6.

If the TRACKING AREA UPDATE REJECT message with EMM cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-REGISTERED.LIMITED-SERVICE.

If the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list, the UE shall remove the entry corresponding to this CSG ID from the Allowed CSG list.

If the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Operator CSG list, the UE shall apply the procedures defined in 3GPP TS 23.122 [6] subclause 3.1A.

The UE shall search for a suitable cell in the same PLMN according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.20.3	Test description

9.2.3.1.20.3.1	Pre-test conditions

System Simulator:

-	cell A (TAI-1, frequency 1, HPLMN,  not a CSG cell) is set to ''Serving cell'';

-	cell B (TAI-2, frequency 1, HPLMN,  is a CSG cell) is set to ''Non-suitable off cell'';

-	cell D (TAI-4, frequency 1, HPLMN,  not a CSG cell) is set to ''Non-suitable off cell''.

-	System information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is used in Cell A and Cell D.

-	System information combination 7 as defined in TS 36.508[18] clause 4.4.3.1 is used in Cell B.

UE:

-	the UE is previously registered on cell B using either manual CSG selection or a USIM with field EFACSGL preconfigured (so the allowed CSG list includes CSG ID of cell B).

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18]

9.2.3.1.20.3.2	Test procedure sequence

Table 9.2.3.1.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits an TRACKING AREA UPDATE REJECT message with EMM cause = "Not authorized for this CSG". | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message on Cell B in the next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 6 | The SS configures: - Cell A as a "Not Suitable off cell". - Cell B as a "Not Suitable cell". - Cell D as a "Suitable cell". | - | - | - | - |
| 7 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message in the next 30 seconds on Cell D? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 9A | The SS releases the RRC connection. | - | - | - | - |
| 10 | The SS configures: - Cell A as a "Not Suitable off cell". - Cell B as a " Serving cell". - Cell D as a " Not Suitable cell". | - | - | - | - |
| 11 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message in the next 30 seconds on Cell B? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |

9.2.3.1.20.3.3	Specific message contents

Table 9.2.3.1.20.3.3-1: TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.20.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Old P-TMSI signature | P-TMSI Signature-1 |  |  |

Table 9.2.3.1.20.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.20.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '00011001'B | #25 " Not authorized for this CSG" |  |

Table 9.2.3.1.20.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7, Table 9.2.3.1.20.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Old P-TMSI signature | P-TMSI Signature-1 |  |  |

Table 9.2.3.1.20.3.3-4: SystemInformationBlockType1 for Cell A, B, CD (Pre-test conditions and all steps in Table 9.2.3.1.20.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell B |
|  | FALSE |  | Cell A |
|  | FALSE |  | Cell CD |
| csg-Identity | Not present |  | Cell A |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell B |
|  | Not present |  | Cell CD |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.20.3.3-4A: SystemInformationBlockType1-BR-r13 for Cell A, B, CD (Pre-test conditions and all steps in Table 9.2.3.1.20.3.2-1 when UE under test is CAT M1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell B |
|  | FALSE |  | Cell A |
|  | FALSE |  | Cell CD |
| csg-Identity | Not present |  | Cell A |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell B |
|  | Not present |  | Cell CD |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.20.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 8, Table 9.2.3.1.20.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |

Table 9.2.3.1.20.3.3-6: SystemInformationBlockType4 for cell  B (Pre-test conditions and all steps, Table 9.2.3.1.20.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| csg-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| Start | 2 |  |  |
| Range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.3.1.20a	Normal tracking area update / Rejected / Congestion

9.2.3.1.20a.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives an integrity protected TRACKING AREA UPDATE REJECT message with the reject cause set to ''Congestion'' and the T3346 value IE present }

then { The UE shall abort the tracking area updating procedure, reset the tracking area updating attempt counter and set the EPS update status to EU2 NOT UPDATED. The UE shall also start timer T3346 with the value provided in the T3346 IE as received in the TRACKING AREA UPDATE REJECT message, UE initiates the tracking area updating procedure when timer T3346 expires}

9.2.3.1.20a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.5.

[TS 24.301 clause 5.5.3.2.5]

If the tracking area updating cannot be accepted by the network, the MME sends a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

If the tracking area update request is rejected due to general NAS level mobility management congestion control, the network shall set the EMM cause value to #22 "congestion" and assign a back-off timer T3346.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, and take the following actions depending on the EMM cause value received.

…

#22	(Congestion);

If the T3346 value IE is present in the TRACKING AREA UPDATE REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below, otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.3.2.6.

The UE shall abort the tracking area updating procedure, reset the tracking area updating attempt counter and set the EPS update status to EU2 NOT UPDATED. If the rejected request was not for initiating a PDN connection for emergency bearer services, the UE shall change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE.

The UE shall stop timer T3346 if it is running.

If the TRACKING AREA UPDATE REJECT message is integrity protected, the UE shall start timer with the value provided in the T3346 value IE.

If the TRACKING AREA UPDATE REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [13].

The UE stays in the current serving cell and applies the normal cell reselection process. The tracking area updating procedure is started, if still necessary, when timer T3346 expires or is stopped.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.1.20a.3	Test description

9.2.3.1.20a.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'';

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	none

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.20a.3.2	Test procedure sequence

Table 9.2.3.1.20a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = '' Congestion '' and T3346 IE set to 5 minutes. The UE starts timer T3346 with the value provided in the T3346 value IE | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message before timer T3346 has expired? | - | - | 1 | F |
| 6 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message after timer T3346 has expired? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 7 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 8 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.20a.3.3	Specific message contents

Table 9.2.3.1.20a.3.3-1: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.20a.3.2-1)

| Derivation path: 36.508 table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00010110'B | #22 ''Congestion'' |  |
| T3346 value | 00100101'B | 5 min |  |

NOTE:	This message is sent within SECURITY PROTECTED NAS MESSAGE message.

##### 9.2.3.1.21	Void

##### 9.2.3.1.22	Normal tracking area update / Abnormal case / access barred due to access class control or NAS signalling connection establishment rejected by the network

9.2.3.1.22.1	Test Purpose (TP)

(1)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is barred for signalling in the cell UE is camping [Access Class barred in System information] }

then { the UE will not initiate the tracking area updating procedure on the current cell }

}

(2)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is barred for signalling in the cell UE is camping [T302 running due to RRCConnectionReject message reception] }

then { the UE will not initiate the tracking area updating procedure on the current cell }

}

(3)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is not barred for signalling in the cell UE is camping }

then { the UE will initiate the tracking area updating procedure on the current cell }

}

(4)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access was barred for signalling in the cell and UE has reselected an new cell where access for "signalling" is granted }

then { the UE will initiate the tracking area updating procedure on the new cell }

}

9.2.3.1.22.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.2.6 and TS 36.331, clause 5.3.3.2

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

a)	Access barred because of access class barring or NAS signalling connection establishment rejected by the network

If access is barred for "signalling" (see 3GPP TS 36.331 [22]), the tracking area updating procedure shall not be started. The UE stays in the current serving cell and applies the normal cell reselection process. The tracking area updating procedure is started as soon as possible and if still necessary, e.g. when access for "signalling" is granted on the current cell or when the UE moves to a cell where access for "signalling" is granted.

[TS 36.331, clause 5.3.3.2]

…

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

9.2.3.1.22.3	Test description

9.2.3.1.22.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B are configured according to table 6.3.2.2-1 in TS 36.508 [18].

-	cell A belongs to TAI-1 (home PLMN)

-	cell B belongs to TAI-2 (home PLMN)

-	cell D belongs to TAI-4 (home PLMN)

UE:

None;

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell B according to TS 36.508 [18].

9.2.3.1.22.3.2	Test procedure sequence

Table 9.2.3.1.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sets the cell type of cell A to the ''Serving cell'', sets the cell type of cell B to the '' Non-Suitable cell'', and sets SystemInformationBlockType2 parameters as described below. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: for 60 seconds if UE initiates the tracking area updating procedure on cell A? | - | - | 1 | F |
| 3 | The SS transmits a Paging message including systemInfoModification. | - | - | - | - |
| 4 | The SS changes SystemInformationBlockType2 parameters to default parameters defined in [18]. | - | - | - | - |
| 5 | The UE transmits RRC Connection Request | - | - | - | - |
| 6 | SS responds with RRCConnectionReject message with IE waitTime set to 10 seconds(Max Value). | - | - | - | - |
| 7 | Check: for 10 seconds if UE initiates the tracking area updating procedure and hence transmits RRC Connection Request? | - | - | 2 | F |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 9 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 10 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 10A | The SS releases the RRC connection. | - | - | - | - |
| 11 | The SS set SystemInformationBlockType1 and SystemInformationBlockType2 parameters as described below. | - | - | - | - |
| 12 | Void | - | - |  |  |
| 13 | The SS sets the cell type of cell B to the '' serving cell'', cell A to “non-suitable off Cell” , and cell D to “Suitable neighbour intra-frequency cell”. |  |  |  |  |
| 13A | Check: For 60 seconds if the UE initiates the tracking area update procedure on Cell B? | - | - | 1 | F |
| 13B | The SS sets the cell type of cell B to “Suitable neighbour intra-frequency cell” and cell D to “serving cell” | - | - | - | - |
| - | The following messages are to be observed on Cell D unless explicitly stated otherwise. | - | - | - | - |
| 14 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 15 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 16 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508 | - | - | - | - |

9.2.3.1.22.3.3	Specific message contents

Table 9.2.3.1.22.3.3-1: SystemInformationBlockType2 for Cell A (step 1, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| accessBarringInformation SEQUENCE { |  |  |  |
| accessBarringForEmergencyCalls | FALSE |  |  |
| accessBarringForSignalling SEQUENCE { |  |  |  |
| accessProbabilityFactor | p00 |  |  |
| accessBarringTime | s4 |  |  |
| accessClassBarringList SEQUENCE (SIZE (maxAC)) OF SEQUENCE { | 5 entries |  |  |
| accessClassBarring[1] | TRUE |  |  |
| accessClassBarring[2] | TRUE |  |  |
| accessClassBarring[3] | TRUE |  |  |
| accessClassBarring[4] | TRUE |  |  |
| accessClassBarring[5] | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| accessBarringForOriginatingCalls | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-1A: SystemInformationBlockType1 for Cell A (step 4, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-1AA: SystemInformationBlockType1-BR-r13 for Cell A (step 4 when UE under test is CAT-M1, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-2: SystemInformationBlockType2 for Cell B (step 11, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| accessBarringInformation SEQUENCE { |  |  |  |
| accessBarringForEmergencyCalls | FALSE |  |  |
| accessBarringForSignalling SEQUENCE { |  |  |  |
| accessProbabilityFactor | p00 |  |  |
| accessBarringTime | s4 |  |  |
| accessClassBarringList SEQUENCE (SIZE (maxAC)) OF SEQUENCE { | 5 entries |  |  |
| accessClassBarring[1] | TRUE |  |  |
| accessClassBarring[2] | TRUE |  |  |
| accessClassBarring[3] | TRUE |  |  |
| accessClassBarring[4] | TRUE |  |  |
| accessClassBarring[5] | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| accessBarringForOriginatingCalls | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-3: SystemInformationBlockType1 for Cell B (step 11, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-4: SystemInformationBlockType1-BR-r13 for Cell B (step 11, Table 9.2.3.1.22.3.2-1 when UE under test is CAT-M1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 9.2.3.1.23	Normal tracking area update / Abnormal case / Success after several attempts due to no network response / TA belongs to TAI list and status is UPDATED / TA does not belong to TAI list or status is not UPDATED

9.2.3.1.23.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Periodic updating' and has the tracking area updating attempt counter set to the value less than four, the TAI of the current serving cell is included in the TAI list and the update status is equal to EU1 UPDATED }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE keeps the update status to EU1 UPDATED, enters state EMM-REGISTERED.NORMAL-SERVICE and starts timer T3411 }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Periodic updating', has the tracking area updating attempt counter set to the value less than four, has detected T3430 expiry, the TAI of the current serving cell is included in the TAI list and the update status is equal to EU1 UPDATED }

ensure that {
  when { UE detects T3411 expiry }

then { UE initiates the tracking area updating procedure }

}

(3)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'TA updating' or 'combined TA/LA updating' and has the tracking area updating attempt counter set to the value less than four and the TAI of the current serving cell is not included in the TAI list or the update status is different to EU1 UPDATED }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE starts timer T3411, sets the update status to EU2 NOT UPDATED and changes to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE }

}

(4)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'TA updating' or 'combined TA/LA updating', has the tracking area updating attempt counter set to the value less than four, has detected T3430 expiry and the TAI of the current serving cell is not included in the TAI list or the update status is different to EU1 UPDATED }

ensure that {
  when { UE detects T3411 expiry }

then { UE initiates the tracking area updating procedure }

}

9.2.3.1.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.1, 5.5.3.2.6 and 5.5.3.3.6.

[TS 24.301, clause 5.5.3.1]

A tracking area updating attempt counter is used to limit the number of subsequently rejected tracking area update attempts. The tracking area updating attempt counter shall be incremented as specified in subclause 5.5.3.2.6. Depending on the value of the tracking area updating attempt counter, specific actions shall be performed. The tracking area updating attempt counter shall be reset when:

-	an attach or combined attach procedure is successfully completed;

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is successfully completed; or

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is rejected with EMM cause #11, #12, #13, #14, #15 or #25.

Additionally the tracking area updating attempt counter shall be reset when the UE is in substate EMM-REGISTERED.ATTEMPTING-TO-UPDATE or EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, and:

-	a new tracking area is entered; or

-	timer T3402 expires.

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

...

b)	Lower layer failure or release of the NAS signalling connection before the TRACKING AREA UPDATE ACCEPT or TRACKING AREA UPDATE REJECT message is received

The tracking area updating procedure shall be aborted, and the UE shall proceed as described below.

c)	T3430 timeout

The UE shall abort the procedure and proceed as described below. The NAS signalling connection shall be released locally.

...

For the cases b, c, d, e, and f, the UE shall stop any ongoing transmission of user data.

For the cases b, c and d the UE shall proceed as follows:

Timer T3430 shall be stopped if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is included in the TAI list and the EPS update status is equal to EU1 UPDATED:

-	the UE shall keep the EPS update status to EU1 UPDATED and enter state EMM-REGISTERED.NORMAL-SERVICE. The UE shall start timer T3411.

If in addition the TRACKING AREA UPDATE REQUEST indicated "periodic updating", the timer T3411 may be stopped when the UE enters EMM-CONNECTED mode.

If  timer T3411 expires the tracking area updating procedure is triggered again.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is not included in the TAI list or the EPS update status is different to EU1 UPDATED:

-	the UE shall start timer T3411, shall set the EPS update status to EU2 NOT UPDATED and change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE. When timer T3411 expires the tracking area updating procedure is triggered again.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GPRS update status as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal or periodic routing area updating procedure fails and the routing area updating attempt counter is less than 5 and the GPRS update status is different from GU1 UPDATED.

[TS 24.301, clause 5.5.3.3.6]

If the tracking area updating attempt counter is incremented according to subclause 5.5.3.2.6 the next actions depend on the value of the tracking area updating attempt counter.

-	if the update status is U1 UPDATED and the tracking area updating attempt counter is less than 5, then the UE shall keep the update status to U1 UPDATED, the new MM state is MM IDLE substate NORMAL SERVICE;

-	if the tracking area updating attempt counter is less than 5 and, additionally, the update status is different from U1 UPDATED UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. The MM state remains MM LOCATION UPDATING PENDING; or

-	if the tracking area updating attempt counter is equal to 5, the UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. A UE operating in CS/PS mode 1 of operation shall select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures.

9.2.3.1.23.3	Test description

9.2.3.1.23.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

none.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.1.23.3.2	Test procedure sequence

Table 9.2.3.1.23.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is powered on or switched on. | - | - | - | - |
| 3-8a2 | Void | - | - | - | - |
| 8A1-8A10 | Steps 2-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 8A | The SS activates UE radio bearer test mode. | - | - | - | - |
| 9 | The SS responds with an ATTACH ACCEPT message with the T3412 value indicating 6 min. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 11A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 11B-11Db1 | Steps 16 to 18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | Wait for 6 min to ensure that T3412 expires. | - | - | - | - |
| 13 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 14 | The SS releases the RRC connection. NOTE: The tracking area updating attempt counter is 1. | - | - | - | - |
| 15 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 16 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 1 | P |
| 17 | The SS transmits a SERVICE REJECT message with EMM cause set to "Congestion". | <-- | SERVICE REJECT | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| 19 | IF pc_ue_CategoryDL_M1 wait for up to 11,25 else wait for 10s after step 14 to ensure that T3411 expires. (Note 2) | - | - | - | - |
| - | EXCEPTION: Steps 19Aa1 to 19Aa4 describes behaviour depending on UE behaviour; the “lower case letter” identifies a step sequence that take place if the UE does not transmit any TRACKING AREA UPDATE REQUEST message | - | - | - | - |
| 19Aa1 | IF the UE does not transmit any TRACKING AREA UPDATE REQUEST message THEN wait for 6 min to ensure that T3412 expires. | - | - | - | - |
| 19Aa2 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to ‘Periodic updating’. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 19Aa3 | The SS releases the RRC connection. | - | - | - | - |
| 19Aa4 | Wait for 10s after step 19Aa3 to ensure that T3411 expires. | - | - | - | - |
| 20 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. NOTE: The tracking area updating attempt counter is reset. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The SS releases the RRC connection. | - | - | - | - |
| 22A | Generic test procedure in TS 36.508 subclause 4.5.3A.3 is performed. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 22B | Generic test procedure in TS 36.508 subclause 4.5.4.3 is performed. NOTE: The UE enters the UE test loop mode. | - | - | - | - |
| 22C | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | - | - | - | - |
| 22D | Wait for 1 s after the IP packet has been transmitted in step 22C. (Note 1) | - | - | - | - |
| 22E | The SS releases the RRC connection. | - | - | - | - |
| 23 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 24 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating' or ‘combined TA/LA updating’. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 24A | Wait for 15s after step 24 to ensure that T3430 expires. | - | - | - | - |
| 25 | The SS releases the RRC connection. NOTE: The tracking area updating attempt counter is 1. | - | - | - | - |
| 26 | Void | - | - | - | - |
| 27 | Check: Does the UE transmit a SERVICE REQUEST message within 5s? | --> | SERVICE REQUEST | 3 | F |
| 28 | Wait for 10s after step 25 to ensure that T3411 expires. | - | - | - | - |
| 29 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating' or ‘combined TA/LA updating’? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 30 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | EXCEPTION: Step 31 and Step 32a1 can occur in any order. | - | - | - | - |
| 31 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. NOTE: The tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Step 32a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 32a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 22C on the DRB associated with the default EPS bearer context on Cell B within 5s. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 22C to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 22E. Note 2: According to TS 36.331 cl. 5.3.8.3 Cat M1 UEs may start T3411 only 1,25 sec after having received RRC Connection Release. T3411 will therefore elapse in the interval from 9 sec to 12.375 sec. |  |  |  |  |  |

9.2.3.1.23.3.3	Specific message contents

Table 9.2.3.1.23.3.3-0: ACTIVATE TEST MODE (step 8A, Table 9.2.3.1.23.3.2-1)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE B

Table 9.2.3.1.23.3.3-1: Message ATTACH ACCEPT (step 9, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| T3412 value |  | 6 minutes |  |
| Timer value | '0 0001'B |  |  |
| Unit | '010'B | value is incremented in multiples of decihours |  |
| GUTI | GUTI-1 |  |  |

Table 9.2.3.1.23.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 13, step 19Aa2 and step 20, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '011'B | Periodic updating |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 21, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| GUTI |  | Not present |  |  |  |  |  |
| MS identity |  | Not present |  |  |  |  |  |

Table 9.2.3.1.23.3.3-3A: CLOSE UE TEST LOOP (step 22B, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0001 0100'B | 20seconds |  |

Table 9.2.3.1.23.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 24, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 29, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| "Active" flag | Any allowed value | The UE may set this flag due to failing SERVICE REQUEST procedure. |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 30, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |

##### 9.2.3.1.24	Void

##### 9.2.3.1.25	Normal tracking area update / Abnormal case / Failure after 5 attempts due to no network response

9.2.3.1.25.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message and has the tracking area updating attempt counter set to four }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE starts timer T3402, sets the update status to EU2 NOT UPDATED, changes to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE or optionally to EMM-REGISTERED.PLMN-SEARCH in order to perform a PLMN selection }

}

9.2.3.1.25.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.1 and 5.5.3.2.6.

[TS 24.301, clause 5.5.3.1]

A tracking area updating attempt counter is used to limit the number of subsequently rejected tracking area update attempts. The tracking area updating attempt counter shall be incremented as specified in subclause 5.5.3.2.6. Depending on the value of the tracking area updating attempt counter, specific actions shall be performed. The tracking area updating attempt counter shall be reset when:

-	an attach or combined attach procedure is successfully completed;

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is successfully completed; or

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is rejected with EMM cause #11, #12, #13, #14, #15 or #25.

Additionally the tracking area updating attempt counter shall be reset when the UE is in substate EMM-REGISTERED.ATTEMPTING-TO-UPDATE or EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, and:

-	a new tracking area is entered; or

-	timer T3402 expires.

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

...

b)	Lower layer failure or release of the NAS signalling connection before the TRACKING AREA UPDATE ACCEPT or TRACKING AREA UPDATE REJECT message is received

The tracking area updating procedure shall be aborted, and the UE shall proceed as described below.

...

For the cases b, c, d, e, and f, the UE shall stop any ongoing transmission of user data.

For the cases b, c and d the UE shall proceed as follows:

Timer T3430 shall be stopped if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

...

If the tracking area updating attempt counter is equal to 5:

-	the UE shall start timer T3402, shall set the EPS update status to EU2 NOT UPDATED, shall delete the list of equivalent PLMNs and shall change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE or optionally to EMM-REGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GPRS update status as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal or periodic routing area updating procedure fails and the routing area updating attempt counter is equal to 5.

9.2.3.1.25.3	Test description

9.2.3.1.25.3.1	Pre-test conditions

System Simulator:

-	Cell A (belongs to TAI-1, PLMN1);

-	Cell B (belongs to TAI-2, PLMN1);

-	Cell C (belongs to TAI-7, PLMN2);

UE:

- 	The UE is configured to initiate EPS attach;

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.1.25.3.2	Test procedure sequence

Table 9.2.3.1.25.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "non-Suitable cell". Set the cell type of Cell C to the "non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is powered on or switched on. | - | - | - | - |
| 3-8D | Steps 2-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | SS responds with ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| 10-11Bb1 | Steps 16-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | Wait for 6 min to ensure that T3412 expires. | - | - | - | - |
| 13 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 14 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 2: The tracking area updating attempt counter is 1. | - | - | - | - |
| 15 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 16 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 3: The tracking area updating attempt counter is 2. | - | - | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 18 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 4: The tracking area updating attempt counter is 3. | - | - | - | - |
| 19 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 20 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 5: The tracking area updating attempt counter is 4. | - | - | - | - |
| 21 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 22 | The SS releases the RRC connection. NOTE 6: The tracking area updating attempt counter is 5. | - | - | - | - |
| 23 | Wait for 12 min to ensure that T3402 expires. | - | - | - | - |
| 24 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'? NOTE 7: The tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 25 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 26 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 27 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'. NOTE 8: The tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 28 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 9: The tracking area updating attempt counter is 1. | - | - | - | - |
| 29 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 30 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 10: The tracking area updating attempt counter is 2. | - | - | - | - |
| 31 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 32 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 11: The tracking area updating attempt counter is 3. | - | - | - | - |
| 33 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 34 | Wait for T3430 and T3411 (25s) expiry (NOTE 1). NOTE 12: The tracking area updating attempt counter is 4. | - | - | - | - |
| 35 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 36 | The SS releases the RRC connection. NOTE 13: The tracking area updating attempt counter is 5 and reset. | - | - | - | - |
| 37 | Set the cell type of Cell B to the "non-Suitable cell". Set the cell type of Cell C to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 38 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating'? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 39 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 40 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 41 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 42a1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 42a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle (E1) according to TS 36.508. | - | - | - | - |
| NOTE 1: If the UE supports CE mode B and UE’s usage setting is not voice centric, T3430 takes a value of 77 sec (see TS 24.301 [28] clause 10.2). Combined with T3411 this gives an expiry value of 87s then. |  |  |  |  |  |

9.2.3.1.25.3.3	Specific message contents

Table 9.2.3.1.25.3.3-1: Message ATTACH ACCEPT (step 9, Table 9.2.3.1.25.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| T3412 value |  | 6 minutes |  |
| Timer value | '0 0001'B |  |  |
| Unit | '010'B | value is incremented in multiples of decihours |  |
| GUTI | GUTI-1 |  |  |

Table 9.2.3.1.25.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 13, step 15, step 17, step 19 and step 21, Table 9.2.3.1.25.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '011'B | Periodic updating |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.25.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 24, step 27, step 29, step 31, step 33, step 35 and step 38, Table 9.2.3.1.25.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | ‘000’B | TA updating |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.25.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 25, Table 9.2.3.1.25.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | Not present |  |  |

Table 9.2.3.1.25.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 39, Table 9.2.3.1.25.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-3 |  |  |

##### 9.2.3.1.26	Normal tracking area update / Abnormal case / TRACKING AREA UPDATE REJECT

9.2.3.1.26.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with reject cause #95"semantically incorrect message" }

then { the UE sets the tracking area updating attempt counter to 5, starts timer T3402, and performs tracking area updating on the expiry of timers T3402}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with reject cause #96" invalid mandatory information"  }

then { the UE sets the tracking area updating attempt counter to 5, starts timer T3402, and performs tracking area updating on the expiry of timers T3402}

(3)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with reject cause #97"message type non-existent or not implemented" }

then { the UE sets the tracking area updating attempt counter to 5, starts timer T3402, and performs tracking area updating on the expiry of timers T3402}

(4)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with reject cause #99"information element non-existent or not implemented" }

then { the UE sets the tracking area updating attempt counter to 5, starts timer T3402, and performs tracking area updating on the expiry of timers T3402}

(5)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with reject cause #111"protocol error, unspecified" }

then { the UE sets the tracking area updating attempt counter to 5, starts timer T3402, and performs tracking area updating on the expiry of timers T3402}

9.2.3.1.26.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.6.

[TS 24.301 clause5.5.3.2.6]

...

d)	TRACKING AREA UPDATE REJECT, other causes than those treated in subclause 5.5.3.2.5

Upon reception of the EMM causes #95, #96, #97, #99 and #111 the UE should set the tracking area updating attempt counter to 5. The UE shall proceed as described below.

If the tracking area updating attempt counter is equal to 5:

-	the UE shall start timer T3402, shall set the EPS update status to EU2 NOT UPDATED, shall delete the list of equivalent PLMNs and shall change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE or optionally to EMM-REGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

9.2.3.1.26.3	Test description

9.2.3.1.26.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	the UE is configured to initiate EPS attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18] except for those shown in table 9.2.3.1.26.3.3-6;

9.2.3.1.26.3.2	Test procedure sequence

Table 9.2.3.1.26.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS waits for T3412 to elapse. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause #95"semantically incorrect message" as specified. (Note) | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: When the timer T3402 expires does the UE re-transmit TRACKING AREA UPDATE REQUEST message on cell A? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 6 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 7A | The SS releases the RRC connection. | - | - | - | - |
| 8 | The SS waits for T3412 to elapse. | - | - | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 10 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause #96" invalid mandatory information" as specified. (Note) | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 11 | The SS releases the RRC connection. | - | - | - | - |
| 12 | Check: When the timer T3402 expires does the UE re-transmit TRACKING AREA UPDATE REQUEST message on cell A? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 13 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 14 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 14A | The SS releases the RRC connection. | - | - | - | - |
| 15 | The SS waits for T3412 to elapse | - | - | - | - |
| 16 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 17 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause #97"message type non-existent or not implemented". (Note) | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| 19 | Check: When the timer T3402 expires does the UE re-transmit TRACKING AREA UPDATE REQUEST message on cell A? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 20 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 21 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 21A | The SS releases the RRC connection. | - | - | - | - |
| 22 | The SS waits for T3412 to elapse. | - | - | - | - |
| 23 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 24 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause #99"information element non-existent or not implemented" as specified. (Note) | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 25 | The SS releases the RRC connection. | - | - | - | - |
| 26 | Check: When the timer T3402 expires does the UE re-transmit TRACKING AREA UPDATE REQUEST message on cell A? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 27 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 28 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 28A | The SS releases the RRC connection. | - | - | - | - |
| 29 | The SS waits for T3412 to elapse. | - | - | - | - |
| 30 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 31 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause #111"protocol error, unspecified" as specified. (Note) | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 32 | The SS releases the RRC connection. | - | - | - | - |
| 33 | Check: When the timer T3402 expires does the UE re-transmit TRACKING AREA UPDATE REQUEST message on cell A? | --> | TRACKING AREA UPDATE REQUEST | 5 | P |
| 34 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 35 | the UE transmit a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note: Upon reception of TRACKING AREA UPDATE REJECT message with EMM causes #95, #96, #97, #99 and #111, timer T3402 shall be started. |  |  |  |  |  |

9.2.3.1.26.3.3	Specific message contents

Table 9.2.3.1.26.3.3-1: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01011111 | #95 "semantically incorrect message " |  |

Table 9.2.3.1.26.3.3-2: Message TRACKING AREA UPDATE REJECT (step 10, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01100000 | #96 " invalid mandatory information " |  |

Table 9.2.3.1.26.3.3-3: Message TRACKING AREA UPDATE REJECT (step 17, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01100001 | #97 " message type non-existent or not implemented " |  |

Table 9.2.3.1.26.3.3-4: Message TRACKING AREA UPDATE REJECT (step 24, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01100011 | #99 " information element non-existent or not implemented " |  |

Table 9.2.3.1.26.3.3-5: Message TRACKING AREA UPDATE REJECT (step 31, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01101111 | #111 " protocol error, unspecified " |  |

Table 9.2.3.1.26.3.3-6: Message ATTACH ACCEPT (For the UE registration procedure in TS 36.508 clause 4.5.2.3)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  | 1 minute |  |
| Timer value | '00001'B |  |  |
| Unit | '001'B |  |  |
| T3402 value |  | 30 seconds |  |
| Timer value | ‘01111’B |  |  |
| Unit | ‘000’B |  |  |

Table 9.2.3.1.26.3.3-7: Message TRACKING AREA UPDATE ACCEPT (steps 6,13,20,27,34, Table 9.2.3.1.26.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  | 1 minute |  |
| Timer value | '00001'B |  |  |
| Unit | '001'B |  |  |
| T3402 value |  | 30 seconds |  |
| Timer value | ‘01111’B |  |  |
| Unit | ‘000’B |  |  |

Table 9.2.3.1.26.3.3-8: Message TRACKING AREA UPDATE REQUEST (steps 2,9,16,23,30 Table 9.2.3.1.26.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '011'B | Periodic updating |  |

##### 9.2.3.1.27	Normal tracking area update / Abnormal case / Change of cell into a new tracking area

9.2.3.1.27.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { cell change into a new tracking area occurs before the tracking area updating procedure is completed }

then { UE aborts the tracking area updating procedure and re-initiates it in the new tracking area immediately }

}

9.2.3.1.27.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.6, and 5.5.3.3.6.

[TS 24.301 clause5.5.3.2.6]

The following abnormal cases can be identified:

…

e)	Change of cell into a new tracking area

If a cell change into a new tracking area occurs before the tracking area updating procedure is completed, the tracking area updating procedure shall be aborted and re-initiated immediately. The UE shall set the EPS update status to EU2 NOT UPDATED.

[TS 24.301 clause5.5.3.3.6]

The UE shall proceed as follows:

...

-	otherwise, the abnormal cases specified in subclause 5.5.3.2.6 apply with the following modification.

If the tracking area updating attempt counter is incremented according to subclause 5.5.3.2.6 the next actions depend on the value of the tracking area updating attempt counter.

9.2.3.1.27.3	Test description

9.2.3.1.27.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B.

UE:

None.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to 36.508 [18].

9.2.3.1.27.3.2	Test procedure sequence

Table 9.2.3.1.27.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B (Note 1). | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | SS does not send TRACKING AREA UPDATE ACCEPT to the UE and update TAC value in SystemInformationBlockType1. | - | - | - | - |
| 4 | The SS transmits a Paging message paging occasion including a systemInfoModification. | <-- | Paging | - | - |
| - | EXCEPTION: IF pc_ue_CategoryDL_M1 THEN execute 5b1 to 5b3 ELSE execute step 5a1. | - | - | - | - |
| 5 a1 | From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1 as specified. | - | - | - | - |
| 5b1 | The SS transmits an RRCConnectionRelease message in Cell B. | - | - | - | - |
| 5b2 | Void | - | - | - | - |
| 5b3 | The SS indicates a new system information by Direct Indication information with bit 1 and transmitted on MPDCCH using P-RNTI. From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1-BR-r13. | - | - | - | - |
| 6 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message on cell B in the next 12 seconds? Note: Wait time is more than 2.1* modification period for the UE to receive system information and inferior to T3430. | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 7 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 8 | UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note 1: Any type of tracking area update is acceptable. |  |  |  |  |  |

9.2.3.1.27.3.3	Specific message contents

Table 9.2.3.1.27.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, step 6, Table 9.2.3.1.27.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.27.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 7, 9.2.3.1.27.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-3 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B |  |  |
| Partial tracking area identity list |  |  |  |
| Number of elements | '00000'B |  |  |
| Type of list | '00'B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| MCC MNC TAC 1 | TAC =4 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-4'' |  |

Table 9.2.3.1.27.3.3-3: SystemInformationBlockType1 for Cell B (From step 3 in Table 9.2.3.1.27.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| trackingAreaCode | TAC =4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.27.3.3-3A: SystemInformationBlockType1-BR-r13 for Cell B (From step 3 in Table 9.2.3.1.27.3.2-1 when UE under test is CAT M1)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| trackingAreaCode | TAC =4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.27.3.3-4: Paging (step 4, Table 9.2.3.1.27.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

##### 9.2.3.1.28	Normal tracking area update / Abnormal case / Tracking area updating and detach procedure collision

9.2.3.1.28.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message}

ensure that {

when { UE receives a DETACH REQUEST message before the tracking area updating procedure has been completed }

then { the tracking area updating procedure shall be aborted and the detach procedure shall be progressed }

}

9.2.3.1.28.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.5.3.2.6 and 5.5.3.3.6.

[TS 24.301 clause5.5.3.2.6]

The following abnormal cases can be identified:

…

f)	Tracking area updating and detach procedure collision

If the UE receives a DETACH REQUEST message before the tracking area updating procedure has been completed, the tracking area updating procedure shall be aborted and the detach procedure shall be progressed.

[TS 24.301 clause5.5.3.3.6]

The UE shall proceed as follows:

...

-	otherwise, the abnormal cases specified in subclause 5.5.3.2.6 apply with the following modification.

If the tracking area updating attempt counter is incremented according to subclause 5.5.3.2.6 the next actions depend on the value of the tracking area updating attempt counter.

9.2.3.1.28.3	Test description

9.2.3.1.28.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on Cell A according to 36.508 [18].

9.2.3.1.28.3.2	Test procedure sequence

Table 9.2.3.1.28.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B (Note 1). | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | SS does not send TRACKING AREA UPDATE ACCEPT to the UE. | - | - | - | - |
| 4 | The SS transmits a DETACH REQUEST message with Detach type = 're-attach not required' on Cell B. Note: this message should be sent before timer T3430 expired. | <-- | DETACH REQUEST | - | - |
| 5 | Check: Does the UE transmit a DETACH ACCEPT message on Cell B in the next 6 seconds? Note: the default value for timer T3422 is 6 seconds | --> | DETACH ACCEPT | 1 | P |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | Check: Does the test result of the generic procedure defined in clause 6.4.2.5 of TS 36.508 indicates that the UE responds to paging when paged with S-TMSI1 and with CN domain indicator set to "PS" on Cell B? | - | - | 1 | - |
| Note 1: Any type of tracking area update is acceptable. |  |  |  |  |  |

9.2.3.1.28.3.3	Specific message contents

Table 9.2.3.1.28.3.3-1: Message DETACH REQUEST (step 4, Table 9.2.3.1.28.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | ‘010’B | “re-attach not required” |  |
| EMM cause | '00001100'B | ''Tracking area not allowed'' |  |

##### 9.2.3.1.29	Normal tracking area update / Accepted / MUSIM

9.2.3.1.29.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { the UE detects entering a new tracking area not included in the TAI list }

then { the UE transmits TRACKING AREA UPDATE REQUEST message  and sets one or more MUSIM feature bits in UE network capability IE to "supported"(i.e. the NAS signalling connection release, the paging indication for voice services, the reject paging request, the paging restriction and the paging timing collision control) }

}

9.2.3.1.29.2	Conformance requirements

[TS 24.301, clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME, unless the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] and is entering a tracking area in a new PLMN that is neither the registered PLMN nor in the list of equivalent PLMNs;

b)	when the periodic tracking area updating timer T3412 expires;

c)	when the UE enters EMM-REGISTERED.NORMAL-SERVICE and the UE's TIN indicates "P-TMSI";

d)	when the UE performs an inter-system change from S101 mode to S1 mode and has no user data pending;

e)	when the UE receives an indication from the lower layers that the RRC connection was released with cause "load balancing TAU required";

f)	when the UE deactivated EPS bearer context(s) locally while in EMM-REGISTERED, because it could not establish a NAS signalling connection, and then returns to EMM-REGISTERED.NORMAL-SERVICE and no EXTENDED SERVICE REQUEST message, CONTROL PLANE SERVICE REQUEST message or DETACH REQUEST message with detach type is "EPS detach" or "combined EPS/IMSI detach" is pending to be sent by the UE;

g)	when the UE changes any one of the UE network capability information, the MS network capability information or the N1 UE network capability information;

h)	when the UE changes the UE specific DRX parameter (in WB-S1 mode or NB-S1 mode);

i)	when the UE receives an indication of "RRC Connection failure" from the lower layers and has no signalling or user uplink data pending (i.e. when the lower layer requests NAS signalling connection recovery);

j)	when the UE enters S1 mode after 1xCS fallback or 1xSRVCC;

k)	when due to manual CSG selection the UE has selected a CSG cell whose CSG identity and associated PLMN identity are not included in the UE's Allowed CSG list or in the UE's Operator CSG list;

l)	when the UE reselects an E-UTRAN cell while it was in GPRS READY state or PMM-CONNECTED mode;

m)	when the UE supports SRVCC to GERAN or UTRAN or supports vSRVCC to UTRAN and changes the mobile station classmark 2 or the supported codecs, or the UE supports SRVCC to GERAN and changes the mobile station classmark 3;

n)	when the UE changes the radio capability for GERAN, or cdma2000® or both;

o)	when the UE's usage setting or the voice domain preference for E-UTRAN change in the UE;

NOTE 1:	For the change of UE's usage setting or the voice domain preference for E-UTRAN which results in disabling UE's E-UTRA capability, the UE can skip sending TRACKING AREA UPDATE REQUEST message and directly perform disabling of UE's E-UTRA capability.

p)	when the UE activates mobility management for IMS voice termination as specified in 3GPP TS 24.008 [13], annex P.2, and the TIN indicates "RAT-related TMSI";

q)	when the UE performs an inter-system change from A/Gb mode to S1 mode and the TIN indicates "RAT-related TMSI", but the UE is required to perform tracking area updating for IMS voice termination as specified in 3GPP TS 24.008 [13], annex P.4;

r)	upon reception of a paging indication using S-TMSI and the UE is in state EMM-REGISTERED.ATTEMPTING-TO-UPDATE;

s)	when the UE needs to update the network with EPS bearer context status due to local de-activation of EPS bearer context(s) as specified in clause 6.5.1.4A;

t)	when the UE needs to request the use of PSM or needs to stop the use of PSM;

u)	when the UE needs to request the use of eDRX or needs to stop the use of eDRX;

v)	when a change in the eDRX usage conditions at the UE requires different extended DRX parameters;

w)	when a change in the PSM usage conditions at the UE requires a different timer T3412 value or different timer T3324 value;

NOTE 2:	A change in the PSM or eDRX usage conditions at the UE can include e.g. a change in the UE configuration, a change in requirements from upper layers or the battery running low at the UE.

x)	when the CIoT EPS optimizations the UE needs to use, change in the UE;

y)	when the Default_DCN_ID value changes, as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17];

NOTE 3:	The tracking area updating procedure is initiated after deleting the DCN-ID list as specified in annex C.

z)	when the UE performs inter-system change from N1 mode to S1 mode in EMM-IDLE mode, the UE operates in single-registration mode, and conditions specified in 3GPP TS 24.501 [54] apply;

za)	when the UE in EMM-IDLE mode changes the radio capability for E-UTRAN;

zb)	when the UE needs to request new ciphering keys for ciphered broadcast assistance data;

zc)	when the UE in EMM-IDLE mode changes the radio capability for NG-RAN;

zd)	when the UE performs inter-system change from N1 mode to S1 mode in EMM-CONNECTED mode;

ze)	in WB-S1 mode, when the applicable UE radio capability ID for the current UE radio configuration changes due to a revocation of the network-assigned UE radio capability IDs by the serving PLMN;

zf)	when the UE needs to use the WUS assistance, stop to use the WUS assistance, or change the conditions for using the WUS assistance; or

zg)	when the MUSIM UE needs to request an IMSI Offset value as specified in 3GPP TS 23.401 [10] that is used for deriving the paging occasion as specified in 3GPP TS 36.304 [21]

…

For all cases except case b, if the MUSIM UE supports the NAS signalling connection release, then the UE shall set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the MUSIM UE supports the paging indication for voice services, then the UE shall set the paging indication for voice services bit to "paging indication for voice services supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the paging indication for voice services bit to "paging indication for voice services supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the MUSIM UE supports the reject paging request, then the UE shall set the reject paging request bit to "reject paging request supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the reject paging request bit to "reject paging request supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the MUSIM UE sets:

-	the reject paging request bit to "reject paging request supported";

-	the NAS signalling connection release bit to "NAS signalling connection release supported"; or

-	both of them;

and supports the paging restriction, then the UE shall set the paging restriction bit to "paging restriction supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the paging restriction bit to "paging restriction supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the MUSIM UE supports the paging timing collision control, then the UE shall set the paging timing collision control bit to "paging timing collision control supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the paging timing collision control bit to "paging timing collision control supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

…

9.2.3.1.29.3	Test description

9.2.3.1.29.3.1	Pre-test conditions

System Simulator:

-	Cell A/Cell B(home PLMN1) and Cell G/Cell H(home PLMN2) are configured according to table 6.3.2.2-1 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

-	Cell A (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell'' for USIM A.

-	Cell B (belongs to TAI-2, home PLMN1, MCC=001,MNC=01) is set to '' Non- Suitable cell'' for USIM A.

-	Cell G (belongs to TAI-7, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell'' for USIM B.

-	Cell H (belongs to TAI-8, home PLMN2, MCC=001,MNC=02) is set to '' Non- Suitable cell '' for USIM B.

UE:

-	the UE is a MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Table 4.5.2E.1-1 and Table 4.5.2E.1-2.roaming is disabled

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.5.2E.3 of TS 36.508 [18].

9.2.3.1.29.3.2	Test procedure sequence

Table 9.2.3.1.29.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 1 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the " Serving cell" | - | - | - | - |
| 2 | Check: Does the USIM A of UE transmit a TRACKING AREA UPDATE REQUEST message including UE network capability which at least one MUSIM feature is set to “support”? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The USIM A of UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. |  |  | - | - |
|  | The following messages are sent and shall be received on cell H. | - | - | - | - |
| 6 | Set the cell type of cell G to the "Non-Suitable cell". Set the cell type of cell H to the " Serving cell" | - | - | - | - |
| 7 | Check: Does the USIM B of UE transmit a TRACKING AREA UPDATE REQUEST message including UE network capability which at least one MUSIM feature is set to “support”? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The USIM B of UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 10 | The SS releases the RRC connection. |  |  | - | - |

9.2.3.1.29.3.3	Specific message contents

Table 9.2.3.1.29.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.29.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| UE network capability |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 10, bit 6 to bit 8 and octet 11,bit 1 to 2 | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 10, bit 7) | ‘1’B | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 10, bit 8) | 1 | Reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 11, bit 1) | 1 | Paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 11, bit 2) | 1 | Paging timing collision control supported | pc_EPC_MUSIM_PTCC |

Table 9.2.3.1.29.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.29.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | “TA only” |  |
| GUTI | GUTI-2 |  |  |
| EPS network feature support |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 5, bit 1 to bit 5 | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 5, bit 1) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 5, bit 2) | ‘1’B | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 5, bit 3) | ‘1’B | Reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 5, bit 4) | ‘1’B | Paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 5, bit 5) | ‘1’B | Paging timing collision control supported | pc_EPC_MUSIM_PTCC |

##### 9.2.3.1.30	Normal Tracking Area Update / Accepted / MUSIM / NAS signalling connection release

9.2.3.1.30.1	Test Purpose (TP)

(1)

with { UE in EMM-CONNECTED state and UE has transmitted an TRACKING AREA UPDATE REQUEST with Request type is "NAS signalling connection release" in the UE request type IE and the "active" flag is 0 in the EPS update type IE}

ensure that {

when { the UE receive TRACKING AREA UPDATE ACCEPT message with NAS signalling connection release bit is "NAS signalling connection release supported" in the EPS network feature support IE }

then { the UE enters the state EMM-REGISTERED }

}

9.2.3.1.30.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.3.1.2.1, 5.5.3.2.2, 6.4.3.4.

[TS 24.301, clause 5.3.1.2.1]

To allow the network to release the NAS signalling connection, the UE:

…

b)	shall start the timer T3440 if:

…

-	the tracking area updating or combined tracking area updating procedure has been initiated in EMM-IDLE mode, or the UE has set Request type to "NAS signalling connection release" in the UE request type IE in the TRACKING AREA UPDATE REQUEST message and the NAS signalling connection release bit is set to "NAS signalling connection release supported" in the EPS network feature support IE of the TRACKING AREA UPDATE ACCEPT message; and

…

Upon expiry of T3440,

-	in cases a, b, c, f, h i and j, the UE shall locally release the established NAS signalling connection; or

-	in cases d and e, the UE shall locally release the established NAS signalling connection and the UE shall initiate the attach procedure as described in clause 5.5.3.2.5, 5.5.3.3.5 or 5.6.1.5.

…

[TS 24.301, clause 5.5.3.2.2]

…

For all cases except case b, if the MUSIM UE supports the NAS signalling connection release, then the UE shall set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message otherwise the UE shall not set the NAS signalling connection release bit to "NAS signalling connection release supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

…

If the network supports the NAS signalling connection release and the MUSIM UE requests the network to release the NAS signalling connection, the UE shall set Request type to "NAS signalling connection release" in the UE request type IE and, if the network supports the paging restriction, may set the paging restriction preference in the Paging restriction IE in the TRACKING AREA UPDATE REQUEST message. In addition, the UE shall

-	set the "active" flag to 0 in the EPS update type IE; and

-	set the "signalling active" flag to 0 in the Additional update type IE, if the Additional update type IE is included.

NOTE 8:	If the network has already indicated support for NAS signalling connection release in the current stored tracking area list and the UE doesn't have a PDN connection for emergency bearer services established, the MUSIM UE is allowed to request the network to release the NAS signalling connection during tracking area updating procedure that is due to mobility to a tracking area outside the current tracking area list even before detecting whether the network supports the NAS signalling connection release in the new tracking area.

NOTE 9:	If the network has already indicated support for paging restriction in the current stored tracking area list and the UE doesn't have a PDN connection for emergency bearer services established, the MUSIM UE is allowed to include paging restriction together with the request to the network to release the NAS signalling connection during tracking area updating procedure that is due to mobility to a tracking area outside the current tracking area list even before detecting whether the network supports the paging restriction in the new tracking area.

…

9.2.3.1.30.3	Test description

9.2.3.1.30.3.1	Pre-test conditions

System Simulator:

-	Cell A (home PLMN1) and Cell G (home PLMN2) are configured according to table 6.3.2.2-1 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

-	Cell A (belongs to TAI-1, home PLMN1, MCC=001, MNC=01) is set to ''Serving cell''.

-	Cell G (belongs to TAI-7, home PLMN2, MCC=001, MNC=02) is set to '' Serving cell''.

UE:

-	The UE is a MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Table 4.5.2E.1-1 and Table 4.5.2E.1-2.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.5.2E.3 of TS 36.508 [18].

9.2.3.1.30.3.2	Test procedure sequence

Table 9.2.3.1.30.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-7 | Steps 2 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed successfully on Cell A to establish a new data radio bearer | - | - | - | - |
| 8 | Configure the UE to perform TRACKING AREA UPDATE REQUEST including NAS signalling connection release related IEs by AT or MMI command on Cell A(Note 1). | - | - | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE REQUEST message including "NAS signalling connection release" of Request type in the UE request type IE and the "active" flag is 0 in the EPS update type IE on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 10 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 11 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 12 | Ensure that the T3440 is expired. The SS transmits a Paging message to UE on Cell A. | - | - | - | - |
| 13 | Check: Does UE transmit an RRCConnectionRequest message on Cell A? | --> | RRCConnectionRequest | 1 | P |
| 14 | The SS transmits an RRCConnectionSetup message on Cell A. | <-- | RRCConnectionSetup | - | - |
| 15 | The UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST on Cell A. | --> | RRCConnectionSetupComplete SERVICE REQUEST | - | - |
| 16-19 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure on Cell A. | - | - | - | - |
| Note 1: The NAS signalling connection release request could be done by use of AT command, such as AT+CNASCREL or other commands. And the TAU can be triggered by UE locally release one of the user plane radio bearer. |  |  |  |  |  |

9.2.3.1.30.3.3	Specific message contents

Table 9.2.3.1.30.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 9, Table 9.2.3.1.30.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value (octet 1, bit 1 to 3) | ‘000’B | TA updating |  |
| "Active" flag (octet 1, bit 4) | ‘0’B | No bearer establishment requested |  |
| UE network capability |  |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| UE request type |  |  |  |
| Request type (bits 4 to 1 of octet 3) | ‘0001’B | The request type is NAS signalling connection release | pc_EPC_MUSIM_NCR |

Table 9.2.3.1.30.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 10, Table 9.2.3.1.30.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | ‘000’B | “TA only” |  |
| EPS network feature support(octet 3 to 4) | ‘0010 0011 0000 0000’B | IMS voice over PS session in S1 mode, emergency bearer services in S1 mode, EXTENDED SERVICE REQUEST for packet services |  |
| NAS signalling connection release (NCR) (octet 5, bit 1) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |

##### 9.2.3.1.31	Normal Tracking Area Update / Accepted / MUSIM / IMSI offset

9.2.3.1.31.1	Test Purpose (TP)

(1)

with { The UE has transmitted TRACKING AREA UPDATE REQUEST message with the Requested IMSI offset IE and EPS update type IE not indicating "periodic updating" }

ensure that {

when { The UE receives a TRACKING AREA UPDATE ACCEPT message with the Negotiated IMSI offset IE and a new GUTI }

then { The UE transmits the TRACKING AREA UPDATE COMPLETE message and receives a paging message in a valid new paging occasion which calculated based on the Negotiated IMSI offset }

}

(2)

with { The UE has transmitted TRACKING AREA UPDATE REQUEST message with the Requested IMSI offset IE and EPS update type IE not indicating "periodic updating" }

ensure that {

when { The UE receives a TRACKING AREA UPDATE ACCEPT message without the Negotiated IMSI offset IE }

then { The UE transmits the TRACKING AREA UPDATE COMPLETE message and receives a paging message in a valid old paging occasion which calculated without any IMSI offset }

}

9.2.3.1.31.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.3.2.4 and in TS 23.401, clause 4.3.33.5.

[TS 24.301, clause 5.5.3.2.4]

…

If the MUSIM UE has included a Requested IMSI offset IE in the TRACKING AREA UPDATE REQUEST message with the EPS update type IE not indicating "periodic updating" and if the MME supports paging timing collision control, the MME shall include the Negotiated IMSI offset IE and assign a new GUTI in the TRACKING AREA UPDATE ACCEPT message, and the MME shall set the IMSI offset value to:

-	A value that is different than what the UE has provided, if the MME has a different value; or

-	A value that is same as what the UE has provided, if the MME does not have a different value;

and the MME shall calculate an alternative IMSI value using the IMSI offset value and store it in the UE context as specified in 3GPP TS 23.401 [10]. The alternative IMSI value is used for deriving the paging occasion as specified in 3GPP TS 36.304 [21].

If the MUSIM UE has not included a Requested IMSI offset IE in the TRACKING AREA UPDATE REQUEST message with the EPS update type IE not indicating "periodic updating" and the MME has stored an alternative IMSI value for that UE, the MME shall erase the alternative IMSI value and assign a new GUTI in the TRACKING AREA UPDATE ACCEPT message.

…

If the ATTACH ACCEPT message contains Negotiated IMSI offset IE, the MUSIM UE shall forward the IMSI offset value to lower layers. If the ATTACH ACCEPT message does not contain Negotiated IMSI offset IE, the MUSIM UE shall indicate to lower layers to erase any IMSI offset value, if available.

…

[TS 23.401, clause 4.3.33.5]

To avoid possible paging occasion collision and to enhance the likelihood that paging is received successfully for different USIMs, a Multi-USIM UE may provide, for at least one USIM, a Requested IMSI Offset value that is used for the determination of paging occasions. Upon reception of a Requested IMSI Offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an Accepted IMSI Offset value to the UE in the Attach Accept or Tracking Area Update Accept message to acknowledge it supports the feature and provide the accepted value. The Accepted IMSI Offset Value may be different from the Requested IMSI Offset provided by the UE. The Alternative IMSI value, determined as below, is stored in the UE context in the MME. If the UE does not provide any Requested IMSI Offset value in Attach Request or Tracking Area Request, the MME removes any stored Alternative IMSI value in the UE context. The UE and the network use the Accepted IMSI Offset to determine the paging occasion. The UE and MME use the Accepted IMSI Offset value to calculate the Alternative IMSI value that is determined based on UE's IMSI as follows:

Alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)]

where: the MCC, MNC and MSIN value are the fields of the UE's IMSI as defined in TS 23.003 [9].

The MME uses the Alternative IMSI value to compute the UE Identity Index Value. The MME sends the UE Identity Index Value to RAN in the Paging message (see TS 36.413 [36]) for RAN to derive the paging occasions according to TS 36.304 [34].

The UE uses the Alternative IMSI value for the determination of paging occasions as specified in TS 36.304 [34].

NOTE 1:	It is recommended to avoid excessive signalling load from UE due to this procedure.

NOTE 2:	The MME does not remove Alternative IMSI value if the Tracking Area Update Request is for periodic Tracking Area Update.

9.2.3.1.31.3	Test description

9.2.3.1.31.3.1	Pre-test conditions

System Simulator:

-	Cell A/Cell B(home PLMN1) and Cell G/Cell H(home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

-	cell A (belongs to TAI-1, home PLMN1, MCC=001, MNC=01) is set to ''Serving cell'' for USIM A.

-	cell B (belongs to TAI-2, home PLMN1, MCC=001, MNC=01) is set to '' Non-Suitable cell'' for USIM A.

-	cell G (belongs to TAI-7, home PLMN2, MCC=001, MNC=02) is set to '' Serving cell'' for USIM B.

-	cell H (belongs to TAI-8, home PLMN2, MCC=001, MNC=02) is set to '' Non-Suitable cell '' for USIM B.

UE:

-	The UE is a MUSIM UE.

-	Each USIM of the UE is configured to initiate EPS attach.

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to TS 36.508 [18] Table 4.5.2E.3-1.

9.2.3.1.31.3.2	Test procedure sequence

Table 9.2.3.1.31.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 1 | Configure the UE to perform TRACKING AREA UPDATE REQUEST including a Requested IMSI offset IE(Note1). | - | - | - | - |
| 2 | The SS configures: - Cell A as the "Non-Suitable Off cell ". - Cell B as a "Serving cell ". | - | - | - | - |
| 3 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message including a Requested IMSI offset IE? (Note1) | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with the Negotiated IMSI offset IE. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 6 | The SS releases the RRC connection. | - | - | - | - |
| 7 | The SS transmits a Paging message in a PO which calculated based on the Negotiated IMSI offset. | - | - | - | - |
| 8-17 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 1 | P |
| - | The following messages are sent and shall be received on cell H. | - | - | - | - |
| 18 | Configure the UE to perform TRACKING AREA UPDATE REQUEST including a Requested IMSI offset IE(Note1). | - | - | - | - |
| 19 | The SS configures: - Cell G as the "Non-Suitable Off cell ". - Cell H as a "Serving cell". | - | - | - | - |
| 20 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message including a Requested IMSI offset IE? (Note1) | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without the Negotiated IMSI offset IE. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 23 | The SS releases the RRC connection. | - | - | - | - |
| 24 | The SS transmits a Paging message in a PO which calculated without the Negotiated IMSI offset. | - | - | - | - |
| 25-34 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 2 | P |
| Note 1: The TRACKING AREA UPDATE REQUEST message including a Requested IMSI offset IE may be performed by MMI or AT command. |  |  |  |  |  |

9.2.3.1.31.3.3	Specific message contents

Table 9.2.3.1.31.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 3 and 20, Table 9.2.3.1.31.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested IMSI offset | Any allowed value |  |  |

Table 9.2.3.1.31.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 4, Table 9.2.3.1.31.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated IMSI offset IE | Same as what the Requested IMSI offset IE |  |  |

#### 9.2.3.2	Combined tracking area updating

##### 9.2.3.2.1	Combined tracking area update / Successful

9.2.3.2.1.1	Test Purpose (TP)

(1)

with { a combined EPS/IMSI attached UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE enters a tracking area included in the TAI list }

then { UE does not transmit a TRACKING AREA UPDATE REQUEST message }

}

(2)

with { a combined EPS/IMSI attached UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { UE enters a tracking area not included in the TAI list }

then { UE transmits a TRACKING AREA UPDATE REQUEST message with "EPS update type = combined TA/LA updating" }

}

(3)

with { UE has sent a combined TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE receives a TRACKING AREA UPDATE ACCEPT message containing a GUTI and/or a mobile identity }

then { UE transmits a TRACKING AREA UPDATE COMPLETE message and enters EMM-REGISTERED state }

}

9.2.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.3.1, 5.5.3.2.2, 5.5.3.2.4, 5.5.3.3.1, 5.5.3.3.2, 5.5.3.3.4.1 and 5.5.3.3.4.2.

[TS 24.301 clause 5.5.3.1]

The tracking area updating procedure is always initiated by the UE and is used for the following purposes:

...-	combined tracking area updating to update the registration of the actual tracking area for a UE in CS/PS mode 1 or CS/PS mode 2 of operation;

...

[TS 24.301 clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

After sending the TRACKING AREA UPDATE REQUEST message to the MME, the UE shall start timer T3430 and enter state EMM-TRACKING-AREA-UPDATING-INITIATED (see example in figure 5.5.3.2.2). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411. If timer T3442 is currently running, the UE shall stop timer T3442.

...

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

...

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

...

If a UE has uplink user data pending when it initiates the tracking area updating procedure, or uplink signalling not related to the tracking area updating procedure, it may also set an "active" flag in the TRACKING AREA UPDATE REQUEST message to indicate the request to establish the user plane to the network and to keep the NAS signalling connection after the completion of the tracking area updating procedure.

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

...

When the tracking area updating procedure is initiated in EMM-IDLE mode, the UE may also include an EPS bearer context status IE in the TRACKING AREA UPDATE REQUEST message, indicating which EPS bearer contexts are active in the UE.

...

[TS 24.301 clause 5.5.3.2.4]

...

Upon receiving a TRACKING AREA UPDATE ACCEPT message, the UE shall stop timer T3430, reset the routing area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED. If the message contains a GUTI, the UE shall use this GUTI as new temporary identity for EPS services and shall store the new GUTI. If no GUTI was included by the MME in the TRACKING AREA UPDATE ACCEPT message, the old GUTI shall be used. If the UE receives a new TAI list in the TRACKING AREA UPDATE ACCEPT message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

...

If the TRACKING AREA UPDATE ACCEPT message contained a GUTI, the UE shall return a TRACKING AREA UPDATE COMPLETE message to the MME to acknowledge the received GUTI.

...

[TS 24.301 clause 5.5.3.3.1]

Within a combined tracking area updating procedure the messages TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE COMPLETE carry information for the tracking area updating and the location area updating.

The combined attach procedure basically follows the normal tracking area updating procedure described in subclause 5.5.3.2.

[TS 24.301 clause 5.5.3.3.2]

...

To initiate a combined tracking area updating procedure the UE sends the message TRACKING AREA UPDATE REQUEST to the network, starts timer T3430 and changes to state EMM-TRACKING-AREA-UPDATING-INITIATED. The value of the EPS update type IE in the message shall indicate "combined TA/LA updating" unless explicitly specified otherwise.

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause 5.5.3.3.4.1]

Depending on the value of the EPS update result IE received in the TRACKING AREA UPDATE ACCEPT message, two different cases can be distinguished:

1)	The EPS update result IE value indicates "combined TA/LA updated": Tracking and location area updating is successful;

...

A TRACKING AREA UPDATE COMPLETE message shall be returned to the network if the TRACKING AREA UPDATE ACCEPT message contains a GUTI and/or a mobile identity.

[TS 24.301 clause 5.5.3.3.4.2]

The description for normal tracking area update as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location area updating applies.

The TMSI reallocation may be part of the combined tracking area updating procedure. The TMSI allocated is then included in the TRACKING AREA UPDATE ACCEPT message together with the location area identification (LAI). In this case the MME shall change to state EMM-COMMON-PROCEDURE-INITIATED and shall start the timer T3450 as described in subclause 5.4.1. The LAI may be included in the TRACKING AREA UPDATE ACCEPT message without TMSI.

The UE, receiving a TRACKING AREA UPDATE ACCEPT message, stores the received location area identification, resets the location update attempt counter, sets the update status to U1 UPDATED and enters MM state MM IDLE.

...

How to handle the old TMSI stored in the UE depends on the mobile identity included in the TRACKING AREA UPDATE ACCEPT message.

-	If the TRACKING AREA UPDATE ACCEPT message contains an IMSI, the UE is not allocated any TMSI, and shall delete any old TMSI accordingly.

-	If the TRACKING AREA UPDATE ACCEPT message contains a TMSI, the UE shall use this TMSI as new temporary identity. The UE shall delete its old TMSI and shall store the new TMSI. In this case, a TRACKING AREA UPDATE COMPLETE message is returned to the network to confirm the received TMSI.

-	If neither a TMSI nor an IMSI has been included by the network in the TRACKING AREA UPDATE ACCEPT message, the old TMSI, if any is available, shall be kept.

...

9.2.3.2.1.3	Test description

9.2.3.2.1.3.1	Pre-test conditions

System Simulator:

-	cell A, cell C and cell D (HPLMN, different TAs);

-	 at most 2 cells are active simultaneously.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.1.3.2	Test procedure sequence

Table 9.2.3.2.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell C. | - | - | - | - |
| 1-13 | Void | - | - | - | - |
| 14 | Set the cell type of cell A to the "non-Suitable neighbour cell ". Set the cell type of cell C to the " Serving cell" | - | - | - | - |
| 15 | Check: Does the UE transmit a combined TRACKING AREA UPDATE REQUEST message as specified? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 16 | The SS sends TRACKING AREA UPDATE ACCEPT message including GUTI, TMSI and LAI. The TAI list includes TAI for cell C and cell D. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 2,3 | P |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| 19 | Check: Does the test results of generic procedure in TS 36.508 [18] subclause 6.4.2.4 indicate that the UE is in E-UTRA EMM-REGISTERED state on cell C with PagingUE-Identity = S-TMSI2? | - | - | 2 | - |
|  | The following messages are sent and shall be received on cell D. | - | - | - | - |
| 20 | Set the cell type of cell A to the "non-Suitable off cell ". Set the cell type of cell C to the "Suitable neighbour cell". Set the cell type of cell D to the " Serving cell" | - | - | - | P |
| 20A | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message in the next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 20B | Check: Does the test results of generic procedure in TS 36.508 [18] subclause 6.4.2.4 indicate that the UE is in E-UTRA EMM-REGISTERED state on cell D with PagingUE-Identity = S-TMSI2? | - | - | 1 |  |
| 20C | Set the cell type of cell A to the "Serving cell". Set the cell type of cell C to the "Non-suitable off cell". Set the cell type of cell D to the "Suitable neighbour cell". |  |  |  |  |
|  | The following messages are sent and shall be received on cell A. |  |  |  |  |
| 21 | Check: Does the UE transmit a combined TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 22 | The SS sends TRACKING AREA UPDATE ACCEPT message. Note: GUTI not present and MS identity present | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 23 | Check: Does the UE send TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 2,3 | P |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

NOTE 1:	It is assumed in the test procedure sequence that the UE initially has a valid GUTI.

NOTE 2:	NAS security procedures are not checked in this TC.

9.2.3.2.1.3.3	Specific message contents

Table 9.2.3.2.1.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 15, Table 9.2.3.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier | KSI sent during the test case preamble in step 7. |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Old GUTI | GUTI-1 | GUTI-1 was allocated in cell A during the preamble |  |
| Last visited registered TAI | TAI-1 | TAI of cell A |  |
| Old location area identification | LAI-1 | LAI received in the ATTACH ACCEPT message in the preamble |  |
| TMSI status | Not Present | Valid TMSI available |  |

Table 9.2.3.2.1.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 16, Table 9.2.3.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00001000'B | 8 octets |  |
| Type of list | '00’B | ''list of TACs belonging to one PLMN, with non-consecutive TAC values'' |  |
| Number of elements | '00001' B | 2 elements |  |
| Partial tracking area identity list | PLMN = MCC/MNC stored in EFIMSI TAC 1 = 3 TAC 2 = 4 | TAI-3 and TAI-4 |  |

Table 9.2.3.2.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 21, Table 9.2.3.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-2 |  |  |
| Last visited registered TAI | TAI-4 |  |  |
| Old location area identification | LAI-2 |  |  |
| TMSI status | Not present | Valid TMSI available |  |

Table 9.2.3.2.1.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 22, Table 9.2.3.2.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present | The SS doesn’t assign a new GUTI |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | ‘00000110’B | 6 octets |  |
| Type of list | ‘00’B | One PLMN with non-consecutive TACs |  |
| Number of elements | ‘00000’B | 1 element |  |
| Partial tracking area identity list | PLMN = MCC/MNC stored in EFIMSI TAC 1 = TAC-1 | TAI-1 |  |
| LAI | LAI-1 |  |  |
| MS identity | TMSI-3 | SS provides a new TMSI |  |

##### 9.2.3.2.1a	Combined tracking area update / Successful / Check of last visited TAI and handling of TAI list, LAI and TMSI

9.2.3.2.1a.1	Test Purpose (TP)

(1)

with { a combined EPS/IMSI attached UE has sent a combined TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating', including a last visited registered TAI }

ensure that {

when { the UE receives a TRACKING AREA UPDATE ACCEPT message containing a new TAI list, a location area information(LAI) and a TMSI as the mobile identity }

then { the UE shall delete its old TMSI and shall store the new TMSI and transmits a TRACKING AREA UPDATE COMPLETE message and enters EMM-REGISTERED state }

}

(2)

Void

(3)

with { UE in state GMM-REGISTERED mode }

ensure that {

when { the UE receives a Paging message including an ue-Identity set to an unmatched TMSI i.e. other than the one allocated to the UE at the UE registration procedure }

then { the UE doesn’t establish an RRC connection to answer the paging }

}

(4)

with { UE in state GMM-REGISTERED mode }

ensure that {

when { the UE receives a Paging message including an ue-Identity set to the TMSI which was allocated to the UE }

then { the UE establishes an RRC connection to answer the paging and the TMSI and the location area information(LAI) as Initial UE Identity are included in RRCConnectionRequest message }

}

(5)

with { a combined EPS/IMSI attached UE has sent a combined TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach ', including a last visited registered TAI }

ensure that {

when { the UE receives a TRACKING AREA UPDATE ACCEPT message containing a new TAI list, a location area information(LAI) (Neither a TMSI nor an IMSI as the mobile identity is included)  }

then { the UE shall keep the old TMSI if any available and enters EMM-REGISTERED state }

}

(6)

Void

(7)

Void

(8)

with { a combined EPS/IMSI attached UE has sent a combined TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach ', including a last visited registered TAI }

ensure that {

when { the UE receives a TRACKING AREA UPDATE ACCEPT message containing a new TAI list, a location area information(LAI) and an IMSI as the mobile identity }

then { the UE shall delete any old TMSI and enters EMM-REGISTERED state }

}

(9)

Void

(10)

with { UE in state GMM-REGISTERED mode }

ensure that {

when { the UE receives a Paging message including an ue-Identity set to a TMSI }

then { the UE doesn’t establish an RRC connection to answer the paging }

}

9.2.3.2.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.3.3.1, 5.5.3.3.2 , 5.5.3.3.4.1 and 5.5.3.3.4.2; TS 24.008, clause 4.4.1.

[TS 24.301 clause 5.5.3.3.1]

Within a combined tracking area updating procedure the messages TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE COMPLETE carry information for the tracking area updating and the location area updating.

The combined tracking area updating procedure follows the normal tracking area updating procedure described in subclause 5.5.3.2.

[TS 24.301 clause 5.5.3.3.2]

The UE operating in CS/PS mode 1 or CS/PS mode 2, in state EMM-REGISTERED, shall initiate the combined tracking area updating procedure:

a)	when the UE that is attached for both EPS and non-EPS services detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

b)	when the UE that is attached for EPS services wants to perform an attach for non-EPS services. In this case the EPS update type IE shall be set to "Combined TA/LA updating with IMSI attach";

c)	when the UE performs an intersystem change from A/Gb mode to S1 mode and the EPS services were previously suspended in A/Gb mode;

d)	when the UE performs an intersystem change from A/Gb or Iu mode to S1 mode and the UE previously performed a location area update procedure in A/Gb or Iu mode, in order to re-establish the SGs association;

e)	when the UE enters EMM-REGISTERED.NORMAL-SERVICE and the UE's TIN indicates "P-TMSI";

f)	when the UE receives an indication from the lower layers that the RRC connection was released with cause "load balancing TAU required";

g)	when the UE deactivated EPS bearer context(s) locally while in EMM-REGISTERED.NO-CELL-AVAILABLE, and then returns to EMM-REGISTERED.NORMAL-SERVICE;

h)	when the UE changes the UE core network capability information or the UE specific DRX parameter or both;

i)	when the UE receives an indication of "RRC Connection failure" from the lower layers and has no user uplink data pending; or

j)	when the UE has selected a CSG cell whose CSG identity is not included in the UE's Allowed CSG list.

To initiate a combined tracking area updating procedure the UE sends the message TRACKING AREA UPDATE REQUEST to the network, starts timer T3430 and changes to state EMM-TRACKING-AREA-UPDATING-INITIATED. The value of the EPS update type IE in the message shall indicate "combined TA/LA updating" unless explicitly specified otherwise.

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause 5.5.3.3.4.1]

Depending on the value of the EPS update result IE received in the TRACKING AREA UPDATE ACCEPT message, two different cases can be distinguished:

1)	The EPS update result IE value indicates "combined TA/LA updated": Tracking and location area updating is successful;

2)	The EPS update result IE value indicates "TA updated": Tracking area updating is successful, but location area updating is not successful.

A TRACKING AREA UPDATE COMPLETE message shall be returned to the network if the TRACKING AREA UPDATE ACCEPT message contains a GUTI or a mobile identity or both.

[TS 24.301 clause 5.5.3.3.4.2]

The description for normal tracking area update as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location area updating applies.

The TMSI reallocation may be part of the combined tracking area updating procedure. The TMSI allocated is then included in the TRACKING AREA UPDATE ACCEPT message together with the location area identification (LAI). In this case the MME shall change to state EMM-COMMON-PROCEDURE-INITIATED and shall start the timer T3450 as described in subclause 5.4.1. The LAI may be included in the TRACKING AREA UPDATE ACCEPT message without TMSI.

The UE, receiving a TRACKING AREA UPDATE ACCEPT message, stores the received location area identification, resets the location update attempt counter, sets the update status to U1 UPDATED and enters MM state MM IDLE.

How to handle the old TMSI stored in the UE depends on the mobile identity included in the TRACKING AREA UPDATE ACCEPT message.

-	If the TRACKING AREA UPDATE ACCEPT message contains an IMSI, the UE is not allocated any TMSI, and shall delete any old TMSI accordingly.

-	If the TRACKING AREA UPDATE ACCEPT message contains a TMSI, the UE shall use this TMSI as new temporary identity. The UE shall delete its old TMSI and shall store the new TMSI. In this case, a TRACKING AREA UPDATE COMPLETE message is returned to the network to confirm the received TMSI.

-	If neither a TMSI nor an IMSI has been included by the network in the TRACKING AREA UPDATE ACCEPT message, the old TMSI, if any is available, shall be kept.

The network receiving a TRACKING AREA UPDATE COMPLETE message stops timer T3450, changes to state EMM-REGISTERED and considers the new TMSI as valid.

[TS 24.008 clause 4.4.1]

The location updating procedure is a general procedure which is used for the following purposes:

.....

-	indicating to the network that the MS, configured to use CS fallback and SMS over SGs, or SMS over SGs only, has entered a GERAN or UTRAN cell in NMO II or III, after intersystem change from S1 mode to Iu or A/Gb mode, the TIN indicates "GUTI" and the location area of the current cell is the same as the stored location area; or

NOTE 1:	The location updating procedure can be delayed when the intersystem change is due to CS fallback. In this case, the MS has to remember that it has to perform a location updating procedure after the RR connection is released, if the MS is still in GERAN/UTRAN.

.....

The normal location updating procedure shall also be started if the MS is configured to use CS fallback and SMS over SGs, or SMS over SGs only, and the TIN indicates "RAT-related TMSI",

-	when the periodic tracking area update timer T3412 expires and the network operates in network operation mode II or III; or

-	when the MS enters a GERAN or UTRAN cell in network operation mode II or III and the E-UTRAN deactivate ISR timer T3423 is running.

NOTE 2:	The timers T3412 and T3423 are specified in 3GPP TS 24.301 [120].

The normal location updating procedure shall also be started when the MS, configured to use CS fallback and SMS over SGs, or SMS over SGs only, enters a GERAN or UTRAN cell in network operation mode II or III and the E-UTRAN deactivate ISR timer T3423 has expired.

If the MS, configured to use CS fallback and SMS over SGs, enters a GERAN or UTRAN cell in network operation mode II or III, after intersystem change from S1 mode to Iu or A/Gb mode due to CS fallback, and the location area of the current cell is not available, the MS should initiate the location updating procedure.

.....

In the case that the mobile station is initiating an emergency call but, due to cell re-selection or redirection by the network, it moves to a different LAI then the mobile station may delay the location updating procedure in the new LA until after the emergency call is completed.

9.2.3.2.1a.3	Test description

9.2.3.2.1a.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1) is set to ''Serving cell'';

-	cell B (belongs to TAI-2) is set to ''Non-suitable cell'';

-	System information combination 9 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

-	cell 5 (UTRA FDD or UTRA TDD, belongs to LAI-3) is set to ''Non-suitable “off” cell''.

-	cell 7 (UTRA FDD or UTRA TDD, as exception contains LAI-4) is set to ''Non-suitable cell''.

-	UTRAN is NOT using Gs interface. (NMO in System Information Block 1 indicates NMO II).

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) with condition CombinedAttach on Cell A according to [18].

9.2.3.2.1a.3.2	Test procedure sequence

Table 9.2.3.2.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 1 | Set cell type of Cell A to the "non-Suitable cell" Set cell type of Cell B to the "Serving cell". | - | - | - | - |
| 2 | Check: Does the UE transmit a combined TRACKING AREA UPDATE REQUEST message with Last visited registered TAI set to TAI-1? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS sends TRACKING AREA UPDATE ACCEPT message including with LAI set to LAI-2, including a new TMSI (TMSI-2) as MS identity. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | Check: Does the UE send TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell 7. | - | - | - | - |
| 6 | Set cell type of Cell B to the "non-Suitable cell" Set cell type of Cell 7 to the "Serving cell". | - | - | - | - |
| 7 | The UE performs Cell Reselection from Cell B (E-UTRAN cell) to Cell 7 (UTRAN cell). | - | - | - | - |
| 8 | Void | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.3.2.1a.3.2-2 occurs in parallel with steps 9-9B. | - | - | - | - |
| 9 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 9A | The SS transmits a ROUTING AREA UPDATE ACCEPT message | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 9B | The UE transmits a ROUTING AREA UPDATE COMPLETE message | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 10 | The SS send Paging message including an unmatched identity. | <-- | Paging | - | - |
| 11 | Check: Does the UE transmit RRCConnectionRequest to answer the Paging? | --> | RRCConnectionRequest | 3 | F |
| 12 | The SS send Paging message including a matched identity (UE Identity = TMSI-2). | <-- | Paging | - | - |
| 13 | Check: Does the test results of Mobile terminated establishment of Radio Resource Connection [5] indicate that the UE transmit RRCConnectionRequest with Initial UE identity set to TMSI-2 and LAI-2 to answers on Paging message? | - | - | 4 | P |
| 13a | The UE sends a PAGING RESPONSE message | --> | PAGING RESPONSE | - | - |
| 14 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell A. | - | - | - | - |
| 15 | Set cell type of Cell 7 to the "non-Suitable cell" Set cell type of Cell A to the "Serving cell". | - | - | - | - |
| 16 | The UE performs Cell Reselection from Cell 7 (UTRAN cell) to Cell A (E-UTRAN cell). | - | - | - | - |
| 17 | Check: Does the UE transmit a combined TRACKING AREA UPDATE REQUEST message with Last visited registered TAI set to TAI-2? | --> | TRACKING AREA UPDATE REQUEST | 5 | P |
| 18 | The SS sends TRACKING AREA UPDATE ACCEPT message with LAI set to LAI-1, not including MS identity. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 19 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell 5. | - | - | - | - |
| 20 | Set cell type of Cell A to the "non-Suitable cell. Set cell type of Cell 7 to the "non-Suitable “off” cell". " Set cell type of Cell 5 to the "Serving cell". | - | - | - | - |
| 21 | The UE performs Cell Reselection from Cell A (E-UTRAN cell) to Cell 5(UTRAN cell). | - | - | - | - |
| 22 | Void | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.3.2.1a.3.2-3 occurs in parallel with steps 23-23B. | - | - | - | - |
| 23 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 23A | The SS transmits a ROUTING AREA UPDATE ACCEPT message | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 23B | The UE transmits a ROUTING AREA UPDATE COMPLETE message | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 24 | The SS send Paging message including a matched identity (UE Identity = TMSI-2). | <-- | Paging | - | - |
| 25 | Check: Does the test results of Mobile terminated establishment of Radio Resource Connection [5] indicate that the UE transmit RRCConnectionRequest with Initial UE identity set to TMSI-2 and LAI-1 to answers on Paging message? | - | - | 4 | P |
| 25a | The UE sends a PAGING RESPONSE message | --> | PAGING RESPONSE | - | - |
| 26 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 27 | Set cell type of Cell 5 to the "non-Suitable cell" Set cell type of Cell B to the "Serving cell". | - | - | - | - |
| 28 | The UE performs Cell Reselection from Cell 5(UTRAN cell) to Cell B (E-UTRAN cell). | - | - | - | - |
| 29 | Check: Does the UE transmit a combined TRACKING AREA UPDATE REQUEST message with Last visited registered TAI set to TAI-1? | --> | TRACKING AREA UPDATE REQUEST | 8 | P |
| 30 | The SS sends TRACKING AREA UPDATE ACCEPT message with LAI set to LAI-2, including the IMSI as MS identity. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | EXCEPTION: Step 30a is optional for Release 8 to Release 10 UE implementation, from release 11 onwards, UE shall not send this message. | - | - | - | - |
| 30a | The UE sends a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 31 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Cell 7. | - | - | - | - |
| 32 | Set cell type of Cell B to the "non-Suitable cell". Set cell type of Cell A to the "non-Suitable “off” cell". Set cell type of Cell 7 to the "Serving cell". | - | - | - | - |
| 33 | The UE performs Cell Reselection from Cell B (E-UTRAN cell) to Cell 7 (UTRAN cell). | - | - | - | - |
| 34 | Void | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.3.2.1a.3.2-3 occurs in parallel with steps 35-35B. | - | - | - | - |
| 35 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 35A | The SS transmits a ROUTING AREA UPDATE ACCEPT message | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 35B | The UE transmits a ROUTING AREA UPDATE COMPLETE message | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 36 | The SS sends Paging message including a TMSI. | <-- | Paging | - | - |
| 37 | Check: Does the UE transmit RRCConnectionRequest to answer the Paging? | --> | RRCConnectionRequest | 10 | F |

Table: 9.2.3.2.1a.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE perform Location updating procedure? | --> | LOCATION UPDATING REQUEST | - | - |
| 2 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 3 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 4 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |

Table: 9.2.3.2.1a.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE perform Location updating procedure? | --> | LOCATION UPDATING REQUEST | - | - |
| 2 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |

9.2.3.2.1a.3.3	Specific message contents

Table 9.2.3.2.1a.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 | Old GUTI is included by UE if valid, IMSI otherwise. |  |
| Last visited registered TAI | TAI-1 | If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| Old LAI | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.2.1a.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value | Not present |  |  |
| GUTI | Not present | This IE may be included to assign a new GUTI |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | 00000110 | 6 octets |  |
| Number of elements | 00000 | 1 element |  |
| Type of list | 00 | One PLMN with non-consecutive TACs |  |
| MCC MNC TAC 1 | TAI-2 |  |  |
| LAC | 2 (LAI-2) |  |  |
| MS identity | TMSI-2 |  |  |
| T3402 value | Not present |  |  |
| Equivalent PLMNs | Not present |  |  |
| EMM cause | Not present |  |  |

Table 9.2.3.2.1a.3.3-3: Message PAGING TYPE 1 (step 10, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 7.1.2.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging record list |  |  |  |
| Paging record |  |  |  |
| CN originator |  |  |  |
| Paging cause CN domain identity UE identity | Terminating Speech Call CS domain TMSI other than TMSI-2 |  |  |

Table 9.2.3.2.1a.3.3-4: Message PAGING TYPE 1 (step 12, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 7.1.2.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging record list |  |  |  |
| Paging record |  |  |  |
| CN originator |  |  |  |
| Paging cause CN domain identity UE identity | Terminating Speech Call CS domain TMSI-2 |  |  |

Table 9.2.3.2.1a.3.3-5: Message RRC CONNECTION REQUEST (step 13, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Initial UE identity |  |  |  |
| TMSI(GSM-MAP) LAI (GSM-MAP) | TMSI-2 LAI-4 |  |  |
| Establishment cause | Terminating Conversational Call |  |  |

Table 9.2.3.2.1a.3.3-6 Message TRACKING AREA UPDATE REQUEST (step 17, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 | Old GUTI is included by UE calculated by using UTRAN P-TMSI. |  |
| Last visited registered TAI | TAI-2 | If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| Old LAI | LAI-4 |  |  |
| TMSI status | Not present |  |  |
| EPS update type | 010 | combined TA/LA updating with IMSI attach |  |
| NonceUE | Any allowed value |  |  |
| Old P-TMSI signature | Present |  |  |
| Additional GUTI | GUTI-1 |  |  |

Table 9.2.3.2.1a.3.3-7: Message TRACKING AREA UPDATE ACCEPT (step 18, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value | Not present |  |  |
| GUTI | Not present | This IE may be included to assign a new GUTI |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | 00000110 | 6 octets |  |
| Number of elements | 00000 | 1 element |  |
| Type of list | 00 | One PLMN with non-consecutive TACs |  |
| MCC MNC TAC 1 | TAI-1 |  |  |
| LAC | 1 (LAI-1) |  |  |
| MS identity | Not present |  |  |
| T3402 value | Not present |  |  |
| Equivalent PLMNs | Not present |  |  |
| EMM cause | Not present |  |  |

Table 9.2.3.2.1a.3.3-8: Message PAGING TYPE 1 (step 24, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 7.1.2.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging record list |  |  |  |
| Paging record |  |  |  |
| CN originator |  |  |  |
| Paging cause CN domain identity UE identity | Terminating Speech Call CS domain TMSI-2 |  |  |

Table 9.2.3.2.1a.3.3-9: Message RRC CONNECTION REQUEST (step 25, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Initial UE identity |  |  |  |
| TMSI(GSM-MAP) LAI (GSM-MAP) | TMSI-2 LAI-3 |  |  |
| Establishment cause | Terminating Conversational Call |  |  |

Table 9.2.3.2.1a.3.3-10: Message TRACKING AREA UPDATE REQUEST (step 29, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 | Old GUTI is included by UE calculated by using UTRAN P-TMSI. |  |
| Last visited registered TAI | TAI-1 | If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| Old LAI | LAI-3 |  |  |
| TMSI status | Not present |  |  |
| EPS update type | 010 | combined TA/LA updating with IMSI attach |  |
| NonceUE | Any allowed value |  |  |
| Old P-TMSI signature | Present |  |  |
| Additional GUTI | GUTI-1 |  |  |

Table 9.2.3.2.1a.3.3-11: Message TRACKING AREA UPDATE ACCEPT (step 30, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 001 | "combined TA/LA" |  |
| T3412 value | Not present |  |  |
| GUTI | Not present | This IE may be included to assign a new GUTI |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | 00000110 | 6 octets |  |
| Number of elements | 00000 | 1 element |  |
| Type of list | 00 | One PLMN with non-consecutive TACs |  |
| MCC MNC TAC 1 | TAI-2 |  |  |
| LAC | 2(LAI-2) |  |  |
| MS identity | IMSI |  |  |
| T3402 value | Not present |  |  |
| Equivalent PLMNs | Not present |  |  |
| EMM cause | Not present |  |  |

Table 9.2.3.2.1a.3.3-12: Message PAGING TYPE 1 (step 36, Table 9.2.3.2.1a.3.2-1)

| Derivation path: 34.108 sec 7.1.2.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging record list |  |  |  |
| Paging record |  |  |  |
| CN originator |  |  |  |
| Paging cause CN domain identity UE identity | Terminating Speech Call CS domain TMSI (any value) |  |  |

Table 9.2.3.2.1a.3.3-13: LOCATION UPDATING REQUEST (step 1, Table 9.2.3.2.1a.3.2-2 and step 1, Table 9.2.3.2.1a.3.2-3)

| Derivation Path: TS 36.508 Table 4.7B.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Location updating type | Normal location updating |  |  |

Table 9.2.3.2.1a.3.3-14: LOCATION UPDATING ACCEPT (step 4, Table 9.2.3.2.1a.3.2-2 and step 2, Table 9.2.3.2.1a.3.2-3)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 9.2.3.2.1b	Combined tracking area update / Success / SMS only

9.2.3.2.1b.1	Test Purpose (TP)

(1)

with { UE, operating in CS/PS mode 1, has sent a combined TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE receives a TRACKING AREA UPDATE ACCEPT message containing a GUTI and/or a mobile identity and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported }

then { UE automatically disable the E-UTRAN capability and performs a registration to UTRAN cell or GERAN cell(depending on the UE capability)}

}

(2)

with { UE, operating in CS/PS mode 2, in state EMM-REGISTERED and state MM-IDLE}

ensure that {

when { UE receives TRACKING AREA UPDATE ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and SS sends Paging message with PS domain }

then { UE sends SERVICE REQUEST message }

}

(3)

with { UE operating in CS/PS mode 2 and configured to use IMS voice, in state EMM-REGISTERED and state MM-IDLE}

ensure that {

when { UE receives TRACKING AREA UPDATE ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and the user initiates an IMS voice call}

then { UE does not initiate an IMS voice call}

(4)

with { UE operating in CS/PS mode 1}

ensure that {

when { UE receives TRACKING AREA UPDATE ACCEPT message with EPS updateresult 'combined EPS/IMSI attach' and Additional update result IE 'SMS only' and indicating that IMS voice over PS sessions is not supported }

then { UE does not set the E-UTRA support bit in the relevant NAS and AS messages}

9.2.3.2.1b.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 4.3.1, 4.5, 4.3.1, 4.5, 5.5.3.3.1, 5.5.3.3.2, 5.5.3.3.4.1, 5.5.3.3.4.2 and 5.6.2.4.

[TS 24.301 clause 4.3.1]

The behaviour of the UE in CS/PS mode 1 of operation, upon failure to access the CS domain or upon reception of a "CS fallback not preferred" or "SMS only" indication, will depend on the availability of voice over IMS. In the present document, "IMS voice not available" refers to one of the following conditions:

-	the UE is not configured to use IMS;

-	the UE is not configured to use IMS voice, i.e. when the voice domain preference for E-UTRAN, as defined in 3GPP TS 24.167 [13B], indicates that voice communication services are allowed to be invoked only over the CS domain;

-	the UE is configured to use IMS voice, but the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are not supported; or

-	the UE is configured to use IMS voice, the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are supported, but the upper layers indicate that the UE is not available for voice calls in the IMS.

[TS 24.301 clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN; and

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability.

NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

The UE shall enable the E-UTRA capability again in the following cases:

- 	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation;

-	the UE powers off and powers on again; or

- 	for the PLMN selection purpose.

[TS 24.301 clause 5.5.3.3.1]

Within a combined tracking area updating procedure the messages TRACKING AREA UPDATE ACCEPT and TRACKING AREA UPDATE COMPLETE carry information for the tracking area updating and the location area updating.

The combined tracking area updating procedure follows the normal tracking area updating procedure described in subclause 5.5.3.2.

[TS 24.301 clause 5.5.3.3.2]

...

To initiate a combined tracking area updating procedure the UE sends the message TRACKING AREA UPDATE REQUEST to the network, starts timer T3430 and changes to state EMM-TRACKING-AREA-UPDATING-INITIATED. The value of the EPS update type IE in the message shall indicate "combined TA/LA updating" unless explicitly specified otherwise.

If the UE initiates the combined tracking area updating procedure for EPS services and "SMS only", the UE shall indicate "SMS only" in the Additional update Type IE.

The UE shall include the TMSI status IE if no valid TMSI is available. Furthermore, if the UE has stored a valid location area identification, the UE shall include it in the Old location area identification IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause 5.5.3.3.4.1]

Depending on the value of the EPS update result IE received in the TRACKING AREA UPDATE ACCEPT message, the following different cases can be distinguished:

1)	The EPS update result IE value indicates "combined TA/LA updated": Tracking and location area updating is successful for EPS and non-EPS services, or for EPS services and "SMS only";

...

A TRACKING AREA UPDATE COMPLETE message shall be returned to the network if the TRACKING AREA UPDATE ACCEPT message contains a GUTI or a mobile identity or both.

[TS 24.301 clause 5.5.3.3.4.2]

The description for normal tracking area update as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location area updating applies.

The TMSI reallocation may be part of the combined tracking area updating procedure. The TMSI allocated is then included in the TRACKING AREA UPDATE ACCEPT message together with the location area identification (LAI). In this case the MME shall change to state EMM-COMMON-PROCEDURE-INITIATED and shall start the timer T3450 as described in subclause 5.4.1. The LAI may be included in the TRACKING AREA UPDATE ACCEPT message without TMSI.

The UE, receiving a TRACKING AREA UPDATE ACCEPT message, stores the received location area identification, resets the location update attempt counter, sets the update status to U1 UPDATED and enters MM state MM IDLE.

...

If the UE requested "SMS only" in the Additional update type IE, the network shall indicate "SMS only" in the Additional update result IE.

If the TRACKING AREA UPDATE ACCEPT message includes the Additional update result IE with value "SMS only" or "CS Fallback not preferred", a UE operating in CS/PS mode 1 with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

If the TRACKING AREA UPDATE ACCEPT message includes the Additional update result IE with value "SMS only", a UE operating in CS/PS mode 2 shall not attempt to use CS fallback for mobile originating services.

…

[TS 24.301 clause5.6.2.4]

The network shall initiate the paging procedure when it receives an incoming mobile terminating SMS to the UE that is IMSI attached for non-EPS services or for "SMS only", and no NAS signalling connection exists.

To initiate the procedure for SMS when no NAS signalling connection exists, the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.413 [23]). The paging message shall include a CN domain indicator set to "PS". The paging procedure is performed according to subclause 5.6.2.2.1. The MME shall not start timer T3413 for this procedure.

9.2.3.2.1b.3	Test description

9.2.3.2.1b.3.1	Pre-test conditions

System Simulator:

-	Cell A and Cell B are configured according to Table 6.3.2.2-1 in [18].

-	Cell A belongs to TAI-1(home PLMN) is set to ''Serving Cell''.

-	Cell B belongs to TAI-2(home PLMN and another TAC) is set to ''Non-Suitable cell''

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to LAI-1 and RAI-1, home PLMN) is set to the ''Non-Suitable cell'';

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN);  is set to the ''Non-Suitable cell'';

-	 System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	The UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on cell A according to [18].

9.2.3.2.1b.3.2	Test procedure sequence

Table 9.2.3.2.1b.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
|  | The following messages are sent and shall be received on Cell B | - | - | - | - |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. Set the cell type of cell 5 or cell 24 to ''Suitable neighbour cell''. |  |  |  |  |  |
| 2 | UE transmits a combined TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |  |
| 3 | The SS sends TRACKING AREA UPDATE ACCEPT message including GUTI, TMSI and LAI. The TAI list includes TAI for cell C and cell D. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |  |
| 4 | Void |  |  | - | - |  |
| 5 | Void |  |  | - | - |  |
| - | EXCEPTION: Steps 6a1 to 6b2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |  |
| 6a1 | IF CS/PS mode 1 of operation is configured on the UE THEN perform actions specified in Table 9.2.3.2.1b.3.2-2. |  |  |  |  |  |
| 6a2-6a6 | Void |  |  |  |  |  |
| 6b1 | ELSE the UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |  |
| 6b1A1 | The SS releases the RRC connection. | - | - | - | - |
| 6b2 | Check: Does the UE respond to paging on cell B with S-TMSI in GUTI-2 for PS domain?(Generic Procedure TS 36.508 6.4.2.4) | - | - | 2 | - |  |
| - | EXCEPTION: Steps 6b3a1 to 6b3a2 describe behaviour that depends on the UE capability. | - | - | - | - |
| 6b3 a1 | IF pc_VoLTE THEN an IMS voice call is initiated (see Note 1) | - | - | - | - |  |
| 6b3a2 | Check: Does the UE send RRC CONNECTION REQUEST message? | --> | RRC CONNECTION REQUEST | 3 | F |  |
| Note 1: The request may be triggered by AT command D. |  |  |  |  |  |

Table 9.2.3.2.1b.3.2-2: CS/PS mode 1 of operation behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 to 1b2 are optional and depend on UE implementation. They shall be executed if the UE sends a TRACKING AREA UPDATE COMPLETE or a DETACH REQUEST message during [2] seconds. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met. | - | - | - | - |
| 1a1 | IF UE sent a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2a2 are optional and depend on UE implementation; the second "lower case letter" identifies a step sequence that takes place if the condition is met. | - | - | - | - |
| 1a2a1 | IF UE sends a DETACH REQUEST message during [2] seconds. | --> | DETACH REQUEST | - | - |
| 1a2a2 | SS sends a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 1a2a3 | The SS releases the RRC connection. |  |  |  |  |
| 1b1 | IF UE sent a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 1b2 | SS sends a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 1b3 | The SS releases the RRC connection. |  |  |  |  |
| 2 | Void |  |  |  |  |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: Step 3a1 to 3b5 are optional and depend on UE implementation. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met. |  |  |  |  |
| 3a1 | IF UE did not sent DETACH REQUEST in step 1a2a1 or 1b1 AND UE transmits a ROUTING AREA UPDATE REQUEST message during [2] seconds THEN check contents of ROUTING AREA UPDATE REQUEST | --> | ROUTING AREA UPDATE REQUEST | 1,4 | P |
| - | EXCEPTION: step 3a1Aa1 to 3a1Aa2 take place if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 3a1Aa1 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message from Cell 24 | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3a1Aa2 | The UE transmits a AUTHENTICATION AND CIPHERING RESPONSE message on Cell 24 | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 3a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 3b1 | ELSE check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 1,4 | P |
| - | EXCEPTION: Step 3b2 to 3b3 are performed if UE sent a DETACH REQUEST in step 1a2a1 or 1b1 | - | - | - | - |
| 3b2 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3b3 | The UE transmits a AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3b4 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 3b5 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 3b6 | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) | - | - | - | - |

9.2.3.2.1b.3.3	Specific message contents

Table 9.2.3.2.1b.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.1b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 with condition combined_TA_LA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-1 | GUTI-1 was allocated in cell A during the preamble |  |
| Last visited registered TAI | TAI-1 | TAI of cell A |  |
| Old location area identification | LAI-1 | LAI received in the ATTACH ACCEPT message in the preamble |  |
| TMSI status | 0 | no valid TMSI available |  |

Table 9.2.3.2.1b.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.2.1b.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 with condition combined_TA_LA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | '00000110'B | 6 octets |  |
| Type of list | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| Number of elements | '00000' | 1 elements |  |
| Partial tracking area identity list | PLMN = MCC/MNC stored in EFIMSI TAC 1 = 2 | TAI-2 |  |
| EPS network feature support | 0000 0000'B | MS voice over PS session in S1 mode NOT supported |  |
| Additional update result | '10'B | SMS only |  |

Table 9.2.3.2.1b.3.3-3:  Void

Table 9.2.3.2.1b.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 3a1, Table 9.2.3.2.1b.3.2-2)

| Derivation path: 36.508 table 4.7B.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |  |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |  |
| MS RA capability value part |  |  |  |  |
| Access capabilities |  |  |  |  |
| E-UTRA FDD support : | ‘0’B |  |  |  |
| E-UTRA TDD support | ‘0’B |  |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.1b.3.3-5: Message ATTACH REQUEST (step 3b1, Table 9.2.3.2.1b.3.2-2)

| Derivation path: 36.508 table 4.7B.2-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access capability |  | The UE shall not indicate support for E-UTRAN |  |
| MS RA capability value part |  |  |  |
| Access capabilities |  |  |  |
| E-UTRA FDD support : | ‘0’B |  |  |
| E-UTRA TDD support | ‘0’B |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode | ‘00’B |  |  |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.1b.3.3-6: Message ATTACH ACCEPT (Preamble)

| Derivation Path: TS 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

##### 9.2.3.2.1c	Combined tracking area update / Success / CS Fallback not preferred

9.2.3.2.1c.1	Test Purpose (TP)

(1)

with { UE operating in CS/PS mode 2 in state EMM-REGISTERED-INITIATED}

ensure that {

when { UE receives a TRACKING AREA UPDATE ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred”}

then { UE stays on the E-UTRAN cell.}

}

(2)

with { UE operating in CS/PS mode 2}

ensure that {

when receives a TRACKING AREA UPDATE ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred” }

then { The UE is able to initiate an MO-CS fallback call}

}

(3)

with { UE operating in CS/PS mode 2}

ensure that {

when receives a TRACKING AREA UPDATE ACCEPT message with “EPS network feature support” IE indicating that “IMS voice over PS session in S1 mode NOT supported” and “Additional update result” IE indicating “CS Fallback not preferred” }

then { The UE is able to receive MT CS fallback call}

}

9.2.3.2.1c.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 4.3.1 and 5.5.3.3.4.2.

[TS 24.301 clause 4.3.1]

The behaviour of the UE in CS/PS mode 1 of operation, upon failure to access the CS domain or upon reception of a "CS fallback not preferred" or "SMS only" indication, will depend on the availability of voice over IMS. In the present document, "IMS voice not available" refers to one of the following conditions:

-	the UE is not configured to use IMS;

-	the UE is not configured to use IMS voice, i.e. when the voice domain preference for E-UTRAN, as defined in 3GPP TS 24.167 [13B], indicates that voice communication services are allowed to be invoked only over the CS domain;

-	the UE is configured to use IMS voice, but the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are not supported; or

-	the UE is configured to use IMS voice, the network indicates in the ATTACH ACCEPT message or the TRACKING AREA UPDATE ACCEPT message that IMS voice over PS sessions are supported, but the upper layers indicate that the UE is not available for voice calls in the IMS.

[TS 24.301 clause 5.5.3.3.4.2]

If the TRACKING AREA UPDATE ACCEPT message includes the Additional update result IE with value "SMS only" or "CS Fallback not preferred", a UE operating in CS/PS mode 1 with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

If the TRACKING AREA UPDATE ACCEPT message includes the Additional update result IE with value "SMS only", a UE operating in CS/PS mode 2 shall not attempt to use CS fallback for mobile originating services.

If the TRACKING AREA UPDATE ACCEPT message includes the Additional update result IE with value "CS Fallback not preferred", this indicates to a UE operating in CS/PS mode 2 that it is attached for EPS and non-EPS services and that it can use CS fallback.

9.2.3.2.1c.3	Test description

9.2.3.2.1c.3.1	Pre-test conditions

System Simulator:

- cell A, cell B and cell 5;

System information combination 9 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells

UE:

- 	the UE is configured to initiate combined EPS/IMSI attach;

-	the UE is configured to be data centric.

-	the UE is previously registered on cell 5.

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	if possible, the UE is configured to operate in CS/PS mode 2

-	The UE is in state Registered, Idle mode (state 2) on Cell A (serving cell) according to [18].

9.2.3.2.1c.3.2	Test procedure sequence

Table 9.2.3.2.1c.3.2-1illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1" is applied at the point indicated in the Main behaviour description in Table 9.2.3.2.1c.3.2-2.

Table 9.2.3.2.1c.3.2-1: instances of cell power level and parameter changes

|  | Parameter name | Unit | Cell A | Cell B | Cell 5 |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  | -70 |
|  | P-CCPCH | dBm/1.28 MHz |  |  | -70 |
| T2 | RS EPRE | dBm/15kHz | -115 | -115 |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  | -70 |
|  | P-CCPCH | dBm/1.28 MHz |  |  | -70 |
| T3 | RS EPRE | dBm/15kHz | -91 | -85 |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  | -70 |
|  | P-CCPCH | dBm/1.28 MHz |  |  | -70 |
| T4 | RS EPRE | dBm/15kHz | OFF | OFF |  |
|  | CPICH_Ec | dBm/3.84 MHz |  |  | -70 |
|  | P-CCPCH | dBm/1.28 MHz |  |  | -70 |

Table 9.2.3.2.1c.3.2-2: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjust the cell power levels according to row T1 in table 9.2.3.2.1c.3.2-1. |  |  | - | - |
| 2-7 | Steps 2-7 of the tracking area updating procedure on cell B as described in TS 36.508 table 4.5A.2.1-1 are performed. |  |  | - | - |
| 8 | Check: Does the UE try to access the UTRAN cell (cell 5) in the next 90s? |  |  | 1 | F |
| 10-13 | Steps 1-4 of the procedure in table 13.1.4.3.2-2 are performed. | - | - | - | - |
| - | The UE accepts CS fallback | - | - | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionSetupComplete message containing an EXTENDED SERVICE REQUEST with message content set the same as in step 5 table 13.1.4.3.2-2? | --> | RRCConnectionSetupComplete NAS: EXTENDED SERVICE REQUEST | 3 | P |
| 15-26 | Steps 5A-19 of the procedure described in table 13.1.4.3.2-2 are performed. SS then adjusts the cell power levels according to row T2 in table 9.2.3.2.1c.3.2-1. | - | - | - | - |
| 26A | CS disconnect procedure defined in TS 36.523-3 table 10.1.3.1-1 is performed. SS then adjusts the cell power levels according to row T3 in table 9.2.3.2.1c.3.2-1 so ensure UE reselects cell B. | - | - | - | - |
| 26B-26G | Steps 2-7 of the tracking area updating procedure on cell B as described in TS 36.508 table 4.5A.2.1-1 are performed. | - | - | - | - |
| 27-27B | Steps 1-3 of the procedure in table 13.1.2.3.2-1. (A CS call is initiated) are performed | - | - | - | - |
| 28 | Check: Does the UE transmit EXTENDED SERVICE REQUEST message with the content set as in step 4 of table 13.1.2.3.2-1? | --> | EXTENDED SERVICE REQUEST | 2 | P |
| 29-33 | Steps 8-12 of the procedure in table 13.1.2.3.2-1 are performed. | - | - | - | - |
| 34 | SS adjusts cell levels according to row T4 of table 9.2.3.2.1c.3.2-1. |  |  |  |  |
| - | The UE is in end state UTRA CS fallback (U4). | - | - | - | - |

9.2.3.2.1c.3.3	Specific message contents

Table 9.2.3.2.1c.3.3-1: Message TRACKING AREA UPDATE ACCEPT (steps 5 and 26C, Table 9.2.3.2.1c.3.2-2)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | '001'B | "combined TA/LA updated" |  |
| EPS network feature support | '0000 0000'B | IMS voice over PS session in S1 mode NOT supported |  |
| Additional update result | ‘01’b | “CS Fallback not preferred” |  |

Table 9.2.3.2.1c.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 26B, Table 9.2.3.2.1c.3.2-2)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | "Combined TA/LA updating with IMSI attach” |  |
| Old P-TMSI Signature | Any Value |  |  |
| Additional GUTI | Any Value |  |  |
| Nonce | Any Value |  |  |

Table 9.2.3.2.1c.3.3-3: Message ATTACH ACCEPT (Preamble)

| Derivation Path: TS 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

##### 9.2.3.2.2	Combined tracking area update / Successful for EPS services only / IMSI unknown in HSS

9.2.3.2.2.1	Test Purpose (TP)

(1)

with { UE has sent a combined TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {

when { UE receives a TRACKING AREA UPDATE ACCEPT message with EPS update result set to "TA updated" and EMM cause set to "IMSI unknown in HSS" }

then { UE considers the USIM as invalid for non-EPS services and enters EMM-REGISTERED.NORMAL-SERVICE state and MM idle state }

}

9.2.3.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.5.3.3.4.3.

[TS 24.301 clause 5.5.3.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for tracking area for EPS services as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location updating for non-EPS services applies.

The UE receiving the TRACKING AREA UPDATE ACCEPT message takes one of the following actions depending on the EMM cause value:

#2		(IMSI unknown in HSS)

The UE shall stop T3430 if still running and shall reset the tracking area updating attempt counter. The UE shall set the update status to U3 ROAMING NOT ALLOWED and shall delete any TMSI, LAI and ciphering key sequence number. The UE shall enter state EMM-REGISTERED.NORMAL-SERVICE. The new MM state is MM IDLE. The USIM shall be considered as invalid for non-EPS services until switching off or the UICC containing the USIM is removed.

…

9.2.3.2.2.3	Test description

9.2.3.2.2.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1) is set to ''Serving cell'';

-	cell B (belongs to TAI-2) is set to ''Non-suitable cell''.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined attach

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.2.3.2	Test procedure sequence

Table 9.2.3.2.2.3.2-1: Main Behaviour

| St | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S |  | Message |  |  |  |  |
| 1 | Set the cell type of cell A to the "non-Suitable cell". Set the cell type of cell B to the "Serving cell". |  | - |  | - |  | - |  | - |
| - | The following messages are sent and shall be received on cell B. |  |  |  |  |  |  |  |  |
| 2 | The UE transmits a combined TRACKING AREA UPDATE REQUEST message. |  | --> |  | TRACKING AREA UPDATE REQUEST |  | - |  | - |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result set to "TA updated" and EMM cause set to "IMSI unknown in HSS" |  | <-- |  | TRACKING AREA UPDATE ACCEPT |  | - |  | - |
| 3A | The UE transmits a TRACKING AREA UPDATE COMPLETE message |  | --> |  | TRACKING AREA UPDATE COMPLETE |  | - |  | - |
| 4 | The SS releases the RRC connection. |  | - |  | - |  | - |  | - |
| 5 | The SS pages the UE using S-TMSI with CN domain indicator set to “PS”. |  | <-- |  | - |  | - |  | - |
| 6 | The UE transmits a SERVICE REQUEST message. |  | --> |  | SERVICE REQUEST |  | - |  | - |
| 7 | The SS transmits a DETACH REQUEST message with Detach Type set to "re-attach required" |  | <-- |  | DETACH REQUEST |  | - |  | - |
| 8 | The UE sends a DETACH ACCEPT message. |  | --> |  | DETACH ACCEPT |  | - |  | - |
| 9 | The SS releases the RRC connection. |  | - |  | - |  | - |  | - |
| - | EXCEPTION: Step 9a describes a behaviour which depends on the UE capability |  | - |  | - |  | - |  | - |
| 9a | IF NOT pc_Automatic_Re_Attach, the user initiates an attach by MMI or by AT command. |  | - |  | - |  | - |  | - |
| 10 | Check: Does the UE send ATTACH REQUEST message with EPS attach type set to “EPS attach”, including a PDN CONNECTIVITY REQUEST message? |  | --> |  | ATTACH REQUEST |  | 1 |  | P |
| 11-19 | Steps 5 to 13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  | - |  | - |  | - |  | - |
| 20 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. |  | <-- |  | ATTACH ACCEPT |  | - |  | - |
| 20A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> |  | RRC: RRCConnectionReconfigurationComplete |  | - |  | - |  |
| 21-23b1 | Steps 16 to 18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. |  | - |  | - |  | - |  | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. |  | - |  | - |  | - |  | - |

9.2.3.2.2.3.3	Specific message contents

Table 9.2.3.2.2.3.3-1: Message TRACKING AREA UPDATE REQ (step 2, Table 9.2.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 001 | "combined TA/LA updating" |  |
| Old GUTI | GUTI-1 | Old GUTI is included by UE if valid, IMSI otherwise. |  |
| Last visited registered TAI | TAI-1 | If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| Old P-TMSI signature | Not present |  |  |
| Old LAI | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.2.2.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA updated" |  |
| T3412 value | Not present |  |  |
| GUTI | GUTI-2 |  |  |
| TAI list |  |  |  |
| Length of tracking area identity list contents | 00000110 | 6 octets |  |
| Number of elements | 00000 | 1 element |  |
| Type of list | 00 | One PLMN with non-consecutive TACs |  |
| MCC MNC TAC 1 | TAI-2 |  |  |
| LAC | Not present |  |  |
| MS identity | Not present |  |  |
| T3402 value | Not present |  |  |
| Equivalent PLMNs | Not present |  |  |
| EMM cause | 00000010 | "IMSI unknown in HSS" |  |

Table 9.2.3.2.2.3.3-3: Message DETACH REQUEST (step 7, Table 9.2.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | 001 | "re-attach required" |  |

Table 9.2.3.2.2.3.3-4: Message ATTACH REQUEST (step 10, Table 9.2.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach type | 001 | “EPS attach” |  |

Table 9.2.3.2.2.3.3-5: Message ATTACH ACCEPT (step 20, Table 9.2.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '001'B | ''EPS only'' |  |

##### 9.2.3.2.3	Combined tracking area update / Successful for EPS services only / MSC temporarily not reachable

9.2.3.2.3.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' or 'Combined TA/LA updating with IMSI attach' and having the tracking area updating attempt counter set to the value less than four }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure' }

then { UE sends TRACKING AREA UPDATE REQUEST message after T3411 expiry }

}

(2)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach', having the tracking area updating attempt counter set to four and operating in CS/PS mode 2 of operation }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure' }

then { UE sends TRACKING AREA UPDATE REQUEST message after T3402 expiry }

}

(3)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach', having the tracking area updating attempt counter set to four and operating in CS/PS mode 1 of operation }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure'}

then { UE attempts to select GERAN or UTRAN radio access technology }

}

9.2.3.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.4.3, TS 24.301, clause 4.5.

[TS 24.301, clause 5.5.3.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for tracking area for EPS services as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location updating for non-EPS services applies.

...

#16	(MSC temporarily not reachable); or

#17	(Network failure)

The UE shall stop timer T3430 if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5:

-	the UE shall start timer T3411, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3411 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again.

If the tracking area updating attempt counter is equal to 5:

-	a UE operating in CS/PS mode 2 of operation shall start timer T3402, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3402 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again;

-	a UE operating in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures. The UE shall disable the E-UTRA capability (see subclause 4.5).

...

[TS 24.301, clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN;

-	the UE shall use the same value of the EPC capability bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message;

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability ; and

-	the UE shall change the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN if any capability bit is subject to change after disabling of the UE's E-UTRA capability.NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

The UE shall enable the E-UTRA capability again in the following cases:

- 	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation; or

-	the UE powers off and powers on again.

...

9.2.3.2.3.3	Test description

9.2.3.2.3.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.2.3.3.2	Test procedure sequence

The sequence is executed for execution counter k = 1, 2.

Table 9.2.3.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "non-Suitable cell". Set the cell type of Cell 5 or Cell 24 to the "non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 0A | The UE is switched on or the USIM is inserted or the UE is powered on. | - | - | - | - |
| 0B-0M | Steps 2-13 of the generic test procedure in TS 36.508 subclause 4.5.2.3 is performed. NOTE: The UE performs an ATTACH procedure | - | - | - | - |
| 0N | The SS transmits an ATTACH ACCEPT message | <- | ATTACH ACCEPT |  |  |
| 0O-0Qb1 | Steps 16-18b1 of the generic test procedure in TS 36.508 subclause 4.5.2.3 is performed. NOTE: The UE completes the ATTACH procedure and the RRC connection is released. |  |  |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. Note: T3411 is started on the UE | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 7 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The step 8 to 13 shall be repeated 3 times. | - | - | - | - |
| 8 | Wait for 10s for T3411 to expire. | - | - | - | - |
| 9 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the time when T3411 expired? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 13A1 | Set the cell type of cell 5 or cell 24 to ''Suitable neighbour cell'' and wait for 10s for T3411 to expire. | - | - | - | - |
| 13A2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the time when T3411 expired? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 13A3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 14-19 | Void | - | - | - | - |
| - | EXCEPTION: Steps 20a1 to 20b17 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place according to UE mode of operation. | - | - | - | - |
| 20a1 | IF the UE is configured to operate in CS/PS mode 2 THEN the SS releases the RRC connection. Note: T3402 is started on the UE. | - | - | - | - |
| 20a2 | The SS waits for 30 sec for T3402 to expire. | - | - | - | - |
| 20a3 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 20a4 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20a5 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 20a6 | The SS releases the RRC connection. | - | - | - | - |
| 20a7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 20a8 describes behaviour that depends on the UE capability. |  |  |  |  |
| 20a8 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 20b1-20b16 | Void | - | - | - | - |
| 20b17 | IF the UE is configured in CS/PS mode 1 of operation with “IMS voice not available” THEN perform actions specified in Table 9.2.3.2.3.3.2-2 | - | - | - | - |
| 21 | Void | - | - | - | - |

Table 9.2.3.2.3.3.2-2: CS/PS mode 1 of operation behaviour

| - | EXCEPTION: Step 1a1 to 1a2 are optional and depend on UE implementation. They shall be executed if the UE sends a DETACH REQUEST message during [2] seconds. The "lower case letter" identifies a step sequence that takes place if a particular condition specified is met. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| 1a1 | UE sends a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 1a2 | The SS transmits a DETACH COMPLETE message. | <-- | DETACH COMPLETE | - | - |
| 2 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: Step 3a1 to 3b5 are optional and depend on UE implementation. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met. | - | - | - | - |
| 3a1 | IF UE has not sent a DETACH REQUEST in step 1a1 AND sends a ROUTING AREA UPDATE REQUEST message during [2] seconds THEN check contents of ROUTING AREA UPDATE REQUEST. | --> | ROUTING AREA UPDATE REQUEST | 3 | P |
| - | EXCEPTION: step 3a1Aa1 to 3a1Aa2 take place if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 3a1Aa1 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message on Cell 24 | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3a1Aa2 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message on Cell 24 | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 3a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 3b1 | ELSE Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 3 | P |
| - | EXCEPTION: Step 3b2 to 3b3 are performed if UE sent a DETACH REQUEST in step 1a1 | - | - | - | - |
| 3b2 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3b3 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3b4 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 3b5 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | Exception: Steps 4AAa1 to 4AAa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 4AAa1 | IF UE has sent ROUTING AREA REQUEST at step 3a1 AND pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 4AAa1a1- 4AAa1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 4AAa1a5 | SS releases RRC Connection | - | - | - | - |
| 4AAa1a6 | SS Stops 6s timer | - | - | - | - |
| 4AAa1b1 | The 6s timer expires | - | - | - | - |
| 4A | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) | - | - | - | - |
| 5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 6 describes behaviour that depends on the UE capability. |  |  |  |  |
| 6 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message | --> | DETACH REQUEST | - | - |

9.2.3.2.3.3.3	Specific message contents

Table 9.2.3.2.3.3.3-0: Message ATTACH ACCEPT (step 0N, Table 9.2.3.2.3.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

Table 9.2.3.2.3.3.3-1: TRACKING AREA UPDATE ACCEPT (step 5, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| GUTI | GUTI-2 |  |  |  |
| EPS update result | 000 | "TA updated" |  |  |
| EMM cause | '0001 0000'B for k=1 or '0001 0001'B for k=2 | MSC temporarily not reachable for k=1 Network failure for k=2 |  |  |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |  |

Table 9.2.3.2.3.3.3-2: TRACKING AREA UPDATE REQUEST (step 9 and 20a3, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '010'B | Combined TA/LA updating with IMSI attach |  |

Table 9.2.3.2.3.3.3-3: TRACKING AREA UPDATE ACCEPT (step 12, and 13A3 Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | 000 | "TA updated" |  |
| GUTI | Not present |  |  |
| EMM cause | '0001 0000'B for k=1 or '0001 0001'B for k=2 | MSC temporarily not reachable for k=1 Network failure for k=2 |  |
| T3402 | ‘000 01111’B | 30 seconds |  |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

Table 9.2.3.2.3.3.3-4: TRACKING AREA UPDATE ACCEPT (step 20a4, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | Not present |  |  |
| Location area identification | LAI-1 |  |  |
| MS identity | TMSI-1 |  |  |

Table 9.2.3.2.3.3.3-5: Message ROUTING AREA UPDATE REQUEST (step 3a1, Table 9.2.3.2.3.3.2-2)

| Derivation path: 36.508 table 4.7B.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.3.3.3-6: Message ATTACH REQUEST (step 3b1, Table 9.2.3.2.3.3.2-2)

| Derivation path: 36.508 table 4.7B.2-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.3.3.3-7: DETACH REQUEST (step 1a1, Table 9.2.3.2.3.3.2-2)

Derivation path: 36.508 table 4.7.2-11, condition EPS_only

##### 9.2.3.2.4	Combined tracking area update / Successful for EPS services only / CS domain not available

9.2.3.2.4.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'CS domain not available' }

then { UE transmits TRACKING AREA UPDATE COMPLETE message and set the update status to U2 NOT UPDATED and enters EMM-REGISTERED state }

}

(2)

with { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'CS domain not available' }

ensure that {

when { UE enters a new tracking area }

then { UE initiates the tracking area updating procedure with EPS update type as "TA updating" }

}

9.2.3.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.4.3.

[TS 24.301, clause 5.5.3.3.4.3]

...

#18	(CS domain not available)

The UE shall stop timer T3430 if still running, shall reset the tracking area updating attempt counter, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.NORMAL-SERVICE.

The UE shall set the update status to U2 NOT UPDATED.

A UE in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology rather than E-UTRAN for the selected PLMN or equivalent PLMN. The UE shall disable the E-UTRA capability (see subclause 4.5). If the UE is in the EMM-CONNECTED mode, the UE shall locally release the established NAS signalling connection and enter the EMM-IDLE mode before selecting GERAN or UTRAN radio access technology.

A UE in CS/PS mode 2 of operation may provide a notification to the user or the upper layers that the CS domain is not available.

The UE shall not attempt combined attach or combined tracking area updating procedure with current PLMN until switching off the UE or the UICC containing the USIM is removed.

9.2.3.2.4.3	Test description

9.2.3.2.4.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to "Serving cell''

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'.'

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined attach

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.4.3.2	Test procedure sequence

Table 9.2.3.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell B to the "Serving cell". Set the cell type of Cell A to the "non-Suitable cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 5A | Wait 15 seconds for UE to camp on Cell B (UE might attempt to select GERAN or UTRAN cells) | - | - | - | - |
| 6 | Void | - | - | - | - |
| 6A | Void | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in state Registered, Idle Mode on Cell B? | - | - | 1 | P |
| 8 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "non-Suitable cell". | - | - | - | - |
| 9 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with EPS update type as "TA updating"? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 10 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 11 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is PS attached in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.2.4.3.3	Specific message contents

Table 9.2.3.2.4.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | "combined TA/LA updating” |  |

Table 9.2.3.2.4.3.3-2: TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.2.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |
| EMM cause | '00010010'B | "CS domain not available" |  |
| EPS update result | '000'B | “TA updated” |  |

Table 9.2.3.2.4.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 9, Table 9.2.3.2.4.3.2-1)

Derivation path: 36.508 table 4.7.2-27 with condition TA_only.

Table 9.2.3.2.4.3.3-4: TRACKING AREA UPDATE ACCEPT (step 10, Table 9.2.3.2.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 with condition TA_only. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-1 |  |  |

##### 9.2.3.2.4a	Combined tracking area update / Successful for EPS services only / Congestion

9.2.3.2.4a.1	Test Purpose (TP)

(1)

with { the UE having initiated a Combined tracking area update by sending a TRACKING AREA UPDATE REQUEST message }

ensure that {

when { the UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to #22 'Congestion' }

then { the UE re-initiates a Combined tracking area procedure indicating 'combined TA/LA updating with IMSI attach' after time=T3402 }

}

9.2.3.2.4a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.3.4.3 and 5.2.3.2.7. Unless otherwise stated these are Rel-11 requirements.

[TS 24.301, clause 5.5.3.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for tracking area for EPS services as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location updating for non-EPS services applies.

The UE receiving the TRACKING AREA UPDATE ACCEPT message takes one of the following actions depending on the EMM cause value:

...

#22	(Congestion)

The UE shall stop timer T3430 if still running. The tracking area updating attempt counter shall be set to 5. The UE shall start timer T3402, shall set the EPS update status to EU1 UPDATED, shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, shall enter state MM IDLE.

[TS 24.301, clause 5.2.3.2.7]

The UE:

-	...

-	shall initiate combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" on the expiry of timers T3411 or T3402 or when the UE enters a tracking area not in the list of registered tracking areas.

9.2.3.2.4a.3	Test description

9.2.3.2.4a.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to "Serving cell''

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell"

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.4a.3.2	Test procedure sequence

Table 9.2.3.2.4a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell B to the "Serving cell". Set the cell type of Cell A to the "non-Suitable cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to #22 'Congestion'. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Wait for time=T3402. Note: The value of T3402 is provided in TRACKING AREA UPDATE ACCEPT message in step 3. | - | - | - | - |
| 7 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message indicating "combined TA/LA updating with IMSI attach"? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of the test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.2.4a.3.3	Specific message contents

Table 9.2.3.2.4a.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.4a.3.2-1)

Derivation path: 36.508, Table 4.7.2-27 with condition combined_TA_LA

Table 9.2.3.2.4a.3.3-2: TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.2.4a.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | '0000'B | "TA updated" |  |
| GUTI | GUTI-2 |  |  |
| EMM cause | '00010110'B | "Congestion" |  |
| T3402 | '00001111'B | 30 seconds |  |

Table 9.2.3.2.4a.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7, Table 9.2.3.2.4a.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | "combined TA/LA updating with IMSI attach" |  |

Table 9.2.3.2.4a.3.3-4: TRACKING AREA UPDATE ACCEPT (step 8, Table 9.2.3.2.4a.3.2-1)

Derivation Path: 36.508, Table 4.7.2-24 with condition combined TA/LA updated

##### 9.2.3.2.5	Combined tracking area update / Rejected / IMSI invalid

9.2.3.2.5.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Illegal UE' }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up }

then { UE send ATTACH REQUEST message with Old GUTI or IMSI IE = ''IMSI''}

9.2.3.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

#3		(Illegal UE);

…

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and eKSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.5.3	Test description

9.2.3.2.5.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell C;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	cell A is "Serving cell" and cell B, cell C, cell 9 and cell 24 are "Non-suitable "off" cell".

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.5.3.2	Test procedure sequence

Table 9.2.3.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable "off" cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Illegal UE'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Set the cell type of Cell B to the "non-Suitable "off" cell". Set the cell type of Cell C to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| - | EXCEPTION: Steps 7a1 to 7a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 7a1 | The SS sets the cell type of the cell other than Cell 9 and Cell 24 to the "non-Suitable "off" cell" and sets the cell type of Cell 9 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 9 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 7a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 8 | Set the cell type of the cell other than Cell A to the "non-Suitable "off" cell". Set the cell type of Cell A to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 10 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 11 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message on cell A? | --> | ATTACH REQUEST | 2 | P |
| 13-26 b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.2.5.3.3	Specific message contents

Table 9.2.3.2.5.3.3-1: TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.5.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 0011'B | Illegal UE |  |

Table 9.2.3.2.5.3.3-2: Message ATTACH REQUEST (step 12, Table 9.2.3.2.5.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |

##### 9.2.3.2.6	Combined tracking area update / Rejected / Illegal ME

9.2.3.2.6.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Illegal ME' }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up }

then { UE send ATTACH REQUEST message with Old GUTI or IMSI IE = ''IMSI''}

9.2.3.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

…

#6		(Illegal ME); or

…

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and eKSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.6.3	Test description

The test description is identical to the one of subclause 9.2.3.2.5 except that the reject cause #3 "Illegal UE" is replaced with the reject cause #6 "Illegal ME".

##### 9.2.3.2.7	Combined tracking area update / Rejected / EPS services and non-EPS services not allowed

9.2.3.2.7.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'EPS services and non-EPS services not allowed' }

then { UE considers the USIM as invalid for EPS services and non-EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { The UE is in the state EMM-DEREGISTERED }

ensure that {

when { UE is powered up }

then { UE send ATTACH REQUEST message with Old GUTI or IMSI IE = ''IMSI''}

9.2.3.2.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

…

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(EPS services and non-EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and KSI.

The UE shall consider the USIM as invalid for EPS and non-EPS services until switching off or the UICC containing the USIM is removed. Additionally, the UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

…

9.2.3.2.7.3	Test description

The test description is identical to the one of subclause 9.2.3.2.5 except that the reject cause #3 "Illegal UE" is replaced with the reject cause #8 " EPS services and non-EPS services not allowed".

##### 9.2.3.2.8	Combined tracking area update / Rejected / EPS services not allowed

9.2.3.2.8.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'EPS services not allowed' }

then { UE considers the USIM as invalid for EPS services and enters state EMM-DEREGISTERED }

}

(2)

with { UE in CS/PS mode 1 or CS/PS mode 2 of operation and have been IMSI attached for non-EPS services }

ensure that {
  when { UE sends a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' and receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'EPS services not allowed' }

then { The UE shall still IMSI attached for non-EPS services }

}

9.2.3.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

…

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and KSI. The UE shall consider then USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.

A UE in CS/PS mode 1 or CS/PS mode 2 of operation is still IMSI attached for non-EPS services. The UE shall set the update status to U2 NOT UPDATED, shall select GERAN or UTRAN radio access technology and proceed with appropriate MM specific procedure according to the MM service state. The UE shall not reselect E-UTRAN radio access technology until switching off or the UICC containing the USIM is removed.

NOTE:	Some interaction is required with the access stratum to disable E-UTRAN cell reselection.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.8.3	Test description

9.2.3.2.8.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2(, home PLMN) is set to ''Non-suitable cell'';

-	cell G (belongs to TAI-7, visited PLMN) is set to ''Non-suitable off cell'';

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	Set the cell type of cell 9 or cell 24 to the '' Non-Suitable off cell ''.

-	the different cells may not be simultaneously activated.

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.8.3.2	Test procedure sequence

Table 9.2.3.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell A as the "non-Suitable cell". Cell B as the "Serving cell". | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell B. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to as ''EPS services not allowed'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | The SS configures: Cell A as the "Serving cell". Cell B as the "non-Suitable cell". | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell A? Note: Cell A belongs to the same PLMN where the UE was rejected but a different TAI | --> | ATTACH REQUEST | 1 | F |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell A? | --> | ATTACH REQUEST | 1 | F |
| 9 | The SS configures: Cell A as the "non-Suitable cell", Cell B as the "non-Suitable “off” cell". Cell G as the "Serving cell". | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? Note: Cell G belongs to a PLMN which is not the same like the one on which the UE was rejected. | --> | ATTACH REQUEST | 1 | F |
| 11 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? | --> | ATTACH REQUEST | 1 | F |
| 13 | The SS configures: Cell G as the "Serving cell". Cell 9 or 24 as the "Suitable cell". | - | - | - | - |
|  | EXCEPTION: Steps 14a1 to 14a5 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 14a1 | The following messages are sent and shall be received on Cell 9 (px_RATComb_Tested = EUTRA_UTRA) or 24 (px_RATComb_Tested = EUTRA_GERAN). | - | - | - | - |
| 14a2 | Check: Does the UE transmit a LOCATION UPDATING REQUEST message with Location updating type set to "Normal location updating" on Cell 9 or 24? | --> | LOCATION UPDATING REQUEST | 2 | P |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 14a2Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |  |  |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 14a2Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |  |  |
| 14a3 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 14a4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 14a5 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 15 | The SS configures: Cell G as the "Serving cell". Cell 9 or 24 as the "non-Suitable cell". | - | - | - | - |
| 16 | The user initiates attach by MMI or by AT command. | - | - | - | - |
| 17 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? | --> | ATTACH REQUEST | 1 | F |

9.2.3.2.8.3.3	Specific message contents

Table 9.2.3.2.8.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.8.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | " combined TA/LA updating " |  |
| Old GUTI | GUTI-1 |  |  |

Table 9.2.3.2.8.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.8.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00000111'B | "EPS services not allowed'' |  |

Table 9.2.3.2.8.3.3-3: LOCATION UPDATING REQUEST (step 14a2, Table 9.2.3.2.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Location updating type | Normal location updating |  |  |

Table 9.2.3.2.8.3.3-4: LOCATION UPDATING ACCEPT (step 14a5, Table 9.2.3.2.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 9.2.3.2.9	Combined tracking area update / Rejected / UE identity cannot be derived by the network

9.2.3.2.9.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'UE identity cannot be derived by the network' }

then { UE deletes GUTI, last visited registered TAI, TAI list and KSI, enters the state EMM-DEREGISTERED, subsequently, automatically initiates the attach procedure and is still IMSI attached for non-EPS services }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'UE identity cannot be derived by the network' }

then { UE deletes P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number }

}

9.2.3.2.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5 and TS 24.008, clause 4.7.5.2.4.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#9		(UE identity cannot be derived by the network);

The UE shall set the EPS update status to EU2 NOT UPDATED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and eKSI. The UE shall delete the list of equivalent PLMNs and enter the state EMM-DEREGISTERED.

Subsequently, the UE shall automatically initiate the attach procedure.

NOTE 2:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

A UE in CS/PS mode 1 or CS/PS mode 2 of operation is still IMSI attached for non-EPS services. The UE shall set the update status to U2 NOT UPDATED.

[TS 24.008, clause 4.7.5.2.4]

The MS shall then take different actions depending on the received reject cause:

...

# 9			(MS identity cannot be derived by the network);

The MS shall set the GPRS update status to GU2 NOT UPDATED (and shall store it according to subclause 4.1.3.2), enter the state GMM-DEREGISTERED, and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. Subsequently, the MS may automatically initiate the GPRS attach procedure.

9.2.3.2.9.3	Test description

9.2.3.2.9.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non-suitable cell'';

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to RAI-1, home PLMN) is set to '"Non-Suitable cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-1, home PLMN) is set to '"Non-Suitable cell";

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

- 	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.9.3.2	Test procedure sequence

Table 9.2.3.2.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the ''Non-Suitable cell''. Set the cell type of Cell B to the ''Serving cell''. | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause set to ''UE identity cannot be derived by the network''. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 3a1-3a2 describes the behaviour that depends on UE behaviour (Note 2). | - | - | - | - |
| 3a1 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 3a2 describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a2 | IF NOT pc_Automatic_EPS_Re_Attach , the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | P |
| 4Aa1-4Aa13 | Void | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5a28 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 5a1 | The SS sets the cell type of Cell B to the “non-Suitable cell" and sets the cell type of Cell 5 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 5a2 | Void | - | - | - | - |
| 5a3 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 5a4-5a24 | Void | - | - | - | - |
| 5a25-5a28 | Void | - | - | - | - |
| 5a29 | The SS transmits an ATTACH REJECT message | <-- | ATTACH REJECT | - | - |
| - | At the end of this test procedure sequence, the UE is in end state UTRA deregistered (U6) or GERAN deregistered (G5) according to TS 36.508. | - | - | - | - |
| Note 2: SS waits for 1 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1 second, existing RRC Connection is released. |  |  |  |  |  |

9.2.3.2.9.3.3	Specific message contents

Table 9.2.3.2.9.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.9.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.2.9.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.9.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1001'B | UE identity cannot be derived by the network |  |

Table 9.2.3.2.9.3.3-3: Message ATTACH REQUEST (step 4, Table 9.2.3.2.9.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI |  |  |
| Old P-TMSI signature | Not present |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not Present or LAI-1 |  |  |
| TMSI status | Not present or any allowed value |  |  |

Table 9.2.3.2.9.3.3-4: Message ATTACH REQUEST (step 5a3, Table 9.2.3.2.9.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | '011'B | Combined GPRS/IMSI attach |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI-1 |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | Not present or any allowed value |  |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.3.2.9.3.3-5: Message ATTACH REJECT (step 5a29, Table 9.2.3.2.9.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GMM cause | ‘00001000’B | GPRS services and non-GPRS services not allowed |  |

##### 9.2.3.2.10	Combined tracking area update / Rejected / UE implicitly detached

9.2.3.2.10.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Implicitly detached' }

then { UE sends an ATTACH REQUEST message }

}

9.2.3.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#10	(Implicitly detached);

The UE shall delete the list of equivalent PLMNs and shall enter the state EMM-DEREGISTERED.NORMAL-SERVICE. The UE shall perform a new attach procedure.

NOTE 3:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

...

9.2.3.2.10.3	Test description

9.2.3.2.10.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	cell A is set to the "Serving cell" and cell B is set to the "non-Suitable cell".

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to 36.508 [18].

9.2.3.2.10.3.2	Test procedure sequence

Table 9.2.3.2.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Implicitly detached'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 3a1-3a2 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 3a1 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 3a2 describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a2 | IF NOT pc_Automatic_EPS_Re_Attach , the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 5-18b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. |  |  |  |  |  |

9.2.3.2.10.3.3	Specific message contents

Table 9.2.3.2.10.3.3-1: TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.10.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1010'B | Implicitly detached |  |

##### 9.2.3.2.11	Combined tracking area update / Rejected / PLMN not allowed

9.2.3.2.11.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'PLMN not allowed' }

then { UE deletes GUTI, last visited registered TAI, TAI list and KSI, enters the state EMM-DEREGISTERED.PLMN-SEARCH, stores the PLMN identity in the "forbidden PLMN list" }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'PLMN not allowed' }

then { UE deletes P-TMSI, P-TMSI signature, TMSI, RAI, LAI, ciphering key sequence number GPRS ciphering key sequence number }

}

(3)

with { UE is switched off and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { UE is powered on this PLMN }

then { UE doesn’t perform an attach procedure }

}

(4)

with { UE in EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { UE enters a cell belonging to a PLMN which is not in the "forbidden PLMN list" }

then { UE initiates an attach procedure }

}

(5)

with { UE in EMM-DEREGISTERED.PLMN-SEARCH state and a PLMN is stored in the "forbidden PLMN list" }

ensure that {

when { UE is in a forbidden PLMN cells and when the PLMN is selected manually }

then { UE initiates an attach procedure }

}

9.2.3.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5, TS 24.008, clause 4.7.5.2.4, and TS 23.122, clause 4.4.3.1.2.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#11	(PLMN not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and eKSI, and reset the tracking area updating attempt counter. The UE shall delete the list of equivalent PLMNs and enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMN list".

The UE shall then perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle and the MM parameters update status, TMSI, LAI, ciphering key sequence number and the location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value and no RR connection exists.

[TS 24.008, clause 4.7.5.2.4]

# 11	(PLMN not allowed);

The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED and the update status to U3 ROAMING NOT ALLOWED (and shall store it according to subclause 4.1.3.2) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, TMSI, RAI, LAI, ciphering key sequence number GPRS ciphering key sequence number, and reset the routing area updating attempt counter and the location update attempt counter.

[TS 23.122, clause 4.4.3.1.2]

The user may select his desired PLMN and the MS then initiates registration on this PLMN using the access technology chosen by the user for that PLMN or using the highest priority available access technology for that PLMN, if the associated access technologies have a priority order. (This may take place at any time during the presentation of PLMNs). For such a registration, the MS shall ignore the contents of the "forbidden LAs for roaming", "forbidden TAs for roaming", "forbidden LAs for regional provision of service", "forbidden TAs for regional provision of service", "forbidden PLMNs for GPRS service" and "forbidden PLMNs" lists.

9.2.3.2.11.3	Test description

9.2.3.2.11.3.1	Pre-test conditions

System Simulator:

-	cell G (belongs to TAI-7, visited PLMN) is set to ''Serving cell'';

-	cell H (belongs to TAI-8, visited PLMN) is set to ''Non-suitable cell'';

-	cell I (belongs to TAI-9, visited PLMN, another PLMN) is set to ''Non-suitable off cell'';

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except replacing f3 with f1;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA,  cell 9 (belongs to LAI-1 and RAI-1, visited PLMN) is configured;

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, visited PLMN) is configured;

-	Set the cell type of cell 5 or cell 24 to the '' Non-Suitable cell ''.

-	cell G,  cell 9 and cell 24 are in same PLMN.

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell G using default message contents according to TS 36.508 [18];

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell G according to [18].

9.2.3.2.11.3.2	Test procedure sequence

Table 9.2.3.2.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell G to the ''Non-Suitable cell''. Set the cell type of Cell H to the ''Serving cell''. | - | - | - | - |
| - | The following messages are to be observed on Cell H unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'PLMN not allowed'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 6 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 7 | Set the cell type of Cell G to the ''Serving cell'''. Set the cell type of Cell H to the ''Non-Suitable cell''. NOTE: Cell G and Cell H are in the same PLMN. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 8 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 9 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 3 | F |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported | - | - | - | - |
| 10a1 | The cell type of Cell G and Cell I to the “non-Suitable cell", the cell type of Cell H to “non-Suitable “off” cell” and sets the cell type of Cell 9 (px_RATComb_Tested = EUTRA_UTRA) or Cell 24 (px_RATComb_Tested = EUTRA_GERAN) to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 9 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 10a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 11 | Set the cell type of Cell I to the “Serving cell". Set the cell type of Cell 9 or Cell 24 to the “non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell I unless explicitly stated otherwise. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1, 2, 4 | P |
| 13-21 | Steps 5 to 13 of the generic test procedure in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs an ATTACH procedure. | - | - | - | - |
| 22 | The SS responds with an ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in step 23 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| 23 | The UE transmits an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | ATTACH COMPLETE | - | - |
| 24 | The SS releases the RRC connection. | - | - | - | - |
| 25 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 26 | The UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 26A | The SS releases the RRC connection. | - | - | - | - |
| 27 | Set the cell type of Cell G to the ''Serving cell'''. Set the cell type of Cell I to the ''Non-Suitable cell''. | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 28 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 29 | The UE is switched to manual PLMN selection mode and is made to select the forbidden PLMN cell. | - | - | - | - |
| 30 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 5 | P |
| 31-43 | Steps 5 to 17 of the generic test procedure in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs an ATTACH procedure and the RRC connection is released. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |

9.2.3.2.11.3.3	Specific message contents

Table 9.2.3.2.11.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI | GUTI-7 |  |  |
| Old location area identification | LAI-1 |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.2.11.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1011'B | PLMN not allowed |  |

Table 9.2.3.2.11.3.3-3: Message ATTACH REQUEST (step 12, Table 9.2.3.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI-1 |  |  |
| Old P-TMSI signature | Not present |  |  |
| Additional GUTI | Not present |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | '0'B | no valid TMSI available |  |

Table 9.2.3.2.11.3.3-4: Message ATTACH ACCEPT (step 22, Table 9.2.3.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-9 |  |  |
| Location area identification | LAI-9 |  |  |
| MS identity | TMSI-9 |  |  |

Table 9.2.3.2.11.3.3-5: Message ATTACH REQUEST (step 30, Table 9.2.3.2.11.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-9 |  |  |
| Old P-TMSI signature | Not present |  |  |
| Additional GUTI | Not present |  |  |
| Last visited registered TAI | TAI-9 |  |  |
| Old location area identification | LAI-9 |  |  |
| TMSI status | Not present |  |  |

##### 9.2.3.2.12	Combined tracking area update / Rejected / Tracking area not allowed

9.2.3.2.12.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Tracking Area not allowed' }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, deletes any GUTI, last visited registered TAI, TAI List and KSI, enters the state EMM-DEREGISTERED.LIMITED-SERVICE and stores the current TAI in the list of "forbidden tracking areas for regional provision of service" }

}

(2)

with { UE in EMM-DEREGISTERED.LIMITED-SERVICE state having the list of "forbidden tracking areas for regional provision of service" }

ensure that {
  when { serving cell belongs to TAI where UE was rejected }

then { UE does not attempt to attach }

}

(3)

with { UE in EMM-DEREGISTERED.LIMITED-SERVICE state having the list of "forbidden tracking areas for regional provision of service" }

ensure that {
  when { UE reselects a new cell which belongs to the TAI in the list of "forbidden tracking areas for regional provision of service" }

then { UE does not attempt to attach }

}

(4)

with { UE is powered off or switched off }

ensure that {
  when { UE is powered on or switched on in the cell belonging to the TAI which was in the list of "forbidden tracking areas for regional provision of service" before the UE was powered off or switched off }

then { UE attempts to attach }

}

(5)

with { UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and the current TAI in the list of "forbidden tracking areas for regional provision of service"}

ensure that {

when { UE reselects a new cell which does not belong to any of the TAI in the list of "forbidden tracking areas for regional provision of service""}

then { UE initiates attach procedure with IMSI }

}

9.2.3.2.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#12	(Tracking area not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI List and KSI. The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for regional provision of service".

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and the location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

...

9.2.3.2.12.3	Test description

9.2.3.2.12.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell M;

-	cell A is set to the "Non-suitable", and cell B set to the “Serving cell” and cell M is set to the "non-Suitable cell".

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell B according to TS 36.508 [18].

9.2.3.2.12.3.2	Test procedure sequence

Table 9.2.3.2.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | Set the cell type of Cell B to the "Non-suitable". Set the cell type of Cell A to the "Serving cell". | - | - | - | - |  |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |  |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |  |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'Tracking Area not allowed'. | <-- | TRACKING AREA UPDATE REJECT | - | - |  |
| 4 | The SS releases the RRC connection. | - | - | - | - |  |
| 4A | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Cell A? | --> | ATTACH REQUEST | 1,2 | F |  |
| 6 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell M to the "Serving cell". | - | - | - | - |  |
| - | The following messages are to be observed on Cell M unless explicitly stated otherwise. | - | - | - | - |  |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 3 | F |  |
| 8 | Set the cell type of Cell B to the "Serving cell". Set the cell type of Cell M to the "non-Suitable cell". | - | - | - | - |  |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |  |
| 8A | Check: Does the UE transmit an ATTACH REQUEST message on cell B including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1,5 | P |
| 8B-8Ob1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| 9 | Void | - | - | - | - |  |
| 10 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |  |
|  | EXCEPTION: Step 10Aa1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 10Aa1 | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST | - | - |
|  | Set the cell type of cell A to the '' Serving cell''. Set the cell type of cell B to the '' Non-Suitable cell''. |  |  |  |  |
| 11 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |  |
| 12 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 4 | P |  |
| 13-26b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |  |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle(E1) according to TS 36.508. | - | - | - | - |  |

9.2.3.2.12.3.3	Specific message contents

Table 9.2.3.2.12.3.3-1: TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.12.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1100'B | Tracking Area not allowed |  |

Table 9.2.3.2.12.3.3-2: ATTACH REQUEST (step 8A, Table 9.2.3.2.12.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI of the UE |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.2.12.3.3-3: ATTACH REQUEST (step 12, Table 9.2.3.2.12.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI or IMSI | GUTI-2 |  |  |
| Last visited registered TAI | TAI-2 |  |  |

NOTE:	GUTI-2 and TAI-2 are GUTI and TAI allocated when the UE registered at Cell B in the preamble.

##### 9.2.3.2.13	Combined tracking area update / Rejected / Roaming not allowed in this tracking area

9.2.3.2.13.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {

when { the UE receives TRACKING AREA UPDATE REJECT message with the reject cause set to "roaming not allowed in this tracking area" }

then { the UE sets the EPS update status to EU3 ROAMING NOT ALLOWED and the UE deletes the last visited registered TAI and the UE enters the state EMM-REGISTERED.PLMN-SEARCH and the UE stores the current TAI in the list of "forbidden tracking areas for roaming" }

}

(2)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the current TAI in the list of "forbidden tracking areas for roaming"}

ensure that {

when { the serving cell belongs to TAI where UE was rejected }

then { the UE does not attempt to send TRACKING AREA UPDATE REQUEST message }

}

(3)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE enters a cell belonging to same PLMN and TAI not in the list of "forbidden tracking areas for roaming"}

then { the UE sends TRACKING AREA UPDATE REQUEST message with EPS update type set to "combined TA/LA updating with IMSI attach"}

}

(4)

with { the UE is in EMM-REGISTERED.PLMN-SEARCH state and the TAI of the current cell belongs to the list of "forbidden tracking areas for roaming"}

ensure that {

when { the UE enters a cell belonging to another PLMN }

then { the UE sends TRACKING AREA UPDATE REQUEST message with EPS update type set to "combined TA/LA updating with IMSI attach" }

}

(5)

with { the UE, which A/Gb mode or Iu mode is supported by the UE, in EMM-DEREGISTERED.PLMN-SEARCH state and the list of "forbidden tracking areas for roaming" contains more than one TAI }

ensure that {

when { the UE move to UTRAN or GERAN cell }

then { the UE sends ROUTING AREA UPDATE REQUEST message }

9.2.3.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#13	(Roaming not allowed in this tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete the list of equivalent PLMNs. The UE shall reset the tracking area updating attempt counter and shall change to state EMM-REGISTERED.PLMN-SEARCH.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list if present.

The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

The UE shall indicate the Update type IE "combined TA/LA updating with IMSI attach" when performing the tracking area updating procedure following the PLMN selection.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status and the location update attempt counter, and the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

…

9.2.3.2.13.3	Test description

9.2.3.2.13.3.1	Pre-test conditions

System Simulator:

-	cell E (belongs to TAI-12, visited PLMN) is set to ''Serving cell'';

-	cell I (belongs to TAI-9, same visited PLMN) is set to ''Non-suitable cell'';

-	cell B (belongs to TAI-2, another visited PLMN) is set to ''Non-suitable off cell'';

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in36.508[18], except that cell E and cell I are on different frequencies;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA,  cell 9 (belongs to RAI-8, visited PLMN) is set to "Non-suitable cell";

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to RAI-8, visited PLMN) is set to "Non-suitable cell";

-	system information indicate that NMO 1 is used.

-	System information combination 10 a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	The UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell E according to TS 36.508 [18].

9.2.3.2.13.3.2	Test procedure sequence

Table 9.2.3.2.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell E to the ''Suitable cell''. Set the cell type of Cell I to the ''Serving cell''. | - | - | - | - |
| 2 | The UE transmits TRACKING AREA UPDATE REQUEST on Cell I. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''Roaming not allowed in this tracking area'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. |  |  |  |  |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message on cell E? | --> | TRACKING AREA UPDATE REQUEST | 1, 3 | P |
| 8 | The SS transmits TRACKING AREA UPDATE REJECT message with EMM cause = ''Roaming not allowed in this tracking area'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 9 | The SS releases the RRC connection. |  |  |  |  |
| 10 | Set the cell type of Cell E and Cell B to the ''Non-Suitable cell”, set the cell type of cell I to the ''Non-Suitable "off" cell''. Set the cell type of cell 9 or 24 to the ''Serving cell''. Note: Cell E and Cell 9 or 24 are in the same PLMN. | - | - | - | - |
|  | EXCEPTION: Steps 11a1 to 11a3 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 11a1 | Check: Does the UE transmit ROUTING AREA UPDATE REQUEST message on cell 9 or 24? | --> | ROUTING AREA UPDATE REQUEST | 5 | P |
| 11a2 | The SS transmits a ROUTING AREA UPDATE REJECT message with cause = '' Roaming not allowed in this routing area '' as specified. | <-- | ROUTING AREA UPDATE REJECT | - | - |
| 11a3 | The SS releases the RRC connection. |  |  |  |  |
| 12 | Set the cell type of cell 9 or 24 to the '' Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. Note: cell 9 or 24 and cell B are in different PLMNs. | - | - | - | - |
| 13 | Check: Does the UE transmit TRACKING AREA REQUEST message on cell B? | --> | TRACKING AREA UPDATE REQUEST | 1, 4 | P |
| 14 | The SS sends TRACKING AREA ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15 | The UE transmits TRACKING AREA COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 16 | SS Releases the RRC Connection. | - | - | - | - |
| 16a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E1) according to TS 36.508. | - | - | - | - |

9.2.3.2.13.3.3	Specific message contents

Table 9.2.3.2.13.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.13.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI | GUTI-7 |  |  |

Table 9.2.3.2.13.3.3-2: TRACKING AREA UPDATE REJECT (step 3 and 8, Table 9.2.3.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1101'B | Roaming not allowed in this tracking area |  |

Table 9.2.3.2.13.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7 and 13, Table 9.2.3.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '010'B | Combined TA/LA updating with IMSI attach |  |

Table 9.2.3.2.13.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 11a1, Table 9.2.3.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.7B.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Update type | ‘010’B | Combined RA/LA updating with IMSI attach |  |

##### 9.2.3.2.14	Combined tracking area update / Rejected / EPS services not allowed in the PLMN

9.2.3.2.14.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'EPS services not allowed in this PLMN' }

then { UE deletes any GUTI, last visited registered TAI, TAI List and eKSI, and UE stores the PLMN identity in the "forbidden PLMNs for GPRS service" list }

}

(2)

with { UE is in EMM-DEREGISTERED.PLMN-SEARCH state }

ensure that {
  when { UE detects a E-UTRAN cell which belongs to the same PLMN which is stored in the "forbidden PLMNs for GPRS service" list }

then { UE does not attempt to access on this cell }

}

(3)

with { UE is IMSI attached for non-EPS services and is in EMM-DEREGISTERED.PLMN-SEARCH state and one PLMN stored in the "forbidden PLMNs for GPRS service" list }

ensure that {
  when { UE update status is U2 NOT UPDATED and detects a new GERAN or UTRAN cell which belongs to the PLMN }

then { UE initiates location update procedure in this GERAN or UTRAN cell }

}

(4)

with { UE is in EMM-DEREGISTERED.PLMN-SEARCH state }

ensure that {
  when { UE detects a E-UTRAN cell which belongs to another PLMN which is not stored in the "forbidden PLMNs for GPRS service" list }

then { UE attempts to access on this cell }

}

9.2.3.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

…

#14	(EPS services not allowed in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). Furthermore the UE shall delete any GUTI, last visited registered TAI, TAI List and eKSI. The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-DEREGISTERED.PLMN-SEARCH.

The UE shall store the PLMN identity in the "forbidden PLMNs for GPRS service" list.

The UE operating in CS/PS mode 1 or CS/PS mode 2 of operation is still IMSI attached for non-EPS services and shall set the update status to U2 NOT UPDATED.

A UE operating in CS/PS mode 1 of operation may select GERAN or UTRAN radio access technology and proceed with the appropriate MM specific procedure according to the MM service state. In this case the UE shall not reselect E-UTRAN radio access technology for the duration the UE is on the PLMN or equivalent PLMN.

A UE in CS/PS mode 1 of operation may perform a PLMN selection according to 3GPP TS 23.122 [6].

A UE operating in CS/PS mode 2 of operation shall perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.14.3	Test description

9.2.3.2.14.3.1	Pre-test conditions

System Simulator:

-	cell G (visited PLMN) is set to ''Serving cell'';

-	cell H (visited PLMN, ) is set to ''Non-suitable cell'';

-	cell I (another visited PLMN) is set to ''Non-suitable off cell'';

-	the cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 36.508[18], except replacing f3 with f1 for cell J, and replacing f3 with f1 for cell I in Table 6.3.2.2-3 in 36.508[18];

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 9 (visited PLMN) is set to ''Non- Suitable cell'';

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (visited PLMN) is set to ''Non- Suitable cell';

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined attach

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell G according to TS 36.508 [18].

9.2.3.2.14.3.2	Test procedure sequence

Table 9.2.3.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell G as the "non-Suitable cell". Cell H as the "Serving cell". | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Cell H. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to as ''EPS services not allowed in this PLMN'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | The SS configures: Cell H as the "non-Suitable cell". Cell G as the "Serving cell". | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on cell G? | --> | ATTACH REQUEST | 1, 2 | F |
| 7 | The SS configures: Cell G and Cell I as a "non-Suitable cell". Cell H as “non-Suitable “off” cell” Cell 9 or 24 as the "Serving cell". | - | - | - | - |
|  | EXCEPTION: Steps 8a1 to 8a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 8a1 | The following messages are sent and shall be received on Cell 9 or 24. Check: Does the UE transmit a LOCATION UPDATING REQUEST message on Cell 9 or cell 24? | --> | LOCATION UPDATING REQUEST | 3 | P |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 8a1Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |  |  |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 8a1Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |  |  |
| 8a2 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 8a3 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 8a4 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 9 | The SS configures: Cell I as the ”Serving cell". Cell 9 or 24 as the "non-Suitable cell". | - | - | - | - |
| 10 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN on cell I? | --> | ATTACH REQUEST | 1, 4 | P |
| 11-224a1 | The attach procedure is completed by executing steps 5 to 18a2of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle(E1) according to TS 36.508. | - | - | - | - |

9.2.3.2.14.3.3	Specific message contents

Table 9.2.3.2.14.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | "combined TA/LA updating " |  |
| Old GUTI | GUTI-7 |  |  |

Table 9.2.3.2.14.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '00001110'B | " EPS services not allowed in this PLMN '' |  |

Table 9.2.3.2.14.3.3-3: Message ATTACH REQUEST (step 10, Table 9.2.3.2.14.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.2.3.2.14.3.3-4: LOCATION UPDATING REQUEST (step 8a1, Table 9.2.3.2.14.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Location updating type | Normal update |  |  |

Table 9.2.3.2.14.3.3-5: LOCATION UPDATING ACCEPT (step 8a4, Table 9.2.3.2.14.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 9.2.3.2.15	Combined tracking area update / Rejected / No suitable cells in tracking area

9.2.3.2.15.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {
  when { UE receives a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'No Suitable Cells In tracking area' }

then { UE selects a suitable cell in another tracking area in the same PLMN and performs the tracking area updating procedure with EPS update type set to 'combined TA/LA updating with IMSI attach' }

}

9.2.3.2.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

If the combined tracking area updating cannot be accepted by the network, the MME shall send a TRACKING AREA UPDATE REJECT message to the UE including an appropriate EMM cause value.

Upon receiving the TRACKING AREA UPDATE REJECT message, the UE shall stop timer T3430, stop any transmission of user data, enter state MM IDLE, and take the following actions depending on the EMM cause value received.

...

#15	(No suitable cells in tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-REGISTERED.LIMITED-SERVICE.

The UE shall store the current TAI in the list of "forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list if present.

The UE shall search for a suitable cell in another tracking area or in another location area in the same PLMN according to 3GPP TS 36.304 [21].

The UE shall indicate the Update type IE "combined TA/LA updating with IMSI attach" when performing the tracking area updating procedure.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status and the location update attempt counter, and the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.15.3	Test description

9.2.3.2.15.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell C;

-	cell A is set to the "Serving cell" and cell B and cell C is set to the "Non-suitable cell".

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.2.15.3.2	Test procedure sequence

Table 9.2.3.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". Set the cell type of Cell C to the "Suitable neighbour cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'No Suitable Cells In tracking area'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are to be observed on Cell C unless explicitly stated otherwise. | - | - | - | - |
| 5 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'combined TA/LA updating with IMSI attach'? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 6 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.2.15.3.3	Specific message contents

Table 9.2.3.2.15.3.3-1: TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.15.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1111'B | No Suitable Cells In tracking area |  |

Table 9.2.3.2.15.3.3-2: TRACKING AREA UPDATE REQUEST (step 5, Table 9.2.3.2.15.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '010'B | Combined TA/LA updating with IMSI attach |  |

##### 9.2.3.2.16	Combined tracking area update / Rejected / Not authorized for this CSG

9.2.3.2.16.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE removes the CSG ID from the Allowed CSG list }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' }

ensure that {

when { UE receives a TRACKING AREA UPDATE REJECT message with the reject cause set to "Not authorized for this CSG" and with integrity protection }

then { UE searches for a suitable cell in the same PLMN and sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to ' combined TA/LA updating with IMSI attach ' }

}

9.2.3.2.16.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.3.5.

[TS 24.301, clause 5.5.3.3.5]

#25	(Not authorized for this CSG);

Cause #25 is only applicable when received from a CSG cell. Cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.3.3.6.

If the TRACKING AREA UPDATE REJECT message with cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). The UE shall reset the tracking area updating attempt counter and shall enter the state EMM-REGISTERED.LIMITED-SERVICE.

If the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list, the UE shall remove the CSG ID from the Allowed CSG list.

If the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Operator CSG list, the UE shall apply the procedures defined in 3GPP TS 23.122 [6] subclause 3.1A.

The UE shall search for a suitable cell in the same PLMN according to 3GPP TS 36.304 [21].

The UE shall indicate the Update type IE "combined TA/LA updating with IMSI attach" when performing the tracking area updating procedure.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the MM parameters update status and the location update attempt counter, and the GMM parameters GMM state, GPRS update status and routing area updating attempt counter as specified in 3GPP TS 24.008 [13] for the case when the combined routing area updating procedure is rejected with the GMM cause with the same value.

9.2.3.2.16.3	Test description

9.2.3.2.16.3.1	Pre-test conditions

System Simulator:

-	cell A (TAI-1, frequency 1, HPLMN,  not a CSG cell) is set to ''Serving cell'';

-	cell B (TAI-2, frequency 1, HPLMN,  is a CSG cell) is set to ''Non-suitable cell'';

-	cell C (TAI-3, frequency 1, HPLMN,  not a CSG cell) is set to ''Non-suitable off cell''.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is previously registered on cell B using either manual CSG selection or a USIM with field EFACSGL preconfigured  (so the allowed CSG list includes CSG ID of cell B).

Preamble:

-	the UE is in state Registered, Idle mode (state 2) with condition CombinedAttach on cell A according to TS 36.508 [18]

9.2.3.2.16.3.2	Test procedure sequence

Table 9.2.3.2.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as a "Not Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = "Not authorized for this CSG". | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message on Cell B in the next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 6 | The SS configures: - Cell A as a "Not Suitable off cell". - Cell B as a "Not Suitable cell". - Cell C as a "Suitable cell". | - | - | - | - |
| 7 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message on Cell C? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 9A | The SS releases the RRC connection. | - | - | - | - |
| 10 | The SS configures: - Cell A as a "Not Suitable off cell". - Cell B as a " Serving cell". - Cell C as a " Not Suitable cell". | - | - | - | - |
| 11 | Check: Does the UE transmit an TRACKING AREA UPDATE REQUEST message in the next 30 seconds on Cell B? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |

9.2.3.2.16.3.3	Specific message contents

Table 9.2.3.2.16.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.2.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | ‘001’B | "combined TA/LA updating" |  |

Table 9.2.3.2.16.3.3-2: Message TRACKING AREA UPDATE REJECT (step 3, Table 9.2.3.2.16.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '00011001'B | #25 " Not authorized for this CSG" |  |

Table 9.2.3.2.16.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7, Table 9.2.3.2.16.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | ‘010’B | " combined TA/LA updating with IMSI attach " |  |

Table 9.2.3.2.16.3.3-4: SystemInformationBlockType1 for Cell A, B, C (Pre-test conditions and all steps in Table 9.2.3.2.16.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell B |
|  | FALSE |  | Cell A |
|  | FALSE |  | Cell C |
| csg-Identity | Not present |  | Cell A |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell B |
|  | Not present |  | Cell C |
| } |  |  |  |
| } |  |  |  |

##### 9.2.3.2.17	Combined tracking area update / Abnormal case / handling of the EPS tracking area updating attempt counter

9.2.3.2.17.1	Test Purpose (TP)

(1)

with { UE has initiated combined tracking area updating procedure and has the tracking area updating attempt counter less than 5 }

ensure that {
  when { UE detects release of the NAS signalling connection and in a EPS update status different to EU1 UPDATED and update status is U1 UPDATED }

then { UE starts timer T3411, does not delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED }

}

(2)

with { UE has initiated combined tracking area updating procedure and has the tracking area updating attempt counter equal to 5 }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE starts timer T3402, deletes any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED }

}

9.2.3.2.17.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.1, clause 5.5.3.2.6 and clause 5.5.3.3.6.

[TS 24.301, clause 5.5.3.1]

A tracking area updating attempt counter is used to limit the number of subsequently rejected tracking area update attempts. The tracking area updating attempt counter shall be incremented as specified in subclause 5.5.3.2.6. Depending on the value of the tracking area updating attempt counter, specific actions shall be performed. The tracking area updating attempt counter shall be reset when:

-	an attach or combined attach procedure is successfully completed;

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is successfully completed; or

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is rejected with EMM cause #11, #12, #13, #14, #15 or #25.

Additionally the tracking area updating attempt counter shall be reset when the UE is in substate EMM-REGISTERED.ATTEMPTING-TO-UPDATE or EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, and:

-	a new tracking area is entered; or

-	timer T3402 expires.

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

…

c)	T3430 timeout

The UE shall abort the procedure and proceed as described below. The NAS signalling connection shall be released locally.

…

For the cases b, c and d the UE shall proceed as follows:

Timer T3430 shall be stopped if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is included in the TAI list and the EPS update status is equal to EU1 UPDATED:

-	the UE shall keep the EPS update status to EU1 UPDATED and enter state EMM-REGISTERED.NORMAL-SERVICE. The UE shall start timer T3411. When timer T3411 expires the tracking area updating procedure is triggered again.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is not included in the TAI list or the EPS update status is different to EU1 UPDATED:

-	the UE shall start timer T3411, shall set the EPS update status to EU2 NOT UPDATED and change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE. When timer T3411 expires the tracking area updating procedure is triggered again.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GPRS update status as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal or periodic routing area updating procedure fails and the routing area updating attempt counter is less than 5 and the GPRS update status is different from GU1 UPDATED.

If the tracking area updating attempt counter is equal to 5:

-	the UE shall start timer T3402, shall set the EPS update status to EU2 NOT UPDATED, shall delete the list of equivalent PLMNs and shall change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE or optionally to EMM-REGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GPRS update status as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal or periodic routing area updating procedure fails and the routing area updating attempt counter is equal to 5.

[TS 24.301, clause 5.5.3.3.6]

If the tracking area updating attempt counter is incremented according to subclause 5.5.3.2.6 the next actions depend on the value of the tracking area updating attempt counter.

-	if the update status is U1 UPDATED and the tracking area updating attempt counter is less than 5, then the UE shall keep the update status to U1 UPDATED, the new MM state is MM IDLE substate NORMAL SERVICE;

-	if the tracking area updating attempt counter is less than 5 and, additionally, the update status is different from U1 UPDATED UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. The MM state remains MM LOCATION UPDATING PENDING; or

-	if the tracking area updating attempt counter is equal to 5, the UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. A UE operating in CS/PS mode 1 of operation shall select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures.

9.2.3.2.17.3	Test description

9.2.3.2.17.3.1	Pre-test conditions

System Simulator:

- cell A and cell B.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE is configured to initiate combined attach in DATA Centric mode

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18] with the exception of the ATTACH ACCEPT message in which the T3402 is explicitly set to 1 min.

9.2.3.2.17.3.2	Test procedure sequence

Table 9.2.3.2.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell B. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | Wait for 25s to ensure that T3430 and T3411 expire. NOTE 1: The tracking area updating attempt counter is 1. | - | - | - | - |
| 4 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 5 | Wait for 25s to ensure that T3430 and T3411 expire. NOTE 2: The tracking area updating attempt counter is 2. | - | - | - | - |
| 6 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 7 | Wait for 25s to ensure that T3430 and T3411 expire. NOTE 3: The tracking area updating attempt counter is 3. | - | - | - | - |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 9 | Wait for 25s to ensure that T3430 and T3411 expire. NOTE 4: The tracking area updating attempt counter is 4. | - | - | - | - |
| 10 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 11 | The SS releases the RRC connection. NOTE 5: The tracking area updating attempt counter is 5 and reset. | - | - | - | - |
| 12 | Wait for 12 min to ensure that T3402 expires. | - | - | - | - |
| 13 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 14 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.2.17.3.3	Specific message contents

Table 9.2.3.2.17.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, step 4, step 6, step 8, step 10, Table 9.2.3.2.17.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | "combined TA/LA updating" |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |
| Old LAI | LAI-1 |  |  |
| TMSI Status | Not Present |  |  |

Table 9.2.3.2.17.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 13, Table 9.2.3.2.17.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | "combined TA/LA updating" |  |
| Old GUTI | GUTI-1 |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | 0 | no valid TMSI available |  |

Table 9.2.3.2.17.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 14, Table 9.2.3.2.17.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |

#### 9.2.3.3	Iu mode to S1 mode inter-system change in idle mode

##### 9.2.3.3.1	First Iu mode to S1 mode inter-system change after attach

9.2.3.3.1.1	Test Purpose (TP)

(1)

with { the UE is powered on in UTRAN with a USIM containing valid EPS identities and EPS security context and has attached in UTRAN and activated a PDP context  }

ensure that {

when { the UE reselects a E-UTRAN cell }

then { the UE  transmits a TRACKING AREA UPDATE REQUEST message including eKSI stored in the USIM, old GUTI mapped from RAI and P-TMSI, GPRS ciphering sequence number, nonceUE and the UE radio capability information update needed IE, the message is integrity protected using the EPS security context stored in the USIM and encapsulated in an RRCConnectionSetupComplete message with registeredMME set to the MME part of the mapped GUTI }

(2)

with { the UE has performed a TAU procedure in EUTRAN after initial registration at power on in UTRAN  }

ensure that {

when { the UE reselects a UTRAN cell }

then { the UE transmits a ROUTING AREA UPDATE REQUEST message including P-TMSI and P-TMSI signature mapped from GUTI, previously allocated P-TMSI as additional mobile identity, RAI where P-TMSI was allocated as additional old routing area identification, eKSI allocated in UTRAN as GPRS ciphering key sequence number, the message is encapsulated in an INITIAL DIRECT TRANSFER message including IDNSS mapped from P-TMSI }

(3)

with { the UE has performed a TAU procedure in EUTRAN after initial registration at power on in UTRAN and has transmitted a ROUTING AREA UPDATE REQUEST message and received an AUTHENTICATION AND CIPHERING REQUSET message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message }

then { the UE transmits a SECURITY MODE COMPLETE and starts performing ciphering an integrity protection using new CK and IK }

(4)

with { the UE has performed initial registration at power on in UTRAN and has EPS security context }

ensure that {

when { UE performs a TAU procedure in E-UTRAN }

then { the UE starts performing ciphering an integrity protection using EPS security context }

NOTE:	ISR considered not in the scope of this test case, so it is never activated.

9.2.3.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.003 clause 2.8.2.1, 2.8.2.2, TS 23.401 clauses 5.3.1.1, 5.3.3.1, 5.3.3.3 and Annex D; TS 25.331 clauses 8.1.8.2 and 10.3.1.6 (on IDNNS); TS 24.008 [NAS message contents]; TS 24.301 clause 5.5.3.2.2 and TS 24.301 V12.2.0 clause 5.5.3.3.2; and TS 36.331 clauses 5.3.3.3 and 5.3.3.4. Unless otherwise stated these are Rel-8 requirements.

[TS 23.003, clause 2.8.2.1]

The mapping of the GUTI shall be done to the combination of RAI of GERAN / UTRAN and the P-TMSI:

E-UTRAN <MCC> maps to GERAN/UTRAN <MCC>

E-UTRAN <MNC> maps to GERAN/UTRAN <MNC>

E-UTRAN <MME Group ID> maps to GERAN/UTRAN <LAC>

E-UTRAN <MME Code> maps to GERAN/UTRAN <RAC> and is also copied into the 8 most significant bits of the NRI field within the P-TMSI;

E-UTRAN <M-TMSI>			maps as follows:

-	6 bits of the E-UTRAN <M-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the GERAN/UTRAN <P-TMSI>;

-	16 bits of the E-UTRAN <M-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the GERAN/UTRAN <P-TMSI>;

-	and the remaining 8 bits of the E-UTRAN <M-TMSI> are mapped into the 8 MBS bits of the <P-TMSI signature> field.

For UTRAN, the 10-bit long NRI bits are masked out from the P-TMSI and also supplied to the RAN node as IDNNS (Intra Domain NAS Node Selector). However, the RAN configured NRI length should not exceed 8 bits.

[TS 23.003, clause 2.8.2.2]

The mapping of P-TMSI (TLLI) and RAI in GERAN/UTRAN to GUTI in E-UTRAN shall be performed as follows:

GERAN/UTRAN <MCC>	maps to E-UTRAN <MCC>

GERAN/UTRAN <MNC>	maps to E-UTRAN <MNC>

GERAN/UTRAN <LAC>	maps to E-UTRAN <MME Group ID>

GERAN/UTRAN <RAC>	maps into bit 23 and down to bit 16 of the M-TMSI

The 8 most significant bits of GERAN/UTRAN <NRI> map to the MME code.

GERAN/UTRAN <P-TMSI> maps as follows:

-	6 bits of the GERAN/UTRAN <P-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the E-UTRAN <M-TMSI>;

-	16 bits of the GERAN/UTRAN <P-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the E-UTRAN <M-TMSI>.

The values of <LAC> and <MME group id> shall be disjoint, so that they can be differentiated. The most significant bit of the <LAC> shall be set to zero; and the most significant bit of <MME group id> shall be set to one. Based on this definition, the most significant bit of the <MME group id> can be used to distinguish the node type, i.e. whether it is an MME or SGSN.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

...

[TS 23.401, clause 5.3.3.1, step 2, “Tracking Area Update procedure with Serving GW change”]

...

If the UE's TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and related RAI then these two elements are indicated as the old GUTI.

...

[TS 23.401, clause 5.3.3.3, step 2, “Routeing Area Update with MME interaction and without S-GW change”]

...

If the UE's internal TIN indicates "GUTI" and the UE holds a valid GUTI then the UE indicates the GUTI as the old P-TMSI and old RAI

...

[TS 24.301, clause 5.5.3.2.2 “Normal and periodic tracking area updating procedure initiation”]

...

If the UE supports A/Gb mode or Iu mode, the UE shall handle the GUTI as follows:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	Mapping the P-TMSI and RAI to the GUTI is specified in Annex H of 3GPP TS 23.401 [10].

-	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

...

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

...

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode and the TIN is set to "P-TMSI", the UE shall include the GPRS ciphering key sequence number applicable for A/Gb mode or Iu mode and a nonceUE in the TRACKING AREA UPDATE REQUEST message.

…

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

[Rel-12]

[TS 24.301, clause 5.5.3.3.2]

The UE operating in CS/PS mode 1 or CS/PS mode 2, in state EMM-REGISTERED, shall initiate the combined tracking area updating procedure:

…

d)	when the UE performs an intersystem change from A/Gb or Iu mode to S1 mode, and the UE previously either performed a combined GPRS attach procedure, an IMSI attach procedure, a location area updating procedure or a combined routing area updating procedure, in A/Gb or Iu mode, or moved to A/Gb or Iu mode from S1 mode through an SRVCC handover or moved to Iu mode from S1 mode through an vSRVCC handover. In this case the EPS update type IE shall be set to "combined TA/LA updating with IMSI attach";

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| Tracking Area Update | MO signalling (See Note 1) | "originating signalling" |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

[TS 36.331, clause 5.3.3.4]

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS  to include the information received from upper layers;

...

[TS 24.008, clause 4.7.5]

This procedure is used for:

...

-	S1 mode to Iu mode or S1 mode to A/Gb mode intersystem change and ISR is not activated;

...

[TS 24.008, clause 4.7.5.1.1]

To initiate the normal routing area updating procedure, the MS sends the message ROUTING AREA UPDATE REQUEST to the network, starts timer T3330 and changes to state GMM-ROUTING-AREA-UPDATING-INITIATED.

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

...

If the routing area updating procedure is initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode and the TIN indicates "GUTI", the MS shall derive a UMTS security context for the PS domain from the current EPS security context as described in the subclause 4.7.7.10. The ROUTING AREA UPDATE REQUEST message shall include a P-TMSI signature filled with a NAS token as specified in 3GPP TS 33.401 [119]. Furthermore, the MS shall indicate the eKSI value, which is associated with the derived UMTS security keys, in the CKSN field of the GPRS GSM ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message.

NOTE:	When the MS includes a P-TMSI signature filled with a NAS token, 8 bits of the NAS token will be filled with bits from the M-TMSI (see 3GPP TS 23.003 [4]).

If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in connected mode, the MS shall derive a UMTS security context for the PS domain from the current EPS security context as described in the subclause 4.7.7.10. Furthermore, the MS shall indicate the eKSI value, which is associated with the derived UMTS security keys, in the CKSN field of the GPRS GSM ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message.

[TS 25.331, clause 8.1.8.2]

...

The UE shall, in the INITIAL DIRECT TRANSFER message:

1>	set the IE "NAS message" as received from upper layers; and

1>	set the IE "CN domain identity" as indicated by the upper layers; and

1>	set the IE "Intra Domain NAS Node Selector" as follows:

2>	derive the IE "Intra Domain NAS Node Selector" from TMSI/PMTSI, IMSI, or IMEI; and

2>	provide the coding of the IE "Intra Domain NAS Node Selector" according to the following priorities:

1.	derive the routing parameter for IDNNS from TMSI (CS domain) or PTMSI (PS domain) whenever a valid TMSI/PTMSI is available;

2.	base the routing parameter for IDNNS on IMSI when no valid TMSI/PTMSI is available;

3.	base the routing parameter for IDNNS on IMEI only if no (U)SIM is inserted in the UE.

1>	if the UE, on the existing RRC connection, has received a dedicated RRC message containing the IE "Primary PLMN Identity" in the IE "CN Information Info":

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the latest PLMN information received via dedicated RRC signalling. If NAS has indicated the PLMN towards which a signalling connection is requested, and this PLMN is not in agreement with the latest PLMN information received via dedicated RRC signalling, then the initial direct transfer procedure shall be aborted, and NAS shall be informed.

1>	if the UE, on the existing RRC connection, has not received a dedicated RRC message containing the IE "CN Information Info" , and if the IE "Multiple PLMN List" was broadcast in the cell where the current RRC connection was established:

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the PLMN chosen by higher layers [5, 25] amongst the PLMNs in the IE "Multiple PLMN List" broadcast in the cell where the RRC connection was established.

1>	if the IE "Activated service list" within variable MBMS_ACTIVATED_SERVICES includes one or more MBMS services with the IE "Service type" set to "Multicast" and;

1>	if the IE "CN domain identity" as indicated by the upper layers is set to "CS domain" and;

1>	if the variable ESTABLISHED_SIGNALLING_CONNECTIONS does not include the CN domain identity 'PS domain':

2>	include the IE "MBMS joined information";

2>	include the IE "P-TMSI" within the IE "MBMS joined information" if a valid PTMSI is available.

1>	if the UE is in CELL_FACH state and the IE "CN domain identity" as indicated by the upper layers is set to "CS domain":

2>	if the value of the variable ESTABLISHMENT_CAUSE is set to "Originating Conversational Call" or "Emergency Call":

3>	set the value of the IE "Call type" to "speech", "video" or "other" according to the call being initiated.

1>	if the variable ESTABLISHMENT_CAUSE is initialised:

2>	set the IE "Establishment cause" to the value of the variable ESTABLISHMENT_CAUSE;

2>	clear the variable ESTABLISHMENT_CAUSE.

1>	calculate the START according to subclause 8.5.9 for the CN domain as set in the IE "CN Domain Identity"; and

1>	include the calculated START value for that CN domain in the IE "START".

9.2.3.3.1.3	Test description

9.2.3.3.1.3.1	Pre-test conditions

System Simulator:

-	cell A, cell B and cell 5 (UTRA cell in LAI-1/RAI-1);

-	cell 5 is configured as Serving cell, cell A as Non-Suitable cell.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	For Cell 5, power levels are defined in TS 34.108 subclause 6.1.5 (FDD) or 6.1.6 (TDD).

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell B using default message contents according to TS 36.508 [18].

NOTE:	Cell B belongs to TAI-2 (unlike cell A).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.3.1.3.2	Test procedure sequence

Table 9.2.3.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message on Cell 5 | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8 | SS responds with ATTACH ACCEPT message including P-TMSI-1 and RAI-1. | <-- | ATTACH ACCEPT | - | - |
| 9 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 9A | The SS releases the RRC connection. | - | - | - | - |
| 9B | Generic test procedure in TS 36.508 subclause 4.5A.3A.2 is performed | - | - | - | - |
| 10 | The activation of a PDP context is triggered by MMI or AT command. | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | The SS establishes a radio bearer associated with the requested PDP context. | - | - | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Cell A is configured as the Serving Cell, Cell 5 is configured as a Suitable Neighbour Cell. | - | - | - | - |
| 15 | Check: Does the UE transmit an RRCConnectionRequest with the InitialUE-Identity set to “randomValue” and the establishmentcause set to MO-signalling? | - | - | 1 | P |
| 16 | The SS responds with RRCConnectionSetup. | - | - |  |  |
| 17 | Check: Does the UE transmit an RRCConnectionSetupComplete message with the mmegi and mmec are set to the values derived from the mapped RAI and P-TMSI, and containing a TRACKING AREA UPDATE REQUEST message as described in the specific message contents? Check2: Does the UE transmit a TRACKING AREA UPDATE REQUEST message integrity protected using IK derived from KASME? | --> | TRACKING AREA UPDATE REQUEST | 1, 4 | P |
| 18 | The SS responds with a TRACKING AREA UPDATE ACCEPT allocating a new GUTI and TAI list. This message is ciphered and integrity protected using CK and IK derived from KASME. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 18A | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message ciphered and integrity protected using CK and IK derived from KASME. | --> | TRACKING AREA UPDATE COMPLETE | 4 | P |
| 19 | The SS releases the RRC connection | - | - | - | - |
| - | Exception: Steps 19A to 19Aa5/19Ab1 describe optional behaviour that depend on the UE capability. | - | - | - |  |
| 19A | If pc_IMS = TRUE SS starts 10s timer | - | - | - |  |
| 19Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | - | - | - |  |
| 19Aa2 | The SS establishes SRB2 and DRB | - | - | - |  |
| 19Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - |  |
| 19Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - |  |
| 19Aa5 | The SS releases the RRC connection | - | - | - |  |
| 19Ab1 | The 10s timer expires | - | - | - |  |
| 20 | Void | - | - | - | - |
| 21 | Cell 5 is configured as the Serving Cell and Cell A is configured as a Non-Suitable Cell as defined in table 6.2.2.1-1 of TS 36.508 [18]. | - | - | - | - |
| 22 | Void | - | - | - | - |
| 23 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message as specified in the specific message contents included in an INITIAL DIRECTE TRANSFER message with intraDomainNasNodeSelector mapped from GUTI allocated in step 18? | --> | ROUTING AREA UPDATE REQUEST | 2 | P |
| 23AA | The SS sends an AUTHENTICATION AND CIPHERING REQUEST message to perform a UMTS AKA procedure. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 23AB | The UE sends an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 23A | The SS sends a SECURITY MODE COMMAND message to activate integrity protection and ciphering. This message is integrity protected using new IK. | - | - | - | - |
| 23B | Check: Does the UE transmit a SECURITY MODE COMPLETE message integrity protected using new IK? | - | - | 3 | P |
| 24 | The SS transmits a ROUTING AREA UPDATE ACCEPT message (no new P-TMSI nor RAI). This message is ciphered and integrity protected using CK and IK derived at step 23AA. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 25 | After the activation time indicated at step 23A, the SS transmits a UE CAPABILITY ENQUIRY message requesting the UE UTRAN capability. This message is ciphered and integrity protected using CK and IK derived at step 23AA. | - | - | - | - |
| 26 | Check: Does the UE transmit a UE CAPABILITY INFORMATION message? Note: the purpose of this message is to show that the UE uses CK and IK derived at step 23AA, and not the CK and IK allocated at step 4. | - | - | 3 | P |
| 27 | The SS transmits a UE CAPABILITY INFORMATION CONFIRM message. | - | - | - | - |
| 28 | The SS releases the RRC connection | - | - | - | - |

9.2.3.3.1.3.3	Specific message contents

Table 9.2.3.3.1.3.3-1: Message RRCConnectionRequest (step 15, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| random-Value | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.1.3.3-2: Message RRCConnectionSetupComplete (step 17, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| registeredMME { |  |  |  |
| plmn-Identity | Not present |  |  |
| mmegi | LAC sent to the UE in step 8 |  |  |
| mmec | Bit 23 to bit 16 of P-TMSI sent to the UE in step 8 |  |  |
| } |  |  |  |
| dedicatedInfoNAS | See table 9.2.3.3.1.3.3-3 |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Details to be added |  |  |  |

Table 9.2.3.3.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 17, Table 9.2.3.3.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 with condition UNCIPHERED (the message is integrity protected using the key stored in the USIM). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 or 001 or 010 | 'TA updating' or 'combined TA/LA updating' or ‘combined TA/LA updating with IMSI attach’ | Rel-8 to Rel-11 only |
|  | 000 or 010 | 'TA updating' or ‘combined TA/LA updating with IMSI attach’ | Rel-12 and later releases |
| NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier stored in the USIM |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Old GUTI or IMSI |  |  |  |
| Type of identity | 110 | GUTI |  |
| MNC/MCC | Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM |  |  |
| MME Group ID | LAI allocated at step 8 |  |  |
| MME Code | Bit 23 to bit 16 of P-TMSI allocated at step 8 |  |  |
| M-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: 1 - bit 29 to bit 24: bit 29 to bit 24 of P-TMSI allocated at step 8 - bit 23 to bit 16: RAC allocated at step 8 - bit 15 to bit 0: bit 15 to bit 0 of P-TMSI allocated at step 8 |  |  |
| GPRS ciphering key sequence number | GPRS ciphering key sequence number allocated at step 3 |  |  |
| Old P-TMSI signature | P-TMSI signature allocated at step 8 |  |  |
| Additional GUTI | GUTI-1 |  |  |
| NonceUE | Any allowed value (must be present) |  |  |
| UE network capability | Any allowed value (must be present) |  |  |
| Last visited registered TAI | TAI stored in the test USIM |  |  |
| UE radio capability information update needed | 1 | UE radio capability information update needed |  |
| MS network capability | Any allowed value (must be present) |  |  |
| Old location area identification | Not present if "EPS update type" is 'TA updating', LAI-1 if "EPS update type" is 'combined TA/LA updating' |  |  |
| TMSI status | Not present |  |  |

Table 9.2.3.3.1.3.3-4: INITIAL DIRECT TRANSFER (step 23, Table 9.2.3.3.1.3.2-1)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialDirectTransfer ::= SEQUENCE { |  |  |  |
| cn-DomainIdentity | ps-domain |  |  |
| intraDomainNasNodeSelector SEQUENCE { |  |  |  |
| version CHOICE { |  |  |  |
| release99 SEQUENCE { |  |  |  |
| gsm-Map-IDNNS SEQUENCE { |  |  |  |
| routingbasis CHOICE { |  |  |  |
| tMSIofsamePLMN { |  |  |  |
| routingparameter | bit 23 to bit 14 of P-TMSI mapped from GUTI allocated to UE in step 18 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nas-Message | See table 9.2.3.3.1.3.3-5 |  |  |
| } |  |  |  |

Table 9.2.3.3.1.3.3-5: Message ROUTING AREA UPDATE REQUEST (step 23, Table 9.2.3.3.1.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 000 or 001 | RA updating or Combined RA/LA updating |  |
| GPRS ciphering key sequence number | eKSI stored on the test USIM |  |  |
| Old routing area identification | GUTI-1 right shifted by 32bits |  |  |
| MS Radio Access capability | Not checked |  |  |
| Old P-TMSI signature | Value is specified bit by bit below: - bit 23 to bit 16: bit 23 to bit 16 of M-TMSI allocated in step 18 - bit 15 to bit 0: Truncated NAS-token |  |  |
| Requested READY timer value | Not checked |  |  |
| DRX parameter | Not checked |  |  |
| TMSI status | Not present |  |  |
| P-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: 1 - bit 29 to bit 24: bit 29 to bit 24 of M-TMSI allocated in step 18 - bit 23 to bit 16: MME code allocated in step 18 - bit 15 to bit 0: bit 15 to bit 0 of M-TMSI allocated in step 18 |  |  |
| MS network capability | Not checked |  |  |
| PDP context status | Not checked |  |  |
| PS LCS Capability | Not checked |  |  |
| MBMS context status | Not checked |  |  |
| UE network capability | Any allowed value |  |  |
| Additional mobile identity | P-TMSI allocated in step 8 |  |  |
| Additional old routing area identification | RAI-1 |  |  |
| Mobile station classmark 2 | Not checked |  |  |
| Mobile station classmark 3 | Not checked |  |  |
| Supported Codecs | Not checked |  |  |

##### 9.2.3.3.2	Iu mode to S1 mode intersystem change / ISR is active / Expiry of T3312 in E-UTRAN or T3412 in UTRAN and further intersystem change

9.2.3.3.2.1	Test Purpose (TP)

(1)

with { the UE is camped on a E-UTRAN cell, ISR is active, T3312 has expired, T3323 hasn't expired }

ensure that {

when { UE enters a UTRAN cell belonging to the RA where the UE was last updated }

then { the UE performs a RAU procedure including the UE P-TMSI and RAI }

}

(2)

with { the UE is camped on a E-UTRAN cell, ISR is active, T3312 and T3323 have expired }

ensure that {

when { UE enters a UTRAN cell belonging to the RA where the UE was last updated }

then { the UE performs a RAU procedure including mapped identities from E-UTRAN and additional P-TMSI and old RAI }

}

(3)

with { the UE operating in PS mode 1 or PS mode 2, is camped on a UTRAN cell, ISR is active, T3412 has expired, T3423 hasn't expired }

ensure that {

when { UE enters a E-UTRAN cell in a TA belonging to the current TA list }

then { the UE performs a TAU procedure including the UE GUTI and last visited TAI}

}

(4)

with { the UE operating in PS mode 1 or PS mode 2, is camped on a UTRAN cell, ISR is active, T3412 and T3423 have expired }

ensure that {

when { the UE enters a E-UTRAN cell in a TA belonging to the current TA list }

then { the UE performs a TAU procedure including mapped GUTI from UTRAN, P-TMSI signature and additional GUTI}

}

9.2.3.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clauses 5.3.5 and 5.5.3.2.2 and TS 24.008 clauses 4.7.2.2 and 4.7.5.1.1.

[24.301, clause 5.3.5]

If ISR is activated, the UE shall keep both the periodic tracking area update timer (timer T3412) and the periodic routeing area update timer (timer T3312). The two separate timers run in the UE for updating MME and SGSN independently. If the periodic tracking area update timer expires and the UE cannot initiate the tracking area updating procedure, as it is in state EMM-REGISTERED.NO-CELL-AVAILABLE, the UE shall start the E-UTRAN deactivate ISR timer T3423. The UE shall initiate the tracking area updating procedure and stop the timer T3423 when it enters state EMM-REGISTERED.NORMAL-SERVICE before timer T3423 expires. After expiry of timer T3423 the UE shall set its TIN to "P-TMSI" in order to initiate the tracking area updating procedure when it returns to state EMM-REGISTERED.NORMAL-SERVICE.

If the UE is attached to both EPS and non-EPS services, and if timer T3412 expires or timer T3423 expires when the UE is in EMM-REGISTERED.NO-CELL-AVAILABLE state, then the UE shall initiate the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" when the UE returns to EMM-REGISTERED.NORMAL-SERVICE state.

[24.301, clause 5.5.3.2.2]

If the UE supports neither A/Gb mode nor Iu mode, the UE shall include a valid GUTI in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

[24.008, clause 4.7.2.2]

If ISR is activated, the MS shall keep both the periodic tracking area update timer (timer T3412) and the periodic routeing area update timer (timer T3312). The two separate timers run in the MS for updating MME and SGSN independently. If the periodic routeing area update timer expires and the MS is in state GMM-REGISTERED.NO-CELL-AVAILABLE, the MS shall set its TIN to "RAT-related TMSI" and start the GERAN/UTRAN Deactivate ISR timer T3323. The MS shall initiate the routeing area updating procedure and stop the timer T3323 when the MS enters the state GMM-REGISTERED.NORMAL-SERVICE before timer T3323 expires. After expiry of timer T3323 the MS shall deactivate ISR by setting its TIN to "GUTI" and initiate the routing area updating procedure when the MS enters the state GMM-REGISTERED.NORMAL-SERVICE.

...

If the MS is both IMSI attached for GPRS and non-GPRS services, and if the MS lost coverage of the registered PLMN and timer T3312 expires or timer T3323 expires, then:

a)	if the MS returns to coverage in a cell that supports GPRS and that indicates that the network is in network operation mode I, then the MS shall either perform the combined routing area update procedure indicating "combined RA/LA updating with IMSI attach"; or

b)	if the MS returns to coverage in a cell in the same RA that supports GPRS and that indicates that the network is in network operation mode II or III, then the MS shall perform the periodic routing area updating procedure indicating "Periodic updating"; or

c)	if the MS was both IMSI attached for GPRS and non-GPRS services in network operation mode I and the MS returns to coverage in a cell in the same LA that does not support GPRS, then the MS shall perform the periodic location updating procedure. In addition, the MS shall perform a combined routing area update procedure indicating "combined RA/LA updating with IMSI attach" when the MS enters a cell that supports GPRS and that indicates that the network is in network operation mode I; or

d)	if the MS returns to coverage in a new RA the description given in subclause 4.7.5 applies.

If the MS is both IMSI attached for GPRS and non-GPRS services in a network that operates in network operation mode I, and if the MS has camped on a cell that does not support GPRS, and timer T3312 expires or timer T3323 expires, then the MS shall start an MM location updating procedure. In addition, the MS shall perform a combined routing area update procedure indicating "combined RA/LA updating with IMSI attach" when the MS enters a cell that supports GPRS and indicates that the network is in operation mode I.

If timer T3312 expires or timer T3323 expires during an ongoing CS connection, then a MS operating in MS operation mode B shall treat the expiry of T3312 when the MM state MM-IDLE is entered, analogous to the descriptions for the cases when the timer expires out of coverage or in a cell that does not support GPRS.

In A/Gb mode, timer T3312 and timer T3323 shall not be stopped when a GPRS MS enters state GMM-REGISTERED.SUSPENDED.

[24.008, clause 4.7.5.1.1]

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

-	If the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and RAI, the MS shall indicate the RAI in the Old routing area identification IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the P-TMSI in the P-TMSI IE.

9.2.3.3.2.3	Test description

9.2.3.3.2.3.1	Pre-test conditions

System Simulator:

-	cell A and cell 5 (HPLMN);

-	cell 5 indicates NMO I.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18] and with M-TMSI = 9a26319c (arbitrary value chosen for this test case).

9.2.3.3.2.3.2	Test procedure sequence

Table 9.2.3.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 2 | The UE transmits a ROUTING AREA UPDATE message on cell 5. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5 with T3312 set to 1 minute and T3323 set to 2 minutes. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a ROUTING AREA UPDATE COMPLETE message on Cell 5. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 4A | The SS releases the RRC connection. | - | - | - | - |
| - | Exception: Steps 4Ba1 to 4Ba1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 4Ba1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 4Ba1a1- 4Ba1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 4Ba1a5 | SS releases RRC Connection | - | - | - | - |
| 4Ba1a6 | SS Stops 6s timer | - | - | - | - |
| 4Ba1b1 | The 6s timer expires | - | - | - | - |
| 5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 5Aa1 to 5Aa4 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional TAU message. | - | - | - | - |
| 5Aa1 | The UE may transmit a TRACKING AREA UPDATE REQUEST message on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 5Aa2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message on Cell A. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 5Aa3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | Exception: Steps 5Aa4 to 5Aa5b1 describe optional behaviour if UE sends optional IMS deregistration at steps 4Ba1 to 4Ba1b1 | - | - | - | - |
| 5Aa4-5AAa5b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 5Aa4 | The SS releases the RRC connection. | - | - | - | - |
| 6 | 90s after step 3, the SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 7 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5 in the time period between expiry of T3312 – started upon RRC Connection Release after step 4A – and before expiry of T3323 – which was started when T3312 expired? | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| 8 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5 with T3312 set to 1 minute and T3323 set to 1 minute. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8A | The SS releases the RRC connection. | - | - | - | - |
| - | Exception: Steps 8Ba1 to 8Ba1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 8Ba1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 8Ba1a1- 8Ba1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 8Ba1a5 | SS releases RRC Connection | - | - | - | - |
| 8Ba1a6 | SS Stops 6s timer | - | - | - | - |
| 8Ba1b1 | The 6s timer expires | - | - | - | - |
| 9 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 9Aa1 to 9Aa4 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional TAU message. | - | - | - | - |
| 9Aa1 | The UE may transmit a TRACKING AREA UPDATE REQUEST message on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 9Aa2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message on Cell A. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9Aa3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Steps 9Aa4 to 9Aa5b1 describe optional behaviour if UE sends IMS deregistration at steps 8Ba1 to 8Ba1b1 | - | - | - | - |
| 9Aa4-9Aa5b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 9Aa4 | The SS releases the RRC connection. | - | - | - | - |
| 10 | 150s after step 8, the SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 11 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5 after both timers expired, T3312 which was started after step 8A and T3323 which was started upon expiry of T3312? | --> | ROUTING AREA UPDATE REQUEST | 2 | P |
| 12 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 12A | The SS releases the RRC connection. | - | - | - | - |
| - | Exception: Steps 12AAa1 to 12AAa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 12AAa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 8AAa1a1- 12AAa1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 12AAa1a5 | SS releases RRC Connection | - | - | - | - |
| 12AAa1a6 | SS Stops 6s timer | - | - | - | - |
| 12AAa1b1 | The 6s timer expires | - | - | - | - |
| 13 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 14 | The UE transmits a TRACKING AREA UPDATE REQUEST message | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 15 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating that ISR is active, T3412 set to 1 minute and T3423 set to 2 minutes. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 16 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | Exception: Steps 16AAa1 to 16AAa2b1 describe optional behaviour if UE sends IMS deregistration at steps 12AAa1 to 12AAa1b1. | - | - | - | - |
| 16AAa1-16AAa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| - | EXCEPTION: Steps 16A to 24 describe behaviour that is performed if PS mode 1 or PS mode 2 of operation is configured on the UE. | - | - | - | - |
| 16A | The SS releases the RRC connection. | - | - | - | - |
| 17 | The SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 17Aa1 | Void | - | - | - | - |
| 17Aa2 | Void | - | - | - | - |
| 18 | 90s after step 16, the SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 19 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell A in the time period between expiry of T3412 – started upon RRC Connection Release after step 16A – and before expiry of T3423 – which was started when T3412 expired? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 20 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating that ISR is active, T3412 set to 1 minute and T3423 set to 1 minute. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20A | The SS releases the RRC connection. | - | - | - | - |
| 21 | The SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 21Aa1 | Void | - | - | - | - |
| 21Aa2 | Void | - | - | - | - |
| 22 | 150s after step 20, the SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 23 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell A after both timers expired, T3412 which was started after step 20A and T3423 which was started upon expiry of T3412? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 24 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.3.2.3.3	Specific message contents

Table 9.2.3.3.2.3.3-1: Message ROUTING AREA UPDATE ACCEPT (step 3, table 9.2.3.3.2.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 100 | RA updated and ISR activated |  |
|  | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 00100001 | 1 minute |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI | efb1ee97 | TMSI is an arbitrary value chosen different from M-TMSI in the preamble |  |
| T3323 value | 00100010 | 2 minutes |  |

Table 9.2.3.3.2.3.3-2: Message ROUTING AREA UPDATE REQUEST (step 7, table 9.2.3.3.2.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 011 | Periodic updating |  |
|  | 001 or 010 | Combined RA/LA updating or combined RA/LA updating with IMSI attach |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old routing area identification | RAI of Cell 5 according to 36.508 table 4.4.4-2 | The UE includes the RAI from UTRAN |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| P-TMSI | Same value like allocated at step 3, see table 9.2.3.3.2.3.3-1 | The UE includes the P-TMSI assigned from UTRAN |  |
| UE network capabilities | Not present |  | Update Type = periodic updating |
|  | Any allowed value |  | Update Type <> periodic updating |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.3.3.2.3.3-3: Message ROUTING AREA UPDATE ACCEPT (step 8, table 9.2.3.3.2.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 100 | RA updated and ISR activated |  |
|  | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 00100001 | 1 minute |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI | Not present |  |  |
| T3323 value | 00100001 | 1 minute |  |

Table 9.2.3.3.2.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 11, table 9.2.3.3.2.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 000 | RA updating |  |
|  | 010 | combined RA/LA updating with IMSI attach |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old routing area identification | TAI of cell A | The value is the same as the RAI of cell 5 |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Any allowed value |  |  |
| P-TMSI | da01319c | Mapped from M-TMSI and MME code assigned in the preamble (start with 11 and MME code in 2nd most significant byte). |  |
| Additional mobile identity | Same value like allocated at step 3, see table 9.2.3.3.2.3.3-1 |  |  |
| Additional routing area identification | RAI of cell 5 | The value is the same as the TAI of cell A |  |

Table 9.2.3.3.2.3.3-5: Message ROUTING AREA UPDATE ACCEPT (step 12, table 9.2.3.3.2.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 000 | RA updated |  |
|  | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 01010101 | 54 minutes |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI | Not present |  |  |

Table 9.2.3.3.2.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 14, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| Additional GUTI | Any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.2.3.3-7: Message TRACKING AREA UPDATE ACCEPT (step 15, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated | TA_only |
|  | 101 | combined TA/LA updated and ISR activated | combined_TA_LA |
| T3412 value | 00100001 | 1 minute |  |
| T3423 value | 00100010 | 2 minutes |  |
| GUTI |  |  |  |
| M-TMSI | 9a26319c | Same value like in preamble, different from P-TMSI |  |

Table 9.2.3.3.2.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 19, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| Old GUTI | Same GUTI as in step 15 see table 9.2.3.3.2.3.3-7 |  | TA_Only |
| Last visited registered TAI | RAI of cell 5 | The value is the same as the TAI of cell A |  |
| NonceUE | Not Present |  | TA_only |
| Additional GUTI | Not present |  | TA_only |

Table 9.2.3.3.2.3.3-9: Message TRACKING AREA UPDATE ACCEPT (step 20, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated | TA_only |
| T3412 value | 00100001 | 1 minute |  |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |
| T3423 value | 00100001 | 1 minute |  |

Table 9.2.3.3.2.3.3-10: Message TRACKING AREA UPDATE REQUEST (step 23, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| Old GUTI |  |  |  |
| M-TMSI | ef01ee97 | Mapped from P-TMSI assigned in the preamble and RAI of cell 5 (start with 11 and RAI in 2nd most significant byte). |  |
| NonceUE | Any allowed value |  |  |
| Last visited registered TAI | RAI of cell 5 |  |  |
| Additional GUTI | Same GUTI as in step 15 see table 9.2.3.3.2.3.3-7 |  |  |

Table 9.2.3.3.2.3.3-11: Message TRACKING AREA UPDATE ACCEPT (step 24, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | TA updated | TA_only |
| T3412 value | 01010101 | 54 minutes |  |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

Table 9.2.3.3.2.3.3-12: Message TRACKING AREA UPDATE REQUEST (steps 5Aa1 and 9Aa1, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS Update Type | 000 | TA updating | TA_only |
|  | 010 | combined RA/LA updating with IMSI attach | Combined TA/LA |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.2.3.3-13: Message TRACKING AREA UPDATE ACCEPT (steps 5Aa2 and 9Aa2, table 9.2.3.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated |  |
|  | 101 | combined TA/LA updated and ISR activated |  |

##### 9.2.3.3.3	Iu mode to S1 mode intersystem change / Periodic TAU and RAU/ ISR activated, T3423 expired

9.2.3.3.3.1	Test Purpose (TP)

(1)

with { the UE is camped on a E-UTRAN cell, ISR is active, T3312 has expired }

ensure that {

when { UE enters a UTRAN cell belonging to the RA where the UE was last updated }

then { the UE performs a RAU procedure }

}

(2)

with { the UE is camped on a UTRAN cell, ISR is active, T3412 and T3423 have expired }

ensure that {

when { the UE enters a E-UTRAN cell in a TA belonging to the current TA list }

then { the UE performs a TAU procedure including mapped GUTI from UTRAN, P-TMSI signature and additional GUTI}

}

9.2.3.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clauses 5.3.5 and 5.5.3.2.2 and TS 24.008 clauses 4.7.2.2 and 4.7.5.1.1.

[24.301, clause 5.3.5]

If ISR is activated, the UE shall keep both the periodic tracking area update timer (timer T3412) and the periodic routeing area update timer (timer T3312). The two separate timers run in the UE for updating MME and SGSN independently. If the periodic tracking area update timer expires and the UE cannot initiate the tracking area updating procedure, as it is in state EMM-REGISTERED.NO-CELL-AVAILABLE, the UE shall start the E-UTRAN deactivate ISR timer T3423. The UE shall initiate the tracking area updating procedure and stop the timer T3423 when it enters state EMM-REGISTERED.NORMAL-SERVICE before timer T3423 expires. After expiry of timer T3423 the UE shall set its TIN to "P-TMSI" in order to initiate the tracking area updating procedure when it returns to state EMM-REGISTERED.NORMAL-SERVICE.

If the UE is attached to both EPS and non-EPS services, and if timer T3412 expires or timer T3423 expires when the UE is in EMM-REGISTERED.NO-CELL-AVAILABLE state, then the UE shall initiate the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" when the UE returns to EMM-REGISTERED.NORMAL-SERVICE state.

[24.301, clause 5.5.3.2.2]

...

If the UE supports neither A/Gb mode nor Iu mode, the UE shall include a valid GUTI in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message.

If the UE supports A/Gb mode or Iu mode, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

[24.008, clause 4.7.2.2]

If ISR is activated, the MS shall keep both the periodic tracking area update timer (timer T3412) and the periodic routeing area update timer (timer T3312). The two separate timers run in the MS for updating MME and SGSN independently. If the periodic routeing area update timer expires and the MS is in state GMM-REGISTERED.NO-CELL-AVAILABLE, the MS shall set its TIN to "RAT-related TMSI" and start the GERAN/UTRAN Deactivate ISR timer T3323. The MS shall initiate the routeing area updating procedure and stop the timer T3323 when the MS enters the state GMM-REGISTERED.NORMAL-SERVICE before timer T3323 expires. After expiry of timer T3323 the MS shall deactivate ISR by setting its TIN to "GUTI" and initiate the routing area updating procedure when the MS enters the state GMM-REGISTERED.NORMAL-SERVICE.

...

If the MS is both IMSI attached for GPRS and non-GPRS services, and if the MS lost coverage of the registered PLMN and timer T3312 expires or timer T3323 expires, then:

a)	if the MS returns to coverage in a cell that supports GPRS and that indicates that the network is in network operation mode I, then the MS shall either perform the combined routing area update procedure indicating "combined RA/LA updating with IMSI attach"; or

b)	if the MS returns to coverage in a cell in the same RA that supports GPRS and that indicates that the network is in network operation mode II or III, then the MS shall perform the periodic routing area updating procedure indicating "Periodic updating"; or

c)	if the MS was both IMSI attached for GPRS and non-GPRS services in network operation mode I and the MS returns to coverage in a cell in the same LA that does not support GPRS, then the MS shall perform the periodic location updating procedure. In addition, the MS shall perform a combined routing area update procedure indicating "combined RA/LA updating with IMSI attach" when the MS enters a cell that supports GPRS and that indicates that the network is in network operation mode I; or

d)	if the MS returns to coverage in a new RA the description given in subclause 4.7.5 applies.

If the MS is both IMSI attached for GPRS and non-GPRS services in a network that operates in network operation mode I, and if the MS has camped on a cell that does not support GPRS, and timer T3312 expires or timer T3323 expires, then the MS shall start an MM location updating procedure. In addition, the MS shall perform a combined routing area update procedure indicating "combined RA/LA updating with IMSI attach" when the MS enters a cell that supports GPRS and indicates that the network is in operation mode I.

If timer T3312 expires or timer T3323 expires during an ongoing CS connection, then a MS operating in MS operation mode B shall treat the expiry of T3312 when the MM state MM-IDLE is entered, analogous to the descriptions for the cases when the timer expires out of coverage or in a cell that does not support GPRS.

In A/Gb mode, timer T3312 and timer T3323 shall not be stopped when a GPRS MS enters state GMM-REGISTERED.SUSPENDED.

[24.008, clause 4.7.5.1.1]

...

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

-	If the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and RAI, the MS shall indicate the RAI in the Old routing area identification IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the P-TMSI in the P-TMSI IE.

[24.008, clause 4.7.5.2.1]

The combined routing area updating procedure is initiated only by a GPRS MS operating in MS operation modes A or B, if the MS is in state GMM-REGISTERED and MM-IDLE, and if the network operates in network operation mode I:

-	when a GPRS MS that is IMSI attached for GPRS and non-GPRS services detects a change of the routing area in state GMM-REGISTERED and MM-IDLE, unless the MS is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [135] or 3GPP TS 31.102 [112] and is entering a routing area in a new PLMN that is neither the registered PLMN nor in the list of equivalent PLMNs;

…

-	when the MS which is configured to use CS fallback and SMS over SGs, or SMS over SGs only, enters a GERAN or UTRAN cell and timer T3423 has expired, or is in the GERAN or UTRAN cell when timer T3423 expires;

…

9.2.3.3.3.3	Test description

9.2.3.3.3.3.1	Pre-test conditions

System Simulator:

-	cell A and cell 5 (HPLMN);

-	cell 5 indicates NMO  I.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18] and with M-TMSI = 9a26319c (arbitrary value chosen for this test case).

9.2.3.3.3.3.2	Test procedure sequence

Table 9.2.3.3.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 2 | The UE transmits a ROUTING AREA UPDATE message on cell 5. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5 with T3312 set to1 minute and T3323 set to 2 minutes. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a ROUTING AREA UPDATE COMPLETE message on Cell 5. Note: RRC connection released | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| - | Exception: Steps 4Aa1 to 4Aa1b1 describe optional behaviour that depends on the UE capability. | - | - | - | - |
| 4Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 4Aa1a1- 4Aa1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 4Aa1a5 | SS releases RRC Connection | - | - | - | - |
| 4Aa1a6 | SS Stops 6s timer | - | - | - | - |
| 4Aa1b1 | The 6s timer expires | - | - | - | - |
| 5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 5Aa1 to 5Aa4 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional TAU message. | - | - | - | - |
| 5Aa1 | The UE may transmit a TRACKING AREA UPDATE REQUEST message on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 5Aa2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message on Cell A. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 5Aa3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
|  | EXCEPTION: Steps 5AAa1 to 5AAa2b1 describe optional behaviour if UE sends optional IMS deregistration at steps 4Aa1 to 4Aa1b1 | - | - | - | - |
| 5AAa1-5AAa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 5Aa4 | The SS releases the RRC connection. | - | - | - | - |
| 6 | 90s after step 3, the SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 7 | Check: After T3312 started after Step 4 expires and before T3323 expires, does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5? | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| 8 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5 | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 8A | The SS releases the RRC connection. |  |  |  |  |
| - | Exception: Steps 8Ba1 to 8Ba1b1 describe optional behaviour that depends on the UE capability. | - | - | - | - |
| - | Exception: Steps 8Ba1 to 8Ba1b1 describe optional behaviour that depends on the UE capability. | - | - | - | - |
| 8Ba1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 8Ba1a1- 8Ba1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 8Ba1a5 | SS releases RRC Connection | - | - | - | - |
| 8Ba1a6 | SS Stops 6s timer | - | - | - | - |
| 8Ba1b1 | The 6s timer expires | - | - | - | - |
| 9 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 10 | The UE transmits a TRACKING AREA UPDATE REQUEST message | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 11 | The SS transmits a TRACKING AREA UPDATE ACCEPT message with EPS update result indicating that ISR is active, T3412 set to 1 minute and T3423 set to 1 minute. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | Exception: Steps 11AAa1 to 11AAa1b1 describe optional behaviour if UE sends optional IMS deregistration at steps 8Ba1 to 8Ba1b1 | - | - | - | - |
| 11AAa1-11AAa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 11A | The SS releases the RRC connection. | - | - | - | - |
| 12 | The SS sets the cell type of Cell 5 to "Serving Cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 12Aa1 to 12Aa3 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional RAU message. | - | - | - | - |
| 12Aa1 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5 after T3412 expires? NOTE: A Rel-11 or earlier UEs will send the ROUTING AREA UPDATE REQUEST message before T3423 expires and Rel-12 or later UEs will send ROUTING AREA UPDATE REQUEST message after T3423 expires. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 12Aa2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5 | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 12Aa3 | SS releases RRC Connection |  |  |  |  |
| - | Exception: Steps 12Aa4 to 12Aa4b1 describe optional behaviour that depends on the UE capability. | - | - | - | - |
| 12Aa4 | IF ROUTING AREA UPDATE received at step 12Aa1 and pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 12Aa4a1- 12Aa4a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 12Aa4a5 | SS releases RRC Connection | - | - | - | - |
| 12Aa4a6 | SS Stops 6s timer | - | - | - | - |
| 12Aa4b1 | The 6s timer expires | - | - | - | - |
| 13 | 150s after step 12, the SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 14 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell A? Note: Both the timers T3412 (1 min) and T3423 (1 min, started after the expiry of T3412) have expired before this test step as there is a wait period of 150s at Step 13 | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 15 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | Exception: Steps 16a1 to 16a2b1 describe optional behaviour if UE sends optional IMS deregistration at steps 12Aa4 to 12Aa4b1. | - | - | - | - |
| 16a1-16a2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| - | The UE is in end state E-UTRA connected (E2_T3440). | - | - | - | - |

9.2.3.3.3.3.3	Specific message contents

Table 9.2.3.3.3.3.3-1: Void

Table 9.2.3.3.3.3.3-2: Message ROUTING AREA UPDATE ACCEPT (step 3, table 9.2.3.3.3.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 100 | RA updated and ISR activated |  |
|  | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 00100001 | 1 minute |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI RAI TMSI | See 36.508 table 4.4.4-2 efb1ee97 | TMSI is an arbitrary value chosen different from M-TMSI in the preamble |  |
| T3323 value | 00100010 | 2 minutes |  |

Table 9.2.3.3.3.3.3-3: Message ROUTING AREA UPDATE REQUEST (step 7 table 9.2.3.3.3.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 011 | Periodic updating |  |
|  | 010 | combined RA/LA updating with IMSI attach |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old routing area identification | RAI of Cell 5 according to 36.508 table 4.4.4-2 | The UE includes the RAI from UTRAN |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| P-TMSI | Same value like allocated at step 3, see table 9.2.3.3.3.3.3-1 | The UE includes the P-TMSI assigned from UTRAN |  |
| UE network capability | Not Present |  | Update Type = periodic updating |
|  | Any allowed value |  | Update Type <> periodic updating |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

Table 9.2.3.3.3.3.3-4: Message ROUTING AREA UPDATE ACCEPT (step 8, table 9.2.3.3.3.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 000 | RA updated |  |
|  | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 01010101 | 126 minutes |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI | Not present |  |  |

Table 9.2.3.3.3.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 10, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| Additional GUTI | Any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.3.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 11, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated | TA_only |
|  | 101 | combined TA/LA updated and ISR activated | combined_TA_LA |
| T3412 value | 00100001 | 1 minute |  |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |
| T3423 value | 00100001 | 1 minute |  |

Table 9.2.3.3.3.3.3-7: Message TRACKING AREA UPDATE REQUEST (step 14, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| Old GUTI |  |  |  |
| M-TMSI | EFB1EE97 | Mapped from P-TMSI assigned in the preamble and RAI of cell 5 (start with 11 and RAI in 2nd most significant byte). |  |
| Last visited registered TAI | RAI of cell 5 |  |  |
| Additional GUTI | Same GUTI as assigned in the preamble |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.3.3.3-8: Message TRACKING AREA UPDATE ACCEPT (step 15, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | TA updated | TA_only |
|  | 001 | combined TA/LA updated | combined_TA_LA |
| T3412 value | 01010101 | 126 minutes |  |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

Table 9.2.3.3.3.3.3-9: Message TRACKING AREA UPDATE REQUEST (step 5Aa1, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
|  | 010 | combined TA/LA updating with IMSI attach |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.3.3.3-10: Message TRACKING AREA UPDATE ACCEPT (step 5Aa2, table 9.2.3.3.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated |  |
|  | 101 | combined TA/LA updated and ISR activated |  |

Table 9.2.3.3.3.3.3-11: Message ROUTING AREA UPDATE ACCEPT (step 12Aa2, table 9.2.3.3.3.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 001 | combined RA/LA updated |  |
| Periodic RA update timer | 01010101 | 126 minutes |  |
| Routing area identification | See 36.508 table 4.4.4-2 | RAI of Cell 5 |  |
| P-TMSI signature | Not present |  |  |
| Allocated P-TMSI | Not present |  |  |

##### 9.2.3.3.4	First S1 mode to Iu mode inter-system change after attach

9.2.3.3.4.1	Test Purpose (TP)

(1)

with { the UE powered on in E-UTRAN with a USIM containing valid E-UTRAN NAS and Security parameters including a valid GUTI, and has successfully performed a combined EPS/IMSI attach or EPS only attach, established PDN connectivity, and performed TRACKING AREA UPDATE REQUEST  }

ensure that {

when { the UE reselects UTRAN configured in MNO=II and TIN is set to the temporary ID belonging to the currently used RAT (i.e. ISR not active) }

then { the UE transmits a LOCATION UPDATING REQUEST to the MSC/VLR and derives the IDNNS from the MSC/VLR TMSI, and transmits a ROUTING AREA UPDATE REQUEST message including P-TMSI, P-TMSI signature including the NAS token, and RAI mapped from GUTI, the old routing area identification mapped from the RAI, the GPRS ciphering key sequence number indicating the eKSI associated with the EPS security context. Old P-TMSI signature and Old routing area identification are mapped from the GUTI. The message is encapsulated in an INITIAL DIRECT TRANSFER message including IDNSS mapped from GUTI, START value set to 0 and PLMN identity }

(2)

with { the UE camped on E-UTRAN and has a valid GUTI and P-TMSI }

ensure that {

when { the UE reselects UTRAN and receives a new P-TMSI as part of the RAU procedure }

then { the UE uses the new P-TMSI (and not GUTI or old P-TMSI) for subsequent IDNNS network access (e.g. via the Service Request procedure }

}

(3)

with { the UE has performed a RAU procedure in UTRAN after initial registration at power on in E-UTRAN  }

ensure that {

when { the UE reselects a E-UTRAN cell with the same LA as previously selected}

then { the UE transmits a TRACKING AREA UPDATE REQUEST message including eKSI stored in the USIM, old GUTI mapped from RAI and P-TMSI, GPRS ciphering sequence number, nonceUE. The UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set to s-TMSI or "Random value” for pre-Rel-12 UE, or set to randomValue from Rel-12 and onwards UE. The establishmentcause is set to MO-signalling; and, in the RRCConnectionSetupComplete message the selectedPLMN-identity, mmegi and mmec indicate the value of the registered MME when ue-Identity is set to randomValue (e.g. as retrieved from the USIM at power-on, or, as received in the last TRACKING AREA UPDATE ACCEPT message) }

}

(4)

with { the UE has transmitted a ROUTING AREA UPDATE REQUEST message after initial registration at power on in E-UTRAN }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message }

then { the UE transmits a SECURITY MODE COMPLETE and starts performing ciphering an integrity protection using CK' and IK' derived from KASME and UL NAS COUNT }

(5)

with { the UE has received an AUTHENTICATION AND CIPHERING REQUSET message after it has performed ciphering an integrity protection using CK' and IK' derived from KASME and UL NAS COUNT }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message }

then { the UE transmits a SECURITY MODE COMPLETE and starts performing ciphering an integrity protection using new CK and IK }

NOTE:	ISR considered not in the scope of this test case, so it is never activated.

9.2.3.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.003 clause 2.8.2.1, 2.8.2.2, TS 23.401 Annex J.2; TS 24.301 clause 5.5.3.2.2 and Annex D; TS 25.331 clauses 8.1.8.2, 8.1.8.3 (on IDNNS) and 8.3.8.2; TS 24.008 clause 4.7.5.1.1 and 4.7.5.1.3; TS 23.236 clause 5.1; and TS 36.331 clauses 5.3.3.3 and 5.3.3.4.

[TS 23.003, clause 2.8.2.1]

The mapping of the GUTI shall be done to the combination of RAI of GERAN / UTRAN and the P-TMSI:

E-UTRAN <MCC> maps to GERAN/UTRAN <MCC>

E-UTRAN <MNC> maps to GERAN/UTRAN <MNC>

E-UTRAN <MME Group ID> maps to GERAN/UTRAN <LAC>

E-UTRAN <MME Code> maps to GERAN/UTRAN <RAC> and is also copied into the 8 most significant bits of the NRI field within the P-TMSI;

E-UTRAN <M-TMSI>			maps as follows:

-	6 bits of the E-UTRAN <M-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the GERAN/UTRAN <P-TMSI>;

-	16 bits of the E-UTRAN <M-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the GERAN/UTRAN <P-TMSI>;

-	and the remaining 8 bits of the E-UTRAN <M-TMSI> are mapped into the 8 MBS bits of the <P-TMSI signature> field.

For UTRAN, the 10-bit long NRI bits are masked out from the P-TMSI and also supplied to the RAN node as IDNNS (Intra Domain NAS Node Selector). However, the RAN configured NRI length should not exceed 8 bits.

[TS 23.003, clause 2.8.2.2]

The mapping of P-TMSI (TLLI) and RAI in GERAN/UTRAN to GUTI in E-UTRAN shall be performed as follows:

GERAN/UTRAN <MCC>	maps to E-UTRAN <MCC>

GERAN/UTRAN <MNC>	maps to E-UTRAN <MNC>

GERAN/UTRAN <LAC>	maps to E-UTRAN <MME Group ID>

GERAN/UTRAN <RAC>	maps into bit 23 and down to bit 16 of the M-TMSI

The 8 most significant bits of GERAN/UTRAN <NRI> map to the MME code.

GERAN/UTRAN <P-TMSI> maps as follows:

-	6 bits of the GERAN/UTRAN <P-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the E-UTRAN <M-TMSI>;

-	16 bits of the GERAN/UTRAN <P-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the E-UTRAN <M-TMSI>.

The values of <LAC> and <MME group id> shall be disjoint, so that they can be differentiated. The most significant bit of the <LAC> shall be set to zero; and the most significant bit of <MME group id> shall be set to one. Based on this definition, the most significant bit of the <MME group id> can be used to distinguish the node type, i.e. whether it is an MME or SGSN.

[TS 23.401, Annex J.2 "Usage of TIN”]

…

When ISR is not active the TIN is always set to the temporary ID belonging to the currently used RAT. This guarantees that always the most recent context data are used, which means during inter-RAT changes there is always context transfer from the CN node serving the last used RAT. The UE identities, old GUTI IE and additional GUTI IE, indicated in the next TAU Request message, and old P-TMSI IE and additional P-TMSI/RAI IE, indicated in the next RAU Request message depend on the setting of TIN and are specified in table 4.3.5.6-2.

[TS 24.301, clause 5.5.3.2.2 "Normal and periodic tracking area updating procedure initiation”]

...

If the UE supports A/Gb mode or Iu mode, the UE shall handle the GUTI as follows:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the old GUTI IE. If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

-	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

...

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

...

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode and the TIN is set to "P-TMSI", the UE shall include the GPRS ciphering key sequence number applicable for A/Gb mode or Iu mode and a nonceUE in the TRACKING AREA UPDATE REQUEST message.

…

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

…[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Tracking Area Update | MO signalling (See Note 1) | "originating signalling" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

[TS 36.331, clause 5.3.3.4]

...

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS  to include the information received from upper layers;

...

[TS 24.008, clause 4.7.5.1.1]

To initiate the normal routing area updating procedure, the MS sends the message ROUTING AREA UPDATE REQUEST to the network, starts timer T3330 and changes to state GMM-ROUTING-AREA-UPDATING-INITIATED.

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

...

If the routing area updating procedure is initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode and the TIN indicates "GUTI", the message ROUTING AREA UPDATE REQUEST shall include a P-TMSI signature filled with a NAS token as specified in 3GPP TS 33.401 [119]. In the GPRS ciphering key sequence number IE the MS shall indicate the value of the eKSI associated with the current EPS security context.

NOTE:	When the MS includes a P-TMSI signature filled with a NAS token, 8 bits of the NAS token will be filled with bits from the M-TMSI (see 3GPP TS 23.003 [4]).

If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in connected mode, the MS shall derive CK' and IK' from KASME and from the NAS downlink COUNT value indicated by lower layers as specified in 3GPP TS 33.401 [119]. If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode when the TIN indicates "GUTI", the MS shall derive CK' and IK' from the KASME and the NAS downlink COUNT value corresponding to the NAS token derived as specified in 3GPP TS 33.401 [119]. The MS shall indicate the eKSI value in the CKSN field of the GPRS ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message. Then, the MS shall reset the START value and store the mapped UMTS security context replacing the established UMTS security context.

[TS 24.008, clause 4.7.5.1.3]

…

A ROUTING AREA UPDATE COMPLETE message shall be returned to the network if the ROUTING AREA UPDATE ACCEPT message contained any of:

-	a P-TMSI;

-	Receive N-PDU Numbers (see 3GPP TS 44.065 [78] and 3GPP TS 25.322 [19b]); or

-	a request for the provision of Inter RAT handover information or E-UTRAN inter RAT handover information or both.

If Receive N-PDU Numbers were included, the Receive N-PDU Numbers values valid in the MS, shall be included in the ROUTING AREA UPDATE COMPLETE message.

If the network has requested the provision of Inter RAT handover information or E-UTRAN inter RAT handover information or both, the MS shall return a ROUTING AREA UPDATE COMPLETE message including the Inter RAT handover information IE or E-UTRAN inter RAT handover information IE or both to the network.

NOTE 1:	In Iu mode, after a routing area updating procedure, the mobile station can initiate Service Request procedure to request the resource reservation for the active PDP contexts if the resources have been released by the network or send upper layer message (e.g. ACTIVATE PDP CONTEXT REQUEST) to the network via the existing PS signalling connection.

In Iu mode, if the network wishes to prolong the PS signalling connection (for example, if the mobile station has indicated "follow-on request pending" in ROUTING AREA UPDATE REQUEST message) the network shall indicate the "follow-on proceed" in the ROUTING AREA UPDATE ACCEPT message. If the network wishes to release the PS signalling connection, the network shall indicate "no follow-on proceed" in the ROUTING AREA UPDATE ACCEPT message.

After that in Iu mode, the mobile station shall act according to the follow-on proceed flag included in the Update result information element in the ROUTING AREA UPDATE ACCEPT message (see subclause 4.7.13).

[TS 23.236, clause 5.1 "MS Functions”]

...

In Iu mode the MS provides the IDNNS to the RNC in the access stratum part of the RRC_initial_DT message as described in TS 25.331 [5].

If the MS is E-UTRAN capable, then TS 23.401 [22], TS 23.060 [2] and TS 23.003 [18] define rules as to how the MS shall select and encode the identity to place in the P-TMSI/TLLI parameters used in the Routing Area Update procedure. For the PS domain, the E-UTRAN capable MS shall use this P-TMSI parameter to derive the UTRAN IDNNS parameter. For the CS domain, the E-UTRAN temporary identities shall not be used to derive the IDNNS: instead the MS shall use its (MSC supplied) TMSI, if that TMSI is valid, to derive the IDNNS.

…

When the MS in Iu mode replies to IMSI paging, it shall derive IDNNS from (P)TMSI if a valid one is available. If (P)TMSI is not available, the MS shall derive IDNNS from IMSI.

[TS 25.331, clause 8.1.8.2]

In the UE, the initial direct transfer procedure shall be initiated, when the upper layers request establishment of a signalling connection. This request also includes a request for the transfer of a NAS message.

Upon initiation of the initial direct transfer procedure the UE shall:

1>	set the variable ESTABLISHMENT_CAUSE to the cause for establishment indicated by upper layers.

Upon initiation of the initial direct transfer procedure when the UE is in idle mode, the UE shall:

1>	perform an RRC connection establishment procedure, according to subclause 8.1.3;

NOTE:	If an RRC connection establishment is ongoing, this procedure continues unchanged, i.e. it is not interrupted.

1>	if the RRC connection establishment procedure was not successful:

2>	if the establishment cause for the failed RRC connection establishment was set to "MBMS reception" and a different cause value is stored in the variable "ESTABLISHMENT_CAUSE":

3>	UE-AS (RRC) initiates a new RRC connection establishment procedure, using the establishment cause as contained in the variable ESTABLISHMENT_CAUSE.

2>	otherwise:

3>	indicate failure to establish the signalling connection to upper layers and end the procedure.

1>	when the RRC connection establishment procedure is completed successfully:

2>	continue with the initial direct transfer procedure as below.

...

The UE shall, in the INITIAL DIRECT TRANSFER message:

1>	set the IE "NAS message" as received from upper layers; and

1>	set the IE "CN domain identity" as indicated by the upper layers; and

1>	set the IE "Intra Domain NAS Node Selector" as follows:

2>	derive the IE "Intra Domain NAS Node Selector" from TMSI/PMTSI, IMSI, or IMEI; and

2>	provide the coding of the IE "Intra Domain NAS Node Selector" according to the following priorities:

1.	derive the routing parameter for IDNNS from TMSI (CS domain) or PTMSI (PS domain) whenever a valid TMSI/PTMSI is available;

2.	base the routing parameter for IDNNS on IMSI when no valid TMSI/PTMSI is available;

3.	base the routing parameter for IDNNS on IMEI only if no (U)SIM is inserted in the UE.

1>	if the UE, on the existing RRC connection, has received a dedicated RRC message containing the IE "Primary PLMN Identity" in the IE "CN Information Info":

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the latest PLMN information received via dedicated RRC signalling. If NAS has indicated the PLMN towards which a signalling connection is requested, and this PLMN is not in agreement with the latest PLMN information received via dedicated RRC signalling, then the initial direct transfer procedure shall be aborted, and NAS shall be informed.

1>	if the UE, on the existing RRC connection, has not received a dedicated RRC message containing the IE "CN Information Info" , and if the IE "Multiple PLMN List" was broadcast in the cell where the current RRC connection was established:

2>	set the IE "PLMN identity" in the INITIAL DIRECT TRANSFER message to the PLMN chosen by higher layers [5, 25] amongst the PLMNs in the IE "Multiple PLMN List" broadcast in the cell where the RRC connection was established.

[TS 25.331, clause 8.1.8.3]

On reception of the INITIAL DIRECT TRANSFER message the NAS message should be routed using the IE "CN Domain Identity". UTRAN may also use the IE "Intra Domain NAS Node Selector" and the IE "PLMN identity" for routing among the CN nodes for the addressed CN domain.

If no signalling connection exists towards the chosen node, then a signalling connection is established.

When the UTRAN receives an INITIAL DIRECT TRANSFER message, it shall not affect the state of any other ongoing RRC procedures, when not stated otherwise elsewhere.

The UTRAN should:

1>	set the START value for the CN domain indicated in the IE "CN domain identity" to the value of the IE "START".

[TS 25.331, clause 8.3.8.2]

When the UE makes an inter-RAT cell reselection to UTRAN according to the criteria specified in [4], it shall initiate this procedure. The inter-RAT cell reselection made by the UE may use system information broadcast from the source radio access technology or UE dedicated information.

If the NAS procedures associated with inter-system change specified in [5] require the establishment of an RRC connection, the UE shall:

1>	set the variable ESTABLISHMENT_CAUSE to "Inter-RAT cell reselection";

NOTE:	This value of ESTABLISHMENT_CAUSE has priority over the cause requested by upper layers.

1>	initiate an RRC connection establishment procedure as specified in subclause 8.1.3;

1>	after initiating an RRC connection establishment:

2>	release all resources specific to the other radio access technology.

If the NAS procedures associated with inter-system change specified in [5] do not require the establishment of an RRC connection, the UE shall:

1>	enter idle mode in the target cell without accessing the cell; and

1>	release all resources specific to the other radio access technology.

9.2.3.3.4.3	Test description

9.2.3.3.4.3.1	Pre-test conditions

System Simulator:

-	cell A is configured as Serving cell, cell 5 as Non-Suitable cell;

-	cell 5 (belongs to LAI-2) is operating in network operation mode II (No Gs interface present);

-	cell A has the absolute priorities set to LTE.

-	System information combination 4 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	For cell 5, power levels are defined in TS 34.108 subclause 6.1.5 (FDD) or 6.1.6 (TDD).

UE:

- the UE is previously registered on UTRAN, and when on UTRAN, the UE is last registered on cell 5 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered Idle Mode (state 2) in cell A according to TS 36.508 [18].

9.2.3.3.4.3.2	Test procedure sequence

Table 9.2.3.3.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The signal strength of Cell 5 is raised to that of the Serving Cell and that of Cell A is lowered to that of a non-Suitable Neighbour Cell as defined in table 6.2.2.1-1 of TS 36.508 [18]. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.2.3.3.4.3.2-2 occurs in parallel with step 5 | - | - | - | - |
| 5 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5 as specified in the specific message contents included in an INITIAL DIRECT TRANSFER message with intraDomainNasNodeSelector mapped from GUTI-1? | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| 5A | The SS sends a SECURITY MODE COMMAND message to activate integrity protection and ciphering. This message is integrity protected using IK' derived from KASME and UL NAS COUNT. | - | - | - | - |
| 5B | Check: Does the UE transmit a SECURITY MODE COMPLETE message integrity protected using IK' derived from KASME and UL NAS COUNT? | - | - | 4 | P |
| 6 | The SS transmits a ROUTING AREA UPDATE ACCEPT message with Update result = "RA updated” and new P-TMSI allocated with a different NRI (=IDNNS) to that in the GUTI and the P-TMSI that was stored on the USIM. This message is ciphered and integrity protected using CK' and IK' derived from KASME and UL NAS COUNT. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 7 | Check: Does the UE transmit a ROUTING AREA UPDATE COMPLETE message ciphered and integrity protected using CK' and IK' derived from KASME and UL NAS COUNT? | --> | ROUTING AREA UPDATE COMPLETE | 4 | P |
| 8 | The SS releases the RRC connection | - | - | - | - |
| 9 | The SS transmits Paging with new P-TMSI allocated in step 6 on cell 5 | <-- | Page | - | - |
| 10 | Check: Does the UE send a Service Request to access the PS domain using IDNNS mapped from the P-TMSI allocated in step 6? | --> | SERVICE REQUEST | 2 | P |
| 10A | The SS sends an AUTHENTICATION AND CIPHERING REQUEST message to perform a UMTS AKA procedure. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 10B | The UE sends an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 11 | The SS sends a SECURITY MODE COMMAND message to activate integrity protection and ciphering. This message is integrity protected using new IK. | - | - | - | - |
| 11A | Check: Does the UE transmit a SECURITY MODE COMPLETE message integrity protected using new IK? | - | - | 5 | - |
| 11B | The SS transmits a UE CAPABILITY ENQUIRY message requesting the UE UTRAN capability. This message is ciphered and integrity protected using new CK and IK. Check: Does the UE transmit a UE CAPABILITY INFORMATION message? Note: the purpose of this message is to show that the UE uses new CK and IK, and not CK' and IK' used in step 7. | - | - | 5 | P |
| 11C | SS transmits a UE CAPABILITY INFORMATION CONFIRM message | <-- | UE CAPABILITY INFORMATION CONFIRM | - | - |
| 12 | The SS releases the RRC connection | - | - | - | - |
| 12A | Cell A is configured as the Serving cell and cell 5 is configured as a Suitable Neighbour Cell. | - | - | - | - |
| 12 B | Check: Does the UE transmit an RRCConnectionRequest with the ue-Identity set for pre-Rel-12 UE to s-TMSI or "random value”, and for Rel-12 and onwards UE set to randomValue, the establishmentcause set to MO-signalling? | --> | RRCConnectionRequest | 3 | P |
| 13 | The SS responds with RRCConnectionSetup. | <-- | RRCConnectionSetup | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionSetupComplete message with the mmegi and mmec set to the values derived from the mapped RAI and P-TMSI and with the gummei type set to mapped when randomValue is included in step 12B, otherwise without the registeredMME included, and containing a TRACKING AREA UPDATE REQUEST message? | --> | RRCConnectionSetupComplete (TRACKING AREA UPDATE REQUEST) | 3 | P |
| 15 | The SS responds with a TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 15A | UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 16 | The SS releases the RRC connection. | - | - | - | - |

Table: 9.2.3.3.4.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Steps 1a1 to 1a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if the UE is configured in CS/PS mode. |  |  |  |  |
| 1a1 | Check: The UE transmits a LOCATION UPDATING REQUEST on cell 5 to the MSC as specified in the specific message contents included in an INITIAL DIRECT TRANSFER message with intraDomainNasNodeSelector mapped from MSC TMSI allocated in the ATTACH ACCEPT message (step 14 table 4.5.2.3-1 of TS 36.508 [18])). | --> | LOCATION UPDATING REQUEST | 1 | P |
| 1a2 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |

            - Specific message contents

Table 9.2.3.3.4.3.3-1: Void

Table 9.2.3.3.4.3.3-2: Void

Table 9.2.3.3.4.3.3-2A: INITIAL DIRECT TRANSFER (step 1a1, Table 9.2.3.3.4.3.2-2)

| Derivation path: 25.331 clause 11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialDirectTransfer ::= SEQUENCE { |  |  |  |
| cn-DomainIdentity | cs-domain |  |  |
| intraDomainNasNodeSelector SEQUENCE { |  |  |  |
| version CHOICE { |  |  |  |
| release99 SEQUENCE { |  |  |  |
| gsm-Map-IDNNS SEQUENCE { |  |  |  |
| routingbasis CHOICE { |  |  |  |
| tMSIofsamePLMN { |  |  | If UE is configured as PS/CS mode 1 or 2 of operation |
| Routingparameter | bit 23 to bit 14 of TMSI allocated in pre-test |  |  |
| } |  |  |  |
| localPTMSI { |  |  | If UE is configured as PS mode 1 or 2 of operation |
| Routingparameter | bit 23 to bit 14 of TMSI allocated in pre-test |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nas-Message | See table 9.2.3.3.4.3.3-5 |  |  |
| measuredResultsOnRACH | Not checked |  |  |
| v3a0NonCriticalExtensions SEQUENCE { |  |  |  |
| initialDirectTransfer-v3a0ext SEQUENCE { |  |  |  |
| start-Value | Any |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE { |  |  |  |
| initialDirectTransfer-r3-add-ext | Not checked |  |  |
| v590NonCriticalExtension SEQUENCE { |  |  |  |
| initialDirectTransfer-v590ext SEQUENCE { |  |  |  |
| establishmentCause | Registration or Not present |  |  |
| } |  |  |  |
| v690NonCriticalExtensions | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.4.3.3-3: LOCATION UPDATING REQUEST (step 1a1, Table 9.2.3.3.4.3.2-2)

| Derivation path: 24.008 table 9.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 0000 | LA updating | If UE is configured as PS/CS mode 1 or 2 of operation |
|  | 0010 | IMSI attach | If UE is configured as PS mode 1 or 2 of operation |
| Ciphering key sequence number | eKSI stored on the test USIM |  |  |
| Location area identification | PLMN= MCC/MNC LAC 1=1 | ''PLMN is set to the same MCC/MNC stored in EFIMSI'' ''TAI-1'' | If UE is configured as PS/CS mode 1 or 2 of operation |
|  | PLMN= MCC/MNC LAC 1=2 |  | If UE is configured as PS mode 1 or 2 of operation |
| Mobile identity | TMSI-1 | Allocated in preamble |  |

Table 9.2.3.3.4.3.3-3A: LOCATION UPDATING ACCEPT (step 4, Table 9.2.3.3.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

Table 9.2.3.3.4.3.3-4: INITIAL DIRECT TRANSFER (step 5, Table 9.2.3.3.4.3.2-1)

| Derivation path: 25.331 clause 11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialDirectTransfer ::= SEQUENCE { |  |  |  |
| cn-DomainIdentity | ps-domain |  |  |
| intraDomainNasNodeSelector SEQUENCE { |  |  |  |
| version CHOICE { |  |  |  |
| release99 SEQUENCE { |  |  |  |
| gsm-Map-IDNNS SEQUENCE { |  |  |  |
| routingbasis CHOICE { |  |  |  |
| tMSIofsamePLMN { |  |  |  |
| Routingparameter | bit 23 to bit 14 of P-TMSI mapped from GUTI allocated to UE in pre-test |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nas-Message | See table 9.2.3.3.4.3.3-5 |  |  |
| measuredResultsOnRACH | Not checked |  |  |
| v3a0NonCriticalExtensions SEQUENCE { |  |  |  |
| initialDirectTransfer-v3a0ext SEQUENCE { |  |  |  |
| start-Value | Any |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE { |  |  |  |
| initialDirectTransfer-r3-add-ext | Not checked |  |  |
| v590NonCriticalExtension SEQUENCE { |  |  |  |
| initialDirectTransfer-v590ext SEQUENCE { |  |  |  |
| establishmentCause | Registration or Not present |  |  |
| } |  |  |  |
| v690NonCriticalExtensions | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.4.3.3-5: Message ROUTING AREA UPDATE REQUEST (step 5, Table 9.2.3.3.4.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 000 | RA updating |  |
| GPRS ciphering key sequence number | eKSI stored on the test USIM |  |  |
| Old routing area identification | GUTI-1 right shifted by 32bits |  |  |
| MS Radio Access capability | Not checked |  |  |
| Old P-TMSI signature | Mapped from GUTI-1 |  |  |
| Requested READY timer value | Not checked |  |  |
| DRX parameter | Not checked |  |  |
| TMSI status | Not present |  |  |
| P-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: 1 - bit 29 to bit 24: bit 29 to bit 24 of M-TMSI allocated in preamble - bit 23 to bit 16: MME code allocated in preamble - bit 15 to bit 0: bit 15 to bit 0 of M-TMSI allocated in preamble |  |  |
| MS network capability | Not checked |  |  |
| PDP context status | Not checked |  |  |
| PS LCS Capability | Not checked |  |  |
| MBMS context status | Not checked |  |  |
| UE network capability | Any allowed value |  |  |
| Additional mobile identity | P-TMSI allocated in pre-test |  |  |
| Additional old routing area identification | RAI-1 |  |  |
| Mobile station classmark 2 | Not checked |  |  |
| Mobile station classmark 3 | Not checked |  |  |
| Supported Codecs | Not checked |  |  |

Table 9.2.3.3.4.3.3-6: Message SERVICE REQUEST (step 10, Table 9.2.3.3.4.3.2-1)

| Derivation path: 24.008 table 9.4.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Ciphering key sequence number | FFS |  |  |
| Service Type | 010 (Paging Response) |  |  |
| P-TMSI | P-TMSI allocated in step 6. |  |  |

Table 9.2.3.3.4.3.3-6A: SECURITY MODE COMMAND (step 11, Table 9.2.3.3.4.3.2-1)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | Set to MAC-I value computed by the SS. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC Message Sequence Number | Set to an arbitrarily selected integer between 0 and 15 |  |  |
| Security capability | Set same value as originally sent by UE in the RRC CONNECTION SETUP COMPLETE message in step 10. |  |  |
| Ciphering mode info |  |  |  |
| - Ciphering mode command | Start/restart |  |  |
| - Ciphering algorithm | Set one of the algorithms supported by the UE as indicated in the IE "security capability" in the RRC CONNECTION SETUP COMPLETE message in step 10. |  |  |
| - Ciphering activation time for DPCH | Not Present |  |  |
| - Radio bearer downlink ciphering activation time info |  |  |  |
| - Radio bearer activation time |  |  |  |
| - RB identity | 1 |  |  |
| - RLC sequence number | Current RLC SN stored in the SS |  |  |
| - RB identity | 2 |  |  |
| - RLC sequence number | Current RLC SN stored in the SS +2 |  |  |
| - RB identity | 3 |  |  |
| - RLC sequence number | Current RLC SN stored in the SS |  |  |
| - RB identity | 4 |  |  |
| - RLC sequence number | Current RLC SN stored in the SS |  |  |
| Integrity protection mode info |  |  |  |
| - Integrity protection mode command | startIntegrityProtection |  |  |
| - Integrity protection initialisation number | Arbitrary selects 32 bits number for FRESH |  |  |
| - Integrity protection algorithm | Set one of the algorithms supported by the UE as indicated in the IE "security capability" in the RRC CONNECTION SETUP COMPLETE message in step 10. |  |  |
| CN domain identity | PS |  |  |
| UE system specific security capability | Not present |  | not GSM |
| UE system specific security capability |  |  | GSM |
| - Inter-RAT UE security capability |  |  |  |
| - CHOICE system | GSM |  |  |
| - GSM security capability | Set the same as the algorithms supported by the UE as indicated in the IE "UE system specific capability" in the RRC CONNECTION SETUP COMPLETE message in step 10. |  |  |

| Condition | Explanation |
| --- | --- |
| GSM | UE supporting GSM |

Table 9.2.3.3.4.3.3-6B: SECURITY MODE COMPLETE (step 11 A , Table 9.2.3.3.4.3.2-1)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRC transaction identifier | The value of this IE is checked to see that it matches the value of the same IE transmitted in the downlink SECURITY MODE COMMAND message. |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC Message Sequence Number | The value is used by SS to compute the XMAC-I value. |  |  |
| Uplink integrity protection activation info |  |  |  |
| - RRC message sequence number list |  |  |  |
| - RRC message sequence number | Check to see if the RRC SN for RB 0 is present |  |  |
| - RRC message sequence number | Check to see if the RRC SN for RB 1 is present |  |  |
| - RRC message sequence number | Check to see if the RRC SN for RB 2 is present |  |  |
| - RRC message sequence number | Check to see if the RRC SN for RB 3 is present |  |  |
| - RRC message sequence number | Check to see if the RRC SN for RB 4 is present |  |  |
| Radio bearer uplink ciphering activation time info |  |  |  |
| - Radio bearer activation time |  |  |  |
| - RB Identity | 1 |  |  |
| - RLC sequence number | Check to see if the RLC SN for RB1is present |  |  |
| - RB Identity | 2 |  |  |
| - RLC sequence number | Check to see if the RLC SN for RB2is present |  |  |
| - RB Identity | 3 |  |  |
| - RLC sequence number | Check to see if the RLC SN for RB3 is present |  |  |
| - RB Identity | 4 |  |  |
| - RLC sequence number | Check to see if the RLC SN for RB4 is present |  |  |

Table 9.2.3.3.4.3.3-7: Message RRCConnectionRequest (step 12 B, Table 9.2.3.3.4.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity CHOICE { |  |  |  |
| s-TMSI | Received from upper layer NAS | Rel-8 to Rel-11 inclusive |  |
| randomValue | Any allowed value | Rel-8 and onwards |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.4.3.3-8: Message RRCConnectionSetupComplete (step 14, Table 9.2.3.3.4.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| registeredMME | Not present | Shall be absent if the upper layers provided s-TMSI in RRCConnectionRequest message in step 12B for pre-Rel-12 UE |  |
| registeredMME { |  | Present only when the ue-Identity in step 12B is set to randomValue |  |
| plmn-Identity | Not present |  |  |
| Mmegi | LAC sent to the UE in step 6 |  |  |
| Mmec | Bit 23 to bit 16 of P-TMSI sent to the UE in step 6 |  |  |
| } |  |  |  |
| dedicatedInfoNAS | See table 9.2.3.3.4.3.3-9 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gummei-Type-r10 | mapped | Present when the registeredMME is included |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.3.4.3.3-9: Message TRACKING AREA UPDATE REQUEST (step 14, Table 9.2.3.3.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 with condition UNCIPHERED (the message is integrity protected using the key stored in the USIM). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 or 010 | 'TA updating' or 'combined TA/LA updating with IMSI attach' |  |
| NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier stored in the USIM |  |  |
| TSC | '0'B | native security context (for KSIASME) |  |
| Old GUTI or IMSI |  |  |  |
| Type of identity | 110 | GUTI |  |
| MNC/MCC | Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM |  |  |
| MME Group ID | LAI allocated at step 8 |  |  |
| MME Code | Bit 23 to bit 16 of P-TMSI allocated at step 8 |  |  |
| M-TMSI | Value is specified bit by bit below: - bit 31 and bit 30: FFS - bit 29 to bit 24: bit 29 to bit 24 of P-TMSI allocated at step 8 - bit 23 to bit 16: RAC allocated at step 8 - bit 15 to bit 0: bit 15 to bit 0 of P-TMSI allocated at step 8 |  |  |
| GPRS ciphering key sequence number | GPRS ciphering key sequence number allocated at step 10A |  |  |
| Old P-TMSI signature | P-TMSI signature allocated at step 8 |  |  |
| Additional GUTI | GUTI-1 |  |  |
| NonceUE | Any allowed value (must be present) |  |  |
| UE network capability | Any allowed value (must be present) |  |  |
| Last visited registered TAI | TAI stored in the test USIM |  |  |
| MS network capability | Any allowed value (must be present) |  |  |
| Old location area identification | Not present if "EPS update type" is 'TA updating', LAI-2 is "EPS update type" is 'combined TA/LA updating' |  |  |
| TMSI status | Not present |  |  |

##### 9.2.3.3.5	Periodic routing area update

9.2.3.3.5.1	Test Purpose (TP)

(1)

with { UE is camped on an E-UTRAN cell or in ECM-CONNECTED state and ISR is activated }

ensure that {

when { UE performs E-UTRA RRC state transitions }

then { UE keeps the periodic routing area update timer T3312 running }

}

(2)

with { UE is camped on an E-UTRAN cell or ECM-CONNECTED state and ISR is activated }

ensure that {

when { the periodic routing area update timer T3312 expires }

then { UE does not change RAT }

}

(3)

with { UE is in state GMM-REGISTERED.NO-CELL-AVAILABLE and ISR is activated and the periodic routing area update timer T3312 expires }

ensure that {

when { UE enters the state GMM-REGISTERED.NORMAL-SERVICE }

then { UE initiates the routing area updating procedure }

}

9.2.3.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.008, clause 4.7.2.2 and TS 23.401, clause 4.3.5.2.

[TS 24.008, clause 4.7.2.2]

If ISR is activated, the MS shall keep both the periodic tracking area update timer (timer T3412) and the periodic routeing area update timer (timer T3312). The two separate timers run in the MS for updating MME and SGSN independently. If the periodic routeing area update timer expires and the MS is in state GMM-REGISTERED.NO-CELL-AVAILABLE, the MS shall set its TIN to "RAT-related TMSI" and start the GERAN/UTRAN Deactivate ISR timer T3323. The MS shall initiate the routeing area updating procedure and stop the timer T3323 when the MS enters the state GMM-REGISTERED.NORMAL-SERVICE before timer T3323 expires. After expiry of timer T3323 the MS shall deactivate ISR by setting its TIN to "GUTI" and initiate the routing area updating procedure when the UE enters the state GMM-REGISTERED.NORMAL-SERVICE.

[TS 23.401, clause 4.3.5.2]

If the UE is camped on an E-UTRAN cell or is in ECM-CONNECTED state when the UE's periodic RAU or periodic LAU timer expires and ISR is activated the UE shall start the GERAN/UTRAN Deactivate ISR timer. After the GERAN/UTRAN Deactivate ISR timer expires the UE shall deactivate ISR by setting its TIN to "GUTI". The GMM/PMM-REGISTERED UE shall remember it has to perform a Routeing Area Update to the SGSN or a Location Area Update to the MSC when it next returns to 2G/3G coverage.

The E-UTRAN Deactivate ISR timer is stopped when the UE performs a successful TAU; and the GERAN/UTRAN Deactivate ISR timer is stopped when the UE performs a successful RAU/LAU.

Expiry of the periodic TAU timer, or, the periodic RAU timer, or, the periodic LAU timer shall not cause the UE to change RAT.

…

E-UTRAN RRC state transitions shall have no impact on the periodic RAU timer or periodic LAU timer except that handover from 2G/3G to E-UTRAN shall cause the periodic RAU timer to be started from its initial value.

9.2.3.3.5.3	Test description

9.2.3.3.5.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to "Serving cell";

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (home PLMN, RAI-1, NMO 1) is set to "Non-suitable cell";

-	 if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (home PLMN, RAI-1, NMO 1) is set to "Non-suitable cell";

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.3.5.3.2	Test procedure sequence

Table 9.2.3.3.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b5 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_UTRA THEN the SS sets the cell type of Cell 5 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 1a2 | The UE transmits a ROUTING AREA UPDATE REQUEST message on Cell 5. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 1a3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 1a4 | Void | - | - |  |  |
| 1a5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 1b1 | ELSE IF pc_GERAN THEN the SS sets the cell type of Cell 24 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 1b2 | The UE transmits a ROUTING AREA UPDATE REQUEST message on Cell 24. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 1b2A1 | The SS transmits an Authentication and Ciphering Request message on Cell 24 | <-- | Authentication and Ciphering Request | - | - |
| 1b2A2 | The UE transmits an Authentication and Ciphering Response message on Cell 24 | --> | Authentication and Ciphering Response | - | - |
| 1b3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 24 | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 1b4 | Void | - | - | - | - |
| 1b5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 24 to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 1Aa1 to 1Aa4 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional TAU message. | - | - | - | - |
| 1Aa1 | The UE may transmit a TRACKING AREA UPDATE REQUEST message on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 1Aa2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message on Cell A. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 1Aa3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 1Aa4 | The SS releases the RRC connection. | - | - | - | - |
| 2 | The SS waits for 1 min. | - | - | - | - |
| 3 | The SS pages the UE on Cell A using S-TMSI with CN domain indicator set to “PS”. | <-- | - | - | - |
| 4 | The UE transmits a SERVICE REQUEST message on Cell A. | --> | SERVICE REQUEST | - | - |
| 5 | The SS transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message on Cell A activating a new EPS bearer context. This message is included in an RRCConnectionReconfiguration message to setup the new radio bearer associated with the dedicated EPS bearer context activated by the NAS message. | <-- | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST | - | - |
| 6 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message on Cell A. | --> | ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT | - | - |
| 7 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message on Cell A deactivating the dedicated EPS bearer context activated at Step 5. This message is included in an RRCConnectionReconfiguration message. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 8 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message on Cell A. | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 8A | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Steps 9a1 to 9b5 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 9a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN the SS sets the cell type of Cell 5 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 9a2 | Check: When T3312 started after Step 1a3 expires, does the UE transmit a ROUTING AREA UPDATE REQUEST on Cell 5? | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| 9a3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 9a4 | Void | - | - |  |  |
| 9a5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 5 to "Non-suitable cell". | - | - | - | - |
| 9b1 | ELSE IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS sets the cell type of Cell 24 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 9b2 | Check: When T3312 started after Step 1b3 expires, does the UE transmit a ROUTING AREA UPDATE REQUEST on Cell 24? | --> | ROUTING AREA UPDATE REQUEST | 1 | P |
| 9b3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 24. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 9b4 | Void | - | - |  |  |
| 9b5 | The SS sets the cell type of Cell A to "Serving cell" and the cell type of Cell 24 to "Non-suitable cell". | - | - | - | - |
| - | EXCEPTION: Steps 9Aa1 to 9Aa4 describe behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE sends an optional TAU message. | - | - | - | - |
| 9Aa1 | The UE may transmit a TRACKING AREA UPDATE REQUEST message on Cell A. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 9Aa2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message on Cell A. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9Aa3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message on Cell A. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 9Aa4 | The SS releases the RRC connection. | - | - | - | - |
| 10 | The SS waits for 1 min. | - | - | - | - |
| 11 | The SS pages the UE on Cell A using S-TMSI with CN domain indicator set to “PS”. | <-- | - | - | - |
| 12 | The UE transmits a SERVICE REQUEST message on Cell A. | --> | SERVICE REQUEST | - | - |
| 13 | The SS transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message on Cell A activating a new EPS bearer context. This message is included in an RRCConnectionReconfiguration message to setup the new radio bearer associated with the dedicated EPS bearer context activated by the NAS message. | <-- | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST | - | - |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message on Cell A. | --> | ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT | - | - |
| 15 | The SS waits until T3312 timer started at Step 9a3/9b3 has expired. | - | - | - | - |
| 16 | Check: After T3312 started after Step 9a3/9b3 expires and before T3323 expires, does the test result of generic test procedure in TS 36.508 subclause 6.4.2.6 indicate that a dedicated EPS bearer context is active on cell A? | - | - | 2 | - |
| 17 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message on Cell A deactivating the dedicated EPS bearer context activated at Step 13. This message is included in an RRCConnectionReconfiguration message. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 18 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message on Cell A. | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 19 | The SS waits for 2 min. | - | - | - | - |
| 19A | The SS releases the RRC connection. | - | - | - | - |
| 20 | Check: After T3312 started after Step 9a3/9b3 expires and before T3323 expires, does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in state Registered, Idle Mode on Cell A? | - | - | 2 | - |
| - | EXCEPTION: Steps 21a1 to 21b4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 21a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN the SS sets the cell type of Cell 5 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 21a2 | Check: After T3312 started after Step 9a3 expires and before T3323 expires, does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 5? | --> | ROUTING AREA UPDATE REQUEST | 3 | P |
| 21a3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 21a4 | Void | - | - |  |  |
| 21b1 | ELSE IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS sets the cell type of Cell 24 to "Serving cell" and the cell type of Cell A to "Non-suitable cell". | - | - | - | - |
| 21b2 | Check: After T3312 started after Step 9b3 expires and before T3323 expires, does the UE transmit a ROUTING AREA UPDATE REQUEST message on Cell 24? | --> | ROUTING AREA UPDATE REQUEST | 3 | P |
| 21b3 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 24. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 21b4 | Void |  | - |  |  |

9.2.3.3.5.3.3	Specific message contents

Table 9.2.3.3.5.3.3-1: Message ROUTING AREA UPDATE REQUEST (steps 1a2/1b2, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update type | Any allowed value |  |  |
| P-TMSI | P-TMSI mapped from GUTI assigned to UE during preamble |  | UTRA |
| MS network capability |  |  |  |
| ISR support | 1 |  |  |

Table 9.2.3.3.5.3.3-2: Message ROUTING AREA UPDATE REQUEST (step 9a2/9b2, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update type | 011 | “Periodic updating” |  |
| P-TMSI | P-TMSI assigned to UE during preamble |  | UTRA |
| MS network capability |  |  |  |
| ISR support | 1 |  |  |
| UE network capability | Not present |  |  |

Table 9.2.3.3.5.3.3-3: Message ROUTING AREA UPDATE ACCEPT (steps 1a3, and 21a3/21b3, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update result | 100 | “RA updated and ISR activated” |  |
|  | 001 | “combined RA/LA updated” |  |
| Periodic RA update timer | 00100111 | 7 min |  |
| Allocated P-TMSI | Not present |  |  |

Table 9.2.3.3.5.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 9Aa1, table 9.2.3.3.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old P-TMSI signature | Any allowed value |  |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.5.3.3-5 Message TRACKING AREA UPDATE ACCEPT (steps 1Aa2 and 9Aa2 Table 9.2.3.3.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 100 | TA updated and ISR activated | TA_only |
|  | 101 | “combined TA/LA updated and ISR activated” | combined_TA_LA |

Table 9.2.3.3.5.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 1Aa1, table 9.2.3.3.5.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating | TA_only |
|  | 010 | combined TA/LA updating with IMSI attach | combined_TA_LA |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old P-TMSI signature | Any allowed value |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| NonceUE | Any allowed value |  |  |

Table 9.2.3.3.5.3.3-7: Message ROUTING AREA UPDATE REQUEST (step 21a2/21b2, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update type | 011 | “Periodic updating” |  |
|  | 010 | “combined RA/LA updating with IMSI attach” |  |
| P-TMSI | P-TMSI assigned to UE during preamble |  | UTRA |
| MS network capability |  |  |  |
| ISR support | 1 |  |  |
| UE network capability | Not present |  | Update Type = periodic updating |
|  | Any allowed value |  | Update Type <> periodic updating |

Table 9.2.3.3.5.3.3-8: Message ROUTING AREA UPDATE ACCEPT (step 9a3, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update result | 100 | “RA updated and ISR activated” |  |
|  | 000 | “RA updated” |  |
| Periodic RA update timer | 00100111 | 7 min |  |
| Allocated P-TMSI | Not present |  |  |

Table 9.2.3.3.5.3.3-9: Message ROUTING AREA UPDATE ACCEPT (steps 1b3, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update result | 100 | “RA updated and ISR activated” |  |
|  | 001 | “combined RA/LA updated” |  |
| Periodic RA update timer | 00100111 | 7 min |  |
| READY Timer | 00000 | 0 sec |  |
| Allocated P-TMSI | Not present |  |  |
| PDP Context Status | Same value as in ROUTING AREA UPDATE REQUEST message |  |  |

Table 9.2.3.3.5.3.3-10: Message ROUTING AREA UPDATE ACCEPT (step 9b3, Table 9.2.3.3.5.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Update result | 100 | “RA updated and ISR activated” |  |
|  | 000 | “RA updated” |  |
| Periodic RA update timer | 00100111 | 7 min |  |
| READY Timer | 00000 | 0 sec |  |
| Allocated P-TMSI | Not present |  |  |
| PDP Context Status | Same value as in ROUTING AREA UPDATE REQUEST message |  |  |

##### 9.2.3.3.5a	Periodic Location Update

9.2.3.3.5a.1	Test Purpose (TP)

(1)

with { UE in MM IDLE substate NORMAL SERVICE with periodic location update timer T3212 running}

ensure that {

when {  UE camps on a E-UTRAN cell and successfully performs a Tracking Area Update and T3212 expires}

then { the UE does not perform a periodic location update whilst in E-UTRAN coverage and does not change RAT because T3212 has expired}

}

9.2.3.3.5a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.401 clauses 4.3.5.2; and TS 24.008 clauses 4.7.1.4.1 and 4.7.5.1.1; TS 24.301 clause 5.1.5.Unless otherwise stated these are Rel-8 requirements.

[TS 23.401, clause 4.3.5.2 "Reachability Management for UE in ECM-IDLE state"]

…

If the UE is EPS attached only and either camps on an E UTRAN cell or is in ECM CONNECTED state when the UE's periodic LAU timer expires, the UE shall perform a Location Area Update procedure in NMO II/III or combined RA/LA update in NMO I when it next returns to GERAN/UTRAN coverage.

…

Expiry of the periodic TAU timer, or, the periodic RAU timer, or, the periodic LAU timer shall not cause the UE to change RAT.

…

E-UTRAN RRC state transitions shall have no impact on the periodic RAU timer or periodic LAU timer except that handover from GERAN/UTRAN to E-UTRAN shall cause the periodic RAU timer to be started from its initial value.

[TS 24.008, 4.2.2.1 "Service State, NORMAL SERVICE"]

When in state MM IDLE and service state NORMAL SERVICE, the mobile station shall:

-	perform normal location updating when a new location area is entered;

-	perform location updating procedure at expiry of timer T3211 or T3213;

-	perform periodic updating at expiration of timer T3212;

-	perform IMSI detach;

-	support requests from the CM layer;

-	respond to paging; and

-	for an eCall only mobile station (as determined by information configured in USIM), perform the eCall inactivity procedure at expiry of timer T3242 or timer T3243.

[TS 24.008, 4.4.1 "Location updating procedure"]

…

Upon successful location updating the mobile station sets the update status to UPDATED in the SIM/USIM, and stores the Location Area Identification received in the LOCATION UPDATING ACCEPT message in the SIM/USIM. The attempt counter shall be reset.

[TS 24.008, 4.4.2 "Periodic updating"]

…

The procedure is controlled by the timer T3212 in the mobile station. If the timer is not already started, the timer is started each time the mobile station enters the MM IDLE substate NORMAL SERVICE or ATTEMPTING TO UPDATE. When the MS leaves the MM Idle State the timer T3212 shall continue running until explicitly stopped.

…

When the timer T3212 expires, the location updating procedure is started and the timer shall be set to its initial value for the next start. If the mobile station is in other state than MM Idle when the timer expires the location updating procedure is delayed until the MM Idle State is entered.

…

If the mobile station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

[TS 24.301, clause 5.1.5]

UEs that operate in CS/PS mode 1 or CS/PS mode 2 of operation should not use any MM timers related to MM specific procedures (e.g. T3210, T3211, T3212, T3213) while camped on E-UTRAN, unless the re-activation of these timers is explicitly described. If the MM timers are already running, the UE should not react on the expiration of the timers.

9.2.3.3.5a.3	Test description

9.2.3.3.5a.3.1	Pre-test conditions

System Simulator:

-	cell A is configured as Non-suitable cell;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (LAI-1, NMO II) is configured as Serving cell. The value of T3212 (in SIB1) is 0110 (6 minutes);

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, Cell 24 (LAI-1, NMO II) is configured as Serving cell and does not support Dual Transfer Mode (DTM_SUPPORT is configured as default (see TS 51.010-1 clause 40.2.1.1 [23]. The value of T3212 (Octet 4 of the Control Channel Description IE) is 0110 (6 minutes);

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	The UE is in state Switched Off (state 1).

9.2.3.3.5a.3.2	Test procedure sequence

Table 9.2.3.3.5a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on and camps on Cell 24 or Cell 5. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 5 to 7 the step specified in Table 9.2.3.3.5a.3.2-3 should take place. | - | - | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 2Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |  |  |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 2Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |  |  |
| 3-4 | Void | - | - | - | - |
| 5 | The UE transmits an ATTACH REQUEST message on Cell 24 or Cell 5 indicating "GPRS attach”. | --> | ATTACH REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are only on Cell 24 |  |  |  |  |
| 5Aa1 | The SS transmits an Authentication and Ciphering Request message on Cell 24 | <-- | Authentication and Ciphering Request | - | - |
| 5Aa2 | The UE transmits an Authentication and Ciphering Response message on Cell 24 | --> | Authentication and Ciphering Response | - | - |
| 6 | The SS transmits an ATTACH ACCEPT message indicating "GPRS only attached”. The UE sets the update status to UPDATED on the USIM. The SS allocates a P-TMSI. | <-- | ATTACH ACCEPT | - | - |
| 7 | The UE transmits an ATTACH COMPLETE to acknowledge the P-TMSI allocation. | --> | ATTACH COMPLETE |  |  |
| 7A | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) |  |  |  |  |
| 8 | The UE requests the activation of a default PDP context | --> | ACTIVATE PDP CONTEXT REQUEST |  |  |
| 9 | The SS accepts the request for default PDP context request and return an ACTIVATE PDP CONTEXT ACCEPT | <-- | ACTIVATE PDP CONTEXT ACCEPT |  |  |
| - | EXCEPTION: Step 9Aa1 describes behaviour that depends on the RAT combination; the "lower case letter" identifies a step sequence that takes place if a UTRA is applied. | - | - | - | - |
| 9Aa1 | IF px_RATComb_Tested = EUTRA_UTRA THEN the SS releases the RRC connection. | - | - | - | - |
| 10 | The signal strength of Cell A is raised to that of the Serving Cell and Cell 24 or Cell 5 is lowered to that of a Suitable Neighbour Cell as defined in table 6.2.2.1-1 of TS 36.508 [18]. Note: Cell 24 or Cell 5 is still suitable but the UE shall select Cell A | - | - | - | - |
| 11 | The UE camps on E-UTRAN cell A and transmits a TRACKING AREA UPDATE REQUEST message as defined in table 6.4.2.7A.1-1 of TS 36.508 [18]. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 12 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 13 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 14 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | SECURITY MODE COMMAND | - | - |
| 15 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE | - | - |
| 16 | SS responds with TRACKING AREA UPDATE ACCEPT message including a valid TAI list containing the TAI of Cell A; with PLMN ID of Cell A included in the GUTI. The UE sets the TIN = GUTI. Note: This Accept message includes EPS update result = combined TA/LA updated. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 18 | The SS releases the RRC Connection | - | - | - | - |
| - | Exception: Steps 18A to 18Aa4/18Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 18A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 18Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | -> | SERVICE REQUEST | - | - |
| 18Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 18Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 18Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.32 steps 4-11. | - | - | - | - |
| 18Aa5 | The SS releases the RRC connection | - | - | - | - |
| 18Ab1 | The 10s timer expires | - | - | - | - |
| 19-28 | Void | - | - | - | - |
| 29 | Check: Does the UE transmit a LOCATION UPDATING REQUEST (on Cell 24 or Cell 5) and changes RAT. Check this until 7 minutes after step 3 in Table 9.2.3.3.5a.3.2-3 (when in GERAN) or 7 (when in UTRAN) | --> | LOCATION UPDATING REQUEST | 1 | F |
| 30-36 | Void | - | - | - | - |

Table: 9.2.3.3.5a.3.2-2: Void

Table 9.2.3.3.5a.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a LOCATION UPDATING REQUEST with location updating type set to "IMSI attach” on Cell 24 or Cell 5. | --> | LOCATION UPDATING REQUEST | - | - |
| 2 | The SS transmits a LOCATION UPDATING ACCEPT, and the UE stores the received LAI and sets the update status to UPDATED on the USIM. The SS allocates a TMSI. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 3 | The UE transmits a TMSI REALLOCATION COMPLETE message to acknowledge the TMSI allocation. | --> | TMSI REALLOCATION COMPLETE | - | - |

9.2.3.3.5a.3.3	Specific message contents

Table 9.2.3.3.5a.3.3-1: Void

Table 9.2.3.3.5a.3.3-2:  Void

##### 9.2.3.3.6	Void

#### 9.2.3.4	A/Gb mode to S1 mode inter-system change

##### 9.2.3.4.1	TAU/RAU procedure for inter-system cell reselection between A/Gb and S1 modes

9.2.3.4.1.1	Test Purpose (TP)

(1)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and Security parameters including a valid GUTI stored on the USIM  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set to “randomValue” and the establishmentcause is set to MO-signalling; and, in the RRCConnectionSetupComplete message the selectedPLMN-identity, mmegi and mmec indicate the value of the registered MME  }

}

(2)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and EPS Security parameters including a valid GUTI stored on the USIM, and ISR not activated  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the parameters in the TRACKING AREA UPDATE REQUEST correctly i.e.:
           - the NAS key set identifierASME IE is set to the value stored in the USIM/allocated in
                  the last TRACKING AREA UPDATE ACCEPT message;
           - the Old GUTI IE is derived from the mapped P-TMSI and RAI;
           - the Additional GUTI IE is set to the GUTI stored in the USIM/allocated in the last
                  TRACKING AREA UPDATE ACCEPT message;
           - the GPRS ciphering key sequence number IE is set to the value allocated in the
                  AUTHENTICATION AND CIPHERING REQUEST message; and
           - the NONCEue IE is included;
           - the DRX parameter IE is not included. }

}

(3)

with { UE powered on in GERAN }

ensure that {

when { UE makes its first Tracking Area Update to E-UTRAN}

then { UE sends the UE radio capability information update needed IE }

}

(4)

with { UE registered in E-UTRAN, and, GERAN NAS and Security parameters including a valid P-TMSI available in the UE and ISR not activated }

ensure that {

when { UE performs a cell reselection to GERAN and performs a Routeing Area Update }

then { the UE derives the TLLI parameter in the RLC/MAC header from the GUTI allocated in the TRACKING AREA UPDATE ACCEPT message }

}

(5)

with { UE registered in E-UTRAN, and, GERAN NAS and GERAN Security parameters including a valid P-TMSI available in the UE, and ISR not activated }

ensure that {

when { UE performs a cell reselection to GERAN and performs a Routeing Area Update }

then { the UE encodes the parameters in the ROUTING AREA UPDATE REQUEST correctly i.e.:
           - [the GPRS ciphering key sequence number IE is set to the value mapped from
                  KSI-ASME;;]            - the Old routing area identification IE, and the Old P-TMSI signature IE are
                  mapped from the GUTI allocated in the TRACKING AREA UPDATE ACCEPT message;
           - the Additional mobile identity IE contains the P-TMSI allocated in the last received
                  ATTACH ACCEPT/ROUTING AREA UPDATE ACCEPT message; 
           - the Additional old routing area identification IE contains the RAI allocated in the
                  last received ATTACH ACCEPT/ROUTING AREA UPDATE ACCEPT message; and
           - [the DRX parameter IE is not included]. }

}

(6)

with { UE registered }

ensure that {

when { UE performs Tracking Area Update to E-UTRAN which is not for the "first TAU following GERAN/UTRAN Attach" or for a "UE radio capability update"}

then { UE does not send the UE radio capability information update needed IE }

}

(7)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and Security parameters including a valid GUTI stored on the USIM  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set to S-TMSI or random value for pre-Rel-12 UE,or set to random value from Rel-12 and onwards, and the establishmentcause is set to MO-signalling; and, in the RRCConnectionSetupComplete message the registeredMME is not included when s-TMSI is included, or include the registeredMME and the mapped GUMMEI otherwise }

}

9.2.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.003 clauses 2.8.2.1 and 2.8.2.2; TS 23.401 clauses 4.3.5.6, 5.3.3.1 and 5.3.3.3; TS 24.008 clauses 4.7.1.4.1 and 4.7.5.1.1; and TS 24.301 clauses 5.3.1.1 and 5.5.3.2.2.

[TS 23.003, clause 2.8.2.1 “Mapping from GUTI to RAI, P-TMSI and P-TMSI signature”]

The mapping of the GUTI shall be done to the combination of RAI of GERAN / UTRAN and the P-TMSI:

E-UTRAN <MCC> maps to GERAN/UTRAN <MCC>

E-UTRAN <MNC> maps to GERAN/UTRAN <MNC>

E-UTRAN <MME Group ID> maps to GERAN/UTRAN <LAC>

E-UTRAN <MME Code> maps to GERAN/UTRAN <RAC> and is also copied into the 8 most significant bits of the NRI field within the P-TMSI;

E-UTRAN <M-TMSI>			maps as follows:

-	6 bits of the E-UTRAN <M-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the GERAN/UTRAN <P-TMSI>;

-	16 bits of the E-UTRAN <M-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the GERAN/UTRAN <P-TMSI>;

-	and the remaining 8 bits of the E-UTRAN <M-TMSI> are mapped into the 8 MBS bits of the <P-TMSI signature> field.

For UTRAN, the 10-bit long NRI bits are masked out from the P-TMSI and also supplied to the RAN node as IDNNS (Intra Domain NAS Node Selector). However, the RAN configured NRI length should not exceed 8 bits.

[TS 23.003, clause 2.8.2.2 “Mapping from RAI and P-TMSI to GUTI”]

The mapping of P-TMSI (TLLI) and RAI in GERAN/UTRAN to GUTI in E-UTRAN shall be performed as follows:

GERAN/UTRAN <MCC> maps to E-UTRAN <MCC>

GERAN/UTRAN <MNC> maps to E-UTRAN <MNC>

GERAN/UTRAN <LAC> maps to E-UTRAN <MME Group ID>

GERAN/UTRAN <RAC> maps into bit 23 and down to bit 16 of the M-TMSI

The 8 most significant bits of GERAN/UTRAN <NRI> map to the MME code.

GERAN/UTRAN <P-TMSI> maps as follows:

-	6 bits of the GERAN/UTRAN <P-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the E-UTRAN <M-TMSI>;

-	16 bits of the GERAN/UTRAN <P-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the E-UTRAN <M-TMSI>.

…

[TS 23.401, clause 4.3.5.6, “Idle mode signalling reduction function”]

The TIN can take one of the three values, "P-TMSI", "GUTI" or "RAT-related TMSI". The UE shall set the TIN when receiving an Attach Accept, a TAU Accept or RAU Accept message according to the rules in table 4.3.5.6-1.

Table 4.3.5.6-1: Setting of the TIN

| Message received by UE | Current TIN value stored by UE | TIN value to be set by the UE when receiving message |
| --- | --- | --- |
| Attach Accept via E-UTRAN (never indicates ISR activation) | Any value | GUTI |
| Attach Accept via GERAN/UTRAN (never indicates ISR activation) | Any value | P-TMSI |
| TAU Accept not indicating ISR | Any value | GUTI |
| TAU Accept indicating ISR | GUTI P-TMSI or RAT-related TMSI | GUTI RAT-related TMSI |
| RAU Accept not indicating ISR | Any value | P-TMSI |
| RAU Accept indicating ISR | P-TMSI GUTI or RAT-related TMSI | P-TMSI RAT-related TMSI |

[TS 23.401, clause 5.3.3.1, step 2, “Tracking Area Update procedure with Serving GW change”]

….

If the UE's TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and related RAI then these two elements are indicated as the old GUTI.

…

[TS 23.401, clause 5.3.3.3, step 2, “Routeing Area Update with MME interaction and without S-GW change”]

…

If the UE's internal TIN indicates "GUTI" and the UE holds a valid GUTI then the UE indicates the GUTI as the old P-TMSI and old RAI

…

[TS 24.008, clause 4.7.1.4.1, “Radio resource sublayer address handling (A/Gb mode only)”]

For an MS supporting S1 mode, the following five cases can be distinguished:

a)	the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and a RAI;

b)	the TIN indicates "GUTI" and the MS holds a valid GUTI;

c)	the TIN is deleted and the UE holds a valid P-TMSI and RAI;

d)	the TIN is deleted and the UE holds a valid GUTI, but no valid P-TMSI and RAI; or

e)	none of the previous cases is fulfilled.

In case a) the MS shall derive a foreign TLLI from the P-TMSI and proceed as specified for case i) above.

In case b), the MS shall derive a P-TMSI from the GUTI and then a foreign TLLI from this P-TMSI and proceed as specified for case i) above.

…

[TS 24.008, clause 4.7.5.1.1, “Normal and periodic routing area updating procedure initiation”]

To initiate the normal routing area updating procedure, the MS sends the message ROUTING AREA UPDATE REQUEST to the network, starts timer T3330 and changes to state GMM-ROUTING-AREA-UPDATING-INITIATED.

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

-	If the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and RAI, the MS shall indicate the RAI in the Old routing area identification IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the P-TMSI in the P-TMSI IE.

If the routing area updating procedure is not initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode intersystem change, the message ROUTING AREA UPDATE REQUEST shall contain the P-TMSI signature when received within a previous ATTACH ACCEPT or ROUTING AREA UPDATE ACCEPT message.

If the routing area updating procedure is initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode and the MS does not have a valid current UMTS security context, the message ROUTING AREA UPDATE REQUEST shall include a P-TMSI signature filled with a NAS token as specified in 3GPP TS 33.401 [119]. If the MS has a valid current UMTS security context, the MS shall indicate it in the GPRS ciphering key sequence number IE.

NOTE:	If the TIN indicates "GUTI", 8 bits of the NAS token will be filled with bits from the M-TMSI (see 3GPP TS 23.003 [4]).

If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in connected mode or in idle mode if the MS does not have a valid current security context, the MS shall derive CK' and IK' from the KASME and the NAS downlink COUNT value corresponding to the NAS token derived as specified in 3GPP TS 33.401 [119]. The MS shall indicate the eKSI value in the CKSN field of the GPRS ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message. Then, the MS shall reset the START value and store the mapped UMTS security context replacing the current UMTS security context.

…

In order to indicate the new DRX parameter while in GERAN or UTRAN coverage, the MS shall send the ROUTING AREA UPDATE REQUEST message containing the DRX parameter in the DRX parameter IE to the network, with the exception of the case if the MS had indicated its UE specific DRX parameter (3GPP TS 24.301 [120]) to the network while in E-UTRAN coverage. In this case, when the MS enters GERAN or UTRAN coverage and initiates a routing area updating procedure, the MS shall not include the DRX parameter in the DRX parameter IE in the ROUTING AREA UPDATE REQUEST message.

[TS 24.008, clause 4.7.5.1.3, “Normal and periodic routing area updating procedure accepted by the network”]

If the MS has indicated in the ROUTING AREA UPDATE REQUEST message that it supports PS inter-RAT HO from GERAN to E-UTRAN, the network may include in the ROUTING AREA UPDATE ACCEPT message a request to provide the E-UTRAN inter RAT information container.

[TS 24.301, clause 5.3.1.1 “Establishment of the NAS signalling connection”]

For the routing of the initial NAS message to the appropriate MME, the UE NAS provides the lower layers with either the S-TMSI or the registered globally unique MME identifier (GUMMEI) that consists of the PLMN ID, the MME group ID, and the MME code (see 3GPP TS 23.003 [2]).

-	When the UE is registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the S-TMSI, but shall not provide the registered MME identifier to the lower layers. Exceptionally, when the UE in EMM-IDLE mode initiates a tracking area updating procedure for load balancing purposes, the UE NAS shall provide the lower layers with neither S-TMSI nor registered MME identifier.

-	When the UE is not registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS does not provide the lower layers with the S-TMSI. If the UE has a valid registered MME identifier from a previous registration, the UE NAS shall provide the lower layers with the registered MME identifier.

[TS 24.301, clause 5.5.3.2.2 “Normal and periodic tracking area updating procedure initiation”]

…

In order to indicate its UE specific DRX parameter while in E-UTRAN coverage, the UE shall send the TRACKING AREA UPDATE REQUEST message containing the UE specific DRX parameter in the DRX parameter IE to the network, with the exception of the case if the UE had indicated its DRX parameter (3GPP TS 24.008 [13]) to the network while in GERAN or UTRAN coverage. In this case, when the UE enters E-UTRAN coverage and initiates a tracking area updating procedure, the UE shall not include the UE specific DRX parameter in the DRX parameter IE in the TRACKING AREA UPDATE REQUEST message.

…

When initiating a tracking area updating procedure as a result of an Iu mode to S1 mode or A/Gb mode to S1 mode inter-system change, the UE shall handle the GUTI as follows:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the old GUTI IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	Mapping the P-TMSI and RAI to the GUTI is specified in Annex H of 3GPP TS 23.401 [10].

-	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

…

When the tracking area updating procedure is initiated to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSIASME in the TRACKING AREA UPDATE REQUEST message if the UE has a cached EPS security context. Otherwise, the UE shall set the KSIASME to the value "no key is available".

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSISGSN in the TRACKING AREA UPDATE REQUEST message. If the UE does not have a cached EPS security context, the UE shall include the NonceUE IE in the TRACKING AREA UPDATE REQUEST message. The TRACKING AREA UPDATE REQUEST message shall be integrity protected with the cached EPS security context if the UE has one. If the UE does not have a cached EPS security context, the TRACKING AREA UPDATE REQUEST message shall not be integrity protected.

…

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSISGSN in the TRACKING AREA UPDATE REQUEST message. If the UE does not have a cached EPS security context, the UE shall include the NonceUE IE in the TRACKING AREA UPDATE REQUEST message. The TRACKING AREA UPDATE REQUEST message shall be integrity protected with the cached EPS security context if the UE has one. If the UE does not have a cached EPS security context, the TRACKING AREA UPDATE REQUEST message shall not be integrity protected.

…

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

…

[TS 24.301, clause 4.4.2 “Handling of EPS security contexts”]

…

The key set identifier eKSI is assigned by the MME either during the authentication procedure or, for the mapped security context, during the handover procedure. The eKSI consists of a value and a type of security context parameter indicating whether an EPS security context is a native EPS security context or a mapped EPS security context. When the EPS security context is a native EPS security context, the eKSI has the value of KSIASME, and when the current EPS security context is a mapped EPS security context, the eKSI has the value of KSISGSN.

…

[TS 24.301, clause 5.3.1.1 “Establishment of the NAS signalling connection”]

b)	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE NAS shall provide the lower layers with the MME identifier part of the mapped GUTI, which is generated from the P-TMSI and RAI.

9.2.3.4.1.3	Test description

9.2.3.4.1.3.1	Pre-test conditions

System Simulator:

NOTE: 	while this test describes the uses of 3 cells, it is intended that this test only requires 2 cells to be active at any one instant.

-	cell A (E-UTRAN), cell B (E-UTRAN), cell 24 and cell 26 (GERAN)

-	with the exception of the MCC and MNC, cell 24 and cell 26 take the default parameters of the GERAN cells as defined in clause 6.3.1 of TS 36.508 [18], cells 24 and 26 are configured with the same MCC and MNC as cell A. Cells 24 and 26 are in the same routing area;

-	as defined in clause 40.1 of TS 51.010 [23], the GERAN cells are configured to use Network Mode of Operation I;

-	the power level of Cell 24 is the Serving Cell level;

-	the power level of Cell A is set to the Non-suitable level;

-	the power level of Cell B is set to the Non-suitable" Off" level;

-	the power level of Cell 26 is set to the Non-suitable Off.

-	System information combination 5 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE was previously registered on E-UTRAN cell B, and when on E-UTRAN, the UE was last authenticated and registered on a cell (no name assigned) frequency 2 belonging to a PLMN with (MCC, MNC) = (65, 987) and allocated a GUTI with (MMEGI, MMEC) = (0xfedc, 0xba) and an arbitrary M-TMSI value.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.4.1.3.2	Test procedure sequence

Table 9.2.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message on Cell 24 | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message | <-- | SECURITY MODE COMMAND | - | - |
| 6 | The UE transmits a NAS SECURITY MODE COMPLETE message ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 5 | --> | SECURITY MODE COMPLETE | - | - |
| 8 | SS responds with ATTACH ACCEPT message including valid TMSI, P-TMSI (TIN set to P-TMSI) and RAI. Editor’s Note: the default message contents in 51.010 clause 40.2.4.3 allocate a Negotiated READY timer value of 32 seconds. | <-- | ATTACH ACCEPT | - | - |
| 9 | The UE transmits an ATTACH COMPLETE message. | - | ATTACH COMPLETE | - | - |
| 9AA | Exception: if pc_IMS = TRUE then the generic procedure specified in TS 36.508 4.5A.3B is performed. NOTE: The SS rejects any PDP context request to prevent IMS signalling on the GERAN cell. | - | - | - | - |
| 9A | The activation of a PDP context is triggered by MMI or AT command. | - | - | - | - |
| 9B | The UE transmits an ACTIVATE PDP CONTEXT REQUEST message | --> | ACTIVATE PDP CONTEXT REQUEST | - | - |
| 9C | The SS responds with an ACTIVATE PDP CONTEXT ACCEPT message | <-- | ACTIVATE PDP CONTEXT ACCEPT | - | - |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | The signal strength of Cell A is raised to that of the Serving Cell and that of Cell 24 is lowered to that of a Suitable Neighbour Cell. Note: Cell 24 is still suitable but the UE is expected to select Cell A. | - | - | - | - |
| 14 | Void | - | - | - | - |
| 15 | Check: Does the UE send an RRCConnectionRequest with the ue-Identity set to “randomValue” and the establishmentcause set to MO-signalling on Cell A? Editor’s Note: Cell reselection to E-UTRAN might not occur until the GPRS READY timer has expired. Hence this step might occur up to 40 seconds after step 13. | --> | RRCConnectionRequest | 1 | P |
| 16 | The SS responds with RRCConnectionSetup | <-- | RRCConnectionSetup |  |  |
| 17 | Check: Does the UE send an RRCConnectionSetupComplete with the mmegi and mmec set to the values derived from the mapped RAI and P-TMSI | --> | RRCConnectionSetupComplete(RRC parameters, TRACKING AREA UPDATE REQUEST) | 1 | P |
|  | Check: Are the contents of the TRACKING AREA UPDATE REQUEST with the correct parameters? |  |  | 2, 3 |  |
| 18 | The SS sends TRACKING AREA UPDATE ACCEPT Note: the default message contents cause the allocation of a new GUTI and new TAI list | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 19 | The UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 20 | The SS releases the RRC connection | - | - | - | - |
| - | Exception: Steps 20A to 20Aa4/20Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 20A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 20Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | -> | SERVICE REQUEST | - | - |
| 20Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 20Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 20Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.32 steps 4-11. | - | - | - | - |
| 20Aa5 | The SS releases the RRC connection | - | - | - | - |
| 20Ab1 | The 10s timer expires | - | - | - | - |
| 21 | Cell 24 is switched off | - | - | - | - |
| 22 | The signal strength of Cell 26 is raised to that of the Serving Cell and that of Cell A is lowered to that of a Non suitable Neighbour Cell. Note: Cell A is still suitable but the UE is expected to select Cell 26. | - | - | - | - |
| 23 | Check: Does the UE send on Cell 26 an RLC/MAC header with the TLLI derived from the P-TMSI that is derived from the GUTI allocated in step 18? | - | - | 4 | P |
| 24 | Check: Does the UE send on Cell 26 a ROUTING AREA UPDATE REQUEST with the correct parameters? | --> | ROUTING AREA UPDATE REQUEST | 5 | P |
| 24A | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 24B | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 25 | The SS sends ROUTING AREA UPDATE ACCEPT (without the allocated P-TMSI IE). | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| - | EXCEPTION: Step 26 describes behaviour that depends on the UE capability. | - | - | - | - |
| 26 | IF pc_GERAN_2_E_UTRAN_PSHO, then the UE sends ROUTING AREA UPDATE COMPLETE message | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 27 | The signal strength of Cell A is raised to that of the Serving Cell and that of Cell 26 is lowered to that of a Suitable Neighbour Cell. Note: Cell 26 is still suitable but the UE is expected to select Cell A. | - | - | - | - |
| 27A | Check: Does the UE send an RRCConnectionRequest with the ue-Identity set for Rel-8 to Rel-11 UE to s-TMSI or randomValue, and, for Rel-12 and onwards UE set to randomValue? | --> | RRCConnectionRequest | 7 | P |
| 27B | The SS responds with RRCConnectionSetup | <-- | RRCConnectionSetup |  |  |
| 28 | Check: Does the UE send on Cell A an RRCConnectionSetupComplete without the registeredMME when s-TMSI is included in step 27A, otherwise with the registeredMME and the mapped GUMMEI? | --> | RRCConnectionSetupComplete(RRC parameters, TRACKING AREA UPDATE REQUEST) | 7 | P |
| - | Check: Does the UE send a TRACKING AREA UPDATE REQUEST with the correct parameters? |  |  | 2, 6 |  |
| 29 | The SS sends TRACKING AREA UPDATE ACCEPT Note: the default message contents cause the allocation of a new GUTI. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 30 | The UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.4.1.3.3	Specific message contents

Table 9.2.3.4.1.3.3-1: Message RRCConnectionRequest (step 15, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialUE-Identity CHOICE { |  |  |  |
| random-Value | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |

Table 9.2.3.4.1.3.3-2: Message RRCConnectionSetupComplete (step 17, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | Indicates the PLMN of cell A. |  |  |
| registeredMME SEQUENCE { |  |  |  |  |
| plmn-Identity |  | Not Checked |  |  |
| Mmegi |  | LAC sent to the UE in step 8 |  |  |
| Mmec |  | Bit 23 to bit 16 of P-TMSI sent to the UE in step 8 |  |  |
| } |  |  |  |  |
| nas-DedicatedInformation |  | Not checked at RRC layer |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.4.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 17, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Sent in SECURITY PROTECTED NAS MESSAGE with valid integrity check |  |  |  |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating | TA_only |
|  | '001'B or ‘010’B | Combined TA/LA updating or combined TA/LA updating with IMSI attach | No_LAU_CombinedRAU_GERAN_UTRA |
|  | ‘010’B | Combined TA/LA updating with IMSI attach | LAU_CombinedRAU_GERAN_UTRA |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The valid NAS key set identifier KSIASME of the UE | As stored on the USIM in EFEPSNSC |  |
| TSC | '0'B | native security context |  |
| Old GUTI | Mapped from the P-TMSI and RAI allocated in step 8 |  |  |
| Additional GUTI | GUTI1 | Set to the value stored in the USIM in EFEPSLOCI |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | ‘1’B | UE radio capability information update needed |  |
| GPRS ciphering key sequence number | Value mapped from the AUTHENTICATION AND CIPHERING REQUEST message |  |  |
| Old Ptmsi Signature | Any Value |  |  |
| Nonce | Any Value |  |  |

Table 9.2.3.4.1.3.3-3A: Message TRACKING AREA UPDATE REQUEST Conditions (Table 9.2.3.4.1.3.3-3)

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below 36.508 table 4.7.2-24. |
| No_LAU_CombinedRAU_GERAN_UTRA | UE has not performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |
| LAU_CombinedRAU_GERAN_UTRA | UE has performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |

Table 9.2.3.4.1.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 24, Table 9.2.3.4.1.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| - Protocol discriminator | GMM |  |  |
| Skip indicator | ‘0000’ |  |  |
| Routing area update request message identity | ‘0000 1000’ B |  |  |
| Update type | Any allowed value |  |  |
| GPRS ciphering key sequence number | set to the value to KSI-ASME | With ISR inactive and TIN indicates GUTI, then eKSI value is set to KSI-ASME |  |
| Old routing area identification | Mapped from the GUTI received in step 18. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Mapped from the GUTI received in step 18. |  |  |
| Requested READY timer value | If present, any allowed value. |  |  |
| DRX parameter | Not present | Shall be absent |  |
| TMSI status | If present, any allowed value. |  |  |
| P-TMSI | Not present |  |  |
| MS network capability | Any allowed value. |  |  |
| PDP context status | If present, any allowed value. |  |  |
| PS LCS Capability | If present, any allowed value. |  |  |
| MBMS context status | If present, any allowed value. |  |  |
| UE network capability | Any allowed value. |  |  |
| Additional mobile identity | Set to the P-TMSI allocated in step 8 |  |  |
| Additional old routing area identification | Set to the RAI allocated in step 8 |  |  |
| Mobile station classmark 2 | If present, any allowed value. |  |  |
| Mobile station classmark 3 | If present, any allowed value. |  |  |
| Supported Codecs | If present, any allowed value. |  |  |

Table 9.2.3.4.1.3.3-5: Message ROUTING AREA UPDATE ACCEPT (step 25, Table 9.2.3.4.1.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| allocated P-TMSI | Absent |  |  |
| PDP context status | Same value as received in ROUTING AREA UPDATE REQUEST in step 24 |  |  |

Table 9.2.3.4.1.3.3-6: Void

Table 9.2.3.4.1.3.3-6A: Message RRCConnectionRequest (step 27A, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ue-Identity CHOICE { |  |  |  |
| s-TMSI | Received from NAS layer | Rel-8 to Rel-11 inclusive |  |
| randomValue | Any allowed value | Rel-8 and onwards |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |

Table 9.2.3.4.1.3.3-7: Message RRCConnectionSetupComplete (step 28, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| Rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | Indicates the PLMN of cell 24. |  |  |
| registeredMME |  | Not present | Shall be absent if the upper layers provided S-TMSI in RRCConnectionRequest message in step 27A for pre-Rel-12 UE |  |
| registeredMME |  | generated from the P-TMSI and RAI | Present only when the InitialUE-Identity in step 27A is set to randomValue |  |
| nas-DedicatedInformation |  | Not checked at RRC layer |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| gummei-Type-r10 |  | mapped | Present when the registeredMME is included, |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.4.1.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 28, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Sent in SECURITY PROTECTED NAS MESSAGE with valid integrity check |  |  |  |
| EPS update type | 000 | TA updating | TA_only |
|  | 010 | combined TA/LA updating with IMSI attach | combined_TA_LA |
| EPS bearer context status | (octet 3 = '00100000'B and octet 4 = '00000000'B) or (octet 3 = '00100000'B and octet 4 = '00010000'B) |  |  |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The valid NAS key set identifier KSIASME of the UE | As stored on the USIM in EFEPSNSC in the pre-test conditions |  |
| TSC | '0'B | native security context |  |
| Old GUTI | Mapped from the P-TMSI allocated in Step 8 and RAI allocated in |  |  |
| Additional GUTI | GUTI1 | Set to the value allocated in step 18 |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | Not present |  |  |
| Old Ptmsi Signature | Any Value |  |  |
| Nonce | Any Value |  |  |

### 9.2.4	Attach and Tracking area updating procedures

#### 9.2.4.1	eDRX

##### 9.2.4.1.1	Attach & Normal tracking area update Procedure / Success / without Idle eDRX parameters / With Idle eDRX parameters

9.2.4.1.1.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message with extended DRX parameters IE}

ensure that {

when { the UE receives the extended DRX parameters in the ATTACH ACCEPT message }

then { the UE sends an ATTACH COMPLETE message }

}

(2)

with { the UE has applied extended DRX parameters received in the ATTACH ACCEPT message }

ensure that {

when { the UE receives the Paging message in a valid paging occasion in paging hyperframe as per idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

(3)

with { the UE has sent a TAU EQUESTR message with extended DRX parameters IE }

ensure that {

when { the UE does not receive the extended DRX parameters in the TAU ACCEPT message }

then { the UE sends a TAU COMPLETE message }

}

(4)

with { the UE has not received extended DRX parameters in the TAU ACCEPT message }

ensure that {

when { the UE receives the Paging message in a valid paging occasion as per normal DRX }

then { the UE sends the SERVICE REQUEST message }

}

(5)

with { the U E has sent an ATTACH REQUEST message with extended DRX parameters IE }

ensure that {

when { the UE does not receive the extended DRX parameters in the ATTACH ACCEPT message }

then { the UE sends an ATTACH COMPLETE message}

}

(6)

with { the UE has not received extended DRX parameters in the ATTACH ACCEPT message }

ensure that {

when { the UE receives the Paging message in a valid paging occasion as per normal DRX }

then { the UE sends a SERVICE REQUEST message}

}

(7)

with { the UE has sent a TAU REQUEST message with extended DRX parameters IE }

ensure that {

when { the UE receives the extended DRX parameters in the TAU ACCEPT message }

then { the UE sends a TAU COMPLETE message}

}

(8)

with { the UE has applied the extended DRX parameters received in the TAU ACCEPT message }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per Idle eDRX }

then { the UE sends a SERVICE REQUEST message}

}

9.2.4.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.12, 5.5.1.2.2, 5.5.1.2.4, 5.5.3.2.2, 5.5.3.2.4 and 5.6.2.2.1.1

[TS 24.301 clause5.3.12]

The UE may request the use of extended idle-mode DRX cycle (eDRX) during an attach or tracking area updating procedure by including the extended DRX parameters IE (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of eDRX during:

…

The network accepts the request to use the eDRX by providing the extended DRX parameters IE when accepting the attach or the tracking area updating procedure. The UE shall use eDRX only if the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure.

NOTE:	If the UE wants to keep using eDRX, the UE includes the extended DRX parameters IE in each attach or tracking area updating procedure.

[TS 24.301 clause5.5.1.2.2]

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

[TS 24.301 clause5.5.1.2.4]

The MME shall include the extended DRX parameters IE in the ATTACH ACCEPT message only if the extended DRX parameters IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

- when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

u)	when the UE needs to request the use of eDRX or needs to stop the use of eDRX;

v)	when a change in the eDRX usage conditions at the UE requires different extended DRX parameters;

…

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause5.5.3.2.4]

The MME shall include the extended DRX parameters IE in the TRACKING AREA UPDATE ACCEPT message only if the extended DRX parameters IE was included in the TRACKING AREA UPDATE REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause5.6.2.2.1.1]

To initiate the procedure the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [20], 3GPP TS 36.413 [23]) and shall start the timer:

-	T3415 for this paging procedure, if the network accepted to use eDRX for the UE.

9.2.4.1.1.3	Test description

9.2.4.1.1.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE messages).

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.4.1.1.3.2	Test procedure sequence

Table 9.2.4.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE | --> | ATTACH REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 12 | SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (NOTE 1) THEN in parallel to the event described in steps 13 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 1 | P |
| 13A1a1-13A3b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 14 | SS transmits a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 15 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 2 | P |
| 16-19 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 20 | The SS releases the RRC connection. | - | - | - | - |
| 21 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the "Serving cell" | - | - | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE REQUEST message including an Extended DRX parameters IE | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 23 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without an Extended DRX parameters IE. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 24 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 3 | P |
| 25 | The SS releases the RRC connection. | - | - | - | - |
| 26 | SS transmits a Paging message to the UE in a valid PO as per normal DRX | <-- | Paging |  |  |
| 27 | Check: Does the UE respond to Paging according to procedure TS 36.508 6.4.2.2-1 Steps 2 to 11? | - | - | 4 | P |
| 28 | If possible (see ICS) switch off is performed or otherwise the power is removed | - | - | - | - |
| - | EXCEPTION: Step 29 describes behaviour that depends on the UE capability. | - | - | - | - |
| 29 | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message. | --> | DETACH REQUEST |  |  |
| 30 | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non-Suitable cell". | - | - | - | - |
| 31 | The UE is switched on | - | - | - | - |
| 32 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE. | --> | ATTACH REQUEST | - | - |
| 33-41 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 42 | SS responds with ATTACH ACCEPT message without including an Extended DRX parameters IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. |  | - | - | - |
| -- | EXCEPTION: IF PDN1_IMS (NOTE 1) THEN in parallel to the event described in steps 13 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 43 | Check: Does the UE transmit an ATTACH COMPLETE message message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 5 | P |
| 43A1a1-43A3b1 | Steps16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 44 | SS transmits a Paging message to the UE in a validPO as per normal DRX | <-- | Paging | - | - |
| 45 | Check: Does the UE respond to Paging according to procedure TS 36.508 6.4.2.2-1 Steps 2 to 11? | - | - | 6 | P |
| 46 | Set the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the "Serving cell" | - | - | - | - |
| 47 | The UE transmits a TRACKING AREA UPDATE REQUEST message including an Extended DRX parameters IE | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 48 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including an Extended DRX parameters IE. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 49 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 7 | P |
| 50 | The SS releases the RRC connection. | - | - | - | - |
| 51 | SS transmits a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging |  |  |
| 52 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 8 | P |
| 53-56 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 57 | The SS releases the RRC connection. | - | - | - | - |
| NOTE 1: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.4.1.1.3.3	Specific message contents

Table 9.2.4.1.1.3.3-1: SystemInformationBlockType1 for Cell A and Cell B

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.1.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell A and Cell B when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.1.3.3-2: Message ATTACH REQUEST (step 2 in Table 9.2.4.1.1.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.1.3.3-3: Message ATTACH ACCEPT (step 12 in Table 9.2.4.1.1.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 seconds |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |

Table 9.2.4.1.1.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 22 in Table 9.2.4.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.1.3.3-5: Message TRACKING AREA UPDATE Accept (step 23 in Table 9.2.4.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters | Not Present |  |  |

Table 9.2.4.1.1.3.3-6: Message ATTACH REQUEST (step 32 in Table 9.2.4.1.1.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.1.3.3-7: Message ATTACH ACCEPT (step 42 in Table 9.2.4.1.1.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters | Not Present |  |  |

Table 9.2.4.1.1.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 47 in Table 9.2.4.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.1.3.3-9: Message TRACKING AREA UPDATE Accept (step 48 in Table 9.2.4.1.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 seconds |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |

##### 9.2.4.1.2	Attach & Normal tracking area update Procedure / Success / With and without Idle eDRX and PSM parameters

9.2.4.1.2.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message including the extended DRX and T3324 IE parameters }

then { the UE sends an ATTACH COMPLETE message }

}

(2)

with { the UE has applied extended DRX parameters received in the ATTACH ACCEPT message }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

(3)

with { the UE has sent a TAU REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the TAU ACCEPT message not including the extended DRX parameters but including the T3324 IE }

then { the UE sends a TAU COMPLETE message }

}

(4)

with { the UE has received a TAU ACCEPT message not including extended DRX parameters IE but including T3324 IE }

ensure that {

when { the UE receives a Paging message after T3324 timer expiry in a valid paging occasion as per normal DRX }

then { the UE does not send the SERVICE REQUEST message }

}

(5)

with { the UE has sent an ATTACH REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message including the extended DRX parameters but not the T3324 IE }

then { the UE sends an ATTACH COMPLETE message }

}

(6)

with { the UE has received an ATTACH ACCEPT message including extended DRX parameters IE but not the T3324 IE }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per Idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

9.2.4.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.11, 5.3.12, 5.3.13, 5.5.1.2.2, 5.5.1.2.4, 5.5.3.2.2, 5.5.3.2.4 and 5.6.2.2.1.1

[TS 24.301 clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

[TS 24.301 clause 5.3.12]

The UE may request the use of extended idle-mode DRX cycle (eDRX) during an attach or tracking area updating procedure by including the extended DRX parameters IE (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of eDRX during:

…

The network accepts the request to use the eDRX by providing the extended DRX parameters IE when accepting the attach or the tracking area updating procedure. The UE shall use eDRX only if the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure.

NOTE:	If the UE wants to keep using eDRX, the UE includes the extended DRX parameters IE in each attach or tracking area updating procedure.

[TS 24.301 clause 5.3.13]

The UE can request the use of both PSM and eDRX during an attach or tracking area update procedure but it is up to the network to decide to enable none, one of them or both (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]).

If the network accepts the use of both PSM (see subclause 5.3.11) and eDRX (see subclause 5.3.12), the extended DRX parameters IE provided to the UE should allow for multiple paging occasions before the active timer expires.

[TS 24.301 clause 5.5.1.2.2]

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

[TS 24.301 clause 5.5.1.2.4]

The MME shall include the extended DRX parameters IE in the ATTACH ACCEPT message only if the extended DRX parameters IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

- when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

u)	when the UE needs to request the use of eDRX or needs to stop the use of eDRX;

v)	when a change in the eDRX usage conditions at the UE requires different extended DRX parameters;

…

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause 5.5.3.2.4]

The MME shall include the extended DRX parameters IE in the TRACKING AREA UPDATE ACCEPT message only if the extended DRX parameters IE was included in the TRACKING AREA UPDATE REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause 5.6.2.2.1.1]

To initiate the procedure the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [20], 3GPP TS 36.413 [23]) and shall start the timer:

-	T3415 for this paging procedure, if the network accepted to use eDRX for the UE.

9.2.4.1.2.3	Test description

9.2.4.1.2.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is configured to request the use of  eDRX  (in the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages).

-	The UE is configured to use Power Saving Mode

-	The UE is configured to set T3324 as two minutes

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.4.1.2.3.2	Test procedure sequence

Table 9.2.4.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and T3324 IE set to two minutes | --> | ATTACH REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 12 | SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE and the T3324 IE set to two minutes. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (NOTE 1) then in parallel to the event described in steps 13 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 1 | P |
| 14a1-16b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 17 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 18 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 2 | P |
| 19-22 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 23 | The SS releases the RRC connection. | - | - | - | - |
| 24 | SS sets the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the "Serving cell" | - | - | - | - |
| 25 | The UE transmits a TRACKING AREA UPDATE REQUEST message including an Extended DRX parameters IE and the T3324 IE set to two minutes | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 26 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without an Extended DRX parameters IE but with the T3324 IE set to 2 minutes. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 27 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 3 | P |
| 28 | The SS releases the RRC connection. | - | - | - | - |
| 29 | When the T3324 timer expires the SS sends a Paging message including a matched identity in a valid PO as per normal DRX | <-- | Paging | - | - |
| 30 | Check: Does the UE respond to the Paging message? | - | - | 4 | F |
| 31 | If possible (see ICS) the UE is switched off or otherwise the power is removed | - | - | - | - |
| - | EXCEPTION: Step 32a1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 32a1 | If pc_SwitchOnOff or pc_USIM_Removal then UE send DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 33 | SS sets the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non-Suitable cell". | - | - | - | - |
| 34 | The UE is switched on | - | - | - | - |
| 35 | The UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and the T3324 IE set to two minutes | --> | ATTACH REQUEST | - | - |
| 36-44 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 45 | SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE but without the T3324 IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 46 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 5 | P |
| 47a1-49b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 50 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 51 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 6 | P |
| 52-55 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 56 | The SS releases the RRC connection. | - | - | - | - |
| NOTE 1: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.4.1.2.3.3	Specific message contents

Table 9.2.4.1.2.3.3-1: SystemInformationBlockType1 for Cell A and Cell B

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.2.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell A and Cell B when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.2.3.3-2: Message ATTACH REQUEST (step 2)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 | ’00100010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.4.1.2.3.3-3: Message ATTACH ACCEPT (step 12)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 second |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |
| T3324 | ’00100010’B | 2 minutes |  |
| T3412 extended value | Value received in ATTACH REQUEST | Included only if the IE 'T3412 extended value' was provided in the ATTACH REQUEST |  |

Table 9.2.4.1.2.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 25)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 value | ’00100010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.4.1.2.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 26)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3324 value | ’00100010’B | 2 minutes |  |
| T3412 extended value | Value received in TRACKING AREA UPDATE REQUEST | Included only if the IE 'T3412 extended value' was provided in the TRACKING AREA UPDATE REQUEST |  |

Table 9.2.4.1.2.3.3-6: Message ATTACH REQUEST (step 35)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 value | ’00100010’B | 2 minutes |  |

Table 9.2.4.1.2.3.3-7: Message ATTACH ACCEPT (step 45)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 seconds |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |
| T3324 | Not Present |  |  |

##### 9.2.4.1.3	Attach & Normal tracking area Procedure / Success / Emergency Calls/ without Idle eDRX parameters / With Idle eDRX parameters

9.2.4.1.3.1	Test Purpose (TP)

(1)

with { UE in EMM-DEREGISTERED state}

ensure that {
  when { the UE initiates an Attach Procedure for emergency bearer services }

then { the UE sends an ATTACH REQUEST message without including the Extended DRX parameters IE }

}

(2)

with { UE attached for emergency bearer services }

ensure that {
  when { the UE detects entering a new Tracking Area that is not included in the TAI list }

then { the UE sends a TAU REQUEST message without including the  Extended DRX parameters IE }

}

(3)

with { the UE resumed eDRX after PDN disconnection for emergency bearer service }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per Idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

(4)

with { the UE resumed eDRX after PDN disconnection for emergency bearer service }

ensure that {

when { the UE detects entering a new Tracking Area not included in the TAI list }

then { the UE sends a TAU REQUEST message with the extended DRX parameters IE }

}

9.2.4.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.12.

[TS 24.301, clause 5.3.12]

The UE may request the use of extended idle-mode DRX cycle (eDRX) during an attach or tracking area updating procedure by including the extended DRX parameters IE (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of eDRX during:

-	an attach for emergency bearer services procedure; or

-	a tracking area updating procedure for the UE attached for emergency bearer services.

The UE and the network may negotiate eDRX parameters during a tracking area updating procedure when the UE has a PDN connection for emergency bearer services.

The network accepts the request to use the eDRX by providing the extended DRX parameters IE when accepting the attach or the tracking area updating procedure. The UE shall use eDRX only if the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure and the UE does not have a PDN connection for emergency bearer services. If the network did not accept the request to use eDRX, or if the UE has a PDN connection for emergency bearer services, the UE and the network shall use the stored UE specific DRX cycle, if any, instead of eDRX.

NOTE:	If the UE wants to keep using eDRX, the UE includes the extended DRX parameters IE in each attach or tracking area updating procedure.

While the UE has a PDN connection for emergency bearer services established, the UE shall not use eDRX. If the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure, upon successful completion of the PDN disconnect procedure of the PDN connection for emergency bearer services or EPS bearer context deactivation procedure of the EPS bearer context for emergency, the UE and the network shall resume eDRX. If the UE or the network locally releases the PDN connection for emergency bearer service, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a tracking area updating procedure with EPS bearer context synchronization or upon successful completion of a service request procedure

9.2.4.1.3.3	Test description

9.2.4.1.3.3.1	Pre-test conditions

System Simulator:

-	Cell A belongs to TAI-1 and is set to ''Serving cell''

-	Cell B belongs to TAI-2 and is set to “Non-suitable off cell”

UE:

-	The UE is configured to request the use of eDRX (the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages).

Preamble:

-	UE sends an ATTACH REQUEST message with eDRX parameters in Cell A. SS sends an ATTACH REJECT message with EMM cause = "Tracking area not allowed". UE stores the TAI in the list of "forbidden tracking areas for regional provision of service" and enters EMM-DEREGISTERED.LIMITED-SERVICE state.

9.2.4.1.3.3.2	 Test procedure sequence

Table 9.2.4.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS emergency call | - | - | - | - |
| 2-3 | Steps 2 to 3 of the generic procedures in TS 36.508 subclause 4.5A.5.3 are performed in Cell A | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services without including “Extended DRX parameters” IE and including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST PDN CONNECTIVITY REQUEST | 1 | P |
| 5-19 | Steps 5 to 19 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 Table 4.5A.5.3-1). | - | - | - | - |
| 20 | Set the cell type of Cell A to the ''Non-Suitable cell''. Set the cell type of Cell B to the ''Suitable cell''. | - | - | - | - |
| 21 | Check: Does the UE send TRACKING AREA UPDATE REQUEST in Cell B without including Extended DRX parameters IE? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 22 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT |  |  |
| 23 | The UE transmit a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE |  |  |
| 24 | The SS releases the RRC connection | - | - | - | - |
| 25 | If possible (see ICS) switch off is performed. Otherwise the power is removed | - | - | - | - |
| - | EXCEPTION: Step 26 describes behaviour that depends on the UE capability. | - | - | - | - |
| 26 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST | --> | DETACH REQUEST | - | - |
| 27 | Set the cell type of cell A to “Suitable cell''. Set the cell type of cell B to ''Non-Suitable off cell'' | - | - | - | - |
| 28 | Power on the UE | - | - | - | - |
| 29-46 | Steps 2 to 18b1 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell A. | - | - | - | - |
| 47 | Cause the UE to request connectivity to an additional PDN for emergency bearer service | - | - | - | - |
| 48-61 | Steps 2 to 15 of the generic test procedure for IMS Emergency call establishment in EUTRA: in EUTRA: Normal Service (TS 36.508 Table 4.5A.4.3-1). | - | - | - | - |
| 62 | The SS waits for 5 seconds. | - |  |  |  |
| 63 | Release IMS Call (see Note 1). | - |  |  |  |
| 64 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST including the EPS bearer identity of the default EPS bearer to the additional PDN | - | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 65 | UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 66 | The SS releases the RRC connection | - | - | - | - |
| 67 | The SS waits 5 second | - | - | - | - |
| 68 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 69 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 3 | P |
| 70-72 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure | - | - | - | - |
| 73 | The SS releases the RRC connection. | - | - | - | - |
| 74 | Set the cell type of cell A to “Non-Suitable cell''. Set the cell type of cell B to ''Suitable cell'' | - | - | - | - |
| 75 | Check: Does the UE send TRACKING AREA UPDATE REQUEST including the Extended DRX parameters IE? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 76 | The SS transmits a TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 77 | The UE transmit a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 78 | SS releases RR the C Connection | - | - | - | - |
| Note 1: The IMS Call is released using the generic procedure in TS 34.229-1 [35] subclause C.32. |  |  |  |  |  |

9.2.4.1.3.3.3	Specific message contents

Table 9.2.4.1.3.3.3-1: SystemInformationBlockType1 for Cell A and Cell B

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.3.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell A and Cell B when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.3.3.3-2: Message ATTACH REQUEST (Preamble)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.3.3.3-2: Message ATTACH REQUEST (step 4 in Table 9.2.4.1.3.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach type | '0110'B | EPS emergency attach |  |
| Extended DRX parameters | Not Present |  |  |

Table 9.2.4.1.3.3.3-3: PDN CONNECTIVITY REQUEST (Step 4, Table 9.2.4.1.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | '0100'B | emergency |  |

Table 9.2.4.1.3.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 21 in Table 9.2.4.1.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters | Not Present |  |  |

Table 9.2.4.1.3.3.3-5: Message ATTACH REQUEST (step 31 in Table 9.2.4.1.3.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

Table 9.2.4.1.3.3.3-6: Message ATTACH ACCEPT (step 41 in Table 9.2.4.1.3.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 second |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |

Table 9.2.4.1.3.3.3-7: Message TRACKING AREA UPDATE REQUEST (step 75 in Table 9.2.4.1.3.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |

### 9.2.5	RACS

#### 9.2.5.1	RACS / Network assigned UE radio capability ID

9.2.5.1.1	Test Purpose (TP)

(1)

with { UE is powered on or switched on and has an applicable UE radio capability ID for the current UE radio configuration for the selected PLMN }

ensure that {

when { UE initiates an Attach procedure }

then { UE indicates the UE radio capability ID availability in ATTACH REQUEST message }

}

(2)

with { UE indicating UE radio capability ID availability in ATTACH REQUEST message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message requesting UE radio capability ID }

then { UE includes UE radio capability ID in SECURITY MODE COMPLETE message }

}

(3)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having assigned a new UE Radio Capability ID in the ATTACH ACCEPT message }

ensure that {

when { the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME }

then { UE sends TRACKING AREA UPDATE REQUEST message including the indication of UE radio capability ID availability }

}

(4)

with { UE indicating UE radio capability ID availability in TRACKING AREA UPDATE REQUEST message }

ensure that {

when { the UE receives a SECURITY MODE COMMAND message requesting UE radio capability ID }

then { UE includes UE radio capability ID in SECURITY MODE COMPLETE message }

}

(5)

with { UE having assigned a UE radio capability ID }

ensure that {

when { the UE enters a cell belonging to another PLMN }

then { UE does not include UE radio capability ID availability indication in TRACKING AREA UPDATE REQUEST message }

}

9.2.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.20.

[TS 24.301, clause 5.3.20]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.401 [10]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is not applicable to NB-S1 mode.

If the UE supports RACS:

-	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages;

-	if the UE performs an attach procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available";

-	if the UE performs a tracking area updating procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available";

-	If the UE is requested to provide the UE radio capability ID by the network during a security mode control procedure, the UE shall include the UE radio capability ID in the UE radio capability ID IE of the SECURITY MODE COMPLETE message according to the rules in clause 5.4.3.3.;

…

-	upon receiving a network-assigned UE radio capability ID in the ATTACH ACCEPT message in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message, the UE shall store the network-assigned UE radio capability ID and the PLMN ID of the serving network along with a mapping to the current UE radio configuration in its non-volatile memory as specified in annex C.

…

If the network supports RACS:

-	if the UE has included the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available", the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available", the network may initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE and no UE radio capability ID is available in the UE context in the MME, the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message; and

9.2.5.1.3	Test description

9.2.5.1.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

-	cell G (belongs to TAI-7, visited PLMN) is set to ''Non- Suitable cell''.

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.5.1.3.2	Test procedure sequence

Table 9.2.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A? | --> | ATTACH REQUEST | 1 | P |
| 3-4 | Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1? | --> | SECURITY MODE COMPLETE | 2 | P |
| 7-11 | Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2. | <-- | ATTACH ACCEPT | - | - |
| 13-16 | Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 17 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 18 | Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 19 | The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID. | <-- | SECURITY MODE COMMAND | - | - |
| 20 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2? | --> | SECURITY MODE COMPLETE | 4 | P |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 23 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on cell G. | - | - | - | - |
| 24 | The SS configures: - Cell B as the "Non-Suitable cell". - Cell G as a "Serving cell". | - | - | - | - |
| 25 | Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 5 | F |
| 26 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 27 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 28 | The SS releases the RRC connection. | - | - | - | - |

9.2.5.1.3.3	Specific message contents

Table 9.2.5.1.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) | '1'B | RACS supported |  |

Table 9.2.5.1.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.1.3.3-3: Message ATTACH REQUEST (step 2)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '001'B | UE radio capability ID available |  |

Table 9.2.5.1.3.3-4: Message SECURITY MODE COMPLETE (step 6)

| Derivation path: 36.508 table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.1.3.3-5: Message ATTACH ACCEPT (step 12)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.1.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 18)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | '000'B | TA updating |  |
| UE radio capability ID availability | '001'B | UE radio capability ID available |  |

Table 9.2.5.1.3.3-7: Message SECURITY MODE COMPLETE (step 20)

| Derivation path: 36.508 table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’ O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.1.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 25)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | '000'B | TA updating |  |
| UE radio capability ID availability | '001'B | UE radio capability ID not available |  |

#### 9.2.5.2	RACS / USIM change / Handling of URCID

9.2.5.2.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having assigned a UE Radio Capability ID in the ATTACH ACCEPT message }

ensure that {

when { UE is powered on or switched on with USIM having different IMSI }

then { UE does not include UE radio capability ID availability indication in ATTACH REQUEST message }

}

9.2.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.20, Annex C.

[TS 24.301, clause 5.3.20]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.401 [10]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is not applicable to NB-S1 mode.

If the UE supports RACS:

-	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages;

-	if the UE performs an attach procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available";

-	if the UE performs a tracking area updating procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available";

-	If the UE is requested to provide the UE radio capability ID by the network during a security mode control procedure, the UE shall include the UE radio capability ID in the UE radio capability ID IE of the SECURITY MODE COMPLETE message according to the rules in clause 5.4.3.3.;

…

-	upon receiving a network-assigned UE radio capability ID in the ATTACH ACCEPT message in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message, the UE shall store the network-assigned UE radio capability ID and the PLMN ID of the serving network along with a mapping to the current UE radio configuration in its non-volatile memory as specified in annex C.

…

If the network supports RACS:

-	if the UE has included the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available", the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available", the network may initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE and no UE radio capability ID is available in the UE context in the MME, the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message; and

[TS 24.301, Annex C]

The following EMM parameters shall be stored in a non-volatile memory in the ME together with the IMSI from the USIM:

-	TIN;

-	DCN-ID list; and

-	network-assigned UE radio capability IDs.

…

Each network-assigned UE radio capability ID is stored together with a PLMN identity of the PLMN that provided it as well as a mapping to the corresponding UE radio configuration, and is valid in that PLMN. A network-assigned UE radio capability ID can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory of the ME, else the UE shall delete the network-assigned UE radio capability ID.

9.2.5.2.3	Test description

9.2.5.2.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving Cell'';

UE:

-	The UE is equipped with a USIM containing default values (as per TS 36.508)

-	USIM A containing default values (as per TS 36.508) except for those listed in Table 9.2.5.2.3.1-1 will be used.

Table 9.2.5.2.3.1-1: USIM A Configuration

| USIM field | Value |
| --- | --- |
| EFIMSI | The HPLMN (MCC+MNC) of the IMSI is set to 001-11 |

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.5.2.3.2	Test procedure sequence

Table 9.2.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 2 | The UE is brought back to operation with USIM A inserted. | - | - | - | - |
| 3 | Check: Does the UE transmit an ATTACH REQUEST message without including UE radio capability availability indication? | --> | ATTACH REQUEST | 1 | P |
| 4-17 | The attach procedure is completed by executing steps 5 to 18 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |

9.2.5.2.3.3	Specific message contents

Table 9.2.5.2.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) | '1'B | RACS supported |  |

Table 9.2.5.2.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.2.3.3-3: Message ATTACH REQUEST (step 3)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '001'B | UE radio capability ID not available |  |

#### 9.2.5.3	RACS / Handling of delete indication for NW assigned UE radio capability ID

9.2.5.3.1	Test Purpose (TP)

(1)

with { UE having an applicable UE radio capability ID for the current UE radio configuration for the selected PLMN performs Attach procedure }

ensure that {

when { UE receives ATTACH ACCEPT message with network assigned UE radio capability ID deletion indication }

then { UE deletes any stored Network-assigned UE radio capability IDs and performs Tracking Area update procedure and does not include UE radio capability ID availability indication in TRACKING AREA UPDATE REQUEST message }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having assigned a new UE Radio Capability ID, enters a tracking area that is not in the list of tracking areas that the UE previously registered in the MME }

ensure that {

when { UE receives TRACKING AREA UPDATE ACCEPT message with network assigned UE radio capability ID deletion indication }

then { UE deletes any stored Network-assigned UE radio capability IDs and performs Tracking Area update procedure and does not include UE radio capability ID availability indication in TRACKING AREA UPDATE REQUEST message }

}

9.2.5.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.20.

[TS 24.301, clause 5.3.20]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.401 [10]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is not applicable to NB-S1 mode.

If the UE supports RACS:

-	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages;

-	if the UE performs an attach procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available";

-	if the UE performs a tracking area updating procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available";

-	If the UE is requested to provide the UE radio capability ID by the network during a security mode control procedure, the UE shall include the UE radio capability ID in the UE radio capability ID IE of the SECURITY MODE COMPLETE message according to the rules in clause 5.4.3.3.;

…

-	upon receiving a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested" in the ATTACH ACCEPT message in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message, the UE shall delete all network-assigned UE radio capability IDs stored at the UE for the serving network and initiate a tracking area updating procedure. If the UE has an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include a UE radio capability ID availability IE set to "UE radio capability ID available" in the TRACKING AREA UPDATE REQUEST message.

If the network supports RACS:

-	if the UE has included the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available", the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available", the network may initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE and no UE radio capability ID is available in the UE context in the MME, the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message; and

-	the network may trigger the UE to delete all network-assigned UE radio capability IDs stored at the UE for the serving network by including a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested" in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message.

9.2.5.3.3	Test description

9.2.5.3.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.5.3.3.2	Test procedure sequence

Table 9.2.5.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3-11 | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | SS transmits an ATTACH ACCEPT message including UE radio capability ID deletion indication. | <-- | ATTACH ACCEPT | - | - |
| 13-14 | Steps 15-16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 15 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message without UE radio capability availability indication? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 16 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including a UE radio capability ID. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 19 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 20 | The UE transmits a TRACKING AREA UPDATE REQUEST message including the UE radio capability ID availability indication. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including UE radio capability ID deletion indication. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 23 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message without UE radio capability availability indication? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 24 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 25 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 26 | The SS releases the RRC connection. | - | - | - | - |

9.2.5.3.3.3	Specific message contents

Table 9.2.5.3.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) | '1'B | RACS supported |  |

Table 9.2.5.3.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.3.3.3-3: Message ATTACH ACCEPT (step 12)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID deletion indication | '001'B | Network-assigned UE radio capability IDs deletion requested |  |

Table 9.2.5.3.3.3-4: Message TRACKING AREA UPDATE REQUEST (steps 15, 23)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '000'B | UE radio capability ID not available |  |

Table 9.2.5.3.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 16)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’O | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.3.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 20)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | '000'B000 | TA updating |  |
| UE radio capability ID availability | '001'B1 | UE radio capability ID available |  |

Table 9.2.5.3.3.3-7: Message TRACKING AREA UPDATE ACCEPT (step 21)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID deletion indication | '001'B | Network-assigned UE radio capability IDs deletion requested |  |

## 9.3	EMM connection management procedures (S1 mode only)

### 9.3.1	Service request procedure

#### 9.3.1.1	Service request initiated by UE for user data

9.3.1.1.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state and EMM-IDLE mode }

ensure that {

when { UE has user data pending }

then { UE establishes the RRC connection with the RRC establishmentCause set to ‘mo-Data’ and sends a SERVICE REQUEST message }

}

9.3.1.1.2	Conformance requirements

The conformance requirements covered in the current TC are specified in: TS 24.301 clauses 5.3.1.1, 5.1.3.2.2.4, 5.3.1.1, 5.6.1.1, 5.6.1.2, 5.6.1.4 and Annex D and TS 36.331 clause 5.3.3.3.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a NAS signalling connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

[TS 24.301 clause 5.1.3.2.2.4]

In the state EMM-REGISTERED an EMM context has been established and a default EPS bearer context has been activated in the UE.

…

The UE may initiate sending and receiving user data and signalling information and reply to paging. Additionally, tracking area updating procedure is performed (see subclause 5.5.3).

[TS 24.301 clause 5.3.1.1]

In S1 mode, when the RRC connection has been established successfully, the UE shall enter EMM-CONNECTED mode and consider the NAS signalling connection established.

[TS 24.301 clause 5.6.1.1]

The purpose of the service request procedure is to transfer the EMM mode from EMM-IDLE to EMM-CONNECTED mode and establish the radio and S1 bearers when uplink user data or signalling is to be sent.

...

The UE shall invoke the service request procedure when:

....

b)	 the UE, in EMM-IDLE mode, has pending user data to be sent;

[TS 24.301 clause 5.6.1.2]

If the UE has pending uplink data or uplink signalling in EMM-IDLE mode to be transmitted or it responds to paging with CN domain indicator set to "PS", the UE initiates the service request procedure by sending a SERVICE REQUEST message to the MME, starts the timer T3417, and enters the state EMM-SERVICE-REQUEST-INITIATED.

[TS 24.301 clause 5.6.1.4]

For cases a, b and c in subclause 5.6.1.1, the UE shall treat the indication from the lower layers that the user plane radio bearer is set up as successful completion of the procedure.

....

Upon successful completion of the procedure, the UE shall stop the timer T3417 and enter the state EMM-REGISTERED.

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Service Request | If a SERVICE REQUEST is to request user plane radio resources, the RRC establishment cause shall be set to MO data. (See Note1) | "originating calls" |
|  | If a SERVICE REQUEST is to request resources for UL signalling, the RRC establishment cause shall be set to MO data. (See Note 1) | "originating calls" |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

...

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

9.3.1.1.3	Test description

9.3.1.1.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Loopback Activated (State 4) according to TS 36.508 [18] using the specific message content for ACTIVATE TEST MODE and CLOSE UE TEST LOOP messages in table 9.3.1.1.3.3-1 and table 9.3.1.1.3.3-2.

9.3.1.1.3.2	Test procedure sequence

Table 9.3.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP Packet to the UE. | <-- | IP packet | - | - |
| 2 | The SS waits 1 second after the IP packet has been transmitted in step 1 and then transmits an RRCConnectionRelease message. (Note 2) | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionRequest message with establishmentCause set to ‘mo-Data’ followed by a SERVICE REQUEST message? (Note 1) | --> | SERVICE REQUEST | 1 | P |
| 4-7 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| Note 1: Triggered when timer T_delay_modeB (IP PDU delay time) expires and pending uplink data exist in buffered PDCP SDUs according to [25] clause 5.4.4.3. Note 2: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 1 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 2. |  |  |  |  |  |

9.3.1.1.3.3	Specific message contents

Table 9.3.1.1.3.3-1: ACTIVATE TEST MODE (preamble)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE B

Table 9.3.1.1.3.3-2: CLOSE UE TEST LOOP (preamble)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |  |
| IP PDU delay |  | 0 0 0 0 0 1 0 1 | 5 seconds |  |

Table 9.3.1.1.3.3-3: Message RRCConnectionRequest (step 3, Table 9.3.1.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 9.3.1.2	Void

#### 9.3.1.3	Service request / Mobile originating CS fallback

9.3.1.3.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-CONNECTED mode}

ensure that {
  when { UE initiates mobile originating CS fallback }

then { UE sends EXTENDED SERVICE REQUEST message }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {
  when { UE initiates mobile originating CS fallback }

then { UE establishes the RRC connection with the RRC establishmentCause set to ‘mo-Data’ and sends EXTENDED SERVICE REQUEST message }

}

9.3.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.1.1, 5.6.1.1, 5.6.1.2 and Annex D and TS 36.331 clause 5.3.3.3.[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a NAS signalling connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

[TS 24.301 clause5.6.1.1]

The purpose of the service request procedure is to transfer the EMM mode from EMM-IDLE to EMM-CONNECTED mode and establish the radio and S1 bearers when uplink user data or signalling is to be sent. Another purpose of this procedure is to invoke MO/MT CS fallback procedures.

This procedure is used when:

…

-	the UE in EMM-IDLE or EMM-CONNECTED mode has requested to perform mobile originating/terminating CS fallback; or

…

The service request procedure is initiated by the UE, however, for the downlink transfer of signalling or user data in EMM-IDLE mode, the trigger is given by the network by means of the paging procedure (see subclause 5.6.2).

The UE shall invoke the service request procedure when:

…

d)	the UE, in EMM-IDLE or EMM-CONNECTED mode, has a mobile originating CS fallback request;.

…

[TS 24.301 clause5.6.1.2]

If the UE has pending uplink data or uplink signalling in EMM-IDLE mode to be transmitted or it responds to paging with CN domain indicator set to "PS", the UE initiates the service request procedure by sending a SERVICE REQUEST message to the MME, starts the timer T3417, and enters the state EMM-SERVICE-REQUEST-INITIATED.

The UE shall send an EXTENDED SERVICE REQUEST message,

-	regardless of the EMM mode, if the UE has a mobile originating CS fallback request; and

…

[TS 24.301 clause 5.6.1.5]

#39	(CS domain temporarily not available);

The UE shall start timer T3442 and shall set the EPS update status to EU2 NOT UPDATED (and shall store it according to subclause 5.1.3.3). The UE shall enter the state EMM-REGISTERED.NORMAL-SERVICE.

The UE shall not try to send an EXTENDED SERVICE REQUEST message for mobile originating services to the network until timer T3442 expires or the UE sends a TRACKING AREA UPDATE REQUEST message.

…

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Service Request | If a EXTENDED SERVICE REQUEST has service type set to "mobile originating CS fallback", the RRC establishment cause shall be set to MO data. (See Note1). | "originating calls" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

...

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

9.3.1.3.3	Test description

9.3.1.3.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.3.1.3.3.2	Test procedure sequence

Table 9.3.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Force the UE to initiate CS Voice call. (Note 1) | - |  | - | - |
| 2 | Check: Does the UE transmit EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 1 | P |
| 3 | The SS sends SERVICE REJECT message with T3442 = 60 seconds in order that the UE enters EMM-REGISTERED.NORMAL-SERVICE. | <-- | SERVICE REJECT | - | - |
| 4 | The SS releases the RRC connection | -- |  | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.3.3.2-2 may occur in parallel with step 4A. | - | - | - | - |
| 4A | SS waits for 60 seconds (T3442) | - | - | - | - |
| 5 | Force the UE to initiate CS Voice call. (Note1) | - |  | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message with establishmentCause set to ‘mo-Data’ followed by EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 2 | P |
| 7 | The SS sends SERVICE REJECT message with T3442 = 60 seconds in order that the UE enters EMM-REGISTERED.NORMAL-SERVICE. | <-- | SERVICE REJECT | - | - |
| 8 | The SS releases the RRC connection | -- |  | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.3.3.2-2 may occur in parallel with step 8A. | - | - | - | - |
| 8A | SS waits for 60 seconds (T3442) | - | - | - | - |
| Note 1: This could be done by e.g. MMI or by AT command. |  |  |  |  |  |

Table 9.3.1.3.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |

9.3.1.3.3.3	Specific message contents

Table 9.3.1.3.3.3-1: Message EXTENDED SERVICE REQUEST (step 2/6, Table 9.3.1.3.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | '0000'B | “mobile originating CS fallback” |  |
| M-TMSI |  | M-TMSI1 |  |  |
| CSFB response |  | Not present |  |  |

Table 9.3.1.3.3.3-2: Message SERVICE REJECT (step 3 and7, Table 9.3.1.3.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | ‘0010 0111’ | “CS domain temporarily not available” |  |
| T3442 value |  | '0010 0001'B | 1 minute |  |

Table 9.3.1.3.3.3-3: Message RRCConnectionRequest (step 6, Table 9.3.1.3.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.1.3.3.3-4 Message TRACKING AREA UPDATE ACCEPT (step 2, Table 9.3.1.3.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

Table 9.3.1.3.3.3-5: ATTACH ACCEPT (Preamble)

| Derivation Path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS network feature support |  | Not present |  |  |

#### 9.3.1.4	Service request / Rejected / IMSI invalid

9.3.1.4.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'Illegal UE' }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, deletes any GUTI, last visited registered TAI, TAI list and KSI, considers the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed, enters the state EMM-DEREGISTERED }

}

(2)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'Illegal UE' }

then { UE handles the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and the MM parameters update status, TMSI, LAI and ciphering key sequence number }

}

9.3.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5 and TS 24.008, clause 4.7.13.4.

[TS 24.301, clause 5.6.1.5]

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate EMM cause value. When the EMM cause value is #39 "CS domain temporarily not available", the MME shall include a value for timer T3442 in the SERVICE REJECT message.

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

#3		(Illegal UE); or

...

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and the MM parameters update status, TMSI, LAI and ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the service request procedure is rejected with the GMM cause with the same value. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed.

NOTE 1:	The possibility to configure a UE so that the radio transceiver for a specific radio access technology is not active, although it is implemented in the UE, is out of scope of the present specification.

[TS 24.008, clause 4.7.13.4]

If the Service request cannot be accepted, the network returns a SERVICE REJECT message to the mobile station. An MS that receives a SERVICE REJECT message stops timer T3317. The MS shall then take different actions depending on the received reject cause value:

# 3			(Illegal MS); or

...

-	The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to subclause 4.1.3.2) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and shall consider the SIM/USIM as invalid for GPRS services until switching off or the SIM/USIM is removed.

-	A GPRS MS operating in MS operation mode A shall in addition set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. If the MS is operating in MS operation mode A and an RR connection exists, the MS shall abort the RR connection, unless an emergency call is ongoing. The SIM/USIM shall be considered as invalid also for non-GPRS services until switching off or the SIM/USIM is removed.

If S1 mode is supported in the MS, the MS shall handle the EMM parameters EMM state, EPS update status, GUTI, last visited registered TAI, TAI list and KSI as specified in 3GPP TS 24.301 [120] for the case when the service request procedure is rejected with the EMM cause with the same value.

9.3.1.4.3	Test description

9.3.1.4.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	if pc_GERAN  AND px_RATComb_Tested = EUTRA_GERAN, cell 24  (belongs to LAI-1 and RAI-1, home PLMN) is configured;

- 	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	cell A is "Serving cell" and cell B, cell 5 and cell 24 are "non-Suitable cell".

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 5 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells.

UE:

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.3.1.4.3.2	Test procedure sequence

Table 9.3.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with the EMM cause set to 'Illegal UE'. | <-- | SERVICE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| - | EXCEPTION: Steps 6Aa1 to 6Ba1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 6Aa1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA OR pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN the SS sets the cell type of Cell B to the "non-Suitable cell" and sets the cell type of Cell 5 or Cell 24 to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| 6Aa2 | Void | - | - | - | - |
| 6Aa3 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 2 | F |
| 6Aa4 | If possible (see ICS) switch off is performed or the USIM is removed, otherwise the power is removed. | - | - | - | - |
| 6Aa5 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | EXCEPTION: Steps 6Aa5Aa1, 6Aa5Aa2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that take place if pc_AutomaticAttachSwitchON is NOT supported |  | - | - | - |
| 6Aa5Aa1 | IF NOT pc_AutomaticAttachSwitchON | - | Registration on CS | - | - |
| 6Aa5Aa2 | IF NOT pc_AutomaticAttachSwitchON the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 6Aa6 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| 6Ba1 | If not (pc_UTRA or pc_GERAN) then (if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed (Note 2)). | - | - | - | - |
| 7-11 | Void |  |  |  |  |
| 11A | Set the cell type of the Cell B to the "non-Suitable cell". Set the cell type of Cell A to the "Serving cell". If px_RATComb_Tested = EUTRA_UTRA set the cell type of Cell 5 to the "Cell Off” or if px_RATComb_Tested = EUTRA_GERAN set Cell 24 to the "Cell Off”. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
|  | EXCEPTION: Steps 12a1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 12 | Void | - | - | - | - |
| 12a1 | If not (pc_UTRA or pc_GERAN), the UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 14-30 | Void | - | - | - | - |
| 31a1-31a9 | Void | - | - | - | - |
| 32-43 | The attach procedure is completed by executing steps 5 to 18a2 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: Void. Note 2: Switching off, USIM removal, or power removal shall be done before T3310 and T3311 expire (30 seconds) so that the UE does not retransmit ATTACH REQUEST message. |  |  |  |  |  |

9.3.1.4.3.3	Specific message contents

Table 9.3.1.4.3.3-1: SERVICE REJECT (step 3, Table 9.3.1.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 0011'B | Illegal UE |  |

Table 9.3.1.4.3.3-2: ATTACH REQUEST (step 13, Table 9.3.1.4.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111". |  |
| Old GUTI or IMSI | IMSI of the UE |  |  |
| Last visited registered TAI | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | Not present or any other allowed value |  |  |

Table 9.3.1.4.3.3-3: ATTACH REQUEST (step 6Aa6, Table 9.3.1.4.3.2-1)

| Derivation Path: TS 24.008 , Table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | Any allowed value |  |  |
| GPRS ciphering key sequence number | '111'B | No key is available (MS to network) |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | IMSI of the UE |  |  |
| Old routing area identification | All bits of octets 5 and 6 are set to 1, except bit 1 of octet 6 which is set to 0. Other bits are not checked. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | Not present |  |  |
|  | '0'B | no valid TMSI available | pc_SupportOpMode AND (px_AttachTypeTested = COMBINED_ATTACH) |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Not present or any allowed value |  |  |
| Additional mobile identity | Not present |  |  |
| Additional old routing area identification | Not present |  |  |

#### 9.3.1.5	Service request / Rejected / Illegal ME

9.3.1.5.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'Illegal ME' }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, deletes any GUTI, last visited registered TAI, TAI list and KSI, considers the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed, enters the state EMM-DEREGISTERED }

}

(2)

with { UE supporting A/Gb mode or Iu mode and having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'Illegal ME' }

then { UE sets the GPRS update status to GU3 ROAMING NOT ALLOWED, deletes any P-TMSI, P-TMSI signature, TMSI, LAI, RAI and GPRS ciphering key sequence number and considers the SIM/USIM as invalid for GPRS services until switching off or the SIM/USIM is removed }

}

9.3.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5 and TS 24.008, clause 4.7.13.4.

[TS 24.301, clause 5.6.1.5]

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

...

#6		(Illegal ME);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall enter the state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and the MM parameters update status, TMSI, LAI and ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the service request procedure is rejected with the GMM cause with the same value. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed.

NOTE 1:	The possibility to configure a UE so that the radio transceiver for a specific radio access technology is not active, although it is implemented in the UE, is out of scope of the present specification.

[TS 24.008, clause 4.7.13.4]

If the Service request cannot be accepted, the network returns a SERVICE REJECT message to the mobile station. An MS that receives a SERVICE REJECT message stops timer T3317. The MS shall then take different actions depending on the received reject cause value:

...

# 6			(Illegal ME);

-	The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to subclause 4.1.3.2) and enter the state GMM-DEREGISTERED. Furthermore, it shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and shall consider the SIM/USIM as invalid for GPRS services until switching off or the SIM/USIM is removed.

-	A GPRS MS operating in MS operation mode A shall in addition set the update status to U3 ROAMING NOT ALLOWED, shall delete any TMSI, LAI and ciphering key sequence number. If the MS is operating in MS operation mode A and an RR connection exists, the MS shall abort the RR connection, unless an emergency call is ongoing. The SIM/USIM shall be considered as invalid also for non-GPRS services until switching off or the SIM/USIM is removed.

9.3.1.5.3	Test description

The test description is identical to the one of subclause 9.3.1.4 except that the reject cause #3 “Illegal UE” is replaced with reject cause #6 “Illegal ME”.

#### 9.3.1.6	Service request / Rejected / EPS services not allowed

9.3.1.6.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'EPS services not allowed' }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, deletes any GUTI, last visited registered TAI, TAI list and KSI, considers the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed and enters the state EMM-DEREGISTERED }

}

(2)

with { UE supporting A/Gb mode or Iu mode and having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'EPS services not allowed' }

then { UE sets the GPRS update status to GU3 ROAMING NOT ALLOWED, deletes any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number and considers the SIM/USIM as invalid for GPRS services until switching off or the SIM/USIM is removed }

}

9.3.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5 and TS 24.008, clause 4.7.13.4.

[TS 24.301, clause 5.6.1.5]

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

...

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed. The UE shall enter the state EMM-DEREGISTERED.

A UE operating in CS/PS mode 1 or CS/PS mode 2 of operation is still IMSI attached for non-EPS services. The UE shall set the update status to U2 NOT UPDATED, shall select GERAN or UTRAN radio access technology and proceed with appropriate MM specific procedure according to the MM service state. The UE shall not reselect E-UTRAN radio access technology until switching off or the UICC containing the USIM is removed

...

If A/Gb mode or Iu mode is supported by the UE, the UE shall handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the service request procedure is rejected with the GMM cause with the same value.

[TS 24.008, clause 4.7.13.4]

If the Service request cannot be accepted, the network returns a SERVICE REJECT message to the mobile station. An MS that receives a SERVICE REJECT message stops timer T3317. The MS shall then take different actions depending on the received reject cause value:

...

# 7			(GPRS services not allowed);

-	The MS shall set the GPRS update status to GU3 ROAMING NOT ALLOWED (and shall store it according to subclause 4.1.3.2) and shall delete any P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number. The SIM/USIM shall be considered as invalid for GPRS services until switching off or the SIM/USIM is removed. The new state is GMM-DEREGISTERED.

9.3.1.6.3	Test description

9.3.1.6.3.1	Pre-test conditions

System Simulator:

-	cell A is configured as the "Serving cell";

-	If (px_RATComb_Tested = EUTRA_UTRA OR px_RATComb_Tested = EUTRA_GERAN);

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to LAI-1 and RAI-1, home PLMN) is configured and set to as a 'Non-suitable cell';

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN) is configured and set to as a 'Non-suitable cell';

-	system information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

-	If (px_RATComb_Tested = EUTRA_Only):

-	neither cell 5 nor cell 24 is configured;

-	system information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is applied in E-UTRA cells;

UE:

- 	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.3.1.6.3.2	Test procedure sequence

Table 9.3.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 1A | Void | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with EMM cause = "EPS services not allowed". | <-- | SERVICE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit a SERVICE REQUEST message in the next 30 seconds? | --> | SERVICE REQUEST | 1 | F |
| 6 | Check: Does the test result of the generic procedure in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
| - | EXCEPTION: Steps 7a1 to 7b8 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 7a1 | IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN SS configures: - Cell A as 'Non-suitable cell' - Cell 5 as 'Serving cell' | - | - | - | - |
| - | The following messages are to be observed on Cell 5 unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.6.3.2-2 occurs in parallel with step 7a2. | - | - | - | - |
| 7a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 7a3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 7a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 7a5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
|  | EXCEPTION: Steps 7a6a1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 7a6a1 | IF pc_CS THEN the UE optionally transmits an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION | - | - |
| 7a7 | The UE is brought back to operation or the USIM is inserted. | - | - |  |  |
| 7a8 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
| - | EXCEPTION: Steps 21a6a1, 21a6a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that take place if pc_AutomaticAttachSwitchON is NOT supported | - | - | - | - |
| 7a8Aa1 | IF NOT pc_AutomaticAttachSwitchON | - | Registration on CS | - | - |
| 7a8Aa2 | IF NOT pc_AutomaticAttachSwitchON the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 7b1 | IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN SS configures: - Cell A as 'Non-suitable cell' - Cell 24 as 'Serving cell' | - | - | - | - |
| - | The following messages are to be observed on Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.6.3.2-2 occurs in parallel with step 7b2. | - | - | - | - |
| 7b2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds? | --> | ATTACH REQUEST | 2 | F |
| 7b3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 7b4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 1 | F |
| 7b5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 7b6 | IF Location Updating procedure has been performed in step 1a1 of behaviour described in table 9.3.1.6.3.2-2 parallel to step 7b2 THEN the UE transmits an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION | - | - |
| 7b6A | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| - | EXCEPTION: Step 7b6B describes behaviour that depends on UE capability. | - | - | - | - |
| 7b6B | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE | - | - |
| 7b7 | The UE is brought back to operation or the USIM is inserted. | - | - |  |  |
| 7b8 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 2 | P |
|  | EXCEPTION: Steps 8a1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 8a1 | IF not (pc_UTRA or pc_GERAN) then (if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed). | - | - | - | - |
| 9 | The SS configures: - Cell A as the 'Serving cell'. - IF pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA THEN Cell 5 'Non-suitable cell off '. - IF pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN THEN Cell 24 'Non-suitable cell off '. | - | - | - | - |
|  | EXCEPTION: Steps 10a1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported |  |  |  |  |
| 10a1 | If not (pc_UTRA or pc_GERAN), the UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 11 | The UE transmits an ATTACH REQUEST message on cell A including a PDN CONNECTIVITY REQUEST message | --> | ATTACH REQUEST | - | - |
| 12-25b1 | The attach procedure is completed by executing steps 5-18b1 of the generic procedure for UE initial registration specified in TS 36.508 subclause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: Void. |  |  |  |  |  |

Table 9.3.1.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 and 1a4 describe a behaviour which depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 1a1 | IF pc_CS THEN the UE optionally transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 1a1Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 1a1Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. | - | - |
| 1a2 | The SS transmits an AUTHENTICATION REQUEST message to initiate the authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 1a3 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 1a4 | The SS transmits a LOCATION UPDATING ACCEPT message including IMSI-1 | <-- | LOCATION UPDATING ACCEPT | - | - |

9.3.1.6.3.3	Specific message contents

Table 9.3.1.6.3.3-1: SERVICE REJECT (step 3, Table 9.3.1.6.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 0111'B | EPS services not allowed |  |

Table 9.3.1.6.3.3-2: Message ATTACH REQUEST (step 11, Table 9.3.1.6.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS key set identifier | '111'B | no key is available |  |
| Old GUTI or IMSI | IMSI1 |  |  |
| Last visited registered TAI | Not present |  |  |

Table 9.3.1.6.3.3-3: Message ATTACH REQUEST (step 7a8 and 7b8, Table 9.3.1.6.3.2-1)

| Derivation path: TS 24.008 table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GPRS ciphering key sequence number | '111'B | no key is available |  |
| P-TMSI or IMSI | IMSI1 |  |  |
| Old routing area identification | all bits of the location area code shall be set to one with the exception of the least significant bit which shall be set to zero | RAI is deleted |  |
| Old P-TMSI signature | Not present |  |  |
| TMSI status | Not present or 0 |  |  |

Table 9.3.1.6.3.3-4: Void

Table 9.3.1.6.3.3-5: Void

Table 9.3.1.6.3.3-6: LOCATION UPDATING ACCEPT (step 1a2, Table 9.3.1.6.3.2-2)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity |  |  |  |
| IMSI | IMSI-1 |  |  |

#### 9.3.1.7	Service request / Rejected / UE identity cannot be derived by the network

9.3.1.7.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {

when { UE receives a SERVICE REJECT message with the EMM cause value = 9 (UE identity cannot be derived by the network) }

then { UE sets the EPS update status to EU2 NOT UPDATED and deletes any GUTI, last visited registered TAI, TAI list and KSI and automatically initiate the attach procedure }

}

9.3.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5.

[TS 24.301, clause 5.6.1.5]

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

…

#9		(UE identity cannot be derived by the network);

The UE shall set the EPS update status to EU2 NOT UPDATED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and KSI. The UE shall enter the state EMM-DEREGISTERED.

Subsequently, the UE shall automatically initiate the attach procedure.

NOTE 3:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

9.3.1.7.3	Test description

9.3.1.7.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.3.1.7.3.2	Test procedure sequence

Table 9.3.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with the EMM cause set to #9 (UE identity cannot be derived by the network). | <-- | SERVICE REJECT | - | - |
| - | EXCEPTION: Steps 3a1-3a2 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 3a1 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 3a2 describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a2 | IF NOT pc_Automatic_EPS_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including IMSI and without integrity protection? | --> | ATTACH REQUEST | 1 | P |
| 5-18b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. |  |  |  |  |  |

9.3.1.7.3.3	Specific message contents

Table 9.3.1.7.3.3-1: SERVICE REJECT (step 3, Table 9.3.1.7.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1001'B | UE identity cannot be derived by the network |  |

Table 9.3.1.7.3.3-2: ATTACH REQUEST (step 4, Table 9.3.1.7.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier | '111'B | no key is available |  |
| Old GUTI or IMSI | IMSI |  |  |
| Last visited registered TAI | Not present |  |  |

#### 9.3.1.7a	Service request / Rejected / UE implicitly detached

9.3.1.7a.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a SERVICE REJECT message with the EMM cause set to 'Implicitly detached' }

then { UE enters the state EMM-DEREGISTERED.NORMAL-SERVICE, delete the EPS mapped EPS security context if any and performs a new attach procedure }

}

9.3.1.7a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5.

[TS 24.301, clause 5.6.1.5]

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate EMM cause value.

…

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

…

#10	(Implicitly detached);

The UE shall enter the state EMM-DEREGISTERED.NORMAL-SERVICE. The UE shall delete any mapped EPS security context or partial native EPS security context. The UE shall perform a new attach procedure.

NOTE 4:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer(s) automatically.

9.3.1.7a.3	Test description

9.3.1.7a.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on cell A according to TS 36.508 [18].

9.3.1.7a.3.2	Test procedure sequence

Table 9.3.1.7a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with the EMM cause set to 'Implicitly detached'. | <-- | SERVICE REJECT | - | - |
| - | EXCEPTION: Steps 3a1 & 3a2 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 3a1 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 3a2 describes a behaviour which depends on the UE capability | - | - | - | - |
| 3a2 | IF NOT pc_Automatic_EPS_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and integrity protected using the native security context resulting from authentication during the test preamble? | --> | ATTACH REQUEST | 1 | P |
| 5-18b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. |  |  |  |  |  |

9.3.1.7a.3.3	Specific message contents

Table 9.3.1.7a.3.3-1: SERVICE REJECT (step 3, Table 9.3.1.7a.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0000 1010'B | Implicitly detached |  |

Table 9.3.1.7a.3.3-2: ATTACH REQUEST (step 4, Table 9.3.1.7a.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | NAS key set identifier allocated to UE during authentication in test preamble |  |  |
| TSC | ‘0’B | Native security context |  |
| Old GUTI or IMSI | GUTI allocated to UE during previous attach on Cell A. |  |  |
| Last visited registered TAI | TAI-1 |  |  |

#### 9.3.1.8	Void

#### 9.3.1.9	Void

#### 9.3.1.10	Void

#### 9.3.1.11	Void

#### 9.3.1.12	Void

#### 9.3.1.12a	Extended service request / Rejected / CS domain temporarily not available

9.3.1.12a.1	Test Purpose (TP)

(1)

with { UE has received a SERVICE REJECT message with the EMM cause set to 'CS domain temporarily not available' and T3442 expired }

ensure that {
  when { UE is requested to initiate a CS call }

then { UE transmit EXTENDED SERVICE REQUEST message }

}

9.3.1.12a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5.

[TS 24.301, clause 5.6.1.5]

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate EMM cause value. When the EMM cause value is #39 "CS domain temporarily not available", the MME shall include a value for timer T3442 in the SERVICE REJECT message.

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

...

#39	(CS domain temporarily not available);

The UE shall start timer T3442 and enter the state EMM-REGISTERED.NORMAL-SERVICE.

The UE shall not try to send an EXTENDED SERVICE REQUEST message for mobile originating services to the network, except for mobile originating CS fallback for emergency calls, until timer T3442 expires or the UE sends a TRACKING AREA UPDATE REQUEST message.

9.3.1.12a.3	Test description

9.3.1.12a.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) according to TS 36.508 [18].

9.3.1.12a.3.2	Test procedure sequence

Table 9.3.1.12a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "CS". | - | - | - | - |
| 2 | The UE transmits an EXTENDED SERVICE REQUEST message. | --> | EXTENDED SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with the EMM cause set to 'CS domain temporarily not available' and T3442 = 60 seconds. | <-- | SERVICE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.12a.3.2-2 may occur in parallel with step 4A. | - | - | - | - |
| 4A | SS waits for 60 seconds (T3442) | - | - | - | - |
| 5 | Cause the UE to originate CS call. (Note 1) | - | - | - | - |
| 6 | Check: Does the UE transmit an EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 1 | P |
| 6A | The SS sends SERVICE REJECT message with the EMM cause set to 'CS domain temporarily not available' and T3442 = 60 seconds. | <-- | SERVICE REJECT | - | - |
| 6B | The SS releases the RRC Connection, | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.1.12a.3.2-2 may occur in parallel with step 6C. | - | - | - | - |
| 6C | The SS waits for 60 seconds (T3442) | - | - | - | - |
| 7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. (Note 2) | - | - | - | - |
| Note 1: The trigger in step 5 is the same as in the generic procedure in 36.508 clause 6.4.3.5. Note 2: The UE is powered off, switched off, or the USIM is removed because the UE may retry a transmission of an EXTENDED SERVICE REQUEST message depending on the UE implementation after timer T3442 expires. Additionally, it is not clear which postamble procedure is performed, since the UE may search the UTRAN/GERAN cell after step 3. |  |  |  |  |  |

Table 9.3.1.12a.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |

9.3.1.12a.3.3	Specific message contents

Table 9.3.1.12a.3.3-1: Message SERVICE REJECT (step 3 and 6A, Table 9.3.1.12a.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '0010 0111'B | CS domain temporarily not available |  |
| T3442 value |  |  |  |
| Timer value | '0 0001'B | 1 minute |  |
| Unit | '001'B | value is incremented in multiples of 1 minute |  |

Table 9.3.1.12a.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 2, Table 9.3.1.12a.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS Identity | Not Present |  |  |

Table 9.3.1.12a.3.3-3: ATTACH ACCEPT (Preamble)

| Derivation Path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS network feature support |  | Not present |  |  |

#### 9.3.1.13	Void

#### 9.3.1.14	Void

#### 9.3.1.15	Void

#### 9.3.1.16	Service request / Abnormal case / Switch off

9.3.1.16.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE is to be switched off }

then { UE performs the detach procedure within 5 sec. }

}

9.3.1.16.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.6 and 5.5.2.1.

[TS 24.301, clause 5.6.1.6]

The following abnormal cases can be identified:

...

g)	Switch off

If the UE is in state EMM-SERVICE-REQUEST-INITIATED at switch off, the detach procedure shall be performed.

[TS 24.301, clause 5.5.2.2.1]

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DETACH REQUEST message. During this period, the UE may be switched off as soon as the DETACH REQUEST message has been sent.

9.3.1.16.3	Test description

9.3.1.16.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) according to TS 36.508 [18].

9.3.1.16.3.2	Test procedure sequence

Table 9.3.1.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message and starts timer T3417. | --> | SERVICE REQUEST | - | - |
| 3 | The SS does not respond to the SERVICE REQUEST message. NOTE: The SS does not transmit both SecurityModeCommand message and RRCConnectionReconfiguration message. | - | - | - | - |
| 4 | The UE is switched off. | - | - | - | - |
| 5 | Check: Does the UE transmit a DETACH REQUEST message in the next 5 sec.? (Note 1) | --> | DETACH REQUEST | 1 | P |
| 6 | Check: Does the test result of the generic procedure in TS 36.508 subclause 6.4.2.5 indicates that the UE responds to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 1 | - |
| Note 1: Even though the UE may have successfully performed an IMS registration during the preamble, an IMS deregistration attempt of the UE before sending DETACH REQUEST is not allowed in step 5 as there is no AS security in the test case body. |  |  |  |  |  |

9.3.1.16.3.3	Specific message contents

None.

#### 9.3.1.17	Service request / Abnormal case / Procedure collision

9.3.1.17.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {
  when { UE receives a DETACH REQUEST message with the Type of detach set to 're-attach required' }

then { UE sends a DETACH ACCEPT message and performs the attach procedure }

}

(2)

with { UE having sent a SERVICE REQUEST message or an EXTENDED SERVICE REQUEST message}

ensure that {
  when { UE receives a DETACH REQUEST message with the Type of detach set to the value other than 're-attach required' }

then { UE sends a DETACH ACCEPT message }

}

9.3.1.17.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.6.

[TS 24.301, clause 5.6.1.6]

The following abnormal cases can be identified:

...

h)	Procedure collision

If the UE receives a DETACH REQUEST message with detach type "re-attach required" or "re-attach not required" with EMM cause other than #2 "IMSI unknown in HSS" from the network in state EMM-SERVICE-REQUEST-INITIATED, the detach procedure shall be progressed and the service request procedure shall be aborted.

Additionally, if the service request was initiated for CS fallback or 1xCS fallback, the EMM sublayer shall indicate to the MM sublayer or the cdma2000® upper layers that the CS fallback or 1xCS fallback procedure has failed.

If the Detach type IE in the DETACH REQUEST message indicated "re-attach required", the attach procedure shall be performed.

9.3.1.17.3	Test description

9.3.1.17.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) according to TS 36.508 [18].

9.3.1.17.3.2	Test procedure sequence

Table 9.3.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a DETACH REQUEST message with the Type of detach set to 're-attach required'. | <-- | DETACH REQUEST | - | - |
| 4 | Check: Does the UE transmit a DETACH ACCEPT message? | --> | DETACH ACCEPT | 1 | P |
| 5 | Void | - | - | - | - |
| - | EXCEPTION: Steps 5a-5b describes a behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 5a | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 5b describes a behaviour which depends on the UE capability | - | - | - | - |
| 5b | IF NOT pc_Automatic_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 7-14 | Void | - | - |  |  |
| 14A1-14A14 b1 | The attach procedure is completed by executing steps 5 to 18b1 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3. |  |  |  |  |
| - | EXCEPTION: Steps 15a1 to 15b2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 15a1 | IF (px_AttachTypeTested == COMBINED_ATTACH) and (pc_CS_Fallback is true) THEN the SS pages the UE using S-TMSI with CN domain indicator set to "CS". | - | - | - | - |
| 15a2 | The UE transmits an EXTENDED SERVICE REQUEST message. | --> | EXTENDED SERVICE REQUEST | - | - |
| 15b1 | ELSE the SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 15b2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 16 | The SS transmits a DETACH REQUEST message with the Type of detach set to the value other than 're-attach required'. | <-- | DETACH REQUEST | - | - |
| 17 | Check: Does the UE transmit a DETACH ACCEPT message? | --> | DETACH ACCEPT | 2 | P |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA deregistered (E4) according to TS 36.508. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. |  |  |  |  |  |

9.3.1.17.3.3	Specific message contents

Table 9.3.1.17.3.3-1: Message DETACH REQUEST (step 3, Table 9.3.1.17.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | '001'B | re-attach required |  |
| EMM cause | Not present | If the detach type IE indicates "IMSI detach" or "re-attach required", then the UE shall ignore the EMM cause IE if received. |  |

Table 9.3.1.17.3.3-2: Message DETACH REQUEST (step 16, Table 9.3.1.17.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | '010'B | re-attach not required |  |
| EMM cause | '0000 0011'B | Illegal UE |  |

#### 9.3.1.18	Service request / Rejected / Not authorized for this CSG

9.3.1.18.1	Test Purpose (TP)

(1)

with { UE having sent a SERVICE REQUEST message }

ensure that {

when { UE receives a SERVICE REJECT message with the EMM cause value = 25 (Not authorized for this CSG) and this SERVICE REJECT message is not without integrity protection }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, removes the CSG ID of the cell that sent SERVICE REJECT message from the Allowed CSG list, searches for a suitable cell in the same PLMN }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and the CSG ID is removed from the Allowed CSG list }

ensure that {
  when { UE detects entering new tracking areas not included in the TAI list }

then { UE attempts to enter a normal cell and does not select a cell which is not included in the allowed CSG list }

}

9.3.1.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.6.1.5.

[TS 24.301, clause 5.6.1.5]

On receipt of the SERVICE REJECT message, the UE shall stop timer T3417 and take the following actions depending on the received EMM cause value.

…

#25	(Not authorized for this CSG);

Cause #25 is only applicable when received from a CSG cell. Cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.6.1.6.

If the SERVICE REJECT message with cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3). The UE shall enter the state EMM-REGISTERED.LIMITED-SERVICE.

The UE shall remove the CSG ID of the cell where the UE has initiated the service request procedure from the Allowed CSG list.

The UE shall search for a suitable cell in the same PLMN according to 3GPP TS 36.304 [21].

9.3.1.18.3	Test description

9.3.1.18.3.1	Pre-test conditions

System Simulator:

-	cell A(TAC1, frequency 1,  is a CSG cell);

-	cell B(TAC2, frequency 1,  not a CSG cell);

-	cell A is "Serving cell" and cell B " Non-suitable cell".

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell B.

-	System information combination 7 as defined in TS 36.508[18] clause 4.4.3.1 is used in cell A;

UE:

-	the UE is previously registered on cell A using either manual CSG selection or a USIM with field EFACSGL preconfigured  (so the allowed CSG list includes CSG ID of cell A).

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on cell A according to [18].

9.3.1.18.3.2	Test procedure sequence

Table 9.3.1.18.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell A. | - | - | - | - |
| 2 | The UE transmits a SERVICE REQUEST message. | --> | SERVICE REQUEST | - | - |
| 3 | The SS transmits a SERVICE REJECT message with the EMM cause = " Not authorized for this CSG " as specified. | <-- | SERVICE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | The SS configures: - Cell A as a "Serving cell". - Cell B as a "Suitable Neighbour intra-frequency cell“. | - | - | - | - |
| 6 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell B? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 7 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 8 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 8A | The SS releases the RRC connection. | - | - | - | - |
| 9 | The SS configures: - Cell A as a " Serving cell". - Cell B as a " Not Suitable cell". | - | - | - | - |
| 10 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell A in the next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 2 | F |

9.3.1.18.3.3	Specific message contents

Table 9.3.1.18.3.3-1: SERVICE REJECT (step 3, Table 9.3.1.18.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | '00011001'B | #25 "Not authorized for this CSG" |  |

Table 9.3.1.18.3.3-2: SystemInformationBlockType1 for Cell A, B(Pre-test conditions and all steps in Table 9.3.1.18.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| csg-Indication | TRUE |  | Cell A |
|  | FALSE |  | Cell B |
| csg-Identity | Not present |  | Cell B |
|  | '000 0000 0000 0000 0000 0000 0010'B |  | Cell A |

#### 9.3.1.19	Service Request / MUSIM / NAS signalling connection release

9.3.1.19.1	Test Purpose (TP)

(1)

with { UE in EMM-CONNECTED state and has transmitted an EXTENDED SERVICE REQUEST message and set the Request type to "NAS signalling connection release" in the UE request type IE and Service type to "packet services via S1"}

ensure that {

when { the UE received SERVICE ACCEPT message }

then { the UE enters the state EMM-REGISTERED }

}

9.3.1.19.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.1.2.1, 5.6.1.1, 5.6.1.2.1, 5.6.1.4.1, 6.4.3.4.

[TS 24.301, clause 5.3.1.2.1]

To allow the network to release the NAS signalling connection, the UE:

…

b)	shall start the timer T3440 if:

…

-	the tracking area updating or combined tracking area updating procedure has been initiated in EMM-IDLE mode, or the UE has set Request type to "NAS signalling connection release" in the UE request type IE in the TRACKING AREA UPDATE REQUEST message and the NAS signalling connection release bit is set to "NAS signalling connection release supported" in the EPS network feature support IE of the TRACKING AREA UPDATE ACCEPT message; and

…

Upon expiry of T3440,

-	in cases a, b, c, f, h i and j, the UE shall locally release the established NAS signalling connection; or

-	in cases d and e, the UE shall locally release the established NAS signalling connection and the UE shall initiate the attach procedure as described in clause 5.5.3.2.5, 5.5.3.3.5 or 5.6.1.5.

…

[TS 24.301, clause 5.6.1.1]

...

This procedure is used when:

...

-	to indicate to the network that the MUSIM UE requests the release of the NAS signalling connection or reject paging.

...

The UE shall invoke the service request procedure when:

...

p)	the network supports the NAS signalling connection release and the MUSIM UE in EMM-CONNECTED mode requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction; or

q)	the network supports the reject paging request and the MUSIM UE, in EMM-IDLE mode when responding to paging rejects the paging request from the network, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction.

...

The MUSIM UE shall not initiate service request procedure for requesting the network to release the NAS signalling connection if the UE is attached for emergency bearer services or if the UE has a PDN connection for emergency bearer services established. To enable the emergency call back, the UE shall not initiate service request procedure for requesting the network to release the NAS signalling connection for a UE implementation-specific duration of time after the completion of the emergency bearer services.

…

[TS 24.301, clause 5.6.1.2.1]

...

For cases p and q in clause 5.6.1.1, the UE shall send an EXTENDED SERVICE REQUEST message,

-	for case p in clause 5.6.1.1, set Request type to "NAS signalling connection release" in the UE request type IE and Service type to "packet services via S1"; or

-	for case q in clause 5.6.1.1, set Request type to "Rejection of paging" in the UE request type IE and Service type to "packet services via S1" if the UE needs to reject PS paging or to "mobile terminating CS fallback or 1xCS fallback" if the UE needs to reject CS paging; and

start T3417, enter the state EMM-SERVICE-REQUEST-INITIATED and may include its paging restriction preference in the Paging restriction IE in the EXTENDED SERVICE REQUEST message.

[TS 24.301, clause 5.6.1.4.1]

...

For cases o, p and q in clause 5.6.1.1, when the MUSIM UE in the EXTENDED SERVICE REQUEST message sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE, the UE shall treat the receipt of SERVICE ACCEPT message as the successful completion of the procedure and the UE shall reset the service request attempt counter, stop timer T3417 and enter the state EMM-REGISTERED.

…

For cases p and q in clause 5.6.1.1 when the MUSIM UE sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE in the EXTENDED SERVICE REQUEST message and if the UE requests restriction of paging by including the Paging restriction IE, the MME:

-	if accepts the paging restriction, shall include the EPS additional request result IE in the SERVICE ACCEPT message and set the Paging restriction decision to "paging restriction is accepted". The MME shall store the paging restriction of the UE and enforce these restrictions in the paging procedure as described in clause 5.6.2; or

-	if rejects the paging restriction, shall include the EPS additional request result IE in the SERVICE ACCEPT message and set the Paging restriction decision to "paging restriction is rejected", and shall discard the received paging restriction. The MME shall delete any stored paging restriction for the UE and stop restricting paging; and

-	shall initiate the release of the NAS signalling connection after the completion of the service request procedure.

When the E-UTRAN fails to establish radio bearers for one or more EPS bearer contexts, then the MME shall locally deactivate the EPS bearer contexts corresponding to the failed radio bearers based on the lower layer indication from the E-UTRAN, without notifying the UE.

If the UE is not using EPS services with control plane CIoT EPS optimization, the network shall consider the service request procedure successfully completed in the following cases:

-	when it receives an indication from the lower layer that the user plane is setup, if radio bearer establishment is required;

-	otherwise when it receives an indication from the lower layer that the UE has been redirected to the other RAT (GERAN or UTRAN in CS fallback, or cdma2000® 1x access network for 1xCS fallback).

9.3.1.19.3	Test description

9.3.1.19.3.1	Pre-test conditions

System Simulator:

-	Cell A(home PLMN1) and Cell G(home PLMN2) are configured according to table 6.3.2.2-1 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

-	For Cell A, (MCC, MNC, TAI) is (001, 01, TAI-1). For Cell G, (MCC, MNC, TAI) is (001, 02, TAI-7).

UE:

-	The UE is a MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Table 4.5.2E.1-1 and Table 4.5.2E.1-2.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.5.2E.3 of TS 36.508 [18].

9.3.1.19.3.2	Test procedure sequence

Table 9.3.1.19.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-7 | Steps 2 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed successfully to establish a new data radio bearer on Cell A. | - | - | - | - |
| 8 | Cause the UE to release the NAS signalling connection by AT or MMI command on Cell A.(Note1) | - | - | - | - |
| 9 | The UE transmits a EXTENDED SERVICE REQUEST message including "NAS signalling connection release" of Request type in the UE request type IE and Service type to "packet services via S1" on Cell A. | --> | EXTENDED SERVICE REQUEST | - | - |
| 10 | The SS transmits a SERVICE ACCEPT message. | <-- | SERVICE ACCEPT | - | - |
| 11 | Ensure that the T3440 is expired. The SS transmits a Paging message to UE on Cell A. | - | - | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionRequest message on Cell A? |  | RRCConnectionRequest | 1 | P |
| 13 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 14 | The UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST on Cell A. | --> | RRCConnectionSetupComplete SERVICE REQUEST | - | - |
| 15-18 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed by the UE to successfully complete the service request procedure on Cell A. | - | - | - | - |
| Note 1: This could be done by use of AT command, such as AT+CNASCREL or other commands. |  |  |  |  |  |

9.3.1.19.3.3	Specific message contents

Table 9.3.1.19.3.3-1: Message EXTENDED SERVICE REQUEST (step 9, Table 9.2.1.19.3.2-1)

| Derivation path: 36.508 table 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Service type |  |  |  |
| Service type value (octet 1) | ‘1000’B | packet services via S1 |  |
| UE request type |  |  |  |
| Request type (bits 4 to 1 of octet 3) | ‘0001’B | NAS signalling connection release | pc_EPC_MUSIM_NCR |
| Bits 5 to 8 of octet 3 are spare and shall be coded as zero |  |  |  |

#### 9.3.1.20	Service Request / MUSIM / Rejection of paging

9.3.1.20.1	Test Purpose (TP)

(1)

with { UE in EMM-IDLE state and has transmitted an EXTENDED SERVICE REQUEST message and set the Request type to "Rejection of paging" in the UE request type IE and Service type to "packet services via S1" }

ensure that {

when { the UE receives SERVICE ACCEPT message }

then { the UE enters the state EMM-REGISTERED }

}

9.3.1.20.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.6.1.4.1 and 5.6.2.2.1.1. Unless otherwise stated these are Rel-17 requirements.

[TS 24.301, clause 5.6.1.4.1]

…

For cases o, p and q in clause 5.6.1.1, when the MUSIM UE in the EXTENDED SERVICE REQUEST message sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE, the UE shall treat the receipt of SERVICE ACCEPT message as the successful completion of the procedure and the UE shall reset the service request attempt counter, stop timer T3417 and enter the state EMM-REGISTERED.

If the SERVICE REQUEST message or an EXTENDED SERVICE REQUEST message for packet services was used, the UE shall locally deactivate the EPS bearer contexts that do not have a user plane radio bearer established upon successful completion of the service request procedure, except for the case when the MUSIM UE in the EXTENDED SERVICE REQUEST message sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE.

…

[TS 24.301, clause 5.6.2.2.1.1]

…

Upon reception of a paging indication, if the network supports the reject paging request and the MUSIM UE decides not to accept the paging, the UE may initiate a service request procedure to reject the paging as specified in clause 5.6.1.1.

…

9.3.1.20.3	Test description

9.3.1.20.3.1	Pre-test conditions

System Simulator:

-	Cell A (home PLMN1) and Cell G (home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

UE:

-	The UE is a MUSIM UE;

-	Each USIM of the UE is configured to initiate EPS attach;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.5.2E.3-1 of  TS 36.508 [18].

9.3.1.20.3.2	Test procedure sequence

Table 9.3.1.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | UE is configured to reject Paging (Note 1) | - | - | - | - |
| 1 | Set the cell type of cell A to the " Serving cell" Set the cell type of cell G to the "Non-Suitable cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell A. | - | - | - | - |
| 2 | The SS sends a paging message. | <-- | Paging | - | - |
| 3 | The UE transmits an EXTENDED SERVICE REQUEST message and set the Request type to "Rejection of paging" in the UE request type IE and Service type to "packet services via S1" on Cell A. | --> | EXTENDED SERVICE REQUEST | - | - |
| 4 | The SS transmits a SERVICE ACCEPT message on Cell A | <-- | SERVICE ACCEPT | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on Cell A? | - | - | 1 | P |
| Note 1: This step is triggered by MMI or AT command |  |  |  |  |  |

9.3.1.20.3.3	Specific message contents

Table 9.3.1.20.3.3-1: EXTENDED SERVICE REQUEST (step 3, Table 9.3.1.20.3.2)

| Derivation Path: 36.508 Table 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | ‘1000’B | packet services via S1 |  |
| UE request type |  |  |  |  |
| UE request type IEI |  | 29 |  |  |
| Length of UE request type contents |  | ‘00000001’B |  |  |
| Request type |  | ‘00000010’B | Rejection of paging |  |

### 9.3.2	Paging procedure

#### 9.3.2.1	Paging procedure

9.3.2.1.1	Test Purpose (TP)

(1)

with { UE in ECM-IDLE }

ensure that {
  when { the network initiates a paging procedure for EPS services using S-TMSI }

then { the UE responds to the paging by establishing an RRC connection and transmitting a SERVICE REQUEST message providing correct S-TMSI in the RRCConnectionRequest }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives a Paging message including an ue-Identity set an unmatched S-TMSI i.e. other than the one allocated to the UE at the UE registration procedure }

then { UE does not establish an RRC connection }

}

9.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.6.1.1, 5.6.2.2.1, TS 33.401 clause 7.2.6.2, TS 36.331 clauses 5.3.2.3, 5.3.3.2, 5.3.3.3 and 5.3.3.4. Unless otherwise stated these are Rel-8 requirements.

[TS 24.301, clause 5.6.2.2.1]

To initiate the procedure the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [20], 3GPP TS 36.413 [23]) and starts the timer T3413 for this paging procedure. Upon reception of a paging indication, the UE shall respond to the paging with a SERVICE REQUEST message (see 3GPP TS 23.401 [10] and 3GPP TS 36.413 [23]). If the paging for EPS services was received during an ongoing UE initiated EMM specific procedure or service request procedure, then the UE shall ignore the paging and the UE and the network shall proceed with the EMM specific procedure or the service request procedure.

[TS 24.301, clause 5.6.1.1]

The UE shall invoke the service request procedure when:

a)	the UE in EMM-IDLE mode receives a paging request with CN domain indicator set to "PS" from the network;

[TS 33.401 clause 7.2.6.2]

The procedure the UE uses to transit from ECM-IDLE to ECM-CONNECTED when in EMM-REGISTERED state is initiated by a NAS Service Request message from the UE to the MME. As the UE is in EMM-REGISTERED state, a EPS security context exists in the UE and the MME, and this EPS security context further contains uplink and downlink NAS COUNTs. The NAS Service Request message sent in EMM-REGISTERED shall be integrity protected and contain the uplink NAS sequence number.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity, and the cn-Domain to the upper layers.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

...

1>if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInfoBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else:

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

[TS 36.331, clause 5.3.3.4]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, include and set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends.

9.3.2.1.3	Test description

9.3.2.1.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

-	none.

Preamble:

-	the UE is in Registered, Idle Mode (state 2) according to TS 36.508 [18].

9.3.2.1.3.2	Test procedure sequence

Table 9.3.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits a Paging message including an unmatched identity (incorrect S-TMSI). | <-- | Paging | - | - |
| 0A | Check: Does the UE transmit an RRCConnectionRequest message within 5 s? | --> | RRCConnectionRequest | 2 | F |
| 1 | SS pages the UE using S-TMSI with CN domain indicator set to "PS" | - | - | - | - |
| 2 | Check: Does the UE transmit RRCConnectionRequest message providing correct S-TMSI? | --> | RRCConnectionRequest | 1 | P |
| 2A | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 2B | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | 1 | P |
| 3 | Check: Does the UE respond with a SERVICE REQUEST message? | --> | SERVICE REQUEST | 1 | P |
| 4-7 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |

9.3.2.1.3.3	Specific message contents

Table 9.3.2.1.3.3-1: RRCConnectionRequest (step 2, Table 9.3.2.1.3.2-1)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity[1] CHOICE { |  |  |  |
| s-TMSI | Set to the value of the S-TMSI of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.2.1.3.3-2: Paging (step 1, Table 9.3.2.1.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { | 1 entry |  |  |
| ue-Identity[1] CHOICE { |  |  |  |
| s-TMSI SEQUENCE { |  |  |  |
| mmec | Set to the different value from the S-TMSI of the UE |  |  |
| m-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 9.3.2.2	Paging for CS fallback / Idle mode

9.3.2.2.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {
  when { UE received Paging for mobile termination CS fallback from NW }

then { UE establishes the RRC connection with the RRC establishmentCause set to ‘mt-Access’ and sends EXTENDED SERVICE REQUEST message }

}

9.3.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.1.1, 5.6.1.1, 5.6.2.3 and Annex D and TS 36.331 clause 5.3.3.3.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a NAS signalling connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

[TS 24.301 clause5.6.1.1]

The purpose of the service request procedure is to transfer the EMM mode from EMM-IDLE to EMM-CONNECTED mode and establish the radio and S1 bearers when uplink user data or signalling is to be sent. Another purpose of this procedure is to invoke MO/MT CS fallback procedures.

This procedure is used when:

…

-	the UE in EMM-IDLE or EMM-CONNECTED mode has requested to perform mobile originating/terminating CS fallback; or

…

The service request procedure is initiated by the UE, however, for the downlink transfer of signalling or user data in EMM-IDLE mode, the trigger is given by the network by means of the paging procedure (see subclause 5.6.2).

The UE shall invoke the service request procedure when:

…

e)	the UE, in EMM-IDLE or EMM-CONNECTED mode, has a CS fallback response to be sent to the network; or

…

[TS 24.301 clause5.6.2.3]

The network may initiate the paging procedure for non-EPS services when the UE is IMSI attached for non-EPS services.

To initiate the procedure when no NAS signalling connection exists, the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [2012], 3GPP TS 36.413 [2315]) and starts the timer T3413 for this paging procedure. The paging message includes a CN domain indicator set to ''CS'' in order to indicate that this is paging for CS fallback. Upon reception of a paging indication, the UE may respond to the paging immediately or may request upper layers input i.e. to accept or reject CS fallback. The response is indicated in the CSFB response information element in the EXTENDED SERVICE REQUEST message in both EMM-IDLE and EMM-CONNECTED modes.

The network shall stop the timer T3413 for the paging procedure when a response is received from the UE.

To notify the UE about an incoming mobile terminating CS service when a NAS signalling connection exists, the EMM entity in the network shall send a CS SERVICE NOTIFICATION message.

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Service Request | If a EXTENDED SERVICE REQUEST has service type set to "mobile terminating CS fallback", the RRC establishment cause shall be set to MT access. (See Note1). | "terminating calls" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

[TS 36.331, clause 5.3.3.3]

...

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

9.3.2.2.3	Test description

9.3.2.2.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

none.

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.3.2.2.3.2	Test procedure sequence

Table 9.3.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a paging message which CN domain indicates ''CS'' domain to the UE. | - | - | - | - |
| 2 | The UE accepts CS fallback | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionRequest message with establishmentCause set to ‘mt-Access’ followed by EXTENDED SERVICE REQUEST message? | --> | EXTENDED SERVICE REQUEST | 1 | P |
| 4 | The SS sends SERVICE REJECT message with T3442 = 60 seconds in order that the UE enters EMM-REGISTERED.NORMAL-SERVICE. | <-- | SERVICE REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.2.2.3.2-2 may occur in parallel with step 6. | - | - | - | - |
| 6 | SS waits for 60 seconds (T3442) | - | - | - | - |

Table 9.3.2.2.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |

9.3.2.2.3.3	Specific message contents

Table 9.3.2.2.3.3-0: Message RRCConnectionRequest (step 3, Table 9.3.2.2.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.3.2.2.3.3-1: Message EXTENDED SERVICE REQUEST (step 3, Table 9.3.2.2.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| M-TMSI |  | M-TMSI1 |  |  |

Table 9.3.2.2.3.3-2: Message  SERVICE REJECT (step 4, Table 9.3.2.2.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | ’0010 0111’B | CS domain temporarily not available |  |
| T3442 value |  | '0010 0001'B | 1 minute |  |

Table 9.3.2.2.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 2, Table 9.3.2.2.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

#### 9.3.2.2a	Paging for CS fallback / Connected mode

9.3.2.2a.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-CONNECTED mode}

ensure that {
  when { UE receives a CS SERVICE NOTIFICATION message for mobile termination CS fallback from NW }

then { UE sends EXTENDED SERVICE REQUEST message }

}

9.3.2.2a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.1.1, 5.6.1.1, 5.6.2.3 and Annex D and TS 36.331 clause 5.3.3.3.

[TS 24.301 clause 5.3.1.1]

When the UE is in EMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a NAS signalling connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification.

[TS 24.301 clause5.6.1.1]

The purpose of the service request procedure is to transfer the EMM mode from EMM-IDLE to EMM-CONNECTED mode and establish the radio and S1 bearers when uplink user data or signalling is to be sent. Another purpose of this procedure is to invoke MO/MT CS fallback procedures.

This procedure is used when:

…

-	the UE in EMM-IDLE or EMM-CONNECTED mode has requested to perform mobile originating/terminating CS fallback; or

…

The service request procedure is initiated by the UE, however, for the downlink transfer of signalling or user data in EMM-IDLE mode, the trigger is given by the network by means of the paging procedure (see subclause 5.6.2).

The UE shall invoke the service request procedure when:

…

e)	the UE, in EMM-IDLE or EMM-CONNECTED mode, has a CS fallback response to be sent to the network; or

…

[TS 24.301 clause5.6.2.3]

The network may initiate the paging procedure for non-EPS services when the UE is IMSI attached for non-EPS services.

To initiate the procedure when no NAS signalling connection exists, the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [2012], 3GPP TS 36.413 [2315]) and starts the timer T3413 for this paging procedure. The paging message includes a CN domain indicator set to ''CS'' in order to indicate that this is paging for CS fallback. Upon reception of a paging indication, the UE may respond to the paging immediately or may request upper layers input i.e. to accept or reject CS fallback. The response is indicated in the CSFB response information element in the EXTENDED SERVICE REQUEST message in both EMM-IDLE and EMM-CONNECTED modes.

The network shall stop the timer T3413 for the paging procedure when a response is received from the UE.

To notify the UE about an incoming mobile terminating CS service when a NAS signalling connection exists, the EMM entity in the network shall send a CS SERVICE NOTIFICATION message.

[TS 24.301, Annex D]

...

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| ... |  |  |
| Service Request | If a EXTENDED SERVICE REQUEST has service type set to "mobile terminating CS fallback", the RRC establishment cause shall be set to MT access. (See Note1). | "terminating calls" |
| ... |  |  |
| Note 1: For these NAS procedures initiated by UEs of access class 12, 13 or 14 in their home country, the RRC establishment cause will be set to "High priority access AC 11 – 15". For this purpose the home country is defined as the country of the MCC part of the IMSI, see 3GPP TS 22.011 [1A]. For these NAS procedures initiated by UE of access class 11 or 15 in their HPLMN or EHPLMN, the RRC establishment cause will be set to "High priority access AC 11 – 15". |  |  |

9.3.2.2a.3	Test description

9.3.2.2a.3.1	Pre-test conditions

System Simulator:

-	Cell A(TAI-1) is set to ''Serving cell''

UE:

None.

Preamble:

-	The UE is in state Generic RB established (state 3) on cell A according to [18].

9.3.2.2a.3.2	Test procedure sequence

Table 9.3.2.2a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a CS SERVICE NOTIFICATION message to the UE. | <-- | CS SERVICE NOTIFICATION | - | - |
| 2 | The UE accepts CS fallback | - |  | - | - |
| 3 | Check: Does the UE transmit an EXTENDED SERVICE REQUEST message to SS? | --> | EXTENDED SERVICE REQUEST | 1 | P |
| 4 | The SS sends a SERVICE REJECT message with T3442 = 60 seconds in order that the UE enters EMM-REGISTERED.NORMAL-SERVICE. | <-- | SERVICE REJECT | - | - |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The behaviour in table 9.3.2.2a.3.2-2 may occur in parallel with step 6. | - | - | - | - |
| 6 | SS waits for 60 seconds (T3442) | - | - | - | - |

Table 9.3.2.2a.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The SS releases the RRC connection. | - | - | - | - |

9.3.2.2a.3.3	Specific message contents

Table 9.3.2.2a.3.3-1: Message EXTENDED SERVICE REQUEST (step 3, Table 9.3.2.2a.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| M-TMSI |  | M-TMSI1 |  |  |

Table 9.3.2.2a.3.3-2: Message SERVICE REJECT (step 4, Table 9.3.2.2a.3.2-1)

| Derivation Path: 36.508 clause 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EMM cause |  | ’00100111’B | CS domain temporarily not available |  |
| T3442 value |  | '0010 0001'B’ | 1 minute |  |

Table 9.3.2.2a.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 2, Table 9.3.2.2a.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| GUTI | Not present |  |  |
| MS identity | Not present |  |  |

## 9.4	NAS Security

### 9.4.1	Integrity protection / Correct functionality of EPS NAS integrity algorithm / SNOW3G

9.4.1.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives a an integrity protected SECURITY MODE COMMAND message instructing to start integrity protection using algorithm SNOW3G }

then { UE transmits an integrity protected SECURITY MODE COMPLETE using SNOW3G and starts applying the NAS Integrity protection in both UL and DL }

(2)

with { Integrity protection successful started by executing Security Mode Procedure}

ensure that {

when { UE receives an IDENTITY REQUEST message without integrity protected }

then { UE foes not transmit an IDENTITY RESPONSE message }

}

9.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 clause 4.4.4.1, 4.4.4.2, 5.4.3.1, 5.4.3.2 and 5.4.3.3.

[TS 24.301, clause 4.4.4.1]

For the UE, integrity protected signalling is mandatory for the NAS messages once a valid EPS security context exists and has been taken into use. For the network, integrity protected signalling is mandatory for the NAS messages once a secure exchange of NAS messages has been established for the NAS signalling connection. Integrity protection of all NAS signalling messages is the responsibility of the NAS. It is the network which activates integrity protection.

[TS 24.301, clause 4.4.4.2]

Once the secure exchange of NAS messages has been established, the receiving EMM or ESM entity in the UE shall not process any NAS signalling messages unless they have been successfully integrity checked by the NAS. If NAS signalling messages, having not successfully passed the integrity check, are received, then the NAS in the UE shall discard that message. If any NAS signalling message is received as not integrity protected even though the secure exchange of NAS messages has been established by the network, then the NAS shall discard this message.

[TS 24.301, clause 5.4.3.1]

The purpose of the NAS security mode control procedure is to take an EPS security context into use, and initialise and start NAS signalling security between the UE and the MME with the corresponding NAS keys and security algorithms.

[TS 24.301, clause 5.4.3.2]

The MME initiates the NAS security mode control procedure by sending a SECURITY MODE COMMAND message to the UE and starting timer T3460 (see example in figure 5.4.3.2.1).

If the security mode control procedure is initiated further to a successful execution of the authentication procedure, the MME shall use the reset downlink NAS COUNT to integrity protect the SECURITY MODE COMMAND message.

The MME shall send the SECURITY MODE COMMAND message unciphered, but shall integrity protect the message with the NAS integrity key based on KASME or mapped K'ASME indicated by the eKSI included in the message. The MME shall set the security header type of the message to "integrity protected with new EPS security context".

...

The MME shall include the replayed security capabilities of the UE (including the security capabilities with regard to NAS, RRC and UP (user plane) ciphering as well as NAS, RRC integrity, and other possible target network security capabilities, i.e. UTRAN/GERAN if UE included them in the message to network), the replayed nonceUE if the UE included it in the message to the network, the selected NAS ciphering and integrity algorithms and the Key Set Identifier (eKSI).

Additionally, the MME may request the UE to include its IMEISV in the SECURITY MODE COMPLETE message.

NOTE:	The AS and NAS security capabilities will be the same, i.e. if the UE supports one algorithm for NAS it is also be supported for AS.

[TS 24.301, clause 5.4.3.3]

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received UE security capabilities and the received nonceUE have not been altered compared to what the UE provided in the initial layer 3 message that triggered this procedure.

If the type of security context flag is set to "native security context" and if the KSI matches a valid native EPS security context held in the UE while the UE has a mapped EPS security context as the current security context, the UE shall take the native EPS security context into use.

If the security mode command can be accepted, the UE shall reset the uplink NAS COUNT and the UE shall take the new EPS security context into use when:

a)	the SECURITY MODE COMMAND message is received further to a successful execution of the authentication procedure; or

b)	the type of security context flag is set to "mapped security context" in the NAS KSI IE included in the SECURITY MODE COMMAND message

If the security mode command can be accepted, the UE shall send a SECURITY MODE COMPLETE message integrity protected with the selected NAS integrity algorithm and the NAS integrity key based on the KASME or mapped K'ASME if the type of security context flag is set to "mapped security context" indicated by the eKSI. If the SECURITY MODE COMMAND message includes the type of security context flag set to "mapped security context" in the NAS KSI IE, nonceMME and nonceUE, the UE shall generate K'ASME from both nonces as indicated in 3GPP TS 33.401 [19] and reset the downlink NAS COUNT to check whether the SECURITY MODE COMMAND can be accepted or not. The UE shall cipher the SECURITY MODE COMPLETE message with the selected NAS ciphering algorithm and the NAS ciphering key based on the KASME or mapped K'ASME indicated by the eKSI. The UE shall set the security header type of the message to "integrity protected and ciphered with new EPS security context".

From this time onward the UE shall cipher and integrity protect all NAS signalling messages with the selected NAS ciphering and NAS integrity algorithms.

If the MME indicated in the SECURITY MODE COMMAND message that the IMEISV is requested, the UE shall include its IMEISV in the SECURITY MODE COMPLETE message.

9.4.1.3	Test description

9.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.4.1.3.2	Test procedure sequence

Table 9.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4B | Steps 2-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security.. It is integrity protected. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message and starts applying the NAS Integrity protection in both UL and DL? | --> | SECURITY MODE COMPLETE | 1 | P |
| 6Aa1-8Ec1 | Steps 9a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS transmits an IDENTITY REQUEST message with Integrity protected and with default ciphering | <- | IDENTITY REQUEST | - | - |
| 10 | Check: Does the UE transmit an IDENTIY RESPONSE message with Integrity Protected and with default ciphering? | -> | IDENTITY RESPONSE | 1 | P |
| 11 | The SS transmits an IDENTITY REQUEST message (not Integrity protected) | <- | IDENTITY REQUEST | - | - |
| 12 | Check: Does the UE transmit an IDENTIY RESPONSE message within the next 5 seconds? | -> | IDENTITY RESPONSE | 2 | F |
| 13-15 | IF MULTI_PDN = TRUE (NOTE) THEN steps 10-12 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| NOTE: MULTI_PDN as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.4.1.3.3	Specific message contents

Table 9.4.1.3.3-1: SECURITY MODE COMMAND (Step 5)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of integrity protection algorithm | 001 | EPS integrity algorithm 128-EIA1[SNOW3G] |  |

### 9.4.2	Integrity protection / Correct functionality of EPS NAS integrity algorithm / AES

9.4.2.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives an integrity protected SECURITY MODE COMMAND message, to start integrity protection using algorithm AES }

then { UE sends SECURITY MODE COMPLETE, integrity protected with AES  and starts applying the NAS Integrity protection in both UL and DL}

(2)

with { Integrity protection successful started by executing Security Mode Procedure}

ensure that {

when { UE receives a IDENTITY REQUEST message (requested identification parameter is not IMSI), without integrity protected }

then { UE Does not transmit IDENTITY Response}

}

9.4.2.2	Conformance requirements

Same Conformance requirements as in clause 9.4.1.2

9.4.2.3	Test description

9.4.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 9.4.1.3.1

9.4.2.3.2	Test procedure sequence

Same Test procedure sequence as in table 9.4.1.3.2.1, except the integrity protection algorithm is AES.

9.4.2.3.3	Specific message contents

Table 9.4.2.3.3-1: SECURITY MODE COMMAND (Step 6)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of integrity protection algorithm | 010 | EPS integrity algorithm 128-EIA2 (AES) |  |

### 9.4.3	Ciphering and deciphering / Correct functionality of EPS NAS encryption algorithm / SNOW3G

9.4.3.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives a SECURITY MODE COMMAND instructing to start ciphering using algorithm SNOW3G }

then { UE sends a SECURITY MODE COMPLETE message ciphered with SNOW3G  and starts applying the NAS ciphering in both UL and DL}

}

9.4.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301 clause 5.4.3.1, 5.4.3.2 and 5.4.3.3.

[TS 24.301, clause 5.4.3.1]

The purpose of the NAS security mode control procedure is to take an EPS security context into use, and initialise and start NAS signalling security between the UE and the MME with the corresponding NAS keys and security algorithms.

[TS 24.301, clause 5.4.3.2]

The MME initiates the NAS security mode control procedure by sending a SECURITY MODE COMMAND message to the UE and starting timer T3460 (see example in figure 5.4.3.2.1).

If the security mode control procedure is initiated further to a successful execution of the authentication procedure, the MME shall use the reset downlink NAS COUNT to integrity protect the SECURITY MODE COMMAND message.

The MME shall send the SECURITY MODE COMMAND message unciphered, but shall integrity protect the message with the NAS integrity key based on KASME or mapped K'ASME indicated by the eKSI included in the message. The MME shall set the security header type of the message to "integrity protected with new EPS security context".

...

The MME shall include the replayed security capabilities of the UE (including the security capabilities with regard to NAS, RRC and UP (user plane) ciphering as well as NAS, RRC integrity, and other possible target network security capabilities, i.e. UTRAN/GERAN if UE included them in the message to network), the replayed nonceUE if the UE included it in the message to the network, the selected NAS ciphering and integrity algorithms and the Key Set Identifier (eKSI).

Additionally, the MME may request the UE to include its IMEISV in the SECURITY MODE COMPLETE message.

NOTE:	The AS and NAS security capabilities will be the same, i.e. if the UE supports one algorithm for NAS it is also be supported for AS.

[TS 24.301, clause 5.4.3.3]

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received UE security capabilities and the received nonceUE have not been altered compared to what the UE provided in the initial layer 3 message that triggered this procedure.

If the type of security context flag is set to "native security context" and if the KSI matches a valid native EPS security context held in the UE while the UE has a mapped EPS security context as the current security context, the UE shall take the native EPS security context into use.

If the security mode command can be accepted, the UE shall reset the uplink NAS COUNT and the UE shall take the new EPS security context into use when:

a)	the SECURITY MODE COMMAND message is received further to a successful execution of the authentication procedure; or

b)	the type of security context flag is set to "mapped security context" in the NAS KSI IE included in the SECURITY MODE COMMAND message

If the security mode command can be accepted, the UE shall send a SECURITY MODE COMPLETE message integrity protected with the selected NAS integrity algorithm and the NAS integrity key based on the KASME or mapped K'ASME if the type of security context flag is set to "mapped security context" indicated by the eKSI. If the SECURITY MODE COMMAND message includes the type of security context flag set to "mapped security context" in the NAS KSI IE, nonceMME and nonceUE, the UE shall generate K'ASME from both nonces as indicated in 3GPP TS 33.401 [19] and reset the downlink NAS COUNT to check whether the SECURITY MODE COMMAND can be accepted or not. The UE shall cipher the SECURITY MODE COMPLETE message with the selected NAS ciphering algorithm and the NAS ciphering key based on the KASME or mapped K'ASME indicated by the eKSI. The UE shall set the security header type of the message to "integrity protected and ciphered with new EPS security context".

From this time onward the UE shall cipher and integrity protect all NAS signalling messages with the selected NAS ciphering and NAS integrity algorithms.

If the MME indicated in the SECURITY MODE COMMAND message that the IMEISV is requested, the UE shall include its IMEISV in the SECURITY MODE COMPLETE message.

9.4.3.3	Test description

9.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell A.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.4.3.3.2	Test procedure sequence

Table 9.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4B | Steps 2-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 5 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. | <-- | SECURITY MODE COMMAND | - | - |
| 6 | Check: Does the UE transmit a SECURITY MODE COMPLETE message ciphered and starts applying the NAS ciphering in both UL and DL? | --> | SECURITY MODE COMPLETE | 1 | P |
| 6Aa1-8Ec1 | Steps 9a1-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS transmits an IDENTITY REQUEST message Ciphered | <- | IDENTITY REQUEST | - | - |
| 10 | Check: Does the UE transmit an IDENTIY RESPONSE message Ciphered? | -> | IDENTITY RESPONSE | 1 | P |
| 11-13 | IF MULTI_PDN = TRUE (NOTE) THEN steps 10-12 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| NOTE: MULTI_PDN as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.4.3.3.3	Specific message contents

Table 9.4.3.3.3-1: SECURITY MODE COMMAND (Step 5)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of ciphering algorithm | 001 | EPS encryption algorithm 128-EEA1 [SNOW3G] |  |

### 9.4.4	Ciphering and deciphering / Correct functionality of EPS NAS encryption algorithm / AES

9.4.4.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives a SECURITY MODE COMMAND, to start encryption using algorithm AES}

then { UE sends SECURITY MODE COMPLETE, encrypted with AES  and starts applying the NAS encryption in both UL and DL }

}

9.4.4.2	Conformance requirements

Same Conformance requirements as in clause 9.4.3.2

9.4.4.3	Test description

9.4.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 9.4.3.3.1

9.4.4.3.2	Test procedure sequence

Same Test procedure sequence as in Table 9.4.3.3.2-1, except the integrity ciphering algorithm is AES.

9.4.4.3.3	Specific message contents

Table 9.4.4.3.3-1: SECURITY MODE COMMAND (Step 5)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of ciphering algorithm | 010 | EPS encryption algorithm 128-EEA2 (AES) |  |

### 9.4.5	Integrity protection / Correct functionality of EPS NAS integrity algorithm / ZUC

9.4.5.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives an integrity protected SECURITY MODE COMMAND message, to start integrity protection using algorithm ZUC }

then { UE sends SECURITY MODE COMPLETE, integrity protected with ZUC and starts applying the NAS Integrity protection in both UL and DL}

(2)

with { Integrity protection successful started by executing Security Mode Procedure}

ensure that {

when { UE receives a IDENTITY REQUEST message (requested identification parameter is not IMSI), without integrity protected }

then { UE Does not transmit IDENTITY Response}

}

9.4.5.2	Conformance requirements

Same Conformance requirements as in clause 9.4.1.2

9.4.5.3	Test description

9.4.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 9.4.1.3.1.

9.4.5.3.2	Test procedure sequence

Same Test procedure sequence as in table 9.4.1.3.2.1, except the integrity protection algorithm is ZUC.

9.4.5.3.3	Specific message contents

Table 9.4.5.3.3-1: SECURITY MODE COMMAND (Step 6)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of integrity protection algorithm | 011 | EPS integrity algorithm 128-EIA3 (ZUC) |  |

### 9.4.6	Ciphering and deciphering / Correct functionality of EPS NAS encryption algorithm / ZUC

9.4.6.1	Test Purpose (TP)

(1)

with { successful completion of EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE receives a SECURITY MODE COMMAND, to start encryption using algorithm ZUC}

then { UE sends SECURITY MODE COMPLETE, encrypted with ZUC and starts applying the NAS encryption in both UL and DL }

}

9.4.6.2	Conformance requirements

Same Conformance requirements as in clause 9.4.3.2

9.4.6.3	Test description

9.4.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 9.4.3.3.1

9.4.6.3.2	Test procedure sequence

Same Test procedure sequence as in Table 9.4.3.3.2-1, except the integrity ciphering algorithm is ZUC.

9.4.6.3.3	Specific message contents

Table 9.4.6.3.3-1: SECURITY MODE COMMAND (Step 5)

| Derivation path: 36.508 table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |
| - Type of ciphering algorithm | 011 | EPS encryption algorithm 128-EEA3 (ZUC) |  |