# 8	RRC

## 8.1	RRC connection management procedures

### 8.1.1	Paging

#### 8.1.1.1	Void

NOTE:	The present test (RRC / Paging for connection in idle mode) is superseded by test case 9.3.2.1 (i.e. all requirements which the present test verified are verified in test case 9.3.2.1).

#### 8.1.1.1a	RRC / Direct Indication Information / Notification of BCCH modification in idle mode

8.1.1.1a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { network indicates availability of new system information by Direct Indication Information field to 1 on MPDCCH }

then { UE applies the new system information by Direct Indication Information about the correct prach-ConfigIndex in random access }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { network broadcast the extended modification period with modificationPeriodCoeff-v1310 set to n64 and network indicates availability of new system information by Direct Indication Information field to 1 on MPDCCH }

then { UE applies the extended modification period and reads the new system information }

}

8.1.1.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.2.1.3, 6.6,  5.2.2.3, 5.2.2.4, TS 36.212, clause 5.3.3.1.14

[TS36.331, clause 5.2.1.3]

In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification and EAB modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

...

For BL UEs or UEs in CE or NB-IoT UEs, the change of specific SI message can additionally be indicated by a SI message specific value tag systemInfoValueTagSI. If systemInfoValueTag included in the SystemInformationBlockType1-BR (or MasterInformationBlock-NB in NB-IoT) is different from the one of the stored system information and if systemInfoValueTagSI is included in the SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) for a specific SI message and is different from the stored one, the UE shall consider this specific SI message to be invalid. If only systemInfoValueTag is included and is different from the stored one, the BL UE or UE in CE should consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, SystemInformationBlockType12 and SystemInformationBlockType14 to be invalid; the NB-IoT UE should consider any stored system information except SystemInformationBlockType14-NB to be invalid.

[TS36.331, clause 6.6]

Direct Indication information is transmitted on MPDCCH using P-RNTI but without associated Paging message. Table 6.6-1 defines the Direct Indication information, see TS 36.212 [22, 5.3.3.1.14].

When bit n is set to 1, UE shall behave as if the corresponding field is set in the Paging message, see 5.3.2.3. Bit 1 is the least significant bit.

Table 6.6-1: Direct Indication information

| Bit | Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | etws-Indication |
| 3 | cmas-Indication |
| 4 | eab-ParamModification |
| 5 | systemInfoModification-eDRX |
| 6, 7, 8 | Not used, and shall be ignored by UE if received. |

[TS 36.331, clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	the MasterInformationBlock, SystemInformationBlockType1-BR and SystemInformationBlockType2;

NOTE:	E-UTRAN may release BL UEs or UEs in CE or NB-IoT UEs to RRC_IDLE if these UEs need to acquire changed system information.

[TS 36.331, clause 5.2.2.4]

The UE shall:

1>	apply the specified BR-BCCH configuration defined in 9.1.1.8;

1>	if the procedure is triggered by a system information change notification:

2>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

...

The UE may apply the received SIBs immediately, i.e. the UE does not need to delay using a SIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 6:	While attempting to acquire a particular SIB, if the UE detects from schedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB.

[TS 36.212, clause 5.3.3.1.14]

DCI format 6-2 is used for paging and direct indication.

The following information is transmitted by means of the DCI format 6-2:

-	Flag for paging/direct indication differentiation – 1 bit, with value 0 for direct indication and value 1 for paging

-	If Flag=0:

-	Direct Indication information – 8 bits provide direct indication of system information update and other fields, as defined in [6]

-	Reserved information bits are added until the size is equal to that of format 6-2 with Flag=1

...

8.1.1.1a.3	Test description

8.1.1.1a.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	DCI format 6-2 flag = 0

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2-CE) according to [18].

8.1.1.1a.3.2	Test procedure sequence

Table 8.1.1.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |  |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |  |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |  |
| 4 | The UE transmits an RRCConnectionSetupComplete message. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |  |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |  |
| 6 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |  |
| 7 | The SS transmits an RRCConnectionReconfiguration message. | <-- | RRCConnectionReconfiguration | - | - |  |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |  |
| 9 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |  |
| 10 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |  |
| 11 | The SS changes the prach-ConfigIndex in the system information | - | - | - | - |  |
| 12 | The SS indicates a new system information by Direct Indication information with bit 1 and transmitted on MPDCCH using P-RNTI | <-- | - | - | - |  |
| 13 | Wait for 240s for the UE to receive system information. (Note 1) | - | - | - | - |  |
| 13A | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |  |
| 14 | Check: Does the UE re-transmit a random access using prach-ConfigIndex given in step 11? | - | - | 1,2 | P |  |
| 15 | The UE transmit an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |  |
| 16 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |  |
| 17 | The UE transmits an RRCConnectionSetupComplete message. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |  |
| 18 | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |  |
| 19 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |  |
| 20 | The SS transmits an RRCConnectionReconfiguration message. | <-- | RRCConnectionReconfiguration | - | - |  |
| 21 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |  |
| 22 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |  |
| 23 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |  |
| 24 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1 | - |  |
| Note 1: The modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. The extended modificationPeriodCoeff broadcasted in SIB1-BR is set to n64 and the defaultPagingCycle is configured as rf128. This means that the modification period is 64*128 = 8192 radio frames equals to 81.92 seconds. 240 seconds wait time is used to make sure the UE have been able to read the new system information. |  |  |  |  |  |

8.1.1.1a.3.3	Specific message contents

Table 8.1.1.1a.3.3-0: RadioResourceConfigCommonSIB-DEFAULT (preamble and step 12, Table 8.1.1.1a.3.2-1)

| Derivation Path: 36.508 Table 4.4.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| bcch-Config-v1310 SEQUENCE { |  |  |  |
| modificationPeriodCoeff-v1310 | n64 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.1a.3.3-1: RRCConnectionRequest (step 2 and 15, Table 8.1.1.1a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.1a.3.3-1A: RRCConnectionReconfiguration (step 7 and 20, Table 8.1.1.1a.3.2-1)

Derivation Path: 36.508 Table 4.6.1-8, condition SRB2-DRB(1, 0)

Table 8.1.1.1a.3.3-2: SystemInformationBlockType1-BR-r13 (step 11, table 8.1.1.1a.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 2 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.1a.3.3-2A: PRACH-ConfigSIB-v1310-DEFAULT (step 11, Table 8.1.1.1a.3.2-1)

| Derivation Path: 36.508 Table 4.6.3-7b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigSIB-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| prach-ParametersListCE-r13 SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13[1] | 5 | Set to index which denote subframe numbers different from the default one. | FDD |
| prach-ConfigIndex-r13[1] | 52 |  | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### 8.1.1.2	RRC / Paging for notification of BCCH modification in idle mode

8.1.1.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives a Paging message including a systemInfoModification }

then { UE re-acquires and applies the new system information about the correct prach-ConfigIndex in random access }

}

8.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.2.3, 5.2.2.3, and 5.2.2.4.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity and the cn-Domain to the upper layers;

1>	if the systemInfoModification is included:

2> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2.

...

[TS 36.331, clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8, depending on support of the concerned RATs;

...

[TS 36.331, clause 5.2.2.4]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the procedure is triggered by a system information change notification:

2>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

...

The UE may apply the received SIBs immediately, i.e. the UE does not need to delay using a SIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 3:	While attempting to acquire a particular SIB, if the UE detects from schedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB.

8.1.1.2.3	Test description

8.1.1.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.1.1.2.3.2	Test procedure sequence

Table 8.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 3A | The UE transmits an RRCConnectionSetupComplete message. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |
| 3B | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |
| 3C | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 3D | The SS transmits an RRCConnectionReconfiguration message. | <-- | RRCConnectionReconfiguration | - | - |
| 3E | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3F | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3G | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 4 | The SS changes the prach-ConfigIndex in the system information | - | - | - | - |
| 5 | The SS transmits a Paging message including a systemInfoModification. | <-- | Paging | - | - |
| 6 | Wait for 15s for the UE to receive system information. | - | - | - | - |
| 7 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 7A | Check: Does the UE re-transmit a random access using prach-ConfigIndex given in step 4? | - | - | 1 | P |
| 8 | The UE transmit an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 9 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 9A | The UE transmits an RRCConnectionSetupComplete message. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |
| 9B | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |
| 9C | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 9D | The SS transmits an RRCConnectionReconfiguration message. | <-- | RRCConnectionReconfiguration | - | - |
| 9E | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9F | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 9G | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1 | - |

8.1.1.2.3.3	Specific message contents

Table 8.1.1.2.3.3-1: RRCConnectionRequest (step 2 and step 8, Table 8.1.1.2.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.3.3-2A: RRCConnectionReconfiguration (step 3D and step 9D, Table 8.1.1.2.3.2-1)

Derivation Path: 36.508 Table 4.6.1-8, condition SRB2-DRB(1, 0)

Table 8.1.1.2.3.3-2: Paging (step 5, Table 8.1.1.2.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

Table 8.1.1.2.3.3-2A: SystemInformationBlockType1 (step 6, table 8.1.1.2.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| } |  |  |  |

Table 8.1.1.2.3.3-2B: SystemInformationBlockType1-BR-r13 (step 6 when UE under test is CAT M1, table 8.1.1.2.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.3.3-3: SystemInformationBlockType2 (step 6, Table 8.1.1.2.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| prach-Config SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 10 (9 for CAT-M1 UEs) |  | FDD |
| prach-ConfigIndex | 52 | Set to index which denote subframe numbers different from the default one. | TDD |
| } |  |  |  |
| } |  |  |  |
| prach-ConfigCommon-v1310 SEQUENCE { |  |  | CEModeA,CEModeB |
| prach-ParametersListCE-r13 SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13 | 10 (9 for CAT-M1 UEs) |  | FDD |
| prach-ConfigIndex-r13 | 52 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

#### 8.1.1.2a	RRC / Paging for notification of BCCH modification in idle mode / eDRX cycle longer than the modification period / eDRX cycle with eDRX Allowed/Not Allowed

8.1.1.2a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state using eDRX cycle longer than System Information Modification period }

ensure that {
  when { UE  receives a Paging message including IE systemInfoModification-eDRX-r13 }

then { UE re-acquires and applies the new system information from the next eDRX acquisition period defined by H-SFN mod 256 = 0 }

}

(2)

with { the UE applied extended DRX parameters received in Attach Accept and is in E-UTRA RRC_IDLE state }

ensure that {

when {  UE receives a Paging message including a systemInfoModification-eDRX-r13 }

then { UE re-acquires and applies the new system information about eDRX not allowed}

}

(3)

with { the UE applied normal DRX parameters and is in E-UTRA RRC_IDLE state }

ensure that {

when {  UE receives a Paging message including a systemInfoModification }

then { UE re-acquires and applies the new system information about eDRX allowed }

}

8.1.1.2a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 7.3, TS 36.331 clauses 5.2.1.3, 5.2.2.4, and 5.3.2.3.

[TS 36.304, clause 7.3]

The UE may be configured by upper layers with an extended DRX (eDRX) cycle TeDRX. The UE may operate in extended DRX only if the cell indicates support for eDRX in System Information.

If the UE is configured with a TeDRX cycle of 512 radio frames, it monitors POs as defined in 7.1 with parameter T = 512. Otherwise, a UE configured with eDRX monitors POs as defined in 7.1 (i.e, based on the upper layer configured DRX value and a default DRX value determined in 7.1), during a periodic Paging Time Window (PTW) configured for the UE or until a paging message including the UE’s NAS identity is received for the UE during the PTW, whichever is earlier. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formulae:

The PH is the H-SFN satisfying the following equation:

H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H), where

-	UE_ID_H:

- 10 most significant bits of the Hashed ID, if P-RNTI is monitored on PDCCH or MPDCCH

- 12 most significant bits of the Hashed ID, if P-RNTI is monitored on NPDCCH

-	T eDRX,H  : eDRX cycle of the UE in Hyper-frames, (TeDRX,H =1, 2, …, 256 Hyper-frames) (for NB-IoT, TeDRX,H =2, …, 1024 Hyper-frames) and configured by upper layers.

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 256* ieDRX, where

-	ieDRX = floor(UE_ID_H /TeDRX,H) mod 4

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window length (in seconds) configured by upper layers

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of S-TMSI

S-TMSI = <b39, b38, …, b0> as defined in [35]

[TS 36.331, clause 5.2.1.3]

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information.

…

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. A UE in RRC_IDLE that is configured to use a DRX cycle longer than the modification period, and receives in an eDRX acquisition period at least one Paging message including the systemInfoModification-eDRX, shall acquire the updated system information at the next eDRX acquisition period boundary. Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

[TS 36.331, clause 5.2.2.4]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8;

1>	if the procedure is triggered by a system information change notification:

2>	if the UE uses an idle DRX cycle longer than the modification period:

3>	start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary;

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

…

- if the UE is configured with a DRX cycle longer than the modification period and the systemInfoModification-eDRX is included:

2>	re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2.

8.1.1.2a.3	Test description

8.1.1.2a.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is configured to request the use of eDRX  (in the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages ).

-	The UE is in state Registered, Idle mode (state 2) according to [18]. UE sent “extended DRX parameters IE” in ATTACH REQUEST message and received “extended DRX parameters IE” in ATTACH ACCEPT  message.

8.1.1.2a.3.2	Test procedure sequence

Table 8.1.1.2a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes prach-ConfigIndex in SystemInformationBlockType2 and does not include eDRX-Allowed-r13 IE in the SystemInformationBlockType1. | - | - | - | - |
| 2 | The SS transmits a Paging message including IE systemInfoModification-eDRX-r13 in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX. | <-- | Paging | - | - |
| 3 | Wait till the beginning of next eDRX acquisition time (next H-SFN for which H-SFN mod 256 =0). | - | - | - | - |
| 4 | After Step 3, wait for 10s for the UE to acquire the new system information. | - | - | - | - |
| 5 | The SS transmits a Paging message to the UE in a valid PO as per normal DRX. | <-- | Paging | - | - |
| 6 | Check: Does the UE transmit a random access using prach-ConfigIndex given in step 1? | - | - | 1 | P |
| 7 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 8 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionSetupComplete message including a SERVICE REQUEST message? | --> | RRCConnectionSetupComplete | 2 | P |
| 10-13 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 14 | SS releases the RRC Connection. | --> | RRCConnectionRelease | - | - |
| 15 | The SS sets eDRX-Allowed-r13 IE to true in SystemInformationBlockType1 and changes prach-ConfigIndex in SystemInformationBlockType2 | - | - | - | .- |
| 16 | The SS transmits a Paging message including a systemInfoModification as per normal DRX | <-- | Paging | - | - |
| 17 | Wait for 15 secs for the UE to acquire the new system information. | - | - | - | - |
| 18 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX. | <-- | Paging | - | - |
| 19 | Check: Does the UE transmit a random access using prach-ConfigIndex given in step 15? | - | - | 3 | P |
| 20 | The UE transmit an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 21 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 22 | Check: Does the UE transmit an RRCConnectionSetupComplete message including a SERVICE REQUEST message? | --> | RRCConnectionSetupComplete | 3 | P |
| 23-26 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 27 | SS releases the RRC Connection. | --> | RRCConnectionRelease | - | - |

8.1.1.2a.3.3	Specific message contents

Table 8.1.1.2a.3.3-1: SystemInformationBlockType1 (Preamble)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-1A: SystemInformationBlockType1-BR-r13 (Preamble) when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-2: ATTACH ACCEPT (Preamble)

| Derivation Path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2,56 seconds |  |
| eDRX value | ‘0101’B | 81,92 seconds |  |

Table 8.1.1.2a.3.3-3: SystemInformationBlockType1 (step 1, table 8.1.1.2a.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-3A: SystemInformationBlockType1-BR-r13 (step 1, table 8.1.1.2a.3.2-1) when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-4: SystemInformationBlockType2 (step 1, Table 8.1.1.2a.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| prach-Config SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 9 |  | FDD |
| prach-ConfigIndex | 52 | Set to index which denote subframe numbers different from the default one. | TDD |
| } |  |  |  |
| } |  |  |  |
| prach-ConfigCommon-v1310 SEQUENCE { |  |  | CEModeA,CEModeB |
| prach-ParametersListCE-r13 SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13 | 9 |  | FDD |
| prach-ConfigIndex-r13 | 52 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

Table 8.1.1.2a.3.3-5: Paging (step 2, Table 8.1.1.2a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| redistributionIndication-r13 | Not present |  |  |
| systemInfoModification-eDRX-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-6: SystemInformationBlockType1 (step 15, table 8.1.1.2a.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 2 | Default value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-6A: SystemInformationBlockType1-BR-r13 (step 15, table 8.1.1.2a.3.2-1) when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2a.3.3-7: SystemInformationBlockType2 (step 15, Table 8.1.1.2a.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| prach-Config SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 3 |  | FDD |
| prach-ConfigIndex | 51 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| prach-ConfigCommon-v1310 SEQUENCE { |  |  | CEModeA,CEModeB |
| prach-ParametersListCE-r13 SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13 | 3 |  | FDD |
| prach-ConfigIndex-r13 | 51 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

Table 8.1.1.2a.3.3-8: Paging (step 16, Table 8.1.1.2a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

#### 8.1.1.3	RRC / Paging for connection in idle mode / Multiple paging records

8.1.1.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives a Paging message including only unmatched identities }

then { UE does not establish any RRC connection }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives a Paging message including any matched identity }

then { UE initiates RRC connection establishment and upon receipt of RRCConnectionSetup including uplinkPowerControlDedicated / deltaMCS-Enabled set to Ks value 1.25 the UE successfully establishes the RRC connection }

}

8.1.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.2.3, 5.3.3.2, 5.3.3.3 and 5.3.3.4.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall

1>	If in RRC_IDLE, for each of the Paging records included in the Paging message:

2>	If the ue-identity included in the pagingRecordList matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity, the cn-Domain and the pagingCause to the upper layers.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE state.

Upon initiation of the procedure, the UE shall:

...

1>	If access to the cell, as specified above, is not barred:

2>	apply the default configuration applicable for the antennaInformation as specified in 9.2.3, until explicitly receiving a configuration;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon re-selection.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the IE ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

[TS 36.331, clause 5.3.3.4]

NOTE:	Prior to this, lower layers allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	establish SRB1 in accordance with the received radioResourceConfiguration and as specified in 5.3.10;

1>	if stored, discard the Inter-frequency priority information and the Inter-RAT priority information provided via dedicated signalling using the IE idleModeMobilityControlInfo;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED state;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers [TS 23.122, TS 24.008] from the PLMN(s) included in the plmn-IdentityList broadcast, within SystemInformationBlockType1, in the cell where the RRC connection was established;

2>		if upper layers provide the ‘Registered MME’, set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers, set the IE plmnIdentity to the value received from upper layers;

3> set the IEs mmegi and mmec to the value received from upper layers;

2>	set the nas-DedicatedInformation to include the information received from upper layers;

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends.

8.1.1.3.3	Test description

8.1.1.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.1.1.3.3.2	Test procedure sequence

Table 8.1.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including only unmatched identities (incorrect S-TMSI). | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message within 10s? | --> | RRCConnectionRequest | 1 | F |
| 3 | The SS transmits a Paging message including two unmatched identities and a matched identity. | <-- | Paging | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 2 | P |
| 5 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | 2 | P |
| 6A-6D | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 2 | - |

8.1.1.3.3.3	Specific message contents

Table 8.1.1.3.3.3-1: Paging (step 1, Table 8.1.1.3.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { | 3 entries |  |  |
| ue-Identity[1] CHOICE { |  |  |  |
| s-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [1] | ps |  |  |
| ue-Identity[2] CHOICE { |  |  |  |
| s-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [2] | ps |  |  |
| ue-Identity[3] CHOICE { |  |  |  |
| s-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [3] | ps |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.3.3-2: Paging (step 3, Table 8.1.1.3.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { | 3 entries |  |  |
| ue-Identity[1] CHOICE { |  |  |  |
| s-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [1] | ps |  |  |
| ue-Identity[2] CHOICE { |  |  |  |
| s-TMSI | Set to the different value from the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [2] | ps |  |  |
| ue-Identity[3] CHOICE { |  |  |  |
| s-TMSI | Set to the value of the S-TMSI of the UE |  |  |
| } |  |  |  |
| cn-Domain [3] | ps |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.3.3-3: RRCConnectionRequest (step 4, Table 8.1.1.3.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.3.3-4: RRCConnectionSetup-DeltaMCS (step 5, Table 8.1.1.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-DeltaMCS ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRB1-DeltaMCS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.3.3.3-5: RadioResourceConfigDedicated-SRB1-DeltaMCS (Table 8.1.1.3.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-15, |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB1-DeltaMCS ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated--DeltaMCS using condition SRB1 |  |  |
| } |  |  |  |

Table 8.1.1.3.3.3-6: PhysicalConfigDedicated--DeltaMCS (Table 8.1.1.3.3.3-5)

| Derivation Path: 36.508, Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated--DeltaMCS ::= SEQUENCE { |  |  |  |
| uplinkPowerControlDedicated | UplinkPowerControlDedicated--DeltaMCS |  | SRB1 |
| } |  |  |  |

Table 8.1.1.3.3.3-7: UplinkPowerControlDedicated--DeltaMCS (Table 8.1.1.3.3.3-6)

| Derivation Path: 36.508, Table 4.6.3-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated--DeltaMCS ::= SEQUENCE { |  |  |  |
| deltaMCS-Enabled | en1 | Corresponds to Ks value 1.25 corresponding to “enabled”. |  |
| } |  |  |  |

#### 8.1.1.4	RRC / Paging for connection in idle mode / Shared network environment

8.1.1.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having been registered in the TA of the current cell which has broadcasted a SystemInformationBlockType1 message including multiple PLMN identities }

ensure that {
  when { UE receives a Paging message including an IE ue-Identity set to the S-TMSI which was allocated to the UE at the UE registration procedure }

then { UE establishes an RRC connection }

}

8.1.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.2.3, 5.3.3.2, 5.3.3.3 and 5.3.3.4.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the  PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity and the cn-Domain to the upper layers.

...

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

...

1> if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2> start timer T300

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

...

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

[TS 36.331, clause 5.3.3.4]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers  (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers;

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends.

8.1.1.4.3	Test description

8.1.1.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.1.1.4.3.2	Test procedure sequence

Table 8.1.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST and an IE selectedPLMN-Identity corresponding to the PLMN on which the UE has been registered to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | 1 | P |
| 4A-4D | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.1.1.4.3.3	Specific message contents

Table 8.1.1.4.3.3-1: SystemInformationBlockType1 (all steps, Table 8.1.1.4.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 2 entries |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| mcc | See Table 8.1.1.4.3.3-1A |  |  |
| mnc | See Table 8.1.1.4.3.3-1A |  |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| plmn-Identity[2] SEQUENCE { |  |  |  |
| mcc | See Table 8.1.1.4.3.3-1A |  |  |
| mnc | See Table 8.1.1.4.3.3-1A |  |  |
| } |  |  |  |
| cellReservedForOperatorUse[2] | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.4.3.3-1A: SystemInformationBlockType1-BR-r13 (all steps when UE under test is CAT M1, Table 8.1.1.4.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 2 entries |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| Mcc | See Table 8.1.1.4.3.3-1A |  |  |
| Mnc | See Table 8.1.1.4.3.3-1A |  |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| plmn-Identity[2] SEQUENCE { |  |  |  |
| Mcc | See Table 8.1.1.4.3.3-1A |  |  |
| Mnc | See Table 8.1.1.4.3.3-1A |  |  |
| } |  |  |  |
| cellReservedForOperatorUse[2] | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

The PLMN Identity list broadcasted on the BCCH in Cell 1 shall be configured as defined in the table below.

Table 8.1.1.4.3.3-1A: PLMN Identity List broadcasted for Cell 1

| Cell | PLMN Identity (1) |  | PLMN Identity (2) |  |
| --- | --- | --- | --- | --- |
|  | MCC digits | MNC digits | MCC digits | MNC digits |
| 1 | PLMN 1 | PLMN 1 | PLMN 2 | PLMN 2 |

The definition of each PLMN code is found in table below.

| PLMN | MCC digit |  |  | MNC digit |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | (NOTE 2) |  |  | (NOTE 2) |  |  |
| 2 | (NOTE 3) |  |  | 0 | 2 | - |

NOTE 1: “–“ (dash) denotes “not present”

NOTE 2: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card for PLMN 1.

NOTE 3: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card for PLMN 2.

Table 8.1.1.4.3.3-2: RRCConnectionRequest (step 2, Table 8.1.1.4.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.4.3.3-3: RRCConnectionSetupComplete (step 4, Table 8.1.1.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | Set to the PLMN selected by upper layers |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.1.5	Void

#### 8.1.1.6	RRC / BCCH modification in connected mode

8.1.1.6.1	Test Purpose (TP)

with { the UE is in E-UTRA RRC_CONNECTED }

ensure that {

when { the UE receives a Paging message including the systemInfoModification }

then { the UE re-acquires the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 }

8.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 36.331 clauses 5.3.2.3, 5.2.2.3 and 5.2.2.4.

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

...

1>	if the systemInfoModification is included:

2> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2.

...

[TS 36.331, clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

...

2>	if in RRC_CONNECTED:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8, depending on support of CDMA2000;

...

[TS 36.331, clause 5.2.2.4]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the procedure is triggered by a system information change notification:

2>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

...

The UE may apply the received SIBs immediately, i.e. the UE does not need to delay using a SIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 2:	While attempting to acquire a particular SIB, if the UE detects from schedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB.

8.1.1.6.3	Test description

8.1.1.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18], and using the condition no_periodic_BSR_or_PHR.

8.1.1.6.3.2	Test procedure sequence

Table 8.1.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message paging occasion including a systemInfoModification. | <-- | Paging | - | - |
| 2 | From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1 and SystemInformationBlockType2 as specified. | - | - | - | - |
| 3 | During the first on duration period after the end of the modification period where SystemInformationBlockType2 was modified (see step 2), the SS transmits a PDCCH order including a dedicated preamble. | - | - | - | - |
| 4 | Check: Does the UE initiate a random access procedure using prach-ConfigIndex given in step 2 (i.e. for FDD n subframe number 7 and for TDD in subframe number 6)? | - | - | 1 | P |
| 5 | The SS transmits Random Access Response with RAPID corresponding to preamble in steps 3 and 4. | - | - | - | - |

8.1.1.6.3.3	Specific message contents

Table 8.1.1.6.3.3-1: RRCConnectionReconfiguration (preamble, step 8, TS 36.508 table 4.5.3.3-1)

| Derivation path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 { |  |  |  |
| radioResourceConfigDedicated { |  |  |  |
| mac-MainConfig { |  |  |  |
| explicitValue { |  |  |  |
| drx-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.6.3.3-2: SystemInformationBlockType1 (step 2, table 8.1.1.6.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Default value is 0 |  |
| } |  |  |  |

Table 8.1.1.6.3.3-3: SystemInformationBlockType2 (step 2, table 8.1.1.6.3.2-1)

| Derivation path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| prach-Config SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 5 | The default is 3 | FDD |
| prach-ConfigIndex | 52 | The default is 51 | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.1.7	RRC / Paging / EAB active

8.1.1.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and configured for “Extended Access Barring” }

ensure that {

when { when EAB that applies to the UE’s Access Classes is being broadcast by the network and UE receives a Paging message including an ue-Identity set to the S-TMSI which was allocated to the UE at the UE registration procedure }

then { UE establishes an RRC connection }

}

8.1.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.2.2.4, and D.1, TS 36.331, clause 5.2.1.6, 5.3.2.3, 5.3.3.2, 5.3.3.3, 5.3.3.12 and 5.3.3.4.

[TS 24.301, clause D.1]

When EMM requests the establishment of a NAS-signalling connection, the RRC establishment cause used by the UE shall be selected according to the NAS procedure as specified in table D.1.1. The EMM shall also indicate to the lower layer for the purpose of access control, the call type associated with the RRC establishment cause as specified in table D.1.1. If the UE is configured for EAB (see the "ExtendedAccessBarring" leaf of NAS configuration MO in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]), the EMM shall indicate to the lower layer for the purpose of access control that EAB applies for this request except for the following cases:

-	the UE is accessing the network with one of the access classes 11 – 15;

-	the UE is answering to paging;

-	the RRC Establishment cause is set to "Emergency call"; or

-	the UE is configured to allow overriding EAB (see the "Override_ExtendedAccessBarring" leaf of the NAS configuration MO as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) and receives an indication from the upper layers to override EAB.

[TS 36.331, clause 5.2.2.4]

The UE shall:

...

1>	not initiate the RRC connection establishment subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

...

1>	if the UE is EAB capable:

2>	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification or upon entering a cell during RRC_IDLE:

3>	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4>	start acquiring SystemInformationBlockType14 immediately;

3>	else:

4>	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

[TS 36.331, clause 5.2.1.6]

Change of EAB parameters can occur at any point in time. The EAB parameters are contained in SystemInformationBlockType14. The Paging message is used to inform EAB capable UEs in RRC_IDLE about a change of EAB parameters or that SystemInformationBlockType14 is no longer scheduled. If the UE receives a Paging message including the eab-ParamModification, it shall acquire SystemInformationBlockType14 according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives a Paging message including the eab-ParamModification while it is acquiring SystemInformationBlockType14, the UE shall continue acquiring SystemInformationBlockType14 based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1.

NOTE:	The EAB capable UE is not expected to periodically check schedulingInfoList contained in SystemInformationBlockType1.

…

[TS 36.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity, and the cn-Domain to the upper layers.

...

1>	if in RRC_IDLE, the eab-ParamModification is included and the UE is EAB capable:

2>	consider previously stored SystemInformationBlockType14 as invalid;

2>	re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.6;

2>	re-acquire SystemInformationBlockType14 using the system information acquisition procedure as specified in 5.2.2.4;

…

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that EAB is applicable, upon which the procedure ends;

...

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

...

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else:

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

[TS 36.331, clause 5.3.3.4]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, include and set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3> set the mmegi and the mmec to the value received from upper layers;

…

2>	set the dedicatedInfoNAS to include the information received from upper layers;

…

2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 36.331, clause 5.3.3.12]

The UE shall:

1>	if SystemInformationBlockType14 is present and includes the eab-Param:

2>	if the eab-Common is included in the eab-Param:

3>	if the UE belongs to the category of UEs as indicated in the eab-Category contained in eab-Common; and

3>	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the eab-BarringBitmap contained in eab-Common is set to one:

4>	consider access to the cell as barred;

3>	else:

4>	consider access to the cell as not barred due to EAB;

8.1.1.7.3	Test description

8.1.1.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	System information combination 21 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The UE is configured for “Extended Access Barring”

-	The UE belong to access class 0

The UE is equipped with a USIM containing values shown in Table 8.1.1.7.3–1.

Table 8.1.1.7.3-1: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFUST | Service 96 is supported. |
| EFNASCONFIG | “ExtendedAccessBarring is set to 1 which indicate the extended access barring is applied for the UE” as defined in TS 24.368, clause 5.6. |
| EFNASCONFIG | “NAS_SignallingPriority is set to NAS signalling low priority” as defined in TS 24.368, clause 5.3. |

NOTE:	As per 23.401 cl. 4.3.17.4, UE’s configurations of low access priority and Extended Access Barring shall match each other.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.1.1.7.3.2	Test procedure sequence

Table 8.1.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits a Paging message including eab-ParamModification. The SS start transmitting SystemInformationBlockType14 as defined in Table 8.1.1.7.3.3-1 in the same system information modification period. | <-- | Paging | - | - |
| 1 | Wait for 15s for the UE to receive system information. | - | - | - | - |
| 2 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | 1 | P |
| 6-9 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | P |
| 11 | SS initiates Detach procedure with the Detach Type IE “re-attach not required” and no EMM cause IE included, as defined in Table 8.1.1.7.3.3-1. | <-- | DETACH REQUEST | - | - |
| 12 | The UE transmits a DETACH ACCEPT message on Cell 1. | --> | DETACH ACCEPT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |

8.1.1.7.3.3	Specific message contents

Table 8.1.1.7.3.3-1: SystemInformationBlockType14 (all steps in table 8.1.1.7.3.2-1)

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Common-r11 ::= SEQUENCE { |  |  |  |
| eab-Config-r11 ::= SEQUENCE { |  |  |  |
| eab-Category-r11 | ‘a’ | All UEs |  |
| eab-BarringBitmap-r11 | ‘1000000000’ | EAB active for access class 0 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.1.1.7.3.3-2: RRCConnectionRequest (step 3, Table 8.1.1.7.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.7.3.3-3: SystemInformationBlockType14 (preamble)

| Derivation path: 36.508 Table 4.4.3.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Common-r11 ::= SEQUENCE { |  |  |  |
| eab-Config-r11 ::= SEQUENCE { |  |  |  |
| eab-Category-r11 | ‘a’ | All UEs |  |
| eab-BarringBitmap-r11 | ‘0000000000’ | EAB not active |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.1.1.7.3.3-4: Message ATTACH REQUEST (preamble)

| Derivation path: TS 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Device properties | 1 | “MS is configured for NAS signalling low priority” |  |

Table 8.1.1.7.3.3-5: Paging (step 0, Table 8.1.1.7.3.2-1)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | Not present |  |  |
| etws-PrimaryNotificationIndication | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cmas-Indication-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| eab-ParamModification-r11 | true |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.7.3.3-6: Message DETACH REQUEST (step 11, Table 8.1.1.7.3.2-1)

| Derivation path: 36.508 Table 4.7.2-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Detach type | ‘010’B | “re-attach not required” |  |
| EMM cause | Not present |  |  |

#### 8.1.1.8	RRC / Paging / DRX Operation / Enhanced Coverage / WUS

8.1.1.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in enhanced cell coverage and configured for and WUS }

ensure that {

when { network sends WUS followed by Paging message with a matched identity }

then { UE establishes RRC connection }

}

8.1.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 7.4

[TS 36.304, clause 7.4]

When the UE supports WUS and WUS configuration is provided in system information, the UE shall monitor WUS using the WUS parameters provided in System Information. When the UE supports GWUS and GWUS configuration is provided in system information, UE shall monitor WUS using the GWUS parameters provided in System Information as defined in clause 7.5. When DRX is used and the UE detects WUS the UE shall monitor the following PO. When extended DRX is used and the UE detects WUS the UE shall monitor the following numPOs POs or until a paging message including the UE's NAS identity is received, whichever is earlier. If the UE does not detect WUS the UE is not required to monitor the following PO(s). If the UE missed a WUS occasion (e.g. due to cell reselection), it monitors every PO until the start of next WUS or until the PTW ends, whichever is earlier.

-	numPOs = Number of consecutive Paging Occasions (PO) mapped to one WUS provided in system information where (numPOs≥1).

The WUS configuration, provided in system information, includes time-offset between end of WUS and start of the first PO of the numPOs POs UE is required to monitor. The timeoffset in subframes, used to calculate the start of a subframe g0 (see TS 36.213 [6]), is defined as follows:

-	for UE using DRX, it is the signalled timeoffsetDRX;

-	for UE using eDRX, it is the signalled timeoffset-eDRX-Short if timeoffset-eDRX-Long is not broadcasted;

-	for UE using eDRX, it is the value determined according to Table 7.4-1 if timeoffset-eDRX-Long is broadcasted

Table 7.4-1: Determination of GAP between end of WUS and associated PO

|  |  | timeoffset-eDRX-Long |  |
| --- | --- | --- | --- |
|  |  | 1000ms | 2000ms |
| UE Reported wakeUpSignalMinGap-eDRX | 40ms or not reported | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 240ms | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 1000ms | timeoffset-eDRX-Long | timeoffset-eDRX-Long |
|  | 2000ms | timeoffset-eDRX-Short | timeoffset-eDRX-Long |

The timeoffset is used to determine the actual subframe g0 as follows (taking into consideration resultant SFN and/or H-SFN wrap-around of this computation):

g0 = PO – timeoffset, where PO is the Paging Occasion subframe as defined in clause 7.1

For UE using eDRX, the same timeoffset applies between the end of WUS and associated first PO of the numPOs POs for all the WUS occurrences for a PTW.

The timeoffset, g0, is used to calculate the start of the WUS as defined in TS 36.213 [6].

8.1.1.8.3	Test description

8.1.1.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1 supports CE.

UE:

-	None

Preamble:

-	The UE is in state Registered, Idle mode (state 2-CE) according to [18].

8.1.1.8.3.2	Test procedure sequence

Table 8.1.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits WUS followed by Paging message with a matched identity | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | UE transmits RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment | --> | RRCConnectionSetupComplete | - | - |
| 5-8 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |

8.1.1.8.3.3	Specific message contents

Table 8.1.1.8.3.3-1: SystemInformationBlockType2 (preamble and all steps, table 8.1.1.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| wus-Config-r15 ::= SEQUENCE { |  |  |  |
| maxDurationFactor-r15 | one16th |  |  |
| numPOs-r15 | n1 |  |  |
| freqLocation-r15 | n0 |  |  |
| timeOffsetDRX-r15 | ms80 |  |  |
| timeOffset-eDRX-Short-r15 | ms160 |  |  |
| timeOffset-eDRX-Long-r15 | Not Present |  |  |
| } |  |  |  |
| wus-Config-v1560 ::= SEQUENCE { |  |  |  |
| powerBoost-r15 | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.1.9	RRC / Paging / eDRX Operation / Enhanced Coverage / WUS

8.1.1.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in enhanced cell coverage and configured for eDRX and WUS }

ensure that {

when { when network sends WUS followed by Paging message with a matched identity }

then { UE establishes RRC connection }

}

8.1.1.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 7.4 [TS 36.304, clause 7.4]

When the UE supports WUS and WUS configuration is provided in system information, the UE shall monitor WUS using the WUS parameters provided in System Information. When the UE supports GWUS and GWUS configuration is provided in system information, UE shall monitor WUS using the GWUS parameters provided in System Information as defined in clause 7.5. When DRX is used and the UE detects WUS the UE shall monitor the following PO. When extended DRX is used and the UE detects WUS the UE shall monitor the following numPOs POs or until a paging message including the UE's NAS identity is received, whichever is earlier. If the UE does not detect WUS the UE is not required to monitor the following PO(s). If the UE missed a WUS occasion (e.g. due to cell reselection), it monitors every PO until the start of next WUS or until the PTW ends, whichever is earlier.

-	numPOs = Number of consecutive Paging Occasions (PO) mapped to one WUS provided in system information where (numPOs≥1).

The WUS configuration, provided in system information, includes time-offset between end of WUS and start of the first PO of the numPOs POs UE is required to monitor. The timeoffset in subframes, used to calculate the start of a subframe g0 (see TS 36.213 [6]), is defined as follows:

-	for UE using DRX, it is the signalled timeoffsetDRX;

-	for UE using eDRX, it is the signalled timeoffset-eDRX-Short if timeoffset-eDRX-Long is not broadcasted;

-	for UE using eDRX, it is the value determined according to Table 7.4-1 if timeoffset-eDRX-Long is broadcasted

Table 7.4-1: Determination of GAP between end of WUS and associated PO

|  |  | timeoffset-eDRX-Long |  |
| --- | --- | --- | --- |
|  |  | 1000ms | 2000ms |
| UE Reported wakeUpSignalMinGap-eDRX | 40ms or not reported | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 240ms | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 1000ms | timeoffset-eDRX-Long | timeoffset-eDRX-Long |
|  | 2000ms | timeoffset-eDRX-Short | timeoffset-eDRX-Long |

The timeoffset is used to determine the actual subframe g0 as follows (taking into consideration resultant SFN and/or H-SFN wrap-around of this computation):

g0 = PO – timeoffset, where PO is the Paging Occasion subframe as defined in clause 7.1

For UE using eDRX, the same timeoffset applies between the end of WUS and associated first PO of the numPOs POs for all the WUS occurrences for a PTW.

The timeoffset, g0, is used to calculate the start of the WUS as defined in TS 36.213 [6].

8.1.1.9.3	Test description

8.1.1.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1 supports CE.

UE:

-	The UE is configured to request the use of eDRX  (in ATTACH REQUEST message ).

Preamble:

-	The UE is in state Registered, Idle mode (state 2-CE) according to [18]. UE sent “extended DRX parameters IE” in ATTACH REQUEST message and received “extended DRX parameters IE” in ATTACH ACCEPT  message

8.1.1.9.3.2	Test procedure sequence

Table 8.1.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits WUS followed by Paging message with a matched identity | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | - | - |
| 5-8 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |

8.1.1.9.3.3	Specific message contents

Table 8.1.1.9.3.3-1: SystemInformationBlockType1-BR-r13 (all steps, table 8.1.1.9.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.9.3.3-2: SystemInformationBlockType2 (all steps, table 8.1.1.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| wus-Config-r15 ::= SEQUENCE { |  |  |  |
| maxDurationFactor-r15 | one16th |  |  |
| numPOs-r15 | n2 |  |  |
| freqLocation-r15 | n4 |  |  |
| timeOffsetDRX-r15 | ms80 |  |  |
| timeOffset-eDRX-Short-r15 | ms160 |  |  |
| timeOffset-eDRX-Long-r15 | Not Present |  |  |
| } |  |  |  |
| wus-Config-v1560 ::= SEQUENCE { |  |  |  |
| powerBoost-r15 | dB0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.9.3.3-3 ATTACH ACCEPT (Preamble)

| Derivation Path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2,56 seconds |  |
| eDRX value | ‘0010’B | 20,48 seconds |  |

### 8.1.2	RRC connection establishment

#### 8.1.2.1	Void

NOTE:	The present test (RRC connection establishment / Success) is superseded by test case 8.1.1.3 (i.e. all requirements which the present test verified are verified in test case 8.1.1.3).

#### 8.1.2.2	RRC connection establishment / Reject with wait time

8.1.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and has sent an RRCConnectionRequest message }

ensure that {

when { UE receives an RRCConnectionReject message including an IE waitTime set to non-zero value }

then { UE doesn't re-send RRCConnectionRequest before the waitTime is expired}

}

8.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.3.2, 5.3.3.3 and 5.3.3.8.

[TS 36.331, clause 5.3.3.2]

…

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

...

1>	If access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon re-selection.

...

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the IE ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	Set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

...

[TS 36.331, clause 5.3.3.8]

The UE shall:

1>	stop timer T300;

1>	reset MAC and release the MAC configuration;

1>	start timer T302, with the timer value set to the waitTime;

1>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls, mobile originating signalling and mobile terminating access is applicable, upon which the procedure ends.

8.1.2.2.3	Test description

8.1.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 (serving cell) according to [18].

8.1.2.2.3.2	Test procedure sequence

Table 8.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 1A | Wait for 1 s after the IP packet has been transmitted in step 1. (Note 3) |  |  |  |  |
| 1B | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease |  |  |
| 1C | The SS transmits a Paging message with a matching UE identity. | <-- | Paging |  |  |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS responds with RRCConnectionReject message with IE waitTime set to 10s. | <-- | RRCConnectionReject | - | - |
| - | EXCEPTION: the behaviour in table 8.1.2.2.3.2-2 runs in parallel with steps 4 and 5 below. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | The SS transmits a Paging message with a matching UE identity. | <-- | Paging | - | - |
| 6 | The SS starts timer Timer_1 = 5 s (Note 1) | - | - | - | - |
| - | EXCEPTION: Steps 7a1 to 7 b2 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens.(Note 2) | - | - | - | - |
| 7a1 | Check: Does UE transmit RRCConnectionRequest message when Cell 1 is treated as normal cell and access is granted for originating call? | - | RRCConnectionRequest | 1 | P |
| 7a2-7a6 | Steps 4 to 8 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed. | - | - | - | - |
|  | EXCEPTION: Step 7a7 and 7a8 can occur in any order |  |  |  |  |
| 7a7 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete |  |  |
| 7a8 | The UE loop backs the IP packet received in step 1 on the DRB associated with the default EPS bearer context on Cell 1. |  |  |  |  |
| 7a9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? |  |  | 1 |  |
| 7b1 | The SS waits for Timer_1 expiry | - | - | - | - |
| 7b2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1 | - |
| Note 1: Test step 6 is always executed 10s after step 3, i.e. when the reject timer is expired in the UE and the UE is allowed to answer paging and initiate calls again. This is because test steps 4 and 5 of the main behaviour run in parallel with test step 1 of the parallel behaviour, which lasts 10s unless the verdict is "Failed". Note 2: A UE may send RRCConnectionRequest for the pending data sent at step 1. Note 3: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 1 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 1B. |  |  |  |  |  |

Table 8.1.2.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an RRCConnectionRequest message within 10s. | --> | RRCConnectionRequest | 1 | F |

8.1.2.2.3.3	Specific message contents

Table 8.1.2.2.3.3-1: RRCConnectionReject (step 3, table 8.1.2.2.3.2-1)

| Derivation path: 36.508 table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReject ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReject-r8 SEQUENCE { |  |  |  |
| waitTime | 10 | 10 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.2.3.3-2: CLOSE UE TEST LOOP (preamble, Table 8.1.2.2.3.2-1)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

#### 8.1.2.3	RRC connection establishment / Return to idle state after T300 timeout

8.1.2.3.1	Test Purpose (TP)

(1)

with {UE in E-UTRA RRC_IDLE state having sent an RRCConnectionRequest message}

ensure that {
  when { the SS does not answer to the UE during T300}

then {UE goes to RRC_IDLE}

}

8.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.3.6.

[TS 36.331, clause 5.3.3.6]

The UE shall:

1> if timer T300 expires

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends.

8.1.2.3.3	Test description

8.1.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.1.2.3.3.2	Test procedure sequence

Table 8.1.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS is configured not to transmit MAC control element with matched UE Contention Resolution Identity | - | - | - | - |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | Paging | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS waits for 2s. Note: the UE may transmit one or more RRCConnectionRequest messages but the SS does not answer to these messages. | --> | RRCConnectionRequest | - | - |
| 3A | SS is configured to transmit MAC control element with matched UE Contention Resolution Identity | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1 | - |

8.1.2.3.3.3	Specific message contents

Table 8.1.2.3.3.3-1: RRCConnectionRequest (step 2 and step 3, Table 8.1.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.2.4	Void

#### 8.1.2.5	RRC connection establishment / 0% access probability for MO calls, no restriction for MO signalling

8.1.2.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received a SystemInformationBlockType2 indicating 0% access probability for MO calls }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 indicating no restriction for MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

...

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Data:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	else:

2>	if the UE is establishing the RRC connection for mobile originating calls and if both timers T302 and T303 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T303 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Data:

T303= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

...

8.1.2.5.3	Test description

8.1.2.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 (HPLMN in different TA).

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 (serving cell) according to [18].

8.1.2.5.3.2	Test procedure sequence

Table 8.1.2.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.1.2.5.3.2-2.

Table 8.1.2.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.1.2.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | Void | - | - | - | - |
| 2A | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2B | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.5.3.2-1. | - | - | - | - |
| 3A | Check: Does the UE transmit an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 2 | P |
| 3B-3F | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 4-9 | Void | - | - | - | - |
| 10 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 10A | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 10B | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| 10C | Wait for 1 s after the IP packet has been transmitted in step 10B. (Note 1) | - | - | - | - |
| 11-13 | Void | - | - | - | - |
| 14 | The SS transmits an RRCConnectionRelease message on Cell 11. | <-- | RRCConnectionRelease | - | - |
| 15 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? | --> | RRCConnectionRequest | 1 | F |
| 16 | Void | - | - | - | - |
| - | EXCEPTION: Step 26 and Step 27a1 can happen in any order | - | - | - | - |
| 17 - 26 | Check: Does the test result of steps 1 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 11? | - | - | 1,2 | - |
| - | EXCEPTION: Step 27a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 27a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 10C on the DRB associated with the default EPS bearer context on Cell 11 within 10s. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 10B to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 14. |  |  |  |  |  |

8.1.2.5.3.3	Specific message contents

Table 8.1.2.5.3.3-1: SystemInformationBlockType2 for Cell 11 (preamble and all steps, Table 8.1.2.5.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling | Not present |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s512 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.5.3.3-1A: CLOSE UE TEST LOOP (preamble, Table 8.1.2.5.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

Table 8.1.2.5.3.3-2: RRCConnectionRequest (step 3A, Table 8.1.2.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.2.6	RRC connection establishment / Non-zero percent access probability for MO calls, no restriction for MO signalling

8.1.2.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received a SystemInformationBlockType2 indicating non zero percent access probability for MO calls }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message or UE transmits an RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 indicating no restriction for MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

...

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Data:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

...

2>	else:

3>	consider access to the cell as not barred;

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	else:

2>	if the UE is establishing the RRC connection for mobile originating calls and if both timers T302 and T303 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T303 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Data:

T303= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

2>	else if the UE is establishing the RRC connection for mobile originating signalling and if both timers T302 and T305 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T305 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Signalling:

T305= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

8.1.2.6.3	Test description

8.1.2.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11.

UE:

None.

Preamble:

-	UE is in state Generic RB Established, UE Test Mode Activated (state 3A) on Cell 1 according to [18].

8.1.2.6.3.2	Test procedure sequence

Table 8.1.2.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.1.2.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS initializes an internal counter K to 0 and L to 0. | - | - | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: Step 2Aa1 to 3b9 shall be repeated maximum 50 times unless K > 0 and L >0. (Note 1) | - | - | - | - |
| - | EXCEPTION: Step 2Aa1 describes behaviour that depends on the internal counters. | - | - | - | - |
| 2Aa1 | IF K>0 or L>0 THEN The SS opens the UE test loop mode. | - | - | - | - |
| 2B | The SS closes the UE test loop mode. | - | - | - | - |
| 2C | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 2D | Wait for 1 s after the IP packet has been transmitted in step 2C. | - | - | - | - |
| 2E | SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 2F | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| - | EXCEPTION: Steps 3a1 to 3b9 describe behaviours which vary depending on whether the UE transmits an RRCConnectionRequest message or not. | - | - | - | - |
| 3a1 | IF the UE does not transmit any RRCConnectionRequest message within 5s (Note 3) THEN the SS increments the counter K by 1. | - | - | - | - |
| - | EXCEPTION: Steps 3a9 to 3a10 can occur in any order | - | - | - | - |
| 3a2-3a9 | Steps 2 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed (Note 2). | - | - | - | - |
| - | EXCEPTION: Step 3a10 describes behaviour that depends on UE implementation; step 3a10 takes place if the UE has user data pending. | - | - | - | - |
| 3a10 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 2D on the DRB associated with the default EPS bearer context on Cell 1 within 10 s. | - | - | - | - |
| 3b1 | ELSE IF the UE transmits an RRCConnectionRequest message on Cell 1 THEN the SS increments the counter L by 1. | --> | RRCConnectionRequest | - | - |
| 3b2 | Void | - | - | - | - |
| - | EXCEPTION: Steps 3b8 to 3b9 can occur in any order | - | - | - | - |
| 3b3-3b8 | Steps 4 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed. | - | - | - | - |
| 3b9 | The UE loop backs the IP packet received in step 4 on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 3b10-3b14 | Void | - | - | - | - |
| 4 | Check: Is the counter K > 0 and L > 0? | - | - | 1 | P |
| 4A | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 4B | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 5 | The SS changes the power level setting according to the row “T1” in table 8.1.2.6.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 11? | --> | RRCConnectionRequest | 2 | P |
| 7-11 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 12 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 11? | - | - | 1,2 | - |
| Note 1: There is an extremely low probability that the UE originating calls are barred, or not barred, 50 times consecutively. (The probability is 1.7810-15). Therefore, 50 times is enough number of trials for this test case. Note 2: After UE enters RRC_CONNECTED T303 is stopped. Note 3: The UE starts T303. |  |  |  |  |  |

8.1.2.6.3.3	Specific message contents

Table 8.1.2.6.3.3-1: SystemInformationBlockType2 for Cell 1 and Cell 11 (preamble and all steps, Table 8.1.2.6.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling | Not present |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p50 |  |  |
| ac-BarringTime | s64 |  |  |
| ac-BarringForSpecialAC | ‘00000’B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6.3.3-2: RRCConnectionRequest (step 3b1, Table 8.1.2.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6.3.3-3: Void

Table 8.1.2.6.3.3-4: RRCConnectionRequest (step 6, Table 8.1.2.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6.3.3-5: CLOSE UE TEST LOOP (step 2B, Table 8.1.2.6.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1000'B | 8 seconds |  |

#### 8.1.2.7	RRC connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed

8.1.2.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having an Access Class with a value in the range 11..15 and having received a SystemInformationBlockType2 indicating 0% access probability for AC 0..9 for both MO call and MO signalling, access restriction for AC 10 and no restriction for AC 11..15 for both MO call and MO signalling }

ensure that {
  when { UE has user data pending }

then { UE transmits an RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having an Access Class with a value in the range 11..15 }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 indicating 0% access probability for AC 0..9 for both MO call and MO signalling, access restriction for AC 10 and no restriction for AC 11..15 for both MO call and MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

...

2>	else:

3>	consider access to the cell as not barred;

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

...

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

...

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

...

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

...

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

...

8.1.2.7.3	Test description

8.1.2.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 (HPLMN in different TA).

UE:

-	None

Preamble:

The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

8.1.2.7.3.2	Test procedure sequence

Table 8.1.2.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions at the beginning of each iteration of the test body, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.2.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy SrxlevCell 1 > SrxlevCell 11. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Off | Off | The power level values are set to “cell off”. |

Table 8.1.2.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 0 to 29 are run 5 times using test parameter values as given for each iteration in table 8.1.2.7.3.2-3 | - | - | - | - |
| 0 | Insert USIM with EFACC as per table 8.1.2.7.3.2-3 |  |  |  |  |
| 0A | Void | - | - | - | - |
| 0B | The SS changes Cell 1 and Cell 11 level according to the row "T0" in table 8.1.2.7.3.2-1. | - | - | - | - |
| 0BA | The UE is switched on and brought to state Loop Back Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18] | - | - | - | - |
| 0C | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 0D | Wait for 1 s after the IP packet has been transmitted in step 0. (Note 1) | - | - | - | - |
| 1 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 3A | The UE transmits an RRCConnectionSetupComplete message on Cell 1. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |
| 3B | The SS transmits a SecurityModeCommand message on Cell 1. | <-- | SecurityModeCommand | - | - |
| 3C | The UE transmits a SecurityModeComplete message on Cell 1. | --> | SecurityModeComplete | - | - |
| 3D | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: Step 3E and 3EA can occur in any order | - | - | - | - |
| 3E | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3EA | The UE loop backs the IP packet received in step 0 on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 3EB | The SS sends OPEN TEST LOOP message to open the UE test loop. | - | DLInformationTransfer | - | - |
| 3EC | The UE responds with OPEN TEST LOOP COMPLETE message. | - | ULInformationTransfer | - | - |
| 3F | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 3G | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.7.3.2-1. | - | - | - | - |
| 4A | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 11 and that the UE has performed TAU procedure with establishment cause ‘high priority access’? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |
| 5-10 | Void | - | - | - | - |
| 11 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 12-20 | Void | - | - | - | - |
| 21 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 11 and that the UE responds to paging with establishment cause ‘high priority access’? | - | - | 1,2 | - |
| 22-28 | The UE Switch off/Power off procedure is performed as per clause 10.1.2.1 of 36.523-3 [20] | - | - | - | - |
| 29 | The SS changes Cell 1 and Cell 11 level according to the row "T2" in table 8.1.2.7.3.2-1. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 0 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 1. |  |  |  |  |  |

Table 8.1.2.7.3.3: Test Parameters

| Iteration | EFACC |
| --- | --- |
| 1 | File size: 2 Bytes Default values (BIN): Byte 1: 100000** Byte 2: ******** |
| 2 | File size: 2 Bytes Default values (BIN): Byte 1: 010000** Byte 2: ******** |
| 3 | File size: 2 Bytes Default values (BIN): Byte 1: 001000** Byte 2: ******** |
| 4 | File size: 2 Bytes Default values (BIN): Byte 1: 000100** Byte 2: ******** |
| 5 | File size: 2 Bytes Default values (BIN): Byte 1: 000010** Byte 2: ******** |
| Note: The test house may set any single bit shown by "*" to "1". This determines the access control class of the USIM |  |

8.1.2.7.3.3	Specific message contents

Table 8.1.2.7.3.3-1: SystemInformationBlockType2 for Cell 1 and Cell 11 (preamble, Table 8.1.2.7.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | TRUE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.7.3.3-1A: CLOSE UE TEST LOOP (preamble, Table 8.1.2.7.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1010'B | 10 seconds |  |

Table 8.1.2.7.3.3-2: RRCConnectionRequest (step 2, step 4A and step 21, Table 8.1.2.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.7.3.3-3: Void

Table 8.1.2.7.3.3-4: Void

Table 8.1.2.7.3.3-5: Void

Table 8.1.2.7.3.3-6: Void

Table 8.1.2.7.3.3-7: Void

Table 8.1.2.7.3.3-8: RRCConnectionReconfiguration (step 3D, Table 8.1.2.7.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SRB2-DRB(1, 0)

#### 8.1.2.8	RRC connection establishment / Range of access baring time

8.1.2.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state with T303 running }

ensure that {
  when { UE is requested to make an outgoing call }

then { UE does not transmit any RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received SystemInformationBlockType2 with no restriction }

ensure that {
  when { UE is requested to make an outgoing call }

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to FALSE:

4>	consider access to the cell as not barred;

3>	else if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

4>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of the valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

5>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as not barred;

3>	else:

4>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Data:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	else:

2>	if the UE is establishing the RRC connection for mobile originating calls and if both timers T302 and T303 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T303 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Data:

T303= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

2>	else if the UE is establishing the RRC connection for mobile originating signalling and if both timers T302 and T305 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T305 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Signalling:

T305= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

2>	else if the UE is establishing the RRC connection for emergency calls:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for emergency calls is applicable, upon which the procedure ends;

2>	else:

3>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

8.1.2.8.3	Test description

8.1.2.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in Registered, Idle mode state (state 2) according to [18].

8.1.2.8.3.2	Test procedure sequence

Table 8.1.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including systemInfoModification. (Note 3) | <-- | Paging | - | - |
| 2 | The SS changes SystemInformationBlockType2 parameters to 0% access probability for MO calls. | - | - | - | - |
| 3 | Wait for 15 s for the UE to receive system information. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Cause the UE to request connectivity to an additional PDN. (Note 5) | - | - | - | - |
| 5A | The UE does not transmit any RRCConnectionRequest message (The UE starts T303) (Note 2) | - | - | - | - |
| 5B | Void | - | - | - | - |
| 6 | Cause the UE to request connectivity to an additional PDN. (Note 5) | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionRequest message within 5s? | --> | RRCConnectionRequest | 1 | F |
| 8 | Wait for 20 s to ensure that T303 expires. | - | - | - | - |
| 9 | The SS transmits a Paging message including systemInfoModification (Note 3). | <-- | Paging | - | - |
| 10 | The SS changes SystemInformationBlockType2 parameters to default values (Note 4). | - | - | - | - |
| 11 | The SS starts timer Timer_1 = 20 s. | - | - | - | - |
| - | EXCEPTION: Steps 12a1 to 12 b3 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens.(Note 6) | - | - | - | - |
| 12a1 | Check: Does the UE transmit an RRCConnectionRequest message? SS stops timer Timer_1 | --> | RRCConnectionRequest | 2 | P |
| 12b1 | The SS waits for Timer_1 expiry | - | - | - | - |
| 12 b2 | Cause the UE to request connectivity to an additional PDN. (Note 5) | - | - | - | - |
| 12b3 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 2 | P |
| 13 | Void | - | - | - | - |
| 14-23 | Steps 2a2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.3.2 are performed on Cell 1 | - | - | - | - |
| Note 1: Void. Note 2: The UE is establishing the RRC connection for mobile originating calls, and consider access to the cell as barred, the UE should draw a rand number “rand” in the range 0 ≤ rand < 1, and start T303 with value set to the maximum value: T303= (0.7+ 0.6 * rand) * ac-BarringTime. Note 3: To guarantee that the UE will receive at least one Paging in the Modification Period preceding the SysInfo change, SS should send the Paging message in every eligible PO in this Modification Period. Note 4: The default values refer to TS36.508 table 4.4.3.3-1. Note 5: The triggers in step 5, step 6 and step 12 b2 are the same as in the generic procedure in 36.508 clause 6.4.3.2. Note 6: A UE may send RRCConnectionRequest for the pending procedure triggered at step 6. |  |  |  |  |  |

8.1.2.8.3.3	Specific message contents

Table 8.1.2.8.3.3-1: Paging (step 1 and step 9, Table 8.1.2.8.3.2-1)

| Derivation path: 36.508 table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | True |  |  |
| } |  |  |  |

Table 8.1.2.8.3.3-1A: SystemInformationBlockType1 (step 2, Table 8.1.2.8.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 8.1.2.8.3.3-1B: SystemInformationBlockType1-BR-r13 (step 2 when UE under test is CAT-M1, Table 8.1.2.8.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.8.3.3-2: SystemInformationBlockType2 (step 2, Table 8.1.2.8.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE | For AC 10 |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s16 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s16 |  |  |
| ac-BarringForSpecialAC | '11111'B | For AC 11..15 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.8.3.3-3: SystemInformationBlockType1 (step 10, Table 8.1.2.8.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 2 |  |  |
| } |  |  |  |

Table 8.1.2.8.3.3-4: SystemInformationBlockType1-BR-r13 (step 10 when UE under test is CAT-M1, Table 8.1.2.8.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 2 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=2, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message) |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.2.9	RRC Connection Establishment / 0% access probability for MO calls, non-zero percent access probability for MO signalling

8.1.2.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and has received SystemInformationBlockType2 indicating non zero percent access probability for MO signalling }

then { UE does not transmit any RRCConnectionRequest message when access to the cell is considered as barred and UE transmits an RRCConnectionRequest message when access to the cell is considered as not barred }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received SystemInformationBlockType2 indicating 0% access probability for MO call }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message }

}

8.1.2.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

...

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Data:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	else:

2>	if the UE is establishing the RRC connection for mobile originating calls and if both timers T302 and T303 are not running:

...

2>	else if the UE is establishing the RRC connection for mobile originating signalling and if both timers T302 and T305 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T305 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Signalling:

T305= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

8.1.2.9.3	Test description

8.1.2.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. The preamble is employed in such a way that user initiated PDN disconnection is avoided because the cell 1 is access barred during the preamble. The 2nd PDN, if established, is deactived by the SS.

8.1.2.9.3.2	Test procedure sequence

Table 8.1.2.9.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 11. |

Table 8.1.2.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 3 | The SS initializes internal counters: K to 0 and L to 0. | - | - | - | - |
| - | EXCEPTION: Steps 4 to 5b15 shall be repeated maximum 50 times unless K > 0 and L >0. (Note 1) | - | - | - | - |
| 4 | The SS changes the power level setting according to the row “T1” in table 8.1.2.9.3.2-1. | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5b15 describe behaviours which vary depending on whether the UE transmits an RRCConnectionRequest message or not. | - | - | - | - |
| 5a1 | IF the UE does not transmit any RRCConnectionRequest message within 34 s when K=0 and L=0 and within 8 s for other repeat times, i.e. K ≠0 or L≠0 (Note 2) THEN the SS increments the counter K by 1. | - | - | - | - |
| 5a2 | The SS changes the power level setting according to the row “T2” in table 8.1.2.9.3.2-1. | - | - | - | - |
| 5a3 | The SS waits for 20s for the UE to perform cell reselection procedure. (Note 3) | - | - | - | - |
| 5b1 | ELSE IF the UE transmits an RRCConnectionRequest message on Cell 11 THEN the SS increments the counter L by 1. | --> | RRCConnectionRequest | - | - |
| 5b2-5b6 | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 5b7 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 5b8 | The SS changes the power level setting according to the row “T2” in table 8.1.2.9.3.2-1. | - | - | - | - |
| 5b9 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 5b10-5b14 | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 1. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 5b15 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 6 | Check: Is the counter K > 0 and L > 0? | - | - | 1 | P |
| 7 | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 1. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 8 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 9 | Wait for 1 s after the IP packet has been transmitted in step 8. (Note 4) | - | - | - | - |
| 10 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 11 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 30s? | --> | RRCConnectionRequest | 2 | F |
| - | EXCEPTION: Step 21 and Step 22a1 can happen in any order. | - | - | - | - |
| 12-21 | Check: Does the test result of steps 1 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1,2 | - |
| - | EXCEPTION: Step 22a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 22a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 8 on the DRB associated with the default EPS bearer context on Cell 1 within 10 s. | - | - | - | - |
| Note 1: There is an extremely low probability that the UE originating calls are barred, or not barred, 50 times consecutively. (The probability is 1.7810-15). Therefore, 50 times is enough number of trials for this test case. Note 2: The UE starts T305. Note 3: The UE stops T305 upon cell reselection. Note 4: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 8 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 10. |  |  |  |  |  |

8.1.2.9.3.3	Specific message contents

Table 8.1.2.9.3.3-1: SystemInformationBlockType2 for Cell 1 (preamble and all steps, Table 8.1.2.9.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE {} | Not present |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s512 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9.3.3-2: SystemInformationBlockType2 for Cell 11 (preamble and all steps, Table 8.1.2.9.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p50 |  |  |
| ac-BarringTime | s64 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9.3.3-3: CLOSE UE TEST LOOP (preamble, Table 8.1.2.9.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

Table 8.1.2.9.3.3-4: RRCConnectionRequest (step 5b1 and step 5b9, Table 8.1.2.9.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.2.10	Void

#### 8.1.2.11	Void

NOTE:	The present test (RRC connection establishment of emergency call) is superseded by test case 11.2.1 (i.e. all requirements which the present test verified are verified in test case 11.2.1).

#### 8.1.2.12	Void

NOTE:	The present test (RRC connection establishment of emergency call / Limited Service) is superseded by test case 11.2.2 (i.e. all requirements which the present test verified are verified in test case 11.2.2).

#### 8.1.2.13	RRC connection establishment / 0% access probability for MO calls, 0% access probability for MO signalling

8.1.2.13.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts SystemInformationBlockType2 indicating 0% access probability for MO signalling }

then { UE does not transmit any RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received SystemInformationBlockType2 indicating 0% access probability for MO call and for MO signalling }

ensure that {
  when { UE receives a Paging message }

then { UE transmits an RRCConnectionRequest message }

}

(3)

with { UE in E-UTRA RRC_IDLE state having received SystemInformationBlockType2 indicating 0% access probability for MO call }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message }

}

8.1.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if the UE is establishing the RRC connection for mobile terminating calls:

...

1>	else if the UE is establishing the RRC connection for emergency calls:

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	if timer T302 or T303 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Data is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Data:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

1>	if access to the cell, as specified above, is not barred:

2>	apply the default physical channel configuration as specified in 9.2.4;

2> apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2> apply the default MAC main configuration as specified in 9.2.2;

2> apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

2>	start timer T300;

2>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2: Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	else:

2>	if the UE is establishing the RRC connection for mobile originating calls and if both timers T302 and T303 are not running:

...

2>	else if the UE is establishing the RRC connection for mobile originating signalling and if both timers T302 and T305 are not running:

3>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3>	start timer T305 with the timer value calculated as follows, using the ac-BarringTime included in ac-BarringForMO-Signalling:

T305= (0.7+ 0.6 * rand) * ac-BarringTime

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

8.1.2.13.3	Test description

8.1.2.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18].

8.1.2.13.3.2	Test procedure sequence

Table 8.1.2.13.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 11. |

Table 8.1.2.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 3 | The SS changes the power level setting according to the row “T1” in table 8.1.2.13.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 40s? (Note 1) | --> | RRCConnectionRequest | 1 | F |
| 5 | The SS transmits a Paging message including a matched identity allocated in Cell 1. | <-- | Paging | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 30s? (Note 2) | --> | RRCConnectionRequest | 1 | F |
| 7 | The SS changes the SystemInformationBlockType1 and SystemInformationBlockType2 on Cell 1 according to Table 8.1.2.13.3.3-1. | - | - | - | - |
| 7A | Wait for 2.1* modification period (Note 5) to allow the new system information to take effect. | - | - | - | - |
| 7B | The SS changes the power level setting according to the row “T2” in table 8.1.2.13.3.2-1. | - | - | - | - |
| 8 | The SS waits for 40s the UE to perform cell reselection procedure (Note 3). | - | - | - | - |
| 9 | The SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists on Cell 1. | <-- | Paging | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 2 | P |
| 11-16 | Steps 4 to 9 of the generic test procedure in TS 36.508 subclause 4.5.3.3 are performed on Cell 1. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 17 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 18 | Wait for 1 s after the IP packet has been transmitted in step 17. (Note 4) | - | - | - | - |
| 19 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 20 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 10s? | --> | RRCConnectionRequest | 3 | F |
| - | EXCEPTION: Step 30 and Step 31a1 can happen in any order. | - | - | - | - |
| 21-30 | Check: Does the test result of steps 1 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1,2,3 | - |
| - | EXCEPTION: Step 31a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 31a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 17 on the DRB associated with the default EPS bearer context on Cell 1 within 10 s. | - | - | - | - |
| Note 1: The UE starts T305. Note 2: In case that the UE did not response to paging on Cell 1, it is considered that the UE is camped on Cell 11. Note 3: The UE stops T305 upon cell reselection. Note 4: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 17 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 19. Note 5: The wait time of 2.1* modification period in step 7A is to allow for the network to paging the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. |  |  |  |  |  |

8.1.2.13.3.3	Specific message contents

Table 8.1.2.13.3.3-1: SystemInformationBlockType2 for Cell 1 (step 7, Table 8.1.2.13.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p0 |  |  |
| ac-BarringTime | s8 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data SEQUENCE { |  |  |  |
| ac-BarringFactor | p0 |  |  |
| ac-BarringTime | s512 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.13.3.3-2: SystemInformationBlockType1 for Cell 1 (step 7, Table 8.1.2.13.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 8.1.2.13.3.3-2A: SystemInformationBlockType1-BR-r13 for Cell 1 (step 7 when UE under test is CAT M1, Table 8.1.2.13.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message) |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.13.3.3-3: SystemInformationBlockType2 for Cell 11 (preamble and all steps, Table 8.1.2.13.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p0 |  |  |
| ac-BarringTime | s512 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.13.3.3-4: CLOSE UE TEST LOOP (preamble, Table 8.1.2.13.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

#### 8.1.2.14	RRC connection establishment / High speed flag

8.1.2.14.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received a SystemInformationBlockType2 indicating an IE highSpeedFlag set to true }

ensure that {
  when { UE receives a Paging message }

then { UE establishes an RRC connection }

}

8.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.2.2.9 and 6.3.2.

[TS 36.331, clause 5.2.2.9]

Upon receiving SystemInformationBlockType2, the UE shall:

...

1>	apply the configuration included in the radioResourceConfigCommon;

[TS 36.331, clause 6.3.2]

PRACH-Config field descriptions

…

highSpeedFlag Parameter: High-speed-flag, see TS 36.211, [21, 5.7.2].TRUE corresponds to Restricted set and FALSE to Unrestricted set.

8.1.2.14.3	Test description

8.1.2.14.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

8.1.2.14.3.2	Test procedure sequence

Table 8.1.2.14.3.2-1 illustrates the downlink power levels to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.1.2.14.3.2-2.

Table 8.1.2.14.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 2. |

Table 8.1.2.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 2 level according to the row “T1” in table 8.1.2.14.3.2-1. | - | - | - | - |
| 2 | The SS waits for 40s for the UE to perform cell reselection procedure. | - | - | - | - |
| 3 | The SS transmits a Paging message on Cell 2. | <-- | Paging | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 5-10 | Steps 4 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure on Cell 2. | - | - | - | - |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.1.2.14.3.3	Specific message contents

Table 8.1.2.14.3.3-1: SystemInformationBlockType2 for Cell 2 (preamble and all steps, Table 8.1.2.14.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| prach-Config SEQUENCE { |  |  |  |
| rootSequenceIndex | 86 |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| highSpeedFlag | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.2.15	RRC connection establishment / Extended value, spare fields and non critical extensions in SI

8.1.2.15.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives extended value that is not defined in the version of the transfer syntax supported by the UE and non critical extensions }

then { UE consider the value as not comprehended and treat the message as if the field were absent and UE establishes an RRC connection }

}

8.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3, 5.7.1, 5.7.3 and A3.3.

[TS 36.331 clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_INACTIVE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType25;

2>	if in RRC_CONNECTED; and

2>	the UE is not a BL UE; and

2>	the UE is not in CE; and

2>	the UE is not a NB-IoT UE:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT SystemInformationBlockType22-NB;

1>	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1>	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12 and systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

[TS 36.331 clause 5.7.1]

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved field.

[TS 36.331 clause 5.7.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 5.7.4;

[TS 36.331 clause A3.3]

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the IEs here. --														-- Cond condTag

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING are facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::=					SEQUENCE {

field1									InformationElement1,

field2									InformationElement2,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/ or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

8.1.2.15.3	Test description

8.1.2.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

8.1.2.15.3.2	Test procedure sequence

Table 8.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH and BCCH-BR. NOTE: Spare field testing is not applicable for Rel-12 UE because a spare field in MIB or SIB1 does not exist in Rel-12. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| NOTE: SIB-Type and SIB-Type-v12j0 in SystemInformationBlockType1 and SystemInformationBlockType1-BR are set to values in Rel-17to check spare and extended fields in UE up to Rel-16. |  |  |  |  |  |

8.1.2.15.3.3	Specific message contents

Table 8.1.2.15.3.3-1: Message MasterInformationBlock (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |
| schedulingInfoSIB1-BR-r13 | 10 |  | when UE under test is CAT M1 |
|  | 0 |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | ‘11’B |  |  |
| } |  |  |  |
| spare | ‘1’B | From Rel-8 to Rel-16, the lowest bit is considered spare according to TS 36.331 [17]. | For Rel-8 to Rel-16 |
| } |  |  |  |

Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1 (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-8 to Rel-15 2 entries for Rel-16 |  |
| si-Periodicity[1] | rf16 |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4 |  |  |
| sib-MappingInfo[2] | sibType3 |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-8 to Rel-15 |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type[1] | sibType29-v1610 | From Rel-8 to Rel-15, the field used for sibType29-v1610 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v890-IEs | For Rel-16 |
| lateNonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v9e0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10j0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10l0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10x0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v12j0-IEs |  |
| schedulingInfoExt-r12 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-16, the field used for sibType32-v1700 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v920-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1130-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1250-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1310-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1320-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1350-IEs |  |
|  |  | SystemInformationBlockType1-v1360-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1430-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1450-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1530-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1540-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1610-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1700-IEs |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred | To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (step 1 when UE under test is CAT M1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-13 to Rel-15 2 entries for Rel-16 |  |
| si-Periodicity[1] | rf16 |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4. |  |  |
| sib-Mappinginfo[2] | sibType3 |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-13 to Rel-15 |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  | For Rel-13 to Rel-15 |
| SIB-Type[1] | sibType29-v1610 | From Rel-13 to Rel-15, the field used for sibType29-v1610 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v890-IE0073 | For Rel-16 |
| lateNonCriticalExtension { |  | SystemInformationBlockType1-v8h0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v9e0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10j0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10l0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10x0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v12j0-IEs |  |
| schedulingInfoExt-v12j0 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-16, the field used for sibType32-v1700 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v920-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1130-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1250-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1310-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1320-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1350-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1360-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1430-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1450-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1530-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1540-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1610-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1700-IEs |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred | To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.1.3	RRC connection release

#### 8.1.3.1	Void

NOTE:	The present test (RRC connection release / Success) is superseded by test case 8.2.1.1 (i.e. all requirements which the present test verified are verified in test case 8.2.1.1).

#### 8.1.3.2	Void

#### 8.1.3.3	Void

#### 8.1.3.4	RRC connection release / Redirection to another E-UTRAN frequency

8.1.3.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with eutra different from the frequency UE was on in RRC_CONNECTED state}

then { UE enters RRC_IDLE state on new frequency included in IE redirectedCarrierInfo }

}

8.1.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.8.3, 5.3.12 and TS 36.304, clauses 5.2.4.1, 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.4.1]

…

UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

…

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, UE shall select a suitable cell to camp on according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. Otherwise UE shall select a suitable cell on an EUTRA carrier. If no suitable cell is found, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

...

8.1.3.4.3	Test description

8.1.3.4.3.1	Pre-test conditions

System Simulator:

-	2 cells on different E-UTRA frequencies and different tracking areas:

-	Cell 1 serving cell

-	Cell 23 suitable neighbour inter-frequency cell

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18] on cell 1.

8.1.3.4.3.2	Test procedure sequence

Table 8.1.3.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra of Cell 23). | <-- | RRCConnectionRelease | - | - |
| 2 | Check: does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 23? | - | - | 1 | - |

8.1.3.4.3.3	Specific message contents

Table 8.1.3.4.3.3-1: Conditions for specific message contents in tables 8.1.3.4.3.3-3 and 8.1.3.4.3.3-3A

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.4.3.3-2: Void

Table 8.1.3.4.3.3-3: SystemInformationBlockType5 for cell 1 (preamble and all steps, Table 8.1.3.4.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 23 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 23 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.4.3.3-3A: SystemInformationBlockType5 for cell 23 (preamble and all steps, Table 8.1.3.4.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.4.3.3-4: RRCConnectionRelease message (step 1, Table 8.1.3.4.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 23 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 23 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.5	RRC connection release / Success / With priority information

8.1.3.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message with the freqPriorityListEUTRA with higher priority frequency}

ensure that {
  when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority frequency }

then { UE reselects the cell which belongs to the higher priority frequency }

}

8.1.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.8.3 and TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

...

1>	if the releaseCause received in the RRCConnectionRelease message indicates 'loadBalancingTAURequired':

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE: The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

8.1.3.5.3	Test description

8.1.3.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell6.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: For Cell 3 and 6 TAI is set to TAI-2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.5.3.2	Test procedure sequence

Table 8.1.3.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "off” | "off” | The power level values are assigned to satisfy SrxlevCell 1 > Sintrasearch. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | -73 | The power level values are assigned to satisfy both Threshx, low < SrxlevCell 3 and Threshx, high <SrxlevCell 6. |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.1.3.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the freqPriorityListEUTRA on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1. | - | - | - | - |
| 3 | The SS changes Cell 1, Cell 3 and Cell 6 level according to the row "T1" in table 8.1.3.5.3.2-1. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 6? | - | - | 1 | - |

8.1.3.5.3.3	Specific message contents

Table 8.1.3.5.3.3-0: Conditions for specific message contents
in Tables 8.1.3.5.3.3-1, 8.1.3.5.3.3-2 and 8.1.3.5.3.3-3.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.5.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.5.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 3 entries |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| carrierFreq[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq[2] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[2] | 1 |  |  |
| carrierFreq[3] | Same downlink EARFCN as used for Cell 6 |  |  |
| carrierFreq[3] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[3] | 5 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  |  |
| freqPriorityListUTRA-TDD | Not present |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  | Band > 64 |
| nonCriticalExtension |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq-v9e0[3] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.3.3-2: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[m] | Same downlink EARFCN as used for Cell 6 |  |  |
| dl-CarrierFreq[m] | maxEARFCN |  | Band > 64 |
| threshX-Low[n] | 11 |  |  |
| threshX-High[m] | 11 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-v9e0[m] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.3.3-3: SystemInformationBlockType5 for Cell 6 (all steps, Table 8.1.3.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.5a	RRC connection release / Success / With extended priority information

8.1.3.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message with the freqPriorityListExtEUTRA with higher priority frequency}

ensure that {
  when { UE detects the cell re-selection criteria are met for the cell, which belongs to the higher priority frequency in freqPriorityListExtEUTRA and this cell is the highest ranked cell }

then { UE reselects the cell which belongs to the higher priority frequency }

}

8.1.3.5a.2	Conformance requirements

Same as 8.1.3.5.2 and

[TS 36.331, clause 6.2.2.]

RRCConnectionRelease field descriptions

carrierFreq or bandClass The carrier frequency (UTRA and E-UTRA) and band class (HRPD and 1xRTT) for which the associated cellReselectionPriority is applied.

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

cellInfoList Used to provide system information of one or more cells on the redirected inter-RAT carrier frequency. The system information can be used if, upon redirection, the UE selects an inter-RAT cell indicated by the physCellId and carrierFreq (GERAN and UTRA TDD) or by the physCellId (other RATs). The choice shall match the redirectedCarrierInfo. In particular, E-UTRAN only applies value utra-TDD-r10 in case redirectedCarrierInfo is set to utra-TDD-r10.

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests.

freqPriorityListX Provides a cell reselection priority for each frequency, by means of separate lists for each RAT (including E-UTRA). The UE shall be able to store at least 3 occurrences of FreqsPriorityGERAN. If E-UTRAN includes freqPriorityListEUTRA-v9e0 and/or freqPriorityListEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListEUTRA (i.e. without suffix). Field freqPriorityListExt includes additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. EUTRAN only includes freqPriorityListExtEUTRA if freqPriorityListEUTRA (i.e. without suffix) includes maxFreq entries. If E-UTRAN includes freqPriorityListExtEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListExtEUTRA-r12.

8.1.3.5a.3	Test description

8.1.3.5a.3.1	Pre-test conditions

Same as 8.1.3.5.3.1 and

System Simulator:

-	Cell 14 is a non used cell.

8.1.3.5a.3.2	Test procedure sequence

Same as 8.1.3.5.3.2 except

Table 8.1.3.5a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the freqPriorityListExtEUTRA-r12 on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2-5 | Same as in table 8.1.3.5.3.2-2. | - | - | - | - |

8.1.3.5a.3.3	Specific message contents

Same as 8.1.3.5.3.3 except

Table 8.1.3.5a.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.5a.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq[1] | EARFCN of Cell 14 | Non used cell |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| carrierFreq[2] | EARFCN of Cell 14 +1 | Non used cell |  |
| carrierFreq[2] | maxEARFCN |  | Band > 64 |
| carrierFreq[3] | EARFCN of Cell 14 +2 | Non used cell |  |
| carrierFreq[3] | maxEARFCN |  | Band > 64 |
| carrierFreq[4] | EARFCN of Cell 14 +3 | Non used cell |  |
| carrierFreq[4] | maxEARFCN |  | Band > 64 |
| carrierFreq[5] | EARFCN of Cell 14 +4 | Non used cell |  |
| carrierFreq[5] | maxEARFCN |  | Band > 64 |
| carrierFreq[6] | EARFCN of Cell 14 +5 | Non used cell |  |
| carrierFreq[6] | maxEARFCN |  | Band > 64 |
| carrierFreq[7] | EARFCN of Cell 14 +6 | Non used cell |  |
| carrierFreq[7] | maxEARFCN |  | Band > 64 |
| carrierFreq[8] | EARFCN of Cell 14 +7 | Non used cell |  |
| carrierFreq[8] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  |  |
| freqPriorityListUTRA-TDD | Not present |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| freqPriorityListExtEUTRA-r12 SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-r12[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| cellReselectionPriority-r12[1] | 4 |  |  |
| carrierFreq-r12[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| cellReselectionPriority-r12[2] | 1 |  |  |
| carrierFreq-r12[3] | Same downlink EARFCN as used for Cell 6 |  |  |
| cellReselectionPriority-r12[3] | 5 |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  | Band > 64 |
| nonCriticalExtension |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | EARFCN of Cell 14 | Non used cell |  |
| carrierFreq-v9e0[2] | EARFCN of Cell 14 + 1 | Non used cell |  |
| carrierFreq-v9e0[3] | EARFCN of Cell 14 + 2 | Non used cell |  |
| carrierFreq-v9e0[4] | EARFCN of Cell 14 + 3 | Non used cell |  |
| carrierFreq-v9e0[5] | EARFCN of Cell 14 + 4 | Non used cell |  |
| carrierFreq-v9e0[6] | EARFCN of Cell 14 + 5 | Non used cell |  |
| carrierFreq-v9e0[7] | EARFCN of Cell 14 + 6 | Non used cell |  |
| carrierFreq-v9e0[8] | EARFCN of Cell 14 + 7 | Non used cell |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.6	RRC connection release / Redirection from E-UTRAN to UTRAN

8.1.3.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectionInformation with utra-CarrierFreq UTRA frequency }

then { UE enters RRC_IDLE state on UTRA frequency included in IE redirectionInformation }

}

8.1.3.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.8.3, 5.3.12 and TS 36.304, clause 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, UE shall select a suitable cell to camp on according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. Otherwise UE shall select a suitable cell on an EUTRA carrier.

...

8.1.3.6.3	Test description

8.1.3.6.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell and different location areas:

-	Cell 1 E-UTRA serving cell

-	Cell 5 suitable neighbour UTRA cell

-	The parameters settings and power levels for Cell 1 and Cell 5 are selected according to [18], [5] and Table 8.1.3.6.3.2-0 so that camping on Cell 1 is guaranteed and no cell re-selection to Cell 5 can take place (utra priority is lower than serving and S Cell 1 > Thresh Cell 1,low).

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.1.3.6.3.2	Test procedure sequence

Table 8.1.3.6.3.2-0 shows the cell power levels after the preamble.

Table 8.1.3.6.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
|  | CPICH_Ec | dBm/3.84 MHz | - | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -72 |  |

Table 8.1.3.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectionInformation including UTRA-CarrierFreq of Cell 5). | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 1 | - |
| 3a1-3a3Aa7a2 | Void | - | - | - | - |

8.1.3.6.3.3	Specific message or IE contents

Table 8.1.3.6.3.3-1: Void

Table 8.1.3.6.3.3-2: Void

Table 8.1.3.6.3.3-3: SystemInformationBlockType6 for cell 1 (preamble and all steps, Table 8.1.3.6.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6.3.3-4 RRCConnectionRelease message (step 1, Table 8.1.3.6.3.2-1)

| Derivation Path: 36.508 table 4.6.1.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6.3.3-5: System Information Block type 19 for Cell 5 (preamble and all steps, Table 8.1.3.6.3.2-1)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| Utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.6a	RRC connection release / Redirection from E-UTRAN to UTRAN / Pre-redirection info

8.1.3.6a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with utra-CarrierFreq UTRA frequency }

then { UE sends an RRC CONNECTION REQUEST message including IE Pre-redirection info set to FALSE on UTRA frequency included in IE redirectionInformation and enters RRC_IDLE state on that UTRA frequency }

}

8.1.3.6a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.8.3, 5.3.12 and TS 36.304, clause 5.2.7 and TS 25.331, clause 8.1.3.3, 10.2.39 with additional clarification in R2-105061.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, UE shall select a suitable cell to camp on according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. Otherwise UE shall select a suitable cell on an EUTRA carrier.

[TS 25.331, clause 8.1.3.3]

...

1>	if the UE supports E-UTRA:

2>	if the UE is attempting to establish the signalling connection as a result of being redirected by E-UTRA; and

2>	if this is the first attempt to establish the signalling connection;

3>	do not include the IE "Pre-Redirection info";

[TS 25.331, clause 10.2.39]

| Pre-redirection info | OP |  | Pre-redirection info 10.3.3.25a | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to, and that the UE was not redirected from EUTRA. | REL-8 |
| --- | --- | --- | --- | --- | --- |

8.1.3.6a.3	Test description

8.1.3.6a.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell and different location areas:

-	Cell 1 E-UTRA serving cell

-	Cell 5 suitable neighbour UTRA cell

-	The parameters settings and power levels for Cell 1 and Cell 5 are selected according to [18], [5] and Table 8.1.3.6a.3.2-0 so that camping on Cell 1 is guaranteed and no cell re-selection to Cell 5 can take place (utra priority is lower than serving and S Cell 1 > Thresh Cell 1,low).

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.1.3.6a.3.2	Test procedure sequence

Table 8.1.3.6a.3.2-0 shows the cell power levels after the preamble.

Table 8.1.3.6a.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
|  | CPICH_Ec | dBm/3.84 MHz | - | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -72 |  |

Table 8.1.3.6a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including UTRA-CarrierFreq of Cell 5). | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 1 | - |

8.1.3.6a.3.3	Specific message or IE contents

Table 8.1.3.6a.3.3-1: SystemInformationBlockType6 for cell 1 (preamble and all steps, Table 8.1.3.6a.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6a.3.3-2 RRCConnectionRelease message (step 1, Table 8.1.3.6a.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellInfoList-r9 ::= CHOICE {} | Not present | redirection with SIB is optional |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6a.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 8.1.3.6a.3.2-1)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| Utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.6a.3.3-4 RRC CONNECTION REQUEST (step 2, Table 8.1.3.6a.3.2-1)

| Derivation Path: 36.508 table 4.7B.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Pre-redirection info | Not present | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to, and that the UE was not redirected from EUTRA |  |

#### 8.1.3.7	RRC connection release / Redirection from UTRAN to E-UTRAN

8.1.3.7.1	Test Purpose (TP)

(1)

with { UE in UTRA Idle state }

ensure that {

when { UE is requested to make an outgoing PS call }

then { UE includes in the RRC CONNECTION REQUEST the IE Pre-Redirection info" }

}

(2)

with { UE in UTRA CELL_DCH state }

ensure that {

when { UE receives an RRC CONNECTION RELEASE message including an IE Redirection info with E-UTRA target info E-UTRA frequency }

then { UE enters RRC_IDLE state on E-UTRAN Carrier included in IE Redirection info }

}

8.1.3.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 25.331, clause  8.1.3.3, 8.1.4.3 and clause 8.5.2.

[TS 25.331, clause 8.1.3.3]

The UE shall, in the transmitted RRC CONNECTION REQUEST message:

…

1>	if the UE supports E-UTRA:

2>	if the variable EUTRA_FREQUENCY_INFO_LIST contains no E-UTRA frequencies;

3>	include the IE "Pre-Redirection info";

3>	if the UE supports E-UTRA FDD:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports E-UTRA TDD:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

2>	if the UE supports any of the bands that the E-UTRA frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to;

3>	include the IE "Pre-Redirection info";

3>	if the UE supports any of the bands that the E-UTRA FDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports any of the bands that the E-UTRA TDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

[TS 25.331, clause 8.1.4.3]

The UE shall receive and act on an RRC CONNECTION RELEASE message in states CELL_DCH and CELL_FACH and CELL_PCH (FDD only). Furthermore this procedure can interrupt any ongoing procedures with the UE in the above listed states.

When the UE receives the first RRC CONNECTION RELEASE message; and

1>	if the message is received on the CCCH, and IE "U-RNTI" is present and has the same value as the variable U_RNTI; or

1>	if the message is received on DCCH:

the UE shall perform the RRC connection release procedure as specified below.

When the UE receives the first RRC CONNECTION RELEASE message; and

1>	if the message is received on the CCCH, the IE "UTRAN group identity" is present and there is a group identity match according to subclause 8.6.3.13:

the UE shall perform the RRC connection release procedure as specified below.

The UE shall:

1>	in state CELL_DCH:

2>	initialise the counter V308 to zero;

2>	set the IE "RRC transaction identifier" in the RRC CONNECTION RELEASE COMPLETE message to the value of "RRC transaction identifier" in the entry for the RRC CONNECTION RELEASE message in the table "Accepted transactions" in the variable TRANSACTIONS;

2>	submit an RRC CONNECTION RELEASE COMPLETE message to the lower layers for transmission using UM RLC on the DCCH to the UTRAN;

2>	if the IE "Rplmn information" is present:

3>	the UE may:

4>	store the IE on the ME together with the PLMN id for which it applies;

3>	the UE may then:

4>	utilise this information, typically indicating where a number of BCCH frequency ranges of a RAT may be expected to be found, during subsequent Rplmn selections of the indicated PLMN.

2>	if the IE "UE Mobility State Indicator" is present:

3>	consider the High-mobility state to have being detected when entering idle mode.

2>	start timer T308 when the RRC CONNECTION RELEASE COMPLETE message is sent on the radio interface.

1>	in state CELL_FACH:

2>	if the RRC CONNECTION RELEASE message was received on the DCCH:

3>	set the IE "RRC transaction identifier" in the RRC CONNECTION RELEASE COMPLETE message to	the value of "RRC transaction identifier" in the entry for the RRC CONNECTION RELEASE message in the table "Accepted transactions" in the variable TRANSACTIONS;

3>	submit an RRC CONNECTION RELEASE COMPLETE message to the lower layers for transmission using AM RLC on the DCCH to the UTRAN.

3>	when the successful transmission of the RRC CONNECTION RELEASE COMPLETE message has been confirmed by the lower layers:

4>	release all its radio resources; and

4>	indicate the release of the established signalling connections (as stored in the variable ESTABLISHED_SIGNALLING_CONNECTIONS) and established radio access bearers (as stored in the variable ESTABLISHED_RABS) to upper layers; and

4>	clear any entry for the RRC CONNECTION RELEASE message in the tables "Accepted transactions" and "Rejected transactions" in the variable TRANSACTIONS;

4>	clear the variable ESTABLISHED_SIGNALLING_CONNECTIONS;

4>	clear the variable ESTABLISHED_RABS;

4>	pass the value of the IE "Release cause" received in the RRC CONNECTION RELEASE message to upper layers;

4>	enter idle mode;

4>	perform the actions specified in subclause 8.5.2 when entering idle mode.

3>	and the procedure ends.

2>	if the RRC CONNECTION RELEASE message was received on the CCCH:

3>	release all its radio resources;

3>	indicate the release of the established signalling connections (as stored in the variable ESTABLISHED_SIGNALLING_CONNECTIONS) and established radio access bearers (as stored in the variable ESTABLISHED_RABS) to the upper layers;

3>	clear any entry for the RRC CONNECTION RELEASE message in the tables "Accepted transactions" and "Rejected transactions" in the variable TRANSACTIONS;

3>	clear the variable ESTABLISHED_SIGNALLING_CONNECTIONS;

3>	clear the variable ESTABLISHED_RABS;

3>	pass the value of the IE "Release cause" received in the RRC CONNECTION RELEASE message to upper layers;

3>	enter idle mode;

3>	perform the actions specified in subclause 8.5.2 when entering idle mode;

3>	and the procedure ends.

[TS 25.331, clause 8.5.2]

When entering idle mode from connected mode, the UE shall:

1>	clear or set variables upon leaving UTRA RRC connected mode as specified in subclause 13.4;

1>	if the RRC CONNECTION RELEASE message was received and the IE "Redirection info" was present therein:

2>	if the IE "Frequency info" is present, attempt to camp on a suitable cell on the indicated UTRA carrier included in the RRC CONNECTION RELEASE message;

2>	if the IE "GSM target cell info" is present, attempt to camp on a suitable cell of the list of cells for the indicated RAT included in the RRC CONNECTION RELEASE message. If no cells were indicated for that RAT or no suitable cell of the indicated cells for that RAT is found within 10s, attempt to camp on any suitable cell of that RAT; or

2>	if the IE "E-UTRA target info" is present, attempt to camp on any of the frequencies for the indicated RAT included in the RRC CONNECTION RELEASE message, excluding any cell indicated in the list of not allowed cells for that RAT, if present. If no suitable cell on the indicated frequencies for that RAT is found within 10s, attempt to camp on any suitable cell of that RAT, excluding any cell indicated in the list of not allowed cells for that RAT; or

2>	if no suitable cell is found on the indicated UTRA carrier or RAT camp on any suitable cell.

8.1.3.7.3	Test description

8.1.3.7.3.1	Pre-test conditions

System Simulator:

-	2 cells, one UTRA and one E-UTRA cell:

-	Cell 5 UTRA serving cell (priority 4 default)

-	Cell 1 suitable neighbour E-UTRA cell (priority 3)

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

UTRAN Idle state

Preamble:

The UE is in state Registered, Idle mode according to section 4.5A.3A [18] on Cell 5 (serving cell)..

8.1.3.7.3.2	Test procedure sequence

Table 8.1.3.7.3.2-0 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.1.3.7.3.2-1.

Table 8.1.3.7.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering condition for event 3c is satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.1.3.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE initiate an outgoing PS call. | - | - | - | - |
| 2 | Check: Does the UE include the IE Pre-redirection info with Support of E-UTRA set to TRUE? | --> | RRC CONNECTION REQUEST | 1 | P |
| 3 | The SS transmits a RRC CONNECTION SETUP on SRB1 on Cell 5. | <-- | RRC CONNECTION SETUP | - | - |
| 4 | The UE transmits an RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 5 | The UE transmits the SERVICE REQUEST message for PS data call | --> | INITIAL DIRECT TRANSFER | - | - |
| - | EXCEPTION: Steps 6a1 and 6a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. UTRAN FDD: IF pc_UTRA_CompressedModeRequired and pc_UTRA_FeatrGrp_2 THEN Steps 6a1, 6a2 UTRAN TDD: go to step 6Aa1 |  |  |  |  |
| 6a1 | The SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 6a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| - | EXCEPTION: Steps 6Aa1 and 6Aa2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. IF pc_UTRA_FeatrGrp_2 THEN 6Aa1 to 6Aa2. |  |  |  |  |
| 6Aa1 | The SS configures the Inter-RAT measurement with E-UTRA frequency list including the DL Carrier frequency of Cell 1 | <-- | MEASUREMENT CONTROL | - | - |
| 6Aa2 | The SS changes Cell 1 and Cell 5 power level according to the row "T1" in table 8.1.3.7.3.2-0. Note: The UE starts sending MEASUREMENT REPORT messages during the Authentication Phase | - | - | - | - |
| 7-9 | Void |  |  |  |  |
| 10 | The SS starts the Authentication and Ciphering Request procedure | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 11 | The UE transmits the Authentication and Ciphering Response | --> | UPLINK DIRECT TRANSFER | - | - |
| 12 | The SS transmits a SECURITY MODE COMMAND message. EXCEPTION: Step 13A and Step 14a can occur in any order. | <-- | SECURITY MODE COMMAND | - | - |
| 13 | The UE transmits a SECURITY MODE COMPLETE message | --> | SECURITY MODE COMPLETE | - | - |
| 13A | The UE transmits an ACTIVATE PDP CONTEXT REQUEST message NOTE: Retransmission of this message is ignored | --> | UPLINK DIRECT TRANSFER |  |  |
| - | EXCEPTION: Step 14a describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. |  |  |  |  |
| 14a | IF pc_UTRA_FeatrGrp_2 THEN if MEASUREMENT REPORT is received during the authentication and security phase (from step 10 to 13) goto next step otherwise the SS waits for MEASUREMENT REPORT message. | --> | MEASUREMENT REPORT | - | - |
| 15 | The SS transmits an RRC CONNECTION RELEASE message (IE E-UTRA target info including DL Carrier frequency of Cell 1). | <-- | RRC CONNECTION RELEASE | - | - |
| 16 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 17 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| - | EXCEPTION : Step 18a1 to 18b6 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN | - | - | - | - |
| 18a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |
| 18b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |
| 18b2 | The UE transmits a RRCConnectionRequest message on E-UTRAN Cell1 | --> | RRC: RRCConnectionRequest | - | - |
| 18b3 | SS transmit an RRCConnectionSetup message | <-- | RRC: RRCConnectionSetup | - | - |
| 18b4 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a EMM SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | - | - |
| 18b5 | SS transmits a SERVICE REJECT with cause Protocol error, unspecified | <-- | SERVICE REJECT | - | - |
| 18b6 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| NOTE: pc_UTRA_FeatrGrp_2 is always set to TRUE when pc_UTRA_CompressedModeRequired is TRUE |  |  |  |  |  |

8.1.3.7.3.3	Specific message or IE contents

Table 8.1.3.7.3.3-0: Conditions for specific message contents
in Table 8.1.3.7.3.3-1

| Condition | Explanation |
| --- | --- |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 8.1.3.7.3.3-1: System Information Block type 19 for Cell 5 (preamble and all steps, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508 table Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry |  |  |
| earfcn[1] | Downlink EARFCN of Cell 1 |  |  |
| priority[1] | 3 |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| earfcn [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [1] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.7.3.3-2: RRC CONNECTION REQUEST (step 2, Table 8.1.3.7.3.2-1)

| Derivation path: 34.108 default RRC CONNECTION REQUEST in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info |  | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to |  |
| Support of E-UTRA FDD | TRUE |  | E-UTRA-FDD |
| Support of E-UTRA TDD | TRUE |  | E-UTRA-TDD |

Table 8.1.3.7.3.3-3: MEASUREMENT CONTROL (step 6Aa1, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508, clause 4.7B.1 Table 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CHOICE Measurement type | Inter-RAT measurement |  |  |
| CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| Parameters required for each event | 1 entry |  |  |
| Inter-RAT event identity | 3c |  |  |
| Threshold own system | Not present |  |  |
| W | Not present |  |  |
| DPCH Compressed mode status info |  |  | UTRA CM-Support |
| TGPS reconfiguration CFN | (Current CFN + (256 – TTI/10msec))mod 256 |  |  |
| Transmission gap pattern sequence | 1 entry |  |  |
| TGPSI | 1 |  |  |
| TGPS Status Flag | activate |  |  |
| TGCFN | (Current CFN + (256 – TTI/10msec))mod 256 |  |  |
| DPCH Compressed mode status info | Not Present |  | UTRA CM-Not Supported |

| Condition | Explanation |
| --- | --- |
| UTRA CM-Supported | UTRAN Compressed Mode configuration supported |
| UTRA CM-Not Supported | UTRAN Compressed Mode configuration not supported |

Table 8.1.3.7.3.3-4: RRC CONNECTION RELEASE (step 15, Table 8.1.3.7.3.2-1)

| Derivation path: 34.108 default RRC CONNECTION RELEASE in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| N308 | 1 |  |  |
| Redirection info |  |  |  |
| Frequency info | Omitted |  |  |
| Inter-RAT info | E-UTRA |  |  |
| E-UTRA target info |  |  |  |
| E-UTRA Target Frequency Info List | 1 Entry |  |  |
| FDD |  |  | E-UTRA-FDD |
| DL Carrier frequency | The DL Carrier frequency of Cell 1 |  |  |
| Exclude-listed cells per freq list | Omitted |  |  |
| DL Carrier frequency | 65535 |  | Band > 64 |
| TDD |  |  | E-UTRA-TDD |
| DL Carrier frequency | The DL Carrier frequency of Cell 1 |  |  |
| Exclude-listed cells per freq list | Omitted |  |  |
| DL Carrier frequency | 65535 |  | Band > 64 |
| E-UTRA Target Frequency Info extension List |  |  | Band > 64 |
| EARFCN extension | EARFCN of the downlink Cell 1 carrier frequency |  |  |

Table 8.1.3.7.3.3-5: PHYSICAL CHANNEL RECONFIGURATION  (step 6a1, Table 8.1.3.7.3.2-1)

| Derivation path: 36.508, clause 4.7B.1 Table 4.7B.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CHOICE UL/DL Mode | UL and DL, UL only or DL only (depending on the UE capability) |  |  |
| Downlink compressed mode method | SF/2(or not sent, depending on the UE capability) |  |  |
| Uplink compressed mode method | SF/2(or not sent, depending on the UE capability) |  |  |

Table 8.1.3.7.3.3-6: SERVICE REJECT (step 18b5, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508 table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | ‘01101111’B | Protocol error, unspecified |  |

#### 8.1.3.8	RRC connection release / Redirection from E-UTRAN to GERAN

8.1.3.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo set to geran }

then { UE enters Idle mode on GERAN Cell included in IE redirectedCarrierInfo  }

}

8.1.3.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.8.3, 5.3.12 and TS 36.304, clause 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, a UE shall attempt to camp on the last cell for which it was in RRC_CONNECTED or any cell on a frequency or frequency of RAT assigned by RRC in the state transition message.

...

8.1.3.8.3	Test description

8.1.3.8.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one GERAN cell and different location areas:

-	Cell 1 E-UTRA serving cell (priority 4 default)

-	Cell 24 GERAN neighbouring cell (priority 3)

-	The parameters settings and power levels for Cell 1 and Cell 24 are selected according to [18], [5], [25] and Table 8.1.3.8.3.2-0 so that camping on Cell 1 is guaranteed and no cell re-selection to Cell 24 can take place (GERAN priority is lower than serving and S Cell 1 > Thresh Cell 1,low)

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.1.3.8.3.2	Test procedure sequence

Table 8.1.3.6.3.2-0 shows the cell power levels after the preamble.

Table 8.1.3.8.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
|  | GERAN Cell Power | dBm | - | -70 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power levels are such that reselection back to cell 1 should not occur |
|  | GERAN Cell Power | dBm |  | -70 |  |

Table 8.1.3.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including startingARFCN of Cell 24). | <-- | RRCConnectionRelease | - | - |
| 1A | The UE sends a CHANNEL REQUEST message on the cell 24 | --> | CHANNEL REQUEST | - | - |
| 1B | An uplink TBF is established in order to allow the UE to transmit a ROUTING AREA UPDATE REQUEST message signalling. | - | - | - | - |
| 1C | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATING REQUEST | - | - |
| 1D | The SS changes cell 1 level according to the row “T1” in table 8.1.3.8.3.2-0 |  |  |  |  |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 steps 4-11 indicate that the UE is camped on GERAN Cell 24? NOTE: The UE completes the RAU procedure. | - | - | 1 | - |

8.1.3.8.3.3	Specific message or IE contents

Table 8.1.3.8.3.3-1: Void

Table 8.1.3.8.3.3-2: SystemInformationBlockType7 for cell 1 (preamble and all steps, Table 8.1.3.8.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqs SEQUENCE [n] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 24 |  |  |
| numberOfFollowingARFCNs | Same as cell 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.8.3.3-3: RRCConnectionRelease message (step 1, Table 8.1.3.8.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| geran SEQUENCE { |  |  |  |
| startingARFCN | Downlink ARFCN of Cell 24 |  |  |
| bandIndicator | The same band indicator of the Cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | 0 Entries |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.9	RRC connection release / Redirection from E-UTRAN to CDMA2000-HRPD

8.1.3.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo set to cdma2000-HRPD }

then { UE enters cdma2000-HRPD mode }

}

8.1.3.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.8.3, 5.3.12 and TS 36.304, clause 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	use the idle mobility parameters broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, a UE shall attempt to camp on the last cell for which it was in RRC_CONNECTED or any cell on a frequency or frequency of RAT assigned by RRC in the state transition message.

...

8.1.3.9.3	Test description

8.1.3.9.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one HRPD cell:

-	Cell 1 E-UTRA serving cell

-	Cell 15 suitable neighbour HRPD cell

-	The parameters settings and power levels for Cell 1 and Cell 15 are selected according to [18], [xx] and Table 8.1.3.9.3.2-1 so that camping on Cell 1 is guaranteed and no cell re-selection to Cell 15 can take place (hrpd priority is lower than serving and S Cell 1 > Thresh Cell 1,low).

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

Note: UE is not pre-registered in Cell 15.

8.1.3.9.3.2	Test procedure sequence

Table 8.1.3.9.3.2-1 shows the cell power levels after the preamble.

Table 8.1.3.9.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 15 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
|  | Îor/Ioc | dB | - | -5 |  |
|  | Ioc | dBm/1.23 MHz | - | -55 |  |
|  | Pilot Ec /Io (Note 1) | dB | - | -6 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 8.1.3.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including cdma2000-HRPD of Cell 15). | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the UE transmit an Access Probe on Cell 15 to initiate registration? | --> | Access Probe | 1 | P |

8.1.3.9.3.3	Specific message or IE contents

Table 8.1.3.9.3.3-1: Void

Table 8.1.3.9.3.3-2: RRCConnectionRelease (step 1, Table 8.1.3.9.3.2-2)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| cdma2000-HRPD | cdma2000-CarrierInfo for Cell 15 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.10	RRC connection release / Redirection from E-UTRAN to CDMA2000-1xRTT

8.1.3.10.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo set to cdma2000-1xRTT }

then { UE enters cdma2000-1xRTT mode }

}

8.1.3.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.8.3, 5.3.12 and TS 36.304, clause 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, a UE shall attempt to camp on the last cell for which it was in RRC_CONNECTED or any cell on a frequency or frequency of RAT assigned by RRC in the state transition message.

...

8.1.3.10.3	Test description

8.1.3.10.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one 1xRTT cell:

-	Cell 1 E-UTRA serving cell

-	Cell 19 suitable neighbour 1xRTT cell

-	The parameters settings and power levels for Cell 1 and Cell 19 are selected according to [18] and Table 8.1.3.10.3.2-1 so that camping on Cell 1 is guaranteed and no cell re-selection to Cell 19 can take place (1xRTT priority is lower than serving and S Cell 1 > Thresh Cell 1,low).

-	System information combination 6 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

- 	UE is not pre-registered in Cell 19.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.1.3.10.3.2	Test procedure sequence

Table 8.1.3.10.3.2-1 shows the cell power levels after the preamble.

Table 8.1.3.10.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
|  | Îor/Ioc | dB | - | 0 |  |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | CPICH_Ec/Io (Note 1) | dB | - | -10 |  |
| Note 1: This parameter is not directly settable, but is derived by calculation from the other parameters set by the SS. |  |  |  |  |  |

Table 8.1.3.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the UE transmit an Access Probe on Cell 19 to initiate registration? | --> | Access Probe | 1 | P |

8.1.3.10.3.3	Specific message or IE contents

Table 8.1.3.10.3.3-1: Void

Table 8.1.3.10.3.3-2: RRCConnectionRelease (step 1, Table 8.1.3.10.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| cdma2000-1xRTT | cdma2000-CarrierInfo for Cell 19 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.11	RRC connection release / Redirection to another E-UTRAN band

8.1.3.11.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with E-UTRA CarrierFreq on different E-UTRAN band }

then { UE enters RRC_IDLE state on new E-UTRA frequency included in IE redirectedCarrierInfo }

}

8.1.3.11.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.8.3, 5.3.12 and TS 36.304, clauses 5.2.4.1, 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.4.1]

…

UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

…

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, UE shall select a suitable cell to camp on according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. Otherwise UE shall select a suitable cell on an EUTRA carrier. If no suitable cell is found, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

...

8.1.3.11.3	Test description

8.1.3.11.3.1	Pre-test conditions

System Simulator:

-	2 cells on different E-UTRA frequencies and different tracking areas:

-	Cell 1 serving cell

-	Cell 10 suitable neighbour inter-band cell

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18] on cell 1.

8.1.3.11.3.2	Test procedure sequence

Table 8.1.3.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra of Cell 10). | <-- | RRCConnectionRelease | - | - |
| 2 | Check: does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |

8.1.3.11.3.3	Specific message contents

Table 8.1.3.11.3.3-1: Void

Table 8.1.3.11.3.3-2: SystemInformationBlockType5 for cell 1 (preamble and all steps, Table 8.1.3.11.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Operating band for inter-band cells > 64 |
| cellReselectionPriority[n] | 3 |  | Cell 1 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band for inter-band cells > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.11.3.3-2A: SystemInformationBlockType5 for cell 10 (preamble and all steps, Table 8.1.3.11.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Operating band under test > 64 |
| cellReselectionPriority[n] | 3 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band under test > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.11.3.3-3: RRCConnectionRelease message (step 1, Table 8.1.3.11.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 10 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Operating band for inter-band cells > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Operating band for inter-band cells > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.11a	RRC connection release / Redirection to another E-UTRAN band / Between FDD and TDD

8.1.3.11a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA FDD RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with E-UTRA CarrierFreq on different E-UTRAN TDD band }

then { UE enters RRC_IDLE state on new E-UTRA TDD frequency included in IE redirectedCarrierInfo }

}

(2)

with { UE in E-UTRA TDD RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with E-UTRA CarrierFreq on different E-UTRAN FDD band }

then { UE enters RRC_IDLE state on new E-UTRA FDD frequency included in IE redirectedCarrierInfo }

}

8.1.3.11a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.8.3, 5.3.12 and TS 36.304, clauses 5.2.4.1, 5.2.7.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

…

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

…

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

…

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

[TS 36.304, clause 5.2.4.1]

…

UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

…

[TS 36.304, clause 5.2.7]

On transition from RRC_CONNECTED to RRC_IDLE, UE shall select a suitable cell to camp on according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. Otherwise UE shall select a suitable cell on an EUTRA carrier. If no suitable cell is found, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

...

8.1.3.11a.3	Test description

8.1.3.11a.3.1	Pre-test conditions

System Simulator:

-	2 cells on different E-UTRA bands which are in different E-UTRA mode (E-UTRA FDD and E-UTRA TDD) and PLMNs, both PLMNs are UE’s EPLMN.

-	Cell 1 is  FDD cell and Cell 28 is TDD cell

-

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 8.1.3.11a.3.1-1.

Table 8.1.3.11a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 28 | PLMN2 |

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

None.

Preamble:

- The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.1.3.11a.3.3-3.

-	The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.1.3.11a.3.2	Test procedure sequence

Table 8.1.3.11a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra CarrierFreq of Cell 28. | <-- | RRCConnectionRelease | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 28? NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | 1 | - |
| 3 | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed to make the UE enter state Generic RB Established (state 3) on Cell 28. | - | - | - | - |
| 4 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra CarrierFreq of Cell 1). | <-- | RRCConnectionRelease | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The TAU is accepted with PLMN2 listed as an Equivalent PLMN | - | - | 2 | - |

.

8.1.3.11a.3.3	Specific message contents

Table 8.1.3.11a.3.3-0: Void

Table 8.1.3.11a.3.3-1: RRCConnectionRelease message (step 1, Table 8.1.3.11a.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of Cell 28 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Operating band for inter-band cells > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Operating band for inter-band cells > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 28 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.11a.3.3-2: RRCConnectionRelease message (step 4, Table 8.1.3.11a.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 1 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Operating band under test > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Operating band under test > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.11a.3.3-3: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.1.3.11a.3.3-4: TRACKING AREA UPDATE ACCEPT for Cell 28 (step 2, Table 8.1.3.11a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.1.3.11a.3.3-5: TRACKING AREA UPDATE ACCEPT for Cell 1 (step 5, Table 8.1.3.11a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.1.3.12	RRC connection release / Success / With priority information / Inter-band

8.1.3.12.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message with the freqPriorityListEUTRA with higher priority frequency in different band}

ensure that {
  when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority band }

then { UE reselects the cell which belongs to the higher priority band}

}

8.1.3.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.8.3 and TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

...

1>	if the releaseCause received in the RRCConnectionRelease message indicates 'loadBalancingTAURequired':

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE 1:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE 2: The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

8.1.3.12.3	Test description

8.1.3.12.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 10.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE 3: For Cell 10 TAI is set to TAI-2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.12.3.2	Test procedure sequence

Table 8.1.3.12.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.12.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "off” | "off” | The power level values are assigned to satisfy SrxlevCell 1 > Snonintrasearch. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | -73 | The power level values are assigned to satisfy both Threshx, low < SrxlevCell 3 and Threshx, high <SrxlevCell 10. |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.1.3.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the freqPriorityListEUTRA on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1. | - | - | - | - |
| 3 | The SS changes Cell 1, Cell 3 and Cell 10 level according to the row "T1" in table 8.1.3.12.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |

8.1.3.12.3.3	Specific message contents

Table 8.1.3.12.3.3-0: Void

Table 8.1.3.12.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.12.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 3 entries |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Operating band under test > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| carrierFreq[2] | Same downlink EARFCN as used for Cell 10 |  |  |
| carrierFreq[2] | maxEARFCN |  | Operating band for inter-band cells > 64 |
| cellReselectionPriority[2] | 5 |  |  |
| carrierFreq[3] | Same downlink EARFCN as used for Cell 3 |  |  |
| carrierFreq[3] | maxEARFCN |  | Operating band under test > 64 |
| cellReselectionPriority[3] | 1 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  |  |
| freqPriorityListUTRA-TDD | Not present |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  | Operating band under test > 64 or Operating band for inter-band cells > 64 |
| nonCriticalExtension |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  | Operating band under test > 64 |
| carrierFreq-v9e0[1] | Not Present |  |  |
| carrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 10 |  | Operating band for inter-band cells > 64 |
| carrierFreq-v9e0[2] | Not present |  |  |
| carrierFreq-v9e0[3] | Same downlink EARFCN as used for Cell 3 |  | Operating band under test > 64 |
| carrierFreq-v9e0[3] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.12.3.3-2: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Operating band under test > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Operating band for inter-band cells > 64 |
| threshX-Low[1] | 11 |  |  |
| threshX-High[2] | 11 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band under test > 64 or Operating band for inter-band cells > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  | Operating band under test > 64 |
| dl-CarrierFreq-v9e0[1] | Not present |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 10 |  | Operating band for inter-band cells > 64 |
| dl-CarrierFreq-v9e0[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.12.3.3-3: SystemInformationBlockType5 for Cell 10 (all steps, Table 8.1.3.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Operating band under test > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Operating band under test > 64 |
| cellReselectionPriority [2] | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band under test > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.12a	RRC connection release / Success / With priority information / Inter- band / Between FDD and TDD

8.1.3.12a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in LTE FDD mode having received an RRCConnectionRelease message with the freqPriorityListEUTRA with higher priority frequency on different LTE TDD band }

ensure that {
  when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority frequency on different LTE TDD band }

then { UE reselects the cell which belongs to the higher priority frequency on different LTE TDD band }

}

(2)

with { UE in E-UTRA RRC_IDLE state in LTE TDD mode having received an RRCConnectionRelease message with the freqPriorityListEUTRA with lower priority frequency on different LTE FDD band }

ensure that {
  when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority frequency on different LTE FDD band }

then { UE reselects the cell which belongs to the lower priority frequency on different LTE FDD band }

}

8.1.3.12a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.8.3 and TS 36.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

...

1>	if the releaseCause received in the RRCConnectionRelease message indicates 'loadBalancingTAURequired':

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE 1:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE 2: The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

8.1.3.12a.3	Test description

8.1.3.12a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are FDD cell and Cell 10 is TDD cell

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

NOTE 1: For Cell 10 TAI is set to TAI-2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.12a.3.2	Test procedure sequence

Table 8.1.3.12a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.12a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | "off” | "off” | The power level values are assigned to satisfy SrxlevCell 1 > Sintrasearch. (NOTE 1) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -79 | -73 | -73 | The power level values are assigned to satisfy SrxlevCell 10 > Threshx, high when reselect to higher priority Cell 10 from lower priority Cell 1 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -79 | -73 | -85 | The power level values are assigned to satisfy both SrxlevCell 2 > Threshx, low (good enough ) and SrxlevCell 10 < Threshserving,low(bad enough) |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.1.3.12a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the freqPriorityListEUTRA on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1. | - | - | - | - |
| 3 | The SS changes Cell 2 and Cell 10 levels according to the row "T1" in table 8.1.3.12a.3.2-1. | - | - | - | - |
| 4-8 | Check: Does the test result of generic test procedure in TS 36.508 steps 1 to 5 of subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10?” | - | - | 1 | - |
| 9 | The SS transmits an RRCConnectionRelease message including the freqPriorityListEUTRA on Cell 10. | <-- | RRCConnectionRelease | - | - |
| 10 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state on Cell 10. | - | - | - | - |
| 11 | The SS changes Cell 1 and Cell 10 level according to the row "T2" in table 8.1.3.12a.3.2-1. | - | - | - | - |
| 12-17 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? | - | - | 2 | - |

8.1.3.12a.3.3	Specific message contents

Table 8.1.3.12a.3.3-0: Void

Table 8.1.3.12a.3.3-1: RRCConnectionRelease (step 1 and step 9, Table 8.1.3.12a.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Operating band under test > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| carrierFreq[2] | Same downlink EARFCN as used for Cell 10 |  |  |
| carrierFreq[2] | maxEARFCN |  | Operating band for inter-band cells > 64 |
| cellReselectionPriority[2] | 5 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  |  |
| freqPriorityListUTRA-TDD | Not present |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  | Operating band under test > 64 or Operating band for inter-band cells > 64 |
| nonCriticalExtension |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  | Operating band under test > 64 |
| carrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 10 |  | Operating band for inter-band cells > 64 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.12a.3.3-2: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.12a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Operating band for inter-band cells > 64 |
| threshX-High[1] | 11 | Actual value of threshold = IE value * 2 [dB] |  |
| threshX-Low[1] | 11 | Actual value of threshold = IE value * 2 [dB] |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band for inter-band cells > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.12a.3.3-3: SystemInformationBlockType3 for Cell 10 (preamble and all steps, Table 8.1.3.12a.3.2-2)

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 14 (28 dB) | 28 is actual value in dB (14 * 2 dB) |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 0 (0 dB) |  |  |
| } |  |  |  |

Table 8.1.3.12a.3.3-4: SystemInformationBlockType5 for Cell 10 (preamble and all steps, Table 8.1.3.12a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Operating band under test > 64 |
| threshX-High[1] | 11 | Actual value of threshold = IE value * 2 [dB] |  |
| threshX-Low[1] | 11 | Actual value of threshold = IE value * 2 [dB] |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Operating band under test > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.12a.3.3-5: SystemInformationBlockType3 for Cells 1 and 2 (preamble and all steps, Table 8.1.3.12a.3.2-2)

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearch | 0 (0 dB) |  |  |
| } |  |  |  |

#### 8.1.3.12b	RRC connection release / Success / With priority information / Inter-band (Single frequency operation in source band)

The scope and description of the present TC is the same as test case 8.1.3.12 with the following differences:

-	Cell configuration: Cell 28 with frequency f6 replaces Cell 3 with frequency f2 with the condition referring to the applicable operating band.

#### 8.1.3.13	LTE RRC connection release / Success / With idle mode measurement information from SIB5

8.1.3.13.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionRelease message with the measIdleConfig and SIB5 includes measIdleCarrierListEUTRA-r15 }

then { UE enters RRC_IDLE state }

}

(2)

with { UE in E-UTRA RRC_IDLE state after receiving RRCConnectionRelease message with measIdleConfig }

ensure that {
  when { UE receives RRCConnectionSetup after sending RRCConnectionRequest and SIB2 includes idleModeMeasurements }

then { UE sends RRCConnectionComplete message including idleMeasAvailable }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives UEInformationRequest message with idleModeMeasurementReq included }

then { UE sends UEInformationResponse message including measResultListIdle }

}

8.1.3.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.8.3, 5.6.5.3 and 5.6.20.2.

[TS 36.331, clause 5.3.3.4]

The UE shall:

- set the content of RRCConnectionSetupComplete message as follows:

2> if the UE is connected to EPC:

3> except for NB-IoT:

4> if the SIB2 contains idleModeMeasurements, and the UE has IDLE mode measurement information available in VarMeasIdleReport:

5> include the idleMeasAvailable;

[TS 36.331, clause 5.3.8.3]

The UE shall:

- if the RRCConnectionRelease message includes the measIdleConfig:

2> clear VarMeasIdleConfig and VarMeasIdleReport;

2> store the received measIdleDuration in VarMeasIdleConfig;

2> start T331 with the value of measIdleDuration;

2> else:

3> store the measIdleCarrierListEUTRA received in SIB5 in VarMeasIdleConfig;

2> start performing idle mode measurements as specified in 5.6.20;

[TS 36.331, clause 5.6.5.3]

The UE shall:

1> if the idleModeMeasurementReq is included in the UEInformationRequest and UE has stored VarMeasIdleReport:

2> set the measResultListIdle in the UEInformationResponse message to the value of idleMeasReport in the VarMeasIdleReport;

2> discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 36.331, clause 5.6.20.2]

While T331 is running, the UE shall:

1> perform the measurements in accordance with the following:

2> for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

3> if UE supports carrier aggregation between serving carrier and the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

4> perform measurements in the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

NOTE: The fields s-NonIntraSearch in SystemInformationBlockType3 do not affect the UE measurement procedures in IDLE mode. How the UE performs measurements in IDLE mode is up to UE implementation as long as the requirements in TS 36.133 [16] are met for measurement reporting. UE is not required to perform idle measurements if SIB2 idle measurement indication is not configured.

4> if the measCellList is included:

5> consider PCell and cells identified by each entry within the measCellList to be applicable for idle mode measurement reporting;

4> else:

5> consider PCell and up to maxCellMeasIdle strongest identified cells whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold (if any) to be applicable for idle mode measurement reporting;

4> store measurement results for cells applicable for idle mode measurement reporting within the VarMeasIdleReport;

3> else:

4> do not consider the carrier frequency to be applicable for idle mode measurement reporting;

1> if validityArea is configured in VarMeasIdleConfig and UE reselects to a serving cell whose physical cell identity does not match any entry in validityArea for the corresponding carrier frequency:

2> stop T331;

8.1.3.13.3	Test description

8.1.3.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 6.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.13.3.2	Test procedure sequence

Table 8.1.3.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.13.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -97 |  |

Table 8.1.3.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the measIdleConfig on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 30 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1 and do measurements over cells 1, 3 and 6. | - | - | - | - |
| 3 | The SS sends RRC Paging message with UE S-TMSI on the cell 1. | <-- | Paging |  |  |
| 4 | Check: Does UE transmits an RRCConnectionRequest message on the cell 1? | --> | RRCConnectionRequest | 1 | P |
| 5 | The SS transmits a RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 6 | Check: Does the UE send RRCConnectionSetupComplete message with idleMeasAvailable? | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | 2 | P |
| 7 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | SecurityModeCommand | - | - |
| 8 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 9 | The SS transmits a RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1, | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfiguration complete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS transmits UEInformationRequest message with idleModeMeasurementReq included. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does UE send UEInformationResponse message including measResultListIdle? | --> | UEInformationResponse | 3 | P |

8.1.3.13.3.3	Specific message contents

Table 8.1.3.13.3.3-0: Conditions for specific message contents
in Tables 8.1.3.13.3.3-1, 8.1.3.13.3.3-2 and 8.1.3.12.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.13.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.13.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| releaseCause | other |  |  |
| redirectedCarrierInfo | Not present |  |  |
| idleModeMobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | Not present |  |  |
| measIdleConfig-r15 | MeasIdleConfigDedicated-r15 |  |  |
| rrc-InactiveConfig-r15 | Not present |  |  |
| cn-Type-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.13.3.3-2: MeasIdleConfigDedicated-r15 (step 1, Table 8.1.3.13.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r15 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | Not present | Provided by SIB5 |  |
| measIdleDuration-r15 | sec60 |  |  |
| } |  |  |  |

Table 8.1.3.13.3.3-3: SystemInformationBlockType2 for Cell 1 (preamble and all the steps in Table 8.1.3.13.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| idleModeMeasurements-r15 | true |  |  |
| } |  |  |  |

Table 8.1.3.13.3.3-4: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleConfigSIB-r15 SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | EUTRA-CarrierList-r15 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.13.3.3-5: EUTRA-CarrierList-r15 (step 1, Table 8.1.3.12.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-CarrierList-r15 SEQUENCE ( | 2 entries |  |  |
| MeasIdleCarrierEUTRA-r15[1] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 3 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasIdleCarrierEUTRA-r15[2] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 6 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.14	LTE RRC connection release / Success / With idle mode measurement information from RRCConnectionRelease

8.1.3.14.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionRelease message with the measIdleConfig with measIdleCarrierListEUTRA-r15 }

then { UE enters RRC_IDLE state }

}

(2)

with { UE in E-UTRA RRC_IDLE state after receiving RRCConnectionRelease message with measIdleConfig }

ensure that {
  when { UE receives RRCConnectionSetup after sending RRCConnectionRequest and SIB2 includes idleModeMeasurements }

then { UE sends RRCConnectionComplete message including idleMeasAvailable }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives UEInformationRequest message with idleModeMeasurementReq included }

then { UE sends UEInformationResponse message including measResultListIdle }

}

8.1.3.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.8.3, 5.6.5.3 and 5.6.20.2.

[TS 36.331, clause 5.3.3.4]

The UE shall:

- set the content of RRCConnectionSetupComplete message as follows:

2> if the UE is connected to EPC:

3> except for NB-IoT:

4> if the SIB2 contains idleModeMeasurements, and the UE has IDLE mode measurement information available in VarMeasIdleReport:

5> include the idleMeasAvailable;

[TS 36.331, clause 5.3.8.3]

The UE shall:

- if the RRCConnectionRelease message includes the measIdleConfig:

2> clear VarMeasIdleConfig and VarMeasIdleReport;

2> store the received measIdleDuration in VarMeasIdleConfig;

2> start T331 with the value of measIdleDuration;

2> if the measIdleConfig contains measIdleCarrierListEUTRA:

3> store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

2> start performing idle mode measurements as specified in 5.6.20;

[TS 36.331, clause 5.6.5.3]

The UE shall:

1> if the idleModeMeasurementReq is included in the UEInformationRequest and UE has stored VarMeasIdleReport:

2> set the measResultListIdle in the UEInformationResponse message to the value of idleMeasReport in the VarMeasIdleReport;

2> discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 36.331, clause 5.6.20.2]

While T331 is running, the UE shall:

1> perform the measurements in accordance with the following:

2> for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

3> if UE supports carrier aggregation between serving carrier and the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

4> perform measurements in the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

NOTE: The fields s-NonIntraSearch in SystemInformationBlockType3 do not affect the UE measurement procedures in IDLE mode. How the UE performs measurements in IDLE mode is up to UE implementation as long as the requirements in TS 36.133 [16] are met for measurement reporting. UE is not required to perform idle measurements if SIB2 idle measurement indication is not configured.

4> if the measCellList is included:

5> consider PCell and cells identified by each entry within the measCellList to be applicable for idle mode measurement reporting;

4> else:

5> consider PCell and up to maxCellMeasIdle strongest identified cells whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold (if any) to be applicable for idle mode measurement reporting;

4> store measurement results for cells applicable for idle mode measurement reporting within the VarMeasIdleReport;

3> else:

4> do not consider the carrier frequency to be applicable for idle mode measurement reporting;

1> if validityArea is configured in VarMeasIdleConfig and UE reselects to a serving cell whose physical cell identity does not match any entry in validityArea for the corresponding carrier frequency:

2> stop T331;

8.1.3.14.3	Test description

8.1.3.14.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 6.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.14.3.2	Test procedure sequence

Table 8.1.3.14.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.14.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -97 |  |

Table 8.1.3.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the measIdleConfig on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 30 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1 and do measurements over cells 1, 3 and 6. | - | - | - | - |
| 3 | The SS sends RRC Paging message with UE S-TMSI on the cell 1. | <-- | Paging |  |  |
| 4 | Check: Does UE transmit an RRCConnectionRequest message on the cell 1? | --> | RRCConnectionRequest | 1 | P |
| 5 | The SS transmits a RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 6 | Check: Does the UE send RRCConnectionSetupComplete message with idleMeasAvailable? | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | 2 | P |
| 7 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | SecurityModeCommand | - | - |
| 8 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 9 | The SS transmits a RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1, | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfiguration complete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS transmits UEInformationRequest message with idleModeMeasurementReq included. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does UE send UEInformationResponse message including measResultListIdle? | --> | UEInformationResponse | 3 | P |

8.1.3.14.3.3	Specific message contents

Table 8.1.3.14.3.3-0: Conditions for specific message contents
in Table 8.1.3.14.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.14.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.14.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| releaseCause | other |  |  |
| redirectedCarrierInfo | Not present |  |  |
| idleModeMobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measIdleConfig-r15 | MeasIdleConfigDedicated-r15 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.14.3.3-2: MeasIdleConfigDedicated-r15 (step 1, Table 8.1.3.14.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r15 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | EUTRA-CarrierList-r15 |  |  |
| measIdleDuration-r15 | sec60 |  |  |
| } |  |  |  |

Table 8.1.3.14.3.3-3: EUTRA-CarrierList-r15 (step 1, Table 8.1.3.14.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-CarrierList-r15 SEQUENCE ( | 2 entries |  |  |
| MeasIdleCarrierEUTRA-r15[1] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 3 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasIdleCarrierEUTRA-r15[2] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 6 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.14.3.3-4: SystemInformationBlockType2 for Cell 1 (preamble and all the steps in Table 8.1.3.14.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| idleModeMeasurements-r15 | true |  |  |
| } |  |  |  |

Table 8.1.3.14.3.3-4: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.14.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.15	LTE RRC connection release / Success / With idle mode measurement information / No idle mode measurement capability provided

8.1.3.15.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionRelease message with the measIdleConfig with measIdleCarrierListEUTRA-r15 }

then { UE enters RRC_IDLE state }

}

(2)

with { UE in E-UTRA RRC_IDLE state after receiving RRCConnectionRelease message with measIdleConfig }

ensure that {
  when { UE receives RRCConnectionSetup after sending RRCConnectionRequest and SIB2 does not include idleModeMeasurements }

then { UE sends RRCConnectionComplete message }

}

8.1.3.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.8.3, 5.6.5.3 and 5.6.20.2.

[TS 36.331, clause 5.3.3.4]

The UE shall:

- set the content of RRCConnectionSetupComplete message as follows:

2> if the UE is connected to EPC:

3> except for NB-IoT:

4> if the SIB2 contains idleModeMeasurements, and the UE has IDLE mode measurement information available in VarMeasIdleReport:

5> include the idleMeasAvailable;

[TS 36.331, clause 5.3.8.3]

The UE shall:

- if the RRCConnectionRelease message includes the measIdleConfig:

2> clear VarMeasIdleConfig and VarMeasIdleReport;

2> store the received measIdleDuration in VarMeasIdleConfig;

2> start T331 with the value of measIdleDuration;

2> if the measIdleConfig contains measIdleCarrierListEUTRA:

3> store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

2> start performing idle mode measurements as specified in 5.6.20;

[TS 36.331, clause 5.6.5.3]

The UE shall:

1> if the idleModeMeasurementReq is included in the UEInformationRequest and UE has stored VarMeasIdleReport:

2> set the measResultListIdle in the UEInformationResponse message to the value of idleMeasReport in the VarMeasIdleReport;

2> discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 36.331, clause 5.6.20.2]

While T331 is running, the UE shall:

1> perform the measurements in accordance with the following:

2> for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

3> if UE supports carrier aggregation between serving carrier and the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

4> perform measurements in the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

NOTE: The fields s-NonIntraSearch in SystemInformationBlockType3 do not affect the UE measurement procedures in IDLE mode. How the UE performs measurements in IDLE mode is up to UE implementation as long as the requirements in TS 36.133 [16] are met for measurement reporting. UE is not required to perform idle measurements if SIB2 idle measurement indication is not configured.

4> if the measCellList is included:

5> consider PCell and cells identified by each entry within the measCellList to be applicable for idle mode measurement reporting;

4> else:

5> consider PCell and up to maxCellMeasIdle strongest identified cells whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold (if any) to be applicable for idle mode measurement reporting;

4> store measurement results for cells applicable for idle mode measurement reporting within the VarMeasIdleReport;

3> else:

4> do not consider the carrier frequency to be applicable for idle mode measurement reporting;

1> if validityArea is configured in VarMeasIdleConfig and UE reselects to a serving cell whose physical cell identity does not match any entry in validityArea for the corresponding carrier frequency:

2> stop T331;

8.1.3.15.3	Test description

8.1.3.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 3 and Cell 6.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.1.3.15.3.2	Test procedure sequence

Table 8.1.3.15.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.15.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | -97 |  |

Table 8.1.3.15.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including the measIdleConfig on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 30 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1 and do measurements over cells 1, 3 and 6. | - | - | - | - |
| 3 | Check: Does UE transmit an RRCConnectionRequest message on the cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits a RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 5 | Check: Does the UE send RRCConnectionSetupComplete message without idleMeasAvailable? | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | 2 | P |

8.1.3.15.3.3	Specific message contents

Table 8.1.3.15.3.3-0: Conditions for specific message contents
in Tables 8.1.3.15.3.3-1, 8.1.3.15.3.3-2 and 8.1.3.15.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.15.3.3-1: RRCConnectionRelease (step 1, Table 8.1.3.15.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| releaseCause | other |  |  |
| redirectedCarrierInfo | Not present |  |  |
| idleModeMobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measIdleConfig-r15 | MeasIdleConfigDedicated-r15 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.15.3.3-2: MeasIdleConfigDedicated-r15 (step 1, Table 8.1.3.15.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r15 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | EUTRA-CarrierList-r15 |  |  |
| measIdleDuration-r15 | sec60 |  |  |
| } |  |  |  |

Table 8.1.3.15.3.3-3: EUTRA-CarrierList-r15 (step 1, Table 8.1.3.15.3.2-2)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-CarrierList-r15 SEQUENCE ( | 2 entries |  |  |
| MeasIdleCarrierEUTRA-r15[1] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 3 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasIdleCarrierEUTRA-r15[2] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 6 |  |  |
| allowedMeasBandwidth-r15 | mbw6 |  |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | 43 | -97 dBm |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.15.3.3-4: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 8.1.3.15.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| dl-CarrierFreq[2] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-v9e0[2] | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.3.16	RRC connection release / Redirection to another E-UTRAN frequency / MPS Priority Indication

8.1.3.16.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including IE mpsPriorityIndication-r16 and an IE redirectedCarrierInfo with eutra frequency different from UE was on in RRC_CONNECTED state}

then { UE establishes an RRC connection with establishmentCause set to highPriorityAccess on the new frequency included in IE redirectedCarrierInfo }

}

8.1.3.16.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.3.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	if the UE is connected to EPC:

2>	set the ue-Identity as follows:

3>	if upper layers provide an S-TMSI:

4>	set the ue-Identity to the value received from upper layers;

3>	else:

4>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

2>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

3>	set the establishmentCause to highPriorityAccess

…

8.1.3.16.3	Test description

8.1.3.16.3.1	Pre-test conditions

System Simulator:

-	2 cells on different E-UTRA frequencies

-	Cell 1 is set to ''Serving cell''

-	Cell 3 is set to ''Non-Suitable Off cell''

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18].

8.1.3.16.3.2	Test procedure sequence

Table 8.1.3.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell 3 as a "Suitable Neighbour cell". | - | - | - | - |
| 2 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | <-- | IP packet | - | - |
| 3 | Wait for 1 s after the IP packet has been transmitted in step 2. (Note 1) | - | - | - | - |
| 4 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra of Cell 3 and IE mpsPriorityIndication). | <-- | RRCConnectionRelease | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 3 with establishmentCause set to ‘highPriorityAccess’? | --> | RRCConnectionRequest | 1 | P |
| 6-10 | Steps 4 to 8 of the generic radio bearer establishment procedure (TS 36.508 [18] 4.5.3.3-1) are executed. | - | - | - | - |
| - | EXCEPTION: Steps 11 and 12 can occur in any order | - | - | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete |  |  |
| 12 | The UE loops back the IP packet received in step 2 on the DRB associated with the default EPS bearer context on Cell 3. | --> | IP packet | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE has received and forwarded the IP Packet transmitted by the SS in step 2 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 4 |  |  |  |  |  |

8.1.3.16.3.3	Specific message contents

Table 8.1.3.16.3.3-1: Conditions for specific message contents in tables 8.1.3.16.3.3-2 and 8.1.3.16.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.16.3.3-2: SystemInformationBlockType5 for cell 1 (preamble and all steps, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-3: SystemInformationBlockType5 for cell 3 (preamble and all steps, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-4: RRCConnectionRelease message (step 4, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 3 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 3 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mpsPriorityIndication-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-5: RRCConnectionRequest (step 5, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-6: CLOSE UE TEST LOOP (preamble, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0010'B | 2 seconds |  |

## 8.2	RRC connection reconfiguration

### 8.2.1	Radio bearer establishment

#### 8.2.1.1	RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC_CONNECTED / Success / Default bearer / Early bearer establishment

8.2.1.1.1	Test Purpose (TP)

(1)

with { UE having completed the RRC connection establishment procedure }

ensure that {
  when { SS sends in sequence a SecurityModeCommand and an RRCConnectionReconfiguration message }

then { UE establishes the initial security configuration in accordance with the received securityConfigSMC included in SecurityModeCommand and successfully completes the connection reconfiguration }

}

(2)

with { UE in RRC_CONNECTED state}

ensure that {
  when { UE receives an RRCConnectionRelease message }

then { UE releases the signalling connection, the established EPS bearer and all radio resources and enters in RRC_IDLE state }

}

8.2.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.1.2, 5.3.1.1, 5.3.5.3, 5.3.10.1, 5.3.10.3, 5.3.8.3 and 5.3.12. The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331, clause 5.1.2]

The UE shall:

1> process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE: 1	E-UTRAN may initiate a subsequent procedure prior to receiving the UEs response of a previously initiated procedure.

[TS 36.331, clause 5.3.1.1]

After having initiated the initial security activation procedure, E-UTRAN initiates the establishment of SRB2 and DRBs, i.e. E-UTRAN may do this prior to receiving the confirmation of the initial security activation from the UE. In any case, E-UTRAN will apply both ciphering and integrity protection for the RRC connection reconfiguration messages used to establish SRB2 and DRBs.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

...

1>	else:

2>	If the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 2:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

...

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.1]

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment):

2>	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2> establish a PDCP entity and configure it with the current security configuration, if applicable;

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	establish a DCCH logical channel in accordance with the received logicalChannelConfig and with the logical channel identity set in accordance with 9.1.2

[TS 36.331, clause 5.3.10.3]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment):

2>	establish a PDCP entity and configure it with the current security configuration and in accordance with the received pdcp-Config;

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	establish a DTCH logical channel in accordance with the received logicalChannelIdentity and the received logicalChannelConfig;

1>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the  RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

...

1>	if the releaseCause received in the RRCConnectionRelease message indicates ‘loadBalancingTAURequired’:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘load balancing TAU required’;

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘other’.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE by performing cell selection in accordance with the cell selection process, defined for the case of leaving RRC_CONNECTED, as specified in TS 36.304 [4].

8.2.1.1.3	Test description

8.2.1.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.2.1.1.3.2	Test procedure sequence

Table 8.2.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | Paging | - | - |
| 2 | UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | SS transmit an RRCConnectionSetup message | <-- | RRCConnectionSetup | - | - |
| 4 | The UE transmits an RRCConnectionSetupComplete to confirm the successful completion of the connection establishment. (Note 1) | --> | RRCConnectionSetupComplete | - | - |
| 5 | SS transmits a SecurityModeCommand message to activate AS security. | <-- | SecurityModeCommand | - | - |
| 6 | Using the same TTI as the SecurityModeCommand message in step 5, the SS transmits an RRCConnectionReconfiguration message to establish a data radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: Does the UE transmit a SecurityModeComplete message? | --> | SecurityModeComplete | 1 | P |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message to confirm the establishment of data radio bearer? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 9 | Check: Does the test result of CALL generic procedure in TS 36.508 subclause 6.4.2.3 indicate that UE is in E-UTRA RRC_CONNECTED state? | - | - | 1 | - |
| 10 | SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 11 | SS waits for 5s. | - | - | - | - |
| 12 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 2 | - |
| Note 1: Contains a SERVICE REQUEST NAS message. |  |  |  |  |  |

8.2.1.1.3.3	Specific message contents

Table 8.2.1.1.3.3-1: RRCConnectionReconfiguration (step 6, Table 8.2.1.1.3.2-1)

Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0)

#### 8.2.1.2	Void

#### 8.2.1.3	RRC connection reconfiguration / Radio bearer establishment / Success / Dedicated bearer

8.2.1.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { SS sends an RRCConnectionReconfiguration message including a drb-Identity that is not part of the current UE configuration and a dedicatedInfoNASList}

then { UE successfully establish the radio bearer according to IE radioResourceConfigDedicated }

}

8.2.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.3 and 5.3.10.3.

The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInformation and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	If the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 2:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2>	forward each element of the dedicatedInfoNASList to upper layers;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment):

2>	establish a PDCP entity and configure it with the current security configuration and in accordance with the received pdcp-Config;

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	establish a DTCH logical channel in accordance with the received logicalChannelIdentity and the received logicalChannelConfig;

1>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

8.2.1.3.3	Test description

8.2.1.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.1.3.3.2	Test procedure sequence

Table 8.2.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to establish a data radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The UE transmits an ULInformationTransfer message. This message includes an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.2.1.3.3.3	Specific message contents

Table 8.2.1.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.1.3.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8, condition DRB(1,0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| dedicatedInfoNASList SEQUENCE (SIZE(1..maxDRB)) OF | 1 entry |  |  |
| dedicatedInfoNAS [1] | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST with EPS bearer identity corresponding to drb-Identity 2 | According 36.508 table 4.7.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.1.4	Void

#### 8.2.1.5	RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check

8.2.1.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and has sent an RRCConnectionRequest message }

ensure that {

when { UE receive a RRCConnectionSetup message and 16 subframes later receives an UL grant }

then { UE successfully transmit RRCConnectionSetupComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receive a SecurityModeCommand message and 11 subframes later receives an UL grant }

then { UE successfully establish the initial AS security activation according to IE securityConfigSMC }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receive a RRCConnectionReconfiguration message including a drb-Identity that is not part of the current UE configuration and a nas-DedicatedInformation and 16 subframes later receives an UL grant }

then { UE successfully establish the radio bearer according to IE radioResourceConfiguration }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an UECapabilityEnquiry message after AS security is activated and at 11 subframes later receives an UL grant }

then { UE successfully transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-RadioAccessCapRequest variable }

}

8.2.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.4.3, 5.3.5.3, 5.6.3.3 and 11.2.

[TS 36.331, clause 5.3.3.4]

The UE shall:

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>		if upper layers provide the ‘Registered MME’, include and set the registeredMME as follows:

3>	if the PLMN identity of the ‘Registered MME’ is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the ‘Registered MME’ received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.4.3]

The UE shall:

1>	derive the KeNB key, as specified in TS 33.401 [32];

1> derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.401 [32];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2> derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.401 [32];

2>	configure lower layers to apply integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply ciphering using the indicated algorithm, the KRRCenc key and the KUPenc key after completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 2:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2>	forward each element of the dedicatedInfoNASList to upper layers;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes ‘eutra’:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to ‘eutra’;

2>	if the ue-CapabilityRequest includes ‘geran-cs’ and if the UE supports GERAN CS domain:

3>	include the UE radio access capabilities for GERAN CS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-cs’;

2>	if the ue-CapabilityRequest includes ‘geran-ps’ and if the UE supports GERAN PS domain:

3>	include the UE radio access capabilities for GERAN PS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-ps’;

2>	if the ue-CapabilityRequest includes ‘utra’ and if the UE supports UTRA:

3>	include the UE radio access capabilities for UTRA within a ue-CapabilityRAT-Container and with the rat-Type set to ‘utra’;

…

1> submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 11.2]

The UE performance requirements for RRC procedures are specified in the following table, by means of a value N:

N = the number of 1ms subframes from the end of reception of the E-UTRAN -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> E-UTRAN response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 11.2-1: Illustration of RRC procedure delay

Table 11.2-1: Procedures

| Procedure title: | E-UTRAN -> UE | UE -> E-UTRAN | N | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC connection establishment | RRCConnectionSetup | RRCConnectionSetupComplete | 15 |  |
| RRC connection re-configuration (radio resource configuration) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 |  |
| Initial security activation | SecurityModeCommand | SecurityModeCommandComplete/SecurityModeCommandFailure | 10 |  |
| Other procedures |  |  |  |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 10 |  |

8.2.1.5.3	Test description

8.2.1.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	Uplink and downlink bandwidth set to the maximum bandwidth for the E-UTRA Band under test as specified in Table 5.6.1-1 in [31] (to enable allocation of Nprb up to maximum value to avoid segmentation of UECapabilityInformation message).

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.2.1.5.3.2	Test procedure sequence

Table 8.2.1.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | Paging | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS transmit an RRCConnectionSetup message. (Note 3) | <-- | RRCConnectionSetup |  |  |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message within 20 (FDD)/23 (TDD) subframes after successful completion of step 3? (Note 2) | --> | RRCConnectionSetupComplete | 1 | P |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. (Note 3) | <-- | SecurityModeCommand |  |  |
| 6 | Check: Does the UE transmit a SecurityModeComplete message within 15 (FDD)/18 (TDD) subframes after successful completion of step 5? (Note 2) | --> | SecurityModeComplete | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message to establish a data radio bearer. (Note 3) | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 20 (FDD)/23 (TDD) subframes after successful completion of step 7? (Note 2) | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 9 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only. (Note 3) | <-- | UECapabilityEnquiry | - | - |
| 10 | Check: Does the UE transmit a UECapabilityInformation message within 15 (FDD)/18 (TDD) subframes after successful completion of step 9? (Note 2) | --> | UECapabilityInformation | 4 | P |
| Note 1: The SS is configured for RRC procedure delay testing according to TS 36.523-3 clause 7.12.1. Note 2: 5 (FDD) / 8 (TDD) subframes is added on top of RRC processing delay requirement, because UL grant is allocated for the next subframe after RRC procedure delay and because of transmission delay after UL grant. In the case of CAT-M1 UEs, if UECapabilityInformation message is above 1000bits, then, it will be transmitted in two RLC segmented MAC PDU. In such case, an additional 10ms delay is required for transmission of second RLC segment ie 15(FDD)/ 18(TDD) subframes is added on top of RRC processing delay requirement. For TDD, consider the max 7 TTI transmission delay after UL grant. Note 3: The DL resource allocation sent on PDCCH is big enough to guarantee transmission of all DL RRC PDU in single TTI. |  |  |  |  |  |

          - Specific message contents

Table 8.2.1.5.3.3-1: RRCConnectionRequest (step 2, Table 8.2.1.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.5.3.3-2: RRCConnectionSetup (step 3, Table 8.2.1.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.5.3.3-3: RRCConnectionReconfiguration (step 7, Table 8.2.1.5.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.5.3.3-4: RRCConnectionReconfiguration (preamble: Table 4.5.2.3-1 [18], step 14)

| Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.1.6	RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check / SecurityModeCommand and RRCConnectionReconfiguration transmitted in the same TTI

8.2.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE received SecurityModeCommand message and RRCConnectionReconfiguration message including a drb-Identity that is not part of the current UE configuration and a nas-DedicatedInformation in the same TTI and 21 subframes later receives an UL grant }

then { UE successfully establish the initial AS security activation according to IE securityConfigSMC and the radio bearer according to IE radioResourceConfiguration }

}

8.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.4.3, 5.3.5.3 and 11.2.

[TS 36.331, clause 5.3.4.3]

The UE shall:

1>	derive the KeNB key, as specified in TS 33.401 [32];

1> derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.401 [32];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2> derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.401 [32];

2>	configure lower layers to apply integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply ciphering using the indicated algorithm, the KRRCenc key and the KUPenc key after completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 2:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2>	forward each element of the dedicatedInfoNASList to upper layers;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 11.2]

The UE performance requirements for RRC procedures are specified in the following table, by means of a value N:

N = the number of 1ms subframes from the end of reception of the E-UTRAN -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> E-UTRAN response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 11.2-1: Illustration of RRC procedure delay

Table 11.2-1: RRC procedure

| Procedure title: | E-UTRAN -> UE | UE -> E-UTRAN | N | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| Initial security activation + RRC connection re-configuration (RB establishment) | SecurityModeCommand, RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 | The two DL messages are transmitted in the same TTI |

8.2.1.6.3	Test description

8.2.1.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

8.2.1.6.3.2	Test procedure sequence

Table 8.2.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | Paging | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 3 | The SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | The UE transmit an RRCConnectionSetupComplete message. | --> | RRCConnectionSetupComplete | - | - |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. (Note 3) | <-- | SecurityModeCommand | - | - |
| 6 | The SS transmits a RRCConnectionReconfiguration message to establish a data radio bearer. (Note 3) | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: Does the UE transmit an SecurityModeComplete message within 25 (FDD)/28 (TDD) subframes after successful completion of step 5? (Note 2) | --> | SecurityModeComplete | 1 | P |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes after successful completion of step 6? (Note 2) | --> | RRCConnectionReconfigurationComplete | 1 | P |
| Note 1: The SS is configured for RRC procedure delay testing according to TS 36.523-3 clause 7.12.1. Note 2: 5 (FDD) / 8 (TDD) subframes is added on top of RRC processing delay requirement, because UL grant is allocated for the next subframe after RRC procedure delay and because of transmission delay after UL grant. For TDD, consider the max 7 TTI transmission delay after UL grant. Note 3: The DL resource allocation sent on PDCCH is big enough to guarantee transmission of all DL RRC PDU in single TTI. |  |  |  |  |  |

8.2.1.6.3.3	Specific message contents

Table 8.2.1.6.3.3-1: RRCConnectionRequest (step 2, Table 8.2.1.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.6.3.3-2: RRCConnectionSetup (step 3, Table 8.2.1.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.6.3.3-3: RRCConnectionReconfiguration (step 6, Table 8.2.1.6.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.6.3.3-4: RRCConnectionReconfiguration (preamble: Table 4.5.2.3-1 [18], step 14)

| Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.1.7	RRC connection reconfiguration / Radio bearer establishment / Success / SRB2

8.2.1.7.1	Test Purpose (TP)

with { UE in E-UTRA RRC_CONNECTED state without SRB2}

ensure that {
  when { SS sends an RRCConnectionReconfiguration message including SRB2 configuration }

then { UE successfully establish the signalling radio bearer }

}

8.2.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.3, and 5.3.10.1.

The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 2:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.1]

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment):

2>	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2> establish a PDCP entity and configure it with the current security configuration, if applicable;

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	establish a DCCH logical channel in accordance with the received logicalChannelConfig and with the logical channel identity set in accordance with 9.1.2;

8.2.1.7.3	Test description

8.2.1.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) according to [18].

8.2.1.7.3.2	Test procedure sequence

Table 8.2.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-7 | Generic Radio Bearer Establishment (State 3) step 1 to 7 in TS 36.508 subclause 4.5.3.3. |  |  | - | - |
| 8 | The SS transmits an RRCConnectionReconfiguration message to establish SRB2. | <-- | RRCConnectionReconfiguration | - | - |
| 9 | Check: Does the UE transmits an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 10 | Check: Does the test result of CALL generic procedure in TS 36.508 subclause 6.4.2.3 indicates that UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.2.1.7.3.3	Specific message contents

Table 8.2.1.7.3.3-1: RRCConnectionReconfiguration (step 8, Table 8.2.1.7.3.2-1))

| Derivation Path: 36.508 table 4.6.1-6, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| C1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  |  |  |
| srb-ToAddMod[1] SEQUENCE { |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| explicitValue | RLC-Config-NON-DEFAULT-SRB2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.7.3.3-2: RLC-Config-NON-DEFAULT-SRB2 ( Table 8.2.1.7.3.3-1))

| Derivation Path: 36.331 clauses 6.3.2, 9.2.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-NON-DEFAULT-SRB2 ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| t-PollRetransmit | ms100 |  |  |
| pollPDU | p16 |  |  |
| pollByte | kb500 |  |  |
| maxRetxThreshold | t6 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| t-Reordering | ms100 |  |  |
| t-StatusProhibit | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.1.8	RRC connection reconfiguration / Radio bearer establishment / Success / Dedicated bearer / ROHC configured

8.2.1.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including IE headerCompression set to "rohc" }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.3, clause 5.3.10.0, clause 5.3.10.3, TS 36.323, clause 5.5.1, 5.5.2 and TS 36.306, clause 4.3.1.1.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

...

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

[TS 36.331, clause 5.3.10.0]

The UE shall:

...

1>	if the received radioResourceConfigDedicated includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.10.3;

[TS 36.331, clause 5.3.10.3]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it with the current security configuration and in accordance with the received pdcp-Config;

[TS 36.323, clause 5.5.1]

The header compression protocol is based on the Robust Header Compression (ROHC) framework [7]. There are multiple header compression algorithms, called profiles, defined for the ROHC framework. Each profile is specific to the particular network layer, transport layer or upper layer protocol combination e.g. TCP/IP and RTP/UDP/IP.

The detailed definition of the ROHC channel is specified as part of the ROHC framework in RFC 4995 [7]. This includes how to multiplex different flows (header compressed or not) over the ROHC channel, as well as how to associate a specific IP flow with a specific context state during initialization of the compression algorithm for that flow.

The implementation of the functionality of the ROHC framework and of the functionality of the supported header compression profiles is not covered in this specification.

In this version of the specification the support of the following profiles is described:

Table 5.5.1.1: Supported header compression protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 4995 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 4996 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

[TS 36.323, clause 5.5.2]

PDCP entities associated with DRBs can be configured by upper layers [3] to use header compression.

[TS 36.306, clause 4.3.1.1]

This field defines which ROHC profiles from the list below are supported by the UE.

-	0x0000 ROHC uncompressed (RFC 4995)

-	0x0001 ROHC RTP (RFC 3095, RFC 4815)

-	0x0002 ROHC UDP (RFC 3095, RFC 4815)

...

'IMS capable UEs supporting voice' shall support ROHC profiles 0x0000, 0x0001, 0x0002 and be able to compress and decompress headers of PDCP SDUs at a PDCP SDU rate corresponding to supported IMS voice codecs.

8.2.1.8.3	Test description

8.2.1.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.1.8.3.2	Test procedure sequence

Table 8.2.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to establish a data radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The UE transmits an ULInformationTransfer message. This message includes an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state? | - | - | 1 | - |

8.2.1.8.3.3	Specific message contents

Table 8.2.1.8.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.1.8.3.2-1)

| Derivation Path: TS 36.508 Table 4.6.1-8, condition DRB(0, 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| dedicatedInfoNASList SEQUENCE (SIZE(1..maxDRB)) OF | 1 entry |  |  |
| DedicatedInfoNAS[1] | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST with EPS bearer identity corresponding to drb-Identity 2 | According 36.508 table 4.7.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.8.3.3-2: PDCP-Config (Table 8.2.1.8.3.3-1)

| Derivation Path: TS 36.508 Table 4.8.2.1.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config-DRB-UM ::= SEQUENCE { |  |  |  |
| rlc-UM SEQUENCE { |  |  |  |
| pdcp-SN-Size | len12bits |  |  |
| } |  |  |  |
| headerCompression CHOICE { |  |  |  |
| rohc SEQUENCE { |  |  |  |
| maxCID | 15 | DEFAULT 15 |  |
| profiles SEQUENCE { |  | IMS capable UEs supporting voice shall support ROHC profiles 0x0000, 0x0001, 0x0002. |  |
| profile0x0001 | TRUE |  |  |
| profile0x0002 | TRUE |  |  |
| profile0x0003 | FALSE |  |  |
| profile0x0004 | FALSE |  |  |
| profile0x0006 | FALSE |  |  |
| profile0x0101 | FALSE |  |  |
| profile0x0102 | FALSE |  |  |
| profile0x0103 | FALSE |  |  |
| profile0x0104 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.8.3.3-3: RLC-Config (Table 8.2.1.8.3.3-1)

| Derivation Path: TS 36.508 Table 4.8.2.1.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-DRB-UM ::= CHOICE { |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  |  |
| ul-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size10 |  |  |
| } |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.2.2	Radio resource reconfiguration

#### 8.2.2.1	RRC connection reconfiguration / Radio resource reconfiguration / Success

8.2.2.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a radioResourceConfigDedicated with SRB, DRB, transport channel and physical channel reconfiguration }

then { UE reconfigures the data and signalling radio bearers and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.3, 5.3.10.1, 5.3.10.3, 5.3.10.4 and 5.3.10.6.

The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

- if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	If the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.1]

The UE shall:

…

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration (SRB reconfiguration):

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the DCCH logical channel in accordance with the received logicalChannelConfig;

[TS 36.331, clause 5.3.10.3]

…

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration (DRB reconfiguration):

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

2>	if the rlc-Config is included:

3>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	if the logicalChannelConfig is included:

3>	reconfigure the DTCH logical channel in accordance with the received logicalChannelConfig;

NOTE:	Removal and addition of the same drb-Identity in single radioResourceConfiguration is not supported.

[TS 36.331, clause 5.3.10.4]

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

8.2.2.1.3	Test description

8.2.2.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

- 	The condition SRB2-DRB(2, 0) is used for step 8 in 4.5.3.3 according to [18].

8.2.2.1.3.2	Test procedure sequence

Table 8.2.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a radioResourceConfigDedicated with SRBs, DRB, transport channel and physical channel reconfiguration. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.2.2.1.3.3	Specific message contents

Table 8.2.2.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.1.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-RECONFIG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.1.3.3-2: RadioResourceConfigDedicated-RECONFIG (Table 8.2.2.1.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | SRB-ToAddModList-RECONFIG | TS 36.508 Table 4.6.3-22A |  |
| drb-ToAddModList | DRB-ToAddModList-RECONFIG | TS 36.508 Table 4.6.3-2A |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| explicitValue | MAC-MainConfig-RECONFIG |  |  |
| } |  |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-RECONFIG |  |  |
| } |  |  |  |

Table 8.2.2.1.3.3-3: MAC-MainConfig-RECONFIG (Table 8.2.2.1.3.3-2)

| Derivation Path: 36.508 table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig ::= SEQUENCE { |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |
| periodicBSR-Timer | sf32 |  |  |
| retxBSR-Timer | sf2560 |  |  |
| } |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| periodicPHR-Timer | sf1000 |  |  |
| prohibitPHR-Timer | sf500 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.1.3.3-4: PhysicalConfigDedicated-RECONFIG (Table 8.2.2.1.3.3-2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated ::= SEQUENCE { |  |  |  |
| pdsch-ConfigDedicated | PDSCH-ConfigDedicated-RECONFIG |  |  |
| pucch-ConfigDedicated | Not present |  |  |
| pusch-ConfigDedicated | Not present |  |  |
| uplinkPowerControlDedicated | Not present |  |  |
| tpc-PDCCH-ConfigPUCCH | Not present |  |  |
| tpc-PDCCH-ConfigPUSCH | Not present |  |  |
| cqi-ReportConfig | Not present |  |  |
| soundingRsUL-ConfigDedicated | Not present |  |  |
| antennaInfo CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| schedulingRequestConfig | Not present |  |  |
| } |  |  |  |

Table 8.2.2.1.3.3-5: PDSCH-ConfigDedicated-RECONFIG (Table 8.2.2.1.3.3-4)

| Derivation Path: 36.508 table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| p-a | dB1 |  |  |
| } |  |  |  |

#### 8.2.2.2	RRC connection reconfiguration / SRB/DRB reconfiguration / Success

8.2.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SRB reconfiguration }

then { UE reconfigures affected SRBs and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing DRB reconfiguration }

then { UE reconfigures affected DRBs and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.1 and 5.3.10.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	else:

2> if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>perform the Radio resource configuration procedure as specified in 5.3.10;

[TS 36.331, clause 5.3.10.1]

The UE shall:

…

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration (SRB reconfiguration):

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the DCCH logical channel in accordance with the received logicalChannelConfig;

[TS 36.331, clause 5.3.10.3]

The UE shall:

…

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration (DRB reconfiguration):

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

2>	if the rlc-Config is included:

3>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	if the logicalChannelConfig is included:

3>	reconfigure the DTCH logical channel in accordance with the received logicalChannelConfig;

NOTE:	Removal and addition of the same drb-Identity in single radioResourceConfiguration is not supported.

8.2.2.2.3	Test Description

8.2.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

-	The condition SRB2-DRB(2, 0) is used for step 8 in 4.5.3.3 according to [8].

8.2.2.2.3.2	Test procedure sequence

Table 8.2.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRConnectionReconfiguration message containing a radioResourceConfigDedicated with SRB reconfiguration. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | SS transmits RRCConnectionReconfiguration message containing a radioResourceConfigDedicated with DRB reconfiguration. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 2 | P |

8.2.2.2.3.3	Specific message contents

Table 8.2.2.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.2.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRBRECONFIG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3-2: RadioResourceConfigDedicated-SRBRECONFIG (Table 8.2.2.2.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | SRB-ToAddModList-RECONFIG | TS 36.508 Table 4.6.3-22A |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3-3: RRCConnectionReconfiguration (step 3, Table 8.2.2.2.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRBRECONFIG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3-4: RadioResourceConfigDedicated-DRBRECONFIG (Table 8.2.2.2.3.3-3)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | DRB-ToAddModList-RECONFIG | TS 36.508 Table 4.6.3-2A |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3-5: Void

Table 8.2.2.2.3.3-6: Void

#### 8.2.2.3	CA / RRC connection reconfiguration / SCell addition/modification/release / Success

##### 8.2.2.3.1	CA / RRC connection reconfiguration / SCell addition/modification/release / Success / Intra-band Contiguous CA

8.2.2.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a SCell addition }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in deactivated state and sends an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with SCell modification }

then { UE modifies the affected SCell dedicated configurations and sends an RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToReleaseList with a sCellIndex equling to one of the current UE SCell configuration }

then { UE releases this SCell and sends an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes }

}

8.2.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3a, 5.3.10.3b and 11.2. Unless otherwise stated these are Rel-10 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3a]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

1>	if the release is triggered by RRC connection re-establishment:

2>	release all SCells that are part of the current UE configuration;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

[TS 36.331, clause 11.2]

The UE performance requirements for RRC procedures are specified in the following table, by means of a value N:

N = the number of 1ms subframes from the end of reception of the E-UTRAN -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> E-UTRAN response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation) where N=20 (Ref. TS 36.331 Fig. 11.2-1).

8.2.2.3.1.3	Test Description

8.2.2.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.3.1.3.2	Test procedure sequence

Table 8.2.2.3.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-4 | Void | - | - | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 addition. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 modification. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 9 | Void | - | - | - | - |
| 10 | Void | - | - | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToReleaseList with an sCellIndex equalling to the Cell 3 cell index in the current UE configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message within 25 (FDD)/28 (TDD) subframes? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |
| Note 1: The SS is configured for RRC procedure delay testing according to TS 36.523-3 clause 7.12.1. Note 2: 5 (FDD) / 8 (TDD) subframes is added on top of RRC processing delay requirement, because UL grant is allocated for the next subframe after RRC procedure delay and because of transmission delay after UL grant. For TDD, consider the max 7 TTI transmission delay after UL grant. |  |  |  |  |  |

8.2.2.3.1.3.3	Specific message contents

Table 8.2.2.3.1.3.3-0: Conditions for specific message contents
in Tables 8.2.2.3.1.3.3-2 and 8.2.2.3.1.3.3-3, 8.2.2.3.1.3.3-7, 8.2.2.3.1.3.3-8 and 8.2.2.3.1.3.3-11

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.3.1.3.3-1: RRCConnectionReconfiguration (step 5, Table 8.2.2.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-2: SCellToAddMod-r10-f2-Add (Table 8.2.2.3.1.3.3-5)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.3.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.3.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| Ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 3 | optional | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-4: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.3.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-5: RRCConnectionReconfiguration (step 7, Table 8.2.2.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig SEQUENCE { |  |  |  |
| measGapConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Mod | SCell modification for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-6: SCellToAddMod-r10-f2-Mod (Table 8.2.2.3.1.3.3-9)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 | Not present |  |  |
| radioResourceConfigCommonSCell-r10 | Not present |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2-Mod |  | pc_FeatrGrp_102 |
| } |  |  |  |

Table 8.2.2.3.1.3.3-7: RadioResourceConfigDedicatedSCell-r10-f2-Mod (Table 8.2.2.3.1.3.3-10)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| antennaPortsCount | an2 |  |  |
| } |  |  |  |
| crossCarrierSchedulingConfig-r10 | Not present |  |  |
| csi-RS-Config-r10 | Not present |  |  |
| pdsch-ConfigDedicated-r10 | Not present |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| antennaInfoUL-r10 SEQUENCE { |  |  |  |
| transmissionModeUL-r10 | tm1 |  |  |
| fourAntennaPortActivated-r10 | Not present | OP setup |  |
| } |  |  |  |
| pusch-ConfigDedicatedSCell-r10 | PUSCH-ConfigDedicated-DEFAULT |  |  |
| uplinkPowerControlDedicatedSCell-r10 | Not present |  |  |
| cqi-ReportConfigSCell-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
|  | SoundingRS-UL-ConfigDedicated-Mod |  | pc_FeatrGrp_113 |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
|  | SoundingRS-UL-ConfigDedicated-v1020-Mod |  | pc_FeatrGrp_113 |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-8: SoundingRS-UL-ConfigDedicated-Mod (Table 8.2.2.3.1.3.3-11)

| Derivation Path: 36.508, Table 4.6.3-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated ::= CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-Bandwidth | bw3 |  |  |
| srs-HoppingBandwidth | hbw3 |  |  |
| freqDomainPosition | 0 |  |  |
| duration | TRUE |  |  |
| srs-ConfigIndex | 7 |  |  |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-9: SoundingRS-UL-ConfigDedicated-v1020-Mod (Table 8.2.2.3.1.3.3-11)

| Derivation Path: 36.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-v1020 ::= SEQUENCE { |  |  |  |
| srs-AntennaPort-r10 | an1 |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-10: RRCConnectionReconfiguration (step 11, Table 8.2.2.3.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 | SCell release for Cell 3 |  |
| } |  |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.3.2	CA / RRC connection reconfiguration / SCell addition/modification/release / Success / Inter-band CA

8.2.2.3.2.1	Test Purpose (TP)

Same as TC 8.2.2.3.1 but applied to Inter-band CA case.

8.2.2.3.2.2	Conformance requirements

Same as TC 8.2.2.3.1 but applied to Inter-band CA case.

8.2.2.3.2.3	Test description

8.2.2.3.2.3.1	Pre-test conditions

Same as test case 8.2.2.3.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

8.2.2.3.2.3.2	Test procedure sequence

Same as test case 8.2.2.3.1 with the following differences:

-	CA configuration: Inter-band CA replaces Inter-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3.

##### 8.2.2.3.3	CA / RRC connection reconfiguration / SCell addition/ modification/release / Success / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.2.3.1 with the following differences:

-	CA configuration Intra-band non-contiguous CA replaces Intra-band Contiguous CA

#### 8.2.2.4	CA / RRC connection reconfiguration / SCell SI change / Success

##### 8.2.2.4.1	CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band Contiguous CA

8.2.2.4.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with SCell(s) configured }

ensure that {

when { UE receives an RRCConnectionReconfiguration message including sCellToReleaseList and sCellToAddModList with the same concerned SCell to release and add, and with the change of the relevant system information of the configured SCell }

then { UE first releases the concerned SCell and then adds the concerned SCell, configures lower layers to consider the concerned SCell to be in deactivated state and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.2.1.1, clauses 5.3.5.3, 5.3.10.3a and 5.3.10.3b.

[TS 36.331, clause 5.2.1.1]

The UE applies the system information acquisition and change monitoring procedures for the PCell only. For an SCell, E-UTRAN provides, via dedicated signalling, all system information relevant for operation in RRC_CONNECTED when adding the SCell. Upon change of the relevant system information of a configured SCell, E-UTRAN releases and subsequently adds the concerned SCell, which may be done with a single RRCConnectionReconfiguration message.

NOTE 2:	E-UTRAN may configure via dedicated signalling different parameter values than the ones broadcast in the concerned SCell.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3a]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

1>	if the release is triggered by RRC connection re-establishment:

2>	release all SCells that are part of the current UE configuration;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.2.2.4.1.3	Test Description

8.2.2.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.4.1.3.2	Test procedure sequence

Table 8.2.2.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with Cell 3 as SCell addition. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToReleaseList with a SCell release and a sCellToAddModList with a SCell addition for SCell relevant system information changes, the SCellIndex-r10 in both IEs indicate the same SCell as configured in step 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |

8.2.2.4.1.3.3	Specific message contents

Table 8.2.2.4.1.3.3-0: Conditions for specific message contents
in Tables 8.2.2.3.1.3.3-2, 8.2.2.4.1.3.3-3, 8.2.2.4.1.3.3-4, 8.2.2.3.1.3.3-6 and 8.2.2.4.1.3.3-7

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.4.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.4.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.4.1.3.3-2: SCellToAddMod-r10-f2-Add (Table 8.2.2.4.1.3.3-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.4.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.4.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 3 | optional | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.4.1.3.3-4: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.4.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.4.1.3.3-5: RRCConnectionReconfiguration (step 3, Table 8.2.2.4.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 | SCell release for Cell 3 |  |
| } |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add-SIchange | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.4.1.3.3-6: SCellToAddMod-r10-f2-Add-SIchange (Table 8.2.2.4.1.3.3-5)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2-SIchange |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.4.1.3.3-7: RadioResourceConfigCommonSCell-r10-f2-SIchange (Table 8.2.2.4.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| antennaInfoCommon-r10 SEQUENCE { |  |  |  |
| antennaPortsCount | an1 |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 3 | optional | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.4.2	CA / RRC connection reconfiguration / SCell SI change / Success / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.2.4.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

##### 8.2.2.4.3	CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.2.4.1 with the following differences:

-	CA configuration Intra-band non-contiguous CA replaces Intra-band Contiguous CA

#### 8.2.2.5	CA / RRC connection reconfiguration / SCell addition without UL / Success

##### 8.2.2.5.1	CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band Contiguous CA

8.2.2.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with SCell addition without UL carrier }

then { UE adds the new SCell and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.5.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3b.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.2.2.5.1.3	Test Description

8.2.2.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.5.1.3.2	Test procedure sequence

Table 8.2.2.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with SCell Cell 3 addition without UL carrier. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |

8.2.2.5.1.3.3	Specific message contents

Table 8.2.2.5.1.3.3-0: Condition for specific message contents in Table 8.2.2.5.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.5.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.5.1.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 without UL carrier |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.5.1.3.3-2: SCellToAddMod-r10-f2-Add (Table 8.2.2.5.1.3.3-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.5.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.5.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| Ul-Configuration-r10 | Not present |  |  |
| } |  |  |  |

Table 8.2.2.5.1.3.3-4: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.5.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.5.2	CA / RRC connection reconfiguration / SCell addition without UL / Success / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.2.5.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

##### 8.2.2.5.3	CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 8.2.2.5.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

#### 8.2.2.5a	CA / RRC connection reconfiguration / TDD SCell addition without UL

##### 8.2.2.5a.1	CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Periodic / multi-SRS switching

8.2.2.5a.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a TDD SCell addition without UL carrier, and srs-SwitchFromServCellIndex is configured for the TDD SCell, and SoundingRS-UL-ConfigDedicated is configured for the TDD SCell for UL subframe as SRS trigger type 0.}

then { the UE transmit SRS on the TDD SCell periodically using the resource on UL subframe configured in SoundingRS-UL-ConfigDedicated }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList, and SoundingRS-UL-ConfigDedicated is configured for the TDD SCell for special subframe as SRS trigger type 0.}

then { the UE transmit SRS on the TDD SCell periodically using the resource on special subframe configured in SoundingRS-UL-ConfigDedicated }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList, and SoundingRS-UL-ConfigDedicated is configured for the TDD SCell for multi-symbols of special subframe as SRS trigger type 0.}

then { the UE transmit SRS on the TDD SCell periodically using the resource on multi-symbols of special subframe configured in SoundingRS-UL-ConfigDedicated }

}

8.2.2.5a.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3b, TS 36.213, clause 8.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

[TS 36.213, clause 8.2]

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3.

…

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in Subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used.

…

For a TDD serving cell not configured for PUSCH/PUCCH transmission, and trigger type 1, a SRS request field [4] shall be included in DCI format 3B if the value of the higher layer parameter fieldTypeFormat3B is set to 3 or 4. If the UE is configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, a single SRS request field is included in DCI format 3B for a set of the TDD serving cells without PUSCH/PUCCH transmission as given in Table 8.2-0C; otherwise one or more SRS request fields is included in DCI format 3B each corresponding to a TDD serving cell without PUSCH/PUCCH transmission as configured by higher layers. If the UE is configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, and the UE does not support transmit antenna selection or multi-antenna transmission, the SRS request field [4] in DCI format 3B is 1-bit, 2-bits otherwise. For the 1-bit SRS request field [4] in DCI format 3B, a type 1 SRS is triggered if the value of the SRS request field is set to '1' with SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling. For the 2-bit SRS request field [4] in DCI format 3B, and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the SRS request field indicates the SRS parameter set given in Table 8.1-1 with the three sets of SRS parameters, srs-ConfigApDCI-Format4, configured by higher layer signalling. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling for the associated serving cell, is used if a type 1 SRS is triggered.

8.2.2.5a.1.3	Test description

8.2.2.5a.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.5a.1.3.2	Test procedure sequence

Table 8.2.2.5a.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with SCell Cell 3 addition without UL carrier, and setup an SRS configuration on the last symbol of 4th subframe every radio frame in SCell Cell3. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit SRS on the last symbol of 4th subframe every radio frame in SCell Cell3? | --> | SRS | 1 | P |
| 4 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList to release the SRS configuration on 4th subframe and to setup an SRS configuration on the last symbol of the 1st UpPTS every radio frame in SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6 | Check: Does the UE transmit SRS on the last symbol of the 1st UpPTS every radio frame in SCell Cell3? | --> | SRS | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList to add a second SRS configuration on the second last symbol of the 1st UpPTS every radio frame in SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | Check: Does the UE transmit SRS on both 2 symbols of the 1st UpPTS every radio frame in SCell Cell3? | --> | SRS | 3 | P |

8.2.2.5a.1.3.3	Specific message contents

Table 8.2.2.5a.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.5a.1.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.1.3.3-2: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.1.3.3-1)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.1.3.3-3: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-2)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.1.3.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-2)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching

Table 8.2.2.5a.1.3.3-5: SoundingRS-UL-ConfigDedicated-DEFAULT (Table 8.2.2.5a.1.3.3-4)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-22, condition TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndex | 18 | INTEGER (0..1023) See Table 8.2-2 in TS 36.213 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.5a.1.3.3-6: RRCConnectionReconfiguration (step 4, Table 8.2.2.5a.1.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.1.3.3-7: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.1.3.3-6)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.1.3.3-8: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-7)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.1.3.3-9: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-7)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching

Table 8.2.2.5a.1.3.3-10: SoundingRS-UL-ConfigDedicated-DEFAULT (Table 8.2.2.5a.1.3.3-9)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-22, condition TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndex | 16 | INTEGER (0..1023) See Table 8.2-2 in TS 36.213 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.5a.1.3.3-11: RRCConnectionReconfiguration (step 7, Table 8.2.2.5a.1.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.1.3.3-12: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.1.3.3-11)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.1.3.3-13: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-12)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.1.3.3-14: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.1.3.3-12)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-PeriodicConfigDedicatedList-r14 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 2 entries |  |  |
| soundingRS-UL-ConfigDedicated[1] CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-Bandwidth | bw0 |  |  |
| srs-HoppingBandwidth | hbw0 |  |  |
| freqDomainPosition | 0 |  |  |
| duration | TRUE |  |  |
| srs-ConfigIndex | 15 | Second last symbol of 1st UpPTS |  |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  |  |
| } |  |  |  |
| soundingRS-UL-ConfigDedicated[2] CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-Bandwidth | bw0 |  |  |
| srs-HoppingBandwidth | hbw0 |  |  |
| freqDomainPosition | 0 |  |  |
| duration | TRUE |  |  |
| srs-ConfigIndex | 16 | Last symbol of 1st UpPTS |  |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14 | Not Present |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 | Not Present |  |  |
| soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14 | Not Present |  |  |
| } |  |  |  |

##### 8.2.2.5a.2	CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Aperiodic

8.2.2.5a.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a TDD SCell addition without UL carrier, and srs-SwitchFromServCellIndex is configured for the TDD SCell, and SoundingRS-UL-AperiodicConfigDedicated is configured for the TDD SCell for UL subframe as SRS trigger type 1, and a SRS request is received in DCI 1A on the TDD SCell.}

then { the UE transmit SRS on the TDD SCell using the resource on UL subframe configured in SoundingRS-UL-AperiodicConfigDedicated }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE is configured with SoundingRS-UL-AperiodicConfigDedicated for the TDD SCell for UL subframe as SRS trigger type 1, and a SRS request is received in DCI 3B on the TDD SCell.}}

then { the UE transmit SRS on the TDD SCell using the resource on UL subframe configured in SoundingRS-UL-AperiodicConfigDedicated }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE is configured with SoundingRS-UL-AperiodicConfigDedicated for the TDD SCell for special subframe as SRS trigger type 1, and a SRS request is received in DCI 1A on the TDD SCell.}}

then { the UE transmit SRS on the TDD SCell using the resource on special subframe configured in SoundingRS-UL-AperiodicConfigDedicated }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE is configured with SoundingRS-UL-AperiodicConfigDedicated for the TDD SCell for special subframe as SRS trigger type 1, and a SRS request is received in DCI 3B on the TDD SCell.}}

then { the UE transmit SRS on the TDD SCell using the resource on special subframe configured in SoundingRS-UL-AperiodicConfigDedicated }

}

8.2.2.5a.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3b, TS 36.213, clause 8.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

[TS 36.213, clause 8.2]

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3.

…

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in Subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used.

…

For trigger type 1 and DCI format 4/4A/4B three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4/4A/4B indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/0A/0B /6-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1 and DCI formats 1A/2B/2C/2D/6-1A, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. For a serving cell that is not a LAA SCell, the SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. For a serving cell that is a LAA SCell, the SRS timing offset field is 3 bits [4] for DCI formats 1A/2B/2C/2D, with a type 1 SRS triggered if the value of the SRS timing offset field is not set to '000'. The SRS request field is 1 bit [4] for DCI formats 0A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. The 2-bit SRS request field [4] in DCI format 0B indicates the type 1 SRS triggering and PUSCH subframe (as determined in Subclause 8.0) with SRS as given in Table 8.2-0A.

…

For a TDD serving cell not configured for PUSCH/PUCCH transmission, and trigger type 1, a SRS request field [4] shall be included in DCI format 3B if the value of the higher layer parameter fieldTypeFormat3B is set to 3 or 4. If the UE is configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, a single SRS request field is included in DCI format 3B for a set of the TDD serving cells without PUSCH/PUCCH transmission as given in Table 8.2-0C; otherwise one or more SRS request fields is included in DCI format 3B each corresponding to a TDD serving cell without PUSCH/PUCCH transmission as configured by higher layers. If the UE is configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, and the UE does not support transmit antenna selection or multi-antenna transmission, the SRS request field [4] in DCI format 3B is 1-bit, 2-bits otherwise. For the 1-bit SRS request field [4] in DCI format 3B, a type 1 SRS is triggered if the value of the SRS request field is set to '1' with SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling. For the 2-bit SRS request field [4] in DCI format 3B, and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the SRS request field indicates the SRS parameter set given in Table 8.1-1 with the three sets of SRS parameters, srs-ConfigApDCI-Format4, configured by higher layer signalling. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling for the associated serving cell, is used if a type 1 SRS is triggered.

8.2.2.5a.2.3	Test description

8.2.2.5a.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.5a.2.3.2	Test procedure sequence

Table 8.2.2.5a.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with SCell Cell 3 addition without UL carrier, and setup an aperiodic SRS configuration on the last symbol of 4th and 9th subframe of every radio frame in SCell Cell3. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an SRS request by using DCI format 1A on 1st subframe in one radio frame in SCell Cell3. | <-- | DCI: (DCI Format 1A defined in TS 36.508 Table 4.3.6.1.3-1 with following exceptions: RBA()= 5, = 5, SRS request = ‘1’.) | - | - |
| 4 | Check: Does the UE transmit SRS on the last symbol of 9th subframe in the same radio frame in SCell Cell3? | --> | SRS | 1 | P |
| 5 | The SS transmits an SRS request by using DCI format 3B on 1st subframe in one radio frame in SCell Cell3. | <-- | DCI: (DCI Format 3B defined in TS 36.508 Table 4.3.6.1.6A-1) | - | - |
| 6 | Check: Does the UE transmit SRS on the last symbol of 9th subframe in the same radio frame in SCell Cell3? | --> | SRS | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList to change the SRS configuration to the last symbol of the 2 UpPTSs every radio frame in SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS transmits an SRS request by using DCI format 1A on 1st subframe in one radio frame in SCell Cell3. | <-- | DCI: (DCI Format 1A defined in TS 36.508 Table 4.3.6.1.3-1 with following exceptions: RBA()= 5, = 5, SRS request = ‘1’.) | - | - |
| 10 | Check: Does the UE transmit SRS on last symbol of the 2nd UpPTS in the same radio frame in SCell Cell3? | --> | SRS | 3 | P |
| 11 | The SS transmits an SRS request by using DCI format 3B on 1st subframe in one radio frame in SCell Cell3. | <-- | DCI: (DCI Format 3B defined in TS 36.508 Table 4.3.6.1.6A-1) | - | - |
| 12 | Check: Does the UE transmit SRS on last symbol of the 2nd UpPTS in the same radio frame in SCell Cell3? | --> | SRS | 4 | P |

8.2.2.5a.2.3.3	Specific message contents

Table 8.2.2.5a.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.5a.2.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.2.3.3-2: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.2.3.3-1)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.2.3.3-3: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.2.3.3-2)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.2.3.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.2.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 SEQUENCE { |  | 1 entry |  |
| soundingRS-AperiodicSet-r14[1] SEQUENCE { |  |  |  |
| srs-CC-SetIndexList-r14 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r14 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 13 | 4th and 9th subframe every radio frame |  |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigApDCI-Format0-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| srs-ConfigApDCI-Format1a2b2c-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.5a.2.3.3-5: RRCConnectionReconfiguration (step 7, Table 8.2.2.5a.2.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.2.3.3-6: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.2.3.3-5)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.2.3.3-7: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.2.3.3-6)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.2.3.3-8: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.2.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 SEQUENCE { |  | 1 entry |  |
| soundingRS-AperiodicSet-r14[1] SEQUENCE { |  |  |  |
| srs-CC-SetIndexList-r14 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r14 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 11 | Last symbol of every UpPTS |  |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigApDCI-Format0-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| srs-ConfigApDCI-Format1a2b2c-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.5a.3	CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Collision handling / Priority

8.2.2.5a.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED }

ensure that {

when { UE in configured with SRS trigger type 0 in PCell, and UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a TDD SCell addition without UL carrier, and srs-SwitchFromServCellIndex is configured for the TDD SCell, and SoundingRS-UL-ConfigDedicated is configured for the TDD SCell for UL subframe as SRS trigger type 0, and the SRS transmission in PCell overlaps in the same symbol with SRS transmission in TDD SCell }

then { the UE transmit SRS in TDD SCell, but doesn’t transmit SRS in PCell }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE receives a MAC PDU retransmission for DRB in PCell, and corresponding HARQ-ACK transmission overlaps in the same symbol with the SRS transmission in TDD SCell }

then { the UE transmit HARQ-ACK in PCell, but doesn’t transmit SRS in TDD SCell }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE receives UL grant triggering the aperiodic CSI reporting in PCell, and corresponding CSI reporting overlaps in the same symbol with the SRS transmission type 0 in TDD SCell }

then { the UE transmit aperiodic CSI reporting in PCell, but doesn’t transmit SRS in TDD SCell }

}

8.2.2.5a.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3b, TS 36.213, clause 8.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

[TS 36.213, clause 8.2]

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3.

…

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in Subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used.

…

For trigger type 1 and DCI format 4/4A/4B three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4/4A/4B indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/0A/0B /6-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1 and DCI formats 1A/2B/2C/2D/6-1A, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. For a serving cell that is not a LAA SCell, the SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. For a serving cell that is a LAA SCell, the SRS timing offset field is 3 bits [4] for DCI formats 1A/2B/2C/2D, with a type 1 SRS triggered if the value of the SRS timing offset field is not set to '000'. The SRS request field is 1 bit [4] for DCI formats 0A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. The 2-bit SRS request field [4] in DCI format 0B indicates the type 1 SRS triggering and PUSCH subframe (as determined in Subclause 8.0) with SRS as given in Table 8.2-0A.

…

For a TDD serving cell not configured for PUSCH/PUCCH transmission, and trigger type 1, a SRS request field [4] shall be included in DCI format 3B if the value of the higher layer parameter fieldTypeFormat3B is set to 3 or 4. If the UE is configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, a single SRS request field is included in DCI format 3B for a set of the TDD serving cells without PUSCH/PUCCH transmission as given in Table 8.2-0C; otherwise one or more SRS request fields is included in DCI format 3B each corresponding to a TDD serving cell without PUSCH/PUCCH transmission as configured by higher layers. If the UE is configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, and the UE does not support transmit antenna selection or multi-antenna transmission, the SRS request field [4] in DCI format 3B is 1-bit, 2-bits otherwise. For the 1-bit SRS request field [4] in DCI format 3B, a type 1 SRS is triggered if the value of the SRS request field is set to '1' with SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling. For the 2-bit SRS request field [4] in DCI format 3B, and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the SRS request field indicates the SRS parameter set given in Table 8.1-1 with the three sets of SRS parameters, srs-ConfigApDCI-Format4, configured by higher layer signalling. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling for the associated serving cell, is used if a type 1 SRS is triggered.

…

If a UE is not configured with multiple TAGs and the UE is not configured with the parameter srs-UpPtsAdd for trigger type 1, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs, the UE shall not transmit SRS whenever SRS transmission on any serving cells and PUCCH transmission carrying HARQ-ACK and/or positive SR using normal PUCCH format as defined in Subclauses 5.4.1 and 5.4.2A of [3] happen to coincide in the same subframe.

…

For a TDD serving cell d not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding serving cell whose UL transmissions may be interrupted as signalled by srs-SwitchFromServCellIndex. Define the set S(d)= {s0(d)… sN-1(d)} as the set of serving cells that meet the all the following conditions:

-	{s0(d)… sN-1(d)} are in the same band as s0(d).

-	{s0(d)… sN-1(d)} have the same CP as s0(d).

-	{s0(d)… sN-1(d)} are in the same TAG as s0(d).

The following prioritization rules shall be applied when transmitting SRS over serving cell d when the simultaneous transmission of SRS and other signal/channel would result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the UE-EUTRA-Capability [12]:

-	If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit SRS. Otherwise,

-	if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise,

-	if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/SRS transmission.

8.2.2.5a.3.3	Test description

8.2.2.5a.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.5a.3.3.2	Test procedure sequence

Table 8.2.2.5a.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message. The RRCConnectionReconfiguration contains sCellToAddModList with SCell Cell 3 addition without UL carrier, and setup a periodic SRS configuration on the last symbol of 4th subframe of every radio frame in SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit SRS in PCell Cell 1 on the last symbol of 4th subframe in radio frame k1? (Note 1) | --> | SRS | 1 | F |
| 4 | Check: Does the UE transmit SRS in SCell Cell 3 on the last symbol of 4th subframe in radio frame k1? (Note 1) | --> | SRS | 1 | P |
| 5 | The SS indicates a new transmission on PDCCH and transmits a MAC PDU (containing an RLC PDU) in PCell Cell 1 on 10th subframe of radio frame k2, with content set so that UE could not successfully decode the data from its soft buffer. | <-- | MAC PDU |  |  |
| 6 | Check: Does the UE transmit a HARQ NACK in PCell Cell 1 on 4th subframe of radio frame k2+1? | --> | HARQ NACK | 2 | P |
| 7 | Check: Does the UE transmit SRS on the last symbol of 4th subframe of radio frame k2+1 in SCell Cell3? | --> | SRS | 2 | F |
| 8 | After 1 second of step 5, the SS sends an UL grant to trigger aperiodic CSI reporting in PCell Cell 1 on 10th subframe of radio frame k3. | <-- | (UL Grant) | - | - |
| 9 | Check: Does the UE perform aperiodical CQI reporting in PCell Cell 1 on 4th subframe of radio frame k3+1? | --> | (CQI) | 3 | P |
| 10 | Check: Does the UE transmit SRS on the last symbol of 4th subframe of radio frame k3+1 in SCell Cell3? | --> | SRS | 3 | F |
| Note 1: Radio frame k1 is randomly selected after step 2. |  |  |  |  |  |

8.2.2.5a.3.3.3	Specific message contents

Table 8.2.2.5a.3.3.3-1: RRCConnectionReconfiguration (Preamble)

| Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated ::= SEQUENCE { |  |  |  |  |
| physicalConfigDedicated ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated ::= CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| srs-ConfigIndex | 5 | See Table 8.2-1 in TS 36.213 | FDD |  |
|  | 13 | INTEGER (0..1023) See Table 8.2-2 in TS 36.213 | TDD |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.2.5a.3.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.2.5a.3.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.3.3.3-3: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.3.3.3-2)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.3.3.3-4: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.3.3.3-3)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching

Table 8.2.2.5a.3.3.3-5: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.3.3.3-3)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching

Table 8.2.2.5a.3.3.3-6: SoundingRS-UL-ConfigDedicated-DEFAULT (Table 8.2.2.5a.3.3.3-5)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-22, condition TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndex | 18 | INTEGER (0..1023) See Table 8.2-2 in TS 36.213 |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.5a.4	CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Collision handling / flexible SRS transmitting

8.2.2.5a.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with a TDD SCell without UL carrier and configured with srs-SwitchFromServCellIndex }

ensure that {

when { UE is configured with soundingRS-FlexibleTiming-r14 and UE is configured with SoundingRS-UL-AperiodicConfigDedicated for the TDD SCell for UL subframe as SRS trigger type 1, and a SRS request is received in DCI 3B on the TDD SCell, and the SRS transmission in subframe n+k happens to overlap with a HARQ-ACK transmission }

then { the UE transmit SRS in a later subframe n+k+l, where l=max(5, TSRS,1) }

}

8.2.2.5a.4.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3b, TS 36.213, clause 8.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

[TS 36.213, clause 8.2]

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3.

…

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in Subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used.

…

For trigger type 1 and DCI format 4/4A/4B three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4/4A/4B indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/0A/0B /6-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1 and DCI formats 1A/2B/2C/2D/6-1A, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. For a serving cell that is not a LAA SCell, the SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. For a serving cell that is a LAA SCell, the SRS timing offset field is 3 bits [4] for DCI formats 1A/2B/2C/2D, with a type 1 SRS triggered if the value of the SRS timing offset field is not set to '000'. The SRS request field is 1 bit [4] for DCI formats 0A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. The 2-bit SRS request field [4] in DCI format 0B indicates the type 1 SRS triggering and PUSCH subframe (as determined in Subclause 8.0) with SRS as given in Table 8.2-0A.

…

For a TDD serving cell not configured for PUSCH/PUCCH transmission, and trigger type 1, a SRS request field [4] shall be included in DCI format 3B if the value of the higher layer parameter fieldTypeFormat3B is set to 3 or 4. If the UE is configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, a single SRS request field is included in DCI format 3B for a set of the TDD serving cells without PUSCH/PUCCH transmission as given in Table 8.2-0C; otherwise one or more SRS request fields is included in DCI format 3B each corresponding to a TDD serving cell without PUSCH/PUCCH transmission as configured by higher layers. If the UE is configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, and the UE does not support transmit antenna selection or multi-antenna transmission, the SRS request field [4] in DCI format 3B is 1-bit, 2-bits otherwise. For the 1-bit SRS request field [4] in DCI format 3B, a type 1 SRS is triggered if the value of the SRS request field is set to '1' with SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling. For the 2-bit SRS request field [4] in DCI format 3B, and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the SRS request field indicates the SRS parameter set given in Table 8.1-1 with the three sets of SRS parameters, srs-ConfigApDCI-Format4, configured by higher layer signalling. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling for the associated serving cell, is used if a type 1 SRS is triggered.

…

For a serving cell that is not a LAA SCell, a non-BL/CE UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe/slot n of serving cell c shall commence SRS transmission in the first subframe satisfying , and

-	if the positive SRS request in PDCCH/SPDCCH with DCI format 7-0A/7-1A is detected in slot n,for TDD

-	 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with DCI format other than DCI format 7-0A/7-1A is in the UE-specific search space,

-	otherwise, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with

where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3. For a TDD serving cell not configured for PUSCH/PUCCH transmission and the positive SRS request detected in PDCCH/EPDCCH scheduling PDSCH and the UE configured with soundingRS-FlexibleTiming-r14 by higher layer signalling, if the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in the first subframe  happens to overlap with a HARQ-ACK transmission for any serving cell, the UE shall commence SRS transmission in subframe n + k + l, where l = max( 5, ).

8.2.2.5a.4.3	Test description

8.2.2.5a.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.5a.4.3.2	Test procedure sequence

Table 8.2.2.5a.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a sCellToAddModList with SCell Cell 3 addition without UL carrier, and setup an aperiodic SRS configuration on the last symbol of 3rd and 8th subframe of every radio frame in SCell Cell3. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits DCI format 1A on 1st subframe in one radio frame in SCell Cell3, and a MAC PDU is transmitted. The corresponding HARQ-ACK is expected to be in 8th subframe. | <-- | DCI: (DCI Format 1A defined in TS 36.508 Table 4.3.6.1.3-1 with following exceptions: RBA()= 5, = 5, SRS request = ‘0’.) | - | - |
| 4 | The SS transmits an SRS request by using DCI format 3B on 2nd subframe in the same radio frame in SCell Cell3. The corresponding SRS transmission is expected to be in 8th subframe. |  | DCI: (DCI Format 3B defined in TS 36.508 Table 4.3.6.1.6A-1) |  |  |
| 5 | Check: Does the UE transmit SRS on the last symbol of 8th subframe in the same radio frame in SCell Cell3? | --> | SRS | 1 | F |
| 6 | Check: Does the UE transmit SRS on the last symbol of 3rd subframe in the next radio frame in SCell Cell3? | --> | SRS | 1 | P |

8.2.2.5a.4.3.3	Specific message contents

Table 8.2.2.5a.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.5a.4.3.2-1)

Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.2.5a.4.3.3-2: SCellToAddMod-r10-DEFAULT (Table 8.2.2.5a.4.3.3-1)

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19D, condition SRS-Switching

Table 8.2.2.5a.4.3.3-3: RadioResourceConfigCommonSCell-r10-DEFAULT (Table 8.2.2.5a.4.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-13A, condition SRS-Switching |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| ul-Configuration-r14 SEQUENCE { |  |  | SRS-Switching |
| soundingRS-FlexibleTiming-r14 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.5a.4.3.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT (Table 8.2.2.5a.4.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A, condition SRS-Switching |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 SEQUENCE { |  | 1 entry |  |
| soundingRS-AperiodicSet-r14[1] SEQUENCE { |  |  |  |
| srs-CC-SetIndexList-r14 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r14 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 12 | 3rd and 8th subframe every radio frame |  |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigApDCI-Format0-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| srs-ConfigApDCI-Format1a2b2c-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.2.6	RRC connection reconfiguration/ UE Assistance Information

##### 8.2.2.6.1	RRC connection reconfiguration/ UE Assistance Information/power preference indication setup and release

8.2.2.6.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and it is not configured to provide power preference indications }

ensure that {

when { UE is caused to send UEAssistanceInformation message indicating "normal" }

then { the UE does not transmit a UEAssistanceInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide power preference indications with T340 set to 0s}

ensure that {

when { UE prefers a configuration primarily optimised for power saving and sends the message UEAssistanceInformation}

then { the UE includes powerPrefIndication  set to lowPowerConsumption in the message UEAssistanceInformation }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide power preference indications and it has previously sent UEAssistanceInformation message indicating "lowPowerConsumption" }

ensure that {

when { UE is caused to send UEAssistanceInformation message indicating "lowPowerConsumption"  }

then { the UE does not transmit a UEAssistanceInformation message with powerPrefIndication  set to lowPowerConsumption }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide power preference indications }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing a otherConfig that includes the powerPrefIndicationConfig set to Release }

then { the UE does not transmit any more UEAssistanceInformation message }

8.2.2.6.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.10.9, 5.6.10.2, 5.6.10.3 and 6.3.6. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.10.9]

The UE shall:

...

1>	if the received otherConfig includes the powerPrefIndicationConfig:

2>	if powerPrefIndicationConfig is set to setup:

3>	consider itself to be configured to provide power preference indications in accordance with 5.6.10;

2>	else:

3>	consider itself not to be configured to provide power preference indications;

[TS 36.331, clause 5.6.10.2]

A UE capable of providing power preference indications in RRC_CONNECTED may initiate the procedure in several cases including upon being configured to provide power preference indications and upon change of power preference.

Upon initiating the procedure, the UE shall:

1>	if configured to provide power preference indications:

2>	if the UE did not transmit a UEAssistanceInformation message since it was configured to provide power preference indications; or

2>	if the current power preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer T340 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

[TS 36.331, clause 5.6.10.3]

The UE shall set the contents of the UEAssistanceInformation message:

1>	if the UE prefers a configuration primarily optimised for power saving:

2>	set powerPrefIndication to lowPowerConsumption;

1>	else:

2>	start or restart timer T340 with the timer value set to the powerPrefIndicationTimer;

2>	set powerPrefIndication to normal;

The UE shall submit the UEAssistanceInformation message to lower layers for transmission.

[TS 36.331, clause 6.3.6]

powerPrefIndicationTimer

Prohibit timer for Power Preference Indication reporting. Value in seconds. Value s0 means prohibit timer is set to 0 second or not set, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on.

8.2.2.6.1.3	Test Description

8.2.2.6.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.6.1.3.2	Test procedure sequence

Table 8.2.2.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 2 | Check: Does the UE send a UEAssistanceInformation message within the next 20s? | --> | UEAssistanceInformation | 1 | F |
| 3 | The SS transmits an RRCConnectionReconfiguration message containing a otherConfig includes the powerPrefIndicationConfig set to Setup | <-- | RRCConnectionReconfiguration |  |  |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 4A | The SS starts Timer T340 |  |  |  |  |
|  | EXCEPTION: Steps 4Ba1 to 4Bb1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  |  |  |  |
| 4Ba1 | The UE transmits a UEAssistanceInformation message. | --> | UEAssistanceInformation | - | - |
| 4Ba2 | Wait for the Timer T340 Expiry to ensure that T340 timer has expired on the device. |  |  |  |  |
| 4Bb1 | The SS waits for the T340 expiry |  |  |  |  |
| 5 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  | - | - |
| 6 | Check: Does the UE transmit a UEAssistanceInformation message with powerPrefIndication set to lowPowerConsumption? | --> | UEAssistanceInformation | 2 | P |
| 7 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  |  |  |
| 8 | Check: Does the UE send a UEAssistanceInformation message within the next 20s? | --> | UEAssistanceInformation | 3 | F |
| 9 | The SS transmits an RRCConnectionReconfiguration message containing a otherConfig includes the powerPrefIndicationConfig set to Release. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 12 | Check: Does the UE send a UEAssistanceInformation message within the next 20s? | --> | UEAssistanceInformation | 4 | F |
| 13 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  | - | - |
| 14 | Check: Does the UE send a UEAssistanceInformation message within the next 20s? | --> | UEAssistanceInformation | 4 | F |
| Note 1: This indication may be triggered with the AT command +CEPPI/+CNMPSD or any alternative way. Note 2: This indication may be triggered with the AT command +CEPPI or any alternative way. |  |  |  |  |  |

8.2.2.6.1.3.3	Specific message contents

Table 8.2.2.6.1.3.3-1: RRCConnectionReconfiguration (Steps 3 and 9, Table 8.2.2.6.1.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition OtherConfig

Table 8.2.2.6.1.3.3-2: OtherConfig-r9 (step 3, Table 8.2.2.6.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.3-28 condition Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 SEQUENCE { |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| powerPrefIndicationTimer-r11 | s0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.1.3.3-1A: UEAssistanceInformation (Step 4B a1, Table 8.2.2.6.1.3.2-1)

Derivation Path: 36.508, Table 4.6.1-26 condition Normal

Table 8.2.2.6.1.3.3-2: UEAssistanceInformation (Step 6, Table 8.2.2.6.1.3.2-1)

Derivation Path: 36.508, Table 4.6.1-26 condition Low_Power

Table 8.2.2.6.1.3.3-3: OtherConfig-r9 (Step 9, Table 8.2.2.6.1.3.2-3)

Derivation Path: 36.508, Table 4.6.3-28 condition Release

##### 8.2.2.6.2	RRC connection reconfiguration/ UE Assistance Information/power preference indication release on connection re-establishment

8.2.2.6.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state & configured to provide power preference indication }

ensure that {

when { RRC Connection re-establishment procedure is initiated }

then { UE shall stop T340 }

}

(2)

with { UE in E-UTRA RRC_Connected state & configured to provide power preference indication }

ensure that {

when { RRC Connection re-establishment procedure is initiated }

then { UE shall release the powerPrefIndicationConfig }

}

8.2.2.6.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.6.10.1, 5.6.10.2, 5.6.10.3 and 5.3.7.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331 clause 5.6.10.1]

Figure 5.6.10.1-1: UE Assistance Information

The purpose of this procedure is to inform E-UTRAN of the UE’s power saving preference. Upon configuring the UE to provide power preference indications E-UTRAN may consider that the UE does not prefer a configuration primarily optimised for power saving until the UE explicitly indicates otherwise.

[TS 36.331 clause 5.6.10.2]

A UE capable of providing power preference indications in RRC_CONNECTED may initiate the procedure in several cases including upon being configured to provide power preference indications and upon change of power preference.

Upon initiating the procedure, the UE shall:

1>	if configured to provide power preference indications:

2>	if the UE did not transmit a UEAssistanceInformation message since it was configured to provide power preference indications; or

2>	if the current power preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer T340 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

[TS 36.331 clause 5.6.10.3]

The UE shall set the contents of the UEAssistanceInformation message:

1>	if the UE prefers a configuration primarily optimised for power saving:

2>	set powerPrefIndication to lowPowerConsumption;

1>	else:

2>	start or restart timer T340 with the timer value set to the powerPrefIndicationTimer;

2>	set powerPrefIndication to normal;

The UE shall submit the UEAssistanceInformation message to lower layers for transmission.

[TS 36.331 clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	upon integrity check failure indication from lower layers; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5;

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T311;

1>	suspend all RBs except SRB0;

1>	reset MAC;

1>	release the SCell(s), if configured, in accordance with 5.3.10.3a;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	release obtainLocationConfig, if configured;

1>	release idc-Config, if configured;

1>	release measSubframePatternPCell, if configured;

1>	if connected as an RN and configured with an RN subframe configuration:

2>	release the RN subframe configuration;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

8.2.2.6.2.3	Test description

8.2.2.6.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 & Cell 2.

-	System information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.6.2.3.2	Test procedure sequence

Table 8.2.2.6.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.2.6.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | The power level values are assigned to satisfy SrxlevCell 2 < 0 such that selecting Cell 1 is guaranteed (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.2.6.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1 & to configure power preference indication. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 2A | SS Starts Timer T340 |  |  |  |  |
|  | EXCEPTION: Steps 2Ba1 to 2Bb1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  |  |  |  |
| 2Ba1 | The UE transmits a UEAssistanceInformation message. | --> | UEAssistanceInformation | - | - |
| 2Ba2 | Wait for the Timer T340 Expiry to ensure that T340 timer has expired on the device. |  |  |  |  |
| 2Bb1 | The SS Waits for the T340 Timer Expiry. |  |  |  |  |
| 3 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note1). | - | - | - | - |
| 4 | UE transmits UEAssistanceInformation message with powerPrefIndication IE set to lowPowerConsumption. | --> | UEAssistanceInformation | - | - |
| 5 | Cause the UE to send UEAssistanceInformation message indicating "normal"(Note2). | - | - | - | - |
| 6 | UE transmits UEAssistanceInformation message with powerPrefIndication IE set to normal and starts timer T340 with the timer value set to the powerPrefIndicationTimer. | --> | UEAssistanceInformation | - | - |
| 7 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.2.6.2.3.2-1. | - | - | - | - |
| 8 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 10 the steps specified in Table 8.2.2.6.2.3.2-3 should take place. | - | - | - | - |
| 10 | The SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.2.2.6.2.3.2-1. | - | - | - | - |
| 11 | The UE releases the configured powerPrefIndicationConfig, stops timer T340 and transmit an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 12 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 13 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1. | --> | RRCConnectionReestablishmentComplete | - | - |
| 14 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note1). | - | - | - | - |
| 15 | Check: Does the UE transmit UEAssistanceInformation message within the next 20s? (Note 3). | --> | UEAssistanceInformation | 2 | F |
| 16 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1 & to configure power preference indication. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 17A | SS Starts Timer T340 |  |  |  |  |
|  | EXCEPTION: Steps 17Ba1 to 17Bb1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  |  |  |  |
| 17Ba1 | The UE transmits a UEAssistanceInformation message. | --> | UEAssistanceInformation | - | - |
| 17Ba2 | Wait for the Timer T340 Expiry to ensure that T340 timer has expired on the device. |  |  |  |  |
| 17Bb1 | The SS Waits for the T340 Timer Expiry. |  |  |  |  |
| 18 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note1). | - | - | - | - |
| 19 | Check: Does the UE transmit UEAssistanceInformation message with powerPrefIndication IE set to lowPowerConsumption? (Note 4). | --> | UEAssistanceInformation | 1 | P |
| Note 1: This indication may be triggered with the AT command +CEPPI/+CNMPSD or any alternative way. Note 2: This indication may be triggered with the AT command +CEPPI or any alternative way. Note 3: This is a negative check for TP2 to make sure that the UE has released the configured powerPrefIndicationConfig at step 11. Note 4: The total time to execute step 6 – step 19 should be less than the T340 timer value powerPrefIndicationTimer configured at step 1 and started at step 6. This is to make sure that timer T340 has been stopped successfully at step 11 and only then UE will initiate UEAssistanceInformation message with powerPrefIndication IE set to lowPowerConsumption at step 19. |  |  |  |  |  |

Table 8.2.2.6.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.2.6.2.3.3	Specific message contents

Table 8.2.2.6.2.3.3-0: Condition for specific message contents in Table 8.2.2.6.2.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.6.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.6.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, conditions MEAS & OtherConfig

Table 8.2.2.6.2.3.3-2: MeasConfig (Table 8.2.2.6.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-3: OtherConfig-r9 (Table 8.2.2.6.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-28 condition Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 SEQUENCE { |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| powerPrefIndicationTimer-r11 | s600 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-4: UEAssistanceInformation (step 4, 15, 17Ba1 & 19, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-26 condition Low_Power |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| powerPreferenceIndication-r11 | lowPowerConsumption |  |  |

Table 8.2.2.6.2.3.3-5: UEAssistanceInformation (step 2 Ba1 & 6, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-26 condition Normal |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| powerPreferenceIndication-r11 | normal |  |  |

Table 8.2.2.6.2.3.3-6: MeasurementReport (step 8, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPcell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-7: RRCConnectionReconfiguration (step 9, Table 8.2.2.6.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.2.6.2.3.3-8: MobilityControlInfo (8.2.2.6.2.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-9: RRCConnectionReestablishmentRequest (step 11, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-10: RRCConnectionReestablishmentComplete (step 13, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-11: RRCConnectionReconfiguration (step 16, Table 8.2.2.6.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 & condition OtherConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.2.3.3-12: OtherConfig-r9 (Table 8.2.2.6.2.3.3-11)

| Derivation Path: 36.508, Table 4.6.3-28, condition Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 SEQUENCE { |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| powerPrefIndicationTimer-r11 | s10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.6.3	RRC connection reconfiguration/ UE Assistance Information/T340 running

8.2.2.6.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and T340 is running }

ensure that {

when { UE is caused to send UEAssistanceInformation message }

then { the UE does not transmit a UEAssistanceInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and the UE has received a new value for powerPrefIndicationTimer}

ensure that {

when { UE is caused to send UEAssistanceInformation message indicating "lowPowerConsumption" after T340 expired }

then { the UE does transmits a UEAssistanceInformation message with powerPrefIndication  set to lowPowerConsumption}

}

8.2.2.6.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.6.10.2 and 5.6.10.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.6.10.2]

A UE capable of providing power preference indications in RRC_CONNECTED may initiate the procedure in several cases including upon being configured to provide power preference indications and upon change of power preference.

Upon initiating the procedure, the UE shall:

1>	if configured to provide power preference indications:

2>	if the UE did not transmit a UEAssistanceInformation message since it was configured to provide power preference indications; or

2>	if the current power preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer T340 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

[TS 36.331, clause 5.6.10.3]

The UE shall set the contents of the UEAssistanceInformation message:

1>	if the UE prefers a configuration primarily optimised for power saving:

2>	set powerPrefIndication to lowPowerConsumption;

1>	else:

2>	start or restart timer T340 with the timer value set to the powerPrefIndicationTimer;

2>	set powerPrefIndication to normal;

The UE shall submit the UEAssistanceInformation message to lower layers for transmission.

8.2.2.6.3.3	Test Description

8.2.2.6.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.6.3.3.2	Test procedure sequence

Table 8.2.2.6.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a otherConfig includes the powerPrefIndicationConfig set to Setup | <-- | RRCConnectionReconfiguration |  |  |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 2A | SS starts Timer T340 |  |  |  |  |
|  | EXCEPTION: Steps 2Ba1 to 2Bb1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  |  |  |  |
| 2Ba1 | The UE transmits a UEAssistanceInformation message. | --> | UEAssistanceInformation | - |  |
| 2Ba2 | Wait for the T340 Timer Expiry to ensure that T340 timer has expired on the device side. |  |  |  |  |
| 2Bb1 | SS waits for the T340 Timer Expiry. |  |  |  |  |
| 3 | Cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  | - | - |
| 4 | The UE transmits a UEAssistanceInformation message with powerPrefIndication set to lowPowerConsumption. | --> | UEAssistanceInformation | - | - |
| 5 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 6 | The UE transmits a UEAssistanceInformation message with powerPrefIndication set to normal. The UE starts T340. | --> | UEAssistanceInformation | - | - |
| 7 | Before T340 expires, cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  | - | - |
| 8 | Check: Does the UE send a UEAssistanceInformation message before T340 expires? | --> | UEAssistanceInformation | 1 | F |
| 9-10 | Void |  |  | - | - |
| - | EXCEPTION: Steps 10Aa1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. | - | - | - | - |
| 10Aa1 | The UE transmits a UEAssistanceInformation message within 5s with powerPrefIndication set to lowPowerConsumption | --> | UEAssistanceInformation | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message containing a otherConfig includes the powerPrefIndicationConfig set to Setup and a new value for powerPrefIndicationTimer. | <-- | RRCConnectionReconfiguration |  |  |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 12A | SS starts Timer T340 |  |  |  |  |
|  | EXCEPTION: Steps 12Ba1 to 12Bb1 describes a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  |  |  |  |
| 12Ba1 | The UE transmits a UEAssistanceInformation message. | --> | UEAssistanceInformation | - | - |
| 12Ba2 | Wait for the T340 Timer Expiry to ensure that T340 timer has expired on the device side. |  |  |  |  |
| 12Bb1 | SS Waits for the T340 Timer Expiry |  |  |  |  |
| 13 | Cause the UE to send UEAssistanceInformation message indicating "normal". (Note 2) |  |  | - | - |
| 14 | The UE transmits a UEAssistanceInformation message with powerPrefIndication set to normal. The UE starts T340. Start timer T=300s(this is the same value as the powerPrefIndicationTimer value set in Step 1). | --> | UEAssistanceInformation | - | - |
| 15 | After T340 expires but before T expires cause the UE to send UEAssistanceInformation message indicating "lowPowerConsumption". (Note 1) |  |  | - | - |
| 16 | Check: Does the UE send a UEAssistanceInformation message with powerPrefIndication set to lowPowerConsumption? | --> | UEAssistanceInformation | 2 | P |
| Note 1: This indication may be triggered with the AT command +CEPPI/+CNMPSD or any alternative way. Note 2: This indication may be triggered with the AT command +CEPPI or any alternative way. |  |  |  |  |  |

8.2.2.6.3.3.3	Specific message contents

Table 8.2.2.6.3.3.3-1: RRCConnectionReconfiguration (Steps 1 and 11 Table 8.2.2.6.3.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition OtherConfig

Table 8.2.2.6.3.3.3-2: OtherConfig-r9 (Step 1, Table 8.2.2.6.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.3-28 condition Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 SEQUENCE { |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| powerPrefIndicationTimer-r11 | s300 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.6.3.3.3-3: UEAssistanceInformation (Steps 4, 10 and 12Ba1 Table 8.2.2.6.3.3.2-1)

Derivation Path: 36.508, Table 4.6.1-26 condition Low_Power

Table 8.2.2.6.3.3.3-4: UEAssistanceInformation (Steps 2 Ba1, 6 and 14, Table 8.2.2.6.3.3.2-1)

Derivation Path: 36.508, Table 4.6.1-26 condition Normal

Table 8.2.2.6.3.3.3-5: OtherConfig-r9 (Step 11, Table 8.2.2.6.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.3-28 condition Setup |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 SEQUENCE { |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| powerPrefIndicationTimer-r11 | s30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.6.4	Void

##### 8.2.2.6.5	Void

##### 8.2.2.6.6	Void

#### 8.2.2.7	CA / RRC connection reconfiguration / sTAG addition/modification/release / Success

##### 8.2.2.7.1	CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Intra-band Contiguous CA

8.2.2.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing stag-ToAddModList with a sTAG addition and sCellToAddModList with a SCell addition which is associated with the new sTAG }

then { UE adds the new SCell and associates it with the new sTAG, sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing stag-ToAddModList with a sTAG modification }

then { UE modifies the affected sTAG dedicated configurations and sends an RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToReleaseList with a sCellIndex equalling to one of the current UE SCell configuration and stag-ToReleaseList to release the empty sTAG after the last SCell is removed from this sTAG }

then { UE releases this SCell and sTAG and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.7.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.3a, 5.3.10.4 and 6.3.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 3:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.3a]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig other than stag-ToReleaseList and stag-ToAddModList;

1>	if the received mac-MainConfig includes the stag-ToReleaseList:

2>	for each STAG-Id value included in the stag-ToReleaseList that is part of the current UE configuration:

3>	release the STAG indicated by STAG-Id;

1>	if the received mac-MainConfig includes the stag-ToAddModList:

2>	for each stag-Id value included in stag-ToAddModList that is not part of the current UE configuration (STAG addition):

3>	add the STAG, corresponding to the stag-Id, in accordance with the received timeAlignmentTimerSTAG;

2>	for each stag-Id value included in stag-ToAddModList that is part of the current UE configuration (STAG modification):

3>	reconfigure the STAG, corresponding to the stag-Id, in accordance with the received timeAlignmentTimerSTAG;

[TS 36.331, clause 6.3.2]

MAC-MainConfig field descriptions

…

stag-Id Indicates the TAG of an SCell, see TS 36.321 [6]. If the field is not configured for an SCell (e.g. absent in MAC-MainConfigSCell), the SCell is part of the PTAG.

stag-ToAddModList, stag-ToReleaseList Used to configure one or more STAGs. E-UTRAN ensures that a STAG contains at least one SCell with configured uplink. If, due to SCell release a reconfiguration would result in an ‘empty’ TAG, E-UTRAN includes release of the concerned TAG.

timeAlignmentTimerSTAG Indicates the value of the time alignment timer for an STAG, see TS 36.321 [6].

…

8.2.2.7.1.3	Test Description

8.2.2.7.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell3 is the SCell to be added.

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.7.1.3.2	Test procedure sequence

Table 8.2.2.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing stag-ToAddModList with a sTAG addition and sCellToAddModList with SCell Cell 3 addition which is associated with the added sTAG. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS transmits an RRCConnectionReconfiguration message containing a stag-ToAddModList with timeAlignmentTimerSTAG-r11 modification. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 5 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToReleaseList with an sCellIndex equalling to the Cell 3 cell index in the current UE configuration and stag-ToReleaseList-r11 with STAG-Id equalling to the stag-id with which the Cell 3 is associated. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.2.2.7.1.3.3	Specific message contents

Table 8.2.2.7.1.3.3-0: Conditions for specific message contents
in Tables 8.2.2.7.1.3.3-2, 8.2.2.7.1.3.3-3, 8.2.2.7.1.3.3-7, 8.2.2.7.1.3.3-8 and 8.2.2.7.1.3.3-11

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.7.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.7.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8, condition SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig SEQUENCE { |  |  |  |
| stag-ToAddModList-r11 SEQUENCE (SIZE (1..maxSTAG-r11)) OF SEQUENCE { |  |  |  |
| stag-Id-r11[1] | 1 |  |  |
| timeAlignmentTimerSTAG-r11[1] | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-2: SCellToAddMod-r10-f2-Add (Table 8.2.2.7.1.3.3-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.7.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.7.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A, condition UL CA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 3 |  | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-4: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.7.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 | PhysicalConfigDedicatedSCell-r10-DEFAULT |  |  |
| mac-MainConfigSCell-r11 SEQUENCE { |  |  |  |
| stag-Id-r11 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-5: RRCConnectionReconfiguration (step 3, Table 8.2.2.7.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig SEQUENCE { |  |  |  |
| stag-ToAddModList-r11 SEQUENCE (SIZE (1..maxSTAG-r11)) OF SEQUENCE { |  |  |  |
| stag-Id-r11[1] | 1 |  |  |
| timeAlignmentTimerSTAG-r11[1] | sf750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-6: RRCConnectionReconfiguration (step 5, Table 8.2.2.7.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig SEQUENCE { |  |  |  |
| stag-ToReleaseList-r11 SEQUENCE (SIZE (1..maxSTAG-r11)) OF SEQUENCE { |  |  |  |
| stag-Id-r11[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 | SCell release for Cell 3 |  |
| } |  |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.2.7.2	CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.2.7.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

##### 8.2.2.7.3	CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.2.7.1 with the following differences:

-	CA configuration: Intra-band non-contiguous CA replaces Intra-band Contiguous CA

#### 8.2.2.8	RRC connection reconfiguration / SIB1 information / Success

8.2.2.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SIB1 information with a change of TAI}

then { UE reads the SIB1 information and sends an RRCConnectionReconfigurationComplete message followed by TRACKING AREA UPDATE REQUEST message to update the registration of the actual tracking area }

}

8.2.2.8.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.2.2.7, and 5.3.5.3. Unless otherwise stated these are Rel-11 requirements

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perform the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

8.2.2.8.3	Test Description

8.2.2.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.2.8.3.2	Test procedure sequence

Table 8.2.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS stops transmitting SIB1 information on Cell 1. | - | - | - | - |
| 1 | The SS transmits an RRConnectionReconfiguration message containing SIB1 information. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does UE transmit a TRACKING AREA UPDATE REQUEST message to update the registration of the actual tracking area? | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST | 1 | P |
| 4 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | The UE send a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 6 | The SS resumes transmitting SIB1 information. | - | - | - | - |

8.2.2.8.3.3	Specific message contents

Table 8.2.2.8.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.8.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | systemInfomationBlockType1 |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.8.3.3-2: SystemInformationBlockType1 (Table 8.2.2.8.3.3-1 and step 6, Table 8.2.2.8.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| trackingAreaCode | TAC =2 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.2.9	Dual Connectivity / RRC connection reconfiguration

##### 8.2.2.9.1	RRC connection reconfiguration / PSCell addition and SCG release / SCG / DRB

8.2.2.9.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SCG-Configuration-r12 with radioResourceConfigDedicatedSCG, drb-ToAddModListSCG and the drb-Identity is not part of the current UE configuration }

then { UE configures the PSCell and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SCG-Configuration-r12 with release }

then { UE reconfigures affected DRBs and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.9.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.10, 5.3.10.11 and 5.3.10.3a1.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.10]

1>	if the received scg-Configuration is set to release:

2>	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

2>	stop timer T313, if running;

2>	stop timer T307, if running;

1>	else:

…

2>	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

[TS 36.331, clause 5.3.10.11]

The UE shall:

1>	if the received radioResourceConfigDedicatedSCG includes the drb-ToAddModListSCG:

2>	for each drb-Identity value included in the drb-ToAddModListSCG perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

…

2>	else (i.e. add SCG DRB):

3>	establish a PDCP entity and configure it with the current SCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModListSCG;

3>	establish an SCG RLC entity or entities and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

8.2.2.9.1.3	Test Description

8.2.2.9.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.2.9.1.3.2	Test procedure sequence

Table 8.2.2.9.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a SCG-Configuration-r12 with PSCell-r12. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 2A | The UE transmits an ULInformationTransfer message containing the ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |
| 3 | SS transmits RRCConnectionReconfiguration message containing a SCG-Configuration-r12 with release. RRCConnectionReconfiguration message contains the DEACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 5 | The UE transmits an ULInformationTransfer message containing the DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |

8.2.2.9.1.3.3	Specific message contents

Table 8.2.2.9.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.9.1.3.2-1)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition PSCell_Add_SCG_DRB

Table 8.2.2.9.1.3.3-2: RRCConnectionReconfiguration (step 3, Table 8.2.2.9.1.3.2-1)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition PSCell_Release

##### 8.2.2.9.2	RRC connection reconfiguration / PSCell addition and SCG release / Split DRB

8.2.2.9.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SCG-Configuration-r12 with radioResourceConfigDedicatedSCG, drb-ToAddModListSCG, drb-ToAddModList including a drb-Identity not part of the current UE configuration }

then { UE configures the PSCell and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing SCG-Configuration-r12 with release when the current UE configuration includes split DRBs}

then { UE reconfigures affected DRBs and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.9.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.10, 5.3.10.11 and 5.3.10.3a1.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.10]

1>	if the received scg-Configuration is set to release:

2>	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

2>	stop timer T313, if running;

2>	stop timer T307, if running;

1>	else:

…

2>	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

[TS 36.331, clause 5.3.10.11]

The UE shall:

1>	if the received radioResourceConfigDedicatedSCG includes the drb-ToAddModListSCG:

2>	for each drb-Identity value included in the drb-ToAddModListSCG perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

2>	if drb-ToAddModList is received and includes the drb-Identity value (i.e. add split DRB):

3>	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

3>	establish an MCG RLC entity and an MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

3>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

8.2.2.9.2.3	Test Description

8.2.2.9.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.2.9.2.3.2	Test procedure sequence

Table 8.2.2.9.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing a SCG-Configuration-r12 with drb-ToAddModList and PSCell-r12. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 2A | The UE transmits an ULInformationTransfer message containing the ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |
| 3 | SS transmits RRCConnectionReconfiguration message containing a SCG-Configuration-r12 with release. RRCConnectionReconfiguration message contains the DEACTIVATE EPS BEARER CONTEXT REQUEST message | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 5 | The UE transmits an ULInformationTransfer message containing the DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |

8.2.2.9.2.3.3	Specific message contents

Table 8.2.2.9.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.9.2.3.2-1)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition PSCell_Add_Split_DRB

Table 8.2.2.9.2.3.3-2: RRCConnectionReconfiguration (step 3, Table 8.2.2.9.2.3.2-1)

Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition PSCell_Release

##### 8.2.2.9.3	RRC connection reconfiguration / SCG change without handover / SCG DRB to MCG DRB and SCG DRB modification

8.2.2.9.3.1	Test Purpose (TP)

(1)

With { UE in connected mode with SCG activated with one MCG DRB and two SCG DRBs established }

ensure that {

when { UE is receiving a RRCConnectionReconfiguration message including the IE scg-Configuration to change one of the SCG DRBs on the PSCell to a MCG DRB on the PCell }
  then { UE responds with a RRCConnectionReconfigurationComplete message }

}

(2)

With { UE in connected mode with SCG activated with one MCG DRB and two SCG DRBs established has performed a SCH changing one of the SCG DRBs on the PSCell to a MCG DRB on the PCell }

ensure that {

when { uplink data are submitted for transmission on the MCG/SCG DRBs }
  then { UE transmits the uplink data on the MCG/SCG DRBs respectively }

}

8.2.2.9.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.3.10.10 and 5.3.10.12. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

…

[TS 36.331, clause 5.3.10.10]

The UE shall:

1>	if the received scg-Configuration is set to release or includes the mobilityControlInfoSCG (i.e. SCG release/ change):

2>	if mobilityControlInfo is not received (i.e. SCG release/ change without HO):

3>	reset SCG MAC, if configured;

3>	for each drb-Identity value that is part of the current UE configuration:

4>	if the DRB indicated by drb-Identity is an SCG DRB:

5>	re-establish the PDCP entity and the SCG RLC entity or entities;

4>	if the DRB indicated by drb-Identity is a split DRB:

5>	perform PDCP data recovery and re-establish the SCG RLC entity;

4>	if the DRB indicated by drb-Identity is an MCG DRB; and

4>	drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to scg (i.e. MCG to SCG):

5>	re-establish the PDCP entity and the MCG RLC entity or entities;

3>	configure lower layers to consider the SCG SCell(s), except for the PSCell, to be in deactivated state;

…

NOTE 2:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

[TS 36.331, clause 5.3.10.12]

The UE shall:

1>	for each split or SCG DRBs that is part of the current configuration:

2>	if the corresponding drb-Identity value is included in the received drb-ToAddModList; and:

2>	if the corresponding drb-Identity value is not included in the received drb-ToAddModListSCG (i.e. reconfigure split, split to MCG or SCG to MCG):

3>	perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1;

8.2.2.9.3.3	Test description

8.2.2.9.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 10.

UE:

Preamble:

-	The UE is in state DC MCG/SCG DRB Loopback Activated (state 6A) on Cell 1 and Cell 10 according to [18] 
UE has 4 DRBs configured. DRB 1(Default bearer, not used in the test sequence) , DRB 2(dedicated bearer) as MCG DRB’s and DRB 3(dedicated bearer) as SCG#1 bearer, DRB 4 (dedicated bearer) as SCG#2 bearer.

8.2.2.9.3.3.2	Test procedure sequence

Table 8.2.2.9.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to change of one of the SCG DRBs to a MCG DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS transmits one IP packet to the UE on all the 3 DRB’s. | <-- | IP packet | - | - |
| 4 | Check: Does the UE loops back the IP packet on each of 3 DRB’s? | --> | IP packet | 2 | P |

8.2.2.9.3.3.3	Specific message contents

Table 8.2.2.9.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.9.3.3.2-1)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRB-Mod |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.9.3.3.3-2: RadioResourceConfigDedicated-DRB-Mod (Table 8.2.2.9.3.3.3-1)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB ::= SEQUENCE { |  |  |  |
| drb-ToAddModList | DRB-ToAddModList-RECONFIG-toMCG |  |  |
| } |  |  |  |

Table 8.2.2.9.3.3.3-3: DRB-ToAddModList-RECONFIG-toMCG (Table 8.2.2.9.3.3.3-2)

| Derivation Path: 36.508, Clause 4.6.3, Table 4.6.3-2A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddModList ::= SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 Entry |  |  |
| eps-BearerIdentity | Not present |  |  |
| drb-Identity | 7 | Same as the DRB ID configured for SCG DRB in preamble |  |
| pdcp-Config | Not present |  |  |
| rlc-Config | Not present |  |  |
| logicalChannelIdentity | Not present |  |  |
| logicalChannelConfig | Not present |  |  |
| drb-TypeChange-r12 | toMCG |  |  |
| } |  |  |  |

##### 8.2.2.9.4	Void

##### 8.2.2.9.5	Void

#### 8.2.2.10	eIMTA / RRC connection reconfiguration / Radio resource reconfiguration / Success

8.2.2.10.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message and successfully completes the RRC Connection Reconfiguration procedure and has data available for transmission }

then { UE use the DRB configured by network side and transfer the data}

}

8.2.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.3, 5.3.10.0 and 5.3.10.6.

The following represent an extraction of the requirements relevant to the test purpose.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	If the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.0]

The UE shall:

…

1>	if the received radioResourceConfigDedicated includes the physicalConfigDedicated:

2>	reconfigure the physical channel configuration as specified in 5.3.10.6.

…

[TS 36.331, clause 5.3.10.6]

The UE shall:

- reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

8.2.2.10.3	Test description

8.2.2.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1(TDD)

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.2.10.3.2	Test procedure sequence

Table 8.2.2.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits the PDCP data on the change subframe to the UE. | <-- | PDCP Data | - | - |
| 4 | Check: Does the UE loops back the PDCP data? | --> | PDCP Data | 1 | P |

8.2.2.10.3.3	Specific message contents

Table 8.2.2.10.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.10.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-eIMTA-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.10.3.3-2: RadioResourceConfigDedicated-eIMTA-DEFAULT (Table 8.2.2.10.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-eIMTA-DEFAULT ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- eIMTA |  |  |
| } |  |  |  |

Table 8.2.2.10.3.3-3: PhysicalConfigDedicated-eIMTA (Table 8.2.2.10.3.3-2)

Derivation Path: 36.508 table 4.8.2.1.6-1A ,condition eIMTA

#### 8.2.2.11	Short Processing Time / SRS configuration / Aperiodic

8.2.2.11.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when {UE receives an RRCConnectionReconfiguration message with SoundingRS-UL-AperiodicConfigDedicated configured for UL subframe as SRS trigger type 1, and a SRS request is triggered via DCI 1A }

then {UE performs SRS transmission using the resource on UL subframe configured in SoundingRS-UL-AperiodicConfigDedicated }

}

8.2.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.213 clause 8.2.

[TS 36.213, clause 8.2]A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15.

…

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in Subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	Otherwise

-	For trigger type 0, if SoundingRS-UL-ConfigDedicatedUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used.

-	For trigger type 1, if SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicatedAperiodic shall be used.

For trigger type 1 and DCI format 4/4A/4B/7-0B three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4/4A/4B/7-0B indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/0A/0B /6-0A/7-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1 and DCI formats 1A/2B/2C/2D/6-1A/7-1E/7-1F/7-1G, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. For a serving cell that is not a LAA SCell, the SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A/7-0A/7-1E/7-1F/7-1G, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. For a serving cell that is a LAA SCell, the SRS timing offset field is 3 bits [4] for DCI formats 1A/2B/2C/2D, with a type 1 SRS triggered if the value of the SRS timing offset field is not set to '000'. The SRS request field is 1 bit [4] for DCI formats 0A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. The 2-bit SRS request field [4] in DCI format 0B indicates the type 1 SRS triggering and PUSCH subframe (as determined in Subclause 8.0) with SRS as given in Table 8.2-0A.

For a serving cell that is not a LAA SCell, a 1-bit SRS request field shall be included in DCI formats 0/1A for frame structure type 1 and 0/1A/2B/2C/2D for frame structure type 2 if the UE is configured with SRS parameters for DCI formats 0/1A/2B/2C/2D by higher-layer signalling. A 1-bit SRS request field shall be included in DCI format 7-0A for frame structure type 2 if the UE is configured with SRS parameters srs-ConfigApDCI-Format0 and srs-DCI7-TriggeringConfig-r15 by higher-layer signalling. A 1-bit SRS request field shall be included in DCI formats 7-1E/7-1F/7-1G for frame structure type 2 if the UE is configured with SRS parameters srs-ConfigApDCI-Format1a2b2c and srs-DCI7-TriggeringConfig-r15 by higher-layer signalling. A 1-bit SRS request field shall be included in DCI formats 6-0A/6-1A, the value of which is reserved if the UE is not configured with SRS parameters for DCI formats 6-0A/6-1A by higher layer signalling.

Table 8.1-1: SRS request value for trigger type 1 in DCI format 4/4A/4B/7-0B

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1 SRS trigger |
| '01' | The 1st SRS parameter set configured by higher layers |
| '10' | The 2nd SRS parameter set configured by higher layers |
| '11' | The 3rd SRS parameter set configured by higher layers |

Table 8.2-0A: SRS request value for trigger type 1 in DCI format 0B

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1 SRS trigger |
| '01' | Type 1 SRS trigger and first scheduled PUSCH subframe |
| '10' | Type 1 SRS trigger and second scheduled PUSCH subframe |
| '11' | Type 1 SRS trigger and last scheduled PUSCH subframe |

Table 8.2-0C: SRS request value for trigger type 1 in DCI format 3B and for UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1 SRS trigger for a 1st set of serving cells configured by higher layers |
| '01' | Type 1 SRS trigger for a 2nd set of serving cells configured by higher layers |
| '10' | Type 1 SRS trigger for a 3rd set of serving cells configured by higher layers |
| '11' | Type 1 SRS trigger for a 4th set of serving cells configured by higher layers |

…

For a serving cell that is not a LAA SCell, a non-BL/CE UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n, slot 2n or slot 2n+1 of serving cell c shall commence SRS transmission in the first subframe satisfying , and

-	 if the positive SRS request in PDCCH/SPDCCH with DCI format 7-0A/7-1A is detected in slot 2n or slot 2n+1, for TDD

-	 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI with DCI format other than DCI format 7-0A/7-0B/7-1E/7-1F/7-1G is in the UE-specific search space,

-	otherwise, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with

where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3.

Table 8.2-3:  for TDD

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |
| in case UpPTS length of 2 symbols |  | 0 | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 |
| in case UpPTS length of 1 symbol |  | 1 |  | 2 | 3 | 4 |  | 6 |  | 7 | 8 | 9 |

8.2.2.11.3	Test description

8.2.2.11.3.1	Pre-test conditions

System Simulator:

- Cell 1

-	RRC Connection Reconfiguration (preamble: Table 4.5.3.3-1, step 8) using parameters as specified in Table 8.2.2.11.3.3-1

UE:

- None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.11.3.2	Test procedure sequence

Table 8.2.2.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup an aperiodic SRS configuration on the last symbol of 4th and 9th subframe of every radio frame. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an SRS request by using DCI format 1A on 1st subframe in one radio frame. | <-- | DCI: (DCI Format 1A defined in TS 36.508 Table 4.3.6.1.3-1 with following exceptions: RBA()= 5, = 5, SRS request = ‘1’.) | - | - |
| 4 | Check: Does the UE transmit SRS on the last symbol of the 4th subframe in the same radio frame? | --> | SRS | 1 | P |
| 5 | Check: Does the UE transmit SRS on the last symbol of the 9th subframe in the same radio frame? | --> | SRS | 1 | F |

8.2.2.11.3.3	Specific Message Contents

Table 8.2.2.11.3.3-1: PhysicalConfigDedicated-DEFAULT (step1, Table 8.2.2.11.3.2-1)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 ::= SEQUENCE { |  |  |  |
| shortProcessingTime-r15 | TRUE |  |  |
| } |  |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 SEQUENCE { |  | 1 entry |  |
| soundingRS-AperiodicSet-r14[1] SEQUENCE { |  |  |  |
| srs-CC-SetIndexList-r14 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r14 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 5 | 4th and 9th subframe every radio frame | FDD |
|  | 13 | 4th and 9th subframe every radio frame | TDD |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigApDCI-Format0-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| srs-ConfigApDCI-Format1a2b2c-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### 8.2.2.12	Short TTI / SRS configuration / TDD / Aperiodic

8.2.2.12.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when {UE receives an RRCConnectionReconfiguration message with srs-DCI7-TriggeringConfig-r15 and SoundingRS-UL-AperiodicConfigDedicated configured for UL slot as SRS trigger type 1, and a SRS request is triggered via DCI 7-0A on the TDD Cell }

then {UE performs SRS transmission on the TDD Cell using the resource on UL slot configured in SoundingRS-UL-AperiodicConfigDedicated }

}

8.2.2.12.2	Conformance requirements

Same conformance requirement as in clause 8.2.2.11.2.

8.2.2.12.3	Test description

8.2.2.12.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.12.3.2	Test procedure sequence

Table 8.2.2.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup an aperiodic SRS configuration on the last symbol of 3rd and 8th subframe of every radio frame. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an SRS request by using DCI format 7-0A on slot 0 of the 1st subframe in one radio frame. | <-- | DCI: (DCI Format 7-0A defined in TS 36.508 Table 4.3.6.1.3-13 with following exceptions: RBA()= 5, = 5, SRS request = ‘1’.) | - | - |
| 4 | Check: Does the UE transmit SRS on the last symbol of the 3rd subframe in the same radio frame? | --> | SRS | 1 | P |
| 5 | Check: Does the UE transmit SRS on the last symbol of the 8th subframe in the same radio frame? | --> | SRS | 1 | F |

8.2.2.12.3.3	Specific Message Contents

Table 8.2.2.12.3.3-1: PhysicalConfigDedicated-DEFAULT (step1, Table 8.2.2.12.3.2-1)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 ::= CHOICE{ |  |  |  |
| srs-DCI7-TriggeringConfig-r15 | TRUE |  |  |
| } |  |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 SEQUENCE { |  | 1 entry |  |
| soundingRS-AperiodicSet-r14[1] SEQUENCE { |  |  |  |
| srs-CC-SetIndexList-r14 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r14 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 12 | 3th and 8th subframe every radio frame |  |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigApDCI-Format0-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| srs-ConfigApDCI-Format1a2b2c-r10 SEQUENCE { |  |  |  |
| srs-AntennaPortAp-r10 | an1 |  |  |
| srs-BandwidthAp-r10 | bw0 |  |  |
| freqDomainPositionAp-r10 | 0 |  |  |
| transmissionCombAp-r10 | 0 |  |  |
| cyclicShiftAp-r10 | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.2.13	CA / RRC connection reconfiguration / SCell addition in dormant mode / Success

##### 8.2.2.13.1	CA / RRC connection reconfiguration / SCell addition in dormant mode / Success / Intra-band Contiguous CA

8.2.2.13.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a SCell addition and sCellState indicates dormant }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in dormant state and sends an RRCConnectionReconfigurationComplete message and report CQI/PMI/RI/PTI/CRI for the SCell according to the periodicity indicated by cqi-ReportPeriodicSCell-r15 }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and sCellToAddModList with SCell addition and sCellState indicates dormant }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in dormant state and sends an RRCConnectionReconfigurationComplete message and report CQI/PMI/RI/PTI/CRI for the SCell according to the periodicity indicated by cqi-ReportPeriodicSCell-r15 }

}

8.2.2.13.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.5.4, 5.3.10.3b and 11.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

…

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

…

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreak is configured.

1>	reset MCG MAC and SCG MAC, if configured;

1>	release uplinkDataCompression, if configured;

1>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the pdcp-reestablish flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

1>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

…

1>	apply the value of the newUE-Identity as the C-RNTI;

…

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

…

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

…

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

…

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

…

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

…

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	perform the measurement related actions as specified in 5.5.6.1;

…

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

…

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

…

2>	else if sCellState is configured for the SCell and indicates dormant:

3>	configure lower layers to consider the SCell to be in dormant state;

…

[TS 36.331, clause 11.2]

The UE performance requirements for RRC procedures are specified in the following table, by means of a value N:

N = the number of 1ms subframes from the end of reception of the E-UTRAN -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> E-UTRAN response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation) where N=20 (Ref. TS 36.331 Fig. 11.2-1).

8.2.2.13.1.3	Test Description

8.2.2.13.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell3 is the SCell to be added.

-	Cell 3 is active SCell according to [18] cl. 6.3.4.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.13.1.3.2	Test procedure sequence

Table 8.2.2.13.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.2.13.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that camping on Cell 1 is guarantee. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | Power levels are such that entry condition for event A3 (measId 1) is satisfied: |

Table 8.2.2.13.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 addition and sCellState indicates dormant. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message to confirm SCell addition of Cell 3 in dormant mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does UE report CQI/PMI/RI/PTI/CRI on Cell 3 for the next 400 ms? | - | (CSI) | 1 | P |
| 4 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup event A3 reporting configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6 | The SS changes Cell 3 parameters according to the row "T1" in table 8.2.4.17.1.3.2-1. | - | - | - | - |
| 7 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 8 | The SS transmits an RRCConnectionReconfiguration message with mobilityControlInfo on Cell 1 to order the UE to perform handover to Cell 3 and sCellToAddModList with SCell Cell 1 addition and sCellState indicates dormant. | <-- | RRCConnectionReconfiguration | - | - |
| 9 | UE transmits an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the handover and SCell addition of Cell 1 in dormant mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10 | Check: Does UE report CQI/PMI/RI/PTI/CRI on Cell 1 for next 400 ms? | - | (CSI) | 2 | P |

8.2.2.13.1.3.3	Specific message contents

Table 8.2.2.13.1.3.3-1: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.13.1.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.2.13.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DormantSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModListExt-v1430 | SCellToAddModListExt-v1430-DormantSCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-3: RadioResourceConfigDedicated-DormantSCell (Table 8.2.2.13.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-4: PhysicalConfigDedicated-DormantSCell (Table 8.2.2.13.1.3.3-3)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r15 | CQI-ReportConfigSCell-r15-DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-5: CQI-ReportConfigSCell-r15-DormantSCell (Table 8.2.2.13.1.3.3-4)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r15-DormantSCell ::= SEQUENCE { |  |  |  |
| cqi-ReportPeriodicSCell-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-pmi-ConfigIndexDormant-r15 | 37 |  | FDD |
| cqi-pmi-ConfigIndexDormant-r15 | 36 |  | TDD |
| ri-ConfigIndexDormant-r15 | 805 |  |  |
| csi-SubframePatternDormant-r15 | Not present |  |  |
| cqi-FormatIndicatorDormant-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-6: SCellToAddMod-r10-f2-Add (Table 8.2.2.13.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.13.1.3.3-7: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.13.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-8: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.13.1.3.3-6)

Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA

Table 8.2.2.13.1.3.3-9: SCellToAddModListExt-v1430-Dormant (Table 8.2.2.13.1.3.3-2)

| Derivation Path: 36.331 clause |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddModListExt-v1430 ::= SEQUENCE (SIZE (1..maxSCell-r13)) OF SEQUENCE { | 1 entry |  |  |
| sCellState-r15 | dormant |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-10: RRCConnectionReconfiguration (step 4, Table 8.2.2.13.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.2.13.1.3.3-11: MeasConfig (Table 8.2.2.13.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-12: MeasurementReport (step 7, Table 8.2.2.13.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | physicalCellId of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | Cell 3 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-13: RRCConnectionReconfiguration (step 8, Table 8.2.2.13.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DormantSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f1-Add | SCell addition for Cell 1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModListExt-v1430 | SCellToAddModListExt-v1430-DormantSCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-14: MobilityControlInfo (Table 8.2.2.13.1.3.3-13)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-15: RadioResourceConfigDedicated-DormantSCell (Table 8.2.2.13.1.3.3-13)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-16: PhysicalConfigDedicated-DormantSCell (Table 8.2.2.13.1.3.3-15)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r15 | CQI-ReportConfigSCell-r15-DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-17: CQI-ReportConfigSCell-r15-DormantSCell (Table 8.2.2.13.1.3.3-16)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r15-DormantSCell ::= SEQUENCE { |  |  |  |
| cqi-ReportPeriodicSCell-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-pmi-ConfigIndexDormant-r15 | 37 |  | FDD |
| cqi-pmi-ConfigIndexDormant-r15 | 36 |  | TDD |
| ri-ConfigIndexDormant-r15 | 805 |  |  |
| csi-SubframePatternDormant-r15 | Not present |  |  |
| cqi-FormatIndicatorDormant-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.13.1.3.3-18: SCellToAddMod-r10-f1-Add (Table 8.2.2.13.1.3.3-13)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f1 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 1 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.13.1.3.3-19: RadioResourceConfigCommonSCell-r10-f1 (Table 8.2.2.13.1.3.3-18)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.2.14	CA / RRC connection reconfiguration / SCell addition in activated mode / Success

##### 8.2.2.14.1	CA / RRC connection reconfiguration / SCell addition in activated mode / Success / Intra-band Contiguous CA

8.2.2.14.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a SCell addition and sCellState indicates activated }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in activated state and sends an RRCConnectionReconfigurationComplete message and starts monitoring PDCCH on added SCell }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and sCellToAddModList with SCell addition and sCellState indicates activated }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in activated state and sends an RRCConnectionReconfigurationComplete message and starts monitoring PDCCH on added SCell }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and sCellState indicates activated }

then { UE maintains the current SCell in activated state and sends an RRCConnectionReconfigurationComplete message and keeps monitoring PDCCH on SCell }

}

8.2.2.14.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.5.4 and  5.3.10.3b.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

…

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

…

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreak is configured.

1>	reset MCG MAC and SCG MAC, if configured;

1>	release uplinkDataCompression, if configured;

1>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the pdcp-reestablish flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

1>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

…

1>	apply the value of the newUE-Identity as the C-RNTI;

…

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

…

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

…

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

…

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

…

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

…

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	perform the measurement related actions as specified in 5.5.6.1;

…

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

…

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

…

2>	if sCellState is configured for the SCell and indicates activated:

3>	configure lower layers to consider the SCell to be in activated state;

…

8.2.2.14.1.3	Test Description

8.2.2.14.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell3 is the SCell to be added and Cell 6 is .neighbouring PCell

-	Cell 3 is active SCell according to [18] cl. 6.3.4.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) on Cell 1 according to [18].

8.2.2.14.1.3.2	Test procedure sequence

Table 8.2.2.14.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.2.14.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -97 | The power level values are such that camping on Cell 1 is guarantee. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | -97 | The power level values are such that camping on Cell 3 is guarantee. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -91 | -97 | -85 | The power level values are such that camping on Cell 6 is guarantee. |

Table 8.2.2.14.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 addition and sCellState indicates activated. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message to confirm SCell addition of Cell 3 in activated mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | 100 ms after step 3, the SS indicates a new transmission on PDCCH of CC2 and transmits a MAC PDU (containing an RLC PDU ) | <-- | MAC PDU (CC2) | - | - |
| 4 | Check: Does the UE transmit a Scheduling Request on PUCCH | --> | (SR) | 1 | P |
| 5 | The SS sends an UL grant suitable for transmitting loop back PDU on Cell 1 | <-- | (UL Grant) | - | - |
| 6 | The UE transmit a MAC PDU containing the loop back PDU corresponding to step 3 | --> | MAC PDU | - | - |
| 7 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 6 | <-- | MAC PDU (CC1) |  |  |
| 8 | The SS changes Cell1, Cell 3 and Cell 6 parameters according to the row "T1" in table 8.2.2.14.1.3.2-1. | - | - | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message with mobilityControlInfo on Cell 1 to order the UE to perform handover to Cell 3 and sCellToAddModList with SCell Cell 1 addition and sCellState indicates activated. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | UE transmits an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the handover and SCell addition of Cell 1 in activated mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | 100 ms after step 13, the SS indicates a new transmission on PDCCH of CC2 and transmits a MAC PDU (containing an RLC PDU ) | <-- | MAC PDU (CC2) | - | - |
| 12 | Check: Does the UE transmit a Scheduling Request on PUCCH | --> | (SR) | 2 | P |
| 13 | The SS sends an UL grant suitable for transmitting loop back PDU on Cell 3 | <-- | (UL Grant) | - | - |
| 14 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 11 | --> | MAC PDU | - | - |
| 15 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 14 | <-- | MAC PDU (CC1) |  |  |
| 16 | The SS changes Cell1, Cell 3 and Cell 6 parameters according to the row "T2" in table 8.2.2.14.1.3.2-1. | - | - | - | - |
| 17 | The SS transmits an RRCConnectionReconfiguration message with mobilityControlInfo on Cell 3 to order the UE to perform handover to Cell 6 and maintaining SCell Cell 1 and sCellState indicates activated. | <-- | RRCConnectionReconfiguration | - | - |
| 18 | UE transmits an RRCConnectionReconfigurationComplete message on Cell 6 to confirm the successful completion of the handover and maintaining SCell Cell 1 in activated mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 19 | 100 ms after step 25, the SS indicates a new transmission on PDCCH of CC2 and transmits a MAC PDU (containing an RLC PDU ) | <-- | MAC PDU (CC2) | - | - |
| 20 | Check: Does the UE transmit a Scheduling Request on PUCCH | --> | (SR) | 3 | P |
| 21 | The SS sends an UL grant suitable for transmitting loop back PDU on Cell 6 | <-- | (UL Grant) | - | - |
| 22 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 19 | --> | MAC PDU | - | - |
| 23 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 22 | <-- | MAC PDU (CC1) |  |  |

8.2.2.14.1.3.3	Specific message contents

Table 8.2.2.14.1.3.3-1: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.14.1.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.2.14.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-ActivatedSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModListExt-v1430 | SCellToAddModListExt-v1430-ActivatedSCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-3: RadioResourceConfigDedicated-ActivatedSCell (Table 8.2.2.14.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-ActivatedSCell ::= SEQUENCE { |  |  |  |
| mac-MainConfig | MAC-MainConfig-ActivatedSCell |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-4: MAC-MainConfig-ActivatedSCell (Table 8.2.2.14.1.3.3-3)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.5-1 MAC-MainConfig-RBC condition DRX_L |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig ActivatedSCell::= SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| mac-MainConfig-v1020 SEQUENCE { |  |  |  |
| sCellDeactivationTimer-r10 | rf32 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-5: SCellToAddMod-r10-f2-Add (Table 8.2.2.14.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.14.1.3.3-6: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.14.1.3.3-5)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-7: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.14.1.3.3-5)

Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA

Table 8.2.2.14.1.3.3-8: SCellToAddModListExt-v1430-ActivatedSCell (Table 8.2.2.14.1.3.3-2)

| Derivation Path: 36.331 clause |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddModListExt-v1430-ActivatedSCell ::= SEQUENCE (SIZE (1..maxSCell-r13)) OF SEQUENCE { | 1 entry |  |  |
| sCellState-r15 | activated |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-9: RRCConnectionReconfiguration (step 9, Table 8.2.2.14.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-ActivatedSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f1-Add | SCell addition for Cell 1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModListExt-v1430 | SCellToAddModListExt-v1430-ActivatedSCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-10: MobilityControlInfo (Table 8.2.2.14.1.3.3-9)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-11: RadioResourceConfigDedicated-ActivatedSCell (Table 8.2.2.14.1.3.3-9)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| mac-MainConfig | MAC-MainConfig-ActivatedSCell |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-12: MAC-MainConfig-ActivatedSCell (Table 8.2.2.14.1.3.3-11)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.5-1 MAC-MainConfig-RBC condition DRX_L |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-ActivatedSCell ::= SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated | Infinity |  |  |
| mac-MainConfig-v1020 SEQUENCE { |  |  |  |
| sCellDeactivationTimer-r10 | rf32 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-13: SCellToAddMod-r10-f1-Add (Table 8.2.2.14.1.3.3-9)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f1 |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f1 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 1 |  | Band > 64 |
| } |  |  |  |

Table 8.2.2.14.1.3.3-14: RadioResourceConfigCommonSCell-r10-f1 (Table 8.2.2.14.1.3.3-13)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-15: RadioResourceConfigDedicatedSCell-r10-f1 (Table 8.2.2.14.1.3.3-13)

Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA

Table 8.2.2.14.1.3.3-16: SCellToAddModListExt-v1430-ActivatedSCell (Table 8.2.2.14.1.3.3-9)

| Derivation Path: 36.331 clause |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddModListExt-v1430-ActivatedSCell ::= SEQUENCE (SIZE (1..maxSCell-r13)) OF SEQUENCE { | 1 entry |  |  |
| sCellState-r15 | activated |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-17: RRCConnectionReconfiguration (step 17, Table 8.2.2.14.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToAddModListExt-v1430 | SCellToAddModListExt-v1430-ActivatedSCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-18: MobilityControlInfo (Table 8.2.2.14.1.3.3-17)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.14.1.3.3-19: SCellToAddModListExt-v1430-ActivatedSCell (Table 8.2.2.14.1.3.3-17)

| Derivation Path: 36.331 clause |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddModListExt-v1430-ActivatedSCell ::= SEQUENCE (SIZE (1..maxSCell-r13)) OF SEQUENCE { | 1 entry |  |  |
| sCellState-r15 | activated |  |  |
| } |  |  |  |

### 8.2.3	Radio bearer release

#### 8.2.3.1	RRC connection reconfiguration / Radio bearer release / Success

8.2.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a drb-ToReleaseList }

then { for each drb-Identity release the PDCP entity and RLC entity and DTCH logical channel

and	indicate release of the DRB(s) to upper layers }

}

8.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.5.3 and 5.3.10.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

- if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

…

1>	else:

2>	If the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.2]

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration (DRB release):

2>	release the PDCP entity;

2>	release the RLC entity;

2>	release the DTCH logical channel;

1>	indicate the release of the DRB(s) and the eps-BearerIdentity of the released DRB(s) to upper layers;

8.2.3.1.3	Test description

8.2.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

- 	The condition SRB2-DRB(2, 0) is used for step 8 in 4.5.3.3 according to [18].

8.2.3.1.3.2	Test procedure sequence

Table 8.2.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message with a drb-ToReleaseList. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 2a | The UE transmits an ULInformationTransfer message. | --> | ULInformationTransfer | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.2.3.1.3.3	Specific message contents

Table 8.2.3.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.3.1.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| dedicatedInfoNASList SEQUENCE (SIZE(1..maxDRB)) OF | 1 entry |  |  |
| DedicatedInfoNAS[1] | DEACTIVATE EPS BEARER CONTEXT REQUEST with EPS bearer identity corresponding to drb-Identity 2 and ESM cause #36 | According 36.508 table 4.7.3-12 and regular deactivation of a second DRB |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRB-RELEASE | Table 8.2.3.1.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.1.3.3-2: RadioResourceConfigDedicated-DRB-RELEASE (Table 8.2.3.1.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF | 1 entry |  |  |
| drb-Identity[1] | 2 |  |  |
| mac-MainConfig CHOICE { } | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |

### 8.2.4	Handover

#### 8.2.4.1	RRC connection reconfiguration / Handover / Success / Dedicated preamble

8.2.4.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement}

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo with a rach-ConfigDedicated }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a nextHopChainingCount which is different from the NCC associated with the currently active KeNB }

then { UE derives new KeNB key from the nextHopChainingCount }

}

(3)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a nextHopChainingCount which is same as the NCC associated with the currently active KeNB }

then { UE derives new KeNB key from the currently active KeNB }

}

8.2.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target cell by acquiring system information from that cell before performing RACH access in the target cell.

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.1.3	Test description

8.2.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 4 and Cell 11.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.1.3.2	Test procedure sequence

Table 8.2.4.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | ”Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy exit condition for event A3 (M4 < M1)(NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | ”Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1)(NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | "Off" | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M1 > M4)(NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | -79 | The power level value are such that measurement results for Cell 1 (M1) and Cell 11 (M11) satisfy entry condition for event A3 (M11 > M1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.2.4.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T1" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 4. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the currently active KeNB? | --> | RRCConnectionReconfigurationComplete | 1,3 | P |
| - | EXCEPTION: Steps 6a1 to 6a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 6a1 | IF pc_intraFreq-CE-NeedForGaps THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 4 | <- | RRCConnectionReconfiguration | - | - |
| 6a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 4 | -> | RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T2" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 8 | The UE transmits a MeasurementReport message to report event A3 on Cell 4 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the nextHopChainingCount? | --> | RRCConnectionReconfigurationComplete | 1,2 | P |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 10a1 | IF pc_intraFreq-CE-NeedForGaps THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 1 | <- | RRCConnectionReconfiguration | - | - |
| 10a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 1 | -> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell 1 and Cell 11 parameters and switches Cell 4 off according to the row "T3" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 12 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP,RSRQ value for Cell 11. | --> | MeasurementReport | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 11. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 11 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the nextHopChainingCount? | --> | RRCConnectionReconfigurationComplete | 1,2 | P |
| 14A | The UE transmit a TRACKING AREA UPDATE REQUEST message on Cell 11. | - | - | - | - |
| 14B | - SS responds with a TRACKING AREA UPDATE ACCEPT message. | - | - - | - | - |
| 14C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 11? | - | - | 1 | - |

8.2.4.1.3.3	Specific message contents

Table 8.2.4.1.3.3-0: Conditions for specific message contents
in Table 8.2.4.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.1.3.3-2: MeasConfig (Table 8.2.4.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.1.3.3-3: MeasurementReport (step 4, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 4 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-5: MobilityControlInfo (Table 8.2.4.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 4 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-6: MeasurementReport (step 8, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-7: RRCConnectionReconfiguration (step 9, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| securityConfigHO | SecurityConfigHO |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-8: MobilityControlInfo (Table 8.2.4.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  | - |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-9: SecurityConfigHO (step 9, Table 8.2.4.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-10: MeasurementReport (step 12, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measuredResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 11 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-11: RRCConnectionReconfiguration (step 13, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| securityConfigHO | SecurityConfigHO |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 11 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-12: MobilityControlInfo (Table 8.2.4.1.3.3-11)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 11 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.2.4.1.3.3-13: SecurityConfigHO (Table 8.2.4.1.3.3-11)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-14: RRCConnectionReconfiguration (step 6a1, step 10a1 Table 8.2.4.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.1.3.3-14: MeasConfig (Table 8.2.4.1.3.3-14)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig-CE |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-15: RadioResourceConfigDedicated-HO (Table 8.2.4.1.3.3-4), Table 8.2.4.1.3.3-7 , Table 8.2.4.1.3.3-11

| Derivation Path: 36.508 Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-16: PhysicalConfigDedicated-DEFAULT(Table 8.2.4.1.3.3-15)

| Derivation Path: 36.508 Table 4.6.3-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| epdcch-Config-r11 | EPDCCH-Config-r11-DEFAULT with condition (CEmodeA OR CEmodeB) |  |  |
| } |  |  |  |

#### 8.2.4.2	RRC connection reconfiguration / Handover / Success / Common preamble

8.2.4.2.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo without a rach-ConfigDedicated }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.2.3	Test description

8.2.4.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.2.3.2	Test procedure sequence

Table 8.2.4.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 using common preamble to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.2.4.2.3.3	Specific message contents

Table 8.2.4.2.3.3-0: Conditions for specific message contents
in Table 8.2.4.2.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.2.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.2.3.3-2: MeasConfig (Table 8.2.4.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.2.3.3-3 MeasurementReport (step 4, Table 8.2.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.2.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 2 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.2.3.3-5: MobilityControlInfo (Table 8.2.4.2.3.3-4)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

#### 8.2.4.3	RRC connection reconfiguration / Handover / Success / Intra-cell / Security reconfiguration

8.2.4.3.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a securityConfigHO }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the eutra-CarrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity; as the C-RNTI

1>	 configure lower layers in accordance with the received radioResourceConfigCommon;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the configuration that do not require the UE to know the SFN of the target cell;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

2>	the procedure ends;

...

8.2.4.3.3	Test description

8.2.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.3.3.2	Test procedure sequence

Table 8.2.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | DLInformationTransfer | - | - |
| 2 | The UE transmits an AUTHENTICATION RESPONSE message and re-establishes mutual authentication. | --> | ULInformationTransfer | - | - |
| 3 | The SS transmits a NAS SECURITY MODE COMMAND message to reactivate NAS security. | <-- | DLInformationTransfer | - | - |
| 4 | The UE transmits a NAS SECURITY MODE COMPLETE message and re-establishes the security configuration. | --> | ULInformationTransfer | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message to perform intra cell handover and security reconfiguration. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message using the security key indicated by the keyChangeIndicator and nextHopChainingCount, as well as the indicated algorithms, to confirm the successful completion of the intra cell handover and security reconfiguration? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state? | - | - | 1 | - |

8.2.4.3.3.3	Specific message contents

Table 8.2.4.3.3.3-1: RRCConnectionReconfiguration (step 5, Table 8.2.4.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| securityConfigHO | SecurityConfigHO |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.3.3.3-2: MobilityControlInfo (Table 8.2.4.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.2.4.3.3.3-3: SecurityConfigHO (Table 8.2.4.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| keyChangeIndicator | TRUE |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.4	RRC connection reconfiguration / Handover / Failure / Intra-cell / Security reconfiguration

8.2.4.4.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a SecurityConfigHO }

ensure that {
  when { UE detects handover failure and the initial cell is selectable }

then { UE performs an RRC connection re-establishment procedure and remains in the E-UTRA RRC_CONNECTED state }

}

8.2.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.3.5.4, 5.3.5.6, 5.3.7.4 and 5.3.7.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC Connection Re-establishment procedure:

2>	re-establish PDCP for SRB2 and for all DRBs that are established, if any;

2>	re-establish RLC for SRB2 and for all DRBs that are established, if any;

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

2> resume SRB2 and all DRBs that are suspended, if any;

NOTE 1:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

…

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

…

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

2>	else:

3>	set the reestablishmentCause to the value 'otherFailure';

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	stop timer T301;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1> submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

8.2.4.4.3	Test description

8.2.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.2.4.4.3.2	Test procedure sequence

Table 8.2.4.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to perform intra cell handover and security reconfiguration. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 2 the steps specified in Table 8.2.4.4.3.2-2 should take place. | - | - | - | - |
| 2 | Wait for 1 s to ensure that T304 expires. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and reactivate security. | <-- | RRCConnectionReestablishment | - | - |
| 5 | The UE transmits an RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message to resume the existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state? | - | - | 1 | - |

Table 8.2.4.4.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra cell handover using MAC Random Access Preamble. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.4.3.3	Specific message contents

Table 8.2.4.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.4.3.3-2: MobilityControlInfo (Table 8.2.4.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.2.4.4.3.3-3: SecurityConfigHO (Table 8.2.4.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.4.3.3-4: RRCConnectionReestablishmentRequest (step 3, Table 8.2.4.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.4.3.3-5: RRCConnectionReconfiguration (step 6, Table 8.2.4.4.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.5	RRC connection reconfiguration / Handover / All parameters included

8.2.4.5.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo is provided with all parameters included }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.5.3	Test description

8.2.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.5.3.2	Test procedure sequence

Table 8.2.4.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of intra frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.5.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP and RSRQ values for Cell 2. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.2.4.5.3.3	Specific message contents

Table 8.2.4.5.3.3-0: Conditions for specific message contents
in Tables 8.2.4.5.3.3-2 and 8.2.4.5.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.5.3.3-2: MeasConfig (Table 8.2.4.5.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.5.3.3-3: MeasurementReport (step 4, Table 8.2.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.5.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 2 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.5.3.3-5: MobilityControlInfo (Table 8.2.4.5.3.3-4)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Same uplink EARFCN as used for Cell 2 |  | FDD |
| ul-CarrierFreq | Not present |  | TDD |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq-v9e0 | Same uplink EARFCN as used for Cell 2 |  | FDD |
| ul-CarrierFreq-v9e0 | Not present |  | TDD |
| } |  |  |  |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 2 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 2 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | Same additionalSpectrumEmission as used for Cell 2 |  |  |
| t304 | ms1000 |  |  |
| newUE-Identity | SS arbitrarily selects a value between ‘003C’H and ‘FFF2’H. |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| prach-Config | PRACH-Config-DEFAULT |  |  |
| pdsch-ConfigCommon | PDSCH-ConfigCommon-DEFAULT |  |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-DEFAULT |  |  |
| phich-Config | PHICH-Config-DEFAULT |  |  |
| pucch-ConfigCommon | PUCCH-ConfigCommon-DEFAULT |  |  |
| soundingRS-UL-ConfigCommon | SoundingRsUl-ConfigCommon-DEFAULT |  |  |
| uplinkPowerControlCommon | UplinkPowerControlCommon-DEFAULT |  |  |
| Prach-Config-v1310 | PRACH-Config-v1310-DEFAULT |  | CEmodeA or CEmodeB |
| pdsch_ConfigCommon_v1310 | PDSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA or CEmodeB |
| pucch_ConfigCommon_v1310 | PUCCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA or CEmodeB |
| pusch-ConfigCommon-v1310 | PUSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA or CEmodeB |
| antennaInfoCommon SEQUENCE { |  |  |  |
| antennaPortsCount | an1 |  |  |
| } |  |  |  |
| p-Max | Not present |  |  |
| tdd-Config | Not present |  | FDD |
|  | TDD-Config-DEFAULT |  | TDD |
| ul-CyclicPrefixLength | len1 |  |  |
| } |  |  |  |
| rach-ConfigDedicated | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### 8.2.4.6	RRC connection reconfiguration / Handover / Success / Inter-frequency

8.2.4.6.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the inter frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA frequency}

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.6.3	Test description

8.2.4.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.6.3.2	Test procedure sequence

Table 8.2.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.6.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 3? | - | - | 1 | - |

8.2.4.6.3.3	Specific message contents

Table 8.2.4.6.3.3-0: Conditions for specific message contents
in Tables 8.2.4.6.3.3-2 and 8.2.4.6.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.6.3.3-2: MeasConfig (Table 8.2.4.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.6.3.3-3: MeasurementReport (step 4, Table 8.2.4.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.6.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.6.3.3-5: MobilityControlInfo (Table 8.2.4.6.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.7	RRC connection reconfiguration / Handover / Failure / Re-establishment successful

8.2.4.7.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA cell having attempted intra frequency handover }

ensure that {
  when { UE detects handover failure and the initial cell is selectable }

then { UE performs an RRC connection re-establishment procedure and remains in the E-UTRA RRC_CONNECTED state }

}

(2)

with { UE having transmitted an RRCConnectionReestablishmentRequest message }

ensure that {
  when { UE receives an RRCConnectionReestablishment message with a nextHopChainingCount which is different from the NCC associated with the currently active KeNB }

then { UE derives new KeNB from the nextHopChainingCount }

}

(3)

with { UE having transmitted an RRCConnectionReestablishmentRequest message }

ensure that {
  when { UE receives an RRCConnectionReestablishment message with a nextHopChainingCount which is same as the NCC associated with the currently active KeNB }

then { UE derives new KeNB from the currently active KeNB }

}

8.2.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.4, 5.3.7.5, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from  E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	stop timer T301;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1> submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.7.3	Test description

8.2.4.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 4 and Cell 11.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.7.3.2	Test procedure sequence

Table 8.2.4.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3", "T4", "T5" and "T6" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -91 | The power level values are such that measurement results for Cell 1 (M1) Cell 4 (M4) and Cell 11(M11) satisfy exit condition for event A3 (M4 < M1 and M11 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | "Off" | The power level values are assigned to satisfy SrxlevCell 1 < 0 and SrxlevCell 11 < 0 such that selecting Cell 4 is guaranteed (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -73 | -79 | "Off" | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M1> M4). (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -73 | "Off" | "Off" | The power level values are assigned to satisfy SrxlevCell 4 < 0 and SrxlevCell 11 < 0 such that selecting Cell 1 is guaranteed. (NOTE 1). |
| T5 | Cell-specific RS EPRE | dBm/15kHz | -73 | "Off" | -67 | The power level values are such that measurement results for Cell 1 (M1) and Cell 11 (M11) satisfy entry condition for event A3 (M11> M1). (NOTE 1). |
| T6 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -67 | The power level values are assigned to satisfy SrxlevCell 1 < 0 and SrxlevCell 4 < 0 such that selecting Cell 11 is guaranteed. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.2.4.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T1" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 4. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.7.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T2" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 4? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 8 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 4. | <-- | RRCConnectionReestablishment | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the currently active KeNB on Cell 4? | --> | RRCConnectionReestablishmentComplete | 3 | P |
| 10 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 4. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 12 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T3" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 13 | The UE transmits a MeasurementReport message on Cell 4 to report event A3 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message on Cell 4 to order the UE to perform intra frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 15 the steps specified in Table 8.2.4.7.3.2-4 should take place. | - | - | - | - |
| 15 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T4" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 1? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 17 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 18 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the nextHopChainingCount on Cell 1? | --> | RRCConnectionReestablishmentComplete | 2 | P |
| 19 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 20 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 21 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T5" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 22 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 11. | --> | MeasurementReport | - | - |
| 23 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 11. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 24 the steps specified in Table 8.2.4.7.3.2-5 should take place. | - | - | - | - |
| 24 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T6" in table 8.2.4.7.3.2-1. | - | - | - | - |
| 25 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 11? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 26 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 11. | <-- | RRCConnectionReestablishment | - | - |
| 27 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the nextHopChainingCount on Cell 11? | --> | RRCConnectionReestablishmentComplete | 2 | P |
| 28 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 11. | <-- | RRCConnectionReconfiguration | - | - |
| 29 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 11. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 29A | The UE transmit a TRACKING AREA UPDATE REQUEST message on Cell 11. | - | - | - | - |
| 29B | SS responds with TRACKING AREA UPDATE ACCEPT message. | - | - | - | - |
| 29C | The UE transmits a TRACKING AREA UPDATE COMPLETE. | - | - | - | - |
| 30 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 11? | - | - | 1 | - |

Table 8.2.4.7.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 4. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

Table 8.2.4.7.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 1. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

Table 8.2.4.7.3.2-5: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 11. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.7.3.3	Specific message contents

Table 8.2.4.7.3.3-0: Conditions for specific message contents
in Table 8.2.4.7.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.7.3.3-0: SystemInformationBlockType2 for Cell 1, cell 4 and Cell 11 (preamble and all the steps in Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.7.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.7.3.3-2: MeasConfig (Table 8.2.4.7.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.7.3.3-3: MeasurementReport (step 4, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 4 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-5: MobilityControlInfo (Table 8.2.4.7.3.3-4)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 4 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-7: RRCConnectionReestablishment (step 8, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-8: RRCConnectionReconfiguration (step 28, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-8A: RRCConnectionReconfiguration (step 10, step 19 Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig-Gap |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-8B: MeasConfig-Gap (Table 8.2.4.7.3.3-8A)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-Gap ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig-CE |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-9: MeasurementReport (step 13, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-10: RRCConnectionReconfiguration (step 14, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-11: MobilityControlInfo (Table 8.2.4.7.3.3-10)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-12: RRCConnectionReestablishmentRequest (step 16, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 4 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-13: RRCConnectionReestablishment (step 17, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-14: MeasurementReport (step 22, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 11 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-15: RRCConnectionReconfiguration (step 23, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 11 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-16: MobilityControlInfo (Table 8.2.4.7.3.3-15)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 11 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-17: RRCConnectionReestablishmentRequest (step 25, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.7.3.3-18: RRCConnectionReestablishment (step 26, Table 8.2.4.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.8	RRC connection reconfiguration / Handover / Failure / Re-establishment failure

8.2.4.8.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including an IE mobilityControlInformation indicating a different E-UTRA cell having attempted intra frequency handover }

ensure that {

when { UE detects handover failure and fails an RRC connection re-establishment procedure }

then { UE enters the E-UTRA RRC_IDLE state and trigger TAU procedure in order to recover RRC connection }

}

8.2.4.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.3, 5.3.7.4, 5.3.7.7 and 5.3.12.

[TS 36.331, clause 5.3.5.4]

...

If the RRCConnectionReconfiguration message includes the mobilityControlInformation and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInformation;

1>	request PDCP to initiate the PDCP Re-establishment procedure for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the L2 re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in [8].

1>	reset MAC and re-establish RLC for all RBs that are established;

1>	If the RRCConnectionReconfiguration message includes the radioResourceConfiguration:

2>	perform the Radio resource configuration procedure as specified in 5.3.10;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	if the eutra-CarrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the eutra-CarrierFreq with a physical cell identity indicated by the targetCellIdentity;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetCellIdentity;

1>	if the dl-Bandwidth is included:

2>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	else:

2>	for the target cell, apply the same downlink bandwidth as for the current cell;

1>	if the ul-Bandwidth is included:

2>	for the target cell, apply the uplink bandwidth indicated by the ul-Bandwidth;

1>	else:

2>	for the target cell, apply the same uplink bandwidth as for the current cell;

1>	 configure lower layers in accordance with the received radioResourceConfigCommon;

1>	If the RRCConnectionReconfiguration message includes the securityConfiguration:

2>	apply the AS-derived keys associated with the AS-base key indicated by the keyIndicator;

2>	configure lower layers to apply the indicated integrity protection algorithm, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE in the target cell, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the indicated ciphering algorithm, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE in the target cell, including the message used to indicate the successful completion of the procedure;

1>	If the RRCConnectionReconfiguration message includes the measurementConfiguration:

2>	perform the Measurement configuration procedure as specified in 5.5.2;

1>	synchronise to the DL of the target cell;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	If MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	If the physicalConfigDedicated is included in the RRCConnectionReconfiguration message:

3>	If the UE needs the SFN of the target cell to apply the PUCCH and Sounding RS configuration:

4>	apply the new PUCCH and Sounding RS configuration upon acquiring the SFN of the target cell;

3>	else:

4>	apply the new PUCCH and Sounding RS configuration;

2> indicate to PDCP to complete the PDCP Re-establishment procedure for all DRBs that are established, if any;

2>	the procedure ends.

...

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	If T304 expires (handover failure):

NOTE 1:	Following T304 expiry dedicated preambles, if provided within the rach-ConfigDedicated, are not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the physical layer configuration;

NOTE 2:	The UE reverts to the RRC configuration as well as the layer 2 configuration (PDCP/RLC/MAC) used in the source cell.

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends.

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

...

1>	start timer T311;

1>	request PDCP to initiate the PDCP Re-establishment procedure for all RBs that are established;

NOTE 1:	The handling of the radio bearers after the successful completion of the L2 re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in [8].

1>	reset MAC and re-establish RLC for all RBs that are established;

1>	select a suitable cell in accordance with the cell selection process as specified in [4];

[TS 36.331, clause 5.3.7.3]

...

2>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

...

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the IE ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover failure case) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the cellIdentity to the Physical layer identity of the source cell (handover failure case) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the concatenation of the ASN.1 encoded CellIdentity of the current cell, PhysicalCellIdentity of the cell the UE was connected to prior to the failure and C-RNTI that the UE had in the cell it was connected to prior to the failure;

3>	with the integrity protection key and integrity protection algorithm that was used in the cell the UE was connected to prior to the failure; and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones.

1>	set the IE reestablishmentCause as follows:

...

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value ‘handoverFailure’;

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.7]

Upon T311 expiry, the UE shall:

1>	perform the actions upon moving from RRC_CONNECTED to RRC_IDLE as specified in 5.3.12.

...

[TS 36.331, clause 5.3.12]

Upon moving from RRC_CONNECTED to RRC_IDLE, the UE shall:

1>	reset MAC and re-establish RLC for all RBs that are established;

...

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers;

1>	enter RRC_IDLE.

8.2.4.8.3	Test description

8.2.4.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.8.3.2	Test procedure sequence

Table 8.2.4.8.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.8.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M1 > M2) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | The power level values are such that SrxlevCell 1 > SrxlevCell 2 and SrxlevCell 2 <0.(NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.8.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP and RSRQ value for Cell 2. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.8.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.2.4.8.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS does not respond to any RRCConnectionReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to RRC_IDLE state on Cell 1. | - | - | - | - |
| 9-13 | The UE will perform TAU procedure based on steps 1 to 5 of subclause 6.4.2.7 in TS 36.508 on Cell 1. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery. | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

Table 8.2.4.8.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304 | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 2 | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.8.3.3	Specific message contents

Table 8.2.4.8.3.3-0: Conditions for specific message contents
in Table 8.2.4.8.3.3-1A

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.8.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.8.3.2-2)

Derivation path: 36.508 Table 4.6.1-8, condition MEAS

Table 8.2.4.8.3.3-1A: MeasConfig (Table 8.2.4.8.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.8.3.3-2: MeasurementReport (step 4, Table 8.2.4.8.3.2-2)

| Derivation path: 36.508 Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.8.3.3-3: RRCConnectionReconfiguration (step 5, Table 8.2.4.8.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 2 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.8.3.3-3A: MobilityControlInfo (Table 8.2.4.8.3.3-3)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.2.4.8.3.3-4: Void

Table 8.2.4.8.3.3-5: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.8.3.2-2)

| Derivation path: 36.508 Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | The value of the C-RNTI of the UE. |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.9	RRC connection reconfiguration / Handover / Inter-band blind handover / Success

8.2.4.9.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating an E-UTRA frequency on different frequency band }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

...

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.9.3	Test description

8.2.4.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.9.3.2	Test procedure sequence

Table 8.2.4.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 10? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 10? | - | - | 1 | - |

8.2.4.9.3.3	Specific message contents

Table 8.2.4.9.3.3-0: Conditions for specific message contents
in Table 8.2.4.9.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.9.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.9.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.9.3.3-2: MobilityControlInfo (Table 8.2.4.9.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.10	RRC connection reconfiguration / Handover (between FDD and TDD)

8.2.4.10.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure in FDD mode}

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including an IE mobilityControlInformation with TDD configuration parameters included }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE having completed the radio bearer establishment and initial security activation procedure in TDD mode}

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including an IE mobilityControlInformation with FDD configuration parameters included }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target cell by acquiring system information from that cell before performing RACH access in the target cell.

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.10.3	Test description

8.2.4.10.3.1	Pre-test conditions

System Simulator:

-	Cell 1 in FDD and Cell 10 in TDD

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.2.4.10.3.2	Test procedure sequence

Table 8.2.4.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including TDD-Config to order the UE to perform inter band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 10? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 10? | - | - | 1 | - |
| 4 | The SS transmits an RRCConnectionReconfiguration message without including TDD-Config to order the UE to perform inter band handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 2 | - |

8.2.4.10.3.3	Specific message content

Table 8.2.4.10.3.3-0: Conditions for specific message contents in Tables 8.2.4.10.3.3-2 and 8.2.4.10.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.10.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.10.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.10.3.3-2: MobilityControlInfo (Table 8.2.4.10.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 10 |  |  |
| ul-Bandwidth | Not present |  |  |
| } |  |  |  |
| additionalSpectrumEmission | Same additionalSpectrumEmission as used for Cell 10 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-withTDDconfig |  |  |
| } |  |  |  |

Table 8.2.4.10.3.3-2A: RadioResourceConfigCommon-withTDDconfig (Table 8.2.4.10.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| tdd-Config | TDD-Config-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.10.3.3-3: RRCConnectionReconfiguration (step 4, Table 8.2.4.10.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.10.3.3-4: MobilityControlInfo (Table 8.2.4.10.3.3-3)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Same uplink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Same uplink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 1 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| additionalSpectrumEmission | Same additionalSpectrumEmission as used for Cell 1 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-withoutTDDconfig |  |  |
| } |  |  |  |

Table 8.2.4.10.3.3-5: RadioResourceConfigCommon-withoutTDDconfig (Table 8.2.4.10.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| tdd-Config | Not present |  |  |
| } |  |  |  |

#### 8.2.4.11	Void

#### 8.2.4.12	RRC connection reconfiguration / Handover / Setup and release of MIMO

8.2.4.12.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a cell with 2 antenna ports }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a cell with 1 antenna port }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

…

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323.

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

…

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

…

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

…

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target cell by acquiring system information from that cell before performing RACH access in the target cell.

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' or 'tm8':

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

2>	if the configured transmissionMode is 'tm8' and pmi-RI-Report is not present:

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.12.3	Test description

8.2.4.12.3.1	Pre-test conditions

System Simulator:

- Cell 1 and Cell 2 (2 antenna ports MIMO)

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on cell 1 according to [18].

8.2.4.12.3.2	Test procedure sequence

Table 8.2.4.12.3.2-0 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.12.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |

Table 8.2.4.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |
| 3A | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.12.3.2-0. | - | - | - | - |
| 4 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 2 | - |

8.2.4.12.3.3	Specific message content

Table 8.2.4.12.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.4.2.12.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.12.3.3-2: MobilityControlInfo (Table 8.2.4.12.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.12.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.12.3.3-2)

Derivation Path: 36.508, Table 4.6.3-13, condition 2TX

Table 8.2.4.12.3.3-4: RRCConnectionReconfiguration (step 4, Table 8.4.2.12.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.12.3.3-5: MobilityControlInfo (Table 8.2.4.12.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.12.3.3-6: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.12.3.3-5)

| Derivation Path: 36.508, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| antennaInfoCommon SEQUENCE { |  |  |  |
| antennaPortsCount | an1 |  |  |
| } |  |  |  |

#### 8.2.4.13	RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band

8.2.4.13.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the inter-band measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating an E-UTRA frequency on different frequency band }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' or 'tm8':

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

2>	if the configured transmissionMode is 'tm8' and pmi-RI-Report is not present:

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.13.3	Test description

8.2.4.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.13.3.2	Test procedure sequence

Table 8.2.4.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.13.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy exit condition for event A3 (M10 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy entry condition for event A3 (M10 > M1). |

Table 8.2.4.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter-band measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 10 parameters according to the row "T1" in table 8.2.4.13.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 10. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 10 to confirm the successful completion of the inter-band handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 10? | - | - | 1 | - |

8.2.4.13.3.3	Specific message contents

Table 8.2.4.13.3.3-0: Conditions for specific message contents in Tables 8.2.4.13.3.3-2 and 8.2.4.13.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.13.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.13.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.13.3.3-2: MeasConfig (Table 8.2.4.13.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f5 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13.3.3-3: MeasurementReport (step 4, Table 8.2.4.13.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 10 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.13.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.13.3.3-5: MobilityControlInfo (Table 8.2.4.13.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.13a	RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band / Between FDD and TDD

8.2.4.13a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA FDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure and performed the inter band LTE TDD measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA TDD frequency }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA TDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure and performed the inter band LTE FDD measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA FDD frequency }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.13a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.13a.3	Test description

8.2.4.13a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is FDD cell and Cell 28 is TDD cell.

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 8.2.4.13a.3.1-1.

Table 8.2.4.13a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 28 | PLMN2 |

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.2.4.13a.3.3-9.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.13a.3.2	Test procedure sequence

Table 8.2.4.13a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.13a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy exit condition for event A3 (M28 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy entry condition for event A3 (M28 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -73 | -85 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy entry condition for event A3 (M1 > M28). |

Table 8.2.4.13a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter band measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 28 power level according to the row "T1" in table 8.2.4.13a.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ values for Cell 28. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter band handover to Cell 28. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 28 to confirm the successful completion of the inter band handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 6A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 28. | - | - | - | - |
| 6B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | - | - | - | - |
| 6C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 28? | - | - | 1 | - |
| 8 | The SS transmits an RRCConnectionReconfiguration message on Cell 28 to setup inter band measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 9 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 28. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10 | The SS changes Cell 1 and Cell 28 power levels according to the row "T2" in table 8.2.4.13a.3.2-1. | - | - | - | - |
| 11 | The UE transmits a MeasurementReport message on Cell 28 to report event A3 with the measured RSRP, RSRQ values for Cell 1. | --> | MeasurementReport | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message on Cell 28 to order the UE to perform inter band handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the successful completion of the inter band handover? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 13A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 1. | - | - | - | - |
| 13B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN2 listed as an Equivalent PLMN. | - | - | - | - |
| 13C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 2 | - |

8.2.4.13a.3.3	Specific message contents

Table 8.2.4.13a.3.3-0: Conditions for specific message contents
in Tables 8.2.4.13a.3.3-2, 8.2.4.13a.3.3-5, 8.2.4.13a.3.3-6 and 8.2.4.13a.3.3-8

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.13a.3.3-1: RRCConnectionReconfiguration (step 1 and step 8, Table 8.2.4.13a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.13a.3.3-2: MeasConfig (step 1, Table 8.2.4.13a.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f6 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f6) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f6 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-3: MeasurementReport (step 4, Table 8.2.4.13a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 28 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-4: RRCConnectionReconfiguration (step 5 and step 12, Table 8.2.4.13a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.13a.3.3-5: MobilityControlInfo (step 5, Table 8.2.4.13a.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 28 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 28 |  |  |
| ul-CarrierFreq | Not present |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 28 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-6: MeasConfig (step 8, Table 8.2.4.13a.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entry |  |  |
| measObjectId[1] | IdMeasObject-f6 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f6) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f1 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f6 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-7: MeasurementReport (step 11, Table 8.2.4.13a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 28 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-8: MobilityControlInfo (step 12, Table 8.2.4.13a.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.13a.3.3-9: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.2.4.13a.3.3-10: TRACKING AREA UPDATE ACCEPT for Cell 28 (step 6B, Table 8.2.4.13a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.2.4.13a.3.3-11: TRACKING AREA UPDATE ACCEPT for Cell 1 (step 13B, Table 8.2.4.13a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.2.4.14	RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band

8.2.4.14.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA cell having attempted inter-band handover }

ensure that {
  when { UE detects handover failure and the target cell is selectable }

then { UE performs an RRC connection re-establishment procedure and remains in the E-UTRA RRC_CONNECTED state }

}

8.2.4.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.4, 5.3.7.5, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 4:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2> store the following handover failure information in VarRLF-Report by setting its fields as follows:

3> clear the information included in VarRLF-Report, if any;

3> set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3> set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on

measurements collected up to the moment the UE detected handover failure;

…

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

NOTE 5:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	stop timer T301;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

NOTE 6:	E-UTRAN should not transmit any message on SRB1 prior to receiving the RRCConnectionReestablishmentComplete message.

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2> if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3> include the rlf-InfoAvailable;

…

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' or 'tm8':

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

2>	if the configured transmissionMode is 'tm8' and pmi-RI-Report is not present:

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.14.3	Test description

8.2.4.14.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.14.3.2	Test procedure sequence

Table 8.2.4.14.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.14.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy exit condition for event A3 (M10 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy entry condition for event A3 (M10 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | The power level values are assigned to satisfy SrxlevCell 1 < 0 such that selecting Cell 10 is guaranteed (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-band measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 10 parameters according to the row "T1" in table 8.2.4.14.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 10. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.14.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 1 and Cell 10 parameters according to the row "T2" in table 8.2.4.14.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 10? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 8 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 10. | <-- | RRCConnectionReestablishment | - | - |
| 9 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 10. | --> | RRCConnectionReestablishmentComplete | - | - |
| 10 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 10. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 12 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 10? | - | - | 1 | - |

Table 8.2.4.14.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 10. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.14.3.3	Specific message contents

Table 8.2.4.14.3.3-0: Conditions for specific message contents
in Tables 8.2.4.14.3.3-3 and 8.2.4.14.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.14.3.3-1: SystemInformationBlockType2 for Cell 10 (preamble and all the steps in Table 8.2.4.14.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.4.14.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.14.3.3-3: MeasConfig (Table 8.2.4.14.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f5 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-4: MeasurementReport (step 4, Table 8.2.4.14.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 10 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-5: RRCConnectionReconfiguration (step 5, Table 8.2.4.14.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.14.3.3-6: MobilityControlInfo (Table 8.2.4.14.3.3-5)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-7: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.14.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-8: RRCConnectionReestablishmentComplete (step 9, Table 8.2.4.14.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | Rel-8 |
| nonCriticalExtension SEQUENCE {} | Not checked |  | Rel-9 |
| nonCriticalExtension SEQUENCE { |  |  | Rel-10 and above |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14.3.3-9: RRCConnectionReconfiguration (step 10, Table 8.2.4.14.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.14a	RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band / Between FDD and TDD

8.2.4.14a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA FDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA TDD cell having attempted inter-mode handover }

ensure that {

when { UE detects handover failure and the initial E-UTRA TDD cell is selectable }

then { UE performs an RRC connection re-establishment procedure and remains in the E-UTRA RRC_CONNECTED state on the E-UTRA TDD cell }

}

(2)

with { UE in E-UTRA FDD RRC_CONNECTED state and having transmitted an RRCConnectionReestablishmentRequest message on E-UTRA TDD cell }

ensure that {
  when { UE receives an RRCConnectionReestablishment message with a nextHopChainingCount which is same as the NCC associated with the currently active KeNB }

then { UE derives new KeNB from the currently active KeNB }

}

(3)

with { UE in E-UTRA TDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA FDD cell having attempted inter-mode handover }

ensure that {
  when { UE detects handover failure and the initial E-UTRA FDD cell is selectable }

then { UE performs an RRC connection re-establishment procedure and remains in the E-UTRA RRC_CONNECTED state on the E-UTRA FDD cell }

}

(4)

with { UE in E-UTRA TDD RRC_CONNECTED state and having transmitted an RRCConnectionReestablishmentRequest message on E-UTRA FDD cell }

ensure that {
  when { UE receives an RRCConnectionReestablishment message with a nextHopChainingCount which is different from the NCC associated with the currently active KeNB }

then { UE derives new KeNB from the nextHopChainingCount }

}

8.2.4.14a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.4, 5.3.7.5, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from  E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

NOTE 5:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	stop timer T301;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

NOTE 6:	E-UTRAN should not transmit any message on SRB1 prior to receiving the RRCConnectionReestablishmentComplete message.

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2> if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3> include the rlf-InfoAvailable;

…

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.14a.3	Test description

8.2.4.14a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is FDD cell and Cell 28 is TDD cell.

-	Cell 28 has a valid UE context

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 8.2.4.14a.3.1-1.

Table 8.2.4.14a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 28 | PLMN2 |

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.2.4.14a.3.3-15.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.14a.3.2	Test procedure sequence

Table 8.2.4.14a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.14a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy exit condition for event A3 (M28 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy entry condition for event A3 (M28 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -73 | The power level values are assigned to satisfy SrxlevCell 1 < 0 such that selecting Cell 28 is guaranteed (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 28 (M28) satisfy entry condition for event A3 (M1> M28). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | The power level values are assigned to satisfy SrxlevCell 28 < 0 such that selecting Cell 1 is guaranteed. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.14a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-band measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 28 power level according to the row "T1" in table 8.2.4.14a.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 28. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 28. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.14a.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 1 power level according to the row "T2" in table 8.2.4.14a.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 28? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 8 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 28. | <-- | RRCConnectionReestablishment | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the currently active KeNB on Cell 28? | --> | RRCConnectionReestablishmentComplete | 2 | P |
| 10 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 28. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 28. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 11A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 28. | - | - | - | - |
| 11B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 11C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 12 | The SS changes Cell 1 and Cell 28 power levels according to the row "T3" in table 8.2.4.14a.3.2-1. | - | - | - | - |
| 13 | The UE transmits a MeasurementReport message on Cell 28 to report event A3 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message on Cell 28 to order the UE to perform inter-band handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 15 the steps specified in Table 8.2.4.14a.3.2-4 should take place. | - | - | - | - |
| 15 | The SS changes Cell 28 power level according to the row "T4" in table 8.2.4.14a.3.2-1. | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 1? | --> | RRCConnectionReestablishmentRequest | 3 | P |
| 17 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 18 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the nextHopChainingCount on Cell 1? | --> | RRCConnectionReestablishmentComplete | 4 | P |
| 19 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 20 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 20A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 1. | - | - | - | - |
| 20B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN2 listed as an Equivalent PLMN | - | - | - | - |
| 20C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 21 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 3 | - |

Table 8.2.4.14a.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 28. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

Table 8.2.4.14a.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 1. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.14a.3.3	Specific message contents

Table 8.2.4.14a.3.3-00: Conditions for specific message contents
in Tables 8.2.4.14a.3.3-2, 8.2.4.14a.3.3-5 and 8.2.4.14a.3.3-12

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.14a.3.3-0: SystemInformationBlockType2 for Cell 1 and Cell 28 (preamble and all the steps in Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.14a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.14a.3.3-2: MeasConfig (Table 8.2.4.14a.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f6 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f6) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f6 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-3: MeasurementReport (step 4, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 28 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.14a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.14a.3.3-5: MobilityControlInfo (Table 8.2.4.14a.3.3-4)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 28 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 28 |  |  |
| ul-CarrierFreq | Not present |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 28 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-7: RRCConnectionReestablishment (step 8, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-7A: RRCConnectionReestablishmentComplete (step 9 and step 18, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | Rel-8 |
| nonCriticalExtension SEQUENCE {} | Not checked |  | Rel-9 |
| nonCriticalExtension SEQUENCE { |  |  | Rel-10 and above |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-8: RRCConnectionReconfiguration (step 10 and step 19, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-9: MeasConfig (Table  8.2.4.14a.3.3-8)

Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ

Table 8.2.4.14a.3.3-10: MeasurementReport (step 13, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 28 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-11: RRCConnectionReconfiguration (step 14, Table 8.2.4.14a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.14a.3.3-12: MobilityControlInfo (Table 8.2.4.14a.3.3-11)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-13: RRCConnectionReestablishmentRequest (step 16, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 28 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-14: RRCConnectionReestablishment (step 17, Table 8.2.4.14a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.14a.3.3-15: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.2.4.14a.3.3-16: TRACKING AREA UPDATE ACCEPT for Cell 28 (step 11B, Table 8.2.4.14a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.2.4.14a.3.3-17: TRACKING AREA UPDATE ACCEPT for Cell 1 (step 20B, Table 8.2.4.14a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

#### 8.2.4.15	RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band

8.2.4.15.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including an IE mobilityControlInformation indicating a different E-UTRA cell having attempted inter-band handover }

ensure that {

when { UE detects handover failure and fails an RRC connection re-establishment procedure }

then { UE enters the E-UTRA RRC_IDLE state and trigger TAU procedure in order to recover RRC connection }

}

8.2.4.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.3, 5.3.7.4, 5.3.7.7 and 5.3.12

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target cell by acquiring system information from that cell before performing RACH access in the target cell.

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 5:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.3]

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	stop timer T311;

1>	start timer T301;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 6:	This procedure applies also if the UE returns to the source cell.

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

2>	else:

3>	set the reestablishmentCause to the value 'otherFailure';

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message:

2>	enter RRC_IDLE  and perform procedures as specified in TS 36.304 [4, 5.2.7];

8.2.4.15.3	Test description

8.2.4.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.15.3.2	Test procedure sequence

Table 8.2.4.15.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.15.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy exit condition for event A3 (M1 > M10) |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy entry condition for event A3 (M10 > M1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | The power level values are such that SrxlevCell 1 > SrxlevCell 10 and SrxlevCell 10 <0. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.15.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-band measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 10 parameters according to the row "T1" in table 8.2.4.15.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP and RSRQ value for Cell 10. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.15.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 1 and Cell 10 parameters according to the row "T2" in table 8.2.4.15.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS does not respond to any RRCConnectionReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to RRC_IDLE state on Cell 1. | - | - | - | - |
| 9-13 | The UE will perform TAU procedure based on steps 1 to 5 of subclause 6.4.2.7 in TS 36.508 on Cell 1. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery. | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

Table 8.2.4.15.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304 | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 10 | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.15.3.3	Specific message contents

Table 8.2.4.15.3.3-0: Conditions for specific message contents
in Tables 8.2.4.15.3.3-2 and 8.2.4.15.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.15.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.15.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition MEAS

Table 8.2.4.15.3.3-2: MeasConfig (Table 8.2.4.15.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f5 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15.3.3-3: MeasurementReport (step 4, Table 8.2.4.15.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 10 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.15.3.2-2)

Derivation path: 36.508 Table 4.6.1-8, condition HO

Table 8.2.4.15.3.3-5: MobilityControlInfo (Table 8.2.4.15.3.3-4)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.15.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | The value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.15a	RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band / Between FDD and TDD

8.2.4.15a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA FDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure on E-UTRA FDD cell and after receiving an RRCConnectionReconfiguration message including an IE mobilityControlInfo indicating a different E-UTRA TDD cell having attempted inter-mode handover }

ensure that {

when { UE detects handover failure and fails an RRC connection re-establishment procedure }

then { UE enters the E-UTRA FDD RRC_IDLE state and trigger TAU procedure in order to recover RRC connection on E-UTRA FDD cell }

}

(2)

with { UE in E-UTRA TDD RRC_CONNECTED state and having completed the radio bearer establishment and initial security activation procedure on E-UTRA TDD cell and after receiving an RRCConnectionReconfiguration message including an IE mobilityControlInfo indicating a different E-UTRA FDD cell having attempted inter-mode handover }

ensure that {

when { UE detects handover failure and fails an RRC connection re-establishment procedure }

then { UE enters the E-UTRA TDD RRC_IDLE state and trigger TAU procedure in order to recover RRC connection on E-UTRA TDD cell }

}

8.2.4.15a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.3, 5.3.7.4, 5.3.7.7 and 5.3.12.

[TS 36.331, clause 5.3.5.4]

...

If the RRCConnectionReconfiguration message includes the mobilityControlInformation and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInformation;

1>	request PDCP to initiate the PDCP Re-establishment procedure for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the L2 re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in [8].

1>	reset MAC and re-establish RLC for all RBs that are established;

1>	If the RRCConnectionReconfiguration message includes the radioResourceConfiguration:

2>	perform the Radio resource configuration procedure as specified in 5.3.10;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	if the eutra-CarrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the eutra-CarrierFreq with a physical cell identity indicated by the targetCellIdentity;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetCellIdentity;

1>	if the dl-Bandwidth is included:

2>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	else:

2>	for the target cell, apply the same downlink bandwidth as for the current cell;

1>	if the ul-Bandwidth is included:

2>	for the target cell, apply the uplink bandwidth indicated by the ul-Bandwidth;

1>	else:

2>	for the target cell, apply the same uplink bandwidth as for the current cell;

1>	 configure lower layers in accordance with the received radioResourceConfigCommon;

1>	If the RRCConnectionReconfiguration message includes the securityConfiguration:

2>	apply the AS-derived keys associated with the AS-base key indicated by the keyIndicator;

2>	configure lower layers to apply the indicated integrity protection algorithm, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE in the target cell, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the indicated ciphering algorithm, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE in the target cell, including the message used to indicate the successful completion of the procedure;

1>	If the RRCConnectionReconfiguration message includes the measurementConfiguration:

2>	perform the Measurement configuration procedure as specified in 5.5.2;

1>	synchronise to the DL of the target cell;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	If MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	If the physicalConfigDedicated is included in the RRCConnectionReconfiguration message:

3>	If the UE needs the SFN of the target cell to apply the PUCCH and Sounding RS configuration:

4>	apply the new PUCCH and Sounding RS configuration upon acquiring the SFN of the target cell;

3>	else:

4>	apply the new PUCCH and Sounding RS configuration;

2> indicate to PDCP to complete the PDCP Re-establishment procedure for all DRBs that are established, if any;

2>	the procedure ends.

...

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	If T304 expires (handover failure):

NOTE 1:	Following T304 expiry dedicated preambles, if provided within the rach-ConfigDedicated, are not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the physical layer configuration;

NOTE 2:	The UE reverts to the RRC configuration as well as the layer 2 configuration (PDCP/RLC/MAC) used in the source cell.

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends.

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

...

1>	start timer T311;

1>	request PDCP to initiate the PDCP Re-establishment procedure for all RBs that are established;

NOTE 1:	The handling of the radio bearers after the successful completion of the L2 re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in [8].

1>	reset MAC and re-establish RLC for all RBs that are established;

1>	select a suitable cell in accordance with the cell selection process as specified in [4];

[TS 36.331, clause 5.3.7.3]

...

2>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

...

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the IE ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover failure case) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the cellIdentity to the Physical layer identity of the source cell (handover failure case) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the concatenation of the ASN.1 encoded CellIdentity of the current cell, PhysicalCellIdentity of the cell the UE was connected to prior to the failure and C-RNTI that the UE had in the cell it was connected to prior to the failure;

3>	with the integrity protection key and integrity protection algorithm that was used in the cell the UE was connected to prior to the failure; and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones.

1>	set the IE reestablishmentCause as follows:

...

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value ‘handoverFailure’;

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.7]

Upon T311 expiry, the UE shall:

1>	perform the actions upon moving from RRC_CONNECTED to RRC_IDLE as specified in 5.3.12.

...

[TS 36.331, clause 5.3.12]

Upon moving from RRC_CONNECTED to RRC_IDLE, the UE shall:

1>	reset MAC and re-establish RLC for all RBs that are established;

...

1>	stop all timers that are running except T320;

1>	release all radio resources, including release of the RLC entity and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers;

1>	enter RRC_IDLE.

8.2.4.15a.3	Test description

8.2.4.15a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is an LTE FDD cell, Cell 10 and Cell 30 are LTE TDD cells.

-	Each cell has only a single PLMN identity. The PLMNs are identified in the test by the identifiers in Table 8.2.4.15 a.3.1-1.

Table 8.2.4.15 a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 10 | PLMN1 |
| 30 | PLMN2 |

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 and Table 6.3.1.2-2 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.2.4.15a.3.3-13.

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.15a.3.2	Test procedure sequence

Table 8.2.4.15a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1",  "T2",  "T3", "T4" and "T5" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.15a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | “Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy exit condition for event A3 (M10 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | “Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 10 (M10) satisfy entry condition for event A3 (M10 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | “Off” | The power level values are assigned to satisfy SrxlevCell 10 < 0 such that selecting Cell 1 is guaranteed (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 30 (M30) satisfy entry condition for event A3 (M30> M1). (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -73 | "Off" | -85 | The power level values are such that measurement results for Cell 30 (M30) satisfy entry condition for event A3 (M1> M30). (NOTE 1). |
| T5 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -85 | The power level values are assigned to satisfy SrxlevCell 1 < 0 such that selecting Cell 30 is guaranteed (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.2.4.15a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-band measurement on Cell 1 for Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 10 parameters according to the row "T1" in table 8.2.4.15a.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP and RSRQ value for Cell 10. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 10. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.2.4.15a.3.2-3 should take place. | - | - | - | - |
| 6 | The SS changes Cell 10 parameters according to the row "T2" in table 8.2.4.15a.3.2-1. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 8 | The SS does not respond to any RRCConnectionReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to RRC_IDLE state on Cell 1. | - | - | - | - |
| 9 | Generic test procedure described in TS 36.508 subclause 6.4.2.7 is performed on Cell 1. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery. NOTE: The TAU is accepted with PLMN2 listed as an Equivalent PLMN | - | - | - | - |
| 10 | Generic test procedure described in TS 36.508 subclause 4.5.3.3.is executed | - | - | 1 | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message to setup inter-band measurement on Cell 1 for Cell 30. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | The SS changes Cell 30 parameters according to the row "T3" in table 8.2.4.15a.3.2-1. | - | - | - | - |
| 14 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP and RSRQ value for Cell 30. | --> | MeasurementReport | - | - |
| 15 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-band handover to Cell 30. | <-- | RRCConnectionReconfiguration | - | - |
| 16 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 30 to confirm the successful handover | --> | RRCConnectionReconfigurationComplete | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 30. | - | - | - | - |
| 18 | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 19 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 20 | The SS changes Cell 1 and Cell 30 parameters according to the row "T4" in table 8.2.4.15a.3.2-1. | - | - | - | - |
| 21 | The UE transmits a MeasurementReport message on Cell 30 to report event A3 with the measured RSRP and RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 22 | The SS transmits an RRCConnectionReconfiguration message on Cell 30 to order the UE to perform inter-band handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 26 the steps specified in Table 8.2.4.15a.3.2-4 should take place. | - | - | - | - |
| 23 | The SS changes Cell 1 parameters according to the row "T5" in table 8.2.4.15a.3.2-1. | - | - | - | - |
| 24 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 30. | --> | RRCConnectionReestablishmentRequest | - | - |
| 25 | The SS does not respond to any RRCConnectionReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to RRC_IDLE state on Cell 30. | - | - | - | - |
| 26-30 | The UE will perform TAU procedure based on steps 1 to 5 of subclause 6.4.2.7 in TS 36.508 on Cell 30. NOTE: The UE performs a TAU procedure due to NAS signalling connection recovery or TAI change. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 31 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 30? | - | - | 2 | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

Table 8.2.4.15a.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304 | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 10 | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

Table 8.2.4.15a.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304 | - | - | - | - |
| 1 | The UE attempts to perform the inter-band handover using MAC Random Access Preamble on Cell 1 | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.15a.3.3	Specific message contents

Table 8.2.4.15a.3.3-0: Conditions for specific message contents
in Tables 8.2.4.15a.3.3-2, 8.2.4.15a.3.3-5, 8.2.4.15a.3.3-8 and 8.2.4.15a.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.15a.3.3-1: RRCConnectionReconfiguration (step 1 and step 11, Table 8.2.4.15a.3.2-2)

Derivation path: 36.508 Table 4.6.1-8, condition MEAS

Table 8.2.4.15a.3.3-2: MeasConfig (step 1 and step 11, Table 8.2.4.15a.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f5 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-3: MeasurementReport (step 4, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 10 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-4: RRCConnectionReconfiguration (step 5 and step 22, Table 8.2.4.15a.3.2-2)

Derivation path: 36.508 Table 4.6.1-8, condition HO

Table 8.2.4.15a.3.3-4A: RRCConnectionReconfiguration (step 15, Table 8.2.4.15a.3.2-2)

Derivation path: 36.508 Table 4.6.1-8, condition HO and condition MEAS

Table 8.2.4.15a.3.3-4B: MeasConfig (step 15 Table 8.2.4.15a.3.3-4A)

Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ

Table 8.2.4.15a.3.3-5: MobilityControlInfo (step 5, Table 8.2.4.15a.3.3-4)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-6: RRCConnectionReestablishmentRequest (step 7, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | The value of the C-RNTI of the UE. |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-7: MeasurementReport (step 14, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 30 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-8: MobilityControlInfo (step 15, Table 8.2.4.15a.3.3-4A)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 30 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 30 |  |  |
| ul-CarrierFreq | Not present |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 30 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-9: MeasurementReport (step 21, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  | Cell 30 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 | Cell 1 |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-10: MobilityControlInfo (step 22, Table 8.2.4.15a.3.3-4)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-11: RRCConnectionReestablishmentRequest (step 24, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | The value of the C-RNTI of the UE. |  |  |
| physCellId | PhysicalCellIdentity of Cell 30 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.15a.3.3-12: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.2.4.15a.3.3-13: TRACKING AREA UPDATE ACCEPT for Cell 1 (steps 9, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.2.4.15a.3.3-14: TRACKING AREA UPDATE ACCEPT for Cell 30 (step 18, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.2.4.15a.3.3-15: TRACKING AREA UPDATE ACCEPT for Cell 30 (steps 26-30, Table 8.2.4.15a.3.2-2)

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

#### 8.2.4.16	CA / RRC connection reconfiguration / Setup and Change of MIMO

##### 8.2.4.16.1	CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band Contiguous CA

8.2.4.16.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and configured an Scell }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message indicating PCell and Scell TM modification}

then { UE sends an RRCConnectionReconfigurationComplete message and changes transmission mode of the Pcell and SCell}

}

8.2.4.16.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.10.6.

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	if the antennaInfo-r10 is included in the received physicalConfigDedicated and the previous version of this field that was received by the UE was antennaInfo (without suffix i.e. the version defined in REL-8):

2>	apply the default antenna configuration as specified in section 9.2.4;

1>	if the cqi-ReportConfig-r10 is included in the received physicalConfigDedicated and the previous version of this field that was received by the UE was cqi-ReportConfig (without suffix i.e. the version defined in REL-8):

2>	apply the default CQI reporting configuration as specified in 9.2.4;

NOTE:	Application of the default configuration involves release of all extensions introduced in REL-9 and later.

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to explicitValue:

2>	if the configured transmissionMode is not tm3 or tm4 or tm8 or tm9; or

2>	if the configured transmissionMode is tm8 and pmi-RI-Report is not present; or

2>	if the configured transmissionMode is tm9 and pmi-RI-Report is not present; or

2>	if the configured transmissionMode is tm9 and pmi-RI-Report is present and antennaPortsCount within csi-RS is set to an1:

3>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to defaultValue:

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.16.1.3	Test description

8.2.4.16.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18] , with TM2 configured.

8.2.4.16.1.3.2	Test procedure sequence

Table 8.2.4.16.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.16.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -107 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |

Table 8.2.4.16.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.16.1.3.2-1. |  |  |  |  |
| 2 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to reconfigure Cell 3 as a Scell and reconfiguring transmission mode for both PCell and SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 3 | Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 4 | The SS transmits an RRCConnectionReconfiguration message to reconfigure transmission mode on both PCell and SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | 1 | P |

8.2.4.16.1.3.3	Specific message contents

Table 8.2.4.16.1.3.3-0: Conditions for specific message contents
in Table 8.2.4.16.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.16.1.3.3-1: RRCConnectionReconfiguration (step 2, Table 8.2.4.16.1.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod and 2TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated-SCell_AddMod SEQUENCE { |  |  |  |
| PhysicalConfigDedicated SEQUENCE { |  |  |  |
| antennaInfo-r10 | TM3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.16.1.3.3-2: SCellToAddMod-r10 (Table 8.2.4.16.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.16.1.3.3-3: RadioResourceConfigDedicatedSCell-r10 (Table 8.2.4.16.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| PhysicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 { |  |  |  |
| transmissionMode-r10 | TM2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.16.1.3.3-4: RRCConnectionReconfiguration (step 4, Table 8.2.4.16.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition SCell_AddMod and 2TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated-SCell_AddMod SEQUENCE { |  |  |  |
| PhysicalConfigDedicated SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | TM2 |  |  |
| codebookSubsetRestriction-r10 | Not present |  |  |
| ue-TransmitAntennaSelection | release |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.16.1.3.3-5: SCellToAddMod-r10 (Table 8.2.4.16.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE {} | Not present |  |  |
| radioResourceConfigCommonSCell-r10 | Not present |  |  |
| } |  |  |  |

Table 8.2.4.16.1.3.3-6: RadioResourceConfigDedicatedSCell-r10 (Table 8.2.4.16.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| PhysicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 { |  |  |  |
| transmissionMode-r10 | TM3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.16.1.3.3-7: RRCConnectionReconfiguration (preamble, step 8, TS 36.508 table 4.5.3.3-1)

| Derivation path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) SEQUENCE { |  |  |  |
| PhysicalConfigDedicated SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | TM2 |  |  |
| codebookSubsetRestriction-r10 | Not present |  |  |
| ue-TransmitAntennaSelection | release |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.16.2	CA / RRC connection reconfiguration / Setup and Change of MIMO / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.16.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: Cell 10 replaces Cell 3.

##### 8.2.4.16.3	CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.16.1 with the following differences:

-	CA configuration: Intra-band non-contiguous CA replaces Intra-band contiguous CA

#### 8.2.4.17	CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition

##### 8.2.4.17.1	CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band Contiguous CA

8.2.4.17.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the inter frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA frequency to be handover to one of the SCell and sCellToAddModList with an SCell addition }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.17.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	configure lower layers to consider the SCell(s), if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

8.2.4.17.1.3	Test description

8.2.4.17.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell and Cell 3 SCell

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.17.1.3.2	Test procedure sequence

Table 8.2.4.17.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.17.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | The power level values are such that camping on Cell 1 is guarantee. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | Power levels are such that entry condition for event A3 (measId 1) is satisfied: |

Table 8.2.4.17.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including sCellToAddModList with Cell 3 as SCell addition. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup event A3 reporting configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 3 parameters according to the row "T1" in table 8.2.4.17.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform handover to Cell 3 with Cell 1 as SCell addition at the same time. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the handover and SCell addition of Cell 1? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 3? | - | - | 1 | - |

8.2.4.17.1.3.3	Specific message contents

Table 8.2.4.17.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.17.1.3.3-2 and 8.2.4.17.1.3.3-6.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.17.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.17.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f2 | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-2: SCellToAddMod-r10-f2 (Table 8.2.4.17.1.3.3-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.17.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.4.17.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-4: Void

Table 8.2.4.17.1.3.3-5: RRCConnectionReconfiguration (step 3, Table 8.2.4.17.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.17.1.3.3-6: MeasConfig (Table 8.2.4.17.1.3.3-5)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-7: MeasurementReport (step 6, Table 8.2.4.17.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | physicalCellId of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | Cell 3 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-8: RRCConnectionReconfiguration (step 7, Table 8.2.4.17.1.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-8 condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| sCellToAddModList-r10 | SCellToAddMod-r10-f1 | SCell addition for Cell 1 |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-9: MobilityControlInfo (Table 8.2.4.17.1.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.17.1.3.3-10: SCellToAddMod-r10-f1 (Table 8.2.4.17.1.3.3-8)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f1 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 1 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.17.1.3.3-11: RadioResourceConfigCommonSCell-r10-f3 (Table 8.2.4.17.1.3.3-10)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.17.2	CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.17.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

##### 8.2.4.17.3	CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.17.1 with the following differences:

-	CA configuration: Intra-band non-contiguous CA replaces Intra-band Contiguous CA.

#### 8.2.4.18	CA / RRC connection reconfiguration / Handover / Success / SCell release

##### 8.2.4.18.1	CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band Contiguous CA

8.2.4.18.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure, performed the intra-frequency measurement and configured an SCell }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo and including sCellToReleaseList with an sCellIndex set to the configured SCell }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

8.2.4.18.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4 and 5.3.10.3a.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	configure lower layers to consider the SCell(s), if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10.3a]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

8.2.4.18.1.3	Test description

8.2.4.18.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is PCell, Cell 2 is intra-frequency cell of PCell, and Cell 3 is SCell to be added

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.18.1.3.2	Test procedure sequence

Table 8.2.4.18.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.18.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) do not satisfy entry condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.18.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to configure Cell 3 as a SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement reporting for event A3 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 2 level according to row "T1" in Table 8.2.4.18.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A3. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to perform intra-frequency handover and SCell release on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.2.4.18.1.3.3	Specific message contents

Table 8.2.4.18.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.18.1.3.3-2 and 8.2.4.18.1.3.3-5.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.18.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.18.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.4.18.1.3.3-2: SCellToAddMod-r10 (Table 8.2.4.18.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.18.1.3.3-3: RadioResourceConfigCommonSCell-r10 (Table 8.2.4.18.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.18.1.3.3-4: RRCConnectionReconfiguration (step 3, Table 8.2.4.18.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.18.1.3.3-5: MeasConfig (Table 8.2.4.18.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.18.1.3.3-5A: ReportConfig-A3 (Table 8.2.4.18.1.3.3-5)

| Derivation path: 36.508, Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| timeToTrigger | ms2560 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.18.1.3.3-6: MeasurementReport (step 6, Table 8.2.4.18.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| servFreqId-r10[1] | 1 | Cell 3 |  |
| measResultSCell-r10[1] SEQUENCE { |  |  |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| measResultBestNeighCell-r10[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.18.1.3.3-7: RRCConnectionReconfiguration (step 7, Table 8.2.4.18.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.18.1.3.3-8: MobilityControlInfo (Table 8.2.4.18.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

##### 8.2.4.18.2	CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.18.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

8.2.4.18.2.1	Specific message contents

Same as test case 8.2.4.18.1 with the following differences.

Table 8.2.4.18.2.1-1: Conditions for specific message contents in Table 8.2.4.18.2.1-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.18.2.1-1: MeasConfig (Table 8.2.4.18.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 Apply conditions “DS_Meas and LAA SCell” to work out the value of this IE |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.18.3	CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.18.1 with the following differences:

-	CA configuration: Intra-band non-contiguous CA replaces Intra-band Contiguous CA

#### 8.2.4.19	CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change

##### 8.2.4.19.1	CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Intra-band Contiguous CA

8.2.4.19.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure, performed the inter-frequency measurement and configured an SCell }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including mobilityControlInfo indicating change of the PCell and sCellToAddModList with an SCell same from configured SCell representing an intra-band contiguous CA }

then { UE sends an RRCConnectionReconfigurationComplete message and does not change SCell }

}

8.2.4.19.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 36.331, clauses 5.3.5.4 and 5.3.10.3b.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	configure lower layers to consider the SCell(s), if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.2.4.19.1.3	Test description

8.2.4.19.1.3.1	Pre-test conditions

System Simulator:

-	Cell 3 is the PCell, Cell 1 is the SCell to be added and Cell 6 is the inter-frequency neighbour cell.

-	Cell 1 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 3 according to [18].

8.2.4.19.1.3.2	Test procedure sequence

Table 8.2.4.19.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.19.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | -97 | The power level values are such that measurement results for Cell 3 (M1) and Cell 1 (M2) do not satisfy entry condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | -79 | The power level values are such that measurement results for Cell 3 (M1) and Cell 6 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.19.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 3 to configure Cell 1 as an SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement reporting for event A3 on Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 6 level according to row "T1" in Table 8.2.4.19.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 3 to report event A3. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to perform inter-frequency handover. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 6? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 6? | - | - | 1 | - |

8.2.4.19.1.3.3	Specific message contents

Table 8.2.4.19.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.19.1.3.3-2, 8.2.4.19.1.3.3-8 and 8.2.4.19.1.3.3-6.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.19.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.19.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.4.19.1.3.3-2: SCellToAddMod-r10 (Table 8.2.4.19.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 1 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 1 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.19.1.3.3-3: RadioResourceConfigCommonSCell-r10 (Table 8.2.4.19.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.19.1.3.3-4: Void

Table 8.2.4.19.1.3.3-5: RRCConnectionReconfiguration (step 3, Table 8.2.4.19.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.19.1.3.3-6: MeasConfig (Table 8.2.4.19.1.3.3-5)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[3] | IdMeasObject-f3 |  |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(f3) |  |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f3 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[3] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.19.1.3.3-6A : ReportConfig-A3 (Table 8.2.4.19.1.3.3-6)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| timeToTrigger | ms2560 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.19.1.3.3-7: MeasurementReport (step 6, Table 8.2.4.19.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 3 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 6 | Cell 6 |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| servFreqId-r10[1] | 1 |  |  |
| measResultSCell-r10[1] SEQUENCE { |  | Cell 1 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| measResultBestNeighCell-r10[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.19.1.3.3-7: RRCConnectionReconfiguration (step 7, Table 8.2.4.19.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.19.1.3.3-8: MobilityControlInfo (Table 8.2.4.19.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 6 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.19.2	CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.19.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 1& Cell 1 replaces Cell 6.

-	Cell 10 is an Inactive SCell according to [18] cl. 6.3.4

8.2.4.19.2.1	Specific message contents

Same as test case 8.2.4.19.1 with the following differences

Table 8.2.4.19.2.1-1: Conditions for specific message contents in Table 8.2.4.19.2.1-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.19.2.1-2: MeasConfig (Table 8.2.4.19.1.3.3-5)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Apply conditions “DS_Meas and LAA SCell” to work out the value of this IE |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[3] | IdMeasObject-f3 |  |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(f3) |  |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f3 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[3] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.19.3	CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.19.1 with the following differences:

-	CA configuration: Intra-band non-contiguous CA replaces Intra-band Contiguous CA.

#### 8.2.4.20	CA / RRC connection reconfiguration / Handover / Success / SCell Change

##### 8.2.4.20.1	CA / RRC connection reconfiguration / Handover / Success / SCell Change / Intra-band Contiguous CA

8.2.4.20.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the inter frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo and sCellToAddModList with an SCell different from configured SCell }

then { UE sends an RRCConnectionReconfigurationComplete message and changes SCell }

}

8.2.4.20.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.10.3b.

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.2.4.20.1.3	Test description

8.2.4.20.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added and Cell 12 is the intra-frequency neighbour cell of Cell 3.

-	Cell 3 is Inactive SCell according to [18] cl. 6.3.4

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	3 E-UTRA Cells with the same PLMN.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.20.1.3.2	Test procedure sequence

Table 8.2.4.20.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.20.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -97 | The power level values are such that measurement results for Cell 12 (M12) and Cell 3 (M2) do not satisfy entry condition for event A6. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | -79 | The power level values are such that measurement results for Cell 12 (M12) and Cell 3 (M3) satisfy entry condition for event A6. |

Table 8.2.4.20.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait 1 s to allow for the switching of cells. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to configure Cell 3 as an SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
|  | EXCEPTION: If UE does not support FGI bit 111, i.e. Event A6, the following step 4 to step 7 except step 6 should not be executed |  |  |  |  |
| 4 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement reporting for event A6 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6 | The SS changes power level according to row "T1" in Table 8.2.4.20.1.3.2-1. | - | - | - | - |
| 7 | The UE transmits a MeasurementReport message on Cell 1 to report event A6. | --> | MeasurementReport | - | - |
| 8 | The SS transmits an RRCConnectionReconfiguration message to perform inter-frequency handover to Cell 12 with original SCell Cell 3 release and new SCell Cell1addition. | <-- | RRCConnectionReconfiguration | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 12? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 10 | The UE transmits a TRACKING AREA UPDATE REQUEST message to update the registration of the actual tracking area. | - | - | - | - |
| 11 | The SS responds with TRACKING AREA UPDATE ACCEPT message. | - | - | - | - |
| 12 | The UE transmits a TRACKING AREA UPDATE COMPLETE. | - | - | - | - |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 12? | - | - | 1 | - |

8.2.4.20.1.3.3	Specific message contents

Table 8.2.4.20.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.20.1.3.3-2, 8.2.4.20.1.3.3-5, 8.2.4.18.1.3.3-5.8.2.4.19.1.3.3-8 and 8.2.4.20.1.3.3-9.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.20.1.3.3-1: RRCConnectionReconfiguration (step 2, Table 8.2.4.20.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.4.20.1.3.3-2: SCellToAddMod-r10 (Table 8.2.4.20.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.20.1.3.3-3: RadioResourceConfigCommonSCell-r10 (Table 8.2.4.20.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-4: RRCConnectionReconfiguration (step 4, Table 8.2.4.20.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.20.1.3.3-5: MeasConfig (Table 8.2.4.20.1.3.3-4)

| Derivation Path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| { |  |  |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A6 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A6 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A6 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-6: MeasurementReport (step 7, Table 8.2.4.20.1.3.2-2)

| Derivation Path: 36.508, clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 | Cell 12 |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| servFreqId-r10[1] | 1 |  |  |
| measResultSCell-r10[1] SEQUENCE { |  | Cell 3 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| measResultBestNeighCell-r10[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-7: RRCConnectionReconfiguration (step 8, Table 8.2.4.20.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-CQIConfig-R10 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| SCellToReleaseList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| SCellToAddModList-r10 | SCellToAddMod-r10-f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-7A: RadioResourceConfigDedicated-CQIConfig-R10 (Table 8.2.4.20.1.3.3-7)

| Derivation Path: 36.508 , condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-CQIConfig-R10 |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-7B: PhysicalConfigDedicated-CQIConfig-R10 (Table 8.2.4.20.1.3.3-7 A)

| Derivation Path: 36.508, Table 4.8.2.1.6-1, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-8: MobilityControlInfo (Table 8.2.4.20.1.3.3-7)

| Derivation Path: clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.20.1.3.3-9: SCellToAddMod-r10-f1 (Table 8.2.4.20.1.3.3-7)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f1 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 1 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.20.1.3.3-10: RadioResourceConfigCommonSCell-r10-f1 (Table 8.2.4.20.1.3.3-9)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |

##### 8.2.4.20.2	CA / RRC connection reconfiguration / Handover / Success / SCell Change / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.20.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3& Cell 30 replaces Cell 12.

-	Cell 10 is Inactive SCell according to [18] cl. 6.3.4.

-	Cell 1, Cell 10 and Cell 30 with the same PLMN.

##### 8.2.4.20.3	CA / RRC connection reconfiguration / Handover / Success / SCell Change Intra-band non-contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.20.1 with the following differences:

-	CA configuration: Intra-band non contiguous CA replaces Intra-band Contiguous CA.

#### 8.2.4.21	CA / RRC connection reconfiguration / Handover / Success / SCell release

##### 8.2.4.21.1	CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band Contiguous CA

8.2.4.21.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure, performed the inter-frequency measurement and configured an SCell }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo and including sCellToReleaseList with an sCellIndex set to the configured SCell }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

8.2.4.21.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4 and 5.3.10.3a.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	configure lower layers to consider the SCell(s), if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in section 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10.3a]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

1>	if the release is triggered by RRC connection re-establishment:

2>	release all SCells that are part of the current UE configuration;

8.2.4.21.1.3	Test description

8.2.4.21.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the SCell to be added and Cell 6 is the inter-frequency neighbour cell of PCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.21.1.3.2	Test procedure sequence

Table 8.2.4.21.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.21.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 6 (M6) do not satisfy entry condition for event A5. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 6 (M6) satisfy entry condition for event A5. |

Table 8.2.4.21.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to configure Cell 3 as an SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement reporting for event A5 on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 6 level according to row "T1" in Table 8.2.4.21.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A5. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to perform inter-frequency handover to Cell 6 and release SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 6? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 6? | - | - | 1 | - |

8.2.4.21.1.3.3	Specific message contents

Table 8.2.4.21.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.21.1.3.3-2, 8.2.4.21.1.3.3-5. and 8.2.4.21.1.3.3-9.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.21.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.21.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.4.21.1.3.3-2: SCellToAddMod-r10 (Table 8.2.4.21.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.3-19D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | 1 |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 3 |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.21.1.3.3-3: RadioResourceConfigCommonSCell-r10 (Table 8.2.4.21.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.21.1.3.3-4: RRCConnectionReconfiguration (step 3, Table 8.2.4.21.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition MEAS

Table 8.2.4.21.1.3.3-5: MeasConfig (Table 8.2.4.21.1.3.3-4)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[3] | IdMeasObject-f3 |  |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(f3) | Cell 6 |  |
| measObject[3] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A5 |  |  |
| reportConfig[1] | ReportConfig-A5 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f3 |  |  |
| reportConfigId[1] | IdReportConfig-A5 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[3] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.21.1.3.3-6: ReportConfig-A5 (Table 8.2.4.21.1.3.3-5)

| Derivation path: 36.508 clause 4.6.6 table ReportConfigEUTRA-A5(-76, -88) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA ::= SEQUENCE { |  |  |  |
| reportAmount | infinity |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| timeToTrigger | ms2560 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.21.1.3.3-7: MeasurementReport (step 6, Table 8.2.4.21.1.3.2-2)

| Derivation Path: 36.508, clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 6 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| servFreqId-r10[1] | 1 |  |  |
| measResultSCell-r10[1] SEQUENCE { |  | Cell 3 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| measResultBestNeighCell-r10[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.21.1.3.3-8: RRCConnectionReconfiguration (step 7, Table 8.2.4.21.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measIdToRemoveList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| SCellToReleaseList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |
| SCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |
| SCellToAddModList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.21.1.3.3-9: MobilityControlInfo (Table 8.2.4.21.1.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 6 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 6 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.21.2	CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.21.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration:

-	Cell 10 replaces Cell 3, and then

-	If f6 is supported as per TS 36.508 Table 6.2.3.2-2 by the band combination in which the test case is executed, then Cell 28 replaces Cell 6; otherwise Cell 3 replaces Cell 6. When Cell 28 is used its PLMN shall be set to PLMN 1.

##### 8.2.4.21.3	CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.21.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

-	Cells configuration:

#### 8.2.4.22	Void

#### 8.2.4.23	CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful

##### 8.2.4.23.1	CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band Contiguous CA

8.2.4.23.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA cell and including sCellToReleaseList with an sCellIndex set to the configured SCell }

ensure that {
  when { UE detects handover failure and the source PCell is selectable }

then { UE successfully performs an RRC connection re-establishment procedure on source PCell }

}

(2)

with { UE having completed the radio bearer establishment and initial security activation procedure and after receiving an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA cell and including sCellToReleaseList with an sCellIndex set to the configured SCell }

ensure that {
  when { UE detects handover failure and the initial SCell is selectable }

then { UE successfully performs an RRC connection re-establishment procedure on original SCell and the original SCell becomes the PCell }

}

8.2.4.23.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.1.3, 5.3.5.4, 5.3.5.6, 5.3.7.2, 5.3.7.4, 5.3.7.5, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.1.3]

…

The source eNB should, for some time, maintain a context to enable the UE to return in case of handover failure. After having detected handover failure, the UE attempts to resume the RRC connection either in the source PCell or in another cell using the RRC re-establishment procedure. This connection resumption succeeds only if the accessed cell is prepared, i.e. concerns a cell of the source eNB or of another eNB towards which handover preparation has been performed. The cell in which the re-establishment procedure succeeds becomes the PCell while SCells and STAGs, if configured, are released..

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.5.6]

The UE shall:

1>	if T304 expires (handover failure):

NOTE:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source cell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon handover failure, in accordance with 5.3.5.6; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.4]

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source cell (handover and mobility from  E-UTRA failure) or used in the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source cell (handover and mobility from E-UTRA failure) or of the cell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3>	set the reestablishmentCause to the value 'reconfigurationFailure';

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value 'handoverFailure';

...

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	stop timer T301;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1> submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.23.1.3	Test description

8.2.4.23.1.3.1.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell and Cell 3 is the SCell

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.23.1.3.2	Test procedure sequence

Table 8.2.4.23.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.23.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | The power level values are assigned to satisfy SrxlevCell 3 < 0 such that selecting Cell 1 is guaranteed (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3> M1). (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | The power level values are assigned to satisfy SrxlevCell 1 < 0 such that selecting Cell 3 is guaranteed. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.23.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to configure Cell 3 as an SCell. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup event A3 reporting configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.23.1.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3 and to release SCell Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.2.4.23.1.3.2-3 should take place. | - | - | - | - |
| 8 | The SS changes Cell 1 and Cell 3 parameters according to the row "T2" in table 8.2.4.23.1.3.2-1. | - | - | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 1? | --> | RRCConnectionReestablishmentRequest | 1 | P |
| 10 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 11 | The UE transmits an RRCConnectionReestablishmentComplete message | --> | RRCConnectionReestablishmentComplete | - | = |
| 12 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 1 and configure Cell 3 as an SCell | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 | --> | RRCConnectionReconfigurationtComplete | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup event A3 reporting configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 16 | The SS changes Cell 1 and Cell 3 parameters according to the row "T3" in table 8.2.4.23.1.3.2-1. | - | - | - | - |
| 17 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 18 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform handover to Cell 3 and to release SCell Cell3. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 19 the steps specified in Table 8.2.4.23.1.3.2-3 should take place. | - | - | - | - |
| 19 | The SS changes Cell 1 and Cell 3 parameters according to the row "T4" in table 8.2.4.23.1.3.2-1. | - | - | - | - |
| 20 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 3? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 21 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 3. | <-- | RRCConnectionReestablishment | - | - |
| 22 | The UE transmits an RRCConnectionReestablishmentComplete message | --> | RRCConnectionReestablishmentComplete | - | - |
| 23 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 24 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationtComplete | - | - |

Table 8.2.4.23.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on Cell 3. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.23.1.3.3	Specific message contents

Table 8.2.4.23.1.3.3-00: Conditions for specific message contents
in Tables 8.2.4.23.1.3.3-2, 8.2.4.23.1.3.3-5 and 8.2.4.23.1.3.3-9.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.23.1.3.3-0: SystemInformationBlockType2 for Cell 1 and cell 3 (preamble and all the steps in Table 8.2.4.23.3.2-2)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-1: RRCConnectionReconfiguration (steps 1 and 12, Table 8.2.4.23.1.3.2-2)

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 8.2.4.23.1.3.3-2: SCellToAddMod-r10-f2 (Table 8.2.4.23.1.3.3-1)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 3 |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f2 |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 3 |  | Band > 64 |
| } |  |  |  |

Table 8.2.4.23.1.3.3-3: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.4.23.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-4: RRCConnectionReconfiguration (steps 3 and 14, Table 8.2.4.23.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.23.1.3.3-5: MeasConfig (Table 8.2.4.23.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfig-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-6: ReportConfig-A3 (Table 8.2.4.18.1.3.3-5)

| Derivation path: 36.508, Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| timeToTrigger | ms2560 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-7: MeasurementReport (steps 6 and 17, Table 8.2.4.23.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | physicalCellId of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |
| servFreqId-r10 | 1 |  |  |
| measResultSCell-r10 SEQUENCE { |  | Cell 3 |  |
| rsrpResultSCell-r10 | (0..97) |  |  |
| rsrqResultSCell-r10 | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-8: RRCConnectionReconfiguration (steps 7 and 18, Table 8.2.4.23.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |
| --- | --- |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |
| criticalExtensions CHOICE { |  |
| c1 CHOICE{ |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |
| nonCriticalExtension SEQUENCE { |  |
| lateNonCriticalExtension |  |
| nonCriticalExtension SEQUENCE { |  |
| nonCriticalExtension SEQUENCE { |  |
| SCellToReleaseList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |
| SCellIndex-r10[1] | 1 |
| } |  |
| SCellToAddModList-r10 | Not present |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Table 8.2.4.23.1.3.3-9: MobilityControlInfo (Table 8.2.4.23.1.3.3-8)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-10: RRCConnectionReestablishmentRequest (steps 9 and 20, Table 8.2.4.23.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.23.1.3.3-11: RRCConnectionReestablishment (steps 10 and 21, Table 8.2.4.23.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.23.2	CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band CA

The scope and description of the present TC is the same as test case 8.2.4.23.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: Cell 10 replaces Cell 3.

-	Cell 10 is Inactive SCells according to [18] cl. 6.3.4

##### 8.2.4.23.3	CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 8.2.4.23.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

#### 8.2.4.24	Void

##### 8.2.4.24.1	Void

#### 8.2.4.25	Dual Connectivity / RRC connection reconfiguration / Handover

##### 8.2.4.25.1	RRC connection reconfiguration / Intra-MeNB Handover / MCG DRB to MCG DRB and MCG DRB to/from SCG DRB

8.2.4.25.1.1	Test Purpose (TP)

(1)

with { UE in connected mode with SCG activated with a MCG DRB and a SCG DRB established }

ensure that {

when { the UE receives a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency }
  then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in connected mode with SCG activated has performed handover to another PCell keeping the UE context for the MCG DRB and SCG DRB }
ensure that {
  when { uplink data are submitted for transmission on the MCG/SCG DRBs }
  then { the UE transmits the uplink data on the radio bearer associated with the MCG/SCG DRBs respectively }
            }

(3)

with { UE in connected mode with SCG activated with a MCG DRB and a SCG DRB established }

ensure that {

when { the UE receives a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency and to change of  one of the MCG DRBs to a SCG DRB }
  then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(4)

with { UE in connected mode with SCG activated has performed handover to another PCell and has re-configured a MCG DRB to a SCG DRB }
ensure that {
  when { uplink data are submitted for transmission on the MCG/SCG DRBs }
  then { the UE transmits the uplink data on the radio bearer associated with the MCG/SCG DRBs respectively }
            }

(5)

with { UE in connected mode with SCG activated with a MCG DRB and two SCG DRBs established }

ensure that {

when { the UE receives a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency and to change of one of the SCG DRBs to a MCG DRB }
  then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(6)

with { UE in connected mode with SCG activated has performed handover to another PCell and has re-configured a SCG DRB to a MCG DRB }
ensure that {
  when { uplink data are submitted for transmission on the MCG/SCG DRBs }
  then { the UE transmits the uplink data on the radio bearer associated with the MCG/SCG DRBs respectively }
            }

8.2.4.25.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.3a1,   5.3.10.12. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

…

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

2>	if drb-ToAddModList is received and includes the drb-Identity value (i.e. add split DRB):

3>	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

3>	establish an MCG RLC entity and an MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

3>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	else (i.e. add SCG DRB):

3>	establish a PDCP entity and configure it with the current SCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModListSCG;

3>	establish an SCG RLC entity or entities and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2>	if the DRB indicated by drb-Identity is a split DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. split to MCG):

4>	release the SCG RLC entity and the SCG DTCH logical channel;

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

3>	else (i.e. reconfigure split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2>	if the DRB indicated by drb-Identity is an SCG DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. SCG to MCG):

4>	reconfigure the PDCP entity with the current MCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity or entities and the SCG DTCH logical channel to be an MCG RLC entity or entities and an MCG DTCH logical channel;

4>	reconfigure the MCG RLC entity or entities and/ or the MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig, if included in drb-ToAddModList;

3>	else (i.e. drb-ToAddModListSCG is received and includes the drb-Identity value i.e. reconfigure SCG):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2>	if the DRB indicated by drb-Identity is an MCG DRB:

3>	if drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to split (i.e. MCG to split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, included in drb-ToAddModListSCG;

3>	else (i.e. drb-Type is included and set to scg i.e. MCG to SCG):

4>	reconfigure the PDCP entity with the current SCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the MCG RLC entity or entities and the MCG DTCH logical channel to be an SCG RLC entity or entities and an SCG DTCH logical channel;

4>	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

[TS 36.331, clause 5.3.10.12]

The UE shall:

1>	for each split or SCG DRBs that is part of the current configuration:

2>	if the corresponding drb-Identity value is included in the received drb-ToAddModList; and:

2>	if the corresponding drb-Identity value is not included in the received drb-ToAddModListSCG (i.e. reconfigure split, split to MCG or SCG to MCG):

3>	perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1;

8.2.4.25.1.3	Test description

8.2.4.25.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the interfrequency neighbour cell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 cell 3 and Cell 10.

UE:

Preamble:

-	The UE is in state DC MCG/SCG DRB Loopback Activated (state 6A) on Cell 1 and Cell 10 according to [18], UE has 3 DRBs configured. DRB 1(Default bearer) and DRB 2(dedicated bearer) as MCG DRB’s and DRB 3 (dedicated bearer) as SCG bearer.

8.2.4.25.1.3.2	Test procedure sequence

Table 8.2.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1, T2" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.25.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -73 | -97 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -73 | -85 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.25.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.6.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | The SS transmits one IP packet to the UE on each of DRB 1 and DRB 2 on Cell 3, DRB 3 on Cell 10 (PSCell). | <-- | IP packets | - | - |
| 8 | Check: Does the UE loops back the IP packet on each of DRB 1 and DRB 2 on Cell 3, DRB 3 on Cell 10 (PSCell)? | --> | IP packets | 2 | P |
| 8A | The SS transmits an RRCConnectionReconfiguration message on Cell 3 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 8B | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS changes Cell 1 and Cell 3 parameters according to the row "T2" in table 8.2.4.6.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message on Cell 3 to report event A3 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message on Cell 3 to order the UE to perform inter frequency handover to Cell 1 and to change of one of the MCG DRBs to a SCG DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 13 | The SS transmits one IP packet to the UE on each of DRB 1 on Cell 1, DRB 2 and DRB 3 on Cell 10 (PSCell). | <-- | IP packets | - | - |
| 14 | Check: Does the UE loops back the IP packet on each of DRB 1 on Cell 1, DRB 2 and DRB 3 on Cell 10 (PSCell)? | --> | IP packets | 4 | P |
| 14A | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 14B | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 15 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.6.3.2-1. | - | - | - | - |
| 16 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 17 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3 and to change of one of the SCG DRBs to a MCG DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 18 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 5 | P |
| 19 | The SS transmits one IP packet to the UE on each of DRB 1 and DRB 2 on Cell 3, DRB 3 on Cell 10 (PSCell). | <-- | IP packets | - | - |
| 20 | Check: Does the UE loops back the IP packet on each of DRB 1 and DRB 2 on Cell 3, DRB 3 on Cell 10 (PSCell)? | --> | IP packets | 6 | P |

8.2.4.25.1.3.3	Specific message contents

Table 8.2.4.25.1.3.3-0: Conditions for specific message contents
in Tables 8.2.4.25.1.3.3-2, 8.2.4.25.1.3.3-5, 8.2.4.25.1.3.3-7 and 8.2.4.25.1.3.3-10.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.1.3.3-1: RRCConnectionReconfiguration (step 1, 8A, 14A, Table 8.2.4.25.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.25.1.3.3-2: MeasConfig (step 1/14A, Table 8.2.4.25.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-2A: MeasConfig (step 8A, Table 8.2.4.25.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-3: MeasurementReport (step 4, 16, Table 8.2.4.25.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-3A: MeasurementReport (step 10, Table 8.2.4.25.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.25.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.25.1.3.3-5: MobilityControlInfo (Table 8.2.4.25.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-6: RRCConnectionReconfiguration (step 11, Table 8.2.4.25.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO and tbd |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig | Not present |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-MCGToSCG |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.4.25.1.3.3-7: MobilityControlInfo (Table 8.2.4.25.1.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-8: SCG-Configuration-r12-MCGToSCG(Table 8.2.4.25.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3-19F conditon PSCell_Add_SCG_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 SEQUENCE { |  |  |  |
| scg-Counter-r12 | Not Present |  |  |
| powerCoordinationInfo-r12 | Not Present |  |  |
| } |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |
| DRB-ToAddModSCG-r12[1] SEQUENCE { |  |  |  |
| drb-Identity-r12 | Same as DRB 2 established as MCG bearer in preamble |  |  |
| drb-Type-r12 CHOICE { |  |  |  |
| scg-r12 SEQUENCE { |  |  |  |
| eps-BearerIdentity-r12 | Not Present |  |  |
| pdcp-Config-r12 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| rlc-ConfigSCG-r12 | Not Present |  |  |
| rlc-Config-v1250 | Not present |  |  |
| logicalChannelIdentitySCG-r12 | 4 |  |  |
| logicalChannelConfigSCG-r12 | LogicalChannelConfig-DRB using condition LO |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-MainConfigSCG-r12 | Not present |  |  |
| rlf-TimersAndConstantsSCG-r12 | Not present |  |  |
| } |  |  |  |
| sCellToReleaseListSCG-r12 | Not Present |  |  |
| pSCellToAddMod-r12 | Not Present |  |  |
| sCellToAddModListSCG-r12 | Not Present |  |  |
| mobilityControlInfoSCG-r12 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-9: RRCConnectionReconfiguration (step 17, Table 8.2.4.25.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |
| measConfig |  | Not present |  |  |  |
| mobilityControlInfo |  | MobilityControlInfo |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 8.2.4.25.1.3.3-10: MobilityControlInfo (Table 8.2.4.25.1.3.3-9)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-11: RadioResourceConfigDedicated (Table 8.2.4.25.1.3.3-9)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.1.3.3-12: DRB-ToAddMod-DEFAULT(Table 8.2.4.25.1.3.3-11)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | Same as DRB 2 established as MCG bearer in preamble |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Same as DRB 2 established as MCG bearer in preamble |  |  |
| logicalChannelConfig | Not Present |  |  |
| drb-TypeChange-r12 | toMCG |  |  |
| } |  |  |  |

##### 8.2.4.25.2	RRC connection reconfiguration / Intra-MeNB Handover / MCG DRBs to/from Split DRB

8.2.4.25.2.1	Test Purpose (TP)

(1)

with { UE in connected mode }

ensure that {

when { the UE receives a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency and to change from the MCG DRB to a Split DRB }
  then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in connected mode with SCG activated has performed handover to another PCell and has re-configured a MCG DRB to a Split DRB }
ensure that {
  when { uplink data are submitted for transmission on the MCG/Split DRBs }
  then { the UE transmits the uplink data on the radio bearer associated with the MCG/Split DRBs respectively }
            }

(3)

with { UE in connected mode with SCG activated with a MCG DRB split DRB established }

ensure that {

when { the UE receives a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency and to change from the Split DRB to a MCG DRB }
  then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(4)

with { UE in connected mode with SCG activated has performed handover to another PCell and has re-configured a Split DRB to a MCG/SCG DRB }
ensure that {
  when { uplink data are submitted for transmission on the MCG/Split DRBs }
  then { the UE transmits the uplink data on the radio bearer associated with the MCG/Split DRBs respectively }
            }

8.2.4.25.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.3a1, 5.3.10.12. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

…

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

2>	if drb-ToAddModList is received and includes the drb-Identity value (i.e. add split DRB):

3>	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

3>	establish an MCG RLC entity and an MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

3>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	else (i.e. add SCG DRB):

3>	establish a PDCP entity and configure it with the current SCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModListSCG;

3>	establish an SCG RLC entity or entities and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2>	if the DRB indicated by drb-Identity is a split DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. split to MCG):

4>	release the SCG RLC entity and the SCG DTCH logical channel;

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

3>	else (i.e. reconfigure split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2>	if the DRB indicated by drb-Identity is an SCG DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. SCG to MCG):

4>	reconfigure the PDCP entity with the current MCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity or entities and the SCG DTCH logical channel to be an MCG RLC entity or entities and an MCG DTCH logical channel;

4>	reconfigure the MCG RLC entity or entities and/ or the MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig, if included in drb-ToAddModList;

3>	else (i.e. drb-ToAddModListSCG is received and includes the drb-Identity value i.e. reconfigure SCG):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2>	if the DRB indicated by drb-Identity is an MCG DRB:

3>	if drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to split (i.e. MCG to split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, included in drb-ToAddModListSCG;

3>	else (i.e. drb-Type is included and set to scg i.e. MCG to SCG):

4>	reconfigure the PDCP entity with the current SCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the MCG RLC entity or entities and the MCG DTCH logical channel to be an SCG RLC entity or entities and an SCG DTCH logical channel;

4>	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

[TS 36.331, clause 5.3.10.12]

The UE shall:

1>	for each split or SCG DRBs that is part of the current configuration:

2>	if the corresponding drb-Identity value is included in the received drb-ToAddModList; and:

2>	if the corresponding drb-Identity value is not included in the received drb-ToAddModListSCG (i.e. reconfigure split, split to MCG or SCG to MCG):

3>	perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1;

8.2.4.25.2.3	Test description

8.2.4.25.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the interfrequency neighbour cell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1 cell 3 and Cell 10.

UE:

Preamble:

-	The UE is in state DC Loopback Activated (state 6) on Cell 1 according to [18], UE has 2 DRBs configured. DRB 1(Default bearer) and DRB 2(dedicated bearer) as MCG DRB.

8.2.4.25.2.3.2	Test procedure sequence

Table 8.2.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1, T2" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.25.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -73 | -97 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -73 | -85 | -75 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.25.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.25.2.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3 and to change from the MCG DRB to a Split DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | The SS transmits one IP packet to the UE on Cell 3 on each 2 DRB’s. | <-- | IP packets | - | - |
| 8 | Check: Does the UE loop back the IP packet on Cell 3 on each of 2 DRB’s? | (NOTE)--> | IP packets | 2 | P |
| 8A | The SS transmits an RRCConnectionReconfiguration message on Cell 3 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 8B | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS changes Cell 1 and Cell 3 parameters according to the row "T2" in table 8.2.4.25.2.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message on Cell 3 to report event A3 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message on Cell 3 to order the UE to perform inter frequency handover to Cell 1 and to change from the Split DRB to a MCG DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 13 | The SS transmits one IP packet to the UE on Cell 1 on all each 2 DRB’s. | <-- | IP packets | - | - |
| 14 | Check: Does the UE loop back the IP packet on Cell1 on each of 2 DRB’s? | --> | IP packets | 4 | P |
| NOTE: The UE loops back the IP packet via Cell 10 (PSCell) at PHY/MAC/RLC level, which is then routed within the SS to the PCell’s PDCP entity. UE’s lower layer transmission on Cell 10 (PSCell) is implicitly tested by assigning an UL Grant only on Cell 10. |  |  |  |  |  |

8.2.4.25.2.3.3	Specific message contents

Table 8.2.4.25.2.3.3-0: Conditions for specific message contents
in Tables 8.2.4.25.2.3.3-2, 8.2.4.25.2.3.3-5 and 8.2.4.25.2.3.3-7.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.2.3.3-1: RRCConnectionReconfiguration (step 1, step 8A, Table 8.2.4.25.2.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.25.2.3.3-2: MeasConfig (step 1, Table 8.2.4.25.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-2A: MeasConfig (step 8A, Table 8.2.4.25.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-3: MeasurementReport (step 4, Table 8.2.4.25.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-3A: MeasurementReport (step 10, Table 8.2.4.25.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.25.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO and PSCell_Add_Split_DRB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig | Not present |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.4.25.2.3.3-5: MobilityControlInfo (Table 8.2.4.25.2.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-5a: SCG-Configuration-r12 (Table 8.2.4.25.2.3.3-4)

| Derivation Path: 36.508 Table 4.6.3-19F conditon PSCell_Add_Split_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |
| DRB-ToAddModSCG-r12[1] SEQUENCE { |  |  |  |
| drb-Identity-r12 | Same as DRB 2 established as MCG bearer in preamble |  |  |
| drb-Type-r12 CHOICE { |  |  |  |
| split-r12 | NULL |  |  |
| } |  |  |  |
| rlc-ConfigSCG-r12 | RLC-Config-DRB-AM |  |  |
| rlc-Config-v1250 | Not present |  |  |
| logicalChannelIdentitySCG-r12 | 4 | drb-Identity-r12+2 |  |
| logicalChannelConfigSCG-r12 | LogicalChannelConfig-DRB using condition LO |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-MainConfigSCG-r12 | Not present |  |  |
| rlf-TimersAndConstantsSCG-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-6: RRCConnectionReconfiguration (step 11, Table 8.2.4.25.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig | Not present |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.4.25.2.3.3-7: MobilityControlInfo (Table 8.2.4.25.2.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-8: RadioResourceConfigDedicated (Table 8.2.4.25.2.3.3-6)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.2.3.3-9: DRB-ToAddMod-DEFAULT(Table 8.2.4.25.2.3.3-8)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | Same as DRB 2 established as MCG bearer in preamble |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Same as DRB 2 established as MCG bearer in preamble |  |  |
| logicalChannelConfig | Not Present |  |  |
| drb-TypeChange-r12 | toMCG |  |  |
| } |  |  |  |

##### 8.2.4.25.3	RRC connection reconfiguration / Intra-MeNB Handover / Split DRB to Split DRB

8.2.4.25.3.1	Test Purpose (TP)

(1)

With { UE in connected mode with SCG activated with a Split DRB established }

ensure that {

when { the UE is receiving a RRCConnectionReconfiguration message including mobilityControlInfo indicating a different E-UTRA frequency for the PCell and keeping the current PSCell and the Split DRB }

then { the UE responds with a RRCConnectionReconfigurationComplete message }

}

(2)

With { UE in connected mode with SCG activated has performed handover to another PCell keeping the PSCell and the Split DRB }
ensure that {
  when { uplink data are submitted for transmission on the Split DRB }
then { the UE transmits the uplink data on the radio bearer associated with the Split DRB }
            }

8.2.4.25.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.10 and 5.3.10.12. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	…;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MCG MAC and SCG MAC, if configured;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

1>	configure lower layers to consider the SCell(s) other than the PSCell, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	…;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	…;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the KASME key taken into use with the latest successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	if connected as an RN:

3>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if connected as an RN:

2>	…;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	…;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	…;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.10.9;

1>	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2>	…;

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	…;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	…;

2>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

3>	…;

2>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	…;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	if the UE is configured to provide IDC indications:

3>	…:

2>	if the UE is configured to provide power preference indications:

3>	…:

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	…;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including commRxInterestedFreq or commTxResourceReq during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including discRxInterest or discTxResourceReq during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

3>	…;

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10.10]

The UE shall:

1>	if the received scg-Configuration is set to release or includes the mobilityControlInfoSCG (i.e. SCG release/ change):

2>	if mobilityControlInfo is not received (i.e. SCG release/ change without HO):

3>	…;

1>	if the received scg-Configuration is set to release:

2>	…;

1>	else:

2>	if the received scg-ConfigPartMCG includes the scg-Counter:

3>	update the S-KeNB key based on the KeNB key and using the received scg-Counter value, as specified in TS 33.401 [32];

3>	derive the KUPenc key associated with the cipheringAlgorithmSCG included in mobilityControlInfoSCG within the received scg-ConfigPartSCG, as specified in TS 33.401 [32];

3>	configure lower layers to apply the ciphering algorithm and the KUPenc key;

2>	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2>	if the received scg-ConfigPartSCG includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

2>	if the received scg-ConfigPartSCG includes the sCellToReleaseListSCG:

3>	…;

2>	if the received scg-ConfigPartSCG includes the sCellToAddModListSCG:

3>	perform SCell addition or modification as specified in 5.3.10.3b;

2>	configure lower layers in accordance with mobilityControlInfoSCG, if received;

2>	if the received scg-ConfigPartSCG includes the mobilityControlInfoSCG (i.e. SCG change):

3> …;

NOTE 2:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

[TS 36.331, clause 5.3.10.12]

The UE shall:

1>	for each split or SCG DRBs that is part of the current configuration:

2>	if the corresponding drb-Identity value is included in the received drb-ToAddModList; and:

2>	if the corresponding drb-Identity value is not included in the received drb-ToAddModListSCG (i.e. reconfigure split, split to MCG or SCG to MCG):

3>	perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1;

8.2.4.25.3.3	Test description

8.2.4.25.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the source PCell, Cell 3 is the target PCell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 3 and Cell 10.

UE:

Preamble:

-	The UE is in state DC Split DRB Loopback Activated (state 6B) on Cell 1 and Cell 10 according to [18]

8.2.4.25.3.3.2	Test procedure sequence

Table 8.2.4.25.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo (handover). | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The SS transmits one IP packet to the UE on Cell 3 on the DRB associated with the Split DRB. | <-- | IP packet | - | - |
| 4 | Check: Does the UE loop back the IP packet on Cell 3 on the Split DRB (NOTE)? | --> | IP packet | 2 | P |
| NOTE: The UE loops back the IP packet via Cell 10 (PSCell) at PHY/MAC/RLC level, which is then routed within the SS to the PCell’s PDCP entity. UE’s lower layer transmission on Cell 10 (PSCell) is implicitly tested by assigning an UL Grant only on Cell 10 |  |  |  |  |  |

8.2.4.25.3.3.3	Specific message contents

Table 8.2.4.25.3.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.25.3.3.2-1)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

##### 8.2.4.25.4	RRC connection reconfiguration / Handover with SCG release / MCG/SCG DRBs to MCG DRB

8.2.4.25.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with SCG activated with a MCG DRB and a SCG DRB established }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with release }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { uplink data are submitted for transmission on the radio bearer of the target PCell UE }

then { UE transmits the uplink data on the radio bearer associated with the dedicated EPS bearer context }

}

8.2.4.25.4.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.4 and 5.3.10.10.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.10]

1>	if the received scg-Configuration is set to release:

2>	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

…

2>	stop timer T313, if running;

2>	stop timer T307, if running;

8.2.4.25.4.3	Test Description

8.2.4.25.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the target PCell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 3 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state DC MCG/SCG DRB Loopback Activation (to state 6A).

8.2.4.25.4.3.2	Test procedure sequence

Table 8.2.4.25.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with release. RRCConnectionReconfiguration message contains the DEACTIVATE EPS BEARER CONTEXT REQUEST message | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | The UE transmits an ULInformationTransfer message containing a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | ULInformationTransfer | - | - |
| 4 | The SS transmits one IP packet to the UE on Cell 3. | <-- | IP packet | - | - |
| 5 | Check: Does the UE loops back the IP packet on Cell 3? | --> | IP packet | 2 | P |

8.2.4.25.4.3.3	Specific message contents

Table 8.2.4.25.4.3.3-0: Conditions for specific message contents
in Table 8.2.4.25.4.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.25.4.3.2-1)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  |  |  |
| drb-Identity | 3 |  |  |
| ‘ } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.4.3.3-2: MobilityControlInfo (Table 8.2.4.25.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.25.5	RRC connection reconfiguration / Handover with SCG release / Split DRB to MCG DRB

8.2.4.25.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with SCG activated with a with a Split DRB established }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with release }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { uplink data are submitted for transmission on the radio bearer of the target PCell UE }

then { UE transmits the uplink data on the radio bearer associated with the dedicated EPS bearer context }

}

8.2.4.25.5.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.3a1 and 5.3.10.10.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

…

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2>	if the DRB indicated by drb-Identity is a split DRB:

3>	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. split to MCG):

4>	release the SCG RLC entity and the SCG DTCH logical channel;

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

[TS 36.331, clause 5.3.10.10]

1>	if the received scg-Configuration is set to release:

2>	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

…

2>	stop timer T313, if running;

2>	stop timer T307, if running;

8.2.4.25.5.3	Test Description

8.2.4.25.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the target PCell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 3 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state DC Split DRB Loopback Activation (to state 6B).

8.2.4.25.5.3.2	Test procedure sequence

Table 8.2.4.25.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with release. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits one IP packet to the UE on Cell 3. | <-- | IP packet | - | - |
| 5 | Check: Does the UE loops back the IP packet on Cell 3? | --> | IP packet | 2 | P |

8.2.4.25.5.3.3	Specific message contents

Table 8.2.4.25.5.3.3-0: Conditions for specific message contents
in Table 8.2.4.25.5.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.25.5.3.2-1)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.5.3.3-2: MobilityControlInfo (Table 8.2.4.25.5.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.5.3.3-3: RadioResourceConfigDedicated-HO (Table 8.2.4.25.5.3.3-1)

| Derivation Path: 36.331 clause 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| DRB-ToAddModList ::= SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  |  |  |
| eps-BearerIdentity | Not present |  |  |
| drb-Identity | 2 |  |  |
| pdcp-Config | Not present |  |  |
| rlc-Config | Not present |  |  |
| logicalChannelIdentity | Not present |  |  |
| logicalChannelConfig | Not present |  |  |
| drb-TypeChange-r12 | toMCG |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.25.6	RRC connection reconfiguration / Handover with SCG reconfiguration / SCG DRB to SCG DRB

8.2.4.25.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with SCG activated with a MCG DRB and a SCG DRB established }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with scg-r12 }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { uplink data are submitted for transmission on the radio bearer of the target PCell UE }

then { UE transmits the uplink data on the radio bearer associated with the dedicated EPS bearer context }

}

8.2.4.25.6.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.10 and 5.3.10.3a1.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.10]

1>	else:

…

2>	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

…

2>	if the received scg-ConfigPartSCG includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

…

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

…

2>	if the DRB indicated by drb-Identity is an SCG DRB:

…

3>	else (i.e. drb-ToAddModListSCG is received and includes the drb-Identity value i.e. reconfigure SCG):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4>	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

8.2.4.25.6.3	Test Description

8.2.4.25.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the target PCell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 3 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state DC MCG/SCG DRB Loopback Activation (to state 6A).

8.2.4.25.6.3.2	Test procedure sequence

Table 8.2.4.25.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with scg-r12. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits one IP packet to the UE on Cell 3. | <-- | IP packet | - | - |
| 5 | Check: Does the UE loops back the IP packet on Cell 3? | --> | IP packet | 2 | P |

8.2.4.25.6.3.3	Specific message contents

Table 8.2.4.25.6.3.3-0: Conditions for specific message contents
in Table 8.2.4.25.6.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.25.6.3.2-1)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 CHOICE { |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |
| drb-Identity-r12 | 3 |  |  |
| drb-Type-r12 CHOICE { |  |  |  |
| scg-r12 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.6.3.3-2: MobilityControlInfo (Table 8.2.4.25.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.25.7	RRC connection reconfiguration / Handover with SCG reconfiguration / Split DRB to Split DRB

8.2.4.25.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with SCG activated with a with a Split DRB established }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with split-r12 }

then { UE sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state}

ensure that {

when { uplink data are submitted for transmission on the radio bearer of the target PCell UE }

then { UE transmits the uplink data on the radio bearer associated with the dedicated EPS bearer context }

}

8.2.4.25.7.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.10 and 5.3.10.3a1.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.10]

1>	else:

…

2>	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

…

2>	if the received scg-ConfigPartSCG includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

[TS 36.331, clause 5.3.10.3a1]

For the drb-Identity value for which this procedure is initiated, the UE shall:

…

1>	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2>	if the DRB indicated by drb-Identity is a split DRB:

…

3>	else (i.e. reconfigure split):

4>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4>	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4>	reconfigure the SCG RLC entity and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

8.2.4.25.7.3	Test Description

8.2.4.25.7.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell 3 is the target PCell and Cell 10 is the PSCell.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1, Cell 3 and Cell 10.

UE:

None.

Preamble:

-	The UE is in state DC Split DRB Loopback Activation (to state 6B).

8.2.4.25.7.3.2	Test procedure sequence

Table 8.2.4.25.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing mobilityControlInfo and SCG-Configuration-r12 with split-r12. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits one IP packet to the UE on Cell 3 on the DRB associated with the Split DRB. | <-- | IP packet | - | - |
| 5 | Check: Does the UE loops back the IP packet on Cell 3? (NOTE) | --> | IP packet | 2 | P |
| NOTE: The UE loops back the IP packet via Cell 10 (PSCell) at PHY/MAC/RLC level, which is then routed within the SS to the PCell’s PDCP entity. UE’s lower layer transmission on Cell 10 (PSCell) is implicitly tested by assigning an UL Grant only on Cell 10. |  |  |  |  |  |

8.2.4.25.7.3.3: Specific message contents

Table 8.2.4.25.7.3.3-0: Conditions for specific message contents
in Table 8.2.4.25.7.3.3-2.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.25.7.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.25.7.3.2-1)

| Derivation Path: 36.508, Clause 4.6.1, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 CHOICE { |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |
| drb-Identity-r12 | 2 |  |  |
| drb-Type-r12 CHOICE { |  |  |  |
| split-r12 | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.25.7.3.3-2: MobilityControlInfo (Table 8.2.4.25.7.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.26	eIMTA / RRC connection reconfiguration / Handover / Success

##### 8.2.4.26.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA frequency}

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

##### 8.2.4.26.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

##### 8.2.4.26.3	Test description

8.2.4.26.3.1	Pre-test conditions

System Simulator:

-	Cell 1(TDD) is not supported eIMTA and Cell 3(TDD) is supported eIMTA.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.26.3.2	Test procedure sequence

Table 8.2.4.26.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.26.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.26.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.26.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 3? | - | - | 1 | - |

8.2.4.26.3.3	Specific message contents

Table 8.2.4.26.3.3-0: Conditions for specific message contents
in Tables 8.2.4.26.3.3-2 and 8.2.4.26.3.3-5.

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.26.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.26.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.26.3.3-2: MeasConfig (Table 8.2.4.26.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.26.3.3-3: MeasurementReport (step 4, Table 8.2.4.26.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.26.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.26.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.26.3.3-5: MobilityControlInfo (Table 8.2.4.26.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.26.3.3-6: RadioResourceConfigDedicated-HO (Table 8.2.4.26.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- eIMTA |  |  |
| } |  |  |  |

Table 8.2.4.26.3.3-7: PhysicalConfigDedicated-eIMTA (Table 8.2.4.26.3.3-6)

Derivation Path: 36.508 table 4.8.2.1.6-1A ,condition eIMTA

#### 8.2.4.27	RRC connection reconfiguration / Handover / Success / Intra-frequency in Enhanced Coverage

8.2.4.27.1	Test Purpose (TP)

(1)

Void

(2)

with { UE supporting CE Mode A in E-UTRA RRC_CONNECTED state supports eventA3 for intra-frequency neighbouring cells and measurement configured for event A3 with event based periodical reporting }

ensure that {

when { Neighbour becomes offset better than serving }

then { UE sends MeasurementReport with correct measId for event A3 }

}

(3)

with { UE supporting CE Mode A in E-UTRA RRC_CONNECTED state supports intra-frequency handover   and UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {

when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.27.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.306, clauses 4.3.29.1, 4.3.29.3, 4.3.29.5.

[TS 36.306, clause 4.3.29.1]

ce-ModeA-r13

This field defines whether the UE supports operation in coverage enhancement mode A, as specified in TS 36.211 [17], TS 36.213 [22] and TS 36.331 [5], and PRACH CE levels 0 and 1 at Random Access, as specified in TS 36.321 [4]. It is mandatory for UEs of DL category M1 and UL category M1.

[TS 36.306, clause 4.3.29.3]

intraFreqA3-CE-ModeA-r13

This field defines whether the UE when operating in CE Mode A supports eventA3 for intra-frequency neighbouring cells in normal coverage and CE Mode A, as specified in TS 36.331 [5] and TS 36.133 [16]. It is mandatory for UEs of this release if ce-ModeA-r13 is supported.

[TS 36.306, clause 4.3.29.5]

intraFreqHO-CE-ModeA-r13

This field defines whether the UE when operating in CE Mode A supports intra-frequency handover to target cell in normal coverage and CE Mode A, as specified in TS 36.331 [5] and TS 36.133 [16]. It is mandatory for UEs of this release if ce-ModeA-r13 is supported.

8.2.4.27.3	Test description

8.2.4.27.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3-CE) on Cell 1 according to [18].

8.2.4.27.3.2	Test procedure sequence

Table 8.2.4.27.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.27.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.27.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void |  |  | - | - |
| 2 | Void |  |  | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.2.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 using common preamble to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | - | - |

8.2.4.27.3.3	Specific message contents

Table 8.2.4.27.3.3-1: Void

Table 8.2.4.27.3.3-2: Void

Table 8.2.4.27.3.3-3: RRCConnectionReconfiguration (step 3, Table 8.2.4.27.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.27.3.3-4: MeasConfig (Table 8.2.4.27.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.27.3.3-5 MeasurementReport (step 6, Table 8.2.4.27.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.27.3.3-6: RRCConnectionReconfiguration (step 7, Table 8.2.4.27.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Octet string of SystemInformationBlockType1-BR of Cell2 |  | CEModeA,CEModeB |
| nonCriticalExtension ::= SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.27.3.3-7: MobilityControlInfo (Table 8.2.4.27.3.3-6)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

#### 8.2.4.28	eCall Only mode / RRC connection reconfiguration / Inter-frequency Handover / Success

##### 8.2.4.28.1	Test Purpose (TP)

(1)

with { UE in eCall Only mode and an eCall over IMS is ongoing }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo indicating a different E-UTRA frequency }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

##### 8.2.4.28.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6. Unless otherwise stated these are Rel-14 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

...

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

##### 8.2.4.28.3	Test description

8.2.4.28.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 both supports eCall over IMS.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	The UE has established an eCall over IMS according to [18] subclause 4.5A.27 by executing steps 1 to 25.

8.2.4.28.3.2	Test procedure sequence

Table 8.2.4.28.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.28.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -97 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 8.2.4.28.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.28.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the inter frequency handover? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 7 | Release eCall over IMS using the generic test procedure described in TS 34.229-1 [35] subclause C.33. | - | - | - | - |
| 8 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |

8.2.4.28.3.3	Specific message contents

Table 8.2.4.28.3.3-0: Conditions for specific message contents in Tables 8.2.4.28.3.3-2 and 8.2.4.28.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.28.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.28.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.28.3.3-2: MeasConfig (Table 8.2.4.28.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.28.3.3-3: MeasurementReport (step 4, Table 8.2.4.28.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.28.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.28.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.28.3.3-5: MobilityControlInfo (Table 8.2.4.28.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.29	UDC/ RRC connection reconfiguration / Handover / Success

##### 8.2.4.29.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with UDC configuration}

ensure that {

when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo }

then { UE release the configured uplinkDataCompression and re-establish PDCP for all RBs configured with pdcp-config that are established }

}

##### 8.2.4.29.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331, clauses 5.3.5.4.

[TS 36.331 clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	if BL UE or UE in CE:

2>	if sameSFN-Indication is not present in mobilityControlInfo:

3>	acquire the MasterInformationBlock in the target PCell;

1>	if makeBeforeBreak is configured:

2>	perform the remainder of this procedure including and following resetting MAC after the UE has stopped the uplink transmission/downlink reception with the source cell(s);

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell [16], if makeBeforeBreak is configured.

1>	reset MCG MAC and SCG MAC, if configured;

1>	release uplinkDataCompression, if configured;

1>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

##### 8.2.4.29.3	Test description

8.2.4.29.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

-	The condition UDC is used for PDCP-Config in step 8 in 4.5.3.3 according to [18].

8.2.4.29.3.2	Test procedure sequence

Table 8.2.4.29.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.29.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.29.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 3 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. After having sent a PDU, the SS set Next_PDCP_TX_SN= 1. | <-- | PDCP PDU DATA #0 | - | - |
| 3 | The UE sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. | --> | PDCP PDU DATA #0 | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. After having sent a PDU, the SS set Next_PDCP_TX_SN= 2. | <-- | PDCP PDU DATA #1 | - | - |
| 5 | The UE sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 1). Data is previously received data from PDU #1 after decompression. | --> | PDCP PDU DATA #1 | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.2.3.2-1. | - | - | - | - |
| 9 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | - | - |
| 10 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 12 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU DATA #0 | - | - |
| 13 | Check: Does the UE sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 1, Checksum = 1111. Data is previously received data from PDU #0 after decompression. | --> | PDCP PDU DATA #0 | 1 | P |
| Note 1: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

8.2.4.29.3.3	Specific message contents

Table 8.2.4.29.3.3-1: RRCConnectionReconfiguration (step 6, Table 8.2.4.29.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.29.3.3-2: MeasConfig (Table 8.2.4.29.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.29.3.3-3 MeasurementReport (step 9, Table 8.2.4.29.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.29.3.3-4: RRCConnectionReconfiguration (step 10, Table 8.2.4.29.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.29.3.3-5: MobilityControlInfo (Table 8.2.4.29.3.3-4)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

#### 8.2.4.30	RRC connection reconfiguration / Handover / DAPS Handover

##### 8.2.4.30.1	RRC connection reconfiguration / Handover / DAPS Handover / Success / Intra-Frequency

8.2.4.30.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover }

ensure that {

when { UE receives an RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

then { UE performs DAPS handover to the target cell. For the DRB configured daps-HO, the UE continues to receive downlink data from both source and target eNBs for the DL and the UE transmits UL data to the source eNB }

}

(2)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when {  UE has performed random access procedure to the target cell successfully }

then { UE transmits an RRCReconfigurationComplete message and  the UE switches its UL data transmission to the target eNB }

}

(3)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message does not include the mobilityControlInfo and includes daps-SourceRelease }

then { UE stops to receive downlink data from source eNB and releases the radio link with source cell }

}

8.2.4.30.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1.0 and 10.1.2.1.1 and TS 36.331 clause 5.3.5.3 and 5.3.5.4. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1.0]

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB. Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	UE maintains only PCell connection with both source and target cells and any other configured serving cells are released by the network before the handover command is sent to the UE.

NOTE:	The handling on RLC and PDCP for DRBs not configured with DAPS is the same as in normal handover.

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell except in DAPS handover or CHO scenarios:

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

-	When initial CHO execution attempt fails or Handover fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

[TS 36.300, clause 10.1.2.1.1]

The preparation and execution phase of the HO procedure is performed without EPC involvement, i.e. preparation messages are directly exchanged between the eNBs. The release of the resources at the source side during the HO completion phase is triggered by the eNB. In case an RN is involved, its DeNB relays the appropriate S1 messages between the RN and the MME (S1-based handover) and X2 messages between the RN and target eNB (X2-based handover); the DeNB is explicitly aware of a UE attached to the RN due to the S1 proxy and X2 proxy functionality (see clause 4.7.6.6). The figure below depicts the basic handover scenario where neither MME nor Serving Gateway changes:

Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO

Below is a more detailed description of the intra-MME/Serving Gateway HO procedure:

0	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1	The source eNB configures the UE measurement procedures according to the roaming and access restriction information and e.g. the available multiple frequency band information. Measurements provided by the source eNB may assist the function controlling the UE's connection mobility.

2	A MEASUREMENT REPORT is triggered and sent to the eNB.

3	The source eNB makes decision based on MEASUREMENT REPORT and RRM information to hand off the UE.

4	The source eNB issues a HANDOVER REQUEST message to the target eNB passing necessary information to prepare the HO at the target side (UE X2 signalling context reference at source eNB, UE S1 EPC signalling context reference, target cell ID, KeNB*, RRC context including the C-RNTI of the UE in the source eNB, AS-configuration, E-RAB context and physical layer ID of the source cell + short MAC-I for possible RLF recovery). The source eNB may also request a DAPS Handover for one or more E-RABs. UE X2 / UE S1 signalling references enable the target eNB to address the source eNB and the EPC. The E-RAB context includes necessary RNL and TNL addressing information, and QoS profiles of the E-RABs.

5	Admission Control may be performed by the target eNB dependent on the received E-RAB QoS information to increase the likelihood of a successful HO, if the resources can be granted by target eNB. The target eNB configures the required resources according to the received E-RAB QoS information and reserves a C-RNTI and optionally a RACH preamble. The AS-configuration to be used in the target cell can either be specified independently (i.e. an "establishment") or as a delta compared to the AS-configuration used in the source cell (i.e. a "reconfiguration").

6	The target eNB prepares HO with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. The HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover. The container includes a new C-RNTI, target eNB security algorithm identifiers for the selected security algorithms, may include a dedicated RACH preamble, and possibly some other parameters i.e. access parameters, SIBs, etc. If RACH-less HO is configured, the container includes timing adjustment indication and optionally a preallocated uplink grant. The HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the forwarding tunnels, if necessary. The target eNB also indicates if a DAPS Handover is accepted.

NOTE 1:	As soon as the source eNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE 1a:	For E-RABs configured with DAPS, downlink PDCP SDUs are forwarded with SN assigned by the source eNB, until SN assignment is handed over to the target eNB in step 11b, for which the normal data forwarding follows as defined in 10.1.2.3.

Steps 7 to 16 provide means to avoid data loss during HO and are further detailed in 10.1.2.1.2 and 10.1.2.3.

7	The target eNB generates the RRC message to perform the handover, i.e. RRCConnectionReconfiguration message including the mobilityControlInfo, to be sent by the source eNB towards the UE. The source eNB performs the necessary integrity protection and ciphering of the message.

The UE receives the RRCConnectionReconfiguration message with necessary parameters (i.e. new C-RNTI, target eNB security algorithm identifiers, and optionally dedicated RACH preamble, target eNB SIBs, etc.) and is commanded by the source eNB to perform the HO. If RACH-less HO is configured, the RRCConnectionReconfiguration includes timing adjustment indication and optionally preallocated uplink grant for accessing the target eNB. If preallocated uplink grant is not included, the UE should monitor PDCCH of the target eNB to receive an uplink grant. The UE does not need to delay the handover execution for delivering the HARQ/ARQ responses to source eNB.

If Make-Before-Break HO is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInfo before the UE executes initial uplink transmission to the target cell.

NOTE 2:	If Make-Before-Break HO is configured, the source eNB decides when to stop transmitting to the UE.

NOTE 3:	The UE can be configured with Make-Before-Break HO and RACH-less HO simultaneously.

In case of DAPS Handover, the UE does not detach from the source cell upon receiving the RRCConnectionReconfiguration message. The UE releases the source SRB resources, security configuration of the source cell and stops DL/UL reception/transmission with the source upon receiving an explicit release from the target node.

NOTE 3a:	The DAPS Handover is considered to only be completed after the UE has released the source cell as explicitly requested from the target node. RRC suspend, a subsequent handover or inter-RAT handover cannot be initiated until the source cell has been released.

NOTE 4:	DC, CHO or RACH-less HO cannot be configured simultaneously with DAPS Handover.

NOTE 5:	For E-RABs configured with DAPS, the source eNB does not stop transmitting downlink packets until it receives the HANDOVER SUCCESS message from the target eNB in step 11a.

8a	For E-RABs configured with DAPS, the source eNB sends the EARLY STATUS TRANSFER message. The DL COUNT value conveyed in the EARLY STATUS TRANSFER message indicates PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB. The source eNB does not stop assigning PDCP SNs to downlink packets until it sends the SN STATUS TRANSFER message to the target eNB in step 11b.

8	For E-RABs not configured with DAPS, the source eNB sends the SN STATUS TRANSFER message to the target eNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of E-RABs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL SDU and may include a bit map of the receive status of the out of sequence UL SDUs that the UE needs to retransmit in the target cell, if there are any such SDUs. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target eNB shall assign to new SDUs, not having a PDCP SN yet. The source eNB may omit sending this message if none of the E-RABs of the UE shall be treated with PDCP status preservation.

NOTE 6:	In case of DAPS Handover, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status for an E-RAB with RLC-AM and not configured with DAPS may be transferred by the SN STATUS TRANSFER message in step 11b instead of step 8.

NOTE 7:	For E-RABs configured with DAPS, the source eNB may additionally send the EARLY STATUS TRANSFER message(s) between step 8 and step 11b, to inform discarding of already forwarded PDCP SDUs. The target eNB does not transmit forwarded downlink PDCP SDUs to the UE whose COUNT is less than the conveyed DL COUNT value and discards them if transmission has not been attempted already.

9	If RACH-less HO is not configured, after receiving the RRCConnectionReconfiguration message including the mobilityControlInfo, UE performs synchronisation to target eNB and accesses the target cell via RACH, following a contention-free procedure if a dedicated RACH preamble was indicated in the mobilityControlInfo, or following a contention-based procedure if no dedicated preamble was indicated. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell. 

If RACH-less HO is configured, UE performs synchronisation to target eNB. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

10	If RACH-less HO is not configured, the target eNB responds with UL allocation and timing advance.

10a If RACH-less HO is configured and the UE did not get the periodic pre-allocated uplink grant in the RRCConnectionReconfiguration message including the mobilityControlInfo, the UE receives uplink grant via the PDCCH of the target cell. The UE uses the first available uplink grant after synchronization to the target cell.

11	When the RACH-less HO is not configured and the UE has successfully accessed the target cell, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB, which indicates that the handover procedure is completed for the UE. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE.

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

11a/b	In case of DAPS Handover, the target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message for E-RABs configured with DAPS for which the description in step 8 applies, and the normal data forwarding follows as defined in 10.1.2.3.

NOTE 8:	For E-RABs configured with DAPS, the source eNB does not stop delivering uplink packets to the S-GW until it sends the SN STATUS TRANSFER message in step 11b. The target eNB does not forward the uplink PDCP SDUs successfully received in-sequence to the S-GW until it receives the SN STATUS TRANSFER message, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the S-GW. The target eNB does not deliver any uplink packet which has an UL COUNT lower than the provided.

NOTE 9:	Void.

12	The target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

13	The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway.

14	The Serving Gateway switches the downlink data path to the target side. The Serving gateway sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

15	The Serving Gateway sends a MODIFY BEARER RESPONSE message to MME.

16	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

17	By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB. The target eNB sends this message after the PATH SWITCH REQUEST ACKNOWLEDGE message is received from the MME.

18	Upon reception of the UE CONTEXT RELEASE message, the source eNB can release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

When an X2 handover is used involving HeNBs and when the source HeNB is connected to a HeNB GW, a UE CONTEXT RELEASE REQUEST message including an explicit GW Context Release Indication is sent by the source HeNB, in order to indicate that the HeNB GW may release of all the resources related to the UE context.

For DAPS handover, upon receiving DAPS handover command message, the UE suspends source cell SRBs, stops sending and receiving any RRC control plane signalling towards the source cell and establishes SRBs for the target cell. The UE releases the source cell SRBs configuration upon receiving source cell release indication from the target cell after successful DAPS handover execution. When DAPS handover to the target cell fails and if the source cell link is available then the UE reverts back to the source cell configuration and activates source cell SRBs for control plane signalling. When DAPS handover is configured, PDCP duplication is not allowed.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if the received RRCConnectionReconfiguration includes the daps-SourceRelease:

2>	reset source MCG MAC and release the source MCG MAC configuration;

2>	for each DAPS bearer:

3>	re-establish the RLC entity or entities for the source PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the source PCell;

3>	reconfigure the PDCP entity to release DAPS, as specified in TS 36.323 [8];

2>	for each SRB:

3>	release the PDCP entity for the source PCell;

3>	release the RLC entity and the associated DCCH logical channel for the source PCell;

2>	release the physical channel configuration for the source PCell;

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if daps-HO is configured for any DRB:

2>	establish a MAC entity for the target PCell, with the same configuration as the MAC entity for the source PCell;

2>	for each DRB configured with daps-HO:

3>	establish the RLC entity or entities and the associated DTCH logical channel for the target PCell, with the same configurations as for the source PCell;

3>	reconfigure the PDCP entity to configure DAPS as specified in TS36.323 [8].

2>	for each DRB not configured with daps-HO:

3>	re-establish PDCP;

3>	re-establish the RLC entity and associate it, and the associated DTCH logical channel, to the target PCell;

2>	for each SRB:

3>	establish a PDCP entity for the target PCell, with the same configuration as the PDCP entity for the source PCell;

3>	establish an RLC entity and an associated DCCH logical channel for the target PCell, with the same configuration as for the source PCell;

2>	suspend the SRBs for the source PCell;

NOTE 1c:	In order to understand if a daps-HO is configured, the UE needs to check the presence of the field daps-HO within the received RadioResourceConfigDedicated IE.

8.2.4.30.1.3	Test description

8.2.4.30.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the serving cell, Cell 2 is the intra-ferquency neighbour cell of Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.30.1.3.2	Test procedure sequence

Table 8.2.4.30.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant in Cell 1. | - | - | - | - |
| 2 | The SS sends one IP Packet to the UE on the DRB#1. | <-- |  | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 2. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step4 to 6, parallel behaviour defined in Table 8.2.4.30.1.3.2-2 is executed repeatedly. |  |  |  |  |
| 4 | The SS allocates uplink grant in Cell 1, allowing the UE to return the IP packet received at step 2 and continues to respond to scheduling requests from UE. | <-- | (UL Grant) | - | - |
| 5 | Check: Does the UE loop back the IP packet received at step 2 in the Cell 1? | - | - | 1 | P |
| 6 | The SS sends one IP Packet to the UE via DRB#1 in the Cell 1 and stops sending RLC acknowledgements for the waiting time period (Note 1) | - | - | - | - |
| 7 | The SS transmit Random Access Response to respond to the latest preamble on Cell 2. | <- | Random Access Response |  |  |
| 8 | The SS configures UL grant for the UE in Cell 2 and continues to respond to scheduling requests from UE. |  | (UL Grant) | - | - |
| - | EXCEPTION: Steps 9 and 10 can occur in any order. | - | - | - | - |
| 9 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message in Cell 2? |  | RRCConnectionReconfigurationComplete | 1,2 | P |
| 10 | Check: Does the UE loop back the IP packet received at step 6 in the Cell 2? | - | - | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message not including mobilityControlInfo and including daps-SourceRelease in Cell 2. |  | RRCConnectionReconfiguration | - | - |
| 12 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message in Cell 2? |  | RRCConnectionReconfigurationComplete | 3 | P |
| 13 | the SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 14 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1? | - | - | 3 | F |
| 15 | the SS transmits one IP Packet to verify data path on DRB#1 on Cell 2. | - | - | - | - |
| 16 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 2? | - | - | 1 | P |
| Note 1: The waiting timer starts after SS sending RRCConnectionReconfiguration message in step 3. The waiting time period should not exceed the value of T304. The default value of T304 is set to 1000ms currently as specified in table 4.6.5-1 in [18]. |  |  |  |  |  |

Table 8.2.4.30.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmit preamble to Cell2. | -> | (PRACH Preamble) | - | - |

8.2.4.30.1.3.3	Specific message contents

Table 8.2.4.30.1.3.3-1: RRCConnectionReconfiguration (step 3, Table 8.2.4.30.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.1.3.3-2: MobilityControlInfo (Table 8.2.4.30.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.1.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.1.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.1.3.3-4: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.1.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.1.3.3-5: RadioResourceConfigDedicated (Table 8.2.4.30.1.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.1.3.3-6: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.1.3.3-5)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.4.30.1.3.3-7: RRCConnectionReconfiguration (step 11, Table 8.2.4.30.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| daps-SourceRelease-r16 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.30.2	DAPS handover / Success / Radio Link Failure in source / Intra-Frequency

8.2.4.30.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { Timer T310 configured by using the value for target's timer T310 expires in source }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

(2)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { random access problem indication from source MCG MAC }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

(3)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

8.2.4.30.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.6 and TS 36.331 clause 5.3.11.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.6]

Two phases govern the behaviour associated to radio link failure as shown on Figure 10.1.6-1:

-	First phase:

-	started upon radio problem detection;

-	leads to radio link failure detection;

-	no UE-based mobility;

-	based on timer or other (e.g. counting) criteria (T1).

-	Second Phase:

-	started upon radio link failure detection or handover failure;

-	leads to RRC_IDLE;

-	UE-based mobility;

-	Timer based (T2).

Figure 10.1.6-1: Radio Link Failure

Table 10.1.6-1 below describes how mobility is handled with respect to radio link failure:

Table 10.1.6-1: Mobility and Radio Link Failure

| Cases | First Phase | Second Phase | T2 expired |
| --- | --- | --- | --- |
| UE returns to the same cell | Continue as if no radio problems occurred | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a different cell from the same eNB | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a prepared eNB (NOTE) | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a different eNB that is not prepared (NOTE) | N/A | Go via RRC_IDLE | Go via RRC_IDLE |
| NOTE: a prepared eNB is an eNB which has admitted the UE during an earlier executed HO preparation phase, or obtains the UE context during the Second Phase. |  |  |  |

For a NB-IoT UE that only uses Control Plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [20] and does not support RRC Connection re-establishment for the control plane as defined in TS 36.331 [16], at the end of the first phase, the UE enters RRC_IDLE (there is no second phase).In the Second Phase, in order to resume activity and avoid going via RRC_IDLE when the UE returns to the same cell or when the UE selects a different cell from the same eNB, or when the UE selects a cell from a different eNB, the following procedure applies:

-	The UE stays in RRC_CONNECTED;

-	The UE accesses the cell through the random access procedure;

-	Except for a NB-IoT UE using only Control Plane CIoT EPS/5GS optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. C-RNTI of the UE in the cell where the RLF occurred + physical layer identity of that cell + short MAC-I based on the keys of that cell) is used by the selected eNB to authenticate the UE and check whether it has a context stored for that UE:

-	If the eNB finds a context that matches the identity of the UE, or obtains this context from the previously serving eNB, it indicates to the UE that its connection can be resumed;

-	If the context is not found, RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

-	For a NB-IoT UE using only Control Plane CIoT EPS/5GS optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. S-TMSI (for EPS) or truncated 5G-S-TMSI (for 5GS) of the UE at the time where the RLF occurred + UL NAS MAC + UL NAS COUNT) is used by the selected (ng-)eNB to request the MME/AMF to authenticate the UE's re-establishment request and provide the UE context:

-	If the authentication of the UE is successful and a context is provided, it indicates to the UE that its connection can be resumed;

-	If no context is provided, the RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

The radio link failure procedure applies also for RNs, with the exception that the RN is limited to select a cell from its DeNB cell list. Upon detecting radio link failure, the RN discards any current RN subframe configuration (for communication with its DeNB), enabling the RN to perform normal contention-based RACH as part of the re-establishment. Upon successful re-establishment, an RN subframe configuration can be configured again using the RN reconfiguration procedure.

For DC, PCell supports above phases. In addition, the first phase of the radio link failure procedure is supported for PSCell. However, upon detecting RLF on the PSCell, the re-establishment procedure is not triggered at the end of the first phase. Instead, UE shall inform the radio link failure of PSCell to the MeNB.

NOTE:	If the recovery attempt in the second phase fails, the details of the RN behaviour in RRC_IDLE to recover an RRC connection are up to the RN implementation.

In case of DAPS handover, the UE continues the detection of radio link failure at the source cell until the successful completion of the random access procedure to the target cell. If RLF is declared in the source cell, the UE:

-	stays in RRC_CONNECTED;

-	stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration;

-	if handover failure is declared at the target cell after source cell RLF was declared,

-	selects a suitable cell and initiates RRC re-establishment;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after handover failure was declared.

In case of CHO, after RLF is declared in the source cell, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and if the selected cell is a CHO candidate and if network configured the UE to try CHO at the selected CHO candidate cell after RLF, then the UE attempts CHO execution, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if NR PSCell change is not ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

4>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

4>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

4>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

5>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

5>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

5>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

4>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if detailed location information is available, set the content of the locationInfo as follows:

5>	include the locationCoordinates;

5>	include the horizontalVelocity, if available;

4>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

4>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

4>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

6>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

4>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

5>	include the drb-EstablishedWithQCI-1;

4>	except for NB-IoT, set the connectionFailureType to rlf;

4>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

4>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

8.2.4.30.2.3	Test description

8.2.4.30.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.30.2.3.2	Test procedure sequence

Table 8.2.4.30.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.30.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -79 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 |  |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.30.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.30.2.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 2. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step 4 to 6, parallel behaviour defined in Table 8.2.4.30.2.3.2-3 is executed repeatedly on Cell 2 |  |  |  |  |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.2.4.30.2.3.2-1. |  |  |  |  |
| 4 | Wait 1s to make t310 expired. |  |  |  |  |
| 5 | The SS changes Cell 1 and Cell 2 parameters according to the row "T3" in table 8.2.4.30.2.3.2-1. |  |  |  |  |
| 6 | Check: Does the UE send RRCConnectionReestablishmentRequest message on Cell 2 in 5 seconds? | --> | RRCConnectionReestablishmentRequest | 1 | F |
| 7 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 8 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 1 | F |
| 9 | The SS transmit Random Access Response to respond to the latest preamble on Cell 2. | <- | Random Access Response |  |  |
| 10 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 1. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
|  | EXCEPTION: During the execution from Step12 to 15, parallel behaviour defined in Table 8.2.4.30.2.3.2-3 is executed repeatedly on Cell 1 |  |  |  |  |
| 12 | The SS transmits a PDCCH order from the Cell 2 providing Random Access Preamble. | <-- | (PDCCH Order) | - | - |
| - | EXCEPTION: Step 13 is executed 3 times and the SS does not transmit Random Access Response to the UE. |  |  |  |  |
| 13 | The UE transmits a preamble on PRACH to the Cell 2 using the same preamble index as given in step 12 | --> | (PRACH Preamble) | - | - |
| 14 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 15 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 2 | F |
| 16 | The SS transmit Random Access Response to respond to the latest preamble on Cell 1. | <- | Random Access Response |  |  |
| 17 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 1? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 18 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 2. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step 19 to 25, parallel behaviour defined in Table 8.2.4.30.2.3.2-3 is executed repeatedly on Cell 2 |  |  |  |  |
| 19 | The SS stops sending RLC acknowledgments on Cell 1 | - | - | - | - |
| 20 | The SS transmits one AMD PDU containing SDU (100 bytes) in its data field. | <-- | (AMD PDU) | - | - |
| 21 | The UE transmits an AMD PDU containing SDU in its data field. | --> | (AMD PDU) | - | - |
| 22 | EXCEPTION: Step 23 shall be repeated maxRetxThreshold times | - | - | - | - |
| 23 | The UE transmits an AMD PDU containing SDU in its data field. | --> | (AMD PDU) | - | - |
| 24 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 25 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 3 | F |
| 26 | The SS transmit Random Access Response to respond to the latest preamble on Cell 2. | <- | Random Access Response |  |  |
| 27 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 3 | P |

Table 8.2.4.30.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmit preamble. | -> | (PRACH Preamble) | - | - |

8.2.4.30.2.3.3	Specific message contents

Table 8.2.4.30.2.3.3-1: SystemInformationBlockType2 (preamble and all steps, Table 8.2.4.30.2.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.2.3.3-2: RRCConnectionReconfiguration (step 2, 11 and 18, Table 8.2.4.30.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.2.3.3-3: MobilityControlInfo (Table 8.2.4.30.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | Step 2 and 18 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 11 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | Step 2 and 18 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 11 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, Step 2 and 18 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 11 |
| } |  |  |  |
| t304 | ms2000 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.2.3.3-4: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.2.3.3-5: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.2.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.2.3.3-6: RadioResourceConfigDedicated (Table 8.2.4.30.2.3.3-2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.2.3.3-7: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.2.3.3-6)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

##### 8.2.4.30.3	DAPS handover / Failure / source link available / Radio Link Failure in source / Intra-Frequency

8.2.4.30.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { T304 expires (handover failure) and radio link failure has not been detected for the source MCG }

then { The UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment }

}

(2)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB and T304 expires (handover failure) and radio link failure has been detected for the source MCG}

ensure that {

when { UE perform RRC re-establishment and selecting a suitable E-UTRA cell while T311 is running }

then { UE stays in RRC_CONNECTED and selects a suitable cell and then initiates RRC re-establishment }

}

(3)

with { UE in RRC_CONNECTED state and supporting Intra-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB and T304 expires (handover failure) and radio link failure has been detected for the source MCG }

ensure that {

when { UE can not find a suitable cell before T311 expires }

then { UE enters RRC_IDLE }

}

8.2.4.30.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1.0 and 10.1.2.1.1 and TS 36.331 clause 5.3.5.6, 5.3.7.6 and 5.6.21.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1.0]

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB. Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	UE maintains only PCell connection with both source and target cells and any other configured serving cells are released by the network before the handover command is sent to the UE.

NOTE:	The handling on RLC and PDCP for DRBs not configured with DAPS is the same as in normal handover.

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell except in DAPS handover or CHO scenarios:

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

-	When initial CHO execution attempt fails or Handover fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

[TS 36.300, clause 10.1.2.1.1]

The preparation and execution phase of the HO procedure is performed without EPC involvement, i.e. preparation messages are directly exchanged between the eNBs. The release of the resources at the source side during the HO completion phase is triggered by the eNB. In case an RN is involved, its DeNB relays the appropriate S1 messages between the RN and the MME (S1-based handover) and X2 messages between the RN and target eNB (X2-based handover); the DeNB is explicitly aware of a UE attached to the RN due to the S1 proxy and X2 proxy functionality (see clause 4.7.6.6). The figure below depicts the basic handover scenario where neither MME nor Serving Gateway changes:

Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO

Below is a more detailed description of the intra-MME/Serving Gateway HO procedure:

0	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1	The source eNB configures the UE measurement procedures according to the roaming and access restriction information and e.g. the available multiple frequency band information. Measurements provided by the source eNB may assist the function controlling the UE's connection mobility.

2	A MEASUREMENT REPORT is triggered and sent to the eNB.

3	The source eNB makes decision based on MEASUREMENT REPORT and RRM information to hand off the UE.

4	The source eNB issues a HANDOVER REQUEST message to the target eNB passing necessary information to prepare the HO at the target side (UE X2 signalling context reference at source eNB, UE S1 EPC signalling context reference, target cell ID, KeNB*, RRC context including the C-RNTI of the UE in the source eNB, AS-configuration, E-RAB context and physical layer ID of the source cell + short MAC-I for possible RLF recovery). The source eNB may also request a DAPS Handover for one or more E-RABs. UE X2 / UE S1 signalling references enable the target eNB to address the source eNB and the EPC. The E-RAB context includes necessary RNL and TNL addressing information, and QoS profiles of the E-RABs.

5	Admission Control may be performed by the target eNB dependent on the received E-RAB QoS information to increase the likelihood of a successful HO, if the resources can be granted by target eNB. The target eNB configures the required resources according to the received E-RAB QoS information and reserves a C-RNTI and optionally a RACH preamble. The AS-configuration to be used in the target cell can either be specified independently (i.e. an "establishment") or as a delta compared to the AS-configuration used in the source cell (i.e. a "reconfiguration").

6	The target eNB prepares HO with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. The HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover. The container includes a new C-RNTI, target eNB security algorithm identifiers for the selected security algorithms, may include a dedicated RACH preamble, and possibly some other parameters i.e. access parameters, SIBs, etc. If RACH-less HO is configured, the container includes timing adjustment indication and optionally a preallocated uplink grant. The HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the forwarding tunnels, if necessary. The target eNB also indicates if a DAPS Handover is accepted.

NOTE 1:	As soon as the source eNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE 1a:	For E-RABs configured with DAPS, downlink PDCP SDUs are forwarded with SN assigned by the source eNB, until SN assignment is handed over to the target eNB in step 11b, for which the normal data forwarding follows as defined in 10.1.2.3.

Steps 7 to 16 provide means to avoid data loss during HO and are further detailed in 10.1.2.1.2 and 10.1.2.3.

7	The target eNB generates the RRC message to perform the handover, i.e. RRCConnectionReconfiguration message including the mobilityControlInfo, to be sent by the source eNB towards the UE. The source eNB performs the necessary integrity protection and ciphering of the message.

The UE receives the RRCConnectionReconfiguration message with necessary parameters (i.e. new C-RNTI, target eNB security algorithm identifiers, and optionally dedicated RACH preamble, target eNB SIBs, etc.) and is commanded by the source eNB to perform the HO. If RACH-less HO is configured, the RRCConnectionReconfiguration includes timing adjustment indication and optionally preallocated uplink grant for accessing the target eNB. If preallocated uplink grant is not included, the UE should monitor PDCCH of the target eNB to receive an uplink grant. The UE does not need to delay the handover execution for delivering the HARQ/ARQ responses to source eNB.

If Make-Before-Break HO is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInfo before the UE executes initial uplink transmission to the target cell.

NOTE 2:	If Make-Before-Break HO is configured, the source eNB decides when to stop transmitting to the UE.

NOTE 3:	The UE can be configured with Make-Before-Break HO and RACH-less HO simultaneously.

In case of DAPS Handover, the UE does not detach from the source cell upon receiving the RRCConnectionReconfiguration message. The UE releases the source SRB resources, security configuration of the source cell and stops DL/UL reception/transmission with the source upon receiving an explicit release from the target node.

NOTE 3a:	The DAPS Handover is considered to only be completed after the UE has released the source cell as explicitly requested from the target node. RRC suspend, a subsequent handover or inter-RAT handover cannot be initiated until the source cell has been released.

NOTE 4:	DC, CHO or RACH-less HO cannot be configured simultaneously with DAPS Handover.

NOTE 5:	For E-RABs configured with DAPS, the source eNB does not stop transmitting downlink packets until it receives the HANDOVER SUCCESS message from the target eNB in step 11a.

8a	For E-RABs configured with DAPS, the source eNB sends the EARLY STATUS TRANSFER message. The DL COUNT value conveyed in the EARLY STATUS TRANSFER message indicates PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB. The source eNB does not stop assigning PDCP SNs to downlink packets until it sends the SN STATUS TRANSFER message to the target eNB in step 11b.

8	For E-RABs not configured with DAPS, the source eNB sends the SN STATUS TRANSFER message to the target eNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of E-RABs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL SDU and may include a bit map of the receive status of the out of sequence UL SDUs that the UE needs to retransmit in the target cell, if there are any such SDUs. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target eNB shall assign to new SDUs, not having a PDCP SN yet. The source eNB may omit sending this message if none of the E-RABs of the UE shall be treated with PDCP status preservation.

NOTE 6:	In case of DAPS Handover, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status for an E-RAB with RLC-AM and not configured with DAPS may be transferred by the SN STATUS TRANSFER message in step 11b instead of step 8.

NOTE 7:	For E-RABs configured with DAPS, the source eNB may additionally send the EARLY STATUS TRANSFER message(s) between step 8 and step 11b, to inform discarding of already forwarded PDCP SDUs. The target eNB does not transmit forwarded downlink PDCP SDUs to the UE whose COUNT is less than the conveyed DL COUNT value and discards them if transmission has not been attempted already.

9	If RACH-less HO is not configured, after receiving the RRCConnectionReconfiguration message including the mobilityControlInfo, UE performs synchronisation to target eNB and accesses the target cell via RACH, following a contention-free procedure if a dedicated RACH preamble was indicated in the mobilityControlInfo, or following a contention-based procedure if no dedicated preamble was indicated. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell. 

If RACH-less HO is configured, UE performs synchronisation to target eNB. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

10	If RACH-less HO is not configured, the target eNB responds with UL allocation and timing advance.

10a If RACH-less HO is configured and the UE did not get the periodic pre-allocated uplink grant in the RRCConnectionReconfiguration message including the mobilityControlInfo, the UE receives uplink grant via the PDCCH of the target cell. The UE uses the first available uplink grant after synchronization to the target cell.

11	When the RACH-less HO is not configured and the UE has successfully accessed the target cell, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB, which indicates that the handover procedure is completed for the UE. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE.

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

11a/b	In case of DAPS Handover, the target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message for E-RABs configured with DAPS for which the description in step 8 applies, and the normal data forwarding follows as defined in 10.1.2.3.

NOTE 8:	For E-RABs configured with DAPS, the source eNB does not stop delivering uplink packets to the S-GW until it sends the SN STATUS TRANSFER message in step 11b. The target eNB does not forward the uplink PDCP SDUs successfully received in-sequence to the S-GW until it receives the SN STATUS TRANSFER message, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the S-GW. The target eNB does not deliver any uplink packet which has an UL COUNT lower than the provided.

NOTE 9:	Void.

12	The target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

13	The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway.

14	The Serving Gateway switches the downlink data path to the target side. The Serving gateway sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

15	The Serving Gateway sends a MODIFY BEARER RESPONSE message to MME.

16	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

17	By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB. The target eNB sends this message after the PATH SWITCH REQUEST ACKNOWLEDGE message is received from the MME.

18	Upon reception of the UE CONTEXT RELEASE message, the source eNB can release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

When an X2 handover is used involving HeNBs and when the source HeNB is connected to a HeNB GW, a UE CONTEXT RELEASE REQUEST message including an explicit GW Context Release Indication is sent by the source HeNB, in order to indicate that the HeNB GW may release of all the resources related to the UE context.

For DAPS handover, upon receiving DAPS handover command message, the UE suspends source cell SRBs, stops sending and receiving any RRC control plane signalling towards the source cell and establishes SRBs for the target cell. The UE releases the source cell SRBs configuration upon receiving source cell release indication from the target cell after successful DAPS handover execution. When DAPS handover to the target cell fails and if the source cell link is available then the UE reverts back to the source cell configuration and activates source cell SRBs for control plane signalling. When DAPS handover is configured, PDCP duplication is not allowed.

[TS 36.331, clause 5.3.5.6]

If T304 expires (handover failure), the UE shall:

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

1>	if no DAPS bearer is configured; or

1>	if any DAPS bearer is configured and radio link failure has been detected for the source MCG in accordance with 5.3.11.3:

2>	if attemptCondReconf is not configured:

3>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	else:

3>	revert back to the configuration used in the source PCell;

NOTE 1a:	In the context above, "the configuration" includes state variables and parameters of each radio bearer. PDCP entities associtated with RLC UM and SRB bearers are reset after the successful RRC connection re-establishment procedure according to clause 5.2 in TS 36.323 [8]. In the above, "the configuration" includes the RB configuration using NR PDCP, if configured (i.e. by nr-RadioBearerConfig1 and nr-RadioBearerConfig2).

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the source PCell based on measurements collected up to the moment the UE detected handover failure and in accordance with the following;

4>	if the UE includes rsrqResult, include the lastServCellRSRQ-Type;

3>	set the measResultNeighCells to include the best measured cells, other than the source PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE includes rsrqResult, include the rsrq-Type;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	if last RRCConnectionReconfiguration message including mobilityControlInfo concerned a failed intra-RAT handover (E-UTRA to E-UTRA):

4>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	else if last MobilityFromEUTRACommand concerned a failed inter-RAT handover from E-UTRA to NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO (EUTRA to NR):

4>	set the failedNR-PCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

1>	else (any DAPS bearer is configured and radio link failure has not been detected for the source MCG):

2>	release the MAC entity for the target PCell;

2>	for each DAPS bearer:

3>	re-establish the RLC entity for the target PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the target PCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 36.323 [8];

2>	for each non-DAPS bearer:

3>	revert back to the configuration used for the DRB in the source PCell, including PDCP and RLC states and the security configuration;

2>	for each SRB:

3>	discard any PDCP SDUs along with the PDCP data PDUs for the source PCell;

3>	re-establish the RLC entity for the source PCell;

3>	release the PDCP entity for the target PCell;

3>	release the RLC entity and the associated DCCH logical channel for the target PCell;

2>	release the physical channel configuration for the target PCell;

2>	resume the SRBs for the source PCell;

2>	initiate the failure information procedure as specified in 5.6.21 to report a DAPS HO failure.

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.6.21.3]

When initiating the procedure according to 5.6.21.2, the UE shall:

1>	set the contents of the FailureInformation message as follows:

2>	if the procedure is initiated to report RLC failure:

3>	set logicalChannelIdentity to the logical channel identity of the RLC entity;

3>	set cellGroupIndication to the cell group where the RLC entity is located;

3>	set failureType to the type of failure that has been detected;

2>	if the procedure is initiated to report a DAPS HO failure:

3>	set failureType to dapsHO-failure;

1>	submit the FailureInformation message to lower layers for transmission.8.2.4.30.3.3	Test description

8.2.4.30.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.30.3.3.2	Test procedure sequence

Table 8.2.4.30.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.30.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -85 |  |
| T5 | Cell-specific RS EPRE | dBm/15kHz | -79 | -85 |  |
| T6 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
| T7 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.30.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 3 the steps specified in Table 8.2.4.30.3.3.2-3 should take place on Cell 2. | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit an FailureInformation message on Cell 1 | --> | FailureInformation | 1 | P |
| 5 | The SS changes Cell 1 and Cell 2 parameters according to the row "T3" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 7 the steps specified in Table 8.2.4.30.3.3.2-3 should take place on Cell 2. | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 2 parameters according to the row "T4" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 2? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 9 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the currently active KeNB on Cell 2? | --> | RRCConnectionReestablishmentComplete | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 13 | The SS changes Cell 1 and Cell 2 parameters according to the row "T5" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 15 the steps specified in Table 8.2.4.30.3.3.2-3 should take place on Cell 2. | - | - | - | - |
| 15 | The SS changes Cell 1 and Cell 2 parameters according to the row "T6" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 16 | Wait for 12s to make T311 expired. | - | - | - | - |
| 17 | The SS changes Cell 1 and Cell 2 parameters according to the row "T7" in table 8.2.4.30.3.3.2-1. | - | - | - | - |
| 18 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 3 | P |

Table 8.2.4.30.3.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the handover using MAC Random Access Preamble. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.30.3.3.3	Specific message contents

Table 8.2.4.30.3.3.3-1: RRCConnectionReconfiguration (steps 2, 6 and 14, Table 8.2.4.30.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.3.3.3-2: MobilityControlInfo (Table 8.2.4.30.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | Step 2 and 6 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 14 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | Step 2 and 6 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 14 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, Step 2 and 6 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 14 |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.3.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.3.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.3.3.3-4: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.3.3.3-5: RadioResourceConfigDedicated (Table 8.2.4.30.3.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.3.3.3-6: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.3.3.3-5)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.4.30.3.3.3-7: FailureInformation (Step 4, Table 8.2.4.30.3.3.2-2)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FailureInformation-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| failureInformation-r16 SEQUENCE { |  |  |  |
| failedLogicalChannelIdentity-r16 SEQUENCE {} | Not present |  |  |
| failureType-r16 | dapsHO-failure |  |  |
| NonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.30.4	RRC connection reconfiguration / Handover / DAPS Handover / Success / Inter-Frequency

8.2.4.30.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover }

ensure that {

when { UE receives an RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

then { UE performs DAPS handover to the target cell. For the DRB configured daps-HO, the UE continues to receive downlink data from both source and target eNBs for the DL and the UE transmits UL data to the source eNB }

}

(2)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when {  UE has performed random access procedure to the target cell successfully }

then { UE tran

smits an RRCReconfigurationComplete message and  the UE switches its UL data transmission to the target eNB }

}

(3)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { UE receives an RRCConnectionReconfiguration message does not include the mobilityControlInfo and includes daps-SourceRelease }

then { UE stops to receive downlink data from source eNB and releases the radio link with source cell }

}

8.2.4.30.4.2	Conformance requirements

Same as 8.2.4.30.1.2.

8.2.4.30.4.3	Test description

8.2.4.30.4.3.1	Pre-test conditions

Same as 8.2.4.30.1.3.1 with following changes:

System Simulator:

-	Cell 1 is the serving cell and Cell 3 is the neighbour inter-frequency cell of Cell 1.

8.2.4.30.4.3.2	Test procedure sequence

Same as 8.2.4.30.1.3.2.

8.2.4.30.4.3.3	Specific message contents

Table 8.2.4.30.4.3.3-0: Conditions for specific message contents in Table 8.2.4.30.4.3.3-2

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.4.3.3-1: RRCConnectionReconfiguration (step 3, Table 8.2.4.30.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-2: MobilityControlInfo (Table 8.2.4.30.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.4.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-4: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-5: RadioResourceConfigDedicated (Table 8.2.4.30.4.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-6: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.4.3.3-5)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.4.30.4.3.3-7: RRCConnectionReconfiguration (step 11, Table 8.2.4.30.1.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| daps-SourceRelease-r16 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.30.5	DAPS handover / Success / Radio Link Failure in source / Inter-Frequency

8.2.4.30.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { Timer T310 configured by using the value for target's timer T310 expires in source }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

(2)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { random access problem indication from source MCG MAC }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

(3)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB }

then { UE stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration and continue the handover procedure to target }

}

8.2.4.30.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.6 and TS 36.331 clause 5.3.11.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.6]

Two phases govern the behaviour associated to radio link failure as shown on Figure 10.1.6-1:

-	First phase:

-	started upon radio problem detection;

-	leads to radio link failure detection;

-	no UE-based mobility;

-	based on timer or other (e.g. counting) criteria (T1).

-	Second Phase:

-	started upon radio link failure detection or handover failure;

-	leads to RRC_IDLE;

-	UE-based mobility;

-	Timer based (T2).

Figure 10.1.6-1: Radio Link Failure

Table 10.1.6-1 below describes how mobility is handled with respect to radio link failure:

Table 10.1.6-1: Mobility and Radio Link Failure

| Cases | First Phase | Second Phase | T2 expired |
| --- | --- | --- | --- |
| UE returns to the same cell | Continue as if no radio problems occurred | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a different cell from the same eNB | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a prepared eNB (NOTE) | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a different eNB that is not prepared (NOTE) | N/A | Go via RRC_IDLE | Go via RRC_IDLE |
| NOTE: a prepared eNB is an eNB which has admitted the UE during an earlier executed HO preparation phase, or obtains the UE context during the Second Phase. |  |  |  |

For a NB-IoT UE that only uses Control Plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [20] and does not support RRC Connection re-establishment for the control plane as defined in TS 36.331 [16], at the end of the first phase, the UE enters RRC_IDLE (there is no second phase).In the Second Phase, in order to resume activity and avoid going via RRC_IDLE when the UE returns to the same cell or when the UE selects a different cell from the same eNB, or when the UE selects a cell from a different eNB, the following procedure applies:

-	The UE stays in RRC_CONNECTED;

-	The UE accesses the cell through the random access procedure;

-	Except for a NB-IoT UE using only Control Plane CIoT EPS/5GS optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. C-RNTI of the UE in the cell where the RLF occurred + physical layer identity of that cell + short MAC-I based on the keys of that cell) is used by the selected eNB to authenticate the UE and check whether it has a context stored for that UE:

-	If the eNB finds a context that matches the identity of the UE, or obtains this context from the previously serving eNB, it indicates to the UE that its connection can be resumed;

-	If the context is not found, RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

-	For a NB-IoT UE using only Control Plane CIoT EPS/5GS optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. S-TMSI (for EPS) or truncated 5G-S-TMSI (for 5GS) of the UE at the time where the RLF occurred + UL NAS MAC + UL NAS COUNT) is used by the selected (ng-)eNB to request the MME/AMF to authenticate the UE's re-establishment request and provide the UE context:

-	If the authentication of the UE is successful and a context is provided, it indicates to the UE that its connection can be resumed;

-	If no context is provided, the RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

The radio link failure procedure applies also for RNs, with the exception that the RN is limited to select a cell from its DeNB cell list. Upon detecting radio link failure, the RN discards any current RN subframe configuration (for communication with its DeNB), enabling the RN to perform normal contention-based RACH as part of the re-establishment. Upon successful re-establishment, an RN subframe configuration can be configured again using the RN reconfiguration procedure.

For DC, PCell supports above phases. In addition, the first phase of the radio link failure procedure is supported for PSCell. However, upon detecting RLF on the PSCell, the re-establishment procedure is not triggered at the end of the first phase. Instead, UE shall inform the radio link failure of PSCell to the MeNB.

NOTE:	If the recovery attempt in the second phase fails, the details of the RN behaviour in RRC_IDLE to recover an RRC connection are up to the RN implementation.

In case of DAPS handover, the UE continues the detection of radio link failure at the source cell until the successful completion of the random access procedure to the target cell. If RLF is declared in the source cell, the UE:

-	stays in RRC_CONNECTED;

-	stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration;

-	if handover failure is declared at the target cell after source cell RLF was declared,

-	selects a suitable cell and initiates RRC re-establishment;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after handover failure was declared.

In case of CHO, after RLF is declared in the source cell, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and if the selected cell is a CHO candidate and if network configured the UE to try CHO at the selected CHO candidate cell after RLF, then the UE attempts CHO execution, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if NR PSCell change is not ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

4>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

4>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

4>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

5>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

5>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

5>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

4>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if detailed location information is available, set the content of the locationInfo as follows:

5>	include the locationCoordinates;

5>	include the horizontalVelocity, if available;

4>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

4>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

4>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

6>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

4>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

5>	include the drb-EstablishedWithQCI-1;

4>	except for NB-IoT, set the connectionFailureType to rlf;

4>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

4>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

8.2.4.30.5.3	Test description

8.2.4.30.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.30.5.3.2	Test procedure sequence

Table 8.2.4.30.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.30.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -79 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -85 |  |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.30.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.30.5.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 3. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step 4 to 6, parallel behaviour defined in Table 8.2.4.30.5.3.2-3 is executed repeatedly on Cell 3 |  |  |  |  |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T2" in table 8.2.4.30.5.3.2-1. |  |  |  |  |
| 4 | Wait 1s to make t310 expired. |  |  |  |  |
| 5 | The SS changes Cell 1 and Cell 3 parameters according to the row "T3" in table 8.2.4.30.5.3.2-1. |  |  |  |  |
| 6 | Check: Does the UE send RRCConnectionReestablishmentRequest message on Cell 3 in 5 seconds? | --> | RRCConnectionReestablishmentRequest | 1 | F |
| 7 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 8 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 1 | F |
| 9 | The SS transmit Random Access Response to respond to the latest preamble on Cell 3. | <- | Random Access Response |  |  |
| 10 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 3? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 1. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
|  | EXCEPTION: During the execution from Step 12 to 15, parallel behaviour defined in Table 8.2.4.30.5.3.2-3 is executed repeatedly on Cell 1 |  |  |  |  |
| 12 | The SS transmits a PDCCH order from the Cell 3 providing Random Access Preamble. | <-- | (PDCCH Order) | - | - |
| - | EXCEPTION: Step 13 is executed 3 times and the SS does not transmit Random Access Response to the UE. |  |  |  |  |
| 13 | The UE transmits a preamble on PRACH to the Cell 3 using the same preamble index as given in step 12 | --> | (PRACH Preamble) | - | - |
| 14 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 15 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 2 | F |
| 16 | The SS transmit Random Access Response to respond to the latest preamble on Cell 1. | <- | Random Access Response |  |  |
| 17 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 1? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 18 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 3. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: During the execution from Step 19 to 25, parallel behaviour defined in Table 8.2.4.30.5.3.2-3 is executed repeatedly on Cell 3 |  |  |  |  |
| 19 | The SS stops sending RLC acknowledgments on Cell 1 | - | - | - | - |
| 20 | The SS transmits one AMD PDU containing SDU (100 bytes) in its data field. | <-- | (AMD PDU) | - | - |
| 21 | The UE transmits an AMD PDU containing SDU in its data field. | --> | (AMD PDU) | - | - |
| 22 | EXCEPTION: Step 23 shall be repeated maxRetxThreshold times | - | - | - | - |
| 23 | The UE transmits an AMD PDU containing SDU in its data field. | --> | (AMD PDU) | - | - |
| 24 | The SS transmits one IP Packet to verify data path on DRB#1 on Cell 1. | - | - | - | - |
| 25 | Check: Does UE send the IP Packet on DRB#1 in the uplink on Cell 1 in 5 seconds? | - | - | 3 | F |
| 26 | The SS transmit Random Access Response to respond to the latest preamble on Cell 3. | <- | Random Access Response |  |  |
| 27 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message on Cell 3? | --> | RRCConnectionReconfigurationComplete | 3 | P |

Table 8.2.4.30.5.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmit preamble. | -> | (PRACH Preamble) | - | - |

8.2.4.30.5.3.3	Specific message contents

Table 8.2.4.30.5.3.3-0: Conditions for specific message contents in Table 8.2.4.30.5.3.3-3

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.5.3.3-1: SystemInformationBlockType2 (preamble and all steps, Table 8.2.4.30.5.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.5.3.3-2: RRCConnectionReconfiguration (step 2, 11 and 18, Table 8.2.4.30.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.5.3.3-3: MobilityControlInfo (Table 8.2.4.30.5.3.3-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  | Step 2 and 18 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 11 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  | Step 2 and 18 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 11 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range, Step 2 and 18 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 11 |
| } |  |  |  |
| t304 | ms2000 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.5.3.3-4: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.5.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.5.3.3-5: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.5.3.3-4)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.5.3.3-6: RadioResourceConfigDedicated (Table 8.2.4.30.5.3.3-2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.5.3.3-7: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.5.3.3-6)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

##### 8.2.4.30.6	DAPS handover / Failure / source link available / Radio Link Failure in source / Inter-Frequency

8.2.4.30.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB }

ensure that {

when { T304 expires (handover failure) and radio link failure has not been detected for the source MCG }

then { The UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment }

}

(2)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB and T304 expires (handover failure) and radio link failure has been detected for the source MCG}

ensure that {

when { UE perform RRC re-establishment and selecting a suitable E-UTRA cell while T311 is running }

then { UE stays in RRC_CONNECTED and selects a suitable cell and then initiates RRC re-establishment }

}

(3)

with { UE in RRC_CONNECTED state and supporting Inter-Frequency DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover to the target cell and daps-HO is configured for any DRB and T304 expires (handover failure) and radio link failure has been detected for the source MCG }

ensure that {

when { UE can not find a suitable cell before T311 expires }

then { UE enters RRC_IDLE }

}

8.2.4.30.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1.0 and 10.1.2.1.1 and TS 36.331 clause 5.3.5.6, 5.3.7.6 and 5.6.21.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1.0]

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB. Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	UE maintains only PCell connection with both source and target cells and any other configured serving cells are released by the network before the handover command is sent to the UE.

NOTE:	The handling on RLC and PDCP for DRBs not configured with DAPS is the same as in normal handover.

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell except in DAPS handover or CHO scenarios:

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

-	When initial CHO execution attempt fails or Handover fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

[TS 36.300, clause 10.1.2.1.1]

The preparation and execution phase of the HO procedure is performed without EPC involvement, i.e. preparation messages are directly exchanged between the eNBs. The release of the resources at the source side during the HO completion phase is triggered by the eNB. In case an RN is involved, its DeNB relays the appropriate S1 messages between the RN and the MME (S1-based handover) and X2 messages between the RN and target eNB (X2-based handover); the DeNB is explicitly aware of a UE attached to the RN due to the S1 proxy and X2 proxy functionality (see clause 4.7.6.6). The figure below depicts the basic handover scenario where neither MME nor Serving Gateway changes:

Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO

Below is a more detailed description of the intra-MME/Serving Gateway HO procedure:

0	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1	The source eNB configures the UE measurement procedures according to the roaming and access restriction information and e.g. the available multiple frequency band information. Measurements provided by the source eNB may assist the function controlling the UE's connection mobility.

2	A MEASUREMENT REPORT is triggered and sent to the eNB.

3	The source eNB makes decision based on MEASUREMENT REPORT and RRM information to hand off the UE.

4	The source eNB issues a HANDOVER REQUEST message to the target eNB passing necessary information to prepare the HO at the target side (UE X2 signalling context reference at source eNB, UE S1 EPC signalling context reference, target cell ID, KeNB*, RRC context including the C-RNTI of the UE in the source eNB, AS-configuration, E-RAB context and physical layer ID of the source cell + short MAC-I for possible RLF recovery). The source eNB may also request a DAPS Handover for one or more E-RABs. UE X2 / UE S1 signalling references enable the target eNB to address the source eNB and the EPC. The E-RAB context includes necessary RNL and TNL addressing information, and QoS profiles of the E-RABs.

5	Admission Control may be performed by the target eNB dependent on the received E-RAB QoS information to increase the likelihood of a successful HO, if the resources can be granted by target eNB. The target eNB configures the required resources according to the received E-RAB QoS information and reserves a C-RNTI and optionally a RACH preamble. The AS-configuration to be used in the target cell can either be specified independently (i.e. an "establishment") or as a delta compared to the AS-configuration used in the source cell (i.e. a "reconfiguration").

6	The target eNB prepares HO with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. The HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover. The container includes a new C-RNTI, target eNB security algorithm identifiers for the selected security algorithms, may include a dedicated RACH preamble, and possibly some other parameters i.e. access parameters, SIBs, etc. If RACH-less HO is configured, the container includes timing adjustment indication and optionally a preallocated uplink grant. The HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the forwarding tunnels, if necessary. The target eNB also indicates if a DAPS Handover is accepted.

NOTE 1:	As soon as the source eNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE 1a:	For E-RABs configured with DAPS, downlink PDCP SDUs are forwarded with SN assigned by the source eNB, until SN assignment is handed over to the target eNB in step 11b, for which the normal data forwarding follows as defined in 10.1.2.3.

Steps 7 to 16 provide means to avoid data loss during HO and are further detailed in 10.1.2.1.2 and 10.1.2.3.

7	The target eNB generates the RRC message to perform the handover, i.e. RRCConnectionReconfiguration message including the mobilityControlInfo, to be sent by the source eNB towards the UE. The source eNB performs the necessary integrity protection and ciphering of the message.

The UE receives the RRCConnectionReconfiguration message with necessary parameters (i.e. new C-RNTI, target eNB security algorithm identifiers, and optionally dedicated RACH preamble, target eNB SIBs, etc.) and is commanded by the source eNB to perform the HO. If RACH-less HO is configured, the RRCConnectionReconfiguration includes timing adjustment indication and optionally preallocated uplink grant for accessing the target eNB. If preallocated uplink grant is not included, the UE should monitor PDCCH of the target eNB to receive an uplink grant. The UE does not need to delay the handover execution for delivering the HARQ/ARQ responses to source eNB.

If Make-Before-Break HO is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInfo before the UE executes initial uplink transmission to the target cell.

NOTE 2:	If Make-Before-Break HO is configured, the source eNB decides when to stop transmitting to the UE.

NOTE 3:	The UE can be configured with Make-Before-Break HO and RACH-less HO simultaneously.

In case of DAPS Handover, the UE does not detach from the source cell upon receiving the RRCConnectionReconfiguration message. The UE releases the source SRB resources, security configuration of the source cell and stops DL/UL reception/transmission with the source upon receiving an explicit release from the target node.

NOTE 3a:	The DAPS Handover is considered to only be completed after the UE has released the source cell as explicitly requested from the target node. RRC suspend, a subsequent handover or inter-RAT handover cannot be initiated until the source cell has been released.

NOTE 4:	DC, CHO or RACH-less HO cannot be configured simultaneously with DAPS Handover.

NOTE 5:	For E-RABs configured with DAPS, the source eNB does not stop transmitting downlink packets until it receives the HANDOVER SUCCESS message from the target eNB in step 11a.

8a	For E-RABs configured with DAPS, the source eNB sends the EARLY STATUS TRANSFER message. The DL COUNT value conveyed in the EARLY STATUS TRANSFER message indicates PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB. The source eNB does not stop assigning PDCP SNs to downlink packets until it sends the SN STATUS TRANSFER message to the target eNB in step 11b.

8	For E-RABs not configured with DAPS, the source eNB sends the SN STATUS TRANSFER message to the target eNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of E-RABs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL SDU and may include a bit map of the receive status of the out of sequence UL SDUs that the UE needs to retransmit in the target cell, if there are any such SDUs. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target eNB shall assign to new SDUs, not having a PDCP SN yet. The source eNB may omit sending this message if none of the E-RABs of the UE shall be treated with PDCP status preservation.

NOTE 6:	In case of DAPS Handover, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status for an E-RAB with RLC-AM and not configured with DAPS may be transferred by the SN STATUS TRANSFER message in step 11b instead of step 8.

NOTE 7:	For E-RABs configured with DAPS, the source eNB may additionally send the EARLY STATUS TRANSFER message(s) between step 8 and step 11b, to inform discarding of already forwarded PDCP SDUs. The target eNB does not transmit forwarded downlink PDCP SDUs to the UE whose COUNT is less than the conveyed DL COUNT value and discards them if transmission has not been attempted already.

9	If RACH-less HO is not configured, after receiving the RRCConnectionReconfiguration message including the mobilityControlInfo, UE performs synchronisation to target eNB and accesses the target cell via RACH, following a contention-free procedure if a dedicated RACH preamble was indicated in the mobilityControlInfo, or following a contention-based procedure if no dedicated preamble was indicated. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell. 

If RACH-less HO is configured, UE performs synchronisation to target eNB. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

10	If RACH-less HO is not configured, the target eNB responds with UL allocation and timing advance.

10a If RACH-less HO is configured and the UE did not get the periodic pre-allocated uplink grant in the RRCConnectionReconfiguration message including the mobilityControlInfo, the UE receives uplink grant via the PDCCH of the target cell. The UE uses the first available uplink grant after synchronization to the target cell.

11	When the RACH-less HO is not configured and the UE has successfully accessed the target cell, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB, which indicates that the handover procedure is completed for the UE. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE.

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

11a/b	In case of DAPS Handover, the target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message for E-RABs configured with DAPS for which the description in step 8 applies, and the normal data forwarding follows as defined in 10.1.2.3.

NOTE 8:	For E-RABs configured with DAPS, the source eNB does not stop delivering uplink packets to the S-GW until it sends the SN STATUS TRANSFER message in step 11b. The target eNB does not forward the uplink PDCP SDUs successfully received in-sequence to the S-GW until it receives the SN STATUS TRANSFER message, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the S-GW. The target eNB does not deliver any uplink packet which has an UL COUNT lower than the provided.

NOTE 9:	Void.

12	The target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

13	The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway.

14	The Serving Gateway switches the downlink data path to the target side. The Serving gateway sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

15	The Serving Gateway sends a MODIFY BEARER RESPONSE message to MME.

16	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

17	By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB. The target eNB sends this message after the PATH SWITCH REQUEST ACKNOWLEDGE message is received from the MME.

18	Upon reception of the UE CONTEXT RELEASE message, the source eNB can release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

When an X2 handover is used involving HeNBs and when the source HeNB is connected to a HeNB GW, a UE CONTEXT RELEASE REQUEST message including an explicit GW Context Release Indication is sent by the source HeNB, in order to indicate that the HeNB GW may release of all the resources related to the UE context.

For DAPS handover, upon receiving DAPS handover command message, the UE suspends source cell SRBs, stops sending and receiving any RRC control plane signalling towards the source cell and establishes SRBs for the target cell. The UE releases the source cell SRBs configuration upon receiving source cell release indication from the target cell after successful DAPS handover execution. When DAPS handover to the target cell fails and if the source cell link is available then the UE reverts back to the source cell configuration and activates source cell SRBs for control plane signalling. When DAPS handover is configured, PDCP duplication is not allowed.

[TS 36.331, clause 5.3.5.6]

If T304 expires (handover failure), the UE shall:

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

1>	if no DAPS bearer is configured; or

1>	if any DAPS bearer is configured and radio link failure has been detected for the source MCG in accordance with 5.3.11.3:

2>	if attemptCondReconf is not configured:

3>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	else:

3>	revert back to the configuration used in the source PCell;

NOTE 1a:	In the context above, "the configuration" includes state variables and parameters of each radio bearer. PDCP entities associated with RLC UM and SRB bearers are reset after the successful RRC connection re-establishment procedure according to clause 5.2 in TS 36.323 [8]. In the above, "the configuration" includes the RB configuration using NR PDCP, if configured (i.e. by nr-RadioBearerConfig1 and nr-RadioBearerConfig2).

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the source PCell based on measurements collected up to the moment the UE detected handover failure and in accordance with the following;

4>	if the UE includes rsrqResult, include the lastServCellRSRQ-Type;

3>	set the measResultNeighCells to include the best measured cells, other than the source PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE includes rsrqResult, include the rsrq-Type;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	if last RRCConnectionReconfiguration message including mobilityControlInfo concerned a failed intra-RAT handover (E-UTRA to E-UTRA):

4>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	else if last MobilityFromEUTRACommand concerned a failed inter-RAT handover from E-UTRA to NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO (EUTRA to NR):

4>	set the failedNR-PCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

1>	else (any DAPS bearer is configured and radio link failure has not been detected for the source MCG):

2>	release the MAC entity for the target PCell;

2>	for each DAPS bearer:

3>	re-establish the RLC entity for the target PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the target PCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 36.323 [8];

2>	for each non-DAPS bearer:

3>	revert back to the configuration used for the DRB in the source PCell, including PDCP and RLC states and the security configuration;

2>	for each SRB:

3>	discard any PDCP SDUs along with the PDCP data PDUs for the source PCell;

3>	re-establish the RLC entity for the source PCell;

3>	release the PDCP entity for the target PCell;

3>	release the RLC entity and the associated DCCH logical channel for the target PCell;

2>	release the physical channel configuration for the target PCell;

2>	resume the SRBs for the source PCell;

2>	initiate the failure information procedure as specified in 5.6.21 to report a DAPS HO failure.

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.6.21.3]

When initiating the procedure according to 5.6.21.2, the UE shall:

1>	set the contents of the FailureInformation message as follows:

2>	if the procedure is initiated to report RLC failure:

3>	set logicalChannelIdentity to the logical channel identity of the RLC entity;

3>	set cellGroupIndication to the cell group where the RLC entity is located;

3>	set failureType to the type of failure that has been detected;

2>	if the procedure is initiated to report a DAPS HO failure:

3>	set failureType to dapsHO-failure;

1>	submit the FailureInformation message to lower layers for transmission.8.2.4.30.6.3	Test description

8.2.4.30.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

8.2.4.30.6.3.2	Test procedure sequence

Table 8.2.4.30.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.30.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -85 |  |
| T5 | Cell-specific RS EPRE | dBm/15kHz | -79 | -85 |  |
| T6 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
| T7 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.2.4.30.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 3 parameters according to the row "T1" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform Inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 3, the steps specified in Table 8.2.4.30.6.3.2-3 should take place on Cell 3. | - | - | - | - |
| 3 | The SS changes Cell 1 and Cell 3 parameters according to the row "T2" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit an FailureInformation message on Cell 1 | --> | FailureInformation | 1 | P |
| 5 | The SS changes Cell 1 and Cell 3 parameters according to the row "T3" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform Inter frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 7, the steps specified in Table 8.2.4.30.6.3.2-3 should take place on Cell 3. | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 3 parameters according to the row "T4" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on Cell 3? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 9 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 3. | <-- | RRCConnectionReestablishment | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReestablishmentComplete message using the security key derived from the currently active KeNB on Cell 3? | --> | RRCConnectionReestablishmentComplete | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 13 | The SS changes Cell 1 and Cell 3 parameters according to the row "T5" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform Inter frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 15, the steps specified in Table 8.2.4.30.6.3.2-3 should take place on Cell 1. | - | - | - | - |
| 15 | The SS changes Cell 1 and Cell 3 parameters according to the row "T6" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 16 | Wait for 12s to make T311 expired. | - | - | - | - |
| 17 | The SS changes Cell 1 and Cell 3 parameters according to the row "T7" in table 8.2.4.30.6.3.2-1. | - | - | - | - |
| 18 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 3 | P |

Table 8.2.4.30.6.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the handover using MAC Random Access Preamble. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.30.6.3.3	Specific message contents

Table 8.2.4.30.6.3.3-0: Conditions for specific message contents
in Table 8.2.4.30.6.3.3-2

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.30.6.3.3-1: RRCConnectionReconfiguration (steps 2, 6 and 14, Table 8.2.4.30.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-2: MobilityControlInfo (Table 8.2.4.30.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  | Step 2 and step 6 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 14 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  | Step 2 and step 6 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 14 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 3 |  | Band 24 High range, Step 2 and 6 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 14 |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  | Step 2 and step 6 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 14 |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 8.2.4.30.6.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-4: RACH-ConfigCommon-DEFAULT (Table 8.2.4.30.6.3.3-3)

| Derivation Path: 36.508, Table 4.6.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-5: RadioResourceConfigDedicated (Table 8.2.4.30.6.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE { | 1 entries |  |  |
| drb-ToAddMod | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| mac-MainConfig | Not Present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-6: DRB-ToAddMod-DEFAULT (Table 8.2.4.30.6.3.3-5)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity | Not Present |  |  |
| drb-Identity | 1 |  |  |
| pdcp-Config | Not Present |  |  |
| rlc-Config | Not Present |  |  |
| logicalChannelIdentity | Not Present |  |  |
| logicalChannelConfig | Not Present |  |  |
| daps-HO-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.4.30.6.3.3-7: FailureInformation (Step 4, Table 8.2.4.30.6.3.2-2)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FailureInformation-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| failureInformation-r16 SEQUENCE { |  |  |  |
| failedLogicalChannelIdentity-r16 SEQUENCE {} | Not present |  |  |
| failureType-r16 | dapsHO-failure |  |  |
| NonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.4.31	RRC connection reconfiguration / Handover / Conditional Handover

##### 8.2.4.31.1	Conditional handover / Success / A3 / A5 / A3+A5

8.2.4.31.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting conditional handover and receiving RRCConnectionReconfiguration message includes the conditionalReconfiguration }

ensure that {

when { any CHO execution condition is not satisfied }
    then { UE maintains connection with source eNB and starts evaluating the CHO execution conditions for the candidate cell(s) }

}

(2)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and only event A3 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A3 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(3)

with {  UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when {  UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and only event A5 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A5 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(4)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and both event A3 and event A5 are configured as conditional handover trigger events }
    then { UE starts evaluating the CHO execution condition A3 and A5 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(5)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE perform conditional handover procedure successfully }
    then { UE releases the stored CHO configurations }

}

8.2.4.31.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.9.1, 5.3.5.9.4, 5.3.5.9.5 and 5.3.5.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.5.9.1]

The network configures the UE with conditional reconfiguration (i.e. conditional handover) including per candidate target cell an RRCConnectionReconfiguration to be stored and to be applied upon the fulfilment of an associated execution condition.

The UE shall:

1>	if the received conditionalReconfiguration includes the condReconfigurationToRemoveList:

2>	perform the conditional reconfiguration removal procedure as specified in 5.3.5.9.2;

1>	if the received conditionalReconfiguration includes the condReconfigurationToAddModList:

2> perform the conditional reconfiguration addition/modification procedure as specified in 5.3.5.9.3;

[TS 36.331, clause 5.3.5.9.4]

If AS security has been activated successfully, the UE shall:

1>	if VarConditionalReconfiguration includes at least one condReconfigurationId:

2>	perform conditional reconfiguration evaluation;

1>	for each condReconfigurationId within the VarConditionalReconfiguration:

2>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon within condReconfigurationToApply to be an applicable cell;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the triggerCondition associated to condReconfigurationId:

3>	if the entry condition(s) applicable for this event associated with the condReconfigurationId, i.e. the event corresponding with the condEventId of the corresponding condReconfigurationTriggerEUTRA within VarConditionalReconfiguration, is fulfilled for the applicable cell for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfiguration:

4> consider the entry condition for the associated measId within triggerCondition as fulfilled;

3> if the leaving condition(s) applicable for this event associated with the condReconfigurationId, i.e. the event corresponding with the condEventId(s) of the corresponding condReconfigurationTriggerEUTRA within VarConditionalReconfiguration, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfiguration:

4> consider the event associated to that measId to be not fulfilled;

2>	if trigger conditions for all associated measId(s) within triggerCondition are fulfilled:

3> consider the target cell candidate within the stored condReconfigurationToApply, associated to that condReconfigurationId, as a triggered cell;

3> initiate the conditional reconfiguration execution, as specified in 5.3.5.9.5;

[TS 36.331, clause 5.3.5.9.5]

The UE shall:

1>	if more than one triggered cell exists:

2>	select one of the triggered cells as the selected cell for conditional reconfiguration;

1>	for the selected cell of conditional reconfiguration:

2>	apply the stored condReconfigurationToApply associated to that condReconfigurationId and perform the actions as specified in 5.3.5.4;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.9;

NOTE 6:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

8.2.4.31.1.3	Test description

8.2.4.31.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell4.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.31.1.3.2	Test procedure sequence

Table 8.2.4.31.1.3.2-1 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", "T2" and "T3", are to be applied in the test procedure in Table 8.2.4.31.1.3.2-2.

Table 8.2.4.31.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -82 | -91 | -91 | Power levels are such that UE registered on Cell 1 and entry condition for event A3 is not satisfied |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -91 | -91 | -79 | Power levels are such that entry condition for event A5 is satisfied for Cell 4 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -91 | -91 | Power levels are such that entry condition for event A3 and event A5 are satisfied for Cell 1 |

Table 8.2.4.31.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration to set Cell 2 and Cell 4 as target candidate cells and configure event A3 as trigger event in Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE attempt to perform the handover using MAC Random Access Preamble on Cell 2 within the next 5s? | --> | (PRACH Preamble) | 1 | F |
| 4 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 5 | Check: Does the UE transmits RRCConnectionReconfigurationComplete message in Cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 2? | - | - | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration to set Cell 1 and Cell 4 as target candidate cells and configure event A5 as trigger event in Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 10 | Check: Does the UE ransmits RRCConnectionReconfigurationComplete in Cell 4. | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 4? | - | - | 3 | P |
| 12 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration for intra-frequency event A3 and A5 in Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 4. | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 15 | Check: Does the UE ransmits RRCConnectionReconfigurationComplete message in Cell 1. | --> | RRCConnectionReconfigurationComplete | 4 | P |
| 16 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 1? | - | - | 4 | P |
| 17 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 18 | Check: Does the UE transmit RRCConnectionReconfigurationComplete message in Cell 2 within 10s? | --> | RRCConnectionReconfigurationComplete | 5 | F |

8.2.4.31.1.3.3	Specific message contents

Table 8.2.4.31.1.3.3-1: RRCConnectionReconfiguration (Step 1, step 7 and step 12, Table 8.2.4.31.1.3.2-3)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT | Table 8.2.4.31.1.3.3-2 |  |
| mobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.2.4.31.1.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-2: MeasConfig-DEFAULT (Table 8.2.4.31.1.3.3-1)

| Derivation Path: TS 36.508 Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| MeasObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC | Table 8.2.4.31.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A3-CHO | Table 8.2.4.31.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A5-CHO(-80,-86) with condition Not RSRQ |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxMeasId)) OF MeasIdToAddMod { | 1 entry |  | Step 1, Step 7 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  | Step 1 |
|  | 2 |  | Step 7 |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxMeasId)) OF MeasIdToAddMod { | 2 entries |  | Step 12 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-3: MeasObjectEUTRA-GENERIC (Table 8.2.4.31.1.3.3-2)

| Derivation Path: TS 36.508 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC::= SEQUENCE { |  |  |  |
| carrierFreq | ARFCN-ValueEUTRA for Cell 1 |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-4: ReportConfigEUTRA-A3-CHO (Table 8.2.4.31.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.6-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| condReconfigurationTriggerEUTRA-r16 ::= SEQUENCE { |  |  |  |
| condEventId-r16 CHOICE { |  |  |  |
| condEventA3-r16 SEQUENCE { |  |  |  |
| a3-Offset-r16 | 2 | 1 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-5: ConditionalReconfiguration (Table 8.2.4.31.1.3.3-1)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 | CondReconfigurationToAddModList-r16 (Step 1 and step 9) | Table 8.2.4.31.1.3.3-6 | Step 1, Step 7 |
|  | CondReconfigurationToAddModList-r16 (Step 16) | Table 8.2.4.31.1.3.3-9 | Step 12 |
| condReconfigurationToRemoveList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-6: CondReconfigToAddModList-r16 (Step 1 and step 7, Table 8.2.4.31.1.3.3-5)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16 { | 2 entries |  |  |
| CondReconfigurationAddMod-r16 [1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 2 | Table 8.2.4.31.1.3.3-7 | Step 1 |
|  | RRCConnectionReconfiguration with condition HO and HO Cell 1 | Table 8.2.4.31.1.3.3-7 | Step 7 |
| } |  |  |  |
| CondReconfigurationAddMod-r16 [2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 4 | Table 8.2.4.31.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-7: RRCConnectionReconfiguration (Table 8.2.4.31.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo | Table 8.2.4.31.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-8: MobilityControlInfo (Table 8.2.4.31.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | HO Cell 2 |
|  | PhysicalCellIdentity of Cell 1 |  | HO Cell 1 |
|  | PhysicalCellIdentity of Cell 4 |  | HO Cell 4 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | HO Cell 2 |
|  | Same downlink EARFCN as used for Cell 1 |  | HO Cell 1 |
|  | Same downlink EARFCN as used for Cell 4 |  | HO Cell 4 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, HO Cell 2 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, HO Cell 1 |
|  | Same uplink EARFCN as used for Cell 4 |  | Band 24 High range, HO Cell 4 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.2.4.31.1.3.3-9: CondReconfigurationToAddModList-r16 (Step 16, Table 8.2.4.31.1.3.3-6)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16{ | 2 entries |  |  |
| CondReconfigurationAddMod-r16 [1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| MeasId [2] | 2 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 1 | Table 8.2.4.31.1.3.3-7 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16 [2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 2 | Table 8.1.4.4.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.31.2	Conditional handover / modify conditional handover configuration

8.2.4.31.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting conditional handover and receiving RRCConnectionReconfiguration message including the conditionalReconfiguration }

ensure that {

when { UE received another conditionalReconfiguration including the condReconfigurationToRemoveList before any CHO execution condition is satisfied }

then { UE removes the entry with the matching condReconfigurationId from the condReconfigurationList within the VarConditionalReconfiguration }

}

(2)

with { UE in RRC_CONNECTED state and supporting conditional handover and receiving RRCConnectionReconfiguration message including the conditionalReconfiguration }

ensure that {

when { UE received another conditionalReconfiguration including the condReconfigurationToAddModList before any CHO execution condition is satisfied }

then { UE performs the conditional reconfiguration addition/modification procedure }

}

8.2.4.31.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1a.1 and 10.1.2.1a.2 and TS 36.331 clause 5.3.5.3, 5.3.5.9.1, 5.3.5.9.2 and 5.3.5.9.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1a.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) for CHO candidate cells upon receiving the CHO configuration, and executes CHO once the execution condition(s) are met for a CHO candidate cell. UE stops evaluating the execution condition(s) once the handover is executed (legacy handover or conditional handover execution).

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by each CHO candidate cell and execution condition(s) generated by the source cell.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	UE maintains connection with source eNB until UE determines a CHO execution condition is met for CHO candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 10.1.2.1, regardless of any previously received CHO configuration.

-	After source eNB sends CHO command to UE, the network is allowed to change source eNB configuration and network can add, modify or release a configured CHO configuration using RRC message (i.e. until UE starts executing CHO.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

NOTE 1:	CHO is not supported for S1 based handover in this release of the specification.

NOTE 2:	In case LTE-DC is configured, CHO is only supported in MeNB to eNB change procedure in this release of the specification.

[TS 36.300, clause 10.1.2.1a.2]

The figure below depicts the CHO scenario where neither MME nor Serving Gateway changes:

Figure 10.1.2.1a-1: Intra-MME/Serving Gateway Conditional Handover

1.	The source eNB configures the UE with measurement configuration, which may be used by the UE to trigger Measurement Reports for potential CHO candidate cell(s).

2.	A MEASUREMENT REPORT is triggered and sent to the source eNB.

3.	The source eNB makes decision on the usage of CHO to handoff the UE based on MEASUREMENT REPORT information.

4.	The source eNB requests a CHO to the eNB(s) of candidate cell(s). A CHO request message is sent for each candidate cell.

5.	Same as step 5 in Figure 10.1.2.1.1-1 of clause 10.1.2.1.1.

6.	The eNB(s) of candidate cell(s) sends CHO response including configuration of CHO candidate cell(s) to the source eNB. The response message is also sent for each candidate cell.

7.	The source eNB sends a RRCConnectionReconfiguration message to the UE, containing configuration of CHO candidate cell(s) and CHO execution condition(s). The source eNB decides on the condition for the execution of CHO and adds the condition(s) to the RRC message sent to the UE.

NOTE 1:	The source eNB may reconfigure the UE's source configuration after providing CHO configuration for candidate target cell(s).

NOTE 1a:	A configuration of a CHO candidate cell cannot contain a DAPS handover.

8.	The UE sends an RRCConnectionReconfigurationComplete message to the source eNB.

8a.	If early data forwarding is applied, the source eNB sends the EARLY STATUS TRANSFER message.

9.	The UE maintains connection with the source eNB after receiving CHO configuration, and starts evaluating the CHO execution condition(s) for the CHO candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source eNB, applies the stored corresponding configuration for that candidate cell and synchronises to that candidate cell.

10-11. The UE accesses to the target eNB and completes the handover procedure by sending RRCConnectionReconfigurationComplete message to the target eNB. The UE releases the stored CHO configurations after successful completion of RRC handover procedure.

11a/b.	The target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message.

NOTE 2:	Late data forwarding may be initiated as soon as the source eNB receives the HANDOVER SUCCESS message.

11c.	The source eNB sends the HANDOVER CANCEL message toward the other signalling connections or other potential target eNBs, if any, to cancel CHO for the UE.

12.	Steps 12-18 as in Figure 10.1.2.1.1-1.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.9;

NOTE 6:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

[TS 36.331, clause 5.3.5.9.1]

The network configures the UE with conditional reconfiguration (i.e. conditional handover) including per candidate target cell an RRCConnectionReconfiguration to be stored and to be applied upon the fulfilment of an associated execution condition.

The UE shall:

1>	if the received conditionalReconfiguration includes the condReconfigurationToRemoveList:

2>	perform the conditional reconfiguration removal procedure as specified in 5.3.5.9.2;

1>	if the received conditionalReconfiguration includes the condReconfigurationToAddModList:

2> perform the conditional reconfiguration addition/modification procedure as specified in 5.3.5.9.3;

[TS 36.331, clause 5.3.5.9.2]

The UE shall:

1>	for each CondReconfigurationId included in the condReconfigurationToRemoveList that is part of the current UE configuration in VarConditionalReconfiguration:

2>	remove the entry with the matching condReconfigurationId from the condReconfigurationList within the VarConditionalReconfiguration.

NOTE:	The UE does not consider the message as erroneous if the condReconfigurationToRemoveList includes any CondReconfigurationId value that is not part of the current UE configuration.

[TS 36.331, clause 5.3.5.9.3]

The UE shall:

1>	for each condReconfigurationId included in the condReconfigurationToAddModList:

2>	if an entry with the matching condReconfigurationId exists in the condReconfigurationList within the VarConditionalReconfiguration:

3>	if the entry in condReconfigurationToAddModList includes a triggerCondition;

4>	replace triggerCondition within the VarConditionalReconfiguration with the value received for this condReconfigurationId

3>	if the entry in condReconfigurationToAddModList includes an condReconfigurationToApply;

4>	replace condReconfigurationToApply within the VarConditionalReconfiguration with the value received for this condReconfigurationId;

2>	else:

3>	add a new entry for this condReconfigurationId within the VarConditionalReconfiguration;

3>	store the associated RRCConnectionReconfiguration in VarConditionalReconfiguration.

8.2.4.31.2.3	Test description

8.2.4.31.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 Cell 2 and Cell 4.

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.31.2.3.2	Test procedure sequence

Table 8.2.4.31.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.31.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | -91 | Power levels are such that UE attached on Cell 1 and entry condition for event A3 is not satisfied |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | -79 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 and Cell 4 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -91 | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied for Cell 4 |

Table 8.2.4.31.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration in Cell 1 to set Cell 2 and Cell 4 as target candidate cells. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message including condReconfigurationToRemoveList in Cell 1 to remove Cell 2 from the target candidate cell list. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 6 | Check: Does the UE attempt to perform the handover using MAC Random Access Preamble on Cell 2 within the next 5s? | --> | (PRACH Preamble) | 1 | F |
| 7 | The SS transmits an RRCConnectionReconfiguration message including condReconfigurationToAddModList in Cell 1 to replace Cell 4 with Cell 2 as the target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 11 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration in Cell 2 to set Cell 1 as target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration in Cell 2 to add Cell 4 as target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 15 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4? | --> | RRCConnectionReconfigurationComplete | 2 | P |

8.2.4.31.2.3.3	Specific message contents

Table 8.2.4.31.2.3.3-1: RRCConnectionReconfiguration (step 1, step 3, step 7, step 11 and step 13, Table 8.2.4.31.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  | Step 3, Step 7, Step 13 |
|  | MeasConfig | Table 8.2.4.31.2.3.3-2 | Step 1, Step 11 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration-r16-HO(Step 1) | Table 8.2.4.31.2.3.3-3 | Step 1 |
|  | ConditionalReconfiguration-r16-HO(Step 3) | Table 8.2.4.31.2.3.3-5 | Step 3 |
|  | ConditionalReconfiguration-r16-HO(Step 7) | Table 8.2.4.31.2.3.3-6 | Step 7 |
|  | ConditionalReconfiguration-r16-HO(Step 13) | Table 8.2.4.31.2.3.3-7 | Step 11 |
|  | ConditionalReconfiguration-r16-HO(Step 15) | Table 8.2.4.31.2.3.3-8 | Step 13 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-2: MeasConfig (Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-CHO | Table 4.6.6-17 specified in TS 36.508 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-3: ConditionalReconfiguration-r16-HO (Step 1, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 2 entries |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 2 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 4 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-4: ConditionalReconfiguration-r16-HO (Step 1, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 2 entries |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 2 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 4 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-5: RRCConnectionReconfiguration (Table 8.2.4.31.2.3.3-3)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  | HO_to_Cell 1 |
|  | PhysicalCellIdentity of Cell 2 |  | HO_to_Cell 1 |
|  | PhysicalCellIdentity of Cell 4 |  | HO_to_Cell 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-6: ConditionalReconfiguration-r16-HO (Step 3, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| CondReconfigurationToRemoveList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationId-r16[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-7: ConditionalReconfiguration-r16-HO (Step 7, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 2 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-8: ConditionalReconfiguration-r16-HO (Step 11, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 1 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.2.3.3-9: ConditionalReconfiguration-r16-HO (Step 13, Table 8.2.4.31.2.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 4 | Table 8.2.4.31.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.2.4.31.3	Conditional handover / Failure

8.2.4.31.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting conditional handover and network configured the UE to try CHO after HO/CHO failure and UE receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE detects conditional handover failure and performs cell selection to a CHO candidate cell before T311 expires }

then { UE perform conditional handover to the selected cell }

}

(2)

with { UE in RRC_CONNECTED state and supporting conditional handover and network configured the UE to try CHO after HO/CHO failure and UE receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE detects conditional handover failure and only finds a selectable cell which is not the candidate cell included in ConditionalReconfiguration before T311 expires }

then { UE initiate the connection re-establishment procedure in the selected cell }

}

(3)

with { UE in RRC_CONNECTED state and supporting conditional handover and network configured the UE to try CHO after HO/CHO failure and UE receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE detects conditional handover failure and cannot find any selectable cell before T311 expires }

then { UE goes to RRC_IDLE and release the stored conditional handover configurations }

}

8.2.4.31.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1.0 and TS 36.331 clause 5.3.5.3, 5.3.7.3and 5.3.7.6. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1.0]

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	Part of the HO command comes from the target eNB and is transparently forwarded to the UE by the source eNB;

-	To prepare the HO, the source eNB passes all necessary information to the target eNB (e.g. E-RAB attributes and RRC context):

-	When CA is configured and to enable SCell selection in the target eNB, the source eNB can provide in decreasing order of radio quality a list of the best cells and optionally measurement result of the cells.

-	When DC is configured, the source MeNB provides the SCG configuration (in addition to the MCG configuration) to the target MeNB.

-	Both the source eNB and UE keep some context (e.g. C-RNTI) to enable the return of the UE in case of HO failure;

-	If RACH-less HO is not configured, the UE accesses the target cell via RACH following a contention-free procedure using a dedicated RACH preamble or following a contention-based procedure if dedicated RACH preambles are not available:

-	the UE uses the dedicated preamble until the handover procedure is finished (successfully or unsuccessfully);

-	If RACH-less HO is configured, the UE accesses the target cell via the uplink grant preallocated to the UE in the RRC message. If the UE does not receive the preallocated uplink grant in the RRC message from the source eNB, the UE monitors the PDCCH of the target cell;

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB. Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	UE maintains only PCell connection with both source and target eNBs. Any other configured serving cells, NR sidelink configurations and V2X sidelink configurations are released by the network before the handover command is sent to the UE.

NOTE:	Void.

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell except in DAPS handover or CHO scenarios:

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

-	When initial CHO execution attempt fails or Handover fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

-	No ROHC and EHC context is transferred at handover;

-	No UDC context is transferred at handover;

-	ROHC and EHC contexts can be kept at handover within the same eNB.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.9;

NOTE 6:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

[TS 36.331, clause 5.3.7.3]

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	if T309 is running:

…

1>	else:

2>	stop timer T311;

2>	if the cell selection is triggered by detecting radio link failure of the MCG or handover failure (including intra-E-UTRA handover and mobility from E-UTRA); and

2>	if attemptCondReconf is configured; and

2>	if the selected cell is one of the target candidate cells in VarConditionalReconfiguration:

3>	apply the stored condReconfigurationToApply of the selected cell and perform the actions as specified in 5.3.5.4;

2> else:

3>	if the UE is configured with conditionalReconfiguration:

4>	release uplinkDataCompression, if configured;

4>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

4>	reset MAC;

4>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

4>	release the SCell group(s), if configured, in accordance with 5.3.10.3d;

4>	apply the default physical channel configuration as specified in 9.2.4;

4>	for the MCG, apply the default semi-persistent scheduling configuration as specified in 9.2.3;

4>	for the MCG, apply the default MAC main configuration as specified in 9.2.2;

4>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

4>	release reportProximityConfig, if configured and clear any associated proximity status reporting timer;

4>	release obtainLocationConfig, if configured;

4>	release idc-Config, if configured;

4>	release sps-AssistanceInfoReport, if configured;

4>	release measSubframePatternPCell, if configured;

4>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

4>	if (NG)EN-DC is configured:

5>	perform MR-DC release, as specified in TS 38.331[82], clause 5.3.5.10;

5>	release p-MaxEUTRA, if configured;

5>	release p-MaxUE-FR1, if configured;

5>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

4>	release naics-Info for the PCell, if configured;

4>	if connected as an RN and configured with an RN subframe configuration:

5>	release the RN subframe configuration;

4>	release the LWA configuration, if configured, as described in 5.6.14.3;

4>	release the LWIP configuration, if configured, as described in 5.6.17.3;

4>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

4>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

4>	release overheatingAssistanceConfig and overheatingAssistanceConfigForSCG, if configured and stop timer T345, if running;

4>	release ailc-BitConfig, if configured;

3>	remove all the entries within VarConditionalReconfiguration, if any;

3>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA configured:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

3>	start timer T301;

3>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

3>	if the UE is a NB-IoT UE connected to EPC, the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation and AS security has not been activated; and

3>	if cp-reestablishment is not included in SystemInformationBlockType2-NB:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	if the selected cell is a UTRA cell, and if the UE supports Radio Link Failure Report for Inter-RAT MRO, include selectedUTRA-CellId in the VarRLF-Report and set it to the physical cell identity and carrier frequency of the selected UTRA cell;

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

8.2.4.31.3.3	Test description

8.2.4.31.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 Cell 2 and Cell 4.

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.31.3.3.2	Test procedure sequence

Table 8.2.4.31.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.31.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | off | off | Power levels are such that UE attached on Cell 1 and entry condition for event A3 is not satisfied |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | -81 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 and Cell 4 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -113 | -113 | -79 | The power level is such that only SrxlevNR Cell 4 > 0 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -81 | -91 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 and Cell 4 |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -113 | -79 | -113 | The power level is such that only SrxlevNR Cell 2 > 0 |
| T5 | Cell-specific RS EPRE | dBm/15kHz | off | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied for Cell 4 |
| T6 | Cell-specific RS EPRE | dBm/15kHz | off | -113 | -113 | The power level is such that all SrxlevNR Cell < 0 |
| T7 | Cell-specific RS EPRE | dBm/15kHz | off | off | -85 | The power level is such that only SrxlevNR Cell 4 > 0 |

Table 8.2.4.31.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration in Cell 1 to set Cell 2 and Cell 4 as target candidate cells. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| - | EXCEPTION: The step 4 below is repeated preambleTransMax times for the duration of T304. (Note 1) | - | - | - | - |
| 4 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 2. | - | - | - | - |
| 5 | The SS does not respond on Cell 2. | - | - | - | - |
| 6 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 7 | The SS starts timer T311 when the number of Random Access Preamble repetition from UE reaches preambleTransMax. |  |  |  |  |
| 8 | Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 4 before T311 expires? | --> | RRCReconfigurationComplete | 1 | P |
| 9 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration in Cell 4 to set Cell 1 as target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 4. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| - | EXCEPTION: The step 12 below is repeated preambleTransMax times for the duration of T304. (Note 1) | - | - | - | - |
| 12 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 1. | - | - | - | - |
| 13 | The SS does not respond on Cell 1. | - | - | - | - |
| 14 | SS adjusts the cell-specific reference signal level according to row "T4". | - | - | - | - |
| 15 | The SS starts timer T311 when the number of Random Access Preamble repetition from UE reaches preambleTransMax. |  |  |  |  |
| 16 | Check: Does the UE transmit RRCConnectionReestablishmentRequest message in NR Cell 2 before T311 expires? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 17 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation and re-activate security on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 18 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 19 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer and set Cell 4 as target candidate cell on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 20 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 21 | SS adjusts the cell-specific reference signal level according to row "T5". | - | - | - | - |
| - | EXCEPTION: The step 22 below is repeated preambleTransMax times for the duration of T304. (Note 1) | - | - | - | - |
| 22 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 4. | - | - | - | - |
| 23 | The SS does not respond on Cell 4. | - | - | - | - |
| 24 | SS adjusts the cell-specific reference signal level according to row "T6". | - | - | - | - |
| 25 | The SS starts timer T311 when the number of Random Access Preamble repetition from UE reaches preambleTransMax and wait timer T311 expires. |  |  |  |  |
| 26 | The SS changes the TAC for Cell 4 to 2. | - | - | - | - |
| 27 | SS adjusts the cell-specific reference signal level according to row "T7". | - | - | - | - |
| 28 | Check: Does the generic test procedure in TS 36.508 subclause 6.4.2.7 is performed on Cell 4? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| Note 1: The default value of preambleTransMax is set to n6. |  |  |  |  |  |

8.2.4.31.3.3.3	Specific message contents

Table 8.2.4.31.3.3.3-1: RRCConnectionReconfiguration (step 1, step 9, step 19, Table 8.2.4.31.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.2.4.31.3.3.3-2 | Step 1, Step 9, Step 19 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration-r16-HO(Step 1) | Table 8.2.4.31.3.3.3-3 | Step 1 |
|  | ConditionalReconfiguration-r16-HO(Step 9) | Table 8.2.4.31.3.3.3-5 | Step 9 |
|  | ConditionalReconfiguration-r16-HO(Step 19) | Table 8.2.4.31.3.3.3-6 | Step 19 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-2: MeasConfig (Table 8.2.4.31.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-CHO | Table 4.6.6-17 specified in TS 36.508 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-3: ConditionalReconfiguration-r16-HO (Step 1, Table 8.2.4.31.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 2 entries |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 2 | Table 8.2.4.31.3.3.3-4 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 4 | Table 8.2.4.31.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-4: RRCConnectionReconfiguration (Table 8.2.4.31.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  | HO_to_Cell 1 |
|  | PhysicalCellIdentity of Cell 2 |  | HO_to_Cell 2 |
|  | PhysicalCellIdentity of Cell 4 |  | HO_to_Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-5: ConditionalReconfiguration-r16-HO (Step 9, Table 8.2.4.31.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 1 | Table 8.2.4.31.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-6: ConditionalReconfiguration-r16-HO (Step 19, Table 8.2.4.31.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO_to_Cell 4 | Table 8.2.4.31.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.3.3.3-7: SystemInformationBlockType1 for Cell 4 (step 26, Table 8.2.4.31.3.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 2 |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

##### 8.2.4.31.4	Conditional handover / Handover / Handover Failure

8.2.4.31.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting conditional handover and receiving RRCConnectionReconfiguration message including the conditionalReconfiguration }

ensure that {

when { UE receives HO command (without CHO configuration) before any CHO execution condition is satisfied }

then { UE executes the HO procedure, regardless of any previously received CHO configuration }

}

(2)

with { UE having stored conditionalReconfiguration and receiving another RRCConnectionReconfiguration message including the mobilityControlInfo for handover to a neighbour cell }

ensure that {

when { UE detects handover failure and finds a selectable cell which is the candidate cell included in conditionalReconfiguration before T311 expires }

then { UE applies the stored conditionalReconfiguration associated to the selected cell and perform conditional handover to the selected cell }

}

(3)

with { UE having stored conditionalReconfiguration and receiving another RRCConnectionReconfiguration message including the mobilityControlInfo for handover to a neighbour cell }

ensure that {

when { UE detects handover failure and cannot find any selectable cell before T311 expires }

then { UE goes to RRC_IDLE and releases the stored conditional handover configurations }

}

8.2.4.31.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 10.1.2.1.0, 10.1.2.1a.1 and 10.1.2.1a.2 and TS 36.331 clause 5.3.7.3 and 5.3.12. Unless otherwise stated these are Rel-16 requirements.

[TS 36.300, clause 10.1.2.1.0]

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	Part of the HO command comes from the target eNB and is transparently forwarded to the UE by the source eNB;

-	To prepare the HO, the source eNB passes all necessary information to the target eNB (e.g. E-RAB attributes and RRC context):

-	When CA is configured and to enable SCell selection in the target eNB, the source eNB can provide in decreasing order of radio quality a list of the best cells and optionally measurement result of the cells.

-	When DC is configured, the source MeNB provides the SCG configuration (in addition to the MCG configuration) to the target MeNB.

-	Both the source eNB and UE keep some context (e.g. C-RNTI) to enable the return of the UE in case of HO failure;

-	If RACH-less HO is not configured, the UE accesses the target cell via RACH following a contention-free procedure using a dedicated RACH preamble or following a contention-based procedure if dedicated RACH preambles are not available:

-	the UE uses the dedicated preamble until the handover procedure is finished (successfully or unsuccessfully);

-	If RACH-less HO is configured, the UE accesses the target cell via the uplink grant preallocated to the UE in the RRC message. If the UE does not receive the preallocated uplink grant in the RRC message from the source eNB, the UE monitors the PDCCH of the target cell;

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB. Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	UE maintains only PCell connection with both source and target eNBs. Any other configured serving cells, NR sidelink configurations and V2X sidelink configurations are released by the network before the handover command is sent to the UE.

NOTE:	Void.

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell except in DAPS handover or CHO scenarios:

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

-	When initial CHO execution attempt fails or Handover fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

-	No ROHC and EHC context is transferred at handover;

-	No UDC context is transferred at handover;

-	ROHC and EHC contexts can be kept at handover within the same eNB.

[TS 36.300, clause 10.1.2.1a.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) for CHO candidate cells upon receiving the CHO configuration, and executes CHO once the execution condition(s) are met for a CHO candidate cell. UE stops evaluating the execution condition(s) once the handover is executed (legacy handover or conditional handover execution).

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by each CHO candidate cell and execution condition(s) generated by the source cell.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	UE maintains connection with source eNB until UE determines a CHO execution condition is met for CHO candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 10.1.2.1, regardless of any previously received CHO configuration.

-	After source eNB sends CHO command to UE, the network is allowed to change source eNB configuration and network can add, modify or release a configured CHO configuration using RRC message (i.e. until UE starts executing CHO.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

NOTE 1:	CHO is not supported for S1 based handover in this release of the specification.

NOTE 2:	In case LTE-DC is configured, CHO is only supported in MeNB to eNB change procedure in this release of the specification.

[TS 36.300, clause 10.1.2.1a.2]

The figure below depicts the CHO scenario where neither MME nor Serving Gateway changes:

Figure 10.1.2.1a-1: Intra-MME/Serving Gateway Conditional Handover

1.	The source eNB configures the UE with measurement configuration, which may be used by the UE to trigger Measurement Reports for potential CHO candidate cell(s).

2.	A MEASUREMENT REPORT is triggered and sent to the source eNB.

3.	The source eNB makes decision on the usage of CHO to handoff the UE based on MEASUREMENT REPORT information.

4.	The source eNB requests a CHO to the eNB(s) of candidate cell(s). A CHO request message is sent for each candidate cell.

5.	Same as step 5 in Figure 10.1.2.1.1-1 of clause 10.1.2.1.1.

6.	The eNB(s) of candidate cell(s) sends CHO response including configuration of CHO candidate cell(s) to the source eNB. The response message is also sent for each candidate cell.

7.	The source eNB sends a RRCConnectionReconfiguration message to the UE, containing configuration of CHO candidate cell(s) and CHO execution condition(s). The source eNB decides on the condition for the execution of CHO and adds the condition(s) to the RRC message sent to the UE.

NOTE 1:	The source eNB may reconfigure the UE's source configuration after providing CHO configuration for candidate target cell(s).

NOTE 1a:	A configuration of a CHO candidate cell cannot contain a DAPS handover.

8.	The UE sends an RRCConnectionReconfigurationComplete message to the source eNB.

8a.	If early data forwarding is applied, the source eNB sends the EARLY STATUS TRANSFER message.

9.	The UE maintains connection with the source eNB after receiving CHO configuration, and starts evaluating the CHO execution condition(s) for the CHO candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source eNB, applies the stored corresponding configuration for that candidate cell and synchronises to that candidate cell.

10-11. The UE accesses to the target eNB and completes the handover procedure by sending RRCConnectionReconfigurationComplete message to the target eNB. The UE releases the stored CHO configurations after successful completion of RRC handover procedure.

11a/b.	The target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message.

NOTE 2:	Late data forwarding may be initiated as soon as the source eNB receives the HANDOVER SUCCESS message.

11c.	The source eNB sends the HANDOVER CANCEL message toward the other signalling connections or other potential target eNBs, if any, to cancel CHO for the UE.

12.	Steps 12-18 as in Figure 10.1.2.1.1-1.

[TS 36.331, clause 5.3.7.3]

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	if the UE is connected to 5GC and the selected cell is only connected to EPC; or

1>	if the UE is connected to EPC and the selected cell is only connected to 5GC:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

1>	else:

2>	stop timer T311;

2>	if the cell selection is triggered by detecting radio link failure of the MCG or handover failure (including intra-E-UTRA handover and mobility from E-UTRA); and

2>	if attemptCondReconf is configured; and

2>	if the selected cell is one of the target candidate cells in VarConditionalReconfiguration:

3>	apply the stored condReconfigurationToApply of the selected cell and perform the actions as specified in 5.3.5.4;

2> else:

3>	if the UE is configured with conditionalReconfiguration:

4>	release uplinkDataCompression, if configured;

4>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

4>	reset MAC;

4>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

4>	release the SCell group(s), if configured, in accordance with 5.3.10.3d;

4>	apply the default physical channel configuration as specified in 9.2.4;

4>	for the MCG, apply the default semi-persistent scheduling configuration as specified in 9.2.3;

4>	for the MCG, apply the default MAC main configuration as specified in 9.2.2;

4>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

4>	release reportProximityConfig, if configured and clear any associated proximity status reporting timer;

4>	release obtainLocationConfig, if configured;

4>	release idc-Config, if configured;

4>	release sps-AssistanceInfoReport, if configured;

4>	release measSubframePatternPCell, if configured;

4>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

4>	if (NG)EN-DC is configured:

5>	perform MR-DC release, as specified in TS 38.331[82], clause 5.3.5.10;

5>	release p-MaxEUTRA, if configured;

5>	release p-MaxUE-FR1, if configured;

5>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

4>	release naics-Info for the PCell, if configured;

4>	if connected as an RN and configured with an RN subframe configuration:

5>	release the RN subframe configuration;

4>	release the LWA configuration, if configured, as described in 5.6.14.3;

4>	release the LWIP configuration, if configured, as described in 5.6.17.3;

4>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

4>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

4>	release overheatingAssistanceConfig and overheatingAssistanceConfigForSCG, if configured and stop timer T345, if running;

4>	release ailc-BitConfig, if configured;

3>	remove all the entries within VarConditionalReconfiguration, if any;

3>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA configured:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

3>	start timer T301;

3>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

3>	if the UE is a NB-IoT UE connected to EPC, the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation and AS security has not been activated; and

3>	if cp-reestablishment is not included in SystemInformationBlockType2-NB:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall:

1>	reset MAC;

…

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

2>	re-establish RLC entities for all SRBs and DRBs, including RBs configured with NR PDCP;

2>	remove all entries within VarConditionalReconfiguration, if any;

2>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA configured:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

…

NOTE 1:	Except when resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18, ciphering is not applied for the subsequent RRCConnectionResume message used to resume the connection and an integrity check is performed by lower layers, but merely upon request from RRC.

1>	else:

2>	upon leaving RRC_INACTIVE:

3>	discard the UE Inactive AS context;

3>	discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key;

2>	release rrc-InactiveConfig, if configured;

2>	remove all entries within VarConditionalReconfiguration, if any;

2>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA configured:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	release all radio resources, including release of the MAC configuration, the RLC entity and the associated PDCP entity and SDAP (if any) for all established RBs, except for the following:

-	pur-Config, if stored;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

…

8.2.4.31.4.3	Test description

8.2.4.31.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 4.

UE:

-	None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.31.4.3.2	Test procedure sequence

Table 8.2.4.31.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.31.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -113 | -88 | Power levels are such that UE registered on Cell 1 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -113 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -113 | -113 | -113 | Power levels are such that all Srxlev Cell < 0 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -113 | -79 | -113 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 |

Table 8.2.4.31.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration on Cell 1 to set Cell 2 and Cell 4 as target candidate cells. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform legacy handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 5 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration on Cell 4 to set Cell 2 as target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message to Cell 4. | --> | RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform legacy handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.2.4.31.4.3.2-3 should take place. | - | - | - | - |
| 8 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 10 | The SS transmits an RRCConnectionReconfiguration message including conditionalReconfiguration on Cell 2 to set Cell 4 as target candidate cell. | <-- | RRCConnectionReconfiguration | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 12 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform legacy handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 12A | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 13 the steps specified in Table 8.2.4.31.4.3.2-3 should take place. | - | - | - | - |
| 13 | SS wait 10s to make sure T304+T311 expire. | - | - | - | - |
| 13A | The SS changes the TAC of Cell 2 to 2. | - | - | - | - |
| 14 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 15 | Check: Does the generic test procedure specified in TS 36.508 subclause 6.4.2.7 take place on Cell 2? | - | - | 3 | - |

Table 8.2.4.31.4.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 1. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.2.4.31.4.3.3	Specific message contents

Table 8.2.4.31.4.3.3-1: RRCConnectionReconfiguration (step 1, step 5, step 10, Table 8.2.4.31.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.2.4.31.4.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration-r16-HO | Table 8.2.4.31.4.3.3-3 | Step 1 |
|  | ConditionalReconfiguration-r16-HO | Table 8.2.4.31.4.3.3-5 | Step 5 |
|  | ConditionalReconfiguration-r16-HO | Table 8.2.4.31.4.3.3-6 | Step 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-2: MeasConfig (Table 8.2.4.31.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-CHO | Table 4.6.6-17 specified in TS 36.508 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-3: ConditionalReconfiguration-r16-HO (Step 1, Table 8.2.4.31.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 2 entries |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING (CONTAINING RRCConnectionReconfiguration with condition HO_to_Cell 2) | Table 8.2.4.31.4.3.3-4 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING (CONTAINING RRCConnectionReconfiguration with condition HO_to_Cell 4) | Table 8.2.4.31.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-4: RRCConnectionReconfiguration (Table 8.2.4.31.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  | HO_to_Cell 1 |
|  | PhysicalCellIdentity of Cell 2 |  | HO_to_Cell 2 |
|  | PhysicalCellIdentity of Cell 4 |  | HO_to_Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-5: ConditionalReconfiguration-r16-HO (Step 5, Table 8.2.4.31.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING (CONTAINING RRCConnectionReconfiguration with condition HO_to_Cell 2) | Table 8.2.4.31.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-6: ConditionalReconfiguration-r16-HO (Step 10, Table 8.2.4.31.4.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 SEQUENCE (SIZE (1..maxCondConfig-r16)) OF SEQUENCE { | 1 entry |  |  |
| CondReconfigurationAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING (CONTAINING RRCConnectionReconfiguration with condition HO_to_Cell 4) | Table 8.2.4.31.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-7: SystemInformationBlockType1 for Cell 2 (step 13A, Table 8.2.4.31.4.3.2-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInformation SEQUENCE { |  |  |  |
| trackingAreaCode | 2 |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 8.2.4.31.4.3.3-8: RRCConnectionReconfiguration (step 3, step 7 and step 12, Table 8.2.4.31.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.31.4.3.3-9: MobilityControlInfo (Table 8.2.4.31.4.3.3-8)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 4 |  | Step 3 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 7, step 12 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 4 |  | Step 3 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 7, step 12 |
| } |  |  |  |
| } |  |  |  |

### 8.2.5	LWA / LWIP / LWI

#### 8.2.5.1	LWA / WLAN Release / WLAN Association / EUTRA RRC_Connected to WLAN (Event W2)

8.2.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W2 reporting }

ensure that {
  when { UE  detects entering condition for Event W2 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToReleaseList and wlanToAddList }

then { UE releases the WLAN association and adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete  }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

8.2.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.12, 5.6.14.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.12]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both conditions W2-1 and W2-2 as specified below are fulfilled;

…

Inequality W2-1 (Entering condition 1)

Inequality W2-2 (Entering condition 2)

[TS 36.331, clause 5.6.14.2]

Upon reception of LWA configuration, the UE shall:

1>	if the received lwa-Configuration is set to release:

…

1>	else:

…

2>	if the received lwa-Config includes lwa-MobilityConfig:

3>	if the received lwa-MobilityConfig includes wlanToReleaseList:

4>	for each WLAN-Identifiers included in wlanToReleaseList:

5>	remove WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwa-MobilityConfig includes wlanToAddList:

4>	for each WLAN-Identifiers included in wlanToAddList:

5>	add WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwa-MobilityConfig includes associationTimer:

4>	start or restart timer T351 with the timer value set to the associationTimer;

3>	if the received lwa-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.1.3	Test description

8.2.5.1.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1, WLAN Cell 27 and WLAN Cell 27a

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.1.3.2	Test procedure sequence

Table 8.2.5.1.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 27 and Cell 27a at various time instants of the test execution. Rows marked "T1" and "T2" and “T3” are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Cell 27a | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -60 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | The power level values are such that conditions for entering Event W1 is met |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -50 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | The power level values are such that conditions for entering Event W2 is met |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -80 | -50 |  |

Table 8.2.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.1.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.1.3.2-1. | - | - | - | - |
| 5 | UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W2) | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell parameters according to the row "T3" in table 8.2.5.1.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report the Event W2 | --> | MeasurementReport | 1 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToReleaseList and Cell 27a in wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmits RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 15 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry? | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.1.3.3	Specific message contents

Table 8.2.5.1.3.3-1: RRCConnectionReconfiguration (step 2 and 9 Table 8.2.5.1.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.2.5.1.3.3-2: MeasConfig (step 2, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.1.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.1.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-3: MeasObjectWLAN-GENERIC (step 2, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-4: ReportConfigInterRAT-W1-WLAN (step 2, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.1.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-5: RRCConnectionReconfiguration (step 6 Table 8.2.5.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.1.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-6: LWA-Configuration (step 6 Table 8.2.5.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3-7: MeasConfig (step 9, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN-W2-GENERIC(Freq, SSID1, SSID2) | See Table 8.2.5.1.3.3-8. SSID1 is for Cell 27. SSID2 is for Cell 27a |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W2-WLAN(-75, -55) | Note: Set as per T3 setting for WLAN RSSI in Table 8.2.5.1.3.2-1. See Table 8.2.5.1.3.3-9. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W2 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-8: MeasObjectWLAN-W2-GENERIC (step 9, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 and Cell 27a | Band and Carrier information for Cell 27 & Cell 27a |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 2 entries |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell 27 |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 of [18] | Cell 27a |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-9: ReportConfigInterRAT-W2 (step 9, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW2-r13 SEQUENCE { | 2 entries |  |  |
| w2-Threshold1-r13 | 26 | Thresh1DB+101=26 where Thresh1DB = -75dBm. Note: Set as per T3 setting for Cell 27 WLAN RSSI plus 5db in Table 8.2.5.1.3.2-1. |  |
| w2-Threshold2-r13 | 46 | Thresh2DB+101=46 where Thresh2DB = -55dBm. Note: Set as per T3 setting for Cell 27a WLAN RSSI minus 5db in Table 8.2.5.1.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-10: RRCConnectionReconfiguration (step 13 Table 8.2.5.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-W2-GENERIC(SSID1, SSID2) Set as per Table 8.2.5.1.3.3-11 | SSID1 set as per Table 4.4.8-1 of [18]. SSID2 set as per Table 4.4.8-2 of [18]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.1.3.3-11: LWA-Configuration-W2-GENERIC (step 13 Table 8.2.5.1.3.2-1)

| Derivation Path: 36.508 clause 4.6.3 4.6.3-36 with condition SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell 27 |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-2 of [18] | Cell 27a |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | True |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.2	LWA / WLAN Release Success / EUTRA RRC_Connected from WLAN (Event W3)

8.2.5.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W3 reporting }

ensure that {
  when { UE  detects entering condition for Event W3 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToReleaseList that includes WLAN Identifiers that are part of the current WLAN mobility set }

then { UE releases the WLAN association and sends RRCConnectionReconfigurationComplete  }

}

8.2.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.13 and 5.6.14.2

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.13]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition W3-1, as specified below, is fulfilled;

…

Inequality W3-1 (Entering condition)

…

The variables in the formula are defined as follows:

Ms is the measurement result of WLAN(s) which matches all WLAN identifiers of at least one entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w3-Threshold as defined within reportConfigInterRAT for this event).

Ms is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.6.14.2]

Upon reception of LWA configuration, the UE shall:

1>	if the received lwa-Configuration is set to release:

…

1>	else:

…

2>	if the received lwa-Config includes lwa-MobilityConfig:

3>	if the received lwa-MobilityConfig includes wlanToReleaseList:

4>	for each WLAN-Identifiers included in wlanToReleaseList:

5>	remove WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

8.2.5.2.3	Test description

8.2.5.2.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.2.3.2	 Test procedure sequence

Table 8.2.5.2.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T1", "T2" and “T3” are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.2.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W1 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -50 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W3 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -80 |  |

Table 8.2.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.2.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.2.3.2-1. | - | - | - | - |
| 5 | UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W3) | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.2.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report the Event W3 | --> | MeasurementReport | 1 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToReleaseList | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmits RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |

8.2.5.2.3.3	Specific message contents

Table 8.2.5.2.3.3-1: RRCConnectionReconfiguration (step 2 and 9 Table 8.2.5.2.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.2.5.2.3-2: MeasConfig (step 2, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.2.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.2.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-3: MeasObjectWLAN-GENERIC (step 2, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-4: ReportConfigInterRAT-W1-WLAN (step 2, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.2.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-5: RRCConnectionReconfiguration (step 6 Table 8.2.5.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.2.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-6: LWA-Configuration-GENERIC (step 6 Table 8.2.5.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3-7: MeasConfig (step 9, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.2.3.3-8 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W3-WLAN(-75) | See Table 8.2.5.2.3.3-9 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-8: MeasObjectWLAN-GENERIC (step 9, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-9: ReportConfigInterRAT-W3-WLAN (step 9, Table 8.2.5.2.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW3-r13 SEQUENCE { | 1 entry |  |  |
| W3-Threshold-r13 | 26 | ThreshDB+101=26 where ThreshDB = -75dBm. Note: Set as per T3 setting for WLAN RSSI plus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-10: RRCConnectionReconfiguration (step 13 Table 8.2.5.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.2.3.3-11 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.2.3.3-11: LWA-Configuration-GENERIC (step 13 Table 8.2.5.2.3.2-1)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition RELEASE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 | Not Present |  |  |
| associationTimer-r13 | Not Present |  |  |
| successReportRequested-r13 | Not Present |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.3

#### 8.2.5.4	LWA / WLAN Association Success / EUTRA RRC_Connected to WLAN (Event W1)

8.2.5.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W1 reporting }

ensure that {

when { UE  detects entering condition for Event W1 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {

when { UE  receives RRCConnectionReconfiguration including wlanToAddList }

then { UE adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

8.2.5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.11, 5.6.14.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.11]

The UE shall:

1>	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

…

Inequality W1-1 (Entering condition)

…

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

[TS 36.331, clause 5.6.14.2]

Upon reception of LWA configuration, the UE shall:

…

1>	else:

…

2>	if the received lwa-Config includes lwa-MobilityConfig:

…

3>	if the received lwa-MobilityConfig includes wlanToAddList:

4>	for each WLAN-Identifiers included in wlanToAddList:

5>	add WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwa-MobilityConfig includes associationTimer:

4>	start or restart timer T351 with the timer value set to the associationTimer;

3>	if the received lwa-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.4.3	Test description

8.2.5.4.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.4.3.2	Test procedure sequence

Table 8.2.5.4.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T0" and "T1" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.4.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -60 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values for entering Event W1 |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -50 |  |

Table 8.2.5.4.3.2-2: Main behaviour

| Step | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T0" in table 8.2.5.4.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.4.3.2-1. | - | - | - | - |
| 5 | Check: UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry? | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.4.3.3	Specific message contents

Table 8.2.5.4.3.3-1: RRCConnectionReconfiguration (Step 2 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.4.3.3-2 |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-2: MeasConfig (Step 2 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.4.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.4.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-3: MeasObjectWLAN-GENERIC (Step 2 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-4: ReportConfigInterRAT-W1-WLAN (Step 5 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.4.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-5: RRCConnectionReconfiguration (Step 6 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.4.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-6: LWA-Configuration-GENERIC (Step 6 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.5	LWIP / WLAN Association Success / EUTRA RRC_Connected to WLAN (Event W1)

8.2.5.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W1 reporting }

ensure that {
  when { UE  detects entering condition for Event W1 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToAddList }

then { UE adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

8.2.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.11, 5.6.17.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.11]

The UE shall:

1>	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

…

Inequality W1-1 (Entering condition)

…

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

[TS 36.331, clause 5.6.17.2]

The UE shall:

…

1>	else:

2>	if lwip-MobilityConfig is included:

…

3>	if the received lwip-MobilityConfig includes wlan-ToAddList:

4>	for each WLAN-Identifiers included in wlan-ToAddList:

5>	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes associationTimer:

4>	start timer T351 with the timer value set according to the value of associationTimer;

3>	if the received lwip-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

2>	if tunnelConfigLWIP is included:

3>	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP [32];

3>	if lwip-Counter is included:

4>	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in TS 33.401 [32];

4>	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

2>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.5.3	Test description

8.2.5.5.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.5.3.2	Test procedure sequence

Table 8.2.5.5.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T0" and "T1" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.5.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -60 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values for entering Event W1 |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -50 |  |

Table 8.2.5.5.3.2-2: Main behaviour

| Step | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row “T0” in table 8.2.5.5.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the r“w ”T1" in table 8.2.5.5.3.2-1. | - | - | - | - |
| 5 | Check: UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.5.3.3	Specific message contents

Table 8.2.5.5.3.3-1: RRCConnectionReconfiguration (Step 2 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.5.3.3-2 |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-2: MeasConfig (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.5.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.5.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-3: MeasObjectWLAN-GENERIC (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-4: ReportConfigInterRAT-W1-WLAN (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.5.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-5: RRCConnectionReconfiguration (Step 6 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 | LWIP |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.5.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-6: LWIP-Configuration-GENERIC (Step 6 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell27 |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.6	LWIP / WLAN Release / WLAN Association / EUTRA RRC_Connected to WLAN (Event W2)

8.2.5.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W2 reporting }

ensure that {
  when { UE  detects entering condition for Event W2 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToReleaseList and wlanToAddList }

then { UE releases the WLAN association and adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete  }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W2 reporting }

ensure that {
  when { UE  detects leaving condition for Event W2 is met }

then { UE stops sending MeasurementReport message }

}

8.2.5.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3,

5.5.4.12, 5.6.17.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.12]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both conditions W2-1 and W2-2 as specified below are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition W2-3 or condition W2-4, i.e. at least one of the two, as specified below is fulfilled;

Inequality W2-1 (Entering condition 1)

Inequality W2-2 (Entering condition 2)

Inequality W2-3 (Leaving condition 1)

Inequality W2-4 (Leaving condition 2)

[TS 36.331, clause 5.6.17.2]

The UE shall:

1>	if the received lwip-Configuration is set to release:

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	else:

2>	if lwip-MobilityConfig is included:

3>	if the received lwip-MobilityConfig includes wlan-ToReleaseList:

4>	for each WLAN-Identifiers included in wlan-ToReleaseList:

5>	remove the WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes wlan-ToAddList:

4>	for each WLAN-Identifiers included in wlan-ToAddList:

5>	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes associationTimer:

4>	start timer T351 with the timer value set according to the value of associationTimer;

3>	if the received lwip-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

2>	if tunnelConfigLWIP is included:

3>	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP [32];

2>	if lwip-Counter is included:

3>	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in TS 33.401 [32];

3>	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change or after a lwa-WT-Counter update (if success report is requested by the eNB) or its connection or connection attempts to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.6.3	Test description

8.2.5.6.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1, WLAN Cell 27 and WLAN Cell 27a

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.6.3.2	 Test procedure sequence

Table 8.2.5.6.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 27 and Cell 27a at various time instants of the test execution. Rows marked "T1" and "T2" and “T3” and "T4" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.6.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Cell 27a | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -60 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | The power level values are such that conditions for entering Event W1 is met |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -50 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | The power level values are such that conditions for entering Event W2 is met |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -80 | -50 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | - | The power level values are such that conditions for leaving Event W2 is met |
|  | RSRQ | dB | -4.15 | - | - |  |
|  | BeaconRSSI |  | - | -60 | -50 |  |

Table 8.2.5.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.6.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.6.3.2-1. | - | - | - | - |
| 5 | UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W2) | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell parameters according to the row "T3" in table 8.2.5.6.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report the Event W2 | --> | MeasurementReport | 1 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToReleaseList and Cell 27a in wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmits RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 15 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry? | --> | WLANConnectionStatusReport | 3 | P |
| 16 | The SS changes Cell parameters according to the row "T4" in table 8.2.5.6.3.2-1. | - | - | - | - |
| 17 | Wait and ignore MeasurementReport messages for 15 s to allow change of power levels for Cell 27 and Cell 27a and UE measurement. | - | - | - | - |
| 18 | Check: Does the UE attempt to transmit an uplink message for the next 10s? | - | - | 4 | F |

8.2.5.6.3.3	Specific message contents

Table 8.2.5.6.3.3-1: RRCConnectionReconfiguration (step 2 and 9 Table 8.2.5.6.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.2.5.6.3-2: MeasConfig (step 2, Table 8.2.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN-GENERIC(Freq, SSID) | See Table 8.2.5.6.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.6.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-3: MeasObjectWLAN-GENERIC (step 2, Table 8.2.5.6.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell 27 |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-4: ReportConfigInterRAT-W1 (step 2, Table 8.2.5.6.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.1.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-5: RRCConnectionReconfiguration (step 6 Table 8.2.5.6.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.6.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-6: LWIP-Configuration (step 6 Table 8.2.5.6.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell27 |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { | 1 entry |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3-7: MeasConfig (step 9, Table 8.2.5.6.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN-W2-GENERIC(Freq, SSID1, SSID2) | See Table 8.2.5.6.3.3-8. SSID1 is for Cell 27. SSID2 is for Cell 27a |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W2-WLAN(-75, -55) | Note: Set as per T3 setting for WLAN RSSI in Table 8.2.5.6.3.2-1. See Table 8.2.5.6.3.3-9. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W2 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-8: MeasObjectWLAN-W2-GENERIC (step 9, Table 8.2.5.6.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 and Cell 27a | Band and Carrier information for Cell 27 & Cell 27a |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 2 entries |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18 | Cell 27 |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 of [18 | Cell 27a |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-9: ReportConfigInterRAT-W2-WLAN (step 9, Table 8.2.5.6.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW2-r13 SEQUENCE { | 2 entries |  |  |
| w2-Threshold1-r13 | 26 | Thresh1DB+101=26 where Thresh1DB = -75dBm. Note: Set as per T3 setting for Cell 27 WLAN RSSI plus 5db in Table 8.2.5.6.3.2-1. |  |
| w2-Threshold2-r13 | 46 | Thresh2DB+101=46 where Thresh2DB = -55dBm. Note: Set as per T3 setting for Cell 27a WLAN RSSI minus 5db in Table 8.2.5.6.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-10: RRCConnectionReconfiguration (step 13 Table 8.2.5.6.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwip-configuration-r13 | LWIP-Configuration-W2-GENERIC(SSID1, SSID2) Set as per Table 8.2.5.6.3.3-11 | SSID1 set as per Table 4.4.8-1 of [18]. SSID2 set as per Table 4.4.8-2 of [18]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.6.3.3-11: LWIP-Configuration (step 13 Table 8.2.5.6.3.2-1)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3--33 with condition ADD, RELEASE, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| LWIP-Config-r13 SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell 27 |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-2 of [18] | Cell 27a |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.7	LWIP / WLAN Release Success / EUTRA RRC_Connected from WLAN (Event W3)

8.2.5.7.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W3 reporting }

ensure that {
  when { UE  detects entering condition for Event W3 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToReleaseList that includes WLAN Identifiers that are part of the current WLAN mobility set }

then { UE releases the WLAN association and sends RRCConnectionReconfigurationComplete  }

}

}

8.2.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.13 and 5.6.17.2

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.13]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition W3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition W3-2, as specified below, is fulfilled;

Inequality W3-1 (Entering condition)

Inequality W3-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of WLAN(s) which matches all WLAN identifiers of at least one entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w3-Threshold as defined within reportConfigInterRAT for this event).

Ms is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 36.331, clause 5.6.17.2]

The UE shall:

1>	if the received lwip-Configuration is set to release:

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	else:

2>	if lwip-MobilityConfig is included:

3>	if the received lwip-MobilityConfig includes wlan-ToReleaseList:

4>	for each WLAN-Identifiers included in wlan-ToReleaseList:

5>	remove the WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes wlan-ToAddList:

4>	for each WLAN-Identifiers included in wlan-ToAddList:

5>	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes associationTimer:

4>	start timer T351 with the timer value set according to the value of associationTimer;

3>	if the received lwip-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

2>	if tunnelConfigLWIP is included:

3>	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP [32];

2>	if lwip-Counter is included:

3>	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in TS 33.401 [32];

3>	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

8.2.5.7.3	Test description

8.2.5.7.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and  WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.7.3.2	 Test procedure sequence

Table 8.2.5.7.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T1", "T2" and “T3” are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.7.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -60 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W1 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -50 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that condition for entering Event W3 is met |
|  | RSRQ | dB | -4.15 | - |  |
|  | BeaconRSSI |  | - | -80 |  |

Table 8.2.5.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.7.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T2" in table 8.2.5.7.3.2-1. | - | - | - | - |
| 5 | UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | - | - |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 8 | The UE transmits a WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W3) | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 110 | The SS changes Cell parameters according to the row "T3" in table 8.2.5.7.3.2-1. | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report the Event W3 | --> | MeasurementReport | 1 | P |
| 13 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToReleaseList | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmits RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |

8.2.5.7.3.3	Specific message contents

Table 8.2.5.7.3.3-1: RRCConnectionReconfiguration (step 2 and 8 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.7.3.3-2 for Step 2. Set as per Table 8.2.5.7.3.3-7 for Step 9. |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-2: MeasConfig (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | -1 | IdMeasObject |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.7.3.3-3 Cell 27 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-3: MeasObjectWLAN-GENERIC (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 (Add Table in test case) | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| ssid-r12[2] | Not Present |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-4: ReportConfigInterRAT-W1-WLAN (step 2, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T2 setting for WLAN RSSI minus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-5: RRCConnectionReconfiguration (step 6 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 | LWIP |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.7.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-6: LWIP-Configuration-GENERIC (step 6 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-7: MeasConfig (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 | IdMeasObject-f20 |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.7.3.3-8 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W3 |  |
| reportConfig[1] | ReportConfigInterRAT-W3-WLAN(-75) | See Table 8.2.5.7.3.3-9 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 | IdMeasObject-f20 |  |
| reportConfigId[1] | 1 | IdReportConfig-W3 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-8: MeasObjectWLAN-GENERIC (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | sValue/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-9: ReportConfigInterRAT-W3-WLAN (step 9, Table 8.2.5.7.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW3-r13 SEQUENCE { | 1 entry |  |  |
| W3-Threshold-r13 | 26 | ThreshDB+101=26 where ThreshDB = -75dBm. Note: Set as per T3 setting for WLAN RSSI plus 5db in Table 8.2.5.7.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-10: RRCConnectionReconfiguration (step 13 Table 8.2.5.7.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 | LWIP |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.7.3.3-11 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.7.3.3-11: LWIP-Configuration-GENERIC (step 13 Table 8.2.5.7.3.2-1)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition RELEASE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| LWIP-Config-r13 SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 | Not Present |  |  |
| associationTimer-r13 | Not Present |  |  |
| successReportRequested-r13 | Not Present |  |  |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity ] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.8	LWA / T351 Expiry

8.2.5.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W1 reporting }

ensure that {
  when { UE  detects entering condition for Event W1 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToAddList }

then { UE adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after T351 expiry not being able to successfully associate with any WLAN identifier in the new WLAN mobility set, the UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to failureTimeout }

}

8.2.5.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.11, 5.6.14.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.11]

The UE shall:

1>	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

…

Inequality W1-1 (Entering condition)

…

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

[TS 36.331, clause 5.6.14.2]

Upon reception of LWA configuration, the UE shall:

…

1>	else:

…

2>	if the received lwa-Config includes lwa-MobilityConfig:

…

3>	if the received lwa-MobilityConfig includes wlanToAddList:

4>	for each WLAN-Identifiers included in wlanToAddList:

5>	add WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwa-MobilityConfig includes associationTimer:

4>	start or restart timer T351 with the timer value set to the associationTimer;

3>	if the received lwa-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.15.3]

Upon T351 expiry, the UE shall:

1>	set the status in VarWLAN-Status to failureTimeout;

1>	perform WLAN connection status reporting procedure in 5.6.15.2;

1>	stop WLAN status monitoring and WLAN connection attempts;

8.2.5.8.3	Test description

8.2.5.8.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.8.3.2	Test procedure sequence

Table 8.2.5.8.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T0" and "T1" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.8.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -60 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values for entering Event W1 |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -50 |  |

Table 8.2.5.8.3.2-2: Main behaviour

| Step | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T0" in table 8.2.5.8.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.8.3.2-1. | - | - | - | - |
| 5 | Check: UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message including an invalid WLAN ssid in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to failureTimeout after T351 expiry? | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.8.3.3	Specific message contents

Table 8.2.5.8.3.3-1: RRCConnectionReconfiguration (Step 2 Table 8.2.5.8.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.8.3.3-2 |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.8.3.3-2: MeasConfig (Step 2 Table 8.2.5.8.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.8.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.8.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.8.3.3-3: MeasObjectWLAN-GENERIC (Step 2 Table 8.2.5.8.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.8.3.3-4: ReportConfigInterRAT-W1-WLAN (Step 5 Table 8.2.5.8.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 Set as per T1 setting for WLAN RSSI in Table 8.2.5.8.3.2-1. E.g -60 dBm is equivalent to 41 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.8.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.8.3.3-5: RRCConnectionReconfiguration (Step 6 Table 8.2.5.8.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.8.3.3-6 | Invalid SSID set i.e not as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.8.3.3-6: LWA-Configuration-GENERIC (Step 6 Table 8.2.5.8.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set an invalid value i.e not same as in Table 4.4.8-1 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |