| 3GPP TS 38.133 V17.3.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

4	SA: RRC_IDLE state mobility	55

4.1	Cell Selection	55

4.2	Cell Re-selection	55

4.2.1	Introduction	55

4.2.2	Requirements	56

4.2.2.1	UE measurement capability	56

4.2.2.2	Measurement and evaluation of serving cell	56

4.2.2.3	Measurements of intra-frequency NR cells	57

4.2.2.4	Measurements of inter-frequency NR cells	58

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	60

4.2.2.6	Maximum interruption in paging reception	62

4.2.2.7	General requirements	62

4.2.2.8	Minimum requirement at transitions	63

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	63

4.2.2.9.1	Introduction	63

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	63

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	64

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	65

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	65

4.2.2.10.1	Introduction	65

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	65

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	66

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	67

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	68

4.2.2.11.1	Introduction	68

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	68

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	69

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	70

4.2A	Cell Re-selection when subject to CCA	70

4.2A.1	Introduction	70

4.2A.2	Requirements	71

4.2A.2.1	UE measurement capability	71

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	71

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	72

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	73

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	75

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	75

4.2A.2.7	General requirements	75

4.3	Minimization of Drive Tests (MDT)	76

4.3.1	Introduction	76

4.3.2	Measurement Requirements	76

4.3.3	Requirements for Relative Time Stamp Accuracy	76

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	77

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	77

4.4	Idle Mode CA/DC Measurements	77

4.4.1	Introduction	77

4.4.2	Measurement Requirements	77

4.4.2.1	Detected cell requirement during state transition and Idle mode	77

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	78

4.4.2.3	Measurements on serving cell	79

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	79

5	SA: RRC_INACTIVE state mobility	81

5.1	Cell Re-selection	81

5.1.1	Introduction	81

5.1.2	Requirements	81

5.1.2.1	UE measurement capability	81

5.1.2.2	Measurement and evaluation of serving cell	81

5.1.2.3	Measurements of intra-frequency NR cells	81

5.1.2.4	Measurements of inter-frequency NR cells	81

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	81

5.1.2.6	Maximum interruption in paging reception	81

5.1.2.7	General requirements	81

5.1A	Cell Re-selection with CCA	81

5.1A.1	Introduction	81

5.1A.2	Requirements	82

5.1A.2.1	UE measurement capability	82

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	82

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	82

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	82

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	82

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	82

5.1A.2.7	General requirements	82

5.2	Void	82

5.3	Minimization of Drive Tests (MDT)	82

5.3.1	Introduction	82

5.3.2	Measurement Requirements	83

5.3.3	Requirements for Relative Time Stamp Accuracy	83

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	83

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	83

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	83

5.4	Idle Mode CA/DC Measurements	83

5.4.1	Introduction	83

5.4.2	Measurement Requirements	83

5.4.2.1	Detected cell requirement during state transition and Idle mode	84

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	84

5.4.2.3	Measurements on serving cell	84

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	84

6	RRC_CONNECTED state mobility	84

6.1	Handover	84

6.1.1	NR Handover	84

6.1.1.1	Introduction	84

6.1.1.2	NR FR1 - NR FR1 Handover	84

6.1.1.2.1	Handover delay	84

6.1.1.2.2	Interruption time	84

6.1.1.3	NR FR2- NR FR1 Handover	85

6.1.1.3.1	Handover delay	85

6.1.1.3.2	Interruption time	85

6.1.1.4	NR FR2- NR FR2 Handover	86

6.1.1.4.1	Handover delay	86

6.1.1.4.2	Interruption time	86

6.1.1.5	NR FR1- NR FR2 Handover	87

6.1.1.5.1	Handover delay	87

6.1.1.5.2	Interruption time	87

6.1.2	NR Handover to other RATs	88

6.1.2.1	NR – E-UTRAN Handover	88

6.1.2.1.1	Introduction	88

6.1.2.1.2	Handover delay	88

6.1.2.1.3	Interruption time	88

6.1.2.2	NR – UTRAN Handover	89

6.1.2.2.1	Introduction	89

6.1.2.2.2	Handover delay	89

6.1.2.2.3	Interruption time	89

6.1.3	NR DAPS Handover	90

6.1.3.1	Introduction	90

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	90

6.1.3.2.1	DAPS handover delay	90

6.1.3.2.2	Interruption time	91

6.1.3.3	NR FR2- NR FR1 DAPS Handover	93

6.1.3.3.1	DAPS handover delay	93

6.1.3.3.2	Interruption time	93

6.1.3.4	NR FR1- NR FR2 DAPS Handover	94

6.1.3.4.1	DAPS handover delay	94

6.1.3.4.2	Interruption time	94

6.1.4	NR Conditional Handover	95

6.1.4.1	Introduction	95

6.1.4.2	NR FR1 – NR FR1 conditional handover	95

6.1.4.3	NR FR2 – NR FR1 conditional handover	96

6.1.4.4	NR FR2 – NR FR2 conditional handover	97

6.1.4.4.1	Handover delay	97

6.1.4.4.2	Measurement time	97

6.1.4.4.3	Preparation time	97

6.1.4.4.4	Interruption time	97

6.1.4.5	NR FR1 – NR FR2 conditional handover	98

6.1A	Void	98

6.1A.1	Void	98

6.1A.1.1	Void	98

6.1A.1.2	Void	98

6.1A.1.2.1	Void	98

6.1A.1.2.2	Void	98

6.1B	Handover to target cell using CCA	98

6.1B.1	NR Handover	98

6.1B.1.1	Introduction	98

6.1B.1.2	NR FR1 - NR FR1 Handover	99

6.1B.1.2.1	Handover delay	99

6.1B.1.2.2	Interruption time	99

6.2	RRC Connection Mobility Control	100

6.2.1	SA: RRC Re-establishment	100

6.2.1.1	Introduction	100

6.2.1.2	Requirements	100

6.2.1.2.1	UE Re-establishment delay requirement	100

6.2.1A	RRC Re-establishment with CCA	101

6.2.1A.1	Introduction	101

6.2.1A.2	Requirements	102

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	102

6.2.2	Random access	103

6.2.2.1	Introduction	103

6.2.2.2	Requirements for 4-step RA type	104

6.2.2.2.1	Contention based random access	104

6.2.2.2.2	Non-Contention based random access	105

6.2.2.2.3	UE behaviour when configured with supplementary UL	106

6.2.2.3	Requirements for 2-step RA type	106

6.2.2.3.1	Contention based random access	106

6.2.2.3.2	Non-Contention based random access	107

6.2.2.3.3	UE behaviour when configured with supplementary UL	108

6.2.2A	Random access when CCA is used on target frequency	108

6.2.2A.1	Introduction	108

6.2.2A.2	Requirements for 4-step RA type	108

6.2.2A.2.1	Contention based random access	108

6.2.2A.2.2	Non-Contention based random access	109

6.2.2A.3	Requirements for 2-step RA type	110

6.2.2A.3.1	Contention based random access	111

6.2.2A.3.2	Non-Contention based random access	112

6.2.3	SA: RRC Connection Release with Redirection	113

6.2.3.1	Introduction	113

6.2.3.2	Requirements	113

6.2.3.2.1	RRC connection release with redirection to NR	113

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	114

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	114

7	Timing	115

7.1	UE transmit timing	115

7.1.1	Introduction	115

7.1.2	Requirements	116

7.1.2.1	Gradual timing adjustment	117

7.1.2.2	Void	117

7.2	UE timer accuracy	117

7.2.1	Introduction	117

7.2.2	Requirements	118

7.3	Timing advance	118

7.3.1	Introduction	118

7.3.2	Requirements	118

7.3.2.1	Timing Advance adjustment delay	118

7.3.2.2	Timing Advance adjustment accuracy	118

7.4	Cell phase synchronization accuracy	118

7.4.1	Definition	118

7.4.2	Minimum requirements	118

7.5	Maximum Transmission Timing Difference	119

7.5.1	Introduction	119

7.5.2	Minimum Requirements for inter-band EN-DC	119

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	119

7.5.3	Minimum Requirements for intra-band EN-DC	120

7.5.4	Minimum Requirements for NR Carrier Aggregation	120

7.5.5	Minimum Requirements for inter-band NE-DC	120

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	121

7.5.6	Minimum Requirements for inter-band NR DC	121

7.6	Maximum Receive Timing Difference	122

7.6.1	Introduction	122

7.6.2	Minimum Requirements for inter-band EN-DC	122

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	122

7.6.3	Minimum Requirements for intra-band EN-DC	123

7.6.4	Minimum Requirements for NR Carrier Aggregation	123

7.6.5	Minimum Requirements for inter-band NE-DC	124

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	124

7.6.6	Minimum Requirements for inter-band NR DC	125

7.7	deriveSSB-IndexFromCell tolerance	125

7.7.1	Minimum requirements	125

7.8	Void	126

8	Signalling characteristics	126

8.1	Radio Link Monitoring	126

8.1.1	Introduction	126

8.1.2	Requirements for SSB based radio link monitoring	127

8.1.2.1	Introduction	127

8.1.2.2	Minimum requirement	128

8.1.2.3	Measurement restrictions for SSB based RLM	129

8.1.3	Requirements for CSI-RS based radio link monitoring	130

8.1.3.1	Introduction	130

8.1.3.2	Minimum requirement	131

8.1.3.3	Measurement restrictions for CSI-RS based RLM	133

8.1.4	Minimum requirement at transitions	133

8.1.5	Minimum requirement for UE turning off the transmitter	134

8.1.6	Minimum requirement for L1 indication	134

8.1.7	Scheduling availability of UE during radio link monitoring	134

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	134

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	134

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	135

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	135

8.1A	Radio Link Monitoring with CCA on Target Frequency	135

8.1A.1	Introduction	135

8.1A.2	Requirements for SSB Based Radio Link Monitoring	136

8.1A.2.1	Introduction	136

8.1A.2.2	Minimum Requirement	137

8.1A.3	Minimum requirement at transitions	138

8.1A.4	Minimum requirement for UE turning off the transmitter	138

8.1A.5	Minimum requirement for L1 indication	139

8.1A.6	Scheduling availability of UE during radio link monitoring	139

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	139

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	139

8.2	Interruption	139

8.2.1	EN-DC Interruption	139

8.2.1.1	Introduction	139

8.2.1.2	Requirements	140

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	140

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	140

8.2.1.2.3	Interruptions at SCell addition/release	141

8.2.1.2.4	Interruptions at SCell activation/deactivation	142

8.2.1.2.5	Interruptions during measurements on SCC	143

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	144

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	145

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	146

8.2.1.2.9	Interruptions at SCell hibernation	146

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	146

8.2.1.2.11	Interruptions due to UE-specific CBW change	147

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	147

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	148

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	149

8.2.1.2.15	Interruptions due to SCell dormancy	150

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	150

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	151

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	151

8.2.2.1	Introduction	151

8.2.2.2	Requirements	152

8.2.2.2.1	Interruptions at SCell addition/release	152

8.2.2.2.2	Interruptions at SCell activation/deactivation	153

8.2.2.2.3	Interruptions during measurements on deactivated SCC	154

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	155

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	155

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	156

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	157

8.2.2.2.8	Interruptions due to UE-specific CBW change	157

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	158

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	159

8.2.2.2.11	Interruptions at direct SCell activation	160

8.2.2.2.12	Interruptions due to SCell dormancy	160

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	160

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	161

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	161

8.2.3	NE-DC Interruptions	162

8.2.3.1	Introduction	162

8.2.3.2	Requirements	162

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	162

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	163

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	163

8.2.3.2.4	Interruptions at SCell activation/deactivation	164

8.2.3.2.5	Interruptions during measurements on SCC	165

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	166

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	167

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	167

8.2.3.2.9	Interruptions at SCell hibernation	167

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	168

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	168

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	169

8.2.3.2.13	Interruptions due to SCell dormancy	170

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	171

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	171

8.2.4	NR-DC: Interruptions	172

8.2.4.1	Introduction	172

8.2.4.2	Requirements	172

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	172

8.2.4.2.2	Interruptions at SCell activation/deactivation	173

8.2.4.2.3	Interruptions during measurements on SCC	174

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	174

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	175

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	175

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	175

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	175

8.2.4.2.9	Interruptions at NR SRS carrier based switching	176

8.2.4.2.10	Interruptions at direct SCell activation	177

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	177

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	178

8.2.4.2.13	 Interruptions due to SCell dormancy	179

8.2.4.2A	Void	179

8.2.4.2A.1	Void	179

8.2.4.2A.2	Void	179

8.2.4.2A.3	Void	179

8.3	SCell Activation and Deactivation Delay	179

8.3.1	Introduction	179

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	179

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	184

8.3.4	Direct SCell Activation at SCell addition	184

8.3.5	Direct SCell Activation at Handover	185

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	187

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	191

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	191

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	192

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	194

8.3A.1	Introduction	194

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	194

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	197

8.4	UE UL carrier RRC reconfiguration delay	197

8.4.1	Introduction	197

8.4.2	UE UL carrier configuration delay requirement	197

8.4.3	UE UL carrier deconfiguration delay requirement	197

8.5	Link Recovery Procedures	197

8.5.1	Introduction	197

8.5.2	Requirements for SSB based beam failure detection	198

8.5.2.1	Introduction	198

8.5.2.2	Minimum requirement	199

8.5.2.3	Measurement restriction for SSB based beam failure detection	200

8.5.3	Requirements for CSI-RS based beam failure detection	201

8.5.3.1	Introduction	201

8.5.3.2	Minimum requirement	201

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	203

8.5.4	Minimum requirement for L1 indication	204

8.5.5	Requirements for SSB based candidate beam detection	204

8.5.5.1	Introduction	204

8.5.5.2	Minimum requirement	205

8.5.5.3	Measurement restriction for SSB based candidate beam detection	207

8.5.6	Requirements for CSI-RS based candidate beam detection	207

8.5.6.1	Introduction	207

8.5.6.2	Minimum requirement	207

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	209

8.5.7	Scheduling availability of UE during beam failure detection	210

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	210

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	210

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	210

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	211

8.5.8	Scheduling availability of UE during candidate beam detection	211

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	211

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	211

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	212

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	212

8.5.9	Requirements for Beam Failure Recovery in SCell	212

8.5.9.1	Introduction	212

8.5.9.2	Requirement	213

8.5.10	Minimum requirement at transitions for beam failure detection	213

8.5A	Link Recovery Procedures when CCA is used on target frequency	213

8.5A.1	Introduction	213

8.5A.2	Requirements for SSB based beam failure detection	214

8.5A.2.1	Introduction	214

8.5A.2.2	Minimum requirement	214

8.5A.2.3	Measurement restriction for SSB based beam failure detection	215

8.5A.4	Minimum requirement for L1 indication	215

8.5A.5	Requirements for SSB based candidate beam detection	215

8.5A.5.1	Introduction	215

8.5A.5.2	Minimum requirement	216

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	216

8.5A.7	Scheduling availability of UE during beam failure detection	216

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	217

8.5A.8	Scheduling availability of UE during candidate beam detection	217

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	217

8.6	Active BWP switch delay	217

8.6.1	Introduction	217

8.6.2	DCI and timer based BWP switch delay on a single CC	217

8.6.2A	DCI based BWP switch delay on multiple CCs	219

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	219

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	220

8.6.2B	Timer based BWP switch delay on multiple CCs	220

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	220

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	221

8.6.3	RRC based BWP switch delay on a single CC	221

8.6.3A	RRC based BWP switch delay on multiple CCs	222

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	222

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.4	BWP switch delay on Consistent UL CCA recovery	223

8.7	Void	223

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	223

8.8.1	Introduction	223

8.8.2	E-UTRAN PSCell Addition Delay Requirement	224

8.8.3	E-UTRAN PSCell Release Delay Requirement	224

8.9	NR-DC: PSCell Addition and Release Delay	224

8.9.1	Introduction	224

8.9.2	PSCell Addition Delay Requirement	224

8.9.3	PSCell Release Delay Requirement	225

8.10	Active TCI state switching delay	225

8.10.4	DCI based TCI state switch delay	227

8.10.5	RRC based TCI state switch delay	227

8.10.6	Active TCI state list update delay	227

8.10A	Active TCI state switching delay with CCA	228

8.10A.1	Introduction	228

8.10A.2	Known conditions for TCI state	228

8.10A.3	MAC-CE based TCI state switch delay	228

8.10A.4	DCI based TCI state switch delay	229

8.10A.5	RRC based TCI state switch delay	229

8.10A.6	Active TCI state list update delay	230

8.11	PSCell Change	230

8.11A	void	230

8.11B	Conditional PSCell Change	230

8.11B.1	Introduction	230

8.11B.2	Conditoinal PSCell Change delay	230

8.11B.2.1	Measurement time	231

8.12	Uplink spatial relation switch delay	231

8.12.1	Introduction	231

8.12.2	Known conditions for spatial relation when associated with DL-RS	231

8.12.3	MAC-CE based spatial relation switch delay	232

8.12.4	DCI based spatial relation switch delay	233

8.12.5	RRC based spatial relation switch delay	233

8.13	UE-specific CBW change	233

8.13.1	Introduction	233

8.13.2	UE-specific CBW change delay	233

8.14	Pathloss reference signal switching delay	233

8.14.1	Introduction	233

8.14.2	Known conditions for pathloss reference signal	234

8.14.3	MAC-CE based pathloss reference signal switch delay	234

9	Measurement Procedure	234

9.1	General measurement requirement	234

9.1.1	Introduction	234

9.1.2	Measurement gap	235

9.1.2.1	EN-DC: Measurement Gap Sharing	246

9.1.2.1a	SA: Measurement Gap Sharing	246

9.1.2.1b	NE-DC: Measurement Gap Sharing	247

9.1.2.1c	NR-DC: Measurement Gap Sharing	248

9.1.3	UE Measurement capability	249

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	249

9.1.3.1a	SA: Monitoring of multiple layers using gaps	249

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	250

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	250

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	251

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	252

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	252

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	253

9.1A.3.2	Void	254

9.1.3A	UE Measurement capability under operation mode with CCA	254

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	254

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	254

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	254

9.1A.3.2a	Void	255

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	255

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	255

9.1.4.1	Introduction	255

9.1.4.2	Requirements	255

9.1.5	Carrier-specific scaling factor	259

9.1.5.1	Monitoring of multiple layers outside gaps	259

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	261

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	262

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	263

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	264

9.1.5.2	Monitoring of multiple layers within gaps	265

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	266

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	268

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	269

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	271

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	273

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	273

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	273

9.1.6	Minimum requirement at transitions	273

9.2	NR intra-frequency measurements	274

9.2.1	Introduction	274

9.2.2	Requirements applicability	274

9.2.3	Number of cells and number of SSB	275

9.2.3.1	Requirements for FR1	275

9.2.3.2	Requirements for FR2	275

9.2.4	Measurement Reporting Requirements	275

9.2.4.1	Periodic Reporting	275

9.2.4.2	Event-triggered Periodic Reporting	275

9.2.4.3	Event Triggered Reporting	275

9.2.5	Intrafrequency measurements without measurement gaps	276

9.2.5.1	Intrafrequency cell identification	276

9.2.5.2	Measurement period	279

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	280

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	281

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	281

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	281

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	282

9.2.5.4	SFTD Measurements between PCell and PSCell	282

9.2.5.4.1	Introduction	282

9.2.5.4.2	SFTD Measurement delay	282

9.2.5.4.3	SFTD Measurement Reporting Delay	283

9.2.6	Intra-frequency measurements with measurement gaps	283

9.2.6.1	Void	283

9.2.6.2	Intra-frequency cell identification	283

9.2.6.3	Intrafrequency Measurement Period	285

9.2A	NR intra-frequency measurements with CCA	286

9.2A.1	Introduction	286

9.2A.2	Requirements applicability	287

9.2A.3	Number of cells and number of SSB	287

9.2A.4	Measurement Reporting Requirements	287

9.2A.5	Intra-frequency measurements without measurement gaps	288

9.2A.5.2	Measurement period	290

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	291

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	291

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	292

9.2A.6	Intra-frequency measurements with measurement gaps	292

9.2A.6.1	Intra-frequency cell identification	292

9.2A.6.2	Intra-frequency Measurement Period	293

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	294

9.2A.7.1	Intra-frequency RSSI measurements	294

9.2A.7.2	Intra-frequency Channel occupancy measurements	295

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	296

9.3	NR inter-frequency measurements	296

9.3.1	Introduction	296

9.3.2	Requirements applicability	297

9.3.2.1	Void	297

9.3.2.2	Void	297

9.3.3	Number of cells and number of SSB	297

9.3.3.1	Requirements for FR1	297

9.3.3.2	Requirements for FR2	297

9.3.4	Inter-frequency measurement with measurement gaps	297

9.3.4.1	Void	299

9.3.4.2	Void	299

9.3.5	Inter-frequency measurements	299

9.3.5.1	Void	299

9.3.5.2	Void	299

9.3.5.3	Void	299

9.3.6	Inter-frequency measurements reporting requirements	299

9.3.6.1	Periodic Reporting	299

9.3.6.2	Event-triggered Periodic Reporting	300

9.3.6.3	Event-triggered Reporting	300

9.3.7	Void	300

9.3.8	Inter-frequency SFTD measurement requirements	300

9.3.8.1	Introduction	300

9.3.8.2	SFTD Measurement delay	300

9.3.8.3	SFTD Measurement reporting delay	301

9.3.9	Inter frequency measurements without measurement gaps	302

9.3.9.1	Inter frequency Cell identification	302

9.3.9.2	Measurement period	303

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	304

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	304

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	305

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	305

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	305

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	306

9.3A.1	Introduction	306

9.3A.2	Requirements applicability	306

9.3A.3	Number of cells and number of SSB	306

9.3A.3.1	Requirements	306

9.3A.4	Inter-frequency cell identification	307

9.3A.5	Inter-frequency measurements	308

9.3A.6	NR Inter-frequency measurements reporting requirements	309

9.3A.6.1	Periodic Reporting	309

9.3A.6.2	Event-triggered Periodic Reporting	309

9.3A.6.3	Event-triggered Reporting	309

9.3A.8	Inter-frequency RSSI measurements	309

9.3A.9	Inter-frequency channel occupancy measurements	310

9.4	Inter-RAT measurements	310

9.4.1	Introduction	310

9.4.2	NR − E-UTRAN FDD measurements	312

9.4.2.1	Introduction	312

9.4.2.2	Requirements when no DRX is used	312

9.4.2.3	Requirements when DRX is used	313

9.4.2.4	Measurement reporting requirements	314

9.4.2.4.1	Periodic Reporting	314

9.4.2.4.2	Event-Triggered Periodic Reporting	314

9.4.2.4.3	Event-Triggered Reporting	314

9.4.3	NR − E-UTRAN TDD measurements	314

9.4.3.1	Introduction	314

9.4.3.2	Requirements when no DRX is used	314

9.4.3.3	Requirements when DRX is used	315

9.4.3.4	Measurement reporting requirements	317

9.4.3.4.1	Periodic Reporting	317

9.4.3.4.2	Event-Triggered Periodic Reporting	317

9.4.3.4.3	Event-Triggered Reporting	317

9.4.4	Inter-RAT RSTD measurements	317

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	317

9.4.4.1.1	Introduction	317

9.4.4.1.2	Requirements	318

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	321

9.4.4.2.1	Introduction	321

9.4.4.2.2	Requirements	322

9.4.5	Inter-RAT E-CID measurements	325

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	325

9.4.5.1.1	Introduction	325

9.4.5.1.2	Requirements	325

9.4.5.1.3	Measurement Reporting Delay	325

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	325

9.4.5.2.1	Introduction	325

9.4.5.2.2	Requirements	325

9.4.5.2.3	Measurement Reporting Delay	326

9.4.6	NR − UTRAN FDD measurements	326

9.4.6.1	Introduction	326

9.4.6.2	Requirements when no DRX is used	326

9.4.6.3	Requirements when DRX is used	327

9.4.7	NR – E-UTRAN measurements with autonomous gaps	329

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	329

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.9.4.7.2	CGI reporting delay	329

9.5	L1-RSRP measurements for Reporting	330

9.5.1	Introduction	330

9.5.2	Requirements applicability	330

9.5.3	Measurement Reporting Requirements	330

9.5.3.1	Periodic Reporting	331

9.5.3.2	Semi-Persistent Reporting	331

9.5.3.3	Aperiodic Reporting	331

9.5.4	L1-RSRP measurement requirements	331

9.5.4.1	SSB based L1-RSRP Reporting	331

9.5.4.2	CSI-RS based L1-RSRP Reporting	333

9.5.4A	Void	336

9.5.4A.1	Void	336

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	336

9.5.5.1	Measurement restriction for SSB based L1-RSRP	336

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	337

9.5.6	Scheduling availability of UE during L1-RSRP measurement	337

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	337

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	337

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	338

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	339

9.5A	L1-RSRP measurements for Reporting under CCA	339

9.5A.1	Introduction	339

9.5A.2	Requirements applicability	339

9.5A.3	Measurement Reporting Requirements	339

9.5A.3.1	Periodic Reporting	340

9.5A.3.2	Semi-Persistent Reporting	340

9.5A.3.3	Aperiodic Reporting	340

9.5A.4	L1-RSRP measurement requirements	340

9.5A.4.1	SSB based L1-RSRP Reporting	340

9.5A.5	Measurement restriction for L1-RSRP measurement	341

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	341

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	342

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	342

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	342

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	342

9.6	NE-DC: Measurements	342

9.6.1	Introduction	342

9.6.2	SFTD Measurements	342

9.6.2.1	Introduction	342

9.6.2.2	SFTD Measurement requirements	342

9.7	Cross Link Interference measurements	343

9.7.1	Introduction	343

9.7.2	SRS-RSRP measurements	343

9.7.2.1	Introduction	343

9.7.2.2	Requirements applicability	344

9.7.2.3	Measurement Reporting Requirements	344

9.7.2.3.1	Periodic Reporting	344

9.7.2.3.2	Event-triggered Periodic Reporting	344

9.7.2.3.3	Event Triggered Reporting	344

9.7.2.4	Measurement capability	344

9.7.2.5	SRS-RSRP measurement period	344

9.7.3	CLI-RSSI measurements	345

9.7.3.1	Introduction	345

9.7.3.2	Requirements applicability	345

9.7.3.3	Measurement Reporting Requirements	345

9.7.3.3.1	Periodic Reporting	345

9.7.3.3.2	Event-triggered Periodic Reporting	345

9.7.3.3.3	Event Triggered Reporting	345

9.7.3.4	Measurement capability	346

9.7.3.5	CLI-RSSI measurement period	346

9.7.4	Scheduling availability of UE during CLI measurements	346

9.7.4.1	Scheduling availability of UE performing measurement on FR1	346

9.7.4.2	Scheduling availability of UE performing measurement on FR2	347

9.8	L1-SINR measurements for Reporting	347

9.8.1	Introduction	347

9.8.2	Requirements applicability	348

9.8.3	Measurement Reporting Requirements	348

9.8.3.1	Periodic Reporting	348

9.8.3.2	Semi-Persistent Reporting	349

9.8.4	L1-SINR measurement requirements	349

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	349

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	352

9.8.5	Measurement restriction for L1-SINR measurement	354

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	354

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	355

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	355

9.8.6	Scheduling availability of UE during L1-SINR measurement	356

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	356

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	356

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	357

9.9	 NR measurements for positioning	357

9.9.1	Introduction	357

9.9.2	RSTD measurements	358

9.9.2.1	Introduction	358

9.9.2.2	Requirements Applicability	358

9.9.2.3	Measurement Capability	358

9.9.2.4	Measurement Reporting Requirements	358

9.9.2.4.1	Void	358

9.9.2.4.2	Void	358

9.9.2.4.3	Void	358

9.9.2.5	Measurements Period Requirements	358

9.9.2.6	Void	360

9.9.3	PRS-RSRP measurements	360

9.9.3.1	Introduction	360

9.9.3.2	Requirements applicability	360

9.9.3.3	Measurement Capability	361

9.9.3.4	Measurement Reporting Requirements	361

9.9.3.5	Measurement Period Requirements	361

9.9.4	UE Rx-Tx time difference measurements	363

9.9.4.1 Introduction	363

9.9.4.2 Requirements Applicability	363

9.9.4.3 Measurement Capability	363

9.9.4.4 Measurement Reporting Requirements	363

9.9.4.5 Measurement Period Requirements	363

9.9.5.1	Introduction	365

9.9.5.2	Measurement Requirements	365

9.9.5.2.1	Intra-frequency Measurement Requirements	365

9.9.5.2.2	Inter-frequency Measurement Requirements	366

9.9.5.2.3	Measurement Reporting Delay	366

9.10	CSI-RS based L3 measurements	366

9.10.1	Introduction	366

9.10.2	CSI-RS based intra-frequency measurements	367

9.10.2.1	Introduction	367

9.10.2.2	Requirements applicability	367

9.10.2.3	Number of cells and number of CSI-RS	368

9.10.2.3.1	Requirements for FR1	368

9.10.2.3.2	Requirements for FR2	368

9.10.2.4	Measurement Reporting Requirements	369

9.10.2.4.1	Periodic Reporting	369

9.10.2.4.2	Event-triggered Periodic Reporting	369

9.10.2.4.3	Event Triggered Reporting	369

9.10.2.5	Intra-frequency measurements without measurement gaps	369

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	370

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	371

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	371

9.10.3	CSI-RS based Inter-frequency measurements	371

9.10.3.1	Introduction	371

9.10.3.2	Requirements applicability	371

9.10.3.3	Number of cells and number of CSI-RS resources	372

9.10.3.3.1	Requirements for FR1	372

9.10.3.3.2	Requirements for FR2	372

9.10.3.4	Measurements reporting requirements	372

9.10.3.4.1	Periodic Reporting	372

9.10.3.4.2	Event-triggered Periodic Reporting	373

9.10.3.4.3	Event-triggered Reporting	373

9.10.3.5	Inter frequency measurements with measurement gaps	373

9.11	NR measurements with autonomous gaps	374

9.11.1	Introduction	374

9.11.2	CGI identification of an NR cell with autonomous gaps	374

9.11.3	CGI reporting delay	375

10	Measurement Performance requirements	376

10.1	NR measurements	376

10.1.1	Introduction	376

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	376

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	376

10.1.2.1.1	Absolute SS-RSRP Accuracy	376

10.1.2.1.2	Relative SS-RSRP Accuracy	377

10.1.2.2	Void	378

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	378

10.1.2.3.1	Absolute CSI-RSRP Accuracy	378

10.1.2.3.2	Relative CSI-RSRP Accuracy	379

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	380

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	380

10.1.2B.1.1	Absolute SS-RSRP Accuracy	380

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	381

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	381

10.1.3.1.1	Absolute SS-RSRP Accuracy	381

10.1.3.1.2	Relative SS-RSRP Accuracy	382

10.1.3.2	Void	383

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	383

10.1.3.3.1	Absolute CSI-RSRP Accuracy	383

10.1.3.3.2	Relative CSI-RSRP Accuracy	383

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	384

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	384

10.1.3B.1.1	Absolute SS-RSRP Accuracy	384

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	385

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	385

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	385

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	386

10.1.4.2	Void	387

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	387

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	387

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	388

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	389

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	389

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	389

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	390

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	390

10.1.5.1.1	Absolute SS-RSRP Accuracy	390

10.1.5.1.2	Relative SS-RSRP Accuracy	391

10.1.5.2	Void	392

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	392

10.1.5.3.1	Absolute CSI-RSRP Accuracy	392

10.1.5.3.2	Relative CSI-RSRP Accuracy	392

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	393

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	393

10.1.5B.1.1	Absolute SS-RSRP Accuracy	393

10.1.6 RSRP Measurement Report Mapping	394

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	396

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	396

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	396

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	397

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	397

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	398

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	398

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	398

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	399

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	399

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	399

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	400

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	400

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	401

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	401

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	401

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	402

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	402

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	402

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	403

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	404

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	404

10.1.9.2.2	Relative CSI-RSRQ Accuracy	405

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	406

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	406

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	406

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	407

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	409

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	409

10.1.10.2.2	Relative CSI-RSRQ Accuracy	409

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	410

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	410

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	410

10.1.11	RSRQ report mapping	411

10.1.12	Intra-frequency SINR accuracy requirements for FR1	411

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	412

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	412

10.1.13	Intra-frequency SINR accuracy requirements for FR2	413

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	414

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	414

10.1.14	Inter-frequency SINR accuracy requirements for FR1	415

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	417

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	417

10.1.15	Inter-frequency SINR accuracy requirements for FR2	419

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	421

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	421

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	421

10.1.16	SINR report mapping	422

10.1.17	Power Headroom	423

10.1.18	PCMAX,c,f	424

10.1.19	L1-RSRP accuracy requirements for FR1	424

10.1.20	L1-RSRP accuracy requirements for FR2	428

10.1.21	SFTD accuracy requirements	431

10.1.22	CLI measurement accuracy requirements	435

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	439

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	439

10.1.23	RSTD Measurements	441

10.1.23.1	Introduction	441

10.1.23.2	Measurement Accuracy Requirements	442

10.1.23.3	Report mapping	446

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	446

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	448

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	450

10.1.24	PRS-RSRP Measurements	452

10.1.24.1	Introduction	452

10.1.24.2	Measurement Accuracy Requirements	452

10.1.24.2.1 Absolute PRS RSRP accuracy	452

10.1.24.2.2 Relative PRS RSRP accuracy	454

10.1.24.3	Report mapping	456

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	456

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	457

10.1.25	UE Rx-Tx Time Difference Measurements	459

10.1.25.1	Introduction	459

10.1.25.2	Measurement Accuracy Requirements	460

10.1.25.3	Report mapping	467

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	467

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	469

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	471

10.1.26	FR2 P-MPR report	473

10.1.26.1	Report mapping	473

10.1.27	L1-SINR accuracy requirements for FR1	473

10.1.28	L1-SINR accuracy requirements for FR2	481

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	488

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	488

10.1.29.1.1	Absolute SS-RSRQ Accuracy	488

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	488

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	488

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	488

10.1.30.1.2	Relative Accuracy of SS-RSRQ	489

10.1.31	Intra-frequency SINR accuracy requirements under CCA	490

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	490

10.1.31.1.1	Absolute SS-SINR Accuracy	490

10.1.32	Inter-frequency SINR accuracy requirements under CCA	490

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	490

10.1.32.1.1	Aboslute Accuracy of SS-SINR	490

10.1.32.1.2	Relative Accuracy of SS-SINR	491

10.1.33	L1-RSRP accuracy requirements under CCA	492

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	492

10.1.33.1.1	Absolute Accuracy	492

10.1.33.1.2	Relative Accuracy	492

10.1.34	RSSI measurements under CCA	493

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	493

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	493

10.1.34.3	RSSI measurement report mapping	493

10.1.35	Channel occupancy measurements under CCA	494

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	494

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	494

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	494

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	494

10.1.36.1.1	Absolute SS-RSRP Accuracy	494

10.1.36.1.2	Relative SS-RSRP Accuracy	495

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	496

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	496

10.1.37.1.1	Absolute Accuracy of SS-RSRP	496

10.1.37.1.2	Relative Accuracy of SS-RSRP	496

10.2	E-UTRAN measurements	497

10.2.1	Introduction	497

10.2.2	E-UTRAN RSRP measurements	497

10.2.3	E-UTRAN RSRQ measurements	497

10.2.4	E-UTRAN RSTD measurements	498

10.2.5	E-UTRAN RS-SINR measurements	498

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	498

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	499

10.3	UTRAN FDD Measurements	499

10.3.1	UTRAN FDD CPICH RSCP	499

10.3.2	UTRAN FDD CPICH Ec/No	500

10.4	V2X measurements	500

10.4.1	Introduction	500

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	500

10.4.2.1	PSBCH-RSRP Absolute Accuracy	500

10.4.2.2	PSBCH-RSRP Relative Accuracy	501

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	502

10.4.3.1	Absolute SL-RSSI Accuracy	502

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	502

10.4.4.1	Absolute L1 SL-RSRP Accuracy	502

11	Void	503

12	V2X Requirements	503

12.1	Introduction	503

12.2	UE Transmit Timing	503

12.2.1	Introduction	503

12.2.2	GNSS as synchronization reference source	504

12.2.3	NR Cell as synchronization reference source	504

12.2.4	E-URTAN Cell as synchronization reference source	504

12.2.5	SyncRef UE as synchronization reference source	505

12.3	Initiation/Cease of SLSS Transmissions	505

12.3.1	Introduction	505

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	505

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	506

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	507

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	507

12.4	Selection / Reselection of V2X Synchronization Reference Source	508

12.5	L1 SL-RSRP measurements	509

12.5.1	Introduction	509

12.5.2	SL-RSRP measurements	509

12.6	Congestion Control measurements	509

12.7	Interruption	509

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	509

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	510

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	512

12.8	Reliability of GNSS signal	512

12.9	Scheduling availability	512

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	512

13	Measurement Performance Requirements for NR gNB	513

13.1	UL-RTOA	513

13.1.1	Report mapping	513

13.2	gNB Rx-Tx time difference	515

13.2.1	Report mapping	515

13.2.2	Measurement Accuracy Requirements	517

13.2.2.1	Introduction	517

13.3	UL SRS RSRP measurement	518

13.3.1	Report mapping	518

13.3.2	Measurement accuracy requirements	519

13.3.2.1	Introduction	519

13.3.2.2	Requirements	519

13.4	AoA/ZoA	520

13.4.1	Report mapping	520

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.1A	Reference measurement channels under CCA	439

A.3.1A.1	PDSCH	439

A.3.1A.1.1	TDD	439

A.3.1A.2	CORESET for RMSI scheduling	440

A.3.1A.2.1	TDD	440

A.3.1A.3	CORESET for RMC scheduling	441

A.3.1A.3.1	TDD	441

A.3.1A.4	TDD UL/DL configuration	442

A.3.1A.5	RMC burst transmission model	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	443

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	444

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	446

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	446

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	447

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	448

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	448

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	449

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	449

A.3.4	Test Cases with Different Channel Bandwidths	449

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	449

A.3.4.1.1	Introduction	449

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	450

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	450

A.3.5.1.1	Introduction	450

A.3.5.1.2	Principle of Testing	450

A.3.6	Antenna configurations	450

A.3.6.1	Antenna configurations for FR1	450

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	450

A.3.6.1.1.1	Introduction	450

A.3.6.1.1.2	Principle of testing	450

A.3.6.2	Antenna configurations for FR2	453

A.3.6A	Antenna configurations with unlicensed bands	453

A.3.6A.1	Antenna configurations for FR1	453

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	453

A.3.6A.1.1.1	Introduction	453

A.3.6A.1.1.2	Principle of testing	453

A.3.7	EN-DC test setup	455

A.3.7.1	Introduction	455

A.3.7.2	E-UTRAN Serving Cell Parameters	455

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	455

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	456

A.3.7A	NR FR1-FR2 test setup	457

A.3.7B	EN-DC test setup with unlicensed bands	458

A.3.7B.1	Introduction	458

A.3.7B.2	E-UTRAN Serving Cell Parameters	458

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	458

A.3.7C	LTE-FR1/FR2 test setup	459

A.3.8	PRACH configurations	459

A.3.8.1	Introduction	459

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	460

A.3.8.2.3	FR1 PRACH configuration 3	461

A.3.8.2.4	FR1 PRACH configuration 4	462

A.3.8.3	PRACH configurations in FR2	463

A.3.8.3.1	FR2 PRACH configuration 1	463

A.3.8.3.2	FR2 PRACH configuration 2	464

A.3.8.3.3	FR2 PRACH configuration 3	465

A.3.8.3.4	FR2 PRACH configuration 4	466

A.3.8A	PRACH configurations under CCA	467

A.3.8A.1	Introduction	467

A.3.8A.2	PRACH configurations in FR1	467

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	467

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	468

A.3.9	BWP configurations	469

A.3.9.1	Introduction	469

A.3.9.2	Downlink BWP configurations	470

A.3.9.2.1	Initial BWP	470

A.3.9.2.2	Dedicated BWP	470

A.3.9.3	Uplink BWP configurations	470

A.3.9.3.1	Initial BWP	470

A.3.9.3.2	Dedicated BWP	471

A.3.10	SSB Configurations	471

A.3.10.1	SSB Configurations for FR1	471

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	471

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	473

A.3.10A	SSB Configurations under CCA	478

A.3.10A.1	SSB Configurations under CCA for FR1	478

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	479

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	480

A.3.11	SMTC Configurations	480

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	481

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	482

A.3.12.2.1	Introduction	482

A.3.12.2.2	Principle of testing	482

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	483

A.3.13A.4	Principle of Testing in E-UTRA	483

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	487

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	493

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	493

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	493

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	493

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	494

A.3.15.3	Setup 3: 2 AoAs	494

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	494

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	494

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	494

A.3.16	TCI State Configuration	495

A.3.16.1	Introduction	495

A.3.16.2	TCI states	495

A.3.17	Configurations of CSI-RS for tracking	495

A.3.17.1	Configuration of CSI-RS for tracking for FR1	495

A.3.17.1.1	FDD	495

A.3.17.1.2	TDD	496

A.3.17.2	Configuration of CSI-RS for tracking for FR2	497

A.3.17.2.1	TDD	497

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	498

A.3.18.1	Introduction	498

A.3.18.2	PRACH Power Measurement	498

A.3.19	Test applicability for DAPS handover	498

A.3.19.1	Introduction	498

A.3.19.2	Principle of testing	498

A.3.20	MsgA configurations	499

A.3.20.1	Introduction	499

A.3.20.2	MsgA configurations in FR1	499

A.3.20.2.1	FR1 MsgA configuration 1	499

A.3.20.2.2	FR1 MsgA configuration 2	500

A.3.20.3	MsgA configurations in FR2	502

A.3.20.3.1	FR2 MsgA configuration 1	502

A.3.20.3.2	FR2 MsgA configuration 2	503

A.3.20A	MsgA configurations under CCA	505

A.3.20A.1	Introduction	505

A.3.20A.2	MsgA configurations in FR1	505

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	505

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	506

A.3.21	V2X sidelink communication	509

A.3.21.1	Introduction	509

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	509

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	515

A.3.22	CSI-IM configurations	515

A.3.22.1	FDD	515

A.3.22.2	TDD	516

A.3.23	Spatial Relation Configuration	518

A.3.23.1	Introduction	518

A.3.23.2	Spatial Relation	518

A.3.24	SRS configuration	519

A.3.25	Channel bandwidth (CBW) configurations	521

A.3.25.1	DL UE specific CBW	521

A.3.25.2	UL UE specific CBW	521

A.3.26	CCA model	521

A.3.26.1	Introduction	521

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	521

A.3.26.2.1	DL CCA model	521

A.3.26.2.2	UL CCA model	522

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	523

A.3.27.1	Introduction	523

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	523

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	523

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	523

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	523

A.3.28	Discovery Burst Transmission Window configuration under CCA	524

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	524

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	524

A.3.29.1	Introduction	524

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	524

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	524

A.3.30	CSI-RS configurations for RRM	525

A.3.30.1	FDD	525

A.3.30.2	TDD	526

A.3.31	PRS Configurations	528

A.3.31.1.	PRS Configurations for FR1	528

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	528

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	529

A.3.31.2.	PRS Configurations for FR2	529

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	529

A.4	EN-DC tests with all NR cells in FR1	530

A.4.1	Void	530

A.4.2	Void	530

A.4.3	RRC_CONNECTED state mobility	530

A.4.3.1	Void	530

A.4.3.2	RRC Connection Mobility Control	530

A.4.3.2.1	Void	530

A.4.3.2.2	Random Access	530

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	530

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	534

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	538

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	542

A.4.3.2.3	Void	546

A.4.4	Timing	546

A.4.4.1	UE transmit timing	546

A.4.4.1.1	NR UE Transmit Timing Test for FR1	546

A.4.4.1.1.1	Test Purpose and environment	546

A.4.4.1.1.2	Test requirements	550

A.4.4.2	UE timer accuracy	550

A.4.4.3	Timing advance	550

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	550

A.4.4.3.1.1	Test Purpose and Environment	550

A.4.4.3.1.2	Test Parameters	550

A.4.4.3.1.3	Test Requirements	554

A.4.5	Signaling characteristics	554

A.4.5.1	Radio link Monitoring	554

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	555

A.4.5.1.1.1	Test Purpose and Environment	555

A.4.5.1.1.2	Test Requirements	559

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	559

A.4.5.1.2.1	Test Purpose and Environment	559

A.4.5.1.2.2	Test Requirements	565

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	565

A.4.5.1.3.1	Test Purpose and Environment	565

A.4.5.1.3.2	Test Requirements	570

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	570

A.4.5.1.4.1	Test Purpose and Environment	570

A.4.5.1.4.2	Test Requirements	576

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	576

A.4.5.1.5.1	Test Purpose and Environment	576

A.4.5.1.5.2	Test Requirements	581

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	581

A.4.5.1.6.1	Test Purpose and Environment	581

A.4.5.1.6.2	Test Requirements	587

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	587

A.4.5.1.7.1	Test Purpose and Environment	587

A.4.5.1.7.2	Test Requirements	592

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	593

A.4.5.1.8.1	Test Purpose and Environment	593

A.4.5.1.8.2	Test Requirements	598

A.4.5.2	Interruption	598

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	598

A.4.5.2.1.1	Test Purpose and Environment	598

A.4.5.2.1.2	Test Requirements	602

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	602

A.4.5.2.2.1	Test Purpose and Environment	602

A.4.5.2.2.2	Test Requirements	606

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	606

A.4.5.2.3.1	Test Purpose and Environment	606

A.4.5.2.3.2	Test Requirements	610

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	611

A.4.5.2.4.1	Test Purpose and Environment	611

A.4.5.2.4.2	Test Requirements	614

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	615

A.4.5.2.5.1	Test Purpose and Environment	615

A.4.5.2.5.2	Test Requirements	618

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	618

A.4.5.2.6.1	Test Purpose and Environment	618

A.4.5.2.6.2	Test Requirements	622

A.4.5.2.7	Void	622

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	622

A.4.5.2.8.1	Test Purpose and Environment	622

A.4.5.2.8.2	Test Requirements	625

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	626

A.4.5.2.9.1	Test Purpose and Environment	626

A.4.5.2.9.2	Test Requirements	629

A.4.5.3	SCell Activation and Deactivation Delay	630

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	630

A.4.5.3.1.1	Test Purpose and Environment	630

A.4.5.3.1.2	Test Requirements	635

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	636

A.4.5.3.2.1	Test Purpose and Environment	636

A.4.5.3.2.2	Test Requirements	636

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	636

A.4.5.3.3.1	Test Purpose and Environment	636

A.4.5.3.3.2	Test Requirements	637

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	637

A.4.5.3.4.1	Test Purpose and Environment	637

A.4.5.3.3.2	Test Requirements	640

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	640

A.4.5.3.5.1	Test Purpose and Environment	640

A.4.5.3.5.2	Test Requirements	645

A.4.5.4	UE UL carrier RRC reconfiguration Delay	645

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	645

A.4.5.4.1.1 Test Purpose and Environment	645

A.4.5.4.1.2	Test Requirements	655

A.4.5.5	Beam Failure Detection and Link recovery procedures	655

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	655

A.4.5.5.1.1	Test Purpose and Environment	655

A.4.5.5.1.2	Test Requirements	661

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	661

A.4.5.5.2.1	Test Purpose and Environment	661

A.4.5.5.2.2	Test Requirements	667

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	667

A.4.5.5.3.1	Test Purpose and Environment	667

A.4.5.5.3.2	Test Requirements	673

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	673

A.4.5.5.4.1	Test Purpose and Environment	673

A.4.5.5.4.2	Test Requirements	679

A.4.5.6	Active BWP switch	693

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	693

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	693

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	698

A.4.5.6.2	RRC-based Active BWP Switch	703

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	707

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	707

A.4.5.6.3.1.1	Test Purpose and Environment	707

A.4.5.6.3.1.2	Test Requirements	712

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	713

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	713

A.4.5.6.4.1.1	Test Purpose and Environment	713

A.4.5.6.4.1.2	Test Requirements	717

A.4.5.6.5	SCell dormancy switch	717

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	717

A.4.5.6.5.1.1	Test Purpose and Environment	717

A.4.5.6.5.1.2	Test Requirements	723

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	723

A.4.5.6.5.2.1	Test Purpose and Environment	723

A.4.5.6.5.2.2	Test Requirements	728

A.4.5.7	PSCell addition and release delay	728

A.4.5.7.1	Addition and Release Delay of known NR PSCell	728

A.4.5.7.1.1	Test purpose and environment	728

A.4.5.7.1.2	Test Requirements	732

A.4.5.8	DL Interruptions at switching between two uplink carriers	733

A.4.5.8.1	Test Purpose and Environment	733

A.4.5.8.2	Test Requirements	736

A.4.6	Measurement procedure	736

A.4.6.1	Intra-frequency Measurements	736

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	736

A.4.6.1.1.1	Test purpose and Environment	736

A.4.6.1.1.2	Test parameters	736

A.4.6.1.1.3	Test Requirements	740

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	740

A.4.6.1.2.1	Test purpose and Environment	740

A.4.6.1.2.2	Test parameters	740

A.4.6.1.2.2	Test Requirements	744

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	744

A.4.6.1.3.1	Test purpose and Environment	744

A.4.6.1.3.2	Test parameters	744

A.4.6.1.3.3	Test Requirements	748

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	748

A.4.6.1.4.1	Test purpose and Environment	748

A.4.6.1.4.2	Test parameters	748

A.4.6.1.4.3	Test Requirements	752

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	752

A.4.6.1.5.1	Test purpose and Environment	752

A.4.6.1.5.2	Test parameters	752

A.4.6.1.5.3	Test Requirements	754

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	755

A.4.6.1.6.1	Test purpose and Environment	755

A.4.6.1.6.2	Test parameters	755

A.4.6.1.6.3	Test Requirements	757

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	758

A.4.6.1.7.1	Test purpose and Environment	758

A.4.6.1.7.2	Test parameters	758

A.4.6.1.7.3	Test Requirements	762

A.4.6.2	Inter-frequency Measurements	762

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	762

A.4.6.2.1.1	Test Purpose and Environment	762

A.4.6.2.1.2	Test Requirements	766

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	766

A.4.6.2.2.1	Test Purpose and Environment	766

A.4.6.2.2.2	Test Requirements	770

A.4.6.2.3	Void	771

A.4.6.2.4	Void	771

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	771

A.4.6.2.5.1	Test Purpose and Environment	771

A.4.6.2.5.2	Test Requirements	775

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	775

A.4.6.2.6.1	Test Purpose and Environment	775

A.4.6.2.6.2	Test Requirements	780

A.4.6.2.7	Void	781

A.4.6.2.8	Void	781

A.4.6.3	Void	781

A.4.6.4	L1-RSRP measurement for beam reporting	781

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	781

A.4.6.4.1.1	Test Purpose and Environment	781

A.4.6.4.1.2	Test parameters	781

A.4.6.4.1.3	Test Requirements	784

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	784

A.4.6.4.2.1	Test Purpose and Environment	784

A.4.6.4.2.2	Test parameters	785

A.4.6.4.2.3	Test Requirements	788

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	788

A.4.6.4.3.1	Test Purpose and Environment	788

A.4.6.4.3.2	Test parameters	789

A.4.6.4.3.3	Test Requirements	792

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	792

A.4.6.4.4.1	Test Purpose and Environment	792

A.4.6.4.4.2	Test parameters	793

A.4.6.4.4.3	Test Requirements	795

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	795

A.4.6.4.5.1	Test Purpose and Environment	795

A.4.6.4.5.2	Test parameters	796

A.4.6.4.2.3	Test Requirements	799

A.4.6.5	CLI measurements	799

A.4.6.5.1	SRS-RSRP measurement with non-DRX	799

A.4.6.5.1.1	Test Purpose and Environment	799

A.4.6.5.1.2	Test Parameters	800

A.4.6.5.1.3	Test Requirements	803

A.4.6.5.2	CLI-RSSI measurement with non-DRX	803

A.4.6.5.2.1	Test Purpose and Environment	803

A.4.6.5.2.2	Test Parameters	804

A.4.6.5.2.3	Test Requirements	805

A.4.6.6.1.2	Test Requirements	810

A.4.6.7	L1-SINR measurement for beam reporting	810

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	813

A.4.6.7.2.1	Test Purpose and Environment	813

A.4.6.7.2.2	Test parameters	813

A.4.6.7.2.3	Test Requirements	816

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	816

A.4.6.7.3.1	Test Purpose and Environment	816

A.4.6.7.3.2	Test parameters	816

A.4.6.7.3.3	Test Requirements	818

A.4.6.8	CSI-RS based intra-frequency Measurement	819

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	819

A.4.6.8.1.1	Test purpose and Environment	819

A.4.6.8.1.2	Test Requirements	823

A.4.6.9	CSI-RS based inter-frequency Measurement	823

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	823

A.4.6.9.1.1	Test Purpose and Environment	823

A.4.6.9.1.2	Test Requirements	827

A.4.7	Measurement Performance requirements	828

A.4.7.1	SS-RSRP	828

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	828

A.4.7.1.1.1	Test Purpose and Environment	828

A.4.7.1.1.2	Test parameters	828

A.4.7.1.1.3	Test Requirements	832

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	832

A.4.7.1.2.1	Test Purpose and Environment	832

A.4.7.1.2.2	Test parameters	833

A.4.7.1.2.3	Test Requirements	837

A.4.7.1.3	Void	837

A.4.7.2	SS-RSRQ	837

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	837

A.4.7.2.1.1	Test Purpose and Environment	837

A.4.7.2.1.2	Test Parameters	838

A.4.7.2.1.3	Test Requirements	843

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	843

A.4.7.2.2.1	Test Purpose and Environment	843

A.4.7.2.2.2	Test Parameters	843

A.4.7.2.2.3	Test Requirements	848

A.4.7.3	SS-SINR	848

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	848

A.4.7.3.1.1	Test Purpose and Environment	848

A.4.7.3.1.2	Test Parameters	848

A.4.7.3.1.3	Test Requirements	852

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	852

A.4.7.3.2.1	Test Purpose and Environment	852

A.4.7.3.2.2	Test Parameters	852

A.4.7.3.2.3	Test Requirements	857

A.4.7.4	L1-RSRP measurement for beam reporting	857

A.4.7.4.1	SSB based L1-RSRP measurement	857

A.4.7.4.1.1	Test Purpose and Environment	857

A.4.7.4.1.2	Test parameters	858

A.4.7.4.1.3	Test Requirements	861

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	861

A.4.7.4.2.1	Test Purpose and Environment	861

A.4.7.4.2.2	Test parameters	862

A.4.7.4.2.3	Test Requirements	865

A.4.7.5	SFTD accuracy	865

A.4.7.5.1	SFTD accuracy	865

A.4.7.5.1.1	Test Purpose and Environment	865

A.4.7.5.1.2	Test Parameters	865

A.4.7.5.1.3	Test Requirements	870

A.4.7.5.2	Void	870

A.4.7.5.3	Void	870

A.4.7.6	CLI measurements	870

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	870

A.4.7.6.1.1	Test Purpose and Environment	870

A.4.7.6.1.2	Test parameters	871

A.4.7.6.1.3	Test Requirements	876

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	876

A.4.7.6.2.1	Test Purpose and Environment	876

A.4.7.6.2.2	Test parameters	877

A.4.7.6.2.3	Test Requirements	880

A.4.7.7	L1-SINR measurement for beam reporting	880

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	884

A.4.7.7.2.1	Test Purpose and Environment	884

A.4.7.7.2.2	Test parameters	884

A.4.7.7.2.3	Test Requirements	888

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	888

A.4.7.7.3.1	Test Purpose and Environment	888

A.4.7.7.3.2	Test parameters	888

A.4.7.7.3.3	Test Requirements	891

A.4.7.8	CSI-RSRP	891

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	891

A.4.7.8.1.1	Test Purpose and Environment	891

A.4.7.8.1.2	Test parameters	892

A.4.7.8.1.3	Test Requirements	897

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	897

A.4.7.8.2.1	Test Purpose and Environment	897

A.4.7.8.2.2	Test parameters	897

A.4.7.8.2.3	Test Requirements	902

A.4.7.9	CSI-RSRQ	902

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	902

A.4.7.9.1.1	Test Purpose and Environment	902

A.4.7.9.1.2	Test Parameters	902

A.4.7.9.1.3	Test Requirements	907

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	907

A.4.7.9.2.1	Test Purpose and Environment	907

A.4.7.9.2.2	Test Parameters	907

A.4.7.9.2.3	Test Requirements	912

A.4.7.10	CSI-SINR	912

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	912

A.4.7.10.1.1	Test Purpose and Environment	912

A.4.7.10.1.2	Test Parameters	913

A.4.7.10.1.3	Test Requirements	917

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	917

A.4.7.10.2.1	Test Purpose and Environment	917

A.4.7.10.2.2	Test Parameters	918

A.4.7.10.2.3	Test Requirements	922

A.4.8	Void	922

A.5	EN-DC tests with one or more NR cells in FR2	923

A.5.1	Void	923

A.5.2	Void	923

A.5.3	RRC_CONNECTED state mobility	923

A.5.3.1	Void	923

A.5.3.2	RRC Connection Mobility Control	923

A.5.3.2.1	Void	923

A.5.3.2.2	Random Access	923

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	923

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	926

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	931

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	935

A.5.3.2.3	Void	939

A.5.4	Timing	939

A.5.4.1	UE transmit timing	939

A.5.4.1.1	NR UE Transmit Timing Test for FR2	939

A.5.4.1.1.1	Test Purpose and environment	939

A.5.4.1.1.2	Test requirements	942

A.5.4.2	UE timer accuracy	943

A.5.4.3	Timing advance	943

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	943

A.5.4.3.1.1 Test Purpose and Environment	943

A.5.4.3.1.2 Test Parameters	943

A.5.4.3.1.3	Test Requirements	947

A.5.5	Signaling characteristics	947

A.5.5.1	Radio link Monitoring	947

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	947

A.5.5.1.1.1	Test Purpose and Environment	947

A.5.5.1.1.2	Test Requirements	950

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	951

A.5.5.1.2.1	Test Purpose and Environment	951

A.5.5.1.2.2	Test Requirements	954

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	955

A.5.5.1.3.1	Test Purpose and Environment	955

A.5.5.1.3.2	Test Requirements	958

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	958

A.5.5.1.4.1	Test Purpose and Environment	958

A.5.5.1.4.2	Test Requirements	962

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	962

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	968

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	973

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	979

A.5.5.1.8.2	Test Requirements	985

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	986

A.5.5.1.9.1	Test Purpose and Environment	986

A.5.5.1.9.2	Test Requirements	988

A.5.5.2	Interruption	988

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	988

A.5.5.2.1.1	Test Purpose and Environment	988

A.5.5.2.1.2	Test Requirements	991

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	991

A.5.5.2.2.1	Test Purpose and Environment	991

A.5.5.2.2.2	Test Requirements	994

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	994

A.5.5.2.3.1	Test Purpose and Environment	994

A.5.5.2.3.2	Test Requirements	998

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	998

A.5.5.2.4.1	Test Purpose and Environment	998

A.5.5.2.4.2	Test Requirements	1001

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1002

A.5.5.2.5.1	Test Purpose and Environment	1002

A.5.5.2.5.2	Test Requirements	1005

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1005

A.5.5.2.6.1	Test Purpose and Environment	1005

A.5.5.2.6.2	Test Requirements	1008

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1008

A.5.5.2.7.1	Test Purpose and Environment	1008

A.5.5.2.7.2	Test Requirements	1011

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1012

A.5.5.2.8.1 Test Purpose and Environment	1012

A.5.5.2.8.3	Test Requirements	1014

A.5.5.3	SCell Activation and Deactivation Delay	1014

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1014

A.5.5.3.1.1	Test Purpose and Environment	1014

A.5.5.3.1.2	Test Requirements	1017

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1017

A.5.5.3.2.1	Test Purpose and Environment	1017

A.5.5.3.2.2	Test Requirements	1021

A.5.5.3.3	Void	1021

A.5.5.3.4	Void	1021

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1021

A.5.5.3.5.1	Test Purpose and Environment	1021

A.5.5.3.5.2	Test Requirements	1026

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1027

A.5.5.3.6.1	Test Purpose and Environment	1027

A.5.5.3.6.2	Test Requirements	1030

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1031

A.5.5.3.7.1	Test Purpose and Environment	1031

A.5.5.3.7.2	Test Requirements	1034

A.5.5.4	Void	1034

A.5.5.5	Beam Failure Detection and Link recovery procedures	1034

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1034

A.5.5.5.1.1	Test Purpose and Environment	1034

A.5.5.5.1.2	Test Requirements	1040

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1040

A.5.5.5.2.1	Test Purpose and Environment	1040

A.5.5.5.2.2	Test Requirements	1046

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1046

A.5.5.5.3.1	Test Purpose and Environment	1046

A.5.5.5.3.2	Test Requirements	1052

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1052

A.5.5.5.4.1	Test Purpose and Environment	1052

A.5.5.5.4.2	Test Requirements	1058

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1058

A.5.5.5.5.1	Test Purpose and Environment	1058

A.5.5.5.5.2	Test Requirements	1064

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1064

A.5.5.5.6.1	Test Purpose and Environment	1064

A.5.5.5.6.2	Test Requirements	1069

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1069

A.5.5.5.7.1	Test Purpose and Environment	1069

A.5.5.5.7.2	Test Requirements	1075

A.5.5.6	Active BWP switch	1075

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1075

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1075

A.5.5.6.1.1.1	Test Purpose and Environment	1075

A.5.5.6.1.1.2	Test Requirements	1079

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1079

A.5.5.6.2	RRC-based Active BWP Switch	1084

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1084

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1088

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1088

A.5.5.6.3.1.1	Test Purpose and Environment	1088

A.5.5.6.3.1.2	Test Requirements	1092

A.5.5.6.4	SCell dormancy switch	1093

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1093

A.5.5.6.4.1.1	Test Purpose and Environment	1093

A.5.5.6.4.1.2	Test Requirements	1097

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1097

A.5.5.6.4.2.1	Test Purpose and Environment	1097

A.5.5.6.4.2.2	Test Requirements	1104

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1104

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1104

A.5.5.7	PSCell addition and release delay	1107

A.5.5.7.1	Addition and Release Delay of NR PSCell	1107

A.5.5.7.1.1	Test purpose and environment	1107

A.5.5.7.1.2	Test Requirements	1110

A.5.5.8	Active TCI state switch delay	1111

A.5.5.8.1	MAC-CE based active TCI state switch	1111

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1111

A.5.5.8.1.1.1	Test Purpose and Environment	1111

A.5.5.8.1.1.2	Test Requirements	1113

A.5.5.8.2	RRC based active TCI state switch	1113

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1113

A.5.5.8.2.1.1	Test Purpose and Environment	1113

A.5.5.8.2.1.2	Test Requirements	1116

A.5.5.9	Uplink spatial relation switch delay	1116

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1116

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1116

A.5.5.9.1.1.1	Test Purpose and Environment	1116

A.5.5.9.1.1.2	Test Requirements	1119

A.5.5.9.2	RRC based spatial relation switch	1119

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1119

A.5.5.9.2.1.1	Test Purpose and Environment	1119

A.5.5.9.2.1.2	Test Requirements	1122

A.5.5.10	UE specific CBW change	1122

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1122

A.5.5.10.1.1	Test Purpose and Environment	1122

A.5.5.10.1.2	Test Requirements	1125

A.5.6	Measurement procedure	1126

A.5.6.1	Intra-frequency Measurements	1126

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1126

A.5.6.1.1.1	Test purpose and Environment	1126

A.5.6.1.1.2	Test Requirements	1130

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1130

A.5.6.1.2.1	Test purpose and Environment	1130

A.5.6.1.2.2	Test Requirements	1132

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1133

A.5.6.1.3.1	Test purpose and Environment	1133

A.5.6.1.3.2	Test Requirements	1137

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1137

A.5.6.1.4.1	Test purpose and Environment	1137

A.5.6.1.4.2	Test Requirements	1140

A.5.6.2	Inter-frequency Measurements	1141

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1141

A.5.6.2.1.1	Test Purpose and Environment	1141

A.5.6.2.1.2	Test Requirements	1144

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1144

A.5.6.2.2.1	Test Purpose and Environment	1144

A.5.6.2.2.2	Test Requirements	1148

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1148

A.5.6.2.3.1	Test Purpose and Environment	1148

A.5.6.2.3.2	Test Requirements	1152

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1152

A.5.6.2.4.1	Test Purpose and Environment	1152

A.5.6.2.4.2	Test Requirements	1156

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1156

A.5.6.2.5.1	Test Purpose and Environment	1156

A.5.6.2.5.2	Test Requirements	1161

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1161

A.5.6.2.6.1	Test Purpose and Environment	1161

A.5.6.2.6.2	Test Requirements	1165

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1165

A.5.6.2.7.1	Test Purpose and Environment	1165

A.5.6.2.7.2	Test Requirements	1170

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1170

A.5.6.2.8.1	Test Purpose and Environment	1170

A.5.6.2.8.2	Test Requirements	1175

A.5.6.3	L1-RSRP measurement for beam reporting	1176

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1176

A.5.6.3.1.1	Test Purpose and Environment	1176

A.5.6.3.1.2	Test parameters	1176

A.5.6.3.1.3	Test Requirements	1178

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1178

A.5.6.3.2.1	Test Purpose and Environment	1178

A.5.6.3.2.2	Test parameters	1179

A.5.6.3.2.3	Test Requirements	1181

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1181

A.5.6.3.3.1	Test Purpose and Environment	1181

A.5.6.3.3.2	Test parameters	1182

A.5.6.3.3.3	Test Requirements	1184

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1184

A.5.6.3.4.1	Test Purpose and Environment	1184

A.5.6.3.4.2	Test parameters	1185

A.5.6.3.4.3	Test Requirements	1187

A.5.6.4	CLI measurements	1188

A.5.6.4.1	SRS-RSRP measurement with DRX	1188

A.5.6.4.1.1	Test Purpose and Environment	1188

A.5.6.4.1.2	Test Parameters	1188

A.5.6.4.1.3	Test Requirements	1190

A.5.6.4.2	CLI-RSSI measurement with DRX	1190

A.5.6.4.2.1	Test Purpose and Environment	1190

A.5.6.4.2.2	Test Parameters	1191

A.5.6.4.2.3	Test Requirements	1193

A.5.6.5	Measurements with autonomous gaps	1193

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1193

A.5.6.5.1.1	Test Purpose and Environment	1193

A.5.6.5.1.2	Test Requirements	1196

A.5.6.6	L1-SINR measurement for beam reporting	1197

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1199

A.5.6.6.2.1	Test Purpose and Environment	1199

A.5.6.6.2.2	Test parameters	1200

A.5.6.6.2.3	Test Requirements	1203

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1203

A.5.6.6.3.1	Test Purpose and Environment	1203

A.5.6.6.3.2	Test parameters	1203

A.5.6.6.3.3	Test Requirements	1205

A.5.6.7	CSI-RS based Intra-frequency Measurements	1205

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1205

A.5.6.7.1.1	Test purpose and Environment	1205

A.5.6.7.1.2	Test Requirements	1208

A.5.6.8	CSI-RS based Inter-frequency Measurements	1208

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1208

A.5.6.8.1.1	Test Purpose and Environment	1208

A.5.6.8.1.2	Test Requirements	1213

A.5.7	Measurement Performance requirements	1213

A.5.7.1	SS-RSRP	1213

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1213

A.5.7.1.1.1	Test Purpose and Environment	1213

A.5.7.1.1.2	Test parameters	1214

A.5.7.1.1.3	Test Requirements	1216

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1217

A.5.7.1.2.1	Test Purpose and Environment	1217

A.5.7.1.2.2	Test parameters	1217

A.5.7.1.2.3	Test Requirements	1221

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1222

A.5.7.1.3.1	Test Purpose and Environment	1222

A.5.7.1.3.2	Test parameters	1222

A.5.7.1.3.3	Test Requirements	1225

A.5.7.2	SS-RSRQ	1226

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1226

A.5.7.2.1.1	Test Purpose and Environment	1226

A.5.7.2.1.2	Test Parameters	1226

A.5.7.2.1.3	Test Requirements	1229

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1229

A.5.7.2.2.1	Test Purpose and Environment	1229

A.5.7.2.2.2	Test Parameters	1229

A.5.7.2.2.3	Test Requirements	1231

A.5.7.3	SS-SINR	1232

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1232

A.5.7.3.1.1	Test Purpose and Environment	1232

A.5.7.3.1.2	Test Parameters	1232

A.5.7.3.1.3	Test Requirements	1235

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1235

A.5.7.3.2.1	Test Purpose and Environment	1235

A.5.7.3.2.2	Test Parameters	1235

A.5.7.3.2.3	Test Requirements	1237

A.5.7.4	L1-RSRP measurement for beam reporting	1237

A.5.7.4.1	SSB based L1-RSRP measurement	1237

A.5.7.4.1.1	Test Purpose and Environment	1237

A.5.7.4.1.2	Test parameters	1238

A.5.7.4.1.3	Test Requirements	1240

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1241

A.5.7.4.2.1	Test Purpose and Environment	1241

A.5.7.4.2.2	Test parameters	1241

A.5.7.4.2.3	Test Requirements	1243

A.5.7.5	CLI measurements	1244

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1244

A.5.7.5.1.1	Test Purpose and Environment	1244

A.5.7.5.1.2	Test parameters	1244

A.5.7.5.1.3	Test Requirements	1247

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1248

A.5.7.5.2.1	Test Purpose and Environment	1248

A.5.7.5.2.2	Test parameters	1248

A.5.7.5.2.3	Test Requirements	1250

A.5.7.6	L1-SINR measurement for beam reporting	1251

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1253

A.5.7.6.2.1	Test Purpose and Environment	1253

A.5.7.6.2.2	Test parameters	1254

A.5.7.6.2.3	Test Requirements	1256

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1257

A.5.7.6.3.1	Test Purpose and Environment	1257

A.5.7.6.3.2	Test parameters	1257

A.5.7.6.3.3	Test Requirements	1259

A.5.7.7	CSI-RSRP	1259

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1259

A.5.7.7.1.1	Test Purpose and Environment	1259

A.5.7.7.1.2	Test parameters	1260

A.5.7.7.1.3	Test Requirements	1262

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1263

A.5.7.7.2.1	Test Purpose and Environment	1263

A.5.7.7.2.2	Test parameters	1263

A.5.7.7.2.3	Test Requirements	1267

A.5.7.8	CSI-RSRQ	1268

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1268

A.5.7.8.1.1	Test Purpose and Environment	1268

A.5.7.8.1.2	Test Parameters	1268

A.5.7.8.1.3	Test Requirements	1270

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1270

A.5.7.8.2.1	Test Purpose and Environment	1270

A.5.7.8.2.2	Test Parameters	1270

A.5.7.8.2.3	Test Requirements	1272

A.5.7.9	CSI-SINR	1272

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1272

A.5.7.9.1.1	Test Purpose and Environment	1272

A.5.7.9.1.2	Test Parameters	1272

A.5.7.9.1.3	Test Requirements	1275

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1275

A.5.7.9.2.1	Test Purpose and Environment	1275

A.5.7.9.2.2	Test Parameters	1275

A.5.7.9.2.3	Test Requirements	1278

A.5.8	Void	1278

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1027

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1027

A.6.1.1.5.1	Test Purpose and Environment	1027

A.6.1.1.5.2	Test Parameters	1027

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1041

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1041

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1041

A.6.1.2.1.1	Test Purpose and Environment	1041

A.6.1.2.1.2	Test Parameters	1041

A.6.1.2.1.3	Test Requirements	1044

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1045

A.6.1.2.2.1	Test Purpose and Environment	1045

A.6.1.2.2.2	Test Parameters	1045

A.6.1.2.2.3	Test Requirements	1048

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1049

A.6.1.2.3.1	Test Purpose and Environment	1049

A.6.1.2.3.2	Test Parameters	1049

A.6.1.2.3.3	Test Requirements	1052

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1053

A.6.1.2.4.1	Test Purpose and Environment	1053

A.6.1.2.4.2	Test Parameters	1053

A.6.1.2.4.3	Test Requirements	1056

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	1057

A.6.1.2.5.1	Test Purpose and Environment	1057

A.6.1.2.5.2	Test Parameters	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.1.1.7	Void	1062

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1115

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1133

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1133

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1137

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1142

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1147

A.6.3.2.3	SA: RRC Connection Release with Redirection	1151

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1151

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1155

A.6.3.3 Conditional handover	1162

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1162

A.6.3.3.1.1	Test Purpose and Environment	1162

A.6.3.3.1.2	Test Parameters	1162

A.6.3.3.1.3 Test Requirements	1166

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1166

A.6.3.3.2.1	Test Purpose and Environment	1166

A.6.3.3.2.2	Test Parameters	1166

A.6.3.3.2.3 Test Requirements	1170

A.6.4	Timing	1170

A.6.4.1	UE transmit timing	1170

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1170

A.6.4.1.1.1	Test Purpose and environment	1170

A.6.4.1.1.2	Test requirements	1174

A.6.4.2	UE timer accuracy	1174

A.6.4.3	Timing advance	1174

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1174

A.6.4.3.1.1	Test Purpose and Environment	1174

A.6.4.3.1.2	Test Parameters	1174

A.6.4.3.1.3	Test Requirements	1178

A.6.5	Signalling characteristics	1178

A.6.5.1	Radio link Monitoring	1178

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1179

A.6.5.1.1.1	Test Purpose and Environment	1179

A.6.5.1.1.2	Test Requirements	1183

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1183

A.6.5.1.2.1	Test Purpose and Environment	1183

A.6.5.1.2.2	Test Requirements	1189

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1189

A.6.5.1.3.1	Test Purpose and Environment	1189

A.6.5.1.3.2	Test Requirements	1195

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1195

A.6.5.1.4.1	Test Purpose and Environment	1195

A.6.5.1.4.2	Test Requirements	1201

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1201

A.6.5.1.5.1	Test Purpose and Environment	1201

A.6.5.1.5.2	Test Requirements	1206

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1206

A.6.5.1.6.1	Test Purpose and Environment	1206

A.6.5.1.6.2	Test Requirements	1212

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1212

A.6.5.1.7.1	Test Purpose and Environment	1212

A.6.5.1.7.2	Test Requirements	1216

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1216

A.6.5.1.8.1	Test Purpose and Environment	1216

A.6.5.1.8.2	Test Requirements	1222

A.6.5.2	Interruption	1222

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1222

A.6.5.2.1.2	Test Requirements	1226

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1227

A.6.5.2.2.1	Test Purpose and Environment	1227

A.6.5.2.2.2	Test Parameters	1227

A.6.5.2.2.3	Test Requirements	1229

A.6.5.3	SCell Activation and Deactivation Delay	1230

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1230

A.6.5.3.1.1	Test Purpose and Environment	1230

A.6.5.3.1.2	Test Requirements	1233

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1234

A.6.5.3.2.1	Test Purpose and Environment	1234

A.6.5.3.2.2	Test Requirements	1234

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1234

A.6.5.3.3.1	Test Purpose and Environment	1234

A.6.5.3.3.2	Test Requirements	1235

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1235

A.6.5.3.4.1	Test Purpose and Environment	1235

A.6.5.3.4.2	Test Requirements	1239

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1239

A.6.5.3.5.1	Test Purpose and Environment	1239

A.6.5.3.5.2	Test Requirements	1243

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1244

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1244

A.6.5.4.1.1	Test Purpose and Environment	1244

A.6.5.4.1.2	Test Requirements	1254

A.6.5.4.2	Void	1255

A.6.5.5	Beam Failure Detection and Link recovery procedures	1255

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1255

A.6.5.5.1.1	Test Purpose and Environment	1255

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1260

A.6.5.5.2.1	Test Purpose and Environment	1260

A.6.5.5.2.2	Test Requirements	1267

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1267

A.6.5.5.3.1	Test Purpose and Environment	1267

A.6.5.5.3.2	Test Requirements	1273

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1273

A.6.5.5.4.1	Test Purpose and Environment	1273

A.6.5.5.4.2	Test Requirements	1279

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1279

A.6.5.5.5.1	Test Purpose and Environment	1279

A.6.5.5.5.2	Test Requirements	1283

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1284

A.6.5.5.6.1	Test Purpose and Environment	1284

A.6.5.5.6.2	Test Requirements	1288

A.6.5.6	Active BWP switch	1288

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1288

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1288

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1294

A.6.5.6.2	RRC-based Active BWP Switch	1298

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1298

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1302

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1302

A.6.5.6.3.1.1	Test Purpose and Environment	1302

A.6.5.6.3.1.2	Test Requirements	1307

A.6.5.6.4	SCell dormancy switch	1308

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1308

A.6.5.6.4.1.1	Test Purpose and Environment	1308

A.6.5.6.4.1.2	Test Requirements	1313

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1314

A.6.5.6.4.2.1	 Test Purpose and Environment	1314

A.6.5.6.4.2.2	 Test Requirements	1318

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1318

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1318

A.6.5.7	DL interruptions at switching between two uplink carriers	1322

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1322

A.6.5.7.1.1	Test Purpose and Environment	1322

A.6.5.7.1.2	Test Requirements	1326

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1326

A.6.5.7.2.1	Test Purpose and Environment	1326

A.6.5.7.2.2	Test Requirements	1330

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1330

A.6.5.8.1.1	Test Purpose and Environment	1330

A.6.5.8.1.2	Test Requirements	1334

A.6.6	Measurement procedure	1334

A.6.6.1	Intra-frequency Measurements	1334

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1334

A.6.6.1.1.1	Test purpose and Environment	1334

A.6.6.1.1.2	Test parameters	1334

A.6.6.1.1.3	Test Requirements	1338

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1338

A.6.6.1.2.1	Test purpose and Environment	1338

A.6.6.1.2.2	Test parameters	1338

A.6.6.1.2.3	Test Requirements	1342

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1342

A.6.6.1.3.1	Test purpose and Environment	1342

A.6.6.1.3.2	Test parameters	1342

A.6.6.1.3.3	Test Requirements	1346

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1346

A.6.6.1.4.1	Test purpose and Environment	1346

A.6.6.1.4.2	Test parameters	1346

A.6.6.1.4.3	Test Requirements	1350

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1350

A.6.6.1.5.1	Test purpose and Environment	1350

A.6.6.1.5.2	Test parameters	1350

A.6.6.1.5.3	Test Requirements	1352

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1353

A.6.6.1.6.1	Test purpose and Environment	1353

A.6.6.1.6.2	Test parameters	1353

A.6.6.1.6.3	Test Requirements	1354

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1355

A.6.6.1.7.1	Test purpose and Environment	1355

A.6.6.1.7.2	Test parameters	1355

A.6.6.1.7.3	Test Requirements	1359

A.6.6.2	Inter-frequency Measurements	1359

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1359

A.6.6.2.1.1	Test Purpose and Environment	1359

A.6.6.2.1.2	Test Requirements	1363

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1363

A.6.6.2.2.1	Test Purpose and Environment	1363

A.6.6.2.2.2	Test Requirements	1367

A.6.6.2.3	Void	1368

A.6.6.2.4	Void	1368

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1368

A.6.6.2.5.1	Test Purpose and Environment	1368

A.6.6.2.5.2	Test Requirements	1372

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1372

A.6.6.2.6.1	Test Purpose and Environment	1372

A.6.6.2.6.2	Test Requirements	1376

A.6.6.2.7	Void	1377

A.6.6.2.8	Void	1377

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1377

A.6.6.2.9.1	Test Purpose and Environment	1377

A.6.6.2.9.2	Test Requirements	1380

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1380

A.6.6.2.10.1	Test Purpose and Environment	1380

A.6.6.2.10.2	Test Requirements	1384

A.6.6.3	Inter-RAT Measurements	1388

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1388

A.6.6.3.1.1	Test Purpose and Environment	1388

A.6.6.3.1.2	Test Requirements	1394

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1394

A.6.6.3.2.1	Test Purpose and Environment	1394

A.6.6.3.2.2	Test Requirements	1401

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1401

A.6.6.3.3.1	Test Purpose and Environment	1401

A.6.6.3.3.2	Test Requirements	1405

A.6.6.4	L1-RSRP measurement for beam reporting	1405

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1405

A.6.6.4.1.1	Test Purpose and Environment	1405

A.6.6.4.1.2	Test parameters	1406

A.6.6.4.1.3	Test Requirements	1409

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1409

A.6.6.4.2.1	Test Purpose and Environment	1409

A.6.6.4.2.2	Test parameters	1410

A.6.6.4.2.3	Test Requirements	1413

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1413

A.6.6.4.3.1	Test Purpose and Environment	1413

A.6.6.4.3.2	Test parameters	1414

A.6.6.4.3.3	Test Requirements	1417

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1417

A.6.6.4.4.1	Test Purpose and Environment	1417

A.6.6.4.4.2	Test parameters	1418

A.6.6.4.4.3	Test Requirements	1421

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1421

A.6.6.4.5.1	Test Purpose and Environment	1421

A.6.6.4.5.2	Test parameters	1422

A.6.6.4.5.3	Test Requirements	1424

A.6.6.5	1424

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1424

A.6.6.5.1.1	Test Purpose and Environment	1424

A.6.6.5.1.2	Test Requirements	1428

A.6.6.6	CLI measurements	1428

A.6.6.6.1	SRS-RSRP measurement with DRX	1428

A.6.6.6.1.1	Test Purpose and Environment	1428

A.6.6.6.1.2	Test Parameters	1429

A.6.6.6.1.3	Test Requirements	1432

A.6.6.6.2	CLI-RSSI measurement with DRX	1432

A.6.6.6.2.1	Test Purpose and Environment	1432

A.6.6.6.2.2	Test Parameters	1433

A.6.6.6.2.3	Test Requirements	1435

A.6.6.7	NR measurements with autonomous gaps	1435

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1435

A.6.6.7.1.1	Test Purpose and Environment	1435

A.6.6.7.1.2	Test Parameters	1435

A.6.6.7.1.3	Test Requirements	1438

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1438

A.6.6.7.2.1	Test Purpose and Environment	1438

A.6.6.7.2.2	Test Requirements	1441

A.6.6.8	L1-SINR measurement for beam reporting	1442

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1444

A.6.6.8.2.1	Test Purpose and Environment	1444

A.6.6.8.2.2	Test parameters	1444

A.6.6.8.2.3	Test Requirements	1449

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1449

A.6.6.8.3.1	Test Purpose and Environment	1449

A.6.6.8.3.2	Test parameters	1450

A.6.6.8.3.3	Test Requirements	1452

A.6.6.9	Idle Mode CA/DC Measurements	1452

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1452

A.6.6.9.1.1	Test Purpose and Environment	1452

A.6.6.9.1.2	Test Requirements	1459

A.6.6.10	CSI-RS based intra-frequency Measurements	1459

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1459

A.6.6.10.1.1	Test purpose and Environment	1459

A.6.6.10.1.2	Test Requirements	1462

A.6.6.11	CSI-RS based inter-frequency Measurements	1462

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1462

A.6.6.11.1.1	Test Purpose and Environment	1462

A.6.6.11.1.2	Test Requirements	1466

A.6.6.12	RSTD measurements	1466

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1466

A.6.6.12.1.1	Test Purpose and Environment	1466

A.6.6.12.1.2	Test Requirements	1472

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1473

A.6.6.12.2.1	Test Purpose and Environment	1473

A.6.6.12.2.2	Test Requirements	1478

A.6.6.13 PRS-RSRP measurements	1479

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1479

A.6.6.13.1.1	Test purpose and Environment	1479

A.6.6.13.1.2	Test Requirements	1483

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1483

A.6.6.13.2.1	Test purpose and Environment	1483

A.6.6.13.2.2	Test Requirements	1487

A.6.6.14	UE Rx-Tx time difference measurements	1487

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1487

A.6.6.14.1.1	Test purpose and environment	1487

A.6.6.14.1.2	Test requirements	1490

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1490

A.6.6.14.2.1	Test purpose and environment	1490

A.6.6.14.2.2	Test requirements	1493

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1493

A.6.6.15.1	Test Purpose and Environment	1493

A.6.6.15.2	Test Requirements	1501

A.6.7	Measurement Performance requirements	1501

A.6.7.1	SS-RSRP	1502

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1502

A.6.7.1.1.1	Test Purpose and Environment	1502

A.6.7.1.1.2	Test parameters	1502

A.6.7.1.1.3	Test Requirements	1507

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1507

A.6.7.1.2.1	Test Purpose and Environment	1507

A.6.7.1.2.2	Test parameters	1507

A.6.7.1.2.3	Test Requirements	1511

A.6.7.1.3	Void	1511

A.6.7.2	SS-RSRQ	1511

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1511

A.6.7.2.1.1	Test Purpose and Environment	1511

A.6.7.2.1.2	Test Parameters	1512

A.6.7.2.1.3	Test Requirements	1517

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1517

A.6.7.2.2.1	Test Purpose and Environment	1517

A.6.7.2.2.2	Test Parameters	1517

A.6.7.2.2.3	Test Requirements	1522

A.6.7.3	SS-SINR	1522

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1522

A.6.7.3.1.1	Test Purpose and Environment	1522

A.6.7.3.1.2	Test Parameters	1523

A.6.7.3.1.3	Test Requirements	1527

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1527

A.6.7.3.2.1	Test Purpose and Environment	1527

A.6.7.3.2.2	Test Parameters	1528

A.6.7.3.2.3	Test Requirements	1532

A.6.7.4	L1-RSRP measurement for beam reporting	1533

A.6.7.4.1	SSB based L1-RSRP measurement	1533

A.6.7.4.1.1	Test Purpose and Environment	1533

A.6.7.4.1.2	Test parameters	1533

A.6.7.4.1.3	Test Requirements	1537

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1537

A.6.7.4.2.1	Test Purpose and Environment	1537

A.6.7.4.2.2	Test parameters	1538

A.6.7.4.2.3	Test Requirements	1542

A.6.7.5	E-UTRAN RSRP	1542

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1542

A.6.7.5.1.1	Test Purpose and Environment	1542

A.6.7.5.1.2	Test parameters	1543

A.6.7.5.1.3	Test Requirements	1548

A.6.7.6	E-UTRAN RSRQ	1549

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1549

A.6.7.6.1.1	Test Purpose and Environment	1549

A.6.7.6.1.2	Test parameters	1549

A.6.7.6.1.3	Test Requirements	1554

A.6.7.7	E-UTRAN RS-SINR	1555

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1555

A.6.7.7.1.1	Test Purpose and Environment	1555

A.6.7.7.1.2	Test parameters	1555

A.6.7.7.1.3	Test Requirements	1560

A.6.7.8	CLI measurements	1561

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1561

A.6.7.8.1.1	Test Purpose and Environment	1561

A.6.7.8.1.2	Test parameters	1561

A.6.7.8.1.3	Test Requirements	1567

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1567

A.6.7.8.2.1	Test Purpose and Environment	1567

A.6.7.8.2.2	Test parameters	1568

A.6.7.8.2.3	Test Requirements	1571

A.6.7.9	L1-SINR measurement for beam reporting	1572

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1576

A.6.7.9.2.1	Test Purpose and Environment	1576

A.6.7.9.2.2	Test parameters	1577

A.6.7.9.2.3	Test Requirements	1582

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1582

A.6.7.9.3.1	Test Purpose and Environment	1582

A.6.7.9.3.2	Test parameters	1582

A.6.7.9.3.3	Test Requirements	1587

A.6.7.10	CSI-RSRP	1587

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1587

A.6.7.9.10.1	Test Purpose and Environment	1587

A.6.7.9.10.2	Test parameters	1588

A.6.7.10.1.3	Test Requirements	1593

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1593

A.6.7.9.10.1	Test Purpose and Environment	1593

A.6.7.10.2.2	Test parameters	1593

A.6.7.10.2.3	Test Requirements	1598

A.6.7.11	CSI-RSRQ	1598

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1598

A.6.7.11.1.1	Test Purpose and Environment	1598

A.6.7.11.1.2	Test Parameters	1598

A.6.7.11.1.3	Test Requirements	1603

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1603

A.6.7.11.2.1	Test Purpose and Environment	1603

A.6.7.11.2.2	Test Parameters	1603

A.6.7.11.2.3	Test Requirements	1609

A.6.7.12	CSI-SINR	1610

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1610

A.6.7.12.1.1	Test Purpose and Environment	1610

A.6.7.12.1.2	Test Parameters	1610

A.6.7.12.1.3	Test Requirements	1614

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1615

A.6.7.12.2.1	Test Purpose and Environment	1615

A.6.7.12.2.2	Test Parameters	1615

A.6.7.12.2.3	Test Requirements	1621

A.6.7.13	RSTD measurements	1621

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1621

A.6.7.13.1.1	Test purpose and Environment	1621

A.6.7.13.1.2	Test Requirements	1624

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1624

A.6.7.13.2.1	Test purpose and Environment	1624

A.6.7.13.2.2	Test Requirements	1628

A.6.7.14	PRS-RSRP measurements	1628

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1628

A.6.7.14.1.1	Test Purpose and Environment	1628

A.6.7.14.1.2	Test parameters	1628

A.6.7.14.1.3	Test Requirements	1633

A.6.7.15	UE Rx-Tx time difference measurements	1633

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1633

A.6.7.15.1.1	Test purpose and environment	1633

A.6.7.15.1.2	Test parameters	1634

A.6.7.15.1.3	Test requirements	1637

A.7	NR standalone tests with one or more NR cells in FR2	1637

A.7.1	SA: RRC_IDLE state mobility	1637

A.7.1.1	Cell re-selection to NR	1637

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1637

A.7.1.1.1.1	Test Purpose and Environment	1637

A.7.1.1.1.2	Test Parameters	1637

A.7.1.1.1.3	Test Requirements	1641

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1641

A.7.1.1.2.1	Test Purpose and Environment	1641

A.7.1.1.2.2	Test Parameters	1641

A.7.1.1.2.3	Test Requirements	1645

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1645

A.7.1.1.3.1	Test Purpose and Environment	1645

A.7.1.1.3.2	Test Parameters	1645

A.7.1.1.3.3	Test Requirements	1648

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1648

A.7.1.1.4.1	Test Purpose and Environment	1648

A.7.1.1.4.2	Test Parameters	1648

A.7.1.1.4.3	Test Requirements	1651

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1651

A.7.1.1.5.1	Test Purpose and Environment	1651

A.7.1.1.5.2	Test Parameters	1651

A.7.1.1.5.3	Test Requirements	1654

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1654

A.7.1.1.6.1	Test Purpose and Environment	1654

A.7.1.1.6.2	Test Parameters	1654

A.7.1.1.6.3	Test Requirements	1656

A.7.2	SA: RRC_INACTIVE state mobility	1657

A.7.3	RRC_CONNECTED state mobility	1657

A.7.3.1	Handover	1657

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1657

A.7.3.1.1.1	Test Purpose and Environment	1657

A.7.3.1.1.2	Test Parameters	1657

A.7.3.1.1.3	Test Requirements	1661

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1661

A.7.3.1.2.1	Test Purpose and Environment	1661

A.7.3.1.2.2	Test Parameters	1661

A.7.3.1.2.3	Test Requirements	1664

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1664

A.7.3.1.3.1	Test Purpose and Environment	1664

A.7.3.1.3.2	Test Parameters	1664

A.7.3.1.3.3 Test Requirements	1666

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1667

A.7.3.1.4.1	Test Purpose and Environment	1667

A.7.3.1.4.2	Test Parameters	1667

A.7.3.1.4.3 Test Requirements	1674

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1674

A.7.3.1.5.1	Test Purpose and Environment	1674

A.7.3.1.5.2	Test Parameters	1674

A.7.3.1.5.3 Test Requirements	1681

A.7.3.2	RRC Connection Mobility Control	1681

A.7.3.2.1	SA: RRC Re-establishment	1681

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1681

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1684

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1687

A.7.3.2.1.3.1	Test Purpose and Environment	1687

A.7.3.2.1.3.2	Test Requirements	1689

A.7.3.2.2	Random Access	1690

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	1690

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	1694

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1698

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	1701

A.7.3.2.3	SA: RRC Connection Release with Redirection	1704

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1704

A.7.3.3 Conditional Handover	1708

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1708

A.7.3.3.1.1	Test Purpose and Environment	1708

A.7.3.3.1.2	Test Parameters	1708

A.7.3.3.1.2.3	Test Requirements	1711

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1711

A.7.3.3.2.1	Test Purpose and Environment	1711

A.7.3.3.2.2	Test Parameters	1711

A.7.3.3.2.3 Test Requirements	1713

A.7.4	Timing	1713

A.7.4.1	UE transmit timing	1713

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1713

A.7.4.1.1.1	Test Purpose and environment	1713

A.7.4.1.1.2	Test requirements	1716

A.7.4.2	UE timer accuracy	1717

A.7.4.3	Timing advance	1717

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1717

A.7.4.3.1.1	Test Purpose and Environment	1717

A.7.4.3.1.2	Test Parameters	1717

A.7.4.3.1.3 Test Requirements	1720

A.7.5	Signaling characteristics	1721

A.7.5.1	Radio link Monitoring	1721

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1721

A.7.5.1.1.1	Test Purpose and Environment	1721

A.7.5.1.1.2	Test Requirements	1724

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1725

A.7.5.1.2.1	Test Purpose and Environment	1725

A.7.5.1.2.2	Test Requirements	1729

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1730

A.7.5.1.3.1	Test Purpose and Environment	1730

A.7.5.1.3.2	Test Requirements	1733

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1733

A.7.5.1.4.1	Test Purpose and Environment	1733

A.7.5.1.4.2	Test Requirements	1737

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1737

A.7.5.1.5.1	Test Purpose and Environment	1737

A.7.5.1.5.2	Test Requirements	1741

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1741

A.7.5.1.6.1	Test Purpose and Environment	1741

A.7.5.1.6.2	Test Requirements	1745

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1745

A.7.5.1.7.1	Test Purpose and Environment	1745

A.7.5.1.7.2	Test Requirements	1748

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1748

A.7.5.1.8.1	Test Purpose and Environment	1748

A.7.5.1.8.2	Test Requirements	1753

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1753

A.7.5.1.9.1	Test Purpose and Environment	1753

A.7.5.1.9.2	Test Requirements	1756

A.7.5.2	Interruption	1756

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1756

A.7.5.2.1.1	Test Purpose and Environment	1756

A.7.5.2.1.2	Test Requirements	1759

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1760

A.7.5.2.2.1	Test Purpose and Environment	1760

A.7.5.2.2.2	Test Parameters	1760

A.7.5.2.2.3	Test Requirements	1762

A.7.5.3	SCell Activation and Deactivation Delay	1762

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1762

A.7.5.3.1.1	Test Purpose and Environment	1762

A.7.5.3.1.2	Test Requirements	1764

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1764

A.7.5.3.2.1	Test Purpose and Environment	1764

A.7.5.3.2.2	Test Requirements	1768

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1769

A.7.5.3.3.1	Test Purpose and Environment	1769

A.7.5.3.3.2	Test Requirements	1772

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1773

A.7.5.3.4.1	Test Purpose and Environment	1773

A.7.5.3.4.2	Test Requirements	1776

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1777

A.7.5.3.5.1	Test Purpose and Environment	1777

A.7.5.3.5.2	Test Requirements	1779

A.7.5.4	Void	1780

A.7.5.5	Beam Failure Detection and Link recovery procedures	1780

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1780

A.7.5.5.1.1	Test Purpose and Environment	1780

A.7.5.5.1.2	Test Requirements	1785

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1785

A.7.5.5.2.1	Test Purpose and Environment	1785

A.7.5.5.2.2	Test Requirements	1790

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1790

A.7.5.5.3.1	Test Purpose and Environment	1790

A.7.5.5.3.2	Test Requirements	1796

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1796

A.7.5.5.4.1	Test Purpose and Environment	1796

A.7.5.5.4.2	Test Requirements	1802

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1802

A.7.5.5.5.1	Test Purpose and Environment	1802

A.7.5.5.5.2	Test Requirements	1807

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1808

A.7.5.5.6.1	Test Purpose and Environment	1808

A.7.5.5.6.2	Test Requirements	1812

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1812

A.7.5.5.7.1	Test Purpose and Environment	1812

A.7.5.5.7.2	Test Requirements	1816

A.7.5.6	Active BWP switch	1816

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1816

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1816

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1821

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1826

A.7.5.6.1.3.1	Test Purpose and Environment	1826

A.7.5.6.1.3.2	Test Requirements	1829

A.7.5.6.2	RRC-based Active BWP Switch	1829

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1833

A.7.5.6.3.1.1	Test Purpose and Environment	1833

A.7.5.6.3.1.2	Test Requirements	1836

A.7.5.6.4	SCell dormancy switch	1836

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1836

A.7.5.6.4.1.1	Test Purpose and Environment	1836

A.7.5.6.4.1.2	Test Requirements	1841

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1841

A.7.5.6.4.2.1	 Test Purpose and Environment	1841

A.7.5.6.4.2.2	 Test Requirements	1846

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1846

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1846

A.7.5.7	PSCell addition and release delay	1849

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1849

A.7.5.7.1.1	Test Purpose and Environment	1849

A.7.5.7.1.2	Test Requirements	1852

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1852

A.7.5.7.2.1	Test Purpose and Environment	1852

A.7.5.7.2.2	Test Requirements	1855

A.7.5.8	Active TCI state switch delay	1855

A.7.5.8.1	MAC-CE based active TCI state switch	1855

A.7.5.8.2	RRC based active TCI state switch	1858

A.7.5.9	Uplink spatial relation switch delay	1861

A.7.5.9.1.1.1	Test Purpose and Environment	1861

A.7.5.9.1.1.2	Test Requirements	1864

A.7.5.9.2	RRC based spatial relation switch	1864

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1864

A.7.5.9.2.1.1	Test Purpose and Environment	1864

A.7.5.9.2.1.2	Test Requirements	1867

A.7.5.10	UE specific CBW change	1867

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1867

A.7.5.10.1.1	Test Purpose and Environment	1867

A.7.5.10.1.2	Test Requirements	1870

A.7.6	Measurement procedure	1871

A.7.6.1	Intra-frequency Measurements	1871

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1871

A.7.6.1.1.1	Test purpose and Environment	1871

A.7.6.1.1.2	Test Requirements	1874

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1874

A.7.6.1.2.1	Test purpose and Environment	1874

A.7.6.1.2.2	Test Requirements	1877

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1878

A.7.6.1.3.1	Test purpose and Environment	1878

A.7.6.1.3.2	Test Requirements	1881

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1881

A.7.6.1.4.1	Test purpose and Environment	1881

A.7.6.1.4.2	Test Requirements	1884

A.7.6.2	Inter-frequency Measurements	1885

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1885

A.7.6.2.1.1	Test Purpose and Environment	1885

A.7.6.2.1.2	Test Requirements	1888

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1888

A.7.6.2.2.1	Test Purpose and Environment	1888

A.7.6.2.2.2	Test Requirements	1892

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1892

A.7.6.2.3.1	Test Purpose and Environment	1892

A.7.6.2.3.2	Test Requirements	1896

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1896

A.7.6.2.4.1	Test Purpose and Environment	1896

A.7.6.2.4.2	Test Requirements	1900

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1900

A.7.6.2.5.1	Test Purpose and Environment	1900

A.7.6.2.5.2	Test Requirements	1904

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1905

A.7.6.2.6.1	Test Purpose and Environment	1905

A.7.6.2.6.2	Test Requirements	1909

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1910

A.7.6.2.7.1	Test Purpose and Environment	1910

A.7.6.2.7.2	Test Requirements	1914

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1915

A.7.6.2.8.1	Test Purpose and Environment	1915

A.7.6.2.8.2	Test Requirements	1919

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1920

A.7.6.2.9.1	Test Purpose and Environment	1920

A.7.6.2.9.2	Test Requirements	1924

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1924

A.7.6.2.10.1	Test Purpose and Environment	1924

A.7.6.2.10.2	Test Requirements	1926

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1926

A.7.6.2.11.1	Test Purpose and Environment	1926

A.7.6.2.11.2	Test Requirements	1929

A.7.6.3	L1-RSRP measurement for beam reporting	1930

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1930

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1930

A.7.6.3.1.1	Test Purpose and Environment	1930

A.7.6.3.1.2	Test parameters	1930

A.7.6.3.1.3	Test Requirements	1932

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1932

A.7.6.3.2.1	Test Purpose and Environment	1932

A.7.6.3.2.2	Test parameters	1933

A.7.6.3.2.3	Test Requirements	1935

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1935

A.7.6.3.3.1	Test Purpose and Environment	1935

A.7.6.3.3.2	Test parameters	1936

A.7.6.3.3.3	Test Requirements	1938

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1939

A.7.6.3.4.1	Test Purpose and Environment	1939

A.7.6.3.4.2	Test parameters	1939

A.7.6.3.3.3	Test Requirements	1941

A.7.6.4	CLI measurements	1942

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1942

A.7.6.4.1.1	Test Purpose and Environment	1942

A.7.6.4.1.2	Test Parameters	1942

A.7.6.4.1.3	Test Requirements	1944

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1944

A.7.6.4.2.1	Test Purpose and Environment	1944

A.7.6.4.2.2	Test Parameters	1945

A.7.6.4.2.3	Test Requirements	1946

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1947

A.7.6.5.1.1	Test Purpose and Environment	1947

A.7.6.5.1.2	Test Requirements	1950

A.7.6.6	L1-SINR measurement for beam reporting	1950

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1952

A.7.6.6.2.1	Test Purpose and Environment	1952

A.7.6.6.2.2	Test parameters	1953

A.7.6.6.2.3	Test Requirements	1955

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1955

A.7.6.6.3.1	Test Purpose and Environment	1955

A.7.6.6.3.2	Test parameters	1956

A.7.6.6.3.3	Test Requirements	1958

A.7.6.7	CSI-RS based intra-frequency Measurements	1958

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1958

A.7.6.7.1.1	Test purpose and Environment	1958

A.7.6.7.1.2	Test Requirements	1961

A.7.6.8	CSI-RS based inter-frequency Measurements	1961

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1961

A.7.6.8.1.1	Test Purpose and Environment	1961

A.7.6.8.1.2	Test Requirements	1964

A.7.6.9	RSTD measurements	1964

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1964

A.7.6.9.1.1	Test Purpose and Environment	1964

A.7.6.9.1.2	Test Requirements	1970

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1970

A.7.6.9.2.1	Test Purpose and Environment	1970

A.7.6.9.2.2	Test Requirements	1975

A.7.6.10 PRS-RSRP measurements	1975

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1975

A.7.6.10.1.1	Test Purpose and Environment	1975

A.7.6.10.1.2	Test Requirements	1979

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1979

A.7.6.10.2.1	Test Purpose and Environment	1979

A.7.6.10.2.2	Test Requirements	1983

A.7.6.11	UE Rx-Tx time difference measurements	1983

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1983

A.7.6.11.1.1	Test purpose and environment	1983

A.7.6.11.1.2	Test requirements	1986

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1986

A.7.6.11.2.1	Test purpose and environment	1986

A.7.6.11.2.2	Test requirements	1989

A.7.7	Measurement Performance requirements	1989

A.7.7.1	SS-RSRP	1989

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1989

A.7.7.1.1.1	Test Purpose and Environment	1989

A.7.7.1.1.2	Test parameters	1989

A.7.7.1.1.3	Test Requirements	1994

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1994

A.7.7.1.2.1	Test Purpose and Environment	1994

A.7.7.1.2.2	Test parameters	1995

A.7.7.1.2.3	Test Requirements	1999

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2000

A.7.7.1.3.1	Test Purpose and Environment	2000

A.7.7.1.3.2	Test parameters	2000

A.7.7.1.3.3	Test Requirements	2004

A.7.7.2	SS-RSRQ	2004

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2004

A.7.7.2.1.1	Test Purpose and Environment	2004

A.7.7.2.1.2	Test Parameters	2004

A.7.7.2.1.3	Test Requirements	2006

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2006

A.7.7.2.2.1	Test Purpose and Environment	2006

A.7.7.2.2.2	Test Parameters	2006

A.7.7.2.2.3	Test Requirements	2008

A.7.7.3	SS-SINR	2008

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2008

A.7.7.3.1.1	Test Purpose and Environment	2008

A.7.7.3.1.2	Test Parameters	2008

A.7.7.3.1.3	Test Requirements	2011

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2011

A.7.7.3.2.1	Test Purpose and Environment	2011

A.7.7.3.2.2	Test Parameters	2011

A.7.7.3.2.3	Test Requirements	2013

A.7.7.4	L1-RSRP measurement for beam reporting	2013

A.7.7.4.1	SSB based L1-RSRP measurement	2013

A.7.7.4.1.1	Test Purpose and Environment	2013

A.7.7.4.1.2	Test parameters	2014

A.7.7.4.1.3	Test Requirements	2016

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2017

A.7.7.4.2.1	Test Purpose and Environment	2017

A.7.7.4.2.2	Test parameters	2017

A.7.7.4.2.3	Test Requirements	2019

A.7.7.5	CLI measurements	2020

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2020

A.7.7.5.1.1	Test Purpose and Environment	2020

A.7.7.5.1.2	Test parameters	2020

A.7.7.5.1.3	Test Requirements	2023

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2024

A.7.7.5.2.1	Test Purpose and Environment	2024

A.7.7.5.2.2	Test parameters	2024

A.7.7.5.2.3	Test Requirements	2026

A.7.7.6	L1-SINR measurement for beam reporting	2027

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2029

A.7.7.6.2.1	Test Purpose and Environment	2029

A.7.7.6.2.2	Test parameters	2030

A.7.7.6.2.3	Test Requirements	2032

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2032

A.7.7.6.3.1	Test Purpose and Environment	2032

A.7.7.6.3.2	Test parameters	2033

A.7.7.6.3.3	Test Requirements	2035

A.7.7.7	CSI-RSRP	2035

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2035

A.7.7.7.1.1	Test Purpose and Environment	2035

A.7.7.7.1.2	Test parameters	2036

A.7.7.7.1.3	Test Requirements	2039

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2040

A.7.7.7.2.1	Test Purpose and Environment	2040

A.7.7.7.2.2	Test parameters	2040

A.7.7.7.2.3	Test Requirements	2043

A.7.7.8	CSI-RSRQ	2043

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2043

A.7.7.8.1.1	Test Purpose and Environment	2043

A.7.7.8.1.2	Test Parameters	2044

A.7.7.8.1.3	Test Requirements	2046

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2046

A.7.7.8.2.1	Test Purpose and Environment	2046

A.7.7.8.2.2	Test Parameters	2046

A.7.7.8.2.3	Test Requirements	2048

A.7.7.9	CSI-SINR	2048

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2048

A.7.7.9.1.1	Test Purpose and Environment	2048

A.7.7.9.1.2	Test Parameters	2048

A.7.7.9.1.3	Test Requirements	2050

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2050

A.7.7.9.2.1	Test Purpose and Environment	2050

A.7.7.9.2.2	Test Parameters	2051

A.7.7.9.2.3	Test Requirements	2053

A.7.7.10	RSTD measurements	2053

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2053

A.7.7.10.1.1	Test purpose and Environment	2053

A.7.7.10.1.2	Test Requirements	2056

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2056

A.7.7.10.2.1	Test purpose and Environment	2056

A.7.7.10.2.2	Test Requirements	2058

A.7.7.11	PRS-RSRP measurements	2058

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2058

A.7.7.11.1.1	Test Purpose and Environment	2058

A.7.7.11.1.2	Test parameters	2058

A.7.7.11.1.3	Test Requirements	2061

A.7.7.12	UE Rx-Tx time difference measurements	2062

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2062

A.7.7.12.1.1	Test purpose and environment	2062

A.7.7.12.1.2	Test parameters	2062

A.7.7.12.1.3	Test requirements	2064

A.8	E-UTRA standalone tests for NR RRM	2064

A.8.1	Void	2064

A.8.2	RRC_IDLE state mobility	2064

A.8.2.1	Inter-RAT NR Cell re-selection	2064

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2064

A.8.2.1.1.1	Test Purpose and Environment	2064

A.8.2.1.1.2	Test Requirements	2069

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2070

A.8.2.1.2.1	Test Purpose and Environment	2070

A.8.2.1.2.2	Test Requirements	2073

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2074

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2074

A.8.2.2.1.1	Test Purpose and Environment	2074

A.8.2.2.1.2	Test Requirements	2077

A.8.3	RRC_CONNECTED state mobility	2078

A.8.3.1	Handover	2078

A.8.3.1.1	E-UTRAN - NR handover in FR1	2078

A.8.3.1.1.1	Test Purpose and Environment	2078

A.8.3.1.1.2	Test Requirements	2083

A.8.4	Measurement procedure	2083

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2083

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2083

A.8.4.1.1.1	Test Purpose and Environment	2083

A.8.4.1.1.2	Test Requirements	2085

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2086

A.8.4.1.2.1	Test Purpose and Environment	2086

A.8.4.1.2.2	Test Requirements	2087

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2087

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2087

A.8.4.2.1.1	Test Purpose and Environment	2087

A.8.4.2.1.2	Test Requirements	2092

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2092

A.8.4.2.2.1	Test Purpose and Environment	2092

A.8.4.2.2.2	Test Requirements	2096

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2096

A.8.4.2.3.1	Test Purpose and Environment	2096

A.8.4.2.3.2	Test Requirements	2101

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2101

A.8.4.2.4.1	Test Purpose and Environment	2101

A.8.4.2.4.2	Test Requirements	2105

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2105

A.8.4.2.5.1	Test Purpose and Environment	2105

A.8.4.2.5.2	Test Requirements	2107

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2108

A.8.4.2.6.1	Test Purpose and Environment	2108

A.8.4.2.6.2	Test Requirements	2110

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2111

A.8.4.2.7.1	Test Purpose and Environment	2111

A.8.4.2.7.2	Test Requirements	2113

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2114

A.8.4.2.8.1	Test Purpose and Environment	2114

A.8.4.2.8.2	Test Requirements	2116

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2117

A.8.4.2.9.1	Test Purpose and Environment	2117

A.8.4.2.9.2	Test Requirements	2121

A.8.5	Measurement performance	2121

A.8.5.1	SFTD accuracy	2121

A.8.5.1.1	SFTD accuracy	2121

A.8.5.1.1.1	Test Purpose	2121

A.8.5.1.1.2	Test Environment	2122

A.8.5.1.1.3	Test Requirements	2128

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2128

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2128

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2133

A.8.5.2.1.2.1	Test Purpose and Environment	2133

A.8.5.2.1.2.2	Test Parameters	2133

A.8.5.2.1.2.3	Test Requirements	2135

A.8.5.2.2	SS-RSRQ	2135

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2135

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2140

A.8.5.2.2.2.1	Test Purpose and Environment	2140

A.8.5.2.2.2.2	Test Parameters	2140

A.8.5.2.2.2.3	Test Requirements	2142

A.8.5.2.3	SS-SINR	2142

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2142

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2146

A.8.5.2.3.2.1	Test Purpose and Environment	2146

A.8.5.2.3.2.2	Test Parameters	2146

A.8.5.2.3.2.3	Test Requirements	2148

A.9	V2X Tests	2149

A.9.1	V2X Tests in FR1	2149

A.9.1.1	Test for V2X UE Transmit Timing	2149

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2149

A.9.1.1.1.1	Test Purpose and Environment	2149

A.9.1.1.1.2	Test requirements	2149

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2149

A.9.1.1.2.1	Test Purpose and Environment	2149

A.9.1.1.2.2	Test requirements	2150

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2150

A.9.1.1.3.1	Test Purpose and Environment	2150

A.9.1.1.3.2	Test requirements	2153

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2153

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2153

A.9.1.2.1.1	Test Purpose and Environment	2153

A.9.1.2.1.2	Test Requirements	2156

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2156

A.9.1.2.2.1	Test Purpose and Environment	2156

A.9.1.2.2.2	Test Requirements	2157

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2158

A.9.1.3.1	 Test for GNSS configured as the highest priority	2158

A.9.1.3.1.1	Test Purpose and Environment	2158

A.9.1.3.1.2	Test Requirements	2160

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2161

A.9.1.3.2.1	Test Purpose and Environment	2161

A.9.1.3.2.2	Test Requirements	2163

A.9.1.4	Test for L1 SL-RSRP Measurement	2164

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2164

A.9.1.4.1.1	Test Purpose and Environment	2164

A.9.1.4.1.2	Test Requirements	2166

A. 9.1.4.2 Test for V2X UE Resource Pre-emption	2166

A. 9.1.4.2.1	Test Purpose and Environment	2166

A.9.1.4.2.2	Test Requirements	2170

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2170

A.9.1.4.3.1	Test Purpose and Environment	2170

A.9.1.4.3.2	Test Requirements	2174

A.9.1.5	Test for Congestion Control Measurement	2174

A.9.1.5.1	Test Purpose and Environment	2174

A.9.1.5.2	Test Requirements	2177

A.9.1.6	Test for Interruption	2177

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2177

A.9.1.6.1.1	Test Purpose and Environment	2177

A.9.1.6.1.2	Test Requirements	2181

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2181

A.10.1	RRC_CONNECTED state mobility	2181

A.10.1.1	RRC connection mobility control	2181

A.10.1.1.1	Random Access	2181

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2181

A.10.1.1.1.1.1	Test Purpose and Environment	2181

A.10.1.1.1.1.2	Test Requirements	2184

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2184

A.10.1.1.1.1.2.2	Random Access Response Reception	2184

A.10.1.1.1.1.2.3	No Random Access Response Reception	2185

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2185

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2185

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2186

A.10.1.1.1.2.1	Test Purpose and Environment	2186

A.10.1.1.1.2.2	Test Requirements	2189

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2189

A.10.1.1.1.2.2.2	Random Access Response Reception	2190

A.10.1.1.1.2.2.3	No Random Access Response Reception	2190

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2190

A.10.1.1.1.3.1	Test Purpose and Environment	2190

A.10.1.1.1.3.2	Test Requirements	2194

A.10.1.1.1.3.2.1	MsgA Transmission	2194

A.10.1.1.1.3.2.2	MsgB Reception	2194

A.10.1.1.1.3.2.3	No MsgB Reception	2195

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2195

A.10.1.1.1.4.1	Test Purpose and Environment	2195

A.10.1.1.1.4.2	Test Requirements	2199

A.10.1.1.1.4.2.1	MsgA Transmission	2199

A.10.1.1.1.4.2.2	MsgB Reception	2200

A.10.1.1.1.4.2.3	No MsgB Reception	2200

A.10.2	Timing	2201

A.10.2.1	UE transmit timing	2201

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2201

A.10.2.1.1.1	Test Purpose and environment	2201

A.10.2.1.1.2	Test requirements	2204

A.10.2.2	UE timing advance	2205

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2205

A.10.2.2.1.1	Test Purpose and Environment	2205

A.10.2.2.1.2	Test Parameters	2205

A.10.2.2.1.3	Test Requirements	2209

A.10.3	Signalling characteristics	2209

A.10.3.1	Radio link monitoring	2209

A.10.3.1.1	Introduction	2209

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2210

A.10.3.1.2.1	Test purpose and environment	2210

A.10.3.1.2.2	Test requirements	2214

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2214

A.10.3.1.3.1	Test purpose and environment	2214

A.10.3.1.3.2	Test requirements	2220

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2220

A.10.3.1.4.1	Test purpose and environment	2220

A.10.3.1.4.2	Test requirements	2220

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2220

A.10.3.1.5.1	Test purpose and environment	2220

A.10.3.1.5.2	Test requirements	2220

A.10.3.2	Interruption	2220

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2220

A.10.3.2.1.1	Test Purpose and Environment	2220

A.10.3.2.1.2	Test Requirements	2225

A.10.3.3	SCell activation and deactivation delay	2225

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2225

A.10.3.3.1.1	Test Purpose and Environment	2225

A.10.3.3.1.2	Test Requirements	2230

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2230

A.10.3.3.2.1	Test Purpose and Environment	2230

A.10.3.3.2.2	Test Requirements	2231

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2231

A.10.3.3.3.1	Test Purpose and Environment	2231

A.10.3.3.3.2	Test Requirements	2232

A.10.3.4	Beam failure detection and link recovery procedures	2232

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2232

A.10.3.4.1.1	Test Purpose and Environment	2232

A.10.3.4.1.2	Test Requirements	2238

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2238

A.10.3.4.2.1	Test Purpose and Environment	2238

A.10.3.4.2.2	Test Requirements	2245

A.10.3.5	Active BWP switching	2245

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2245

A.10.3.5.1.2	Test Requirements	2250

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2251

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2251

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2254

A.10.3.5.3	RRC-based Active BWP Switch	2258

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2258

A.10.3.6	PSCell addition and release delay	2262

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2262

A.10.3.6.1.1	Test purpose and environment	2262

A.10.3.6.1.2	Test Requirements	2266

A.10.3.7	Active TCI state switching delay	2267

A.10.4	Measurement procedure	2267

A.10.4.1	Intra-frequency measurements	2267

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2267

A.10.4.1.1.1	Test purpose and environment	2267

A.10.4.1.1.2	Test parameters	2267

A.10.4.1.1.3	Test Requirements	2267

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2268

A.10.4.1.2.1	Test purpose and environment	2268

A.10.4.1.2.2	Test parameters	2268

A.10.4.1.2.3	Test Requirements	2268

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2268

A.10.4.1.3.1	Test purpose and environment	2268

A.10.4.1.3.2	Test parameters	2268

A.10.4.1.3.3	Test Requirements	2268

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2269

A.10.4.1.4.1	Test purpose and environment	2269

A.10.4.1.4.2	Test parameters	2269

A.10.4.1.4.3	Test Requirements	2269

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2269

A.10.4.1.5.1	Test purpose and environment	2269

A.10.4.1.5.2	Test parameters	2269

A.10.4.1.5.3	Test Requirements	2269

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2270

A.10.4.1.6.1	Test purpose and environment	2270

A.10.4.1.6.2	Test parameters	2270

A.10.4.1.6.3	Test Requirements	2270

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2270

A.10.4.1.7.1	Test purpose and environment	2270

A.10.4.1.7.2	Test parameters	2270

A.10.4.1.7.3	Test Requirements	2270

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2271

A.10.4.1.8.1	Test purpose and environment	2271

A.10.4.1.8.2	Test parameters	2271

A.10.4.1.8.3	Test Requirements	2271

A.10.4.1.9	RSSI measurement reporting on PSCC	2271

A.10.4.1.9.1	Test purpose and environment	2271

A.10.4.1.9.2	Test parameters	2271

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2272

A.10.4.1.10.1	Test purpose and environment	2272

A.10.4.1.10.2	Test parameters	2272

A.10.4.1.11	RSSI measurement reporting on SCC	2272

A.10.4.1.11.1	Test purpose and environment	2272

A.10.4.1.11.2	Test parameters	2272

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2273

A.10.4.1.12.1	Test purpose and environment	2273

A.10.4.1.12.2	Test parameters	2273

A.10.4.2	Inter-frequency measurements	2273

A.10.4.2.1	RSSI measurement reporting	2273

A.10.4.2.1.1	Test purpose and environment	2273

A.10.4.2.1.2	Test parameters	2273

A.10.4.2.2	Channel occupancy measurement reporting	2274

A.10.4.2.2.1	Test purpose and environment	2274

A.10.4.2.2.2	Test parameters	2274

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2274

A.10.4.2.3.1	Test Purpose and Environment	2274

A.10.4.2.3.2	Test Requirements	2279

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2280

A.10.4.2.4.1	Test Purpose and Environment	2280

A.10.4.2.4.2	Test Requirements	2284

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2285

A.10.4.2.5.1	Test Purpose and Environment	2285

A.10.4.2.5.2	Test Requirements	2289

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2290

A.10.4.2.6.1	Test Purpose and Environment	2290

A.10.4.2.6.2	Test Requirements	2294

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2295

A.10.4.2.7.1	Test Purpose and Environment	2295

A.10.4.2.7.2	Test Requirements	2301

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2301

A.10.4.2.8.1	Test Purpose and Environment	2301

A.10.4.2.8.2	Test Requirements	2307

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2308

A.10.4.2.9.1	Test Purpose and Environment	2308

A.10.4.2.9.2	Test Requirements	2313

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2313

A.10.4.2.10.1	Test Purpose and Environment	2313

A.10.4.2.10.2	Test Requirements	2319

A.10.4.3	L1-RSRP measurements for beam reporting	2320

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2320

A.10.4.3.1.1	Test Purpose and Environment	2320

A.10.4.3.1.2	Test parameters	2320

A.10.4.3.1.3	Test Requirements	2322

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2323

A.10.4.3.2.1	Test Purpose and Environment	2323

A.10.4.3.2.2	Test parameters	2323

A.10.4.3.2.3	Test Requirements	2325

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2326

A.10.4.3.3.1	Test Purpose and Environment	2326

A.10.4.3.3.2	Test parameters	2326

A.10.4.3.3.3	Test Requirements	2329

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2330

A.10.4.3.4.1	Test Purpose and Environment	2330

A.10.4.3.4.2	Test parameters	2330

A.10.4.3.4.3	Test Requirements	2333

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2334

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2334

A.10.4.4.1.1	Test Purpose and Environment	2334

A.10.4.4.1.2	Test Requirements	2338

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2339

A.10.4.4.2.1	Test Purpose and Environment	2339

A.10.4.4.2.2	Test Requirements	2344

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2345

A.10.4.4.3.1	Test Purpose and Environment	2345

A.10.4.4.3.2	Test Requirements	2349

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2350

A.10.4.4.4.1	Test Purpose and Environment	2350

A.10.4.4.4.2	Test Requirements	2355

A.10.5	Measurement performance	2356

A.10.5.1	SS-RSRP	2356

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2356

A.10.5.1.1.1	Test Purpose and Environment	2356

A.10.5.1.1.2	Test parameters	2356

A.10.5.1.1.3	Test Requirements	2359

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2359

A.10.5.1.2.1	Test Purpose and Environment	2359

A.10.5.1.2.2	Test parameters	2359

A.10.5.1.2.3	Test Requirements	2362

A.10.5.2	SS-RSRQ	2362

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2362

A.10.5.2.1.1	Test Purpose and Environment	2362

A.10.5.2.1.2	Test Parameters	2362

A.10.5.2.1.3	Test Requirements	2365

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2365

A.10.5.2.2.1	Test Purpose and Environment	2365

A.10.5.2.2.2	Test Parameters	2365

A.10.5.2.2.3	Test Requirements	2368

A.10.5.3	SS-SINR	2368

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2368

A.10.5.4.1	SSB based L1-RSRP measurement	2368

A.10.5.4.1.1	Test Purpose and Environment	2368

A.10.5.4.1.2	Test parameters	2368

A.10.5.4.1.3	Test Requirements	2371

A.10.5.5	RSSI	2371

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2371

A.10.5.5.1.1	Test Purpose and Environment	2371

A.10.5.5.1.2	Test parameters	2371

A.10.5.5.1.3	Test Requirements	2374

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2374

A.10.5.5.2.1	Test Purpose and Environment	2374

A.10.5.5.2.2	Test parameters	2374

A.10.5.5.2.3	Test Requirements	2377

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2377

A.10.5.5.3.1	Test Purpose and Environment	2377

A.10.5.5.3.2	Test parameters	2377

A.10.5.5.3.3	Test Requirements	2380

A.10.5.6	Channel occupancy	2380

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2380

A.10.5.6.1.1	Test Purpose and Environment	2380

A.10.5.6.1.2	Test parameters	2380

A.10.5.6.1.3	Test Requirements	2384

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2384

A.10.5.6.2.1	Test Purpose and Environment	2384

A.10.5.6.2.2	Test parameters	2384

A.10.5.6.2.3	Test Requirements	2387

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2387

A.10.5.6.3.1	Test Purpose and Environment	2387

A.10.5.6.3.2	Test parameters	2387

A.10.5.6.3.3	Test Requirements	2391

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2391

A.11.1	RRC_IDLE state mobility	2391

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2391

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2391

A.11.1.1.1.1	Test Purpose and Environment	2391

A.11.1.1.1.2	Test Parameters	2391

A.11.1.1.1.3	Test Requirements	2394

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2394

A.11.1.1.2.1	Test Purpose and Environment	2394

A.11.1.1.2.2	Test Parameters	2394

A.11.1.1.2.3	Test Requirements	2398

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2398

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2398

A.11.1.2.1.1	Test Purpose and Environment	2398

A.11.1.2.1.2	Test Parameters	2399

A.11.1.2.1.3	Test Requirements	2405

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2406

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2406

A.11.1.3.1.1	Test Purpose and Environment	2406

A.11.1.3.1.2	Test Parameters	2406

A.11.1.3.1.3	Test Requirements	2412

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2413

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2413

A.11.1.4.1.1	Test Purpose and Environment	2413

A.11.1.4.1.2	Test Parameters	2413

A.11.1.4.1.3	Test Requirements	2416

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2417

A.11.1.4.2.1	Test Purpose and Environment	2417

A.11.1.4.2.2	Test Requirements	2420

A.11.2	RRC_CONNECTED state mobility	2421

A.11.2.1	Handover	2421

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2421

A.11.2.1.1.1	Test Purpose and Environment	2421

A.11.2.1.1.2	Test Parameters	2421

A.11.2.1.1.3 Test Requirements	2424

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2424

A.11.2.1.2.1	Test Purpose and Environment	2424

A.11.2.1.2.2	Test Parameters	2424

A.11.2.1.2.3	Test Requirements	2427

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2427

A.11.2.1.3.1	Test Purpose and Environment	2427

A.11.2.1.3.2	Test Parameters	2427

A.11.2.1.3.3 Test Requirements	2430

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2430

A.11.2.1.4.1	Test Purpose and Environment	2430

A.11.2.1.4.2	Test Parameters	2430

A.11.2.1.4.3 Test Requirements	2434

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2435

A.11.2.1.5.1	Test Purpose and Environment	2435

A.11.2.1.5.2	Test Parameters	2435

A.11.2.1.5.3 Test Requirements	2438

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2439

A.11.2.1.6.1	Test Purpose and Environment	2439

A.11.2.1.6.2	Test Parameters	2439

A.11.2.1.6.3 Test Requirements	2442

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2443

A.11.2.1.7.1	Test Purpose and Environment	2443

A.11.2.1.7.2	Test Requirements	2449

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2449

A.11.2.1.8.1	Test Purpose and Environment	2449

A.11.2.1.8.2	Test Requirements	2454

A.11.2.2	RRC connection mobility control	2454

A.11.2.2.1	RRC re-establishment	2454

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2454

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2457

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2464

A.11.2.2.2	Random Access	2469

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2469

A.11.2.2.2.1.1	Test Purpose and Environment	2469

A.11.2.2.2.1.2	Test Requirements	2472

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2472

A.11.2.2.2.1.2.2	Random Access Response Reception	2472

A.11.2.2.2.1.2.3	No Random Access Response Reception	2473

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2473

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2473

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2474

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2474

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2474

A.11.2.2.2.2.1	Test Purpose and Environment	2474

A.11.2.2.2.2.2	Test Requirements	2477

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2477

A.11.2.2.2.2.2.2	Random Access Response Reception	2478

A.11.2.2.2.2.2.3	No Random Access Response Reception	2478

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2478

A.11.2.2.2.3.1	Test Purpose and Environment	2478

A.11.2.2.2.3.2	Test Requirements	2482

A.11.2.2.2.3.2.1	MsgA Transmission	2482

A.11.2.2.2.3.2.2	MsgB Reception	2482

A.11.2.2.2.3.2.3	No MsgB Reception	2483

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2483

A.11.2.2.2.4.1	Test Purpose and Environment	2483

A.11.2.2.2.4.2	Test Requirements	2487

A.11.2.2.2.4.2.1	MsgA Transmission	2487

A.11.2.2.2.4.2.2	MsgB Reception	2488

A.11.2.2.2.4.2.3	No MsgB Reception	2488

A.11.2.2.3	RRC connection release with redirection	2489

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2489

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	2493

A.11.3	Timing	2498

A.11.3.1	UE transmit timing	2498

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	2498

A.11.3.1.1.1	Test Purpose and environment	2498

A.11.3.1.1.2	Test requirements	2501

A.11.3.2	UE timing advance	2502

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2502

A.11.3.2.1.1	Test Purpose and Environment	2502

A.11.3.2.1.2	Test Parameters	2502

A.11.3.2.1.3	Test Requirements	2506

A.11.4	Signalling characteristics	2506

A.11.4.1	Radio link monitoring	2506

A.11.4.1.1	Introduction	2506

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2507

A.11.4.1.2.1	Test purpose and environment	2507

A.11.4.1.2.2	Test requirements	2511

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2511

A.11.4.1.3.1	Test purpose and environment	2511

A.11.4.1.3.2	Test requirements	2517

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2517

A.11.4.1.4.1	Test purpose and environment	2517

A.11.4.1.4.2	Test requirements	2517

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2517

A.11.4.1.5.1	Test purpose and environment	2517

A.11.4.1.5.2	Test requirements	2517

A.11.4.2	Interruption	2518

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2518

A.11.4.2.1.1 Test Purpose and Environment	2518

A.11.4.2.1.2	Test Requirements	2522

A.11.4.3	SCell activation and deactivation delay	2522

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	2522

A.11.4.3.1.1	Test Purpose and Environment	2522

A.11.4.3.1.2	Test Requirements	2527

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	2527

A.11.4.3.2.1	Test Purpose and Environment	2527

A.11.4.3.2.2	Test Requirements	2528

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	2528

A.11.4.3.3.1	Test Purpose and Environment	2528

A.11.4.3.3.2	Test Requirements	2529

A.11.4.4	Beam failure detection and link recovery procedures	2529

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2529

A.11.4.4.1.1	Test Purpose and Environment	2529

A.11.4.4.1.2	Test Requirements	2535

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2535

A.11.4.4.2.1	Test Purpose and Environment	2535

A.11.4.4.2.2	Test Requirements	2542

A.11.4.5	Active BWP switching	2542

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2542

A.11.4.5.1.1	Test Purpose and Environment	2542

A.11.4.5.1.2	Test Requirements	2547

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	2548

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	2548

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	2551

A.11.4.5.3	RRC-based Active BWP Switch	2554

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2554

A.11.4.6	Active TCI state switching delay	2557

A.11.5	Measurement procedure	2557

A.11.5.1	Intra-frequency measurements	2557

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2557

A.11.5.1.1.1	Test purpose and environment	2557

A.11.5.1.1.2	Test parameters	2557

A.11.5.1.1.3	Test Requirements	2558

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2558

A.11.5.1.2.1	Test purpose and environment	2558

A.11.5.1.2.2	Test parameters	2558

A.11.5.1.2.3	Test Requirements	2558

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2559

A.11.5.1.3.1	Test purpose and environment	2559

A.11.5.1.3.2	Test parameters	2559

A.11.5.1.3.3	Test Requirements	2559

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2559

A.11.5.1.4.1	Test purpose and environment	2559

A.11.5.1.4.2	Test parameters	2559

A.11.5.1.4.3	Test Requirements	2559

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2560

A.11.5.1.5.1	Test purpose and environment	2560

A.11.5.1.5.2	Test parameters	2560

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2560

A.11.5.1.6.1	Test purpose and environment	2560

A.11.5.1.6.2	Test parameters	2560

A.11.5.1.6.3	Test Requirements	2560

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2561

A.11.5.1.7.1	Test purpose and environment	2561

A.11.5.1.7.2	Test parameters	2561

A.11.5.1.7.3	Test Requirements	2561

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2561

A.11.5.1.8.1	Test purpose and environment	2561

A.11.5.1.8.2	Test parameters	2561

A.11.5.1.8.3	Test Requirements	2561

A.11.5.1.9	RSSI measurement reporting on PCC	2562

A.11.5.1.9.1	Test purpose and environment	2562

A.11.5.1.9.2	Test parameters	2562

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2562

A.11.5.1.10.1	Test purpose and environment	2562

A.11.5.1.10.2	Test parameters	2562

A.11.5.1.11	RSSI measurement reporting on SCC	2562

A.11.5.1.11.1	Test purpose and environment	2562

A.11.5.1.11.2	Test parameters	2563

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2563

A.11.5.1.12.1	Test purpose and environment	2563

A.11.5.1.12.2	Test parameters	2563

A.11.5.2	Inter-frequency measurements	2563

A.11.5.2.1	RSSI measurement reporting	2563

A.11.5.2.1.1	Test purpose and environment	2563

A.11.5.2.1.2	Test parameters	2564

A.11.5.2.2	Channel occupancy measurement reporting	2564

A.11.5.2.2.1	Test purpose and environment	2564

A.11.5.2.2.2	Test parameters	2564

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2564

A.11.5.2.3.1	Test Purpose and Environment	2564

A.11.5.2.3.2	Test Requirements	2568

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2569

A.11.5.2.4.1	Test Purpose and Environment	2569

A.11.5.2.4.2	Test Requirements	2573

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2574

A.11.5.2.5.1	Test Purpose and Environment	2574

A.11.5.2.5.2	Test Requirements	2578

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2579

A.11.5.2.6.1	Test Purpose and Environment	2579

A.11.5.2.6.2	Test Requirements	2583

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2584

A.11.5.2.7.1	Test Purpose and Environment	2584

A.11.5.2.7.2	Test Requirements	2588

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2589

A.11.5.2.8.1	Test Purpose and Environment	2589

A.11.5.2.8.2	Test Requirements	2594

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2594

A.11.5.2.9.1	Test Purpose and Environment	2594

A.11.5.2.9.2	Test Requirements	2599

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2600

A.11.5.2.10.1	Test Purpose and Environment	2600

A.11.5.2.10.2	Test Requirements	2605

A.11.5.3	Inter-RAT E-UTRAN measurements	2606

A.11.5.4	L1-RSRP measurements for beam reporting	2606

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2606

A.11.5.4.1.1	Test Purpose and Environment	2606

A.11.5.4.1.2	Test parameters	2606

A.11.5.4.1.3	Test Requirements	2608

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2608

A.11.5.4.2.1	Test Purpose and Environment	2608

A.11.5.4.2.2	Test parameters	2609

A.11.5.4.2.3	Test Requirements	2611

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2611

A.11.5.4.3.1	Test Purpose and Environment	2611

A.11.5.4.3.2	Test parameters	2612

A.11.5.4.3.3	Test Requirements	2615

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2616

A.11.5.4.4.1	Test Purpose and Environment	2616

A.11.5.4.4.2	Test parameters	2616

A.11.5.4.4.3	Test Requirements	2619

A.11.6	Measurement performance	2620

A.11.6.1	SS-RSRP	2620

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2620

A.11.6.1.1.1	Test Purpose and Environment	2620

A.11.6.1.1.2	Test parameters	2620

A.11.6.1.1.3	Test Requirements	2622

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2622

A.11.6.1.2.1	Test Purpose and Environment	2622

A.11.6.1.2.2	Test parameters	2622

A.11.6.1.2.3	Test Requirements	2624

A.11.6.2	SS-RSRQ	2624

A.11.6.3	SS-SINR	2624

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2624

A.11.6.4.1	SSB based L1-RSRP measurement	2624

A.11.6.4.1.1	Test Purpose and Environment	2624

A.11.6.4.1.2	Test parameters	2624

A.11.6.4.1.3	Test Requirements	2627

A.11.6.5	RSSI	2627

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2627

A.11.6.5.1.1	Test Purpose and Environment	2627

A.11.6.5.1.2	Test parameters	2627

A.11.6.5.1.3	Test Requirements	2630

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2630

A.11.6.5.2.1	Test Purpose and Environment	2630

A.11.6.5.2.2	Test parameters	2630

A.11.6.5.2.3	Test Requirements	2633

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2633

A.11.6.5.3.1	Test Purpose and Environment	2633

A.11.6.5.3.2	Test parameters	2633

A.11.6.5.3.3	Test Requirements	2636

A.11.6.6	Channel occupancy	2636

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2636

A.11.6.6.1.1	Test Purpose and Environment	2636

A.11.6.6.1.2	Test parameters	2636

A.11.6.6.1.3	Test Requirements	2639

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2639

A.11.6.6.2.1	Test Purpose and Environment	2639

A.11.6.6.2.2	Test parameters	2639

A.11.6.6.2.3	Test Requirements	2642

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2642

A.11.6.6.3.1	Test Purpose and Environment	2642

A.11.6.6.3.2	Test parameters	2642

A.11.6.6.3.3	Test Requirements	2645

A.11.6.7	E-UTRAN RSRP	2645

A.11.6.8	E-UTRAN RSRQ	2645

A.11.6.9	E-UTRAN SINR	2645

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2645

A.12.1	RRC_IDLE state mobility	2645

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2645

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2645

A.12.1.1.1.1	Test Purpose and Environment	2645

A.12.1.1.1.2	Test Requirements	2650

A.12.2	RRC_CONNECTED state mobility	2651

A.12.2.1	Handover	2651

A.12.2.1.1	E-UTRAN - NR with CCA handover	2651

A.12.2.1.1.1	Test Purpose and Environment	2651

A.12.2.1.1.2	Test Requirements	2656

A.12.3	Signalling characteristics	2657

A.12.3.1	Interruptions	2657

A.12.4	Measurement procedure	2657

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2657

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2657

A.12.4.1.1.1	Test Purpose and Environment	2657

A.12.4.1.1.2	Test Requirements	2661

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2661

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2661

A.12.4.2.1.1	Test Purpose and Environment	2661

A.12.4.2.1.2	Test Requirements	2665

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2666

A.12.4.2.2.1	Test Purpose and Environment	2666

A.12.4.2.2.2	Test Requirements	2670

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2670

A.12.4.2.3.1	Test Purpose and Environment	2670

A.12.4.2.3.2	Test Requirements	2674

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2675

A.12.4.2.4.1	Test Purpose and Environment	2675

A.12.4.2.4.2	Test Requirements	2679

A.12.4.2.5	RSSI measurement reporting	2679

A.12.4.2.5.1	Test purpose and environment	2679

A.12.4.2.5.2	Test parameters	2679

A.12.4.2.6	Channel occupancy measurement reporting	2680

A.12.4.2.6.1	Test purpose and environment	2680

A.12.4.2.6.2	Test parameters	2680

A.12.5	Measurement performance	2680

A.12.5.1	E-UTRANNR SFTD	2680

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	2680

A.12.5.1.1.1	Test Purpose	2680

A.12.5.1.1.2	Test Environment	2681

A.12.5.1.1.3	Test Requirements	2686

A.12.5.2	E-UTRANNR SS-RSRP	2686

A.12.5.3	E-UTRANNR SS-RSRQ	2686

A.12.5.4	E-UTRANNR SS-SINR	2686

A.12.5.5	E-UTRANNR RSSI	2686

A.12.5.6	E-UTRANNR channel occupancy	2686

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2686

A.13.1	Timing	2686

A.13.1.1	UE transmit timing	2686

A.13.1.2	Timing advance	2686

A.13.2	Signalling characteristics	2686

A.13.2.1	Interruption	2686

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2686

A.13.2.1.1.1	Test Purpose and Environment	2686

A.13.2.1.1.2	Test Requirements	2690

A.13.2.2	SCell activation and deactivation delay	2690

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	2690

A.13.2.2.1.1	Test Purpose and Environment	2690

A.13.2.2.1.2	Test Requirements	2695

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	2695

A.13.2.2.2.1	Test Purpose and Environment	2695

A.13.2.2.2.2	Test Requirements	2696

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2696

A.13.2.2.3.1	Test Purpose and Environment	2696

A.13.2.2.3.2	Test Requirements	2697

A.13.2.3	Active TCI state switching delay	2697

A.13.3	Measurement procedure	2697

A.13.3.1	Intra-frequency measurements	2697

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2697

A.13.3.1.1.1	Test purpose and environment	2697

A.13.3.1.1.2	Test parameters	2697

A.13.3.1.1.3	Test Requirements	2702

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2702

A.13.3.1.2.1	Test purpose and environment	2702

A.13.3.1.2.2	Test parameters	2702

A.13.3.1.2.3	Test Requirements	2706

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2706

A.13.3.1.3.1	Test purpose and environment	2706

A.13.3.1.3.2	Test parameters	2706

A.13.3.1.3.3	Test Requirements	2711

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2711

A.13.3.1.4.1	Test purpose and environment	2711

A.13.3.1.4.2	Test parameters	2711

A.13.3.1.4.3	Test Requirements	2716

A.13.3.1.5	RSSI measurement reporting on SCC	2716

A.13.3.1.5.1	Test purpose and environment	2716

A.13.3.1.5.2	Test parameters	2716

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2717

A.13.3.1.6.1	Test purpose and environment	2717

A.13.3.1.6.2	Test parameters	2718

A.13.3.2	Inter-frequency measurements	2718

A.13.3.2.1	RSSI measurement reporting	2718

A.13.3.2.1.1	Test purpose and environment	2718

A.13.3.2.1.2	Test parameters	2718

A.13.3.2.2	Channel occupancy measurement reporting	2719

A.13.3.2.2.1	Test purpose and environment	2719

A.13.3.2.2.2	Test parameters	2719

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2719

A.13.3.2.3.1	Test Purpose and Environment	2719

A.13.3.2.3.2	Test Requirements	2724

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2724

A.13.3.2.4.1	Test Purpose and Environment	2724

A.13.3.2.4.2	Test Requirements	2729

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2730

A.13.3.2.5.1	Test Purpose and Environment	2730

A.13.3.2.5.2	Test Requirements	2735

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2735

A.13.3.2.6.1	Test Purpose and Environment	2735

A.13.3.2.6.2	Test Requirements	2741

A.13.3.3	L1-RSRP measurements for beam reporting	2742

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2742

A.13.3.3.1.1	Test Purpose and Environment	2742

A.13.3.3.1.2	Test parameters	2742

A.13.3.3.1.3	Test Requirements	2746

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2746

A.13.3.3.2.1	Test Purpose and Environment	2746

A.13.3.3.2.2	Test parameters	2746

A.13.3.3.2.3	Test Requirements	2750

A.13.4	Measurement performance	2750

A.13.4.1	SS-RSRP	2750

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2750

A.13.4.1.1.1	Test Purpose and Environment	2750

A.13.4.1.1.2	Test parameters	2750

A.13.4.1.1.3	Test Requirements	2752

A.13.4.2	SS-RSRQ	2752

A.13.4.3	SS-SINR	2752

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2752

A.13.4.4.1	SSB based L1-RSRP measurement	2752

A.13.4.4.1.1	Test Purpose and Environment	2752

A.13.4.4.1.2	Test parameters	2752

A.13.4.4.1.3	Test Requirements	2757

A.13.4.5	RSSI	2757

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2757

A.13.4.5.1.1	Test Purpose and Environment	2757

A.13.4.5.1.2	Test parameters	2757

A.13.4.5.1.3	Test Requirements	2760

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2760

A.13.4.5.2.1	Test Purpose and Environment	2760

A.13.4.5.2.2	Test parameters	2760

A.13.4.5.2.3	Test Requirements	2764

A.13.4.6	Channel occupancy	2764

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2764

A.13.4.6.1.1	Test Purpose and Environment	2764

A.13.4.6.1.2	Test parameters	2764

A.13.4.6.1.3	Test Requirements	2768

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2768

A.13.4.6.2.1	Test Purpose and Environment	2768

A.13.4.6.2.2	Test parameters	2768

A.13.4.6.2.3	Test Requirements	2772

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1787

B.1	Conditions for NR RRC_IDLE state mobility	1787

B.1.1	Introduction	1787

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1787

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1788

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1788

B.2.1	Introduction	1788

B.2.1.1	General	1788

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1789

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1789

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1789

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1790

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1790

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1790

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1790

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1791

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1791

B.2.2	Conditions for NR intra-frequency measurements	1792

B.2.3	Conditions for NR inter-frequency measurements	1793

B.2.4	Conditions for NR L1-RSRP reporting	1794

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1794

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1795

B.2.5	Conditions for RRC connection release with redirection to NR	1796

B.2.6	Void	1798

B.2.6.1	Void	1798

B.2.6.2	Void	1798

B.2.8	Conditions for NR L1-SINR reporting	1799

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1799

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1800

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1800

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1800

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1801

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1801

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1802

B.2.9	Conditions for NR intra-frequency measurements under CCA	1804

B.2.10	Conditions for NR inter-frequency measurements under CCA	1804

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1804

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1804

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1805

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1806

B.2.14	Conditions for NR PRS-based measurements	1808

B.3	RRM Requirements Exceptions	1809

B.3.1	Introduction	1809

B.3.2	Receiver sensitivity relaxation for CA	1809

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1809

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1810

B.3.2.2.1	Inter-band carrier aggregation	1810

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1810

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1810

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1810

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1810

B.3.2.4.1	Intra-band contiguous carrier aggregation	1810

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1810

B.3.3	Receiver sensitivity relaxation for DC	1811

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1811

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1811

B.3.4	Receiver sensitivity relaxation for SUL	1811

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1811

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1811

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1811

B.4	Conditions for V2X	1811

B.4.1	Test parameters for GNSS signals	1811

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1812

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1812

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1812

Annex C (informative): Change history	1814