3GPP TS 38.533 V17.1.0 (2021-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

User Equipment (UE) conformance specification;

Radio Resource Management (RRM)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NR, RRM, terminal, testing

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	26

1	Scope	27

2	References	27

3	Definitions, symbols and abbreviations	28

3.1	Definitions	28

3.2	Symbols	29

3.3	Abbreviations	30

3A	Requirements for the support of RRM	32

3A.1	General	32

3A.1.1	Test coverage across 5G NR connectivityoptions	32

3A.2	Requirements Classification for Statistical Testing	33

3A.3	Antenna Configuration	33

3A.4	NR band groups	33

3A.4.1	NR operating bands in FR1	33

3A.4.2	NR operating bands in FR2	34

3A.5	NR operating band configuration	35

4	EN-DC with all NR cells in FR1	36

4.1	Void	36

4.2	Void	36

4.3	RRC_CONNECTED state mobility	36

4.3.1	Void	36

4.3.2	RRC connection mobility control	36

4.3.2.1	Void	36

4.3.2.2	Random access	36

4.3.2.2.1	EN-DC FR1 contention based random access	36

4.3.2.2.2	EN-DC FR1 non-contention based random access	42

4.3.2.2.3	EN-DC FR1 2-step contention based random access	49

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	53

4.3.2.3	Void	56

4.4	Timing	56

4.4.1	UE transmit timing	56

4.4.1.0	Minimum conformance requirements	56

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	56

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	58

4.4.2	UE timer accuracy	64

4.4.3	Timing advance	64

4.4.3.0	Minimum conformance requirements	64

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	64

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	64

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	65

4.5	Signaling characteristics	70

4.5.1	Radio link monitoring	70

4.5.1.0	Minimum conformance requirements	70

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	70

4.5.1.0.2	Void	71

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	71

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	72

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	74

4.5.1.1.1	Test purpose	74

4.5.1.1.2	Test applicability	74

4.5.1.1.3	Minimum conformance requirement	74

4.5.1.1.4	Test description	74

4.5.1.1.5	Test Requirement	78

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	79

4.5.1.2.1	Test purpose	79

4.5.1.2.2	Test applicability	79

4.5.1.2.3	Minimum conformance requirements	79

4.5.1.2.4	Test description	79

4.5.1.2.5	Test Requirement	83

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	84

4.5.1.3.1	Test purpose	84

4.5.1.3.2	Test applicability	84

4.5.1.3.3	Minimum conformance requirement	84

4.5.1.3.4	Test description	84

4.5.1.3.5	Test Requirement	88

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	89

4.5.1.4.1	Test purpose	89

4.5.1.4.2	Test applicability	89

4.5.1.4.3	Minimum conformance requirements	89

4.5.1.4.4	Test Description	89

4.5.1.4.5	Test Requirement	93

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	94

4.5.1.5.1	Test purpose	94

4.5.1.5.2	Test applicability	94

4.5.1.5.3	Minimum conformance requirements	94

4.5.1.5.4	Test description	94

4.5.1.5.5	Test requirement	98

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	99

4.5.1.6.1	Test purpose	99

4.5.1.6.2	Test applicability	99

4.5.1.6.3	Minimum conformance requirements	99

4.5.1.6.4	Test description	99

4.5.1.6.5	Test requirement	103

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	104

4.5.1.7.1	Test purpose	104

4.5.1.7.2	Test applicability	104

4.5.1.7.3	Minimum conformance requirements	104

4.5.1.7.4	Test description	104

4.5.1.7.5	Test requirement	108

4.5.1.8.1	Test purpose	109

4.5.1.8.2	Test applicability	109

4.5.1.8.3	Minimum conformance requirements	109

4.5.1.8.4	Test description	109

4.5.1.8.5	Test requirement	114

4.5.2	Interruption	115

4.5.2.0	Minimum conformance requirements	115

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	115

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	115

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	116

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	117

4.5.2.1.1	Test purpose	117

4.5.2.1.2	Test applicability	117

4.5.2.1.3	Minimum conformance requirements	117

4.5.2.1.4	Test description	117

4.5.2.1.5	Test requirement	119

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	120

4.5.2.2.1	Test purpose	120

4.5.2.2.2	Test applicability	121

4.5.2.2.3	Minimum conformance requirements	121

4.5.2.2.4	Test description	121

4.5.2.2.5	Test requirement	123

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	124

4.5.2.3.1	Test purpose	125

4.5.2.3.2	Test applicability	125

4.5.2.3.3	Minimum conformance requirements	125

4.5.2.3.4	Test description	125

4.5.2.3.5	Test requirement	127

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	130

4.5.2.4.1	Test purpose	130

4.5.2.4.2	Test applicability	130

4.5.2.4.3	Minimum conformance requirements	130

4.5.2.4.4	Test description	130

4.5.2.4.5	Test requirement	133

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	135

4.5.2.5.1	Test purpose	135

4.5.2.5.2	Test applicability	135

4.5.2.5.3	Minimum conformance requirements	135

4.5.2.5.4	Test description	135

4.5.2.5.5	Test requirement	138

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	139

4.5.2.6.1	Test purpose	139

4.5.2.6.2	Test applicability	139

4.5.2.6.3	Minimum conformance requirements	139

4.5.2.6.4	Test description	139

4.5.2.6.5	Test requirement	142

4.5.3	SCell activation and deactivation delay	144

4.5.3.0	Minimum conformance requirements	144

4.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	144

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	148

4.5.3.1.1	Test purpose	148

4.5.3.1.2	Test applicability	148

4.5.3.1.3	Minimum conformance requirements	148

4.5.3.1.4	Test description	148

4.5.3.1.5	Test requirement	153

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle	156

4.5.3.2.1	Test purpose	156

4.5.3.2.2	Test applicability	156

4.5.3.2.3	Minimum conformance requirements	157

4.5.3.2.4	Test description	157

4.5.3.2.5	Test requirement	157

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	158

4.5.3.3.1	Test purpose	158

4.5.3.3.2	Test applicability	158

4.5.3.3.3	Minimum conformance requirements	158

4.5.3.3.4	Test description	158

4.5.3.3.5	Test requirement	160

4.5.4	UE UL carrier RRC reconfiguration delay	160

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	160

4.5.4.1.1	Test purpose	160

4.5.4.1.2	Test applicability	160

4.5.4.1.3	Minimum conformance requirements	160

4.5.4.1.4	Test description	160

4.5.4.1.5	Test requirements	163

4.5.5	Link recovery procedures	167

4.5.5.0	Minimum conformance requirements	168

4.5.5.0.1	Minimum conformance requirements for SSB-based beam failure detection	168

4.5.5.0.2	Minimum conformance requirements for CSI-RS-based beam failure detection	168

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	169

4.5.5.1.1	Test purpose	169

4.5.5.1.2	Test applicability	170

4.5.5.1.3	Minimum conformance requirements	170

4.5.5.1.4	Test description	170

4.5.5.1.5	Test requirements	175

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	176

4.5.5.2.1	Test purpose	176

4.5.5.2.2	Test applicability	176

4.5.5.2.3	Minimum conformance requirements	176

4.5.5.2.4	Test description	176

4.5.5.2.5	Test requirements	181

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	182

4.5.5.3.1	Test purpose	183

4.5.5.3.2	Test applicability	183

4.5.5.3.3	Minimum conformance requirements	183

4.5.5.3.4	Test description	183

4.5.5.3.5	Test requirements	188

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	189

4.5.5.4.1	Test purpose	189

4.5.5.4.2	Test applicability	189

4.5.5.4.3	Minimum conformance requirements	189

4.5.5.4.4	Test description	189

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	192

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	197

4.5.6	Active BWP switch delay	205

4.5.6.1	DCI-based and time-based active BWP switch	205

4.5.6.1.0	Minimum conformance requirements	205

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	207

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	214

4.5.6.2	RRC-based active BWP switch	222

4.5.6.2.0	Minimum conformance requirements	222

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	222

4.5.7	PSCell addition and release delay	228

4.5.7.0	Minimum conformance requirements	228

4.5.7.0.1	NR PSCell Addition Delay Requirement	228

4.5.7.0.2	NR PSCell Release Delay Requirement	229

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	229

4.5.7.1.1	Test purpose	229

4.5.7.1.2	Test applicability	229

4.5.7.1.3	Minimum conformance requirements	229

4.5.7.1.4	Test description	229

4.5.8	UL switching	234

4.5.8.0	Minimum conformance requirements	234

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	235

4.6	Measurement procedures	245

4.6.1	Intra-frequency measurements	245

4.6.1.0	Minimum conformance requirements	245

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	245

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	249

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	252

4.6.1.1.1	Test purpose	252

4.6.1.1.2	Test applicability	252

4.6.1.1.3	Minimum conformance requirements	252

4.6.1.1.4	Test description	252

4.6.1.1.5	Test requirement	254

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	256

4.6.1.2.1	Test purpose	256

4.6.1.2.2	Test applicability	256

4.6.1.2.3	Minimum conformance requirements	256

4.6.1.2.4	Test description	256

4.6.1.2.5	Test requirement	258

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	259

4.6.1.3.1	Test purpose	260

4.6.1.3.2	Test applicability	260

4.6.1.3.3	Minimum conformance requirements	260

4.6.1.3.4	Test description	260

4.6.1.3.5	Test requirement	263

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	265

4.6.1.4.1	Test purpose	265

4.6.1.4.2	Test applicability	265

4.6.1.4.3	Minimum conformance requirements	265

4.6.1.4.4	Test description	265

4.6.1.4.5	Test requirement	268

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	270

4.6.1.5.1	Test purpose	270

4.6.1.5.2	Test applicability	270

4.6.1.5.3	Minimum conformance requirements	270

4.6.1.5.4	Test description	270

4.6.1.5.5	Test requirement	272

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	274

4.6.1.6.1	Test purpose	274

4.6.1.6.2	Test applicability	274

4.6.1.6.3	Minimum conformance requirements	274

4.6.1.6.4	Test description	274

4.6.1.6.5	Test requirement	277

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	278

4.6.1.7.1	Test purpose	278

4.6.1.7.2	Test applicability	278

4.6.1.7.3	Minimum conformance requirements	278

4.6.1.7.4	Test description	279

4.6.1.7.5	Test requirement	281

4.6.2	Inter-frequency measurements	283

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	283

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	285

4.6.2.1.1	Test purpose	285

4.6.2.1.2	Test applicability	285

4.6.2.1.3	Minimum conformance requirements	285

4.6.2.1.4	Test description	285

4.6.2.1.5	Test requirement	288

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	290

4.6.2.2.1	Test purpose	290

4.6.2.2.2	Test applicability	290

4.6.2.2.3	Minimum conformance requirements	290

4.6.2.2.4	Test description	290

4.6.2.2.5	Test requirement	293

4.6.2.3	Void	295

4.6.2.4	Void	295

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	295

4.6.2.5.1	Test purpose	295

4.6.2.5.2	Test applicability	295

4.6.2.5.3	Minimum conformance requirements	296

4.6.2.5.4	Test description	296

4.6.2.5.5	Test requirement	299

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	300

4.6.2.6.1	Test purpose	300

4.6.2.6.2	Test applicability	300

4.6.2.6.3	Minimum conformance requirements	301

4.6.2.6.4	Test description	301

4.6.2.6.5	Test requirement	304

4.6.2.7	Void	306

4.6.2.8	Void	306

4.6.3	Void	306

4.6.4	L1-RSRP measurement for beam reporting	306

4.6.4.0	Minimum conformance requirements	306

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	306

4.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	307

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	309

4.6.4.1.1	Test purpose	309

4.6.4.1.2	Test applicability	309

4.6.4.1.3	Minimum conformance requirements	309

4.6.4.1.4	Test description	310

4.6.4.1.5	Test requirement	312

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	313

4.6.4.2.1	Test purpose	313

4.6.4.2.2	Test applicability	313

4.6.4.2.3	Minimum conformance requirements	314

4.6.4.2.4	Test description	314

4.6.4.2.5	Test requirement	316

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	317

4.6.4.3.1	Test purpose	317

4.6.4.3.2	Test applicability	317

4.6.4.3.3	Minimum conformance requirements	317

4.6.4.3.4	Test description	317

4.6.4.3.5	Test requirement	320

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	321

4.6.4.4.1	Test purpose	321

4.6.4.4.2	Test applicability	321

4.6.4.4.3	Minimum conformance requirements	321

4.6.4.4.4	Test description	321

4.6.4.4.5	Test requirement	323

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	324

4.6.4.5.1	Test purpose	324

4.6.4.5.2	Test applicability	325

4.6.4.5.3	Minimum conformance requirements	325

4.6.4.5.4	Test description	325

4.6.4.5.5	Test requirement	327

4.6.5	CLI measurements	328

4.6.5.0	Minimum conformance requirements	328

4.6.5.0.1	Minimum conformance requirements for SRS-RSRP measurement	328

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	329

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	330

4.6.5.1.1	Test purpose	330

4.6.5.1.2	Test applicability	330

4.6.5.1.3	Minimum conformance requirements	330

4.6.5.1.4	Test description	330

4.6.5.1.5	Test requirement	331

4.6.5.2	EN-DC FR1 CLI-RSSI measurement with non-DRX	333

4.6.5.2.1	Test purpose	333

4.6.5.2.2	Test applicability	333

4.6.5.2.3	Minimum conformance requirements	334

4.6.5.2.4	Test description	334

4.6.5.2.5	Test requirement	335

4.6.6	336

4.6.7	L1-SINR measurement for beam reporting	336

4.6.7.0	Minimum conformance requirements	336

4.6.7.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	336

4.6.7.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	337

4.6.7.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	339

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	341

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	353

4.7	Measurement performance requirements	358

4.7.1	SS-RSRP	358

4.7.1.0	Minimum conformance requirements	358

4.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	358

4.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	360

4.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	361

4.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	362

4.7.1.1	Intra-frequency measurements	363

4.7.1.1.1	EN-DC FR1 SS-RSRP absolute measurement accuracy	363

4.7.1.1.2	EN-DC FR1 SS-RSRP relative measurement accuracy	369

4.7.1.2	Inter-frequency measurements	372

4.7.1.2.1	EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy	372

4.7.1.2.2	EN-DC FR1-FR1 SS-RSRP relative measurement accuracy	377

4.7.2	SS-RSRQ	379

4.7.2.0	Minimum conformance requirements	379

4.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	379

4.7.2.0.2	Inter-frequency absolute SS-RSRQ measurement accuracy requirements	380

4.7.2.0.3	Inter-frequency relative SS-RSRQ measurement accuracy requirements	380

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	381

4.7.2.1.1	Test purpose	381

4.7.2.1.2	Test applicability	381

4.7.2.1.3	Minimum conformance requirements	381

4.7.2.1.4	Test description	381

4.7.2.1.5	Test requirement	383

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	385

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	385

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	389

4.7.3	SS-SINR	391

4.7.3.0	Minimum conformance requirements	391

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	391

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	392

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	393

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	394

4.7.3.1.1	Test purpose	394

4.7.3.1.2	Test applicability	394

4.7.3.1.3	Minimum conformance requirements	394

4.7.3.1.4	Test description	394

4.7.3.1.5	Test requirements	396

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	398

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	398

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	401

4.7.4	L1-RSRP	403

4.7.4.0	Minimum conformance requirements	403

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	403

4.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	403

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	404

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	404

4.7.4.1	SSB based L1-RSRP measurements	404

4.7.4.1.1	EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy	404

4.7.4.1.2	EN-DC FR1 SSB based L1-RSRP relative measurement accuracy	409

4.7.4.2	CSI-RS based L1-RSRP measurements	411

4.7.4.2.1	EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy	411

4.7.4.2.2	EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy	415

4.7.5	SFTD	416

4.7.5.0	Minimum conformance requirements	416

4.7.5.0.1	SFTD Accuracy Requirement	417

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	418

4.7.5.1.1	Test purpose	418

4.7.5.1.2	Test applicability	418

4.7.5.1.3	Minimum conformance requirements	418

4.7.5.1.4	Test description	418

4.7.6	CLI measurements	422

4.7.6.0	Minimum conformance requirements	422

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	422

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	424

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	425

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	430

4.7.7	L1-SINR measurement for beam reporting	434

4.7.7.0	Minimum conformance requirements	434

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	434

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	437

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	439

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	443

4.7.7.1	448

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	448

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	453

5	EN-DC with at least one NR cell in FR2	460

5.1	Void	460

5.2	Void	460

5.3	RRC_CONNECTED state mobility	460

5.3.1	Void	460

5.3.2	RRC connection mobility control	460

5.3.2.1	Void	460

5.3.2.2	Random access	460

5.3.2.2.1	EN-DC FR2 contention based random access	460

5.3.2.2.2	EN-DC FR2 non-contention based random access	468

5.3.2.2.3	EN-DC FR2 2-step contention based random access	476

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	480

5.3.2.3	Void	484

5.4	Timing	484

5.4.1	UE transmit timing	484

5.4.1.0	Minimum Conformance Requirements	484

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	484

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	486

5.4.1.1.1	Test purpose	486

5.4.1.1.2	Test applicability	486

5.4.1.1.3	Minimum conformance requirements	486

5.4.1.1.4	Test Description	486

5.4.1.1.5	Test Requirements	489

5.4.2	UE timer accuracy	492

5.4.3	Timing advance	492

5.4.3.0	Minimum conformance requirements	492

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	492

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	492

5.4.3.1.1	Test purpose	492

5.4.3.1.2	Test applicability	492

5.4.3.1.3	Minimum conformance requirement	492

5.4.3.1.4	Test description	492

5.5	Signaling characteristics	497

5.5.1	Radio link monitoring	497

5.5.1.0	Minimum conformance requirements	497

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	497

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	498

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	499

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	502

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	505

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	506

5.5.1.2	EN-DC FR2 Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	512

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	518

5.5.1.4	EN-DC FR2 Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	524

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	529

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	534

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	539

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	550

5.5.2	Interruption	555

5.5.2.0	Minimum conformance requirements	555

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	555

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	555

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	556

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	557

5.5.2.1.2	Test applicability	557

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	561

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	565

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	570

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	574

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	579

5.5.3	SCell activation and deactivation delay	583

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	583

5.5.4	UE UL carrier RRC reconfiguration delay	587

5.5.5	Link recovery procedures	587

5.5.5.0	Minimum conformance requirements	587

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	587

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	591

5.5.5.0.3	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	594

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	595

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	600

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	605

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	610

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	615

5.5.6	Active BWP switch delay	620

5.5.6.1	DCI-based and time-based active BWP switch	620

5.5.6.1.0	Minimum conformance requirements	620

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	620

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	621

5.5.6.2	RRC-based active BWP switch	622

5.5.6.2.0	Minimum conformance requirements	622

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	622

5.5.7	Void	626

5.5.8	Active TCI state switch delay	626

5.5.8.0	Minimum conformance requirements	626

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	626

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	627

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	628

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	632

5.6	Measurement procedures	636

5.6.1	Intra-frequency measurements	636

5.6.1.0	Minimum conformance requirements	636

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	636

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	638

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	640

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	644

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	649

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	653

5.6.2	Inter-frequency measurements	657

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	657

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	659

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	663

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	668

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	673

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	677

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	682

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	688

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	693

5.6.3	L1-RSRP measurement for beam reporting	699

5.6.3.0	Minimum conformance requirements	699

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	699

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	700

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	703

5.6.3.1.1	Test purpose	703

5.6.3.1.2	Test applicability	703

5.6.3.1.3	Minimum conformance requirements	703

5.6.3.1.4	Test description	703

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	707

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	711

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	715

5.7	Measurement performance requirements	718

5.7.1	SS-RSRP	718

5.7.1.0	Minimum conformance requirements	718

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	718

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	719

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	721

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	727

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	734

5.7.2	SS-RSRQ	735

5.7.2.0	Minimum conformance requirements	735

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	735

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	736

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	737

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	741

5.7.3	SS-SINR	746

5.7.3.0	Minimum conformance requirements	746

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	746

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	747

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	748

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	752

6	NR standalone in FR1	759

6.1	RRC_IDLE state mobility	759

6.1.1	NR cell re-selection	759

6.1.1.0	Minimum conformance requirements	759

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	759

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	759

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	760

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	761

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	763

6.1.1.1	NR SA FR1 cell re-selection	765

6.1.1.2	NR SA FR1-FR1 cell re-selection	771

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	775

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	782

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	788

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	794

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	800

6.1.2	NR – E-UTRA cell re-selection	805

6.1.2.0	Minimum conformance requirements	805

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	805

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	807

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	809

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	814

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	820

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	826

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	831

6.2	RRC_INACTIVE state mobility	837

6.3	RRC_CONNECTED state mobility	837

6.3.1	Handover	837

6.3.1.0	Minimum conformance requirements	837

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	837

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	837

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	838

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	839

6.3.1.1	NR SA FR1 handover with known target cell	841

6.3.1.2	NR SA FR1 handover with unknown target cell	845

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	848

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	851

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	857

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	862

6.3.1.7	NR SA FR1 synchronous DAPS handover	868

6.3.1.8	NR SA FR1 asynchronous DAPS handover	876

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	881

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	889

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	896

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	902

6.3.2	RRC connection mobility control	908

6.3.2.1	RRC re-establishment	908

6.3.2.1.0	Minimum conformance requirements	909

6.3.2.1.1	NR SA FR1 RRC re-establishment	910

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	914

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	918

6.3.2.2	Random access	922

6.3.2.2.0	Minimum conformance requirements	922

6.3.2.2.1	NR SA FR1 contention based random access	924

6.3.2.2.2	NR SA FR1 non-contention based random access	930

6.3.2.2.3	NR SA FR2 2-step contention based random access	937

6.3.2.2.4	NR SA FR2 2-step non-contention based random access	939

6.3.2.3	RRC connection release with redirection	942

6.3.2.3.0	Minimum conformance requirements	942

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	943

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	947

6.3.3	Conditional handover	951

6.3.3.0	Minimum conformance requirements	951

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	951

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	955

6.3.3.1	NR SA FR1 conditional handover	958

6.3.3.2	NR SA FR1-FR1 conditional handover	964

6.4	Timing	974

6.4.1	UE transmit timing	974

6.4.1.0	Minimum conformance requirements	974

6.4.1.0	Minimum conformance requirements	974

6.4.1.1	NR SA FR1 UE transmit timing accuracy	975

6.4.2	UE timer accuracy	981

6.4.3	Timing advance	981

6.4.3.0	Minimum conformance requirement	981

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	981

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	981

6.5	Signaling characteristics	986

6.5.1	Radio link monitoring	986

6.5.1.0	Minimum conformance requirements	987

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	987

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	989

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	990

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	991

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	996

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1003

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1008

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1015

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1022

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1028

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1035

6.5.2	Interruption	1041

6.5.2.0	Minimum conformance requirements	1041

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1041

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1042

6.5.3	SCell activation and deactivation delay	1048

6.5.3.0	Minimum conformance requirements	1048

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1048

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1049

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle	1058

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1059

6.5.4	UE UL carrier RRC reconfiguration delay	1061

6.5.4.0	Minimum conformance requirements	1061

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1061

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1062

6.5.5	Link recovery procedures	1070

6.5.5.0	Minimum conformance requirements	1070

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1070

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1071

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1073

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1081

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1088

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1096

6.5.6	Active BWP switch delay	1104

6.5.6.1	DCI-based and time-based active BWP switch	1104

6.5.6.1.0	Minimum conformance requirements	1104

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1105

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1115

6.5.6.2	RRC-based active BWP switch	1124

6.5.6.2.0	Minimum conformance requirements	1124

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1124

6.6	Measurement procedures	1133

6.6.1	Intra-frequency measurements	1133

6.6.1.0	Minimum conformance requirements	1133

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1133

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1134

6.6.1.0.3	Minimum conformance requirements for event-triggered measurement without gap with SSB index reading	1135

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1137

6.6.1.0.5	Minimum conformance requirements for event-triggered measurement without gap for UE configured with highSpeedMeasFlag-r16	1138

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1140

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1144

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1149

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1155

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1160

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1164

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1168

6.6.2	Inter-frequency measurements	1173

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1173

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1173

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1179

6.6.2.3	Void	1185

6.6.2.4	Void	1185

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1185

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1192

6.6.2.7	Void	1198

6.6.2.8	Void	1198

6.6.3	Inter-RAT Measurements	1198

6.6.3.0	Minimum conformance requirements	1198

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	1198

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	1201

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	1203

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	1211

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	1217

6.6.4	L1-RSRP measurement for beam reporting	1222

6.6.4.0	Minimum conformance requirements	1222

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1222

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1222

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	1222

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	1226

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	1230

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	1234

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	1237

6.6.5	UTRAN inter-RAT measurement	1243

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	1243

6.6.6 to 6.6.7	1250

6.6.8	L1-SINR measurement for beam reporting	1250

6.6.8.0	Minimum conformance requirements	1250

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1250

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1250

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1250

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1250

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1255

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1263

6.7	Measurement performance requirements	1268

6.7.1	SS-RSRP	1268

6.7.1.0	Minimum conformance requirements	1268

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	1268

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	1268

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	1268

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	1268

6.7.1.1	Intra-frequency measurements	1268

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	1268

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	1275

6.7.1.2	Inter-frequency measurements	1277

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	1277

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	1284

6.7.2	SS-RSRQ	1286

6.7.2.0	Minimum conformance requirements	1286

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1286

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	1286

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	1286

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	1286

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	1290

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	1290

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	1295

6.7.3	SS-SINR	1297

6.7.3.0	Minimum conformance requirements	1297

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1297

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	1297

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	1297

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	1297

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	1301

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	1301

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	1305

6.7.4	L1-RSRP	1307

6.7.4.0	Minimum conformance requirements	1307

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	1307

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	1307

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	1307

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	1307

6.7.4.1	SSB based L1-RSRP measurements	1307

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	1307

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	1312

6.7.4.2	CSI-RS based L1-RSRP measurements	1314

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	1314

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	1318

6.7.5	E-UTRAN RSRP	1320

6.7.5.0	Minimum conformance requirements	1320

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	1320

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	1321

6.7.6	E-UTRAN RSRQ	1328

6.7.6.0	Minimum conformance requirements	1328

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	1328

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	1329

6.7.7	E-UTRAN RS-SINR	1335

6.7.7.0	Minimum conformance requirements	1335

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	1335

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	1336

6.7.8	1343

6.7.9	L1-SINR measurement for beam reporting	1343

6.7.9.0	Minimum conformance requirements	1343

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1343

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	1346

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	1348

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1352

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1356

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1361

7	NR standalone with ²at least one NR cell in FR2	1367

7.1	RRC_IDLE state mobility	1367

7.1.1	NR cell re-selection	1367

7.1.1.0	Minimum conformance requirements	1367

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1367

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1367

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	1369

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	1370

7.1.1.1	NR SA FR2 cell re-selection	1371

7.1.1.2	NR SA FR2-FR2 cell re-selection	1376

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1381

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1386

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1391

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1396

7.2	RRC_INACTIVE state mobility	1401

7.3	RRC_CONNECTED state mobility	1401

7.3.1	Handover	1401

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	1401

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	1411

7.3.2	RRC connection mobility control	1418

7.3.2.1	RRC re-establishment	1418

7.3.2.1.0	Minimum conformance requirements	1418

7.3.2.1.1	NR SA FR2 RRC re-establishment	1419

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	1422

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	1426

7.3.2.2	Random access	1431

7.3.2.2.0	Minimum conformance requirements	1431

7.3.2.2.1	NR SA FR2 contention based random access	1432

7.3.2.2.2	NR SA FR2 non-contention based random access	1440

7.3.2.3	RRC connection release with redirection	1447

7.3.3	Conditional handover	1447

7.3.3.0	Minimum conformance requirements	1447

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	1447

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	1451

7.3.3.1	NR SA FR2 conditional handover	1455

7.3.3.2	NR SA FR2-FR2 conditional handover	1462

7.4	Timing	1469

7.4.1	UE transmit timing	1469

7.4.2	UE timer accuracy	1469

7.4.3	Timing advance	1469

7.5	Signalling characteristics	1469

7.5.1	Radio link monitoring	1469

7.5.1.0	Minimum conformance requirements	1469

7.5.1.0.1	1469

7.5.1.0.2	1469

7.5.1.0.3	1469

7.5.1.0.4	1469

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	1469

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	1470

7.5.2	Interruption	1474

7.5.3	SCell activation and deactivation delay	1474

7.5.3.0	Minimum conformance requirements	1474

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	1474

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	1477

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	1477

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	1479

7.5.4	UE UL carrier RRC reconfiguration delay	1482

7.5.5	Link recovery procedures	1482

7.5.5.0	Minimum conformance requirements	1482

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1482

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1484

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	1486

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	1492

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	1498

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	1505

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	1511

7.5.6	Active BWP switch delay	1516

7.5.6.1	DCI-based and time-based active BWP switch	1516

7.5.6.1.0	Minimum conformance requirements	1516

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	1517

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	1518

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	1518

7.5.6.2	RRC-based active BWP switch	1520

7.5.6.2.0	Minimum conformance requirements	1520

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	1520

7.5.7	PSCell addition and release delay	1521

7.5.7.0	Minimum conformance requirements	1521

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	1521

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	1522

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	1522

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	1524

7.6	Measurement procedures	1526

7.6.1	Intra-frequency measurements	1526

7.6.1.0	Minimum conformance requirements	1526

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1526

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1528

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	1529

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	1533

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	1538

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	1544

7.6.2	Inter-frequency measurements	1549

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1549

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	1549

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	1554

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1560

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1565

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	1571

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	1577

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1583

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1589

7.6.3	L1-RSRP measurement for beam reporting	1595

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	1595

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1595

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1595

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	1595

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	1601

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1605

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	1609

7.7	Measurement performance requirements	1612

7.7.1	SS-RSRP	1612

7.7.1.0	Minimum conformance requirements	1612

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1612

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1613

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	1613

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	1620

7.7.1.3	Inter-frequency measurements between FR1 and FR2	1628

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	1628

7.7.1.3.2	Void	1633

7.7.2	SS-RSRQ	1633

7.7.2.0	Minimum conformance requirements	1633

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1633

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1633

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	1634

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	1638

7.7.3	SS-SINR	1644

7.7.3.0	Minimum conformance requirements	1644

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1644

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1644

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	1644

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	1649

8.2.1.0	Minimum conformance requirements	1657

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	1657

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	1659

8.2.1.1.1	Test purpose	1659

8.2.1.1.2	Test applicability	1659

8.2.1.1.3	Minimum conformance requirements	1659

8.2.1.1.4	Test description	1659

8.2.1.1.5	Test requirement	1662

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	1665

8.2.1.2.1	Test purpose	1665

8.2.1.2.2	Test applicability	1665

8.2.1.2.3	Minimum conformance requirements	1665

8.2.1.2.4	Test description	1665

8.2.1.2.5	Test requirement	1667

8.3.1.0	Minimum conformance requirements	1670

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	1670

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	1671

8.3.1.1.1	Test purpose	1671

8.3.1.1.2	Test applicability	1671

8.3.1.1.3	Minimum conformance requirements	1671

8.3.1.1.4	Test description	1671

8.3.1.1.5	Test requirement	1674

8.4.1.0	Minimum conformance requirements	1676

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	1676

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	1677

8.4.1.1.1	Test purpose	1677

8.4.1.1.2	Test applicability	1678

8.4.1.1.3	Minimum conformance requirements	1678

8.4.1.1.4	Test description	1678

8.4.1.1.5	Test requirement	1680

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	1683

8.4.1.2.1	Test purpose	1683

8.4.1.2.2	Test applicability	1683

8.4.1.2.3	Minimum conformance requirements	1683

8.4.1.2.4	Test description	1683

8.4.1.2.5	Test requirement	1685

8.4.2.0	Minimum conformance requirements	1688

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	1688

8.4.2.0.2	Void	1692

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	1692

8.4.2.1.1	Test purpose	1692

8.4.2.1.2	Test applicability	1692

8.4.2.1.3	Minimum conformance requirements	1692

8.4.2.1.4	Test description	1692

8.4.2.1.5	Test requirement	1695

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	1698

8.4.2.2.1	Test purpose	1698

8.4.2.2.2	Test applicability	1698

8.4.2.2.3	Minimum conformance requirements	1698

8.4.2.2.4	Test description	1698

8.4.2.2.5	Test requirement	1701

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	1704

8.4.2.3.1	Test purpose	1704

8.4.2.3.2	Test applicability	1704

8.4.2.3.3	Minimum conformance requirements	1704

8.4.2.3.4	Test description	1704

8.4.2.3.5	Test requirement	1707

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	1710

8.4.2.4.1	Test purpose	1710

8.4.2.4.2	Test applicability	1710

8.4.2.4.3	Minimum conformance requirements	1710

8.4.2.4.4	Test description	1710

8.4.2.4.5	Test requirement	1713

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	1716

8.4.2.5.1	Test purpose	1716

8.4.2.5.2	Test applicability	1716

8.4.2.5.4	Test description	1716

8.4.2.5.5	Test requirement	1719

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	1720

8.4.2.6.1	Test purpose	1720

8.4.2.6.2	Test applicability	1720

8.4.2.6.3	Minimum conformance requirements	1720

8.4.2.6.4	Test description	1720

8.4.2.6.5	Test requirement	1723

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	1725

8.4.2.7.1	Test purpose	1725

8.4.2.7.2	Test applicability	1725

8.4.2.7.3	Minimum conformance requirements	1725

8.4.2.7.4	Test description	1725

8.4.2.7.5	Test requirement	1728

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	1729

8.4.2.8.1	Test purpose	1729

8.4.2.8.2	Test applicability	1729

8.4.2.8.3	Minimum conformance requirements	1729

8.4.2.8.4	Test description	1729

8.4.2.8.5	Test requirement	1732

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	1734

8.4.2.9.1	Test purpose	1734

8.4.2.9.2	Test applicability	1734

8.4.2.9.3	Minimum conformance requirements	1734

8.4.2.9.4	Test description	1734

8.4.2.9.5	Test requirement	1737

8.5.1	SFTD measurement accuracy	1740

8.5.1.0	Minimum conformance requirements	1740

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	1740

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	1741

8.5.1.1.1	Test purpose	1741

8.5.1.1.2	Test applicability	1741

8.5.1.1.3	Minimum conformance requirements	1741

8.5.1.1.4	Test description	1741

8.5.1.1.5	Test requirement	1743

8.5.2	Inter-RAT measurement accuracy	1747

8.5.2.1	SS-RSRP	1747

8.5.2.1.0	Minimum conformance requirements	1747

8.5.2.1.1	SS-RSRP with NR FR1 target cell	1747

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	1752

8.5.2.2	SS-RSRQ	1757

8.5.2.2.0	Minimum conformance requirements	1757

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	1757

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	1760

8.5.2.3	SS-SINR	1765

8.5.2.3.0	Minimum conformance requirements	1765

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	1765

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	1768

Annex A (normative):	RRM test configurations	1773

A.1	Reference measurement channels	1773

A.1.1	PDSCH	1773

A.1.1.1	FDD	1773

A.1.1.2	TDD	1774

A.1.2	CORESET for RMSI scheduling	1777

A.1.2.1	FDD	1777

A.1.2.2	TDD	1778

A.1.3	CORESET for RMC scheduling	1781

A.1.3.1	FDD	1781

A.1.3.2	TDD	1781

A.1.4	CSI-RS	1783

A.1.4.1	FDD	1783

A.1.4.2	TDD	1784

A.1.4A	CSI-RS for tracking	1788

A.1.4A.1	FR1	1788

A.1.4A.1.1	FDD	1788

A.1.4A.1.2	TDD	1789

A.1.4A.2	FR2	1789

A.1.4A.2.1	TDD	1789

A.1.4B	CSI-IM configurations	1789

A.1.4B.1	FDD	1789

A.1.4B.2	TDD	1790

A.1.5	TDD UL/DL configuration	1792

A.1.6	PUSCH	1793

A.2	Reference OCGN configuration	1793

A.2.1 Generic OFDMA channel noise generator (OCNG)	1793

A.3	Reference SSB configuration	1796

A.3.1	SSB configuration for FR1	1796

A.3.2	SSB configuration for FR2	1796

A.4	Reference SMTC configuration	1797

A.5	Reference DRX configurations	1797

A.6	EN-DC test setup	1797

A.6.1	E-UTRA serving cell parameters	1797

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	1797

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	1799

A.6A	NR FR1-FR2 test setup	1801

A.7	Reference PRACH configurations	1801

A.7.1	PRACH configurations for FR1	1801

A.7.2	PRACH configurations for FR2	1802

A.7A	Reference MsgA configurations	1803

A.7A.1	MsgA configurations for FR1	1803

A.7A.2	MsgA configurations for FR2	1806

A.8	Reference BWP configurations	1808

A.8.1	Downlink BWP configurations	1808

A.8.2	Uplink BWP configurations	1808

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	1809

A.9.1	Setup 1: Single AoA in Rx beam peak direction	1809

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	1809

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1809

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1809

A.9.3	Setup 3: 2 AoAs	1809

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1810

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1810

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1810

A.10	TCI State Configuration	1810

A.10.1	Introduction	1810

A.10.2	TCI states	1811

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	1812

B.1	Conditions for NR RRC_IDLE state mobility	1812

B.1.1	Introduction	1812

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1812

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1813

B.2	Conditions for NR RRC_CONNECTED state	1813

B.2.1	Introduction	1813

B.2.2	Conditions for NR intra-frequency measurements	1814

B.2.3	Conditions for NR inter-frequency measurements	1815

B.2.4	Conditions for NR L1-RSRP reporting	1815

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1815

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1816

B.2.5	Conditions for RRC connection release with redirection to NR	1817

B.2.6	Conditions for UE transmit timing	1818

B.2.6.1	Conditions for SSB based UE transmit timing	1818

B.3	RRM requirement exceptions	1819

B.3.1	Introduction	1819

B.3.2	Receiver sensitivity relaxation for CA	1819

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1819

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1820

B.3.2.2.1	Inter-band carrier aggregation	1820

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1820

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1820

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1820

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1820

B.3.2.4.1	Intra-band contiguous carrier aggregation	1820

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1821

B.3.3	Receiver sensitivity relaxation for DC	1821

B.3.4	Receiver sensitivity relaxation for SUL	1821

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1821

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1821

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1821

Annex C (normative):	Downlink physical channels and propagation conditions	1822

C.1	Downlink physical channels	1822

C.1.1	General	1822

C.1.2	Default downlink signal levels	1822

C.1.3	Default connection setup	1823

C.2	Propagation conditions	1823

C.2.1	No interference	1824

C.2.2	Static propagation conditions	1824

C.2.2.1	UE receiver with 2Rx antenna connectors	1824

C.2.2.2	UE receiver with 4Rx antenna connectors	1824

C.2.3	Multi-path fading propagation conditions	1824

Annex D (normative):	Deviations from standard test configuration	1826

D.1	Test cases with different numerologies	1826

D.2	EN-DC test cases with different EN-DC configurations	1826

D.2.1	Principle of testing	1826

D.3	Carrier aggregation test cases with different CA configurations	1826

D.3.1	Principle of testing	1826

D.4	Antenna connection for 4Rx capable UEs	1826

D.4.1	Principle of testing	1827

D.4.1.1	Single carrier tests	1827

D.4.1.2	Carrier aggregation tests	1828

D.4.1.3	EN-DC tests	1828

D.4.2	Antenna connection	1828

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	1828

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	1828

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	1828

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	1829

D.5	Test Cases with Different Channel Bandwidths	1829

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	1829

D.5.1.1	Introduction	1829

D.5.1.2	Principle of testing	1829

D.6	Test Cases for Synchronous and Asynchronous DC Operations	1829

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1829

D.6.1.1	Introduction	1829

D.6.1.2	Principle of Testing	1829

Annex E (normative):	Cell configuration mapping	1830

E.1	Test frequency selection	1830

E.1.1	E-UTRA PCell for EN-DC test cases	1830

E.1.2	Test cases with one NR cell	1830

E.1.3	Test cases with more than one NR cell	1830

E.1.3.1	Intra-frequency test cases	1830

E.1.3.2	Inter-frequency test cases	1830

E.1.4	Carrier aggregation test cases	1831

E.1.4.1	Inter-band carrier aggregation	1831

E.1.4.2	Intra-band contiguous carrier aggregation	1831

E.1.4.3	Intra-band non-contiguous carrier aggregation	1831

E.1.5	E-UTRA – NR inter RAT test cases	1831

E.1.6	Intra-band EN-DC test cases	1831

E.1.6.1	Intra-band non-contiguous EN-DC	1831

E.1.6.1.1	Inter frequency neighbour cell	1831

E.1.6.2	Intra-band contiguous EN-DC	1831

E.1.6.1.1	E-UTRA PCell	1831

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	1831

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	1834

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	1837

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	1840

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	1842

Annex F (normative):	Measurement uncertainties and test tolerances	1844

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	1844

F.1.1	Acceptable uncertainty of test system (normative)	1844

F.1.1.1	Measurement of test environments	1844

F.1.1.2	Measurement of RRM requirements	1844

F.1.2	Interpretation of measurement results (normative)	1868

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	1868

F.1.3.1	Measurement of test environments	1868

F.1.3.2	Measurement of RRM requirements	1868

Annex G (normative):	Statistical testing	1926

G.1	General	1926

G.2	Statistical testing of delay and UE measurement performance in RRM tests	1926

G.2.1	General	1926

G.2.2	Design of the test	1926

G.2.3	Numerical definition of the pass fail limits	1926

G.2.4	Pass fail decision rules	1927

G.2.5	Void	1928

G.2.6	Test conditions for delay tests and UE measurement performance	1928

G.X	Theory to derive the numbers in Table G.2.3-1 (informative)	1928

Annex H (normative):	Default message contents for RRM	1929

H.1	Void	1929

H.2	System information blocks message content exceptions	1929

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	1929

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	1931

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	1932

H.3	RRC message content exceptions	1933

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	1933

H.3.2	RRC messages and information elements contents exceptions for NR cell re-selection and handoverRACH-ConfigGeneric: for NR cell re-selection and handover	1947

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	1948

H.3.4	E-UTRA RRC messages and information elements contents exceptions for NR measurement configuration	1949

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	1956

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	1958

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	1964

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	1966

H.3.9	RRC messages and information elements contents exceptions for UL timing	1968

Annex I (normative):	RRM OTA procedures	1969

I.0	Test applicability per permitted test method	1969

I.1	Direct far field (DFF)	1969

I.1.1	RX beam peak direction search	1969

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	1969

I.2	Direct far field (DFF) simplification	1969

I.2.1	RX beam peak direction search	1969

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	1970

I.3	Indirect far field (IFF)	1970

I.3.1	RX beam peak direction search	1970

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	1970

I.3A	Enhanced indirect far field (Enhanced IFF)	1970

I.3A.1	RX beam peak direction search	1970

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	1970

I.4	Procedures to search test directions for RRM FR2	1970

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	1970

Annex J (informative):	Change history	1972

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain requirements for support of RRM (Radio Resource Management) as part of the 5G New Radio (5G-NR). The present document covers NR Range 1, NR Range 2 and Interworking.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the "test applicability" part of the test.

For example, only Release 15 and later UE declared to support NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[6]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

[8]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[9]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[10]	3GPP TS 38.215: "NR; Physical layer measurements".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[15]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[16]	3GPP TS 38.509: "5GS; Special Conformance Testing Functions for UE".

[17]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[18]	3GPP TS 38.521-2: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[19]	3GPP TS 38.521-3: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[20]	3GPP TS 38.521-4: "NR; User Equipment (UE) conformance specification; Part 4: Performance".

[21]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases".

[22]	3GPP TS 38.903: "NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".

[23]	3GPP TS 36.133: "E-UTRA requirements for support of radio resource management".

[24]	3GPP TS 36.211: "E-UTRA Physical Channels and Modulation".

[25]	3GPP TS 36.508: "Common test environments for User Equipment (UE)".

[26]	3GPP TS 36.521-3: "E-UTRA; UE conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing"

[27]	3GPP TS 36.101: "E-UTRA UE radio transmission and reception".

[28]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC) Protocol Specification".

[30]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[31]	3GPP TS 38.212 "NR; Multiplexing and channel coding".

[32]	3GPP TR 38.810: "Study on test methods for New Radio".

[33]	3GPP TR 36.213: "E-UTRA; Physical layer procedures".

[34]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[35]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Active DL BWP: Active DL bandwidth part as defined in 3GPP TS 38.213 [8].

Blackbox Approach: Testing methodology, in which the UE internal implementation of certain specific UE functionality involved in the test, is unknown.

Control Resource Set: As defined in 3GPP TS 38.213 [8].

DL BWP: DL bandwidth part as defined in 3GPP TS 38.213 [8].

EN-DC: E-UTRA-NR Dual Connectivity as defined in clause 4.1.2 of 3GPP TS 37.340 [34].

en-gNB: As defined in 3GPP TS 37.340 [34].

FR1: Frequency range 1 as defined in clause 5.1 of 3GPP TS 38.104 [28].

FR2: Frequency range 2 as defined in clause 5.1 of 3GPP TS 38.104 [28].

gNB: as defined in 3GPP TS 38.300 [TBD].

Master Cell Group: As defined in 3GPP TS 38.331 [13].

Multi-Radio Dual Connectivity: Dual Connectivity between E-UTRA and NR nodes, or between two NR nodes, as defined in 3GPP TS 37.340 [34].

ng-eNB: As defined in 3GPP TS 38.300 [35].

NE-DC: NR-E-UTRA Dual Connectivity as defined in clause 4.1.3.2 of 3GPP TS 37.340 [34].

NGEN-DC: NG-RAN E-UTRA-NR Dual Connectivity as defined in clause 4.1.3.1 of 3GPP TS 37.340 [34].

NR-DC: NR-NR Dual Connectivity as defined in clause 4.1.3.3 of 3GPP TS 37.340 [34].

Primary Cell: As defined in 3GPP TS 38.331 [13].

Quasi Co-Location: As defined in 3GPP TS 38.214 [9].

RLM-RS resource: A resource out of the set of resources configured for RLM by higher layer parameter RLM-RS-List [2] as defined in 3GPP TS 38.213 [8].

SA operation mode: Operation mode when the UE is configured with at least PCell and not any MR-DC.

Secondary Cell: As defined in 3GPP TS 38.331 [13].

Secondary Cell Group: As defined in 3GPP TS 38.331 [13].

Serving Cell: As defined in 3GPP TS 38.331 [13].

SMTC: An SSB-based measurement timing configuration configured by SSB-MeasurementTimingConfiguration as specified in 3GPP TS 38.331 [13].

Special Cell: As defined in 3GPP TS 38.331 [13].

SSB: SS/PBCH block as defined in clause 7.8.3 of 3GPP TS 38.211 [7].

Timing Advance Group: As defined in 3GPP TS 38.331 [13].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

[…]	Values included in square bracket must be considered for further studies, because it means that a decision about that value was not taken.

Tc	Basic time unit, defined in clause 4.1 of 3GPP TS 38.211 [7].

Ts	Reference time unit, defined in clause 4.1 of 3GPP TS 38.211 [7].

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AoA	Angle of Arrival

AoD	Angle of Departure

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BS	Base Station

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CDF	Cumulative Distribution Function

CC	Component Carrier

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

DUT	Device under test

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA – NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

FR1	Frequency Range 1

FR2	Frequency Range 2

HARQ	Hybrid Automatic Repeat Request

HO	Handover

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

MAC	Medium Access Control

MCG	Master Cell Group

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSA	Non-Standalone

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PSCell	Primary Secondary Cell

PSS	Primary Synchronization Signal

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SA	Standalone

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS	System Simulator

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

# 3A	Requirements for the support of RRM

## 3A.1	General

Radio Resource Management (RRM) ensures the efficient use of the available radio resources and also provides mechanisms that enable NR to meet radio resource related requirements. The requirements are divided in four main clauses according to the network deployment and the frequency range:

-	Clause 4 for EN-DC option 3 test cases where all NR cells are in FR1.

-	Clause 5 for EN-DC option 3 test cases where at least one NR cell is in FR2.

-	Clause 6 for SA option 2 test cases where all NR cells are in FR1.

-	Clause 7 for SA option 2 test cases where at least one NR cell is in FR2.

The requirements that are tested include:

-	Idle mode, the cell re-selection algorithms that are controlled by the setting of parameters (thresholds and hysteresis values) that define the best cell and/or determine when the UE should select a new cell.

Inactive mode, the cell re-selection algorithms that are controlled by the setting of parameters (thresholds and hysteresis values) that define the best cell and/or determine when the UE should select a new cell.

-	The configuration of the UE measurement and reporting procedures that are transmitted via dedicated signalling in connected mode and the reporting accuracy of the required measurements.

-	Connected mode, the mobility of radio connections that has to be supported.

-	Handover decisions that may be based on UE or gNB measurements.

-	Inter-RAT RRM, the management of radio resources in connection with inter-RAT mobility, e.g. Inter-RAT handover.

Inter frequency and inter-RAT test cases are performed without frequency overlapping between cells required in the test:

-	For bands with bandwidth not accommodating all the NR cells required in the test without frequency overlapping, inter band testing shall be done according to clause 3A.5. If the UE does not support the combination given in clause 3A.5, the relevant tests are applicable only to the bands with the necessary bandwidth.

-	In case when frequency overlapping occurs due to the frequency channel selection defined for the test (i.e. Cell number as per Annex D), other frequency channels which avoid the frequency overlapping shall be selected. If no suitable selection is found the test is not applicable for the affected band.

### 3A.1.1	Test coverage across 5G NR connectivityoptions

The test cases in this specification cover both NR/5GC (including FR1+ FR2 CA or FR1+FR2 NR-DC) as well as EN-DC and NGEN-DC testing. Below shall be the understanding with respect to coverage across 5G NR connectivity options:

1. Unless otherwise stated within the test case, it shall be understood that test requirements are agnostic of the EN-DC and NGEN-DC connectivity option configured within the test. The test coverage across the EN-DC and NGEN-DC connectivity options shall be considered fulfilled by executing the test case in one of these connectivity options.

2. EN-DC is the default connectivity option used for EN-DC and NGEN-DC test cases.

3. If a UE does not support EN-DC, NGEN-DC can be configured to execute the test. The leverage rule detailed in (1) would apply.

Table 3A.1.1-1: Void

Table 3A.1.1-2: Void

Editor’s Note: Any additional test config details needed for NE-DC and NGEN-DC are FFS.

## 3A.2	Requirements Classification for Statistical Testing

The test requirements are expressed as absolute requirements with a single value stating the requirement or expressed as a success rate. The statistical nature depends on the type of test requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a Device Under Test (DUT) passing the test actually meets the test requirement and determines how many times a test have to be repeated and what the pass and fail criteria is. The statistical significance shall be set according to Annex G.

## 3A.3	Antenna Configuration

Unless otherwise specified, NR FDD or NR TDD cells in all RRM test cases in AWGN propagation condition are configured with antenna configuration 1x2.

## 3A.4	NR band groups

The intention of the band grouping defined in this clause is to increase the readability of the test specification.

The frequency bands grouping is derived based on UE REFSENS requirements specified in 3GPP TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4] and assuming 0.5 dB step between the neighbour groups. The groups are defined in the order of increasing REFSENS, i.e., the group A has the smallest REFSENS among the groups. For the same SCS and a given bandwidth, the bands within the same group have the same Io conditions in a corresponding requirement in this specification, provided the bands support this SCS. For different SCSs supported by a frequency band and the same bandwidth, different Io conditions may apply for the frequency band in the requirements, while the band group is the same, based on the lowest REFSENS requirement normalized by the number of subcarriers among its supported SCSs for this bandwidth. For the same SCS but different supported bandwidths, the group for a band is determined based on the lowest REFSENS requirement normalized by the number of subcarriers among its supported bandwidths.

### 3A.4.1	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3A.4.1-1.

Table 3A.4.1-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51, n53 | NR_SDL_FR1_A | n75, n76 |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41 | NR_SDL_FR1_E | - |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |
| G | NR_FDD_FR1_G | n3, n8, n12, n14, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | n29 |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in 3GPP TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |

Table 3A.4.1-2: Power offsets for the test configuration between NR frequency band groups for FR1 with respect to NR_FDD_FR1_A

| Group | NR FDD |  | NR TDD |  | NR SDL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Power Offset [dB], ΔBG_offset | Band group notation | Power Offset [dB], ΔBG_offset | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | - | NR_TDD_FR1_A | 0.0 | NR_SDL_FR1_A | 0.0 |
| B | NR_FDD_FR1_B | 0.5 | NR_TDD_FR1_B | 0.5 | NR_SDL_FR1_B | - |
| C | NR_FDD_FR1_C | 1.0 | NR_TDD_FR1_C | 1.0 | NR_SDL_FR1_C | - |
| D | NR_FDD_FR1_D | 1.5 | NR_TDD_FR1_D | 1.5 | NR_SDL_FR1_D | - |
| E | NR_FDD_FR1_E | 2.0 | NR_TDD_FR1_E | 2.0 | NR_SDL_FR1_E | - |
| F | NR_FDD_FR1_F | 2.5 | NR_TDD_FR1_F | 2.5 | NR_SDL_FR1_F | - |
| G | NR_FDD_FR1_G | 3.0 | NR_TDD_FR1_G | 3.0 | NR_SDL_FR1_G | - |
| H | NR_FDD_FR1_H | 3.5 | NR_TDD_FR1_H | 3.5 | NR_SDL_FR1_H | - |
| NOTE: In the test parameters table, only the power configuration for NR_FDD_FR1_A or NR_TDD_FR1_A will be given. |  |  |  |  |  |  |

### 3A.4.2	NR operating bands in FR2

NR frequency bands grouping for FR2 is specified in Table 3A.4.2-1.

Table 3A.4.2-1: NR frequency band groups for FR2

| Group | Band group notation | Operating bands |
| --- | --- | --- |
| A | NR_TDD_FR2_A | n2571, n2581, n2611 |
| B | NR_TDD_FR2_B | n2574, n2584, n2614 |
| C | NR_TDD_FR2_C |  |
| D | NR_TDD_FR2_D |  |
| E | NR_TDD_FR2_E |  |
| F | NR_TDD_FR2_F | n2604 |
| G | NR_TDD_FR2_G | n2601 |
| H | NR_TDD_FR2_H |  |
| I | NR_TDD_FR2_I |  |
| J | NR_TDD_FR2_J |  |
| K | NR_TDD_FR2_K |  |
| L | NR_TDD_FR2_L | n2572, n2582, n2612 |
| M | NR_TDD_FR2_M |  |
| N | NR_TDD_FR2_N |  |
| O | NR_TDD_FR2_O |  |
| P | NR_TDD_FR2_P |  |
| Q | NR_TDD_FR2_Q |  |
| R | NR_TDD_FR2_R |  |
| S | NR_TDD_FR2_S |  |
| T | NR_TDD_FR2_T | n2573, n2583, n2613 |
| U | NR_TDD_FR2_U |  |
| V | NR_TDD_FR2_V |  |
| W | NR_TDD_FR2_W |  |
| X | NR_TDD_FR2_X |  |
| Y | NR_TDD_FR2_Y | n2603 |
| Z | NR_TDD_FR2_Y |  |
| AA | NR_TDD_FR2_AA | n2593 |
| NOTE 1: UE power class 1. NOTE 2: UE power class 2. NOTE 3: UE power class 3. NOTE 4: UE power class 4. |  |  |

Table 3A.4.2-2: FFS

## 3A.5	NR operating band configuration

The additional band defined in Table 3A.5-1 shall be used for RRM inter-frequency tests where the band under test cannot accommodate two (or more) non-overlapping inter-frequency cells. The usage of the additional band is conditioned to the UE supporting this band, the corresponding channel bandwidth and other band-dependent parameters. If the UE does not support the additional band or associated parameters, the inter-frequency test shall be considered Not Applicable for the band under test.

Inter-band configuration is not affecting the Test purpose since the minimum requirements are valid regardless of band. Band combinations defined in table 3A.5-1 shall be used for testing.

Table 3A.5-1: Inter-band configuration

| Band under test | Additional band (s) |
| --- | --- |
| n12 | n66 |
| n14 | n66 |
| n18 | n1 |
| n30 | n66 |
| n34 | n41 |
| n38 | n41 |
| n53 | n41 |
| n70 | n66 |
| NOTE 1: The band under test should contain the inter-frequency (neighbour) cell. NOTE 2: The additional band should contain the serving cell of the test. If more than one inter-frequency cell is needed, that cell should be on the additional band. NOTE 3: The bands and cells referred in this table are NR bands and cells only. For instructions on how to configure the E-UTRA operating band please refer to 3GPP TS 36.521-3 [26]. NOTE 4: The additional bands in this table are to be used in NR SA test cases only. EN-DC test cases cannot make use of additional bands. |  |

## 3A.6	UE with Multiband Capability

The Radio Resource Management performance of a UE in all sections except 4.7, 5.7, 6.7, 7.7 and 8.5.2 is independent from all bands. Therefore, the required performance in the respective test cases can be verified in one of the bands supported by the UE, except for inter-band testing requirements in clause 3A.1.

The NR standalone test cases in sections 6.7 and 7.7 are dependent on the NR band of the target cell, and is required to be verified in all UE supported NR bands.

The EN-DC test cases in sections 4.7, 5.7 and inter-RAT test cases in clause 8.5.2 are dependent on the NR band of the target cell, and is required to be verified in all UE supported NR bands. It is not necessary to repeat the tests for different LTE bands. For EN-DC configurations with the same NR band, it is sufficient to configure any one of them to execute the tests.