# 7	NR standalone with ²at least one NR cell in FR2

This section contains test scenarios for NR standalone. This configuration is also known as SA Option 2. At least one NR cell is in Frequency Range 2.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 7.1	RRC_IDLE state mobility

### 7.1.1	NR cell re-selection

#### 7.1.1.0	Minimum conformance requirements

##### 7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [30] within Tdetect,NR_Intra as defined in table 4.2.2.3-1 of TS 38.133 [6] when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 of TS 38.133 [6] for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1 of TS 38.133 [6]) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 of TS 38.133 [6] provided that the cell has at least [3]dB better ranked.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and non-zeroa-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.2 and 4.2.2.3.

##### 7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 of TS 38.133 [6].

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [30] within Kcarrier * Tdetect,NR_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5] dB for reselections based on ranking or [6]dB for SS-RSRP reselections based on absolute priorities or [4]dB for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 of TS 38.133 [6] for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-1 of TS 38.133 [6]) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [30] within Kcarrier * Tevaluate,NR_Inter when Treselection = 0 as specified in table 4.2.2.4-1 of TS 38.133 [6] provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and the cell has at least [5]dB better ranked

-	[6]dB for SS-RSRP reselections based on absolute priorities or

-	[4]dB for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 of TS 38.133 [6] under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively,

-	SMTC occasions configured for the inter-frequency carrier occur up to TBD ms before the start or up to TBD ms after the end of the SMTC occasions configured for the intra-frequency carrier and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to TBD ms before the start or up to TBD ms after the end of the paging occasion [1].

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.4.

##### 7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

7.1.1.0.3.1	Introduction

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in TS 38.304 [30],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in TS 38.304 [30],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in TS 38.304 [30] respectively.

7.1.1.0.3.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with lowMobilityEvaluation [13] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvaluation [13] criterion and cellEdgeEvaluation [13] criterion and combineRelaxedMeasCondition [13] not configured, and UE has fulfilled only the lowMobilityEvaluation [13] criterion.

The requirements defined in clause 7.1.1.0.1 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.2-1 in TS 38.133 [6].

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.2-1 in TS 38.133 [6].

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.2-1 in TS 38.133 [6].

7.1.1.0.3.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [13] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvaluation [13] criterion and cellEdgeEvaluation [13] criteria and combineRelaxedMeasCondition [13] not configured, and UE has fulfilled only the cellEdgeEvaluation [13] criterion.

The requirements defined in clause 7.1.1.0.1 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.3-1 in TS 38.133 [6].

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.3-1 in TS 38.133 [6].

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.3-1 in TS 38.133 [6].

7.1.1.0.3.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvaluation [13] criterion and cellEdgeEvaluation [13] criterion, and

-	has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1 of TS 38.133[6].

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.9.

##### 7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion

The cell re-selection delay shall be less than Tevaluate NR_Inter + TSI-NR in RRC_IDLE state.

7.1.1.0.4.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells when the UE is configured with any of following relaxed measurement criteria:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in TS 38.304 [30],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4. 9.2 in TS 38.304 [30],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4. 9.1 and 5.2.4.9.2 in TS 38.304 [30] respectively.

7.1.1.0.4.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with lowMobilityEvaluation [13] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvaluation [13] and cellEdgeEvaluation [13] criterion and combineRelaxedMeasCondition [13] not configured, and UE has fulfilled only the lowMobilityEvaluation [13] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.2-1 in TS 38.133 [6].

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.2-1 in TS 38.133 [6].

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.2-1 in TS 38.133 [6].

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [13] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [13] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 of TS 38.133[6].

7.1.1.0.4.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with cellEdgeEvaluation [13] criterion, and UE has fulfilled or

-	UE is configured with both lowMobilityEvaluation [13] criterion and cellEdgeEvaluation [13] criterion and combineRelaxedMeasCondition [13] not configured, and UE has fulfilled only the cellEdgeEvaluation [13] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then te requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.3-1 in TS 38.133 [6].

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.3-1 in TS 38.133 [6].

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.3-1 in TS 38.133 [6].

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [13] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7

7.1.1.0.4.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with both lowMobilityEvaluation [13] criterion and cellEdgeEvaluation [13] criterion, and

-	Has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.4-1 of TS 38.133[6].

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.10.

#### 7.1.1.1	NR SA FR2 cell re-selection

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing.

- Connection diagram is TBD.

7.1.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements specified in TS 38.133 clause 4.2.2.3.

7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.1.

7.1.1.1.4	Test description

7.1.1.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.1.4.1-1.

Table 7.1.1.1.4.1-1: Supported test configurations for NR SA FR2 cell re-selection

| Configuration | Description |
| --- | --- |
| 7.1.1.1-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.1-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.1.4.1-2.

Table 7.1.1.1.4.1-2: Initial conditions for NR SA FR2 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.1.4.1-3.

2. Message contents are defined in clause 7.1.1.1.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.1.4.1-3: General test parameters for NR SA FR2 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Visited cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

7.1.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial Cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 7.1.1.1.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.1.5-1.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 130 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 7.1.1.1.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 27 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, continues with step 11a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.1.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

7.1.1.1.5	Test requirement

Tables 7.1.1.1.4.1-3 and 7.1.1.1.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case.

Table 7.1.1.1.5-1: Cell specific test parameters for NR SA FR2 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
|  |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.9.1 |  |  | Setup 1 defined in A.3.9.1 |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -96 | -91.5 | -infinity | -91.5 | -96 |
|  |  | 2 | -82 | -93 | -88.5 | -infinity | -88.5 | -93 |
| Io | dBm/95.04 MHz | 1 | -59.37 | -63.40 | -62.47 | -64.01 | -62.47 | -63.40 |
|  |  | 2 | -57.18 | -62.86 | -61.67 | -64.01 | -61.67 | -62.86 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell re-selection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 128 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 129.28 seconds in this test case (note: this gives a total of 129.28 seconds but the test allows 130 seconds).

The cell re-selection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on Cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 25.6 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 26.88 seconds in this test case (note: this gives a total of 26.88 seconds but the test allows 27 seconds).

For the test to pass, both events above shall pass.

The statistical pass/ fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 7.1.1.2	NR SA FR2-FR2 cell re-selection

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing.

- Connect diagram is TBD

7.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements specified in TS 38.133 clause 4.2.2.4.

7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.2.

7.1.1.2.4	Test description

7.1.1.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.2.4.1-1.

Table 7.1.1.2.4.1-1: Supported test configurations for NR SA FR2-FR2 cell re-selection

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 7.1.1.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.2-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.2.4.1-2.

Table 7.1.1.2.4.1-2: Initial conditions for NR SA FR2-FR2 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.2.4.1-3.

2. Message contents are defined in clause 7.1.1.2.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 2 is the PCell and Cell 1 is the neighbour cell in a different carrier than cell 2. Cell 1 and Cell 2 are configured according to Annex C.1.2.

Table 7.1.1.2.4.1-3: General test parameters for NR SA FR2-FR2 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

7.1.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

2.	Set the parameters according to T1 in Table 7.1.1.2.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Void.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

5.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 27 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

6.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6a.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

6a.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.2.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

8.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.1.1.2.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

10.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 87 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

11.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, continues with step 11a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

13.	Repeat step 3-12 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 7.1.1.2-1 | Table H.2.2-2 with Condition SSB.1 FR2 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 7.1.1.2-2 | Table H.2.2-2 with Condition SSB.2 FR2 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.1.1.2.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

7.1.1.2.5	Test requirement

Tables 7.1.1.2.4.1-3 and 7.1.1.2.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 7.1.1.2.5-1: Cell specific test parameters for NR SA FR2-FR2 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.9.1 |  |  | Setup 1 defined in A.3.9.1 |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -85 | -96 | -infinity | -85 |
|  |  | 2 | -82 | -82 | -82 | -93 | -infinity | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -55.37 | -62.25 | -infinity | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -52.37 | -59.25 | -infinity | -52.37 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 2.

The cell re-selection delay to a higher priority cell test requirement in this case is expressed as:

Cell re-selection delay to a higher priority cell = Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR

Thigher_priority_search = 60 s, as specified in TS 38.133 [6] clause 4.2.2.7;

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a higher priority cell shall be less than a total of 86.88 seconds in this test case (note: this gives a total of 86.88 seconds but this test allows 87 seconds)

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 1.

The cell re-selection delay to a lower priority cell test requirement in this case is expressed as:

Cell re-selection delay to a lower priority cell = Tevaluate, NR_ inter + TSI-NR

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a lower priority cell shall be less than a total of 26.88 seconds in this test case (note: this gives a total of 26.88 seconds but this test allows 27 seconds)

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.1.1.3.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements when UE fulfils the low mobility relaxed measurement criterion specified in TS 38.133 clause 4.2.2.9.

7.1.1.3.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards supporting relaxed RRM measurement.

7.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.3.

7.1.1.3.4	Test description

7.1.1.3.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.3.4.1-1.

Table 7.1.1.3.4.1-1: Supported test configurations for NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Configuration | Description |
| --- | --- |
| 7.1.1.3-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.3-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.3.4.1-2.

Table 7.1.1.3.4.1-2: Initial conditions for NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.3.4.1-3.

2. Message contents are defined in clause 7.1.1.3.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.3.4.1-3: General test parameters for NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

7.1.1.3.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 2 successive time periods, with time duration of T1, T2 respectively. Both cell 1 and cell2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

The UE is configured only low mobility criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvaluation [2] is configured according to the parameters listed in Table 7.1.1.3.5-1;

-	cellEdgeEvaluation [2] is not configured;

-	combineRelaxedMeasCondition [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.3.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 100 seconds from the beginning of time period T1, then count a success for cell reselection. Otherwise count a fail for cell reselection.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 10.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.3.5-1. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

8.	If the UE responds to Cell 1 during time duration T2 within 100 seconds from the beginning of time period T2, then count a success for cell reselection. Otherwise count a fail for cell reselection.

9.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, continue with step 10. Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.1.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.3.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.3.4.3-3: SIB2

| Derivation Path: TS 38.508-1 [14] Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchDeltaP-r16 | dB6 |  |  |
| t-SearchDeltaP-r16 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluation-r16 | Not present |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.3.5	Test requirement

Tables 7.1.1.3.4.1-3 and 7.1.1.3.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE fulfilling low mobility relaxed measurement criterion.

Table 7.1.1.3.5-1: Cell specific test parameters for NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
|  |  | 2 | -137 |  | -137 |  |  |
| SSearchDeltaP | dB | 1, 2 | 6 |  | 6 |  |  |
| TSearchDeltaP | s | 1,2 | 5 |  | 5 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -96 | -91.5 | -91.5 | -96 |  |
|  |  | 2 | -93 | -88.5 | -88.5 | -93 |  |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -63.40 | -62.47 | -62.47 | -63.40 |  |
|  |  | 2 | -62.86 | -61.67 | -61.67 | -62.86 |  |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |  |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

The cell reselection delay to an already detected cell for UE fulfilling low mobility relaxed criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell reselection delay to an already detected cell for UE fulfilling low mobility relaxed criterion is also defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected cell shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 76.8 s; as specified in TS 38.133 [6] clause 4.2.2.9.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

This gives a total of 78.08 s, allow 79s for the cell re-selection delay to an already detected cell for UE fulfilling low mobility criterion in the test case.

#### 7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.1.1.4.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements when UE fulfils the not-at-cell edge relaxed measurement criterion specified in TS 38.133 clause 4.2.2.9.

7.1.1.4.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards supporting relaxed RRM measurement.

7.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.4.

7.1.1.4.4	Test description

7.1.1.4.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.4.4.1-1.

Table 7.1.1.4.4.1-1: Supported test configurations for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Configuration | Description |
| --- | --- |
| 7.1.1.4-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.4-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.4.4.1-2.

Table 7.1.1.4.4.1-2: Initial conditions for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.4.4.1-3.

2. Message contents are defined in clause 7.1.1.4.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.4.4.1-3: General test parameters for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

7.1.1.4.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 2 successive time periods, with time duration of T1, T2 respectively. Both cell 1 and cell2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

The UE is configured only not-at-cell edge criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvaluation [2] is not configured;

-	cellEdgeEvaluation [2] is configured according to the parameters listed in Table 7.1.1.4.5-1;

-	combineRelaxedMeasCondition [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.4.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 100 seconds from the beginning of time period T1, then count a success for cell reselection. Otherwise count a fail for cell reselection.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 10.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.4.5-1. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

8.	If the UE responds to Cell 1 during time duration T2 within 100 seconds from the beginning of time period T2, then count a success for cell reselection. Otherwise count a fail for cell reselection.

9.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, continue with step 10. Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.1.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.4.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.4.4.3-3: SIB2

| Derivation Path: TS 38.508-1 [14] Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 | Not present |  |  |
| cellEdgeEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchThresholdP-r16 | 35 |  |  |
| } |  |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.4.5	Test requirement

Tables 7.1.1.4.4.1-3 and 7.1.1.4.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE fulfilling not-at-cell edge relaxed measurement criterion.

Table 7.1.1.4.5-1: Cell specific test parameters for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
|  |  | 2 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -96 | -91.5 | -91.5 | -96 |  |
|  |  | 2 | -93 | -88.5 | -88.5 | -93 |  |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -63.40 | -62.47 | -62.47 | -63.40 |  |
|  |  | 2 | -62.86 | -61.67 | -61.67 | -62.86 |  |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |  |
| SSearchThresholdP |  | 1, 2 | 35 | 35 | 35 | 35 |  |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

The cell reselection delay to an already detected cell for UE fulfilling not-at-cell edge relaxed criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell reselection delay to an already detected cell for UE fulfilling not-at-cell edge relaxed criterion is also defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected cell shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 76.8 s; as specified in TS 38.133 [6] clause 4.2.2.9.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

This gives a total of 78.08 s, allow 79s for the cell re-selection delay to an already detected cell for UE fulfilling not-at-cell edge criterion in the test case.

#### 7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.1.1.5.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements when UE fulfils the low mobility relaxed measurement criterion specified in TS 38.133 clause 4.2.2.10.

7.1.1.5.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards supporting relaxed RRM measurement.

7.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.5.

7.1.1.5.4	Test description

7.1.1.5.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.5.4.1-1.

Table 7.1.1.5.4.1-1: Supported test configurations for NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Configuration | Description |
| --- | --- |
| 7.1.1.5-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.5-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.5.4.1-2.

Table 7.1.1.5.4.1-2: Initial conditions for NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.5.4.1-3.

2. Message contents are defined in clause 7.1.1.5.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.5.4.1-3: General test parameters for NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell2 and fulfils low mobility (lowMobilityEvalutation [2]) criterion. |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to low priority cell1 during T1 |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| T2 final condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to high priority cell2 during T2 |
|  | Neighbour cell |  |  | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 85 | T1 needs to be long enough to allow cell re-selection to already known cell1 |
| T2 |  | s | 1, 2 | 85 | T2 needs to be long enough to allow cell re-selection to already known cell2 |

7.1.1.5.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of two successive time periods, with time duration of T1, T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

The UE is configured only low mobility criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvaluation [2] is configured according to the parameters listed in Table 7.1.1.5.5-1;

-	cellEdgeEvaluation [2] is not configured;

-	combineRelaxedMeasCondition [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.5.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

4.	If the UE responds on lower priority Cell 1 during time duration T1 within 85 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 10.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.5.5-1. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 2.

8.	If the UE responds to higher priority Cell 2 during time duration T2 within 85 seconds from the beginning of time period T2, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”

9.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 10. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.5.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.5.4.3-3: SIB2

| Derivation Path: TS 38.508-1 [14] Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchDeltaP-r16 | dB6 |  |  |
| t-SearchDeltaP-r16 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluation-r16 | Not present |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.5.5	Test requirement

Tables 7.1.1.5.4.1-3 and 7.1.1.5.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case for UE fulfilling low mobility relaxed measurement criterion.

Table 7.1.1.5.5-1: Cell specific test parameters for NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |
|  |  | 2 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |
| Beam assumptionNote 4 |  | 1, 2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 8 | [8] | -3 | [8] |
| Note2 | dBm/SCS | 1 | -93 |  | -93 |  |
|  |  | 2 | -90 |  | -90 |  |
| Note2 | dBm/15 kHz | 1, 2 | -102 |  | -102 |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -96 | -85 |
|  |  | 2 | -82 | -82 | -93 | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -62.25 | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -59.25 | -52.37 |
| TreselectionNR | s | 1, 2 | 0 |  | 0 |  |
| SnonintrasearchP | dB | 1, 2 | 50 |  | Not sent |  |
| SSearchDeltaP | dB | 1, 2 | 6 |  | 6 |  |
| TSearchDeltaP | s | 1, 2 | 5 |  | 5 |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The cell reselection delay to an already detected low priority cell (Cell 1) for UE fulfilling low mobility criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than 79 s.

The cell reselection delay to an already detected high priority cell (Cell 2) for UE fulfilling low mobility criterion is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than [79] s.

NOTE 1:	The cell re-selection delay to an already detected low priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

NOTE 2:	The cell re-selection delay to an already detected higher priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

Where:

Tevaluate, NR_ inter	as speciied in TS 38.133 Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected low priority cell for UE fulfilling low mobility criterion in the test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected high priority cell for UE fulfilling low mobility criterion in the test case.

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.1.1.6.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements when UE fulfils the not-at-cell edge relaxed measurement criterion specified in TS 38.133 clause 4.2.2.10.

7.1.1.6.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards supporting relaxed RRM measurement.

7.1.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.6.

7.1.1.6.4	Test description

7.1.1.6.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.6.4.1-1.

Table 7.1.1.6.4.1-1: Supported test configurations for NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Configuration | Description |
| --- | --- |
| 7.1.1.6-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.6-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.6.4.1-2.

Table 7.1.1.6.4.1-2: Initial conditions for NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.6.4.1-3.

2. Message contents are defined in clause 7.1.1.6.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.6.4.1-3: General test parameters for NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell2 and fulfils not-at-cell edge (cellEdgeEvaluation [2]) criterion. |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to low priority cell1 during T1 |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| T2 final condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to high priority cell2 during T2 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 85 | T1 needs to be long enough to allow cell re-selection to already known cell. |
| T2 |  | s | 1, 2 | 85 | T2 needs to be long enough to allow cell re-selection to already known cell. |

7.1.1.6.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of two successive time periods, with time duration of T1, T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

The UE is configured only not-at-cell edge criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvaluation [2] is not configured;

-	cellEdgeEvaluation [2] is configured according to the parameters listed in Table 7.1.1.6.5-1;

-	combineRelaxedMeasCondition [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.6.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

4.	If the UE responds on lower priority Cell 1 during time duration T1 within 85 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 10.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.6.5-1. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 2.

8.	If the UE responds to higher priority Cell 2 during time duration T2 within 85 seconds from the beginning of time period T2, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”

9.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 10. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.6.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.6.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3-3: SIB2

| Derivation Path: TS 38.508-1 [14] Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 | Not present |  |  |
| cellEdgeEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchThresholdP-r16 | 35 |  |  |
| } |  |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.6.5	Test requirement

Tables 7.1.1.6.4.1-3 and 7.1.1.6.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge relaxed measurement criterion.

Table 7.1.1.6.5-1: Cell specific test parameters for NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |
|  |  | 2 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |
| Beam assumptionNote 4 |  | 1, 2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| Note2 | dBm/SCS | 1 | -93 |  | -93 |  |
|  |  | 2 | -90 |  | -90 |  |
| Note2 | dBm/15 kHz | 1, 2 | -102 |  | -102 |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -96 | -85 |
|  |  | 2 | -82 | -82 | -93 | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -62.25 | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -59.25 | -52.37 |
| SSearchThresholdP |  | 1, 2 | 35 | 35 | 35 | 35 |
| TreselectionNR | s | 1, 2 | 0 |  | 0 |  |
| SnonintrasearchP | dB | 1, 2 | 50 |  | Not sent |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The cell reselection delay to an already detected low priority cell (Cell 1) for UE fulfilling not-at-cell edge criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than 79 s.

The cell reselection delay to an already detected high priority cell (Cell 2) for UE fulfilling not-at-cell edge criterion is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than [79] s.

NOTE 1:	The cell re-selection delay to an already detected low priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

NOTE 2:	The cell re-selection delay to an already detected higher priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

Where:

Tevaluate, NR_ inter	as speciied in TS 38.133 Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected low priority cell for UE fulfilling not-at-cell edge criterion in the test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected high priority cell for UE fulfilling not-at-cell edge criterion in the test case.

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

## 7.2	RRC_INACTIVE state mobility

## 7.3	RRC_CONNECTED state mobility

### 7.3.1	Handover

#### 7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover

This test case is incomplete. Following aspects are still missing:

- TT & MU analysis are missing.

- Connection diagram is missing

7.3.1.4.1	Test purpose

To verify the requirement for the NR FR1-NR FR2 inter-band inter-frequency synchronous DAPS handover requirements in synchronous scenario specified in 38.133 [6] clause 6.1.3.4.

7.3.1.4.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards and support inter-frequency DAPS handover.

7.3.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.1.4.

7.3.1.4.4	Test description

7.3.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.1.4.4.1-1.

Table 7.3.1.4.4.1-1: NR SA FR1-FR2 synchronous DAPS handover test configurations

| Config | Description |
| --- | --- |
| 7.3.1.4-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.1.4-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.1.4-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.1.4.4.1-2

Table 7.3.1.4.4.1-2: Initial conditions for NR SA FR1-FR2 synchronous DAPS handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.1.4.4.3.

2.	The power levels and settings for NR FR1 Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.1.4.4.1-3 below, with A4-Threshold modified by Test Tolerance.

Table 7.3.1.4.4.1-3: General test parameters for NR SA FR1-FR2 synchronous DAPS handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Threshold |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 33 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in 38.133 [6] Table 6.1.3.4.2-2 for synchronous DAPS HO |
| T5 |  | ms | 100 |  |

7.3.1.4.4.2	Test procedure

The test scenario comprises of two NR carriers each with one cell. NR Cell 1 is the source PCell on a FR1 carrier and NR Cell 2 is the target PCell on a FR2 carrier. General parameters for Cell 1 and Cell 2 are given in Table 7.3.1.4.4.1-3 and Table 7.3.1.4.4.1-4 respectively.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to the Cell 1 and not aware of the Cell 2. During T1, the UE does not have any timing information of the Cell 2.

Before the start of T2, the UE is configured with event A4 triggered measurement reporting on target carrier and measurement gaps (gap pattern ID # 0). From start of T2, the Cell 2 becomes detectable. The UE performs cell detection and measurements on the Cell 2 and shall send event report to the network. After receiving the event report A4, the network sends a RRC message implying DAPS handover command for target cell addition to the UE.

T3 starts from the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing DAPS handover command for target cell addition sent to the UE. During T3, UE shall be able to perform random access, DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After successful RACH procedure on the Cell 2, UE is scheduled with PDSCH from Cell 1 and Cell 2 in alternative TTIs. In the end of T3 the network sends a RRC message implying DAPS handover command for source cell release to the UE. During T3, the handover delay Dhandover1 for target cell addition need to be verified.

T4 starts from the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing DAPS handover command for source cell release sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2

From start of T5, the UE shall stop sending periodical CSI report on Cell 1.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.1.4.5-1 and Table 7.3.1.4.5-2 respectively. Propagation conditions are set according to Annex C clause C.2.2. T1 starts and the SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedbacks sent by the UE.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2).

4.	The SS shall transmit an RRCReconfiguration message to configure event A4 triggered measurement reporting on the inter-frequency carrier and periodical CSI reporting on Cell 1.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.1.4.5-1 and Table 7.3.1.4.5-2 respectively.

7.	UE shall transmit a MeasurementReport message triggered by Event A4.

8.	SS shall transmit an RRCReconfiguration with reconfigurationWithSync message which reconfiguring DRB as a DAPS radio bearer to the UE, at that instant the SS receives the ACK of the PDSCH corresponding to the RRCReconfiguration with reconfigurationWithSync message the SS shall switch the power settings from T2 to T3 as specified in Table 7.3.1.4.5-1 and Table 7.3.1.4.5-2. T3 starts and the SS stops scheduling the UE to perform DL reception on Cell 1.

9.	If the UE transmits PRACH preamble to Cell 2 within Dhandover1 from the beginning of time period T3 then the number of successful tests is increased by one, continue to step 10. Otherwise, the number of failure tests is increased by one, go to step 17. where:

-	Dhandover1 = 92 ms.

10.	The UE transmits an RRCReconfigurationComplete message on Cell 2.

11.	Upon receiving RRCReconfigurationComplete message sent by UE, the SS immediately starts scheduling UE to perform DL reception in Cell 1 and Cell 2 in an alternative manner, and monitoring corresponding ACK/NACK feedbacks sent by the UE.

12.	Upon T3 expiring, the SS immediately transmit an RRCReconfiguration with daps-SourceRelease-r16 = true on Cell 2 to the UE. T4 starts when the SS receives the ACK of the PDSCH corresponding to the RRCReconfiguration with daps-SourceRelease-r16 = true.

13.	The UE transmits an RRCReconfigurationComplete message on Cell 2.

14.	When T4 expires, T5 starts.

15.	If

- The UE can report ACK/NACK from the first DL reception scheduled on Cell 2 after the beginning of time period T5,

and

- The UE doesn't send periodical CSI report during entire time period T5.

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

16.	After T5 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

17.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

18.	Repeat steps 2-17 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.1.4.4.3-1: RRCReconfiguration (Step 4)

| Derivation Path: TS 38.508-1 [14], 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 7.3.1.4.4.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.3.1.4.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-2: ServingCellConfig (Table 7.3.1.4.4.3-1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM Specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-3: MeasConfig (Table 7.3.1.4.4.3-1)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR(-120) specified in Table H.3.1-4AA | Acutal value of A4-threshold is -120dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-4: MeasurementReport (Step 7)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults specified in Table H.3.1-7 with condition INTER-FREQ |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.1.4.4.3-5: RRCReconfiguration (Step 8)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_NoKeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 7.3.1.4.4.3-6 |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | Table 7.3.1.4.4.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-6: RadioBearerConfig (Table 7.3.1.4.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-132 with condition DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRBn | DRB #n is a DRB established before DAPS HO. Actual value of n is left to TE implementation |  |
| daps-Config-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-7: CellGroupConfig (Table 7.3.1.4.4.3-6)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | PhysCellId for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1.4.4.3-8: RRCReconfiguration (Step 12)

Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition DAPS_HO_ReleaseSource

7.3.1.4.5	Test requirements

Table 7.3.1.4.5-1 and Table 7.3.1.4.5-2 defines the primary level settings including test tolerances for all tests.

Table 7.3.1.4.5-1: Cell specific test parameters for NR SA FR1-FR2 synchronous DAPS handover (Cell 1 in FR1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | NA Link only, see clause A.3.7A |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz |  |  |  |  |  |
|  | Config 3 |  | dBm/ 38.16MHz |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 7.3.1.4.5-2: Cell specific test parameters for NR SA FR1-FR2 synchronous DAPS handover (Cell 2 in FR2)

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Duplex mode | Config 1,2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  |  |  |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| BWP BW | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR3.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR3.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | SSB.1 FR2 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 2 |  |  |  |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |  |  |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note2 |  | dBm/SCS | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
| IoNote3 |  | dBm/ 9.36MHz | -66.7 | -55.4 | -55.4 | -55.4 | -55.4 |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in 38.133 [6] B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 from the beginning of time period T3. The interruption on Cell 1 shall not exceed Tinterrupt1 as defined in TS 38.133 [6] Table 6.1.3.4.2-1 for synchronous DAPS HO. The rate of correct handovers observed during repeated tests shall be at least 90%, where:

Dhandover1= TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

-	Tsearch = 0 ms for known target cell, is the the time required to search the target cell specified in 38.133 [6] clause 6.1.1.2.2;

TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell specified in 38.133 [6] clause 6.1.1.2.2.

-	Tprocessing = 40 ms, is the time for UE processing specified in 38.133 [6] clause 6.1.1.2.2.

-	T∆ = 20 ms, is the time for fine time tracking and acquiring full timing information of the target cell specified in 38.133 [6] clause 6.1.1.2.2.

-	Tmargin = 2 ms, is the time for SSB post-processing specified in 38.133 [6] clause 6.1.1.2.2.

This gives a total of 92 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from Cell 1 and Cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release Cell 1 less than Dhandover2 from the beginning of time period T4, where:

Dhandover2= TRRC_procedure + Tinterrupt2

-	TRRC_procedure= 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

-	Tinterrupt2 = 1.125 ms for sync intra-frequency DAPS handover, is the allowed interruption length during Dhandover2 as in 38.133 [13] clause 6.1.1.2.2;

This gives a total of 11.125 ms.

UE shall not report CSI to Cell 1 during T5.

#### 7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover

This test case is incomplete. Following aspects are still missing:

- TT & MU analysis are missing.

- Connection diagram is missing

7.3.1.5.1	Test purpose

To verify the requirement for the NR FR1-NR FR2 inter-band inter-frequency asynchronous DAPS handover requirements in synchronous scenario specified in 38.133 [6] clause 6.1.3.4.

7.3.1.5.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards and supporting inter-frequency asynchronous DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell in DAPS handover.

7.3.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.1.5.

7.3.1.5.4	Test description

7.3.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.1.5.4.1-1.

Table 7.3.1.5.4.1-1: NR SA FR1-FR2 asynchronous DAPS handover test configurations

| Config | Description |
| --- | --- |
| 7.3.1.5-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.1.5-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.1.5-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.1.5.4.1-2

Table 7.3.1.5.4.1-2: Initial conditions for NR SA FR1-FR2 asynchronous DAPS handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.1.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.1.5.4.3.

2.	The power levels and settings for NR FR1 Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.1.5.4.1-3 below, with A4-Threshold modified by Test Tolerance.

Table 7.3.1.5.4.1-3: General test parameters for NR SA FR1-FR2 asynchronous DAPS handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Threshold |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 62.5 | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.4.2-2 for asynchronous DAPS HO. |
| T5 |  | ms | 100 |  |

7.3.1.5.4.2	Test procedure

Same test procedure as described in clause 7.3.1.4.4.2.

7.3.1.5.4.3	Message contents

Same message contents as described in clause 7.3.1.4.4.3.

7.3.1.5.5	Test requirements

Table 7.3.1.5.5-1 and Table 7.3.1.5.5-2 defines the primary level settings including test tolerances for all tests.

Table 7.3.1.5.5-1: Cell specific test parameters for NR SA FR1-FR2 asynchronous DAPS handover (Cell 1 in FR1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | NA Link only, see clause A.3.7A |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz |  |  |  |  |  |
|  | Config 3 |  | dBm/ 38.16MHz |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 7.3.1.5.5-2: Cell specific test parameters for NR SA FR1-FR2 asynchronous DAPS handover (Cell 2 in FR2)

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Duplex mode | Config 1,2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  |  |  |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| BWP BW | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR3.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR3.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | SSB.1 FR2 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 2 |  |  |  |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |  |  |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note2 |  | dBm/SCS | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
| IoNote3 |  | dBm/ 9.36MHz | -66.7 | -55.4 | -55.4 | -55.4 | -55.4 |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

Same Test requirements as described in clause 7.3.1.4.5.

### 7.3.2	RRC connection mobility control

#### 7.3.2.1	RRC re-establishment

##### 7.3.2.1.0	Minimum conformance requirements

In RRC_CONNECTED state the UE shall be capable of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay = TUE_re-establish_delay + TUL_grant

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay = 50ms + Tidentify_intra_NR +i=1Nfreq-1 i=1Nfreq-1Tidentify_inter_NR,i + TSI-NR + TPRACH

The intra-frequency target NR cell shall be considered detectable when for each relevant SSB can satisfy that:

-	SS-RSRP related side conditions given in Section 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band are fulfilled.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Section 10.1.4 and 10.1.5 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band are fulfilled.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

TSI-NR = It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

TPRACH = It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR (ms) |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < [-8] dB. |  |  |  |

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i (ms) |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 104 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 6.2.1.

##### 7.3.2.1.1	NR SA FR2 RRC re-establishment

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

7.3.2.1.1.1	Test purpose

The purpose of this test is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits.

7.3.2.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.1

7.3.2.1.1.4	Test description

7.3.2.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.3.2.1.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 7.3.2.1.1.4.1-2. Test environment parameters are given in Table 7.3.2.1.1.4.1-3.

Table 7.3.2.1.1.4.1-1: Intra-frequency RRC re-establishment in FR2 supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 7.3.2.1.1.4.1-2: General test parameters for Intra-frequency RRC re-establishment in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 [6]) |
| T3 |  | s | 1 | 3 |  |

Table 7.3.2.1.1.4.1-3: Test Environment Intra-frequency RRC re-establishment in FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5.1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Message contents are defined in clause 7.3.2.1.1.4.3.

2. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.3.2.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in RRC_CONNECTED state with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.1.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.1.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.1.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 3 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Set cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.2.1.1.4.3-1: Common Exception messages for NR intra-frequency RRC re-establishment test case in FR2

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-3 with Condition INTRA-FREQ MO and Synchronous cells |

Table 7.3.2.1.1.4.3-2: RLF-TimersAndConstants for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms5000 |  |  |
| } |  |  |  |

Table 7.3.2.1.1.4.3-3: MeasConfig for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

7.3.2.1.1.5	Test requirement

Table 7.3.2.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.2.1.1.5-1: Cell specific test parameters for NR intra-frequency RRC re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 |  |  | Setup 1 |  |  |
|  | dB | 1 | -3.07+TT | -infinity | -infinity | -5.07+TT | 2+TT | 2+TT |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89+TT |  |  |  |  |  |
|  | dB | 1 | 4+TT | -infinity | -infinity | 2+TT | 2+TT | 2+TT |
| SS-RSRP Note3 | dBm/SCS | 1 | -85+TT | -infinity | -infinity | -87+TT | -87+TT | -87+TT |
| Io | dBm/95.04 MHz | 1 | -52.94+TT | -55.89 | -55.89 | -52.94+TT | -55.89+TT | -55.89+TT |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 in Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell shall be less than 5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

##### 7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

7.3.2.1.2.1	Test purpose

The purpose of this test is to verify that the NR inter-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits.

7.3.2.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.2

7.3.2.1.2.4	Test description

7.3.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.3.2.1.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 7.3.2.1.2.4.1-2. Test environment parameters are given in Table 7.3.2.1.2.4.1-3.

Table 7.3.2.1.2.4.1-1: Inter-frequency RRC re-establishment in FR2 supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 7.3.2.1.2.4.1-2: General test parameters for Inter-frequency RRC re-establishment in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 [6]) |
| T3 |  | s | 1 | 6 |  |

Table 7.3.2.1.2.4.1-3: Test Environment Inter-frequency RRC re-establishment in FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table FFS and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Message contents are defined in clause 7.3.2.1.2.4.3

2. There are two cells on two NR carriers specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.3.2.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

1.	Ensure the UE is in RRC_CONNECTED state with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.2.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.2.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.2.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 6 seconds from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Set cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.2.1.2.4.3-1: Common Exception messages for NR intra-frequency RRC re-establishment test case in FR2

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-3 with Condition INTRA-FREQ MO and Synchronous cells |

Table 7.3.2.1.2.4.3-2: RLF-TimersAndConstants for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms5000 |  |  |
| } |  |  |  |

Table 7.3.2.1.2.4.3-3: MeasConfig for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

7.3.2.1.2.5	Test requirement

Table 7.3.2.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.2.1.2.5-1: Cell specific test parameters for NR intra-frequency RRC re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 5+TT | -infinity | -infinity | -infinity | -infinity | 8+TT |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89+TT |  |  |  |  |  |
|  | dB | 1 | 5+TT | -infinity | -infinity | -infinity | -infinity | 8+TT |
| SS-RSRP Note3 | dBm/SCS | 1 | -84+TT | -infinity | -infinity | -infinity | -infinity | -81+TT |
| Io | dBm/95.04 MHz | 1 | -53.82+TT | -infinity | -infinity | -infinity | -infinity | -51.37+TT |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

##### 7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Antenna diagram is TBD

-	Message content is TBD

7.3.2.1.3.1	Test purpose

The purpose of this test is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits, and to verify the requirements in TS 38.133 [6] clause 6.2.1

7.3.2.1.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.3

7.3.2.1.3.4	Test description

7.3.2.1.3.4.1	Initial conditions

The test shall be tested using any of the test configuration in Table 7.3.2.1.3.4.1-1.

Table 7.3.2.1.3.4.1-1: Supported test configurations for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Configure the test requirement and the DUT according to the parameters in Table 7.3.2.1.3.4.1-2.

Table 7.3.2.1.3.4.1-2: Initial conditions for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5.1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

1.	The general test parameter settings are set up according to Table 7.3.2.1.3.4.1-3.

2.	Message contents are defined in clause 7.3.2.1.3.4.3.

There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.3.2.1.3.4.1-3: General test parameters for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration index |  |  | 1 | PRACH.1 FR2 |  |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 10.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 [6]) |
| T3 |  | s | 1 | 5 |  |

7.3.2.1.3.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.3.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.3.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.3.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 5 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Set cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.2.1.3.4.3-1: Common Exception messages for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | FFS |
| Default RRC messages and information elements contents exceptions | FFS |

7.3.2.1.3.5	Test requirement

Table 7.3.2.1.3.5-1 defines the primary level settings including test tolerances for NR SA FR2 - FR2 RRC re-establishment without serving cell timing tests.

Table 7.3.2.1.3.5-1: Cell specific test parameters for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| TCI state |  | 1 | CSI-RS.Config.0 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A. 9 |  |  | Setup 1 defined in A.9 |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -93 | -infinity | -infinity | -infinity | -infinity | -93 |
| Io | dBm/95.04 MHz | 1 | -62.82 | -infinity | -infinity | -infinity | -infinity | -62.82 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 3520 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 4865 ms, allow 5 s in the test case.

#### 7.3.2.2	Random access

##### 7.3.2.2.0	Minimum conformance requirements

7.3.2.2.0.1	Minimum conformance requirements for Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-2 [3] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-2 [3] Tables 6.3.4.3-1 and 6.3.4.3-2.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2 in TS 38.321 [12].

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [12].

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2.

7.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-2 [3] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-2 [3] Tables 6.3.4.3-1 and 6.3.4.3-2.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2.

Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

##### 7.3.2.2.1	NR SA FR2 contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.1.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

7.3.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.1.

7.3.2.2.1.4	Test description

7.3.2.2.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.1.4.1-1.

Table 7.3.2.2.1.4.1-1: Contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.1-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.1.4.1-2.

Table 7.3.2.2.1.4.1-2: Test Environment for Contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.1.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.1.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 7.3.2.2.1.5-1 and 7.3.2.2.1.5-2.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

5.	Test 2: Correct behaviour when receiving Random Access Response

5.1. Repeat steps 1-3.

5.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

5.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

5.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

5.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

5.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.1.5.

6.	Test 3: Correct behaviour when not receiving Random Access Response

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

6.3. As no Random Access Response was received within the RA Response window, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.1.5.

7.	Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

7.1. Repeat steps 1-3.

7.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

7.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

7.4. The System Simulator shall send PDCCH addressed to the Temporary C-RNTI after receiving the msg3.

7.5. The UE shall re-transmit the msg3.

7.6. The System Simulator shall check if UE re-transmit the msg3.

8.	Test 5: Correct behaviour when receiving an unsuccessful UE Contention Resolution

8.1. Repeat steps 1-3.

8.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

8.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

8.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

8.5. As the UE Contention Resolution Identity included in the MAC control element did not match the CCCH SDU transmitted in the uplink message, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

8.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5.

9.	Test 6: Correct behaviour when receiving a successful UE Contention Resolution

9.1. Repeat steps 1-3.

9.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

9.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

9.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in msg3 uplink message.

9.5. As the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU, the Contention Resolution is successful and the UE shall send ACK.

10.	Test 7: Correct behaviour when contention Resolution timer expires

10.1. Repeat steps 1-3.

10.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

10.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

10.4. The System Simulator shall not send a response.

10.5. As there was no response, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the Contention Resolution Timer expires and then after the backoff timer expires.

10.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5.

7.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.1.4.3-1: FrequencyInfoUL-SIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-2: RACH-ConfigCommon for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  | FR2 |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-3: RACH-ConfigGeneric for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-4: ServingCellConfigCommonSIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

7.3.2.2.1.5	Test requirement

Table 7.3.2.2.1.5-2 defines the primary level settings for contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.1.5-3, 7.3.2.2.1.5-4 and 7.3.2.2.1.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.1.5-1: General test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.2 |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 of TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.1 FR2 | As defined in A.7.2, with exceptions as defined below |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 7.3.2.2.1.5-2: OTA-related test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  |  | Rough |  |
| SSB with index 0 | Es Note1 |  | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP |  | dBm/SCS | -80.6 |  |
|  | Es/IotBB |  | dB | 21.09 |  |
|  | Io |  | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 |  | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | SSB_RP | dBm/SCS | -95.0 |  |
|  |  | Es/IotBB | dB | 6.69 |  |
|  |  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

Test 2: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.1.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 3: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.1.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

-	The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

Test 5: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires.

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 6: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 7: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if the contention resolution timer expires.

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Table 7.3.2.2.1.5-3 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.1.5-4 Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.1.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 7.3.2.2.2	NR SA FR2 non-contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

7.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble.

7.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.2.

7.3.2.2.2.4	Test description

7.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.2.4.1-1.

Table 7.3.2.2.2.4.1-1: Non-Contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.2-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.2.4.1-2.

Table 7.3.2.2.2.4.1-2: Test Environment for Non-Contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.2.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.2.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 1.

3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3

6.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.2.4.3-1: FrequencyInfoUL-SIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-2: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | Not present |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-3: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 2 entries |  |  |
| ssb[1] | 0 |  |  |
| ssb[2] | 1 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-4: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-5: ServingCellConfigCommonSIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

7.3.2.2.2.5	Test requirement

Table 7.3.2.2.2.5-2 defines the primary level settings for non-contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.2.5-3, 7.3.2.2.2.5-4 and 7.3.2.2.2.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS Configuration | Config 1 |  | N/A | CSI-RS.3.1 TDD | As defined in A.1.4 |
| Duplex Mode for Cell 2 | Config 1 |  | TDD | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | OP.3 | As defined in A.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | SR3.1 TDD | As defined in A.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.2 FR2 | PRACH.3 FR2 | As defined in A.7.2, with exceptions as defined below. |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table 7.3.2.2.2.5-2: OTA-related test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Table 7.3.2.2.2.5-3 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.2.5-4 Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.2.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

#### 7.3.2.3	RRC connection release with redirection

### 7.3.3	Conditional handover

#### 7.3.3.0	Minimum conformance requirements

##### 7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover

[TS 38.133, clause 6.1.4.4.1]

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [13].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

Tmeasure is the measurements time stated in TS 38.133 [6] clause 6.1.4.4.2.

TCHO_execution is the conditional execution preparation time in TS 38.133 [6] clause 6.1.4.4.3.

Tinterrupt is the interruption time stated in TS 38.133 [6] clause 6.1.4.4.4.

[TS 38.133, clause 6.1.4.4.2]

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in TS 38.133 [6] clause 9.2.5.1 or clause 9.2.6.2.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 6.1.4.4.3]

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

[TS 38.133, clause 6.1.4.4.4]

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [13] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.[TS 38.133, clause 9.2.2]

The requirements in TS 38.133 [6] clause 9.2 apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in  TS 38.133 [6] clauses 10.1.3 for FR2, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133, clause 9.2.5.1]

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in TS38.133 [6] Table 9.2.5.1-2

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in TS38.133 [6] Table 9.2.5.2-2

CSSFintra: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2 power class 1, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps =24

Mmeas_period_w/o_gaps : For a UE supporting power class 1, Mmeas_period_w/o_gaps =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 9.2.5.1]

The measurement period for intra-frequency measurements without gaps is as shown in 38.133 [6] Table 9.2.5.2-2.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps(FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 10.1.3.1.1]

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in TS 38.133 [6] Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [3] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [3].

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in TS 38.133 [6] Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [3]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

[TS 38.133, annex B.2.2]

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in TS 38.133 [6] Table B.2.2-2 for FR2 NR cells.

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |  |
|  |  | n262 | -123.3+Y1 | -108,6 | -106.6 | -121.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |  |
|  |  | n262 | -115.1+Z1 | -96.7 | -93.5 | -109.7+Z4 |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [3]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [3]. |  |  |  |  |  |  |  |  |  |

##### 7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover

[TS 38.133, clause 6.1.4.4.1]

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [13].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

Tmeasure is the measurements time stated in TS 38.133 [6] clause 6.1.4.4.2.

TCHO_execution is the conditional execution preparation time in TS 38.133 [6] clause 6.1.4.4.3.

Tinterrupt is the interruption time stated in TS 38.133 [6] clause 6.1.4.4.4.

[TS 38.133, clause 6.1.4.4.2]

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in TS 38.133 [6] clause 9.2.5.1 or clause 9.2.6.2.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 6.1.4.4.3]

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

[TS 38.133, clause 6.1.4.4.4]

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [13] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

[TS 38.133, clause 9.3.2]

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] clauses 10.1.5 for FR2, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.3 for a corresponding Band.

[TS 38.133, clause 9.3.4]

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in 38.133 [6] Table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in 38.133 [6] Table 9.3.4-4.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in 38.133 [6] Table 9.3.5-2.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync_inter = 40 samples.

MSSB_index_inter: For a UE supporting FR2 power class 1, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 4, MSSB_index_inter = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1, Mmeas_period_inter =64 samples. For a UE supporting FR2 power class 2, Mmeas_period_inter=40 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 40 samples.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Table 9.3.4-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_inter Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Mpss/sss_sync_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-4: Time period for time index detection (Frequency range FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_inter Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | MSSB_index_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133, clause 9.3.5]

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively,  as shown in TS 38.133 [6] Table and 9.3.5-2:

Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (Frequency FR2)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_inter Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Mmeas_period_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133, clause 10.1.5.1.1]

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in TS 38.133 [6] Table 10.1.5.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [3] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [3].

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [3]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

[TS 38.133, annex B.2.3]

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |  |
|  |  | n262 | -121.3+Y1 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |  |
|  |  | n262 | -113.1+Z1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [3]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [3]. |  |  |  |  |  |  |  |  |  |

#### 7.3.3.1	NR SA FR2 conditional handover

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

7.3.3.1.1	Test purpose

To verify the requirement for the NR FR2 intra frequency conditional handover requirements specified in 38.133 [6] clause 6.1.4.4.

7.3.3.1.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards and supporting conditional handover.

7.3.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.3.1.

7.3.3.1.4	Test description

7.3.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.3.1.4.1-1.

Table 7.3.3.1.4.1-1: NR SA FR2 conditional handover test configurations

| Config | Description |
| --- | --- |
| 7.3.3.1-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.3.1.4.1-2

Table 7.3.3.1.4.1-2: Initial conditions for NR SA FR2 conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.3.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.3.1.4.1-3 below, with A3-Offset modified by Test Tolerance.

Table 7.3.3.1.4.1-3: General test parameters for NR SA FR2 conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset for condition |  | dBm | -1 | Trigger HO to cell which may be measured as -1dB relative to cell 1. Actual SS-RSRP is 5dB stronger. |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

7.3.3.1.4.2	Test procedure

The test scenario comprises of 1 NR carrier and two cells, Cell 1 and Cell 2, on this carrier. General parameters and Cell-specific parameters for Cell 1 and Cell 2 are given in Table 7.3.3.1.4.1-3 and 7.3.3.1.5-1 respectively. No measurement gap is configured in the test case.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

At the start of time duration T1, the UE may not have any timing information of cell 2. The UE is configured with a condition implying handover to Cell 2 at a time earlier than TRRC ms before the beginning of T2. No interruption shall be observed in time period T1. where,

-	TRRC = 10ms, is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

From start of T2, the Cell 2 becomes detectable and handover condition is satisfied. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition, and starts handover procedure when execution condition is satisifed. The UE shall sent PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.3.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedbacks sent by the UE.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2).

4.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure CHO execution condition for the UE.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.3.1.5-1. T2 starts.

7.	If

- the UE transmits the PRACH preambles to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the beginning of time period T2,

and

- no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = 8∙Tinterrupt for test configuration 7.3.3.1-1

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

9.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.3.1.4.3-1: RRCReconfiguration (Step 4)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 7.3.3.1.4.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 7.3.3.1.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.3.1.4.3-2: MeasConfig (Table 7.3.3.1.4.3-1)

| Derivation Path: Table H.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.3.3.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 7.3.3.1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig specified in Table H.3.1-5 |  |  |
| } |  |  |  |

Table 7.3.3.1.4.3-3: MeasObjectNR (Table 7.3.3.1.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for PCell |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1[14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 7.3.3.1.4.3-4: ReportConfigNR (Table 7.3.3.1.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-142 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -2 | actuall value = -2*0.5 = -1dB |  |
| } |  |  |  |
| hysteresis | 0 | actuall value = 0*0.5 = 0dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.3.1.4.3-5: ConditionalReconfiguration (Table 7.3.3.1.4.3-1)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16 [1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | The MeasId configured in Table 7.3.3.1.4.3-2 |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTET STRING (CONTAINING RRCReconfiguration Specified in Table 4.8.1-1A with condition RBConfig_NoKeyChange) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.3.3.1.5	Test requirements

Table 7.3.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.3.1.5-1: Cell specific test parameters for NR SA FR2 conditional handover

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  |  |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -98.7 |  |  | -98.7 |  |  |  |
|  |  |  | dB | 6 | -5.33 |  | -Infinity |  | 4.02 |  |
|  |  |  | dB | 6 | 6 |  | -Infinity |  | 11 |  |
| IoNote3 | Config 1 |  | dBm/ BW | -62.7 | -57.2 |  | -62.7 |  | -57.2 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

The rate of correct handovers observed during repeated tests shall be at least 90%.

The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt from the start of T2. where:

-	Tmeasure = 1600 ms for power class 1 UE and 1080ms for power class 2/3/4 UE, is the measurements time specified in 38.133 [6] clause 6.1.4.2.2.

-	TCHO_execution = 10 ms, is the conditional execution preparation time specified in 38.133 [6] clause 6.1.4.2.2.

-	Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms, is the interruption time specified in 38.133 [6] 6.1.4.2.4.

-	Tprocessing = 20 ms, is time for UE processing;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell;

-	T∆ = 20 ms, is time for fine time tracking and acquiring full timing information of the target cell;

-	Tmargin = 2 ms, is time for SSB post-processing.

This gives a total of 1672 ms for power class 1 UE and 1152 ms for power class 2/3/4 UE.

The interruption during T2 shall not exceeed Tinterrupt = 62ms.

#### 7.3.3.2	NR SA FR2-FR2 conditional handover

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Some test parameters are still FFS

7.3.3.2.1	Test purpose

To verify the requirement for the NR FR2 inter-frequency conditional handover requirements specified in 38.133 [6] clause 6.1.4.4.

7.3.3.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards and supporting conditional handover.

7.3.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.3.2.

7.3.3.2.4	Test description

7.3.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.3.2.4.1-1.

Table 7.3.3.2.4.1-1: NR SA FR2-FR2 conditional handover test configurations

| Config | Description |
| --- | --- |
| 7.3.3.2-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.3.2.4.1-2

Table 7.3.3.2.4.1-2: Initial conditions for NR SA FR2-FR2 conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.3.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.3.2.4.1-3 below, with A3-Offset modified by Test Tolerance.

Table 7.3.3.2.4.1-3: General test parameters for NR SA FR2-FR2 conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset for handover condition |  | dB | FFS |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 7 |  |

7.3.3.2.4.2	Test procedure

The test scenario comprises of 2 NR carriers and two cells, Cell 1 and Cell 2, on each carrier respectively. General parameters and Cell-specific parameters for Cell 1 and Cell 2 are given in Table 7.3.3.2.4.1-3 and 7.3.3.2.5-1 respectively. Measurement gap (gap pattern #0) is configured in the test case.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

At the start of time duration T1, the UE may not have any timing information of cell 2. The UE is configured with a condition implying handover to Cell 2 at a time earlier than TRRC ms before the beginning of T2. No interruption shall be observed in time period T1. where,

-	TRRC = 10ms, is the RRC procedure delay defined in clause 12 in TS 38.331 [13].

From start of T2, the Cell 2 becomes detectable and handover condition is satisfied. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition, and starts handover procedure when execution condition is satisifed. The UE shall sent PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.3.2.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedbacks sent by the UE.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2).

4.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure CHO execution condition and measurement gap pattern #0 for the UE.

5.	The UE shall transmit an RRCReconfigurationComplete message.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.3.2.5-1. T2 starts.

7.	If

- the UE transmits the PRACH preambles to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the beginning of time period T2,

and

- no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = 8∙Tinterrupt for test configuration 7.3.3.2-1

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

9.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.3.2.4.3-1: RRCReconfiguration (Step 4)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 7.3.3.2.4.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 7.3.3.2.4.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.3.2.4.3-2: MeasConfig (Table 7.3.3.2.4.3-1)

| Derivation Path: Table H.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 2 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 7.3.3.2.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 7.3.3.2.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 7.3.3.2.4.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  |  |
| quantityConfig | QuantityConfig specified in Table H.3.1-5 |  |  |
| } |  |  |  |

Table 7.3.3.2.4.3-3: MeasObjectNR-f1 (Table 7.3.3.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for Cell 1 |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1[14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 7.3.3.2.4.3-4: MeasObjectNR-f2 (Table 7.3.3.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for Cell 2 |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1[14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 7.3.3.2.4.3-5: ReportConfigNR (Table 7.3.3.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-142 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | FFS |  |  |
| } |  |  |  |
| hysteresis | 0 | actuall value = 0*0.5 = 0dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.3.2.4.3-6: ConditionalReconfiguration (Table 7.3.3.2.4.3-1)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16 [1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | The MeasId configured in Table 7.3.3.2.4.3-2 |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTET STRING (CONTAINING RRCReconfiguration Specified in Table 4.8.1-1A with condition RBConfig_NoKeyChange) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.3.3.2.5	Test requirements

Table 7.3.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.3.2.5-1: Cell specific test parameters for NR SA FR2-FR2 conditional handover

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  |  |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | gp0 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -95.7 |  |  | -95.7 |  |  |  |
|  | Config 3 |  |  | -95.7 |  |  | -95.7 |  |  |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
| IoNote3 | Config 1,2 |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
|  | Config 3 |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

The rate of correct handovers observed during repeated tests shall be at least 90%.

The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt from the start of T2. where:

-	Tmeasure = 6720 ms for power class 1 UE and 4160 ms for power class 2/3/4 UE, is the measurements time specified in 38.133 [6] clause 6.1.4.2.2.

-	TCHO_execution = 10 ms, is the conditional execution preparation time specified in 38.133 [6] clause 6.1.4.2.2.

-	Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms, is the interruption time specified in 38.133 [6] 6.1.4.2.4.

-	Tprocessing = 20 ms, is time for UE processing;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell;

-	T∆ = 20 ms, is time for fine time tracking and acquiring full timing information of the target cell;

-	Tmargin = 2 ms, is time for SSB post-processing.

This gives a total of 6792 ms for power class 1 UE and 4232 ms for power class 2/3/4 UE.

The interruption during T2 shall not exceeed Tinterrupt = 62ms.

## 7.4	Timing

### 7.4.1	UE transmit timing

### 7.4.2	UE timer accuracy

### 7.4.3	Timing advance

## 7.5	Signalling characteristics

### 7.5.1	Radio link monitoring

#### 7.5.1.0	Minimum conformance requirements

##### 7.5.1.0.1

##### 7.5.1.0.2

##### 7.5.1.0.3

##### 7.5.1.0.4

##### 7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring

[TS 38.133, clause 8.1.7.3]

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell and/or PSCell.

-	If the RLM-RS is CSI-RS which is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON,

-	There are no scheduling restrictions due to radio link monitoring based on the CSI-RS.

-	Otherwise

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for RLM; and

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for RLM.

#### 7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Message content is TBD

7.5.1.9.1	Test purpose

The purpose of this test is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in TS 38.133 [6] clause 8.1.7, and to verify that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly.

7.5.1.9.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

This test is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

7.5.1.9.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.5.1.0.5.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.1.9.

7.5.1.9.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of one time period with time duration of T1. Figure 5.5.1.9.4-1 shows the Time multiplexed downlink transmissions from each Angle of Arrival. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Figure 7.5.1.9.4-1: Time multiplexed downlink transmissions

7.5.1.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.1.9.4.1-1.

Table 7.5.1.9.4.1-1: Supported test configurations for NR SA FR2 radio link monitoring UE scheduling restrictions

| Configuration | Description |
| --- | --- |
| 7.5.1.9-1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.1.9.4.1-2

Table 7.5.1.9.4.1-2: Initial conditions for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.1.9.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters for PCell are given in Table 7.5.1.9.4.1-3

2.	Message contents are defined in clause 7.5.1.9.4.3.

3.	There is one carrier and one cell specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.1.9.4.1-3: General test parameters for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1 | 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC pattern 1 |  |
| DRX cycle length | s | 1 | OFF |  |
| T1 | s | 1 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

7.5.1.9.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. During the test PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending.

1. Set the parameters according to T1 in Table 7.5.1.9.4.4-1. Propagation conditions are set according to Annex C.2.2. T1 starts.

2. If the SS receives ACK/NACK on each UL transmission occasion scheduled by PDCCH which are not overlapped with SSBs configured for radio link monitoring during T1, the number of successful tests is increased by one. otherwise the number of failed tests is increased by one.

3. The UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5.

4. Repeat steps 1-3 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.1.9.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.1.9.4.3-1: Common Exception messages for NR SA FR2 radio link monitoring UE scheduling restrictions

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.5.1.9.5	Test Requirement

Table 7.5.1.9.4.1-3 and 7.5.1.9.5-1 define the primary level settings including test tolerances for NR SA FR2 radio link monitoring UE scheduling restrictions.

Table 7.5.1.9.5-1: Cell specific test parameters for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
| AoA setup |  | 1 | Setup 3 defined in A.9.3 |  |
|  |  |  | AoA1 | AoA2 |
| Assumption for UE beams Note 1 |  |  | Rough | Rough |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  |
| PDSCH Reference measurement channel |  | 1 | SR.3.2 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1 | TCI.State.2 | N/A |
| OCNG Pattern |  | 1 | OP.5 | Not sent |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |
| RLM-RS |  | 1 | SSB with index 0 | SSB with index 1 |
|  | dBm/15kHz | 1 | -92.1 | -92.1 |
| Note2 | dBm/SCS | 1 | -83.1 | -83.1 |
|  | dB | 1 | 2 | 2 |
| BB Note 4 | dB | 1 | 1 | 1 |
| SSB_RP Note3 | dBm/SCS | 1 | -81.1 | -81.1 |
| Io | dBm/95.04 MHz | 1 | -54.35 | -54.35 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1 | Defined in Figure 7.5.1.9.4-1 |  |
| Propagation Condition |  | 1 | AWGN | AWGN |
| Note 1: Information about types of UE beam is given in TS 38.133 [6] B.2.1.3 and does not limit UE implementation or test system implementation. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |

The UE behaviour during time duration T1 follows the requirements defined in TS 38.133 [6] clause 8.1.7.3:

The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 7.5.2	Interruption

### 7.5.3	SCell activation and deactivation delay

#### 7.5.3.0	Minimum conformance requirements

##### 7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [8]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the SCell measurement cycle is equal to or smaller than 160ms.

-	TFirstSSB_MAX + Trs + 5ms, if the SCell measurement cycle is larger than 160ms.

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in TS 38.133 [6] clause 3.6.3,

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max (Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

If the PCell/PSCell and the target SCell are in a band pair with independent beam management and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

If the PCell/PSCell and the target SCell are in a band pair with independent beam management and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where,

TSMTC_MAX:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms

TFirstSSB: is the time to the end of the first completed SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length

TFirstSSB_MAX: Is the time to the end of the first completed SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB (ms) or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in TS 38.133 [6] clause 9.5 assuming M=1.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

Longer dekays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max (5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in TS 38.133 [6] clause 9.2 and 9.3.

-	the SSB measured during the period equal to max (5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in TS 38.133 [6] clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in TS 38.133 [6] clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [13] signalling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [13] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The interruption on PSCell or any activated SCell in SCG for EN-DC mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+ and not occur after slot n+1+.

The interruption  on PCell or any activated SCell in MCG for NR standalone mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+ and not occur after slot n+1+.

Starting from the slot specified in TS 38.213 [8] clause 4.3 (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in TS 38.213 [8] clause 4.3 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.2.

##### 7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell

Upon receiving SCell deactivation command or upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n+.

The interruption on PSCell or any activated SCell in SCG for EN-DC mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+[] and not occur after slot n+1+.

The interruption on PCell or any activated SCell in MCG for NR standalone mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+[] and not occur after slot n+1+.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.3.

#### 7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.3.1.1	Test purpose

The purpose of this test is:

- To verify the requirement for the SCell activation and deactivation times are within the requirements specified in TS 38.133 [6] clause 8.3, when the PCell and SCell are is in FR2 intra-band and SCell is known by the UE at the time of activation.

7.5.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

7.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.3.0.1 and 7.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.3.1.

7.5.3.1.4	Test description

7.5.3.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.3.1.4.1-1.

Table 7.5.3.1.4.1-1: Supported test configurations for NR SA FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 7.5.3.1-1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.3.1.4.1-2.

Table 7.5.3.1.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.3.1.4.1-3.

2. Message contents are defined in clause 7.5.3.1.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is SCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.3.1.4.1-3: General test parameters for NR SA FR2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test, cell 1 and cell2 use RF channel 1 and 2, respectively. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | slot | k | k is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. |

7.5.3.1.4.2	Test procedure

TBD

7.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.3.1.5	Test requirement

TBD

#### 7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.3.2.1	Test purpose

The purpose of this test is:

- To verify the requirement for the SCell activation and deactivation times are within the requirements specified in TS 38.133 [6] clause 8.3, when the PCell is in FR1 and SCell is in FR2 and SCell is known by the UE at the time of activation.

7.5.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

7.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.3.0.1 and 7.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.3.2.

7.5.3.2.4	Test description

7.5.3.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.3.2.4.1-1.

Table 7.5.3.2.4.1-1: Supported test configurations for NR SA FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 7.5.3.2-1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.5.3.2-2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.5.3.2-3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.3.2.4.1-2.

Table 7.5.3.2.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.3.2.4.1-3.

2. Message contents are defined in clause 7.5.3.2.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is SCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.3.2.4.1-3: General test parameters for NR SA FR2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 2 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | slot | k | k is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. |

7.5.3.2.4.2	Test procedure

TBD

7.5.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.3.2.5	Test requirement

TBD

### 7.5.4	UE UL carrier RRC reconfiguration delay

### 7.5.5	Link recovery procedures

#### 7.5.5.0	Minimum conformance requirements

##### 7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB [ms] period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB [ms] period.

The value of TEvaluate_BFD_SSB is defined in Table 7.5.5.0.1-1 for FR2 with N=8

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TSSB <MGRP) and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 3.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 7.5.5.0.1-1: Evaluation period TEvaluate_BFD_out for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | max([50], ceil(5*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max([50], ceil(7.5*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(5*P*N)*TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB [ms] period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The value of TEvaluate_CBD_SSB is defined in Table 7.5.5.0.1-2 for FR2 with N=8.

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is 3, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)*3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TSSB /MGRP)*3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it is assumed that the SSB configured for candidate beam detection and each CSI-RS resource shall be TDMed transmitted.

In FR2, it is assumed that the SSB configured for candidate beam detection and each CSI-RS resource shall be TDMed transmitted.

Table 7.5.5.0.1-2: Evaluation period TEvaluate_CBD_out for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | ceil([3]*P*N) * TSSB |
| DRX cycle ≤ 320ms | ceil([3]*P*N*1.5) * max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil([3]*P*N) * TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.2.2, 8.5.4 and 8.5.5.2.

##### 7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS [ms] period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS [ms] period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 7.5.5.0.2-1 for FR2 with N=1

Editor’s Note: It is FFS if a CSI-RS resource in the resource set with repetition “ON” can be configured as a BFD-RS. If CSI-RS for BFD can be in the resource set with repetition “ON”, N=8 may apply.

Editor’s Note: FFS if there are other conditions with N=8.

For FR2,

-	P=1, when BFD-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when BFD-RS is partially overlapped with measurement gap and BFD-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

NOTE: The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of MBFD used in Table 7.5.5.0.2-1 is defined as

-	MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3.

Table 7.5.5.0.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | max([50], [MBFD *P*N] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD*P*N]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD *P*N] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The value of TEvaluate_CBD_CSI-RS is defined in Table 7.5.5.0.2-2 for FR2 with N=8.

Editor’s Note: FFS whether N=1 need to be applied for CSI-RS based candidate beam detection in FR2.

For FR2,

-	P=1, when candidate beam detection RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is 3, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* 3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TCSI-RS /MGRP)* 3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP) [Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM/BFD/BM-RS, or other CBD-RS, according to the measurement restrictions defined in section TBD.]

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it iss assumed that the CSI-RS configured for candidate beam detection and each SSB shall be TDMed transmitted.

In FR2, It is assumed that the CSI-RS configured for candidate beam detection with N=1 shall be TDMed with any RS resources configured for RLM/BFD/CBD/L1-RSRP reporting which is not is QCL-Type D with this CSI-RS resource or under the conditions of N>1 as specified in TS 38.133 [6] section 8.1.2.2, 8.1.2.3, 8.5.2.2, 8.5.2.3, 8.5.2.5, 8.5.2.6, 9.5.4.1 and 9,5,4,2.

The values of MCBD used in Table 7.5.5.0.2-2 is defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Table 7.5.5.0.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([25], ceil(MCBD *P*N) * TCSI-RS) |
| DRX cycle ≤ 320ms | ceil(MCBD *P*N*1.5) * max(TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceil(MCBD *P*N) *TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.3.2, 8.5.4 and 8.5.6.2.

#### 7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.1.

7.5.5.1.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.1.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 7.5.5.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.1.4-1: SNR and L1-RSRP variation for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

7.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.1.4.1-1.

Table 7.5.5.1.4.1-1: Supported test configurations for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.1.4.1-2.

Table 7.5.5.1.4.1-2: Initial conditions for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.1.4.1-3. The measurement gap configuration is according to Table 7.5.5.1.4.1-4.

2. Message contents are defined in clause 7.5.5.1.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.1.4.1-3: General test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1, 2 |  |  |  | TDD |  |
| BWchannel |  | Config 1, 2 |  |  |  | 100: NRB,c = 66 |  |
| DL initial BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1, 2 |  |  |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  | Config 1, 2 |  |  |  | CR. 3.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  |  | SSB.1 FR2 |  |
|  |  | Config 2 |  |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  | Config 1, 2 |  |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  |  | 120 KHz |  |
| PRACH Configuration |  | Config 1, 2 |  |  |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| TCI Configuration |  | Config 1, 2 |  |  |  | TBD |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0.. |
| rsrp-ThresholdSSB |  |  | Config 1 |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
|  |  |  | Config 2 |  |  | TBD |  |
| powerControlOffsetSS |  |  |  |  |  |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD |  |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 2.61 |  |
| T3 |  |  |  |  | s | 1.64 |  |
| T4 |  |  |  |  | S | 0 |  |
| T5 |  |  |  |  | s | 1.01 |  |
| D1 |  |  |  |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table 7.5.5.1.4.1-4: Void

7.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.1.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.1.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.1.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.1.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.1.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.1.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 for until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.1.4.3-1: Common Exception messages for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-11 |

7.5.5.1.5	Test requirement

Tables 7.5.5.1.4.1-3 and 7.5.5.1.5-1 define the primary level settings including test tolerances for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.1.5-1: NR Cell specific test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
|  | Config 2 |  | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE hich supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 960+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.2.

7.5.5.2.4	Test description

There are one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.2.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 7.5.5.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.2.4-1: SNR and L1-RSRP variation for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

7.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.2.4.1-1.

Table 7.5.5.2.4.1-1: Supported test configurations for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.2.4.1-2.

Table 7.5.5.2.4.1-2: Initial conditions for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.2.4.1-3. The measurement gap configuration is according to Table 7.5.5.2.4.1-4. The DRX configuration is according to Table 7.5.5.2.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2. Message contents are defined in clause 7.5.5.2.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.2.4.1-3: General test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1, 2 |  |  | TDD |  |
| BWchannel |  |  | Config 1, 2 |  |  | 100: NRB,c = 66 |  |
| DL initial BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1, 2 |  |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  |  | Config 1, 2 |  |  | CR. 3.1 TDD |  |
| SSB Configuration |  |  | Config 1 |  |  | SSB.1 FR2 |  |
|  |  |  | Config 2 |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  |  | Config 1, 2 |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  |  | 120 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| TCI Configuration |  |  | Config 1, 2 |  |  | TBD |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | DRX.3 |  |
| Gap pattern ID |  |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1 |  |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
|  |  | Config 2 |  |  |  | TBD |  |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD |  |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 3.37 |  |
| T3 |  |  |  |  | s | 2.8 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.61 |  |
| D1 |  |  |  |  | s | 0.57 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table 7.5.5.2.4.1-4: Void

7.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.2.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.2.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.2.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.2.4.3-1: Common Exception messages for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-11 Table H.3.7-1 with condition DRX.7 |

7.5.5.2.5	Test requirement

Tables 7.5.5.2.4.1-3 and 7.5.5.2.5-1 define the primary level settings including test tolerances for NR SA FR2 SSB-based beam failure detection and link recovery in DRX.

Table 7.5.5.2.5-1: NR Cell specific test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -12 | -12 | -12 | -3 | 10 |
|  | Config 2 |  | -12 | -12 | -12 | -3 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
|  | Config 2 |  | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 560+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting CSI-RS-based RLM and link recovery  .

7.5.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.3.

7.5.5.3.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.3.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure. Figure 7.5.5.3.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.3.4-1: SNR and L1-RSRP variation for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

7.5.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.3.4.1-1.

Table 7.5.5.3.4.1-1: Supported test configurations for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.3.4.1-2.

Table 7.5.5.3.4.1-2: Initial conditions for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.3.4.1-3. The NZP-CSI-RS configuration is according to Table 7.5.5.3.4.1-3.

2. Message contents are defined in clause 7.5.5.3.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.3.4.1-3: General test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.3 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120KHz |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | NA | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |  |
| csi-RS-Index assigned as RLM RS |  |  | 0, 1 |  |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 1.17 |  |
| T3 |  | s | 0.9 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

7.5.5.3.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is not enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.3.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.3.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.3.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.3.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.3.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.3.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.3.4.3-1: Common Exception messages for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTRA-FREQ and L3 FILTERING NEEDED; Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-11 |

7.5.5.3.5	Test requirement

Tables 7.5.5.3.4.1-3 and 7.5.5.3.5-1 define the primary level settings including test tolerances for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.3.5-1: NR Cell specific test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/15 KHz | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.3.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 =260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and CSI-RS-based RLM and link recovery and long DRX cycle.

7.5.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.4.

7.5.5.4.4	Test description

There is one NR serving cell configured in this test. This testconsists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.4.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure. Figure 7.5.5.4.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.4.4-1: SNR and L1-RSRP variation for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

7.5.5.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.4.4.1-1.

Table 7.5.5.4.4.1-1: Supported test configurations for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 7.5.5.4-1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.4.4.1-2.

Table 7.5.5.4.4.1-2: Initial conditions for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.4.4.1-3. The measurement gap configuration is according to Table 7.5.5.4.4.1-4. The NZP-CSI-RS configuration is according to Table 7.5.5.4.4.1-3. The DRX configuration for is according to Table 7.5.5.4.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2. Message contents are defined in clause 7.5.5.4.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.4.4.1-3: General test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.3 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | DRX.3 |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |  |
| csi-RS-Index assigned as RLM RS | Config 1 |  | CSI-RS.3.2 TDD |  |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table 7.5.5.4.4.1-4: Void

7.5.5.4.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.4.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.4.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.4.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.4.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.4.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.4.5-1.

9.	Wait [1s] for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within [1s] continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.4.4.3-1: Common Exception messages for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-11; Table H3.7-1 with Condition DRX.7 |

7.5.5.4.5	Test requirement

Tables 7.5.5.4.4.1-3 and 7.5.5.4.5-1 define the primary level settings including test tolerances for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX.

Table 7.5.5.4.5-1: NR Cell specific test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.4.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.5.1	Test purpose

The purpose of this test is to test scheduling availability restrictions when the UE is performing beam failure detection or when the UE is performing L1-RSRP measurement for candidate beam detection, when no DRX is used, and to verify the scheduling availability restriction requirements for SSB based beam failure detection and link recovery for an FR2 serving cell in TS 38.133 [6] clause 8.5.7 and 8.5.8.

7.5.5.5.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.5.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.5.

7.5.5.5.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.5.4-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure 7.5.5.5.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.5.4-1: SNR and L1-RSRP variation for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

7.5.5.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.5.4.1-1.

Table 7.5.5.5.4.1-1: Supported test configurations for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 7.5.5.5-1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.5.4.1-2.

Table 7.5.5.5.4.1-2: Initial conditions for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.5.5.4.1-3.

2.	Message contents are defined in clause 7.5.5.5.4.3.

3.	There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.5.4.1-3: General test parameters for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |
| CORESET Reference Channel | Config 1 |  | CR. 3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| PRACH Configuration | Config 1 |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  | 1 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | TCI.State.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| AoA Setup |  |  | Setup 1 | A.9 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF | DRX is not in use |
| Gap pattern ID |  |  | N.A. | No measurement gap pattern is configured |
| ssb-Index |  |  | 2 | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the 10% |
| rsrp-ThresholdSSB |  | dBm/SCS KHz | -94.5 | Threshold used for Qin_LR |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI Configuration for reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 2.6 |  |
| T3 |  | s | 1.64 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 1.01 |  |
| D1 |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

7.5.5.5.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. This test will focus on the scheduling availability during beam failure detection and candidate beam detection. In the test, DRX configuration is not enabled. During the test the UE is scheduled to transmit continuously in UL.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.5.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.5.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.5.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.5.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.5.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) which are not overlapped with SSBs configured for beam failure detection during the period from time point B to time point D

and

b) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point D until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.5.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.5.5.4.3-1: Common Exception messages for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.5.5.5	Test requirement

Table 7.5.5.5.4.1-3 and 7.5.5.5.5-1 define the primary level settings including test tolerances for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.5.5-1: NR Cell specific test parameters for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| SNR_SSB of set q1 | Config 1 | dB | -12 | -12 | 5 | 5 | 5 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.5.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time duration T3 follows the requirements defined in TS 38.133 [6] clause 8.5.7.3:

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on BFD-RS symbols to be measured for beam failure detection.

The UE behaviour during time durations T4 and T5 follows the requirements defined in TS 38.133 [6] clause 8.5.8.3:

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on reference symbols to be measured for candidate beam detection.

### 7.5.6	Active BWP switch delay

#### 7.5.6.1	DCI-based and time-based active BWP switch

##### 7.5.6.1.0	Minimum conformance requirements

[TS 38.133, clause 8.6.2]

The requirements in this clause only apply to the case that the BWP switch is performed on a single CC.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after the beginning of DL slot n+ TBWPswitchDelay.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 7.5.6.1.0-1.

Table 7.5.6.1.0-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in Clause 8.10 in the new BWP.

##### 7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.1.1	Test purpose

FFS

7.5.6.1.1.2	Test applicability

FFS

7.5.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.1.

7.5.6.1.1.4	Test description

7.5.6.1.1.4.1	Initial conditions

FFS

7.5.6.1.1.4.2	Test procedure

FFS

7.5.6.1.1.4.3	Message contents

FFS

7.5.6.1.1.5	Test requirements

FFS

##### 7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.2.1	Test purpose

FFS

7.5.6.1.2.2	Test applicability

FFS

7.5.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.2.

7.5.6.1.2.4	Test description

7.5.6.1.2.4.1	Initial conditions

FFS

7.5.6.1.2.4.2	Test procedure

FFS

7.5.6.1.2.4.3	Message contents

FFS

7.5.6.1.2.5	Test requirements

FFS

##### 7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.3.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6

7.5.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.3.

7.5.6.1.3.4	Test description

There is one cell configured in this test. Cell 1 is PCell. This test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

7.5.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.6.1.3.4.1-1.

Table 7.5.6.1.3.4.1-1: Supported test configurations for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 7.5.6.1.3-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: A UE which fulfils the requirements in test case 7.5.6.1.1 or 7.5.6.1.2 can skip the test cases in 7.5.6.1.3. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.6.1.3.4.1-2.

Table 7.5.6.1.3.4.1-2: Initial conditions for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.5.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.6.1.3.4.1-3.

2.	Message contents are defined in clause 7.5.6.1.3.4.3.

3.	There are one NR cell specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.6.1.3.4.1-3: General test parameters for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| bwp-InactivityTimer | ms | [200] |  |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

7.5.6.1.3.4.2	Test procedure

FFS

7.5.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.6.1.3.4.3-1: Common Exception messages for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.6.1.3.5	Test requirements

FFS

#### 7.5.6.2	RRC-based active BWP switch

##### 7.5.6.2.0	Minimum conformance requirements

FFS

##### 7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.2.1.1	Test purpose

FFS

7.5.6.2.1.2	Test applicability

FFS

7.5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.2.1.

7.5.6.2.1.4	Test description

7.5.6.2.1.4.1	Initial conditions

FFS

7.5.6.2.1.4.2	Test procedure

FFS

7.5.6.2.1.4.3	Message contents

FFS

7.5.6.2.1.5	Test requirements

FFS

### 7.5.7	PSCell addition and release delay

#### 7.5.7.0	Minimum conformance requirements

##### 7.5.7.0.1	Minimum conformance requirements for PSCell addition delay

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in subframe n + Tconfig_PSCell:

Where:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms.

Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [8].

Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the PSCell configuration command:

-	the UE has sent a valid measurement report for the PSCell being configured and

-	One of the SSBs measured from the PSCell being configured remains detectable according to the cell identification conditions specified in TS 38.133 [6] clause 9.3.

-	One of the SSBs measured from PSCell being configured also remains detectable during the PSCell configuration delay Tconfig_PSCell according to the cell identification conditions specified in TS 38.133 [6] clause 9.3.

otherwise it is unknown.

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only during the RRC reconfiguration procedure in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.9.2.

##### 7.5.7.0.2	Minimum conformance requirements for PSCell release delay

The requirements in this clause shall apply for a UE which is configured with PCell and one PSCell.

Upon receiving PSCell release in subframe n, the UE shall accomplish the release actions specified in TS 38.331 [13] no later than in subframe n+ TRRC_delay:

Where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only during the RRC reconfiguration procedure in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.9.3.

#### 7.5.7.1	NR SA FR2 addition and release delay of known PSCell

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- Test Requirements (still brackets in core-spec for PRACH preamble time [112] ms and CSI report time [20] ms)

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.7.1.1	Test purpose

The purpose of this test is:

- To verify the requirement for the PSCell addition and release delay are within the requirements specified in TS 38.133 [6] clause 8.9.2, when the PSCell is known to the UE at the time of addition.

7.5.7.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.7.0.1 and 7.5.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.7.1.

7.5.7.1.4	Test description

7.5.7.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.7.1.4.1-1.

Table 7.5.7.1.4.1-1: Supported test configurations for NR SA FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.7.1.4.1-2.

Table 7.5.7.1.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.7.1.4.1-3.

2. Message contents are defined in clause 7.5.7.1.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell (PSCell-to-be). Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.7.1.4.1-3: General test parameters for NR SA FR2 PSCell addition and release delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for event A4 |
|  | Threshold RSRP | dBm | -97 | Threshold for event A4 |
|  | Time to Trigger | S | 0 | Time to trigger for event A4 |
| DRX |  |  | OFF | For both PCell and PSCell once activated |
| Measurement gap pattern ID |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration in Cell 2 |  |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 |  | ms | [2] |  |
| T1 |  | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 1 | During this time the UE shall identify neighbour cell 2 and report event B1. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |

7.5.7.1.4.2	Test procedure

TBD

7.5.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.7.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.7.1.5	Test requirement

TBD

#### 7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete

- Test Requirements (still brackets in core-spec for PRACH preamble time [572] ms and CSI report time [20] ms)

- TT analysis is missing

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.7.2.1	Test purpose

The purpose of this test is:

- To verify the requirement for the PSCell addition and release delay are within the requirements specified in TS 38.133 [6] clause 8.9.2, when the PSCell is unknown to the UE at the time of addition.

7.5.7.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.7.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.7.0.1 and 7.5.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.7.2.

7.5.7.2.4	Test description

7.5.7.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.7.2.4.1-1.

Table 7.5.7.2.4.1-1: Supported test configurations for NR SA FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.73.2.4.1-2.

Table 7.5.7.2.4.1-2: Initial conditions for NR SA FR2 PSCell addition and release delay

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.7.2.4.1-3.

2. Message contents are defined in clause 7.5.7.2.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell (PSCell-to-be). Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.7.2.4.1-3: General test parameters for NR SA FR2 PSCell addition and release delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| DRX |  | OFF | For both PCell and PSCell once activated |
| PRACH configuration in Cell 2 |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | [2] |  |
| T1 | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 | s | 1 | During this time the UE adds the PSCell. |
| T3 | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T4 | s | 1 | During this time the UE releases the PSCell. |

7.5.7.2.4.2	Test procedure

TBD

7.5.7.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.7.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.7.2.5	Test requirement

TBD

## 7.6	Measurement procedures

### 7.6.1	Intra-frequency measurements

#### 7.6.1.0	Minimum conformance requirements

##### 7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap

[TS 38.133, clause 9.2.5.1 and 9.2.5.2]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCellis always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Where:

TPSS/SSS_sync_intr: it is the time period used in PSS/SSS detection given in table 7.6.1.0.1-1.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 7.6.1.0.1-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps.

Mpss/sss_sync_w/o_gaps : For a UE supporting FR2 power class 1, Mpss/sss_sync_w/o_gaps = 40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps = 24. For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps = 24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps =24

Mmeas_period_w/o_gaps : For a UE supporting power class 1, Mmeas_period_w/o_gaps = 40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps = 24. For a UE supporting power class 3, Mmeas_period_w/o_gaps = 24. For a UE supporting power class 4, Mmeas_period_w/o_gaps = 24.

When intrafrequency SMTC is fully non-overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1.

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

For FR2 when any of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap is fully overlapping with intra-frequency SMTC, Klayer1_measurement= 1.5, otherwise Klayer1_measurement=1.

Table 7.6.1.0.1-1: Time period for PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 7.6.1.0.1-2: Measurement period for intra-frequency measurements without gaps (Frequency FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 9.2.4.3]

Reported RSRP, RSRQ, and RS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in TS 38.133 [6] clause 10.1.2.1, 10.1.7.1 and 10.1.12.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra without index defined in TS 38.133 [6] clause 9.2.5.1. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period than Tidentify intra without index defined in TS 38.133 [6] clause 9.2.5.1 becomes undetectable for a period and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period, Intra provided the timing to that cell has not changed more than  3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

References: The conformance requirements covered in the current TC are specified in: TS 38.133 [6], clauses 9.2.2, 9.2.4.3, 9.2.5.1 and 9.2.5.2.

##### 7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 7.6.1.0.2-1.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 7.6.1.0.2-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS 38.133 [6] section 9.1.5.2.2 for measurement conducted within measurement gaps.

Mpss/sss_sync_with_gaps : For a UE supporting FR2 power class 1, Mpss/sss_sync with_gaps=40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting power class 4, Mpss/sss_sync with_gaps =24

Mmeas_period_ with_gaps: For a UE supporting power class 1, Mmeas_period_ with_gaps =40. For a UE supporting power class 2, Mmeas_period_ with_gaps =24. For a UE supporting power class 3, Mmeas_period_ with_gaps =24. For a UE supporting power class 4, Mmeas_period with_gaps =24.

Table 7.6.1.0.2-1: Time period for PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_gaps x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_gaps) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps x max(MGRP, DRX cycle) x CSSFintra |

Table 7.6.1.0.2-2: Measurement period for intra-frequency measurements with gaps (Frequency Range FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period with_gaps) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320ms | Mmeas_period with_gaps x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

#### 7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Some parameters are TBD

- Test tolerance is missing.

7.6.1.1.1	Test purpose

The purpose of this test is to verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under non-DRX. This test will partly verify the TDD intra-frequency cell search requirements defined in TS 38.133 clause 9.2.5.1 and 9.2.5.2

7.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.1.

7.6.1.1.4	Test description

7.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.1.4.1-1.

Table 7.6.1.1.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting without gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.1-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 7.6.1.1.4.1-2.

Table 7.6.1.1.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 7.6.1.1.4.1-3 below.

2. Message contents are defined in clause 7.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.1.4.1-3: General test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

7.6.1.1.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (NR Cell 1) and a FR2 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.1.4.1-3 and Table 7.6.1.1.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.1.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	2402 ms for UE supporting power class 1,

-	1442 ms for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.1.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting without gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.3 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.4 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.1-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ |

7.6.1.1.5	Test requirement

Table 7.6.1.1.4.1-3, Table 7.6.1.1.5-1 and Table 7.6.1.1.5-2 define the primary level settings including test tolerances for NR SA FR2 event-triggered reporting without gap in non-DRX.

Table 7.6.1.1.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.state.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.1.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.8.15.3 |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  | Rough |  |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity |  | 8+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -102+TT |  |  |  |  |
| Note 2 | dBm/SCS | 1 | -93+TT |  |  |  |  |
|  |  | 2 | -90+TT |  |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -89+TT | -89+TT | -Infinity |  | -85+TT |
|  |  | 2 | -86+TT | -86+TT | -Infinity |  | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity |  | 8+TT |
|  | dBm/95.04MHz | 1, 2 | -58.56+TT |  | -55.38+TT |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4s for a UE supporting power class 1,

-	1.44s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Test tolerance is missing.

7.6.1.2.1	Test purpose

Ther purpose of this test is to verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.2.

7.6.1.2.4	Test description

7.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.2.4.1-1.

Table 7.6.1.2.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting without gap in DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.2-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.2-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.2.4.1-2.

Table 7.6.1.2.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 7.6.1.2.4.1-3 below.

2. Message contents are defined in clause 7.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.2.4.1-3: General test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.2 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

7.6.1.2.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (NR Cell 1) and a FR2 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.2.4.1-3, Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for sub-test 1 or less than Y ms for sub-test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	7202 for UE supporting power class 1,

-	4322 for UE supporting power class 2, 3 or 4.

and Y is

-	51202 for UE supporting power class 1,

-	30722 for UE supporting power class 2, 3 or 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.1.2.4.1-1 as appropriate.

7.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.2.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting without gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.3 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.2-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.4 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.2-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 for sub-test 1 Table H.3.7-1 with Condition DRX.2 for sub-test 2 |

7.6.1.2.5	Test requirement

Table 7.6.1.2.4.1-3, Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used test.

Table 7.6.1.2.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.state.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.2.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9 |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -89+TT |  |  |  |
|  |  | 2 | -86+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -85+TT | -85+TT | -Infinity | -85+TT |
|  |  | 2 | -82+TT | -82+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
|  | dBm/95.04MHz | 1 | -54.53+TT | -52.18+TT | -54.53+TT | -52.18+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

For both tests:

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 40 ms (sub-test 1) is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 3600 ms, T SSB_measurement_period_intra= 3600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 2160 ms, T SSB_measurement_period_intra= 2160 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of X ms, where X is

- X = 7202 for UE supporting power class 1,

- X = 4322 for UE supporting power class 2, 3 or 4,

The overall delay measured when DRX cycle length is 640 ms (sub-test 2) is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 25600 ms, T SSB_measurement_period_intra= 25600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 15360 ms, T SSB_measurement_period_intra= 15360 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of X ms, where

- X = 51202 for UE supporting power class 1,

- X = 30722 for UE supporting power class 2, 3 or 4,

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Some parameters are TBD

- Test tolerance is missing.

7.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.3.

7.6.1.3.4	Test description

7.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.3.4.1-1.

Table 7.6.1.3.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting with gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.3-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.3-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.3.4.1-2.

Table 7.6.1.3.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.6.1.3.4.1-3.

2. Message contents are defined in clause 7.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.3.4.1-3: General test parameters for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |
| Measurement gap length | ms | 1, 2 | 6 |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

7.6.1.3.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.3.4.1-3, Table 7.6.1.3.5-1 and Tavke 7.6.1.3.5-2, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.3.5-1 and Table 7.6.1.3.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.3.5-1 and Table 7.6.1.3.5-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	X = 3202 for UE supporting power class 1,

-	X = 1922 ms for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.3 FR2, SMTC.1 and Synchronous cells for Configuration 7.6.1.3-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.4 FR2, SMTC.1 and Synchronous cells for Configuration 7.6.1.3-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ |

Table 7.6.1.3.4.3-: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

7.6.1.3.5	Test requirement

Table 7.6.1.3.4.1-3 and Table 7.6.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in non-DRX test.

Table 7.6.1.3.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | CSI-RS |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

7.6.1.3.5-2: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.9 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 8+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -102+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -93+TT |  |  |  |
|  |  | 2 | -90+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -89+TT | -89+TT | -Infinity | -85+TT |
|  |  | 2 | -86+TT | -86+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 8+TT |
|  | dBm/95.04MHz | 1, 2 | -58.56+TT |  | -55.38+TT |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

For UE supporting power class 1, TPSS/SSS_sync_intr = 1600 ms, T SSB_measurement_period_intra= 1600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intr = 960 ms, T SSB_measurement_period_intra= 960 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of X ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty), where

X = 3202 for UE supporting power class 1,

X = 1922 for UE supporting power class 2, 3 or 4,

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Test tolerance is missing.

7.6.1.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under DRX within intra-frequency cell search with gaps requirements. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.4.

7.6.1.4.4	Test description

7.6.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.4.4.1-1.

Table 7.6.1.4.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting with gap in DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.4-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.4-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.4.4.1-2.

Table 7.6.1.4.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.6.1.4.4.1-3.

2. Message contents are defined in clause 7.6.1.4.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.4.4.1-3: General test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |  |
| Measurement gap length | ms | 1, 2 | 6 |  |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.2 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

7.6.1.4.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.4.4.1-3 and Table 7.6.1.4.4.2-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In Test 1 when DRX cycle = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle. In Test 2 when DRX = 640 ms is used, the uplink time alignment is not maintained and the UE needs to use RACH to obtain uplink allocation for measurement reporting.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for Test 1 or less than Y ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X and Y is

-	X = 7202, Y = 51202 for UE supporting power class 1,

-	X = 4322, Y = 30722 for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.1.4.4.1-3 as appropriate.

7.6.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.4.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting with gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.4-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.4-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 and Gap for test 1 Table H.3.7-1 with Condition DRX.1 and Gap for test 2 |

Table 7.6.1.4.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

7.6.1.4.5	Test requirement

Table 7.6.1.4.4.1-3 , Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in DRX test.

Table 7.6.1.4.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SCSI-RS |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| TCI state |  | 1, 2 | CSI-RS.Config.0 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.4.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -89+TT |  |  |  |
|  |  | 2 | -86+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -85+TT | -85+TT | -Infinity | -85+TT |
|  |  | 2 | -82+TT | -82+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
|  | dBm/95.04MHz | 1 | -54.53+TT | -52.18+TT | -54.53+TT | -52.18+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report on PUSCH.

For both tests:

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 1: The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 7200ms, T SSB_measurement_period_intra= 7200 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 2160 ms, T SSB_measurement_period_intra= 2160 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of X ms, where X is

-	X = 7202 for UE supporting power class 1,

-	X = 4322 for UE supporting power class 2, 3 or 4,

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 25600 ms, T SSB_measurement_period_intra= 25600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 15360 ms, T SSB_measurement_period_intra= 15360 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of X ms, where

-	X = 51202 for UE supporting power class 1,

-	X = 30722 for UE supporting power class 2, 3 or 4,

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 7.6.2	Inter-frequency measurements

#### 7.6.2.0	Minimum conformance requirements for Inter-frequency measurements

Same as clause 5.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

#### 7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.1.

7.6.2.1.4	Test description

7.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.1.4.1-1.

Table 7.6.2.1.4.1-1: NR FR2-FR2 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.1.4.1-1: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

Table 7.6.2.1.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.1.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 3 as defined in clause A.9 The UE RX spherical coverage direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.2.1.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.1.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.1.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.1.4.1-2. The TE shall ensure that the NR FR2 cells are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and relative angular offset between active probes are according to Table A.9.3-1. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.1.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms for UE supporting power class 1, or 3202 ms for UE supporting other power class for Test 1 and Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. TE shall change the active probes in such way that relative angular offset between active probes differs in the following iteration.

10. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:

- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

12. Repeat step 1-11 for each sub-test in Table 7.6.2.1.4.1-2 as appropriate.

7.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.1.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with A3-offset = -11dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.1.4.3-2: MeasObjectNR-DEFAULT: SA FR2-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB16 |  |  |
| } |  |  |  |

7.6.2.1.5	Test requirement

Table 7.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.9 |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beamsNote 7 |  |  | Config 1,2 | Rough |  |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 | -Infinity | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 |  | -Infinity | -87 |
| BBNote 8 |  | dB | Config 1 | 1.89 | 1.89 |  | -Infinity | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 5122 ms in this test for power class UE and 3202 ms for other power classes.

#### 7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.2.

7.6.2.2.4	Test description

7.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.2.4.1-1.

Table 7.6.2.2.4.1-1: NR FR2-FR2 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.2-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -12 |  | Value modified by TT |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

Table 7.6.2.2.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.2.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.2.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.2.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.2.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 7682 ms for UE supporting power class 1, or 4802 ms for UE supporting other power class for Test 1 and Test 3 and 81922 ms for UE supporting power class 1, or 51202 ms for UE supporting other power class for Test 2 and Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.2.4.1-2 as appropriate.

7.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.2.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and and Synchronous cells Table H.3.1-4 with A3-offset = -12dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and DRX.2 for Test 2 |

7.6.2.2.5	Test requirement

Table 7.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table A.7.6.2.2.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.9 |  |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity |  | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity |  | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity |  | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 |  | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 7682 ms for power class 1 UE and 4802 ms for other power classes in test 1 and 81922 for power class 1 UE and 51202 ms for other power classes in test 2.

#### 7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.3.

7.6.2.3.4	Test description

7.6.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.3.4.1-1.

Table 7.6.2.3.4.1-1: NR FR2-FR2 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.3-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.3.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table 7.6.2.3.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.3.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 3 as defined in clause A.9. The UE RX spherical coverage direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.2.3.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.3.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.3.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.3.4.1-2. The TE shall ensure that the NR FR2 cells are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and relative angular offset between active probes are according to Table A.9.3-1. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.3.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 6722 ms for UE supporting power class 1, or 4162 ms for UE supporting other power class for Test 1 and Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. TE shall change the active probes in such way that relative angular offset between active probes differs in the following iteration.10. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

12. Repeat step 1-11 for each sub-test in Table 7.6.2.3.4.1-2 as appropriate.

7.6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.3.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A3-offset = -11dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.3.4.3-2: MeasObjectNR-DEFAULT: SA FR2-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB16 |  |  |
| } |  |  |  |

7.6.2.3.5	Test requirement

Table 7.6.2.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.3.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.9 |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 | -Infinity | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 |  | -Infinity | -87 |
| BBNote 8 |  | dB | Config 1 | 1.89 | 1.89 |  | -Infinity | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

The UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 6722 ms in this test for power class UE and 4162 ms for other power classes.

#### 7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.4.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.4.

7.6.2.4.4	Test description

7.6.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.4.4.1-1.

Table 7.6.2.4.4.1-1: NR FR2-FR2 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.4-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.4.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -12 |  | Value modified by TT |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

Table 7.6.2.4.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.4.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.4.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.4.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.4.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.4.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.4.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 10082 ms for UE supporting power class 1, or 6242 ms for UE supporting other power class for Test 1 and Test 3 and 107522 ms for UE supporting power class 1, or 66562 ms for UE supporting other power class for Test 2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.4.4.1-2 as appropriate.

7.6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.4.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A3-offset = -12dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and DRX.2 for Test 2 |

7.6.2.4.5	Test requirement

Table 7.6.2.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.4.5-1: Cell specific test parameters for CA inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.9 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 , the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 10082 ms for power class 1 UE and 6242 ms for other power classes in test 1 and 107522 for power class 1 UE and 66562 ms for other power classes in test 2.

#### 7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.5.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.5.

7.6.2.5.4	Test description

7.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.5.4.1-1.

Table 7.6.2.5.4.1-1: SA FR1-FR2 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.5-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.5.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 in FR1 |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2 in FR2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in TS 38.133 [6]clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 5.2 for PC1; 3.5 for other PC | 3 for PC1; 2 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.5.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.5.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 2. Otherwise it is only required to pass test 1.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.5.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.5.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.5.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.5.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.5.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms for UE supporting power class 1, or 3202 ms for UE supporting other power class for Test 1 and 2562 ms for UE supporting power class 1, or 1602 ms for UE supporting other power class for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.5.4.1-2 as appropriate.

7.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.5.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 Table H.3.1-2 with Condition INTER-FREQ for Test 2 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4AA with A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.5.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.5.5	Test requirement

Table 7.6.2.5.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.5.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  |
| AoA setup |  |  | Config 1,2,3 | N/A |  | Setup 1 as specified in clause A.9 |  |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |  |
|  |  |  | Config 3 | 30 |  | 120 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2,3 |  |  | -Infinity |  | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |  |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |  |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |  |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | -Infinity | -58.01 |  |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

In test 2, without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

2560 for UE supporting power class 1, or

1600 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 5122 ms for power class 1 UE and 3202 ms for other power classes in test 1 and 2562 for power class 1 UE and 1602 ms for other power classes in test 2.

#### 7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.6.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.6.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.6.

7.6.2.6.4	Test description

7.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.6.4.1-1.

Table 7.6.2.6.4.1-1: SA FR1-FR2 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.6-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.6-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.6.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | DRX is used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.6.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.6.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.7.6.2.6.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.6.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.6.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.6.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.6.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 7682 ms for UE supporting power class 1, or 4802 ms for UE supporting other power class for Test 1 and Test 3 and 81922 ms for UE supporting power class 1, or 51202 ms for UE supporting other power class for Test 2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.6.4.1-2 as appropriate.

7.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.6.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 and Test 2 Table H.3.1-2 with Condition INTER-FREQ for Test 3 and Test 4 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4AA with A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 and Test 2 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and Test 3 and DRX.2 for Test 2 and Test 4 |

Table 7.6.2.6.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.6.5	Test requirement

Table 7.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
|  |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | NA Link only, see clause A.3.7A |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 7682 ms for power class 1 UE and 4802 ms for other power classes in test 1 and test 3 and 81922 for power class 1 UE and 51202 ms for other power classes in test 2 and test 4.

#### 7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.7.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.7.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.7.

7.6.2.7.4	Test description

7.6.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.7.4.1-1.

Table 7.6.2.7.4.1-1: SA FR1-FR2 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.7-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.7-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.7-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.7.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two NR carrier frequencies is used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in TS 38.133 [6] clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 7 for PC1; 4.5 for other PC | 3.5 for PC1; 2.5 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.7.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.7.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 2. Otherwise it is only required to pass test 1.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.7.4.2	Test procedure

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.7.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.7.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.7.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.7.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 6722 ms for UE supporting power class 1, or 4162 ms for UE supporting other power class for Test 1 and 3362 ms for UE supporting power class 1, or 2082 ms for UE supporting other power class for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.7.4.1-2 as appropriate.

7.6.2.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.7.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 Table H.3.1-2 with Condition INTER-FREQ for Test 2 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4AA with Condition SSB Index and A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.7.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.7.5	Test requirement

Table 7.6.2.7.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.7.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2, 3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

In test 1 with per-UE gap , the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

In test 2 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

3360 for UE supporting power class 1, or

2080 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 6722 ms for power class 1 UE and 4162 ms for other power classes in test 1 and 3362 for power class 1 UE and 2082 ms for other power classes in test 2

#### 7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.8.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.8.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.8.

7.6.2.8.4	Test description

7.6.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.8.4.1-1.

Table 7.6.2.8.4.1-1: SA FR1-FR2 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.8-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.8-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.8-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.8.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | DRX is used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.8.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.8.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.8.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.8.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.8.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.6.2.8.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3.	The SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.8.4.1-2.

6.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 10082 ms for UE supporting power class 1, or 6242 ms for UE supporting other power class for Test 1 and Test 3 and 107522 ms for UE supporting power class 1, or 66562 ms for UE supporting other power class for Test 2 and Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11.	Repeat step 1-10 for each sub-test in Table 7.6.2.8.4.1-2 as appropriate.

7.6.2.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.8.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 and Test 2 Table H.3.1-2 with Condition INTER-FREQ for Test 3 and Test 4 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4AA with Condition SSB Index and A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 and Test 2 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and Test 3 and DRX.2 for Test 2 and Test 4 |

Table 7.6.2.8.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.8.5	Test requirement

Table 7.6.2.8.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.8.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  |  |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 | A.3.7A |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 10082 ms for power class 1 UE and 6240 ms for other power classes in test 1 and test 3 and 107522 for power class 1 UE and 66562 ms for other power classes in test 2 and test 4.

### 7.6.3	L1-RSRP measurement for beam reporting

#### 7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting

##### 7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

Same as clause 5.6.3.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.5.3.1, 9.5.4.1 and 9.5.5.1.

##### 7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

Same as clause 5.6.3.0.2

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3.1, 9.5.4.2 and 9.5.5.2.

#### 7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test case is incomplete for extreme conditions

7.6.3.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

7.6.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.1.

7.6.3.1.4	Test description

7.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.1.4.1-1.

Table 7.6.3.1.4.1-1: NR SA FR2 SSB-based L1-RSRP measurement in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.6.3.1-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 7.6.3.1.4.1-2: General test parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
|  |  |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | Off |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 320 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.1.4-3: Test Environment parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.3.1.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.3.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 7.6.3.1.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 7.6.3.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 320 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 7.6.3.1.5-1. T2 starts.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check the following requirements:

- R1: the UE shall start to transmit valid L1-RSRP reports no later than 1760ms for UE supporting power class 1 in configuration 1, no later than 1720 ms for UE supporting power class 1 in configuration 2, no later than 1280 ms for UE supporting power class other than 1 in configuration 1 and no later than 1240 ms for UE supporting  power class other than 1 in configuration 2 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 Table 5.6.3.1.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 5.6.3.1.5-3 for test configuration 2. If the first valid report is received before the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2: the UE shall transmit L1-RSRP reports every 320 slots. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3: The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding absolute accuracy requirements in Table 7.6.3.1.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 7.6.3.1.5-3 for test configuration 2 the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4: The DIFF-RSRP value of SSB#0 reported by the UE is compared to the expected DIFF-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding relative accuracy requirements in Table 7.6.3.1.5-4 for all test configurations, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.3.1.4.3-1: Common Exception messages NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB Table H.3.4-1 |

Table 7.6.3.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.6.3.1.5	Test requirement

Table 7.6.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.1.5-1: Cell specific test parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.9 |  |  |  |
| Assumption for UE beamsNote 4 | 1~2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 320 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	1680 for UE supporting power class 1

-	1200 for UE supporting power class 2,3 or 4.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 7.6.3.1.5-2 for for test configuration 1  and the corresponding absolute accuracy requirements in Table 7.6.3.1.5-3 for test configuration 2 and the corresponding relative accuracy requirements in Table 7.6.3.1.5-4 for all test configurations.

Table 7.6.3.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configuration 1

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 40 |
| Highest reported value (SSB#1) | - | 99 |

Table 7.6.3.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 43 |
| Highest reported value (SSB#1) | - | 102 |

Table 7.6.3.1.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 1 |
| Highest DIFF RSRP reported (SSB#0) | - | 7 |

For the test to pass, the ratio of successful reported valued for each requirement (R1 to R4) shall be at least 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

#### 7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test case is incomplete for extreme conditions

7.6.3.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

7.6.3.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.2.

7.6.3.2.4	Test description

7.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.2.4.1-1.

Table 7.6.3.2.4.1-1: NR SA FR2 SSB-based L1-RSRP measurement in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.6.3.2-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 7.6.3.2.4.1-2: General test parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | DRX.3 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 320 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 3 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.2.4-3: Test Environment parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Message contents are defined in clause 7.6.3.2.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.3.2.4.2	Test procedure

Same as in 7.6.3.1.4.2 with the following exception:

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check the following requirements:

- R1: the UE shall start to transmit valid L1-RSRP reports no later than 2960ms for UE supporting power class 1 in configuration 1, no later than 2920ms for UE supporting power class 1 in configuration 2, no later than 2000ms for UE supporting power class other than 1 in configuration 1 and no later than 1960 ms for UE supporting  power class other than 1 in configuration 2 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 Table 5.6.3.1.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 5.6.3.1.5-3 for test configuration 2. If the first valid report is received before the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2: the UE shall transmit L1-RSRP reports every 320 slots. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3: The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding absolute accuracy requirements in Table 7.6.3.2.5-2 for test configurations 1 and 2 and the corresponding absolute accuracy requirements in Table 7.6.3.2.5-3 for test configurations 3 and 4 the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4: The DIFF-RSRP value of SSB#0 reported by the UE is compared to the expected DIFF-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding relative accuracy requirements in Table 7.6.3.2.5-4 for all test configurations, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

7.6.3.2.4.3	Message contents

Same message content as in subclause 7.6.3.1.4.3 with the following exception:

Table 7.6.3.2.4.3-1: Common Exception messages NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

7.6.3.2.5	Test requirement

Table 7.6.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.2.5-1: Cell specific test parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.9 |  |  |  |
| Assumption for UE beamsNote 4 | 1~2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 320 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	2880 for UE supporting power class 1

-	1920 for UE supporting power class 2,3 or 4.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 7.6.3.2.5-2 for test configuration 1  and the corresponding absolute accuracy requirements in Table 7.6.3.2.5-3 for test configuration 2 and the corresponding relative accuracy requirements in Table 7.6.3.2.5-4 for all test configurations.

Table 7.6.3.2.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configuration 1

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 40 |
| Highest reported value (SSB#1) | - | 99 |

Table 7.6.3.2.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 43 |
| Highest reported value (SSB#1) | - | 102 |

Table 7.6.3.2.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 1 |
| Highest DIFF RSRP reported (SSB#0) | - | 7 |

For the test to pass, the ratio of successful reported valued for each requirement (R1 to R4) shall be at least 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

#### 7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test case is incomplete for extreme conditions

7.6.3.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.2.

7.6.3.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.3.

7.6.3.3.4	Test description

7.6.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.3.4.1-1.

Table 7.6.3.3.4.1-1: NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.4-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table 7.6.3.3.4.1-2: General test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.3.4-3: Test Environment parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.3.3.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.3.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-RSRP based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 7.6.3.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.6.3.3.5-1. T1 starts.

3.	After 480ms from the start of the test the SS transmits the DCI trigger in slot 1.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-RSRP report at slot 8 from the reception of DCI trigger. If the report is received at slot 8 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the L1-RSRP value of CSI-RS#1 reported by the UE is compared to the expected L1-RSRP value for CSI-RS #1. If the resulting value is outside the limits in Table 7.6.3.3.5-2 or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

-R3:	The DIFF RSRP value of CSI-RS #0 reported by the UE is compared to the expected DIFF RSRP value. If the resulting value is outside the limits in Table 7.6.3.3.5-4 or the UE fails to report the measurement value for CSI-RS #0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.6.3.3.4.3-1: Common Exception messages NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions APERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS and APERIODIC |

Table 7.6.3.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

7.6.3.3.5	Test requirement

Table 7.6.3.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.3.5-1: Cell specific test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 480ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 7.6.3.3.5-2 the corresponding relative accuracy requirements in Table 7.6.3.3.5-3.

Table 7.6.3.3.5-2: L1-RSRP absolute accuracy requirements for the reported values

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 40 |
| Highest reported value (CSI-RS#1) | 99 |

Table 7.6.3.3.5-3: L1-RSRP relative accuracy requirements for the reported values

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 1 |
| Highest DIFF RSRP reported (CSI-RS#0) | 7 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test case is incomplete for extreme conditions

7.6.3.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.2.

7.6.3.4.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.3.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.4.

7.6.3.4.4	Test description

7.6.3.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.4.4.1-1.

Table 7.6.3.4.4.1-1: NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.4-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table 7.6.3.4.4.1-2: General test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.4.4-3: Test Environment parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.3.4.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.3.4.4.2	Test procedure

Same test procedure as in subclause 7.6.3.3.4.2 with tables 7.6.3.3.4.1-2 and 7.6.3.3.5-1 replaced by tables 7.6.3.4.4.1-2 and 7.6.3.4.5-1 and following change in step 3.

3.	After 1440ms from the start of the test the SS transmits the DCI trigger in slot 1.

7.6.3.4.4.3	Message contents

Same message content as in subclause 7.6.3.3.4.3 with the following exception:

Table 7.6.3.4.4.3-1: Common Exception messages NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

7.6.3.4.5	Test requirement

Table 7.6.3.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.4.5-1: Cell specific test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Assumption for UE beamsNote 3 | 1~2 |  | Rough |  |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

After 1440ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 7.6.3.4.5-2 the corresponding relative accuracy requirements in Table 7.6.3.4.5-3.

Table 7.6.3.4.5-2: L1-RSRP absolute accuracy requirements for the reported values

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 40 |
| Highest reported value (CSI-RS#1) | 99 |

Table 7.6.3.4.5-3: L1-RSRP relative accuracy requirements for the reported values

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 1 |
| Highest DIFF RSRP reported (CSI-RS#0) | 7 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 7.7	Measurement performance requirements

### 7.7.1	SS-RSRP

#### 7.7.1.0	Minimum conformance requirements

##### 7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements

Same as in clause 5.7.1.0.1.

##### 7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements

Same as in clause 5.7.1.0.2.

#### 7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.1.

7.7.1.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the NR FR2 neighbour cell.

7.7.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.1.4.1-1.

Table 7.7.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.1.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.1.4.1-2.

Table 7.7.1.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.1.1.4.1-3.

2.	Message contents are defined in clause 7.7.1.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-RSRP measurements.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.1.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.1.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport for the following requirements:

- R1: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.1.1.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

- R2: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.1.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.1.1.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G for each of the requirements is achieved. The evaluation of a specific requirement is concluded when the confidence level for that requirement is reached, even if more measurement reports are required for the remaining requirements.

8.	Set the parameters according to Test 2 in Table 7.7.1.1.5-2 as appropriate and repeat steps 5-7. In Step 6, the SS shall check the following requirements:

- R4: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.1.1.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

- R5: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.1.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R5 is increased by one. Otherwise, the number of passed iterations for R5 is increased by one.

- R6: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.1.1.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R6 is increased by one. Otherwise, the number of passed iterations for R6 is increased by one.

- R7: The SS-RSRP value of Cell 1 reported by the UE during Test 2 is compared to the reported SS-RSRP of Cell 1 during Test 1 for the same iteration. If the resulting value is outside the limits in Table 7.7.1.1.5-5 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R7 is increased by one. Otherwise, the number of passed iterations for R7 is increased by one

- R8: The SS-RSRP value of Cell 2 reported by the UE during Test 2 is compared to the reported SS-RSRP of Cell 2 during Test 1 for the same iteration. If the resulting value is outside the limits in Table 7.7.1.1.5-5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R8 is increased by one. Otherwise, the number of passed iterations for R8 is increased by one

9.	If more measurement reports with Test 1 configuration are needed in order to complete the evaluation R7 or R8, the SS shall set the parameters according to Table 7.7.1.1.5-2 as appropriate and repeat steps 5 to 8, evaluating R7 and / or R8 as appropriate.

7.7.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition Synchronous cells Table H.3.1-5 Table H.3.1-7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.1.1.5	Test requirement

Table 7.7.1.1.5-1 defines the cell specific settings for all tests. Table 7.7.1.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clause 7.7.1.0.1.1 and relative accuracy requirements in clause 7.7.1.0.1.2. The following eight requirements are to be verified:

During T1:

R1: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.1.1.5-3.

R2: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.1.1.5-3.

R3: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.1.5-4.

During T2:

R4: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.1.1.5-3.

R5: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.1.1.5-3.

R6: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.1.5-4.

During T1 and T2:

R7: Relative accuracy of Cell 1 during T2 compared with Cell 1 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.1.5-5.

R8: Relative accuracy of Cell 2 during T2 compared with Cell 2 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.1.5-5.

Table 7.7.1.1.5-1: SS-RSRP Intra frequency general test parameters

| ParameterNote 5 | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Physical cell ID |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 24 |  | 24 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.2 TDD | - | SR.3.2 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: All parameters apply for configuration 1 and 2 Note 6: Void |  |  |  |  |  |

Table 7.7.1.1.5-2: SS-RSRP Intra frequency OTA related test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 |  |  |  |
| Assumption for UE beamsNote 8 |  | Rough |  |  |  |
| Note1 | dBm/15kHzNote4 | -97.4 |  | N/A |  |
| Note1 | dBm/SCSNote4 | -88.37 |  | N/A |  |
|  | dB | 6.0 | 1.4 | N/A | N/A |
| Es | dBm/SCSNote4 |  |  | (Table B.2.2-2 Rx Beam Peak +9.8dB) | (Table B.2.2-2 Rx Beam Peak +9.8dB) |
| SSB_RPNote2 | dBm/SCS | -82.37 | -86.97 | (Table B.2.2-2 Rx Beam Peak +9.8dB) | (Table B.2.2-2 Rx Beam Peak +9.8dB) |
| BB Note6 | dB | 2.20 | -5.59 | -1.77 | -1.77 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.74 |  | (Table B.2.2-2 Rx Beam Peak +37.40dB) |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. Note 7: All parameters apply for configurations 1 and 2 Note 8: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 7.7.1.1.5-3: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 50 | n257, n258, n261 | 31 |
|  |  | n260 | 33 |
|  |  | n259 | FFS |
| Highest reported value (Cell 1) | 108 | n257, n258, n261 | 88 |
|  |  | n260 | 90 |
|  |  | n259 | FFS |
| Lowest reported value (Cell 2) | 46 | n257, n258, n261 | 31 |
|  |  | n260 | 33 |
|  |  | n259 | FFS |
| Highest reported value (Cell 2) | 103 | n257, n258, n261 | 88 |
|  |  | n260 | 90 |
|  |  | n259 | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 47+ FFS | n257, n258, n261 | 28 + FFS |
|  |  | n260 | 30 + FFS |
|  |  | n259 | FFS |
| Highest reported value (Cell 1) | 111+ FFS | n257, n258, n261 | 91 + FFS |
|  |  | n260 | 93 + FFS |
|  |  | n259 | FFS |
| Lowest reported value (Cell 2) | 46+ FFS | n257, n258, n261 | 28+ FFS |
|  |  | n260 | 30+ FFS |
|  |  | n259 | FFS |
| Highest reported value (Cell 2) | 106+ FFS | n257, n258, n261 | 91+ FFS |
|  |  | n260 | 93+ FFS |
|  |  | n259 | FFS |

Table 7.7.1.1.5-4: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 2 – Cell 1) | - 12 | - 6 |
| Highest reported value (Cell 2 – Cell 1) | + 2 | + 6 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 2 – Cell 1) | - 15+ FFS | - 9+ FFS |
| Highest reported value (Cell 2 – Cell 1) | + 5+ FFS | + 9+ FFS |

Table 7.7.1.1.5-5: evaluation limits for the reported values for T2 with respect to T1 relative accuracy rules R7, R8

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Test 2 | Bands | Normal Conditions | Extreme Conditions |
| Lowest reported value (Cell 1 T2 – Cell 1 T1) | n257, n258, n261 | - 28 | - 31+ FFS |
|  | n260 | - 26 | - 29+ FFS |
|  | n259 | FFS | FFS |
| Highest reported value (Cell 1 T2 – Cell 2 T1) | n257, n258, n261 | - 12 | - 9+ FFS |
|  | n260 | - 10 | - 7+ FFS |
|  | n259 | FFS | FFS |
| Lowest reported value (Cell 2 T2 – Cell 2 T1) | n257, n258, n261 | - 24 | - 27+ FFS |
|  | n260 | - 21 | - 24+ FFS |
|  | n259 | FFS | FFS |
| Highest reported value (Cell 2 T2 – Cell 2 T1) | n257, n258, n261 | - 8 | - 5+ FFS |
|  | n260 | - 5 | - 2+ FFS |
|  | n259 | FFS | FFS |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R8) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

#### 7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.1.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.2.

7.7.1.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.2.4.1-1.

Table 7.7.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| 7.7.1.2-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.2.4.1-2.

Table 7.7.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.1.2.4.1-3.

2.	Message contents are defined in clause 7.7.1.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-RSRP measurements.

4.	The rx beam peak and directions in which the UE meets the EIS spherical coverage criteria have been found with one of the procedures from Annex I.

7.7.1.2.4.2	Test procedure

1.	Configure the positioning system for a valid test point as defined in A.9.4. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.1.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport for the following requirements:

- R1: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.1.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

- R2: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.1.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.1.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G for each of the requirements is achieved. The evaluation of a specific requirement is concluded when the confidence level for that requirement is reached, even if more measurement reports are required for the remaining requirements.

8.	The SS shall select a new test point as defined in A.9.4 and rotate the positioning system for the selected test point.

9.	Set the parameters according to Test 2 in Table 7.7.2.1.5-2 as appropriate and repeat steps 5-7. In Step 6, the SS shall check the following requirements:

- R4: The SS-RSRP value of Cell 1 reported by the UE is compared to the expected SS-RSRP for Cell 1. If the value is outside the limits in Table 7.7.2.1.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

- R5: The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP for Cell 2. If the value is outside the limits in Table 7.7.2.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R5 is increased by one. Otherwise, the number of passed iterations for R5 is increased by one.

- R6: The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 7.7.2.1.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R6 is increased by one. Otherwise, the number of passed iterations for R6 is increased by one.

7.7.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-5 Table H.3.1-6 with condition Pattern #0 Table H.3.1-7 with condition INTER-FREQ Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.1.2.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.1.2.5	Test requirement

Table 7.7.1.2.5-1 defines the cell specific settings for all tests. Table 7.7.1.2.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clause 7.7.1.0.2.1 and relative accuracy requirements in clause 7.7.1.0.2.2. The following eight requirements are to be verified:

During T1:

R1: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.1.2.5-3 for test configuration 1 and in Table 7.7.1.2.5-4 for test configuration 2.

R2: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table 7.7.1.2.5-3 for test configuration 1 and in Table 7.7.1.2.5-4 for test configuration 2.

R3: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.2.5-5.

During T2:

R4: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.1.2.5-3 for test configuration 1 and in Table 7.7.1.2.5-4 for test configuration 2.

R5: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table 7.7.1.2.5-3 for test configuration 1 and in Table 7.7.1.2.5-4 for test configuration 2.

R6: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 7.7.1.2.5-5.

Table 7.7.1.2.5-1: SS-RSRP Inter frequency general test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1~2 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | 1 |  | 24 |  | 24 |  |
|  | 2 |  | 48 |  | 48 |  |
| Gap pattern ID |  |  | 0 |  | 0 |  |
| Duplex mode | 1~2 |  | TDD |  | TDD |  |
| TDD configuration | 1~2 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3. 2 TDD | - | SR.3. 2 TDD | - |
|  | 2 |  | SR.3.3 TDD |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
|  | 2 |  | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
|  | 2 |  | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  | SSB.4 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | kHz | 120 |  | 120 |  |
| OCNG Patterns | 1~2 |  | OP.3 |  | OP.3 |  |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 2 and Cell 1 | 1~2 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~2 | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration | 1~2 | - | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void. |  |  |  |  |  |  |

Table 7.7.1.2.5-2: SS-RSRP Inter frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | 1~2 |  | Setup 4b according to clause A.9.4 |  | Setup 4b according to clause A.9.4 |  |
|  |  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |
| Assumption for UE beamsNote 7 | 1~2 |  | Rough |  | Rough |  |
| Note1 | 1, | dBm/15kHzNote4 | -96.3 | -96.3 | (Table B.2.3-2 Rx Beam PeakNote 8 -4.63dB) | (Table B.2.3-2 Rx Beam PeakNote 8 -3.03dB) |
|  | 2 |  | -99.3 | -99.3 |  |  |
| Note1 | 1 | dBm/SCSNote4 | -87.3 | -87.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +4.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +6.0dB) |
|  | 2 |  | -87.3 | -87.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +7.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +9.0dB) |
|  | 1~2 | dB | 6.0 | 6.0 | 17.0 | 1.0 |
| SSB_RPNote2 | 1 | dBm/SCS | -81.3 | -81.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +21.4dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +7.0dB) |
|  | 2 |  | -81.3 | -81.3 | (Table B.2.3-2 Rx Beam PeakNote 8 +24.4dB) | (Table B.2. 3-2 Rx Beam PeakNote 8 +10.0dB) |
| (SSB_RPCell 1 – SSB_RPCell 2) | 1~2 | dB | 0 |  | 14.40 |  |
| BBNote6 | 1 | dB | 5.23 | 5.93 | 5.38 | -1.46 |
|  | 2 |  | 4. 58 | 5.87 |  |  |
| IoNote2 | 1 | dBm/95.04 MHz Note4 | -55.70 | -55.70 | (Table B.2.3-2 Rx Beam PeakNote 8 +46.08dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +35.13dB) |
|  | 2 |  | -55.7 | -55.7 | (Table B.2.3-2 Rx Beam PeakNote 8 +49.09dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +38.14dB) |
| (Iofreq 1 – Io freq 2) | 1~2 | dB | 0 |  | 11.95 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot, Io, (SSB_RPCell 2 – SSB_RPCell 1) and (Iofreq 2 – Io freq 1) levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP or ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation Note 8: The value in Table B.2.3-2 of TS 38.133 [6] is the Minimum SSB_RP for SCSSSB = 120 kHz, selected according to the operating band of Cell 2 and UE power class, without ∆MBP,n adjustment. |  |  |  |  |  |  |

Table 7.7.1.2.5-3: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5 for test configuration 1

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 | n257, n258, n261 | 33 |
|  | n260 | 39 | n260 | 34 |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 |  | n257, n258, n261 | 101 |
|  |  |  | n260 | 104 |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 |  | n257, n258, n261 | 32 |
|  |  |  | n260 | 34 |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 |  | n257, n258, n261 | 87 |
|  |  |  | n260 | 90 |
|  |  |  | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 + FFS | n257, n258, n261 | 33 + FFS |
|  | n260 | 39 + FFS | n260 | 34 + FFS |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 + FFS |  | n257, n258, n261 | 101 + FFS |
|  |  |  | n260 | 104 + FFS |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 + FFS |  | n257, n258, n261 | 32+ FFS |
|  |  |  | n260 | 34+ FFS |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 + FFS |  | n257, n258, n261 | 87+ FFS |
|  |  |  | n260 | 90+ FFS |
|  |  |  | n259 | FFS |

Table 7.7.1.2.5-4: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5 for test configuration 2

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 | n257, n258, n261 | 36 |
|  | n260 | 39 | n260 | 37 |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 |  | n257, n258, n261 | 104 |
|  |  |  | n260 | 107 |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 |  | n257, n258, n261 | 35 |
|  |  |  | n260 | 37 |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 |  | n257, n258, n261 | 90 |
|  |  |  | n260 | 93 |
|  |  |  | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 |  |
| Lowest reported value (Cell 1) | n257, n258, n261 | 41 + FFS | n257, n258, n261 | 36 + FFS |
|  | n260 | 39 + FFS | n260 | 37 + FFS |
|  | n259 | FFS | n259 | FFS |
| Highest reported value (Cell 1) | All bands: 109 + FFS |  | n257, n258, n261 | 104 + FFS |
|  |  |  | n260 | 107 + FFS |
|  |  |  | n259 | FFS |
| Lowest reported value (Cell 2) | All bands: 52 + FFS |  | n257, n258, n261 | 35+ FFS |
|  |  |  | n260 | 37+ FFS |
|  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | All bands: 109 + FFS |  | n257, n258, n261 | 90+ FFS |
|  |  |  | n260 | 93+ FFS |
|  |  |  | n259 | FFS |

Table 7.7.1.2.5-5: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 |  | Test 2 All bands |  |
| Lowest value (RSRP report Cell 2 – RSRP report Cell 1) | All bands: -15 |  | -29 |  |
| Highest value (RSRP report Cell 2 – RSRP report Cell 1) | n257, n258, n261 | +25 | n257, n258, n261 | +11 |
|  | n260 | +27 | n260 | +13 |
|  | n259 | FFS | n259 | FFS |
| Extreme Conditions | Test 1 |  | Test 2 All bands |  |
| Lowest value (RSRP report Cell 2 – RSRP report Cell 1) | -15 + FFS |  | -29+ FFS |  |
| Highest value (RSRP report Cell 2 – RSRP report Cell 1) | n257, n258, n261 | +25 + FFS | n257, n258, n261 | +11 + FFS |
|  | n260 | +27+ FFS | n260 | +13 + FFS |
|  | n259 | FFS | n259 | FFS |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R6) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

#### 7.7.1.3	Inter-frequency measurements between FR1 and FR2

##### 7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy

Editor’s Note:

- Test tolerance analysis is missing.

- Connection diagram is TBD.

- Table 7.7.1.3.1.5-3 of reported value is FFS

- RAN4 dependency: Some test parameters are still TBD.

7.7.1.3.1.1	Test Purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy with FR1 serving cell and FR2 target cell.

7.7.1.3.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.7.1.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.3.

7.7.1.3.1.4	Test description

7.7.1.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.3.1.4.1-1.

Table 7.7.1.3.1.4.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.7.1.3.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.7.1.3.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.7.1.3.1-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.3.1.4.1-2.

Table 7.7.1.3.1.4.1-2: Initial conditions for NR SA FR1-FR2 SS-RSRP absolute measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.7.1.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 7.7.1.3.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR FR2 neighbour cell (the target cell for SS-RSRP measurements) on a different frequency than the PCell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

7.7.1.3.1.4.2	Test procedure

Same as test procedure in clause 6.7.1.1.1.4.2 with the following changes:

-	Table 6.7.1.1.1.5-1 is replaced by Table 7.7.1.3.1.5-1 and 7.7.1.3.1.5-2;

-	Table 6.7.1.2.1.5-2 is replaced by Table 7.7.1.3.1.5-3 for test 1 and Table 7.7.1.3.1.5-4 for test 2.

7.7.1.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.3.1.4.3-1: Common Exception messages for NR SA FR1-FR2 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table H.3.1-7 with Condition INTER-FREQ |

Table 7.7.1.3.1.4.3-2: ReportConfigNR-DEFAULT

Derivation Path: TS 38.508-1 [14] Table 4.6.3-142 with Condition PERIODICAL

7.7.1.3.1.5	Test requirement

Table 7.7.1.3.1.5-1 and Table 7.7.1.3.1.5-2 define the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 7.7.1.3.1.5-1 and 7.7.1.3.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 7.7.1.3.1.5-3.

Table 7.7.1.3.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 100: NRB,c = 66 | 10: NRB,c = 52 | 100: NRB,c = 66 |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD | TDD | FDD | TDD |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A | TDDConf.3.1 | N/A | TDDConf.3.1 |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 | SSB.1 FR2 | SSB.1 FR1 | SSB.1 FR2 |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~3 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~3 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 1 and Cell 2 | 1~3 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~3 | - | NA Link only, see TS 38.133 [6] clause A.3.7A | AWGN | NA Link only, see TS 38.133 [6] clause A.3.7A | AWGN |
| Antenna configuration | 1~3 | - |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table 7.7.1.3.1.5-2: SS-RSRP inter-frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
|  | 1~3 | dBm/15kHz | NA Link only, see 38.133 [6] clause A.3.7A | TBD | NA Link only, see 38.133 [6] clause A.3.7A | NA |
|  | 1,2 | dBm/SSB SCS |  | TBD |  | NA |
|  | 3 |  |  | TBD |  | NA |
|  | 1~3 | dB |  | TBD |  | NA |
| SS-RSRPNote1 | 1,2 | dBm/SCS |  | TBD |  | As in 38.133 [6] Table B.2.3-2 |
|  | 3 |  |  | TBD |  | As in 38.133 [6] Table B.2.3-2 |
| IoNote1 | 1~3 | dBm/95.04MHz |  | TBD |  | SS-RSRP+28.98 |
|  | 1~3 | dB |  | TBD |  | NA |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. |  |  |  |  |  |  |

Table 7.7.1.3.1.5-3: SS-RSRP inter-frequency absolute accuracy requirements for the reported values for Test 1

FFS

Table 7.7.1.3.1.5-4: SS-RSRP inter-frequency absolute accuracy requirements for the reported values for Test 2

FFS

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 7.7.1.3.2	Void

### 7.7.2	SS-RSRQ

#### 7.7.2.0	Minimum conformance requirements

##### 7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 5.7.2.0.1.

##### 7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 5.7.2.0.2.

#### 7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

7.7.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.2.1.

7.7.2.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the intra-frequency NR FR2 neighbour cell.

7.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.2.1.4.1-1.

Table 7.7.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.2.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.2.1.4.1-2.

Table 7.7.2.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.2.1.4.1-3.

2.	Message contents are defined in clause 7.7.2.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-RSRQ measurements.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.2.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.2.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 7.7.2.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 7.7.2.1.5-2 as appropriate and repeat steps 5-7.

7.7.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition Synchronous cells Table H.3.1-5 Table H.3.1-7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.2.1.5	Test requirement

Table 7.7.2.1.5-1 defines the cell specific settings for all tests. Table 7.7.2.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 7.7.2.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 7.7.2.1.5-3.

Table 7.7.2.1.5-1: SS-RSRQ Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | Freq1 |  | Freq1 |  |
| Duplex mode |  |  | TDD |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| TCI state |  |  | TCI.State.0 |  | TCI.State.0 |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - | CR.3.1 TDD |  |
| Control channel RMC |  |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  |  | SMTC.1 |  |  |  |
| SSB configuration |  |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 | 120 | 120 | 120 |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition |  |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void. |  |  |  |  |  |  |

Table 7.7.2.1.5-2: SS-RSRQ Intra frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 |  | Setup 1 |  |
| Assumption for UE beamsNote 9 |  | Rough |  |  |  |
| Note1 | dBm/15kHzNote4 | -100.7 |  | -96.7 |  |
| Note1 | dBm/SCSNote3 | -91.67 |  | -87.67 |  |
| SSB_RPNote2 | dBm/SCS Note4 | -88.67 | -88.67 | -90.67 | -90.67 |
| SS-RSRQ Note2 | dB | -14.81 | -14.81 | -16.84 | -16.84 |
|  | dB | -1.76 | -1.76 | -4.76 | -4.76 |
|  | dB | 3 | 3 | -3 | -3 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.7 |  | -55.67 | -55.67 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void. Note 7: Void Note 8: Void Note 9: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 7.7.2.1.5-3: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

| UE Power Class 3 | Test 1 (All bands) | Test 2 |  |
| --- | --- | --- | --- |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_41 | n257, n258, n261 | SS-RSRQ_35 |
|  |  | n260 | SS-RSRQ_34 |
|  |  | n259 | FFS |
| Highest reported value (Cell 2) | SS-RSRQ_73 | All bands | SS-RSRQ_71 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_38+ FFS | n257, n258, n261 | SS-RSRQ_34+ FFS |
|  |  | n260 | SS-RSRQ_33+ FFS |
|  |  | n259 | FFS |
| Highest reported value (Cell 2) | SS-RSRQ_76+ FFS | All bands | SS-RSRQ_72+ FFS |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.2.2.

7.7.2.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.2.2.4.1-1.

Table 7.7.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.2.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.2.2.4.1-2.

Table 7.7.2.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.2.2.4.1-3.

2.	Message contents are defined in clause 7.7.2.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-RSRQ measurements.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.2.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.2.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport for the following requirements:

- R1: The SS-RSRQ value of Cell 1 reported by the UE is compared to the expected SS-RSRQ for Cell 1. If the value is outside the limits in Table 7.7.2.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

- R2: The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ for Cell 2. If the value is outside the limits in Table 7.7.2.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3: The SS-RSRQ value of Cell 2 reported by the UE is compared to the reported SS-RSRQ of Cell 1. If the resulting value is outside the limits in Table 7.7.2.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 7.7.2.2.5-2 as appropriate and repeat steps 5-7. In Step 6, the SS shall check the following requirements:

- R4: The SS-RSRQ value of Cell 1 reported by the UE is compared to the expected SS-RSRQ for Cell 1. If the value is outside the limits in Table 7.7.2.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

- R5: The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ for Cell 2. If the value is outside the limits in Table 7.7.2.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R5 is increased by one. Otherwise, the number of passed iterations for R5 is increased by one.

- R6: The SS-RSRQ value of Cell 2 reported by the UE is compared to the reported SS-RSRQ of Cell 1. If the resulting value is outside the limits in Table 7.7.2.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R6 is increased by one. Otherwise, the number of passed iterations for R6 is increased by one.

7.7.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-5 Table H.3.1-6 with condition Pattern #0 Table H.3.1-7 with condition INTER-FREQ Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.2.2.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.2.2.5	Test requirement

Table 7.7.2.2.5-1 defines the cell specific settings for all tests. Table 7.7.2.2.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-RSRQ measurement accuracy shall fulfil the absolute accuracy requirements in clause 7.7.2.0.2.1 and relative accuracy requirements in clause 7.7.2.0.2.2. The following eight requirements are to be verified:

During T1:

R1: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRQ is in the range shown in Table 7.7.2.2.5-3.

R2: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRQ is in the range shown in Table 7.7.2.2.5-3.

R3: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRQ meets the requirements in Table 7.7.2.2.5-4.

During T2:

R4: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-RSRQ is in the range shown in table 7.7.2.2.5-3.

R5: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRQ is in the range shown in table 7.7.2.2.5-3.

R6: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRQ meets the requirements in Table 7.7.2.2.5-4.

Table 7.7.2.2.5-1: SS-RSRQ Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq2 | freq1 | Freq2 |
| SSB Configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  | 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |  |  |

Table 7.7.2.2.5-2: SS-RSRQ Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 |  | Setup 1 |  |
| Assumption for UE beamsNote 8 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHzNote4 | -95.93 | -95.93 | -95.44 | -95.44 |
| Note1 | dBm/SCSNote3 | -86.9 | -86.9 | -86.41 | -86.41 |
|  | dB | -1.75 | -1.75 | -3 | -3 |
| SSB_RPNote2 | dBm/SCS Note4 | -88.65 | -88.65 | -89.41 | -89.41 |
| SS-RSRQNote2 | dB | -14.75 | -14.75 | -15.56 | -15.56 |
|  | dB | -1.75 | -1.75 | -3 | -3 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.7 | -55.7 | -55.7 | -55.7 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 7.7.2.2.5-3: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 1) | RSRQ_41 | RSRQ_37 |
| Highest reported value (Cell 1) | RSRQ_73 | RSRQ_74 |
| Lowest reported value (Cell 2) | RSRQ_41 | RSRQ_37 |
| Highest reported value (Cell 2) | RSRQ_73 | RSRQ_74 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 1) | RSRQ_41 + FFS | RSRQ_37 + FFS |
| Highest reported value (Cell 1) | RSRQ_73 + FFS | RSRQ_74 + FFS |
| Lowest reported value (Cell 2) | RSRQ_41 + FFS | RSRQ_37 + FFS |
| Highest reported value (Cell 2) | RSRQ_73 + FFS | RSRQ_74 + FFS |

Table 7.7.2.2.5-4: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest value (RSRQ report Cell 2 – RSRQ report Cell 1) | - 7 | - 9 |
| Highest value (RSRQ report Cell 2 – RSRQ report Cell 1) | + 7 | + 9 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest value (RSRQ report Cell 2 – RSRQ report Cell 1) | – 7 + FFS | – 9 + FFS |
| Highest value (RSRQ report Cell 2 – RSRQ report Cell 1) | + 7 + FFS | + 9 + FFS |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R6) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

### 7.7.3	SS-SINR

#### 7.7.3.0	Minimum conformance requirements

##### 7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

Same as in clause 5.7.3.0.1.

##### 7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements

Same as in clause 5.7.3.0.2.

#### 7.7.3.1	NR SA FR2 SS-SINR measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

7.7.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting ss-SINR-Meas.

7.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.3.1.

7.7.3.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the intra-frequency NR FR2 neighbour cell.

7.7.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.3.1.4.1-1.

Table 7.7.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.3.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.3.1.4.1-2.

Table 7.7.3.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.3.1.4.1-3.

2.	Message contents are defined in clause 7.7.3.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-SINR measurements.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.3.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.3.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 7.7.3.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 7.7.3.1.5-1 as appropriate and repeat steps 5-7.

7.7.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition Synchronous cells Table H.3.1-5 Table H.3.1-7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.3.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.3.1.5	Test requirement

Table 7.7.3.1.5-1 defines the cell specific settings for all tests. Table 7.7.3.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 7.7.3.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 7.7.3.1.5-3.

Table 7.7.3.1.5-1: SS-SINR Intra frequency test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq2 |  | Freq2 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated RMSI CORESET Reference Channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 |  |  |  |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| SS-RSSI-Measurement |  | Not Applicable |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void. Note 3: Void Note 4: Void |  |  |  |  |  |

Table 7.7.3.1.5-2: SS-SINR Intra frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 |  |  | Cell 2 |
| Angle of arrival configuration |  |  | Setup 1 |  | Setup 1 |  |  |  |
| Assumption for UE beamsNote 9 |  |  | Rough |  | Rough |  |  |  |
| Note1 |  | dBm/15kHz Note4 | -105 |  | -105 |  |  |  |
| Note1 |  | dBm/SCS Note3 | -96 |  | -96 |  |  |  |
| SSB_RPNote2 |  | dBm/SCS Note4 | -91.46 | -93.34 | -98.8 |  |  | -98.8 |
| SS-SINR Note2 | n257, 258, n261 | dB | -0.18 | -3.32 | -4.95 | -4.95 |  |  |
|  | n260 |  | -0.31 | -3.42 | -5.20 | -5.20 |  |  |
|  | n259 |  | -0.39 | -3.48 | -5.48 | -5.48 |  |  |
|  | n257, 258, n261 | dB | -0.18 | -3.32 | -4.95 | -4.95 |  |  |
|  | n260 |  | -0.31 | -3.42 | -5.20 | -5.20 |  |  |
|  | n259 |  | -0.39 | -3.48 | -5.48 | -5.48 |  |  |
|  |  | dB | 4.54 | 2.66 | -3 |  | -3 |  |
| IoNote2 |  | dBm/95.04 MHz Note4 | -59.43 |  | -63.87 |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void. Note 7: Void Note 8: Void Note 9: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

Table 7.7.3.1.5-3: SS-SINR Intra frequency absolute accuracy requirements for the reported values

| UE Power Class 3 | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- |
| Normal Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | n257, n258, n261 | SS-SINR_22 | All bands | SS-SINR_18 |
|  | n260 | SS-SINR_21 |  |  |
|  | n259 | FFS |  |  |
| Highest reported value (Cell 2) | All bands | SS-SINR_58 | n257, n258, n261 | SS-SINR_55 |
|  |  |  | n260 | SS-SINR_54 |
|  |  |  | n259 | FFS |
| Extreme Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | n257, n258, n261 | SS-SINR_21+ FFS | All bands | SS-SINR_17+ FFS |
|  | n260 | SS-SINR_20+ FFS |  |  |
|  | n259 | FFS |  |  |
| Highest reported value (Cell 2) | All bands | SS-SINR_59+ FFS | n257, n258, n261 | SS-SINR_56+ FFS |
|  |  |  | n260 | SS-SINR_55+ FFS |
|  |  |  | n259 | FFS |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

7.7.3.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

7.7.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting ss-SINR-Meas.

7.7.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.3.2.

7.7.3.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.3.2.4.1-1.

Table 7.7.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.3.2.4.1-2.

Table 7.7.3.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.3.2.4.1-3.

2.	Message contents are defined in clause 7.7.3.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-SINR measurements.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.3.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.3.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport for the following requirements:

- R1: The SS-SINR value of Cell 1 reported by the UE is compared to the expected SS-SINR for Cell 1. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

- R2: The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR for Cell 2. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3: The SS-SINR value of Cell 2 reported by the UE is compared to the reported SS-SINR of Cell 1. If the resulting value is outside the limits in Table 7.7.3.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 7.7.3.2.5-2 as appropriate and repeat steps 5-7. In Step 6, the SS shall check the following requirements for Test 2:

- R4: The SS-SINR value of Cell 1 reported by the UE is compared to the expected SS-SINR for Cell 1. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

- R5: The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR for Cell 2. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R5 is increased by one. Otherwise, the number of passed iterations for R5 is increased by one.

- R6: The SS-SINR value of Cell 2 reported by the UE is compared to the reported SS-SINR of Cell 1. If the resulting value is outside the limits in Table 7.7.3.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R6 is increased by one. Otherwise, the number of passed iterations for R6 is increased by one.

And the following requirements for Test 3:

- R7: The SS-SINR value of Cell 1 reported by the UE is compared to the expected SS-SINR for Cell 1. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 1, the number of failed iterations for R7 is increased by one. Otherwise, the number of passed iterations for R7 is increased by one.

- R8: The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR for Cell 2. If the value is outside the limits in Table 7.7.3.2.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R8 is increased by one. Otherwise, the number of passed iterations for R8 is increased by one.

- R9: The SS-SINR value of Cell 2 reported by the UE is compared to the reported SS-SINR of Cell 1. If the resulting value is outside the limits in Table 7.7.3.2.5-4 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations for R9 is increased by one. Otherwise, the number of passed iterations for R9 is increased by one.

7.7.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-5 Table H.3.1-6 with condition Pattern #0 Table H.3.1-7 with condition INTER-FREQ Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.7.3.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.7.3.2.5	Test requirement

Table 7.7.3.2.5-1 defines the cell specific settings for all tests. Table 7.7.3.2.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-SINR measurement accuracy shall fulfil the absolute accuracy requirements in clause 7.7.3.0.2.1 and relative accuracy requirements in clause 7.7.3.0.2.2. The following eight requirements are to be verified:

During Test 1:

R1: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in Table 7.7.3.2.5-3.

R2: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in Table 7.7.3.2.5-3.

R3: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-SINR meets the requirements in Table 7.7.3.2.5-4.

During Test 2:

R4: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in table 7.7.3.2.5-3.

R5: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in table 7.7.3.2.5-3.

R6: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-SINR meets the requirements in Table 7.7.3.2.5-4.

During Test 3:

R7: Absolute accuracy of Cell 1. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in table 7.7.3.2.5-3.

R8: Absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-SINR is in the range shown in table 7.7.3.2.5-3.

R9: Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-SINR meets the requirements in Table 7.7.3.2.5-4.

Table 7.7.3.2.5-1: SS-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  | 66 |  | 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |  |  |  |  |

Table 7.7.3.2.5-2: SS-SINR Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | degrees | Setup 1 |  | Setup 1 |  | Setup 1 |  |
| Assumption for UE beamsNote 10 |  | Rough |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 | -105 | -105.1 | -105.1 | -105 | -105 |
| Note1 | dBm/SCS Note3 | -96 | -96 | -96.1 | -96.1 | -96 | -96 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -2.1 | -2.1 |
| SS-RSRPNote2 | dBm/SCS Note4 | -96.5 | -96.5 | -85.1 | -85.1 | -98.1 | -98.1 |
| SS-SINRNote2 | dB | -0.5 | -0.5 | 11 | 11 | -2.1 | -2.1 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -2.1 | -2.1 |
| IoNote2 | dBm/95.04 MHz Note4 | -69.3 | -69.3 | -55.75 | -55.75 | -64.9 | -64.9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

Table 7.7.3.2.5-3: evaluation limits for the reported values for T1 and T2 absolute accuracy rules R1, R2, R4, R5

| UE power class 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands |  | Test 2 All bands |  | Test 3 All bands |  |
| Lowest reported value (Cell 1) | SINR_27 |  | SINR_48 |  | n257, n258, n261 | SINR_23 |
|  |  |  |  |  | n260 | SINR_22 |
|  |  |  |  |  | n259 | FFS |
| Highest reported value (Cell 1) | n257, n258, n261 | SINR_62 | n257, n258, n261 | SINR_87 | n257, n258, n261 | SINR_60 |
|  | n260 | SINR_61 | n260 | SINR_86 | n260 | SINR_59 |
|  | n259 | FFS | n259 | FFS | n259 | FFS |
| Lowest reported value (Cell 2) | SINR_27 |  | SINR_48 |  | n257, n258, n261 | SINR_23 |
|  |  |  |  |  | n260 | SINR_22 |
|  |  |  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | n257, n258, n261 | SINR_62 | n257, n258, n261 | SINR_87 | n257, n258, n261 | SINR_60 |
|  | n260 | SINR_61 | n260 | SINR_86 | n260 | SINR_59 |
|  | n259 | FFS | n259 | FFS | n259 | FFS |
| Extreme Conditions | Test 1 All bands |  | Test 2 All bands |  | Test 3 All bands |  |
| Lowest reported value (Cell 1) | SINR_27 + FFS |  | SINR_48 + FFS |  | n257, n258, n261 | SINR_23 + FFS |
|  |  |  |  |  | n260 | SINR_22 + FfS |
|  |  |  |  |  | n259 | FFS |
| Highest reported value (Cell 1) | n257, n258, n261 | SINR_62 + FFS | n257, n258, n261 | SINR_87 + FFS | n257, n258, n261 | SINR_60 + FFS |
|  | n260 | SINR_61 + FFS | n260 | SINR_86 + FFS | n260 | SINR_59 + FFS |
|  | n259 | FFS | n259 | FFS | n259 | FFS |
| Lowest reported value (Cell 2) | SINR_27 + FFS |  | SINR_48 + FFS |  | n257, n258, n261 | SINR_23 + FFS |
|  |  |  |  |  | n260 | SINR_22 + FfS |
|  |  |  |  |  | n259 | FFS |
| Highest reported value (Cell 2) | n257, n258, n261 | SINR_62 + FFS | n257, n258, n261 | SINR_87 + FFS | n257, n258, n261 | SINR_60 + FFS |
|  | n260 | SINR_61 + FFS | n260 | SINR_86 + FFS | n260 | SINR_59 + FFS |
|  | n259 | FFS | n259 | FFS | n259 | FFS |

Table 7.7.3.2.5-4: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest value (SINR report Cell 2 – SINR report Cell 1) | - 8 | - 17 | - 9 |
| Highest value (SINR report Cell 2 – SINR report Cell 1) | + 8 | + 17 | + 9 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest value (SINR report Cell 2 – SINR report Cell 1) | – 8 + FFS | – 17 + FFS | – 9 + FFS |
| Highest value (SINR report Cell 2 – SINR report Cell 1) | + 8 + FFS | + 17 + FFS | + 9 + FFS |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R6) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.